OROMIA IRRIGATION DEVELOPMENT AUTHORITY

ALENTU SMALL SCALE IRRIGATION PROJECT

IN

DODOLLA WOREDA OF WEST ARSI ZONE

FINAL FEASIBILITY STUDY

FINAL FEASIBILITY STUDY

FINAL FEASIBILITY STUDY

FINAL FEASIBILITY STUDY

BY

VIRGINIA CONSULTANTS IN ASSOCIATION WITH BERUH CONSULTING ENGINEERS AND HYDRO GEOLOGISTS

HYDROLOGY, IRRIGATIION AND HEAD WORK

NOVEMBER, 2016

TABLE OF CONTENTS PAGE

1.	HYDROLOGY AND HEAD WORK DESIGN	1

HYDROLOGY	2

1.1.	Introduction	2

1.1.1.	Location and catchment characteristics of the project area	2

1.1.2.	Topography	2

1.2.	Objective of the Study	2

1.3.	Methodology of the hydrological study	3

1.3.1.	Data Availability	3

1.3.2.	Climate of the project area	3

1.3.3.	Meteorological Observation Stations	3

1.3.4.	Rainfall	3

1.3.5.	Monthly Rainfall Profile and Seasonal Variation	4

1.3.6.	Temperature	5

1.4.	Hydrological Analysis	6

1.4.1.	Water Resources of the Project Area	6

1.4.2.	Dependability and Water Availability Analysis	7

1.4.3.	Hydrological Analysis	7

1.4.4.	Water Availability	13

1.5.	Drainage Duty	13

1.6.	Sediment	13

HEAD WORK	14

1.7.	Introduction	14

1.8.	Evaluation of the Selected Weir Site	14

1.9.	Weir Type and Headwork Layout Alternatives	14

1.9.1.	General	14

1.9.2.	Type of Weir	15

1.10.	Engineering of Major Works	17

1.10.1.	Hydrology	17

1.10.2.	Engineering Geology	17

1.10.3.	Hydraulics	18

1.11.	Downstream Hydraulics	21

1.11.1.	Water Surface at Downstream	21

1.11.2.	Sluice Gates (Scouring Sluice)	24

1.11.3.	Intake	24

1.12.	Structural Design of the Weir & Appurtenants	25

1.12.1.	Sub-Surface Hydraulic	25

1.12.2.	Structural Analysis and Design	26

1.13.	Cross Drainage Structure	28

1.13.1.	General	28

IRRIGATION SYSTEM	31

1.14.	Introduction	31

1.14.1.	System Layout	31

1.14.2.	System Description	35

1.14.3.	Irrigation System Design	36

1.14.4.	Irrigation Infrastructures	39

1.14.5.	Drainage System Design	43

1.14.6.	Drainage Infrastructure	45

1.14.7.	Hydraulics & Structural Design Criteria	46

1.14.8.	Stone Protection Works	49

1.14.9.	Sub-Surface Hydraulics	50

1.14.10.	Project Design Criteria	52

OPERATION AND MAINTENANCE MANUAL	62

1.15.	Irrigation System	62

1.15.1.	General	62

1.15.2.	System Description	62

1.15.3.	Definition and Nomenclature	63

1.16.	Operation of Irrigation Canals	64

1.16.1.	General	64

1.16.2.	System Layout	65

1.16.3.	Operation of Irrigation Canals	65

1.16.4.	Tertiary Canals	66

1.16.5.	Capacity of Irrigation Canals and Their Operation	66

1.16.6.	Field Canals & Siphons	66

1.16.7.	Furrows	67

1.16.8.	Request for Water	67

1.17.	Drainage System	70

1.17.1.	Operation of Drainage System	70

1.18.	Maintenance	70

1.18.1.	Regular & Periodic Maintenance	71

1.18.2.	Incidental Maintenance	71

LIST OF TABLES

Table 1: slope classes and their distribution in Alentu Watershed	2

Table 2: mean monthly max. rainfall at Adaba	4

Table 3: mean monthly max. temp at Adaba	5

Table 4: mean monthly max. temp at Adaba	5

Table 5: values of runoff coefficient	8

Table 6: estimated rainfall intensity for two year period	8

Table 7: approximate time of concentration in min. for watershed of various length and grades	9

Table 8: estimated values of runoff coefficient ''C''	9

Table 9: computed flow volume of Alentu stream	10

Table 10: rainfall intensity at Adaba	11

Table 11: rainfall intensity for various return periods	11

Table 12: spread sheet for pick flow determination	11

Table 13: channel parameters and stream conveyance capacity at the weir axis	13

Table 14: soil loss and their severity classes in Alentu watershed	13

Table 15: Consultant’s comparison and evaluation water abstraction alternatives	16

Table 16: gives the computed flow volume of Alentu stream at the diversion site	17

Table 17: absolute roughness of materials	21

Table 18: surface hydraulics design (Alentu weir)	23

Table 19: sub-surface analysis & design for Alentu weir	25

Table 20: Alentu weir general calculation of free standing rationing wall	27

Table 21: derived value of roughness coefficient for lined and unlined canals	37

Table 22: sample flow capacity and geometric parameters for main canals	38

Table 23: sample canal geometric design results	39

Table 24: summary of main canal structures & quantity in the project	40

Table 25: summary of tertiary canal structures	42

Table 26: lane's safe weighted creep ratio	50

Table 27: safe exit gradient values	50

Table 28: limiting velocities in non-vegetated channels	54

Table 29: Main canals and their duty	65

LIST OF FIGURES

Figure 1: mean monthly rainfall at Adaba(mm)	4

Figure 2: mean monthly max. Temperature at Adaba	5

Figure 3: mean monthly max. Temperature at Adaba	6

Figure 4: regional drainage setting of the area	6

Figure 5: Alentu stream cross sections	12

Figure 6: adopted head work layout for Alentu project	15

Figure 7: geological cross section at the weir axis	17

Figure 8: reproduced from ven. T chow	24

Figure 9: longitudinal profile of the pump and raiser main.	29

Figure 10: Typical Cross Section Adopted	32

Figure 11: project layout	33

Figure 12: typical cross section of field canal	37

Figure 13: typical main canal check	41

Figure 14: typical main canal off take	41

Figure 15: typical tertiary off take	43

Figure 16: Typical Field Drain	44

Figure 17: typical drainage outfall cum drop structure	46

Figure 18: Typical cross section of canal	65

LIST OF ANNEX

Annex 1: Left tertiary canal profiles	61

Annex 2: Left tertiary canal table of labels and dimensions (off take & checks)	61

Annex 3: Left tertiary canal hydraulic design results	61

Annex 4: Left tertiary canal structural design results	61

Annex 5: Right main canal & tertiary canal profiles	61

Annex 6: Right tertiary canal table of labels and dimensions (checks & off takes)	61

Annex 7: Right tertiary canal hydraulic design results	61

Annex 8: Right tertiary canal structural design results	61

# HYDROLOGY AND HEAD WORK DESIGN

# HYDROLOGY

## Introduction

Alentu small scale irrigation project is locate South-East of Ethiopia within Dodola Woreda in WestArsi Zone of the Oromia Regional State. Geographically it is situated at Lat. 505578E, 0758018N Long.

The Major stream that drains most of the excesses flow from Alentu catchment is Alentu Stream. Besides this the flow of the stream is augmented by minor intermittent tributaries.

### Location and catchment characteristics of the project area

Geographically Alentu small scale irrigation project is located at 505200E, 0788390N Long nearby Siroftu Village. The main source of Water in the project area is Alentu Stream. The river source is located at about 6km South-East of Siroftu Village. It covers a total drainage area of 22 km2.  Out of these 15% is Cultivated land, 41% Dense forest,10% Grazing/grass land, 25%  open forest, 9% Open wood land.

### Topography

The topography of the project area is characterized by the very steep high mountainous terrain ranging from 2800m a.m.s.l in altitude at the north end and reaches up to 3460 meters at southern tip of the watershed. The slope range of the area ranges at Alentu.

Table 1: slope classes and their distribution in Alentu Watershed

| No | Slope Gradient (%) | Slope class | Area (Ha.) | percent |
| --- | --- | --- | --- | --- |
| 1 | 0 to 3 | Flat or almost flat | 338.63 | 15.28 |
| 2 | 3 to 8 | Gently sloping | 256.27 | 11.56 |
| 3 | 8 to 15 | Sloping | 605.43 | 27.27 |
| 4 | 15 to 30 | Moderately steep | 846.88 | 38.21 |
| 5 | 30 to 50 | Steep | 157.49 | 7.11 |
| 6 | >50 | Very steep | 12.67 | 0.57 |
| Total |  |  | 2216.38 | 100 |

Source: Water Shade Report

## Objective of the Study

The main aim of the hydrological study is to assess the availability of surface water resources for Alentu Small Scale Irrigation Project.

The present hydrological study will focus on estimation of flood volume that would be produced from the catchment area and carry out the water availability and dependability study for the project. The study will mainly focus on estimation of probable maximum flow which could occur at the weir site which will be an input for the design of the weir and head works.

## Methodology of the hydrological study

The following methodologies were applied for the study.

- Collect relevant secondary data from previous studies and residents of the area;

- Collect primary data from field and relevant hydro meteorological data from Meteorological Agency

- Prepare hydrological database mainly for rainfall

- Estimate the water availability of the catchment

- Estimate the peak flood of the proposed catchment area based on monthly rainfall data and assess the magnitude of design flood.

### Data Availability

For the determination of design flood, stream flow or rainfall data are necessary.  The present irrigation scheme is planned on Alentu stream. As Alentu stream is un-gauged stream it will be obligatory to estimate the flow of the stream quantifying the yield from the catchment area.

### Climate of the project area

The climate of the area comes under the influence of the Inter-Tropical Convergence Zone (ITCZ). The seasonal rainfall distribution within the study area results from the annual migration of the ITCZ. One major rainy season has been experienced. The main rainy season often extends from March till October.

### Meteorological Observation Stations

One meteorological observation station is located at Adaba Town which is only 35km from the project site. The meteorological data recorded at these stations is obtained from the National Meteorological Agency. This meteorological station is class I stations that include observations of rainfall, temperature, relative humidity, sunshine duration and wind speed.

### Rainfall

Rainfall data is needed to represent the monthly rainfall time series at Adaba, station. All available meteorological data were obtained from the National Meteorological Agency (NMA). The monthly rainfall patterns and other climatic elements such as temperature, relative humidity, wind speed and sunshine duration are computed based on standard FAO published databases.

In this kind of study it is customary to face data shortage and even if data is available its consistency and reliability could be questionable. For the present study hourly maximum rainfall data was retrieved from NMA. Historically, the available hourly rainfall data at Adaba is quite impressing (from 1965 to 2011). However, on most instances the data gap observed is alarming and sometimes the data for an entire year is missing. Therefore, for the present study the monthly rainfall data at Adaba station is used. A total of 18 years recorded rainfall data was found for this station. Compared to the hourly rainfall data the monthly rainfall data is more consistent with low value of standard deviation. The recoded data as well as the consistency result for the monthly rainfall data is as given in table -10

### Monthly Rainfall Profile and Seasonal Variation

For the present study monthly maximum precipitation data were used to illustrate local seasonal rainfall variations as well as spatial differences within the catchment area. Monthly max. Precipitation data was mainly used to comply with the proposed hydrological model that is to be employed in the hydrological study.

The average max. Monthly rainfall of the selected station (Adaba) is asgiven in the Table-2. The corresponding graphical profile is as indicated in Figure 1.

The monthly profile shows the occurrence and relative strength of the dry, wet and intermediate season. The rainfall pattern shows a prolonged wet season starting from March till October reflecting a Uni-Modal Type regime.

| Table 2: mean monthly max. rainfall at Adaba |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Year | Jan | Feb | Mar | Apr | May | Jun | Jly | Aug | Sep | Oct | Nov | Dec |
| Adaba | 24.0 | 23.8 | 66.3 | 73.9 | 55.2 | 91.1 | 202.7 | 197.1 | 89.0 | 49.2 | 10.8 | 22.4 |

Seasonal variations, shown during rainy and dry seasons has different trend of rainfall distribution, but only small differences of values can be observed between these two seasons, depending on the actual chronological period covered by the historical record .

Figure 1: mean monthly rainfall at Adaba(mm)

Seasonal variations, showing during rainy and dry seasons has different trend of rainfall distribution, but only small differences of values can be observed between these two seasons, depending on the actual chronological period of covered by the historical record .

### Temperature

The mean monthly maximum temperature at the project area varies from 220c to 260c. The mean monthly minimum temperature at the project area varies from 50c to 90c. The mean annual temperature is about 15.50c.

| Table 3: mean monthly max. temp at Adaba |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Year | Jan | Feb | Mar | Apr | May | Jun | Jly | Aug | Sep | Oct | Nov | Dec |
| Mean | 24.6 | 25.8 | 25.2 | 24.9 | 25.1 | 24.5 | 22.7 | 22.3 | 23.1 | 23.1 | 23.6 | 24.1 |

Figure 2: mean monthly max. Temperature at Adaba

| Table 4: mean monthly max. temp at Adaba |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Year | Jan | Feb | Mar | Apr | May | Jun | Jly | Aug | Sep | Oct | Nov | Dec |
| Mean | 5.4 | 5.1 | 6.8 | 7.9 | 7.8 | 7.8 | 8.7 | 8.6 | 7.9 | 7.7 | 5.1 | 4.7 |

Figure 3: mean monthly max. Temperature at Adaba

## Hydrological Analysis

### Water Resources of the Project Area

The main source of water for Alentu irrigation scheme is Alentu stream. Alentu stream is the major drainage channel that collects water from the catchment. The major source of water for the stream is rainfall runoff.Figure 4 gives the regional drainage setting of the area.

Figure 4: regional drainage setting of the area

Alantu River

Alentu Stream

The project is supposed to take irrigation water from Alentu Stream. Diversion weir is proposed to be constructed across the stream at geographic coordinate of 505578E, 0758018N latitude and an average altitude of about 2800 ma.m.s.l.

Besides irrigation, Alentu stream is also used for domestic and livestock drinking and as a water supply source for Siroftu village.

### Dependability and Water Availability Analysis

The water availability and dependability for the project has been worked out based on hydrological data. As stream flow gauging station is not available on Alentu stream, the stream flow of Alentu is deduced from basic deterministic principles using simple yet powerful model.

In the design of hydraulic and irrigation structures, the peak flow that can be expected within the specified return period is important. Under estimating of peak flow would result inadequate capacity of structures and subsequent failure of the structure. For safety of structures, the magnitude of a peak flood may be estimated by the following different methods: Rational method, Empirical formulas, Unit hydrograph technique, flood frequency studies, Snyder’s and SCS methods. Out of which the Consultant has proposed to use the Rational Formula.

Regarding irrigation water supply the critical phenomenon is the prediction of the available flow during the dry season, where rainfall is minimal. As the rational method could give estimates of peak flow the dependable flow for the scheme during the dry season could not be predicted with any certainty. From the site visit and discussions made with the inhabitants of the project area, major water supply for Alentu comes from rainfall and some springs of the surrounding area.

### Hydrological Analysis

For the general hydrological analysis the rational formula is used. This method is used to estimate peak discharges for small watershed using hourly rainfall intensity.

The rational method is used to estimate peak discharges for small watersheds using hourly rainfall intensity data. In this method it is always considered that the maximum flow rate is resulted from uniform rainfall intensity over the entire catchment area where the rainfall duration is equal to the time of concentration.  Time of concentration (tc) is the time which takes for water to travel from the most distant point of a watershed to the outlet or to some other downstream reference point. The rational formula reads as;

qp = C*(It)*A/3.6

Where:-	qp = Peak discharge (m3/s)

C = Runoff Coefficient

It= Rainfall Intensity for different return period (mm/hr)

A = Catchment Area (km2)

Runoff Coefficient (C)

Runoff Coefficient (C) will vary from 0-1 according to watershed conditions. Table-5 gives values of C for various watershed/land uses and slopes.

Table 5: values of runoff coefficient

| Land use/Type | Flat | Rolling | Hilly | Mountainous |
| --- | --- | --- | --- | --- |
| Slope | 4-7% | 8-10% | 11-20% | 21-30% |
| Paved Area | 0.8 | 0.85 | 0.90 | 0.95 |
| Bare Area | 0.7 | 0.75 | 0.8 | 0.85 |
| Cultivated Area | 0.3 | 0.35 | 0.4* | 0.5 |
| Grass Land | 0.2 | 0.25 | 0.3* | 0.4 |
| Forest | 0.1 | 0.15 | 0.2* | 0.3 |

*Values adopted for Alentu Project

Time of Concentration (tc)

The time of concentration (tc) is computed by Kirpich’s formula as given by:

tc = (0.0195*(l0.77/(s0.385)))/60

Where; tc=Time of concentration in hrs

l =   length of water course in meters

s =   slope of water course

Since the present water shade area is relatively small catchment, they will have tc values less than one hour. Under this circumstance Bell’s formula is used to determine the rainfall depth (h) for the time equal to the time of concentration, tc. The formula is given as:

(htc)T = (0.35*ln(T)+0.76)*(0.54*tc0.25-0.5)*(h60)2

In the absence of test data and standard procedure the following data and guideline may be used.

Rain fall Intensity

Although annual rainfall may vary, intensity over short period tends to be similar. Based on this argument, estimated rainfall intensities as given in Table-1 could be used for design purposes. Table-6 is drawn based on rainfall intensity observed in tropical countries.

Table 6: estimated rainfall intensity for two year period

| Duration of Rainfall (min.) | Total Rainfall for Duration (mm) | Rainfall Intensity mm/hr |
| --- | --- | --- |
| 5 | 14.2 | 170 |
| 10 | 23.3 | 140 |
| 30 | 45.0 | 90 |
| 60 | 65.0* | 65* |
| 360 | 90.0 | 15 |
| 24 hr | 100.0 | 4 |

*Values adopted for Alentu Project

Time of Concentration

Knowing the maximum length of flow and the average gradient of the catchment, the value of the time of concentration was obtained from Table-7.

Table 7: approximate time of concentration in min. for watershed of various length and grades

| Maximum Length of Flow (m) | Slope |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | 2% | 5% | 10% | 20% | 30% |
| 100 | 4 | 3 | 2 | 2 | 1 |
| 150 | 5 | 4 | 3 | 2 | 2 |
| 200 | 7 | 5 | 4 | 3 | 2 |
| 300 | 9 | 6 | 5 | 4 | 3 |
| 500 | 14 | 10 | 7 | 6 | 5 |
| 1000 | 24 | 17 | 13 | 10 | 8 |
| 1500 | 32 | 23 | 17 | 13* | 11 |

*Values adopted for Alentu Project

Thus with the knowledge of the time of concentration, the maximum rainfall intensity for the catchment can be assumed. However, not all rainfall will result in run-off. Some will infiltrate into the soil and much will be absorbed in micro basins and intercepted by vegetation. Thus the final runoff will always be less than the rainfall intensity on the area.  The ration between peak runoff and rainfall intensity is called coefficient of runoff and is designated by the letter "C".

Table-8 gives the estimated values of the runoff coefficient C for different soil texture and land cover types.

Table 8: estimated values of runoff coefficient ''C''

| Topographyand Vegetation | Soil Texture |  |  |
| --- | --- | --- | --- |
|  | Open Sandy Loam | Clay &Silt Loam | Tight Clay |
| Forest Land Flat (0-5% slope) Rolling (5-10% slope) Hilly (10-30% slope) Steep (> 30% slope) | 0.10 0.25 0.30 0.35 | 0.30 0.35 0.40 0.45 | 0.40 0.50 0.60 0.70 |
| Pasture Flat (0-5% slope) Rolling (5-10% slope) Hilly (10-30% slope) Steep (> 30% slope) | 0.10 0.16 0.22* 0.30 | 0.30 0.36 0.42 0.50 | 0.40 0.55 0.60 0.70 |
| Cultivated Flat (0-5% slope) Rolling (5-10% slope) Hilly (10-30% slope) | 0.30 0.40 0.52 | 0.50 0.60 0.72 | 0.60 0.70 0.82 |

*Values adopted for Alentu Project

Flow Volume Using Rational Formula

The total drainage area of the catchment is 22 km2 while the total area of the drainage that culminates at the weir site is 2.1 km2.

Table-9 gives the computed flow volume of Alentu stream at the diversion site using the Rational Formula.

Table 9: computed flow volume of Alentu stream

| C | Area (km2) | q10 (m3/s) | q25 (m3/s) | q50(m3/s) |
| --- | --- | --- | --- | --- |
| 0.2 | 2.1 | 6.76 | 8.56 | 10.21 |

The maximum design discharge (Q50) is used for the design of diversion weir, its appurtenant works and to determine the back water effect as a result of constructing the weir, Further maximum discharge determines the water afflux on the weir and hence the height of the weir wing walls.

Table-10 gives the hydrologic parameters and data computation sheet for peak flow.

Table 10: rainfall intensity at Adaba

| Year | Jan | Feb | March | April | May | June | July | August | Sep | Oct | Nov | Dec | Max |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1990 | 10.6 | 161.8 | 100.4 | 151.6 | 57.0 | 6.0 | 118.5 | 337.6 | 56.3 | 4.2 | 0.0 | 0.0 | 337.60 |
| 1994 | 28.7 | 12.8 | 63.3 | 68.4 | 53.8 | 143.1 | 219.6 | 118.6 | 81.3 | 14.4 | 27.5 | 0.6 | 219.60 |
| 1995 | 0.4 | 10.0 | 86.7 | 131.4 | 19.1 | 52.6 | 219.0 | 227.0 | 88.5 | 26.8 | 0.5 | 5.8 | 227.00 |
| 1996 | 30.1 | 14.2 | 72.4 | 82.6 | 81.2 | 75.1 | 177.7 | 230.0 | 94.9 | 0.0 | 6.0 | 0.0 | 230.00 |
| 1997 | 22.5 | 0.0 | 54.2 | 125.5 | 50.0 | 62.8 | 230.5 | 149.4 | 27.7 | 94.3 | 8.5 | 1.6 | 230.50 |
| 1998 | 106.7 | 69.9 | 31.2 | 48.2 | 48.6 | 67.6 | 143.1 | 219.6 | 118.6 | 146.4 | 3.7 | 1.5 | 219.60 |
| 1999 | 23.9 | 0.0 | 68.3 | 12.1 | 55.1 | 97.8 | 250.4 | 165.8 | 101.5 | 90.0 | 0.0 | 0.2 | 250.40 |
| 2000 | 1.4 | 0.0 | 2.6 | 49.1 | 63.7 | 90.1 | 261.6 | 202.9 | 74.8 | 42.2 | 15.9 | 0.0 | 261.60 |
| 2001 | 21.6 | 15.7 | 139.3 | 17.7 | 87.7 | 96.7 | 157.2 | 137.8 | 59.7 | 28.6 | 2.0 | 9.4 | 157.20 |
| 2002 | 40.1 | 6.9 | 70.2 | 42.1 | 47.7 | 56.7 | 135.0 | 125.8 | 53.7 | 25.7 | 0.3 | 66.9 | 135.00 |
| 2003 | 13.3 | 4.8 | 53.3 | 86.8 | 31.3 | 125.2 | 275.3 | 163.3 | 80.0 | 14.0 | 0.8 | 43.8 | 275.30 |
| 2004 | 26.7 | 10.6 | 23.7 | 66.1 | 3.0 | 48.3 | 175.3 | 156.3 | 70.9 | 43.6 | 8.9 | 23.4 | 175.30 |
| 2005 | 28.7 | 9.0 | 94.8 | 91.2 | 143.0 | 69.8 | 261.6 | 201.6 | 100.5 | 39.3 | 14.2 | 0.0 | 261.60 |
| 2006 | 0.2 | 25.2 | 53.9 | 32.2 | 15.9 | 76.3 | 215.3 | 187.4 | 143.7 | 94.9 | 37.5 | 11.8 | 215.30 |
| 2007 | 26.3 | 24.3 | 34.2 | 113.1 | 105.6 | 271.7 | 289.9 | 337.3 | 192.8 | 67.8 | 36.5 | 0.0 | 337.30 |
| 2008 | 26.3 | 24.3 | 60.4 | 69.1 | 57.8 | 91.6 | 214.8 | 192.6 | 92.9 | 54.9 | 10.4 | 12.6 | 214.76 |
| 2009 | 0.0 | 15.3 | 117.9 | 69.1 | 18.8 | 89.5 | 101.9 | 197.1 | 75.6 | 49.2 | 10.7 | 192.6 | 197.10 |
| 2010 | 24.0 | 23.8 | 66.3 | 73.9 | 53.8 | 119.1 | 202.7 | 197.1 | 89.0 | 49.2 | 10.8 | 33.0 | 202.74 |
| Mean | 23.97 | 23.81 | 66.28 | 73.90 | 55.18 | 91.11 | 202.74 | 197.06 | 89.02 | 49.19 | 10.79 | 22.40 | 202.74 |
| Max | 106.70 | 161.80 | 139.30 | 151.60 | 143.00 | 271.70 | 289.90 | 337.60 | 192.80 | 146.40 | 37.50 | 192.60 | 337.60 |
| STD | 29.21 | 47.32 | 35.65 | 40.37 | 37.69 | 65.84 | 56.08 | 65.55 | 41.67 | 41.77 | 12.73 | 58.06 | 55.87 |
| Cv | 1.22 | 1.99 | 0.54 | 0.55 | 0.68 | 0.72 | 0.28 | 0.33 | 0.47 | 0.85 | 1.18 | 2.59 | 0.28 |

Table-10 gives the computed rainfall intensity values for various return periods while Table 12 presents the spread sheet for pick flow determination.

| Table 11: rainfall intensity for various return periods. It=Imean(1+Kt*Cv). Kt=-(-0.45+0.78*ln(ln(T)-ln(T-1))). T(Year); Kt; It(mm/60min). 2; -0.16; 19.6. 5; 0.72; 25.0. 10; 1.31; 28.7. 20; 1.87; 32.1. 25; 2.04; 33.2. 50; 2.59; 36.6. 100; 3.14; 40.0. 150; 3.46; 42.0. 200; 3.68; 43.3. 250; 3.86; 44.4. |  |  |
| --- | --- | --- |

| Table 12: spread sheet for pick flow determination |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Length |  | Slope(%) | Slope | tc(hr) | tc(sec) | h(10) | h(25) | h(50) |  | C |  | A |  | q10 |  | q25 | q50 |
| 2000 |  | 2.5 | 0.025 | 0.46 | 28.09 | 27.12 | 34.35 | 40.99 |  | 0.20 |  | 2.10 |  | 6.76 |  | 8.56 | 10.21 |

Slope Area Method

The slope area method is be used to check the flood magnitude computed as given thereof and at the same time to ascertain whether the river section can pass the estimated discharge or not.

The peak flood could be estimated from high flood marks on the riverbank and bushes and villagers in the area could also provide valuable information. During the field visit for the present study the inhabitants of the project area ascertained that Shaya stream has not overtopped its banks since time in memorial

The slope area method is a procedure for estimating the mean flow velocity assuming that the flow is uniform over a certain reach of the river. Knowing the river channel slope and area at least at three locations i.e. at weir axis and 50m up and downstream of the weir axis, the cross sectional area of the river could reasonably calculated. Knowing the channel cross section and the mean velocity the river discharge can be estimated.

The river channel conveyance will be calculated from

Q = 1/n [R2/3 S1/2]*A

Where;		Q= Conveyance (m3/s)

n= Manning’s coefficient

R= Hydraulic radius of the channel

S= Bed slope of the channel

A= Cross sectional area (m2)

At the weir axis the channel capacity of Alentu is estimated to be 70m3/s. Table 4-4 gives the channel parameters and the stream conveyance capacity at the weir axis. In Figure 4-7, the stream cross section at 50m downstream, at the weir axis and 50m upstream of the weir axis is given. Evidently the stream conveyance capacity at the weir axis is quite higher than the 25 year peak flow in the stream.

Figure 5: Alentu stream cross sections

Table 13: channel parameters and stream conveyance capacity at the weir axis

| No. | A (m2) | P (m) | R (m) | s | N | Q (m3/s) |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 8.15 | 7.7 | 1.06 | 0.04 | 30.00 | 50.83 |

### Water Availability

Since Alentu stream is not a gauged stream it will not be possible to exercise neither statistical nor probabilistic analysis on the stream flow. As a result of this a deterministic approach is used to estimate the dependable flow. The peak flow occurs during the wet season between June and September. Evidently, during these months there won’t be moisture deficit since the rainfall exceeds the demand for irrigation. Besides irrigation during the wet season the flow in the stream will be adequate to meet the water supply demand for Werka and Siroftu Village.

## Drainage Duty

To estimate the drainage duty for the project the following three types of runoffs are considered viz.

i	Surface runoff from rainfall

ii	Surface runoff from irrigation application

In calculating surface runoff from rainfall it is considered that the period for which the crops can be submerged and their growth is retarded without their yields being reduced at the time of harvest is two days.

The volume of rainfall to be removed is calculated from the daily rainfall data of the area. The maximum daily rainfall observed was 95.5mm. Considering the semi-permeability of the soil a flow coefficient of 0.1 could be assumed. Therefore, the drainage duty due to rainfall will be;

95.5*0.3*104	= 1.65 l//s/ha

2*86400

The surface runoff due to excess irrigation runoff is estimated to be 20% of the flow in field canals. This will result in a characteristics flow of (0.2*16./0.258) = 6.02 l//s/ha. Since at the field level up to eight fields could be irrigated simultaneously the characteristics flow will be 1/8th of this i.e. (6.02 l//s/ha/8) = 0.77 l//s/ha.

Therefore, total drainage duty at the filed level will be = 2.42 l//s/ha.

## Sediment

The general sediment yield of the catchment is from non to slight. By and large the hazard due to soil loss is minimal and the sediment transported into the hydrologic system is minimal.

Table 14: soil loss and their severity classes in Alentu watershed

| Soil loss(T/ha/yr) | Severity Class | Priority Class | Area in ha | % | Annual soil loss (T/ha/Yr) |
| --- | --- | --- | --- | --- | --- |
| 0 - 5 | None to slight | II | 1357.01 | 61.23 | 3392.525 |
| 16 – 30 | Moderate | I | 859.37 | 38.77 | 26640.47 |
| Total |  |  | 2216.38 | 100 | 30,033 |

Source: Water Shade Study Report

# HEAD WORK

## Introduction

Based on the Clients requirement diversion weir is sought to be the head work alternative for Alentu project. Earlier after scrutinizing existing topographic conditions of the area the Consultant had proposed a diversion and a mechanical lifting system to be the water abstraction structures for Alentu Project. However, due to the clients decision, the mechanical lifting has been discarded and the left side irrigable area at Alentu is made to be irrigated by gravity. This situation has resulted in a very long main canal with (2.55 km) with a major cross drainage work of 140m long at Km 1+000.

As in any weir design assignment, the weir for Alentu irrigation project has got its own uniqueness which will necessitate an individual approach towards its site selection, design, construction and maintenance.

## Evaluation of the Selected Weir Site

The Consultant has evaluated the selected weir site based on standard criteria encompassing the following;

- General requirement

- Hydraulic requirement

- Topographic requirement

- Geotechnical requirement

Whilst the other requirements are self explanatory, the general requirement is concerned with the assertion of gravity diversion at the lowest water level in the river. Further, this requirement encompasses water management plan of the river catchment such as mitigating backwater effects and water quality degradation at the source. From topography point of view, the present weir site is located at the piedmont region of the hills in the area. The average elevation of the site is about 2800m a.m.s.l. This particular location does not give much freedom to relocate the weir site further downstream without the losing some part of the irrigable land.

Therefore, based on the above criteria and constraints the final evaluation of the weir site for Alentu was found acceptable.

## Weir Type and Headwork Layout Alternatives

### General

For the present project different alternatives were brought into perspective. The alternatives are concerned with the selection of the type of diversion structure and headwork layout alternatives particularly to overcome the problem of installing the intake structure for water users on either side of the riverbank.

Figure-6 gives the adopted head work layout for Alentu Project.

Figure 6: adopted head work layout for Alentu project

### Type of Weir

For Alentu project three types of weirs will be considered. Broadly speaking this type of weirs can be classified from functional and/or construction material point of view thus,

- Masonry weir

- Gabion Weir

- Rock fill weir

- Masonry Weir

When efficiently, designed, constructed and operated masonry weirs are appropriate for small diversion works. Further other head work components such as intakes and under sluices can easily be made the integral part of masonry weir. In addition construction of masonry weir does not call for specialized workmanship and once adequately constructed the operation and maintenance cost of masonry weir is minimal.

With minimum training and assistance from local technicians the farmers themselves could easily operate the intake.

- Gabion Weir

Gabions are frequently used as river training works, check dams and as low sill built to reduce the longitude slope of streams. When used as weir structure gabion has got significant advantages compared to conventional types of weirs.

Major advantages of gabion weirs are that they are flexible and are adaptable to local topography and to possible foundation settlement. Being highly pervious they are not subjected to uplift pressures thus their stability could be insured with moderate volume of material. In addition, gabions are produced using medium sized rocks and wire mesh with relatively simple technology. Moreover, gabion structures are easily integrated into various environmental conditions and, if well executed, can be aesthetic.

The weakest point of gabion weir is the resistance of the wire mesh; if part of the mesh of a gabion is broken, the rock-fill can escape and the gabion empties; if one gabion fails, others will follow, thus endangering the entire structure

Rock-Fill Weir

Overflow earth and/or rock-fill weirs are the cheapest types of weirs and could be        constructed totally by local material. Usually rock-fill and/or overfill earth weirs are adopted as auxiliary spill ways which are rarely over flowed where the unit discharge do not exceed 5 m3/s/m length of the weir crest. One major problem with this type of weir, when used as diversion structure, is its lack of resistance to the erosive action of the overflowing nappy which in retrospect calls for carefully placed large sized riprap stone throughout the exposed surface of the weir body. Moreover, in order to attenuate stilling work will be required. In addition, to maintain the anticipated discharge coefficient of the weir and to attain a degree of imperiousness the entire crest and the vertical face of the weir shall be lined with concrete copping, thereby boosting the capital investment.

The merits and demerit of these alternative weir types were ranked from the point of view of technical suitability, cost of operation and maintenance and social and environmental friendliness. Table-15 gives the Consultants comparison and evaluation of the different water abstraction alternatives for Alentu project.

Table 15: Consultant’s comparison and evaluation water abstraction alternatives

| Type of Weir | Technical Suitability | Cost |  |  | Social Friendliness | Rank |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Construction | Maintenance | Operation |  |  |
| Masonry | VH | H | OPT | OPT | FRIENDLY | 1 |
| Gabion | VH | OPT | OPT | OPT | FRIENDLY | 3 |
| Rock-Fill | VH | OPT | OPT | H | FRIENDLY | 2 |

## Engineering of Major Works

### Hydrology

At the inception level it was proposed that the diversion structure will be designed for 1:50 years return flood and its adequacy will be checked for 1:100 years return flood. However after careful deliberation on the matter and after examining the general site condition the Consultant is of the opinion that constructing a diversion structure with a 1:50 years flood would be economically prohibitive. Therefore, the Consultant has revised his design criteria by designing the weir for 1:10 year flood and checking its adequacy for 1:25 year flood.

Table 16: gives the computed flow volume of Alentu stream at the diversion site

| C | Area (km2) | q10 (m3/s) | q25 (m3/s) | q50(m3/s) |
| --- | --- | --- | --- | --- |
| 0.2 | 2.1 | 6.76 | 8.56 | 10.21 |

### Engineering Geology

In general, the proposed headwork site is underlined by rock. This shows that Alentu is a stream that typically has little to no alluvium mantling. At the proposed weir site has medium river gradient and bed is rock. The prominent rock type present at the weir site is thick bedded, moderately jointed light gray to white and hard ignimbrite rock. The spacing of the joints varies from 50-100 cm in the river bed and 30-50 cm in river banks.

The rock is also exposed, without overburden material, in the upstream and downstream side of the proposed weir site. The intact strength of the exposed bed rock has been checked using Digital Test hammer along the weir axis and river banks, and thus the average compressive strength of the rock was found to be 55 N/mm2. Hence the foundation has to be considered as relatively impervious.

Either of the river banks has moderate to steep slope with exposed rock. Since the proposed weir is located on bed rock. Figure-7 shows the geological cross section at the weir axis.

Figure 7: geological cross section at the weir axis

### Hydraulics

#### Surface Hydraulics

The analysis of surface hydraulics is meant to determine flow conditions upstream and downstream of the weir at different flow rates and to size the different part of the structure accordingly, so that the structure serves the purpose it is built for. Further, the water surface profile upstream of the weir is required to determine the safe level of the embankment and superstructure and to find out whether the canal intake for which the weir is built receives water at all stages of the river.

#### General Design Considerations

The surface flow in the vicinity of the weir is highly affected by the shape and geometry of the weir which again has a far reaching consequence on the hydraulic performance of the weir and project cost.  Major features of the weir that will affect the structures hydraulic performance will be dealt as below;

Crest Elevation

The crest level of the weir will be fixed by considering the canal intake requirement as well as the topographic situation at the weir site in relation to command area.

Figure below gives the procedure followed to fix the weir crest level.

As the crest elevation of the weir affect the height of the weir in addition to the intake requirement particular attention will be given to the backwater effect (afflux) upstream of the weir and the shape and location of the hydraulic jump downstream of the weir.

The backwater curve at the upstream side of the weir will be traced using the equation;

[dy/dx]		=	[So-Sf]/[1+d(v2/2g)/dy]

Where;		dy/dx	 = Water Surface Gradient

So 	= Bed Slope

Sf	= Friction slope

- = Velocity distribution factor

d	= Flow depth

v	= Velocity

g	= Acceleration due to gravity

The above equation will be solved by the direct step method.

Length of the Weir

While fixing the length of the weir physical features of the site as well as its effect on upstream water head and sedimentation will be considered. Although a weir with longer length would result in a smaller discharge per unit length which again results in smaller energy dissipation devise than a shorter weir, particular attention will be given not to disturb the river morphology. This is mainly to avoid the formation of island in front of the intake and to avoid the cutoff of the intake from the river flow.

Weir Shape

While choosing the shape of the weir the following two important factors will be scrutinized;

- Practicality during construction and ease of operation by farmer

- Economy or cost of the structure

At this juncture it is worth to mention that worldwide two types of weirs are very common in small scale irrigation schemes viz.

- Concrete weir with vertical upstream and sloping apron

- Stepped weir constructed out of gabions

Although the type of weir material is to be decided at a later stage, it would be awkward to proposed ogee-shaped weir in such remote area just for the sake of having high discharge coefficient.

At this stage it is incepted that the most probable type of weir for the present work would be a  broad crested weir with a horizontal crest between two vertical abutment walls and a vertical and/or sloping aprons on either side.

The discharge over the weir will be computed from;

Q=CLHe3/2

Where; 	Q = Discharge

L = Length of weir

He= Height of energy line above the crest

= V2/2g+Hd

C= Discharge coefficient

Once the weir section is designed its hydraulic performance will be evaluated by the boundary drag method.

Hydraulic Performance Evaluation

For flow profile computation in the immediate vicinity of hydraulic structures the Consultant will adopt a mathematical model developed based on the boundary drag theory. This mathematical model will also be used to calculate the head-discharge relationship for the weir and its appurtenants.

Description of the Model

This mathematical procedure due to Replogle, 1975, aims at computing two empirical coefficients Cdand Cv (i.e. discharge and velocity coefficients). To arrive at these empirical coefficients it would be necessary to compute the associated head losses in the approach channel, the control section and the tail water channel. This associated head loss is mainly due to friction and is computed by the boundary drag method.

The boundary drag method calls for the computation of the combined laminar and turbulent coefficients which are related to the absolute roughness coefficient and the Reynolds number. The Reynolds number further calls for the knowledge of the kinematic viscosity of the fluid. For this reason the range of values given in Table-17  are adopted to determine the absolute roughness coefficient of materials.

Table 17: absolute roughness of materials

| Material | Range of "k" Absolute Roughness Height (m) |
| --- | --- |
| Glass | 0.000001 - 0.000010 |
| Metal-Painted or Smooth | 0.000020 - 0.000100 |
| Metal - Rough | 0.000100 - 0.001000 |
| Wood | 0.000200 - 0.001000 |
| Concrete - Smooth Troweled | 0.000100 - 0.002000 |
| Concrete - Rough* | 0.000500 - 0.005000 |

* Range of Values To Be Used in Prest Project

Together with the absolute roughness coefficient the kinematic viscosity of water will be fixed to be 1.14e-6 m2/s and the velocity distribution factor will be taken to be 1.

Once the values of Cd and Cv are computed the model generates the rating table for the desired flow range. Parallel with the rating table, the model gives important hydraulic parameters associated with weir structure and its appurtenants such as, discharge coefficient, velocity coefficient, maximum tail water depth, the required head loss and the modular limit of the flow.

## Downstream Hydraulics

Construction of a weir will elevate the total energy line upstream of the structure. As a result the difference between the upstream and downstream energy level will be high. To void damage to be caused by the excess energy at the downstream side of the structure, it would be necessary to dissipate excess energy while the flow transits across the structure.

### Water Surface at Downstream

Downstream water surface profile is required to;

- Carryout stability analysis of the weir

- Design the weir structurally

- Design the downstream wing wall and protection works downstream of the apron.

The downstream hydraulic analysis will be made for Qmax which correspond to 25 to 50 years return period flood and for a flow which correspond to a certain draught level. In due course the formation and location of the hydraulic jump will be determined from basic principles (Bernoulli equation). Depending on the type and location of the jump appropriate type of energy dissipating structure will be selected.

The hydraulic jump profile computation will be carried out for 10 to 25 years return period floodand will consider the following two cases are:

- When there is no either concentration factor flow for cross flow and retrogression of the downstream channel.

- When concentration of flow and retrogression are considered.

The basic parameters which are required to define the hydraulic jump profile will be;

- Discharge per unit width of the crest

- Approach velocity in the upstream channel

- High flood level

- Downstream tail water level

- Weir cross sectional profile

Once the input parameters are obtained the Froude Number of the flow will be computed and the location, height and length of the jump will be obtained from standard monographs or can be determined from standard hydraulic jump computation procedures. Based on the type and nature of the hydraulic jump, appropriate type of stilling basin will be provided as per the USBR standards.

Table-18 gives the spread sheet analysis and design of surface hydraulics for Alentu Weir.

Alentu Weir Tail Water Computation Sheet

| Parameters |  |  |  | Q 50 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Q |  |  |  | 10.21 |  |  |
| n |  |  |  | 0.03 |  |  |
| Z |  |  |  | - |  |  |
| S |  |  |  | 0.001 |  |  |
| b |  |  |  | 9 |  |  |
| A |  |  |  | 9.00 |  |  |
| P |  |  |  | 11.00 |  |  |
| R |  |  |  | 0.82 |  |  |
| T |  |  |  | 9.00 |  |  |
| A/T |  |  |  | 1.00 |  |  |
| q=unit discharge |  |  |  | 1.13 |  |  |
| Qn/S^0.5 |  |  |  | 8.50 |  |  |
| AR^.666 |  |  |  | 7.87 |  |  |
| Yn |  |  |  | 1.00 |  |  |
| Vn |  |  |  | 1.13 |  |  |
| hv=V^2/2g |  |  |  | 0.0656 |  |  |
| Fr= v/sqrt(g*(A/T)) |  |  |  | 0.36 |  |  |
| b/Yn ratio |  |  |  | 9.0 |  |  |
| end of computation |  |  |  |  |  |  |
| Table 18: surface hydraulics design (Alentu weir) Alentu SSIP, Surface Hydraulics Design For Weir. Gravity const.; 9.81. Qd; 10.21; m3/s. Design head and critical depth over the weir crest. Regime width →B=4.75√Q. Length of weir; B=; 15.18. try l=; L; 9.00; trial. Cd; Cd; 1.70. Q=CLHd^1.5. Design head; Hd; 0.76. Unit discharge; q; 1.13. yc=(q^2/9.81)^(1/3); Yc; 0.51. "H"=1.5yc; Hd; 0.76. Crest level; 2806.96; data. Max water level; 2807.72. BLu; 2800.13; data. ht of weir; P; 6.83. Yu=; P+Hd; 7.59. Vu; q/(P+Hd); 0.15. D = n; 0.25; Calc started with nil. Vu^2/2g; 0.00114. E=D+Yu+v^2/2g; 7.84. q2/2g; 0.06559. F(y)=0; -1.36; F(y)=y1^3-Ey1^2+q2/2g; Goal seek. y1; 0.44. V1=q/y1; 2.58. Fr=V1/Sqrt(g*y1); 1.25. y2; 0.58; .5y1[-1+(1+8Fr^2)^.5]. v2; 1.94. Tail water depth; Yd; 1.00; see tail water depth sheet. where n= end sill depth; Yd+n; 1.25. If Y2<Yd+D ok; OK. (Yd+n)-y2; 0.67. Basin floor level; 2799.88. d/s water level; 2801.13. Crest submergence "Sm"; 5.83; negative submerged. IS Sm=2/3yc; 0.34; allowable submergence. Stilling basin for horizontal channel. For Fr1 <3 use K=4. L=K(y2-y1); K; 4.00; data from Chow figure 15.4. L=; 0.58. Use; 1.00; m. toe curve radius =R>4y1; R=; 1.76. |  |  |  |  |  |  |  |

Figure 8: reproduced from ven. T chow

Produced f

### Sluice Gates (Scouring Sluice)

Based on the sediment yield of the catchment, at the present stage a scouring sluice is not provided as a head work component Alentu project. Should the Client insist on the provision of the same the Consultant will follow the following design methodology.

The function of the scouring sluice is to prevent high sediment loads entering the intake. There are no standard technical criteria to fix the dimension of the sluice. Hence the Consultant will adopt the relationship developed based on experimental data. Thus;

Qs=0.196+0.48Qc

Where 	Qs = Discharge through the sluice

Qc = Discharge through canal

The above relationship appears to be suitable for most seasonal rivers and will be used for the present project.

The discharge through the sluice gate will be computed from a submerged orifice gate formula.

### Intake

To control the amount of flow in to the main canal most of the time the intake structure will be gated. The associated intake component could be either an open intake with bay type structure or closed or culvert intake. If open intake is the choice the intake will be designed as using the orifice formula and the hydraulics of the downstream floor of the intake would be treated in the same way as the main weir downstream.

In the case of a closed intake the flow through the culvert could be classified as inlet or outlet controlled. The hydraulic design will best be approximated by considering the culvert to flow full and the downstream floor of the intake would be treated in the same way as that of open intake.

## Structural Design of the Weir &Appurtenants

### Sub-Surface Hydraulic

The weir structures will be subject to sub-surface hydraulic analysis. Sub-surface hydraulic analysis and design will be made to check the adequacy of structure against the prevailing uplift pressure that is responsible to create excess over turning moment that could endanger the stability of the weir. Parallel with this, from the results of the sub-surface hydraulics study the piping effect of water seeping beneath the structure will be studied and adequate measures will be taken to safeguard the weir from the adverse effect of piping that would endanger the stability of the structure by washing away bed materials. The following two major cases will be treated under the sub-surface hydraulics design;

i.	Piping

ii.	Uplift Pressure

To quantify piping, up-lift pressure and the safe exit gradient Khosla’s method of independent variables will be used by splitting up the base of the structure into simple standard forms of known analytical solutions. The sub-surface hydraulics analysis and design result for Alentu weir is given in Table-19.

Table 19: sub-surface analysis & design for Alentu weir

| d1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| U/s head | Hd |  |  | 0.76 |  |  | data |  |  |  |  |  |  |  |  |  |  |  |
| Floor level depressed | n |  |  | 0.25 |  |  | data |  |  |  |  |  |  |  |  |  |  |  |
| Input data from Hydraulic design of the weir |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | L1 |  |  | 12.30 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | b |  |  | 0.00 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | L2 |  |  | 1.30 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | L3 |  |  | 0.00 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | n |  |  | 0.25 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | d1 |  |  | 0.50 |  |  | from cad drwg |  |  |  |  |  |  |  |  |  |  |  |
|  | d2 |  |  | 0.60 |  |  | from cad drwg |  |  |  |  |  |  |  |  |  |  |  |
|  | H= |  |  | 1.01 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| sum of vertical contact=V | d1+d2 |  |  | 2.200 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | L1+L2 |  |  | 13.600 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1/3 of sum of horz contact | 1/3N= |  |  | 4.533 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Lw= |  |  | 6.733 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Koslas Exit gradient GE | GE= |  |  | 0.400 |  |  | 1/ |  |  | 3 |  |  | Rock |  |  |  |  |  |
| the exit gredint GE=H/d2*(1/pi*√λ) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| where λ=1/2 [1+√(1+α^2)] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| α=b/d2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | H= |  |  | 1.01 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | b= |  |  | 13.600 |  |  | total length of impervious floor |  |  |  |  |  |  |  |  |  |  |  |
|  | d2= |  |  | 0.60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | α=b/d2 |  |  | 22.667 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | λ= |  |  | 11.8444 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Ge= |  |  | 0.1557 |  |  | 1/ |  |  | 6.4 |  |  |  |  |  |  |  |  |
| If computed Ge<Ge " ok" |  |  |  | OK. |  |  |  |  |  |  |  |  |  |  |  |  |
| UPLIFT CALC OF FLOOR SLAB AND DETERMINE FLOOR THICKNESS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Pressure head at a =ha=H(1-La/Lw) where H =the total head differ and La =length of weighted creep up to point "a" the same applies to point "b" |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Therefore La= | 4.533 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Lw= | 6.733 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Presure head at a=ha= | 0.330 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Lb= | 4.283 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Presure head at b=hb= | 0.368 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Thickness of floor t=FS[ha/(Sm-1)] where FS is the factor of safety |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| and Sm is the specific gravity |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| FS= | 1.00 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Sm= | 2.6 |  |  | Stone Masonry |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ta= | 0.21 |  |  | min. 0.3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| tb= | 0.23 |  |  | min. 0.3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| end of computation |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

### Structural Analysis and Design

Structural analysis and design will be made to quantify the different stress, strains and bending moment that will come on the structure and to proportion the structural/appurtenant dimensions in such a way that the structures would be safe and stable under the effect of conceivable loads.

Earth Retaining Structures

By and large most of the load bearing structural components in the weir and its appurtenants will be made up of stone masonry in mortar. All earth retaining structures will be made to sustain by their self-weight (gravity retaining walls). For the general structural analysis and design of the scheme the Figure is shown below. The stability analysis for side and wing walls of Alentu Weir has been also shown.

| Table 20: Alentu weir general calculation of free standing rationing wall |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | W4. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | p |  | Surcharge |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | W3' |  |  |  |  |  |  |  |  |  |  | HOR. PRESSURE |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  | H/3 |  |  |  |  | H2 |  |  |  |  |  |  |  |  |
|  |  |  |  |  | WL |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | H W1 |  | w3'' |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | W5 |  |  |  |  |  | H1 |  |  |  |  |  |  |  |  |  |  |  |
|  | w2 |  |  |  |  |  | 2H/3 |  | H3 |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  | H4 |  |  |  |  |  | H5 |  |  |  |  |  |
|  | x |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | q |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | q2=RC base slab |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Abutment Wall |  | Calculation of free standing masonry retaining wall. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Height |  | H= |  | 8 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Top width |  | p= |  | 0.5 |  | data |  |  |  |  |  |  |  |  |  |  |  |
| Bottom width |  | q= |  | 1.75 |  | data |  |  |  |  |  |  |  |  |  |  |  |
|  |  | q2= |  | 0 |  |  | (not applicble) |  |  |  |  |  |  |  |  |  |  |
| thickness of slab=t |  | t= |  | 0 |  | data | (not applicble) |  |  |  |  |  |  |  |  |  |  |
|  |  | T=q+q2= |  | 1.75 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Surcharge (KN/m2) |  | S= |  | 10 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Soil unit wt (KN/m3) |  | Gama dry= |  | 18 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Soil unit wt (KN/m3) |  | Gama wet= |  | 8 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Concrete unit wt (KN/m3) |  |  |  | 24 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Masonry unit wt (KN/m3) |  | Gama m= |  | 21 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Water unit wt (KN/m3) |  | Gama m= |  | 10 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Angle of reposelamd |  | deg. |  | 30 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| =1-SIN(Lam)/(1+SIN(Lam)) |  |  |  | 0.3333 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Friction factor |  | f= |  | 0.4 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| weights |  | force |  | Moment arm |  | Resisting moment | Horizontal forces | force |  |  | Moment arm |  |  | Overturning moment |  |  |  |
| w1 |  | 84.000 |  | 0.250 |  | 21.000 | H1 | 0.000 |  |  | 0.000 |  |  | 0.000 |  |  |  |
| w2 |  | 105.000 |  | 0.917 |  | 96.250 | H2 | 0.000 |  |  | 0.000 |  |  | 0.000 |  |  |  |
| w3' |  | 30.000 |  | 0.917 |  | 27.500 | H3 | 0.000 |  |  | 0.000 |  |  | 0.000 |  |  |  |
| w3" |  | 20.000 |  | 1.472 |  | 29.444 | H4 | 0.000 |  |  | 0.000 |  |  | 0.000 |  |  |  |
| w3"' |  | 40.000 |  | 1.472 |  | 58.889 | H5 | 0.000 |  |  | 0.000 |  |  | 0.000 |  |  |  |
| w4 |  | 12.500 |  | 1.125 |  | 14.063 | SUM | 0.00 |  |  |  |  |  | 1.000 |  |  |  |
| w5 |  | 0.000 |  | 1.750 |  | 0.000 |  |  |  |  |  |  |  |  |  |  |  |
| SUM |  | 291.500 |  |  |  | 247.146 |  |  |  |  |  |  |  |  |  |  |  |
| x bar |  | 0.8478 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Result Moment |  |  |  | 8.92 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| eccentricty e=sum M/sum V |  |  |  | 0.00 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | <q/6 |  | 0.29 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | check if <q/6 |  | OK |  |  |  |  |  |  |  |  |  |  |  |  |  |
| F.S against overturning |  |  |  | 247.15 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Rec. F.S |  | 1.5 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | check |  | OK |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Chechk for Over turning |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Fs = Mr/Mo = |  | 247.1458 |  | > 1.5 |  | Ok! |  |  |  |  |  |  |  |  |  |  |  |
| Chechk for Stress |  | σz(u/s)= |  | 168.5306 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | σz(d/s)= |  | 164.6122 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| (Horizontal forces are ignored the weir will be built on solid rock on either side) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| end of computation |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

## Cross Drainage Structure

### General

To irrigate the left side command area at Alentuthe main canal has to traverse through arduous and topographically very andulating terrain. At Km 1+000 the canal has to cross over a relatively large natural drainage channel in order to convey the diverted water to the desired point of discharge. The top width of this natural drainage channel over which the main canal has to pass over is 140m.

Figure 9 gives the longitudinal profile of the cross drainage structure at Km 1+000/

Figure 9: longitudinal profile of the pump and raiser main.

The main pipe line is designed as a pressure conduit flowing full. The geometric dimensions of the pipe are fixed in such a way that excessive rough turbulence is avoided and friction and other local losses are taken care of. The total head loss across the crossing is computed to be 2.41m, which necessitates a drop structure immediately downstream of the crossing.

| YBC-Acad- Alentu Project Hydraulic Design Cross Drainage |  |  |  |  |
| --- | --- | --- | --- | --- |
| g | 9.81 |
| n | 0.0128 |
| L | 140 |
| Q | 0.03 |
| D | 0.2 |
| A | 0.03142 |
| v | 0.954806 |
| hi | 0.023233 |
| hf | 2.336792 |
| ho | 0.046466 |
| ht | 2.41 |

# IRRIGATION SYSTEM

## Introduction

Based on topography a total of 15 ha of land are found suitable for gravity irrigation on the right bank of Alentu stream with possible expansion to 25 ha. On the left bank of the stream a gravity system is envisaged to irrigate a gross area of 25ha. This will bring the total irrigable area at Alentu to 50ha.

The right bank irrigable area will be supplied by one main canal which will supply irrigation water to tertiary canals. The same scenario applies for the left bank irrigable area. The two main canals will supply water to 27 tertiary canals. Each tertiary command unit will be a self contained irrigation sub-system within the primary command unit it is found in.

### System Layout

#### Topography

In order to prepare a realistic system layout it would be inevitable to scrutinize the local topographic condition of the area. For this purpose and as part of the present feasibility study, detailed topographic survey was conducted.

The topography of the project area is characterized by the difference between the Southern end which has a very steep high mountainous terrain upto 3460m a.m.s.l in altitude and  the Northern part which is rolling plain now and then dissected by streams, gorges, ravins and hill side relief forms at an average altitude of 2800m a.m.s.l

Reservation Width

Prior to the irrigation system layout design, it was necessary to standardize the relative centre to centre distances and in extension the reservation width to be maintained between the different components of the system i.e. canals, drains and roads etc.

While fixing the reservation widths for the various components, the following aspects were taken into consideration;

Earlier experience from similar projects

- Type of canals and drains

- Type of road (access truck)

Accordingly, the centre to centre distance between each component of the system is made to maintain sufficient horizontal relative positions in such a way that the final finished components would lie within the anticipated finished line as given in the typical cross section of the components.

Hydraulic requirement was not a dictating factor for the relative horizontal distance between components.

Figures-9 gives the typical cross sections adopted for relative horizontal positioning of components.

Figure 10: Typical Cross Section Adopted

5,000 Max

Main Canal

C/L

Tertiary

Canal

C/L

3,000 Max

Tertiary

Canal

C/L

Field Canal

C/.L

7,000 Max

Tertiary

Canal

C/L

Access

Road

C/.L

4000

4,000

Tertiary

Canal

C/L

Tertiary

Drainl

C/L

3,000

Field Drain

C/.L

Field Canal

C/.L

#### Project Layout

The irrigation system layout for the project was made based on the 1:1,000 scale topographic maps (with 0.5m contour interval) of the area prepared by the Consultant. The topography of the area is steep with an average slope of more than20 % falling towards the river on either side of the bank.  Besides localized dunes and hills ranging and few natural depressionsand rolling features the command area is devoid of other topographic barriers.

The steep slope along the contour lines coupled with the infiltration rate of the soils, which is from medium to moderately high, does not allow to align furrows along the natural slope i.e. perpendicular to contours. To this effect, to layout the irrigation canals, furrows are made to run more or less parallel to contours and field canals are aligned perpendicular to contour lines, subsequent higher level canals were made to run perpendicular and/or near perpendicular to lower level canals that they discharge into. The alignment of drainage canals is made to follow the alignment of irrigation canals.

As the topography of the area has an average slope of 20% conservation based agriculture is a must. In principle when the terrain slope is more than 5 to 8% and when there is minimum moisture deficit in an area to build artificial application of water is not advisable. As it is evident from the layout and the designed longitudinal profile of the canals the possibility of irrigating 55 to 75 ha of land by gravity alone is nearly impossible.

Figure 11: project layout

#### System Nomenclature

##### General

The multitude irrigation and drainage canals including their appurtenant hydraulic structures in the present irrigation system will call for proper designation and naming of each canal, drain, road and their accompanying structures in a systematic way. Ultimately every drawing will be prepared in relation to the designation and naming of each canal, drain, road and their accompanying structures. For this purpose it will be inevitable to designate and name every canal, drain and road in the irrigation system in a systematic and standard manner.

##### Definition of Irrigation Canals

Main Canal:	A canal taking-off irrigation water from the diversion head works and supplying water to primary canals.

Tertiary Canal:	A canal taking off irrigation water from the main/primary canal and supplying water to two or more field canals.

Field Canal:	A canal taking off irrigation water from a tertiary canal and supplying water directly to farm fields.

##### Definition of Drainage Canals

Tertiary Drain	:	A drain that collects rejected water from field drains and convey the same to secondary and/or natural drain.

Field Drain	:	A drain that collects rejected water directly from farm fields and convey the same to tertiary drains

##### Definition of Roads

Connecting Road:	A road that already exists and run along a main supply canals on either side of the stream and convey traffic from outside the irrigation area.

Farm Road:	A road that runs along tertiary canals and convey traffic to and from connecting roads.

#### Nomenclature

##### Irrigation Canals

Main Canal:	There exist two main canals in Alentu Irrigation Project. These main canals are located on either side of the stream and are designated as MC-ALR and MC-ALLrespectively. Where MC indicates that the canal is a main canal and ALR and ALL indicates the project area Alentu Right and Left respectively

Tertiary Canals:	Tertiary canals are given the name of the main canal it takes off from suffixed sequential number indicating the appearance of the canal along the flow direction.

#### Drainage Canals

Tertiary Drain:	Tertiary drains are given the name of the secondary drain they dewaters in to from followed by sequence number to indicate the number of the tertiary Thus , TD-ALR-1indicates the first tertiary in Alentu  area on the right side of the stream.

### System Description

#### General

Based on the water abstraction system of the irrigable area the command area for Alentu project is divided into two main command units. The first main command unit comprises a total of 25 ha land (left Side) while the second main command unit comprises a total of 25 ha land (Right Side).

#### Irrigation Sub-System

#### Main Command Units

The entire command area at Alentuis supplied with irrigation water by two main canals designated as MC-ALR and MC-ALL and start at the diversion outlet. The main canalsare designed for 24 hrs contentious supply.

Based on the geographic setting of the command area the command area for Alentu is divided into 24tertiary command units on the right bank and 5 tertiary command units on the left bank. Each tertiary command unit will be supplied by one tertiary canal. Each of the tertiary canals will take off from the main canal to convey water to subsequent field canals.

#### Tertiary Command Units

To facilitate a systematic classification for design and management purposes each main command unit is sub divided into tertiary command units. Each tertiary command unit is a self contained irrigation sub-system within the main command unit it is found in. Based on social aspects and topography and size of irrigable area 34tertiary command units are defined.

Unless situation dictates, each of the tertiary command units is supplied with irrigation water via one tertiary canal which takes off from either of the main canals. As a result the designation of the tertiary command units will be similar to the tertiary canal they are supplied from.

#### Drainage Sub-System

To evacuate excess irrigation water and rainfall runoff a network of drainage channels are provided. At field and tertiary level, the designation, arrangement and layout of the present drainage system more or less follows the designation, arrangement and layout of the irrigation system. As each tertiary command unit is a self contained system, the higher level drainage canal that exists in the system is tertiary drain.

For the systematic execution of the design work for drainage channels, the Consultant has followed a descritized system approach. For this purpose the available tertiary drains in the system were used to define the various drainage units.

#### Road System

The present irrigation area is provided with limited road systems. The connecting road which runs parallel to the main canals and serves as a major artery to connect the project area with the nearest national highway.

By and large, the following types of roads are defined for the present system:

Field Roads:	Field roads will run parallel to shallow field drains and will have 3m width. Traffic could enter the fields at the end of tertiary canals via these local widening. In principle, field roads will not be surfaced and are not shown on the project layout map.

Access Road:	Access roads run alongside the main canals and are meant to link the connecting roads with the nearest national highway.

### Irrigation System Design

#### General

The design of irrigation canals is mainly concerned with the adoption of proper canal geometry to accommodate the design discharge within the specified limits and at the same time to allow gravitational flow of irrigation water to farm fields. This situation will call the close scrutiny of the farm fields which are to be irrigated as the level of the farm field is the major parameter that would influence the relative vertical positioning of irrigation canals in the system.

Prior to the design of the longitudinal profile of irrigation canals it would be necessary to define the design flow in each irrigation canal and to proportion the geometric parameters of the canal in such a way that it is capable to carry the design flow within acceptable tolerances.

#### Canal Geometry Design

To proportion the geometric dimensions of all irrigation canals Manning method is employed. To solve the resulting equation in terms of the canal bed width and depth of flow a bed width depth of flow ratio (b:d) of 1. 5 to 1.75is used.

Q = 1/n [R2/3 S1/2]*A

Where;		Q= discharge (m3/s)

n= Manning’s Roughness coefficient

R= Hydraulic radius

S= Bed slope

A= Area (m2)

#### Manning’s Roughness Coefficient

The roughness coefficient recommended by USBR design standard No. 3 is used to derive the roughness coefficients to be used in the present project. USBR recommends a value of n of 0.014 for a canal with a concrete lining with a hydraulic radius, R, of less 1.2 m for a clean and straight canal. For the present work, this recommended value of “n” is adjusted to meet the situation of the project to arrive at a reasonable “n” value.

The adjustment of the USBR “n” value to the project was made taking the following particulars into consideration;

Channel alignment (occurrence of bending points)

Type of canal material (lined/unlined)

Seasonal variability of flow resistance

Table-21 gives the derived value of roughness coefficient for lined and unlined canals.

Table 21: derived value of roughness coefficient for lined and unlined canals

| Canal | Lining | Standard “n” | Adjusted “n” |
| --- | --- | --- | --- |
| Main | Lined | 0.014 | 0.020 |
| Tertiary | Lined | 0.014 | 0.020 |

##### Permissible Velocity

The limiting velocity (non erodible velocity) in field canals is deduced based on the tractive force method the values of which are taken as published by Fortier and Scobeyand are shown inTable-21.

#### Flow Profile Computation for Irrigation Canals

Field Canals

Field canals are part of the on farm work development. These canals are the last watercourses that directly supply irrigation water to the farm plots. Irrigation water is supplied to each farm plot by one field canal. Furrows will be fed from field canals using siphons. Hence, the longitudinal profile of the field canal will be standardized in such a way that the hydraulic grade line in the field canals would allow gravitational flow to the farm fields by overcoming the prevailing losses due to siphon (Hsip) and the friction loss (Hfri) until the water reaches the maximum ground level (MGL) in the farm plot.

Due to the commonness of the above-mentioned methodology for all field canals, the entire cross section of all filed canals was standardized allowing the adoption of a typical section for all. Figure-12 gives the typical standardized cross section adopted for field canals.

Figure 12: typical cross section of field canal

##### Flow Profile Design Criteria

In order to arrive at a reliable flow profile in irrigation canals, three design criterion are adopted. These design criterions were deduced based on;

- Hydraulic requirement

- Operation requirement

- Management requirement

Based on these requirements the following design criteria were adopted to determine the flow profile in irrigation canals, thus:

##### Criteria-I

All secondary and tertiary canals are designed to accommodate uniform discharges throughout their stretch while the primaries and main canal are designed for non uniform flow.

##### Criteria-II

When partial supply of the design discharge Qdis is passed through the canal the flow pattern in the canal is considered to be gradually varied flow, which is approximated by M1 flow profile. In canals with partial supply case, this criteria is used to determine the sill level of off-taking structures with due attention to guarantee the modularity of upstream head regulating structure as a result of the back water effect.

All irrigation canals that were supposed to operate at partial supply were checked for partial supply flow condition. For this purpose, the M1 flow profile equation is used. This equation reads;

[dy/dx]		=	[So-Sf]/[1+d(v2/2g)/dy]

Where;

dy/dx	 = Water Surface Gradient

So 	= Bed Slope

##### Criteria-III

When the entire design discharge Qdis is diverted to laterals the flow profile is considered to have zero slopes(pool) throughout the canal stretch. This criterion is used to set up operational rules at a later stage.

#### Main Canals

#### Flow Capacity

Generally, the main canals will flow for 24hr/day and flow will be distributed down each tertiary canal in proportion to the area they irrigate. The two main canals run down the natural slope. To achieve the required head at off-take structures check structures are provided.Table-22 gives flow capacities and canal geometric parameters for the two main canals.

Table 22: sample flow capacity and geometric parameters for main canals

| Turm-out | Q | s | z | 1/n | b | d | v | FB | Bw |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TO-1 | 0.030 | 0.0002 | 1.5 | 50.000 | 0.392 | 0.224 | 0.187 | 0.15 | 0.3 |
| TO-2 | 0.030 | 0.0002 | 1.5 | 50.000 | 0.392 | 0.224 | 0.187 | 0.15 | 0.3 |
| TO-3 | 0.030 | 0.0002 | 1.5 | 50.000 | 0.392 | 0.224 | 0.187 | 0.15 | 0.3 |
| TO-4 | 0.030 | 0.0002 | 1.5 | 50.000 | 0.392 | 0.224 | 0.187 | 0.15 | 0.3 |
| TO-5 | 0.030 | 0.0002 | 1.5 | 50.000 | 0.392 | 0.224 | 0.187 | 0.15 | 0.3 |
| TO-6 | 0.030 | 0.0002 | 1.5 | 50.000 | 0.392 | 0.224 | 0.187 | 0.15 | 0.3 |
| TO-7 | 0.030 | 0.0002 | 1.5 | 50.000 | 0.392 | 0.224 | 0.187 | 0.15 | 0.3 |
| TO-8 | 0.030 | 0.0002 | 1.5 | 50.000 | 0.392 | 0.224 | 0.187 | 0.15 | 0.3 |
| TO-9 | 0.030 | 0.0002 | 1.5 | 50.000 | 0.392 | 0.224 | 0.187 | 0.15 | 0.3 |
| TO-10 | 0.030 | 0.0002 | 1.5 | 50.000 | 0.392 | 0.224 | 0.187 | 0.15 | 0.3 |
| TO-11 | 0.030 | 0.0002 | 1.5 | 50.000 | 0.392 | 0.224 | 0.187 | 0.15 | 0.3 |

#### Flow Regime

The two main canals run along ridges. Due to the topography of the terrain,even though the alignment is along ridges the elevation along the main canals drops very rapidly. From the topographic survey result it was noted that the variation of the ground level along the main canals results in a longitudinal slope of around 18%.  With this respect the flow regime in the main canals is expected to be in the range of critical to supercritical.  To avoid the prevalence of super-critical flow in the main canals it was inevitable to provide drop structures. Where conditions permit the drop structures are combined at the location of check structures in the form of check-drops.

The main canal profile drawings for the project are given in the drawing album.

#### Tertiary Canals

Flow Capacity

Tertiary canals will flow for 24hr/day and flow will be distributed down each off-taking field canals in proportion to the area they irrigate.  In general tertiary canals run along contours (contour canals). To achieve the required head at off-take structures check structures are provided.Table-23 gives flow capacities and canal geometric parameters for representative canal in the priority areas.

Table 23: sample canal geometric design results

| Turn-Out | Q | s | Z | N | Bw | d | v | FB | BTw |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TO-1 | 0.005 | 0.00015 | 1.5 | 50.000 | 0.188 | 0.125 | 0.107 | 0.15 | 0.3 |
| TO-2 | 0.005 | 0.00015 | 1.5 | 50.000 | 0.188 | 0.125 | 0.107 | 0.15 | 0.3 |
| TO-3 | 0.005 | 0.00015 | 1.5 | 50.000 | 0.188 | 0.125 | 0.107 | 0.15 | 0.3 |
| TO-4 | 0.005 | 0.00015 | 1.5 | 50.000 | 0.188 | 0.125 | 0.107 | 0.15 | 0.3 |
| TO-5 | 0.005 | 0.00015 | 1.5 | 50.000 | 0.188 | 0.125 | 0.107 | 0.15 | 0.3 |
| TO-6 | 0.005 | 0.00015 | 1.5 | 50.000 | 0.188 | 0.125 | 0.107 | 0.15 | 0.3 |
| TO-7 | 0.005 | 0.00015 | 1.5 | 50.000 | 0.188 | 0.125 | 0.107 | 0.15 | 0.3 |
| TO-8 | 0.005 | 0.00015 | 1.5 | 50.000 | 0.188 | 0.125 | 0.107 | 0.15 | 0.3 |
| TO-9 | 0.005 | 0.00015 | 1.5 | 50.000 | 0.188 | 0.125 | 0.107 | 0.15 | 0.3 |
| TO-10 | 0.005 | 0.00015 | 1.5 | 50.000 | 0.188 | 0.125 | 0.107 | 0.15 | 0.3 |
| TO-11 | 0.005 | 0.00015 | 1.5 | 50.000 | 0.188 | 0.125 | 0.107 | 0.15 | 0.3 |
| TO-12 | 0.005 | 0.00015 | 1.5 | 50.000 | 0.188 | 0.125 | 0.107 | 0.15 | 0.3 |

Since all tertiary canals in the system are small for the purpose of standardization, ease of operation and construction the minimum flow in tertiary canals is fixed to be 5 l/s. Further, to avoid the provision of safety structures all tertiary canals are designed for uniform flow from head to tail.

Flow Regimes

As tertiary canals are aligned nearly along contours in most cases the flow regime in tertiary canals will be sub-critical. Standard tertiary canal profile drawings for Alentu area showing their capacity, hydraulic property and geometric parameters is included in the drawing album.

### Irrigation Infrastructures

#### General

The selection and design of irrigation infrastructures is primarily dictated by various factors among which are economy, safety and ease of operation and maintenance. By and large the following factors were considered to select the required structure in the present system:

- Operation and maintenance of the structures should be simple

- The structures were evaluated from safety perspectives, such that the system be safe from unforeseen damages

- Availability of construction material in the vicinity of the project site was considered

#### Types of Irrigation Infrastructures

The type and nature of irrigation infrastructures that are adopted in the present system are varied. Some are found in proliferation while others are found in small numbers and/or in isolation. In general the irrigation infrastructures that are found in the present system are classified in to the following three major groups:

- Conveyance structures

- Regulating structures

Under conveyance structures, all off-takes and irrigation culverts are included, while cross regulating structures (head regulators) are placed under regulating structures. By designing each canal for the maximum uniform flow safety structures i.e. side channel spillways are totally avoided from the system.

#### Main Canal Structures

Table-24gives the summary of available secondary canal structures for the project.

Table 24: summary of main canal structures & quantity in the project

| Main Canal | Type of Irrigation Canal Structures & Their Quantity |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Check | Off-Take | RSPW* | Escape | Culvert |
| MC-ALENTU R | 22 | 24 | - | 1 | 1 |
| MC-ALENTU L | 5 | 5 | - | 1 | 1 |
| Total | 27 | 29 | - | 2 | 2 |

RSPW* Rejection Spillway

##### Main Canal Head Regulators

Each of the main canals abstracts water directly from the head work. To maintain the required head to divert flow to laterals (to tertiary canals) the main canals are provided with straight type weirs as head regulating structures. Figure-12 gives typical main canal head regulating structure.

To evaluate the hydraulic performance of main canal head regulating structures the broad crested weir formula was used.

Figure 13: typical main canal check

1:Zd

1:Zu

500 Min.

CL

100x100 AIRATION

GROOVE

500

LONGITUDINAL SECTION

(Scale 1:100)

1:Zd

1:Zu

PLAN

(Scale 1:100)

(HALF PLAN SHOWN W/O BACKFILL)

A

A

750 Min.

200*100 Drainage

Slot

Lu

LTu

Cw

Ln

Lb

LTd

Ld

ECLu

SEE DETAIL-A

(ECLu+0.1)

ECLd

BLu

BLd

FLu

FLd

300 THICK CEMENTED PITCHING

ON CONSOLIDATED GROUND

300 THICK RIP RAP PROTECTION

ON 150 THICK FILTER BEDDING

Tu

Td

Lc

BLu

BLd

ECLu

ECLu

OGL

OGL

BWd/2

BWu/2

n

BL

Cw

Ktd

K1

Main Canal Off-Takes

Main canal conveyance structures that takeoff irrigation water to laterals (to tertiary canals) are made to be standard gated off-takes. To evaluate the hydraulic performance of secondary off-take inlets the broad crested weir formula was used. Figure-13 gives typical main canal off take structure.

Figure 14: typical main canal off take

1:Zu

1:Zu

OGL

ECLu

OGL

BLu

BWu/2

FLOW

(ECLu+0.1)

BLu

SL

IL

BLd

ECLd

FLu

FLd

300 THICK RIP RAP PROTECTION

ON 150 THICK FILTER BEDDING

300 THICK CEMENTED PITCHING

ON CONSOLIDATED GROUND

K2

K1

D

MASS CONCRETE

BEDDING & HAUNCH

50 THICK MASS

CONCRETE BEDDING

300 THICK CEMENTED PITCHING

ON CONSOLIDATED GROUND

Li

Lb

Ktu

Rw

ROAD

C/L

PARENT

C/L

FSLu

200 MIN.

Zu

1

BLd

FLOW

ECLd

Tu

W/2

250 MIN.

100 *120 GATE

GROOVE

100*100

KEY WAY

1000 MIN.

1:Zu

BWd/2

(ECLd+0.1)

Td



Lp

250 MIN.

(HALF PLAN SHOWN W/O BACKFILL)

(Scale 1:1,000)

PLAN

LONGITUDINAL SECTION

(Scale 1:1,000)

A

A

750 MIN.

500

MIN.

CHA

K1

K1

Ltd

Ld

FLin

Lcr

Ktd

#### Tertiary Canal Structures

Table-25 gives the summary of available tertiary canal structures within the three priority areas.

Table 25: summary of tertiary canal structures

| Tertiary Canal Under MC | Type of Irrigation Canal Structures & Their Quantity |  |  |  |
| --- | --- | --- | --- | --- |
|  | Check | Off-Take | RSPW | Escape |
| MC-ALR | 110 - | 110 - | - | 24 - |
| MC-ALL | 35 - | 35 - | - | 5 - |
| Total | 135 - | 145 - | - | 29 - |

##### Tertiary Canal Head Regulators

Tertiary canals are supplied directly from main canals. To maintain the required head to divert flow in to field canals, tertiary are provided with straight type head regulating structures. As most of the tertiary canals run along contour lines the head regulators for tertiary canals are located at his tail end of tertiary canals.

To evaluate the hydraulic performance of tertiary canal straight weir type head regulating structures, the broad crested weir formula is used.

##### Tertiary Off-Takes

Tertiary canals conveyance structures are made to be standard open type off-takes (channel off takes).

To evaluate the hydraulic performance of tertiary and quaternary off-take inlets the broad crested weir formula was used. Figure-15 gives typical tertiary canal off take adopted in the present study.

Figure 15: typical tertiary off take

Li

LTd

ECLd

(ECLd+0.1)

ECLu

(ECLu+0.1)

Parent Canal

C/L

IL

SL

BLu

Ktd Min.

Lb

1:Zd

Zu

FLu

FLd

300 THICK CEMENTED PITCHING

ON CONSOLIDATED GROUND

300 THICK RIP RAP PROTECTION

ON 150 THICK FILTER BEDDING

LONGITUDINAL SECTION

(Scale 1:1,000)

500

CHA

(HALF PLAN SHOWN W/O BACKFILL)

(Scale 1:1,000)

PLAN

BLd

Ld

Ld

Lb

ECLd

FLOW

BLd

B

1:Zd

750

(BWd/2)

Ktu Min.

W/2

Tu

250 Min.

K1

500 Min.

Td

K1

K2

1:Zu

BWu

OGL

1

FLin

### Drainage System Design

#### Design Methodology

The design of drainage canals is mainly concerned with the adoption of proper canal geometry to accommodate the design discharge within the specified limits and at the same time to allow the gravitational flow of the water to be drained. Further, to dissipate excess hydraulic energy at outfall points protection works are provided. Parallel with this, when the topography of the area dictates, drops are provided to create a hydraulic head difference and/or a bed level difference along the course of a drainage canal the adverse effect of which is taken care by the drop structures.

#### Flow Profile Computation for Drainage Canals

##### Design Flows in Drainage Canals

The flow capacity of the drainage canals is assessed based on the results of the drainage duties. Apart from direct rainfall runoff and excess irrigation water to be drained from the farm fields, the drainage canals starting from tertiary drains on will collect irrigation water that would be rejected from the system as a result of mal operation and or accidental events in the irrigation operation. For this reason, the drainage canals starting from tertiary drains on are designed by considering the additional irrigation water to be rejected via irrigation canals.

#### Drainage Canal Geometric Design

To proportion the geometric dimensions of drainage canals Manning’s method is employed. The Manning's method reads as;

Q = 1/n [R2/3 S1/2]*A

Where;		Q= discharge (m3/s)

n= Roughness coefficient (take to be 0.03)

R= Hydraulic radius

S= Bed slope

A= Area (m2)

The canal geometric design results for drainage canals including related hydraulic particulars such as design discharge, flow depth, velocity etc. are given in the standard longitudinal profile drawings included in the drawing album.

#### Field Ditch Profiles

Field ditches are the initial and smallest drainage networks that collect drainage water directly from farm plots. Excess irrigation and rainfall runoff is collected from each farm plot by one field ditch. Due to the compatible size of farm plots to be drained by one field ditch, the general design flow in each field ditch is similar, and this situation would allow the standardization of the cross sections of field ditches. Hence, the longitudinal profile of the field ditches was standardized in such a way that the hydraulic grade line in the field ditch would allow gravitational flow starting from the head end of the field ditch by overcoming the prevailing friction loss along their course until the water reaches the final point of discharge at a tertiary drains. At the same time, it was assured that at any given critical point there is a total 10-15 cm free board between the full flow level in the field ditch and the surrounding ground level.

Figure-16 gives the typical standardized cross section adopted for field ditch profile design.

Figure 16: Typical Field Drain

### Drainage Infrastructure

#### Types of Drainage Infrastructure

The type and nature of the drainage infrastructures that were adopted in the present system have shared hydraulic and structural similarity. Since drains are used to evacuate excess irrigation and rainfall runoff from the higher-level irrigation fields, mostly they are formed in full cut sections.

As such the drainage infrastructures provided in the present system are mainly meant to dissipate the excess potential energy of the drained water while it transits from higher-level out falling drains into lower level collector drains. Sometimes depending on the topography it would be required to create a hydraulic head difference or drain bed level difference along the course of a drainage canal to maintain gravitational flow or to avoid over toping of the drain, in which case drop structures are provided.

In principle, all drainage infrastructures provided are energy dissipaters. Based on the commonality of drainage structures, both structurally and hydraulically, it was found necessary to classify and describe them in terms of their naming rather than their function. Therefore, all drain structures are classified into two groups:

Outfalls Structures

Drop Structures

#### Outfall cum Drop Structures

Outfall structures are structures that are provided at the tail end of each drainage channel to transit the drained water in to subsequent lower leveled collector drains. Drain outfall structures that are found in proliferation are field ditch outfalls. These are provided at the confluence point of the tail of the field ditch and the tertiary drain that it culminates in.

When the topography of the area along which a drainage channel is aligned dictates, drop structures are provided to create a hydraulic head difference and/or a bed level difference along the course of a drainage canal to maintain gravitational flow and/or to maintain the full flow level in the drain below the surrounding ground level. In all cases, in the present system, drop structures are purposely associated with drain outfall structures.

Once constructed outfall cum drop structures will operate without outside intervention, therefore, their hydraulic design mainly focuses in securing the hydraulic head difference to encourage gravitational flow and to provide adequate crest length for their weir portion in such a way that the flow could be accommodated within the available levels. Figure-17shows a typical drain outfall cum drop structure in the area.

Figure 17: typical drainage outfall cum drop structure

### Hydraulics & Structural Design Criteria

#### General

The general engineering analysis and design for the present system include hydraulics and structural analysis and design for each hydraulic structure. While the hydraulic design is concerned with the adoption of appropriate type of structure within the available head tolerances, the structural analysis is meant to guarantee the stability of the structure from conceivable loads.

#### Hydraulics

The hydraulic design of all structures was categorized in to two viz.

- Surface hydraulics

- Sub-surface hydraulics

Surface Hydraulics

Surface hydraulics analysis is made in order to visualize the performance of hydraulic structures within the available head tolerance. Moreover, the plan dimensions of some structural components were fixed from the prevailing surface hydraulic conditions. In the present design the following procedure is used for surface hydraulic design.

##### Off take Structures

In most cases off take structures are made simple openoff takes with a suppressed rectangular inlet part followed by a rectangular bay and a masonry guide walls.

##### Control Inlet Dimension

The control inlet width of off takes is computed using the broad crested weir formula. The final inlet width was fixed based on the greater value of the following two 	limiting cases;

i) According to USBR, the inlet width opening is limited to;

Where; b = Opening width (ft)

Q = Design Discharge (ft3/s)

ii) Inlet velocity should be (Vc/2), where Vc is the critical velocity at the inlet

##### Outlet Velocity

For earth canals, the maximum allowable pipe velocity at the pipe outlet was limited to 1.2 m/s.

##### Head Losses

Three type of losses are considered for the pipe section;

Inlet Loss

Inlet losses are computed from

Where;	v= pipe velocity (m/s)

= contraction coefficient (0.5)

g= acceleration due to gravity (m/s2)

##### Friction (Conveyance) Loss

The friction loss is computed from

Where;	 n= Friction coefficient (0.013 for concrete)

v= Pipe velocity (m/s)

g= Acceleration due to gravity (m/s2)

L= Pipe length (m)

R= Hydraulic radius of the pipe (m)

##### Outlet Loss

The outlet loss is computed from;

Where; v= Pipe velocity (m/s)

= Expansion coefficient (1.0)

g = Acceleration due to gravity (m/s2)

##### Bay Gradient

To avoid stagnation of water in the bay the bay will have a minimum gradient of 5% along the flow direction

##### Slide Gates

One type of gate is provided for main and tertiaryoff take structures viz.

Simple manually operated slide gates

Simple Manually Operated Slide Gates

- These gates are made to be locally manufactured and manually operated gates. The major components of these gates are angle irons and plates and 	individual members are connected using simple welds.

- These gates are to be operated to have either a fully opened or closed positionand at the same time provision is made to open the gates at intermediate positions.

To avoid misappropriate actions the gates are provided with protective lugs

For ease of operation and maneuverability, the total area of the gates is limited to 0.4 – 0.5 m2. This will result in a total gate weight of 40-50 kg which is the lifting capacity of one person at normal conditions.

At normal conditions a friction coefficient of 0.2 (steel to steel) is used to estimate the effort required to lift the gates, and at dry condition (where operation is made after a prolonged period of time) a friction coefficient of 0.4 is adopted.

##### Check Structures

In the present system there is one type of head regulating structure namely straight type weir. The control section of all head regulators is designed using the broad crested weir formula while other structural components are fixed based on the prevailing flow condition.

Straight Weir

The head discharge relationship and in extension the control section of straight weirs is computed from the broad crested weir formula which reads;

Where; Q= design discharge (m3/s)

Cd= discharge coefficient (1.706)

Lc= crest length (m)

H= upstream sill referenced head (m)

##### Stilling Basin

All straight weirs and drop structures for drains are provided with a straight drop immediately after the control section. All drop associated with straight weirs and drop structures are designed using the drop number according to Rand. Subsequently the following parametric relationship is defined;

Where; D = Drop number

q = the unit weir discharge over the weir (m2/s)

h = Drop height (m)

g= acceleration due to gravity (m/s2)

Based on the drop number the downstream hydraulic phenomenon was computed as a function of the drop number;

### Stone Protection Works

#### General

To protect the channel bed from being eroded by the current leaving the solid apron of the structure the channel bed and bank immediately downstream of the structure are protected by stone work. On the upstream side of the structure, in order to maintain a smooth transition stone works are provided. In the present system the following three types of stone works are provided viz.

- Riprap protection

- Dry stone pitching

- Cemented stone pitching

#### Riprap Protection

In general randomly dumped riprap will be used to protect canal beds and banks. The following guiding rule is used to limit the riprap protection length, however as per the circumstances the designer could limit the protection length based on the prevailing hydraulic condition and the flow nature;

L= 3.00q 2/3-1.5dd or L>4*ddwhichever is greater

Where; L = Downstream length of protection (m)

q= Discharge per unit width of the canal (m2/s)

dd= Tail water depth (m)

#### Dry Stone and Cemented Stone Pitching

In general well dressed stones shall be used as in dry and/or cemented stone pitching works and the same shall carefully be placed in position. The following guiding rule is used to limit the riprap protection length, however as per the circumstances the designer could limit the protection length based on the prevailing hydraulic condition and the flow nature;

L= 2.55q 2/3-1.5dd/s or L>4*ddwhichever is greater

Where; L = Downstream length of protection (m)

q= Discharge per unit width of the canal (m2/s)

dd= Tail water depth or upstream water depth (m)

c.	Layer Thickness

Unless indicated in the drawings, in general the thickness of stone protection works is limited to 300mm. Where filter bedding is to be provided the thickness of the same is limited to 150mm.

Where excess uplift pressure is expected energy relief holes in the form of weep holes are provided. The arrangement and size of weep holes are as indicated in the drawings.

### Sub-Surface Hydraulics

#### General

Sub-surface hydraulic analysis and design is made to check the adequacy of structural components against the prevailing uplift pressure that is responsible to create excess over turning moment that could endanger the stability of the structure. Parallel with this, from the results of the sub-surface hydraulics study the piping effect of water seeping beneath the structure is studied and adequate measures were taken to safeguard the structure from the adverse effect of piping that could endanger the stability of the structure by washing away bed materials. The following two major cases were treated under sub-surface hydraulics design;

i.	Piping

ii.	Uplift Pressure

#### Piping

Safety against piping was assured by providing sufficient creep length beneaththe structure. The following two methods were used to compute the safe creep length and the safe exist gradient of the foundation particle.

##### Conventional Method (Lane’s Creep Ratio)

To compute the safe creep length the following mathematical relationship 	due to Lane is adopted;

Where; C = Weighted creep ratio

L = Length of horizontal or vertical floor (m)

Factor= 1 for vertical face

= 1/3 for horizontal face

= 2 for undisturbed soil

The computed weighted creep ratio was compared with Lane’s weighted creep ratio as given in Table-26.

Table 26: lane's safe weighted creep ratio

| Material | C |
| --- | --- |
| Very fine sand or silt | 1/8.5 |
| Fine sand | 1/7 |
| Medium sand | 1/6 |
| Coarse sand | 1/5 |
| Fine gravel | 1/4 |
| Medium gravel | 1/3.5 |
| Gravel and sand | 1/3 |
| Coarse gravel including cobbles | 1/3 |
| Boulders with some cobble and gravel | 1/2 |
| Soft clay | 1/3 |
| Medium clay | 1/1.8 |
| Hard clay | 1/6 |

Table 27: safe exit gradient values

| Type of Soil | Safe “Ge” |
| --- | --- |
| Shingle | 0.25-0.20 |
| Coarse sand | 0.20-0.17 |
| Fine sand | 0.17-0.14 |

#### Uplift

The type and nature of the farm structures does not call for rigorous analysis using independent variables. Therefore, due to its simplicity and reasonable accuracy the conventional method due to Bligh was used.

Accordingly the vertical uplift pressure at any distance “x” from the upstream end of the structure will be;

Where; hx = Uplift pressure at distance “x” (m)

H = Maximum differential head (m)

#### Floor Thickness

Once the uplift pressure at key points is found the floor thickness of the structure is computed from;

Where; tx = Floor thickness at point “x” (m)

hx = Uplift pressure at distance “x” (m)

(r-1)= Buoyant unit weight of floor material (t/m3)

#### Structural Analysis and Design

General

Structural analysis and design was made to quantify the different stress, strains and bending moment that will come on the structure and accordingly to proportion the structural dimensions in such a way that the structure would be safe and stable under the effect of conceivable loads.

Retaining Walls

By and large most of the loads bearing structural components in the present system are made up of stone masonry in mortar.

For the general analysis and design of masonry components the following procedure was followed.

Assumptions

2/3 of the soil mass from the bottom of the component is assumed to be submerged

The stabilizing effect of surcharge loads is ignored and the triangular wage of soil mass on the tapering portion of the wall is taken to assist the stabilizing effect

The passive earth pressure is assumed to be counterbalanced by the active earth pressure and is ignored in the analysis.

Material Data

The following material properties were adopted;

Unit weight of masonry 		= 2.1 t/m3

Unit weight of concrete		= 2.6 t/m3

Unit weight of dry soil			= 1.9 t/m3

Unit weight of saturated soil		= 1.2 t/m3

Unit weight of road base material 	= 2.1 t/m3

Unit weight of Water			= 1.0 t/m3

The active internal pressure is worked from;

With the preliminary data and assumptions at hand the base width of the component is worked out from the first principles and the safety of the same is checked for different governing conditions.

##### Safety against Tension

Safety against tension was assured by observing the following requirement;

Where; e = Eccentricity of the resultant from the centre

K = Bottom width of wall (m)

##### Safety against Sliding

To secure the structure’s against sliding a general safety factor of 1.5 – 2.0 is adopted;

Where; FSlide= Factor of safety against sliding

W = Stabilizing force

P = Over turning force

##### Safety against Overturning

To safe guard the structure against overturning  ratio of resisting moment to overturning moment is limited to be between of 1.5 – 2.0;

Where;  FSover= Factor of safety against overturning

WX = Stabilizing resisting moment (N-m)

PY = Overturning moment (N-m)

### Project Design Criteria

Irrigation System

##### IrrigationCanal Design

Manning’s - Coefficient   (n) For Main Canals Straight     ……	0.02

Manning’s - Coefficient n) For Q <0.5 m3/s	……………	50

Maximum Design Velocity (Tertiary/Secondary) Canals	…….	0.7 m/s

Minimum Design Velocity (Tertiary/Secondary)	…………….	0.15 m/s

Maximum Tractive Force	………………………………………. 	2-3 N/m2

Inner Bank Slope (Main/Tertiary)…………………………..….	1:1.5

Outer Bank Slope	………………………………………………….1:1.5

##### Free Board

Main & Tertiary Canals………………………….…………	0.15 m

##### Maximum Slope Seepage Line	……………………	1:4

Minimum Radius of Bend w/o Protection	……………	8*T

(Where T is top width of design water surface)

##### Bank Top Width (BTW)

Tertiary Canals	…………………………………..	0.3 m

Main Canals ……………………………………………	0.3 m

Drainage System

##### Manning’sCoefficient

Manning’s - Coefficient (n) Field/Tertiary Drains……………	0.03

Maximum Design Velocity (Tertiary)	…………………..…….	0.5 m/s

Minimum Design Velocity (Tertiary)	………………………...	0.15 m/s

Inner Bank Slope (Tertiary Drains)	………………………….	1:1.5

##### Free Board

Field Drains		……………………………………	0.10 m

Tertiary Drains	……………………………………	0.15 m

Maximum Slope Seepage Line	……………………	1:4

Minimum Radius of Bend w/o Protection	……………	8*T

(Where T is top width of design water surface)

On Farm Works

Length of Field Canals	……………………………..…..	40 m max.

Furrow Slope	…………………………………………..………	0.1 - 0.5 %

Furrow Length	………………………………………….	10-40m

Concrete & Steel

Mass Concrete Grade……………………………………	C15 (ESCP)

Reinforced Concrete Grade	…………………………..	C25 (ESCP)

Concrete Material Quality	…………………………..	As Per ESCP

Characteristic Strength of Steel	………………….	220 N/mm2

Limiting Velocity (All Earth Canals)

The limiting velocity (non erodible velocity) in all artificial channels will be assessed based on the tractive force of the flow and will be taken as published by Fortier and Scobey. Table-28 gives the maximum velocity as published by Fortier and Scobey.

Table 28: limiting velocities in non-vegetated channels

| Original Material | Manning's "n" | Permissible Velocity (m/s) |  |  |
| --- | --- | --- | --- | --- |
|  |  | Clear Water | Water Transporting Colloidal Silt | Water Transporting Sand, Gravel Fragments |
| Fine sand, colloids | 0.020 | 0.45 | 0.75 | 0.45 |
| Sandy loam, non-colloidal | 0.020 | 0.53 | 0.75 | 0.60 |
| Silt loam, non-colloidal | 0.020 | 0.60 | 0.90 | 0.60 |
| Alluvial silt, non-colloidal | 0.020 | 0.060 | 1.05 | 0.60 |
| Ordinary firm loam | 0.020 | 0.75 | 1.05 | 0.68 |
| Volcanic ash | 0.020 | 0.75 | 1.05 | 0.60 |
| Stiff clay, very colloidal | 0.025 | 1.13 | 1.50 | 0.90 |
| Alluvial silt, colloidal | 0.025 | 1.13 | 1.50 | 0.90 |
| Shale’s and hard pans | 0.025 | 1.80 | 1.80 | 1.50 |
| Fine gravel | 0.020 | 0.75 | 1.50 | 1.13 |
| Graded loam to cobbles colloidal | 0.030 | 1.13 | 1.50 | 0.90 |
| Graded loam to cobbles, non-colloidal | 0.030 | 1.20 | 1.65 | 1.50 |
| Coarse gravel, non-colloidal | 0.025 | 1.20 | 1.80 | 1.95 |
| Cobbles and shingles | 0.035 | 1.80 | 1.80 | 1.50 |

(Fortier and Scobey, 1925)

# Bill of Quantity and Cost Estimates

# Alentu SSIP (Dodola District, West Arsi Zone)

| No | Types of Work | Unit | Quantity | Unit Cost (Birr) | Total Cost (Birr) | Community Share |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | Mobilization & Demobilization (fifty-fifty in weight) | LS | 1 | 30,000.00 | 30,000 |  |
| 2 | Access Road Clearance into and within the project area | km | 3 | 2,000.00 | 6,000.00 |  |
| 3 | Camp Construction |  |  |  | 0 |  |
|  | Living Rooms 2 (4m x 4m) |  |  |  | 0 |  |
|  | Office 1 (4m x 4m) |  |  |  | 0 |  |
|  | Store 1 (5m x 5m) |  |  |  | 0 |  |
|  | Kitchen (3m x 3m) |  |  |  | 0 |  |
|  | Toilet (2m x2m) |  |  |  | 0 |  |
|  | Guard House (2m x 2m) |  |  |  | 0 |  |
|  | Fence (50m x 50m) |  |  |  | 0 |  |
| 3.1 | Site clearance | m2 | 96 | 7 | 672 |  |
| 3.2 | Excavations on soft formations | m3 | 45 | 55 | 2475 |  |
| 3.3 | Cart away surplus excavations | m3 | 13 | 40 | 520 |  |
| 3.4 | Masonry (1:3 mortar) | m3 | 32 | 1,200.00 | 38400 |  |
| 3.5 | Hard coring with hard rock | m3 | 26 | 475 | 12350 |  |
| 3.6 | Concrete (1:3:4) | m3 | 10 | 1,950.00 | 19500 |  |
| 3.7 | Plastering (1:2 mortar, 3cm thick in two rounds) | m2 | 50 | 120 | 6000 |  |
| 3.8 | CIS walling (G-32) | m2 | 290 | 150 | 43500 |  |
| 3.9 | CIS roofing (G-32) | m2 | 160 | 165 | 26400 |  |
| 3.1 | Fencing with 10cm diameter termite resistent poles of 3m tall at 1m spacing with barbed wires of 6-lines | m | 160 | 8 | 1280 |  |
| 3.11 | Back filling and compactions | m3 | 12 | 45 | 540 |  |
|  | Sub Total A |  |  |  | 181,637.00 | 0 |
| 4 | Suitable temporary river water diversion channel excavation and compacted refilling (Fifty fifty) | LS | 1 | 20,000.00 | 20,000.00 |  |
|  | Sub Total B |  |  |  | 20,000.00 |  |
| 5 | Water Diversion Concrete Weir Construction |  |  |  | 0 |  |
| 5.1 | Excavations on soft formations | m3 | 110 | 55 | 6,050.00 |  |
| 5.2 | Excavations on soft rock formations | m3 | 110 | 130 | 14,300.00 |  |
| 5.3 | Excavation on hard rock formation | m3 | 440 | 280 | 123,200.00 |  |
| 5.4 | Hard coring (20cm) mingled with mortar (1:2) | m3 | 48 | 600 | 28,800.00 |  |
| 5.5 | Masonry (1:2 mortar) | m3 | 66 | 1,200.00 | 79,200.00 |  |
| 5.6 | Concrete (1:2:4) for weir body, cut offs, sulice way columns, platform and abutments | m3 | 195 | 2,000.00 | 390,000.00 |  |
| 5.7 | Supply and fix properly reinforcing bars with stirrups (10mm diameter encasing the whole weir outline, crossing at 50cm spacings both ways) | Kg | 800 | 40 | 32,000.00 |  |
| 5.8 | Supply and fix properly reinforcing bars for intake and sluice way columns, abutments and platform with stirrups (10mm diameter at 30cm spacing crossing ) | Kg | 200 | 40 | 8,000.00 |  |
| 5.9 | Plastering the weir body surfaces (1:2 mortar, 3cm thick) | m2 | 85 | 120 | 10,200.00 |  |
| 5.1 | Pointing critical joints (1:2 mortar) | m2 | 45 | 80 | 3,600.00 |  |
| 5.11 | Intake gate (3mm thick, 60cm X 60cm, and supported by 10mm bars, with 2.80m high frame of 5mm thickness and 40mm spindle) supply and proper installation | Pcs | 2 | 20,000.00 | 40,000.00 |  |
| 5.12 | Silt sluice gate (3mm thick, 1.5m X1.0m, and supported by 10mm bars, with 3.20m high frame of 5mm thickness and 40mm spindle) supply and proper installation | Pcs | 1 | 22,000.00 | 22,000.00 |  |
| 5.13 | Compacted back filling | m3 | 35 | 45 | 1,575.00 |  |
|  | Sub Total C |  |  |  | 758,925.00 |  |
| 6 | Retaining Wall Construction |  |  |  | 0 |  |
| 6.1 | Excavations on soft formations | m3 | 100 | 55 | 5,500.00 |  |
| 6.2 | Excavations on soft rock formations | m3 | 150 | 130 | 19,500.00 |  |
| 6.3 | Excavations on hard rock formations | m3 | 180 | 280 | 50,400.00 |  |
| 6.4 | Hard coring with hard rock | m3 | 9 | 475 | 4,275.00 |  |
| 6.5 | Concrete (1:3:5) over the hardcore | m3 | 7 | 2,000.00 | 14,000.00 |  |
| 6.6 | Masonry (1:2 mortar) | m3 | 84 | 1,200.00 | 100,800.00 |  |
| 6.7 | Reinforcement bar for joining the wall concrete with the weir's floor (10mm dia, single layer at 50cm distances) with stirrups and black wire | Kg | 150 | 40 | 6,000.00 |  |
| 6.8 | Plastering (1:2 mortar, 3cm thick at two rounds) | m2 | 200 | 120 | 24,000.00 |  |
| 6.9 | Compacted back filling | m3 | 110 | 45 | 4,950.00 |  |
|  | Sub Total D |  |  |  | 229,425.00 |  |
| 7 | Main Canals (Right and Left) Construction |  |  |  |  |  |
| 7.1 | Excavations on soft formations | m3 | 1800 | 55 | 99,000.00 |  |
| 7.2 | Excavations on soft rock formations | m3 | 900 | 130 | 117,000.00 |  |
| 7.3 | Excavations on hard rock formations | m3 | 2300 | 280 | 644,000.00 |  |
| 7.4 | Hard coring with hard rock (20cm) | m3 | 760 | 475 | 361,000.00 |  |
| 7.5 | Concrete (1:3:4) for canal bed and under the walls (10cm thick) | m3 | 380 | 1,950.00 | 741,000.00 |  |
| 7.6 | Masonry (1:3 mortar) with hard rock | m3 | 912 | 1,200.00 | 1,094,400.00 |  |
| 7.7 | Plastering (1:2 mortar, 3cm thick at two rounds) the inner and top of the walls | m2 | 5320 | 120 | 638,400.00 |  |
| 7.8 | Compacted back filling | m3 | 1520 | 45 | 68,400.00 |  |
|  | Sub Total E |  |  |  | 3,763,200.00 |  |
| 8 | Water Division Structures Construction (12 in No) |  |  |  |  |  |
| 8.1 | Excavations for the structure | m3 | 207.8 | 55 | 11,431.20 |  |
| 8.2 | Hard coring with hard rock (20cm) | m3 | 31.2 | 475 | 14,820.00 |  |
| 8.3 | Concrete (1:3:4) for canal bed and under the walls | m3 | 15.6 | 1,950.00 | 30,420.00 |  |
| 8.4 | Masonry (1:3 mortar) with hard rock | m3 | 41.8 | 1,200.00 | 50,112.00 |  |
| 8.5 | Plastering (1:2 mortar, 3cm thick at two rounds) the inner and topof the walls | m2 | 147.9 | 120 | 17,748.00 |  |
| 8.6 | Gates (sliding in frames, 3mm thick, 45cm x 45cm) supply and installation | Pcs | 48 | 2,500.00 | 120,000.00 |  |
| 8.7 | Compacted back filling | m3 | 47 | 45 | 2,114.10 |  |
|  | Sub Total F |  |  |  | 246,645.30 |  |
| 9 | Road Crossing Structures Construction (6m in span, 4 in No) |  |  |  |  |  |
| 9.1 | Excavations for the structure | m3 | 57.1 | 55 | 3,141.60 |  |
| 9.2 | Hard coring with hard rock (20cm) | m3 | 6.7 | 475 | 3,192.00 |  |
| 9.3 | Concrete (1:3:4) for bed and under the walls | m3 | 5 | 1,950.00 | 9,828.00 |  |
| 9.4 | Masonry (1:3 mortar) for the walls with hard rock | m3 | 19.3 | 1,200.00 | 23,184.00 |  |
| 9.5 | Plastering (1:2 mortar, 3cm thick at two rounds) the inner faces of the walls | m2 | 89.6 | 120 | 10,752.00 |  |
| 9.6 | Reinforced concrete slab (1:2:4) (1.2m x 7m, 16cm thick, 10mm bars with 0.25m spaings in both directions in double layers) | Pcs | 4 | 6,000.00 | 24,000.00 |  |
| 9.7 | Compacted back filling | m3 | 15.4 | 45 | 693 |  |
|  | Sub Total G |  |  |  | 74,790.60 |  |
| 10 | Cattle Crossing Structures Construction (4m in span, 10 in No) |  |  |  | 0 |  |
| 10.1 | Excavations for the structure | m3 | 81.6 | 55 | 4,488.00 |  |
| 10.2 | Hard coring with hard rock (20cm) | m3 | 9.6 | 475 | 4,560.00 |  |
| 10.3 | Concrete (1:3:4) for bed and under the side walls | m3 | 7.2 | 1,950.00 | 14,040.00 |  |
| 10.4 | Masonry (1:3 mortar) for the walls with hard rock | m3 | 27.6 | 1,200.00 | 33,120.00 |  |
| 10.5 | Plastering (1:2 mortar, 3cm thick at two rounds) the inner faces of the walls | m2 | 128 | 120 | 15,360.00 |  |
| 10.6 | Reinforced concrete slab (1:2:4) (1.2m x 4m, 12cm thick, 8mm bars with 0.25m spaings in both directions in a single layer) | Pcs | 10 | 3,000.00 | 30,000.00 |  |
| 10.7 | Compacted back filling | m3 | 22 | 45 | 990 |  |
|  | Sub Total H |  |  |  | 102,558.00 |  |
| 11 | Foot/Flood Passage Structures Construction (2.6m in span, 6 in No) |  |  |  |  |  |
| 11.1 | Excavations for the structure | m3 | 31.8 | 55 | 1,750.32 |  |
| 11.2 | Hard coring with hard rock (20cm) | m3 | 3.7 | 475 | 1,778.40 |  |
| 11.3 | Concrete (1:3:4) for bed and under the side walls | m3 | 2.8 | 1,950.00 | 5,475.60 |  |
| 11.4 | Masonry (1:3 mortar) for the walls with hard rock | m3 | 10.8 | 1,200.00 | 12,916.80 |  |
| 11.5 | Plastering (1:2 mortar, 3cm thick at two rounds) the inner faces of the walls | m2 | 49.9 | 120 | 5,990.40 |  |
| 11.6 | Reinforced concrete slab (1:2:4) (1.2m x 4m, 10cm thick, 8mm bars with 0.25m spaings in both directions in a single layer) | Pcs | 6 | 2,500.00 | 15,000.00 |  |
| 11.7 | Compacted back filling | m3 | 8.6 | 45 | 386.1 |  |
|  | Sub Total I |  |  |  | 43,297.62 |  |
| 12 | Drop Structures Construction (1m fall, 10 in No) |  |  |  | 0 |  |
| 12.1 | Excavations for the structure | m3 | 165.8 | 55 | 9,116.25 |  |
| 12.2 | Hard coring with hard rock (20cm) | m3 | 17.1 | 475 | 8,122.50 |  |
| 12.3 | Concrete (1:3:4) for entry, pool and exit bed (15cm thick) | m3 | 12.8 | 1,950.00 | 25,008.75 |  |
| 12.4 | Masonry (1:3 mortar) for the walls with hard rock | m3 | 44.8 | 1,200.00 | 53,712.00 |  |
| 12.5 | Plastering (1:2 mortar, 3cm thick at two rounds) the inner and top faces of the walls | m2 | 157.5 | 120 | 18,900.00 |  |
| 12.6 | Reinforcing bar for the pool, the concrete and bar layer extending under the walls (2.2m x 2.3m, 10mm bars with 0.25m spaings in both directions in a single layer) | Kg | 277.5 | 40 | 11,101.06 |  |
| 12.7 | Compacted back filling | m3 | 25.7 | 45 | 1,157.63 |  |
|  | Sub Total J |  |  |  | 127,118.19 |  |
| 13 | Turnouts Construction (10 in No) |  |  |  |  |  |
| 13.1 | Excavations for the structure | m3 | 107.2 | 55 | 5,896.00 |  |
| 13.2 | Hard coring with hard rock (20cm) | m3 | 14.2 | 475 | 6,745.00 |  |
| 13.3 | Concrete (1:3:4) for bed and under the walls (10cm thick) | m3 | 7.1 | 1,950.00 | 13,845.00 |  |
| 13.4 | Masonry (1:3 mortar) with hard rock | m3 | 16.9 | 1,200.00 | 20,304.00 |  |
| 13.5 | Plastering (1:2 mortar, 3cm thick at two rounds) the inner and top of the walls | m2 | 105.4 | 120 | 12,648.00 |  |
| 13.6 | Gates (sliding in frames, 3mm thick, 40cm x 40cm) supply and installation | Pcs | 30 | 1,100.00 | 33,000.00 |  |
| 13.7 | Compacted back filling | m3 | 17.3 | 45 | 776.25 |  |
|  | Sub Total K |  |  |  | 93,214.25 |  |
| 14 | Canal-side Outlets Construction (4 in No) |  |  |  | 0 |  |
| 14.1 | Excavations for the structure | m3 | 10.5 | 55 | 577.5 |  |
| 14.2 | Hard coring with hard rock (20cm) | m3 | 3.6 | 475 | 1,710.00 |  |
| 14.3 | Concrete (1:3:4) for bed and under the walls (10cm thick) | m3 | 1.8 | 1,950.00 | 3,510.00 |  |
| 14.4 | Masonry (1:3 mortar) with hard rock | m3 | 2.8 | 1,200.00 | 3,326.40 |  |
| 14.5 | Plastering (1:2 mortar, 3cm thick at two rounds) the inner and topof the walls | m2 | 15 | 120 | 1,795.20 |  |
| 14.6 | Gates (sliding in frames, 3mm thick, 40cm x 40cm) supply and installation | Pcs | 4 | 1,100.00 | 4,400.00 |  |
| 14.7 | Compacted back filling | m3 | 2.8 | 45 | 123.75 |  |
|  | Sub Total L |  |  |  | 15,442.85 |  |
| 15 | Culvert Construction (3 in No) |  |  |  |  |  |
| 15.1 | Excavations for the structure | m3 | 17.1 | 55 | 940.5 |  |
| 15.2 | Hard coring with hard rock (20cm) | m3 | 1 | 475 | 456 |  |
| 15.3 | Concrete (1:2:4) for culvert roof, floor and under walls (15cm thick) | m3 | 0.9 | 2,000.00 | 1,800.00 |  |
| 15.4 | Masonry (1:3 mortar) for the walls with hard rock | m3 | 3.3 | 1,200.00 | 3,974.40 |  |
| 15.5 | Plastering (1:2 mortar, 3cm thick at two rounds) the inner and top faces of the walls | m2 | 14.4 | 120 | 1,728.00 |  |
| 15.6 | Reinforcing bar for the culvert roof and floor (10mm bars with 0.25m spaings in both directions in a single layer in each slab) | Kg | 67.6 | 40 | 2,702.46 |  |
| 15.7 | Compacted back filling | m3 | 9 | 45 | 405 |  |
|  | Sub Total M |  |  |  | 12,006.36 |  |
| 16 | Flumes Construction (50m span) |  |  |  |  |  |
| 16.1 | Excavations for the structure | m3 | 32.8 | 55 | 1,804.00 |  |
| 16.2 | Hard coring with hard rock (20cm) | m3 | 4 | 475 | 1,900.00 |  |
| 16.3 | Concrete (1:2:4) for pads, columns, barrel and transitions | m3 | 25.7 | 2,000.00 | 51,400.00 |  |
| 16.4 | Reinforcing bar for the pad (10mm bars with 15cm spaings in both directions in a closed double layer) | Kg | 547.1 | 40 | 21,882.93 |  |
| 16.5 | Reinforcing bar for the columns (10mm bars rounding and 16mm bars standing vertically, with 20cm spaings in both directions in a cylinderical layer) | Kg | 1104 | 40 | 44,160.00 |  |
| 16.6 | Reinforcing bar for the flume barrel (16mm bars at 15cm spacings for barrel bed, 8mm bars for the barrel side walls, and 10mm bars supporting at 20cm spaings) | Kg | 691.8 | 40 | 27,673.76 |  |
| 16.7 | Masonry (1:2 mortar) for the tranisition (2m in length) walls | m3 | 9.7 | 1,200.00 | 11,592.00 |  |
| 16.8 | Plastering (1:2 mortar, 3cm thick at two rounds) all faces of the flume columns and barrel | m2 | 241.5 | 120 | 28,980.00 |  |
| 16.9 | Compacted back filling | m3 | 11.9 | 45 | 534.38 |  |
|  | Sub Total N |  |  |  | 189,927.07 |  |
| 17 | Secondary Canals Excavation (Length = 1512m; 12 in number) |  |  |  |  |  |
| 17.1 | Excavations of the canal with bank sides slopy (1V:0.5H) | m3 | 3000 | 55 | 165,000.00 |  |
| 17.2 | Bank formations by very hard compaction | m3 | 1600 | 45 | 72,000.00 |  |
|  | Sub Total O |  |  |  | 237,000.00 |  |
| 18 | Excavation of drains (Length = 3000m; 8 in number) |  |  |  |  |  |
| 18.1 | Excavations of the canal with bank sides slopy (1V:0.5H) | m3 | 800 | 55 | Community | 44,000.00 |
| 18.2 | Bank formations by very hard compaction | m3 | 600 | 45 | Community | 27,000.00 |
|  | Sub Total P |  |  |  | Community | 71,000.00 |
| 19 | Excavation of runoff diversion channel |  |  |  | 0 |  |
| 19.1 | Excavations of the canal with bank sides slopy (1V:0.5H) | m3 | 375 | 55 | Community | 20,625.00 |
| 19.2 | Bank formations by very hard compaction | m3 | 300 | 45 | Community | 13,500.00 |
|  | Sub Total Q |  |  |  | Community | 34,125.00 |
| 20 | Excavation of tertiary canals (5800m; 27 in No) |  |  |  |  |  |
| 20.1 | Excavations of the canals | m3 | 1400 | 55 | Community | 77,000.00 |
| 20.2 | Bank formations by very hard compaction | m3 | 1000 | 45 | Community | 45,000.00 |
|  | Total Fund Required |  |  |  | 6,095,187.24 | 122,000.00 |
|  | Total Community Share |  |  |  |  | 227,125.00 |
|  | Total Construction Cost |  |  |  | 6,322,312.24 |  |
|  | Supervision Cost |  |  |  | 316,115.61 |  |
|  | Grand Total |  |  |  | 6,638,427.85 |  |
|  | Capital/Hectar |  |  |  | 132,768.56 |  |

Annex 1: Left tertiary canal profiles

Annex 2: Left tertiary canal table of labels and dimensions(off take & checks)

Annex 3: Left tertiary canal hydraulic design results

Annex 4: Left tertiary canal structural design results

Annex 5: Right main canal &tertiary canal profiles

Annex 6: Right tertiary canal table of labels and dimensions (checks & off takes)

Annex 7: Right tertiary canal hydraulic design results

Annex 8: Right tertiary canal structural design results

# OPERATION AND MAINTENANCE MANUAL

## Irrigation System

### General

Based on topography and soils a total of 50 ha of land are found suitable for gravity irrigation at DodolaWoreda of West Arsi Zone at a particular place called Alentu. The irrigable area is located on either side of the AlentuStream. Within the system there exist two main command units which will be suppliedby twenty seven tertiary canals. Each of the main canals will take off from the diversion weir.

### System Description

#### General

Based on the geographic setting of the irrigable area with respect to Alentu Stream, the command area for Alentu project is divided into two main command units. The first main command unit comprises a total of 25 ha land (left Side) while the second main command unit comprises a total of 15 ha land (Right Side)Irrigation Sub-System

#### Main Command Units

The entire command area at Alentu is supplied with irrigation water by two main canals designated as MC-ALR and MC-ALL and start at the diversion outlet. The main canals are designed for 24 hrs contentious supply. Each of the tertiary canals will take off from the main canal to convey water to subsequent field canals.

#### Tertiary Command Units

To facilitate a systematic classification for design and management purposes each main command unit is sub divided into tertiary command units. Each tertiary command unit is a self contained irrigation sub-system within the main command unit it is found in. Based on social aspects and topography and size of irrigable area 27 tertiary command units are defined.

Unless situation dictates, each of the tertiary command units is supplied with irrigation water via one tertiary canal which takes off from either of the main canals. As a result the designation of the tertiary command units will be similar to the tertiary canal they are supplied from.

#### Drainage Sub-System

To evacuate excess irrigation water and rainfall runoff a network of drainage channels are provided. At field and tertiary level, the designation, arrangement and layout of the present drainage system more or less follows the designation, arrangement and layout of the irrigation system. As each tertiary command unit is a self-contained system, the higher level drainage canal that exists in the system is tertiary drain.

For the systematic execution of the design work for drainage channels, the Consultant has followed a discretised system approach. For this purpose the available tertiary drains in the system were used to define the various drainage units.

#### Road System

The present irrigation area is provided with limited road systems. The connecting road which runs parallel to the main canals and serves as a major artery to connect the project area with the nearest national highway.

By and large, the following types of roads are defined for the present system:

Field Roads:	Field roads will run parallel to shallow field drains and will have 3m width. Traffic could enter the fields at the end of tertiary canals via these local widening. In principle, field roads will not be surfaced and are not shown on the project layout map.

Access Road:	Access roads run alongside the main canals and are meant to link the connecting roads with the nearest national highway.

### Definition and Nomenclature

#### General

The multitude irrigation and drainage canals including their appurtenant hydraulic structures in the present irrigation system will call for proper designation and naming of each canal, drain, road and their accompanying structures in a systematic way. Ultimately every drawing will be prepared in relation to the designation and naming of each canal, drain, road and their accompanying structures. For this purpose it will be inevitable to designate and name every canal, drain and road in the irrigation system in a systematic and standard manner.

##### Definition of Irrigation Canals

Main Canal:	A canal taking-off irrigation water from the diversion head works and supplying water to primary canals.

Tertiary Canal:	A canal taking off irrigation water from the main canal and supplying water to two or more field canals.

Field Canal:	A canal taking off irrigation water from a tertiary canal and supplying water directly to farm fields.

##### Definition of Drainage Canals

Tertiary Drain	:	A drain that collects rejected water from field drains and convey the same to secondary and/or natural drain.

Field Drain	:	A drain that collects rejected water directly from farm fields and convey the same to tertiary drains

##### Definition of Roads

Connecting Road:	A road that already exists and run along a main supply canals on either side of the stream and convey traffic from outside the irrigation area.

Farm Road:	A road that runs along tertiary canals and convey traffic to and from connecting roads.

#### Nomenclature

##### Irrigation Canals

Main Canal:  	There exist two main canals in Alentu Irrigation Project. These main canals are located on either side of the stream and are designated as MC-ALR and MC-ALL respectively. Where MC indicates that the canal is a main canal and ALR and ALL indicates the project area Alentu Right and Left respectively

Tertiary Canals: 	Tertiary canals are given the name of the main canal it takes off from suffixed by sequential number indicating the appearance of the canal along the flow direction.

#### Drainage Canals

Tertiary Drain: 	Tertiary drains are given the name of the secondary drain they dewaters in to from followed by sequence number to indicate the number of the tertiary Thus , TD-ALR-1indicates the first tertiary in Alentu area on the right side of the stream.

## Operation of Irrigation Canals

### General

The typical cross sections and arrangement of the various components of the irrigation and drainage system is as given on the respective drawings of the canals and drains. For ease of operation and maintenance of the system it would be necessary to maintain the reservation width of each component as indicated on the drawings. While fixing the reservation widths for the various components, the following aspects were taken into consideration;

Earlier experience from similar projects

- Type of canals and drains (Main, primary, secondary etc.)

- Type of road (Main, connecting, access truck)

- Hydraulic requirement

Accordingly, the centre to centre distance between each component of the system is made to maintain sufficient horizontal relative positions in such a way that the final finished components would lay within the anticipated finished line as given in the typical cross section of the components.

Figure 18: Typical cross section of canal

5,000 Max

Main Canal

C/L

Tertiary

Canal

C/L

3,000 Max

Tertiary

Canal

C/L

Field Canal

C/.L

7,000 Max

Tertiary

Canal

C/L

Access

Road

C/.L

4000

4,000

Tertiary

Canal

C/L

Tertiary

Drainl

C/L

3,000

Field Drain

C/.L

Field Canal

C/.L

### System Layout

The irrigation system layout for the project was made based on the 1:10,000 scale topographic map (with 1m contour interval) of the area prepared by the Consultant from radar data. Figure 25: gives the general system layout for the present area.

### Operation of Irrigation Canals

Main Canals

Table-34 gives the irrigable area commanded by each main canal and the carrying capacity of the same.

Table 29: Main canals and their duty

| Main Canal | Irrigable Area | Discharge |
| --- | --- | --- |
| MCBU-RT | 35 ha | 30 l/s |
| MCBU-LT | 20 ha | 30 l/s |

Both of the main canals are designed for uniform flow along their route. This situation makes primary canals to have uniform shape on plan. Evidently the carrying capacity of the main canals will not vary as one goes from the head end of these canals to their tail. This will call not call close observance of the operation of main canals. Although the primary canals are designed for uniform flow it is always advisable to pass a discharge equal to the design capacity of each reach.

Passing a discharge more than the capacity of any reach could result in overtopping of the canal which gives way to breaching and ultimately the total disruption of irrigation.

Therefore, the head work gate operator should regulate the flow into main canals in such a way that the diverted flow at the main off-take be equal to the irrigation demand. Hence for the safe operation of primary canals the head work gate operator and the main canal gate operator should have a clear communication prior to any operation. For this purpose the application of request form No.1 to 3 are recommended.

### Tertiary Canals

Every tertiary canal has a capacity to reject the total amount of discharge coming in the canals. For this purpose a tail escape is provided at the tail of tertiary canals. Therefore, these canals are less susceptible for damages caused by excess flow. However, it is highly advisable to abstract a discharge equal to the design capacity of each tertiary canal.

### Capacity of Irrigation Canals and Their Operation

For proper irrigation water management it is highly advisable to run any canal at its design capacity. In the present irrigation system a number of tertiary canals are provided to convey water to field canals. The irrigating capacity of each tertiary canal is standardized. The irrigating capacity and method of water application is as given on the respective drawings of each canal.

To convey more water than the capacity of the canals and to serve more than the specified amount of field canals may result in breaching of the canal and inefficient water application respectively.

### Field Canals & Siphons

Filed Canals

Field canals are supposed to carry a total discharge of 5 l/s. The average design discharge head to siphons delivering water to individual furrows from field canals is 10cm above the leveled ground level. In practice this discharge head may be a little on the lower side due to the formation of the field canal banks.

The number of siphons per hectare to be used depends on the siphon diameter that the farmer is to use and the actual available differential head.

In the future the number of siphons needed to convey 5 l/s from the field canal into furrows is to be determined from actual irrigation observation.

Siphons should be laid out along the banks of field canals until the cutback time for irrigation is reached. The siphons that become available after the cutback time of the advance flow should be moved to new furrows and started immediately bringing them too under irrigation with water flowing into the second section of the field canal. The process involving the leap-forging of siphons will continue until the entire field served by the field canal has been irrigated.firest field is nearing completion the tertiary off take should be partially closed and part of the water diverted to the next field canal inlet. This will insure a smooth switch over from one field canal to another and saves valuable time on a tight irrigation schedule.

Field Canal Embankment

In the usual practice field canal embankments are supposed to be formed to the required shape and size after every irrigation season. Hence forming and maintaining field canal embankments is the sole responsibility of the farmer.

### Furrows

Characteristics

Compared to other methods of surface irrigation the furrow method hs distinct advantages. Adequacy and efficiency of irrigation depends considerably on the shape of furrows. The cross section of the furrow should be sufficient to carry a discharge needed to give uniform distribution throughout the furrow.

Furrow Slope

Furrows should have a uniform slope for efficient application of water. Uneven slope causes uneven depths of irrigation along the furrow with erosion on steep areas and deposition in flatter areas. The damage of ponding and injuring the crop is also possible.

Furrow Head

Prior to pre irrigation the upper end of all furrows should be built up manually as close as possible to the field canal banks so as to ensure that each furrow will have its own independent siphon stream.

### Request for Water

In order to avoid excessive rejection of irrigation water and malfunctioning of irrigation canals which again result in damage of embankments and wastage of water all personnel working in the system should give their request for water release and should at least 48 hr notice of the anticipated time for shut down or reduction of flow.

Since individual fields in any unit are at different distance from the main canal supply canal water when released from the main system will take varying time to reach different fields. Due to this fact in order to make use of the backup water that is hold in the canal system and to avoid excess rejection of water in completion of irrigation or when a reduction in demand is required a 48 hr. notice of the anticipated time for shut down or reduction in flow should be given.

In order to establish a suitable information exchange system the following communication means are proposed;

    - Verbal Communication: Verbal communication could be effected either through oral discussions held face to face using walkie-talkies or using telephone line.

    - Oral Communication: This kind of communication could be effective when mutual consent or understanding exists between the different parties involved. However for each oral discussion held both parties should keep a written record concerning the notified time to release water or shut down.

    - Walkie Talkie: This means of communication should be used as a means of communication between main systems and unit feeder systems (tertiary canals). Communication through walkie talkie may be valuable when immediate shutdown or reduction in flow in any lateral is needed.

Form No.1

Alentu Small Scale Irrigation Project

Record of Water Release or Shutdown

Person Keeping the Diary: --------------------------------	Date: ------------------------------------

Request Made By: ------------------------------------------	Farm: -----------------------------------

Oral Request Made to: ------------------------------------

| Name of Canal | Name of Off take | Requested Discharge |  | Period For Release |  | Period of Shutdown |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Release | Shut | From | To | From | To |

Form No.2

Alentu Small Scale Irrigation Project

Request form For Water Requirement

To Farm Manager: ---------------------------------------		Unit: ---------------------------

Request Made By: ------------------------------------------	Date: ------------------------------------

| Tertiary Canal No. | Field No. | Irrig. No. | Required Water l/s | From | To | Remark |
| --- | --- | --- | --- | --- | --- | --- |

Form No.3

Alentu Irrigation Project

Request Form for Water Requirement

To: Control Center

Name of Farm ---------------------------------------

Request Made By: ------------------------------------------	Date: ------------------------------------

| Off take | Discharge (l/s) | Irrigation No. | Seven Day Period |  |
| --- | --- | --- | --- | --- |
|  |  |  | From | To |

## Drainage System

The present system is provided with a network of drainage canals to evacuate excess irrigation and rainfall runoff from the system. For the general description of the same refer the description given thereof.

### Operation of Drainage System

Once effectively constructed the drainage system functions completely by gravity and needs no especial operation.

## Maintenance

Depending upon the nature of maintenance work to be carried out the general maintenance activity is classified in to the following three groups.

- Regular Maintenance

- Periodic Maintenance

- Incidental Maintenance

### Regular & Periodic Maintenance

Regular maintenance should be carried out at regular intervals. As for irrigation canals to avoid the deposition of sediments all irrigation canals should be desalted two times in a year. Moreover primary and secondary canals need a third round desalting per year. After desalting the bed slope as well as the side slopes of the canals should be maintained.

Parallel with this a regular inspection should be made in each canal and all kinds of defects must be maintained right away.

As for drains all drains have to be de-weeded twice a year. After the de-weeding activity bed and side slopes of all drains must be regarded. Except this no extra desiling and grass cutting activity is required.

All structures should be inspected regularly and all remedial measures should be taken immediately.

Cleaning and greasing of gates water level gauges and other movable parts of steel structures must be done every two months and leakages at water stops and joints must be repaired immediately.

### Incidental Maintenance

All maintenance works which are sought necessary due to damage caused by unforeseen circumstances are classified as incidental maintenance. Incidental maintenance which need close attention are;

- Maintaining structures and embankments which are damaged as a result of high flood

- Maintaining structures and embankments which are damaged as a result accident