Amhara National Regional State

Water, Irrigation and Energy Bureau

(BOWIE)

Detail Design

Of

Mayis-2 Small-Scale Irrigation Project

Volume IV: Engineering Design Final Report

October, 2018

Dessie

Client: Bureau of Water, Irrigation and Energy

(BOWIE)

Address:

P. O. Box: 88
Telephone: 0528-200853/855
Fax: 251-08-20-65-68/204676/202040

Consultant: Amhara Design & Supervision Works Enterprise

(ADSWE)

Address:

P. O. Box: 4921
Telephone: +251-333-124954

Fax: (033) 3124954
E-mail:amhara design@yahoo.com

Dessie, Ethiopia

FEASIBILITY STUDY & DETAIL DESIGN REPORT STRUCTURE

- VolumeI: Watershed Management

- VolumeII: Engineering Geology

- VolumeIII: Irrigation Agronomy

- Volume IV: Engineering Design

- VolumeV: Socio Economy

- Volume VI: Environmental Impact Assessment

# TABLE OF CONTENT

Table of Contents

TABLE OF CONTENT	iii

LIST OF TABLE	vi

LIST OF FIGURES	vii

SAILENT FEATURES	viii

1	INTRODUCTION	10

1.1	Background	10

1.2	Description of the Project Area	10

1.2.1	Location and Accessibility	10

1.2.2	Previous Irrigation Practices	11

1.3	Objectives of the Study	12

1.3.1	Major Objective	12

1.3.2	Specific Objectives	12

1.4	Scope of the Study	13

1.5	Methodology	13

SECTION-I: HYDROLOGY	15

2	HYDROLOGY	16

2.1	Watershed Characteristics	16

2.2	Hydro-Metrological Data Availability	17

2.2.1	Climate	17

2.2.2	Rainfall Data	17

2.2.3	Upstream & Downstream Utilization	18

2.3	Design Flood Analysis	18

2.3.1	Rainfall Data Quality Test	18

2.3.2	Design Rainfall Computation	20

2.3.3	Peak Discharge Determination	22

2.3.4	Tail Water Depth Computation	26

SECTION-II: HEAD WORKDESIGN	29

3	HEADWORK STRUCTURES DESIGN	30

3.1	Headwork Site Selection	30

3.2	River Geomorphology	30

3.2.1	River Bank Condition	31

3.3	Sources of Construction Materials	32

3.3.1	Rock For Masonry And Crushed Coarse Aggregate	32

3.3.2	Fine Aggregates	32

3.3.3	Water	33

3.4	Headwork Type Selection	33

3.4.1	Hydraulic Design of Headwork Structure	34

3.4.2	U/S and D/S HFL Calculation & Determination	38

3.4.3	Hydraulic Jump Calculation	39

3.4.4	Impervious floor	39

3.4.5	Cut off Depth Calculation	40

3.4.6	Stability Analysis of weir	41

3.4.7	Design of Divide Wall, Under Sluice, And Canal Outlet	42

3.5	Bill of Quantity and Cost Estimation	48

SECTION-III: IRRIGATION AND DRAINAGE SYSTEMS	53

INFRASTRUCTURE	53

4	IRRIGATION AND DRAINAGE SYSTEMS DESIGN	54

4.1	Irrigable Area Description	54

4.1.1	Topography	54

4.1.2	Climate	54

4.1.3	Soil characteristics	54

4.1.4	Existing Irrigation Practices in the Project Area	54

4.2	Irrigation Water Requirement	55

4.2.1	Crop Water Requirement (CWR)	55

4.2.2	Irrigation efficiency (Ep)	55

4.2.3	Irrigation Duty	56

4.2.4	Irrigation Methods	57

4.2.5	Irrigation and Drainage System Layout	58

4.2.6	Conveyance System	58

4.3	Design of the Canal System	58

4.3.1	Main Canal	58

4.3.2	Secondary Canals	59

4.3.3	Tertiary canals	59

4.3.4	Field Canals	60

4.3.5	Catch Drainage Canals	61

4.4	Canal Structures Design	62

4.4.1	Design of a typical flume	62

4.4.2	Design of division box	65

4.4.3	Design of Turnout	67

4.4.4	Design of Drop Structure	68

4.5	Irrigation Infrastructure Bill of Quantities and Cost Estimate	70

4.6	Summary of Bills	78

5	CONCLUSION AND RECOMMENDATION	79

6	OPERATION AND MAINTENANCE	80

6.1	General	80

6.2	Operation of the Head Works	80

6.3	Irrigation System Operation	80

6.4	Maintenance Requirement	80

REFERENCE	82

# LIST OF TABLE

Table 2-1: Outlier Test Analysis	19

Table 2-2: Test for goodness to fit using D-index	21

Table 2-3: Runoff Analysis	24

Table 2-4Hydrograph Coordinates	25

Table 2-5Weir Site River Cross section Coordinate Data	26

Table 2-6Stage discharge analysis	26

Table 3-1Weir Height Determination	34

Table 3-2: D/S face profile Coordinates of (X, Y)	37

Table 3-3: U/S face profile coordinates(X,Y)	37

Table 3-4 Load calculation of weir at static condition	41

Table 3-5 Computation of forces per unit width and momentum of divide wall	43

Table 3-6Stability analysis of U/S retaining wall	47

Table 3-7 D/s retaining wall Stability Analysis	47

Table 3-8 Bill of Quantity and Cost Estimation of General Item	48

Table 3-9: Bill of Quantity of Head Work Structure	49

Table 4-1 hydraulic parameter for main canal	59

Table 4-2 hydraulic parameter for secondary canal	59

Table 4-3 hydraulic parameter for tertiary canal	60

Table 4-4:- GIS output for flum-2 hydrological analysis	62

Table 4-5:- hydrological analysis of flum-2	62

Table 4-6:- tail water discharge computation of flume 2	63

Table 4-7:- hydraulic parameter of flume	63

Table 4-8:- hydraulic parameter of division box	67

Table 4-9:- hydraulic parameter of turn outs	67

Table 4-10:-hydraulic parameter of drop structure	68

Table 4-11:- irrigation infrastructure BOQ & cost estimation	70

Table 4-12 summary of bill of Quantity	78

# LIST OF FIGURES

Figure 1-1Location map of the project area Accessibility	11

Figure 2-1: Drainage map of Mayis-2 weir Irrigation Project watershed	17

Figure 2-2 Complex Hydrograph	25

Figure 2-3 Rating Curve	27

Figure 2-4 River profile	27

Figure 3-1River bed at the proposed weir site	31

Figure 3-2 Stability analysis of weir at static condition	41

Figure 3-3 Stability analysis of Divide wall	42

Figure 4-1typical cross section of catch drain canals	61

Figure 4-2  live load in flume & Figure 4-3 service bending moment	64

Figure 4-4shear forces diagram (service) & Figure 4-5shear forces diagram (strength)	65

Figure 4-6 pier cap cross section	65

Figure 4-7 division box	66

Figure 4-8 typical cross section of vertical drop structure	69

# SAILENT FEATURES

- Project Name: Mayis-2 Diversion Weir Irrigation Project

- Name of the stream: Mayis River

- Location of the weir site using

- North: 1212325.475

- Easting:  470210.76

- Zone: South wollo

- Woreda: Mehal Sayint

- Average Altitude: 2605.38

- Hydrology

- Design rainfall: 77.55 mm

- Catchment area: 27.29 Km2

- Longest flow path length: 18.7 Km

- Peak Flood: 202.11m3/sec

- Base Flow of the River: 205lit/se.

- Diversion Weir

- Weir type: Ogee weir

- Height: 2.85 m

- Effective crest length: 15 m

- Weir crest level: 2597.54 m.a.s.l

- U/S  HFL: 2600.65 m.a.s.l

- U/S TEL: 2600.92  m.a.s.l

- D/s TEL: 2598.45 m.a.s.l.

- D/s HFL: 2598.19 m.a.s.l

- Afflux: 2.47m

- Under sluice

- Sill level: 2596.19 m.a.s.l

- Dimension: H * W = 0.9 * 1.0m2

- Discharge capacity:1.605 m3/sec

- Outlet

- Sill level: 2597 m.a.s.l

- Opening dimension: H * W = 0.5 * 0.5 m2

- Discharge capacity:0.1166m3/sec

- Irrigation and drainage systems Infrastructure

- Net command area size: 72.45 ha

- Type of soil of the command area is dominantly clay soil

- Design discharge of the main canal = 88.391 l/sec

- Irrigation system layout consists of

- One main lined masonry canal

- Two lined masonry Secondary canal

- 8 Tertiary canals

- 5  Turnouts

- 12 Drop Structures

- Main irrigation structures designed are;

- 3 Flume Structures

- 2 supper passages

- Project Cost

- General Items cost= 1,129,451.44Birr

- Headwork cost =  1,483,002.22 Birr

- Irrigation infrastructure cost =12,100,046.31Birr

- Total project cost = 14,712,499.97Birr

- Cost per hectare = 203,071.08 Birr

# INTRODUCTION

## Background

In Ethiopia, under the prevalent rain-fed agricultural production system, the progressive degradation of the natural resource base, especially in highly vulnerable areas of the highlands coupled with climate variability have aggravated the incidence of poverty and food insecurity. The major source of growth for Ethiopia is still conceived to be the agriculture sector. Hence, this sector has to be insulated from drought shocks through enhanced utilization of the water resource potential of the country, (through development of small-scale irrigation, water harvesting, and on-farm diversification) coupled with strengthened linkages between agriculture and industry (agro-industry), thereby creating a demand for agricultural output. In line with the above, efforts have been made by the government to improve the situation in the country in areas of domestic water supply provision, irrigation, watershed management, etc. The Amhara Water, Irrigation and Energy Bureau is playing its role in the development of small scale irrigation projects in the region. Accordingly, as part of the water sector development program, the office has initiated the study and design of a small scale irrigation scheme on Mayis River signed an agreement with Amhara Design & Supervision Works Enterprise (ADSWE) for the study and design of the project.

## Description of the Project Area

### Location and Accessibility

This irrigation project is located mainly at Alegeta Kebele, Mehal Saint Wereda of south wollo Zone in the Amhara Region. The proposed irrigation project is to be undertaken on mayis River and the headwork structures are specifically located at an altitude of about 2605.38 masl and geographical coordinates of 1212325.475N (UTM) and 470210.76 E (UTM).

Figure 1-1Location map of the project area Accessibility

From the Woreda capital, the project Headwork site is found at 9 km to the north-east direction with dry weathered road and before we cross Mayis river turn to the right with the farm lands and the remain  0.5 km needs access road up to the head work site.

### Previous Irrigation Practices

There are not traditional diversions on the river using different irrigation practices. The base flow of the river at the upstream is much small because of sufficient tributary and narrows of catchment area. But there is a recharge on the left and right bank of the river at the downstream. In the other hand, there is a modern irrigation system means using a lined masonry canal on the left side of river. So this new irrigation project solves this social problem be making modern irrigation system for the right side. So, the farmers in the project area are very much interested to upgrading the traditional scheme to modern scheme.

## Objectives of the Study

### Major Objective

The project area faces variability of rainfall distribution though the overall rainfall generally suffices the rain-fed agriculture. Accordingly, the rain-fed agriculture needs means of supplementing during distribution failures and further full irrigation is required to maximize the use of the potential land and water resources.

Hence the objective of this project is to contribute a substantial share in the effort to reduce the risk of production decrease due to rainfall variability and increase the productivity of the resource in the project specific area. Specifically, the project is targeted for the following.

- To make sustainable the rain-fed crop production and make extra production in the dry season possible for 72.45 ha of land through irrigation.

- There is a general consensus that irrigation investments will achieve broader food security and poverty reduction impacts and if efforts are also geared towards up-grading existing traditional farming practices with support to enhance access to input supply, output marketing and extension to facilitate access to information and innovations.

- This objective is to be realized by constructing diversion weir structures across the Mayis River and diverting the River flow.

### Specific Objectives

Other benefits that can be expected to appear with the launching of the project are:

- Efficiency of water use improvement;

- Improved local nutrition/food security gains;

- Improved management of scarce natural resources (land and water);

- Resilience against drought risk;

- Rationale for erosion control and watershed management;

- Rationale for the intensification and modernization of small-holder agriculture and rural lifestyles.

The engineering study and design enables the realization of the project by the provision of engineering structures that will allow the appropriate abstraction of the MayisRiver water for delivery in to the identified irrigation fields of the study area. Hence, this engineering design is specifically targeted to:

- Analyze hydrologic requirements of the project and engineering structures;

- The formulation of sound and stable structure,  with necessary provisions that allow safe, easy and low-maintenance operation in the service life of the project;

- Develop working drawings;

- Estimation of construction costs.

## Scope of the Study

- The irrigation design shall ensure reliability, equity and flexibility of water delivery to farmers. It will aim at reducing conflicts among water users and will lead to lower operation and maintenance costs.

- Updating the existing, if available, computation of the actual evapotranspiration, crop water requirement, irrigation demand/duty using the existing and recent agronomic, climatologic and soil data using more appropriate methodologies.

- Establish design criteria for irrigations structures to be approved by the client and to be used in the final design stage,

- Design proper irrigation system compatible with local conditions and management capabilities,

- Establish flood protection measures for the command area and canal structures and design the respective drainage system accordingly,

- Planning  and layout of the irrigation system, which include irrigation canals, drainage channels, inspection roads and alignments, canal spacing, canal length, location of structures, and water profiles along canal and drains at specified reaches, which is most economical easily manageable and aligned with topographic feature and geological investigation.

- Determination and estimation of water application conveyance and other losses and irrigation efficiencies and consideration of those parameters in design steps.

- Check and test hydraulic and structural designs of main canal considering total demand and the required capacity and the base flow availability,

- Prepare general plans and drawings for all irrigation infrastructure and irrigation systems designs,

## Methodology

In the study and design procedure the following steps are included.

- Specific Site Identification:

  - Review of the reconnaissance survey conducted by the Client

  - 50,000 scale top map and GIS information

  - Local farmers interview and discussion

  - Woreda and Zone Agriculture section expertise

  - Previous studies

  - On foot travel along the river channel and farm areas.

- Topographic Survey:

  - Surveying the headwork site and the Command area with sufficient radius, using Total station

- Flow Estimation

  - Physical observation on flood mark indications and local information about high flood and critical flow condition of the river

  - Analyzing the recorded river flow data and use watershed inputs for further analysis.

  - Base flow estimated during the reconnaissance field visit by floating method.

- Irrigable Area Identification:

  - Using local information

  - 50,000 Topographic map, and GIS information, GPS to see elevation

The design report is organized in three sections. In Section I the Hydrology study is presented and in Sections II and III the Headwork and Irrigation and Drainage Systems designs are discussed respectively. In Section III, planning and design of the irrigation system after diverting the water using the weir will be dealt. The following are major areas of concern in this part.

- Study and design of the irrigation method to be adopted,

- Study and design of the irrigation system layout and associated structures,

- Design of the different conveyance canals,

- Planning and design of the different irrigation and drainage structures,

- Preparation of the longitudinal profiles of the different irrigation and drainage canals.

# SECTION-I: HYDROLOGY

# HYDROLOGY

## Watershed Characteristics

The topographic feature of Mayis-2 Watershed encompasses all types of slop classes from flat to steep slope. The dominant slope classes are moderately steep (15-30%), slopping (8—15%) steep (30-50%) 49.6%, 25.8% and 14.7% of the total area of a watershed respectively. The rest gently slopes, flat and steep area sharing 7.4%, 0.8% and 1.6 % of the total area of the watershed respectively.

Study Report of the same project. In summary:

- Catchment Area  = 27.29 km2

- Stream Length = 18.70Km

- CN(III) = 87.88

(Extracted from the Watershed Study Report of the same project)

At the selected reference point, the area of Mayis-2 catchment is 27.29 km2 and consists of a network of tributaries as shown in Figure 2.1 below.

Mayis River at the headwork site is characterized by well-defined channel system and considerable flows. It looks that the gradient of the river/ is getting low and hence there exists significant deposition of sediment.

| No | Slope Slope Class (%) |  | Area Coverage |  |
| --- | --- | --- | --- | --- |
|  | Description | Class (%) | Hectare (ha) | Proportion (%) |
| 1 | Flat or almost flat | 0--35 | 22.86 | 0.8 |
| 2 | Gently sloping | 3--8% | 203.12 | 7.4 |
| 3 | Sloping | 8--15% | 704.37 | 25.8 |
| 4 | Moderately steep | 15--30% | 1352.82 | 49.6 |
| 5 | Steep | 30--50% | 402.10 | 14.7 |
| 6 | Very steep | >50% | 42.83 | 1.6 |

Slope Classification

Figure 2-1: Drainage map of Mayis-2 weir Irrigation Project watershed

## Hydro-Metrological Data Availability

### Climate

The interaction of human activity on the distribution of vegetation through land management practices and seemingly benign rainfall events can make land more vulnerable to degradation. These vulnerabilities become more acute when the prospect of climate change is introduced.

Rainfall and temperature are the prime factors in determining the climate and the distribution of vegetation types. There is a strong correlation between rainfall and biomass since water is one of primary inputs to photosynthesis.

### Rainfall Data

The extremes of either too much or too little rainfall can determine soil erosion that can lead to land degradation. However, soil scientists consider rainfall the most important erosion factor among the many factors that cause soil erosion. Rainfall can erode soil by the force of raindrops, surface and subsurface runoff, and river flooding. The velocity of rain hitting the soil surface produces a large amount of kinetic energy, which can dislodge soil particles. The watershed comprises two rainfall classes, the mean annual precipitation ranges from 900-1200 mm, average annual of 1050 mm.

River flow data

The base flow which is measured on May is 205 l/s. Since this base flow is measured during the dry months of the year, this figure is adopted for design.

### Upstream & Downstream Utilization

The proposed command area has net irrigable land of 72.45 ha with duty of 1.22l/s/ha. Based on the duty of the area, the total amount of water required is 88.39l/s. And the base flow of the river is 205l/s.

For the sake of planning and design, however, the outlet for the diversion is designed for a discharge of 90.16l/s for this project and the project is to be developed for 72.45 ha of land on the left side, which is most of the time achievable as the flow for most of the time is significant to support this size of command area.

## Design Flood Analysis

For the design and analysis of structures to be constructed on the River estimation of flood magnitude is an important task. This can be done using different techniques depending on the data available. For this particular case, there are no river flow data and hence the flood estimation is done using the rainfall data and applying SCS Curve Method

### Rainfall Data Quality Test

Quality of the maximum daily rainfall data plays great role in the proper designing of the project structures, particularly in irrigation project head work & drainage structures. Quality of data may be suspected due to systematic and random errors. Errors may occur during measuring, reading, recording, transferring etc. of the data. There are various methods of error analysis depending on the purpose. Here, data variability, reliability, and symmetricity

#### Outlier Test

Table 2-1: Outlier Test Analysis

| S.No. | Year | Max. RF | Descending Order | Rank | Logarithmic Value/Yo/ |
| --- | --- | --- | --- | --- | --- |
| 1.0 | 1996 | 40.5 | 67.2 | 1.0 | 1.8274 |
| 2.0 | 1997 | 46.0 | 57.0 | 2.0 | 1.7559 |
| 3.0 | 1998 | 30.3 | 52.3 | 3.0 | 1.7185 |
| 4.0 | 1999 | 39.6 | 46.0 | 4.0 | 1.6628 |
| 5.0 | 2000 | 27.6 | 41.7 | 5.0 | 1.6201 |
| 6.0 | 2001 | 28.2 | 40.5 | 6.0 | 1.6075 |
| 7.0 | 2002 | 22.6 | 39.6 | 7.0 | 1.5977 |
| 8.0 | 2003 | 57.0 | 39.4 | 8.0 | 1.5955 |
| 9.0 | 2004 | 39.4 | 38.1 | 9.0 | 1.5809 |
| 10.0 | 2005 | 29.2 | 37.0 | 10.0 | 1.5682 |
| 11.0 | 2006 | 37.0 | 30.3 | 11.0 | 1.4814 |
| 12.0 | 2007 | 52.3 | 30.1 | 12.0 | 1.4786 |
| 13.0 | 2008 | 67.2 | 29.2 | 13.0 | 1.4654 |
| 14.0 | 2009 | 38.1 | 28.2 | 14.0 | 1.4502 |
| 15.0 | 2010 | 30.1 | 27.6 | 15.0 | 1.4409 |
| 16.0 | 2011 | 41.7 | 22.6 | 16.0 | 1.3541 |
| SUM |  |  | 626.80 |  | 25.2051 |
| MEAN |  |  | 39.18 |  | 1.5753 |
| STANDARD DEVIATION |  |  | 11.91 |  | 0.1268 |
| SKEWNESS COEFICIENT |  |  | 0.921 |  | 0.2756 |

| Kn |  | 2.278438 |
| --- | --- | --- |
| Higher Limit ; YH= Ymean+Kn*бy |  | 1.86424481 |
| Higher Limit ; Qh= |  | 73.1551335 |
| Lower Limit ; YL= Ymean- Kn*бy |  | 1.28638912 |
| Lower Limit ; Ql= |  | 19.3370011 |

The maximum and the minimum Rainfall data are within the range of higher and lower limit. Hence, there is no outlier rainfall data.

#### Check for Variance

After checking the outliers, the data should be checked for variability. For variability the formula used is

α=δ∩-1N*Mean*100%

Where, δn-1 = Standard deviation =11.91

N = Nr of recorded data = 16

Mean = 39.18

α = Standard error

α=14.9714*59.46*100%=7.59%<10%  Acceptable

Therefore the data shows no variability.

### Design Rainfall Computation

#### Determination of Design Period

Selection of the design return period, also called recurrence interval, depends on economic considerations such as the cost of periodic repair or replacement of the facility and the cost of providing additional capacity to reduce the frequency of repair or replacement. Vegetated control and temporary structures are usually designed for a runoff that may be expected to occur once in 10 years while expensive permanent structures may be designed for runoffs expected only once in 50 or 100 years.

For the small–scale irrigation project, it would be recommended that the project design flood once in 100 years be used for design of storage dams, the flood once in 50 years for design of diversion weirs, intake, and the flood once in 10-20 years for design of drainage structures. For Mayis-2 irrigation project, 50 years return period for head work structures, and 10 years for drainage structures has been recommended.

#### D-Index Test

Based on the data of 24 hr. peak rainfall given in Table1, The design rainfall, RF is computed using Gumble Method after estimating D-index.

After checking the consistency of the data for higher and lower outlier, the 16 years data is obtained as representative for the analysis using D-index. The D-Index test is believed to be the better goodness to fitness in many literatures. Hence in this study it was used to determine the best statistical distribution to estimate the peak rainfall. The D-index for the comparison of the fit of various distributions is summarized as follows.

D-Index=(1Xm)*i=114Abs(Xi-Xi')

Where Xi and Xi’ are the ith highest observed and computed values for the distribution respectively

Table 2-2: Test for goodness to fit using D-index

| Rank | XI | Normal | Log Pearson Type III | Log Normal | Pearson Type III | Gumbel EVI | Gumbel |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | XI -'XI' | XI -'XI' | XI -'XI' | XI -'XI' | XI -'XI' | XI -'XI' |
| 1 | 67.20 | 9.393 | 3.439 | 7.801 | 9.038 | 7.364 | 2.114 |
| 2 | 57.00 | 3.694 | 5.093 | 3.810 | 3.601 | 3.893 | 0.137 |
| 3 | 52.30 | 2.067 | 6.167 | 2.971 | 2.110 | 3.268 | 0.417 |
| 4 | 46.00 | 1.762 | 3.524 | 0.428 | 1.634 | 0.031 | 2.216 |
| 5 | 41.70 | 3.916 | 1.835 | 2.348 | 3.732 | 1.907 | 3.554 |
| 6 | 40.50 | 3.163 | 2.634 | 1.487 | 2.943 | 1.037 | 2.224 |
| Sum |  | 23.994 | 22.693 | 18.845 | 23.059 | 22.693 | 10.661 |
| Sum/Mean |  | 0.612 | 0.579 | 0.481 | 0.589 | 0.579 | 0.272 |
| Point Rainfall |  | 63.63 | 71.48 | 68.52 | 65.36 | 70.04 | 77.55 |
| Design Point Rainfal = |  | 77.551 |  |  |  |  |  |

All the candidate distributions give almost identical correlation coefficients. From the table, the standard errors are significantly lower for the Gumbel Method which is 0.272 and has been taken to fix the design point rain fall.

Gumbel Method is the most widely used method for the prediction of maximum annual rain falls and flood flows. It is applicable only to annual extreme values. For this particular case, this method is used to determine maximum rainfall for 50 years return periods.

The design rainfall using Gumbel Method is given as

XT=Xmean+KT*σn-1

Where

- XT is Design Point Rainfall

- Xmean is average of all values of annual heaviest fall = 38.18 mm

- σn-1 is  standard deviation of the series = 11.91 mm

- KT is  KT=YT-YnSn

- YT is  YT=ln⁡(lnTT-1)

- Yn is reduced mean, depends on N value which is 0.5100 for N = 16

- Sn is reduced standard deviation, depends on N values which is  1.01 for N =16

- T is return Period, which is 50

- YT=lnln5050-1=3.902

- KT=3.902-0.52001.0095=3.2

- XT=77.55

The design rainfall at points for 50 years return period is 77.55 mm and the areal design rainfall is calculated in the following section.

### Peak Discharge Determination

#### General

The River is not gauged river. The design flood is calculated by using SCS unit hydrograph method. Thus, it is preferred to base the flood analysis on rainfall data, which are better both in quantity and quality of data. In the hydrologic analysis for drainage structures, it must be recognized that there are many variable factors that affect floods. Some of the factors that need be recognized and considered on an individual site by site basis are; rainfall amount and storm distribution; catchment area, shape and orientation; ground cover; type of soil; slopes of terrain and stream(S); antecedent moisture condition; Storage potential (over bank, ponds, wetlands, reservoirs, channel, etc.)

#### Peak Flood Analysis by SCS Unit Hydrograph Method

Design flood is calculated SCS (The United States Soil Conservation Service) method. This method is widely adopted and more reliable method for flood estimation. The approach considers, watershed parameters, like Area, Curve number, and time of concentration.

#### Time of Concentration (Tc)

Time of concentration has been calculated by taking the stream profile of the longest streamline and dividing it in to different elevation. Kirpich formula is adopted for computation.

The formula is,

Tc=0.948L13H10.385+L23H20.385+…+Ln3Hn0.385

- Tc = 1.97	Since Tc < 3hr., duration of excess rainfall difference, D = 0.5 hr.

- Time to peak,

Tp   =D2+0.6*Tc  = 1.432 hr

- Base time,

Tb=2.67*Tp = 3.82

- Recession time,

Tr=1.67*Tp  = 1.18 hr.

#### Curve Number (CN)

Curve number (CN) is achieved based on USSCS method by watershed characterization in terms of land cover, treatment, hydrologic condition and soil group. From the watershed analysis curve number at condition II =74.42, since peak rainfall is found at an antecedent moisture condition III state, this value has to be changed to antecedent moisture condition III.

- Conversion factor = 1.181

- CN Condition (III) = (Factor from Table x CN condition II) =74.42.0*1.18 = 87.88

#### Area Rainfall

As the area of the catchment gets larger, coincidence of all hydrological incidences becomes less and less. This can be optimized by changing the calculated point rainfall to aerial rainfall. The conversion factor is taken from standard table that relate directly with the size of watershed area and type of the gauging station. (IDD manual)

For the case of Mayis-2 irrigation project,

- Total watershed area = 27.29 km2

- Type of gauging station = Daily rainfall (24 hr.)

- Aerial Rainfall = (Point Rainfall) x (Conversion factor)

#### Runoff Analysis

Input data:

- Design Point Rainfall = 77.55 mm

- Curve number at antecedent moisture condition III = 87.88

- Catchment Area, A = 27.29 km2

- Tc = 1.97 hr, D = 0.5 hr, Tp = 1.432hr, Tb = 3.82 hr, Tr = 1.18 hr.

Direct run-off,   Q=(I-0.2*S)2(I+0.8*S)

Where,

- I = Rearranged cumulative run-off depth (mm

- S = Maximum run off potential difference,

S=25400CN-254

Peak run-off for incremental;

Qp=0.21*(A*Q)Tp

Where,

- A = Catchment area = 27.29 km2

- Tp = Time to peak (hr)

- Q = Incremental run-off (mm)

Table 2-3: Runoff Analysis

| Duration | Cumulative RF | Accumulative Runoff | Incremental Runoff | Runoff in | Unit Hydrograph Time |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| hr | Mm | mm | mm | m3/s | Beginning | Peak | Time To End |
| 0.50 | 2.09 | 0.00 | 0.00 | 0.00 | 0.00 | 1.43 | 3.82 |
| 1.00 | 4.85 | 0.00 | 0.00 | 0.00 | 0.50 | 1.93 | 4.32 |
| 1.50 | 13.62 | 1.78 | 1.78 | 7.12 | 1.00 | 2.43 | 4.82 |
| 2.00 | 28.56 | 10.32 | 8.54 | 34.18 | 1.50 | 2.93 | 5.32 |
| 2.50 | 39.62 | 18.63 | 8.31 | 33.26 | 2.00 | 3.43 | 5.82 |
| 3.00 | 50.25 | 27.40 | 8.77 | 35.10 | 2.50 | 3.93 | 6.32 |

Table 2-4Hydrograph Coordinates

| HYDROGRAPH TIME | H1 | H2 | H3 | H4 | H5 | H6 | HT |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 0.00 | 0.00 |  |  |  |  |  | 0.00 |
| 0.50 | 0.00 | 0.00 |  |  |  |  | 0.00 |
| 1.00 | 0.00 | 0.00 | 0.00 |  |  |  | 0.00 |
| 1.43 | 0.00 | 0.00 | 2.15 | 0.00 |  | 0.00 | 2.15 |
| 1.50 | 0.00 | 0.00 | 2.49 | 0.00 | 0.00 | 0.00 | 2.49 |
| 1.93 | 0.00 | 0.00 | 4.63 | 0.00 | 0.00 | 5.75 | 10.38 |
| 2.00 | 0.00 | 0.00 | 4.97 | 11.94 | 0.00 | 6.74 | 23.65 |
| 2.43 | 0.00 | 0.00 | 7.12 | 22.25 | 10.03 | 13.08 | 52.48 |
| 2.50 | 0.00 | 0.00 | 6.92 | 23.87 | 11.6 | 14.08 | 56.48 |
| 2.93 | 0.00 | 0.00 | 5.63 | 34.18 | 21.65 | 20.42 | 81.89 |
| 3.43 | 0.00 | 0.00 | 4.14 | 27.04 | 33.26 | 27.76 | 92.32 |
| 3.82 | 0.00 | 0.00 | 2.98 | 21.44 | 27.82 | 35.10 | 87.34 |
| 3.93 | 0.00 | 0.00 | 2.65 | 19.89 | 26.31 | 35.10 | 83.95 |
| 4.32 | 0.00 | 0.00 | 1.49 | 14.29 | 20.86 | 29.36 | 66.00 |
| 4.82 |  | 0.00 | 0.00 | 7.15 | 13.91 | 22.02 | 43.07 |
| 5.32 |  |  | 0.00 | 0.00 | 6.95 | 14.68 | 21.63 |
| 5.82 |  |  |  | 0.00 | 0.00 | 7.34 | 7.34 |
| 6.32 |  |  |  |  | 0.00 | 0.00 | 0.00 |
|  |  |  |  |  |  | 0.00 | 0.00 |
|  |  |  |  |  |  |  | 0.00 |

Figure 2-2 Complex Hydrograph

### Tail Water Depth Computation

Tail water depth of the river is equal to the flood depth and amount at the proposed weir site before construction of the weir. It is used to crosscheck peak flood estimated by the SCS unit hydrograph method with flood mark method and to see the flood feature after the hydraulic jump. During field visit, the flood mark of the river at the proposed diversion site was marked based on dwellers information and physical indicative marks. The river cross-section was surveyed.

Table 2-5Weir Site River Cross section Coordinate Data

|  | X-Coordinate | Y-Coordinate | Partial Distance | Cumulative Distance | Elevation |
| --- | --- | --- | --- | --- | --- |
| 1.0 | 470210.76 | 1212325.475 | 0.00 | 0.00 | 2605.379 |
| 2.0 | 470215.265 | 1212330.718 | 6.91 | 6.91 | 2603.227 |
| 3.0 | 470217.728 | 1212333.577 | 3.77 | 10.69 | 2600.603 |
| 4.0 | 470218.475 | 1212334.444 | 1.14 | 11.83 | 2598.71 |
| 5.0 | 470220.482 | 1212336.774 | 3.08 | 14.91 | 2596.374 |
| 6.0 | 470222.407 | 1212339.007 | 2.95 | 17.85 | 2595.025 |
| 7.0 | 470224.609 | 1212341.563 | 3.37 | 21.23 | 2594.773 |
| 8.0 | 470226.409 | 1212343.653 | 2.76 | 23.99 | 2594.686 |
| 9.0 | 470228.677 | 1212346.285 | 3.47 | 27.46 | 2595.400 |
| 10.0 | 470230.666 | 1212348.595 | 3.05 | 30.51 | 2597.112 |
| 11.0 | 470234.406 | 1212352.933 | 5.73 | 36.24 | 2599.054 |
| 12.0 | 470235.681 | 1212354.416 | 1.96 | 38.19 | 2599.715 |
| 13.0 | 470237.255 | 1212356.248 | 2.42 | 40.61 | 2600.987 |
| 14.0 | 470239.359 | 1212358.696 | 3.23 | 43.84 | 2601.955 |
| 15.0 | 470242.277 | 1212362.092 | 4.48 | 48.31 | 2602.825 |
| 16.0 | 470244.045 | 1212364.149 | 2.71 | 51.03 | 2603.773 |

Table 2-6Stage discharge analysis

| No | ELEVATION | DEPTH | WET AREA | WET PERIM | Hydraulic radius R (m) | Velocity V (m/sec.) | Discharge Q (m^3/sec.) | Remark |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2594.69 | 0.0 | 0.00 | 0.00 | 0.0 | 0.0 | 0.0 |  |
| 2 | 2595.19 | 0.5 | 2.94 | 9.08 | 0.32 | 1.17 | 3.45 |  |
| 3 | 2595.69 | 1.0 | 8.17 | 11.90 | 0.69 | 1.94 | 15.86 |  |
| 4 | 2596.19 | 1.5 | 14.46 | 14.12 | 1.02 | 2.53 | 36.66 |  |
| 5 | 2596.69 | 2.0 | 21.69 | 16.10 | 1.35 | 3.04 | 65.98 |  |
| 6 | 2597.04 | 2.35 | 27.31 | 17.46 | 1.56 | 3.36 | 92.21 | Peak Discharge |
| 7 | 2597.19 | 2.5 | 29.73 | 18.04 | 1.65 | 3.48 | 103.49 |  |
| 8 | 2597.69 | 3.0 | 38.73 | 20.43 | 1.90 | 3.82 | 148.03 |  |
| 9 | 2598.19 | 3.5 | 48.79 | 22.81 | 2.14 | 4.14 | 202.11 | Flood mark |
| 10 | 2598.69 | 4.0 | 59.89 | 25.19 | 2.38 | 4.44 | 266.18 |  |

Figure 2-3 Rating Curve

From the above stage discharge table and curve the maximum flood level corresponding to the computed design peak discharge at flood mark is 2598.189(3.5 m from the river bed) and it is considered as the d/s high flood level i.e. expected at the weir axis before construction of the weir.

- Average River Bed Slope

Average river bed slope of River is estimated by two different techniques. One is by end area method and the other is by using best fit line method. Designers have adopted the best fit line method output for further analysis.

The water level of the river is taken at different points along the river channel around the head work site. Surveying work done for 51.03m length. And then, average water surface slope is considered as the river bed slope. For comparison of the two procedures, refer the attached Excel file.

Figure 2-4 River profile

- Manning’s Roughness Coefficient

The Manning’s roughness coefficient is taken from standard table based on the river nature. The river at the headwork site has got braded feature and curving nature. The river banks are defined and relatively smooth. Manning’s roughness coefficient (n = 0.045 is adopted).

- Discharge Of The River

Input data:

- Manning's roughness coefficient, n = 0.045

- Average river bed slope, S = 0.013

- V=1n*R23*S^0.5,  Where, R = Hydraulic radius = (Area/Perimeter)

- Q=V*A

- DSHFL=Min. Bed Level+Tail wated depth = 2594.686+ 3.50 = 2598.19 m.

# SECTION-II: HEAD WORKDESIGN

# HEADWORK STRUCTURES DESIGN

## Headwork Site Selection

The headwork site is situated at1212325.475 N, 470210.76 mE and river bed elevation of 2605.08m above sea level. At this site the river course is well defined, matured with fixed width and forms nearly a U-shaped valley. The river is flowing through a defined channel and its scouring effect on the river course is not significant as its bed rock.  The different sections of the stream at the proposed headwork site are described separately below:-

## River Geomorphology

It is a common fact that the river development tends to accommodate itself to the local geology that develops along the structurally weak zones like faults, joints, folds, etc. The drainage system of the study area is strongly influenced by geological structures and formations, the nature of the vegetation cover and climate. The nature of geological formations and structures has also strong influence on the development of the channel. The river bed formation and bank conditions are essential to decide the structure types on a diversion site. Accordingly, Mayis River does have the following river geomorphology.

The headwork site geological surface and subsurface conditions have been investigated based on the nature of the proposed structure. At the site and immediate vicinity, the stream flows along moderate slope course and its bed is mainly covered with rock.  And also both banks of the stream at the headwork site are made up of bed rock.

River Bed Condition

For this project a simple river bed level weir structure has been proposed for diverting the water to the main canal. The headwork site geological surface and subsurface conditions have been investigated based on the nature of the proposed structure. At the site and immediate vicinity, the stream flows along steep slope course. The right bed and bank at weir site are made up agglomerates (if the top 0.5 m units are removed fresh rock will be expected) and also the active stream beds are covered with bedrocks. On the other hand, the both of left and right bank is covered with the agglomerate basalts.  The detail geologic nature of the banks, and bed of the stream along the headwork axis and immediate vicinity are described and their potential geotechnical influence on the proposed structures also discerned/detected below, with remedial measures.

Figure 3-1River bed at the proposed weir site

### River Bank Condition

#### Right Bank

At the headwork site/axis, the right bank is characterized by relatively steep slope, having about more than 5 m height from stream bed. It reveals nearly vertical section within this height. From visual observation of the natural exposure, there are one geologic unit (agglomerate) forming the bank section. (See plate 4.2). And it is portion of the older bedrock at the project site. The rock is agglomerate, which is affected by slightly degree of weathering and joint (joint is not persistent). If the top 0.5 m of the banks is fresh parts of rocks will be expected. It is resistant to flood erosion that there is no need to proposed bank protection works.

#### Left Bank

At the proposed headwork axis and immediate vicinities, the left bank forms steep slope topography, having about more than 5 m height from stream bed. From surface observation, the bank is covered entirely with one geological unit such as agglomerate basalts. It has coarse grain texture, dark gray in color, slightly weathered and jointed. But the joints filled by clay soil. So that, if the top 0.5 m of agglomerate basalt is removed fresh rocks will be expected. The bank is not now affected by flood erosion. So, it is not necessary to provide some bank protection works.

## Sources of Construction Materials

During the site investigation, natural construction materials required for the construction of the various proposed engineering structures at the headwork site and within the farmland have been assessed, and possible quarry sites and borrow areas have been identified within the vicinity of the area as much as possible. In addition to the identification of the quality, quantity, accessibility condition and ownership of each proposed production sites have also been studied and described in this report; on separate sub-sections below. The natural materials required for the construction of the proposed hydraulic structures include rock for masonry stones, aggregates (both coarse and fine), impervious soil for fill and/or lining, backfill soil, and water.

### Rock For Masonry And Crushed Coarse Aggregate

. Quarry site that can be used for production of rock for masonry stone and crushed coarse aggregates has been assessed during the field work session within the vicinity of the project area at economic distance for hauling.

One possible quarry site has been identified along the left sides of Mayis Rivers following the main canal route at co-ordinates of about 468877mE and 1212745mEwith elevation 2718 m. Here slightly weathered basaltic rock exposed and forms a continuous ridge parallel to the main canal. It is believed that below this weathered rock, fresh portion of the rock is found and can be used for the intended purpose. This quarry site has good potential and an economic distance from the head work site. It has about 4km from the head site downstream direction and found nearby the small village Gimbaro (with continues ridge segments).

In this field study, another sources for rock also proposed. The second possible quarry site has been assessed in Mayis stream itself in upstream direction. It has good potential of rocks for an intended purpose.

### Fine Aggregates

Borrow areas for fine aggregate or natural sand have been assessed starting from the project stream itself. Natural deposits of such materials couldn’t be found when assessed within the beds of the stream in the project area; rather very coarser sediments and rock exposures are found covering almost the entire bed of the workie stream. Seeing to this nature of the stream, other distant streams have been explored to identify the best source areas for fine aggregate or natural sand that can be used for this particular project. During exploration of this natural sand, at a distant one stream was identified as a possible source of fine sand. The stream is known as ‘jara’. It is located at about 40 km from the project site, within chefa Robit woreda. A potential source area within the jara stream bed is located alongside the main kemise Robit-Ataye Asphalt road, near to a small village of chefa Robit. Here, there are local legalized sand miners associations, and the sand is acquired from them through negotiation and agreement.

The samples have been analyzed at Amhara Design and Supervision material testing laboratory to characterize the gradation of the sand deposit. According to these laboratory test results, the sand deposits from various portion of the jara stream have grading.  As the sand will be used as fine aggregate in the construction of concrete and as mortar ingredient for masonry works, its quality has been evaluated based on ASTM C33 specification, especially its grading.

### Water

Water for construction purposes can be getting from the project stream, Mayis itself. The stream is perennial that throughout the year there is some amount of water flow along its course. During this field study time the stream flow was about 320L/second. The stream water originates from mainly fractured basaltic and Rhyolitic rock aquifer, which is portion of the Ethiopian western highland that is known to discharge good quality potable water

## Headwork Type Selection

The type of the weir is selected based on

- River nature,

- Availability of construction materials and

- Experience of the workmanship.

In the case of mayis irrigation project, the river is on the gentle slope and doesn’t carry river born materials like boulders; ogee weir typeis selected

### Hydraulic Design of Headwork Structure

#### Weir Height Determination

The following major factors have been seen in determining the weir crest level:

- Maximum  command area elevation

- Deriving head of the intake structure

- Main canal slope

- Loss

- Minimum Bed Level

Table 3-1Weir Height Determination

| Parameters | Units |
| --- | --- |
| Length of the canal (from outlet to first irrigation) | 335 |
| Slope of the canal | 0.001 |
| Maximum Command Area | 2596.5 |
| Head Regulator Loss | 0.1 |
| Head due toTurn out & bending | 0.1 |
| Head Due to Canal Slope | 0.34 |
| Canal total depth | 0.50 |
| Canal Outlet Sill Level | 2597.036 |
| Freeboard | 0.1 |
| Weir Crest Level | 2597.54 |
| Min. River Bed Level | 2594.686 |
| Weir Height | 2.85 |

#### Weir Dimensions

- Crest Length

Actual river section width of the over flow section of the river is = 18m, used for design analysis effective length of the crest by deducting under sluice and divide wall is 15m.

- Discharge over the weir section

- Design discharge, Q=202.11 m3/s

Ogee weir type has been selected for Mayis one project. The flood passing this weir type is given by:

Q=CLHe3/2    where, Q= discharge in m3/s

C=coefficient of discharge

He =total head over the crest including velocity head in m.

L= effective length of crest in m

In this case, Q= 202.11m3/s, L=15m, C=2.2, but, it depends upon various factor, such approach depth, upstream slope of the weir and submergence effect. Since, the values of Q, L& C are known the value of He can be approximated, of which the correct value is calculated after the correction of coefficient of discharge.

The approximate,     He=QCL23=202.112.2*1523=3.35m

Approach velocity, Va=QHd+p    where, flow depth over weir, Hd=He-Va22*g  and     p=effective weir height that does not fill up with silt, P=2.8 in this case.

Then,Va22g=He-Hd=3.35-Hd, but  Va = QL(Hd+P

Therefore, 202.112(15*(Hd+2.8))2*12*9.81=3.35-Hd

The approximate value of Hd   =3.08m by the iterating the above equation. But, variation of discharge coefficient has to be checked against the factors that affect it.

- Approach depth of water:-for the effective depth of approach of water, P is greater than 1.33 times the over flow depth, Hd, the velocity of approach has been found to have a negligible effect upon discharge and as such Hd becomes equal to He. In such case, the coefficient of discharge=2.2.however, for P to Hd ratio less than 1.33, the approach velocity has appreciable effect.

For Mayis one project ogee weir, P=2.80 and Hd =3.08, then P to Hd ratio is equal to 0.91, which is less than 1.33. So, approach velocity has significant effect. From developed curve of He/Hd versus C/Cd, the value of C/Cd=0.985 for a given P/Hd. Hence, corrected coefficient of discharge=Cd*0.985=2.167.

- Upstream slope of the weir: - the upstream slope of the weir is considered to be vertical in case of Mayis one head work weir structure. So., it doesn’t have any impact on the coefficient of discharge. Then, C= Cd.

- Submergence effect: - when the tail water level is at above of the weir level the weir can’t discharge freely. This effect is commonly expressed in the ratio of (He+weir height) to He. If this ratio is greater than 1.7. There is no effect of submergence. Weir height of the project=2.85 & He=3.35, then, He+weir heightHe=3.35+2.853.35=1.85>1.7. So there is no effect of submergence. Hence, corrected coefficient of discharge= C =2.167

Now, corrected value of He and Ha can be calculated as follows.

He=QCL23=202.112.167*1523=3.38m

Then,Va22g=He-Hd=3.38-Hd, but  Va = QL(Hd+P

Therefore, 202.112(15*(Hd+2.8))2*12*9.81=3.38-Hd

The corrected value of discharge depth passing over the weir, Hd =3.12m by the iterating equation and approach velocity head,

The velocity head, ha is computed from the approach velocity as shown below

Ha=He –Hd = 3.38 - 3.12 = 0.26m

- Downstream profile of the ogee weir

The downstream profile of the Ogee weir is represented by equation:

Xn=K.Hdn-1.Y Where,( X,Y) are the coordinates of points on the crest profile with origin at the highest point C of the crest called apex.

Hd is design head including velocity head in m.

K and n constant values depending up on the upstream slope

For this design case, the slope of the upstream face is vertical. For vertical upstream face value of K=2.0 & value of n =1.85 from standard table and calculated Hd= 3.38m. Then, substituting these values into the above equation, it becomes:-

X1.85=2*3.381.85-1.Y⇒X1.85=2*3.380.85*Y

⇒Y=X1.85/(2*3.380.85)

Consider the value of X at 0.5m interval of increment, then values of Y is given below in the table.

Table 3-2: D/S face profile Coordinates of (X, Y)

| X | 0.0 | 0.5 | 1.0 | 1.50 | 2.0 | 2.5 | 3.00 | 3.53 | 3.90 | 4.10 | 4.30 | 4.48 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Y | 0.00 | 0.05 | 0.18 | 0.38 | 0.64 | 0.97 | 1.35 | 1.83 | 2.20 | 2.41 | 2.64 | 2.85 |

Smooth gradual reverse curvature is provided at the bottom of the downstream face to turn the flow into the apron of a stilling basin. A radius of about one fourth of the weir height is considered to be satisfactory for this curvature.

Radius of the curve,r=  weir height4=2.854=0.71m

- Upstream profile of the Ogee weir

As per studies  of U.S Army corporation the upstream curve of the Ogee weir having a vertical upstream face,  is given by equation:-

Y=0.724*X+0.27Hd1.85Hd0.85+0.126Hd-0.4315Hd0.375*X+0.27Hd0.625—(a)

And the upstream profile extends up to X=-0.27Hd.

The maximum value of X=-0.27*3.38=-0.91m. Then, the value of X and Y is given below with the above equation (a).

Table 3-3: U/S face profile coordinates(X,Y)

| X | Y |
| --- | --- |
| 0.00 | 0.00 |
| -0.10 | 0.00 |
| -0.20 | 0.01 |
| -0.30 | 0.03 |
| -0.40 | 0.05 |
| -0.500 | 0.08 |
| -0.600 | 0.13 |
| -0.700 | 0.18 |
| -0.800 | 0.26 |
| -0.91 | 0.426 |

- Top and bottom width

##### The top and bottom width of the ogee weir is determined using ogee spillway profile formula. As discussed above section downstream profile calculation at 2.85m weir height x=3.48m and upstream profile equation x=0.27Hd =0.91. So the bottom width of the weir calculated as the sum of upstream and downstream width.

##### B = 0.91+4.48 = 5.49m ~ 5.5m

### U/S and D/S HFL Calculation & Determination

From the stage –discharge curve prepared (Section 1, in Hydrology Part) the high flood level before construction (i.e. D/s HFL) corresponding to the design flood is 2598.19m a.s.l.

- D/s HFL = 2598.19m  masl ------------------------------------- (a)

- U/s HFL = U/s bed level + weir height + Hd ----------------- (b)

Hd is the depth of water over the weir/intake crest. This is calculated by assuming broad crested weir formula.

- Q=C*L*He32

- He=QC*L23=202.111.7*1523=3.38m

The velocity head, ha is computed from the approach velocity as shown below

- ha=Va22g

Where

- g: acceleration due to gravity = 9.81m/sec2

- Va is Approach velocity determined by,   Va=QL*ha

- L is Weir crest length = 15m

- hd is flow depth over the weir and also,

Hence,   hd will calculate as follow by trial and error;

hd=He-ha=He -Hd*  Hd+P2-q22g=0

Finally, by trial and error method, hd is found to be 3.12 m

- ha = He - hd  =  3.38 – 3.12 m  = 0.26 m

### Hydraulic Jump Calculation

The length of wing walls is determined based on the length of Jump, and it is calculated as shown below.

- Weir crest length = 15m

- Weir height =2.85 m

- Pre-jump depth = y1

- Post -jump depth =y2

Neglecting losses between point A and B and considering similar datum

z + He = y1 + ha

But, He = 3.38 m

- q=Ql=202.11m3/s15m=13.47m2/s

- ha=q22*g*y2=13.4722*g*y12

- Fr=V1gy1=9.759.81*1.38=2.65

- y2=y121+8*Fr2-1y2=`1.3821+8*2.652-1 =4.53m

### Impervious floor

#### D/s Impervious Floor (Ld)

For under seepage the worst condition would be when the water on the upstream side is at the level of the weir crest & there is no tail water. Seepage head loss at

- Pond level case:

Hs = crest level –bed level

= 2.85 m

- Maximum flood case:

Hs = U/s HFL- D/s HFL

= 2.47m

Therefore maximum seepage head occurs when water is stored up to the pond level and there is no water on the d/s.

Bligh’s constant, Cb is depending on the type of the foundation.

Ld=2.2*Cb*HW10Ld=2.2*12*2.8510 =9.56 take 2.0m

Therefore D/s impervious floor is taken to be 2 m long.

#### U/S Impervious Floor Length, (Lu)

The u/s impervious floor, (Lu) is provided 1m with a nominal thickness of 0.5 m

### Cut off Depth Calculation

#### U/s cut off

- Q=202.11m3/sec

- q=13.47 m3/s/m

- Silt factor, f=1.76*(d50)0.5 , f =14.99

- D50= 72.5mm

- R=1.35*(q2/f)(1/3), R=1.35*(q2/f)(1/3)=2.77

- Scouring Depth = 1.5 * R = 1.5 * 2.77= 4.155

- Hence bottom level of u/s cut off=U/S HFL–4.155=2595.77m

- Therefore, U/S Cut off depth = min bed level – 2596.5 m = 1.81 m take 1.0m including the floor thickness.

#### D/s cut off

- Q = 202.11m3/sec

- q = 13.47 m3/s/m

- Silt factor, f =1.76*(d50)0.50 ,   f =14.99

- D50= 72.5mm

- R =1.35*(q2/f)(1/3) ,   R=1.35*(q2/f)(1/3) = 2.77

- Scouring Depth = 1.5 * R = 1.5 * 2.77= 4.155

- Hence bottom level of d/s cut off=D/S HFL–2R =2592.65 m

- Therefore, D/S Cut off depth = min bed level – 2592.65 = 2.04 m take 1 m including the floor thickness for hard rock foundation.

### Stability Analysis of weir

Stability analysis is carried out to see the already determined weir section is safe against overturning, sliding, tension. The stability analysis is carried out considering the effect of the following forces.

• Water pressure

• Weight of the over flow weir section

The extreme load combination is the case where the head is at crest level of the weir (dynamic Case) and there is no flow over the weir (static case)

#### Stability Analysis of Weir at Static Condition

Figure 3-2 Stability analysis of weir at static condition

- Load Calculation

Table 3-4 Load calculation of weir at static condition

| No | Force |  |  | Lever Arm | Moment |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Formula | Vertical | Horizontal |  | Positive | Negative |
| 1 | W1 | 62.24 |  | 5.05 | 314.02 |  |
| 2 | W2 | 8.26 |  | 4.06 | 33.49 |  |
| 3 | W3 | 92.93 |  | 3.79 | 352.20 |  |
| 4 | W4 | 17.76 |  | 2.50 | 44.34 |  |
| 5 | W5 | 50.44 |  | 2.25 | 113.49 |  |
| 6 | W6 | 25.05 |  | 0.98 | 24.55 |  |
| 7 | Ph |  | 39.81 | 0.95 |  | -37.81 |
| 8 | Ps |  | 68.99 | 0.95 |  | -65.52 |
| Sum | SUM | 256.68 | 108.81 |  | 882.08 | -103.33 |

- Stability Analysis At The Toe

| 1. Overturning |  |  |  |
| --- | --- | --- | --- |
| Sum of stabilizing moment (+M) | 882.08 |  | KNm |
| Sum of destabilizing moment (-M) | 103.33 |  | KNm |
| Factor of Safety (+M/-M)>1.5 | 8.54 |  | Safe |
| 2. Sliding |  |  |  |
| Sum of vertical force | 256.68 | KN |  |
| Horizontal resisting force | 108.81 | KN |  |
| Factor of Safety (R/Fh) >1.5 | 1.65 | Safe |  |
| 3. Tension |  |  |  |
| Net moment | 778.76 |  | KNm |
| Net vertical force | 256.68 |  | KNm |
| X | 3.03 |  | m |
| e | 0.30 |  | m |
| B/6 , e <B/6 | 0.91 |  | Safe |

### Design of Divide Wall, Under Sluice, And Canal Outlet

Divide wall is designed in order to create separation between outlet canal and natural river course. The divide wall allows safe and stable base flow to the canal outlet. Flow turbidity created by current flow impact over the weir/intake body is reduced.

#### Wall Height Fixation Of Divide Wall

The existing topographical condition at the weir axis and HFL are considered to be most governing parameters for fixing the wall height.

- After construction of the weir (U/S HFL) is 2600.92 m.a.s.l

- Minimum bed Level is 2594.686 m a.s.l.

- U/S Divide wall Height = U/S HFL – Min. bed level + Freeboard

= 2600.92 – 2594.686 + 0.5

= 6.5 m

#### Divide wall Stability Analysis

Figure 3-3 Stability analysis of Divide wall

- Load Calculation

Table 3-5 Computation of forces per unit width and momentum of divide wall

| Height, H | Top width, B | Bottom Width, | under sluice side silt and water depth |  | Weir side silt and water depth |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 6.50 | 1.00 | 1.00 | 1.5 |  | 2.8 |  |  |  |  |
| Code | Load |  | Lever Arm, R |  | Moment |  |  |  |  |
|  | Vertical | Horizontal |  |  | Positive |  |  | Negative |  |
| W1 | 156.0 |  | 0.5 |  | 78.0 |  |  |  |  |
| W2 | 0.0 |  | 1.0 |  | 0.0 |  |  |  |  |
| Ps |  | -14.1 | 0.9 |  |  |  |  | 13.2 |  |
| Pw |  | -38.5 | 0.9 |  |  |  |  | 35.9 |  |
| Pw2 |  | 11.0 | 0.5 |  | 5.52 |  |  |  |  |
| Ps2 |  | 4.1 | 0.5 |  | 2.0 |  |  |  |  |
| Ww | 0 |  | 1.0 |  | 0.0 |  |  |  |  |
| Ws | 0 |  | 1.0 |  | 0.0 |  |  |  |  |
| Pu |  |  | 0.6 |  |  |  |  | 0.00 |  |
| SUM | 156.0 | -37.5 |  |  | 85.5 |  |  | 49.1 |  |
| Over turning = (M+ve/M-ve) >1.5 |  |  |  |  | Fo | 1.74 | >1.5 |  | OK |
| Sliding = (µxFv/Fh),---µ=0.75 >1.5 |  |  |  |  | Fs | 3.12 | >1.5 |  | OK |

#### Under sluice

The under sluice is mainly provided here to remove silt deposition at a result of barrier structure. Hence the sill level of the under sluice is fixed to facilitate this deposited silt to increase the efficiency of water abstracting to the main canal through the head regulator from the pocket. The inverted sill level of this sluice is fixed to be above minimum bed level which is 2596.186m a.s.l. Hence the top level of the under sluice is minimum bed level plus sluice height. Even if the position of the under sluice is on concave side that is on scouring side, there might be boulders that may come into the pocket of the under sluice due to the barrier structure. Hence in addition to the supply of water to the intake and the removal of silt, this acts to remove the boulder that comes to wards it. Considering this, the opening size of the gate is 1 m*0.9 m with spindle type operating from the operation slab. Considering rectangular notch profile of flow of water at the under sluice, the discharge passing is computed using the following formula.

- The capacity should be at least five times the canal discharge to ensure proper scouring.

- Capacity of passing about 10% to 20% of the maximum flood discharge at high floods.

- During construction, it should be able to pass the prevailing (at least base flow) discharge of the river.

But the practical application of those values is not sound so the discharging capacity of under sluice is determined by using rectangular notch formula for maximum flood condition.

Q=23*Cd*2g0.5*B*h^(32)

Considering the boulder and the silt of the river the under sluice is provided 1.0 m x 0.90 m.

#### Head Regulator

The head regulator is provided on the left side .The sill level of this head regulator is fixed from different angle observations. Hence this level is fixed based on the optimum route alignment and the maximum irrigated command level including minor and major losses criteria. Based on this condition, the sill level is fixe 2596.94m.

The canal capacity should be determined for maximum command area and the corresponding discharge. In this case the outlet capacity is fixed considering maximum duty and command area and 1.15 correction factors are considered to account the variation of duty.

- Outlet capacity = Duty x command area x correction factor

Where,

- maximum duty for 24 hr irrigation = 1.22 l/s/ha

- Command area = 72.45 ha.

- Outlet capacity = 1.22 l/s/ha x 72.45 ha x 1.15 = 101.6l/s

- Outlet size, from the rectangular notch discharge formula the outlet size is determined as follows

h=Q*3Cd2g0.5*B23=0.116*30.62*2*9.810.5*0.523=0.4

H=h+Fb=0.4+0.1=0.5 m

- Adopt the height, H  = 0.5m and width, B = 0.5m

Hence, provide an outlet size of 0.5m x 0.5 m (length x height) .The gate of the off take canal is to be vertical sheet metal of 0.55 m x 0.6m for the closure of the opening space. Provide some extra dimensions for groove insertion. The grooves are to be provided on the walls using angle iron frames at the two sides of the gate openings.

Trash racks of diameter 20 mm with c/c spacing of 10cm has to be provided u/s of the gate to prevent entry of debris to the canal.

#### Breast Wall and Operation Slab

A vertical raised gate is designed for the head regulator and under sluice. These gates slide over the breast wall using spindle during opening and closing.

For easy operation of these gates, operation slab is provided. The size of the operation slab is fixed from the point of construction and free movement. The size of the operation slab is shown in the drawing for both head regulator and under sluice with thickness 20cm.

The thickness of the breast wall is also the same as that of the operation slab. The nominal thickness is fixed from the point of construction rather than the imposed load. The thickness required for the imposed load is less than the nominal value and treated as cantilever retaining wall. For the breast wall, the minimum reinforcement area is taken as 15% along the respective direction.

- Hence	Asteel = 0.0015*1650*200=495

- Asteel = 495mm2/m

- Provide 12@C/c 200 mm

- Considering cover thickness of 50 mm, effective depth = 50 + 12/6 = 56

- Hence spacing of reinforcement = 200mm < 432mm

- Asteel = 565.2

Therefore the actual provided steel area per meter width is 565 mm2/m >495 mm2/m, which is safe. This is the same procedure for operation slab except bar diameter. Hence minimum reinforcement area is 0.15% of the concrete area per meter length, Asteel = 495 mm²/m.

Provide 12 @ c/c 200 mm.

Therefore the actual provided steel area per meter width is 587 mm2/m>495 mm2/m Ok! The actual arrangement of angle irons spindles, shafts and operation slab including other components is shown on the design drawing. For each arrangement the actual arrangement of angle irons, spindles, shafts and operation slab including other components is shown on the design drawing. For each arrangement and further information, refer to the design drawing.

#### Retaining Walls

At the two ends of the diversion weir, walls have been provided to safeguard the structure from scour at the ends and also as a facility to the canal outlet operation and maintenance at the canal outlet portion.

  - Upstream Retaining wall

The upstream masonry retaining wall is designed for 0.40 m and 3.1m top and bottom width respectively and the stability analysis is checked as shown below.

Figure 3 - 1: Stability analysis of retaining wall

Table 3-6Stability analysis of U/S retaining wall

| Item Description |  | Forces(KN.m) |  |  |  | Lever arm | Moment about toe(KN.m) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Vertical |  | Horizontal |  | (m.) | Resisting | Overturning |
|  |  | +ve | -ve | +ve | -ve |  |  |  |
| 1. Vertical force |  |  |  |  |  |  |  |  |
| 1.1. Self-weight(W1) |  | 47.84 |  |  |  | 0.2 | 9.57 |  |
| 1.2. Self-Weight(W2) |  | 161.46 |  |  |  | 1.30 | 209.90 |  |
| 1.4. Silt pressure(Pv) |  | 119.34 |  |  |  | 2.20 | 262.55 |  |
| 2. Horizontal force |  |  |  |  |  |  |  |  |
| 2.2. Active pressure (ps) |  |  |  |  | -76.613 | 1.73 |  | -132.80 |
| TOTAL |  | 328.64 | 0.00 | 0.00 | -76.61 |  | 482.01 | -132.80 |
| I) | Overturning Stability |  | Fo=SMr/SMo |  | 3.63 | Safe | 1.50 |  |
| II) | Sliding Stability |  | Fss=ɳΣFv/FH |  | 2.57 | Safe | 1.50 |  |
| III) | Overstressing Stability |  | ΣM=Mr+Mo |  | 349.22 |  |  |  |
|  |  |  | X=M/Fv |  | 1.0626 |  |  |  |
|  |  |  | L/2 |  | 1.55 |  |  |  |
|  |  |  | e |  | 0.4874 | safe | 0.52 | B/6 |

  - Downstream Retaining Wall

The downstream masonry retaining wall is designed for 0.4 m and 1.6 m top and bottom width respectively and the stability analysis is checked as shown below.

- Stability Analysis

Table 3-7 D/s retaining wall Stability Analysis

| Item Description |  | Forces(KN) |  |  |  | Lever arm | Moment about toe(KN.m) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Vertical |  | Horizontal |  | (m.) | Resisting | Overturning |
|  |  | +ve | -ve | +ve | -ve |  |  |  |
| 1. Vertical force |  |  |  |  |  |  |  |  |
| 1.1. Self-weight(W1) |  | 24.84 |  |  |  | 0.4 | 9.94 |  |
| 1.2. Self-Weight(W2) |  | 37.26 |  |  |  | 1.00 | 37.26 |  |
| 1.4. Water pressure(Ph) |  |  |  |  | 0 |  |  | 0.00 |
| 2. Horizontal force |  |  |  |  |  |  |  |  |
| 2.1 Active pressure (Phs) |  |  |  |  | -7.25 | 0.67 |  | -4.84 |
| 2.2. Active pressure (Pvs) |  |  |  |  |  |  |  |  |
| TOTAL |  | 62.10 | 0.00 | 0.00 | -7.25 |  | 47.20 | -4.84 |
| I) | Overturning Stability |  | Fo=Mr/Mo |  | 9.76 | Safe | 1.50 |  |
| II) | Sliding Stability |  | Fss=ɳΣFv/FH |  | 5.99 | Safe | 1.50 |  |
| IV) | Overstressing Stability |  | ΣM=Mr+Mo |  | 42.36 |  |  |  |
|  |  |  | M/Fv |  | 0.68 |  |  |  |
|  |  |  | L/2 |  | 0.80 |  |  |  |
|  |  |  | e |  | 0.1179 | safe | 0.27 |  |

Here the top and bottom dimension of the wall is fixed 0.4 m and 1.6 m respectively

## Bill of Quantity and Cost Estimation

The quantities of the various items have been worked out as per the final design and final drawings prepared for the scheme. The unit rates analysis has been carried out based on the data available in the vicinity of the project area.

Bill No. 1- General Items

Table 3-8 Bill of Quantity and Cost Estimation of General Item

| Item | Bill No. 1. General | Unit | Quantity | Rate (Birr) | Amount ( Birr) |
| --- | --- | --- | --- | --- | --- |
| 1.1 | Allow for mobilization | L.S | 1 | 108,146.67 | 108,146.67 |
| 1.2 | Allow for demobilization | L.S | 1 | 108,146.67 | 108,146.67 |
| 1.3 | Allow for contractor’s camping facilities 3*4m2, Living room for contractors key personnel, CIS and internally painted clip wood wall, Masonry floor cement screened and well ventilated room complete with doors and windows. | No | 4 | 59,214.99 | 236,859.96 |
|  | Allow for Consultant’s camping facilities 3*4m2, Living room for contractors key personnel, CIS and internally painted clip wood wall, Masonry floor cement screened and well ventilated room complete with doors and windows. | No | 2 | 59,214.99 | 118,429.98 |
|  | 5*5m2, Store and dining room constructed from CIS with doors and windows, Masonry floor cement screened | No | 1 | 65,982.6 | 65,982.57 |
|  | Barbed wire fence 60*40m and 1.5m high treated timber post complete with 3m wide gate and a CIS guard house (1.5*2m) | No | 1 | 65,551.49 | 65,551.49 |
| 1.4 | Allow for temporary access road (in km) to site including drainage culvert and ditches in the direction of the engineer. | km | 0.5 | 506,534.07 | 253,267.04 |
| 1.5 | Dewatering of open trenches and excavations, pumps | LS | 1 | 113,040.00 | 113,040.00 |
| 1.6 | Provide project indicator post starting from the construction time | LS | 1 | 7,807.68 | 7,807.68 |
| 1.7 | Provision of as built drawings for the project | LS | 1 | 52,219.39 | 52,219.39 |
|  | Sub Total |  |  |  | 1,129,451.44 |

Bill No. 1- Head Work Structures

Table 3-9: Bill of Quantity of Head Work Structure

| Item No. | Item Of Work | Unit |  | Quantity |  | Unite Price | Amount Birr |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1.0 | WEIR BODY |  |  |  |  |  |  |
| 1.1 | Earth work |  |  |  |  |  |  |
| 1.1.1 | Top soil Clearing and grubbing to a maximum depth of 20 cm as per the design and to the satisfaction of the Engineer. | m2 |  | 74.25 | 11.18 |  | 830.12 |
| 1.1.2 | Soil excavation up to a depth 0.5 m | m3 |  | 0.75 | 77.39 |  | 58.04 |
| 1.1.3 | Excavation in hard rock | m3 |  | 72.60 | 601.78 |  | 43,689.23 |
| 1.1.4 | Excavation of weathered rock with (Machine) | m3 |  | 45.38 | 246.91 |  | 11,203.54 |
| 1.2 | Cyclopean concrete(60% C-20,40% graded stone) | m3 |  | 211.60 | 2,158.41 |  | 456,710.97 |
| 1.3 | C-20 concrete work | m3 |  | 19.38 | 3,117.00 |  | 60,416.81 |
| 1.4 | Reinforcement bars Ø 12 mm bars @ 200 mm c/c spacing (bending, cutting, placing and tying with construction wire) | kg |  | 1343.61 | 88.00 |  | 118,237.77 |
| Sub Total |  |  |  |  |  |  | 691,146.48 |
| 2.0 | U/S and D/S Apron |  |  |  |  |  |  |
| 2.1 | Soil excavation up to a depth of 0.5 m | m3 | 0.25 |  | 77.39 |  | 19.348 |
| 2.2 | Excavation in hard rock | m3 | 19.00 |  | 601.78 |  | 11,433.820 |
| 2.3 | Excavation in Weathered rock | m3 | 28.60 |  | 246.91 |  | 7,061.626 |
| 2.4 | Cyclopean concrete(60% C-20,40% graded stone) | m3 | 15.00 |  | 2,158.41 |  | 32,376.153 |
| 2.5 | C-20 concrete work | m3 | 10 |  | 3,117.00 |  | 31,170.000 |
| 2.6 | Reinforcement bars Ø 12 mm bars @ 200 mm c/c spacing(bending, cutting, placing and tying with construction weir) | Kg | 407.92 |  | 88.00 |  | 35,896.634 |
| Sub Total |  |  |  |  |  |  | 117,957.58 |
| 3.0 | U/S and D/S Cut off |  |  |  |  |  |  |
| 3.1 | Soil excavation up to a depth of 0.5 m | m3 | 0.00 |  | 77.39 |  | 0.00 |
| 3.2 | Excavation in hard rock | m3 | 11.55 |  | 601.78 |  | 6,950.56 |
| 3.3 | Excavation in Weathered rock | m3 | 11.55 |  | 246.91 |  | 2,851.81 |
| 3.5 | C-20 concrete work | m3 | 9.90 |  | 3,117.00 |  | 30,858.30 |
| 3.6 | Reinforcement bars Ø 12 mm bars @ 200 mm c/c spacing(bending, cutting, placing and tying with construction weir) | kg | 161.50 |  | 88.00 |  | 14,212.02 |
| 3.7 | Reinforcement bars Ø 12 mm bars @ 200 mm c/c spacing(bending, cutting, placing and tying with construction weir) | kg | 133.10 |  | 88.00 |  | 11,713.20 |
| Sub Total |  |  |  |  |  |  | 66,585.90 |
| 4.0 | Under Sluice and Divide Wall |  |  |  |  |  |  |
| 4.1 | Divide wall |  |  |  |  |  |  |
| 4.1.1 | Soil excavation up to a depth of 0.5 m | m3 | 0.00 |  | 77.39 |  | 0.00 |
| 4.1.2 | Excavation in hard rock | m3 | 7.44 |  | 601.78 |  | 4,477.24 |
| 4.1.3 | Excavation in weathered rock | m3 | 3.10 |  | 246.91 |  | 765.42 |
| 4.1.4 | Masonry wall construction using 1:3 mortar of any height with dimensions as indicated on the drawing. Price should include scaffolding. | m3 | 38.80 |  | 1,687.28 |  | 65,466.46 |
| 4.1.5 | Concrete work, C-20 | m3 | 13.64 |  | 3,117.00 |  | 42,515.88 |
| 4.1.6 | Reinforcement bar, φ12 with 200mm spacing including (bending, cutting & placing) | kg | 326.99 |  | 88.00 |  | 28,775.44 |
| 4.1.7 | Reinforcement bar, φ10 with 200mm spacing including (bending, cutting & placing) | kg | 252.90 |  | 88.00 |  | 22,255.09 |
| Sub Total |  |  |  |  |  |  | 164,255.54 |
| 4.2 | Under Sluice |  |  |  |  |  |  |
| 4.2.1 | Excavation weathered rock with (Machine) | m3 | 2.55 |  | 246.91 |  | 629.62 |
| 4.2.2 | excavation of hard rock with (Machine) | m3 | 4.08 |  | 601.78 |  | 2,455.26 |
| 4.2.3 | Concrete work, C-20 | m3 | 0.65 |  | 3,117.00 |  | 2,026.05 |
| 4.2.4 | Reinforcement bar, φ12 with 200mm spacing including (bending, cutting & placing) | Kg | 8.16 |  | 88.00 |  | 718.41 |
| Sub Total |  |  |  |  |  |  | 5,829.34 |
| 5.0 | Operational slab of under sluice and offtake |  |  |  |  |  |  |
| 5.1 | Concrete work, C-20 | m3 | 0.56 |  | 3,117.00 |  | 1,745.52 |
| 5.2 | Reinforcement bars Ø 12 mm bars @ 200 mm c/c spacing(bending, cutting, placing and tying with construction ) | kg | 38.64 |  | 88.00 |  | 3,400.27 |
| Sub total |  |  |  |  |  |  | 5,145.79 |
| 6 | Brest wall for under sluice and Offtake |  |  |  |  |  |  |
| 6.1 | Concrete work, C-20 | m3 | 2.336 |  | 3,117.00 |  | 7,281.31 |
| 6.2 | Reinforcement bar, φ12 with 200mm spacing including (bending, cutting & placing) | kg | 92.46 |  | 88.00 |  | 8,136.77 |
| Sub total |  |  |  |  |  |  | 15,418.08 |
| 7.0 | Retaining wall |  |  |  |  |  |  |
| 7.1 | Upstream Retaining Wall |  |  |  |  |  |  |
| 7.1.1 | Soil excavation up to a depth of o.5 m | m3 | 0.25 |  | 77.39 |  | 19.35 |
| 7.1.2 | Excavation in hard rock | m3 | 38.75 |  | 601.78 |  | 23,318.98 |
| 7.1.3 | Excavation in soft rock | m3 | 23.250 |  | 246.91 |  | 5,740.66 |
| 7.1.4 | Masonry wall construction using 1:3 mortar of any height with dimensions as indicated on the drawing. Price should include scaffolding. | m3 | 91.00 |  | 1,687.28 |  | 153,542.48 |
| 7.1.5 | Fill and compaction with selected material from excavated. | m3 | 31.25 |  | 45.36 |  | 1,417.50 |
| 7.1.6 | Pointing (1:2) | m² | 52.00 |  | 53.03 |  | 2,757.56 |
| 7.1.7 | Provide C-10 Lean concrete for masonry footing of thickness as per the drawing and the direction of the engineer. | m3 | 3.10 |  | 125.00 |  | 387.50 |
| Sub Total |  |  |  |  |  |  | 187,184.02 |
| 7.2 | Downstream Retaining Wall |  |  |  |  |  |  |
| 7.2.1 | Soil excavation up to a depth of 0.5 m | m3 | 0.25 |  | 77.39 |  | 19.35 |
| 7.2.2 | Excavation in hard rock | m3 | 3.24 |  | 601.78 |  | 1,946.76 |
| 7.2.3 | Excavation in weathered rock | m3 | 67.50 |  | 246.91 |  | 16,666.43 |
| 7.2.4 | Masonry wall construction using 1:3 mortar of any height with dimensions as indicated on the drawing. Price should include scaffolding. | m3 | 80.18 |  | 1,687.28 |  | 135,277.67 |
| 7.2.5 | Fill and compaction with selected material from excavated. | m3 | 60.00 |  | 45.36 |  | 2,721.60 |
| 7.2.6 | Pointing (1:2) | m² | 45.63 |  | 53.03 |  | 2,419.49 |
| 7.2.7 | Provide C-10 Lean concrete for masonry footing of thickness as per the drawing and the direction of the engineer. | m3 | 2.40 |  | 125.00 |  | 300.00 |
| Sub total |  |  |  |  |  |  | 159,351.30 |
| 8.0 | Gates |  |  |  |  |  |  |
| 8.1 | Under sluice gate installation consist of: | No | 1.00 |  | 28,321.47 |  | 28,321.47 |
|  | 6mm thickness sheet metal,1.1 m X0.95 m |  |  |  |  |  |  |
|  | 50 x 50 x 10mm angle iron for groove, 4.85m length |  |  |  |  |  |  |
|  | Φ10mm bar for anchorage, 20Kg |  |  |  |  |  |  |
|  | Stiffening angle iron (30*30*10) |  |  |  |  |  |  |
|  | Spidle of 40mm dia ,4.6m length |  |  |  |  |  |  |
| 8.2 | Offtake canal gate installation consist of: |  |  |  |  |  |  |
|  | 4mm thickness sheet metal,0.55m X0.6m. | No | 1.00 |  | 25,321.47 |  | 25,321.47 |
|  | 40x40x10mm angle iron, 4.1m length |  |  |  |  |  |  |
|  | Stiffening angle iron (30*30*10) |  |  |  |  |  |  |
|  | Anchor bar , φ10mm |  |  |  |  |  |  |
|  | Spindle Ø40mm,4.6m long |  |  |  |  |  |  |
| Sub Total |  |  |  |  |  |  | 53,642.94 |
| 8.3 | Trash rack reinforcement bar, φ20 with 100mm spacing | kg | 187.33 |  | 88 |  | 16485.2512 |
|  | Sub Total |  |  |  |  |  | 70,128.19 |
| Grand Total |  |  |  |  |  |  | 1,483,002.22 |

# SECTION-III: IRRIGATION AND DRAINAGE SYSTEMS

# INFRASTRUCTURE

# IRRIGATION AND DRAINAGE SYSTEMS DESIGN

## Irrigable Area Description

### Topography

Topography is an important factor for the planning of any irrigation project as it influences method of irrigation, drainage, erosion, mechanization, and cost of land development, labor requirement and choice of crops.

The topographic feature of the project command area is mainly sloping type. Its elevation range is from 2596.50 to 2460 meters above sea level. The slope gradient also ranges from gently sloping (30%) to strongly sloping (10%). However, it has identified to be suitable for surface irrigation. Nevertheless, it requires soil and water conservation measures or structures (i.e. constructing bunds, bio-physicals, check dams, artificial water ways, etc.).

The project command area is situated at the left side of Mayis River. The natural topographic feature of the command area has inclined from the West to the East direction.

### Climate

Climate has an important influence on the nature of the natural vegetation, the characteristics of the soil, the crops that can be grown and the types of farming that can be practiced in any region. The climate of an area is highly correlated with its vegetation and, by extension, the types of crops that can be cultivated.

### Soil characteristics

Soil properties (physical, chemical, etc.) greatly influence the growth and thereby yield of crops which is grown. The command area has predominantly clay textured soils which can be classified as sandy loam soil.  Most of the study area soils are categorized as deep soil (>1.5 meter depth). Soils of the command area are suitable for most of the selected crops to be grown (for further detail see the Agronomy Study of the same project.

### Existing Irrigation Practices in the Project Area

The pressure of survival and the need for additional food supplies to meet the demands of the increasing population is necessitating a rapid expansion of irrigation schemes. Thus, irrigation is becoming a basic part of well-developed agriculture wherever there is water and irrigable land potential. Accordingly, traditional irrigation practices are under taken by individual farmers that use the river flow is with laborious temporary diversions. So, the farmers in the project area are very much interested in the idea of upgrading the traditional scheme to modern scheme.

## Irrigation Water Requirement

### Crop Water Requirement (CWR)

The calculation of crop water requirement is a very important aspect for planning of any irrigation project. Several methods and procedures are available for this. The Food and Agriculture Organization (FAO) of the United Nations has also made available several publications on this subject and other issues related with this. The computer program available in FAO Irrigation and Drainage Paper No. 56 “CROPWAT” has been used for the calculation of Crop Water requirement. This program is based on Penman-Monteith approach and procedures for calculation of crop water requirements and irrigation requirements are mainly based on methodologies presented in FAO Irrigation and Drainage Paper No. 24 “Crop Water Requirements” and No. 33 “Yield Response to Water”.

The corresponding values of the crop water requirements of the proposed crops of the project are presented in the Agronomy Study of the same project.

### Irrigation efficiency (Ep)

To complete the evaluation of the demand, efficiency of water distribution system and of efficiency field application must be known.

The gross requirement of water for irrigation system is very much dependent on the overall efficiency of the irrigation system, which in turn is dependent on several factors: Method of irrigation, type of canal (Lined and/or Unlined), method of operations (simultaneously and continuous or rotational water supply), and availability of structures (for controlling and distribution and measuring and monitoring).

On the basis of these factors it has planned to impose surface (furrow, etc.) irrigation method. Concerning the irrigation canal systems of the project it has designed to have one main canal and multi field canals using PVC pipes. Therefore, conveyance efficiency has been estimated to be 95% (based on the engineering study report); while the field application efficiency 60% which is determined by soil texture. The command area is clay loam type (based on soil lab results). Hence, the overall/project efficiency is estimated 55% as it has seen from the calculation below.

e = ec * ea = 95*60 = 57%≈55%

100        100

Where, e = Scheme irrigation efficiency (%)

ec = Conveyance efficiency (%)

ea = Field application efficiency (%)

The aforementioned three steps will be able to give the crop water requirement during its cropping period. The losses in the irrigation system have to be incorporated to arrive at the water requirement at the head of the irrigation system.

### Irrigation Duty

Irrigation duty is the volume of water required per hectare for the full flange of the crops. It also helps in designing efficient irrigation canal systems. The irrigable area can be calculated by knowing the total available water at the source; and the overall duty for all crops required to be irrigated in the two seasons of the year.

For the designing of the irrigation water application and flows in the entire irrigation system it is advisable to take the average net irrigation requirement of scheme supply for the computation of irrigation duty. The average net irrigation requirement for the computation of dry season irrigation duty for 20 irrigation hours is 4.2mm/day in the month of February.  For wet season crops the average net irrigation requirement is 2.7mm/day in the month of October.

For the designing of the project, the GIWR for dry season crops is given as follows:

GIWR = NIWR/e = 4.2/0.48 = 8.75 [mm/day]

The GIWR (8.75mm/day) represents the daily quantity of water that is required to be applied for dry season crops. This water quantity is also used for the determination of the system discharge in consideration of the time of flow and also defined as the duty, expressed in l/s/ha.

The duty is calculated by:

Duty (D) = GIWR × 1000 × 10 / (t × 60×60)

Where; Duty – the duty [l/s/ha]

GIWR – Gross Irrigation Requirement [mm/day]

t – Daily irrigation or flow hours [hrs.]

The duty for GIWR of 8.75mm/day for daily irrigation time 20 hours of dry season full irrigation is computed as:

D = (8.75x10x1000) / (20x3600) = 1.22l/s/ha

The GIWR for wet season crops is given as follows:

GIWR = NIWR/e = 2.7/0.48 = 5.6mm/day

The GIWR (5.6mm/day) represents the daily quantity of water that is required to be applied for wet season crops as supplementary irrigation.

Duty (D) = GIWR × 1000 × 10 / (t × 60×60)

Where; Duty – the duty (l/s/ha)

GIWR – Gross Irrigation Requirement (mm/day)

t – Daily irrigation or flow hours (hrs.)

The duty for GIWR of 5.6mm/day for daily irrigation time of 18 hours for supplementary irrigation has been computed as:

D = (5.6x10x1000) / (20x3600) = 0.78l/s/ha

### Irrigation Methods

The irrigation water is obtained from ground water and the distribution system will be through main, secondary and tertiary canals using PVC pipelines leading to field canals. Furrow irrigation method is best suited for most of the proposed and row planted crops. In general, furrow irrigation method is simple, manageable and widely practiced irrigation method. This method is suitable for row crops that cannot stand in water for long periods. The only thing required to use this method is row planting of crops. Rotational flow water distribution is also recommended for the project area.

### Irrigation and Drainage System Layout

The irrigation system layout for the project is prepared taking the following points into consideration besides other factors.

- A primary concern in the layout of the system is that it serves the purpose of conveying and distributing water to the command area.

- The excavation and earth fill volumes not be excessive, otherwise the construction costs can be tremendous.

- The selection of longitudinal bed slope is made taking into account the existing slopes of the terrain, so as to minimize deviations in canal routing.

- Curves in canals should not be too sharp.

The proposed irrigation system layout comprises 1 main canal, 2 secondary canal, 8 tertiary canal and 15 field canals as shown on the layout Drawings. The main canal runs for most of its length parallel to the contours and several changes of direction are necessary to follow the topography. It crosses 3 main gullies. The main canal is masonry lined for a length of 3650 meters starting from the weir outlet to make maintenance easier since this part of the canal may be subjected to flooding during high flood flows.

### Conveyance System

The conveyance system consists of 1 Main canal to irrigate total command area of 72.45 ha. The main canal starts from Water abstraction site on right side and conveys water for a length of 3.65 Km. Main canal is aligned along contours and supplies to 15 field units.

## Design of the Canal System

### Main Canal

The main canal is designed for a discharge of 85.49 l/s and depending on the site specific condition, appropriate slope is provided. Hydraulic parameters of the main canal are shown is shown in detail below.

Table 4-1 hydraulic parameter for main canal

### Secondary Canals

In the layout system there are 2 secondary canals, the designed discharge is determined based on the duty of irrigation. The number of secondary canals for development of 72.5ha net command area is eight in which the density of canal is based on the number of command between gullies is accounted and command topography accounted.  The sections of the canals are determined by using manning’s formula, and they are rectangular section.

Table 4-2 hydraulic parameter for secondary canal

### Tertiary canals

In the layout system there are 8 tertiary canals, the designed discharge is determined based on the duty of irrigation and the command area which each tericiary canal can irrigate.  The sections of the canals are determined by using manning’s formula, and they are rectangular section. Among the different irrigation systems rotational irrigation system will be used for the tertiary canals; and the irrigation water will be obtained from the secondary canals. It is impossible to attach all the hydraulic parameter for 8 tericiary canals.

Table 4-3 hydraulic parameter for tertiary canal

### Field Canals

The earthen field canals have been designed with a trapezoidal shape and the lined ones with rectangular x-section using Manning's Formula:

Where

- Q is  discharge (m3/s)

- R is Hydraulic radius (Flow area/wetted perimeter)

- S is Hydraulic gradient

- N is Manning's roughness coefficient, n = 0.025 is adopted for the earth channels and n = 0.018 for the masonry lined part of the main canal

As shown in the layout, field canals run across the contours and hence face relatively steeper gradient. The discharge of most of the field canals is very small and this is taken as an advantage to cope up with the relatively steeper gradient. As much as possible field canals shall be made in fill in order to easily irrigate the adjacent command area. As can be seen from the layout, majority of the filed canals can be used to irrigate both sides of the command area depending on the condition of the individual plots of land owned by individual farmers.

The design approach of field canals for this particular project is done considering the followings:

1. The scale of the project: small scale project

2. The natural topographic condition of the command area/slope and soil natures undulating and sloppy area.

3. The maximum and minimum length of field canals to irrigate the farm lands

4. The velocity of flow in the canals whether to be erosive or silting

5. The construction of these field canals is to be covered by the farmers in order to increase sense of ownership.

From the above considerations the field canals of this small scale project are designed to be excavated with the same slope of the natural ground as far as the flow velocities are found neither scouring nor silting based on the ground slope as estimated in the following table. This is done by keeping the minimum workable dimensions for field canals and according to the alignments of the field canals on the topographic map.

### Catch Drainage Canals

In the layout system Catch drain canals are designed for headwork one main canal which can also serve for headwork two main canals as well. And hence there is no catch drain canal for headwork two. Discharge is determined based on the Catchment area, run off coefficient and rain fall intensity (Rational formula).  The sections of the canals are determined by using manning’s formula, and they are trapezoidal section.

Figure 4-1typical cross section of catch drain canals

## Canal Structures Design

### Design of a typical flume

For Maise two project  the rout of main canal  follows counter along terrain topography as a result there are 5 gully crossing structures from here 3 are flume structures and the remaining are  2 supper passage. For 3 flumes structures there hydrologic, hydraulic and structural analyses have been done.

#### Hydrologic Characteristics of flume

Hydrologic analysis  have been done for large gullies which has relatively significant stream flow, catchment area and stream line at GIS delineation. Hence the contribution catchment area very small we used the rational method to compute the peak flood analysis of the gullies.  The analysis is as follows in the table.

Table 4-4:- GIS output for flum-2 hydrological analysis

| maise2- flum-2 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | com(L) | Elevation | Elevation-diff | TC | slope | catchmnt area(ha) |
|  | 0 | 2941 | 0 | 0 |  |  |
| 0.324 | 0.324 | 2900 | 141 | 0.038374 |  |  |
| 1.77 | 2.094 | 2700 | 220 | 0.229818 |  |  |
| 0.71 | 2.804 | 2638 | 140 | 0.095225 | 108.0599 |  |
| 2.804 |  |  |  | 0.363418 | 0.009254 | 265.98 |

Table 4-5:- hydrological analysis of flum-2

| Chain age | Drainage name | Length(m) | Time of concentration(min) | Runoff coefficient( C ) | RF Intensity(I)mm/hr | Catchment area A(ha). | Q from catch (m3/s) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1+444 | flume-2 | 2804.000 | 21.810 | 0.720 | 150 | 265.980 | 79.794 |

| Table 4-6:- tail water discharge computation of flume 2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| S.N | Elevation/m/ | water depth /m/ | wetted Area /m^2 | perimeter /m/ | Hydraulic radius/R/ | slope | velocity /m/s | Discharge/m^3/s/ |  |
| 1 | 2592.104 | 0 | 0 | 0 | 0 | 0.009 | 0 | 0 |  |
| 2 | 2592.604 | 0.5 | 3.285 | 26.300 | 0.125 | 0.009 | 0.687 | 2.257 |  |
| 3 | 2593.104 | 1 | 12.714 | 46.093 | 0.276 | 0.009 | 1.165 | 14.812 |  |
| 4 | 2593.604 | 1.5 | 25.868 | 59.422 | 0.435 | 0.009 | 1.579 | 40.850 |  |
|  |  |  |  |  |  |  |  | 79.794 | flood at gully |
| 5 | 2594.104 | 2 | 42.331 | 72.558 | 0.583 | 0.009 | 1.920 | 81.257 |  |
| 6 | 2594.604 | 2.5 | 61.769 | 83.749 | 0.738 | 0.009 | 2.244 | 138.628 |  |
| 7 | 2595.104 | 3 | 84.012 | 95.405 | 0.881 | 0.009 | 2.526 | 212.198 |  |
| 8 | 2595.604 | 3.5 | 109.581 | 110.378 | 0.993 | 0.009 | 2.736 | 299.818 |  |

#### Hydraulic Characteristics of the canal

There are 3main flumes across the main canal. The following table is containing their chain age, length, and depth.

Table 4-7:- hydraulic parameter of flume

| Flume No. | Starting Chain age | Type of Drainage Flow | Flume Length | Q(m3/s) | V(m/s) | h | t1 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 0+110.74 | Flume | 20 | 0.085 | 0.763 | 2.89 | 0.5 |
| 2 | 1+444.26 | Flume | 70 | 0.085 | 0.763 | 7.60 | 0.5 |
| 3 | 3+379.02 | Flume | 30 | 0.085 | 0.763 | 1.16 | 0.5 |

#### Structural Characteristics of flume

In case of Maise-two irrigation project there are 3 flume structures which has maximum pier depth of 8.6m and minimum depth of 1m so the structural analysis have done for pier height greater than 6m one sample using 8.6m pier height and another one sample for pier height less than 6m done one sample structural analysis using maximum pier height 5m. But the slab and the side wall analysis are the same for all flume structures. The detail analysis at the excel analysis.

|  | Design of Flume for Maise Main Canal |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Flume Cross Section |  |  |  |  |  |
|  |  |  | in side dimensions |  |  |  |
|  |  |  |  | h = | 0.6 | m |
|  |  |  |  | w= | 0.5 | m |
|  |  |  |  | t (Wall) | 0.2 | m |
|  |  |  |  | t(Bas S.) | 0.2 | m |
|  |  |  | Material Properties |  |  |  |
|  |  |  | Unit Wt. of Water |  | 10 | KN/m3 |
|  |  |  | Unit Wt. of RC |  | 24 | KN/m3 |
|  |  |  | fck |  | 25 | Mpas |
|  |  |  | fy |  | 400 | Mpa. |
|  |  | Design of Flume Section |  |  |  |  |
|  |  | Load Asseeement |  |  |  |  |
|  |  |  | Concrete load ( DL) |  | 10.08 | KN/m |
|  |  |  | Water Load |  | 3 | KN/m |
|  |  | Load on Flume Section |  |  |  |  |
|  |  | at Bottom of Wall water Load |  |  | 7 | KN/m2 |

Figure 4-2  live load in flume                                                    Figure 4-3 service bending moment

Figure 4-4shear forces diagram (service)                                       Figure 4-5shear forces diagram (strength)

|  | 3 | Bracket Design ( Pier Cup) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |

Figure 4-6 pier cap cross section

### Design of division box

For Mayis-2 irrigation project there are 8 division box provided at different points of the main and secondary canals division boxes are provided which divert the flow to the secondary canal and tertiary canals. Because the command area is inclined Standard division box plan and sections are for one way opening Gate should be provided at the outlet of the boxes. For detail refer the drawing.

Figure 4-7 division box

Using broad crested formula,

Q= CL (h)3/2

Where; Q= discharge over rectangular weir/intake (opening), m3/s

C = discharge coefficient, c= 1.7

L= effective length of crest form in m

h= over flow depth, m

Assuming equal discharge coefficient & sill height for two or three dividing canals, the proportion becomes.

Q0/ Q1= Q1/ Q2 =L0/ L1 = L1/ L2

Where Q0= is flow in canal 1

Q1 = is flow in canal 2

Q2 = is flow in canal 3

L0= is effective crest length of weir/intake sill across opening to canal 1

L1= is effective crest length of weir/intake sill across opening to canal 2

L2 = is effective crest length of weir/intake sill across opening to canal 3

Q1= CL1 (h)3/2,

L0 = Q1/Ch3/2

L1 = L1*Q1/ Q0

L32= L0*Q2/ Q0

The depth of (height of) the division box,

D = d + fb

The width of the division box,

B = b + 2*m*D

Where b= base width of the incoming canal

D = total canal depth of the incoming canal

Table 4-8:- hydraulic parameter of division box

### Design of Turnout

At different points of the main and field canals turnouts are provided which divert the flow from main canal to field canals. Gate should be provided at the outlet of the turnouts and the main canal.

Table 4-9:- hydraulic parameter of turn outs

### Design of Drop Structure

For irrigation canals vertical drop structure of U.S.B R type standard is selected to convey water from higher to lower elevation of the ground.  The design procedures are shown below.

a. Critical hydraulic

- Design discharge, Q (m3/s)

- Height of drop, h (m)

- Width of drop, bc =

Where d = water depth of the canal, m

- Critical discharge, q = Q/bc

- Critical depth, dc =

- Lip height, a = dc/2, a  0.1

b. Stilling basin

- Basin width, B =

- Basin length, L =

All the hydraulic parameters computed using the above procedures with the type of

- L1=U/S protection

- L2= D/s protection

- b1=top width of drop wall =0.4m

- t=Thickness of the stilling basin=0.3m

Table 4-10:-hydraulic parameter of drop structure

| Drop On | Chain age | Q(m3/s) | V(m/s) | Bed Width, B | Depth, D | Drop ,H |
| --- | --- | --- | --- | --- | --- | --- |
| SC | 0+00 | 0.057 | 1.138 | 0.40 | 0.30 | 1.0 |
| SC | 0+30 | 0.057 | 1.138 | 0.40 | 0.30 | 1.0 |
| SC | 0+310 | 0.057 | 1.138 | 0.40 | 0.30 | 0.5 |
| SC | 0+360 | 0.057 | 1.138 | 0.40 | 0.30 | 0.8 |

(Note; Generally, the detail drawing and hydraulic parameters of the infrastructure are described in the drawing part.)

Figure 4-8 typical cross section of vertical drop structure

## Irrigation Infrastructure Bill of Quantities and Cost Estimate

Table 4-11:- irrigation infrastructure BOQ & cost estimation

| BILL NO- 3 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| B | INFRASTRUCTURES |  |  |  |  |
| Item No. | ITEM OF WORK | UNIT | QUANTITY | UNITE PRICE | AMOUNT BIRR |
| 1.0 | MAIN CANAL |  |  |  |  |
| 1.1 | Earth work |  |  |  |  |
| 1.1.1 | Top soil Clearing and grubbing to a maximum depth of 20 cm as per the design and to the satisfaction of the Engineer. | m2 | 10318.51 | 11.18 | 115360.942 |
| 1.1.2 | Soil excavation up to a depth of 1.5 m including cart away | m3 | 9079.73 | 88.20 | 800832.362 |
| 1.1.3 | Excavation in hard rock up to a depth of 1.5 m | m3 | 399.29 | 646.22 | 258032.027 |
| 1.1.4 | Excavation in soft rock up to a depth of 1.5 m | m3 | 99.82 | 316.54 | 31597.023 |
| 1.1.5 | Fill and compaction with selected material from excavated. | m3 | 12390.55 | 45.36 | 562035.44 |
| 1.1.6 | Fill and compaction with selected material exported from approved boro pits of radius less than 5 km. | m3 | 3097.64 | 125.33 | 388226.97 |
| 1.2 | Canal Lining |  |  |  |  |
| 1.2.1 | Masonry Lining and wall construction using 1:3 mortars for canals of any height with dimensions as indicated on the drawing. Price should include scaffolding. | m3 | 3875.10 | 1436 | 5564643.60 |
| 1.2.2 | Plastering (three coats) with a 1:2 cement mortar of 25 mm thickness | m2 | 9687.75 | 109.42 | 1060033.61 |
| 1.2.3 | Provide C-10 Lean concrete for masonry footing lining of thickness as per the drawing and the direction of the engineer. | m2 | 474.50 | 125 | 59312.50 |
| 3.8 | escaping canal |  |  |  |  |
| 3.8.1 | excavation of ordinary soil | m3 | 27.9 | 88.20 | 2460.78 |
| 3.8.2 | masonry work(1:3 ratio) | m3 | 6.5 | 1,436 | 9305.28 |
| 3.8.3 | back fill with excavated material | m3 | 3.2 | 102.31 | 331.48 |
| 3.8.4 | plane concrete (1:3 ratio) | m3 | 3.8 | 646.22 | 2442.71 |
| 3.8.5 | plastering work (1:3 ratio) | m2 | 16.2 | 109.42 | 1772.60 |
| 3.8.6 | 0.7m*0.6m gate 4mm thick sheet metal with 10mm dia with ring dia.15cm for handling | no | 9.0 | 6500 | 58500.00 |
| 3.9 | gabion work |  |  |  |  |
| 3.9.1 | excavation of ordinary soil | m3 | 13.5 | 88.20 | 1190.70 |
| 3.9.2 | stone field gabion work | m3 | 13.5 | 1,400 | 18900.00 |
| 3.9.3 | back fill with excavated material | m3 | 5.4 | 102.31 | 552.47 |
| 1.3 | Division Boxes |  |  |  |  |
| 1.3.1 | Soil excavation up to a depth of 1.5m including cart away | m3 | 1.510 | 88.2 | 133.18 |
| 1.3.2 | Masonry Lining and wall construction using 1:3 mortar of any height with dimensions as indicated on the drawing. Price should include scaffolding. | m3 | 2.87 | 1436 | 4121.32 |
| 1.3.3 | Plastering (three coats) with a 1:2 cement mortar of 25 mm thickness | m2 | 6.12 | 109.42 | 669.65 |
| 1.4 | Gate |  |  |  |  |
| 1.4.1 | 6 mm thick sheet metal | m2 | 0.60 | 2000 | 1200.00 |
| 1.4.2 | 60*40*3 mm angle iron | m | 3.76 | 700 | 2632.00 |
| 1.4.3 | C-20 Concrete work | m3 | 0.19 | 3117 | 592.23 |
| 1.4.4 | ф 12 mm bar for handling | kg | 0.70 | 88 | 61.60 |
| 1.5 | Turnout Structures |  |  |  |  |
| 1.5.1 | Soil excavation up to a depth of 1.5 m | m3 | 2.13 | 77.39 | 164.84 |
| 1.5.2 | Masonry Lining and wall construction using 1:3 mortar of any height with dimensions as indicated on the drawing. Price should include scaffolding. | m3 | 2.16 | 1436 | 3101.76 |
| 1.5.3 | Plastering (three coats) with a 1:2 cement mortar of 25 mm thickness | m2 | 10.5 | 109.42 | 1148.91 |
| 1.5.4 | Stone pitching | m3 | 7.78 | 536.22 | 4171.79 |
| 1.5.5 | Provide C-10 Lean concrete for masonry footing lining of thickness as per the drawing and the direction of the engineer. | m3 | 8.82 | 125 | 1102.500 |
| 1.6 | Gate |  |  |  |  |
| 1.6.1 | 6 mm thick sheet metal | m2 | 1.93 | 2000 | 3860.000 |
| 1.6.2 | 60*40*3 mm angle iron | m | 4.50 | 700 | 3150.000 |
| 1.6.3 | ф 10 mm bar for handling | kg | 0.27 | 88 | 23.760 |
| 1.7 | Flume Structure -1@ 110.74 |  |  |  |  |
| 1.7.1 | Top soil Clearing and grubbing to a maximum depth of 20 cm as per the design and to the satisfaction of the Engineer. | m2 | 15.67 | 11.18 | 175.19 |
| 1.7.2 | Soil excavation up to a depth of 1.5 m without caraway | m3 | 17.867 | 77.39 | 1382.73 |
| 1.7.3 | Excavation in hard rock up to a depth of 1.5 m | m3 | 25.60 | 646.22 | 16543.23 |
| 1.7.4 | Fill and compaction with selected material from excavation | m3 | 4.6956 | 45.36 | 212.99 |
| 1.7.5 | C-20 Concrete work | m3 | 8.95 | 3117 | 27897.15 |
| 1.7.6 | Provide C-10 Lean concrete for masonry footing lining of thickness as per the drawing and the direction of the engineer. | m3 | 0.57 | 125.00 | 70.94 |
| 1.7.7 | Masonry wall construction using 1:3 mortar of any height .The thickness of masonry wall will be as indicated on the drawing. Price should include scaffolding. | m3 | 24.2 | 1436 | 34751.20 |
| 1.7.8 | Plastering (three coats) with a 1:2 cement mortar of 25 mm thickness | m2 | 45.6 | 109.42 | 4988.46 |
| 1.7.9 | Supply and installation of 25 mm thick water stops for construction joints as per the drawings. | m | 45.0 | 400 | 18000 |
| 1.7.10 | Gabion works | m3 | 5.34 | 793.44 | 4236.97 |
| 1.7.11 | Reinforcing Bars |  |  |  |  |
| 1.7.12 | Reinforcement bars Ø 12 mm bars @ 200 mm c/c spacing(bending, cutting, placing and tying with construction wire) | kg | 1472.88 | 88 | 129613.44 |
| 1.7.13 | Reinforcement bars Ø 8 mm bars @ 200 mm c/c spacing(bending, cutting, placing and tying with construction wire) | Kg | 174.01 | 88 | 15312.79 |
| 1.8 | Flume Structure-2 @ 1+444.26 |  |  |  |  |
| 1.8.1 | Top soil Clearing and grubbing to a maximum depth of 20 cm as per the design and to the satisfaction of the Engineer. | m2 | 19.87 | 11.18 | 222.11 |
| 1.8.2 | Soil excavation up to a depth of 1.5 m | m3 | 22.19 | 77.39 | 1717.12 |
| 1.8.3 | Fill and compaction with selected material from excavation | m3 | 5.17 | 45.36 | 234.29 |
| 1.8.4 | C-20 Concrete work | m3 | 17.65 | 3117 | 55008.82 |
| 1.8.5 | Provide C-10 Lean concrete for masonry footing lining of thickness as per the drawing and the direction of the engineer. | m3 | 0.55 | 125 | 68.75 |
| 1.8.6 | Masonry wall construction using 1:3 mortar of any height .The thickness of masonry wall will be as indicated on the drawing. Price should include scaffolding. | m3 | 25.31 | 1436 | 36345.33 |
| 1.8.7 | Plastering (three coats) with a 1:2 cement mortar of 25 mm thickness | m2 | 107.1 | 109.42 | 11717.13 |
| 1.8.8 | Supply and installation of 25 mm thick water stops for construction joints as per the drawings. | m | 126.00 | 400 | 50400 |
| 1.8.9 | Gabion works | m3 | 7.23 | 793.44 | 5736.57 |
| 1.8.10 | Reinforcing Bars |  |  |  |  |
| 1.8.11 | Reinforcement bars Ø 12 mm bars @ 200 mm c/c spacing(bending, cutting, placing and tying with construction wire) | kg | 1675.83 | 88 | 147473.04 |
| 1.8.12 | Reinforcement bars Ø 8 mm bars @ 200 mm c/c spacing(bending, cutting, placing and tying with construction wire) | Kg | 548.59 | 88 | 48275.57 |
| 1.9 | Flume Structure-3 @ 3+379.02 |  |  |  |  |
| 1.9.1 | Top soil Clearing and grubbing to a maximum depth of 20 cm as per the design and to the satisfaction of the Engineer. | m2 | 14.9 | 11.18 | 166.58 |
| 1.9.2 | Soil excavation up to a depth of 2.5 m | m3 | 16.63 | 77.39 | 1287 |
| 1.9.3 | Excavation in hard rock up to a depth of 0.5 m | m3 | 11.23 | 646.22 | 7257.05 |
| 1.9.4 | Fill and compaction with selected material from excavation | m3 | 3.87 | 85.25 | 329.92 |
| 1.9.5 | C-20 Concrete work | m3 | 13.24 | 3117 | 41256.61 |
| 1.9.6 | Provide C-10 Lean concrete for masonry footing lining of thickness as per the drawing and the direction of the engineer. | m3 | 0.57 | 125.0 | 70.94 |
| 1.9.7 | Masonry wall construction using 1:3 mortar of any height .The thickness of masonry wall will be as indicated on the drawing. Price should include scaffolding. | m3 | 18.98 | 1687.3 | 32024.57 |
| 1.9.8 | Plastering (three coats) with a 1:2 cement mortar of 25 mm thickness | m2 | 80.3 | 109.42 | 8788.61 |
| 1.9.9 | Supply and installation of 25 mm thick water stops for construction joints as per the drawings. | m | 64.0 | 400.0 | 25600 |
| 1.9.10 | Gabion works | m3 | 5.42 | 793.44 | 4300.44 |
| 1.9.11 | Reinforcing Bars |  |  |  |  |
| 1.9.12 | Reinforcement bars Ø 12 mm bars @ 200 mm c/c spacing(bending, cutting, placing and tying with construction wire) | kg | 895.10 | 88 | 78768.80 |
| 1.9.13 | Reinforcement bars Ø 8 mm bars @ 200 mm c/c spacing(bending, cutting, placing and tying with construction wire) | Kg | 646.27 | 88 | 56871.58 |
| 2.0 | foot path |  |  |  |  |
| 2.0.1 | masonry work(1:3 ratio) | m3 | 11.0 | 1,436 | 15796.00 |
| 2.0.2 | plastering work(1:3 ratio) | m2 | 11.0 | 109.42 | 1203.62 |
| 2.0.3 | concrete work (c-20) | m3 | 5.1 | 3,117 | 15943.46 |
|  | reinforced bar including supply cutting & welding |  |  |  |  |
|  | Ø12mm | kg | 96.1 | 88 | 8457.35 |
| 2.1 | Supper Passage Structure @ 0+820.23 |  |  |  |  |
| 2.1.1 | Top soil Clearing and grubbing to a maximum depth of 20 cm as per the design and to the satisfaction of the Engineer. | m2 | 7.50 | 11.18 | 83.85 |
| 2.1.2 | Soil excavation up to a depth of 2.5 m without caraway | m3 | 20.25 | 77.39 | 1567.1475 |
| 2.1.3 | Fill and compaction with selected material from excavation | m3 | 5.34 | 125.33 | 669.2622 |
| 2.1.4 | C-20 Concrete work | m3 | 2.70 | 3117 | 8415.90 |
| 2.1.5 | Reinforcement bars Ø 12 mm bars @ 200 mm c/c spacing(bending, cutting, placing and tying with construction wire) | Kg | 366.30 | 88 | 32234.40 |
| 2.1.6 | Provide C-10 Lean concrete for masonry footing lining of thickness as per the drawing and the direction of the engineer. | m3 | 0.75 | 125.0 | 93.75 |
| 2.1.7 | Masonry wall construction using 1:3 mortar of any height .The thickness of masonry wall will be as indicated in the drawing. Price should include scaffolding. | m3 | 8.10 | 1436.0 | 11631.60 |
| 2.1.8 | Plastering (three coats) with a 1:2 cement mortar of 25 mm thickness | m2 | 13.50 | 109.42 | 1477.17 |
| 2.1.9 | Gabion works | m3 | 7.50 | 793.44 | 5950.80 |
| 2.2 | Supper Passage Structure @ 3+168.88 |  |  |  |  |
| 2.2.1 | Top soil Clearing and grubbing to a maximum depth of 20 cm as per the design and to the satisfaction of the Engineer. | m2 | 15.01 | 11.18 | 167.8118 |
| 2.2.2 | Soil excavation up to a depth of 2.5 m | m3 | 40.50 | 77.39 | 3134.295 |
| 2.2.3 | Fill and compaction with selected material exported from approved boro pits of radius less than 5 km. | m3 | 10.50 | 125.33 | 1315.965 |
| 2.2.4 | C-20 Concrete work | m3 | 5.4 | 3117 | 16831.80 |
| 2.2.5 | Reinforcement bars Ø 12 mm bars @ 200 mm c/c spacing(bending, cutting, placing and tying with construction wire) | Kg | 732.60 | 88.0 | 64468.80 |
| 2.2.6 | Provide C-10 Lean concrete for masonry footing lining of thickness as per the drawing and the direction of the engineer. | m3 | 1.50 | 125.0 | 187.50 |
| 2.2.7 | Masonry wall construction using 1:3 mortar of any height .The thickness of masonry wall will be as indicated in the drawing. Price should include scaffolding. | m3 | 16.20 | 1436.0 | 23263.20 |
| 2.2.8 | Plastering (three coats) with a 1:2 cement mortar of 25 mm thickness | m2 | 27.00 | 109.42 | 2954.34 |
| 2.2.9 | Gabion works | m3 | 15.00 | 793.44 | 11901.60 |
| C | SECONDARY CANAL |  |  |  |  |
| 1.0 | MC-SC2 | UNIT |  |  |  |
| 1.1 | Earth work |  |  |  |  |
| 1.1.1 | Top soil Clearing and grubbing to a maximum depth of 20 cm as per the design and to the satisfaction of the Engineer. | m2 | 1568.47 | 11.18 | 17535.49 |
| 1.1.2 | Soil excavation up to a depth of 1.5 m including cart away | m3 | 1234.49 | 88.20 | 108882.02 |
| 1.1.3 | Excavation in soft rock up to a depth of 1.5 m | m3 | 12.34 | 316.54 | 3906.10 |
| 1.1.4 | Filling with selected material from excavation including compaction. | m3 | 1687.88 | 45.36 | 76562.24 |
| 1.1.5 | Fill and compaction with selected material exported from approved boro pits of radius less than 5 km. | m3 | 421.97 | 125.33 | 52885.50 |
| 1.2 | Canal Lining |  |  |  |  |
| 1.2.1 | Masonry Lining and wall construction using 1:3 mortar for canals of any height .The thickness of masonry wall will be as indicated in the drawing. Price should include scaffolding. | m3 | 511.64 | 1436 | 734715.04 |
| 1.2.2 | Plastering (three coats) with a 1:2 cement mortar of 25 mm thickness | m2 | 1279.09 | 109.42 | 139958.03 |
| 1.2.3 | Provide C-10 Lean concrete for masonry footing lining of thickness as per the drawing and the direction of the engineer. | m2 | 71.50 | 125 | 8937.50 |
| 1.3 | Division Boxes |  |  |  |  |
| 1.3.1 | Soil excavation up to a depth of 1.50 m | m3 | 3.23 | 77.39 | 249.97 |
| 1.3.2 | Masonry wall construction using 1:3 mortar of any height .The thickness of masonry wall will be as indicated in the drawing. Price should include scaffolding. | m3 | 13.39 | 1436 | 19228.04 |
| 1.3.3 | Plastering (three coats) with a 1:2 cement mortar of 25 mm thickness | m2 | 38.25 | 109.42 | 4185.32 |
| 1.3.4 | Stone pitching | m3 | 14.18 | 536.22 | 7603.60 |
|  | Gate |  |  |  |  |
| 1.3.6 | 6 mm thick sheet metal | m2 | 1.77 | 2000 | 3540.00 |
| 1.3.7 | 60*40*3 mm angle iron | m | 7.50 | 700 | 5250.00 |
| 1.3.8 | Provide C-10 Lean concrete for masonry footing lining of thickness as per the drawing and the direction of the engineer. | m2 | 0.84 | 125 | 105.00 |
| 1.3.9 | ф 10 mm bar for handling | kg | 5.55 | 88 | 488.40 |
| 2.4 | Drop Structures |  |  |  |  |
| 2.4.1 | Soil excavation up to a depth of 1.5 m | m3 | 5.84 | 77.39 | 451.96 |
| 2.4.2 | Masonry wall construction using 1:3 mortar of any height .The thickness of masonry wall will be as indicated in the drawing. Price should include scaffolding. | m3 | 3.68 | 1436 | 5284.48 |
| 2.4.3 | Plastering (three coats) with a 1:2 cement mortar of 25 mm thickness | m2 | 9.35 | 109.42 | 1023.08 |
|  | MC-SC1 |  |  |  |  |
| 1.1 | Earth work |  |  |  |  |
| 1.1.1 | Top soil Clearing and grubbing to a maximum depth of 20 cm as per the design and to the satisfaction of the Engineer. | m2 | 976.17 | 11.18 | 10913.58 |
| 1.1.2 | Soil excavation up to a depth of 1.5 m including cart away | m3 | 1195.99 | 88.20 | 105486.32 |
| 1.1.3 | Excavation in soft rock up to a depth of 1.5 m | m3 | 12.34 | 254.90 | 3145.47 |
| 1.1.4 | Filling with selected material from excavation including compaction. | m3 | 756.09 | 45.36 | 34296.24 |
| 1.1.5 | Fill and compaction with selected material exported from approved boro pits of radius less than 5 km. | m3 | 226.83 | 125.33 | 28428.23 |
| 1.2 | Canal Lining |  |  |  |  |
| 1.2.1 | Masonry Lining and wall construction using 1:3 mortar for canals of any height .The thickness of masonry wall will be as indicated in the drawing. Price should include scaffolding. | m3 | 308.39 | 1436 | 442848.04 |
| 1.2.2 | Plastering (three coats) with a 1:2 cement mortar of 25 mm thickness | m2 | 770.98 | 109.42 | 84360.63 |
| 1.2.3 | Provide C-10 Lean concrete for masonry footing lining of thickness as per the drawing and the direction of the engineer. | m2 | 71.50 | 125 | 8937.50 |
| 1.3 | Division Boxes |  |  |  |  |
| 1.3.1 | Soil excavation up to a depth of 1.50 m | m3 | 3.1 | 77.39 | 239.91 |
| 1.3.2 | Masonry wall construction using 1:3 mortar of any height .The thickness of masonry wall will be as indicated in the drawing. Price should include scaffolding. | m3 | 11.39 | 1436 | 16356.04 |
| 1.3.3 | Plastering (three coats) with a 1:2 cement mortar of 25 mm thickness | m2 | 30.25 | 109.42 | 3309.96 |
| 1.3.4 | Stone pitching | m3 | 10.18 | 536.22 | 5458.72 |
|  | Gate |  |  |  |  |
| 1.3.6 | 6 mm thick sheet metal | m2 | 1.87 | 2000 | 3740.00 |
| 1.3.7 | 60*40*3 mm angle iron | m | 7.00 | 700 | 4900.00 |
| 1.3.8 | Provide C-10 Lean concrete for masonry footing lining of thickness as per the drawing and the direction of the engineer. | m2 | 0.82 | 125 | 102.50 |
| 1.3.9 | ф 10 mm bar for handling | kg | 4.32 | 88 | 379.87 |
| 2.4 | Drop Structures |  |  |  |  |
| 2.4.1 | Soil excavation up to a depth of 1.5 m | m3 | 46.72 | 77.39 | 3615.66 |
| 2.4.2 | Masonry wall construction using 1:3 mortar of any height .The thickness of masonry wall will be as indicated in the drawing. Price should include scaffolding. | m3 | 55.68 | 1436 | 79956.48 |
| 2.4.3 | Plastering (three coats) with a 1:2 cement mortar of 25 mm thickness | m2 | 148.6 | 109.42 | 16259.81 |
| Total infrastructure cost |  |  |  | 12,100,046.31 |  |
| 15 % Vat |  |  |  | 1,815,006.95 |  |
| Total Project Cost With Vat |  |  |  | 13,915,053.26 |  |
| cost per hectare without VAT |  |  |  | 166,897.19 |  |
| cost per hectare with VAT |  |  |  | 191,931.77 |  |

## Summary of Bills

Table 4-12 summary of bill of Quantity

| Mayis-2 Small Scale Irrigation Project |  |  |
| --- | --- | --- |
| SUMMARY OF BILLS |  |  |
| Bill No. | Description | Amount (Birr) |
| 1 | General Items | 1,129,451.44 |
| 2 | Head work | 1,483,002.22 |
| 3 | Infrastructure | 12,100,046.31 |
| Total |  | 14,712,499.97 |
| VAT (15%) |  | 2206874.996 |
| Grand Total with VAT |  | 16,919,374.97 |
| Command Area(ha) |  | 72.45 |
| Per hectare Cost without VAT |  | 203,071.08 |
| Per hectare Cost with VAT |  | 233,531.75 |

# CONCLUSION AND RECOMMENDATION

- The infrastructure of this project area is designed to irrigate about 72.45 ha of land by taking its supply from mayis-2 diversion weir irrigation project. The maximum duty of the command area for 20 hours per day irrigation with overall project efficiency of 55%.  The method of irrigation of the project area is furrow surface irrigation in which the main and field canals are working continuously.

- As the dominant soil type is clay soil, the main canal system is designed to be masonry.

- The reason why the main canal is to be lined up to the end is to avoid the siltation problem, time saving to reach at the tail part, reduce maintenance cost.

- On the main canal the irrigation systems, there are about drop structures. They are designed as far as possible to decrease the amount of cut of depth. The layout is designed as far as possible to avoid cross-structures within them.

- The design of the canal dimensions of the irrigation canal is done by applying the manning’s uniform flow equation. The variable of the hydraulic parameters are calculated using iteration or flow master program.

- The design discharge of the drainage canals are determined using rational formula and Gamble Powell method.

The following recommendations are drown:

- For better performance and long service year of the project regular inspection and maintenance is highly required.

- Farmers training, how to operate and maintain the project structures as a whole and available and water resources has a paramount important.

- The irrigation hours per day and per week should be flexible based on base flow amount of each week or month.

- Close supervision of the construction should be made to modify each Components of irrigation system based on specific site conditions.

# OPERATION AND MAINTENANCE

## General

The main objective of the operation and maintenance aspect of an irrigation scheme is to facilitate the timely delivery of the required irrigation water to farms and to keep the irrigation system in an optimum operating condition.  This section therefore, discusses the main functions of the subject matter under consideration for the scheme.

## Operation of the Head Works

Operation at the diversion weir mainly focuses on the diversion of a controlled flow of river water, timely cleaning of floating debris in front of intake and removal of sediment deposits in front of the weir and intake structures.

## Irrigation System Operation

The operation of the irrigation system depends mainly on the method of water delivery at farm level.  Surface irrigation method is the recommended type of water distribution and application method for Mayis-2 diversion irrigation scheme.

The farmers would organize themselves and form groups in order to handle the water management.  Since flow is low Rotational water distribution would be applied within the group.  The rotational distribution is then to distribute water by turn to the whole scheme according to the timely need of crop water requirement. For better and efficient water management, crop diversification should be avoided within a group.  This would reduce the complexity of water distribution system of the scheme during one irrigation season. At farmers’ level of operation, a constant flow and variable irrigation time is advisable.

The operation of the irrigation system is continuous for 18 hours per day in main, secondary and tertiary canals whereas field canals within a tertiary block are operating in rotational system with each other for irrigation hours proportional to their size.  Since the tertiary canal discharges are within the manageable range and the irrigation canal structures are accordingly designed for simple operation, the farmers can open and close easily whenever they required.

## Maintenance Requirement

The canal system of the project is earthen canal except main canal and secondary canal, which is susceptible to siltation, erosion, growth of weeds and or breaching problems.  Though the canal sections are designed for non- silting and non-scouring conditions, the above mentioned problems are unavoidable and hence periodical and yearly inspection and maintenance of canals and structures are obligatory in order to fulfill the design objective of the project area.

The maintenance tasks are categorized into two types: - routine activities, and repairs. The routine maintenance activities that are carried out periodically include:-

- Regular cleaning of sediments and weeds from canals and drains;

- Inspection and lubrication of gates; and

- Maintenance of cracked lined canals, regulating and control structures.

Repair works include task carried out more frequently and quickly, and include those tasks that are generally unpredictable.  They also include emergency works.  The activities included in this category are:-

- Repairing overtopped or breached canals, drains, and flood protection dykes;

- Repairing jammed gates;

- Filling holes made by wild animals; and

- Reduced free board due to walking over by people and livestock.

Regular inspection of the irrigation facilities should be carried out as part of the maintenance activities.  These tasks could be carried out immediately after the end of the main rains in September and during the rainy season.  This could concentrate on the interceptor drains and the flood protection dykes, the main canal and the field drains.  The inspection of the other works like the tertiary canals, field drains, and the water control and regulating structures could be carried out as part of routine operation activities.

- Beneficiaries of the project need to have operation and maintenance budget, For O&M cost incurring entity area:

- Purchase sing of gate lubricate (grease)

- Replacing and maintenance of Stolen and damaged gates

- Repair Damages on the cross drainage structures.

The expense for O&M should be collected from the beneficiaries. Of course, much of the task is done by the labor and skill of the community. For cost incurring activities beneficiaries have to collect money based on the proportion of the command area they owned.

# REFERENCE

- FAO (1977) guidelines for predicting crop water requirements. No 24, Rome Italy

- Design of small Canal structures , USBR

- Soft copies of hydraulic structure publishing

- IDD manual

- ESRDF manual

- Ethiopian Ministry of Water Resource