Amhara National Regional State

Water Resources Development Bureau

(BOWRD)

Detail Design

Of

Aderkayna Diversion Small-Scale Irrigation Project

Volume IV: Engineering Design Final Report

November, 2012(Revised 2016)

Bahir Dar

Client: Water Resource Development Bureau

(BoWRD)

Address:

P.O.Box: 88
Telephone: 0528-200853/855
Fax: 251-08-20-65-68/204676/202040

Consultant: Amhara Design & Supervision Works Enterprise

(ADSWE)

Address:

P.O.Box: 1921
Telephone: +251-582-181023/ 180638/181201/181254
Fax: (058) 2180550/ (058) 2180560
E-mail: amhara design@yahoo.com

Bahir Dar, Ethiopia

# FEASIBILITY STUDY & DETAIL DESIGN REPORT STRUCTURE

- Volume I: Watershed Management

- Volume II: Engineering Geology

- Volume III: Irrigation Agronomy

- Volume IV: Engineering Design

- Volume V: Socio Economy

- Volume VI: Environmental Impact Assessment

Table of Contents								 Page FEASIBILITY STUDY & DETAIL DESIGN REPORT STRUCTURE	i

LIST OF TABLES	vi

LIST OF FIGURES	vi

SAILENT FEATURE	vii

1	INTRODUCTION	1

1.1	Background	1

1.1.1	Description of the Project Area	1

1.1.1.1	Location	1

1.1.1.2	Accessibility	2

1.1.1.3	Previous Irrigation Practices	2

1.2	Objectives of the study	3

1.2.1	Major Objective	3

1.2.2	Specific Objectives	3

1.3	Scope of the study	4

1.4	Methodology	4

SECTION-I:  HYDROLOGY	6

2	HYDROLOGY	7

2.1	Watershed characteristics	7

2.2	Hydro-metrological data availability	8

2.2.1	Climate	8

2.2.2	Rainfall Data	8

2.2.3	River flow data	9

2.2.4	Upstream & Downstream utilization	9

2.3	Design flood analysis	9

2.3.1	Design Rainfall computation	9

2.3.2	Outlier Test	9

2.3.2.1	Check for variance	11

2.3.2.2	D-Index test	12

2.3.3	Peak Discharge Determination	13

2.3.3.1	General	13

2.3.3.2	Peak flood analysis by SCS unit hydrograph method	13

2.3.3.3	Time of concentration (Tc)	13

2.3.3.4	Curve number (CN)	14

2.3.3.5	Area Rainfall	14

2.3.3.6	Run off Analysis	15

2.3.4	Tail Water Depth Computation	19

2.3.4.1	Average river bed slope	22

2.3.4.2	Manning’s Roughness coefficient	22

2.3.4.3	Discharge of the river	22

SECTION-II: HEADWORK DESIGN	24

3	HEADWORK STRUCTURES DESIGN	1

3.1	Headwork site selection	1

3.1.1	River Geomorphology	1

3.1.1.1	River Bed condition	1

3.1.1.2	River Bank condition	2

3.1.2	Sources of construction materials	3

3.1.2.1	Rock for Masonry and Crushed Coarse Aggregate	3

3.1.2.2	Fine Aggregates	4

3.1.2.3	Water	4

3.2	Headwork type selection	4

3.3	Hydraulic design of headwork structure	4

3.3.1	Weir Height Determination	4

3.3.2	Base flow of the River	5

3.3.3	Weir Dimensions	5

3.3.3.1	Flow over the Weir crest	5

a)	Crest Length	5

b)	Discharge over the weir section	6

3.3.3.2	Top and bottom width	6

3.3.4	U/S and D/S HFL Calculation & Determination	6

3.3.5	Hydraulic Jump Calculation	7

3.3.6	Impervious floor &cut-off	8

3.4	Stability Analysis of weir	8

3.4.1	Wall height fixation of Divide Wall	11

3.4.2	Under sluice	12

3.4.3	Canal outlet level	13

3.4.4	Outlet capacity	13

3.4.5	Outlet size	13

3.4.6	Breast Wall and Operation Slab	14

3.4.7	Retaining Walls	14

SECTION-III: IRRIGATION AND DRAINAGE SYSTEMS INFRASTRUCTURE	17

4	IRRIGATION AND DRAINAGE SYSTEMS DESIGN	18

4.1	Irrigable area description	18

4.1.1	Topography	18

4.1.2	Climate	18

4.1.3	Soil characteristics	18

4.1.4	Existing Irrigation Practices in the Project Area	19

4.2	Irrigation water requirement	19

4.2.1	Crop Water Requirement (CWR)	19

4.2.2	Irrigation efficiency (Ep)	19

4.2.3	Irrigation duty	20

4.2.4	Irrigation methods	21

4.3	Irrigation and drainage system layout	21

4.3.1	Conveyance System	22

4.4	Design of the canal system	22

4.4.1	Main Canal	22

4.4.2	Secondary Canals	24

4.4.3	Tertiary canals	25

4.4.4	Field Canals	27

4.5	Drainage Canals	28

4.5.1	Catch drainage canals	28

4.5.2	Tertiary canal drainage canals	29

4.6	Canal structures design	29

4.6.1	Design of a typical flume	29

4.6.2	Design of Division box	31

4.6.3	Design of Drop structure	34

4.6.4	Road crossing structure	34

4.6.5	Design of a typical Super passage	34

4.7	Irrigation Infrastructure Bill of Quantities And Cost Estimate	35

5	CONCLUSION AND RECOMMENDATION	39

REFERENCE	40

LIST OF TABLES

Table 2-1:  Outlier test analysis	10

Table 2-2: Test for goodness to fit using D-index	12

Table 2-3: Determination of Time of Concentration	14

Table 2-4: Design Rainfall arrangement	17

Table 2-5: Runoff analysis	18

Table 2-6: Hydrograph coordinates	18

Table 2-7: Weir Site River Cross section Coordinate Data	20

Table 2-8:  Stage discharge analysis	21

Table 2-9:  River discharge computation at different stages of flow	23

Table 3-1: Weir height determination	5

Table 3-2: Typical design of masonry retaining wall	15

Table 4-1: Hydraulic Parameters of main canal	23

Table 4-2: Hydraulic Parameters of secondary canals	24

Table 4-3: Hydraulic Parameters of tertiary canals	26

Table 4-4: Hydraulic Parameters of Catch drains	29

Table 4-5:  Hydraulic Parameters of Flume	30

Table 4-6: Hydraulic parameters of Division Boxes	33

Table 4-7:  Hydraulic Parameters of Super passage	34

Table 4-8:  Canal elevation at each section	34

LIST OF FIGURES

Figure 1-1:  Location map of the project area	2

Figure 2-1: Drainage map of Aderkayna Irrigation Project watershed	8

Figure 2-2: Complex Hydrograph	19

Figure 2-3: Weir Site River Cross Section	20

Figure 2-4: Rating Curve	21

Figure 2-5: River Profile	22

Figure 3-1: Picture showing the nature of the weir axis bed rock	2

Figure 3-2: Tail water depth and jump height depth comparison	8

Figure 4-1: Typical section of lined canal	23

Figure 4-2: Typical Cross Section of secondary canals	25

Figure 4-3: Typical Cross Section of tertiary canals	26

Figure 4-4: Typical Field Canal X-section	28

Figure 4-5: Typical Cross Section of Catch drain canals	28

Figure 4-6: Typical Division Box plan	31

# SAILENT FEATURE

- Project name: Aderkayna Diversion  Irrigation Project

- Name of the stream: Aderkayna River

- Location of the  site using

- Easting:379446.63 (UTM)

- North:1396513.46(UTM)

- Zone: North Gonder Zone

- Wereda:  West Belesa

- Average Altitude: 1724 m amsl

- Hydrology

- Design rainfall: 88.63mm

- Catchment area: 171.75km2

- Longest flow path length: 23.35Km

- Design flood: 229m3/sec

- Design base flow: 400.lit/se.

- Diversion Weir

- Weir type: broad crested

- Height: 2.18m

- Gross crest length: 40m

- Weir crest level: 1726.20m a.m.s.l

- U/S HFL: 1727.95 m a.m.s.l

- U/S TEL: 1728.4m .a.s.l

- D/s TEL: 1727.52 m a.m.s.l.

- D/s HFL: 1727.02

- m a.m.s.l

- Afflux: 0.93m

- Under sluice

- Sill level: 1724.02m.a.s.l

- Dimension: 0.8*1m2

- Discharge amount: 2.77m3/sec

- Outlet

- Sill level: 1726.1m a.m.s.l

- Opening dimension: 0.6*0.8m2

- Discharge capacity: 320 lit/sec.

Irrigation and drainage systems Infrastructure

- Command area size: 240 ha

- Type of soil of the command area is dominantly clay loam

- Design discharge of the main canal = 320 l/sec

- Irrigation system layout consists of  lined main canal and  4 tertiary canals

- Main irrigation structures designed are;

- Gully crossing structures

Project cost

# INTRODUCTION

## Background

In Ethiopia, under the prevalent rain-fed agricultural production system, the progressive degradation of the natural resource base, especially in highly vulnerable areas of the highlands coupled with climate variability have aggravated the incidence of poverty and food insecurity. The major source of growth for Ethiopia is still conceived to be the agriculture sector. Hence, this sector has to be insulated from drought shocks through enhanced utilization of the water resource potential of the country, (through development of small-scale irrigation, water harvesting, and on-farm diversification) coupled with strengthened linkages between agriculture and industry (agro-industry), thereby creating a demand for agricultural output. In line with the above, efforts have been made by the government and NOG’s to improve the situation in the country in areas of domestic water supply provision, irrigation, watershed management, etc. Amhara Water Resources Development Bureau is playing its role in the development of small scale irrigation projects in the region. Accordingly, as part of the water sector development program, the office has initiated the study and design of a small scale irrigation scheme on Aderkayna  River at Tebtebita  Kebele  and signed an agreement with Amhara Design & Supervision Works Enterprise (ADSWE) for the study and design of the project.

### Description of the Project Area

#### Location

Aderkayna irrigation project is located at Tebitebta Keble, in West Belesa Wereda of West Gonder Zone in the Amhara National Regional State.  The weir site is located at 379446.63 E, and 1396513.46N (UTM) at an altitude of 1724m above sea level.

Figure 1-1:  Location map of the project area

#### Accessibility

The project site away from the Zonal town (Addis Zemen) is about 130km, and from Wereda town it is about 23km.

#### Previous Irrigation Practices

There is one diversion weir on the up stream of this river but at this time it is not functioning properly .other traditional practice is well adapted at the downstream of the river. The hydrology and Hydrogeology study and respondent farmers indicated, the river has capacity of recharging as it stretches down from the source area of the river. As a result there will not be a marked reduction or fluctuation of water flows both for the already existing and the newly proposed irrigation schemes. The traditional irrigation practices are under taken by individual farmers that use the river flow to the extreme left side is with hardship. So, the farmers in the project area are very much interested to upgrading the traditional scheme to modern scheme.

## Objectives of the study

### Major Objective

The general objective of Aderkayna diversion irrigation study is to prepare the required design for utilizing the available water and land resources in the project area. The design focused on utilizing these resources for dry period and supplementing the rain fed agriculture during moisture stress in wet seasons. Particularly in this study it focuses the preparation headwork and appurtenant structures design for the project. Specifically, the project is targeted for the following points.

- To make sustainable the rain-fed crop production and make extra production in the dry season possible for 240ha of land through irrigation. There is a general consensus that irrigation investments will achieve broader food security and poverty reduction impacts and if efforts are also geared towards up-grading existing traditional farming practices with support to enhance access to input supply, output marketing and extension to facilitate access to information and innovation. This objective is to be realized by constructing diversion structures across the Aderkayna River and diverting the river flow.

- The large group of rural poor, who are chronically food-insecure and have minimal market opportunities and minimal alternative employment opportunities, can get employment opportunities during project implementation.

### Specific Objectives

Other benefits that can be expected to appear with the launching of the project are:

- Efficiency of water use improvement;

- Improved local nutrition/food security gains;

- Improved management of scarce natural resources (land and water);

- Resilience against drought;

- Rationale for erosion control and watershed management;

- Rationale for the intensification and modernization of small-holder agriculture and rural lifestyles.

The engineering study and design enables the realization of the project by the provision of engineering structures that will allow the appropriate abstraction of the river water for delivery in to the identified irrigation fields of the study area. Hence, this engineering design is specifically targeted to:

- Analyze hydrologic requirements of the project and engineering structures;

- The formulation of sound and stable structure,  with necessary provisions that allow safe, easy and low-maintenance operation in the service life of the project;

- Develop working drawings;

- Estimation of construction costs.

## Scope of the study

- The irrigation design shall ensure reliability, equity and flexibility of water delivery to farmers. It will aim at reducing conflicts among water users and will lead to lower operation and maintenance costs.

- Updating the existing, if available, computation of the actual vapor-transpiration, crop water requirement, irrigation demand/duty using the existing and recent agronomic, climatologic and soil data using more appropriate methodologies.

- Establish design criteria for irrigations structures to be approved by the client and to be used in the final design stage,

- Design proper irrigation system compatible with local conditions and management capabilities,

- Establish flood protection measures for the command area and canal structures and design the respective drainage system accordingly,

- Planning  and layout of the irrigation system, which include irrigation canals, drainage channels, inspection roads and alignments, canal spacing, canal length, location of structures, and water profiles along canal and drains at specified reaches, which is most economical easily manageable and aligned with topographic feature and geological investigation.

- Determination and estimation of water application conveyance and other losses and irrigation efficiencies and consideration of those parameters in design steps.

- Check and test hydraulic and structural designs of main canal considering total demand and the required capacity and the base flow availability,

- Prepare general plans and drawings for all irrigation infrastructure and irrigation systems designs,

## Methodology

In the study and design procedure, Designers used the following steps.

- Site identification:

  - 50,000 scale top map and GIS information

  - Local farmers interview and discussion

  - Wereda and Zone Agriculture section expertise

  - Previous studies

  - On foot travel along the river channel and farm areas.

- Topographic survey:

  - Surveying the head work area and  Command area with sufficient radius, using Total station

- Flow estimation

  - Physical observation on flood mark indications and local information about high flood and critical flow condition of the river

  - Analyzing the recorded river flow data and use water shed inputs for further analysis.

  - Base flow estimated during the field visit by floating method.

- Irrigable area identification:

  - Using local information

  - 50,000 top map, and GIS information, GPS to see elevation

The design report is organized in three sections. In section I the Hydrology study is presented and in sections II and III the Headwork and Irrigation and Drainage Systems designs are discussed respectively. In section III, planning and design of the irrigation system after diverting the water using the weir will be dealt. The following are major areas of concern in this part.

- Study and design of the irrigation method to be adopted,

- Study and design of the irrigation system layout and associated structures,

- Design of the different conveyance canals,

- Planning and design of the different irrigation and drainage structures,

- Preparation of the longitudinal profiles of the different irrigation and drainage canals.

# SECTION-I:  HYDROLOGY

# HYDROLOGY

## Watershed characteristics

The Watershed has marked topographic variation. All types of slopes are present. The dominant slope class is steep (30-50%) which covers 31.75% of the total area followed Sloping (15-30%) which is 27.34%. Moderately steep (8-15%) which covers 16.44%, or moderately flat (3-8%) is about 15.42% and absolutely flat (0-3%), accounts 3.92%, respectively. Table 1 shows the slope classes and proportion of the watershed.

Certain physical properties of watersheds significantly affect the characteristics of the runoff and sediment yield and are of great interest in hydrologic analyses. The rate and volume of runoff, and sediment yield from the watershed have much to do with shape, size, slope and other parameters of the landscape. These suggest that there should be some important relations between basin form and hydrologic performance. If the basin and hydrologic characteristics are to be related, the basin form must also be represented by quantitative descriptors. These parameters can be measured from maps.

The watershed characteristics are analyzed and presented in Table 2 of the Watershed Feasibility Study Report of the same project. In summary:

Catchment Area  = 171.75 km2

- Stream Length= 23.35Km

- CN(III) = 86.61

(Extracted from the Watershed Study Report of the same project)

At the selected reference point, the area of Aderkayna catchment is 171.75km2 and consists of a network of tributaries as shown in Figure 2 below.

Aderkayna River at the headwork site is characterized by well-defined channel system and considerable flows. It looks that the gradient of the river is getting low and hence there exists significant deposition of sediment mainly deposit.

Figure 2-1: Drainage map of Aderkayna Irrigation Project watershed

## Hydro-metrological data availability

### Climate

Small scale irrigation project designers and planners are faced with lack of good data on the hydrology of the stream/river system that will be their water source and on local weather and climate conditions. Stream gauging stations are virtually non-existent in remote rural areas of Ethiopia; meteorological stations are almost rare near to the project area. Likewise, data for the hydro-meteorological analysis is taken from Gonder metrological station and similar areas.

As per the data of the station, March – April are identified as high temperature periods whereas December–January are low temperature periods. The mean annual rainfall amount is more is 51.16mm (1953 - 2002data) and most of it occurs from June to August.

### Rainfall Data

In order to compute the design flood for the Diversion structure, the daily maximum rainfall is collected from Gonder metrological station with a record of 42 years.

### River flow data

The flow which is measured on driest month (March, 2012 is 219.69l/s. Since this base flow is measured and (20 % of this amount has to be released for downstream ecosystem)during the dry months of the year, this figure is not adopted for design because the irrigation process supplementary irrigation we adopt 400 l/s for design because it was measured after one month of summer season(at Nov,2013).

### Upstream & Downstream utilization

At Aderkayna river there is one structure built for irrigation purpose at upstream and there is traditional irrigation practice at left downstream side .Downstream of the proposed site, appreciable need for water is anticipated for locals and cattle provisions. Therefore, at least 20% of the minimum flow has to be released for downstream requirements.

For the sake of planning and design, the outlet for the diversion is designed for a discharge of 320 l/s for this project and the project is to be developed for 240 ha of land, which is most of the time achievable as the flow for most of the time is significant to support this size of command area by supporting supplementary condition.

## Design flood analysis

For the design and analysis of structures to be constructed on the river, estimation of flood magnitude is an important task. This can be done using different techniques depending on the data available. For this particular case, there are no river flow data and hence the flood estimation is done using the rainfall data and applying SCS Curve Method.

### Design Rainfall computation

Based on the data of 24hr peak rainfall given in Table 2.1 the design rainfall, Rf is computed using Gamble’s Extreme Value Method.

### Outlier Test

Higher Limit, YH=Ymean+Kn*Sy,      Kn = 2.7for 42 Years of data.

Lower Limit, YH=Ymean-Kn*Sy,     Kn = 2.7 for 42Years of data.

Table 2-1:

Higher Limit, YH = 1.993

Lower Limit, YL = 1.397

Therefore,

Upper limit of rainfall = 101.9993 = 98.57 mm

Lower Limit of rainfall = 101..397= 24.96mm

Conclusion: The rainfall values are within the limits.

Outlier test analysis

| Sr. Nr | Annual year | Heaviest rainfall (mm/day) | Descending order | Rank | Log (Y) |
| --- | --- | --- | --- | --- | --- |
| 1 | 1953 | 62.3 | 82.0 | 1 | 1.913813852 |
| 2 | 1955 | 47.8 | 75.8 | 2 | 1.879669206 |
| 3 | 1956 | 31.0 | 72.3 | 3 | 1.859138297 |
| 4 | 1957 | 47.0 | 69.5 | 4 | 1.841984805 |
| 5 | 1958 | 46.5 | 65.4 | 5 | 1.815577748 |
| 6 | 1959 | 75.8 | 65.4 | 6 | 1.815577748 |
| 7 | 1964 | 59.2 | 65.0 | 7 | 1.812913357 |
| 8 | 1965 | 48.2 | 63.5 | 8 | 1.802773725 |
| 9 | 1966 | 42.1 | 63.0 | 9 | 1.799340549 |
| 10 | 1967 | 82.0 | 62.3 | 10 | 1.794488047 |
| 11 | 1969 | 51.9 | 60.2 | 11 | 1.779596491 |
| 12 | 1970 | 57.1 | 59.3 | 12 | 1.773054693 |
| 13 | 1971 | 36.0 | 59.2 | 13 | 1.772321707 |
| 14 | 1972 | 59.3 | 58.3 | 14 | 1.765668555 |
| 15 | 1973 | 53.1 | 57.1 | 15 | 1.756636108 |
| 16 | 1974 | 69.5 | 54.3 | 16 | 1.73479983 |
| 17 | 1975 | 65.0 | 54.3 | 17 | 1.73479983 |
| 18 | 1976 | 65.4 | 53.1 | 18 | 1.725094521 |
| 19 | 1977 | 63.5 | 51.9 | 19 | 1.715167358 |
| 20 | 1978 | 50.3 | 51.3 | 20 | 1.710117365 |
| 21 | 1979 | 44.0 | 50.3 | 21 | 1.701567985 |
| 22 | 1980 | 54.3 | 48.2 | 22 | 1.683047038 |
| 23 | 1981 | 30.0 | 47.9 | 23 | 1.680335513 |
| 24 | 1982 | 34.3 | 47.8 | 24 | 1.679427897 |
| 25 | 1983 | 65.4 | 47.8 | 25 | 1.679427897 |
| 26 | 1984 | 43.9 | 47.0 | 26 | 1.672097858 |
| 27 | 1985 | 47.8 | 46.5 | 27 | 1.667452953 |
| 28 | 1986 | 37.0 | 45.5 | 28 | 1.658011397 |
| 29 | 1987 | 41.3 | 44.0 | 29 | 1.643452676 |
| 30 | 1988 | 51.3 | 43.9 | 30 | 1.64246452 |
| 31 | 1989 | 47.9 | 42.1 | 31 | 1.624282096 |
| 32 | 1990 | 63.0 | 41.3 | 32 | 1.615950052 |
| 33 | 1991 | 36.8 | 40.7 | 33 | 1.609594409 |
| 34 | 1993 | 54.3 | 40.1 | 34 | 1.603144373 |
| 35 | 1994 | 40.1 | 37.0 | 35 | 1.568201724 |
| 36 | 1995 | 30.9 | 36.8 | 36 | 1.565847819 |
| 37 | 1996 | 45.5 | 36.6 | 37 | 1.563481085 |
| 38 | 1997 | 72.3 | 36.0 | 38 | 1.556302501 |
| 39 | 1998 | 40.7 | 34.3 | 39 | 1.53529412 |
| 40 | 1999 | 36.6 | 31.0 | 40 | 1.491361694 |
| 41 | 2000 | 58.3 | 30.9 | 41 | 1.489958479 |
| 42 | 2002 | 60.2 | 30.0 | 42 | 1.477121255 |
| Sum |  |  | 2148.900 |  | 71.21 |
| Mean |  |  | 51.164 |  | 1.70 |
| Standard Deviation(σn-1) |  |  | 12.787 |  | 0.11 |
| Skewness coefficient(Cs) |  |  |  |  | -0.17 |
| No of data(N) = 42 |  | 42 |  |  |  |

#### Check for variance

After checking the outliers, the data should be checked for variability. For variability the formula used is

α=δ∩-1N*Mean*100%

Where, δn-1 = Standard deviation =12.787

N = Nr of recorded data =42

Mean = 51.16

α = Standard error

α=12.78742*51.16*100%=3.86<10%  Acceptable

Therefore the data shows no variability.

#### D-Index test

After checking the consistency of the data for higher and lower outlier, the 42 years data is obtained as representative for the analysis using D-index. The D-Index test is believed to be the better goodness to fitness in many literatures. Hence in this study it was used to determine the best statistical distribution to estimate the peak rainfall. The D-index for the comparison of the fit of various distributions is summarized as follows.

D-Index=(1Xm)*i=16Abs(Xi-Xi')

Where Xi and Xi’ are the ith highest observed and computed values for the distribution respectively.

Table 2-2: Test for goodness to fit using D-index

| Rank | XI | Normal | Log person Type III | Log Normal | Person Type III | Gumbel EVI |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | XI -'XI' | XI -'XI' | XI -'XI' | XI -'XI' | XI -'XI' |
| 1 | 82.0 | 13.15 | 11.963 | 11.484 | 13.491 | 0.000 |
| 2 | 75.8 | 12.27 | 12.354 | 12.362 | 12.259 | 0.000 |
| 3 | 72.3 | 12.51 | 13.208 | 13.421 | 12.333 | 0.000 |
| 4 | 69.5 | 12.83 | 13.852 | 14.163 | 12.552 | 0.000 |
| 5 | 65.4 | 11.55 | 12.727 | 13.078 | 11.214 | 0.000 |
| 6 | 65.4 | 14.24 | 15.448 | 15.800 | 13.880 | 0.000 |
| Sum |  | 76.55 | 79.552 | 80.306 | 75.730 | 0.000 |
| Sum/Mean |  | 1.496 | 1.555 | 1.570 | 1.480 | 0.000 |
| Point Rain fall |  | 77.43 | 81.7 | 81.7 | 76.276 | 84.31 |

Comparing the value of D-index for the above calculation, we get the least value for Log Pearson type III; this indicates that Log Pearson type III distribution is the best fit probability distribution to the data. But the rain fall obtained by Gamble distribution is greater than that of log Pearson type III distribution so to be more safe take Gambles distribution, in addition Gambles is the most common used method of distribution. Hence peak daily point rainfall of 50 years return period is 88.63mm.

The design rainfall using gamble Method is given as

Where Rf  = Design rainfall

Rmean = average of all values of annual heaviest fall = 51.16mm

σn-1 =  standard deviation of the series = 12.787 mm

, T= Return period = 50 years

Yn, Sn = constant found from Gamble’s extreme value distribution table for N= 42Years

- Yn = 51.16 and Sn = 3.86

k=2.93

rf=51.16+2.59*12.787=88.63

- Point Design Rainfall = 88.63 mm

The design rainfall at points for 50 years return period is 88.63mm and the areal design rainfall is calculated in the following section.

### Peak Discharge Determination

#### General

The River is not gauged river. The design flood is calculated by using SCS unit hydrograph method. Thus, it is preferred to base the flood analysis on rainfall data, which are better both in quantity and quality of data. In the hydrologic analysis for drainage structures, it must be recognized that there are many variable factors that affect floods. Some of the factors that need be recognized and considered on an individual site by site basis are; rainfall amount and storm distribution; catchment area, shape and orientation; ground cover; type of soil; slopes of terrain and stream(S); antecedent moisture condition; Storage potential (over bank, ponds, wetlands, reservoirs, channel, etc.)

#### Peak flood analysis by SCS unit hydrograph method

Design flood is calculated SCS (The United States Soil Conservation Service). This method is widely adopted and more reliable method for flood estimation. The approach considers, watershed parameters, like Area, Curve number, and time of concentration.

#### Time of concentration (Tc)

Time of concentration has been calculated by taking the river profile of the longest streamline and dividing it in to different elevation. Kirpich formula is adopted for computation

Table 2-3: Determination of Time of Concentration

| Partial Distance (km) | Cumulative distance (km) | Elevation (m) | Elevation diff. (m) | TC (hr) |
| --- | --- | --- | --- | --- |
| 0 | 0 | 2880 | 0 |  |
| 2.16 | 2.16 | 2720 | 160 | 0.34 |
| 3.65 | 5.81 | 2220 | 500 | 0.40 |
| 1.44 | 7.25 | 2110 | 110 | 0.25 |
| 14.37 | 21.62 | 1740 | 370 | 2.19 |
| 1.73 | 23.35 | 1716 | 24 | 0.55 |
| 23.35 |  |  |  |  |
|  |  |  | Tc | 3.72 |

The formula is,

Tc=0.948L13H10.385+L23H20.385+…+Ln3Hn0.385

- Tc = 3.72	Since Tc >3hr., duration of excess rainfall difference, D = 1 hr.

- Time to peak,

Tp=D2+0.6*Tc  = 2.732hr

- Base time,

Tb=2.67*Tp  = 7.29

- Recession time,

Tr=1.67*Tp  = 6.25hr.

#### Curve number (CN)

Curve number (CN) is achieved based on USSCS method by watershed characterization in terms of land cover, treatment, hydrologic condition and soil group. From the watershed analysis curve number at condition II =88.71 since peak rainfall is found at an antecedent moisture condition III state, this value has to be changed to antecedent moisture condition III.

- Conversion factor = 1.13

- CN Condition (III) = (Factor from Table x CN condition II) =76.99*1.13 =86.61.

#### Area Rainfall

As the area of the catchment gets larger, coincidence of all hydrological incidences becomes less and less. This can be optimized by changing the calculated point rainfall to aerial rainfall. The conversion factor is taken from standard table that relate directly with the size of watershed area and type of the gauging station. (IDD manual)

For the case of Zeya irrigation project,

- Total watershed area = 171.75 Km2

- Type of gauging station = Daily rainfall (24 hr.)

- Aerial Rainfall = (Point Rainfall) x (Conversion factor)

#### Run off Analysis

Input data:

Design Point Rainfall = 88.63mm

Curve number at antecedent moisture condition III = 95.72

Catchment Area, A = 171.35 Km2

Tc = 3.72hr, D = 1., Tp = 2.73hr; Tb = 7.29hr; Tr = 6.25 hr.

Direct run-off,          Q=(I-0.2*S)2(I+0.8*S)

Where, I = Rearranged cumulative run-off depth (mm

S = Maximum run off potential difference,

S=25400CN-254

Peak run-off for incremental;

Qp=0.21*(A*Q)Tp

Where,   A = Catchment area = 171.75Km2

Tp = Time to peak (hr)

Q = Incremental run-off (mm)

Table 2-4: Design Rainfall arrangement

| Time (hr) | Design point Rainfall (mm) | Rainfall Profile (%) | Rainfall Profile (mm) | Area to point rain fall ratio % | Area rainfall (mm) | Incremental Rainfall (mm) | Ascending Order | Ascending Order | Rearranged order | Rearranged Rainfall(mm) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  |  |  | Incremental | Accumulative |
| 0-1 | 88.63 | 46 | 40.77 | 67.88 | 27.68 | 27.68 | 27.68 | 1 | 6 | 4.07 | 4.07 |
| 1-2 | 88.63 | 58 | 51.41 | 73.44 | 37.75 | 10.08 | 10.08 | 2 | 4 | 4.77 | 8.84 |
| 2-3 | 88.63 | 67 | 59.38 | 76.72 | 45.56 | 7.81 | 7.81 | 3 | 3 | 7.81 | 16.65 |
| 3-4 | 88.63 | 71 | 62.93 | 78.86 | 49.63 | 4.07 | 4.77 | 4 | 1 | 27.68 | 44.33 |
| 4-5 | 88.63 | 75 | 66.47 | 80.86 | 53.75 | 4.13 | 4.13 | 5 | 2 | 10.08 | 54.41 |
| 5-6 | 88.63 | 79 | 70.02 | 83.58 | 58.52 | 4.77 | 4.07 | 6 | 5 | 4.13 | 58.54 |
| 6.0-12 | 88.63 | 87 | 77.11 | 85.72 | 66.10 | 61.62 |  |  |  | 61.62 | 120.15 |
| 12.0-24.0 | 88.63 | 100 | 88.63 | 86.86 | 76.99 | 72.51 |  |  |  | 72.51 | 192.66 |

Table 2-5: Runoff analysis

| Time | Incremental runoff | Qp for incremental runoff | Incremental Hydrograph |  |  |
| --- | --- | --- | --- | --- | --- |
|  | (mm) | (m3/s) |  |  |  |
|  | 1 | 2 | Begin time | peak time(Tp) | end time(Tb) |
| 0-.5 | 0.35 | 1.37 | 0 | 1.74 | 4.64 |
| .5-1 | 1.76 | 5.32 | 0.5 | 2.24 | 5.14 |
| 1-1.5 | 4.35 | 10.73 | 1 | 2.74 | 5.64 |
| 1.5-2 | 19.33 | 40.33 | 1.5 | 3.24 | 6.14 |
| 2.0-2.5 | 7.51 | 13.57 | 2 | 3.74 | 6.64 |
| 2.5.3.0 | 3.48 | 5.56 | 2.5 | 4.24 | 7.14 |

Table 2-6: Hydrograph coordinates

| Time | H1 | H2 | H3 | H4 | H5 | H6 | SUM | Remark |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0.00 | 0.00 |  |  |  |  |  | 0 |  |
| 1.00 | 0.29 | 0.00 |  |  |  |  | 0 |  |
| 2.00 | 0.58 | 7.64 | 0.00 |  |  |  | 8 |  |
| 2.73 | 0.80 | 13.24 | 11.49 | 0.00 |  |  | 26 |  |
| 3.00 | 0.55 | 15.29 | 15.70 | 0.00 |  |  | 32 |  |
| 3.73 | 0.46 | 20.88 | 27.16 | 42.16 | 0.00 |  | 91 |  |
| 4.00 | 0.42 | 14.42 | 31.39 | 57.76 | 0.00 |  | 104 |  |
| 4.73 | 0.33 | 11.97 | 42.88 | 99.92 | 13.88 | 0.00 | 169 |  |
| 5.00 | 0.29 | 11.06 | 29.61 | 115.52 | 19.02 | 0.00 | 176 |  |
| 5.73 | 0.20 | 8.61 | 24.57 | 157.80 | 32.90 | 4.99 | 229 | PEAK |
| 6.73 | 0.07 | 5.24 | 17.67 | 90.43 | 51.95 | 11.83 | 177 |  |
| 7.29 | 0.00 | 3.36 | 13.81 | 76.20 | 33.45 | 15.66 | 142 |  |
| 7.73 |  | 1.88 | 10.77 | 65.03 | 29.77 | 18.69 | 126 |  |
| 8.29 |  | 0.00 | 6.90 | 50.80 | 25.09 | 12.03 | 95 |  |
| 9.29 |  |  | 0.00 | 25.40 | 16.72 | 9.02 | 51 |  |
| 10.29 |  |  |  | 0.00 | 8.36 | 6.02 | 14 |  |
| 11.29 |  |  |  |  | 0.00 | 3.01 | 3 |  |
| 12.29 |  |  |  |  |  | 0.00 | 0 |  |

Figure 2-2: Complex Hydrograph

From the analysis, the 50 year return period design run off is 429m 3/s

### Tail Water Depth Computation

Tail water depth of the river is equal to the flood depth and amount at the proposed weir site before construction of the weir. It is used to crosscheck peak flood estimated by the SCS unit hydrograph method with flood mark method and to see the flood feature after the hydraulic jump. During field visit, the flood mark of the river at the proposed diversion site was marked based on dwellers information and physical indicative marks. The river cross-section was surveyed.

Table 2-7: Weir Site River Cross section Coordinate Data

| ELEVATION | NORTH | EAST | ELEVATION | DES | pd | chainage |
| --- | --- | --- | --- | --- | --- | --- |
| 1729.82 | 1396538 | 379456 | 1729.82 | BM1 | 0 | 0 |
| 1728.68 | 1396530.788 | 379453.25 | 1728.68 | RL | 7.71851307 | 7.718513069 |
| 1727.78 | 1396527.038 | 379451.819 | 1727.78 | RLBD | 4.01375896 | 11.73227203 |
| 1724.02 | 1396513.422 | 379446.626 | 1724.02 | RC | 14.5726698 | 26.30494183 |
| 1725.48 | 1396496.218 | 379439.977 | 1725.48 | RRBD | 18.444154 | 44.74909583 |
| 1726.5 | 1396493.786 | 379439.136 | 1726.5 | RR | 2.57330624 | 47.32240207 |
| 1727.5 | 1396468.1 | 379429.235 | 1727.5 | BM2 | 27.5132459 | 74.83564799 |

Figure 2-3: Weir Site River Cross Section

Table 2-8:  Stage discharge analysis

| Sr. Nr | Elevation | water depth | Area of the cross section | Wetted Perimeter (m) | Hydraulics Radius (m) | Velocity (m/sec) | Discharge (m3/sec) | Water depth | di |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1724.02 | 0.00 | 0 | 0 | 0 | 0 | 0 | 0.00 | 0 |
| 2 | 1725.00 | 0.98 | 6.76 | 12.341 | 0.548 | 2.2224 | 15.024 | 0.98 | 15.02 |
| 3 | 1725.48 | 1.46 | 13.89 | 16.804 | 0.827 | 2.9239 | 40.613 | 1.46 | 40.61 |
| 4 | 1726.00 | 1.98 | 23.59 | 20.407 | 1.156 | 3.6565 | 86.257 | 1.98 | 86.26 |
| 5 | 1726.50 | 2.48 | 34.53 | 23.9 | 1.445 | 4.2426 | 146.5 | 2.48 | 146.5 |
| 6 | 1727.00 | 2.98 | 46.69 | 26.804 | 1.742 | 4.806 | 224.39 | 2.98 | 224.4 |
| 7 | 1727.50 | 3.48 | 59.92 | 29.759 | 2.013 | 5.2934 | 317.18 | 3.48 | 317.2 |
| 8 | 1728.00 | 3.98 | 74.29 | 31.895 | 2.329 | 5.8332 | 433.35 | 3.98 | 433.3 |
| 9 | 1728.68 | 4.66 | 95.01 | 33.6166 | 2.826 | 6.636 | 630.49 | 4.66 | 630.49 |
| 10 | 1729.00 | 4.98 | 105.05 | 34.4391 | 3.05 | 6.9822 | 733.48 | 4.98 | 733.485 |

Figure 2-4: Rating Curve

From the above stage discharge table and curve the maximum flood level corresponding to the computed design peak discharge is 1727.3 (3.0m from the river bed) and it is considered as the d/s high flood level i.e. expected at the weir axis before construction of the weir.

D/S HFL = 1727 masl.

#### Average river bed slope

Average river bed slope of River is estimated by best fit line method. The water level of the river is taken at different points along the river channel around the head work site. Surveying work done for more than 100m length. And then, average water surface slope is considered as the river bed slope.

Figure 2-5: River Profile

#### Manning’s Roughness coefficient

The Manning’s roughness coefficient is taken from standard table based on the river nature. The river at the headwork site has got braded feature and curving nature. Manning’s roughness coefficient (n = 0.035) is adopted.

#### Discharge of the river

Input data:

- Manning's roughness coefficient, n = 0.035

- Average river bed slope, S = 0.0135

- ,  	Where, R = Hydraulic radius = (Area/Perimeter)

- Q=V*A

Table 2-9:  River discharge computation at different stages of flow

| North | East | Elvation | Code | Pd | Chainage | Elevation |
| --- | --- | --- | --- | --- | --- | --- |
| 1396540 | 379332 | 1726.68 | RC | 0 | 0 | 1726.68 |
| 1396539 | 379368 | 1726.08 | RC | 35.693 | 35.69297 | 1726.08 |
| 1396535 | 379394 | 1725.8 | RC | 25.847 | 61.54 | 1724.55 |
| 1396530 | 379415 | 1724.55 | RC | 21.3438 | 82.88376 | 1724.55 |
| 1396443 | 379564 | 1722.57 | RC | 172.579 | 255.4627 | 1722.57 |
| 1396373 | 379594 | 1721.42 | RC | 76.3708 | 331.8335 | 1721.42 |
| 1396286 | 379653 | 1720.5 | RC | 105.105 | 436.9387 | 1720.5 |

# SECTION-II: HEADWORK DESIGN

# HEADWORK STRUCTURES DESIGN

## Headwork site selection

The headwork site is situated at 1396513.46m N, 379446.63m E and river bed elevation of 1724.02m above sea level. At this site the river course is well defined, matured with fixed width and forms nearly a U-shaped valley. At this specific site, covered by outcropped with highly jointed Basaltic Rock at the river bed where as the left bank is also made from alluvial deposit (cobble, gravel, sand and silt) and with about 2m height and with few centimeter silt clay soil at the top but the right bank is covered by the loose transported and  insitu developed brown silt clay soil with about 2-3m height and the bank shows steep slope and it has sliding behavior The river is flowing through a defined channel and its scouring effect on the river course is  not significant as its bed covered by outcropped with highly jointed Basaltic rock.

### River Geomorphology

The pattern of the drainage system of the study area is strongly influenced by geological structures and formations, the nature of the vegetation cover and climate. It is a common fact that the development of the river tends to accommodate itself to the local geology that develops along the fault zone. Geological formations and structures have strong influence on the channel development.  The river course is pre- matured, undefined and does not have stable abutments and fixed width rather liable to change time to time. Alluvial deposits fill the river channels so that the course (route) might be changed through time.

#### River Bed condition

The weir bed is the main section to be investigated because this part is the section in which the foundation of the structure will have settled. Here our weir site bed is outcropped with highly jointed Basaltic Rock. The nature of the joints and their characteristics has been well investigated. The joints are interconnected each other by making square shapes and the spacing between the joints is very close. Therefore this type of the joint characteristics is not recommended for the large structures because the joint allows seepage and leakage through and below the foundations of the structure.

Figure 3-1: Picture showing the nature of the weir axis bed rock

But in the case of our site it does not have the problem. Since the structure which is going to be built up on is small scale the presence of joints never bring the problem. But the Adrikayna site weir site bed rock should be excavated up to .5-1 meter depth vertically down to remove the upper weathered and jointed rocks and the foundation should be settled on the fresh basaltic rock.

#### River Bank condition

Right Bank

It is made from the loose transported and  insitu developed brown silt clay soil with about 2-3m height and the bank shows steep slope and it has sliding behavior. One test pit was taken during investigation time in this side of the command area. And it shows dark brown silt clay soil with few granular materials at top part of the pit but to the depth moist, stiff dark brown insitu developed clay soil encountered.

From visual method, it is dry to moist, low reaction to shaking, high dry strength and toughness and named as Medium to high plasticity CLAY to silt CLAY soil.

As its bank show sliding nature retaining wall for both upstream and downstream should be consider during design.

Left Bank

Left bank is made from alluvial deposit (cobble, gravel, sand and silt) and with about 2m height and with few centimeter silt clay soil at the top. Using visual method, this older, denser sediment layer was classified according to Unified Soil Classification System (USCS), and falls in poorly graded GRAVEL (GP) with silt soil group having about 40% silt, 1.5% fine Sand and 58.5% brownish to grayish colored clay by weight. The maximum observed size within the sediment reaches about 8mm diameter, and less than 3% oversize materials, by volume, (greater than 10mm) also incorporated within it. The average grain size of the sediment is estimated to be 3mm in diameter.

### Sources of construction materials

During site investigation, natural construction materials required for the construction of the various proposed engineering structures at the headwork and within the farmland have been assessed, and possible quarry sites and borrow areas have been identified within the vicinity of the study area as close to the project site as possible. In addition to identifying the quality, quantity and accessibility conditions of the construction materials, ownerships of each proposed production sites have also been studied and described in this report, on separate sub-sections below. The materials needed for the construction of the structures include rock for masonry stones, aggregates (both coarse and fine), and water.

#### Rock for Masonry and Crushed Coarse Aggregate

During investigation time, the masonry source area delineation (definition) was done. The particular source area that will use as masonry stone source is found around the right side ridge 3km far away. Most ridge and mountain of the surrounding project site in both right and left side of the command area is occupied by boulder agglomerate rock and insitu fissure flood basalts. This rock is basic in nature. And the fresh basalt has strength that needs for the construction of the proposed main canal route and the weir. It has required rock strength that used for masonry rock source hence by collecting these basaltic rocks from mostly from the right side of the ridge by crushing to the necessary rock size. It is tough and durable, and abundant in quantity.

#### Fine Aggregates

Sand source is located at very far distant place. It is found at Aderkayna irrigation project site specifically within the Debary River bed which is about 22km far away from the project. This sand is currently used for the construction of the bridge on the Debary River.  It can be collected by sieving and washing of the deposits to remove undesirable sizes. At the time of investigation it is used as sand source for the building of Bridge. The sand is dark in color and sand sizes dominate. The amount and quality of this sand is enough and good. From field method, it is classified as poorly graded SAND(SP) having 80% fine sand,10-15% gravel and <5% silt soil

#### Water

During construction time water is important to mix concrete with cement then at the time of investigation the availability of enough water should be checked around the proposed project site. The river Aderkayna is perennial and water can get from the proposed river for mix purpose. The quantity and quality is enough for the project

## Headwork type selection

Looking the availability of natural construction materials and considering the river features and expected flood amount, broad crest type of weir is chosen. As it is:

- Simple for construction

- Defined channel formation bank top level

- At relatively narrower X-section of the stream

- The maximum irrigable command area

- economical weir height

- Geological condition(Foundation)

## Hydraulic design of headwork structure

### Weir Height Determination

The following major factors have been seen in determining the weir crest level:

- Maximum  command area elevation

- Deriving head of the weir structure

- Main canal slope

- Loss

- Lowest Point of river center

Table 3-1: Weir height determination

| Description | Result |
| --- | --- |
| Peak Discharge , Q in m3/s | 229.00 |
| Crest length ,L in m | 40.00 |
| Discharge coefficient | 1.70 |
| Max. Command area elevation | 1725.00 |
| River Bed level | 1724.02 |
| Water depth required | 0.60 |
| Free board | 0.10 |
| Head loss across the field | 0.01 |
| Head loss at the turn out | 0.00 |
| Slope of the canal | 0.00 |
| Distance from the weir(outlet) | 298.89 |
| Loss in the canal | 0.43 |
| Head loss across head regulator | 0.06 |
| Total loss | 0.50 |
| FSL | 1726.10 |
| Outlet sill level | 1725.50 |
| Crest level of the weir | 1726.20 |
| Crest height | 2.18 |

### Base flow of the River

The study team has assessed that the stream is used for irrigation along its entire course except at the proposed diversion site where farmers are using the stream for traditional SSI (Irrigation Infrastructure Report). Study team has calculated flow of the river at the weir site as 400l/s.  Out of this 320 l/s will be required for the proposed scheme and the rest will be released for downstream. The purpose of releasing the 80 l/s to downstream is for the sake of downstream users.

### Weir Dimensions

#### Flow over the Weir crest

### Crest Length

- Lacey’s regime width, L=4.75*Q,  =4.75* 229 = 71.5m.but taken the actual width of the river which is about 40m length.

### Discharge over the weir section

- Design discharge, Q = 229m3/s

#### Top and bottom width

According to the Bligh’s formula, top and bottom width of the weir body is determined as follows

- Input Data:

P: Height of weir/intake (m) = 2.18

He: specific energy head (over flow depth + approaching velocity head (m))

: Specific weight of weir body (2.3 for cyclopean concrete)

Top width,  2.01m ----but taken 1.6m

Bottom width,   `3.96....but taken 3.5m

Provide 1.6m and 3.5m top and bottom width respectively, which will be tested for adequacy during stability analysis.

### U/S and D/S HFL Calculation & Determination

From the stage –discharge curve prepared (Section 1, in Hydrology Part) the high flood level before construction (i.e. D/s HFL) corresponding to the design flood is 1727.98m a.m.s.l.

D/s HFL = 1727.02 amsl ------------------------------------- (a)

U/s HFL = U/s bed level + weir height + Hd -------- (b)

Hd is the depth of water over the weir crest. This is calculated by assuming broad crested weir formula.

Q=C*L*He32

He=QC*L23 = 2.2

Schematic Diagram of weir Section

The velocity head, ha is computed from the approach velocity as shown below

Where g: acceleration due to gravity = 9.81m/sec2

Va is Approach velocity determined by

L is Weir crest length = 50m,

hd is flow depth over the weir and also,

By trial and error method, hd is found to be 1.78m

- ha = He-hd = 2.2m-0.48m = 1.74

- Velocity head, ha = 2.24m

U/s HFL =1727.95m a.s.l

- Afflux

⇒ Afflux = U/s HFL- D/s HFL = 1727.95m a.s.l – 1727.02 m a.s.l =0.93m.

### Hydraulic Jump Calculation

As discussed in the geologic report, the river bed is weather rock and hence stilling basin for energy dissipation is required. Both left and right side banks are not sound rock, a wing walls are required at u/s and D/s sides, so as to protect the scouring of the bank due to the formation of jumps, and not to flow the river out of river bank in high flood cases.

The length of wing walls is determined based on the length of Jump, and it is calculated as shown below.

• Weir crest length = 40m

• Weir height = z = 2.18m

• Pre-jump depth = y1

• Post -jump depth =y2

z + He = y1 + ha

but,  He = 2.18m

q=Ql=229m3/s40m=5.73m2/s    ha=q22*g*y2=5.7322*g*y12

2.18m + 2.25m=y1+2.18/y12

After iterations Y1 = 0.66m

V1=q/y1=5.73/0.66=8.67 Fr=8.67gy1=8.4239.81*0.66=3.41

y2=y121+8*Fr2-1             y2=0.6621+8*3.412-1 =2.87m

Hydraulic jump length (L) for Fr=3.41, using equation L=5*(y2-y1 ) =11.05m,but taken 3 m

Figure 3-2: Tail water depth and jump height depth comparison

### Impervious floor &cut-off

The upstream & d/s cutoff depth is provided to the extent where scouring damage could extend over the years. For Aderkayna project a nominal provision of 0.5 m below river bed is provided at upstream side and 1m for downstream. Based on the geologic information, expected hydraulic condition and the sediment characteristics exhibited at the weir location this provision is considered sufficient.

The thickness for the cutoff  is maintained at 0.5 m, as is the case in the downstream provisions, considering the small size of the structure, this provision will provide optimal thickness

## Stability Analysis of weir

Stability analysis is carried out to see the already determined weir section is safe against overturning, sliding, tension. The stability analysis is carried out considering the effect of the following forces.

• Water pressure

• Weight of the over flow weir section

• Sediment load

The extreme load combination is the case where the head is at crest level of the weir and there is no flow over the weir (static case)

| Stability analysis |  |
| --- | --- |
| Unit weight of materials |  |
| cyclopean concrete | 22.50 |
| Mass concrete KN/m3 | 23.00 |
| Reinforced concrete KN/m3 | 24.00 |
| Water KN/m3 | 9.81 |
| specific weight of silt KN/m3 | 18 |
| Bottom width | 3.5 |
| Top width | 1.60 |
| weir height | 2.18 |
| α ( angle of internal friction) | 25 |

a. Stability of the weir on Static condition

| 1.60. Toe. B=3.5. Pu. |  |  |  |
| --- | --- | --- | --- |

| Code | Dimension |  | Load |  | Lever arm (about the toe) | Moment |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Width | Depth | Vertical | Horizontal | R | Positive | Negative |
| Pw | 2.18 | 2.18 | 0.00 | 23.31 | 0.73 | 0.00 | 16.94 |
| W1 | 1.60 | 2.18 | 78.48 | 0.00 | 2.70 | 211.90 | 0.00 |
| W2 | 1.90 | 2.18 | 46.60 | 0.00 | 1.27 | 59.02 | 0.00 |
| Pu | 3.20 | 2.20 | 34.53 | 0.00 | 2.33 | 0.00 | 80.57 |
| Ps | 17.85 | 2.18 | 0.00 | 8.55 | 0.73 | 0.00 | 6.22 |
| Sum | 26.73 | 10.92 | 90.55 | 31.86 |  | 270.92 | 103.73 |

Factor of safety against,

| Over turning = (M+ve/M-ve) >1.5 |  |  |  | Fo | 2.61 | >1.5 | OK |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Sliding = (µxFv/Fh),---µ=0.7 >1.5 |  |  |  | Fs | 1.99 | >1.50 | OK |
| Tension: X= (Net Moment/Sum Fv), e=x-B/2, e<B/6 |  |  |  | X | 1.85 |  |  |
|  |  | B/6= | 0.58 | e | 0.10 | <B/6 | OK |

b. Stability of the weir at Dynamic condition

| Code | Dimension |  | Load |  | Lever arm about the toe | Moment |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Width | Depth | Vertical | Horizontal |  | Positive | Negative |
| P1 | 2.18 | 2.180 | 0.00 | 23.31 | 0.73 | 0.00 | 16.94 |
| P2 | 2.18 | 1.090 |  | 23.31 | 1.09 |  | 25.41 |
| P3 | 0.88 | 0.875 | 0.00 | 3.76 | 0.29 | 1.10 | 0.00 |
| W3 | 1.90 | (1.109) | (10.34) | 0.00 | 0.63 | (6.55) | 0.00 |
| W4 | 1.60 | 1.090 | 17.11 | 0.00 | 2.70 | 46.19 | 0.00 |
| W1 | 1.60 | 2.180 | 80.22 | 0.00 | 2.70 | 216.60 | 0.00 |
| W2 | 2.40 | 2.180 | 60.17 | 0.00 | 1.27 | 76.21 | 0.00 |
| U1 | 3.50 | 0.875 | 30.04 | 0.00 | 1.75 | 0.00 | 52.58 |
| U2 | 3.50 | 2.395 | 41.12 | 0.00 | 2.33 | 0.00 | 95.94 |
| Ps | 17.85 | 2.180 | 0.00 | 8.55 | 0.73 | 0.00 | 6.22 |
| Sum | 37.59 | 13.94 | 76.01 | 28.11 | 14.22 | 333.56 | 197.08 |

|  |  |  |  |  | 1.2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Pu |  |  |  |  |  |

Factor of safety against,

| Over turning = (M+ve/M-ve) >1.5 |  |  |  | Fo | 1.69 | >1.5 | OK |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Sliding = (µxFv/Fh),---µ=0.7 >1.5 |  |  |  | Fs | 1.89 | >1.50 | OK |
| Tension: X= (Net Moment/Sum Fv), e=x-B/2, e<B/6 |  |  |  | X | 1.80 |  |  |
|  |  | B/6= | 0.58 | e | 0.05 | <B/6 | OK |

### Wall height fixation of Divide Wall

The existing topographical condition at the weir axis and HFL are considered to be most governing parameters for fixing the wall height.

After construction of the weir its crest level is= 1726.25m.a.s.l

River bed level=1724.02m.a.s.l

Free board=0.1m

Afflux=0.93m

Hence the level of divide wall and weir body foundation should be the same and is 1724.02m

U/s wall height = U/s HFL - foundation level + free board.=1729.19-1724.02+0.1=5.27m

The result found by this formula is very high so, it is preferable to find other way of finding its height

U/s wall height=weir crest level+afllux+free board-river bed level=3.5m

D/s wall height=post jump level+fb-river bed level=1727.57+0.1-1724.02=3.65m

### Under sluice

The under sluice is mainly provided here to remove silt deposition at a result of barrier structure. Hence the sill level of the under sluice is fixed to facilitate this deposited silt to increase the efficiency of water abstracting to the main canal through the head regulator from the pocket.  Even if the position of the under sluice is on concave side that is on scouring side, there might be boulders that may come into the pocket of the under sluice due to the barrier structure. Hence in addition to the supply of water to the intake and the removal of silt, this acts to remove the boulder that comes to wards it. Considering this, the opening size of the gate is .5m*1m with spindle type operating from the operation slab. Considering rectangular notch profile of flow of water at the under sluice, the discharge passing is computed using the following formula.

- The capacity should be at least five times the canal discharge to ensure proper scouring.

- Capacity of passing about 10% to 20% of the maximum flood discharge at high floods.

- During construction, it should be able to pass the prevailing (at least base flow) discharge of the river.

But the practical application of those values is not sound so the discharging capacity of under sluice is determined by using orifice flow formula for maximum flood condition.

i.e. Q=Cd*A*2gh,

Where, Cd = Discharge coefficient =0.6

A = under sluice opening area (m2)

g = 9.81m/s2

Width (b) = Qd Cd*H2g*h      20.8*12*9.81*1=   =0.8m

Hence the size of the under sluice is 0.6m×1m    (height *width)

### Canal outlet level

The head regulator is provided on the right side. The sill level of this head regulator is fixed from different angle observations. The main conveyance system is more than 2km which passes more gullies and undulating alignment. Hence this level is fixed based on the optimum route alignment and the maximum irrigated command level including minor and major losses criteria. Based on this condition, the sill level is fixed to be 1725.50m.

### Outlet capacity

The minimum command area is determined by the minimum flow of the river. But the canal capacity should be determined for maximum command area and the corresponding discharge. In this case the outlet capacity is fixed considering maximum duty and command area and correction factors are considered to account the variation of duty.

- Outlet capacity = Duty x command area x correction factor

Where, maximum duty for 16 hr irrigation = 1.35 L/s/ha

Command area = 240ha.

- Outlet capacity = 1.35 L/s/ha x 240ha = 320L/sec

### Outlet size

From the weir discharge formula the outlet size is determined as follows

Q = CLHe3/2

Where; C = coif. Of discharge = 1.7

L = Length of water way (m)

He = head above sill level (neglecting the velocity head) = 0.5m

L=QCHe32=0.21.7*2.9432=0.6m

⇒Adopt water way length = 0.6m

Hence, provide an outlet size of 0.8m x 0.6m (length x height) .The gate of the off take canal is to be vertical sheet metal of 0.8m x 0.60m for the closure of the opening space. Provide some extra dimensions for groove insertion. Gross area of sheet metals for the off take canal gate will be 0.85m x 0.650m (allowing 5cm insertion for grooves and above the weir crest level). The grooves are to be provided on the walls using angle iron frames at the two sides of the gate openings.

Trash racks of diameter 14mm with c/c spacing of 50cm has to be provided u/s of the gate to prevent entry of debris to the canal.

### Breast Wall and Operation Slab

A vertical raised gate is designed for the head regulator and under sluice. These gates slide over the breast wall using spindle during opening and closing.

For easy operation of these gates, operation slab is provided. The size of the operation slab is fixed from the point of construction and free movement. The size of the operation slab is shown in the drawing under sluice with thickness 0.2m.

The nominal thickness is fixed from the point of construction rather than the imposed load. The thickness required for the imposed load is less than the nominal value and treated as cantilever retaining wall. For the breast wall, the minimum reinforcement area is taken as 13% along the respective direction.

Hence	Asteel = 0.0013*1000*200=288

Asteel =288mm2/m

Provide 12@C/c 300 mm

Considering cover thickness of 50 mm, effective depth = 50 + 12/6 = 56

Hence spacing of reinforcement =300mm < 396mm

Asteel = 491mm2/m

Therefore the actual provided steel area per meter width is 491 mm2/m > 288mm2/m. It is ok!

The actual arrangement of angle irons, spindles, shafts and operation slab including other components is shown on the design drawing.

For each arrangement and further information, refer to the design drawing.

### Retaining Walls

At the two ends of the diversion weir, walls have been provided to safeguard the structure from scour at the ends and also as a facility to the canal outlet operation and maintenance at the canal outlet portion.

Table 3-2: Typical design of masonry retaining wall

| USFL | 1727.95 |  |
| --- | --- | --- |
| Post jump level | 1726.89 |  |
| Free board | 0.10 |  |
| U/S River Bed level | 1724.02 |  |
| Weir Crest Level | 1726.20 |  |
| Under sluice Channel Sill level | 1724.02 |  |
| Under sluice channel Total Depth | 1.00 |  |
| Jump Length | 11.00 |  |
| Height of U/S Wing Wall | 4.03 | take=4.1m |
| Height of D/S Wing Wall | 2.97 | Take h=3m |
| Top level of the U/S wall | 1728.0 |  |
| Top level of the D/S wall | 1727.0 |  |

U/S Right & Left Side dimension

| H | FB | HT |  | B1 | B2 |
| --- | --- | --- | --- | --- | --- |
| 4.00 | 0.10 | 4.10 |  | 0.5 | 1.5 |

Stability analysis

| Code | Load |  | Lever Arm, R | Moment |  |
| --- | --- | --- | --- | --- | --- |
|  | Vertical | Horizontal |  | Positive | Negative |
| W1 | 46.1 |  | 0.3 | 11.5 |  |
| W2 | 69.2 |  | 1.0 | 69.2 |  |
| W Soil | 58.4 |  | 1.5 | 87.6 |  |
| P Soil |  | 52.7 | 1.4 |  | 72.0 |
| SUM | 173.7 | 52.7 |  | 168.4 | 72.0 |

Factor of safety against

| Over turning = (M+ve/M-ve) >1.5 |  |  | Fo | 2.34 | >1.5 | OK |
| --- | --- | --- | --- | --- | --- | --- |
| Sliding = (µxFv/Fh),---µ=0.75 >1.5 |  |  | Fs | 2.47 | >1.50 | OK |
| Tension: X= (Net Moment/Sum Fv), e = x-B/2, e<B/6 |  |  | X | 0.55 |  |  |
|  | B/6= | 0.25 | e | 0.20 | <B/6 | ok |

D/S Right & Left Side dimension

| H | FB | HT |  | B1 | B2 |
| --- | --- | --- | --- | --- | --- |
| 3.0 | 0.00 | 3.0 |  | 0.4 | 1.2 |

Stability analysis

| Code | Load |  | Lever Arm, R | Moment |  |
| --- | --- | --- | --- | --- | --- |
|  | Vertical | Horizontal |  | Positive | Negative |
| W1 | 26.7 |  | 0.20 | 5.3 |  |
| W2 | 40.1 |  | 0.80 | 32.1 |  |
| W Soil | 33.8 |  | 1.20 | 40.6 |  |
| P Soil |  | 27.6 | 0.99 |  | 27.3 |
| SUM | 100.6 | 27.6 |  | 78.0 | 27.3 |

Factor of safety against,

| Over turning = (M+ve/M-ve) >1.5 |  |  |  | Fo | 2.85 | >1.5 | OK |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Sliding = (µxFv/Fh),---µ=0.75 >1.5 |  |  |  | Fs | 2.73 | >1.50 | OK |
| Tension: X= (Net Moment/Sum Fv), e=x-B/2, e<B/6 |  |  |  | X | 0.50 |  |  |
|  |  | B/6= | 0.20 | e | 0.10 | <B/6 | OK |

Determination of cutoff (scour depth)

| Descripation | Relation | result |
| --- | --- | --- |
| Peak Discharge , Q in m3/s |  | 229 |
| crest length ,L in m |  | 40 |
| Unit Discharge q,m3/s/m | Q/L | 5.725 |
| average particle size in (mm) D50 |  | 45 |
| Silt factor(f) | f=1.76*d^0.5 | 11.80643892 |
| Depth of scour( R) | R=1.35(q^2/f)^(1/3) | 1.897336905 |
| u/s HFL |  | 1727.946731 |
| river bed level,a.m.s |  | 1724.02 |
| D/S HFL |  | 1727.02 |
| Weir crest level ,a.m.s |  | 1726.197114 |

Since the river bed is massive rock protection for both upstream and downstream is given nominally

Upstream cut off depth=0.5

Thickness=0.5

Length of apron =2m

Downstream cutoff depth=1.0m

Thickness=0.5m

Length of apron =

# SECTION-III: IRRIGATION AND DRAINAGE SYSTEMS INFRASTRUCTURE

# IRRIGATION AND DRAINAGE SYSTEMS DESIGN

## Irrigable area description

### Topography

Topography is an important factor for the planning of any irrigation project as it influences method of irrigation, drainage, erosion, mechanization, and cost of land development, labour requirement and choice of crops.

The topographic feature of the project command area is naturally inclined from South-West to North-East directions. Its elevation range is from 1740 to 1817 meters above sea level. The slope gradient also ranges from very gently sloping (2%) to sloping (10%). However, it has identified to be suitable for surface irrigation. Nevertheless, it requires soil and water conservation measures or structures (i.e. constructing bunds, bio-physicals, check dams, artificial water ways, etc).

The project command area is situated at the left side of Aderkayina River (to the East side of the river). The natural topographic feature of the command area has from South-West to North-East direction.

### Climate

As per the hydrological analysis and on the basis of the traditional Ethiopian Agro-Ecological Zones (MOA, 2001), the UGDWIP area is basically classified as Dry Woina Dega (Dry warm) agro-ecological zone, indicating rainfall distribution problem and poor moisture condition in the area. There is no belg rain season in the project area. Although Meher rain is considered adequate, there is notable variation in terms of onset, distribution and withdrawal rain season from year to year affecting crop production and productivity.

As the project site has no its own meteorological station, Arbaya (for rainfall and minimum and maximum temperature) and Gondar (for relative humidity, wind speed and sunshine hour) meteorological stations data were used for the project study as long as these stations are relatively near to the proposed command area. In general, the sources of meteorological data are the National Meteorology Service Agency (NMSA).

### Soil characteristics

Soil properties (physical, chemical, etc.) greatly influence the growth and thereby yield of crops which is grown. The command area has predominantly heavy textured clay soils which can be classified imperfectly drained soil.  Most of the study area soils are categorized as very deep soil (≥1.5 meters).   Soils of the command area are suitable for most of the selected crops to be grown (for further detail see the Agronomy Study of the same project.

### Existing Irrigation Practices in the Project Area

The pressure of survival and the need for additional food supplies to meet the demands of the increasing population is necessitating a rapid expansion of irrigation schemes. Thus, irrigation is becoming a basic part of well developed agriculture wherever there is water and irrigable land potential. Accordingly, traditional irrigation practices are under taken by individual farmers that use the river flow to the right side is with laborious temporary diversions. So, the farmers in the project area are very much interested in the idea of upgrading the traditional scheme to modern scheme.

## Irrigation water requirement

### Crop Water Requirement (CWR)

The calculation of crop water requirement is a very important aspect for planning of any irrigation project. Several methods and procedures are available for this. The Food and Agriculture Organization (FAO) of the United Nations has also made available several publications on this subject and other issues related with this. The computer program available in FAO Irrigation and Drainage Paper No. 56 “CROPWAT” has been used for the calculation of Crop Water requirement. This program is based on Penman-Monteith approach and procedures for calculation of crop water requirements and irrigation requirements are mainly based on methodologies presented in FAO Irrigation and Drainage Paper No. 24 “Crop Water Requirements” and No. 33 “Yield Response to Water”.

The corresponding values of the crop water requirements of the proposed crops of the project are presented in the Agronomy Study of the same project.

### Irrigation efficiency (Ep)

To complete the evaluation of the demand, the efficiency of the water distribution system and of application must be known.

The gross requirement of water for irrigation system is very much dependent on the overall efficiency of the irrigation system, which in turn is dependent on several factors: Method of irrigation, type of canal (Lined and/or Unlined), method of operations (simultaneously and continuous or Rotational water supply), and availability of structures (for controlling and distribution and measuring and monitoring).

On the basis of these factors, the project has planned to impose surface irrigation method (using furrows). The canal system is both lined and unlined. Hence, the conveyance efficiency has been estimated to be 85%, and field application efficiency 60%. As a result of these the overall irrigation efficiency has been estimated to be 50%. According to soil Lab result, soils of the command area are predominantly characterized as clayey soils.

### Irrigation duty

Irrigation duty is the volume of water required per hectare for the full flange of the crops. Moreover, it helps in designing an efficient irrigation canal system.

The area, which will be irrigated, can be calculated by knowing the total available water at the source and the overall duty for all crops required to be irrigated in different seasons of the years.

The proposed cropping pattern of Adirkayina diversion irrigation project has showed a maximum net irrigation water requirement (NIWR) in the month of February with the amount of 5mm/day for 24 working hours (for overall proposed crops).

For Aderkayna River Diversion Irrigation Project, it decided to adopt 60% field application efficiency, and 85% conveyance efficiency as the soil is clay textured and the canal systems are estimated to be unlined except small portion near head work site. Hence, the overall/project efficiency for the selected surface irrigation method has been estimated to be 51% (60/100*85/100) which is rounded to 50%.

For the designing of the project, the GIWR is given as follows:

GIWR = 5/0.50 = 10 [mm/day] for full irrigation

GIWR = 3.4/0.50 = 6.8 [mm/day] for supplementary irrigation

The GIWR, 9.4 mm/day, represents the daily quantity of water that is required to be applied. This water quantity is also used for the determination of the canal discharge in consideration of the time of flow and is defined as the duty, expressed as l/s/ha.

The duty is calculated by:

Duty (D) = GIWR × 1000 × 10 / (t × 60×60)

Where; Duty – the duty [l/s/ha]

GIWR – Gross Irrigation Requirement [mm/day]

t – Daily irrigation or flow hours [hrs]

The duty for the GIWR of 10mm/day and 14 hours of daily irrigation time (t = 14), is supported to be used with furrow irrigation method. Hence, Duty for 14 working hours, as the site is a little bit farer to farmers’ village) has computed as follows:

D = (10 x 1000 x 10) / (14 x 3600) = 2.0 l/s/h (for full irrigation)

D = (6.8 x 1000 x 10) / (14 x 3600) = 1.35 l/s/h (for supplementary irrigation)

### Irrigation methods

Among the different irrigation systems perennial irrigation system will be used for the project area; and the irrigation water will be obtained from Aderkayna  River and by constructing diversion weir and convoying the water commonly through both  lined(PC,SC) & earthen canal (TC) and then leading to field canals; and finally irrigation takes place mostly in furrows.

For this project, among the various irrigation methods, surface irrigation method has been selected.  Of the surface irrigation methods furrow, border and basin irrigation methods can be used to supply irrigation water to the plants/crops. However, each method has its own advantages and disadvantages. Care should be taken when choosing the method which is best suited to the local circumstances, i.e., depending on slopes, soil types, selected crop types, amount of water available, etc. of the command area.

Based on the above factors surface irrigation method has been proposed for the proposed crops in this project. The method allows applying light irrigation and can be laid out in sloping fields along the contour. Furrow irrigation method is best suited for most of the proposed and row planted crops. In general, furrow irrigation method is simple, manageable and widely practiced irrigation method. This method is suitable for row crops that cannot stand in water for long periods. The only thing required to use this method is row planting of crops. Besides, basin and border irrigation method would be used for the non-row planted crops. Rotational flow water distribution is also recommended for the project area.

## Irrigation and drainage system layout

The irrigation system layout for the project is prepared taking the following points into consideration besides other factors.

- A primary concern in the layout of the system is that it serves the purpose of conveying and distributing water to the command area.

- The excavation and earth fill volumes not be excessive, otherwise the construction costs can be tremendous.

- The selection of longitudinal bed slope is made taking into account the existing slopes of the terrain, so as to minimize deviations in canal routing.

- Curves in canals should not be too sharp.

The proposed irrigation system layout comprises 1 (Nr) main canal, 5 (Nr) secondary canals and 29(Nr) tertiary canals as shown on the layout Drawings. The main canal runs for most of its length parallel to the contours and several changes of direction are necessary to follow the topography. It crosses One main gully. The main canal is masonry lined for a length of 639.32meters starting from the weir outlet to make maintenance easier since this part of the canal may be subjected to flooding during high flood flows.

### Conveyance System

The conveyance system consists of 1(Nr) Main canal to irrigate total command area of 240 ha. The main canal starts from Water abstraction site on left side and conveys water for a length of 639.343 m.

Main canal is aligned along contours and supplies to Five secondary unit and 29 tertiary canals.

## Design of the canal system

Flow Depth and Section Capacity

The earthen canals have been designed with a trapezoidal shape and the lined ones with rectangular x-section using Manning's Formula:

Where Q= discharge (m3/s)

R= Hydraulic radius (Flow area/wetted perimeter)

S= Hydraulic gradient

n= Manning's roughness coefficient, n=0.025 is adopted for the earth channels and n=0.018 for the masonry lined part of the main canal

### Main Canal

Figure 4-1: Typical section of lined canal

The main canal is designed for a discharge of 320l/s and depending on the site specific condition, appropriate slope is provided. Hydraulic parameters of the main canal are shown below.

At a chain age of 470.093 there is Flume structure.

Table 4-1: Hydraulic Parameters of main canal

| Reach | Value of n | Bed Slope | BW (m) | FSD (m) | FB (m) | V (m/sec) | WP (m) | SS | QR cumec | QD cumec |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 to 298.898 | 0.018 | 700 | 0.7 | 0.6 | 0.1 | 0.77 | 1.9 | 0:1 | 0.32 | 0.32224 |
| 298.898to299.998 | 0.018 | 700 | 0.7 | 0.6 | 0.1 | 0.77 | 1.9 | 0:1 | 0.317 | 0.32224 |
| 299.998to470.093 | 0.018 | 500 | 0.7 | 0.52 | 0.18 | 0.88 | 1.74 | 0:1 | 0.317 | 0.3187 |
| 470.093to480.176 | 0.018 | 500 | 1 | 0.362 | 0.188 | 0.88 | 1.724 | 0:1 | 0.317 | 0.3177 |
| 480.176to639.343 | 0.018 | 500 | 0.7 | 0.52 | 0.18 | 0.88 | 1.74 | 0:1 | 0.317 | 0.31869 |

BW= Canal bottom width                         QR= Required discharge

FSD= Full supply depth                             QD= Designed Discharge

FB= Free board

SS= Side slope

Vel= Velocity

### Secondary Canals

Table 4-2: Hydraulic Parameters of secondary canals

| Name | Reach | Value of n | Bed Slope | BW (m) | FSD (m) | FB (m) | V (m/sec) | WP (m) | SS | QR cumec | QD cumec |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SC-1 | 0+0-224.979 | 0.018 | 70 | 0.4 | 0.3 | 0.1 | 1.6155 | 1 | 0:1 | 0.1919 | 0.1938 |
|  | 224.979-749.957 | 0.018 | 100 | 0.55 | 0.25 | 0.3 | 1.4326 | 1.05 | 0:1 | 0.1919 | 0.1969 |
|  | 749.957-754.53 | 0.018 | 50 | 0.45 | 0.25 | 0.2 | 1.895 | 0.95 | 0:1 | 0.1919 | 0.2131 |
|  | 754.53-773.273 | 0.018 | 50 | 0.4 | 0.25 | 0.15 | 1.816 | 0.9 | 0:1 | 0.1777 | 0.1816 |
|  | 773.273-874.957 | 0.018 | 50 | 0.4 | 0.25 | 0.15 | 1.816 | 0.9 | 0:1 | 0.1626 | 0.1816 |
|  | 874.957-932.605 | 0.018 | 70 | 0.45 | 0.25 | 0.2 | 1.6013 | 0.95 | 0:1 | 0.1626 | 0.1801 |
|  | 932.605-1074.957 | 0.018 | 70 | 0.4 | 0.25 | 0.15 | 1.535 | 0.9 | 0:1 | 0.1523 | 0.1535 |
|  | 1074.957-1093.894 | 0.018 | 80 | 0.45 | 0.25 | 0.2 | 1.498 | 0.95 | 0:1 | 0.1523 | 0.1685 |
|  | 1093.894-1524.957 | 0.018 | 80 | 0.4 | 0.25 | 0.15 | 1.436 | 0.9 | 0:1 | 0.1304 | 0.1436 |
|  | 1524.957-1547.45 | 0.018 | 100 | 0.45 | 0.25 | 0.2 | 1.339 | 0.9 | 0:1 | 0.1304 | 0.1507 |
|  | 1547.45-1849.957 | 0.018 | 100 | 0.4 | 0.25 | 0.15 | 1.284 | 0.9 | 0:1 | 0.1195 | 0.1284 |
|  | 1849.957-1858.517 | 0.018 | 70 | 0.45 | 0.2 | 0.25 | 1.486 | 0.95 | 0:1 | 0.1195 | 0.1337 |
|  | 1858.517-2349.957 | 0.018 | 70 | 0.4 | 0.2 | 0.2 | 1.431 | 0.8 | 0:1 | 0.0994 | 0.1145 |
|  | 2349.957-2490.357 | 0.018 | 100 | 0.45 | 0.2 | 0.25 | 1.243 | 0.85 | 0;1 | 0.0994 | 0.1119 |
|  | 2490.357-2887.726 | 0.018 | 100 | 0.35 | 0.2 | 0.15 | 1.143 | 0.75 | 0:1 | 0.0751 | 0.0800 |
|  | 2887.726-3042.635 | 0.018 | 50 | 0.35 | 0.15 | 0.2 | 1.468 | 0.65 | 0:1 | 0.0751 | 0.0771 |
|  | 3042.635-3362.52 | 0.018 | 70 | 0.4 | 0.15 | 0.25 | 1.291 | 0.7 | 0:1 | 0.0751 | 0.0775 |
|  | 3362.52-3587.52 | 0.018 | 80 | 0.45 | 0.15 | 0.3 | 1.247 | 0.75 | 0:1 | 0.0751 | 0.0842 |
|  | 3587.52-3615.337 | 0.018 | 50 | 0.35 | 0.15 | 0.2 | 1.468 | 0.65 | 0:1 | 0.0751 | 0.0771 |
|  | 3615.337-3687.521 | 0.018 | 50 | 0.35 | 0.15 | 0.2 | 1.468 | 0.65 | 0:1 | 0.065 | 0.0771 |
|  | 3687.521-4342.705 | 0.018 | 80 | 0.35 | 0.2 | 0.15 | 1.278 | 0.75 | 0:1 | 0.065 | 0.0895 |
|  | 4342.705-4537.521 | 0.018 | 80 | 0.35 | 0.15 | 0.2 | 1.161 | 0.65 | 0:1 | 0.0569 | 0.0609 |
|  | 4537.521-4572.163 | 0.018 | 70 | 0.35 | 0.15 | 0.2 | 1.241 | 0.65 | 0:1 | 0.0569 | 0.065 |
|  | 4572.163-4825.05 | 0.018 | 70 | 0.3 | 0.15 | 0.15 | 1.181 | 0.6 | 0:1 | 0.0497 | 0.0531 |
| SC-2 | 0 to49.99 | 0.018 | 250 | 0.5 | 0.3 | 0.2 | 0.8308 | 1.1 | 0:1 | 0.1252 | 0.1386 |
|  | 49.99to949.81 | 0.025 | 1000 | 0.5 | 0.35 | 0.3 | 0.4321 | 1.49 | 1:1 | 0.1252 | 0.1286 |
|  | 949.81-1299.66 | 0.018 | 150 | 0.4 | 0.3 | 0.1 | 1.1036 | 1 | 0:1 | 0.1252 | 0.1324 |
|  | 1299.66 to1499.57 | 0.018 | 50 | 0.4 | 0.2 | 0.2 | 1.693 | 0.8 | 0:1 | 0.1252 | 0.1354 |
|  | 1499.57 to 1924.44 | 0.018 | 250 | 0.5 | 0.3 | 0.2 | 0.9308 | 1.1 | 0:1 | 0.1252 | 0.1396 |
|  | 1924.44 to 2199.355 | 0.018 | 100 | 0.5 | 0.2 | 0.3 | 1.284 | 0.9 | 0:1 | 0.1252 | 0.1284 |
|  | 2199.355 to 2236.46 | 0.018 | 36 | 0.45 | 0.15 | 0.3 | 1.859 | 0.75 | 0:1 | 0.1252 | 0.1255 |
|  | 2236.46 to 2240.501 | 0.018 | 36 | 0.4 | 0.15 | 0.3 | 1.8 | 0.7 | 0:1 | 0.1038 | 0.1080 |
|  | 2240.501 to 2249.35 | 0.018 | 36 | 0.5 | 0.12 | 0.38 | 1.735 | 0.74 | 0:1 | 0.1037 | 0.1047 |
|  | 2249.35 to 2552.245 | 0.018 | 50 | 0.35 | 0.2 | 0.15 | 1.616 | 0.75 | 0:1 | 0.1038 | 0.1132 |
|  | 2552.245 to 2849.206 | 0.018 | 50 | 0.35 | 0.2 | 0.15 | 1.617 | 0.75 | 0:1 | 0.0925 | 0.1132 |
|  | 2849.206 to 2924.206 | 0.018 | 100 | 0.4 | 0.2 | 0.2 | 1.197 | 0.8 | 0:1 | 0.0925 | 0.0958 |
|  | 2924.206 to 3149.199 | 0.018 | 250 | 0.45 | 0.25 | 0.2 | 0.8473 | 0.95 | 0:1 | 0.0925 | 0.0953 |
|  | 3149.199 to 3324.199 | 0.018 | 80 | 0.5 | 0.15 | 0.35 | 1.2818 | 0.8 | 0:1 | 0.0925 | 0.0961 |
|  | 3324.199 to 3521.938 | 0.018 | 150 | 0.4 | 0.25 | 0.15 | 1.0484 | 0.9 | 0:1 | 0.0925 | 0.1048 |
|  | 3521.938 to 3666.837 | 0.018 | 700 | 0.5 | 0.35 | 0.15 | 0.5818 | 1.2 | 0:1 | 0.0925 | 0.1018 |
|  | 3666.837 to 3821.92 | 0.018 | 700 | 0.45 | 0.3 | 0.15 | 0.5349 | 1.05 | 0:1 | 0.0689 | 0.0722 |
|  | 3821.92 to 4136.66 | 0.018 | 150 | 0.4 | 0.2 | 0.2 | 0.977 | 0.8 | 0:1 | 0.0689 | 0.0782 |
|  | 4136.66 to 4171.917 | 0.018 | 150 | 0.3 | 0.15 | 0.15 | 0.8067 | 0.6 | 0:1 | 0.0301 | 0.0363 |
|  | 4171.917 to 4896.902 | 0.018 | 50 | 0.3 | 0.1 | 0.2 | 1.2043 | 0.5 | 0:1 | 0.030 | 0.0361 |
|  | 4896.902 to 5022.436 | 0.018 | 150 | 0.3 | 0.15 | 0.15 | 0.8067 | 0.6 | 0:1 | 0.030 | 0.0363 |
|  | 5022.436 to 5046.876 | 0.018 | 150 | 0.3 | 0.15 | 0.15 | 0.8067 | 0.6 | 0:1 | 0.03 | 0.0363 |
|  | 5046.876 to 5086.71 | 0.018 | 50 | 0.3 | 0.1 | 0.2 | 1.2041 | 0.5 | 0:1 | 0.0141 | 0.0361 |
| TC-2-1 | 0 to 49.99 | 0.025 | 1000 | 0.3 | 0.2 | 0.2 | 0.300 | 0.87 | 1:1 | 0.0214 | 0.0300 |
|  | 49 to 249.99 | 0.018 | 50 | 0.3 | 0.1 | 0.2 | 0.867 | 0.5 | 0:1 | 0.0214 | 0.026 |
|  | 249.99 to 499.98 | 0.025 | 1000 | 0.3 | 0.2 | 0.2 | 0.3 | 0.87 | 1:1 | 0.0214 | 0.030 |
|  | 499.98 to 649.57 | 0.025 | 250 | 0.3 | 0.15 | 0.3 | 0.52 | 0.72 | 1:1 | 0.0214 | 0.035 |
|  | 649.57 to 799.98 | 0.025 | 500 | 0.3 | 0.15 | 0.3 | 0.37 | 0.72 | 1:1 | 0.0214 | 0.025 |
|  | 799.98 to 924.94 | 0.025 | 70 | 0.3 | 0.1 | 0.35 | 0.8014 | 0.58 | 1:1 | 0.0214 | 0.0321 |
|  | 924.94 to 1077.84 | 0.025 | 500 | 0.3 | 0.15 | 0.3 | 0.368 | 0.72 | 1:1 | 0.0214 | 0.025 |
| Fc-2-1-1 | 0 to 135 | 0.018 | 36 | 0.3 | 0.03 | 0.27 | 0.792 | 0.36 | 0:1 | 0.0051 | 0.0071 |
|  | 135 to 203.07 | 0.018 | 50 | 0.3 | 0.05 | 0.25 | 0.88 | 0.4 | 0:1 | 0.0051 | 0.0132 |
| Fc-2-1-2 | 0 to 90 | 0.018 | 36 | 0.3 | 0.1 | 0.2 | 1.419 | 0.5 | 0:1 | 0.0163 | 0.0426 |
|  | 90 to 180 | 0.018 | 50 | 0.3 | 0.1 | 0.2 | 1.204 | 0.5 | 0:1 | 0.0163 | 0.0361 |
|  | 180 to 375 | 0.018 | 150 | 0.3 | 0.1 | 0.2 | 0.695 | 0.5 | 0:1 | 0.0163 | 0.0208 |
|  | 375 to 442.64 | 0.018 | 50 | 0.3 | 0.1 | 0.2 | 1.204 | 0.5 | 0:1 | 0.0163 | 0.0361 |

Note: Hydraulic parameters of all canals are described at Canal L-section drawing.

Figure 4-2: Typical Cross Section of secondary canals

### Tertiary canals

In the layout system there are 30(Nr) tertiary canals, the designed discharge is determined based on the duty of irrigation.  The sections of the canals are determined by using manning’s formula, and they are both Rectangular and trapezoidal section. The hydraulic characteristics is Presented in Table 13 below.

Figure 4-3: Typical Cross Section of tertiary canals

Table 4-3: Hydraulic Parameters of tertiary canals

| Type of canal | Command area(ha) | Design discharge Q(m3/s) | Bed width (m) | Flow depth (m) | velocity (m/s) | Free board (m) | Total canal depth (m) | Top width (m) | Canal length (m) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Tc-1 | 2.17 | 0.01151 | 0.3 | 0.05 | 0.6577 | 0.4 | 0.45 | 1.2 | 200.02 |
| Tc-1-1 | 3.77 | 0.01696 | 0.3 | 0.1 | 0.4241 | 0.35 | 0.45 | 1.2 | 139.26 |
| Tc-1-2 | 6.79 | 0.01199 | 0.3 | 0.1 | 0.3 | 0.35 | 0.45 | 1.2 | 424.04 |
| Tc-1-3 | 10.54 | 0.02392 | 0.35 | 0.15 | 0.3189 | 0.35 | 0.5 | 1.35 | 339.91 |
| Tc-1-3 | 10.54 | 0.01545 | 0.35 | 0.1 | 0.343 | 0.4 | 0.45 | 1.35 | 15 |
| Tc-1-3 | 10.54 | 0.01603 | 0.3 | 0.1 | 0.401 | 0.35 | 0.5 | 1.2 | 50 |
| Tc-1-4 | 11.19 | 0.0219 | 0.3 | 0.1 | 0.5475 | 0.35 | 0.45 | 1.2 | 159.99 |
| Tc-1-4 | 11.19 | 0.017 | 0.3 | 0.1 | 0.4241 | 0.35 | 0.45 | 1.2 | 210.04 |
| Tc-1-5 | 7.58 | 0.017 | 0.3 | 0.1 | 0.4241 | 0.35 | 0.45 | 1.2 | 129.99 |
| Tc-1-5 | 7.58 | 0.0138 | 0.35 | 0.1 | 0.307 | 0.4 | 0.5 | 1.35 | 513.067 |
| Tc-1-6 | 9.47 | 0.0169 | 0.3 | 0.1 | 0.4241 | 0.35 | 0.45 | 1.2 | 99.98 |
| Tc-1-6 | 9.47 | 0.0138 | 0.35 | 0.1 | 0.307 | 0.4 | 0.5 | 1.35 | 164.7 |
| Tc-1-7 | 6.75 | 0.0189 | 0.3 | 0.1 | 0.474 | 0.35 | 0.45 | 1.2 | 89.99 |
| Tc-1-7 | 6.75 | 0.0169 | 0.3 | 0.1 | 0.424 | 0.35 | 0.45 | 1.2 | 70.01 |
| Tc-1-7 | 6.75 | 0.01382 | 0.35 | 0.1 | 0.307 | 0.4 | 0.5 | 1.35 | 349.98 |
| Tc-1-8 | 2.02 | 0.0058 | 0.3 | 0.05 | 0.3289 | 0.4 | 0.45 | 1.2 | 69.99 |
| Tc-1-8 | 2.02 | 0.00515 | 0.3 | 0.05 | 0.3 | 0.4 | 0.45 | 1.2 | 50.01 |
| Tc-1-8 | 2.02 | 0.01199 | 0.3 | 0.1 | 0.3 | 0.35 | 0.45 | 1.2 | 93.866 |
| Tc-1-9 | 8.09 | 0.01434 | 0.3 | 0.1 | 0.3584 | 0.35 | 0.45 | 1.2 | 376.84 |
| Tc-1-10 | 14.93 | 0.0218 | 0.35 | 0.1 | 0.485 | 0.4 | 0.5 | 1.35 | 208.48 |
| Tc-1-11 | 10.15 | 0.0189 | 0.3 | 0.1 | 0.474 | 0.35 | 0.45 | 1.2 | 336.21 |
| Tc-1-12 | 7.85 | 0.1897 | 0.3 | 0.1 | 0.474 | 0.35 | 0.45 | 1.2 | 18568 |
| Tc-1-13 | 2.87 | 0.00576 | 0.3 | 0.05 | 0.3289 | 0.4 | 0.45 | 1.2 | 191.32 |
| Tc-1-14 | 3.17 | 0.0066 | 0.3 | 0.05 | 0.38 | 0.4 | 0.45 | 1.2 | 125.21 |
| Tc-1-15 | 4.54 | 0.02189 | 0.3 | 0.1 | 0.547 | 0.35 | 0.45 | 1.2 | 79.99 |
| Tc-1-15 | 4.54 | 0.01696 | 0.3 | 0.1 | 0.424 | 0.35 | 0.45 | 1.2 | 81.79 |
| Tc-1-1-16 | 7.26 | 0.0219 | 0.3 | 0.1 | 0.5475 | 0.35 | 0.45 | 1.2 | 141.83 |
| Tc-1-1-17 | 4.63 | 0.0134 | 0.3 | 0.1 | 0.335 | 0.35 | 0.45 | 1.2 | 143.62 |
| Tc-1-18 | 5.81 | 0.01199 | 0.3 | 0.1 | 0.3 | 0.35 | 0.45 | 1.2 | 129.59 |
| Tc-1-19 | 6.21 | 0.01199 | 0.3 | 0.1 | 0.3 | 0.35 | 0.45 | 1.2 | 224.95 |
| Tc-1-20 | 5.36 | 0.01199 | 0.3 | 0.1 | 0.3 | 0.35 | 0.45 | 1.2 | 185.67 |
| Tc-1-21 | 3.13 | 0.01199 | 0.3 | 0.1 | 0.3 | 0.35 | 0.45 | 1.2 | 175.77 |
| Tc-2-1 | 15.86 | 0.0300 | 0.3 | 0.2 | 0.300 | 0.2 | 0.4 | 1.1 | 49.99 |
| Tc-2-1 | 15.86 | 0.026 | 0.3 | 0.1 | 0.867 | 0.2 | 0.3 | 0.3 | 200.99 |
| Tc-2-1 | 15.86 | 0.030 | 0.3 | 0.2 | 0.3 | 0.2 | 0.4 | 1.1 | 250 |
| Tc-2-1 | 15.86 | 0.035 | 0.3 | 0.15 | 0.52 | 0.3 | 0.45 | 1.2 | 149.59 |
| Tc-2-1 | 15.86 | 0.025 | 0.3 | 0.15 | 0.37 | 0.3 | 0.45 | 1.2 | 150.41 |
| Tc-2-1 | 15.86 | 0.0321 | 0.3 | 0.1 | 0.8014 | 0.35 | 0.45 | 1.2 | 124.96 |
| Tc-2-1 | 15.86 | 0.025 | 0.3 | 0.15 | 0.368 | 0.3 | 0.45 | 1.2 | 152.9 |
| Tc-2-2 | 1.31 | 0.0155 | 0.3 | 0.1 | 0.387 | 0.4 | 0.5 | 1.35 | 124.11 |
| Tc-2-3 | 2.69 | 0.01199 | 0.3 | 0.1 | 0.3 | 0.35 | 0.45 | 1.2 | 147.04 |
| Tc-2-4 | 8.31 | 0.01199 | 0.3 | 0.1 | 0.3 | 0.35 | 0.45 | 1.2 | 186.12 |
| Tc-2-5 | 4.02 | 0.01696 | 0.3 | 0.1 | 0.4241 | 0.35 | 0.45 | 1.2 | 125.93 |
| Tc-2-8 | 11.85 | 0.0248 | 0.3 | 0.15 | 0.368 | 0.3 | 0.45 | 1.2 | 252.54 |
| TC-2-6 | 17.47 | 0.0240 | 0.3 | 0.1 | 0.791 | 0.2 | 0.3 | 0.3 | 375 |
| TC-2-6 | 17.47 | 0.0260 | 0.3 | 0.1 | 0.8669 | 0.2 | 0.3 | 0.3 | 243.15 |
| TC-2-7 | 28.79 | 0.0438 | 0.35 | 0.1 | 1.2523 | 0.25 | 0.35 | 0.35 | 825.00 |

### Field Canals

As shown in the layout, field canals run across the contours and hence face relatively steeper gradient. The discharge of most of the field canals is very small and this is taken as an advantage to cope up with the relatively steeper gradient. Figure 15 below shows a typical field canal x-section. As much as possible field canals shall be made in fill in order to easily irrigate the adjacent command area. As can be seen from the layout, majority of the filed canals can be used to irrigate both sides of the command area depending on the condition of the individual plots of land owned by individual farmers.

Figure 4-4: Typical Field Canal X-section

## Drainage Canals

### Catch drainage canals

In the layout system there are 3(Nr) Catch drain canals, the designed discharge is determined based on the Catchment area, run off coefficient and rain fall intensity(Rational formula).  The sections of the canals are determined by using manning’s formula, and they are trapezoidal section. The hydraulic characteristics is Presented in Table 14 below.

Figure 4-5: Typical Cross Section of Catch drain canals

Table 4-4: Hydraulic Parameters of Catch drains

| Type of canal | Catchment area(ha) | Design discharge Q(m3/s) | Bed width (m) | Flow depth (m) | velocity (m/s) | Free board (m) | Total canal depth (m) | Top width (m) | Canal length (m) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CD-1 | 3.24 | 0.253 | 0.6 | 0.25 | 1.2 | 0.35 | 0.6 | 1.8 | 104.117 |
|  | 3.24 | 0.256 | 0.4 | 0.25 | 1.6 | 0.15 | 0.4 | 1.2 | 149 |
|  | 3.24 | 0.254 | 0.7 | 0.3 | 0.8 | 0.4 | 0.7 | 2.1 | 125 |
|  | 3.24 | 0.256 | 0.4 | 0.25 | 1.6 | 0.15 | 0.4 | 1.2 | 45 |
| CD-2 | 36.6 | 2.165 | 1.3 | 1.05 | 0.9 | 0.25 | 1.3 | 3.9 | 124.871 |
|  | 36.6 | 2.406 | 1.0 | 0.75 | 1.8 | 0.25 | 1.0 | 3 | 150 |
|  | 36.6 | 2.196 | 1.2 | 0.76 | 1.5 | 0.44 | 1.2 | 3.6 | 250 |
|  | 36.6 | 2.180 | 1.3 | 0.88 | 1.1 | 0.42 | 1.3 | 3.9 | 470 |
| CD-3 | 27 | 1.624 | 0.8 | 0.5 | 2.5 | 0.30 | 0.8 | 2.4 | 149.823 |
|  | 27 | 1.633 | 1.0 | 0.7 | 1.4 | 0.30 | 1.0 | 3 | 92 |
|  | 27 | 1.624 | 0.8 | 0.5 | 2.5 | 0.30 | 0.8 | 2.4 | 97.278 |
|  | 27 | 1.631 | 1.2 | 0.78 | 1.1 | 0.42 | 1.2 | 3.6 | 175 |
|  | 27 | 1.624 | 0.8 | 0.5 | 2.5 | 0.30 | 0.8 | 2.4 | 195.213 |
|  | 27 | 1.631 | 1.2 | 0.78 | 1.1 | 0.42 | 1.2 | 3.6 | 229.217 |
|  | 27 | 1.612 | 0.9 | 0.57 | 1.9 | 0.33 | 0.9 | 2.7 | 325.051 |
|  | 27 | 1.624 | 0.8 | 0.5 | 2.5 | 0.30 | 0.8 | 3.6 | 484.317 |
|  | 27 | 1.633 | 1.0 | 0.7 | 1.4 | 0.30 | 1.0 | 3.6 | 143.428 |

### Tertiary canal drainage canals

Drainage Canals aren't provided for the Tertiary canals, because all of the tertiary canals are entered to gullies and also there is shortage of irrigation water. It is assumed that there is no excess irrigation water during dry season. For the rainy season the water that comes through the tertiary canals directly entered to gullies. So, Gully protection is necessary at the tertiary canal out fall. For the protection gabion is recommended.

## Canal structures design

### Design of a typical flume

Hydraulic Characteristics of the canal

Length of the flume: 47.74m

Shape of the flume:  Rectangular, Roughness coefficient, n =0.014

From the canal longitudinal profiles, u/s canal bed level (CBL) = 1684.063

D/s Canal bed level (CBL) =1683.87m

U/s full supply level (FSL) = 1684.313

D/s Full Supply Level = 1684.12

Total head loss between the inlet & Outlet =0.193m

Water surface change at the u/s and d/s of the flume

Consider inlet and outlet loss coefficients =0.2&0.3 respectively

i. Drop of water surface at the inlet of the flume

=00

ii.  Rise of water surface at the d/s of canal section

=00

iii. Friction head loss, hf

=0.193

Total loss = inlet loss +friction loss + outlet loss = U/s FSL-D/s FSL =0.193m

=0.193m

Where V2= velocity of flow in the flume, V1 = velocity of flow in the canal, L= length of flume

For the other Flume follow the above step,Table 14 below summarizes the hydraulic parameters of the flumes.

Table 4-5:  Hydraulic Parameters of Flume

| Flume name | Flume on | Start/End Chain age | Q | V | L | h | t |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Flume 1 | MC-1 | 0+470.093-480.176 | 0.31869 | 0.8755 | 10.68 | 0.7 | 0.15 |
| Flume 2 | SC-2 | 3449.199-3496.938 | 0.1048 | 1.0484 | 47.7 | 0.4 | 0.15 |
| Flume 3 | SC-2 | 3940.184-3945.871 | 0.07818 | 0.9773 | 5.687 | 0.4 | 0.15 |
| Flume 4 | TC-2-1 | 624.981-630.599 | 0.0351 | 0.52 | 5.618 | 0.3 | 0.15 |

Note: For flume one, the high flood level is higher than the canal bed level, modification of gully is needed. The high flood level is 1725.17m, the inlet and out let CBL of the flume is 1724.731&1724.71m respectively. So, to be safe the OGL of gully should be excavated up to a depth of 1m to lower the high flood level below the canal bed level at the section where the flume crosses the natural gully.

Expansion Joint

Since Flume two is very long which is 47.7m that crosses Quara river, expansion joint is needed. so, at each support which are located at the middle, fabricated plastic is recommended to use as expansion joint. The number of expansion joint needed is 6.

### Design of Division box

At different points of the main and secondary canals division boxes are provided which divert the flow to the secondary canal and tertiary canals. Gate should be provided at the outlet of the boxes. For detail refer the drawing.

Figure 4-6: Typical Division Box plan

Using broad crested formula,

Q= CL (h)3/2

Where; Q= discharge over rectangular weir/intake (opening), m3/s

C = discharge coefficient, c= 1.7

L= effective length of crest form in m

h= over flow depth, m

Assuming equal discharge coefficient & sill height for two or three dividing canals, the proportion becomes.

Q1/ Q2= Q2/ Q3 = L1/ L2 = L2/ L3

Where Q1= is flow in canal 1

Q2 = is flow in canal 2

Q3 = is flow in canal 3

L1= is effective crest length of weir/intake sill across opening to canal 1

L2= is effective crest length of weir/intake sill across opening to canal 2

L3 = is effective crest length of weir/intake sill across opening to canal 3

Q1= CL1 (h)3/2,

L1 = Q1/Ch3/2

L2 = L1*Q2/ Q1

L3 = L1*Q3/ Q1

The depth of (height of) the division box,

D = d + fb

The width of the division box,

B = b + 2*m*D

Where b= base width of the incoming canal

D = total canal depth of the incoming canal

Table 4-6: Hydraulic parameters of Division Boxes

| Division box Nr | Incoming flow (L/s) | Design Discharge to TC or SC (L/s) | Ongoing discharge (L/s) | MC bed width for ongoing discharge (m) | TC/SC outlet width (m) | Top width of division box (m) | Depth of division box (m) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| M.C 1 & TC-1 | 322 | 3 | 322 | 0.70 | 0.30 | 1.50 | 0.70 |
| MC1 & SC1 | 342 | 140 | 202 | 0.40 | 0.50 | 1.50 | 0.70 |
| MC1 & TC-1-1 | 194 | 17 | 177 | 0.40 | 0.30 | 1.20 | 0.40 |
| MC1 & TC-1-2 | 197 | 17 | 180 | 0.55 | 0.30 | 1.35 | 0.55 |
| MC1 & TC-1-3 | 197 | 24 | 173 | 0.40 | 0.35 | 1.35 | 0.55 |
| MC1 & TC-1-4 | 182 | 22 | 160 | 0.40 | 0.30 | 1.20 | 0.40 |
| MC1 & TC-1-5 | 180 | 17 | 163 | 0.40 | 0.30 | 1.25 | 0.45 |
| MC1 & TC-1-6$7 | 169 | 144 | 25 | 0.40 | 0.30 | 1.25 | 0.45 |
| MC1 & TC-1-8 | 144 | 6 | 138 | 0.40 | 0.30 | 1.20 | 0.40 |
| MC1 & TC-1-9 | 151 | 14 | 136 | 0.40 | 0.30 | 1.25 | 0.45 |
| MC1 & TC-1-10 | 134 | 22 | 112 | 0.40 | 0.35 | 1.25 | 0.45 |
| MC1 & TC-1-11$12 | 112 | 19 | 74 | 0.35 | 0.30 | 1.25 | 0.45 |
| MC1 & TC-1-13 | 77 | 6 | 77 | 0.35 | 0.30 | 1.15 | 0.35 |
| MC1 & TC-1-14 | 77 | 7 | 77 | 0.40 | 0.30 | 1.20 | 0.40 |
| MC1 & TC-1-15 | 84 | 22 | 84 | 0.45 | 0.30 | 1.25 | 0.45 |
| MC1 & TC-1-16 | 77 | 22 | 77 | 0.35 | 0.30 | 1.15 | 0.35 |
| MC1 & TC-1-17 | 89 | 13 | 89 | 0.35 | 0.30 | 1.15 | 0.35 |
| MC1 & TC-1-18 | 89 | 12 | 89 | 0.30 | 0.30 | 1.15 | 0.35 |
| MC1 & TC-1-19 | 89 | 12 | 61 | 0.35 | 0.30 | 1.15 | 0.35 |
| MC1 & TC-1-20 | 65 | 12 | 53 | 0.30 | 0.30 | 1.15 | 0.35 |
| MC1 & TC-1-21 | 53 | 12 | 53 | 0.30 | 0.30 | 1.10 | 0.30 |
| SC-2$ TC-2-1 | 128 | 30 | 108 | 0.40 | 0.30 | 1.25 | 0.45 |
| SC-2$ TC-2-2 | 113 | 15 | 113 | 0.35 | 0.30 | 1.15 | 0.35 |
| SC-2$ TC-2-3 | 113 | 12 | 113 | 0.35 | 0.30 | 1.15 | 0.35 |
| SC-2$ TC-2-4 | 113 | 12 | 113 | 0.35 | 0.30 | 1.15 | 0.35 |
| SC-2$ TC-2-5 | 95 | 17 | 95 | 0.45 | 0.30 | 1.25 | 0.45 |
| SC-2$ TC-2-8 | 36 | 25 | 36 | 0.30 | 0.15 | 1.10 | 0.30 |
| TC-2-1$ FC-2-1-1 | 30 | 7 | 30 | 0.30 | 0.30 | 1.10 | 0.40 |
| TC-2-1$FC-2-1-2 | 32 | 43 | 32 | 0.30 | 0.30 | 1.10 | 0.45 |
| SC-2$ TC-2-6 | 102 | 24 | 72 | 0.45 | 0.30 | 1.30 | 0.50 |
| SC-2$ TC-2-7 | 78 | 44 | 36 | 0.30 | 0.35 | 1.20 | 0.40 |

### Design of Drop structure

Since the Secondary canals are aligned across the contour it needs fill which is not recommended. So, to compromise the cut and fill Drop structure is necessarily provided. For more detail refer the Drawing.

### Road crossing structure

2(Nr) road crossing structures (Box type culvert) are provided on the Secondary Canal, at the existing gravel road. The road crossing structures are Box type culverts. For the first Road crossing structure (Box type culvert-1) the slab is reinforced with 14mm @150mmc/c at bottom and 12mm@200mmc/c at the top the length of the slab is 1m which is the same as the respective canal bed width plus Center to center of the canal wall at this section, its width and thickness is 3.6m and 20mm respectively.

For the 2nd Road crossing structure (Box type culvert-2) the slab is reinforced with 14mm @150mmc/c at bottom and 12mm@200mmc/c at the top the length of the slab is 0.8 m which is the same as the respective canal bed width plus Center to center of the canal wall at this section, its width and thickness is 3.28m and 20mm respectively.

### Design of a typical Super passage

The Hydraulic design of the supper passage is similar to Flume as described above. The following Table 4-7 and Table 4-8 describe the hydraulic parameters and canal elevation at each section of the supper passage respectively.

Table 4-7:  Hydraulic Parameters of Super passage

| Name | Super passage on | Start/End Chain age | Q | V | L | h |
| --- | --- | --- | --- | --- | --- | --- |
| Super passage- 1 | SC-2 | 2905.117-2909.972 | 0.096 | 1.197 | 4.855 | 0.4 |

Table 4-8:  Canal elevation at each section

| Section | Location | Water depth (m) | Change in water level |  | Elevation of |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Canal bed | Water surface |
|  | Supper passage-1 |  |  |  |  |  |
| A B C D | u/s canal Super passage Super passage d/s canal | 0.2 0.2 0.2 0.2 | - drop drop rise | - | 1688.007 | 1688.207 |
|  |  |  |  | 0.000 | 1688.007 | 1688.207 |
|  |  |  |  | 0.029 | 1687.977 | 1688.177 |
|  |  |  |  | 0.000 | 1687.977 | 1688.177 |

## Bill of Quantities and Cost Estimate

General Item

Head work

# CONCLUSION AND RECOMMENDATION

- The infrastructure of this project area is designed to irrigate about 240ha of land by taking its supply from the Adirkayina diversion weir irrigation project. The maximum duty of the command area for 14 hours per day irrigation with overall project efficiency of 50%.  The method of irrigation of the project area is furrow surface irrigation in which the main and tertiary canals are working continuously where as the field canals within a tertiary block are working rotational system.

- As the dominant soil type is clay soil, the main canal system is designed to be masonry.

- The reason why the main and Secondary canal is to be lined up to the end is ,the velocity of flow is very high which is not recommended for an earthen canal, to increase the irrigation efficiency since the flow is minimum so that we can avoid loss of water, to avoid the siltation problem, time saving to reach at the tail part, reduce maintenance cost.

- On the secondary unit of the irrigation systems, some are associated with chute. They are designed as far as possible to be partially filled and cut. The layout is designed as far as possible to avoid cross-structures within them.

- The design of the canal dimensions of the irrigation canal is done by applying the manning’s uniform flow equation. The variable of the hydraulic parameters are calculated using iteration or flow master program.

- As soils of the command area are predominantly clay textured; and hence water and soil management measures should be undertaken; and optimum moisture content should be maintained to improve workability of the soil during land preparation and planting time.

The following recommendations are drown:

- For better performance and long service year of the project regular inspection and maintenance is highly required.

- Farmers training, how to operate and maintain the project structures as a whole and available and water resources has a paramount important.

- The irrigation hours per day and per week should be flexible based on base flow amount of each week or month.

- Close supervision of the construction should be made to modify (if need be) each Components of irrigation system based on specific site conditions.

# REFERENCE

- FAO (1977) guidelines for predicting crop water requirements. No 24, Rome Italy

- Design of small Canal structures , USBR

- Soft copies of hydraulic structure publishing

- IDD manual

- ESRDF manual

- Ministry of water resource