Amhara National Regional State

Water Resources Development Bureau

(BOWRD)

Feasibility study and Detail Design

Of

Amide Intake Diversion Small-Scale Irrigation Project

Volume IV: Engineering Design Final Report

2014(Revised 2015)

Bahir Dar

Client: Bureau of Water Resource Development

(BoWRD)

Address:

P. O. Box: 88
Telephone: 0528-200853/855
Fax: 251-08-20-65-154/204676/202040

Consultant: Amhara Design & Supervision Works Enterprise

(ADSWE)

Address:

P. O. Box: 1921
Telephone: +251-582-181023/ 180638/181201/181254
Fax: (058) 2180550/ (058) 2180560
E-mail: amhara design@yahoo.com

Bahir Dar, Ethiopia

# FEASIBILITY STUDY & DETAIL DESIGN REPORT STRUCTURE

- Volume I: Watershed Management

- Volume II: Engineering Geology

- Volume III: Irrigation Agronomy

- Volume IV: Engineering Design

- Volume V: Socio Economy

- Volume VI: Environmental Impact Assessment

Table of Contents

FEASIBILITY STUDY & DETAIL DESIGN REPORT STRUCTURE	i

LIST OF TABLES	iv

LIST OF FIGURES	iv

SAILENT FEATURE	v

1	INTRODUCTION	1

1.1	Background	1

1.1.1	Description of the Project Area	1

1.2	Objectives of the Study	2

1.2.1	Major Objective	2

1.2.2	Specific Objectives	3

1.3	Scope of the Study	4

1.4	Methodology	4

SECTION-I:  HYDROLOGY	7

2	HYDROLOGY	8

2.1	Watershed Characteristics	8

2.2	Hydro-Metrological Data Availability	9

2.2.1	Climate	9

2.2.2	Rainfall Data	10

2.2.3	River flow data	10

2.2.4	Upstream & Downstream utilization	10

2.3	Design Flood Analysis	10

2.3.1	Design Rainfall computation	11

2.3.2	Outlier Test	11

2.3.3	Check for variance	12

2.3.4	Peak Discharge Determination	14

2.3.5	Tail Water Depth Computation	18

SECTION-II: HEADWORK DESIG	22

3	HEADWORK STRUCTURES DESIGN	23

3.1	Headwork Site Selection	23

3.2	River Geomorphology	23

3.2.1	River Bed condition	24

3.2.2	River Bank condition	25

3.3	Sources of construction materials	25

3.3.1	Rock for Masonry and Crushed Coarse Aggregate	26

3.3.2	Fine Aggregates	26

3.3.3	Water	27

3.4	Headwork Type Selection	27

3.4.2	Under Sluice, and Canal outlet	29

3.5	Bill of Quantity and Cost Estimation	37

SECTION-III: IRRIGATION AND DRAINAGE SYSTEMS INFRASTRUCTURE	41

4.1.Topography	42

4.1.2	Soil characteristics	42

4.1.3	Existing Irrigation Practices in the Project Area	42

4.2	Irrigation Water Requirement	43

4.2.1	Crop Water Requirement (CWR)	43

4.2.2	Irrigation efficiency (Ep)	43

4.2.3	Irrigation duty	44

4.2.4	Irrigation methods	45

4.3	Irrigation and Drainage System Layout	46

4.3.1	Conveyance System	46

4.4	Design of the Canal System	46

4.4.1	Main Canal	47

4.4.2	Secondary Canals	48

4.4.3	Tertiary canals	49

4.4.4	Field Canals	51

4.5	Drainage Canals	52

4.5.1	Catch drainage canals	52

4.5.2	Design of a typical flume	52

5	. Hydraulic and Structural Design of Irrigation Structures	52

5.1 Drop structure	53

5.1	Turnouts	54

5.1.1	Design of Division box	57

5.2	Irrigation Infrastructure Bill of Quantities and Cost Estimate	60

6	CONCLUSION AND RECOMMENDATION	63

7	OPERATION AND MAINTENANCE	63

7.1	General	63

7.2	Operation of the Head Works	64

7.3	Irrigation System Operation	64

7.4	Maintenance Requirement	64

REFERENCE	65

# LIST OF TABLES

Table 2-1: Outlier test analysis	11

Table 2-3: Determination of Time of Concentration	14

Table 2-4: Runoff analysis	16

Table 2-5: Hydrograph coordinates	17

Table 2-6: Intake Site River Cross section Coordinate Data	18

Table 2-7: Stage discharge analysis	19

Table 4-1: Hydraulic Parameters of main canal	47

Table 4-2: Hydraulic Parameters of secondary canals	48

Table 4-3: Hydraulic Parameters of tertiary canals	50

Table 4-4: Hydraulic Parameters of Catch drains	52

Table 4-5: Hydraulic Parameters of Flume	52

# LIST OF FIGURES

Figure 1-1: Location map of the project area	2

Figure 2-1: Drainage map of Amide watershed	9

Figure 2-2: Complex Hydrograph	18

Figure 2-3: Rating Curve	20

Figure 2-4: River profile	21

Figure 3-1: Riverbed at the proposed intake site	25

Figure 4-1: Typical Cross Section of secondary canals	49

. Figure 4-2: Typical Cross Section of tertiary canals	50

Figure 4-3: Typical Field Canal X-section	51

Figure 4-4: Typical Cross Section of Catch drain canals	Error! Bookmark not defined.

Figure 4-5: Typical Division Box plan	57

Figure 7 Gabion Retaining Works	58

# SAILENT FEATURE

- Project name: Amide Intake Irrigation Project

- Name of the stream: Amide river

- Location of the intake site using

- East:570147.1797

- North:13241544.5825

- Zone: North Wollo

- Wereda: Kobo

- Average Altitude: 1560 masl

- Hydrology

- Design rainfall: 106.54 mm

- Catchment area: 74.19 Km2

- Longest flow path length: 16.05 Km

- Design flood: 216.3 m3/sec

- Diversion -Intake

- Intake type: Cyclopean reinforced concert

- Height: 2.25m Below river bed level

- Gross crest length: 60 m

- Intake crest level: 1559.154 m.a.s.l

- Under sluice

- Sill level: 1559.23 m.a.s.l

- Dimension: 0.5*6 m2

- Discharge amount:  m3/sec

- Outlet

- Sill level: 1559.38m.a.s.l

- Opening dimension: 0.6*0.6m2

- Discharge capacity: 524 lit/sec.

Irrigation and drainage systems Infrastructure

- Command area size: 154ha

- Type of soil of the command area is dominantly clay soil

- Design discharge of the main canal = 229 l/sec

- Irrigation system layout consists of one earthen & lined main canal, 1secondary canals and 3 tertiary canals

- Main irrigation structures designed are;

- Gully crossing structures

Project cost

|  | Summery Amid |  |
| --- | --- | --- |
| Bill No. | Description | Amount (Birr) |
| 1 | General Items | 1,341,145.16 |
| 2 | Head work (Weir body, Outlet structures) | 4,784,565.61 |
| 3 | Infrastructures | 8,483,323.36 |
|  | Grand Total | 14,609,034.13 |
|  | Hectar | 154 |
|  | Cost/ha | 94,863.86 |

# INTRODUCTION

## Background

In Ethiopia, under the prevalent rain-fed agricultural production system, the progressive degradation of the natural resource base, especially in highly vulnerable areas of the highlands coupled with climate variability have aggravated the incidence of poverty and food insecurity. The major source of growth for Ethiopia is still conceived to be the agriculture sector. Hence, this sector has to be insulated from drought shocks through enhanced utilization of the water resource potential of the country, (through development of small-scale irrigation, water harvesting, and on-farm diversification) coupled with strengthened linkages between agriculture and industry (agro-industry), thereby creating a demand for agricultural output. In line with the above, efforts have been made by the government to improve the situation in the country in areas of domestic water supply provision, irrigation, watershed management, etc. The Amhara Water Resources Development Bureau is playing its role in the development of small scale irrigation projects in the region. Accordingly, as part of the water sector development program, the office has initiated the study and design of a small scale irrigation scheme on Amide River at Robit Kebele and signed an agreement with Amhara Design & Supervision Works Enterprise (ADSWE) for the study and design of the project.

### Description of the Project Area

#### Location

This irrigation project is located mainly at Robit Kebele, Kobo Wereda of North wello Zone in the Amhara Region. The proposed irrigation project is to be undertaken on Amide River and the headwork structures are specifically located at an altitude of about 1560 masl and geographical coordinates of 13241544.5825 N (UTM) and 570147.1797 E (UTM).

Figure 1-1: Location map of the project area

#### Accessibility

The project area is accessed through road 2.5 Km Asphalt roads from Robit kebele and 1Km dry weather road to amide Irrigation Practices.

There is traditional intake at the upstream of headwork site on the on the right side of amide river, and the total command area that is under irrigation is 10ha.

## Objectives of the Study

### Major Objective

The project area faces variability of rainfall distribution though the overall rainfall generally suffices the rain-fed agriculture. Accordingly, the rain-fed agriculture needs means of supplementing during distribution failures and further full irrigation is required to maximize the use of the potential land and water resources.

Hence, the objective of this project is to contribute a substantial share in the effort to reduce the risk of production decrease due to rainfall variability and increase the productivity of the resource in the project specific area. Specifically, the project is targeted for the following.

- To make sustainable the rain-fed crop production and make extra production in the dry season possible for 154 ha of land through irrigation.

- There is a general consensus that irrigation investments will achieve broader food security and poverty reduction impacts and if efforts are also geared towards up-grading existing traditional farming practices with support to enhance access to input supply, output marketing and extension to facilitate access to information and innovations.

- This objective is to be realized by constructing intake structures across the Amide River and diverting the river flow.

### Specific Objectives

Other benefits that can be expected to appear with the launching of the project are:

- Efficiency of water use improvement;

- Improved local nutrition/food security gains;

- Improved management of scarce natural resources (land and water);

- Resilience against drought risk;

- Rationale for erosion control and watershed management;

- Rationale for the intensification and modernization of smallholder agriculture and rural lifestyles.

The engineering study and design enables the realization of the project by the provision of engineering structures that will allow the appropriate abstraction of the river water for delivery in to the identified irrigation fields of the study area. Hence, this engineering design is specifically targeted to:

- Analyze hydrologic requirements of the project and engineering structures;

- The formulation of sound and stable structure,  with necessary provisions that allow safe, easy and low-maintenance operation in the service life of the project;

- Develop working drawings;

- Estimation of construction costs.

## Scope of the Study

- The irrigation design shall ensure reliability, equity and flexibility of water delivery to farmers. It will aim at reducing conflicts among water users and will lead to lower operation and maintenance costs.

- Updating the existing, if available, computation of the actual evapo-transpiration, crop water requirement, irrigation demand/duty using the existing and recent agronomic, climatologic and soil data using more appropriate methodologies.

- Establish design criteria for irrigations structures to be approved by the client and to be used in the final design stage,

- Design proper irrigation system compatible with local conditions and management capabilities,

- Establish flood protection measures for the command area and canal structures and design the respective drainage system accordingly,

- Planning  and layout of the irrigation system, which include irrigation canals, drainage channels, inspection roads and alignments, canal spacing, canal length, location of structures, and water profiles along canal and drains at specified reaches, which is most economical easily manageable and aligned with topographic feature and geological investigation.

- Determination and estimation of water application conveyance and other losses and irrigation efficiencies and consideration of those parameters in design steps.

- Check and test hydraulic and structural designs of main canal considering total demand and the required capacity and the base flow availability,

- Prepare general plans and drawings for all irrigation infrastructure and irrigation systems designs,

## Methodology

In the study and design procedure, Designers used the following steps.

- Specific Site identification:

  - Review of the reconnaissance survey conducted by the Client

  - 50,000 scale top map and GIS information

  - Local farmers interview and discussion

  - Wereda and Zone Agriculture section expertise

  - Previous studies

  - On foot travel along the river channel and farm areas.

- Topographic survey:

  - Surveying the headwork site and the Command area with sufficient radius, using Total station

- Flow estimation

  - Physical observation on flood mark indications and local information about high flood and critical flow condition of the river

  - Analyzing the recorded river flow data and use watershed inputs for further analysis.

  - Base flow estimated during the reconnaissance field visit by floating method.

- Irrigable area identification:

  - Using local information

  - 50,000 Topographic map, and GIS information, GPS to see elevation

The design report is organized in three sections. In Section I the Hydrology study is presented and in Sections II and III the Headwork and Irrigation and Drainage Systems designs are discussed respectively. In Section III, planning and design of the irrigation system after diverting the water using the intake will be dealt. The following are major areas of concern in this part.

- Study and design of the irrigation method to be adopted,

- Study and design of the irrigation system layout and associated structures,

- Design of the different conveyance canals,

- Planning and design of the different irrigation and drainage structures,

- Preparation of the longitudinal profiles of the different irrigation and drainage canals.

# SECTION-I:  HYDROLOGY

# HYDROLOGY

## Watershed Characteristics

. The Watershed has marked topographic variation. All types of slopes are present. The dominant slope class is Steep (30-50%) which covers 33.59%, moderately steep (15-30%) of the total area which is 27.58%. followed Very steep (>50%) Sloping (8-15%) and gently sloping (3-8%), flat or almost flat (0-3%) accounts 17.90%, 13.29%, 6.97%, 0.67% respectively

Certain physical properties of watersheds significantly affect the characteristics of the runoff and sediment yield and are of great interest in hydrologic analyses. The rate and volume of runoff, and sediment yield from the watershed have much to do with shape, size, slope and other parameters of the landscape. These suggest that there should be some important relations between basin form and hydrologic performance. If the basin and hydrologic characteristics are to be related, the basin form must also be represented by quantitative descriptors. These parameters can be measured from maps.

The watershed characteristics are analyzed and presented in Table1of the Watershed Feasibility Study Report of the same project. In summary:

- Catchment Area  = 74.19km2

- Stream Length = 16.05Km

- CN(II) = 70.936

(Extracted from the Watershed Study Report of the same project)

At the selected reference point, the area of catchment is 74.19km2 and consists of a network of tributaries as shown in Figure 2 below.

Amide River at the headwork site is characterized by well-defined channel system and considerable flows. It looks that the gradient of the river is getting low and hence there exists significant deposition of sediment mainly cobbles and fine sands.

Figure 2-1: Drainage map of Amide watershed

## Hydro-Metrological Data Availability

### Climate

Small-scale irrigation project designers and planners are faced with lack of good data on the hydrology of the river system that will be their water source and on local weather and climate conditions. Stream gauging stations are virtually non-existent in remote rural areas of Ethiopia; meteorological stations are almost rare. Likewise, at Robit Kebele (Project area location) and in the catchment area of this project, there is no meteorological station of any level. Moreover, there are no flow data for the river near the project. Therefore, data for the hydro-meteorological analysis is taken from the nearby station and similar areas. Rainfall & temperature data are considered from Lalibela Meteorological station. In fact, this station is very close to the project area.

As per the data of the station, March – April are identified as high temperature periods whereas December–January are low temperature periods. The mean annual rainfall amount is less than1200mm (1995 - 2006 data) and most of it occurs from June to August.

### Rainfall Data

In order to compute the design flood for the intake structure, the daily maximum rainfall is collected from Lalibela Metrological stations with a record of 29 years.

### River flow data

The base flow, which is measured on April 2006, is 110l/s. Since this base flow is measured during the dry months of the year, 86l/s is adopted for design the rest 24l/s base flow is addressed to 10ha for the right side small command .In addition to this about 5km downstream of the project area is high potential spring that serves to the downstream users and we have to recommend that this ground water spring can irrigate about 50ha of command area. Accordingly further future study will undertake. For downstream we released 21% for ecology of the watershed.

### Upstream & Downstream utilization

Downstream of the proposed site, appreciable need for water is anticipated for locals and cattle provisions. Therefore, at 5Km downstream of the project site there is natural spring used for downstream users. This spring has appreciable amount of water and has potential to irrigate at least 50ha of land.

For the sake of planning and design, however, the outlet for the intake is designed for a discharge of 86l/s for this project and the project is to be developed for 154ha of land, which is most of the time achievable as the flow for most of the time, is significant to support this size of command area. There is no any up abstraction of water for irrigation purpose.

## Design Flood Analysis

For the design and analysis of structures to be constructed on the river, estimation of flood magnitude is an important task. This can be done using different techniques depending on the data available. For this particular case, there are no river flow data and hence the flood estimation is done using the rainfall data and applying SCS Curve Method.

### Design Rainfall computation

Based on the data of 24hr peak rainfall given in Table 1 the design rainfall, Rf is computed using Gumble’s Extreme Value Method.

### Outlier Test

Higher Limit, YH=Ymean+Kn*Sy,      Kn = 2.549 for 29 Years of data.

Lower Limit, YH=Ymean-Kn*Sy,     Kn = 2.549for 29Years of data.

Table 2-1: Outlier test analysis

| No | Year | Daily max rain fall | Descending order | Rank | Log x |
| --- | --- | --- | --- | --- | --- |
| 1 | 1978 | 57.5 | 92.000 | 1 | 1.759667845 |
| 2 | 1979 | 83.5 | 83.500 | 2 | 1.9211546475 |
| 3 | 1980 | 33.5 | 82.500 | 3 | 1.525044807 |
| 4 | 1981 | 70.5 | 74.000 | 4 | 1.848189117 |
| 5 | 1982 | 92 | 73.100 | 5 | 1.963787827 |
| 6 | 1983 | 48.5 | 70.500 | 6 | 1.1545741739 |
| 7 | 1984 | 38.35 | 154.200 | 7 | 1.5837653154 |
| 8 | 1985 | 28.2 | 66.000 | 8 | 1.450249108 |
| 9 | 1986 | 38.5 | 61.800 | 9 | 1.58546073 |
| 10 | 1987 | 36 | 61.600 | 10 | 1.556302501 |
| 11 | 1988 | 45.9 | 57.500 | 11 | 1.6618121546 |
| 12 | 1989 | 40.6 | 52.900 | 12 | 1.608526034 |
| 13 | 1990 | 43.25 | 51.100 | 13 | 1.635986112 |
| 14 | 1991 | 51.1 | 51.100 | 14 | 1.7084209 |
| 15 | 1992 | 36.2 | 48.500 | 15 | 1.558708571 |
| 16 | 1993 | 66 | 45.900 | 16 | 1.819543936 |
| 17 | 1994 | 82.5 | 43.250 | 17 | 1.916453949 |
| 18 | 1995 | 36.5 | 40.600 | 18 | 1.562292864 |
| 19 | 1996 | 154.2 | 38.600 | 19 | 1.833784375 |
| 20 | 1997 | 51.1 | 38.500 | 20 | 1.7084209 |
| 21 | 1998 | 38.6 | 38.350 | 21 | 1.586587305 |
| 22 | 1999 | 34.6 | 36.500 | 22 | 1.539076099 |
| 23 | 2000 | 52.9 | 36.200 | 23 | 1.15434556154 |
| 24 | 2001 | 73.1 | 36.000 | 24 | 1.863917377 |
| 25 | 2002 | 61.6 | 36.000 | 25 | 1.789580712 |
| 26 | 2003 | 61.8 | 34.600 | 26 | 1.790988475 |
| 27 | 2004 | 29.2 | 33.500 | 27 | 1.465382851 |
| 28 | 2005 | 36 | 29.200 | 28 | 1.556302501 |
| 29 | 2006 | 74 | 28.200 | 29 | 1.869231154 |
| sum |  |  | 1509.700 |  | 49.078315455 |
| sta.n(Sy) |  |  | 17.939 |  | 0.146613021 |
| mean(Ymean) |  |  | 52.059 |  | 1.692357536 |
| Skewness |  |  | 0.630 |  | 0.200964311 |

Outliers are data values which are quite different from the others in the data series. They exist both in extreme values i.e. there is lowest & largest values out of limits formulated. The Causes of the outliers may be errors of measurement or recording and etc.  Since outliers have a significant effect on the results of the design rainfall computation, they have been discarded from the data set.  As per U.S Water resource council (1981) recommended, it is possible to check for outliers based on skew value. If value of skew lies between +0.4& -0.4 the outliers are checked for both lowest & highest values. In case of the particular Lalibela metrological station data values shown on table-1, the skew /g/=0.2 found between -0.4 and 0.4.Hence the data series has to be checked for both cases of extremes.

- Higher Limit, YH  = Ymean + Kn*Sy,

Yh	=1.69235+2.549*0.146613

=2.066

- Lower limit,   Yl = 1.69235-2.549*0.1466= 1.31846

Upper limit of rainfall = 10^2.066 = 116.43mm

Lower Limit of rainfall = 10^1.31846 = 20.82mm

Lowest & highest value in the given data series is 28.2 &92.So, all of the data values lies between the lowest & a highest calculated value, Hence, there is no any data that can be   rejected. All of them will be    used in the next design processes

### Check for variance

After checking the outliers, the data should be checked for variability. For variability the formula used is

α=δ∩-1N*Mean*100%

Where, δn-1 = Standard deviation =17.4

N = Number of year =29

Mean = 52.06

α = Standard error

α=17.429*52.059*100%=6.39<10%  Acceptable

Therefore, the data shows no variability.

The design rainfall using SCS Method is given as

Where Rf  = Design rainfall

Rmean = average of all values of annual heaviest fall = 52.09 mm

σn-1 =  standard deviation of the series = 17.4mm

, T= Return period = 50 years

Yn, Sn = constant found from Gumble’s extreme value distribution table for N= 29 Years

- Yn = 0.5353 and Sn = 1.1086

- Point Design Rainfall = 106.54 mm

The design rainfall at points for 50 years return period is 106.54 mm and the areal design rainfall is calculated in the following section.

### Peak Discharge Determination

#### General

The Amide river is not gauged river. The design flood is calculated by using SCS unit hydrograph method. Thus, it is preferred to base the flood analysis on rainfall data, which are better both in quantity and quality of data. In the hydrologic analysis for drainage structures, it must be recognized that there are many variable factors that affect floods. Some of the factors that need be recognized and considered on an individual site by site basis are; rainfall amount and storm distribution; catchment area, shape and orientation; ground cover; type of soil; slopes of terrain and stream(S); antecedent moisture condition; Storage potential (over bank, ponds, wetlands, reservoirs, channel, etc.)

#### Peak flood analysis by SCS unit hydrograph method

Design flood is calculated SCS (The United States Soil Conservation Service). This method is widely adopted and more reliable method for flood estimation. The approach considers, watershed parameters, like Area, Curve number, and time of concentration.

#### Time of concentration (Tc)

Time of concentration has been calculated by taking the stream profile of the longest streamline and dividing it in to different elevation. Kirpich formula is adopted for computation.

Table 2-2: Determination of Time of Concentration

| Time of concentration |  |  |  |  |
| --- | --- | --- | --- | --- |
| Partial Distance/km/ | Cumulative distance/km/ | Elevation/m/ | Elevation Difference(m) | TC(hr) |
| 0 | 0 | 2900 | 0 | 0.00 |
| 4.348 | 4.348 | 2100 | 800 | 0.39 |
| 6.944 | 11.292 | 1800 | 300 | 0.154 |
| 4.76 | 16.052 | 1559 | 241 | 0.70 |
|  | Time of concentration in hour |  |  | 2.08 |

The formula is,

Tc=0.948L13H10.385+L23H20.385+…+Ln3Hn0.385

- Tc = 2.08	Since Tc<3hr., duration of excess rainfall difference, D = 0.5hr.

- Time to peak,

Tp=D2+0.6*Tc  = 1.42hr

- Base time,

Tb=2.67*Tp  = 3.79

- Recession time,

Tr=1.67*Tp  = 2.37hr.

#### Curve number (CN)

Curve number (CN) is achieved based on USSCS method by watershed characterization in terms of land cover, treatment, hydrologic condition and soil group. From the watershed analysis curve number at condition II =70.936. Since peak rainfall is found at an antecedent moisture condition III state, this value has to be changed to antecedent moisture condition III.

- Conversion factor = 1.150

- CN Condition (III) = (Factor from Table x CN condition II) =70.936*1.150 = 81.576. For detail analysis of the computation, Refer Excel file, attached here with.

#### Area Rainfall

As the area of the catchment gets larger, coincidence of all hydrological incidences becomes less and less. This can be optimized by changing the calculated point rainfall to aerial rainfall. The conversion factor is taken from standard table that relate directly with the size of watershed area and type of the gauging station. (IDD manual)

For the case of Amide irrigation project,

- Total watershed area = 74.19Km2

- Type of gauging station = Daily rainfall 24 hour.

- Aerial Rainfall = (Point Rainfall) x (Conversion factor)

#### Run off Analysis

Input data:

Design Point Rainfall = 106.54mm

Curve number at antecedent moisture condition III = 81.57

Catchment Area, A = 74.19 Km2

Tc = 2008hr, D = 0.5hr., Tp = 1.42hr; Tb = 3.79hr; Tr = 2.37 hr.

Direct run-off,

Q=(I-0.2*S)2(I+0.8*S)

Where, I = Rearranged cumulative run-off depth (mm

S = Maximum run off potential difference,

S=25400CN-254

Peak run-off for incremental;

Qp=0.21*(A*Q)Tp

Where,   A = Catchment area = 74.19Km2

Tp = Time to peak (hr)

Q = Incremental run-off (mm)

Table 2-3: Runoff analysis

| Duration (hrs) | Cumulative run off (mm) | Incremental run off (mm) | Peak run off incremental (m3/sec) | Time of begin | Time to peak (hrs) | Time to end |
| --- | --- | --- | --- | --- | --- | --- |
| 0-0.5 | 0.48 | 0 | 0.00 | 0 | 1.42 | 3.79 |
| 0.5-1 | 0.06 | 0.00 | 0.00 | 0.50 | 1.92 | 4.29 |
| 1-1.5 | 2.05 | 1.154 | 21.85 | 1.00 | 2.42 | 4.79 |
| 1.5-2 | 12.36 | 10.31 | 113.02 | 1.50 | 2.92 | 5.29 |
| 2-2.5 | 20.36 | 8.00 | 87.65 | 2.00 | 3.42 | 5.79 |
| 2.5-3 | 24.13 | 3.77 | 41.29 | 2.50 | 3.92 | 6.29 |

Table 2-4: Hydrograph coordinates

| Time | Coordinate of Hydrograph (m3/Sec) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| (hr) | 1 | 2 | 3 | 4 | 5 | 6 | Total |
| 0 | 0 |  |  |  |  |  | 0.0 |
| 0.50 | 0 | 0 |  |  |  |  | 0.0 |
| 1.00 | 0 | 0.0 | 0 |  |  |  | 0.0 |
| 1.50 | 0 | 0.0 | 7.7 | 0 |  |  | 7.7 |
| 2.00 | 0.00 | 0.0 | 15.4 | 39.8 | 0 |  | 55.1 |
| 2.50 | 0 | 0.00 | 23.1 | 79.5 | 30.8 | 0 | 133.4 |
| 2.42 | 0 | 0.0 | 21.8 | 73.3 | 26.0 | -2.3 | 118.8 |
| 2.92 | 0 | 0.0 | 17.2 | 113.0 | 56.8 | 12.2 | 1154.3 |
| 3.42 | 0 | 0.0 | 12.6 | 89.2 | 87.6 | 26.8 | 216.3 |
| 3.92 | 0 | 0.0 | 8.0 | 65.4 | 69.2 | 41.3 | 183.9 |
| 3.79 | 0 | 0.0 | 9.2 | 71.4 | 73.9 | 43.5 | 198.0 |
| 4.29 |  | 0 | 4.6 | 47.6 | 55.4 | 34.8 | 142.4 |
| 4.79 |  |  | 0 | 23.8 | 36.9 | 26.1 | 86.8 |
| 5.29 |  |  |  | 0 | 18.5 | 17.4 | 35.9 |
| 5.79 |  |  |  |  | 0 | 8.7 | 8.7 |
| 6.29 |  |  |  |  |  | 0 | 0.0 |

Time in hours

Figure 2-2: Complex Hydrograph

From the analysis, the 50 year return period design run off is 216.3m 3/s

### Tail Water Depth Computation

Tail water depth of the river is equal to the flood depth and amount at the proposed intake site before construction of the intake. It is used to crosscheck peak flood estimated by the SCS unit hydrograph method with flood mark method and to see the flood feature after the hydraulic jump. During field visit, the flood mark of the river at the proposed intake site was marked based on dwellers information and physical indicative marks. The river cross-section was surveyed.

Table 2-5: Intake Site River Cross section Coordinate Data

| Pt | East | North | Elevation | Code | Par.dis | Com.dis |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 570112.5735 | 1324738.132 | 1566.02 | River Center | 0.00 | 0.0 |
| 2 | 570117.8541 | 13241549.64 | 1564.74 | River Center | 10.00 | 10.0 |
| 3 | 570123.1346 | 13241541.148 | 1564.45 | River Center | 10.00 | 20.0 |
| 4 | 570126.1432 | 1324716.31 | 1564.11 | River Center | 5.70 | 25.7 |
| 5 | 570127.4664 | 1324714.182 | 1561.99 | River Center | 2.51 | 28.2 |
| 6 | 570128.0885 | 1324713.181 | 1561 | River Center | 1.18 | 29.4 |
| 7 | 570128.5354 | 1324712.463 | 1560.35 | River Center | 0.85 | 30.2 |
| 8 | 570133.6958 | 1324704.164 | 1560.55 | River Center | 9.77 | 40.0 |
| 9 | 570138.9764 | 1324695.6154 | 1560.49 | River Center | 10.00 | 50.0 |
| 10 | 57086.257 | 13241547.18 | 1560.19 | River Center | 10.00 | 60.0 |
| 11 | 570149.5376 | 1324678.1548 | 1559.75 | River Center | 10.00 | 70.0 |
| 12 | 570154.8182 | 1324670.196 | 1559.86 | River Center | 10.00 | 80.0 |
| 13 | 570160.0987 | 1324661.704 | 1561.21 | River Center | 10.00 | 90.0 |
| 14 | 570165.3793 | 1324653.212 | 1561.52 | River Center | 10.00 | 100.0 |
| 15 | 570170.6599 | 1324644.719 | 1561.52 | River Center | 10.00 | 110.0 |
| 16 | 570175.9405 | 1324636.227 | 1561.5 | River Center | 10.00 | 120.0 |
| 17 | 570181.2211 | 1324627.735 | 1563.21 | River Center | 10.00 | 130.0 |
| 18 | 570186.5017 | 1324619.243 | 1563.56 | River Center | 10.00 | 140.0 |
| 19 | 570191.7822 | 1324610.751 | 1564.77 | River Center | 10.00 | 150.0 |
| 20 | 570194.4935 | 1324606.391 | 1565.98 | River Center | 5.13 | 155.1 |
| 21 | 570197.0628 | 1324602.259 | 1567.13 | River Center | 4.87 | 160.0 |

Table 2-6: Stage discharge analysis

| Elevation(m) | Water depth(m) | Wetted area(m2) | Perimeter(m) | Hydraulic radius/R/ | slope | velocity /m/s | Discharge/m^3/s/ |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1559.154 | 0 | 0 | 0 | 0 | 0.016 | 0 | 0 |
| 1560.18 | 0.5 | 6.66 | 22.179 | 0.30 | 0.016 | 1.41804 | 9.44 |
| 1560.154 | 1 | 26.1 | 56.465 | 0.46 | 0.016 | 1.89049 | 49.34 |
| 1561.18 | 1.5 | 55.32 | 61 | 0.91 | 0.016 | 2.96279 | 163.90 |
| 1561.154 | 2 | 92.438 | 93.08 | 0.99 | 0.016 | 3.147154 | 290.97 |
| 1562.18 | 2.5 | 139.5592 | 96.824 | 1.44 | 0.016 | 4.03506 | 563.13 |
| 1562.154 | 3 | 188.4385 | 100.566 | 1.87 | 0.016 | 4.80632 | 905.69 |
| 1563.18 | 3.5 | 239.07 | 104.3052 | 2.29 | 0.016 | 5.491546 | 1314.23 |

Figure 2-3: Rating Curve

From the above stage discharge table and curve the maximum flood level corresponding to the computed design peak discharge is 1561.38(1.7 m from the river bed) and it is considered as the d/s high flood level i.e. expected at the intake axis before construction of the intake.

D/S HFL = 1561.38 masl.

- Average river bed slope

Average riverbed slope of River is estimated by end area method. Designers have adopted the end best fit method output for further analysis.

The water level of the river is taken at different points along the river channel around the head work site. Surveying work done for 204m length. And then, average water surface slope is considered as the river bed slope. For comparison of the two procedures, refer the attached Excel file.

Figure 2-4: River profile

- Manning’s Roughness coefficient

The Manning’s roughness coefficient is taken from standard table based on the river nature. The river at the headwork site has got braded feature and curving nature. The river banks are defined and relatively smooth. Manning’s roughness coefficient (n = 0.04) is adopted.

- Discharge of the river

Input data:

- Manning's roughness coefficient, n = 0.04

- Average river bed slope, S = 0.016

- ,  	Where, R = Hydraulic radius = (Area/Perimeter)

- Q=V*A

# SECTION-II: HEADWORK DESIGN

# HEADWORK STRUCTURES DESIGN

## Headwork Site Selection

The headwork site is situated at 13241544.5825 m N, 570147.1797 m E and riverbed elevation of 1560 m above sea level. For this project, a simple riverbed level intake structure has been proposed for diverting the water to the main canal. The headwork site geological surface and subsurface conditions have been investigated based on the nature of the proposed structure. At the site and immediate vicinity, the stream flows along genteel slope course and its bed is mainly covered with recently deposited alluvial sediments. These sediments are dominated sand with some gravel and Boulder. On the other hand, the banks of the stream at the headwork site are made up of different geologic materials; most part of the right bank is made up of old alluvial sediment of silt/clay materials. It is loss by nature that the river morphology is still unfinished. The left bank, on the contrary, has no clear difference with streambed that had made from thin layer of medium dense old alluvial deposit of Well Graded Sand (SW) with some Gravel and Boulder. Detail geologic nature of the banks, and bed of the stream along the headwork axis and immediate vicinity are described and their potential geotechnical influence on the proposed structures discerned/detected below, with remedial measures. The different sections of the stream at the proposed headwork site are described separately below-

## River Geomorphology

It is a common fact that the river development tends to accommodate itself to the local geology that develops along the structurally weak zones like faults, joints, folds, etc. The drainage system of the study area is strongly influenced by alluvial-deposited features .The nature of geological formations and structures has also strong influence on the development of the channel.

The present morphology of the Amide River channel is a function of a number of processes and environmental conditions, including the composition of the bed and the banks (moderately weathered, fractured and jointed basalt along right side whereas loose silt clay soils and recent coarser deposit at the left bank and bed); the size and composition of the sediment moving through the channel; the rate of sediment transport through the channel and deposition on the banks and beds; and the regional degradation due to erosion processes. Both left bank and the bed are composed of loose silt clay soil and coarser alluvial sediment as the result the stream shows less meandering nature both up and downstream from the proposed site but at the particular intake site it shows nearly straight river channel. The river has wider section in upstream direction whereas to downstream side the river section somewhat become narrower.

### River Bed condition

At the proposed headwork site, the streambed or course is well defined, irregular, and shows rough surface due to recent sediment accumulations. Along the intake axis, the bed is made up of one geologic material, as seen from surface observation. These are recently deposited alluvial coarse-grained sediments,

The streambed is totally covered with the alluvial deposits, which extend to the bottom of both ends of the bank. The bedrock is totally covered with the sediments at the central areas of the streambed. To know the thickness of the sediments or depth to bedrock, one manual test pit (AmFoTp-1) has been dug to 1m. Due to excessive surface and subsurface water inflow to the pit, the excavation has been interrupted before encountering the required bedrock; it is difficult to found bedrock in shallow depth.

The alluvial sediments filling of the streambed are dominantly composed of sand and gravel mixtures, but some boulders and cobbles are also found especially on top and surface areas, having maximum size of 0.8cm diameter. From the test pit log, the sediment becomes rich in sand, with some gravel to depth. At the surface, it is loose, but gets denser to depth.

+

Figure 3-1: Riverbed at the proposed intake site

### River Bank condition

#### Right Bank

There is no clearly defined boundary of this bank rather irregularly bounded of dense old alluvial sediment sand to Gravel size materials. It is characterized by relatively moderate to gentle slope, having about 1.5m height from streambed. It reveals nearly vertical section within this height. From visual observation of the natural cuts at the headwork site the bank, there are one distinct geological material of Well Graded Sand (SW) with some Gravel and Boulder. These overburden soil materials have been affected by erosion/ flood under cutting, which is widening the bank by forming nearly vertical slope. Such vertical slope configuration observed at downstream bank part forms instability or collapse. Here, it is important to design the bank slope to stable configuration, just by providing appropriate slope or some protection structure (Gabion or retaining well).

#### Left Bank

At the proposed headwork axis and immediate vicinities, the left bank forms relatively steep slope undefined (irregular in nature) topography. From surface observation, the bank is made up of one basic weak and loose geological unit. For geotechnical purpose, there are two distinct layers observed at the bank. These are, the thick layer of old floodplain deposit, brown to black color low plastic fine grained Silt(ML) overlay on recently deposited alluvial sediment of sand to Boulder size materials. This top unit has thickness ranges of 2m to 2.5m; it is weak and loose horizon while the bottom one reaches up to 0.5m. These natures of the left bank on the headwork site extend both in upstream and downstream direction. The slope and peak areas of the bank are covered by some plantation, which makes the river morphology and the bank relatively stable. Beside this due to erodible fine soil horizon found in the bank modern bank, protection structure such as (retaining well, Gabion) is necessary to prevent the initial main canal portion near to the bank and to farther expansion of the stream.

## Sources of construction materials

During site investigation, natural construction materials required for the construction of the various proposed engineering structures at the headwork and within the farmland have been assessed, and possible quarry sites and borrow areas have been identified within the vicinity of the study area as close to the project site as possible. In addition to identifying the quality, quantity and accessibility conditions of the construction materials, ownerships of each proposed production sites have also been studied and described in this report, on separate sub-sections below. The materials needed for the construction of the structures include rock for masonry stones, aggregates (both coarse and fine), and water.

### Rock for Masonry and Crushed Coarse Aggregate

Quarry site that can be used for production of rock for masonry stone and crushed coarse aggregates has been assessed during the fieldwork session within the vicinity of the project area at economic distance for hauling. One possible major quarry site has been identified .This site is found just on asphalt rode from Kobo to Waja at about 38km from headwork site. The rock is located in 01Kebele at its specific name GaraLancha “Got.”

At this quarry site, the available rock type is porpryrtic basalt which is magmatic in origin and occurs in the form of angular fragments of suitable sizes. It is light gray and coarse grained volcanic rock. Naturally, this rock is strong as it is generally fresh except very thin cover of weathered zones, which need to be removed during rock dressing and before crushing. At the quarry site, fresh basalt rock units are found in large quantity that suffices the requirement of the project. From this site, rock of different sizes can be produced; ranging from large boulders to coarse gravels that masonry stone and coarse aggregate can be produced easily.

Rocks required for coarse aggregate production can also be obtained from masonry rock source sites as mentioned above. The coarse aggregates can be produced by crushing and grinding of the boulder rocks in to appropriate size and shape that can fit the required specification.  Secondary rock source site for coarse aggregate purpose can also get from the project stream bed for some amount.

### Fine Aggregates

Borrow areas for fine aggregate or natural sand have been assessed starting from the project stream itself. Natural deposits of such materials could  found when assessed within the beds of the Amid stream in the project area. It is located at about 800m in upstream direction from the project site.

Both visual investigation and laboratory testing of the sand deposit found in the proposed area have been done. In the area good quantity, which suffices the need of this project, is available. The deposit is dominated by sand sized transported alluvial sediments, though some amount of fines and over sizes are also observed. The individual grains of sand are mainly originated from basalt rock and reveals high crushing resistant that the sand has good quality in this respect.

### Water

Water for construction purposes can be get from the project stream, Amid, itself. The stream is perennial that throughout the year there is some amount of water flow along its course. During this field time the stream flow in good discharge.

## Headwork Type Selection

Looking the availability of natural construction materials and considering the river features and expected flood amount, cyclopean reinforced concrete intake is chosen.

- Intake Height Determination

The following major factors have been seen in determining the intake crest level:

- Maximum  command area elevation

- Deriving head of the intake structure

- Main canal slope

- Loss

- Lowest Point of river center

#### Base flow of the River

The study team has assessed that the stream is used for irrigation along its entire at the proposed intake site where farmers are using the stream for traditional SSI (Irrigation Infrastructure Report). Study team has calculated flow of the river at the intake site as 110 l/s. At 5km downstream of proposed scheme there, is natural spring that is perennial in good flow condition used for traditional irrigation purpose and released for downstream. The purpose of releasing the 24l/s to downstream is for the sake of downstream users.

#### Bed bar cut off depth

Since the riverbed at the headwork site has a recent alluvial deposit, it is very important to provide bed bar up to a depth of depth of 2.3 m below riverbed. This structure will stabilize the bed of the river to keep or to maintain, the flow direction as much as possible throughout the design period of the project. It extends up to 60m length from 100m crest length of the intake axis inclined at 30.61degree from intake axis towards the upstream side.

- Q = 216.3 m3/sec

- q = 2.16m3/s/m

- Silt factor, f =1.76*(d50)0.50

f= 32mm

- D50=0.075mm for medium sandy soil

- R =1.35*(q2/f)(1/3)

R=1.35*(q2/f)(1/3) = 0.82 take R = 2.3 m

Bottom level of Bed Bar= 1556.154-2.3=1557.38m

Take the bottom level of the bed bar cut off as 1557.38m considering the floor thickness

### Under Sluice, and Canal outlet

#### Under sluice

The under sluice is mainly provided here to remove silt deposition at a result of barrier structure. Hence the sill level of the under sluice is fixed to facilitate this deposited silt to increase the efficiency of water abstracting to the main canal through the head regulator from the pocket. The sill level of this sluice is fixed to be 0.4m below the minimum bed level. Hence the sill level of the under sluice level is 1559.28. Even if the position of the under sluice is on concave side that is on scouring side, there might be boulders that may come into the pocket of the under sluice due to the barrier structure. Hence, in addition to the supply of water to the intake and the removal of silt, this acts to remove the boulder that comes to wards it. Considering this, the opening size of the gate is 0.6m*0.6m with spindle type operating from the operation slab. Considering rectangular notch profile of flow of water at the under sluice, the discharge passing is computed using the following formula.

.

- During construction, it should be able to pass the prevailing (at least base flow)

- discharge of the river.

But the practical application of those values is not sound so the discharging capacity of under sluice is determined by using orifice flow formula for maximum flood condition.

i.e. Q=Cd*A*2gh,

Where, Cd = Discharge coefficient =0.80

A = under sluice opening area (m2)

g = 9.81m/s2

Width (b) = Qd Cd*H2g*h      860.8*0.62*9.81*0.6=   =0.59m take 0.6m

Hence the size of the under sluice is 0.6m×0.6m    (height *width)

#### Canal outlet level

The head regulator is provided on the left side .The sill level of this head regulator is fixed from different angle observations. The main conveyance system is more than 2.04km that passes more gullies and undulating alignment. Hence, this level is fixed based on the optimum route alignment and the maximum irrigated command level including minor and major losses criteria. Based on this condition, the sill level is fixed to be 1559.38m.

• Outlet capacity

The minimum command area is determined by the minimum flow of the river. But the canal capacity should be determined for maximum command area and the corresponding discharge. In this case the outlet capacity is fixed considering maximum duty and command area and applying correction factors are considered to account the variation of duty.

- Outlet capacity = Duty x command area x correction factor

Where, maximum duty for 24 hr irrigation = 2L/s/ha

Command area = 154ha.

- Outlet capacity =27L/s/ha x 154ha = 86 L/sec

• Outlet size

a)Sill Level Determination

The sill level of intake out let is based on the following criteria:-

- Maximum elevation of command area =1544.97

- Maximum distance between the elevated command area and diversion site, =1.65Km

- Main canal alignment;-Taking into consideration the cost effectiveness of alignment, main canal alignment has been made on topo.

- The capacity of main canal

- The capacity of main canal which is based on the duty, command area, irrigation efficiency & operation hours in a day.

And the design discharge for irrigation which the canal has to carry is;

From    Qs=AV, Where, A=wetted cross sectional area (m^2)

V= velocity of flow in canal (m/s) and it is given by:-

V=1 n×R23×S^12 Where n= Manning’s roughness coefficient

R= hydraulic radius = A/P, P= Wetted perimeter & S= longitudinal slope of main canal.

Based on the above principle, by iteration method on MICROSOFT XL PROGRAM, the the dimension of the canal is computed as shown in table below.

|  |  | Main Canal Hydraulic Parameters |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Canal name | Change | Area(ha) | Duty(l/s/ha) | Q (m3/s) | m (H:V) |  | B (m) | D (m) | Fb (m) | Area | Perimeter | R=A/P | S | N | V=1/N*R^(2/3)*S^0.5 | Q req. (m3/s) |
| M.C | 00-137 | 154 | 0.0013 | 0.09 |  | 1.00 | 0.60 | 0.60 | 0.20 | 0.36 | 1.8 | 0.20 | 0.001 | 0.017 | 0.6 | 0.229020088 |
| M.C | 137-436 | 154 | 0.0013 | 0.09 |  | 1.00 | 0.60 | 0.50 | 0.20 | 0.30 | 1.6 | 0.19 | 0.001 | 0.017 | 0.5 | 0.163512707 |
| M.C | 436-570 | 154 | 0.0013 | 0.09 |  | 1.00 | 0.60 | 0.40 | 0.20 | 0.24 | 1.4 | 0.17 | 0.02000 | 0.017 | 2.6 | 0.62 |
| M.C | 570-773 | 154 | 0.0013 | 0.09 |  | 1.00 | 0.60 | 0.40 | 0.20 | 0.24 | 1.4 | 0.17 | 0.01000 | 0.023 | 1.34 | 0.32 |
| M.C | 773-948 | 154 | 0.0013 | 0.09 |  | 1.00 | 0.60 | 0.40 | 0.20 | 0.40 | 1.73 | 0.23 | 0.02857 | 0.023 | 2.8 | 1.11 |
| M.C | 948-1273 | 154 | 0.0013 | 0.09 |  | 1.00 | 0.60 | 0.40 | 0.20 | 0.40 | 1.73 | 0.23 | 0.013 | 0.023 | 1.8 | 0.73 |
| M.C | 1273-2040 | 55 | 0.0013 | 0.07 |  | 1.00 | 0.60 | 0.40 | 0.20 | 0.40 | 1.73 | 0.23 | 0.001 | 0.023 | 0.5 | 0.19 |

- Based on the above computed value, the sill level of intake outlet is fixed to be at 1559.38m above mean sea level.

b) Capacity determination

The capacity of the intake outlet is the function of discharge required for the irrigation based on crop water requirements.

Generally, duty of the project area is 1.27l/s/ha taking into consideration the total efficiency of 51%.

Then, the required discharge, Qr= command area*Duty=154*1.27= 0.86m^3/s.

So, the canal & intake capacity has to be fixed in a manner that can convey the required discharge.

The intake dimension is computed using rectangular notch formula given by:-

Q = 2/3Cd*(2*g) 1/2B*H (3/2)

Where: Q=diverted discharge= the required discharge, Qr, 0.86m3/sec

Cd=Coefficient of discharge, Cd=0.6

B=Width of intake, 0.5m

H=Height of the intake =flow depth above sill level

G=Acceleration due to gravity

Therefore for a width of 0.6m

H= (Q*3/ (2*Cd*(2*g)1/2*B))2/3=0.6m.  Providing 0.005m clearance on both sides, then, 0.6m width & 0.6m height has to be provided.

#### Breast Wall and Operation Slab

Dimension of brust wall1.9mX0.2mX is provided

And operation slab 1.2m*1.2*0.2m is provided.

Minimum area of reinforcement 0.15% is taken along the respective direction

=0.0015*1200*150 =270mm2

Taking 50mm cover thickness, the effective depth is de=D-(50+12/2)=94

And spacing 250<3*de =282.

Therefore, 12@250 mm C/C spacing is provided.

Area of steel 678.24mm2 >Minimum area of reinforcement, 0.15% of the concrete ie202.5mm2.

A vertical raised gate is designed for the head regulator and under sluice. This gate slides over the breast wall-using spindle during opening and closing.

For easy operation of these gates, operation slab is provided. The size of the operation slab is fixed from the point of construction and free movement. The size of the operation slab is shown in the drawing for both head regulator and under sluice with thickness 0.2m.

The thickness of the breast wall is also the same as that of the operation slab. The nominal thickness is fixed from the point of construction rather than the imposed load. The thickness required for the imposed load is less than the nominal value and treated as cantilever retaining wall. For the breast wall, the minimum reinforcement area is taken as 15% along the respective direction.

Hence	Asteel = As=3.14*124=113.4mm^2

25% Grose area of the section=0.0012mm^2

Provide 12 c/c150 mm for under sluice and off take head regulator.

Considering cover thickness of 50 mm, effective depth = 50+12/6=56mm

The dimension of the operation slab is 1.2m*1.2m to give better safety for operators for both cases.

The actual arrangement of angle irons, spindles, shafts and operation slab including other components is shown on the design drawing.

For each arrangement and further information, refer to the design drawing.

#### Divide wall

The dimension of divide wall is 2m*1m*4.5m is provided for dividing of the flow of the water to the inlet and used as the purpose for guiding and operation mechanism.

#### Retaining Walls

3.4.2.5.1Design of protection works and guide w



The guide wall height is fixed by deducting total energy level from lowest riverbed level (LRBL) and adding free board as follows:

U/S HFL=1561.34

Minimum bed level=1559.154

Therefore Height of bank=U/S HFL+Fb-RBL=1.9m and 1m depth below riverbed level.

A guide wall 1.9 m height is provided.

#### Stractural stablity

Unite weight of masonry=25KN/m3

Angle of repose=300

Unite weight of concrete





#### Figure ;upstream retaing wall

Dimension

H=3.5m, D=0.4m, bf=0.2m, b=0.4m, bh=0.1m, B=2.5m

S1=0, S2=0.9(H):2.8(V)

##### 

| Parameters | Vertical forces | Horizontal forces | Arm length | stabilizing moments M+ | overturning moment M- | Remarks |
| --- | --- | --- | --- | --- | --- | --- |
| Ws= active lateral soil force |  | 34.70833 | 1.166667 | 0 | 40.49305556 |  |
| Ww1= weight of wall | 23.52 |  | 0.15 | 3.528 |  |  |
| Ww2= weight of wall | 86.24 |  | 1.766667 | 152.3573 |  |  |
| uplift | 42.91875 |  | 0.833333 |  | 35.765625 |  |
| sum | 109.76 | 34.70833 |  | 155.8853 | 76.258154056 |  |



Factor of safety= Horizontal resisting force

Horizontal sliding force

=∑V/∑H =109.76/34.7 =1.58 > 1.5

Factor of safety for granular back fill is 1.5 and for cohesive back fill it is 2.

Therefore for   granular and cohesive back fills it is Safe against sliding!

Stability against Overturning

Factor of safety=   Stabilizing moment

Overturning moment

= ∑Ms/∑Mo =155.88/76.25=2.044>1.5

Factor of safety for granular back fill is 1.5 and for cohesive back fill it is 2.

For granular and cohesive back fill, it is Safe against overturning!

Tension development checking

Centroid of the gravity wall is given Xavg=(∑Ms-∑Mo)/∑V

Xavg=0.73

Eccentricity of vertical component of the resultant force,Rv from the center of the footing is : e = B/2-Xavg

e =0.3

e < = B/6, with in the middle third of footing length, B .

Therefore no tension development.

Stability analysis of downstream wing wall

| H | B | b | D | bh | bf |
| --- | --- | --- | --- | --- | --- |
| 2 | 2.00 | 0.3 | 0.8000 | 0.2 | 0.1 |

| Description | Horizontal forces (KN/M3) | Vertical forces (KN/M3) | Lever arm(m) | Restoring moment(M+) | Disturbing moment (M-) |  |
| --- | --- | --- | --- | --- | --- | --- |
| pa= active soil pressure |  | 12.495 | 0.7 | 0 | 8.7465 |  |
| W1= weight of wall | 14.112 |  | 0.15 | 2.11154 |  |  |
| W2= weight of wall | 39.984 |  | 1.433333 | 57.3104 |  |  |
| Uplift force | 20.601 |  | 0.666667 |  | 13.734 |  |
| sum | 54.096 | 12.495 |  | 59.42154 | 22.4805 |  |
| safety against;- |  |  |  |  |  |  |
| Overturning(FSO) = |  |  | 2.742709>1.5 | ok |  |  |
| Sliding( FSS) = |  |  | 2.27>1.5 | ok |  |  |
|  | e= | 0.30 | <B/6=0.333 | no tension, hence it is save! |  |  |

## Bill of Quantity and Cost Estimation

The quantities of the various items have been worked out as per the final design and final drawings prepared for the scheme. The unit rates analysis has been carried out based on the data available in the vicinity of the project area.

Bill No. 1- General Items (AMID)

|  | Bill No. 1- General Items(AMID) |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Item No | Description | Unit | Quantity | Rate Birr | Amount Birr |
| 1.1 | Allow for mobilization | L.S | 1 | 262,450.92 | 262,450.92 |
| 1.2 | Allow for de -mobilization | Q | 1 | 209,960.74 | 209,960.74 |
| 1.3 | Camp (Staff residence, office, Store, Toilet & Shower, Guard House and ) | L.S | 1 | 650,000.00 | 650,000.00 |
| 1.4 | Allow for temporary access road to site | km | 1 | 160,000.00 | 160,000.00 |
| 1.5 | Dewatering of open trenches and excavations, pumps | LS | 1 | 23,310.00 | 23,310.00 |
| 1.6 | Provide project indicator post starting from the construction time | LS | 1 | 9,225.90 | 9,225.90 |
| 1.7 | Provision of as built drawings for the project | LS | 1 | 26,197.60 | 26,197.60 |
|  | Total |  |  |  | 1,341,145.16 |

HEAD WORK

| S.no | Item of work | Unite | Quantity | Unite price | Amount (eth. Birr) |
| --- | --- | --- | --- | --- | --- |
| 1 | Bed Bar |  |  |  |  |
| 1.1 | Excavation(soil) | m3 | 672.00 | 90 | 60,480.00 |
| 1.2 | Back fill | m3 | 270.48 | 114 | 30,834.72 |
| 1.3 | Cyclopean concrete(60% C-20,40% graded stone) | m3 | 85.00 | 1704 | 144,840.00 |
| 1.4 | ф12mm bar | kg | 1,440.95 | 47 | 67,263.55 |
|  | Subtotal |  |  |  |  |
| 2 | operational slab & breast wall and under sluice |  |  |  |  |
| 2.1 | Excavation(soil) | m3 | 6.89 | 90 | 619.76 |
| 2.2 | backfill | m3 | 0.98 | 114 | 112.01 |
| 2.3 | C-20 concrete | m3 | 0.73 | 2866 | 2,104.79 |
| 2.4 | ф12mm bar | kg | 72.48 | 46.68 | 3,383.37 |
| 2.5 | plastering | m2 | 7.44 | 122.2 | 909.17 |
| 3 | Divide Wall |  |  |  |  |
| 3.1 | Cyclopean concrete(60% C-20,40% graded stone) | m3 | 8.26 | 1632 | 13,469.90 |
| 3.2 | Plastering | m2 | 5.14 | 122 | 627.62 |
|  | sub total |  |  |  |  |
| 3 | River retaing upstream and downstream |  |  |  |  |
| 3.1 | Excavation(soil) | m3 | 373.42 | 90 | 33,608.11 |
| 3.2 | Back fill | m3 | 97.89 | 114 | 11,159.23 |
| 3.3 | Masonry | m3 | 339.68 | 1646 | 559,108.01 |
| 3.4 | plastering | m2 | 10.98 | 122 | 1,341.76 |
|  | Subtotal |  |  |  |  |
| 4 | left side of river with retaing gabion works |  |  |  |  |
| 4.1 | excavation(soil) | m^3 | 2,422.50 | 90 | 218,024.82 |
| 4.2 | Gabion mattress | m3 | 2,562.26 | 1400 | 3,587,160.50 |
| 4.3 | Back fill with compacted granular material 20cm thick | m3 | 18.64 | 114 | 2,129.90 |
|  | subtotal |  |  |  |  |
| 5 | Trash rack |  |  |  |  |
| 5.1 | 12mm reinforcement bar | kg | 10.22 | 47 | 477.26 |
| 5.2 | angle iron 1m for trash rack 60mm thickness | m2 | 1.00 | 3500 | 3,500.00 |
|  | subtotal |  |  |  |  |
| 6 | Gates |  |  |  |  |
| 6.1 | intake gate installation consist of:- | No | 2.00 | 14233 | 28,466.32 |
|  | 6mm thickness sheet metal, 0.5m X 0.5m |  |  |  |  |
|  | 50X50X10mm angle iron, 1.8m length |  |  |  |  |
|  | n12mm bar for anchorage, 1.50Kg |  |  |  |  |
|  | n10mm bar for anchorage, Angle iron with the masonry wall 2.0Kg |  |  |  |  |
| 7 | under sluice gate |  |  |  |  |
| 7.1 | intake gate installation consist of:- | No | 1.00 | 14,944.82 | 14,944.82 |
|  | 6mm thickness sheet metal, 0.5m X 0.5m |  |  |  |  |
|  | 50X50X10mm angle iron, 1.95m length |  |  |  |  |
|  | 12mm bar for anchorage, 1.50Kg |  |  |  |  |
|  | 10mm bar for anchorage, Angle iron with the masonry wall 2.0Kg |  |  |  |  |
|  | sub total |  |  |  |  |
|  | TOTAL COST |  |  |  | 4,784,565.61 |

# SECTION-III: IRRIGATION AND DRAINAGE SYSTEMS INFRASTRUCTURE

# IRRIGATION AND DRAINAGE SYSTEMS DESIGN

## Irrigable Area Description

### Topography

The nature of topography can influence soils of an area in many ways. In irrigated agriculture, topography influences the choice of irrigation method, the labour requirement, the irrigation efficiency, the cost of land development, the problems of drainage, the hazards of erosion, the range of possible crops that can be grown  and the size/shape of fields to put under cultivations. Furthermore, the suitable soils occur on a range of slopes up to 12%. A slope of land greater than 12% is considered permanently unsuitable for irrigation for all land use types (LUTs). Based on secondary data obtained from 012 (Robit) Kebele Agricultural Development Office, the topography of the kebele contains 154% plain, 14% rugged and 18% mountainous. On the other hand, based on slope gradient classes which have developed from topographic survey data of the command area, using ArcGIS 10.1 Software, the command area has consisted of flat to very gently sloping topographic feature. In general, the slope gradient classes of the command area range from 2% to 10%.

From the abovementioned evidences and the agro-ecological suitability of the area, development of modern irrigation scheme is believed to be encouraging for the production of high value crops to bring rapid and sustainable development in the project area.

### Soil characteristics

Soil properties (physical, chemical, etc.) greatly influence the growth and thereby yield of crops which is grown. For the determination of important micro/macro elements for the required fertilizer application purposes a composite nature of sampling has been made with 0 - 30 cm and 30 - 60 cm soil depths. The laboratory results of physical and chemical analysis of the composite soil samples has presented in Table 3 of agronomic report.

### Existing Irrigation Practices in the Project Area

The pressure of survival and the need for additional food supplies to meet the demands of the increasing population is necessitating a rapid expansion of irrigation schemes. Thus, irrigation is becoming a basic part of well-developed agriculture wherever there is water and irrigable land potential. Accordingly, individual farmers that use the river flow to the left side and right sideis with laborious temporary diversions under take traditional irrigation practices. Therefore, the farmers in the project area are very much interested in the idea of upgrading the traditional scheme to modern scheme.

## Irrigation Water Requirement

### Crop Water Requirement (CWR)

The calculation of crop water requirement is a very important aspect for planning of any irrigation project. Several methods and procedures are available for this. The Food and Agriculture Organization (FAO) of the United Nations has also made available several publications on this subject and other issues related with this. The computer program available in FAO Irrigation and Drainage Paper No. 56 “CROPWAT” has been used for the calculation of Crop Water requirement. This program is based on Penman-Monteith approach and procedures for calculation of crop water requirements and irrigation requirements are mainly based on methodologies presented in FAO Irrigation and Drainage Paper No. 24 “Crop Water Requirements” and No. 33 “Yield Response to Water”.

The corresponding values of the crop water requirements of the proposed crops of the project are presented in the Agronomy Study of the same project.

### Irrigation efficiency (Ep)

To complete the evaluation of the demand, the efficiency of the water distribution system and of application must be known.

The gross requirement of water for irrigation system is very much dependent on the overall efficiency of the irrigation system, which in turn is dependent on several factors: Method of irrigation, type of canal (Lined and/or Unlined), method of operations (simultaneously and continuous or rotational water supply), and availability of structures (for controlling and distribution and measuring and monitoring).

On the basis of these factors, the project has planned to impose surface (furrow & basin) irrigation method. Concerning the canal systems, main canal is lined with about 1.1 km distance while secondary & tertiary canals are unlined (earthen). Hence, the conveyance efficiency has estimated to be 85% (as main and secondary canals will be lined and the rest are earthen type, based on engineering study data) and the field application efficiency has also estimated to be 60% seeing that the soil texture of the command area is predominantly clay loam type (according to ADSWE soil Lab result). Thus, the overall/project efficiency has estimated to be 51%.

The aforementioned three steps as enumerated above will be able to give the crop water requirement during its cropping period. The losses in the irrigation system have to be incorporated to arrive at the water requirement at the head of the irrigation system.

### Irrigation duty

Irrigation duty is the volume of water required per hectare for the full flange of the crops. Moreover, it helps in designing an efficient irrigation canal system. The area, which will be irrigated, can be calculated by knowing the total available water at the source and the overall duty for all crops required to be irrigated in different seasons of the years.

The proposed cropping pattern of See irrigation project has showed maximum net irrigation water requirement (NIWR) of 4.5 mm/day for dry season in the month of March and 3.6 mm/day for wet season in the month of September for all crops. However, for the designing of the irrigation water application and for the flows in the entire canal system, of the proposed or selected crops the one that has maximum NIWR has been taken (maize). Therefore, maize has showed the maximum NIWR which is 5.59 mm/day in the month of April and taken for the irrigation project duty calculation for 24 irrigation hours (see the calculation below).

e = ec * ea = 60*85 =51%

100        100

Where, e = Scheme irrigation efficiency (%)

ec = Conveyance efficiency (%)

ea = Field application efficiency (%)

For the designing of the project, the GIWR is given as follows:

GIWR = NIWR/e = 5.59/0.51 = 10.96 [mm/day] for full irrigation

GIWR = NIWR/e = 4.7/0.51 = 9.22 [mm/day] for supplementary irrigation

The GIWR (10.96 and 9.22 mm/day) represents the daily quantity of water that is required to be applied for dry and wet season crops, respectively. This water quantity is also used for the determination of the canal discharge in consideration of the time of flow and also defined as the duty, expressed as l/s/ha.

The duty is calculated by:

Duty (D) = GIWR × 1000 × 10 / (t × 60×60)

Where; Duty – the duty [l/s/ha]

GIWR – Gross Irrigation Requirement [mm/day]

t – Daily irrigation or flow hours [hrs]

The duty for GIWR of 10.96 mm/day (dry season) and 9.22 mm/day (wet season) for daily irrigation time (t = 24 hrs for dry and 20 hrs for wet seasons) is supported to be used with surface irrigation method. Thus, duty for 24 and 20 irrigation hours of full and supplementary irrigations, respectively, has computed as follows:

D = (10.96 x 10 x 1000) / (24 x 3600) = 1.27 l/s/h (for full irrigation during dry season)

D = (9.22 x 10 x 1000) / (20 x 3600) = 1.27

### Irrigation methods

Among the different irrigation systems, Amide irrigation system will be used for the project area; and the irrigation water will be obtained from Amide River. In addition, by constructing diversion intake and convoying the water commonly through earthen canals (PC, SC, and TC) and then leading to field canals; and finally irrigation takes place mostly in furrows.

For this project, among the various irrigation methods, surface irrigation method has been selected.  Of the surface irrigation methods furrow, border and basin irrigation methods can be used to supply irrigation water to the plants/crops. However, each method has its own advantages and disadvantages. Care should be taken when choosing the method, which is best suited to the local circumstances, i.e., depending on slopes, soil types, selected crop types, amount of water available, etc. of the command area.

Based on the above factors surface irrigation method has been proposed for the proposed crops in this project. The method allows applying light irrigation and can be laid out in sloping fields along the contour. Furrow irrigation method is best suited for most of the proposed and row planted crops. In general, furrow irrigation method is simple, manageable and widely practiced irrigation method. This method is suitable for row crops that cannot stand in water for long periods. The only thing required to use this method is row planting of crops. Besides, basin and border irrigation method would be used for the non-row planted crops. Rotational flow water distribution is also recommended for the project area.

## Irrigation and Drainage System Layout

The irrigation system layout for the project is prepared considering the following points besides other factors.

- A primary concern in the layout of the system is that it serves the purpose of conveying and distributing water to the command area.

- The excavation and earth fill volumes not be excessive, otherwise the construction costs can be tremendous.

- The selection of longitudinal bed slope is made taking into account the existing slopes of the terrain, to minimize deviations in canal routing.

- Curves in canals should not be too sharp.

The proposed irrigation system layout comprises one main canal, one secondary canal and three tertiary canals as shown on the layout Drawings. The main canal runs for most of its length parallel to the contours and several changes of direction are necessary to follow the topography. It crosses four gullies, one-foot path. The main canal is masonry lined for a length of 1100meters starting from the intake outlet to 1100m the main canal is lined since the soil is pervious and 1700 m is earthen canal.

### Conveyance System

The conveyance system consists of one Main canal to irrigate total command area of 154ha. The main canal starts from Water abstraction site on left side and conveys water for a length of 2.040Km.

Main canal is aligned along contours and supplies to one secondary unit and four tertiary canals.

## Design of the Canal System

Flow Depth and Section Capacity

The earthen canals have been designed with a trapezoidal shape and the lined ones with rectangular x-section using Manning's Formula:

Where Q= discharge (m3/s)

R= Hydraulic radius (Flow area/wetted perimeter)

S= Hydraulic gradient

n= Manning's roughness coefficient, n=0.027 is adopted for the earth channels and n=0.023for the masonry lined part of the main canal

### Main Canal

. The main canal is lined rectangular (from 0-1100m) and unlined trapezoidal

(1100m-2700m) canal. Determine the depth of flow in the canal for design

discharge using Manning’s equation: i.e. V=1/n* R^(2/3)*S^.5.

At a change of 948m and1085m, there are Flume structures.

Table 4-1: Hydraulic Parameters of main canal

|  |  | Main Canal Hydraulic Parameters |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Canal name | Change | Area(ha) | Duty(l/s/ha) | Q (m3/s) | m (H:V) | B (m) | D (m) | Fb (m) | Area | Perimeter | R=A/P | S |  | N | V=1/N*R^(2/3)*S^0.5 | Q req. (m3/s) |
| M.C | 00-137 | 154 | 0.0013 | 0.09 | 1.00 | 0.60 | 0.60 | 0.20 | 0.36 | 1.8 | 0.20 | 0.001 |  | 0.017 | 0.6 | 0.229020088 |
| M.C | 137-436 | 154 | 0.0013 | 0.09 | 1.00 | 0.60 | 0.50 | 0.20 | 0.30 | 1.6 | 0.19 | 0.001 |  | 0.017 | 0.5 | 0.163512707 |
| M.C | 436-570 | 154 | 0.0013 | 0.09 | 1.00 | 0.60 | 0.40 | 0.20 | 0.24 | 1.4 | 0.17 | 0.02000 |  | 0.017 | 2.6 | 0.62 |
| M.C | 570-773 | 154 | 0.0013 | 0.09 | 1.00 | 0.60 | 0.40 | 0.20 | 0.24 | 1.4 | 0.17 | 0.01000 |  | 0.023 | 1.34 | 0.32 |
| M.C | 773-948 | 154 | 0.0013 | 0.09 | 1.00 | 0.60 | 0.40 | 0.20 | 0.40 | 1.73 | 0.23 | 0.02857 |  | 0.023 | 2.8 | 1.11 |
| M.C | 948-1273 | 154 | 0.0013 | 0.09 | 1.00 | 0.60 | 0.40 | 0.20 | 0.40 | 1.73 | 0.23 | 0.013 |  | 0.023 | 1.8 | 0.73 |
| M.C | 1273-2040 | 55 | 0.0013 | 0.07 | 1.00 | 0.60 | 0.40 | 0.20 | 0.40 | 1.73 | 0.23 | 0.001 |  | 0.023 | 0.5 | 0.19 |

BW= Canal bottom width                         QR= Required discharge

FSD= Full supply depth                             QD= Designed Discharge

FB= Free board

SS= Side slope

V= Velocity

### Secondary Canals

The capacity of the two secondary canals is determined by multiplying the irrigable area under specified canal the same as main canals. The secondary canals are designed separately as the slope is very steep and to treat the high velocity before supplying to the tertiary canal. They are designed to  be rectangular masonry and earthen trapezoidal sections. The design principle is the same like that of tertiary canal. The irrigation canal sections are designed according to manning’s formula as follow. The hydraulic variables are calculated either iteration or flow master version 4.3

Q =

Where

Q = Design discharge, m3/s

R = Hydraulic radius of the canal, m

A = Wetted cross sectional area of canal, m2

n = Manning’s roughness coefficient

S = Bed slope of the canal

Table 4-2: Hydraulic Parameters of secondary canals

|  |  | Hydraulic Parameters |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Canal name | Change | Area(ha) | Duty(l/s/ha) | Q (m3/s) | m (H:V) | B (m) | D (m) | Fb (m) | Area | Perimeter | R | S | N | V | Q d (m3/s) |
| SC.1 | 00-375 | 55 | 0.002 | 0.11 | 1.00 | 0.40 | 0.30 | 0.20 | 0.12 | 1.25 | 0.10 | 0.0140 | 0.023 | 1.08 | 0.13 |
|  | 375-792 | 35 | 0.002 | 0.07 | 1.00 | 0.40 | 0.20 | 0.20 | 0.12 | 0.97 | 0.12 | 0.0080 | 0.023 | 0.97 | 0.12 |

Figure 4-1: Typical Cross Section of secondary canals

### Tertiary canals

The capacity of the canal is determined by multiplying the irrigable area under specified canal the same as main canals. The irrigation canal sections are designed according to manning’s formula as follow. The hydraulic variables are calculated either iteration or flow master version 4.3

Q =

Where

Q = Design discharge, m3/s

R = Hydraulic radius of the canal, m

A = Wetted cross sectional area of canal, m2

n = Manning’s roughness coefficient

S = Bed slope of the canal

The shape of the canal cross section, A is trapezoidal with 1 to 1.5--side slope as shown below

. Figure 4-2: Typical Cross Section of tertiary canals

Where;

b = Bed width of the canal, m

d = Water depth of the canal, m

FB = free board of the canal, m

p = wetted perimeter of the canal, m

Table 4-3: Hydraulic Parameters of tertiary canals

|  |  | Hydraulic Parameters |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Canal name | Change | Area(ha) | Duty(l/s/ha) | Q (m3/s) | m(H:V) | B (m) | D (m) | Fb (m) | Area | Perimeter | R=A/P | S | N | V=1/N*R^(2/3)*S^0.5 | Q req. (m3/s) |
| TC- 1 | 0+00-0+211 | 14.4 | 0.002 | 0.029 | 1.00 | 0.30 | 0.20 | 0.20 | 0.10 | 0.87 | 0.12 | 0.0038 | 0.025 | 0.6 | 0.06 |
| TC-2 | 00-444 | 4 | 0.002 | 0.008 | 1.00 | 0.30 | 0.20 | 0.20 | 0.10 | 0.87 | 0.12 | 0.000 | 0.025 | 0.8 | 0.08 |
| TC-3 | 0-425 | 19.23 | 0.002 | 0.038 | 1.00 | 0.30 | 0.20 | 0.20 | 0.10 | 0.87 | 0.12 | 0.004 | 0.024 | 0.63 | 0.063 |

### Field Canals

As shown in the layout, field canals run across the contours and hence face relatively steeper gradient. The discharge of most of the field canals is very small and this is taken as an advantage to cope up with the relatively steeper gradient. As much as possible field canals shall be made in fill in order to easily irrigate the adjacent command area. As can be seen from the layout, majority of the filed canals can be used to irrigate both sides of the command area depending on the condition of the individual plots of land owned by individual farmers.

Figure 4-3: Typical Field Canal X-section

## Drainage Canals

### Catch drainage canals

In the layout system there are four Catch drain canals, the designed discharge is determined based on the Catchment area, run off coefficient and rainfall intensity (Rational formula). The sections of the canals are determined by using man

ning is formula, and they are trapezoidal section.

Table 4-4: Hydraulic Parameters of Catch drains

| Type of canal | Catchment area(ha) | Design discharge Q(m3/s) | Bed width (m) | Flow depth (m) | velocity (m/s) | Free board (m) | Total canal depth (m) | Top width (m) | Canal length (m) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CD-1 | 0.28 | 9.14 | 1 | 0.8 | 3.14 | 0.3 | 1.1 | 2 | 149.95 |
| CD-2 | 0.4 | 3.34 | 1 | 0.8 | 1.14 | 0.3 | 1.1 | 1.5 | 258.58 |
| CD-3 | 0.28 | 4.154 | 1 | 0.8 | 1.62 | 0.3 | 1.1 | 1.5 | 132.65 |
| CD-4 | 0.9 | 4.154 | 1 | 0.8 | 1.62 | 0.3 | 1.1 | 1.5 | 1181.28 |

### Design of a typical flume

In amide irrigation project three flumes with span length of26m, 28m and 34m with different hydraulic property are designed. For further information see the excel sheet and drawing.

Table 4-5: Hydraulic Parameters of Flume

| Flume on | start Change | Q | V | L | h | t |
| --- | --- | --- | --- | --- | --- | --- |
| Flume 1At Mc | 250 | 0.73 | 1.8 | 25 | 4 | 0.4 |
| Flume2 At Mc | 650 | 0.19 | 0.5 | 27 | 1.2 | 0.4 |
| Flume 3 At Mc | 951 | 0.13 | 1 | 26 | 4 | 0.4 |
| Flume 4at Mc | 1098 |  | .25 | 28 | 2 | 0.3 |
| Flume 5at Sc | 331 | .0.12 | 0.25 | 34 | 1.2 | 0.3 |

# . Hydraulic and Structural Design of Irrigation Structures

The types of structures that are designed on this project are drop, division box, and turnout, flume, and drainage structures. Bligh theory is used to ensure against piping and uplifting pressure. All the structures are provided with sufficient depth of cutoffs as well as thickness to make structurally stable.

## 5.1 Drop structure

For irrigation canals, vertical drop structure of U.S.B.R type standard is selected to convey water from higher to lower elevation of the ground. The design procedures are shown below:

A. Critical hydraulic

- Design discharge, Q (m3/s)

- Height of drop, h(m)

- Width of drop, bc =

Where; d = water depth of the canal, m

- Critical discharge, q = Q/bc

- Critical depth, dc =

B. Stilling basin

- Basin width, B =

- Basin length, L =

- Lip height, a = dc/2, a  0.15

All the hydraulic parameters computed using the above procedures with the type of canals are shown on table 5.1 below.

| Change | Q | V | b | He | d | h | b1 | q | dc | L | B | t | a | bc | L1 | L2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | (m3/s) | (m/s) | (m) | (m) | (m) | (m) | (m) | (m3/s/m) | (m) | (m) | (m) | (m) | (m) | (m) | (m) | (m) |
| Mc-137 | 0.23 | 0.636 | 0.6 | 0.52 | 0.5 | 1 | 0.4 | 0.51 | 0.30 | 1.56 | 0.87 | 0.4 | 0.37 | 0.45 | 1.9 | 1.9 |
| Mc-143 | 0.16 | 0.545 | 0.6 | 0.42 | 0.4 | 1 | 0.4 | 0.36 | 0.24 | 1.35 | 0.74 | 0.4 | 0.33 | 0.45 | 1.8 | 1.8 |
| Mc-0+562 | 0.62 | 2.567 | 0.6 | 0.74 | 0.4 | 1 | 0.4 | 0.86 | 0.42 | 1.96 | 1.38 | 0.4 | 0.52 | 0.154 | 2.4 | 2.4 |
| Mc-0+699 | 0.32 | 1.342 | 0.6 | 0.49 | 0.4 | 1 | 0.4 | 0.47 | 0.28 | 1.50 | 1.02 | 0.4 | 0.38 | 0.69 | 2.1 | 2.1 |
| Mc-848 | 1.11 | 2.767 | 0.6 | 0.79 | 0.4 | 1 | 0.4 | 0.96 | 0.45 | 2.06 | 1.76 | 0.4 | 0.54 | 1.16 | 2.8 | 2.8 |
| Mc-1003 | 0.73 | 1.830 | 0.6 | 0.57 | 0.4 | 1 | 0.4 | 0.59 | 0.33 | 1.65 | 1.48 | 0.4 | 0.43 | 1.25 | 2.5 | 2.5 |
| Mc-1056 | 0.19 | 0.4154 | 0.6 | 0.41 | 0.4 | 1 | 0.4 | 0.36 | 0.24 | 1.35 | 0.79 | 0.4 | 0.33 | 0.52 | 1.9 | 1.9 |
| Mc-1148 | 0.19 | 0.4154 | 0.6 | 0.41 | 0.4 | 1 | 0.4 | 0.36 | 0.24 | 1.35 | 0.80 | 0.4 | 0.33 | 0.53 | 1.9 | 1.9 |
| Sc-50 | 0.13 | 1.079 | 0.3 | 0.46 | 0.4 | 1 | 0.4 | 0.42 | 0.26 | 1.44 | 0.66 | 0.4 | 0.36 | 0.31 | 1.74 | 1.7 |
| Sc-75 | 0.13 | 1.079 | 0.3 | 0.46 | 0.4 | 1 | 0.4 | 0.42 | 0.26 | 1.44 | 0.66 | 0.4 | 0.36 | 0.31 | 1.74 | 1.7 |
| Sc-125 | 0.13 | 1.079 | 0.3 | 0.46 | 0.4 | 1 | 0.4 | 0.42 | 0.26 | 1.44 | 0.66 | 0.4 | 0.36 | 0.31 | 1.74 | 1.7 |
| Sc-150 | 0.13 | 1.079 | 0.3 | 0.46 | 0.4 | 1 | 0.4 | 0.42 | 0.26 | 1.44 | 0.66 | 0.4 | 0.36 | 0.31 | 1.74 | 1.7 |
| Sc-175 | 0.13 | 1.079 | 0.3 | 0.46 | 0.4 | 1 | 0.4 | 0.42 | 0.26 | 1.44 | 0.66 | 0.4 | 0.36 | 0.31 | 1.74 | 1.7 |

## Turnouts

The turnouts are used to direct water form supply canals into laterals; it designed with rectangular notch weir formula with the same principle as that of the division boxes.  Field canals on the same tertiary block will receive the full design discharge of the tertiary canal for irrigation hours proportional to their hectare as possible as to manage effectively and efficiently.

| Canal Name | No. of Turn outs | Q (lit/sec) | D=d+fb (m) | B (m) | L (m) | Lup (m) | Ldp (m) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| MC,FC1-1,1-2,1-3,1-4 | 4 | 1.80 | 0.40 | 0.60 | 1.4 | 0.00 | 1.60 |
| TC-1,FC1-12,1-13,1-14,1-15 | 4 | 1.80 | 0.40 | 0.60 | 1.4 | 0.00 | 1.60 |
| TC-2,FC1-5,1-6,1-7,1-8 | 4 | 0.53 | 0.40 | 0.60 | 1.4 | 0.00 | 1.60 |
| TC-3,FC1-12,1-13,1-14,1-15 | 4 | 2.50 | 0.40 | 0.60 | 1.4 | 0.00 | 1.60 |
| TC-4,FC1-16,1-17,1-18,1-19 | 4 | 4.20 | 0.40 | 0.60 | 1.4 | 0.00 | 1.60 |

Note:

a) B = B1 = B2, since the system is rotational

b) No of sheet metal per tertiary canal is number of turn out in the

Tertiary =NT

NS = NT

Where:

NS = Number of stop logs per tertiary canal

NT = Number of turnouts per tertiary canal

For emergency purpose, it is possible to have one more stop logs called sheet metal log per tertiary canal.

For clear understanding, refer the typical drawings

### Design of Division box

Figure 5-1: Typical Division Box plan

River Protection Works by Gabion

Though for some part of the river left banks  masonry retaining wall have been designed, the rest left banks are being affected by intense erosion. Therefore, to keep the base flow direction, and to protect the bank from erosion the bank should be treated with gabions. The right bank is relatively stable and masonry retaing walls treated the banks. Therefore, no need of designing the protection works. As we have observed during site study about 480.1m length of the bank is severely eroded and damaged, so that to protect this 3m height and 480.1m length of gabion is provided since the main canal passes through the bank.

Gabion works design;-

- Gabion mattress size: - this the gabion laid at the toe of the river left bank. As computed above the scour depth at headwork is 0.82m. Hence, the mattress extends 1m below the riverbed.

The size of stone to be used is 180mm diameter within gabion mesh size of 60 *80mm.

- Gabion box size determination: - the gabion in box form is used as standing wall by over laying each other. Its height is equal to the bank height for this project, which is 3m. The size of stone 200mm should be used with gabion mesh size of 60*80mm.for better insight see figure below.

Figure 2 Gabion Retaining Works

- Stability Analysis Gabion,

Data required;-

- Unit weight of dry soil =19KN/M^3

- Unit weight of Saturated soil =12KN/M^3

- Unit weight of stone=14-20KN/M^3( for 100-200mm diameter),

- take  the average unit weight of stone to be16KN/M^3

- Φ=300 (for dry condition)

- Then , Ka =1 -sinΦ1+sinΦ=1 -sin351+sin35=0.33

| Table-9 stability analysis of gabion |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| DESCRIPTION | horizontal forces (KN/M3) | vertical forces (KN/M3) | lever arm(m) | restoring moment(M+) | Disturbing moment (M-) |  |
| W1= weight of stonewall |  | 4.75 |  |  |  |  |
| W2= weight of stonewall |  | 19 |  |  |  |  |
| W3= weight of stonewall |  | 23.75 |  |  |  |  |
| W4= weight of stonewall |  | 28.5 |  |  |  |  |
| ps1= pressure of soil | 28.215 |  | 0.609 |  | 17.19 |  |
| summation | 28.25 | 76 | 0.54 | 41.56 | 17.19 |  |
| safety against;- |  |  |  |  |  |  |
| Overturning(FSO) = | 2.41 | >1.5 | ok |  |  |  |
| Sliding( FSS) = | 1.56 | >1.5 | ok |  |  |  |
| Existence of tension | X= | 0.930 |  |  |  |  |
|  | e= | 0.27 | <B/6=0.33 | Tension he exists ,not safe |  |  |

Table 10. Irrigation Infrastructure Bill of Quantities and Cost Estimate

| S.NO | ITEM OF WORK | UNITE | QUANTITY | UNITE PRICE | AMOUNT (ETH. BIRR) |
| --- | --- | --- | --- | --- | --- |
|  | A.CANALS |  |  |  |  |
| 1 | MAIN CANAL |  |  |  |  |
| 1.1 | 20cm thick granular material below the bottom level of the canal ,well compacted | m3 | 92.00 | 114 | 10,512.84 |
| 1.2 | Ordinary earth excavation | m3 | 4,685.42 | 90 | 421,687.98 |
| 1.3 | Back fill and compaction | m3 | 1,188.18 | 114 | 135,452.52 |
| 1.4 | Masonry work | m3 | 1,584.00 | 1646 | 2,607,264.00 |
| 1.5 | Earthen canal(1700m) main canal | m3 | 1,904.00 | 90 | 171,360.00 |
| 1.6 | Back fill and compaction main canal |  | 540.00 | 114 | 61,560.00 |
| 1.7 | Plastering | m2 | 3,300.00 | 122 | 403,260.00 |
| 2 | Secondary canal 1 |  |  |  |  |
| 2.1 | 20cm thick granular material below the bottom level of the canal ,well compacted | m2 | 17.25 | 114 | 1,971.16 |
| 2.1 | Ordinary earth excavation | m3 | 626.77 | 90 | 56,409.57 |
| 2.2 | Back fill and compaction | m3 | 385.93 | 114 | 43,995.85 |
| 2.3 | Masonry work(chainage 00-450m) | m3 | 796.00 | 1646 | 1,310,216.00 |
| 2.4 | Plastering | m2 | 1,990.00 | 122 | 243,178.00 |
| 3 | Secondary canal 2 |  |  |  |  |
| 3.2 | Ordinary earth excavation(Trapezoidal section 450-790m) | m3 | 650.00 | 90 | 58,500.00 |
| 3.3 | Back fill and compaction | m3 | 422.00 | 114 | 48,108.00 |
| 3 | Tertiary canal(TC-1,2,3&4) |  |  |  |  |
| 3.1 | Ordinary earth excavation | m3 | 1,152.07 | 90 | 103,686.30 |
| 3.2 | Back fill and compaction |  | 21.79 | 114 | 2,484.35 |
|  | Subtotal |  |  |  |  |
| 4 | Turn out |  |  |  |  |
| 4.1 | Ordinary earth excavation | m3 | 40.30 | 90 | 3,626.64 |
| 4.2 | Back fill and compaction | m3 | 11.64 | 114 | 1,326.73 |
| 4.3 | Masonry work | m3 | 18.77 | 1646 | 30,892.13 |
| 4.4 | Plastering | m2 | 9.66 | 122 | 1,180.45 |
|  | sub total |  |  |  |  |
| 5 | Division Box |  |  |  |  |
| 5.1 | excavation | m3 | 5.98 | 90 | 538.20 |
| 5.2 | Masonry work | m3 | 4.47 | 1646 | 7,363.38 |
| 5.3 | Cemented stone pitching | m3 | 2.42 | 1413 | 3,412.78 |
| 5.4 | Plastering | m2 | 10.50 | 122 | 1,283.04 |
|  | subtotal |  |  |  |  |
| 6 | Gate |  |  |  |  |
|  | 4mm thick sheet metal | Pcs | 4.00 | 7600 | 30,400.00 |
|  | Stiffening angle iron (30*30*4) |  |  |  |  |
|  | Angle iron for groove(40*40*4) |  |  |  |  |
|  | 16mm reinforcement bar for handling |  |  |  |  |
|  | Subtotal |  |  |  |  |
| 7 | Drop structure |  |  |  |  |
| 7.1 | excavation | m3 | 59.92 | 90 | 5,392.35 |
| 7.2 | Masonry work | m3 | 241.50 | 1646 | 397,509.00 |
| 7.3 | Cemented stone pitching | m3 | 39.10 | 1632 | 63,792.82 |
| 7.4 | Plastering | m2 | 98.10 | 122 | 11,987.21 |
| 8 | Catch Drain |  |  |  |  |
| 8.1 | Ordinary earth excavation | m3 | 9,422.07 | 90 | 847,985.85 |
| 8.2 | Back fill and compaction | m3 | 50.30 | 114 | 5,734.31 |
|  | Subtotal |  |  |  |  |
| 8 | flume 1 at change 250 span length of 25m |  |  |  |  |
| 8.1 | ordinary soil excavation | m3 | 286.00 | 90 | 25,740.00 |
| 8.2 | backfill and compaction with selected material | m3 | 72.00 | 114 | 8,208.00 |
| 8.3 | c-20concrete | m3 | 18.00 | 2866 | 51,588.00 |
| 8.4 | ф10mm bar | kg | 586.00 | 47 | 27,354.48 |
| 8.5 | ф24mm bar | kg | 793.00 | 47 | 37,017.24 |
| 8.6 | masonry work for flume and abutment work | m3 | 96.60 | 1646 | 159,003.60 |
| 8.7 | plastering for only flume part excluding abutment | m2 | 5.80 | 122 | 708.27 |
|  | sub total |  |  |  |  |
| 9 | flume 2 at change 650 span length of 27m |  |  |  |  |
| 9.1 | ordinary soil excavation | m3 | 286.00 | 90 | 25,740.00 |
| 9.2 | backfill and compaction with selected material | m3 | 72.00 | 114 | 8,208.00 |
| 9.3 | c-20concrete | m3 | 19.00 | 2,866.00 | 54,454.00 |
| 9.4 | ф12mm bar | kg | 163.31 | 47 | 7,623.38 |
| 9.5 | ф24mm bar | kg | 460.76 | 47 | 21,508.23 |
| 9.6 | masonry work for flume and abutment work | m3 | 96.60 | 1646 | 159,003.60 |
| 9.7 | plastering for only flume part excluding abutment | m2 | 5.80 | 122 | 708.27 |
|  | sub total |  |  |  |  |
| 10 | flume 3 at change 951 span length of 26m |  |  |  |  |
| 10.1 | ordinary soil excavation | m3 | 286.00 | 90 | 25,740.00 |
| 10.2 | backfill and compaction with selected material | m3 | 72.00 | 114 | 8,208.00 |
| 10.3 | c-20concrete | m3 | 18.43 | 2,866.00 | 52,820.38 |
| 10.4 | ф12mm bar | kg | 163.31 | 47 | 7,623.38 |
| 10.5 | ф24mm bar | kg | 460.76 | 47 | 21,508.23 |
| 10.6 | masonry work for flume and abutment work | m3 | 96.60 | 1646 | 159,003.60 |
| 10.7 | plastering for only flume part excluding abutment | m2 | 5.80 | 122 | 708.27 |
|  | sub total |  |  |  |  |
| 11 | flum 4 at change 1098 span length of 28m |  |  |  |  |
| 11.1 | ordinary soil excavation | m3 | 286.00 | 90 | 25,740.00 |
| 11.2 | backfill and compaction | m3 | 72.00 | 114 | 8,208.00 |
| 11.3 | c-20concrete | m3 | 20.16 | 2866 | 57,777.13 |
| 11.4 | ф12mm bar | kg | 163.31 | 47 | 7,623.38 |
| 11.5 | ф24mm bar | kg | 460.76 | 47 | 21,508.23 |
| 11.6 | masonry work for abutment work | m3 | 96.60 | 1,646.00 | 159,003.60 |
| 11.7 | plastering | m2 | 5.80 | 122 | 708.27 |
|  | sub total |  |  |  |  |
| 12 | Flume 5 at secondary,chainage of 331m span length of 34m |  |  |  |  |
| 12.1 | ordinary soil excavation | m3 | 286.00 | 114 | 32,604.00 |
| 12.2 | backfill and compaction of selected material | m3 | 72.00 | 90 | 6,480.00 |
| 12.3 | c-20concrete | m3 | 25.00 | 2866 | 71,650.00 |
| 12.4 | ф12mm bar | kg | 338.10 | 47 | 15,782.51 |
| 12.5 | ф16mm bar | kg | 931.50 | 47 | 43,482.42 |
| 12.6 | masonry work for abutment work | m3 | 46.81 | 1646 | 77,041.03 |
| 12.7 | plastering | m2 | 9.55 | 122.2 | 1,166.40 |
| Total cost |  |  |  |  | 8,483,323.36 |

# CONCLUSION AND RECOMMENDATION

- The infrastructure of this project area is designed to irrigate about 154ha of land by taking its supply from the Amide diversion intake irrigation project. The maximum duty of the command area for 1.27l/s/ha four 24-hour irrigation hour with overall project efficiency of 51%.

- On the secondary unit of the irrigation systems, some are associated with drop. They are designed as far as possible to be partially filled and cut. The layout is designed as far as possible to avoid cross-structures within them.

- The design of the canal dimensions of the irrigation canal is done by applying the manning’s uniform flow equation. The variable of the hydraulic parameters are calculated using iteration or flow master program.

- The design discharge of the drainage canals are determined using rational formula and Gamble Powell method.

- As soils of the command area are predominantly clay textured; and hence water and soil management measures should be undertaken; and optimum moisture content should be maintained to improve workability of the soil during land preparation and planting time.

The following recommendations are drown:

- For better performance and long service year of the project regular inspection and maintenance is highly required.

- Farmers training, how to operate and maintain the project structures as a whole and available and water resources has a paramount important.

- The irrigation hours per day and per week should be flexible based on base flow amount of each week or month.

- Close supervision of the construction should be made to modify (if need be) each Components of irrigation system based on specific site conditions.

# OPERATION AND MAINTENANCE

## General

The main objective of the operation and maintenance aspect of an irrigation scheme is to facilitate the timely delivery of the required irrigation water to farms and to keep the irrigation system in an optimum operating condition.  This section therefore, discusses the main functions of the subject matter under consideration for the scheme.

## Operation of the Head Works

Operation at the diversion weir mainly focuses on the diversion of a controlled flow of river water, timely cleaning of floating debris in front of intake and removal of sediment deposits in front of the weir and intake structures.

## Irrigation System Operation

The operation of the irrigation system depends mainly on the method of water delivery at farm level. Surface irrigation method is the recommended type of water distribution and application method for Amide diversion irrigation scheme.

The farmers would organize themselves and form groups in order to handle the water management.  Since flow is low Rotational water distribution would be applied within the group.  The rotational distribution is then to distribute water by turn to the whole scheme according to the timely need of crop water requirement. For better and efficient water management, crop diversification should be avoided within a group.  This would reduce the complexity of water distribution system of the scheme during one irrigation season. At farmers’ level of operation, a constant flow and variable irrigation time is advisable.

The operation of the irrigation system is continuous for 14 hours per day in main, secondary and tertiary canals whereas field canals within a tertiary block are operating in rotational system with each other for irrigation hours proportional to their size. Since the tertiary canal discharges are within the manageable range and the irrigation canal structures are accordingly designed for simple operation, the farmers can open and close easily whenever they required.

## Maintenance Requirement

The canal system of the project is earthen canal except main canal and secondary canal, which is susceptible to siltation, erosion, growth of weeds and or breaching problems. Though the canal sections are designed for non- silting and non scouring conditions, the above mentioned problems are unavoidable and hence periodical and yearly inspection and maintenance of canals and structures are obligatory in order to fulfill the design objective of the project area.

The maintenance tasks are categorized into two types: - routine activities, and repairs. The routine maintenance activities that are carried out periodically include-

- Regular cleaning of sediments and weeds from canals and drains;

- Inspection and lubrication of gates; and

- Maintenance of cracked lined canals, regulating and control structures.

Repair works include task carried out more frequently and quickly, and include those tasks that are generally unpredictable. They also include emergency works. The activities included in this category are:-

- Repairing overtopped or breached canals, drains, and flood protection dykes;

- Repairing jammed gates;

- Filling holes made by wild animals; and

- Reduced free board due to walking over by people and livestock.

Regular inspection of the irrigation facilities should be carried out as part of the maintenance activities.  These tasks could be carried out immediately after the end of the main rains in September and during the rainy season.  This could concentrate on the interceptor drains and the flood protection dykes, the main canal and the field drains.  The inspection of the other works like the tertiary canals, field drains, and the water control and regulating structures could be carried out as part of routine operation activities.

- Beneficiaries of the project need to have operation and maintenance budget, For O&M cost incurring entity area:

- Purchase sing of gate lubricate (grease)

- Replacing and maintenance of Stolen and damaged gates

- Repair Damages on the cross drainage structures.

The expense for O&M should be collected from the beneficiaries. Of course, much of the task is done by the labor and skill of the community. For cost incurring activities beneficiaries have to collect money based on the proportion of the command area they owned.

# REFERENCE

- FAO (1977) guidelines for predicting crop water requirements. No 24, Rome Italy

- Design of small Canal structures , USBR

- Soft copies of hydraulic structure publishing

- IDD manual

- ESRDF manual