Amhara National Regional State

Water Resources Development Bureau

(BOWRD)

Detail Design

Of

Diversion Small-Scale Irrigation Project

Volume IV: Engineering Design Final Report

September, 2018

Bahir Dar

Client: Bureau of Water, Irrigation & Energy Development

(BoWRD)

Address:

P. O. Box: 88
Telephone: 0528-200853/855
Fax: 251-08-20-65-68/204676/202040

Consultant: Amhara Design & Supervision Works Enterprise

(ADSWE)

Address:

P. O. Box: 1921
Telephone: +251-582-181023/ 180638/181201/181254
Fax: (058) 2180550/ (058) 2180560
E-mail: amhara design@yahoo.com

Bahir Dar, Ethiopi

# FEASIBILITY STUDY & DETAIL DESIGN REPORT STRUCTURE

- Volume I: Watershed Management

- Volume II: Engineering Geology

- Volume III: Irrigation Agronomy

- Volume IV: Engineering Design

- Volume V: Socio Economy

- Volume VI: Environmental Impact Assessment
Table of Contents							 Page Nr

FEASIBILITY STUDY & DETAIL DESIGN REPORT STRUCTURE	i

LIST OF TABLES	v

LIST OF FIGURES	v

SAILENT FEATURE	vii

1	INTRODUCTION	1

1.1	Background	1

1.1.1	Description of the Project Area	1

1.2	Objectives of the Study	2

1.2.1	Major Objective	2

1.2.2	Specific Objectives	3

1.3	Scope of the Study	3

1.4	Methodology	4

SECTION-I:  HYDROLOGY	7

2	HYDROLOGY	9

2.1	Watershed Characteristics	9

2.2	Hydro-Metrological Data Availability	10

2.2.1	Climate	10

2.2.2	Rainfall Data	10

2.2.3	River flow data	10

2.2.4	Upstream & Downstream utilization	10

2.3	Design Flood Analysis	11

2.3.1	Design Rainfall computation	11

2.3.2	Outlier Test	11

2.3.3	Check for variance	12

2.3.4	Peak Discharge Determination	14

2.3.5	Tail Water Depth Computation	18

SECTION-II: HEADWORK DESIGN	21

3	HEADWORK STRUCTURES DESIGN	23

3.1	Headwork Site Selection	23

3.2	River Geomorphology	23

3.2.1	River Bed condition	23

3.2.2	River Bank condition	25

3.3	Sources of construction materials	26

3.3.1	Rock for Masonry and Crushed Coarse Aggregate	26

3.3.2	Fine Aggregates	26

3.3.3	Water	27

3.4	Headwork Type Selection	27

3.4.1	Hydraulic Design of Headwork Structure	27

3.4.2	U/S and D/S HFL Calculation & Determination	29

3.4.3	Hydraulic Jump Calculation	30

3.4.4	Impervious floor	31

3.4.5	Cut off Depth Calculation	33

3.4.6	Stability Analysis of weir/intake	34

3.4.7	Design of Divide wall, Under Sluice, and Canal outlet	36

3.5	Bill of Quantity and Cost Estimation	43

SECTION-III: IRRIGATION AND DRAINAGE SYSTEMS INFRASTRUCTURE	47

4	IRRIGATION AND DRAINAGE SYSTEMS DESIGN	49

4.1	Irrigable Area Description	49

4.1.1	Topography	49

4.1.2	Climate	49

4.1.3	Soil characteristics	50

4.1.4	Existing Irrigation Practices in the Project Area	50

4.2	Irrigation Water Requirement	50

4.2.1	Crop Water Requirement (CWR)	50

4.2.2	Irrigation efficiency (Ep)	50

4.2.3	Irrigation duty	51

4.2.4	Irrigation methods	52

4.3	Irrigation and Drainage System Layout	53

4.3.1	Conveyance System	53

4.4	Design of the Canal System	53

4.4.1	Main Canal	54

4.4.2	Secondary Canals	54

4.4.3	Tertiary canals	55

4.4.4	Field Canals	56

4.5	Drainage Canals	57

4.5.1	Catch drainage canals	57

4.5.2	Tertiary canal drainage canals	58

4.6	Canal Structures Design	58

4.6.1	Design of a typical flume	58

4.6.2	Design of Division box	59

4.6.3	Road crossing structure	62

4.7	Irrigation Infrastructure Bill of Quantities and Cost Estimate	63

5	CONCLUSION AND RECOMMENDATION	66

6	OPERATION AND MAINTENANCE	67

6.1	General	67

6.2	Operation of the Head Works	67

6.3	Irrigation System Operation	67

6.4	Maintenance Requirement	68

REFERENCE	69

# LIST OF TABLES

Table 2-1: Outlier test analysis	11

Table 2-2: Test for goodness to fit using D-index	12

Table 2-3: Determination of Time of Concentration	14

Table 2-4: Runoff analysis	16

Table 2-5: Hydrograph coordinates	16

Table 2-6: Weir/intake Site River Cross section Coordinate Data	18

Table 2-7: Stage discharge analysis	18

Table 2-8: River discharge computation at different stages of flow	20

Table 3-1: Computation of forces per unit width and momentum	37

Table 3-2: Typical design of masonry retaining wall	40

Table 4-1: Hydraulic Parameters of main canal	54

Table 4-2: Hydraulic Parameters of secondary canals	54

Table 4-3: Hydraulic Parameters of tertiary canals	55

Table 4-4: Hydraulic Parameters of Catch drains	57

Table 4-5: Hydraulic Parameters of Flume	59

Table 4-6: Hydraulic parameters of Division Boxes	61

# LIST OF FIGURES

Figure 1-1: Location map of the project area	1

Figure 2-1: Drainage map of (Name of Project) watershed	10

Figure 2-2: Complex Hydrograph	17

Figure 2-3: Rating Curve	19

Figure 2-4: River profile	19

Figure 3-1: River bed at the proposed weir/intake site	24

Figure 4-1: Typical Cross Section of secondary canals	55

Figure 4-2: Typical Cross Section of tertiary canals	55

Figure 4-3: Typical Field Canal X-section	56

Figure 4-4: Typical Cross Section of Catch drain canals	57

Figure 4-5: Typical Division Box plan	60

# SAILENT FEATURE

- Project name:-Awajo diversion Intake Irrigation Project

- Name of the stream: (awajo) river

- Location of the weir/intake site using

- North: 1099721

- South: 576316

- Zone: (North shewa Zone)

- Wereda: (Mojana wedera)

- Average Altitude: 2501 masl

- Hydrology

- Design rainfall: 62.22 mm

- Catchment area:22.59 Km2

- Longest flow path length: 9.69 Km

- Design flood: 74.13m3/sec

- Design base flow:-100 lit/se.

- Diversion Weir/Intake

- type: Intake

- Height: 2.00 m

- Weir/Intake crest level: 2512m.a.s.l

- U/S HFL: 2511.5 m.a.s.l

- Outlet

- Sill level:-2510. m.a.s.l

- Opening dimension:0.65*0.6 m2

- Discharge capacity: 136 lit/sec.

Irrigation and drainage systems Infrastructure

- Command area size: 80 ha

- Type of soil of the command area is dominantly clay soil

- Design discharge of the main canal =108.8 l/sec

- Irrigation system layout consists of one lined main canal, 3 secondary canals and16tertiary canals

- Main irrigation structures designed are;

- 2 Gully crossing structures

- 5 Foot path

- 1 flume

Project cost

| Item No-1 | Total for General Items | 2,838,000.00 |
| --- | --- | --- |
| Item No-2 | Total for headwork Structure Items | 1,313,808.77 |
| Item No-3 | Total for Infrastructure | 8,489,179.22 |
|  | TOTAL | 12,640,987.99 |
|  | 15% VAT | 1,896,148.20 |
|  | GRAND TOTAL | 14,537,136.19 |
|  | Proposed command area, ha | 80.00 |
|  | Head work cost / hectare | 181,714.20 |

# INTRODUCTION

## Background

In Ethiopia, under the prevalent rain-fed agricultural production system, the progressive degradation of the natural resource base, especially in highly vulnerable areas of the highlands coupled with climate variability have aggravated the incidence of poverty and food insecurity. The major source of growth for Ethiopia is still conceived to be the agriculture sector. Hence, this sector has to be insulated from drought shocks through enhanced utilization of the water resource potential of the country, (through development of small-scale irrigation, water harvesting, and on-farm diversification) coupled with strengthened linkages between agriculture and industry (agro-industry), thereby creating a demand for agricultural output. In line with the above, efforts have been made by the government to improve the situation in the country in areas of domestic water supply provision, irrigation, watershed management, etc. The Amhara Water Resources Development Bureau is playing its role in the development of small scale irrigation projects in the region. Accordingly, as part of the water sector development program, the office has initiated the study and design of a small scale irrigation scheme on Awajo River at Wodera Kebele and signed an agreement with Amhara Design & Supervision Works Enterprise (ADSWE) for the study and design of the project.

### Description of the Project Area

#### Location

This irrigation project is located mainly at Wodera Kebele, Mojana Wodera Wereda of North ShewaZone in the Amhara Region. The proposed irrigation project is to be undertaken on Awajo River and the headwork structures are specifically located at an altitude of about 2513 masl and geographical coordinates of 1099669 N (UTM) and 575957 E (UTM).

Figure 1-1: Location map of the project area

#### Accessibility

The project area is accessed through Dry Weather road 13Km from (Tarmaber) to (Mezezo), and 5Km very challenged road to the Project area which need series access rod works.

#### Previous Irrigation Practices

There are no modern and traditional diversions on the upstream/downstream of this river using different irrigation practices but as the hydrology and Hydrogeology study and respondent farmers indicated, the river has capacity so, the farmers in the project area are very much interested to this new modern scheme.

## Objectives of the Study

### Major Objective

The project area faces variability of rainfall distribution though the overall rainfall generally suffices the rain-fed agriculture. Accordingly, the rain-fed agriculture needs means of supplementing during distribution failures and further full irrigation is required to maximize the use of the potential land and water resources.

Hence the objective of this project is to contribute a substantial share in the effort to reduce the risk of production decrease due to rainfall variability and increase the productivity of the resource in the project specific area. Specifically, the project is targeted for the following.

- To make sustainable the rain-fed crop production and make extra production in the dry season possible for 100 ha of land through irrigation.

- There is a general consensus that irrigation investments will achieve broader food security and poverty reduction impacts and if efforts are also geared towards up-grading existing traditional farming practices with support to enhance access to input supply, output marketing and extension to facilitate access to information and innovations.

- This objective is to be realized by constructing diversion/intake structures across the Awajo River and diverting the river flow.

### Specific Objectives

Other benefits that can be expected to appear with the launching of the project are:

- Efficiency of water use improvement;

- Improved local nutrition/food security gains;

- Improved management of scarce natural resources (land and water);

- Resilience against drought risk;

- Rationale for erosion control and watershed management;

- Rationale for the intensification and modernization of small-holder agriculture and rural lifestyles.

The engineering study and design enables the realization of the project by the provision of engineering structures that will allow the appropriate abstraction of the Awajo river water for delivery in to the identified irrigation fields of the study area. Hence, this engineering design is specifically targeted to:

- Analyze hydrologic requirements of the project and engineering structures;

- The formulation of sound and stable structure,  with necessary provisions that allow safe, easy and low-maintenance operation in the service life of the project;

- Develop working drawings;

- Estimation of construction costs.

## Scope of the Study

- The irrigation design shall ensure reliability, equity and flexibility of water delivery to farmers. It will aim at reducing conflicts among water users and will lead to lower operation and maintenance costs.

- Updating the existing, if available, computation of the actual evapo-transpiration, crop water requirement, irrigation demand/duty using the existing and recent agronomic, climatologic and soil data using more appropriate methodologies.

- Establish design criteria for irrigations structures to be approved by the client and to be used in the final design stage,

- Design proper irrigation system compatible with local conditions and management capabilities,

- Establish flood protection measures for the command area and canal structures and design the respective drainage system accordingly,

- Planning  and layout of the irrigation system, which include irrigation canals, drainage channels, inspection roads and alignments, canal spacing, canal length, location of structures, and water profiles along canal and drains at specified reaches, which is most economical easily manageable and aligned with topographic feature and geological investigation.

- Determination and estimation of water application conveyance and other losses and irrigation efficiencies and consideration of those parameters in design steps.

- Check and test hydraulic and structural designs of main canal considering total demand and the required capacity and the base flow availability,

- Prepare general plans and drawings for all irrigation infrastructure and irrigation systems designs,

## Methodology

In the study and design procedure, Designers used the following steps.

- Specific Site identification:

  - Review of the reconnaissance survey conducted by the Client

  - 50,000 scale top map and GIS information

  - Local farmers interview and discussion

  - Wereda and Zone Agriculture section expertise

  - Previous studies

  - On foot travel along the river channel and farm areas.

- Topographic survey:

  - Surveying the headwork site and the Command area with sufficient radius, using Total station

- Flow estimation

  - Physical observation on flood mark indications and local information about high flood and critical flow condition of the river

  - Analyzing the recorded river flow data and use watershed inputs for further analysis.

  - Base flow estimated during the reconnaissance field visit by floating method.

- Irrigable area identification:

  - Using local information

  - 50,000 Topographic map, and GIS information, GPS to see elevation

The design report is organized in three sections. In Section I the Hydrology study is presented and in Sections II and III the Headwork and Irrigation and Drainage Systems designs are discussed respectively. In Section III, planning and design of the irrigation system after diverting the water using the Intake will be dealt. The following are major areas of concern in this part.

- Study and design of the irrigation method to be adopted,

- Study and design of the irrigation system layout and associated structures,

- Design of the different conveyance canals,

- Planning and design of the different irrigation and drainage structures,

- Preparation of the longitudinal profiles of the different irrigation and drainage canals.

# SECTION-I:  HYDROLOGY

# HYDROLOGY

## Watershed Characteristics

The Watershed has marked topographic variation. All types of slopes are present. The dominant slope class is 15-30% which covers 42.62% of the total area followed by 8-15% which is 29.1%. Flat and almost Flat and Gentle Slopping accounts 1.92 and 12.58 percent respectively, very steep slopes also cover about 12.95% of the total area. Table 1 of the watershed feasibility study report shows the slope classes and proportion of the watershed.

Certain physical properties of watersheds significantly affect the characteristics of the runoff and sediment yield and are of great interest in hydrologic analyses. The rate and volume of runoff, and sediment yield from the watershed have much to do with shape, size, slope and other parameters of the landscape. These suggest that there should be some important relations between basin form and hydrologic performance. If the basin and hydrologic characteristics are to be related, the basin form must also be represented by quantitative descriptors. These parameters can be measured from maps.

The watershed characteristics are analyzed and presented in Table 2 of the Watershed Feasibility Study Report of the same project. In summary:

- Catchment Area  = 22.59km2

- Stream Length = 9.69Km

- CN(II) = 82.05

(Extracted from the Watershed Study Report of the same project)

At the selected reference point, the area of the catchment is 22.59km2 and consists of a network of tributaries as shown in Figure 2 below.

Awajo River at the headwork site is characterized by well-defined channel system and considerable flows. It looks that the gradient of the river is getting low and hence there exists significant deposition of sediment mainly cobbles and boulders.

## Hydro-Metrological Data Availability

### Climate

Small scale irrigation project designers and planners are faced with lack of good data on the hydrology of the stream/river system that will be their water source and on local weather and climate conditions. Stream gauging stations are virtually non-existent in remote rural areas of Ethiopia; meteorological stations are almost rare. Likewise, at Wodera Kebele and in the catchment area of this project, there is no meteorological station of any level. Moreover, there are no flow data for the river near the project. Therefore, data for the hydro-meteorological analysis is taken from the nearby station and similar areas. Rainfall & temperature data are considered from D/Birhan Meteorological station. In fact, this station Is some haw far to the project area.

As per the data of the station, March – April are identified as high temperature periods whereas December–January are low temperature periods. The mean annual rainfall amount is more than 1200mm (1995 - 2006 data) and most of it Occurs from June to August.

### Rainfall Data

In order to compute the design flood for the intake structure, the daily maximum rainfall is collected from D/Birhan Metrological stations with a record of 22 years.

### River flow data

The base flow which is measured on May 2015 is 200 l/s. Since this base flow is measured during the dry months of the year,

### Upstream & Downstream utilization

Downstream of the proposed site, appreciable need for water is anticipated for locals and cattle provisions. Therefore, at least 20% of the minimum flow has to be released for downstream requirements.

For the sake of planning and design, however, the outlet for the intake is designed for a discharge of 136l/s for this project and the project is to be developed for 80 ha of land, which is most of the time achievable as the flow for most of the time is significant to support this size of command area.

## Design Flood Analysis

For the design and analysis of structures to be constructed on the river estimation of flood magnitude is an important task. This can be done using different techniques depending on the data available. For this particular case, there are no river flow data and hence the flood estimation is done using the rainfall data and applying SCS Curve Method.

### Design Rainfall computation

Based on the data of 24hr peak rainfall given in Table 1 the design rainfall, R.F is computed using Gumble’s Extreme Value Method.

### Outlier Test

Higher Limit, YH=Ymean+Kn*Sy,      Kn = 2.45 for 22 Years of data.

Lower Limit, YH=Ymean-Kn*Sy,     Kn =2.45 for 22 Years of data.

Table 2-1: Outlier test analysis

- Conclusion: The rainfall values are within the limits.

### Check for variance

After checking the outliers, the data should be checked for variability. For variability the formula used is

α=δ∩-1N*Mean*100%

Where, δn-1 = Standard deviation =7.02

N = Nr of recorded data =22

M ean= 40.35

α = Standard error

α=7.0222*40.35*100%=3.63<10%  Acceptable

Therefore the data shows no variability.

#### D-Index test

After checking the consistency of the data for higher and lower outlier, the 22 year’s data is obtained as representative for the analysis using D-index. The D-Index test is believed to be the better goodness to fitness in many literatures. Hence in this study it was used to determine the best statistical distribution to estimate the peak rainfall. The D-index for the comparison of the fit of various distributions is summarized as follows.

D-Index=(1Xm)*i=16Abs(Xi-Xi')

Where Xi and Xi’ are the ith highest observed and computed values for the distribution respectively

Table 2-2: Test for goodness to fit using D-index

The design rain for this distribution has been selected as the best fit for this study

The design rainfall using Gumble’s Method is given as

Where Rf  = Design rainfall

Rmean = average of all values of annual heaviest fall = 40.35 mm

σn-1 =  standard deviation of the series = 7.02 mm

, T= Return period = 50 years

Yn, Sn = constant found from Gumble’s extreme value distribution table for N= 22 Years

- Yn = 0.53and Sn = 1.08

- Point Design Rainfall = 62.22 mm

The design rainfall at points for 50 years return period is 62.22 mm and the areal design rainfall is calculated in the following section.

### Peak Discharge Determination

#### General

The River is not gauged river. The design flood is calculated by using SCS unit hydrograph method. Thus, it is preferred to base the flood analysis on rainfall data, which are better both in quantity and quality of data. In the hydrologic analysis for drainage structures, it must be recognized that there are many variable factors that affect floods. Some of the factors that need be recognized and considered on an individual site by site basis are; rainfall amount and storm distribution; catchment area, shape and orientation; ground cover; type of soil; slopes of terrain and stream(S); antecedent moisture condition; Storage potential (over bank, ponds, wetlands, reservoirs, channel, etc.)

#### Peak flood analysis by SCS unit hydrograph method

Design flood is calculated SCS (The United States Soil Conservation Service). This method is widely adopted and more reliable method for flood estimation. The approach considers, watershed parameters, like Area, Curve number, and time of concentration.

#### Time of concentration (Tc)

Time of concentration has been calculated by taking the stream profile of the longest streamline and dividing it in to different elevation. Kirpich formula is adopted for computation.

Table 2-3: Determination of Time of Concentration

| Reach | length (km) | Vertical Distance | slope | Time of concentration (hr) |
| --- | --- | --- | --- | --- |
| 0 | 0 | 0.0 |  | 0.000 |
| 1 | 1368.1 | 68.00 | 0.050 | 0.268 |
| 2 | 3595.39 | 69.00 | 0.019 | 0.814 |
| 3 | 5042.42 | 34.00 | 0.007 | 1.580 |
| 4 | 7696.61 | 35.00 | 0.005 | 2.547 |
| 5 | 9286.28 | 31.00 | 0.003 | 3.315 |
| Total | 26988.8 | 237.0 |  | 6.85 |

The formula is,

Tc=0.948L13H10.385+L23H20.385+…+Ln3Hn0.385

- Tc = 6.85	Since Tc > 3hr., duration of excess rainfall difference, D = 1hr.

- Time to peak,

Tp=D2+0.6*Tc  = 5.61hr

- Base time,

Tb=2.67*Tp  = 14.9

- Recession time,

Tr=1.67*Tp  = 9.3hr.

#### Curve number (CN)

Curve number (CN) is achieved based on USSCS method by watershed characterization in terms of land cover, treatment, hydrologic condition and soil group. From the watershed analysis curve number at condition II =82.05 since peak rainfall is found at an antecedent moisture condition III state, this value has to be changed to antecedent moisture condition III.

- Conversion factor = 1.150

- CN Condition (III) = (Factor from Table x CN condition II) =82.05*1.150 = 91.81. For detail analysis of the computation, Refer Excel file, attached here with.

#### Area Rainfall

As the area of the catchment gets larger, coincidence of all hydrological incidences becomes less and less. This can be optimized by changing the calculated point rainfall to aerial rainfall. The conversion factor is taken from standard table that relate directly with the size of watershed area and type of the gauging station. (IDD manual)

For the case of Awajo Intake irrigation project,

- Total watershed area = 22.59 Km2

- Type of gauging station = Daily rainfall (24 hr.)

- Aerial Rainfall = (Point Rainfall) x (Conversion factor)

#### Run off Analysis

Input data:

Design Point Rainfall = 62.22mm

Curve number at antecedent moisture condition III =91.81

Catchment Area, A = 22.59 Km2

Tc = 8.52hr, D = 1hr., Tp = 5.61 hr; Tb = 14.9 hr; Tr = 9.3 hr.

Direct run-off,

- Q=(I-0.2*S)2(I+0.8*S)

- Where, I = Rearranged cumulative run-off depth (mm

S = Maximum run off potential difference,

- S=25400CN-254

Peak run-off for incremental;

- Qp=0.21*(A*Q)Tp

- Where,   A = Catchment area = 22.59 Km2

- Tp = Time to peak (hr)

- Q = Incremental run-off (mm)

Table 2-4: Runoff analysis

Table 2-5: Hydrograph coordinates

Figure 2-2: Complex Hydrograph

- From the analysis, the 50 year return period design run off is 74.13 m 3/s

### Tail Water Depth Computation

Tail water depth of the river is equal to the flood depth and amount at the proposed Intake site before construction of the Intake. It is used to crosscheck peak flood estimated by the SCS unit hydrograph method with flood mark method and to see the flood feature after the hydraulic jump. During field visit, the flood mark of the river at the proposed Intake site was marked based on dwellers information and physical indicative marks. The river cross-section was surveyed.

Table 2-6: Intake Site River Cross section Coordinate Data

Table 2-7: Stage discharge analysis

Figure 2-3: Rating Curve

From the above stage discharge table and curve the maximum flood level corresponding to the computed design peak discharge is 2511.53 (1.53 m from the river bed) and it is considered as the d/s high flood level i.e. expected at the Intake axis before construction of Intake.

D/S HFL = 2511.53m asl.

- Average river bed slope

Average river bed slope of River is estimated by two different techniques. One is by end area method and the other is by using best fit line method. Designers have adopted the end area method output for further analysis.

The water level of the river is taken at different points along the river channel around the head work site. Surveying work done for 100m length. And then, average water surface slope is considered as the river bed slope. For comparison of the two procedures, refer the attached Excel file.

Figure 2-4: River profile

- Manning’s Roughness coefficient

The Manning’s roughness coefficient is taken from standard table based on the river nature. The river at the headwork site has got braded feature and curving nature. The river banks are defined. Manning’s roughness coefficient (n = 0.04) is adopted.

- Discharge of the river

Input data:

- Manning's roughness coefficient, n = 0.04

- Average river bed slope, S = 0.02

- ,  	Where, R = Hydraulic radius = (Area/Perimeter)

- Q=V*A

- See Table 2-7 River discharge computation at different stages of flow

# SECTION-II: HEADWORK DESIGN

# HEADWORK STRUCTURES DESIGN

## Headwork Site Selection

The headwork site is situated at1099669m N, 575957m E and river bed elevation of 2510 m above sea level. At this site the river course is well defined, matured with fixed width and forms nearly a U-shaped valley. At this specific site, covered by recent alluvial deposit sediment at the river bed where as the left bank is also made from silt clay soil but the right bank is covered by silt clay soil at the top for few centimeter thickness below this soil there is highly to moderately weathered aphanitic rock exposure which extend in upstream direction./ The river is flowing through a defined channel and its scouring effect on the river course is significant as its bed covered by loose, alluvial deposit which is easily erodible and transported by running water. The different sections of the stream at the proposed headwork site are described separately below:-

## River Geomorphology

It is a common fact that the river development tends to accommodate itself to the local geology that develops along the structurally weak zones like faults, joints, folds, etc. The drainage system of the study area is strongly influenced by geological structures and formations, the nature of the vegetation cover and climate. The nature of geological formations and structures has also strong influence on the development of the channel.

The present morphology of the Awajo River channel is a function of a number of processes and environmental conditions, including the composition of the bed and the banks the size and composition of the sediment moving through the channel; the rate of sediment transport through the channel and deposition on the banks and beds; and the regional degradation due to erosion processes. Both left bank and the bed are composed of loose silt clay soil and coarser alluvial sediment as the result the stream shows highly meandering nature both up and downstream from the proposed site but at the particular intake site it shows nearly straight river channel. The river has wider section in upstream direction whereas to downstream side the river section become narrower.

### River Bed condition

At the proposed headwork site the stream bed or course is well defined, nearly straight, and shows rough surface due to recent sediment accumulations and undulating appearance of bedrock outcrops. Along the intake axis, the bed is made up of two basically different geologic materials, as seen from surface observation. These are recently deposited alluvial coarse grained sediments, and underling bedrock.

The central and left areas of the bed are totally covered with the alluvial deposits. The bedrock is totally covered with the sediments at the central and left areas of the stream bed. The alluvial sediment observed at the center, left and right beds are composed of silt-sand with significant amount of gravel, boulder and cobbles. It is loose, dry (at the time of study) and easily workable. From surface geological understanding of the area, the thickness of this sediment is expected in the range 2 to 2.5m. This coarser sediment is believed to be underlain by the bedrock extension that exposed at the right bed which is clearly observed in the downstream beds.  On the other hand, the bedrock found at stream beds bed is affected by slight degree of weathering and erratically oriented joints.  The foundation of the proposed intake structure; therefore, can be lie on the bedrock after removing the top 2m to 2.5m deposits of silt sand at left side. In addition to this, the foundation of the bed bar (if proposed), along the axis should lie on the bedrock, after excavating the top alluvial deposit covering the center, and left bed of the river.

Figure 3-1: River bed at the proposed Intake site

### River Bank condition

#### Right Bank

This abutment forms nearly small ridge. It is characterized by moderately to highly weathered, jointed and fractured rock. It is covered by dark brown silt clay soil with few centimeter thicknesses on the top part of this bank. This rock is not extending in downstream direction instead replaced by dark brown silt clay soil as the river bank but in upstream side it extends for some meters. At the proposed weir/intake site the height of this bank is about 3.5m with steeply slope but to up and downstream direction it decreases from 3.5m height to 2.5m.This rock is good for anchoring the proposed intake structure and the recommended retaining wall along this side with it.

#### Left Bank

At the proposed headwork axis and immediate vicinities, the left bank forms relatively moderately to gentle slope topography. From surface observation, the bank is made up of two basic geological units. The unit is silt- sand and Aphanitic basalt. For geotechnical purpose, there are two distinct layers observed at the bank. These are top silt sand deposit and underlain massive basalt rock. The slope and peak areas of the bank are covered by the silt sand deposit material. This top unit has thickness ranges of 1m to 1.5m, decreasing towards the bank foot. It is weak and loose horizon. It is flood plain deposit having gray- brown color. It has stiff consistency and dry moisture content. The bank is now affected by flood erosion that active eroded surfaces are visible. So, it is necessary to provide some bank protection structures to prevent ongoing bank erosion.

## Sources of construction materials

During site investigation, natural construction materials required for the construction of the various proposed engineering structures at the headwork and within the farmland have been assessed, and possible quarry sites and borrow areas have been identified within the vicinity of the study area as close to the project site as possible. In addition to identifying the quality, quantity and accessibility conditions of the construction materials, ownerships of each proposed production sites have also been studied and described in this report, on separate sub-sections below. The materials needed for the construction of the structures include rock for masonry stones, aggregates (both coarse and fine), and water.

### Rock for Masonry and Crushed Coarse Aggregate

Quarry site that can be used for production of rock for masonry stone and crushed coarse aggregates has been assessed during the field work session within the vicinity of the project area at economic distance for hauling. One possible quarry site has been identified along the right side ridge following the main canal route from chain age100m to 2000m. Here moderately weathered rhyolite rock exposed and forms a continuous ridge parallel to the main canal. It is believed that below this weathered rock, fresh portion of the rock is found and can be used for the intended purpose.

In this field study, another sources for rock also proposed. The first one is from the stream bed of awajo River. The bed is covered with very large quantity of boulders to cobble sized rocks of different types. From the headwork to about 1 to 2km distance especially in the up-stream direction large amount of such sediments can be collected by considering only strong black cored basaltic rock fragments. These fragments are strong, fresh and used for intended purpose.

### Fine Aggregates

Borrow areas for fine aggregate or natural sand have been assessed starting from the project stream itself. Natural deposits of such materials couldn’t be found when assessed within the beds of the stream in the project area; rather very coarser sediments and rock exposures are found covering almost the entire bed of theAwajo stream. Seeing to this nature of the stream, other distant streams have been explored to identify the best source areas for fine aggregate or natural sand that can be used for this particular project. During exploration of this natural sand, at a distant one stream was identified as a possible source of fine sand. The stream is known as ‘Jewuha’. It is located at about 120km from the project site, within Kewuat Woreda. A potential source area within the Jewuha stream bed is located alongside the main Showa Robit-Ataye Asphalt road, near to a small village of Jewuha. Here, there are local legalized sand miners associations, and the sand is acquired from them through negotiation and agreement.

Both visual investigation and laboratory testing of the sand deposit found in the proposed area have been done. In the area good quantity, which suffices the need of this project, is available. The deposits are dominated by sand sized transported alluvial sediments, though some amount of fines and over sizes are also observed. The individual grains of sand are mainly originated from basalt rock and reveals high crushing resistant that the sand has good quality in this respect.

### Water

Water for construction purposes can be found from the project river, itself. The stream is perennial throughout the year that there is some amount of flow along its course. During this field time the stream flow was more than 200L/second.

## Headwork Type Selection

Looking the availability of natural construction materials and considering the river features and expected flood amount, Intake is chosen. As it is:

- Simple for construction

- There is significant bed load (boulder effect) in the river.

### Hydraulic Design of Headwork Structure

#### Intake Height Determination

The following major factors have been seen in determining the weir/intake crest level:

- Maximum  command area elevation

- Deriving head of the intake structure

- Main canal slope

- Loss

- Lowest Point of river center

#### Base flow of the River

The study team has assessed that the stream is not used for irrigation along its entire course except at the proposed diversion site where farmers are using the stream for traditional SSI (Irrigation Infrastructure Report). Study team has calculated flow of the river at the Intake site as 100 l/s.  Out of this 90 l/s will be required for the proposed scheme and the rest will be released for downstream. The purpose of releasing the10 l/s and other d/s spring water to downstream is for the sake of downstream users.

#### Canal outlet level

The head regulator is provided on the left side .The sill level of this head regulator is fixed from different angle observations. The main conveyance system is more than 2km which passes two gullies and undulating alignment. Hence this level is fixed based on the optimum route alignment and the maximum irrigated command level including minor and major losses criteria. Based on this condition, the sill levels of the outlets are controlled by deducting the proposed flow depths and free board values from the bed bar crest levels which are equivalent to the river bed levels at both locations. Thus, the outlet sill level at headwork is found as 2510.05m a.s.l.

• Outlet capacity

The minimum command area is determined by the minimum flow of the river. But the canal capacity should be determined for maximum command area and the corresponding discharge. In this case the outlet capacity is fixed considering maximum duty and command area and 1.5 correction factors are considered to account the variation of duty.

- Outlet capacity = Duty x command area x correction factor

Where, maximum duty for 18 hr irrigation = 1.2 L/s/ha

Command area = 100ha.

- Outlet capacity = 1.2 L/s/ha x 100ha = 120 L/sec, Say 150 L/s.

• Outlet size

From the weir/intake discharge formula the outlet size is determined as follows

Q = CLHe3/2

Where;  C = coeff. of discharge = 1.7

L = Length of water way (m)

He = head above sill level (neglecting the velocity head) = 0.4m

⇒Adopt water way length = 0.6m

Hence, provide an outlet size of 0.60m x 0.60m (length x height) .The gate of the off take canal is to be vertical sheet metal of 0.6m x 0.6m for the closure of the opening space. Provide some extra dimensions for groove insertion. Gross area of sheet metals for the off take canal gate will be 0.7m x 0.7m (allowing 5cm insertion for grooves and above the weir/intake crest level). The grooves are to be provided on the walls using angle iron frames at the two sides of the gate openings.

Trash racks of diameter 20mm with c/c spacing of 15cm has to be provided u/s of the gate to prevent entry of debris to the canal.

The intake dimension is computed by using rectangular notch formula is given by:-

Q = 2/3Cd*(2*g) ½*B*H (3/2)

Where: Q=diverted discharge= the required discharge, Qr, 0.204m3/sec

Cd=Coefficient of discharge, Cd=0.6

B=Width of intake, 0.6m

H=Height of the intake =flow depth above sill level

G=Acceleration due to gravity

Therefore for a width of 0.6m

H= (Q*3/ (2*Cd*(2*g)1/2*B))2/3=0.361m. Including free board and 0.005m clearance on both sides, then, 0.6m width & 0.60m height has to be provided.

#### Breast Wall and Operation Slab

A vertical raised gate is designed for the head regulator. These gates slide over the breast wall using spindle during opening and closing.

For easy operation of these gates, operation slab is provided. The size of the operation slab is fixed from the point of construction and free movement. The size of the operation slab is shown in the drawing for both breast wall and operation slab with thickness 0.2m.

The thickness of the breast wall is also the same as that of the operation slab. The nominal thickness is fixed from the point of construction rather than the imposed load. The thickness required for the imposed load is less than the nominal value and treated as cantilever retaining wall. For the breast wall, the minimum reinforcement area is taken as 0.15% along the respective direction.

Hence area of steel per one meter width,

# Asteel=0.0015*W*t, where, w= width & t=thickness both in cm

Asteel=0.0015*1000*200   =300mm^2

Asteel=300mm2, Provide  12 @ c/c 200 mm

I. Checking the appropriateness of spacing:-Considering cover thickness of 50 mm, effective depth, and de= t-(50+/2) =200-(50+12/2) =144. Hence, spacing of reinforcement should be less of the following values:-

- Three times the effective depth =3*144=432mm

- 450mm

The spacing of bars, which is 200mm, is less of the above listed values. Hence, it is acceptable!

The actual arrangement of angle irons, spindles, shafts and operation slab including other components is shown on the design drawing. For each arrangement and further information, refer to the design drawing.

#### Bed bar provision

Although it is fractured, the headwork sites are made up of rock as the geological report describes and it can be seen from photos. The river beds at these locations are covered with alluvial deposits to some 2m deep; hence, it is very important to provide bed bar up to a scour depth of 2.5 m or more below river bed to key them with the parent rock. These structures will stabilize the beds of the river to keep or to maintain the levels and the flow direction as much as possible throughout the design period of the project.  The bed bar has 30m length. With the above 30 lengths the bed bars connect the banks at the bed levels. These structures are inclined at 32 degrees from the orthogonal headwork axes towards the upstream direction.

#### Retaining Walls

#### At the two ends of the diversion intake, walls have been provided to safeguard the structure from scour at the ends and also as a facility to the canal outlet operation and maintenance at the canal outlet portio

Table 3-2: Typical design of masonry retaining wall

## Bill of Quantity and Cost Estimation

The quantities of the various items have been worked out as per the final design and final drawings prepared for the scheme. The unit rates analysis has been carried out based on the data available in the vicinity of the project area.

Bill No. 1- General Items

Bill No. 2- Headwork Structure

# SECTION-III: IRRIGATION AND DRAINAGE SYSTEMS INFRASTRUCTURE

# IRRIGATION AND DRAINAGE SYSTEMS DESIGN

## Irrigable Area Description

### Topography

Topography is an important factor for the planning of any irrigation project as it influences method of irrigation, drainage, erosion, mechanization, and cost of land development, labour requirement and choice of crops.

The topographic feature of the project command area is mainly sloping type. Its elevation range is from 2456 to 2387 meters above sea level. The slope gradient also ranges from gently sloping (3%) to strongly sloping (12%). However, it has identified to be suitable for surface irrigation. Nevertheless, it requires soil and water conservation measures or structures (i.e. constructing bunds, bio-physicals, check dams, artificial water ways, etc).

The project command area is situated at the left side of Awajo River The natural topographic feature of the command area has inclined from the North-East to the South-West direction.

### Climate

As per the hydrological analysis and on the basis of the traditional Ethiopian Agro-Ecological Zones (MOA, 2001), the UGDWIP area is basically classified as Moist Woina Dega (sub-moist cool) agro-ecological zone, indicating better moisture condition in the area in wet seasons. There is no belg rain season in the project area. Despite the fact that the Meher rains are considered adequate, there is notable variation in terms of onset, distribution and withdrawal from year to year affecting crop production in general and crop productivity in particular.

As the project site has no its own meteorological station,Debrebrhan (for rainfall and minimum and maximum temperature) and Debrebrhan (for relative humidity, wind speed and sunshine hour) meteorological stations data were used for the project study as long as these stations are relatively near to the proposed command area. In general, the sources of meteorological data are the National Meteorology Service Agency (NMSA).

### Soil characteristics

Soil properties (physical, chemical, etc.) greatly influence the growth and thereby yield of crops which is grown. The command area has predominantly clay textured soils which can be classified as moderately drained soil.  Most of the study area soils are categorized as deep soil (1-1.5 meter depth). Soils of the command area are suitable for most of the selected crops to be grown (for further detail see the Agronomy Study of the same project.

### Existing Irrigation Practices in the Project Area

The pressure of survival and the need for additional food supplies to meet the demands of the increasing population is necessitating a rapid expansion of irrigation schemes. Thus, irrigation is becoming a basic part of well developed agriculture wherever there is water and irrigable land potential. Accordingly, traditional irrigation practices are not under taken by individual farmers that use the river flow to the left side is with laborious temporary diversions. So, the farmers in the project area are very much interested in the idea of modern scheme.

## Irrigation Water Requirement

### Crop Water Requirement (CWR)

The calculation of crop water requirement is a very important aspect for planning of any irrigation project. Several methods and procedures are available for this. The Food and Agriculture Organization (FAO) of the United Nations has also made available several publications on this subject and other issues related with this. The computer program available in FAO Irrigation and Drainage Paper No. 56 “CROPWAT” has been used for the calculation of Crop Water requirement. This program is based on Penman-Monteith approach and procedures for calculation of crop water requirements and irrigation requirements are mainly based on methodologies presented in FAO Irrigation and Drainage Paper No. 24 “Crop Water Requirements” and No. 33 “Yield Response to Water”.

The corresponding values of the crop water requirements of the proposed crops of the project are presented in the Agronomy Study of the same project.

### Irrigation efficiency (Ep)

To complete the evaluation of the demand, the efficiency of the water distribution system and of application must be known.

The gross requirement of water for irrigation system is very much dependent on the overall efficiency of the irrigation system, which in turn is dependent on several factors: Method of irrigation, type of canal (Lined and/or Unlined), method of operations (simultaneously and continuous or Rotational water supply), and availability of structures (for controlling and distribution and measuring and monitoring).

On the basis of these factors, the project has planned to impose surface irrigation method (using furrows). The canal system is lined hence, the conveyance efficiency has been estimated to be 90%, distribution efficiency 80%, and field application efficiency 65%. As a result of these the overall irrigation efficiency has been estimated to be 50%. According to soil Lab result, soils of the command area are predominantly characterized as clayey soils.

### Irrigation duty

Irrigation duty is the volume of water required per hectare for the full flange of the crops. Moreover, it helps in designing an efficient irrigation canal system.

The area, which will be irrigated, can be calculated by knowing the total available water at the source and the overall duty for all crops required to be irrigated in different seasons of the years.

The proposed cropping pattern of Awajo intake irrigation project has showed a maximum net irrigation water requirement (NIWR) in the month of March with the amount of XXX mm/day for 24 working hours (for overall proposed crops).

However, for the designing of the irrigation water application and the flows in the entire canal systems, from the overall proposed crops the one that has maximum NIWR was used for irrigation duty calculation. Accordingly, potato has showed the maximum NIWR (i.e. XXX mm/day); and hence taken for the irrigation project duty calculation as indicated here below:

For the designing of the project, the GIWR is given as follows:

GIWR = 5.86/0.50 = 11.72 [mm/day]

The GIWR, 11.72 mm/day, represents the daily quantity of water that is required to be applied. This water quantity is also used for the determination of the canal discharge in consideration of the time of flow and is defined as the duty, expressed as l/s/ha.

The duty is calculated by:

Duty (D) = GIWR × 1000 × 10 / (t × 60×60)

Where; Duty – the duty [l/s/ha]

GIWR – Gross Irrigation Requirement [mm/day]

t – Daily irrigation or flow hours [hrs]

The duty for the GIWR of 11.72mm/day and 14 hours of daily irrigation time (t = 14), is supported to be used with furrow irrigation method. Hence, Duty for 18 working hours, as the site is nearer to farmers’ village and local farmers have experiences in irrigation, is computed as follows:

D = (11.72 x 1000 x 10) / (14 x 3600)

### Irrigation methods

Among the different irrigation systems furrows irrigation system will be used for the project area; and the irrigation water will be obtained from Awajo River and by constructing diversion intake and convoying the water commonly through lined canals (PC, SC, and TC) and then leading to field canals; and finally irrigation takes place mostly in furrows.

For this project, among the various irrigation methods, surface irrigation method has been selected.  Of the surface irrigation methods furrow, border and basin irrigation methods can be used to supply irrigation water to the plants/crops. However, each method has its own advantages and disadvantages. Care should be taken when choosing the method which is best suited to the local circumstances, i.e., depending on slopes, soil types, selected crop types, amount of water available, etc. of the command area.

Based on the above factors surface irrigation method has been proposed for the proposed crops in this project. The method allows applying light irrigation and can be laid out in sloping fields along the contour. Furrow irrigation method is best suited for most of the proposed and row planted crops. In general, furrow irrigation method is simple, manageable and widely practiced irrigation method. This method is suitable for row crops that cannot stand in water for long periods. The only thing required to use this method is row planting of crops. Besides, basin and border irrigation method would be used for the non-row planted crops. Rotational flow water distribution is also recommended for the project area.

## Irrigation and Drainage System Layout

The irrigation system layout for the project is prepared taking the following points into consideration besides other factors.

- A primary concern in the layout of the system is that it serves the purpose of conveying and distributing water to the command area.

- The excavation and earth fill volumes not be excessive, otherwise the construction costs can be tremendous.

- The selection of longitudinal bed slope is made taking into account the existing slopes of the terrain, so as to minimize deviations in canal routing.

- Curves in canals should not be too sharp.

The proposed irrigation system layout comprises one (1) main canal, two(2) secondary canals and seven(7) tertiary canals as shown on the layout Drawings. The main canal runs for most of its length parallel to the contours and several changes of direction are necessary to follow the topography. It crosses two main gullies, one. The main canal is masonry lined for a length of 2665 meters starting from the intake outlet to make maintenance easier since this part of the canal may be subjected to flooding during high flood flows.

### Conveyance System

The conveyance system consists of one (1) Main canal to irrigate total command area of 80ha. The main canal starts from Water abstraction site on left side and conveys water for a length of 2800Km.

Main canal is aligned along contours and supplies to two secondary unit and seven tertiary canals.

## Design of the Canal System

Flow Depth and Section Capacity

The earthen canals have been designed with a trapezoidal shape and the lined ones with rectangular x-section using Manning's Formula:

Where Q= discharge (m3/s)

R= Hydraulic radius (Flow area/wetted perimeter)

S= Hydraulic gradient

n= Manning's roughness coefficient, n=0.022 is adopted for the earth channels and n=0.018 for the masonry lined part of the main canal

### Main Canal

The main canal is designed for a discharge of 108 l/s and depending on the site specific condition, appropriate slope is provided. Hydraulic parameters of the main canal are shown below.

At a Chainage of 717m, there are Flume structures.

Table 4-1: Hydraulic Parameters of main canal

| CHANNEL | Area | Chainage | Q required | B | D | A | P | R | S | n | V | Qdesign |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| MC-1 | 80 | 0+000-1+826 | 0.1088 | 0.6 | 0.35 | 0.21 | 1.30 | 0.161 | 0.001 | 0.018 | 0.5204 | 0.1088 |
| MC-1 | 80 | 1+985-2+220 | 0.1088 | 0.6 | 0.35 | 0.21 | 1.30 | 0.161 | 0.001 | 0.018 | 0.5206 | 0.1090 |
| MC-1 | 45 | 2+220-2+670 | 0.0612 | 0.5 | 0.27 | 0.14 | 1.04 | 0.130 | 0.001 | 0.018 | 0.4513 | 0.0613 |
| MC-1 | 31 | 2+670-2+890 | 0.0422 | 0.5 | 0.21 | 0.10 | 0.91 | 0.113 | 0.001 | 0.018 | 0.4103 | 0.0422 |

B= Canal bottom width                         Qr = Required discharge

D= Full supply depth                             Qd= Designed Discharge

FB= Free board

S= Side slope

V= Velocity

### Secondary Canals

Table 4-2: Hydraulic Parameters of secondary canals

| SEC 1 | 35 | 0+000-0+010 | 0.0476 | 0.4 | 0.05 | 0.02 | 0.50 | 0.039 | 0.143 | 0.018 | 2.4308 | 0.0477 | 0.000 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SEC 1 | 35 | 0+010-0+030 | 0.0476 | 0.4 | 0.14 | 0.06 | 0.68 | 0.082 | 0.007 | 0.018 | 0.8576 | 0.0479 | 0.000 |
| SEC 1 | 35 | 0+030-0+070 | 0.0476 | 0.4 | 0.09 | 0.04 | 0.59 | 0.064 | 0.020 | 0.018 | 1.2596 | 0.0476 | 0.000 |
| SEC 1 | 35 | 0+070-0+120 | 0.0476 | 0.4 | 0.07 | 0.03 | 0.54 | 0.053 | 0.045 | 0.018 | 1.6647 | 0.0476 | 0.000 |
| SEC 1 | 35 | 0+120-0+170 | 0.0476 | 0.4 | 0.06 | 0.03 | 0.53 | 0.048 | 0.067 | 0.018 | 1.8920 | 0.0477 | 0.000 |
| SEC 1 | 35 | 0+170-0+200 | 0.0476 | 0.4 | 0.08 | 0.03 | 0.55 | 0.055 | 0.037 | 0.018 | 1.5487 | 0.0471 | 0.000 |
| SEC 1 | 35 | 0+200-0+220 | 0.0476 | 0.4 | 0.14 | 0.06 | 0.68 | 0.082 | 0.007 | 0.018 | 0.8578 | 0.0479 | 0.000 |
| SEC 1 | 35 | 0+220-0+270 | 0.0476 | 0.4 | 0.08 | 0.03 | 0.56 | 0.056 | 0.037 | 0.018 | 1.5639 | 0.0486 | 0.001 |
| SEC 1 | 35 | 0+270-0+300 | 0.0476 | 0.4 | 0.06 | 0.02 | 0.51 | 0.044 | 0.091 | 0.018 | 2.0978 | 0.0478 | 0.000 |
| SEC 1 | 35 | 0+300-0+330 | 0.0476 | 0.4 | 0.07 | 0.03 | 0.54 | 0.052 | 0.050 | 0.018 | 1.7283 | 0.0484 | 0.001 |
| SEC 1 | 35 | 0+330-0+360 | 0.0476 | 0.4 | 0.16 | 0.06 | 0.71 | 0.088 | 0.005 | 0.018 | 0.7759 | 0.0485 | 0.001 |
| SEC 1 | 35 | 0+360-0+400 | 0.0476 | 0.4 | 0.10 | 0.04 | 0.59 | 0.065 | 0.020 | 0.018 | 1.2639 | 0.0482 | 0.001 |
| SEC 1 | 35 | 0+400-0+524 | 0.0476 | 0.4 | 0.06 | 0.02 | 0.51 | 0.043 | 0.100 | 0.018 | 2.1727 | 0.0483 | 0.001 |
| SC2 | 15 | 0+000-0+020 | 0.0204 | 0.4 | 0.03 | 0.01 | 0.45 | 0.023 | 0.200 | 0.018 | 1.9959 | 0.0205 | 0.000 |
| SC2 | 15 | 0+020-0+100 | 0.0204 | 0.4 | 0.03 | 0.01 | 0.47 | 0.028 | 0.100 | 0.018 | 1.6193 | 0.0211 | 0.001 |
| SC2 | 15 | 0+100-0+120 | 0.0204 | 0.4 | 0.05 | 0.02 | 0.51 | 0.042 | 0.020 | 0.018 | 0.9563 | 0.0206 | 0.000 |
| SC2 | 15 | 0+120-0+210 | 0.0204 | 0.4 | 0.03 | 0.01 | 0.47 | 0.028 | 0.100 | 0.018 | 1.6193 | 0.0211 | 0.001 |
| SC2 | 15 | 0+210-0+260 | 0.0204 | 0.4 | 0.02 | 0.01 | 0.45 | 0.021 | 0.250 | 0.018 | 2.1423 | 0.0206 | 0.000 |
| SC2 | 15 | 0+260-0+356 | 0.0204 | 0.4 | 0.03 | 0.01 | 0.47 | 0.028 | 0.100 | 0.018 | 1.6193 | 0.0211 | 0.001 |
| SC3 | 30 | 0+000-0+060 | 0.0408 | 0.4 | 0.03 | 0.01 | 0.46 | 0.028 | 0.400 | 0.018 | 3.2077 | 0.0411 | 0.000 |
| SC4 | 30 | 0+060-0+110 | 0.0408 | 0.4 | 0.05 | 0.02 | 0.50 | 0.040 | 0.100 | 0.018 | 2.0502 | 0.0408 | 0.000 |
| SC5 | 30 | 0+110-0+120 | 0.0408 | 0.4 | 0.06 | 0.02 | 0.51 | 0.045 | 0.067 | 0.018 | 1.8033 | 0.0414 | 0.001 |
| SC6 | 30 | 0+120-0+195 | 0.0408 | 0.4 | 0.08 | 0.03 | 0.56 | 0.057 | 0.025 | 0.018 | 1.3040 | 0.0418 | 0.001 |
| SC7 | 30 | 0+195-0+390 | 0.0408 | 0.4 | 0.16 | 0.06 | 0.72 | 0.089 | 0.003 | 0.018 | 0.6386 | 0.0408 | 0.000 |
| SC8 | 30 | 0+390-0+420 | 0.0408 | 0.4 | 0.06 | 0.02 | 0.51 | 0.045 | 0.067 | 0.018 | 1.8033 | 0.0414 | 0.001 |
| SC9 | 30 | 0+420-0+450 | 0.0408 | 0.4 | 0.05 | 0.02 | 0.50 | 0.040 | 0.100 | 0.018 | 2.0502 | 0.0408 | 0.000 |
| SC10 | 30 | 0+450-0+500 | 0.0408 | 0.4 | 0.06 | 0.02 | 0.51 | 0.045 | 0.067 | 0.018 | 1.8033 | 0.0414 | 0.001 |

Figure 4-1: Typical Cross Section of secondary canals

### Tertiary canals

| TC-1 | 7.9 | 0+000-0+220 | 0.0140 | 0.3 | 0.16 | 0.05 | 0.74 | 0.064 | 0.002 | 0.022 | 0.2981 | 0.0142 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TC-2 | 7 | 0+00-0+351 | 0.0124 | 0.3 | 0.15 | 0.05 | 0.73 | 0.063 | 0.001 | 0.022 | 0.2713 | 0.0124 |
| TC-3 | 5.4 | 0+00-0+210 | 0.0095 | 0.3 | 0.10 | 0.02 | 0.58 | 0.043 | 0.005 | 0.022 | 0.3931 | 0.0098 |
| TC-4 | 6 | 0+00-0+215 | 0.0106 | 0.3 | 0.16 | 0.05 | 0.75 | 0.065 | 0.001 | 0.022 | 0.2322 | 0.0112 |
| TC-5 | 3 | 0+00-0+221 | 0.0053 | 0.3 | 0.09 | 0.02 | 0.56 | 0.040 | 0.003 | 0.022 | 0.2663 | 0.0060 |
| TC-6 | 5 | 0+00-0+332 | 0.0088 | 0.3 | 0.12 | 0.03 | 0.64 | 0.050 | 0.002 | 0.022 | 0.2775 | 0.0089 |
| TC-7 | 2.6 | 0+00-0+215 | 0.0046 | 0.3 | 0.06 | 0.01 | 0.48 | 0.028 | 0.010 | 0.022 | 0.4185 | 0.0056 |
| TC-8 | 3.5 | 0+00-0+491 | 0.0062 | 0.3 | 0.10 | 0.03 | 0.60 | 0.045 | 0.002 | 0.022 | 0.2558 | 0.0068 |
| TC-9 | 3.4 | 0+00-0+130 | 0.0060 | 0.3 | 0.13 | 0.04 | 0.68 | 0.056 | 0.001 | 0.022 | 0.1710 | 0.0064 |
| TC-10 | 2.5 | 0+00-0+168 | 0.0044 | 0.3 | 0.07 | 0.02 | 0.50 | 0.032 | 0.005 | 0.022 | 0.3220 | 0.0051 |
| TC-11 | 2.6 | 0+00-0+279 | 0.0046 | 0.3 | 0.09 | 0.02 | 0.55 | 0.038 | 0.002 | 0.022 | 0.2298 | 0.0048 |
| TC-12 | 3.3 | 0+00-0+375 | 0.0058 | 0.3 | 0.08 | 0.02 | 0.53 | 0.035 | 0.005 | 0.022 | 0.3434 | 0.0063 |
| TC-13 | 11.2 | 0+00-0+331 | 0.0198 | 0.3 | 0.21 | 0.07 | 0.88 | 0.083 | 0.001 | 0.022 | 0.2736 | 0.0201 |
| TC-14 | 5.8 | 0+00-0+263 | 0.0103 | 0.3 | 0.15 | 0.05 | 0.74 | 0.064 | 0.001 | 0.022 | 0.2299 | 0.0109 |
| TC-15 | 4 | 0+00-0+300 | 0.0071 | 0.3 | 0.09 | 0.02 | 0.54 | 0.038 | 0.005 | 0.022 | 0.3607 | 0.0074 |
| TC-16 | 7.6 | 0+00-0+301 | 0.0134 | 0.3 | 0.12 | 0.03 | 0.64 | 0.051 | 0.004 | 0.022 | 0.4191 | 0.0139 |

In the layout system there are seven (7) tertiary canals, the designed discharge is determined based on the duty of irrigation.  The sections of the canals are determined by using manning’s formula, and they are trapezoidal section. The hydraulic characteristics is Presented in Table 5.3 below.

Figure 4-2: Typical Cross Section of tertiary canals

Table 4-3: Hydraulic Parameters of tertiary canals

### Field Canals

As shown in the layout, field canals run across the contours and hence face relatively steeper gradient. The discharge of most of the field canals is very small and this is taken as an advantage to cope up with the relatively steeper gradient. Figure 15 below shows a typical field canal x-section. As much as possible field canals shall be made in fill in order to easily irrigate the adjacent command area. As can be seen from the layout, majority of the filed canals can be used to irrigate both sides of the command area depending on the condition of the individual plots of land owned by individual farmers.

Figure 4-3: Typical Field Canal X-section

## Drainage Canals

### Tertiary canal drainage canals

Drainage Canals aren't provided for the Tertiary canals, because all of the tertiary canals are entered to gullies. It is assumed that there is no excess irrigation water during dry season. For the rainy season the water that comes through the tertiary canals directly entered to gullies. So, Gully protection is necessary at the tertiary canal out fall. For the protection gabion is recommended.

### Design of Division box

At different points of the main and secondary canals division boxes are provided which divert the flow to the secondary canal and tertiary canals. Gate should be provided at the outlet of the boxes. For detail refer the drawing.

Figure 4-5: Typical Division Box plan

Using broad crested formula,

Q= CL (h)3/2

Where; Q= discharge over rectangular weir/intake (opening), m3/s

C = discharge coefficient, c= 1.7

L= effective length of crest form in m

h= over flow depth, m

Assuming equal discharge coefficient & sill height for two or three dividing canals, the proportion becomes.

Q1/ Q2= Q2/ Q3 =L1/ L2 = L2/ L3

Where Q1= is flow in canal 1

Q2 = is flow in canal 2

Q3 = is flow in canal 3

L1= is effective crest length of weir/intake sill across opening to canal 1

L2= is effective crest length of weir/intake sill across opening to canal 2

L3 = is effective crest length of weir/intake sill across opening to canal 3

Q1= CL1 (h)3/2,

L1 = Q1/Ch3/2

L2 = L1*Q2/ Q1

L3 = L1*Q3/ Q1

The depth of (height of) the division box,

D = d + fb

The width of the division box,

B = b + 2*m*D

Where b= base width of the incoming canal

D = total canal depth of the incoming canal

Table 4-6: Hydraulic parameters of Division Boxes

### Gully crossing structure

One flume structures are provided on the main canal and one gully crossing structures are provided on se rectangular reinforced concrete slab. The slab is reinforced with 10mm @150mmc/c the length of the slab is depends on width of the gully and slab width which is the same as the respective canal bed width, for further See drawing document. and 4 Escape canal are provided on MC to control u/s flood according to topographic condition of the project to protect mc.

## Irrigation Infrastructure Bill of Quantities and Cost Estimate

# CONCLUSION AND RECOMMENDATION

- The infrastructure of this project area is designed to irrigate about 80ha of land by taking its supply from the Awajo diversion weir/intake irrigation project. The maximum duty of the command area for 14 hours per day irrigation with overall project efficiency of 45%.  The method of irrigation of the project area is furrow surface irrigation in which the main and tertiary canals are working continuously where as the field canals within a tertiary block are working rotational system.

- As the dominant soil type is clay soil, the main canal and secondary canal system is designed to be masonry.

- The reason why the main canal and secondary canal is to be lined up to the end is to avoid the siltation problem, time saving to reach at the tail part, reduce maintenance cost.

- On the main unit of the irrigation systems, some are associated with chute. They are designed as far as possible to be partially filled and cut. The layout is designed as far as possible to avoid cross-structures within them.

- The design of the canal dimensions of the irrigation canal is done by applying the manning’s uniform flow equation. The variable of the hydraulic parameters are calculated using iteration or flow master program.

- As soils of the command area are predominantly clay textured; and hence water and soil management measures should be undertaken; and optimum moisture content should be maintained to improve workability of the soil during land preparation and planting time.

The following recommendations are drown:

- For better performance and long service year of the project regular inspection and maintenance is highly required.

- Farmers training, how to operate and maintain the project structures as a whole and available and water resources has a paramount important.

- The irrigation hours per day and per week should be flexible based on base flow amount of each week or month.

- Close supervision of the construction should be made to modify (if need be) each Components of irrigation system based on specific site conditions.

# OPERATION AND MAINTENANCE

## General

The main objective of the operation and maintenance aspect of an irrigation scheme is to facilitate the timely delivery of the required irrigation water to farms and to keep the irrigation system in an optimum operating condition.  This section therefore, discusses the main functions of the subject matter under consideration for the scheme.

## Operation of the Head Works

Operation at the diversion intake mainly focuses on the diversion of a controlled flow of river water, timely cleaning of floating debris in front of intake and removal of sediment deposits in front of the weir and intake structures.

## Irrigation System Operation

The operation of the irrigation system depends mainly on the method of water delivery at farm level.  Surface irrigation method is the recommended type of water distribution and application method for awajo diversion irrigation scheme.

The farmers would organize themselves and form groups in order to handle the water management.  Since flow is low Rotational water distribution would be applied within the group.  The rotational distribution is then to distribute water by turn to the whole scheme according to the timely need of crop water requirement. For better and efficient water management, crop diversification should be avoided within a group.  This would reduce the complexity of water distribution system of the scheme during one irrigation season. At farmers’ level of operation, a constant flow and variable irrigation time is advisable.

The operation of the irrigation system is continuous for 14 hours per day in main, secondary and tertiary canals where as field canals within a tertiary block are operating in rotational system with each other for irrigation hours proportional to their size.  Since the tertiary canal discharges are within the manageable range and the irrigation canal structures are accordingly designed for simple operation, the farmers can open and close easily whenever they required.

## Maintenance Requirement

The canal system of the project is earthen canal except main canal and secondary canal, which is susceptible to siltation, erosion, growth of weeds and or breaching problems.  Though the canal sections are designed for non- silting and non scouring conditions, the above mentioned problems are unavoidable and hence periodical and yearly inspection and maintenance of canals and structures are obligatory in order to fulfill the design objective of the project area.

The maintenance tasks are categorized into two types: - routine activities, and repairs. The routine maintenance activities that are carried out periodically include:-

- Regular cleaning of sediments and weeds from canals and drains;

- Inspection and lubrication of gates; and

- Maintenance of cracked lined canals, regulating and control structures.

Repair works include task carried out more frequently and quickly, and include those tasks that are generally unpredictable.  They also include emergency works.  The activities included in this category are:-

- Repairing overtopped or breached canals, drains, and flood protection dykes;

- Repairing jammed gates;

- Filling holes made by wild animals; and

- Reduced free board due to walking over by people and livestock.

Regular inspection of the irrigation facilities should be carried out as part of the maintenance activities.  These tasks could be carried out immediately after the end of the main rains in September and during the rainy season.  This could concentrate on the interceptor drains and the flood protection dykes, the main canal and the field drains.  The inspection of the other works like the tertiary canals, field drains, and the water control and regulating structures could be carried out as part of routine operation activities.

- Beneficiaries of the project need to have operation and maintenance budget, For O&M cost incurring entity area:

- Purchase sing of gate lubricate (grease)

- Replacing and maintenance of Stolen and damaged gates

- Repair Damages on the cross drainage structures.

The expense for O&M should be collected from the beneficiaries. Of course, much of the task is done by the labor and skill of the community. For cost incurring activities beneficiaries have to collect money based on the proportion of the command area they owned.

# REFERENCE

- FAO (1977) guidelines for predicting crop water requirements. No 24, Rome Italy

- Design of small Canal structures , USBR

- Soft copies of hydraulic structure publishing

- IDD manual

- ESRDF manual