Amhara National Regional State

Water, Irrigation and Energy Development Bureau

(BOWIED)

Feasibility Study and Detail Design

Of

Azewari-2 Small Scale Irrigation Project

Volume V: Engineering Design Report

Final Report

Feberury, 2019

Bahir Dar

Client: -   Water, Irrigation and Energy Development Bureau

(BoWIED)

Address:

P.O.Box: 88
Telephone: 0528-200853/855
Fax: 251-08-20-65-68/204676/202040

Consultant: Amhara Design & Supervision Works Enterprise

(ADSWE)

Address: P.O.Box: 1921
Telephone: +251-582-181023/ 180638/181001/181254
Fax:(058) 2180550/ (058) 2180560
E-mail:amhara design@yahoo.com

Bahir Dar, Ethiopia

FEASIBILITY STUDY & DETAIL DESIGN REPORT STRUCTURE

- Volume I: Watershed Management

- Volume II: Engineering Geology

- Volume III: Irrigation Agronomy

- Volume IV: Engineering Design

- Volume V: Socio Economy

- Volume VI: Environmental and social Impact Assessment

- Volume VII: Financial and Economic Analysis

- Volume VIII: Agricultural Marketing and Agribusiness Study

- Volume IX: Korch-Tejatell Micro Watershed  Management

Table of Contents								 Page

Contents

SAILENT FEATURE	x

1	INTRODUCTION	12

1.1	Background	12

1.2	Description of the Project Area	12

1.2.1	Location	12

1.2.2	Accessibility	13

1.3	Objectives of the study	13

1.3.1	Specific Objectives	14

1.4	Scope of the study	15

1.5	Methodology	15

SECTION-I: HYDROLOGY	17

2	Watershed characteristics	18

3	Hydro-metrological data availability	19

3.1	Hydro-meteorological data availability	19

3.1.1	Hydrometric data	19

3.1.2	Climate	20

3.1.3 Daily Heaviest Rainfall Data	20

4	Daily Heaviest Rainfall data availability and checking	21

4.1	Data Consistency Test	23

4.1.1	Checking Data Reliability	23

4.1.2	Data Outlier Test	23

5	Design Storm Computation	25

5.1	D-Index test	26

6	Design Flood Determination	27

6.1	Peak flood analysis by SCS method	27

6.1.1	Time of concentration (Tc)	27

6.1.2	Curve number (CN)	28

6.1.3	Areal Rainfall	28

6.1.4	Rainfall Profile	28

6.1.4	Direct Run off Analysis	29

6.2	Flood mark Method	32

6.2.1	Tail Water Depth Computation	32

6.2.2	Average river bed slope	33

6.2.3	Manning’s Roughness coefficient for the river bed and banks (average)	33

6.2.4	Discharge of the river	33

6.3	Selected Design flood	35

7	Basin Base flow, Demand and Water balance	35

7.1	Basin Base flows	35

7.2	Existing water use and Irrigation Practices	37

7.3	Upstream & Downstream utilization	37

7.1	Conclusion	38

SECTION-II: HEADWORK DESIGN	39

8	Headwork site selection	40

9	River Geomorphology	40

9.1	Left Bank conditions	40

9.2	Right bank condition	41

9.3	Stream Bed	43

9.4	Natural Construction Materials	43

9.4.1	Rock for Masonry and Coarse Aggregates	43

9.4.2	Sand	43

9.4.3	Water	44

10	Headwork type selection	44

10.1	Hydraulic design of headwork structure	44

10.1.1	Weir Height Determination	44

10.1.2	Design Base flow of the River	44

10.2	Weir Dimensioning	45

10.2.1	Weir crest length determination	45

10.2.2	Top and bottom width determination	46

10.2.3	Determination of Upstream & Downstream High flood level (HFL)	46

10.2.4	Afflux determination	48

10.3	Hydraulic Jump computation	48

10.4	Impervious floor design	52

10.5	Stability Analysis of weir	53

10.5.1	Resisting Load:	53

10.5.2	Overturning loads:	53

10.6	Design of Divide wall, Under Sluice, and Canal outlet	56

10.6.1	Under sluice	57

10.6.2	Gate for the under sluice	58

10.6.1	Design of operation slab and Breast wall	58

10.7	Design of Retaining Walls	59

10.8	Upstream Retaining/wing walls Design	59

10.9	Downstream Retaining walls Design	60

10.10	WEIR, CUTOFF, UNDERSLUICE BED & OFFTAKE RCC CAPPING	61

10.11	Design of camp facility	61

11	Bill of Quantity and cost estimation	62

12	Conclusion and recommendation	66

SECTION-III: IRRIGATION AND DRAINAGE SYSTEMS INFRASTRUCTURE	67

13	Irrigation Infrastructure	68

13.1	Irrigable command area	68

13.1.1	Topography and soil	68

13.1.2	Irrigation Efficiency and irrigation duty	69

13.1.3	Irrigation Interval	70

13.2	Irrigation & Drainage system lay out	71

13.2.1	Canal layout	71

13.2.2	Drainage system	71

13.3	Selection of Canal Design Section	72

13.3.1	Main Canal	72

13.3.2	Secondary Canal	74

13.3.3	Tertiary canal	74

13.4	Design of Irrigation canals	75

13.4.1	Main canal	75

13.4.2	Tertiary Canal	76

13.4.3	Field Canals	78

13.5	Design of Drainage System	79

13.6	Collector drain canal design	80

14	Irrigation Structures	81

14.1	Design of Division boxes and turnouts	81

14.2	Design of field canal turnouts	82

14.3	Design of Drops	83

14.4	Cross drainage	85

14.5	Design of Road Crossing Structures	87

14.6	Supper passage structure	87

15	Bill of Quantity and cost estimation	87

16	CONCLUSION AND RECOMMENDATION	91

SECTION IV: OPERATION AND MAINTENANCE	92

17	OPERATION AND MAINTENANCE	93

17.1	INTRODUCTION	93

17.2	Description of irrigation system	93

17.3	Irrigation management committee	93

17.2	OPERATION OF THE SYSTEM	94

17.2.1	Diversion System	95

17.2.3	Irrigable Land	96

17.2.4	Canal Systems	96

17.2.5	Farm Structures	101

17.3	MAINTENACE	103

17.3.1	Regular Maintenance	103

17.3.2	Periodic Maintenance	105

17.3.3	Incidental Maintenance	105

17.4	CONCLUSIONS AND RECOMMIENDATIONS	106

REFERENCE	107

List of Tables

Table 3-1:   Data availability and adequacy Gundewoyn station	20

Table 4-1:   storm Analysis for Gundewoyn Rainfall Station	22

Table 5-1: Test for goodness to fit using D-index	26

Table 6-1: Determination of Time of Concentration	27

Table 6-2: Design Rainfall Arrangement	29

Table 6-3: Direct Runoff analysis	30

Table 6-4: Hydrograph coordinates	31

Table 6-5: River discharge computation at different stages of flow, (downstream)	34

Table 10-1:  jump and tail water profiles	51

Table 10-2: weir stability analysis	55

Table 10-3: upstream Retaining wall stability analysis	59

Table 10-4: Downstream Retaining wall stability analysis	60

Table 11-1: Bill No. 1- General Items	62

Table 11-2:  Bill No. 2- Head Work Structure	64

Table 13-1:  off take sizing	72

Table 13-2: hydraulic characteristics of the main canal	76

Table 13-3  Hydraulic characteristics of the tertiary canal	77

Table 13-4 Hydraulic parameters of tertiary drain canals	79

Table 13-5 Hydraulic parameters of  collector drain	81

Table 14-1 Hydraulic characteristics of drops	84

Table 14-2: Hydraulic Parameters of Flume  at chain age 1+329	86

Table 15-1:  Bill No. 3- Engineering cost estimation for Infrastructure	87

Table 15-2:  Cost summary	91

LIST OF FIGURES

Figure 1-1:  Location map of the project area	13

Figure 2-1:  Drainage map of Azuwary-2 watershed	19

Figure 3-1 : Trend of daily heaviest rainfall in Gundewoyn NMS	21

Figure 6-1: Complex Hydrograph	32

Figure 6-2:  The river cross-section for Azuwary-2weir axis (from left to right)	33

Figure 6-3 Stage discharge curve	35

Figure 7-1:   Base flow of the river	36

Figure 9-1: Photo showing left bank	41

Figure 9-2: Right Bank and Stream Bed Geologic Formations	42

Figure 9-3: Geological x section along Azuwary-2 weir axis	43

Figure 10-1: Fraud number vs jump length chart	50

Figure 10-2  Jump versus Tail water Depth	52

Figure 13-1:   lined canal section view	73

Figure 13-2 Typical lined secondary canal fully in cut	74

Figure 13-3Typical lined secondary canal in partial fill & cut	74

Figure 13-4 : tertiary canal section	75

Figure 13-5Typical collector drain partially in fill & cut section	80

Figure 14-1: typical division box drawing	82

Figure 14-2 : Turnouts from masonry lined TC-canals (left) and from earthen TC canals (right)	83

Figure 14-3 Standard drawing of drop structures is given below	85

Figure 14-4:  gully crossing plan	86

# SAILENT FEATURE

- Project name: Azuary-2 diversion weir Irrigation Project

- Name of the stream: Azuwary river

- Location of the weir site using

- North: 1205967.6  (UTM)

- East:267575.57  (UTM)

- Zone: (Este Gojjame )

- Wereda: Goncha siso Enesie

- Kebeele: 03

- Accessibility from woreda town: 9km

- Average Altitude: 2454ma.m.s.l

- Hydrology

- Design rainfall:100.448mm

- Catchment area:80.61km2

- Longest flow path length:32.95Km

- Design flood:156.96m3/sec

- Design base flow: 110lit/s

- Diversion Weir

- Weir type,Broad crest

- Height:2.0m

- Gross crest length:17m

- Weir crest level:2455.76m.a.s.l

- U/S HFL:2458.5m.a.s.l

- D/s HFL:242457.41m.a.s.l

- Afflux:1.75m

- Under sluice

- Number of undersluicce : 1

- Sill level:2454.26m.a.s.l

- Dimension:0.1m x1m ( lengthxheight)

- Discharge amount:0.50m3/sec

- Outlet

- Sill level:2454.86m.a.s.l

- Opening dimension:

    - Right canal outlet: 0.5x0.5

- Discharge capacity:

      - Right :120 l/s

Irrigation and drainage systems Infrastructure

- Command area size: 100ha

- Design discharge of the main canal:

  - Main  canal (MC: 120l/sec )

- Canals

  - No of main canal: 1

  - No of Secondary  canal: 8

  - No Tertiary canal : 20

Project cost (without VAT)

- General Items Cost= 700,000Birr

- Headwork Cost =	3,434,336.46Birr

- Irrigation Infrastructure Cost =9,086,813.48Birr

- Total Project Cost = 		13,221,149.94Birr

- Cost per hectare = 		132,211.5Birr

# INTRODUCTION

## Background

In Ethiopia, under the prevalent rain-fed agricultural production system, the progressive degradation of the natural resource base, especially in highly vulnerable areas of the highlands coupled with climate variability have aggravated the incidence of poverty and food insecurity. The major source of growth for Ethiopia is still conceived to be the agriculture sector. Hence, this sector has to be insulated from drought shocks through enhanced utilization of the water resource potential of the country through development of small-scale irrigation.

In line with the above, in the area of irrigation efforts have been made by the government to improve the situation in the country. Amhara Water Resources Development Bureau is playing its role in the development of small scale irrigation projects in the region.

Accordingly, as part of the water sector development program, the bureau has initiated the study and design of a small scale irrigation scheme on Azuwary-2 River at 03 kebele Genet gote  and signed an agreement with Amhara Design & Supervision Works Enterprise (ADSWE) for the study and design of the project.

## Description of the Project Area

### Location

This irrigation project is located mainly at 03 Kebele, Goncha siso Wereda of East Gojjam Zone in the Amhara Region. The proposed irrigation project is to be undertaken on Azuwary-2 River and the headwork structures are specifically located at an altitude of about 2453 masl and geographical coordinates of1204926N(UTM)& 396285(UTM).

Figure 1-1:  Location map of the project area

### Accessibility

The project area is located in Goncha Siso Enesie Wereda, 03 Kebele in a specific location of Genet gote. It can be accessed along 13 km from wereda town) to the North West direction and it is accessed by vehicles using URAP project road.

## Objectives of the study

The Enterprise is required to make a complete feasibility and or detail design study by conducting a complete assessment and investigation of all available water and land resources all along the Azuwary-2 River. The service is also mainly focused in searching and selecting the best intervention /weir in this case/ technology and tries to locate it together with local users.  All the process of the service is purely bounded by the terms of reference and other guidelines and formats as per the client’s interest.

### Specific Objectives

a/ The objective of completing the hydrology component study is primarily to produce a comprehensive assessment and study about the agro meteorology, water demand and supply, the hydrology of the overall given watershed area in terms of spatial (U/s and D/s) aspect and temporal (wet and dry/critical) aspects so that the intervention is at least with minimum risk and does make environmental imbalance in the project at hand and in the u/s and d/s aspect.

- To identify, estimate and allocate, according to the water balance study, proportional irrigation water resource for the given diversion  as well as for other competitive uses around the project area

- To appropriately estimate the design flood across the given cross section of the diversion and/or the surrounding area by  using various acceptable hydrological methods and models, so that after construction of the structure any unmanaged/overtopping, over flooding, back flooding and scouring ,etc/ will be minimized and the structure will be safe throughout the intended  life time.

- To estimate reasonably design storm and design flood using required return period to properly size the recommended hydraulic structures for the purpose.

- To properly investigate and recommend the current and the future uses of the water source and make a good water balance study that can enhance the equity of the resource; mitigate any anticipated risks that can probably happen due to the project.

b/ To produce a comprehensive study about the project area and find appropriate intervention for the purpose called diversion weir. Properly selected headwork and its appurtenant structures associated with its designs. Proper lay out of the design on to the topographic map with the given scale is also the main objective.

c/ to produce a comprehensive study about the irrigation system layout, supply and drainage canal design and other infrastructure designs such as access road, cross drainage, cross road and other system facilities in Azuwary-2 irrigation  project.

- To identify, and locate, the diversion weir which can command the proposed command area (100ha).

- Structural dimensioning and setting out of the specifications

- To appropriately estimate the engineering design bills and cost estimates for headwork structures.

- To properly design the engineering part based on the Terms of Reference.

- To identify the irrigable land topography and arrange the irrigation layout system and other mechanisms.

- To select practicable irrigation method for the project and use this selection for the overall design purpose,

- To assess and design the canal lining requirements and select appropriate canal section that fits the site conditions.

- To design all accessory irrigation infrastructures and justify all necessary reasons based on the site conditions.

- To make ground truthing and complete detail design thereby producing the working drawing.

## Scope of the study

The study uses meteorological stations from Gundewoyn National Meteorological station due to various hydrological reasons, therefore meteorologically the study covers wider areas but hydro-logically the study is limited to data that are entirely in the Azuary2 and adjacent watersheds which have basically an influence to the watershed of Azuary2. The study has intensively used topographic maps generated using AutoCAD civil 3D software at a larger scale of mapping (0.5m contour intervals. The drawing outputs or deliverables will be as stated in the terms of reference. Headwork site plan and irrigation system sit plan shall be prepared at scales of 1:500 to 1: 1000 respectively. All other topographic data are obtained through visual observation and field investigation.

## Methodology

The study has primarily focused to preparation & collection of the secondary data required to undertake the given assignment at required standards. Interdisciplinary team of experts has assessed and collected necessary data both at office and field survey/investigation.

After the team has moved to the field, detail field observation and investigation has been conducted to carefully select and decide which site will be hydraulically best and then various topographic; geotechnical, social and environment data had been collected.

The design of Azuary-2 irrigation project follows the principles of participatory design criteria for which the site is selected because people are exercising irrigation over the proposed site. All other design criteria are that of clients TOR guidelines.

The following methods /procedures, but not the only one, have been used in the accomplishment of this service:

- Site identification:

- 50,000 scale top map and GIS information

- Local farmers interview and discussion

- Previous assessment studies

- Transect walk along the river longitudinal axis.

- Topographic survey:

- Surveying the head work ( as part of the whole topo map ) area with sufficient radius to u/s and d/s reaches, using Total station

- Flow estimation

- Physical observation on flood mark indications and local information about high flood and base flow condition of the river

- Analyzing the recorded rainfall data and use water shed inputs for further analysis.

# SECTION-I: HYDROLOGY

# Watershed characteristics

In any small scale modern irrigation system, most of the headwork component structures are to be designed considering the magnitude of flood produced by a fifty years return period design rainfall. Once the rainfall is determined the next step is to investigate about the characteristics of the watershed.

Determination of catchment area, main stream length and the vertical elevation difference are the major and the primary activity for watershed runoff simulation using various accepted models. Azuwary-2irrigation project has a total catchment area of 80.61km2 having the main stream length of 11960m, the maximum watershed elevation being 3410m and the minimum elevation is 2500m. The watershed has an average main stream bed slope of 0.023.

100% of the catchment area falls in Goncha Siso Enesie   woreda and almost more than 56.11% of the watershed is cultivated land the major and dominant land use coverage The major land use types in the watershed include Afro alpian , Cultivated, , grassland ,plantation and sharp land . All the watershed parameters have been determined by using a topographic map at a scale 1:50000 and Digital Elevation Model of 30x30m resolution.  By using SWAT GIS and field ground truthing or data collection, the average Curve Number in Antecedent Moisture Condition II is found to be 80.3 Based on the observation, curve number for the watershed is determined. All the details of the watershed characterization and development plan is shown in the watershed report.

Figure 2-1:  Drainage map of Azuwary-2 watershed

# Hydro-metrological data availability

## Hydro-meteorological data availability

### Hydrometric data

Hydrologists and designers are faced with lack of good or non-recorded hydrometric data on the target stream/river and on local weather and climate conditions. Stream gauging stations are non-existent in remote rural areas of the region and even meteorological stations are almost rare near small rivers in the region. Likewise, at the watershed of Azuary River, there is neither gauged river nor meteorological station.  Therefore investigating other stations near the project area is the primary step in hydrological analysis of the given Azuwary-2 watershed.

### Climate

Data for the hydro-meteorological analysis is therefore taken from the nearby station call Gundewoyn Meteorological station which is located at a distance of 13 km from the project site. In this station there is Rainfall data element recorded in Gundewoyn meteorological station which is about 13 km air distance from the project site. Therefore, rainfall and daily heaviest rainfall elements have been taken in this station. Other elements used for irrigation crop water requirement computation shall be calculated by using CLIM WAT. The completeness of the selected Gundewoyn station data is tabulated as shown in the following table.211

Table 3-1:   Data availability and adequacy Gundewoyn station

| climate data Elements | No. years | Range of Data Years | Missing Data | Adequacy | Remark |
| --- | --- | --- | --- | --- | --- |
| Minimum temperature | - | - | - | - |  |
| Maximum temperature | - | - | - | - |  |
| Daily heaviest Rainfall | 54 | 1954-2010 | 34 | Adequate |  |
| monthly rainfall | 54 | 1954-2010 | 24 | Adequate |  |
| wind speed | - | - | - | - |  |
| sunshine hours | - | - | - | - |  |
| relative humidity | - | - | - | - |  |

The mean annual rainfall amount of Gundewoyn meteorological station is 1600 mm (1983 - 2010 data) and most of it occurred from June to August.

### 3.1.3 Daily Heaviest Rainfall Data

In order to compute the design flood for design of the diversion structure, the daily maximum rainfall is collected from Gundewoyn Metrological stations with a record period of 31years. This station is selected because it is the nearest one as compared to other NMSA stations (Mertolemariam  and Motta ).Those years with higher number of winter season missed data are rejected and are not included here.

Figure 3-1 : Trend of daily heaviest rainfall in Gundewoyn NMS

# Daily Heaviest Rainfall data availability and checking

Rainfall and other related meteorological data availability is core for any projects that require          hydrological analysis. However, sufficient availability of such data in the target position is a rarely     phenomenon in developing countries.

At the proposed outlet point or diversion point, Azuwary-2 River has a catchment area of 80.61 km2. The nearest meteorological station for the watershed is that of Gundewoyn.

Table 4-1:   storm Analysis for Gundewoyn Rainfall Station

| S.No. | Year | Max. RF | Descending Order | Rank | Logarithmic Value/Yo/ | (Yo-Ym)2 | (Yo-Ym)3 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1983 | 51.8 | 91.6 | 1 | 1.9619 | 0.0320232 | 0.0057306 |
| 2 | 1985 | 83.5 | 84.4 | 3 | 1.9263 | 0.0205627 | 0.0029486 |
| 3 | 1986 | 57.2 | 83.5 | 4 | 1.9217 | 0.0192491 | 0.0026706 |
| 4 | 1987 | 53.7 | 79.3 | 5 | 1.8993 | 0.0135322 | 0.0015742 |
| 5 | 1988 | 51 | 73.6 | 6 | 1.8669 | 0.0070447 | 0.0005913 |
| 6 | 1989 | 57.5 | 72.8 | 7 | 1.8621 | 0.0062704 | 0.0004965 |
| 7 | 1990 | 84.4 | 72.1 | 8 | 1.8579 | 0.0056235 | 0.0004217 |
| 8 | 1991 | 79.3 | 70 | 9 | 1.8451 | 0.0038630 | 0.0002401 |
| 9 | 1992 | 51.7 | 69.1 | 10 | 1.8395 | 0.0031960 | 0.0001807 |
| 10 | 1993 | 68.2 | 68.2 | 11 | 1.8338 | 0.0025846 | 0.0001314 |
| 11 | 1994 | 66.5 | 66.5 | 12 | 1.8228 | 0.0015901 | 0.0000634 |
| 12 | 1995 | 91.6 | 63.3 | 13 | 1.8014 | 0.0003407 | 0.0000063 |
| 13 | 1996 | 63.3 | 62.6 | 14 | 1.7966 | 0.0001858 | 0.0000025 |
| 14 | 1997 | 47.3 | 62.5 | 15 | 1.7959 | 0.0001673 | 0.0000022 |
| 15 | 1998 | 62.6 | 62 | 16 | 1.7924 | 0.0000892 | 0.0000008 |
| 16 | 1999 | 57.4 | 58.3 | 17 | 1.7657 | 0.0002985 | -0.0000052 |
| 17 | 2000 | 58.3 | 57.5 | 18 | 1.7597 | 0.0005418 | -0.0000126 |
| 18 | 2001 | 53.6 | 57.4 | 19 | 1.7589 | 0.0005776 | -0.0000139 |
| 19 | 2002 | 38.9 | 57.2 | 20 | 1.7574 | 0.0006528 | -0.0000167 |
| 20 | 2003 | 72.8 | 54.3 | 21 | 1.7348 | 0.0023180 | -0.0001116 |
| 21 | 2004 | 62.5 | 53.7 | 22 | 1.7300 | 0.0028059 | -0.0001486 |
| 22 | 2005 | 46.4 | 53.6 | 23 | 1.7292 | 0.0028923 | -0.0001556 |
| 23 | 2006 | 73.6 | 51.8 | 24 | 1.7143 | 0.0047081 | -0.0003230 |
| 24 | 2007 | 54.3 | 51.7 | 25 | 1.7135 | 0.0048240 | -0.0003350 |
| 25 | 2008 | 48 | 51 | 26 | 1.7076 | 0.0056814 | -0.0004282 |
| 26 | 2009 | 48 | 48 | 27 | 1.6812 | 0.0103437 | -0.0010520 |
| 27 | 210 | 70 | 48 | 28 | 1.6812 | 0.0103437 | -0.0010520 |
| 28 | 2011 | 72.1 | 47.3 | 29 | 1.6749 | 0.0116822 | -0.0012627 |
| 29 | 2012 | 62 | 46.4 | 30 | 1.6665 | 0.0135553 | -0.0015782 |
| 30 | 2013 | 69.1 | 38.9 | 31 | 1.5899 | 0.0372473 | -0.0071886 |
| SUM |  |  | 1856.60 |  | 53.4884 | 0.2247951 | 0.0013770 |
| MEAN |  |  | 61.89 |  | 1.7829 | 0.0060115 | 0.0000459 |
| STANDARD DEVATION |  |  | 12.68 |  | 0.0880 |  |  |
| SKEWNESS COEFICIENT |  |  | 0.520 |  | 0.3649 |  |  |

## Data Consistency Test

At first trial we have checked the 31 years of data for its outliers and one data set is outlier. After removing the lower outlier rainfall record (32mm) at the year of 1982, the data has been again checked for higher and lower outlier and no data is found outlier.  There for the computation of design rainfall is done by using 30 years of data. Rejecting this data and rechecking for its quality shows the following results.

30 years of daily heaviest rainfall records of Gundewoyn meteorological station, from 1983 to 2013, were used for the estimation of point design rainfall. In spite of less number of rainfall data, the designer selected an event that recurs in every 50 years of return period to compute the design rainfall, which is common for small scale irrigation projects. These data should be checked for their consistency or outlier test to assure the reliability of data for further design flood simulation.

### Checking Data Reliability

Number of data = 30

Standard deviation,  12.68

Mean, X= 61.89 mm

Standard error of mean, = =

Relative standard, ==3.73% < 10 %

Hence, the data series is regarded as reliable and adequate since the value of "Relative standard" is relatively small enough. Now, let us check the data outlier test.

### Data Outlier Test

This is done to check whether the adopted data are within a limited range or not. Outliers are data points that depart significantly from the trend of the remaining data. The retention or deletion of these outliers can significantly affect the magnitude of statistical parameters as mean and standard deviation that are computed from the data, especially for small samples. Procedures for treating outliers require judgment involving both mathematical and hydrologic considerations. However, here simple mathematical approaches are practiced to sort out the data that seem reliable of the trend of the parent.

Input data:

Summation of the daily maximum rainfall data records of 30 years,

Arithmetic mean of the data, X,

Summation of common logarithms of the data,

Mean value of common logarithms of the data,

Standard deviation of the common logarithms,

Skewness of the common logarithms of the daily maximum rainfall data, Cs

Tests for Outliers

Outliers are data points that depart from the trend of the remaining data especially at the two extreme ends. The deletion or retention of these outliers can significantly affect the magnitude. As shown from the above calculation the Station Skewness is greater than +0.4 ; thus, test for lower

Outliers are not important. All the data population are in the same group however to complete the process and see the result,

Test for higher outlier

Higher outlier

Where:  = mean of data in common log unity

= From table for sample size N

From Table for data N=30, ,,  and

Skewness coefficients  = 0.1005

Higher outlier

The highest recorded value (91.6mm) is less than the higher outlier (102.02 mm). Therefore, there is no   higher outlier data. it is not included in the above table.

Test for lower outlier.

Lower outlier

The lowest recorded value as shown from the table is 38.9 mm which is greater than lower outlier (36.07mm) and therefore no lower outlier found.

# Design Storm Computation

After checking consistency (reliability and outliers) test, the rainfall data are obtained as representative for the analysis. The magnitude of the design rainfall of 50 years of   return period is estimated by the recommended distributions such as Gumble, EVI, LogPearson and LogNormal distributions. The best fitting distribution to be used can be done by using D-index test as follows.

## D-Index test

After checking the consistency of the data for higher and lower outlier, the 30 years data is obtained as representative for the analysis using D-index. The D-Index test is believed to show the better goodness or fitness of the data to frequency models. Hence in this study it was used to determine the best statistical distribution to estimate the peak rainfall. The D-index for the comparison of the fit of various distributions is summarized as follows.

D-Index=(1Xm)*i=16Abs(Xi-Xi')

Where Xi and Xi’ are the ith highest observed and computed values for the distribution respectively.

Table 5-1: Test for goodness to fit using D-index

| Rank | XI | Normal | Log Pearson Type III | Log Normal | Pearson Type III | Gumbel EVI | Gumbel |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | XI -'XI' | XI -'XI' | XI -'XI' | XI -'XI' | XI -'XI' | XI -'XI' |
| 1 | 91.60 | 6.273 | 3.341 | 3.345 | 5.827 | 1.637 | 3.897 |
| 2 | 84.40 | 3.266 | 1.867 | 1.869 | 3.028 | 1.455 | 4.839 |
| 3 | 83.50 | 5.128 | 4.535 | 4.536 | 5.005 | 4.733 | 7.558 |
| 4 | 79.30 | 3.075 | 2.999 | 3.000 | 3.028 | 3.553 | 13.390 |
| 5 | 73.60 | 0.826 | 0.536 | 0.536 | 0.818 | 0.240 | 20.654 |
| 6 | 72.80 | 0.049 | 0.510 | 0.510 | 0.002 | 1.430 | 23.018 |
| Sum |  | 18.617 | 13.789 | 13.796 | 17.708 | 13.049 | 73.356 |
| Sum/Mean |  | 0.301 | 0.223 | 0.223 | 0.286 | 0.211 | 1.185 |
| Point Rainfall |  | 87.93 | 92.75 | 92.00 | 88.43 | 94.75 | 100.45 |
| Design Point Rainfal = |  |  | 100.448 |  |  |  |  |

All the candidate distributions give almost identical correlation coefficients. However, the standard errors are significantly lower for the Gumbel EVI distribution which is 0.211. Therefore Gumbel EVI  distribution is found to  be used for the design flood simulation in the next section. But for this specific design to be risk free can be used Gamble. Therefore, the design point rainfall for 50 years return period is found to be 100.448 mm.

# Design Flood Determination

## Peak flood analysis by SCS method

For ungagged rivers, the design flood can be simulated by using SCS unit hydrograph method. The computation is done using design rainfall or storm estimated earlier, In the hydrologic analysis of flood using SCS method, rainfall amount and storm distribution; catchment area, shape and orientation; ground cover; type of soil; slopes of terrain and stream(S); antecedent moisture condition; Storage potential (over bank, ponds, wetlands, reservoirs, channel, etc.) can be used and all such data shall be carefully determined before proceeding to SCS simulation.

### Time of concentration (Tc)

Time of concentration has been calculated by taking the stream profile of the longest streamline and dividing it in to different elevation ranges. Kirpich formula is adopted for computation.

Table 6-1: Determination of Time of Concentration

| Partial | Elevation | Elevation Difference in m |  |  |
| --- | --- | --- | --- | --- |
| Length | in m |  | Slope of river, Decimal |  |
| 0 | 2480 | 0 | 0.00 | 0.00 |
| 3660 | 2460 | 550 | 15.03 | 0.37 |
| 11960 | 2409 | 660 | 5.52 | 1.37 |
| 9820 |  | 180 | 1.83 | 1.80 |
| 7510 |  | 70 | 0.93 | 1.90 |
| Total Tc,in hr |  |  |  | 5.43 |

Hence

- Tc = 5.43 hr Since it is  >3 hr ., duration of excess rainfall difference, D =1 hr

- Time to peak,

Tp=D2+0.6*1.2Tc= 3.76 hr

- Base time,

Tb=2.67*Tp =10.04 hr

- Recession time,

Tr=1.67*Tp=6.27hr.

### Curve number (CN)

Curve number (CN) is achieved based on SCS method by watershed characterization in terms of land cover, treatment, hydrologic condition and soil group. From the watershed analysis curve number at condition II =80.3 for the good watershed cover and highly hard pan soil nature.  Since peak rainfall is found at an antecedent moisture condition III state, this value has to be changed to antecedent moisture condition III.

- Conversion factor = 1.076

- CN Condition (III) = (Factor from Table x CN condition II) = 90.86

### Areal Rainfall

As the area of the catchment gets larger, coincidence of all hydrological incidences becomes less and less. This can be optimized by changing the calculated point rainfall to aerial rainfall. The conversion factor is taken from standard table or curves that relate directly with the size of watershed area and type of the gauging station (IDD manual).  But in this project a table was used.

### Rainfall Profile

Rainfall profile is the distribution of the proportion of design rainfall during every incremental time on the watershed area during the 24 hours duration. Well-developed models are needed to determine such an event for the selected basin area. But there are no sufficient modeling studies in the vicinity and adaptation of standard curves has been taken as the only option. Designer of this project has adopted the standard curve from Design Guidelines for Small Scale Irrigation Projects in Ethiopia. With the aid of rainfall profile versus duration curve the percentages of design rainfall distribution on the catchment area are computed for the first most intensive storm duration.

Table 6-2: Design Rainfall Arrangement

| Duration (hr) | Design Rainfall | Rainfall Profile |  | Area to Point Ratio % | Areal Rainfall (mm) | Incremental Rainfall (mm) | Descending order | Rank |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | % | mm |  |  |  |  |  |
| 1.00 | 100.5 | 45 | 46.4 | 66.6 | 30.9 | 30.90 | 30.90 | 1 |
| 2.00 |  | 58 | 59.8 | 74.6 | 44.6 | 13.71 | 13.71 | 2 |
| 3.00 |  | 68 | 70.1 | 78.6 | 55.1 | 10.50 | 10.50 | 3 |
| 4.00 |  | 71 | 73.2 | 81.6 | 59.7 | 4.63 | 4.91 | 4 |
| 5.00 |  | 75 | 77.3 | 83.6 | 64.6 | 4.91 | 4.63 | 5 |
| 6.00 |  | 76 | 78.4 | 84 | 65.8 | 1.18 | 1.18 | 6 |

### Direct Run off Analysis

Input data:

- Curve number at antecedent moisture condition III =90.86

- Catchment Area, A = 80.61Km2

- Direct run-off,

- Q=(p-0.2*S)2(p+0.8*S)

- Where, I = Rearranged cumulative run-off depth (mm

- S = Maximum run off potential difference,

- S=25400CN-254

- Peak run-off for incremental;

- Qp=0.21*(A*Q)Tp

- Where,   A=Catchment area (Km2)

- Tp=Time to peak (hr)

- Q = Incremental run-off (mm)

Table 6-3: Direct Runoff analysis

| Duration | cumulative RF | Incremental RF | accumulative runoff | incremental runoff | runoff in | unit hydrograph time |  |  | Remark |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| hr | mm | mm | mm | mm | m3/s | Beginning | peak | End |  |
| 1.00 | 1.18 | 1.18 | 0.00 | 0.00 | 0.00 | 0.00 | 3.76 | 10.04 | H1 |
| 2.00 | 6.09 | 4.91 | 0.04 | 0.04 | 0.16 | 1.00 | 4.76 | 11.04 | H2 |
| 3.00 | 16.58 | 10.50 | 3.56 | 3.52 | 15.86 | 2.00 | 5.76 | 12.04 | H3 |
| 4.00 | 47.49 | 30.90 | 26.45 | 22.89 | 103.03 | 3.00 | 6.76 | 13.04 | H4 |
| 5.00 | 61.20 | 13.71 | 38.55 | 12.10 | 54.47 | 4.00 | 7.76 | 14.04 | H5 |
| 6.00 | 65.82 | 4.63 | 42.74 | 4.20 | 18.90 | 5.00 | 8.76 | 15.04 | H6 |

Table 6-4: Hydrograph coordinates

| HYDROGRAPH TIME | H1 | H2 | H3 | H4 | H5 | H6 | HT |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 0.00 | 0.00 |  |  |  |  |  | 0.00 |
| 1.00 | 0.00 | 0.00 |  |  |  |  | 0.00 |
| 2.00 | 0.00 | 0.20 | 0.00 |  |  |  | 0.20 |
| 3.00 | 0.00 | 0.39 | 5.67 | 0.00 |  |  | 6.07 |
| 3.20 | 0.00 | 0.43 | 6.81 | 6.21 | 0.00 |  | 13.45 |
| 4.20 | 0.00 | 0.63 | 12.48 | 37.24 | 3.46 |  | 53.81 |
| 5.00 | 0.00 | 0.53 | 17.02 | 62.06 | 17.31 | 0.00 | 96.92 |
| 5.20 | 0.00 | 0.51 | 18.16 | 68.26 | 20.77 | 0.74 | 108.44 |
| 6.00 | 0.00 | 0.41 | 15.44 | 93.09 | 34.62 | 3.70 | 147.26 |
| 6.20 | 0.00 | 0.39 | 14.76 | 99.29 | 38.08 | 4.44 | 156.96 |
| 7.00 | 0.00 | 0.30 | 12.04 | 84.43 | 51.93 | 7.39 | 156.09 |
| 7.20 | 0.00 | 0.27 | 11.36 | 80.71 | 55.39 | 8.13 | 155.87 |
| 8.00 | 0.00 | 0.18 | 8.64 | 65.85 | 47.10 | 11.09 | 132.86 |
| 8.20 | 0.00 | 0.16 | 7.97 | 62.13 | 45.02 | 11.83 | 127.11 |
| 8.54 | 0.00 | 0.12 | 6.80 | 55.74 | 41.46 | 11.07 | 115.18 |
| 9.54 | 0.00 | 0.00 | 3.40 | 37.16 | 31.09 | 8.86 | 80.51 |
| 10.54 | 0.00 |  | 0.00 | 18.58 | 20.73 | 6.64 | 45.95 |
| 11.54 | 0.00 |  |  | 0.00 | 10.36 | 4.43 | 14.79 |
| 12.54 | 0.00 |  |  |  | 0.00 | 2.21 | 2.21 |
| 13.54 | 0.00 |  |  |  |  | 0.00 | 0.00 |

- From the analysis, the 50 years return period design flood is 156.96m 3/s at 6.2hr peak time. This implies that for this watershed the peak flood rate per km2 area of the watershed is about 1.95 m3/s/km2. This value ensures that high rainfall of the area can generate higher rate of runoff per km2 of the generating catchment in spite of its forest coverage.  Graphical plot of all the 6 hr synthetic hydrographs are shown below.

Figure 6-1: Complex Hydrograph

## Flood mark Method

During field assessment and topographic survey, the normal and maximum flood mark points has been selected by interviewing the local aged farmers and marked in cross section of selected river geometry. This is just to check the design flood what we have determined using rough simulation methods such as SCS for engaged catchments. That was the main purpose of taking flood marks during field assessment and that will be checked using the stage discharge analysis in the following section.

### Tail Water Depth Computation

Tail water depth of the river is equal to the flood depth and amount at the proposed weir site before construction of the weir. It is used to crosscheck peak flood estimated by the SCS unit hydrograph method with flood mark method and to see the flood feature after the hydraulic jump. During field visit, the flood mark of the river at the proposed diversion site was marked based on dwellers information and physical indicative marks. The data required to do stage discharge analysis has been discussed in the following section.

Figure 6-2:  The river cross-section for Azuwary-2weir axis (from left to right)

### Average river bed slope

Average river bed slope of River is estimated by using partial area method and direct measurement method. The water level of the river is taken at different points along the river channel around the headwork site. Surveying work has been done for 100m length. And then, average water surface slope is considered as the river bed slope which is on Average, S = 0.011. The detail is arranged in spread sheet.

### Manning’s Roughness coefficient for the river bed and banks (average)

The Manning’s roughness coefficient is taken from standard table based on the river nature. The river at the headwork site has got relatively U-shape feature and straight nature. The river banks have grasses and tree roots but on the river bed there are out cropped weathered rocks whence Manning’s roughness coefficient (n = 0.030) is adopted.

### Discharge of the river

Input data:

- Manning's roughness coefficient, n = 0.03

Average river bed slope, S = 0.011

,

- Where, R = Hydraulic radius = (Area/Perimeter)

- Q=V*A

Table 6-5: River discharge computation at different stages of flow, (downstream)

| Elevation | Depth | Total Area | Total Perm | Top Length | Wet Area | Hydraulic Radius R (m) | Velocity V (m/sec.) | Discharge Q (m^3/sec.) | Remark |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 2453.76 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |  |
| 2454.06 | 0.30 | 1.57 | 12.97 | 6.43 | 6.54 | 0.24 | 1.04 | 1.63 |  |
| 2454.36 | 0.60 | 3.73 | 16.17 | 7.98 | 8.20 | 0.46 | 1.59 | 5.93 |  |
| 2454.66 | 0.90 | 6.36 | 19.37 | 9.52 | 9.86 | 0.64 | 2.01 | 12.75 |  |
| 2454.96 | 1.20 | 9.44 | 22.58 | 11.06 | 11.52 | 0.82 | 2.36 | 22.24 |  |
| 2455.26 | 1.50 | 12.99 | 25.78 | 12.60 | 13.17 | 0.99 | 2.66 | 34.61 |  |
| 2455.56 | 1.80 | 17.04 | 29.05 | 14.18 | 14.87 | 1.15 | 2.94 | 50.16 |  |
| 2455.86 | 2.10 | 21.53 | 32.37 | 15.79 | 16.58 | 1.30 | 3.20 | 68.90 |  |
| 2456.16 | 2.40 | 26.61 | 35.69 | 17.39 | 18.30 | 1.45 | 3.45 | 91.81 |  |
| 2456.46 | 2.70 | 32.08 | 39.25 | 19.12 | 20.13 | 1.59 | 3.67 | 117.66 |  |
| 2456.75 | 2.99 | 38.18 | 40.90 | 20.89 | 20.01 | 1.91 | 4.13 | 156.96 |  |
| 2456.76 | 3.00 | 38.18 | 40.90 | 20.89 | 20.01 | 1.91 | 4.14 | 157.93 |  |

From the above stage discharge table the maximum flood estimated from SCS is 156.96 m3/s (at a depth of 2.99 m from the river bed).

Figure 6-3 Stage discharge curve

## Selected Design flood

Based on the stage analysis result of the flood mark elevation, the amount of flood computed in that elevation has been found to be 156.96m3/s which is too high having a specific flood of 2.06m3/s/km2 and is located far from the bank. The overflow over the length becomes minimum and therefore for the case of designing the protection walls of a weir it is advisable to take the design flood obtained by using the maximum flood mark method value and hence design flood is taken as 156.96 m3/

Based on the stage discharge analysis of the project at d/s of the weir section or design discharge capacity of the river channel before the construction or intervention, the downstream high (HFL) flood level can be fixed by the selected designed flood in the previous section.

Therefore, the D/S HFL=river bed elevation + depth of Tail-water (as obtained from SCS flood at a depth of 2.4m).  D/S HFL=2453.76+2.99 =2456.75 m asl.

# Basin Base flow, Demand and Water balance

## Basin Base flows

Today the question of water right becomes a nationwide burning issue. Crop cultivation based on modern or semi-modern irrigation schemes becomes the solely alternative for developing countries   like Ethiopia to feed their rapidly growing population. Though it is the bare fact, downstream water rights and ecological securities should be preserved to make the project environmental friendly whereby resolving social conflicts too.

Azuwary-2 River is Perennial River and on May 22/2010 E.C during site investigation period, the base flow had been measured by using floating method and the discharge was found to be 110 l/s.

Therefore, this project has been designed to exploit the water potential effectively and efficiently in environmentally friendly manner so as to ensure food   security and to increase the income of the farmers living around the area.

Figure 7-1:   Base flow of the river

The location of another modern irrigation scheme is 2km downstream of the current project but it’s not operations and will not affected by the upper Azuwary-2 water abstractions and hence the two projects are included with this modern diversion project. This situation and the presence of wet lands are good evidences for the availability of potential ground water storage.

The farmers can be benefited from the excess flow of late winter season.  The river has good base flows till the end of February without any conflict caused by water scarcity. Moreover the project area is a highland area and many springs are seen developed along the river course .So, the proposed Azuwary-2 Diversion Irrigation project will not cause any environmental disturbances resulting from shortage of water during the irrigation period

In addition to this, there is significant amount of effective rainfall in the month of May that can supplement the irrigation cultivation. This condition resolves the probable conflict for water and makes the project very ideal to be practiced. In addition to the presence of tributaries at downstream of the headwork site, at least 20% of the above base flow should be permanently released for downstream users for cattle and other demands but for this specific project there are different perennial tributary immediately dawn stream of the head work site therefore. Again the tail water from the proposed command area joins the main Azuwary-2 River this also replenishes the available water resource.

Finally, the irrigation hours need to be increased to cultivate larger area keeping the diverted water volume constant. Longer irrigating period can be adopted exercised as the majority of the command area is   found along residential areas.

## Existing water use and Irrigation Practices

The watershed is currently covered with less populated trees. The river water is used for domestic water supply and irrigation uses downstream.

There is a traditional irrigation practice along the river, and especially on the proposed site, about 30 ha on the right is being irrigated for vegetables and cash crops. There exists traditional (near 350m) irrigation practices observed in the near downstream parts of the river. Generally in the late season up to the month of March there is no shortage of water and afterwards water decrease. The water resource becomes minimum in the driest months of April and May as of the interviewed aged people saying.

## Upstream & Downstream utilization

Azuwary-2 irrigation project will not affect the downstream eco-system since there are small stream around/and just downstream of the newly proposed weir that join the River. These streams produce reliable discharge throughout the year which contributes a flow of more than 136 l/s. These include: the stream named Agancha which has a flow of 8 l/s, Kuntibit which has a flow of 15l/s Telef  which has a flow of 98 L/s and Teyati which has a flow of 15L/s and more of the downstream community is beneficiary of the irrigation scheme. So, there will not be conflict on downstream of the proposed site. Though the community around the proposed area, on the upstream side,  showed their interest.

## Conclusion

Azuwary-2Riverhas a 80.61 km2 watershed and Gundewoyn station is the nearest Meteorological station. The design storm computed by using Gumbells has been selected for design purpose.

The design flood was found to be 156.96m3/s from SCS method of analysis and its specific design flood was 1.95 m3/s/km2. The base flow measured in May, 2010 by using floating method was 110l/s

# SECTION-II: HEADWORK DESIGN

# Headwork site selection

The headwork site was initially located at the uppermost traditional practice site where above no traditional irrigation is practiced.  In selecting or comparing the best site, it was practically important to put criteria’s such as considering the flood the channel has to accommodate, the length of the weir crest, topographic suitability, the relative economic considerations , the availability of enough  command  and other selection or comparison criteria’s.

During the period of field assessment, it was tried to critically observe all possibilities by moving 3-4kms u/s and d/s from the initial options. The 1stand the uppermost option, was traditionally diverted site where it is low height weir located at a high slope and a meandering section of the river. The canal has 500m idle length to the left. This is the best option selected since it and the commands area only the left side.

Therefore, the headwork site of the best and selected option is situated at 1204926N, 396285 m E and river bed elevation of 2453.76m above sea level. At this site the river course is well defined, stable and forms nearly a U-shaped cross section. Both The left and right sides bank of headwork site earthen alluvial types.

The headwork site geological surface and subsurface conditions have been investigated based on the nature of the proposed structure. At the site and immediate vicinity, the stream flows along gentle slope course and its bed is mainly covered with recently deposited sand with gravels and boulders.

# River Geomorphology

Both surface and sub-surface investigations have been conducted at the proposed weir site and along the main canal route. A source area for required natural construction materials has also been studied by the geologist. The results of the investigations are presented in concise form in the geology report with appropriate recommendations based on findings and requirements of the proposed engineering structures in the respective parts of the project report.

## Left Bank conditions

This bank is made up of weathered and very closely jointed volcanic rock. The slop of the bank to bed is very steep. Next to the bank or behind the bank, just 3m to 5m distance from right bank, there is a ridge which is made up of highly to moderately weathered volcanic rock. The workability of this rock is not difficult. It can excavate with either jack hammer or packet excavator. During construction on the left side of the bank, the weir body as well as the cut off shall be extend up to this weathered rock. The ridge that is formed from weathered rock is long and will be more than 10ms vertical height.

Figure 9-1: Photo showing left bank

## Right bank condition

It is made up of in situ residual clay soil which has reddish brown color. The soil is high plastic clay. The bank is not stable rather it is susceptible to erosion resulting in falling of the bank soil time to time and hence it needs the protection structure/retaining wall. The length of protection structure will be 50ms on the downstream side and 20ms meters on the upstream side. Unless and otherwise the bank cannot resist the energy coming from the flood. As a result of this, the weir structure will totally fail down. The vertical height of this bank from river bed to OGL of the bank is about 2.8ms.

STREAM FLOW DIRECTION

Figure 9-2: Right Bank and Stream Bed Geologic Formations

To summarize the geological condition of the headwork, a geological X- section is prepared by combining the obtained and interpreted surface and subsurface data.

Figure 9-3: Geological x section along Azuwary-2 weir axis

## Stream Bed

The river bed is made up of recent transported deposit. The deposits have a grain size ranging from sand to coble. The thickness of this deposit could not know because during investigation time (09/10/2010 E.C), the river bed was covered with high discharge water. But conservatively, the depth to bed rock has estimated and put here. That is it will be found at a depth of 3meetrs from the bed. The width of bed from foot of left bank to foot of right bank is about 10mters. The slope of the bed is gentler.

## Natural Construction Materials

As stated in the geology report , During the site investigation, natural construction materials required for the construction of the various proposed engineering structures at the headwork and within the farmland have been assessed, and possible quarry sites and borrow areas have been identified within the vicinity of the area as much as possible.

### Rock for Masonry and Coarse Aggregates

The rock for masonry and coarse aggregates for this particular projects is identified at the vicinity of this project area. The quantity and quality of this material has also been estimated and evaluated. The quantity of the rock is estimated to 4,000cubic meter. Moreover, the rock is made up of fine grained fresh aphanitic basalt. The workability or method of excavation will done either man power or packet excavator because the material is columnar jointed and can easily produce. So that since the rock is made up of aphanitic basalt, its quality is excellent.

### Sand

The sand for Azuari 2 is identified and inspected at Feres  Mada River. It is found within Enebsie Sar mider woreda in kebele 07 at the locality known as Gunaguna. It is accessible 40km through asphalt road and 6.6km through all weathered gravel road. In general the sand quarry is located 46.6km distance from project site. The sand is volcanic rock derived and well graded which is suitable for construction. The quantity of the sand within this stream is more than the need of project. As information gathered from local miners, more than 50,000cubic meter/3125 damp trucks of sand had been mined and transported per year from this stream quarry.

### Water

Water for construction is available in the river Azuari River itself. The discharge of the river flow is more than the need of the project / which is 110L/Sec during investigation time or in the month of June

# Headwork type selection

After detail observation of the available water balance, the target command area and the cost of future idle canal length, it is decided to design a diversion weir with commanding elevation. Weir type selected for the given height and flood amount is broad crest weir type which could be subjected to the expected hydrostatic and hydrodynamic forces that can be generated from peak flood. The weir is to be constructed with masonry material with a 20cm thickness RCC capping.

## Hydraulic design of headwork structure

### Weir Height Determination

The following major factors have been reasonably assessed during the field study and the final topographic map has been investigated whether the target area can be commanded at less weir height or not. By using necessary parameters, the weir crest elevation has been determined on both sides of the river as follows:

- Maximum  command area elevation =2454 (bed)

- Main canal idle Length (m) =840m

- Main canal slope=0.02

- Canal depth =0.50

- Head regulator Losses =0.1

- Lowest Point of river center=2453.76

Therefore, Weir crest level =2453.76+0.0013*840+0.1+0.40+0.2=2455.76m

Weir height (h) = 2455.76 -2423.76=2.0 m (roundup value), therefore take a weir height of 2.0 m.

### Design Base flow of the River

Based on the water balance analysis, the minimum flow of May measurements have been used. Therefore, considering 20l/s another stream immediately dawn stream flow for downstream users and for environmental purpose as well, a design flow of 110 l/s has been used for the purpose.

## Weir Dimensioning

### Weir crest length determination

In fixing the weir crest length Lacey’s flow regime width may be used when the actual width of the river is higher. The Lacey’s crest length value is most of the time higher and hence weir crest length selection depends according to the actual river section width. However, it may be advisable to check the target river section width with Lacey’s regime width.

Therefore, based on Lacey’s regime width, the design width of river section at the weir axis is:                                          L=4.75*Q,=4.75*110 = 49.81 m. But Actual river section width of the over flow section of the river is equal to 11m.  Therefore weir crest length used for designing other weir hydraulic parameters is taken as the average river width of 12m for safe passage of the design flood after construction.

### Top and bottom width determination

Bligh’s formula can be used to approximately estimate the top and bottom width of a broad crest weir. However, the dimension values obtained by using this formula should be checked structurally in the stability section.

Top width T in m,

Bottom width B in m,

Where, h is Height of weir (m), He is specific energy head (over flow depth + approaching velocity head (m)),	ρ is specific weight of weir body (2.35 for cyclopean concrete, 2.3 for masonry and 2.4 for reinforced concrete).  In selecting the specific weight of weir body, it is advisable to take lower values, since during construction the quality may be deteriorated and hydraulic parameters may be under estimated. For our case, masonry weir body capped with RCC is selected and the value of G =2.3

Therefore taking L=17m, C=1.7, H=1.5m and Q=156.96m3/s

Hence,

Top width

Bottom width,

This is an intial weir dimensioning which may undersize or oversize the weir and therefore this computed value of the dimensions shall be fixed after stability analysis of the structure.

### Determination of Upstream & Downstream High flood level (HFL)

Determination of high flood level for both u/s and d/s case of the weir is essential in determining the wall heights.

D/s high flood level is required because, the flood after passing the weir, will join the natural channel section and therefore From the stage –discharge curve prepared in hydrology Section, the high flood level before construction (i.e. D/s HFL) corresponding to the design flood is found to be 2457.51m a.s.l.

Therefore, crest level of the downstream High flood level is,   D/s HFL = 2456.75m amsl

However u/s High flood level is determined by assuming the designed height of weir would be implemented and therefore U/s HFL = U/s bed level + weir height + Design (Hd)  implying that to find the u/s HFL, some procedural calculations are required as stated in the following section.

Hd is the depth of water over the weir crest. This is calculated by the selected weir type (broad crested weir) formula.

The velocity head, ha is computed from the approach velocity as shown below

Where g: acceleration due to gravity = 9.81m/sec2,

Va is Approach velocity determined by

L is Weir crest length + under sluice length = 18m, hd is flow depth over the weir and also, .

Based on the field observation, the u/s side of the weir face partially filled by silt (i.e. water height u/s of the weir after construction is taken to be h=0.2 m, because all the weir height is to be filled by alluvial deposition

By using Goal Seek of MS Excel, hd is found to be 2.84m

- Therefore, ha = He-hd = 3.21m-2.84m = 0.37m

Therefore:

This facilitates the solution for u/s TEL=weir crest level + He=2455.76 +1.48=2459.24m

Therefore, U/s HFL =U/s TEL –velocity head =2458.97-0.37m = 2458.50m a.s.l which is the existing maximum river bank elevation.

### Afflux determination

Afflux = U/s HFL- D/s HFL = 2458.5m a.s.l – 2456.75m a.s.l =1.75 m.

As it is seen from the computed afflux calculation, the value of 1.75 m is greater   than the allowable value which is 1m for lower flood designs and for downstream apron is provided and the retaining well height considering the [post jump elevation.

## Hydraulic Jump computation

As discussed in the geology report, the river bed is alluvial and hence some length of apron for energy dissipation is required. Both left and right side banks are not relatively sound (earthen banks).

Retaining walls at upstream right and left sides are mainly needed to confine the peak flood within the bank. Hence bank protection work is needed upstream of the weir site to direct the flood in to the course.

However protection works are required for the downstream case and a need of the jump profile is necessary. The length of protection walls is determined based on the length of Jump as well as the nature of the foundation soil. Jump is calculated as shown below.

• Weir crest length = 17.0 m

• Weir height = H = 2.0m

• Pre-jump depth = y1

• Post -jump depth =y2

Neglecting losses between point U/s and D/s and considering similar datum

H+Z1 + He = y1 + had +z2

But, He = 3.09m

q=Ql=156.96m3/s17m=9.23m2/s

For small distance of the weir body width, there may be insignificant ground elevation difference, z1=z2 and hence

- had=q22*g*y2=q22*g*y12

- He+h=y1+had=Y1+q22*g*y12

- Y1+q22*g*y12-  (H+He)=0 where H is weir height

After iterations Y1 =1.04m

Velocity at Y1 can be calculated by:

V1=q/y1=9.23/1.04 =8.87m/s

Fr1=V1gy1=8.879.81*1.04=2.76

This can be termed as Oscillatory jump but since the downstream bed is rock. This jump type has less hazard.

Sequent depth can be calculated using the following equation:

y2=y121+8*Fr12-1

y2=1.2821+8*2.762-1 =3.72m

V2=q/y2=9.23/3.72=2.62m/s

Fr2=V2gy2=2.629.81*3.72=0.18

Which shows no jump and hence no further calculation for Y3.

- The length of a hydraulic jump

The length of a jump is required to determine the apron length and the height of the side walls of a stilling basin. By using the criterion that the turbulence has diminished at the end of the jump as quoted by Hanif, Haga (1991a) developed the following equation for the length of jump computation (Hanif & Chaunhry, 1994) .

L=220*tanhFr1-122

Or simply

L=5.6*y2   , for fr1=2.44<4

From L/y2 ratio graph

Figure 10-1: Fraud number vs jump length chart

Therefore, Hydraulic jump length (L) for Fr1=2.76 from the graph L=5.6*y2=5.6*3.72=20.83 say 21.0m.

- Energy loss

The difference between the total head upstream and downstream of the jump is the energy loss in the jump and it is given by:

HL=E1-E2  +v122g-v222g

In which the subscript 1 and 2 indicate the quantities upstream and downstream of the jump

The above equation can be simplified by substituting the value of V1 and V2 as follows  (Smith, 1995)q=v1y1=v2y2

HL=y1-y234y1y2   =   1.09-3.7234*1.09*3.72

=    0.43m

Where HL is the total head loss due to hydraulic jump for rectangular channel with zero bottom slope structure. No further jump is expected since it is nearer to the afflux height.

- Location of jump

Tail water level plays a significant role in the formation of jump at a particular location. In most practical situation the tail water level depends upon the channel discharge Q.

Tail water rating curves for the regime of river below a diversion weir are fixed by the natural conditions along the stream and ordinarily cannot be altered by the diversion weir design or by the release characteristics. The basin floor/apron level must therefore be selected to provide jump depths that most nearly agree with the tail water depths.

For the given basin/apron design, the tail water depth for each discharge seldom corresponds to the conjugate depth needed to form a perfect jump. Thus, the relative shapes and relationships of the tail water curve to the depth curve will determine the required minimum depth to the basin floor. For lesser discharges the tail water depth will be greater than the required conjugate depth, thus providing an excess of tail water, forcing the hydraulic jump up the slope of the diversion.

Hence the jump and tail water profiles are computed and put in tabular and graph as follow

Table 10-1:  jump and tail water profiles

| Discharge | Discharge /length | Velocity | Y1 | Fraud No | y2 | Tail water depth |
| --- | --- | --- | --- | --- | --- | --- |
| m3/s |  | m/s |  | Fr1 |  |  |
| 0.00 | 0.00 | 0.00 | 0.00 | 0.000 | 0.000 | 0.00 |
| 1.63 | 0.10 | 1.04 | 0.01 | 16.971 | 0.348 | 0.30 |
| 5.93 | 0.35 | 1.59 | 0.05 | 9.395 | 0.666 | 0.60 |
| 12.75 | 0.75 | 2.01 | 0.11 | 6.773 | 0.979 | 0.90 |
| 22.24 | 1.31 | 2.36 | 0.18 | 5.418 | 1.300 | 1.20 |
| 34.61 | 2.04 | 2.66 | 0.27 | 4.583 | 1.632 | 1.50 |
| 50.16 | 2.95 | 2.94 | 0.38 | 4.013 | 1.978 | 1.80 |
| 68.90 | 4.05 | 3.20 | 0.51 | 3.603 | 2.334 | 2.10 |
| 91.81 | 5.40 | 3.45 | 0.65 | 3.284 | 2.715 | 2.40 |
| 117.66 | 6.92 | 3.67 | 0.81 | 3.043 | 3.096 | 2.70 |
| 156.96 | 9.23 | 4.13 | 1.04 | 2.777 | 3.599 | 2.99 |
| 157.93 | 9.29 | 4.14 | 1.04 | 2.794 | 3.624 | 3.00 |

Figure 10-2  Jump versus Tail water Depth

We can see from the graph that, the sequent depth, y2 is almost equal to the tail water depth at the beginning or for lower discharges and greater for large discharges, and hence to be more safe, the maximum wall depth without including the free board shall be the sequent depth. Adding freeboard is also important.

## Impervious floor design

a/ Downstream impervious floor design  Requirement

The river bed is covered with alluvial deposit and hence impervious floor design analysis is  important. Considering hydraulic jump length 21m apron is recommended to protect scouring of the weir

## Stability Analysis of weir

In design of diversion weir relevant external forces acting on it must be calculated. Forces which include the soil and uplift pressures can be evaluated whenever the surface water profile at the weir location is drown. The critical discharge for the uplift pressure may be when there is no surface flow and no tail water ( dynamic case),  while for the design  of wing walls the high and low floods levels may be considered .

The designed weir should be enough to resist the piping effect, overturning moment, sliding effect and there should be no tension developed at the base of the dam.  The stability analysis is carried out considering the effect of the following forces.

• Water pressure

• Weight of the over flow weir section

• Sediment load

The extreme load combination is the case where the head is at crest level of the weir and there is no flow over the weir (static case) in which case the computation is made using this condition.

### Resisting Load:

Self-weight load (W): - It is determined with respect to an appropriate unit weight (γm) for the material. For simple elastic analysis the resultant is considered to operate through the centroid of the section.

Where A is the cross sectional area.

### Overturning loads:

Water Load:-If the weir has a water height ‘H’, then the hydrostatic pressure distribution on the horizontal resultant force (Pwh) is given by:

Where Pwh  in  [kN/m]; acting at P/3 from the base of the weir.

Uplift pressure:  The uplift pressure if no pressure relief are provided is given by:

Where Z1 and Z2 is the water heads at u/s and d/s respectively. However since the foundation is hard rock; such forces are not used in stability analysis

Sediment Load: The gradual deposition of sediments creates overturning force at the upper face of the weir. This force is a function of the submerged unit weight of the sediment ‘γs’ and depth of accumulation ‘Z’ (in this case equal to H). Mostly as the deposition is reached to crest level, it is normal to take depth of weir height as depth of accumulation. The force is given by:

Acting at z3from the base of the weir.

Where Φ is the angle of shearing resistance of the sediment

Stability of Azuwary-2 headwork has been done on both non monolith and monolithic conditions. The factor of safety for overturning and sliding will be too high when we check for monolithically weir body condition. There existed tension also. This happens due to long arm length away from the weir body.

The values are tabulated as in the following case.

Table 10-2: weir stability analysis

| 1 | Dimension |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Height,H | Triangle ,B1 | Rectangle,B2 | Bed width, B |  |  |
|  | 2.00 | 1.70 | 1.20 | 2.90 |  |  |
| 2 | Stability analysis |  |  |  |  |  |
|  | Code | Load |  | Lever Arm, R | Moment (about toe) |  |
|  |  | Vertical | Horizontal |  | Positive | Negative |
|  | W1 | 56.400 |  | 0.600 | 33.840 |  |
|  | W2 | 39.950 |  | 1.767 | 70.578 |  |
|  | Ps |  | -6.600 | 0.500 |  | -3.300 |
|  | ph |  | -19.600 | 0.667 |  | -13.067 |
|  | SUM | 96.350 | -26.200 | 3.533 | 104.418 | -16.367 |
| 3 | Factor of safety against, |  |  |  |  |  |
|  | overturning test |  | Fo | 1.99 | >1.5 | OK |
|  | Sliding test |  | Fs | 1.750 | >1.50 | OK |
|  | Tension test |  | X | 1.254 |  |  |
|  | B/6= | 0.483 | e | 0.196 | <B/6 | OK |

Factor of safety against overturning, F0, in terms of moments about the toe of the weir:

Where;  and is summation of stabilizing moment and overturning moment respectively. F0 should be greater than or equal to 1.5

Stability against Sliding

The weir should be stable against sliding at the base for different conditions and it is the function of the shear strength of the construction materials. It is given by:

Where;  and is summation of vertical and horizontal forces respectively and μ is coefficient of friction b/n the material and the horizontal section and its value varies b/n 0.65 to 0.75 up on the materials used ( here 0.75 is taken ) . Fs should be greater than or equal to 1.5.

Safety against Tension

For no tension on the base of the head work structure, for critical section, the resultant (R) should act as the middle third part of the critical section. This implies that the eccentricity (e) should be less than or equal to one-sixth (1/6) of the base width (b) of the weir at the critical section.

And the eccentricity,

The resultant lays within the middle third     implying that there is no tension   developed at the weir body at the toe.

Conclusion: From stability analysis, the designed weir section dimensions will be fixed to 1.2m top width and 2.0m bottom width.

## Design of Divide wall, Under Sluice, and Canal outlet

Divide wall is designed in order to arrange an operation slab so that users can access easily during operation. This structure is not always necessary for all weir structures, the divide wall height may be shortened to as high as the weir height and sluicing mechanism can be done with a bed elevation lower than the canal off take elevation, especially for small flows and commands. Rejection of such walls will also reduce the costs of breast wall and operation slab and also a bigger spindle. However, such type of wall is necessary for practical operation. Such gates can be taken during sudden discharge and hence operation mechanism for gates is essential and divide wall is required. Height above the weir crest is calculated by U/s HFL minus the weir crest elevation.

Hw= 2458.6-2455.76=2.74m

The wall is checked by stability analysis and a top thickness of 1.5m is required.  Its total length including the curves is 3.10m.

### Under sluice

The under sluice is mainly provided here to remove silt deposition as a result of barrier structure/weir. Hence the sill level of the under sluice is fixed to facilitate the disposal of this deposited silt away downstream to increase the off take efficiency. Therefore considering the amount of incoming silts from the watershed, the sluice opening is fixed to be 0.80m above the minimum bed level. Hence the sill level of the under sluice elevation is

2453.76+0.50=2454.26m.a s.l.

Hence the under sluice facilitates the supply of water to the off take and the removal of silt near the off take bed level.

The capacity of under sluice is determined by considering a discharge equal to 5 times of the base flow i.e 5*0.1=0.55m3/s. The dimensions of under sluice are determined by using orifice for maximum flood condition.

Under sluice Outlet size therefore is determined by using the orifice formula as follows

Q = Cd*A*2*g*h

Where; Cd = Coefficient of discharge = 0.80

L = Length of water way (m)

h = depth of afflux=0.74

#### For a bed width of an overflow section=0.5m, the depth of flow over the opening will be :

#### H=A/b=0.25/0.5=0.5m:

#### However to allow some boulders/logs can pass through the opening, the size is taken to be 1.0mx1.0m (height x width).

### Gate for the under sluice

Considering this, the opening size of the gate is 1.05m*1.05m with spindle operated gate.

The gate for under sluice is to be vertical sheet metal of the indicated size for the closure of the opening space providing some extra dimensions for the groove insertion (5mm on each grove total =2*2.5mm=5mm). The grooves are to be provided on the walls using angle iron frames at the two sides of the gate opening. Therefore, The gate of the off take canal is to be vertical sheet metal of 1.05m x 1.05m.

### Design of operation slab and Breast wall

To avoid spilling of water during HFL over the under sluice and canal regulator gate, a R.C.C wall is provided from the gate top level up to the river HFL (i.e known as breast wall). More over to operate the gates operation slab is provided.

The operation slab is made to have dimensions of 1.5m x 1.5m and to make the operator free from risk during operation, the slab is enclosed with hand rails. Refer the drawing  for detail.

The thickness of the slab and breast wall are simply determined from the recommendations.

The recommendation is

L  = 20 to 35,  where     L = length of greater span = 1.50m

t

t= thickness

take   = L  = 20   t = L  = 1.5m  = 0.10m

t		                   20     20

However, provide t = 0.30m = 30cm thickness for the slab and breast wall work. And provide the reinforcement bar of  12mm at 150mm c/c spacing in all directions with reinforcement cover of 15mm.

## Design of Retaining Walls

At the two ends of the diversion weir, walls have been provided to safeguard the structure from scour of banks at the ends .The walls are basically provided to keep the highest flood flow within the weir crest section and to safeguard areas out of the river bank from scouring and cavity.

## Upstream Retaining/wing walls Design

Using stability analysis, wall dimensions are found for protection walls. a free board of 0.3m is selected for both u/s and d/s walls. The bed elevation the upstream bank is found to be 2453.76. Therefore the height of the upstream retaining wall is given by subtracting the bed elevation from the upstream HFL and then adding a nominal freeboard of 0.3m.

H wall= 2458.6-2453.76+0.3=5.14m Adopt =5.15

By checking all the factor of safeties using spread sheet analysis, the dimensions of the u/s protection wall are with top width of 0.70m and bottom width 3.0m. Stability is checked in submerged lateral earth pressure condition which is the critical case. The stability checks are tabulated below.

The wall foundation must be reinforced with a lean concrete (c-10) of thickness 0.15m at the bed for good bondage between the foundation and the wall material.

Table 10-3: upstream Retaining wall stability analysis

| H | FB | TH | B1 | B2 | B3 | BW |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0.3 | 5.15 | 0.7 | 2.3 | 0 | 3 |  |
| Code | Load |  | Lever Arm, R | Moment |  |  |  |
|  | Vertical | Horizontal |  | Posetive | Negative |  |  |
| W1 | 81.1 |  | 0.4 | 28.4 |  |  |  |
| W2 | 133.3 |  | 1.5 | 195.4 |  |  |  |
| W3 | 112.5 |  | 2.2 | 251.3 |  |  |  |
| Ps |  | 83.1 | 1.7 |  | 142.7 |  |  |
| Pw |  |  |  |  |  |  |  |
| SUM | 326.9 | 83.1 |  | 475.1 | 142.7 |  |  |
| Factor of safety against, |  |  |  |  |  |  |  |
| Over turning = (M+ve/M-ve) >1.5 |  |  |  | Fo | 3.33 | >1.5 | OK |
| Sliding = (µxFv/Fh),---µ=0.75 >1.5 |  |  |  | Fs | 2.95 | >1.50 | OK |
| Tension: X= (Net Moment/Sum Fv), e=x-B/2 |  |  |  | X | 1.02 |  |  |
|  |  | B/6= | 0.50 | e | 0.48 | <B/6 | OK |

## Downstream Retaining walls Design

The bed elevation of the bank at downstream was found to be 2453.76and therefore the height of the retaining wall is given by subtracting the bed elevation from the downstream HFL and then adding a nominal freeboard of 0.20m.

H wall =2457.51-2453.76+0.3=4.05m Adopt 4.10m

By checking all the factor of safeties using spread sheet analysis, the dimensions of the d/s protection wall are with top width of 0.70m and bottom width 2.50m. Stability is checked in submerged lateral earth pressure condition which is the critical case. The stability checks are tabulated below.

The wall foundation must be reinforced with a lean concrete (c-10) of thickness 0.15m at the bed for good bondage between the foundation and the wall material.

Table 10-4: Downstream Retaining wall stability analysis

| H | FB | TH | B1 | B2 | B3 | BW |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 4.1 | 0.6 | 2 | 0 | 2.6 |  |
| Code | Load |  | Lever Arm, R | Moment |  |  |  |
|  | Vertical | Horizontal |  | Posetive | Negative |  |  |
| W1 | 55.4 |  | 0.3 | 16.6 |  |  |  |
| W2 | 92.3 |  | 1.3 | 116.9 |  |  |  |
| W3 | 77.9 |  | 1.9 | 150.6 |  |  |  |
| Ps |  | 52.7 | 1.4 |  | 72.0 |  |  |
| Pw |  |  |  |  |  |  |  |
| SUM | 225.5 | 52.7 |  | 284.1 | 72.0 |  |  |
| Factor of safety against, |  |  |  |  |  |  |  |
| Over turning = (M+ve/M-ve) >1.5 |  |  |  | Fo | 3.94 | >1.5 | OK |
| Sliding = (µxFv/Fh),---µ=0.75 >1.5 |  |  |  | Fs | 3.21 | >1.50 | OK |
| Tension: X= (Net Moment/Sum Fv), e=x-B/2 |  |  |  | X | 0.94 |  |  |
|  |  | B/6= | 0.43 | e | 0.36 | <B/6 | OK |

## WEIR, CUTOFF, UNDERSLUICE BED & OFFTAKE RCC CAPPING

In order to avoid cracking and shearing of the weir, the cutoff, under sluice bed and off take slab during overflowing and incoming of boulders, percentage reinforcement cross sectional area width is taken as 0.15% of the concrete. Area of the weir section is a=2.1m2

Area of bar= Asteel=0.0015*2.1=0.00315m2,

Area of steel per meter length of the weir bottom width=0.00315/2.0=1575mm2

Actual area of steel=as=3.14*12^2/4=113mm2

Spacing= (1000*113)/1575mm2=72mm, too small. Take 100mm spacing

But, Provide 12 @ C/C 200mm in the longitudinal direction &12 @ C/C 200mm across.

## Design of camp facility

Arranging camp facility for use during project construction according to the site condition and the society is a crucial issue which must be considered during feasibility and detail design study of a given scheme. Based on the suitability of the topography and the access conditions, a good camp site will selected. The proposed camp site is center for all activities to be done in the both the headwork and the infrastructure.  The site is located in the communal grazing land and will not have any compensation problem. The general location is shown in the drawing

# Bill of Quantity and cost estimation

In this basic topic three major tasks have been conducted. Th1st and the most important is the preparation of quantity take off sheet. The 2nd part of this topic shall be summarizing the take off sheet and preparing the Bill of quantities for each work items in the headwork. The last activities to be performed here is estimation of the unit rate specific to the project area.

Finally the engineering cost estimate for the head work shall be computed and summed up for procurement specification. Take off sheet will be annexed or given in softcopy.

Table 11-1: Bill No. 1- General Items

| Bill Nr. | Description | Unit | Quantity | Unit Rate (Birr) | Total amount (Birr) |
| --- | --- | --- | --- | --- | --- |
| 1 | General |  |  |  |  |
| 1.1 | Allow for resources Mobilization | Ls | 1 | 80,000.00 | 80,000.00 |
| 1.2 | Allow for resources Demobilization | Ls | 1 | 65,000.00 | 65,000.00 |
| 1.3 | 5*8m2, store constructed from CIS with doors and windows, masonry floor cement screened. | No | 1 | 60,000.00 | 60,000.00 |
| 1.4 | Construction of consultant's camp and camp facilities and fencing two rooms of size 3m*4m of which one for living room, and one for office; constructing from CGIS and internally painted chip wood wall, masonry floor screed and well ventilated room complete with doors and windows | No | 1 | 55,000.00 | 55,000.00 |
| 1.5 | Allow for contractors Camp and facilities 4*3m2, living room for contractors key personnel, CIS and internally painted chip wood wall , masonry floor cement screened and well ventilated room complete with doors and windows. | No | 3 | 55,000.00 | 165,000.00 |
| 1.6 | Kitchen (3*3 m2) and Daining room 4*4m2 | No | 1 | 55,000.00 | 55,000.00 |
| 1.6 | shower (2*2m2) and toilet (2*2m2) | No | 1 | 40,000.00 | 40,000.00 |
| 1.7 | Barbed wire fence 50*50 m and 1.5m high treated timber post complete with 3m wide gate and a CIS guard house (1.5*2m) | No | 1 | 50,000.00 | 50,000.00 |
| 1.8 | Allow for temporary Access road to site | km | 1 | 30,000.00 | 30,000.00 |
| 1.9 | dewatering of open trenches and excavations, temporary diversion of the river flow and pumps | Ls | 1 | 40,000.00 | 40,000.00 |
| 2 | Generator including fuel daily services 8hr | Ls | 1 | 45,000.00 | 45,000.00 |
| 2.1 | Provide project indicator post using 1m*2m flat sheet metal using RHS as column and stiffening starting from the construction time | Ls | 1 | 5,000.00 | 5,000.00 |
| 2.2 | Provision of as built drawing for the project | Ls | 1 | 10,000.00 | 10,000.00 |
|  | Total amount for general items |  |  |  | 700,000.00 |

Table 11-2:  Bill No. 2- Head Work Structure

| ` | Description | Unit | Quantity | Unit Rate | Total Cost |
| --- | --- | --- | --- | --- | --- |
|  | A. Head Work Structures |  |  |  |  |
| 1 | Weir Body including cutoff and Apron |  |  |  |  |
| 1.1 | Earth Excavation weathered Rock | m3 | 28.01 | 475.51 | 13319.42 |
| 1.2 | Earth Excavation Ordinary Soil | m3 | 313.50 | 58.72 | 18408.96 |
| 1.3 | Hard Rock | m3 | 76.50 | 630.00 | 48195.00 |
| 1.5 | cyclopean concrete(60% C-20, 40% graded stones) | m3 | 420.00 | 1872.85 | 786,597.00 |
| 1.6 | Concrete work |  |  |  |  |
| 1.6.1 | C-20 concrete for weir cap | m3 | 152.66 | 3000.00 | 457,980.00 |
| 1.7 | Reinforcement bars |  |  |  |  |
| 1.7.2 | Reinforce bar Ø12 | Kg | 905.76 | 55.00 | 38250.24 |
|  | Subtotal |  |  |  | 1,433,545.82 |
| 2 | Divided wall ,operational slab & breast wall |  |  |  |  |
| 2.1 | Earth Excavation Ordinary Soil | m3 | 1.50 | 65.00 | 97.50 |
| 2.2 | Masonry (1:3) | m3 | 7.68 | 1600.00 | 12288.00 |
| 2.3 | C-20 concrete | m3 | 12.50 | 3000.00 | 37,500.00 |
|  | ф12mm bar | Kg | 156.29 | 55.00 | 8595.84 |
| 2.4 | Plastering with a 1:3 cement mortar | m2 | 49.92 | 150.00 | 7488.00 |
| 2.5 | plastering with a 1:3 cement mortar | m2 | 4.60 | 150.00 | 690.00 |
| 2.6 | plastering with a 1:3 cement mortar | m2 | 3.50 | 150.00 | 525.00 |
|  | Sub total |  |  |  | 67,184.34 |
| 3 | UPSTREAM RETAINING WALL |  |  |  |  |
| 3.1 | Left Side Retaining Wall |  |  |  |  |
| 3.1.1 | Earth Excavation Ordinary Soil | m3 | 319.20 | 65.00 | 20748.00 |
| 3.1.2 | Back fill | m3 | 223.97 | 42.00 | 9406.82 |
| 3.1.3 | Masonry (1:3) | m3 | 362.14 | 1600.00 | 579424.00 |
| 3.1.4 | Plastering with a 1:3 cement mortar | m2 | 195.70 | 150.00 | 29355.00 |
|  | Sub total |  |  |  | 638,933.82 |
| 3.2 | Right Side Retaining Wall |  |  |  |  |
| 3.2.1 | Earth Excavation Ordinary Soil |  | 360.69 | 65.00 | 23445.05 |
| 3.2.2 | Back fill |  | 319.21 | 42.00 | 13406.95 |
| 3.2.3 | Masonry (1:3) |  | 518.50 | 1600.00 | 829,600.00 |
| 3.2.4 | Plastering with a 1:3 cement mortar |  | 314.15 | 150.00 | 47,122.50 |
|  | Sub total |  |  |  | 913,574.49 |
| 4 | DOWNSTREAM RETAINING WALL |  |  |  |  |
| 4.1 | Left Side Retaining Wall |  |  |  |  |
| 4.1.1 | Earth Excavation Ordinary Soil | m3 | 120.69 | 65.00 | 7844.85 |
| 4.1.2 | Back fill | m3 | 112.59 | 42.00 | 4728.78 |
| 4.1.3 | Masonry (1:3) | m3 | 160.38 | 1600.00 | 256608.00 |
| 4.1.4 | Plastering with a 1:3 cement mortar | m2 | 110.70 | 150.00 | 16605.00 |
|  | Sub total |  |  |  | 285,786.63 |
| 4.2 | Right Side Retaining Wall |  |  |  |  |
| 4.2.1 | Earth Excavation Ordinary Soil | m3 | 324.80 | 65.00 | 21112.00 |
| 4.2.2 | Back fill | m3 | 255.20 | 42.00 | 10718.40 |
| 4.2.3 | Masonry (1:3) | m3 | 460.52 | 1600.00 | 736832.00 |
| 4.2.4 | Plastering with a 1:3 cement mortar | m3 | 237.80 | 150.00 | 35670.00 |
|  | Sub total |  |  |  | 804,332.40 |
| 5 | Gates |  |  |  |  |
| 5.1 | Off take canal gate and installation, 6mm thickness sheet metal ,40mmX30mmX4mm angle iron, ∅12mm bar for anchorage square iron tube with 2mm thickness | No. |  |  | 25,000.00 |
| 5.2 | Under sluice gate and installation, 8mm thickness sheet metal or each 30mmX30mmX4mm angle iron, ∅12mm bar for anchorage, 40mmx30mm square iron tube with 2mm thickness stifenning | No |  |  | 45,000.00 |
|  | Grand total Headwork |  |  |  | 3,434,336.46 |

# Conclusion and recommendation

Azuwary-2 irrigation scheme is proposed for irrigating a traditionally developed irrigable land of area 100ha. The headwork site is located at an altitude of 2453m asl.  The crest length of the weir is 17m and a regulator weir height of 2.0 m.

The weir abstracts water and supply to the command area only right side of the river with one masonry lined main canals.The total headwork engineering estimated cost is 3,434,336.46 birr.

# SECTION-III: IRRIGATION AND DRAINAGE SYSTEMS INFRASTRUCTURE

# Irrigation Infrastructure

## Irrigable command area

### Topography and soil

Topography is an important factor for the planning of any irrigation project as it influences method of irrigation, drainage, erosion, mechanization, and cost of land development, labour requirement and choice of crops.

The topographic feature of the far surroundings of the project area, especially in the –North and South a direction, is dominated by chains of mountainous escarpments. Nonetheless, the topographic feature of the command area slope is ranging from 2-5 %. The slope gradient class also ranges from nearly level (2%). Hence, it has identified to be almost suitable for surface irrigation.

### Irrigation Efficiency and irrigation duty

Irrigation duty is the volume of water required per hectare for the full flange of the crops. Moreover, it helps in designing an efficient irrigation canal system. The area which will be irrigated can be calculated by knowing the total available water at the source and the overall duty for all crops required to be irrigated in different seasons of the years. The proposed cropping pattern of Azuwary-2 diversion weir irrigation project has showed a maximum net irrigation water requirement (NIWR) in the month of March with the amount of 3.3  mm/day for annual proposed crops for 24 working/irrigation hours.

Irrigation duty is the volume of water required per hectare for the full flange of the crops. Moreover, it helps in designing an efficient irrigation canal system. The area which will be irrigated can be calculated by knowing the total available water at the source and the overall duty for all crops required to be irrigated in different seasons of the years. Once the conveyance and field application efficiencies are determined or estimated, the scheme irrigation efficiency (e) for the selected surface irrigation method can be calculated using the following formula:

e = ec × ea/100 = 60 × 80 ≈ 50% Where, e = scheme irrigation efficiency (%) ec = conveyance efficiency (%) ea = field application efficiency (%)

For the designing of the project, the GIWR is given as follows: GIWR = NIWR/e = 3.3/0.50 = 7.4 [mm/day]

The GIWR, 8.8 mm/day, represents the daily quantity of water that is required to be applied. This water quantity is also used for the determination of the canal discharge in consideration of the time of flow and is defined as the duty, expressed as l/s/ha. The duty is calculated by:

Duty (D) = GIWR × 1000 × 10 / (t × 60×60)

Where; Duty – the duty [l/s/ha] ,GIWR – Gross Irrigation Requirement [mm/day],  t – Daily irrigation or flow hours [hrs] The duty for the GIWR of 8.8 mm/day and 18 hours of daily irrigation time (t = 18), is supported to be used with furrow irrigation method.

Hence, Duty for 18 working hours, as the site is nearer to farmers’ village, is computed as follows: D = (7.4 x 1000 x 10) / (18 x 3600) = 1.2l/s/h

Based on this and the type of crops proposed for the project area, the design duty for Azuwary-2 irrigation project has been determined as 1.2l/s/ha. For detail, please refer agronomist report.

The gross command area is large enough available all downstream of the proposed irrigation project. However, the surveyed irrigable land gross size is about 130ha and the net irrigable land for the project is 100ha. This amount may increase in the 1st period irrigation where water source is available enough for all uses.

### Irrigation Interval

Correct timing of application has a great importance for a given crop. The following relationship is used to determine the irrigation interval and schedules of the proposed crops in the scheme which can be clearly set in the operation and maintenance document.

- Irrigation Interval: (I) = P x Sa x d

- Depth of Irrigation application (d)  =  P x Sa x D/   Ea= 60%

Where Sa = Total available soil, mm/M soil depth

P = Fraction of available water

ETC= maximum crop evapotranspiration per day

Ea = Application efficiency, fraction

D= Root Depth

Since P, D, and ETC will vary over the growing season, the depth in mm and interval of irrigation in days will vary. Hence, if there is a need of irrigation application, the farmer may use hand touching and observation.

Though the canal is design to carry the critical base flow of 110 l/s, the canal has been given enough freeboard and can be used in excess flows in good season flows available. The design discharges for each canal type is presented in the previous table.

## Irrigation & Drainage system lay out

### Canal layout

The irrigation scheme layout was prepared by considering the general topography of the land, the soil characteristics, minimizing crossing structure and slope of the canal. Several alternatives were considered for the conveyance and distribution system. The most promising and economical of these alternatives were recommended for more details. The irrigation system comprises three major components: the main canal, tertiary canal and field canal system.

The general layout system consists of one main canals namely right canal, Secondary canal, tertiary canals and field canals. In order to facilitate the drainage system the natural drains available across the command area has been used as a tertiary drain and field drains are provided based on the topography of the site. The layout system covers a net command area of 100ha

Water is conveyed through the main canals from the diversion weir. The flow through the main canal is continuous type. And the flow through tertiary and field canals is on rotational base. For easy distribution, division boxes and turnouts are provided at the head of tertiary and filed canals respectively.

The conveyance system consists of Main canals to irrigate total command area of 100.0ha. Main canal (MC-1) has a length of 4.5 km and starts 6.3m away from the offtake location. Tertiary canals are designated as distribution system.  The Layout process of the distribution system has involved several steps and the parameters Topographical, Field slope and its uniformity and Sizes of the canals are determined based on the requirement in the field unit. The following plate shows the general feature of the command area for which irrigation system is to be developed.

### Drainage system

The drainage system is provided for effective disposal of excess irrigation water and runoff from the irrigation field during rainy season. Field drains and catch drain collect from each field units and topo above the canals will be conveyed with drainage crossing structure and directly discharge into the river and natural Drainage system. All the drains are natural and except the protection of their nature, no artificial drain has been made.

## Selection of Canal Design Section

### Main Canal

From the head work report we have got the canal outlet capacity is fixed considering optimum design base flow.  Therefore Outlet capacity for the main canal is sized based on the available command area. Therefore the command area available in the right side of the river is 100ha and the MCis designed with a Design base flow of123 l/s.

Using weir discharge equation the outlet size has been determined as follows

Q= CLH3/2

Where   C= coeff.of discharge = 1.7

L= Length of water way (m)

H= head above crest level (m

Table 13-1:  off take sizing

| Items | MC computed |
| --- | --- |
| design base flow, m3/s | 0.10 |
| Height of flow, H | 0.45 |
| C | 1.70 |
| width of offtake | 0.19 |
| gate size, 5mm in | 0.55x0.50 |
| offtake bed elevation | 2454.85 |

All of the command area is found in the right side. Geotechnical investigation of the canal route has been carried out in view of permeability, workability, seepage and instability. Accordingly, a number of traverses following the existing canal have been made to characterize the permeability nature of the soil in the area.

Therefore, from permeability point of view the rout is relatively thin impervious underlain by the pervious deposits and there will be significant amount of seepage if the canal is unlined (earthen). However, other canal systems other than the primary canal can be earthen. All the deposits are easily excavatable. So, there is no any workability problem.

At around a km away from the weir structure there is a gorge crossed by the main canal. As a result, a crossing structure is needed. The foundation geology of the crossing site is the silty clay of thickness to more than 2m. So, it needs either excavating this top soil until the lower gravel-cobble layer is exposed or back filling it by better graded material or use some shallow seated piling. Furthermore, bank erosion is higher by this non-perennial gorge and it needs erosion resistant structures confined to crossing foundation.

This design is made with the intension of lining the main canal considering the pervious nature of the soil. The lined canal will save the limited water source available there. The topography is increasingly dropping the whole length of the canal and therefore the main canal is designed with the ground surface slope with total in cut and zero fill. This design arrangement with the natural slope changes the canal design to run in chute section for variable ground slopes as shown in the above section. Both Main canal sections is designed by using Manning’s equation for both masonry and earthen sections as shown below.

Q =     , R = A/P

Where Q = Design discharge, m3/s

R = Hydraulic radius of the canal, m

A = Wetted cross sectional area of canal, m2

P = Wetted Perimeter, m

n = Manning’s roughness coefficient

S = Bed slope of the canal

Figure 13-1:   lined canal section view

### Secondary Canal

The secondary canal is aligned across the contour to convey water from main canal to tertiary. The secondary canal is designed as lined canal as shown below in the figure and designed using manning’s formula. Hydraulic parameters of the secondary canal are shown in the table below.

Figure 13-2 Typical lined secondary canal fully in cut

Figure 13-3Typical lined secondary canal in partial fill & cut

### Tertiary canal

Considering the topography orientation and which is tapered and the command area small in size there is no secondary canal in this project. But instead tertiary canals take water from the Main canal and distribute to field canals.

There are 25 tertiary canals and their sections are proposed to be trapezoidal Side slope for the earthen canals is taken to be 1:1.

Q =     , R=A/P

Where

Q = Design discharge, m3/s

R = Hydraulic radius of the canal, m

A = Wetted cross sectional area of canal, m2

P = Wetted Perimeter, m

n = Manning’s roughness coefficient

S = Bed slope of the canal

Figure 13-4 : tertiary canal section

The hydraulic Parameters for each variable are calculated by Iteration method with MS-EXL program for main canal, secondary canal & tertiary canals as shown in table below.

## Design of Irrigation canals

### Main canal

Canal Section

Considering the length of the main canals, the slope of the land, the seepage of water, the scarcity of water and the costs during operation and maintenance, the main canal is proposed to have masonry lining so as to maximize the efficiency of the irrigation. The detail is presented in the above table. The canal section is rectangular masonry lined and is designed using the manning’s formula.

Using this formula for different design discharge, the canal section is determined and the result is shown below in the table. The canal design slope is probably the ground slope where there is zero fill principle. There are four major ground slopes as shown in the following table. However, based on the information from main canal profile the slopes may vary and the detail shall be shown in canal profile drawing.

Table 13-2: hydraulic characteristics of the main canal

| Chain age(m) | Qd (m3/sec) | B (m) | D (m) | Fb (m) | D+Fb | Z | n | S | V |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 +00-0+840 | 0.123 | 0.60 | 0.47 | 0.20 | 0.67 | 1 | 0.018 | 0.0007 | 0.46 |
| 0 +00-0+840 | 0.123 | 0.6 | 0.45 | 0.3 | 0.75 | 1 | 0.018 | 0.00077 | 0.48 |
| 0+840-2+139 | 0.123 | 0.6 | 0.45 | 0.3 | 0.75 | 1 | 0.018 | 0.00057 | 0.43 |
| 2+139-2+239 | 0.123 | 0.6 | 0.45 | 0.3 | 0.75 | 1 | 0.018 | 0.00455 | 0.94 |
| 2+239-2+339 | 0.123 | 0.6 | 0.45 | 0.3 | 0.75 | 1 | 0.018 | 0.00125 | 0.58 |
| 2+339-2+839 | 0.123 | 0.6 | 0.45 | 0.3 | 0.75 | 1 | 0.018 | 0.00067 | 0.46 |
| 2+839-3+099 | 0.123 | 0.6 | 0.45 | 0.3 | 0.75 | 1 | 0.018 | 0.00286 | 0.79 |
| 3+099-3+218 | 0.123 | 0.6 | 0.45 | 0.3 | 0.75 | 1 | 0.018 | 0.00071 | 0.47 |
| 3+218-3+718 | 0.123 | 0.6 | 0.45 | 0.3 | 0.75 | 1 | 0.018 | 0.00125 | 0.58 |
| 3+718-3+818 | 0.123 | 0.6 | 0.45 | 0.3 | 0.75 | 1 | 0.018 | 0.01250 | 1.38 |
| 3+818-3+978 | 0.123 | 0.6 | 0.45 | 0.3 | 0.75 | 1 | 0.018 | 0.00119 | 0.57 |
| 3+978-4+118 | 0.123 | 0.6 | 0.45 | 0.3 | 0.75 | 1 | 0.018 | 0.00417 | 0.91 |
| 4+118-4+497 | 0.123 | 0.6 | 0.45 | 0.3 | 0.75 | 1 | 0.018 | 0.00100 | 0.53 |

### Tertiary Canal

There are 25 tertiary canals running across the main canals and the length of the tertiary canal varies from place to place in the command area. Considering the capacity of an irrigator to handle the discharge and the existing natural land block, the design discharge of the tertiary vary .All tertiary canals are trapezoidal earthen canals except TC-1-1 which is lined and the section of the canals are determined using the manning’s formula. Considering the siltation problems of earthen canals 0.03 manning’s coefficient is adopted for earthen canals. Detail hydraulic characteristics of the tertiary canals are shown below. In the table the computed values are inserted but for practical case take the bed width for all tertiary as b=0.3m when it is below 0.30m.

Table 13-3  Hydraulic characteristics of the tertiary canal

| Canal name | Chain age(m) | Qd (m3/sec) | B (m) | D (m) | Fb (m) | D+Fb | Z | n | S | V(m/s) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TC-1-1-1 | 0+ 000-0+040 | 0.006 | 0.3 | 0.1 | 0.2 | 0.3 | 1.5 | 0.025 | 0.0083 | 0.43 |
|  | 0+040 -0+222 | 0.006 | 0.3 | 0.1 | 0.2 | 0.3 | 1.5 | 0.025 | 0.0036 | 0.31 |
| TC-1-1-2 | 0+ 000-0+0140 | 0.008 | 0.3 | 0.1 | 0.2 | 0.3 | 1.5 | 0.025 | 0.0025 | 0.30 |
|  | 0+140 -0+200 | 0.008 | 0.3 | 0.1 | 0.2 | 0.3 | 1.5 | 0.025 | 0.0031 | 0.32 |
|  | 0+ 200-0+280 | 0.008 | 0.3 | 0.1 | 0.2 | 0.3 | 1.5 | 0.025 | 0.0050 | 0.38 |
|  | 0+280 -0+340 | 0.008 | 0.3 | 0.1 | 0.2 | 0.3 | 1.5 | 0.025 | 0.0067 | 0.43 |
|  | 0+ 340-0+490 | 0.008 | 0.3 | 0.1 | 0.2 | 0.3 | 1.5 | 0.025 | 0.0025 | 0.30 |
| TC-1-1-3 | 0+ 000-0+080 | 0.008 | 0.3 | 0.1 | 0.25 | 0.35 | 1.5 | 0.025 | 0.0100 | 0.50 |
|  | 0+080 -0+220 | 0.008 | 0.3 | 0.1 | 0.25 | 0.35 | 1.5 | 0.025 | 0.0028 | 0.30 |
|  | 0+ 220-0+340 | 0.008 | 0.3 | 0.1 | 0.25 | 0.35 | 1.5 | 0.025 | 0.0056 | 0.40 |
|  | 0+340 -0+480 | 0.008 | 0.3 | 0.1 | 0.25 | 0.35 | 1.5 | 0.025 | 0.0038 | 0.35 |
| TC-1-1-4 | 0+ 000-0+080 | 0.005 | 0.3 | 0.1 | 0.2 | 0.3 | 1.5 | 0.025 | 0.0067 | 0.38 |
|  | 0+080 -0+160 | 0.005 | 0.3 | 0.1 | 0.2 | 0.3 | 1.5 | 0.025 | 0.0077 | 0.40 |
|  | 0+ 160-0+420 | 0.005 | 0.3 | 0.1 | 0.2 | 0.3 | 1.5 | 0.025 | 0.0034 | 0.30 |
| TC-1-2-1 | 0+ 000-0+200 | 0.007 | 0.3 | 0.1 | 0.2 | 0.3 | 1.5 | 0.025 | 0.0026 | 0.30 |
|  | 0+200 -0+340 | 0.007 | 0.3 | 0.1 | 0.2 | 0.3 | 1.5 | 0.025 | 0.0033 | 0.32 |
| TC-1-2-2 | 0+ 000-0+120 | 0.008 | 0.3 | 0.1 | 0.2 | 0.3 | 1.5 | 0.025 | 0.0182 | 0.63 |
|  | 0+120 -0+340 | 0.008 | 0.3 | 0.1 | 0.2 | 0.3 | 1.5 | 0.025 | 0.0053 | 0.39 |
| TC-1-3-1 | 0+ 000-0+120 | 0.002 | 0.3 | 0.1 | 0.2 | 0.3 | 1.5 | 0.025 | 0.0063 | 0.31 |
| TC-1-3-2 | 0+ 000-0+130 | 0.003 | 0.3 | 0.1 | 0.2 | 0.3 | 1.5 | 0.025 | 0.0050 | 0.30 |
| TC-1-3-3 | 0+ 000-0+100 | 0.003 | 0.3 | 0.1 | 0.2 | 0.3 | 1.5 | 0.025 | 0.0045 | 0.30 |
| TC-1-3-4 | 0+ 000-0+140 | 0.003 | 0.3 | 0.1 | 0.2 | 0.3 | 1.5 | 0.025 | 0.0063 | 0.32 |
| TC-1-3-5 | 0+ 000-0+040 | 0.003 | 0.3 | 0.1 | 0.2 | 0.3 | 1.5 | 0.025 | 0.0063 | 0.32 |
|  | 0+040 -0+160 | 0.005 | 0.3 | 0.1 | 0.2 | 0.3 | 1.5 | 0.025 | 0.0033 | 0.30 |
| TC-1-4-1 | 0+ 000-0+100 | 0.002 | 0.3 | 0.1 | 0.2 | 0.3 | 1.5 | 0.025 | 0.0071 | 0.32 |
| TC-1-4-2 | 0+ 000-0+060 | 0.006 | 0.3 | 0.1 | 0.2 | 0.3 | 1.5 | 0.025 | 0.0056 | 0.37 |
|  | 0+ 060-0+158 | 0.006 | 0.3 | 0.1 | 0.2 | 0.3 | 1.5 | 0.025 | 0.0031 | 0.30 |
| TC-1-4-3 | 0+ 000-0+105 | 0.003 | 0.3 | 0.1 | 0.2 | 0.3 | 1.5 | 0.025 | 0.0083 | 0.37 |
| TC-1-5-1 | 0+ 000-0+165 | 0.002 | 0.3 | 0.1 | 0.2 | 0.3 | 1.5 | 0.025 | 0.0063 | 0.31 |
| TC-1-5-2 | 0+ 000-0+095 | 0.002 | 0.3 | 0.1 | 0.2 | 0.3 | 1.5 | 0.025 | 0.0069 | 0.30 |
| TC-1-5-3 | 0+ 000-0+136 | 0.003 | 0.3 | 0.1 | 0.2 | 0.3 | 1.5 | 0.025 | 0.0069 | 0.35 |
| TC-1-5-4 | 0+ 000-0+090 | 0.002 | 0.3 | 0.1 | 0.2 | 0.3 | 1.5 | 0.025 | 0.0071 | 0.32 |
| TC-1-5-5 | 0+ 000-0+154 | 0.003 | 0.3 | 0.1 | 0.2 | 0.3 | 1.5 | 0.025 | 0.0048 | 0.30 |
| TC-1-5-6 | 0+ 000-0+080 | 0.003 | 0.3 | 0.1 | 0.2 | 0.3 | 1.5 | 0.025 | 0.0050 | 0.30 |
| TC-1-6-1 | 0+ 000-0+144 | 0.002 | 0.3 | 0.1 | 0.2 | 0.3 | 1.5 | 0.025 | 0.0067 | 0.31 |
| TC-1-6-2 | 0+ 000-0+060 | 0.003 | 0.3 | 0.1 | 0.2 | 0.3 | 1.5 | 0.025 | 0.0100 | 0.38 |
|  | 0+ 060-0+179 | 0.003 | 0.3 | 0.1 | 0.2 | 0.3 | 1.5 | 0.025 | 0.0100 | 0.38 |
| TC-1-6-3 | 0+ 000-0+037 | 0.002 | 0.3 | 0.1 | 0.2 | 0.3 | 1.5 | 0.025 | 0.0080 | 0.32 |
|  | 0+ 037-0+112 | 0.003 | 0.3 | 0.1 | 0.2 | 0.3 | 1.5 | 0.025 | 0.0047 | 0.30 |
| TC-1-7-1 | 0+ 000-0+136 | 0.002 | 0.3 | 0.1 | 0.2 | 0.3 | 1.5 | 0.025 | 0.0067 | 0.31 |
| TC-1-7-2 | 0+ 000-0+134 | 0.003 | 0.3 | 0.1 | 0.2 | 0.3 | 1.5 | 0.025 | 0.0054 | 0.30 |
| TC-1-7-3 | 0+ 000-0+100 | 0.002 | 0.3 | 0.1 | 0.2 | 0.3 | 1.5 | 0.025 | 0.0087 | 0.35 |
| TC-1-7-4 | 0+ 000-0+040 | 0.003 | 0.3 | 0.1 | 0.2 | 0.3 | 1.5 | 0.025 | 0.0100 | 0.38 |
|  | 0+ 040-0+140 | 0.004 | 0.3 | 0.1 | 0.2 | 0.3 | 1.5 | 0.025 | 0.0042 | 0.31 |
| TC-1-8-1 | 0+ 000-0+126 | 0.002 | 0.3 | 0.1 | 0.2 | 0.3 | 1.5 | 0.025 | 0.0063 | 0.30 |
| TC-1-8-2 | 0+ 000-0+133 | 0.003 | 0.3 | 0.1 | 0.2 | 0.3 | 1.5 | 0.025 | 0.0050 | 0.30 |
| TC-1-8-3 | 0+ 000-0+120 | 0.003 | 0.3 | 0.1 | 0.2 | 0.3 | 1.5 | 0.025 | 0.0100 | 0.38 |
| TC-1-8-4 | 0+ 000-0+140 | 0.003 | 0.3 | 0.1 | 0.2 | 0.3 | 1.5 | 0.025 | 0.0059 | 0.31 |

### Field Canals

Field canals are supplied water from tertiary canals through turnout structures. The field canals will be constructed by the farmers and remove while they plough the irrigation field.

For the next irrigation, they can easily construct again. Field canals are constructed with the same sections of the respective tertiary canals.  Field canals are designed based on the existing canal system and footpath system alignment and hence the length of field canals varies for ease of operation.

## Design of Drainage System

The main purpose of the drainage system in the command area is to remove the flood during rain

Season and surplus irrigation water from the entire system. As discussed earlier, all the drains are natural ones. Collector drains takes from each field drains and empties to the natural waterways.  The field drains will be constructed by the farmers and remove while they plough the irrigation field. For the next irrigation, they can easily construct again. Field canals are constructed with the same sections of the respective field canals.

Table 13-4 Hydraulic parameters of tertiary drain canals

| Canal name | Chain age(m) | Qd (m3/sec) | B (m) | D (m) | Fb (m) | D+Fb | Z | n | S | V(m/s) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TD-1-1-1 | 0 +00-0+562 | 0.27 | 0.55 | 0.35 | 0.20 | 0.55 | 1.5 | 0.025 | 0.0022 | 0.67 |
| TD-1-1-2 | 0 +00-0+433 | 0.29 | 0.50 | 0.40 | 0.20 | 0.60 | 1.5 | 0.025 | 0.0020 | 0.66 |
| TD-1-2-1 | 0 +00-0+407 | 0.23 | 0.50 | 0.35 | 0.20 | 0.55 | 1.5 | 0.025 | 0.0022 | 0.65 |
| TD-1-3-1 | 0 +00-0+123 | 0.08 | 0.40 | 0.25 | 0.20 | 0.45 | 1.5 | 0.025 | 0.0014 | 0.42 |
| TD-1-3-2 | 0 +00-0+040 | 0.09 | 0.40 | 0.25 | 0.20 | 0.45 | 1.5 | 0.025 | 0.0022 | 0.51 |
|  | 040+00-0+176 | 0.09 | 0.40 | 0.25 | 0.20 | 0.45 | 1.5 | 0.025 | 0.0059 | 0.70 |
| TD-1-3-3 | 0 +00-0+237 | 0.1 | 0.40 | 0.30 | 0.20 | 0.50 | 1 | 0.025 | 0.0009 | 0.38 |
| TD-1-4-1 | 0 +00-0+030 | 0.09 | 0.35 | 0.25 | 0.20 | 0.45 | 1.5 | 0.025 | 0.0019 | 0.48 |
|  | 0+030-00+ | 0.09 | 0.35 | 0.25 | 0.20 | 0.45 | 1.5 | 0.025 | 0.0050 | 0.70 |
| TD-1-5-1 | 0 +00-0+050 | 0.08 | 0.35 | 0.20 | 0.20 | 0.40 | 1.5 | 0.025 | 0.0043 | 0.65 |
|  | 0+050-00+184 | 0.08 | 0.35 | 0.20 | 0.20 | 0.40 | 1.5 | 0.025 | 0.0056 | 0.70 |
| TD-1-5-2 | 0 +00-0+125 | 0.06 | 0.30 | 0.20 | 0.20 | 0.40 | 1.5 | 0.025 | 0.0043 | 0.60 |
| TD-1-5-3 | 0 +00-0+229 | 0.1 | 0.30 | 0.25 | 0.20 | 0.45 | 1.5 | 0.025 | 0.0034 | 0.63 |
| TD-1-5-4 | 0 +00-0+116 | 0.05 | 0.30 | 0.20 | 0.20 | 0.40 | 1.5 | 0.025 | 0.0050 | 0.61 |
| TD-1-6-1 | 0 +00-0+208 | 0.08 | 0.30 | 0.25 | 0.20 | 0.45 | 1.5 | 0.025 | 0.0050 | 0.68 |
| TD-1-6-2 | 0 +00-0+111 | 0.12 | 0.40 | 0.25 | 0.20 | 0.45 | 1.5 | 0.025 | 0.0038 | 0.68 |
|  | 0+000-0+203 | 0.12 | 0.40 | 0.25 | 0.20 | 0.45 | 1.5 | 0.025 | 0.0025 | 0.58 |
| TD-1-7-1 | 0 +00-0+127 | 0.07 | 0.30 | 0.25 | 0.20 | 0.45 | 1.5 | 0.025 | 0.0013 | 0.39 |
| TD-1-7-2 | 0 +00-0+167 | 0.09 | 0.30 | 0.25 | 0.20 | 0.45 | 1.5 | 0.025 | 0.0043 | 0.66 |
| TD-1-8-1 | 0 +00-0+156 | 0.07 | 0.30 | 0.20 | 0.20 | 0.40 | 1.5 | 0.025 | 0.0042 | 0.61 |
| TD-1-8-2 | 0 +00-0+154 | 0.09 | 0.30 | 0.20 | 0.20 | 0.40 | 1.5 | 0.025 | 0.0042 | 0.65 |

## Collector drain canal design

The collector drain canals have been designed with a rectangular x-section using Manning's Formula:

Where Q= discharge (m3/s)

R= Hydraulic radius (Flow area/wetted perimeter)

S= Hydraulic gradient

n= Manning's roughness coefficient n=0.018

There are 3 collector drain aligned across the contour to discharge excess irrigation water collecting from tertiary field drain to the nearby gully or stream. The reason which are to be designed as rectangular x-section lined canals is, the maximum velocity flow since its alignment is across the contour. Hydraulic parameter and section of them are shown below in table 7

Figure 13-5Typical collector drain partially in fill & cut section

Table 13-5 Hydraulic parameters of  collector drain

| Canal name | Chain age(m) | Qd (m3/sec) | B (m) | D (m) | Fb (m) | D+Fb | Z | n | S | V(m/s) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CD-1 | 0 +00-0+076 | 0.02 | 0.3 | 0.15 | 0.2 | 0.35 | 1.5 | 0.018 | 0.0667 | 1.55 |
|  | 0 +076-0+303 | 0.02 | 0.3 | 0.15 | 0.2 | 1.35 | 1.5 | 0.018 | 0.0100 | 0.75 |
| CD-2 | 0 +00-0+077 | 0.01 | 0.3 | 0.15 | 0.2 | 0.35 | 1.5 | 0.018 | 0.0833 | 1.48 |
|  | 0 +077-0+173 | 0.01 | 0.3 | 0.15 | 0.2 | 0.35 | 1.5 | 0.018 | 0.0125 | 0.73 |
| CD-3 | 0 +00-0+041 | 0.05 | 0.3 | 0.1 | 0.2 | 0.3 | 1.5 | 0.018 | 0.0667 | 1.16 |
|  | 0+041-0+151 | 0.05 | 0.3 | 0.1 | 0.2 | 0.3 | 1.5 | 0.018 | 0.0333 | 0.86 |

# Irrigation Structures

## Design of Division boxes and turnouts

The proportional division of water among receiver canal and delivery canal is effected by providing division box. There are 20 tertiary division structures to be developed from the one main canals. Since all the TC division structures /division boxes are one sided boxes, design of one sided division box is shown. Turnouts are recommended for diverting flows from TCs to FCs (field canals). The principle of designing of the division box a opening width is computed based on the rectangular notch weir formula as follow.

The proportionality of the division boxs is adjusted with the width of the outlet and   the head over the sill which can be the same on both canals. At each division boxes, control gates are provided. When the computed division box crust width is too small, for simplicity for small design flows as Azuwary-2, basin width of the turnout is equal to the bed width of the main canal

Width of tertiary canal /outlet = (QTC /QO) x bo

Where QTC = Design discharge to tertiary canal

Qo = Design on going discharge in the main canal

bo = Main canal bed width for ongoing discharge

Using the above relations the hydraulic characteristics of the turnouts are determined as shown below figure.

Figure 14-1: typical division box drawing

## Design of field canal turnouts

Turnouts are provided at the tertiary canals to divert or to release the flow into the field canals. The width of the turnouts has been decided to take as the same as the bed width of tertiary canal since the computed value for such small flows is minimum.  On both side of the Tertiary and field canals control gates are provided.

Based on the system alignment and the nature of topography, a turnout can supply for two field canals. Considering the managing capacity of the farmers,  the field canals shall be designed by uing a discharge of  10 l/s.  the detail is shown  in the following table.

Figure 14-2 : Turnouts from masonry lined TC-canals (left) and from earthen TC canals (right)

## Design of Drops

Well, the design of drops may be necessary in steep slope commands like Azuwary-2. The length of tertiary canals is higher than that of the main canal. When looking the general topography of the Azuwary-2 site, all the length the main canal is running in a flat area so that note more than one drop required  for main canal  in order to find an ideal command area which may save water and protect seepage. This was due to limited discharge available in the basin. Therefore, main canal (MC-1) requires one droop structures.  Critical flow hydraulics has been made in excel spread sheet analysis and the result is attached here in the following table.

Table 14-1 Hydraulic characteristics of drops

| Drop on | Chain age | Q(m3/s) | V (m/s) |  | b(m) | H(m) | d(m) | h(m) | b1(m) | q(m) | dc(m) | L(m) | B(m) | t(m) | a(m) | bc(m) | L1(m) | L2(m) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SC-1-3 | 0+020 | 0.008 | 1.1 |  | 0.35 | 0.16 | 0.10 | 0.50 | 0.4 | 0.03 | 0.04 | 0.4 | 0.3 | 0.4 | 0.2 | 0.3 | 1.3 | 1.3 |
| SC-1-4 | 0+020 | 0.005 | 1.1 |  | 0.35 | 0.16 | 0.10 | 0.60 | 0.4 | 0.02 | 0.03 | 0.4 | 0.3 | 0.4 | 0.2 | 0.3 | 1.3 | 1.3 |
|  | 0+100 | 0.005 | 1.0 |  | 0.35 | 0.15 | 0.10 | 0.50 | 0.4 | 0.02 | 0.03 | 0.3 | 0.3 | 0.4 | 0.2 | 0.3 | 1.3 | 1.3 |

Standard drawing of drop structures is given below

Figure 14-3 Standard drawing of drop structures is given below

## Cross drainage

There is one large gully width of various from 18m and therefore for sustainability purpose depending on the site condition, an RCC cross gully structure is designed.

Sample drawing is shown below. All the structures are checked against stability analysis. Each wall has a line concrete of C-5 of thickness 15cm at the bed of the walls.

Flumes are most known irrigation structures that are used to convey water across depressed land and gully. In this project, one flume is designed to pass water.

Hydraulic characteristics of the canal flume at chain age 1+329 m

Length of the flum: 18m

Shape of the flume:  rectangular, Roughness coefficient, n =0.014

From the canal longitudinal profiles,

u/s canal bed level (CBL) = 2453.926m

D/s Canal bed level (CBL) =2453.903m

U/s full supply level (FSL) =2454.326

D/s Full Supply Level =2454.303 m

Total head loss between the inlet & Outlet =0.0149m

Table 14-2: Hydraulic Parameters of Flume  at chain age 1+329

| Flume on | start Chain age | Q | V | L | h | t |
| --- | --- | --- | --- | --- | --- | --- |
| Flume 1 | 1+329 | 120 | 0.48 | 18 | 0.4 | 0.2 |

Figure 14-4:  gully crossing plan

## Design of Road Crossing Structures

  - Three road crossing structures are provided on the main canal, at the existing road places. There are three road crossing structure. The road crossing structures are rectangular double reinforced concrete slab. The slab is reinforced with Ø 12mm @150mmc/c top slab & Ø 10mm @150mmc/c bottom slab. Width of the slab is may differ with the perspective canal bed width. But the length of each slab & its thickness is the same except one road which is 8m, &20cm respectively.

## Supper passage structure

Super passage is the most known irrigation structures that are used to convey water across depressed land and gully. In this project, there are nine super passages are designed to pass water.

These supper passage structures are provided on main canals at the existing flumes where the main canal crosses. The slab is single reinforced with Ø 10mm @130mmc/c. Width of the slab is may differ with the perspective canal bed width. But the length of each slab & its thickness is the same which is 4m&15cm respectively

# Bill of Quantity and cost estimation

In this basic topic three major tasks have been conducted. The 1st and the most important is the preparation of quantity take off sheet. The 2nd part of this topic shall be summarizing the take-off sheet and preparing the Bill of quantities for each work items in the headwork. The last activities to be performed here is estimation of the unit rate specific to the project area.

Finally the engineering cost estimate for the infrastructure shall be computed and summed up for procurement specification. Take off sheet will be annexed or given in softcopy.

Table 15-1:  Bill No. 3- Engineering cost estimation for Infrastructure

| I. No | Descriptions | Unit | Qty | Rate (Birr) | Amount (Birr) |
| --- | --- | --- | --- | --- | --- |
| 1.1 | Main Canals Earthworks |  |  |  |  |
| 1.1.1 | Ordinary soil excavation | m3 | 6534.4 | 65 | 424734.50 |
| 1.1.2 | fill & compaction | m³ | 340.5 | 80 | 27242.42 |
| 1.1.3 | Back fill | m³ | 1706.6 | 42 | 71675.10 |
| 1.1.4 | Masonry work (30:70 masonry) with 1:3 mortar ratio | m3 | 2112.5 | 1600 | 3380051.46 |
| 1.1.5 | Plastering of 8mm thickness with 1:3 mortar | m2 | 9740.2 | 150 | 1461027.67 |
| 2 | Flumes |  |  |  |  |
| 2.1.1 | Ordinary soil excavation | m3 | 10.3 | 65 | 668.20 |
| 2.1.2 | Back fill | m3 | 4.1 | 80 | 329.03 |
| 2.1.3 | Lean concrete ( C-10) for masonry pad | m³ | 0.6 | 1900 | 1187.50 |
| 2.1.4 | C-20 concrete(1:2:3) | m³ | 9.6 | 3000 | 28680.00 |
| 2.1.6 | Masonry work (30:70 masonry) with 1:3 mortar ratio | m³ | 14.3 | 1600 | 22824.96 |
| 2.1.7 | RCC bar Ø8 | kg | 20.6 | 55 | 1133.09 |
| 2.1.8 | RCC bar Ø10 | kg | 547.7 | 55 | 30123.43 |
| 2.1.9 | RCC bar Ø12 | kg | 163.3 | 55 | 8982.58 |
| 2.1.10 | RCC bar Ø14 | kg | 207.2 | 55 | 11398.19 |
| 2.1.11 | Plastering of 8mm thickness with 1:3 mortar | m2 | 74.8 | 150 | 11220.00 |
| 3 | Turnout/ Division box |  |  |  | 0.00 |
| 3.1 | Lean concrete ( C-10) for masonry pad | m³ | 2.7 | 1900 | 5195.75 |
| 3.1.1 | Masonry work (30:70 masonry) with 1:3 mortar ratio | m3 | 8.9 | 1600 | 14216.00 |
| 3.1.2 | Plastering of 8mm thickness with 1:3 mortar | m2 | 51.3 | 150 | 7690.50 |
| 3.1.3 | Stone Pitching | m3 | 30.5 | 353 | 10771.39 |
| 4 | All Gate |  |  |  | 0.00 |
| 4.1 | Off take get | no | 1.0 | 10000 | 10000.00 |
| 4.1.2 | Turn out | no | 6.0 | 5000 | 30000.00 |
| 5 | Supper passage |  |  |  |  |
| 5.1 | Ordinary soil excavation | m3 | 22.0 | 1600 | 35200.00 |
| 5.1.1 | Stone Pitching | m3 | 9.6 | 353 | 3388.12 |
| 5.1.2 | Back fill | m3 | 10.0 | 42 | 420.00 |
| 5.1.3 | Lean concrete ( C-10) for masonry pad | m³ | 7.2 | 1900 | 13680.00 |
| 5.1.4 | C-20 concrete(1:2:3) | m³ | 2.5 | 3000 | 7488.00 |
| 5.1.5 | Masonry work (30:70 masonry) with 1:3 mortar ratio | m3 | 110.0 | 1600 | 176019.20 |
| 5.1.6 | Plastering of 8mm thickness with 1:3 mortar | m2 | 106.4 | 85 | 9047.40 |
| 5.1.7 | RCC bar Ø10 | kg | 211.1 | 55 | 11612.54 |
| 5.1.8 | RCC bar Ø12 | kg | 329.7 | 55 | 18131.85 |
| 6 | Road crossing |  |  |  |  |
| 6.1.1 | Ordinary soil excavation | m3 | 14.9 | 65 | 965.25 |
| 6.1.2 | Back fill | m3 | 6.8 | 42 | 283.50 |
| 6.1.3 | Lean concrete ( C-10) for masonry pad | m³ | 1.9 | 1900 | 3591.00 |
| 6.1.4 | C-20 concrete(1:2:3) | m³ | 12.6 | 3000 | 37800.00 |
| 6.1.5 | Masonry work (30:70 masonry) with 1:3 mortar ratio | m3 | 102.2 | 1600 | 163558.40 |
| 6.1.6 | Plastering of 8mm thickness with 1:3 mortar | m2 | 107.8 | 150 | 16170.00 |
| 6.1.7 | RCC bar Ø10 | kg | 284.2 | 55 | 15632.97 |
| 6.1.8 | RCC bar Ø12 | kg | 380.9 | 55 | 20948.60 |
|  | Sub total |  |  |  | 6093088.61 |
| 7 | Secondary Canal |  |  |  | 12186177.23 |
| 7.1 | Ordinary soil excavation | m3 | 1134.5 | 65 | 73743.19 |
| 7.1 | fill & compaction | m³ | 166.1 | 80 | 13291.52 |
| 7.1 | Back fill | m³ | 328.8 | 42 | 13807.92 |
| 7.1 | Masonry work (30:70 masonry) with 1:3 mortar ratio | m3 | 765.1 | 1600 | 1224166.84 |
| 7.1 | Plastering of 8mm thickness with 1:3 mortar | m2 | 2537.1 | 150 | 380570.00 |
| 8 | Turnout/ Division box |  |  |  | 0.00 |
| 8.1 | Masonry work (30:70 masonry) with 1:3 mortar ratio | m3 | 17.5 | 1600 | 28032.00 |
| 8.1.1 | Plastering of 8mm thickness with 1:3 mortar | m2 | 101.9 | 150 | 15283.65 |
| 8.1.2 | Stone Pitching | m3 | 31.0 | 0 | 0.00 |
| 9 | All Gate |  |  |  | 0.00 |
| 9.1.1 | Division box | no | 69.0 | 5000 | 345000.00 |
| 10 | Drop |  |  |  | 0.00 |
|  | Ordinary soil excavation | m3 | 0.1 | 65 | 73743.19 |
| 10.1 | Lean concrete ( C-10) for masonry pad | m3 | 3.8 | 1900 | 7232.94 |
| 10.1 | Masonry work (30:70 masonry) with 1:3 mortar ratio | m3 | 0.9 | 1600 | 1474.33 |
| 10.1.2 | Plastering of 8mm thickness with 1:3 mortar | m2 | 2.1 | 150 | 312.28 |
| 10.1.3 | Stone Pitching | m2 | 3.9 | 353 | 1382.76 |
|  | Sub total |  |  |  | 2178040.62 |
| 11 | Tertiary Canals |  |  |  |  |
| 11.1 | Ordinary soil excavation | m3 | 738.5 | 65 | 48001.58 |
| 11.1.1 | Filling | m³ | 5.9 | 80 | 468.66 |
|  | Sub total |  |  |  | 48470.23 |
| 12 | Collector drain Canal |  |  |  |  |
| 12.1 | Ordinary soil excavation | m3 | 707.2 | 65 | 45965.38 |
| 12.1.1 | fill & compaction | m³ | 31.2 | 80 | 2492.35 |
| 12.1.2 | Back fill | m³ | 215.7 | 42 | 9058.56 |
| 12.1.3 | Masonry work (30:70 masonry) with 1:3 mortar ratio | m3 | 217.2 | 1600 | 347478.34 |
| 12.1.4 | Plastering of 8mm thickness with 1:3 mortar | m2 | 1101.6 | 150 | 165241.06 |
|  | Sub total |  |  |  | 570235.69 |
| 13 | Tertiary drain Canals |  |  |  |  |
| 13.1 | Ordinary soil excavation | m3 | 1706.7 | 65 | 110938.56 |
| 13.1 | fill & compaction | m³ | 23.7 | 80 | 1894.27 |
| 14 | Drop |  |  |  | 0.00 |
| 14.1 | Ordinary soil excavation | m3 | 1.3 | 65 | 73743.19 |
| 14.1.1 | Lean concrete ( C-10) for masonry pad | m3 | 5.0 | 1900 | 7232.94 |
| 14.1.2 | Masonry work (30:70 masonry) with 1:3 mortar ratio | m3 | 10.0 | 1600 | 1474.33 |
| 14.1.3 | Plastering of 8mm thickness with 1:3 mortar | m2 | 3.5 | 150 | 312.28 |
| 14.1.4 | Stone Pitching | m2 | 7.5 | 353 | 1382.76 |
|  | Sub total |  |  |  | 196978.32 |
|  | Grand Total Cost |  |  |  | 9086813.48 |
|  | Vat 15% |  |  |  | 1339338.72 |
|  | Grand Total Cost |  |  |  | 10268263.50 |

Table 15-2:  Cost summary

| Components | Costs ETB |
| --- | --- |
| General items | 700,000.00 |
| Headwork | 3,434,336.46 |
| Infrastructure | 9,086,813.48 |
| Total | 13,221,149.94 |
| VAT 15% | 1,983,172.49 |
| Total | 15,204,322.43 |

# CONCLUSION AND RECOMMENDATION

The infrastructure of this project area is designed to irrigate about 100.0 ha of land by taking its supply from the Azuwary-2 diversion weir irrigation project. The project is diverted on right side of the river with one main canal. There are 20 tertiary blocks irrigated with tertiary canals diverted from eight secondary canals.

The method of irrigation of the project area is furrow surface irrigation in which the main and tertiary canals will work continuously whereas the field canals within a tertiary block works in rotation. As the dominant soil type is clay soil, all the length of the main canal system is designed to be masonry.

# SECTION IV: OPERATION AND MAINTENANCE

# OPERATION AND MAINTENANCE

## INTRODUCTION

The proposed SSI project, Azuwary-2 Small Scale Irrigation Project, is located in 03 kebele in Gondewyn woreda in East Gojam Zone Amhara National Regional State (ANRS). It can be accessed along 13 km from wereda town.

The project aim is to introduce a modern and reliable long serving irrigation system that could bring effective water management system without disturbing the existing culture and indigenous knowledge of the beneficiaries. It is the farmer-managed scheme which is model of self-determination and capacity building. Strong irrigation management committee is essential for good performance by avoiding demands of supplies from the government such as material and technical supplies and enabling farmers to rely on their own resources .This reduces water use conflicts, enhances on time and proper irrigation system maintenances and as a result, establishes good irrigation operations.

## Description of irrigation system

The irrigation system described hereof is Azuwary-2 SSIPS. The source of irrigation water is Azuwary-2 stream base flow and the existing traditional intakes and canals area assumed to be replaced by newly constructed, modern and upgraded system. The referred farm system comprises a net command area of 100.0 ha. Irrigation structures both canals and drains are designed suitably at desired locations to ensure uniform conveyance, divert and control the irrigation water on the farms and to dispose of excess irrigation water and runoff from the irrigation field and upstream catchments. The general conveyance systems are masonry section on the main and the earthen canal sections are on the existing field canals.

The existing farm road networks are provided to direct access to the irrigation fields of each farmer without crossing other farmer’s field. Moreover, Cattle crossings are provided at suitable intervals near the canals to give access for cattle to and for the grazing land and water.

## Irrigation management committee

The irrigation management committee is responsible for managing the implemented irrigation system, the physical and financial affairs. In some cases, the committee is supposed to be successful in expanding their duties beyond water management to matters such as supplying input, searching market for their irrigation products. However, the process should be encouraged until the basic operation and maintenance management procedures are well-established and the committee has the confidence of the group.

Management is likely to be more effective when both genders are represented on the farmers committee. Elected members should represent the interest of the groups of farmers organizing and overseeing activities such as water sharing and maintenance.

The duties and the responsibilities of the irrigation management committee should be set out in the document of the rules and regulations. The operation and maintenance manual can be used as a standard format by farmers to draw up by-laws. The committee must have legal standing to allow them to enforce decisions. Assistance from stakeholders and community developers will almost certainly be required to help farmers deal with the issues.

It may be difficult for farmers to include management within their existing workload, and there may be requests for the necessary technical training or skills. Accordingly, the project has been designed with the aim of minimizing operational problems. Prepare and publish a roster of regulation and publicize it in advance so that the framers may plan their sowing and irrigation accordingly.

Framers should be trained to identify warning signs of poor maintenance, such as inequity of supply, water logging, deteriorating structures or general decline in available water. Irrigation management committees must consider the best way to delegate tasks amongst sub-groups so as to make the best use of members' skills. If the committee intended to extend their role beyond water management, sub-groups must be accountable to the elected body. Opportunity for blocks exists as soon as out of rule activities are observed and transactions are not transparent. Moreover, works have to be divided into the committee and the community depending up on nature of individual the work

## OPERATION OF THE SYSTEM

In the peak irrigation period, there were considerable leakages of water. However, the newly constructed diversion and conveyance have been designed upgrading the traditional diversion and conveyance system into a sustainable permanent structure that can safely transfer a maximum discharge of more than 95 lit/sec. The whole system has the following components.

DIVERSTION SYSTEM, INCLUDES

- The diversion structures , they include:

  - Under sluice  and off take gates

  - Under sluice approach guide

  - Trash rack

  - Steel Gates

THE IRRIGATION SYSTEM

  - Irrigable land

  - Canal system

  - Canal structures

### Diversion System

#### Diversion Structure

Under Sluice and Off Take Gates

The headwork is meant to take water from the streams base flow. High floods water entering are not allowed in to the irrigation system. Thus steel gates are provided to serve this purpose. The gates are made of sheet metal having 6mm thick and vertically operated the platform constructed at the top of the  under sluices and off take gates wall.

In the rain fall seasons, all the rain falls are not expected produce peak floods flows. The farmers need to identified the peak flood period and remove the under sluice and close the off take gates. The opening and closing schedules and operations have to be done by the farmers and the committee.

Under Sluice Approach Guide

The under sluice gates and approach water way expected to be silted up in the main rain fall season. A temporary guide canal and removing deposited at the under sluice gates requires that have to be done by the farmers every after the high silt transporting floods are over. This meant, simply to guides the wet season flow towards the off take gates. Divide wall has been adopted for the proper flow of water through the approach canal.

Trash Rake

Fixed trash rack of wider spacing (10cm) and made of 12mm diameter is provided in the vertical opening as a front guard to protect larger floating debris from entering the off take canal. Clearing opening from debris have been carried before canal operation.

#### The Irrigation System

The irrigation systems in the command area of azuwary-2 diversion SSIPS have the following major components used in the water distribution system of the projects.

### Irrigable Land

According to visual observation and laboratory result the dominant soils of the command area are clay soil with moderately to deep soil thickness. Most part of command areas have slope 4-10% which are permissible to surface irrigation methods and properly connected to the existing drains network. There are needs to facilitate the farmers to prepare appropriate irrigation methods. Assistance from stakeholders and community developers will almost certainly be required to help farmers deal with the issues.

There are severe sheet erosion problems in the whole part of the command areas in the scheme. The runoff from surrounding hill catchment passes crossing the lands should be continued to protect soil erosion the command area. The existing upstream of the command gazing land practices shall be change to Zero gazing land. Stream bank shall be provided at least three meter width none cultivated areas to protect the embankment slide and expansion.

### Canal Systems

These are canal structures used to convey irrigation water from the source (head work) to the command area. The canal systems in the command area of AZUWARY-2 SSIPS are dominantly designed to have two levels, the main canal and Field canals. The existing irrigation systems consist of one main traditional canal running from the head work to the command area. The main canal is almost running at the head of command area. The field canals are running parallel to the main and across the contour. In order to arrive at a suitable operational guidance the following operational categories are considered.

- Priming of irrigation canals

- Embankment protection

- Emergency rejection spill ways

- Capacity of irrigation canals and their operation

- Field canals and turn outs

- Irrigation methods (surface irrigation methods)

Priming of Irrigation Canals

It is always necessary to prim field irrigation canals to run at its fully capacity. Priming is sought necessary if the canal is a newly constructed canal or if the canal has not been operational for a prolonged period of time, during which course it has lost considerable amount of its moisture or priming will be necessary during every new season as the farming interval may be.

Priming is the stage by stage wetting of irrigation canals which helps to saturate the body of the canal gradually so that sudden saturation which could result in negative pour pressure within the pore space of the soil mass is avoided. This negative pressure could resulted in the total embankment collapse or could impact subsidence and after that sliding of portion of the canal embankment.

Experience has shown that operating the new canal with its 25 % capacity could be a good starting. Depending on the nature of the soil this discharge is to be run for some time. For the present area a first stage priming time of 6 hours is recommended. There after the discharge may be increased to its 50% amount gradually and then this discharge is allowed to run for about 6 hours. Consequently the discharge is increased gradually until the full supply discharge is passed through the canal.

Embankment Protection

Most farmers are experienced cultivate their land up to canals embankments and consider as the part of their farm lands. Due to this cultivation, the canals may expose to seepage and this may causes of water logging. The embankments are required special care to protect for damage and exposed to damage.  Hence, the embankments are recommended to have at least one meter width and to be serving as means to access the farm land and for operation activities.

Emergency Rejection Spillways

During the course of irrigation it may sometimes be necessary to reject all or some of the water flowing through main and field canals. The reasons for rejection are:

- Requesting more water than required at any required interval of time.

- Delaying the opening of field canals turnout gates

- Not informing gate operators when flow rates should be reduced etc

Capacity of Irrigation Canals and Their Operation

For proper irrigation water management it is highly advisable to run any canal at its design capacity. In this irrigation system field canals are provided which convey water to field. It is proposed to irrigate a irrigators at the same time in the block. The irrigation capacity of the field canal is standardized. To convey more than the capacity of the canals and to serve more than the specified amount of field canals may result in breaching of the canal and inefficient water application respectively.

Irrigation Methods

There are many methods of applying water to the field.  Surface, sprinkler and drip irrigation are the main irrigation methods. Surface irrigation is the application of water to the fields at ground level. In doing so, the entire field is flooded or the water is directed into furrows or borders. Different crops require different methods of irrigation. Among the surface irritation methods; basin, furrow and boarder methods are recommended for the sound cultivation of crops.

- Furrow irrigation

Furrows are narrow ditches dug on the field between the rows of crops. The water runs along them as it moves down the slope of the field. The water flows from the field ditch into the furrows by opening up the bank of the ditch or by means of siphons or spills. Crops like, potato, onion, pepper, maize, etc are irrigated by this method.

- Border irrigation

In border irrigation, the field to be irrigated is divided into strips (also called borders or border strips) by parallel border ridges. The water is released from the field ditch onto the border through gate structures called outlets.  The water can also be released by means of siphons or spills. The sheet of flowing water moves down the slope of the border, guided by the border ridges. Crops like Maize, haricot bean, onion, pepper, etc, are irrigated by this method.

- Basin irrigation

Basins are horizontal; flat plots of land, surrounded by small bunds. The bunds prevent the water from flowing to the surrounding fields. Basin irrigation is commonly used for terraces on hillsides. By this method maize, teff, Rice etc, can be irrigated.

Irrigation water management

Water shortages, quality deterioration and flood impacts are among the problems, which require greater attention in the schemes. Irrigation systems are never fully efficient. Some water is lost in canals and on the farmers' fields. Part of the water seeps into the soil. This requires careful management of the irrigation system.

Improved water management techniques such as proper irrigation scheduling, soil moisture measurement techniques and water measurement should be introduced for the project in order to improve the timing and duration of the water delivery. Hence, continuous, well-organized and site- specific (on the scheme) trainings on water management, irrigation and irrigated crop production should be given for beneficiary farmers of the project.

The total scheme flow is alternatively diverted to blocks. All farmers belonging to one block should be irrigated during the period in which their block receives the total available flow. The field canals of the same capacity should equalize the intake of individual users within a given rotational period. For the scheme, 18 hours per day and 126 hours per week irrigation are proposed to cover the whole command area with the estimated design supply.

Organizational aspect of the diversion irrigation project

Beneficiary farmers of this scheme should be organized into water users association (WUA). To establish a WUA, however, the water users need to be assisted by project owners, state structures, or any other facilitating organization. There is a need to work with the ordinary grassroots level to establish WUA so that they understand what a WUA is and how they will benefit from it.  By water users we mean that the cultivators of land of diversion irrigation projects command area. These are potential members of the WUA, who take responsibility for financial, material, technical and human resources for the operation and maintenance of the irrigation and drainage system within their jurisdiction for the benefit of all the members.

WUA committee is elected by the general assembly of the water users. Dispute settings and input and credit supply subcommittee are also established during the general assembly of the users.

Water users may select appropriate block leaders for representing them in the representative assembly and a group leader that represents to oversee the internal affairs within that specific field blocks.

The farmers, by joining a WUA can have some, or all of the following benefits:

- Equitable water distribution among farmers regardless of their location, type of farm, or size of the farm,

- More reliable water supply,

- Water supply becomes more responsive to crop needs,

- Quick dispute resolution at the local level,

- Well-maintained canals (decreasing the time of irrigation due to less fluctuation of discharges, reduced losses, etc),

- Less water theft

Requests for Water

In order to avoid excessive rejection of irrigation water and malfunction of irrigation canals which again resulting in the damage of embankment and wastage of water all personnel working on the distribution system should give their request for irrigation water release and should give at least 12 hours’ notice of anticipated time for shut-down or reduction of flow. Since individual fields in any unit are at different distance from the main supply canal. Water when released from the main system will take varying time to reach different fields. That is fields near the main supply canal will be served faster than fields located further away.

The person who demands a water release should know the approximate elapsed time before the required discharge reaches the point of application and he should give attention to the fact that when a shutdown is requested that there is the requested amount of discharge in the substantial farm land. Therefore, in order to make use of the backup water and avoid excessive rejection of irrigation water on completion of irrigation or when a reduction in demand is required the person in charge should give at least 6 hours’ notice of the anticipated time for shut-down or reduction in flow.

In order to establish a suitable information exchange system the following communication means are proposed.

    - Verbal communication: - Verbal communication is face to face interaction of farmers.

    - Oral communication:- This kind of communication could be effective when mutual consent and understanding exists between different parties involved, however, for each oral discussion to be held both parties should keep a written record concerning the notified time to release irrigation water and or the anticipated time to shut down or reduce incoming flow, for this purpose the operators should keep a water request diary containing data on release and shut down of conveyances system and records of oral conversations and discussions pertinent to the subject matter of water requirement are valuable documents that should be carefully preserved.

### Farm Structures

In order to secure proper distribution and efficient conveyance of water throughout the irrigation network appropriate numbers of irrigation structures are provided. Depending on the nature of their services the structures are classified in the following groups.

- Conveyance structures

- Regulating structures/rejection structures

Conveyance Structures

These structures are mainly meant to convey irrigation water to the required point of discharge.  Various conveyance structures are provided in the irrigation system which can be categorized in to three types; these include grade control structures such as vertical drop structures, cross drainage works, and distribution structures such as turnout.

Field off Takes

These are structures which are also found in large number and are responsible to abstract water from Main canals and convey to irrigation method supplier. All field off takes are operated with simple sheet metal slide gates and slide gates which are easy to operate and control.

## MAINTENACE

Proper maintenances are an essential part of managing a water system, failures to provide timely maintenance, including making minor repairs before they becomes major , are a primary cause of system failures. For this reason the maintenance works to be undertaken seems complicated since all irrigation and drainage canals with their appurtenant structure are to be maintained when the canals are flowing full.

Depending up on the nature of maintenance work to be carried out the general maintenance activities is classified under the following groups.

- Regular maintenance

- Periodical maintenance

- Incidental maintenance

### Regular Maintenance

Regular Maintenances of Canals

The dry season flows in the irrigation systems do not have much excess sediment load reaching the canal system since the main water sources was base flow. In those cases an additional runoff water source are considered which have high sediment contain. Hence, the new design systems are required de-silting. Naturally, it is possible to reduce the problem not avoid completely. The part concentrated sediment load is disposed along different stretches of the canal system. Due to this fact all irrigation canals have to be de-silted at least twice a year. More over the main canal needs third round of de-silting per year. Before each de-silting of canals, the grass on the banks and side slopes is to be cut and weeds are to be removed. After de-silting, the grass on the banks and side slopes is to be cut and weeds are to be removed. After de-silting, the bed and side slopes of each earth canal must be re-graded. Besides, the abovementioned routine maintenance work regular inspection should be made in each canal and all kind of defects must be maintained right away.

Regular Maintenance of Drainage Canals

All drains have to be de-weeded twice a year around April and October. After the de-weeding operation bed and side slope of drains must be regarded. Except this no especial extra de-silting and grass cutting activity is required.

Regular Maintenance of Drainage Structures

Structures should be inspected regularly and remedial measures should be taken immediately. Within this context the term structure included the upstream stone protecting work, the main structural frame and the downstream stone protection work.

- Upstream stone protection

Practically all irrigation and drainage structures are provided with an upstream dry stone protection work which is placed in open cut over the canal bed and embankment. These protections are meant to facilitate a smooth transition of flow between the approach channel and structures.

Regular visual inspection must be made so that any failure may be maintained immediately. The maintenance work in this kind of stone protection works is suitably done using manual labor.

Structures

Structures must be inspected at least twice a year and repaired if necessary. Frequent de-silting may be necessary since the cross section of the canal immediately upstream of the structures is increased there is relatively dead velocity which initiates the water to drop its silt load. This accumulated silt load should be flushed regularly.

- Downstream stone pitching

The downstream part of a structure is provided with a stone rip-rap protection only. The downstream side of structures must be carefully watched as the final effect of piping and undermining is pronounced at this end. If damage is observed it should immediately be repaired. The under laying graded sand and the rip-rap must be dumped to the pervious thickness.

- Land grading

Land grading or land leveling is the process of improving the surface by earth movement for the purpose of better water control. The surface created by land grading lends itself to a more even distribution of a required amount of water over the irrigated area. This reduces the labor required for irrigation and above all increase the efficient application of water.

Hence areas under cultivation should be inspected in order to identify low spots which are characterized by water logging causing aeration problem in the root zone and restricting growth. The field where low spots are detected during one season should be ploughed and graded before the next season to eliminate these low spots.

### Periodic Maintenance

Periodical maintenance is generally encompassed within regular maintenance work, however the basic difference being periodical maintenance work i.e. effected after regular inspection are made  on structures , canals and other infrastructure within the farm. At this junction it should be born in mind that even though regular maintenance is to be effected after regular periods of time sometimes it is indispensable to maintain and replace damaged and worn out parts right away after periodic inspections, therefore, there should have a maintenance crew specifically concerned with periodic inspection of the system and timely measures should be taken to avoid adverse damage.

### Incidental Maintenance

All maintenance works which are sought necessary due to damages caused by unforeseen circumstance are classified as incidental maintenances.

Incidental maintenances which needs close attention are:

- Maintaining structures and embankments which are damaged as a result of  high flood

- Maintaining structures and embankments which are damaged as a result of  accidents

- Maintaining structures and embankments which are damaged as a result of foundation subsidence.

## CONCLUSIONS AND RECOMMIENDATIONS

Farmers should be required to consider the effect of irrigation development on other activities and identify possible of interest.  They will need:

- Advice and assistance in irrigated production system. Investment must be paid for by adequate returns from cash cropping.

- A reliable source of agriculture in puts, which are basic to viable irrigated production.

- Realistic judgment have been made about the way in which the scheme is to be operated and maintained with very limited management and technical help from government or other stakeholders. This operation and maintenance manual therefore has been made suited to use by relatively untrained farmers.

- Continuing maintenance is required to sustain scheme performance, farmers must understand from the outset the nature, approximate magnitude and timing of necessary work on the system.

- Deposited sediment is major maintenance task on many schemes. Apart from reducing conveyance capacity it can interfere with the proper functioning of division and control structures. Technical solutions applicable to large schemes are probably not economically viable on small schemes. Unless farmers are prepared to assign personnel to shut down head gates prior to, and during floods, they will be needed to clean out the system manually.

- The farmers must set aside money and time for the tasks and develop skills in organization and execution of the work.

- Government will need to assign staff and resources for initial support and training in the necessary tasks.

# REFERENCE

- FAO (1977) guidelines for predicting crop water requirements. No 24, Rome Italy

- Garg Irrigation & Hydraulics structure

- Design of small Canal structures , USBR

- Soft copies of hydraulic structure

- IDD manual

- ESRDF manual