Amhara National Regional State

Water Resources Development Bureau

(BOWIED)

Detail Design

Of

Azuwari  -1 Diversion Small-Scale Irrigation Project

Volume IV: Engineering Design Final Report

February , 2019

Bahir Dar

Client: Bureau of Water Resource Development

(BoWIERD)

Address:

P. O. Box: 88
Telephone: 0528-200853/855
Fax: 251-08-20-65-68/204676/202040

Consultant: Amhara Design & Supervision Works Enterprise

(ADSWE)

Address:

P. O. Box: 1921
Telephone: +251-582-181023/ 180638/181201/181254
Fax: (058) 2180550/ (058) 2180560
E-mail: amhara design@yahoo.com

Bahir Dar, Ethiopia

# FEASIBILITY STUDY & DETAIL DESIGN REPORT STRUCTURE

- Volume I: Watershed Management

- Volume II: Engineering Geology

- Volume III: Irrigation Agronomy

- Volume IV: Engineering Design

- Volume V: Socio Economy

- Volume VI: Environmental and social Impact Assessment

- Volume VII: Financial and Economic Analysis

- Volume VIII: Agricultural Marketing and Agribusiness Study

- Volume IX: Wegelal Micro Watershed  Management

Table of Contents								 Page Nr

FEASIBILITY STUDY & DETAIL DESIGN REPORT STRUCTURE	i

LIST OF FIGURES	v

SALIENT FEATURE	vi

SECTION-I: INTRODUCTION	viii

1	INTRODUCTION	9

1.1	Background	9

1.1.1	Description of the Project Area	9

1.2	Objectives of the Study	11

1.2.1	Major Objective	11

1.2.2	Specific Objectives	11

1.3	Scope of the Study	12

1.4	Methodology	12

SECTION-II:  HYDROLOGY	14

2	HYDROLOGY	15

2.1	Watershed Characteristics	15

2.2	Hydro-Metrological Data Availability	17

2.2.1	Climate	17

2.2.2	Rainfall Data	17

2.2.3	River flow data	17

2.2.4	Upstream & Downstream utilization	17

2.3	Design Flood Analysis	18

2.3.1	Design Rainfall computation	18

2.3.2	Outlier Test	18

2.3.3	Check for variance	19

2.3.4	Peak Discharge Determination	22

2.3.5	Tail Water Depth Computation	26

SECTION-II: HEADWORK DESIGN	31

3	HEADWORK STRUCTURES DESIGN	32

3.1	Headwork Site Selection	32

3.2	River Geomorphology	32

3.2.1	River Bed condition	32

3.2.2	River Bank condition	33

3.3	Sources of construction materials	34

3.3.1	Rock for Masonry and Crushed Coarse Aggregate	34

3.3.2	Fine Aggregates	35

3.3.3	Water	35

3.4	Headwork Type Selection	35

3.4.1	Hydraulic Design of Headwork Structure	36

3.4.2	U/S and D/S HFL Calculation & Determination	37

3.4.3	Hydraulic Jump Calculation	39

3.4.4	Impervious floor	41

3.4.5	Cut off Depth Calculation	41

3.4.6	Stability Analysis of weir	41

Stability Analysis of weir under dynamic load combination	43

3.4.7	Design of Divide wall, Under Sluice, and Canal outlet	45

3.5	Bill of Quantity and Cost Estimation	54

SECTION-III: IRRIGATION AND DRAINAGE SYSTEMS INFRASTRUCTURE	58

4	IRRIGATION AND DRAINAGE SYSTEMS DESIGN	59

4.1	Irrigable Area Description	59

4.1.1	Topography	59

4.1.2	Climate	59

4.1.3	Soil characteristics	59

4.1.4	Existing Irrigation Practices in the Project Area	60

4.1.5	Crop Water Requirement (CWR)	60

4.1.6	Irrigation efficiency (Ep)	61

4.1.7	Irrigation duty	61

4.1.8	Irrigation methods	62

4.2	Irrigation and Drainage System Layout	63

4.2.1	Conveyance System	63

4.3	Design of the Canal System	64

4.3.1	Main Canal	64

4.3.2	Secondary Canals	65

4.3.3	Tertiary canals	66

4.3.4	Field Canals	71

4.4	Drainage Canals	72

4.4.1	Catch drainage canals	72

4.4.2	Tertiary canal drainage canals	72

4.5	Canal Structures Design	72

4.5.1	Design of a typical flume	72

4.5.2	Design of Division box	74

4.5.3	Design of Drop structures	78

4.5.4	Road crossing structure	78

4.6	Irrigation Infrastructure Bill of Quantities and Cost Estimate	80

5	CONCLUSION AND RECOMMENDATION	84

6	OPERATION AND MAINTENANCE	85

6.1	General	85

6.2	Operation of the Head Works	85

6.3	Irrigation System Operation	85

6.4	Maintenance Requirement	86

REFERENCE	87

LIST OF TABLES

Table -2-1: The slope classes and proportion of the water shed	15

Table 2-2: Outlier test analysis	18

Table 2-3: Variability test analysis	19

Table 2-4: Rain fall result for different distribution methods	21

Table 2-5: Determination of Time of Concentration	23

Table 2-6: Runoff analysis	25

Table 2-7: Hydrograph coordinates	25

Table 2-8: Weir Site River Cross section Coordinate Data	27

Table 2-9: Stage discharge analysis	28

Table 2-10: River discharge computation at different stages of flow	30

Table 3-1:River discharge computation at different stages of flow	38

Table 3-2: Post jump computation:	40

Table 3-3: Computation of forces per unit width and momentum	42

Table 3-4: Computation of forces per unit width and momentum	44

Table 3-5: Computation of forces per unit width and momentum	46

Table 3-6: Typical design of masonry retaining wall	50

Table 3-7: Computation of forces per unit width and momentum For Upstream retaining wall	51

Table 3-8: Computation of forces per unit width For Downstream retaining wall	52

Table 3-9:  Summary of Stability analysis of Retaining Walls	53

Table 4-1: Hydraulic Parameters of right main canal	64

Table 4-2: Hydraulic Parameters of left main canal	65

Table 4-3: Hydraulic Parameters of secondary canals	65

Table 4-3: Hydraulic Parameters of tertiary canals	67

Table 4-5: Hydraulic Parameters of Flume and supper passages	73

Table 4-6: Hydraulic parameters of Division Boxes on MC	76

Table 4-7: Hydraulic parameters of Division Boxes on SC	77

Table 4-8: Hydraulic parameters of Drop Structures on SC	78

# LIST OF FIGURES

Figure 1-1: Location map of the project area	10

Figure 2-1: Drainage map of Azuwari-1 watershed	16

Figure 2-2: Complex Hydrograph	26

Figure 2-3: Rating Curve	28

Figure 2-4: River profile	29

Figure 3-1: River bed at the proposed weir site	33

Figure 3-2: River profile	39

Figure 3-3:Load combination of weir under Static  load combination	42

Figure 3-4:Load combination of weir under Dynamic load combination	44

Figure 3-5: Load combination of Retaining walls	50

Figure 4-1: Typical Cross Section of secondary canals	66

Figure 4-2: Typical Cross Section of tertiary canals	66

Figure 4-3: Typical Field Canal X-section	71

Figure 4-4: Typical Cross Section of Catch drain canals	72

Figure 4-5: Typical Division Box plan	75

# SALIENT FEATURE

- Project name: Azuwari-1 diversion weir Irrigation Project

- Name of the stream: Azuwari

- Location of the weir

- North: 1198645

- East: 394216

- Zone: East Gojjam

- Wereda: Goncha Siso Enese

- Average Altitude: 2583.masl

- Hydrology

- Design rainfall: 100.45 mm

- Catchment area: 65.427 Km2

- Longest flow path length 23.21Km

- Design flood: 151.344m3/sec

- Design base flow: 250 lit/se.

- Diversion Weir

- Weir type: Broad crest weir

- Height: 1.9 m

- Gross crest length: 17 m

- Weir crest level: 2589.088m.a.s.l

- U/S HFL: 2591.68 m.a.s.l

- U/S TEL: 2592.104m.a.s.l

- D/s TEL: 2591.02m.a.s.l.

- D/s HFL: 2589.91 m.a.s.l

- Afflux: 1.77 m

- Under sluice

- Sill level: 2587.888 m.a.s.l

- Dimension 1.2*1.2m2

- Discharge amount: 5.50m3/sec

- Outlet

- Sill level: 2588.525m.a.s.l

- Opening dimension: 0.6*0.5 m2

- Discharge capacity: 187.5lit/sec.

Irrigation and drainage systems Infrastructure

- Command area size: 156ha

- Type of soil of the command area is residual silty clay soil

- Design discharge of the main canal = 187.5l/sec

- Irrigation system layout consists of two lined main canal, 5 lined secondary canals and 44 earthen tertiary canals

- Main irrigation structures designed are;

- Gully crossing structures

- Drop and division boxes

- Road crossing structures

Project cost

- General Items cost= 1,464, 456.98 Birr

- Headwork cost = 2,066,119.53 Birr

- Irrigation infrastructure cost = 21,439,442.46 Birr

- Total project cost without VAT =24,970,018.98 Birr

- Cost per hectare  without VAT= 160.064.22Birr

# SECTION-I: INTRODUCTION

# INTRODUCTION

## Background

In Ethiopia, under the prevalent rain-fed agricultural production system, the progressive degradation of the natural resource base, especially in highly vulnerable areas of the highlands coupled with climate variability have aggravated the incidence of poverty and food insecurity. The major source of growth for Ethiopia is still conceived to be the agriculture sector. Hence, this sector has to be insulated from drought shocks through enhanced utilization of the water resource potential of the country, (through development of small-scale irrigation, water harvesting, and on-farm diversification) coupled with strengthened linkages between agriculture and industry (agro-industry), thereby creating a demand for agricultural output. In line with the above, efforts have been made by the government to improve the situation in the country in areas of domestic water supply provision, irrigation, watershed management, etc. The Amhara Water Resources Development Bureau is playing its role in the development of small scale irrigation projects in the region. Accordingly, as part of the water sector development program, the office has initiated the study and design of a small scale irrigation scheme on Azuwari River at 09/ barjano   Kebele and signed an agreement with Amhara Design & Supervision Works Enterprise (ADSWE) for the study and design of the project.

### Description of the Project Area

#### Location

This irrigation project is located mainly at 09/ barjano   Kebele, Goncha Siso Enese Wereda of East Gojjam Zone in the Amhara Region. The proposed irrigation project is to be undertaken on Azuwari River and the headwork structures are specifically located at an altitude of about 2583 masl and geographical coordinates of 1198645N (UTM) and 394216E (UTM).

Figure 1-1: Location map of the project area

#### Accessibility

The project site is located after traveling 15km from Ginde-Woin (woreda town) to the south West direction and it is accessed by vehicles using URAP project road until 13km, but the rest 2km is accessed by foot.

#### Previous Irrigation Practices

There are traditional diversions on the upstream and downstream of this river using different irrigation practices but as the hydrology and Hydrogeology study and respondent farmers indicated, the river has capacity of recharging as it stretches down from the source area of the river. As a result there will not be a marked reduction or fluctuation of water flows both for the already existing and the newly proposed irrigation schemes. The traditional irrigation practices are under taken by individual farmers that use the river flow to the extreme left and right side is with hardship.

## Objectives of the Study

### Major Objective

The project area faces variability of rainfall distribution though the overall rainfall generally suffices the rain-feed agriculture. Accordingly, the rain-feed agriculture needs means of supplementing during distribution failures and further full irrigation is required to maximize the use of the potential land and water resources.

Hence the objective of this project is to contribute a substantial share in the effort to reduce the risk of production decrease due to rainfall variability and increase the productivity of the resource in the project specific area. Specifically, the project is targeted for the following.

- To make sustainable the rain-fed crop production and make extra production in the dry season possible for 156 ha of land through irrigation.

- There is a general consensus that irrigation investments will achieve broader food security and poverty reduction impacts and if efforts are also geared towards up-grading existing traditional farming practices with support to enhance access to input supply, output marketing and extension to facilitate access to information and innovations.

- This objective is to be realized by constructing diversion structures across the Azuwari River and diverting the river flow.

### Specific Objectives

Other benefits that can be expected to appear with the launching of the project are:

- Efficiency of water use improvement;

- Improved local nutrition/food security gains;

- Improved management of scarce natural resources (land and water);

- Resilience against drought risk;

- Rationale for erosion control and watershed management;

- Rationale for the intensification and modernization of small-holder agriculture and rural lifestyles.

The engineering study and design enables the realization of the project by the provision of engineering structures that will allow the appropriate abstraction of the river water for delivery in to the identified irrigation fields of the study area. Hence, this engineering design is specifically targeted to:

- Analyze hydrologic requirements of the project and engineering structures;

- The formulation of sound and stable structure,  with necessary provisions that allow safe, easy and low-maintenance operation in the service life of the project;

- Develop working drawings;

- Estimation of construction costs.

## Scope of the Study

- The irrigation design shall ensure reliability, equity and flexibility of water delivery to farmers. It will aim at reducing conflicts among water users and will lead to lower operation and maintenance costs.

- Updating the existing, if available, computation of the actual evapo-transpiration, crop water requirement, irrigation demand/duty using the existing and recent agronomic, climatologic and soil data using more appropriate methodologies.

- Establish design criteria for irrigations structures to be approved by the client and to be used in the final design stage,

- Design proper irrigation system compatible with local conditions and management capabilities,

- Establish flood protection measures for the command area and canal structures and design the respective drainage system accordingly,

- Planning  and layout of the irrigation system, which include irrigation canals, drainage channels, inspection roads and alignments, canal spacing, canal length, location of structures, and water profiles along canal and drains at specified reaches, which is most economical easily manageable and aligned with topographic feature and geological investigation.

- Determination and estimation of water application conveyance and other losses and irrigation efficiencies and consideration of those parameters in design steps.

- Check and  test hydraulic and  structural designs of main canal considering total demand and the required capacity and the base flow availability,

- Prepare general plans and drawings for all irrigation infrastructure and irrigation systems designs,

## Methodology

In the study and design procedure, Designers used the following steps.

- Specific Site identification:

  - Review of the reconnaissance survey conducted by the Client

  - 50,000 scale top map and GIS information

  - Local farmers interview and discussion

  - Wereda and Zone Agriculture section expertise

  - Previous studies

  - On foot travel along the river channel and farm areas.

- Topographic survey:

  - Surveying the headwork site and the Command area with sufficient radius, using Total station

- Flow estimation

  - Physical observation on flood mark indications and local information about high flood and critical flow condition of the river

  - Analyzing the recorded river flow data and use watershed inputs for further analysis.

  - Base flow estimated during the reconnaissance field visit by floating method.

- Irrigable area identification:

  - Using local information

  - 50,000 Topographic map, and GIS information, GPS to see elevation

The design report is organized in three sections. In Section I the Hydrology study is presented and in Sections II and III the Headwork and Irrigation and Drainage Systems designs are discussed respectively. In Section III, planning and design of the irrigation system after diverting the water using the weir will be dealt. The following are major areas of concern in this part.

- Study and design of the irrigation method to be adopted,

- Study and design of the irrigation system layout and associated structures,

- Design of the different conveyance canals,

- Planning and design of the different irrigation and drainage structures,

- Preparation of the longitudinal profiles of the different irrigation and drainage canals.

# SECTION-II:  HYDROLOGY

# HYDROLOGY

## Watershed Characteristics

The watershed consists of various land forms with all slope classes, from flat to very steep, in excess of 3-8 and 8-15 percent in the more flat and rolling plain areas. The land escape of the watershed is then characterized by flat plain and rolling plain, which constitute 70-18 % of the watershed area.

Table -2-1: The slope classes and proportion of the water shed

| No | Slope Slope Class (%) |  | Area Coverage |  |
| --- | --- | --- | --- | --- |
|  | Description | Class (%) | Hectare (ha) | Proportion (%) |
| 1 | Flat Plain | 0-3% | 149.02 | 1.713589 |
| 2 | Rolling Plain | 3-8% | 893.6 | 10.27555 |
| 3 | Undulating Plain | 8-15% | 2550.49 | 29.32821 |
| 4 | Hill | 15-30% | 4052.06 | 46.59484 |
| 5 | Mountain | 30-50% | 1019.35 | 11.72156 |
|  |  | >50% | 31.85 | 0.366245 |
| Total |  |  | 8696.37 | 100 |

Certain physical properties of watersheds significantly affect the characteristics of the runoff and sediment yield and are of great interest in hydrologic analyses. The rate and volume of runoff, and sediment yield from the watershed have much to do with shape, size, slope and other parameters of the landscape. These suggest that there should be some important relations between basin form and hydrologic performance. If the basin and hydrologic characteristics are to be related, the basin form must also be represented by quantitative descriptors. These parameters can be measured from maps.

The watershed characteristics are analyzed and presented in Table 2 of the Watershed Feasibility Study Report of the same project. In summary:

- Catchment Area  = 65.427 km2

- Stream Length = 23.21Km

- CN(III) = 89.459

(Extracted from the Watershed Study Report of the same project)

At the selected reference point, the area of Azuwari-1catchment is 65.427km2 and consists of a network of tributaries as shown in Figure 2 below.

Azuwari River at the headwork site is characterized by  well-defined channel system and considerable flows. It looks that the gradient of the river is getting low and hence there exists significant deposition of sediment  and cobbles and boulders.

Figure 2-1: Drainage map of Azuwari-1 watershed

## Hydro-Metrological Data Availability

### Climate

Small scale irrigation project designers and planners are faced with lack of good data on the hydrology of the stream/river system that will be their water source and on local weather and climate conditions. Stream gauging stations are virtually non-existent in remote rural areas of Ethiopia; meteorological stations are almost rare. But near the catchment area of this project, there is meteorological station .But, there are no flow data for the river near the project. Therefore, data for the hydro-meteorological analysis is taken from the nearby station. Rainfall & temperature data are considered from Gundeweyn  and mota Meteorological station. In fact, this station is very close to the project area.

As per the data of the station, March – April are identified as high temperature periods whereas December–January are low temperature periods. The mean annual rainfall amount is more than 1678mm (1983 – 2010 e.c data of Gundeweyn and 2011-2013 e.c data of Motta taken) and most of it occurs from June to August.

### Rainfall Data

In order to compute the design flood for the Azuwari-1 Diversion structure, the daily maximum rainfall is collected from Gundeweyn Metrological stations with a record of 27years and Motta Metrological stations with a record of 3years.

### River flow data

The base flow which is measured on 08-09-2010 E.C is 250 l/s. Since this base flow is measured during the dry months of the year, this figure is adopted for design by allowing 25% of the flow for downstream users.

### Upstream & Downstream utilization

Downstream of the proposed site, appreciable need for water is anticipated for locals and cattle provisions. Therefore, at least 25% of the minimum flow has to be released for downstream requirements. Since Azuwari-2 irrigation project is proposed downstream of this project the designer propose to release 25% of the base flow which is 62.5l/s. In addition to this there are other small streams which contribute to flow to the river downstream of Azuwari-1 project. This include: the stream named Shetit which has a flow of 15 l/s, Tumadeley which has a flow of 18l/s and Goma which contribute to a flow of 15l/s. So the study team propose Azuwari-2 River downstream of Azuwari-1 project.

For the sake of planning and design, the outlet for the diversion is designed for a discharge of 187.5l/s for this project and the project is to be developed for 156 ha of land, which is most of the time achievable as the flow for most of the time is significant to support this size of command area.

## Design Flood Analysis

For the design and analysis of structures to be constructed on the Azuwari river, estimation of flood magnitude is an important task. This can be done using different techniques depending on the data available. For this particular case, there are no river flow data and hence the flood estimation is done using the rainfall data and applying SCS Curve Method.

### Design Rainfall computation

Based on the data of 24hr peak rainfall given in Table2-1 the design rainfall, Rf  is computed using 5 methods.

### Outlier Test

Higher Limit, YH=Ymean+Kn*Sy,      Kn = 2.564for 30Years of data.

Lower Limit, YH=Ymean-Kn*Sy,       Kn = 2.564 for 30 Years of data.

Table 2-2: Outlier test analysis

| S.No. | Year | Max. RF | Descending Order | Rank | Logarithmic Value/Yo/ |
| --- | --- | --- | --- | --- | --- |
| 1 | 1983 | 51.8 | 91.6 | 1 | 1.9619 |
| 2 | 1985 | 83.5 | 84.4 | 2 | 1.9263 |
| 3 | 1986 | 57.2 | 83.5 | 3 | 1.9217 |
| 4 | 1987 | 53.7 | 79.3 | 4 | 1.8993 |
| 5 | 1988 | 51 | 73.6 | 5 | 1.8669 |
| 6 | 1989 | 57.5 | 72.8 | 6 | 1.8621 |
| 7 | 1990 | 84.4 | 72.1 | 7 | 1.8579 |
| 8 | 1991 | 79.3 | 70 | 8 | 1.8451 |
| 9 | 1992 | 51.7 | 69.1 | 9 | 1.8395 |
| 10 | 1993 | 68.2 | 68.2 | 10 | 1.8338 |
| 11 | 1994 | 66.5 | 66.5 | 11 | 1.8228 |
| 12 | 1995 | 91.6 | 63.3 | 12 | 1.8014 |
| 13 | 1996 | 63.3 | 62.6 | 13 | 1.7966 |
| 14 | 1997 | 47.3 | 62.5 | 14 | 1.7959 |
| 15 | 1998 | 62.6 | 62 | 15 | 1.7924 |
| 16 | 1999 | 57.4 | 58.3 | 16 | 1.7657 |
| 17 | 2000 | 58.3 | 57.5 | 17 | 1.7597 |
| 18 | 2001 | 53.6 | 57.4 | 18 | 1.7589 |
| 19 | 2002 | 38.9 | 57.2 | 19 | 1.7574 |
| 20 | 2003 | 72.8 | 54.3 | 20 | 1.7348 |
| 21 | 2004 | 62.5 | 53.7 | 21 | 1.7300 |
| 22 | 2005 | 46.4 | 53.6 | 22 | 1.7292 |
| 23 | 2006 | 73.6 | 51.8 | 23 | 1.7143 |
| 24 | 2007 | 54.3 | 51.7 | 24 | 1.7135 |
| 25 | 2008 | 48 | 51 | 25 | 1.7076 |
| 26 | 2009 | 48 | 48 | 26 | 1.6812 |
| 27 | 2010 | 70 | 48 | 27 | 1.6812 |
| 28 | 2011 | 72.1000 | 47.3 | 28 | 1.6749 |
| 29 | 2012 | 62.000 | 46.4 | 29 | 1.6665 |
| 30 | 2013 | 69.1000 | 38.9 | 30 | 1.5899 |
| SUM |  |  | 1856.60 |  | 53.4884 |
| MEAN |  |  | 61.89 |  | 1.7829 |
| STANDARD DEVATION |  |  | 12.68 |  | 0.0880 |
| SKEWNESS COEFICIENT |  |  | 0.520 |  | 0.0745 |

Higher Limit, YH = 2.008688

Lower Limit, YL = 1.5572

Therefore,

Upper limit of rainfall = 10^2.008688 = 102.021mm

Lower Limit of rainfall = 10^1.482 = 36.075mm

Conclusion: no rain fall is higher than the higher outlier and lower than Lower outlier.

### Check for variance

The Variability test is done for 30 years data of 24hr peak rainfall given in  the following table.

Table 2-3: Variability test analysis

| S.No. | Year | Max. RF | Descending Order | Rank | Logarithmic Value/Yo/ |
| --- | --- | --- | --- | --- | --- |
| 1 | 1983 | 51.8 | 91.6 | 1 | 1.9619 |
| 2 | 1985 | 83.5 | 84.4 | 2 | 1.9263 |
| 3 | 1986 | 57.2 | 83.5 | 3 | 1.9217 |
| 4 | 1987 | 53.7 | 79.3 | 4 | 1.8993 |
| 5 | 1988 | 51 | 73.6 | 5 | 1.8669 |
| 6 | 1989 | 57.5 | 72.8 | 6 | 1.8621 |
| 7 | 1990 | 84.4 | 72.1 | 7 | 1.8579 |
| 8 | 1991 | 79.3 | 70 | 8 | 1.8451 |
| 9 | 1992 | 51.7 | 69.1 | 9 | 1.8395 |
| 10 | 1993 | 68.2 | 68.2 | 10 | 1.8338 |
| 11 | 1994 | 66.5 | 66.5 | 11 | 1.8228 |
| 12 | 1995 | 91.6 | 63.3 | 12 | 1.8014 |
| 13 | 1996 | 63.3 | 62.6 | 13 | 1.7966 |
| 14 | 1997 | 47.3 | 62.5 | 14 | 1.7959 |
| 15 | 1998 | 62.6 | 62 | 15 | 1.7924 |
| 16 | 1999 | 57.4 | 58.3 | 16 | 1.7657 |
| 17 | 2000 | 58.3 | 57.5 | 17 | 1.7597 |
| 18 | 2001 | 53.6 | 57.4 | 18 | 1.7589 |
| 19 | 2002 | 38.9 | 57.2 | 19 | 1.7574 |
| 20 | 2003 | 72.8 | 54.3 | 20 | 1.7348 |
| 21 | 2004 | 62.5 | 53.7 | 21 | 1.7300 |
| 22 | 2005 | 46.4 | 53.6 | 22 | 1.7292 |
| 23 | 2006 | 73.6 | 51.8 | 23 | 1.7143 |
| 24 | 2007 | 54.3 | 51.7 | 24 | 1.7135 |
| 25 | 2008 | 48 | 51 | 25 | 1.7076 |
| 26 | 2009 | 48 | 48 | 26 | 1.6812 |
| 27 | 2010 | 70 | 48 | 27 | 1.6812 |
| 28 | 2011 | 72.1000 | 47.3 | 28 | 1.6749 |
| 29 | 2012 | 62.000 | 46.4 | 29 | 1.6665 |
| 30 | 2013 | 69.1000 | 38.9 | 30 | 1.5899 |
| SUM |  |  | 1856.60 |  | 53.4884 |
| MEAN |  |  | 61.89 |  | 1.7829 |
| STANDARD DEVATION |  |  | 12.68 |  | 0.0880 |
| SKEWNESS COEFICIENT |  |  | 0.520 |  | 0.0745 |

For variability the formula used is:

α=δ∩-1N*Mean*100%

Where, δn-1 = Standard deviation =12.68

N = Nr of recorded data =30

Mean = 61.89

α = Standard error

α=12.68√30+61.89*100%=3.739%<10%  Acceptable

Therefore the data shows no variability.

The design rain for Gumble distribution has been selected as the best fit for this study, because it gives nearly the same result as the flood mark discharge when we calculate the design peak flood as compared to other distribution methods, like Normal distribution, Log normal distribution, Gumble EVI, Log-Pearson type III and Pearson Type III distributions. The result is given in table below:

X50=Xmean+Stand deviation*KT, For Normal distribution

X50=Xmean+XStand deviation*KT, For Gumble EVI

Y=Ymean+YStand deviation*KT,    X50 = 10^y,For Log-normal distribution

Y=Ymean+YStand deviation*KT,    X50 = 10^y,For Log-Pearson type III

X50=Xmean+XStand deviation*KT, For Pearson Type III distributions

Table 2-4: Rain fall result for different distribution methods

| Rank | XI | Normal | Log Pearson Type III | Log Normal | Pearson Type III | Gumbel EVI | Gumbel |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | XI -'XI' | XI -'XI' | XI -'XI' | XI -'XI' | XI -'XI' | XI -'XI' |
| 1 | 91.60 | 6.273 | 3.325 | 3.345 | 5.827 | 1.637 | 3.897 |
| 2 | 84.40 | 3.266 | 1.859 | 1.869 | 3.028 | 1.455 | 5.623 |
| 3 | 83.50 | 5.128 | 4.530 | 4.536 | 5.005 | 4.733 | 8.339 |
| 4 | 79.30 | 3.075 | 2.998 | 3.000 | 3.028 | 3.553 | 14.168 |
| 5 | 73.60 | 0.826 | 0.536 | 0.536 | 0.818 | 0.240 | 21.430 |
| 6 | 72.80 | 0.049 | 0.511 | 0.510 | 0.002 | 1.430 | 23.793 |
| Sum |  | 18.617 | 13.760 | 13.796 | 17.708 | 13.049 | 77.251 |
| Sum/Mean |  | 0.301 | 0.222 | 0.223 | 0.286 | 0.211 | 1.248 |
| Point Rainfall |  | 87.93 | 92.75 | 92.00 | 88.43 | 94.75 | 100.45 |
| Design Point Rainfall= |  |  | 100.448 |  |  |  |  |

The design rainfall using Gumbel Method is given as: Y=Ymean+YStand deviation*KT,    X50 = 10^y

Where X50  = Design rainfall

Ymean = average of all values of annual heaviest fall = 1.11 mm

YStand deviation =  standard deviation of the series = 0.53 mm

yt	=-LN(LN(T / (T - 1)))

=-LN(LN(50 / (50 - 1))) =3.9

frequency factor , KT=(Yt - Ys) / Sn=3.04

Y=Xmean + Z*XStandared deviation

=61.89+3.04*12.68

=100.448mm

- Point Design Rainfall = 100.448mm

The design rainfall at points for 50 years return period is 100.448 mm and  the areal design rainfall is calculated in the following section.

### Peak Discharge Determination

#### General

The Azuwari River is not gauged river. The design flood is calculated by using SCS unit hydrograph method. Thus, it is preferred to base the flood analysis on rainfall data, which are better both in quantity and quality of data. In the hydrologic analysis for drainage structures, it must be recognized that there are many variable factors that affect floods. Some of the factors that need be recognized and considered on an individual site by site basis are; rainfall amount and storm distribution; catchment area, shape and orientation; ground cover; type of soil; slopes of terrain and stream(S); antecedent moisture condition; Storage potential (over bank, ponds, wetlands, reservoirs, channel, etc.)

#### Peak flood analysis by SCS unit hydrograph method

Design flood is calculated SCS (The United States Soil Conservation Service). This method is widely adopted and  more reliable method for flood estimation. The approach considers, watershed parameters, like Area, Curve number, and time of concentration.

#### Time of concentration (Tc)

Time of concentration has been calculated by taking the stream profile of the longest streamline and dividing it in to different elevation. Kirpich formula is adopted for computation.

Table 2-5: Determination of Time of Concentration

| class | length km | Elevation,msl | elevation deference meter | Tc=0.948*(L^3/H) ^0.385 hr | TC minute |
| --- | --- | --- | --- | --- | --- |
| 1 | 0 | 3960 | 0 | 0.00 | 0 |
| 2 | 3.66 | 3410 | 550 | 0.37 | 22.42542 |
| 3 | 11.96 | 2750 | 660 | 1.37 | 82.07572 |
| 4 | 7.59 | 2582 | 168 | 1.37 | 82.20434 |
| total | 23.21 |  |  | 3.11 | 186.71 |

The formula is,

Tc=0.948L13H10.385+L23H20.385+…+Ln3Hn0.385

- Tc = 3.11	Since Tc >3hr., duration of excess rainfall difference, D = 1hr.

- Time to peak,

Tp=D2+0.6*Tc  = 2.367hr

- Base time,

Tb=2.67*Tp  = 6.32hr

- Recession time,

Tr=1.67*Tp  =3.95 hr.

#### Curve number (CN)

Curve number (CN) is achieved based on USSCS method by watershed characterization in terms of land cover, treatment, hydrologic condition and soil group. From the watershed analysis curve number at condition II =77.1 Since peak rainfall is found at an antecedent moisture condition III state, this value has to be changed to antecedent moisture condition III.

- Conversion factor = 1.1603

- CN Condition (III) = (Factor from Table x CN condition II) =77.1*1.1603 = 89.459. For detail analysis of the computation, Refer Excel file, attached here with. Head Work\Azuwari-1 design calculation sheet (Autosaved) (Autosaved).xls

#### Area Rainfall

As the area of the catchment gets larger, coincidence of all hydrological incidences becomes less and less. This can be optimized by changing the calculated point rainfall to aerial rainfall. The conversion factor is taken from standard table that relate directly with the size of watershed area and type of the gauging station. (IDD manual)

For the case of Azuwari-1  irrigation project,

- Total watershed area = 65.427 Km2

- Type of gauging station = Daily rainfall (24 hr.)

- Aerial Rainfall = (Point Rainfall) x (Conversion factor)

#### Run off Analysis

Input data:

Design Point Rainfall = 100.448mm

Curve number at antecedent moisture condition III = 89.459

Catchment Area, A = 65.427  Km2

Tc = 3.11hr, D = 1hr., Tp = 2.367 hr; Tb = 6.32hr; Tr = 3.95hr.

Direct run-off,

- Q=(I-0.2*S)2(I+0.8*S)

- Where, I = Rearranged cumulative run-off depth (mm

S = Maximum run off potential difference,

- S=25400CN-254

Peak run-off for incremental;

- Qp=0.21*(A*Q)Tp

- Where,   A = Catchment area = 65.427Km2

- Tp = Time to peak (hr)

- Q = Incremental run-off (mm)

Table 2-6: Runoff analysis

| Duration | Hr | 1 | 2.0 | 3 | 4.0 | 5.00 | 6 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Daily point R.F | Mm | 86.4932 |  |  |  |  |  |
| R.F profile | % | 45 | 59.0 | 68 | 72.0 | 75.00 | 79 |
| R.F profile | Mm | 38.922 | 51.0 | 58.8154 | 62.3 | 64.87 | 68.32967 |
| Areal of point R.F Ratio | % | 68.5316 | 76.8 | 80.1487 | 82.8 | 84.38 | 84.53165 |
| Areal of point R.F Ratio | Mm | 26.6739 | 39.2 | 47.1398 | 51.5 | 54.74 | 57.76019 |
| Incremental R.F | Mm | 26.6739 | 12.5 | 7.96543 | 4.4 | 3.20 | 3.021052 |
| Desending order | No | 1 | 2.0 | 3 | 4.0 | 5.00 | 6 |
| Rearraged order | No | 6 | 4.0 | 3 | 1.0 | 2.00 | 5 |
| Rearraged Incremental R.F | Mm | 3.02105 | 4.4 | 7.96543 | 26.7 | 12.50 | 3.196609 |
| Cumulative R.F | Mm | 3.02105 | 7.4 | 15.3892 | 42.1 | 54.56 | 57.76019 |
| Time of incremental Hydrograph | Beginning | 1 | 2.0 | 3 | 4.0 | 5.00 | 6 |
|  | Peak | 2.36705 | 3.4 | 4.36705 | 5.4 | 6.37 | 7.367055 |
|  | End | 6.32004 | 7.3 | 8.32004 | 9.3 | 10.32 | 11.32004 |

Table 2-7: Hydrograph coordinates

| HYDROGRAPH TIME | H1 | H2 | H3 | H4 | H5 | H6 | HT |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 0.00 | 0.00 |  |  |  |  |  | 0.00 |
| 1.00 | 0.00 | 0.00 |  |  |  |  | 0.00 |
| 2.00 | 0.00 | 0.16 | 0.00 |  |  |  | 0.16 |
| 2.37 | 0.00 | 0.22 | 1.96 |  |  |  | 2.19 |
| 3.00 | 0.00 | 0.32 | 5.35 | 0.00 |  |  | 5.67 |
| 3.37 | 0.00 | 0.38 | 7.32 | 15.73 |  |  | 23.42 |
| 4.00 | 0.00 | 0.32 | 10.70 | 42.84 | 0.00 |  | 53.87 |
| 4.37 | 0.00 | 0.29 | 12.67 | 58.57 | 9.31 |  | 80.83 |
| 5.00 | 0.00 | 0.22 | 10.64 | 85.69 | 25.36 | 0.00 | 121.90 |
| 5.37 | 0.00 | 0.19 | 9.46 | 101.41 | 34.66 | 2.48 | 148.20 |
| 6.32 | 0.00 | 0.10 | 6.41 | 76.96 | 58.83 | 8.90 | 151.20 |
| 6.37 |  | 0.09 | 6.26 | 75.76 | 60.02 | 9.22 | 151.3443 |
| 7.32 |  | 0.00 | 3.20 | 51.31 | 45.55 | 15.65 | 115.71 |
| 7.37 |  |  | 3.05 | 50.10 | 44.83 | 15.96 | 113.95 |
| 8.32 |  |  | 0.00 | 25.65 | 30.37 | 12.12 | 68.14 |
| 9.32 |  |  |  | 0.00 | 15.18 | 8.08 | 23.26 |
| 10.32 |  |  |  |  | 0.00 | 4.04 | 4.04 |
| 11.32 |  |  |  |  |  | 0.00 | 0.00 |

- From the analysis, the 50 year return period design run off is 151.3443 m 3/s

Figure 2-2: Complex Hydrograph

### Tail Water Depth Computation

Tail water depth of the river is equal to the flood depth and amount at the proposed weir site before construction of the weir. It is used to cross check peak flood estimated by the SCS unit hydrograph method with flood mark method and to see the flood feature after the hydraulic jump. During field visit, the flood mark of the river at the proposed diversion site was marked based on dwellers information and physical indicative marks. The river cross-section was surveyed.

Table 2-8: Weir Site River Cross section Coordinate Data

| River cross section |  |  |  |  |
| --- | --- | --- | --- | --- |
| Y | X | Elevation | Par.dis | Cum.dis |
| 1198640 | 394187.7 | 2593.177 | 0 | 0 |
| 1198640 | 394189.7 | 2592.95 | 2.02249 | 2.02249 |
| 1198640 | 394191.7 | 2592.723 | 2.0225 | 4.04499 |
| 1198640 | 394193.7 | 2592.496 | 2.02249 | 6.06748 |
| 1198641 | 394195.7 | 2592.241 | 2.0226 | 8.09008 |
| 1198641 | 394197.8 | 2591.973 | 2.02249 | 10.11257 |
| 1198641 | 394199.8 | 2591.711 | 2.0225 | 12.13507 |
| 1198641 | 394201.8 | 2591.471 | 2.0225 | 14.15757 |
| 1198641 | 394203.8 | 2591.231 | 2.022589 | 16.18016 |
| 1198642 | 394205.8 | 2589.873 | 2.0225 | 18.20266 |
| 1198642 | 394207.8 | 2588.148 | 2.02249 | 20.22515 |
| 1198642 | 394208.9 | 2587.188 | 1.11613 | 21.34128 |
| 1198642 | 394209.8 | 2587.203 | 0.922147 | 22.26343 |
| 1198642 | 394211.8 | 2587.246 | 2.0225 | 24.28593 |
| 1198642 | 394213.9 | 2587.34 | 2.022589 | 26.30852 |
| 1198642 | 394213.9 | 2587.34 | 0 | 26.30852 |
| 1198643 | 394215.9 | 2587.6 | 2.0225 | 28.33102 |
| 1198643 | 394217.9 | 2587.86 | 2.02249 | 30.35351 |
| 1198643 | 394219.9 | 2589.109 | 2.0225 | 32.37601 |
| 1198643 | 394221.9 | 2590.188 | 2.0226 | 34.39861 |
| 1198644 | 394223.9 | 2590.732 | 2.02249 | 36.4211 |
| 1198644 | 394225.9 | 2591.022 | 2.0225 | 38.4436 |
| 1198644 | 394227.9 | 2591.131 | 2.02249 | 40.46609 |
| 1198644 | 394229.9 | 2591.239 | 2.0226 | 42.48869 |
| 1198644 | 394232 | 2591.328 | 2.0225 | 44.51119 |
| 1198645 | 394234 | 2591.632 | 2.02249 | 46.53368 |
| 1198645 | 394236 | 2592.113 | 2.0225 | 48.55618 |
| 1198645 | 394238 | 2592.595 | 2.022589 | 50.57877 |
| 1198645 | 394240 | 2593.076 | 2.0225 | 52.60127 |
| 1198645 | 394242 | 2593.557 | 2.0225 | 54.62377 |

Table 2-9: Stage discharge analysis

| S/N | Elevation | Depth | Arae (m2) | Perimeter (m) | Hydraulic Radius (m) | Velocity (m/s) | Discharge (m3/s) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1.00 | 2587.19 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.0 |
| 2.00 | 2587.39 | 0.20 | 0.73 | 5.65 | 0.13 | 0.87 | 0.6 |
| 3.00 | 2587.59 | 0.40 | 2.03 | 7.53 | 0.27 | 1.42 | 2.9 |
| 4.00 | 2587.79 | 0.60 | 3.68 | 9.41 | 0.39 | 1.83 | 6.7 |
| 5.00 | 2587.99 | 0.80 | 5.64 | 10.52 | 0.54 | 2.25 | 12.7 |
| 6.00 | 2588.19 | 1.00 | 7.72 | 11.21 | 0.69 | 2.66 | 20.6 |
| 7.00 | 2588.39 | 1.20 | 9.92 | 11.90 | 0.83 | 3.03 | 30.0 |
| 8.00 | 2588.59 | 1.40 | 12.23 | 12.59 | 0.97 | 3.35 | 41.0 |
| 9.00 | 2588.79 | 1.60 | 14.65 | 13.27 | 1.10 | 3.65 | 53.4 |
| 10.00 | 2588.99 | 1.80 | 17.18 | 13.96 | 1.23 | 3.92 | 67.4 |
| 11.00 | 2589.19 | 2.00 | 19.83 | 14.67 | 1.35 | 4.17 | 82.8 |
| 12.00 | 2589.39 | 2.20 | 22.59 | 15.40 | 1.47 | 4.41 | 99.6 |
| 13.00 | 2589.59 | 2.40 | 25.48 | 16.14 | 1.58 | 4.63 | 118.0 |
| 14.00 | 2589.79 | 2.60 | 28.48 | 16.87 | 1.69 | 4.84 | 137.9 |
| **** | 2589.91 | 2.73 | 30.44 | 17.34 | 1.76 | 4.97 | 151.344 |
| 15.00 | 2589.99 | 2.80 | 31.62 | 17.63 | 1.79 | 5.04 | 159.4 |
| 16 | 2590.01 | 2.82 | 31.96 | 17.71 | 1.80 | 5.06 | 161.8 |

Figure 2-3: Rating Curve

From the above stage discharge table and curve the maximum flood level corresponding to the computed design peak discharge is 2589.91(2.73 m from the river bed) and it is considered as the d/s high flood level i.e. expected at the weir axis before construction of the weir.

D/S HFL = 2589.91masl.

- Average river bed slope

Average river bed slope of River is estimated by two different techniques. One is by end area  method and the other is by using best fit line method. Designers have adopted the best fit line method output for further analysis.

The water level of the river is taken at different points along the river channel around the head work site. Surveying work done for 174m length. And then, average water surface slope is considered as the river bed slope. For comparison of the two procedures, refer the attached Excel file.Head Work\Azuwari-1 design calculation sheet (Autosaved) (Autosaved).xls

Figure 2-4: River profile

- Manning’s Roughness coefficient

The Manning’s roughness coefficient is taken from standard table based on the river nature. The river at the headwork site has winding nature. The river banks are defined and relatively smooth. Manning’s roughness coefficient (n = 0.035) is adopted.

- Discharge of the river

Input data:

- Manning's roughness coefficient, n = 0.035

- Average river bed slope, S = 0.0143

- ,  	Where, R = Hydraulic radius = (Area/Perimeter)

- Q=V*A

Table 2-10: River discharge computation at different stages of flow

| S/N | Elevation | Depth | Arae (m2) | Perimeter (m) | Hydraulic Radius (m) | Velocity (m/s) | Discharge (m3/s) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1.00 | 2587.19 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.0 |
| 2.00 | 2587.39 | 0.20 | 0.73 | 5.65 | 0.13 | 0.87 | 0.6 |
| 3.00 | 2587.59 | 0.40 | 2.03 | 7.53 | 0.27 | 1.42 | 2.9 |
| 4.00 | 2587.79 | 0.60 | 3.68 | 9.41 | 0.39 | 1.83 | 6.7 |
| 5.00 | 2587.99 | 0.80 | 5.64 | 10.52 | 0.54 | 2.25 | 12.7 |
| 6.00 | 2588.19 | 1.00 | 7.72 | 11.21 | 0.69 | 2.66 | 20.6 |
| 7.00 | 2588.39 | 1.20 | 9.92 | 11.90 | 0.83 | 3.03 | 30.0 |
| 8.00 | 2588.59 | 1.40 | 12.23 | 12.59 | 0.97 | 3.35 | 41.0 |
| 9.00 | 2588.79 | 1.60 | 14.65 | 13.27 | 1.10 | 3.65 | 53.4 |
| 10.00 | 2588.99 | 1.80 | 17.18 | 13.96 | 1.23 | 3.92 | 67.4 |
| 11.00 | 2589.19 | 2.00 | 19.83 | 14.67 | 1.35 | 4.17 | 82.8 |
| 12.00 | 2589.39 | 2.20 | 22.59 | 15.40 | 1.47 | 4.41 | 99.6 |
| 13.00 | 2589.59 | 2.40 | 25.48 | 16.14 | 1.58 | 4.63 | 118.0 |
| 14.00 | 2589.79 | 2.60 | 28.48 | 16.87 | 1.69 | 4.84 | 137.9 |
| **** | 2589.91 | 2.73 | 30.44 | 17.34 | 1.76 | 4.97 | 151.344 |
| 15.00 | 2589.99 | 2.80 | 31.62 | 17.63 | 1.79 | 5.04 | 159.4 |
| 16 | 2590.01 | 2.82 | 31.96 | 17.71 | 1.80 | 5.06 | 161.8 |

# SECTION-II: HEADWORK DESIGN

# HEADWORK STRUCTURES DESIGN

## Headwork Site Selection

The headwork site is situated at 1198645m N, 394216m E and river bed elevation of 2587.188m above sea level. At this site the river course is well defined, matured with fixed width and forms nearly a U-shaped valley. This specific site is covered by massive rock of aphanitic basalt which is characterized by very widely spaced joints at the river bed where as the left  and right bank is low plastic residual soils which is low plastic clay soil. The river is flowing through a defined channel. The different sections of the stream at the proposed headwork site are described separately below.

## River Geomorphology

It is a common fact that the river development tends to accommodate itself to the local geology that develops along the structurally weak zones like faults, joints, folds, etc. The drainage system of the study area is strongly influenced by geological structures and formations, the nature of the vegetation cover and climate. The nature of geological formations and structures has also strong influence on the development of the channel.

The present morphology of the Azuwari River channel is a function of a number of processes and environmental conditions, including the composition of the bed and the banks (massive rock of aphanitic basalt, low plastic residual soils which is low plastic clay soil); the size and composition of the sediment moving through the channel; the rate of sediment transport through the channel and deposition on the banks and beds; and the regional degradation due to erosion processes. Both left and right banks composed low plastic residual soils which is low plastic clay soil has reddish brown color lateritic clay soil and at the top silty clay soil which is low plastic clay and has medium to low permeability. As the result the stream shows highly meandering nature both up and downstream from the proposed site but at the particular weir site it shows nearly straight river channel.

### River Bed condition

The stream bed or course at the proposed headwork site is nearly straight, well defined, and shows almost uniform surface and straight rich from one bank to another bank. It is made up of massive rock of aphanitic basalt. This aphanitic basalt extends in both up and downstream direction from the proposed weir site.  Since the river bed geological condition is very good, its bearing capacity is high and no seepage problem, the designer provide nominal apron thickness and length and cut off depth. So, remove 20cm of the top weathered rock and construct the weir and provide 1.0m and 1.0m depth cut off upstream and downstream respectively.

Figure 3-1: River bed at the proposed weir site

### River Bank condition

#### Right Bank

At the proposed headwork axis and immediate vicinities, the Right bank forms vertical or very steep slope topography. The formation and stratification of the right bank is the same with the left bank from surface observation, the bank is made up of one basic geological unit. The unit is composed massive rock of aphanitic basalt at the bottom and  low plastic residual soils which is low plastic clay soil has reddish brown color lateritic clay soil  at the middle and at the top silty clay soil which is low plastic clay and has medium to low permeability. The slope and peak areas of the bank are covered by silty clay soil which is low plastic clay and has medium to low permeability.

These three units have variable thickness/depth at the area. Just at the diversion axis, the top clay soil has about 2.0m, whereas the middle and  low plastic residual soils which is low plastic clay soil has reddish brown color lateritic clay soil possesses 0.5m to 1m thickness and the underlying bedrock is at the bank foot. These overburden soil materials have been affected by erosion/ flood under cutting which is widening the bank by forming nearly vertical slope. Such vertical slope configuration observed at upstream and downstream bank part forms instability or collapse. Here, it is important to design the bank slope to stable configuration, just by providing appropriate slope (indifferent from the present vertical slope).

#### Left Bank

The formation and stratification of the left bank is the same with the right bank from surface observation, the bank is made up of one basic geological unit. The unit is composed massive rock of aphanitic basalt at the bottom and  low plastic residual soils which is low plastic clay soil has reddish brown color lateritic clay soil  at the middle and at the top silty clay soil which is low plastic clay and has medium to low permeability. The slope and peak areas of the bank are covered by silty clay soil which is low plastic clay and has medium to low permeability.

These three units have variable thickness/depth at the area. Just at the diversion axis, the top clay soil has about 2.0m, whereas the middle and  low plastic residual soils which is low plastic clay soil has reddish brown color lateritic clay soil possesses 0.5m to 1m thickness and the underlying bedrock is at the bank foot. These overburden soil materials have been affected by erosion/ flood under cutting which is widening the bank by forming nearly vertical slope. Such vertical slope configuration observed at upstream and downstream bank part forms instability or collapse. Here, it is important to design the bank slope to stable configuration, just by providing appropriate slope (indifferent from the present vertical slope).

## Sources of construction materials

During site investigation, natural construction materials required for the construction of the various proposed engineering structures at the headwork and within the farmland have been assessed, and possible quarry sites and borrow areas have been identified within the vicinity of the study area as close to the project site as possible. In addition to identifying the quality, quantity and accessibility conditions of the construction materials, ownerships of each proposed production sites have also been studied and described in this report, on separate sub-sections below. The materials needed for the construction of the structures include rock for masonry stones, aggregates (both coarse and fine), and water.

### Rock for Masonry and Crushed Coarse Aggregate

The rock for masonry and coarse aggregates for this particular project is identified at the vicinity of this project area. It is found within kebele 08 in the locality called Aqabit. It is found east of Arjano town. It is located at 5km distance from headwork site. From this 3km is distance is accessible through all weathered URRAP road and the rest 2km is inaccessible. The quantity and quality of this material has also been estimated and evaluated. The quantity of the rock is estimated to 100Mx50mx10m=50,000cubic meter. Moreover, the rock is made up of fine grained fresh aphanitic basalt. The workability or method of excavation will done either man power or packet excavator because the material is columnar jointed and can easily produced. So that its quantity is excellent.  The selected rock quarry site is existing quarry and it belongs to the private licensed ownership known as Geta Birara. The GPS point location of rock quarry site is;

X=396620, Y=1198034, Z= 2691

### Fine Aggregates

The sand for Azuari1 is identified and inspected at Feres Mada River. It is found within Enebsie Sir Mider woreda in kebele 07 at the locality known as Gunaguna. It is accessible 46km through asphalt road and 28km through all weathered gravel road and 2km inaccessible (from Michael church to head work). Therefore, the distance of sand quarry from project site is 66kms. The sand is volcanic rock derived and well graded which is suitable for construction. The quantity of the sand within this stream is more than the need of project. As information gathered from local miners, more than 50,000cubic meter/3125 damp trucks of sand had been mined and transported per year from this stream quarry.

### Water

Water for construction purposes can be found from the project stream, Azuwari, itself. The stream is perennial throughout the year that there is some amount of flow along its course. During this field time the stream flow was 250L/second.

## Headwork Type Selection

Looking the availability of natural construction materials and considering the river features and expected flood amount, broad crest type of weir is chosen. As it is:

- Simple for construction

- Weir section is expected only 50% of the peak flood while the remaining flood will pass over the overflow section of the river course.

- There is no significant bed load (boulder effect) in the river.

### Hydraulic Design of Headwork Structure

#### Weir Height Determination

The following major factors have been seen in determining the weir crest level:

- Maximum  command area elevation

- Deriving head of the intake structure

- Main canal slope

- Loss

- Lowest Point of river center

Minimum RBL =                                                                                                                   2587.188

Bottom of intake level=Max. command level +Hl along MC=                                        2588.525

Intake level above avg. RBL=                                                                                            1.337

Water depth in MC                                                                                                            0.38

Length of MC from head regulator to the 1st off-take                                                        260

Bed slope of the main canal,S                                                     	0.00125

Head loss along MC, hf=(1/800*260)	0.325

Head loss at the head regulator, hg=	0.1

Operational head losses(Head losss the field,loss at the turn out)	0.1

Peak irrigable land level in the command area	2588

Water surface level at 1st off-take down the main canal	2588.5

Therefore, Weir Crest Level =	2589.025

Calculated Weir Height = Weir Crest Level- Minimum RBL	1.90m

#### Base flow of the River

The study team has assessed that the stream is not used for irrigation along its entire course except at some places along the river reach where farmers are using the stream for traditional SSI (Irrigation Infrastructure Report). Study team has calculated flow of the river at the weir site as 250 l/s.  Out of this 187.5 l/s will be required for the proposed scheme and the rest will be released for downstream. The purpose of releasing the 62.5l/s to downstream is for the sake of downstream users.

#### Weir/intake Dimensions

##### Flow over the Weir crest

a. Crest Length

- Lacey’s regime width, L=4.75*Q,  =4.75*151.344 = 58.44m.

- Actual river section width of the over flow section of the river is =17m

b. Discharge over the weir section

- Design discharge, Q = 151.344 m3/s

##### Top and bottom width

According to the Beligh’s formula, top and bottom width of the weir body is determined as follows

- Input Data:

P: Height of weir (m) = 1.9

He: specific energy head (over flow depth + approaching velocity head (m))

: Specific weight of weir body (22 for massonry)

Top width, 0.66m ----say 1.0

Bottom width,   1.1m---2.9

Provide 1.0 m and 2.9 m top and bottom width respectively, which will be tested for adequacy during stability analysis.

### U/S and D/S HFL Calculation & Determination

From the stage –discharge curve prepared ( Stage discharge curve, in Hydrology Part) the high flood level before construction (i.e. D/s HFL) corresponding to the design flood is 2589.913 ma.s.l.

D/s HFL = 2589.913 amsl ------------------------------------- (a)

U/s HFL = U/s bed level + weir height + Hd -------- (b)

Hd is the depth of water over the weir crest. This is calculated by assuming broad crested weir formula.

Q=C*L*He32

He=QC*L23 = 3.016m

The velocity head, ha is computed from the approach velocity as shown below

Where g: acceleration due to gravity = 9.81m/sec2

Va is Approach velocity determined by

L is Weir crest length = 12m,

hd is flow depth over the weir and also,

ha=He-hd=(Q/(L*hd))^2/2*g

=(151.344/(17*hd))^2/2*9.81=ZZZ/hd^2

| Q(m3/sec) | He(m) | q (m^3/s/m) | Hd (m) | Ha(m) | U/S HFL(m) | U/s TEL(m) | V(m/s) | TWD(m) | D/S HFL(m) | D/s TEL(m) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 2589.09 | 2589.09 | 0.00 | 0.00 | 2587.19 | 2587.19 |  |
| 0.64 | 0.08 | 0.04 | 0.08 | 0.00 | 2589.17 | 2589.17 | 0.87 | 0.20 | 2587.39 | 2589.15 |  |
| 2.88 | 0.22 | 0.17 | 0.21 | 0.00 | 2589.30 | 2589.30 | 1.42 | 0.40 | 2587.59 | 2589.24 |  |
| 6.71 | 0.38 | 0.39 | 0.37 | 0.01 | 2589.46 | 2589.47 | 1.83 | 0.60 | 2587.79 | 2589.35 |  |
| 12.69 | 0.58 | 0.75 | 0.55 | 0.03 | 2589.64 | 2589.67 | 2.25 | 0.80 | 2587.99 | 2589.47 |  |
| 20.57 | 0.80 | 1.21 | 0.75 | 0.05 | 2589.84 | 2589.89 | 2.66 | 1.00 | 2588.19 | 2589.60 |  |
| 30.00 | 1.03 | 1.76 | 0.95 | 0.08 | 2590.04 | 2590.11 | 3.03 | 1.20 | 2588.39 | 2589.74 |  |
| 40.96 | 1.26 | 2.41 | 1.15 | 0.11 | 2590.24 | 2590.35 | 3.35 | 1.40 | 2588.59 | 2589.89 |  |
| 53.42 | 1.51 | 3.14 | 1.36 | 0.15 | 2590.45 | 2590.59 | 3.65 | 1.60 | 2588.79 | 2590.05 |  |
| 67.37 | 1.76 | 3.96 | 1.57 | 0.19 | 2590.66 | 2590.85 | 3.92 | 1.80 | 2588.99 | 2590.21 |  |
| 82.76 | 2.02 | 4.87 | 1.79 | 0.23 | 2590.87 | 2591.10 | 4.17 | 2.00 | 2589.19 | 2590.37 |  |
| 99.58 | 2.28 | 5.86 | 2.00 | 0.28 | 2591.09 | 2591.37 | 4.41 | 2.20 | 2589.39 | 2590.54 |  |
| 117.97 | 2.55 | 6.94 | 2.22 | 0.33 | 2591.31 | 2591.64 | 4.63 | 2.40 | 2589.59 | 2590.71 |  |
| 137.94 | 2.83 | 8.11 | 2.45 | 0.39 | 2591.54 | 2591.92 | 4.84 | 2.60 | 2589.79 | 2590.88 |  |
| 151.34 | 3.02 | 8.90 | 2.59 | 0.42 | 2591.68 | 2592.10 | 4.97 | 2.73 | 2589.91 | 2590.99 |  |
| 159.37 | 3.12 | 9.37 | 2.68 | 0.44 | 2591.77 | 2592.21 | 5.04 | 2.80 | 2589.99 | 2591.06 |  |
| 161.79 | 3.15 | 9.52 | 2.70 | 0.45 | 2591.79 | 2592.24 | 5.06 | 2.82 | 2590.01 | 2591.08 |  |
| Table 3-1:River discharge computation at different stages of flow Comparsion b/n Tail water depth and Hydraulic Jump |  |  |  |  |  |  |  |  |  |  |  |  |

From the above table , hd is found to be 2.59 m

- ha = He-hd =3.02m-2.59m = 0.42m

U/s HFL =U/s TEL –velocity head =2592.10m a.s.l – 0.42m = 2591.68m a.s.l

- Afflux

⇒ Afflux = U/s HFL- D/s HFL = 2591.68m a.s.l – 2589.91m a.s.l =1.77m.

Figure 3-2: Post jump Vs Tail water rating curve

From the flood level analysis, it is seen that the flood overtops the banks of the river u/s of the structure. This condition is not allowed to take place as it may bring pronounced negative impacts on the structures, so constructing structures/Retaining walls needed to confine it.

### Hydraulic Jump Calculation

As discussed in the geologic report, the river bed is solid rocks of basalt and hence stilling basin for energy dissipation is not required. But both left and right banks are not sound rock, a wing walls are required at u/s and D/s sides, so as to protect the scouring of the bank due to the formation of jumps, and not to flow the river out of river bank in high flood cases.

The length of wing walls is determined based on the length of Jump, the designer provide length of wing wall based on the length of Jump. The jump is calculated as shown below.

• Weir crest length = 17m

• Weir height = z =1.9m

• Pre-jump depth = y1

• Post -jump depth =y2

Neglecting losses between point A and B and considering similar datum

z + He = y1 + ha

but,  He = 3.02m

q=Ql=151.344m3/s17m=8.9m2/s

ha=q22*g*y2=8.90322*g*y12

1.9m + 3.02m=y1+388.525/y12

After iterations Y1 = 1.02m

V1=q/y1=8.9/1.02=8.745

Fr=V1gy1=8.7459.81*1.02=2.77

y2=y121+8*Fr2-1             y2=1.0221+8*2.772-1 =3.507m

Table 3-2: Post jump computation:

| q(m^3/s/m) | y1+((q/y1)^2/2g) | Y1 | Froud number, Fr1 | Post jump depth, Y2 | V2 | Fr2 | Post jump length |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 0.0 | 0.0 | 0.0 | 0.00 | 0.0 | 0.0000 | 0.0000 | 0.000 |
| 0.0 | 2.0 | 0.01 | 25.54 | 0.2 | 0.1700 | 0.1200 | 1.050 |
| 0.2 | 2.1 | 0.03 | 12.56 | 0.5 | 0.3700 | 0.1800 | 2.150 |
| 0.4 | 2.3 | 0.06 | 8.61 | 0.7 | 0.5600 | 0.2200 | 3.200 |
| 0.8 | 2.5 | 0.11 | 6.58 | 1.0 | 0.7700 | 0.2500 | 4.280 |
| 1.2 | 2.7 | 0.17 | 5.42 | 1.2 | 0.9800 | 0.2800 | 5.310 |
| 1.8 | 2.9 | 0.24 | 4.70 | 1.5 | 1.1800 | 0.3100 | 6.280 |
| 2.4 | 3.2 | 0.32 | 4.19 | 1.8 | 1.3700 | 0.3300 | 7.190 |
| 3.1 | 3.4 | 0.41 | 3.82 | 2.0 | 1.5500 | 0.3500 | 8.050 |
| 4.0 | 3.7 | 0.50 | 3.54 | 2.3 | 1.7400 | 0.3700 | 8.890 |
| 4.9 | 3.9 | 0.60 | 3.31 | 2.5 | 1.9100 | 0.3800 | 9.690 |
| 5.9 | 4.2 | 0.71 | 3.13 | 2.8 | 2.0900 | 0.4000 | 10.470 |
| 6.9 | 4.5 | 0.82 | 2.97 | 3.1 | 2.2600 | 0.4100 | 11.230 |
| 8.1 | 4.7 | 0.94 | 2.84 | 3.3 | 2.4300 | 0.4300 | 11.980 |
| 8.9 | 4.9 | 1.02 | 2.77 | 3.5100 | 2.5400 | 0.4300 | 12.450 |
| 9.4 | 5.0 | 1.06 | 2.73 | 3.6 | 2.6000 | 0.4400 | 12.710 |
| 9.5 | 5.1 | 1.08 | 2.72 | 3.6 | 2.6200 | 0.4400 | 12.790 |

Hydraulic jump length (L) for Fr=2.77from the above table L=5*(y2- y1) =5*(3.51-1.02) =12.45 but we don’t use this value to provide downstream Apron because the river bed is hard strata and hence the designer provide nominal downsteam and upstream apron length and thickness.

### Impervious floor

As explained before the river bed is solid rocks, So, the structure is not exposed to seepage and uplift force. Hence no need of calculation for apron length and thickness. So, the designer provides nominal downstream and upstream apron length and thickness.

#### D/s impervious floor (Ld)

Since the floor is hard strata, downstream apron is not provided and 1.5 m nominal length for upstream apron provided. Retaining wall is needed to confine the flow .So, the designer provides 13m long retaining wall.

### Cut off Depth Calculation

#### U/s cut off

As explained before the river bed is solid rocks, anchor at 1m depth to key to the hard rock and hence the total depth of Cut off is 1m which is quite enough.

#### D/s Cut off

Since the river bed is solid rocks, anchor at 1m depth to key to the hard rock and hence the total depth of Cut off is 1m which is quite enough as the upstream cutoff.

### Stability Analysis of weir

Stability analysis is carried out to see the already determined weir section is safe against overturning, sliding, tension. The stability analysis is carried out considering the effect of the following forces.

• Water pressure

• Weight of the over flow weir section

• Sediment load

|  |  |  | 1.0m |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| h=1.9m |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  | Toe |  |
|  |  |  |  | B=2.9 |  |  |  |  |
|  | Ps = Hw*γs |  |  |  |  |  |  |  |

Figure 3-3:Load combination of weir under Static  load combination

Table 3-3: Computation of forces per unit width and momentum

| Name of forces | Symbol | Description | Forces (KN) |  | Lever arm | Moments (KN-m) |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Vertical (+) | Horizontal (-) |  | (+) | (-) |
| 1.Vertical forces |  |  |  |  |  |  |  |
| 1.1 down ward forces |  |  |  |  |  |  |  |
| Weight of weir | W1 |  | 41.80 |  | 2.40 | 100.32 |  |
|  | W2 |  | 39.71 |  | 1.27 | 50.30 |  |
|  | W3 |  | 22.800 |  | 1.27 | 28.88 |  |
|  | W4 |  | 45.600 |  | 0.95 | 43.32 |  |
| Subtotal of (1) |  |  | 149.91 | 0.00 |  | 222.82 | 0.00 |
| 1.2 Upward forces |  |  |  |  |  |  |  |
| uplift | Pu1 |  |  | 7.250 | 1.93 |  | 0.00 |
|  | Pu2 |  |  | 14.500 | 1.45 |  | 0.00 |
| Subtotal | Σpu |  | 0.000 | 21.750 |  | 0.000 | 0.000 |
| Summation ΣV & ΣM |  |  | 149.910 | 21.750 |  | 222.819 | 0.000 |
| 2. Horizontal force |  |  |  |  |  |  |  |
| 2.1 u/s water pressure |  |  |  |  |  |  |  |
|  | P1 |  |  | 18.050 | 0.63 |  | 11.43 |
| 2.2 u/s silt pressure |  |  |  |  |  |  |  |
|  | Ps |  |  | 19.548 | 0.63 |  | 12.38 |
| Summation ΣH & ΣM |  |  | 0.000 | 37.598 |  | 0.000 | 23.812 |
| Total |  |  | 149.910 | 59.348 |  | 222.819 | 23.812 |

∑V = 149.910KN                                 ∑M(+) = 222.819KN.m

∑H=59.348KN	  ∑M(-) = 23.812KN.m

i) Factor of safety against overturning (Fo)

=9.36 Safe!

ii) Factor of safety against sliding (FS)

, 1.64>1.5          Safe!

iii) Check for tension (i.e. whether the resultant lies within the middle third)

The location of the resultant force from the toe is given by

The eccentricity (e) = X – B/2, B =2.9m

Hence, e = 0.122m

The eccentricity (e) should be less than B/6 =0.4833, Hence the obtained e = 0.122m < 0.4833m.

⇒The resultant lays within the middle third                  no tension

### Stability Analysis of weir under dynamic load combination

|  |  |  |  |  | 1.2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |

Figure 3-4:Load combination of weir under Dynamic load combination

Table 3-4: Computation of forces per unit width and momentum

| Name of forces | Symbol | Description | Forces (KN) |  | Lever arm | Moments (KN-m) |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Vertical (+) | Horizontal (-) |  | (+) | (-) |
| 1.Vertical forces |  |  |  |  |  |  |  |
| 1.1 down ward |  |  |  |  |  |  |  |
| Weight of weir | W1 |  | 41.80 |  | 2.40 | 100.32 |  |
|  | W2 |  | 39.71 |  | 1.27 | 50.30 |  |
|  | W3 |  | 22.800 |  | 1.27 | 28.88 |  |
|  | W4 |  | 45.600 |  | 0.95 | 43.32 |  |
| Subtotal |  |  | 149.91 | 0.00 |  | 222.82 | 0.00 |
| weight of water | Ph2 |  | 12.968 |  | 0.67 | 8.65 |  |
|  | Ph3 |  | 18.050 |  | 0.63 | 11.43 |  |
| Subtotal | ΣWw |  | 31.018 | 0.000 |  | 20.077 | 0.000 |
| Subtotal of (1) |  |  | 180.928 | 0.000 |  | 242.897 | 0.000 |
| 1.2 Upward forces |  |  |  |  |  |  |  |
| uplift | Pu1 |  |  | 7.250 | 1.93 |  | 0.00 |
|  | Pu2 |  |  | 14.500 | 1.45 |  | 0.00 |
| Subtotal | Σpu |  | 0.000 | 21.750 |  | 0.000 | 0.000 |
| Summation ΣV & ΣM |  |  | 330.838 | 21.750 |  | 465.716 | 0.000 |
| 2. Horizontal force |  |  |  |  |  |  |  |
| 2.1 u/s water pressure |  |  |  |  |  |  |  |
|  | P1 |  |  | 18.050 | 0.63 |  | 11.432 |
| 2.2 u/s silt pressure |  |  |  |  |  |  |  |
|  | Ps |  |  | 19.548 | 0.63 |  | 12.380 |
| Summation ΣH & ΣM |  |  | 0.000 | 37.598 |  | 0.000 | 23.812 |
| Total |  |  | 330.838 | 59.348 |  | 465.716 | 23.812 |

Summary of Factor of safety

| Force and/or moment | Values & Comments |  |  | Remark |
| --- | --- | --- | --- | --- |
|  | Computed | Recommended | Condition | critical section |
| For high flood condition |  |  |  |  |
| 1. Sliding | 3.62 | 1.50 | ok! |  |
| 2. Over-turning | 19.56 | 1.50 | ok! |  |
| 3. Vertical stress/Tension | 0.11 | 0.48 | ok! | e < B/6 |
| 4. Contact Pressure |  |  |  |  |
| Pmax | 141.1 | 880 | ok! |  |
| Pmin | 87.1 | 440 | ok! |  |
| For downstream empty condition |  |  |  |  |
| 1. Sliding | 1.64 | 1.50 | ok! | Governing |
| 2. Over-turning | 9.36 | 1.50 | ok! |  |
| 3. Vertical stress/Tension | 0.12 | 0.48 | ok! |  |
| 4. Contact Pressure |  |  |  |  |
| Pmax | 64.8 | 880 | ok! |  |
| Pmin | 38.59 | 440 | ok! |  |

Conclusion: The extreme load combination is the dynamic case where the head is at crest level of the weir and there is flow over the weir .From stability analysis, the designed weir section is safe. So, Provide 1.0m top width and 2.9m bottom width.

### Design of Divide wall, Under Sluice, and Canal outlet

Divide wall is designed in order to create separation between outlet canal and natural river course. The divide wall allows safe and stable base flow to the canal outlet. Flow turbidity created by current flow impact over the weir body is reduced.

#### Wall height fixation of Divide Wall

The existing topographical condition at the weir axis and HFL are considered to be most governing parameters for fixing the wall height.

After construction of the weir  (u/s HFL) = 2591.68m.a.s.l

Hence the level of divide wall and weir body foundation should be the same and is 2587.188m

U/s wall height = U/s HFL - foundation level + free board.

Adapt 0.2m free board

U/s wall height = (2591.68) m.a.s.l – 2587.188m + 0.2m =4.80m

Provide 4.80m wall height and the top level of the divide wall =2587.188m + 4.8m =2592.0m

D/s wall height = Max(D/s HFL/Jump level) - foundation level + free board

=(2590.695m.a.s.l-2587.188m.a.s.l+0.2=3.8 take 3.8m

Provide 3.8m wall height and the top level of the divide wall =2587.188+3.8=2591.0m.a.s.l

Hence including free board, take the critical height of the divide wall = 3.8m

Divide wall Stability Analysis

Table 3-5: Computation of forces per unit width and momentum

| Code | Load |  | Lever Arm, R | Moment |  |
| --- | --- | --- | --- | --- | --- |
|  | Vertical | Horizontal |  | Posetive | Negative |
| W1 | 162.6 |  | 0.5 | 81.3 |  |
| W2 | 0.0 |  | 1.0 | 0.0 |  |
| Ps |  | -24.0 | 1.0 |  | 23.2 |
| Pw |  | -41.3 | 1.0 |  | 39.9 |
| Pw2 |  | 41.3 | 1.0 | 39.88 |  |
| Ps2 |  | 10.3 | 1.0 | 10.0 |  |
| Ww | 0 |  | 1.0 | 0.0 |  |
| Ws | 0 |  | 1.0 | 0.0 |  |
| Pu | 0.0 |  | 0.0 |  | 0.00 |
| SUM | 162.6 | -13.6 |  | 131.2 | 63.0 |

Factor of Safety: Against Overturning, Sliding and Tension

| Over turning = (M+ve/M-ve) >1.5 |  |  |  | Fo | 2.08 | >1.5 | OK |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Sliding = (µxFv/Fh),---µ=0.75 >1.5 |  |  |  | Fs | 8.94 | >1.5 | OK |
| Tension: X= (Net Moment/Sum Fv), e=x-B/2, e<B/6 |  |  |  | X | 0.41 |  |  |
|  |  | B/6= | 0.166 | e | 0.081 | <B/6 | OK |

#### Under sluice

The under sluice is mainly provided here to remove silt deposition at a result of barrier structure. Hence the sill level of the under sluice is fixed to facilitate this deposited silt to increase the efficiency of water abstracting to the main canal through the head regulator from the pocket. The Sill level of this sluice is fixed to be 0.7m higher than the minimum bed level. Hence the Sill level of the under sluice = 2587.188+0.7=2587.888 Even if the position of the under sluice is on concave side that is on scouring side, there might be boulders that may come into the pocket of the under sluice due to the barrier structure. Hence in addition to the supply of water to the intake and the removal of silt, this acts to remove the boulder that comes to wards it. Considering this, the opening size of the gate is 1.2m*1.2m with spindle type operating from the operation slab. Considering rectangular notch profile of flow of water at the under sluice, the discharge passing is computed using the following formula.

- The capacity should be at least five times the canal discharge to ensure proper scouring.

- Capacity of passing about 10% to 20% of the maximum flood discharge at high floods.

- During construction, it should be able to pass the prevailing (at least base flow) discharge of the river.

But the practical application of those values is not sound so the discharging capacity of under sluice is determined by using orifice flow formula for maximum flood condition.

i.e. Q=Cd*A*2gh,

Where, Cd = Discharge coefficient =0.60

A = under sluice opening area (m2)

g = 9.81m/s2

But using Q=10%Maximum discharge leads for exaggerated size of opening =10*0.15134=15.134m3/s, we recommend to use 3.6% of Qd which is 5.5 m3/s for computation.

Width (b) = Qd Cd*H2g*h      5.50.6*1.22*9.81*2.06  =1.201m take 1.2m

Hence the size of the under sluice is 1.2m×1.2m    (height *width)

#### Canal outlet level

The head regulator is provided on the right side .The Sill level of this head regulator is fixed from different angle observations. At 240m the left main canal breach from right main Canal which crosses the main river/Azuwari. The main conveyance systems are 3.940km, 5.038 km right and left respectively which passes more gullies and undulating alignment. Hence this level is fixed based on the optimum route alignment and the maximum  irrigated command level including minor and major losses criteria. Based on this condition, the Sill level is fixed to be 2588.525m.

• Outlet capacity

The minimum command area is determined by the minimum flow of the river. But the canal capacity should be determined for maximum command area and the corresponding discharge. In this case the outlet capacity is fixed considering maximum duty and command area and 1.0 correction factors are considered to account the variation of duty.

- Outlet capacity = Duty x command area x correction factor

Where, maximum duty for 18 hr irrigation = 1.2 L/s/ha

Command area = 156ha.

- Outlet capacity = 1.2 L/s/ha x 156 ha x1.0 = 188 L/sec

• Outlet size

From the weir discharge formula the outlet size is determined as follows

Q = CLHe3/2

Where;  C = coeff. of discharge = 1.7

L = Length of water way (m)

He = head above sill level (neglecting the velocity head) = 0.5m

=0.188/(1.7*0.5^3/2)=0.339

⇒Adopt water way length = 0.5m

Hence, provide an outlet size of 0.6m x 0.5m (length x height) .The gate of the off take canal is to be vertical sheet metal of 0.6m x 0.5m for the closure of the opening space. Provide some extra dimensions for groove insertion. Gross area of sheet metals for the off take canal gate will be 0.7m x 0.55m (allowing 5cm insertion for grooves). The grooves are to be provided on the walls using angle iron frames at the two sides of the gate openings.

Trash racks of diameter 10mm with c/c spacing of 10cm has to be provided u/s of the gate to prevent entry of debris to the canal.

#### Breast Wall and Operation Slab

A vertical raised gate is designed for the head regulator and under sluice. These gates slide over the breast wall using spindle during opening and closing.

For easy operation of these gates, operation slab is provided. The size of the operation slab is fixed from the point of construction and free movement. The size of the operation slab is shown in the drawing for both head regulator and under sluice with thickness 0.2m.

The thickness of the breast wall is also the same as that of the operation slab. The nominal thickness is fixed from the point of construction rather than the imposed load. The thickness required for the imposed load is less than the nominal value and treated as cantilever retaining wall. For the breast wall, the minimum reinforcement area is taken as 15% along the respective direction.

Hence	Asteel = 0.0015*1000*200=300

Asteel = 300mm2/m

Provide 12@C/c 200 mm

Considering cover thickness of 50 mm, effective depth = 50 + 12/6 = 56

Hence spacing of reinforcement = 200mm < 432mm

Asteel = 565.2

Therefore the actual provided steel area per meter width is 565 mm2/m > 300 mm2/m. It is ok!

The actual arrangement of angle irons, spindles, shafts and operation slab including other components is shown on the design drawing.

For each arrangement and further information, refer to the design drawing.

#### Retaining Walls

At the two ends of the diversion weir, walls have been provided to safeguard the structure from scour at the ends and also as a facility to the canal outlet operation and maintenance at the canal outlet portion.

Upstream Retaining Wall

3.4

0.5

Figure 3-5: Load combination of Retaining walls

Table 3-6: Typical design of masonry retaining wall

| Considered soil data for selected granular material |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Ø | 30 | º |  |  |  |
| Ÿsoil | 19 | KN/M3 |  |  |  |
| C | 0 |  |  |  |  |
| Ÿmasonry | 23 | KN/M3 |  |  |  |
| Ø | 0.52 | radians |  |  |  |
| Delta | 20 |  |  |  |  |
| tan(delta)=µ | 0.364 |  |  |  |  |
| Allowable soil pressure= |  | 400 | KN/M2 |  |  |
|  |  | Ka = | 0.33 |  |  |
| Kp = ( 1+sinØ )/( 1- sinØ ) |  | Kp = | 3 |  |  |
| Table 3-7: Computation of forces per unit width and momentum For Upstream retaining wall Name of forces; Symbol; Description; Forces (KN); Lever arm; Moments (KN-m). Vertical (+); Horizontal (-); (+); (-). Weight of Retaining Walls; W1; 52.84; 0.25; 13.21. W2; 153.24; 1.47; 224.75. W3; -; 1.70; 0.00. Soil self weight; Ps1; 125.76; 2.433; 306.02. Active soil pressure; Ps2; 125.76; 1.601; 201.37. Water Pressure; Ph1; 115.38; 1.601; 184.74. Total; 447.22; 125.76; 728.72; 201.37. 1. Sliding. 2.31; ok!; Resultant force R =; 799.58. 2. Over-turning; Allowable compressive stress in masonry kg/cm2 =. 3.52; ok!; Allowable tensile stress in masonry kg/cm2=. 3. Tension; B or L=; 3.4; B/6 =; 0.57. M=; 527.3. Arm of Resultant force from toe, X i.e. centroidal distance=M/Fv=. Eccentricity, e =; 0.52; ok!. Downstream Retaining Wall Table 3-8: Computation of forces per unit width For Downstream retaining wall Name of forces; Symbol; Forces (KN); Lever arm; Moments (KN-m). Vertical (+); Horizontal (-); (+); (-). Weight of Retaining Walls; W1; 41.77; 0.25; 10.44. W2; 91.90; 1.97; 180.73. W3; 5.28; 3.00; 15.84. Soil self weight; Ps1; 78.59; 1.97; 154.56. Active soil pressure; Ps2; 78.59; 1.27; 99.48. Total; 217.54; 78.59; 361.58; 99.48. 0.5 2.7 3.8 Factor of safety 1. Sliding. 1.80; ok!; Resultant force R =; 231.3. 2. Over-turning; Allowable compressive stress in masonry kg/cm2 =; 120. 3.63; ok!; Allowable tensile stress in masonry kg/cm2=; 20. 3. Tension; B or L=; 2.7; B/6 =; 0.45. M=; 262.1. Arm of Resultant force from toe, X i.e. centroidal distance=M/Fv=; 1.2. Eccentricity, e =; 0.145; ok!. Table 3-9: Summary of Stability analysis of Retaining Walls Force and/or moment; Values & Comments; Remark. Computed; Recommended; Condition. For WL at its max i.e. HFL. 1. Sliding; 2.31; 1.50; ok!. 2. Over-turning; 3.62; 1.50; ok!. 3. Vertical stress/Tension; 0.52; 0.45; ok!; e < B/6. 4. Contact Pressure. Pmax; 252.4; 400.0; ok!. Pmin; 10.6; 20; ok!. For no overflow condition on the d/s. 1. Sliding; 1.80; 1.50; ok!. 2. Over-turning; 3.63; 1.50; ok!. 3. Vertical stress/Tension; 0.145; 0.45; ok!. 4. Contact Pressure. Pmax; 106.6; 400; ok!. Pmin; 54.58; 20; ok!. |

## Bill of Quantity and Cost Estimation

The quantities of the various items have been worked out as per the final design and final drawings prepared for the scheme. The unit rates analysis has been carried out based on the data available in the vicinity of the project area.

Bill No. 1- General Items

| BILL No. 1General Items |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Item No | Description | Unit | Quantity | Rate Birr | Amount Birr | Rate Birr | Amount Birr |
| 1.1 | 2km Accesses road | L.S | 1 | 667,723.29 | 667,723.29 | 667,723.29 | 667,723.29 |
| 1.2 | Allow for mobilization | L.S | 1 | 173,789.70 | 173,789.70 | 173,789.70 | 173,789.70 |
| 1.3 | Allow for demobilization | L.S | 1 | 156,410.73 | 156,410.73 | 156,410.73 | 156,410.73 |
| 1.4 | Allow for contractors camp and facilities =4*5m2 , Living rooms & offices for construction key personnel, CIS and Interminally painted chipwood wall, Masonary floor cement screened and well ventilated room complet with doors and windows. | No | 4 | 61,256.73 | 245,026.92 | 61,256.73 | 245,026.91 |
| 1.5 | Allow for consultants camp and facilities =4*5m2 , Living rooms & offices forConsultants CIS and Interminally painted chipwood wall, Masonary floor cement screened and well ventilated room complet with doors and windows. | No | 2 | 61,256.73 | 122,513.46 | 61,256.73 | 122,513.45 |
| 1.6 | 5*5m2, Store constructed from CIS with doors and windows, Masonary floor cenmet screened | No | 1 | 40,447.44 | 40,447.44 | 40,447.44 | 40,447.44 |
| 1.7 | Barded wire fence 50*40m and 1.5m high treated timber post complet with 3m wide gate and a CIS grard house (1.5*2m) | LS | 1 | 22,235.44 | 22,235.44 | 22,235.44 | 22,235.44 |
| 1.8 | Dewatering of open trenches and excavations, pumps | LS | 1 | 23,310.00 | 23,310.00 | 23,310.00 | 23,310.00 |
| 1.9 | Provide project indicator post starting from the construction time | LS | 1 | 3000 | 3000 | 3000 | 3000 |
| 1.10 | Provision of as built drawings for the project | LS | 1 | 10000 | 10000 | 10000 | 10000 |
| Total |  |  |  |  | 1,464,456.98 |  | 1,464,456.96 |
| Total for general Items |  |  |  |  | 1,464,456.98 |  | 1,464,456.96 |

| S. no | Item of work |  |  |  | Unit |  | Quantity | Unit Price | Amount (ETB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Over flow weir body |  |  |  |  |  |  |  |  |
| 1.1 | Site clearing |  |  |  | m2 |  | 505.912 | 60.47 | 30,591.75 |
| 1.2 | Ordinary soil excavation |  |  |  | m3 |  | 284.9965 | 65.00 | 18,524.77 |
| 1.3 | Hard rock excavation |  |  |  | m3 |  | 10.846 | 400.00 | 4,338.40 |
| 1.5 | Masonry (1:3) |  |  |  | m3 |  | 58.007 | 2,009.16 | 116,545.24 |
| 1.6 | C-20 concrete(1:2:3) |  |  |  | m3 |  | 21.131 | 2,981.49 | 63,001.84 |
| 1.7 | Ф14mm bar |  |  |  | kg |  | 431.5227648 | 38.50 | 16,614.57 |
| 1.8 | Ф12mm bar |  |  |  | kg |  | 331.87376 | 38.50 | 12,777.87 |
| 2 | Apron and cutoff |  |  |  |  |  |  |  |  |
| 2.1 | Ordinary soil excavation |  |  |  | m3 |  | 19.008 | 65.00 | 1,235.52 |
| 2.2 | Hard rock excavation |  |  |  | m3 |  | 24.833 | 400.00 | 9,933.20 |
| 2.4 | C-20 concrete(1:2:3) |  |  |  | m3 |  | 24.123 | 2,981.49 | 71,922.46 |
| 2.5 | Ф14mm bar |  |  |  | kg |  | 292.8193404 | 38.50 | 11,274.19 |
| 2.6 | Ф12mm bar |  |  |  |  |  | 232.311632 | 38.50 | 8,944.51 |
| 3 | Operational slab,breast wall,undersluice & dividewall |  |  |  |  |  |  |  | 0.00 |
| 3.1 | Ordinary soil excavation |  |  |  | m3 |  | 80.766 | 65.00 | 5,249.79 |
| 3.2 | Hard rock excavation |  |  |  | m3 |  | 7.841 | 400.00 | 3,136.40 |
| 3.4 | Masonry (1:3) |  |  |  | m3 |  | 2.154 | 2,009.16 | 4,327.73 |
| 3.5 | C-20 concrete(1:2:3) |  |  |  | m3 |  | 25.131 | 2,981.49 | 74,927.80 |
| 3.6 | Ф14mm bar |  |  |  | kg |  | 215.23 | 38.50 | 8,286.82 |
| 3.7 | Ф12mm bar |  |  |  | kg |  | 200.490 | 38.50 | 7,719.33 |
| Sub total |  |  |  |  |  |  |  |  | 469,352.16 |
| 4 |  | Retaining wall |  |  |  |  |  |  |  |
| 4.1 |  | Upstream River retaining wall |  |  |  |  |  |  |  |
| 4.1.1 |  | Excavation(soil) |  |  | m3 |  | 663.641 | 65.00 | 43,136.67 |
| 4.1.2 |  | Hard rock excavation |  |  | m3 |  | 20.473 | 150.00 | 3,070.95 |
| 4.1.3 |  | Back fill |  |  | m3 |  | 419.328 | 15.00 | 6289.92 |
| 4.1.4 |  | Masonry (1:3) |  |  | m3 |  | 428.5336 | 2,009.16 | 860,991.83 |
| 4.1.5 |  | mass concrete C-10(1:3:6) |  |  | m3 |  | 16.9835 | 1,942.89 | 32,997.14 |
| 4.1.6 |  | Plastering(1:3) |  |  | m2 |  | 338.58 | 156.58 | 53,014.51 |
| 4.2 |  | Downstream of river retaining wall |  |  |  |  |  |  |  |
| 4.2.1 |  | Excavation(soil) |  |  | m3 |  | 239.58 | 65.00 | 15,572.70 |
| 4.2.2 |  | Hard rock excavation |  |  | m3 |  | 11.682 | 150.00 | 1,752.30 |
| 4.2.3 |  | Masonry (1:3) |  |  | m3 |  | 244.1376 | 2,009.16 | 490,511.08 |
| 4.2.4 |  | mass concrete C-10(1:3:6) |  |  | m3 |  | 12.331 | 1,942.89 | 23,957.83 |
| 4.2.4 |  | Back fill |  |  | m3 |  | 218.88 | 43.64 | 9,551.92 |
| 4.2.5 |  | plastering(1:3) |  |  | m2 |  | 204.6 | 156.58 | 32,036.06 |
| Sub total |  |  |  |  |  |  |  |  | 1,572,882.89 |
| 7 |  |  | Gates & trash rack |  |  |  |  |  |  |
| 7.1 |  |  | under sluice gate installation consist of: |  |  | No | 1 | 9,799.78 | 9,799.78 |
|  |  |  | 6mm thickness sheet metal, 1.3m X 1m |  |  |  |  |  |  |
|  |  |  | 50x50x10mm angle iron, |  |  |  |  |  |  |
|  |  |  | spidle of 60mm dia stem, 2.4m length |  |  |  |  |  |  |
| 7.2 |  |  | offtake canal gate installation consist of:- |  |  | No | 1 | 9,799.78 | 9,799.78 |
|  |  |  | 6mm thickness sheet metal, 0.5m X 0.5m |  |  |  |  |  |  |
|  |  |  | 40x40x10mm angle iron |  |  |  |  |  |  |
|  |  |  | spidle of 60mm dia stem, 2.73m length |  |  |  |  |  |  |
| 7.3 |  |  | Trash rack |  |  |  |  |  |  |
| 7.3.1 |  |  | Ф12mm reinforcement bar |  |  | kg | 9 | 45.44 | 408.98 |
| 7.3.2 |  |  | Trash rack concrete cover C-20 |  |  | m2 | 1.3 | 2,981.49 | 3,875.94 |
|  |  |  |  |  |  |  |  |  | 23,884.48 |
| total |  |  |  |  |  |  |  |  | 2,066,119.53 |

# SECTION-III: IRRIGATION AND DRAINAGE SYSTEMS INFRASTRUCTURE

# IRRIGATION AND DRAINAGE SYSTEMS DESIGN

## Irrigable Area Description

### Topography

Topography is an important factor for the planning of any irrigation project as it influences method of irrigation, drainage, erosion, mechanization, and cost of land development, labour requirement and choice of crops.

The topographic feature of the far surroundings of the project area, especially in the –North and South a direction, is dominated by chains of mountainous escarpments. Nonetheless, the topographic feature of the command area slope is ranging from 2-5 %. The slope gradient class also ranges from nearly level (2%). Hence, it has identified to be almost suitable for surface irrigation.

The project command area is situated both at the left and right side of Azuwari River (to the East side of the river). The natural topographic feature of the command area has inclined from the North-East to the South-West direction.

### Climate

As per the hydrological analysis and on the basis of the traditional Ethiopian Agro-Ecological Zones (MOA, 2001), the UGDWIP area is basically classified as Dega (moist cool) agro-ecological zone, indicating better moisture condition in the area in wet seasons. There is no belg rain season in the project area. Despite the fact that the Meher rains are considered adequate, there is notable variation in terms of onset, distribution and withdrawal from year to year affecting crop production in general and crop productivity in particular.

As the project site has no its own meteorological station, Gundeweyn (for rainfall and minimum and maximum temperature  relative humidity, wind speed and sunshine hour) meteorological station data was used for the project study as long as this station is  relatively near to the proposed command area. In general, the sources of meteorological data are the National Meteorology Service Agency (NMSA).

### Soil characteristics

Soil properties (physical, chemical, etc.) greatly influence the growth and thereby yield of crops which is grown. For the recommendation of important macro and /or micro elements for the required fertilizer application purposes a soil composite sampling has been made with the soil depth intervals of 0 - 25 cm and 25 - 50 cm. (for further detail see the Agronomy Study of the same project).

### Existing Irrigation Practices in the Project Area

The pressure of survival and the need for additional food supplies to meet the demands of the increasing population is necessitating a rapid expansion of irrigation scheme throughout the world. Thus, irrigation is becoming a basic part of well-developed agriculture throughout the world.

The existing low productivity and production capacity of rain-fed crops, due to uneven distribution of rainfall and heavy hail damage, has enforced farmers to develop traditional irrigation practices. Therefore, irrigated agriculture has gradually developing following rainfall abnormalities and scarcity of land. In the project area irrigated crop production is being practiced using water from Azuwari river.

In the project area irrigated crop production is being practiced using water from Azuwari river  by diverting traditionally and leading water to plots of farmlands. Furthermore, farmers in the project area are traditionally using Azuwari river as a source of water for irrigation. Hence, some parts of farmers of the project kebele are traditionally trying to practice two seasons’ crop production (rain fed and irrigation).

Based on data obtained from Berjano Kebele agriculture development Office, in the 2018 cropping year, a total of 396.7 hectares of land was covered by irrigated crops in the project kebele and about 58885quintals yield was produced (Table 13). Farmers of the project kebele have strong demand on the constructing of Diverion/weir structure on Azuwari  river.

### Crop Water Requirement (CWR)

The calculation of crop water requirement is a very important aspect for planning of any irrigation project. Several methods and procedures are available for this. The Food and Agriculture Organization (FAO) of the United Nations has also made available several publications on this subject and other issues related with this. The computer program available in FAO Irrigation and Drainage Paper No. 56 “CROPWAT” has been used for the calculation of Crop Water requirement. This program is based on Penman-Monteith approach and procedures for calculation of crop water requirements and irrigation requirements are mainly based on methodologies presented in FAO Irrigation and Drainage Paper No. 24 “Crop Water Requirements” and No. 33 “Yield Response to Water”.

The corresponding values of the crop water requirements of the proposed crops of the project are presented in the Agronomy Study of the same project.

### Irrigation efficiency (Ep)

To complete the evaluation of the demand, the efficiency of the water distribution system and of application must be known.

The gross requirement of water for irrigation system is very much dependent on the overall efficiency of the irrigation system, which in turn is dependent on several factors: Method of irrigation, type of canal (Lined and/or Unlined), method of operations (simultaneously and continuous or Rotational water supply), and availability of structures (for controlling and distribution and measuring and monitoring).

On the basis of these factors, the project has planned to impose surface irrigation method (using furrows). The canal system is lined canal. Hence, the conveyance efficiency has been estimated to be 95%, distribution efficiency 85%, and field application efficiency 62%. As a result of these the overall irrigation efficiency has been estimated to be 50%. According to soil Lab result, soils of the command area are dominantly characterized as Clay textured soils.

### Irrigation duty

Irrigation duty is the volume of water required per hectare for the full flange of the crops. Moreover, it helps in designing an efficient irrigation canal system.

The area, which will be irrigated, can be calculated by knowing the total available water at the source and the overall duty for all crops required to be irrigated in different seasons of the years.

The proposed cropping pattern of Azuwari one Diversion Weir irrigation project has showed maximum net irrigation water requirement (NIWR) for overall proposed crops is in the month of March with the amount of mm/day for 18 working or irrigation hours.

For Azuwari river Diversion Irrigation Project, it decided to adopt 62% field application efficiency, 85% distribution efficiency, and 95% conveyance efficiency as the soil is clay textured soils and the canal systems are estimated to be lined. Hence, the overall/project efficiency for the selected surface irrigation method has been estimated to be 50% (62/100*95/100*85/100) =50.065% which is rounded to 50%.

For the designing of the project, the GIWR is given as follows:

GIWR = 3.8/0.50 = 7.6[mm/day]

The GIWR, 7.6 mm/day, represents the daily quantity of water that is required to be applied. This water quantity is also used for the determination of the canal discharge in consideration of the time of flow and is defined as the duty, expressed as l/s/ha.

The duty is calculated by:

Duty (D) = GIWR × 1000 × 10 / (t × 60×60)

Where; Duty – the duty [l/s/ha]

GIWR – Gross Irrigation Requirement [mm/day]

t – Daily irrigation or flow hours [hrs]

The duty for the GIWR of  7.6 mm/day and 18 hours of daily irrigation time (t = 18), is supported to be used with furrow irrigation method. Hence, Duty for 18 working hours, as the site is nearer to farmers’ village and local farmers have experiences in irrigation, is computed as follows:

D = (7.6 x 1000 x 10) / (18 x 3600)= 1.17 which is rounded to 1.2l/s/ha

### Irrigation methods

Among the different irrigation systems surface irrigation system will be used for the project area; and the irrigation water will be obtained from  Azuwari  River and by constructing diversion weir and convoying the water commonly through earthen canals (MC, SC, and TC) and then leading to field canals; and finally irrigation takes place mostly in furrows.

For this project, among the various irrigation methods, surface irrigation method has been selected.  Of the surface irrigation methods furrow, border and basin irrigation methods can be used to supply irrigation water to the plants/crops. However, each method has its own advantages and disadvantages. Care should be taken when choosing the method which is best suited to the local circumstances, i.e., depending on slopes, soil types, selected crop types, amount of water available, etc. of the command area.

Based on the above factors surface irrigation method has been proposed for the proposed crops in this project. The method allows applying light irrigation and can be laid out in sloping fields along the contour. Furrow irrigation method is best suited for most of the proposed and row planted crops. In general, furrow irrigation method is simple, manageable and widely practiced irrigation method. This method is suitable for row crops that cannot stand in water for long periods. The only thing required to use this method is row planting of crops. Besides, basin and border irrigation method would be used for the non-row planted crops. Rotational flow water distribution is also recommended for the project area.

## Irrigation and Drainage System Layout

The irrigation system layout for the project is prepared taking the following points into consideration besides other factors.

- A primary concern in the layout of the system is that it serves the purpose of conveying and distributing water to the command area.

- The excavation and earth fill volumes not be excessive, otherwise the construction costs can be tremendous.

- The selection of longitudinal bed slope is made taking into account the existing slopes of the terrain, so as to minimize deviations in canal routing.

- Curves in canals should not be too sharp.

The proposed irrigation system layout comprises two (2) main canal, Five (5) secondary canals and  (44) tertiary canals as shown on the layout Drawings. The main canal runs for most of its length parallel to the contours and several changes of direction are necessary to follow the topography. It crosses many main gullies. The main canal is masonry lined for a length of 3940m and 5038 meters right and left respectively starting from the weir outlet to make maintenance easier since this part of the canal may be subjected to flooding during high flood flows.

### Conveyance System

The conveyance system consists of two (2) Main Canal to irrigate total command area of 156 ha. The main canal starts from Water abstraction site on right side and conveys water for a length of 3.94Km and 5.038km left main canal.

Main canal is aligned along contours and supplies to 5 secondary unit and 44 tertiary canals.

## Design of the Canal System

Flow Depth and Section Capacity

The earthen canals have been designed with a trapezoidal shape and the lined ones with rectangular x-section using Manning's Formula:

Where Q= discharge (m3/s)

R= Hydraulic radius (Flow area/wetted perimeter)

S= Hydraulic gradient

n= Manning's roughness coefficient, n=0.025 is adopted for the earth channels and n=0.018 for the masonry lined part of the main canal

### Main Canal

The main canal is designed for a discharge of 188l/s and depending on the site specific condition, appropriate slope is provided. Hydraulic parameters of the main canal are shown below.

Table 4-1: Hydraulic Parameters of right main canal

| Canal | Chainage (m) | Anet (ha) | 24hr Duty (l/s/ha) | Qreqd (m3/s) | n | s | b | m | d | V (m/s) | Qcalcu | Qcalc -Qreqd | Fb(m) | D(m) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0+000-0+240 | 156.25 | 1.20 | 0.188 | 0.018 | 0.0013 | 0.76 | 0 | 0.38 | 0.65 | 0.188 | 0.000 | 0.25 | 0.65 |
| RMC | 0+240-3+170 | 46.04 |  | 0.055 | 0.018 | 0.0017 | 0.46 | 0 | 0.23 | 0.53 | 0.056 | 0.000 | 0.25 | 0.50 |
|  | 3+170-3+940 | 31.46 |  | 0.038 | 0.018 | 0.0017 | 0.40 | 0 | 0.20 | 0.49 | 0.038 | 0.000 | 0.25 | 0.45 |

BW= Canal bottom width                         QR= Required discharge

FSD= Full supply depth                             QD= Designed Discharge

FB= Free board

SS= Side slope

Vel= Velocity

Table 4-2: Hydraulic Parameters of left main canal

| Canal | Chainage (m) | Anet (ha) | 18hr Duty (l/s/ha) | Qreqd (m3/s) | n | s | b | m | d | V (m/s) | Qcalcu | fb(m) | D(m) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| LMC | 0+000-0+875 | 110 | 1.20 | 0.132 | 0.018 | 0.0013 | 0.67 | 0 | 0.33 | 0.60 | 0.133 | 0.25 | 0.58 |
|  | 0+4134-0+4184,0+4404-0+4504&0+4644-0+4704 |  |  | 0.132 | 0.018 | 0.0250 | 0.38 | 0 | 0.19 | 1.83 | 0.133 | 0.25 | 0.44 |
|  | 0+875-0+3064 |  |  | 0.132 | 0.018 | 0.0010 | 0.70 | 0 | 0.35 | 0.55 | 0.132 | 0.25 | 0.60 |
|  | 0+3064-0+4134,0+4184-0+4404,0+4504-0+4644&0+4704-0+5038 |  |  | 0.132 | 0.018 | 0.0020 | 0.61 | 0 | 0.31 | 0.71 | 0.132 | 0.25 | 0.56 |

### Secondary Canals

Table 4-3: Hydraulic Parameters of secondary canals

| Canal | Chainage (m) | Anet (ha) | 18hr Duty (l/s/ha) | Qreqd (m3/s) | n | s | b | m | d | R(m) | V (m/s) | Qcalcu | Qcalc -Qreqd | Fb(m) | D(m) | T(m) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SC-1-LMC-1 | 0+000-0+040 | 5.16 | 1.20 | 0.006 | 0.018 | 0.0833 | 0.10 | 0 | 0.05 | 0.02 | 1.35 | 0.006 | 0.000 | 0.25 | 0.30 | 0.70 |
|  | 0+040-0+190 |  |  | 0.000 | 0.018 | 0.0588 | 0.11 | 0 | 0.05 | 0.03 | 1.20 | 0.007 | 0.007 | 0.25 | 0.30 | 0.71 |
| SC-2-LMC-1 | 0+000-0+030 | 9.96 | 1.20 | 0.012 | 0.018 | 0.1111 | 0.12 | 0 | 0.06 | 0.03 | 1.76 | 0.012 | 0.000 | 0.25 | 0.31 | 0.72 |
|  | 0+030-0+060 |  |  | 0.012 | 0.018 | 0.1429 | 0.11 | 0 | 0.06 | 0.03 | 1.95 | 0.012 | 0.000 | 0.25 | 0.31 | 0.71 |
|  | 0+060-0+120 |  |  | 0.012 | 0.018 | 0.0833 | 0.12 | 0 | 0.06 | 0.03 | 1.58 | 0.012 | 0.000 | 0.25 | 0.31 | 0.72 |
|  | 0+120-0+250 |  |  | 0.012 | 0.018 | 0.0333 | 0.15 | 0 | 0.07 | 0.04 | 1.12 | 0.012 | 0.000 | 0.25 | 0.32 | 0.75 |
| SC-3-LMC-1 | 0+000-0+090 | 25.73 | 1.20 | 0.031 | 0.018 | 0.0500 | 0.19 | 0 | 0.10 | 0.05 | 1.65 | 0.031 | 0.000 | 0.25 | 0.35 | 0.79 |
|  | 0+090-0+170 |  |  | 0.031 | 0.018 | 0.0167 | 0.24 | 0 | 0.12 | 0.06 | 1.09 | 0.031 | 0.000 | 0.25 | 0.37 | 0.84 |
|  | 0+170-0+290 |  |  | 0.031 | 0.018 | 0.0033 | 0.32 | 0 | 0.16 | 0.08 | 0.60 | 0.031 | 0.000 | 0.25 | 0.41 | 0.92 |
|  | 0+290-0+380 |  |  | 0.031 | 0.018 | 0.0500 | 0.19 | 0 | 0.10 | 0.05 | 1.65 | 0.031 | 0.000 | 0.25 | 0.35 | 0.79 |
|  | 0+380-0+440 |  |  | 0.031 | 0.018 | 0.0625 | 0.19 | 0 | 0.09 | 0.05 | 1.79 | 0.031 | 0.000 | 0.25 | 0.34 | 0.79 |
|  | 0+440-0+580 |  |  | 0.031 | 0.018 | 0.0333 | 0.21 | 0 | 0.10 | 0.05 | 1.42 | 0.031 | 0.000 | 0.25 | 0.35 | 0.81 |
| SC-4-LMC-1 | 0+000-0+180 | 27.40 | 1.20 | 0.033 | 0.018 | 0.0667 | 0.19 | 0 | 0.09 | 0.05 | 1.87 | 0.033 | 0.000 | 0.25 | 0.34 | 0.19 |
|  | 0+180-0+270 |  |  | 0.000 | 0.018 | 0.0833 | 0.18 | 0 | 0.09 | 0.05 | 2.04 | 0.033 | 0.033 | 0.25 | 0.34 | 0.18 |
| SC-1-RMC-1 | 0+000-0+090 | 21.50 | 1.20 | 0.026 | 0.018 | 0.0455 | 0.19 | 0 | 0.09 | 0.05 | 1.53 | 0.026 | 0.000 | 0.25 | 0.34 | 0.79 |
|  | 0+090-0+230 |  |  | 0.026 | 0.018 | 0.0400 | 0.19 | 0 | 0.09 | 0.05 | 1.45 | 0.026 | 0.000 | 0.25 | 0.35 | 0.79 |
|  | 0+230 -0+360 |  |  | 0.026 | 0.018 | 0.0500 | 0.19 | 0 | 0.09 | 0.05 | 1.60 | 0.027 | 0.002 | 0.25 | 0.34 | 0.79 |
|  | 0+360 -0+450 |  |  | 0.026 | 0.018 | 0.0833 | 0.30 | 0 | 0.05 | 0.04 | 1.77 | 0.026 | 0.000 | 0.25 | 0.30 | 0.90 |

Figure 4-1: Typical Cross Section of secondary canals

### Tertiary canals

In the layout system there are fourtty four (44) tertiary canals, the designed discharge is determined based on the duty of irrigation.  The sections of the canals are determined by using manning’s formula, and they are trapezoidal section. The hydraulic characteristics is Presented in Table 5.3 below.

Figure 4-2: Typical Cross Section of tertiary canals

Table 4-3: Hydraulic Parameters of tertiary canals

| Canal | Chainage (m) | Anet (ha) | 18hr Duty (l/s/ha) | Qreqd (m3/s) | n | s | b | m | d | R(m) | V (m/s) | Qcalcu | Qcalc -Qreqd | Fb(m) | D(m) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TC-LMC-1 | 0+000-0+039 | 1.67 | 1.20 | 0.002 | 0.025 | 0.0167 | 0.08 | 1 | 0.04 | 0.02 | 0.43 | 0.002 | 0.000 | 0.25 | 0.29 |
|  | 0+039 -0+189 |  |  | 0.000 | 0.025 | 0.0020 | 0.12 | 1 | 0.06 | 0.04 | 0.20 | 0.002 | 0.002 | 0.25 | 0.35 |
| TC-2-LMC-1 | 0+000-0+040 | 1.16 | 1.20 | 0.001 | 0.025 | 0.0167 | 0.07 | 1 | 0.03 | 0.02 | 0.40 | 0.001 | 0.000 | 0.25 | 0.28 |
|  | 0+040 -0+140 |  |  | 0.001 | 0.025 | 0.0020 | 0.12 | 1 | 0.06 | 0.04 | 0.19 | 0.002 | 0.001 | 0.25 | 0.35 |
| TC-3-LMC-1 | 0+000-0+038 | 3.51 | 1.20 | 0.004 | 0.025 | 0.0167 | 0.11 | 1 | 0.05 | 0.03 | 0.53 | 0.005 | 0.000 | 0.25 | 0.30 |
|  | 0+038 -0+278 |  |  | 0.004 | 0.025 | 0.0100 | 0.12 | 1 | 0.06 | 0.04 | 0.44 | 0.005 | 0.000 | 0.25 | 0.35 |
| TC-4-LMC-1 | 0+000-0+038 | 1.14 | 1.20 | 0.001 | 0.025 | 0.0167 | 0.07 | 1 | 0.04 | 0.02 | 0.41 | 0.002 | 0.000 | 0.25 | 0.29 |
|  | 0+038 -0+98 |  |  | 0.001 | 0.025 | 0.0100 | 0.09 | 1 | 0.05 | 0.03 | 0.37 | 0.002 | 0.001 | 0.25 | 0.30 |
| TC-5-LMC-1 | 0+000-0+028 | 2.36 | 1.20 | 0.003 | 0.025 | 0.0200 | 0.09 | 1 | 0.05 | 0.03 | 0.53 | 0.004 | 0.001 | 0.25 | 0.30 |
|  | 0+028 -0+228 |  |  | 0.003 | 0.025 | 0.0050 | 0.12 | 1 | 0.06 | 0.04 | 0.32 | 0.003 | 0.001 | 0.25 | 0.35 |
| TC-6-LMC-1 | 0+000-0+029 | 2.70 | 1.20 | 0.003 | 0.025 | 0.0500 | 0.08 | 1 | 0.04 | 0.02 | 0.76 | 0.004 | 0.000 | 0.25 | 0.29 |
|  | 0+029 -0+239 |  |  | 0.003 | 0.025 | 0.0033 | 0.12 | 1 | 0.06 | 0.04 | 0.26 | 0.003 | 0.000 | 0.25 | 0.35 |
| TC-7-LMC-1 | 0+000-0+029 | 8.41 | 1.20 | 0.010 | 0.025 | 0.0200 | 0.14 | 1 | 0.07 | 0.04 | 0.70 | 0.010 | 0.000 | 0.25 | 0.32 |
|  | 0+209-0+369 |  |  | 0.010 | 0.025 | 0.0125 | 0.16 | 1 | 0.08 | 0.05 | 0.60 | 0.011 | 0.001 | 0.25 | 0.33 |
|  | 0+029-0+209&0+379-0+479 |  |  | 0.010 | 0.025 | 0.0040 | 0.19 | 1 | 0.09 | 0.06 | 0.38 | 0.010 | 0.000 | 0.25 | 0.35 |
| TC-8-LMC-1 | 0+000-0+140 | 6.10 | 1.20 | 0.007 | 0.018 | 0.0714 | 0.11 | 0 | 0.05 | 0.03 | 1.32 | 0.007 | 0.000 | 0.25 | 0.30 |
|  | 0+140-0+300 |  |  | 0.007 | 0.025 | 0.0083 | 0.15 | 1 | 0.07 | 0.05 | 0.47 | 0.007 | 0.000 | 0.25 | 0.32 |
|  | 0+300-0+400 |  |  | 0.007 | 0.018 | 0.0556 | 0.11 | 0 | 0.06 | 0.03 | 1.21 | 0.008 | 0.000 | 0.25 | 0.35 |
| TC-9-LMC-1 | 0+000-0+50 | 2.31 | 1.20 | 0.003 | 0.025 | 0.0500 | 0.08 | 1 | 0.04 | 0.02 | 0.73 | 0.003 | 0.000 | 0.25 | 0.29 |
|  | 0+50-0+233 |  |  | 0.003 | 0.025 | 0.0100 | 0.11 | 1 | 0.05 | 0.03 | 0.41 | 0.003 | 0.001 | 0.25 | 0.30 |
| TC-10-LMC-1 | 0+000-0+29 | 3.65 | 1.20 | 0.004 | 0.018 | 0.0588 | 0.10 | 0 | 0.05 | 0.02 | 1.12 | 0.005 | 0.001 | 0.25 | 0.30 |
|  | 0+029-0+129 |  |  | 0.004 | 0.025 | 0.0033 | 0.15 | 1 | 0.08 | 0.05 | 0.30 | 0.005 | 0.001 | 0.25 | 0.33 |
|  | 0+129-0+279 |  |  | 0.004 | 0.025 | 0.0125 | 0.12 | 1 | 0.06 | 0.04 | 0.50 | 0.005 | 0.001 | 0.25 | 0.31 |
| TC-11-LMC-1 | 0+000-0+39 | 3.79 | 1.20 | 0.005 | 0.025 | 0.0333 | 0.10 | 1 | 0.05 | 0.03 | 0.72 | 0.005 | 0.001 | 0.25 | 0.30 |
|  | 0+039-0+129 |  |  | 0.005 | 0.025 | 0.0033 | 0.15 | 1 | 0.07 | 0.05 | 0.30 | 0.005 | 0.000 | 0.25 | 0.32 |
|  | 0+129-0+189 |  |  | 0.005 | 0.025 | 0.0250 | 0.10 | 1 | 0.05 | 0.03 | 0.64 | 0.005 | 0.000 | 0.25 | 0.30 |
|  | 0+189-0+299 |  |  | 0.005 | 0.025 | 0.0050 | 0.14 | 1 | 0.07 | 0.04 | 0.34 | 0.005 | 0.000 | 0.25 | 0.32 |
| TC-1-RMC-1 | 0+000-0+29 | 2.84 | 1.20 | 0.003 | 0.025 | 0.0400 | 0.08 | 1 | 0.04 | 0.02 | 0.68 | 0.003 | 0.000 | 0.25 | 0.29 |
|  | 0+029-0+159 |  |  | 0.003 | 0.025 | 0.0067 | 0.12 | 1 | 0.06 | 0.04 | 0.36 | 0.004 | 0.000 | 0.25 | 0.31 |
| TC-2-RMC-1 | 0+000-0+29 | 1.36 | 1.20 | 0.002 | 0.025 | 0.0500 | 0.06 | 1 | 0.03 | 0.02 | 0.64 | 0.002 | 0.000 | 0.25 | 0.28 |
|  | 0+029-0+239 |  |  | 0.002 | 0.025 | 0.0083 | 0.09 | 1 | 0.04 | 0.03 | 0.33 | 0.002 | 0.000 | 0.25 | 0.29 |
| TC-3-RMC-1 | 0+000-0+28 | 4.24 | 1.20 | 0.005 | 0.025 | 0.0500 | 0.10 | 1 | 0.05 | 0.03 | 0.86 | 0.006 | 0.001 | 0.25 | 0.30 |
|  | 0+028-0+208 |  |  | 0.005 | 0.025 | 0.0040 | 0.15 | 1 | 0.08 | 0.05 | 0.33 | 0.006 | 0.001 | 0.25 | 0.33 |
| TC-4-RMC-1 | 0+000-0+28 | 1.42 | 1.20 | 0.002 | 0.025 | 0.0500 | 0.07 | 1 | 0.03 | 0.02 | 0.68 | 0.002 | 0.001 | 0.25 | 0.28 |
|  | 0+028-0+218 |  |  | 0.002 | 0.025 | 0.0033 | 0.11 | 1 | 0.06 | 0.04 | 0.25 | 0.002 | 0.001 | 0.25 | 0.31 |
| TC-5-RMC-1 | 0+000-0+40 | 4.01 | 1.20 | 0.005 | 0.018 | 0.2000 | 0.08 | 0 | 0.04 | 0.02 | 1.79 | 0.005 | 0.001 | 0.25 | 0.29 |
|  | 0+040-0+120 |  |  | 0.005 | 0.018 | 0.0769 | 0.09 | 0 | 0.04 | 0.02 | 1.23 | 0.005 | 0.000 | 0.25 | 0.29 |
|  | 0+120-0+200 |  |  | 0.005 | 0.025 | 0.0167 | 0.12 | 1 | 0.06 | 0.04 | 0.56 | 0.006 | 0.001 | 0.25 | 0.31 |
| TC-6-RMC-1 | 0+000-0+109 | 4.21 | 1.20 | 0.005 | 0.025 | 0.0100 | 0.13 | 1 | 0.06 | 0.04 | 0.47 | 0.006 | 0.001 | 0.25 | 0.31 |
|  | 0+109 -0+369 |  |  |  | 0.025 | 0.0020 | 0.30 | 1 | 0.07 | 0.05 | 0.24 | 0.006 | 0.001 | 0.25 | 0.35 |
| TC-7-RMC-1 | 0+000-0+090 | 8.95 | 1.20 | 0.011 | 0.018 | 0.0714 | 0.12 | 0 | 0.06 | 0.03 | 1.45 | 0.011 | 0.000 | 0.25 | 0.31 |
|  | 0+090-0+160 |  |  | 0.011 | 0.018 | 0.0625 | 0.13 | 0 | 0.06 | 0.03 | 1.41 | 0.012 | 0.001 | 0.25 | 0.31 |
|  | 0+160 -0+270 |  |  | 0.011 | 0.018 | 0.0455 | 0.14 | 0 | 0.07 | 0.03 | 1.24 | 0.011 | 0.001 | 0.25 | 0.32 |
| TC-1-1-RMC-1 | 0+000-0+200 | 1.85 | 1.20 | 0.002 | 0.025 | 0.0040 | 0.12 | 1 | 0.06 | 0.04 | 0.28 | 0.003 | 0.001 | 0.25 | 0.31 |
| TC-2-1-RMC-1 | 0+000-0+330 | 2.89 | 1.20 | 0.003 | 0.025 | 0.0033 | 0.13 | 1 | 0.07 | 0.04 | 0.28 | 0.004 | 0.000 | 0.25 | 0.32 |
| TC-3-1-RMC-1 | 0+000-0+80 | 1.50 | 1.20 | 0.002 | 0.025 | 0.0050 | 0.10 | 1 | 0.05 | 0.03 | 0.28 | 0.002 | 0.000 | 0.25 | 0.30 |
|  | 0+80-0+160 |  |  | 0.002 | 0.025 | 0.0182 | 0.08 | 1 | 0.04 | 0.02 | 0.46 | 0.002 | 0.000 | 0.25 | 0.29 |
| TC-4-1-RMC-1 | 0+000-0+220 | 5.43 | 1.20 | 0.007 | 0.025 | 0.0050 | 0.16 | 1 | 0.08 | 0.05 | 0.38 | 0.007 | 0.001 | 0.25 | 0.33 |
|  | 0+220-0+370 |  |  | 0.007 | 0.025 | 0.0222 | 0.12 | 1 | 0.06 | 0.04 | 0.65 | 0.007 | 0.000 | 0.25 | 0.31 |
| TC-5-1-RMC-1 | 0+000-0+130 | 5.07 | 1.20 | 0.006 | 0.025 | 0.0050 | 0.16 | 1 | 0.08 | 0.05 | 0.37 | 0.007 | 0.001 | 0.25 | 0.33 |
| TC-6-1-RMC-1 | 0+000-0+120 | 2.31 | 1.20 | 0.003 | 0.025 | 0.0071 | 0.10 | 1 | 0.05 | 0.03 | 0.34 | 0.003 | 0.000 | 0.25 | 0.30 |
| TC-1-1-LMC-1 | 0+000-0+063 | 0.76 | 1.20 | 0.001 | 0.025 | 0.0125 | 0.07 | 1 | 0.03 | 0.02 | 0.35 | 0.001 | 0.000 | 0.25 | 0.28 |
| TC-2-1-LMC-1 | 0+000-0+099 | 1.38 | 1.20 | 0.002 | 0.025 | 0.0100 | 0.09 | 1 | 0.04 | 0.03 | 0.37 | 0.002 | 0.001 | 0.25 | 0.29 |
| TC-3-1-LMC-1 | 0+000-0+130 | 1.14 | 1.20 | 0.001 | 0.025 | 0.0250 | 0.07 | 1 | 0.03 | 0.02 | 0.49 | 0.002 | 0.000 | 0.25 | 0.28 |
| TC-4-1-LMC-1 | 0+000-0+170 | 1.35 | 1.20 | 0.002 | 0.025 | 0.0100 | 0.07 | 1 | 0.03 | 0.02 | 0.31 | 0.001 | 0.000 | 0.25 | 0.28 |
| TC-1-2-LMC-1 | 0+000-0+081 | 2.86 | 1.20 | 0.003 | 0.025 | 0.0125 | 0.07 | 1 | 0.03 | 0.02 | 0.35 | 0.001 | -0.002 | 0.25 | 0.28 |
| TC-2-2-LMC-1 | 0+000-0+160 | 1.99 | 1.20 | 0.002 | 0.025 | 0.0067 | 0.10 | 1 | 0.05 | 0.03 | 0.33 | 0.003 | 0.000 | 0.25 | 0.30 |
| TC-3-2-LMC-1 | 0+000-0+270 | 4.64 | 1.20 | 0.006 | 0.025 | 0.0100 | 0.13 | 1 | 0.07 | 0.04 | 0.47 | 0.006 | 0.000 | 0.25 | 0.32 |
| TC-1-3-LMC-1 | 0+000-0+090 | 1.18 | 1.20 | 0.001 | 0.025 | 0.0083 | 0.09 | 1 | 0.04 | 0.03 | 0.33 | 0.002 | 0.001 | 0.25 | 0.29 |
| TC-2-3-LMC-1 | 0+000-0+080 | 1.55 | 1.20 | 0.002 | 0.025 | 0.0050 | 0.11 | 1 | 0.06 | 0.03 | 0.30 | 0.003 | 0.001 | 0.25 | 0.31 |
| TC-3-3-LMC-1 | 0+000-0+050 | 3.29 | 1.20 | 0.004 | 0.025 | 0.0200 | 0.10 | 1 | 0.05 | 0.03 | 0.57 | 0.004 | 0.000 | 0.25 | 0.30 |
|  | 0+050-0+0140 |  |  | 0.004 | 0.025 | 0.0500 | 0.08 | 1 | 0.04 | 0.03 | 0.79 | 0.004 | 0.000 | 0.25 | 0.29 |
| TC-4-3-LMC-1 | 0+000-0+070 | 3.93 | 1.20 | 0.005 | 0.025 | 0.0100 | 0.12 | 1 | 0.06 | 0.04 | 0.46 | 0.005 | 0.001 | 0.25 | 0.31 |
|  | 0+070-0+170 |  |  | 0.005 | 0.025 | 0.0500 | 0.09 | 1 | 0.04 | 0.03 | 0.82 | 0.005 | 0.000 | 0.25 | 0.29 |
| TC-5-3-LMC-1 | 0+000-0+160 | 7.70 | 1.20 | 0.009 | 0.025 | 0.0040 | 0.18 | 1 | 0.09 | 0.06 | 0.37 | 0.009 | 0.000 | 0.25 | 0.34 |
|  | 0+160-0+230 |  |  | 0.009 | 0.025 | 0.0250 | 0.13 | 1 | 0.07 | 0.04 | 0.75 | 0.010 | 0.001 | 0.25 | 0.32 |
|  | 0+230-0+330 |  |  | 0.009 | 0.025 | 0.0100 | 0.16 | 1 | 0.08 | 0.05 | 0.54 | 0.010 | 0.001 | 0.25 | 0.33 |
| TC-6-3-LMC-1 | 0+000-0+240 | 2.80 | 1.20 | 0.003 | 0.025 | 0.0050 | 0.12 | 1 | 0.06 | 0.04 | 0.32 | 0.004 | 0.000 | 0.25 | 0.31 |
| TC-7-3-LMC-1 | 0+000-0+200 | 2.80 | 1.20 | 0.003 | 0.025 | 0.0125 | 0.10 | 1 | 0.05 | 0.03 | 0.45 | 0.004 | 0.000 | 0.25 | 0.30 |
| TC-1-4-LMC-1 | 0+000-0+120 | 1.53 | 1.20 | 0.002 | 0.025 | 0.0033 | 0.11 | 1 | 0.05 | 0.03 | 0.24 | 0.002 | 0.000 | 0.25 | 0.30 |
| TC-2-4-LMC-1 | 0+000-0+050 | 5.91 | 1.20 | 0.007 | 0.025 | 0.0200 | 0.13 | 1 | 0.06 | 0.04 | 0.65 | 0.008 | 0.001 | 0.25 | 0.31 |
|  | 0+050-0+150 |  |  | 0.007 | 0.025 | 0.0083 | 0.14 | 1 | 0.07 | 0.04 | 0.46 | 0.007 | 0.000 | 0.25 | 0.32 |
|  | 0+150-0+420 |  |  | 0.007 | 0.025 | 0.0040 | 0.17 | 1 | 0.08 | 0.05 | 0.35 | 0.007 | 0.000 | 0.25 | 0.33 |
| TC-3-4-LMC-1 | 0+000-0+190 | 2.11 | 1.20 | 0.003 | 0.025 | 0.0143 | 0.09 | 1 | 0.05 | 0.03 | 0.46 | 0.003 | 0.001 | 0.25 | 0.30 |
| TC-4-4-LMC-1 | 0+000-0+040 | 2.11 | 1.20 | 0.003 | 0.025 | 0.0200 | 0.09 | 1 | 0.05 | 0.03 | 0.53 | 0.003 | 0.001 | 0.25 | 0.30 |
|  | 0+040-0+260 |  |  | 0.003 | 0.025 | 0.0033 | 0.12 | 1 | 0.06 | 0.04 | 0.26 | 0.003 | 0.000 | 0.25 | 0.31 |
| TC-5-4-LMC-1 | 0+000-0+140 | 5.98 | 1.20 | 0.007 | 0.025 | 0.0125 | 0.14 | 1 | 0.07 | 0.04 | 0.54 | 0.008 | 0.000 | 0.25 | 0.32 |
|  | 0+140-0+290 |  |  | 0.007 | 0.025 | 0.0040 | 0.17 | 1 | 0.08 | 0.05 | 0.35 | 0.007 | 0.000 | 0.25 | 0.33 |
| TC-6-4-LMC-1 | 0+000-0+210 | 8.88 | 1.20 | 0.011 | 0.025 | 0.0083 | 0.17 | 1 | 0.08 | 0.05 | 0.51 | 0.011 | 0.000 | 0.25 | 0.33 |
|  | 0+210-0+490 |  |  | 0.011 | 0.025 | 0.0050 | 0.19 | 1 | 0.09 | 0.06 | 0.42 | 0.011 | 0.000 | 0.25 | 0.34 |

### Field Canals

As shown in the layout, field canals run across the contours and hence face relatively steeper gradient. The discharge of most of the field canals is very small and this is taken as an advantage to cope up with the relatively steeper gradient. Figure 15 below shows a typical field canal x-section. As much as possible field canals shall be made in fill in order to easily irrigate the adjacent command area. As can be seen from the layout, majority of the filed canals can be used to irrigate both sides of the command area depending on the condition of the individual plots of land owned by individual farmers.

Figure 4-3: Typical Field Canal X-section

## Drainage Canals

### Catch drainage canals

In the layout system there are  Catch drain canals, the designed discharge is determined based on the Catchment area, run off coefficient and rain fall intensity (Rational formula).  The sections of the canals are determined by using manning’s formula, and they are trapezoidal section.

Figure 4-4: Typical Cross Section of Catch drain canals

### Tertiary canal drainage canals

Drainage Canals aren't provided for the Tertiary canals, because all of the tertiary canals are entered to gullies and also there is shortage of irrigation water. It is assumed that there is no excess irrigation water during dry season. For the rainy season the water that comes through the tertiary canals directly entered to gullies. So, Gully protection is necessary at the tertiary canal out fall. For the protection gabion is recommended.

## Canal Structures Design

### Design of a typical flume

Hydraulic Characteristics of the canal

Flume-1

Length of the flume: 56m

Shape of the flume:  Rectangular, Roughness coefficient, n =0.016

From the canal longitudinal profiles, u/s canal bed level (CBL) = 2588.113m

D/s Canal bed level (CBL) =2587.739m

U/s full supply level (FSL) = 2588.446m

D/s Full Supply Level = 2588.073m

Total head loss between the inlet & Outlet =0.327m

=0.305m

The hydraulic parameters of the flumes and the supper passages are shown below

Table 4-5: Hydraulic Parameters of Flume and supper passages

| Name | Location | Chain age | Q | V | Length,m | Bed width,B | Wall hight,m |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Flume-1 | LMC | 0+10-0+66 | 0.132251 | 1.095 | 56 | 0.35 | 0.6 |
| Flume-2 |  | 0+1053-0+1082 | 0.132251 | 1.095 | 29 | 0.35 | 0.6 |
| Flume-3 |  | 0+1250-0+1270 | 0.132251 | 1.095 | 15 | 0.35 | 0.6 |
| Flume-4 |  | 0+1445-0+1469 | 0.132251 | 1.095 | 24 | 0.35 | 0.6 |
| Flume-5 |  | 0+2522-0+2545 | 0.132251 | 1.095 | 23 | 0.35 | 0.6 |
| Flume-6 |  | 0+3428-0+3458 | 0.132251 | 1.095 | 30 | 0.35 | 0.6 |
| Flume-7 | RMC | 0+204-0+216 | 0.188 | 1.195 | 12 | 0.45 | 0.7 |
| Flume-8 |  | 0+800-0+830 | 0.0708 | 0.940 | 30 | 0.3 | 0.55 |
| Flume-9 |  | 0+955-0+976 | 0.0708 | 0.940 | 21 | 0.3 | 0.55 |
| Flume-10 |  | 0+1116-0+1123 | 0.0708 | 0.940 | 7 | 0.3 | 0.55 |
| Flume-11 |  | 0+1651-0+1730 | 0.0708 | 1.135 | 79 | 0.3 | 0.55 |
| Flume-12 |  | 0+2088-0+2105 | 0.0708 | 1.135 | 17 | 0.3 | 0.55 |
| Flume-13 |  | 0+2225-0+2237 | 0.0708 | 1.135 | 12 | 0.3 | 0.55 |
| Flume-14 |  | 0+2567-0+2593 | 0.0708 | 1.135 | 26 | 0.3 | 0.55 |
| Super passage | LMC | 0+770-0+780 | 0.132 | 0.55 | 10 | 0.7 | 0.6 |
|  |  | 0+1200-0+1205 | 0.132 | 0.55 | 5 | 0.7 | 0.6 |
|  |  | 0+1490-0+1500 | 0.132 | 0.55 | 10 | 0.7 | 0.6 |
|  |  | 0+2200-0+2205 | 0.132 | 0.55 | 5 | 0.7 | 0.6 |
|  |  | 0+2400-0+2405 | 0.132 | 0.55 | 5 | 0.7 | 0.6 |
|  |  | 0+2720-0+2725 | 0.132 | 0.55 | 5 | 0.7 | 0.6 |
|  |  | 0+3000-0+3010 | 0.132 | 0.55 | 10 | 0.7 | 0.6 |
|  |  | 0+3110-0+3115 | 0.132 | 0.71 | 5 | 0.6 | 0.6 |
|  |  | 0+3250-0+3255 | 0.132 | 0.71 | 5 | 0.6 | 0.6 |
|  |  | 0+3370-0+3380 | 0.132 | 0.71 | 10 | 0.6 | 0.6 |
|  |  | 0+3980-0+3990 | 0.132 | 0.71 | 10 | 0.6 | 0.6 |
|  |  | 0+4240-0+4245 | 0.132 | 0.71 | 5 | 0.6 | 0.6 |
|  |  | 0+4340-0+4345 | 0.132 | 0.71 | 5 | 0.6 | 0.6 |
|  |  | 0+4880-0+4885 | 0.132 | 0.71 | 5 | 0.6 | 0.6 |
|  |  | 0+4920-0+4925 | 0.132 | 0.71 | 5 | 0.6 | 0.6 |
|  | RMC | 0+260-0+265 | 0.055 | 0.53 | 5 | 0.5 | 0.5 |
|  |  | 0+470-0+475 | 0.055 | 0.53 | 5 | 0.5 | 0.5 |
|  |  | 0+560-0+565 | 0.055 | 0.53 | 5 | 0.5 | 0.5 |
|  |  | 0+580-0+585 | 0.055 | 0.53 | 5 | 0.5 | 0.5 |
|  |  | 0+730-0+735 | 0.055 | 0.53 | 5 | 0.5 | 0.5 |
|  |  | 0+1060-0+1065 | 0.055 | 0.53 | 5 | 0.5 | 0.5 |
|  |  | 0+1150-0+1155 | 0.055 | 0.53 | 5 | 0.5 | 0.5 |
|  |  | 0+1370-0+1375 | 0.055 | 0.53 | 5 | 0.5 | 0.5 |
|  |  | 0+1780-0+1785 | 0.055 | 0.53 | 5 | 0.5 | 0.5 |
|  |  | 0+1870-0+1875 | 0.055 | 0.53 | 5 | 0.5 | 0.5 |
|  |  | 0+2330-0+2335 | 0.055 | 0.53 | 5 | 0.5 | 0.5 |
|  |  | 0+2540-0+2545 | 0.055 | 0.53 | 5 | 0.5 | 0.5 |
|  |  | 0+2650-0+2655 | 0.055 | 0.53 | 5 | 0.5 | 0.5 |
|  |  | 0+2790-0+2800 | 0.055 | 0.53 | 10 | 0.5 | 0.5 |
|  |  | 0+2910-0+2920 | 0.055 | 0.53 | 10 | 0.5 | 0.5 |
|  |  | 0+2980-0+2985 | 0.055 | 0.53 | 5 | 0.5 | 0.5 |
|  |  | 0+3040-0+3045 | 0.055 | 0.53 | 5 | 0.5 | 0.5 |
|  |  | 0+3150-0+3160 | 0.055 | 0.53 | 10 | 0.5 | 0.5 |

### Design of Division box

At different points of the main and secondary canals division boxes are provided which divert the flow to the secondary canal and tertiary canals. Gate should be provided at the outlet of the boxes. For detail refer the drawing.

Figure 4-5: Typical Division Box plan

Using broad crested formula,

Q= CL (h)3/2

Where; Q= discharge over rectangular weir/intake (opening), m3/s

C = discharge coefficient, c= 1.7

L= effective length of crest form in m

h= over flow depth, m

Assuming equal discharge coefficient & sill height for two or three dividing canals, the proportion becomes.

Q1/ Q2= Q2/ Q3 =L1/ L2 = L2/ L3

Where Q1= is flow in canal 1

Q2 = is flow in canal 2

Q3 = is flow in canal 3

L1= is effective crest length of weir/intake sill across opening to canal 1

L2= is effective crest length of weir/intake sill across opening to canal 2

L3 = is effective crest length of weir/intake sill across opening to canal 3

Q1= CL1 (h)3/2,

L1 = Q1/Ch3/2

L2 = L1*Q2/ Q1

L3 = L1*Q3/ Q1

The depth of (height of) the division box,

D = d + fb

The width of the division box,

B = b + 2*m*D

Where b= base width of the incoming canal

D = total canal depth of the incoming canal

Table 4-6: Hydraulic parameters of Division Boxes on MC

| Dividing canal Name | Chainage | Incomming flow(lit/sec) | Outgoing flow (lit/sec) | Discharge to Tc or Sc (lit/sec) | MC bed width for ongoing discharge (m) | TC/SC outlet width | Top width of Division box, m | Depth of division box, m |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| MCR & MCL | 0+240 | 188 | 132 | 55 | 0.8 | 0.55 | 1.60 | 0.65 |
| MCL & SC-1-LMC-1 | 0+1340 | 132 | 132 | 6 | 0.7 | 0.3 | 1.50 | 0.60 |
| MCL & SC-2-LMC-1 | 0+2099 | 132 | 132 | 12 | 0.7 | 0.3 | 1.50 | 0.60 |
| MCL & SC-3-LMC-1 | 0+2629 | 132 | 132 | 31 | 0.7 | 0.3 | 1.50 | 0.60 |
| MCL & SC-4-LMC-1 | 0+5322 | 132 | 132 | 33 | 0.7 | 0.3 | 1.50 | 0.60 |
| MCR & SC-1-RMC-1 | 0+3930 | 73 | 73 | 26 | 0.5 | 0.3 | 1.30 | 0.50 |
| MC L & TC-1-LMC-1 | 0+120 | 132 | 132 | 2 | 0.70 | 0.30 | 1.50 | 0.60 |
| MC L & TC-2-LMC-1 | 0+620 | 132 | 132 | 1 | 0.70 | 0.30 | 1.50 | 0.60 |
| MC L & TC-3-LMC-1 | 0+780 | 132 | 132 | 4 | 0.70 | 0.30 | 1.50 | 0.60 |
| MC L & TC-4-LMC-1 | 0+1100 | 132 | 132 | 1 | 0.70 | 0.30 | 1.50 | 0.60 |
| MC L & TC-5-LMC-1 | 0+2738 | 132 | 132 | 3 | 0.70 | 0.30 | 1.50 | 0.60 |
| MC L & TC-6-LMC-1 | 0+3128 | 132 | 132 | 3 | 0.65 | 0.30 | 1.45 | 0.60 |
| MC L & TC-7&8-LMC-1 | 0+3468 | 132 | 132 | 17 | 0.65 | 0.30 | 1.45 | 0.60 |
| MC L & TC-9-LMC-1 | 0+3998 | 132 | 132 | 3 | 0.65 | 0.30 | 1.45 | 0.60 |
| MC L & TC-10-LMC-1 | 0+4518 | 132 | 132 | 4 | 0.65 | 0.30 | 1.45 | 0.60 |
| MC L & TC-11-LMC-1 | 0+4818 | 132 | 132 | 5 | 0.65 | 0.30 | 1.45 | 0.60 |
| MC R & TC-1-RMC-1 | 0+590 | 55 | 55 | 3 | 0.50 | 0.30 | 1.35 | 0.50 |
| MC R & TC-2-RMC-1 | 0+1379 | 55 | 55 | 2 | 0.50 | 0.30 | 1.30 | 0.50 |
| MC R & TC-3-RMC-1 | 0+1890 | 55 | 55 | 5 | 0.50 | 0.30 | 1.30 | 0.50 |
| MC R & TC-4-RMC-1 | 0+2349 | 55 | 55 | 2 | 0.50 | 0.30 | 1.30 | 0.50 |
| MC R & TC-5-RMC-1 | 0+2660 | 55 | 55 | 5 | 0.50 | 0.30 | 1.30 | 0.50 |
| MC R & TC-6&7-RMC-1 | 0+3160 | 55 | 55 | 17 | 0.50 | 0.30 | 1.30 | 0.50 |

Table 4-7: Hydraulic parameters of Division Boxes on SC

| Dividing canal Name | Chain age | Incoming flow(lit/sec) ,Q0 | Outgoing flow (lit/sec) ,Q1 | Discharge to Tc (lit/sec),Q2 | Discharge to Tc (lit/sec),Q3 | SC bed width for ongoing discharge (m) | TC out let width | Top width of Division box, m | Depth of division box, m |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SC-1-RMC-1 & TC-1&2-1-RMC-1 | 0+10 | 26 | 26 | 2 | 3 | 0.30 | 0.3 | 1.10 | 0.30 |
| SC-1-RMC-1 & TC-3&4-1-RMC-1 | 0+160 | 26 | 26 | 2 | 7 | 0.30 | 0.3 | 1.10 | 0.30 |
| SC-1-RMC-1 & TC-5&6-1-RMC-1 | 0+340 | 26 | 26 | 6 | 3 | 0.30 | 0.3 | 1.10 | 0.30 |
| SC-1-LMC-1 & TC-1&2-1-LMC-1 | 0+10 | 6 | 6 | 1 | 2 | 0.30 | 0.3 | 1.10 | 0.30 |
| SC-1-LMC-1 & TC-3&4-1-LMC-1 | 0+110 | 6 | 6 | 1 | 2 | 0.30 | 0.3 | 1.10 | 0.30 |
| SC-2-LMC-1 & TC-1-2-LMC-1 | 0+10 | 12 | 12 | 3 | 0 | 0.30 | 0.3 | 1.10 | 0.30 |
| SC-2-LMC-1 & TC-2-2-LMC-1 | 0+130 | 12 | 12 | 2 | 0 | 0.30 | 0.3 | 1.10 | 0.30 |
| SC-2-LMC-1 & TC-3-2-LMC-1 | 0+250 | 12 | 12 | 6 | 0 | 0.30 | 0.3 | 1.10 | 0.30 |
| SC-3-LMC-1 & TC-1&2-3-LMC-1 | 0+10 | 31 | 31 | 1 | 2 | 0.30 | 0.3 | 1.10 | 0.30 |
| SC-3-LMC-1 & TC-3&4-3-LMC-1 | 0+140 | 31 | 31 | 4 | 5 | 0.30 | 0.3 | 1.10 | 0.30 |
| SC-3-LMC-1 & TC-5-3-LMC-1 | 0+340 | 31 | 31 | 9 | 0 | 0.30 | 0.3 | 1.10 | 0.30 |
| SC-3-LMC-1 & TC-6-3-LMC-1 | 0+430 | 31 | 31 | 3 | 0 | 0.30 | 0.3 | 1.10 | 0.30 |
| SC-3-LMC-1 & TC-7-3-LMC-1 | 0+580 | 31 | 31 | 3 | 1 | 0.30 | 0.3 | 1.10 | 0.30 |
| SC-3-LMC-1 & TC-1&2-4-LMC-1 | 0+10 | 33 | 33 | 2 | 7 | 0.30 | 0.3 | 1.10 | 0.30 |
| SC-3-LMC-1 & TC-3&4-4-LMC-1 | 0+150 | 33 | 33 | 5 | 3 | 0.30 | 0.3 | 1.10 | 0.30 |
| SC-3-LMC-1 & TC-5&6-4-LMC-1 | 0+270 | 33 | 33 | 7 | 11 | 0.30 | 0.3 | 1.10 | 0.30 |

### Design of Drop structures

At different points of the main canals Drops are provided. For detail refer the drawing.

Table 4-8: Hydraulic parameters of Drop Structures on SC

| Drop on | Chain age | Q | V | B | H | d | D, canal depth | h | t | a |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (m3/s) | (m/s) | (m) | (m) | (m) |  | (m) | (m) | (m) |
| MCL- 1 | 0+3668 | 0.132 | 0.71 | 0.65 | 0.33 | 0.31 | 0.60 | 0.5 | 0.4 | 0.21 |
|  | 0+3758 | 0.132 | 0.71 | 0.65 | 0.33 | 0.31 | 0.60 | 0.8 | 0.4 | 0.27 |
|  | 0+3857 | 0.132 | 0.71 | 0.65 | 0.33 | 0.31 | 0.60 | 0.5 | 0.4 | 0.21 |
|  | 0+4338 | 0.132 | 0.71 | 0.65 | 0.33 | 0.31 | 0.60 | 1 | 0.4 | 0.30 |
|  | 0+4568 | 0.132 | 0.71 | 0.65 | 0.33 | 0.31 | 0.60 | 0.5 | 0.4 | 0.21 |
| MCR-1 | 0+1229 | 0.055 | 0.53 | 0.5 | 0.29 | 0.23 | 0.5 | 0.5 | 0.4 | 0.19 |

### Road crossing structure

Road crossing structures are provided on the main canals. These structures are foot paths and Cattle crossings. On Right side main Canal at chain age 0+240,0+560,0+680,0+700,0+800,0+1120,0+1300,0+2600 and on the left side at Chain age 0+1240,0+1980,0+2700,0+3200,0+3560,0+3400,0+4680,there are cattle crossing structures and foot paths. So that 15 foot paths are available. Both of the crossing structures are constructed using slab cover on the main canal. The road crossing structures are rectangular reinforced concrete slabs. The slab is reinforced with 12mm @200mmc/c the length of the slab is 1.2m which is the same as the respective canal bed width, its width and thickness is 3m and 15mm respectively.

## Irrigation Infrastructure Bill of Quantities and Cost Estimate

| S.N | ITEM OF WORK | UNITE | QUANTITY | UNITE PRICE | AMOUNT (ETH. BIRR) |
| --- | --- | --- | --- | --- | --- |
|  | A. CANALS |  |  |  |  |
| 1 | MAIN CANAL |  |  |  |  |
| 1.1 | Site clearing | m2 | 2100 | 60.47 | 126,983.88 |
| 1.2 | Ordinary Soil excavation | m3 | 12,554.67 | 65.00 | 816,053.57 |
| 1.3 | highly weathered rocks | m3 | 0.00 | 150.00 | 0.00 |
| 1.4 | Back fill and compaction by excavated material | m3 | 5,377.94 | 72.62 | 390,524.71 |
| 1.5 | Back fill and compaction by selected material | m3 | 70.312 | 72.17 | 5,074.63 |
| 1.6 | Masonry(1:3) | m3 | 4,831.35 | 2,009.16 | 9,706,944.88 |
| 1.7 | Plastering(1:3) | m2 | 22,634.40 | 156.58 | 3,544,071.10 |
| Subtotal |  |  |  |  | 14,589,652.77 |
| 2 | SECONDARY CANAL |  |  |  |  |
| 2.1 | Ordinary Soil excavation | m3 | 1,187.23 | 65.00 | 77,169.95 |
| 2.2 | highly weathered rocks | m3 | 0 | 150.00 | 0.00 |
| 2.3 | Back fill and compaction by excavated material | m3 | 329.34 | 72.17 | 23,769.47 |
| 2.4 | Masonry (1:3) | m3 | 530.46 | 2,009.16 | 1,065,778.10 |
| 2.5 | Plastering(1:3) | m2 | 3,132.00 | 156.58 | 490,405.34 |
| Subtotal |  |  |  |  | 1,657,122.86 |
|  | TERTIARY CANALS |  |  |  |  |
| 3.1 | Ordinary Soil excavation | m3 | 2,286.22 | 65.00 | 148,604.55 |
| 3.2 | Back fill and compaction by excavated material | m3 | 2,157.91 | 72.17 | 155,742.92 |
| 3.3 | Masonry (1:3) | m3 | 139.5 | 2,009.16 | 280,277.58 |
| 3.4 | Plastering(1:3) | m2 | 465 | 156.58 | 72,809.22 |
| Subtotal |  |  |  |  | 657,434.27 |
|  | B. Division Box&Turn out |  |  |  |  |
| 5 | Division & Turn out work |  |  |  |  |
| 5.1 | Ordinary Soil excavation | m3 | 136.9 | 65.00 | 8,898.50 |
| 5.2 | Masonry (1:3) | m3 | 58.70626066 | 2,009.16 | 117,950.17 |
| 5.3 | Back fill and compaction by excavated material | m3 | 78.19373934 | 72.17 | 5,643.48 |
| 5.4 | Plastering(1:3) | m2 | 198.7874155 | 156.58 | 31,125.93 |
| 5.5 | Stone pitching | m3 | 76.73107224 | 194.54 | 14,927.54 |
| 5.6 | Gate |  |  |  |  |
|  | 4mm thick sheet metal | PCS | 56 | 1,800.00 | 100,800.00 |
|  | Stiffening angle iron (30*30*4) |  |  |  |  |
|  | Angle iron for groove(40*40*4) |  |  |  |  |
|  | 16mm reinforcement bar for handling |  |  |  |  |
| Sub total |  |  |  |  | 279,345.62 |
| 6 | C. Drop structure |  |  |  |  |
|  | Drop |  |  |  |  |
| 6.1 | Ordinary Soil excavation | m3 | 57.7 | 65.00 | 3,750.50 |
| 6.2 | Masonry (1:3) | m3 | 26.58107124 | 2,009.16 | 53,405.58 |
| 6.3 | Back fill and compaction by excavated material | m3 | 31.11892876 | 72.17 | 2,245.95 |
| 6.4 | Plastering(1:3) | m2 | 74.3126996 | 156.58 | 11,635.81 |
| 6.5 | Stone pitching | m3 | 45.18 | 194.54 | 8,789.83 |
| Subtotal |  |  |  |  | 79,827.66 |
| 7 | D.FLUMES |  |  |  |  |
| 7.1 | Ordinary Soil excavation | m3 | 531.76 | 65.00 | 34,564.40 |
| 7.2 | highly weathered rocks |  | 265.88 | 150.00 | 39,882.00 |
| 7.3 | Hard basalt rocks |  | 67.82 | 200.00 | 13,564.00 |
| 7.4 | Back fill and compaction by excavated material | m3 | 324.7 | 72.17 | 23,434.59 |
| 7.5 | Masonry(1:3) | m3 | 207.06 | 2,009.16 | 416,016.31 |
| 7.6 | Plastering(1:3) | m2 | 394.4 | 156.58 | 61,754.75 |
| 7.7 | C-20 concrete (1:2:3) | m3 | 226.338 | 2,981.49 | 674,824.22 |
| 7.8 | Lean concrete C-10 (1:2:4) | m3 | 51.272 | 1,942.89 | 99,616.06 |
| 7.9 | Reinforcement |  |  |  | 0.00 |
| 7.91 | ф8mm bar | Kg | 675.8164224 | 38.50 | 26,020.41 |
| 7.9.2 | ф10mm bar | Kg | 342.004875 | 38.50 | 13,167.94 |
| 7.9.3 | ф12mm bar | Kg | 132.7496544 | 38.50 | 5,111.15 |
| 7.9.4 | ф14mm bar | Kg | 12442.7659 | 38.50 | 479,073.70 |
| 7.9.5 | ф16mm bar | Kg | 474.7545294 | 38.50 | 18,279.09 |
| Subtotal |  |  |  |  | 1,905,308.62 |
| 8 | D.SUPER PASSAGES |  |  |  |  |
| 8.1 | Ordinary Soil excavation | m3 | 195.9 | 65.00 | 12,733.50 |
| 8.2 | Back fill and compaction by excavated material | m3 | 379.5 | 72.17 | 27,389.67 |
| 8.3 | Masonry(1:3) | m3 | 768.24 | 2,009.16 | 1,543,515.75 |
| 8.4 | Plastering(1:3) | m2 | 1785.3 | 156.58 | 279,540.44 |
| 8.5 | C-20 concrete work | m3 | 64.35 | 2,981.49 | 191,858.81 |
|  | Reinforcement |  |  |  |  |
| 8.6 | ф10mm bar | Kg | 677.8475 | 38.50 | 26,098.61 |
| 8.7 | ф12mm bar | Kg | 862.8727536 | 38.50 | 33,222.49 |
| 9 | E.Road crossing slab |  |  |  |  |
| 9.1 | Excavation/soil | m3 | 120 | 77.96 | 9355.2 |
| 9.2 | massonry(1:3) | m3 | 37.632 | 2,009.16 | 75608.64396 |
| 9.3 | Back fill and compaction by excavated material | m3 | 82.368 | 72.17 | 5944.749575 |
| 9.4 | Plastering(1:3) | m2 | 96 | 156.58 | 15031.58138 |
| 9.5 | Concrete C-20(1:2:3) | m3 | 6.6 | 2,981.49 | 19677.82641 |
| 9.6 | Reinforcement |  |  |  |  |
| 9.6.1 | Reinforcement bar, ф 14mm | Kg | 518.146629 | 38.50 | 19,949.78 |
| 9.6.2 | Reinforcement bar, ф 12mm | Kg | 281.1169152 | 38.50 | 10,823.62 |
| Subtotal |  |  |  |  | 2,270,750.66 |
| Grand Total |  |  |  |  | 21,439,442.46 |

# CONCLUSION AND RECOMMENDATION

- The infrastructure of this project area is designed to irrigate about 156 ha of land by taking its supply from the Azuwari diversion weir irrigation project. The maximum duty of the command area for 18hours per day irrigation with overall project efficiency of 50% is 1.2 l/s/ha.  The method of irrigation of the project area is furrow surface irrigation in which the main and tertiary canals are working continuously where as the field canals within a tertiary block are working rotational system.

- As the dominant soil type is clay soil, the main canal system is designed to be masonry.

- The reason why the main canal is to be lined up to the end is to avoid the siltation problem, time saving to reach at the tail part, reduce maintenance cost.

- On the secondary unit of the irrigation systems, some are associated with chute. They are designed as far as possible to be partially filled and cut. The layout is designed as far as possible to avoid cross-structures within them.

- The design of the canal dimensions of the irrigation canal is done by applying the manning’s uniform flow equation. The variable of the hydraulic parameters are calculated using iteration or flow master program.

- As soils of the command area are predominantly clay textured; and hence water and soil management measures should be undertaken; and optimum moisture content should be maintained to improve workability of the soil during land preparation and planting time.

The following recommendations are drown:

- For better performance and long service year of the project regular inspection and maintenance is highly required.

- Farmers training, how to operate and maintain the project structures as a whole and available and water resources has a paramount important.

- The irrigation hours per day and per week should be flexible based on base flow amount of each week or month.

- Close supervision of the construction should be made to modify (if need be) each Components of irrigation system based on specific site conditions.

# OPERATION AND MAINTENANCE

## General

The main objective of the operation and maintenance aspect of an irrigation scheme is to facilitate the timely delivery of the required irrigation water to farms and to keep the irrigation system in an optimum operating condition.  This section therefore, discusses the main functions of the subject matter under consideration for the scheme.

## Operation of the Head Works

Operation at the diversion weir mainly focuses on the diversion of a controlled flow of river water, timely cleaning of floating debris in front of intake and removal of sediment deposits in front of the weir and intake structures.

## Irrigation System Operation

The operation of the irrigation system depends mainly on the method of water delivery at farm level.  Surface irrigation method is the recommended type of water distribution and application method for Azuwari diversion irrigation scheme.

The farmers would organize themselves and form groups in order to handle the water management.  Since flow is low Rotational water distribution would be applied within the group.  The rotational distribution is then to distribute water by turn to the whole scheme according to the timely need of crop water requirement. For better and efficient water management, crop diversification should be avoided within a group.  This would reduce the complexity of water distribution system of the scheme during one irrigation season. At farmers’ level of operation, a constant flow and variable irrigation time is advisable.

The operation of the irrigation system is continuous for 14 hours per day in main, secondary and tertiary canals where as field canals within a tertiary block are operating in rotational system with each other for irrigation hours proportional to their size.  Since the tertiary canal discharges are within the manageable range and the irrigation canal structures are accordingly designed for simple operation, the farmers can open and close easily whenever they required.

## Maintenance Requirement

The canal system of the project is earthen canal except main canal and secondary canal, which is susceptible to siltation, erosion, growth of weeds and or breaching problems.  Though the canal sections are designed for non- silting and non-scouring conditions, the above mentioned problems are unavoidable and hence periodical and yearly inspection and maintenance of canals and structures are obligatory in order to fulfill the design objective of the project area.

The maintenance tasks are categorized into two types: - routine activities, and repairs. The routine maintenance activities that are carried out periodically include:-

- Regular cleaning of sediments and weeds from canals and drains;

- Inspection and lubrication of gates; and

- Maintenance of cracked lined canals, regulating and control structures.

Repair works include task carried out more frequently and quickly, and include those tasks that are generally unpredictable.  They also include emergency works.  The activities included in this category are:-

- Repairing overtopped or breached canals, drains, and flood protection dykes;

- Repairing jammed gates;

- Filling holes made by wild animals; and

- Reduced free board due to walking over by people and livestock.

Regular inspection of the irrigation facilities should be carried out as part of the maintenance activities.  These tasks could be carried out immediately after the end of the main rains in September and during the rainy season.  This could concentrate on the interceptor drains and the flood protection dykes, the main canal and the field drains.  The inspection of the other works like the tertiary canals, field drains, and the water control and regulating structures could be carried out as part of routine operation activities.

- Beneficiaries of the project need to have operation and maintenance budget, For O&M cost incurring entity area:

- Purchase sing of gate lubricate (grease)

- Replacing and maintenance of Stolen and damaged gates

- Repair Damages on the cross drainage structures.

The expense for O&M should be collected from the beneficiaries. Of course, much of the task is done by the labor and skill of the community. For cost incurring activities beneficiaries have to collect money based on the proportion of the command area they owned.

# REFERENCE

- FAO (1977) guidelines for predicting crop water requirements. No 24, Rome Italy

- Design of small Canal structures , USBR

- Soft copies of hydraulic structure publishing

- IDD manual

- ESRDF manual