Amhara National Regional State

Water, Irrigation and Enery Development Bureau

(BOWIED)

Detail Design Of Bahir Libo Small-Scale Irrigation Project

Volume IV: Engineering Design Draft Report

September 2016

Bahir Dar

Client: Bureau of Water, Irrigation and Energy Development

(BoWRD)

Address:

P.O.Box 88
Telephone: 0528-200853/855
Fax: 251-08-20-65-68/204676/202040

Consultant: Amhara Design & Supervision Works Enterprise

(ADSWE)

Address:

P.O.Box  1921
Telephone: +251-582-181023/ 180638/181201/181254
Fax: (058) 2180550/ (058) 2180560
E-mail: amhara design@yahoo.com

Bahir Dar, Ethiopia

# FEASIBILITY STUDY & DETAIL DESIGN REPORT STRUCTURE

- Volume I: Watershed Management

- Volume II: Engineering Geology

- Volume III: Irrigation Agronomy

- Volume IV: Engineering Design

- Volume V: Socio Economy

- Volume VI: Environmental Impact Assessment

Table of Contents								 Page Nr

FEASIBILITY STUDY & DETAIL DESIGN REPORT STRUCTURE	iii

LIST OF TABLES	vii

LIST OF FIGURES	ix

SAILENT FEATURE	x

1	INTRODUCTION	12

1.1	Background	12

1.2	Description of the Project Area	13

1.2.1	Location & Accessibility	13

1.2.2	Previous Irrigation Practices	15

1.3	Objectives of the Study	15

1.3.1	Major Objective	15

1.3.2	Specific Objectives	16

1.4	Scope of the Study	17

1.5	Methodology	18

1.6	Watershed Characteristics	19

1.7	Data Requirement	19

1.8	Meteorological data availability	20

1.8.1	Climate	21

1.8.2	Rainfall Data	21

1.8.3	Water resource Availability	21

1.9	Upstream & Downstream utilization	21

1.10	Data screening	22

1.11	Data Quality Check	28

1.11.1	General	28

1.11.2	Data Consistency Test	28

1.11.3	Checking Data Reliability	30

1.11.4	Data Outlier Test	30

1.12	Peak Discharge determination	32

1.12.1	General	32

1.12.2	Flood Analysis	33

1.12.3	Estimation of Design Rainfall (Frequency Analysis)	33

1.12.4	Selection of Probability Distribution Method and Design point Rainfall	34

1.12.5	Time of concentration	35

1.12.6	Curve Number	36

1.13	Flood Analysis by SCS Unit Hydrograph Method (Composite Hydrograph Method)	36

1.13.1	Rainfall Profile	37

1.13.2	Runoff Analysis Using CN-SCS Method	37

1.14	Tail Water Depth Computation	41

1.14.1	Average river bed slope	41

1.14.2	Manning’s Roughness Coefficient	42

1.14.3	Tail Water depth analysis	42

1.15	Upstream Water surface profile	45

2	HEADWORK STRUCTURES DESIGN	48

2.1	Headwork Site Selection	48

2.2	Geological condition of the river	49

2.3	Left and Right Banks	50

2.4	River Bed	51

2.5	Sources of construction materials	52

2.5.1	Rock for Masonry and Crushed Coarse Aggregate	52

2.5.2	Fine Aggregates	53

2.5.3	Canal Fill Material	54

2.5.4	Water	55

2.6	Headwork Type Selection	55

2.7	Hydraulic Design of Headwork Structure	55

2.7.1	Weir Height Determination	55

2.7.2	Base flow of the River	56

2.7.3	Flow over the Weir crest	56

2.7.4	Top and bottom width of weir	57

2.7.5	U/S and D/S HFL Calculation & Determination	57

2.7.6	Hydraulic Jump Calculation	58

2.7.7	Impervious floor Length	60

2.7.8	Cut off Depth Calculation	62

2.8	Stability Analysis of weir	63

2.9	Design of Divide wall, Under Sluice, and Canal outlet	65

2.9.1	Breast Wall and Operation Slab	68

2.10	Retaining Walls	69

3	IRRIGATION AND DRAINAGE SYSTEMS DESIGN	78

3.1	Command Area	78

3.1.1	Topography	79

3.1.2	Climate	79

3.1.3	Soil characteristics	79

3.1.4	Existing Irrigation Practices in the Project Area	80

3.2	Irrigation method selection	81

3.2.1	Methods of irrigation	81

3.3	Naming of canal units	86

3.3.1	System Layout and Command Units	86

3.3.2	Irrigation Efficiencies and Water Demands	87

4	IRRIGATION CANALS	90

4.1	General	90

4.2	Canal lining option	90

4.3	Canal cross Section	92

4.4	Longitudinal slope	93

4.5	Hydraulic Design	93

4.5.1	Minimum allowable velocity	94

4.5.2	Maximum allowable velocity	94

4.6	Roughness Coefficients	95

4.7	Freeboard of Canals	96

5	CANAL STRUCTURES	101

5.1	General	101

5.2	Types and Hydraulic Design of the structures	101

5.2.1	Crossing Structures	101

5.3	Drainage Canals	106

5.4	Drainage System	106

5.5	Design of Division box	106

5.6	Drop structure	110

6	OPERATION AND MAINTENANCE	125

6.1	General	125

6.2	Operation of the Head Works	125

6.3	Irrigation System Operation	125

6.3.1	Maintenance Requirement	126

7	CONCLUSION AND RECOMMENDATION	127

8	REFERENCE	128

# LIST OF TABLES

Table 2-1: Trend analysis of the Maximum Daily rainfall (in mm) at the Ebnat Meteorological Department from 1997 to 2008.	14

Table 2-2: Computation of Ft and tt for two sub-sets of  the Maximum Daily rainfall (in mm) at the Ebnat Meteorological Department from 1997 to 2008.	16

Table 2-3: Computation of the lag 1 serial-correlation coefficient for the Maximum Daily rainfall (in mm) at the Ebnat Meteorological Department from 1997 to 2008.	18

Table 2-4:  Sample Rainfall Analysis for Ebnat Metrological Station	19

Table 2-5: Design Rainfall Arrangement	28

Table 2-6: Computation of peak discha

Table 5. 1: Flume 1 hydraulic Parameters at chain age 0+807.	104

Table 5. 2: Flume 2 hydraulic Parameters. At chain age 1+428m	105

Table 5. 3: Road crossing Dimensions at the main canal.	106

Table 5. 4: Hydraulic parameters of Division Boxes.	110

Table 5. 5: Drop structures hydraulic parameters	112

Table 5. 6: Infrastructure cost	113

rge for each incremental runoff	29

Table 2-7: Peak Discharge Analysis Using Complex Hydrograph Method	30

Table 3-1: Short summary of Mena sand laboratory result	45

Table 3-2: Weir height determination.	47

Table 3-3: Floor thickness of dimensions.	52

Table 3-4: Moment and Load Calculation result	54

Table 3-5: Divide wall Stability analysis result	56

Table 3-6: Retaing wall Design input parameters.	60

Table 3-7: Stability analysis for the u/s Left hand side masonry retaining wall	61

Table 4-1: Summary of physical and chemical analysis of soils of the command area	63

Table 4-2: Conveyance Efficiencies	72

Table 4-3Crop Water Requirements and Indicative Design Flows	73

Table 5-1: Permissible maximum velocities	79

Table 5-2: Manning’s Roughness Coefficient for different canal types (n)	81

Table 5-3: Recommended Canal Freeboards	82

Table 5-4: Sumary of main canal Hydrulics	84

Table 5-5: Sumary of Secendary canal Hydrulics	84

Table 6-1: Flume 1 hydrulic Parameteres.	90

Table 6-2: Flume 2 hydrulic Parameters.	90

Table 6-3: Road crossing Dimensions at the main canal.	91

Table 6-4: Catch drain canals design Hydraulics Parameters.	94

Table 6-5: Hydraulic parameters of Division Boxes.	97

Table 6-6: Drop structures hydraulic parameters	99

# LIST OF FIGURES

Figure 1-1 Project Woreda Location map	3

Figure 1-2 Project Area Location map	4

Figure 2-1: Incremental hydrographs curves	32

Figure 2-2: River profile	33

Figure 2-3: Tail water rating curve	36

Figure 2-4: upstream water surface profile.	38

Figure 3-1: Right Banks                                                                                                      Figure 3-2: Left Banks	41

Figure 3-3: Thick unit of alluvial deposit and test pit taken on the stream bed of Bahir libo at the weir axis.	42

Figure 3-4: Geological X_ Section of Bahir Lib headwork axis.	43

Figure 3-5: Rock Quarry Site for Masonry Stone and Crushed Aggregates.	44

Figure 3-6: Sand Sources in Mena Stream.	45

Figure 3-7: Flow over weir body.	50

Figure 4-1: Furrow shape depending on soil type.	68

Figure 6-1: Flume -1 plan and section view at SC1-1at chainage 0+369	89

Figure 6-2: Typical Cross Section of Catch drain canals	93

Figure 6-3: Typical Division Box plan.	95

# SAILENT FEATURE

- Project name: Bahir Libo Small Scale Irrigation Project

- Name of the stream: (Name of the water source)  Bahir Liboriver

- Location of the Weir site

- North: 1376602

- Easting:__ 411036

- Zone: (N/Gondar)

- Wereda: (East Belesa)

- Average Altitude: 1647 masl

- Hydrology

- Design rainfall: 72.29 mm

- Catchment area: 88.59Km2

- Longest flow path length: 18.50Km

- Design flood: 191 m3/sec

- Design base flow: 184 lit/se.

- Diversion Weir

- Weir type: Broad crested

- Height: 3.1 m

- Gross crest length: 28 m

- Weir crest level: 1648.34 m.a.s.l

- U/S HFL: 1650.553 m.a.s.l

- U/S TEL: 1650.73m.a.s.l

- D/s TEL 1649.61 m.a.s.l.

- D/s HFL: 1648.91 m.a.s.l (Post Jump)

- Afflux: 1.64 m

- Under sluice

- Sill level: 1646.18m.a.s.l

- Dimension: 1*1 m2

- Discharge amount:2.02 m3/sec

- Outlet

- Sill level: 1647.5m.a.s.l

- Opening dimension: 0.55*0.5 m2

- Discharge capacity: 147 lit/sec.

Irrigation and drainage systems Infrastructure

- Command area size: 104 ha

- Type of soil of the command area is dominantly clay soil

- Design discharge of the main canal = 147l/sec

- Irrigation system layout consists of one lined main canal, 4 Secondary canals,9 Tertiary canal

- Main irrigation structures designed are;

- 4 flume Gully crossing structures

- 3 Box Culverts

- 12 Road crossing Structure

- 12 turn outs

Project cost without Vat

| Bahir Libo Irrigation Project |  |  |
| --- | --- | --- |
| SUMMARY OF BILLS |  |  |
| Bill No. | Description | Amount (Birr) |
| 1 | General Items | 927,610.00 |
| 2 | Head work (Weir body, Retaining wall, Outlet structures) | 5,395,148.44 |
| 3 | Infrastructure (Main canal, Secondary canal,teriterycanal,Fiel canals, Flumes, division canal & Turnouts | 14,722,709.93 |
| Total |  | 21,045,468.36 |
| VAT |  | 3,156,820.25 |
| Grand Total cost with VAT |  | 24,202,288.62 |
| Ha |  | 120.00 |
| Cost /Ha |  | 175,378.90 |

# INTRODUCTION

## Background

The study and design work of Bahir Libo Small Scale Irrigation Project is being carried out by Amhara Design and supervision works Enterprise (ADSWE). The owner of the project is Amhara Region Bureau of Water, Irrigation and Energy Resource development.

It is known that agriculture is the backbone of our economy. Hence, to maximize agricultural production for the domestic use and to supplement industry lead economy, irrigation is the non-alternate option. Therefore, it will assist rain fed agriculture and alleviate the impact of recurrent draught in the country. This can be achieved by working together with the community, local and international NGOs and the government organizations to use all the available resources efficiently and bring significant change.

The Amhara Water Irrigation and energy Bureau is playing its role in the development of small-scale irrigation projects in the region. Accordingly, as part of the water sector development program, the office has initiated the study and design of a small scale irrigation scheme on Bahir LiboRiver  at Goga Kebele and signed an agreement with Amhara Design & Supervision Works Enterprise (ADSWE) for the study and design of the project.

Amhara design and supervision Works Enterprise (ADSWE) is established to do its part in the study, design and supervision of water structures, in the area of land use and water shed management, irrigation and water supply of the region as well as the country. ADSWE has assigned a multidisciplinary qualified professional crew to study and design on the Irrigation projects.

The project Woreda, East Belesa Woreda, is one of the draught prone Woreda in the Amhara Region. Bahir Libo Small Scale Irrigation Project is designed based on the client’s requirement to address the beneficiaries demand. It is believed that this irrigation project will change the living standard of that society in supplementary irrigation as well as in full irrigation system. Technical, environmental and socio economic issues are well addressed by the respective disciplinary. The potential beneficiaries’ show their interest in the execution of the project and the scheme after execution is supposed to be handed over to the farmers cooperatives for operation and maintenance of canal system and water regulation.

## Description of the Project Area

Location & Accessibility

The project area is located in East Belesa Woreda of North Gondar Zone, Amhara Region. The head work site and command area are located in same kebele, Goga Kebele. The head work site is located in sheha Got.

The project Head work site is accessible by vehicle via 85km asphalt road from the regional city Bahir Dar through Addis Zemen, 84km all-weather road from Addis Zemen to Guhal, East Belesa Wereda capital town and 30km all weather road from Guhala to Project Kebele. The head work site and the Main command area is not accessible by road but on foot. The headwork is 3km far from the access road.

The geographic coordinate of the position of the headwork site is 411036 Easting, 1376602m Northing, and 1647m Altitude in UTM_ADINDAN Coordinate system. The command area lays between 11376444.837 & 1378190.97 North and between 410939.737 & 413920.18 Easting. (See fig 1.1 & 1.2). The command elevation also ranges from 1660.4 to 1598.5.

Figure 1. 1: Project Woreda Location map

Figure 1. 2: Project Area Location map

Previous Irrigation Practices

There is traditional irrigation practice in the right side of the command area using water pumps for cultivating papaya and onion. Therefore the farmers are experienced for traditional irrigation, and demanding modern irrigation.

## Objectives of the Study

### Major Objective

The project area faces variability of rainfall distribution though the overall rainfall generally suffices the rain-fed agriculture. Accordingly, the rain-fed agriculture needs means of supplementing during distribution failures and further full irrigation is required to maximize the use of the potential land and water resources.

Hence the objective of this project is to contribute a substantial share in the effort to reduce the risk of production decrease due to rainfall variability and increase the productivity of the resource in the project specific area. Specifically, the project is targeted for the following.

- To make sustainable the rain-fed crop production and make extra production in the wet season.

- There is a general consensus that irrigation investments will achieve broader food security and poverty reduction impacts and if efforts are also geared towards up-grading existing traditional farming practices with support to enhance access to input supply, output marketing and extension to facilitate access to information and innovations.

- This objective is to be realized by constructing a Diversion irrigation structures across the Bahir LiboRiver and divert the river flow to elevated area to make it gravitational flow.

### Specific Objectives

Other benefits that can be expected to appear with the launching of the project are:

- Efficiency of water use improvement;

- Improved local nutrition/food security gains;

- Improved management of scarce natural resources (land and water);

- Resilience against drought risk;

- Rationale for erosion control and watershed management;

- Rationale for the intensification and modernization of small-holder agriculture and rural lifestyles.

The engineering study and design enables the realization of the project by the provision of engineering structures that will allow the appropriate abstraction of the river water for delivery in to the identified irrigation fields of the study area. Hence, this engineering design is specifically targeted to:

- Analyze hydrologic requirements of the project and engineering structures;

- The formulation of sound and stable structure,  with necessary provisions that allow safe, easy and low-maintenance operation in the service life of the project;

- Develop working drawings;

- Estimation of construction costs.

The engineering study and design enables the realization of the project by the provision of engineering structures that will allow the appropriate abstraction of the river water for delivery in to the identified irrigation fields of the study area. Hence, this engineering design is specifically targeted to:

- Analyze hydrologic requirements of the project and engineering structures;

- The formulation of sound and stable structure,  with necessary provisions that allow safe, easy and low-maintenance operation in the service life of the project;

- Develop working drawings;

- Estimation of construction costs.

## Scope of the Study

- The irrigation design shall ensure reliability, equity and flexibility of water delivery to farmers. It will aim at reducing conflicts among water users and will lead to lower operation and maintenance costs.

- Establish design criteria for irrigations structures to be approved by the client and to be used in the final design stage,

- Design proper irrigation system compatible with local conditions and management capabilities,

- Establish flood protection measures for the command area and canal structures and design the respective drainage system accordingly,

- Planning  and layout of the irrigation system, which include irrigation canals, drainage channels, inspection roads and alignments, canal spacing, canal length, location of structures, and water profiles along canal and drains at specified reaches, which is most economical easily manageable and aligned with topographic feature and geological investigation.

- Determination and estimation of water application conveyance and other losses and irrigation efficiencies and consideration of those parameters in design steps.

- Check and test hydraulic and structural designs of main canal considering total demand and the required capacity and the base flow availability,

- Prepare general plans and drawings for all irrigation infrastructure and irrigation systems designs.

## Methodology

In the study and design procedure, Designers used the following steps.

- Specific Site identification:

  - Review of the reconnaissance survey conducted by the Client

  - 50,000 scale top map and GIS information

  - Local farmers interview and discussion

  - Interview and discussion  with Wereda and Zone Agriculture section expertise

  - Previous studies

  - On foot travel along the river channel and farm areas.

- Topographic survey:

  - Surveying the headwork site and the Command area with sufficient radius, using Total station

- Flow estimation

  - Physical observation on flood mark indications and local information about high flood and critical flow condition of the river

  - Analyzing the recorded river flow data and use watershed inputs for further analysis.

  - Base flow estimated during the reconnaissance field visit by floating method.

- Irrigable area identification:

  - Using local information

  - 50,000 Topographic map, and GIS information, GPS to see elevation

The design report is organized in three sections. In Section I the Hydrology study is presented and in Sections II and III the Headwork and Irrigation and Drainage Systems designs are discussed respectively. In Section III, planning and design of the irrigation system after diverting the water will be dealt. The following are major areas of concern in this part.

- Study and design of the irrigation method to be adopted,

- Study and design of the irrigation system layout and associated structures,

- Design of the different conveyance canals,

- Planning and design of the different irrigation and drainage structures,

- Preparation of the longitudinal profiles of the different irrigation and drainage canals

## Watershed Characteristics

The Watershed has marked topographic variation; all types of slopes are present. The topographic feature of Bahir libo Watershed encompasses all types of slop classes from flat to very steep slope. The dominant slope classes are moderately steep slope 15-30% and steep slope (30%-50%)   they are sharing 23% and 31.7% of a total area of a watershed respectively. The rest flat almost flat  0—3%, Gently sloping 3—8%,).

Certain physical properties of watersheds significantly affect the characteristics of the runoff and sediment yield and are of great interest in hydrologic analyses. The rate and volume of runoff, and sediment yield from the watershed have much to do with shape, size, slope and other parameters of the landscape. These suggest that there should be some important relations between basin form and hydrologic performance. If the basin and hydrologic characteristics are to be related, the basin form must also be represented by quantitative descriptors. These parameters can be measured from maps.

In summary, the catchement area has the following details:

- Catchment Area  = 88.59km2

- Stream Length = 18.5

- CN(II) = 74.6

(Extracted from the Watershed Study Report of the same project)

At the selected reference point, the area of Bahir Libo catchment is 88.59km2 and consists of a network of tributaries.

Bahir Libo River at the headwork sites is characterized by well-defined channel system and considerable flows. It looks that the gradient of the river is getting low and hence there exists significant deposition of sediment mainly cobbles and boulders.

## Data Requirement

For all hydrologic analysis, the following factors shall be evaluated and included when they will have a significant effect on the result. These are:

- Catchment area characteristics including size, shape, slope, land use, geology, soil type, surface infiltration and storage.

- Stream channel characteristics including geometry and configuration, natural and artificial control, channel modification, aggradations-degradation, and debris;

- Flood plain characteristics;

- Metrological characteristics such as precipitation amount and type, storm cell size and distribution characteristics, storm direction etc.

Data required for this hydrological study is categorized into the following three major groups:

- Catchment and land use data

Slope classes, altitude and drainage characteristics of a catchment are considered constant over short period of time thus the information is extracted from the topographic map of scale 1:50,000 prepared by the Ethiopian Mapping Authority (EMA) in 1993 and a 90 by 90 m digital elevation model available in our archive. The slope of the longest stream is also measured from the same map and DEM. Information on land use and soils of the catchment is collected during field study.

- Hydrometric data

The catchment under consideration is not gauged at the proposed diversion site. The hydrologic characteristics of ungauged catchments can be estimated from the flood frequency analysis. No flow records are available for the River. So, only rainfall data are used for further analysis.

- Climatic data

Daily rainfall data for Ebnat meteorological station is collected from the National Meteorological Service Agency (NMSA) for years from 1997 to 2008.

## Meteorological data availability

Hydrological data is important in order to make hydrological computations, to establish design criteria and to make any forecast. Moreover, the most important steps in any hydrological study are the collection, screening and processing of input data for further analysis. Time spent on ensuring that the best possible data set is used generally speeds up the analysis to follow. To be able to make good use of hydrologic data, the data has to be stored in such a way that all possible errors are removed and that the data is accessible. However, sufficient availability of such data in a target position is a rarely happening phenomenon in developing countries.

At the proposed outlet point, Bahir Libo River has a catchment area of 88.59km2, say 89km2. Ebnat Station dominates the majority of the watershed. Consequently, the designer has adopted Ebnat’s station rainfall data for peak flood analysis.

### Climate

Small-scale irrigation project designers and planners are faced with lack of good data on the hydrology of the river system that will be their water source and on local weather and climate conditions. River gauging stations are virtually non-existent in remote rural areas of Ethiopia; meteorological stations are almost rare. Likewise, at Goga Kebele and in the catchment area of this project, there is no meteorological station of any level. Moreover, there are no flow data for the river near the project. Therefore, data for the hydro-meteorological analysis is taken from the nearby station and similar areas. Rainfall data is considered from Ebnat Meteorological station.

### Rainfall Data

In order to compute the design flood for the Diversion and associated structures, the daily maximum rainfall is collected from Ebnat Metrological stations with a record of 12 years. The mean annual rainfall amount is more than 1400mm (1997 - 2008 data) and most of it occurs from June to August.

### Water resource Availability

The base flow, which is measured in November 2015 G.C, is 184 lit/s Source report of Amhara Water Irrigation and energy Bureau, Irrigation and Drainage Study, Design and Construction Core Process (North Gondar Zone 2008 Site Identification Report). This figure is adopted for supplementary Irrigation design because the base flow become reduced at dray season, so that the system is designed mainly supplementary case because rainfall become stop at the beginning of septmber. For downstream users and ecology we have to release 20% of the base flow, due to this 38 l/s released to downstream.

## Upstream & Downstream utilization

The riverbed is covered with Cobles and deposition and hence it is expected/be lived/ that there will be significant amount of discharge through the deposits. Hence, even if we divert the flow totally, there will not be a problem for the downstream needs and they can get the accumulated flow at the downstream. In addition, the proposed irrigation hour for the project is 18hours and the downstream users can use the discharge during off time of irrigation. Hence, the irrigation system is designed for 147 lit/sec and the rest 38l/s released to the downstream ecology.

## Data screening

Engineering studies of water resources development and management depend heavily on hydrological data. These data should be stationary, consistent, and homogeneous when they are used for frequency analyses or to simulate a hydrological system. To determine whether the data meet these criteria, the engineer needs a simple but efficient screening procedure. Such a procedure is described below.

A time series of hydrological data is strictly stationary if its statistical properties (e.g. its mean, variance, and higher-order moments) are unaffected by the choice of time origin. (By ‘unaffected’, we mean that estimates of these properties agree within the range of expected statistical variability.) The basic data-screening procedure pre- sented here is based upon split-record tests for stability of the variance and mean of such a time series.

Test for Absence of Trend by Spearman’s Rank-Correlation Method

One must be sure that there is no correlation between the order in which the data have been collected and the increase (or decrease) in magnitude of those data. It is common practice to test the whole time series for absence of trend.

Where            Rsp= Spearman’s Rank-Correlation.

n is the total number of data.

D is difference.

i is the chronological order number.

Di = Kxi-Kyi

Kxi, is the rank of the variable, x, which is the chronological order number of the observation.

The series of observations, yi is transformed to its rank equivalent, Kyi by assigning the chronological order number of an observation in the original series to the corresponding order number in the ranked series, y.

Where

t, has Student’s t-distribution with v = n-2  degrees of freedom.

One should therefore keep in mind that t{v,p} = -t{v,l  -p}  when using such tables.) At a significance level of 5 per cent (two-tailed), the two-sided critical region, U, of tt, is bounded by:

{- ∞, t {v, 2.5%}} U {t {v, 97.5%}, +∞)

Table 1. 1: Trend analysis of the Maximum Daily rainfall (in mm) at the Ebnat Meteorological Department from 1997 to 2008.

| No. |  | Year | Rain fall | y,Ranked Rain fall | Kxi | Kyi | Di | Di*Di |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 |  | 1997.00 | 43.100 | 30.70 | 1 | 2 | -1.00 | 1.00 |
| 2 |  | 1998.00 | 30.700 | 31.30 | 2 | 7 | -5.00 | 25.00 |
| 3 |  | 1999.00 | 56.600 | 34.20 | 3 | 8 | -5.00 | 25.00 |
| 4 |  | 2000.00 | 51.600 | 43.10 | 4 | 1 | 3.00 | 9.00 |
| 5 |  | 2001.00 | 50.900 | 48.70 | 5 | 10 | -5.00 | 25.00 |
| 6 |  | 2002.00 | 49.500 | 49.50 | 6 | 6 | 0.00 | 0.00 |
| 7 |  | 2003.00 | 31.300 | 50.90 | 7 | 5 | 2.00 | 4.00 |
| 8 |  | 2004.00 | 34.200 | 51.60 | 8 | 4 | 4.00 | 16.00 |
| 9 |  | 2005.00 | 53.200 | 53.20 | 9 | 9 | 0.00 | 0.00 |
| 10 |  | 2006.00 | 48.700 | 54.00 | 10 | 12 | -2.00 | 4.00 |
| 11 |  | 2007.00 | 58.600 | 56.60 | 11 | 3 | 8.00 | 64.00 |
| 12 |  | 2008.00 | 54.000 | 58.60 | 12 | 11 | 1.00 | 1.00 |
| Sum |  |  |  |  |  |  |  | 174.00 |
|  |  | MEAN | 46.867 |  |  |  |  |  |
|  |  | STDV | 9.77328 |  |  |  |  |  |
|  |  | SKEW | -0.7868 |  |  |  |  |  |
|  |  | KURTOSIS | -0.8221 |  |  |  |  |  |

After calculating Di Values, we are compulsory to estimate the superman rank correlation coefficient by using =0.392 by directly substuting all the values on the formula and then calculate the student t distribution by using the =1.346

The table of percentile points for the t-distribution (Appendix I ) gives the critical values of t, at the 5-per-cent level of significance for 12 - 2 = 10 degrees of freedom as: t{10,2.5%} = -2.23, and t{10,97.5%} = 2.23 Checking this result against the condition expressed above  -2.243< 1.346< 2.23  one finds that the condition is satisfied. Thus, there is no trend.

Tests for Stability of Variance and Mean

In addition to testing the time series for absence of trend, one must test it for stability of variance and mean. The test for stability of variance is done first by using F-test.

There are two reasons for this sequence: firstly, instability of the variance implies that the time series 20 is not stationary and, thus, not suitable for further use; secondly, the test for stability of mean is much simpler if one can use a pooled estimate of the variances of the two sub-sets. (This is permissible, however, only if the variances of the two sub-sets are statistically similar.) The test statistic is the ratio of the variances of two split, non-overlapping, sub-sets of the time series. The distribution of the variance-ratio of samples from a normal distribution is known as the F, or Fisher, distribution. Even if the samples are not from a normal distribution, the F-test will give an acceptable indication of stability of variance.

Where

S2 is variance. Note that, to compute Ftt it is irrelevant whether one uses the sample standard deviation, s, or the population standard deviation,σ. We give here two convenient formulae for computing the sample standard deviation, s, namely:

S=  i=1nxi2-n*Xavn-10.5

Where:-

xi, is the observation, n is the total number of data in the sample, and Xav is the mean of the data.

The variance of the time series is stable, and one can use the sample standard deviation, s, as an estimate of the population standard deviation, δ, if

F {v1, v2, 2.5%} < Ft < F {v1, v2, 97.5%}

where vI = n1-1  is the number of degrees of freedom for the numerator, v2 = n2-l is the number of degrees of freedom for the denominator, and n1 and n2 are the number of data in each sub-set.

Table 1. 2: Computation of Ft and tt for two sub-sets of the Maximum Daily rainfall (in mm) at the Ebnat Meteorological Department from 1997 to 2008.

| Rank | Sub-set 1 |  |  | Sub-set 2 |  |
| --- | --- | --- | --- | --- | --- |
|  | Xi | Xi2 |  | Xi | Xi2 |
| 1.00 | 43.10 | 1857.61 |  | 31.30 | 979.69 |
| 2.00 | 30.70 | 942.49 |  | 34.20 | 1169.64 |
| 3.00 | 56.60 | 3203.56 |  | 53.20 | 2830.24 |
| 4.00 | 51.60 | 2662.56 |  | 48.70 | 2371.69 |
| 5.00 | 50.90 | 2590.81 |  | 58.60 | 3433.96 |
| 6.00 | 49.50 | 2450.25 |  | 54.00 | 2916.00 |
| Total | 282.40 | 13707.28 |  | 280.00 | 13701.22 |
| No.data(n) | 6 |  |  | 6 |  |
| Xave. | 17.65 |  |  | 17.50 |  |
| S1 | 48.66 |  |  | 48.71 |  |
| S2 | 2367.63 |  |  | 2372.74 |  |
| Ft | 1.00 |  |  | V1 | 5 |
|  | 1.00 |  |  | V2 | 5 |
| tt | 0.005 |  |  | V | 10 |

F {vI,v2,2.5%} < Ft, < F{vI,v2,97.5%}=0.14<1<7.15

The t-Test for Stability of Mean

The t-test for stability of mean involves computing and then comparing the means of two non-overlapping sub-sets of the time series (the same subsets from the F-test for stability of variance).

Where n is the number of data in the sub-set, X the mean of the sub-set, and s2 its variance.

The time series is considered stable if:

t(v,2.5%} < tt < t(v,97.5} =-2.23<0.005<2.23  The rainfall data which is taken

from Ebnat station is stable for all stastical parameters of variance and mean so the data can be used for further analaysis.

Test for Absence of Persistence bySerial-Correlation Coefficient

The serial-correlation coefficient can help to verify the independence of a time series.

Where xi is an observation, xi+1 is the following observation, X ave is the mean of the time series, and n is the number of data.

Table 1. 3: Computation of the lag 1 serial-correlation coefficient for the Maximum Daily rainfall (in mm) at the Ebnat Meteorological Department from 1997 to 2008.

|  | maximum daily rain fall | Xi-Xav. | (Xi-Xav.)*(Xi+1-Xav.) | (Xi-Xav.)i*(Xi-Xav.)i |
| --- | --- | --- | --- | --- |
|  | 43.10 | -3.77 | 60.89 | 14.19 |
|  | 30.70 | -16.17 | -157.36 | 261.36 |
|  | 56.60 | 9.73 | 46.07 | 94.74 |
|  | 51.60 | 4.73 | 19.09 | 22.40 |
|  | 50.90 | 4.03 | 10.62 | 16.27 |
|  | 49.50 | 2.63 | -40.99 | 6.93 |
|  | 31.30 | -15.57 | 197.18 | 242.32 |
|  | 34.20 | -12.67 | -80.22 | 160.44 |
|  | 53.20 | 6.33 | 11.61 | 40.11 |
|  | 48.70 | 1.83 | 21.51 | 3.36 |
|  | 58.60 | 11.73 | 83.70 | 137.67 |
|  | 54.00 | 7.13 |  | 50.88 |
| Sum | 562.40 | 0.00 | 172.11 | 1050.69 |
| Xave. | 46.87 |  |  |  |

To accept the hypothesis Ho: r1 = 0, the value of r1 should fall between the UCL and the LCL.

Applying the above condition to the time series, we see that the condition: -0.654< 0.16 < 0.473 is satisfied. Therefore, no correlation exists between successive observations. The data are independent, and there is no persistence in the time series due to this we can use the data for flood analysis.

## Data Quality Check

### General

In general, it is thought that the more gaps there are in a record, the less reliable the data is. Instrument failure can hardly be a reason for gaps in the observations, since apart from the rain gauge and a measuring glass; only a pencil and booklet are needed to record the readings.

The available records were first visually checked on clear outliers. Most of these appeared to be simple typing errors. An outlier test was of course carried out to discard some unreasonable data.

### Data Consistency Test

12 years of daily heaviest rainfall records of Ebnat meteorological station, from 1997 to 2008, was used for the estimation of point design rainfall. In spite of less number of rainfall data, the designer assumes an event that happens at every 50 years of return period to compute the design rainfall, which is common for small-scale irrigation projects like Bahir Libo Diversion.

These data should be checked for their consistency by outlier test following the assurance of the reliability of them.

Table 1. 4:  Sample Rainfall Analysis for Ebnat Metrological Station

| S.No. | Year | Max. RF | Descending Order | Rank | Logarithmic Value/Yo/ | (Yo-Ym)2 | (Yo-Ym)3 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1997 | 43.1 | 58.6 | 1 | 1.7679 | 0.0114323 | 0.0012224 |
| 2 | 1998 | 30.7 | 56.6 | 2 | 1.7528 | 0.0084347 | 0.0007747 |
| 3 | 1999 | 56.6 | 54 | 3 | 1.7324 | 0.0051006 | 0.0003643 |
| 4 | 2000 | 51.6 | 53.2 | 4 | 1.7259 | 0.0042167 | 0.0002738 |
| 5 | 2001 | 50.9 | 51.6 | 5 | 1.7126 | 0.0026702 | 0.0001380 |
| 6 | 2002 | 49.5 | 50.9 | 6 | 1.7067 | 0.0020923 | 0.0000957 |
| 7 | 2003 | 31.3 | 49.5 | 7 | 1.6946 | 0.0011310 | 0.0000380 |
| 8 | 2004 | 34.2 | 48.7 | 8 | 1.6875 | 0.0007051 | 0.0000187 |
| 9 | 2005 | 53.2 | 43.1 | 9 | 1.6345 | 0.0007022 | -0.0000186 |
| 10 | 2006 | 48.7 | 34.2 | 10 | 1.5340 | 0.0161162 | -0.0020459 |
| 11 | 2007 | 58.6 | 31.3 | 11 | 1.4955 | 0.0273675 | -0.0045274 |
| 12 | 2008 | 54 | 30.7 | 12 | 1.4871 | 0.0302194 | -0.0052533 |
| SUM |  |  | 562.40 |  | 19.9317 | 0.1101881 | -0.0089197 |
| MEAN |  |  | 46.87 |  | 1.6610 | 0.0091823 | -0.0007433 |
| STANDARD DEVATION |  |  | 9.77 |  | 0.1001 |  |  |
| SKEWNESS COEFICIENT |  |  | -0.787 |  | -0.9706 |  |  |

### Checking Data Reliability

Number of data = 12

Standard deviation, 9.77

Mean, X= 46.87 mm

Standard error of mean, = =

Relative standard, ==6.01 % < 10 %

Hence, the data series is regarded as reliable and adequate since the value of "Relative standard" is relatively small. Now, let us check the data outlier test.

### Data Outlier Test

This is done to check whether the adopted data are within a limited range or not. Outliers are data points that depart significantly from the trend of the remaining data. The retention or deletion of these outliers can significantly affect the magnitude of statistical parameters as mean and standard deviation that are computed from the data, especially for small samples. Procedures for treating outliers require judgment involving both mathematical and hydrologic considerations. However, here simple mathematical approaches are practiced to sort out the data that seem reliable of the trend of the parent.

Input data:  Summation of the daily maximum rainfall data records of nineteen years,

Arithmetic mean of the data,

Summation of common logarithms of the data,

Mean value of common logarithms of the data,

,

Standard deviation of the common logarithms = 0.1. Skewness of the common logarithms of the daily maximum rainfall data, Cs=-0.97

Tests for Outliers

Outliers are data points that depart from the trend of the remaining data especially at the two extreme ends. The deletion or retention of these outliers can significantly affect the magnitude. As shown from the above calculation the Station Skewness is less than -0.4; thus, test for lower outliers are focused.

Test for higher outlier

Higher outlier

Where:  = mean of data in common log unit

= Frequency factor (Can be obtained from standard Hydrological textbooks and depends on the sample size N)

From Applied Hydrology text book for data N=12, kN= 2.133 and Sy = 0.1 (Standard deviation of the common logarithms)

Higher outlier

Higher outlier

The highest recorded value (58.60) is less than the higher outlier value (74.90mm). Hence, there is no higher outlier.

Test for lower outlier

Lower outlier

Lower outlier

The lowest recorded value (30.7 mm) is greater than lower outlier (28.01mm) and no lower outlier is detected. After all this, the recorded data is consistent for lower outlier and higher outlier. Therefore, the analysis could be commenced using all the recorded data.

Conclusion:  for the 12 years data, there is no higher and lower outlier value. Hence, all the 12 years data are used for the determination of design Rainfall and then design flood.

## Peak Discharge determination

### General

All hydraulic works sized by a flood estimate are designed on a risk basis, and none is completely safe. The most important factors in selecting the design peak flood are cost and safety.  It is too costly to design a small structure for very large peak flood. It is not also safe to design a very large structure (like large dams) for very small peak discharge. An optimal magnitude for design is therefore one that balances the conflicting condition between cost and safety. Chow et.al. 1988 stated that the following three approaches are commonly employed to determine the hydrologic design value:

- Frequency analysis

- Risk analysis

- Hydro economic analysis

Data to both economy and social caused due to a flood magnitude of a certain recurrence interval is rarely available. Thus, it is often practically difficult to design a certain hydraulic structure based on hydro-economic analysis and consequently, this method is not adapted for this study.

Small hydraulic structures like the one, which will be treated here, where the damage caused to property due to the occurrence of floods larger than the design magnitude is not significant, are often designed for a flood magnitude of 50-years recurrence interval. The catchment under investigation is not gauged at the proposed site. Various methods are available for flood estimation. Some of them are based on the characteristics of drainage basin, others are based on the theory of probability applied to the previous known flow data, and lastly the others are based on study of rainfall and runoff data. Two methods namely, SCS-CN method and flood frequency analysis are employed to estimate the design flood in this study. These techniques are discussed in the following sections.

### Flood Analysis

In the designing history of irrigation structures, determination of peak flood value is the first step. This because they should be resistant to any thrust resulting from flood water. For determining extreme flood/storm events, specific extreme values distributions are assumed, and the required statistical parameters are determined from the available data, from which the flood/storm magnitude Q/P for any specific return period T can be determined. The Gumbel extreme value distribution is one of them and is largely used to extrapolate flood data. According to this distribution, a storm of return period T can be quantified by:

Where QT is a discharge corresponding to return period T years and k is a frequency factor depending on the number of data we have. Since no river flow data was available for this River, this analysis was not carried out. However, it has to be understood that the Gumbel method will give reasonable results if we have greater number of available past records. For example if we want to predict the magnitude of flood having a return period of 50 years and only 50 years of record are available, then the error is likely to be great, because our result is based on one sample alone let alone records of less than 50 years.

### Estimation of Design Rainfall (Frequency Analysis)

The design storm is characterized by its duration and temporal distribution. In the absence of continuous records of rainfall, the design storm can be derived from the 24-hour rainfall. According to the SCS, four distributions are developed for the entire United States. Other countries have to determine which of the four will best fit the local condition or develop their own relationships.

The objective of the frequency analysis is to relate the magnitude of the extreme events with their frequency of occurrence using probability distribution. The hydrologic system producing the runoff (storm rainfall system) is also considered stochastic, space-independent and time- invariant.

After checking consistency (reliability and outliers) test, the rainfall data are obtained as representative for the analysis. The magnitude of the probable maximum design rainfall of fifty years of   return period is estimated by the following methods as shown.

### Selection of Probability Distribution Method and Design point Rainfall

The collected data can be tested using different probability distribution techniques. However, the   most commonly used distribution methods to fit extreme rainfall events are: Normal, Gumbell, Log Normal, Pearson Type III and Log Pearson Type III. Although it relatively over estimates the design storm, Gumbell Probability Distribution is preferred to be safe from flood risks. This is because the computed design rainfall  passes through different processes like Rainfall Profile and Ariel to Point ratio that further reduce the magnitude of expected design flood. Thus, it is better to take a design storm of highest value. The result of the analysis is shown in the following sections.

Determination of design storm using Gumbel's method: - For determining extreme flood/storm events, specific extreme values distributions are assumed, and the required statistical parameters are determined from the available data, from which the flood/storm magnitude Q/P for any specific return period T can be determined. The Gumbel extreme value distribution is one of them and is largely used to extrapolate flood data. According to this distribution, a storm of return period T can be quantified by:

QT = QMEAN + K*σ

Where:

Where

QT 	 = Peak design storm of a given return period (50 years in this case)

Qmean    = Mean of the daily maximum 24 hr storm

K = Frequency factor and

σ = Standard deviation

Where K= (YT -Yn)/Sn       	Yn = Reduced mean

Sn = Reduced standard deviation

Yn (12) = 0.51 Sn (12) = 1.02

K= (YT -Yn)/Sn

Values of reduced mean=Yn and reduced standard deviation=Sn are obtained at Table 4.5 from     a book of Rozgar Baban for sample size of 20 while YT is calculated from the above equation

The nearest meteorological station considered is Ebnat. Consequently, the different headwork component structures are designed based on the peak flood that will be formed from the computed heaviest storm for the Ebnat meteorological station.

### Time of concentration

Time of concentration has been calculated by taking the profile of the longest streamline and dividing it in to different reaches of similar slope. To calculate the time of concentration the water course is divided according to its slope .The longest water course is divided into 5 intervals and the time of concentration is computed using Kirpich formula.

Where Tc= time of concentration [Minute]

L= maximum length of flow [m]

S= the difference in elevation b/n the outlet and most remote point divided by the length L in m/m.

From the analaysis, which is calculated using kiproch, formula is 3.13hr

As the Tc is greater than 3hrs, the duration of the rainfall is taken 1hrs.

### Curve Number

Curve number (CN) was obtained based on USSCS method by watershed characterization in terms of vegetation cover, land use, hydrologic condition and soil group. From the watershed analysis the curve number at condition II is 74.6.

However, peak run off is estimated for antecedent moisture condition III and the curve number value at condition II should be converted to antecedent moisture condition III.

CN at Condition (III) = 88.6

## Flood Analysis by SCS Unit Hydrograph Method (Composite Hydrograph Method)

Techniques developed by the U. S. Soil Conservation Service for calculating rates of runoff require the same basic data as the catchment area, a runoff factor, time of concentration, and rainfall. The SCS-CN approach, however, is more sophisticated in that it considers also the time distribution of the rainfall, the initial rainfall losses to interception and depression storage, and an infiltration rate that decreases during the course of a storm.

With the SCS method, the direct runoff can be calculated for any storm, either real or fabricated, for small to medium-sized un gauged catchments by subtracting infiltration and other losses from the rainfall to obtain the precipitation excess.

The SCS-CN runoff equation is therefore a method of estimating direct runoff from 24-hour or 1-day storm rainfall and comprises the following relation as:

Where P is total precipitation, Ia is initial abstraction, Q is direct runoff and S is potential maximum retention, which can range (0, ∞). This method provides only the depth of runoff generated by a given rainfall from the catchment. However, for the design of hydraulic structures it is not the depth of runoff but the peak flow rate (L3T- 1), which is required. Thus the depth of runoff generated from the given rainfall is converted to run off hydrograph using synthetic unit hydrograph technique. In this technique, it is assumed that a rainfall of a certain recurrence interval generates discharge of the same recurrence interval. Rainfall of 50 years return period is first estimated and this rainfall is finally transformed to discharge hydrograph following the method discussed above. For further details of the method, reference is made to Hand book of Hydrological analysis (ESRDF MANUAL) Vol: IV

### Rainfall Profile

Rainfall profile is the distribution of the proportion of design rainfall during every incremental time on the watershed area during the 24 hours duration. Well-developed models are needed to determine such an event for the selected basin area. However, there are no sufficient modelling studies in the vicinity and adaptation of standard curves has been taken as the only option. Designer of this project has adopted the standard curve from Design Guidelines for Small Scale Irrigation Projects in Ethiopia. With the aid of rainfall profile versus duration curve the percentages of design rainfall distribution on the catchment area are computed for the first most intensive storm duration. The following table describes all the information about the areal rainfall and rainfall profile.

Table 1. 5: Design Rainfall Arrangement

| Duration ,D (hour) | Design point rainfall | Rain Fall profile (%) | Rain Fall profile (mm) | Areal to point RF (%) | Areal rainfall (mm) | Rainfall increment(mm) | Descending order | Rearranged order | Re -arranged RF(mm) | Comm. Rainfall (mm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0-1 | 79.25 | 44.00 | 34.8700 | 65.91 | 22.983 | 22.983 | 1 | 6 | 3.014 | 3.014 |
| 1-2 | 79.25 | 58.00 | 45.9650 | 73.91 | 33.973 | 10.990 | 2 | 4 | 4.507 | 7.521 |
| 2-3 | 79.25 | 67.00 | 53.0975 | 78.46 | 41.660 | 7.688 | 3 | 3 | 7.688 | 15.209 |
| 3-4 | 79.25 | 72.00 | 57.0600 | 80.91 | 46.167 | 4.507 | 4 | 1 | 22.983 | 38.192 |
| 4-5 | 79.25 | 76.00 | 60.2300 | 82.91 | 49.937 | 3.769 | 5 | 2 | 10.990 | 49.181 |
| 5-6 | 79.25 | 80.50 | 63.7963 | 83.00 | 52.951 | 3.014 | 6 | 5 | 3.769 | 52.951 |

### Runoff Analysis Using CN-SCS Method

Input data:

Design Point Rainfall = 79.25mm

Curve number at antecedent moisture condition III = 88.6

Catchment Area, A = 88.59 Km2

Tc =3.13 hrs, D = 1.0 hr., Tp = D /2+ 0.6*Tc = 2.38hrs; Tb = 2.67*2.38 = 6.35hrs,

Direct run-off,

Where, 	I = Rearranged cumulative run-off depth (mm)

S = Maximum run of potential difference, = (25400/CN)-254

Peak run-off rate for incremental runoff depth;

Where,  	A = Catchment area= Km2

Tp = Time to peak hr.

Q = Incremental run-off (mm)

Peak run-off for 1mm incremental runoff;

Table 1. 6: Computation of peak discharge for each incremental runoff

| Time increment | 0-1 | 1_2 | 2_3 | 3_4 | 4_5 | 5_6 |
| --- | --- | --- | --- | --- | --- | --- |
| RF. Profile, (%) | 44.00% | 58.00% | 67.00% | 72.00% | 76.00% | 80.50% |
| Area to point ratio, (%) | 65.91% | 73.91% | 78.46% | 80.91% | 82.91% | 83.00% |
| RF. Profile, (mm) | 22.98 | 33.97 | 41.66 | 46.17 | 49.94 | 52.95 |
| Incremental RF, (mm) | 23.0 | 11.0 | 7.7 | 4.5 | 3.8 | 3.0 |
| Descending order | 1.0 | 2.0 | 3.0 | 4.0 | 5.0 | 6.0 |
| Rearranged order | 6.0 | 4.0 | 3.0 | 1.0 | 2.0 | 5.0 |
| RF in rearranged incr. | 3.01 | 4.51 | 7.69 | 22.98 | 10.99 | 3.77 |
| Commulative RF. I, (mm) | 3.01 | 7.52 | 15.21 | 38.19 | 49.18 | 52.95 |
| Commulative runoff, (mm) | 0.00 | 0.03 | 1.82 | 15.58 | 24.14 | 27.24 |
| Incremental run-off, (mm) | 0.00 | 0.03 | 1.79 | 13.76 | 8.57 | 3.09 |
| Peak run-off for incr. | 0.00 | 0.23 | 14.00 | 107.62 | 67.03 | 24.20 |
| Time to begin , (hr) | 0.0 | 1.0 | 2.0 | 3.0 | 4.0 | 5.0 |
| Time to peak | 2.38 | 3.38 | 4.38 | 5.38 | 6.38 | 7.38 |
| Time to end. | 6.35 | 7.35 | 8.35 | 9.35 | 10.35 | 11.35 |

Table 1. 7: Peak Discharge Analysis Using Complex Hydrograph Method

| Time | Q1(0.5) | Q2(1) | Q3(1.5) | Q4(2) | Q5(2.5) | Q6(3) | Base flow | Qtotal |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 0.00 |  |  |  |  |  | 0.184 | 0.2 |
| 1 | 0.00 | 0.00 |  |  |  |  | 0.184 | 0.2 |
| 2 | 0.00 | 0.06 | 0.00 |  |  |  | 0.184 | 0.2 |
| 1.5 | 0.00 | 0.11 | 2.94 | 0.00 |  |  | 0.184 | 3.2 |
| 2.0 | 0.00 | 0.17 | 5.89 | 22.63 | 0.00 |  | 0.184 | 28.9 |
| 2.5 | 0.00 | 0.23 | 8.83 | 45.26 | 14.09 | 0 | 0.184 | 68.6 |
| 3.4 | 0.00 | 0.18 | 14.00 | 84.99 | 38.84 | 8.94 | 0.184 | 147.1 |
| 3.9 | 0.00 | 0.15 | 12.24 | 107.62 | 52.94 | 14.03 | 0.184 | 187.2 |
| 4.4 | 0.00 | 0.13 | 10.48 | 94.07 | 67.03 | 19.11 | 0.184 | 191.0 |
| 4.9 | 0.00 | 0.10 | 8.71 | 80.52 | 58.59 | 24.203 | 0.184 | 172.3 |
| 6.3 | 0.00 | 0.03 | 3.53 | 40.65 | 33.76 | 15.24 | 0.184 | 93.4 |
| 6.8 |  | 0.00 | 1.76 | 27.10 | 25.32 | 12.19 | 0.184 | 66.6 |
| 7.3 |  |  | 0.00 | 13.55 | 16.88 | 9.14 | 0.184 | 39.8 |
| 7.8 |  |  |  | 0.00 | 8.44 | 6.09 | 0.184 | 14.7 |
| 8.3 |  |  |  |  | 0.00 | 3.05 | 0.184 | 3.2 |
| 8.8 |  |  |  |  |  | 0.00 |  | 0.0 |

From the analysis, the fifty years return period design flood is 191m 3/s in the above table, the ordinates of every triangular hydrographs of incremental runoff are calculated by applying the concept of similarity of a triangle. This is done for known values of   "time to begin, time to peak, time to end and peak run off" to every hydrograph. Here independent formulas are used for the rising and recession limbs to compute runoff magnitudes among the time to begin, time to Peak and time to end.

---------------used to compute the values of flood on the rising limb, and

---------------used to compute the values of flood on the recession limb

Where: Qi = Runoff value at any time‘t’ between time to begin and peak for H1,H2, H3,....or H6 triangular hydrograph, m3/sec

Qp = Peak runoff value for H1, H2, H3 ...or H6 triangular hydrograph, m3/sec

Qj = Runoff value at any time 't' between time to peak and end  for H1, H2, H3 ...or H6 triangular    hydrograph, m3/sec

To = Beginning time of every incremental runoff, hrs

Ti = Instantaneous time between time to begin and peak, hrs

Tp = Time to peak runoff, hrs

Tj = Instantaneous time between time to peak and end, hrs

TE = Time to end, hr

Figure 2-1: Incremental hydrographs curves

From the analysis, the 50 year return period design run off is 168.5 m 3/s

## Tail Water Depth Computation

The analysis of tail water depth is very important for the determination of maximum water depth before the construction of a hydraulic structure. It is also used to determine the required river training work to prevent the entrance of flood to the Diversion.  The tail water depth is determined based on manning’s equation. Hence, the wetted area, perimeter, manning’s roughness coefficient and the river slope are the main important data required for the analysis. Wetted area and perimeter can be easily determined from the river cross-section with the specified depth of flow.

### Average river bed slope

For Bahir Libo Diversion, there is a need to determine the slope of the river at proposed location. Hence, the Average riverbed slope at the proposed Diversion is estimated by two different techniques, one is by end area method and the other is by using best-fit line method. Designers have adopted the end area method output for further analysis. The result estimated using the surveying data and it was found to be 0.009 by using End area method and 0.008 best fit  respectively for comparison of the two result refer the attached excel file and the figure 2.2.

Figure 2-2: River profile

### Manning’s Roughness Coefficient

The river channel is well defined and the river bed is covered with recently transported materials while the river banks are covered with highly fragmented rock.The Manning`s roughness coefficient is taken from standard table based on the river nature. The river reach at the headwork site has meandering nature and made of alluvium deposits at the bed with small size boulders and cobbles. The banks are also defined and it is stable. Hence, a Manning`s roughness coefficient (n = 0.04) has been adopted.

### Tail Water depth analysis

Input data : Manning’s roughness coefficient, n =0.04 and Average river bed slope, S = 0.009

Manning Equation for velocity:

Where, R = Hydraulic radius = (Area/Perimeter)

Discharge:

1.8 1: Tail Water Depth for Bahir Libo Diversion

| Elevation | Depth(m) | Area(m2) | Slope(S) | Manning Roughness(n) | wetted perimeter(m) | Hydraulic radius R (m) | Velocity V (m/sec.) | Discharge Q (m3/sec.) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1645.18 | 0 | 0.0 | 0.009 | 0.04 | 0.0 | 0 | 0 | 0 |
| 1645.41 | 0.23 | 0.3 | 0.009 | 0.04 | 2.4 | 0.13 | 0.60 | 0.18 |
| 1645.91 | 0.73 | 2.5 | 0.009 | 0.04 | 6.8 | 0.37 | 1.23 | 3.12 |
| 1646.41 | 1.23 | 6.9 | 0.009 | 0.04 | 11.3 | 0.61 | 1.71 | 11.89 |
| 1646.91 | 1.73 | 13.4 | 0.009 | 0.04 | 14.9 | 0.90 | 2.22 | 29.70 |
| 1647.41 | 2.23 | 21.1 | 0.009 | 0.04 | 17.1 | 1.23 | 2.73 | 57.54 |
| 1647.91 | 2.73 | 29.8 | 0.009 | 0.04 | 19.4 | 1.54 | 3.16 | 94.14 |
| 1648.41 | 3.23 | 39.4 | 0.009 | 0.04 | 21.6 | 1.83 | 3.55 | 139.92 |
| 1648.91 | 3.73 | 50.1 | 0.009 | 0.04 | 23.9 | 2.10 | 3.89 | 195.15 |
| 1648.97 | 3.79 | 51.4 | 0.009 | 0.04 | 24.1 | 2.13 | 3.93 | 202 |
| 1649.41 | 4.23 | 61.9 | 0.009 | 0.04 | 26.5 | 2.34 | 4.19 | 258.98 |
| 1649.91 | 4.73 | 75.4 | 0.009 | 0.04 | 31.9 | 2.36 | 4.21 | 317.51 |
| 1650.41 | 5.23 | 92.1 | 0.009 | 0.04 | 38.6 | 2.38 | 4.24 | 390.29 |
| 1650.91 | 5.73 | 112.0 | 0.009 | 0.04 | 45.3 | 2.47 | 4.34 | 486.71 |
| 1651.41 | 6.23 | 135.3 | 0.009 | 0.04 | 52.0 | 2.60 | 4.50 | 608.51 |
| 1651.91 | 6.73 | 161.8 | 0.009 | 0.04 | 58.6 | 2.76 | 4.67 | 756.56 |
| 1652.41 | 7.23 | 190.4 | 0.009 | 0.04 | 61.0 | 3.12 | 5.08 | 966.34 |

Figure 2-3: Tail water rating curve

Tail water curve

Post jump

Figure 2-4:-Tail water curve versus post jump curve

From the above table and Tail water rating curves, the tail water depth equivalent to the peak flood discharge (i.e. Q =191m3/sec) is found to be 3.73m (1648.91Masl).

The computed tail water depth  indicates that the river meet its maximum flood (Q 191m3/sec) at a level of 1648.91masl which is 3.71m above the river bed level before construction of headwork so during construction the upstream flood will rise up  because of the hindrance of the structure. As we observe from the above graph post jump curve is above tail water curve, due to this higher energy created at the apron so that, we should excavate the downstream apron level until both the curves over lay each other. In addition energy stilling mechanism should be provided at the end of the apron like end sill.

## Upstream Water surface profile

The importance of doing the upstream water surface slope is to know the extent of the backwater effect of the water due to the construction of weirs.

Yn= ((q*n)/(s^0.5)) ^3/5

∆ₒ=H+Hd-Yn

Y= (XS-2∆ₒ)^2/(4∆ₒ)

X Extends up to= 2∆ₒ/S

Where:

∆ₒ=Rise of water above normal depth at the weir siteX=0

S=Slope the river bed

X=Distance from the crest to the point where Y is required to be determined

Y=Water rise at distance X upstream of the weir above the normal water depth

Yn=Normal water depth

Yn= ((q*n)/(s^0.5))^3/5 = ((7.77*0.04)/(0.009^0.5))^3/5  =2.03m

∆ₒ=H+Hd-Yn   =3.1+2.34-2.03=3.36

Y= (XS-2∆ₒ)^2/(4∆ₒ) =747.48*0.009-2*3.36)^2/(4*3.36) =0

X Extends up to= 2∆ₒ/S =2*3.36/(0.009) =747.4m

Figure 2-4: upstream water surface profile.

From the figure above the backwater, effect due to the construction of weir reaches up to 747m back and has no effect for the community it lays within the banks of the river.

# HEADWORK STRUCTURES DESIGN

## Headwork Site Selection

The headwork site is situated at 1376602m N, 411036 m E and elevation of 1647m above sea level by diverting water from the Bahir Libo River. To make the irrigation system as efficient as possible, proper planning and design of the Diversion headwork is vital. To this effect field visit was undertaken to get data and information mainly on the following areas:

- Appropriate Diversion site identification and selection

- Topographic map preparation for the irrigation system design of head work structures and the irrigation system layout

- Geological and geotechnical investigations for foundation stability, river bank conditions, construction materials etc.

Hydrological assessments, mainly interview of the local elder people as regard to flooding conditions during the rainy season and during river flows in the year.

The location of a diversion headwork is determined by considering different factors as indicated below.

- Hydrologic and hydraulic considerations

- Existing foundation conditions

- Access requirements,

- Bank conditions, etc.

In  this  particular  case,  it  is  tried  to  consider  the  above  and  related  factors  during  site selection by the team.

Maximum efforts were made by considering various alternatives to identify relatively acceptable sites for the head works construction.

Hydraulic requirements: Head works should be located on a relatively stable river reach keeping the flow within the reach.  During  the  field  visit,  the  team  has  thoroughly  discussed  and  seen  various  alternatives and selected the best option based on stability.

Topographical requirements: The selected site should possibly minimize the conveyance length. The topography on the selected Diversion headwork is not flat.

Geotechnical  requirements:  The  channel  bed  and  banks  at  the  site  should  be  stable resistant  soils,  with  minimum  permeability  and  sufficient  bearing  capacity.  The risk of percolation around the planned retaining walls should be minimal.

Finally, accessibility from an existing road and availability of local construction materials is also considered during the field visit.

The headwork site geological surface and subsurface conditions have been investigated based on the nature of the proposed structure. At the site and immediate vicinity, the stream flows along moderate slope course. At this site the river course is well defined, matured with fixed width and forms nearly a U-shaped valley. The river streambed is covered with thick unit of transported sediment following the rocky cliff of the river banks such deposit covers from bank to bank in the downstream and upstream directions. On the other hand, the right bank of the stream at the headwork site is made up of different geologic materials; both banks have well defined vertical well with moderately to highly weathered and jointed basaltic agglomerate. The river is flowing through a defined channel and its scouring effect on the river course is significant on none bed rock sections of the river section.

## Geological condition of the river

For this project Diversion weir has been proposed for diverting the water from the river to the command area.  According to the Engineering Geological report of the project, The headwork site geological surface and subsurface conditions have been investigated based on the nature of the proposed structure. At the site and immediate vicinity, the stream flows along moderate slope course. At this site, the river course is well-defined, matured with fixed width and forms nearly a U-shaped valley. The river streambed is covered with thick unit of transported sediment following the rocky cliff of the riverbanks such deposit covers from bank to bank in the downstream and upstream directions. On the other hand, the right banks of the stream at the headwork site are made up of different geologic materials; both banks have well defined vertical well with moderately to highly weathered and jointed basaltic agglomerate.

The river is flowing through a defined channel and its scouring effect on the river course is significant on none bed rock sections of the river section. The different sections of the stream at the proposed headwork site are described separately below:-

## Left and Right Banks

The engineering geological foundation conditions at both banks are nearly similar, except variation in extent or dimensions of the rock units.

It has steep slope nearly vertical well-defined rock banks. The surfaces of both banks are covered with moderately to highly weathered and jointed rock. This rock has a characteristic of light yellowish color coarse grain basaltic agglomerate. These rock units have steep slope nature with widely spaced joints with maximum opening of 2cm. Beside to its jointed nature it undergoes moderately to highly weathering stage. On the right bank, it has up to 5m height while in the left bank it reaches up to 8m. At about 7m upstream, the stream channel minders at a bank of highly jointed and weathered nature. Hence, both banks are unstable and pervious protection works are necessary 20m in the upstream and downstream directions (Source: Engineering Geological Report).

Figure 3-1: Right Banks                                                                                                      Figure 3-2: Left Banks

## River Bed

At the proposed headwork site the stream bed or course is well defined, nearly straight, and minder at about 7m upstream from headwork site.  Along the weir axis, the  bed is made up of one major geologic materials, as seen from surface observation. These are all sized recently deposited alluvial deposit.

On the proposed weir axis and some K.ms upstream and downstream, the stream is covered with thick unit of transported sediment. At the foundation test was taken (B.Libo Fo.T.p_1), but due to in flow of water through these thick sediment test was interrupted at shallow depth (0.5m). Due to these generally the structure are founded on these thick alluvial sediment. Hence, from test pit taken on the foundation area, samples are analyzed at ADSWE soil laboratory for gradation test.

Since the structure are founded on these thick alluvial deposit, effective grain size (Silt Factor) for such deposit are taken 3mm from gradation test of foundation sample(Source: Engineering Geological Report).

Figure 3-3: Thick unit of alluvial deposit and test pit taken on the stream bed of Bahir libo at the weir axis.

Figure 3-4: Geological X_ Section of Bahir Lib headwork axis.

## Sources of construction materials

During the site investigation, natural construction materials required for the construction of the various proposed engineering structures at the headwork and within the farmland have been assessed, and possible quarry sites and borrow areas have been identified within the vicinity of the area as much as possible. In addition to the identification, the quality, quantity, accessibility condition and ownership of each proposed production sites have also been studied and described in this report; on separate sub-sections below. The natural materials required for the construction of the proposed hydraulic structures include rock for masonry stones, aggregates (both coarse and fine), impervious soil for fill and/or lining, backfill soil, and water.

### Rock for Masonry and Crushed Coarse Aggregate

Quarry site that can be used for production of rock for masonry stone and crushed coarse aggregates has been assessed during the field work session within the vicinity of the project area at economic distance for hauling. One possible quarry site has been identified within left side/bank of the headwork at about 200m and 650m downstream following a continuous ridge bordering the left side of Bahir Libo stream. The rock is located in the same Kebele, at Zenaboch and Sheha  “Got” to the project respectively.

Figure 3-5: Rock Quarry Site for Masonry Stone and Crushed Aggregates.

Naturally, this rock is strong as it is generally fresh except very thin cover of weathered zones, which need to be removed during rock dressing and before crushing From this site, rock of different sizes can be produced; ranging from large boulders to coarse gravels that masonry stone and coarse aggregate can be produced easily.

### Fine Aggregates

Borrow areas for fine aggregate or natural sand have been assessed starting from the project stream itself. Natural deposits of such materials could not be found when assessed within the beds of the stream in the project area; rather very coarser sediments and rock exposures are found covering almost the entire bed of the Bahir Libo stream. Seeing to this nature of the stream, other distant streams have been explored to identify the best source areas for fine aggregate or natural sand that can be used for this particular project. During exploration of this natural sand, at a distant one stream was identified as a possible source of fine sand. The stream is known as ‘Mena’. It is located at about 90km from the project site, within Arbaya Belesa Woreda.

Both visual investigation and laboratory testing of the sand deposit found in the proposed area have been done.  It has more than 60% of sand with almost nil percent of silt and clay even if it has significant percent of Gravel.  In the area good quantity, which suffices the need of this project, is available. Sand sized transported alluvial sediments dominate the deposit, though some amount over sizes is also observed. The individual grains of sand are mainly originated from basalt rock and reveals high crushing resistant that the sand has good quality in this respect.

The samples have been analyzed at Amhara Design and Supervision material testing laboratory to characterize the gradation of the sand deposit. According to these laboratory test results (See Annexture-2).

Table 2. 1: Short summary of Mena sand la

| Sand Sources | Mena Pump_sample_1 | Mena Pump_Sample_2 |
| --- | --- | --- |
| Gravel | 37.69 | 20.05 |
| Coarse Sand | 58.32 | 61.14 |
| Fine Sand | 3.38 | 16.35 |
| Silt and Clay | 0.61 | 2.46 |

Figure 3-6: Sand Sources in Mena Stream.

### Canal Fill Material

Along the main canal route, there are some places where the original ground level is lower than the designed canal bed level (CBL). In this case the OGL need to be filled till at least to the CBL.

For such fill, natural fine grained soil material having low to medium plasticity, non-expansive and impervious is required. For this project such soil material source/borrow area is identified at about Tire kebele at about 1.8km from the headwork Specific Got of Bisrako

From visual examination of the borrow area, the soil has reddish brown color, and composed of dominantly Clay with Gravel. It is residual soil having low plasticity and low liquid limit. It is non expansive soil.

According to the Unified Soil Classification System, the soil group lies in Low plastic Clay (CL). Such soil type possesses a maximum dry density (MDD) lies within 1.80gm/cc, and the associated optimum moisture content (OMC) 17.53%. The construction of the fill can be done using these parameters

### Water

Water for construction purposes can be found from the project stream, itself. The stream is perennial throughout the year that there is some amount of flow along its course.

## Headwork Type Selection

Looking the availability of natural construction materials and considering the river features and expected flood amount, Broad crested type of Weir is chosen. As it is:

- High flood

- There is cobbles and small boulder transported.

## Hydraulic Design of Headwork Structure

### Weir Height Determination

The following major factors have been seen in determining the Weir crest level:

- Maximum  command area elevation

- Deriving head of the intake structure

- Main canal slope

- Loss

- Lowest Point of river center

Table 2. 2: Weir height determination.

| 1. River Bed Level at weir axis | 1645.18 | From River x-section |
| --- | --- | --- |
| 2. Canal length from outlet to the maximum command area | 440 | From topo map |
| 3. Average level of the highest field of the command area | 1647.00 | Topo map & observation |
| 4. Water depth required at canal outlet | 0.50 | Hydraulic computation |
| 5. Free board at canal outlet | 0.20 | Assumed Loss |
| 6. Head loss across the field | 0.05 | Estimated Loss |
| 7. Head loss at the turnout | 0.05 | Estimated Loss |
| 10. Head loss across head regulator | 0.10 | Estimated Loss |
| 8. Canal slope | 0.001 | Hydraulic computation |
| 9. (Canal slope) * (Canal Length) | 0.44 | Canal slope Loss |
| Total Loss | 0.64 | Summ of all Losses |
| 11. Crest level of the weir = Command area Level + Total Loss | 1648.34 |  |
| 12. Weir height = Crest level – River bed level | 3.1 |  |

Finally, the weir height is fixed to be 3.16m and the weir crest level is fixed to be 1648.34m.

### Base flow of the River

Study team has calculated flow of the river at the Weir site measured on November 2015 as 184 l/s.  Out of this, 147 l/s will be required for the proposed scheme and the rest will be released for downstream use.

### Flow over the Weir crest

a. Crest Length

- Lacey’s regime width, L=4.75*Q,  =4.75*191 = 65.55 m.

- Actual river section width of the over flow section of the river is = 28m

- Since the river section is not fully alluvial formation, designers adopted the actual river width. Out of this, 2m has been left for under sluice and divide wall provision and the net overflow section shall be 26m.

b. Discharge over the Weir section

- Design discharge, Q = 191 m3/s from the complex unit hydrograph

### Top and bottom width of weir

According to the Beligh’s formula, top and bottom width of the weirbody is determined as follows

- Input Data:

P: Height of weir (m) = 3.1m

He: specific energy head (over flow depth + approaching velocity head (m)) =2.525

: Specific weight of weir body (2.3 for cyclopean concrete)

Top width, 2.2m but we take 1m

Bottom width,

Provide 1m and 3.1m top and bottom width respectively, which will be tested for adequacy during stability analysis.

### U/S and D/S HFL Calculation & Determination

From the stage –discharge curve prepared (Section I, in Hydrology Part) the high flood level before construction (i.e. D/s HFL) corresponding to the design flood is 1648.91m a.s.l.

D/s HFL=1648.91m amsl------------------------------------- (a)

U/s HFL = U/s bed level+weir height+Hd-------- (b)

Hd is the depth of water over the weir crest. This is calculated by assuming broad crested weir formula.

Q=C*L*He32

He=QC*L23= 2.525

The velocity head, ha is computed from the approach velocity as shown below

Where g: acceleration due to gravity = 9.81m/sec2

Va is Approach velocity determined by

L is Weir crest length =26m,

hd is flow depth over the weir and also,

By trial and error method, hd is found to be 2.35m

- ha = He-hd = 2.525m-2.35m = 0.175m

- Velocity head, ha = 0.175m

- U/s TEL=weir crest level +Specifc energy head(He) =1648.20+2.525=1650.725

U/s HFL =U/s TEL –velocity head =1650.725m a.s.l – 0.175m = 1650.55m a.s.l

- Afflux

⇒ Afflux = U/s HFL- D/s HFL = 1650.525m a.s.l – 1648.91 m a.s.l = 1.64m.

The afflux is somewhat maximum, but we adopt it because the effect is almost nil due to high  bank levels at the upstream and the flood do not overtope.In addition to minimize the effect of afflux at downstream we have to take care at the apron thickness and providing end sill to decrease retrogression effect.

### Hydraulic Jump Calculation

As discussed in the geologic report,the river bed is alluvial deposit and hence stilling basin for energy dissipation is required. Both left and right side banks are not sound rock, a wing walls are required at u/s and D/s sides, to protect the scouring of the bank due to the formation of jumps, and not to flow the river out of riverbank in high flood cases.

The length of wing walls is determined based on the length of Jump, and it is calculated as shown below.

EGL

A	B

Ha

h	He	ha

Z		Y2

y1

Figure 3-7: Flow over weir body.

• Weir crest length = 26m

• Weir height = z = 3.1m

• Pre-jump depth = y1=0.61

• Post -jump depth =y2=4.02m

Neglecting losses between point A and B and considering similar datum

z + He = y1 + ha

but,  He = 2.525m

q=Ql=191m3/s26m=7.34m3/s/m

ha=q22*g*y2=7.3422*g*y12

3.1m+2.525m=y1+2.74/y12

After iterations Y1=0.61m

V1=q/y1=7.34/0.61=12m/s

Fr=V1gy1=129.81*0.61=4.91

y2=y121+8*Fr2-1,y2=0.6121+8*4.91^2-1 =4.02m

So, hydraulic jump length (L) for Fr=4.91from the graph L=5*(y2-y1)=5*3.63=17.1m

### Impervious floor Length

#### D/s impervious floor Length (LD)

For under seepage the worst condition would be when the water on the upstream side is at the level of the weir crest & there is no tail water. Seepage head loss at

- Pond level case:

Hs = crest level –down stream bed level =1648.34-1644.9

= 3.44m

- Maximum flood case:

Hs = U/s HFL- D/s HFL

= 1650.553-1648.91m

= 1.64m

Take the maximum of the two, so that Hs=3.44m

Therefore maximum seepage head occurs when water is stored up to the pond level and there is no water on the d/s.

= Bligh’s constant, Cb is depending on the type of the foundation.

Ld=2.2*Cb*Hs10,Ld=2.2*9*3.4410 =11.6  take 17.1m

Total Required creep length, LT = C*HL=9*3.44=30.96m

Therefore, D/s impervious floor is taken to be 17.1m long.

#### U/S Impervious Floor Length, (Lu)

U/S Apron length, L2 =LT- (L1+2*d1+2*d2+B)=1m

Therefore, total length of the u/s impervious floor is taken 1m long.

#### Floor thickness (t)

The apron thickness that we are provided for the floor is indicated below

Table 2. 3: Floor thickness of dimensions.

| Location | Creep Length up to the point |  | Uplift head, (h) | Thickness, (t) | Remarks |
| --- | --- | --- | --- | --- | --- |
|  | Formula | Value | h=HL - GE*L | t= 4/3*h/(G-1.0) | Adopted value |
| Heal of the weir (K) = 2*d1 | 2*1.5 | 3 | 3.11 | 3.0 | 0.80m b/s there is counterbalance water load |
| Toa of the weir (L) = 2*d1+L2+3.31 | 2*1.5+1+3.31 | 7.31 | 2.63 | 2.5 | 1.5 |
| Apron (M) = 2*d1+L2+3.31+5 | 2*1.5+1+3.31+5 | 12.31 | 2.08 | 2.0 | 1.2 |
| Apron (N) = 2*d1+L2+3.31+5+5 | 2*1.5+1+3.31+5+5 | 17.31 | 1.53 | 1.5 | 0.7 |
| Apron (O) = 2*d1+L2+3.31+5+5+6.64+2d2 | 2*1.5+1+3.31+5+5+6.64 | 30.95 | 0.0 | 0.0 | 0.5 |

### Cut off Depth Calculation

#### U/s cut off

- Q=191m3/sec

- q=7.34 m3/s/m

- Silt factor can be calculated by using Lacey’s formula

Where:-

f is Lacey’s silt factor

f = 1.76 * d50^0.5 and ‚d‛ is the mean diameter of the bed material (in mm) and

“q” is the discharge per unit width of channel.

d50=3mm taken from the geology report of this project (gradation test of foundation sample)

f=3.05mm

Scour depth(R)	 =1.35*(q2/f)(1/3)

R=1.35*(q2/f) (1/3) =3.48

Hence bottom level of u/s cut off=U/S HFL–1.5*R= 1650.73-1.5*3.48=1645.51

Cut off depth=RBL-Bottom level of the upstream cut off  =1645.18-1645.51=0.33m but take 1.5m

#### D/s cut off

- Q=191m3/sec

- q=7.34m3/s/m

- Silt factor, f=1.76*(d50)0.50

f=3.05

- D50=3mm

- R=1.35*(q2/f)(1/3)

R=1.35*(q2/f) (1/3) =3.48

Hence bottom level of d/s cut off=D/S HFL–2*R==1648.91-2*3.48=1641.95

Cut off depth=RBL-1641.95=1645.18-1641.95=3.23 take 3.5m

## Stability Analysis of weir

Stability analysis is carried out to see the already determined weir section is safe against overturning, sliding, tension.

The stability analysis is carried out considering the effect of the following forces.

• Water pressure

• Weight of the over flow weir section

• Sediment load

The extreme load combination is the case where the head is at crest level of the weir and there is no flow over the weir (static case)

Table 2. 4: Moment and Load Calculation result

| Unit weight of materials |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Cyclopian Concrete, ϒc |  |  | 23.5 |  | KN/m3 |  |  |  |
| Water, ϒw |  |  | 9.81 |  | m2/s |  |  |  |
| Silt, ϒ'sV |  |  | 9.07 |  | KN/m3 |  |  |  |
| Silt, ϒ'sH |  |  | 3.53 |  | KN/m3 |  |  |  |
| HFL Level Condition: |  |  |  |  |  |  |  |  |  |  |
| Load | Force |  |  |  |  | Lever Arm | Moment |  |  |  |
|  |  | Vertical |  | Horizontal |  |  | Positive | Negative |  |  |
| Weir Body Cyclopean Load, W1, | b*P*ϒc | 72.85 |  |  |  | 2.60 | 189.41 |  |  |  |
| W2 | 0.5*P*b'*ϒc | 76.49 |  |  |  | 1.40 | 107.09 |  |  |  |
| Water Load, Ww |  | 22.56 |  |  |  | 2.60 | 58.66 |  |  |  |
| Water Pressure, PW1 |  |  |  | 47.14 |  | 1.55 |  | -73.06 |  |  |
| PW2 |  |  |  |  |  |  |  |  |  |  |
| Tail Water PT |  |  |  | 2.91 |  | 0.26 | 0.75 |  |  |  |
| Silt Load, PsH |  |  |  | 16.96 |  | 1.03 |  | -17.53 |  |  |
| Uplift pressure | u1 |  |  | 2.39 |  | 1.55 |  | -3.70 |  |  |
| Uplift pressure | u2 |  |  | 3.61 |  | 2.07 |  | -7.46 |  |  |
| Sum |  | 171.91 |  | 73.01 |  |  | 355.91 | -101.75 |  |  |
| ` |  |  |  |  |  |  |  |  |  |  |
| Over turning = (M+ve/M-ve) >1.5 |  |  |  |  |  | Fo | 3.50 | >1.5 |  |  |
| Sliding = (µxFv/Fh), |  |  |  |  |  | Fs | 1.65 | >1.5 |  | OK |
| Tension: X= (Net Moment/Sum Fv), e=x-B/2, e<B/6 |  |  |  |  |  | X | 1.48 |  |  |  |
|  |  | B/6= |  | 0.516667 |  | e | 0.07 | e<B/6 |  | Ok |

i) Factor of safety against overturning (Fo)

>1.5 Safe!

ii) Factor of safety against sliding (FS)

,

>1.0 Safe!

iii) Check for tension (i.e. whether the resultant lies within the middle third)

The location of the resultant force from the toe is given by

The eccentricity (e) = X – B/2, B = 3.1m

Hence, e = 1.48-3.1/2 = 0.07m

The eccentricity (e) should be less than B/6 =, Hence the obtained e = 0.07<0.51m.

⇒The resultant lies within the middle third                  no tension

## Design of Divide wall, Under Sluice, and Canal outlet

Divide wall is designed in order to create separation between outlet canal and natural river course. The divide wall allows safe and stable base flow to the canal outlet. Flow turbidity created by current flow impact over the weir body is reduced.

#### Wall height fixation of Divide Wall

The existing topographical condition at the weir axis and HFL are considered most governing parameters for fixing the wall height.

After construction of the weir (u/s HFL) = 1650.73m.a.s.l

Hence the level of divide wall and weir body foundation should be the same and is 1645.18

U/s wall height = U/s HFL - foundation level.

U/s wall height = 1650.73m.a.s.l – 1645.18m + 0.4m =5.95 m

Provide5.95 m wall height and the top level of the divide wall =1645.18+5.95m =1651.13masl.

D/s wall height = D/s HFL - foundation level

=1648.91m.a.s.-1645.18ma.s.l=3.73 take 4.1m

Provide 4.1 m wall height and the top level of the divide wall =1645.18+4.1=1649.28m.a.s.l

Table 2. 5: Divide wall Stability analysis result

| Analysis |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| HFL Level Condition: |  |  |  |  |  |  |  |
| Load | Force |  |  | Lever Arm | Moment |  |  |
|  | Formula | Vertical | Horizontal |  | Positive | Negative |  |
| Weir Body Masonry Load, W1, |  | 70.20 |  | 0.67 | 46.80 |  |  |
| W2 |  | 140.40 |  | 1.00 | 140.40 |  |  |
| W3 |  |  |  | 1.50 | 0.00 |  |  |
| W4 |  |  |  | 0.67 | 0.00 |  |  |
| Water Load, Ww1 |  | 15.21 |  | 2.00 | 30.41 |  |  |
| Ww2 |  | 0.00 |  | 1.50 | 0.00 |  |  |
| Ww3 |  |  |  | 0.00 | 0.00 |  |  |
| Ww4 |  |  |  | 0.50 | 0.00 |  |  |
| Silt Load, Ws' |  |  |  | 2.00 | 0.00 |  |  |
| Water Pressure, F1 |  |  |  | 2.93 |  | 0.00 |  |
| F2 |  |  | 26.86 | 1.00 |  | -26.86 |  |
| Tail Water Pressure, F3 |  |  | -47.14 | 1.03 | 48.71 |  |  |
| Silt Load, PsH |  |  |  | 1.95 |  | 0.00 |  |
| Uplift Pressure, U1 |  | -60.82 |  | 1.00 |  | -60.82 |  |
| U2 |  | -5.00 |  | 1.33 |  | -6.67 |  |
| Sum |  | 159.98 | -20.28 |  | 266.32 | -94.35 |  |
| ` |  |  |  |  |  |  |  |
| Over turning = (M+ve/M-ve) >1.5 |  |  |  |  | Fo | 2.82 | >1.5 |
| Sliding = (µxFv/Fh), |  |  |  |  | Fs | 6.43 | >1.0 |
| Tension: X= (Net Moment/Sum Fv), e=x-B/2, e<B/6 |  |  |  |  | X | 1.07 |  |
|  |  |  | B/6= | 0.33333 | e | 0.07 | e<B/6 |

#### Under sluice

The under sluice is mainly provided here to remove silt deposition as a result of barrier structure and guide base flow water to the canal outlet. Hence the sill level of the under sluice is fixed to facilitate this deposited silt to increase the efficiency of water abstracting to the main canal through the head regulator from the pocket.

The sill level of this under sluice is fixed at minimum bed level of the river. Hence the sill level of the under sluice = 1646.48m Considering manageability, the opening size of the gate is 1m*1m with spindle type operating from the operation slab.

Considering rectangular notch profile of flow of water at the under sluice, the discharge passing is computed using the following formula.

- The capacity should be at least five times the canal discharge to ensure proper scouring.

- Capacity of passing about 10% to 20% of the maximum flood discharge at high floods.

- During construction, it should be able to pass the prevailing (at least base flow) discharge of the river.

But the practical application of those values is not sound so the discharging capacity of under sluice is determined by using orifice flow formula for maximum flood condition.

i.e. Q=Cd*A*2gh,

Where, Cd = Discharge coefficient =0.70

A = under sluice opening area (=1*1 =1 m2)

g = 9.81m/s2

Width (b) = Qd Cd*H2g*h=0.1911.7*12*9.81*0.5=   =0.035m take 1m

Hence the size of the under sluice is 1m×1    (height *width) considering construction condition.

#### Canal outlet level

The head regulator is provided on the left side .The sill level of this head regulator is fixed from different angles observations. The main conveyance system is more than 3km, which passes a number of gullies and undulating route. Hence, this level is fixed based on the optimum route alignment and the maximum irrigated command level including minor and major losses criteria. Based on this condition, the sill level is fixed to be 1647.5m

Outlet capacity

The minimum command area is determined by the minimum flow of the river. But the canal capacity should be determined for maximum command area and the corresponding discharge. In this case, the outlet capacity is fixed considering maximum duty and command area

Outlet capacity = Duty x command area

Where, duty for 18 hr irrigation = 1.41L/s/ha

Command area = 104ha.

- Outlet capacity = 1.41 L/s/ha x 104ha = 147L/sec.

Outlet size

From the weir/intake discharge, formula the outlet size is determined as follows

Q = CLHe3/2

Where; C = coeff. of discharge = 1.7

L = Length of water way (m)

He = head above sill level (neglecting the velocity head) = 0.5m

⇒Adopt waterway length 0.55m considering construction case.

Hence, provide an outlet size of 0.55m x 0.5m (Widith x height) providing 0.3m free board.

The gate of the off take canal is to be vertical sheet metal of 0.55m x 0.5m for the closure of the opening space.

### Breast Wall and Operation Slab

A vertical raised gate is designed for the head regulator and under sluice. These gates slide over the breast wall-using spindle during opening and closing.

For easy operation of these gates, operation slab is provided. The size of the operation slab is fixed from the point of construction and free movement. The size of the operation slab is shown in the drawing for both head regulator and under sluice with thickness 0.2m.

The thickness of the breast wall is also the same as that of the operation slab. The nominal thickness is fixed from the point of construction rather than the imposed load. The thickness required for the imposed load is less than the nominal value and treated as cantilever retaining wall. For the breast wall, the minimum reinforcement area is taken as 15% along the respective direction.

Refer drawings for Reinforcement arrangements (Drawing numer-01)

## Retaining Walls

River training works are required near the weir site in order to ensure a smooth and an axial flow of water, and thus, to prevent the river from outflanking the works due to a change in its course.

The rock units have steep slope nature with widely spaced joints with maximum opening of 2cm.

Beside to its jointed nature it undergoes moderately to highly weathering stage to safeguard the structure from scour, damage and to make the structure sustainable we are design masonry retaing wall is designed on both side having different dimensions for upstream and downstream side of the river.

Table 2. 6: retaining wall Design input parameters.

| Wing Wall Design |  |  |  |  |
| --- | --- | --- | --- | --- |
| U/s River bed level |  | 1645.18 |  |  |
| D/s river bed level |  | 1645.18 |  |  |
| D/s Apron Level |  | 1644.9 |  |  |
| Weir crest level |  | 1648.34 |  |  |
| U/s HFL |  | 1650.73 |  |  |
| D/s HFL Tail water |  | 1648.91 |  |  |
| Post Jump Level |  | 1646.38 |  |  |
| Max D/s HFL |  | 1648.97 |  |  |
| Free board for U/s wingwall |  | 0.4 |  |  |
| Free board for D/s wingwall |  | 0.3 | Adopt |  |
| U/s Guidewall Height |  | 5.85 | 5.85 | 1651.03 |
| D/s Guide wall Height |  | 4.09 | 4.1 | 1649.28 |
| Unit weight Masonry |  | 22.5 |  |  |
| Unit weight of backfill soil |  | 19 |  |  |

Table 2. 7: Stability analysis for the u/s Left hand side masonry retaining wall

| Code | Load |  | Lever Arm, R | Moment |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Vertical | Horizontal |  | Posetive | Negative |  |  |
| W1 | 65.8 |  | 0.3 | 16.5 |  |  |  |
| W2 | 197.4 |  | 1.5 | 296.2 |  |  |  |
| W3 | 166.7 |  | 2.5 | 416.8 |  |  |  |
| Ps |  | 107.3 | 2.0 |  | 209.2 |  |  |
| Pw |  |  | 2.0 |  |  |  |  |
| SUM | 430.0 | 107.3 |  | 729.4 | 209.2 |  |  |
| Factor of safety against, |  |  |  |  |  |  |  |
| Over turning = (M+ve/M-ve) >1.5 |  |  |  | Fo | 3.49 | >1.5 | OK |
| Sliding = (µxFv/Fh),---µ=0.75 >1.5 |  |  |  | Fs | 3.01 | >1.50 | OK |
| Tension: X= (Net Moment/Sum Fv), e=x-B/2, e<B/6 |  |  |  | X | 1.21 |  |  |
|  |  | B/6= | 0.58 | e | 0.54 | <B/6 | OK |

Table 2. 8: General Item

| BILL PART- 1 - General Items of work |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| I. No | Descriptions | Unit | Qty | Rate (Birr) | Amount (Birr) |
| 1.1 | Mobilization and Camp Facilities |  |  |  |  |
| 1.1.1 | Allow for Mobilization | Ls | 1.00 | 77,812.00 | 77,812.00 |
| 1.1.2 | Allow for Demobilization | Ls | 1.00 | 70,031.00 | 70,031.00 |
| 1.1.3 | Construction of Consultants camp and camp facilities size 3m*4m from CGIS for roof & wall and internally painted chip wood wall & ceilings, with hardcore filled cement screed floor. The rooms are well ventilated equipped with windows and doors | No | 2.00 | 54,600.00 | 109,200.00 |
| 1.1.4 | Construction of contractor's camp and camp facilities size 3m*4m from CGIS for roof & wall and internally painted chip wood wall & ceilings, with hardcore filled cement screed floor. The rooms are well ventilated equipped with windows and doors | No | 4.00 | 54,600.00 | 218,400.00 |
| 1.1.5 | 4*5m2, cafe construction from from CGIS for roof & wall and internally painted chip wood wall & ceilings, with hardcore filled cement screed floor. | No | 1.00 | 67,600.00 | 67,600.00 |
| 1.1.6 | 4*6m2, store construction from CIS with doors and windows, Masonry floor cement screened. | No | 1.00 | 58,500.00 | 58,500.00 |
| 1.1.7 | 3*4m2, Toilet and Shower construction from Masonry floor cement screened | No | 1.00 | 39,375.00 | 39,375.00 |
| 1.1.8 | Construction of fence with barbed wired fixed at 30cm interval to erected eucalyptus poles which have at least 2m height and installed at 1.5m interval | m | 200.00 | 280.00 | 56,000.00 |
| 1.1.9 | 2.5*2.5m2, Generator house construction | No | 1.00 | 21,600.00 | 21,600.00 |
| 1.1.10 | Construction of Access roads (Allow for temporary access road to site ) | Km | 3.00 | 41,264.00 | 123,792.00 |
| 1.1.11 | Dewatering of open trenches and excavation, temporary diversion of the river flow and pumps | Ls | 1.00 | 72,000.00 | 72,000.00 |
| 1.1.12 | Project indicator Post | No | 1.00 | 6,500.00 | 6,500.00 |
| 1.1.13 | As built drawing and site plan | Ls | 1.00 | 6,800.00 | 6,800.00 |
| TOTAL CARRIED TO BILL TOTAL SUMMARY |  |  |  |  | 927,610.00 |

Table 2. 9: Head work cost estimation

| BILL PART- 2 - Headwork |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| I. No | Descriptions | Unit | Qty | Rate (Birr) | Amount (Birr) |
| 2.1 | Wier Body |  |  |  |  |
|  | Earthworks |  |  |  |  |
| 2.1.1 | Ordinary Soil Excavation including haul to a distance not exceeding 200m. | m3 | 845.38 | 68.91 | 58,255.14 |
| 2.1.4 | Soft Rock Excavation including haul to a distance not exceeding 200m. | m3 | 475.69 | 468.38 | 222,803.68 |
| 2.1.7 | Hard Rock Excavation including haul to a distance not exceeding 200m. | m3 | 193.90 | 591.43 | 114,678.28 |
| 2.1.10 | Disposing and spreading ( or Dozing) of surplus excavated materials to the distance of not greater than 5km and as per the direction of the Engineer. | m3 | 757.49 | 173.18 | 131,182.12 |
| 2.1.11 | Back fill and Compaction of the excavated material |  | 53.20 | 51.40 | 2,734.48 |
|  | Concrete |  |  |  |  |
| 2.1.12 | 10cm thick lean concrete of class C-10 | m3 | 11.55 | 1,299.35 | 15,007.49 |
| 2.1.13 | Reinforced concrete of class C -25. Formwork will be measured separately | m3 | 126.22 | 2,390.43 | 301,714.99 |
| 2.1.14 | Cyclopean concrete of class C -25. Formwork will be measured separately | m3 | 223.61 | 2,256.59 | 504,591.58 |
|  | Formwork |  |  |  |  |
| 2.1.15 | Supply, install and remove after use plane vertical formwork, timbered or steel or a combination of two, sufficient to contain the wet concrete without leakage. Enough to support temporary loading and pressure from placing compaction or vibration without displacement or appreciable deflection. Formwork shall not be prepared for concrete with old oil, but be fit for fair face internal concrete surfaces. | m2 | 191.80 | 357.94 | 68,652.89 |
|  | Reinforcement |  |  |  |  |
| 2.1.16 | Provide and install mild steel reinforcement according to structural drawings. Reinforcement shall be free from dirt, oil, grease, paint, retarders, etc. and any other substances which may effect the reinforcement and concrete bond. Price should include cutting, bending and placing. |  |  |  |  |
|  | A. Ø12 | kg | 1742.91 | 80.00 | 139,432.80 |
|  | B. Ø14 | kg | 2407.82 | 80.00 | 192,625.60 |
|  | Total Summary Carrierd out |  |  |  | 1,751,679.05 |
| 2.2 | Aprons |  |  |  |  |
|  | Earthworks |  |  |  |  |
| 2.2.1 | Ordinary Soil Excavation including haul to a distance not exceeding 200m. | m3 | 616.20 | 68.91 | 42,462.34 |
| 2.2.2 | Disposing and spreading ( or Dozing) of surplus excavated materials to the distance of not greater than 5km and as per the direction of the Engineer. | m3 | 462.15 | 173.18 | 80,035.14 |
|  | Concrete |  |  |  |  |
| 2.2.3 | 10cm thick lean concrete of class C-10 | m3 | 55.60 | 1299.35 | 72,243.86 |
| 2.2.4 | Reinforced concrete of class C -25. Formwork will be measured separately | m3 | 177.34 | 2390.43 | 423,918.86 |
| 2.2.5 | Cyclopean concrete of class C -25. Formwork will be measured separately | m3 | 434.98 | 2256.59 | 981,571.52 |
|  | Formwork |  |  |  |  |
| 2.2.6 | Supply, install and remove after use formwork, timbered or steel or a combination of two, sufficient to contain the wet concrete without leakage. Enough to support temporary loading and pressure from placing compaction or vibration without displacement or appreciable deflection. Formwork shall not be prepared for concrete with old oil, but be fit for fair face internal concrete surfaces. | m2 | 54.00 | 357.94 | 19,328.76 |
|  | Reinforcement |  |  |  |  |
| 2.2.7 | Provide and install mild steel reinforcement according to structural drawings. Reinforcement shall be free from dirt, oil, grease, paint, retarders, etc. and any other substances which may effect the reinforcement and concrete bond. Price should include cutting, bending and placing. |  |  |  |  |
|  | A. Ø10 | kg | 1839.89 | 80.00 | 147,191.20 |
|  | A. Ø12 | kg | 2649.44 | 80.00 | 211,955.20 |
|  | Total Summary Carrierd out |  |  |  | 1,978,706.87 |
| 2.3 | Divide wall |  |  |  |  |
|  | Earthworks |  |  |  |  |
| 2.3.1 | Ordinary Soil Excavation including haul to a distance not exceeding 200m. | m3 | 20.24 | 68.91 | 1,394.74 |
| 2.3.2 | Disposing and spreading ( or Dozing) of surplus excavated materials to the distance of not greater than 5km and as per the direction of the Engineer. | m3 | 20.24 | 173.18 | 3,505.16 |
|  | Concrete |  |  |  |  |
| 2.3.3 | Reinforced concrete of class C -25. Formwork will be measured separately | m3 | 1.38 | 2390.43 | 3,298.79 |
| 2.3.4 | Cyclopean concrete of class C -25. Formwork will be measured separately | m³ | 14.26 | 2256.59 | 32,178.97 |
|  | Masonry Work |  |  |  |  |
| 2.3.5 | Basaltic or equivalent stone masonry wall erection which will be well bedded in cement sand mortar of 1:3 mix ratio for Retaining wall. The thickness of masonry wall will be as indicated in the drawing. The work should be to the satisfaction of the Engineer. | m3 | 21.15 | 1804.55 | 38,166.23 |
| 2.3.6 | Pointing | m2 | 31.54 | 47.03 | 1,483.33 |
|  | total Summary Carrierd out |  |  |  | 80,027.23 |
| 2.4 | Under sluice |  |  |  |  |
|  | Concrete |  |  |  |  |
| 2.4.1 | Cyclopean concrete of class C -20. Formwork will be measured separately | m3 | 5.52 | 2256.59 | 12,456.38 |
| 2.4.2 | Reinforced concrete of class C -25. Formwork will be measured separately | m3 | 4.60 | 2390.43 | 10,995.98 |
|  | Formwork |  |  |  |  |
| 2.4.3 | Supply, install and remove after use formwork, timbered or steel or a combination of two, sufficient to contain the wet concrete without leakage. Enough to support temporary loading and pressure from placing compaction or vibration without displacement or appreciable deflection. Formwork shall not be prepared for concrete with old oil, but be fit for fair face internal concrete surfaces. | m2 | 10.12 | 357.94 | 3,622.35 |
|  | Reinforcement |  |  |  |  |
| 2.4.4 | Provide and install mild steel reinforcement according to structural drawings. Reinforcement shall be free from dirt, oil, grease, paint, retarders, etc. and any other substances which may effect the reinforcement and concrete bond. Price should include cutting, bending and placing. |  |  |  |  |
|  | A. Ø10 | kg | 17.01 | 80.00 | 1,360.80 |
|  | B. Ø12 | kg | 21.30 | 80.00 | 1,704.00 |
|  | Gate (including installing and Commissioning) |  |  |  |  |
| 2.4.5 | Supply and install Gates with all its accessories including angle iron, sheet metal etc as per the drawing | Pcs | 1.00 | 37,750.00 | 37,750.00 |
|  | total Summary Carrierd out |  |  |  | 67,889.51 |
| 2.5 | Breast Wall and operation slab |  |  |  |  |
|  | concrete |  |  |  |  |
| 2.5.1 | Reinforced concrete of class C -25. Formwork will be measured separately | m³ | 1.27 | 2,390.43 | 3,035.85 |
|  | Formwork |  |  |  |  |
| 2.5.2 | Supply, install and remove after use formwork, timbered or steel or a combination of two, sufficient to contain the wet concrete without leakage. Enough to support temporary loading and pressure from placing compaction or vibration without displacement or appreciable deflection. Formwork shall not be prepared for concrete with old oil, but be fit for fair face internal concrete surfaces. | m2 | 14.81 | 357.94 | 5,301.09 |
|  | Reinforcement |  |  |  |  |
| 2.5.3 | Provide and install mild steel reinforcement according to structural drawings. Reinforcement shall be free from dirt, oil, grease, paint, retarders, etc. and any other substances which may effect the reinforcement and concrete bond. Price should include cutting, bending and placing. |  |  |  |  |
|  | A. Ø10 | kg | 17.92 | 80.00 | 1,433.60 |
|  | B. Ø12 | kg | 44.30 | 80.00 | 3,544.00 |
|  | total Summary Carrierd out |  |  |  | 13,314.54 |
| 2.6 | Upstream Side Retaing wall |  |  |  |  |
|  | Earthworks |  |  |  |  |
| 2.6.1 | Ordinary Soil Excavation including haul to a distance not exceeding 200m. | m³ | 208.30 | 68.91 | 14,353.95 |
| 2.6.2 | Soft Rock Excavation including haul to a distance not exceeding 200m. | m³ | 260.38 | 468.38 | 121,956.78 |
| 2.6.3 | Hard Rock Excavation including haul to a distance not exceeding 200m. | m³ | 52.08 | 591.43 | 30,801.67 |
| 2.6.4 | Back fill and Compaction of the excavated material | m³ | 304.95 | 51.40 | 15,674.43 |
|  | Concrete |  |  |  |  |
| 2.6.5 | 10cm thick lean concrete of class C-10 | m³ | 10.76 | 1,299.35 | 13,984.25 |
| 2.6.6 | Cyclopean concrete of class C -25. Formwork will be measured separately | m³ | 51.28 | 2,256.59 | 115,717.94 |
|  | Masonry Work |  |  |  |  |
| 2.6.7 | Basaltic or equivalent stone masonry wall erection which will be well bedded in cement sand mortar of 1:3 mix ratio for Retaining wall. The thickness of masonry wall will be as indicated in the drawing. The work should be to the satisfaction of the Engineer. | m³ | 254.79 | 1804.55 | 459,781.29 |
| 2.6.8 | Pointing | m2 | 122.16 | 90.00 | 10,994.40 |
|  | total Summary Carrierd out |  |  |  | 783,264.73 |
| 2.7 | Downstream Retaing wall |  |  |  |  |
| 2.7.1 | Ordinary Soil Excavation including haul to a distance not exceeding 200m. | m³ | 167.71 | 68.91 | 11,556.90 |
| 2.7.2 | Soft Rock Excavation including haul to a distance not exceeding 200m. | m³ | 251.56 | 468.38 | 117,825.67 |
| 2.7.3 | Back fill and Compaction of the excavated material. | m³ | 126.76 | 51.40 | 6,515.46 |
|  | Concrete |  |  |  |  |
| 2.7.4 | 10cm thick lean concrete of class C-10 | m³ | 8.16 | 1,299.35 | 10,602.70 |
| 2.7.5 | Cyclopean concrete of class C -25. Formwork will be measured separately. | m³ | 70.35 | 2,256.59 | 158,751.11 |
|  | Masonry Work |  |  |  |  |
| 2.7.6 | Basaltic or equivalent stone masonry wall erection which will be well bedded in cement sand mortar of 1:3 mix ratio for Retaining wall. The thickness of masonry wall will be as indicated in the drawing. The work should be to the satisfaction of the Engineer. | m³ | 203.60 | 1804.55 | 367,406.38 |
| 2.7.7 | Pointing | m2 | 192.87 | 90.00 | 17,358.30 |
|  | total Summary Carrierd out |  |  |  | 690,016.52 |
| 2.8 | Offtake canal Gate (including installing and Commissioning) |  |  |  |  |
| 2.8.1 | Supply and install Gates with all its accessories including angle iron, sheet metal etc as per the drawing | Pcs | 1.00 | 30250.00 | 30,250.00 |
|  | total Summary Carrierd out |  |  |  | 30,250.00 |
| TOTAL CARRIED TO BILL TOTAL SUMMARY |  |  |  |  | 5,395,148.44 |

# IRRIGATION AND DRAINAGE SYSTEMS DESIGN

## Command Area

The Feasibility study carried out by Water, irrigation and energy Bureau document (report) sates that the River is Perennial and it have an excess amount of water to Irrigate 255 ha of land on Goga Kebeles but the base flow during our detail study flow have no capacity to irrigate the figure which is listed above , due to this we are designed the project to irrigate 104 ha of land.

The Bahir LiboRiver divides the command area different ridges to the North East. The alignment of the canal is defined by the need to be able to command the area at the Left bank of Bahir LiboRiver. The main canal therefore follows the possible high-level ground and gentle main canal slope is aligned to irrigate more area around the command area.

Slope is the most important site characteristics as it influences the suitability to irrigation and methods of irrigation and type and kinds of farm operations. In this look upon, the majority of the irrigation command area is steep and, still other slope classes constitute limited proportions.

In the project area, the total area covered by the study is more than 104ha but due to soil suitability and water head, the net area is limited to about 104ha.

### Topography

Topography is an important factor for the planning of any irrigation project as it influences method of irrigation, drainage, erosion, mechanization, and cost of land development, labor requirement and choice of crops.

The topographic feature of the surroundings of the project area is predominantly steep lands. Hence, the topographic feature of the command area is ranging from very steep slope to moderately steep slope but centrally crossed by few gullies. Its elevation range is 1647 to 1598.5 masl. The slope gradient class also ranges from very steep (30%-50%) to moderately steep sloping (15%-30%). Hence, it has identified to be suitable for surface irrigation.

### Climate

Based on the Ethiopian Agro-Ecological Zones (MOA, 2001), Bahirelebo irrigation Project, which is located at the vicinity of Arebaya town, is classified as Kola agro-ecological zone that indicates rainfall distribution problem and poor moisture condition in the area. There is notable variation in terms of onset, distribution and withdrawal rain season from year to year affecting crop production and productivity.

As the project site has no meteorological station, Ebnat (rainfall) and Amedework (for temperature, relative humidity, wind speed and sunshine hour) meteorological stations data were used for the project study as long as these stations are relatively nearer to the proposed command area. In general, the source of meteorological data is the National Meteorology Service Agency (NMSA).

### Soil characteristics

Soil properties (such as physical, chemical, etc. properties) greatly influence the growth and thereby yield of crops, which is grown. For the recommendation of important macro and / or microelements for the required fertilizer, application purposes a soil composite sampling has been made with the soil depth intervals of0 - 30 cm and 30 - 60 cm. The submitted composite soil samples Lab results for physical and chemical properties has presented in Table 3.

| Soil auger composite sample no | Soil depth (cm) | % Sand | % Clay | % Silt | Textural Class | pH (H2O) (1:2.5) | ECe, (dS/m) | % OC | % OM | % TN | Av. P (mg/kg, ppm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| BL-1 | 0-30 | 30 | 60 | 10 | Clay | 7.95 | 0.57 | 0.74 | 1.28 | 0.06 | 6.13 |
| BL-2 | 30-60 | 26 | 48 | 26 | Clay | 8.19 | 1.12 | 0.82 | 1.41 | 0.07 | 4.63 |
| G-1 | 0-30 | 52 | 16 | 32 | Sandy loam | 8.04 | 0.582 | 0.84 | 1.45 | 0.07 | 4.75 |
| G-2 | 30-60 | 58 | 14 | 28 | Sandy loam | 8.23 | 1.58 | 0.78 | 1.34 | 0.07 | 3.63 |
| R-1 | 0-30 | 36 | 32 | 32 | Clay loam | 7.57 | 0.343 | 0.33 | 0.57 | 0.03 | 5.63 |
| R-2 | 30-60 | 36 | 38 | 26 | Clay loam | 7.74 | 0.367 | 0.39 | 0.67 | 0.03 | 3.88 |
| Soil auger composite sample no | Soil depth (cm) | Exchangeable bases (cmol(+)/kg |  |  |  | CEC |  | ESP (%) |  |
|  |  | Ca | Mg | Na | K | Cmol(+)/kg |  | (Na/CEC)*100 |  |
| BL-1 | 0-30 | 28.80 | 3.38 | 1.05 | 2.06 | 58.20 |  | - |  |
| BL-2 | 30-60 | 25.38 | 3.50 | 1.43 | 1.31 | 61.40 |  | - |  |
| G-1 | 0-30 | 18.58 | 2.74 | 0.71 | 1.12 | 43.40 |  | - |  |
| G-2 | 30-60 | 20.00 | 3.76 | 2.38 | 0.58 | 48.20 |  | - |  |
| R-1 | 0-30 | 17.88 | 3.40 | 0.44 | 1.50 | 53.60 |  | - |  |
| R-2 | 30-60 | 20.35 | 3.54 | 0.58 | 1.06 | 55.00 |  | - |  |

Table 3. 1: Summary of physical and chemical analysis of soils of the command area

Note: At field condition, it has estimated that about 20% of soils of the command area is clay loam, 30% sandy loam and 50% clay textured soils (Agronomy Report).

### Existing Irrigation Practices in the Project Area

The pressure of survival and the need for additional food supplies to meet the demands of the increasing population is necessitating a rapid expansion of irrigation scheme throughout the world. Thus, irrigation is becoming a basic part of well-developed agriculture throughout the world.

Accordingly, farmers that use the river flow to the right side with laborious temporary pumps under take traditional irrigation practices. Therefore, the farmers in the project area are aware of irrigation and very much interested in the idea of upgrading the traditional scheme to modern scheme.

## Irrigation method selection

Surface irrigation is the most common method of irrigation in the world. Soils with high infiltration rate are commonly not suitable to surface irrigation, because the distribution of irrigation water is difficult to maintain without short furrows. As a result, loamy soils may be considered as marginally suitable, despite the potential optimum nutrient and moisture holding capacity.

As slopes increase to 12%, so  does the need for soil conservation measures to accompany irrigation; on slopes greater than 12% land forming for surface irrigation is seldom economically viable. The risks of erosion are potentially greater on increasingly sloping land so a sufficient minimum soil depth of 1.0m on slopes between 8% and 12% must be maintained to allow maximum root and soil structural development and to enhance infiltration and reduced run-off.

On slopes up to 6% and so long as soil depth exceeds 1.0m, land can be safely formed to gently sloping benches with gentle and vegetated risers.

Where groundwater is high, the pressure irrigation may be preferable because percolation and run-off and hence the rate of groundwater rise can be minimized more easily than the case with surface irrigation, and any need for drainage can be deferred. Where drainage exists or planned alongside the irrigation development, the choice of irrigation method is not critical so long as the drainage system can handle the extra runoff water generated by surface irrigation.

In Bahir Libo Project, the dominant soil type is Clay and the dominant command slope is less than 6%.Therefore, surface Irrigation method is the recommended method of irrigation in the project considering the above factor and also operation and maintenance factors.

### Methods of irrigation

No single method of water application is equally efficient for all situations, nor are all methods equally efficient for a particular situation. Each method has certain limitations but most methods can be adapted to a wide range of conditions. The method of water lifting is applied in the project area Gravity Surface irrigation method.

- Gravity Surface Irrigation

In gravity surface methods of irrigation, water is applied directly to the soil surface from a channel located at the upper reach of the field by gravity. A flow is introduced at high point or along the high ridge of the field and allowed to cover the field by gravity.

The rate of coverage of land depends almost entirely on the quantitative difference between the inlet discharges and cumulative infiltration rate. Two general requirements of prime importance to obtain high efficiency in surface methods of irrigation are properly constructed water distribution system to provide adequate control of water to the fields to permit uniform distribution.

The common method of surface irrigation is furrow type. The furrow method of irrigation is used to irrigate row crops with furrows developed between the rows in the plan and cultivation process. Water in the furrows contacts only 1/2 to 1/5 of land surface, thus reducing pudding and crusting of soils and renders early cultivation. Water infiltrates into the soil and spreads laterally. It is more suitable method of surface irrigation for crops sensitive to pounded water. Furrows are most commonly made down the slope but when land slope exceed the safe limit soil erosion of soil appears, they are constructed nearly on contour or obliquely. Similarly, when rainwater is to be conserved, furrows act as an effective means to catch and conserve the rainfall. When irrigation water is very scarce, the system of alternate/skip furrow irrigation, results in considerable saving of water.

Surface (Furrow method) is the most common form of irrigation around the world and hence it is recommended for this particular project.

Furrow Irrigation Design Considerations:

Efficient irrigation by the furrow method is obtained by selecting proper combinations of: shape, length, slope of furrows, and suitable size of the irrigation stream and duration of the water application.

- Furrow shape

The furrows are designed to have good permissible velocity with shape of either V or U-cross-sections as shown in the Fig. This design approach is based on the Recommendation of FAO-Paper-volume-II in module-7. The first section is common for sandy texture of deep and narrow furrows while the second is common for clay texture of wide and shallow furrows.

Hence, in Bahir Libo Project the soil is clay soil, the second cross-section type can be applied. The depth, d, varies from 10-30 cm.

Figure 4-1: Furrow shape depending on soil type.

- Furrow Spacing

The spacing between furrows depends on the water movement in the soil, which is texture related, on the crop agronomic requirements as well as on the type of equipment used in the construction of furrows. In practice, a compromise often has to be reached between these factors.

When water is applied to a furrow, it moves vertically under the influence of gravity and laterally by capillarity. Clay soils have more lateral movement of water than sandy soils because of their small pores, which favour capillary action. In this regard, larger spacing can be used in heavier soils than in light soils. In general, a spacing of 0.3m and 0.6m has been proposed, for coarse soils and fine soils respectively. For heavy clay soils up to 1.2m has been recommended. It should also be realized that each crop has its own optimum spacing and the ridges should be spaced according to the agronomic recommendations. In addition, the equipment available on the farm determines the furrow spacing, as this is adjustable only within limits. However, in all instances the furrow spacing adopted should ensure a lateral spread of water between adjacent furrows that will adequately wet the entire root zone of the plants.

- Furrow length

The optimum length of a furrow is usually the longest furrow that can be safely and efficiently irrigated. Proper furrow length depends largely on the hydraulic conductivity of the soil. Furrows shall be shorter on a porous sandy soil than on a tight clay soil. The length of furrow that can be efficiently irrigated may be as short as 80m on sandy soils that take up water rapidly, or as much as 100m or longer on clay soils with low infiltration rates. The length of furrow may often be limited by the size and shape of the field.

- Furrow slope:

The slope or grade of the furrow is important because it controls the speed at which water flows down the furrow. A minimum furrow grade of 0.06 per cent is needed to ensure surface drainage. When the slope of the land is too steep, the furrows should be round the hill rather than straight down the slope; thus, the contour furrow method permits the use of furrows even on fairly steep land. For the project, all furrows aligned across contour and hence the minimum slope is 0.5%.

- Furrow stream:

The size of the furrow stream can be varied even after the furrow has been installed. The maximum size of the irrigation stream that can be used at the start of the irrigation is limited by considerations of erosion in furrows, overtopping of furrows and prevention of runoff at the downstream end. The maximum non-erosive flow rate in furrows is estimated by the following empirical equation:

Where;

qm = maximum non-erosive stream, l/s

s = slope of furrow expressed as a percent

The average depth of water applied during irrigation can be calculated from the following relationship:

Where;

d = average depth of water applied, cm

q = stream size, l/s

t = duration of irrigation (elapsed time), hours

w = furrow spacing, m

L = furrow length, m

The size of the furrow stream varies from 0.5 to 2.5litters per second. To obtain the uniform irrigation, the largest stream of water that will not cause erosion is used in each furrow at the beginning of irrigation. Its purpose is to wet the entire length of each furrow as quickly as possible, thus enabling the soil to absorb water evenly through the entire furrow length. After the water reaches the lower end of a furrow, the stream is reduced or cut back so that it will just keep the furrow wet throughout its length with a minimum waste at the end. This cut back stream flows until the required amount of water has been applied. With level furrows, however, the initial stream is continued from the beginning to the end of irrigation. The water is ponded in the furrow until it is absorbed by the soil.

Furrow sizes and stream sizes can be easily selected in the field for different soil and farmer can easily manipulate crop conditions, as the stream size.

## Naming of canal units

In naming of the canals, a hierarchical system has been adopted. The naming of secondary units reflects the name of the canal that supplies it except from the main canal where the off taking canal des not reflect the name of the supplying canal.

Main Canal

MC = Main Canal that convey water from chain age 0+000 to 2+915 which is all the length is lined canal to increase the efficiency because the base flow is limited at dry season .

Secondary Canals

The canals directly off taking from main canal have been named with two suffixes. The first suffix describes the Main Canal while the second suffix is for secondary canal.

E.g. SC1-1 = Secondary canal 1 off taking from Main Canal.

SC1-1= Secondary canal 1 off taking from Main Canal.

Tertiary Canals

The tertiary canals off- taking from a secondary canal are named with two suffixes. The first suffix describes the Tertiary Canal while the second suffix is for Secondary canal.

TC1-1-1  	=   	Tertiary canal 1 off taking from Secondary Canal 1 of main canal.

### System Layout and Command Units

The main canal is about 3.57km long and serves four secondary units defined by the existing drainage network and the proposed land use. The secondary units contain a lot of turn out units.

#### Tertiary and On-Farm System Layout

The irrigable area is subdivided into tertiary blocks with a target area as permitted by the topography. The direction of the furrows will depend on the ground slope. If the overall slope is less than 0.5% then the furrow direction will be down-slope while, as the ground slope steepens, the furrows will aligned at an angle to the maximum slope (i.e. closer to the contour). Land steeper than 3% slope will be terraced. Terraces on ground slopes of between 3% and 6% may also be provided with furrows. Terraces for furrow irrigation will be provided with a longitudinal slope of about 0.5% downwards from the irrigation source to facilitate movement of water along furrows, and across slope up to about 2% to reduce the earthmoving.

Tertiary canals will normally be aligned along a contour and serve field canals running down-slope. Normally a single tertiary canal will supply a tertiary block. However, a second tertiary canal may be provided where the irrigation block layout results in very long field canals or the second tertiary canal facilitates a more efficient irrigation rotation.

Unlined tertiary contour canal aligned along the uphill boundary of the tertiary unit supplies field canals, which are spaced approximately 100m apart and run down the slope.

### Irrigation Efficiencies and Water Demands

(a) Conveyance Losses

Conveyance losses comprise seepage losses, operational (management) losses, for example due to mismatch between supply and demand and evaporation losses. Of these, the most significant are seepage and operational losses. Seepage losses are generally predictable, are dependent on canal type (i.e. lined/unlined), canal condition, soil type, and canal fill. Operational losses can be low (i.e. less than 5%) with good canal management, or high (i.e. more than 20%) with poor management. Good canal management requires appropriate management institutions and users training/ awareness, with appropriate flow control and measurement infrastructure.

A flat percentage approach may be used for conveyance losses and the following efficiencies are suggested. These percentages assume good canal operation (i.e. operational losses in the order of 5%).

Table 3. 2: Conveyance Efficiencies

| Description | Lined Canals | Unlined Canals |  |  | Adopted for Baherelibo |
| --- | --- | --- | --- | --- | --- |
|  |  | Clays | clay Loams | sandy loam |  |
| Main canal | 95% |  |  |  | 95% |
| Secondary canal | 95% |  |  |  | 95% |
| Tertiary and field canals |  | 43% | 14% | 21% | 78% |
| Overall conveyance efficiency | 90% | 0% | 0% |  | 70% |

For Bahrilib Project both Main canal and the secondary is  lined due to water shortage and the slopes greater than about 0.8%. The adopted main and secondary conveyance efficiencies are 95%. The adopted tertiary distribution system efficiencies are 78%. The overall conveyance efficiency is therefore 70% for furrow irrigation.

(b) Application Losses

Application losses for the various surface application methods are 65% for Clay/ and 60% for Loam Soils. The project application efficiency is 65%.Therefore; the overall efficiency of the project is 45.5%.

(c) Irrigation Duties

Irrigation duty is the volume of water required per hectare of land for the full outcropping of the crops. Moreover, it helps in designing an efficient irrigation canal system. The area, which will be irrigated, can be calculated by knowing the total available water at the source and the overall duty for all crops required to be irrigated in different seasons of the years.

The proposed cropping pattern of Bahir Libo diversion irrigation project has showed a maximum net irrigation water requirement (NIWR) in the month of February with the amount of 4.4 mm/day (1.4/s/ha) for 18 working hours (for overall proposed crops).

Allowing for application and conveyance losses the continuous flow duties at the head of tertiary and secondary canals and at the head of the main canal were determined as tabulated table 5-4. The design (peak) duty at the head of the Main canal is 1.4l/s/ha. However, actual demand will fluctuate, typically increasing from zero during much of the wet season and peaking in March.

Table 3. 3: Crop Water Requirements and Indicative Design Flows

| Item | Units | Efficiency |
| --- | --- | --- |
| Total net irrigated area | ha | 104 |
| Net peak crop water requirement | l/s/ha | 1.47 |
| Application efficiency | % | 65% |
| CWR at field | l/s/ha | 1.4 |
| Tertiary & Field system conveyance efficiency | % | 78% |
| CWR at the head of tertiary | l/s/ha | 1.47 |
| Secondary canal conveyance efficiency | % | 95% |
| CWR at the head of secondary | l/s/ha | 1.47 |
| Main canal conveyance efficiency | % | 95% |
| CWR at the head of main canal | l/s/ha | 1.47 |
| Design discharge at the head of main canal | m3/s | 0.147 |

# IRRIGATION CANALS

## General

Irrigation water after being diverted from the Bahir LiboRiver is carried to the farm through main canal, secondary canals, tertiary canals and field canals.

For shallow slopes, lined regime-slope and regime-width canals are invariably the cheapest option unless seepage losses would otherwise be high, for example due to light textured soils. Seepage is a major problem in much of the command area where canals are excavated into rock.

## Canal lining option

Design of unlined canals should be based on criteria that to be non-silting when conveying sediment laden water, and to be non-scouring when carrying silt free water. Unlined trapezoidal shaped canals are the most common and economic solution in most irrigation schemes in all situation of terrain. The canal sections and alignment should be chosen ideal for construction and economical section.

For canals aligned down a steep slope, the lining allows greater flow velocities and both small prism section size and reduced quantities of drop structures. The geological report shows the main canal route have three major geotechnical properties.

- From about out 0+00 to about 1+480m: - the ground is characterized by relatively Thin layer of Silty clayey soil (which ranges from 0.4 to 0.6m), as it can be seen from test pit taken (B.libo M.cT.p_1). The soil has light brown color. It is dry in all depth. The soil possesses medium plasticity, with stiff consistency. Under this unit highly friable weathered ash had encounter (see in test pit log ANXXTURE 1). It had whitish color with pervious nature. This portion of the main canal route is a farmland having moderate slope. Since the top soil is removed during excavation and the rout lies on the underlying pervious rock, the canal should be designed as lined.

On the above chain age (From 0+00 to 1 + 480), there are five gullies (G_1, G_2 , G-3,G-4 and G-5) found on 0+564m a 0+807m , 1+447m,2+351m and 2+502m from headwork site respectively. They have similar geological nature at the streambed, with recently deposited alluvial sediment dominantly fine sand and silt. The width of G_1 is wide and very short bank relatively to G2. Banks of both gullies are light brown color, weak silty clayey soil. The thickness of this soil in G-2 reach up to maximum of 1m as easily sees from natural cut. Under that weathered and fragmented volcanic rock are expected.

- From 1+480 to1+592m, the route passes on clearly exposed dark gray color highly jointed rock. These rocks have very narrowly spaced linearly aligned joints with very tight opening. Hence these jointed rock have pervious nature proposed main canal route along the chain age should be lined probably in masonry.

- From 1+592m up to End, the ground is characterized by relatively Thin layer of  dark brown color Clay  soil (average thickness of  0.9m), as it can be seen from test pit taken(B.libo M.cT.p_2) . The soil has dark brown color. It is dry in all depth. The soil possesses medium to high plasticity. Under this unit highly weathered and jointed rock had encounter (see in test pit log ANXXTURE 1). It had dark gray color with pervious nature. This portion of the main canal route have moderate slope and since the top soil is removed during excavation and the rout lies on the underlying pervious rock, the canal should be designed as lined.

Secondary Canals

Unlined canals with frequent drops is not a better option, largely due to the cost of transitions to ensure non-turbulent flow leaving the drop structures and eroding the unlined canal prism (narrowing structures to reduce the basic structure cost will result in larger, more expensive, transitions). The option of using lined canals for downhill conveyance is provided to be economical. In Bahir LiboProject four secondary canals aligned across in steep slope due to steep slope topography all secondary canals are lined. Secondary canal one crosses One Main River and one gully at 0+ 260 and 0+ 951m respectively.

Tertiary Canals

- The tertiary canals are aligned along contour, so the unlined canal is proposed in all tertiary canals. There are nine tertiary canals in baharlibo irrigation project.

## Canal cross Section

Shape and size, including the hydraulic section, service and feeder roads, banks widths, side drains, free board, side slopes, bank slopes etc. are defined by cross section of a channel.

Despite that the most economical section of unlined canals is semi-circle, in practice trapezoidal canals are most often used for easiness of construction. The ideal hydraulic shape for a trapezoidal canal leads to deep and narrow section such sections are only possible for small capacities (up to 0.5m3/s). Canals with high discharges are generally wide and shallow with large bed width/depth rations. It is also necessary to keep them within maximum allowable velocity limit. Wider canals also facilitate better distribution due to low variations in water level with changing discharge.

The maximum economy can be achieved in canal construction, if the earthwork in excavation equals the earthwork in filling. Such a thing is possible when a canal is constructed partly in filling and partly in cutting, which mostly happens in practical life.

For a given cross section of a channel, there can be only one depth for which such a balance between cutting and filling will occur. Such a depth is termed as balancing depth. This depth can be worked out easily by equating the areas of cutting and filling. If such balance between cutting and filling can occur, then the need for demolish banks or borrow pits is eliminated, and earthwork has to be paid only once in single item.

Some studies show that the following guidelines are recommended for the design of main canal sections.

- For cross slope <10% a trapezoidal section is likely to be more cost effective.

- Where a canal is in cut with shallow rock, a rectangular section is likely to be more cost effective.

- Where a canal is in fill, a trapezoidal section is likely to be more cost effective.

- Where there is no rock encountered, a trapezoidal section is likely to be more cost effective.

- On very steep cross slopes, a rectangular section is likely to be more cost effective.

- Where the terrain is very irregular then it may be more cost effective to place the canal service road on a separate, but nearby, alignment.

For Bahir LiboProject rectangular lined canal is recommended with the B/D ratio range from 1to 3.

## Longitudinal slope

The longitudinal slope is another very important issue in the hydraulic design of irrigation channel.  The slope of any irrigation channel will depend on the topographic features and available natural slope.  However, if the natural slope of the area is very steep, only permissible slope, which generates permissible velocity, will be adopted and the balance difference in slope will be adjusted by providing drops and chutes.

The recommended Bahir Libomain canal longitudinal slope is range from 1:1,000 to 1:16.

## Hydraulic Design

There are different formulas for design of canals. Determination of the design velocity is one of the important factors for the design of canals and the cross sections of canal structures. The design velocity is determined between the minimum and maximum allowable velocity taking into consideration for Canal and Canal related structures. The design velocity of canals is estimated by checking its non-erosive and non-silting criteria for different canal lining materials proposed for this project. The design velocity is determined by Manning’s equation.

v=1n*R(23)*s

Where, V: Mean velocity (m/s)

n : Coefficient of roughness

R: Hydraulic radius (m) (R=Cross-section area (A)/Wetted perimeter (P))

P: Wetted perimeter (m)

S: Canal bed slope

### Minimum allowable velocity

It is not easy to determine a proper value of the minimum allowable velocity because there are under finable factors, which place restrictions on the minimum allowable velocity. It is obtained so as not to produce sand deposits and hinder the flow capacity of the canal by the presence of waterweeds.

From previous experiences and literatures, the minimum flow velocity for the canals has been kept to be 0.3m/s for very little discharges especially in tertiary and filed canals.

### Maximum allowable velocity

Since the maximum allowable velocity varies remarkably with materials used for canals, limited velocities found to be applicable to different types of materials used for canals are given in Table below.

According to the Federal Democratic Republic of Ethiopian Ministry of Water Resources Irrigation System Design Guide Line (2002), the recommended maximum velocity for each material type has been listed in the following table. Therefore, the design velocity determination of Bahir Libo irrigation system canals has followed these recommended permissible maximum velocities as design criteria for design velocity of canals.

Table 4. 1: Permissible maximum velocities

| SI.No. | Type of materials | Max. Permissible Velocity (m/s) |
| --- | --- | --- |
| 1 | Sandy Soil | 0.45 |
| 2 | Sandy Loam | 0.6 |
| 3 | Loam | 0.7 |
| 4 | Clayey Loam | 0.9 |
| 5 | Clay | 1.0 |
| 6 | Sandy Clay | 1.2 |
| 7 | Soft Rock | 2 |
| 8 | Semi-hard rock | 2.5-3 |
| 9 | Hard rock | 4 |
| 10 | Concrete face | 6 |
| 11 | Stone masonry face with cement pointing | 4 |
| 12 | Stone or brick masonry face with cement plaster | 4 |
| 13 | Brick masonry with cement pointing | 2.5 |
| 14 | Soil silt | 1 -2 |

Source: Federal Democratic Republic of Ethiopian Ministry of Water Resources Irrigation System Design Guide Line (2002)

## Roughness Coefficients

Selection of the coefficient of roughness is very important in use of the Manning’s formula, and therefore careful consideration is required in determining the coefficient on various influencing factors such as surface roughness, vegetation, bends, cross-section area and shape, velocity, hydraulic radius, sediment, scour, suspended materials, canal conditions of operation and maintenance stage, etc. In canals constructed with the same materials, the coefficient of roughness tends to become larger in the case of extremely slow velocity or small hydraulic radius. Standard values in the table are generally applied to design. It must also be considered that the smoothness of the internal section of the canal decreases gradually due to friction and scour, etc. are produced by the flow in the canal, and the growth of aquatic plants.

The USBR Design Standard Nr 3 [USBR, 1967] recommends a value of n of 0.016 to 0.018 for a canal with a stone masnory lining with a hydraulic radius, R, of less than 4 ft. (1.22 m) for a clean and straight canal.

Channel alignment (sinuosity) has an influence on the roughness coefficient.  The contour section of the Main Canal has a number of curves, most of which are of comparatively small radius. This will increase channel resistance to some extent.  The degree of meandering of the Main Canal is considered moderate, which indicates an increase of 15% to the value of n [French, 1987].

Considering the effects of the above scenarios to the specific Bahir Libo irrigation canals, the roughness coefficients have been selected from standard tables recommended by Federal Democratic Republic of Ethiopian Ministry of Water Resources Irrigation System Design Guide Line (2002)

The derived Manning roughness coefficients to be used in the design of the irrigation conveyance system are given.

Table 4. 2: Manning’s Roughness Coefficient for different canal types (n)

| Materials and conditions of canal | Value of `n` |
| --- | --- |
| Covered by Boulder and gravel | 0.03 to 0.05 |
| Dressed stone masonry | 0.019 to 0.021 |
| Stone masonry with plaster | 0.016 to 0.018 |
| Concrete surface | 0.012 to 0.016 |
| Earthen canals, curved and non-uniform No vegetation coverage | 0.023 to 0.03 |
| Some weeds | 0.025 to 0.033 |
| Earthen canals, uniform and straight No weeds (after the canal has been exposed to weather ) | 0.018 to 0.025 |
| Few weeds with short grasses | 0.022 to 0.035 |

Source: Federal Democratic Republic of Ethiopian Ministry of Water Resources Irrigation System Design Guide Line (2002)

For Bahir LiboProject the recommended Main Canal and other canals roughness coefficient is 0.018 for lined part& 0.035 for unlined part.

## Freeboard of Canals

The term freeboard refers to the vertical distance between either the top of the canal and design flow at normal depth. The purpose of freeboard is to prevent the overtopping of either the lining or the top of the canal fluctuations in the water surface caused by;

- Wind - driven waves,

- Hydraulic jumps,

- Super elevation of the water surface as the flow goes round curves at high velocities,

- The interception of storm runoff by the channel,

- The occurrence of greater than design depths of flow caused by canal sedimentation or an increased coefficient of friction, or

- Temporary miss-operation of the canal system,

According to the Ethiopian Ministry of Water Resources the recommended canal free boards are listed in the following table.

Table 4. 3: Recommended Canal Freeboards

| Scale of the project | Canal discharge, Q(m3/s) | Free board(m) |
| --- | --- | --- |
| Small | <0.1 | 0.2 |
|  | 0.1 to 0.5 | 0.3 |
|  | 0.5 to 1.0 | 0.4 |
| Medium | 1.0 to 3.0 | 0.5 |
|  | 3.0 to 5.0 | 0.6 |
|  | 5.0 to 10.0 | 0.7 |
| Large | 10.0 to 30.0 | 0.8 |
|  | >30.0 | 1.0 |

Source: Federal Democratic Republic of Ethiopian Ministry of Water Resources Irrigation System Design Guide Line (2002)

By using the above Recommendation, the design of each canal free board has been fixed for each corresponding canal discharges. For unlined Tertiary canals of discharges out of the recommended ranges, the freeboard has been set to be 300 mm from practical point of view.

Considering the above factors the Bahir Libo Main Canal hydraulic Parameter is summarized in the following table.

| Canal name | Chain age | Area(ha) | Duty(l/s/ha) | Qr (m3/s) | m (H:V) | B (m) | D (m) | Fb (m) | Area | Perimeter | R=A/P | S | N | V=1/N*R^(2/3)*S^0.5 | Q d (m3/s) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| MC-1 | 00-414 | 104 | 0.0014 | 0.147 | 1.00 | 0.70 | 0.50 | 0.20 | 0.35 | 1.7 | 0.21 | 0.0014 | 0.018 | 0.7 | 0.26 |
| MC-1 | 414-546 | 104 | 0.0014 | 0.147 | 1.00 | 0.70 | 0.50 | 0.20 | 0.35 | 1.7 | 0.21 | 0.0014 | 0.018 | 0.7 | 0.26 |
| MC-1 | 546-1240 | 104 | 0.0014 | 0.147 | 1.00 | 0.70 | 0.50 | 0.20 | 0.35 | 1.7 | 0.21 | 0.0008 | 0.018 | 0.6 | 0.20 |
| MC-1 | 1240-1767 | 104 | 0.0014 | 0.147 | 1.00 | 0.70 | 0.50 | 0.20 | 0.35 | 1.7 | 0.21 | 0.0250 | 0.018 | 3.1 | 1.07 |
| MC-1 | 1767-1880 | 104 | 0.0014 | 0.147 | 1.00 | 0.70 | 0.50 | 0.20 | 0.35 | 1.7 | 0.21 | 0.0250 | 0.018 | 3.1 | 1.07 |
| MC-1 | 1880-1993 | 83 | 0.0014 | 0.117 | 1.00 | 0.50 | 0.45 | 0.20 | 0.225 | 1.4 | 0.16 | 0.0111 | 0.018 | 1.7 | 0.39 |
| MC-1 | 1993-2144 | 83 | 0.0014 | 0.117 | 1.00 | 0.50 | 0.45 | 0.20 | 0.225 | 1.4 | 0.16 | 0.0056 | 0.018 | 1.2 | 0.28 |
| MC-1 | 2144-2295 | 83 | 0.0014 | 0.117 | 1.00 | 0.50 | 0.45 | 0.20 | 0.225 | 1.4 | 0.16 | 0.0083 | 0.018 | 1.5 | 0.34 |
| MC-1 | 2295-2540 | 83 | 0.0014 | 0.117 | 1.00 | 0.50 | 0.45 | 0.20 | 0.225 | 1.4 | 0.16 | 0.0011 | 0.018 | 0.5 | 0.12 |
| MC-1 | 2540-2915 | 83 | 0.0014 | 0.117 | 1.00 | 0.50 | 0.45 | 0.20 | 0.225 | 1.4 | 0.16 | 0.0010 | 0.018 | 0.5 | 0.12 |

Table 4. 4: Summary of main canal Hydraulics

The secondary canal hydraulic parameter is presented in below.All secondary canals are lined

| Canal name | Chain age | Area(ha) | Duty(l/s/ha) | Qr (m3/s) | m (H:V) | B (m) | D (m) | Fb (m) | Area | Perimeter | R=A/P | S | N | V=1/N*R^(2/3)*S^0.5 | Q d (m3/s) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SC-1 | 00-325 | 21 | 0.00141 | 0.03 | 1 | 0.4 | 0.25 | 0.2 | 0.1 | 0.9 | 0.111 | 0.02222 | 0.018 | 1.9 | 0.19 |
|  | 325-454 | 21 | 0.00141 | 0.03 | 1 | 0.4 | 0.25 | 0.2 | 0.1 | 0.9 | 0.111 | 0.00083 | 0.018 | 0.4 | 0.04 |
|  | 454-972 | 21 | 0.00141 | 0.03 | 1 | 0.4 | 0.25 | 0.2 | 0.1 | 0.9 | 0.111 | 0.00333 | 0.018 | 0.7 | 0.07 |
|  | 972-1382 | 21 | 0.00141 | 0.03 | 1 | 0.4 | 0.25 | 0.2 | 0.1 | 0.9 | 0.111 | 0.00083 | 0.018 | 0.4 | 0.04 |
|  | 1382-1534 | 21 | 0.00141 | 0.03 | 1 | 0.4 | 0.25 | 0.2 | 0.1 | 0.9 | 0.111 | 0.00667 | 0.018 | 1.0 | 0.10 |
| SC-2 | 00-129 | 30 | 0.00141 | 0.04 | 1 | 0.7 | 0.5 | 0.2 | 0.35 | 1.7 | 0.206 | 0.03333 | 0.018 | 3.5 | 1.24 |
|  | 129-517 | 30 | 0.00141 | 0.04 | 1 | 0.7 | 0.5 | 0.2 | 0.35 | 1.7 | 0.206 | 0.02 | 0.018 | 2.7 | 0.96 |
| SC-3 | 00-117 | 25 | 0.00141 | 0.04 | 1 | 0.7 | 0.5 | 0.2 | 0.35 | 1.7 | 0.206 | 0.03333 | 0.018 | 3.5 | 1.24 |
|  | 117-500 | 25 | 0.00141 | 0.04 | 1 | 0.7 | 0.5 | 0.2 | 0.35 | 1.7 | 0.206 | 0.02 | 0.018 | 2.7 | 0.96 |
| SC-4 | 00-90 | 14 | 0.00141 | 0.01974 | 1 | 0.4 | 0.4 | 0.2 | 0.16 | 1.2 | 0.13333 | 0.025 | 0.018 | 2.29 | 0.37 |
|  | 90-320 | 14 | 0.00141 | 0.01974 | 1 | 0.4 | 0.4 | 0.2 | 0.16 | 1.2 | 0.13333 | 0.00667 | 0.018 | 1.18 | 0.19 |

| Channel | Area(ha) | chain age(m) | Q required(m^3/s) | B(m) | D(m) | FB(m) | A(m^2) | P(m) | R(m) | Z | S | n | V(m/s) | Q Design(m^3/s) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TC-1 | 10 | 0+00-0+275 | 0.014 | 0.4 | 0.3 | 0.2 | 0.255 | 1.4817 | 0.1721 | 1.5 | 0 | 0 | 0.71454 | 0.18 |
| TC-2 | 9 | 0+00-0+366 | 0.0126 | 0.4 | 0.3 | 0.2 | 0.255 | 1.4817 | 0.1721 | 1.5 | 0 | 0 | 0.71454 | 0.18 |
| TC-3 | 5 | 0+00-0+308 | 0.007 | 0.4 | 0.3 | 0.2 | 0.255 | 1.4817 | 0.1721 | 1.5 | 0 | 0 | 0.61881 | 0.16 |
| TC-4 | 6 | 0+00-0+237 | 0.0084 | 0.4 | 0.3 | 0.2 | 0.255 | 1.4817 | 0.1721 | 1.5 | 0 | 0 | 0.61881 | 0.16 |
| TC-5 | 9 | 0+00-0+383 | 0.0126 | 0.4 | 0.3 | 0.2 | 0.255 | 1.4817 | 0.1721 | 1.5 | 0 | 0 | 0.71454 | 0.18 |
| TC-6 | 5 | 0+00-0+246 | 0.007 | 0.4 | 0.2 | 0.2 | 0.14 | 1.1211 | 0.1249 | 1.5 | 0 | 0 | 0.57697 | 0.08 |
| TC-7 | 5 | 0+50-0+345 | 0.007 | 0.4 | 0.3 | 0.2 | 0.255 | 1.4817 | 0.1721 | 1.5 | 0 | 0 | 0.55348 | 0.14 |
| TC-8 | 15 | 0-0+309 | 0.02115 | 0.4 | 0.3 | 0.2 | 0.12 | 1 | 0.12 | 0 | 0 | 0 | 1.91145 | 0.23 |
| TC-9 | 12 | 0-0+136 | 0.0168 | 0.4 | 0.3 | 1.2 | 0.255 | 1.4817 | 0.1721 | 2.5 | 0 | 0 | 0.55348 | 0.14 |
| TC-9 | 12 | 0+136-0+340 | 0.0168 | 0.4 | 0.3 | 2.2 | 0.255 | 1.4817 | 0.1721 | 3.5 | 0 | 0 | 0.55348 | 0.14 |

Table 4. 5: Summary of Tertiary c canal Hydraulics

All tertiary canals are earthen except tertiary canal eight.Tc-8 is lined.

# CANAL STRUCTURES

## General

Modern irrigation aims at high efficiency of water conveyance, distribution and application. Sustainability of the system should be well conceived as irrigation has its own ill effects. This situation calls for system to be studied and designed properly considering the local conditions. Commonly an irrigation filed layout contains irrigation and drainage canals networks, hydraulic structures and roads network.

The provided structures should have to be hydraulically and structurally safe stable and of appropriate design criteria have to be followed with special attention to their locations, function, topography, foundation, construction material and construction methods being adopted.

## Types and Hydraulic Design of the structures

The canal and related structure should perform their functions efficiently and competently with minimum maintenance, ease of operation, and minimum water loss. The types of structures proposed in the Bahir LiboDiversion Irrigation Project are identified and characterized in the following subsections.

### Crossing Structures

In addition to the canal network, it is usually necessary to use canal structures to convey water along the canal route. Some these structures include:-

- Drainage crossing structures like box culvert to convey drainage water under canal and Flumes to conduct canal water across deep rivers/gullies.

- Road crossings to carry canal water under roadways,

- Drainage Crossing Structures

Drainage crossing structures are required wherever the canal line crosses natural drainage channels. As far as possible, the canal should be carried above or below the channel, and level crossing should be avoided since they cause silt to enter the canals and, in floods, debris and excess water may damage the canal.

To select the most appropriate structure, the factors to be considered are:

- Type and size of drainage channel in relation to canal size:

  - Small local drainage way

  - Seasonal stream

  - Perennial stream

- Usefulness as a supplementary sources

- Sediment and/or debris loads during flood

- Relative levels of canal water level and bed and stream bed

- Foundation conditions in and adjacent to the channel

- The strategic importance of the structure in terms of the scheme performance

Design Procedure for cross drainage structures

- Establish levels and dimension of canal

- Establish levels and sections of drainage ways;

- Estimate the drain flow for the appropriate return period and estimate the corresponding flow depth at the crossing site;

- Compare levels and sizes of canal and drain. In hill areas, it is generally possible to route the canal to achieve level conditions appropriate to almost any type of crossing (by moving the alignment into or out of the slope);

- Select a structure, which is suited to the levels and dimensions of the two channels.

In Bahir Libo Diversion Irrigation Project the proposed Crossing Structures are; Two Flume (Located at 0+807,1+447m on  the main canal and three box culverts at chain age 564m,2351m and 2521m), two flume on secondary Canal (Located at SC1-1 0+260, 0+951m).

- Flume Structures

A flume is used where canals cross over deeply incised streams or rivers where a short crossing will be cheaper than long alternative route with a super passage. The structure usually has masonry abutments while the flume may be of various materials. For larger spans, central piers will be provided to be economical and structurally safe.

The hydraulic gradient of the flume should be provided between the canal and the river, dependent on the ground conditions. There are different options to select types of Flumes. Reinforced concrete flume with central pier is recommended for secondary canals flumes.

Reinforced concrete flume is recommended for the main canal crossing structures in Bahir Libo Diversion Irrigation Project.

Figure 6-1: Flume -1 plan and section view at SC1-1at chainage 0+369

Table 5. 1: Flume 1 hydraulic Parameters at chain age 0+807.

| Section | Location | Water depth (m) | Change in water level |  | Elevation of |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Canal bed | Water surface |
| A | u/s canal | 0.50 | rise | - | 1644.800 | 1645.300 |
| B | Flume | 0.50 | drop | -0.026 | 1644.826 | 1645.326 |
| C | Flume | 0.50 | drop | 0.033 | 1644.793 | 1645.293 |
| D | d/s canal | 0.50 | rise | -0.040 | 1644.833 | 1645.333 |

| Section | Location | Water depth (m) | Change in water level |  | Elevation of |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Canal bed | Water surface |
| A | u/s canal | 0.50 | rise | - | 1641.960 | 1642.460 |
| B | Flume | 0.50 | drop | -0.026 | 1641.986 | 1642.486 |
| C | Flume | 0.50 | drop | 0.050 | 1641.937 | 1642.437 |
| D | d/s canal | 0.50 | rise | -0.040 | 1641.976 | 1642.476 |

Table 5. 2: Flume 2 hydraulic Parameters. At chain age 1+428m

- Drainage Culverts

The drainage culverts are provided when the drain discharge is quite small and the H.F.L. of the drain is very much below the bed level of the canal. The drain discharge is normally carried through the Box culvert and slab culverts. The canal above is generally carried with full section, without any fluming at box culverts, as the cost of the transitions will be more than the saving in the cost of pipes.

Box Culverts are located in three different locations in the main canal in different sizes (0+546, 0+2351, and 2502).

- Road crossings

Road crossings are recommended at existing roads pathway crossing to maintain the communication. In addition, such crossings are also generally provided as required at existing cattle tracks and facilitate access into and out of the farm. They are also recommended in crossing of irrigation and drainage canals.  Pre casts Concrete are commonly used in the construction of roads. In addition, footbridges will be required at intermediate locations, maximum walkway distance of 0.5km in local community living areas.

Table 5. 3: Road crossing Dimensions at the main canal.

| S.No. | Location | Length(L) (m) | Bed width of the canal(B)(m) | D(m) | Thickness of masonry(m) | Crossing Type | Remark |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 0+0288 | 2 | 0.55 | 0.8 | 0.3 | Foot bridge | For Pedestrian |
| 2 | 0+584 | 2.5 | 0.55 | 0.8 | 0.3 | Foot bridge | For Pedestrian |
| 3 | 0+799 | 2.5 | 0.45 | 0.7 | 0.3 | Foot bridge | For Pedestrian |
| 4 | 0+959 | 2 | 0.45 | 0.7 | 0.3 | Foot bridge | For Pedestrian |
| 5 | 1+323 | 3 | 0.4 | 0.6 | 0.3 | Foot bridge | For Pedestrian |
| 6 | 1+573 | 2 | 0.4 | 0.6 | 0.3 | Foot bridge | For Pedestrian |
| 7 | 1+871 | 2.5 | 0.35 | 0.6 | 0.3 | Foot bridge | For Pedestrian |
| 8 | 2+108 | 2 | 0.3 | 0.5 | 0.3 | Foot bridge | For Pedestrian |
| 9 | 2+527 | 2 | 0.3 | 0.45 | 0.3 | Foot bridge | For Pedestrian |
| 10 | 2+932 | 2 | 0.3 | 0.53 | 0.3 | Foot bridge | For Pedestrian |
| 11 | 3+247 | 2.5 | 0.3 | 0.4 | 0.3 | Foot bridge | For Pedestrian |
| 12 | 3+423 | 2.5 | 0.3 | 0.4 | 0.3 | Foot bridge | For Pedestrian |

## Drainage Canals

In this regard escape canals are very important to flash sediments in to natural drainage and when at high flood season un necessary water moves in the canal, so during this condition escape canals used to divert the flow in to natural drainage

## Drainage System

The construction of subsurface, surface or the combination of both systems depends on the above-mentioned conditions.

In the case of this small-scale irrigation project, it has been observed that the topography and relief of the land provides adequate natural drainage systems and there is no fear of rise of ground water. Hence irrigated land drainage system for this project is not considered.

## Design of Division box

At different points of the main and secondary canals, division boxes are provided which divert the flow to the secondary canal and tertiary canals. Gate should be provided at the outlet of the boxes.

For detail, refer the drawing.

Figure 6-3: Typical Division Box plan.

Using broad crested formula,

Q= CL (h)3/2

Where; Q= discharge over rectangular weir/intake (opening), m3/s

C = discharge coefficient, c= 1.7

L= effective length of crest form in m

h= over flow depth, m

Assuming equal discharge coefficient & sill height for two or three dividing canals, the proportion becomes.

Q1/ Q2= Q2/ Q3 =L1/ L2 = L2/ L3

Where Q1= is flow in canal 1

Q2 = is flow in canal 2

Q3 = is flow in canal 3

L1= is effective crest length of weir/intake sill across opening to canal 1

L2= is effective crest length of weir/intake sill across opening to canal 2

L3 = is effective crest length of weir/intake sill across opening to canal 3

Q1= CL1 (h)3/2,

L1 = Q1/Ch3/2

L2 = L1*Q2/ Q1

L3 = L1*Q3/ Q1

The depth of (height of) the division box,

D = d + fb

The width of the division box,

B = b + 2*m*D

Where b= base width of the incoming canal

D = total canal depth of the incoming canal

Table 5. 4: Hydraulic parameters of Division Boxes.

| Hydraulic and related parameters for Division box |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Canal name | Chainage | Q0 (L/s) | Q1 (L/s) | Q2 (L/s) | Q3 (L/s) | B, m | d(m) | D (d+fb) | L, m | W0, m | W1, m | W2, m | W3, m | H0 | H1 | H2 | H3 | Hs1 | Hs2 | Hs3 | L0, m | L1, m | L2, m | L3, m |
| DV1-1 | 0+940 | 159 | 139 | 43 | 0 | 0.7 | 0.5 | 0.7 | 1.85 | 0.40 | 0.40 | 0.30 | 0.00 | 0.40 | 0.35 | 0.19 | 0.00 | 0.05 | 0.21 | 0.00 | 1.20 | 1.20 | 1.10 | 0.00 |
| DV2-1 | 1+180 | 104 | 73 | 70 | 0 | 0.7 | 0.5 | 0.7 | 1.80 | 0.30 | 0.30 | 0.30 | 0.00 | 0.40 | 0.27 | 0.27 | 0.00 | 0.13 | 0.13 | 0.00 | 1.10 | 1.10 | 1.10 | 0.00 |
| DV3-1 | 2+020 | 82 | 47 | 40 | 0 | 0.7 | 0.5 | 0.7 | 1.80 | 0.30 | 0.30 | 0.30 | 0.00 | 0.40 | 0.21 | 0.18 | 0.00 | 0.19 | 0.22 | 0.00 | 1.10 | 1.10 | 1.10 | 0.00 |
| DV4-1 | 2+920 | 47 | 27 | 60 | 0 | 0.7 | 0.5 | 0.7 | 1.20 | 0.50 | 0.30 | 0.70 | 0.00 | 0.15 | 0.14 | 0.14 | 0.00 | 0.01 | 0.01 | 0.00 | 1.30 | 1.10 | 1.50 | 0.00 |
| DV1-1-1 | 0+60 | 65 | 0 | 0 | 68.82 | 0.4 | 0.3 | 0.5 | 1.30 | 0.50 | 0.00 | 0.00 | 0.50 | 0.20 | 0.00 | 0.00 | 0.19 | 0.00 | 0.00 | 0.01 | 1.30 | 0.00 | 0.00 | 1.30 |
| DV1-1-2 | 0+180 | 100 | 91 | 5 | 3.7 | 0.4 | 0.3 | 0.5 | 1.20 | 1.10 | 1.00 | 0.30 | 0.30 | 0.15 | 0.14 | 0.05 | 0.04 | 0.01 | 0.10 | 0.11 | 1.90 | 1.80 | 1.10 | 1.10 |
| DV2-1-3 | 1+400 | 80 | 73 | 3 | 4.4 | 0.4 | 0.3 | 0.5 | 1.20 | 0.90 | 0.80 | 0.30 | 0.30 | 0.15 | 0.14 | 0.03 | 0.04 | 0.01 | 0.12 | 0.11 | 1.70 | 1.60 | 1.10 | 1.10 |
| DV3-2-1 | 0+100 | 60 | 55.4 | 1 | 3.5 | 0.4 | 0.3 | 0.5 | 1.10 | 1.20 | 1.10 | 0.30 | 0.30 | 0.10 | 0.10 | 0.02 | 0.04 | 0.00 | 0.08 | 0.06 | 2.00 | 1.90 | 1.10 | 1.10 |
| DV4-2-1 | 0+400 | 50 | 43.7 | 1 | 5.3 | 0.4 | 0.3 | 0.5 | 1.10 | 1.00 | 0.90 | 0.30 | 0.30 | 0.10 | 0.09 | 0.01 | 0.05 | 0.01 | 0.09 | 0.05 | 1.80 | 1.70 | 1.10 | 1.10 |
| DV1-3-1 | 0+20 | 100 | 120.0 | 42 | 1.5 | 0.4 | 0.3 | 0.5 | 1.20 | 1.10 | 1.30 | 0.50 | 0.30 | 0.15 | 0.14 | 0.13 | 0.02 | 0.01 | 0.02 | 0.13 | 1.90 | 2.10 | 1.30 | 1.10 |
| DV2-3-1 | 0+260 | 120 | 120.0 | 9 | 7.5 | 0.4 | 0.3 | 0.5 | 1.30 | 0.80 | 0.80 | 0.30 | 0.30 | 0.20 | 0.20 | 0.07 | 0.06 | 0.00 | 0.13 | 0.14 | 1.60 | 1.60 | 1.10 | 1.10 |
| DV1-4-1 | 0+60 | 60 | 50.0 | 3 | 2 | 0.4 | 0.3 | 0.5 | 1.10 | 1.20 | 1.00 | 0.30 | 0.30 | 0.10 | 0.10 | 0.03 | 0.02 | 0.00 | 0.07 | 0.08 | 2.00 | 1.80 | 1.10 | 1.10 |
| DV2-4-1 | 0+180 | 50 | 30.0 | 6 | 4 | 0.4 | 0.3 | 0.5 | 1.10 | 1.00 | 0.60 | 0.30 | 0.30 | 0.10 | 0.10 | 0.05 | 0.04 | 0.00 | 0.05 | 0.06 | 1.80 | 1.40 | 1.10 | 1.10 |
| DV3-4-1 | 0+320 | 30 | 0.0 | 5 | 25 | 0.4 | 0.3 | 0.5 | 1.10 | 0.60 | 0.00 | 0.30 | 0.50 | 0.10 | 0.00 | 0.05 | 0.10 | 0.00 | 0.05 | 0.00 | 1.40 | 0.00 | 1.10 | 1.30 |

## Drop structure

The drops are provided at places where the ground slope is steeper than the canal bed slopes. For irrigation canals, vertical drop structure of U.S.B R type standard is selected to convey water from higher to lower elevation of the ground.  The design procedures are shown below:

a. Critical hydraulic

Design discharge, Q (m3/s)

Height of drop, h (m)

Width of drop, bc =

Where d = water depth of the canal, m

Critical discharge, q = Q/bc

Critical depth, dc =

Lip height, a = dc/2, a  0.15

b. Stilling basin

Basin width, B =

Basin length, L =

The standard drawing for all the drops to be constructed is found in the irrigation infrastructure drawings on which the above parameters are used to code each of the dimensions of the drops.

All the hydraulic parameters computed using the above procedures with the type of canals are shown in

Table 5. 5: Drop structures hydraulic parameters

| Canal name | Chainage (m) | D1, (m) | b, m | Q (m3/sec) | h, m | bc, m | q (m3/s/m) | dc, m | a, m | L, m | B, m | Lup (m) | Head Above Crest, m | Ldp (m) | Dc U/s, m | Dc D/s, m | tb, m | tp, m |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| MC-1 | 0+451 | 0.7 | 0.7 | 0.0475 | 1 | 0.30 | 0.16 | 0.14 | 0.15 | 0.98 | 0.40 | 0.60 | 0.10 | 3.00 | 0.30 | 0.30 | 0.30 | 0.30 |
| MC-1 | 0+489 | 0.7 | 0.7 | 0.0475 | 0.55 | 0.30 | 0.16 | 0.14 | 0.15 | 0.76 | 0.40 | 0.60 | 0.12 | 3.00 | 0.30 | 0.30 | 0.30 | 0.30 |
| MC-1 | 0+527 | 0.7 | 0.7 | 0.097 | 0.55 | 0.30 | 1.83 | 0.70 | 0.35 | 3.31 | 1.31 | 0.45 | 0.21 | 3.00 | 0.30 | 0.30 | 0.30 | 0.30 |
| Mc-1 | 1+278 | 0.7 | 0.7 | 0.075 | 0.75 | 0.30 | 2.50 | 0.86 | 0.43 | 3.87 | 1.50 | 0.45 | 0.23 | 3.00 | 0.30 | 0.30 | 0.30 | 0.30 |
| Mc-1 | 1+296 | 0.7 | 0.7 | 0.07 | 0.9 | 0.30 | 3.00 | 0.97 | 0.49 | 4.27 | 1.62 | 0.45 | 0.24 | 3.00 | 0.30 | 0.30 | 0.30 | 0.30 |
| Mc-1 | 1+372 | 0.7 | 0.7 | 0.04 | 0.55 | 0.30 | 1.83 | 0.70 | 0.35 | 3.31 | 1.31 | 0.46 | 0.21 | 3.00 | 0.30 | 0.30 | 0.30 | 0.30 |
| Mc-1 | 1+409 | 0.7 | 0.7 | 0.07 | 0.45 | 0.30 | 0.23 | 0.18 | 0.15 | 0.84 | 0.49 | 0.50 | 0.14 | 3.00 | 0.30 | 0.30 | 0.30 | 0.30 |
| Mc-1 | 1+485 | 0.7 | 0.7 | 0.11 | 0.5 | 0.30 | 0.37 | 0.24 | 0.15 | 1.07 | 0.61 | 0.45 | 0.16 | 3.00 | 0.30 | 0.30 | 0.30 | 0.30 |
| Mc-1 | 1+767 | 0.7 | 0.7 | 0.0475 | 1 | 0.30 | 0.16 | 0.14 | 0.15 | 0.98 | 0.40 | 0.60 | 0.10 | 3.00 | 0.30 | 0.30 | 0.30 | 0.30 |
| Mc-1 | 1+786 | 0.7 | 0.7 | 0.0475 | 0.55 | 0.30 | 0.16 | 0.14 | 0.15 | 0.76 | 0.40 | 0.60 | 0.12 | 3.00 | 0.30 | 0.30 | 0.30 | 0.30 |
| Mc-1 | 1+805 | 0.7 | 0.7 | 0.097 | 0.55 | 0.30 | 1.83 | 0.70 | 0.35 | 3.31 | 1.31 | 0.45 | 0.21 | 3.00 | 0.30 | 0.30 | 0.30 | 0.30 |
| Mc-1 | 1+824 | 0.7 | 0.7 | 0.075 | 0.75 | 0.30 | 2.50 | 0.86 | 0.43 | 3.87 | 1.50 | 0.45 | 0.23 | 3.00 | 0.30 | 0.30 | 0.30 | 0.30 |
| Mc-1 | 1+842 | 0.7 | 0.7 | 0.07 | 0.9 | 0.30 | 3.00 | 0.97 | 0.49 | 4.27 | 1.62 | 0.45 | 0.24 | 3.00 | 0.30 | 0.30 | 0.30 | 0.30 |
| Mc-1 | 2+125 | 0.5 | 0.5 | 0.04 | 0.55 | 0.30 | 1.83 | 0.70 | 0.35 | 3.31 | 1.31 | 0.46 | 0.21 | 3.00 | 0.30 | 0.30 | 0.30 | 0.30 |
| Mc-1 | 2+163 | 0.5 | 0.5 | 0.07 | 0.45 | 0.30 | 0.23 | 0.18 | 0.15 | 0.84 | 0.49 | 0.50 | 0.14 | 3.00 | 0.30 | 0.30 | 0.30 | 0.30 |
| Mc-1 | 2+182 | 0.5 | 0.5 | 0.11 | 0.5 | 0.30 | 0.37 | 0.24 | 0.15 | 1.07 | 0.61 | 0.45 | 0.16 | 3.00 | 0.30 | 0.30 | 0.30 | 0.30 |
| Mc-1 | 2+201 | 0.5 | 0.5 | 0.04 | 0.55 | 0.30 | 1.83 | 0.70 | 0.35 | 3.31 | 1.31 | 0.46 | 0.21 | 3.00 | 0.30 | 0.30 | 0.30 | 0.30 |
| Mc-1 | 2+2019 | 0.5 | 0.5 | 0.07 | 0.45 | 0.30 | 0.23 | 0.18 | 0.15 | 0.84 | 0.49 | 0.50 | 0.14 | 3.00 | 0.30 | 0.30 | 0.30 | 0.30 |
| Mc-1 | 2+238 | 0.5 | 0.5 | 0.11 | 0.5 | 0.30 | 0.37 | 0.24 | 0.15 | 1.07 | 0.61 | 0.45 | 0.16 | 3.00 | 0.30 | 0.30 | 0.30 | 0.30 |
| Mc-1 | 2+257 | 0.5 | 0.5 | 0.04 | 0.55 | 0.30 | 1.83 | 0.70 | 0.35 | 3.31 | 1.31 | 0.46 | 0.21 | 3.00 | 0.30 | 0.30 | 0.30 | 0.30 |
| Mc-1 | 2+276 | 0.5 | 0.5 | 0.07 | 0.45 | 0.30 | 0.23 | 0.18 | 0.15 | 0.84 | 0.49 | 0.50 | 0.14 | 3.00 | 0.30 | 0.30 | 0.30 | 0.30 |
| Mc-1 | 2+295 | 0.5 | 0.5 | 0.11 | 0.5 | 0.30 | 0.37 | 0.24 | 0.15 | 1.07 | 0.61 | 0.45 | 0.16 | 3.00 | 0.30 | 0.30 | 0.30 | 0.30 |
| Mc-1 | 2+314 | 0.5 | 0.5 | 0.04 | 0.55 | 0.30 | 1.83 | 0.70 | 0.35 | 3.31 | 1.31 | 0.46 | 0.21 | 3.00 | 0.30 | 0.30 | 0.30 | 0.30 |
| Mc-1 | 2+333 | 0.5 | 0.5 | 0.07 | 0.45 | 0.30 | 0.23 | 0.18 | 0.15 | 0.84 | 0.49 | 0.50 | 0.14 | 3.00 | 0.30 | 0.30 | 0.30 | 0.30 |
| Mc-1 | 2+746 | 0.5 | 0.5 | 0.11 | 0.5 | 0.30 | 0.37 | 0.24 | 0.15 | 1.07 | 0.61 | 0.45 | 0.16 | 3.00 | 0.30 | 0.30 | 0.30 | 0.30 |
| Mc-1 | 2+896 | 0.5 | 0.5 | 0.11 | 0.5 | 0.30 | 0.37 | 0.24 | 0.15 | 1.07 | 0.61 | 0.45 | 0.16 | 3.00 | 0.30 | 0.30 | 0.30 | 0.30 |

There are a lot of drops also at secondary canal please sea at secondary canal profiles

Table 5. 6: Infrastructure cost

|  | Bahirilibo Irrigation Project Infrastucture Cost Estimate |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | BILL PART-3 INFRASTRUCTURE |  |  |  |  |
| I. No | Descriptions | Unit | Qty | Rate (Birr) | Amount (Birr) |
| 3.100 | Main Canal |  |  |  |  |
|  | Earthworks |  |  |  |  |
| 3.1.1 | Common Soil Excavation to a any depth including haul to a distance not exceeding 200m. | m3 | 3,844.2 | 68.9 | 264,901.1 |
| 3.1.2 | Soft Rock Excavation including haul to a distance not exceeding 200m. | m3 | 427.1 | 468.4 | 200,059.1 |
| 3.1.3 | Back filling with selected material from excavated material including compaction. | m³ | 3,074.0 | 51.4 | 158,005.5 |
|  | Masonry Work |  |  |  |  |
| 3.1.4 | Basaltic or equivalent stone masonry canal wall erection well bedded in cement sand mortar of 1:3 mix ratio .The thickness of masonry wall will be as indicated in the drawing. The work should be to the satisfaction of the Engineer | m3 | 2,823.0 | 1,804.6 | 5,094,190.5 |
| 3.1.5 | Internal plastering for the masonry wall | m2 | 6,924.0 | 92.0 | 637,008.0 |
|  | Turnout/ Division box |  |  |  |  |
|  | Earth Work and Excavatio |  |  |  |  |
| 3.1.6 | Common Soil Excavation to a any depth including haul to a distance not exceeding 200m. | m3 | 59.75 | 68.9 | 4,117.03 |
|  | Concrete |  |  |  |  |
| 3.1.7 | Supply and cast lean concrete quality C-10 filled in to formwork and vibrated. | m3 | 17.86 | 1,299.35 | 23,209.64 |
|  | Masonry Work |  |  |  |  |
| 3.1.8 | Basaltic or equivalent stone masonry canal wall erection well bedded in cement sand mortar of 1:3 mix ratio .The thickness of masonry wall will be as indicated in the drawing. The work should be to the satisfaction of the Engineer.Division box should be constructed at each secondary canal intrance point | m3 | 72.06 | 1,804.55 | 130,035.87 |
| 3.1.9 | Internal plastering for the masonry wall | m2 | 183.52 | 92.00 | 16,883.84 |
| 3.200 | HDPE pipe 10mm internal diameter and 1m length for turnouts to addresss required discharge to the field canal | No | 10.00 | 150.00 | 1,500.00 |
| 3.2.1 | 4mm thick sheet metal |  |  |  |  |
|  | Stiffening angle iron (50x50x4) | Pcs | 8.00 | 3000.00 | 24,000.00 |
|  | Angle iron for groove (0.8x0.8x10) |  |  |  |  |
|  | 16mm reinforcement bar for handling |  |  |  |  |
| 3.2.2 | Drop structure |  |  |  |  |
|  | Concrete |  |  |  |  |
| 3.2.3 | Supply and cast lean concrete quality C-10 filled in to formwork and vibrated. | m3 | 1.27 | 1299.35 | 1650.17 |
|  | Masonry Work |  |  |  |  |
| 3.2.4 | Basaltic or equivalent stone masonry retaining wall erection well bedded in cement sand mortar of 1:3 mix ratio .The thickness of masonry wall will be as indicated in the drawing. The work should be to the satisfaction of the Engineer | m3 | 165.60 | 1804.55 | 298833.48 |
| 3.2.5 | Internal plastering for the masonry wall | m2 | 225.60 | 92.00 | 20755.20 |
| Total Carried to Summary for Main Canal |  |  |  |  | 6875149.47 |
| 3.3.1 | Secondary Canal SC1 |  |  |  |  |
|  | Earthworks |  |  |  |  |
| 3.3.2 | Common Soil Excavation to a any depth including haul to a distance not exceeding 200m. | m3 | 1899.57 | 68.91 | 130,899.16 |
| 3.3.3 | Filling with selected material from excavated material including compaction. | m³ | 1248.90 | 51.40 | 64,193.46 |
| 3.3.4 | Hard rock excavation | m3 | 325.00 | 650.00 | 211,250.00 |
|  | Masonry Work |  |  |  |  |
| 3.3.4 | Basaltic or equivalent stone masonry canal wall erection well bedded in cement sand mortar of 1:3 mix ratio .The thickness of masonry wall will be as indicated in the drawing. The work should be to the satisfaction of the Engineer | m3 | 874.38 | 1804.55 | 1,577,862.43 |
| 3.3.5 | Internal plastering for the masonry wall | m2 | 2914.60 | 92.00 | 268,143.20 |
|  | Turnout/ Division box |  |  |  |  |
|  | Concrete |  |  |  |  |
| 3.3.6 | Supply and cast lean concrete quality C-10 filled in to formwork and vibrated. | m³ | 5.20 | 1299.35 | 6,756.62 |
|  | Masonry Work |  |  |  |  |
| 3.3.7 | Basaltic or equivalent stone masonry retaining wall erection well bedded in cement sand mortar of 1:3 mix ratio .The thickness of masonry wall will be as indicated in the drawing. The work should be to the satisfaction of the Engineer | m3 | 76.45 | 1804.55 | 137,957.85 |
| 3.3.8 | Internal plastering for the masonry wall | m2 | 75.60 | 92.00 | 6,955.20 |
| 3.3.9 | 30cm Cemented Stone Pitching as per the engineers interest | m3 | 18.63 | 98.25 | 1,830.40 |
|  | Gate |  |  |  |  |
|  | 4mm thick sheet metal |  |  |  |  |
| 3.4.1 | Stiffening angle iron (50x50x4) |  |  |  |  |
|  | Angle iron for groove (0.8x0.8x10) | Pcs | 6.00 | 3000.00 | 18,000.00 |
|  | 16mm reinforcement bar for handling |  |  |  |  |
| 3.4.2 | HDPE pipe 10mm internal diameter and 1m length for turnouts to addresss required discharge to the field canal | Pcs | 7.00 | 250.00 | 1,750.00 |
|  | Drop structure |  |  |  |  |
|  | Concrete |  |  |  |  |
| 3.4.3 | Supply and cast lean concrete quality C-10 filled in to formwork and vibrated. | m³ | 10.30 | 1299.35 | 13383.31 |
|  | Masonry Work |  |  |  |  |
| 3.4.4 | Basaltic or equivalent stone masonry retaining wall erection well bedded in cement sand mortar of 1:3 mix ratio .The thickness of masonry wall will be as indicated in the drawing. The work should be to the satisfaction of the Engineer | m3 | 25.60 | 1804.55 | 46196.48 |
| 3.4.5 | Internal plastering for the masonry wall | m2 | 125.40 | 92.00 | 11536.80 |
| 3.4.6 | 30cm Cemented Stone Pitching as per the engineers interest | m3 | 35.40 | 98.25 | 3478.05 |
| Total Carried to Summary for Secondary Canals 1 |  |  |  |  | 2,500,192.95 |
| 3.4.7 | Secondary Canal (Sc-2) |  |  |  |  |
|  | Earthworks |  |  |  |  |
| 3.4.8 | Common Soil Excavation to a any depth including haul to a distance not exceeding 200m. | m3 | 867.56 | 68.91 | 59783.56 |
| 3.4.9 | Filling with selected material from excavated material including compaction. |  | 195.95 | 51.40 | 10071.83 |
|  | Masonry Work |  |  |  |  |
| 3.5.1 | Basaltic or equivalent stone masonry canal wall erection well bedded in cement sand mortar of 1:3 mix ratio .The thickness of masonry wall will be as indicated in the drawing. The work should be to the satisfaction of the Engineer | m3 | 418.37 | 1804.55 | 754969.58 |
| 3.5.2 | Internal plastering for the masonry wall |  | 1394.57 | 92.00 | 128300.71 |
|  | Turnout/ Division box |  |  |  |  |
|  | Concrete |  |  |  |  |
| 3.5.3 | Supply and cast lean concrete quality C-10 filled in to formwork and vibrated. | m3 | 6.70 | 1299.35 | 8705.65 |
|  | Masonry Work |  |  |  |  |
| 3.5.4 | Basaltic or equivalent stone masonry retaining wall erection well bedded in cement sand mortar of 1:3 mix ratio .The thickness of masonry wall will be as indicated in the drawing. The work should be to the satisfaction of the Engineer | m3 | 12.30 | 1804.55 | 22195.97 |
| 3.5.6 | Internal plastering for the masonry wall |  | 24.20 | 92.00 | 2226.40 |
| 3.5.7 | 30cm Cemented Stone Pitching as per the engineers interest |  | 16.20 | 98.25 | 1591.65 |
|  | Gate |  |  |  |  |
|  | 4mm thick sheet metal |  |  |  |  |
| 3.5.8 | Stiffening angle iron (50x50x4) |  |  |  |  |
|  | Angle iron for groove (0.8x0.8x10) | Pcs | 4.00 | 3000.00 | 12000.00 |
|  | 16mm reinforcement bar for handling |  |  |  |  |
| 3.5.9 | HDPE pipe 10mm internal diameter and 1m 1m length for turnouts to addresss required discharge to the field canal | Pcs | 4.00 | 250.00 | 1000.00 |
|  | Drop structure |  |  |  |  |
|  | Concrete |  |  |  |  |
| 3.6.1 | Supply and cast lean concrete quality C-10 filled in to formwork and vibrated. | m3 | 5.00 | 1299.35 | 6496.75 |
|  | Masonry Work |  |  |  |  |
| 3.6.2 | Basaltic or equivalent stone masonry retaining wall erection well bedded in cement sand mortar of 1:3 mix ratio .The thickness of masonry wall will be as indicated in the drawing. The work should be to the satisfaction of the Engineer | m3 | 23.20 | 1804.55 | 41865.56 |
| 3.6.3 | Internal plastering for the masonry wall |  | 63.40 | 92.00 | 5832.80 |
| Total Carried to Summary for Secondary Canals 2 |  |  |  |  | 1,055,040.45 |
|  | Secondary Canal SC-3 |  |  |  |  |
|  | Earthworks |  |  |  |  |
| 3.6.4 | Common Soil Excavation to a any depth including haul to a distance not exceeding 200m. | m3 | 582.00 | 68.91 | 40105.62 |
| 3.6.5 | Filling with selected material from excavated material including compaction. |  | 631.41 | 51.40 | 32454.47 |
|  | Masonry Work |  |  |  |  |
| 3.6.6 | Basaltic or equivalent stone masonry canal wall erection well bedded in cement sand mortar of 1:3 mix ratio .The thickness of masonry wall will be as indicated in the drawing. The work should be to the satisfaction of the Engineer | m3 | 454.58 | 1804.55 | 820312.34 |
| 3.6.6 | Internal plastering for the masonry wall |  | 1348.76 | 92.00 | 124085.92 |
|  | Turnout/ Division box |  |  |  |  |
|  | Concrete |  |  |  |  |
| 3.6.7 | Supply and cast lean concrete quality C-10 filled in to formwork and vibrated. | m3 | 25.60 | 1,299.35 | 33,263.36 |
|  | Masonry Work |  |  |  |  |
| 3.6.8 | Basaltic or equivalent stone masonry retaining wall erection well bedded in cement sand mortar of 1:3 mix ratio .The thickness of masonry wall will be as indicated in the drawing. The work should be to the satisfaction of the Engineer | m3 | 15.00 | 1,804.55 | 27,068.25 |
| 3.6.9 | Internal plastering for the masonry wall | m2 | 45.60 | 92.00 | 4,195.20 |
| 3.7.1 | 30cm Cemented Stone Pitching as per the engineers interest |  | 5.30 | 98.25 | 520.73 |
|  | Gate |  |  |  |  |
| 3.8.1 | 4mm thick sheet metal |  |  |  |  |
|  | Stiffening angle iron (50x50x4) | Pcs | 4.00 | 3,000.00 | 12,000.00 |
|  | Angle iron for groove (0.8x0.8x10) |  |  |  |  |
|  | 16mm reinforcement bar for handling |  |  |  |  |
| 3.8.2 | Hdp pipe 10mm internal diameter and 1m 1m length for turnouts to addresss required discharge to the field canal | Pcs | 2.00 | 250.00 | 500.00 |
|  | Drop structure |  |  |  |  |
|  | Concrete |  |  |  |  |
| 3.8.3 | Supply and cast lean concrete quality C-10 filled in to formwork and vibrated. |  | 36.30 | 1,299.35 | 47,166.41 |
|  | Masonry Work |  |  |  |  |
| 3.8.4 | Basaltic or equivalent stone masonry retaining wall erection well bedded in cement sand mortar of 1:3 mix ratio .The thickness of masonry wall will be as indicated in the drawing. The work should be to the satisfaction of the Engineer | m3 | 12.30 | 1,804.55 | 22,195.97 |
| 3.8.5 | Internal plastering for the masonry wall |  | 45.60 | 92.00 | 4,195.20 |
| Total Carried to Summary for Secondary Canals 3 |  |  |  |  | 1,168,063.46 |
|  | Secondary Canal SC-4 |  |  |  |  |
|  | Earthworks |  |  |  |  |
| 3.8.6 | Common Soil Excavation to a any depth including haul to a distance not exceeding 200m. | m3 | 317.68 | 68.9 | 21,891.33 |
| 3.8.7 | Filling with selected material from excavated material including compaction. | m3 | 176.32 | 51.4 | 9,062.85 |
|  | Masonry Work |  |  |  |  |
| 3.8.8 | Basaltic or equivalent stone masonry canal wall erection well bedded in cement sand mortar of 1:3 mix ratio .The thickness of masonry wall will be as indicated in the drawing. The work should be to the satisfaction of the Engineer | m3 | 211.20 | 1,804.6 | 381,120.96 |
| 3.8.8 | Internal plastering for the masonry wall | m2 | 704.00 | 92.0 | 64,768.00 |
|  | Turnout/ Division box |  |  |  |  |
|  | Concrete |  |  |  |  |
| 4.1.1 | Supply and cast lean concrete quality C-10 filled in to formwork and vibrated. |  | 5.00 | 1,299.35 | 6,496.75 |
|  | Masonry Work |  |  |  |  |
| 4.1.2 | Basaltic or equivalent stone masonry retaining wall erection well bedded in cement sand mortar of 1:3 mix ratio .The thickness of masonry wall will be as indicated in the drawing. The work should be to the satisfaction of the Engineer |  | 9.60 | 1,804.55 | 17,323.68 |
| 4.1.3 | Internal plastering for the masonry wall |  | 25.30 | 92.00 | 2,327.60 |
| 4.1.4 | 30cm Cemented Stone Pitching as per the engineers interest |  | 4.30 | 98.25 | 422.48 |
|  | Gate |  |  |  |  |
|  | 4mm thick sheet metal |  |  |  |  |
| 4.1.5 | Stiffening angle iron (50x50x4) | Pcs | 4.00 | 3,000.00 | 12,000.00 |
|  | Angle iron for groove (0.8x0.8x10) |  |  |  |  |
|  | 16mm reinforcement bar for handling |  |  |  |  |
| 4.1.6 | Hdp pipe 10mm internal diameter and 1m 1m length for turnouts to addresss required discharge to the field canal | Pcs | 8.00 | 250.00 | 2,000.00 |
|  | Drop structure |  |  |  |  |
|  | Concrete |  |  |  |  |
| 4.1.7 | Supply and cast lean concrete quality C-10 filled in to formwork and vibrated. |  | 5.30 | 1,299.35 | 6,886.56 |
|  | Masonry Work |  |  |  |  |
| 4.1.8 | Basaltic or equivalent stone masonry retaining wall erection well bedded in cement sand mortar of 1:3 mix ratio .The thickness of masonry wall will be as indicated in the drawing. The work should be to the satisfaction of the Engineer |  | 36.34 | 1,804.55 | 65,577.35 |
|  | Internal plastering for the masonry wall |  | 125.00 | 92.00 | 11,500.00 |
|  | Tertiary Canal Earthworks (Tc-1,2,3,4,5,6,7&9) |  |  |  |  |
| 4.2.1 | Common Soil Excavation in trapezoidal Canal to a any depth including haul to a distance not exceeding 200m. | m3 | 2400.56 | 68.91 | 165,422.59 |
| 4.2.2 | Filling with selected material from borrow pits including compaction (hauling distance not exceeding 5km) | m³ | 255.00 | 51.40 | 13,107.00 |
|  | Tertiary Canal Earthworks (Tc-8) |  |  |  |  |
| 4.2.3 | Common Soil Excavation to a any depth including haul to a distance not exceeding 200m. | m³ | 215.30 | 68.91 | 14,836.32 |
| 4.2.4 | Filling with selected material from excavated material including compaction. |  | 210.60 | 51.40 | 10,824.84 |
|  | Masonry Work |  |  |  |  |
| 4.2.5 | Basaltic or equivalent stone masonry canal wall erection well bedded in cement sand mortar of 1:3 mix ratio .The thickness of masonry wall will be as indicated in the drawing. The work should be to the satisfaction of the Engineer | m³ | 185.40 | 1,804.55 | 334,563.57 |
| 4.2.6 | Internal plastering for the masonry wall | m2 | 616.00 | 92.00 | 56,672.00 |
|  | Drop structure For Tritary canals |  |  |  |  |
|  | Concrete |  |  |  |  |
| 4.2.7 | Supply and cast lean concrete quality C-10 filled in to formwork and vibrated. |  | 6.50 | 1,299.35 | 8,445.78 |
|  | Masonry Work |  |  |  |  |
| 4.2.8 | Basaltic or equivalent stone masonry retaining wall erection well bedded in cement sand mortar of 1:3 mix ratio .The thickness of masonry wall will be as indicated in the drawing. The work should be to the satisfaction of the Engineer |  | 76.15 | 1,804.55 | 137,416.48 |
| 4.2.9 | Internal plastering for the masonry wall |  | 153.45 | 92.00 | 14,117.40 |
| Total Carried to Summary for Secondary Canals 4 |  |  |  |  | 1,356,783.52 |
| Total Carried to Summary for secondary &Tertiary Canals |  |  |  |  | 6,080,080.38 |
| 4.3.1 | Flume |  |  |  |  |
| 4.3.2 | Common Soil Excavation to a any depth including haul to a distance not exceeding 200m | m3 | 2600.00 | 68.91 | 179,166.00 |
| 4.3.3 | Hard Rock excavation | m3 | 156.30 | 650.00 | 101,595.00 |
| 4.3.4 | Filling with selected material from excavated material including compaction. | m3 | 145.60 | 51.40 | 7,483.84 |
|  | Masonry Work |  |  |  |  |
| 4.3.5 | Basaltic or equivalent stone masonry retaining wall erection well bedded in cement sand mortar of 1:3 mix ratio .The thickness of masonry wall will be as indicated in the drawing. The work should be to the satisfaction of the Engineer | m3 | 176.40 | 1,804.55 | 318,322.62 |
| 4.3.6 | pointing | m2 | 345.70 | 150.00 | 51,855.00 |
|  | Concrete |  |  |  |  |
| 4.3.7 | Reinforced concrete of class C -20. Formwork will be measured separately | m3 | 125.60 | 2,234.34 | 280,633.10 |
|  | Reinforcement |  |  |  |  |
| 4.3.8 | dia 12 | kg | 3500.00 | 33.09 | 115,815.00 |
| 4.3.9 | dia 14 | kg | 3027.50 | 33.79 | 102,299.23 |
|  | Formwork |  |  |  |  |
| 4.4.1 | Supply, install and remove after use formwork, timbered or steel or a combination of two, sufficient to contain the wet concrete without leakage. Enough to support temporary loading and pressure from placing compaction or vibration without displacement or appreciable deflection. Formwork shall not be prepared for concrete with old oil, but be fit for fair face internal concrete surfaces. | m2 | 404.48 | 357.94 | 144,779.57 |
| 4.4.2 | Supply and install 300mm wide water stop as mentioned in the drawing and as per the Engineer's instruction. | m | 604.00 | 5.20 | 3,140.80 |
| 4.4.3 | 0.3cm thick styrofoam | m | 78.02 | 10.40 | 811.41 |
| Total Carried to Summary for Flume |  |  |  |  | 1,305,901.57 |
| 4.4.4 | Box culvert |  |  |  |  |
| 4.4.5 | Common Soil Excavation to a any depth including haul to a distance not exceeding 200m | m3 | 6.56 | 68.91 | 452.05 |
| 4.4.6 | Filling with selected material from borrow pits including compaction (hauling distance not exceeding 5km) |  | 1.64 | 51.40 | 84.30 |
|  | Concrete |  |  |  |  |
| 4.4.7 | Reinforced concrete of class C -20. Formwork will be measured separately | m3 | 7.30 | 2234.34 | 16,310.68 |
|  | Reinforcement |  |  |  |  |
| 4.4.8 | Dia.6 | kg | 7.10 | 70.00 | 497.00 |
| 4.4.9 | dia 10 | kg | 96.18 | 70.00 | 6,732.60 |
| 4.4.10 | dia 12 | kg | 127.85 | 70.00 | 8,949.50 |
| Total Carried to Summary for Box Culvert |  |  |  |  | 33,026.13 |
| 3.5.1 | Road crossing |  |  |  |  |
|  | Masonry Work |  |  |  |  |
| 3.5.6 | Basaltic or equivalent stone masonry canal wall erection well bedded in cement sand mortar of 1:3 mix ratio .The thickness of masonry wall will be as indicated in the drawing. The work should be to the satisfaction of the Engineer | m3 | 160.50 | 1804.55 | 289,630.28 |
| 3.5.7 | pointing | m2 | 125.60 | 75.00 | 9,420.00 |
| 3.5.8 | Supply and cast Reinforced concrete quality C-20 filled in to formwork and vibrated. | m3 | 25.60 | 2234.34 | 57,199.10 |
|  | Reinforcement |  |  |  |  |
| 3.5.9 | dia 10 | kg | 562.30 | 70.00 | 39,361.00 |
| 3.6.1 | dia 12 | kg | 470.60 | 70.00 | 32,942.00 |
| Total Carried to Summary for Road Crossing |  |  |  |  | 428552.38 |
| Total Carried to Summary for Infrastructure |  |  |  |  | 14,722,709.93 |

# OPERATION AND MAINTENANCE

## General

The main objective of the operation and maintenance aspect of an irrigation scheme is to facilitate the timely delivery of the required irrigation water to farms and to keep the irrigation system in an optimum operating condition.  This section therefore, discusses the main functions of the subject matter under consideration for the scheme.

## Operation of the Head Works

Operation at the Headwork mainly focuses on the  a controlled flow of river water, timely cleaning of floating debris and removal of sediment deposits in front of  offtake  .

The design discharge of the main canal is 152 l/sec for continuous flow of 18-hour irrigation period during wet season irrigation.

## Irrigation System Operation

The operation of the irrigation system depends mainly on the method of water delivery at farm level.  Surface irrigation method is the recommended type of water distribution and application method for Bahir Libo project.

The farmers would organize themselves and form groups in order to handle the water management.  Rotational water distribution would be applied within the group.  The rotational distribution is then to distribute water by turn to the whole scheme according to the timely need of crop water requirement. For better and efficient water management, crop diversification should be avoided within a group.  This would reduce the complexity of water distribution system of the scheme during one irrigation season. At farmers’ level of operation, a constant flow and variable irrigation time is advisable. Detail of operation and maintenance will be worked out in the final report as separate document.

### Maintenance Requirement

The maintenance tasks are categorized into two types: - routine activities, and repairs.  The routine maintenance activities that are carried out periodically include-

- Regular cleaning of sediments and weeds from canals and drains;

- Inspection and lubrication of gates; and

- Maintenance of cracked lined canals, regulating and control structures.

Repair works include task carried out more frequently and quickly, and include those task that are generally unpredictable.  They also include emergency works.  The activities included in this category are-

- Repairing overtopped or breached canals, drains, and flood protection dykes;

- Repairing jammed gates;

- Filling holes made by wild animals; and

- Reduced free board due to walking over by people and livestock.

Regular inspection of the irrigation facilities should be carried out as part of the maintenance activities.  These tasks could be carried out immediately after the end of the main rainy sesean in September and during the rainy season.  This could concentrate on the interceptor drains and the flood protection dykes, the main canal and the field drains.  The inspection of the other works like the tertiary canals, field drains, and the water control and regulating structures could be carried.

# CONCLUSION AND RECOMMENDATION

- The infrastructure of this project area is designed to irrigate about 104 ha of land by taking its supply from the Bahir LiboRiver.

- The base flow of the river is too low to irrigate the required amount of the command area due to this we are design the canal dimensions and other structures for supplementary irrigation.

- Under considering the standard of living of farmers, the concerned body should have to be implementing the project early.

- The method of irrigation of the project area is furrow, surface irrigation in which the main and tertiary canals are working continuously where as the field canals within a tertiary block are working rotational system.

- The project is fissible from the economic persepective,social and Environmentally

The following recommendations are drown:

- For better performance and long service year of the project regular inspection and maintenance is highly required.

- Farmers training, how to operate and maintain the project structures as a whole and available and water resources has a paramount important.

- The irrigation hours per day and per week should be flexible based on base flow amount of each week or month.

- The main canal should be protected from erosion and sedimentation by growing suitable grasses

- Water management system should be well organized and implemented for better use of the limited water resource.

# REFERENCE

- Gilgel Abay Diversion Irrigation Project Design Report(GADIP)

- Ethiopian Ministry of irrigation department manual(IDD)

- Ajema Chacha Dam Irrigation Project Design Report

- Design of small Canal structures , USBR

- Applied Hydrology by Ven Te Chow Dvid R. Maidment

- Soft copies of hydraulic structure publishing

- IDD manual