Contents

1-EXECUTIVE SUMMARY OF Balo-habebe SSIP FINANICIAL ANALAYSIS RESULT	5

1.1Balo Habebe SSIP-Socio-Economy and community participation	6

1. 1. 2. OBJECTIVE OF SOCIO-ECONOMIC STUDY	8

1.1.3 The scope of the study	8

1.1.4  METHODOLOGY	9

1.1.5.LOCATION AND PHYSICAL SOCIO ECONOMIC CHARACTERISTIC	9

1.3. Land Use and Land Cover	14

1.4. LIVESTOCK DEVELOPMENT	19

1.5. Urban- rural market linkage	21

1.6.BASIC SOCIAL SERVICE AND INFRASTRUCTURE	25

1.7.GENDER ISSUES	30

Conclusion and Recommendation	31

1.7. COMMUNITY PARTICIPATION	31

2. Agronomy	36

2.4. Length of growing period (Lgp) of existing crops of project area.	38

2.1.6. Existing cropping season of the project area.	39

2.8. Existing cropping pattern of rain fed crop production	40

2.1.9.1 Land preparation	40

2.1.9.2 Planting (sowing) methods	40

2.11. Existing crop disease pests and control methods of the project area.	41

2.17 constraints of crop production system of the project area	46

2.21. The possible list of crops (crop basket) for proposed irrigation project area	47

2.28. Management practices of proposed crops	62

2.28.2. Establishment methods of proposed crops in the field	62

2.28.3 Methods of planting and seed rate of proposed crops	63

2.28.8 Maintaining soil productivity of project area	66

2.29. Labour requirement of proposed crops	71

2.30. Conclusion and recommendation	72

3. SOIL and land suitability	74

3.3 Soil physical   properties	75

3.8 Basic soil—water relation	79

3.8.4 Description OF INDIVIDUAL SOIL PROFILE	80

3.8.5   Soil chemical characteristics	82

4. WATERSHED MANAGEMENT	88

4.6. PROPOSED WATERSHED AREA	90

VERTICAL EXTEYSION	91

4.7 MAJOR PROBLEMS OF THE WATERSHED AREA	95

5. ENVIRONMENTAL IMPACT ASSESSMENT	98

Mitigation measure	99

On soil	100

5.5 CONCLUSION AND RECOMMENDATION	101

6. HYDROLOGY	102

MEAN MONTHLY TEMPERATURE (in 0C),	104

MONTHLY RAINFALL (in mm)	106

MEAN MONTHLY SUNSHINE DURATION (in hrs),	108

MEAN MONTHLY HUMIDITY (in %),	109

MEANS MONTHLY WIND SPEED (m/s)	109

7. GEOLOGY, HYDROGEOLOGY AND GEOTECHNIQUES	122

7.1 BACKGROUND	122

7.1.1   LOCATION AND ACCESSIBILITY`	122

7.4 HYDROGEOLOGY	125

7.4.1 Tebela spring nature	125

7.4.2 Groundwater condition of the area	125

7.5. Regional and catchments geology	126

7.6. LOCAL GEOLOGY	126

7.9. ENGINEERING GEOLOGICAL CONDITIONS	133

7.10. CONSTRUCTION MATERIAL ASSESSMENT	133

7.10.1 Coarse aggregate and building stone	133

7.10.2 Sand	133

7.11. RECOMMENDATION AND CONCLUSIONS	134

8.ENGINEERING	135

8.1.2 Methodology of data collection	135

8.1.3 PROJECT HEAD WORK	135

8.2 DESIGN OF PROJECT SYSTEMS AND ITS STRUCTURES	136

8.3. Design of Canals and structures on the canals	141

8.3.2 Design of right side Main canal	144

144

[9	144

8.4 DESIGN OF PROJECT SYSTEM STRUCTURES	145

8.4.2 Design RCC flume	149

8.4.2.1     Characteristics of Mother canal	149

8.4.2.3.2, Design for links (stirrups)	153

I = moment of inertia of the base area	159

8.5 DESGIN OF FOOT PATH	165

.5 Design of Drop	169

8.6 SAMPLE DESIGN OF Off Takes	173

Operation of the Diversion Guide wall	175

Annex 2. SPECIFICATION	176

8. Earth work-	177

Annex 3. Unit Rate Analysis	179

8.7  Unit Rate Calculation for the project.	180

8.10. Operation and maintenance	201

8.10.1 OPERATION	201

8.10.2 MAINTENANCE	201

8.11 . Specification	202

8.11.1 Materials	202

8.11.2 Box Specification	205

-     Conclusion and recommendation	206

Conclusion	206

-Recommendation:	206

# List of participant

| No | Name | Profession | Contribution |
| --- | --- | --- | --- |
| 1 | Bayan Ahmad | Economist | -Socio-economy -Community participation |
| 2 | Sefu Hussen | Agronomist | Agricultural Development |
| 3 | Abdurashid Namo | Engineer | -Water shed management -Environmental impact Assessment - Hydrology - Compiling and editing the Whole Document and revision for design and drawing part -Consulting for all irrigation infrastructures |
| 4 | Zeleke Agonafir | Agronomist | -Soil study |
| 5 | Diriba Bayane | Engineer | - Head work design - Canal network system design - All farm structure design |
| 6 | Kalid Husen | Surveyor | Surveying the project area |

# Salient Feature

Name and type of the project: Balo Habebe  (Weir)

River basin: Wabe shabele

Source: - spring

Diversion system:- simple diversion structure

Location: -

a.   Longitude: 631606.58 E (UTM)

b.   Latitude: 816853.51 N (UTM)

c. Zone:  Bale  Woreda:  Gasara    PA:  Balo Habebe

Altitude (for swampy): 1700.8m a.s.l

Hydrology

a. Catchments area=25.3km2

b. Average annual rain fall=1036mm

c. Command area proposed= 65ha.

d. Max. Design flood= 30m3/s.

e. Minimum Flow=0.108m3/s

f. Spring eye level =1700.6 m

Engineering (simple diversion structure)

- Intake Capacity =0.108m3/s

- Project cost 5,467,630.85ET. Birr including Vat and 3% Super vision Cost.

- Useful life of the project: 25 years

- Total command area=65ha and 130HH beneficiaries

- Cost per hectare 84,117.40birr

# 1-EXECUTIVESUMMARY OF Balo-habebe SSIPFINANICIAL ANALAYSIS RESULT

There is no readily identifiable yield-increasing technology other than the improved seed-water-fertilizer approach. Thus irrigation of crops is a primary route to bring more land under cultivating and to increasing production yields from existing farm land.

The report includes Socio economic studies; Geology, Hydrology, Agronomy, soil, Engineering cost estimates & bill of quantities and financial analysis so as to see the viability & soundness of the project from financial point of view.

The total area to be developed is about 80 ha and would be beneficiaries are about 97 households.

The project cost is about Br 5604,350.58 Five million Six hundred Four Thousand Three hundred Fifty Birr and 38/100 cents for its construction.

The financial analysis of the project resulted in NPV of Br 73399.089 IRR 94% and BCR 2.09

### 1.1Balo Habebe SSIP-Socio-Economyandcommunity participation

1.1.1 INTRODUCTION

Balo-Habebe is one of the rural kebele found in gasara district Bale zone nearby gasara town the main district town that intimately established in Gasara Qola gorge and high valley and surrounded by High Mountain in all direction and the area has high potentials of natural spring water that is very important to use irrigation technology for existing community. The kebele was found at north direction of Gasara town about 7 km all weather road and about 65km far away from Robe town the main Zonal town of Bale. According to Balo- habebe peasant association managerial office data the kebele has 1232 male and 1120 female population. Among 2352 total population there are 344 male and 56 female are the households. From the total households currently there are only 97 households are primary beneficiaries.

Gasera district is one of the administrative territory of Bale zones which located North West Part of the zone.It has distance of 58 km from zonal capital city called Robe and 488km from center of the country and the region called FINFINE.

The district is bounded Agarfa in the west, Sinana in the south west, and Ginir in the southeast, Gololecha district in the East and Arsi zone in the North. It have a total area of 1113.5km2 which ranked the district 17th largest (the second Smallest district next to Dinsho district) among the zonal districts. The area of the district leads the district to have a share of 1.6 percent from the total area of the Zone

According to the 2002 projection of population, the populations of the district were84903& 87192 mail female respectively.

The settlement patterns of the district have a great variation because of physical factors (climate), availability of resources, fertility of soil, land form/topography, socio-economic situation and demographic factors of the district kebeles. All this characteristics can be analyzed by population density, which express the distribution of the district population per unit area.  The land use of Gasera district highly comprises of intensively cultivated and moderately cultivated land. The intensively cultivated land consists of peasant mixed farming.  It is where high population pressure on land has critically seen. The largest percentage of its total area is devoted to grain productions of mechanized farming abundantly found in north and northwestern part of the district. Non-mechanized cultivated land found in central, southern and Eastern part of the district.

The highest elevation of the district is lies between 2000-2500m that are found along northern part of the district that share boundary with Gololecha district, where as the lowest point of the district is roughly below 1100m   which found along Wabe Shabele River. The mean annual temperature of the district is 20oc.The lowest temperature is 15oc and highest is 25oc respectively. The mean annual rainfall is 819.25mmwhereas the lowest and highest rainfall is 735.9mm and 912.5mm respectively

Gasera district has two types of rainfall season. The first season is characterized by one long rainy season that extends from March to Augusts. But months with highest rainfall concentrations are June, July and Augusts. This type of rainfall covers all part of the district except small parts of the margin of southeast part of the district.

The second type of rainfall in the district is the one with spring and autumn maximum. It is influenced by equatorial a westerly and easterly wind that prevails over the area. This type of rainfall covers the margin of southeast part of the district.

Even though some variations are observed in seasonal distribution of months within the district, there are four seasons that are frequently occurred in the district. These are:

Winter- includes December, January and February months

Autumn- includes March, April and May months

Summer -includes June, July and August months

D. Spring- includes September, October and November

In the district the amount of temperature that receives is greatly modified by latitude and longitude extent. Based on altitude the districts are classified in to three-agro climatic zone. Those are High land, Semi High land and lowland consists 32.59%’ 37% and29.41respectevily.

# 1.1.2. OBJECTIVE OF SOCIO-ECONOMIC STUDY

The main objective of this socio- Economic study is the following

To increase crop production and productivity of farming activities in the proposed area

To highlight the existing socio-economic conditions

Contribute to the partial fulfilment of this irrigation project document and to give some suggestion about the implementation of the project

To give some socio-economic remarks about the project based on socio-economic analysis results.

# 1.1.3 The scope of the study

The study deals all demographic features of the project area examining the existing economic activities and livelihood bases, existing basic social infrastructure and social service, and identify major economic and social problems development priority of the project area. Next the study attempt to analysis the positive and the negative impact of the project area In general the reports tried to verify underling situation, population demographic, features, economic and social condition potential problem priorities, opportunity and challenges

Generally the scope of the study included the following

Asses and identify the socio economic problems Asses and identify existing socio economic problems, constraints and social mitigation measures

Asses gender relations and gender disaggregated data with respect to land access, control over resource, capital and income.

Assess household economy

Asses conflicting and or competing demand if any

Asses unlikely social impacts of the projects

Asses and evaluate farmers attitude toward the project and

# 1.1.4  METHODOLOGY

The socio economic study report was prepared based on field assessment, site visits as well as primary and secondary data collected at zone, woreda and kebele levels.

Generally data collection method can be classified in to the following method

Group discussion

Community interviews

Focus group interviews

Key informant interviews

key observant interviews

filed visits

Are the main methodology of data collection system in and the around the proposed small scale irrigation project.

Existing Situation Assessment

In the proposed small irrigation project the farming system is traditional farming which plough by oxen and using man power. Although during the harvesting crops are collected by man power cutting by sickles. The existing crops produced in the project proposed project area are like sweet potato, sugar cane, Banana, mango, orange, chat, papaya, Teff, wheat and other similar crop. So agricultural is the main economic activity of the existing society. The main income for the farming community is the income get from the farming land and from their live stock products such as milk, better, egg and other animal products, the majority of farmers are strong struggle with nature to change their life from lower economic income to better life this is what the crew see during our felid study time and there is food insecurity in and around the project are.

# 1.1.5.LOCATION AND PHYSICAL SOCIO ECONOMIC CHARACTERISTIC

Balo-Habebe is one the rural kebele found in.gasara district nearby gasara town the main district town that intimately established in gasara Qola gorge and surrounded by High Mountain in all direction and the area has high potentials of natural spring water that is very important to use irrigation technology for existing community. The kebele was found at north direction of Gasara town around 5-7 km all weather road and about 65km far away from Robe town the main Zonal town of Bale. According to Balo- habebe peasant association managerial office the kebele has 1232 male and 1120 female population. Among 2352 total population there are 344 male and 56 female are the households. From the total households currently there are only 97 households are primary beneficiaries.

1.1.6. Demographic characteristic

From the given community of proposed project area the one that necessary to determine the category of community according to age ,sex and religion composition were applied to know how many labor force are existing in proposed irrigation project and how many of them are unemployment’s and the same as how many peoples are participating during implementation and after project implemented, therefore  the categories this type of splitting community has its own purpose .Generally the proposed irrigation project area was determine as follows

1.1.7. Family size

Many factors determine a given community family size and demographic structures. Among these includes religion, marriage structure, household economic status, educational level as well as economic and social importance for children. Household’s family size is large in most rural Ethiopia in general and same is at proposed irrigation project area.

In terms of religion the community at project kebele and command area belongs to Muslim and Christian religion followers. With regards to marriage structures, polygamy marriage structures widely practiced where one male couple with more than one female as family counterpart. As a result, observed high number of children per household head.

Based on the assessment study minimum and maximum family size ranges from 2 to 10 number of family size and six average family sizes, this indicates high family size compared to the national average of five family sizes that mainly attributed due to cultural, religious, low education level and low awareness in family planning.

The high average household size of the study population appears to be partly indicative of the fact that, they are still characterized by the traditional extended and nuclear family system, whereby women were more exposed to early marriage at their young age and the community’s perception to have more children.

1.18. Family Planning Awareness and Practice

IN proposed project area Although, one health extension worker assigned at project kebele and family planning is not yet practically defused and community attitude is not changed as needed mainly due to religion view in most Muslims households, as observed during discussion, male groups strongly resist family planning in the context of religion and importance attached to have more number of children as economic asset. However, majority of women well understood the burden and problem with having more children and   changed their attitude.

1.1.9. Settlement pattern and community migration/movement

Regarding of settlement pattern of a community is very important in project identification and planning processes. The population density and settlement pattern also has significant implication on the socioeconomic impacts of Irrigation project particularly with activities related to displacement effects and mitigation measures.

Specifically the study project area was village style and semi scattered distribution and there is no displacement for the future due to the implementation of the irrigation project on the study area but before the study of irrigation project a few households are displaced from their original home due to the construction of road pass through belo-habebe village before two years..

1.1.10. Population projection

Population projection is one of the instruments to measure the future generation of one territory /environment after limit duration.

According to Oromia human population projection applying formula after ten years the total population of the project area were3345 population and these implies demand for food were increased after few period.

5.6. Population Density

The combined effect of the diversity of relief, climate, soil fertility and historical events has brought variation in the density and distribution of people. On the other hand, level of infrastructure development, Urbanization and resource endowment are factors that play a vital role on the distribution of the people.

The average density of population for proposed irrigation area was 67.75km2/person it implies there is high population growth in the area

1.1.10. House status

The features ofhouse was based on economic situation it means the one who has better income generation was live better life and owning better house and better housing equipments. Regarding to the specific proposed project area the majority of house are constructed from local materials such as thatch and wood except a few houses that made from corrugated iron sheet which has separating kitchen from living house and toilet. According data obtain from field study survey made estimate the number thatched constructed houses are 75%and the only 25% are made from corrugated iron sheet and there is no house facility such as electricity, potable water modern house equipment are the incomplete requirement for the study area of the project of house hold.

1.1.12 Ethnic composition

The communities in project area are two ethnic compositions those are Oromo and Amara ethnic group and both are respect each other without any contradiction and living in assisting each other’s and sharing cultural habits and communicating in both language Afaan Oromo and Amharic language.

5.8. Religion

Regarding to the religious pattern, of the households residing in and around the project area are belong to followers of the Muslim and Christian religion the Muslims, have two common

Mosque celebrating that their religions on Friday in common while the Christians have one church that celebrating their religion on sun day.

5.8. Social and Cultural Features

With regards to social and cultural features the existing community has their own social and cultural features such as working habit, feeding habit, clothing /wearing habit are express social and cultural features of one community, regarding to food intake their known food is porridge and Injera usually prepared from the flour of mixing teff and wheat.  as far as working habit the  community were working together during crop harvesting time and during facing of difficulties to help each other’s in form of traditional institution such as Idir, Iqub and debo(local word)

In terms of religion some Muslim religion followers a man can marry more than one   wives (Polygamy) depending on his economic status mainly livestock possession like much number of cattle and farming land.  Wealth and economic status, mainly livestock is one of the determinant factors to engage where an old aged man couple with teenager girl as far as he own and provides the required wealth as a prestige. These days such habit is completely improved and changed to according to female interest or willingness to marry young boy who have equity age and based on common agreement to marry or divorce.

1.2 ECONOMIC ACTIVITIES AND LIVELIHOOD BASIS

1.2.1 Economic and livelihood base

Economic status of population is classified in to three main groups namely, (0-14) Young (children) age group, (15-64) Adult (working) age group and (>64) old age group. The age structure of population tells the economic status of population of a country.

In balo habebe peasant association the majority of community categorized under adult age/working age group. Therefore the economic status of population in this area was based on the activity of man that who struggle with nature to win his life. In general, agriculture is the economic and livelihood bases of the community and crop production contributes the major share and mainstay of the household economy at project area. Crop production contributes the major livelihood basis and food source

1.2.2. Estimated income and income source

As estimated about 100% of the household in proposed project area economy depends on mixing farm activity  such as  crop production and rearing animal .in proposed irrigation project area there is not seen other income activities like, petty trade and other income generating source and off farm activities. From crop type fruit products are the main cash crop of the given community.

1.2.3AGRICULTURE production and Land use

Agriculture is the back bone of the Economy of the district. It provides means of occupation for almost all population of the country

The Constitution of the Federal Democratic Republic of Ethiopia (1995) on article 40.3 states the public ownership of both rural and urban land as well as all natural resources. It further states that land is common property of the Ethiopian people and cannot be subject to sale or to other means of exchange.

However the distribution of land for land less youth was practiced in the last few years land holding entitlement is mostly from family inherited was known in our country this kind of distribution was common for country side. The majority of lands in proposed irrigation project area are used for cultivation purpose, the other main purpose of land categorized was for livestock grazing, and this sharing category is very less in amount the community feed their livestock crop residual and other alternatives. In general the following table indicate land use pattern

1.2.4. Land use pattern in Balo-habebe PA and G

| S.NO | Type of land distribution | Hectare |
| --- | --- | --- |
| 1 | Cultivated Land | 56.5 |
| 2 | Bush& shrub | 130 |
| 3 | Grass Land | 80.5 |
| 4 | Forest Land | 750 |
| 5 | Valley& Hill | 25 |
| 6 | Housing | - |
| 7 | Others | 65 |
| 8 | Total |  |

Source;- Balo-habebe PA office

# 1.3. Land Use and Land Cover

Land utilization is one of the principal ways to measure the performance of Agricultural production.  Land is one of the most important natural resources provided by nature for Man's aid, utilizing without any effect increase the output generate from it. This is true for Gasera district economy where the Agricultural sector is the main stay to the livelihood of the majority people.

In  2005 & 2006 the total area of Gasera district  is 1113.5km2(111,350ha.) out of this  land  47.44%  is under crop production,  3.3% is  under grazing land,  1.25% is covered by  forest   10 % is covered by barren/degraded area,  16 % is arable land and  21.9 %is covered by others( such as river ,mountains,   construction etc respectively.

Table 2. Land use and Land cover of Gasera District

| Land Resource | 2002 | Share (%) | 2003 | Share (%) | 2004 | Share (%) | 2005 | Share (%) | 2006 | Share (%) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Land Under crop | 52949.6 | 47.55 | 53001 | 47.44 | 53001 | 47.44 | 53001 | 47.44 | 53001 | 47.44 |
| Grazing Land | 3670 | 3.3 | 3670 | 3.3 | 3670 | 3.3 | 3670 | 3.3 | 3670 | 3.3 |
| Forest | 1392 | 1.25 | 1392 | 1.25 | 1392 | 1.25 | 1392 | 1.25 | 1392 | 1.25 |
| Swampy Marshy | - | - |  |  |  |  |  |  |  |  |
| Degraded Area | 11073 | 10 | 11073 | 10 | 11073 | 10 | 11073 | 10 | 11073 | 10 |
| Arable Land | 17794 | 16 | 17794 | 16 | 17794 | 16 | 17794 | 16 | 17794 | 16 |
| Others | 24471.4 | 21.9 | 24471.4 | 21.9 | 24471.4 | 21.9 | 24471.4 | 21.9 | 24471.4 | 21.9 |
| Total Areas of the district(Hectares) | 111350 |  | 113404.4 | 57.194 | 113411.4 | 54.139 | 113411.4 | 54.139 | 113411.4 | 54.139 |

Source;-Gesara woreda Environmental protection and Land Administration office

Land holding size and beneficiaries

Land holding size determines production and productivity in agriculture and land access and important to give overview of land access and land holding size at project area. According to the assessment result

Land holding size ranges minimum of 0.5 hectare to maximum of 2 hectare and the majority lies at the range of 0.5 and average land holding size is expected below two hectare and the average is 0.8 hectare per household.

Majority of the household (70%) land holding size is 0.5-1 hectare and about 30% of household land holding size ranges 1 -2 hectare and the average 0.8 hectare.  Moreover, except the irrigable command area, adjoining land topography is covered semi plain near the mountain land and suitable for irrigation livelihood and crop production largely depend on intensive cultivation of the river side.   As a result, land access for the youth limited and some youth have no access to farming plots on the irrigable area.

Farm access and possession of land is mainly through family inheritance and there was no land redistribution.

1.3.1. Rain fed crop production

Major economic base of the project area derived from crop production and animal rearing. Sweet potato Wheat, maize, and teff and chat are dominant crops to the project area. As farmers indicate among other crops that can adopt to grow includes, sugarcane, bananas are also widely cultivated and the other crops fruit and vegetable are also cultivated in proposed irrigation project area. Wheat and maize teff and sweet potato are used mainly for food consumption whereas Banana , sugar cane and chat  are generating income source to meet family financial needs.

In general, at existing condition, maize and teff are the most important crops to meet the family food demand and the intended cropping patterns with development of the irrigation project require giving priority for local food crops as well as high market value crops that meet local market demand to generate household income. Farming system is traditional farming system that is plowing by man power and oxen.

1.3.1.2. Irrigated crops

In propose irrigation project area the community has high experience about traditional irrigation practice but unable to use efficiently and effectively due to lack of knowledge and economically back ground to change the traditional trend to modernized trend. Currently, the community cultivated irrigated crops such As Banana, chat and sugarcane is widely produced and the community has high interested about irrigation crops.

1.3.1.3. Agricultural input supply and accessibility

1.3.1.4. Input supply and utilization

In the proposed irrigation project area using  Modern fertilize is not much known comparing with other villages, according to data obtain from development agent more than 75% farmers are not use fertilizer due to back ward of attitude.

7.1.5.  Development Agent

Development   Agent will be more effective to understand the social and cultural back ground of farmers with whom he/she works. Extension service workers disseminate information from the source to end user. It used as a communication techniques to transfer technology to ultimate beneficiary and also dealing with managing agricultural knowledge and information. In proposed irrigation project area there is only one development agent was assigned by government and working in project area.

The contribution of development agent to ward Agricultural Development is significant. By taking in to consideration Gasera district have the following development Agent during the past consecutive year.

Table3. Gasera Agricultural Extension Service Coverage (2002-2006)

| Year | Number Of Development Agent | Number Of Farmer Served | DA/FS |
| --- | --- | --- | --- |
| 2002 | 71 | 11,393 | 1:160 |
| 2003 | 71 | 11173 | 1:157 |
| 2004 | 71 | 11173 | 1:157 |
| 2005 | 59 | 11172 | 1;189 |
| 2006 | 65 | 11172 | 1;172 |

Source –Gasera district Agricultural and Rural Development

13.1.6. Constraint of Agriculture

Since Agriculture in the District is characterized by traditional, more or less it is dominated by nature. Agricultural out puts and productivity is constrained by several factors. Among this factors;

Resource and Environmental problems; Rapid population pressure creates fragmentation of agricultural land because the share of land among families is getting smaller and smaller when the member of families married. The decreasing of farmland from time to time, which in turn creates irrational forest resource by deforestation to get arable and grazing land and as a result massive erosion, will happen.

Socio-economic problem; Farmers' precipitation is that increasing output is only possible through increasing arable land because they have limited knowledge about modern technology, cost returns and risks. Poor economic status of the farmers that is not viable to invest on Agricultural inputs because of lack of credit facility.

Technological problem; in the district peasants depend on traditional tools, low application of modern technologies. The accesses of credit facility in peasant area are low because they lack personal knowledge about how they use credit. Again this problem leads for low utilization of agricultural technology, i.e. fertilizers and improved seeds In addition farming system in district is characterized by oxen .As a result, Yields are low. They follow inappropriate farming method too.

1.3. 1.7 Agricultural extension

Agricultural extension service is one of the essential back bones of the farmers in all direction for sustainable development of agricultural   productivity and related to increasing production as well as fulfill skill gap on agricultural extension service in Gesara district is currently well improvement regarding to using in extension service are increased from time to time. Farmers Training and extension in most case considered as an integral parts of irrigation projects Therefore, training and extension should be considered as a strategy to introduce skills and knowledge and other appropriate technologies to achieve intended development objectives of the project. Technical training needs to be given for both women and men groups as well as implementing partner staffs, i.e. development agents and other technical experts at district office. Both women and men need to be included in extension and training particularly irrigation management (water management, Operation and management, specifically the proposed project area agricultural extension service is observed on rain crop production therefore the existing a such a situation made pre condition for proposed irrigation project implementation

# 1.4. LIVESTOCK DEVELOPMENT

Livestock production plays a significant role that contributes the second major livelihood source next to crop production. Livestock has three advantage to existing communities such as for farming, transporting and for food consumption. In general   the major sources of farm power, for transportation especially to transport agricultural products from home to market and from market to home, beside to transportation it play significant role for successful of irrigation implementation to prepared land preparation and additional livelihood for a given community in order to give animal products such as milk, better, egg, hide and meat. The existing livestock management is very poor that is not well developed it is traditional way rearing animal and traditional feeding system or free grazing on open area.  In general the following table indicates the number and the type of livestock in proposed area and distri

|  | Cattle type | At kebele level | At district level | Remark |  |
| --- | --- | --- | --- | --- | --- |
| 1 | Cattle | 40200 | 120800 |  |  |
|  | Oxen |  | - |  |  |
| 2 | Cows |  | - |  |  |
| 3 | Heifer |  | - |  |  |
| 4 | Bull |  | - |  |  |
| 5 | Sheep | 1070 | - |  |  |
| 6 | Goat | 1680 | 2466 |  |  |
| 7 | Donkey | 320 | 62 |  |  |
| 8 | Horse | - | 7040 |  |  |
| 9 | Mule | 11 | 1240 |  |  |
| 10 | Poultry |  |  |  |  |
|  | Traditional | 750 | 46560 |  |  |
|  | modern | - |  |  |  |
| 11 | Beehive | 190 | 1470 | Traditional |  |
|  | modern | - | 30 |  |  |

Sourec;- Gasera district Livestock development agency &balo habebePA office.

1.4.1. Livestock Disease

Diseases have numerous negative impacts on productivity of herds' i.e. death of animals, loss of weights, slow down growth, poor fertility performance, decrease in physical power and the like. There have been many ways of fighting against disease and among these, Vaccination and treatments are the major ones. The availability of data on prevalence of disease is also very important to set up strategies that can assist in preventing and controlling disease, by and large in improving veterinary service of the district.

There are different diseases that affect livestock production in Gasera district. The major disease which affect bovine, ovine, equine, poultry and bees are listed on the following tables

Table  4.   Major Animal's disease.

|  | Type of Animals | Type of Disease 2004 | Type of Disease2005 | Type of Disease2006 |
| --- | --- | --- | --- | --- |
| 1 | Bovine (cattle) | Anthrax | Anthrax | Anthrax |
|  |  | Black leg | Black leg | Black leg |
|  |  | Pasteurollosis | Pasteurollosis | pasteurollosis |
|  |  | Foot and Mouth disease | Foot and Mouth disease | Foot and Mouth disease |
|  |  | Coccidian | Coccidian | Coccidian |
|  |  | Tetanus | Tetanus | Tetanus |
|  |  | Internal parasite | Internal parasite | Internal parasite |
|  |  | External parasite | External parasite | External parasite |
| 2 | Ovine (Sheep, Goat) | Pasteurollosis | Pasteurollosis | Pasteurollosis |
|  |  | Internal parasite | Internal parasite | postinernal parasite |
|  |  | External parasite | External parasite | External parasite |
| 3 | Equine (Horse, Donkey, Mule) | Tympengitis | Vicerativelegs | lynpenzitis |
|  |  | African Horse Sickness | African Horse Sickness | African Horse Sickness |
|  |  | Dourine | Dourine | Dourine |
|  |  | Tetanus | Tetanus | Tetanus |
|  |  | Internal parasite | Internal parasite | Internal parasite |
|  |  | External parasite | External parasite | External parasite |
| 4 | Poultry | Coccidian | Coccidian | Coccidian |
|  |  | Mew castle | Mew castle | Mew castle |
|  |  | Fowcholera | Fowcholera | Fowcholera |
| 5 | Bees | Norseman | Norseman | Norseman |
|  |  | Accra | Accra | Accra |
|  |  | Amoeba | Amoeba | Amoeba |

Source- Gasera district  Livestock dev’t and health Agency  Office

# 1.5. Urban- rural market linkage

Market is an area in which one or more sellers of given products /services and their close substitutes exchange with and compute to the partner group of buyers. Originally the term market stood for the place where buyers and sellers are gathered to change their goods such as village square.   As to the case of the project area there is no local market in the study project area. the existing community was  sold their agricultural product and buying  industrial goods  from the nearest  market called  Gasera market, Gasera market is the main market for district population two days within a week Wednesday and Thursday ,the  other markets exists in the distance of 60km radius are Robe and salu market which is promising and colorful market. in case market price the farmers sold in cheap price when they are sold in local market and sold in better price in Robe and other markets for instance one stick of sugarcane sold by 10-12birr in local area but in Robe it is about 15-20birr, therefore  the implementation of proposed project were increasing urban –rural market linkage

Here below alternative lists of markets in and around the area with their distance and market days.

| 1 | Gasera | 7km | Thursday& Saturday |  |
| --- | --- | --- | --- | --- |
| 2 | Robe | 58km | Thursday |  |
| 3 | Danbal | 25km | Monday |  |
| 4 | Salu | 20 | Sunday |  |

Source; - Assessment during survey study

1.5.1. Agricultural output Marketing.

The area is suitable for production of Banana, sugar cane, maize, wheat, chat and mango   is produced and transported to gasera market for the consumers and retailers.

Table5: Market price trend at project area (Birr/Qt)

| Crop type | price | Remark |  |
| --- | --- | --- | --- |
| Maize | 750 |  |  |
| Sweet potato | 300 |  |  |
| Wheat | 850 |  |  |
| Chat | 120 | Zorba |  |
| Banana | 11 | kg |  |
| Sugar cane | 15 | In number |  |
| Honey | 90 | kg |  |
| Teff | 1000 | Qt |  |

Source;-   survey  made during filed study(market price)

Transport and communication services are fundamental factors in determining development endeavors being undertaken in a given locality. Since they provide services to other sectors of the economy, it would be impossible to conceive of social and economic development in the absences of adequate transport and communication infrastructure and services.  The existing basic infrastructure like telephone service, electricity is health post, all weather road are one of the pre condition of contribution for the enhancement market formation improvement and extension of basic infrastructures

1.5.1.2. Agricultural Cooperatives

Agricultural cooperatives primarily important to meet farmers priority agricultural needs of input supply and market arrangement of their agricultural product output. In woreda there is an agricultural development cooperative office that facilitate to originating may type of cooperative organization in kebele and at woreda level. For instance the following table below indicates the number and the type of cooperatives.

Table C.3.1:  Number of cooperatives, members and their capital in the year 2013/2014

| S.Number | Type of cooperatives |  | Member |  |  | Capital(Birr) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | No of cooperative | Male | Female | Total |  | Fixed capital in birr | Operational capital |  | Total |  |  |  |
| 1 2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Primary cooperative | 61 | 10081 | 1446 | 11527 |  | 18389711 | 1903108.02 |  | 20292819.02 |  |  |  |
|  | Multi-Multi Purpose | 23 | 8866 | 1184 | 10050 |  | 1818350.24 | 12059014.31 |  | 13877364.55 |  |  |  |
|  | Forest Development | 2 | 198 | 39 | 237 |  | 12452 | 865444 |  | 877896 |  |  |  |
|  | Mining | 1 | 12 | 0 | 12 |  | 0 | 4457.55 |  | 4457.55 |  |  |  |
|  | Electricity | 2 | 163 | 35 | 198 |  | 0 | 37251.9 |  | 37251.9 |  |  |  |
|  | Irrigation | 3 | 71 | 10 | 81 |  | 20000 | 27147 |  | 47142 |  |  |  |
|  | Consumers | 4 | 415 | 111 | 526 |  | 92180 | 411102 |  | 503282 |  |  |  |
|  | Saving & Credit | 21 | 1553 | 1871 | 3424 |  | 140528 | 1782525.76 |  | 1923053.76 |  |  |  |
|  | Others | 5 | 946 | 223 | 1169 |  | 207999.36 | 1461165.49 |  | 1669164.25 |  |  |  |

Source;-Gasera District cooperative Agency.

1.5.1.3. Agricultural credit and saving

One main constraint in many rural areas is the limited access to credit and agricultural input. Expected result of increased agriculture production will realize only with sufficient access for credit and agricultural input for both men and women. woreda cooperative office  is operating in the providing credit for vulnerable people to build their asset through business activities. The project area is also part of the target area of the program where the vulnerable households are benefiting from the credit facility. . The purpose and use of the credit service should be for various agricultural activities and input purchase like ox, improved seeds, grain trading, and animal husbandry and needs to introduce and create accessibility for credit service. In general, access to agricultural credit and saving for both men and female is low and needs to be promoted for effective and efficient benefit from the irrigation project.

The purpose and use of the credit service should be for various agricultural activities and input purchase like ox, improved seeds, grain trading, and animal husbandry and needs to introduce and create accessibility for credit service. In general, access to agricultural credit and saving for both men and female is low and needs to be promoted for effective and efficient benefit from the irrigation project.

# 1.6. BASIC SOCIAL SERVICE AND INFRASTRUCTURE

The development of education, health and water supply interventions are important investment sectors for the attainment of social service development in a given district/area. .

1.6.1. Education and school infrastructures

Education plays a crucial role in the process of social and economic transformation. In its broader social objectives of increasing the stock of knowledge and outlook of the population, raising output and productivity, education stands out as a key factor for poverty reduction.  It is also argued that, when farmers are able to acquire basic education, it becomes easy for them to adopt new technology. Cognizant of this fact, education service at project area assessed.

In the project area there is one first cycle 1-8 established on1978 by Government found and community participation that consists /enrolled 164 male student 155 female students. Students those complete grades 8 are forced to travel 7km to continue the next grade in gasrea town. The number of teachers in school was 6 female and 2 males all teachers are diploma graduated and assigned by government. In case of school facility there is no potable water, Electricity and other import facility.

1.6.1.2Health and health related disease

Regarding to health status health has direct relationship with development in that it can negatively affect on the human element or labor productivity. Currently there is one health post in Balo-habebe village established in1998 by government budget which the existing communities were getting service. The post office has no fully facility such as water, light power and all necessary equipments needs for one health post and the other  post office problem is the awareness of people having on family planning is poor contacted or related  with religion but few women are using family planning in hidden way according health post office  only 20% women are the beneficiaries  of post office .According to health extension report the main service of this post office is the follow up of delivery service. The community was travel 7km to get health service to reach gasera health center and 65km for referral case Robe hospital. The health center has the following professional staff members

10.1.4. Health

1.6.1.5. Health Institution

Health institution includes Hospital, Health centers, and clinic and health post. Hospitals, health centers and health stations are owned and financed mainly by government, while community manages health post, which are the smallest health institution.

Health institutions are unevenly distributed across the districts kebeles. The existing is concentrated in relative urban area but currently health posts are expanding in rural areas from time to time. In 2003 & 2004 the Health Centers, Health Posts & Drug Vendors increases from 3-4, 17-18 & 1-2 respectively. Table 34.Number of Health Facilities (Government)  (2000-2006)

| Health Facilities | 2002 | 2003 | 2004 | 2005 | 2006 |
| --- | --- | --- | --- | --- | --- |
| Hospital | 0 | 0 | 0 | 0 | 0 |
| Health Centers | 3 | 3 | 4 | 5 | 5 |
| Health Post | 17 | 17 | 18 | 16 | 16 |
| Clinics | 2 | 0 | 0 | 0 | 0 |
| Pharmacies | 1 | 0 | 0 | 0 | 0 |
| Drug Vendors | 0 | 1 | 2 | 0 | 0 |

Source-Gasera district Health office

1.6.1.6 Health Personnel

Qualified and sufficient health personnel should be available in order to provide appropriate health service for society. Health personnel like doctors, nurses, health assistants, are the most important factors for the improvement of the health service

The situation of Medical Personnel in Gasera district over the past two years shows a relative increasing trend except Community health agents that decreases from 43-40.

Table 5..Numbers of Medical Personnel (Government)  (2000-2004)

| Medical Personnel | 2002 | 2003 | 2004 | 2005 | 2006 |
| --- | --- | --- | --- | --- | --- |
| Health Officers | 3 | 3 | 3 | 3 | 4 |
| Nurse | 26 | 29 | 35 | 36 | 38 |
| Health Assistant | 1 | 1 | 1 | 1 | 1 |
| Laboratory Technicians | 3 | 4 | 5 | 4 | 4 |
| X-ray Technicians | 0 | 0 | 0 | 0 | 0 |
| Sanitarians | 3 | 2 | 3 | 2 | 3 |
| Pharmacy technician | 2 | 3 | 5 | 5 | 6 |
| Community health agents | 43 | 43 | 40 | 40 | 41 |

Source-Gasera district Health office

In the district for the continuous four years there is complied data about selected ten top diseases were available as indicated table below.

|  | 2002 |  |  |  | 2003 |  |  |  | 2004 |  |  | 2005 |  |  |  | 2006 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Type of disease |  | Treated population |  | Type of disease |  | Treated population |  | Type of disease | Treated population |  | Type of disease |  | Treated population |  | Type of disease | Treated population |
| 1 | Pneumonia |  | 2636 |  | Risparentar infection |  | 3823 |  | Pneumonia | 2432 |  | acute upper respiratory infection |  | 960 |  | acute upper respiratory infection | 2405 |
| 2 | Parasite |  | 1672 |  | Parasites |  | 826 |  | Parasite | 1193 |  | Pneumonia |  | 762 |  | Pneumonia | 1869 |
| 3 | Gastritis |  | 1306 |  | Gastritis |  | 1407 |  | Gastritis | 990 |  | trauma |  | 706 |  | trauma | 1817 |
| 4 | Diarrhea |  | 1286 |  | Diarrhea |  | 1616 |  | Diarrhea | 1030 |  | Diarrhea |  | 490 |  | Diarrhea | 1469 |
| 5 | Rheumatism |  | 1042 |  | Rheumatism |  | 802 |  | Rheumatism | 731 |  | musculoskeletal and connective tissue |  | 454 |  | musculoskeletal and connective tissue | 969 |
| 6 | Tonsilities |  | 812 |  | HCC menthiasis |  | 1557 |  | Tonsillitis | 580 |  | urinary track infection |  | 448 |  | urinary track infection | 917 |
| 7 | Disease of Skin |  | 801 |  | Infection and substa |  | 1113 |  | Disease of Skin | 496 |  | dyspepsia |  | 438 |  | dyspepsia | 843 |  |
| 8 | Anemia | 478 |  | Anemia |  | 703 |  | Helminthiasis |  |  | 426 |  | helminthiasis |  | 802 |  |  |  |
| 9 | Eye Disease |  | 539 |  | Eye Disease |  | 454 |  | Eye Disease | 527 |  | infections of the skin and subpotanes tissue |  | 368 |  | infections of the skin and subpotanes tissue | 762 |  |
| 10 | Disease of Digestive System |  | 522 |  | Disease of Digestive System |  | 991 |  | Disease of Digestive System | 907 |  | acute febrile illness |  | 356 |  | acute febrile illness |  | 740 |

Source;- Gasera district health office

1.6.2.Energy Supply

Energy plays a major role in the development and progress of the economic as well as social activity of any society. Hence, the availability of adequate and cheap alternative energy sources is a paramount importance for the development of small, medium and large-scale industries, and strengthening back ward and forward linkages with other sector of the economy

use of modern energy source, like fuel and electricity for domestic consumption also reduce the use charcoal, which in turn reduce deforestation.

16.2.1. Source of Energy Supply

The source of energy supply is classified in to traditional and modern energy source. Traditional energy source are obtained from plant (wood, charcoal, crop residue etc) and animal dung. These traditional source accounts for more than 95 percent of the total energy source being consumed in proposed project area and in the district at whole.

Regarding to specific study project area the majority of a given community uses fire wood for cocking food and for lighting addition to kerosene

In general for domestic energy supply come from forest resource in the form of fuel wood and charcoal, and the supply is increasing from time to time due to rapid population growth.

In urban areas of the  district fire wood, charcoal, kerosene, electricity and crop residue are  the major source of energy, where as in rural area fire wood, dung, crop residue, charcoal and to some extent kerosene is consumed as energy source.

1.6.2.2. Potable Water supply coverage

Adequate water supply is not only vital for the survival of human being, but also contribute importantly both to the prevention of diseases and improvement of the health of people. Access to quality pure and adequate water supply is also used as an indicator of the socio-economic of any region.

According to the 2003 & 2004 district water resource office report, out of the district population, about 48 & 52 percent of rural population has an access to potable water respectively. On the other hand, about 52 & 48 percent of rural population of the district has no access to potable water respectively. They depend upon traditional and often contained water source like unprotected well/spring, river or pond. As a result, the health and well being of the population is at risk.

In case of study proposed irrigation project area according to PA office data obtain more than 20 spring waters are available including the study spring (tebela) for drinking human and livestock and also for traditional irrigation.

Table 7   Potable water coverage (2002-2006)

| Year | URBAN |  |  | RURAL |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Total Population | Population Supplied With Potable Water | Percentage Of Population Supplied With Potable Water | Total Population | Population Supplied With Potable Water | Percentage Of Population Supplied With Potable Water |
| 2002 | 5237 | 5237 | 100 | 80223 | 32283 | 40.24 |
| 2003 | 14965 | 13500 | 90 | 73056 | 41882 | 48 |
| 2004 | 14965 | 14000 | 93 | 73056 | 42382 | 52 |
| 2005 | 15100 | 1500 | 80 | 74000 | 43100 | 57.8 |
| 2006 | 15294 | 1500 | 80 | 74000 | 43194 | 65.08 |

# 1.7.GENDER ISSUES

In most cases society used give privileges to males while disfavouring females. for instance the right to leadership, the right to property ownership and opportunity of education are given mainly to men attitude was seen for several years but know gender issues is the one which is get priority under equality of men and women in all opportunity. in case of proposed irrigation project area in all manner the equality of gender issues is approved by respecting of equality of gender issue. Gender is seen as a social construct that. It focuses not only on women but also looks at ways in which men and women interact with each other, the gendered nature of economic and social activities role, relations and control over resource.

With regards to gender relation, women involve in all household chores water fetching, firewood collecting, cooking, marketing and also other farm activities as required.

These day the legally acknowledged entitlement of women to own land equally with men are respected in the land reform proclamation, traditionally, they are neither entitled to inherit the wealth of their father, nor to take their share during divorce but know the situation was completely changed and improved well. In general women and men have equal right on decision making on and around study project area.

# Conclusion and Recommendation

The command area estimated 30 hectare and expected beneficiaries of about 120 households. The proposed irrigation project is located in kola agro ecology and about 40-50% of the population at project area is in food shortage/deficits. The economic and livelihood source is derived from crop production and livestock and generally considered as Crop production, although, considered major economic activities.

Traditional irrigation exist since it has dominated rain fed cultivation the majority farm lands in the project area were covered by traditional irrigation  and  long years,  but inefficient utilization of water resource, land and labour and economic loss/wastage observed  due to limited knowledge and skills without disregarding the long years traditional practice knowledge.

The challenges facing the  irrigation project area in particular includes climate change, shortage of rainfall, limited water availability coupled with ever increasing and unprecedented family size and population growth...etc.

The socio-economic problem of the study area is multidimensional and calls for a concerted efforts involving all development actors. Based on the discussions made and the outstanding issues discussed in this report, some plausible solutions are forwarded with regard to major socio-economic challenges facing the project area.

Generally the proposed project will have greater contribution for increasing social welfare through increasing productivity and enhancing income level of the households.

# 1.7. COMMUNITY PARTICIPATION

Community participation in planning, implementation and management is believed to ensure sustainable development. Accordingly, participatory discussion made with community embracing the marginalized populations and discussed people attitude about the project and how to play a central role in implementation and management of the project. As noted  on the discussion  community participation has been high  in a forestation, soil and water conservation, construction of social infrastructures such as schools, health facilities, access road constriction and high labor mobilization in traditional  irrigation canal development and clearing as witnessed by community on the discussion meeting.

Regarding the proposed irrigation project, communities are willing to contribute their share in the form of free labor contribution and local material supply.

The importance and contribution of irrigation provided lesson from the existing traditional irrigation and communities have strong desire in the development of the irrigation project. As viewed during nation of spring water as whole

Moreover, communities have been asked about land loss or relocation if any arise and expressed their full willingness to accept   land loss due to canal or other   if any arises due to the project and agreed to solve at community level.

Institutional arrangement

1.7.1. NEED FOR INSTITUTIONAL ASSESSMENT

The command area for  and  irrigation Project is anticipated to adopt gravity irrigation system, whereby the water flows divided into main canals, secondary canals and  as well as farm components in left side of the spring water The net irrigable land in the command area is envisaged to be about 30 ha

After construction of the irrigation project formal handing over of the system shall be made to the intended beneficiary community that further demands operation and management of the scheme. Hence, the scheme requires the formal community organization that will take full responsibility of managing the irrigation project. It’s required to review scope of available technical support, community organizations, capacity building and training, as well as existing status of irrigation management practices, so as to obtain views and experiences in the project area irrigation agriculture.

The organization and management aims to identify the institutions at the local and or community level that will be established and formally organized to take over the responsibilities of managing and operation of the proposed irrigation Project. It further aims to enhance the local implementation and management capacities that are expected to result from implementation of the proposed irrigation system towards ensuring the sustainable management of the project.

Accordingly, the future community organizations, the legal framework, promotion & institutional options, irrigation water use and setups of envisaged schemes, as well as the by-laws, the extent of required technical support from concerned sector offices or involved organizations and existing practices of irrigation management and related institutional issues are described in section that follows.

1.7.2 EXISTING ORGANIZATION AND MANAGEMENT

1.7.2.1. Organization and Management

The existing traditional irrigation canals are expected beneficiaries of irrigation scheme are estimated to be 120 households.

Existing Community Organizations

The community was renews the existing irrigation committees during our field study according to the following table list.

NO; Name; Age; Education Level; Job assignment. 1; Ahmed Kadir; 38; 6; Chairperson. 2; Abraham Mulugeta; 24; 10+4; Vice chairman. 3; Muhammad husein Taha; 28; 10; Secretary. 4; Fatuma Jemal; 27; 7; Cashier. 5; Sintayow Taye; 24; 4; Accountant. 6; Worko Jower; 36; 6; Store keeper. 7; Abdurab Husena; 24; 10; Member.

Source;- From community meeting.

The project scheme is anticipated to serve kebele found in the proposed command area and irrigation infrastructure is planned to be diversion irrigation system for the people living in the aforementioned command area. The estimated beneficiaries of the intended irrigation project are about 120 households that organized in one irrigation association having management committee which lead the community

The institutional assessment that the beneficiaries in the proposed command area have long years traditional and informal community organization for irrigation management. It is traditionally and conventionally managed by traditional institutional management system whereby seven people appointed by the community on each canal to distribute water. Each season

Community provides free labor contribution for canal clearing and other work related with irrigation system and have traditional bylaws tied with penalty.

1.7.2.2. Community Consultation and Participation

As the organization and management assessment prevails, the farmers in the project area have good attitude for implementation of the envisaged irrigation project. As the results of community consultation in the project area, they have committed and promised the following issues;

They are fully aware of the importance of irrigated agriculture and willing to use all irrigable land in the command area on equitable basis and mutual benefit to meet food security and local development.

The beneficiaries are willing to fully participate in construction through labour contribution and to take responsibility of O & M of the scheme then after.

They are willingly agreed to establish the required Water Users Association to properly utilize the water and for sustainable operation and management of their irrigation scheme.

Moreover, they decided to carry out required maintenance of irrigation structures, main & secondary canals and any in-farm structures by themselves and cooperatively manage the scheme through their community organization or Association. In general the proposed irrigation project area community was decided and gives their general petition based on total agreement including no asking about compensation if any in case of construction arise should be   attached on back of this document.

1.7.2.3. Potential, Opportunities, Constraints and Treats

1.7.2.4. POTENTIALs

Generally, command and surrounding project site is characterised in un fair rainfall distribution  and limited water potential, the availability Tabela spring water source considered as potential resource base and the life and livelihood base of the people in contributing human and animal drinking water as well as crop production with  suitable land topography.

`12.2.5. OPPORTUNITIES

Traditional irrigation has long years’ experience at the area is valuable opportunity for organizing water user association and sustainable operation of the system.

The area has suitable irrigable land with fertile soil that would be good opportunity to increase production and productive.

12.2.6. CHALLENGES

Some of the multidimensional issues recognized to be the major challenges to the project area includes:

Recurrent drought and erratic rainfall, and limited water resource

High flooding from the upstream the vast command areas and cause great crop damage every other year.

Low experience in application of agricultural input, mainly fertilizer and improved seeds.

High family size and the resulting pressure on limited

# 2. Agronomy

2.1. Introduction

Feasibility study of Balloo Habeebe small-scale irrigation project will be carried out in Oromia regional state, Bale zone, Gasara district, Balloo Habeebe peasant association on Xabala spring. Balloo Habeebe peasant association found in dry low land agro ecological Zone. Because of this moisture scarcity is the main limiting factor of crop production system in and around of this peasant association. To alleviate this problems, the study area, adopted using of traditional irrigation even if irrigation water management is poor.

2.1.1. Major agro- ecological zone of the project area.

Agro-ecologically, the project area belongs to low land climatic Zone what call locally <<Gamoj>>. Data (information) obtained from communities of the project area (during discussion) and district’s agricultural development  office indicate that, rainfall pattern of project area show notable variation from year to year and from season to season. This weather erratic happens, due to delaying on set of rainfall, occurrence of drought periods in between two rains and early withdrawal of rainfall. Because of this shortage of rainfall both in amount and distribution, is the main Constraints of the project area. In addition to this farmers of the study area suggest that, temperature of the project area  show an incremental variation from year to year especially during off season period, even if recorded data is un available. In general data obtained from, community, observation carried out during feasibility study and district’s pastoralist office show that, project area known by abnormal rainfall pattern, (in amount , distribution and duration) high  temperature (especially  during dry  season ) and low  humidity.

2.1.2. Potential evapotranspiration (ETO) of the project area.

There is no meteorological station near the project area. Because of this meteorological data of Metehara station, which is approximately275 km far from the project area, was taken to calculate   ETO of the project area.

Table 2.2 Rain fall pattern of the project area

Country: Location 18   Station: Metehar

Altitude: 930 m                   Latitude: 8.86         Longitude: 39 E

| Maths | Rain fall | Effective rain |
| --- | --- | --- |
| Jan | 15 | 0 |
| Feb | 45 | 17 |
| Mar | 44 | 16.4 |
| Apr | 63 | 27.8 |
| May | 34 | 10.4 |
| June | 21 | 2.6 |
| July | 120 | 72 |
| August | 122 | 73.6 |
| Sept | 46 | 26.6 |
| Oct | 9 | 0 |
| Nov | 3 | 0 |
| Dec | 2 | 0 |

Table 2.3 - reference crop evapotransparation of project area

Country: Location 18                                 Station: Metehara

Altitude : 930 m                Latitude: 8.86 N        Longitude :39 E

| Months | Number of days in months | Rainfall mm/month | Min Temp | Max Tem | Mean Temp(cº) | Sun (hours) | Humidity (%) | Wind (KM/day) | Rad MJ/M/day | ETO/day | ETO /month |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Jan | 31 | 21.04 | 13.9 | 31.3 | 22.6 | 7.8 | 41 | 121 | 19.2 | 4.51 | 139.81 |
| Feb | 28 | 27.28 | 16.2 | 31.8 | 24.00 | 7.8 | 46 | 104 | 20.4 | 4.52 | 126.56 |
| Mar | 31 | 85.76 | 18.3 | 34.0 | 26.15 | 7.9 | 43 | 104 | 21.5 | 5.11 | 158.41 |
| Apr | 30 | 209.44 | 18.3 | 33.6 | 25.95 | 7.6 | 45 | 86 | 21.2 | 4.9 | 147 |
| May | 31 | 195.61 | 18.2 | 34.9 | 26.55 | 7.8 | 37 | 78 | 21.0 | 4.81 | 149.11 |
| June | 30 | 34.18 | 20.9 | 36.2 | 28.55 | 7.7 | 36 | 130 | 20.5 | 5.64 | 169.2 |
| July | 31 | 14.70 | 20.4 | 33.2 | 26.8 | 6.6 | 50 | 164 | 18.9 | 5.34 | 165.54 |
| August | 31 | 26.27 | 19.4 | 32.2 | 25.8 | 6.8 | 50 | 112 | 19.7 | 4.82 | 149.42 |
| Sept | 30 | 84.18 | 19.6 | 33.9 | 26.75 | 7.1 | 40 | 78 | 20.2 | 4.72 | 141.6 |
| Oct | 31 | 191.12 | 16.0 | 33.8 | 24.9 | 7.7 | 31 | 78 | 20.4 | 4.66 | 144.46 |
| Nov | 30 | 69.24 | 12.7 | 32.2 | 22.45 | 8.2 | 34 | 86 | 19.9 | 4.37 | 131.1 |
| Dec | 31 | 21.60 | 13.6 | 30.8 | 22.2 | 8.4 | 40 | 95 | 19.5 | 4.18 | 129.58 |

Source: Clim Wat- 2.0

### 2.4. Length of growing period (Lgp) of existing crops of project area.

Time of planting or sowing date of each crops of the project area was taken properly from societies of project area, while length of growing period of each crop was determined based on real condition of the project area and information obtained from different literature review.

Table 2.4 Length of growing period of crops of project area.

| No | Types of cultivated crops | Time of sowing | Time of harvesting | Length of growing period (LGP) |
| --- | --- | --- | --- | --- |
| I | Arfasa (Belg) |  |  |  |
| 1 | Sorghum | March 1 | July 15 | 135 |
| 2 | Maize | March 10 | July 20 | 130 |
| 3 | Haricot bean | ‘’ 10 | July 10 | 120 |
| 4 | Teff | ;; 10 | July 10 | 120 |
| 5 | Coffee | ;; 10 | perennial | perennial |
| II | Hagaya (Meher) |  |  |  |
| 1 | Teff | ‘’ 5 | ‘’ 5 | 120 |
| 2 | Sorghum | August 5 | January 20 | 135 |
| 3 | Haricot bean | ‘’ 5 | ‘’ 5 | 120 |
| 4 | Cat | ‘’ 5 | ‘’ 5 |  |

Source- District’s Agricultural development office and societies project area

2.1.5. Existing farming systems of the project area

Before certain years ago livestock production (sedentary pastoralist farming system) is the main farming systems carried out in and around project area. At that time crop production farming system didn’t known more (source community). But now a day even if pastoralist farming system is the dominant one mixed agricultural farming system carried out by all pastoralist of the project area.

### 2.1.6. Existing cropping season of the project area.

Balloo Habeebe small -scale irrigation project area has two rainy season, what call locally Ganna (Belg) and Hagaya (Meher) according to local people. Rain duration of Ganna and Hagaya rainy season covers from mid of March to end of June and from end of July to end of November respectively. Specific planting time of project area somewhat vary from year to year.  Because, planting time of project area is highly affected by rain fall condition of the project area. Cultivated area coverage in both rainy seasons more or less similar except there is little variation in types of planted crops.

2.1.7. Existing crop pattern of rain fed crop production

A few annual crops or crops which completes their entire life cycle (from seed to seed) in single growing season, like Teff, Sorghum and maize are cultivated in both rainy seasons around project area

Table 2.5 crop pattern of rain fed crop production

| S.no | Type of cultivated crops | Area coverage (ha) | Yield per ha(Qt) | Total Yield(Qt) |
| --- | --- | --- | --- | --- |
| I | Belg( Arfasa) rainy season |  |  |  |
| 1 | Sorghum | 35 | 12 | 420 |
| 2 | Maize | 20 | 8 | 160 |
| 3 | Teff | 45 | 5 | 225 |
| 4 | Haricot bean | 5 | 3 | 15 |
| II | Meher(Hagaya)rainy season |  |  |  |
| 1 | Sorghum | 19 | 12 | 228 |
| 2 | Teff | 76 | 5 | 380 |
| 3 | Haricot bean | 10 | 3 | 30 |

Source –District’s Agricultural development office

### 2.8. Existing cropping pattern of rain fed crop production

Cropping pattern of certain agro ecology is highly affected by amount and distribution of rainfall of that area. Rainfall pattern of project area was erratic, due to delaying onset of rainfall, occurrence of drought in between two rains and early withdrawal of rains. Because of this single cropping pattern (growing only one per season) is the only cropping pattern adopted in rain fed crop production system carried out in the project area.

2.1.9. Existing agronomic practice of rain fed crop production

### 2.1.9.1 Land preparation

Farmers of the project area prepared (ploughed) their farmlands three or four times depending on condition of their farmlands. Like other parts of the country iron end what call locally ‘’ Marsha ‘’ is one of the main tools used to plough the farmlands by using pair of oxen or main draft power

### 2.1.9.2 Planting (sowing) methods

Normally two methods of planting (sowing) methods are known. These are Broad casting and row planting methods. Out of these, broad casting is the one and the main planting methods adopted in and around project area for all cereal crops.

2.1.9.3. Seeding rate of existing crops

In principle planting rate is depend on crop species, growth habit, tillering (branching) character, soil fertility status, seed purity and viability. However societies of the project area using planting rate shown in the following table depending on their indigenous knowledge.

Table 2.6 Existing seeding rate of project area.

| Type of existing crops | Existing methods of planting | Existing seed rate |
| --- | --- | --- |
| I Belg (Arfasa) rainy season |  |  |
| Maize | Broad casting | 45kg |
| Sorghum | ‘’ | 65kg |
| Teff | ‘’ | 20kg |
| Haricot bean | ‘’ | 20kg |
| II Meher (Hegaya) rainy season |  |  |
| Sorghum | ‘’ | 65kg |
| Teff | ‘’ | 20kg |
| Haricot bean | ‘’ | 20kg |

Source – District‘s Agricultural Development office

2.10. Existing weed control methods of project area.

From known weed control methods (physical, chemical and Biological weed control methods) physical weed control methods is the major weed control methods adopted in the project area.  Out of these physical weeds control methods mowing and hand weeding are the main weed control methods carried out in the project area while few farmers used chemical weed control methods for cereal crops like Teff.

### 2.11. Existing crop disease pests and control methods of the project area.

Data obtained from societies of project area and district’s pastoralist office show that, there are different plant disease and plant pests in and around project area.

\

\

Table 2.7 crops disease, pest and existing control methods

| Existing wild animals pests | Existing control methods | Existing insect pest | Existing control methods | Existing plant disease | Existing control methods |
| --- | --- | --- | --- | --- | --- |
| Monkey | Guarding | Grass hopper | No control methods -using insect side | Rust Smut | No control methods |
| Baboon |  | Cut worm |  |  |  |
| Pig |  | Aphids |  |  |  |
| Hedgehog |  | Army worm |  |  |  |
| Birds |  | Termite |  |  |  |
|  |  | Weevils |  |  |  |

Source: - district Agricultural Development office and societies of project area

2.12. Labor requirement for rain fed cultural practices

Labor is key input of agricultural production particularly in hoe culture. Detail labor requirement of each rain fed crops of project area given in table below.

Table 2.8 existing labour requirements for agronomic practice of rain fed crop production in the project area per hectare

| Crop | Cultural practices | Frequency | Required man day | Oxen day pair |
| --- | --- | --- | --- | --- |
| Sorghum | Plowing | 4 | 8 | 8 |
|  | Sowing | 1 | 10 | 8 |
|  | Weeding | 1 | 10 |  |
|  | Harvesting | 1 | 10 |  |
|  | Thrashing and Bagging | 1 | 5 |  |
|  | Loading and unloading | 1 | 2 |  |
| Maize | Plowing | 4 | 8 | 8 |
|  | Sowing | 1 | 10 | 8 |
|  | Weeding | 1 | 10 |  |
|  | Harvesting | 1 | 9 |  |
|  | Threshing and Bagging | 1 | 8 |  |
|  | Loading and un Loading | 1 | 2 |  |
| Teff | Plowing | 4 | 8 | 8 |
|  | Sowing | 1 | 10 | 8 |
|  | Weeding | 1 | 10 |  |
|  | Harvesting | 1 | 12 |  |
|  | Thrashing and Bagging | 1 | 6 |  |
|  | Loading and unloading | 1 | 2 |  |
| Haricot bean | Plowing | 4 | 8 | 8 |
|  | Sowing | 1 | 10 | 8 |
|  | Weeding | 1 | 10 |  |
|  | Harvesting | 1 | 11 |  |
|  | Threshing and Bagging | 1 | 6 |  |
|  | Loading and un Loading | 1 | 2 |  |

Source –Societies of project area

2.13. Existing crop pattern of traditionally irrigated crop production

Crop pattern of irrigated crops adopted in the project area and out of the project all in all is similar .Some of the traditionally irrigated crop pattern of the project area given below.

Table 2.9 crop patterns of traditionally irrigated crops of project area

|  | Type of cultivated crops | Area coverage( ha) | Yieldper ha(Qt) | Total Yield (Qt) | Existing seed rate |
| --- | --- | --- | --- | --- | --- |
| I. | Belg (Arfasa) rainy season |  |  |  |  |
|  | -Maize | 20 | 35 | 700 | 45kg |
|  | -Tomato | 1 | 65 | 65 | 0.75kg |
|  | -Avocado | 5 | 50 | 250 | 360 seedling |
|  | -sugar cane | 10 | 150 | 1275 | 15150set |
|  | -Orange | 1 | 55 | 55 |  |
|  | -Banana | 10 | 50 | 500 | 1886 |
|  | -Mango | 5 | 55 | 275 | 480 |
|  | -Papaya | 3 | 50 | 150 | 3200 |
|  | -Coffee | 25 | 4 | 100 | 3500seedling |
|  | -Cat | 10 | 90(wasla) | 900(wasla) | 4445cutting |
| II | Meher (Hegaya) rainy season |  |  |  |  |
|  | -Maize | 20 | 35 | 700 | 45kg |
|  | -Tomato | 1 | 65 | 65 | 0.75kg |

2.14. Existing cropping pattern of traditionally irrigated crop production

Project area is known by erratic rainfall pattern due to delaying on set of rainfall, occurrence of drought in between two rains and early withdrawal of rains. However, farmers of the project area produce annual and perennial crops by using traditional irrigation, by adopting singlecropping pattern for perennial crops and double cropping pattern for annual crops.

2.15. Existing agronomic practices of traditionally irrigated crop production

2.15.1 Methods of land preparation, planting and weeding

Adopted methods of land preparation, planting and weeding of irrigated crop production is, the same as with adopted methods of cultural practices for rain fed agriculture.

2.15.2 Irrigation methods

Irrigation water may be applied to crop by several ways (methods). Out of these methods traditional surface irrigation is practiced in the project area, that can be used to irrigate all crops planted in rows, including orchards and vegetables .Among the cultivated common crops the methods is suitable to irrigate maize, sun flower, Sorghum, Sugar cane, Soybean, Cotton and so on. However even if Maize , vegetables and someperennialcrops are the main crops, which cultivated in the project area, Uncontrolled flooding irrigation methods and somewhat basin irrigation methods  is  main one which adopted in the project area.

2.16. Existing labour requirement for traditionally irrigated agronomic practices

Labor is a key input of agricultural production particularly in hoe culture. Detail labor requirement of each traditionally irrigated crops of project area were given below in the following table.

Table 2.10 existing labour requirements for traditionally irrigated agronomic practices.

| Crop | Cultural practices | Frequency | Required man day | Oxen day pair |
| --- | --- | --- | --- | --- |
| Sugar cane | Plowing | 4 | 8 | 8 |
|  | Preparing planting materials | 1 | 10 |  |
|  | Fertilizing | 1 | 10 |  |
|  | Planting | 1 | 11 |  |
|  | Irrigation | 10 | 6 |  |
|  | Hoeing | 1 | 20 |  |
|  | Threshing | 1 | 30 |  |
|  | Loading and un loading | 1 | 5 |  |
| Banana | Plowing | 4 | 8 | 8 |
|  | Preparing planting materials | 1 | 10 |  |
|  | Fertilizing | 1 | 10 |  |
|  | Planting | 1 | 10 |  |
|  | Irrigation | 10 | 6 |  |
|  | Hoeing | 2 | 20 |  |
|  | Harvesting | 1 | 20 |  |
|  | Loading and un loading | 1 | 10 |  |
| Orange | Plowing | 4 | 8 | 8 |
|  | Transplanting | 1 |  |  |
|  | Weeding | 1 | 6 |  |
|  | Fertilizing | 1 | 10 |  |
|  | Irrigating | 10 | 6 |  |
|  | Harvesting | 1 | 20 |  |
|  | Loading and un Loading | 1 | 10 |  |
| Mango | Plowing | 4 | 8 | 8 |
|  | Transplanting | 1 |  |  |
|  | Weeding | 1 | 6 |  |
|  | Fertilizing | 1 | 10 |  |
|  | Irrigating | 10 | 6 |  |
|  | Harvesting | 1 | 20 |  |
|  | Loading and un Loading | 1 | 10 |  |
| Papaya | Plowing | 4 | 8 | 8 |
|  | Preparing planting materials | 1 | 10 |  |
|  | Fertilizing | 1 | 10 |  |
|  | Planting | 1 | 10 |  |
|  | Irrigation | 10 | 6 |  |
|  | Hoeing | 2 | 20 |  |
|  | Harvesting | 1 | 20 |  |
|  | Loading and un loading | 1 | 10 |  |
| Maize | Plowing | 4 | 8 | 8 |
|  | Sowing | 1 | 10 | 8 |
|  | Irrigation | 4 | 4 |  |
|  | Weeding | 1 | 12 |  |
|  | Harvesting | 1 | 6 |  |
|  | Threshing and Bagging | 10 | 9 |  |
|  | Loading and un Loading | 1 | 4 |  |
| Coffee | Plowing | 4 | 8 | 8 |
|  | Fertilizing | 1 | 2 |  |
|  | Transplanting | 1 | 20 |  |
|  | Irrigation | 8 | 4 |  |
|  | Hoeing | 4 | 20 |  |
|  | Harvesting and Bagging | 1 | 30 |  |
|  | Loading and un loading | 1 | 4 |  |
| Avocado | Plowing | 4 | 8 | 8 |
|  | Fertilizing | 1 | 2 |  |
|  | Transplanting | 1 | 20 |  |
|  | Irrigating | 7 | 4 |  |
|  | Hoeing | 4 | 15 |  |
|  | Harvesting and Bagging | 1 | 15 |  |
|  | Loading and un Loading | 1 | 5 |  |
| Tomato | Plowing | 4 | 8 | 8 |
|  | Fertilizing | 1 | 2 |  |
|  | Transplanting | 1 | 20 |  |
|  | Irrigating | 7 | 4 |  |
|  | Hoeing | 3 | 20 |  |
|  | Harvesting and Bagging | 1 | 30 |  |
|  | Loading and un loading | 1 | 5 |  |

Source: - Societies of project area

# 2.17 constraints of crop production system of the project area

Date obtained from, observation during feasibility study, societies of project area and district’s pastoralist office show that, crop production and productivity is back ward and quite low in the project area. This is due to:-

2.17.1 Agronomic and environmental constraints

- Shortage and Un even distribution of rain fall

- early with  drawl of rain fall

- occurrence of  irregular drought between two rains

- short duration of rain fall

- high Temperature

- Pest infestation

- In appropriate cultural practice

- Lack of water conservation activities

- Poor genetic potential of crop varieties

- Absent of improved agricultural inputs

- Lack of knowledge on input utilization

- Absent of modern small scale irrigation

2.17.2 Institutional constraints

- Poor supporter service

- Poor marketing system

- Lack of required infrastructure

2.18. Existing potential of crop production of project area

Even if constraints of crop production in project area are still enormous and require further improvement to indorse productivity. On the other hand the project area has high potential, for crop production.

Has high potential for photosynthesis

- There is labor availability

- There is suitable altitude

- Different crop can grow

2.19. Existing agricultural input

Survey carried out during feasibility study and data obtained from development agents of peasant association show that, Fertilizer and pesticide like 2.4.D somewhat utilizedbya few farmers around project area. Except the mentioned one, up to now the remains agricultural inputs are unknown in the project area.

2.20. Existing supportive service

Extension service and credit service are existing supportive service in andaround project area. Out of these, agricultural extension service is the one and the main supportive service given to pastoralist of the project area, by development agents and district’s agricultural office experts to facilitate implementation of planned development programs.

### 2.21. The possible list of crops (crop basket) for proposed irrigation project area

A list of a range of crops (crop basket) which can be grown in agro- climatic and soil conditions of project area are listed in the following table.

2.11 Table possible list of crops for proposed irrigation project area

| s.n | Crop group | Types of crops |
| --- | --- | --- |
| 1 | Cereals | Sorghum ,maize ,Teff, |
| 2 | Pulses | Haricot bean |
| 3 | Oil crops | sesame |
| 4 | Vegetables | Onion ,pepper ,sweet potato ,tomato ,Eggplant , |
| 5 | Fruits | Guava ,papaya ,orange ,mango, lemon , Banana, Mandarin, pin apple |
| 6 | Other perennials | Sugarcane |

2.22. Proposed crop pattern of project area

Agro- ecologically, Balloo Habeebe small-scale irrigation project area is belong to low land climatic zone.  This implies   that, low land crops (annual and perennial crops) can be cultivated in and around project area. However, rainfall pattern of the project area is very poor in amount, duration and distribution. Because of these, farmers of the project are producing annual and perennial crops by diverting Xabala spring. This implies that annual and perennial crops are suitable for newly proposed irrigation project.

2.12 Table Crop pattern of proposed crops

| S.n | Proposed crops | Area coverage (ha) |  | Yield/ha |  |
| --- | --- | --- | --- | --- | --- |
| I | Belg (Arfasa) |  |  |  |  |
| 1 | Maize | 25 |  | 65 |  |
| 2 | Onion |  | 10 |  | 230 |  |
| 3 | H. Bean |  | 6 |  | 20 |  |
| 4 | Pepper |  | 15 |  | 12 |  |
| 6 | Avocado |  | 6 |  | 152 |  |
| 7 | Mango |  | 5 |  | 152 |  |
| 8 | Banana |  | 10 |  | 105 |  |
| 9 | Papaya |  | 3 |  | 100 |  |
| II | Meher (Hagaya) |  |  |  |  |  |
| 1 | Maize |  | 21 |  | 65 |  |
| 2 | Haricot bean |  | 12 |  | 20 |  |
| 3 | Onion |  | 14 |  | 230 |  |
| 4 | Pepper |  | 1 |  | 12 |  |
| 5 | Tomato |  | 8 |  | 350 |  |

2.23 .Ecological suitability of proposed crops

Project area’s ecology should be suitable for proposed crops, in order to gain potential yield, which expected from proposed crops since   one cultivar can give potential yield, if cultivated in suitable agro-ecology and managed properly. Recommended ecological requirement (suitability) of proposed crops which organized from different literature review and project area’s existing ecology were given in the table below.

Table 2.13 ecological suitability of proposed crops and existing Project area’s ecology

| Proposed crops | Ecological requirement (suitability) of proposed crops | Existing project area’s ecology |
| --- | --- | --- |
| Onion | -Grow at 500-2400 m.a.s.l -18-24 optimum temperature for bulb formation -Medium light loving plants -Require short day length for bulb formation | - Low land ecological zone - Found at m .a.s.l. RH Jan 41 Feb 46 March 43 April 45 May 37 Jun 36 July 50 August 50 Sap 40 Oct 31 Nov 34 Dec 40 Jan 121 Feb 104 March 104 April 86 May 78 Jun 130 July 164 August 112 Sap 78 Oct 78 Nov 86 Dec 95 Temperature (cº) MaxiMin Jan 31.3 13.9 Feb 31.8 16.2 March 34 18.3 April 33.6 18.3 May 34.9 18.2 Jun 36.2 20.9 July 33.3 20.4 August 32.2 19.4 Sept 33.9 19.6 Oct 33.8 16 Nov 32.2 12.7 Dec 30.8 13.6 Rainfall (mm) Total effective rain fall rain fall Jan 15 0 Feb 45 17 March 44 16.4 April 63 27.8 May 34 10.4 Jun 21 2.6 July 120 72 August 122 73.6 Sap 46 26.6 Oct 9 0 Nov 2 0 Dec 3 0 -high light intensity during of season - medium light intensity during cropping season - short day length |
| Tomato | - Grow at 700-2000 m.a.s.l - Grown in worm and dry day - Love cool night - 21-24 cº optimum temperature for growth - Sun loving plants - Grown in high light intensity - Day neutral plants |  |
| Haricot bean | -1400 – 2000m.a.s.l - Grow in short day length for flowering -lowland and crop - Grown in tropical -Grow in frost free area |  |
| Pepper | 21-24cº is optimum temperature -Tropical crop -Grow at 1000-2000 m.a.s.l - Grow in short lay length for flowering |  |
| Maize | -Grow at 500-2800 m.a.s.l minimum to maximum- - Temp 10cº -38cº minimum to maximum - Grow in short day length for flowering - Tropical crop -can grow in worm temperature |  |
| Avocado | -Grow in tropical low land -Grow from sea level-1915 masl -25 is optimum temperature -Grow up to 915 masl |  |
| Mango | -Grow in tropical low land -Grow from sea level-1915 masl -25 is optimum temperature -Grow up to 915 masl -25 Optimum temperature -Drought tolerant crop |  |
| Banana | -Grow in tropical -Grow from sea level -1800 masl -Temp 25 – 30 considered as ideal -Grow in high light intensity -Require water -Grow in deep light soil |  |
| Papaya | --Grow in tropical up to 1500 masl Grow in frost free sub tropical -25 – 38 optimum temperature -R. H should be at least 60% |  |

Source: - Different literature review

2.24. Soil suitability of proposed crops

Soil types and their physical, chemical biological properties influence the choice of crops. Any adverse soils characteristics will impede the growth and thereby reduce yield of the crop. The crops generally require good physical and chemical conditions of soils. Slope gradient is a physical feature of the land taken as selection criteria for identification of land suitability for agriculture in general and for irrigated agriculture in particular. As recommended in different literatures, the type of irrigation application system is mainly relying on steepness nature of land. In most cases, the command area with the slope gradient of less than 3% is preferred for surface irrigation system. however due to different reasons, most importantly shortage of arable land, the project will be in condition to use the land up to 12% of slopes for irrigated agriculture.

Result of feasibility soil study of Ballo Habeebe irrigation project show that soil of the command area known by sandy loam soil almost good soil depth, and gently slope and also soil of the command area is free from salinity and acidity. In addition to these soils of the command area also known by recommended soil chemical properties, except available phosphorus (p) and sodium need correction.

2.25. Cropping pattern of proposed crops

Cropping pattern practiced in certain agro-ecology is highly affected by water (rainfall) availability and Climatic condition of the project area.  These condition show that single cropping pattern (growing only once per a year) is suitable for perennial crops while double cropping pattern per year is suitable for annual crops.

Table 2.14 proposed crop pattern and cropping   intensity of Balloo Habeebe small scale irrigation scheme

| Crop | Year 1 |  |  | Year 2 |  |  | Year 3 |  |  | Year 4 |  |  | Year 5 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Ph. 1 | Ph.2 | Total | Ph. 1 | Ph.2 | Total | Ph. 1 | Ph.2 | Total | Ph. 1 | Ph.2 | Total | Ph. 1 | Ph.2 | Total |
|  | Ha | ha | ha | ha | ha | ha | ha | ha | ha | ha | ha | ha | ha | ha | ha |
| Maize | 12 | 15 | 30 | 15 | 17 | 32 | 20 | 25 | 45 | 25 | 25 | 50 | 25 | 25 | 50 |
| Haricot bean | 6 | 2 | 8 | 8 | 4 | 12 | 11 | 5 | 16 | 12 | 6 | 18 | 12 | 6 | 18 |
| Onion | 7 | 5 | 12 | 10 | 7 | 17 | 12 | 9 | 21 | 14 | 10 | 24 | 14 | 10 | 24 |
| Pepper | 1 | 5 | 6 | 1 | 10 | 11 | 1 | 12 | 13 | 1 | 14 | 15 | 1 | 15 | 16 |
| Tomato | 4 | - | 4 | 5.5 | - | 5.5 | 7 | - | 7 | 7.5 | - | 7.5 | 7.5 | - | 8 |
| Mango | - | 5 | 5 | - | 5 | 5 | - | 5 | 5 | - | 5 | 5 | - | 5 | 5 |
| Avocado | - | 6 | 6 | - | 6 | 6 | - | 6 | 6 | - | 6 | 6 | - | 6 | 6 |
| Banana | - | 10 | 10 | - | 10 | 10 | - | 10 | 10 | - | 10 | 10 | - | 10 | 10 |
| Papaya | - | 3 | 3 | - | 3 | 3 | - | 3 | 3 | - | 3 | 3 | - | 3 | 3 |
| Total |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 140 |
| Intensity | 175% |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

2.26. Cropping calendar of proposed crops

Depending on water availability, climatic and farmers trend of the project area cropping calendar of the project area was proposed as following.

Table 2.15 cropping calendars of proposed crops

| Proposed crops | Area coverage (ha) |  | Time of planting | Time of harvesting | Length of growing periods |
| --- | --- | --- | --- | --- | --- |
| Belg (Arfasa) |  |  |  |  |  |
| Maize | 25 |  | March 1 | Jun 30 | 120 |
| Onion | 10 |  | March 1 | Jun 30 | 120 |
| H. Bean | 6 |  | March 1 | Jun 30 | 120 |
| Pepper | 15 |  | March 1 | July 10 | 130 |
| Avocado | 6 |  | March 55 | Perennial |  |
| Mango | 5 | March 15 |  | Perennial | -3-8 years form newly transplanted |
| Banana | 10 | March 15 |  | Perennial | 300-360 days |
| Papaye | 3 | March 15 |  | Perennial | 300-360 days |
| Meher (Hagaya) |  |  |  |  |  |
| Maize | 21 | August 25 |  | Nover 25 | 120 |
| H.bean | 12 | August 10 |  | Nover10 | 120 |
| Onion | 14 | August 10 |  | Noveber10 | 120 |
| Pepper | 1 | August 10 |  | January 20 | 130 |
| Tomato | 8 | August 10 |  | Noveber10 | 120 |

Source: - District’s Agricultural Development office, societies of project area anddifferent literaturereview

Table 2.16 proposed crop pattern and yearly yield build up pattern of Balloo Habeebe small scale irrigation scheme

| crops | Year 1 |  |  |  |  | Year 2 |  |  |  |  | Year 3 |  |  |  |  | Year 4 |  |  |  |  | Year 5 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Phase 1 |  | Phase 2 |  | Total | Phase 1 |  | Phase 2 |  | Total | Phase 1 |  | Phase 2 |  | Total | Phase 1 |  | Phase 2 |  | Total | Phase 1 |  | Phase 2 |  |  | Total |
|  | ha | Yield/ha | ha | Yield/ha | Yield/ha | ha | Yield/ha | ha | Yield/ha | Yield | ha | Yield/ha | ha | Yield/ha | Yield | ha | Yield/ha | ha | Yield/ha | Yield | ha | Yield/ha | ha | Yield/ha |  | Yield |
|  |  | (Qt) |  | (Qt) | (Qt) |  | (Qt) |  | (Qt) | [Qt] |  | (Qt) |  | (Qt) | [Qt] |  | (Qt) |  | (Qt) | [Qt] |  | (Qt) |  | (Qt) |  | [Qt] |
| Maize | 12 | 35 | 15 | 33 |  | 15 | 45 | 17 | 43 |  | 20 | 55 | 25 | 53 |  | 25 | 65 | 25 | 63 |  | 25 | 65 | 25 | 65 |  |  |
| H. Bean | 6 | 8 | 2 | 9 |  | 8 | 11 | 4 | 12 |  | 11 | 14 | 5 | 15 |  | 12 | 17 | 6 | 18 |  | 12 | 20 | 6 | 20 |  |  |
| Onion | 7 | 120 | 5 | 100 |  | 10 | 135 | 7 | 115 |  | 12 | 170 | 9 | 150 |  | 14 | 200 | 10 | 185 |  | 14 | 230 | 10 | 230 |  |  |
| Pepper | 1 | 4 | 5 | 3 |  | 1 | 5 | 10 | 4 |  | 1 | 9 | 12 | 8 |  | 1 | 11 | 14 | 10 |  | 1 | 12 | 15 | 12 |  |  |
| Tomato | 4 | 100 | - | 150 |  | 5.5 | 150 | - | 200 |  | 7 | 200 | - | 250 |  | 7.5 | 250 | - | 300 |  | 7.5 | 350 | - | 350 |  |  |
| Mango | -- |  | 5 | 80 |  | - |  | 5 | 85 |  | - |  | 5 | 120 |  | - |  | 5 | 150 |  | - |  | 5 | 160 |  |  |
| Avocado | - |  | 6 | 70 |  | - |  | 6 | 80 |  | - |  | 6 | 100 |  | - |  | 6 | 150 |  | - |  | 6 | 152 |  |  |
| Banana | - |  | 10 | 70 |  | - |  | 10 | 75 |  | - |  | 10 | 85 |  | - |  | 10 | 95 |  | - |  | 10 | 105 |  |  |
| Papaya | - |  | 3 | 65 |  | - |  | 3 | 75 |  | - |  | 3 | 85 |  | - |  | 3 | 95 |  | - |  | 3 | 100 |  |  |

2.27. Crop water requirement of proposed crops

There is no meteorological station near the project area. Because of this meteorological data of Metehara Metrological station, which is approximately 275 km far from the project area, was taken to calculate   ETO of the project area.

Table 2.17 reference crop evapotransparation of project area

Country: Location 18                    Station: Metehara

Altitude: 930                 Latitude: 8.86 N    Longitude: 39 E

| Months | Number of days in months | Min Temp | Max Tem | Mean Temp(cº) | Sun (hours) | Humidity (%) | Wind (KM/day) |  | ETO/day | ETO/Month |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Jan | 31 | 13.9 | 31.3 | 22.6 | 7.8 | 41 | 121 | 19.2 | 4.51 | 139.81 |
| Feb | 28 | 16.2 | 31.8 | 24.00 | 7.8 | 46 | 104 | 20.4 | 4.52 | 126.56 |
| Mar | 31 | 18.3 | 34.0 | 26.15 | 7.9 | 43 | 104 | 21.5 | 5.11 | 158.41 |
| Apr | 30 | 18.3 | 33.6 | 25.95 | 7.6 | 45 | 86 | 21.2 | 4.9 | 147 |
| May | 31 | 18.2 | 34.9 | 26.55 | 7.8 | 37 | 78 | 21.0 | 4.81 | 149.11 |
| June | 30 | 20.9 | 36.2 | 28.55 | 7.7 | 36 | 130 | 20.5 | 5.64 | 169.2 |
| July | 31 | 20.4 | 33.2 | 26.8 | 6.6 | 50 | 164 | 18.9 | 5.34 | 165.54 |
| August | 31 | 19.4 | 32.2 | 25.8 | 6.8 | 50 | 112 | 19.7 | 4.82 | 149.42 |
| Sept | 30 | 19.6 | 33.9 | 26.75 | 7.1 | 40 | 78 | 20.2 | 4.72 | 141.6 |
| Oct | 31 | 16.0 | 33.8 | 24.9 | 7.7 | 31 | 78 | 20.4 | 4.66 | 144.46 |
| Nov | 30 | 12.7 | 32.2 | 22.45 | 8.2 | 34 | 86 | 19.9 | 4.37 | 131.1 |
| Dec | 31 | 13.6 | 30.8 | 22.2 | 8.4 | 40 | 95 | 19.5 | 4.18 | 129.58 |

Source: Clim Wat- 2.0

Table 2.18 Balloo habeebe irrigation project crop water requirement / design supply/   for crop maize

| S/N | Designaton | Symbol | Operation | Unit | Months |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Jan. | Feb. | Mar. | Apr. | May. | Jun. | Jul. | Aug. | Sep. | Oct. | Nov. | Dec. |
| 1 | ETO | ETO | Pen Man | mm/month | 139.81 | 126.56 | 158.41 | 147 | 149.11 | 169.2 | 165.54 | 149.42 | 141.6 | 144.46 | 131.1 | 129.54 |
| 2 | Crop factor | Kc | From Table |  |  |  | 0.3 | 0.7 | 0.85 | 0.85 | 0.6 |  |  |  |  |  |
| 3 | Crop Evap. (pot) | ETcrop | 1x2 | mm/month |  |  | 47.523 | 102.9 | 126.24 | 143.82 | 99.324 |  |  |  |  |  |
| 4 | Rain fall | P | Meteoro Data | mm/month | 15 | 45 | 44 | 63 | 34 | 21 | 120 | 122 | 46 | 9 | 3 | 2 |
| 5 | Effective Rainfall | Pe | 0.8p-25/0.6p-10 | mm/month | 0 | 17 | 16.4 | 27.8 | 10.4 | 2.6 | 72 | 73.6 | 26.6 | 0 | 0 |  |
| 6 | Effective Rainfall | Pe | 0.8p-25/0.6p-10 | mm/month | 0 | 17 | 16.4 | 27.8 | 10.4 | 2.6 | 72 | 73.6 | 26.6 | 0 | 0 |  |
| 7 | TNI Req’t | INT | 3 minus 6 | mm/month |  |  | 31.123 | 75.1 | 116.34 | 141.22 | 27.324 |  |  |  |  |  |
| 8 | Project eff | EP | Calc overall eff |  | 0.45 | 0.45 | 0.45 | 0.45 | 0.45 | 0.45 | 0.45 | 0.45 | 0.45 | 0.45 | 0.45 | 0.45 |
| 9 | TGI Req’t | Lt | 7\8 | mm/month |  |  | 69.16 | 166.69 | 258.53 | 313.82 | 60.72 |  |  |  |  |  |
| 10 | sup req/ha | Qt | 9/259.2 | L/s |  |  | 0.267 | 0.64 | 0.997 | 1.21 | 0.23 |  |  |  |  |  |
| 11 | Irri area | A | Crop pattern | Ha |  |  | 25 | 25 | 25 | 25 | 25 | 25 |  |  |  |  |
| 12 | supp Reqt | Q | 10x11 | L/s |  |  | 6.675 | 16 | 24.925 | 30.25 | 5.75 |  |  |  |  |  |
| 13 | Hr app/d | Hr | Given | Hour | 12.0 | 12.0 | 12.0 | 12.0 | 12.0 | 12.0 | 12.0 | 12.0 | 12.0 | 12.0 | 12.0 | 12.0 |
| 14 | Ratiohours of appl/24 | R | H/24 | Hour | 0.5 | 0.5 | 0.5 | 0.5 | 0.5 | 0.5 | 0.5 | 0.5 | 0.5 | 0.5 | 0.5 | 0.5 |
| 15 | Projectsupply req’t | Q/2 | 12/14 | L/s |  |  | 13.35 | 32 | 49.85 | 60.5 | 11.5 |  |  |  |  |  |
| 16 | Des. supp | Q | Max (15 | L/s/ha |  |  |  |  |  | 2.42 |  |  |  |  |  |  |

Table 2.19 Balloo habeebe irrigation project crop water requirement / design supply/   for crop hairy coat bean

| S/N | Designaton | Symbol | Operation | Unit | Months |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Jan. | Feb. | Mar. | Apr. | May. | Jun. | Jul. | Aug. | Sep. | Oct. | Nov. | Dec. |
| 1 | ETO | ETO | PEN MAN | mm/month | 139.81 | 126.56 | 158.41 | 147 | 149.11 | 169.2 | 165.54 | 149.42 | 141.6 | 144.46 | 131.1 | 129.54 |
| 2 | Crop factor | Kc | From Table |  |  |  | 0.3 | 0.7 | 0.75 | 0.75 | 0.65 |  |  |  |  |  |
| 3 | Crop Evap. (pot) | ETcrop | 1x2 | mm/month |  |  | 47.523 | 102.9 | 111.83 | 126.9 | 107.6 |  |  |  |  |  |
| 4 | Rain fall | P | Meteoro Data | mm/month | 15 | 45 | 44 | 63 | 34 | 21 | 120 | 122 | 46 | 9 | 3 | 2 |
| 5 | Effective Rainfall | Pe | 0.8p-25/0.6p-10 | mm/month | 0 | 17 | 16.4 | 27.8 | 10.4 | 2.6 | 72 | 73.6 | 26.6 | 0 | 0 |  |
| 6 | Effective Rainfall | Pe | 0.8p-25/0.6p-10 | mm/month | 0 | 17 | 16.4 | 27.8 | 10.4 | 2.6 | 72 | 73.6 | 26.6 | 0 | 0 |  |
| 7 | TNI Req’t | INT | 3 minus 6 | mm/month |  |  | 63.923 | 75.1 | 101.4 | 124.3 | 35.6 |  |  |  |  |  |
| 8 | Project eff | EP | Calc overall eff |  | 0.45 | 0.45 | 0.45 | 0.45 | 0.45 | 0.45 | 0.45 | 0.45 | 0.45 | 0.45 | 0.45 | 0.45 |
| 9 | TGI Req’t | Lt | 7\8 | mm/month |  |  | 142.05 | 166.89 | 225.33 | 276.2 | 79.11 |  |  |  |  |  |
| 10 | sup req/ha | Qt | 9/259.2 | L/s |  |  | 0.55 | 0.64 | 0.87 | 1.066 | 0.3 |  |  |  |  |  |
| 11 | Irri area | A | Crop pattern | Ha |  |  | 6 | 6 | 6 | 6 | 6 |  |  |  |  |  |
| 12 | supp Reqt | Q | 10x11 | L/s |  |  | 3.3 | 3.84 | 5.22 | 6.396 |  |  |  |  |  |  |
| 13 | Hr app/d | Hr | Given | Hour | 12.0 | 12.0 | 12.0 | 12.0 | 12.0 | 12.0 | 12.0 | 12.0 | 12.0 | 12.0 | 12.0 | 12.0 |
| 14 | Ratiohours of appl/24 | R | H/24 | Hour | 0.5 | 0.5 | 0.5 | 0.5 | 0.5 | 0.5 | 0.5 | 0.5 | 0.5 | 0.5 | 0.5 | 0.5 |
| 15 | Projectsupply req’t | Q/2 | 12/14 | L/s |  |  | 6.6 | 7.68 | 10.44 | 12.792 |  |  |  |  |  |  |
| 16 | Des. supp | Q | Max (15) | L/s/ha |  |  |  |  |  | 2.132 |  |  |  |  |  |  |

Table 2.20 Balloo habeebe irrigation project crop water requirement / design supply/   for crop onion

| S/N | Designaton | Symbol | Operation | Unit | Months |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Jan. | Feb. | Mar. | Apr. | May. | Jun. | Jul. | Aug. | Sep. | Oct. | Nov. | Dec. |
| 1 | ETO | ETO | PEN MAN | mm/month | 139.81 | 126.56 | 158.41 | 147 | 149.11 | 169.2 | 165.54 | 149.42 | 141.6 | 144.46 | 131.1 | 129.54 |
| 2 | Crop factor | Kc | From Table |  |  |  | 0.3 | 07 | 0.85 | 0.7 5 | 0.6 5 |  |  |  |  |  |
| 3 | Crop Evap. (pot) | ETcrop | 1x2 | mm/month |  |  | 47.523 | 102.9 | 126.74 | 126.9 | 107.6 |  |  |  |  |  |
| 4 | Rain fall | P | Meteoro Data | mm/month | 15 | 45 | 44 | 63 | 34 | 21 | 120 | 122 | 46 | 9 | 3 | 2 |
| 5 | Effective Rainfall | Pe | 0.8p-25/0.6p-10 | mm/month | 0 | 17 | 16.4 | 27.8 | 10.4 | 2.6 | 72 | 73.6 | 26.6 | 0 | 0 |  |
| 6 | Effective Rainfall | Pe | 0.8p-25/0.6p-10 | mm/month | 0 | 17 | 16.4 | 27.8 | 10.4 | 2.6 | 72 | 73.6 | 26.6 | 0 | 0 |  |
| 7 | TNI Req’t | INT | 3 minus 6 | mm/month |  |  | 63.923 | 75.1 | 116.34 | 124.3 | 35.6 |  |  |  |  |  |
| 8 | Project eff | EP | Calc overall eff |  | 0.45 | 0.45 | 0.45 | 0.45 | 0.45 | 0.45 | 0.45 | 0.45 | 0.45 | 0.45 | 0.45 | 0.45 |
| 9 | TGI Req’t | Lt | 7\8 | mm/month |  |  | 142.05 | 166.89 | 258.53 | 276.2 | 79.11 |  |  |  |  |  |
| 10 | sup req/ha | Qt | 9/259.2 | L/s |  |  | 0.55 | 0.64 | 0.997 | 1.066 | 0.3 |  |  |  |  |  |
| 11 | Irri area | A | Crop pattern | Ha |  |  | 10 | 10 | 10 | 10 | 10 | 10 |  |  |  |  |
| 12 | supp Reqt | Q | 10x11 | L/s |  |  | 5.5 | 6.4 | 9.97 | 10.66 |  |  |  |  |  |  |
| 13 | Hr app/d | Hr | Given | Hour | 12.0 | 12.0 | 12.0 | 12.0 | 12.0 | 12.0 | 12.0 | 12.0 | 12.0 | 12.0 | 12.0 | 12.0 |
| 14 | Ratiohours of appl/24 | R | H/24 | Hour | 0.5 | 0.5 | 0.5 | 0.5 | 0.5 | 0.5 | 0.5 | 0.5 | 0.5 | 0.5 | 0.5 | 0.5 |
| 15 | Projectsupply req’t | Q/2 | 12/14 | L/s |  |  | 11 | 12 | 19.94 | 21.32 |  |  |  |  |  |  |
| 16 | Des. supp | Q | Max (15) | L/s/ha |  |  |  |  |  | 2.132 |  |  |  |  |  |  |

Table 2.21 Balloo habeebe irrigation project crop water requirement / design supply/   for crop pepper

| S/N | Designaton | Symbol | Operation | Unit | Months |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Jan. | Feb. | Mar. | Apr. | May. | Jun. | Jul. | Aug. | Sep. | Oct. | Nov. | Dec. |
| 1 | ETO | ETO | PEN MAN | mm/month | 139.81 | 126.56 | 158.41 | 147 | 149.11 | 169.2 | 165.54 | 149.42 | 141.6 | 144.46 | 131.1 | 129.54 |
| 2 | Crop factor | Kc | From Table |  |  |  | 0.30 | 0.40 | 0.85 | 0.95 | 0.70 |  |  |  |  |  |
| 3 | Crop Evap. (pot) | ETcrop | 1x2 | mm/month |  |  | 47.523 | 102.9 | 111.83 | 126.9 | 115.88 |  |  |  |  |  |
| 4 | Rain fall | P | Meteoro Data | mm/month | 15 | 45 | 44 | 63 | 34 | 21 | 120 | 122 | 46 | 9 | 3 | 2 |
| 5 | Effective Rainfall | Pe | 0.8p-25/0.6p-10 | mm/month | 0 | 17 | 16.4 | 27.8 | 10.4 | 2.6 | 72 | 73.6 | 26.6 | 0 | 0 |  |
| 6 | Effective Rainfall | Pe | 0.8p-25/0.6p-10 | mm/month | 0 | 17 | 16.4 | 27.8 | 10.4 | 2.6 | 72 | 73.6 | 26.6 | 0 | 0 |  |
| 7 | TNI Req’t | INT | 3 minus 6 | mm/month |  |  | 63.923 | 75.1 | 101.43 | 124.3 | 43.88 |  |  |  |  |  |
| 8 | Project eff | EP | Calc overall eff |  | 0.45 | 0.45 | 0.45 | 0.45 | 0.45 | 0.45 | 0.45 | 0.45 | 0.45 | 0.45 | 0.45 | 0.45 |
| 9 | TGI Req’t | Lt | 7\8 | mm/month |  |  | 142.05 | 166.69 | 225.4 | 276.2 | 97.51 |  |  |  |  |  |
| 10 | sup req/ha | Qt | 9/259.2 | L/s |  |  | 0.55 | 0.64 | 0.87 | 1.066 | 0.376 |  |  |  |  |  |
| 11 | Irri area | A | Crop pattern | Ha |  |  | 15 | 15 | 15 | 15 | 15 |  |  |  |  |  |
| 12 | supp Reqt | Q | 10x11 | L/s |  |  | 8.25 | 9.6 | 13.05 | 15.99 | 5.64 |  |  |  |  |  |
| 13 | Hr app/d | Hr | Given | Hour | 12.0 | 12.0 | 12.0 | 12.0 | 12.0 | 12.0 | 12.0 | 12.0 | 12.0 | 12.0 | 12.0 | 12.0 |
| 14 | Ratiohours of appl/24 | R | H/24 | Hour | 0.5 | 0.5 | 0.5 | 0.5 | 0.5 | 0.5 | 0.5 | 0.5 | 0.5 | 0.5 | 0.5 | 0.5 |
| 15 | Projectsupply req’t | Q/2 | 12/14 | L/s |  |  | 16.5 | 19.2 | 26.1 | 31.98 | 11.28 |  |  |  |  |  |
| 16 | Des. supp | Q | Max (15) | L/s/ha |  |  |  |  |  | 2.132 |  |  |  |  |  |  |

Table 2.22 Balloo habeebe irrigation project crop water requirement / design supply/   for crop tomato

| S/N | Designaton | Symbol | Operation | Unit | Months |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Jan. | Feb. | Mar. | Apr. | May. | Jun. | Jul. | Aug. | Sep. | Oct. | Nov. | Dec. |
| 1 | ETO | ETO | PEN MAN | mm/month | 139.81 | 126.56 | 158.41 | 147 | 149.11 | 169.2 | 165.54 | 149.42 | 141.6 | 144.46 | 131.1 | 129.54 |
| 2 | Crop factor | Kc | From Table |  |  |  |  |  |  |  |  | 0.4 | 0.75 | 0.85 | 0.8 | 0.6 |
| 3 | Crop Evap. (pot) | ETcrop | 1x2 | mm/month |  |  |  |  |  |  |  | 59.768 | 106.2 | 122.79 | 104.88 | 77.724 |
| 4 | Rain fall | P | Meteoro Data | mm/month | 15 | 45 | 44 | 63 | 34 | 21 | 120 | 122 | 46 | 9 | 3 | 2 |
| 5 | Effective Rainfall | Pe | 0.8p-25/0.6p-10 | mm/month | 0 | 17 | 16.4 | 27.8 | 10.4 | 2.6 | 72 | 73.6 | 26.6 | 0 | 0 | 0 |
| 6 | Effective Rainfall | Pe | 0.8p-25/0.6p-10 | mm/month | 0 | 17 | 16.4 | 27.8 | 10.4 | 2.6 | 72 | 73.6 | 26.6 | 0 | 0 | 0 |
| 7 | TNI Req’t | INT | 3 minus 6 | mm/month |  |  |  |  |  |  |  | 0 | 79.6 | 122.79 | 104.88 | 77.724 |
| 8 | Project eff | EP | Calc overall eff |  | 0.45 | 0.45 | 0.45 | 0.45 | 0.45 | 0.45 | 0.45 | 0.45 | 0.45 | 0.45 | 0.45 | 0.45 |
| 9 | TGI Req’t | Lt | 7\8 | mm/month |  |  |  |  |  |  |  | 0 | 176.89 | 272.87 | 233.0.7 | 172.72 |
| 10 | sup req/ha | Qt | 9/259.2 | L/s |  |  |  |  |  |  |  |  | 0.68 | 1.05 | 0.899 | 0.67 |
| 11 | Irri area | A | Crop pattern | Ha |  |  |  |  |  |  |  | 8 | 8 | 8 | 8 | 8 |
| 12 | supp Reqt | Q | 10x11 | L/s |  |  |  |  |  |  |  | 0 | 5.44 | 8.4 | 7.192 | 5.36 |
| 13 | Hr app/d | Hr | Given | Hour | 12.0 | 12.0 | 12.0 | 12.0 | 12.0 | 12.0 | 12.0 | 12.0 | 12.0 | 12.0 | 12.0 | 12.0 |
| 14 | Ratiohours of appl/24 | R | H/24 | Hour | 0.5 | 0.5 | 0.5 | 0.5 | 0.5 | 0.5 | 0.5 | 0.5 | 0.5 | 0.5 | 0.5 | 0.5 |
| 15 | Projectsupply req’t | Q/2 | 12/14 | L/s |  |  |  |  |  |  |  | 0 | 10 | 16.8 | 14.384 | 10.72 |
| 16 | Des. supp | Q | Max (15) | L/s/ha |  |  |  |  |  |  |  |  |  | 2.1 |  |  |

# 2.28. Management practices of proposed crops

The only way, to increase agricultural production and productivity in small marginal units of farming is increasing productivity and production per unit time and area. This may be achieved by using more productive cultivar and by improving technique of culture, fertilizer use weeds and pests control.

2.28.1. Land preparation of proposed crops

Farm land (seed bed) is the place where the seeds germinate and the medium from which the plants get the moisture and minerals through their roots. Therefore the farm should be in position to supply enough moisture, nutrient and air to all full penetration of the plant roots.

Therefore, farm land should:-

- Not be ploughed when too wet

- Be ploughed right after harvest

- Be reploughed after the short rains when weeds  seeds  have  germinated as, this kills first flush of weeds

### 2.28.2. Establishment methods of proposed crops in the field

Maize Haricot been are established  by direct  sowing in the field  using recommended space between row and  plants , While onion, pepper ,Tomato and others  perennial crops  are transplanted by raising  seedlings  on nursery site and also by direct sowing . Stage of seedling for transplanting determined based on the density of sowing on  nursery site , size of the containers and vigor  of seedlings and  also determined  based on number of days , which recommended  for seedling to reach stage of transplanting on nursery site . For instance seedlings of Onion, Tomato and Pepper may transplant after 30-40, 30-45, and 45 days respectively. During transplanting seedlings should be watered before uprooting and also transplanting in the cloud day, or in the morning or in the afternoon, is recommended for success full of seedlings establishment. Post transplanting care, such as hoeing and weeding of transplanted crops should be carried out on time and also balancing rate of water supply and rate of respiration of transplanted crops should be maintained.

### 2.28.3 Methods of planting and seed rate of proposed crops

To gain the required production and productivity the proposed crops should be planted using appropriate methods of planting and seeding rate .To facilitate irrigation agronomic management practices, row planting methods ,were selected for proposed crops and also recommended seeding rate of proposed crops were given in the following table

.

Table 2.24 seed rate and planting methods of proposed crops

|  | Types of proposed crops | Proposed methods of planting | Spacing |  | Seed rate per hectare |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Between rows(cm) | Between plants(cm) |  |
| I | Belg(Areas) rainy season |  |  |  |  |
|  | Maize | row planting | 75 | 30 | 30kg |
|  | Haricot bean | ‘’ | 40 | 10 | 50kg |
|  | Pepper | ‘’ | 80 | 30 | 0.75kg |
|  | Onion |  | 40 between furrow | 20 between row 10 between plants | 4.5kg |
|  | Avocado |  | 5m | 8m | 300 seedling |
|  | Mango | “ | 8m | 8m | 157 seedling |
|  | Banana | “ | 250cm | 250cm | 1600 seedling |
|  | Papaya | “ | 2.5m | 2m | 2000 seedling |
| II | Maher (Hagaya ) rainy season |  |  |  |  |
|  | Maize | row planting | 75 | 30 | 30kg |
|  | Haricot bean | ‘’ | 40 | 10 | 50kg |
|  | Onion | Double row planting | 40 between furrow | 20 between rows 10 between plants | 4.5kg |
|  | Pepper | ‘’ | 80cm | 30cm | 0.75kg |
|  | Tomato | ‘’ | 70cm | 30cm | 0.5kg |

2.28.4 Fertilization of proposed crops

A healthy agricultural economy depends on productive soils .Soils may be naturally fertile or may be naturally rather infertile and unproductive. Application of fertilizers to the soil is the most common methods, which used to maintain or to replace minerals that are essential for plant growth. As a general rule amounts and types of required fertilizers is determined through soil analysis. However, for this project amount and type of required fertilizer time of application required for proposed crops were recommended depending on blanket recommendation of the country by using different literature review.

Table 2.25 required type and rate fertilizer with its time of application

| S.no | Types of proposed crops | Required fertilizers | Application rate per hectare (kg) | Time and mode of application |
| --- | --- | --- | --- | --- |
| 1 | maize | DAP Urea | 100 100 | -Side dressing at time of sowing -Split application at four leaf stage and at knee-stage |
| 2 | Onion | DAP Urea | 100 150 | -Side dressing or direly dressing in the hole where the seedlings were transplanted and mix with soil at time of transplanting -Top dressing after transplanting Within 30-40 days. |
| 3 | Haricot bean | DAP | 100 | -Basal application at time of planting |
| 4 | Pepper | DAP Urea | 200 100 | -Side dressing or directly dressing in the hole and mix well with soil at time of transplanting -Top dressing at early flowering stage. |
| 5 | Tomato | DAP Urea | 200 150 | -Side dressing or directly dressing in the hole where the seedlings were transplanted and mix well with soil at time of transplanting. -Top dressing at early flow wiring stage |
| 7 | Avocado | Apply farm yard manure to improve structure and aeration of the soil but highly fertile soil favours vegetative growth &reduces flowering &fruit set. | - | . -Mix it up with soil |
|  | Mango | Apply farm yard manure to improve structure and aeration of the soil but highly fertile soil favours vegetative growth &reduces flowering &fruit set. | - | . -Mix it up with soil |
| 9 | Banana | Apply farm yard manure Or Organic matter | - | -Mix it up with soil and then cover it with mulch -Applied at 50 cm from the stool |
| 10 | Papaya | Apply farm yard manure Or Organic matter | - | -Mix it up with soil and then cover it with mulch |

Source –Different literature review

2.28.5 Irrigation methods of proposed crops

Irrigation water may be applied to crops by surface irrigation, subsurface irrigation, sprinkler irrigation and drip irrigation methods. In surface irrigation methods, water is applied directly to the soil surface from a channel located at the upper reach of the field. It is the oldest and the most extensively used methods of irrigation in the world. Water is distributed by the gravity over the surface and applied to the soil by different means. Common methods of surface irrigation are border strip, Check basin, Basin and furrow irrigation. Each method has their  own advantage and disadvantage. Depending on natural circumstances of the communed area (slope and soil type) type of proposed crops and previous experience with irrigation, furrow irrigation Basin irrigation and border strip irrigation methods were selected to irrigate proposed crops.

2.28.6 Irrigation scheduling of proposed crops

It is the process of determining how much water to apply per irrigation and when to irrigate. Proper scheduling is important for the efficient use of water, energy and other production inputs. It allows irrigation to be co-ordinate with other farming activities.

How much water should be applied per irrigation or depth of application calculated by the following formula.

D= (p.sa) D/ Ea

Where:

Sa - the total available soil water

P- The fraction of the total available soil water which can be used by the crop without affecting its growth

D- Root depth (m)

Ea – application efficiency

When to irrigate (irrigation application interval) can calculated by the following formula

Irrigation interval (i) = (p.sa) D / ETcrop

Table 2.26 irrigation methods and interval of proposed crops

|  | Types of proposed crops | Recommended methods of irrigation |  | Irrigation interval (days) |
| --- | --- | --- | --- | --- |
|  |  | Furrow | Border strip |  |
| 1 | Maize | Furrow | Border strip | 14 |
| 2 | Haricot bean | Furrow | Border strip | 8 |
| 3 | Onion | Furrow | - | 7 |
| 4 | Tomato | Furrow | - | 7 |
| 5 | Pepper | Furrow | - | 8 |
| 7 | Mango | Furrow | Basin | 7-20 |
| 8 | Avocado | Furrow | Basin | 7-20 |
| 9 | Banana | Furrow | Basin | 7-15 |
| 10 | Papaya | Furrow | Basin | 10-15 |

2.28.7 Improving water use efficiency of proposed crops

Water consumed for each unit of yield is shows water use efficiency of that crop from which, that yield is obtained. In order to use water (moisture) effectively, if there is shortage of moisture in the soil; improving water use efficiency of proposed crops can be attained through.

- Using improved varieties, that is using improved cultivar which resist drought and matured early

- Using of fertilizer

- Using appropriate planting standard

- Timely planting

- Timely weed control

### 2.28.8 Maintaining soil productivity of project area

Maintenance soil productivity and quality of soil is a crucial and assumes as greater importance in the context of sustainable agriculture .soil productivity might be maintained by using so many options. For instant, we can maintain soil productivity through the following options.

### By using crop residue management

Crop residue represents a valuable resource in terms of plant nutrients. crop residue , if placed on surface as much and  incorporated in to  the soil after it decomposed in subsequent years, it used  to:-

- reduce soil erosion

- improve rainfall infiltration

- Modify the movement and retention of water  and air in the soil

- Prevent surface crusting

- Maintain soil structural stability

- Recycling plant nutrients particularly nitrogen and phosphorus

- Reduce evaporation during the initial  stage of the crop growth

### By using of farm yard manure (fYM) and commercial fertilizers

Using farm yard manure and commercial fertilizers increases soil fertility to satisfy the needs of plants and also increases organic content of soil as a result structure, organic content and PH of the soil is improved.

### By using soil conservation and management

Soil erosion control using different techniques (physical and Agronomic technique) is parts of soil conservation and management. Controlling soil erosion, reducing losses of water from the root zone, enhancing soil fertility and improving and sustaining ergonomic productivity are, all encompassed in soil conservation.

2.28.9 Crop disease control methods of proposed crops

Data obtained from district’s pastoralist development office and societies of project area show that, there are different types of crop diseases exist in the project area .Recommended crop disease control options were given in the table below this

Table 2.27 Crop disease and recommended control methods

| Types of proposed crops | Existing crop disease | Recommended control methods |
| --- | --- | --- |
| Maize | Maize rust, leaf blight, head smut | -Seed dressing ,crop potation ,use of resistant varieties |
| H. bean | Bacterial blight leaf spot Bean rot | -Using resistant varieties ,crop rotation -Using disease free seed |
| Onion | -Purple blotch -downy mildew - anthracnose | -Crop rotation, plant hygiene -Removal and burning of onion stubble after harvest |
| Pepper | -Anthracnose, fruit rot -powdery mildew ,bacterial leaf spot , leaf curl virus | - Avoiding planting during times of heavy rains - Destroying crop residues - Using resistant varieties -seed treatment with copper based fungicide at the rate of 1 to 2 table spoon full in 4 liter of water |
| Tomato | Early blight, late blight, Septoria leaf spot, wilt, fruit rot | Using resistant varieties Crop sanitation Crop rotation |
| Avocado | Stem end rot Phytohptora fungis | -Using resistant varieties - Destroying crop residues |
| Mango |  | -Using resistant varieties - Destroying crop residues |
| Banana | Leaf spot disease called black sigatoka | -Using resistant varieties - Destroying crop residues |
| Papaya | -Phytophtraroot rot Papaya ring spot virus Stem end rot. | -Using resistant varieties - Destroying crop residues |

### 2.28.10 weed management options of proposed crops

Weeds affect production and productivity of cultivated crops through, competition of growth factors (moisture C02, sun light, space and nutrient), Harbor disease and insect pest as alternative host and create inconvenience for harvesting. To minimize these effect weeds should be controlled by using existing weed control options depending on the condition of proposed crops and types of existing weeds. From existing weed control options, using mechanical  weed control methods(proper plowing , Hand pulling , hoeing , slashing , sanitation ,crop rotation , using resistant varieties ) is highly recommended , while chemical weed  control option was recommended as last option for certain  crops and weeds.

2.28.11 crop pests (insect, wild animals and birds) control methods of proposed crops

Data obtained from district’s pastoralist office and societies of project area show that, there are different types of insects, wild animals and Birds attack cultivated crops in the field starting from seedling as well as in the storage. Some of crop pests found in and around project area and their recommended control options were given in the table.

Table 2.28 crop pests and recommended control methods

| Typesofproposed crops | Insect pest | Recommended control methods |  | Application rate of chemicals | Existing wild animals pest | Control methods |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Cultural control methods | Chemical control methods |  |  |  |
| Maize | Stalk borer Africanworm, maize aphids army worm | Destroying the previous crop residues | -Application (inside each leaf funnel of the plant ) of -Cypermetrin 1% granuler or - Endosulfan 3% granular or - Diazinon 5% granular Application of Endosulfan 35 % EC | 2.5- 5 kg /ha | Monkey, Baboon , pig Hedgehog Birds | Guarding |
| H .bean | Aphids Jassids African ballworm | - | Spraying marshal 25% Ec Application of Endosulfan 35% Ec | 2 lit/ha 1.5 lit/ha |  |  |
| Tomato | African bollworm Cut worm | - | Very selective application of insecticides based on established thresh holds | Tomato |  |  |
| Onion | Trips,Cutworm onion fly army worm | - | Endo sulfan 35% Ec Cyper metrine | 1.5 lit/ha 50-75 gm a.l/ha 3-4 times |  |  |
| papper | Thrips ,white fly African ballworm | - | Spraying of Endo sulfan 35% Ec | 1.5 lit /ha |  |  |
| Avocado |  |  |  |  |  |  |
| Mango | -Mangostone weevil -Fruit flies -Scale -Mites | -Destroying the previous crop residues Using resistant varieties |  |  |  |  |
| Banana | Borers Nematods | -Destroying the previous crop residues -Using resistant varieties |  |  |  |  |
| Papaya | -Fruit flies -Leaf hopper | -Destroying the previous crop residues -Usingresistant varieties |  |  |  |  |

# 2.29. Labour requirement of proposed crops

Labor is a key inputs of agricultural production particularly in irrigated agriculture especially for hoeing cultural practices. Required labor requirements of each proposed crops was given below this in table.

Table 2.29 proposed labour requirements for Agronomic practices.

| Crop | Cultural practice | Frequency | Required man day | Oxen day (pair) |  |
| --- | --- | --- | --- | --- | --- |
| Maize | Plowing | 4 | 8 |  | 8 |
|  | Fertilizing | 1 | 2 |  |  |
|  | Sowing | 1 | 10 |  | 8 |
|  | Irrigation | 6 | 4 |  |  |
|  | Hoeing | 1 | 16 |  |  |
|  | Weeding | 1 | 10 |  |  |
|  | Harvesting | 1 | 16 |  |  |
|  | Thrashing and bagging | 1 | 11 |  |  |
|  | Loading and un loading | 1 | 4 |  |  |
| Haricot bean | Plowing | 4 | 8 |  | 8 |
|  | Sowing | 1 | 10 |  | 8 |
|  | Fertilizing | 1 | 2 |  |  |
|  | Irrigating | 8 | 4 |  |  |
|  | Weeding | 1 | 12 |  |  |
|  | Harvesting | 1 | 16 |  |  |
|  | Threshing and Bagging | 1 | 7 |  |  |
|  | Loading and unloading | 1 | 4 |  |  |
| Tomato | Plowing | 4 | 8 |  | 8 |
|  | Fertilizing | 1 | 2 |  |  |
|  | Transplanting | 1 | 20 |  |  |
|  | Irrigating | 7 | 4 |  |  |
|  | Hoeing | 3 | 20 |  |  |
|  | Harvesting and Bagging | 1 | 30 |  |  |
|  | Loading and un loading | 1 | 5 |  |  |
| Onion | Plowing | 4 | 8 |  | 8 |
|  | Fertilizing | 1 | 2 |  |  |
|  | Transplanting | 1 | 20 |  |  |
|  | Irrigation | 8 | 4 |  |  |
|  | Hoeing | 4 | 20 |  |  |
|  | Harvesting and Bagging | 1 | 30 |  |  |
|  | Loading and un loading | 1 | 4 |  |  |
| Pepper | Plowing | 4 | 8 |  | 8 |
|  | Fertilizing | 1 | 2 |  |  |
|  | Transplanting | 1 | 20 |  |  |
|  | Irrigating | 7 | 4 |  |  |
|  | Hoeing | 4 | 15 |  |  |
|  | Harvesting and Bagging | 1 | 15 |  |  |
|  | Loading and un Loading | 1 | 5 |  |  |
| Mango | Plowing | 4 | 8 |  | 8 |
|  | Fertilizing | 1 | 2 |  |  |
|  | Transplanting | 1 | 20 |  |  |
|  | Irrigating | 7 | 4 |  |  |
|  | Hoeing | 3 | 20 |  |  |
|  | Harvesting and Bagging | 1 | 30 |  |  |
|  | Loading and un loading | 1 | 5 |  |  |
| Avocado | Plowing | 4 | 8 |  | 8 |
|  | Fertilizing | 1 | 2 |  |  |
|  | Transplanting | 1 | 20 |  |  |
|  | Irrigating | 7 | 4 |  |  |
|  | Hoeing | 3 | 20 |  |  |
|  | Harvesting and Bagging | 1 | 30 |  |  |
|  | Loading and un loading | 1 | 5 |  |  |
| Banana | Plowing | 4 | 8 |  | 8 |
|  | Fertilizing | 1 | 2 |  |  |
|  | Transplanting | 1 | 20 |  |  |
|  | Irrigating | 7 | 4 |  |  |
|  | Hoeing | 3 | 20 |  |  |
|  | Harvesting and Bagging | 1 | 30 |  |  |
|  | Loading and un loading | 1 | 5 |  |  |
| Papaya | Plowing | 4 | 8 |  | 8 |
|  | Fertilizing | 1 | 2 |  |  |
|  | Transplanting | 1 | 20 |  |  |
|  | Irrigating | 7 | 4 |  |  |
|  | Hoeing | 3 | 20 |  |  |
|  | Harvesting and Bagging | 1 | 30 |  |  |
|  | Loading and un loading | 1 | 5 |  |  |

# 2.30. Conclusion and recommendation

Project area is belongs to dry low lands agro-ecological zone. Farming system of the project area is characterized by crop and livestock production system. Project area also has two rain seasons, what call locally arfasa and hagaya. Result of agronomic feasibility study show that, project area known by cultivating low land crops, low production and productivity, limited crop pattern and failure of crop production due to so many limiting factors of crop production mainly due to climatic difficulties .In addition to these result of study show that, project area known by different potential factors of crop production to cultivate lowland crops. These suggest that  developing modern small –scale irrigation project is one of the important option to, minimize existing limiting factors  of crop production , increase numbers of cultivated crop pattern , increase production and productivity.  Also it is necessary to use existing potential factors of crop production of project area efficiently. In general, result of agronomic feasibility study show that, project area is suitable to cultivate annual and perennial irrigated crops through developing modern small-scale irrigation

# 3. SOIL and land suitability

Introduction

Soil-   water -  plant – relationship.

Plants grow on the soil that   provide   them water and nutrients.   They  absorb  the  water  from  the  soils  mainly  through  roots  to  build  their  vegetative structures  and  performing  various  physiological  and   bio-chemical  activities.  Between  soil, water  and  plant relationship  should  be  clearly  understood  to  decide upon  the  time  and  depth  of  irrigation,  to  make  the  most  efficient  use  of  irrigation  water. An excess orless water reduce the yield of production.

3.1 Methodology of soil study

According  to  soil  study,  soil  map    survey  was  conducted  to  identify  soil  units  and  series.  Field  observation  and  soil  physical  investigation  was  carried  out  through  visual   observation.    As observed physically, the soil porosity is enough to permit the water into the plant roots.  Dug soil   pits and auger tests randomly have been done.  From  the  command  area through  cross  section,  a  soil  samples  was taken  a depth  of  100cm.  The  soil  samples  collected  from  the  command  area  tested physically as well as  in  soil laboratory  for  detail chemical  analysis in Oromiya Water Work Design and Supervision Enterprise.

3. 2 Land   use and farming system

The command area is cultivated   more than 97%.

The area is covered   with different type of perennial crops. All      farmers   are using   traditional irrigation system. The soil fertility of  cultivated area is declined  due to care of it.The watershed  area  is  deforested  and  over grazed   for  this matter the cultivated  and watershed area are exposed  for danger erosion.

# 3.3 Soil physical   properties

For  better  understanding  of  soil,  water  and  plant  relation,  it  is  very  important  to  estimate  of  soil  physical  and  chemical  properties,  which  are  important from   point  of  view.

3.4 Major aspects of irrigation   crop production

The  crops  recommended   for  production   must  be  selected  based  on  the  following  criteria

~ Agro-climate condition.

~ Food habits of the community.

~Soil fertility and suitability for crops grown.

~Market value of the crops.

3.5To use Suitable Soils

Furrow irrigation can be used on moist soil types. However, as with all surface irrigation methods, very coarse soils are not recommended as percolation losses can be high. Soils that crust easily are especially suited to furrow irrigation because the water dose not flows over the ridge and so the soil in which the plants grow remains friable. The selected command area, the soil must be treated with organic fertilizer. The farmers must use economically water during irrigate the crops.

3.5.1 Organic fertilizer application

The  need  for  organic  fertilizer application  is  to  improve  nutrients  holding  capacity  of  the  soil  and  to  decrease  specific  toxicities  and  to  achieve  optimum  crop  production .

This  practice  gives  better  yield  production  and  improves  soil  productivity .It must  be added  regularly  to  provide  necessary  nutrients and to improve  soil structure and  increase  water retention capacity.

3.5.2   Crop rotation

Continuously  growing  the  same  crops  reduce  soil  fertility  which  remove  certain elements  or nutrients  from   the  same  soil  depth. Growing  different  crops  one  after  another  on  the  same  land, e.g.   After cereal legume crops planted and others.

-Building up soil fertility

-Building up organic matter and improving the

Physical condition of soil..

3.5.3 Intercropping

Intercropping means growing two or more crops at the same time. Intercropping  can  provide  the  required  ground  cover  while  also  improving  soil  fertility  when legumes  are used. Intercropping also allows for intensive land use where land holdings are small.

3.5.4      Soil fertility management

The  focus  of  any  soil  fertility  replenishment  should  be  the  integrated  nutrient  management  involving  the  growing  leguminous  and  agro-forestry  as  well as  technologies  that reduce the risks of acidification and salinization. Soil  fertility  replenishment  should  be  considered  as  an  investment  in  natural  resource  capital.

3.5.5 Soil and water management/Conservation

To   improve   soil  fertility   and crop productivity contour  tillage  and  tied  ridging,  minimum  tillage,  crop  rotation  and  cover  cropping  must  be  done  on  cultivated  land .

The  objective  of  any  soil  and  water  management exercise  should  be  to  improve  soil  structure,  increase  soil  and  water  storage,  increase  infiltration  rate  and to minimize   soil  water  loss  by  evaporation .

3.5.6 Soil conservation on cultivated land

Soil  conservation  is  a  set  of  good  land  husbandry  practice  and  techniques  to  protect  soil  and  water  resources  that  sustain  agricultural  production.                                                                        Soil and water conservation, soil moisture management, drainage

of  surplus  water  and  harvesting  of  water  under  dry  conditions,  as  well  as  to  enhance  soil  fertility  to  ensure  that  land  productivity  is  sustained    and to  improve  indefinitely.

3.5.7 Agronomic and vegetative measures

Vegetative  measures prevents  the  direct  impact  of  rain  drops  on  bare  soil, Specially on watershed area  reducing  splash  erosion.  It reduces evaporation from the soil surface hence conserving soil moisture.

-Water  running  down  slops  is  slowed  down, thus  reducing  the  erosive  power

-The wind cannot blow the soil away.

-Soil  structure  and  soil  fertility  are  improved  as direct  effect  of  the  organic  matter.

-In Ethiopia every year more than one billion tons of soil eroded.

3.5.8 Soil   conservation techniques

Such measures are grouped as follows

~Agronomic and vegetative techniques.

~ Physical   techniques such as wooden and stone check dams.

3.6 Use of organic manure

Using   compost  such  as  green  manure  and  farm  yard manure,  besides  supplying  some  of  the  required  plant  nutrients,  this  practice  also  improves  soil  structure  through  supply  of  organic  matter,  which  improves  the  water  holding  capacity  of  the  soil.

3.6.1 Soil   management

Incorporation of organic materials (plant residues, animal’s manures etc) that supply decomposition products that stabilize soil aggregates

Adoption of suitable cropping systems that increase and maintain the organic matter level of the soil (application of fertilizers)

3.6.2 Agro-forestry for environment and soil conservation.

Agro-forestry  has  the  potential  for  increasing  the  organic  matter  and  nitrogen  content  of  soil.

Agro-forestry prevents soil erosion conserve soil and water.  Improve soil fertility and microclimate.

3.6.3 The benefit of agro-forestry on the soil include;

Addition of nutrients through nitrogen fixation, green leaf manure and root decomposition

Increased soil water retention through organic matter wind break and shade trees.

Soil erosion control through roots, leaf litter on the soil surface and wind                             breaks and .Reduction of soil acidity through addition of bases in tree litter.

Reduction of salinity and sodicity Improve soil structure and infiltration rate and replace sodium ions.                                                                                                                                                                  Growing of salt tolerant woody species such as Prosopis julifloraa and Sespania that          improve soil quality.

3.6.4 Contour Ploughing/ Tillage

Contour  ploughing can be  used  in combination with  contour strip  cropping  to reduce soil erosion.

Along  with  contour  sloughing  it is  also  possible to  introduce  the  practice  of minimum tillage  in order  to  minimize  the rapid  break down  of soil  structure  to reduce  the credibility  of the  soil. The problem of soil  losses  through  surface  runoff is one of the major limiting factors in agriculture production  to day.

3.7 Land  degradation

Land  degradation  mostly  caused  mainly  by soil  erosion  (sheet  and  gully  erosion). At the  project  area  there  is  poor  land  management  practice,  deforestation  (agricultural  land  expansion) , forest  burning  and  over  grazing ,  the  removal  of  soil  by  rain,  during  rainy  season  and  the  removal  of  soil   by  wind , during  dry season.  It is common at  the proposed  command  and  watershed  area.

3.7.1 Low crop production

Low organic matter content

Low soil water storage

Low soil fertility

Seasonal soilmoisturedeficits                                                                                                       High evaporation rates from bare soils

# 3.8 Basic soil—water relation

The  soil  is  composed  of  three  major  parts  air,  water and  solids . Important ofsoil characteristics in irrigated agriculture.                                                                                          The water holding or storage capacity of the soil.

The permeability of the soil to the flow of water and air.

The physical feature of the soil like organic matter

Content, depth texture and structure.

According to soil physical test, the soil is dominated by

Sandy loam soil and it needs adding some more organic matter.

3.8 Soil depth

It  is  very  important   criteria  for  selecting   sustainable  and  suitable  land  for  irrigation  use.  The selected project of this command area has poor soil type. Some parts of the command area is not suitable for deep rooted crops, unless the soil treated with organic fertilizers (like composed )

3.8.1    Soil texture

The soil   texture of the command area is dominated by sandy loam soil.  According  to  soil  physical  analysis,  sequentially,  soil  texture  is as  follows  sandy and sandy loam soil simultaneously the soil fertility of the command area is  declined.

3.8.2 Soil structure.

Good soil structure allows plant growth factors to function at optimum efficiency. It refers to the arrangement of primary and secondary particles. Soil structure influences soil air moisture regions and hydraulic conductivity. As it is observed physically at field the soil is black sandy loam in nature generally the soil structure is medium for agricultural irrigation.

3.8.3 Soil color

The soil color differs with variation of soil types.

Different  factors   such  as  organic  matter  content,  organic  carbon  and  parent  materials  are  much  responsible  to  determine  the  soil  color. The  soil  color  of  the  command  area  is  week red,   black,   brown,   and  as  it is described  in the  brief  profile  description.

# 3.8.4 Description OF INDIVIDUAL SOIL PROFILE

Soil profile number 1(pit 1)

X&Y coordinate

X-632265.38E

Y-816404.12N

Altitude 1691.16

Slope                      gentle land

Surface                  stone or sandy loam soil

Erosion at site        there is   sheet erosion

Land use                    cultivated

Brief soil profile description

0---70 cm color week red, friable (moist) ,  and common  fine  roots.

70---100cm   color brown, some root hairs sandy loam.

Soil profile number 2(pit 2)

X & Y coordinate

X-632265.38E

Y-816404.12N

Altitude 1664.61

Land use                cultivated

Slope                       gentle land

Surface                    most of the soil is sandy loam

Erosion at site    in some areas there is sheet erosion.

Brief soil profile description

0---55 cm color dark (moist), sandy loam and many root hairs

55—100cm color dark brown rarely root hairs sticky and wet

Soil profile number 3(pit 3)

X & Y coordinate

X-632265.38E

Y-816263.54N

Altitude-1646.56

Slope                           gentle land

Land use                     cultivated

Surface                     most of the soil sandy loam

Erosion at site           there is sheet erosion

Brief soil profile description

0—60cm dark grayish, sandy loam, root hairs and moist                                                           60—100cm dusky red, moist and wet

Soil profile number 4(pit 4)

X & Y coordinate

X-631617.05E

Y-816531.68N

Altitude-1679.43

Land use                    cultivated

Slope                        gentle land

Erosion                     there is sheet erosion

Surface                     sandy soil

Soil profile description

0—65 cm dark gray, root hairs sandy loam and moist soil porosity and water permeability is good

65—100cm brown,moist and wet.

Soil profile number 5(pit 5)

X &Y coordinate

X-631695.33E

Y-816461.31N

Altiude-1666.33

Land use            UN cultivated

Surface               sandy and sandy loam

Slope                  gentle land

Erosion             there is sheet erosion

Brief  soil profile  description

0—55cm dark red root hairs sandy loam and moist soil.

Soil porosity is good for permeability water and air                                                                   55—100cm dusky red moist and wet.

3.8.1   Modern Irrigation Mission

Modern irrigation mission is to provide sustainable solution for ending hunger and poverty by using modern agricultural irrigation science and technology.

# 3.8.5   Soil chemical characteristics

General

Soil  chemical  properties  have  a great influence  on planning  soil and agronomic  programs best  suited  to  the proposed   irrigation  project area of  the schemes. The soil chemical  characteristics  of  the soil mapping  units  have  analyzed  based  on laboratory  analytical  results of  soil samples  collected  in the  field.

3.9.Acidity and alkalinity

Soil reaction (PH—H2O 1: 2.5)

Soil  PH is  important  as it is  an indicator  of  acidity  and  alkalinity  in the  soil. The PH  of  the  soil  determines  availability  of  macro  and  micro nutrients  in  the  soil as  shown  below.

PH         Rating                                     Comments

6.0-7.0   Slightly alkaline                     Fe may be deficient .Optical

Availability of P

7.0-8.5   moderately alkaline                 Low available of P and micro

nutrients .

8.5>     Alkaline                                  only few crops grow excess of

Na deficiency of  P  and  micro

Nutrients toxicity  of  Boron

Nitrification of OM,Ca may need

correction.

The PH of all soil mapping units are available of the project area between 8.2-8.7 moderately alkaline. Low available of  P and micro nutrients as well as OM and Ca need for correction.

3.9.1  Electrical  conductivity (ds/m)

Electrical conductivity measurements (EC) are used as indicators of total quantities  of soluble salts in soil. General inter predation  of  Fe values in soil mapping and land evaluation  is as follow

Rating                   ECmmhos       Comments

Salt free                  0-2                 Salinity effects are mostly negligible

Slightly saline          2-8               Yield of many crops restricted

Moderately saline     15                  Only tolerant crop yield satisfactory

Strongly saline         >15               Only very tolerant crops yield satisfactory

The EC value of all soil types in the irrigable project area is  0.13-0.23  salinity effects are mostly negligible  ( free from salt)

3.9.2 Caution Exchangeable Capacity (CEC) and Base Saturation (BS)

Caution Exchange capacity (CEC) values are indicators of the potential inherent fertility of a soil and possible response to fertilizer application. Rating of CEC is given bellow

CEC(me/100g)     Rating             Comments

>40                          v. high               Good agricultural soil

25-40                      high                    As above but minor amendment

is required

15-25                      medium               Major amendment required

5-15                       low                       Moderate to poor response to

fertilizer

<5                           v. low                      Poor agricultural land

The minimum CEC values recorded  in the soils of the irrigable project area is                                                                                                                                                                                                                                     47.7 meg/100gm of  soil  for SMU and maximum  is 59.34 meg/100gm   for SUM  that indicating v. high  natural  fertility of the  soil but in some parts  of the  command  area the soil fertility is poor because the  top soil eroded.

The calculated base saturation range of all soil mapping units arebetween 80--104%which indicates good agricultural   soil asshown below

BS                             Rating

<20                            low fertility

20-60                         medium fertility

>60                             good fertility but there is lack of some nutrients.

3.9.3Exchangeable Cautions (E.C)

The levels of exchangeable cautions in the soil indicate the existing nutrient status and can also be used to assess balances amongst cautions. Many effects on soil structure and on nutrient up take by crops are influenced by the relative contractions of cautions as well as by their absolute levels.

Rating of exchangeable caution (meg/100g of soil) is as follow

Ca           Mg                      K                                      Na

Very high             >20          >8                    >1.2                                 >2.0

High                    10-20        3-8                   0.6-1.20                          1-2

Medium               5-10          1.5-3                0.3-0.6                            3-07

Low                     22-5           0.5-1.5            0.1-0.3                             1-0.3

Very low              <2              <0.5                <0.1                                  <0.1

The value of calcium(34.73—45.61magnesium(8.27-10.35),potassium(0.97—1.19)     ,Sodium(0.06--0.11) The values of  Ca & Mg  are in a range of  high to very high, but K &Na are in a range of  very low. Hence there is an urgent  need for application  of K &Na.

3.9.4 Organic Carbon (% O.C)

Organic Carbon of the soil in  the  project area is between  1.41--2.34%   which is at the range of low,  addition of plant residues and other organic matter  is needed  to improve soil fertility.

Rating                                       %OC

Very  high                                   >20

High                                            10-20

Medium                                       4-10

Low                                           0--4

3.9.5  Organic Matter (% O.M)

Organic Matter influences  soil structure ,water infiltration ,soil bulk density permeability and plant growth  and root development. The range of organic matter

is in between 2.43--4.03%  medium to high .

Rating                                    %OM

V.high                                    >5

High                                       3-5

Medium                                  2-3

Low                                         1-2

3.9.6  Total  Nitrogen

Total Nitrogen  is an indicator  of the soil potential  for  the element  not the measure  in which  it becomes  available to  the plants.

Rating  for evaluation  of the soil  for total  Nitrogen  is  as flow

Total  N%                               Comments

< 0.03                                        low

0.03-0.06                                   medium

> 0.06                                        high

The  total  Nitrogen  of the major  soil  units  in Baloo Hbeebe irrigable project area ranges  from 0.19--0.32  high  category which indicates  high natural fertility of soil.

3.9.7 Available  Phosphorus

Available Phosphorus is the amount of phosphorus readily  available for nutrient  absorption  by the plant roots.

P(pmm)           Rating             Comments

>15                 High                 Fertilizer response unlikely

5-15                medium          Fertilizer response probable

<5                  low                  Fertilizer response most likely

Available Phosphorus of the command area is   2.24—40.04 indicating that  low to high phosphorus fertilizer  response. Hence there is un urgent need for application of  P.

3.9.8  Soil texture  (particle analysis)

The surface layers and upper horizons of soils are sandy loam textured.  The maximum of the soils in the project area is categorized in to sandy and sandy loam textured soil .The soil is none calcareous and is free of salinity and acidity problems.  The soil water holding capacity and ability to store nutrients are medium.

3.10.0  Suggestion  and  conclusion

Soil  and  land  suitability  for  irrigation,  soil  physical  investigation  and  auger tests  has been  done  randomly from  different directions of  the command  area

Land use and farming system has been investigated.  Soil texture, soil structure and  soil  color has been  identified  through physical  tests.

From  the command  area soil  samples  has been  collected and tested  in  soil laboratory  for  chemical  analysis. According to soil tests  in the laboratory,  the soil is free from  acidity and  salinity, but  there is shortage of Potassium, K and Ca which it need urgent application

To  increase  crop production  and productivity  the farmers  able to use more composed (Organic Fertilizer) .If there is shortage  of  composed   they have to use  artificial  fertilizer (DAP  and  UREA).  Over all   Baloo Hbebee irrigable project area, the command and the surrounding (watershed)area  inherited by series soil erosion  within poor  soil and water management.                                                                                                                                           Generally,the modern irrigation mission is to provide sustainable                                                                                                                                                Solution for ending hunger and poverty with the help of integrates modern irrigation agriculture.

# 4. WATERSHED MANAGEMENT

4.1. EXECUTIVE SUMMARY

Developing the watershed area may result to form the suitable: -

- Land forms

- Land structure

Again developing the watershed area also includes factors affecting land farms and its structures these factors are: -

- Erosive by eroding agent (raindrop. Wind and flood)

- Erodeblity due to soil texture and structures

- Slope of the land due steepness and length

- Nature of the land covers (i.e forest, bush’s, shrub, and trees)

The watershed development include the conservation measures for factors affecting land forms and its structures which we call integrated watershed conservation measures i.e

- Bio-physical soil and water conservation techniques

- Gully rehabilitation.

- Rehabilitation of degraded lands

- Improvement of forest pasture / shrub land

4.2. BACKGROUND

From the community upon interviewing point of view the general background of the watershed summarized as follows. Its Ecologically rich (i.e. bushes and shrubs) suitable climatologically parameter (i.e temperature, wind speed, solar radiation, humidity and rain fall) and there are not suitable land forms i.e more degraded land, erosion hazard and gully formed.

4.3. OBJECTIVE

The expected achievement to which the overall watershed management practice are;-

- To control the environment and their negative impact such as :-

* Desertification

* Ozone depletion

* Draught and famine problem

- For the safety of different structure per their designed life time by controlling different factors such.

- Erosive

- Erodablity

- Deforestation

- Overgrazing

- Fuel wood other factors

4.4. NEED OF WATERSHED MANAGEMENT

The developed watershed management will describe / shows different land forms i.e land quality, land suitability land structures and its characteristics whether it is exposed for erosive of eroding agent (i.e Rain drop, wind, floods and etc), erodability of the soil due to soil texture and structure, slope of the land (i.e steepness and length) and the nature of the land cover (i.e forest, shrubs and trees).

It also describes the type of technical requirement and treatment for present landforms and used to control environment and its ecological and climatic parameters as well.

4.5. METHODS FOR WATERSHED MANAGEMENT STUDY

- Detailed field observation of the project area with the survey crew about:-

- Topography of the area

- Geology of the area

- Land quality and suitability

- Land characteristics

- Ecological and climatic parameters

- Physical structures and biological intervention.

- House hold and group interval of farmers about

- Social

- Economical and

- Ecological effect in the watershed area

- Key informant interviews: - about Vulnerability of the watershed area for

- Erosion hazard

- Flood effect

- Rill and gully formation

- Deforestation and

- Over grazing problems

# 4.6. PROPOSED WATERSHED AREA

General description of the watershed area.

Balo Habebe SSIP is located in Oromiya regional state of Bale Zone, Gasara district in Balo Habebe P.A. It is about 60 km from zonal capital city Robe and 7 km from district center Gasara town. It is average altitude 1880m above m.s.l that can be categorization high land ecological region . The land formation of the upstream watershed project area is (mountainous). The land use of downstream land is used for grazing land (under controlled condition) farm land, village and small area of the land is covered by forest.

On the other land, most of the downstream land slope is plain and gent with average slope of

(0-10%)  in the downstream area , certain land formation in the command area has irregularity in  slope  the expected achievement, goal to which the overall watershed management practice  under  this project is that the project is intended to contribute a good idea (knowledge )  for  community above  the natural resource  by integrated agricultural practice . The long rainy season (wet summer, afrasa) from March to June while the short rainy season (winter, Ganna) from September to the beginning of November  .The temperature range from 15.6oc– to 28.5oc .The dry months  of the  catchments are from late December  to early march ..

The agro ecological zonation of the area is estimated that categorized under arid zone as discussed above from place to place most of soil type visible in the area is black in colour . The soil erosion in  the area is found at sheet erosion level which can  be  easily controlled by farm mechanism  for land , used most past  of the land found  in the  catchments is raged and plain .

Most vegetation type of the area is shrubs. The species compositions of the vegetation are trees and shrubs.

Geographical position of the watershed area.

## VERTICAL EXTEYSION

Lower end: X = 631423.001m E longitude

(At weir site)Y = 819968.692m N latitude

Z= 1386.939m

SHAPE AND SIZE OF THE WATERSHED AREA.

The shape of the watershed is nearly the same with Ethiopian map, with surface area of about 25.22 km2 and lengths of5.6km as measured from 1:50,000 scale topo sheet of Gasara

4.6.1. TOPOGRAPHY.

Even if the major features of the catchments area are dominated by mountains land forms, the district also characterized by other different land forms i.e  Raged, Gorges, plains, ridge hill and villages are included the area also prominent with different land slopes i.e Gentle slope, steep slope and plains respectively in order of their prominent. Generally the land forms in the catchments area is characterized by very less fractured and cracked land forms as well as very less degraded land, gully and gorges.

4.6.2. CLIMATE

Climatically the whole woreda is characterized by climatological zonation (i.e  Weyna dega and kola)

4.6.3 SOIL

The main types of soil of the area are reddish sand and on some places black clay is also visible. On the area covered by black clay weathered basalt is exposed at low depths, showing newly generated soil from the basalt.

4.6.4 VEGETATION COVER

The type of vegetation cover in the catchments area include different Acacia forest, bush , hills and mountainous that contains different species such as xadacha, (Acacia tortillas) Haroressa (Gerbi, batula,  Koso, Birbisa, Galo, Bekenisa,Hammerless and others. The dominant species of trees and shrubs that cover the large area of the catchments locally called ‘’ Gerbi’’, “Bekennisa” In contrary to this the dominant grass that cover large percent of the area locally known as ‘’ Batula ‘’ (source Gasara DA , office).

4.6.5. PRESENT LAND USE OF THE CATCHMENTS AREA

Due to non uniformity of the land forms i.e land structure and its characteristics it makes to be responsible for different land use type depending on the land quality and suitability of the catchments area for different land use type (i.e cultivable land, cultivated land, Grazing land, forest land, homestead (village) and other.

The present land use pattern of the catchments area shall be presented   as follows.

Land use / Land cover of the catchment area is presented as follows.

Land use type	 Area aver (%)Area (ha)

1. Cultivated land		   5.1%			                     56.5

2. Bushs aand shrubs                 11.7 %                                            130

3. Grazing land		    7.3%			           80.5

4. Forest land		               67.7%			           750

5. Valley and Hills                    2.3%                                                 25

. Miscellaneous land		5.9 %			65

Total	                            100%				1107

# 4.7 MAJOR PROBLEMS OF THE WATERSHED AREA

- New Road construction(i.e. Indeto Gasara Road) effect on watershed

Due to  different development strategies at the  national level ,  the  national government was responsible to  construct  and expand the road network system to achieve the desired strategies, enhance , Indato Gasara road construction where pass  across  Balo Habebe  watershed. Due to this the natural watershed areas are highly disturbed and it was easily viable to flood hazard. In general the downstream settled population where offering different negative impact such as:- flood hazards on  farm land,  eroded boulders and stones on both livestock  and farmer houses and the like.

- Absence of construction Measure.

Even if the project area has serious erosion and degradation problems, the construction measures are not much more viable in the area. The areas where venerable for both erosion and degradation problem. As a result  more work  for different construction measures such as check dams , soil bund, stone bund , terraces, water ways cut-off drain , and etc expected from PA, DAs and  Woreda’s  Natural resource office  experts as per requirement.

- Over stocking and overgrazing

The project area is affected by over stocking and over grazing which can be easy for erosion and degradation the main cause of erosion and degradation are overgrazing, cattle track and footpath which increase venerability of the soil for different eroding agent (i.e Rain drop, wind, floods and ect,) flood effect and erosion nazard . Generally overstocking and overgrazing reduce the erosive resistance (Grass, bushes and shrubs) and they are the starting point for rill and gully formation.

- Shortage of drinking water.

The main problem of PA in the project area is inadequate of their drinking water and its quality status. The community uses spring water as a source of supply for multipurpose such as for clinking livestock.

- Development strategies (General description)

As we know the main chronic and critical problems of the landforms i.e soil degradation and erosion is very high in the area and strategies for proper conservation measures must be high. But as development strategies, the different proper conservation measures that suitable for different land use classes should be expected from both peasant association and natural resource office of the woreda, this can be generalized, as Bio-physical soil and water conservation measures / techniques. The bio-physical soil and water conservation proposed techniques should be based on a combination of physical treatments (i.e construction of physical structures such as terraces, check dams, cut off drains, water ways and etc ) and bio-logical intervention (i.e plantation  of trees, shrubs , legumes, grasses and others).

The biological intervention is more favorable for the area inaddition to physical treatment, to give emphasis to a drought tolerant plant mix beside different erosion and flood hazard.

Implementation schedule

The physical structures that resist the erosion hazard and  erodability of the soil (i.e check dams, bunds  terraces, cutoff drains , waterways) should be  implemented from November to January and the biological intervention (i.e plantation of contour planting  of votiver grass, different legumes and shrubs) in the bank of degraded land , in the drainage bank , in rangeland  in gully bed, cruelly floors, Gully floors  , gully seine  wall and gully  offset  should be implemented from murch to June.

Equipment and man power requirement

Different machine, materials and manpower required for implementation of the project and for construction of different structures as well as canal alignment these machines and materials are:

Machine   materials

- Dozer                                      - Stone

- Grader                                   - Sand

- Excavator                              - Cement and

- Roller                                     - Other

- Crasher

- Truck

Manpower requirement

As we know man power / labor is the main agricultural input as a result for a project implementation there would be excess unskilled human power / labourforce in the project area  and from woreda town ‘’ Gasara ‘‘and Dembel.  The skilled man power also obtained from this town or from nearby towns such as Robe and Goba.

Over all Opinion about the watershed.

Identifying thoroughly the main parameters that explain the characteristics of the watershed, will give a better outcome for the sustainability of the project. As a result different physical structures and biological intervention that resist flood effect and erosion hazard which affect different designed structures (i.e. culvert flumes, division boxes canals and etc) and that reduce / minimize their safety and life time as predesigned.  Due to this, different physical structures and biological methods should be required and constructed. Beside this, these physical structures and biological interventions used to control the negative environmental impact such as desertification, ozone depletion, erosion and flood effect, as well as drought and famine problems in the area. To do so more work will be expected from development agents (DA) of the project area woreda natural resource office, Agricultural and rural development office to increase both the attitude and awareness of the local people about the negative impact on the watershed development and their corresponding mitigation measures.

# 5. ENVIRONMENTAL IMPACT ASSESSMENT

5.1 INTRODUCTION

The construction of some infrastructures can cause negative impact to the construction areas on natural resources, livestock and human directly or indirectly. It also can have positive impact to the above things therefore before construction activities are under taken environmental impact assessment should to be done.

Thus, the design for Balo Habebe Irrigation scheme has been made based on this environmental impact assessment. To do so, the following system and methods are used.

- Detailed field observation with the survey crew.

- Interview of farmers and other professionals in the area.

- Review of data about   environment in the past.

5.2 OBJECTIVES

- To identity the potential problem followed by the construction of Balo Habebe irrigation project in the catchments area.

- To asses, the social, livestock and cultural area of the catchment’s area and

- To identify the positive and negative impact of the project concerned on the Environment.

5.3 DESCRIPTION OF THE AREA

Location the Balo Habebe irrigation project is located at latitude 631635 longitude 816829 and at an altitude of about 1709 above sea level in Oromia regional state Bale zone in Gasara district on Balo Habebe kebele. The project is accessible by all weathered gravel road from zonal town ‘’ Robe ‘’ to entrance village Balo Habebe. It covers a distance of 65 km to site and the rest 3 km accessible by dry weathered road

5.4 ASSESSMENT OF ENVIRONMENTAL IMPACT

5.4.1 .ASSESSMENT OF POSITIVE ENVIRONMENTAL IMPACT

This irrigation project has positive impact for the community of the area in terms of:

- Creating sustainable agriculture.

- It improves food security for the farmers and communities.

- Increases food production and income rate.

5.4.2. ASSESSMENT OF NEGATIVE ENVIRONMENTAL IMPACT

During the construction of the project, there may be a negative impact on the environment but the negative impact due to the construction also replace by positive impact of the project.

I. Impact during design

- Bushes, and shrubs  may be disturbed

II. Impact due to project location.

The location of the farm structure will not negatively affect the existing natural and human environment rather it protect the command area from flood

III. Impact during construction

Positive impact

- It can create an employment opportunity for the local labourers. And thus, serves as a means of income generation.

- Bushes and shrubs  may be disturbed

- Natural land characteristics  may disturbed

Negative impact

- Clearance of the existing natural, vegetation, shrubs, due to construction of canal route.

- Risk on local lab outers due to migration of neighboring workers in to the area.

## Mitigation measure

- Substituting the loss of vegetation cover by planting new vegetation.

- Exercising migration control.

III. Impact during operation

Positive impact

When the proposed irrigation scheme starts to give service the following potential- positive impacts can be seen.

- Increased revenue of the users.

- Improved living standard of the users.

- The more it adds to food self-sufficiency programmed.

Negative impact

### On soil

- Soil fertility loss that may come due to the increased intensive production from the given area of land.

Mitigation measure

- Use of local manure.

- Use of selected seed varieties.

- Use of agricultural fertilizers.

Weed growth

- Growth of weed on agricultural land and other.

Mitigation measure

- Practicing efficient application of irrigation water to crops produced

- Design of surface drainage system.

- Use of selective weed Killer.

Crop Disease

- Through time the possibility pests and insects on these irrigable lands is a problem.

Mitigation measure

- Use of disease resistant varieties

- Use of pesticides and insecticides.

Incidence of Water borne disease, malaria,

- Through time the possibility of cold climate on this irrigable land can facility the breeding place for mosquito.

Mitigation measure

- Efficient application of irrigation water

- Design of surface drainage system.

- Preparation to combat malaria breakout.

# 5.5 CONCLUSION AND RECOMMENDATION

CONCLUSION

According to the feasibility, study made searching out about the proposed Environment and its main parameters that define the environmental condition. Most of these parameters indicate that the project is positive attitude. The attitude of the communities towards the project is positive.

RECOMMENDATION

The construction of this project in the area have positive impact to the communities through economically, socially and environmentally and this shows the project is very essential for the communities, therefore, the community should responsible to conserve the environment and reserve the irrigation infrastructures as per design for the project sustainability.

# 6. HYDROLOGY

6.1 INTRODUCTION

6.1.1 DESCRIPTION OF THE STUDY AREA.

6.1.1.1 Location

The Balo Habe project was found in Oromia Rigion Bale zone Gasara distirict Balo Habe PA. The catchment’s area falls in the Wabe Shebele drainage basin directly. Which characterized by high rain fall. The outlet of the catchment is located around Wabe Shebele river  (6, 14'44" N , 39,46'25"E and 1050 m a.s.l)

6.1.1.2 Topography

The major features of the catchments area are bounded by mountainous landforms, the district also characterized by other different landforms i.e Rugged, Gorges. Plains, ridge hill and valleys are included in the area. The area also dominated with steep slope. Generally, the land forms in the study area characterized by less fractured and cracked land forms as well as very less degraded land, gully and gross in the project area.

6.2. OBJECTIVE OF THE HYDRO-METROLOGICAL STUDY.

The purpose of hydro-metrological study is to assess water availability for irrigation development and planning the existing water resource among the target groups, at upstream and downstream users. Not only this but also to determine maximum flood for hydraulically efficient, structurally safe and economically viable irrigation structures design, and for water balance analysis.

6.3. CATCHMENT’S CHARACTERISTICS

The shape of the catchment’s resembles Ethiopian maps to the flow direction. The catchments area as delineated from Global Mapper is 25.22 km2 having a catchment perimeter of 21 km. The land use cover of the watershed area is 62.5% highly populated (dense) rainy forest and wood land, 21.0% with cultivated land and  the rest 16.5% is covered by pasture and /or grass land.

The catchment area of Balo Habebe falls directily to Wabe Shebele drainage basin.

6.4. METHOD

The methods used to gather different hydrological data’s are as follows.

- Using 1:50000 topographic maps of the area and hand held G.P.S in the field assessments to obtain sufficient information about the area.

- Filled survey by visualizing the existing catchment characteristics

- Interviewing the community elders in the project site.

- Collecting Climatic data from the nearby sub-metrological agency.

- Using guidelines or manuals related to the subject matter of the study.

- The contribution of sub basin delineated by using global map per and 1:50,000 scale of top map and measured a total of 25.22km2.

The availability of the required information (hydro-meteorological data) in or around the project area are assessed and discussed. Available data shall be described and presented in the form of tables, which includes source, location and altitude of the station, drainage area, and distance from the site under consideration.

| Meteorological station | Class | Distance from the project area | Location |  | Elevation |
| --- | --- | --- | --- | --- | --- |
|  |  |  | X | Y | Z |
| Agarfa | B | 25km | 39049'00" E | 70 16'00" N | 2484m |
| Robe | A | 60km | 40°00’00” E | 7° 07’ 00’’ N | 2486 m |

The nearest meteorological station of Agarfa (7016'00"N and 39049'00"E with elevation 2484m a.s.l. Due to the similarity of an altitude, closeness of the air distance Agarfa weather station is selected. Thus, the climatic parameters, like temperature and rainfall are selected for analysis of hydrological events.

6.7. CLIMATOLOGICALLY PARAMETERS

Climatologically parameters are the basic inputs in evaluation of water resource and its development activities. As a result the area is characterized by agro ecological zone of Semi-arid and agro-climatic zone of “Kola” with un predictable rainfall pattern,

1. Daily rainfall for each month for 22 years (1987 to 2009)

2. Mean monthly rainfall for 22 years (1987 to 2009)

3. Daily minimum and max. Temperature for 22 years (1987 to 2009) the recent data is mean monthly.

4. Daily max rain fall for 22 years (1987 to 2007)

5. Mean, monthly minimum and maximum temperature for 22 years (1987 to 2009)

The monthly climatic data were obtained from stations as described below. The monthly climatic data are collected from Agarfa and Robe stations. The climatic data such as temperature (max, min and mean), Humidity, wind speed and sunshine duration for the project area are shown in table below.

## MEAN MONTHLY TEMPERATURE (in 0C),

Station: Agarfa

Element: Temperature    	Lat.  7016’ Long.  39049’		Alt. 2484m.a.s.l

| Year | Jan | Feb | Mar | Apr | May | Jun | Jul | Aug | Sep | Oct | Nov | Dec |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1983 |  |  |  |  |  | 16.15 | 16.2 | 16 | 15.35 | 14.3 | 13.85 | 13.55 |
| 1984 | 13.85 | 14.35 | 15.45 | 16.5 | 15.8 | 15.55 | 15.55 | 15.8 | 14.55 | 14 | 14.3 | 14.25 |
| 1985 | 14.8 | 15.3 | 15.4 | 15.05 | 14.45 | 14.25 | 14.85 | 14.5 | 14.15 | 13.3 | 13.5 | 13.7 |
| 1986 |  | 14.85 | 15.9 | 15.35 |  | 15.4 | 15.3 | 15 | 14.5 | 13.9 | 14.15 | 14.5 |
| 1987 | 14.4 | 15.25 | 16 | 15.3 | 15.65 | 15.95 | 16 | 15.55 | 15.7 | 15.1 | 14.2 | 14.95 |
| 1988 | 15.75 | 15.9 | 16.75 | 16.55 | 16.2 | 16.05 | 15.7 | 15.25 | 14.8 | 14.55 | 13.1 | 13.5 |
| 1989 | 14.1 | 15.1 | 15.8 | 14.85 | 15.15 | 15.45 | 15.6 | 15.75 | 15.05 | 14.1 | 14.4 |  |
| 1990 | 14.55 | 18.05 | 15.6 | 15.9 | 16.2 | 16.2 | 16.15 | 16 | 15.25 | 14.9 | 14.1 | 14.15 |
| 1991 |  |  |  |  |  |  |  |  |  |  |  |  |
| 1992 | 13.35 | 15.85 | 16.95 | 16.65 | 16.3 | 16.2 | 15.6 | 14.7 | 14.1 | 14.45 | 13.35 | 14.25 |
| 1993 | 14.5 | 14.1 | 14.45 | 15.65 | 15.75 | 15.65 | 15.4 | 15.2 | 14.7 | 14.5 |  | 14 |
| 1994 | 14.35 | 15.8 | 16.4 | 16.1 | 15.95 | 16.25 | 15.25 | 14.8 | 14.85 | 13.85 | 13.6 | 13.15 |
| 1995 | 13.8 | 15.75 | 15.75 | 17.15 | 16 | 16.5 | 15.9 | 15.6 | 15.15 | 14.75 | 13.55 | 14.5 |
| 1996 | 15.3 | 15.95 | 16.5 | 16.2 | 15.8 | 15.75 | 15.6 | 15.5 | 15.8 | 14.25 | 14.2 |  |
| Mean | 14.43 | 15.52 | 15.91 | 15.94 | 15.75 | 15.80 | 15.62 | 15.36 | 14.92 | 14.30 | 13.86 | 14.05 |

| Months | Temperature Max 0C | Temperature Min 0C | Temperature Mean 0C | Relative humidity % | Wind speed m/s | Sunshine hours |
| --- | --- | --- | --- | --- | --- | --- |
| Jan | 24.0 | 3.9 | 14.4 | 56.2 | 1.7 | 7.7 |
| Feb | 26.9 | 5.2 | 15.5 | 57.0 | 1.9 | 7.2 |
| Mar | 24.9 | 5.8 | 15.9 | 64.2 | 2.0 | 7.0 |
| Apr | 23.6 | 9.3 | 15.9 | 73.2 | 1.7 | 5.9 |
| May | 23.7 | 8.3 | 15.8 | 71.1 | 1.7 | 7.6 |
| Jun | 24.6 | 6.4 | 15.8 | 67.3 | 1.8 | 7.5 |
| Jul | 22.9 | 7.9 | 15.6 | 68.8 | 1.8 | 6.4 |
| Aug | 23.1 | 8.0 | 15.4 | 73.7 | 1.8 | 6.3 |
| Sep | 22.8 | 7.4 | 14.9 | 73.9 | 1.6 | 5.2 |
| Oct | 22.2 | 6.3 | 14.3 | 74.3 | 1.4 | 6.1 |
| Nov | 22.1 | 4.8 | 13.9 | 69.6 | 1.5 | 7.6 |
| Dec | 23.7 | 4.6 | 14.0 | 61.9 | 1.6 | 6.5 |
|  | 23.7 | 6.5 | 15.1 | 67.6 | 1.7 | 6.7 |

## MONTHLY RAINFALL (in mm)

Station: Agarfa

Element: Rainfall  	Lat.  7016’ Long.  39049’		Alt. 2484m.a.s.l

| Year | Jan | Feb | Mar | Apr | May | Jun | Jul | Aug | Sep | Oct | Nov | Dec | Annual |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1987 | 15.0 | 31.9 | 140.2 | 159.6 | 214.0 | 37.1 | 47.6 | 86.8 | 96.7 | 69.9 | 4.5 | 8.3 | 911.6 |
| 1988 | 1.2 | 50.0 | 54.9 | 189.2 | 0.0 | 116.4 | 177.5 | 206.9 | 99.3 | 130.1 | 0.0 | 7.8 | 1033.3 |
| 1989 | 0.0 | 30.7 | 100.0 | 166.9 | 37.6 | 45.0 | 89.2 | 67.9 | 79.9 | 127.4 | 74.7 | 91.1 | 910.4 |
| 1990 | 0.0 | 98.0 | 48.6 | 252.6 | 48.6 | 63.1 | 105.5 | 222.1 | 52.2 | 56.2 | 34.0 | 9.2 | 990.1 |
| 1991 | 15.5 | 32.3 | 174.2 | 71.8 | 101.9 | 59.3 | 154.9 | 101.3 | 48.5 | 17.9 | 14.9 | 7.2 | 799.8 |
| 1992 | 70.2 | 39.8 | 0.0 | 16.2 | 88.4 | 60.7 | 94.5 | 153.4 | 256.4 | 152.4 | 55.5 | 71.0 | 1058.5 |
| 1993 | 46.6 | 169.9 | 0.0 | 145.9 | 104.5 | 58.3 | 38.5 | 132.9 | 114.7 | 136.4 | 4.1 | 2.5 | 954.3 |
| 1994 | 0.0 | 0.0 | 35.8 | 178.9 | 68.6 | 107.9 | 163.3 | 177.1 | 103.0 | 69.8 | 101.8 | 13.2 | 1019.4 |
| 1995 | 0.0 | 31.0 | 58.5 | 144.9 | 31.1 | 33.2 | 191.8 | 121.5 | 87.5 | 133.3 | 10.3 | 9.8 | 853.0 |
| 1996 | 0.0 | 42.4 | 134.7 | 148.7 | 235.2 | 185.5 | 109.0 | 110.4 | 221.0 | 30.1 | 36.3 | 3.6 | 1256.9 |
| 1997 | 6.4 | 0.0 | 93.9 | 78.6 | 51.7 | 42.8 | 62.0 | 78.5 | 24.5 | 144.5 | 90.8 | 20.3 | 694.0 |
| 1998 | 133.0 | 61.7 | 106.5 | 166.2 | 110.9 | 79.6 | 130.4 | 482.0 | 249.4 | 236.4 | 71.2 | 0.0 | 1827.3 |
| 1999 | 14.2 | 0.0 | 208.3 | 71.6 | 109.8 | 140.1 | 262.1 | 97.8 | 167.1 | 339.5 | 0.0 | 9.7 | 1420.2 |
| 2000 | 0.0 | 0.0 | 58.3 | 133.8 | 200.9 | 47.4 | 107.0 | 307.5 | 161.6 | 237.1 | 18.0 | 72.4 | 1344.0 |
| 2001 | 0.0 | 41.3 | 168.8 | 143.7 | 172.5 | 100.5 | 137.3 | 269.5 | 131.3 | 131.4 | 50.7 | 22.2 | 1369.2 |
| 2002 | 41.6 | 0 | 105.3 | 212.2 | 91.8 | 71.8 | 78.8 | 108.5 | 164.8 | 117.4 | 0 | 228.5 | 1220.7 |
| 2003 | 2.2 | 0 | 17.2 | 194.3 | 61.2 | 72.4 | 156.9 | 89.9 | 88.8 | 46.7 | 26.1 | 50.6 | 806.3 |
| 2004 | 53.3 | 0 | 23.1 | 205.8 | 28.5 | 57.1 | 128.1 | 110.8 | 264.9 | 80 | 30.7 | 22.2 | 1004.5 |
| 2005 | 98.9 | 3.2 | 94.7 | 157.7 | 176.9 | 31.6 | 141.5 | 86.2 | 113.3 | 99.5 | 40.5 | 0 | 1044.0 |
| 2006 | 4.3 | 53.5 | 44.8 | 135.6 | 90.4 | 37.1 | 81.3 | 97.1 | 97.5 | 64.9 | 36.7 | 45.7 | 788.9 |
| 2007 | 0 | 55.2 | 55 | 152.2 | 58.7 | 98.8 | 184 | 254.3 | 133 | 59.1 | 57.8 | 0 | 1108.1 |
| 2008 | 15.8 | 9.4 | 35.7 | 89.0 | 53.6 | 59.8 | 62.4 | 125.3 | 97.0 | 54.1 | 65.0 | 4.8 | 672.0 |
| 2009 | 24.7 | 11.5 | 30.9 | 89.3 | 40.2 | 62.8 | 56 | 122.2 | 74.4 | 95.8 | 44.7 | 89.9 | 742.5 |
| Mean | 23.6 | 33.1 | 77.8 | 143.7 | 94.6 | 72.5 | 120.0 | 157.0 | 127.2 | 114.3 | 37.8 | 34.4 | 1036.0 |
| Stdv | 35.6 | 39.7 | 57.4 | 54.3 | 64.4 | 37.6 | 54.8 | 97.0 | 66.9 | 75.0 | 29.7 | 51.5 | 273.7 |
| Cv | 1.51 | 1.20 | 0.74 | 0.38 | 0.68 | 0.52 | 0.46 | 0.62 | 0.53 | 0.66 | 0.79 | 1.50 | 0.26 |

Agarfa Max daily rainfall (in mm)

| Year | Daily rainfall mm | Year | Daily rainfall mm |
| --- | --- | --- | --- |
| 1987 | 51.3 | 1998 | 103.2 |
| 1988 | 101 | 1999 | 56 |
| 1989 | 50 | 2000 | 57.9 |
| 1990 | 72 | 2001 | 45.7 |
| 1991 | 54.2 | 2002 | 54 |
| 1992 | 45.5 | 2003 | 46.2 |
| 1993 | 42.7 | 2004 | 48.1 |
| 1994 | 41.7 | 2005 | 66 |
| 1995 | 36.6 | 2006 | 37.8 |
| 1996 | 70.6 | 2007 | 40 |
| 1997 | 40.3 | 2009 | 23.7 |

## MEAN MONTHLY SUNSHINE DURATION (in hrs),

Station: Agarfa

Element: Sunshine duration		Long.  42.21	Lat.  9.26		Alt. 2125m.a.s.l

| Year | Jan | Feb | Mar | Apr | May | Jun | Jul | Aug | Sep | Oct | Nov | Dec |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1983 | x | x | x | x | x | 8.40 | 5.50 | 3.20 | 4.80 | 5.30 | 7.30 | x |
| 1984 | x | x | 8.80 | 8.60 | 7.00 | 6.60 | 7.00 | 7.40 | 5.90 | 7.50 | 6.90 | 5.60 |
| 1985 | 7.50 | 8.70 | 7.50 | 5.50 | 7.00 | 8.60 | 5.80 | 6.60 | 5.10 | 5.20 | 7.30 | x |
| 1986 | 10.10 | 7.10 | 7.70 | 3.90 | 7.30 | 4.30 | 7.50 | 6.90 | 5.90 | 5.80 | x | 7.70 |
| 1987 | 8.60 | 7.30 | 5.20 | 6.20 | 6.20 | 8.30 | 8.00 | 6.70 | 5.80 | 5.30 | 8.30 | 8.60 |
| 1988 | 8.00 | 7.10 | 7.30 | 5.20 | 9.30 | 7.90 | 5.50 | 6.10 | 4.30 | 4.70 | 9.20 | 9.30 |
| 1989 | 9.70 | 9.00 | 6.40 | 4.40 | 9.50 | 7.90 | 5.90 | 7.10 | 3.20 | 4.90 | 6.90 | 8.20 |
| 1990 |  | 4.50 | 6.60 | 6.50 | 7.20 | 7.70 | 6.50 | 6.00 | 5.90 | 6.50 | 6.50 | 5.90 |
| 1991 | 5.50 | 7.30 | 6.30 | 6.90 | 7.30 | 7.80 | 5.40 |  |  | 7.40 | 6.60 | 0.10 |
| 1992 | 4.80 | 6.80 |  |  |  |  |  |  |  |  |  |  |
| 1993 |  |  |  |  |  |  |  |  |  |  |  |  |
| 1994 |  |  |  |  |  |  |  |  |  |  |  |  |
| 1995 |  |  |  |  |  |  |  |  |  |  |  |  |
| 1996 |  |  |  |  |  |  | 6.70 |  | 6.00 | 8.30 | 9.70 |  |
| Mean | 7.74 | 7.23 | 6.98 | 5.90 | 7.60 | 7.50 | 6.38 | 6.25 | 5.21 | 6.09 | 7.63 | 6.49 |

## MEAN MONTHLY HUMIDITY (in %),

Station: Robe

Element: Humidity		Lat.  7007’ Long. 40000’		Alt. 2486m.a.s.l

| Year | Jan. | Feb. | Mar. | Apr. | May | Jun. | Jul. | Aug. | Sept. | Oct. | Nov. | Dec. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1985 |  | 50.3 | 64.0 | 77.3 | 75.7 | 69.7 |  | 75.7 | 76.3 | 76.7 | 71.3 | 60.0 |
| 1986 | 62.3 | 76.0 | 86.7 | 87.3 | 85.7 | 84.7 | 84.0 | 82.0 | 84.7 | 82.3 | 75.7 | 67.3 |
| 1987 | 51.0 | 53.3 | 75.0 | 78.0 | 74.7 |  |  |  |  |  |  |  |
| 1988 |  |  |  |  |  |  |  |  |  |  |  | 55.3 |
| 1989 | 49.3 | 53.0 | 63.0 | 78.7 | 63.7 | 69.0 | 77.7 | 75.7 | 79.7 | 76.3 | 70.0 | 75.3 |
| 1990 | 63.0 | 72.7 | 74.3 | 77.7 | 67.3 | 66.3 | 72.7 | 79.3 | 77.7 | 72.7 | 76.7 | 61.0 |
| 1991 |  |  |  |  |  |  |  |  |  |  |  |  |
| 1992 |  |  |  |  |  |  |  |  |  |  | 68.0 | 74.3 |
| 1993 | 70.7 | 69.3 | 53.7 | 84.0 | 76.7 |  |  |  |  |  |  |  |
| 1994 |  |  |  |  |  |  |  |  | 75.3 | 70.7 | 70.0 | 58.0 |
| 1995 | 46.7 | 51.0 | 62.0 | 73.7 | 67.3 | 61.7 | 73.3 | 75.3 | 71.3 | 81.7 | 66.0 | 58.0 |
| 1996 | 59.0 | 45.0 | 66.0 | 71.0 | 71.3 |  |  |  |  |  |  |  |
| 1997 | 45.7 | 36.3 | 40.7 | 44.7 |  | 52.7 | 43.3 | 61.0 | 61.3 | 66.7 | 64.7 | 58.0 |
| 1998 | 58.3 | 62.7 | 56.7 | 59.3 | 57.7 |  | 61.7 | 66.7 | 65.0 | 67.0 | 63.7 | 51.7 |
| 2000 | 57.7 | 50.3 | 35.0 | 37.0 | 39.3 | 55.7 | 53.3 | 58.7 | 61.7 | 63.3 | 62.3 | 53.3 |
| 2001 | 15.0 | 16.7 | 13.0 | 23.3 | 25.0 | 21.7 | 18.3 | 17.0 | 20.7 | 19.7 | 21.7 | 19.3 |
| 2002 | 58.0 | 38.3 | 55.3 | 61.0 | 71.3 | 39.7 | 68.7 | 77.3 | 76.3 | 77.3 | 76.3 | 68.3 |
| 2003 | 59.3 | 55.3 | 57.0 | 67.0 | 70.0 | 70.0 | 73.7 | 80.3 | 86.3 | 82.3 | 75.7 | 61.3 |
| Mean | 56.2 | 57.0 | 64.2 | 73.2 | 71.1 | 67.3 | 68.8 | 73.7 | 73.9 | 74.3 | 69.6 | 61.9 |

## MEANS MONTHLY WIND SPEED (m/s)

Station: Robe

Element: Humidity		Lat.  7007’ Long. 40000’		Alt. 2486m.a.s.l

| Year | Jan | Feb | Mar | Apr | May | Jun | Jul | Aug | Sep | Oct | Nov | Dec |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1984 | x | x | x | x | x | x | x | x | 1.5 | 1.6 | 1.6 | 1.5 |
| 1985 | 1.7 | 1.9 | 2.1 | 1.7 | 1.6 | 2.0 | 1.9 | 2.2 | 1.8 | 1.6 | 1.6 | 1.7 |
| 1986 | 2.0 | 2.0 | 2.2 | 1.7 | 1.7 | 1.8 | 2.1 | 2.3 | 1.8 | 1.6 | 1.9 | 2.0 |
| 1987 | 1.9 | 2.1 | 2.1 | 1.7 | 1.5 | 1.7 | 2.1 | 2.1 | 1.9 | 1.3 | 1.5 | 1.7 |
| 1988 | 1.8 | 1.9 | 1.9 | 1.6 | 2.0 | 2.2 | 2.2 | 1.8 | 1.6 | 1.3 | 1.4 | 1.6 |
| 1989 | 1.6 | 2.0 | 2.2 | 1.6 | 1.9 | 2.0 | 1.7 | 2.0 | 1.7 | 1.5 | 1.4 | 1.4 |
| 1990 | 1.8 | 2.0 | 1.9 | 1.7 | 1.2 | 1.4 | 1.4 | 1.0 | 1.0 | 1.6 | 1.5 | 1.7 |
| 1991 | 1.8 | 2.0 | 2.0 | 1.9 | 1.9 | 2.2 | 1.7 | 2.2 | 2.0 | 1.8 | 1.9 | 2.0 |
| 1992 | 1.9 | 2.0 | 2.4 | 2.0 | 1.8 | 2.1 | 2.2 | 2.0 | 1.9 | 1.4 | 1.5 | 1.6 |
| 1993 | 1.6 | 2.0 | 2.2 | 2.1 | 1.7 | 2.1 | 1.9 | 2.2 | 2.0 | 1.5 | 1.7 | 1.9 |
| 1994 | 2.1 | 2.2 | 2.4 | 1.8 | 1.7 | 2.1 | 2.0 | 2.4 | 1.9 | 1.5 | 1.4 | 1.5 |
| 1995 | 1.7 | 1.9 | 2.1 | 1.7 | 1.9 | 2.1 | 2.3 | 2.3 | 1.6 | 1.3 | 1.4 | 1.6 |
| 1996 | 1.8 | 2.0 | 2.0 | 1.8 | 1.9 | 1.5 | 1.7 | 1.6 | 1.5 | x | x | x |
| 1997 |  |  |  |  |  |  |  |  |  |  |  |  |
| 1998 | 0.9 | 2.2 | 1.2 | 1.4 | 1.3 | 1.1 | x | 1.2 | 1.4 | 0.9 | 1.0 | 1.0 |
| 1999 |  |  |  |  |  |  |  |  |  |  |  |  |
| 2000 |  |  |  |  |  |  |  |  |  |  |  |  |
| 2001 | 1.2 | 1.1 | 0.2 | 1.2 | 1.1 | 1.2 | 1.1 | 1.1 | 1.0 | 1.2 | 1.1 | 1.2 |
| 2002 |  |  |  |  |  |  |  |  |  |  |  |  |
| 2003 |  |  |  |  |  |  |  |  |  |  |  |  |
| 2004 | 1.4 | 1.5 | 1.6 | 1.4 | 1.9 | 1.9 | 1.6 | 1.4 | 1.1 | 0.8 | 0.9 | 1.2 |
| 2005 | 1.4 | 1.7 | 1.8 | 1.4 | 1.0 | 1.4 | 1.5 | 1.5 | 1.3 | 0.9 | 1.1 | 1.5 |
| Mean | 1.7 | 1.9 | 2.0 | 1.7 | 1.7 | 1.8 | 1.8 | 1.8 | 1.6 | 1.4 | 1.5 | 1.6 |

i. Rainfall

- Mean total annual rainfall = 1036mm

- The longest rain season is Afrassa (late March to May) which accounts 50% of total mean annual rainfall

- * The shortest rainfall season is Ganna (late September to November) that is about 35% of the total mean annual rainfall.

- December, January and February are the driest months in which little rainfall is recorded so far.

ii. Temperature

The lowest mean minimum temperature occurs in the month of January while the highest mean minimum temperature occurs in the month of June.

- The average of mean minimum temperature is 6.5c0

- The lowest mean maximum temperature occurs in November and the highest mean maximum temperature occurs in April.

- The average of mean maximum temperature is 23.7 c0

6.8. LEAN FLOW ANALYSIS

6.8.1. Floating method (March 01/03/ 2015)

This way of estimating was taken at month of March and it was measured at a place where the spring channel cross section is almost rectangular.

I.e. QL= VxA

Where:-QL = lean flow in m3/s

- V = velocity of flow in m/s

- V = distance moved/average time elapsed

- A = cross sectional area of the flow channel at the measurement point, m2

- Average base flow depth, d in m

- Cross sectional area, A = dxL in  m2

- Longitudinal river length at point of measurement in m

- Time taken at different trials, t1, t2, t3, t4------ in  sec

Trial I - at point A (March 01/03/2015)   L=2.2m     W1= 0.55m, W=0.65m, W=0.88m, W=1.2m

Then average width of the measured (W1, W2, W3, W4), Wav=0.82m

The different time spent for 2.2m flow length i.e. t1=3.09sec, t2=3.19sec, t3=3.1sec,   t4=3.09sec, t5=3.27sec, then the average time spent, tav=3.148sec

Cross sectional depth determination at different location

Location I, Dl=0.06m, Dc=0.09m, Dr=0.07m, average depth at location-I, Dav=0.073m

Location II, Dl=0.08m, Dc=0.09m, Dr=0.07m, average depth at location-II, Dav=0.08m

Location III, Dl=0.09m, Dc=0.12m, Dr=0.09m, average depth at location-III, Dav=0.1m

Location IV, Dl=0.08m, Dc=0.12m, Dr=0.08m, average depth at location-IV, Dav=0.093m

Then the overall average depth (0.073+0.08+0.1+0.093)/4, Dav=0.0866m

- The flow area= Wav*Dav =0 .82m*0.0866m= 0.071m2

- the velocity of flow = l/taver = 2.2m/3.148 sec=0.698m/sec

QL = VxA = 0.698x0.071 = 0.04955m3/s =49.6l/sec

Trial I - at point B (March 01/03/2015)   L=3.0m     W1= 0.54m, W2=0.7m, W3=0.6m,

Then average width of the measured (W1, W2, W3), Wav=0.6m

The different time spent for 3.0m flow length i.e. t1=7.97sec, t2=8.79sec, t3=8.42sec,   t4=8.46sec, then the average time spent, tav=8.42sec

Cross sectional depth determination at different location

Location I, Dl=0.32m, Dc=0.2m, Dr=0.19m, average depth at location-I, Dav=0.24m

Location II, Dl=0.3m, Dc=0.3m, Dr=0.25m, average depth at location-II, Dav=0.28m

Location III, Dl=0.32m, Dc=0.27m, Dr=0.28m, average depth at location-III, Dav=0.29m

Then the overall average depth (0.24+0.28+0.29)/3, Dav=0.27m

- The flow area= Wav*Dav =0 .6m*0.27m= 0.162m2

- the velocity of flow = l/taver = 3.0m/8.42 sec=0.356m/sec

QL = VxA = 0.356x0.162 = 0.057672m3/s =57.67l/sec

The overall measured discharge on march 01/03/2015 = (QA+QB)=(49.6l/sec+57.67l/sec)=107.27l/sec

Trial II - at point A (March 03/03/2015)   L=2.43m     W1= 0.6m, W2=0.7m, W3=0.75m, W4=0.8m

Then average width of the measured (W1, W2, W3, W4), Wav=0.713m

The different time spent for 2.43m flow length i.e. t1=2.66sec, t2=2.52sec, t3=2.84sec,   t4=2.39sec, t5=2.52sec, then the average time spent, tav=2.586sec

Cross sectional depth determination at different location

Location I, Dl=0.07m, Dc=0.08m, Dr=0.05m, average depth at location-I, Dav=0.067m

Location II, Dl=0.08m, Dc=0.1m, Dr=0.06m, average depth at location-II, Dav=0.08m

Location III, Dl=0.07m, Dc=0.095m, Dr=0.06m, average depth at location-III, Dav=0.075m

Then the overall average depth (0.067+0.08+0.075)/3, Dav=0.074m

- The flow area= Wav*Dav =0 .713m*0.074m= 0.0527m2

- the velocity of flow = l/taver = 2.43m/2.586sec=0.94m/sec

QL = VxA = 0.94x0.0527 = 0.04954m3/s =49.54l/sec

Trial II - at point B (March 03/03/2015)   L=2.45m     W1= 0.6m, W2=0.7m, W3=0.67m, W4=.6

Then average width of the measured (W1, W2, W3 and W4), Wav=0.643m

The different time spent for 3.0m flow length i.e. t1=7.6sec, t2=6.5sec, t3=6.77sec,   t4=6.8sec, t5=6.29sec, t6=5.85, t7=6.5 then the average time spent, tav=6.616sec

Cross sectional depth determination at different location

Location I, Dl=0.2m, Dc=0.3m, Dr=0.2m, average depth at location-I,   Dav=0.233m

Location II, Dl=0.17m, Dc=0.29m, Dr=0.17m, average depth at location-II, Dav=0.21m

Location III, Dl=0.2m, Dc=0.3m, Dr=0.23m, average depth at location-III, Dav=0.243m

Location IV, Dl=0.25m, Dc=0.33m, Dr=0.305m, average depth at location-IV, Dav=0.295m

Then the overall average depth (0.233+0.21+0.243+0.295)/3, Dav=0.245m

- The flow area= Wav*Dav =0 .643m*0.245m= 0.157m2

- the velocity of flow = l/taver = 2.45m/6.616 sec=0.3703m/sec

QL = VxA = 0.37033x0.157 = 0.058139m3/s =58.14l/sec

The overall measured discharge on march 01/03/2015 = (QA+QB)=(49.54l/sec+58.14l/sec)=107.68l/sec

In general, the spring capacity from conclusion of Trial-I and Trial-II= (107.27+107.68)/2=107.475l/sec

Considering, the relationship between surface velocity and normal spring flow velocity, the net discharge capacity of the spring can be determined by:- Qnet=0.8Qs=0.8*107.475=86l/sec(so, use 86 l/sec for your canal and farm structure purpose)

6.8.2 PEAK FLOW ANALYSIS

Flood means the draining or flowing of surplus water from rainfall as runoff from catchment area through a surface channel or streams. It is used for designing sustainable and long live structure such as weir and culvert in order to minimize their failure probability as much as possible. For safe structural design associated with risk of failure, which should be justified from economical point of view different design flood analysis approaches were used during the design. There are different alternative methods that have been used for the analysis of design flood.

6.8.2.1 Complex Unit hydrograph

The recent metrological data is obtained from nearest station Delo Mena metrological station

  - Station: Gasara  (Bale)

  - Coordinate: 06º 24’30’’ N and 039º50’10’’E

  - Elevation: 1280m

Table 5.2 Daily Maximum Rainfalls of the year (mm)

| A summarized Gumble- powell’s table |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| No |  | Designation /Formula/ |  |  |  |  |  |  |  |  |  |  |  | Symbol |  |  |  | Unit |  |  | Value |  |  |  |
| 1 |  | Area of the catchment measured from 1:250,000 topographic map of aerial photo |  |  |  |  |  |  |  |  |  |  |  | A |  |  |  | Km2 |  |  | 25.3 |  |  |  |
| 2 |  | Length of main water course b/n H1&H2 |  |  |  |  |  |  |  |  |  |  |  | L1 |  |  |  | m |  |  | 1700 |  |  |  |
|  |  | Length of main water course b/n H2&H3 |  |  |  |  |  |  |  |  |  |  |  | L2 |  |  |  | m |  |  | 2500 |  |  |  |
|  |  | Length of main water course b/n H3&H4 |  |  |  |  |  |  |  |  |  |  |  | L3 |  |  |  | m |  |  | 860 |  |  |  |
|  |  | Total length |  |  |  |  |  |  |  |  |  |  |  | L |  |  |  | m |  |  | 5060 |  |  |  |
| 3 |  | Level of head of main water course |  |  |  |  |  |  |  |  |  |  |  | H1 |  |  |  | m |  |  | 2334 |  |  |  |
|  |  | Level of head of main water course |  |  |  |  |  |  |  |  |  |  |  | H2 |  |  |  | m |  |  | 1641 |  |  |  |
|  |  | Level of head of main water course |  |  |  |  |  |  |  |  |  |  |  | H3 |  |  |  | m |  |  | 1404 |  |  |  |
|  |  | Level of middle of main water course |  |  |  |  |  |  |  |  |  |  |  | H4 |  |  |  | m |  |  | 1387 |  |  |  |
|  |  | Level of lower main water course |  |  |  |  |  |  |  |  |  |  |  | H5 |  |  |  | m |  |  | 1800 |  |  |  |
| 4 |  | Slope of main water course S1=[H1-H2]/L1 |  |  |  |  |  |  |  |  |  |  |  | S1 |  |  |  | m |  |  | 0.407647 |  |  |  |
|  |  | Slope of main water course S1=[H2-H3]/L2 |  |  |  |  |  |  |  |  |  |  |  | S2 |  |  |  | m |  |  | 0.094800 |  |  |  |
|  |  | Slope of main water course S1=[H3-H4]/L3 |  |  |  |  |  |  |  |  |  |  |  | S3 |  |  |  | m |  |  | 0.019767 |  |  |  |
|  |  | Average Slope of main water course S=(S1+S2+S3+S4+S5)/5 |  |  |  |  |  |  |  |  |  |  |  | S |  |  |  | m/m |  |  | 0.17 |  |  |  |
|  |  | Time of Concentration ,TC1=1/3000(L1/(S1)1/2 )0.77 |  |  |  |  |  |  |  |  |  |  |  | TC1 |  |  |  | hr |  |  | 0.14 |  |  |  |
|  |  | Time of Concentration ,TC2=1/3000(L2/(S2)1/2 )0.77 |  |  |  |  |  |  |  |  |  |  |  | TC2 |  |  |  | hr |  |  | 0.34 |  |  |  |
|  |  | Time of Concentration ,TC3=1/3000(L3/(S3)1/2 )0.77 |  |  |  |  |  |  |  |  |  |  |  | TC3 |  |  |  | hr |  |  | 0.27 |  |  |  |
|  |  | Time of Concentration ,TC=TC1+Tc2+TC3+TC4+TC5 |  |  |  |  |  |  |  |  |  |  |  | TC |  |  |  | hr |  |  | 0.76 |  |  |  |
| 6 |  | Rain fall excess duration, D= TC/6 |  |  |  |  |  |  |  |  |  |  |  | D |  |  |  | hr |  |  | 0.13 |  |  |  |
| 7 |  | Time to peak Tp=0.5D+0.6TC |  |  |  |  |  |  |  |  |  |  |  | Tp |  |  |  | hr |  |  | 0.52 |  |  |  |
| 8 |  | Time of base of hydrograph Tb=2.67Tp |  |  |  |  |  |  |  |  |  |  |  | Tb |  |  |  | hr |  |  | 1.39 |  |  |  |
| 9 |  | Lag time of Tl=0.6TC |  |  |  |  |  |  |  |  |  |  |  | Tl |  |  |  | hr |  |  | 0.46 |  |  |  |
| 10 |  | Peak rate of discharge created by 1mm RF excess on whole of the catchment Qp=0.21*QA/Tp ,Qp=1mm(assumed) |  |  |  |  |  |  |  |  |  |  |  | Qp |  |  |  | m3/s/mm |  |  | 10.22 |  |  |  |
| 11 |  | 12 | 13 | 14 | 15 | 16 |  | 17 |  | 18 |  | 19 |  |  | 20 |  | 21 |  |  | 22 |  |  |  |  |  | 23 |  | 24 |
| Duration |  | Daily point Rainfall of Return period Of 100 year | Rain Fall Profile | Rainfall Profile | Area to Point Rainfall Ratio | Areal Rainfall |  | Incremental Rainfall |  | Descending Order |  | Rearranged Order |  |  | Rearranged Incremental Rainfall |  | Cumulative Rain fall |  |  | Time of Incremental unit hydrograph |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Time of Incremental unit hydrograph |  |  |  |  |  |
| hr |  | mm | % | mm | % | mm |  | mm |  | No |  | No |  |  | mm |  | mm |  |  | hr |  |  |  |  |  |  | hr |  |
| 0-0.13 |  | 99.44 | 26 | 25.85 | 39.5 | 10.21 |  | 10.21 |  | 1 |  | 6 |  |  | 3.90 |  | 3.90 |  |  | 0 |  | 0.52 |  |  |  |  | 1.39 |  |
| 0.13-0.26 |  | 99.44 | 35 | 34.80 | 49.5 | 17.23 |  | 7.02 |  | 4 |  | 4 |  |  | 3.24 |  | 7.14 |  |  | 0.13 |  | 0.65 |  |  |  |  | 1.51 |  |
| 0.26-0.39 |  | 99.44 | 44 | 43.75 | 59.5 | 26.03 |  | 8.81 |  | 2 |  | 3 |  |  | 8.81 |  | 15.95 |  |  | 0.25 |  | 0.77 |  |  |  |  | 1.64 |  |
| 0.39-0.52 |  | 99.44 | 46 | 45.74 | 64.0 | 29.28 |  | 3.24 |  | 6 |  | 1 |  |  | 10.21 |  | 26.16 |  |  | 0.38 |  | 0.90 |  |  |  |  | 1.77 |  |
| 0.52-0.63 |  | 99.44 | 53 | 52.70 | 69.5 | 36.63 |  | 7.35 |  | 5 |  | 2 |  |  | 7.02 |  | 33.17 |  |  | 0.51 |  | 1.03 |  |  |  |  | 1.89 |  |
| 0.63-0.76 |  | 99.44 | 57 | 56.68 | 71.5 | 40.53 |  | 3.90 |  | 6 |  | 5 |  |  | 7.35 |  | 40.53 |  |  | 0.63 |  | 1.15 |  |  |  |  | 2.02 |  |
| 25 |  |  |  |  |  | 26 |  | 27 |  | 28 |  | 29 |  |  | 30 |  |  |  |  |  |  |
| Land use Cover |  |  |  |  |  | Area Ratio % |  | Hydro soil group |  | Curve No”CN” |  | Weighted “CN” |  |  | Sum Weighted “CN” |  |  |  |  |  |  |
| Wood land(Forest) |  |  |  |  |  | 67.5 |  | Fair |  | 36 |  | 24 |  |  | AMC |  |  | CN |  |  |  |
| Cultivated land |  |  |  |  |  | 16.8 |  | Fair |  | 56 |  | 9 |  |  | II |  |  | 44 |  |  |  |
| Pasture land |  |  |  |  |  | 15.5 |  | Fair |  | 69 |  | 11 |  |  | III |  |  | 64.8 |  |  |  |

| No | Designation /Formula/ | Symbol | Unit | Magnitude/Value/ |
| --- | --- | --- | --- | --- |
| 31 | S is maximum potential difference between rainfall (P) and direct runoff(Q) which is S=(25400/CN)-254 | S | mm | 138.28 |
| 32 | Relation between direct Runoff( Q) and Rainfall (P), Q=(P-0.2*S) 2/P+0.8S | Q | mm | P-0.2*S |
| 33 | Using Q= (P-0.2*S) 2/(P+0.8*S),it is possible to define the direct run off Q,(mm) | C=21 | C=33 |  |
|  |  | P (mm) | Q (mm) |  |
|  |  | 3.90 | 4.93 | -23.76 |
|  |  | 7.14 | 3.57 | -20.52 |
|  |  | 15.95 | 1.08 | -11.71 |
|  |  | 26.16 | 0.02 | -1.50 |
|  |  | 33.17 | 0.21 | 5.52 |
|  |  | 40.53 | 1.10 | 12.87 |

| 34 | 35 | 36 | 37 | 38 |  |  | 39 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Duration | Cumulative | Incremental | Peak Runoff | Time of beginning | Time to peak | Time to end | Composite Hydrograph |
| Hr | mm | mm | M3/s | Hr | Hr | Hr | f(t)=Q |
| 0-0.13 | 4.93 | 4.93 | 50.39 | 0 | 0.52 | 1.39 |  |
| 0.13-0.26 | 3.57 | -1.35 | 0.00 | 0.13 | 0.65 | 1.51 |  |
| 0.26-0.39 | 1.08 | -2.49 | 0.00 | 0.25 | 0.77 | 1.64 |  |
| 0.39-0.52 | 0.02 | -1.07 | 0.00 | 0.38 | 0.90 | 1.77 |  |
| 0.52-0.63 | 0.21 | 0.20 | 2.00 | 0.51 | 1.03 | 1.89 |  |
| 0.63-0.76 | 1.10 | 0.88 | 9.04 | 0.63 | 1.15 | 2.02 |  |

Rain fall analysis

| S.No | Year | RF(mm) | S.No | Year | RF(mm) |
| --- | --- | --- | --- | --- | --- |
| 1 | 1968 | 26.9 | 21 | 1988 | 40.4 |
| 2 | 1969 | 48.8 | 22 | 1989 | 42 |
| 3 | 1970 | 40.5 | 23 | 1990 | 34.7 |
| 4 | 1971 | 54.5 | 24 | 1991 | 25.6 |
| 5 | 1972 | 32.7 | 25 | 1992 | 35.8 |
| 6 | 1973 | 38 | 26 | 1993 | 55.2 |
| 7 | 1974 | 41.4 | 27 | 1994 | 58.2 |
| 8 | 1975 | 30.6 | 28 | 1995 | 36.2 |
| 9 | 1976 | 51.8 | 29 | 1996 | 27.2 |
| 10 | 1977 | 81.1 | 30 | 1997 | 45.9 |
| 11 | 1978 | 77.1 | 31 | 1998 | 49.5 |
| 12 | 1979 | 55.1 | 32 | 1999 | 44.1 |
| 13 | 1980 | 60.8 | 33 | 2000 | 72.8 |
| 14 | 1981 | 97.9 | 34 | 2001 | 31.8 |
| 15 | 1982 | 57.6 | 35 | 2002 | 26.9 |
| 16 | 1983 | 100.3 | 36 | 2003 | 52.8 |
| 17 | 1984 | 80.9 | 37 | 2004 | 35.7 |
| 18 | 1985 | 57.8 | 38 | 2005 | 65.2 |
| 19 | 1986 | 72.8 | 39 | 2006 | 42.2 |
| 20 | 1987 | 48.2 | 40 | 2007 | 32.2 |
|  | Mean Q |  | 50.23 |  |  |
|  | SD |  | 18.98377 |  |  |
|  | P50 |  | 99.43594 |  |  |

| Unit Hydrograph |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 1 | 2 | 3 | 4 | 5 | 6 |  |  |
| to | 0 | 0.13 | 0.25 | 0.38 | 0.51 | 0.63 |  |  |
| Qo | 0 | 0 | 0 | 0 | 0 | 0 |  |  |
| tp | 0.52 | 0.65 | 0.77 | 0.90 | 1.03 | 1.15 |  |  |
| Qp | 50.39 | 0.00 | 0.00 | 0.00 | 2.00 | 9.04 |  |  |
| tb | 1.39 | 1.51 | 1.64 | 1.77 | 1.89 | 2.02 |  |  |
| Qo' | 0 | 0 | 0 | 0 | 0 | 0 |  |  |
| Time(hr) | Ordinate of Hydrograph(m3/s) |  |  |  |  |  | Total |  |
|  | 1 | 2 | 3 | 4 | 5 | 6 |  |  |
| 0 | 0.00 |  |  |  |  |  | 0 |  |
| 0.13 | 12.29 | 0 |  |  |  |  | 12 |  |
| 0.25 | 24.58 | 0 | 0.00 |  |  |  | 25 |  |
| 0.38 | 36.87 | 0 | 0.00 | 0.00 |  |  | 37 |  |
| 0.51 | 49.16 | 0 | 0.00 | 0.00 | 0.00 |  | 49 |  |
| 0.52 | 50.39 | 0 | 0.00 | 0.00 | 0.05 |  | 50 |  |
| 0.63 | 43.76 | 0 | 0.00 | 0.00 | 0.49 | 0.00 | 44 |  |
| 0.65 | 43.03 | 0 | 0.00 | 0.00 | 0.54 | 0.22 | 44 |  |
| 0.77 | 35.67 | 0 | 0.00 | 0.00 | 1.02 | 2.43 | 39 |  |
| 0.90 | 28.31 | 0 | 0.00 | 0.00 | 1.51 | 4.63 | 34 |  |
| 1.03 | 21.0 | 0 | 0.00 | 0.00 | 2.00 | 6.84 | 30 |  |
| 1.15 | 13.6 | 0 | 0.0 | 0.0 | 1.71 | 9.04 | 24 |  |
| 1.39 | 0.0 | 0 | 0.0 | 0.0 | 1.17 | 6.60 | 8 |  |
| 1.51 |  | 0 | 0.0 | 0.0 | 0.88 | 5.28 | 6 |  |
| 1.64 |  |  | 0.0 | 0.0 | 0.58 | 3.96 | 5 |  |
| 1.77 |  |  |  | 0.0 | 0.29 | 2.64 | 3 |  |
| 1.89 |  |  |  |  | 0.00 | 1.32 | 1 |  |
| 2.02 |  |  |  |  |  | 0.00 | 0 |  |

Flood control trence profiles

| S.NO. | x | y | p-distance | T-distance | Elevation |  |
| --- | --- | --- | --- | --- | --- | --- |
| Balo habebe recommended trench excavation profile to control flood damage on farmers farm land and all plantation from flood erosion hazard that is highly dangerous on both farm land and crops, by degradation Of all fertile soil from the watershed, due to slope steepness, disturbed natural watershed due to indato gasara Road construction and none viable watershed treatment after the road construction. |  |  |  |  |  |

# 7. GEOLOGY, HYDROGEOLOGY AND GEOTECHNIQUES

## 7.1 BACKGROUND

The Balo_Abebe SSIP study has been made in due  to fulfill food self sufficient and also to be competitive in the market and at the same time educating and initiating farmers to be play their own parts on soil and water conservation practices and thereby to create a sustainable environment. to make complete the overall study different disciplines have participated of which the geologists has made an overall study on sustainability of the geology of the area for the proposed irrigation schemes.

# 7.1.1   LOCATION AND ACCESSIBILITY`

Balo_Abebe SSIP project is located in Gassera District and particularly in the so-called Balo_Abebe peasant association /Villages with an approximate distance of 10km from Gassera town, which is districts capital town, and70km from Robe which is zonal capital town. The project site can be accessed by travelling 67kmon all weathered gravel packed road that joins the zonal capital, Robe town with Balo_Abebe villages junction and then by detouring to the right and travelling for about3km on all weathered desert road length of the journey up to project site or headwork.

7.2 OBJECTIVE OF THE GEOLOGICAL & GEOTECHNICAL INVESTIGATION

The main objectives of the geological and geotechnical investigation study of the project should be listed and described.

- The main purpose of the study is to generate data for the headworks, night storages and other hydraulic structures, irrigation and drainage system design as well as catchment development planning.

- The study will also provide information on the availability and location of construction materials required for the project construction, water quality, and ground water conditions of the commandarea. Thus the study will cover geology, geomorphology, hydrogeology and geophysics.

- The aim of the geological and geotechnical investigation is to address all issues necessary to present the technical and economic feasibility of the proposed engineering structures with sufficient level of investigation.

- An investigation on the suitability of the proposed diversion weir or dam site for the intended construction of engineering structures.

- Identification of surface and subsurface lithological and structural data of the foundation of the main canal alignment.

- Evaluation of the types of construction materials available around the project area and determining their quality and quantity in order to suit for the project.

- To grant a final geotechnical recommendation on remedial measures and foundation design based on the study and investigation carried out.

7.2.1 SCOPE OF THE GEOLOGICAL & GEOTECHNICAL INVESTIGATION

The scope of work of the geological and geotechnical investigations should be briefly described.

- Geological and Engineering geological mapping of the weir site, pump site, main canal sites, dam & reservoir sites.

- Investigation on the type and nature of geological structures pertinent in the locality.

- Examination on the nature of river valley such as bank slopes, prospective land sliding, valley width and stability of cut slopes.

- To determine the nature of the foundation on the headworks site, powerhouse and other appurtenant structure’s sites.

- The assessment on the presence of construction materials in the vicinity of the project area.

- The investigation of the geological nature of the foundation on headrace routes.

- Laboratory analysis results on foundation and construction materials.

- Seismic hazard assessment of the project sites and recommendation of earthquake parameters.

7.2.2 Approach & Methodology

Describe the techniques, approaches & methods used for the study in the following phases- collection, compilation and evaluation of existing data, fieldwork on geological and geotechnical investigation, data integration and interpretations.

- The geological and geotechnical investigation will follow a systematic approach. This will be office level review of previous studies on geological structures including aerial photo/satellite imagery interpretation to identify major lineaments, fault lines and regional structures and conduction of intensive field surveys for in-situ testing, material sampling for lab analysis and interpretation of field and laboratory testing.

- The geological and geotechnical investigations of an irrigation project are usually conducted in three phases:- collection, compilation and evaluation of existing data, field work on geological and geotechnical investigation, and data integration and interpretation.

(1) Collection, compilation and evaluation of existing data

- This deals with gathering of existing information on the general geological, structural and geomorphological setting of the target area and the regional river basin in general. This activity includes review of all relevant geological, structural and geomorphological information including the geological map of Ethiopia mainly from the Geological survey of Ethiopia.

- Identify data gaps, which need to be considered during the field study.

- Interpretation of aerial photographs and Landsat imageries.

- Planning of traverse routes for geological fieldwork and identify any potential problems that may need to be dealt, so that it will not hinder the project activities.

- Compilation and evaluation of the gathered information on the major geological, structural, and geomorphological activities, will be appropriately evaluated, filtered and compiled in summarized form in such a way suitable to serve the objective.

The summarized information, which is an inventory of geology, structure, and geomorphology will be analyzed comparatively with other published irrigation scheme models.

(2) Fieldwork on geological and geotechnical investigation

- Field work on geological & geotechnical investigation is usually conducted in stages of starting from the initial reconnaissance to the last detail investigation.

- Site visit inspection of exposed rock outcrops and observation of stratigraphic successions of soils and rocks in river cut exposures, and pitting along the axis of the weir and following the main canal are the main methodologies used in studying the geological and structural setup of a target area. Traverses should also be carried out along the main canal route and in the command area, in order to pin point the possible structural features pertinent in the locality.

- The general geological features and lithologies of the command area should also been inspected. Geological apparatus like hand lens, geological hammer, GPS and topographic maps will be used for the study purposes.

In summery the main activities that should be accomplished during the field investigation include-

- Geological and structural mapping of the area in and around the locality with particular emphasis to weir site, main canal, dam and reservoir areas, night storage areas & headrace and its alignment.

- Detailed mapping on large scale for instance 1:2,000 of the area extending up to 200-300 m upstream and downstream of the proposed weir axis or dam axis.

- Preparation of geological maps of the study area along with geological cross-section of the weir axis or dam axis.

- Test pitting at weir site, dam site, main canal line, and quarry sites.

- Test pit logging and sampling

- Construction material identification and field description using geological hammer and lens and their laboratory analysis results & interpretation.

- Sampling of disturbed and undisturbed samples and Laboratory testing of rock and soil samples.

(3) Data integration and interpretations

- Collection of all the data using appropriate software

- Analysis and interpretation of field data

- Preparations and submission of reports

7.3 GENERAL TOPOGRAPHY,

7.3.1Geomorphology/drainage pattern and Land Cover

The geomorphological set of the project area is undulating landscape with rolling erosional surfaces.

In general, the project area is characterized by a moderately undulating topography, in which perennial and intermittent springs and other seasonal streamlines whose sources are from the surrounding high peak chained mountains belts, which bounds north, northwest and northeast parts of the area.

The upstream of the weir site contains a portion of the land covered by a dense tropical rain forest .while the downstream of the headwork site is covered by moderately dense forest of desert tress .Tebela spring generally flows to north east direction and meandering at various places. It has a narrow valley which is characterized by gently slop land on either side of the spring.

The vegetation cover in this part of the area is moderately dense, where forests of the savanna type, i.e. Acacia species, and thorn growth, occupy majority of the land surface. In addition, particularly along permanent stream course, there are riverside trees grown taking the advantage of the spring water.

## 7.4 HYDROGEOLOGY

  - 7.4.1 Tebela spring nature

As to the information from the local people and data from topographic map of the area Tebela spring originates at the foot of chained mountain belt of  Wabe Gorge, which surround the spring on the north part of Gassera town.

In its upper course, i.e. upstream of the headwork site, a number of perennial, intermittent and seasonal springs and tributaries recharge it. The headwork axis is centered on 783494N latitude and longitude 687618.4 E, with an average altitude of1639.7651a.m.s.l. The Weir site is characterized by about 10m wide river channel over saturated basalt. At this selected site, the spring has a narrow poured, more or less straight bank in which the stream bed is composed of gravels, pebbles and cobbles of basaltic rock composition.

### 7.4.2 Groundwater condition of the area

The presence of weathering and fracture zones, faults and folds, joints and vesicles impart the limestone rock exposed in the area are a potential water bearing formation. However, the above mentioned points are not the sufficient conditions for the availability of ground water in a certain area.

The general recharge and geomorphological set up of the area the prevailing annual precipitation rate and existence of thick silty clay soil over burden which facilitates infiltrationof the metric water rather than the down percolation depending on the degree of compaction are also necessary precondition for ground water to occur in the area.

### 7.5. Regional and catchments geology

The regional geology of Wabe gorge is characterized by volcanics of the Trap series at the top and sedimentary rocks at the bottom. The volcanic rock which outcrops at the project area is comprised of ignimbrite, trachytes, rhyolites and basalt.

Geologically, the village Balo_Abebe lies on tertiary deposited, which overlain by Tertiary rocks of volcanic rock units. The overlaying trap lavas form the mountain chains and covered the underlying sedimentary rock formations. The flood basalt sequence is attributed to the development of the Ethiopia rift system. The lava that outcropped during this geological period formed the Bale mountain chains with an extensively formed plateau. These volcanic caps are predominantly basaltic rock in composition but with subordinate rhyolites and agglomerates.

The geological formation in the upstream drainage of the spring Balo_Abebe is totally constituted by the aforementioned igneous rock bodies. The occurrence of the already transported volcanic rock, which is largely composed of pebbles, cobbles, and boulders of basaltic rock composition reflect this condition.

## 7.6. LOCAL GEOLOGY

The area is covered by volcanic rocks of Tertiary period that overlain the limestone unit. The out crops are clearly exposed along main canal route and on the tops and slopes of elevated ridges of the project area. The limestone rock has a black and dark grey colour and is relatively fresh to slightly weathered and fractured, fine-grained and vesiculated. The limestone rock where exposed at places is layered and stiratified, laminated and bedded, jointed and affected by solution activities. Where the overlaying basaltic rock unit is exposed on a flat- plain land and / or gently sloping hills of the project area is covered by a purely silty clay soil (black cotton soil) and / or reddish brown silt- clay soil overburden of varying thickness and having a high plasticity. The overburden material is the result of insitu weathering of the underlying volcanic rock unit in the area.

The area near the proposed headwork site is overlain by organic rich dark brown thick clay soil overburden material along the left and right bank of the spring. The underlying lithological unit in the area is affected by erosion and transportation to such a significant extent. These geological problems are attributed to the original formation condition of the basaltic rock (vesicular in native), which later are manifested by the actions of various denudation agents.

7.7. GEOTECHNICAL SURVEY

7.7.1. Slope stability

The proposed irrigation area is the valley bottom and river banks. The area around the weir site and the main canal has gentle slope. No earth flow sign was noticed in the site that wetting is not also expected to generate earth flow or slide. Therefore the slope of the weir site and the main canal area are stable.

As far as the specific headwork site is concerned, there is no active fault that liable to disturb the structure to be erected. However, the limestone rock exposed at and around the weir site is vesiculated and affected by fractures and joints

7.7.2 Diversion weir

In order to observe the sub surface geological formations and to investigate the variation in geotechnical properties of the different lithological units exposed at the headwork site with depth engineering geological investigation has been conducted. In addition, to perform this, geological test pits on the left side and right of the riverbank and at the center along the weir axis has sunk. Therefore basaltic rocks are the main rock types that found at the selected foundation site up to depth of 2m.Further more the geology of the main headwork or weir site is given as follow.

- PIT #1; This pit is taken for weir foundation on weir axis at right side bank. It’s found at geographic coordinate location of 37N0631628 and UTM0816847, with an altitude of 1709m.It has a depth of 2m and its subsurface geologic formation and stiratigraphic layer is given as follows;

From 0.00 to 0.30m Weathered top soil

From 0.30m to 0.70m Silt soil with weathered soft rock

From 0.70m to 1.50m Sand with Gravel

From 1.50m to2.00m Pebbles

- PIT #2; this pit is taken for weir foundation on weir axis at left side bank. It’s found at geographic coordinate location of 37N0631616 and UTM0816825, with an altitude of 1707m.It has a depth of 2m and its subsurface geologic formation and stiratigraphic layer is given as follows;

From 0.00 to 0.30m Weathered top soil

From 0.30m to 0.80m Silt soil with weathered soft rock

From 0.80m to 1.50m Gravel and Pebbles with Soft rock

From 1.50m to2.00m rounded river Gravels with Pebbles

- PIT #3; this pit is taken for weir foundation at the center. It’s found at geographic coordinate location of 37N0631623 and UTM0816844, with an altitude of 1703m.It has a depth of 1m and its subsurface geologic formation and stiratigraphic layer is given as follows;

From 0.00 to 0.20m Weathered top soil

From 0.20m to 0.50m Silt and Sand soil

From 0.50m to 1.50m Silt and Sand with Gravels and Cobbles

7.7.3. SUPPLY CANAL ROUTE

7.7.3.1 Geology of main canal

Different lithologies are exposed along/within, and on the area of the, main canal route. Basaltic rocks and soils of different color and character are the main once. The basaltic rocks found at the surface of the study area are weathered and fractured, and eroded and transported by different agents. While the most of it is exposed below the soil of the area, at different depths. The main types of soil of the area are reddish silt and sand and on some places, black clay is also visible.

The main canal is supposed to run on the both side of the riverbank on a sloppy to gently topography. It has a total length of approximately 1.5km which is already excavated and serves as traditional canal.

The excavated traditional main canal has good compacted clay foundation as well as porous and consolidated loose materials of silt and sand in composition .However, in some areas along the main canal route the foundation material is porous and rocky, which is weathered/soft to hard basaltic rock in nature.

Furthermore, to examine the subsurface geological formations that are found along the main canal route, test pits have sunk at different distances from the outlet and the lithological description of  the test pits are given as follows;

7.7.3.2. Left side canal

- PIT   # 1; Geologic test pit on main canal route. It’s found at a geographic coordinate location of 37N0631526, UTM0816762 with an altitude of1707m.It has a depth of 1.5m and its subsurface geologic formation and stiratigraphic layer is as below;

From 0.00m to 0.30m Top Soil

From 0.30m to 1.00m Silt Soil (dark colored & highly weathered)

From 1.00m to 1.5m Sand & Silt with Soft rock

- PIT   # 2; Geologic test pit on main canal route, natural drainage called kimigne area. It’s found at a geographic coordinate location of 37N0631496, UTM0816747 with an altitude of1706m.It has a depth of 1.5m and its subsurface geologic formation and stiratigraphic layer is as below;

From 0.00m to 0.30m Silt & Sand with Gravels

From 0.30m to 1.00m Gravel with Pebble and cobble

From 1.00m to 1.5m Pebbles & Cobbles

- PIT   # 3; Geologic test pit on main canal route. It’s found at a geographic coordinate location of 37N0631471, UTM0816794 with an altitude of1700m.It has a depth of 1.5m and its subsurface geologic formation and stiratigraphic layer is as below;

From 0.00m to 0.20m Weathered Top Soil

From 0.20m to 1.00m Silt & Sand Soil with Soft rock and weathered Gravel

From 1.00m to 1.5m Gravels and Cobbles

At 1.5m Continuous Cobbles and Pebbles

- PIT   # 4; Geologic test pit on main canal route. It’s found at a geographic coordinate location of 37N0631317, UTM0816936 with an altitude of1693m.It has a depth of 1.5m and its subsurface geologic formation and stiratigraphic layer is as below;

From 0.00m to 0.30m Weathered Top Soil

From 0.30m to 1.00m Weathered Silt Soil with Soft rock and Gravels

From 1.00m to 1.5m Silt & Sand with Pebbles and Cobbles

At 1.50m Continuous Pebbles and Cobbles

- PIT   # 5; Geologic test pit on main canal route, natural drainage area. It’s found at a geographic coordinate location of ____________(BM_6).It has a depth of 1.5m and its subsurface geologic formation and stiratigraphic layer is as below;

From 0.00m to 0.70m Sand and Gravel

From 0.70m to 1.50m Weathered Soft rock

From 1.50m to 2.00m Cobbles and Pebbles

- PIT   # 6; Geologic test pit on main canal route, natural drainage called Warabeti flume area. It’s found at a geographic coordinate location of ___________(BM8).It has a depth of 2.00m and its subsurface geologic formation and stiratigraphic layer is as below;

From 0.00m to 0.30m Sand Soil

From 0.30m to 1.00m Gravel with soft rock

From 1.00m to 1.50m Pebbles and Cobbles

From 1.50m to 2.00m Surface Boulders

At 2.00m at Valley bed Hard Rocks, big Boulders

- PIT   # 7; Geologic test pit on main canal route. It’s found at a geographic coordinate location of 37N0631002, UTM0817443 with an altitude of1666m.It has a depth of 1.5m and its subsurface geologic formation and stiratigraphic layer is as below;

From 0.00m to 0.20m Weathered Top Soil

From 0.20m to 1.00m Silt and Sand Soil with weathered Soft rock

From 1.00m to Continuous Soft rock

7.7.3 .3    Right side canal

- PIT   # 1; Geologic test pit on main canal route. It’s found at a geographic coordinate location of 37N0631633, UTM0816937 with an altitude of1695m.It has a depth of 1.5m and its subsurface geologic formation and stiratigraphic layer is as below;

From 0.00m to 0.30m Top Soil Black cotton Soil

From 0.30m to 0.60m Reddish Silt Soil with Sand

From 0.60m to 1.00m Sand with Gravels and Soft rocks

From 1.00m to 1.50m Cobbles and Pebbles

- PIT   #2; Geologic test pit on main canal route, flume area. It’s found at a geographic coordinate location of 37N0631735 and UTM0816907 with an altitude of 1702m.It has a depth of 1.5m and its subsurface geologic formation and stiratigraphic layer is as below;

From 0.00m to 0.30m Top Soil

From 0.30m to 1.00m Sand and Gravel

From 1.00m to 1.5m Gravel and Pebbles

At 1.50m Hard rock

- PIT   # 3; Geologic test pit on main canal route. It’s found at a geographic coordinate location of 37N0631740, UTM0816988 with an altitude of1697m.It has a depth of 1.5m and its subsurface geologic formation and stiratigraphic layer is as below;

From 0.00m to 0.30m Top Soil

From 0.30m to 0.50m Silt Soil (dark colored & highly weathered)

From 0.50m to 1.00m Reddish-Brown Silt

From 1.00m to 1.50m Silt and Sand with Soft rocks

At 1.50m Soft rocks

- PIT   # 4; Geologic test pit on main canal route. It’s found at a geographic coordinate location of 37N0631706, UTM081817143 with an altitude of1694m.It has a depth of 1.5m and its subsurface geologic formation and stiratigraphic layer is as below;

From 0.00m to 0.30m Sand with Silt

From 0.30m to 1.00m Sand with Gravels

From 1.00m to 1.5m Pebbles, Cobbles and Boulders

At 1.50m Hard rock

- PIT   # 5; Geologic test pit on main canal route. It’s found at a geographic coordinate location of 37N0631669, UTM08180 with an altitude of1692m.It has a depth of 1.5m and its subsurface geologic formation and stiratigraphic layer is as below;

From 0.00m to 0.30m Sand with small Gravels

From 0.30m to 0.80m Sand with Big Gravels and Pebbles

From 1.00m to 1.5m Pebbles and Cobbles

At 1.50m Hard rock

- PIT   # 6; Geologic test pit on main canal route. It’s found at a geographic coordinate location of 37N0631780, UTM0817350 with an altitude 1683m.It has a depth of 1.00m and its subsurface geologic formation and stiratigraphic layer is as below;

From 0.00m to 0.20m Top Soil

From 0.20m to 1.00m Reddish Silt and Sand Soil

At 1.00mSoft rock

- PIT   # 7; Geologic test pit on main canal route. It’s found at a geographic coordinate location of 37N0631724, UTM0817431 with an altitude of1680m.It has a depth of 1.50m and its subsurface geologic formation and stiratigraphic layer is as below;

From 0.00m to 0.30m Silt with Sand Soil

From 0.30m to 1.00m Sand with Gravels

From 1.00m to 1.50m Continuous Sand and Gravels with Soft rock

At 1050m Soft rock

- PIT   # 8; Geologic test pit on main canal route. It’s found at a geographic coordinate location of 37N0631727, UTM0817646 with an altitude of1678m.It has a depth of 1.5m and its subsurface geologic formation and stiratigraphic layer is as below;

From 0.00m to 0.30m Overburden material

From 0.30m to 1.00m Silt and Sand with Weathered rocks

From 1.00m to 1.50m Hard rock

- PIT   #9; Geologic test pit on main canal route. It’s found at a geographic coordinate location of 37N0631848, UTM0817608 with an altitude of1677m.It has a depth of 1.5m and its subsurface geologic formation and stiratigraphic layer is as below;

From 0.00m to 0.30m Sand and Silt Soil

From 0.30m to 1.00m Soft rock

From 1.00m to 1.50m Hard rock

7.8. GEOLOGICAL STRUCTURES

From the field observation made and geological map of the area, different geological structure have been identified. Geological structures such as jointing of rock (both the systematic and the non-systematic ones), bedding, and inclination/ tilting of beds were identified from the field observation. Besides these, faulting and folding are the major geological structures that are sorted from the geological map of the area.

## 7.9. ENGINEERING GEOLOGICAL CONDITIONS

The major geological problems that could be expected to occur at the selected diversion weir site are siltation and leakages, mainly, which can respectively cause reduction in the storage capacity and also liable to increase the pore water pressure. These problems are also possible to occur along the main canal route.

As far as the specific headwork site is concerned, there is no active fault that liable to disturb the structure to be erected. However, the limestone rock exposed at and around the weir site is vesiculated and affected by fractures and joints.

  - 7.10. CONSTRUCTION MATERIAL ASSESSMENT

Surface geological map and assessment of availability of construction material at proximal distance to the hydraulic structure sites is one of the major tasks in the geological and geotechnical investigation. Consequently, the major construction materials such as sand, rock for coarse aggregate and building are identified.

## 7.10.1 Coarse aggregate and building stone

A good quality and huge quantity basaltic and sedimentary (limestone) rock that can be used as a construction stone and needs some processing and some excavation works is located on sloppy ridges that found near the main road side of about 2km from head work site. And also some sedimentary rocks that used as construction material is found within main canal routes.

## 7.10.2 Sand

The area is characterized by insitu weathering and formation of residual soils which is reddish in color and predominantly clayey material. Sand material suitable for construction within Balo_Aebe valley( Wabe valley) is not available. However, Welmel valley sand is abundant with the best quality (quartz sand) formed from the weathering of gneiss.

Therefore, a good quality and tremendous amount of sand used for construction purposes at the head work site and main canal route will be brought from Welmel valley around the village name Melka_Amana of the Dallo_Menna district .The quarry site at which this deposit is found is a bit far from the project area i.e. around 270km away from the headwork site.

## 7.11. RECOMMENDATION AND CONCLUSIONS

The Riverbed at which the proposed headwork structure is supposed to be erected is mainly composed of the loosely deposited pebbles, cobbles and boulders as the components of the foundation material. Hence, to protect seepage through the bed and at the same time to control the uplift pressure that could possibly occur on to the structure to be erected up on these materials .It is advisable to excavate them  up to the depth where a consolidated and stable foundation materials is found  if these material are deposited at such minimum depth or excavating a portion of these materials and grouting the bed with the appropriate admixtures.

Most of the length of the main canal runs on massive rock at right side that can be used for construction purpose at the main canal route, where it should necessarily be lined. In addition this plays a major role in the reduction of the overall construction cost.

The basaltic rock that is identified as a construction stone to be used for construction work at the headwork site is not accessible to transport it from the quarry site to the headwork site. Hence, an appropriate road should be constructed to transport construction stone from the quarry site to the headwork site.

Overall, the engineer should make a close observation during the design and construction of both the headwork and main canal and should take the necessary safety factors to be on the safe side.

# 8.ENGINEERING

8.1 GENERAL INFORMATION OF BALOHABEBE SSIP DESIGN

The project is traditionally used by the farmers of Balo Habebe P.A.

The existing command area that used by farmers traditionally during the project study was not more than 65 hectares. The proposed command area for which this project designed is 80 hectares. The minimum design discharge measured during the feasibility study of the project for the whole system is roughly estimated 81 l/sec, (the discharge measured during the project study), but the discharge during the six rainy months in which the ground water provides the required flow will be greater than this value. Because the site is found in GASARA KOLa Gorge and the recharging rate of the springs highly increase during the rainy seasons of the district high lands. The duty of the project taken for the design of the structure is 2.4l/s/ha from agronomy report.The designed discharge for the different canals of the project is described in the project system design

### 8.1.2 Methodology of data collection

- Spring Eye charging point coordinate was taken using GPS & this data was used as input for total station for the fixation of BM.

- Project site Top graphic data was collected using Total station

- Main canals & other network structures line profile data was also collected using Total station.

Note - GPS North reference was adjusted (during setup) on the True North and UTM system was adopted for data collection for project detail study.

### 8.1.3 PROJECT HEAD WORK

- There are so many spring water which flows out to the surface in the vicinity of project area due to the existing Topographic nature of the watershed namely Gasera Kola gorge.

- The specific name of biggest Spring selected for detail study is Tebela Spring;

(With the UTM co-ordinates of 631606.58E, 816853.51N and at an altit.1701.51m.a.s.l taken at spring eye)

- The structures proposed for the head work is a simple one sideopen collection chamber or channel as a guide wall canal constructed on the downstream of the spring Eye.

- The irrigation water can be simply guided in to the proposed canal on lined canal and leads to the command area through the proposed two main canals (Right and Left Main canals) combined with their conveyance structures.

- The different surface flows can be collected in to the required directions of the command areas through the lined canals.

### 8.2 DESIGN OF PROJECT SYSTEMS AND ITS STRUCTURES

  - DESIGN OF PROJECT SYSTEMS

8.2-1 Design of One Side Guide wall/ Collection Chamber

8.2.1.1 Introduction

The design of the Guide wall is taken to be in a range of minimum requirement and topographic limitations. In addition to the main Guide wall, the design of complementary structures will also be in consideration to satisfy the requirement to supply design discharge to the command area.

Therefore the guide wall mainly serves as guide the flow of water to the two main canals that extends to the command area. It will also serves as temporary storage of water.

8.2.1.2. Guide wall design

0.5m

1.5m

GL

0.7m

1m

1m

The water depth up stream of the guide wall is expected not to exceed 1.5m. From the spring capacity and the demand of the water for the command area, also shows that the water level up stream guide wall will not exceed 1.5m. Since, the left bank of the spring area is topographically vertical slope the construction of this guide wall to the right bank will make the enclosed section to serves as collection chamber.

The extent of the guide wall starting from the spring eye starches to the main canal one intake. The height of the guide wall gradually decreases and terminate at the intake of canal one or left canal bank. In case of main canal two or right side canal, the intake is located at the base of the guide wall, and departed from the guide wall by drop structure which will be manual gate operated or regulated.

8.2.1.3 Guide wall stability

Stability of any retaining structure is mainly compromised due to load that exists due several modes. Modes of failures of such structure may take in three forms.

- Overturning; due to moment generated by the forces on the structure, the structure may face probable overturning failure. This is due to the unbalanced negative and positive moments. The safety factor for overturning mathematically can be defined as;

For the stability of the structure against overturning the safety factor value can be >1.25 for acceptable value and greater than or equal to1.5 of desirable value

- Sliding; the ratio of summation of horizontal forces to summation of vertical forces is a defining ratio to structural stability against sliding.

The Fss value range for stable structure is less than or equal to 0.75.

Once we defined possible modes failure of the structure we need to determine the forces on the structure to proof whether the structure is stable or not.

8.2.1.4 Load calculation

Primary loads on guide wall are

- Upstream water load

- Active lateral earth pressure

- Self-weight

Pw	W2	Psv

W1	Psd

Psu

Pu

Upstream water load; since we have small amount of water storage on the upstream of the guide wall, there will be load imposed due to this stored water. As stated earlier in this section the water storage on the upstream is expected not to exceed 1.5m.

Water load is calculated as;

Where; H- upstream water depth

- Unit weight of water (which is equals to 9.81KN/m3)

Active lateral pressure; the load due to soil is of two types, having vertical and horizontal alignment.

- Horizontal component; known as active lateral pressure and can be calculated by

Where; Ps- active lateral pressure

- Depth of soil

- saturate unit weight of the soil (where in this case have the value of 19.2 KN/m3)

Ka – active lateral pressure coefficient

- shear resistance angle (for our case 30o)

- Vertical component; the soil on the inclined surface of the downstream face of the guide wall will impose a vertical force. The magnitude of this force can be determined easily by calculating the volume of the soil and multiplying by the unit weight of the soil.

Mathematically can be presented as;

Psv= Where; - Unit weight of the soil

Vs- Volume of soil

Self-weight load; any matter in the world which can occupy a volume has a mass. And where there is gravity every object having mass will certainly have weight. This dead load is calculated by determining the volume of masonry and multiplying by unit weight of the masonry.

W=Where; - Unit weight of the masonry

Vc- Volume of masonry

| Load name |  | Magnitude (kN/m) |  | Moment arm to the toe of the guide wall | moment |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | vertical | horizontal |  |  |  |  |
|  |  |  |  |  | -veM | +veM |  |
| Self-weight | W1 | 22 | - | 0.75 | - | 16.5 |  |
|  | W2 | 11 | - | 0.33 | - | 3.67 |  |
| Water load | Pw | - | 11.04 | 0.5 | 5.52 | - |  |
|  | Pu | -10.791 | - | 0.67 | 7.194 | - |  |
| Soil load | Psu | - | 1.8 | 0.233 | 0.4194 | - |  |
|  | Psd | - | -1.8 | 0.233 | - | 0.4194 |  |
|  | Psv | 2.3 | - | 0.233 | - | 0.536 |  |
| sum |  | 24.509 | 11.04 |  | 13.133 | 21.1254 |  |

Therefore from the above table of load calculation, the load that tend to roll the guide wall are taken as negative moments whereas the stabilizing moments as positive moments. The stability is done for unit length of the guide wall.

8.2.1.5 SUMMARY OF STABILITY ANALYSIS

1. Stability against Overturning

Thus, = 1.61, it is safe.

2. Stability against Sliding

Thus,  = 0.45, it is safe

Gate design

# 8.3. Design of Canals and structures on the canals

One sideopen collection chamber with a length of 15m, height of 1.5m foundation depth and width of 1m respectively is proposed.

1-One sided guide wall with lined trapezoidal main canal one (Left side main canal, 0+00 to 1.345 km)

2-Main canal two departs from the guide wall joined by drop structure (Right side main canal, 0+00 to 1.462k/m)

3-The trapezoidal earthen secondary canal (from DB-1 at chainage of 500m on main canal -1)

3-The trapezoidal earthen secondary canal (from DB-2 at chain age of 530m on main canal -2)

4. The trapezoidal earthen Tertiary canal one (on sc1, 0+00 to 240m after flume on main canal -1)

5. The trapezoidal earthen Tertiary canal two (on mc2, 0+00 to 250m)

These two main canals are counter canals existing and constructed by the beneficiaries before 50 years ago. Due to the existing geographic feature of the command area (undulating and gorgy features of the project site) almost all or more than 95% of thecommand area hassloppy nature and occupied by permanent fruit trees and orchards and coffee plants.Secondary canals are located on the chain age where the beneficiaries constructed it and use it before. In general on secondary and tertiary canals no surveying have been done and no any structures are amended on these canal because of the following reasons:

- Inside the command it is not accessible to construct any kind of structure

- All of the command is occupied by permanent fruit and perennial orchards

- The beneficiaries are not volunteer enough to do any construction work in their farm land as we have discussed with theme during the field survey. Only off-takes and division boxes are designed for these canals at the chainage where they depart from the main canals.

The engineering design is focused only on the one side open collection chamber (guide wall)at the head work, left and right main canals and structures on main canals.

| Balo Habebe irrigation project8.3.1 Design of left side Main canal |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Canal | Length(m) | A(ha) | Duty (l/s/ha) | Q(m3/s) | V | A(m2) | b | m | d | b/d | P | R | n | bed Slope | fb | D | T | Remark |
| MC-2 | 00-10 | 45 | 2.4 | 0.086 | 0.90 | 0.12 | 0.35 | 1.25 | 0.20 | 1.8 | 0.99 | 0.12 | 0.013 | 0.002283 | 0.35 | 0.55 | 1.7 | Lined rectangular |
| MC-2 | 00+10-30 | 45 | 2.4 | 0.086 | 2.92 | 0.04 | 0.25 | 0 | 0.15 | 1.7 | 0.55 | 0.07 | 0.013 | 0.0522 | 0.35 | 0.50 | 0.3 | Chute NO-1 |
| MC-2 | 00+30-90 | 45 | 2.4 | 0.086 | 1.90 | 0.057 | 0.35 | 0 | 0.16 | 2.2 | 0.67 | 0.08 | 0.013 | 0.0165 | 0.35 | 0.51 | 0.4 | Chute NO-1 |
| MC-2 | 90-180 | 45 | 2.4 | 0.086 | 0.90 | 0.12 | 0.35 | 1.25 | 0.20 | 1.8 | 0.99 | 0.12 | 0.013 | 0.0023 | 0.35 | 0.55 | 1.7 | transition |
| MC-2 | 180-260 | 45 | 2.4 | 0.086 | 1.53 | 0.07 | 0.3 | 1.25 | 0.15 | 2.1 | 0.77 | 0.09 | 0.013 | 0.0095 | 0.35 | 0.50 | 1.5 | Lined trapezoidal |
| MC-2 | 270-390 | 45 | 2.4 | 0.086 | 0.90 | 0.12 | 0.35 | 1.25 | 0.20 | 1.8 | 0.99 | 0.12 | 0.013 | 0.0023 | 0.35 | 0.55 | 1.7 | Chute NO-2 |
| MC-2 | 390-520 | 45 | 2.4 | 0.086 | 1.39 | 0.08 | 0.35 | 1.25 | 0.15 | 2.4 | 0.82 | 0.10 | 0.013 | 0.0075 | 0.35 | 0.50 | 1.6 | Lined trapezoidal |

### 8.3.2 Design of right side Main canal

### [9

# 8.4 DESIGN OF PROJECT SYSTEM STRUCTURES

8.4.1 -DESIGN OF DIVISION BOX

Division Boxes

One division box which collects discharge fromthe Spring Eye at the end of collection chamberand leads in to MC1&MC2 at 0+00m &0+005m for the two canals respectively.

The box is gate operated.

Design of division box: (sample design)

Station: 860m (before the flume) &604 on of MC1

Using broad crest formula,

Q=CL (h) 3/2

Were Q=discharge over rectangular weir, m3/s

C= discharge coefficient, =1.7

L= length of crest in meter.

h=over flow depth

Assuming equal discharge coefficient& sill height for the whole division box openings.

The proportional becomes,

Q1/Q2=Q2/Q3=L1/L2=L2/L3

Were Q1=Flow in mc 1

Q2=Flow in mc2

L1= length of crest opening to the canal one 1

L2= length of crest opening to the canal one 2

L1=Q1/C (h) 3/2

L2=L1*Q2/Q1

The depth (height of) the division box=d+fb

The width of the division box=b+2*m*D

Were b=base width of the incoming canal

m=side slope of the incoming canal

D=total canal depth of the incoming canal

Q1=0.086 m3/s (mc1)

Q2=0.58 m3/s (mc2)

L1=Q1/ C (h) 3/2 =0.086/1.7*(0.20)3/2 =0.36m(use h to be 0.20 &L1=0.40m)

L2=L1*Q2/Q1=0.40(0.058)/0.089=0.42m use L2=0.45m

L0=0.75m

Depth of division box, H=depth of parent canal(MC)+ fb(0.4m)

= 0.50m+0.40m=0.9m,use H=1.0m

Width of division box bed width of 2* MC+2*(D*1.2) (1.0) =1.2M, Use 1.8m

| DB | canal | Q1 | Q2 | h | L1 | L2 | T | d | fb | D | b | B | Sat(m) | Gr.Elev | DBL | Remark |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Mc1& mc2 to Sc1 | 0.086 | 0.058 | 0.20 | 0.40 | 0.45 | 1.6 | 0.6 | 0.4 | 1.0 | 0.30 | 1.8 | Mc1& 2 | 1758.1m a.s.l | 1757.18 |  |

Table of dimension of division box

Note; Refer division box drawing

FIG 2-TYPICAL DIVISION BOX PLAN

A	        0.35M

1.5m

B	0.15m

B

b	      L	b

L1

L2	0.15m

T=0.30m

A

fb

D

d	h

L1

Fig 3   SECTION    A-A

### 8.4.2 Design RCC flume

There isone flume on Balo Habebe SSIP. One flume is found on the main canal one on WARABETI stream .

# 8.4.2.1Characteristics of Mother canal

Q = 0.089m3/s b = 0.35m, d = 0.2m,

fb =0.25m, D = 0.45m

m = 1:1.25, T (B) = 0.75m, V = 1.33m/s n = 0.013

A. Hydraulic Design

  - Flume section

- flume length, L = 13mdescribed in the flume table of dimension  (Refer flume drawing

- Discharge, Q = 0.089 m3/s and let Velocity of flow, V= 1.330 m/s and b = 0.35m

- Wetted area, AW = QV=0.089/1.33=0.064m2

- From this, the water depth d= 0. 064m2/0.35=0.20 m, and use fb = 0.25m

D= d + fb = 0.20+0.2 = 0.45m

Fruid No,

Since 0.95> 1, the flow is sub critical flow, then it is ok

* Wetted per meter (Pw) = 0.35 +0.20*2 =0.75m

- Wetted  area (AW )  =0.067m2

- Hydraulic Radius  ( R )

- Slope ,

- Friction head loss hf = L* s, where S = 0.0075 and L= length of flume which is described in flume table of dimension.13×0.0075=0.095m

B  Structural Design of Slab

      - Input Data

- Allowable stress in concrete = 55 kg/cm2

- Allowable shear stress in concrete = 9.5 kg/cm2

- allowable stress in reinforcement = 1400 kg/cm2

- unit weight of water = 1 ton/m3

- unit weight of Rcc = 2.5 t/m3

- unit weight of concrete = 2.4 t/m3

- unit weight of masonry = 2.0 t/m3

- unit weight of steel = 1.15 t/m3

- characteristic strength of normal steel , fy = 415 N/mm2

- characteristic strength of normal concrete, fck = 20 N/mm2 ( ie for grade 20 concrete with 1:1.5:3 ratio)

- characteristic strength of the stirrups, fyy = 250N/mm2

- partial safety factors for both live and dead loads, Fs = 1.5

- the minimum cover (nominal ) against main bars is 25mm and that of stirrups is 15mm

- Single RCC is used.

8.4.2.3. Load Analysis

- Supper imposed load of the two side walls with the 0.15m thick

- Live load  = Silt load + water load, taking silt load 5% of water load

= b*d * ﻻ w * 1.05 = 0.35 * 0.2 * 1 *1.05 = 0.0735 t/m

- Let the span of the beam  = 5.0m and thickness, t= 0.20m and width of the beam

= (0.2+0.2) +0.35= 0.75m

-DL=0.75*0.20+(0.45*2*0.2)=0.31t/m

- Self-weight of the beam, Wm  = B* t * ﻻc = 0.75 * 0.2 * 2.5 = 0.375 t/m

- The design load ;  Wd  = DL + LL + Wm = 0.31 + 0.0735 +0.375=0.7785 t/m

8.4.2.3. Bending Moment Analysis

The max Bending moment which can force the simply supported slab to collapse will be at the middle span which is 5m apart is

Collapse moment (Mc)   = t/m

Equating the Balance moment to Collapse moment

Mu= Mc to find the effective depth (d) of reinforcement.

Kbd2 fck =mc

d= (mc/ Kb fcr)1/2    = ( 3.649/  0.138* 0.75 * 2000) ½ = 0.132m

An overall depth of the slab from deflection consideration using a 10mm reinforcement bar placed at a clear caver of 2.5 cm is

D= t + (12/2) + 25 = 0.15 +(0.012/2) + 0.025 =  0.18m

From which Mu = Kb d 2 fck = 6.706 t/m

Since the resisting moment mu > Mc , the max collapse moment for the effective depth of the beam is 0.132m

8.4.2.3.1The balanced steel required for the moment.

- percentage of area of steel, Pt, ie the percentage balanced steel

Pt = 19.82fck/fy = 19.82*20/415 = 0.96%

Pt> 0.80% (the minimum balanced steel percentage)

Since 0.96>0.80% shows the member is neither over reinforced nor under reinforced.

a,  Ast (Area of steel reinforcement)

Ast =where   Z = d (1-0.416 * x/d)

Where Z = Lever Arm depth

X = the depth of neutral axis

d= effective depth of reinforcement

X =0.75 – ((0.75)2 – 3.649 mu / fck bd 2) ½ = 0.5497

d

Z= 0.131 (1-0.416 * 0.5497) =0.102m

Ast =

If 12 No of 14 mm Ф at 125mm is used, Ast = 7*(14)2 * π/4 = 1847mm2

Since 1847mm2 >1820 mm2 it is ok

b, Area of steel reinforcement, Ast

Ast = Mu/0.87fyz, where z = d(1-0.416* x/d)

z = is the lever arm depth

x= is the depth of neutral axis

d = is effective depth of reinforcement

x/d = 0.75 - [(0.75)2 - 3.645*6.706/fckbd2]1/2

= [0.75)2 - 6.68*7.6/2000*0.75*(0.132)2]1/2 = 0.5497

Z = 0.132 (1 - 0.416*0.549) = 0.109m

As = Mu/0.87fy*z = 6.706/0.87*41500*0.109 = 1763 mm2

- If 12 number of Φ14 mm @ 110 mm is used, the area of steel, Ast = 12x (14)2x π/4= 1847 mm2 Since 1847 mm2  > 1763 mm2  , it is ok!

c,   nominal shear stress, τ = v/b d

where,	 b = effective width of the beam

d = effective depth of the beam

V = shear force

τ = nominal shear stress

V = 1.5 (W d) * 5 = 1.5 * 0.75 * 5 = 5.25t

d, nominal share stress, J = V/b d = 5.625   = 41.66 t/m2

0.75*0.181

The max shear stress in concrete, J max is approximated as 95 t/m2 , since 95 t/m2 > 41.66 t/m2 the member is safe.

### 8.4.2.3.2, Design for links (stirrups)

The general principle for the design of stirrup is that the spacing should not exceed 0.75   and the diameter of the stirrup should not less than 6mm   If ø 8 mm with the spacing of 150mm is used , the cross sectional area of the beam b/n the two stirrups   b* su is  = 1000 mm * 150 mm  = 150, 000 mm2

The area of stirrups Asu = 0.35 b su /0.87 * fy = 145.4mm2

Since 145.4mm2< 150,000 mm2 it is Ok

a, Design of pillar or column

Since total span of the flume is short only one column at the center of the stream required

- Design data

Max height of pillar 3.m

Characteristics strength of concrete fck = 20 mpa

Direct crushing strength of concrete fcc = 8mpa

‘’               ‘’            ‘‘Reinforcement, fsc = 278 mpa

Yield or proof strength of steel,  f4  = 4/5 map

L= Fsc / fcc = 278 /8 = 24.75

- Partial safety factor for both live and dead load is 1.5

2) Load on the pillar

* Axial load, Dl = Weight of the beam above the pillar  + side walls

= 0.75*5.2 * 0.18 * 2.5 + 0.45 * 0.15 * 5.* 2 = 2.625t

- Live load, L1  =b*d * L* ﻻw * 1.05 = 0.55 t

- Self weight of the pillar (wm)

let the size of the pillar, D = 0.40m & b = 0.40m

Wm = D*b*ﻻ e * l  = 0.4*0.4*2.5 t/m3 * 3.m  = 1.40t

The design load P =  F.s   (DL  + L2 + Wm)

= 1.5 * (2.25 + 0.55 + 1.40)

p = 6.3t

3) Moments

The moment due to the design load (p) at the toe of the pillar

Mdm = F.S* P * 0.4 =1.9 tm

2

4) Structural Analysis

a) Slenderness ratio

As the pillar is fixed at only one end

Le = effective pillar length = 3m and     Le  =   3.   =7.5

D       0.4

Since Le / D < 12 the pillar can be considered as short pillar

b) Eccentricity Consideration

1) Considering the long direction D = 0.40m

Lmin  = Le    + D   = 2500 + 300  = 16.25

400      30      400      30

the  accidental eccentricity , e min  must  be greater than the minimum  eccentricity  (20mm) i.e  16.25   < 20, take  e min  = 20.33

Then e,min / D  = 16.25   = 0.054

300

- Considering the short direction

e min = 2500/500+200/30 = 11.66 mm<20

and emin/b = 11.66/200 = 0.058>0.054

Since the accidental eccentricity for both lateral dimensions is not less than 5% consequent, eccentricity has to be checked.

- Consequent eccentricity, lc

lc  = Md           =         1.90

p(fcrbo2)      (2000*0.2 * (0.4)2)  = 0.03

P/fek  bD   = 6.3/(2000*0.4*0.2)  = 0.039

The corresponding point of md/fek bd 2 verses P/fekbD for P/fck = 0.1 is inside the interaction curve, the pillar is safe.

4) Amount of the reinforcement steel required

P/fex  = 0.31, P =  20 * 0.31 = 6.2%

- Load carrying capacity of the pillar for the proposed size.

= Ac*fck = 0.4 *0.20 * 200 =16t >> 6.3 (the design load) therefore, the pillar has to be reinforced with the minimum steel % which is 0.8 %

Ast  =  pmin * Ac/100  = 0.8 * 400 * 200 = 640mm2

100

If 4 No of Ø 16mm is used at each corner]

Total Ast = 4* 162mm * π = 803.84mm2

4

Since 803.84mm2 > 640mm2 it is Ok.

  - Eccentricity considerations

- Considering the long direction, D = 0.40m

emin = Le/500+D/30 = 2500/500+300/30 = 15mm

The accidental eccentricity, emin = 15mm is greater than the minimum specified eccentricity = 20, there fore take emin = 20.33

Then e min/D = 15/300 = 0.05=0.05

- Considering the short direction

e min = 2500/500+200/30 = 11.66 mm<20

and emin/b = 11.66/200 = 0.058>0.05

Since the accidental eccentricity for both lateral dimensions is not less than 5% consequent, eccentricity has to be checked.

- Consequent eccentricity, lc

lc  =Md           =         1.90

p(fcrbo2)      (2000*0.2 * (0.4)2)  = 0.03

P/fek  bD   = 6.3/(2000*0.4*0.2)  = 0.039

The corresponding point of md/fek bd2 verses P/fekbD for P/fck = 0.1 is inside the interaction curve, and therefore the pillar is safe for its symmetrical distribution of the steel.

4) Amount of the reinforcement steel required

P/fex = 0.31, P =  20 * 0.31 = 6.2%

- Load carrying capacity of the pillar for the proposed size.

= Ac*fck = 0.4 *0.20 * 200 =16t >> 6.3 (the design load) therefore, the pillar has to be reinforced with the minimum steel % which is 0.8 %

Ast  =  pmin * Ac/100  = 0.8 * 400 * 200 = 640mm2

100

If 4 No of Ø 16mm is used at each corner]

Total Ast = 4* 162mm * π = 803.84mm2

4

Since 803.84mm2 > 640mm2 it is Ok.

5. Design for the Transversal steel

The following limitation should be considered .The dia of the link- should be at least ¼ of the longitudinal steel i.e ø 16mm  = 4 mm or not less than  5mm, use 8mm ø 4

- The spacing should not exceed the least of the following

a)  The least lateral dimension of the pillar = 200mm

b) 16 times the smallest longitudinal bar = 16*16 = 256mm

  - forty eight times the diameter of the links = 6x48 = 288mm

Use ø 8 with the spacing of 200mm C/c

2.10 Design for foundation

1. Design data

Total axial load = 6.3 t

Grade of concrete, M20 = 20N/mm2

Grade of steel, Fe = 415 N/mm2

Allowable stress in concrete J cb = 7 N /mm2

‘’             ‘‘Steel Jst = 140 N /mm2

Soil bearing capacity = 30 t/m2 for cohesive hard clay.

Thickness of pillar above foundation = 400 * 200mm

2) Design co-efficient

N= 0.4, J 0.87, K = 0.174, Pt – 1%, J = 0.62 mm2 for the selected grade

3) Moments

Let the depth of foundation below OGL be 1.0m

Total supper imposed load = P= 6.3t

Assuming 1.25 * 1.25m foundation base.

Self weight Mf = B* D* ﻻc * h = 1.25 * 1.25*2.5 t/m3 *1 = 3.9t

Wf = P+ self weight = 6.3t+3.9t = 10.20t at 0.75m

- The total design moment transferred by the pillar above the found. Is Mf = 3.9

As the horizontal forced due to flood water 0.42t/m and the up lift pressure, U= 0.3* 0.42 * Lt/m3 =0.126t/m

Check for over truing moment   = Mf

(0.637+ (0.126*(0.3) = 6.0 >>1.5,it is safe.

3

- The maximum bearing stress

δ max = W/A+ML/2I,

δ max = 10.20/1.56+3.9/2*0.203=16.14t/m2

where I = LB3/12 =2.44/12 = 0.203

## I = moment of inertia of the base area

L = length of the foundation

B = base of the foundation

Wf = superimposed load + self weight

A = area of the base foundation

For the design condition that the maximum stress should be less than     the allowable value gives Wf/A + Mf*L/2I < Pa, where Pa is the allowable bearing capacity of foundation soil & Pa = 30 t/m2

16.14 t/m2 < 30 t/m2 - & therefore the size of the base foundation is safe.

1. Design of Reinforcement

    - Tension steel

Ast  = Mf/j*d*δst = 3.9/(0.8*1*14000t/m2) = 348.2 mm2

if 4,Ø12 mm @300 mm is used, the area of reinforcement

Ast = 4*(12)2xπ/4 = 452 mm2

Since 452 mm2 > 348.2 mm2,  it is ok!

    - Distribution reinforcement

Asd = 0.2*Ast = 0.2*452 = 90 mm2

If 4,Ø8 mm @300mm is used, Asd= 4*(8)2xπ/4 = 100 mm2

Since 100 mm2 > 90 mm2.  It is ok!

Table of dimension of Flume (on main  canal )

| flume1 | canal | Q1 | b | t | L | v | B1 | t0 | h | t1 | d1 | d | fb | D | bc | B | T | Gr.Ele at inlet | DBL at inlet | Remark |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | MC1 | 0.086m3/s | 0.35m | 0.15m | 13m | 1.34 m/s | 1.5m | 0.75m | 1.0m | 0.40m | 0.50m | 0.21 | 0.25m | 0.45m | 0.35m | 0.35 | 0.35m | 1685.661 m a.s.l | 1685.581 |  |

Fig 9.  Flume in plan

L

G

t

b                                              B                                                                           bc

fig 10  Section of flume

0	  3.5              3		3		3.5		2

1685.58

1684.508d1	t1

3m

1684.508

1682.542

1681.622

to                                            h        to

B1

0                  3.5	3	3	                 3.5

0.15m

3cm plastering                                   0.5                      10cm plain concret(1:1.5:3)

(1:3)                                 0.55                                     12mmØ at 13cm c/c                                                                                 0                                                               0.20

1.5m                0.5

12 No of 14mm@125mm

0.40	C/c for the beam slab

16mm Main bar

OGL	6mm Ø stirrup (20cm c/c)

1m

o.40m

12mm Ø at 13cm c/c

# 8.5 DESGIN OF FOOT PATH

8.5.1 (SAMPLE DESIGN OF FOOT PATH)(On Mc &scs2)

##### Design data

Canal characteristic before footpath.

Q= 0.086m3/s V = 1.330m/s, b = 0.30m, d = 0.14m, fb = 0.35m, D= 0.49m

T= 1.5m, m= 1.25, n = 0.013

1) Hydraulic design of foot path

A 2.6m long R-cc that is 0.15m thick is made to be supported by a 30 cm thick masonry wall the slope of the box under F.P is similar to that of the lined canal and the concrete is single reinforced and a main bar of 12mmØ at 150mm c/c and distribution bar of Ø 10mm at 180mm c/c is used. In general there are four-foot paths on mc1 that can serve vehicle, human and cattle to cross the canal. Using a rectangular box under the footpath

Q = 0.089m/s , V = 1.3410m/s , b = 0.35m, and   A  =  Q   = 0.08m2

V

d=   A = 0.08m2

b      0.35m

d =0.14m

Use fb = 0.39 to easily remove the silt accumulation and D= d + fb = 0.50m, N =0.023

Slope   S = (nv) 2, Pw = b+2d = 1.0m, R = A = 0.08m

R 2/3	  P

So, S = (0.023 * 0.71)2 = 0.0014

(0.08)0.66

2) Structural design of footpath.

As 2.6m long R.C.C concrete slab of 15cm thick lies on the 30 cm thick wall of canal with a single reinforcement bar of Ø12mm at 150 mm c/c main bar and a distribution on bar of Ø 10mm at 180mm c/c.

For its detail of structural parts refer footpath drawings

Table of dimension of foot path

| symbol | L | L1 | L2 | L3 | W2 | b | D | Do | bo | d2 | to | t1 | t |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| dimension | 2.6m | 1.5m | 1.5m | 1.2 | 2.25m | 0.30m | 0.40m | 0.50m | 0.75m |  | 0.15m | 0.20m | 0.30m |
| Remark |  |  |  |  |  |  |  |  |  |  |  |  |  |

FIG 10. Plan of foot bridge

t1

L2             L3	L	L4	L1           t1

D

t

t

W2

bo

b

t

D

To	t

To

Do

bo

0.40m

0.40m

Reinforcement bar arrangements

12mmØ at 15cm c/c

10mmØ at 18 cm c/c

5cm concrete cover

5cmconcretecover

### .5 Design of Drop

There are 5 number of drops on MC1, 9 number of drops on MC2, 1 drop on MCo,

Its hydraulic &structural analysis are as following

TYIPICAL DROP DRAWING (IN PLAN)

t2

tt            t

s

t

b1                                                         t

bc	B	b2

L

s

L1	t	L	t	L2

D

d

t1	H                                                                                                      D

L                                                                                               t

a

t1

0.30m

40cm	40cm

|  | Type 1 a. given |  |  |  |
| --- | --- | --- | --- | --- |
| SN |  | Design of Drop Structure on MC- 1 | Value | Remark |
| 1 | Discharge(Q) | m3/s | 0.086 |  |
| 2 | U/S water Depth(h1) | m | 0.21 |  |
| 3 | U/S Velocity(v1) | m/s | 0.68 |  |
| 4 | D/S Water Depth (h2) | m | 0.21 |  |
| 5 | D/S Velocity (v2) | m/s | 0.68 |  |
| 6 | Drop Height(z) | m | 0.75 | <- 1forQ<2 |
|  | Description | Symbol | Formula | Computation |
| 1 | Drop Width | bc | 0.734Q/(h11.5) | 0.419497916 |
| 2 | Unit Discharge | q | Q/bc | 0.131109114 |
| 3 | Critical depth | hc | (q2/g)(1/3) | 0.120558801 |
|  | C.Stilling Basin |  | 0.03 |  |
|  | Description | Symbol | Formula | Computation |
| 1 | Basin width | B | 18.46(Q)0.5/(Q+9.91) | 0.45 |
| 2 | Length | L | [2.5+1.1(hc/z)+0.7(hc/z)3](z*hc)0.5 | 0.9511257 |
| 3 | Lip Height | a | hc/2 | 0.0802794 |
|  | Type-2 |  |  |  |
|  | d. Stilling basin |  |  |  |
|  | Description | Symbol | Formula | Computation |
| 1 | U/S Energy level | He | h1+(v12)/(2g) | 0.221258919 |
| 2 | Energy loss | F | He+z-h2-(v22)/(2g) | 1 |
| 3 | Basin width | B | 18.46(Q)0.5/(Q+9.91) | 0.4935 |
| 4 | Length | L | 3(He*F)(1/2) | 1.411145023 |
| 5 | Depth | d | 0.5(He*F)(1/2) | 0.235190837 |
| 6 | U/S Protection Work | l | 1.2+1.5Q(1/2) | 1.551781182 |
|  | Drop on Canal | Drop ht | Drop width | Basin Lengith |
|  | MC-1,MC2, | 1m | 0.65 | 1.51 |
|  |  | .75m | 0.57 | 1.05 |
|  |  | .75m | 0.57 | 1.0 |
|  |  | .5m | 0.57 | 0.90 |

### 8.6 SAMPLE DESIGN OF Off Takes

Off- takes of Balo Habebe SSIP is designed for gate controlled flows in which the land to be irrigated will get irrigation water during its irrigation interval by rotation. This option is preferred to simplify the defect of the continuous flow through circular pipes at the beneficiaries (farmers) during the project implementation.

There are two gates at every off takes

The dimension of the free turn out to the irrigated land is 0.4m *0.4m.

For its detail refer typical off take drawing

Chute design

Table 8-3 Hydraulics Design of chute on Main Canal -1 Left

chute On MC 1

| chainage | B(m) | d(m) | Q(m3/s) | S | S3/4 | B*d | B+2d |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 780-860 | 0.2 | 0.14 | 0.089 | 0.0833 | 0.155 | 0.03 | 0.481 |
| 230 - 300 | 0.2 | 0.15 | 0.089 | 0.0680 | 0.133 | 0.03 | 0.508 |
| 300-480 | 0.35 | 0.128 | 0.089 | 0.0250 | 0.063 | 0.04 | 0.606 |

| [n*Q/B*d]3/2 | Left | Right | fb | D (m) | V (m/sec) | Remark |
| --- | --- | --- | --- | --- | --- | --- |
| 0.0093 | 0.058 | 0.060 | 0.32 | 0.46 | 3.1622 | chut no-1 |
| 0.0081 | 0.061 | 0.061 | 0.32 | 0.47 | 2.8831 | chut no-2 |
| 0.0046 | 0.074 | 0.07 | 0.32 | 0.45 | 1.9821 |  |

Canal chute part

| chainage | Q | d | bc (m) | dc | fb | Dc | Ac | Vc |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 780-860 | 0.086 | 0.45 | 0.22 | 0.26 | 0.16 | 0.42 | 0.06 | 1.59 |
| 230 - 300 | 0.086 | 0.45 | 0.22 | 0.26 | 0.16 | 0.42 | 0.06 | 1.59 |
| 300-480 | 0.086 | 0.45 | 0.22 | 0.26 | 0.18 | 0.44 | 0.06 | 1.59 |

| Hvc | Pc | Rc | Ic | Z (m) | L(m) | sc |
| --- | --- | --- | --- | --- | --- | --- |
| 0.129 | 0.73 | 0.08 | 0.0154 | 6.664 | 80 | 0.015392 |
| 0.129 | 0.73 | 0.08 | 0.0154 | 4.76 | 70 | 0.015392 |
| 0.129 | 0.73 | 0.08 | 0.0154 | 4.5 | 180 | 0.015392 |

| chainage | Qsp | Ec | d1 | b1 | A1 | V1 | Hv1 | P1 | R1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 780-860 | 0.086 | 7.05 | 0.108 | 0.22 | 0.023 | 3.81 | 0.74 | 0.43 | 0.05 |
| 230 - 300 | 0.086 | 5.15 | 0.117 | 0.22 | 0.025 | 3.50 | 0.63 | 0.45 | 0.06 |
| 300-480 | 0.086 | 4.89 | 0.187 | 0.22 | 0.040 | 2.20 | 0.25 | 0.59 | 0.07 |

| I1 | Im | hf1 | E1 | 5 | d2 | Lb | Bb | a |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0.14 | 0.077 | 6.19 | 7.03 | 3.70 | 0.51 | 2.7 | 1.5 | 0.1 |
| 0.11 | 0.064 | 4.45 | 5.19 | 3.26 | 0.49 | 2 | 1.5 | 0.1 |
| 0.03 | 0.025 | 4.43 | 4.87 | 1.62 | 0.35 | 2 | 1.8 | 0.1 |

Table 8-3 Hydraulics Design of chute on Main Canal -2 Right

| chainage | B(m) | d(m) | Q(m3/s) | S | S3/4 | B*d | B+2d |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 00+10-30 | 0.35 | 0.19 | 0.108 | 0.183 | 0.279 | 0.02 | 0.488 |
| 00+30-90 | 0.325 | 0.17 | 0.108 | 0.022 | 0.057 | 0.06 | 0.665 |
| 520-580 | 0.325 | 0.11 | 0.108 | 0.066 | 0.131 | 0.04 | 0.545 |
| 580-680 | 0.39 | 0.16 | 0.108 | 0.015 | 0.044 | 0.06 | 0.71 |
| 680-740 | 0.255 | 0.15 | 0.108 | 0.06 | 0.121 | 0.04 | 0.555 |
| chainage | Q | d | bc (m) | dc | fb | Dc | Ac |
| 00+10-30 | 0.108 | 0.4 | 0.31 | 0.23 | 0.16 | 0.39 | 0.07 |
| 00+30-90 | 0.108 | 0.29 | 0.51 | 0.17 | 0.16 | 0.33 | 0.08 |
| 520-580 | 0.108 | 0.29 | 0.51 | 0.17 | 0.18 | 0.35 | 0.08 |
| 580-680 | 0.108 | 0.29 | 0.51 | 0.17 | 0.16 | 0.33 | 0.08 |
| 680-740 | 0.108 | 0.29 | 0.51 | 0.17 | 0.14 | 0.31 | 0.08 |

Canal part in chute

| [n*Q/B*d]3/2 | Left | Right | fb | D (m) | V (m/sec) | Remark |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 0.117 | 0.042 | 0.42 | 0.21 | 0.4 | 17.057 |  |  |
| 0.005 | 0.083 | 0.08 | 0.23 | 0.4 | 1.9559 |  |  |
| 0.009 | 0.066 | 0.07 | 0.24 | 0.35 | 3.0237 |  |  |
| 0.004 | 0.088 | 0.09 | 0.2 | 0.36 | 1.7329 |  |  |
| 0.008 | 0.069 | 0.07 | 0.2 | 0.35 | 2.8235 |  |  |
| Vc | Hvc | Pc | Rc | Ic | Z (m) | L(m) | sc |
| 1.50 | 0.115 | 0.77 | 0.09 | 0.0105 | 3.65 | 20 | 0.010459 |
| 1.28 | 0.083 | 0.84 | 0.10 | 0.0068 | 1.33 | 60 | 0.006848 |
| 1.28 | 0.083 | 0.84 | 0.10 | 0.0068 | 3.98 | 60 | 0.006848 |
| 1.28 | 0.083 | 0.84 | 0.10 | 0.0068 | 1.55 | 100 | 0.006848 |
| 1.28 | 0.083 | 0.84 | 0.10 | 0.0068 | 3.576 | 60 | 0.006848 |

Spill way for chute

| chainage | Qsp | Ec | d1 | b1 | A1 | V1 | Hv1 | P1 | R1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 00+10-30 | 0.108 | 3.99 | 0.071 | 0.31 | 0.022 | 4.87 | 1.21 | 0.45 | 0.05 |
| 00+30-90 | 0.108 | 1.58 | 0.096 | 0.51 | 0.049 | 2.22 | 0.25 | 0.70 | 0.07 |
| 520-580 | 0.108 | 4.23 | 0.064 | 0.51 | 0.032 | 3.32 | 0.56 | 0.64 | 0.05 |
| 580-680 | 0.108 | 1.80 | 0.109 | 0.51 | 0.055 | 1.95 | 0.19 | 0.73 | 0.08 |
| 680-740 | 0.108 | 3.83 | 0.067 | 0.51 | 0.034 | 3.19 | 0.52 | 0.64 | 0.05 |

| I1 | Im | hf1 | E1 | Fr | d2 | Lb | Bb | a |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0.26 | 0.136 | 2.71 | 3.99 | 5.84 | 0.55 | 2.2 | 0.6 | 0.1 |
| 0.03 | 0.020 | 1.21 | 1.56 | 2.28 | 0.27 | 1.1 | 0.6 | 0.0 |
| 0.11 | 0.061 | 3.63 | 4.26 | 4.20 | 0.35 | 1.4 | 1.0 | 0.1 |
| 0.02 | 0.015 | 1.49 | 1.80 | 1.89 | 0.24 | 1.0 | 1.0 | 0.0 |
| 0.10 | 0.054 | 3.22 | 3.81 | 3.94 | 0.34 | 1.4 | 1.0 | 0.0 |

Conclusion and Recommendations

Balo abebe SSIP, which covers 65ha of command land, is expected to benefit about 130beneficiaries assuming 0.5 ha per Family.

For Project sustainability, the following recommendations have to be considered.

-Water users associations should be established and an Irrigation

-Development agent should be assigned right before project implementation

-Though the field ditches & field drain collectors are designed in this booklet, the construction part is left for farmers as community   participation.

-There should be good water management as the main canalsare too long and the command area itself again is wide sloppy and long too.

- For future expansion of the project, big reservoir should be designed to solve the water shortage that will be encountered when the demand for irrigation is increased.

Annex 1. Operation and Maintenance

Objective: Timely and optimum delivery of water to farmers

-  Organizational setup of the scheme

- Water users Association

The Peasant association is responsible for overall administrative matters in the PA that is handle administration aspects including border conflicts, inheritance conflicts, etc and implements all government regulations at grass root level. The water users association is responsible to the regulation & equitable distribution of water use, mobilization of the community to maintain irrigation system, resolving conflicts between their members, collecting fee and other activities.

-Bale Zone OIDA Office sector of Irrigation Development should provide technical support during its implementation. There should be follow up and evaluation coherently with other district departments for its completion and for the successful achievement of this project objective. In addition, the office will give technical training to district departments in implementing the effective water and soil management in addition to maintenance and operation practices, use of modern inputs and agricultural practices. This office also should give due attention in organizing and strengthening water users association in all aspects

# Operation of the Diversion Guide wall

- The guide wall is gate controlled on both main canals intake and the gates are opened rotationally as per the water scheduling of the commands(left & right)

- Grasses should be grown on the banks of the river as well as the main canal to avoid serious erosion.

- Operation of the irrigation system & Drainage

- Unless practically made by the farmer themselves (based on their need of irrigation time) the flow is made discontinuous, the design throughout 80 ha of command area is assumed continuous flow over 12 hours of irrigation per day.

- At every end of field ditches, the tail water is made to flow into the natural water way beneath the command area or to enter the stream or simply drain to drainages that join the River.

- The field off take is simple open gate controlled box safely designed & they can give out water about the designed capacity for the determined hectares of command area. The water users association & the irrigation DA may help the farms in rational distribution of water among each tertiary block.

Maintenance Activity

- Structures & canals are exposed to damage if not properly maintained. Frequent maintenance of irrigation system is needed to prolong the life of irrigation scheme. Problems such as the growth of weeds & grasses, siltation may hinder the proper operation of irrigation scheme. Therefore, WUA committee must carry out timely inspection & regular maintenance by taking any necessary technical assistance from the irrigation DA.

The major maintenance work on Balo abebe SSIP

- In Irrigation project, the canals embankment & canals body might be damaged due to cattle over run. Therefore, care should be given to the construction of adequate farm bridges & footbridges as per the design to avoid such problems.

- Silt should be removed from the canals frequently.

- The threads of all gates should regularly be greased.

Location, and  at an average altitude of 1701 m a.s.l (GPS reading )

Balo abebe small-scale project is located in Bale Zone, Gaseradistrict, in one peasant association namely Balo abebe. The scheme is located at06042΄& 06055΄ N latitude and 40038΄&40026E longitude E.The project site is accessible at a distance 6km from Gaseratown to wards North West direction on all weather road . There are four major potential markets near the project area namely: gasera,Danbal  and Dalo sebro towns.

Description of works

The works to be carried out include the following:

- Guide Wall(one side collection chamber

- Flumes

- Chute

- Canals System and structures net work

- Minor Soil conservation works

Balo Abebe  Irrigation project List of drawings

| Item | Title |  | Drawing No | No of Sheets |
| --- | --- | --- | --- | --- |
| Head Work (Guide wall) | One Side open collection chamber plan and sections with Gate detail |  | 1 | 3 |
| Flume plan & sections | Flume plan & sections |  | 3 | 1 |
| Chute and Drop | Plan and section of chute and drop |  | 2 | 1 |
| Main canal profile | Main canal profile |  | - | 7 |
| Foot path &section |  |  | 1 |  |
| Off-takes |  | Off-takes & sections | 2 |  |
| Project Topo |  | Command topomap 1 |  |  |
| Division box |  |  |  |  |
| Total |  |  |  |  |

# Annex 2. SPECIFICATION

MATERIAL

General

All materials to be used in the work shall be confirmed with the requirement laid down in this chapter. If any special material not covered here is required to be used, it shall be confirmed or specified by the construction Engineer or Supervision Engineer.

1. Cement is transported directly from Adama to Gasera site storage. Care should be given while transportation. That is it should be transported under complete damp proof covers and stored in damp proof storage in which it can be stored about 1.0m high from the ground and not grater than 2.5 staking height. During allowable storing time (maximum 6 months), the cement should be kept away from moisture.

- Aggregate- should be a crushed stone from quality quarry or buying the aggregate from other places.

  - Size: 20 mm to 40 mm &Shape: cubical

  - Location: 10-15km from the site (maximum distance, as indicated in unit rate analysis and construction material location map in geology part)

- Sand:  quality sand (clean, hard, strong, durable, uncoated, free from injurious) should be transported from sand quarry.

  - Size ranges between 0.2mm to 5mm in diameter.

  - Allowable impurity = 5% (shale, clay, humus, mica, silt etc)

4. Reinforcement bar (steel)- A clean steel bar as confirmed by ASTMA-15-65, and free from rust is transported from Finfinne/Adama or other near by towns (if any) to Gasera. On the site, the bars are made bent cold as  indicated in the design. Weight per meter of steel bar is equal to square of the bar diameter in mm times 0.006165 in kg.

5. Water- the water that can be used for concrete & mortar should be free   from                  organic matters, oils, acids, bases and salts etc

-Use potable water (that used for drinking) for concrete & mortar mixing

-The turbidity of water for such purpose should not exceed 1000 ppm (of suspended solid)

6. Boulder or rubble stone – can be granite & basalt of high quality

-Size length to width ratio should not exceed 2

-Volume size 30 cm by 15 cm by 10 cm with allowable variation of 10% on either side.

-The quality of stone – free from defects like cracks, holes, flows, veins, patches etc

-Allowable water absorption percentage =5%

-Free from crystalline silicon & mica

-Minimum crushing strength should not be less than 100 kg/m2 and 400 kg / cm2 for granite & basalt respectively

- Compacted fill &back fill-The canals body and other structures should be well compacted and filled with the clay material, which probably excavated from the mother canal. The efficiency of compaction shall not be less than 130%.

# 8. Earth work-

8.1. Excavation

Canals- The center line and the top width (considering berms) of the canal is marked on the ground at interval of 10m in straight reach & 10m on curves.

The excavated soil should carefully make a bit away from the most top periphery of the canal at OGL. This is because, to avoid the excavated soil not to slid in to the canal again.

8.2 Backfill:

- a back fill soil should be free from rocks, boulders, roots etc

-In Balo Abebe irrigation, the soil used for back fill can be brought from that carted away from canal shaping and structures foundation excavation. Hence, no needs of borrow area for back fill materials.

-The depth of the back fill varies from structure to structure.

-During compacted earth fill main canal & all structures the compaction efficiency is not less than 130%

-Finally, on the embankment that resulted from compacted fill, soil should be placed on it to a thickness of about 15 to 20 cm & grass (artificial) should made to be grown on it to make the embankment very stable.

-Earth fill material should be within optimum moisture content of  ± 2 to 1 % before compaction

-Then tamping can be used as a temporary solution.

-For mechanical fills a 2% height must be left for settlement.

9. Stone pitching – this is used at  farm structures & canals where necessary.

-  Pitching should have average thickness = 20cm

It should be bedded in 7.5cm in depth on 2.5cm aggregate.

It should be made from rough dressed durable stones.

Mortar grouting should be applied between the stones as indicated on the drawing.

Then the pitched stone is to be cured starting from 24 hr after completion up to at least for consecutive 14 days.

10. Plastering – plastering work has to be done in two phases. The first coat (1.5cm thick) is with mortar ratio of 1:3 and the final finish work is with mortar ratio of 1:2 (1.5 cm thick) of plastering.

-On all structures, where the water is expected to reach (water depth plus free board) should be plastered.

11. Concrete

In Balo abebe irrigation project, the class of concrete used is C-15 chutes &C-20 for Flumes.

Either type of mixing concrete is possible, but as Balo Abebe site is concerned, a mixer machine can be used. After placing the concrete in appropriate place as per the design (drawings), it should be compacted thoroughly and uniformly.

At the surface of the concrete, sufficient mortar should be available for finishing work. The concrete should be cured for at least 21 days after 24 hrs from placing

Before placing new concrete over the old one, the previous concrete should be chiseled & the surface should be washed and construction joints should be, kept wet for at least 27 hours before placing new concrete. Strong (un deflectable) form-work should be used to support concrete moreover, the form-work should be flat, clean, tight & smooth on the side placed to the concrete face.

After concrete work is accomplished, the work area must be well cleaned

All places or structures required to be concrete are clearly indicated on all drawings.

12. Mortar -the ratio used is 1:3; the allowable moisture of sand is 20 to 27 %

-No need of using mortars that stayed a long since mixed

-Cement sand mortared masonry should be watered frequently for at least a week after 24 hr of construction

13. Masonry- stone used for masonry should be from quality quarry

-It should be well jointed

-Where needed, it should be well back filled

-All structures need to be masonry is clearly shown on all drawings.

14

.Gates – Gates are required at turn outs, off takes in different thickness of sheet metals.

The gates of turn outs & off takes have to be provided with a chain & water regulation holes at different heights. The gates should be welded & prepared as per the IDD design. The gates have to have a lock

Box Specification

1. To produce a 1:3 cement: sand mortar mix, a one bag Portland cement (0.035m3) is used with one box (0.40x0.30 m x 0.30 m) sand i.e. 1 bag: 3 box

2. To produce a 1:3:6 cement: sand: aggregate class 10 concrete mix, a one bag Portland cement (0.035m3) is used with three box (0.40 m x 0.30 m x 0.30 m) sand & with six boxes (0.40 m x 0.30 m x 0.30 m each) aggregate i.e. 1 bag: 3 box: 6 boxes

- To produce a 1:2:4 cement sand aggregate class – 15 concrete mix, a one bag Portland cement (0.035m3) is used with two box (0.40 m x 0.30 m x 0.30 m) sand and four (0.40m x 0.30 m x 0.30 m) box aggregate

- To produce a 1:2:3 cement sand aggregate class – 25 concrete mix reinforced with 2% of its volume, one bag Portland cement is used with two box (0.40 m x 0.30 m x 0.30 m) sand and three boxes (0.40 m x 0.30 m x0.30 m) aggregate etc.

# Annex 3. Unit Rate Analysis

Unit rate analysis is a base for project cost analysis.

After the quantities of each engineering work of the project are determined in quantity take off sheet, it is to be multiplied by a unit cost of quantity to estimate the price of the quantified activity.The unit costs adjusted to the current prevailing market situation & activity ratings are shown.

### 8.7  Unit Rate Calculation for the project.

| No | Approach to the computation |  |
| --- | --- | --- |
| I | Material cost |  |
| 1 | Sand (1m3) |  |
| 1.1 | Quarry distance Chalchal Malka Oda) about 100 km from the site |  |
| 1.2 | Truck Capacity = 8m3 |  |
| 1.3 | Cost of 8 m3 at quarry = 1600 Birr including loading |  |
| 1.4 | Fuel consumption of truck 1.5 km /lt |  |
| 1.5 | One trip dumping - 2*100 km /1.5 km/lt = 133 lt |  |
| 1.6 | Time needed for one tripe = 1.5 day |  |
| 1.7 | Perdium expense 2(145 +96 ) =511$ including assistance |  |
| 1.8 | Cost of 1lt fuel = 16.370 Birr = fuel cost = 133.3*16.370 =2182Birr |  |
| 1.9 | Oil , Lubricant simple maintenance = 20% fuel cost = 436.40 Birr |  |
| 1.10 | Tire and other simple maintenance = 500 Birr per tripe |  |
| 1.11 | Total cost of 8m3 sand at project site = 4718.5 Birr |  |
| 1.12 | Cost of 1m3 of sand at the site 3359.50 Birr = 589.82 Birr 8 |
| 2 | CEMEMT (1 bag) |  |
| 2.1 | Distance from Robe =65km |  |
| 2.2 | Truck capacity = 150 quintals |  |
| 2.3 | One tripe dumping fuel cost = (2*65km ) 16.37 Birr /lt =1418.75$ 1.5km/lt |  |
| 2.4 | Time needed for 1 tripe = 1 days |  |
| 2.5 | Perdium expense = 1* 145+96 = 241 Birr (including Assis ) |  |
| 2.6 | Oil , Lub & simple repair = 20 % of fuel cost = 283.75 birr |  |
| 2.7 | Tire & other spare part =275 Birr |  |
| 2.8 | cost of 150 quintal at the Robe = 150 * 350$ = 52500 birr |  |
| 2.9 | Un loading &loading cost = 600 birr |  |
|  | Total cost of 150 quintal cement at project site = 55318.50 Cost of 1 quintal of cement = 368.79 Birr Cost of 1 bag of cement = 184.50 birr |  |
| 3 | Stone (1m3) |  |
| 3.1 | Distance from quarry = 7km(radius) |  |
| 3.2 | Truck capacity = 7m3 |  |
| 3.3 | Cost of 7m3 stone at quarry = 1400 birr (in cluding loading) |  |
| 3.4 | Fuel consumption = 2 * 7km * 16.37 =214 birr 1.5km/lt |  |
| 3.5 | Time required = ½ day |  |
| 3.6 | Perdium cost = ½ (145 +96 ) =150 birr |  |
| 3.7 | Oil, Lub . & simple repair = 150 birr |  |
| 3.8 | Tire & other repair cost = 125 birr / trip = 125 birr |  |
| 3.9 | Total cost of 7m3 stone at the site = 2039 birr |  |
|  | Cost of 1m3 of stone at site = 291.30 .0 birr |  |
| 4 | Aggregated (gravel) |  |
| 4.1 | Distance =70 km (from ROBE ) to site |  |
| 4.2 | Truck capacity = 8m3 |  |
| 4.3 | Cost of 8m3 Aggregate = 3000birr |  |
| 4.4 | Fuel consumption = 2 * 70 km * 16.370 birr / lt = 1527.87 birr 1.5 km/lt |  |
| 4.5 | Oil & Lub & simple maintenance cost = 20% of fuel = 305.57 |  |
| 4.6 | Time needed = 1 Day |  |
| 4.7 | Perduim cost = 1* (145+96 ) = 241 birr |  |
| 4.8 | Tyre & simple spare part = 250 / tripe Total cost of 8m3 Aggregate at site =5360.14 birr Cost of 1m3 aggregate at project site = 670.01 birr |  |

II Material &Labor Requirement

1) For wet masonry (1:3) ratio of mortar.Per 1m3

a) Stone   ………………………………………………….1.5 m3

b) Cement …………………………………………………2.2 (bags)

c) Sand …………………………………………………….0.42m3

d) Mason Labor ………………………………………….1.5 man days

e) Daily (unskilled labor …………………………………2.8 man-days

The following co-efficient are used in quantity estimation – considering shrinkage & wastage.

Stone …………………1.73

Sand………………….. 1.63

Cement ……………….1.51

Aggregate……………..1.63

2) Lean concrete (1:2:4)……………………per 1m3

b) Sand ………………………………………0.98m3

c) Cement …………………………………...0.22m3 = 6(bags)

d) Mason……………………………………..1.1 man-day

e) Daily (unskilled) lab our ……………….4.0 man-day

3)  RCC (1:2:4)  ……………………………per 1m3

a) Aggregate ……………………………….. 0.98m3

b) Sand …………………………………….. .0.47m3

c) Cement …………………………………... 6 bags

d) Reinforcement ……………………………. 156.6 kg

e) Form work ………………………………. 4.0m3

f) Mason ……………………………………. 2 man-day

g) Daily (unskilled) labor …………….. …5 man-day

4) Hard Coring (stone pitching) (1:3 mortarper 1m3

- Stone ……………………………………..1.50m3

- Sand ……………………………………...0.02m3

- Cement …………………………………...0.15 bags

- Mason …………………………………….1 Man-day

- Unskilled labor…………………………..2 man-day

5) Plastering (1:3 mortar) 0.25cm thick     …    per 1m2

- Sand ………………………………………0.037m3

- Cement ……………………………………0.27 bags

- Mason …………………………………….0.14 man-day

- Unskilled labor………………………... 0.22 man-day

6) Site clearing ……………………………….. ..Per 1m2

- Unskilled labor………………………….0.45 man-day

7)  Excavation …………………………………..  per 1m2

a) Unskilled labor                               medium soil = 1.5manday

Soft soil = 1. man-day

Rocky hard soil = 2.5 man-day

8) Backfill & compaction …………………… … Per 1m3

- Unskilled labor …………………………1.5 man-day

- Unskilled lab……………………………….1 man-day

9) Rip-rap ……………………………………….  .per –1m2

- Stone …………………………………… ..0.46m3

- Mason …………………………………… 0.25manday

- Unskilled labor ……………………………  0.5 man-day

10) Cart away ……………………………………...per 1m3

Unskilledlabor 0.96 man-day

Salary of Mason ………………………. ………175 birr

Salary of daily lab ………………………………75 birr

Overhead cost………………………………….30% of total labor cost

Table 1-3 Unit Rate for both Material and Labour

| Total Unit Rate (Material + Labor costs) for Balo abebe SSIP |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| No | Activities | Materials | Unit | Quantity | Unit cost | Total Material cost (Birr/U) | Labour cost (Birr/U) | Total cost (Birr/U) |
| 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 |
| 1 | Site clearing | Soil | m2 | 1.00 | 0.00 | 0.00 | 18.75 | 18.75 |
|  |  | Total |  |  | 0.00 | 0.00 | 18.45 | 18.75 |
| 2 | Soil Excavation | Soil | m3 | 1.00 | 0.00 | 0.00 | 75 | 75 |
|  |  | Total |  |  | 0.00 | 0.00 | 75 | 75 |
| 3 | Soft rock Excavation | Soft rock | m3 | 1.00 | 0.00 | 0.00 | 250.25 | 250.25 |
|  |  | Total |  |  | 0.00 | 0.00 | 250.25 | 250.25 |
| 4 | Hard rock Excavation |  | m3 | 1.00 | 0.00 | 0.00 | 325.00 | 325.00 |
|  |  |  |  |  | 0.00 | 0.00 | 325.00 | 325.00 |
| 5 | Compacted fill ( Local Soil) | Soil(Clay loam & Clay) | m3 | 1.00 | 0.00 | 0.00 | 75 | 75 |
| 6 | Transporting &Compacted fill of ( Local Soil near the project) | Soil(Clay loam & Clay) | m3 | 1.00 | 156.90 | 156.90 | 57.50 | 214.40 |
| 7 | Cart away | excavated Material | m3 | 1.00 |  | ` | 62.25 | 62.25 |
| 8 | Masonry | Cement | Quint | 1.30 | 466.83 | 606.88 |  |  |
|  |  | sand | m3 | 0.30 | 666.70 | 233.30 |  |  |
|  |  | stone | m3 | 1.20 | 222.40 | 266.88 |  |  |
|  |  | Total |  |  |  | 1107.06 | 456.50 | 1563.56 |
| 9 | Concrete (1:2:4) | Cement | Quint | 2.65 | 466.83 | 1237.10 |  |  |
|  |  | sand | m3 | 0.40 | 666.70 | 266.68 |  |  |
|  |  | Aggregate | m3 | 0.86 | 389.29 | 334.79 |  |  |
|  |  | Total |  |  |  | 1838.57 | 510 | 2348.57 |
| 10 | Concrete (1:3:6) | Cement | bags | 6 | 184.6 | 1107.6 |  |  |
|  |  | sand | m3 | 0.42 | 589.82 | 247.72 |  |  |
|  |  | Aggregate | m3 | 0.90 | 670 | 603 |  |  |
|  |  | Total |  |  |  | 1958.32 | 456.50 | 1955.17 |
| 11 | Reinforced Concrete(1:2:4) | Cement | Quint | 2.65 | 466.83 | 1237.10 |  |  |
|  |  | sand | m3 | 0.40 | 159.64 | 63.86 |  |  |
|  |  | Aggregate | m3 | 0.86 | 389.29 | 334.79 |  |  |
|  |  | Steel bar | Kg | 175.00 | 19.75 | 3456.25 |  |  |
|  |  | Form work | m2 | 4.00 | 136.97 | 547.88 |  |  |
|  |  | Total |  |  |  | 5639.88 | 472.50 | 6112.38 |
| 12 | Hard coring | Cement | Quint | 0.77 | 466.83 | 359.46 |  |  |
|  |  | sand | m3 | 0.18 | 159.64 | 28.74 |  |  |
|  |  | stone | m3 | 1.42 | 222.40 | 315.81 |  |  |
|  |  | Total |  |  |  | 704.00 | 194.40 | 898.4 |
| 13 | Plastering(1:3 & 1:2 mortar in 2cm & 1cm first and last coats respectively) | Cement | Quint | 0.15 | 466.83 | 70.72 |  |  |
|  |  | sand | m3 | 0.03 | 666.70 | 20 |  |  |
|  |  | Total |  |  |  | 90.72 | 90.30 | 180 |
| 14 | Plastering(1:3 & 1:2 mortar in 2cm & 2cm first and last coats respectively) | Cement | Quint | 0.21 | 466.83 | 99.20 |  |  |
|  |  | sand | m3 | 0.04 | 159.64 | 6.39 |  |  |
|  |  | Total |  |  |  | 105.59 | 95.30 | 200.89 |
| 15 | GIS Walling / Roofing | G-32 CIS | Pcs | 0.50 | 150.00 | 75.00 |  |  |
|  |  | Nail | Kg | 0.25 | 25.00 | 6.25 |  |  |
|  |  | truss members (8-10cm) | m | 4.00 | 42.00 | 168.00 |  |  |
|  |  | Poles or purin(7cm) | Pcs | 1.50 | 25.00 | 37.50 |  |  |
|  |  | Total |  |  |  | 286.75 | 151.30 | 438.05 |
| 16 | Rip rapping | stone | m3 | 1.70 | 222.40 | 378.08 |  |  |
|  |  | Total |  |  |  | 378.08 | 101.25 | 479.3 |
| 18 | Off-takes gate | Sheet metal 3mm thick | m2 | 0.0625 | 470.00 | 29.38 |  |  |
|  |  | Transporting & loading & unloading | % | 6.00 | 470.00 | 28.20 |  |  |
|  |  | Chain for gate hang | m | 1.00 | 25.00 | 25.00 |  |  |
|  |  | Angle iron 2.5cm*2.5cm*3mm | m | 4.00 | 19.20 | 76.80 |  |  |
|  |  | 12 mm iron bar for handling with painting | kg | 0.90 | 27.90 | 25.11 |  |  |
|  |  | Total |  |  |  | 239.83 | 65.10 | 304.93 |
| 19 | Gate for division box on Main canal | Sheet metal 3mm thick | m2 | 0.455 | 470.00 | 213.62 |  |  |
|  |  | Transporting & loading & unloading | % | 6.00 | 470.00 | 28.20 |  |  |
|  |  | Chain for gate hang | m | 1.50 | 28.00 | 42.00 |  |  |
|  |  | Angle iron 4.0cm*4.0cm*3mm | m | 7.80 | 60.00 | 468.00 |  |  |
|  |  | painting in three coats with anti-rust | % | 25.00 | 470.00 | 117.50 |  |  |
|  |  | Total |  |  |  | 1303.97 | 195.75 | 1500 |

Table 1-4 Bill of Quantity

| No | Types of work | Unit | Quantity | Unit Cost |  | Total Cost(Birr) |  | Community Share |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Camping and mobilization. |  |  |  |  |  |  |  |
| 1.1 | Mobilization & Demobilization | LS | 1 | 100000.0 |  | 100000.00 |  |  |
| 1.2 | Camping |  |  |  |  |  |  |  |
|  | Living Rooms 3m*3m | Pcs | 2 |  |  |  |  |  |
|  | Office 3m*3m | " | 1 |  |  |  |  |  |
|  | Store 5m *5m | " | 1 |  |  |  |  |  |
|  | Kitchen 3m*3m | " | 1 |  |  |  |  |  |
|  | Dining room 3m*3m |  | 1 |  |  |  |  |  |
|  | Toilet 2m*2m | " | 1 |  |  |  |  |  |
|  | Guard House 2m*2.5m | " | 1 |  |  |  |  |  |
|  | Fence 50m* 50m | " | 1 |  |  | 100000 |  |  |
| 1.2.1 | Site clearance | m2 | 120.74 | 18.75 |  | 2263.88 |  | 1020.25 |
| 1.2.2 | Foundation excavation | m3 | 27.14 | 112.5 |  | 3053.25 |  |  |
| 1.2.3 | Carting away excavated materials (not less than 200m ) | m3 | 39.63 | 42.25 |  | 1674.37 |  | 1674.37 |
| 1.2.4 | Masonry in ( 1:4 cement :sand mixing ratio) | m3 | 19.1 | 1563.56 |  | 29864 |  |  |
| 1.2.5 | 20cm thick hard core | m3 | 12.5 | 898.40 |  | 11230.00 |  |  |
| 1.2.6 | 10cm thick lean concrete(1:3:6) | m3 | 6.93 | 1955.17 |  | 13549.33 |  |  |
| 1.2.7 | CIS walling G-32 | m2 | 240.45 | 438.05 |  | 105329.12 |  |  |
| 1.2.8 | GIS roofing G-32 | m2 | 123.92 | 438.05 |  | 54283.16 |  |  |
| 1.2.9 | Fencing (1.5m spacing pole to pole with 7 rows in 30cm spacing )barbed iron wire and 8cm wooden pole | m | 200 | 200.00 |  | 40000.00 |  |  |
| 2 | Access Road that must be 3cm depth cut and average width of 4m and cleared | km | 7 | 55000.00 |  | 385000 |  |  |
|  | Sub-total 1 & 2 |  |  |  |  | 746,247.11 |  | 2694.20 |
| 3 | Head work |  |  |  |  |  |  |  |
| 3.1 | Earth work |  |  |  |  |  |  |  |
| 3.1.1 | Site clearance | m2 | 100 | 18.75 |  | 1875 |  | 1345 |
| 3.1.2 | Soft rock Excavation for retaining wall bellow OGL & of foundation 0.7m below Intake level | m3 | 45 | 250.25 |  | 11261.25 |  |  |
| 3.1.4 | Carting away excavated materials place of 200m far away and safeloctions. | m3 | 50 | 42.25 |  | 2275 |  |  |
| 3.1.5 | Back fill | m3 | 35 | 57.75 |  | 2024.25 |  |  |
|  | Sub-total of 3.1 |  |  |  |  | 17435.50 |  | 1345 |
| 3.2 | Structuresguide wall |  |  |  |  |  |  |  |
| 3.2.1 | Hard Core(1:3:20 cement :sand :stone ratio) | m3 | 18.00 | 898.40 |  | 16171.2 |  |  |
| 3.2.2 | Masonry in ( 1:4 cement :sand mixing ratio) | m3 | 35.25 | 1563.56 |  | 55115.49 |  |  |
| 3.2.3 | Lean Concrete under foundation in 10cm thick as shown on drawing C-15 (1:3:6 Mixing ratio) | m3 | 3.33 | 1955.17 |  | 6510.72 |  |  |
| 3.2.4 | Concrete cupping on top of guide wall C-20 in (1:2:4 mixing ratio) | m3 | 0.75 | 2348.57 |  | 1761.43 |  |  |
| 3.2.5 | Plastering |  |  |  |  | 00000 |  |  |
| 3.2.5.4 | Plastering all the exposed surfaces 2cm thick first coat with mortar of 1:3 mixing ratio and the second coat is also with 2cm thick 1:2 mixing ratio | m2 | 30.32 | 165.80 |  | 5027.06 |  |  |
| 3.2.5.2 | Plastering work for foundation cut-off below river bed for its depth in 3cm thick with ratio of 1:3 | m2 | 22.5 | 165.8 |  | 3730.5 |  |  |
|  | Sub-total of 3.2 |  |  |  |  | 88316.40 |  |  |
| 3.3 | Gate works |  |  |  |  |  |  |  |
| 3.3.1 | Left side canal Intake gate fixing sheet metal of 4mm thick (0.50x0.550m) with all its accessories as shown on drawing | No | 1 | 10000.00 |  | 11500.00 |  |  |
| 3.3.2 | Right side canalintake gate fixing sheet metal of 4mm thick (0.60x 0.60m) with all its accessories as shown on drawing | No | 1 | 11000.00 |  | 11000.00 |  |  |
|  | Sub-total of 3.3 |  |  |  |  | 22500 |  |  |
|  | Sub-total of 3 |  |  |  |  | 128251.90 |  |  |
| 4 | Left Main Canals |  |  |  |  |  |  |  |
| 4.1 | lined Canal |  |  |  |  |  |  |  |
| 4.1.1 | Site clearance | m2 | 2864 | 18.755 |  | 53714.32 |  | 1453.40 |
| 4.1.2 | Foundation excavation (Hard rock) for 76m long in the stream bed for the closed canal | m3 | 82.10 | 325.00 |  | 26682.5 |  |  |
| 4.1.3 | Foundation excavation for earthen parts | m3 | 300.25 | 112.5 |  | 33778.13 |  |  |
| 4.1.4 | Hard Core(1:3:20 cement :sand :stone ratio) | m3 | 21.56 | 898.40 |  | 19369.50 |  |  |
| 4.1.5 | Masonry(in 1:4 cement :sand ratio) | m3 | 368.80 | 1563.56 |  | 576640.93 |  |  |
| 4.1.6 | RCC main canal slab C-20 at 45cm width interval and 10 cm thickness for 76m on two drainages where it cross the canal | m3 | 35.21 | 7538.60 |  | 265434.11 |  |  |
| 4.1.7 | Plastering 3cm thick ( 1:3 & 1:2 cement :sand ratio in 2cm & 1cm 1st and last coats respectively ) | m2 | 1560 | 165.70 |  | 258492 |  |  |
| 4.1.8 | Back fill | m3 | 156 | 75 |  | 11700 |  | 732.38 |
|  | Sub total of4.1 |  |  |  |  | 1245811.49 |  | 2185.78 |
| 4.4 | Chute Structures ( 3 in numbers) |  |  |  |  |  |  |  |
| 4.4.1 | Site clearance | m2 | 4800 | 18.75 |  | 90000 |  | 2152 |
| 4.4.2 | Foundation excavation | m3 | 165 | 112.5 |  | 18562.5 |  |  |
| 4.4.3 | Hard Core(1:3:20 cement :sand :stone ratio) | m3 | 60.27 | 898.40 |  | 54146.57 |  |  |
| 4.4.4 | Masonry using (1:4 cement sand ratio) | m3 | 60 | 1563.56 |  | 93813.6 |  |  |
| 4.4.5 | Plastering 3cm thick ( 1:3 & 1:2 cement :sand ratio in 2cm & 1cm 1st and last coats respectively ) | m2 | 210.5 | 165.70 |  | 34879.85 |  |  |
| 4.4.6 | Concrete on hard core at the base floor of chute using C-15(1:3:6 mixing ratio | m3 | 29.7 | 2348.57 |  | 69752.53 |  |  |
| 4.4.7 | Back fill | m3 | 19 | 57.75 |  | 1097.25 |  |  |
|  | Sub total of 4.4 |  |  |  |  | 362252.30 |  | 2152 |
| 4.6 | Division Boxes ( 2 in numbers) |  |  |  |  |  |  |  |
| 4.6.1 | Site clearance | m2 | 52.5 | 18.75 |  | 984.38 |  | 443.63 |
| 4.6.2 | Foundation Excavation | m3 | 21.56 | 112.5 |  | 2425.5 |  |  |
| 4.6.3 | Masonry using (1:4 cement:sandratio) | m3 | 18.24 | 1563.56 |  | 28519.33 |  |  |
| 4.6.4 | Plastering 3cm thick ( 1:3 & 1:2 cement :sand ratio in 2cm & 1cm 1st and last coats respectively ) | m2 | 73.19 | 165.70 |  | 12127.58 |  |  |
| 4.6.5 | Back fill | m3 | 5.39 | 57.75 |  | 311.27 |  | 181.91 |
| 4.6.6 | Hard Core(1:3:20 cement :sand :stone ratio | m3 | 5.2 | 898.40 |  | 4657.3 |  |  |
| 4.6.7 | Gate fixing at each DB with 30cm 80cm size using 3mm sheet metal and 3x6cm*3.0mm angle iron including all necessary accessories | No | 2 | 3000 |  | 6000 |  |  |
|  | Sub total of 4.6 |  |  |  |  | 55025.36 |  | 625.54 |
| 4.7 | Drops ( 5 in numbers with two off-take) |  |  |  |  |  |  |  |
| 4.7.1 | Site clearance | m2 | 100.25 | 18.75 |  | 1879.69 |  |  |
| 4.7.2 | Foundation excavation | m3 | 195.76 | 112.5 |  | 22023 |  |  |
| 4.7.3 | Hard Core(1:3:20 cement :sand :stone ratio | m3 | 8.9 | 898.40 |  | 7995.76 |  |  |
| 4.7.4 | Masonry using (1:4 cement :sand ratio) | m3 | 30 | 1563.56 |  | 46906.8 |  |  |
| 4.7.5 | Plastering 3cm thick ( 1:3 cement :sand ratio) | m2 | 120 | 165.56 |  | 19867.2 |  |  |
| 4.7.7 | Gate fixing at each off-takes with 25cmx25cm size using 3mm sheet metal and 2.5x2.5cm*3.0mm angle iron including all necessary accessories | no | 2 | 275.00 |  | 550.00 |  |  |
| 4.7.8 | Gates fixing C-20 concrete (1:2:4) | m3 | 0.06 | 2348.57 |  | 140.91 |  |  |
| 4.7.9 | Back fill | m3 | 39 | 75 |  | 2925 |  | 1316.25 |
|  | Sub total of 4.7 |  |  |  |  | 102288.4 |  | 3813.23 |
| 4.8 | Off-Takes ( 3 in numbers) |  |  |  |  |  |  |  |
| 4.8.1 | Site clearance | m2 | 135.4 | 18.75 |  | 2538.75 |  | 1144.13 |
| 4.8.2 | Foundation excavation | m3 | 47.25 | 112.5 |  | 5315.63 |  |  |
| 4.8.3 | Hard Core(1:3:20 cement :sand :stone ratio | m3 | 8.9 | 898.40 |  | 7995.76 |  |  |
| 4.8.4 | Masonry using (1:4 cement :sand ratio) | m3 | 24.8 | 1563.56 |  | 38776.29 |  |  |
| 4.8.5 | Plastering 3cm thick ( 1:3 & 1:2 cement :sand ratio in 2cm & 1cm 1st and last coats respectively ) | m2 | 68.22 | 165.56 |  | 11294.5 |  |  |
| 4.8.7 | Gate fixing at each off-takes with 25cmx25cm size using 3mm sheet metal and 2.5x2.5cm*3.0mm angle iron including all necessary accessories | no | 9 | 275.00 |  | 2475.00 |  |  |
| 4.8.8 | C-20 concrete (1:2:4) for Gates fixing | m3 | 0.16 | 2348.57 |  | 375.77 |  |  |
| 4.8.9 | Back fill | m3 | 21.5 | 57.75 |  | 1241.63 |  | 725.63 |
|  | Sub total of 4.8 |  |  |  |  | 70013.33 |  | 1869.76 |
| 4.9 | Farm bridge/culvert #1 (3) |  |  |  |  |  |  |  |
| 4.9.1 | Site clearance | m2 | 125 | 18.75 |  | 2343.75 |  | 1056.25 |
| 4.9.2 | Foundation Excavation | m3 | 49.45 | 112.5 |  | 5563.13 |  |  |
| 4.9.3 | Masonry(in 1:4 cement :sand ratio) | m3 | 40.05 | 1563.56 |  | 62620.58 |  |  |
| 4.9.4 | Plastering 3cm thick ( 1:3 & 1:2 cement :sand ratio in 2cm & 1cm 1st and last coats respectively ) | m2 | 71.4 | 165.56 |  | 11820.98 |  |  |
| 4.9.5 | RCC C-2- (1:2:4) including bars and formwork | m3 | 1.15 | 7538.60 |  | 8669.39 |  |  |
| 4.9.6 | Back fill | m3 | 9.5 | 57.75 |  | 548.63 |  | 320.63 |
| 4.9.7 | Stone Riprap | m3 | 8.06 | 479.30 |  | 3863.16 |  |  |
|  | Sub total of 4.9 |  |  |  |  | 95429.62 |  | 1376.88 |
|  | Sub total of 4 |  |  |  |  | 1930820.50 |  | 165156.48 |
| 5 | Flume on main canal -1(left canal) |  |  |  |  |  |  |  |
| 5.1 | Site clearance | m2 | 320 | 18.75 |  | 6000 |  | 2704.00 |
| 5.1.1 | Foundation Excavation soft rock | m3 | 75 | 250.25 |  | 18768.75 |  |  |
| 5.1.2 | RCC C-2- (1:2:4) including bars and formwork | m3 | 20.5 | 7538.60 |  | 154,541.30 |  |  |
| 5.1.3 | masonry Masonry(in 1:4 cement :sand ratio) | M3 | 7.5 | 1563.56 |  | 11726.7 |  |  |
| 5.1.4 | plastering | M2 | 33.45 | 165.56 |  | 5537.98 |  |  |
| 5.1.5 | Plastering 3cm thick ( 1:3 & 1:2 cement :sand ratio in 2cm & 1cm 1st and last coats respectively ) | m3 | 3.5 | 75 |  | 262.5 |  |  |
|  | Sub total of 5.1 |  |  |  |  | 196,837.23 |  | 10683.91 |
|  | Sub total of 5&4 |  |  |  |  | 2127657.69 |  | 19809.53 |
| 6 | Right side main canal |  |  |  |  |  |  |  |
| 6.1 | Lined canal |  |  |  |  |  |  |  |
| 6.1.1 | Earth work |  |  |  |  |  |  |  |
| 6.1.1.1 | Site clearance | m2 | 262.5 | 18.75 |  | 4921.88 |  | 2218.13 |
| 6.1.1.2 | Foundation excavation | m3 | 1215.70 | 112.5 |  | 136766.25 |  | 2138.95 |
| 6.1.1.3 | Foundation excavation soft rock | m3 | 210.63 | 250.25 |  | 52710.16 |  |  |
| 6.1.2.4 | Hard Core(1:3:20 cement :sand :stone ratio | m3 | 100 | 898.40 |  | 89804.00 |  |  |
| 6.1.1.5 | Masonry(1:4 cement sand ratio) | M3 | 780 | 1563.56 |  | 123784.54 |  |  |
| 6.1.6 | plastering | M3 | 2567.38 | 165.56 |  | 425055.43 |  |  |
| 6.1.7 | RCC main canal slab C-20 at 45cm width interval and 10 cm thickness for 76m on two drainages to where it cross the canal | m3 | 35.21 | 7538.60 |  | 265434.11 |  |  |
|  | Sub total of 6.1 |  |  |  |  | 1098476.4 |  | 4357.07 |
| 6.2. | Chute Structures ( 6 in numbers ) |  |  |  |  |  |  |  |
| 6.2.1.1 | Site clearance | m2 | 340 | 18.75 |  | 2873.00 |  | 2873.00 |
| 6.2.1.2 | Foundation excavation | m3 | 170.94 | 112.5 |  | 19230.75 |  |  |
| 6.2.1.3 | Hard Rock excavation | M3 | 175 | 325 |  | 56875 |  |  |
| 6.2.3.4 | Hard Core(1:3:20 cement: sand :stone ratio) | m3 | 29.87 | 898.40 |  | 26835.21 |  |  |
| 6.2.1.5 | Masonry using (1:4 cement sand ratio) | m3 | 178.8 | 1563.56 |  | 279564.53 |  |  |
| 6.2.1.6 | Plastering 3cm thick ( 1:3 & 1:2 cement :sand ratio in 2cm & 1cm 1st and last coats respectively ) | m2 | 615 | 165.70 |  | 101905.5 |  |  |
| 6.2.1.7 | Concrete on hard core at the base floor of chute using C-15 (1:3:6 mixing ratio) | m3 | 30 | 2348.57 |  | 70457.10 |  |  |
| 6.2.1.8 | Back fill | m3 | 15.66 | 75 |  | 1174.5 |  | 528.53 |
|  | Sub total of 6.2.1 |  |  |  |  | 558915.59 |  | 3401.53 |
| 6.2.2 | Off-takes ( 4 in number ) |  |  |  |  |  |  |  |
| 6.2.2.1 | Site clearance | m2 | 22.45 | 18.45 |  | 414.2 |  | 189.70 |
| 6.2.2.2 | Foundation excavation | m3 | 3.15 | 75 |  | 236.25 |  |  |
| 6.2.2.3 | Masonry using (1:4 cement :sand ratio) | m3 | 4.75 | 1596.20 |  | 7581.95 |  |  |
| 6.2.2.4 | Plastering 3cm thick ( 1:3 & 1:2 cement :sand ratio in 2cm & 1cm 1st and last coats respectively ) | m2 | 16.15 | 165.70 |  | 2676.06 |  |  |
| 6.2.2.5 | Gate fixing at each off-takes with 25cmx25cm size using 3mm sheet metal and 2.5x2.5cm*3.0mm angle iron including all necessary accessories | no | 4 | 275.00 |  | 1100.00 |  |  |
| 6.2.2.7 | Gates fixing with C-20 concrete (1:2:4) including of off take bed accessory | m3 | 1.3 | 2348.57 |  | 3053.14 |  |  |
| 6.2.2.8 | Back fill | m3 | 3.8 | 75 |  | 285 |  | 128.25 |
|  | Sub total of 6.2.2 |  |  |  |  | 15478.12 |  | 317.95 |
|  | Sub total of 6.2 |  |  |  |  | 574393.7 |  | 3719.48 |
| 7.1.1 | Off-takes ( 5 in numbers ) |  |  |  |  |  |  |  |
| 7.1.1.1 | Site clearance | m2 | 80 | 18.75 |  | 1500 |  | 676.00 |
| 7.1.1.2 | Foundation excavation | m3 | 16.14 | 75 |  | 1210.5 |  |  |
| 7.1.1.3 | Masonry using (1:4 cement :sand ratio) | m3 | 12.22 | 1563.56 |  | 19106.7 |  |  |
| 7.1.1.4 | Plastering 3cm thick ( 1:3 & 1:2 cement :sand ratio in 2cm & 1cm 1st and last coats respectively ) | m2 | 33.07 | 165.56 |  | 3244.17 |  |  |
| 7.1.1.5 | Gate fixing at each off-takes with 25cmx25cm size using 3mm sheet metal and 2.5x2.5cm*3.0mm angle iron including all necessary accessories | no | 5 | 275.00 |  | 1375.00 |  |  |
| 7.1.1.7 | Gates fixing with C-20 concrete (1:2:4) including of off take bed accessory | m3 | 1.3 | 2348.57 |  | 3053.14 |  |  |
| 7.1.1.8 | Back fill | m3 | 4.1 | 75 |  | 307.5 |  | 138.38 |
|  | Sub total of 7.1.1 |  |  |  |  | 29797.01 |  | 814.38 |
| 7.2.2 | Drop( 9 in number ) |  |  |  |  |  |  |  |
| 7.2.2.1 | Site clearance | m2 | 101.25 | 18.75 |  | 1898.44 |  | 95.06 |
| 7.2.2.2 | Foundation excavation | m3 | 50.96 | 122.5 |  | 6242.6 |  |  |
| 7.2.2.3 | Foundation excavation soft rock | M3 | 35 | 250.25 |  | 8758.75 |  |  |
| 7.2.2.4 | Masonry using (1:4 cement :sand ratio) | m3 | 27.9 | 1563.56 |  | 43623.32 |  |  |
| 7.2.2.5 | Plastering 3cm thick ( 1:3 & 1:2 cement :sand ratio in 2cm & 1cm 1st and last coats respectively ) | m2 | 150.07 | 165.70 |  | 24866.6 |  |  |
| 7.2.2.5 | Back fill | m3 | 18.85 | 75 |  | 1413.75 |  | 62.44 |
|  | Sub total of 7.2.2 |  |  |  |  | 86803.46 |  | 157.50 |
| 7.3.3 | Division Boxe ( 2 in number ) |  |  |  |  |  |  |  |
| 7.3.3.1 | Site clearance | m2 | 25.3 | 18.45 |  | 466.79 |  | 213.79 |
| 7.3.3.2 | Foundation Excavation | m3 | 10.5 | 112.5 |  | 1181.3 |  |  |
| 7.3.3.3 | Masonry using (1:4 cement :sand ratio) | m3 | 6.56 | 1563.56 |  | 10256.95 |  |  |
| 7.3.3.4 | Plastering 3cm thick ( 1:3 & 1:2 cement :sand ratio in 2cm & 1cm 1st and last coats respectively ) | m2 | 27.7 | 165.70 |  | 4589.89 |  |  |
| 7.3.3.5 | Back fill | m3 | 3.6 | 75 |  | 270 |  | 121.50 |
| 7.3.3.6 | Gate fixing for Division box 6 only toward outlet of TC2-with 50.5cmx90cm size using 3mm sheet metal and 3.5x3.5 cm angle iron including all necessary accessories | no | 1 | 3000 |  | 3000 |  |  |
| 7.3.3.7 | C-20 concrete (1:2:4 ) for Gate fixing | m3 | 0.024 | 2348.57 |  | 56.366 |  |  |
|  | Sub total of 7.3.3 |  |  |  |  | 18321.30 |  | 335.29 |
| 8.1 | Farm bridge/culvert #1 |  |  |  |  |  |  |  |
| 8.1.1 | Site clearance | m2 | 125 | 18.75 |  | 2343.75 |  | 1056.25 |
| 8.8.2 | Foundation Excavation | m3 | 49.45 | 112.5 |  | 5563.13 |  |  |
| 8.9.3 | Hard Core(1:3:20 cement :sand :stone ratio | m3 | 13 | 898.40 |  | 11679.20 |  |  |
| 8.9.4 | Masonry(in 1:4 cement :sand ratio) | m3 | 40.05 | 1563.56 |  | 62620.58 |  |  |
| 8.9.5 | Plastering 3cm thick ( 1:3 & 1:2 cement :sand ratio in 2cm & 1cm 1st and last coats respectively ) | m2 | 71.4 | 165.56 |  | 11820.98 |  |  |
| 8.9.6 | RCC C-2- (1:2:4) including bars and formwork | m3 | 1.15 | 7538.60 |  | 8669.39 |  |  |
| 8.9.7 | Back fill | m3 | 9.5 | 57.75 |  | 548.63 |  | 320.63 |
| 8.9.8 | Stone Riprap | m3 | 8.06 | 479.30 |  | 3863.16 |  |  |
|  | Sub total of 4.9 |  |  |  |  | 95429.62 |  | 1376.88 |
|  | Sub-total of 8.2.2 |  |  |  |  | 95429.62 |  | 225.44 |
|  | Total |  |  |  |  |  |  |  |
|  | Supervision cost 5.0% |  |  |  |  |  |  |  |
|  | Grand total |  |  |  |  |  |  |  |
| No | Work Item | Cost (ETB) |  |  |  |  |
| 1 | Access road, Camping Mobilization | 746,247.11 |  |  |  |  |
| 2 | Head Work | 128251.90 |  |  |  |  |
| 3 | Canals and other irrigation infrastructures | 4012761.35 |  |  |  |  |
| 4 | total | 4887260.36 |  |  |  |  |
| 5 | supervision cost=5%project cost | 244363.00 |  |  |  |  |
| 6 | VAT 15% | 769743.50 |  |  |  |  |
| 7 | Grand total project cost | 5901366.86 5,901,366.86 |  |  |  |  |
| 8 | Cost per hectare | 90,790.30 |  |  |  |  |

## 8.10. Operation and maintenance

Operation and maintenance of the scheme requires strong and effective organizations of the beneficiary community /farmers/. That is the sustainability of irrigation system is determined by the strong participation   of the farmers involved in irrigation practices in the command area. Structures and all distribution systems may be expected to damage if not properly maintained .therefore, frequent maintenance of irrigation system is needed to prolong the life of irrigation scheme. Therefore WUA committee must carry out timely inspected and regular maintenance by taking any necessary technical assistance from the irrigation DA.

### 8.10.1 OPERATION

Good operation of any irrigation systems includes matching the irrigation duration with the rate of application and the intake rate of the soil to maximize the fraction of water stored in the root. Therefore evaluation of the irrigation system is recommended if it is suspected that the system is not as efficient as it should be. Checking the water distribution determines the depth of water being applied, and distribution uniformity. If water is divided by structures, measuring the flow rate is essential, Farmers must Contact development agent in order to ensure fair distribution of water, to avoid social conflict among the farmers and protect the looting of water.

### 8.10.2 MAINTENANCE

a. Spring

Since spring area is not managed properly, the spring is charged with weeds and debris, therefore the upstream and downstream of the spring near the intake should be free from weed and it should frequently cleaned after each seasons.

.

b. Canal

In order insure the proper supply of irrigation water, the canal should be clear from unwanted materials that retard the water flow rate, i.e, the Grasses, weeds grown in the canal, and the silt should be removed in regulated manner.  During clearing, the cross section of the canal has to be maintained as per the design.

In irrigation project the canals embankment and canal body might be damaged due to cattle over run. Therefore care should give to the utilization of the foot bridge.

C.  Drainage

During rainy season the main canal may be affected from lateral flood and the canal may be damaged and broken. The farmers should take care of diverting the lateral water to the canal and from closing drainage structures constructed as per design.

## 8.11 . Specification

### 8.11.1 Materials

8.11.1.1 .Cement

The cement is Portland cement and Care should be given while transportation. I.e. transported under complete damp proof covers and stored in damp proof storage in which it can be stored about 1.0m high from the ground and not greater than 2.5 staking height. In the storage it should not stay more than six months.

8.11.1. 2 Aggregate

It should be a crushed stone from quality quarry  & its Size: 20 mm to 40 mm

Shape of  aggregate is  cubical

8.11.1.3  Sand

A quality sand (clean, hard, strong, durable, uncoated)

a.  Size = 1- 2mm (about)

b.  Allowable impurity = 5% (shale, clay, humus, mica silt etc)

8.11.1. 4 Reinforcement ( steel )bar

A clean steel bar as confirmed by ASTMA-15-65, free from rust And the bars are made bent cold as indicated in the design.

8.11.1.5 Water

The water that can be used for concrete & mortar should be free from organic matters,

oils, acids, bases and salts etc.

- Use potable water (that used for drinking) for concrete & mortar mixing

-The turbidity of water for such purpose should not exceed 1000 ppm (of suspended solid)

8.11.1.6 Rubble stone

-  It can be granite & basalt of high quality

-  Size: Length to width ratio should not exceed two.

- Quality of stone – free from defects like cracks, holes, flows, veins, patches etc

- Allowable water absorption percentage =5%

- Free from crystalline silicon & mica

- Minimum crushing strength should not be less than 100 kg/m2 and 400 kg / cm2 for

granite & basalt respectively

8.11.1.7  Site clearance

Before any earth work of canals and structures  etc  proceed, the routes of such net-works

should be free from trees, bushes, Insitu stone & rock. The forest that secondary of three & for few distance secondary one pass should not be damaged. For this purpose the path should be as narrower as possible

8.11.1.8 Earth works

a . Excavations

The center line for excavation of canals routes marking on the ground shall be at interval

of 10 to 25 m & for curved  part, it shall not be more than 5m .To avoid slipping or washing down to canal after excavation, the excavated soil should be thrown on right  and not  at the edge of the canal ,but a little bit far away to reasonable distance.

b. Backfill

A back fill soil should be free from rocks, boulders, roots etc

Depending on the areas to be compacted, the soil for compaction can be used from the

Excavated canal soil. For compaction of soil, the thickness of compaction should not be more than 15cm at the optimum moisture. For mechanical fills a 2% height must be left for settlement.

7.11.1.9. Plastering

Plastering work has to be done in two phases. The first coat (1.5 cm) thick is in 1:3

mortar of cement sand ratio and the second  & finished work is with mortar coat of 1 to

1.5 cm thick of mortar.

All structures parts where water expected to reach including freeboard should be plastered.

8.11.1. 10 Concrete

The concrete class to be used in this project is C-15 & C 20.For the two types of the concrete, it is possible to use either hand mixing or mechanical by mixing machine.	 The concrete should be cured for at least 21 days after 24 hrs from placing. Before placing new concrete over the old one, the previous concrete should be chiseled & the surface should be washed and construction joints should be, kept wet for at least one day before placing new c0ncrete.

8.11.1.1. Mortar

The ratio of plastering to be used is 1: 3 cement sand mortar

It is not recommendable to use mortar stayed long time (hours) since mixed.

The plastered part of the structure with mortar should be watered per day twice for at least

a week after 24hr of construction.

8.11.1.12. Masonry

Stone used for masonry should be from quality quarry

It should be well jointed, where needed, it should be well back filled

### 8.11.2 Box Specification

1. To produce a 1:3 cement: sand mortar mix, a one bag Portland cement (0.035m3) is used with one box (0.40x0.30 m x 0.30 m) sand i.e. 1 bag: 3 box

2. To produce a 1:3:6 cement: sand: aggregate class 10 concrete mix, a one bag Portland cement (0.035m3) is used with three box (0.40 m x 0.30 m x 0.30 m) sand & with six boxes (0.40 m x 0.30 m x 0.30 m each) aggregate i.e. 1 bag: 3 box: 6 boxes

- To produce a 1:2:4 cement sand aggregate class – 15 concrete mix, a one bag Portland cement (0.035m3) is used with two box (0.40 m x 0.30 m x 0.30 m) sand and four (0.40m x 0.30 m x 0.30 m) box aggregate

To produce a 1:2:3 cement sand aggregate class – 25 concrete mix reinforced with 2% of its volume, one bag Portland cement is used with two box (0.40 m x 0.30 m x 0.30 m) sand and three boxes (0.40 m x 0.30 m x0.30 m) aggregate •

|  | Table-1 Farm gate price of out puts/qt or kg |  |  |
| --- | --- | --- | --- |
|  | Crops | Unit | Total price in birr |
| 1 | Maize | Qt | 700 |
| 2 | Cabbage(habesha) | Qt | 90 |
| 3 | Carrot | Qt | 400 |
| 4 | Sweet potato | Qt | 450 |
| 5 | Haricot bean | Qt | 900 |
| 6 | onion | kg | 1700 |
| 7 | Pepper | kg | 800 |
| 8 | Tomato | kg | 2300 |
| 9 | Sugare Cane | kg | 800 |
| 10 | Mango | kg | 850 |
| 11 | Avocado | kg | 1200 |
| 12 | Banana | kg | 1200 |
| 13 | Papaye | kg | 400 |
| 14 | Teff | Qt | 1200 |
| 15 | Wheat | Qt | 850 |
| 16 | Barely | Qt | 850 |

| Input type and unit measurement |  |
| --- | --- |
| Inputs type | Unit |
| Maize | Qt |
| Onion | Packed seed/kg |
| Cabbage | Packed seed/kg |
| Carrot | Packedseed/kg |
| Sweet potato | cutting |
| Haricot bean | Qt |
| Pepper | Qt |
| Tomato | Packedseed/kg |
| Sugare Cane | seedling |
| Mango | seedling |
| Avocado | seedling |
| Banana | seedling |
| Papaye | seedling |
| Teff | kg |
| Wheat | Qt |
| Barely | Qt |
| FERTILIZER |  |
| DAP | Qt |
| Urea | Qt |
| Others |  |
| Chemicals |  |
| Hirbiside | lit |
| Pestiside | lit |
| labour | man/day |
| Donkey power | D/D |
| Oxen power (pair with Labour) | oxen/day |
| Packing materials | Pcs |
| Land tax <0.5ha | Hectare |
| Farm and Hand tools |  |
| Knapsack sprayer with accessories | Birr |
| Plough & Accessories | Birr |
| Hoe | ,, |
| Shovels | ,, |
| Spade | " |
| Bangaa | " |
| Sickle | " |

| Table-3 Without project inputs and its cost/ha |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Type of existing crops | Price | Seed rate/ha(kg) | Cost | Required man day |  | Hired labour | unit price | Total price | Urea | price | Total price | Oxen day pair | Unit Price | Cost | Total Cost/ha | Area in Ha | Total Cost |
| Belg (Arfasa) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Maize | 700 | 45 | 315 |  | 71 | 42 | 60 | 2520 | 100 | 16 | 1580 | 40 | 80 | 3200 | 7,615 | 20 | 152,300 |
| Teff | 1200 | 20 | 240 |  | 82 | 48 | 60 | 2880 | 100 | 16 | 1580 | 40 | 80 | 3200 | 7,900 | 5 | 39,500 |
| Haricot bean | 900 | 120 | 1080 |  | 71 | 42 | 60 | 2520 | 100 | 16 | 1580 | 40 | 80 | 3200 | 8,380 | 5 | 41,900 |
| Sorghum | 750 | 65 | 487.5 |  | 71 | 42 | 60 | 2520 | 100 | 16 | 1580 | 40 | 80 | 3200 | 7,788 | 35 | 272,563 |
| Meher (Hegaya) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Sorghum | 750 | 65 | 487.5 |  | 71 | 42 | 60 | 2520 | 100 | 16 | 1580 | 40 | 80 | 3200 | 7,788 | 19 | 147,963 |
| Teff | 1200 | 100 | 1200 |  | 71 | 42 | 60 | 2520 | 100 | 16 | 1580 | 40 | 80 | 3200 | 8,500 | 76 | 646,000 |
| Haricot bean | 770 | 120 | 900 |  | 71 | 42 | 60 | 2520 | 100 | 16 | 1580 | 40 | 80 | 3200 | 8,200 | 12 | 98,400 |
| Total |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1,398,625 |

| Table-3 Without project inputs and its cost/ha |  |  |  |  | Traditional irrigation |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Type of existing crops | Price | Seed rate/ha(kg) | Cost | Required man day | Hired labour | unit price | cost | Unit price |  | cost | Oxen day pair | Unit Price | Cost | Total Cost/ha | Area in Ha | Total Cost |
| Belg (Arfasa) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Maize | 700 | 45 | 315 | 170 | 34 | 60 | 2040 | 16 |  | 1580 | 40 | 80 | 3200 | 7,135 | 20 | 142,700 |
| Tomato | 3000 | 0.75 | 22.5 | 209 | 42 | 60 | 2508 | 16 |  | 1580 | 32 | 80 | 2560 | 6,671 | 1 | 6,671 |
| Avocado | 2 | 360 | 720 | 177 | 35 | 60 | 2124 | 16 |  | 1580 | 32 | 80 | 2560 | 6,984 | 5 | 34,920 |
| Sugarcane | 0.01 | 15150 | 151.5 | 179 | 36 | 60 | 2148 | 16 |  | 1580 | 32 | 80 | 2560 | 6,440 | 10 | 64,395 |
| Orange | 2 |  |  | 138 | 28 | 60 | 1656 | 16 |  | 1580 | 32 | 80 | 2560 | 5,796 | 1 | 5,796 |
| Banana | 2 | 1886 | 3772 | 154 | 31 | 60 | 1848 | 16 |  | 0 | 32 | 80 | 2560 | 8,180 | 10 | 81,800 |
| Mango | 2 | 480 | 960 | 138 | 28 | 60 | 1656 | 16 |  | 0 | 32 | 80 | 2560 | 5,176 | 5 | 25,880 |
| Papapa | 2 | 3200 | 6400 | 190 | 38 | 60 | 2280 | 16 |  | 0 | 32 | 80 | 2560 | 11,240 | 3 | 33,720 |
| Coffee | 3 | 3500 | 10500 | 200 | 40 | 60 | 2400 | 16 |  | 0 | 32 | 80 | 2560 | 15,460 | 25 | 386,500 |
| Chat | 0.05 | 4445 | 222.25 |  | 40 | 60 | 2400 | 16 |  | 0 | 32 | 80 | 2560 | 5,182 | 10 | 51,823 |
| Meher (Hegaya) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Maize | 700 | 45 | 315 | 170 | 34 | 60 | 2040 | 16 |  | 1580 | 32 | 80 | 2560 | 4,455 | 20 | 89,100 |
| Tomato | 3000 | 0.75 | 2250 | 209 | 42 | 60 | 2508 | 16 |  | 1580 | 32 | 80 | 2560 | 6,390 | 1 | 6,390 |
| Total |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 929,694 |

| Table-4 Without Project total Value from Crops Production |  |  |  |  |
| --- | --- | --- | --- | --- |
| Type of existing crops | Average yields per ha (Qt) | Area in hect. | Price | Total Value |
| Belg (Arfasa) |  |  |  |  |
| Maize | 25 | 8 | 700 | 140,000.00 |
| Teff | 12 | 5 | 1200 | 72,000.00 |
| Haricot bean | 120 | 3 | 900 | 324,000.00 |
| Sorghum | 15 | 12 | 750 | 135,000.00 |
| Meher (Hegaya) |  |  |  |  |
| Sorghum | 12 | 12 | 750 | 108,000.00 |
| Teff | 12 | 5 | 1200 | 72,000.00 |
| Haricot bean | 12 | 3 | 900 | 32,400.00 |
| total |  |  |  | 883,400.00 |

| Table-5 With the project Inputs and its cost/ha |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Proposed crops | unit price | Seed rate |  | Fertlizer |  |  |  |  |  |  | Labour | Hired Labour | Price | Cost | Oxen day |  | cost |  |  |  |  |  |  | Total Cost/ha |
|  |  |  | cost | DAP | unit price | total price | Urea | Unit Price | total price | Total Cost |  |  |  |  |  | price |  | chemicals | price | cost | packing | price | cost |  |
| Belg(Arfasa) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Maize | 1450 | 45 | 653 | 100 | 16 | 1580 | 100 | 12.3 | 1230 | 2810 | 115 | 69 | 50 | 3450 | 40 | 80 | 3200 | 3 | 80 | 240 | 65 | 12 | 780 | 11,133 |
| Onion | 1028 | 4.5 | 46.3 | 100 | 16 | 1580 | 100 | 12.3 | 1230 | 2810 | 103 | 62 | 50 | 3090 | 37 | 80 | 2960 | 1.5 | 80 | 120 | 230 | 12 | 2760 | 11,786 |
| Haricot bean | 850 | 50 | 425 | 100 | 16 | 1580 | _ | 12.3 | 0 | 1580 | 142 | 85 | 50 | 4260 | 32 | 80 | 2560 | 2 | 80 | 160 | 20 | 12 | 240 | 9,225 |
| Pepper | 80 | 0.75 | 60 | 100 | 16 | 1580 | 100 | 12.3 | 1230 | 123000 | 196 | 118 | 50 | 5880 | 32 | 80 | 2560 | 1.5 | 80 | 120 | 12 | 12 | 144 | 131,764 |
| Banana | 3 | 1600 | 4800 | 0 | 0 |  | 0 | 0 | 0 | 0 | 172 | 103 | 50 | 5160 | 40 | 80 | 3200 | 0 | 0 | 0 | 0 | 0 | 0 | 13,160 |
| Papaya | 3 | 2000 | 2000 | 0 | 0 |  | 0 | 0 | 0 | 0 | 172 | 103 | 50 | 5150 | 40 | 80 | 3200 | 0 | 80 | 0 | 105 | 12 | 1260 | FALSE |
| Mango | 3 | 157 | 471 | 0 | 0 |  | 0 | 0 | 0 | 0 | 172 | 103 | 50 | 5150 | 40 | 80 | 3200 | 0 | 80 | 0 | 152 | 12 | 1824 | 10,645 |
| Avocado | 3 | 300 | 600 | 0 | 0 |  | 0 | 12.3 | 0 | 0 | 168 | 101 | 50 | 5040 | 37 | 80 | 2960 | 0 | 0 | 0 | 152 | 12 | 1824 | 10,424 |
| Maher (Hagaya ) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Onion | 1028 | 4.5 | 4626 | 100 | 16 | 1580 | 150 | 12.3 | 1845 | 3425 | 115 | 69 | 50 | 3450 | 40 | 80 | 3200 | 1.5 | 80 | 120 | 230 | 12 | 2760 | 17,691 |
| Haricot bean | 1500 | 1 | 750 | 100 | 16 | 1580 | _ | 12.3 | 0 | 1580 | 196 | 118 | 50 | 5880 | 32 | 80 | 2960 | 2 | 80 | 160 | 20 | 12 | 240 | 1,303 |
| Pepper | 80 | 0.75 | 60 | 200 | 16 | 3160 | 150 | 12.3 | 1845 | 5005 | 196 | 118 | 50 | 5880 | 32 | 80 | 2560 | 1.5 | 80 | 120 | 12 | 12 | 144 | 13,889 |
| Tomato | 3000 | 0.5 | 1500 | 200 | 16 | 3160 | 100 | 12.3 | 1230 | 4390 | 167 | 100 | 50 | 5010 | 37 | 80 | 2960 | 1.5 | 80 | 120 | 350 | 12 | 4200 | 18,180 |

|  |  |  | Table-6-production cost with project situation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| No |  | Cost of Prod./ha | Year 1 |  |  |  |  | Year 2 |  |  |  |  |
|  | Crop type |  | Ph.1 | Total | Ph.2 | Total | Total | Ph.1 | Total | Ph2 | Total | Total |
|  |  |  | Ha | cost | Ha | Cost | cost | Ha | Cost | Ha | Cost | Cost |
| 1 | Maize | 11,133 | 12 | 133,596 | 15 | 166,995 | 300,591 | 15 | 166,995 | 17 | 189,261 | 356,256 |
| 2 | Banana | 13,160 |  |  | 10 | 131,600 | 131,600 |  | - | 13 | 171,080 | 171,080 |
| 3 | Haricot bean | 92,225 | 6 | 553,350 | 2 | 184,450 | 737,800 | 8 | 737,800 | 11 | 1,014,475 | 1,752,275 |
| 4 | Mango | 10,645 |  |  | 5 | 53,225 | 53,225 |  |  | 5 | 53,225 | 53,225 |
| 5 | Onion | 11,786 |  |  | 5 | 58,930 | 58,930 | 10 | 117,860 | 13 | 153,218 | 271,078 |
| 6 | Pepper | 131,764 | 1 | 131,764 | 5 | 658,820 | 790,584 | 1 | 131,764 | 10 | 1,317,640 | 1,449,404 |
| 7 | Tomato | 18,180 | 4 | 72,720 |  |  | 72,720 | 8 | 145,440 | 7 | 127,260 | 272,700 |
| 8 | Avocado | 10,424 |  |  | 6 | 62,544 | 62,544 |  |  | 7 | 72,968 | 72,968 |
| 9 | Papaya | 11,610 |  |  | 3 | 34,830 | 34,830 |  |  | 3 | 34,830 | 34,830 |
|  | total |  |  |  |  |  | 2,242,824 |  |  |  |  | 4,433,816 |

| Year 3 |  |  |  |  | Year 4 |  |  |  |  | Year 5 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Ph. 1 | Total | Ph.2 | Total | Total | Ph. 1 | Total | Ph.2 | Total | Total | Ph. 1 | Total | Ph.2 | Total | Total |
| Ha | Cost | Ha | Cost | cost |  | Cost |  | Cost | cost | Ha | Cost | Ha | Cost | cost |
| 20 | 222,660 | 25 | 278,325 | 500,985 | 25 | 278,325 | 25 | 278,325 | 556,650 | 25 | 278,325 | 25 | 278,325 | 556,650 |
|  |  | 10 | 131,600 | 131,600 |  | - | 10 | 131,600 | 131,600 |  |  | 10 | 131,600 | 131,600 |
| 11 | 1,014,475 | 5 | 461,125 | 1,475,600 | 12 | 119,520 |  | 6 | 119,526 | 12 | 1,106,700 | 12 | 1,106,700 | 2,213,400 |
|  |  | 5 | 53,225 | 53,225 | 0 | - | 8 | 85,160 | 85,160 | 1 | 10,645 | 7 | 74,515 | 85,160 |
| 12 | 141,432 | 15 | 176,790 | 318,222 | 14 | 165,004 | 10 | 117,860 | 282,864 | 14 | 165,004 |  | - | 165,004 |
| 4 | 527,056 | 15 | 1,976,460 | 2,503,516 | 4 | 527,056 | 14 | 1,844,696 | 2,371,752 | 4 | 527,056 | 15 | 1,976,460 | 2,503,516 |
| 7 | 127,260 | 4 | 72,720 | 199,980 | 7.5 | 136,350 |  | - | 136,350 | 7.5 | 136,350 | 0 | - | 136,350 |
|  |  | 6 | 62,544 | 62,544 | 0 | - | 6 | 62,544 | 62,544 |  |  | 7 | 72,968 | 72,968 |
|  |  | 3 | 34,830 | 34,830 |  |  | 3 | 34,830 | 34,830 |  |  | 6 | 69,660 | 69,660 |
|  |  |  |  | 5,280,502 |  |  |  |  | 3,781,276 |  |  |  |  | 5,934,308 |

|  | Table-7 With the project Land use pattern |  |  |  | And Cropping Intesity |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| No | Crop | Year 1 |  |  | Year 2 |  |  | Year 3 |  |  | Year 4 |  |  | Year 5 |  |  |
|  |  | Ph.1 | Ph.2 | Total | Ph.1 | Ph2 | Total | Ph. 1 | Ph.2 | Total | Ph. 1 | Ph.2 | Total | Ph. 1 | Ph.2 | Total |
|  |  | Ha | Ha | Ha | Ha | Ha | Ha | Ha | Ha | Ha | Ha | Ha | Ha | Ha | Ha | Ha |
| 1 | Maize | 12 | 15 | 27 | 15 | 17 | 32 | 20 | 25 | 45 | 25 | 25 | 50 | 25 | 25 | 50 |
| 2 | Haricoat bean | 6 | 2 | 8 | 8 | 4 | 12 | 11 | 5 | 16 | 12 | 6 | 18 | 12 | 6 | 18 |
| 3 | Onion | 7 | 5 | 12 | 10 | 7 | 17 | 12 | 9 | 21 | 14 | 10 | 24 | 14 | 10 | 24 |
| 4 | Pepper | 1 | 5 | 6 | 1 | 10 | 11 | 1 | 12 | 13 | 1 | 14 | 15 | 14 | 10 | 16 |
| 5 | Tomato | 4 | 0 | 4 | 5.5 |  | 5.5 | 7 |  | 7 | 7.5 | 0 | 7.5 | 0 | 8 | 8 |
| 6 | Mango | 0 | 5 | 5 | 0 | 5 | 5 |  | 5 | 5 |  | 5 | 5 | 0 | 5 | 5 |
| 7 | Avocad | 0 | 6 | 6 | 0 | 6 | 6 |  | 6 | 6 | 0 | 6 | 6 | 0 | 6 | 6 |
| 8 | Banana | 0 | 10 | 10 | 0 | 10 | 10 |  | 10 | 10 | 0 | 10 | 10 | 0 | 10 | 10 |
| 9 | Papaya | . | 3 | 3 | 15 | 17 | 32 |  | 3 | 3 | 0 | 3 | 3 | 0 | 3 | 3 |
|  | Total | 30 | 51 | 81 | 54.5 | 76 | 131 | 51 | 75 | 126 | 59.5 | 79 | 138.5 | 65 | 83 | 140 |
|  | 159% |  |  |  | 171% |  |  | 168 |  |  | 175.30% |  |  | 169% |  |  |

| Table-8 Value with project situation |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| No | crops | Price | Year 1 |  |  |  |  |  |  | Year 2 |  |  |  |  |  |  |
|  |  |  | Phase 1 |  | Value | Phase 2 |  | Value | Total | Phase 1 |  | Value | Phase 2 |  | Value | Total |
|  |  |  | ha | Yield/ha |  | ha | Yield/ha |  | value | ha | Yield/ha |  | ha | Yield/ha |  | Value |
|  |  |  |  | (Qt) |  |  | (Qt) |  |  |  | (Qt) |  |  | (Qt) |  |  |
| 1 | Maize | 700 | 12 | 35 | 294,000.00 | 15 | 33 | 346500 | 640,500 | 15 | 45 | 472,500 | 17 | 43 | 511,700 | 984,200 |
| 2 | Haricoat bean | 850 | 6 | 8 | 40,800.00 | 2 | 9 | 15300 | 56,100 | 8 | 11 | 74,800 | 4 | 12 | 40,800 | 115,600 |
| 3 | Onion | 800 | 7 | 120 | 672,000.00 | 5 | 100 | 400000 | 1,072,000 | 10 | 135 | 1,080,000 | 7 | 115 | 644,000 | 1,724,000 |
| 4 | Pepper | 8000 | 1 | 4 | 32,000.00 | 5 | 3 | 120000 | 152,000 | 1 | 5 | 40,000 | 10 | 4 | 320,000 | 360,000 |
| 5 | Tomato | 1300 | 4 | 100 | 520,000.00 | 0 | 150 | 0 | 520,000 | 6 | 150 | 1,072,500 | 0 | 200 | - | 1,072,500 |
| 6 | Mango | 500 | 0 | 80 | - | 5 | 80 | 200000 | 200,000 | 0 | 150 | - | 5 | 85 | 212,500 | 127,500 |
| 7 | Avocado | 600 | 0 | 70 | - | 6 | 70 | 252000 | 252,000 | 0 | 80 | - | 6 | 80 | 288,000 | 720,000 |
| 8 | Banana | 1200 | 0 | 70 | - | 10 | 70 | 840000 | 840,000 | 0 | 85 | - | 10 | 85 | 1,020,000 | 1,020,000 |
| 9 | papaya | 200 | 0 | 65 | - | 3 | 65 | 39000 | 39,000 | 0 | 75 | - | 3 | 75 | 45,000 | 28,125 |
|  | Total |  |  |  |  |  |  |  | 3,771,600 |  |  |  |  |  |  | 6,151,925 |

| Year 3 |  |  |  |  |  |  | Year 4 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Phase 1 |  | Value | Phase 2 |  | Value | Total | Phase 1 |  | Value | Phase 2 |  | Value | Total |
| ha | Yield/ha |  | ha | Yield/ha |  | Vlue | ha | Yield/ha |  | ha | Yield/ha |  | Value |
|  | (Qt) |  |  | (Qt) |  |  |  | (Qt) |  |  | (Qt) |  |  |
| 20 | 25 | 350,000 | 25 | 53 | 927,500 | 1,277,500 | 25 | 65 | 1,137,500 | 25 | 63 | 1,102,500 | 2,240,000 |
| 5 | 15 | 63,750 | 0 | 15 |  | 63,750 | 12 | 17 | 173,400 | 6 | 18 | 91,800 | 265,200 |
| 12 | 170 | 1,632,000 | 9 | 150 | 1,080,000 | 2,712,000 | 14 | 200 | 2,240,000 | 10 | 185 | 1,480,000 | TRUE |
| 1 | 9 | 72,000 | 12 | 8 | 768,000 | 840,000 | 1 | 11 | 88,000 | 14 | 10 | 1,120,000 | 1,208,000 |
| 7 | 200 | 1,820,000 | 0 | 250 | - | 1,820,000 | 8 | 250 | 2,437,500 | 0 | 300 | - | 2,437,500 |
| 7 | 200 | 700,000 | 0 | 250 | - | 700,000 | 0 | 250 | - | 5 | 150 | 375,000 | 375,000 |
| 0 | 120 | - | 5 | 120 | 360,000 | 360,000 | 0 | 250 | - | 6 | 150 | 540,000 | 540,000 |
| 0 | 85 | - | 10 | 85 | 1,020,000 | 1,020,000 | 0 | 10 | - | 10 | 95 | 1,140,000 | 1,140,000 |
| 0 | 85 | - | 3 | 85 | 51,000 | 51,000 | 0 | 95 | - | 3 | 95 | 57,000 | 57,000 |
|  |  |  |  |  |  | 8,844,250 |  |  |  |  |  |  | 8,262,700 |

| Year 5 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Phase 1 |  | Value | Phase 2 |  | Value | Total |
| ha | Yield/ha |  | ha | Yield |  |  |
|  | (Qt) |  |  | /ha |  |  |
| 25 | 65 | 1,137,500.00 | 12 | 65 | 546,000 | 1,683,500 |
| 12 | 20 | 204,000.00 | 6 | 30 | 153,000 | 357,000 |
| 14 | 230 | 2,576,000.00 | 10 | 230 | 1,840,000 | 4,416,000 |
| 1 | 12 | 96,000.00 | 8 | 12 | 768,000 | 864,000 |
| 7.5 | 350 | 3,412,500.00 | 0 | 350 | - | 3,412,500.00 |
| 0 | 160 | - | 5 | 160 | 400,000 | 400,000.00 |
| 0 | 152 | - | 6 | 152 | 547,200 | 547,200.00 |
| 0 | 105 | - | 5 | 105 | 630,000 | 630,000 |
| 0 | 100 | - | 3 | 100 | 60,000 | 60,000 |
|  |  |  |  |  |  | 12,370,200 |

| Table-9 With the project investment cost (000) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Sir |  | investment |  |  | Period of Implemntation Year |  |  |  |  |  |  |  |  |  |
| No | Destription | Foreign | Local | Total | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10to20 |
| 1 | Proposed Scheme (New) |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1.1 | Investment cost | - | 109.00 | 109.00 | 109.00 |  |  |  |  |  |  |  |  |  |
| 1.2 | Meintenance and Replecment |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1.2.1 | Maintenance /2%/ |  | 2.18 | 2.18 | 2.18 |  | 2.18 | 2.18 | 2.18 | 2.18 | 2.18 | 2.18 | 2.18 | 2.18 |
| 1.2.2 | Replecment Cost of the major structure (Weir site)10% | 0.10 | 10.90 | 10.90 | 10.9 | 10.90 | 10.90 | 10.90 | 10.90 | 10.90 | 10.90 | 10.90 | 10.90 | 10.90 |
|  | Total |  | 13.08 | 13.08 | 13.08 | 13.03 | 13.03 | 13.03 | 13.03 | 13.03 | 13.03 | 13.03 | 13.03 | 13.03 |

| Table-10-With " the Project Other investment Costs |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| No | Cost Item | Unit | Quantity by project yers |  |  | Unit Price | Total Costs by Project year |  |  |
|  |  |  | 2 | 5 | 7 | (Birr) | 2 | 5 | 7 |
| A | Other investment Costs |  |  |  |  |  |  |  |  |
| I | Farm Tools |  |  |  |  |  |  |  |  |
| 1.1 | knapsack sprayerwith accessories | No | 70 | 85 | 70 | 500 | 35000 | 12500 | 25000 |
| 1.2 | Plough | No | 97 | 90 | 90 | 150 | 14550 | 13500 | 13500 |
| 1.3 | wheel | ,, | 97 | 45 | 90 | 70 | 6,790 | 3,150 | 6,300 |
| 1.4 | Light axe | ,, | 97 | 45 | 100 | 75 | 7,275 | 3,375 | 7,500 |
| 1.5 | Hoe | ,, | 97 | 45 | 90 | 80 | 7,760 | 3,600 | 7,200 |
| 1.6 | spade | ,, | 97 | 45 | 90 | 75 | 7,275 | 3,375 | 6,750 |
| 1.7 | Shovel | ,, | 97 | 45 | 90 | 85 | 8,245 | 3,825 | 7,650 |
| 1.8 | Rake | ,, | 97 | 45 | 100 | 150 | 14,550 | 6,750 | 15,000 |
| 1.9 | Wogal | ,, | 97 | 45 | 90 | 80 | 7,760 | 3,600 | 7,200 |
| 10.0 | Sickle | ,, | 97 | 45 | 100 | 120 | 11,640 | 5,400 | 12,000 |
|  | sub Total |  |  |  |  |  | 120,845 | 59,075 | 108,100 |
| II | Motor cycle |  | 1 |  |  | 60000 | 60,000 |  |  |
|  | Total |  |  |  |  |  | 180,845 | 59,075 | 108,100 |

| Training cost with project situation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Description | total day | perdium | year-2 | Yeear-5 | year-7 | 9 | 11 | 15 | 20 |
| Development agent (2)/year | 15 | 147 | 4410 | 4410 | 4410 | 4410 | 4410 | 4410 | 4410 |
| District irrigation exprt(3)/year | 15 | 147 | 6615 | 6615 | 6615 | 6615 | 6615 | 6615 | 6615 |
| Inovative Farmers (120)/year | 15 | 70 | 126,000 | 126,000 | 126,000 | 126,000 | 126,000 | 126,000 | 126,000 |
| WUA Members (7)/year | 15 | 70 | 7350 | 7350 | 7350 | 7350 | 7350 | 7350 | 7350 |
| Total |  |  | 144,375.00 | 144,375.00 | 144,375.00 | 144,375.00 | 144,375.00 | 144,375.00 | 144,375.00 |

| Table 12 Tax Paid ''With Out'' the project |  |  |  |  |
| --- | --- | --- | --- | --- |
| No | Tax – Item | Rate/ha | Area | Total tax/year |
| 1 | Land Rent | 80 | 30 | 2,400.00 |
| 2 | Income Tax | 250 | 30 | 7,500.00 |
|  | Total |  |  | 9,900.00 |
| Table -13 Tax Paid '' with '' the project |  |  |  |  |
| No | Tax – Item | Rate/ha | Area | Total tax/year |
| 1 | Land Rent | 150 | 80 | 12,000.00 |
| 2 | Income Tax | 200 | 80 | 16,000.00 |
|  | Total |  |  | 28,000.00 |

| Table- 15-Summary of Costs for Bill of Quantity= |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| No | Work Item | Cost (ETB) | % Cost | Comm. Share | Community (%) |
| 1 | Access road,Camping & Moblization | 367,039.83 | 5.25 | 560435.45 | 9.89 |
| 2 | Head Work | 109,662.08 | 21.51 |  |  |
| 3 | Canals and other irrigation infrasturactures | 4,193,590.17 | 69.4 |  |  |
| 4 | Total cost | 4,670,292.08 |  |  |  |
|  | supervision cost=5%project cost | 233,514.68 | 5 |  |  |
| 5 | VAT =15 | 700,543.81 |  |  |  |
| 6 | Grand total project cost | 5,604,350.58 |  |  |  |
| 7 | Cost per hectare | 70,054.38 |  |  |  |
|  | Total |  | 100 |  |  |

| Appendix - 18 Financial Analysis Result summery (000) |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Project | Without project |  |  |  | With the project investment cost |  |  | With the project operating cost |  |  |  |  |  |
| Year | Traditional irr. | Tax | Direct pro. | Total cost | Irrgation | Farm | Total Inv. | Production | Training | Main. | Repl. | Tax | Total oper. |
|  |  |  | Cost |  | Structure | tools | Cost | Cost |  |  | Cost |  | cost |
| 1 | 841 | 9.00 | 929.7 | 1,779.69 | 5,604.00 | 0 | 5604 | 2242 | 144.38 | 0 | 0 | 28.00 | 2414.38 |
| 2 | 841 | 9.00 | 929.7 | 1,779.70 | 0 | 180.845 | 180.845 | 4433 | 144.38 | 2.18 | 10.90 | 28.00 | 4618.46 |
| 3 | 841 | 9.00 | 929.7 | 1,779.70 | 0 | 0 | 0 | 5280 | 144.38 | 2.18 | 10.90 | 28.00 | 5465.46 |
| 4 | 841 | 9.00 | 929.7 | 1,779.70 | 0 | 0 | 0 | 3781 | 144.38 | 2.18 | 10.90 | 28.00 | 3966.46 |
| 5 | 841 | 9.00 | 929.7 | 1,779.70 | 0 | 59.075 | 59.075 | 5934 | 144.38 | 2.18 | 10.90 | 28.00 | 6119.46 |
| 6 | 841 | 9.00 | 929.7 | 1,779.70 | 0 | 0 | 0 | 5934 | 144.38 | 2.18 | 10.90 | 28.00 | 6119.46 |
| 7 | 841 | 9.00 | 929.7 | 1,779.70 | 0 | 108.1 | 108.11 | 5934 | 144.38 | 2.18 | 10.90 | 28.00 | 6119.46 |
| 8 | 841 | 9.00 | 929.7 | 1,779.70 | 0 | 0 | 0 | 5934 | 144.38 | 2.18 | 10.90 | 28.00 | 6119.46 |
| 9 | 841 | 9.00 | 929.7 | 1,779.70 | 0 | 0 | 0 | 5934 | 144.38 | 2.18 | 10.90 | 28.00 | 6119.46 |
| 10 | 841 | 9.00 | 929.7 | 1,779.70 | 0 | 0 | 0 | 5934 | 144.38 | 2.18 | 10.90 | 28.00 | 6119.46 |
| 11 | 841 | 9.00 | 929.7 | 1,779.70 | 0 | 81 | 0 | 5934 | 144.38 | 2.18 | 10.90 | 28.00 | 6119.46 |
| 12 | 841 | 9.00 | 929.7 | 1,779.70 | 0 | 0 | 0 | 5934 | 144.38 | 2.18 | 10.90 | 28.00 | 6119.46 |
| 13 | 841 | 9.00 | 929.7 | 1,779.70 | 0 | 0 | 0 | 5934 | 144.38 | 2.18 | 10.90 | 28.00 | 6119.46 |
| 14 | 841 | 9.00 | 929.7 | 1,779.70 | 0 | 0 | 0 | 5934 | 144.38 | 2.18 | 10.90 | 28.00 | 6119.46 |
| 15 | 841 | 9.00 | 929.7 | 1,779.70 | 0 | 0 | 0 | 5934 | 144.38 | 2.18 | 10.90 | 28.00 | 6119.46 |
| 16 | 841 | 9.00 | 929.7 | 1,779.70 | 0 | 0 | 0 | 5934 | 144.38 | 2.18 | 10.90 | 28.00 | 6119.46 |
| 17 | 841 | 9.00 | 929.7 | 1,779.70 | 0 | 0 | 0 | 5934 | 144.38 | 2.18 | 10.90 | 28.00 | 6119.46 |
| 18 | 841 | 9.00 | 929.7 | 1,779.70 | 0 | 0 | 0 | 5934 | 144.38 | 2.18 | 10.90 | 28.00 | 6119.46 |
| 19 | 841 | 9.00 | 929.7 | 1,779.70 | 0 | 0 | 0 | 5934 | 144.38 | 2.18 | 10.90 | 28.00 | 6119.46 |
| 20 | 841 | 9.00 | 929.7 | 1,779.70 | 0 | 0 | 0 | 5934 | 144.38 | 2.18 | 10.90 | 28.00 | 6119.46 |
| NPV at 11% Discount Rate |  |  |  |  |  |  |  |  |  |  |  |  |  |
| IRR at 11% Discount Rate |  |  |  |  |  |  |  |  |  |  |  |  |  |
| B/C at 11% Discount Rate |  |  |  |  |  |  |  |  |  |  |  |  |  |

| Base | Physical | Total | Increm. | Gross value of production |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Cost | conti. | W.Project | proj. | with out | with the | Increm. | Net incr. |
| (BC) | 5% of BC | cost | cost | proj. | proj. | benefit | Benefit |
| 8018.38 | 400.92 | 8419.29 | 6640 | 883.00 | 3,771.60 | 2888.6 | -3751 |
| 4799.30 | 239.97 | 5039.27 | 3260 | 883.00 | 6151.93 | 5268.9 | 2009 |
| 5465.46 | 273.27 | 5738.73 | 3959 | 883.00 | 8844.25 | 7961.3 | 4002 |
| 3966.46 | 198.32 | 4164.78 | 2385 | 883.00 | 8262.70 | 7379.7 | 4995 |
| 6178.53 | 308.93 | 6487.46 | 4708 | 883.00 | 12370.20 | 11487.2 | 6779 |
| 6119.46 | 305.97 | 6425.43 | 4646 | 883.00 | 12370.20 | 11487.2 | 6841 |
| 6227.57 | 311.38 | 6538.94 | 4759 | 883.00 | 12370.20 | 11487.2 | 6728 |
| 6119.46 | 305.97 | 6425.43 | 4646 | 883.00 | 12370.20 | 11487.2 | 6841 |
| 6119.46 | 305.97 | 6425.43 | 4646 | 883.00 | 12370.20 | 11487.2 | 6841 |
| 6119.46 | 305.97 | 6425.43 | 4646 | 883.00 | 12370.20 | 11487.2 | 6841 |
| 6119.46 | 305.97 | 6425.43 | 4646 | 883.00 | 12370.20 | 11487.2 | 6841 |
| 6119.46 | 305.97 | 6425.43 | 4646 | 883.00 | 12370.20 | 11487.2 | 6841 |
| 6119.46 | 305.97 | 6425.43 | 4646 | 883.00 | 12370.20 | 11487.2 | 6841 |
| 6119.46 | 305.97 | 6425.43 |  | 883.00 | 12370.20 | 11487.2 | 11487 |
| 6119.46 | 305.97 | 6425.43 | 4646 | 883.00 | 12370.20 | 11487.2 | 6841 |
| 6119.46 | 305.97 | 6425.43 | 4646 | 883.00 | 12370.20 | 11487.2 | 6841 |
| 6119.46 | 305.97 | 6425.43 | 4646 | 883.00 | 12370.20 | 11487.2 | 6841 |
| 6119.46 | 305.97 | 6425.43 | 4646 | 883.00 | 12370.20 | 11487.2 | 6841 |
| 6119.46 | 305.97 | 6425.43 | 4646 | 883.00 | 12370.20 | 11487.2 | 6841 |
| 6119.46 | 305.97 | 1516.68 | 4734 | 883.00 | 12370.20 | 11487.2 | 6753 |
|  |  |  |  |  |  | $73,399.08 |  |
|  |  |  |  |  |  |  | 94% |
|  |  |  | $35,202.89 |  |  | $73,399.08 | 2.09 |

|  | Sensitivity Analysis (000) |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Increasing project cost by 20% maintaing other factors constant |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Project | Without project |  |  |  | With the project investment cost |  |  | With the project operating cost |  |  |  |  |  | Base |
| Year | Traditional irr. | Tax | Direct pro. | Total cost | Irrgation | Farm | Total Inv. | Production | Training | Main. | Repl. | Tax | Total oper. | Cost |
|  |  |  | Cost |  | Structure | tools | Cost | Cost |  |  | Cost |  | cost | (BC) |
| 1 | 841 | 9.00 | 930 | 1,779.70 | # | 0 | 5604 | 2242 | 144.38 | 0 | 0 | 28.00 | 2414.38 | 8018.38 |
| 2 | 841 | 9.00 | 1112 | 1,961.70 | 0 | 153.000 | 153.000 | 4433 | 144.38 | 2.18 | 10.90 | 28.00 | 4618.46 | 4771.46 |
| 3 | 841 | 9.00 | 930 | 1,779.70 | 0 | 0 | 0 | 5280 | 144.38 | 2.18 | 10.90 | 28.00 | 5465.46 | 5465.46 |
| 4 | 841 | 9.00 | 930 | 1,779.70 | 0 | 0 | 0 | 3781 | 144.38 | 2.18 | 10.90 | 28.00 | 3966.46 | 3966.46 |
| 5 | 841 | 9.00 | 930 | 1,779.70 | 0 | 55 | 55 | 5934 | 144.38 | 2.18 | 10.90 | 28.00 | 6119.46 | 6174.46 |
| 6 | 841 | 9.00 | 930 | 1,779.70 | 0 | 0 | 0 | 5934 | 144.38 | 2.18 | 10.90 | 28.00 | 6119.46 | 6119.46 |
| 7 | 841 | 9.00 | 930 | 1,779.70 | 0 | 81 | 81 | 5934 | 144.38 | 2.18 | 10.90 | 28.00 | 6119.46 | 6200.46 |
| 8 | 841 | 9.00 | 930 | 1,779.70 | 0 | 0 | 0 | 5934 | 144.38 | 2.18 | 10.90 | 28.00 | 6119.46 | 6119.46 |
| 9 | 841 | 9.00 | 930 | 1,779.70 | 0 | 0 | 0 | 5934 | 144.38 | 2.18 | 10.90 | 28.00 | 6119.46 | 6119.46 |
| 10 | 841 | 9.00 | 930 | 1,779.70 | 0 | 0 | 0 | 5934 | 144.38 | 2.18 | 10.90 | 28.00 | 6119.46 | 6119.46 |
| 11 | 841 | 9.00 | 930 | 1,779.70 | 0 | 81 | 0 | 5934 | 144.38 | 2.18 | 10.90 | 28.00 | 6119.46 | 6119.46 |
| 12 | 841 | 9.00 | 930 | 1,779.70 | 0 | 0 | 0 | 5934 | 144.38 | 2.18 | 10.90 | 28.00 | 6119.46 | 6119.46 |
| 13 | 841 | 9.00 | 930 | 1,779.70 | 0 | 0 | 0 | 5934 | 144.38 | 2.18 | 10.90 | 28.00 | 6119.46 | 6119.46 |
| 14 | 841 | 9.00 | 930 | 1,779.70 | 0 | 0 | 0 | 5934 | 144.38 | 2.18 | 10.90 | 28.00 | 6119.46 | 6119.46 |
| 15 | 841 | 9.00 | 930 | 1,779.70 | 0 | 0 | 0 | 5934 | 144.38 | 2.18 | 10.90 | 28.00 | 6119.46 | 6119.46 |
| 16 | 841 | 9.00 | 930 | 1,779.70 | 0 | 0 | 0 | 5934 | 144.38 | 2.18 | 10.90 | 28.00 | 6119.46 | 6119.46 |
| 17 | 841 | 9.00 | 930 | 1,779.70 | 0 | 0 | 0 | 5934 | 144.38 | 2.18 | 10.90 | 28.00 | 6119.46 | 6119.46 |
| 18 | 841 | 9.00 | 930 | 1,779.70 | 0 | 0 | 0 | 5934 | 144.38 | 2.18 | 10.90 | 28.00 | 6119.46 | 6119.46 |
| 19 | 841 | 9.00 | 930 | 1,779.70 | 0 | 0 | 0 | 5934 | 144.38 | 2.18 | 10.90 | 28.00 | 6119.46 | 6119.46 |
| 20 | 841 | 9.00 | 930 | 1,779.70 | 0 | 0 | 0 | 5934 | 144.38 | 2.18 | 10.90 | 28.00 | 6119.46 | 6119.46 |
| NPV at 11% Discount Rate |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| IRR at 11% Discount Rate |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| B/C at 11% Discount Rate |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

| Physical | Total | Increases | Increm. | Gross value of production |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| conti. | W.Project | Proj.cost | proj. | with out | with the | Increm. | Net incr. |
| 5% of BC | cost | By 20% | cost | proj. | proj. | benefit | Benefit |
| 324.77 | 8343.14 | 8343.14 | 6563 | 883.00 | 3,771.60 | 2888.6 | -3675 |
| 238.57 | 6886.48 | 6886.48 | 4925 | 883.00 | 6151.93 | 5268.9 | 344 |
| 273.27 | 5738.73 | 6886.48 | 5107 | 883.00 | 8844.25 | 7961.3 | 2854 |
| 198.32 | 4164.78 | 4997.74 | 3218 | 883.00 | 8262.70 | 7379.7 | 4162 |
| 308.72 | 6483.18 | 7779.82 | 6000 | 883.00 | 12370.20 | 11487.2 | 5487 |
| 305.97 | 6425.43 | 6728.77 | 4949 | 883.00 | 12370.20 | 11487.2 | 6538 |
| 310.02 | 6510.48 | 6830.81 | 5051 | 883.00 | 12370.20 | 11487.2 | 6436 |
| 305.97 | 6425.43 | 6745.76 | 4966 | 883.00 | 12370.20 | 11487.2 | 6521 |
| 305.97 | 6425.43 | 6745.76 | 4966 | 883.00 | 12370.20 | 11487.2 | 6521 |
| 305.97 | 6425.43 | 6745.76 | 4966 | 883.00 | 12370.20 | 11487.2 | 4741 |
| 305.97 | 6425.43 | 6745.76 | 4966 | 883.00 | 12370.20 | 13747.7 | 7002 |
| 305.97 | 6425.43 | 6745.76 | 4966 | 883.00 | 12370.20 | 13747.7 | 7002 |
| 305.97 | 6425.43 | 6745.76 | 4966 | 883.00 | 12370.20 | 13747.7 | 7002 |
| 305.97 | 6425.43 | 6745.76 | 4966 | 883.00 | 12370.20 | 13747.7 | 7002 |
| 305.97 | 6425.43 | 6745.76 | 4966 | 883.00 | 12370.20 | 13747.7 | 7002 |
| 305.97 | 6425.43 | 6745.76 | 4966 | 883.00 | 12370.20 | 13747.7 | 7002 |
| 305.97 | 6425.43 | 6745.76 | 4966 | 883.00 | 12370.20 | 13747.7 | 7002 |
| 305.97 | 6425.43 | 6745.76 | 4966 | 883.00 | 12370.20 | 13747.7 | 7002 |
| 305.97 | 6425.43 | 6745.76 | 4966 | 883.00 | 12370.20 | 13747.7 | 7002 |
| 305.97 | 1516.68 | 1837.01 | 57 | 883.00 | 12370.20 | 13747.7 | 11911 |
|  |  |  | $86,813.21 |  |  | $78,087.5 |  |
|  |  |  |  |  |  |  | 71% |
|  |  |  | $39,940.09 |  |  | $78,087.54 | 1.96 |

|  | Sensitivity Analysis (000) |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Decreasing project benefit by 20% maintaing other factors constant |  |  |  |  |  |  |  |  |  |  |  |  |
| Project | Without project |  |  |  | With the project investment cost |  | With the project operating cost |  |  |  |  |  | Base |
| Year | Traditional irr. | Tax | Direct pro. | Total cost | Irrgation | Total Inv. | Production | Training | Main. | Repl. | Tax | Total oper. | Cost |
|  |  |  | Cost |  | Structure | Cost | Cost |  |  | Cost |  | cost | (BC) |
| 1 | 841 | 9.00 | 930 | 1,779.70 | 5,604.00 | 5604 | 2242 | 144.38 | 0 | 0 | 28.00 | 2414.38 | 8018.38 |
| 2 | 841 | 9.00 | 930 | 1,779.70 | 0 | 153.000 | 4433 | 144.38 | 2.18 | 10.90 | 28.00 | 4618.46 | 4771.46 |
| 3 | 841 | 9.00 | 930 | 1,779.70 | 0 | 0 | 5280 | 144.38 | 2.18 | 10.90 | 28.00 | 5465.46 | 5465.46 |
| 4 | 841 | 9.00 | 930 | 1,779.70 | 0 | 0 | 3781 | 144.38 | 2.18 | 10.90 | 28.00 | 3966.46 | 3966.46 |
| 5 | 841 | 9.00 | 930 | 1,779.70 | 0 | 55 | 5934 | 144.38 | 2.18 | 10.90 | 28.00 | 6119.46 | 6174.46 |
| 6 | 841 | 9.00 | 930 | 1,779.70 | 0 | 0 | 5934 | 144.38 | 2.18 | 10.90 | 28.00 | 6119.46 | 6119.46 |
| 7 | 841 | 9.00 | 930 | 1,779.70 | 0 | 81 | 5934 | 144.38 | 2.18 | 10.90 | 28.00 | 6119.46 | 6200.46 |
| 8 | 841 | 9.00 | 930 | 1,779.70 | 0 | 0 | 5934 | 144.38 | 2.18 | 10.90 | 28.00 | 6119.46 | 6119.46 |
| 9 | 841 | 9.00 | 930 | 1,779.70 | 0 | 0 | 5934 | 144.38 | 2.18 | 10.90 | 28.00 | 6119.46 | 6119.46 |
| 10 | 841 | 9.00 | 930 | 1,779.70 | 0 | 0 | 5934 | 144.38 | 2.18 | 10.90 | 28.00 | 6119.46 | 6119.46 |
| 11 | 841 | 9.00 | 930 | 1,779.70 | 0 | 0 | 5934 | 144.38 | 2.18 | 10.90 | 28.00 | 6119.46 | 6119.46 |
| 12 | 841 | 9.00 | 930 | 1,779.70 | 0 | 0 | 5934 | 144.38 | 2.18 | 10.90 | 28.00 | 6119.46 | 6119.46 |
| 13 | 841 | 9.00 | 930 | 1,779.70 | 0 | 0 | 5934 | 144.38 | 2.18 | 10.90 | 28.00 | 6119.46 | 6119.46 |
| 14 | 841 | 9.00 | 930 | 1,779.70 | 0 | 0 | 5934 | 144.38 | 2.18 | 10.90 | 28.00 | 6119.46 | 6119.46 |
| 15 | 841 | 9.00 | 930 | 1,779.70 | 0 | 0 | 5934 | 144.38 | 2.18 | 10.90 | 28.00 | 6119.46 | 6119.46 |
| 16 | 841 | 9.00 | 930 | 1,779.70 | 0 | 0 | 5934 | 144.38 | 2.18 | 10.90 | 28.00 | 6119.46 | 6119.46 |
| 17 | 841 | 9.00 | 930 | 1,779.70 | 0 | 0 | 5934 | 144.38 | 2.18 | 10.90 | 28.00 | 6119.46 | 6119.46 |
| 18 | 841 | 9.00 | 930 | 1,779.70 | 0 | 0 | 5934 | 144.38 | 2.18 | 10.90 | 28.00 | 6119.46 | 6119.46 |
| 19 | 841 | 9.00 | 930 | 1,779.70 | 0 | 0 | 5934 | 144.38 | 2.18 | 10.90 | 28.00 | 6119.46 | 6119.46 |
| 20 | 841 | 9.00 | 930 | 1,779.70 | 0 | 0 | 5934 | 144.38 | 2.18 | 10.90 | 28.00 | 6119.46 | 6119.46 |
| NPV at 11% Discount Rate |  |  |  |  |  |  |  |  |  |  |  |  |  |
| IRR at 11% Discount Rate |  |  |  |  |  |  |  |  |  |  |  |  |  |
| B/C at 11% Discount Rate |  |  |  |  |  |  |  |  |  |  |  |  |  |

| Physical | Total | Increm. | Gross value of production |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| conti. | W.Project | proj. | with out | with the | Decr.proj. benefitt | Increm. | Net incr. |
| 5% of BC | cost | cost | proj. | proj. | by 20% | benefit | Benefit |
| 400.92 | 8419.29 | 7490 | 883.00 | 3,771.60 | 3017.6 | 2135 | -5355 |
| 238.57 | 5010.03 | 4080 | 883.00 | 6151.93 | 4922.0 | 4039 | -41 |
| 273.27 | 5738.73 | 4809 | 883.00 | 8844.25 | 7075.4 | 6192 | 1383 |
| 198.32 | 4164.78 | 3235 | 883.00 | 8262.70 | 8262.7 | 7380 | 4145 |
| 308.72 | 6483.18 | 5496 | 883.00 | 12370.20 | 9896.2 | 9013 | 3517 |
| 305.97 | 6425.43 | 5496 | 883.00 | 12370.20 | 11235.2 | 10352 | 4856 |
| 310.02 | 6510.48 | 5581 | 883.00 | 12370.20 | 11235.2 | 10352 | 4771 |
| 305.97 | 6425.43 | 5496 | 883.00 | 12370.20 | 11235.2 | 10352 | 4856 |
| 305.97 | 6425.43 | 5004 | 883.00 | 12370.20 | 11235.2 | 10352 | 5348 |
| 305.97 | 6425.43 | 5004 | 883.00 | 12370.20 | 11235.2 | 10352 | 5348 |
| 305.97 | 6425.43 | 5004 | 883.00 | 12370.20 | 11235.2 | 10352 | 5348 |
| 305.97 | 6425.43 | 5004 | 883.00 | 12370.20 | 11235.2 | 10352 | 5348 |
| 305.97 | 6425.43 | 5004 | 883.00 | 12370.20 | 11235.2 | 10352 | 5348 |
| 305.97 | 6425.43 | 5004 | 883.00 | 12370.20 | 11235.2 | 10352 | 5348 |
| 305.97 | 6425.43 | 5004 | 883.00 | 12370.20 | 11235.2 | 10352 | 5348 |
| 305.97 | 6425.43 | 5004 | 883.00 | 12370.20 | 11235.2 | 10352 | 5348 |
| 305.97 | 6425.43 | 5004 | 883.00 | 12370.20 | 11235.2 | 10352 | 5348 |
| 305.97 | 6425.43 | 5004 | 883.00 | 12370.20 | 11235.2 | 10352 | 5348 |
| 305.97 | 6425.43 | 5004 | 883.00 | 12370.20 | 11235.2 | 10352 | 5348 |
| 305.97 | 1516.68 | 5004 | 883.00 | 12370.20 | 11235.2 | 10352 | 5348 |
|  |  |  |  |  |  | $64,116.44 |  |
|  |  |  |  |  |  |  | 45% |
|  |  | $41,077.38 |  |  |  | $64,116.44 | 1.56 |

| Sensitivity Analysis (000) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| The simultaneous increase of cost by 20% and decrease of benefit by 20% maintaining other factors constant |  |  |  |  |  |

| Project | Without project |  |  |  | With the project investment cost |  |  | With the project operating cost |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Year | Traditional irr. | Tax | Direct pro. | Total cost | Irrgation | Farm | Total Inv. | Production | Training | Main. | Repl. | Tax | Total oper. |
|  |  |  | Cost |  | Structure | tools | Cost | Cost |  |  | Cost |  | cost |
| 1 | 841 | 9.00 | 930 | 1,779.70 | 5,604.00 | 0 | 5604 | 2242 | 144.38 | 0 | 0 | 28.00 | 2414.38 |
| 2 | 841 | 9.00 | 930 | 1,779.70 | 0 | 153.000 | 153.000 | 4433 | 144.38 | 2.18 | 10.90 | 28.00 | 4618.46 |
| 3 | 841 | 9.00 | 930 | 1,779.70 | 0 | 0 | 0 | 5280 | 144.38 | 2.18 | 10.90 | 28.00 | 5465.46 |
| 4 | 841 | 9.00 | 930 | 1,779.70 | 0 | 0 | 0 | 3781 | 144.38 | 2.18 | 10.90 | 28.00 | 3966.46 |
| 5 | 841 | 9.00 | 930 | 1,779.70 | 0 | 55 | 55 | 5934 | 144.38 | 2.18 | 10.90 | 28.00 | 6119.46 |
| 6 | 841 | 9.00 | 930 | 1,779.70 | 0 | 0 | 0 | 5934 | 144.38 | 2.18 | 10.90 | 28.00 | 6119.46 |
| 7 | 841 | 9.00 | 930 | 1,779.70 | 0 | 81 | 81 | 5934 | 144.38 | 2.18 | 10.90 | 28.00 | 6119.46 |
| 8 | 841 | 9.00 | 930 | 1,779.70 | 0 | 0 | 0 | 5934 | 144.38 | 2.18 | 10.90 | 28.00 | 6119.46 |
| 9 | 841 | 9.00 | 930 | 1,779.70 | 0 | 0 | 0 | 5934 | 144.38 | 2.18 | 10.90 | 28.00 | 6119.46 |
| 10 | 841 | 9.00 | 930 | 1,779.70 | 0 | 0 | 0 | 5934 | 144.38 | 2.18 | 10.90 | 28.00 | 6119.46 |
| 11 | 841 | 9.00 | 930 | 1,779.70 | 0 | 81 | 0 | 5934 | 144.38 | 2.18 | 10.90 | 28.00 | 6119.46 |
| 12 | 841 | 9.00 | 930 | 1,779.70 | 0 | 0 | 0 | 5934 | 144.38 | 2.18 | 10.90 | 28.00 | 6119.46 |
| 13 | 841 | 9.00 | 930 | 1,779.70 | 0 | 0 | 0 | 5934 | 144.38 | 2.18 | 10.90 | 28.00 | 6119.46 |
| 14 | 841 | 9.00 | 930 | 1,779.70 | 0 | 0 | 0 | 5934 | 144.38 | 2.18 | 10.90 | 28.00 | 6119.46 |
| 15 | 841 | 9.00 | 930 | 1,779.70 | 0 | 0 | 0 | 5934 | 144.38 | 2.18 | 10.90 | 28.00 | 6119.46 |
| 16 | 841 | 9.00 | 930 | 1,779.70 | 0 | 0 | 0 | 5934 | 144.38 | 2.18 | 10.90 | 28.00 | 6119.46 |
| 17 | 841 | 9.00 | 930 | 1,779.70 | 0 | 0 | 0 | 5934 | 144.38 | 2.18 | 10.90 | 28.00 | 6119.46 |
| 18 | 841 | 9.00 | 930 | 1,779.70 | 0 | 0 | 0 | 5934 | 144.38 | 2.18 | 10.90 | 28.00 | 6119.46 |
| 19 | 841 | 9.00 | 930 | 1,779.70 | 0 | 0 | 0 | 5934 | 144.38 | 2.18 | 10.90 | 28.00 | 6119.46 |
| 20 | 841 | 9.00 | 930 | 1,779.70 | 0 | 0 | 0 | 5934 | 144.38 | 2.18 | 10.90 | 28.00 | 6119.46 |
| NPV at 11% Discount Rate |  |  |  |  |  |  |  |  |  |  |  |  |  |
| IRR at 11% Discount Rate |  |  |  |  |  |  |  |  |  |  |  |  |  |
| B/C at 11% Discount Rate |  |  |  |  |  |  |  |  |  |  |  |  |  |

| Base | Physical | Total | Increases | Increm. | Gross value of production |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Cost | Conti. | W.Project | Proj.cost | proj. | with out | with the | Decr.proj.benefitt | Increm. | Net incr. |
| (BC) | 5% of BC | cost | By 20% | cost | proj. | proj. | by 20% | benefit | Benefit |
| 8018.38 | 324.77 | 8343.14 | 10011.77 | 8232 | 883.00 | 3,771.60 | 3017.6 | 2135 | -6097 |
| 4771.46 | 238.57 | 5010.03 | 6012.04 | 4232 | 883.00 | 6151.93 | 4922.0 | 4039 | -193 |
| 5465.46 | 273.27 | 5738.73 | 6886.48 | 5107 | 883.00 | 8844.25 | 7075.4 | 6192 | 1086 |
| 3966.46 | 198.32 | 4164.78 | 4997.74 | 3218 | 883.00 | 8262.70 | 6610.2 | 5727 | 2509 |
| 6174.46 | 308.72 | 6483.18 | 7779.82 | 5931 | 883.00 | 12370.20 | 9896.2 | 9013 | 3082 |
| 6119.46 | 305.97 | 6425.43 | 7710.52 | 5931 | 883.00 | 12370.20 | 9896.2 | 9013 | 3082 |
| 6200.46 | 310.02 | 6510.48 | 7812.58 | 5931 | 883.00 | 12370.20 | 9896.2 | 9013 | 3082 |
| 6119.46 | 305.97 | 6425.43 | 7710.52 | 5931 | 883.00 | 12370.20 | 9896.2 | 9013 | 3082 |
| 6119.46 | 305.97 | 6425.43 | 7710.52 | 5931 | 883.00 | 12370.20 | 9896.2 | 9013 | 3082 |
| 6119.46 | 305.97 | 6425.43 | 7710.52 | 5931 | 883.00 | 12370.20 | 9896.2 | 9013 | 3082 |
| 6119.46 | 305.97 | 6425.43 | 7710.52 | 5931 | 883.00 | 12370.20 | 9896.2 | 9013 | 3082 |
| 6119.46 | 305.97 | 6425.43 | 7710.52 | 5931 | 883.00 | 12370.20 | 9896.2 | 9013 | 3082 |
| 6119.46 | 305.97 | 6425.43 | 7710.52 | 5931 | 883.00 | 12370.20 | 9896.2 | 9013 | 3082 |
| 6119.46 | 305.97 | 6425.43 | 7710.52 | 5931 | 883.00 | 12370.20 | 9896.2 | 9013 | 3082 |
| 6119.46 | 305.97 | 6425.43 | 7710.52 | 5931 | 883.00 | 12370.20 | 9896.2 | 9013 | 3082 |
| 6119.46 | 305.97 | 6425.43 | 7710.52 | 5931 | 883.00 | 12370.20 | 9896.2 | 9013 | 3082 |
| 6119.46 | 305.97 | 6425.43 | 7710.52 | 5931 | 883.00 | 12370.20 | 9896.2 | 9013 | 3082 |
| 6119.46 | 305.97 | 6425.43 | 7710.52 | 5931 | 883.00 | 12370.20 | 9896.2 | 9013 | 3082 |
| 6119.46 | 305.97 | 6425.43 | 7710.52 | 5931 | 883.00 | 12370.20 | 9896.2 | 9013 | 3082 |
| 6119.46 | 305.97 | 6425.43 | 7710.52 | 5931 | 883.00 | 12370.20 | 9896.2 | 9013 | 3082 |
|  | $57,313.60 |  |  |  |  |  |  |  | $32,613.09 |
|  |  |  |  |  |  |  |  |  | 30% |
|  |  |  |  | $23,364.05 |  |  |  |  | 1.25 |

# -     Conclusion and recommendation

# Conclusion

An attempt was made to see every technical part of the headwork, canals and all hydraulic structures on the systems to minimizing any risky & constraints, the whole analyses made are on the safest side. Therefore, the project is technically feasible in engineering part.

# -Recommendation:

-To make the spring (head work) sustainable the following   measures have to be taken especially during construction.

-Both left & right banks of the river including the spring eye shouldn’t be disturbed while constructing the guide wall, the main canal and temporary guiding canal for head work construction as much as possible as the spring sours and type is not differentiated geo-physically and can be the cause to change the spring eye and its rout as a whole. If in case touched for an obligatory works, the banks should be restrictedly compacted and grasses must be grown on it.

-All excavated materials should be removed accordingly in such away that not to hinder the normal flow of the spring

-Necessary supervision work has to be there to control the construction quality and some specifications.

-The foundation condition needs routine supervision by the design engineer and the geologist during construction of the headwork.

-It is good task to put the guide wall completely as per the drawing provided.