Amhara National Regional State

Water, Irrigation and Energy Bureau (BoWIE) Detail Design Of Challie Diversion Small-Scale Irrigation ProjectVolume IV: Engineering Design Final Report

Client:Bureau of Water, Irrigation and Energy Bureau(BoWIE)

Address:

PO.Box:88
Telephone: 0528-200853/855
Fax: 251-08-20-65-68/204676/202040

Consultant: Amhara Design and Supervision Works Enterprise

(ADSWE)

Address:

P.O.Box:1921
Telephone: +251-582-181023/180638/181201/181254
Fax :( 058)2180550/ (058)2180560
E-mail:amhara design@yahoo.com

FEASIBILITY STUDY & DETAIL DESIGN REPORT STRUCTURE

- VolumeI: Watershed Management

- VolumeII: Engineering Geology

- VolumeIII: Irrigation Agronomy

- Volume IV: Engineering Design

- VolumeV: Socio Economy

- Volume VI: Environmental Impact Assessment

TABLE OF CONTENTS

SALIENT FEATURE	x

1	INTRODUCTION	1

1.1	General	1

1.2	Location and accessibility	2

1.3	Previous Irrigation Practices	3

1.4	Objectives of the Study	3

1.5	Scope of the Study	4

1.6	Methodology of study	4

2	HYDROLOGY	5

2.1	Hydro-Metrological Data Availability	5

2.1.1	Climate	5

2.1.2	Rainfall Data	5

2.1.3	River flow data	5

2.1.4	Upstream & Downstream utilization	6

2.2	Design Rainfall computation	6

2.2.1	Checking the existence of outliers	11

2.2.2	Checking reliability & Adequacy	12

2.2.3	Design Point Rainfall	12

2.3	Design Flood Analysis	13

2.3.1	Peak flood analysis by SCS-CN method	13

2.3.2	Peak flood analysis by Flood mark method	22

2.4	Tail water depth	25

3	HEAD WORK STRUCTURES DESIGN	28

3.1	Head work site selection	28

3.2	River Geomorphology	28

3.2.1	River Bed condition	28

3.2.2	River Bank condition	29

3.3	Sources of construction materials	30

3.4	Design of head work structures	31

3.4.1	Headwork Type Selection	31

3.4.2	Design of Diversion	32

3.4.3	Design of operation slab and Breast wall	33

3.4.4	Guide Walls	34

3.5	Bill of Quantity and Cost Estimation	37

4	IRRIGATION SYSTEM DESIGN	43

4.1	Irrigable Area Description	43

4.1.1	Topography	43

4.1.2	Climate	43

4.1.3	Soil characteristics	44

4.1.4	Existing Irrigation Practices in the Project Area	44

4.2	Irrigation Water Requirement	44

4.2.1	Crop Water Requirement (CWR)	44

4.2.2	Irrigation efficiency	45

4.2.3	Irrigation duty	45

4.2.4	Irrigation methods	47

4.2.5	Irrigation System Layout	47

4.2.6	Conveyance System	48

4.3	Hydraulic Design of Canals	48

4.3.1	Main Canal	49

4.3.2	Secondary Canals	51

4.3.3	Tertiary canals	53

4.3.4	Field Canals	55

4.4	Canal related Structures Design	55

4.4.1	Cross drainage works	55

4.4.2	Design of Division box	60

4.4.3	Chute structure	63

4.4.4	Drop structure	65

4.4.5	Road crossing structure	67

5	OPERATION AND MAINTENANCE	83

5.1	General	83

5.2	Irrigation System Operation	83

5.2.1	Method of operation	83

5.2.2	Methods of Maintenance	84

6	CONCLUSION AND RECOMMENDATIONS	85

6.1	Conclusions	85

6.2	Recommendations:	85

7	REFERENCES	86

LIST OF TABLES

Table 0-1 : Summary of Project Construction Costs	xi

Table 2-1: Haik metrological station maximum daily rainfall data	6

Table 2-2: Parts of catchment area	14

Table 2-3: Determination of Time of Concentration	15

Table 2-4: The Rainfall profile & areal to point ratio	17

Table 2-5: Runoff analysis	19

Table 2-6: Hydrograph coordinates	20

Table 2-7: Actual Surveying data taken along the river center	23

Table 2-8: Area, Perimeter and Discharge of the river using flood mark method	25

Table 2-9: Tail water depth and corresponding discharge at the head work site	26

Table 3-1: Typical design of masonry Guide wall	35

Table 3-2: Bill of quantity and cost estimation of general items	37

Table 3-3: Bill of quantity and cost estimation of general items	40

Table 4-1: Hydraulic Parameters of main canal	49

Table 4-2: Hydraulic Parameters of secondary canals	51

Table 4-3: Hydraulic Parameters of tertiary canals	53

Table 4-4: Hydraulic Parameters of Flumes	59

Table 4-5: Summery of Hydraulic parameters of Division Boxes	62

Table 4-6: Summery of Hydraulic parameters of Division Boxes	63

Table 4-7: Summery of Hydraulic parameters of Division Boxes	66

Table 4-8: Irrigation Infrastructure Bill of Quantities and Cost Estimate	68

LIST OF FIGURES

Figure 1-1: Location Map of Challie Watershed	2

Figure 2-1: Complex Hydrograph	22

Figure 2-2: River bed profile	24

Figure 4-1: Typical Cross Section of main and secondary canals	52

Figure 4-2: Typical Division Box plan	60

Figure 4-3: Typical chute section	63

Figure 4-4: Typical drop structure section	65

# SALIENT FEATURE

- Project name: Challie Diversion Small-Scale Irrigation Project

- Location

- North: 1263478.75

- East : 598131.06

- Average Altitude: 1366.62masl

- Zone: (South Wollo)

- Wereda: (WerebaboWereda)

- Hydrology

- Design rainfall: 91.12 mm

- Catchment area: 44.2 Km2

- Longest flow path length: 13.26 Km

- Design flood: 248.52 m3/sec

- Design base flow: 310 lit/se.

- Haik metrological station with 55 years Number of data

- Head work /Diversion Weir

- Type: Broad Crested Cyclopean Concrete Weir

- Height: 1.50m

- Gross crest length: 24.40m

- Weir crest level: 1368.12 m.a.s.l

- U/S TEL: 1372.67 m.a.s.l

- HFL: 1369.90m.a.s.l

- D/s TEL: 1369.98 m.a.s.l.

- D/s HFL: 1369.0 m.a.s.l

- Afflux: 0.90 m

- Under sluice

- Sill level: 1366.82 m.a.s.l

- Dimension: 1.10*1.52 m2

- Discharge amount: 5.65 m3/sec

- Outlet

- Sill level: 1367.12 m.a.s.l

- Opening dimension: 0.6*0.8 m2

- Discharge capacity: 280 lit/sec.

- Irrigation system Infrastructure

- Command area size: 165 ha

- Type of soil of the command area is dominantly clay loam soil

- Design discharge of the main canal = 280 l/sec

- Irrigation system layout consists of one lined main canal, one lined secondary canal, 27earthen tertiary canal and 72 field canals.

- Main irrigation structures designed are;

- Gully crossing structures(flumes & super passage)

- Foot path

- Culvert

- Summary of project construction costs

Table 0-1 : Summary of Project Construction Costs

| Challi Irrigation Project Summary of Bills |  |  |
| --- | --- | --- |
| Bill No | Description | Cost |
| I | General items | 2,909,110.46 |
| II | Head work | 3,500,656.29 |
| III | Infrastructure | 13,713,323.79 |
|  | Total | 20,123,090.54 |
|  | 15 % VAT | 3,018,463.58 |
|  | Grand Total | 23,141,554.12 |
|  | Command area, ha | 163 |
|  | Cost per ha | 123,454.54 |

# INTRODUCTION

## General

Development of Irrigation contributes a valuable share in the production of agricultural products which enhances the removal of food security problem & leads the economic growth of a country where there are sufficient water resources for this purpose.

Different study document shows, in our country, there are sufficient resources of water that can be utilized for various purposes. But only few percent of existing water resources is underutilization. On other side there are a number of chronically food insecure families. This may be due to lack of good attitude, skill & budget. Nowadays, the degrees of these problems are slightly in the stage of minimizing as per activities of past indicate because of attention given to   irrigation practices by Ethiopian government & other concerned bodies.

Similarly, Challie diversion irrigation project study was executed to increase the agricultural product, its operation managed by farmers of the project area. This document contains study & design of Challie project. It is a Medium scale project. The project is located Amhara regional state in South Wollo zone, in Werebabo Wereda.

At the project area, agriculture is the main pillar of the economy, which depends on the annual crops. For the production of annual crops there is insufficient & non- uniform rainfall distribution for the production of annual crops at this project area. Therefore, the study found out that the existence of the promise of gathering additional agricultural product by irrigation. Again, supplementing yearly crops by irrigation can be undertaken. Hence, it is possible gather product twice & more per year. As per this study, executing the irrigation practices during dry & wet seasons, the livelihood of population of the project can be improved due to the registered economic growth as a result of agriculture product increment. On average Challie diversion irrigation project can give benefits on 165ha of command area.

This design document comprises of three sections.  Section one contains Hydrology study ,Section two deals with head work structure design and section three about infrastructure.

## Location and accessibility

This irrigation project is located mainly at 012 Kebele, WerebaboWereda of SouthWollo Zone in the Amhara Region. The proposed irrigation project is to be undertaken on Challie River and the headwork structures are specifically located at an altitude of about 1366.62 masl and geographical coordinates of 1263478.75N (UTM) and 598131.06E (UTM).

#### Accessibility

The project site is along the Dessie- Haik-Chifra road way. The project site is 50Km far from Haik town in the East direction. From this 47 Km is accessible by URAP road and for the left 3kms along Challie River.

Figure 1-1: Location Map of Challie Watershed

## Previous Irrigation Practices

The existing low productivity and production capacity of rain-fed crops, due to uneven distribution of rainfall and heavy hail damage, has enforced farmers to develop traditional irrigation practices. Therefore, irrigated agriculture has gradually developing following rainfall abnormalities and scarcity of land. In the project area irrigated crop production has been practicing using water from Challie River by constructing traditional diversion structures and fetching water from reservoirs with plastic water can. Furthermore, farmers in the project area have constructed small water wells to use as a source of water for irrigation. Thus, the project area is trying to practice two seasons’ crop productions. The first is rain-fed crop production which is the main income base and constitutes more than 90% of the annual cultivated land; and the second is irrigated crop production (traditional& modern) that accounts about 10% of the whole cultivated land area.

## Objectives of the Study

The main objective of the study is to conduct feasibility and detail study of this irrigation project so as to utilize water resource efficiently. Moreover the project carried out has the following main objectives;

- Efficient utilization of the resources

- To minimize land degradation due to poor drainage system of the area.

- To promote easy & manageable irrigation operation system for the area.

- To mitigate problems of food shortage through irrigation development.

- To increase agricultural products through irrigation in dry and wet seasons.

The overall objectives of Challie diversion irrigation development project is to make farmers to be full benefited from Challie river water resource, thereby to ensure sustainable food security, increased crop production and to increase income and sustainable livelihood to the community of the project area.

## Scope of the Study

This section of study of the project mainly focuses on the identification of Water resource, head work site selection, checking, point rainfall analysis , design flood determination, head work structure design and cost of the head work structure with the inclusion of the general item.

## Methodology of study

The methodology employed for the study is the conventional engineering study with integrated multi-disciplinary approaches. Among These;

- Collecting relevant information and data useful for the study.

- Conduct fieldwork to collect all data required for the study in a multi –disciplinary manner that include field investigation and surveying works.

- Conduct data analysis collected in the field by all disciplines participated in the study.

- Conduct the design and study work at office level using all inputs

# HYDROLOGY

In the study of project, hydrological events, such as occurrence of peak floods & base flows are very important in the designing of the irrigation structure. On one way, these events determine the sustainability of the irrigation project structures, which means structures that can withstand these events & service for intended purpose within the project life has to be adopted. On another way, hydrological events enable us to design projects structures economically. Hence, the occurrence peak flood within the intended return period and low flows will be analyzed & interpreted properly in section.

## Hydro-Metrological Data Availability

### Climate

Small scale irrigation project designers and planners are faced with lack of good data on the hydrology of the stream/river system that will be their water source and on local weather and climate conditions. Stream gauging stations are virtually non-existent in remote rural areas of Ethiopia; meteorological stations are almost rare. Likewise, at 012 Kebele (Project area location) and in the catchment area of this project, there is no meteorological station of any level. Moreover, there are no flow data for the river/stream near the project. Therefore, data for the hydro-meteorological analysis is taken from the nearby station and similar areas. Rainfall data are considered from Haik meteorological station. In fact, this station is relatively close to the project area.

### Rainfall Data

In order to compute the design flood for the Weir structure, the daily maximum rainfall is collected from Haik Metrological stations with a record of 55 years.

### River flow data

The base flow which is measured on 07/10/08E.C is 310 l/s. Since this base flow is measured during the driest months of the year it is taken as the design base flow of the river.

### Upstream& Downstream utilization

Challie River is perennial. However, this steam flows under river bed materials, i.e., it is totally converted in to subsurface flow after about 1.2km  the downstream of the proposed headwork site and no one can fetch water from it. The existing traditional irrigation practices are located on the side of the selected canal alignment; thus, those farm plots will be supplied by the new project. Since it is mandatory to release at least 10% of the river flow for downstream ecological demand it has been designed to divert only 280 l/s for irrigating 165 ha of command area with a design duty of 1.43 l/s/ha. In addition since the irrigation time is 14 hours only due to the climate condition of the locality the river is allowed to flow in its course for about 10 hours every day. This means about 40% of the day the steam flows in its course and keeps the environment wet. Moreover, the irrigation water drained from the command area also joins the river in order to enhance the flow. Due to the above reasons, the total abstraction of base flow from Challie River will not have a pronounced effect on the downstream users.

## Design Rainfall computation

Based on the data of 24hr peak rainfall given in Table 2.1 the design rainfall is computed using Gumble’s Extreme Value Method.

Table 2-1: Haik metrological station maximum daily rainfall data

| 1 | Hydroclimatic Data Availability and Its Quality |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| S.No. | Year | Daily max. RF | Descending Order | Rank | Logarithmic Value/Yo/ | (Yo-Ym)2 | (Yo-Ym)3 |
| 1 | 1954 | 41.000 | 104.200 | 1.000 | 2.018 | 0.0792261 | 0.0222999 |
| 2 | 1955 | 60.000 | 91.700 | 2.000 | 1.962 | 0.0510638 | 0.0115390 |
| 3 | 1956 | 46.000 | 85.400 | 3.000 | 1.931 | 0.0380490 | 0.0074219 |
| 4 | 1957 | 43.000 | 81.500 | 4.000 | 1.911 | 0.0305415 | 0.0053375 |
| 5 | 1958 | 56.000 | 77.800 | 5.000 | 1.891 | 0.0238960 | 0.0036939 |
| 6 | 1959 | 50.000 | 76.000 | 6.000 | 1.881 | 0.0208563 | 0.0030120 |
| 7 | 1960 | 41.000 | 72.000 | 7.000 | 1.857 | 0.0146255 | 0.0017688 |
| 8 | 1961 | 48.000 | 71.900 | 8.000 | 1.857 | 0.0144799 | 0.0017424 |
| 9 | 1962 | 30.000 | 71.600 | 9.000 | 1.855 | 0.0140462 | 0.0016647 |
| 10 | 1963 | 56.000 | 70.700 | 10.000 | 1.849 | 0.0127742 | 0.0014438 |
| 11 | 1964 | 77.800 | 70.300 | 11.000 | 1.847 | 0.0122233 | 0.0013514 |
| 12 | 1965 | 49.400 | 67.400 | 12.000 | 1.829 | 0.0085126 | 0.0007854 |
| 13 | 1966 | 70.700 | 66.500 | 13.000 | 1.823 | 0.0074693 | 0.0006455 |
| 14 | 1967 | 50.000 | 65.200 | 14.000 | 1.814 | 0.0060608 | 0.0004718 |
| 15 | 1968 | 85.400 | 64.000 | 15.000 | 1.806 | 0.0048698 | 0.0003398 |
| 16 | 1969 | 56.500 | 62.700 | 16.000 | 1.797 | 0.0037053 | 0.0002255 |
| 17 | 1970 | 47.000 | 62.600 | 17.000 | 1.797 | 0.0036214 | 0.0002179 |
| 18 | 1971 | 42.300 | 61.600 | 18.000 | 1.790 | 0.0028286 | 0.0001504 |
| 19 | 1973 | 40.800 | 60.700 | 19.000 | 1.783 | 0.0021895 | 0.0001025 |
| 20 | 1974 | 42.500 | 60.600 | 20.000 | 1.782 | 0.0021230 | 0.0000978 |
| 21 | 1975 | 65.200 | 60.000 | 21.000 | 1.778 | 0.0017435 | 0.0000728 |
| 22 | 1976 | 39.700 | 59.400 | 22.000 | 1.774 | 0.0013980 | 0.0000523 |
| 23 | 1977 | 44.300 | 58.800 | 23.000 | 1.769 | 0.0010877 | 0.0000359 |
| 24 | 1979 | 46.300 | 56.700 | 24.000 | 1.754 | 0.0002954 | 0.0000051 |
| 25 | 1980 | 40.600 | 56.500 | 25.000 | 1.752 | 0.0002450 | 0.0000038 |
| 26 | 1981 | 60.600 | 56.500 | 26.000 | 1.752 | 0.0002450 | 0.0000038 |
| 27 | 1982 | 44.800 | 56.000 | 27.000 | 1.748 | 0.0001390 | 0.0000016 |
| 28 | 1983 | 60.700 | 56.000 | 28.000 | 1.748 | 0.0001390 | 0.0000016 |
| 29 | 1984 | 71.900 | 52.600 | 29.000 | 1.721 | 0.0002375 | -0.0000037 |
| 30 | 1985 | 72.000 | 50.000 | 30.000 | 1.699 | 0.0014007 | -0.0000524 |
| 31 | 1986 | 44.200 | 50.000 | 31.000 | 1.699 | 0.0014007 | -0.0000524 |
| 32 | 1987 | 81.500 | 49.400 | 32.000 | 1.694 | 0.0018207 | -0.0000777 |
| 33 | 1988 | 62.600 | 48.400 | 33.000 | 1.685 | 0.0026575 | -0.0001370 |
| 34 | 1989 | 42.800 | 48.000 | 34.000 | 1.681 | 0.0030421 | -0.0001678 |
| 35 | 1990 | 67.400 | 47.000 | 35.000 | 1.672 | 0.0041343 | -0.0002658 |
| 36 | 1991 | 56.500 | 47.000 | 36.000 | 1.672 | 0.0041343 | -0.0002658 |
| 37 | 1992 | 76.000 | 46.300 | 37.000 | 1.666 | 0.0050148 | -0.0003551 |
| 38 | 1993 | 62.700 | 46.000 | 38.000 | 1.663 | 0.0054226 | -0.0003993 |
| 39 | 1994 | 61.600 | 44.800 | 39.000 | 1.651 | 0.0072451 | -0.0006167 |
| 40 | 1995 | 52.600 | 44.300 | 40.000 | 1.646 | 0.0080987 | -0.0007288 |
| 41 | 1996 | 48.400 | 44.200 | 41.000 | 1.645 | 0.0082763 | -0.0007529 |
| 42 | 1997 | 47.000 | 43.900 | 42.000 | 1.642 | 0.0088232 | -0.0008288 |
| 43 | 1998 | 71.600 | 43.200 | 43.000 | 1.635 | 0.0101834 | -0.0010276 |
| 44 | 1999 | 43.200 | 43.000 | 44.000 | 1.633 | 0.0105942 | -0.0010904 |
| 45 | 2000 | 56.700 | 42.800 | 45.000 | 1.631 | 0.0110150 | -0.0011561 |
| 46 | 2001 | 104.200 | 42.500 | 46.000 | 1.628 | 0.0116656 | -0.0012600 |
| 47 | 2002 | 58.800 | 42.300 | 47.000 | 1.626 | 0.0121123 | -0.0013330 |
| 48 | 2003 | 43.900 | 42.100 | 48.000 | 1.624 | 0.0125696 | -0.0014092 |
| 49 | 2004 | 70.300 | 41.000 | 49.000 | 1.613 | 0.0152801 | -0.0018888 |
| 50 | 2005 | 59.400 | 41.000 | 50.000 | 1.613 | 0.0152801 | -0.0018888 |
| 51 | 2006 | 64.000 | 40.800 | 51.000 | 1.611 | 0.0158096 | -0.0019878 |
| 52 | 2007 | 40.600 | 40.600 | 52.000 | 1.609 | 0.0165508 | -0.0020908 |
| 53 | 2008 | 66.500 | 40.600 | 53.000 | 1.609 | 0.0165508 | -0.0020908 |
| 54 | 2009 | 91.700 | 39.700 | 54.000 | 1.599 | 0.0189354 | -0.0026056 |
| 55 | 2010 | 42.100 | 30.000 | 55.000 | 1.477 | 0.0672236 | -0.0174294 |
| SUM |  |  | 3095.800 |  | 95.502 | 0.6535337 | 0.0225263 |
| MEAN |  |  | 56.287 |  | 1.736 | 0.0119 | 0.0004 |
| STANDARD DEVATION |  |  | 14.947 |  | 0.110 | 0.0155 | 0.0045 |
| SKEWNESS COEFICIENT |  |  | 0.951 |  | 0.325 | 2.784 | 1.407 |

### Checking the existence of outliers

Outliers are data values which are quite different from the others in the data series. They exists both in extreme values i.e. there is lowest & largest values out of limits formulated. The Causes of the outliers may be errors of measurement or recording and etc.  Since outliers have a significant effect on the results of the design rainfall computation, they have been discarded from the data set.  As per U.S Water resource council/1981/ recommended that, it is possible to check for outliers based on skew value. If value of skew is greater than +4 & -4, outlier has to be checked for highest & lowest value respectively. But if value of skew lies between +4 & -4 the outliers are checked for both lowest & highest values.

In case of Haik maximum daily rainfall data values shown on table-2.1, the skew /g/=0.951,which is,-4<0.0.951<+4. Hence, the data series has be checked for both cases of extremes.

Let,y=logx, where x rainfall data series value

Lowest data value,RL=10YL,YL=Ym-kn*Syn-1

Highest data value,RH=10YH,whereYH=Ym+kn*Syn-1

Ym=log arithmetic mean value=1.74

Syn-1=standard deviation of log value=0.11

Kn =2.8

YL=1.43 & YH=2.04

Then, the lowest data RL=26.79mm/day & the highest data RH=110.89mm/day. But, the lowest & highest value in the given data series is 30 &104.2 respectively. So, all of the data values lies between the lowest & a highest calculated value (26.79 &110.89), Hence, there is no any data that can be rejected.

### Checking reliability & Adequacy

Standard deviation of the data series from the mean value is an indicator of the variation of data series. Variation is the inverse function of the number of the data. The variation is also the reflection

Standard deviation of the data series from the mean value is an indicator of the variation of data series. Variation is the inverse function of the number of the data. The variation is also the reflection Standard deviation of the data series from the mean value is an indicator of the variation of data series. Variation is the inverse function of the number of the data. The variation is also the reflection of the errors in the series. The data series could be considered reliable & adequate if the percent of errors is less than or equal to ten percent. Percent of errors is given by:-

P =stdv (αn-1)/ (SQRT (N)* µ) *100

P=3.58% < 10. The percent of errors is acceptable. Hence, the data series is adequate & reliable.

### Design Point Rainfall

In this study, design rainfall is to mean the amount of 24 hours rainfall that is computed from the available data series based on the given return periods recommended at the rain gauge point. Gamble puwell is the most widely used method for the prediction of annual extreme values like annual rain falls, flood flows and the like for the desired return period. This method is also adopted to estimate the design rainfall for this particular study. Its formula is given by:-

XT=µ+KT*σn-1                    Where KT=YT-YnSn

Yn = Reduced mean in Gamble’s extreme value distribution for N sample size from table;

Sn= Reduced standard deviation in Gumble’s extreme value distribution for N sample size from table;

T=Return period, for weir diversion of 50 years return periods;

σn-1   = Standard deviation of annual rain fall; and

µ = Mean of all values annual rain fall.

For sample size N=55, Yn =0.5504, and Sn =1.1681

YT=-(ln (ln(T(T-1)))) =(ln (ln (5050-1)))=3.9

KT=YT-YnSn=3.9-0.55041.1681=2.87

X50=µ+KT*σn-1=56.29+2.87*14.95=91.12mm

- Point Design Rainfall = 95.12 mm

The design rainfall at points for 50 years return period is 91.12 mm and the areal design rainfall is calculated in the following section.

## Design Flood Analysis

For the design and analysis of structures to be constructed on the river/stream, estimation of flood magnitude is an important task. This can be done using different techniques depending on the data available. For this particular case, the flood estimation is done by using Flood Mark method and SCS Curve No Method.

### Peak flood analysis by SCS-CN method

For drainage basins where no runoff has been measured, the Curve Number Method can be used to estimate the depth of direct runoff from the rainfall depth, given an index describing runoff response characteristics. This method is widely adopted and simple method for runoff estimation. The approach considers, watershed parameters, like Area, Curve number, and time of concentration.

#### Catchments characteristics

Catchment  characteristics is the most determinant factor in the production  water towards the point of interest called head work position, which is considered as outlet of this catchment. The catchment has an area of 4420.38 hectare. And longest stream in the catchment has a length of 13.26km. 75.54% of catchment area is shrub land and 19.94% is cultivated land. The rest parts of the catchment are grass land, Bare land, grass land & plantation area as shown in the table b

Table 2-2: Parts of catchment area

| S/N | Major Land Cover | Area (ha) | Cover (%) |
| --- | --- | --- | --- |
| 1 | Bare land | 74.81 | 1.69 |
| 2 | Cultivation | 881.41 | 19.94 |
| 3 | Grassland | 49.05 | 1.11 |
| 4 | Plantation | 76 | 1.72 |
| 5 | Shrub land | 3339.11 | 75.54 |
|  | Total | 4420.38 | 100.0 |

#### Time of concentration (Tc)

The time of concentration is defined as the time for runoff to travel from the hydraulically most distant point in the drainage basin to the outlet or point of interest; it is also defined as the distance between the end of excess rainfall and the inflection point in the recession limb of the dimensionless curvilinear unit hydrograph. Time of concentration has been calculated by taking the stream profile of the longest streamline and dividing it in to different elevation. Kirpich formula is adopted for computation.

The formula is,

Tc=0.948L13H10.385+L23H20.385+…+Ln3Hn0.385

Table 2-3: Determination of Time of Concentration

| Partial distance, L (km) | Commutative distance (km) | Elevation (m) | Elevation diff (m) | Tc (hr) |
| --- | --- | --- | --- | --- |
| 0 | 0 | 2420 | 0 |  |
| 3.03 | 3.03 | 1920 | 500 | 0.32 |
| 2.69 | 5.72 | 1620 | 300 | 0.34 |
| 4.71 | 10.43 | 1460 | 160 | 0.83 |
| 2.83 | 13.26 | 1380 | 80 | 0.61 |
|  |  | Total Tc (hr.) |  | 2.10 |

- Tc = 2.10hr,	Since Tc< 3hr., duration of excess rainfall difference, D = TC/6

D=2.11/6=0.35hr

- Time to peak

- . Tp=D2+0.6*Tc, where Tc=time of concentration

⇒Tp=0.352+0.6*2.11=1.44hours

- Base time

Tb=2.67*Tp

⇒Tb=2.67*1.44=3.83hours

- Lag time, te

te=0.6*TC=0.6*2.11=1.27hr  =

- q=0.21*A*QTp  Where, A=area of the catchments in square km

Tp=time of peak in hour

⇒  Qp=0.21*A*QTp=0.21*44.2*11.44=6.45m3/s/mm

#### Curve number (CN)

The Curve Number is a dimensionless parameter indicating the runoff response characteristic of a drainage basin. In the Curve Number Method, this parameter is related to land use, land treatment, hydrological condition, hydrological soil group, and antecedent soil moisture condition in the drainage basin.

From the watershed analysis curve number at condition II =89.36 since peak rainfall is found at an antecedent moisture condition III state, this value has to be changed to antecedent moisture condition III.

- Conversion factor = 1.074

- CN Condition (III) = (Factor from Table x CN condition II) =89.36* 1.074= 95.97. For detail analysis of the computation, Refer Excel file, attached here with.

#### Rainfall profile and Areal Rain fall distribution

Rain fall profile is the distribution of design rain fall with respect to time in the whole watershed area. It needs developed models for the selected drainage area. If there is no sufficient modeling data in the vicinity, standard curves may be adopted. Similarly, as the area of the watershed gets larger, coincidence of all hydrological incidences becomes less and less. This can be optimized by changing the calculated point rainfall to areal rainfall. The conversion factor is taken from standard table that relate directly with the size of watershed area and type of the gauging station. The Rainfall profile & areal to point ratio are obtained from the IDD manual graphs and tables respectively.

Table 2-4: The Rainfall profile & areal to point ratio

| Duration ,D (hour) | Design point rainfall | Rain Fall profile (%) | Rain Fall profile (mm) | Areal to point rainfall(%) | Areal rainfall (mm) | Rainfall increment(mm) | dscending order | Rearranged order | Re -arranged rainfall(mm) | Comm. Rainfall,P (mm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0-1 | 91.12 | 45 | 41.0018 | 54.11 | 22.186 | 22.186 | 22.186 | 6 | 3.441 | 3.441 |
| 1-2 |  | 58 | 52.8468 | 65.11 | 34.409 | 12.222 | 12.222 | 4 | 6.385 | 9.826 |
| 2-3 |  | 66 | 60.1360 | 70.11 | 42.161 | 7.753 | 7.753 | 3 | 7.753 | 17.578 |
| 3-4 |  | 72 | 65.6029 | 74.00 | 48.546 | 6.385 | 6.385 | 1 | 22.186 | 39.765 |
| 4-5 |  | 76 | 69.2475 | 76.11 | 52.704 | 4.158 | 4.158 | 2 | 12.222 | 51.987 |
| 5-6 |  | 79 | 71.9810 | 78.00 | 56.145 | 3.441 | 3.441 | 5 | 4.158 | 56.145 |

#### Run off Analysis

Input data are design point rainfall with rainfall profile and areal distribution conversion factors, Curve Number (CN), Time of concentration (Tc), Time to peak (Tp) and lag time (te) and watershed area.

Peak rate of discharge created by 1mm runoff excess on whole of the catchment is,

- Where,  q p:     Peak runoff rate (m3/S)

- A:	watershed area (km2)

- Q:	runoff volume (mm) = area under the hydrograph

- T P:	time of peak (hrs.)

Direct surface runoff Q may be obtained from the standard SCS-CN formula.

Where, Q:  direct surface runoff in depth (mm)

I:    Storm rainfall (mm)

S:   Potential retention capacity of the watershed (mm),

- Design Point Rainfall = 91.12mm

- Curve number at antecedent moisture condition III = 95.97

- Catchment Area, A = 44.2 Km2

- Tc=2.10hr, D = 0.35hr., Tp =1.44 hr.; Tb = 3.83.5hr; te = 1.27hr.

Table 2-5: Runoff analysis

| Rainfall Duration(D) | Direct runoff increment, Ri(mm) | qp for 1mm runoff /m3/s/mm | qp for incremental run off m3/s/ | begin time, hour | peak time, hour | end time, hour | remarks | TC | Time to peak, Tp | Total time of flow,Tb |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0-1 | 0.00 | 6.5 | 0.00 | 0 | 1.44 | 3.83 |  | 2.10 | 1.44 | 3.8 |
| 1-2 | 3.23 | 6.5 | 20.86 | 1 | 2.44 | 4.83 |  |  |  |  |
| 2-3 | 5.91 | 6.5 | 38.3 | 2 | 3.44 | 5.83 |  |  |  |  |
| 3-4 | 20.19 | 6.5 | 130.6 | 3 | 4.44 | 6.83 |  |  |  |  |
| 4-5 | 11.75 | 6.5 | 76.0 | 4 | 5.44 | 7.83 |  |  |  |  |
| 5-6 | 4.04 | 6.5 | 26.1 | 5 | 6.44 | 8.83 |  |  |  |  |

Table 2-6: Hydrograph coordinates

| Time | Q1 | Q2 | Q3 | Q4 | Q5 | Q6 | Base Flow(m^3/se) | Q_Total |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0.00 | 0.00 |  |  |  |  |  | 0.3 | 0.30 |
| 1.00 | 0.00 | 0.00 |  |  |  |  | 0.3 | 0.30 |
| 2.00 | 0.00 | 5.22 | 0.00 |  |  |  | 0.3 | 5.52 |
| 3.00 | 0.00 | 10.43 | 7.65 | 0.00 |  |  | 0.3 | 18.38 |
| 3.00 | 0.00 | 10.43 | 7.65 | 0.00 |  |  | 0.3 | 18.38 |
| 4.00 | 0.00 | 15.65 | 15.30 | 26.11 | 0.00 |  | 0.3 | 57.36 |
| 5.00 | 0.00 | 20.86 | 22.95 | 52.23 | 15.20 |  | 0.3 | 111.54 |
| 6.00 | 0.00 | 17.88 | 30.60 | 78.34 | 30.39 | 0.000 | 0.3 | 157.53 |
| 7.00 | 0.00 | 14.90 | 38.26 | 104.46 | 45.59 | 6.529 | 0.3 | 210.04 |
| 8.00 | 0.00 | 11.92 | 31.88 | 130.57 | 60.79 | 13.058 | 0.3 | 248.52 |
| 9.00 | 0.00 | 8.94 | 25.50 | 108.81 | 75.98 | 19.586 | 0.3 | 239.13 |
| 10.00 | 0.00 | 5.96 | 19.13 | 87.05 | 63.32 | 26.115 | 0.3 | 201.87 |
| 11.00 | 0.00 | 2.98 | 12.75 | 65.29 | 50.66 | 21.76 | 0.3 | 153.74 |
| 12.00 |  | 0.00 | 6.38 | 43.52 | 37.99 | 17.41 | 0.3 | 105.60 |
| 13.00 |  |  | 0.00 | 21.76 | 25.33 | 13.06 | 0.3 | 60.45 |
| 14.00 |  |  |  | 0.00 | 12.66 | 8.71 | 0.3 | 21.67 |
| 15.00 |  |  |  |  | 0.00 | 4.35 | 0.3 | 4.65 |
| 16.00 |  |  |  |  |  | 0.00 | 0.3 | 0.30 |

Figure 2-1: Complex Hydrograph

- From the above analysis, the 50 year return period design run off is 248.52 m 3/s.

### Peak flood analysis by Flood mark method

Flood mark method is also one of maximum flood estimation technique by tracing the high flood level of a drainage area using local elder’s information and looking river transported debris over the river bank. For this project, the flood level of the catchment is 1372m which is around 50m U/S of the headwork site. And the river profile along the river bed is collected during the study of the project.

- Slope of river(S); - it is the longitudinal slope of river/stream at headwork position. Best fit method is used to determine this slope.

Table 2-7: Actual Surveying data taken along the river center

| S.N | Northing | Easting | Elevation | Patial Height, (H), m | Partial Distance(L) m | Comulative Distance, (m) | Area (m²) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1263446.24 | 598093.22 | 1367.87 | 0 | 0 | 0 | 0 |  |
| 2 | 1263458.34 | 598101.29 | 1367.72 | 0.15 | 14.54 | 14.54 | 1.09 |  |
| 3 | 1263464.64 | 598108.86 | 1367.40 | 0.32 | 9.85 | 24.39 | 2.31 |  |
| 4 | 1263469.68 | 598118.19 | 1367.09 | 0.31 | 10.60 | 35.00 | 3.34 |  |
| 5 | 1263473.20 | 598121.72 | 1366.92 | 0.17 | 4.99 | 39.98 | 1.20 |  |
| 6 | 1263478.75 | 598131.06 | 1366.62 | 0.30 | 10.86 | 50.85 | 2.55 |  |
| 7 | 1263485.05 | 598145.69 | 1366.25 | 0.37 | 15.93 | 66.78 | 5.34 |  |
| 8 | 1263495.13 | 598165.35 | 1366.14 | 0.11 | 20.33 | 87.11 | 4.88 |  |
| 9 | 1263498.40 | 598179.25 | 1365.79 | 0.35 | 16.23 | 103.34 | 3.73 |  |
| Summation |  |  |  | 2.08 | 103.343 |  | 24.445 |  |
| Average Height , H = |  |  |  | 2*A/L |  | 0.473 |  |  |
| Average Slope , S = |  |  |  | H/L |  | 0.005 |  |  |
| Adopted Slope of the river at the head work site, S = |  |  |  | 1:200 |  |  |  |  |

Figure 2-2: River bed profile

From the above graph, Slope=0.0259m/m.

- Manning’s roughness coefficient (n); the river has vegetated banks & curvature. Based on this, n=0.04 is adopted.

- Area (A) & Perimeter (P):- the area & perimeter of the river cross section at head work site, was computed on the Auto cad software.

Table 2-8: Area, Perimeter and Discharge of the river using flood mark method

| S.Nr | Elevation/m/ | Water depth /m/ | Wetted Area (A), m^2 | Perimeter /m/ | Hydrualic radius/R/ | Velocity /m/s/ | Discharge /m^3/s/ |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1367.87 | 0 | 0.00 | 0.00 | 0 | 0 | 0 |
| 2 | 1367.72 | 0.15 | 4.35 | 6.59 | 0.66 | 1.282 | 5.58 |
| 3 | 1367.40 | 0.32 | 11.42 | 12.78 | 0.89 | 1.569 | 17.92 |
| 4 | 1367.09 | 0.31 | 21.52 | 19.49 | 1.10 | 1.807 | 38.89 |
| 5 | 1366.92 | 0.17 | 32.68 | 26.18 | 1.25 | 1.961 | 64.09 |
| 6 | 1366.62 | 0.30 | 48.79 | 34.86 | 1.40 | 2.116 | 103.26 |
| 7 | 1366.48 | 0.35 | 65.29 | 36.58 | 2.10 | 2.53 | 241.65 |
| 8 | 1366.25 | 0.37 | 87.65 | 39.52 | 2.22 | 2.877 | 252.14 |
| 9 | 1366.14 | 0.11 | 94.82 | 45.69 | 2.08 | 2.752 | 260.97 |
| 10 | 1365.79 | 0.35 | 113.10 | 49.84 | 2.27 | 2.921 | 330.35 |

From the table above, the discharge corresponding to the flood mark elevation (1366.48m) is 241.65m3/s.

But, the discharge computed by SCS curve No method is 248.52m3/s which is larger than the flood mark method. To minimize the risk we take the larger value (248.52m3/s).

## Tail water depth

Tail water depth is a flow depth of peak flood in the stream channel during the occurrence of design flood, particularly at the point of interest. In this case of study, head work site is the point of interest. This depth is obtained by Manning’s empirical formula. To use this formula properties river foundation, stream cross section area & its longitudinal slope has be determined first.

- The slope and manning’s roughness coefficient are computed above.

S=0.0259m/m and n=0.04

- Area (A) & Perimeter (P):- the area & perimeter of the river cross section at head work site, was computed on the Auto cad software.

Table 2-9: Tail water depth and corresponding discharge at the head work site

| S.Nr | Elevation/m/ | Tail Water depth /m/ | Wetted Area (A), m^2 | Perimeter /m/ | Hydrualic radius/R/ | Velocity /m/s/ | Discharge /m^3/s/ |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1367.87 | 1.25 | 0.00 | 0.00 | 0 | 0 | 0.00 |
| 2 | 1367.72 | 1.10 | 7.50 | 10.23 | 0.73 | 1.375 | 10.31 |
| 3 | 1367.40 | 0.78 | 9.85 | 16.81 | 0.59 | 1.184 | 11.67 |
| 4 | 1367.09 | 0.47 | 16.54 | 17.92 | 0.92 | 1.604 | 26.52 |
| 5 | 1366.92 | 0.30 | 45.63 | 20.32 | 2.25 | 2.901 | 132.35 |
| 6 | 1366.62 | 0.00 | 52.00 | 23.65 | 2.20 | 2.860 | 148.73 |
| 7 | 1368.75 | 2.13 | 63.25 | 24.84 | 2.55 | 3.154 | 199.50 |
| 9 | 1369.00 | 2.38 | 89.76 | 27.95 | 3.21 | 3.682 | 330.49 |
| 10 | 1370.35 | 3.73 | 91.35 | 31.58 | 2.89 | 3.434 | 313.70 |
| 11 | 1371.45 | 4.83 | 95.64 | 37.42 | 2.56 | 3.162 | 302.41 |
|  | BY Interpolation | elevation | depth | Discharge(M3) |  |  |  |
|  |  | 1368.75 | 2.1 | 199.50 |  |  |  |
|  |  | 1368.84 | 2.2 | 248.52 |  |  |  |
|  |  | 1369.00 | 2.4 | 330.49 |  |  |  |

As shown on the table above, by interpolation, the tail water depth that corresponds to peak flood, 248.52m3/s discharge is 2.2m from river bed. The HFL of the river at the head work site is 1369.90m.a.s.l.

The HFL of the river is not only checked at the headwork site, but also at different chain ages at D/S of the river. This is why, over topping is allowed b/c of the foundation nature of the river bank till the HFL becomes at lower elevation than the canal top bank level.

# HEAD WORK STRUCTURES DESIGN

## Head work site selection

The headwork is at this location because of the following reasons:-

- Due to the outside curvature it is an ideal site for Weir.

- It has enough head to convey the water to the command

- During low flood coming from the catchment, the river flows close to the left bank that allows water to divert to the Canal

- It is relatively close to the command area.

- It has good geological formation on the left side of the river bank.

## River Geomorphology

### River Bed condition

At the proposed headwork site the stream bed or course is well defined, meander, and shows rough surface due to recent sediment accumulations and undulating appearance of bedrock outcrops. Along the Weir axis, the bed is made up of two basically different geologic materials, as seen from surface observation. These are recently deposited alluvial coarse grained sediments and underling bedrock.

The central areas of the bed are totally covered with the alluvial deposits, and the left end of the bed is made up of slightly jointed basalt rock on contrary at the right end the bed covered with silt sand soil near the headwork axis. The bedrock is totally covered with the sediments at the central areas of the stream bed which is dominantly gravely sand soil. To know the thickness of the sediments or depth to bedrock, a test pit has been taken at 598089 E, 1263436 N and 1367m river bed elevation of the headwork axis. However, the excavation was only 0.5m, due to excessive subsurface water inflow to the pit; it has been interrupted before encountering the required bedrock. From surface geological understanding of the area, the sediment covered the stream bed area of river channel is estimated about 3.5m thickness at the center of stream bed. As described above, the foundation area of the headwork structure is characterized by non-uniform geologic materials of the stream bed; the sediments and bedrock. The former is loose and pervious, while the bedrock is strong and impervious. It is therefore better to incorporate a positive cut-off masonry wall at the central portion of the bed that anchored to the bedrock after intercepting the 4m thickness sediment layer. This will help for both seepage barrier and also stability conditions categorized into coarse sand.

### River Bank condition

Left Bank

At the proposed headwork axis and immediate vicinities, the left bank forms relatively steep slope topography. From surface observation, the bank is made up of one basic geological unit. The unit is slightly jointed basalt bedrock and it has about 3.5m thickness. This bedrock exposed on the surface at the bank foot. This rock formation exposed both along the downstream and upstream part of the bank. The bed rock continues for 15m of river upstream and also continues for about 300m downstream. It may not necessary to provide some bank protection structure because of the bank is protected by the bed rock itself.

Right Bank

At the headwork site or axis, the right bank is characterized by relatively moderate to gentle slope, having about 1m height from stream bed.  From visual observation of the natural cuts at the bank, there are two distinct geological materials forming the bank section. From top to bottom, these are:-

A. silt clay

B. Gravely silt clay

These two units have variable thickness or depth at the area. Just at the weir axis, the top gravely silt clay soil has about 0.7m, whereas the underlying is relatively compacted silt clay soil horizon. This bank section created soil materials have been affected by erosion/ flood under cutting which is widening the bank. However, it is protected by plantation and with a bed rock at a distant of about 30m upstream of this bank which is used to protect from erosion by minimize the flood force.

## Sources of construction materials

Rock Quarry

Quarry site that can be used for production of rock for masonry stone has been assessed during the field work session within the vicinity of the project area at economic distance for hauling. For this project two quarry sites were proposed. The first quarry site is within Project River. At this proposed quarry site there is fresh basalt which is river transported good quality rock that could be used as source area for masonry works. Since this fresh rock is not easily shapeable, and other optional quarry site is also taken.

The second quarry site has been identified as optional due to difference in engineering properties between the two types of rocks. This quarry site locates at about 7km to East direction of the headwork site which is towards the command area. Within this quarry site there is aplanatic basaltic rock, which is found naturally and easily shapeable rock. This rock quarry site is used for Bistima, the woreda town water supply construction during field study of this project.

Coarse Aggregates

Coarse aggregate is required for concrete construction during masonry channel construction and head work construction. There is excess gravel and cobble size rock fragments within the river so by only crushing and gridding it in to required size and required amount coarse aggregate to use them. Its source is the river so there is no owner ship and deficiency problem as well as transportation cost.

From this site, rock of different sizes can be produced; ranging from coarse gravels to large boulders that masonry stone and coarse aggregate can be produced easily.

Fine Aggregates

Borrow areas of fine aggregate or natural sand have been assessed starting from the project stream itself. A natural deposit of such materials is found when assessed within the beds of the stream in the project area; source of these materials are available within the project stream. In the stream good quantity, which suffices the need of this project, is available. The deposit dominated by sand sized transported alluvial sediments, though some amount of fines and over sizes are also observed. The individual grains of sand are mainly originated from basalt rock and reveals high crushing resistant that the sand has good quality in this respect. The sand has good strength as it mainly develops from basalt rock and experiences long stream transportation, which normally removes any weak zones of the grains. The river sand is taken to Amhara Design & supervision laboratory section as much as possible the result will be expected.

Canal Fill Material

Along the main canal route, if there are places where the original ground level is lower than the designed canal bed level (CBL). Fortunately, the canal need of more fill for the project because of the canal is passes across ragged topography. Therefore, for fill of different structures that constructed at the head work site and along the main canals will be needed. For such fill, natural fine grained soil material having low to medium plasticity, non-expansive and impervious is required. For this project such soil material is proposed near command area which is silt clay soil of grazing land and it is enough for fill of the structures cited above that is found through the canal route.

From visual examination of the material source, the soil has black to brown color, and composed of silt and clay. It is residual soil having low plasticity and low liquid limit. It is non-expansive soil. According to the Unified Soil Classification System, the soil group lies in Low plastic SILT (MH). Such soil type possess a maximum dry density (MDD) lies within 1.25 to 1.37gm/cc; on average 1.31gm/cc, and the associated optimum moisture content (OMC) of 33.1% to 39.5%; on average 36.3%. The construction of the fill can be then done using these parameters.

Water

Water for construction purposes can be getting from the project stream Challie River itself. Water for construction purposes can be getting from the project perennial river that flows throughout the year used for local irrigation purpose. During this field time the stream flow was about 170L/s.

## Design of head work structures

### Headwork Type Selection

At the selected head work site, it is possible to irrigate the command area by gravity, through only diverting water directly by raising the level of water considering reference to the highest point of the command area. Hence, broad crested weir structure has been recommended for Challie diversion small-scale irrigation project.

B/c of the foundation nature of the river bank along the canal alignment & impossibility of entry to the out let of headwork structure for closing & opening of the gate, overtopping is allowed till the HFL becomes at a lower elevation than the canal elevation. At the point where the HFL is at a lower elevation than the canal elevation (after 111.7m of canal length) two gates are provided. One gate for off take canal, and the other is for escape canal, which evacuates the canal water when the off take canal closes.

Generally, the headwork structure includes breast wall, operation slab, bed bar, trash rack, Guide walls& right side RC protection structures are the main components of the head work structures.  Right side RC protection structures are provided for the following reasons:

- The right side of the river bank at the headwork site is composed of silt clay and the top level bank is nearly at the same elevation with the river bed. So the right side of the river bank is scoured during flood flow and large scour holes may develop and also this bank lowers below the river bed elevation. Due to this, either the flow changes its flow direction (i.e. at the opposite direction of the off take canal) along this weak zone or the water seeps at this silt clay composition bank.

So, to stabilizes this river bank from possible lowering of below riverbed elevation and  to abstract water  which will seep to the off take this RC protection structure is provided as a maximum score depth i.e. downstream and upstream cut off depth.

Breast wall & operation slab have been provided to aid the operation of gate & to prevent over flow of river flood into the main canal. Similarly, trash rack is provided at the point of off take canal (i.e after 0.060m length of the former Weir or 0+00m of the new diversion site) to exclude boulders, trees &etc from entering into outlet pocket that clog the gate. To retain river bed from lowering and to abstract more water to the off take by acting as a cut off, cyclopean concrete under sluice is provided within river bed.

### Design of Off take

- Sill Level Determination

The sill level of off take out let is based on the required amount of water availability that can be easily diverted throughout the dry season.

- Capacity determination

The off take size is fixed taking in to consideration the maximum Duty of the command area.

Q = Max. Duty * factor of safety * Command area

=1.43*1.2*165ha=0.28m3/s

The regulator dimension is computed using rectangular notch formula given by:-

Where: Q= diverted discharge, m3/sec

Cd= Coefficient of discharge, Cd=0.6

B=Width of off take, 0.60 m (assumed)

H=Height of the Weir =flow depth above sill level

g=Acceleration due to gravity H=πr2

H= (Q*3/ (2*Cd*(2*g)1/2*B))2/3=0.80m.

Trash racks of diameter 14mm with c/c spacing of 10cm has to be provided u/s of the off take gate to prevent entry of debris to the canal.

### Design of Operation slab and Breast wall

A vertical raised gate is designed for the head regulator and under sluice. These gates slide over the breast wall using spindle during opening and closing.

For easy operation of these gates, operation slab is provided. The size of the operation slab is fixed from the point of construction and free movement. The size of the operation slab is shown in the drawing for both off take canal gate and escape canal gate with thickness 0.20m.

The thickness of the breast wall is also the same as that of the operation slab. The nominal thickness is fixed from the point of construction rather than the imposed load. The thickness required for the imposed load is less than the nominal value and treated as cantilever retaining wall. For the breast wall, the minimum reinforcement area is taken as 15% along the respective direction.

Hence	Asteel = 0.0015*1000*150=225

Asteel = 225 mm2/m

Provide 12@C/c 200 mm

Considering cover thickness of 50 mm, effective depth =

Effective depth, de=t-(50+/2) =150-(50+12/2) =106

Hence spacing of reinforcement should be less of the following values:

- Five times the effective depth =5*144=720 mm

- 450mm

The spacing of bars, which is 200 mm, is less of the above listed values. Hence, it is acceptable!

The actual arrangement of angle irons, spindles, shafts and operation slab including other components is shown on the design drawing.

For each arrangement and further information, refer to the design drawing.

### Guide Walls

These are, also called retaining walls, provided for the facility to the canal off take canal gate & escape canal gate operation and maintenance. They are designed as rectangular in shape and they are provided at three positions with short length. Their width and height is fixed based on the operation slab dimensions and the height of the gate respectively.

Guide wall data and Analysis

Table 3-1: Typical design of masonry Guide wall

|  | U/S Guide walls | D/S Guide walls |
| --- | --- | --- |
| Bed width of walls | 1.7 | 1.5 |
| height of walls above river bed level | 3.5 | 2.6 |
| foundation depth below RBL | 1.5 | 1.5 |

| Ka=(1 -sin Φ /1+sin Φ ) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Φ= for dry condition | 30 |  |  |  |  |  |  |
| Ka= | 0.33 |  |  |  |  |  |  |
| HEIGHT | Forces(KN) |  |  | Lever arm | Moment about toe(KN.m) |  |  |
| 3.5 | Vertical |  | Horizontal | (m.) | Stabilizing | Disturbing |  |
| WIDTH |  |  |  |  |  |  |  |
| 1.7 |  |  |  |  |  |  |  |
| self weight(W1) | 30.61 |  |  | 0.2 | 6.12 |  |  |
| self weight(W2) | 51.23 |  |  | 0.85 | 43.4 |  |  |
| lateral earth pressure(Pa) |  |  | 38.36 | 1.16 |  | 44.47 |  |
| earth load on wall (Ps) | 44.24 |  |  | 1.29 | 57.2 |  |  |
| Total | 126.08 |  | 38.36 |  | 106.7 | 44.47 |  |
| Factor of safety against, |  |  |  |  |  |  |  |
| Over turning = (M+ve/M-ve) >1.5 |  |  |  | Fo | 2.40 | >1.5 | safe |
| Sliding = (µxFv/Fh),---µ=0.75 >1.5 |  |  |  | Fs | 2.47 | >1.5 | safe |
| Tension: X= (Net Moment/Sum Fv), e=x-B/2, e<B/6 |  |  |  | X | 0.49 |  |  |
|  |  | B/6= | 0.29 | e | 0.20 | <B/6 | safe |

∑V = 126.08KN                                 ∑M (+) = 106.7KN.m

∑H=38.36KN	 ∑M (-) = 44.47KN.m

i) Factor of safety against overturning (Fo)

>1.5 →	Safe!

ii) Factor of safety against sliding (FS)

, µ=0.75

>1.5 → Safe!

iii) Check for tension (i.e. whether the resultant lies within the middle third)

The location of the resultant force from the toe is given by

The eccentricity (e) = │X – B/6│, B = 1.7m

Hence, e = │0.49-1.7/6│ = 0.2m

The eccentricity (e) should be less than B/6 =, Hence the obtained e = 0.2m < 0.29m.

⇒The resultant lies within the middle third                  no tension

Conclusion: From stability analysis, the designed wall section is over safe. But we can’t decrease the dimensions of the section to be economical because the dimensions are fixed based on the required dimensions of operation slabs.

## Bill of Quantity and Cost Estimation

It includes the estimation of the cost of material & activities to be accomplished.

Table 3-5.1: Bill of quantity and cost estimation of General items

Bill No I. General Item

| Bill No.1 General Items |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Item No | Description | Unit | Quantity | Rate Birr | Amount Birr |
| 1.1 | Allow for mobilization | L.S | 1 | 303,480.58 | 303,480.58 |
| 1.2 | Allow for demobilization | L.S | 1 | 273,132.52 | 273,132.52 |
| 1.3 | Allow for consultant's camp and facilities =4*5m2 , Living rooms & offices for supervision key personnel, CIS and Internally painted chip wood wall, Masonry floor cement screened and well ventilated room complete with doors and windows. | L.S | 2 | 56,454.94 | 112,909.88 |
| 1.4 | Allow for contrator's camp and facilities =4*5m2 , Living rooms & offices for construction key personnel, CIS and Internally painted chipwood wall, Masonry floor cement screened and well ventilated room complete with doors and windows. | No | 5 | 56,454.94 | 282,274.70 |
| 1.5 | 5*5m2, Store constructed from CIS with doors and windows, Masonry floor cement screened | No | 1 | 63,434.84 | 63,434.84 |
| 1.6 | Barbed wire fence 60*20m and 1.5m high treated timber post complete with 3m wide gate and a CIS guard house (1.5*2m) | LS | 1 | 61,052.55 | 61,052.55 |
| 1.7 | Toilet and Shower (2*1.5) as per Drawing | No | 2 | 20,000.00 | 40,000.00 |
| 1.8 | Allow for temporary access road to site | km | 3 | 507,475.13 | 1,522,425.39 |
| 1.9 | Dewatering of open trenches and excavations, pumps | LS | 1 | 200,000.00 | 200,000.00 |
| 2 | Provide project indicator post starting from the construction time | LS | 1 | 15,000 | 15,000.00 |
| 2.1 | Provision of as built drawings for the project | LS | 1 | 35,400 | 35,400.00 |
| Total for General Items |  |  |  |  | 2,909,110.46 |

Table 3-3: Bill of quantity and cost estimation for Headwork

| Bill No.2 Head work |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| S. no | Item of work | Unit | Quantity | Unit Price | Amount (ETB) |
| 1.00 | Over Flow Broad Crested Weir Body |  |  |  |  |
| 1.10 | Excavation river diposite (sand,gravel , pebble and boulder) | m3 | 120.78 | 110.00 | 13,285.80 |
| 1.20 | Weathered rock excavation | m3 | 36.23 | 536.41 | 19,434.13 |
| 1.30 | Hard Rock Excavation by jack hamer | m3 | 32.21 | 640.47 | 20,629.54 |
| 1.40 | Cyclopean concrete(60% C-20,40% graded stone) | m3 | 106.14 | 1,804.21 | 191,498.85 |
| 1.50 | Reinforced C-20 concrete | m3 | 55.63 | 2,000.00 | 111,260.00 |
| 1.60 | Ф12mm bar | kg | 824.9 | 75.25 | 62,073.73 |
| Sub Total |  |  |  |  | 418,182.05 |
| 2 | U/S and D/S Apron |  |  |  |  |
| 2.1 | U/S Apron |  |  |  |  |
| 2.1.1 | Excavation river diposite (sand,gravel , pebble and boulder) | m3 | 24.96 | 110 | 2,745.60 |
| 2.1.2 | Weathered rock excavation | m3 | 7.02 | 536.41 | 3,765.60 |
| 2.1.3 | Hard Rock Excavation by jack hamer | m3 | 6.24 | 640.47 | 3,996.53 |
| 2.1.4 | Cyclopean concrete(60% C-20,40% graded stone) | m3 | 10.92 | 1,804.21 | 19,701.97 |
| 2.1.5 | Reinforced C-20 concrete | m3 | 4.68 | 2,000.00 | 9,360.00 |
| 2.1.6 | Ф12mm bar | Kg | 167.02 | 75.25 | 12,568.26 |
| Sub Total |  |  |  |  | 52,137.96 |
| 2.2 | D/S Apron |  |  |  |  |
| 2.2.1 | Excavation river diposite (sand,gravel , pebble and boulder) | m3 | 395.2 | 110 | 43,472.00 |
| 2.2.2 | Weathered rocky excavation | m3 | 111.15 | 536.41 | 59,621.97 |
| 2.2.3 | Hard Rock Excavation by jack hamer | m3 | 74.1 | 640.47 | 47,458.83 |
| 2.2.4 | Cyclopean concrete(60% C-20,40% graded stone) | m3 | 370.5 | 1,804.21 | 668,459.81 |
| 2.2.5 | Reinforced C-20 concrete | m3 | 74.1 | 2,000.00 | 148,200.00 |
| 2.2.6 | Ф12mm bar | Kg | 1,518.88 | 75.25 | 114,295.72 |
| Sub Total |  |  |  |  | 1,081,508.32 |
| 3 | U/S and D/S Cut off |  |  |  |  |
| 3.1 | U/S Cut off |  |  |  |  |
| 3.1.1 | Excavation river diposite (sand,gravel , pebble and boulder) | m3 | 98.67 | 110 | 10,853.70 |
| 3.1.2 | Weathered rock excavation | m3 | 12.87 | 536.41 | 6,903.60 |
| 3.1.3 | Hard Rock Excavation by jack hamer | m3 | 19.31 | 640.47 | 12,367.48 |
| 3.1.4 | Reinforced C-20 concrete | m3 | 20.8 | 2,000.00 | 41,600.00 |
| 3.1.5 | Back fill and Compact with excavated material | m3 | 81.25 | 46.33 | 3,764.31 |
| 3.1.6 | Ф12mm bar | Kg | 332.41 | 75.25 | 25,013.85 |
| Sub Total |  |  |  |  | 100,502.94 |
| 3.2 | D/S Cut off |  |  |  |  |
| 3.2.1 | Excavation river diposite (sand,gravel , pebble and boulder) | m3 | 178.88 | 110 | 19,676.80 |
| 3.2.2 | Weathered rock excavation | m3 | 119.25 | 536.41 | 63,966.89 |
| 3.2.3 | Hard Rock Excavation by jack hamer | m3 | 89.44 | 640.47 | 57,283.64 |
| 3.2.4 | Reinforced C-20 concrete | m3 | 39.75 | 2,000.00 | 79,500.00 |
| 3.2.5 | Back fill and Compact with excavated material | m3 | 139.13 | 46.33 | 6,445.89 |
| 3.2.6 | Ф12mm bar | Kg | 488.93 | 75.25 | 36,791.98 |
| Sub Total |  |  |  |  | 263,665.20 |
| 4 | Under Sluice and Divide Wall |  |  |  |  |
| 4.1 | Under Sluice |  |  |  |  |
| 4.1.1 | Excavation river diposite (sand,gravel , pebble and boulder) | m3 | 52.8 | 110 | 5,808.00 |
| 4.1.2 | Weathered rock excavation | m3 | 4.95 | 536.41 | 2,655.23 |
| 4.1.3 | Hard Rock Excavation by jack hamer | m3 | 1.65 | 640.47 | 1,056.78 |
| 4.1.4 | Cyclopean concrete(60% C-20,40% graded stone) | m3 | 3.96 | 1,804.21 | 7,144.67 |
| 4.1.5 | Reinforced C-20 concrete | m3 | 0.63 | 2,000.00 | 1,260.00 |
| 4.1.6 | Ф12mm bar | Kg | 18.42 | 75.25 | 1,386.11 |
| Sub Total |  |  |  |  | 19,310.78 |
| 4.2 | Divide Wall |  |  |  |  |
| 4.2.1 | Excavation river diposite (sand,gravel , pebble and boulder) | m3 | 2.97 | 110 | 326.7 |
| 4.2.2 | Weathered rock excavation | m3 | 0.89 | 536.41 | 477.4049 |
| 4.2.3 | Hard Rock Excavation by jack hamer | m3 | 0.79 | 640.47 | 505.9713 |
| 4.2.4 | Cyclopean concrete(60% C-20,40% graded stone) | m3 | 6.93 | 1,804.21 | 12503.1753 |
| 4.2.5 | plastering(1:2) | m3 | 30.1 | 126.95 | 3821.195 |
| Sub Total |  |  |  |  | 17,634.45 |
| 5 | Operational slab and |  |  |  |  |
|  | breast wall of under sluice |  |  |  |  |
| 5.1 | Operational slab |  |  |  |  |
| 5.1.1 | C-20 concrete | m3 | 0.28 | 2,000.00 | 560 |
| 5.1.2 | Ф12mm bar | kg | 14.78 | 75.25 | 1112.195 |
| 5.2 | breast wall of under sluice |  |  |  | 0 |
| 5.2.1 | C-20 concrete | m3 | 0.7 | 2,000.00 | 1400 |
| 5.2.2 | Ф12mm bar | kg | 84.83 | 75.25 | 6383.4575 |
| Sub Total |  |  |  |  | 9,455.65 |
| 6 | Retaining wall & bank protection |  |  |  |  |
| 6.1 | U/s Retaining Wall Right & Left |  |  |  |  |
| 6.1.1 | Excavation river diposite (sand,gravel , pebble and boulder) | m3 | 392.63 | 110 | 43,189.30 |
| 6.1.2 | Weathered Rock excavation | m3 | 87.63 | 536.41 | 47,005.61 |
| 6.1.3 | Hard Rock Excavation by jack hamer | m3 | 40 | 640.47 | 25,618.80 |
| 6.1.4 | Masonry (1:3) | m3 | 120 | 1,250.00 | 150,000.00 |
| 6.1.5 | Back fill and Compact with excavated material | m3 | 350 | 46.33 | 16,215.50 |
| 6.1.6 | Pointing (1:2) | m² | 175 | 51.97 | 9,094.75 |
| 6.1.7 | Lean Concrete(C-15) | m3 | 8.5 | 1,250.00 | 10,625.00 |
| Sub Total |  |  |  |  | 301,748.96 |
| 6.2 | Downstream Retaining Wall Right & Left |  |  |  |  |
| 6.2.1 | Excavation river diposite (sand,gravel , pebble and boulder) | m3 | 63 | 110 | 6,930.00 |
| 6.2.2 | Weathered Rock excavation | m3 | 18.9 | 536.41 | 10,138.15 |
| 6.2.3 | Hard Rock Excavation by jack hamer | m3 | 16.8 | 640.47 | 10,759.90 |
| 6.2.4 | Masonry (1:3) | m3 | 210 | 1,250.00 | 262,500.00 |
| 6.2.5 | Back fill and Compact with excavated material | m3 | 63 | 46.33 | 2,918.79 |
| 6.2.6 | Pointing (1:2) | m² | 72.8 | 51.97 | 3,783.42 |
| 6.2.7 | Lean Concrete(C-15) | m3 | 4.2 | 1,250.00 | 5,250.00 |
| 6.2.8 | Cyclopean concrete(60% C-20,40% graded stone) |  | 63 | 1,804.21 | 113,665.23 |
| 6.3 | Bank protection work |  |  |  | 0.00 |
| 6.3.1 | Excavation river diposite (sand,gravel , pebble and boulder) | m3 | 300 | 110 | 33,000.00 |
| 6.3.2 | gabion work | m3 | 780 | 973.98 | 759,704.40 |
| Sub total |  |  |  |  | 1,208,649.88 |
| 7 | Gates & trash rack |  |  |  |  |
| 7.1 | Under sluice gate installation consist of: | No | 1 | 18,452.00 | 18,452.00 |
|  | 8mm thickness sheet metal,1.1m X1.0m |  |  |  |  |
|  | 50x50x10mm angle iron, 10.4m length |  |  |  |  |
|  | 60 x 60 x 10mm stiffing angle iron, 3m length |  |  |  |  |
|  | Φ12mm bar for anchorage, 20Kg |  |  |  |  |
|  | Spidle of 60mm dia ,4.2m length |  |  |  |  |
| 7.2 | Offtake canal gate installation consist of: |  |  |  |  |
|  | 4mm thickness sheet metal,0.6m X0.65m. With 1.8m length | No | 1 | 8,231.94 | 8,231.94 |
|  | 40x40x10mm staffing angle iron, 2.5m length |  |  |  |  |
|  | 50x50x10mm angle iron, 2.5mm length |  |  |  |  |
|  | Φ12mm bar for anchorage, 3Kg |  |  |  |  |
|  | handle made of 20 dia steel bar of 0.7m length |  |  |  |  |
|  | Sub Total |  |  |  | 26,683.94 |
| 7.3 | Trash rack |  |  |  |  |
| 7.3.1 | Φ12mm reinforcement bar | kg | 15.63 | 75.25 | 1176.1575 |
|  | Sub Total |  |  |  | 1176.1575 |
| Grand Total |  |  |  |  | 3,500,656.29 |

# IRRIGATION SYSTEM DESIGN

## Irrigable Area Description

### Topography

Topography is an important factor for the planning of any irrigation project as it influences method of irrigation, drainage, erosion, mechanization, and cost of land development, labor requirement and choice of crops.

The topographic feature of the project command area is mainly sloping type. Its elevation range is from 1367.56 to 1279.07 meters above sea level. The slope gradient also ranges from gently sloping (0.1%) to strongly sloping (11.11%). However, it has identified to be suitable for surface irrigation. Nevertheless, it requires soil and water conservation measures or structures (i.e. constructing bunds, bio-physicals, check dams, artificial water ways, etc.).

The project command area is situated at the left side of Challie River (to the East side of the river). The natural topographic feature of the command area has inclined from the South-West to the South-East direction.

### Climate

According to the traditional Ethiopian agro-ecological zones classification (MOA, 2001) the proposed irrigation project area is basically categorized under Kolla agro-ecology which is conducive to the production of tropical lowland crops.The project area has bimodal and uneven distribution pattern of rainfall. The main rain season, locally known as Kiremt, occurs from July to August with about 94% share of annual rainfall; and about 6% of the annual rainfall occurs during the dry season from October to May.

In the project area the main bottle neck for the successful crop production is uneven distribution of rainfall and hail damage, especially in the months of July and August. The highest rainfall occurs in the months of July and August with a better intensity and spatial distribution. Had the annual rainfall been evenly distributed throughout the year, the amount of rainfall would have to been sufficed for the crops grown in both dry and wet seasons.

As the project site has no its own meteorological station, Haik meteorological stations data (for rainfall, minimum and maximum temperature,  relative humidity, wind speed and sunshine hour) were used for the project agronomy study as long as these stations are relatively near to the proposed command area. In general, the sources of meteorological data are the National Meteorology Service Agency (NMSA).

### Soil characteristics

Soil properties (physical, chemical, etc.) greatly influence the growth and thereby yield of crops which is grown. The command area has predominantly clay loam textured soils which can be classified as imperfectely drained soil.  Most of the study area soils are categorized as deep soil (0.5-1.0 meter depth). Soils of the command area are suitable for most of the selected crops to be grown (for further detail see the Agronomy Study of this project).

### Existing Irrigation Practices in the Project Area

The pressure of survival and the need for additional food supplies to meet the demands of the increasing population is necessitating a rapid expansion of irrigation schemes. Thus, irrigation is becoming a basic part of well-developed agriculture wherever there is water and irrigable land potential. Accordingly, traditional irrigation practices are practicing using water from Challie River by constructing traditional diversion structures and fetching water from reservoirs with plastic water can. Furthermore, farmers in the project area have constructed small water wells to use as a source of water for irrigation. So, the farmers in the project area are very much interested in the idea of constructing modern irrigation scheme.

## Irrigation Water Requirement

### Crop Water Requirement (CWR)

The calculation of crop water requirement is a very important aspect for planning of any irrigation project. Several methods and procedures are available for this. The Food and Agriculture Organization (FAO) of the United Nations has also made available several publications on this subject and other issues related with this. The computer program available in FAO Irrigation and Drainage Paper No. 56 “CROPWAT” has been used for the calculation of Crop Water requirement. This program is based on Penman-Monteith approach and procedures for calculation of crop water requirements and irrigation requirements are mainly based on methodologies presented in FAO Irrigation and Drainage Paper No. 24 “Crop Water Requirements” and No. 33 “Yield Response to Water”.

The corresponding values of the crop water requirements of the proposed crops of the project are presented in the Agronomy Study of the same project.

### Irrigation efficiency

To complete the evaluation of the demand, the efficiency of the water distribution system and of application must be known.

The gross requirement of water for irrigation system is very much dependent on the overall efficiency of the irrigation system, which in turn is dependent on several factors: Method of irrigation, type of canal (Lined and/or Unlined), method of operations (simultaneously and continuous or Rotational water supply), and availability of structures (for controlling and distribution and measuring and monitoring).

On the basis of these factors, the project has planned to impose surface irrigation method (using furrows). The irrigation canal system in the project area designed as lined main & secondary canals and unlined other tertiary & field canals. Hence, the conveyance efficiency has been estimated to be 90% and field application efficiency 65%. As a result of these the overall irrigation efficiency has been estimated to be 58%. According to soil Lab result, soils of the command area are predominantly characterized as clay loam soils.

### Irrigation duty

Irrigation duty is the volume of water required per hectare for the full flange of the crops. Moreover, it helps in designing an efficient irrigation canal system.

The area, which will be irrigated, can be calculated by knowing the total available water at the source and the overall duty for all crops required to be irrigated in different seasons of the years.

The proposed cropping pattern of Challie diversion irrigation project has showed a maximum net irrigation water requirement (NIWR) in the month of March with the amount of 4.2 mm/day for 24 working hours (for overall proposed crops).

However, for the designing of the irrigation water application and flows in the entire canal system for overall dry & wet season proposed crops of Challie diversion irrigation project, the maximum gross scheme irrigation water requirement (MGSIWR) has given here below:

For the designing of the project, the GIWR is given as follows:

GIWR = NIWR/e = 4.2/0.58 = 7.24=7.2 [mm/day] for full irrigation

GIWR = NIWR/e = 2.8/0.58= 4.828=4.83 [mm/day] for supplementary irrigation

The GIWR (7.2 and 4.83 mm/day) represents the daily quantity of water that is required to be applied for dry and wet season crops, respectively. This water quantity is also used for the determination of the canal discharge in consideration of the time of flow and also defined as the duty, expressed as l/s/ha.

The duty is calculated by:

Duty (D) = GIWR × 1000 × 10 / (t × 60×60)

Where; Duty – the duty [l/s/ha]

GIWR – Gross Irrigation Requirement [mm/day]

t – Daily irrigation or flow hours [hrs.]

The duty for GIWR of 7.2 mm/day (dry seasons) for daily irrigation time (t =14 hrs.) and 4.83 mm/day (wet season) for daily irrigation time (12 hrs.). Thus, duty for 14and 12 irrigation hours of dry and wet seasons’ full and supplementary irrigation has computed as follows, respectively:

D = (7.2x10x1000) / (14x3600) =1.428= 1.43 l/s/h (for full irrigation during dry season)

D = (4.83x10x1000) / (12x3600) =1.099=1.12 l/s/h (for supplementary irrigation during wet season)

### Irrigation methods

Among the different irrigation systems perennial irrigation system will be used for the project area; and the irrigation water will be obtained from Challie River and by constructing diversion Weir and convoying the water commonly through main, secondary and tertiary canals and then leading to field canals and finally turning to irrigation furrows.

For this project, among the various irrigation methods, surface irrigation method has been selected.  Of the surface irrigation methods furrow, border and basin irrigation methods can be used to supply irrigation water to the plants/crops. However, each method has its own advantages and disadvantages. Care should be taken when choosing the method which is best suited to the local circumstances, i.e., depending on slopes, soil types, selected crop types, amount of water available, etc. of the command area.

Based on the above factors surface irrigation method with the combination of bench terraces has proposed for the selected crops in this project. The method allows applying light irrigation and can be used on a variety of fields (sloping fields) along the contour. Furrow irrigation method is best suited for most of the proposed and row planted crops. In general, furrow irrigation method is simple, manageable and widely practiced irrigation method. This method is suitable for row crops that cannot stand in water for long periods. The only thing required to use this method is row planting of crops. Besides, basin and border irrigation method would be used for the non-row planted crops. Rotational flow water distribution is also recommended for the project area.

### Irrigation System Layout

The irrigation system layout for the project is prepared taking the following points into consideration besides other factors.

- A primary concern in the layout of the system is that it serves the purpose of conveying and distributing water to the command area.

- The excavation and earth fill volumes not be excessive, otherwise the construction costs can be tremendous.

- The selection of longitudinal bed slope is made taking into account the existing slopes of the terrain, so as to minimize deviations in canal routing.

- Curves in canals should not be too sharp.

The proposed irrigation system layout comprises one main canal, one secondary canal, 27 tertiary canals and 72 Field canals as shown on the layout Drawings. The main canal runs for almost all of its length parallel to the contours and several changes of direction are necessary to follow the topography. It crosses eight main gullies, two foot paths and one URAP road.

All the length of main canal is lined from which the first 111.7m (i.e. 0+00m-111.7 m) is Reinforced concrete and the left 4212.3m (i.e. 111.7m-4324m) are Masonry lined.

### Conveyance System

The conveyance system consists of one Main canal to irrigate total command area of 165 ha. The main canal starts from Water abstraction site on left side of the river and conveys water for a length of 4323.53m.

Main canal is aligned along contours and supplies to 17 field canals, three tertiary canals, and at its final chain age it supplies for secondary.

## Hydraulic Design of Canals

The earthen canals have been designed with a trapezoidal shape and the lined ones with rectangular x-section using Manning's Formula:

Where Q= discharge (m3/s)

R= Hydraulic radius (Flow area/wetted perimeter)

S= Hydraulic gradient

n= Manning's roughness coefficient, n=0.03 is adopted for the clay loam earth channels, n=0.018 for the masonry lined part and n=0.013 for concrete lined part of the canal

### Main Canal

The main canal is designed as lined for a discharge of 280l/s and depending on the site specific condition, appropriate slope is provided. Hydraulic parameters of the main canal are shown below.

At a chain age of 0+120,0+460,0+996.97, 1+126.4, 2+502.78, 2+910.14, 3+070.47 and 3+548.17 & 4+170 there are Flume structures.

At a chain age of 0+471.39 there is super passage structures and at 0+020 & 4+287.24 there are culvert structures.

Table 4-1: Hydraulic Parameters of main canal

| Chain age | QR (m3/s) | n | S | BW (m) | FSD (m) | A(m2) | WP (m) | R(m) | FB (m) | V(m/s) | QD (m3/s) | TD (m) | Remark |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0+00-0+111.7 | 0.280 | 0.013 | 0.0014 | 0.6 | 0.45 | 0.29 | 1.55 | 0.19 | 0.25 | 0.96 | 0.280 | 0.8 | RC Canal |
| 0+111.7-0+340.87 | 0.280 | 0.018 | 0.0014 | 0.6 | 0.59 | 0.38 | 1.82 | 0.21 | 0.31 | 0.74 | 0.280 | 0.8 | Masonry Canal |
| 0+340.87-0+471.39 | 0.280 | 0.018 | 0.0010 | 0.6 | 0.68 | 0.44 | 2.01 | 0.22 | 0.22 | 0.64 | 0.283 | 0.8 | Masonry Canal |
| 0+471.39-0+474.43 | 0.280 | 0.018 | 0.0010 | 0.6 | 0.68 | 0.44 | 2.01 | 0.22 | 0.22 | 0.64 | 0.283 | 0.8 | Super passage |
| 0+475.17-0+816.73 | 0.280 | 0.018 | 0.0010 | 0.6 | 0.68 | 0.44 | 2.01 | 0.22 | 0.22 | 0.64 | 0.283 | 0.8 | Masonry Canal |
| 0+816.73-0+996.97 | 0.280 | 0.018 | 0.0014 | 0.6 | 0.59 | 0.38 | 1.82 | 0.21 | 0.31 | 0.74 | 0.280 | 0.8 | Masonry Canal |
| 0+996.97-1+019.26 | 0.280 | 0.013 | 0.0020 | 0.6 | 0.35 | 0.23 | 1.35 | 0.17 | 0.3 | 1.05 | 0.280 | 0.7 | Flume |
| 1+019.26-1+126.4 | 0.280 | 0.018 | 0.0014 | 0.6 | 0.59 | 0.38 | 1.82 | 0.21 | 0.31 | 0.74 | 0.280 | 0.8 | Masonry Canal |
| 1+126.4-1+162.9 | 0.280 | 0.013 | 0.0020 | 0.6 | 0.35 | 0.23 | 1.35 | 0.17 | 0.3 | 1.05 | 0.280 | 0.7 | Flume |
| 1+162.9-2502.78 | 0.280 | 0.018 | 0.0014 | 0.6 | 0.59 | 0.38 | 1.82 | 0.21 | 0.31 | 0.74 | 0.280 | 0.7 | Masonry Canal |
| 2+502.78-2+526.62 | 0.280 | 0.013 | 0.0020 | 0.6 | 0.35 | 0.23 | 1.35 | 0.17 | 0.3 | 1.05 | 0.280 | 0.7 | Flume |
| 2+526.62-2+910.14 | 0.280 | 0.018 | 0.0014 | 0.6 | 0.59 | 0.38 | 1.82 | 0.21 | 0.31 | 0.74 | 0.280 | 0.8 | Masonry Canal |
| 2+910.14-2+934.24 | 0.280 | 0.013 | 0.0020 | 0.6 | 0.35 | 0.23 | 1.35 | 0.17 | 0.3 | 1.05 | 0.280 | 0.7 | Flume |
| 2+934.24-3+070.47 | 0.280 | 0.018 | 0.0014 | 0.6 | 0.59 | 0.38 | 1.82 | 0.21 | 0.31 | 0.74 | 0.280 | 0.8 | Masonry Canal |
| 3+070.47-3+093.14 | 0.280 | 0.013 | 0.0020 | 0.6 | 0.35 | 0.23 | 1.35 | 0.17 | 0.3 | 1.05 | 0.280 | 0.7 | Flume |
| 3+093.14-3+548.17 | 0.280 | 0.018 | 0.0014 | 0.6 | 0.59 | 0.38 | 1.82 | 0.21 | 0.3 | 0.74 | 0.280 | 0.8 | Masonry Canal |
| 3+548.17-3+575.24 | 0.280 | 0.013 | 0.0020 | 0.6 | 0.35 | 0.23 | 1.35 | 0.17 | 0.3 | 1.05 | 0.280 | 0.7 | Flume |
| 3+575.24-4+287.24 | 0.280 | 0.018 | 0.0014 | 0.6 | 0.59 | 0.38 | 1.82 | 0.21 | 0.31 | 0.74 | 0.282 | 0.8 | Masonry Canal |
| 4+287.24-4+293.24 | 0.280 | 0.018 | 0.0014 | 0.6 | 0.59 | 0.38 | 1.83 | 0.21 | 0.31 | 0.74 | 0.284 | 0.8 | Culvert |
| 4+293.24-4+323.53 | 0.280 | 0.018 | 0.0014 | 0.6 | 0.59 | 0.38 | 1.82 | 0.21 | 0.31 | 0.74 | 0.280 | 0.8 | Masonry Canal |

BW= Canal bottom width          QR= required discharge, FB= Free board                 S= longitudinal slope

FSD= Full supply depth     QD= Designed Discharge, WP=Wetted perimeter, TD=Total depth of canal

V= Velocity                                           R=Hydraulic Radius

### Secondary Canals

These canals also designed as lined with a rectangular x-section.

Table 4-2: Hydraulic Parameters of secondary canals

| Chain age | QR (m3/s) | n | S | BW (m) | FSD (m) | A (m2) | P(m) | R(m) | FB (m) | V (m/s) | QD (m3/s) | TD (m) | Remark |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0+000-0+070.2 | 0.177 | 0.013 | 0.0714 | 0.3 | 0.16 | 0.048 | 0.62 | 0.077 | 0.2 | 3.73 | 0.179 | 0.4 | Chute |
| 0+070.2-0+190.55 | 0.177 | 0.013 | 0.0714 | 0.3 | 0.16 | 0.048 | 0.62 | 0.077 | 0.2 | 3.73 | 0.179 | 0.4 | Chute |
| 0+190.55-0+300.87 | 0.177 | 0.013 | 0.0690 | 0.3 | 0.17 | 0.051 | 0.64 | 0.080 | 0.2 | 3.74 | 0.191 | 0.4 | Chute |
| 0+300.87-0+441.13 | 0.177 | 0.013 | 0.0417 | 0.3 | 0.2 | 0.06 | 0.7 | 0.086 | 0.2 | 3.05 | 0.183 | 0.4 | Chute |
| 0+441.13-0+556.34 | 0.069 | 0.018 | 0.0222 | 0.3 | 0.16 | 0.048 | 0.62 | 0.077 | 0.3 | 1.50 | 0.072 | 0.5 | Canal |
| 0+556.34-0+716.63 | 0.069 | 0.018 | 0.0067 | 0.3 | 0.25 | 0.075 | 0.8 | 0.094 | 0.3 | 0.94 | 0.070 | 0.5 | Canal |
| 0+716.63-0+771.73 | 0.069 | 0.018 | 0.0080 | 0.3 | 0.23 | 0.069 | 0.76 | 0.091 | 0.3 | 1.00 | 0.069 | 0.5 | Canal |
| 0+771.73-0+866.91 | 0.069 | 0.018 | 0.0526 | 0.3 | 0.12 | 0.036 | 0.54 | 0.067 | 0.3 | 2.10 | 0.075 | 0.5 | Canal |
| 0+866.91-0+962.08 | 0.069 | 0.018 | 0.0400 | 0.3 | 0.13 | 0.039 | 0.56 | 0.070 | 0.3 | 1.88 | 0.073 | 0.5 | Canal |
| 0+962.08-1+257.62 | 0.069 | 0.018 | 0.0014 | 0.3 | 0.46 | 0.138 | 1.22 | 0.113 | 0.3 | 0.49 | 0.068 | 0.7 | Canal |
| 1+257.62-1+307.71 | 0.055 | 0.013 | 0.1111 | 0.3 | 0.08 | 0.024 | 0.46 | 0.052 | 0.2 | 3.58 | 0.086 | 0.3 | Chute |
| 1+307.71-1+362.81 | 0.055 | 0.018 | 0.0333 | 0.3 | 0.12 | 0.036 | 0.54 | 0.067 | 0.3 | 1.67 | 0.060 | 0.4 | Canal |
| 1+362.81-1+478.01 | 0.055 | 0.018 | 0.0200 | 0.3 | 0.14 | 0.042 | 0.58 | 0.072 | 0.3 | 1.36 | 0.057 | 0.4 | Canal |
| 1+478.01-1+568.18 | 0.055 | 0.018 | 0.0385 | 0.3 | 0.11 | 0.033 | 0.52 | 0.063 | 0.3 | 1.73 | 0.057 | 0.4 | Canal |
| 1+568.18-1+633.3 | 0.055 | 0.018 | 0.0455 | 0.3 | 0.11 | 0.033 | 0.52 | 0.063 | 0.3 | 1.88 | 0.062 | 0.4 | Canal |
| 1+633.3-1+733.48 | 0.055 | 0.018 | 0.0167 | 0.3 | 0.15 | 0.045 | 0.6 | 0.075 | 0.3 | 1.28 | 0.057 | 0.4 | Canal |
| 1+733.48-1+813.63 | 0.055 | 0.018 | 0.0250 | 0.3 | 0.13 | 0.039 | 0.56 | 0.070 | 0.3 | 1.49 | 0.058 | 0.4 | Canal |
| 1+813.63-1+893.78 | 0.055 | 0.018 | 0.0400 | 0.3 | 0.11 | 0.033 | 0.52 | 0.063 | 0.3 | 1.77 | 0.058 | 0.4 | Canal |
| 1+893.78-2+029.03 | 0.055 | 0.018 | 0.0100 | 0.3 | 0.18 | 0.054 | 0.66 | 0.082 | 0.3 | 1.05 | 0.057 | 0.4 | Canal |
| 2+029.03-2+129.21 | 0.055 | 0.018 | 0.0278 | 0.3 | 0.12 | 0.036 | 0.54 | 0.067 | 0.3 | 1.52 | 0.055 | 0.4 | Canal |

Figure 4-1: Typical Cross Section of main and secondary canals

### Tertiary canals

Since, the type of soil in the command area is clay loam tertiary canals are designed as earthen with trapezoidal x-section.

In the layout system there are 27 tertiary canals, the designed discharge is determined based on the duty of irrigation. The hydraulic characteristics is Presented in Table 4.3 below.

Table 4-3: Hydraulic Parameters of tertiary canals

| Parameters | TC1-1 | TC1-2 | TC1-3 | TC1-1-1 | TC1-1-2 | TC1-1-3 | TC1--1-4 | TC1-1-5 | TC1--1-6 | TC1-1-7 | TC1-1-8 | TC1-1-9 | TC1-1-10 | TC1-1-11 | TC1-1-12 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Design discharge ,Q | 0.011 | 0.004 | 0.001 | 0.006 | 0.006 | 0.008 | 0.013 | 0.010 | 0.011 | 0.007 | 0.022 | 0.007 | 0.010 | 0.010 | 0.007 |
| (b/d) ratio | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
| Side slope ,Z | 1.5 | 1.5 | 1.5 | 1.5 | 1.5 | 1.5 | 1.5 | 1.5 | 1.5 | 1.5 | 1.5 | 1.5 | 1.5 | 1.5 | 1.5 |
| Manning's roughness, n | 0.03 | 0.03 | 0.03 | 0.03 | 0.03 | 0.03 | 0.03 | 0.03 | 0.03 | 0.03 | 0.03 | 0.03 | 0.03 | 0.03 | 0.03 |
| Lon gitudinal slope, S | 600 | 600 | 600 | 250 | 250 | 300 | 500 | 300 | 400 | 500 | 600 | 300 | 400 | 500 | 350 |
| water depth, d | 0.14 | 0.09 | 0.06 | 0.09 | 0.09 | 0.11 | 0.14 | 0.12 | 0.12 | 0.11 | 0.18 | 0.10 | 0.12 | 0.13 | 0.11 |
| bed width, b | 0.14 | 0.09 | 0.06 | 0.09 | 0.09 | 0.11 | 0.14 | 0.12 | 0.12 | 0.11 | 0.18 | 0.10 | 0.12 | 0.13 | 0.11 |
| Area | 0.047 | 0.020 | 0.010 | 0.020 | 0.020 | 0.029 | 0.048 | 0.034 | 0.039 | 0.031 | 0.078 | 0.026 | 0.036 | 0.041 | 0.028 |
| Wetted perimeter, p | 0.634 | 0.411 | 0.291 | 0.414 | 0.416 | 0.496 | 0.639 | 0.538 | 0.575 | 0.513 | 0.813 | 0.472 | 0.552 | 0.591 | 0.488 |
| Hydraulic radius | 0.075 | 0.048 | 0.034 | 0.049 | 0.049 | 0.058 | 0.075 | 0.063 | 0.068 | 0.060 | 0.096 | 0.056 | 0.065 | 0.070 | 0.057 |
| Velocity, v | 0.241 | 0.181 | 0.144 | 0.282 | 0.282 | 0.290 | 0.266 | 0.306 | 0.277 | 0.230 | 0.285 | 0.280 | 0.270 | 0.252 | 0.265 |
| Canal Discharge capacity | 0.011 | 0.004 | 0.001 | 0.006 | 0.006 | 0.008 | 0.013 | 0.010 | 0.011 | 0.007 | 0.022 | 0.007 | 0.010 | 0.010 | 0.007 |
| Free board | 0.15 | 0.15 | 0.15 | 0.15 | 0.15 | 0.15 | 0.15 | 0.15 | 0.15 | 0.15 | 0.15 | 0.15 | 0.15 | 0.15 | 0.15 |
| Total depth | 0.29 | 0.24 | 0.21 | 0.24 | 0.24 | 0.26 | 0.29 | 0.27 | 0.27 | 0.26 | 0.33 | 0.25 | 0.27 | 0.28 | 0.26 |
| Adopted, depth, d | 0.2 | 0.2 | 0.2 | 0.2 | 0.2 | 0.2 | 0.2 | 0.2 | 0.2 | 0.2 | 0.2 | 0.2 | 0.2 | 0.2 | 0.2 |
| Adopted, Bed width, b | 0.3 | 0.3 | 0.3 | 0.3 | 0.3 | 0.3 | 0.3 | 0.3 | 0.3 | 0.3 | 0.3 | 0.3 | 0.3 | 0.3 | 0.3 |

| Parameters | TC1-1-13 | TC1-1-14 | TC1-1-15 | TC1-1-16 | TC1-1-17 | TC1-1-18 | TC1-1-19 | TC1-1-20 | TC1-1-21 | TC1-1-22 | TC1-1-23 | TC1-1-24 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Design discharge ,Q | 0.009 | 0.013 | 0.004 | 0.004 | 0.004 | 0.008 | 0.004 | 0.007 | 0.005 | 0.002 | 0.001 | 0.003 |
| Bed width to depth ratio (b/d) | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
| Side slope ,Z | 1.5 | 1.5 | 1.5 | 1.5 | 1.5 | 1.5 | 1.5 | 1.5 | 1.5 | 1.5 | 1.5 | 1.5 |
| Manning's roughness, n | 0.03 | 0.03 | 0.03 | 0.03 | 0.03 | 0.03 | 0.03 | 0.03 | 0.03 | 0.03 | 0.03 | 0.03 |
| Longitudinal slope, S | 300 | 500 | 400 | 250 | 250 | 150 | 250 | 400 | 300 | 180 | 300 | 200 |
| water depth, d | 0.11 | 0.14 | 0.09 | 0.08 | 0.08 | 0.09 | 0.08 | 0.11 | 0.09 | 0.06 | 0.08 | 0.07 |
| bed width, b | 0.11 | 0.14 | 0.09 | 0.08 | 0.08 | 0.09 | 0.08 | 0.11 | 0.09 | 0.06 | 0.08 | 0.07 |
| Area | 0.031 | 0.047 | 0.018 | 0.014 | 0.014 | 0.021 | 0.016 | 0.028 | 0.020 | 0.009 | 0.017 | 0.013 |
| Wetted perimeter | 0.516 | 0.635 | 0.394 | 0.348 | 0.346 | 0.423 | 0.367 | 0.488 | 0.414 | 0.278 | 0.379 | 0.327 |
| Hydraulic radius | 0.061 | 0.075 | 0.046 | 0.041 | 0.041 | 0.050 | 0.043 | 0.058 | 0.049 | 0.033 | 0.045 | 0.039 |
| Velocity, v | 0.298 | 0.265 | 0.215 | 0.251 | 0.250 | 0.369 | 0.260 | 0.249 | 0.257 | 0.255 | 0.242 | 0.269 |
| Canal Discharge capacity | 0.009 | 0.013 | 0.004 | 0.004 | 0.004 | 0.008 | 0.004 | 0.007 | 0.005 | 0.002 | 0.004 | 0.003 |
| Free board | 0.15 | 0.15 | 0.15 | 0.15 | 0.15 | 0.15 | 0.15 | 0.15 | 0.15 | 0.15 | 0.15 | 0.15 |
| Total depth | 0.26 | 0.29 | 0.24 | 0.23 | 0.23 | 0.24 | 0.23 | 0.26 | 0.24 | 0.21 | 0.23 | 0.22 |
| Adopted, depth, d | 0.2 | 0.2 | 0.2 | 0.2 | 0.2 | 0.2 | 0.2 | 0.2 | 0.2 | 0.2 | 0.2 | 0.2 |
| Adopted, Bed width, b | 0.3 | 0.3 | 0.3 | 0.3 | 0.3 | 0.3 | 0.3 | 0.3 | 0.3 | 0.3 | 0.3 | 0.3 |

### Field Canals

As shown in the layout, field canals run across the contours and hence face relatively steeper gradient. The discharge of most of the field canals is very small and this is taken as an advantage to cope up with the relatively steeper gradient. All of the filed canals designed to irrigate both sides of the command area depending on the condition of the individual plots of land owned by individual farmers. These are constructed by farmers.

## Canal related Structures Design

### Cross drainage works

Generally, these structures are provided on a canal when it crosses a drainage channels (rivers, streams) and it also provided whenever canals cross each other. As far as possible, the canal should be carried above or below the channel, and “level crossings” should be avoided since they cause silt to enter the canals and, in floods, debris and excess water may damage the canal.

Accordingly, the draft design of Challie diversion irrigation project includes nine flume structures , one super passage structure and two culvert structures.

Typical Hydraulic design of flume1 @ 0+996.97.

1. Given Data

Hydraulic Characteristics of the canal

| Discharge, m3/s | Bed width b1(m) | Water depth d1(m) | Side slope(m) | Velocity v(m/s) |
| --- | --- | --- | --- | --- |
| 0.28 | 0.6 | 0.59 | 0 | 0.73 |

Length of the flume: 22m

Shape of the flume:  Rectangular

2. Flume Section

Assume

Roughness coefficient, n =0.013

Water area,

Bed width,

Water depth,

Velocity,

This is equal to the assumed velocity. O.K.

Froude number,

. This proves to be stable sub critical flow.

Wetted perimeter, P= b +2d = (0.65) +2x (0.35) =1.35m

Hydraulic radius,

Friction head loss,

3.Water surface change at the u/s and d/s of the flume

i. Drop of water surface at the inlet of the flume

ii. Rise of water surface at the d/s of canal section

4.  Elevation

- Invert elevation at u/s canal		= 1365.98m

- Water surface elevation at u/s canal	= (1365.98)+(0.59) =1366.573m

- Water surface at the flume inlet	= (Water surface elevation at u/s canal)-

= (1366.573)-(0.01) =1366.563m

- Invert elevation at the flume inlet= (Water surface at the flume inlet) –d2

= (1366.563) – (0.350) = 1366.213 m

- Invert elevation at the flume inlet =(Invert elevation at the flume outlet) -hf

= (1366.213) - (0.055) =1366.152m

- Water surface elevation at the flume outlet = (Invert elevation at the flume outlet) + d2

= (1366.152) + (0.350) =1366.502m

- Water surface elevation at the d/s canal 	= (Water surface elevation at the flume Outlet) +

= (1366.502) + (0.015) =1366.487m

- Invert elevation at the d/s canal	= (Water surface elevation at the d/s canal) -

= (1366.487) – (0.59) =1365.897m

5. Total head loss

HT= 0.086m

Check water surface difference between u/s and d/s canal

The difference = (water surface elevation at u/s canal) – (Water surface elevation at d/s canal = (1366.573) – (1366.487) =0.086m O.K.

6.Canal elevation at each section

| Section | Location | Water depth (m) | Change in water level (m) |  | Elevation of |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Canal bed | Water surface |
| A B C D | u/s canal Flume Flume d/s canal | 0.59 0.35 0.35 0.59 | - drop drop rise | - 0.010 0.061 0.015 | 1365.983 1366.213 1366.152 1365.897 | 1366.573 1366.563 1366.502 1366.487 |

For the detail analysis of flume structures and super passage see excel analysis.

Table 4-4: Hydraulic Parameters of Flumes

| Chainage | Value of n | Bed slope | BW (m) | FSD (m) | A(m2) | P(m) | R (m) | FB (m) | V (m/s) | QR(m3/s) | QD (m3/s) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0+120-1+019.26 | 0.013 | 500 | 0.65 | 0.35 | 0.228 | 1.35 | 0.169 | 0.3 | 1.05 | 0.28 | 0.280 |
| 1+126.4-1+162.9 | 0.013 | 500 | 0.65 | 0.35 | 0.228 | 1.35 | 0.169 | 0.3 | 1.05 | 0.28 | 0.280 |
| 2+502.78-2+526.62 | 0.013 | 500 | 0.65 | 0.35 | 0.228 | 1.35 | 0.169 | 0.3 | 1.05 | 0.28 | 0.280 |
| 2+910.14-2+934.24 | 0.013 | 500 | 0.65 | 0.35 | 0.228 | 1.35 | 0.169 | 0.3 | 1.05 | 0.28 | 0.280 |
| 3+070.47-3+093.14 | 0.013 | 500 | 0.65 | 0.35 | 0.228 | 1.35 | 0.169 | 0.3 | 1.05 | 0.28 | 0.280 |
| 3+548.17-3+575.24 | 0.013 | 500 | 0.65 | 0.35 | 0.228 | 1.35 | 0.169 | 0.3 | 1.05 | 0.28 | 0.280 |

### Design of Division box

At different points of the main and secondary canals division boxes are provided to divert the flow in to the tertiary canals. Gates should be provided at the outlet of the boxes. These are designed Using broad crested formula, Q= CL (h) 3/2

Figure 4-2: Typical Division Box plan

Broad crested formula,

Q= CL (h) 3/2

Where; Q= discharge over rectangular weir/Weir (opening), m3/s

C = discharge coefficient, c= 1.7

L= effective length of crest form in m

h= over flow depth, m

Assuming equal discharge coefficient & sill height for two or three dividing canals, the proportion becomes;

Q1/ Q2= Q2/ Q3 =L1/ L2 = L2/ L3

Where Q1= is flow in canal 1

Q2 = is flow in canal 2

Q3 = is flow in canal 3

L1= is effective crest length of weir/Weir sill across opening to canal 1

L2= is effective crest length of weir/Weir sill across opening to canal 2

L3 = is effective crest length of weir/Weir sill across opening to canal 3

Q1= CL1 (h) 3/2,

L1 = Q1/Ch3/2

L2 = L1*Q2/ Q1

L3 = L1*Q3/ Q1

The depth of (height of) the division box,

D = d + fb

The width of the division box,

B = b + 2*m*D

Where b= base width of the incoming canal

D = total canal depth of the incoming canal

Table 4-5: Summery of Hydraulic parameters of Division Boxes

| Divider canal name | Dividing canal name | Chain age (m) | Ongoing, Q0 (lit/sec) | Outgoing, Q1 (lit/sec) | Tc flow, Q2 (lit/sec) | Q3 (lit/sec) | B0 (m) | D= (d0+fb) (m) | B (m) | B1 (m) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| MC | TC1-1 | 2278.00 | 280 | 280.0 | 11.4200 | 0.0 | 0.65 | 0.90 | 12.22 | 0.65 |
|  | TC1-2 | 4297.43 | 280 | 280.0 | 3.5900 | 0.0 | 0.65 | 0.90 | 4.39 | 0.65 |
|  | TC1-3 & SC1 | 4323.43 | 280 | 177.0 | 1.4300 | 0.0 | 0.65 | 0.90 | 2.23 | 0.65 |
| SC | TC_1-1-1&TC1-1-2 | 70.20 | 177 | 177.0 | 5.7000 | 5.8 | 0.30 | 0.40 | 6.50 | 0.3 |
|  | TC1-1-3 & TC 1-1-4 | 190.55 | 177 | 177.0 | 8.7200 | 12.8 | 0.30 | 0.40 | 9.52 | 0.3 |
|  | TC1-1-5& TC1-1-6 | 300.87 | 177 | 177.0 | 10.4200 | 10.8 | 0.30 | 0.40 | 11.22 | 0.3 |
|  | TC1-1-7&TC1-1-8 | 436.12 | 69 | 69.0 | 7.1300 | 22.2 | 0.30 | 0.40 | 7.93 | 0.3 |
|  | TC9 | 706.61 | 69 | 69.0 | 7.3500 | 0.0 | 0.30 | 0.50 | 8.15 | 0.3 |
|  | TC1-1-10 & TC1-1-11 | 826.83 | 69 | 69.0 | 9.6700 | 10.4 | 0.30 | 0.50 | 10.47 | 0.3 |
|  | TC1-1-12 | 958.20 | 69 | 69.0 | 7.4400 | 0.0 | 0.30 | 0.50 | 8.24 | 0.3 |
|  | TC1-1-13 | 1257.62 | 69 | 69.0 | 9.3500 | 0.0 | 0.30 | 0.70 | 10.15 | 0.3 |
|  | TC1-1-14& TC1-1-15 | 1362.81 | 55 | 55.0 | 12.5700 | 3.9 | 0.30 | 0.40 | 13.37 | 0.3 |
|  | TC1-1-16& TC1-1-17 | 1513.08 | 55 | 55.0 | 3.5800 | 3.5 | 0.30 | 0.40 | 4.38 | 0.3 |
|  | TC1-1-18 & TC1-1-19 | 1618.27 | 55 | 55.0 | 7.7800 | 4.1 | 0.30 | 0.40 | 8.58 | 0.3 |
|  | TC1-1-20 &TC1-1-21 | 1851.28 | 55 | 55.0 | 6.9900 | 5.2 | 0.30 | 0.40 | 7.79 | 0.3 |
|  | TC1-1-22 | 2034.03 | 55 | 55.0 | 2.3200 | 0.0 | 0.30 | 0.40 | 3.12 | 0.3 |
|  | TC1-1-23 &TC1-1-24 | 2129.21 | 55 | 55.0 | 4.1100 | 3.4 | 0.30 | 0.40 | 4.91 | 0.3 |

### Chute structure

Chutes are inclined drops which are used to carry water to a lower elevation. When there is more than three meters of fall in the ground sloping condition, chutes are provided. (Design guide line on Irrigation structures, 2002)

For Challie irrigation project, five chutes are provided on secondary canal for the drop height of 5.01m, 8.6m, 7.6m, 5.8m and 5.56m. All chutes are rectangular in cross section. The chute slopes are in the range of 9:1 to 24:1 according to topography of the terrain. According to their velocity, the chute beds are constructed from mass concrete (C-15) and banks from masonry.

The hydraulic parameters of the chutes are summerized in table. For detail information see excell analysis

Figure 4-3: Typical chute section

Table 4-6: Summery of Hydraulic parameters of chutes

| Design parameters | Formula | Chute 1 | Chute 2 | Chute 3 | Chute 4 | Chute 5 |
| --- | --- | --- | --- | --- | --- | --- |
| Design discharge, Q (m3/sec) | input data | 0.177 | 0.177 | 0.177 | 0.177 | 0.177 |
| Drop height, h(m) | input data | 5.01 | 8.6 | 7.61 | 5.84 | 5.56 |
| Length of chute, L (m) | input data | 70.2 | 129.35 | 110.32 | 140.26 | 50.09 |
| Width of drop ,bc | bc>=0.3m=0.734*Q/(d1.5) | 2.04 | 2.04 | 1.86 | 1.46 | 1.79 |
| Unit discharge, q= | q=Q/bc (m3/s/m) | 0.09 | 0.09 | 0.1 | 0.12 | 0.03 |
| Critical depth, dc | dc=(q2/g)(1/3) (m) | 0.09 | 0.09 | 0.1 | 0.11 | 0.05 |
| Critical velocity, vc | vc= (q/dc) | 0.95 | 0.95 | 0.98 | 1.06 | 33.9 |
| Velocity head, hvc | hvc=Vc2/2g | 0.05 | 0.05 | 0.05 | 0.06 | 58.4 |
| Water area, Ac | Ac=bc*d1 | 0.33 | 0.33 | 0.32 | 0.29 | 2.2 |
| Wetted perimeter, WP | Wp=bc+2d1 | 2.36 | 2.36 | 2.2 | 1.86 | 4.3 |
| Hydraulic radius, RC | Rc=Ac/wp | 0.14 | 0.14 | 0.14 | 0.16 | 0.5 |
| Water surface slope, Ic | Ic=((n*Vc)/(R(2/3)))2 | 0.004 | 0.004 | 0.004 | 0.004 | 0.004 |
| Energy at C, EC | Ec=Z+dc+hvc | 5.15 | 8.73 | 7.75 | 6.02 | 58.8 |
| Froud nr, fr | fr=V1/(g*d1)0.5 | 2.95 | 2.95 | 2.69 | 2.11 | 2.6 |
| Conjugate depth, d2 | d2=d1/2*((1+8*fr2)0.5-1) | 0.59 | 0.59 | 0.57 | 0.51 | 0.26 |
| Settling basin length, L (m) | L=5*d2 | 2.96 | 2.9 | 2.84 | 2.53 | 1.28 |
| Settling basin width, B (m) | B=18.46*Q0.5/(Q+9.91) | 0.77 | 0.77 | 0.77 | 0.77 | 0.44 |

### Drop structure

A canal drop is a structure in a canal to lower down the bed level and water level of the canal. It is required when the natural ground slope is steeper than the slope of the canal bed.

According to the slope of the terrain in main and secondary canals of Challie irrigation project 14 vertical drops have been provided between 0.5 to 0.8m height. But, from these drops the first drop (which is found at 4283.37m) is provided even the canal bed level is below the OGL. WHY? Because to lower down the canal top bank level than  the road level and construct a culvert at the existing URAP road path way crossing to maintain the communication.

The hydraulic parameters of vertical drops are summarized in table 4.7. For detail information see excell analysis.

Figure 4-4: Typical drop structure section

Table 4-7: Summery of Hydraulic parameters of Division Boxes

| Drops On | Chain age | Q (m3/s) | V (m/s) | b (m) | H (m) | d (m) | h (m) | b1 (m) | q (m3/s/m) | dc (m) | L (m) | B (m) | t(m) | a(m) | bc (m) | Lup (m) | Ldp (m) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| MC | 4+283.37 | 0.280 | 0.74 | 0.65 | 0.62 | 0.59 | 0.80 | 0.40 | 0.66 | 0.35 | 1.62 | 0.96 | 0.40 | 0.4 | 0.4 | 1.99 | 1.99 |
|  | 4+323.43 | 0.280 | 0.74 | 0.65 | 0.62 | 0.59 | 0.60 | 0.40 | 0.66 | 0.35 | 1.52 | 0.96 | 0.40 | 0.3 | 0.4 | 1.99 | 1.99 |
| SC | 0+060.17 | 0.177 | 3.56 | 0.30 | 0.82 | 0.17 | 0.50 | 0.40 | 0.59 | 0.33 | 1.39 | 0.77 | 0.40 | 0.4 | 0.3 | 1.83 | 1.83 |
|  | 0+696.6 | 0.069 | 0.94 | 0.30 | 0.30 | 0.25 | 0.50 | 0.40 | 0.22 | 0.17 | 0.84 | 0.49 | 0.40 | 0.2 | 0.3 | 1.59 | 1.59 |
|  | 0+771.73 | 0.069 | 1.00 | 0.30 | 0.28 | 0.23 | 0.50 | 0.40 | 0.20 | 0.16 | 0.82 | 0.49 | 0.40 | 0.2 | 0.3 | 1.59 | 1.59 |
|  | 0+831.84 | 0.069 | 2.10 | 0.30 | 0.34 | 0.12 | 0.50 | 0.40 | 0.23 | 0.18 | 0.86 | 0.49 | 0.40 | 0.3 | 0.3 | 1.59 | 1.59 |
|  | 1+102.34 | 0.069 | 0.49 | 0.30 | 0.47 | 0.46 | 0.50 | 0.40 | 0.23 | 0.18 | 0.86 | 0.49 | 0.40 | 0.2 | 0.3 | 1.59 | 1.59 |
|  | 1+257.62 | 0.069 | 0.49 | 0.30 | 0.47 | 0.46 | 0.60 | 0.40 | 0.23 | 0.18 | 0.92 | 0.49 | 0.40 | 0.3 | 0.3 | 1.59 | 1.59 |
|  | 1+362.81 | 0.055 | 1.67 | 0.30 | 0.26 | 0.12 | 0.50 | 0.40 | 0.18 | 0.15 | 0.78 | 0.43 | 0.40 | 0.2 | 0.3 | 1.55 | 1.55 |
|  | 1+478.01 | 0.055 | 1.36 | 0.30 | 0.23 | 0.14 | 0.50 | 0.40 | 0.15 | 0.13 | 0.73 | 0.43 | 0.40 | 0.2 | 0.4 | 1.55 | 1.55 |
|  | 1+568.18 | 0.055 | 1.73 | 0.30 | 0.26 | 0.11 | 0.50 | 0.40 | 0.18 | 0.15 | 0.78 | 0.43 | 0.40 | 0.2 | 0.3 | 1.55 | 1.55 |
|  | 1+733.48 | 0.055 | 1.28 | 0.30 | 0.23 | 0.15 | 0.50 | 0.40 | 0.15 | 0.13 | 0.73 | 0.43 | 0.40 | 0.2 | 0.4 | 1.55 | 1.55 |
|  | 1+813.63 | 0.055 | 1.49 | 0.30 | 0.24 | 0.13 | 0.50 | 0.40 | 0.16 | 0.14 | 0.75 | 0.43 | 0.40 | 0.2 | 0.3 | 1.55 | 1.55 |
|  | 2+029.03 | 0.055 | 1.05 | 0.30 | 0.24 | 0.18 | 0.60 | 0.40 | 0.16 | 0.14 | 0.79 | 0.43 | 0.40 | 0.2 | 0.4 | 1.55 | 1.55 |

### Road crossing structure

A box culvert is located at existing URAP road pathway crossing to maintain the communication. The wall of the culvert is designed to be masonry and the slab is from Reinforced concrete with 12mm @ 150mm C/C double reinforcement. The length of RC slab is 1.25m and its width and thickness is 6m & 30 cm respectively.

In addition, two foot path structures are provided at the existing foot paths. These structures are designed to be constructed from Reinforced concrete with 12mm @300mm C/C single reinforcement. These have 2m length and 1.25m width.

Table 4-8: Irrigation Infrastructure Bill of Quantities and Cost Estimate

| Bill No.3 Infrastructure |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Sr. No | Description | Unit | Qty | Unit price (Birr) | Amount (Birr) |
| 3 | Infrastructure |  |  |  |  |
| 3.1 | Main Canal ( Lined) |  |  |  |  |
| 3.1.1 | Excavation of ordinary soil | m3 | 4,624.20 | 116.26 | 537,609.49 |
| 3.1.2 | Excavation in weathered rock | m3 | 1.00 | 370.21 | 370.21 |
| 3.1.3 | Excavation in medium rock | m3 | 1.00 | 590.02 | 590.02 |
| 3.1.4 | Hard Rock Excavation | m3 | 706.33 | 1,026.12 | 724,779.34 |
| 3.1.5 | Backfill and compaction | m3 | 1,200.00 | 152.62 | 183,144.00 |
| 3.1.6 | Masonry work(1:3 ratio) | m3 | 3,633.60 | 1,520.00 | 5,523,072.00 |
| 3.1.7 | Plastering (1:2 Ratio) | m2 | 9,690.25 | 145.58 | 1,410,706.60 |
| 3.1.8 | C-20 concrete (1:1 1/2:3 ) | m3 | 25.54 | 2,750.00 | 70,235.00 |
| 3.1.9 | Reinforcement bar, φ12 including bending and cutting | Kg | 3,968.83 | 75.25 | 298,654.46 |
|  | Sub Total |  |  |  | 8,749,161.11 |
| 3.2 | Secondary Canal (lined) |  |  |  |  |
| 3.2.1 | Excavation of ordinary soil | m3 | 1,535.53 | 116.26 | 178,520.72 |
| 3.2.2 | Backfilling and compaction | m3 | 538.32 | 152.62 | 82,158.40 |
| 3.2.3 | Masonry work(1:3 ratio) | m3 | 1,036.86 | 1,520.00 | 1,576,027.20 |
| 3.2.4 | Plastering (1:2 Ratio) | m2 | 2,454.24 | 145.58 | 357,288.26 |
|  | Sub Total |  |  |  | 0.00 |
| 3.3 | Tertiary Canal (For a total of 27 unlined canals) |  |  |  | 0.00 |
| 3.3.1 | Excavation of ordinary soil | m3 | 2,445.92 | 116.26 | 284,362.66 |
| 3.3.2 | Fill and compaction | m3 | 90.3 | 152.62 | 13,781.59 |
|  | Sub Total |  |  |  | 2,492,138.82 |
| 3.4 | Drainage Crossing structures |  |  |  |  |
| 3.4.1 | Flume1 @0+120m & 0+996 |  |  |  |  |
| 3.4.1.1 | Excavation of ordinary soil | m3 | 29.41 | 116.26 | 3,419.21 |
| 3.4.1.2 | Backfilling | m3 | 14.08 | 152.62 | 2,148.89 |
| 3.4.1.3 | Masonry work | m3 | 14.72 | 1,520.00 | 22,374.40 |
| 3.4.1.4 | Concrete C-20 (1:2:3) | m3 | 27.66 | 2,750.00 | 76,065.00 |
| 3.4.1.5 | C-25 concrete (1:1 1/2:3 ) | m3 | 1.84 | 2,802.17 | 5,155.99 |
| 3.4.1.6 | Reinforcement bar, φ16 including bending and cutting | Kg | 622.2 | 75.25 | 46,820.55 |
| 3.4.1.7 | Reinforcement bar, φ12 including bending and cutting | Kg | 91.6 | 75.25 | 6,892.90 |
| 3.4.1.8 | Reinforcement bar, φ10 including bending and cutting | Kg | 485.32 | 75.25 | 36,520.33 |
|  | Sub Total |  |  |  | 199,397.27 |
| 3.4.2 | Flume2 @1126.4m |  |  |  |  |
| 3.4.2.1 | Excavation of ordinary soil | m3 | 29.41 | 116.26 | 3,419.21 |
| 3.4.2.2 | Backfilling | m3 | 11.97 | 152.62 | 1,826.86 |
| 3.4.2.3 | Masonry work | m3 | 16.8 | 1,520.00 | 25,536.00 |
| 3.4.2.4 | Concrete C-20 (1:2:3) | m3 | 2.72 | 2,750.00 | 7,480.00 |
| 3.4.2.5 | C-25 concrete (1:1 1/2:3 ) | m3 | 2.72 | 2,802.17 | 7,621.90 |
| 3.4.2.6 | Reinforcement bar, φ16 including bending and cutting | Kg | 793.33 | 75.25 | 59,698.08 |
| 3.4.2.7 | Reinforcement bar, φ12 including bending and cutting | Kg | 137.5 | 75.25 | 10,346.88 |
| 3.4.2.8 | Reinforcement bar, φ10 including bending and cutting | Kg | 652.19 | 75.25 | 49,077.30 |
|  | Sub Total |  |  |  | 165,006.23 |
| 3.4.3 | flume3@ 2502.78m |  |  |  |  |
| 3.4.3.1 | Excavation of ordinary soil | m3 | 29.41 | 116.26 | 3,419.21 |
| 3.4.3.2 | Backfilling | m3 | 11.89 | 152.62 | 1,814.65 |
| 3.4.3.3 | Masonry work | m3 | 2.72 | 1,520.00 | 4,134.40 |
| 3.4.3.4 | Concrete C-20 (1:2:3) | m3 | 28.04 | 2,750.00 | 77,110.00 |
| 3.4.3.5 | C-25 concrete (1:1 1/2:3 ) | m3 | 2.08 | 2,802.17 | 5,828.51 |
| 3.4.3.6 | Reinforcement bar, φ16 including bending and cutting | Kg | 566.85 | 75.25 | 42,655.46 |
| 3.4.3.7 | Reinforcement bar, φ12 including bending and cutting | Kg | 91.6 | 75.25 | 6,892.90 |
| 3.4.3.8 | Reinforcement bar, φ10 including bending and cutting | Kg | 459.63 | 75.25 | 34,587.16 |
|  | Sub Total |  |  |  | 176,442.29 |
| 3.4.4 | flume4 @2910.14m |  |  |  |  |
| 3.4.4.1 | Excavation of ordinary soil | m3 | 29.41 | 116.26 | 3,419.21 |
| 3.4.4.2 | Backfilling | m3 | 13.41 | 152.62 | 2,046.63 |
| 3.4.4.3 | Masonry work | m3 | 15.36 | 1,520.00 | 23,347.20 |
| 3.4.4.4 | Concrete C-20 (1:2:3) | m3 | 6.89 | 2,750.00 | 18,947.50 |
| 3.4.4.5 | C-25 concrete (1:1 1/2:3 ) | m3 | 1.76 | 2,802.17 | 4,931.82 |
| 3.4.4.6 | Reinforcement bar, φ16 including bending and cutting | Kg | 496.33 | 75.25 | 37,348.83 |
| 3.4.4.7 | Reinforcement bar, φ12 including bending and cutting | Kg | 91.6 | 75.25 | 6,892.90 |
| 3.4.4.8 | Reinforcement bar, φ10 including bending and cutting | Kg | 407.88 | 75.25 | 30,692.97 |
|  | Sub Total |  |  |  | 127,627.06 |
| 3.4.5 | flume5 @3070.47m |  |  |  |  |
| 3.4.5.1 | Excavation of ordinary soil | m3 | 29.41 | 116.26 | 3419.2066 |
| 3.4.5.2 | Backfilling | m3 | 13.41 | 152.62 | 2046.6342 |
| 3.4.5.3 | Masonry work | m3 | 15.36 | 1,520.00 | 23347.2 |
| 3.4.5.4 | Concrete C-20 (1:2:3) | m3 | 27.7 | 2,750.00 | 76175 |
| 3.4.5.5 | C-25 concrete (1:1 1/2:3 ) | m3 | 1.84 | 2,802.17 | 5155.9928 |
| 3.4.5.6 | Reinforcement bar, φ16 including bending and cutting | Kg | 550.64 | 75.25 | 41435.66 |
| 3.4.5.7 | Reinforcement bar, φ12 including bending and cutting | Kg | 91.6 | 75.25 | 6892.9 |
| 3.4.5.8 | Reinforcement bar, φ10 including bending and cutting | Kg | 453.77 | 75.25 | 34146.1925 |
|  | Sub Total |  |  |  | 192618.7861 |
| 3.4.6 | flume6 @3548.17m |  |  |  |  |
| 3.4.6.1 | Excavation of ordinary soil | m3 | 29.41 | 116.26 | 3419.2066 |
| 3.4.6.2 | Backfilling | m3 | 13.31 | 152.62 | 2031.3722 |
| 3.4.6.3 | Masonry work | m3 | 15.36 | 1,520.00 | 23347.2 |
| 3.4.6.4 | Concrete C-20 (1:2:3) | m3 | 9.58 | 2,750.00 | 26345 |
| 3.4.6.5 | C-25 concrete (1:1 1/2:3 ) | m3 | 2.51 | 2,802.17 | 7033.4467 |
| 3.4.6.6 | Reinforcement bar, φ16 including bending and cutting | Kg | 675.25 | 75.25 | 50812.5625 |
| 3.4.6.7 | Reinforcement bar, φ12 including bending and cutting | Kg | 91.6 | 75.25 | 6892.9 |
| 3.4.6.8 | Reinforcement bar, φ10 including bending and cutting | Kg | 541.38 | 75.25 | 40738.845 |
|  | Sub Total |  |  |  | 160620.533 |
| 3.4.7 | Super passage @4171.39m |  |  |  |  |
| 3.4.7.1 | Excavation of ordinary soil | m3 | 8.4 | 116.26 | 976.584 |
| 3.4.7.2 | Backfilling | m3 | 2.1 | 152.62 | 320.502 |
| 3.4.7.3 | Masonry work | m3 | 4.78 | 1,520.00 | 7265.6 |
| 3.4.7.4 | Concrete C-20 (1:2:3) | m3 | 1.13 | 2,750.00 | 3107.5 |
| 3.4.7.5 | Reinforcement bar, φ10 including bending and cutting | Kg | 44.71 | 75.25 | 3364.4275 |
| 3.4.7.6 | Reinforcement bar, φ14 including bending and cutting | Kg | 54.15 | 75.25 | 4074.7875 |
|  | Sub Total |  |  |  | 19109.401 |
| 3.5 | ROAD CROSSING @ 0+25 |  |  |  |  |
| 3.5.1 | Wearthed Rock excavation | m3 | 23.85 | 583.04 | 13905.504 |
| 3.5.2 | Lean Concrete C-5(1:4:8) | m2 | 13.5 | 1,277.05 | 17240.175 |
| 3.5.3 | Masonry work(1:3 ratio) | m3 | 13.08 | 1,520.00 | 19881.6 |
| 3.5.4 | Reinforcement bar, φ16 including bending and cutting | Kg | 370.41 | 75.25 | 27873.3525 |
| 3.5.5 | C-20 concrete(1:2:3) | m3 | 2.27 | 2,750.00 | 6242.5 |
| 3.5.6 | Back fill and compaction | m3 | 5.04 | 152.62 | 769.2048 |
|  | Sub Total |  |  |  | 85912.3363 |
| 3.6 | foot paths |  |  |  |  |
| 3.6.1 | C-20 concrete(1:2:3) | m3 | 0.6 | 2,750.00 | 1650 |
| 3.6.2 | Reinforcement bar, φ12 including bending and cutting | kg | 18.01 | 75.25 | 1355.2525 |
|  | Sub Total |  |  |  | 3005.2525 |
| 3.7 | Division Box ( for a total of 17) |  |  |  |  |
| 3.7.1 | Masonry work(1:3 ratio) | m3 | 21.36 | 1,520.00 | 32467.2 |
| 3.7.2 | Plastering (1:2 Ratio) | m2 | 29.82 | 145.58 | 4341.1956 |
| 3.7.3 | Cemented stone pitching | m3 | 11.71 | 1,289.62 | 15101.4502 |
| 3.7.4 | Gates suppy, cutting, welding and installation (for a total of 27 gates) as per the drawings |  |  |  |  |
|  | 3mm thick sheet metal | pcs | 27 | 2,964.23 | 80034.21 |
|  | Angle iron for groove (40mm*40mm*4mm) |  |  |  |  |
|  | Dia 16mm reinforcemnt handle bar including bending & fixing |  |  |  |  |
|  | Sub Total |  |  |  | 131944.0558 |
| 3.8 | Drop (for a total of 10) |  |  |  |  |
| 3.8.1 | Excavation of ordinary soil | m3 | 74.89 | 116.26 | 8706.7114 |
| 3.8.2 | Masonry work(1:3 ratio) | m3 | 12.35 | 1,520.00 | 18772 |
| 3.8.3 | Plastering (1:2 Ratio) | m2 | 30.08 | 145.58 | 4379.0464 |
| 3.8.4 | Cemented stone pitching | m3 | 32.77 | 1,289.62 | 42260.8474 |
| 3.8.5 | Lean Concrete C-5(1:4:8) | m3 | 34.07 | 1,277.05 | 43509.0935 |
|  | Sub Total |  |  |  | 117627.6987 |
| 3.9 | Chute (for a total of 5) |  |  |  |  |
| 3.9.1 | Excavation of ordinary soil | m3 | 95 | 116.26 | 11044.7 |
| 3.9.2 | Masonry work(1:3 ratio) | m3 | 116.16 | 1,520.00 | 176563.2 |
| 3.9.3 | C-20 concrete(1:2:3) | m3 | 66.31 | 2,485.29 | 164799.5799 |
| 3.9.4 | Plastering (1:2 Ratio) | m2 | 382.95 | 111.54 | 42714.243 |
|  | Sub Total |  |  |  | 395121.7229 |
| 3.1 | Turnout (For a total of 72) |  |  |  |  |
| 3.10.1 | Excavation of ordinary soil | m3 | 85.84 | 116.26 | 9979.7584 |
| 3.10.2 | Masonry work(1:3 ratio) | m3 | 85.55 | 1,520.00 | 130036 |
| 3.10.3 | Plastering (1:2 Ratio) | m2 | 494.56 | 145.58 | 71998.0448 |
| 3.10.4 | Cemented stone pitching | m3 | 234.16 | 1,289.62 | 301977.4192 |
| 3.10.5 | Gates (for a total of 72 gates) | pcs | 72 | 2,550.00 | 183,600.00 |
|  | 3mm thick sheet metal |  |  |  |  |
|  | Agle iron for groove (40*40*4) |  |  |  |  |
|  | Dia 16mm reinforcemnt handle bar including bending & fixing |  |  |  |  |
|  | Sub Total |  |  |  | 697,591.22 |
| Total cost for Infrastructures |  |  |  |  | 13,713,323.79 |

| Challi Irrigation Project Summary of Bills |  |  |
| --- | --- | --- |
| Bill No | Description | Cost |
| I | General items | 2,909,110.46 |
| II | Head work | 3,500,656.29 |
| III | Infrastructure | 13,713,323.79 |
|  | Total | 20,123,090.54 |
|  | 15 % VAT | 3,018,463.58 |
|  | Grand Total | 23,141,554.12 |
|  | Command area, ha | 163 |
|  | Cost per ha | 123,454.54 |

# OPERATION AND MAINTENANCE

## General

The main objective of the operation and maintenance aspect of an irrigation scheme is to facilitate the timely delivery of the required irrigation water to farms and to keep the irrigation system in an optimum operating condition.  This section therefore, discusses the main functions of the subject matter under consideration for the scheme.

## Irrigation System Operation

### Method of operation

The farmers would organize themselves and form groups (water user association-WUA) in order to handle the water management.  Rotational water distribution would be applied within the group.  The rotational distribution is then to distribute water by turn to the whole scheme according to the timely need of crop water requirement. For better and efficient water management, crop diversification should be avoided within a group.  This would reduce the complexity of water distribution system of the scheme during one irrigation season. At farmers’ level of operation, a constant flow and variable irrigation time is advisable.

To ensure equitable distribution of water the amount of water released through field canals should be proportional to the available command area. .

In order to minimize the water losses during conveyance and application:

  - Water users association (WUA) should assign gate keeper who is responsible to control unnecessary gate opening, damage and losses through gates.

  - Damages on the canal system, and gates should be followed by the WUA and immediate action should be undertaken.

  - For technical failure beyond the capacity of the local community it should be informed to the Woreda Irrigation and Drainage sector and immediate action should be taken.

  - Frequent supervision to the canal system is needed. Remove silt, weed, and any flow blocking objects from the canal system.

  - Water application should be done by furrow rather than flooding.

  - Uniform cropping season and pattern within nearby blocks is good for irrigation water management.

  - Water application should be based on the proposed irrigation hour and rotation. This will reduce water application losses.

### Methods of Maintenance

The maintenance tasks are categorized into two types (i.e., routine activities, and repairs). The routine maintenance activities that are carried out periodically include:

- Regular cleaning of sediments and weeds from canals and drains;

- Inspection and lubrication of gates; and

- Maintenance of cracked regulating and control structures.

Repair works include task carried out more frequently and quickly, and include those task that are generally unpredictable.  They also include emergency works.  The activities included in this category are:

- Repairing overtopped or breached canals

- Repairing jammed gates.

Regular inspection of the irrigation facilities should be carried out as part of the maintenance activities.  These tasks could be carried out immediately after the end of the main rains in September and during the rainy season.  This could concentrate on the interceptor drains and the flood protection works, and the main canal. This could be important especially if supplementary water is needed late rainy season for some crops. The inspection of other works like the water control and regulating structures could be carried out as part of routine operation activities.

# CONCLUSION AND RECOMMENDATIONS

## Conclusions

- The infrastructure of this project area is designed to irrigate about 165 ha of land by taking its supply from the Challie diversion irrigation project. The maximum duty of the command area for 14 hours per day irrigation with overall project efficiency of 58%.  The method of irrigation of the project area is furrow surface irrigation.

- The main and secondary canal system is designed to be lined and tertiary canals are earthen. Field canals to be constructed by farmers.

- Lined canals have rectangular section and earthen ones are trapezoidal.

The per hectare cost of the project is 148,707.8 Birr and the total project cost is 24,536,779.31

Birr without VAT.

- As the left side of the river bank is rock formation overtopping is allowed in the canal and gates are to be installed after 111.7m length of the canal. This 111.7m length of the canal is designed to be reinforced concrete.

- The design point rainfall of the project is determined using Gumble Powell method, and the designed flood is determined by SCS-CN method & flood mark method. Finally SCS-CN method is adopted.

- As there is no flow data for the river/stream near the project. Therefore, data for the hydro-meteorological analysis is taken from the nearby station called Haik station.

- The design of the canal dimensions of the irrigation canal is done by applying the manning’s uniform flow equation.

## Recommendations:

- Since, for the first 111.7m of the canal overtopping of the river water is allowed. So, this 111.7m of the canal length requires a    continuous clearing.

- For better performance and long service year of the project regular inspection and maintenance is highly required.

- Farmers training, how to operate and maintain the project structures as a whole and available and water resources has a paramount important.

- Close supervision of the construction should be made to modify (if need be) each Components of irrigation system based on specific site conditions.

# REFERENCES

- Design Guide Line On Irrigation Structures. (2002). Addis Ababa, Ethiopia: The Federal Democratic Republic Of Ethiopia Ministry Of Water Resources.

- Design Guide Line On Irrigation System. (2002). Addis Ababa: The Federal Democratic Republic Of Ethiopia Minstry Of Water Resources.

- IDD manual

- K.S. ARORA