OROMINA IRRIGATION DEVELOPMENT AUTHORITY (OIDA)

HORO GUDRU WELLEGA IRRIGATION DEVELOPMENT AUTHORITY

FEASIBILITY STUDY AND DETAIL DESIGN OF

DILDILA ABA OROMO SMALL-SCALE IRRIGATION PROJECT

REVIWED BY OIDA-AGP TEAM IN CONSULTATION WITH BALA ZONE IRRIGATION

DEVELOPMENT AUTHORITY

OCTOBER, 2016

BATU

1. SOCIO-ECONOMIC STUDY	4

1.1 Objective of Socio-Economic study	2

1.2 Scope	2

1.3 Methodology and Approaches	2

1.4 Existing Situation Assessments of the project Area	4

1.3.1Head Work Area Kebeles	5

1.3.2 Demographic features of the project area	6

1.3.3	 Irrigation and Crop Production	12

1.3.4 RESOURCE OWNERSHIP	17

1.3.5Water Supply	24

1.3.6Gender issue	33

1.3.7Irrigation Organization Situation (WUA's)	37

1.3.8 COMMUNITY PARTICIPATION	42

2. GENERAL SOIL Description	48

2.1. Methodology	48

2.2 Specific Observation	48

2.3 Summary of physical Soil properties	50

2.4 Chemical Analysis	51

2.5. Producing soil Map Unit	53

3. Agronomy	56

3.1. Existing Agriculture	57

3.2. Potential and actual average yield	59

3.3. Existing Irrigation Practices	60

3.4 Livestock, production	62

3.5Proposed crops for the project	64

3.6. Proposed cropping pattern	64

3.7. Proposed crop and cropping calendar	65

3.9. Net and gross irrigation requirements.	74

3.10. Crop System and Management	77

3.11 Cultivation and plant protection	78

3.12.Plant protection	78

3.13. Irrigation Practices and Harvesting	81

3.14. AGRICULTURAL SUPPORTING SURVICES	83

4.1Geological Study	85

4.2 Objective of the geological and geotechnical investigations	86

4.4. GENERAL Geological Setup	88

4.5. Hydro geological Condition of the Catchment Area	89

4.6. Engineering Geology of the proposed Headwork site	90

4.8. Availability of Natural Construction Material	92

5. Hydrology	95

5.1 Description of the Project Area	95

5.2 Objective of the study	96

5.3 Methodology and Materials used	96

5.4 SCOPE	97

5.5 BASE FLOW [THE STREAM MINIMUM FLOW] MEASUREMENT	103

5.6 Irrigation demand and potential	105

5.7 Water Planning	105

5.9 DESIGN FLOOD ANALYSIS	107

5.10 THE PEAK FLOW ANALYSIS	111

5.11Sediment Transport & water quality	117

6.HEAD WORK, IRRIGATION AND DRAINAGE SYSTEM DESIGN	120

6.1 Hydraulic and structural design of Head work	123

6.2. weir Design	123

6.3. Estimationof tail water depth	123

6.4River Discharge Curve Calculation	125

6.5 Determination of weir crest level	126

6.6. Determination of stilling basin bed level	130

6.7Stability analysis of weir	135

6.8 Stability analysis of Wing wall Bod	140

6.9 Design of Intake and Scouring Sluice	144

6.10. Design Of Canal And Related Structure	146

6.11 Irrigation System Design	147

6.13Turnout Structure	152

6.14 Design of RCC Flume	156

6.15.Design of pond storage	167

6.16 Chute structure	170

6.17.Drop strcture	173

6.18 Division Box	175

6.19 Drainage pipe Culvert	177

6.20 Road crossing box culvert	179

6.21.Design of foot bridge	184

6.22. Bill of Quantity Of Dildila Abba Oromo	186

7. Watershed Management project	191

7.1 . Watershed Description	191

7.2 Land Use and Cover	193

7.3 Watershed problems	195

7.3 Management programs for erosion Hazards	198

8. ENVIROMENTAL IMPACT ASSESSMENT	201

8.1 Methodologies	201

8.2 MAJOR EXISTING ENVIRONMENTAL CONSTRAINTS	201

8.3 Environmental mitigation measures	204

8.4  Mitigation measures for water supply & sanitation	204

8.5 Conclusion and recommendation	205

9. FINANCIAL ANALYSIS OF DILDILA ABBA OROMO PROJECT	206

9.1 Introduction	206

9.3 Farm Income Analysis	207

9.4 Financial price of Inputs and Outputs	212

9.5 Project Cost and Return	216

10. Economic Analysis	220

10.1  Introduction	220

10.2 Purpose of Economic Analysis	221

10.3  Theoretical Basis of Economic valuation	223

10.4 SENSITIVITY ANALYSIS	232

Annexes	265

Community Petions	265

1. SOCIO-ECONOMIC STUDY

GENERAL BACKGROUND

Dildila aba Oromo SSI project is situated in Oromia regional state, Horo Guduru Wellegga Zone of Jimma Rare district, Bada Warke and Gudata Doby kebeles.

The project site is located at a distance of 233 km from the regional state, Addis Ababa of which 98 km from Zone capital (Shambu to Wayu), 17 km from district capital, Wayu and 216 km (Wayu to Addis Abeba). The accessible road from zone to district is all weathered road while from district to the project site is dry weather road.

The land use pattern of the district is highly comprises intensive cultivated land. The intensively cultivated land consists of peasant mixed farming. The largest percentage of its total area is devoted to grain productions via non-mechanized or traditional farming system. Socio-economic assessment has taken as one of the basic part of the proposed feasibility study. The basic aim of this socio-economic feasibility study is to assess and analyze household survey primary and secondary data within the Kebeles and district documented materials to establish the acceptability of the project for funding.

As a result, Socio-economic surveys has conducted in the project area because; irrigation is not only a simply technical task of diverting and delivering water to crops or seek developments but also it needs a human activity, acceptability and availability of positive attitude towards to the project. Accordingly, socio-economic factors like social group, association, settlement pattern, resource ownership (land use, housing, and livestock and farm equipment), educational level, family labor, irrigation and crop production practices etc analysis have incorporated in this study.

The study deals directly with the people and area in which the project is located, to:-

forecast how to optimize social benefit from the implementation of the expected  project, analyze the existing and the projected socio- economic conditions that needs to be included in to the project, discusses the potential positive and negative project impacts in line with remedial measures, identification of stocks holders, opportunities and constraints that enable to run the project in coordinated ways. This undertaking study generally follows certain applicable methodologies and procedures.   Generally, the socio-economic condition of the project area study is suitable to undertake this developmental activity. Due to these facts, it is very crucial as well as recommendable to implement the proposed small-scale irrigation project to ensure sustainable development for the target project area particularly and for the country generally.

1.1 Objective of Socio-Economic study

General objective

The overall objective of the study is to assess the general existing and proposed socio-economic condition of the project area.

Specific objectives

The specific objective of this investigation is -

To identify the project beneficiaries population and households of the area

To assess the economic activities being practiced in the project area

To analyzing the existing and the projected demographic features and household characteristics of Socio- economic condition of the project area

To assess the existing major social, economic and institutional issues and constraints  that hinders the successful implementation of the proposed project

To assess the potential positive and negative impact which comes from the project

To provide recommendation for sustainable use of the project

1.2 Scope

There is a need to carry out feasibility study and detail design for construction of irrigation projects. Therefore, the scope of socioeconomic study is limited between assessing the existing and the proposed socioeconomic situation, in line with potentials, constraints and realizes the project study results in favor of the desired social, economic and institutional objectives.

1.3 Methodology and Approaches

Methods of data collection

In this study, both primary and secondary data have utilized. To obtain primary data, both closed and open-ended questionnaires on socio-economic, institutional and other related

variables that are supposed to be important for the study has developed. Primary data have collected from Bada Warke and Gudata Doby kebeles sample respondents through questionnaire using face-to-face interview of the sample households through enumerators. The numbers of enumerators were three, who speak the local language, who recruited from Jimma Rare Woreda, where the project is located and made familiar with the questions, methods of data collection and interviewing techniques. In order to obtain detail socioeconomic information about the project area, the appropriate data collection instruments employed were primary and secondary data collection methods.

Primary Data Collection

Primary data collection methods have applied through appropriate instrument prepared and tested, which is simple sample household survey method. In the household survey, first structured questionnaires were prepared in English and Afan Oromo version, in which detailed household characteristics and family size, economic base and occupation, resource ownership, farm size and income were included. Data collections have done via Structured and less structured methods.

The Structured methods include: Sample Surveys (sampling a representative share of household), Self-administered questionnaires,

The Less Structured methods include: Group discussions, Focus group interviews, Key informant interviews (KII), Key observant interviews, Field visits, Participant observation and measurements, Community interviews (General Meeting) and Conversation with stakeholders

To conduct the household survey, using simple random sampling method with a sample of 40 household were selected from  both kebeles expected beneficiaries and the household survey were carried out on face to face interview of these households.

Sampling

All farmers in the irrigation scheme have access and right to use irrigation water for crops production. In this study, a purposive sampling technique has used to select sample households. The household sample survey size would be determined based on the total number of household, the diversity of the population and the availability of local resource have collected and analyzed. Based on this reality the sample household size were taken as 40 farmers in the upstream, down and middle part of the project area Bada Warke and Gudata Doby kebeles were selected.

Table 1 Number of sample households

| Name of Scheme | Name of Kebeles | Command area Hectare | Total number of sampled HH |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Male hh | Female hhh | Total |
| Dindila aba Oromo | Bada Warke | 35 | 20 | 5 | 25 |
|  | Gudata Doby | 21.5 | 13 | 2 | 15 |
| Total |  | 56.5 | 33 | 7 | 40 |

HH = Household survey 2014

To supplement the primary data, personal observations of physical features, informal discussions with experienced farmers, Kebele officials, DAs and wereda experts of the project area have made.

Secondary Data collection

Secondary data is recorded data or information in the project area socio economic issues, parameters and variable data source. The data sources have gathered from Project area Specific socio economic information at kebele level Manager, development agency office/DAs, Woreda socio economic overview   and the respective woreda sector office. Therefore, Secondary data collection has made via qualitative and quantitative information methods and prepared the project for feasibility study of socioeconomic report.

Methods of Data Analysis

In this study, the Socio-Economic raw data were properly gathered and it has analyzed in order to get results, make conclusions and develop recommendations for the problem stated around the project area. The method that used to analyze the data is descriptive statistics through quantitative and qualitative analysis method.

Descriptive statistics

Descriptive statistics has used to explain the project area socio-economic, institutional and other characteristics of the sample households. These include mean, percentage; Sum, Range and frequency of occurrence for the sampled household survey irrigation water used.

Data presentation

Different primary and secondary data have collected in the feasibility study of Dindila aba Oromo irrigation projects. As the result, relevant and realistic data have collected and presented in the socioeconomic report of the project, depending on the type of qualitative and quantitative data analysis.Therefor; the data presentation of the project has conducted in Table and Graph.

Project rationale and Justification

At this moment, more of the farming activities in the area are uneven or untimed rain fed based agriculture with small portion of traditional irrigation practices.  The agricultural output practiced around the project area farmers were earned low production and productivity.   Dindila aba Oromo small-scale irrigation project has proposed to overcome from such problem and increase production and productivity of the target community by utilize the maximum potential of the resource (river). Therefore, the study of the project aimed to escape the expected farmers from being waiting seasonal rain fed for running sustainable development. Also it  eradicate gradually rural poverty level existing in the project area and bring dramatic social change in terms of living standard, give equal job opportunity for both sexes since it is labor intensive work due to these fact it is highly recommended to implement the proposed  small scale irrigation project.

1.4 Existing Situation Assessments of the project Area

The existing agricultural practice of the project area communities applied on traditional agriculture practice including crop production by annual rain fed and traditional irrigation and animal rearing which incorporates cattle, equines and ruminants are the dominant once. There is practice of traditional irrigation situation in the project area farmers through diverting Dindila aba Oromo River to increase crop production. The employment and the income of the indicated area is traditional farming extensively or widely and generate annual and seasonal income either from sales of crop production, livestock’s and from traditional irrigation development. The poverty status of the project area society has more or less forced to say very low, which means they have almost ensured their food security or they did not suffered as much by food insecurity problems.

Socio-Economic Profiles and Characteristics

Command Area Kebeles

The total command area of the proposed project has estimated to about 56.6hectares, which entirely falling in Jimma Rare Wereda, between Bada Warke and Gudata Doby kebeles, which is cultivated by rain fed, and traditional irrigation by the expected communities of the project area.

1.3.1Head Work Area Kebeles

Head work area of the propose project is totally located in  Jimma Rare Wereda,in Gudata Doby kebele to the right side of Bada Warke kebele.

Administrative Structure

The administrative structures of Dildila aba Oromo irrigation Project include Region, Zone, wereda, kebeles, villages and sub-villages.

Horo Guduru Wollega Zone

Horo Guduru Wellegga Zone has ample potential of irrigable land that would developed through Small-scale irrigation systems even if there is scarcity Water resource that used as weir Diversion or surface water and it is established independently from East Wollegga zone in 1998 and has 314 km distance from Addis Ababa, Capital city of Oromia regional state. It has nine rural districts and one zonal town. This zone has 186 administrative kebeles out of these, 172 and 14 are rural and town kebeles respectively. The zone has- estimated to cover 7,867.6 Square kilometer hectare of land area. The total population of the zone is estimated to 637,448, out of which 319,319 male and 318,129 female populations according to 2003 CSA made, when we projecte from 2003-2006 the number of zone population increased to 694,529.8 at this moment or currently.

Source:-  Zonal Finance and Economic development office.

Jimma Rare Wereda

Jimma Rare Woreda is one of the nine rural weredas of Horo Guduru Wollegga Zone, Oromia Regional State. The wereda has administratively sub-divided into 18 rural kebeles and 2 towns; its capital is Wayu town.  The wereda has estimated to cover 396.7 Squar km ha of land area. The current total rural population of the wereda is 71,723 out of this 35,503 are male and 36,220 are female populations. The wereda has demarcated by Guduru Wereda from Northern, Jimma Ganat from West, North Showa Zone from northeastern and Chaliya wereda from Southern direction. According to data obtained from the woreda, 45% and 55% of woreda is Tropical humid and high land area respectively. The rural populations of Jimma Rare Woreda are mostly practicing mixed farming, dominated by crop production followed by livestock rearing.

1.3.2 Demographic features of the project area

Beneficiaries of the project

There are direct and indirect beneficiaries from the expected project. Similarly, there is about 150 hhs from Bada warke kebele and 80 hhs from Gudata Doby kebele were practiced existing traditional irrigation by traditional diversion weir on Dindila aba Oromo river vastly. However, this Small-scale Irrigation Development project has a total population of 2708 out of this 1312 is male and 1396 is Female population at present time for both kebeles

Whereas there is 420 from Bada warke kebele 325 from Gudata Doby kebele expected household beneficiaries, totally 745 proposed beneficiaries are expected, of this 711 hhs are male family headed and 34 hhs are Female family headed from both kebeles directly.  The beneficiaries of the project area economy have based on - Animals raring and Crop production through traditional farming system

Direct beneficiaries

| No. | Description | The total population of both kebeles |  |  |
| --- | --- | --- | --- | --- |
|  |  | Bada Warke | Gudata Doby | Total |
| 1 | Households | 420 | 325 | 745 |
|  | Male | 395 | 316 | 711 |
|  | Female | 25 | 9 | 34 |
| 2 | Total population | 1098 | 1,610 | 2,708 |
|  | Male | 956 | 756 | 1312 |
|  | Female | 1012 | 854 | 1,396 |

Indirect beneficiaries

The indirect beneficiaries are those farmers whose parents had not farmlands inside the proposed command area and any individual who take party on the consumptions of project out puts that means Landless, women, children, adjacent urban and nearby kebeles population are the indirect beneficiary of the project. Therefore, indirect beneficiaries of the project who will get the project output from the access market have estimated to be 105,015 beneficiaries as the conducted data reviled or showed.

Population Composition

Population

Population composition of the project area have assessed in terms of sex (male, female), age, and ethnic groups. According to the data obtained from both kebeles administration office, the total populations of the existing and proposed project area kebeles were about 9,060, Out of the total population, 4, 725 are males while the remaining 4,335 of them are females.The family size of the household is medium even if there is low knowledge about family planning. The population size of the kebeles of the project site has shown as follows.

Table 3- Population of the project area kebeles.

| No | Description | The total population of both kebeles |  |  |
| --- | --- | --- | --- | --- |
|  |  | Bada Warke | Gudata Doby | Total |
| 1 | Total population | 5765 | 3295 | 9060 |
|  | Male | 2985 | 1740 | 4725 |
|  | Female | 2780 | 1555 | 4335 |
| 2 | Households | 793 | 523 | 1316 |
|  | Male | 697 | 448 | 1,145 |
|  | Female | 96 | 75 | 171 |

Source: Bada Warke & Gudata Doby kebeles administration office by 2014

Population distribution and projection

Population density in the rural setting is completely differs from that of urban setting because of large farmland ownership. As it is observed, density of population of the project area is appeared to be clustery populated. The underlying reason why the population concentration found to be cluster or grouped is that traditional farmland holding system from the ever beginning.

Population Density= Total population of the project area/Total delineated area of the project study =2,708/56.6=47.84 km3 that means one hectare net command area will service 47.84 population of the project area

Agricultural Density= Total Rural population of the project area/Total cultivated land area in km2=9060/2874=3.15 that means one hectare net cultivated land area will service 3.15 rural population of the project area

The greater part of the population of the project area is young population whose age group is falls between 19 and 65. From this, we have concluded that there is much more daily laborers that will have using during implementation of this project.

Population statistics had said the necessity issues in the planning and implementation of development projects. It is very important mainly to estimate the population size that has expected to benefit from the intended irrigation project, their willingness and estimate the available human resource to realize the envisaged or predicted project.

To estimate population projection of the direct and indirect beneficiary of Dindila aba Oromo SSI project, Taking fixed population growth rates (2.9%), for 20 years and applying geometric progression method of population forecasting, the population projection of the kebeles have summarized as follows:

The formula used for the population projection is:-

Pn = Po (1+r) n

Where      Pn     = Projected Population

Po    = Current Population

r     = Population growth rate

n    = Design period (20 years)

Table-4: Population Projections in and around the proposed project area (2006-2031)

| Description | 2006 | 2011 | 2016 | 2021 | 2026 | 2031 |
| --- | --- | --- | --- | --- | --- | --- |
| Growth rate (%) | - | 2.9 | 2.9 | 2.9 | 2.9 |  |
| Projected population | 6,845 | 7,896.79 | 9,110.19 | 10,510.03 | 12,124.98 | 13,988.07 |

According to the latest available CSA, population growth rate varies from year to year, between rural and urban areas, and between woredas. Urban population has a higher growth rate than rural population because of migration to the towns. Rural population projections for the project area have estimated in Table 4. As to the population projection of the project area has conducted, the projected population for 2006, 2011, 2016, 2021, 2026 and 2031 is about 7,897, 9,110, 10,510 and 12,125. The rapidly expanding population in the area is increasing pressure on existing agricultural land. Similarly, the increasing population will create great pressure on the existing social and institutional service. Thus for this matter the forecasted project is tangible solution to overcome the expected problem.

Household Characteristics

Family Size

Analyses of family sizes in the project area were the basic part of socio economic feasibility study. Accordingly, household survey has conducted in the command area of proposed project area. The total number of households (HH) selected for conducting sample household survey were 40 hhs (i.e. 25 households from Bada Warke and 15 households from Gudata Doby kebeles depend up on the majority of beneficiaries of the project.

As survey result indicates, 33 interviewed households are male and the rest 7 are female. The minimum and maximum the average age of the sample household is 19 and 75 years respectively. The average sampled household age is 40 years. As to the family size, the respondents have 240 total family sizes, out which 118 and 122 is male and female member of households respectively. It indicates that, there is proportional number of male and female family size has observed in the project area.  In addition, the minimum and maximum family sizes household is 1 and 11 respectively.

| Table.5 Distribution of Sample HH by Age, Family members, Active and non active labor force, Family member permanently work on farm, hhs farming experience and off farm income |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Items | No. of Resp. | Range | Minimum | Maximum | Sum | Mean | Std. Deviation |
| Age of the household | 40 | 56.00 | 19.00 | 65.00 | _____ | 39.8750 | 14.15154 |
| Total family members of the household | 40 | 10.00 | 1.00 | 11.00 | 218.00 | 5.4500 | 2.51100 |
| Number of Male household member | 40 | 5.00 | 1.00 | 6.00 | 113.00 | 2.8250 | 1.39390 |
| Number of Female household member | 40 | 6.00 | .00 | 6.00 | 103.0 | 2.5750 | 1.41217 |
| Economically active family members(age 15-65) | 40 | 6.00 | 1.00 | 7.00 | 127.0 | 3.1750 | 1.66237 |
| Economically Non active family members(age<15 or >65) | 40 | 5.00 | .00 | 5.00 | 90.00 | 2.2500 | 1.29595 |
| Number of family members permanently work on farm | 40 | 6.00 | 1.00 | 7.00 | 107.0 | 2.6750 | 1.22762 |
| Household farming experience | 40 | 51.00 | 9.00 | 60.00 | _______ | 26.0250 | 13.73091 |
| Off-farm income of the HH | 40 | 3000.0 | .00 | 3000.00 | 9390 | 2.3475 | 700.14280 |

Source: HH survey result, 2013

Household Labor

Household labor is one of the basic resources and inputs that are critically required by an irrigation project. According to sample survey result, of the total family sizes, economically active family sizes is 127and 90 is none active labor forces. This result indicates that, according to sample household heads survey conducted, there is higher economically active labor force in compare with economically non-active labor forces. The supply of family labor force or unskilled labor is highly available during slack period or at a time, they did not busy with farm work and at the peak period or during harvesting time the skilled labors force has employed for the project activities. The allocation of labor has intensively acted on farm work. Hired labor is rare in the study area even there are wealth households who can hire laborer whenever they like. Using oxen for cultivation is a prevailing or a widespread technology in the course of farming around the project area. June- August is the main rainfall season and sometimes begins in March and lasts to September. Within this period the larger part of crop production activities like part of clearing, first to fifth round tilling, sowing and weeding takes place in this months and harvesting takes place during the month of November to the end of December . In the peak months of agricultural activities (weeding and harvesting) all the labor of the family and additional labor mobilized in order to run these activities. Unskilled labor is available in and around the study area during slack period. Therefore, the careful selection of laborer is required to avoid uncertainty that might arise during labor requirement.

Table 6- indicates economically active and non-active family member labor around the project area.

| items | No. of Resp. | Range | Minimum | Maximum | Sum | Mean |
| --- | --- | --- | --- | --- | --- | --- |
| Economically active family members(age 15-65) | 40 | 6.00 | 1.00 | 7.00 | 127.00 | 3.2 |
| Economically Non active family members(age<15 or >65) | 40 | 5.00 | .00 | 5.00 | 90.00 | 2.3 |

Education Level

Education is the base for any social and economic development activities. Based on the household survey conducted, from the total sample households, 52.5% of households are literate, 7.5% read and write and 40% households are illiterate. This indicates that, households of the project area have been more educated. As the data conducted shows male households was more educated than female households.

Table .7 Distributions of household heads by educational status.

| No | Items | Numbers | Percent (%) | Remark |
| --- | --- | --- | --- | --- |
| 1 | Literate | 21 | 52.5 |  |
| 2 | Illiterate | 16 | 40 |  |
| 3 | Read and write | 3 | 7.5 |  |
|  | Total | 40 | 100.0 |  |

Source: HH survey result, 2014

The average education level of the expected household

| Item | No. of respondents | Range | Minimum | Maximum | Sum | Mean | Std. Deviation |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Literate Level of hhs formal education | 40 | 9.00 | .00 | 9.00 | ________ | 3.7000 | 3.18812 |

Source: HH survey result, 2014

Settlement Pattern

The beneficiary Peoples have settled or established in and around the project area permanently. Thus, the settlement patterns of the people in the project area are semi-populated type, there is township development tendency, and rural centers have observed during the feasibility study.

Ethnic Groups, Language and Religion

According to the socio-economic survey conducted, 100% the respondents are belonging to Oromo ethnic group. Similar to ethnic group of the sampled household, Afan Oromo is the largest first language, which isspeaking by the respondent’s project area. The Survey result also reveals that from the total sampled household, 25% of the household heads can speak Amharic language as second language. The religion of the project area is very Christian flowers that mean Orthodox and Protestant .The description of ethnic group and language of the headed household has analyzed in the table 5 below.

Table.9. Distribution of Sampled household head by Ethnic group and languages

| No | Items | No. of Respondents | Percent (%) | Remark |
| --- | --- | --- | --- | --- |
| 1 | Ethnic group |  |  |  |
| 1.1 | Oromo | 40 | 100 |  |
| 1.2 | Amhara | 0 | 0 |  |
| 1.3 | Tigre | 0 | 0 |  |
| Total |  | 40 | 100 |  |
| 2 | First Language of the house hold |  |  |  |
| 2.1 | Afan Oromo | 40 | 100 |  |
| 2.2 | Amharic | 0 | 0 |  |
| 2.3 | Tigre | 0 | 0 |  |
| Total |  | 40 | 100.0 |  |
| 3 | Second language of the house hold |  |  |  |
| 3.1 | Afan Oromo | 15 | 37.5 |  |
| 3.2 | Amharic | 25 | 52.5 |  |
| 3.4 | Households who have no second language |  | 15 |  |
| Total |  | 40 | 100.0 |  |

Source: HH survey result, 2014

Regarding to marital status, out of the total respondents, 85% were married and 2.5% were single or not married, 7.5% divorced and 5% is widowed. Regarding to religion of the sampled household heads, all the total respondents are Christian religion followers. It has analyzed in the table 6 below.

Table .10 Distributions of sample household heads by marital status and Religion.

| No | Items | No. of Resp. | Percent (%) | Remark |
| --- | --- | --- | --- | --- |
| 1 | Marital Status |  |  |  |
| 1.1 | Married | 34 | 85 |  |
| 1.2 | Single | 1 | 2.5 |  |
| 1.3 | Divorced | 3 | 7.5 |  |
| 1.4 | Widowed | 2 | 5 |  |
| Total |  | 40 | 100.0 |  |
| 2 | Religion |  |  |  |
| 2.1 | Protestant | 12 | 30 |  |
| 2.2 | Catholic | 0 |  |  |
| 2.3 | Orthodox | 28 | 70 |  |
| 2.4 | Muslim | 40 | 100.0 |  |
|  | Total | 40 | 100.0 | , |

Source: HH survey result, 2014

Economic Base and Occupation

The major means of Livelihood or economic source and the potential income generating of the communities in the project area are mainly dependent on agricultural income or mixed farming i.e. crop and livestock production. As a result, mixed farmingis the main sources of livelihood and occupation of the communities in the proposed project area. The farming system of the project area is Tradition farming based up on the annual rain fed in addition with traditional irrigation practice. The major potential crops type that cultivated around the project area includes - Maize, Teff, Wheat, Nouger, H/Bean, Pea and Barley. Here, crop production complements livestock production i.e. crop enterprise deliver different crop residues as a source feed to livestock and livestock provides draft power and organic fertilizer for the crop enterprise. The main crop production constraint that faced the project area communities are un adequate and untimely input supply, Erosion, Land degradation, Siltation problem,  pests, Worms and Weeding

1.3.3	 Irrigation and Crop Production

Crop production is the basic economic activities and occupations of the communities in the project area. The community of project area is traditionally practice vastly Maize and Potato by this River for the purpose of increase production and productivity. According to the household survey result, more of the interviewed household heads have an access to traditional irrigation development and they use for production of Cattle’s and Ruminants. Data obtained from Sample household also showed that there is above half of the  household heads got irrigation extension services except some individual person and 3-8 times in a year they could been visited by extension agents.

Table:12 Major crops grown, Area cultivated, Area cul./hh, Yield/qt and Price/qt in the project area kebele

| S/N | Types of crop | Unit | Area cultivated | Area cultivated/hh | Total Yield/hec | Price/qtIn Birr | Total |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Maize | He | 335 | 1.2 | 45 | 500 | 27000 |
| 2 | Teff | He | 435 | 1.5 | 9 | 900 | 12150 |
| 3 | Wheat | He | 296 | 1.1 | 25 | 600 | 16500 |
| 4 | Nougar | He | 320 | 1.2 | 5 | 1050 | 6300 |
| 5 | Bean | He | 104 | 0.5 | 12 | 750 | 4500 |
| 6 | Pea | He | 80 | 0.3 | 9 | 850 | 2295 |
| 7 | Barley | He | 60 | 0.2 | 12 | 550 | 1320 |
|  | Total |  | 1624 | 6 | 117 | 5200 | 70,065 |
| Types of crops grown by Traditional irrigation |  |  |  |  |  |  |  |
| 6 | Maize | he | 15 | 0.25 | 20 | 550 | 2750 |
| 8 | Potato | he | 5 | 0.125 | 50 | 300 | 1875 |
|  | Total |  | 20 | 0.375 | 70 | 850 | 4625 |
|  | Grand total |  |  |  |  |  | 74,690 |

Source: - Plant science Development agent of the kebele

Table.13 Distribution of sample household by irrigation situation

| No | Items | Frequency | Percent (%) | Remarkk |
| --- | --- | --- | --- | --- |
| 1 | Do have traditional irrigation Services? |  |  |  |
| 1.1 | Yes | 37 | 92.5 |  |
| 1.2 | No | 3 | 3.5 |  |
|  | Total | 40 | 100 |  |
| 2 | Input supply for rain fed |  |  |  |
| 2.1 | Yes | 40 | 100.0 |  |
| 2.2 | No | 0 | 0 |  |
| Total |  | 40 | 100 |  |
| 3 | Irrigation Extension service |  |  |  |
| 3.1 | Yes | 37 | 92.5 |  |
| 3.2 | No | 3 | 3.5 |  |
| Total |  | 40 | 100.0 |  |
| 4 | Have you been ever visited by an extension agent? |  |  |  |
| 4.1 | Yes | 28 | 70 |  |
| 4.2 | No | 12 | 30 |  |
| Total |  | 40 | 100.0 |  |
| 5 | How many times in a year? |  |  |  |
| 5.1 | 2-3 times | 10 | 20.0 |  |
| 5.2 | 3-5 times | 15 | 25 |  |
| 5.3 | 6-8 times | 8 | 20 |  |
| 5.4 | More than 8 times | 4 | 10 |  |
| 5.5 | Missing system | 3 | 20 |  |
| Total |  | 40 | 100.0 |  |

Source: HH survey result, 2006

Livestock production

Next to crop production, livestock are the basic sources of income and economic base of the households. Livestock serves as a source of draft power (oxen), provides milk and milk products (Cow), meat, hides and skins (Beef), are source of cash and serve as a social security asset. Based on the household survey result live stocks production has the highest economic contribution to the existing communities. The type of livestock production applied by the communities of the project area incorporates Oxen, Cow, Heifer, Bull, Calf, Sheep, Goat, Poultry, Donkey, Mule and Horse.

There is only one animal veterinary service around the project area, which did not accommodate the type, and the size of livestocks production healthy as the society expected. In addition, there are constraints that hinder livestock’s production as the community wants around the project area, of these, over grazing or shortage of grazing land, medicine supply and shortage of timely get appropriate services.

Jimma Rare district has large Livestock and poultry resource. From early days, livestock rearing has played an important role in the life of district population. In majority of districts’ kebeles less attention is given for preparation of livestock food. In some area to some extent livestock feed are good especially for cows that serve milk. Most of the time animals have feed from green fodders (grazing) which are simply pasture grasses and any kind of grasses. In addition to this in small amount crop by product including chaff of cereals and pulses and traditional alcohol, (‘Tella’ and 'Areke') residues are the major animal feed in Jimma Rare district. In general, the preparation of animals feed and the way they feed are very poor when compared with their benefits. Thus, to increase the livestock sub sectors improving animals feed have needed.

There are different kinds of diseases that affect livestock production in Jimma rare. The major diseases that affect Catteles, Rumenants, equine, poultry and bees are the following:

Tripianosis (Gandii), Bovine pasterolosses (Gororsa), Blackleg (abba Gorba), Pasteurollosis, Foot and Mouth diseases (Kote Bakakisa), Sheep and goat pox, Coccidian, Lamp skin disease, Anthrax, Internal and external parasites etc.

Livestock product and by product in the form of meat, milk, honey, eggs, cheese, and butter supply the needed animal protein that contribute to the improvement of the people life. Livestock also plays important role in providing export commodities, such as live animals, hides and skin to earn foreign exchange. On the other hand, draught animals provide power for the cultivation of the smallholding, for crop threshing, use as mode of transport, and the like are the major benefits of livestock. Thus, to care livestock's from different disease are the major economic importance for the district as well as for the nation.

To alleviate the above problems increasing the number of veterinary service centers in the district is the vita solution. So far there are only one veterinary clinics (type c), which provide veterinary services. On the other hand, one veterinary Medical Doctor, two veterinary assistant, and   four animal health assistants are currently providing services in this respect.

Beekeeping activity: in the district is somewhat practiced. According to the data obtained from the district agriculture office, there are 1520 traditional beehives and 48 modern beehives. The major problem here is that no proper data has consolidated due to lack of skilled workers in this respect. Bee toxicity, Herbicide, and absence of proper management are among problems that affect beekeeping rearing.

According the survey, the total number of Livestock held by respondents was 569, of which cattle accounted for 31.35%:  cows’ oxen, calves, and heifers.7.26 percentage, 19.14%, 6.27%and 35.97% are the quantity of sheep, goat, donkey and hens held by the sample household respectively. The average number of livestock per household is 14.08.

Table.15 Respondents Total size of livestock holding

| No | Items | Total(Qty) | Average price | Value/Ravenous | Average | % |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | Ox | 66 | 4000 | 264000 | 1.65 | 11.4 |
| 2 | Cow | 75 | 3000 | 225000 | 1.88 | 12.95 |
| 3 | Heifer | 35 | 1500 | 52500 | 0.88 | 6.15 |
| 4 | Calf | 55 | 950 | 52250 | 1.38 | 9.67 |
| 5 | poultry | 193 | 65 | 12545 | 4.83 | 33.92 |
| 6 | Sheep | 69 | 600 | 41400 | 1.73 | 12.13 |
| 7 | Goat | 32 | 450 | 16000 | 0.8 | 5.62 |
| 8 | Horse | 14 | 900 | 12600 | 0.35 | 2.4 |
| 9 | Donkey | 36 | 1500 | 54000 | 0.85 | 6.2 |
| Total |  | 579 |  | 737895 826495 |  | 100 |

Source: HH survey result, 2006

Graphically

Off-farm income

It is obvious that, off-farm income is one of the source of income and economic bases of any community. But as socio-economic survey of the project area was conducted, the contribution of off-farm income is insignificant, for the reason that the communities of the project area depends more on agricultural practice, crop production and animal rearing. According to household survey conducted, only few households gained off-farm income from small-scale trade activities.   The minimum and maximum off-farm income of the household is 0 Birr and 3000 birr per year respectively and the average off-farm income of the sample household head is 234.75 Birr per year.

| Table 16 The minimum and maximum off-Farm income of the respondent |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | No. of Resp. | Minimum | Maximum | Sum | Mean |
| Off-farm income of the HH | 40 | .00 | 3000.00 | 9390.00 | 234.75 |

.Food Insecurity/food deficit/ Situations

Food insecurity occurrence in the area/ kebeles

According to the conducted data from the kebeles administrative office the population of the project area were somewhat not as much suffered by food insecurity unless very few household populations. The season of food insecurity months were starting from July to October for those few households populations who suffered by such problems. However, acute food insecurity and food aid were not available   around the project area population, as the result of these there is no safety-net program assistance is applicable as zone.. The cause for food insecurity observed around the project area was insufficient land holding capacity of few respondents. However, the most affected people by such problem were those who earn low income annually men and women households. Finally, the best main focused strategies that mitigate those suffered by food insecurity problem will be organizing them on the proposed modern irrigation practice properly.

Irrigation & Water Harvesting Activities

The type of irrigation that practiced around the project area was traditional weir diversion on the expected river with 150 hectare of irrigable land, Maze and potato crops grown vastly by 450 household beneficiaries. The existing beneficiaries accounts some few years on this traditional irrigation activities. Nevertheless, water-harvesting technology was not applied around the project area, whereas, water management practice and institution involved in irrigation sector were not efficiently practiced by existing beneficiaries.

1.3.4 RESOURCE OWNERSHIP

Land access and Existing Management

Land is the critical and basic resources for the project area communities. They could be acquired by inheritance, contractual rent, sharecropping and formal distribution in particular for a limited number of young people. Parents are obligated to provide land plots for the matured or young married boys. Economically poor and weak households and women headed households usually give their holdings use right to financially strong individuals on contract or by share cropping arrangement.

To alleviate the problems related to scarcity of tenure, the Government launched new land use proclamations to define farmers’ rights more firmly, namely Proclamation 456/2005 entitled Rural Land use Administration and Jimma Rare wereda Land and Environmental Protection office is responsible for providing farmers with land use certificates. Land certification has completed in both kebeles of the wereda farmers.

The rural land using payments either rain fed or irrigated cultivation, land tax and income tax is ETB 75 and 150 for rain fed, ETB 20 and 40 for irrigated development 0.5 hectare around the project area.

Table -17 the land use patterns of both kebeles have shown.

| No | Land used types | Land holding size by kebeles |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Bada Warke | Gudata Doby | Total |  |
| 1 | Cultivated land | 1479 | 1395 | 2874 |  |
| 2 | Grazing land | 235 | 189 | 424 |  |
| 3 | Forest Land | 38 | 32 | 70 |  |
| 4 | Construction land | 28 | 35 | 63 |  |
| 5 | Rock, Mountain, Road & Water | 7 | 9 | 16 |  |
|  | Total | 1787 | 1660 | 3447 |  |

Source: Bada Warke and Gudata Doby  Kebele offices

Farm Size

The agricultural farm size holding around the project area were varies among the households. As sample survey conducted, all of the respondents have land for cultivation and all household heads acquired land by contractual agreement, crop sharing, formal distribution through their kebele administration offices.  As to the land size analyzed, the respondents have 200 hectare of land existing by traditional irrigation, the minimum and maximum land sizes hold by the sample households is 0.08 hectare and 4hectare respectively. In addition, the average land holding of the sampled household is 1.59 hectare of land(Table.9)

Table.18  How respondents access to land holdings

| No No | Items | No. of Resp. | Percent (%) | Remark |
| --- | --- | --- | --- | --- |
| 1 | Do you have land for cultivation? |  |  |  |
| 1.1 | Yes | 40 | 100.0 |  |
| 1.2 | No | 0 | 00 |  |
| 2 | How did you get land? |  |  |  |
| 2.1 | By inheritances | 3 | 7.5 |  |
| 2.2 | By share cropping | 5 | 12.5 |  |
| 2.3 | By contractual | 0 | 0 |  |
| 2.3 | Formal distribution by kebele administration | 32 | 80 |  |
| Total |  | 40 | 100.0 |  |

Table 9 the minimum and maximum land holding of the respondents

|  | N | Range | Minimum | Maximum | Sum | Mean |
| --- | --- | --- | --- | --- | --- | --- |
| How much the total size of land | 40 | 3.92 | .08 | 4.00 | 63.98 | 1.59 |
| For how long have been working in irrigation? | 40 | 25.00 | 0.00 | 25.00 | --------- | 10.82 |

Source: HH survey result, 2006

According to household survey conducted, 4.37ha (0.14%) is Homestead, 58.58 ha (53.68%) is rain fed, 32ha (29.33%) is irrigated, 2.77 ha (2.54%) is grazing, 7.34ha (6.73%) is Leased in and 4.06 ha (3.72%) is Leased out land.   The land use types by the respondents have shown below

Table.20 Respondents by land use types

| No | Land used | Total Holding in Hectares of respondent | Average holding | % |
| --- | --- | --- | --- | --- |
| 1 | Homestead | 4.37 | 0.14 | 4.00 |
| 2 | Rain Fed | 58.58 | 1.00 | 53.68 |
| 3 | Irrigated | 32 | 0.15 | 29.33 |
| 4 | Grazing | 2.77 | 0.09 | 2.54 |
| 5 | Leased in | 7.34 | 0.00 | 6.73 |
| 6 | Leased out | 4.06 | 0.00 | 3.72 |
| Total |  | 109..2 | 1.38 | 100 |

Source: HH survey result, 2006

Housing status

It is clear that house is the most important household basic resource next to land resource. According to the survey conducted, greater than half (95%) of the sample household had houses with iron sheet roofing and the rest 5% were as grass roofing houses. The minimum and maximum age of the sample house was 1 and 42 years respectively. The average age of houses has calculated, as the total sampled household age divided by the total number of sampled hh is equal to 10.57 years. The average house size reported by respondents was 51.52 m2. The minimum and maximum house sizes of the respondents are 6 m2 and 160 m2 respectively. The average number of houserooms reported by them is 2.6 rooms, the minimum number rooms of house is one and the maximum number of rooms of the house is four.

In addition, there is inadequate house utilities and house furniture around the project area, which means water supply, Electricity, telephone and communication facilities.

Table 21 Sample of housing condition of the household’s ages

| No | Types of house |  |  | House conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Items | Numbers | (%) | Items | Range | Minimum | Maximum | Sum | Mean |
| 1 | Iron sheet roofing | 38 | 95 | Number of rooms | 3.00 | 1.00 | 4.00 | 104.00 | 2.6000 |
| 2 | Grass roofing | 2 | 5 | Area of house(square meter) | 154.00 | 6.00 | 160.00 | 2061.00 | 51.5250 |
|  | Total | 40 | 100 | Year of the house(age) | 41.00 | 1.00 | 42.00 | 423.00 | 10.5750 |

Source: HH survey result, 2014

In respect of utilities, no electric connection service in the project area exsept few households. From the survey result, about 95% of households had a latrine. Similarly, household furniture and equipment was found to be very low, and mostly limited to beds, tables and chairs. Only about 35% of households had a radio and all the respondents  had watering can.

Table 22 Housing facilities of the sample house holds

| No | Items | Frequency | Percent (%) | Remark |
| --- | --- | --- | --- | --- |
| 1 | Do you have toilet? |  |  |  |
|  | Yes | 38 | 95.0 |  |
|  | No | 2 | 5.0 |  |
|  | Total | 40 | 100.00 |  |
| 2 | Do you have radio? |  |  |  |
|  | Yes | 18 | 35.0 |  |
|  | No | 22 | 65.0 |  |
|  | Total | 40 | 100.0 |  |
| 3 | Do you have Watering can? |  |  |  |
|  | Yes | 40 | 100.0 |  |
|  | No | 0 | 0 |  |
|  | Total | 40 | 100.0 |  |
| 4 | Do have access of Electric services? |  |  |  |
|  | Yes | 14 | 37.5 |  |
|  | No | 26 | 62.5 |  |
|  | Total | 40 | 100.0 |  |
| 5 | Do you have Bed? |  |  |  |
|  | Yes | 28 | 75 |  |
|  | No | 12 | 25 |  |
|  | Total | 40 | 100.0 |  |
|  | Do you have Television? |  |  |  |
|  | Yes | 5 | 0 |  |
|  | No | 35 | 100.0 |  |
|  | Do you have mobile? |  |  |  |
|  | Yes | 10 | 35.0 |  |
|  | No | 30 | 65.0 |  |
|  |  | 40 | 100.0 |  |

Source: HH survey result, 2013

Farm equipment and transportation

Farm equipment is one of the basic assets to carry out the farming operation of an irrigation project. According to the household heads survey conducted, the most important item of equipment used on the sampled farms was oxen and from the sample, household reflection shows that oxen ownership is very high.The oxen will regularly replace after few years; because all of ox owning households have, oxen aged 4 Years old or less.

The number of hand tools is large, but hand tools represent the only other mechanical resource available for agricultural activities, which are consequently highly labor intensive. The basic farm equipment, number owned and average age that the sampled household used have explained as follow.

Table 23 Respondents Farm equipment holding

| No | Items | Number owned | Average age | Remark |
| --- | --- | --- | --- | --- |
| 1 | Oxen | 66 | 4.2 |  |
| 2 | Plow | 47 | 3.2 |  |
| 3 | Hoe | 64 | 2.0 |  |
| 4 | Sickle | 65 | 2.4 |  |
| 5 | Shovel | 31 | 3.1 |  |
| 6 | Slash | 53 | 1.9 |  |
|  | Total | 326 | 16.0 |  |

Source: HH survey result, 2014

Graphically

Source: HH survey result, 2014

According to other resources, transport resources are very critical for the irrigation project, but in and around the proposed project transport resources are equally scarce. As socioeconomic survey was conducted no bicycles and motorcycles were used by the respondents, but the transport resource that serve the project area society were Donkey and Horse even if the holding capacity of households were not enough as it required.

Household Incomes and Expenditure

Households Income

Regarding to income related issues; Assessment of sampled household survey income have done, since it is the basic component of socio-economic feasibility study for the proposed project. The sources of existing sampled households income have generated from rain fed crop production, traditional irrigation, livestocks production and off-farm jobs sales. The households’ survey reveals that households earn their income, 29% from crop cultivation, 17.6% from sale of vegetables &fruits, 13.6% from sale of livestock and livestock products, 15.6% from off farm and nonfarm activities and 24.25 % from others. Thus, agriculture is the dominant sector of employment in the kebele in addition to crop cultivation; the community also raises cattle, sheep and goats (Ruminants) and Donkey, Horse and Mule (equines) as the major livestock.

The gross cash income by the sampled household from sales of rain fed crops 440,913(11023 per hh), traditional irrigation 327,875(8,197) livestock (784953)(19,624) and from sale of off-farm jobs were(9390)(235) respectively as calculated from the survey data. The total gross cash income from these sources by the 40 households sampled was ETB 1,563,131 per year. The total average gross cash income per household from the above source mentioned was ETB 39,078.83 per year.

Table 24: source of existing household’s income by type

| Source of Income | Total income | Average Amount per hhs (Br) | % |
| --- | --- | --- | --- |
| From rain fed crops | 440,913 | 11,023 | 28.21 |
| From traditional irrigation | 327,875 | 8,197 327,875 327,875 | 20.98 |
| Livestock and livestock product | 784,953 | 19,623.83 | 50.22 |
| Off-farm jobs | 9390 | 235 | 0.61 |
| Total income | 1,563,131 | 39,078.83 | 100 |

Households expenditure

As the sampled household income, consumption and expenditure survey almost all the household spent their income on purchase of household food grain 26.48%, household clothing and foot wears 18.21%, Education 9.76%,Health 8.95%,Land tax 3.66%,Farm inputs 10.5%, farm implement 4.37%,Livestocks cost 2.92%,Household utilities 8.14%,Social contribution 6.51% and other social affairs while saving is almost minimal. Therefore, the total sampled household’s annual expense or expenditure was ETB 489,160 and the annual expenditure per household was ETB 12,289.5 Table below shows surveyed household percentage of estimated annual expenditure category. The survey indicated that the difference of annual income and expenditure is the food crops the household consumes daily as breakfast, lunch and dinner including other social ceremonies, which is ETB 26,788.5 estimated to food crops.

Table 25: Household consumption Expenditure in 2013

| Major consumption items | Total expenditure | Average amount per hhs (Br) | % |
| --- | --- | --- | --- |
| Food/food items | 130,160 | 3,254 | 26.48 |
| Clothing and foot wears | 89,520 | 2,238 | 18.21 |
| Education | 48,000 | 1,200 | 9.76 |
| Health | 44,000 | 1100 | 8.95 |
| Land Taxes | 18,000 | 450 | 3.66 |
| Farm implements | 21,500 | 537.5 | 4.37 |
| Farm inputs | 51,600 | 1290 | 10.5 |
| Livestock cost | 14,360 | 359 | 2.92 |
| Household utensils | 40,000 | 1000 | 8.14 |
| Social obligation/contribution to village activities | 32,000 | 800 | 6.51 |
| Total | 489,160 | 12,289.5 | 100 |

Source: household survey 2014

From this we can learn that the culture of saving is very poor associated with subsistence level of income at household and community level. Thus, further intervention is required to enhance culture of saving because of the fact that one cannot promote further investment without saving.

SOCIAL SERVICES, INFIRASTRUCTURES AND LOCAL LIVING STANDERD

1.3.5Water Supply

According to the data gathered by observation and discussion with beneficiary community existing source of the kebele water supply are unprotected sources such as rivers, springs, and temporal streams that are not safe for drinking. The survey conducted indicated that there is very few potable water in the kebeles (65% responded by the respondents) Most beneficiaries do not get potable water. The respondents reported that potable water supply is the major problem in the area, which has also expected to be aggravated as the result of inundation of the springs by the reservoir.

Though there is variation in the fetch, distance from household to household, Water shortage is the major problem of the existing system. The survey conducted has also indicated that 65% of the beneficiaries get adequate water and the rest 35% reported it to be inadequate.  According to the household survey conducted by the Economist, majority of the respondent were not satisfied with the quality of water they have supplied while they considered the quality very poor. The majority of the kebele residents fetch their daily drinking water from Doby River. That is why some of the respondents explain Typhoid and Diarrhea to be the 1st and second serious disease in the area.

The unprotected source administered by the beneficiary community that they have responsibility to oversee the overall operation and maintenance of the sources. The benefiting community members have responsibility to oversee the overall operation and maintenance of the unprotected source.

Sanitation

The sanitation condition of the area is poor for which the Wereda health officials reported below 5%. Defecation and domestic solid waste disposal in any available open field is common. This has observed during the survey. Thus, the sanitary condition of the kebele is one of the problem areas that require corrective action by the community and concerned local authorities.

Water supply in the project area is the duty of Jimma Rare Water, Mineral and Energy Office. Thus, the source of water supply for human being for the indicated society is spring, deep well, Hand pump and Spring with motorized and there is no modern livestocks drinking scheme services. Concerning to the proposed project area, there are 7 developed springs on spot with 3,051 beneficiaries, 2 Deep well(1,961), and 1 spring motorized(795) are giving drinking service for both kebeles of the project area and about 5,807 people benefited from these sources. The demand and supply of potable water around the project area is not proportional that means the demand of the communities is greterthan the supply of water relatively. The major problems faced around the project area were inadequate potable water, low sanitation - community awareness and lack of preparing the appropriate place for solid and liquid disposal. The solution related these problem will estimated to be invited the concerned sector

Table 26: - Type, number and current status of existing water supply schemes in the district

| S/n | Types of scheme | Number in |  | Current status |  |  |  | Beneficiaries |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Functional |  | Non Functional |  |  |
|  |  | wereda | Pir.area | wereda | Pir.area | wereda | Pir.area | Pir.area |
| 1 | Hand pump | 11 | 0 | 7 | 0 | 4 | 0 | 0 |
| 2 | Spring on spots | 103 | 7 | 95 | 4 | 8 | 3 | 3,051 |
| 3 | deep well | 4 | 2 | 2 | 1 | 1 | 1 | 1,961 |
|  | Spring with motorized | 3 | 1 | 2 | 1 | 1 | 0 | 795 |
|  | Total | 121 | 10 | 106 | 6 | 14 | 4 | 5,807 |

Source: - Jimma Rare Water, Mineral and Energy Office

Education

Education is one of critical factors contributing to the development of any country. Regarding the proposed project area, there are two primary schools (1-8th grade) in Bada Warke and Gudata Doby kebeles, which giving education services for about 1,264 total students, out of which 626 students are male and 671 are female students. The numbers of teachers that have given educational services in these schools is 31. For further educational, the students of the project area go to Wayu High School (9-10th) and preparatory school (11-12th) in Wayu town, the capital of the district office. In order to provide educational services to all societies in the district, educational curriculum is on implementation. As to the district, currently there are 30 total numbers of schools, which are providing educational services for the people in the district. Total number of schools and total number of student population has shown as follows table.

Table.27 Numbers of students and schools in the district.

| No | Type of Schools Grade | No. of Schools |  | District no. of students |  |  | Pir.area students |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | District | Pir.area | Male | Female | Total | male | Female | Total |
| 1 | Kindergarten | 1 | 0 | 168 | 171 | 339 | 0 | 0 | 0 |
| 2 | 1-4th | 18 | 2 | 5,632 | 5,746 | 13,378 | 626 | 638 | 1264 |
| 3 | 5-8th | 8 | 1 | 1,311 | 1,366 | 2,677 | 164 | 171 | 335 |
| 4 | 9-10th | 2 | 0 | 781 | 798 | 1,579 | 0 | 0 | 0 |
| 5 | 11-12th | 1 | 0 | 160 | 122 | 282 | 0 | 0 | 0 |
| Total |  | 30 |  | 8,052 | 8,1039,203 | 16,155 |  | 809 | 1,599 |

Source: - Jimma Rare Education District office.

In order to provide the educational services in the district and the total teachers available in the district is 365. Out of the total number of teachers 238 is male and the rest 130 is female. Whereas at the project area the total no. of teachers was 20 out of this 10 of them is male and the remaining 10 is females. The total numbers of teachers who are providing educational services at present by their profession have listed as below table

Table.28 Numbers of teachers in the district by professions

| No | Status | Number of teachers as district |  |  | Number of teachers as project area |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Male | Female | Total | Male | Female | Total |
| 1 | TTI | 6 | 15 | 21 | 0 | 2 | 2 |
| 2 | Diploma | 187 | 107 | 294 | 8 | 7 | 15 |
| 3 | Degree | 45 | 8 | 53 | 2 | 1 | 3 |
| Total |  | 238 | 130 | 368 | 10 | 10 | 20 |

Source: Jimma Rare Educational District office.

Shortage of potable water, Electricity, some access road, workers, classrooms, textbooks or library, source of income and shortage of facilities are the major problems concerned witheducational matter of the district and the project area. In the school, there is a ceaseless school dropout because of some basic social and personal problems. The enrollment rate as district is high. Even if,  there was major reason behind the school dropout, some of them are:- Early marriages, Parents’ problem (lack of labor during harvesting crops.),Family background i.e. low level of living (rural poverty) and To sustain one’s own life they involving in minor trade activities.To overcome these problems mentioned above, construction of schools by participation of community and government, understanding students and student communities the benefit comes from education are the main factors.

On the other hand, there are students’ clubs in the school undertaking different encouraging activities together with active participation of school communities. Adult females’ consultation club, Environmental protection club, Civic and ethical club, and HIV AIDS club are among the existing clubs in the school.

Health

As to any other factors of development, heath services have a great contribution to the development of a given society particularly. Regarding to heath situation, there are no health center, no private clinics and two-health post available in both the project area kebeles. According to data obtained from district health office, there are 28 health institutions giving services for the communities.

The whole health institutions available in the district at present time have shown as below table.

Table.29 Number of health institutions found in the district by 2005/2013

| No | Types of health institutions | No of institutions | Remark |
| --- | --- | --- | --- |
| 1 | Hospital | 0 | On construction |
| 2 | Health centre | 5 |  |
| 3 | Clinics | 3 |  |
| 4 | Health post | 18 |  |
| 5 | Rural drug store | 2 |  |
| Total |  | 28 |  |

Source: -Jimma Rare District Health office.

In the district there are 89 health professionals giving health services for communities of the district. The medical staff available in the district giving health service has depicted on the following table.

Table 30. Number of health professionals in the district by 2005.

| No | Type of Professions | No of health professionals |  |  | Remark |
| --- | --- | --- | --- | --- | --- |
|  |  | Male | Female | Total |  |
| 1 | Doctor | - | - | - |  |
| 2 | Clinical Nurse | 17 | 10 | 27 |  |
| 3 | Public Nurse | 1 | 2 | 3 |  |
| 4 | Senior Lab. Tech. Diploma | 3 | 1 | 4 |  |
| 5 | BSC Midwifery | - | 3 | 3 |  |
| 6 | Midwifery Diploma | - | 7 | 7 |  |
| 7 | BSC Nurse | 7 | - | 7 |  |
| 8 | BSC Lab Tech. | 4 | - | 4 |  |
| 9 | H/officer | 4 | 1 | 5 |  |
| 10 | Pharmacist | 2 | - | 2 |  |
| 11 | Druggist Diploma | 2 | 1 | 3 |  |
| 12 | BSC Environmental Health | 3 | - | 3 |  |
| 13 | Health extension | - | 21 | 21 |  |
| Total |  | 43 | 46 | 89 |  |

Source: - Jimma Rare District Health office.

The ten top diseases around the project area includes:-Typhoid fever,Pneumonia (Sambaa Michii) ,Malaria,Gastritis,Intestinal parasite,Romanism( Qurxumatii),Upper respiratory track infection,Urinary tract infection (Dhuk. Ujjummoo),Hypertension andSkin infection

As information obtained from district health office, the health coverage of the district is about 90%. As to the district and project area, the major problems face health offices of the district are lack of transportation, Shortage of professional workers, potable water, and untimely supply of medicine and shortage of facilities have extensively seen. The remedial major solution forecasted will be understand the issue to the concerned body  with a great attention to full fill the above mentioned shortages faced the area.

The intent or purpose of the health data over view is to identify and evaluate changes in human health, risk that might arise as the result of change in physical, biological, social and psychosocial factors of the environment. Potential risks should be screened and if deemed significant further investigated and mitigated during the final design of the project. Currently, the survey conducted indicates that Typhoid is the very serious disease in the project area (93% of the respondents responded) and the other disease mentioned during the survey is Diarrhea (7% of the respondents responded).

Though the government health care system is mainly rely on prevention in the view of Wereda Health Office and from the household survey result, Typhoid is highly prevalent in the area. As the result of daily laborer influx for employment during the construction period there, it has expected that stress will have created on presently existing health institutions as the number of patients probably might increase. Thus, it is advisable to integrate both prevention and curative health care system during the construction of the small-scale irrigation project. Besides, the weredas health officials reveal that though the health intuitions situated at close proximity there is shortage of medical equipments and workers in both quality and quantity.

Since the health institution in the area does not keep separate records for aforementioned kebeles inhabitant and non-resident patients, it is difficult to quantify the number of patients accordingly.  However, Typhoid, diarrhea and headache are the leading disease in the Kebeles. According to the information obtained, the existing health institution could not provide the desired services to their level because of shortage of workers and medical equipment. Furthermore, as it expected that the constructions of Dindila abba Oromo Small Scale Irrigation Project escalates or rise up health service demand and increases health problem. Therefore, it is clear that addition health service giving institution and improving present health service delivery will enable to cope up the health related problems likely to occur as the result of construction of the project.

HIV/AIDS and other diseases

HIV/AIDS is now a global problem, not only a public health issue of particular locality/countries. It affects the economic as well as the social setting of every sphere of society. Ethiopia is one of the more severed and the hardest hit country from African countries. This situation calls for devising ways and means for controlling HIV/AIDS transmission and prevention.

In order to bring about the desired behavioral changes in the kebeles advocacy work have done by NGO’s, community based and governmental organization. Hence, it has reported during the discussion with members of the community that there are low-risk perception yet high-risk activities.

Despite the efforts of different stakeholders to contain the endemic disease, the achievement is encouraging; the weredas health officials express their fear of sexual transmitted disease because of large number of population influx or enter to the area during the construction period. Therefore, enhancing the awareness of the population and establishing counseling and idea related treatment IRT delivery center is required to minimize this fatal disease. To conclude there is no disclosed reports of HIV/AIDS report in Bada Warke and Gudata Doby kebeles.

Roads

Road is essential factor in the study and implementation of an irrigation project. Thus, regarding to the Dindila aba Oromo project, the road takes to the site is dry weathered road from wereda to the site and all weathered road from that wereda to zonal capital, Shambu. Both Kebele has 165 km access rural road or dry weathered road, which has taken to the main road of the distinct, and 50 km all weathered road.   The distance of the project area is 118 km from Shambu, zonal capital. It is also about 20 km far from the district town, Wayu and 20km far from the main road of Addis Ababa to Shambu Road.  There is dry weathered road from this main road to the project site, which means inaccessible especially during rainy season.

Table 31 the quantity of Road served in Jimma Rare district by type.

| S/N | Type of Roads | Distance in km from |  |  | Time taken to arrive it |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Proj. area | Wereda | Total | Proj. area | Wereda | Total |
| 1 | Dry weathered | 20 km | 82 km | 102 km | 40 min | 3 hr &.40min. | 4.20 |
| 2 | All weathered Road | 0 | 50 km | 50 km | 0 | 1 hr | 1 hr |
| 3 | Asphalt | 0 | 0 |  | 0 | 0 | 0 |
| Total |  | 20 km | 132 km | 152 km | 40 min | 4 hr &40 min. | 5.20 min. |

20 Other Social Services

Electricity and Telephone

There is very few electric services in and around the proposed project area. Concerned the telecommunication services, kebeles of the project area have two wireless telephone and there is a mobile service around the project. Other than the above social services and infrastructure such as source of power, banking & insurance, and postal services are not applicable around the proposed project

Marketing and Pricing

Main product sold in the primary market (Balbala and Bada Warke market) is the grains and industrial products such as clothes and food substances that consist of the cereals and pluses. Among the cereals, wheat, Barley, Teff, Bean, Potato and Maize are economically important once.

Market-days and places in the study area are two, namely, Balbala and Bada Warke market are the big market and market place in the kebele on Wednesday and Saturday.  The primary market has held at nearby village level, usually two day in a week, for trading mainly farm produce, coffee, salt, sugar and edible oil. The market held at the open locations.  A range of commodities are traded in the market, which includes household food, and other consumable offered by outside traders and farm produce such as grains. The quantities of farm produce are small and have purchased by outside collectors as well as local consumers. According to the informants, the magnitude of this market decreased during rainy season.

At the town, the main market operated two day in a week, mostly. However, the main market day is on Saturday and wider ranges of items have transacted, including livestock. They have characterized by the presence of permanent market operators such as merchants and grain traders and other farm products collected by the minor trader at village fairs have sold to the grain market operators at the town market. Men are responsible for selling livestock and other agricultural product at Wayu and Balbala markets.

Therefore, farmers in and around the proposed project area deliver their products to local market hold on Sunday, Saturday, Tuesday and Wednesday at Wayu,Gedo (secondary market), Balbala,Bada Warkeand Gaba Kidame , Goban(primary market) and Finfine town(Tertiary market) which is 20, 32,64,10 and 200km far away from the proposed project area respectively. These reveals the potential markets chains for the proposed project output major crop consumptions. As a whole, the project direct beneficiaries will use to sell their products at all the above-mentioned market alternatives.The market availability is necessary for a given project effectiveness. Market allows the beneficiaries to produce crops, which have highly demanded at the market place and supply them to the needy market with minimum cost. The existing markets channels in the project area have mentioned with its size and distance on the following table

Table 32 Existing market size and its distance from the project area

| S/n | Name of the market | size | Distance | Market day |
| --- | --- | --- | --- | --- |
| 1 | Wayu | Bigger | 20 | Wenesday&satarday |
| 2 | Goban | Medium | 32 | Tuesday &Thuresday |
| 3 | Gedo | Medium | 64 | Monday &Firday |
| 4 | Ejaji | Biggest | 88 | Wenesday&satarday |

Source: - Development agent of the Kebeles

Price determination is a process of balancing demand and supply of the products. The price of product relatively high if the products have high demand and the supply of product is low or when supply of products are larger than their demand. Nevertheless, in most cases the farmers of the products area have forced to sell their products at low price during the production period to cover the inputs cost and taxes. There is some similar irrigation outputs supply by rain fed and traditional irrigation, but there is no modern irrigation project output supply near by the project area. The major existing and future problems that hinder the effectiveness of the expected project has forecasted.  The exiting problem that affects the communities’ income to generated as expected includes:

lack of accessibility and market infrastructures such as  lack of market information on :-Product supply, Demand & prices, Supply of agricultural inputs & prices, Lack of quality control, Poor forecasting) storage, road, and transport facilities, Poor marketing linkage ( input & output) ,undefined supply chain (input & output),Low level of value added activities and Lack of finance and hence weak bargaining power. The measure Solution that recommended or ensured the project sustainability has estimated as -

Conduct the proposed product Supply and   Demand analysis that means, how much volume the project can supply and what will be the demand for the product. Identifying and forecasting the consumption capacity of the potential or target market for the project supply.   Minimize the supply chain for the product, Improve transportation facilities, Improve storage and product handling systems, Establishing  Market Information System ,Timely disseminating market information in input and outputs & prices, Awareness creation regarding the benefits and utilization of information in production decision making, Develop a manual on information gathering, Capacity building for DAs, woreda experts in information processing & analyzing. Establish awareness creation forum for producers, traders, consumers and government implementing bodies found at various levels on the rules and regulations. Establishing an awareness creating media program for all producers, intermediates consumers based on quality and standard criteria, services for certification of agricultural input and output have established and strengthened. Support farmers with financial Institutions linkages for the purchase of modern inputs such as Fertilizer, Improved seeds and Irrigation facilities to enhance their capacity to meet credit demand of the communities.

Agro-processing/agri-business/farms or other investments

There is no agro processing and mechanizations or computerized system extensive investment activities around the project area. While, agri-business/production and the agricultural farm existing is low and traditional farming practice respectively.

NGOs & Religious organizations

There is no international and local NGO’s, development organization which contributes for irrigation development and community management around the project area. However, there is a religion organization, which does not contribute for irrigation development and community management around the project area.

Small scales Micro-enterprise

There is  small scale micro-enterprise/activities Currently plays a leading role in both kebele level development in organizing  youths,women,landless and farmers on traditional irrigation practice with the aim of improving their live standards. But there is no micro-enterprise which providing fund for credits support and facilitating wide transaction around the project area. Therefore, micro-enterprise should be organized properly to create job opportunity   to the target project beneficiaries and the surrounding societies with wide range support.

26 Micro-finance institutions/credits sources/ and others

There are no financial institutions, which were very critical in providing finance for irrigation development and for farm operation and management around the project area. Therefore, micro-finance institution have to provide fund institution to the target project beneficiaries and the surrounding societies to wide range support and to facilitating wide transaction.

1.3.6Gender issue

Gender issue assessment in the project area  was undertaken in respect of domestic agricultural and livestock production, labour divison, asset ownership, decision making, control over asset and fianacial resources, the degree of access to resources, and other basic rights of men and women. Many cultural, social, economic and environmental constraints affect both men and women. However, the magnitude of the problems is highly affected womens around the project area.

By definition Gender is the socially constructed roles and responsibilities assigned to women and men by society but not a biological factor in a given location. The sex based division of labor prevails in the project area from the past to current its degree is vary, that means even if currently gender main streaming is applicable. In the study area regarding to labor division women actively participates in all aspect of agriculture except ploughing. Women evolved in weeding, harvesting, land preparation, sowing, prepare the food, cleaning and managing the house, purchasing food items, fetching the water from source and collecting firewood, threshing, caring the children and seek member of family. Men at the project area mostly engaged in agricultural activities though there potential on the socially accepted work of women particularly during difficulties. In the decision, making affairs of the family husband or men plays pivotal role though wife consulted. Although men heads commands household agricultural labor, they do not exercise their right of commanding. Instead, they play the role of coordinating. The efforts made by different stakeholders change the value system that propagates or equalize the notion of women and men’s work. Therefore, in line with rights on asset ownership, decision making on household issue, control over common asset and fianacial resources, the degree of access to resources, and other basic rights of men and women are almost proportionally equal applicable around the project area.

Local /Traditional institutions/cooperatives

In conducting socio-economy study and design stage the existing local or traditional community institutions or organization  around the project area were Idir, Debo,  Senbete and Iqub established by the community and similar other institutions, which has been direct linkages and play agreat role in contribution of the future project operation and management.In the study area communities have their own mutual help organizations established to meet different purposes. CBO’s Functions and Structures in Bada Warke and Gudata Doby Kebelesexpressed as follows.

Iddir

Iddir is the most frequent type of community-based organizations in the study community. Like other such organizations in other parts of Ethiopia Iddir is primarily aimed to give support both financially and labour following the incidents of death. A grieved or feel sad family that lose a member will be given the above mentioned types of support.

Iddirs have capital that is obtained from members’ monthly contribution. The amount of contribution that members give varies from Iddir to Iddir. In general members are expected to contribute money on monthly bases or most often on the happenings depending on their bylaws. Iddir in the study community mainly gives support following the incident of death. On the other hand Iddirs in the study kebele area sometimes give support during times of sicknesses and robbery. This type of support is not common in different Iddirs.

Everyone can be a member as far as he/she fulfils the following criteria:

A member  should be willing and committed to obey the bylaws of Iddir

A member should be able to contribute some amount of money every time when need arise to assist members

A member  should agree to be governed or controlled  by the by laws of the Iddir

A member has to agree to attend the monthly meeting of the Iddir, which usually is held on Sunday morning.

Eligibility to member ship depends upon the bylaws that agreed by all members. Volunteers elected by the majority vote from members commonly manage these Iddirs. For Iddir leaders, there is no defined term of service as far as there is no discontent or dissatisfactions from members.

Iqub

Iqub is a traditional and rotational credit. Iqub is another form of organization that is practiced by theKebele community. The main aim of an Iqub is to serve as a traditional credit and saving scheme. Members need to save similar amount of money in different times based on the timing and scale of the Iqub. Each member will take the collected money in different times and expected to continue the saving even after one receives the collected money.  It is established when the need arises and stopped when the purpose is completed. The Iqub type varies depending on the purpose and economic capacity of the members.

Debo

Debo is normally organized for work day activity during peak period or during harvesting crops especially by active labor force that have positive attitude and holistic agreement each other. The organizer  requires to prepare a written agreement bylaw that guide them and force them to obey each other.at working day at working place and in the respected individual house  food(mullu) and drinks(Farsoo),properly prepared for the member of the working team or Dabo served as well as to supporting  individuals that will participate in the planned activity. According to member of the study community upon the arrival of the participants the one who seek support of others, begin the first activity.

Similarly, the organizer members express his/her wishes of success and expected to take part in any activity friends ask him to accomplish at any time as every  members of the society expected to pay hands to all participants when need arises. Therefore, the above mentioned community based organization will have a great contribution for the proposed project sustainable operation, mentainance and management  by passing direction,information,awaranss and control each other facilitates through this organization.

Agricultural extensions/inputs used, FTCs, Health posts, DAs.

Irrigation extension services have a great role in the development of an irrigation project. Therefore, the socio-economist has to assess in detail the existing irrigation extension services, inputs supply and demand, the existence of Development Agent (DA), its role, availability of FTC, its functions, problems, availability of animal health services and other technology utilization of farmers. Similarly, the linkage between research and extension has assessed and incorporated in the study document.

Development Assistants & Extension

In order to support as well as equipped the wereda and project area farmers with the technical knowhow and enable them to catch up with the current local technologies, the existence of development agents is inevitable. There are about 58 development agent workers as wereda of them 20-plant science, 19 Animal science and 17 Natural resource workers incorporated in it. According information obtained from kebeles administration offices, the development agent extension workers to farmer  house hold ratio is 1:165.Hence, in both project area kebeles there are eight extension workers of which six of them are development agent experts, Animal, Natural resource and plant science, while two of the them is health extension. The extension workers of the kebeles are not specialized on animal science, plant science and natural resources and on average; they had four years working experiences. Regarding to office equipment resources, each kebeles have DAs office with inadequate office equipment resources like table, seats, shelf, and other materials, which help them for demonstration field and reference materials. There is farmers training centre and demonstration plots areas in the two kebeles of the project area.

The office of DA is located in FTC compound of the Kebele administration office. The major duty of DA’s found in the area is/are: -Rendering training for the farmers, Giving technical knowledge on the demonstration site and Closely supervising the farmers’ daily activities on the field to check whether they are on the right track or not

Agricultural Development Office

Irrigation is an agricultural activities and it is interdependent factor of economic development. Therefore, during the feasibility study of the project, there was discussion about cooperative activities with agricultural development office concerning providing extension works such as input supply, facilitating credit and service providing activities and there will be sustainable works planned with each other. Regarding to Environmental protection and land administration, land surveying and registration has conducted in the project area and in the district as a whole and all farmers of the district has land certification.

Cooperative and Marketing Organization

There is one marketing cooperative organization for the two kebeles which provde inputs supply such as selected seed, Fertlizers and different chemicals marketing services for the intended project beneficiary farmers of the project area.

Agro-processing

There is no any large and medium scale agro-processing and food processing industries in and around the proposed project area as well as in the district.

Domestic energy and construction wood sources

There is no domestic energy and construction wood resource used around the project area communities. Therefore, the responsible bodies should aware and take responsibility to establish these domestic energy and construction of wood to the target communities with in short period by giving attention too.

1.3.7Irrigation Organization Situation (WUA's)

Regarding to organization situation, Dindila aba Oromo traditional irrigation has no well-organized irrigation organization. The traditional irrigation has working committees, which have five members (i.e. chairman, Secretary, casher and other members).  This traditional irrigation member structural working conditions were through equal sharing, contractual and self owned activities. Peacefully dividing water equally between them is their strong side and some committee members did not carry out their duties is their weakness.

Future Organization and implementation Plan

During the feasibility study of Dindila aba Oromo SS irrigation, there are several discussion made  with beneficiaries, local administrations and all concerned bodies. From this discussion and meeting with the users of the proposed project and based on the existed traditional irrigation organization situation, one Water User Associations were established and each of them knows and takes their responsibilities and duties.

Besides to committees of the project, the farmers of the area including the administrative officials of the district and kebeles have good and positive attitudes towards the proposed project future plan of activities with concerns to the project implementation because it can cease or come to an end the problem of traditional weir diversion  which affected by high flooding during rainy season year by year .

Developmental potentials, constraints and opportunities

Based on the household survey conducted and detailed discussion with the project beneficiaries, a development agents and kebeles official, the study has identified opportunities and constraints for efficient project development. The identified opportunities and constraints are described below.

Opportunities

High Project Beneficeries willingness and Participation: During houshold survey and discussion, farmers were highly interested to upgrade the existing traditional irrigation system. They are positive to describe their problem and open to propose its solution.

Access to Markets:  The project relatively has better access to nearest markets.

The existence of labor resources

Access road: relatively there is good  access to main roads

Constraints

High population pressure as a result of illigal migration  from other area on the existing farm land and lack of alternative livelihoods as a result there is high level of underemployment, unemployment of youth and food insecurity constraints in the area.

Poor human health, low availability of Health Posts per capita, serious incidence of malaria and water born diseases and weak family planning services.

Lack of quality water supply.

Inadequate education services-lack of educational materials, low quality of education and high school drop out rate.

Poor grass root level institutional capacity (primary cooperative, MFI, FTCs, Kebele administrations etc.)

Capacity problem of farmers to access different farm implements.

Low level of women’s participation in the local level development process  and Degrading natural environment and resources through traditional forest burning in dry season

PROJECT BENEFITS AND IMPACTS.

Expected project benefits

The Benefit and positive socio-economic impacts forecasted by the expected small-scale irrigation project will has the following impacts:-

The major expected socioeconomic benefit and impacts of the project are probably deriving from project overall development activities and project objectives. These expected benefits and positive impacts of project will described in the following manner.

Ensuring Sustainable Food Security

According to the households survey data conducted in the intended project area, food insecurity has observed on very few household populations during this study. Inconsistence or insufficient rainfall and low productivity reduces the ability of farmers to produce adequate food crops, which trapping the rural poor in a vicious circle.   Therefore, the introduction of improved and expanded irrigation farming in the area should enable farmers to produce adequate crops on smallholder farm thereby ensuring sustainable food supplies in the area and increasing household income.

Promoting Family Welfare

As per socio-economic study of the area conducted, the living standard of the communities is vulnerable to lack of income and consequently suffers considerably. Inadequate access to balanced food leads to malnutrition and susceptibility or vulnerability to diseases. The proposed irrigation project can boost or improve the production and productivities of the beneficiaries by double farming in a year efficiently and effectively. Thus, the intended project should have a positive impact by alleviating disease caused by a deficient diet. The beneficiaries should be able to satisfy their clothing needs, furnish their homes, cover medical and other related social services expenses from their enhanced incomes through increased production by the proposed project.

Create employment Opportunities

The majority of the population of the project area is under working age. Irrigated farming is labor-intensive and therefore the existing under-utilized work force or unemployed labor should obtain employment. In addition, during construction, the project will require a significant skilled and un skilled labor force, but although temporary work force from the surrounding area; this will be advantageous for local people.

Increase the contribution for Social Services and Infrastructural Development

The proposed project should make a greater contribution to increased productivity and enhanced household income levels.  In turn, this will enable parents to send their children to school, providing their necessities and covering their expenses. There should be opportunities to expand social services and infrastructure in the project area. In view of the disadvantaged landless, the construction of project can be considered as a step forward in temporary alleviating the problem of unemployment

Income Generating for Women

Expansion of irrigation project will help women to participate in irrigation farming and increases their production and productivity. Limited income generating activities were one of the restricting factors for women in the project area.

In summary, there are several tangible and intangible (direct and indirect) benefits expected to accrue or accumulate from the proposed irrigation, in addition to those mentioned above.   However, in order to ensure the accrual of all these benefits, the project has to be coordinated properly within the existing socioeconomic context.

Improving employment opportunity to the farmers and others from adjoining and different area around  the project  during the construction period , Stimulating the economic activity creation of employment opportunity and increased income by enabling the establishment of manufacturing plants,   Enhancing the development of township by attracting commercial activities and investment that pave way for urbanization,

The Negative impact of the projects

Any project will have its own positive and negative impacts. Benchmark assessments have needed to assess project development in terms of expected negative impacts and need for early mitigation measures. For the proposed irrigation project, the following are major expected problems, which will have adverse impacts unless appropriate corrective measures have incorporated or included into the project design.

The project will probably have negative impacts as well. The anticipated impacts of the project have listed as of the following.

-The social conflicts may be occurred because the command area delineation is on an individual land holding demarcates.  After implementation of the project, the contending or opposing kebele or the adjacent kebele may quarrel over its consumption.

-The new technology (modern diversion weir) going have to introduced in the project may not match with capability of the expected beneficiaries because of low knowledge of it.

Any project will have its own positive and negative impacts. Benchmark assessments have needed to assess project development in terms of expected negative impacts and need for early mitigation measures.For the proposed irrigation project, the following are major expected problems, which will have adverse impacts unless appropriate corrective measures have incorporated into the project design.

Malaria:

The project area is malarias area and malaria is the one of the top ten diseases prevailing currently in the project area. It has known that irrigation development will create favorable conditions for mosquitoes to breed, in the reservoir, command areas and along the main irrigation structures and, which may aggravate the situation. Serious and careful attentions have given to the design of the irrigation structures to incorporate appropriate mitigation measures.

Resources Displacement

Dindila aba Oromo SS Irrigation Project requires the construction of head work and different irrigation structures. Because of the headwork and different irrigation structures, some household resources will displace from the area. These factors influence the affected an individual economically, financially and environmentally. For such problem the expected beneficiaries have entered promise to replaced the amount land displaced by the intended project.

Avoidance of grazing land

Because of the implementation of improved irrigation project, grazing land in command area should be cultivated and it has used for crop production, livestock get watering from this river will be decrease. Thus, the conduct of modern irrigation project has negative impact on livestock production.

Mitigation measures

In order to mitigate the forecasted (probable) negative impacts of the project, the following possible measures have taken into consideration while the project is under implementation. To overcome the potential conflicts especially with the regard to its consumption, the ways in which the adjacent kebele’s use the irrigation water should be considered and Rendering continuous training for the whole users on the areas of irrigation water use and its management to capacitate them to adapt with the technology

Socio economic interventions

Since there is inadequate, social service infrastructural development around the project, area socio-economic intervention is the responsive measure to the expected project efficiency and effectiveness. Therefore, adequate potable water and cattle trough establishment, strengthening of primary cooperative, improving market infrastructure management, Micro-Enterprise establishment, and farm transport system, Health services improvement on malaria, family planning, HIV/AIDs control & sanitation and Education facilities provisions are the care attention measure until the project implementation undertaken through the encouragements of respective sectors

13.1 Table 33:-Type of Proposed Socio economic interventions

| Proposed Socio economic interventions for project area/Bada Warke and Gudata Doby Kebeles | Socio economic interventions cost |  |  |  |
| --- | --- | --- | --- | --- |
|  | Unit | Quantity | Unit price | Total cost |
| A-Social and Institutional Development for Bada Warke and Gudata Doby Kebeles |  |  |  |  |
| 1-Potable water supply |  |  |  |  |
| -Expansion | Sites | 6 | 65,000 | 390,000 |
| -Cattle trough construction | Sites | 4 | 20000 | 80,000 |

| B-Livelihood improvement interventions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Micro-Enterprises establishment (745 HH)-30 Group | Group | 25 | 900 | 22500 |
| C-1-Institutional Capacity Building |  |  |  |  |
| 1-Health services improvement & material provision |  |  |  |  |
| -Malaria control (agober, medicine,)-745HH | HP | 5 | 10000 | 50,000 |
| -Family planning | HP | 2 | 8000 | 16,000 |
| -HIV/AIDs control | HP | 3 | 5000 | 15,000 |
| 2-Education facilities and materials provision (2, 1-4 grade and 2 kindergarten schools) | Schools | 4 | 20,000 | 100,000 |
| 3-Strengthening existing grass root level institutions | ,, |  |  |  |
| -Farmers’ Training Centers (FTCs)-materials, practical training area & materials fulfillment | ,, | 1 | 75000 | 75,000 |
| D-Infrastructure Development |  |  |  |  |
| -Existing Road Improvement | KM | 20 | 17000 | 340,000 |
| Grand total |  |  |  | 2,137,000 |

1.3.8 COMMUNITY PARTICIPATION

Introduction

Irrigation development incorporates, Individuals, groups of individuals, people in the kebeles, villages and often the state must join to study, construct, operate and maintain the irrigation system as a whole.

AS it has summarized that the interaction and interdependence of the social and technical subsystems are the building blocks to form a purposeful whole in irrigation farm activities. Irrigation requires the integration of both dimensions that are social and technical. Without the incorporation of the social relations such as human interest, rational decision-making, power-relations, conflict etc. into the technical irrigation discourse, there cannot be a complete understanding of smallholders' irrigations because community irrigations are more than hydraulics and agronomy.

Accordingly, identifying community participation and willingness in the planning, implementation and operational stages of a project is the basic and necessary task in the feasibility study of an irrigation and drainage project. Thus, to achieve the goals of the project, beneficiaries have to be involved and contribute to the effectiveness and sustainability of the project.

Al most all of the households are members of traditional community organizations like Edir, Debo, Womfel and Church based self-assisted association. The traditional organizations are helpful for labour coordination and labour exchange during peak production period among the communities. The WUSC mobilizes member farmers to carry out labour-intensive maintenance, irrigation and flood prevention activities at appropriate times during the year. As large numbers of farmers are involved, and this work has carried out on a voluntary basis without payment.

Moreover, communities of the project area were participated in soil and water conservation and construction of social service delivery institutions such as health posts, schools, school farm operations, water supply schemes, animal health posts and access road constructions and road maintenance. There are ways that the community members participated for the accomplishment of different tasks and some of the forms of community participations in the area are through their labor, material, and cash/money and idea/information provisions. The attitude of the community towards the project has also investigated through stockholder consultations, where different groups of farmers’ development agents, Elders and Keble administrative participated in discussions and were clearly know about the proposed project.

Irrigation development is the basic  working system in alleviating or altering the living standard of the target  societies  through rendering  tangible  benefit such increasing production and productivity ,improving  quality of production, increasing time of scale, improve the consumptions of inputs ,increase the health status of the societies and increase the annual income of  house hold communities. Therefore, to achieve this development successfully community organization is the Maine issue, since it implies a coherent unity, having common interest and equally shared risk and benefit among the target communities.

Objective of the study

General Objectives

The general objective of community organization, participation and management is to identify existing community and existing local level support institutions willingness, their share of participation and propose future community organization.

Specific Objectives

Identify existing communities’ willingness and decision on the proposed projects

Understand community perception, attitude, participation, and demand driven of the project expressed in terms of demonstrable willingness of the user community to commit themselves in contributing resource and acceptance/ownership responsibility for Operation and Maintenance of schemes.

Identify forms (modes) of community participation or ways of maximizing communities participation

Propose workable institutional systems that promote effective planning, design, construction and O & M of Small-scale irrigation project.

Methods used

Several community approaches have employed with a view to have exactly the perception of the expected beneficiaries’ attitude towards the project. Among them:-

Discussion with district administration office and all stakeholder sectors

Development Agents and Kebeles leader’s discussion.

Discussion with all beneficiaries of the proposed project is among the major techniques used to get exact outlook of the user communities.

Procedure

The involvement of the user communities from the inception of the project up to its completion is inevitable for they play irreplaceable role. The project will have a long life span than expected, if the beneficiaries take part at all stages of the project cycle.  In order to create sense of ownership in the mind of beneficiaries, we do arrange general meeting to involve them in the study, design and construction of the project. Accordingly, to attain the intended goal of the proposed project discussion has conducted with future beneficiaries. In order to meet the targeted group the writer went through several stages. The procedures passed through the following.

At the beginning, discussion conducted with stakeholders like woreda officials of different sectors to inform them about the project plan.

The Kebeles administration office informed through formal written letter from woreda administration office

A short meeting held with kebeles administrators in order to conduct community discussion

Finally, general meeting declared to the communities through the existing leaders of development teams.

Agenda of the meeting

The agenda of the meeting focuses on- what the future beneficiaries of the project will contribute while this project is being undertaken (project cost share); sharing the farmland beyond their capacity to develop (future land redistribution); and covering the operation and maintenance costs by themselves (Operation and maintenance costs).

Community perception

In order to know attitude of the beneficiaries toward the project to implement in the near future, discussion has conducted. At the general meeting, for the sake of the discussion the main agendas mentioned above described for the users and have thoroughly discussed accordingly.  According to the beneficiaries, the proposed project have implemented in the area has a greater contribution to bringing sustainable development and producing surplus production. They added that the project would alter the existing socio-economic condition of the project area. Generally, the agendas of the meeting has briefly discussed as follows.

Project cost share

All project beneficiaries participated on the meeting were well oriented by zonal and district experts about the cost sharing. The project beneficiaries’ opinion is positive and encouraging Moreover, the participants themselves exchange different idea about the cost sharing. Finally, they agreed up on to cover all cost of unskilled labor. They also agree to contribute even interims of money if the implementations of the project will realize.

Therefore, during the meeting session, the whole customers of the project forwarded what they have to contribute when the proposed project will implemented. Accordingly, what beneficiaries promised to contribute are the following.

- Clearing the access road

- Providing construction materials

- Participating on the excavation of canal route

- Giving place for store camp if it has requested

- Involving in any activities that demand labor force

In general speaking, what the beneficiaries agreed to contribute is their labor more than financial contribution. Therefore, the labor costs of the above mentioned activities will covered by the community participation as per agreements at the general meeting.

Community Share and responsibility

| Table 34.Dindila aba Oromo SSIP Bill of Quantity and Specification by community participation |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Bill No | Item of work | Unit | Quantity | Unit Cost | Total Cost |
| 4 | Main canals |  |  |  |  |
| 4.1.1 | Excavation and shaping | m3 | 1200 | 70 | 84000 |
| 4.1.2 | Compacted Fill | m3 | 250 | 80 | 20000 |
| 5 | Secondary Canals |  |  |  |  |
| 5.1.1 | Excavation and shaping | m3 | 1350 | 70 | 94500 |
| 5.1.2 | Compacted Fill | m3 | 280 | 80 | 22400 |
| 6 | Tertiary canals |  |  |  |  |
| 6.1.1 | Excavation and shaping | m3 | 1540 | 70 | 107800 |
| 6.1.2 | Compacted Fill | m3 | 810 | 80 | 50220 |
|  | Total |  |  |  | 378,920 |

Future land reallocation

As it has discussed under main agendas of the meeting, the issue of future land reallocation will exhaustively discussed with the beneficiaries. At the beginning, the users perceived land reallocation as negative connotation if their farmland has taken away from them. However, later on, they agreed to redistribute their irrigable farmland among their respective family members and also for their close relatives or for their neighbors by exchanging with the same size of their rain fed farmland.

Operation and maintenance (O&M)

Users of the scheme also discussed in detail about the operation and maintenance issues which come after the implementations of the scheme. There is no resistance from the side of future beneficiaries and no one has raised contradicting idea but rather the whole users are looking forward to seeing the implementation of the proposed project. Finally, they agree to operate and maintain their scheme by themselves and to use it in sustainable manner. They also agreed up on taking the responsibility of scheme management aspect and to cover all costs that has required for the operation and maintenance of the scheme

Organizational management

The participants of the meeting also discussed on future organizational issue. The existence of organizational management is predictable for the project. Since it, play a key role in the utilization and management of irrigation water in sustainable manner.Therfore, for the sake of the proposed project, strong water user’s association committee (WUA) established with consisting of seven members such as chairperson, vice chairperson, secretary, cashier, purchaser, store man and one member nominated among the beneficiaries. The selection and position assignment has conducted based on the consent of the user communities. Hence, the names’ of the members of the committee with their assigned position have depicted here below.

Conclusion and Recommendation

The development and wise efficient use of medium and small-scale irrigation schemes in the country have currently improved than ever before. Therefore, full implementation of the project with the existing community participation and reorganizing the WUSC to operate and maintain as an independent institution will be the best alternative options.

The proposed Dindila aba Oromo irrigation project area is located at Bada Warke and Gudata Doby on Dindila aba Oromo River at the both sides of the river.   The design and planned irrigable command area is-56.5 hectare and expected to serve a total of 745 household beneficiaries. The project area has potential water source and suitable agro ecology for agriculture development.

Whereas, in cause of the assessment has been made in order to recognize the socio-economic condition of the project area, Based on the assessment findings, the following points boldly concluded and recommended.

According to the raw data analyzed, the project study is feasible and acceptable in line with, the socio- economic status of the targeted beneficiaries. The attitude of the expected society to wards to the project development is fully interested and hope full to participate on  What every activities that concern them during the project implementation in kind and cash starting from the study to the end of the project completion stage.

Due to lack of job considerable numbers of job seekers move to cities closer to their area and alien countries in quest or search of jobs and satisfying their material needs. So, the implementation of this project will highly recommend in mitigating the ceaseless migration. Moreover, it will create permanent employment opportunity in the expected area since it is labor-intensive work.

Shortages of social service infrastructures that have seen around the project area were inadequate. That means potable water supply, Health and Education service, all weathered Roads, Electricity service to all parts of the kebele, Animal health center and Credit financial institution are the main factors that will fill full and should take a great attention by the respective sectors and these shortages have to alleviate to the target communities until the project implementation started.

2. GENERAL SOIL Description

Soil has an important place in agricultural development. Its part in crop, soil and water relationship is very determined. It is the first media for valuable nutrients, water and other fertility factors that limit the crop yield and general productivity of the future irrigated agriculture in the area. Hence, before planning any cropping pattern and related activities, both physical and chemical pattern and related activities, both physical and chemical soil investigation should be done .As per this requirement all the possible soil observations in the would be project is conducted.

Within the given subs watershed and the command area soil is generalized as alluvial clay in nature with black (dark) color. At the upper part of any land unit, some rock fragments are distributed. It is deep and thick with medium cracking condition during dry and much sticky and plastic when gets wet. It is a transported soil from the upper catchment area (mountainous) parts. It is suitable for agriculture from observation of crop stand and present rain-fed production system. Due to its existing nature, workability is good, especially by traditional (ox-tillage) practice. A number of cereals and field crops are cultivating.

2.1. Methodology

Preliminary methodology is done as basic transect walk and deep observation in the field. An interview made with local farmers and the agriculturalists (experts of Agricultural office), referring the previous OIDA study guideline and discussion with the rest crew members, is considered as methodology for investigation principles.

Procedure: Allocating soil pits randomly and taking basic physical characteristics, like, depth, slope, color (slight), vegetation cover, physiographic and other factors, is major procedure taken during the field study period. And hence, physical and chemical analysis is made as final soil investigation level in the proposed command area.

2.2 Specific Observation

A particular observation (attention) was given in the command area specially, physical characteristics like texture, color, slope, depth and consistency, are visualized as far as possible at field level of investigation. Here the existing soil workability and the future improvement, that will be implemented during the development phase is also seen.

Land unit

The site have sever run off in and around it, the effective depth of the soil is very deep (>1.5m) and the color of the surface soil is red and reddish brown 5 YR 3/2 to reddish brown 5YR 6/3. It has moderate, medium, granular structure.

For the laboratory analysis six composite samples sited above from three places (each 15 ha) have been collected. Two composite samples from each place were taken. The analytical result shows that the soil of the command area is acidic with Ph (6.2) .As it is indicated in the earlier procedure part, pits are allocated randomly considering all the existing physical factors. After allocating the required or designed pits, samples are taken based on the horizon difference (layer appearance). The difference can be due to depth and texture, color and structural formation. According to this principle, the following pits are registered, with their samples in the study area.

| Pit Codes | Sample Quantity | Profile Depth (cm) | Remark |
| --- | --- | --- | --- |
| P1 | 2 | 0-30 | Cultivated land |
|  |  | 30-60 |  |
| p2 | 2 | 0-30 | Grazing transported soil land |
|  |  | 30-60 |  |

.   SOIL PROFLLE DESCRIPTION SHEET

| LOCATION Dildila Aba oromo |  |  | MAPSHEET |  | Field n No 1(right side) |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | MAPPING UNIT 1 |  |  |  | GRID REF. |  |
| SUTOR (S) Tarekegn Olana |  |  |  | /DATE 05/09/2006 | COOR DINATES |  |
| SOIL CLASSIFCATION (FAO) Fluvisol (ST) |  |  |  |  | PHASE high land |  |
| SURROUNDING LAND FORM convex |  |  |  |  | ROCK CROPS yes there is rock in and around command area |  |
| PHYSIOGRAPHIC POSITION SLOPE |  |  |  |  | ELEVATION 2295-2240 m.a.s.l |  |
| MICROTOPOGRAPHY |  |  |  |  | SLOPE nearly |  |
| VEGETATION/LAND USE 85%crop land and 10% bush, trees , shrubs land and marsh land |  |  |  |  |  |  |
| PARENT MATERIAL volcanic rock |  |  |  |  |  |  |
| FLOODING |  | high flooding |  |  |  | SEEPAGE yes observed |
| PARMEABIL Medium |  |  |  |  |  |  |
| SURFACE STONINESS/ rock observed in and around area |  |  |  |  |  |  |
| EROSION (A) AT SITE high B) SURROUNDING very high (gully) |  |  |  |  |  |  |
| CRACK A) DEPTH slightly B) WIDTH slightly C) FREQUENCY -No |  |  |  |  |  |  |

HOERIZONE DESCRIPTION

| HORIZON SYMBOL | 1 | 2 | 3 | 4 |
| --- | --- | --- | --- | --- |
| DEPTH (CM) | 0-30 | 30-60 | >90 |  |
| COLOUR - MOIST -DRY | Dark reddish brown | Dark brown reddish | Reddish brown reddish |  |
| MOTLING | yes | yes | yes |  |
| TEXTURE | Clay loam | Clay | Heavy clay |  |
| TRUCTURE/CRACKS |  |  |  |  |
| CONSISTENCE -DRY -MOIST -WET | hard slightly plastics plastics | Slightly Hard Slightly plastics Plastics | Moderately hard Plastics Plastics &stick |  |
| CUTANS/SLIKEN SIDES |  |  |  |  |
| CEMENTATION | medium | medium | high |  |
| PORES/BURROWS | - | - |  |  |
| ROCK FRAGMENT | - | - |  |  |

2.3 Summary of physical Soil properties

From specific observation and profile assessment basic soil parameters especially useful for determination of fertility and design of cropping pattern in future scheduling process. Considering basic factors as in the following table also summarizes soil water relation.

| Basic and common physical properties | Range of Consistency (Average Condition) |  |
| --- | --- | --- |
| Color | red & reddish brown | All physical rates are determined from field observation and the existing soil nature. |
| Depth | Thick (> 150cm) |  |
| Bulk Density | - |  |
| Field Capacity | - |  |
| Permanent witting point | - |  |
| Available Moisture | - |  |
| Red. Available Moisture | - |  |
| Expected porosity | - |  |
| Drainage status | Imperfect |  |
| Regional Slope | Nearly-gently (0-5%) |  |
| Land use | Cultivation land ,marginal &marsh land |  |
| Infiltration | mm/hr) |  |
| Water holding capacity | High (clay) |  |

2.4 Chemical Analysis

This is done in the laboratory, for specific and detail analysis. All required soil nutrients are determined. The required chemical investigation is done on the following parameters.

| PH C.E.C (meq/100gm soil) E.C (mmhos/cm2) T.N (%) Av. P (ppm) | Exchange Capacity of K. Mg, Ca, Na) Base Saturation (%) O.C (%) O.M Texture |
| --- | --- |

Results of the laboratory analysis are shown in the following table.

| Pit | Depth (cm) | Base Sa. | PH | EC | O.C | T.N | O.M | Ex. Base Cmol(+)Kg-1 Na K Ca Mg |  |  |  | SUM% | C,E.C | Av.p | Tex |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| P1 | 0 -30 | 71 | 6.8 | 0.16 | 2.27 | 0.20 | 3.91 | 0.05 | 1.28 | 30.94 | 7.35 | 39.63 | 55.6 | 8.82 | Clay loam |
|  | 30-06 | 69 | 6.7 | 0.07 | 1.77 | 0.15 | 3.05 | 0.04 | 1.02 | 30.31 | 6.81 | 38.18 | 55.5 | 5.94 | Clay |
| P2 | 0-30 | 53 | 6.1 | 0.09 | 2.81 | 0.24 | 4.84 | 0.17 | 0.32 | 26.51 | 5.32 | 32.31 | 60.6 | 16.14 | Clay loam |
|  | 30-60 | 57 | 6.0 | 0.04 | 0.92 | 0.08 | 1.58 | 0.23 | 0.12 | 27.71 | 5.65 | 33.71 | 59.3 | 8.10 | clay |
| P3 | 0-30 | 73 | 5.8 | 0.22 | 2.85 | 0.25 | 4.91 | 0.00 | 3.25 | 32.05 | 6.24 | 41.51 | 57.1 | 60.34 | Clay loam |
|  | 30-60 | 68 | 5.8 | 0.08 | 3.82 | 0.33 | 6.59 | 0.00 | 0.71 | 31.37 | 5.94 | 37.99 | 55.9 | 47.30 | Clay loam |

| Lab No | Depth (cm) | Sand% | Silt% | Clay% | EX. Acidity | Ex.Al 3+ |
| --- | --- | --- | --- | --- | --- | --- |
| P1 | 0 -30 | 34 | 28 | 38 |  |  |
|  | 30-06 | 32 | 28 | 40 |  |  |
| P2 | 0-30 | 42 | 26 | 32 |  |  |
|  | 30-60 | 36 | 18 | 46 |  |  |
| P3 | 0-30 | 36 | 34 | 30 |  |  |
|  | 30-60 | 38 | 32 | 30 |  |  |

| Soil Chemical & physical Condition | V. Low |  | Low |  | Normal |  | High |  | V. High |  | Remark |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | H1 | H2 | H1 | H2 | H1 | H2 | H1 | H2 | H1 | H2 |  |
| PH |  |  |  |  |  |  |  |  |  |  |  |
| C.E.C |  |  |  |  |  |  |  |  |  |  |  |
| E.C |  |  |  |  |  |  |  |  |  |  |  |
| T.N |  |  |  |  |  |  |  |  |  |  |  |
| Av. P |  |  |  |  |  |  |  |  |  |  |  |
| Ex. K+ |  |  |  |  |  |  |  |  |  |  |  |
| Na+ |  |  |  |  |  |  |  |  |  |  |  |
| Ca++ |  |  |  |  |  |  |  |  |  |  |  |
| Mg++ |  |  |  |  |  |  |  |  |  |  |  |
| Ba. Sa. |  |  |  |  |  |  |  |  |  |  |  |
| O.m |  |  |  |  |  |  |  |  |  |  |  |
| Tex |  |  |  |  |  |  |  |  |  |  | Clay loam and clay |

2.5. Producing soil Map Unit

Mostly the physical characteristics and certain chemical factors, Specially ESP are considered for the map unit production. The detail is indicated on the following table

| Soil Physical Characteristics | Class C -1 | Class C -2 | Class C -3 | Class N -1 | Class N -2 |
| --- | --- | --- | --- | --- | --- |
| Drainage |  |  |  |  |  |
| Slope (command) |  |  |  |  |  |
| Salinity (ESP) |  |  |  |  |  |
| Erosion |  |  |  |  |  |
| Depth |  |  |  |  |  |
| Rex |  |  |  |  |  |
| Structure |  |  |  |  |  |
| Physiographic |  |  |  |  |  |
| Regional slope |  |  |  |  |  |

Specific Soil Map Unit

| Items |  | Soil Unit | Remark |
| --- | --- | --- | --- |
| Area (ha) | 50 | UIB to UIA | Clay loam and clay soil character |
| (%) | 100 | - |  |
| Suitability | S2 | - |  |
| Capability | C1 | - |  |

Conclusion & Recommendation

Command Soil

The soil of the command area is fertile having high  organic matter content, high % total nitrogen content at the surface, moderately  acid reaction which is suitable for growing most of the irrigation crops and as clay loam and clay   texture which has high ‘water holding capacity

The command soil has high available phosphorus in its surface and also high at the subsurface layer and so has to be not amended by applying phosphate fertilizers. However, the results of other necessary soil nutrients, such as Mg, K, Ca, indicated the required amount thus salinity is expected as a serious case expect Na.

The other correction to be done is the soil conservation practice. Most of the around command area has a rolling slope which is very dangerous, care is taken. So, contour furrows for irrigation, contour bands and an agro forestry practices are recommended to the around command area. Trainings should be provided on these matters for the farmers.

3. Agronomy

The project area is agro-climatically classified as sub tropical humid with altitude of 2295-2240 m.a s.a and its topography is variable from sloppy to plain with average slop of 0 to 5 %.The land use at project area is classified for d/t purpose like cultivated land, grazing land and non Agricultural land (rocky, river stream embankments and gorges), little but not well conserved forest land is exist above the command area.

Agriculturally cultivated land in the project area is around 1239 ha which the majority and wide area is cultivated by rain fed agriculture and only around 327 ha is cultivated by traditional irrigation during dry season. The major crop produced under rain fed in project area wheat, Teff, barley, maize, Noug, pea, faba bean, potato and onions respectively. Crop produced under traditional irrigation are. Sugar cane, pepper, onion, potato and Spics are produced. Except for same perennial crops, crops produced by traditional irrigation are during dry season b/c during rainy season the farmers are prefer to produce rain fed crops b/c it is less coasty and less labor require if it is compared with irrigation. The farming system practiced in and around project area is mixed farming system both crop production and animal rearing which are the major means of livelihood of the area peoples

The farmers in and around project areas are sedentary mixed farmers which are living therefore along period of time and they are acclimatized with existing environmental condition this help them for better agricultural selection and adoptability.

Table 1 -Major land use type around project area. (Gudata Dobi and Lalistu Bada Warke   PAS)

| No | Land use type | Area/ha | Proportion % | Remark |
| --- | --- | --- | --- | --- |
| 1. | Cultivated land. | 1604 | 71.65 |  |
| 2. | Grazing land. | 178 | 7.95 |  |
| 3. | Forest land | 62.76 | 2.80 |  |
| 4. | (Non Agricultural) uncultivated land | 393.6 | 17.58 |  |
| Total |  | 2238.36 | 100 |  |

Source: - J/Rare district Lalistu Bada Warke and Gudata Dobi development Agents.

Table2 Land use type in project area.

| No | Land use type | Area/ha | Proportion % | Remark |
| --- | --- | --- | --- | --- |
| 1. | Cultivated land. | 140 | 93.80 |  |
| 2. | Grazing land. | 7 | 4.69 |  |
| 3. | Forest land | 0.25 | 0.16 |  |
| 4. | Non Agricultural land (Rocky, Gorge’s) | 2 | 1.34 |  |
| Total |  | 149.25 | 100 |  |

Source: - from study team estimation and elders Farmers interview.

3.1. Existing Agriculture

Existing Agriculture in a project area is crop production and Animal production respectively. The crop production in the area is used both for house hold consumption and income earning. The animal production in the area is used for draft power; transportation, food and income earning by sell.

Existing crop production

The largest part of the project area is cultivated by rain fed agriculture with cereal, pulls crop, oil crops and vegetables which are full of constraints, due to traditional agronomic practices and lack of improved technologies and the yield obtained is under average actual yield of the area. Traditional irrigation cultivated crops are diversified in varieties around house farm yard for example in single farmer house garden  greater than 2 variants of vegetable, fruit, stimulant and spice can be observed but varieties they use are local and their source of the varieties are unknown due to this and other cause the product obtained is very poor.

Existing cropping pattern and intensity

The existing cropping intensity in the area is gets rain fall once in a year and thus, and in wide area crops are produced once during the main rain season in and around project area is mainly cereals which cover the lively hood main stable food and followed by oil crops and legume crops this crops grown inter changeable and they also preserve soil fertility and decrees disease occurrence

In principle with sufficient irrigation in the dry season gradual change in cropping pattern and intensity are possible on either rain fed fields or traditional irrigation fields. In addition inter cropping and double cropping will be planed and practices in project area after construction of the project. Traditionally farmers cultivate their summer barley, peas and faba bean field after harvesting same vegetable crops like Maize, onion, tomato, pepper and cabbages then after harvest they cultivate barley which is harvested in three months at the beginning of summer rain

this indicate intensity

| cultivation Rain fed Agriculture | No | Type of crop | Area cultivated ha |  | Yield /ha | Operational calendar |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | In com | Around comman | Around comm. | Tillage |  | Sowing | Weeding |  | Harvest |
|  |  |  |  |  |  | Plowing season | Freq | Date | Start | Freq |  |
| Rain fed Rain fed | 1. | Teff |  | 175 | 16 | Mar-July | 6 | July | Aug-October. | 3 | December |
|  | 2. | maize |  | 207 | 40 | Mar-April | 4 | may | June-septum | 3 | Dec |
|  | 3. | Wheat |  | 271 | 25 | May-June | 5 | July | August | 2 | December |
|  | 4. | Noug |  | 11 | 4 | May | 2 | June | -- | - | December |
|  | 5. | Pea |  | 54 | 8 | June-end | 2-3 | August | August | 1-2 | November |
|  | 6. | F bean |  | 107 | 12 | June-end | 2-3 | ,, | August | 1-2 | November |
|  | 7. | Barley |  | 54 | 6 | June-end | 3-4 | June | august | 1-2 | September |
|  | 8. |  |  |  | - | June | 4 | June mid | June-septum | 3-4 | November |
|  | 9. |  |  |  |  | June end | 4 | June | July | 2-3 | August |
|  | 10. |  |  |  | - | June –end | 2-3 | June | July | 1-2 | December |
|  | Sub Total |  |  |  |  |  |  |  |  |  |  |
| Traditional irrigation Traditional | 1. | Potato | 169 | 80 |  | October-Dece | 4 | End Decem | February | 2-3 | March |
|  | 2. | barely | 6 | 12 | - | , | 4-5 | ,, | 2-3/a year | - | After 18month |
|  | 3 | Tomat |  | - |  | ,, | 4 | ,, | February- | 2-3 | March |
|  | 4. | Onion |  | - |  | ;; | 3 | .,, | 2-3/a year | - | - |
|  | 5 | maize | 175 | 60 | - |  |  |  |  |  |  |
|  | Sub-Total |  |  |  |  |  |  |  |  |  |  |
|  | G- Total |  |  |  |  |  |  |  |  |  |  |

Table 3 Existing major cropping calendar and pattern in and around project area. (2004/2005)

3.2. Potential and actual average yield

The actual average yield is the yield obtained in and around the project area under the management of farmers without use of inputs (fertilizer and improved seed). Potential average yield is the potential of varieties of improved seed that they can give an optimum yield under good agronomic and cultural practices using necessary agricultural inputs and technologies. This data was obtained from zonal irrigation project area and mostly Bakko Agricultural research center 2000/2001 Annual report and OIDA Guide line for feasibility study volume II

This actual and potential average yield is also used for further yield projection yield build up estimation with 5 years for the project.

The actual and potential average yields for the proposed crops.

Table -4 potential Actual average yield and potential average yield.

| No | Types of crop | Actual average yield /ha/at | Potential average yield /ha/at | Remark |
| --- | --- | --- | --- | --- |
| 1. | Potato | 100 | 250-320 |  |
| 2. | Tomato | 100 | 300-45o |  |
| 3. | Onion | 80 | 150-180 |  |
| 4. | Pepper | 15 | 25-30 |  |

Source J/Rare woreda Agriculture office for actual average yield and Bakko research center Annual report manual and OIDA Guide line.

Existing agricultural inputs and supporting services

The use of proper agricultural inputs at the recommended rate with a type of soil and plant plays a significant role in yield increment per unit area in crop production. Based up on this truth for the increment of yield in J/Rare Agricultural office, farmer co-operative and co-operative unions, for the project area d/t fertilizers and herbicides in addition to Improved seed of maize and wheat’s are supplied by “Lalistu bada warke and Gudata Dobi” farmers co-operative union and distributed to the farmers by cash/hand to hand/ sale by the Area of union   farmers co-operative. Improved seed of vegetables and different pesticides are supplied to project area farmers through district Agricultural office in forms of cash sale based up on both rain fed agriculture and traditional irrigation.

Most of the rain fed agriculture is used local seed without chemical fertilizer using animal dung and crop rotation to improve soil fertility in addition to fallowing system. For traditional irrigation project area farmers are use animal dung’s and compost rather than chemical fertilizer but yet application of chemical fertilizer is started by same innovators farmers but in same vegetable crops like onion, tomato, cabbage, carrot improved seed use is common. Fruits and S. cane produced at the area are their source of seed and its varieties is unknown b/c it is collected everywhere by the farmers and no source at all. The common known improved seed for rain fed cultivated crops are for maize BH 660 and HAR 1685 wheat with 2-4D herbicide and actinic 2 % for weevils.

The farmer at project area knows the importance and advantage of agricultural inputs but the price of the inputs and shortage of inputs are the basic problem to use properly. At this time the agricultural in puts supplied for the farmers are only through cash sale, but no credit facility for the farmers and they can’t to buy inputs for their different agricultural activities. The solution for this problem is in addition to rain fed creel crop production for stable food consumption it is better to use dry season irrigation production depends on high yielding and production crops which increase their ability to afford high input cost by gain additional income through market supply of irrigation products.

3.3. Existing Irrigation Practices

At the proposed irrigation area there is a practice of traditional irrigation which was started 1978 E.C. But due to lack of proper irrigation policy at the time and less co-operation of the farmer, it is not this much effective in production and yield obtained, In addition to this the way the water is used for irrigation is traditional and water management is poor there for the product obtained from total water is very poor. It is for this case that our team is initiated to study the project for better and modern way of this great resource.

Crop production constraints in and around project area

There are different production constraints, which cause the problem of low crop productivity in and around the project area. The major factors are: -

lack of improved agricultural technologies

In adequate supply and un affordable cost Agricultural inputs

Traditional agronomic practice

Disease, pest and weeds (vertebrate predators)

Soil erosion and degradation

Improved technology: - Improved ways agricultural production and technologies are not well distributed and introduced to the farmers, there is no demonstration area and on farm research done by agricultural experts at the area the constructed farmers train center (FTC) at the area is used only for d/t meeting wholes no more value it gives in order to improve farmers attitude and knowledge toward agricultural technologies.

There for farmers don’t have adoption trial with new and improved technology, But state policy toward   agriculture and improvement of farm technology is really negotiable and there is a great hope to alleviate this problem in a very few near future.

Inadequate supply of inputs and high cost of inputs

Except for 2003/2004 production year shortage of maize seed Bh660, BH 140, 3G-19 are common in the area, on other crops produced in generally access of improved seed except same vegetable crops like onion, carrot, Tomato etc.

The cost of agricultural input chemical fertilizer, Herbicide and pesticides are an affordable and very cost due to the progressive increment of dollars and other foreign changes b/c most of them are imported from abroad.

To overcome this problem the farmers in project area start to produce Niger seed during summer rain season which has good market valve and increase farmers purchasing capacity. The other solution subjected the farmers able to produce high productive and price valve crops and this may increase their income and farmers have ability to purchase improved agricultural technology and in puts

Traditional Agronomic practices

The way the farmer plowing their fields and the instrument used are hand plowing materials and oxen power this result in poor land preparation and aggravate soil erosion. Due to wide area farming to feed their family weeds management is poor and it is practiced by hand there for it is great constraints in yield reduction. Way of harvesting trashing and storing grains and vegetable are back ward and play great in yield quality and quantity reduction.

Disease pest weeds and predators

Disease: - It is harmful to growth and reproduction of the green plants as an organism. Disease adversely affects crop yield and quality. The major crop disease in and around the project area are:-

- Rust: - on tiff and wheat

- Cut warm: - on maize Tomato, other vegetables

- Stock borer: - on maize

- Mildew on wheat, onion, pepper

- Blight: - on potato, tomato, pepper

Insect Pest: - They are invertebrate animals which choose host plants or animals for the most basic of reason to obtain food or protection (over wintering, ovipositor) or to complete life cycle. These organisms usually cause same injury to their host. The common insect pests in and around the project area are:-

stalk borer & weevils  On maize

shoot fly and red tiff warm :- on tiff

shoot fly:- on barley

boll warm& shoot fly on faba bean

Aphids on cabbage

Cut warm on all vegetable & cereals

Most condoling method for insect pest is traditional and cultural method except using actilic and malathion 1% for where they are not wanted. They cause yield reduction by computing with required crops for light, moisture and nutrient. The common weeds in and around the project area are erucastrum arabicum, digitraspp, avena spp, Guizotia pp and Tribalism etc. The control measure practiced in and around project area is hand weeding and spraying 2,4-D herbicide for broad leaf weeds. The yield loss due to the abase mentioned problems it differ due to the climate, crop type and Agronomic practice due to J/Rare woreda Agriculture office crop production above problem was estimated 20-30 yields loss.

3.4 Livestock, production

The project area and around farmers are producing livestock as one of their livelihood means next to crop production the domestic animals found at the area are cattle, goat, sheep, equines and poultry respectively except for poultry in population. The farmers in and around project area used their live stock for: -Food (meat, milk and egg) draft power (plowing, loading cart and, trashing)

Transport (on the back of hors and mule)

Income generation from sale (stand) animal, products, contract)

The oxen is only draft power animal used for plough, but for donkey, mule and horse for trashing. Transportation animals for mans and agricultural products are equines.

In general the production gain form Livestock in and around project area is very less due to    :-* Traditional way of production.

- luck of productive breads

- local way of feeding and shortage of feed

- Less healthy service. Etc.

Table -5 live stock population in  Gudata Dobi and Lalistu Bada Warke PAS. (2004)

| No | Types of animals | Popn | Remark |
| --- | --- | --- | --- |
| 1. | Oxen | 2238 |  |
| 2. | Cows | 2840 |  |
| 3. | Heifers | 937 |  |
| 4. | Bulls | 755 |  |
| 5. | Calves | 320 |  |
| 6. | Horses | 277 |  |
| 7. | Mules | 112 |  |
| 8. | Donkeys | 194 |  |
| 9. | Sheep | 1989 |  |
| 10. | Goat | 620 |  |
| 11. | Poultry | 2862 |  |

Source: - Lalistu bada Warke and Gudata Dobi PAS development agents

The main forage source for animals are communal grazing land, free grazing after harvesting, small plots of personal grazing and cut and carry system respectively. There is shortage of grazing land during summer rain season due to expansion of farm land and more live stocks are excess for good grazing during harvesting time but after the dry season of the year during April, May and beginners of Jun there is shortage of grazing due to wilting and dryness of natural forage. Same innovative farmers store the crop residues for this shortage time and used to overcome the problem.

The pre-dominant livestock disease in and around the project area are :-trypanosomiasis, pastrollosis, Blackleg, Internal and  external parasite, mastitis, coccid sis African hors sickness etc. for this disease vaccination and treatment is given by Lalistu Bada warke and Gudata Bada Warke animal health center and J/Rare animal clinics but the timely and amount of medicine and vaccination supplied is scarce.

3.5Proposed crops for the project

The existing agricultural system in the project area is production of cereal crops, pulls, oil crops and vegetable respectively by rain fed agriculture.  The area under traditional irrigation is cultivated with vegetables, sugar cane fruits, coffee and spice respectively.

The proposed irrigable agriculture gives focus for high yielding and good price vegetables those enables farmers to gain good return based up on:-

Climatic condition and adoption

Soil condition

Experience and interest of the farmers

The length of growing period water need

Yield potential and market value.

Based upon this condition the following crops are proposed for command area. These are onion, potato tomato and pepper. These crops are suited for the given agro-climatic condition and soil type. The project area has an altitude range of 2520-2630 m.a.s.l and the rang of production of our crop is 1600-2800, 700-2500,600-2500 500-2400 m.a.s.l for maize, potato, anion, pepper and Tomato respectively. Then it is in the range of given agro climate condition for the proposed crops on the project area.

Income Generation and Nutritional value of proposed crop

The proposed crops are mainly these which have short life span and can give high yield per unit area. The vegetable proposed are which have high Nutritional value and good market demand. These proposed vegetables crops are sensitive for disease during rainy summer season and they are productive during dry season due to less aggressiveness and reproduction of disease and it is expected to gain high quality and quantity production at time.

3.6. Proposed cropping pattern

For the project area two cropping patterns each for full irrigation and supplementary irrigation are proposed without summer rain season to adopt by the farmers in the project area. The crop rotation and fallow system will be given consideration for the soil fertility and break the reproductive cycle of disease and pest

Table 11 Labor requirement and work activities for proposed crop at one cropping season for one hectare

3.7. Proposed crop and cropping calendar

Cropping calendar is a time at which crop production is under taken it is starting from land preparation and end at harvest at this range different agricultural activities clearing land plowing, bed preparation sowing, transplanting, watering, weeding and harvesting will be performed at this activities. The time this cropping calendar is differ based upon crop length of growing period (LGP) and climatic condition of the area in addition to verities of crop. At irrigation project area LGP and time of the year at which crop will be grown is affect the crop water requirement and it is play decisive factor on irrigation agronomy.

Table -12 cropping calendar for proposed crop phase I

| No | Type of crop | Operational colander |  |  |  |  | Harvesting |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Tillage |  | Date of sowing or plan time | Weeding. |  |  |
|  |  | Season of plugging | Frequency |  | Start of weeding | Freq |  |
| 1 | Maize | September | 4-5 | End October | Mid Nov. | 3-4 | Feb. |
| 2. | Onion | October | 3-4 | November 1st | Dice 1st | 2-4 | Feb-end |
| 3. | Potato | October | 3-4 | November 1st | Dice 1st | 2-3 | Feb- 1st |
| 4. | Pepper | October | 3-4 | November 1st | Dice 1st | 3-4 | Feb-end |
| 5. | Tomato | October | 3-4 | November 1st | Dice 1st | 2-4 | Feb-end |

| No | Type of crop | Operational colander |  |  |  |  | Harvesting |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Tillage |  | Date of sowing or plan tint | Weeding. |  |  |
|  |  |  | Frequency |  | Start of weeding | Frequency |  |
| 1 | Maize | March 1st | 3-4 | April | Mid-April | 2-3 | End-June |
| 2. | Onion | March 1st | 2-3 | March – Mid | April-Mid | 2-3 | End- June |
| 3. | Potato | March 1st | 2-3 | March – Mid | April-Mid | 2-3 | End- June |
| 4. | Pepper | March 1st | 2-3 | March – Mid | April-Mid | 2-3 | End- June |
| 5. | Tomato | March 1st | 2-3 | March – Mid | April-Mid | 2-3 | End- June |

Table -13 cropping calendar for proposed crop phase II

Source Bakkoo research center annual report and OIDA training manual (1997)

| Crop | Area | Feb | Mar | Apr | May | June | July | August | Sep | Oct | Nov | Dec |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Onion | 34 |  |  | 14.5 |  |  |  |  |  |  |  |  |
| Potato | 45 |  |  | 15 |  |  |  |  |  |  |  |  |
| Pepper | 25 |  |  | 15 |  |  |  |  |  |  |  |  |
| Tomato | 29 |  |  | 24 |  |  |  |  |  |  |  |  |
|  | 149 |  |  | 74.5 |  |  |  |  |  |  |  |  |

4.9. Proposed Agricultural inputs and supporting services

Agricultural inputs and improved technologies in agricultural yield increment play a decisive role without improved agronomic practices, use of fertilizers, improved seeds and pesticide and weeds chemical it is impossible to produce satisfactory agricultural production in addition to these strong Agricultural extension service and day to day introduction of farmer with those input and technology should be must in order to run the project according to its proposal.

| No | Type of crop. | Yield per ha/Q/t |  |
| --- | --- | --- | --- |
|  |  | Under research station | At farmers field. |
| 1 | maize | 80-120 | 45-60 |
| 2. | Onion | 160-180 | 80-120 |
| 3. | Potato | 200-260 | 90-160 |
| 4. | Pepper | 30 | 10-15 |
| 5. | Tomato | 314 | 120-160 |

Table 9. The proposed cropping pattern for five year in project is

| Year one |  | Year two |  | Year three |  | Year four |  | Year five |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| First phase | Second phase | First phase | Second phase | First phase | Second phase | First phase | Second phase | First phase | Second phase |
| Maize17 | Potato 11 | Onion 9 | Tomato9 | Pepper10 | Maize16 | Potato12.5 | pepper 10.5 | Onion10 | Maize15 |
| Potato 10 | Pepper 8 | Maize14 | Onion 9 | Tomato 10 | Potato13.5 | Maize 13 | Tomato10 | Pepper10 | Onion 11 |
| Onion 8 | Tomato10 | Potato 10 | Maize 15 | Potato10 | Onion 9 | pepper10 | Maize 13 | potato 12 | Pepper 9.5 |
| Pepper 7 | maize 12 | Pepper 8 | Potato10 | Onion 10 | Pepper8 | tomato 11 | Onion11 | Tomato11.5 | Potato 11 |
| Tomato 8 | Onion9 | Tomato10 | Pepper 9 | Maize16.5 | Tomato10 | Onion 10 | Potao12 | Maize14 | Tomato10 |
| 50 | 50 | 51 | 52 | 56.5 | 56.5 | 56.5 | 56.5 | 56.5 | 56.5 |

|  | Crop type. | Year one |  |  |  | Year two |  |  |  | Year three |  |  |  | Year four |  |  |  | Year five |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | First phase |  | Second phase |  | First phase |  | Second phase |  | First phase |  | Second phase |  | First phase |  | Second phase |  | First phase |  | Second phase |  |
|  |  | Ha | Yield | ha | Yield | ha | Yield | ha | Yield | Ha | Yield | ha | Yield | ha | Yield | ha | Yield | ha | Yield | Ha | Yield |
| 1 | Maize | 17 | 1020 | 12 | 720 | 14 | 980 | 15 | 1050 | 16.5 | 1320 | 16 | 1280 | 13 | 1170 | 13 | 1170 | 14 | 1400 | 15 | 1500 |
| 2 | Onion | 8 | 1360 | 9 | 1530 | 9 | 1620 | 9 | 1620 | 10 | 1850 | 9 | 1665 | 10 | 1950 | 11 | 2145 | 10 | 2000 | 11 | 2200 |
| 3 | Potato | 10 | 2300 | 11 | 2530 | 10 | 2400 | 10 | 2400 | 10 | 2500 | 13.5 | 3375 | 12.5 | 3250 | 12 | 3120 | 12 | 4560 | 11 | 3080 |
| 4 | Pepper | 7 | 126 | 8 | 144 | 8 | 160 | 9 | 180 | 10 | 220 | 8 | 176 | 10 | 250 | 10.5 | 262.5 | 10 | 300 | 9.5 | 285 |
| 5 | Tomato | 8 | 2512 | 10 | 3140 | 10 | 3150 | 9 | 2835 | 10 | 3200 | 10 | 3200 | 11 | 3575 | 10 | 3250 | 11.5 | 3795 | 10 | 3300 |
| Total |  | 50 | 7402 | 50 | 8160 | 51 | 8390 | 52 | 8175 | 56.5 | 9190 | 56.5 | 9776 | 56.5 | 10295 | 56.5 | 10052.5 | 56.5 | 1210.5 | 56.5 | 10412.5 |

Table -10 proposed cropping pattern and intensity in ha. Yield /year/phase

| No | Type of crop | For disease |  |  |  |  |  |  | Fore pest cont |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Frequency | Helcozeb 80%WP | Rove 75%WP | Mancozeb 80%WP | Cocide |  |  | Frequency |  | Carboril86% | Endosalphan 25% | Diama cron 50% | Malathione 50% | Cypermetire 10% | Endosalpha 40% |
| 1. | Onion | 3 |  | 2kg/ha |  |  |  |  |  |  |  |  |  |  |  |  |
| 1.1 1.2 | Disease:- dawonmid |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | e purple blotch |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Pest:- thrips. | 3 |  |  | 2kg/ha |  |  |  | 1 |  |  |  |  |  | 0.5lt/ha |  |
| 2. 2.1 2.2 | Potato |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Disease:- lateblight- | 1 | 2kg/ha |  |  |  |  |  | 1 |  |  |  |  |  |  |  |
|  | early Blight |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Pest:- |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | tuber moth | 1 | 2kg/ha |  |  |  |  |  | 1 |  |  |  | 0.8lt/ha | 2lt/ha |  |  |

| No | Type of crop | For disease |  |  |  |  |  |  | Fore pest cont |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Frequency | Helcozeb 80%WP | Rove 75%WP | Mancozeb 80%WP | Cocide |  |  | Frequency |  | Carboril86% | Endosalphan 25% | Diama cron 50% | Malathione 50% | Cypermetire 10% | Endosalpha 40% |
| 1. | Onion | 3 |  | 2kg/ha |  |  |  |  |  |  |  |  |  |  |  |  |
| 1.1 1.2 | Disease:- dawonmid |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | e purple blotch |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Pest:- thrips. | 3 |  |  | 2kg/ha |  |  |  | 1 |  |  |  |  |  | 0.5lt/ha |  |
| 2. 2.1 2.2 | Potato |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Disease:- lateblight- | 1 | 2kg/ha |  |  |  |  |  | 1 |  |  |  |  |  |  |  |
|  | early Blight |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Pest:- |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | tuber moth | 1 | 2kg/ha |  |  |  |  |  | 1 |  |  |  | 0.8lt/ha | 2lt/ha |  |  |

Table 11- in puts required for crop protection

3.8 Proposed cropping pattern and intensity

The main objective of proposed irrigation project is to maximize production per unit area by doubling and rippling the agricultural activity in a determined plot of land by supplying water all over the year in addition to summer rain season for supplementary and full irrigation purpose. Therefore all beneficiaries in project area are capable of implementing the proposed program in term of supplying labor of themselves and their family with appropriates in puts and technologies.

The cropping pattern is already mentioned and it is desirable for the cropping intensity to approach 200%  traditional irrigation and with rain feed agriculture respectively on the proposed cropping calendar and rotation it is assumed that there is interruption of cereal or pulse at the summer rain season there for when we observe consider interruption of cereal and pulse at the summer example on one plot of land faba bean grow at end of June and harvested at end of march and onion transplanted at April 1st and crop intensity grow to 200% with rain fed agriculture over a season cropping calendar.

Crop   Water Requirement (ETcr)

Crop water requirement is the depth of water needed to meet the loss through evapo-transpiration of disease   free, growing in large fields under non restricting soil conditions water and fertility including soil water and fertility and achieving full production potential under the given growing environment.

ETcr =ETo*Kc

3.3.1. Reference Crop Evapo-transpiration (ETo)

Reference crop evapo-transpiration is the rate of evaporation from an extensive surface of 8 to 15cm tall green grass cover of uniform height, actively growing, completely shading the ground and without shortage of water. It is calculated by using the pen man method. The ETo used for the purpose of calculating ETcr is taken from the monthly reference evapo-transpiration mm/day already calculated and given for shambo station from the manual of Oromia irrigation development authority (OIDA) Irrigation crops development extension package first draft 1994 E.C. pp 24.

ETo=[W.Rn+(1-w) f(u),(ea-ed)]

Where ETo= reference evapo-transpiration in mm/day

W=temperature related weighting factor

RN=net radiation on equivalent evaporation in mm/day

F(u)=Wind related function

(ea-ed)= difference b/n the saturation vapor pressure at mean air temperature and the mean actual                                                                                                                                                                      vapor  pressure of the  air both in mbar

C= adjustment factor to compensate for the effect of day and night weather condition.

Length of crop growing stage

| No | Type of crop |  | initial |  | development | mid |  | Late |  | Total |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Tomato |  | 25 |  | 35 | 35 |  | 25 |  | 120 |  |
| 2 | Potato |  | 25 |  | 25 | 30 |  | 25 |  | 105 |  |
| 3 | Onion |  | 15 |  | 25 | 60 |  | 20 |  | 120 |  |
| 4 | Pepper |  | 25 |  | 35 | 40 |  | 20 |  | 120 |  |
| 5 | maize |  | 30 |  | 45 | 55 |  | 35 |  | 165 |  |

There are four main growing stages of crop during its growth period where crop water requirements is vary i.e. the seasonal use of water by plant is determined by their stage of growing.

These growing stages are:

The initial stage- germination and early crop growth ,where the ground cover is less than 10%

Crop development stages from the end of initial stage to full ground stage cover usually b/n 70-80% of the land surface.

Mid season stage from the attainment of full ground cover to the start of maturity indicated by leaf senescence or loss.

Late season stage from the end of the mid season stage until harvest.

During the vegetative stage consumptive use continues to increase. Flowers occurs near and the peak of consumptive use of water. The fruiting stage is accompanied by a decrease in consumptive use until the transpiration essentially cease during the latter part of the formation of dry fruits.

The growing stage of proposed crops for the project are shown on the following table

Crop Coefficient

The effect of crop characteristics on water crop requirement is given by the crop coefficient. It represent the relationship b/n reference (ETo) and crop evapo-transpiration (ETcr) or

ETcr=Kc *ETo

Value of crop coefficients given are shown to vary with crop, its stage of growth, growing season and prevailing weather condition. In our case, the crop coefficients used are taken from the table of crop coefficient (kc). Irrigation agronomy manual revised version, March 1990.

Crop coefficients (kc) for proposed crops

| No | Type of crop | Crop coefficient in growing stage |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Initial | development | mid | Late |
| 1 | Tomato | 0.5 | 0.7 | 0.9 | 0.65 |
| 2 | Potato | 0.5 | o.7 | 1.05 | 0.85 |
| 3 | Onion | 0.4 | 0.6 | 0.95 | 0.95 |
| 4 5 | Pepper Maize | 0.4 0.5 | 0.75 1.05 | 1.1 0.875 | 1.0 0.6 |

Effective rain fall

Effective rain is the proportion of the rain, which is stored in the root zone and therefore available to the plant .Rainfall which percolates beyond the root zone is lost to the plants through run off is not effective, in that it is un available for plant growth.

It is calculated using USBR method using the formula;

Peff= potal(125-0.2potal)/125,for potal less than 250mm/month

Peff=125+0.1 potal, for potal greater than 250mm/month

Where,peff=effective rain fall

Potal=total rain fall in a month (mm/month)

Irrigation efficiency (E)

It is the percentage of applied irrigation water stored in the soil and available for consumptive use by the crop. Field losses consist of surface run off and deep percolation. For small schemes estimate of 50%losses are used, for larger irrigation schemes losses of 30-40% are applicable . From irrigation agronomy manual revised version march,1990 is propose 50% efficiency but for this our  project we proposed 50% starting from long term Experian’s of farmers from traditional irrigation

3.9. Net and gross irrigation requirements.

a) Net irrigation requirement (NIR);- It is the depth of water needed to bring the soil moisture level in the effective root zone to field capacity from the soil moisture. It is calculated by using the relationship b/n crop water requirement (ETcr) and effective rain fall. i.e .Net irrigation requirement =ETcr-effective rain fall.

b)Gross irrigation requirement. It is the net irrigation requirement plus water application losses in the conveyance system due to seepage, evaporation etc

Availability of irrigation water

Even though the command area of the project is large, due to the shortage of water the command area proposed for development is 56.5ha.

The water discharge measurement of the river in question is 34lt/sec but after night storage the discharge is 102lt/sec. The whole command area is after night storage. Hence the discharge is 102lt/sec and the water duty used 1.81Lt/sec/hs. Thus the command area to be developed is equal to discharge measured divided by the water duty i.e Command area =discharge/water duty=102lt/1.8lt/sec/ha=56.5ha.

Therefore, there is enough irrigation water for the proposed command area. For the downstream users (demand) there are streams for peoples and their discharge is measured and studied in the hydrology part. But for Livestock’s, there is enough amount of water near the command area at the right and left near area which is now days commonly used.

3.10. Crop System and Management

Crop management of proposed crops

1. Tomato:- The cropusually sown in the nursery and transplanted after 30-35 days. Under irrigation the crop is grown on ridges. Space b/n plants  on main field at row is 1.2-1.8m and space b/n row is 30cm. tomato fruit is sensitive sunburn and high hot temperature, to decrease this problem by orienting the direction of row to ward prevailing wind direction to increase air movement.

2. Potato:- the crop is grown on ridges or on the flat. Under irrigation the crop is mainly grown on ridges. For planting in rows the recommended spacing is 75cm b/n rows and 30cm b/n plants. A seeding rate of 2000kg (20ku)/ha is required. The fertilization recommendation is 200kg/ha of DAP and 150kg /ha of Urea. Like many other crops, potato during its establishment is very susceptible to competition from weeds. Therefore, weed control during the early stages of growth essential.

3. Onion:-The cropusually sown in the nursery and transplanted after 30-35 days. In the nursery the row distance is about 10cm. in the field about 10cm. in the field a bed which has 60cm width is required and on this bed, two rows of 20cm apart are required. Seedlings will be planted on two rows at a distance of 7cm b/n plants.

A seeding rate of 3.5-4 kg/ha of seeds is required. Fertilizer recommendation is 200kg /ha of DAP and 100kg /ha of Urea. A planting depth

Depth required is 3-4cm. to prepare seedlings for one ha the area needed is 560m2.

Pepper:- the seed rate required is 0.6-0.75kg/ha. It is planted in double rows on flat topped ridge. Double rows spacing is 40cm and the space b/n double rows is 80cm. spacing within the rows is 40cm and plant population /ha is 42,000 plants. Depth of planting is 4-5cm. fertilizer recommendation is 100kg/ha of DAP and 100kg/ha of Urea. Seedlings of 10-12cm height are transplanted in the field after 25-35 days. To prepare the seedlings for one ha the area needed is 300m2. The plants are some time topped 10 days before transplanted to encourage branching. At its early stage the crop is easily attacked by weeds so that it should be free of weed in the first months. It is also susceptible to breakage and to shed its flower and thus, it is better stop movement inside the field of the crop.

3.11 Cultivation and plant protection

Cultivation: - is for purpose of removing weeds which compute with the crops for water, light and nutrients. Cultivation must be timely that Is while the weeds are still small and before any seeding takes place.

Tomato:- tomato is a vegetables which can be tolerate weeds competition but after 30-40 days of transplant the initial weeding is recommended for aeration of soil at root depth and to remove weeds. Finally when plant is start to flowering sticking should be done to support the plant from feeling down during fruit bearing.

Potato:- The effect of cultivation should be to earth up the ridges, to kill the weeds and to control tuber moth. Cultivation during the growing period must avoid damage to roots and tubers and the ridges are earthen up to avoid greenings of tubers.

Onion:- onion does not compute well with weeds. Timely shallow cultivation to destroy weeds while they are still young is essential for the production of good crops.

Pepper:- cultivation to control  weeds should be aimed at building up the ridges, particularly if they become washed down by heavy rain.

3.12.Plant protection

Tomato:-

Early blight (Alternaria solani) :- It is a fungal disease which causes canker and coulter rot on the steam of seedlings and young plants in the fields.

On the leaves the fungal develops the spot that may partially defoliate the plant and reduce the yield and quality of fruit. The control methods are seed treatment with fungicides, spraying with copper compound or organic fungicide and rouging of infected plants in the seedbeds

Bacterial spots (Xanthomonas capestris):- It is bacterial disease which cause wilt on fruits, leaves and seedlings.

Fruit spots begin when dark raised dots are surrounded by the water soaked border. Spots continue to expand until the fruit matures. Older spots are black, slightly  raised, and superficial and up to 1/3 inch in diameter.

They have lobed margins and water soaked borders or holes. The center of the spots disintegrates, sinks and sharply sunken to form a crater with a ruptured epidermis evident at its margin. The leaf spots on mature leaves are small, black and same what angular.

Control method:-captafol 80% wp at 0.3% concentration and Ridomil MZ 63.5% wp at 0.27%, separately or in combined form, significantly reduce the incidence of blight and septoria leaf spot.

Late blight (phyatophtera infestant).

Symptoms; the fungus causes severe defoliations and a very destructive rot of the plants. The first symptoms are irregular, greenish-black and water soaked spots on the leaves. These spots enlarge rapidly in moist weather conditions and sometimes show white, downy growth on the lower surfaces. The stem often shows spots similar to those on the leaves. Fruit infection occurs near the stem end and it may take place at any stage of growth. Small, grayish-green, water soaked areas, which develop on the fruit, enlarge rapidly and may cover half of the fruit. The spots become dark-green, and blotched with brown as the fruits become older. Rainy and cloudy environment is suitable is suitable for them.

Control methods; Culturally, crop rotation, frequent plowing and exposing the soil to the sun are recommended and using fungicides also controls the disease

Potato:-

Diseases

Late blight (phytophterainfestans)

This is a fungus disease. The fungus causes defoliation and a very destructive rot of the plants. The first symptoms are irregular, greenish black and water soaked spot on the leaves.

This spot enlarge rapidly in moist weather and same times show white, downy growth on the lower surface, the stem often show spots similar to those on the leaves.

Control measure are using resistance varieties, rouging out infected plant, crop rotation, removing host weeds, spraying Helcozeb 80% powder at the rate of 2kg/ha mixed with 400-600lt of water and spraying one time during growing periods.

Early blight(alternaria solani)

It is also fungus disease. The fungus causes rot on the  canker and collar rot on the seedlings and young plants in the field. On the leaves, the fungus develops spots that may partly defoliate the plants and reduce the yield and quality of the crop.

Control measures are the same as for late blight.

Pests

Potato tuber moth (phthorimaea operculella)

The caterpillars have a brown head and a triangular brown patch behind the head. Young caterpillars mine the leaves, producing brown blotches. The destruction caused by this insect is not limited to tubers in the field but it will also destroy the tubers in store by boring. It can be controlled by using clean seeds, field sanitation of plant residues, spraying Diamacron 50% at the rate of 0.8 it per ha. Mixed with 200 liters of water.

Aphids (Myzus persical)

There are several kinds of aphids that feed on the potato plant. The winged aphids lay eggs on the plant hatch out in to wingless aphids and the number increases rapidly during warm dry weather.It can be controlled by spraying malathion at the rate of 2. 0 It /ha mixed with 200 liter of water.

3) Onion

A) Diseases

1.  Downy mildew (pernospora destructor)

The fungus coats the outer surface of affected leaves and they soon turn yellow and die. The disease usually appears at a few points in the field and spreads rapidly under favorable conditions.

It can be controlled by use of crop rotation, use of resistant varieties, and spraying Rov 75% powder at the rate of 20kg/ha mixed with 400 liter of water and spray three times at an interval of 10 days.

purple blotch(Alternaria porri)

Purple blotch occurs on the leaves, seed stems and bulb. It first shows on the leaves as small, sunken white, circular or irregular spots. These spot have purple centers and increases in size to form elongated purplish blotches which may be bordered with organ bands. Darkened zone consisting of massed of spores develop in the center of lesions. Usually the infected leaves turned to yellow and die within three to four weeks. Flower stalks are often girdled and fall over grows down the leaves in to the bulb .infected bulbs appear same what watery at first. Later the tissues turn deep yellow, then gradually dark red and finally dark brown to black. The spores may remain viable in the soil or plants refuse for over a year. It can be controlled by use of crop rotation, use of resistant varieties, spraying Mancozeb 80% powder at the rate of 2.0kg/ha mixed with 400 lt of water and spray three times at an interval of 10 days.

B)Pests

1. Thrips (thripstobaci)

They are sucking insects. They are small yellowish which attack the leaf, central leaves curled and deform. They are more injurious during dry period. The control measure is spraying cypermeterin 10% at the rate of 0.5 lt/ha mixed with 400-600lt of water.

4) Pepper

A) Diseases

1. Virus diseases;- virus disease on pepper are caused mainly by cucumber mosaic virus, tobacco mosaic virus, and alfalfa mosaic virus. There are different symptoms of affected plants such us yellow spots on the leaves, dwarf growth, poor fruit setting, small fruits, extremely malformed leaves, etc. the virus is transmitted by seed, sucking insects and by the laborers during transplanting, etc.

Control of virus disease consist to large extent on sanitation to prevent infection, distraction of host plants near to the field, as well as destruction of infected seedlings and using virus free seed is important control measure.

2. Powdery mildew;- Powdery mildew occurs both on hot pepper and on sweet pepper. The symptoms are chlorotic blotches on the upper side of the leaves and powdery blotches on the lower side. The control measure is spraying cocide when powder like cover starts to seen on the leaves at the rate of 1.5 kg/ha mixed with 400lt of water two times at an interval of 10 days.

B) American boll worm;- the worm attacks the  pepper by entering in to the un ripen fruits and feeding on the fruits. The measure is spraying endosphan 40% liquid at the rate of 1.5kg/ha mixed with 200lt of water two times at an interval of 10 days.

Weeds;- for the control of weeds in all crops, cultivation timely weed control by hand weeding while the weeds are young before any seed setting are recommended.

3.13. Irrigation Practices and Harvesting

Irrigation practices;- Irrigation is any process other than natural precipitation which supplies water to field crops, orchards or any other cultivated land. In arid and semi-arid regions, irrigation water is applied to produce crops, where as in other regions where the natural rain fall or to lengthen the periods of crops. For all the proposed crops the irrigation methods recommended is surface irrigation by furrow. The amount of irrigation water applied for the crops vary depending on the growth stages of crops, climatic condition and soil types. Based on these conditions, for the proposed crops the recommended irrigation intervals are mentioned below.

1. Tomato:-Tomato is relatively resistant to soil moisture deficit. But it need enough amount of water during transplanting, flowering and fruit bearing. Irrigation interval is 7-10 days during normal growth.

2. Potato:- potato is relatively sensitive to soil water deficit. Water deficit during the period Stalinization, tuber initiation and yield formation have the greatest a diverse effect on yield, where as ripening and the early vegetative period as less sensitive. Intervals of 6-7 days b/n irrigation are recommended for the crops.

3. Onion:- As it is common with most vegetable crops, onion is sensitive to water stress. When the soil is kept relatively wet, root growth is reduced and this favors bulb enlargement irrigations should be discontinued as the crop approaches maturity (15-25days before harvest) t0 allow the tops to dry out, and also to prevent a second flush of root growth. Intervals of four days b/n irrigation are recommended for the crop.

4.Pepper:-  Due to the low depletion level, light irrigation application are required. Irrigation interval of 8 days is recommended

Summary of irrigation method and irrigation intervals for the proposed crops.

The irrigation method:- recommended for the proposed crops on the project site is surface irrigation by furrow method.

The irrigation intervals:- irrigation intervals depend on the climatic condition, crop development stage and soil condition. Based on this condition of the project area the following irrigation intervals are recommended after fully establishment of crop.

| No | Crops | Irrigation method | Irri interval/day | Remark |
| --- | --- | --- | --- | --- |
| 1 | Tomato | Furrow | 8 |  |
| 2 | Potato | Furrow | 6 |  |
| 3 | Onion | Furrow | 4 |  |
| 4 | Pepper | Furrow | 8 |  |
| 5 | Maize | Furrow | 8 |  |

Harvesting

Tomato;- It is gradually fruit bearing and maturing vegetables except for few commercial varieties, there for it is recommended that continuous harvesting of green reddish colored fruits when it reached  for harvesting. Rather than fully ripened reddish colored fruit it is better to harvest fully sized greenish red colored fruit for better shelf life and to avoid damage during transportation.

Potato:- if the leaves of healthy potato plants start to be wilted or if the leaves start to be yellow, it is the sign of maturity and thus discontinues watering.To make the skin of tubers strong for far distance transportation and not to be attacked easily by diseases, it is good to cut the parts above ground before harvest and keep the tubers in the soil for a week. Then, with good care harvest the potatoes and keep under shade to avoid greenings by sun.

Potatoes can be harvested for immediate consumption in an immature state, usually from the time it reaches full flowering. At this time the skin is thin and soft and the potatoes cannot be stored.

Onion:- when the bulbs developing from the leaf bases of both garlic and onions are fully formed, the leafy green tops begin to be yellow and eventually collapse at point a little above the top of the bulb leaving an upright short neck. When the tops go down the bulbs are ready for harvesting. When the crop reaches maturity, it is good to discontinue watering (15-25 days before harvesting) to allow the upper leaves to dry and to prevent the initiation of roots growth for the second time. Dry the harvested onions 5to7 days in the field by covering the bulbs with the leaves of other onions. Since on the onions do not mature at the same time, harvest progressively at the tops go down to store the dry onions for sale or use at a later time.

Pepper:-sweet peppers are harvested when the fruits have reached full size but are still green. By picking the fully shiny colored fruits and keeping under shed and cool places take to market within a short period of time. Keeping harvested fruits under cool and sunny condition makes the fruits to be wilted and there by leads to loss of market demand. However, during the latter stage of the crops, fruits may be breaking color or be fully red.

3.14. AGRICULTURAL SUPPORTING SURVICES

AGRICULTURAL EXTENSION

Agriculturalextension is the transfer of new improved technologies from the research center to the user /farmers to make them benefited of the disseminated technologies.

The extension package plots on farmer field performed around the project area    On cereal crops are serving as demonstration sites to make the farmers familiarize with the use of improved cultural practices such as good seed bed preparation, timely sowing, weeding etc. and the use of improved seeds, fertilizers, herbicides and pesticides. These helped the farmers to get an optimum output per unit area of land. But concerning the vegetables, there are no improved seeds available both on credit bases and for sale around the project area and the production is also less both in areal extent and yield. Thus, strengthening the irrigation extension with qualified development agents to give technical advices for the farmers on the use of irrigation water, land and proposed improved agronomic practices for production is very essential.

Supply of agricultural inputs

The agricultural inputs which are proposed for the project to be used by the farmers should be made available to the farmers   of the area to use as proposed and to get the optimum proposed output. The concerned enterprise is needed to supply the required inputs based on the felt need of the farmers requested by extension worker working with farmers.

Credit and marketing

Farmers of the area have got an experience of using agricultural inputs on the extension package plots for rain fed agriculture. These different inputs were supplied of extension to the participant of extension package programs performed on the farmer’s plots.

The inputs supplied for extension package program were given to the farmers on cash to buy. Due to this reason the farmers of the area already not access to use credit system to get the inputs. Thus, for the irrigated agriculture the required inputs should also be arranged to be on the credit bases to facilitate for the farmer on how to get the inputs and use them accordingly to full fill the objective of the project. After crops are produced the burning issue is the means of transportation facility. But during the study of project the road is considered and may solve the problem of the area.

Conclusion and recommendation

The construction of this project can be increase the productivity of the land and create a job opportunities for a young land less farmers. The climatic condition of an area is favorable to produce the proposed crops which are short growing period and highly productive.  This can be good opportunities for the demand of the market. In addition to this the traditional experience of the farmers on the production of cereal crops, fruits, vegetables and stimulant crops at the area can be highly increase the production and productivity of the farmers and improve the subsistence  agriculture of the area. This is a great strategic action to support the government’s transformation and development plan by producing high value and productive crops 2 times a year. Finally it is important to train development agents at the project area and water use committees on the modern ways of production and appropriate use of water. Farmers should be introduced with improved seeds and fertilizers with timely supply and credit facility.

4  Introduction

4.1Geological Study

Since agriculture is one of the principal means of bringing sustainable development in the country, it is vital to plan and modify the farming system by adopting modern agricultural practice and utilizing naturally available water resources for irrigation purpose. In this line, the regional government has planned to implement irrigation projects, with the help of various funding agencies and by its own budget in order to solve food security problems. One of these budgets, which are playing significant role in the region in this irrigation sector, is the Millennium Development Goal fund (MDG). The Oromia Irrigation Development Authority is the sole agent or client to implement the engineering identification, study and design, and later construction supervision MDG supported small scale irrigation schemes following various approaches; on force and/or contractual modalities. For the design of various stable and cost effective engineering structures in different parts of the project, the areas were investigated to gather engineering geological and geotechnical input data. This engineering geological feasibility study was carried out from March 18 to 30, 2006EC.

Geotechnical investigations are performed to evaluate those geologic, and soil conditions that affect the safety, cost effectiveness, design and excavation of the proposed project. The fact that insufficient, faulty interpretation of results, or failure to portray results in a clearly understandable manner may contribute to inappropriate designs, delay in construction schedules, costly construction modifications, use of sub-standard borrow material, environmental damage to the site, post construction remedial work and even failure of a structure and subsequent litigation. Hence, geotechnical investigation is usually thought of as the backbone of engineering structure design.

Investigations performed to determine the geologic setting of the project include the geology (nature and behavior of rock materials), soil and rock behaviors, and seismic conditions. This parameters influence the construction site selection through the control of the characteristics of the foundation soil and rocks, geotechnical conditions, project safety, design, and construction, critical geomorphic processes and source of natural construction materials.

Based on the above facts, the site selection criteria and factors that should be taken in to account in studying the diversion site are as listed below:

Nature of subsurface material at the Weir axis and potential cannel route with the fact that they should be sound and resistant to bear the load imposed by the weight of the overlying materials, including the structures to be built.Suitability (shape and depth) and stability of the valley slopes at the abutment and capability to bear the lateral pressure imposed by the nearby soil material.

Nature of the abutment ground material in reaction with water (solution resistant rock, soil, or nature of clay material Uniformity of geological formations, geological structures and their continuity.

Topographic conditions of the river both upstream and downstream of the anticipated weir axis.Relation of the anticipated dam axis with the potential canal route and the available command area.Availability and quality of naturally occurring construction materials within an economically justified distance from the project site.

4.2 Objective of the geological and geotechnical investigations

The primary objective of the geological and geotechnical study of the project is to assess and analyze geological and engineering geological data related to stability, workability, water tightness and bearing capacity of the various foundation materials encountered at the exact sites of hydraulic structures involved in the irrigation system. The study further aims at assessing the exact location, quantity, quality, workability and proximity of naturally occurring construction materials that are considered to be proportionate with the magnitude and type of hydraulic structures to be constructed. Moreover, the study is targeted to forecast engineering geological risks and provide possible mitigation measures that will have to be considered in the detail design and construction of the project.

The main objectives of this phase of study are briefly summarized below.

To assess the foundation condition of the Headwork site.

To assess the foundation condition of the pond site.

To assess the stability of the river banks (abutments) of the Headwork site.

To study canal route and crossings from stability and water-tightness perspectives.

To investigate local construction materials with respect to quality, quantity and distance of hauling.

To collect representative soil samples and conduct simple field test to determine engineering behavior of the foundation and canal route materials.

Approaches and Methodologies

To achieve the above main objectives the following methods/procedures were used during this phase of investigation.The approach and methods followed to undertake the engineering and geo-technical study involves two major activities: the office work and field work which of vital importance to achieve the intended objective. The office work includes the revision of literature (previous works), reports, and maps (geological, hydro geological, topographic, etc) to get secondary data. The fieldwork mainly involves the acquisition of primary data from visual site observation on rock exposures, outcrops; test pitting at selected points.

Materials used

To facilitate the geotechnical feasibility study assigned for the project work and to address the objectives set for it, the following materials were used during field investigation.

Shovel

Measuring Tape meter,Topographic map at a scale of 1:50,000 to view the relief of the area, Handheld GPS,Digital camera and Sample Bags.

4.3. Location and Accessibility of the project

Location

The study area is located in western part of Oromia regional State, Horo Guduru Wollega Zone Jimma Rare district to West direction of Wayu town in between Gudata Dobi and Bada Worke kebele, Laga Aba Oromo site, in western part of Ethiopia at the distance of 301km from Addis Ababa, capital city of Ethiopia, 114 km from Shambo, zonal capital & 17 km from Wayu town, district capital.  The geographic co-ordinates of the site is defined by the UTM location of 1018818mN, 0307223mE and river bed elevation of 2273m a.s.l.

Accessibility

Laga Abba Oromo site is transverse by the main gravelly road from Shambo to Wayu crossing Fincha town and another gravel road branched from Wayu which connect Wayu to project site to the west direction of Wayu town. As the option there is also another gravel road which connect Hareto town with Wayu town and the project is found between Hareto and Wayu.  Generally the study area is well accessed with vehicles.

Physiographic and Drainage

The average elevation of the basin ranges from 2348-2292m above sea level. The area is also characterized by flat to gentle slopes.

The studied area is drained by Perennial River & seasonal streams which drain the area from South West direction to North East direction. All the major streams are seasonal and also the area is drained by a number of small intermittent streams. Generally the study area has a dendritic drainage pattern.

Climate, Soil and Vegetation

The climate of Ethiopia ranges from equatorial desert to hot and cool steppe, and from tropical savannah and rain forest to warm temperate, from hot lowland to cool highlands. Jimma Rare woreda is divided into three agro-climate zone mainly due to variation in altitude (Kolla, WeinaDega Dega and Dega). But, the project area belongs in Dega. The mean annual temperature of the woreda is found between 15-200c.However, there is a slight variation of temperature from months to month. For most of the areas in the woreda, the rainy season starts in March and extends to October with the highest concentration in June, July and August. The number of rainy days varies from cool moderately to cool in the woreda. Jimma Rare woreda receives abundant and well-distributed rainfall both in amount and season, which is conducive for different types of vegetation growth and agricultural activities. The soil type that covers the study area (catchment area) is red clay loam and black fertile soil hence most of people use actively farming. Due to fertility of the soil there is various vegetation covers along the spring flow.

Topography and settlement

The topographic feature of study area was seems like rough land with elevation difference. The settlement pattern of the area was densely populated.

4.4. GENERAL Geological Setup

Regional Geology

The Ethiopian volcanic province is characterized by two volcanic series: the Tertiary Trap Series of the high plateau and the recent (Pliocene-Quaternary) volcanic series. Contemporaneous with the major uplift of the Horn of Africa, extrusion occurred from fissures and centres of immense quantities of flood lavas (Traps). These lavas covered the greater part of the Mesozoic rocks in Ethiopia.

The Trap Series consists of a very thick succession of lava flows, chiefly flood basalts (Traps), but with Trachytes and Rhyolites occurring especially near the top of the Series. The Trap Series essentially pre-dates the Rift faulting and therefore forms the great heights of the Ethiopian plateau. The Pliocene-Quaternary Volcanic Series post-dated the Rift faulting and is largely confined to the floor of the Rift System where it lies on the Trap Series (Mohr, 1962).

The project site is situated in the western region of the country. In the Western region, some part of the geological terrain is mapped as Nekemte sheet which is the eastern extension of Gimbi sheet (GSE, 2005).The regional geology of the project and the surrounding area is characterized by the rocks of Precambrian, Mesozoic and Cenozoic eras. The Precambrian rocks in the area form the basement of all other rocks. They are metamorphic rocks and are considered part of the Proterozoic rocks of the Western Ethiopian Shield, which is assumed to be the southern extension of the Arabian-Nubian Shield (ANS) (e.g. Aya1ew et al., 1990; Berhe, 1990). Amenti,(1989) have studied the Proterozoic rocks of Western Ethiopia and classified them into three N-S running zones: the western high grade gneisses, the central low-grade volcano-sedimentary belt and the eastern high-grade belt. The low-grade volcano-sedimentary belt bounded both to the east and west by high-grade gneisses (Amenti 1989). The study (project) area is indicated to be part of the Mesozoic rocks dominantly Sandstone and  Cenozoic rocks in the western region which comprise rocks that are assumed to be the result of the tertiary flood volcanism (Trapean series) which gave rise to a pile of basalt, intermediate and silicic rocks. Mohr (1962) classified these volcanic rocks in to trap series which includes Oligocene to Late Miocene plateau flood basalts. The terrain underlying Laga Abba Oromo area is specifically situated in the volcanic of tertiary flood basalts.

Geology of the Project area

Geologically, the project area is the solid geology (hard rock geology) that underlain the project area is portion of the Cenozoic volcanic formations that characterize the Ethiopian Western Highland. These formations are known to be various lava and phyroclastic flows dominated by basalt rocks and agglomerates. At the project area only one type of rock uniformly characterize the area. It is the result of lava flow during the Tertiary volcanic eruptions. The rock type is aphanitic basalt. This rock unit presently observed at some portion of the stream bed and banks, including at headwork site, and slopes of some of surrounding hills. It has characteristic dark gray color, and fine texture. The rock unit has been affected by joints and weathering at some places, and it is massive and fresh elsewhere. These secondary phenomena plays significant role for its geotechnical characteristics.

Following the Tertiary volcanic basaltic rock formation, the area has been affected by different surface processes of weathering, and sediment transportation that superficial soil are developed and cover some areas of the site. These superficial soil materials observed at the site are classified into five major units for the sake of this project. These are:-Silt, with some clay, Floodplain Deposit,Sand with Gravels and boulders, recent loose alluvium,Silty Gravel, with few clay and some boulders, dense old alluvium,High Plastic Clay, black cotton Clay, residual soilSandy Silt, residual soil

Quaternary Sediments of mostly residual soil is also found on the flat parts, mainly in the proposed command area and downstream parts of the proposed dam site. The materials that compose this deposit are dominantly composed of silt and clay. The thickness varies from place to place and thickness increases away the river channels and the central flood plain in the project area.

The proposed Headwork site is also characterized by boulder rocks mixed with the soil and fractured basaltic rock beneath the boulder. This rock type is exposed on bed and banks of the river both in upstream and downstream of the proposed Weir axis and in command area. Along the proposed main canal and command area, the rock is overlain by deep soil cover. The soil type is red silty clay soil.

4.5. Hydro geological Condition of the Catchment Area

The whole terrain of the study area is covered by extrusive volcanic rocks mainly trachi-basalts. These rocks are subjected to weathering, fracturing, jointing and faulting at varying degrees.

In some localities the Basalt is massive, slightly weathered and fractured. Thus, the secondary processes have very small contribution in the overall water circulation and storage capacity of Basalt. On the contrary, there are places where block fractures divided the massive Basalt into rectangular blocks. Mostly these fractures are open to a considerable depth and transmit large quantities of water. Therefore, in most localities Basalt developed good secondary permeability largely from open fractures and to some extent from weathering zone. When there is high degree of fracturing and weathering, Basalts have the capacity to hold water and become a productive aquifer.

The geology of the Catchment area, topography, geologic structures (faults, fractures, inclined bedding planes) and conservation practice in the area provoke surface run-off and leads to infiltration/percolation and finally to replenish the groundwater (i.e. high recharge). Springs are also common along the river side’s. Hence, Laga Abba Oromo River is effluent stream and thus, it becomes wider and deeper downstream due to increased discharge.

During the field survey no major soluble rock that creates hazardous water or soil quality problems is observed. The trachi-basalts rocks in the study area are generally categorized under low permeability Aquifers and fractured basalt as moderate to high productive aquifer. The most upper part of the catchment area is classified as Recharge area. To improve the discharge of the river and minimize erosion some soil and water conservation, construction of ditches, a forestation practice etc is very important on the upper part of catchment the catchment area.

4.6. Engineering Geology of the proposed Headwork site

Engineering Geological and Geotechnical condition of Weir foundation

For this project a simple river bed level intake structure has been proposed for diverting the water to the main canal. The headwork site geological surface and subsurface conditions have been investigated based on the nature of the proposed structure. At the site and immediate vicinity, the stream flows along moderate slope course and its bed is mainly covered with recently deposited alluvial sediments. These sediments are dominated with cobbles and boulders at the top and become sand and gravel size fragments to depth. At some portions of the stream bed, especially at both left and right ends, there is an outcrop of the bedrock (basalt rock).  On the other hand, the banks of the stream at the headwork site are made up of different geologic materials; most part of the right and left banks  made up of old alluvial sediment of pebbles and cobbles cemented by silt/clay materials. It is dense due to consolidation of the sediment. The detail geologic nature of the banks, and bed of the stream along the headwork axis and immediate vicinity are described and their potential geotechnical influence on the proposed structures also discerned/detected below, with remedial measures.

The weir site is selected around the existing traditional diversion site, which is also the best location with the key consideration like topography, command area, stability of the river banks, engineering geological properties of the materials and canal routes.

In general, selected Headwork Site is on Bedrock River, a river that typically has alluvium mantling the bedrock over which it flows. At the selected Headwork site is at medium river gradient and the bed is exposed bed rock. The prominent rock type present in the weir site is hard basaltic rock. The rock is exposed at some place, and overburdened by soil material and transported materials like cobbles, boulders and gravel, in the upstream and downstream side of the proposed Headwork. Hence the foundation has to be considered as relatively impervious basaltic bed rock of strong, weathering resistant and with good bearing capacity after some excavations of rocks.

In both sides’ river banks, the slope is low to moderately steep and the slope consists of rock exposure of the same rock type. Since the proposed weir is located on bed rock, there is no stability problem on banks and river bed and also enables to prevent flow of water beneath the Weir.

Stream Bed

At the proposed headwork site the stream bed or course is well defined, nearly straight, and shows rough surface due to recent sediment accumulations and undulating appearance of bedrock outcrops. Along the intake axis, the bed is made up of two basically different geologic materials, as seen from surface observation. These are recently deposited alluvial coarse grained sediments, and clay soil.

The central areas of the bed (lower elevation portions) are totally covered with the alluvial deposits like boulders, cobbles and gravels. The bedrock is totally covered with the sediments at the central areas of the stream bed. To know the thickness of the sediments or depth to bedrock, two manual test pits has been dug to 3m from river banks. Due to excessive subsurface water inflow to the pit, the excavation has been interrupted before encountering the required bedrock.. it is hence, the bedrock depth reaches up to 3 to 4m, at narrow sections of the bed (at center), and exposed on the surface at left and right ends, till both banks feet .

Figure 1:Geological Cross-Section along the proposed Weir

Geological & Geotechnical Condition of Canal Route and Command Area

Canal Route Geology

Canal route geology is observed from water tightness, stability and workability point of view. The main geotechnical problems with regard to canal route are water-tightness and stability problem along the ridge sides and where there is deep excavation.

For this particular project, the main canal route has length of about 1.9km and secondary canal with length of 870m. Within this length, the canal passes different geologic materials, which varies both surfacially and to depth. Based on the geomorphology, soil type and observed problem, the main canal is classified in to three. The first 0+200 m of the proposed Main canal alignments are aligned along the left side banks of the river and proposed to be lie mostly on the low to moderate steep slope and soil rocks. Thus, workability problem (rock excavation) is not expected on such section of the canal. The next 0+200m to 0+1050 m (up night storage) the main canal is aligned on the foot of a ridge (small hill) which is characterized by colluvium deposit (cobbles, pebbles and gravely clayey sandy silt soil.  Leakage of water may be occurs on the section of the canal due to lack of appropriate canal bed gradient (slope) and permeability of the soil. The remaining section of the canal both main canal and secondary canal after pond site is alighted on gentle slope characterized by black silty clay soil, inorganic silty clay of medium plasticity. The depth of soil is greater than 2 m.

4.7. Groundwater Condition

Groundwater is convenient when we require a water supply to be able to drill or dug a well into an aquifer; insert a pump and abstract water. What is less convenient is that groundwater will flow into any excavation in the saturated zone. Groundwater plays considerable role in many geological processes. The properties of engineering geological materials are often controlled by groundwater. Moreover, changes in groundwater or moisture conditions have great impact on the stability of slopes, river banks and other civil engineering structures. Shallow water tables should receive considerable attention if an engineering structure or a part of it extends to subsurface. Knowledge of groundwater level fluctuations and positions is essential for determining uplift pressures on the floor and foundation of structures. Hence most types of engineering geological investigations require acquisition and use of groundwater data. The need to determine ground-water conditions for a site is one of the major reasons for performing a subsurface investigation.With regard to the specific project (weir) the materials that cover the surrounding of the area under question are generally impermeable and problems like water flow to excavations, slope stability problem due to groundwater, change in engineering behavior of materials, etc is not critical. Moreover groundwater depth is very deep i.e. below the depth of our interest or foundation depth.Very slight construction problem is expected from surface water at the base of the structure (trench), crossing number one and crossing number three. Diversion or Dewatering systems or other methods of surface water flow can be designed to provide dry conditions for construction at the initial phase.Determination of the physical, bacteriological and chemical quality of water is essential for assessing the suitability of water for various purposes like agriculture/irrigation, drinking, domestic use and industries. A study of the chemistry of water helps in interpreting the geological environment, source, direction and movement of groundwater, recharge-discharge relationships, presence of ore bodies, etc.  Knowledge of water quality is essential for proper management of water, land resource, irrigation projects, etc. Even though the water is not tested by laboratory it is good for livestock for drinking and for irrigation and is not polluted water.

4.8. Availability of Natural Construction Material

One of the major activities conducted during this study is assessment of local construction materials from quantity, quality, workability and distance of hauling point of view. The main local construction materials required for the proposed project are stone for masonry, sand and gravel for concrete/aggregate and water for mixing and curing.

Sources of rocks for masonry works

From the general geology of the area, the dominant rocks are basalt. The rock type have identified in immediate vicinity of the project. Thus, an outcrop of basalt identified to be used as stone masonry and aggregate production. The rock is occurred on the top and sides of small ridges and hills at a distance of about 3km to 4Km, which is located just both left and right side of the proposed head work site. In this area the rocks are outcropped on the surface and, therefore there is no need to excavate any overburden.

In general, the existence of the rocks as blocks, boulders, cobbles and gravels indicate the workability of the selected rocks as masonry stone and therefore there is no need to excavate any overburden.

Quantity: Excess rock source is available in the area. In general quantity of masonry stone or limitation is not a critical problem to this project.

Quality: Stones for masonry shall be quarried, fractured, or otherwise predominantly angular. All stones shall be hard, sound, durable, and highly resistant to weathering. Generally the rock is strong, non-friable, weathering resistant, and workable to any size and shape.

Distance of Hauling: The distance from the weir axis ranges from 3km- 4km.

Gravel source: is also proposed from the area selected as rock source i.e. by crushing manually or with crusher plant or from Wayu town as another option.

Sources of Sand

Quantity: sufficient quantity of sand is available on the identified source.

Quality: The sand used for mortar and concrete shall be well graded and material passing standard sieve size 0.075 mm.,the selected sand is found suitable to be the required purpose without washing or any other processing.

Distance of Hauling: The distance from project to Wayu 15km and from wayu to Agamsa 70km gravelly road.

Sources of Water

Water is available along the Abba Oromo River. Water used for mortar and concrete Shall be clean and free from detrimental concentration of acids, alkalis, salts, sugar and other organic or chemical substances.Therefore, the proposed Abba Oromo river water is estimated to be suitable for the specified works.

Conclusion and Recommendation

Conclusions

The dominant litho logic unit in the area is basalts. From strength and distance of hauling point of view, basaltic rock selected to be used for masonry stone and aggregate production. Its suitability has been verified with in-situ. Suitable Water and sand sources are also available at reasonable distance.

The rock is exposed at some place, and overburdened by soil material and transported materials like cobbles, boulders and gravel, in the upstream and downstream side of the proposed Headwork.

Based on surface observation and inspection, the proposed command area is covered by back silty clay soil. Surface and sub-surface investigation will be carried out when the alignment is determined in detail feasibility period.

From data collected during the fieldwork, observation of existing traditional canals, hand dug test pits excavated along the proposed canal route; no excessive seepage is expected except at some places of the proposed canal, especially around the weir and at about 0+200m to 0+1050m, in which the canal is to be aligned in the foot of the ridge characterized by semi-permeable colluvial material.

After the night storage the canal is divided in to two directions.

Recommendations

Based on the results, analysis and interpretation of geological, geotechnical, hydro- geological, topographical/geomorphologic and engineering geological data the following recommendations are forwarded from long and short term point of view. As already mentioned the central foundation of the weir is sedimentary deposits of cobble, boulder and gravel rocks with admixtures of alluvial silt and sand soil. So to stabilize the weir foundation, the transported materials should be removed or excavated

The main canal route, starting from 0+200m of the weir up to the pond site i deviates from the main river needs some erosion protective structure. This part of the river bank (left bank) is susceptible to erosion and local mass movements. To prevent from these active geodynamic earth processes this section of the canal route needs special design.

The main canal 0+200m to 0+1050 m (up night storage) the main canal is aligned on the foot of a ridge (small hill) which is characterized by colluvium deposit (cobbles, pebbles and gravely clayey sandy silt soil.  Leakage of water may be occurs on the section of the canal due to lack of appropriate canal bed gradient (slope) and permeability of the soil. Moreover in areas (canals) with deep excavation due consideration should be given to stability problem and the canal should be lined canal.

Secondary canal crosses the river by flume structures and around the proposed flume hard flooding were expected and the measure should be taken.

The foundation of the flume is proposed on the transported silt soil and clay soil for the flooding of river the measure should be taken.

Due to the foundation of the flume is soft materials to decrease the over load and fear of subsidence the measure should be taken.

5. Hydrology

5.1 Description of the Project Area

Laga Abba-Oromo irrigation project area is located in western part of Ethiopia, Oromia regional state, Horo guduru wellega zone, J/Rare district specifically at Gudata Dobi and Bada Worke peasant associations.

This project area is found in the North eastern direction of the zonal capital called Shambo and Southern direction of the district capital called wayu.

This area has about 114km far from this zonal capital & about 17km from the district capital.

The catchment area of the head structure measured from topographic map of 1:50,000 scale of [0937-A1] (GEDO) & [0937-1] (BAKO) sheet is 84 km2 wide from the head of the main water course to the diversion (weir) site.

Thus the project area has the grid coordinate of 1018827N latitude 307226E longitude of UTM.

The average altitude (elevation) of the project area at the target of irrigable land is2240 m.a.s.l

According to agro climatic classification (i.e.) low land (desert) [<1500m.a.s.l], tropical humid [1500 to2500 m.a.s.l], humid (high land) [>2500 m.a.s.l], the catchment area is categorized under tropical humid agro climatic zone.

The maximum length of the main water course from water shed divide to the proposed diversion (storage) site is 7km.

The altitude (elevation) of the water shed divide from the head of the main water course to the stream bed at the proposed storage site ranges in between 2560 to 2270 m.a.s.l. The main gradient of the catchment is [4.1%][0.041].

Finally this river drains at Fincha-a sub basin.

For Hydrologic analysis from the total catchment area the estimated land use coverage is about 70% cultivated [farm land] with settlement, 15% forest (with bush and shrubs), 14% grazing [pasture] land and 1% Fallow (un cultivated land) was considered in the catchment.

Since project area has got a general topography of flat to gently undulating topography & the behavior of this basin is fan (sector) shaped catchment that can contribute high runoff water response for the downstream because all the tributaries are approximately of the same size. Thus the catchment gives greater run off since the peak flood from the tributaries s likely to reach the main stream approximately at the same time. Thetexture of the soil in and around the command area is dominantly clay loam that has high water holding capacity. The top soil type (surface) is clay loam and the sub soil is loam as it is expressed more in soil part of this project.

5.2 Objective of the study

(A) General objectives

The overall objective of this study is to deal the systematic and scientific use of irrigation system that is not only the application of water to the irrigable land but also with the planning and design of various structures relative to the command area and the water potential of the stream flow.

(B)   Specific objectives

The main objective of the hydrometeorology study is to gather and analysis field and other hydro meteorological data necessary for design of the irrigation scheme to be implemented. These data include climatic data, river flow data and study on river morphology, then give more &more confidence in predicting hydrological events such as RF, Runoff, Ground water accretion etc..

In addition objective of the study is to achieve overall responsibility of keeping the irrigation working system in satisfactory manner with the limitation imposed by the initial farmer’s traditional existing scheme and to increase the number of beneficiaries and command area with maintaining and up grading the existing traditional scheme, in order to increase the irrigation need and match with the water potential by this upgrading study.

Also identify the problems of the water losses that caused by seepage and percolation and giving a solution to minimize this water loss in order to increase the potential.

To determine the Availability of water for irrigation development and planning of the existing water resource, among the target group of up and down stream users.

Also to determine the maximum design flood of the river for water balance analysis, to design a long line diversion structure and more over, to determine the drainage modules of the specific project area.

To estimate the hydro geological features and the ground water of the proposed study area.

5.3 Methodology and Materials used

Using 1:50,000 topographic maps of the area & hand held GPS in field assessment during our field visit & grid square for area measurement.

Information and discussion with the elder groups of the local farmers (settlers).

Climatic data from national meteorological agency.

Grid square sheet for calculation of area of the catchment.

Site surveys & X-section

Using guide lines & manuals related to the study

Information and data from District Agricultural Office and Development agents

Field assessment, taking measurement, data collection

Using  stopwatch and tape meter

Primary data’s are collected by simple visual observation, interviewing the setters, plan metered topographic map of 1:50,000 scale, floating method & flood mark method of discharge measurement & using GPS & altimeter.   In this study secondary data’s are interpreted from hydrological and meteorological data’s that are collected from national meteorological agency & hydrological data. Then after gathering all these primary and secondary data’s the final report of the area was prepared in detail manner.

N.B:- The More over irrigation development with respect to water availability, Stream flow and demand analysis in relation to crop water requirement and agro climatic elements are discussed in detailed manner at this section and in agronomy part of this document.

5.4 SCOPE

The scope of the study is to study the basic hydro meteorological element related to design of head structure, supply canal, drainage canal & other irrigation hydraulic infrastructure.

CLIMATIC ELEMENTS THAT DESCRIBES THE AREA

(A) Hydro- Meteorological Information

The project area experiences one long rainy season that starts in May & lasts in September. Since from the month of October to December has almost lesser amount of rain fall. Thus during the month of December has lesser amount of rain fall.  While during the months of January & February gets little rain fall (almost no RF).

(B) Surface Water Resource Assessment

[I] Meteorological Elements and Data Interpretation

All the weather data’s of meteorological parameters  such as precipitation(mean annual RF), monthly average maximum & minimum temperature, monthly average relative humidity, monthly average  wind run(wind speed) ,sunshine Hr and such meteorological data’s  are taken (collected) from the main zonal district office called Shambo meteorological station as representative of laga abba oromo River small scale irrigation project. Because of having the same (homogeneous) climatic behavior the similarity in elevation &agro climatic categories at its source area & also there is efficient [luck] of meteorological station near (around) the project area. The project area has a distance about 114km from Shambo meteorological station. By taking the project area under consideration these meteorological parameters best describes both the catchments [watershed] and the command area.

[II]   Data Availability & Quality

The quality of the collected data is good in terms of consistency, methods of observation & standards of measurement, coherence & fitness of correlation. The meteorological elements analyzed below are rainfall, temperature, and relative humidity, wind speed and sunshine Hr as relevant to hydro meteorological & engineering Agronomy study. Details are depicted as under in tabular form.

Fig. [1] Graphical Representation of Monthly Rain Fall [mm] from 2000-2013

TABLE (1):-Summary of average monthly Rainfall from (2000-2013), Max & Min temp (2002-2011)

(A)DISCRIPTION OF METEOROLOGICAL PARAMETER AND ESTIMATION OF ETo

USING Pennman METHOD----------------------------------------- [2011G.C]

(B) MONTHLY AVERAGE RAIN FALL [mm] AND MONTHLY EFFECTIVE RAIN FALL [mm]

Where; Eff. RF=Ptotal(125-0.2Ptotal)/125     for; Ptotal < 250mm &

Eff. RF=125+0.1Ptotal          for; Ptotal>250mm

Ptotal-is total monthly precipitation (RF)

Using Kohosla’s (1960) method is used to analysis of RF runoff.

Which gives us:-

Rm = Pm – Lm and Lm = 0.48Tm for Tm >4.50c

Where; Rm = monthly Run off in cm and Rm > 0

Pm = monthly Rain fall in cm

Tm= monthly temperature & Lm= monthly losses

Table (2);- summary of the meteorological parameter of monthly Mean annual RF (mm); Effective RF (mm) and Annual runoff (mm) those are taken from shambo meteorological station.

Source: - National meteorological agency Adama branch data management & dissemination depr’t.

Fig. [2]Graphical Representation of Monthly Rain Fall [mm] & Effective RF[mm]

Fig. [3] GRAPHICAL REPRESENTATION OF ALL METEOROLOGICAL PARAMETERS

(C) INTERPRETATION OF DATA FROM THE ABOVE CLIMATIC TABLE

From analysis the mean annual RF depth is 1543.4mm = 1543mm& the mean monthly depth of RF is 129mm & the mean annual runoff will be 95.24cm = 952.4mm.

The maximum RF occurred during the month of July which is 369.5mm, while the minimum [little] RF occurred in the month of February that is 1.2mm rainfall depth.

The rainy season (from May to September) =97% of the total RF.

Maximum & minimum mean monthly temperature is 22.70c&11.3 oc respectively. While the average annual temperature is17.04 0c.

The hottest months are February & April (26.00c) & the coldest month is October (100c)

Monthly Condition of general climate

Irrigation Season

(D)  STATISTICAL PARAMETERS OF ANNUAL RAINFALL SERIES

As mentioned above monthly mean annual RF &T0 data summarized in table (2(B)) helps us for evaluation of mean annual runoff depth & the run off coefficient.

Since for the run off depth and Annual run off coefficient for the given catchments from the available historical rainfall and temperature data relationship; Kohosla’s (1960) method is used to analysis of RF runoff.

Which gives us:-

Rm = Pm – Lm and Lm = 0.48Tm for Tm >4.50c

Where; Rm = monthly Run off in cm and Rm > 0

Pm = monthly Rain fall in cm

Tm= monthly temperature

Lm = monthly Losses in cm

So from the above relation the annual catchment run off Coefficient is 0.617=0.62 & the mean annual run off depth is 95.24cm= [952.4mm].

And thus the annual runoff volume (ARV) will be:-

ARV=MAR*CA*1000=952.4mm*23*1000=21,905,200m3 of annual run off

Where; -   [ARV]-Annual runoff volume (m3)

[MAR]-Mean annual runoff (mm) &

[CA] - Catchment area (km2)

5.5 BASE FLOW [THE STREAM MINIMUM FLOW] MEASUREMENT

For better water planning different base flow analysis methods have to be used, from which the most appropriate value is selected for the design. The methods comprise floating, catchment yield & monthly base flow using rain fall frequency.

[A] FLOATING METHOD

Base flow for Laga abba Oromo River

Zone – Horo Guduru Wellega

District – J/Rare

Name of the stream – Laga abba Oromo

Location (Grid coordinate):– 1018827N latitude 307226E longitude of UTM.

Place of measurement; - at the selected diversion site

Method apply to evaluate is Floating method

from the field data observation the steam flow evaluation was performed as below:-

Data of observation – 03/07/2006 E.C

The total length travel taken using floating method = 5m

Width = 2.8m

* The time elapsed to travel 4m length using floating method and releasing different cross section of the flowing river 5 times is taken as follows and also the surface and average (Actual) velocity are calculated respectively.

T ave=Ttotal/n=20sec

Average time = 20 sec

Surface velocity (V)=Length/Tave=5m/20sec=0.25m/sec=0.25m/sec

Average velocity (Actual velocity) =V*Ca; where Ca is the coefficient factor which is equal to 0.85.

So Vave=V*Ca=0.25*0.85=0.213m/sec

Surface velocity and Average velocity is respectively 0.25 m/s and 0.213 m/s as calculated above

Laga Abba Oromo discharge measurement

∑Q=40lit/sec

Table [3]:- Summary of the estimation & measurement of minimum flow of the Laga abba oromo River.Hence the total minimum available measured water potential of the project using floating method is 40 lit/sec during the study period.

Since the measurement of this river lean flow was taken twice that is during this pre and detail study.

(B) Catchment yield

From Dr. Admasu’s case study the lean flow is;-

QL=0.85*P*A0.83

Where, QL= is estimated monthly stream flow (low flow) in mm3/month

A= cathment area in Km2

P= Regional monthly flow ratio:-

=0.011(Region-16 according to hydrological classification of the country by Dr. Admasu Gebeyehu

Therefore QL=0.85*0.011*230.83 =0.1262 mm3/month =48.7 lit/sec

[C]Monthly base flow estimation using Rain fall frequency

Table [4]:- mean monthly RF & monthly base flow

Fig (4);-graphical representation of monthly mean RF & base flow

As mentioned above the discharge obtained at the weir site is 40lit/sec using floating method and 48.7 lit/sec by catchment yield.

Since as mentioned above our field visit is in the most driest month of the year and there is no recent rain fall contribution to become the base flow to be increased (delayed flow) and the measured discharge is not a combination of prompt inter flow and delayed flow. Therefore we decide that even if the measurement is taken twice (during pre feasibility & this detail study) the direct base flow measurement is taken using floating method. So it is selected for the design.

Thus the total estimated minimum flow will be 40lit/sec from this total minimum flow by considering the downstream user’s about 15% of the discharge is supplied.

Therefore 34lit/sec of water is implemented for the design & used by upper & middle catchment farm hoods by this irrigation scheme as it studied.

5.6Irrigation demand and potential

The potential irrigable land on laga abba oromo site can be as high as 150ha. Out of this 56.5ha is planned for this upgrade irrigation scheme.

The demand for irrigation is dependent on the crop water requirement which is in turn a function of cropping pattern, crop type, cropping schedule, climatic factors and so on.

For 100 % cropping intensity and 50% over all irrigation efficiency summery of monthly crop water requirement in l/s/ha as calculated in irrigation agronomy section is 1.81

5.7 Water Planning

The ultimate goal of water planning (water budget analysis is to estimate the shortage or surplus of the water resource from which irrigation is planned. Such planning should be with respect to irrigation potential, irrigation system demand, upstream and downstream user’s right that brings a project in to sound irrigation development unit.

In the project area, the potential land to be developed by irrigation is about 56.5ha. From agronomy the water duty of the site is 1.81 L/s/ha.

As observation during field visit shows, there were traditional irrigation systems seen at both upstream and downstream.

WATER BUDGET

For further hydrological information about the Laga Abba Oromo River, it is not gauged either at the head or at the footer of the proposed site. In order to explain (describe) this river morphology of water resource characteristics the river is perennial that flows continually all over the year & there are perennial and intermittent rivers and springs of tributaries those recharge the main river.

[I] Water balance analysis

For the downstream users as mentioned above there are small springs and also there is a tributary river. And also additionally by taking consideration for their cattle & other uses from the measured total minimum flow 15 % of the discharge is released and 85 % of the total measured discharge is implemented for the design. Form the total water shed area the expected volume of water is about 21,905,200 m3 (21.905Mm3) of runoff per annul. Therefore by taking the consideration for their cattle water demand consumption of this project area in both kebeles will be mentioned as;-

Table [5]:-description of cattle water demand consumption

Since as mentioned above from the total minimum flow 15% is released that is about 518400 lit/day relatively their cattle wants about 182738 lit/day.

(A) SURFACE WATER

Hence the area has long precipitation and high rain fall as observed from the meteorological data, the project area is characterized by sufficient rain fall amount.

Since this area has sufficient rain fall amount that has a great contribution for recharge this river.Also this river is coming to this project site after traveling about 7km and it gets many springs and tributaries that recharge the Laga Abba Oromo River as one can understood from the topographic map. Those tributaries are perennial and intermittent those recharge the main river all over the year.

Therefore this all have a great contribution for having a good surface water resource to the project area.

(B) GROUND WATER

The high precipitation period of the area has a good contribution to the local ground water reservoir. Although the area has tributaries that is added to the Main River and drainage part that is discharged around the area. Moreover as mentioned above there are different perennial and seasonal spring that are discharged around the area. But from the local farmers’ information, the physical observation of the general geological feature and topography around the project area and the test pits the ground water table will be expected as at shallow depth.

5.9 DESIGN FLOOD ANALYSIS

Design flood analysis is the flood discharge adopted for the sake of design of a hydraulic structure with its safety to be highly structured, by taking careful consideration of hydrologic condition. This is the highest flood expected in return period of design interest here 50 years return period is used for weir design. The project design flood is used for the design against failure. It is determined by estimating the run off that result on occurrence of the design storm based on the meteorological factors. This hydro meteorological approach is necessary because of stream records are often not available.

COMPOSITE HYDROGRAPH METHOD

[I] PROBABLE MAXIMUM FLOOD ESTIMATION BY STATISTICAL METHOD

[A]Maximum one day RF historical data

The maximum one day RF historical data is taken from Shambo meteorological station.

The location of this meteorological station is at 9034’07’’N latitude& 37006’35’’E longitude with an altitude of 2400 m.a.s.l that have about114km air distance from the project area.

Since there is no recent daily max RF recorded data but for estimation we use the past 38 years [1969-2006 E.C] recorded data.

The recorded data of max 24 hr RF data is for 36 years [1969-2006] for this estimation and arranged in decreasing order for storm analysis & expressed in [mm] as follows;-

Table [6]:- Descriptions of Maximum one day RF historical data

N.B:- Since there is only one rain fall gauge around the project area is used, the rain fall is applied over the entire area. So the average depth over a watershed area can be adjusted for the different size of a watershed by Areal Reduction Factor (ARF).

Therefore the design rainfall values will be obtained by applying the area reduction factors (ARF). The factors are calculated using the following formula or using graph.

ARF = 1-0.044A0.275

Where, A = catchment area in km2

ARF=Areal Reduction Factor

ARF = 1-0.044A0.275 = 1-0.044*230.275 = 0.895 = 89.5%

ARF =89.5%

The average rain fall depth over 23km2 would be 89.5% of 24-hour point rainfall which is 124.05mm.

Therefore, the design rainfall will be (0.895*124.08) =111.05mm=111.05mm

Table [7]; - Description of maximum 24 hr historical data with probability

∑x = 1956.2                                                        ∑x2 =120248.15

∑X mean= 51.48                                                    [∑X2] mean= 3164.425

[∑X mean] 2=2650.08

the plotting position formula which has the following form was developed by Weiball;-

Pe (%) = [M/N+1]*100 see table [6] of (column 5)

Where; - Pe - the position of probability of accidence’s of the empirical points.

M-rank number

N-total number of observations

[B] RETURN PERIOD APPROACH

[I] Hydrologic Frequency Analysis

From the relation of the worst storm estimation with 50 years return period becomes as follows:-

The standard deviation of the series is:-

Sp= [(n/n-1) ([∑X2] mean-[∑X mean] 2)] 1/2= [(38/37) (2862.27-2397)] 1/2

= [(38/37) (3164.425-2650.08)] 1/2 = (1.027*514.35) ½ = (528.23) 1/2

Sp = 22.98 mm

The extreme value law that the annual max. Value approaches a definite pattern of frequency distribution when the number of observation in each year becomes large.

The event ‘P’ of return period of 50 years is:-

P50=Pave + Sp (0.78YT-0.45)

Since YT=-[0.834+2.303log (log (T/T-1))] =3.9

Where T-is return period=50Yrs

Therefore the design storm with 50 years recurrence becomes:-

P50=105.6mm=111.05mm

P50=111.05mm

Where; - PT50= Design storm with 50yrs recurrence interval in [mm]

Pave=Average value of precipitation in [mm]

Sp=Standard deviation of the event in [mm]

YT= frequency factor & n= number of years recorded=35yr’s data

5.10 THE PEAK FLOW ANALYSIS

For safe structure design associated with risk of failure which should be justified for different design flood analysis of approaches is used among which the best value for the design is selected.

The alternative can be: - RF frequency analysis, Complex unit hydrograph, Empirical (Dicken) Formula.

[A] RF Frequency Analysis

Daily maximum rain fall (mm) of Shambo meteorological station that describe above in table [5]

YT=-[0.834+2.303log (log (T/T-1))] =3.9

Where T-is return period=50Yrs

From table for n-38yr’s    K=(Y-Yn)/Sn= 2.52

Then Qp= Q+KSp; where, Q=∑X mean =51.48mm

Thus   QP =51.48+ (2.52*22.98)

Qp=109.4 m3/sec

[B] PEAK RUN OFF CALCULATION using unit hydrograph method

Table [8] Design flood parameters;-

From the inflow unit hydrograph, the peak discharge is 48m3/s at the 1.4 hours

Fig.[5] Graphical representation of Unit Hydrograph

Thus the design flood was found to be 48m3/sec, from the upstream catchment.

[C] Rechard’s Method

For the Laga Abba Oromo river case the peak flood estimated using Rechard’s method will be:-

QP=0.278*K*I*f (a)*A

Since for this case:-  A=23 Km2                 f(a)=1-0.044(22.3)0.275 =0.90

Lc=7K m                        Tc= [0.87L3 /H] 0.385 =1.01hr

S=0.041                        I= 84(Tc+1) 0.475 =277.8mm

H=290m

Then moreover all the measured and estimated values are described below in the tabular form

Table [8];- Estimation of peak flood using Rechard’s Method

Therefore as seen by using Rechard’s method QP = 84m3/sec

[D] Using Empirical Formula

Taking Dicken formula, that is: -

Q=cA3/4

C=14 to 28 for mountainous and

C= 2.8 to 5.6 for plain.

So, for this project catchment take C=14

Q=14*233/4 =14*10.5= 147 m3/sec

Qp= 147m3/sec

Table [9];- Summary of peak flood as analyzed by different methods

[5.9] Analyzing hydrological data using Empirical for predicting the mean annual  flood

5.11Sediment Transport & water quality

[A] Sediment Transport

Laga abba Oromo River has a sediment flow or high amount of silt deposited during the rainy season. As visually observed recently there is alluvial deposit exposed along the main stream side of the river edge and within the command area that accumulate during the rainy season (maximum flow of the river). This shows us there is a sediment flow around the project area.

[B] Water Quality

The quality of Laga abba Oromo river water is not checked by laboratories. But from the farmers experiences, that use the river water at the upper and downstream by traditional irrigation system describe the stream have a good quality of water. This deduced that, it has no negative impact or ill effect on their crops and on their cattle also. This indicates us it have good quality of water.

CONCLUSION AND RECOMENDATION

As an overall opinion regarding hydrological& hydro geological features of the project the water resource potential relative to the command area of the catchment yield is insufficient. The discharge obtained at the weir site is 0.04m3/sec & from this 0.06 m3/sec is implemented for the design and the rest is released for the downstream users.

In order to get enough (sufficient) water potential it is possible to use water planning that is advisable. Water planning is a wise use of water among different stake holders in the upstream and downstream of the project area. It is simply balancing the water budget for safe use without causing conflict among different beneficiaries especially during peak water demand periods. Also to increase the potential water resource relative to the command area of the catchment supporting with night storage (pond) will be recommended.

The design discharge of the maximum flow (the peak flood) performed by using complex unit hydrograph with 50 years return period occurrence of the worst storm to be 48m3/sec.

If so the hydrologic situation implies there is no as much problem to be the project done, i.e the site is feasible with regarding to water resource assessment & the hydrologic condition done during the studyperiod

6.HEAD WORK, IRRIGATION AND DRAINAGE SYSTEM DESIGN

Dildala Abba Oromo River is located in GU/HO/Wollaga Zone,Jimma Rare  Woreda at the two  intermidate kebeles :Gudata Dobi kebeles at the right side and Beda Warke Kebles at Left side of the river. The proposed site location for the head work is about 17km from Wayu and 114km from Shambu[Zonal place] Geographically, thelocation of weir site is 288200m East Longitudinal, 1070621 North latitudinal and at 2268m altitudinal elevation. The water resources of this river is initially comes from Shoa Zone from two waredas. Oda Gudaya Kebele : Wabo, Kersa,Gorash, Ali,Lege Abba Gizaw streams and come give the Gudaya Kebele: Wabo, Kersa. Gorash, Ali, Lege Abba Gizaw streams and come give the name Bongollo River at the down stream of the drainage and finally take the Dildila Abba Oromo River by obtaing small tributaries…So,there is shortage  of water due to the implementation of this project in downstream, upstream,and within the project area. Diversion weir is selected based on the suitability of land i.e slope,stability,and other criteria. Farmers of the two kebeles have Started to use this water traditional system in the downstream side of the river. The proposed irrigable land is about 150ha but from  agronomy 56.4ha.

As visually observed  the physical feature of the command area generally can be classified as plain/flat terrain with the slope less than 2%.

Back ground Information about the project, detail design of the weir, protection works, stability analysis of the weir and retaining walls, design of appurtenant structures like sluice and intake gates. Finally, the overall conclusion & recommendation of the headwork is made.

Background Information

The Project area, which includes location & accessibility, climate, soil and water resources, Hydrology, Geo-technique & Soil parts are dealt in introduction part of the project document.

Both general and specific objectives of DILDILA ABA OROMO  SSIP are discussed in detail in the socio –economy part.

Methodology: The following procedures are followed to come up with the detail weir design of DILDILA ABA OROMO SSIP.

Appropriate site selection is made depending on weir site selection factors and irrigable command area as well as community parbticipation.

Section at downstream of the weir axis, a tail water depth is calculated.

Average river slope is determined.

Hydraulic design of the weir body is followed & structures like retaining walls, sluice, and Intake are designed.

Finally a plan view or layout of the weir with its accompanying structures is seen on a headwork contour map prepared.

Shape of the weir: - The shape of the weir is selected based on the existing conditions of the site, run of the River and the simplicity and practicability during the construction time. The existed diversion site shows that there are silt coming from the upstream of the weir site and accumulated around and in the courses of the river. Hence, to safely dispose the flood with silt a broad crest weir is preferable.

Length of the weir: - From the physical field observations and peak flood the width of the river course the weir crest length is around to be 10m.

Summary of Costs

| No | Work Item | Cost (ETB) | % Cost | Comm. Share |
| --- | --- | --- | --- | --- |
| 1 | Access road, Camping & Mobilization | 365516.10 | 6.3 | 504458.187 (10%of total main cost) |
| 2 | Head Work | 1971592.30 | 33.98 |  |
| 3 | Canals and other irrigation infrastructures | 2707473.47 | 46.67 |  |
| 4 | Total Costs | 5044581.87 | 86.9 |  |
| 6 | supervision cost | 290063.45 | 5% |  |
| 7 | VAT | 756687.28 |  |  |
|  | Grand total project cost | 5801269.15 |  |  |
| 6 | Cost per hectare | 89442.94 |  |  |

The study and design of the project had been undertaken in a multidisciplinary approach by the following crew of technical experts:

| No. | Type of Activity | Name | Remark |
| --- | --- | --- | --- |
| 1 | Engineering Aspects | Tesfaye Darse |  |
| 2 | Surveying and plotting | Indalo Wakjira |  |
| 4 | Geology | Ambaye Alamgena |  |
| 5 | Hydrology & Watershed Management | Qumneger Teshoma |  |
| 6 | Socio-Economy, Financial analysis and Community Participation | Tesfaye Cimdessa |  |
| 7 | Project Cost Estimation & Document Compilation | Tesfaye Darse |  |
| 8 | Irrigation Agronomy | Tareqegn Olana |  |
| 9 | Environment impact Assessment | Tesfaye Tolasa |  |

6.1 Hydraulic and structural design of Head work

6.2. weir Design

Geology of the Weir Site

The geology of the weir site is composed of alluvial deposits from the size of boulders up to small sized gravel around the riverbed. From observations of the test pits on the axis of the weir on both edges of the river the profile up to a depth of3m-4m is characterized by the fractured basaltic rock.

6.3. Estimationof tail water depth

The design peak flood discharge for the specific catchment is 48m3/sec.

a. Average River Bed Slope

Dildala Aba Oromo River x-section of the river at Weir

Longitudinal profile of River

U/S cross section at  weir

Downstream cross section at the weir

6.4River Discharge Curve Calculation

Stage discharge curve

After preparing the Rating Curve (plotting discharge versus depth from the above table), the tail water depth (depth of water during the expected maximum flow, d3) is estimated as 1.74m.

Therefore, high flood level of the river (HFL) before construction of the structure is:

HFL = 2269.087m + 1.74m = 2270.827m. @ FM

- Manning’s Roughness coefficient of the River, n is assumed to be 0.014.

- Average River bed slope as calculated earlier is0.0101

- The maximum Flood Mark (collected data) is 2270.827m  or D/s water level

- Average River bed level =2269.087m

- Maximum Flood Depth = 1.74m

From the above Discharge curve, the Design Flood equivalent to the Flood Mark(2270.835m) is 48m3/sec. Here the flood mark is equivalent to the Tail Water Depth (TWD). There fore from the water Depth-Discharge  Curve the maximum discharge from the tail water depth equivalent to 1.74m from river bed level is found to be 48m3/sec.

From the above Discharge curve, the Design Flood equivalent to the Flood Mark is 48m3/sec. Here the flood mark is equivalent to the Tail Water Depth (TWD).Thus, for the structural design of head work peak discharge of 48m3/sec is used. Thus, for the structural design of head work peak discharge of 48m3/sec is used than the result of the UH method which is 47m3/sec. This all is to reduce fear of overturning in all the cases.

6.5 Determination of weir crest level

a. Length of the Weir

Taking in to account the width of the river bank, 10m of the length of the overflow section has been taken.

b) Determination of weir Crest Level

a. Average elevation of the highest field in the command area= 2269.58(from canal bed)

b. Water depth required = 0.3m (canal Water depth)

c. Assumed freeboard =0.15m

d. Head loss along the main canal up to highest field=Slope of canal * distance from

the weir = 1/1000x150=0.15m @ first turnout i.e frictional head loss

e.   Head loss across head regulator

Head loss due to intake pipe=Entrance + Friction + Exit loss

Pipe head Losses:

Considering the pipe was a plain concrete of diameters   20cm having a total length of 8m, and      0.034m3/s canal discharge,

Q=dry discharge (m3/s)

A=area of pipe (m2)

V=velocity of flow through the pipe (m/s)

Cross sectional area, A = ЛD2/4 = 3.14x0.22/4 = 0.0314m2

Velocity, V = Q/A = 0.034/0.0314= 1.08m/s

Coefficient of roughness for pipe, n = 0.014

Coefficient of friction loss,

f = 124.5n2/D1/3 = 124.5x0.0142/ (0.2)1/3

→       f = 0.042

Coefficient of losses

- Friction loss, fL= f L/D = 0.042x8/0.2= 1.67

- Inlet loss, fi = 0.20

- Outlet loss, fo = 0.3

Total coefficient of loss, fT = fi+ fL +fo

fT = 0.2+1.68+0.3

fT = 2.18

Total pipe head loss, hL = V2/2g x fT

Where, hL = head loss though the pipe at entry, due to frication and exit (m)

V= velocity through the pipe (m/s)

fT= Total friction coefficient loss

L=length of the pipe (m)

D= diameter of the pipe (m)

hL= (V2/2g) x fT

= (1.082/19.62) x2.18

hL = 0.1296m

Average river bed elevation=2269.087m

Total head loss=hL+ WD+ hL turnout=0.1296m+0.3m+0.15m

=0.58m

Therefore weir crest level=2269.087m +0.58 +0.15(freeboard) =2269.817m

Weir height =Weir Crest level-bed level

Crest height =2269.817m -2269.087m

= 0.73m~1.4m

Hence crest level= river bed + 1.4m =2269.087m +1.4m=2270.487m

Height of crest, p= 0.50m (silt height = 0.90m assumed)

iii) Man hole: is provided between the intake pipe and a steel bar mesh is used as at the inlet of the pipe to protect debris and other foreign materials.

D) DYKE: - dyke also provided at the right side of head work to control high flood of the water.

d. Water Depth on the Crest (He)

Shape of the weir: considering no boulder coming from the upstream of the weir, broad

Crested   weir with vertical upstream face is designed.

Crest length of the weir: taking in to account of water depth and physical field

Observation, weir length of 10m is considered.

Discharge over the weir is generally expressed as:

Figure 2- 1 Flow condition over broad crested weir

Q=CLHe3/2

Q=1.70LHe3/2

Where:      Q=Peak flood (m3/sec) =48 m3/s

L=Weir crest length (m) = 10m

C=Coefficient of discharge =1.7 for broad crest weir

He=Height of specific energy   line above the weir crest (m)

He= [Q/CL] 2/3

He= (Q/1.7L) 2/3 = [(136m3/sec)/ (1.7*20)] 2/3=2m

He = 2m

Therefore Hd =1.9m

Hence Hav= He- Hd

=2-1.9=0.1m

Total energy, He=2m

Overflow depth, Hd = 1.9m

Velocity head, hva = 0.1m

6.6. Determination of stilling basin bed level

Design of Stilling Basin

a) Energy Method

Design information

۰ Average river bed elevation = 2269.087m

۰ D/S water level		= 2270.827m (BL+TWD@FM) =2269.087+1.74

۰ Weir crest level		= 2270.487m

۰ weir crest height		= 1.4m

۰ F (afflux) = He + crest level – down stream water level

= 2 + 2270.487m– 2270.827m= 1.66m

Bed level of stilling basin:

First find the jump height, D2 or sequent depth.

Assuming the floor level is 0.50m lower than river bed,

Assumed EL1 = EL3-0.50 = 2269.087-0.50=2268.587m

From energy equation Eo = E1

E0 = He+ weir ht = h+ hvo+ weir ht = 1.9+ 0.1+1.40= 3.4m

q = Q/L  = 48/10  = 4.80m3/s/m

From Bernoulli’s energy equation b/n u/s & just to left of jump:

Z0+ weir ht + h + hv= Z1+d1+V12/2g

0.50+1.40+1.9+0.1 = d1+V12/2g

3.9 = d1+ v12/2g -----------------1

V1= q/d1 = 4.0/d1

V1 = 4.80/ d1   ---------------------2

Substituting 2 in 1:

3.9 = 4.80/V1+V12/19.62

By trial & Error V1=8 m/s By changing v1

Hence d1 = 0.6m

Froude number, Fr =

d2 = m

The tail water depth d3=1.74m is less than the sequent depth, d2 and thus the jump recedes downstream for downstream floor level 0.50 m lower than upstream apron level.

The length of the stilling basin is:

L = 5(d2-d1)=5(2.5-0.6)=9.5 or 4d2 = 4x2.5= 10 Take L basin=10.00m

Where    He = 2m

Crest level = 2270.487m

D/S water level = Average river bed level + TWD (@FM)

= 2270.487m +1.74m = 2270.827m

F = 2 + 2270.487m – 2270.827m= 1.66m

Hydraulic Jump

Assuming the floor level is 0.50m lower than the river bed level,

Assumed EL1 = EL3 - 0.50m

= 2269.087-0.50m

= 2268.587m

H =Eo = E1

Z = ELO – Assumed EL1

2270.487m - 2268.587m

= 1.9m

H = He+z =2m+2m=3.9m

Where Z=total energy head the static case (m)

Discharge intensity over the weir crest qc = Q/L=48/10=4.8m3/sec/m

Eo = E1 = d1+hv1    Where hv1=V12/2g

= d1+V12/2g but V1= qc /d1

By the principle conservation of energy,

E0=weir crest level He-Assumed EL1

Eo =2270.487m +2.m- 2268.587m

= 3.9m

For, qc =4.8m3/sec/m and Eo= 3.9, by energy conservation

d1+V12/2g = 3.9

d1+ (qc/d1)2 1/2g = 3.9

d1+ [(3.9)/ (d1)]2  1/2x9.81 = 3.9

d1 = 3.9-1.174/d12

By trial and error methods

d1 = 3.9-1.174/d12

Applying Bernoulli’s equation

dc+ Vc2/2g +H = d1 + V12/2g

For, Discharge intensity over the weir crest qc = Q/L=48/10=4.8m3/sec/m

But dc = (qc2/g) 1/3

= (4.82/9.81)1/3 = 1.33m

And Vc = √dcg or Vc = qc/dc

⇒Vc = √1.67x 9.81 = 3.6m/sec

V1 = q/d1

∴ dc + Vc2/2g + H = d1 + V12/2g = d1+ (qc/d1)2/2g

d1 = 3.9-1.174/d12

d) Length of percolation

a) Lane method:

According to Lane, the percolation distance (creep length) can be expressed as

L>C*H, for hydraulic structure to be safe against piping failure, the creep length L, should be equal to or > the product of coefficient & seepage head.

VT=2+1.5+1.5+1.3+2+2 =10.3

HT=3+2.5+11.2+2+2=20.7

Where         LT= creep length (m) = VT+HT/3 =10.3+20.7/3=17.2m

C= percolation coefficient (Lane creep coefficient)

= 2.5 in case of Lane’s for boulders,cobbles and gravel including sand soil foundation type.

Hmax= maximum head in meter=1.66m for dynamic head

Note: for sloping surface > 45 degree; take as vertical otherwise horizontal.

The maximum static head should be used for design purposes.

L≥ C*H

17.2m ≥2.5x1.66m

17.2m ≥4.15m

So the Lanes methods tell us that the apron provided is safe against piping & seepage.

b) Blight method:

L > C*H

Where LT – total percolation distance (m) = VT+HT=10.3+20.7=31m

H - Maximum head on the structure (m) =1.66m

C –percolation coefficient (in case of Blight creep coefficient)

= 9 for boulders,cobbles and gravel including sand soil foundation type.

L > C*H

→ 31m   >   9*1.66m

= 31m >14.94m safe

Weighted Creep length total LC= VT+HT/3 =10.3+20.7/3=17.2m………… by Lane method

LC=10+20.7/3

LC=10.3+6.9

LC=17.2m

Calculation  of u/s and d/s cutoff

Depth of scour:-

R1=1.35(q2/f)1/3     q=Q/L=48m3/s/10m=4.8m3/s/m

F=1

R1=11.045 take 5

a)U/S CUTOFF

d1=1.25R-HD-h

=1.25*5-1.9-1.4=2.95 but take d1=2m

b) D/S CUTOFF

d2=1.5R-d3=1.5*5-1.74=2.5m

c)APRON LENGTH

U/s apron length

L=2d1~2*0.6~1.2~3m

U/s apron thickness=0.5 from past experience

D/s apron  thickness checked as below table

Static case (Hmax=1.4)    By Lane method

| Point | Weight creep length | Hmax [1-LA/LC] | (TWL-WLA,B,C)= (d3-d1,d2,d3) | t= (Safety factor/rm-1)X (i+ii) |
| --- | --- | --- | --- | --- |
| Case | H V L Static case (Hmax=1.4m) | I | ii | Saf.fac. =1.3& rm=2.3 |
| A B C | 5.5 10.3 12.13 16.7 10.3 15.81 18.7 10.3 16.58 | 0.41 0.11 0.05 | 0 0 0 | 0.41 0.11 0.05 |
|  | Dynamic case (Hmax=1.74) |  |  |  |
| A B C | 5.5 10.3 12.13 16.7 10.3 15.81 18.7 10.316.58 | 0.51 0.14 0.06 | = (1.74-0.6) +0.5= =1.64 =1.74-2.8= 0 =1.74-1.74 = 0 | =50%*1.64+0.51=1.33 0.14 0.06 |

However, for dynamic case, the term TWL-WLA should be reduced by 50%.

i.e 1.33x 50% = 0.82⇒ Total =  0.82+0.51= 1.33

Checking the thickness at each point

Point A      2.5>1.33---------------ok (dynamic case) take 1.4

Point B      0.95>0.11--------------ok (static case)

Point C      0.8> 0.05---------------ok (static case)

d) Exit gradient (Ge)

It tells us whether the downstream cut is sufficient.

The final exit gradient upward thrust should not exceed certain value for the foundation materials to avoid piping. Safe exit gradient for coarse sand soil is limited to values of 0.17 to 0.2.Khosla formula to determine exit gradient at downstream side for structure with floor length, B and vertical cutoff, d is;

Ge = Hmax/d* 1/ (√λ),

Where,

α =B/d =11.4/1.5=7.6   B= HT=20.7   d=2.5m downstream cut-off

λ = (1+√(1+ α ^2))/2

λ= (1+√ (1+8.28^2))/2

=4.64

G e is the exit gradient, which must be less than 1/5 for fine gravel

Ge = (1.4/2.5)*(1/ (3.14√ (4.64)))

=0.083≈ Safe

The total depth of 2m of the downstream cut-off wall proves to be sufficient to safeguard against piping for materials of coarse sand soil. From observations of the test pits geology report on the axis of the weir on both edges of the river the profile up to a depth of 2m on the left side and up to 2.5m on the right side is characterized by the fractured basaltic rock.

Longitudinal Cross Section Of Head Work

6.7Stability analysis of weir

The Stabilizing force this represent the total weight of the weir calculated by considering a unit length of the weir by dividing the cross-section in to rectangles and triangles.

The structural design of the weir involves the stability analysis against failures that comes due to forces causing over turning, sliding and over stress. Especial, those acts on the weir body are: -                                   i) Weight of Material (weir body) Masonry

ii) Silt pressure

iii).Uplift pressure: is the silt that will accumulate u/s of the weir exerts a force in addition to external water pressure, Psilt.

iv).Water pressure for Dynamic and static case

For the analysis of stability the following parameters are considered:

Uplift pressure was considered for the weir section.

Downstream water pressure was considered.

Unit weight of masonry ɣm= 21KN/m3

Unit weight of concrete ɣc= 23KN/m3

Unit weight of water pressure, ɣw=10KN/m3

Unit weight of Silt pressure, ɣs=13.6 KN/m3

Where, pw= water pressure   Wm=weight of material (weir body)

Weir Stability  Analysis In Case Of D/S Water Height Is  Greater Than Weir Height

Weir Stability  Analysis In Case Of Dynamic

Weir Stability  Analysis In Case Of  Static

Weir Stability  Analysis In Case Of  Earthquick

6.8 Stability analysis of Wing wall Bod

In designing masonry retaining walls, the first step is to establish the pressure due to the back fill soil. Once the acting forces are known the stability analysis can be carried out in the same way as for the weir.

In our case the height of the u/s and d/s wing walls were found to be=

H1 = H+He+Fb

U/s wing wall height = weir height + water depth over crest + free board

= 1.4+2+0.5 = 3.9m

D/s wing wall height = H2 = d2+Fb

=conjugate d2+ free board

= 2.8m+0.5= 3.3m

The base width of the wall was chosen based on the general assumption of taking 50% to 70% of the wing wall height. By the same indication the max 70% of H was taken for the purpose of ensuring stability analysis. Hence the maximum bed width 2.6m and 2m was taken for the u/s and d/s wing walls respectively. The top width remains 50cm for both cases.

The soil active pressure pa representing triangular stress distribution at a depth (h) against the retaining wall exerts a force.

-Top width of the wall =0.5m

-Top level: for U/s = bed level +H1

=1378.361m+4.7m

= 1383.061m

D/s (at d2) = bed level +H2 (max)

=1378.361m +3.4m (max)

= 1381.761m

-Weight of masonry per m3, γm =21KN/ m3

-Coefficient of Active earth pressure, Ka, (take Ka=1/3)

-γs =18KN

Where Wm=weight of the wall material Ws= weight of the soil material

Ps=active earth pressure

NOTE: use weep holes for retaining walls pressure relief holes for downstream apron and expansion joint for long masonry work.

i) Stability analysis of upstream guide wall

W1=ɣm*A1*b

=21*1.9*0.5*1=19.95KN acting at (1/2*0.5)=0.25 m from the toe

W2=ɣm*A2*b

=21*(0.5*1.9*2.1)*1=41.895KN acting at (2/3*2.1+0.5)=1.9m from the toe

W3=ɣm*A3*b

=21*(2*2.6)*1=109.2KN acting at (1/2*2.6)=1.3m from the toe

Ws=ɣm*As*b

=18*(0.5*2.1*2.6)*1=49.14KN acting at (2/3*2.6+0.5)=2.23m from the toe

Ps1=1/2*ka* ɣs*hs12*b

=1/2*1/3*18*(2+1.9)2*1=45.17KNacting at (1/3*3.9)=1.3m from the toe

Ps2=1/2*ka* ɣs*hs22 *b

=1/2*1/3*18*22 =11.88KN acting at (1/3*2)=0.67m from the toe

| No | Name & Description | Magnitude of force (KN) |  | Arm length (m) | Moment about toe(KN-m) |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Vertical Horizontal |  |  | MR (+) | MO (-) |
| 1 | Self weight-W1 | 19.95 | - | 0.25 | 4.9875 |  |
| 2 | Self weight-W2 | 41.895 | - | 1.9 | 79.6 |  |
| 3 | Self weight-W3 | 109.2 | - | 1.3 | 141.96 |  |
| 4 | Soil weight-Ws | 49.14 | - | 2.23 | 109.58 |  |
| 5 | Soil pressure-Ps1 | - | -45.17 | 1.3 |  | -58.721 |
| 6 | Soil pressure-Ps2 | - | 11.88 | 0.67 | 7.9596 |  |

Summation of all Vertical forces, ------------∑Fv=220.185KN

Summation of all Horizontal forces, ---------∑FH=-33.23KN

Summation of all Resisting moment, --------∑MR=344.0871KN-m

Summation of all Overturning moment, ----∑MO=-58.721KN-m

Summation of all Moments, ------------------ ∑M=285.3661KN-m

Summary

Stability against over turning

∑MR/∑MO ≥1.5=344.0871/ 58.721=5.859>1.5--------ok

Stability against Sliding

∑FH/∑Fv <0.75, =33.23/220.185=0.15<0.75---------ok

Stability against over stress

Eccentricity: e = b/2-x, where X = centroidal distance (m)

B1= Bottom width (m)

X=∑M/∑Fv= 285.3661/220.185=1.29<2/3*B1=2/3*2.6=1.72---ok

B1/2=2.6/2=1.3

a=B1/6=2.6/6=0.43

e=│B1/2 -X│=│1.3-1.09│=0.21

Since e<a i.e 0.21<0.43-------------------------------ok

Stability against compression

ɣctoe=[{∑Fv/B1}/(1+6e/B1)]=[(220.185/2.6)/(1+6*0.21/2.6)]=37.47KN/m2.

Since the allowable compressive strength of masonry extends up to 1000KN/m2, and the calculated value is 37.47KN/m2there is no fear of compression.

ii).   Stability Analysis of Downstream guide wall

W1=ɣm*A1*b

=21*(1.3*0.5)*1=13.65KN acting at (1/2*0.5) = 0.25 m from the toe

W2=ɣm*A2*b

=21*(0.5*1.3*1.5)*1=20.475KN acting at (2/3*1.3+0.5)=1.36m from the toe

W3=ɣm*A3*b

=21*(2*2)*1= 84KN acting at (1/2*2) =1m from the toe

Ws=ɣm*As*b

=18*(0.5*1.5*1.3)*1=17.55KN acting at (2/3*2+0.5)=1.83m from the toe

Ps1=1/2*ka* ɣs* hs12*b

=1/2*1/3*18*(1.3+2)2*1=32.34KN acting at ((1/3(1.3+2)) =1.3m from the toe

Ps2=1/2*ka* ɣs*hs22 *b

=1/2*1/3*18*22 =11.88KN acting at (1/3*2) =0.67m from the toe

| No | Name &Description | Magnitude of force (KN) |  | Arm length(m) | Moment about toe(KN-m) |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Vertical Horizontal |  |  | MR(+) | MO(-) |
| 1 | Self weight-W1 | 13.65 | - | 0.25 | 3.4125 | - |
| 2 | Self weight-W2 | 20.475 | - | 1.366 | 27.9668 | - |
| 3 | Self weight-W3 | 84 | - | 1 | 84 | - |
| 4 | Soil weight-Ws | 17.55 | - | 1.83 | 32.1165 |  |
| 5 | Soil pressure-Ps1 | - | -32.34 | 1.3 |  | -42.042 |
| 6 | Soil pressure-Ps2 | - | +11.88 | 0.67 | 7.9596 | - |

Summation of all Vertical forces, ------------∑Fv=+135.675KN

Summation of all Horizontal forces, ---------∑FH=-20.46KN

Summation of all Resisting moment, --------∑MR=155.4549KN-m

Summation of all Overturning moment, ----∑MO=-42.042KN-m

Summation of all Moments, ------------------ ∑M=113.4129KN-m

Summary

Stability against over turning

∑MR/∑MO ≥1.5 =155.4549/42.042=3.69>1.5-------- ok

Stability against Sliding

∑FH/∑Fv <0.75, =20.46/135.675=0.15<0.75---------ok

Stability against over stress

Eccentricity: e = b/2-x, where X = centroidal distance (m)

B1= Bottom width (m)

a=emax

X=∑M/∑Fv=113.4129/135.675=0.835<2/3*B1=2/3*2=1.33---ok

B1/2=2/2=1

a=B1/6=2/6=0.33

e=│B1/2 -X│=│1-0.675│=0.325

Sincé  e<a i.e 0.325<0.33------------------------------ok

Stability against compression

ɣctoe=[{∑Fv/B1}/(1+6e/B1)]=[(135.675/2)/(1+6*0.325/2)]=34.348KN/m2.

Since the allowable compressive strength of masonry extends up to 1000KN/m2, and the calculated value is 34.348N/m2 there is no fear of compression

6.9 Design of Intake and Scouring Sluice

Assume pipe diameter equal to 20 cm and pipe length 8m from head work Topo.

Coefficient of roughness for pipe, n = 0.014

Intake capacity, q=34lit/sec =0.034 m3/sec

Length of concrete pipe, LP   =8m

In take loss, h= (v2/2g)*FT

Where: FT= (Entry loss +Friction loss + Exit loss) coefficients

0.2+FL+0.3

FL=fL/D and f=124.5n2/D1/3

Hence, using a full pipe of diameter 0.20m, the intake loss, h is calculated as:

f=124.5*0.0142/0.21/3 =0.042

FL=0.042*8/0.2=1.67

FT=0.2+1.68+0.3=2.08

But, Velocity of flow, V=q/A, where: A=ΠD2/4=3.14(0.72)/4=0.0314m2

V=0.034/0.0314=1.08m/s

Therefore, h= (V2/2g)* FT

h= (1.082/ (2*9.81))* 2.18=0.1296m

Check: using orifice formula

Q in take=C*A*(2gh) 1/2

Where: C, orifice coefficient=0.81 for circular intake

A & h are as described above

Qintake = C*A*(2gh) 1/2

Qintake =0.81*0.38*(2*9.81*0.1296) 1/2=0.41m3/sec > 0.034 m3/sec-----ok.

►River bed level=2269.087m

►Intake level at the intake gate=level of canal bed @ (0+00) +head losses

=2270m+ 0.1296

=2270.1296

►Top level of intake pipe=Intake level at the intake gate + Pipe diameter

=2270.1296+0.2m

=2270.3296

►Crest level=River bed level +Weir height

=2269.087+1.4=2270.487m

The intake is aligned 90-120o to weir axis. The intake level is fixed to 2270.487m

-0.15740=2270.1296,  Top level of intake pipe is 2270.3296 and intake flow is 34l/s. Opening diameter by general Orifice formula for free outflow (Cd=0.81) and above center of pipe to create flow will be concrete pipe. However, it is better to use larger diameter pipe so that farmers may store and use flows above lean flow in the future. Thus 40 cm concrete pipe was proposed. It is to be operated (opened and closed) by manual spindle gate.

Since the top level of the intake pipe (2270.3296m) is less than the crest level of the weir (2270.487m), the desired capacity of water can easily flow in to the intake pipe.

ii) Silt gate (Under Sluice gate)

The silt gate is located very close to(adjacent) the intake gate. It is 0.40m lower than the intake level i.e. 2269.7296m and 0.50m wide. Hence depth below crest level is 0.75740m; this gate is to be operated (lifted) manually(spindle gate) during the rainy season. The plate thickness shall be 5mm thick.

0.2

0.2

2270.1296

50 mm shaft or road

5mm plate

0.5

INTAKE GATE

SILT GATE

2270.487

2269.7

6.10. Design Of Canal And Related Structure

Irrigation system layout

Irrigation system layout for a Dildala Abbaa Oromo small scale irrigation project includes one diversion weir structure, crossing structures,division boxes, drop structures, lined canal, earthen canal, Chute structure, turnouts,flume, drainage culver &,Night storage,foot path and The main canal is contour canal throughout its length and it has a total length of 1900m, of this length the canal is rectangular earth and the is trapezoidal linend canal except at the recommended structure locations. Even though the command area of the project is large, due to the shortage of water the command area proposed for development is 56.5ha.

The water discharge measurement of the river in question is 34lt/sec but after night storage the discharge is 102lt/sec. The whole command area is after night storage. Hence the discharge is 102lt/sec and the water duty used 1.81Lt/sec/hs. Thus the command area to be developed is equal to discharge measured divided by the water duty i.e Command area =discharge/water duty=102lt/1.8lt/sec/ha=56.5ha.

6.11 Irrigation System Design

For small a Dildala Abbaa Oromo scale irrigation project, the type of Irrigation method proposed is furrow irrigation method mainly for the following reasons:-

The project is finally handed over to the farmers for managing it, in this case it will be easy for them to apply.

Since this method is gravitational in its nature there is no need of external power source to drive the system component, that makes the method economical to apply

It is also applicable and suitable for wide range of crops and soil type, etc.

Irrigation interval

Irrigation Interval for different proposed crops and identified soil units are computed and document under agronomy part of this document.. Averaging these values 6 days interval is taken

Unit Irrigation Flow

Unit Irrigation flow = 1.8l/s/ha * 6 days = 10.8l/s/ha

6.12 Irrigation Canal Design

A canal is said to be designed when the flow velocity in the canal is non silting and non-scouring, which depend up on its design bed slope and also it dimensions are sufficient enough to accommodate the design discharge. The capacity of the canals was fixed based on the demand to irrigate the available command area.The dimensions of the canal were worked and by applying the well known hydraulic formula called Manning’s formula.

Thus,  Q = 1/n*R2/3*S1/2*AWhere, Q = Design discharge (m3/sec)

n = Roughness coefficient

R = Hydraulic radius (m)

S = Design canal bed slope (m/m)

A = Cross sectional area of canal (m2)

V = Average flow of water (m/s)

| Hydraulic Parameters |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Given Data |  |  |  |  | Calculated Value |  |  |  |  |  |  |
| Description | Q | Roughness, n | Side slope, m | Slope, s | d | b | Area, A | Perimeter, P | Hydraulic Radius, R | Velocity, V | Discharge, Q | Free board, Frb |
|  | m3/s |  |  |  | m | m | m2 | m | m | m/s | m3/s | m |
| Lined | 0.048 | 0.014 |  | 0.001 | 0.30 | 0.35 | 0.105 | 0.950 | 0.111 | 0.520 | 0.055 | 0.15 |
| Earthen | 0.048 | 0.025 | 1 | 0.001 | 0.30 | 0.25 | 0.165 | 1.099 | 0.150 | 0.357 | 0.059 | 0.15 |

A. Main canal

i- Earth canal (Rectangular) for the first 0-200m.

Given data: -

Design Discharge, Q = 0.034m3/s

Roughness coefficient, n= 0.025

Canal design bed slope, S = 1:1000

Water depth in canal, d = 0.30m

- Design calculation

From Q = 1/n*R2/3*S1/2*A

A = b*d = 0.30*b= 0.30*0.25= 0.075

P = b+2d = 0.25+2*0.3=0.85

R = A/P = 0.088

⇛ 0.034 = 1/0.025* (0.3b/b+1.4) 2/3*1/10001/2*(0.3b)

⇛ 0.034*0.025/0.00051/2= (0.70b) 5/3/ (b+1.4) 2/3

By trial and error method the value of b is obtained as, b=0.25m

From this A = 0.70*0.25=0.175m2

P = b+2d = 0.25+2*0.30=0.85m

R = A/P = 0.075/0.85 =0.088

And V = 1/n R2/3S1/2

= 1/0.025*(0.088) 2/3*

V = 0.0079m/s

Free board, FB = 0.15  Therefore, hydraulic parameters for this canal are: -

Q = 0.0340m3/s         		A = 0.075m2

n = 0.025                        P = 0.85m

S = 1:1000		                   R = 0.088m

d = 0.30m                       V = 0.0079m/s

b = 0.25m                       F.B = 0.15m

0.3

0.1

5

0.3

0.3

CANAL SECTION

0.2

0

0.

2

0

0.25

0.30

ii)linend canal (Trapezoidal) for the canal length from 0+200m—1245

Given Data: -

Design Discharge, Q = 0.034m3/sec

Roughness coefficient, n = 0.014

Canal designed bed slope. S = 1:1000

Water depth in canal, d = 0.3m (Assumed

Side slope=1:1

Design calculation

From Q = 1/n*R2/3*S1/2*A

A = bd+md2 = b*0.3+1(0.3) 2 = 0.3*0.35+0.09=0.195m2

P = b+2d√(1+m2) = b+2*0.3*√ (1+12)= 0.35+0.424=0.774m

R = A/P= 0.25m

0.034= 1/0.014*[(0.7b+0.49)/(b+0.0.424)) 2/3* *(0.7b+0.49)

0.034*0.014/0.0011/2=(0.3b+0.09) 5/3/ (b+1.424) 2/3

By trial and error method the value of b is obtained as b = 0.35m

From this A = 0.3*0.35+1(0.3) 2 = 0.195m2

P = 0.35+2*0.3(1+1)=0.774m

R = 0.195/0.774 = 0.25m

And V = 1/0.025*(0.25) 2/3*

⇛ V = 0.39m/s

Therefore, hydraulic parameters for this canal are:-

Q = 0.0340m3/sec		A = 0.195m2

n = 0.014      P = 0.774m

S = 1:1000		R = 0.25m

b= 0.35m	V = 0.39m/s

d=0.3m	 F.B = 0.16m

Fb

d

b

iii)Earthen canal (Trapezoidal) for the canal length from 01+245-01+900

Given Data: -

Design Discharge, Q = 0.01m3/sec

Roughness coefficient, n = 0.015

Canal designed bed slope. S = 1:1000

Water depth in canal, d = 0.3m (Assumed)

Side slope=1:1

Design calculation

From Q = 1/n*R2/3*S1/2*A

A = bd+md2 =0.25*0.3+1*0.32=0.165m2

P = b+2d√(1+m2) = 0.25+2*0.3*√ (1+12)= 0.25+0.846=1.096m

R = A/P=0.165/1.096=0.15m

And V = 1/0.025*(0.15) 2/3*

⇛ V = 0.36m/s

Therefore, hydraulic parameters for this canal are:-

Q = 0.01m3/sec		A = 0.165m2

n = 0.05	P = 1.096m

S = 1:2000		 R = 0.15m

b= 0.25mF.B = 0.15m

V = 0.36m/s

d=0.3

B)Secondary canal

ii)linend canal (Trapezoidal) : SC start from division box of MC(1245m)  to Night storage(20m from MC)  then go up to right road (0+392m)

Given Data: -

Design Discharge, Q = 0.024m3/sec

oughness coefficient, n = 0.014

Canal designed bed slope. S = 1:1000

Water depth in canal, d = 0.3m (Assumed

Side slope=1:1

Design calculation

From Q = 1/n*R2/3*S1/2*A

A = bd+md2=0.35*0.3+1*0.9=0.195m2

P = b+2d√(1+m2) = 0.35+2*0.3*√ (1+12)= 1.2m

R = A/P=0.16m

V = 1/0.014*(0.16) 2/3*

V = 0.65m/s

Therefore, hydraulic parameters for this canal are:-

Q = 0.0240m3/sec		A = 0.195m2

n = 0.014	           P = 1.2m

S = 1:1000		 R = 0.16m

b= 0.35m	V = 0.65m/s	     d=0.3m	         F.B = 0.1

ii)Earthen canal (Trapezoidal) for the canal length from 392-862

Given Data: -

Design Discharge, Q = 0.024m3/sec

Roughness coefficient, n = 0.014

Canal designed bed slope. S = 1:1000

Water depth in canal, d = 0.3m (Assumed)

Side slope=1:1

Design calculation

From Q = 1/n*R2/3*S1/2*A

A = bd+md2 =0.25*0.3+1*0.32=0.165m2

P = b+2d√(1+m2) = 0.25+2*0.3*√ (1+12)= 0.25+0.846=1.096m

R = A/P=0.165/1.096=0.15m

And V = 1/0.025*(0.15) 2/3*

⇛ V = 0.36m/s

Therefore, hydraulic parameters for this canal are:-

Q = 0.01m3/sec		              A = 0.165m2

n = 0.05	                         P = 1.096m

S = 1:2000                    		 R = 0.15m

b= 0.25m                          F.B = 0.15m

V = 0.36m/s

d=0.3m

d) Field Canals/feeders/

Field canals are schemed to off take water from main & secondary canals to the field. To divert and regulate the field canals water requirement, turnout structures with gate are provided at the point of diversion on main & secondary canals.

The amount of water supplied to the field through turnouts is very small. Therefore; it is believed that the farmer’s furrows like field canal without any slope-adjusting structures are sufficient for this purpose.

6.13Turnout Structure

Turnouts are off takes that take fixed amount of water to the field ditches. The turnouts are

Provided on the main, secondary The irrigation duty of the project is 1.81lit/s/ha.

The average irrigation interval for the whole crops is 6 days. Therefore, the average discharge for each turnout is calculated as:

Q = duty * area * Irrigation interval and this amount of water should be applied for 8hrs.

Turnouts are designed by the general orifice formula,

Q = CA-------------------i

Where:      A=ΠD2/4---------------ii

h=Y-D/2----------------iii

Substituting equation ii & iii in i results in

[4Q/ (IIC)] 2 /2g=D4{Y-D/2}

Therefore the exact pipe diameter is obtained from the above equation by trial and error.

Where Q = discharge for each turnout (m3/s)

C = discharge coefficient (0.61)

A = Area of pipe (m2)

h = Driving head (m)

Y=Water depth in canal

On Dildila Aba Oromo  canal profiles, there are15 turn outs.9 - turn outs on MC $  6-   turn outs on SC.

6.14 Design of RCC Flume

Flumes are structures required to pass irrigation water across natural depressions the main river courses. There is one flume is  located along the secondary canal at 544m -555m to cross the main river. This flumeis designed as fixed flumes, which are made up ofreinforced concrete materials.

The total length of the flume was fixed to 11.5m.

Flume Design

Hydraulic design of flume

Characteristics of mother canal

a)  -Station

- Bed width of the canal = 0.25

- Water depth in the canal = 0.34

- Side slope =0(lined)

- Velocity of u/s canal = 0.45m/sec

- Canal discharge = 0.024 m3/sec

b) Flume section (L = 11.5m)

Assume b = 2d, v = 1m/sec, n = 0.015

A = Q/V = 0.024/1 = 0.024m2

A = bd = 2d2 = 0.024m2d= = d = 0.11m  USE  d2=0.2 & b = 0.4m

Check froud number (Fr should not be > 0.7)

Fr = V/√gd = 1/√9.81x0.2 = 0.509< 0.7 → Ok

Fr = < 0.7 proves stable sub critical flow.

P = b+2d = 0.4+2x0.2 = 0.8m

R = A/P = 0.292/1.52 = 0.19m

Friction head loss, hf = Lx (n*V/R 2/3)2

= 11.5x (0.015x1/0.032/3)2 = 0.28m~0.02

c) Water surface elevation at u/s and d/s of the flume.

i) ∆h1 = 1.5(v22-v12)/2g = 1.5(12-0.452)/2x9.81 = 0.061

ii) ∆h 2 = 0.3(v22-v12)/2g = 0.3(12-0.452)/2x9.81 = 0.012

d) Elevation

Invert elevation at u/s canal (flume inlet) = 2247.805m

Water surface elevation at u/s canal +depth1(0.34m)= 2248.145m

Water surface level at the flume inlet

= 2248.145m - ∆h1 = 1373.556m – 0.061 = 2248.084m

Invert elevation at the flume inlets

2248.084m –d2 = 2248.084m -0.2 = 2247.884m

Invert elevation at the flume outlet

= 2247.884m  – hf = 2247.884m -0.02 = 2247.864m

Water surface elevation at the flume outlet

= 2247.864m + d2 = 2247.864m +0.2 = 2248.064m

Water surface elevation at d/s canal

= 2248.064m +∆h2 = 2248.064m +0.012 = 2248.076m

Invert elevation at d/s canal

= 2248.076m – d1 = 2248.076m -0.34 = 2247.736m

Where d1 = 0.6m for lined canal

Structural design of flume

Station 544m -555m

Flume meant, in this project, a structure used to pass the canal over drainage by supporting with columns without reducing its dimensions.

The structural design of flume that spanned over columns is as shown below.

Considerations

Uniformly distributed load

Simply supported flume

The beam was designed for concrete

The beam was designed for a balanced section.

Specific weight of concrete = 23KN/m3

Specific weight of water = 10KN/m3

Calculations

60

15

35

45.5

25

45.5

15

5

76

15

5

1) Dead load = (0.15x0.6x2+0.15x1.16+0.25x0.35) x23 = 10.15KN/m

2) Life load = (0.38x0.76)x10KN/m3 = 2.58KN/m

W = 10.15 + 2.58 = 12.73KN/m

If the factor of safety is assumed to be 1.3,

WT = 12.73x1.3 = 16.549KN/m

W=16.549KN/m

A

B

C

D

E

6m

6m

6m

6m

74.47KN/m

63.83KNm

42.55KNm

63.83KNm

Load Diagram

B.M Diagram

38.83KN

54.93KN

46.89KN

62.99KN

-62.99KN

-46.89KN

-54.93KN

-38.83KN

S.F Diagram

To determine the bending moments, support forces and support moments, chaperon’s theorem of three moments is used. i.e.

MAℓ1 + 2MB (ℓ1+ℓ2) + Mcℓ2 = - (6a1X1/ℓ1 + 6a2X2/ℓ2)

ℓ6

ℓ5

ℓ1

ℓ4

ℓ3

ℓ2

3) Mid Span bending moments

= Wℓ2/8= 16.549x62/8	= 74.47KNm/m

For span ABC,

a1x 1 = (2/3 * 74.47*6)*6/2 = 893.64- remains the same for all spans since the spans are equal.

Then the term  (6a1x1/ℓ1 +6a2x2/ℓ2 = -(6*893.64/6+6*893.64/6)

= -1787.28

MAℓ1 + 2MB (ℓ1+ℓ2) + Mcℓ2 = -1787.28

0

6MA+ 2MB (6+6) + 6Mc = -1787.28

24MB + 6Mc = -1787.28--------------------------------------------1

For span BCD,

MBℓ2 + 2MC (ℓ2+ℓ3) + MDℓ3 = -1787.28

6MB+ 2MC (6+6) + 6MD= -1787.28

0

6MB + 24Mc +6MD = -1787.28 -------------------------------------2

For span CDE,

MCℓ3 + 2MD (ℓ3+ℓ4) + MEℓ4 = -1787.28

6MC+ 2MD (6+6) + 0ME= -1787.28

6MC + 24MD = -1787.28-----------------------------------------3

From equation 1, MB = (-1787.28-6MC)/24

Putting the value of MB in equation 2, and solving,

22.5MC + 6MD = -1363.14

90MC + 24MD = -5361.84--------------------------------④

Solving 3 and 4 simultaneously,

90MC + 24MD = -5452.56

6MC + 24MD = -1787.28

MA = 0KNm/m

MC = -42.55KNm/m

MB = -63.83KNm/m

MD = - 63.83 KNm/m

ME= 0KNm/m

3) Support forces

Moment at B (point A is our reference point)

∑MB = -63.83

⇒ 6RA- WT*x*x/2= - 63.83 (x = L = 6m)

⇒ 6RA- 16.549x6x6/2 = -63.83

⇒ RA = 38.83KN = RE

⇒ RA = RE= 39.08KN

Moment at C:

∑MC = -42.55

⇒ 12RA+6RB – WT *X*(6+X/2) – WT*X*X/2 = -42.55(x = L = 6m)

⇒ 12*39.09+6RB – 16.549*6*9 – 16.549*6*3 = -42.55

⇒ RB = 117.08KN = RD

⇒ RB= RD= 113.336KN

Moment at D:

∑MD = -63.83

⇒ 18RA+12RB+6RC-WT*X*(12+X/2) – WT*X*(6+X/2)-WT*X*X/2 = -63.83

⇒ 18x39.08+12x113.336+6RC-16.549x6x15-16.549x6x9-16.549x6x3= -63.83

⇒ RC = 92.27KNA

Maximum +ve bending moments: Span AB: X= 2.3534m ⇒B.M = W.X.X/2 = 12.73x2.35342/2 = 35.25KNm

4) Shear force

Wherever the shear force that tends the structure to shear off is zero, the bending moment is maximum.

Span AB: RA x.

S.F = RA – WT*X = 0

⇒ 39.08-16.549x = 0

⇒ X = 38.83/16.549 = 2.36m~2.4m

Now using the BM and S-F diagram, the maximum positive BM between each span is calculated and the largest value is taken.

Span AB:

B.M = WT.X.X/2-Mid span       where:  x=RA/WT

+B. Mmax = [(39.08/16.549)2 x16.549]/2-MA

+B. Mmax = [(39.08/16.549)2 x16.549]/2-0 = 46.14KNm/m

+B. Mmax = [(2.36)2 x16.549]/2-0 = 46.08KNm/m

Span BC:

RARB

X

6

Span BC:

S.F = RA +RB– WT*(x+6) = 0

⇒ 39.08+113.336-16.549*(x+6) = 0

⇒x = 53.122/16.549 = 3.21m

+ B.Mmax = [(53.122 /16.549)2x16.549]/2-MB

+ B.Mmax = [(53.122 /16.549)2x16.549]/2 -63.83 = 21.4 KNm/m

Span CD:

S.F = RA +RB+RC– WT*(x+12) = 0

⇒ 39.08+113.336+92.27-16.549*(x+12) = 0

⇒x = 46.098/16.549 = 2.785m

+ B.Mmax = [(46.098 /16.549)2x16.549]/2-MC

+ B.Mmax = [(53.122 /16.549)2x16.549]/2-42.55 = 42.71 KNm/m

Span DE:

S.F = RA +RB+RC+RD– WT*(x+18) = 0

⇒ 39.08+113.336+92.27+113.336-16.549*(x+18) = 0

⇒ x = 60.14/16.549 = 3.63m

+ B.Mmax = [(60.14/16.549)2x16.549]/2-MD

+ B.Mmax = [(60.14 /16.549)2x16.549]/2-63.83 = 45.45 KNm/m

⇒+B. Mmax = 45.45KNm/m

Now using the B-M and S-F diagrams, the maximum positive B-M between each span is calculated and the largest value is taken.

Therefore, from the S-F and B-M diagram,

- Maximum shear force (SF) = 60.14KN

- Maximum bending moment (positive) = 45.45KNm

- Maximum bending moment (negative) = 63.83KNm

5) Reinforcement bar design:

Assume the flume as a T-beam of 150mm flange thickness, 900mm flange width and 250x350mm web width and depth respectively.

900

150

350

250

D

Assuming one layer of ∅14 longitudinal bars and ɸ6 stirrup and 30mm cover,

d = 500 - ɸ/2 - ɸ(stirrup) – cover

=    500 – 14/2 – 6-30

= 457mm

Trial area of steel:

Taking fs = 140N/mm2, Modular ratio (n) = 8 and fc = 6,

As = +M/[fs(d-t/2)] = 45.45*106/ [140(457-150/2)]

= +M/ [fs (d-n/3)] = 849.85mm2

Where n = 0.39d.

ℓ = As/bd = 849.85/ (900*457) = 0.00206 = 2.06x10-3

ℓn = 2.06*10-3*8 = 1.648*10-2

t/d = 150/457 = 0.3282

k = [ℓn+0.5(t/d) 2]/ [ℓn+ t/d]

= [1.648x10-2 + 0.5(0.3282)2] / [1.648x10-2 + 0.3282]

= 0.203

kd = 0.203*457 = 93.168mm < 150mm. Therefore the structure should be designed as rectangular beam. i.e rectangular shape flume.

Check whether the concrete resists the above maximum positive bending moment,

Kb = n/(n+r), where r= fs/fc= 140/6

= 8/ (8+140/6)

= 0.2553

jb = 1-kb/3 = 1-0.2553/3 = 0.9149

M = fc/2 [kbd2 jb] → b = 250mm & not 900mm

= 6/2 * 0.2553*250*4572 *0.9149

= 36.59KNm< 45.45KNm

Since M<+B-Mmax, double reinforcement of the beam required.

For the tension side (Maximum Positive bending moment)

Total area, As = +B.M/ (fs x d x jb)-for rectangular beam

= +M/fs (d-n/3)

= 45.45*106/ (140*457*0.9149)

= 776.45mm2

Single area = πr2 = πD2/4 = πx142/4 = 153.938mm2

No of bars = 904.92/153.938 = 5.8=6

Spacing = 250/6= 41.66mm

∴ provide 6 No, ∅14 @ 4.166cm c/c at the bottom.

For compression side (Maximum Negative bending moment)

As = -B-Mmax / fs * d * ja- (for rectangular beam)

=63.83* 106/ 140 *457* 0.9149 = 1090.453mm2

No of bars = 1090.453/153.938 = 7.08=7

Spacing = 250/7 = 35.7mm=4cm

Therefore, use 7Nos, ∅14 @35.7mm c/c at the top.

For construction convenience,

Use 7No, ∅14 @ 4cm c/c at the bottom

and use 7No, ∅14 @ 4cm c/c at the top.

and ∅6 @ 20cm c/c transverse reinforcement.

6) Slab and wall design of the flume

Spacing of the main bars in slabs should not be greater than 3xthe effective depth of the slab or 450mm or it should not be greater than> 2xthe thickness of the slab.

Spacing of the distribution reinforcement in slabs should not be greater than 5x the effective depth of the slab or 450mm. The slabs and walls for the above flume were designed as shown below.

i) For the slab, use 6 Nos, ∅12@10cm c/c for the main bars.

Use the transverse reinforcement bar of ∅6 at 20cm c/c

ii) For the walls, use 2 Nos, ∅12@ 36 cm c/c main bars and transverse bar of ∅6 at 20 cm c/c

7) Design of column of the flume

The maximum height of columns of flume is 4.0m. Assuming the depth or thickness of the column to be 0.20m,

Le = 0.65L for tied column

Where Le = effective height

L = height of the column

Le = 0.65 x 4 = 2.60m

g = Led/d, where g = slenderness factor

Led = effective height about depth plane

d = depth of the section.

g = 2.60/0.20 = 13.0

K/g = 3.45 for rectangular section where K = slenderness ratio

K = 3.45*g = 3.45x13 = 44.85

∴ Since K = 44.85 < 45, the column is treated as short column

WSD of short column

The allowable load carrying capacity of a tied column is given by

Pa = Ac σcc + Asc σsc

Where Pa = allowable load

Ac = Area of column section

σcc = Compressive stress in concrete

Asc = Areas of steel in compression

σsc = Compressive stress in steel

Taking   σcc = 6MPa, ∅ = 14, σsc = 130MPa, and the cross sectional area to be 0.2x0.4,

Ac = 0.2 x0.4 = 0.08m2

Asc = 4* π (0.014)2/4 = 6.16x10-4m2

Pa = 0.08 * 6 * 106 + 6.16* 10-4* 130*106

= 560KN

External loads (Px)

Weight of column = 0.2 * 0.4 *4 *24 = 7.68KN

Weight of beam & canal = 16.549KN/m*6 = 99.294KN

Weight total = 7.68 + 99.294 = 106.974KN If WT is multiplied by F.S. of 1.3

Total weight = 106.974*1.3 = 139.0662KN

Since PX = 139.0662KN < Pa = 560KN, the cross sectional area (0.2x0.4) and 4 No s, ∅14

will be sufficient to carry all the loads:

And also provide ∅6@ 20cm c/c ties.

8) Design of foundation/footing

The soil type of the above flume on which the foundation rests is clay soil. Taking the safe bearing capacity for moist clay and sand clay mixture to be 150KN/m2,

Let the dimension of the footing be LxWxD= 1m*0.9m*1m = 0.9m3

Total load on the soil

Load due to foundation = (1 * 0.9 *1) * 24 = 21.60KN

PX = 141.258KN

Total load = 139.0662 + 21.60 = 160.6662KN

Safe area of footing = P/Pa = 160.6662/150 = 1.071m2

Area provided = L * W = 1*1.1 = 1.1m2→safe

Pressure exerted on the soil:

Pa = P/ (B*L) = 160.6662/1.1 = 146.06KN/m2

Since 146.06< 150, the soil is safe from settling.

9) Design for bending

Cantilever span of the footing for max B. M is:

Lm = B/2 – t/2 where t = thickness of the wall = 0.2m assumed.

= 1.1/2 – 0.2/2 = 0.45m

The load intensity on the footing is q = P/A

q = 160.6662/1.1 = 146.06KN/m2

Maximum bending moment on footing slab:

B. M = q*Lm*Lm/2 = 146.06*0.452/2 = 14788.575Nm/m

In this flume deign,

σc= 6MPa  Kb = 0.2553, jb = 0.9149,

fs = 140N/mm2

The value of d is therefore, d2 ≥ M/kb σc = 14788.575/0.2553*0.9*6*106

Or d ≥0.1m ≥100mm

∴ As = M/ (fs *jb * d)

=14788.575Nm/(140N/mm2 * 0.9149 * 0.1m)

= 1154.58mm2

Single area of ∅12 = π *122/4 =113.04mm2

No s = 1154.58mm2/113.04 = 10 No s

Spacing = (113.04/ 1154.58) * 0.9 = 8.8cm

∴ Provide 11No s, ∅12@8.7 cm c/c main bars and 7 No s ∅6@15cm c/c transverse bars.

LS = Lm-d = 0.45-0.1 = 0.35

Shear force, V = q *Ls = 146.06*0.35= 51121N/m

Nominal shear stress = V/bd = 51121/ (0.9*0.1)

= 0.57Mpa< 0.3% Ⴀ for steel (0.24)∴Modify d ≥ V/b x Ⴀ) = 51121/ (0.9*0.24)

d ≥ 0.23m

⇒ Use = 0.3m

∴ Overall depth, D = d+ cover + 1/2ɸ of bar

= 0.30+0.094+0.06 = 45cm

∴ Dimension of bending for footing = (1 x 0.9 x 0.45)

6.15.Design of pond storage

- 36.2 l/s flow is to be stored for 14hr and thus volume of storage is 1824m3 - For 2.00m useful storage in 1v: 2H inner slope pond, 0.50m dead storage and 0.75m free board, 50m at bottom and 60m at top square pond with live storage of 2m is enough to store 2700m3 - Top Bank width 3m, inner slope 1V: 2H and outer slope 1v: 1.5H was adopted and analyzed for stability under homogenous clay fil

This pond structure is located on the secondary canal (SC) on the left side of Dildila Aba Oromoat station 0+50(SC-2). The maximum net command area after this pond is 20ha. Taking the duty of the command area to be 1.81 l/sec/ha, the maximum discharge of the area= 1.81x20= 36.2lit/sec. For the design of the canals and its structures after the pond, 36.2lit/sec was taken.

A)  Assuming 8hrs irrigation time, the maximum volume of water required is:

V = 36.2l/s x 8x60x60 sec

= 1042560litres.

= 10425.6m3

Taking evaporation and percolation loss to be 10%, the total volume will be

VT = 10425.6x10%+10425.6

= 11468.16m3

Therefore the pond should be designed to store this much volume of water.

B)  Storage time

The total discharge entering the pond is 24lit/s.

24lit/sec*t = 1042560lit.

t = 1042560/24=43440sec = 25,278.9sec /3600=   12.06hrs take 12hrs.

C)  Dead storage

The dead storage capacity for this pond was considered to be 10% of the total storage volume.

Dead storage = 10% x 11468.16m3= 1146.816m3

Vtotal = 11468.16+1146.816m3= 12614.976m3

Assuming the dead storage area to be 40x50m2

60

50

6050   dead storage

½(60+50) x Hx60= 12614.976m3

H = 3.8m

Providing a Freeboard and dead storage height of 0.04m, then the height of the pond will be

H = 3.8m+0.04m=3.84m

That means, the bed level of the storage should be 3.84m below the inlet bed level of canal.

⇒Bed level = 2252.981m – 3.84m = 2249.141m

D) Storage intake design

In order to fix the intake level of the storage, the dead storage height (h) should be known.

60

5m 	5m

L=60m

x			H=  3.84m

θhθ

50m

tan θ = 5/3.84 = 1.300

tan θ = x/h ⇒ x = h*tan θ

Volume of the dead storage is 1146.816m3

∴1146.816m3= ½ (50+50+2x) * h *60

= (100+2x) x 30h

= (100+2h tan θ) x 30h   where  tan θ  = 1.300

1146.816    =   (100+2h*1.3)*30h

=    3000h+78h2

78h2+3000h – 1146.816 = 0

x=-b±b2-4ac2a

h = 0.386m

If the intake level is fixed at h = 0.386m, the total volume will be:

x = 0.386 tan θ = 0.386*1.3 =0.502m

2x = 1.004

50+1.004= 51.004m

∴V = ½ ((51.004+60) x (3.84-0.386) x 60)

V = 55.48*2.26*60

= 11502.2m3 >10425.6m3→ Ok

At h = 2.26m

Q = CA*    (orifice formula)

0.233m3 = 0.61 * Π*d2/4*

0.233 = 0.61*3.14*d2/4*6.66

0.932 =12.75*d2

d = 0.27m

But the above pipe size is not found on the market. Instead, if ф = 30cm is used, the total discharge will be:

Q = CA =C Π r2= 0.61* Π *0.152 *

=0.61*3.14*0.0225*6.66

= 286 lit/s > 36 lit/s→ Ok

Therefore, intake bed level of pond

= Pond level (inlet bed level of canal) – (2.26 + ф/2)

= 2252.981m – (2.26+ 0.15)

= 2250.571m

CBL at pipe out let=2248.243m.

E) Fence around the pond

Fence should be constructed around the pond for the safety of animals and human being from falling in to the pond. It was designed to be 2m away from the edge of the pond. The fence is simply wooden pole with lean concrete foundation. The maximum height of the pole is 2m. There is a thorny wire rounding the fence having 25cm c/c spacing.

F) An over flow spill way

An overflow spill way was also provided just at the net full supply level of the pond. It was constructed from masonry and it is rectangular in shape and the maximum length of the spill way is about 10m laid horizontally.

6.16 Chute structure

This structure is used to convey water to a lower elevation. Because of the high velocity in the chute the entire structure should be lined and should be furnished with a transition at the inlet and with stilling basin & transition at the outlet. There is one chute designed for the main canal (MC-1000-1) & one chute for secondary canal (SC2-12).Chute structure is provided where the slope of existing land is steeper than the canal that is going to be constructed and too long to provide other slope braking structure like drop.

Toward its beginning, the secondary canal run through profiles of steeply slopes. Therefore chute structure is provided to this canal at this canal at this steeply slope profiles. The lengths and chainages of

the chute structures are described by the following table:

The structural dimension of chute structtures

| no | canal | L | U/SCBL | ELSTLB | D/SCBL | Bs | Ls | b | B | dw | Fb |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | MC | 50 | 2265.05 | 2261.28 | 2261.38 | 0.55 | 2.3 | 0.25 | 0.34 | 0.3 | 0.15 |

Chute structure on mc(chute-1)

1. Critical flow Hydraulics

- Discharge, Q = 0.034m3/s

- Flow depth, d1 = 0.7m

- Flow velocity, Vo = 0.39m/s

- EL.A = 2265.05m(canal bed) chute inlet

- EL.B = 2261.38m(chute outlet canal bed) 2261.38  -0.7

- Length, L = 49m

d3 = 0.7m

V3 = 0.39m/s

Width of notch; bc = 0.734Q/d03/2 = 0.734*(0.034)/(0.7)3/2 = 0.042m

Unit Discharge, qc = Q/bc = 0.034/0.042= 0.809m3/s/m

Critical depth, dc = (qc2/g)1/3 = [(0.809)2/9.81)1/3 = 0.41m

Critical velocity, Vc = qc/dc = 0.809/0.41= 1.97m/s

Velocity head, hvc = Vc2/2g = (1.97)2/2*9.81 = 0.198m

Water area, Ac = bc*dc = 0. 042*0.41 = 0.0176m2

Wetted perimeter, Pc =bc+2dc=0.042+2*0.41= 0.862m

Hydraulic radius, RC = Ac/Pc = 0.0176/0.862 = 0. 02m

Water surface slope, Ic = [nVc/Rc2/3]2 = [0.015*1.98/(0.02)2/3]2

Ic = 0.1667~0.0056 use it

2. Energy calculation:

At section C

- EL.A = 2265.05m (canal bed) chute inlet

- EL.B = 2261.38m (chute outlet canal bed)

dc =0.42m

hvc =0.198m

Z = ( EL.A-EL.B) +0.1 (assumed)

= 2265.05m -2261.38m = 3.77m

EC = dc+hvc+Z = 0.42+0.198+3.77

⇛ EC = 4.388

Energy at section 1

EC =E1

Calaculation values

d1=0.4

bc=b1=0.042

A= d1 * b1 =0.4*0.042=0.0168

V1=Q/A1 =0.034/0.0168=2m/s

hv1=v12/(2g)=0.2

P1=b1+2d1=0.842

R1=A1/P1=0.0199m

I1=(nv1/R12/3)2=0.17

Im=(Ic+I1)/2=(0.0056+0.12)/2=0.0878

hfl=Im*L=4.3022

E1=d1+hv1+hfl=4.9~4.377

4. Conjugate depth after jump

- Froude number, Fr = V1/= 2/ = 1

- Conjugate depth, d2 = d1/2 [-1] = 0.4/2[]

d2 = 0.56m

5. Stilling Basin

- Length, L = 4*d2 = 4*0.56 = 2.3m

- Width, B = 18.46/ (Q+9.91) = 18.46/ (0.034+9.91) = 0.34m

- Bottom elevation (EL.C)

V2 = qc/d2 = 0.809/0.56 = 1.44m/s

hv2 = V22/2g = (1.44)2/ (2*9.81) = 0.11m

E2 = d2+hv2 = 0.56+0.11 = 0.67m use E2=0.8

E3 = d3+hv3 = 0.7+ (0.39)2/ (2*9.81) = 0.707m

Leap height, a = E2-E3 = 0.8-0.707=0.09 use= 0.1m

E1.C = El.B -a = 2261.38-0.1m = 2261.28

6.17.Drop strcture

This is designed to dissipate excess water energy as a result of steep gradient along the main canal alignment and to keep the longitudinal slope of the canal with in acceptable limit so as to avoid non- permissible rapid flow in the canals. There are five (5) vertical drops(USBRType) two drops on MC & three vertical drops on S.C designed for the system as shown in the Drawing.

Discharge Q, upstream water depth h1, upstream water velocity V1, downstream water depth h2, downstream water velocity V2 and drop ht Z

rop width = bc=0.734 Q/(h1)1.5

Unit discharge=q=Q/bc

Critical depth = hc = (q2/g)1/3 Stilling basin:

Basin width, B=18.46(Q)½/(Q+9.91)

Basin length = L=[2.5+1.1(hc/z)+(hc/z)3](z*hc)0.5

Lip height a =hc/2

Protection work at upstream and downstream= 1.2+1.5Q1/

These are provided to make the slope of the old main canal compatible with our design slope.

· On main canal- 2,there is two drop of 1.50m height.

· On secondary canal –3, there are three drops of 1m height. All of the

drops are vertical drops (USBR type).

Table 7.3.2 Drop Type, location & its elevation

| No | Types of canal | Station | Canal bed Elevation |  | Drop height(H) (m) | Drop width | Bw | BL | Lipheight | U/s &D/s transition | Remark |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Up stream | Down stream |  |  |  |  |  |  |  |
| 1 | MC | 900-950 | 2268.08 | 2266.59 | 1.5 | 0.4 | 0.6 | 0.85 | 0.1 | 1.55 | vertical drop |
| 2 | Mc | 950-1000 | 2266.59 | 2265.05 | 1.5 |  |  |  |  |  | vertical drop |
| 3 | SC | 200-250 | 2251.747 | 2250.7984 | 1 |  |  |  |  |  | vertical drop |
| 4 | SC | 350-392 | 2250.377 | 2249.13 | 1 |  |  |  |  |  | vertical drop |
| 5 | SC | 493-544 | 2249.239 | 2247.805 | 1 |  |  |  |  |  | vertical drop |

i) Design of vertical drops (H=1.5m) on main canal -2

Critical flow hydraulics

a) Design discharge, Q= 0.034m3/sec

b) Height of drop, H= 1.5m

c) Width of drop, bc = 0.734Q/ (d1)3/2 = 0.734(0.034)/ (0.7)3/2 = 0.043m where d1=0.07m

d) Unit discharge, q = Q/bc =0.034/0.043= 0.0.798m3/sec/m

e) Critical depth, dc = [q2/g] 1/3 = (0.7982/9.81)1/3= 0.4m

2. Stilling basin

a) Lip height, a= dc/2=0.4/2= 0.2m

b) Length, L = [2.5+ (1.1 dc/H) +0.7 (dc/H) 3] √(Hdc) = 2.81m

c) Width, B= [(18.46√Q)/ (Q+9.91)]= 0.34m

Design of vertical drops (H=1.5)

Critical flow hydraulics

a) Design discharge, Q= 0.034m3/sec

b) Height of drop, H= 1m

c) Width of drop, bc = 0.734Q/ (d1)3/2 = 0.734(0.034)/ (0.34)3/2 = 0.13m where d1=0.34m

d) Unit discharge, q = Q/bc =0.034/0.13= 0.27m3/sec/m

e) Critical depth, dc = [q2/g] 1/3 = (0.272/9.81)1/3= 0.195m

2. Stilling basin

a) Lip height, a= dc/2=0.195/2= 0.098m

b) Length, L = [2.5+ (1.1 dc/H) +0.7 (dc/H) 3] √ (Hdc) = 2.72m

c) Width, B= [(18.46√Q)/ (Q+9.91)]= 0.34m

6.18 Division Box

Division boxes are provided in order to distribute water from a canal to different branching canals.

These are structures used to distribute the required canal discharge from the main, secondary or tertiary canals that is to be conveyed to the required point.

Division boxes are proportional flow dividing structures in the command area and gate operated as the farmers practically use rotational irrigation.

Division box are provided on main canal and Branch canals at the station on which the irrigation water

divided to Branch canal and secondary canals, so that the design of such structures will be made based on

the required irrigation water to the area which covered by such canals

There is one division boxes at MC (1) to devide the main canal to the night storage & to the secondary canal and both are operated by gate.

Sample Design of DB

Station: MC1(DB1) to MC2(DB2) to SC ( to night storage) or(DB3)

Using broad crested weir formula, Q= CLh3/2

Where Q= discharge through rectangular opening, 0.034m3/s

c = coefficient of discharge, C= 1.7

L = effective length of crest form, in m

h = flow depth over the weir, m

Assuming equal c &sill height, S for two or three divided canals, the proportion is: Q1/Q2 = Q2/Q3= L1/L2=L2/L3

Where Q1= flow in the first canal(MC1)

Q2= flow in the 2nd canal(MC2)and

Q3= flow in the 3rd canal(SC)

L1= crest length of opening a cross the first canal(MC1)

L2= crest length of opening a cross the 2nd canal(MC2)

L3= crest length of opening a cross the 3rd canal(SC)

Q1 = CL1*(h)3/2 & L1= Q1/ch3/2, L2= L1xQ2/Q1,  L3= (Q3/Q2)xL2

The total depth of the DB is the total depth of the incoming canal plus some free board, DDB= d+ fb, DDB-2= 0.30+0.16=0.460m

The width of the DB is B= b+2xmxD

Where b= bed width of the incoming canal, b= 0.20m

m= side slope of the incoming canal, m= 1.5m

D= total depth of the incoming canal, D= 0.46(50)m

Therefore, B= b+2xmx0.3 = 1.10m

QMC= 0.034m3/s, QSC= 0.024m3/s  andQmc2=0.01m3/s

L1 = QSC/Ch3/2 = 0.2m, h is 0.15m, L1 is an opening to mc and sc

L2 = L1xQmc/QSC= 0.28m L2 is an opening to SC1

L3=L2×QMC2/QSC=0.12m L3 is an opening to MC2

Depth, D= 0.3+0.16= 0.460m

Bottom width, B= 2x1.5x0.3+0.20 = 1.1m, use B=1.1

6.19 Drainage pipe Culvert

Drainage Culverts are provided in this project to pass safely floods and stream flows from the upstream under irrigation canal to the downstream for which the bed level of the canal is above the stream or gorge bed level. Those, the size of the pipes are fixed considering the size of the gorge in addition to collected flood from these catchments.

Sample Design (Culvert no 1, station MC-200m)

Given data: -

Catchment’s area, Ac= 0.3km2

Specific Discharge, q = 0.5m3/sec/km2

Vo = 1.0m/sec (Assumed)

Design calculation

Therefore,

Qd, Design Discharge, = Ac*q = 0.3*0.5 = 0.15m3/sec.

Ao, Estimated pipe area from Qd , Ao= Q/Vo = 0.15/1= 0.15m2

D, Pipe diameter D= √ (1.485*Ao) =√ (1.485*0.15)= 0.472m------take, D=0.5m

A, Calculated wetted area A = πD2/4 = π*(0.5) 2/4 =0.168m2

P,perimeter , P = πD =π*0.5=1.57m

R, Hyd. radius  R = A/P =D2/4πD =D/4 =0.5/4=0.125m

V,Calculated velocity V = Q/A = 0.15/0.168=0.89m/s

I,Slope  I= [nv/R2/3]2 = [0.014*0.89/(0.1252/3)]2 = 0.002

ΔH,Head difference = I*L = 0.002*3=0.006m

d,Water depth  d = 0.80D = 0.80*0.5 = 0.4m

Table7.1. Stations of pipe culverts and its hydraulic parameters

| At necessary station | Catchment’s area, Ac | q Specific discharge | Assumed Velocity, Vo | Designed discharge, Qd= Ac*q | Ao= Q/Vo | Diameter of pipe, D=√ (1.485*Ao) | Wetted area, A= πD2/4 | Hyd. radius, R= D/4 | Velocity, V= Q/A | Slope, I=[nv/R2/3]2 | Head difference ΔH= I*L | Water depth, d=0.80D | Pipe Length, L |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| no | km2 | m3/s/km2 | m/sec | m3/sec | m2 | m | m2 | m | m/s | m/m | M | m | m |
| 1 | 0.3 | 0.5 | 1 | 0.15 | 0.15 | 0.5 | 0.168 | 0.125 | 0.89 | 0.002 | 0.006 | 0.4 | 3 |
| 2 | 0.25 | 0.20 | 0.5 | 0.05 | 0.10 | 0.4 | 0.10 | 0.11 | 0.499 | 0.00085 | 0.0025 | 0.32 | 3 |
| 3 | 0.25 | 0.30 | 1.25 | 0.075 | 0.06 | 0.3 | 0.061 | 0.091 | 1.237 | 0.007 | 0.022 | 0.24 | 3 |
| 4 | 0.25 | 0.30 | 1.25 | 0.075 | 0.06 | 0.3 | 0.061 | 0.091 | 1.237 | 0.007 | 0.022 | 0.24 | 3 |
| 5 | 0.20 | 0.50 | 1.00 | 0.100 | 0.10 | 0.4 | 0.108 | 0.122 | 0.928 | 0.003 | 0.008 | 0.32 | 3 |
| 6 | 0.25 | 0.30 | 1.25 | 0.075 | 0.06 | 0.3 | 0.061 | 0.091 | 1.237 | 0.007 | 0.022 | 0.24 | 3 |
| 7 | 0.35 | 0.25 | 0.25 | 0.087 | 0.35 | 0.7 | 0.35 | 0.23 | 0.25 | 0.00009 | 0.00027 | 0.56 | 3 |

6.20 Road crossing box culvert

These structures are laid on the main canal  andsecondary canal crossing the main road .For the structure a box culvert having similar depth and width with the parent canal is designed. The Bridge is designed for providing bearing capacity to heavy trucks. The thickness of RCC used in the bridge is 0.30m and it has double RCC. The culvert extends from station 396  SC up to SC 404. That means , the maximum width of the road was assumed to be 8m.

The location of the road crossing irrigation box culvert is at MC-1221$ at sc-392. There locations and dimensions are shown in the drawing.

The design procedure

Hydraulic calculation

Hydraulic characteristics of the canal

Hydraulic characteristics of these box culverts are the same as that of parent or mother    canal.

2) Structural design

Design conditions

Allowable Stress, ∂ca (concrete)=55Kg/cm2

∂sa  (reinforcement) =1400Kg/cm2

ﺝa (Shear) =9.5Kg/cm2

The maximum load assumed here in the analysis of the slab was 28 tone (28,000kg)

A.  Moment from the bridge cross section

The moment to be calculated should be considered across the canal flow.

Assuming the value of C = 30cm,

L=b+2*d=0.2+2*0.7=1.6m, L is length of cross section (m)

P=28.0tone, P is the load transferred by a tire (t)

Bending moment, BM=P*L/4 =28*1.6/4=11.2tm=1120000Kg.cm

Note: The central point load, P is exerted by one tire at the center of culvert section.

Shear force, ﺝ=P/2=28/2=14 tone=14000Kg

B.  Reinforcement.

Both the tensile and compression zones of the concrete is reinforced

ị. Effective depth of reinforcement

Assume t = 36cm and a cover of 5cm, where t = thickness of slab.

d1= t –5cm, where t is thickness of concrete,

Say, t=36cm→ d1=36-5=31cm.

M = 8.5bd2 for rectangular beam

d2= 0.334 √ (BM/b) = d2=C*(BM/b) ½=0.334* (1120000/100) ½ = 61.16cm

d2=C*(BM/b) ½=0.334* (1120000/100) ½ = 61.16cm

→d2=61.16cm

Where, C=0.334

b=unit cross section=100cm

For check, d1< d2 since 31<31.24--------ok!

Providing a cover of 5cm, the overall depth will be 31+5 = 36cm

ị ị. Main Reinforcement (tensile)

Ast= C*(BM*b) 1/2=0.00244*(1120000*100) 1/2=25.82cm2

Where: C=0.00244

If Ф18mm@ 8cm interval is used, the total area of reinforcement per meter will be,

(π *D2/4)*100cm/8cm= (π*1.82/4)*100/8=25.12cm2

Since 31.8<25.82----------ok!

ịịị. Negative reinforcement (compression)

Asc=Area of compressive reinforcement of steel

=0.2* Ast=0.2*25.82=5.164cm2

If Ф16mm steel bar with spacing of 25cm is used, then the total area of compressive reinforcement per meter is Ф16mm@ 25cm = (π*1.22/4)*100/25=8.042cm2

Since 8.042cm2>4.44cm2--------ok!

ịv. Distribution bars

Asd=Area of distribution bars reinforcement

=0.2* Ast=0.2*25.82=5.164cm2

If Ф12mm steel bar with 17cm interval is used, the total area of distribution

bars reinforcement per meter is (π*1.22cm/4)/100/17=6.65cm2

Since 5.164cm2<6.65cm2-------ok!

C. Shear Stress.

ﺝ=S/ (b*j*d), where: ﺝ=Shear Stress

b=Unit cross section=1m=100cm

j=Effective depth factor, =7/8

d=Effective depth, =31cm

Shear force(S)= P/2=28/2=14t=14000Kg

P=Central point load, =28 tone

→ﺝ= [14000/ (100*(7/8)*31)] =5.16Kg/cm2

Since 5.16Kg/cm2<9.5Kg/cm2 (allowable) -----safe!

D. Foundation

Area of soil to be stressed in the foundation, As=(C+W/2)*2a

Where: C=the length from highly stressed extremes to one of the adjacent truck

=1.25m is used

W= the distance between two adjacent tires. Take W=2.5m

a=Width of masonry foundation wall (one side) that carries the self weight,

Use, a=0.9m

Hence, As= (1.25+2.5/2)*2*0.9=4.5m2

Loads

ị. Live loads=i*P, where: i=1.3, it is a factor of safety.

P=Central point load =28 tone.

Hence Live Load=1.3*28=36.4tone

ị ị. Dead Load =Weight of span of masonry wall plus weight of concrete

Weight of concrete=Span volume of concrete times its specific weight, γc=2.3t/m3

Volume=L*T* Ir*γc

Where: L=Concrete span length

T=Total concrete thickness

Ir=(C+W/2) =1.25+2.5/2=2.5m

Weight of concrete =2*0.37*2.5*2.3=4.25tone

Weight of masonry wall=Volume of masonry wall*its specific weight (γm=2.0t/m3

Considering a unit masonry cross section:

Volume = {2*(0.8*0.8)) + (0.3*0.65) + (1/2*2(0.65*0.31))

= (1.28+0.195+0.201) =1.67m3

→Weight=1.67*2=3.35

Therefore: total dead load=(4.25*40%) +3.35=5.05tone

Live Load=36.4tone

Total load=5.05+36.4=41.45t

Calculated bearing capacity=Total load

Stressed area of foundation

=41.45

4.5	     =9.21t/m2

And this value should be less or equal to the allowable bearing capacity of medium stiff cohesive clay soil i.e10t/m2

Since 9.21<10------------safe!

NOTE: The construction material for the masonry part should be the selected one and the stone should be well brushed. Mix ratio for masonry should be 1:3 and that of concrete should be 1:2:4.

6.21.Design of foot bridge

A 10m long RCC that is 0.10m this is supported by a 30cm masonry wall of the canal. The slope of the box canal under the footbridge is similar to that of the parent canal. The place of the footbridge is on main canal at 1600m,on secondry  canal at 396 mdistance from main canal but if it is required they construct additionally.

The concrete is reinforced by Φ12mm@ 200mm center to center in the tensile zone only.

Sample design

Canal MC Location at required place on this canal Using rectangular canal cross section under the foot bridge, Q= Qparent, dcanal=dculvert=0.47m, fb=0.150m, D= 0.45m, n= 0.014 & S= 0.001

Vo= (Qo/bo*do)=(1/n)*Ro2/3* So (1/2). From which by trial & error, bo =0.90m for Vo=0.89m/s.

Structural design of foot bridges

A 10m long and a (bo +0.60m) wide RCC lies on a 30cm thick canal wall. The RCC is reinforced by 8mm steel bar @200mm center to center in the tensile zone. The thickness of the concrete is 10cm.

RCC

0.45m

0.45m

bo

0.30m

t = 0.10m

D

L=0.6+bo

Design of bed Bar with slab

Bed bars are nothing but frame works constructed on earthen channel having the same hydraulic dimensions as that of the channel.

Bed bar is nothing but it is a structure made of wood, steel, masonry or concrete having the same shape as the earthen canal. The bed bar was covered with a 10cm thick slab of 10m (two meters) length serving as animals and human being crossing. The structural design of the slab is same to that of the foot bridge above. The location of the bed bars is at  necessary station.

10cm concrete slab

Fb

d

b

Side slope=1:1

L=2m.

W

They are used as a frame work to keep the shape and design bed elevation of the channel during canal cleaning and maintenance. They can be constructed from wood, masonry, concrete, wet stone pitch or any other construction materials.

In this project, bed bars with slab were recommended at all points where the canal crosses foot path so that they can function as a frame work and also for animal and human crossing. The total length of each bed bar is 2m.

6.22. Bill of Quantity Of Dildila Abba Oromo

| Bill No | Item | Unit | Quantity | Unit rate | cost |
| --- | --- | --- | --- | --- | --- |
| 1 | Access road clearance on soil formation to the headwork | km | 2.50 | 30000.00 | 75000 |
| 2 | mobilization and demobilization | lps | 1 | 80000.00 | 80000 |
|  | Subtotal |  |  |  | 155000 |
| 3 | Camping in CI sheets |  |  |  |  |
|  | A. Lving rooms 4(3mx3m) |  |  |  |  |
|  | B. Store 1(5mx5m) |  |  |  |  |
|  | C. Kitchen 1(3mx3m) |  |  |  |  |
|  | D. Guard house 1(2.5mx2m) |  |  |  |  |
|  | E. Office 1(5mx5m) |  |  |  |  |
| 3.1 | Site clearance | m2 | 134.70 | 18.00 | 2424.6 |
| 3.2 | Trench and other excavations | m3 | 36.11 | 75.00 | 2708.25 |
| 3.3 | Cart away surplus excavations | m3 | 65.85 | 45.00 | 2963.25 |
| 3.4 | Stone masonry beded in1:3 mortar | m3 | 38.00 | 1700.00 | 64600 |
| 3.5 | 20cm hard core blinded 1:3 mortar | m3 | 24.00 | 700.00 | 16800 |
| 3.6 | 10cm lean concrete | m3 | 9.80 | 2000.00 | 19600 |
| 3.7 | CIS walling (G-32) | m2 | 294.80 | 200.00 | 58960 |
| 3.8 | CIS roofing (G-32) | m2 | 240.00 | 200.00 | 48000 |
| 3.9 | Fencing in barbed wire with wooden poles at 2m interval and wire at 0.20m interval to ht of 1.6m | m | 200.00 | 150.00 | 30000 |
|  | Subtotal |  |  |  | 246056.1 |
| 4 | Head work (10m span masonry broad crested weir weir) |  |  |  |  |
| 4.1 | site clearing | m2 | 500.00 | 18.00 | 9000 |
| 4.3 | Cover dam construction | m3 | 375.00 | 75.00 | 28125 |
| 4.4 | Excavation for foundation of soft soil formation | m3 | 750.00 | 75.00 | 56250 |
| 4.5 | Excavation for foundation of rock soil(soil with cobbles) | m3 | 372.00 | 150.00 | 55800 |
| 4.6 | Excavation of soft soil for river trainings | m3 | 320.00 | 75.00 | 24000 |
| 4.7 | Mass concrete works (1:3:6 mix) | m3 | 306.00 | 2000.00 | 612000 |
| 4.8 | Masonary bedded in 1:3 mortar | m3 | 568.00 | 1700.00 | 965600 |
| 4.9 | Plastering in 1:3 mortarto weir body | m2 | 698.00 | 90.00 | 62820 |
| 4.11 | Intake gate of size 0.70x0.7x3.08m size operated with spindle and shaft of size 50mm diameter and plate thickness 0.50m as per the drawing and fixed in position | pcs | 1.00 | 16000.00 | 16000 |
| 4.12 | Manualy lifted silt gate of size 4m height and 5mm thick plate in 1x1m mm angle iron slot well reinforced as per drawing and fixed in position. | pcs | 1.00 | 4500.00 | 4500 |
| 4.13 | Gabion works at upstream and downstream | m3 | 165.00 | 900.00 | 148500 |
| 4.14 | Dewatering works during construction | lps | 1.00 | 5000.00 | 5000 |
| 4.15 | Dia 60cm concrete pipe supply and fix in position. | pcs | 6.00 | 6000.00 | 36000 |
| 4.16 | weep hole over the guide wall with 2 inch PVC pipe imbedded in masonry walls at 2m vertical and 1.5m horizontal. | lps | 1.00 | 8936.53 | 8936.53 |
| 4.17 | Removal of temporary diversion works and shaping to natural condition | lps | 1.00 | 10581.69 | 10581.69 |
| 4.18 | Fill and compactions with red clay | m3 | 310.00 | 130.00 | 40300 |
| 5 | Subtotal |  |  |  | 2,083,413.22 |
| 5 | Earthen conveying canal MC-655 M long |  |  |  |  |
| 5.1 | Excavation on ordinary soil | m3 | 1830.00 | 75.00 | 137250 |
| 5.2 | Fill and compaction with red clay | m3 | 10.00 | 130.00 | 1300 |
|  | Subtotal |  |  |  | 138550 |
| 6 | Lined Conveyance canal (MC-1)- 1245 M long |  |  |  |  |
| 6.1 | Excavations | m3 | 1494.00 | 75.00 | 112050 |
| 6.2 | Hard core well blinded with 1:3 mortar | m3 | 298.80 | 700.00 | 209160 |
| 6.3 | Masonry bedded in 1: 3 mortar | m3 | 896.40 | 1700.00 | 1523880 |
| 6.4 | Plastering in 1:3 mortar | m2 | 2988.00 | 80.00 | 239040 |
| 6.5 | Lean concrete works (1:3:6 mix) | m3 | 62.25 | 2000.00 | 124500 |
| 6.7 | Back fill and compactions | m3 | 84.40 | 130.00 | 10972 |
|  | Subtotal |  |  |  | 2219602 |
| 7 | Secondary canals(862m earthen) |  |  |  |  |
| 7.1 | Excavation | m3 | 827.52 | 75.00 | 62064 |
| 7.2 | Fill and compaction | m3 | 315.20 | 130.00 | 40976 |
|  | Subtotal |  |  |  | 103040 |
| 8 | Cattle crossing (#4) |  |  |  |  |
| 8.1 | Excavation and Earth work |  |  |  |  |
| 8.2 | Excavation of ordinary soil | m3 | 49.25 | 75.00 | 3,693.75 |
| 8.3 | Compacted Back fill | m3 | 4.93 | 130.00 | 640.25 |
| 8.4 | Masonry and RCC Works |  |  |  | - |
| 8.5 | 30cm thick basaltic semi-dressed masonry | m3 | 23.52 | 1700.00 | 39,984.00 |
| 8.6 | Plaster top & internal face of the structure | m2 | 23.40 | 80.00 | 1,872.00 |
| 8.7 | Pointing back face of the structure | m2 | 9.60 | 389.00 | 3,734.40 |
| 8.8 | C-20 concrete for slab | m3 | 11.23 | 2600.00 | 29,198.00 |
| 8.9 | Supplying ,cuttimg and placing of Ø12 reinforcement bar for slab | Kg | 83.50 | 58.90 | 4,918.15 |
| 8.10. | Soft wire | Kg | 8.35 | 48.50 | 404.98 |
|  | Subtotal |  |  |  | 84,445.53 |
| 9 | Chute structure(#1) |  |  |  |  |
| 9.1 | Excavation and Earth work |  |  |  |  |
| 9.2 | Excavation of ordinary soil | m3 | 76.12 | 700.00 | 53,284.00 |
| 9.3 | compacted Back fill | m3 | 76.41 | 1700.00 | 129,897.00 |
| 9.4 | Masonry works for chute structure contruction |  |  |  | - |
| 9.5 | 30cm thick basaltic semi-dressed masonry | m3 | 45.00 | 130.00 | 5,850.00 |
| 9.6 | Plaster top & internal face of the structure | m2 | 130.00 | 138.93 | 18,060.90 |
| 9.7 | Pointing back face of the structure | m2 | 15.75 | 389.00 | 6,126.75 |
| 9.8 | C-7 mass concrete laying on masonry surface | m3 | 30.00 | 2300.00 | 69,000.00 |
|  | Subtotal |  |  |  | 282,218.65 |
| 10 | Road crossing(4 in no) |  |  |  | - |
| 10.1 | Excavation and Earth work |  |  |  | - |
| 10.2 | Excavation of ordinary soil | m3 | 240 | 75.00 | 18,000.00 |
| 10.3 | compacted Back fill under structure and over pipe | m3 | 76.5 | 130.00 | 9,945.00 |
| 10.4 | Masonry works |  |  |  | - |
| 10.5 | 30cm thick basaltic semi-dressed masonry | m3 | 23.5 | 1300.00 | 30,550.00 |
| 10.6 | Plaster top & internal face of the structure | m2 | 9.6 | 138.93 | 1,333.73 |
| 10.7 | Pointing back face of the structure | m2 | 1.92 | 389.00 | 746.88 |
| 10.8 | supply and installation of concrete pipe Ø 500mm | psc | 16 | 650.00 | 10,400.00 |
| 10.9 | C-10 mass concrete laying on masonry surface | m3 | 4.8 | 2400.00 | 11,520.00 |
| 10.10. | laying ofdry ston riprap as per drawing | m3 | 7 | 462.85 | 3,239.95 |
|  | Subtotal |  |  |  | 85,735.56 |
| 11 | Drop structures (13 in No.) |  |  |  |  |
| 11.1 | Excavation on ordinary soil | m3 | 91.00 | 75.00 | 6825 |
| 11.2 | Hard core well blinded with 1:3 mortar | m3 | 35.10 | 700.00 | 24570 |
| 11.3 | Masonry bedded in 1: 3 mortar | m3 | 84.50 | 1700.00 | 143650 |
| 11.4 | Plastering in 1:3 mortar | m2 | 372.40 | 80.00 | 29792 |
| 11.5 | Lean concrete works (1:3:6 mix) | m3 | 1.12 | 2000.00 | 2240 |
| 11.6 | Fill and compaction with red clay | m3 | 10.24 | 130.00 | 1331.2 |
|  | Subtotal |  |  |  | 208408.2 |
| 12 | Field offtakes (34 in No) |  |  |  |  |
| 12.1 | Excavation on ordinary soil | m3 | 68.00 | 75.00 | 5100 |
| 12.2 | Hard core well blinded with 1:3 mortar | m3 | 40.80 | 700.00 | 28560 |
| 12.3 | Masonry bedded in 1: 3 mortar | m3 | 68.00 | 1700.00 | 115600 |
| 12.4 | plastering in 1:3 mortar | m2 | 272.00 | 80.00 | 21760 |
| 12.5 | Supply and install Dia 15cm concrete pipe of 1.5m | pcs | 34.00 | 200.00 | 6800 |
| 12.6 | Gates of size 600x300x3mm in 50x50mm angle iron slot (frame) as per drawing. | Pcs | 34.00 | 4000.00 | 136000 |
| 12.7 | Back fill and compaction | m3 | 12.22 | 130.00 | 1588.6 |
|  | Subtotal |  |  |  | 315408.6 |
| 13 | pipe culverts (7 in No) |  |  |  |  |
| 13.1 | Excavation | m3 | 140.75 | 75.00 | 10556.25 |
| 13.2 | Hard core well blinded with 1:3 mortar | m3 | 56.80 | 700.00 | 39760 |
| 13.3 | Lean concrete works (1:3:6 mix) | m3 | 2.49 | 2000.00 | 4980 |
| 13.4 | Masonry bedded in 1: 3 mortar | m3 | 48.71 | 1700.00 | 82807 |
| 13.5 | plastering in 1:3 mortar | m2 | 192.04 | 80.00 | 15363.2 |
| 13.6 | Concrete pipe of 50cm dia supply & install as per drawing | pcs | 42.60 | 450.00 | 19170 |
| 13.7 | fill and compaction with red clay | m3 | 62.05 | 130.00 | 8066.5 |
|  | Subtotal |  |  |  | 180,702.95 |
| 14 | Pond and related structures |  |  |  |  |
| 14.1 | Excavation on ordinary soil | m3 | 3500.00 | 75.00 | 262500 |
| 14.2 | fill and compaction with red clay | m3 | 1315.00 | 130.00 | 170950 |
| 14.3 | pond inlet |  |  |  | 0 |
|  | a. Excavation on ordinary soil | m2 | 13.44 | 75.00 | 1008 |
|  | b. Hard core blindded with 1:3 mortar | m3 | 5.04 | 700.00 | 3528 |
|  | c. Masonry bedded in 1: 3 mortar | m3 | 25.30 | 1700.00 | 43010 |
|  | d. plastering in 1:3 mortar | m2 | 53.00 | 80.00 | 4240 |
|  | e. fill and compaction with red clay | m3 | 15.80 | 130.00 | 2054 |
| 14.4 | Pond spillway |  |  |  | 0 |
|  | a. excavation on ordinary soil | m3 | 13.86 | 75.00 | 1039.5 |
|  | b. Hard core well blinded with 1:3 mortar | m3 | 4.62 | 700.00 | 3234 |
|  | c. Masonry bedded in 1: 3 mortar | m3 | 32.00 | 1700.00 | 54400 |
|  | d. plastering in 1:3 mortar | m2 | 68.30 | 80.00 | 5464 |
|  | e. clay back fill and compaction | m3 | 4.20 | 130.00 | 546 |
| 14.5 | Pond outlet |  |  |  | 0 |
|  | a. Excavation on ordinary soil | m3 | 94.50 | 75.00 | 7087.5 |
|  | b. Hard core well blinded with 1:3 mortar | m3 | 10.56 | 700.00 | 7392 |
|  | c. Masonry bedded in 1: 3 mortar | m3 | 24.00 | 1700.00 | 40800 |
|  | d. RCC in 1:2:4 mix as per reinforcement detail of drawing. | m3 | 2.30 | 3800.00 | 8740 |
|  | e. supply and fix 50cm diameter concrete pipe | pcs | 10.00 | 450.00 | 4500 |
|  | f. Gate of size 2600x60x5mm operated with spindle and 50mm square threaded shaft as per drawing and specifications. | lps | 1.00 | 25935.63 | 25935.63 |
|  | g. plastering in 1:3 mortar | m2 | 29.20 | 80.00 | 2336 |
|  | h. Back fill and compaction | m3 | 70.50 | 130.00 | 9165 |
|  | j. Fencing in barbed wire with wooden poles at 2m interval and wire at 0.20m interval to ht of 1.6m | m | 280.00 | 150.00 | 42000 |
|  | Subtotal |  |  |  | 699,929.63 |
| 15 | Division boxes (4 in No.) |  |  |  |  |
| 15.1 | excavation on ordinary soil | m3 | 12.30 | 75.00 | 922.5 |
| 15.2 | Hard core well blinded with 1:3 mortar | m3 | 10.80 | 700.00 | 7560 |
| 15.3 | Masonry bedded in 1: 3 mortar | m3 | 14.10 | 1700.00 | 23970 |
| 15.4 | plastering in 1:3 mortar | m2 | 36.00 | 80.00 | 2880 |
| 15.5 | Gates of size 500x300x3mm in 50x50mm angle iron slot (frame) as per drawing. | pcs | 4.00 | 4000.00 | 16000 |
| 15.6 | Back fill and compaction | m3 | 6.00 | 130.00 | 780 |
|  | Subtotal |  |  |  | 52112.5 |
| 16 | Flume (12m) |  |  |  |  |
| 16.1 | Excavation of soil | m3 | 20.00 | 75.00 | 1500 |
| 16.2 | Masonary bedded in 1:3 mortar | m3 | 15.70 | 1700.00 | 26690 |
| 16.3 | Reinforment concrete work | m3 | 28.00 | 2600.00 | 72800 |
| 16.4 | Plastering works | m2 | 60.20 | 200.00 | 12040 |
|  | Subtotal |  |  |  | 113030 |
|  | Grand Total |  |  |  | 6,883,207.41 |
|  | vat 15% |  |  |  | 1,032,481.11 |
|  | Grand Total with VAT |  |  |  | 7,915,688.52 |

7. Watershed Management project

7.1 . Watershed Description

Location

The watershed of Dindila Abba Gada river lies geographically between 1018760N    latitude and 307238  E longitude. It is located at a distance of 57 km from Shambu to the project site, all 65 km dry weather road from shambu to project site. On the other hand the project site is 15 km away from the district capital town of Wayu is and all are dry weather road. The altitude of the watershed varies from the highest elevation of 2560 m.a.s.l to lowest elevation of 2270 m.a.sl.

Size and shape

The total area of the watershed is 84 km2 that includes about 2(two) PAs`by the name Badda warqee and Guddata dobii. And the shape is long and narrow and likely to have longer time of concentration.

Climate

The watershed of Dindila Abba Gada project is categorized under Dega, and Weyna Dega   agro climatic zone, i.e dega and  Moist Weyna Dega’ in the upstream and downstream area respectively. The metrological data is taken from shambu.

The existing crops are Annual and perennial field crops. The most parts of upstream and downstream cultivated agricultural farm land by field crops of teff, been,wheat, sourgam,bee and maize. The agro ecological zone of this watershed covers about Dega 15.16%, and  Weyna Dega84.84%  from the total area of the watershed.

Rain fall:-

The mean annual rain falloff this watershed area that is taken from the 2 years recorded values from similar catchments of rain fall of nearest district town metrological data of shambu is about 1794.7 mm/year.

Temperature

The total monthly mean maximum and minimum temperature ranges from 22.7 0C to 11.3 0C respectively.The dry month from October to May and the rainy moth are from end of May to midi September

Geologic Characteristic

The nature of parent rocks are basalt rocks of sand stone. From the parent volcanic rock i.e the sandstone rock found in the area undergoes weathering and due to this it is very susceptible for erosion. And the command area encompasses between long mountainous on both sides. That the poor soil depth due to resistances of this lime stone has resulted in to very sparseness of the vegetation cover on the mountainous indicates the reasonably probable amount of siltation problems.

Slope

The watershed area has different slope ranges in different land uses, in that In cultivated land use the slope moderate strong to moderate steep slope land form slope in ranges of (3-8)% and (8-15)% in the command area land uses the slope ranges nearly gently plain(flat area and of (3-5)% to (5-8)% and rolling to steep slope of (15-30)% to (30-50)% in rain fed agricultural lands of the watershed respectively. In Homestead land uses in all land uses the slope ranges from moderate steep slope of (3-8)%to (8-15)% and in rolling slope range (15-30)%. In bush and shrubs (scrub) and forest land uses the slope ranges from rolling to nearly very steep slopes of (15-30)% to(30-45)% and >50% In some places of the two land uses respectively. In Grazing land use they use the grazing area in mountainous hilly area and in some places of downstream end area of plain land form that the slope ranges from (15-35)% and (0-3)% respectively.

Surface Drainage.

In up and down streams of right and left sides of the watershed catchments land almost covered by more of bush and shrubs and agricultural perennial field crops vegetation so that the area has relatively rapid response to rain fall therefore, the existing natural vegetation and field crops are capable of percolating the water in the ground so that surface drainage of the area some what good.

Physiographic

The landscape of the watershed area features indicates the existence of high mountain relief, rugged, moderate land forms, and small gully formation at some area of the up and down stream of the watershed. The land forms mountainous plateau and hilly in forest and bush and shrubs grazing land uses. And also, undulating, gently undulating and plain land forms in cultivated, Homestead and some places of grazing land uses.

The high mountain relief elevated _____ m.a.s.l and decrease down to the head work site2276 m.a.s.l and to the downstream end point ______ m.a.s.l.

Soils

The soil formation of the sub-watershed is dominated by physical and chemical weathering of the parent material which is loam and clay loam in texture and brown in color from both sides. The topographic feature, vegetation, climate of the area, the land use practices and other associated factors also contributed their share for the general characteristics of the soil.

7.2 Land Use and Cover

Land use

In this watershed there is limited number present land use areas are found which exists in different slope ranges of the land.

The following table: - shows the Present land use of the watershed Area in tabular form.

Present Land use & cover condition

| No | Land use type | Area coverage of the total % | Area in ha | Slope in % and its hectare |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | (0-3) | (3-8) | (8-!5) | (!5-30) | (30-50) | >50 |
| 1 | Cultivated and settlement area | 82.94 | 2490 |  |  |  |  |  |  |
| 2 | Grazing land | 13.05 | 392 |  |  |  |  |  |  |
| 3 | Forestland with Bush shrubs(scrub) | 1.69 | 51 |  |  |  |  |  |  |
| 4 | Construction area | 1.49 | 45 |  |  |  |  |  |  |
| 5 | Miscellaneous land | 0.79 | 24 |  |  |  |  |  |  |
| Total |  | 100 | 3002 |  |  |  |  |  |  |

Cultivated and settlement area.

the crop types grown in the watershed of agricultural rain fed and command farm land area are:- Maize, sorghum, Teff, wheat, nug, bean, bee, potato, cabbage, pepper and sugar cane. It covers 82.94% of the land as a whole as cultivated. And out of this 0.5 % irrigated traditionally & the farmers focus to grow more chat than crops the command area.

Grazing land:-

The area for grazing purposes used by the farmer is more in the mountain and hilly land and on both sides along the river course. That covers about 13.05% the total land and also, since the major dominant grazing land is not sufficient enough to satisfy the need of the farmers, they cover the deficiency by using crop residue.

Forest land, bush and shrubs

There are some forests found in some mountainous region of the watershed (about 1.69). This covers almost about 1.69% the total coverage. The dominant species of trees found in the area are:- eucalyptus, cheka,bargamo, ejjersa, rejjii,birbirsa, bakkanisa, muka arba, and harbu(by local name).

Miscellaneous:-

This land use include, Gorge, seasonal and perennial river, watercourse (watershed) drainage line, road sides, gullies, chiffs, etc and covers about 0.79% of the total watershed area.

Economic Data

Human population:-

The beneficiaries of this project are two peasant associations from the total of four peasant associations in the watershed area of dindila abba gala but only ______ of the population are beneficiary of the command area. The total populations of the two PAs’ are 6845, out of which 3418 are male and 3427 are female. The average family size of person per house hold is 3 persons.

Land holdings:-

The total estimated agricultural land of the two PA’s are 3002ha and the average size of land in one family is 2.05 ha.

Livestock production:-

According to the data obtained from Badda Warkee and Guddata dobii PA’S office of agricultural agent, the following table presents the livestock population.

This Table shows livestock Type and quantity

| No | PA | Cows | Sheep | Goat | Heifers | Hen | Donkey | Mule | Horse | Bull | Oxen | Total |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Badda warkee | 1590 | 1314 | 290 | 532 | 1582 | 99 | 35 | 160 | 420 | 1215 |  |
| 2 | Guddata Dobii | 991 | 675 | 350 | 405 | 1280 | 95 | 27 | 117 | 335 | 1023 |  |
| Total |  | 2581 | 1989 | 640 | 937 | 2862 | 194 | 62 | 277 | 755 | 2238 |  |

Source:  Guddata dobii and Badda warkee agricultural PA’S of agricultural agent.

Socio-economic conditions:-

The socio-conditions status of the peoples are better due to per house hold land holding and the availability of rain and irrigation water but the land form also almost steepe and not comfortable for intensive agricultural practices even though, different types of crops production practices are poor, livestock production also is so good even though the grazing land is limited.

Infrastructures:-

In the project site there is a clinic, but there is two schools (1-8) and (1-6) in two PAs’ and two Development agent (DA) in all two PA’s, market was available in the area.

7.3 Watershed problems

Sediment Damage:-

During rainy season the flood overflow out of the river course in some places of agriculture land on both side and un load. Its sediment this has been done due to the slope of agricultural land become flat in some place of command area

Erosion Hazard:-

There are two places where erosion affects the land in the right side by flowing through water way drainage and crosses and cuts the canal and create gullies and bring loss of agricultural land this effect is observed on this natural drainage watercourse but need attention before great disaster happened.

Water resource and its management problems

Since the water resource is from one river all peoples are their serve in side due to this mismanagement of water use ling malaria diseases and intestinal worm infestation is great.  But on irrigation water use, they use still traditional irrigation system, therefore it is necessary to provide for all lands of existing command area available technical of irrigation use.

Proposed Management programs

Management programs for agricultural land

In the agricultural land hill and gully erosion occurs from the above mountainous down to the canal route and agricultural farm land at two places of the right side of the command area of the project. Therefore, according to the land capability classification the following measures should be considered.

Land class unit II and III

This class unit of land has limiting factor severe erosion (E3) in side agricultural farm land above and below the right canal route both in gully therefore, it needs special soil conservation practices of like contour farming

Land class unit IV and VI

These class unit of land has limiting factor very server erosion (E4) in between agricultural farm land and miscellaneous (gullies)  land use  above and below the gully side the drainage line which that cuts the canal line and destructs both the canal and agricultural farm land. Therefore, these class units of drainage line lands since strongly sloppy with shallow depth soil it is highly suceptable to erosion to pass the above flood agricultural farm and canal. It needs the conservation measure of check dam (physical conservation structures) with integrations of biological conservation measures like planting Grass strips and forage crops inside the gap between check dams and on the check dam line that conserve the farm lands and to protect the line in the command area of agricultural land.

a/  Contour farming

Objectives:- Water infiltration in to the soil will be encouraged

- Down slope runoff water will be prevented

- Easy for draft animals during ploughing

Method:- During ploughing  apply 0.5 along the contour towards the water ways. By any means do not deviate during   ploughing from the contour more than 3 % Apart from this crops should be planted up & down hill from the line while rows should be normally planted on contours,

b/ strip cropping

Objectives:- to increase infiltration and to provide cattle feed mulch

to hold back the run off water with soil particles from the poorly protected strip, in the strip with the dense cover crop.

Method:- Alternate an erosion-permitting crop sorghum or maize with an erosion resisting crop ground nut or sweet potato. Each year alter the place of strip from the previous area.

C/ multiple cropping

Objectives:-  Improve soil structure, reduce diseases & pests

prevent nutrients from being washed out

Protect the soil by ensuring a more intensive and prolonged coverage.

Methods

Inter cropping:- the different crops are grown in rows next to each other. Use   haricot bean to be grown between or under maize.

For Land class Unit VIID

All of these land class units found on agricultural land as well as on bush & shrubs land should be treated in the same way as described below.

a/ Area closure:-

Objectives:- To prevent further degradation of Land

-To avoid human & animals intervention

- To help to rehabilitate vegetation cover to its natural condition.

Method:- (1) to keep human & animal intervention away from the bounded area for 3 to 5 years until 80% natural gross cover is obtained.

2/ To teach the farmers found in the area about the conservation of natural resources and its wise use of these resources to keep balance of nature.

3/ It should only be done totally with the agreement of the members of the watershed area in habitants.

b/ covert to forest land

Objectives:- To restore the land in to its farmer natural vegetation cover

to minimize soil & water loss

To make use of trees at later time for construction material, fuel & etc.

Method:- Use the following two method

Pitting:- the depth of pitting should be 30-50 cm and its diameter will be 30 cm . Spacing between pits within the row should be 3.5m and spacing between two pit lines will be 4m.Therefore the number of pits per hectare will be 714.

Tree species:- According to the  agro climatic condition of the area and depending on the existing tree species of the area, the following selection will fit to the environment and have economical value.

Acacia grrardi

Olea Africana

Moringa(shifara)

Sesbania

Avocado

Eucalyptus

Jacaranda

Gravilia

Gatira (local name)

Qundibarbare(local name

2/ Planting:- The required tree species of seedlings taken from the nursery site will    be planted in the prepared pits after separating the plastic tube from the seedling.

Light compaction around plant during planting.

Area:- Total Land area to be treated as forest land from the class unit of VIII will have 62 ha.

Cost:- the necessary cost for establishing forest land described above are estimated in the following manner taking in to consideration the remaining works.

This Table shows Forest Land establishment cost estimation

| No | Work item | Unit | Quantity | Unit cost (Birr) | Total cost (Birr) | Remark |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | Pitting | No | 7881 | 1 | 7881 |  |
| 2 | Seedling | No | 7881 | 1 | 7881 | Without transport |
| 3 | Planting | No | 7881 | 0.5 | 3940.5 |  |
|  |  |  |  | Total | 19702.5 |  |

C/ Cut and carry

Objectives:  - To shun over grazing and their trampling of soil

- To increase the growth of grasses by the use of programmed cutting

- To help wild life growth better than before

- To maximize animal fodder thereby increase their production in       quantity &  Quality

Method:- As to the name also implies the grown grasses should be cut according to the traditional practice and carried to their animals. But direct feeding of animals over the pastures should be forbidden.

7.3 Management programs for erosion Hazards

A. gullies treatment

Objective:- to create staggering techniques to reduce the velocity and there by minimize erosion

To provide vegetation cover & protects the gullies against scouring

To make the silt to deposit over the gullies bed which carried by the running water

To increase the time of opportunity of flow in the gullies and thereby increase the scope for per collating of water in to the soil

Method:-

Around all the gullies at least 10m width on each side along its length should be protected from human and animals interventions.

At the bed of the gullies gravilia and jacaranda should be planted across the flow alternately at a distance of 5m each between row & 1m between plants.

At the top embankment of the gullies, the first 5m from the edge via the farm land on both side along the gullies length should be planted with gravilia only at 1m x1m But the rest of the remaining 1m on both side via gravilia plantation should be planted with thorny species such as cactus to inhibit human & animals.

Cost:- the estimated cost comprise not for particular gully but instead over all area of gullies shown below in table 10

Table 10 Cost estimation for gullies vegetation.

| NO | Work Item | Unit | Quantity | Unit cost (Birr) | Total Cost (Birr) | Remark |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | Pitting | No | 2563 | 1 | 2563 |  |
| 2 | Seedling | No | 2563 | 1 | 2563 | Without transport |
| 3 | Planting | No | 2563 | 0.5 | 1281.5 |  |
|  |  |  |  | Total cost | 6407.5 |  |

Tools& Instruments

The tools & instruments which are used & needed to facilitate the work for the intended conservation work for the proposed watershed area are listed below in table 11 with their respective cost estimation

Table:-  tools quantity & their Cost estimation

| No | Type of tools | Unit | Quantity | Unit price(birr) | Total price(birr) |
| --- | --- | --- | --- | --- | --- |
| 1 | Spade | Pcs | 10 | 35 | 350 |
| 2 | Gajjamo | “ | 5 | 70 | 350 |
| 3 | Pickaxe | “ | 5 | 75 | 375 |
| 4 | Hoe | “ | 30 | 60 | 1800 |
| 5 | Hammer(big) | “ | 2 | 150 | 300 |
| 6 | Hammer(small) | “ | 2 | 80 | 160 |
| 7 | Plastic rope | Roll | 2 | 100 | 200 |
| 8 | Tape meter(50m) | “Pcs | 2 | 250 | 500 |
| 9 | GPS | “ | 1 | 6000 | 6000 |
| 10 | SICLE( maciidi) | “ | 5 | 50 | 250 |
| 11 | “ Geso” | “ | 30 | 45 | 1350 |
| 12 | Scissors | “ | 10 | 15 | 150 |
|  |  |  |  | Total | 11785 |

4. Summary of cost

The main tasks of watershed management programs are to bring a solution for the constraint & problems of the catchment especially which affects the command area as well as canals & farm structures

Within this frame work it will reduce and minimize the erosion hazard as its main objectives indicate so.Consequently it will decrease and minimize sediment deposit from agricultural land to irrigation canals and farm structures including nigh storage

Last but not least it will improve animals feeding and enhance soil moisture and infiltration action.

Table 12 cost estimated summary for watershed

| NO | Work item | Unit | Total cost (birr) | Remark |
| --- | --- | --- | --- | --- |
| 1 | A forestation | “ | 19702.5 |  |
| 2 | Gullies treatment | “ | 6407.5 |  |
| 3 | Tools | “ | 11785 |  |
|  | Total | “ | 37895 |  |

7.4 Conclusion and Recommendations

The following recommendation should be considered seriously and followed accordingly.

Planting of seedlings should be taken place just at the comment of any rainy season.

Protect the seedlings planted and the area closure from human & animals’ intervention.

Therefore,when the management programs & the recommendation taken in to consideration will bring the target result without doubt.

8. ENVIROMENTAL IMPACT ASSESSMENT

8.1 Methodologies

The information required for  measured environmental impact assessment of irrigation for Dindila Abba Gala diversion weir project are taken in the following order of manner taking into consideration the main and major existing environmental constraints  the area.

To observe the existing natural resource at which level and extent found

To inquire the farmers in & around the project area about the constraints faced and the impact of the project if it is going to be implemented and its operation after completion. It is also discussed on resource uses and major disease problems

To identify and assure the required information & further data were taken from the peasant association of Guddata dobii and Badda warqee health post & developmental of agricultural agent.

Time given for environmental assessment were restricted to seven days because of this reason the time taken for environmental assessment as a whole is limited to the given five days& the result depends on it.

8.2 MAJOR EXISTING ENVIRONMENTAL CONSTRAINTS

Public health problems:-

There is two health extension agents in the project area, but this not solve the problem of the area except in the district of J/Raree at wayu. The neighboring health facilities are about 8km far away from the project area..

The serious major diseases in the project area are malaria, diarrhea, intestinal parasite, pneumonia and Fever, the reason behind these are

a) Sanitation since human & animals are living together under the same roof.

b) There is no pure supply of drinking water.

Furthermore there is no regular health education and treatment in the area even if there is planned program for this purpose. This problem happened because of the reason of absence of enough health officers.

The main water related diseases in the project area are:-

- Water borne diseases:- bacterial dysentery, typhoid & ameba.

- Insect vector borne:- malaria.

In addition to the above human diseases mentioned, animal diseases are seen in the area and can be depicted here under on the following condition

Therefore the prevalent livestock diseases found in the area are:-

I  Bacterial diseases

a) Anthrax

b) Black log

c) pasterolosis

II. Internal parasite

a) Strongyle

b) Ascaris

c) Dictyocules

d) Taenia

III External Parasite

Tick

Lice

Manage

Water supply and sanitation

The farmers uses the river as their water supply and irrigation, which is not clean for drinking purposes, the most serious diseases cited above are the result of the uses of quality water, which is also aggravated by sorts sanitation i.e., framers excrete everywhere in the field and no latrine available; living together with animals in a single, and their low level of living standard.

The villages in the project area have some latrine but not all peoples because of this many farmers daily excrete in the field, around home, along the road, in the river course and ever where without knowing its impact to their health and the water they drink.

Deforestation

One of the many reasons of deforestation seen in the area is the expansion of agricultural land; for the uses of construction materials: for local consumption of fire wood and charcoal sell.

Because deforestation result brought negative impact of the environment, which land the area into rainfall shortage unreliability and uneven distribution As a consequence of this food insecurity became reality

Deforestation not only affect human but also great impact on animals & wildlife by decreasing their number due to  the less of grazing area, fodder, shelter and breeding  places.

Due to the above reasons the area were affected by drought. Therefore, the existing natural resource degradation should be controlled.

Potential environmental impact

The potentiality of the project area can be treated in two ways.

Positive environmental impact

Theproject of Dindila Abba Oromo can help the farmers to have first of all permanent diversion weir, farm structures and increase the command area by making use of supplementary irrigation it will help them to solve the crop  moisture stress during uneven distribution & shortage of rainfall applying supplementary irrigation.

The implementation of the project substantially changes to mode of life of the society as the user receives more income from intensified crop production.

Conveyances less will be minimized and permanent farm structures grants them to use water effectively & efficiently for the required aim & purpose.

During the construction of the project, the project increase income of the local farmers by providing job opportunity for the people. These also improve the socio-economic environment.

- Crop and animal productions will be improved. This is attained by the sub-watershed management component of the project. The sub-water shed management and development is aimed at conserving the already existing resources, regeneration of the degraded resource, grass land improvement and over all integrated sub-watershed development and management that will increase both crop and animal production.

- In short all these are the major positive environmental impacts of the project.

Negative environmental impact

Impact related to the project construction

Since the project site is found in narrow place, it will make road construction difficult and affect some of cultivated land out of question.

Unless and other wise care should be taken and considered seriously it will drag construction progress especially for heavy vehicles movement. The existing dry whether roads unless well maintained it is not accessible so that material supplies and other things can be hindered and construction progress will lag.

The first job to be done before construction will be to make new road access to the head work & command area where farm structures constructed and to maintain the existing access road from Dindila Abba Gala to the project site.

Impact related to the project operation

Apart from the existing irrigation land, the added additional command area uses only supplementary irrigation, hence during operation misuse management of flood may affect and lead to land degradation and recurrent silt or sediment deposit will happen in canals and night storage due to the uses of flood for irrigation purpose.

The technical faults that may happen during the whole project construction can cause serious damage on the environment.

8.3 Environmental mitigation measures

Mitigation Measures for public health

The strategies should be in the establishment of health post service and quality water supply for the project area.

Furthermore the following preventive measures can also be done by the farmers without external aid.

Clear out stagnant water , which mosquitoes breeding sites,

Decrease human- insect contact by the use of mosquitoes not.

Improve surface water management

Improve hygiene by the use of latrine, boil water before drinking, washing clothes &body regularly, and shun human &animals living together.

8.4  Mitigation measures for water supply & sanitation

Some of the many mitigation measures concerning water supply and sanitation can be described in the following manner

Improve water quality by developing the existing spring and boil water before use.

Improve hygiene by not using unimproved water sources;

Improve hygiene by not using unimproved water sources; use latrine and boil water before use separate why human & animal body at regular basis.

To make this mitigation  measure practical the following table 1 shows the cost estimation used for the necessary structure that can alleviate the foretold constraints

Table 1 cost estimation for water supply & sanitation program

| No. | Work Item | Unit | Quantity | Unit cost birr | Total cost |
| --- | --- | --- | --- | --- | --- |
| 1 | Spring development | No. | 1 | 15,000 | 15,000 |
| 2 | Latrine | “ | 130 | 200 | 26000 |
| Total cost |  |  |  |  | 41,000 |

Mitigation measures for deforestation

To protect & minimize deforestation due to land expansion, and construction material and etc uses of reason, the following major activities lessen the aggravation to reduced extent and should be followed and considered seriously.

Area closure should be applied to on steeply slope land

Initiate and encourage individual and community forestry development

Prevent further deforestation and train farmers about conservation practice of soil, water and forest.

Designate land according to the low and supported by protection institutions.

8.5 Conclusion and recommendation

Considering the existing negative environmental impact the following work should be given priority and constructed with the project. Hence the recommendation will be:-

construct spring development

construct latrine per individual family head

Designate land for area closure

Most likely the above cited recommendation can alleviate the constraints so that its implementation & management should go in accordance of the project plan.

Generally, the project is environmentally feasible with the above positive impacts and controllable negative impacts.

9. FINANCIAL ANALYSIS OF DILDILA ABBA OROMO PROJECT

9.1 Introduction

The Financial analysis of Dindila aba Oromo River Small-Scale Irrigation project is conducted since it is the crucial measure to identify the general effects of the project, in financial terms, on farmers to determine the profitability of the project for the intended beneficiaries.This financial analysis is intended to assess the financial impacts on project participant (farmers) and which seek to assess the financial effects on the existing farm household and their families, which will be involved in investing in and operating the project.

The analysis will indicate the net returns, which will accrue to the farmers. The magnitude of these returns will in turn; reflect the extent of incentive for them in order to adopt new and improved inputs and irrigation practices. Also financial analysis is expected to assess commercial feasibility of the project in terms of the availability of inputs supply and marketing of outputs with their appropriate market prices at the local level.

The development of Dindila aba Oromo SS irrigation project will essential for agricultural growth and its benefit will be well recognized by the expected farmers around the project area. Thus standing from the study stage to the end of implementation period due attention should/will  be given  to ,Communities promotions, awareness, and detail understanding  on the way to practice and the benefit gained from the proposed  modern irrigation activities. These enables farmers to increase cropping intensity that will be grown in two seasons on the same land as the result they: - bring new land under cultivation, get higher crop yields from existing land, shift to higher value crops and optimize crop verities in line with their major uses and achieve other benefits.

These benefits initiate the expected farmers to have considerable positive impact in their economy in the form of rising demand to irrigation practice. In addition to scale up production benefits, irrigation also normally generates increased demand for labor and indirectly stimulates other economic activities in the surrounding areas. An assessment of project outputs and returns is the basis for evaluating the project’s profitability and viability

Therefore, in this study, the three measures of project viability are- Net Present Value (NPV), Internal Rate of Return (IRR) and Benefit Cost Ratio (BCR), are used to determine the viability of the project. Thus, the main estimates included with respect to financial and economic analysis are: cost structure and benefit streams, net returns from the existing and proposed crops, farm budgets, calculations of NPV, IRR and B/C, tentative source of finance, share of investments and sensitivity analysis. Accordingly this financial analysis is conducted for Dindila aba Oromo SSIP in all the detail procedures and processes that would presented in the following sub topics.

Objective

The major objective of the financial analysis of this study is to determine the financial effect of the project from the points of view of the target beneficiary (i.e. production levels, labor requirements and net income ‘with’ and ‘without’ the project) and scheme level (i.e. costs incurred in constructing, operating and managing the whole scheme). Scheme level costs are then compared with estimated income from the scheme to assess the financial benefits of investing in irrigation. Methods used

The methods used to undertake the financial analysis of Dindila aba Oromosmall-scale irrigation project include, discussions made with different district offices, the target beneficiary, development agents of the project site and reviews of secondary data.

In the analysis, the preparation of crop budgets “with” and “without” project was considered. The price of inputs, outputs and the overall operating expenses are considered in the financial analysis both in the costs and benefits which incurred and sold at farm gate price obtained from local nearest market such as Wayu, Goban, Balbala, Bada Warke and Addis Aeba, Regional town.

Purpose of financial analysis

The purpose of financial analysis of a project is to examine the financial viability of the project from the farmers’ point of view.

9.3 Farm Income Analysis

In analyzing Dindila aba Oromo project, the underlying assumption made is that, farming community, the objective will be maximization of the income that the families will earn because of participating in the project. To achieve the objective, the resource use, the income generated by the operation of the project and the investment were analyzed.

The resources used consists land, water, labor and inputs. The tools used to evaluate these resources are cropping patterns, labor requirements and crop budgets.

Crop Budgets and Cropping patterns

The financial crop budgets are designed to calculate the financial gross and net returns to crops per ha under the project. Generally, the crop budgets with and without project conditions have been prepared because of the requirements of determining the incremental return which can be solely attributed to the project. Under this financial analysis crop budgets were prepared for both with and without project situation. When an irrigation project is introduced, the area for irrigation might be taken from the participating farmers’ landholdings being used for rain-fed cultivation. If the farmers become full-time irrigators, they might commit all their rain-fed land. This means that by switching to irrigation the income that used to come from this rain-fed land is lost and the income from irrigation is gained. In order to assess the impact of this, the existing cropping pattern on the rain-fed land was established and then look at the new cropping pattern for the irrigated area.

Components of Crop Budgets with Project

In this financial analysis the preparation of crop budgets has taken from Agronomist includes:

The preparation of cropping intensity (Land Development Plan)

The selection of crops to be grown

Determination of optimum yield for each crop to be achieved.

Yield build-up period from year 1 to the year when optimum yield is achieved.

Optimum level of uses of inputs at the optimum yield level

Pattern of uses of inputs from year 1 to the year when optimum yield is achieved

Prices  of inputs and outputs

Components of Crop Budges ‘Without” Project.

Existing crops, cultivated land, yield and existing price of outputs and inputs are considered for the preparation of crop budget.

Comparison Between With and Without Project

This analysis involves the comparison of financial situation with project and without project. It is not worthy that the analysis does not seek to compare the financial situation before and after the project. In making this analysis therefore, all the relevant cost and benefit items with and without the project have been identified and valued with appropriate prices and incremental net return with the project have been calculated.

Net return arises because of the project and the net return also arises without the project as well in many cases. Therefore, incremental net return as a result of the project has been calculated. This is determined by deducting the net return without project from the net return with project.

Benefit

Benefit may be direct and indirect. Direct benefits may be tangible and intangible. Tangible benefits for an agricultural project may result from increased production, improvement in quality, increase in value or reduction in cost etc. In case of a production orientated project like this one; increased production is the most important tangible benefit. In financial analysis, of this project, the tangible benefits are normally quantified and analyzed. In such analysis, intangible benefits and costs are not quantified.

Treatment of Transfer Payments

The most common transfer payments in an agricultural project includes taxes, subsides, loan receipt, interest and repayment of principal. In financial analysis of this project taxes are treated as out flows since these are considered as expenses. Interest also paid to the services of loan and subsidies are benefits to the households from the government.

The ‘without-project’ situation

Table 1 lists the cropping pattern for the ‘without-project’ situation of Dindila abba Oromo small-scale irrigation scheme, which the farmers are currently practicing on their land under traditional irrigation and rain-fed conditions.

As discussed with the target beneficiaries, the command area farmland was occupied not by more than 600 households from both kebeles. This implies that on average more farmers has about 1 hectare of rain-fed plots and traditional irrigation from the net command area. Thus, those farmers’ will be expected to give-up an average of 0.5 hectare towards irrigation development and retain 0.5 hectare for irrigated crop production.

Table 10.1: Current cropping pattern under rain-fed conditions of the proposed irrigation scheme

| No. | Crop type | Area cultivated (ha) |  | % of total area cultiveted |  | Average yield (qt/ha) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Bada Warke | Gudata Doby | Bada Warke | Gudata Doby | Bada Warke | Gudata Doby |  |
| 1 | Teff | 20 | 2 | 23.53 | 33.33 | 9 | 9 |  |
| 2 | Maize | 15 | 0 | 17.65 | 0 | 45 | 45 |  |
| 3 | Wheat | 20 | 2 | 23.53 | 33.33 | 25 | 25 |  |
| 4 | Barley | 5 | 2 | 5.88 | 33.33 | 12 |  | 12 |
| 5 | Bean | 6 | 0 | 7.06 | 0 | 12 |  | 12 |
| 6 | Pea | 4 | 0 | 4.71 | 0 | 9 |  | 9 |
| 7 | Nougar | 15 | 0 | 17.65 | 0 | 5 |  | 5 |
|  | Total | 85 | 6 |  |  | 116 |  | 116 |

Types of crops grown by Traditional irrigation

| Types of crop | Unit | Area cultivated |  | Total Yield/hec |  | Price/qtIn Birr |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Bada Warke | Gudata Doby | Bada Warke | Gudata Doby | Bada Warke | Gudata Doby |
| Maize | he | 55 | 10 | 50 | 50 | 500 | 500 |
| Potato | he | 45 | 30 | 80 | 80 | 250 | 250 |
| Total |  | 100 | 50 |  |  |  |  |

The ‘with-project’ situation

To estimate the benefits of the project, a cropping pattern for the irrigated area is proposed. In proposing a cropping pattern, several factors were taken into consideration (i.e. the farmers’ wishes, marketing aspects, agronomic aspects (soils, climatic condition, crop water requirements, and rotational consideration), access to inputs, labor requirements, etc).

Farmers will have some idea of what they want to grow, stemming from their knowledge of the area and of what is especially in demand in terms of their household requirements or the market. This might or might not coincide with what is feasible, since the proposed crops should be in accordance with the agronomic conditions above. One of the major aspects of choosing a cropping pattern under irrigation is to determine whether there is a market for the crops. The production structure has to fit the market, and possible markets and their supply and demand have to be determined. During the preparation of the cropping pattern, marketing conditions are considered.

The availability of inputs is a factor that influences the implementation of the actual cropping pattern. It may be difficult, however, to take this into account when preparing the feasibility study as supply situations change continuously.

In choosing the crops the following factors are also considered whether the most profitable ones fulfill other requirements that include reliable demand, local consumption potential and food security.

Lastly, the labor requirements were determined in order to establish whether the farmers can provide the extra labor needed.  Having taken all these factors into consideration, a proposed cropping pattern was established (Source: Agronomist).

Table 10.2: Proposed cropping pattern for Dindila abba Oromo irrigation scheme (56.5ha, 200% cropping intensity after fourth year)

Yield estimates were made based on experience from various irrigation schemes which are found around the project area, results found from different research centers and national average. Accordingly, the estimated yields for the proposed irrigation scheme are listed below.

Table 10.3: Yield projection in Dindila abba Oromo proposed irrigation scheme

Relevant Prices

Constant Price

In this project, prices for both cost and benefit items will be taken to be constant.  This means that prices should not vary with inflation over years, although consideration will have to be given if there is a distinct possibility of changes in relative prices.

Farm gate price

Prices of crops here are actually received by farmers and are attributable to their crop production activities and not to any other services. In other words, these prices should therefore, relate to farm gate ones which are actually received by the farmers. Farmers may sell their crops at their farm gate or at the nearest village markets without involving marketing services.  However, if the sale of their crops involves marketing and other services like transport, speculation storage for long time etc, cost on account of these services should be deducted to arrive at the farm gate prices. Therefore farm gate price is considered or used for this financial analysis.

Time of Sale

The prices of farmers’ crops are those which prevail around harvest months. Since farmers normally sale most of their surplus crops during two to four months offer harvest and higher prices which may be possible if crops are stored for a long period involving storage loss and costs are attributable to storage and speculation. Prices of crops to be used to estimate crop budgets are those which should relate to the period that is close to the harvest months.

9.4 Financial price of Inputs and Outputs

Financial price of Inputs

Labor requirements

The labor requirements are calculated on a crop-by-crop basis and added up to estimate the total requirements in any given situation. For Dindila aba Oromo small scale proposed irrigation scheme, labor requirements for each crop were estimated based on information from the target beneficiary. Accordingly, labor requirements for the common irrigated and rain-fed crops are listed in table 4 & 5. They indicate the number of labor days required for individual activities such as land preparation, sowing, fertilizing, irrigating, weeding, etc.

When looking at the labor requirements associated with the project, two main questions have to be addressed. These are whether additional labor is required for   the project or family labor cope with

the total labor requirements in the ‘with-project’ situation. Thus, to answer these questions, it is important to assess total labor requirements for the ‘without-project’ situation and compare the figure to the ‘with-project’ situation. Total labor requirements for the project are compared with the total labor available so that periods of labor shortage can be identified.

Oxen power

Oxen power will be used by the kebeles for land preparation, threshing and transport of crops. In the project area the supply of oxen is rather limited although the ownership of livestock per family is relatively low.  The hiring charge for oxen is relatively high per ox-pair day for ploughing in the area. This high charge is mainly due to shortage in supply of trained oxen and the problem of oxen in the area. Moreover, the area is

subject to occasional shortage of fodder for their animals. The oxen power cost is 55 ETB pair of oxen per day included in the analysis of this project.

Fertilizers

Chemical fertilizers such as Urea and DAP are recommended to be used for the proposed crops. Very little fertilize is applied and utilized for the existing crop production.  Different study indicates that fertilizer application contributes to the yield increase at the rate of 40%. Therefore, the supply of fertilizers in required quantity and at the right time will be essential for their use.

Until now, the supply situation of fertilizers for this district and project area is reported to be unsatisfactory. The District Rural Development office will therefore, have to make necessary arrangements for the timely supply of fertilizers in adequate to the district in general and the project area in particular. The cost of fertilizer for DAP 1689 for UREA 1356 ETB per quintal is included as a component in the analysis.

Seeds (Cereals/Vegetables)

Only few improved seeds are used for the existing crop production and existing communities are utilizing local seeds. It is envisaged that the present farmers will use improved seeds under the project condition. The contribution of seed to yield will vary in accordance with the difference in quality of seed. The difference in the quality of seed is reflected in its cost. The cost of improved seeds is estimated at higher than the farm gate price of crops which means 1840 per quintals With the increased demand for improved seeds under project condition.

Chemicals

Different Chemicals are to be used to save crops from damage by pests, diseases and weeds. Different types of chemicals will be used depending on the types of pests, diseases and weeds and prices are different for different types of chemicals. Cost on account of chemical has therefore, been calculated in lump sum.

Out Put Prices

Present Situation on Marketing of Agricultural Crops in the area

Existing farmers in the area are predominantly surplus type. They economically depend on crops and livestock production. They generate a limited surplus from their production. They generally sell their surplus in the local market, of which Wayu is the most important one.

Demand and Supply

The area appears to be characterized by subsistence in supply of major food crops especially vegetables. The production of major food crops in the area is almost medium and most of the time the area experienced by shortage of major food crops or vegetables. Therefore, there is a wide in balance (gap) between demand and supply of major food crops in the project area.

Estimated  output Prices at the area

In this project analysis, in prices estimation various adjustments were tried with caution. Prices of crops thus estimated are presented in this financial analysis of (Table -1).

Agricultural Practices and Management

The quality of agricultural practices and management will have significant role in increasing the yield of crops. Agricultural practices and management are expected to contribute 40% of the increase in yield. Therefore, major costs of agricultural practices and management included in this financial cost and benefit analysis of the project.

Tools equipment and packing materials (Sacks)

The tools and packing materials for target groups generally included Hoe, spade, axe, sickles and etc. These last for a number of years. The cost on account of these items has been estimated in lump sum for crop production activities in accordance with the annual repair and maintenance requirement. Packing materials (Sacks) are needed to transport and store crops. The requirement of sacks is calculated on the basis of amount and type of harvested crops.

Yield, harvest

The basis for estimating the total income earnings from production are the harvest (= yield x area) and the unit price that farmers are likely to obtain, taking into account the season and the local market conditions. Multiplying the harvest and the estimated unit price gives the estimated gross income.

The estimated yields for Dindila aba Oromo proposed SS irrigation scheme were presented in Table 3. For horticultural crops, the marketable or saleable harvest takes into account losses that might occur since it is unlikely that the entire crop can be marketed. Losses can be due to poor harvesting methods or they can occur during storage and problems in marketing (for example not providing sufficient transport and not reaching the market at the right time). The exact rate of loss will vary depending on the type of crop and the distance to market. If a crop is highly perishable and is grown in a remote area with unreliable transport facilities, then higher losses should be anticipated compared to a crop that stores well in a scheme that is favorably located in respect of transport and market. However, the proposed irrigation scheme is found on 17 km distance far from the main Finfine – Wayu all weathered road, marketing and transportation problems will not as much be expected, except from wayu to the project site.

Gross income

The gross income is the total value of production from a proposed project. It includes sales plus value of retained produce for consumption at home and any by-products with value, such as retentions for livestock feed:

Gross income of marketed output = marketed output (quantity) x market blend or mix together price (Birr/unit quantity)

Gross income of retained output = output retained (quantity) x farm gate price (Birr/unit quantity), where farm gate price is the value the produce would have fetched if sold

Total gross income = gross income of marketed output + gross income of retained output

Variable costs

Variable costs are the costs that can be directly allocated to a particular enterprise or project in a production season, tend  to change with the size of the enterprise and the scale of production, can be avoided if management so decide, for example by not harvesting a crop the labor cost for harvesting, fuel and overtime costs are saved.

The variable costs included in our crop budget are costs for land preparation (hired labor or equipment, planting material (seed), fertilizers (both organic and inorganic), chemicals (pesticides, insecticides, herbicides), transport of inputs, casual labor for weeding, harvesting, etc, packing material, transport of outputs and marketing costs.

Household labor is not costed in this case. The gross margin for each enterprise or crop is assumed to be the return to family labor and capital.

Hired labor cost is included in the crop budget because of disparity between available household labor and required labor during the production period of the crop.

Packing material should be included in the crop budget to an extent that is deemed realistic.

As far as the transport of output is concerned, some output will be sold locally at the farm-gate, with little or no transport costs involved, and some will be sold in markets further away as local markets normally cannot absorb the total output from the project.

Gross margin or profit

The gross margin of a project is the difference between the total gross income earnings from the project and the total variable & fixed costs.

This is then the estimated gross return to the labor and capital that a farmer has invested for a unit land area of the particular crop. The gross margin or profit is usually expressed on a per hectare basis to allow for comparison of different crops.

The gross margin of different crops on the farm can be added up to come up with a whole farm margin.

9.5 Project Cost and Return

Project Cost

Investment cost

Investment refers to the initial costs of construction of the irrigation scheme. The cost items included depend on the type of system. The investment cost estimates for Dindila aba Gala proposed irrigation scheme are listed in Table 4.

Table 5 Summary of economic Initial Investment costs for Dindila  aba Oromo SSI scheme (56.5ha)

| No | Work Item | Cost (ETB) | % Cost | Comm. Share | Community (%) |
| --- | --- | --- | --- | --- | --- |
| 1 | Access road, Camping & Mobilization | 365516.10 | 6.3 | 504,458.18 | 10 |
| 2 | Head Work | 1971592.30 | 33.98 |  |  |
| 3 | Canals and other irrigation infrastructures | 2707473.47 | 46.67 |  |  |
| 4 | Total Costs | 5044581.87 | 86.9 |  |  |
| 6 | supervision cost | 290063.45 | 5% |  |  |
| 7 | VAT | 756687.28 |  |  |  |
|  | Grand total project cost | 5,801,269.15 |  |  |  |
| 6 | Cost per hectare | 102859.38 |  |  |  |

Operating costs

The operating expenditure is calculated for the costs of equipment utilized in making the investment functional and would include the ones described below.

Replacement costs

These are the costs incurred to replace specific items. In the proposed Dindila aba Oromo irrigation scheme, materials will be replaced every year from 2nd year to the end the project life. Its cost is estimated to be 10% of the main project structures (Source: Design Engineer).

Repair and maintenance costs

These costs are usually assumed to depend on the cost of the equipment utilized. Thus, for the project under study, maintenance cost of 79,168 ETB per year is proposed that means 2% of the main project structures (Designing Engineer).

Return to project

The return to project includes net return with project. However, the calculation of incremental net return involves the deduction of net return without project from the net return with project.

Contingency

There is a provision for contingency to allow for any under estimation of prices or quantity for those items which involve cash payment. The cost for contingency is estimated at the rate of 10% on all cash cost for the crop in question

Time value of money

When the costs and benefits of a project are spread over time, the problem arises of how to compare future income with present income. To be able to do this, the value of future income has to be reduced to its present worth. This is based on the principle that a dollar buys more today than it buys tomorrow.

Determining the discount rate

The factor used to reduce projected future income or to accumulate loans taken now, is the discount rate or interest. Interest can be explained as:

Time preference: Having money in your hand today is better than having it in your hand a year from now, when its value may have been eroded by inflation. Uncertainty about the future also results in preference for money today rather than in the future.

Opportunity cost of capital: If you lend out money, you give somebody else the opportunity to invest the money productively while foregoing the possibility of doing this yourself. There is an opportunity cost to lending out your capital. This should be covered by the interest charged.

In accordance with the findings of the study on Commercial Bank of Ethiopia, the opportunity cost of capital (discount rate) used is 10 % and this discount rate was taken for financial analysis of this projects.

Measuring the project worthiness

Having introduced methods to deal with the time dimension, the viability or worthiness of the project that takes the timing of costs and benefits into account can now be measured, using Net Present Value (NPV), Benefit/Cost (B/C) ratio and Internal Rate of Return (IRR).

Technique of Calculating IRR, NPV & B/C

Since difference amounts of costs are incurred and different quantities of benefits arises in different years over the life of the project, present values of both costs and benefits have to be determined in order to compare them. The process of estimating present value of costs and benefits is said to be discounting. The FIRR and NPV are determined by discounting incremental benefits (return) stream or incremental cash flow. However, B/C criterion does not require cash flow, here incremental cost and incremental benefits are discounted independently and benefit to cost ratio will be taken.

Project Period

There is no hard and fast rule about the length of the project period to be analyzed. One of the indicators for the length of the project period for which analysis has to be carried out is the technical life of the major capital assets. However, the economic life of such capital assets may be shorter because of technological obsolescence. Therefore, a period of about 20 will be taken as appropriate for this project analysis.

Opportunity Cost of Capital (Discount Rate)

An appropriate discount rate which is supposed to reflect the opportunity cost of capital to the economy should be determined. This discount rate indicates the cut of rate i.e. the financial internal rate of return below this rate for any project in general, makes it generally non-viable given other factors constants.

In accordance with the findings of the study on Commercial Bank of Ethiopia this cut of discount rate is 11% and this has been accepted for financial analysis of this project.

Net Present Value (NPV)

The NPV is defined as the present worth of the net benefit of the project. In the financial analysis, it is the present value of the net income stream accruing to the entity from whose point of view the analysis is being undertaken. Net benefits for Dindila aba Oromo irrigation project were shown in the Table 17.

To calculate the NPV, one must determine a discount rate. This should be the rate below which it will be unacceptable for the return to capital to fall. For the purposes here, this will be the cost of borrowing money. If the project cannot generate an income stream sufficient to pay back the loan plus the interest, then it is not worth undertaking.

Benefits accruing every year are multiplied by the appropriate discount factor to give the present value (PV) of the benefits. The NPV then is the sum of all the present values or cash flow for the project life.

The NPV in Table17 is positive (24,013,287) which mean that, at the chosen discount rate, the investment will be more than recovered and it will be profitable to go ahead with the project. If the NPV had been negative, it would mean that the return of the investment would not be acceptable and one would have to look elsewhere to invest money. The selection criterion is thus NPV > 0. The NPV being an absolute measure, it does not give us any idea of the relative return of capital. It only tells us that all the projects with a positive NPV are profitable to initiate.

The Benefit/Cost (B/C) ratio

The B/C ratio is the ratio between the PV of the benefit stream and the PV of the cost stream. Thus it is an indication of how much the benefits exceed the costs. For the project the B/C is greater than one (1.77%) indicating that the project is viable.

This ratio being greater than 1 shows that, at the current discount rate, the benefits exceed the costs. This means that it would be profitable to go ahead with this project. If the ratio had been below 1, the project would not be viable. The selection criterion is  B/C ratio > 1.

Internal Rate of Return (IRR)

The IRR is the rate of discount at which the total discounted cash benefits expected from the project equal the total discounted cash costs required by the investments. It is the rate that makes the NPV of the project equal to zero. The IRR can also be described as the rate of growth of an investment. This rate can be interpreted as the highest rate of interest an investor could pay, without losing money, if all the funds to finance the investment are borrowed and if the debt service (loan and accrued interest) was repaid by use of cash proceeds from the investment. The investment criterion is that the IRR should be greater than the discount rate. Accordingly, the IRR for the project is greater than the discount rate (80%) indicating that the project is viable.

Planning under uncertainty

Planning irrigation schemes involves projections of inputs and outputs over a period of time. There is thus a need to consider the element of uncertainty. What happens if the future developments are not as envisaged or predict? How can the uncertainty element be taken into consideration in the financial analysis?

Sensitivity analysis

Sensitivity analysis is done to allow planners and investors to take into consideration eventualities that cannot be predetermined or are beyond the direct control of those involved in project implementation. It is done in order to protect investment decisions from risk. For most agricultural projects four major risk areas affect the viability of projects. These are unstable prices, delay in implementation, cost overrun and unstable yields.

Process of sensitivity analysis

Sensitivity analysis involves recalculating the measures of project worthiness under new assumptions and including the results in the analysis. Sensitivity analysis for this project was carried out under the following major variables:

Increasing the cost of the project by 20% remaining other factors constant;

Decreasing the benefit of the project by 20% remaining other factors constant;

The simultaneous increases of cost by 20% and decrease of benefit by 20% maintaining other factors constant.

Accordingly all the calculations are repeated and checked for financial results for this Dindila aba Oromo SSI project. According to the results of the sensitivity analyses conducted the viability of the project is much affected by the increase of costs than the decreasing of the revenues. If both the costs increase and the revenues decrease at the same time, the feasibility of the project will be additionally affected (the IRR and B/C ratio drops from 80% to 10% 1.774to 1.01) and needs serious monitoring and follow ups.

In all cases, the project is viable based on the evaluation criterion. For the detail, see Table 18, 19 and 20.

Table: 10. 5 Sensitivity Analysis Results

| S/No | Conditions | Financial analysis results |  |  |
| --- | --- | --- | --- | --- |
|  |  | FNPV (Birr) | FIRR (%) | B/C Ratio |
| 1 | Base values | 24,013,287 | 80 | 1.77 |
| 2 | 20% increase in costs and other factors remain constant | 16,830,384 | 43 | 1.44 |
| 3 | 20% decrease in benefits and other factors remain constant | 7,661,000 | 27 | 1.13 |
| 4 | Simultaneous 20% increase in costs and 20% decrease in benefits provided that other factors remain constant | 224,706.53 | 10 | 1.01 |

Based on the evaluation criterion, the net present value (NPV) is positive; the internal rate of return (IRR) is greater than the discount factor and the benefit to cost ratio (B/C) is greater than one. However, the project becomes sensitive for the increase of costs by 20% and simultaneous increase and decrease of costs and benefits by 20% respectively. As a result, consideration must be given to those factors that have impacts on the benefit stream and as much as possible the cost of production must stay at the minimum level. Hence, with the above considerations, the project is viable and can proceed to the next step.

10. Economic Analysis

10.1  Introduction

In economic analysis, the decision maker is seen to know the total return or productivity and profitability to the whole of the society or the economy of all the resources committed to the development of project regardless of whom in the economy contributes them and regardless of who in the society receives the benefits from the proposed project. The project, which maximizes returns to capital, is given the highest rank and the analysis helps in identifying those projects, which make the greatest contribution to national income. It allows for remuneration or payment to labor and other inputs at economic prices or shadow prices which are intended to approximate true opportunity costs. This approach assumes that capital is the most important limit to the faster economic growth

10.2 Purpose of Economic Analysis

The purpose of economic analysis of a Project is to measure the real contribution, which the project will make to the national income. Since in case of a public sector project as this one, the nation as a whole invests its resources, it should have to determine the extent of contribution such an investment of resources makes to the national income in order to decide whether it is worthwhile to invest in this project or not.This helps to ensure that public investment funds are used for economically viable projects and also to ensure that a convincing economic case can be made for projects to benefit from external funding. Economic analysis is conducted to correct two important distortions. The first is whenever financial analysis does not give a true picture of economic impact of a project due to the existence of market imperfections, external effects and government controls while the second is due to the existence of externalities and linkages that are caused by a project but having no corresponding payments or compensation. The basic assumption applied for the economic analysis is that market prices do not reflect the real costs of inputs and output to the national economy and therefore must be adjusted to reflect the true value of goods and services at national level. In this regard, prices to be used in the economic analysis have to be real reflecting the true scarcity value or opportunity cost value. However, market prices which are used in the financial analysis do not often reflect such an opportunity cost value due to the existence of imperfections in the market.

Methodology for Economic analysis

All the financial accounts on costs and benefits which could be estimated for making financial analysis have to be adjusted to reflect the opportunity cost values. In order to determine appropriate prices and conversion factors for converting financial values into economic values, the little and Mirrlees method of project analysis is used in this public investment Dindila aba Oromo SSI project. The approach used for economic analysis is the same as used by international financing agencies including the World Bank (11%). The method essentially consists of expressing all the costs and benefits of a project at world shadow or efficiency prices by using conversion factors to change market price values into economic values In order to overcome the distortions. World prices are used to measure the opportunity cost of goods and services to the economy which can be actually traded in the international market. Sensitivity of the rate of return to changes in variables of cost increment and benefit reduction have to be tested to verify the soundness of the economic feasibility of the project. According to the standard and acceptable methodology, the analysis should be done in constant prices of this studying year. The economic parameters to be considered for Dindila aba Oromo economic analysis could be: - discount rate, standard conversion factor, traded goods consisting of exported (exportable) and imported (importable) and which are defined by their border prices, none traded goods which are not traded at international market. Finally In this study, economic viability of Dindila aba Oromo Irrigation Project has been made by estimating measures which includes ENPV, EIRR and B/C Ratio.

Economic Crop Budget

Economic crop budget has to be prepared for the “with” and “without” the project cases. The economic crop budgets are the bases for the preparation of farm operational economic variable costs as well as for the preparation of farm net incomes. Economic crop budget is prepared from the financial crop budget by converting financial prices of inputs and outputs by their respective conversion factors. Thus, the formats and the contents are similar with the financial crop budget and they differ only in the amount of costs, output prices and benefits

Components of Economic Crop Budgets with the project

As in the case of financial crop budgets, the preparation of economic crop budgets includes:

(1) The selection of crops to be grown

(2) Determination of optimum yield for each crop to be achieved,

(3) Yield build-up period from year 1 to the year when optimum yield is achieved

(4) Optimum level of uses of inputs at optimum yield level

(5) Pattern of uses of inputs from year 1 to the year when optimum yield is achieved and

(6)Economic prices of outputs and inputs. In case of economic crop budgets, all the components except prices of inputs and outputs are the same as in financial crop budgets. (Source:-Agronomy expert)

Components of Crop Budges ‘Without” Project.

Existing crops, cultivated land, yield and existing price of outputs and inputs multiplying by conversion factors are considered for the preparation of crop budget.

Comparison Between With and Without Project

This analysis involves the comparison of economic situation with project and without project. It is not worthy that the analysis does not seek to compare the economic situation before and after the project. In making this analysis therefore, all the relevant cost and benefit items with and without the project have been identified and valued with appropriate economic prices and incremental net return with the project have been calculated.

Net return arises because of the project and the net return also arises without the project as well in many cases. Therefore, incremental net return as a result of the project has been calculated. This is determined by deducting the net return without project from the net return with project.

Preparation of economic Crop Budgets

An economic crop budget essentially involves the change in the valuation of financial costs and returns by using appropriate opportunity cost prices and conversion factors. Suitable adjustments in the values of financial analysis have been made to determine the economic crop budgets.

10.3  Theoretical Basis of Economic valuation

Prices to be used in the economic analysis have to be real reflecting the true scarcity value or opportunity cost value. However, market prices as used in financial analysis do not often reflect such opportunity cost values because there are various imperfections in the market. In addition, there are government fixed prices such as in cases of fertilizers, pesticides, improved seeds etc. Such prices are also not expected to reflect true values. Therefore, in making economic analysis opportunity cost values (real values) of all items of cost and benefits have to be estimated.

Conceptual Framework for Determining Economic Prices

All the financial accounts on costs and benefits which have been estimated for making financial analysis have to be adjusted to reflect the opportunity cost values. In order to determine appropriate prices and conversion factors for converting financial values into economic values, the well-known UNIDO method of project analysis has been used in this case. This method essentially consists of expressing all the costs and benefits of a project at domestic prices.

World Prices and National Economic Parameters

In this project analysis conversion factors (CF) for traded and non-traded goods was used. Once such prices or conversion factors are worked out, these can be used to projects with slight and no adjustments depending on how these have been estimated. In Ethiopia a set of appropriate values and conversion factors called ‘National Economic Parameters for Ethiopia’ have already been worked out on the basis of a detailed study. These parameters have been used in valuing the cost and benefit items for this project. Thus Economic Price= Market Price * CF

Determination of Economic Values

In order of traded items, import parity prices as applicable to various inputs and outputs have to be made by (1) adjusting values for traded items (2) adjusting values for non-traded items and (3) eliminating transfer payments like taxes and subsidies.

Import Parity Prices

In case of traded items, import parity prices as applicable to various inputs and outputs estimated. The estimation of import parity prices involves the CIF price has to be adjusted on the basis of appropriate conversion factors.

Export parity Prices

In case of inputs and outputs which are exportable, export parity prices have to be calculated. The calculation of such prices involves the FOB prices at the port of export and other relevant costs from the project area up to the exporting port. These costs other than the FOB price have to been adjusted with the appropriate conversion factors.

Constant Prices

Internationally traded items involve prices in international market while non-traded items require competitive prices in the domestic market. But, in both cases prices in constant terms have been used.

Conversion factors in National Economic Parameters

In determining economic prices of inputs and outputs, the conversion factors for traded and non-traded items as estimated and recommended in the ‘National Economic Parameters for Ethiopia’ have been used. These conversion factors are as follows:

Table   1The conversion factors for traded and non-traded by items

| No | Item | Average Conversion Factor(ACF) |
| --- | --- | --- |
| 1 | Unskilled Lbour | 0.31 |
| 2 | Skilled Labour | 0.76 |
| 3 | Construction | 0.623 |
| 4 | Transport | 0.9 |
| 5 | Oxen Power | 0.9 |
| 6 | Fertilizer | 0.99 |
| 7 | pulse | 1.17 |
| 8 | Wheat | 1.28 |
| 9 | Oil seeds | 1.14 |
| 10 | Others | 0.9 |

Other conversion factors for traded and non-traded goods are available in the Table -2 of economic analysis table

Procedure of Estimating Conversion Factors for Other Non-Traded Item

In case of such items, specific conversion factor for each item has been estimated and used in the analysis of this project.

Economic Prices of Internationally Traded and non-traded Inputs

In case of inputs and outputs, which are traded and non-traded internationally, the economic prices will be estimated on the basis of import or export parity prices as the case may be. However, for the main input and output conversion factors are used and applied.

Fertilizers and Pesticides

These inputs are imported to the country. Fertilizers are imported and so the import parity prices of fertilizers and pesticides could be represented by the mentioned products.

The import Parity price at Project site of these imports could be derived by adding the

Entire cost from FOB up to the site of the project. Thus, cost of Freight, Insurance, Bank

Charges, Overhead, Transport, handling etc. from port to project site, contingencies, and VAT

are first estimated and converted in to their economic values by appropriate conversion factors

of which standard conversion factor is one. The summation figure of economic values is then divided by the summation figure of financial values to get its Conversion factors.

Seeds: Seeds are not normally imported and the seeds for the proposed crops under the

Project will be available locally and treated as non-traded items. The economic prices of seeds

are therefore estimated by adjusting the financial prices with the established SCF of the Country.

Taxes: The project pays taxes but the country doesn’t and it is a transfer of payment from project to government. Transfer payments may have impacts upon savings and income distributions. This however is not dealt with under efficiency pricing. Therefore, taxes and subsidies are transfer payments and are multiplied by a CF of 0.00.

Unskilled Labour: The unskilled labour is a non-traded item and its opportunity cost value i.e. In economic analysis of a project, the shadow price of labour or shadow wage rate is built through the opportunity cost of labour which is the marginal opportunity cost of labour. The opportunity cost of labour refers to a static cost which relates to current output lost i.e., for every worker employed in one project, the economy probably loses output in another activity or sector. The opportunity cost is the cost of a lost opportunity elsewhere in the economy. This is the economic resource cost of employing labour and may differ from the actual rate paid. The economic price of unskilled labour could be obtained through the application of its conversion factor presented in the national economic parameters.

Economic Input and Output Prices

The economic prices of variable farm operational inputs as well as crop and their by products are to be established using their established conversion factors discussed above. Accordingly, unit prices and crop budgets for the existing and proposed crops would be prepared using same methodologies. In economic terms, the amount of products remains the same as in financial terms. The amount of outputs never gets changed whereas the difference lies on valuation of outputs and input prices. As an example, the economic input and output prices for the existing and proposed farmlands of Dindila aba Oromo SSIP are given in Table 2 and Table 3 respective

Table 2    Economic Price of Inputs

| Items | Measurement |  | Price Per Unit In Birr | Item | Measurement | Price Per Unit In Birr |
| --- | --- | --- | --- | --- | --- | --- |
| Labor | MD | 21.5 |  |  | Seed |  |
| Oxen Power | OD | 49.5 |  | Teff | Qt | 855 |
| DAP Fertilizer | Qt |  | 1,511.73 | Maize | Qt | 1656 |
| UREA Fertilizer | Lit |  | 1234.53 | Mung Bean | Qt | 819 |
| Pesticides | Lit |  | 138.25 | Onion | Qt | 540 |
| Transport | Qt |  | 22.5 | Potato | Qt | 225 |
| Land Tax | Ha |  | - | Tomato | Qt | 1530 |

Table 3 Producer economic price for output

| Status | Crops | unite | Price per Unit |
| --- | --- | --- | --- |
| Main |  | seed |  |
| Proposed crops | Maize | Qt | 405 |
|  | Onion | Qt | 554 |
|  | Tomato | Qt | 450 |
|  | Potato | Qt | 225 |
|  | Pepper | Qt | 405 |
| Rain fed crops | Maize | Qt | 405 |
|  | Teff |  |  |
|  | Mung Bean | Qt | 819 |
|  | Wheat | Qt | 768 |
|  | Pea | Qt | 936 |
|  | Nouger | Qt | 1231.2 |
|  | Barley | Qt | 450 |

Summary for the Existing Economic Costs and Returns: The summary of economic costs and benefits of the existing crop budget has to be prepared. Once the basic financial inputs and outputs are converted, the preparation of the summary doesn’t require any further adjustment and would be computed automatically. Such a summary shows the extent of the net benefits that are achieved without the project case. The higher the net return, the lower would be the net benefit of the proposed project. This is one of the basic parameter that could justify the implementation of the project from the view point of the country. The summary could take various components and however, contains basic information such as gross return cost of cultivation, net return ,area cultivated and total returns from the currently farm lands. As a sample, the total economic returns from the existing farm operation of the Dindila aba Oromo SSIP.

Table---The annual Economic Benefit from Existing Crops

| No. | Type of crops | Gross Return (Birr/ha) | Cost Of Cultivation (Birr/ha) | Net Returns (Birr/ha) | Area Cultivated (Ha) with Intensity | Total Returns |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | Teff | 45360 | 27621.65 | 17,738 | 2 | 35,477 |
| 2 | Maize | 28350 | 12987.76 | 15,362 | 7 | 107,536 |
| 3 | Wheat | 32000 | 17720.25 | 14,280 | 4 | 57,119 |
| 4 | M/ Bean | 19656 | 6342.8 | 13,313 | 2 | 26,626 |
| 5 | Pea | 25272 | 8872.8 | 16,399 | 2 | 32,798 |
| 6 | Noug | 24624 | 10208.96 | 14,415 | 3 | 43,245 |
|  | Total |  |  | 91,508 |  | 302,801 |

Economic Benefits from Proposed Crops

The economic benefit and cost streams could be prepared similar with the financial analysis. Recognizing this possibility, the economic analysis could focus on the major summary items. The presentation of project product projections, area development plan, land intensity and yield build-up analysis are identical with that of the financial analysis and it would be a repetition to include them in the economic analysis. Rather, the economic analysis could include major conclusions such as cost and benefits summary computations. The economic returns from the proposed irrigated crops are obtained from the results of financial results using the established conversion factors. In economic terms, the benefits could be estimated for per ha bases and for the entire area of farm lands as whole. There would be a gradual increase in the returns mainly because of higher yields in successive years. There will also be gradual increase of irrigated area in the command. Thus, it is evident that the optimum level of economic net return reaches at their optimum in a phased manner on full realization of yield potential projection. It is possible to include the summary of annual return build-up or else the annual return at optimum level of production could be given. For demonstration purpose, the annual economic costs and returns for the sampled Dindila aba Oromo SSIP is given in Table

Table 4 the annual Economic cost and benefits for proposed crops at optimum level by two phase

| Type of crops | Gross Return (Birr/ha) | Cost Of Cultivation (Birr/ha) | Net Returns (Birr/ha) | Area Cultivated (Ha) with Intensity | Total Returns |
| --- | --- | --- | --- | --- | --- |
| Maize | 688500 | 215170.35 | 473329.65 | 17 | 8,046,604.05 |
| Onion | 2632500 | 541020.6 | 2091479.4 | 18 | 37,646,629.2 |
| Potato | 1377000 | 377394.9 | 999605.1 | 17 | 16,993,286.7 |
| pepper |  |  |  |  |  |
| Tomato | 504000 | 219783.2 | 284216.8 | 8 | 2,273,734.4 |
| Total | 5202000 | 1,353,369.05 | 3,848,630.95 | 60 | 64,960,254.4 |

Income Comparison between Existing and Proposed

The income that could be derived from the existing and the proposed cropping patterns could be compared for the purpose of evaluating the extent of achievable crop and employment benefits from the proposed project. The bases of the comparison are the figures which are contained within the crop budgets of the crops. The net benefits from each are taken by considering the extent of existing applied cropping intensities for the “without “and proposed intensity for the “with” the project cases.

While making the comparison, the assumption is that a typical household grows all the identified crops of the existing as well as the proposed crops. The comparison is made on per ha bases assuming that all the crops are grown over one ha of land with the given established cropping patterns. The net return from one ha is multiplied by the area of crop land and the summation gives the total net income from one ha of farm land. Then, the net return from the existing farm land is deducted from the proposed. If the result is negative, then the implementation of the project would result in negative net benefits to the farm operators and would be better not to implement the project. If it is zero, then the project implementation does not generate additional incomes. Besides, the inclusion of other fixed investment and operational costs makes the project non viable. In such results, it would be better to decide not to implement the project. It would be an indication to implement the project if the result is positive. I f the result is positive, the extent of the project benefit to accommodate investment and other fixed annual operating costs should be examined further before deciding for its implementation.

However, the positive results in terms of benefit and employment generation could be taken as to contribution criteria to accept or reject a given project.

Return to Project (Economic)

The economic return to project includes not return with project. The calculation of incremental net return involves the deduction of net return without project from the net return with project.

Project’s Cost and return (Economic)

Economic costs of the project and the economic returns to it were determined by converting its financial costs and returns into economic ones through appropriate prices and conversion factors. Thus, the calculation of economic costs of and returns to the project is essentially the appropriate valuation of financial costs and returns as estimated in the annex of economic analysis. The financial costs and returns will be converted into economic costs and returns on the basis of methodology and appropriate prices as stated earlier.

Project Cost (Economic)

The economic costs of the project include the costs on account of investment, operation and maintenance and replacement.

Economic Investment Cost

The total economic investment cost is composed of costs on account of irrigation and drainage works, infrastructure, equipments for project operating agency, peasant farms. Total economic investment costs are estimated at 5044581.87Birr.

Investment Cost: The cost template of the project is to be used as source for the types and magnitudes of financial costs from which the economic cost is developed. Finally, the conversion factors for the major investment components are established through the division of economic values by the corresponding financial values.

Table 5 Summary of economic Initial Investment costs for Dindila  aba Oromo SSI scheme (56.5ha)

| No | Work Item | Cost (ETB) | % Cost | Comm. Share |
| --- | --- | --- | --- | --- |
| 1 | Access road, Camping & Mobilization | 365516.10 | 6.3 | 504458.187 (10%of total main cost) |
| 2 | Head Work | 1971592.30 | 33.98 |  |
| 3 | Canals and other irrigation infrastructures | 2707473.47 | 46.67 |  |
| 4 | Total Costs | 5044581.87 | 86.9 |  |
| 6 | supervision cost | 290063.45 | 5% |  |
| 7 | VAT | 756687.28 |  |  |
|  | Grand total project cost | 5801269.15 |  |  |
| 6 | Cost per hectare | 89442.94 |  |  |

Source –Engineer expert

Operation and Maintenance Cost (Economic)

Components of operation and maintenance costs are 100891.63 Birr which is 2% of the total main investment cost.

Replacement Cost (Economic)

The structure and equipments for the operation of the project will have to be replaced periodically according to the life span of each item. Replacement Costs are also taken 10% of the major structure 504458.18.

Project Period

As stated in case of financial analysis, there is no hard and fast rule about the length of the project period to be analyzed as informed from our engineer. One of the indicators for the length of the project period for which analysis has to be carried out is the technical life of the major capital assets. However, the effect of such capital assets for which investment is made at the initial stage on the rate of return based on discounted cash flow after 20 years because it is so insignificant that it makes hardly any difference in the degree of viability of a project. Therefore, a period of about 20 years may be appropriate for the project analysis.

Discount Rate

Appropriate discount rate which is supposed to reflect the opportunity cost of capital to the economy should be determined. This discount rate indicates the cut off rate i.e. the economic internal rate of return below this rate for any projects in general, make it economically non-viable given other factors constant. In accordance with the findings of the study on the National Economic Parameters for Ethiopia, this cut off discount rate is 10% and the Government of Ethiopia has accepted this rate for economic analysis of projects.

Risk and Uncertainty

In order to allow for various uncertainties and unforeseen circumstances, sensitivity analysis has to be carried out to test the soundness of economic results. Sensitivity tests are generally carried out by changing variables like net return, output price, yield, investment cost, etc.

Measures of Economic Viability

ECONOMIC VIABILITY

In order to determine the degree to which the project is viable from the national point of view, the economic internal rate of return has to be calculated. The internal rate of return is computed from the incremental cash flow over years by deducting net return without the project and all project costs –investment, operation and maintenance, and replacement from the net returns with project.

There are three measures of project’s viability such as internal rate of return (EIRR), net present value (NPV), and benefit cost (B-C) ratio are generally used to determine the Economic viability of projects. Of these measures, the IRR is most widely used. Most of the international financial institutions including the World Bank use the IRR in making economic analysis of projects. In this study, economic IRR (EIRR) and economic NPV (ENPV) have been used in determining the economic viability of the project.

Technique of Calculating EIRR and ENPV

Since different amounts of costs are incurred and different quantities of benefits arise in different years over the life of a project, present values of both costs and benefits have to be determined in order to compare them. The process of estimating present values of costs and benefits is called discounting. The EIRR and ENPV are determined by discounting incremental benefit (return) stream or incremental cash flow.

Economic Net Present Value (ENPV)

The ENPV is defined as the present worth of the net benefits and the sum of all the present values or cash flow for the project life. In the economic analysis, it is the present value of the net income stream accruing to the entity from whose point of view the analysis is being undertaken. Net benefits for Dindila aba Oromo irrigation project were shown in the Table 17 Economic analysis.

The ENPV in Table17 is positive (38,409849) which mean that, at the chosen discount rate, the investment will be more than recovered and it will be profitable to go ahead with the project. The selection criterion is thus NPV > 0. The ENPV being an absolute measure, it does not give us any idea of the relative return of capital. It only tells us that all the projects with a positive ENPV are profitable to initiate.

Economic the Benefit/Cost (EB/C) ratio

The EB/C ratio is the ratio between the EPV of the benefit stream and the EPV of the cost stream. Thus it is an indication of how much the benefits exceed the costs. For the project the EB/C is greater than one (2,761%) indicating that the project is viable.

This ratio being greater than 1 shows that, at the current discount rate, the benefits exceed the costs. This means that it would be profitable to go ahead with this project. If the ratio had been below 1, the project would not be viable. The selection criterion is EB/C ratio > 1.

Economic Internal Rate of Return (EIRR)

The EIRR is the rate of discount at which the total discounted cash benefits expected from the project equal the total discounted cash costs required by the investments. It is the rate that makes the ENPV of the project equal to zero. The EIRR can also be described as the rate of growth of an investment. This rate can be interpreted as the highest rate of interest an investor could pay, without losing money, if all the funds to finance the investment are borrowed and if the debt service (loan and accrued interest) was repaid by use of cash proceeds from the investment. The investment criterion is that the EIRR should be greater than the discount rate. Accordingly, the EIRR for the project is greater than the discount rate (478 %) indicating that the project is extremely viable.

10.4 SENSITIVITY ANALYSIS

The future is less amenable to precise prediction. Although efforts are usually made to make estimates as realistic as possible, there always remain some uncertainties. In order to determine the soundness of the results on the economic viability of projects, allowances for such probable uncertainties have to be made. Therefore, the EIRRs have to be subjected to sensitivity tests by changing important variables. The favorable circumstances (positive changes in variables) will increase the viability results. Therefore, sensitivity tests have to be carried out by changing the important variables on the assumption of unfavorable circumstances as follows.

Delay in the Accrual of Benefits: The accrual benefit is often delayed because of failure to complete construction of project facilities in time and to undertake production activities in time. It is therefore, worthwhile to test the impact of such delays on the rate of return of the project.

Increase in the Investment Costs: Generally, there occurs over run of costs during the implementation of small to large scale irrigation projects. It is therefore necessary to test the effects of such cost increments on the viability of the project.

Decrease in Net Returns: The net return can decrease either by a fall in the yield of output or fall in the price of output or increase in production costs or a combination of all these three variables. Some uncertainties are always involved in the prediction of these variables. Sometimes, the yield and price may bay move in opposite direction and may cancel out the effects of respective movements. However, it would be quite pertinent to test the probable negative effects which may be brought about by changes in these variables.

Decrease in Price of Crops: prices of crops may fluctuate. Although crop prices which are estimated in the analysis are quite realistic, the sensitivity of the economic rate of return is still tested by decreasing the prices of crops by certain percentages in order to find out the impact of such a situation given. As an example, the results of the sensitivity tests for a 20 % decrease in the yield level and no change in costs, a 20% simultaneous increment of investment and operational costs and no change in the benefits, a 20% reduction in the price of the crops and decline of production and crop prices and simultaneous cost increment by 20% , while a test viability analysis for the Decline of Production & Prices and simultaneous cost increment by 20% is given in Annex 3 for the sample SSIP of Dindila aba Oromo.

Result of the Sensitivity Tests

The sensitivity tests of economic viability or non-viability of the project have been carried out. The results of the sensitivity tests are as follows:

Table 1- 1: Economic Analysis Results

| S.No. | Conditions | Economic Analysis Results |  |  |
| --- | --- | --- | --- | --- |
|  |  | ENPV | EIRR | E B/C Ratio |
| 1 | Base value | 38,409,849 | 2761% | 3.74 |
| 2 | Increase cost of production of the project by 20 % R.O.F.C. | 18982943 | 166% | 2.24 |
| 3 | Decrease benefit of the project by 20 % R.O.F.C. | 18393208 | 95% | 1.79 |
| 4 | Increase and decrease cost and benefit of production of the project by 20 % R.O.F.C. | 13,992,885.69 | 55% | 1.6 |

Concluding Remark

As it is observed from both financial and economic sensitivity analysis result that, the project become sensitive for production variable changes, that is increase or decrease in cost and benefit. Therefore, consideration must be given to those factors that have distress the benefit stream (Yield, area of cultivation and price of products) and in this case the cost of production must stay at the minimum level as much as possible. In order to carry out this activities close follow up and extension service must be given to the farm households in the area. Moreover the sustainable development of this irrigation scheme belongs to the organized beneficiaries of the project. It is known that the whole responsibility of the project management is laid on the beneficiaries. Therefore, the community must organize by establishing Water User Association so that the scheme can operate successfully and full fill the above mentioned objectives.

CONCLUSION & RECOMMANDATION OF THE PROJECT

It is clear that one of the major strategies to insure food self-sufficiency in the country is to induce or develop irrigation systems to where we have potential land and water resources. This includes introducing or constructing new modern irrigation schemes, upgrading the existing traditional irrigation schemes, expansions & rehabilitation (maintenance) of the existing modern schemes. Dindila aba Oromosmall scale irrigation project is intended to develop new irrigation system by means of weir diversion on Dindila aba Oromo River.

Irrigation system enables (achieves) increasing production and productivity through the following mechanisms:-By reducing dependency on rain fall for agricultural Production, By producing crops twice or more a year, By increasing land productivity, and By manipulating time for crop production, etc.

Follow up and training for the irrigation water users before and after project implementation on scheme management, operation how to operate Weir diversion and illustration of user manual/guide and maintenance and adoption of crops for irrigation especially for the first two years is necessary. This can ensure sustainable project life and make use of irrigation water profitable.

The surveyed command area is about 250 ha, but the net area to be developed under irrigation is not more than 56.5 ha. This is because the available lean discharge limited us to develop the described one. To expand the irrigation land/command area for future, construction of reservoir/night storage is desirable.

During construction, making use of locally available construction materials must be put in action so as to minimize cost of construction. The project design has considered all necessary conditions such as investment cost, community participation, utilization of local materials, community interest, etc. for feasibility of the project and its sustainability.

Generally, it makes me to say that Dindila aba Oromo small scale irrigation project is feasible financially and economically from implementation and sustainable points of view. Because of these facts it is essential to implement the project as   the expected plan and can proceed to the next step of financing for implementation.

Major Assumption

A peasant association named Bada Warke and Gudata Doby,20 and 17 km far from Wayu town respectively is taken as project area and it is assumed that the local community initiates the project idea and also farmers are using irrigation traditionally on the bank of the river.

River named Dindila aba Oromo which found beween tow kebeles. It is taken as a source of water (-----m3/sec. dis.)

The net Irrigation area or command area of 56.5 hectare assumed (average holding=0.125ha )

Total household or number of beneficiaries =480

Irrigation scheme design  life is assumed to be 20 years

Irrigation investment cost provided by an irrigation engineer5801269.15 Birr,

Opportunity cost of capital for financial cost benefit analysis 11% is taken

Opportunity cost of capital for economic  cost benefit analysis 11% is taken

Salvage value assumed to be zero

Sources of labour is from family members and hired labour

Only hired labour is taken in FCBA & total labour is taken for ECBA

Production grows very steadily in the without project situation  (annex)

2014 Price of the area assumed & constant farm gate prices are used in the analysis

For ECBA Conversion factors used i.e. Economic price = Market Price * CF

Intangible benefits are subjectively discussed and they are not valued

Optimum production will take place at the fifth year of production period.

Crop yield, land development plan and different input cost were taken assuming the project area for both with and without project case.

Crop yield, land development plan and different input cost were taken assuming the project area for both with and without project case.

Annexes

Community Petions