TABLE OF CONTENTS

1.	HEAD WORK……………………………………………………………………………………….1

1.1. General	1

1.1.1. Methodology	1

1.1.2. Irrigable area	1

1.1.3. Water Availability	1

1.1.4. Major Components of the project	1

1.1.5. Selection of Headwork site	2

1.1.6. Geology of weir site	2

1.2. Design of Headwork (weir)	2

1.2.1. Intake Design	3

1.2.2. Capacity of sluiceway	3

1.2.3. Water depth on the crest for broad crested weir	4

1.2.4. The approach Velocity	4

1.3. Stilling basin	5

1.3.1. Energy Method	5

1.3.2. Hydraulic Jump	5

1.3.3. Bed level of stilling basin:	5

1.3.4. Bernoulli’s energy equation between upstream & downstream of wire body	5

1.3.5. Froude number	5

1.3.6. Conjugate Depth	6

1.3.7. Tail water depth (d3)	6

1.3.8. Length of the stilling	6

1.3.9. Conventional method	7

1.4. Determination of Detailed parts	8

1.4.1. Bottom width	8

1.4.2. Top width	8

1.5. Thickness of the floor	8

1.5.1. Foundation material:	9

1.5.2. Length of percolation:	9

1.5.3. Blight's method	9

1.5.4. Lane method	9

1.5.5. Maximum head on the structure	11

1.5.6. Exit gradient	12

1.6. Stability Analysis of weir	13

1.6.1. Dynamic Case	13

1.6.2. Foundation Reaction	14

1.6.3. Safety factors	14

1.6.4. Static case	14

1.6.5. Safety factors	15

1.7. Design of Wing walls	15

1.7.1. Safety against overturning	16

1.7.2. Safety against sliding	16

1.7.3. Location of resultant (middle-third- rule)	16

1.8. Backwater effect	16

1.9. Intake Gate	16

1.10. Silt Gate	16

2.	IRRIGATION AND DRAINAGE………………………………………………………………..18

2.1. Design Criteria	18

2.2. Irrigation System Design	18

2.2.1. Design of Main Canal system	19

2.2.2. Design of Earthen Tertiary canals	20

2.2.3. Design of field channels	21

2.2.4. Design of furrow irrigation methods	21

2.3. Drainage Modulus	22

2.3.1. Rational method of flood determination	22

2.4. Design of Pipe Culverts	23

2.5. Design of Aqueducts	24

2.6. Design of turnout and Field off takes	25

2.7. Design of Vertical Drop (U.S.B.R.Type)	25

LIST OF TABLES

Table 1-1: Hydraulic Calculations of Sluice Capacity for 50 year flood	3

Table 1-2: Blign’s Coefficient and Lane’s Creep Ration	9

Table 1-3: Summery of downstream apron floor thickness	10

Table 1-4: Safe exit gradient against soil type	12

Table 2-1: Summary of canal dimension	21

Table 2-2: Summary of pipe culvert design	24

Table 2-3: Summary of Aqueduct	24

Table 2-4: Summary of turnout	25

Table 2-5: Summary of Vertical Drop Calculation	26

LIST OF FIGURES

Figure 1-1: Intake level of weir body	3

Figure 1-2: Water depth of broad crested weir	4

Figure 1-3: Discharge Vs Depth Curve of tail water depth	6

Figure 1-4: Energy profile of the weir	7

Figure 1-5: Weir section	10

Figure 1-6: Detail section of weir body	11

Figure 1-7: Details of guide wall	16

Figure 1-8: Section of intake and silt gates	17

Figure 2-1: Lined and Earthen canal section	20

Figure 2-2: Main drain section	23

Figure 2-3: Section of pipe culverts	23

Figure 2-4: Section of vertical drop design	25

LIST OF ANNEX

Annex 1: Bill of Quantity, Specification and Cost Estimate	28

# HEAD WORK

## General

Engineering aspects related to project study and design is addressed under this section. All influential factors that determine project sustainability are accounted to come up with reasonable results of study. In so doing, idea from different disciplines was given due attention to make the scheme smart. The project is gravity diversion system on Dalole River that delineates Bonaya boshe districts. One-way intake is suggested to serve the district. Manageable systematic layouts were adopted to minimize costs and to enable easier application of water. A total of 36 ha of land were surveyed and net irrigable area of 32 ha of land was planned for development.

### Methodology

The methodologies adopted are:

- Existing localities and community experiences and information.

- Using 1:50,000 contour maps

- Site survey and investigations

- Secondary data from concerned areas

- Hydro-meteorological data, crop data, soil data etc. collecting and organizing for this study.

- Using guidelines manuals and other reference materials

### Irrigable area

The irrigable area is moderately flat with slope ranging between 1-5% towards Dalole River. The soil of the area is silt clay soil and thick in depth. The radish clay soil is good for structures. The proposed crops under irrigation have duty of 0.53l/s/ha and net irrigable area is 32 ha.

### Water Availability

It could have been good if monthly flow data were available; but at this level of study it was not possible to have reliable data. Hence dry season flow was estimated and reasonably safer flow was taken as lean flow available for design. The lean flow as reported under hydrology section is 16.96 l/s.

### Major Components of the project

1. Diversion Head work (Broad crested weir) and temporary cofferdam

2. Lined main conveyance canal on one sides of the river 600m long, Secondary supply canals and Tertiary canals

3. Aqueducts

4. Culvers

5. Turnouts and Field off take (No)

6. Cut off drain along Main Canal (600m) and Other Drainage canals.

7. Access road clearance 5km and Camping.

9. Drops

### Selection of Headwork site

The headwork site was selected under the following factors:

- Possibility to command the anticipated area.

- Geological formation of the site.

- Relative cost of weir.

- Possibility to protect flooding of off taking canal

- Relatively area convenient for construction and less flood heights & effects.

- Access to the site

- Area simpler for temporary diversion during construction.

### Geology of weir site

The Dalole Riverbed at the diversion headwork site is formed of Rock blocks, boulders, cobbles and pebbles filled with gravel and fines. These fragmental deposits make the riverbed. A test pit (fig 3 a) dug at the central part of the riverbed at weir site revealed the dominant foundation material to be sands with gravel to about 1m depth underlain by coarse gravel with sand and clay. Trench excavation at both riverbanks showed the entire riverbanks are formed of unconsolidated silty clay soil (Illustrated in Geotechnics Section of the report Figure 4-2a) that has to be removed and provided with retaining structure to provide safety against slope failure due to saturation to be caused by the back-water effect.

Based  on  the  data  obtained  from  the  excavated  test  pit  and  trench  together  with  data  from  visual observation, thickness of the fragmental deposits making the riverbed is estimated to be more than 2mts (See Fig 4a Dalole River Valley Geological Cross Section along Weir Axis at Geotechnics Section of the report).

Both  the  upstream  and  downstream  reaches  around  the headwork  site  are  straight retch (See Fig  4b and 4c of Geotechnics Section of the report). The foundation material is in general taken as pervious foundation of loose gravel with sand and clay and it  requires  engineering  provision  of  safety  against piping,  scouring  and  uplift  effects. The bearing capacity of such material, as taken from literature, is 245KN/m2.

## Design of Headwork (weir)

Weir type: Broad crested masonry weir

Length or span of the weir overflow was taken 16m from the existing situation of the river section.

1) Determination of crest level

- Average level of the highest field = 1648.10m

- Water depth required  = 0.30m

- Head loss across the field = 0.00m

- Head loss at turnout = 0.05m

- Slope of canal x distance from weir =(0.001x210) = 0.21m

- Head loss across head regulator = 0.10m

- Crest level of the weir = 1648.76m

- Upstream river bed level =1647.32m

i.e. Crest height = crest level - upstream river bed level

= 1648.76-1647.32 = 1.44m

### Intake Design

Given: Q=16.961l/s

Formula adopted: Orifice (free outflow)

Q=CdA(2gh)……………………………………………….………………….Eq1.1

Where h=y-(D/2) and A=D2/4

Cd=0.62 for free out flow condition

=0.8 for submerge out flow

| Cd | D | y | A | h | Q |
| --- | --- | --- | --- | --- | --- |
| 0.62 | 0.263 | 0.145 | 0.054 | 0.014 | 0.017 |

Since 26 concrete pipes is not found on market and thus 30cm concrete pipe was adopted and regulate by gate. The Intake level is 1648.46m.

Figure 1-1: Intake level of weir body

### Capacity of sluiceway

The capacity of the sluice decided based on the maximum of the values of discharge obtained by twice the intake capacity or the 20% of the design discharge.

- Intake capacity:0.01696 m3/s→ Sluiceway capacity: 2 x 0.6 = 1.2 m3/s

- 50 year design flood: 18.16 →Sluiceway capacity: 0.2 x 90.80 = 18.16 m3/s

Thus the minimum sluiceway capacity has to be 18.16m3/s.

Table 1-1: Hydraulic Calculations of Sluice Capacity for 50 year flood

| Feature | Data | Formula | Calculation | Result |
| --- | --- | --- | --- | --- |
| Width of sluiceway | 0.6 m |  |  |  |
| Flood discharge | 90.80 m3/s |  |  |  |
| Capacity of sluice |  | Section 1.3 | 0.2 x 90.80 | 18.16 m3/s |
| Crest elevation of sluiceway | 1804.74m |  |  |  |
| Using the formula for a broad crested weir |  | Q = 1.7LH1.5 | 1.7x 0.8 x (2.2 + 3.42)1.5 | 18.16 m3/s |

### Water depth on the crest for broad crested weir

Q = 1.7LH3/2…………………………………………………………………..Eq1.2

Q = Design flood = 90.8m3/s (from Hydrology report)

L = Weir span = 16m

H = Total Head over crest = water ht + velocity head

Hence 90.80 = 1.7x16xH3/2

H=2.23m

Figure 1-2: Water depth of broad crested weir

### The approach Velocity

Va=QL(h+P)……………………………………………………………….Eq1.3

P was taken 0.50m after silt up

Where h=Va22g

Va22g=H-h=2.23-h

But Va=QL(h+P)=90.816(h+0.5)

90.82(16h+0.5)2*119.62=2.23-h

1.64(h+0.5)2=2.23-h

By trial & error h = 2.09m

## Stilling basin

### Energy Method

Velocity head in approach channel

hva = H-h ……………………………………………………………………Eq1.4

= 2.23 – 2.09 = 0.14m

### Hydraulic Jump

For cobbles and pebbles filled with gravel and fines foundation:

River bed elevation = 1647.32m

Downstream water level (d3) = 1647.32-1.6 =1645.72m

Weir crest level = 1648.76m

Height of crest, p = 0.50m (silt height assumed)

### Bed level of stilling basin:

First find the jump height, D2 or sequent depth.

#### First Trial:

Assuming the floor level is 0.00m lower than river bed,

Assumed EL1 = EL3-0.00 = 1647.32-0.00 = 1647.32m

H = Eo = E1

Z = EL0- Assumed EL1 ……………………………………..………Eq5

= (+ 1648.76)-(+1647.32) = 1.44m

E0 = h+ hvo+Z…………………………………………………………Eq6

= He+Z = 2.23+1.44 = 3.67m

q = Q/L……………………………………………………….………...Eq7

= 90.8/16 = 5.67m3/s/m

### Bernoulli’s energy equation between upstream & downstream of wire body

Z0+ weir ht + h + hv= Z1+d1+V12/2g…………………………..………………Eq8

0.00+1.44+2.09+0.14 = d1+V12/2g

3.67 = d1+ v12/2g

V1= q/d1 = 5.67/d1 ………………………………………………….………….Eq9

Substituting 9 in 8:

3.67 = 5.67/ V1+V12/19.62

By trial & Error V1= 7.57m/s

Hence d1 = 0.75m

### Froude number

Fr = ………………………………………………………….……….Eq10

Where:

Fr= Froude number

V1 = mean velocity of flow (m/s)

g = acceleration of gravity (m/s2)

d1 = depth of flow (m)

### Conjugate Depth

d2 = …………………………………….………..Eq11

Where:

d2 = conjugate depth (m)

d1 = upstream depth (m)

Fr = Froude number

### Tail water depth (d3)

Figure 1-3: Discharge Vs Depth Curve of tail water depth

The tail water depth d3=1.6m is less than the sequent depth, d2 and thus the jump recedes downstream for downstream floor level that the same as upstream apron level. Hence the bottom level of stilling basin will be EL.1 = 1647.32m.

### Length of the stilling

L = 4d2…………………………………………………………….……………..Eq12

= 4x2.61 = 10.43m

Figure 1-4: Energy profile of the weir

### Conventional method

Experimental formula to determine the length and depth of stilling basin

#### Length of stilling basin

L = …………………………………………………………………………..Eq13

#### Depth of stilling basin

d = ≥d ≥ F/3…………………………………………………….Eq14

Where:

L = Length of basin (Horizontal length of weir glacis + length of downstream apron floor in m)

d = depth of basin (m)

h = over flow depth (m)

F = (upstream water level + velocity head) - (downstream water level)

h = 2.09m

F = (1650.85+0.14)-(1648.92) = 2.07m

L = L1 =

d = = 1.04m it is fulfilled the criteria F/2 ≥d ≥ F/3.

Here the tail water depth is higher than the sequent depth and thus the flow gets submerged. Hence the stilling basin is so designed as to be equipped with the floor length of L = 6.24m and the depth of d= 1.03 m and the transition length of L1= 6.00m.

From economic point of view this conventional method is selected in determining the length and depth of stilling basin. The detail design of the weir body and apron is as follows:

## Determination of Detailed parts

According to Bligh’s formula, the basic section of weir body for the un submerged condition is determined as follows.

### Bottom width

L = …………………………………………………………..Eq15

### Top width

B = ……………………………………………………………..Eq16

Where:

H = height of weir = 1.44m

He = Specific energy head   = 2.09m

= Specific weight of weir body = 2.3

L = m

B =

L = 2.33m use L = 2.84 for the weir sloping surface (glacis) are 1V:1H.

## Thickness of the floor

- Upstream apron thickness for leakage proof can be taken 0.50m.

- Downstream apron thickness can be determined for static & dynamic hydraulic condition. The larger thickness will be selected.

The thickness at any point (A) is calculated from:

tA = {Hmax[1- (La/ Lc) ]+ (TwL-wLA)}x (safety factor /(m-1))……………….……….Eq17

Where:

tA = Thickness of apron at  the point A (m)

f = factor of safety = 1.30

Hmax= upstream & downstream head differential (m)

LA = Weighted creep length at the point A (m)

Lc = Weighted creep length total (m)

TwL = tail water level (m)

WLA = Water level at point A (m)

m= Specific gravity of masonry (2.3)

Notes: -At the toe of weir the term (TWL-WLA) should be reduced by 50% for dynamic case.

### Foundation material:

Foundation material is formed of Rock blocks, boulders, cobbles and pebbles filled with gravel and fines. For the sake of this design fine sand formation was taken.

### Length of percolation:

Cut off walls of upstream & downstream besides the apron floor shall be installed with good anchorage with weir body to prevent piping & limit intensity of uplift pressure.

### Blight's method

L> CH ………………………………………………………………………Eq18

Where L = total percolation distance (m) = Horizontal+ vertical

h = max head (m)

C= percolation co-efficient = 15

### Lane method

L>CH………………………………………………………………………..Eq19

Where L = percolation distance = V+H/3

-For sloping surface > 450 taken as vertical otherwise horizontal.

C = Percolation co-efficient = 7

H = max head (m)

* Adopt Lane's methods

Table 1-2: Blign’s Coefficient and Lane’s Creep Ration

The weir section will be like the following:

Figure 1-5: Weir section

Table 1-3: Summery of downstream apron floor thickness

| Point | Weighted creep length |  |  | Hmax [1-La/Lc] | TWL-WLA | (1.3)/ (2.3-1) | t | Remark |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | H | V | L |  |  |  |  |  |
| Dynamic Case-Hmax |  | 1.93 |  |  |  |  |  | Take reduced value of 1.375m instead of 2.75. |
| A | 5.34 | 6.5 | 8.28 | 0.72 | 2.03 | 1 | 2.75 |  |
| B | 9.9 | 6.5 | 9.80 | 0.50 | 0 | 1 | 0.50 |  |
| C | 15.4 | 6.5 | 11.63 | 0.23 | 0 | 1 | 0.23 |  |
| Static case-Hmax |  | 1.44 |  |  |  |  |  |  |
| A | 5.34 | 6.5 | 8.28 | 0.54 | 1.85 | 1 | 2.39 |  |
| B | 9.9 | 6.5 | 9.80 | 0.37 | 0.4 | 1 | 0.77 |  |
| C | 15.4 | 6.5 | 11.63 | 0.17 | 0.35 | 1 | 0.52 |  |

Figure 1-6: Detail section of weir body

### Maximum head on the structure

* Dynamic case= Head water- tail water = 1650.85-1648.92 = 1.93m

* Static case = weir crest -river bed =1.44m

Lc= Σlv + 1/3 Σlh ………………………………………………………………………………………………………………..…… Eq20

LC = (2+1.5 + 2.0+1.0+1.4) + 1/3 (15.9) = 7.90 + 5.30 = 13.20m

Hence thickness of each point will be:

Point A    1.38>1.375 (Dynamic case)

Point B   0.8>0.77 (static case)

Point C   0.6> 0.52 (static case)

Weighted creep ratio   = Lc/ Head on structure = 13.2/2.09 = 6.32

According to Lane's recommended ratios, this structure would be safe from piping on either clay or on fine sand. The base material of the river consists of fine sand. Therefore the length of the apron is sufficient to prevent the piping,

Lane recommends; C = 7

L > CH   → 7x1.44 = 10.08m

15.9 > 10.08   Safe!

### Exit gradient

The final exit up ward thrust should not exceed certain value for the foundation material to avoid piping; safe exit gradient for course silt is 0.17-0.14.

Khosla formula to determine exit gradient at downstream side for structure with floor length (b) and vertical cut off (d)

GE = H/d* …………………………………………………………..Eq21

Where 1= =1/2+ =b/d

GE must be less than 0.17 to 0.14 for this case

Table 1-4: Safe exit gradient against soil type

= b/d =15.9/1.4 =11.36      …………………………………..………..Eq22

= 1+  ……………………………...…….Eq23

GE=  ………………………….…..…Eq24

Recommended GE is 0.17 to 0.14 and 0.169 is fulfilled the critical values stated by Khosla thus it is safe to use 1.40 m cutoffs at downstream!

Note: - Use weep holes for guide walls, pressure relief holes for downstream apron and expansion joint for long masonry work

Use free board for guide wall of 0.50m and thus total height of wall is 1.44 + 2.09+0.50 = 4.03m at upstream. For downstream wall 1.6+0.50 = 2.10m, take 4.03m for typical stability analysis.

## Stability Analysis of weir

### Dynamic Case

Conditions:

- Up lift pressure was considered for the General weir section; pu = gwht2/2

- Downstream water pressure was considered; pd = gwh2/2

- Water wedge weight was considered for weir crest only; Ww= gwA

- Upstream water pressure was considered; Pu2 = gwh2/2

- Dead weight also considered; W1 = gcA

Where:

gw- Unit Wt of water which is 10kN/m3 and

gc - unit Wt of masonry work that  is 23kN/m3

Area in m2

h- Height that the pressure apply on it, in m

Moment is taken about the toe per meter width.

| No | Item | Force (KN) | Arm (m) | Moment (KN.m) | Sum |
| --- | --- | --- | --- | --- | --- |
| 1 | Pu1 =(w(2.312/2) x 0.8 | 31.45 | 1.29 | -40.68 | -189.57 |
| 2 | Pu2 = wh2/2 | 10.37 | 0.98 | -10.16 |  |
| 3 | pu = wht2/2 | 62.30 | 2.23 | -138.73 |  |
| 4 | pd = wh2/2 | 2.09 | 0.22 | 0.45 | 285.88 |
| 5 | Ww =  wA | 20.81 | 2.64 | 54.93 |  |
| 6 | W1 = cA | 65.18 | 2.64 | 172.09 |  |
| 7 | W2 = cA | 45.16 | 1.29 | 58.41 |  |
| 8 | Foundation reaction | 1.67(P1+p2) |  | B2(p1+2p2)/6 1.86(p1+2p2) |  |

### Foundation Reaction

1) 1.67(p1+p2) = ΣV = 68.85………………………………………………………...Eq25

2) 1.86(p1+2p2) = ΣM = 96.31………………………………………………………Eq26

p1+p2 = 41.23		p1 = 10.55 KN

p1+2p2 = 51.78		p2=30.68KN

Allowable bearing capacity of foundation material was taken 10 ton/m2 for silt formation is (100KN/m2) and thus it is safe.

### Safety factors

#### Over turning satiability

0 = Σm+/Σm- ……………………………………………………………..Eq27

= 285.88/189.57 = 1.51> 1.5 ok.

#### Sliding safety factor:

Ss = ΣH/ ΣV<f ………………………………………………………….….Eq28

Where f = 0.65 for this case

Ss = 39.73/68.85 < f

Ss = 0.58< 0.65 ok!

#### Check for tension:

x = Σm/ΣV ………………………………………………………………….Eq29

= 96.31/68.85 = 1.4m and eccentricity for no tension

e≤ B/6,……………………………………………………………………...Eq30

e = (B/2)-x ≤ B/6 that is =1.67-1.4 ≤ 3.34/6

= 0.27≤0.56 ok no tension!

### Static case

| S.N | Item | Force (KN) | Arm (m) | Moment (KN.m) | Sum |
| --- | --- | --- | --- | --- | --- |
| 1 | PU1 | 10.37 | 0.98 | -10.16 | -33.25 |
| 2 | PU | 10.37 | 2.23 | -23.09 |  |
| 3 | W1 | 65.18 | 2.64 | 172.09 | 230.50 |
| 4 | W2 | 45.16 | 1.29 | 58.41 |  |

### Safety factors

#### Over turning satiability

So = Σm+/Σm-…………………………………………………………….Eq31

= 230.5/33.25 = 6.93>1.5 ok!

#### Sliding safety factor:

Ss = ΣH/ΣV < f…………………………………………………………...Eq32

= 10.37/99.98= 0.10<0.65

#### Check for tension:

X = ΣM/ΣV………………………………………………………………..Eq33

=197.25/99.98 = 1.97

e = 0.30 < 0.56 ok no tension upstream.

## Design of Wing walls

Considerations:

- Analysis per meter span and moment about heel.

- Earth pressure at rest was considered.

- Drained silty clay back fill was considered.

- m = 23 KN/m3, w = 10KN/m3, soil = 18kN/m3

- Drained angle of internal friction was taken 280

- ps= ½ kos H2  where K0= 1-sinϕ and hence ps = 39.61KN/m

| Force (KN) | Sum of forces | Arm (m) | Moment (KNm) | Sum of moments |
| --- | --- | --- | --- | --- |
| Ps = | 39.61 | 1.34 | -53.21 | -53.21 |
| Ws1= | 27.27 | 1.56 | 42.64 | 174.11 |
| WS2= | 27.27 | 1.75 | 47.72 |  |
| Wm1= | 34.85 | 0.25 | 8.71 |  |
| Wm2= | 34.85 | 0.83 | 29.04 |  |
| Wm3= | 46.00 | 1.00 | 46.00 |  |

Figure 1-7: Details of guide wall

### Safety against overturning

174.11/53.21 = 3.27 > 1.5 ok

### Safety against sliding

39.61/170.23 = 0.23 < 0.65 OK

### Location of resultant (middle-third- rule)

X = ΣM/ΣV = 120.9/170.23 = 0.71

e = (B/2)-X  = 1-0.71 = 0.29

e ≤ B/6 = 0.33m  →0.29 < 0.33 Ok!

## Backwater effect

Due to the new barrier it is obvious that the raise in flood height will cover extra banks and this was considered in the design to protect effects by flood protection dyke and keep flood height of 2.09m above crest not to result any upstream damages.

## Intake Gate

The intake level is fixed to 1648.46m. Opening diameter by general Orifice formula for free outflow is 30 cm concrete pipe. Here the head required above center of pipe to create flow of 16.69 l/s is 0.014m. It is to be operated by spindle gate. One-way intake was proposed.

## Silt Gate

The silt gates are located adjacent to the intake gate. There is one flushing gates on intake side. They are 0.70m lower than the intake level i.e. 1647.46. It is 0.60m wide. Hence depth below crest level is 1.00m; this gate is to be operated (lifted) manually during the rainy season. The plate thickness shall be 5mm thick.

Figure 1-8: Section of intake and silt gates

# IRRIGATION AND DRAINAGE

## Design Criteria

The irrigation design project needs to be simple so that users can understand and participate in the operation and maintenance. Complex designs are avoided as much as possible. Designing cost effective structures is taken as one of the approaches in this study and design work. The irrigation system and structures are designed to use the water as efficient as possible by minimizing the losses in conveyance, distribution and application system. In this context, the criteria proposed to be adapted for the planning and design of Dalole small scale irrigation project is discussed below under various heads/sub heads.

## Irrigation System Design

The planning and layout of an irrigation system design is dictated by the area to be irrigated and the source of water supply. The data considered for designing irrigation system are:

- The water resources to be used, including source of water, flow rates/ availability and water quality

- Topography of the land surface

- The physical and chemical characteristics of the soil, including infiltration rates, soil moisture holding capacities, salinity

- Expected cropping pattern

- Economic and marketing situation in the area and the availability of services, including the availability of labor (family labor), maintenance services by the farmers themselves

Before commencement of design of entire irrigation structure, the detailed irrigation and drainage system layout were prepared. This layout contains information on field configuration, canal networks, natural drainage channels network, field drains, access roads and service roads, etc. Key dimensions for all layout components and irrigation and drainage infrastructure are determined.

Ground level profiles of canal systems are also taken and analyzed in accordance with the acceptable field layout. The detail levels are then used for the longitudinal and cross section of the canals and drains and to determine the levels of canal control and regulating structures.

Dalole main canal that carries water directly from the diversion weir is divided into two, which are one secondary canal and one tertiary canal directly off taking from the main canal. The design of the canal is concerned with the determination of the cross sectional dimension of the canal to convey the required discharge needed to meet the peak requirement of crops grown in the entire command area. The whole section of the canal is designed for adequate capacity, to provide sufficient capacity.

The main conveyance canal is designed as lined and the other distribution systems are designed as unlined section, based on the criteria such that the canal is non-silting when conveying sediment-laden water, and non-scouring when conveying silt-free water.

The canal flows were classified as steady and uniform and were designed using the Manning’s equation for open canals.

Q = A * V………………………………………………………Eq34

Where: Q = Discharge (m3/s)

A = Average of cross-section (m2)

V = Average velocity (m/s)

The velocity of flow was computed using Manning Formula as follows.

V=1n*R2/3*S12…………………………………………………….Eq35

Where: V = Average velocity (m/s)

n=Rugosity coefficient, depends on canal material roughness

R = Hydraulic mean radius (m)

S = Bed slope of canal

### Design of Main Canal system

The purpose of canal system design is to provide water, adequate in quantity and command to all farms within the area serviced. The system should be convenient to operate and should blend with the pattern of farming. Canals are normally designed to properly distribute water to a system that will make irrigation easy and efficient.

The conveyance and distribution systems include the main and tertiary canals that distribute water to the field canals. The main and tertiary canals will have a constant volume of water in each reach. Flows in the field canals are distributed among farmers according to the size of land holding on a time sharing basis i.e. a farmer will take the full field flow for a scheduled period of time. This allocation is managed by the farmers themselves, through their Water Users’ Association.

It is designed to supply the irrigation water from the diversion weir. The detail hydraulic computation is shown in Table 2-1. The longitudinal profile and cross section of the main canal is shown on Figure 2-1.The values of the design parameters of the typical main canal are shown in Table 2-1.

Flow = 16.96 l/s

Bed slope of canal = 0.001

Velocity in earth canal for clayey soil = 0.60-0.70m/s,

Q = A/n R 3/2 s ½…………………………………………………..……….Eq36		n = 0.025 for earth

n = 0.014 for lined section

- Economical lined Rectangular section criteria;

B = 2y………………………………………………………………………….Eq37			R = y/2………………………………………………………………………..Eq38

Where

B = bed width of canal

y = water depth

R = Hydraulic radius

y = 0.13m    B= 0.26m    V =0.58m/s    F=0.25m    D= 0.38m

- Economical Earthen Trapezoidal section criteria:

B = 0.6y, m=1.5for canal inner side slope

R = y/2

A= y(b+my)  ……………………………………………………………..Eq39

P=b+2ym2+1…………………………………………….……………Eq40

y=0.12	B=0.2     V= 0.48m/s	F=0.25 	D= 0.37    w=0.50m

Figure 2-1: Lined and Earthen canal section

### Design of Earthen Tertiary canals

The water budget for the condition that continuous flow in main, secondary and tertiary canal and rotated flow in field canals for duty of 0.53l/s/ha it could be seen that the flow that is required for each area when multiplied by the duty is less than the stream size required to irrigate 1ha of land per day in 10 hr irrigation. Unit flow or stream size q:

qt =DA ……………………………………………..……………………….Eq41

Where q= unit flow (m3/s)

t= irrigation time per day (10 hr)

D= depth of application (168mm or 0.168m for this case)

A= Area to be irrigated (1ha)

Hence q=0.168m*(100mx100m)/(10*60*60)=0.047m3/s=47l/s

It is this stream size that was used to design each tertiary canal as Duty times Area is less than 47l/s for each tertiary canal in the system layout plan.

Q=16.96l/s      n = 0.025	        m=1.5

The condition B = 0.6y provides non practical Bed width & thus use B = 0.20 and determine y:

For slope 0.003   v=0.32m/s     B=0.15m   y=0.07m     F=0.25   D=0.32m	w=0.40m

For slope 0.002   v=0.24m/s     B=0.15m     y=0.06m   F=0.25    D=0.31m w=0.40m

Here though the flow velocity is slightly smaller, it was accepted & silt clearance may be frequent.

| Table 2-1: Summary of canal dimension |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Canals |  | Q(m3/s) | V(m/s) | b(m) | y(m) | F | D(m) | m | slope | w(m) |
| Lined MC |  | 0.017 | 0.78 | 0.24 | 0.12 | 0.25 | 0.37 | 1 | 0.002 |  |
| Earthen MC |  | 0.017 | 0.46 | 0.20 | 0.11 | 0.25 | 0.36 | 1.5 | 0.002 | 0.4 |
| SC-1 |  | 0.007 | 0.340 | 0.150 | 0.100 | 0.25 | 0.35 | 1.5 | 0.002 | 0.4 |
| TC-1 |  | 0.006 | 0.323 | 0.150 | 0.070 | 0.25 | 0.32 | 1.5 | 0.003 | 0.4 |
| TC-2 |  | 0.003 | 0.236 | 0.150 | 0.06 | 0.25 | 0.31 | 1.5 | 0.002 | 0.4 |

### Design of field channels

Field channel is designed as an open channel aligned along the natural slope. The bed width, water depth and side slopes of 1:5 Vertical: Horizontal of the field channel designed is to be used for distributing the required discharge is varied. The channel is designed using Manning’s equation. The Typical longitudinal section is shown on drawing.

### Design of furrow irrigation methods

#### 2.2.4.1 Selection of irrigation methods

The selection of irrigation method/system is largely influenced by the topographic nature of the command area, soil and crop type that is planned to be grown. Farmers experience and level of technology being used in ploughsystem are also other important factors, which have got considerable impact on the selection of irrigation methods.

The morphological feature of the area is characterized by undulating valley bottom flanked by valley slopes 1 to 5% slopes and it is suitable for contour furrow. Furrow method of application is quite suitable for row crops like maize, potatoes, and vegetables and adaptable to most soils of different textures. The actual irrigation operation will be carried out according to irrigation schedule to be worked out on the basis of the stage of crop development, prevailing climatic data and water management practices to be adopted during operation of the system.

In this project, with the help of development agents supported with necessary trainings, the farmers are supposed to construct the contour furrow on their own farm lands.

## Drainage Modulus

### Rational method of flood determination

Q = 2.78 CiA……………………………………………………………..Eq42

Where

C = 0.3, Constant that depend on catchment character,

i = Intensity of rainfall for time of concentration, in cm/hr

A = Area of catchment in km2 = 5ha=0.05km2

Time of Concentration tc

tc = 1/3000  ………………………………………………..Eq43

Where

L= Length of remotes point in watercourse: 1400m

S= Slope of the water course for well managed irrigation farm = 0.002

tc = 0.96hr

Intensity of rainfall, i for time of concentration tc is then,

i = R50/tc…………………………………………………………………Eq44

= 89.86mm/0.96 hr= 93.12mm/hr = 9.312cm/hr

Q = 2.78CiA

Q = 2.78x0.30x9.312x0.055

Q =0.43m3/s for 5.5ha area and thus 21.36l/s/ha for 1 ha area.

As stated earlier the runoff expected from design storm results in 21.36l/s/ha. For the major cut off drain along the main canal maximum area draining towards the conveyance canal is 5.5ha between outlet points, which result in 427.16l/s flow to be intercepted at various points on the canal route and natural drains. Thus it was designed for this flow manning roughness co-efficient of 0.03 was adopted. And as per the IDD manual in design of drainage canals the following standard bed widths are to be adopted for the corresponding flows:

B=0.40m for Q<100l/s

B=0.50m for 100<Q<150l/s

B=0.60m for 150<Q<200l/s

B=0,80m for Q>200l/s

Hence for the main drain or cutoff drain the design parameters are as follows:

Q = 0.43m3/s   m =1.5       S = 0.002   n = 0.03       B = 0.80m

Q = A/n R 2/3 S 1/2…………………………………………………………….………Eq45

Thus for B = 0.80

| B | Y | n | A | p | R | S | Q | V | F | D |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0.80 | 0.70 | 0.03 | 1.30 | 15.59 | 0.083 | 0.002 | 0.43 | 0.33 | 0.2 | 0.90 |

Figure 2-2: Main drain section

Majority of the area drain to natural drainage routes and thus it was not essential to consider flow hydraulics of these natural water courses, but it is obvious that the local community has take care of these drainage courses not to develop gullies.

## Design of Pipe Culverts

Adopt velocity in pipe greater than 1m/s design under partial flow condition each culvert is designed to pass supply flow designed for each canal.

Figure 2-3: Section of pipe culverts

H= 0.8D

For table for H/r = 1.60 = A/r2 = 2.694 p/r = 4.428 = R/r = 0.608

For D = 0.10  A = 0.006735	P = 0.2214	R = 0.0304

For D = 0.20   A = 0.02694	          P= 0.4428	R = 0.0608

For D = 0.25   A = 0.0420		P = 0.5535	R = 0.0759

For D = 0.30   A = 0.0606		P = 0.6642	R = 0.0912

For D = 0.40   A = 0.1078		P = 0.8856	R = 0.1216

For D = 0.50   A = 0.1684		P =1.107	R = 0.1520

For D = 0.60   A = 0.2425		P = 1.328	R = 0.1824

For D = 0.80   A = 0.4310		P = 1.1771	R = 0.2432

For D = 1.00   A = 0.6735		P = 2.214	R = 0.3040

Downstream water level = u/s canal water level-Hydraulic losses

Hydraulic loss=1.2 (V22-V12)/2g + friction loss………………………………………..Eq67

Friction loss for canal bed slope, I =IL………………………………………………….Eq68

Table 2-2: Summary of pipe culvert design

| For table for H/r = 1.60 = A/r2 = 2.694 p/r = 4.428 = R/r = 0.608 |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Item | Flow (l/s) | Pipe Diameter (m) | Flow Area (m2) | V2 (m/s) | V1 (m/S) | Span, L | I | 1.2(V22-V12)/2g | IxL | Total Hydraulic loss | Level difference adopted (m) |
| Culvert (Typical) | 0.017 | 0.19 | 0.024 | 0.7 | 0.64 | 6 | 0.002 | 0.0048 | 0.012 | 0.017 | 0.1 |
| Culvert (Typical) | 0.006 | 0.12 | 0.010 | 0.6 | 0.55 | 4 | 0.005 | 0.0049 | 0.02 | 0.025 | 0.1 |

## Design of Aqueducts

Here since the pipe length is too short, total hydraulic loss is negligible, and hence it was designed to pass, the maximum drainage based on the drainage module previously stated.

The orifices formula governs the flow through the pipe under.

Q=CA (2gh) ……………………………………-………………………….Eq69

Where C=1.1+0.026L/(D) 1.2 -0.50 for concrete pipe of beveled lip entrance

h= level difference between upstream and downstream water surface.

A=0.1964 for diameter=0.50m

A=0.2827 for diameter=0.60m

A=0.5027 for diameter=0.80m

Table 2-3: Summary of Aqueduct Particular; Canal Above; Canal below; Drainage flow (m3/s); Pipe size(d); Maximum length L (m); C; Min. h(m); Area; Flow for head h; Remark. Aqueduct (Typical); MC; MD; 0.43; 0.44; 1.1; 0.92; 0.48; 0.15; 0.427; Canal above is trapezoidal.

## Design of turnout and Field off takes

The flow in turnouts and field off takes is governed by the orifice formula. It basically depends on the pipe size and head creating flow.

Q=CdA(2gh)…………………………………………………………………Eq70

Where h=y-(D/2), Creating flow in meter

A=D2/4, area of pipe

Cd -Coefficient of discharge,

0.80 - submerged out flow and

0.62 - free out flow,

g- Acceleration due to gravity 9.81 m/s

Each turn out is to be controlled with simple shutters with chain to lift to required level:

Table 2-4: Summary of turnout

| Turnout | On canal | Q (m3/s) | Y(m) | (4Q/3.14C)2/ 19.62 | Trial value of D (m) | D4(y - 0.5D) | Diameter Adopted (cm) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| TO(Typical) | Mc | 0.017 | 0.13 | 0.00061 | 0.25 | 0.000020 | 25 controlled |
| FOT (Typical) | Sc | 0.01 | 0.09 | 0.00021 | 0.1 | 0.000004 | 15 controlled |
| FOT (Typical) | Tc | 0.006 | 0.087 | 0.00008 | 0.1 | 0.000004 | 15 controlled |

## Design of Vertical Drop (U.S.B.R.Type)

Figure 2-4: Section of vertical drop design

Input Parameters:

- Discharge - Q

- Upstream water depth - h1

- Upstream water velocity - v1

- Downstream water depth - h2

- Downstream water velocity - v2

- Drop - ht z

Drop width: bc=0.734 Q/(h1)1.5

Unit discharge=q=Q/bc

Critical depth: hc=(q2/g)1/3

Stilling basin:

Basin width:  B=18.46(Q)½/(Q+9.91)

Basin length: L=[2.5+1.1(hc/z)+(hc/z)3](z*hc)0.5

Lip height:  a =hc/2

U/s Transition Length:  L = B + 3/2Q1/2

Dimensions that are not convenient during construction were changed reasonably.

Table 2-5: Summary of Vertical Drop Calculation

| Drop on Canal | Q | Drop ht | Drop width(bc) | Basin width(B) | q | dc | Basin length(L) | Lip ht | U/s Transition |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| MC | 0.017 | 1.20 | 0.27 | 0.24 | 0.06 | 0.07 | 0.77 | 0.04 | 0.44 |
|  |  | 1.00 | 0.27 | 0.24 | 0.06 | 0.07 | 0.71 | 0.04 | 0.44 |
|  |  | 0.80 | 0.27 | 0.24 | 0.06 | 0.07 | 0.64 | 0.04 | 0.44 |
|  |  | 0.50 | 0.27 | 0.24 | 0.06 | 0.07 | 0.52 | 0.04 | 0.44 |
| SC | 0.01 | 1.00 | 0.16 | 0.19 | 0.06 | 0.07 | 0.71 | 0.04 | 0.34 |
|  |  | 0.80 | 0.16 | 0.19 | 0.06 | 0.07 | 0.64 | 0.04 | 0.34 |
|  |  | 0.60 | 0.16 | 0.19 | 0.06 | 0.07 | 0.56 | 0.04 | 0.34 |
|  |  | 0.50 | 0.16 | 0.19 | 0.06 | 0.07 | 0.52 | 0.04 | 0.34 |
| TC | 0.006 | 1.00 | 0.16 | 0.19 | 0.06 | 0.07 | 0.71 | 0.04 | 0.34 |
|  |  | 0.80 | 0.16 | 0.19 | 0.06 | 0.07 | 0.64 | 0.04 | 0.34 |
|  |  | 0.60 | 0.16 | 0.19 | 0.06 | 0.07 | 0.56 | 0.04 | 0.34 |
|  |  | 0.50 | 0.16 | 0.19 | 0.06 | 0.07 | 0.52 | 0.04 | 0.34 |

ANNEX

BILL OF QUANTITIY, SPECIFICATION

AND COST ESIMATE

Annex 1: Bill of Quantity, Specification and Cost Estimate

| BILL OF QUANTITY, SPECIFCATION AND COST ESTIMATE |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Project - Dalole SSIP |  |  |  |  |  |
| No | Item | Unit | Qty | Unit Rate | Amount |
| 1 | Access road clearance ordinary soil | Km | 5.00 | 6,000.00 | 30,000.00 |
| 2 | Mobilization and Demobilization | Ls | 1.00 | 10,000.00 | 10,000.00 |
| 3 | Camping |  |  |  |  |
|  | A. Living rooms 4(3mx3m) |  |  |  |  |
|  | B. Store 1(5mx5m) |  |  |  |  |
|  | C.Kitchen1 (3mx3m) |  |  |  |  |
|  | D. Guard house1 (2.5x 2m) |  |  |  |  |
|  | E. Toilet house1 (2.5x 2m) |  |  |  |  |
|  | F.Office1 (3x3) |  |  |  |  |
|  | G. Fencing |  |  |  |  |
| 3.1 | Site clearance | m2 | 132.72 | 35.10 | 4,658.47 |
| 3.2 | Trench and other excavations | m3 | 38.00 | 128.70 | 4,890.60 |
| 3.3 | Stone masonry | m3 | 32.28 | 1,650.53 | 53,279.15 |
| 3.4 | 20 cm hard core | m3 | 49.05 | 1,359.82 | 66,702.61 |
| 3.5 | 10 cm lean concrete (1:2:4) mix | m3 | 24.53 | 2,442.51 | 59,905.65 |
| 3.6 | CIS walling (G-32) | m2 | 249.20 | 324.00 | 80,740.80 |
| 3.7 | CIS roofing (G-32) | m2 | 146.40 | 324.00 | 47,433.60 |
| 3.8 | Fencing | m | 160.00 | 80.00 | 12,800.00 |
|  | Total Carried To Summary |  |  |  | 370,410.89 |
| 4 | Headwork (16m span Masonry Broad Crested Weir) |  |  |  |  |
| 4.1 | Site clearance | m2 | 418.00 | 35.10 | 14,671.80 |
| 4.2 | Coffer dam for temporary flow diversion with sand filled bags(dry time flow is estimated to 30 l/s and river span about 10m | m3 | 36.00 | 156.00 | 5,616.00 |
| 4.3 | Excavation for temporary diversion channel |  |  |  | - |
| 4.3.1 | Soft soil formation | m3 | 30.00 | 128.70 | 3,861.00 |
| 4.4 | Excavation for foundations |  |  |  | - |
| 4.4.1 | Soft soil formation | m3 | 569.29 | 128.70 | 73,268.23 |
| 4.4.2 | Rocky soil (Soil with cobbles) | m3 | 55.83 | 255.00 | 14,235.94 |
| 4.5 | Compacted rock fragments on loose foundation to avoid differential settlements | m3 | 25.38 | 117.00 | 2,968.99 |
| 4.6 | Lean concrete works for foundation (1:2:4 mix) | m3 | 329.89 | 2,442.51 | 805,755.33 |
| 4.7 | Lean concrete for Brest wall and Gate operating slab (1:2:4) mix | m3 | 3.96 | 2,442.51 | 9,669.42 |

| No | Item | Unit | Qty | Unit Rate | Amount |
| --- | --- | --- | --- | --- | --- |
| 4.8 | Masonry bedded in 1:3 mortar | m3 | 216.26 | 1,650.53 | 356,943.50 |
| 4.9 | Plastering in 1:3 mortar | m2 | 237.10 | 119.00 | 28,214.58 |
| 4.10 | Manually lifted silt gate of size 60x50cm and 5mm thick plate in 60x60 mm angle iron slot well reinforced as per drawing, fixed in position | Pcs | 1 | 3,000.00 | 3,000.00 |
| 4.11 | Intake gate operated by spindle and shaft with square thread, plate size is 600x500x5mm as per drawing and specifications, fixed in position | Ls | 1 | 4,500.00 | 4,500.00 |
| 4.12 | Dia 30 cm concrete pipe supply and fix in position | Pcs | 6 | 2,500.00 | 15,000.00 |
| 4.13 | Fill and compactions with red clay | m3 | 242.915 | 156.00 | 37,894.71 |
| 4.14 | Rip raping | m3 | 15.00 | 252.17 | 3,782.48 |
|  | Total Carried To Summary |  |  |  | 1,379,381.97 |
| 5 | Ford Structures for River crossing (1 No) |  |  |  |  |
| 5.1 | Excavation for on ordinary soil | m3 | 82.26 | 128.70 | 10,586.86 |
| 5.2 | Lean concrete with 1:2:4 ratio | m3 | 9.36 | 1,650.53 | 15,448.97 |
| 5.3 | RCC in 1:2:4 mix as per reinforcement detail of drawing | m3 | 27.00 | 4,749.84 | 128,245.71 |
| 5.4 | Red clay back fill and compactions | m3 | 0.64 | 156.00 | 99.84 |
|  | Total Carried To Summary |  |  |  | 154,381.38 |
| 6 | Lined Conveyance Canal(MC)-625m long |  |  |  |  |
| 6.1 | Soil Excavations |  |  |  |  |
| 6.1.1 | Soil excavation | m3 | 612.50 | 128.70 | 78,828.75 |
| 6.2 | Hard core well blinded with 1:3 mortar | m3 | 171.875 | 1,359.82 | 233,719.21 |
| 6.3 | Masonry bedded in 1:3 mortar | m3 | 200 | 1,650.53 | 330,106.28 |
| 6.4 | Lean concrete with 1:2:4 ratio | m3 | 18.75 | 2,442.51 | 45,797.10 |
| 6.5 | Plastering in 1:3 mortar | m2 | 875 | 119.00 | 104,122.07 |
| 6.6 | Fill and compactions with red clay | m3 | 243.75 | 156.00 | 38,025.00 |
|  | Total Carried To Summary |  |  |  | 830,598.40 |
| 7 | Secondary canals (790m) |  |  |  |  |
| 7.1 | Soil Excavation | m3 | 165.9 | 128.70 | 21,351.33 |
| 7.2 | Fill and compaction | m3 | 47.4 | 156.00 | 7,394.40 |
|  | Total Carried To Summary |  |  |  | 28,745.73 |
| 8 | Tertiary canals (1937m) |  |  |  |  |
| 8.1 | Soil Excavation | m3 | 364.35 | 128.70 | 46,891.81 |
| 8.2 | Fill and compaction | m3 | 154.96 | 156.00 | 24,173.76 |
|  | Total Carried To Summary |  |  |  | 71,065.57 |
| 9 | Drainage canals |  |  |  |  |
| 9.1 | Main Drain (cutoff drain) excavation (625m) | m3 | 375 | 128.70 | 48,262.50 |
| 9.2 | Tertiary drain excavation (1500m) | m3 | 360 | 128.70 | 46,332.00 |
| 9.3 | Fill and compaction with red clay | m3 | 146 | 156.00 | 22,776.00 |
|  | Total Carried To Summary |  |  |  | 117,370.50 |

| No | Item | Unit | Qty | Unit Rate | Amount |
| --- | --- | --- | --- | --- | --- |
| 10 | Drop structure(37No) |  |  |  |  |
| 10.1 | Excavation on ordinary soil | m3 | 155.74 | 128.70 | 20,044.30 |
| 10.2 | Hard core well blinded with 1:3 mortar | m3 | 46.49 | 1,359.82 | 63,219.24 |
| 10.3 | Masonry bedded in 1:3mortar | m3 | 39.85 | 1,650.53 | 65,772.53 |
| 10.4 | Plastering in 1:3 mortar | m2 | 132.83 | 119.00 | 15,806.45 |
| 10.5 | Lean concrete works(1:2:4 mix) | m3 | 0.40 | 2,442.51 | 978.92 |
| 10.6 | Fill and compaction with red clay | m3 | 3.65 | 156.00 | 569.85 |
|  | Total Carried To Summary |  |  |  | 166,391.28 |
| 11 | Turnout (4No) |  |  |  |  |
| 11.1 | Excavation on ordinary soil | m3 | 8.40 | 128.70 | 1,081.08 |
| 11.2 | Hard core well blinded with 1:3 mortar | m3 | 5.61 | 1,359.82 | 7,630.99 |
| 11.3 | Masonry bedded in 1:3 mortar | m3 | 5.46 | 1,650.53 | 9,009.96 |
| 11.4 | Plastering in 1:3 mortar | m2 | 15.39 | 119.00 | 1,831.15 |
| 11.5 | Dia 15 cm concrete pipe supply and fix in position | Pcs | 4.00 | 2,000.00 | 8,000.00 |
| 11.6 | Gates of size 450x300x3mm in 40x40 mm angle iron slot(frame) and 1m long chain to control flow rates as per drawing | pcs | 4.00 | 2,500.00 | 10,000.00 |
| 11.7 | Red clay fill and compaction | m3 | 5.99 | 156.00 | 934.16 |
|  | Total Carried To Summary |  |  |  | 38,487.35 |
| 12 | Field offtakes (22in No) |  |  |  |  |
| 12.1 | Soil Excavation | m3 | 13.15 | 128.70 | 1,692.79 |
| 12.2 | Hard core well blinded with 1:3 mortar | m3 | 4.22 | 1,359.82 | 5,734.12 |
| 12.3 | Masonry bedded in 1:3mortar | m3 | 2.16 | 1,650.53 | 3,571.13 |
| 12.4 | Plastering in 1:2 mortar | m2 | 22.60 | 119.00 | 2,689.05 |
| 12.5 | Supply and install Dia 15 cm concrete pipes | pcs | 10.00 | 500.00 | 5,000.00 |
| 12.6 | Steel Shutters(gates) as per drawing | lps | 10.00 | 800.00 | 8,000.00 |
| 12.7 | Red clay Fill and compaction | m3 | 1.20 | 156.00 | 187.94 |
|  | Total Carried To Summary |  |  |  | 26,875.03 |
| 13 | Aquiducts(1No) |  |  |  |  |
| 13.1 | Excavation on ordinary soil | m3 | 14.16 | 128.70 | 1,822.39 |
| 13.2 | Hard core well blinded with 1:4 mortar | m3 | 5.71 | 1,359.82 | 7,770.40 |
| 13.3 | Masonry bedded in 1:3 mortar | m3 | 0.25 | 1,650.53 | 412.63 |
| 13.4 | Lean concrete works(1:3:6 mix) |  | 4.90 |  | - |
| 13.5 | Plastering in 1:3 mortar | m2 | 19.32 | 119.00 | 2,299.02 |
| 13.6 | Concrete pipe of 50 cm diameter supply and install as per drawing | Pcs | 1.00 | 1,500.00 | 1,500.00 |
| 13.7 | Red clay fill and compaction | m3 | 6.24 | 156.00 | 973.89 |
|  | Total Carried To Summary |  |  |  | 14,778.33 |

| No | Item | Unit | Qty | Unit Rate | Amount |
| --- | --- | --- | --- | --- | --- |
| 14 | Pipe Culverts (5No) |  |  |  |  |
| 14.1 | Excavation on ordinary soil | m3 | 32.62 | 128.70 | 4,198.48 |
| 14.2 | Hard core well blinded with 1:4 mortar | m3 | 14.29 | 1,359.82 | 19,426.01 |
| 14.3 | Lean concrete works(1:2:4 mix) | m3 | 0.63 | 2,442.51 | 1,526.57 |
| 14.4 | Masonry bedded in 1:3 mortar | m3 | 12.25 | 1,650.53 | 20,219.01 |
| 14.5 | Plastering in 1:3 mortar | m2 | 48.30 | 119.00 | 5,747.54 |
| 14.6 | Concrete pipe of 15 cm diameter supply and install | Pcs | 1.67 | 1,000.00 | 1,666.67 |
| 14.7 | Concrete pipe of 20 cm diameter supply and install | Pcs | 3.33 | 1,200.00 | 4,000.00 |
| 14.8 | Selected fill and compaction | m3 | 15.61 | 156.00 | 2,434.71 |
|  | Total Carried To Summary |  |  |  | 59,218.99 |
|  | Total Project Cost |  |  |  | 3,257,705.43 |
|  | VAT (15%) |  |  |  | 488,655.81 |
|  | Grand total |  |  |  | 3,746,361.24 |