oROMIA IRRIGATION DEVELOPMENT AUTHORITY

UKKE SMALL SCALE IRRIGATION PROJECT FINAL FEASIBILITY STUDY AND DESIGN REPORT

PREPARED BY BONEYA PLC Reviewed By OIDA-AGP Team October, 2016 Batu

TABLE OF CONTENTS

1.1. Headwork	4

1.1.2. River Hydraulics	4

1.1.4. Detail Part	8

1.1.5. Stability Analysis of the Weir	14

1.6 Stability of Retaining /Wing Wall	19

1.2. Irrigation System Design and Layout	20

1.2. 1. Design of Main Canal	21

1.2.2. Design of Earthen Secondary Canal, SC1, SC2&SC3	22

1.2.3 Flume	23

1.2.1. Flume, F1	23

1.3. CHUTES	30

1.4. Division Boxes	33

1.4.1. Hydraulic Design	33

1.5. Turn Outs	35

1.6. Culverts	39

1.6.1. Road Crossing Box Culvert	39

3. Bill of Quantities and specifications for the construction of Ukke SSIP	47

LIST OF TABLES

Table 1: Discharge Calculations	6

Table 2: Water Depth Calculation on Crest using trial and error method	8

Table 3: Stability Analysis for High Flood Condition	16

Table 4: Stability analysis for no Over Flow Condition	18

Table 5:  Flumes Parameters	23

Table 6: Division Box Hydraulics	33

Table 7:  Dimensions for Turn Outs	35

Table 8: Typical Canal Elevations at Each Section	41

LIST OF FIGURES

Figure 1: Stage Discharge Curve	6

Figure 2: Depth versus Discharge graph	7

Figure 3: Longitudinal Cross Section of Head work	14

Figure 4: Typical Canal Cross Section	31

Figure 5: Schematic Layout of Considered/Sample Division Box	34

1. ENGINEERING & HYDRAULIC STRUCTURES

# 1.1. Headwork

Purpose: To raises the water surface level on its upstream side in order to create the head necessary and divert the water through the canal intake.

Location-Condition to the topography of the area, the headwork site is located in mountainous and boulder stage of the river.

1.1.1.Headwork Site Condition

a. Geology of the Site

Based upon surface and subsurface exploration of the weir site superficially the riverbed is consisted by gravel and pebble to boulder size. Its permeability is high and the sound foundation basaltic rock is expected at a depth of 1.00m to 1.50m from the riverbed.

b. Weir shape

The weir shape designed determined based on the geological and pedalogical investigation of the headwork site and the selected shape is a broad crested weir type.

c. Design Parameters

The design parameters, which considered through the design are:

- Discharge coefficient C=1.70

- Coefficient of friction = (0.65~0.75)

- Specific weight of water w=1 ton/m3

- Angle of internal friction =16o

- Specific weight of masonry (2.0~2.3) ton/m3

- Specific weight of Concrete (2.4) ton/m3

# 1.1.2. River Hydraulics

a. Average River Bed Slope

According to the river longitudinal profile survey data, the bed slope up and downstream of weir axis calculated as follows:

| b. longitudinal |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Station (m) | Partial distance (m) | Elevation (m) | Accumulative Height (m) | Area (m2) | Remark |
| 0 | 0 | 1462.80 | 0.00 | 0.000 |  |
| 22 | 22 | 1463.21 | 0.41 | 2.075 |  |
| 47 | 25 | 1463.57 | 0.78 | 5.960 |  |
| 72 | 25 | 1463.57 | 0.78 | 7.770 | Weir Axis |
| 84 | 12 | 1464.49 | 1.69 | 12.335 |  |
| 94 | 10 | 1464.69 | 1.90 | 17.930 |  |
| 108 | 14 | 1465.63 | 2.83 | 23.630 |  |
| 122 | 14 | 1465.63 | 2.83 | 28.310 |  |
| 172 | 50 | 1465.54 | 2.74 | 27.865 |  |
| 197 | 25 | 1466.03 | 3.23 | 29.875 |  |
| 222 | 25 | 1466.37 | 3.57 | 34.025 |  |
|  | 222 |  |  | 189.775 |  |
| Have= | 2*A/L= | 1.710 |  |  |  |
| Iave= | Have/L= | 0.0077 |  |  |  |

C. River X-Section

The X-section at proposed weir axis:

Figure 1: Stage Discharge Curve

c. Stage Discharge Curve

River discharge is calculated based on Manning’s formula, Q = AV

Where V=1/n I1/2 R2/3

Table 1: Discharge Calculations

| Iave | 0.0077 |  |  | n | 0.035 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Elevation | Depth | Water Area | Wetted Perimeter | Hydraulic Radius | Velocity | Discharge | Remark |
| El(m) | D(m) | A(m2) | P(m) | R(m) | V(m/s) | Q(m3) |  |
| 1463.572 | 0.00 | 0.00 | 0.00 | 0.000 | 0.00 | 0.00 | River Bed |
| 1464.351 | 0.78 | 3.815 | 9.89 | 0.386 | 1.33 | 5 |  |
| 1464.841 | 1.27 | 13.374 | 22.38 | 0.598 | 1.78 | 24 |  |
| 1465.464 | 1.89 | 33.642 | 27.99 | 1.202 | 2.83 | 95 | Inner bank |
| 1466.065 | 2.49 | 55.440 | 30.02 | 1.847 | 3.77 | 209 | LRB |
| 1466.508 | 2.94 | 72.272 | 32.17 | 2.247 | 4.30 | 311 | RRB |
| 1467.200 | 3.63 | 101.226 | 38.16 | 2.653 | 4.80 | 486 |  |
| 1467.841 | 4.27 | 131.639 | 37.72 | 3.490 | 5.77 | 759 |  |

Figure 2: Depth versus Discharge graph

Therefore, the discharge is 110m3/s at tail water depth equivalent to 1.83m.

1.1.3. Estimation of Water Depth on Crest

a. Length of the Weir

From the existing topography of river bank width, the weir length is taken to be 35m.

b. Determination of Weir Crest level

- Average U/S river bed level ------------------------ 1463.72m

- Average Level of the highest field at 25m ------ 1465.12m

- Water depth required -------------------------------- 0.52m

- Slope of the canal x distance from the weir ----  0.025m (25m form intake)

- Head loss across head regulator ------------------- 0.10m

- Weir crest level ---------------------- 1465.12+0.52+0.025+0.10 = 1465.72m

- Weir height --------------------------- 1465.72-1463.72 = 2.00m

c. Water Depth on Crest

The river is non boulder river and also from the appropriate workmanship, the weir shape is selected to be broad crested with vertical upstream. Discharge over the weir is generally expressed as:-

Q = CLHe3/2,

Where: -Q=Peak flood (110m3/sec)

L =Effective weir length

C =Coefficient of discharge =1.70

He =Height of energy line  = [Q/CL] 2/3 = [(110/ (1.70*35)] 2/3= 1.52m

Va =Q/A=Q/ [L (hd+p]

Where: -Va= approach velocity, p=silt height ≈ Weir height (2.05m)

Va= 110/ (35(hd+2.05)) = ------------------------------------------ i

But, He =hd + hva hva=Va2/ (2g),

Va = (2g (1.52 -hd))1/2 ----------------------------------------------  ii

Equating (i) and (ii), the value of hd calculated by trial & errors as:

Table 2: Water Depth Calculation on Crest using trial and error method

|  | Trial & Error |  |
| --- | --- | --- |
| h | Left | Right |
| 1.450 | 0.81 | 1.37 |
| 1.460 | 0.80 | 1.18 |
| 1.48 | 0.79 | 0.78 |
| 1.500 | 0.78 | 0.39 |
| 1.520 | 0.78 | 0.00 |
| 1.540 | 0.77 | -0.39 |

Therefore, hd = 1.48m, & Hav= He-Hd = 1.51-1.48 = 0.03m.

# 1.1.4. Detail Part

a. Hydraulic Jump & Basin level

U/s riverbed elevation	 ---------------------- 1463.72m

Applying the Bernoulli's Energy Equations b/n u/s just before the jump

Z+ He = d1+hv1→ 2.05+1.51 = d1+hv1 but hv1= v12/2g

3.56 = d1+v12/2g → v1 = (2g (3.56-d1)) 1/2 --------------------------------- (i)

Unit discharge, q = Q/L = 110/35 = 3.14m3/m

V1 = q/d1 = 3.14/d1 ------------------------------------------------------------- (ii)

By trial & error d1 computed as follows

| Trial & Error |  |  |
| --- | --- | --- |
| d1 | i | ii |
| 0.36 | 7.92 | 8.72 |
| 0.38 | 7.90 | 8.26 |
| 0.40 | 7.87 | 7.85 |
| 0.41 | 7.86 | 7.66 |
| 0.42 | 7.85 | 7.48 |

∴ d1 = 0.40m

V1 = q/d1 = 3.14/0.40 = 8.55m/s

Fr = V1/(gd1)1/2	= 8.55/(9.81x 0.40)1/2 = 0.64

d2 = d1/2 [√1+8Fr2 - 1] = 0.40/2 [√1+8x0.642 - 1] = 1.36m

Conjugate depth d2 (1.36) is lower than tail water depth d3 (1.83). So the structure is for stilling basin slopping apron.

b. The Length of the Stilling Basin

∴ By energy method, L = 4d2 = 4*1.36 = 5.44m take 6.00m

c. Determination of the Detailed Parts

i. Weir section

For submerged conditions, the basic section of weir body can be calculated as follow:

Top width, Tw = He/√ ρ & Bottom width, Bw = (H +He)/√ ρ

Where, H- Weir height, &   ρ – Specific weight of weir body (2.25)

Tw = He/√ρ =2.90/√2.25 = 1.94 ≈2.00m take 1.20m but check stability

Bw = (H +He)/√ρ = (0.90+2.90)/ √2.25 =2.47 ≈2.58m take 2.80m

ii. Length & Thickness of Apron

For the design of the length and thickness of apron the maximum head differential on the structure, score depth and cut off depth of the apron are calculated as follow.

For dynamic case; Head on structure = Head water - Tail water

Hmax = 1467.20 – 1465.54 = 1.66m

For static case; Head on structure = Weir crest 	- river bed level

Hmax = 1465.75 – 1463.72 = 2.03m

Score depth

R = 1.35 [q2/f] 1/3

Where R = Lacey’s Regime score depth (m)

q = Discharge intensity (m3/sm)

f =   Lacey’s silt factor

f = 1.76√dmm

d = average particle size (mm)

q = Q/b = 158/20 = 7.9m

Type of river bed material is Gravel &Average grain size be 5.1mm

∴ Lacey’s silt factor f = 1.76 √5.1

f = 3.9, R = 1.35 [(7.9) 2/3.9] 1/3   = 3.4m

Downstream cutoff

Let us provide a downstream cutoff up to depth 1.5R-Hd below the downstream water level, which is 1535.94m

Upstream cutoff depth (D) =1.5R-Hd=1.5*3.4-1.79 ~ 2.43, but from geology basaltic rock is found at depth of 2m

∴Dup =2m is taken

Downstream cutoff

Ddown=1.5R-d3=1.5x3.4 -3.41=1.66m but to prevent the piping effect along the base of the structure the same cutoff depth of the will be provided at downstream apron as u/stream apron.

∴Ddow=2m

Total Floor Length and Exit gradient

The worst condition of flow occurred when the maximum flood is passing in the river and there is no water in the canal (i.e. the canal is completely closed) the bed level of the downstream floor. Therefore the maximum head differential on the structure is the static case i.e.

The maximum static head under this condition H=2.2, depth of the downstream cutoff d is equal 2.0m and safe Exit gradient is 1/5 given as the geologic information of the weir site.

GE = 'c    	=1/2+((1+2)/2) and =b/d

=

=

= 0.119 from plate – 11,2

= b/d where b stilling basin length, d=D/S cut off length

= b/d= 15.92/1.5= 10.6m

Adapted total floor length b = 16.00m. Hence, a balance of 2m lengths can be provided as u/s floor length. The floor length will be provided as below and as shown in fig:

Downstream horizontal length---------------10.0m

Crest width ---------------------------------------4.2m

Balance provided as upstream floor--------1.8m

Total     -------------------------------------------16.00m

Thickness of the floor

Based on Lanes conclusion that vertical parts of lines of creep are more effective than horizontal parts and recommended weighted creep ration in the form

L > CH	where-

L – Percolation distance (m)

H – Maximum Head (m)

C – Percolation coefficient

Percolation distance L = ≥CH from table 4-1 RIDD. Where

C = 3.0		= Sum of all vertical creep distance (m)

Hmax = 2.2m              = Sum of all horizontal creep distance (m)

L > 3* 2.2

L > 7.8m

L = (2+1.5+1.50+0.8+0.8+1.5)+1/3 (16)

L = 8.1+1/3(16)

L = 14.07

Weight creep ratio= L/Hmax = 14.07/2.2 = 7.76

According to Lanes recommended ratio, this structure would be safe from piping on coarse gravel including cobbles. The base material of the river consists of coarse gravels including cobbles. Therefore the length of the apron is sufficient to prevent the piping.

Thickness of floor at point A, B and C using Lanes creep theory and = 2.4

t =	      Where t – thickness of floor (m)

H – Uplift pressure at any point along the base (m)

- Specific gravity of floor material for concert (2.4)

Up lift pressure at any point is determined as.

h =H- , Where H – actual water head (m)

L – Equivalent creep length at the point along the base. (m)

| Point | Weighted creep length |  |  | Hmax (1-LA/LC) | TWL-WLA | f/(m-1) | t (m) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | LH | LV | Lc. |  |  |  |  |  |
| Dynamic Case Hmax = |  | 1.689 |  |  |  |  | Calculated | Adopted |
| A | 7.5 | 5.5 | 8.00 | (1.06) | 1.447 | 1.0 | 0.39 | 0.5 |
| B | 11.5 | 5.5 | 9.33 | (1.39) | 0 | 1.0 | (1.39) | 0.7 |
| C | 19.50 | 5.5 | 12.00 | (1.83) | 0 | 1.0 | (1.83) | 0.5 |
| Static Case Hmax = |  | 2.53 |  |  |  |  |  |  |
| A | 7.5 | 5.5 | 8.00 | (1.58) | 0 | 1.0 | (1.58) | 0.5 |
| B | 11.5 | 5.5 | 9.33 | (2.08) | 0 | 1.0 | (2.08) | 0.5 |
| C | 19.50 | 5.5 | 12.00 | (2.74) | 0 | 1.0 | (2.74) | 0.5 |

| C*Hmax dyn. = | 4.39. |  |  |  |
| --- | --- | --- | --- | --- |
| C*Hmax Stat= | 6.58 | Required condition: Lc C*Hmax |  |

Figure 3: Longitudinal Cross Section of Head work

# 1.1.5. Stability Analysis of the Weir

| 1.Stability Analysis of the Weir Body |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| The weir body will be analyzed for the following Cases: |  |  |  |  |  |  |
| 1. Over turning Failures |  |  |  |  |  |  |
| 2. Sliding Failures |  |  |  |  |  |  |
| 3. Tension failure |  |  |  |  |  |  |
| 4. Crushing (compression) Failure |  |  |  |  |  |  |
| To carry out the analysis, all the forces acting on the weir body that have stabilizing and |  |  |  |  |  |  |
| De-stabilizing potential has to be identified and quantified. |  |  |  |  |  |  |
| These are:- |  |  |  |  |  |  |
| 1. Self weight of the weir---------------Stabilizing |  |  |  |  |  |  |
| 2. Hydrostatic force ( U/S)--------------De-stabilizing |  |  |  |  |  |  |
| 3. Uplift pressure ------------------------De-stabilizing |  |  |  |  |  |  |
| 4. Silt pressure----------------------------De-stabilizing |  |  |  |  |  |  |
| Assumptions:- |  |  |  |  |  |  |
| i. Weir body material is masonry with specific wt.(t/m3)= |  |  |  | 2.3 |  |  |
| ii. Specific .wt of water (t/m3)= | 1 |  |  |  |  |  |
| iii. Back fill material specific wt. (t/m3)= |  | 1.8 |  |  |  |  |
| iv. μ = coefficient of friction value b/n masonry and masonry or concrete and concrete |  |  |  |  |  |  |
| range ( 0.65 - 0.75 ) |  |  |  |  |  |  |
| v. Angle of internal friction for the back fill soil is |  |  | 30⁰ |  |  |  |
| vi. (b)Unit cross section of weir= 1 |  |  |  |  |  |  |

| Name of forces | Symbol | Description | Forces (KN) |  |  | Lever arm | Moments (KN-m) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Vertical (+) | Horizontal (-) |  |  | (+) | (-) |  |
| 1.Vertical forces |  |  |  |  |  |  |  |  |  |
| 1.1 down ward |  |  |  |  |  |  |  |  |  |
| Weight of weir | W1 |  | 6.88 |  |  | 2.75 | 18.93 |  |  |
|  | W2 |  | 4.59 |  |  | 1.33 | 6.12 |  |  |
|  | W3 |  | 0.86 |  |  | 3.33 | 2.88 |  |  |
|  | W4 |  | 0.000 |  |  | 2.00 | 0.00 |  |  |
|  | W5 |  | 2.400 |  |  | 1.33 | 3.20 |  |  |
|  | W6 |  | 4.800 |  |  | 1.00 | 4.80 |  |  |
| Subtotal |  |  | 19.54 |  |  |  | 35.92 |  |  |
| weight of water | Ph2 |  | 1.110 |  |  | 1.00 | 1.11 |  |  |
|  | Ph3 |  | 1.995 |  |  | 0.67 | 1.33 |  |  |
| Subtotal | ΣWw |  | 3.105 |  |  |  | 2.437 |  |  |
| Sub - total of (1) |  |  | 22.641 |  |  |  | 38.361 |  |  |
| 1.2 Upward forces |  |  |  |  |  |  |  |  |  |
| uplift | Pu1 |  |  | 2.625 |  | 2.33 |  | 6.13 |  |
|  | Pu2 |  |  | 1.750 |  | 1.75 |  | 3.06 |  |
| Subtotal | Σpu |  |  | 4.375 |  |  |  | 9.188 |  |
| Summation ΣV & ΣM |  |  | 42.176 | 4.375 |  |  | 74.285 | 9.188 |  |
| 2. Horizontal force |  |  |  |  |  |  |  |  |  |
| 2.1 u/s water pressure |  |  |  |  |  |  |  |  |  |
|  | P1 |  |  | 1.991 |  | 0.67 |  | 1.324 |  |
| 2.2 u/s silt pressure |  |  |  |  |  |  |  |  |  |
|  | Ps |  |  | 2.156 |  | 0.67 |  | 1.434 |  |
| Summation ΣH & ΣM |  |  | 0.000 | 4.146 |  |  | 0.000 | 2.758 |  |
| Total |  |  | 42.176 | 8.521 |  |  | 74.285 | 11.945 |  |

Table 3: Stability Analysis for High Flood Condition

| 1. Sliding |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 3.22 | ok! | Resultant force R = |  | 43.03 | KN |
| 2. Over-turning |  |  |  |  | Max. Allowable BC of fine sand.KN/m2 |  |  | 100 |
|  |  |  | 6.22 | ok! | Min. Allowable BC of fine sand.KN/m2 |  |  | 75 |
| 3. Tension |  |  |  | B or L= | 3.50 | B/6 = | 0.58 |  |
|  |  |  | åM=åM+ - åM- |  | 62.3 |  |  |  |
|  |  |  | Arm of Resultant force from toe, X i.e. centroidal distance=åM/åFv= |  |  |  |  | 1.48 |
|  |  |  | Eccentricity, e = | 0.27 | ok! |  |  |  |
| 4. Vertical stress Pmax/min |  |  |  |  |  |  |  |  |
|  |  |  | Pmax = | 17.7 | ok! |  |  |  |
|  |  |  | Pmin = | 6.4 | ok! |  |  |  |

Table 4: Stability analysis for no Over Flow Condition

| Name of forces | Symbol | Description | Forces (KN) |  | Lever arm | Moments (KN-m) |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Vertical (+) | Horizontal (-) |  | (+) | (-) |
| 1.Vertical forces |  |  |  |  |  |  |  |
| 1.1 down ward |  |  |  |  |  |  |  |
| Weight of weir | W1 |  | 6.88 |  | 2.75 | 18.93 |  |
|  | W2 |  | 4.59 |  | 1.33 | 6.12 |  |
|  | W3 |  | 0.86 |  | 3.33 | 2.88 |  |
|  | W4 |  | 0.000 |  | 2.00 | 0.00 |  |
|  | W5 |  | 2.400 |  | 1.33 | 3.20 |  |
|  | W6 |  | 4.800 |  | 1.00 | 4.80 |  |
| Subtotal of (1) |  |  | 19.54 | 0.00 |  | 35.92 | 0.00 |
| 1.2 Upward forces |  |  |  |  |  |  |  |
| uplift | Pu1 |  |  | 2.625 | 2.33 |  | 6.13 |
|  | Pu2 |  |  | 1.750 | 1.75 |  | 3.06 |
| Subtotal | Σpu |  | 0.000 | 4.375 |  | 0.000 | 9.188 |
| Summation ΣV & ΣM |  |  | 19.535 | 4.375 |  | 35.924 | 9.188 |
| 2. Horizontal force |  |  |  |  |  |  |  |
| 2.1 u/s water pressure |  |  |  |  |  |  |  |
|  | P1 |  |  | 1.991 | 0.67 |  | 1.32 |
| 2.2 u/s silt pressure |  |  |  |  |  |  |  |
|  | Ps |  |  | 2.156 | 0.67 |  | 1.43 |
| Summation ΣH & ΣM |  |  | 0.000 | 4.146 |  | 0.000 | 2.758 |
| Total |  |  | 19.535 | 8.521 |  | 35.924 | 11.945 |

| 1. Sliding |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 1.49 | ok! | Resultant force R = |  | 21.31 | KN |
| 2. Over-turning |  |  |  |  | Max. Allowable BC of fine sand.KN/m2 |  |  | 100 |
|  |  |  | 3.01 | ok! | Min. Allowable BC of fine sand.KN/m3 |  |  | 75 |
| 3. Tension |  |  |  | B or L= | 3.50 | B/6 = | 0.583 |  |
|  |  |  | åM=åM+ - åM- |  | 24.0 |  |  |  |
|  |  |  | Arm of Resultant force from toe, X i.e. centroidal distance=åM/åFv= |  |  |  |  | 1.23 |
|  |  |  | Eccentricity, e = | 0.523 | ok! |  |  |  |
| 4. Vertical stress Pmax/min |  |  |  |  |  |  |  |  |
|  |  |  | Pmax = | 10.6 | ok! |  |  |  |
|  |  |  | Pmin = | 0.58 | ok! |  |  |  |

# 1.6 Stability of Retaining /Wing Wall

Upstream Wing Wall

Total Height of wall H, =Weir Height + He

H=h+He =2.00+1.52 = 3.52m but H=3.50m is taken for economic reason

U/s wing wall level = Upstream river bed level +height of wall

=1463.72 +3.50 = 1467.22m a.s.l

Taking depth of foundation 1m below upstream river bed level

Bottom width =2/3(height of wing wall + depth of foundation)

=2/3*(3.5+1) =3m

Top width =0.5m commonly used value

-Provide weep holes at least at every 1m vertical interval and 2m horizontal interval.

-Provide expansion joints for long masonry work.

| Considerations: |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| - Analysis per meter span and moment about heel. |  |  |  |  |  |  |
| - Earth pressure at rest was considered. |  |  |  |  |  |  |
| - Drained silty clay back fill was considered. |  |  |  |  |  |  |
| wm=2.3 ton/m3 | ww=1.0 ton/m3 | wsoil=2.0 ton /m3 |  |  |  |  |
| -Drained angle of internal friction() was taken 300. |  |  |  |  |  |  |
| ps= ½ kows H2 where K0= 1-sinϕ and hence ps = 95.819 kN/m say 96KN/m |  |  |  |  |  |  |
| design parameters |  |  |  |
| Wm=weight of masonry |  |  |  |
| Ws=weight of soil material |  |  |  |
| ᵞm=specific weight of masonry= |  |  | 2.2 |
| ᵞs=specific weight ofsoil = |  |  | 1.8 |
| Ф=angle of repose of the soil =300 |  |  |  |
| Ka =Rankin's coefficient of active earth pressure = 0.33 |  |  |  |

# 1.2. Irrigation System Design and Layout

The system layout of Uke irrigation project was summarized as follows:

- Consists of one-way diversion structure with canal head regulator to the Left side of the River

- A 5.150km Main Canal profile (both lined &earthen)

- Three Secondary canals

- Chutes structures on secondary canals

- Six drainage culvert on both main & secondary canal

- Two Flumes on main canal

- Six Road crossing box culverts as shown

- Field roads with slabs along the main canal and secondary canals

- Turnouts as shown with their Engineering symbols on the lay out

- Three Division box

- Drop with chute as shown with its Engineering symbol on the lay out

# 1.2. 1. Design of Main Canal

The main canal is one way left side and its total length is 5.15km of which 2000m (from 00- 2000) is lined and the rest is earthen canal

a. Line Main Canal

Q = 0.230m3/s, S = 1/1000, n = 0.014

For rectangular lined canal, the most efficient cross section is

Let    b/d = 1.15 & b = 0.60m

∴ Q = 1/n. R2/4. S1/2*1.15d2

0.230 = (1/0.014) x (1.15d2/3.15d) 2/3 x (1/1000)1/2

A=b*d = 1.15d2 = 0.31m2, P = b+2d = 3.15d = 1.64m & R = A/P = 0.19 of which

b = 60cm, b = 520cm & V = 0.75m/s provide fb =18cm

b.Earthen Main Canal

Q = 0.123m3/s, S = 1/100, n= 0.025, m=1

For trapezoidal efficient cross section of side slope of unity,

b/d = 0.67, & A = b*d = 0.67d2

∴ Q = (1/n) x (R2/3) x (s1/2) x1.84xd2

0.230 = 1/0.025 * 0.5d2/3 * 1/10001/2* bd+d2

d = 52cm & hence b = 35cm providing Fb of 18cm

V = Q/A = 0.142/0.28 = 0.51m/s

# 1.2.2. Design of Earthen Secondary Canal, SC1, SC2&SC3

a. Earthen Canal SC1

The secondary canal, SC1, total length is 720m (the 1st 240m is chute & the rest 480m is rest is earthen). The junction from main canal is at 1km+ 100m and its hydraulic analysis as followswhere as the chute part has been put in the in chute section:

∴   Q = 0.018m3/s,   n = 0.025, s = 1/1000, b = 0.86d & A = 1.86d2

Q = (1/n) * (R2/3) * (S1/2) * A

d = 35cm   b = 30cm, V = 0.40m/s providing, Fb = 15cm

b. Design of Earthen Secondary Canal SC2

The secondary canal, SC2, total length is 1260m (the 1st 280m is chute & the rest 980m is rest is earthen). The junction from main canal is at 1km+ 800m and earthen hydraulic analysis is as follows where as the chute part has been put in the in chute section:

∴   Q = 0.018m3/s,   n = 0.025, s = 1/1000, b = 0.86d & A = 1.86d2

Q = (1/n) * (R2/3) * (S1/2) * A

d = 35cm   b = 30cm, V = 0.40m/s providing, Fb = 15cm

c. Design Earthen Secondary Canal SC3

The secondary canal, SC3, total length is 1440m (from chain age 0 to 180m & also from 380 to 720m is chute & the rest is earthen). The junction from main canal is at 3km+ 900m and the earthen hydraulic analysis is as followswhere as the chute part has been put in the in chute section:

∴   Q = 0.018m3/s,   n = 0.025, s = 1/1000, b =d & A =2bd

Q = (1/n) * (R2/3) * (S1/2) * A

d = 30cm   b = 30cm, V = 0.40m/s providing, Fb = 15cm

# 1.2.3 Flume

Table 5:  Flumes Parameters

| No | Location Km + m |  | Length m | Q l/s |  |
| --- | --- | --- | --- | --- | --- |
|  | from | to |  |  |  |
| 1 | 1+760 | 1+782 | 22 | 230 |  |
| 2 | 3+717 | 3+740 | 23 | 230 |  |

# 1.2.1. Flume, F1

- Hydraulic Design:

- Flume Section:

Assume b = 1.5d, V1 = 1m/s, and n = 0.014

A = Q/V = 0.23/1 = 0.23m2

Assume b = 0.60m  d = 0.40m

A = b*d= 0.60x0.40 = 0.24 m2

V = Q/A = 0.23/0.24 = 0.96m/s

∴ Fr = V/= 0.96/

Fr = 0.48 ≤0.7 (subcritical flow) ----- Ok!

P = b+2d = 0.6+2x0.40 = 1.40m &  R = A/P = 0.23/1.40 = 0.17

Friction Head loss, Hf = L [nV/R2/3]2 = 22x [0.014x0.96/ (0.17)2/3]2 = 0.041

Drop of water surface at inlet, ∆h1 = 1.5 x (V22-V12)/2g

∆h1 1.5 x [0.962-0.702/ (2x9.81) = 0.03m

Raise of water surface at outlet, ∆h2 = 0.3(V22-V12)/2g = 0.01m

ii. Elevation:

- Inverted elevation at u/s canal = 1463.36m

- Water surface elevation at u/s canal = 1463.36+0.52= 1463.88m

- Water surface elevation at flume inlet = 1463.88+0.03= 1463.85m

- Invert elevation at the flume inlet = 1463.85-0.40 =1463.45m

- Invert elevation at the fume out let = 1463.45-0.04=1463.41m

-Water surface elevation at flume outlet =1463.41+0.40= 1463.81m

- Water surface elevation at d/s canal =1463.81-0.01=1463.80m

- Invert elevation at the d/s canal = 1463.80-0.52=1463.29m

iii. Structural Design:

Flume is a structure used to pass the canal over the cross drainage by supporting with columns without reducing its dimension. The structural design of flume and its columns that spanned over columns was as shown below.

Considerations:   - Uniformly distributed load

- Simply supported structure

- Unit weight of concrete = 24KN/m3

- Acceleration due to gravity = 10m/sec

- Unit weight of water = 10KN/m3

- The beam was designed for rectangular type

Dead load,W1 = (0.5x0.15x2+0.15x1) x 24 = 7.2KN/m

Life Load, W2 = (0.6x0.3) x 10 = 1.8KN/m

Factor of safety (F.S) = 1.3

Therefore, the overall load will be:

WT = (W1+W2) x F.S = (7.2+1.8) x 1.3  WT = 11.70KN/m

Mid span bending moment:

B.M = WL2/8 = 11.7x52/8 = 36.6KNm/m

Therefore, from the bending moment and shear force diagram,

+ B. Mmax = 22.72KNm/m

S. Fmax   = 35.44KN

- B. Mmax = 30.94KNm/m

Assuming one layer of ø14 main bar,

d = 150- ø/2-stirrup-cover     = 150-7-6-25 = 112mm

Check the resisting moment:

Assuming fs = 140, c= 6, kb= 0.2553, jb= 0.9149

M. R = (C/2) x kb x d2 x b x jb

= (6 x 103/2) x 0.2553 x 0.1122 x 1 x 0.9149 = 8.8KN/m <22.72KNm/mnot safe!

Therefore, the beam should be doubly reinforced.

For doubly-reinforced rectangular beam,

M. R = (bnc/2) x (d-n/3)

Where n = 0.39d = 0.39x0.112 = 0.044m

M.R. = (1x0.044x6x103/2) x (0.112-0.015)

M.R. = 12.8KNm/m = 12.6x106N-mm/m

At = M.R/ (fs x d x jb) = 12.6x106/ (140x112x0.9149) = 892.258mm2

Single area of ø14mm main bar = 153.938mm2

Therefore, 6ø14, @17cm c/c at the top and bottom

- Structural Design

i. Column Part:

A column is said long or short based on the slenderness ratio i.e, k = Le/r = 3.45Le/b

Where Le= 0.65L (Le = effective height, L = total height)

The length of the longest column in this flume is 9m.

Le = 0.65x9 = 5.85m

If the cross sectional area of the column is 20mx40m, then b = 0.4m and hence

K = 3.45Le/b = 3.45x5.85/0.4 = 50.45m = 50.45m > 45m

Therefore, the column is treated as long column and the design procedure for long column is as follows:

Pa = Ac cc + Asc + sc, where

- Compressive stress, cc in concrete = 6N/mm2

- Compressive tress in steel, sc, = 140N/mm2

- Pa = total allowable load

Assuming 4ø 16 main bars for the column, total area, Asc = 4x л (0.022)2/4 = 0.0008m2

Pa = (0.2x0.4) x 6x103+0.0008x140x103 = 592KN

But for long columns, the allowable load (Pa) should be multiplied by some factor (cr)

Cr = 1.25 – Le/48b = 1.25-5.85/ (48x0.4),   Cr = 0.95

Pallowable = Pa x cr = 592x0.95 = 562.40KN

The external load, Px = 0.2x0.4x9x24+11.7x5 = 75.78KN

Px x F. S = 75.78x1.3 = 98.51<562.40

Therefore, the dimension provided for the column is sufficient to carry the loads and use 4ø16, at the corners and ø6 ties at 20cm c/c.

ii. Footing

Assume the footing has a length of 1m, a width of 0.6m, and a depth of 1m and the footing rests on hard clay, safe bearing of which is 30tone/m2.

Pexternal     = 98.51+weight of footing = 98.51+0.6 x1x1x24 = 112.91KN

Pallowable      = Px/A

A = 0.6x1 = 0.6m2

Pallowable = 112.91/0.6 = 188.2KN/m2

188.2KN/m2< 300KN/m2 →safe

Bending moment, +M = Px x b/2 = 112.91x0.3 = 33.87KNm

Assuming ø16 main bar, and D = 350mm

d = 350-8-6-30= 306mm

MR = (C/2) x kb x d2 x bx jb

= (6x103/2) x 0.2553 x 0.3032 x 0.6 x 0.9149 = 38.60KNm > 33.87KNm → OK

∴ At = +M/ (fs x d x jb) = 33.87 x 106/ (140 x 306 x 0.9149) = 864.156mm2

Therefore, use 4ø16, @ 14cm c/c at the bottom

1.2.2. Flume, F2

This structure is located between stations 3+717 and 3+740 and the total flume length is 23m.

Hydraulic design:

i. Flume section:

Assume b = 1.5d, V2 = 1m/s and n = 0.014

A = Q/V = 0.24/1 = 0.24m2

A = b*d = 2d2 =  d = VA/2 =  = 0.40m

b = 2d = 2x027 = 0.54m

New A = b x d = 0.60x0.40 = 0.24m2

V2 = Q/A = 0.142/0.1458 = 0.9738m/s

Fr = V/ = 0.9739/ = 0.5984 ≤ 0.70 OK

P = b+2d = 0.60+2(0.40) = 1.40m

R = A/P = 0.24/1.40 = 0.170m

hf = 0.04, ∆h1=0.03m, ∆h2=0.010m

Total head loss: HT = 1.2 (V22-V12)/2g+ hf = 0.1764m

ii. Elevation:

-Inverted elevation at u/s canal = 1461.35m

- Water surface elevation at u/s canal = 1461.35+0.52= 1461.87m

- Water surface elevation at flume inlet = 1461.87+0.30= 1461.84m

- Invert elevation at the flume inlet = 1461.84-0.40 =1461.44m

- Invert elevation at the fume out let = 1461.44-0.04=1461.39m

-Water surface elevation at flume outlet =1461.39+0.40= 1461.79m

- Water surface elevation at d/s canal =1461.79-0.01=1461.80m

- Invert elevation at the d/s canal = 1461.80-0.52=1461.28m

b.Structural design:

For the definition and considerations (assumptions), see structural design of flume one

(F1) The load intensity, the spacing and all the hydraulic calculations are the same as that of flume1 (F1).

Therefore, from the shear force ad bending moment diagram,

+ B.Mmax = 22.72KN/m

- B.Mmax = 30.94KNm/m

S. Fmax = 35.44KN

Since the dimension and the spacing are the same as that of flume1,

Use 6 ø 14, @ 17cm c/c at the top and bottom

The design of columns and footing is the same as that of flume1 except that the column here is treated as short column since the max height of the column is 5m (k ≤45). Hence, the dimensions provided for the column (0.2x0.4) and footing (1x0.6x1) are sufficient to carry the loads.

For the columns, Use 4 ø16 at the corners and ø6 ties at 20cm c/c spacing

For the footing, Use 4 ø 16, @ 14cm c/c at the bottom

# 1.3. CHUTES

The structures are located on secondary canals and their specific parameters are described below.

| Canal Type | Station Km + m |  | Elevation |  | Chute length | Stilling Basin |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | from | to | A | B | m | Length, L | Width, W |
| SC1 | 0+000 | 0+100 |  |  |  |  |  |
| SC1 | 0+100 | 0+240 |  |  |  |  |  |
| SC2 | 0+000 | 0+140 |  |  |  |  |  |
| SC2 | 0+140 | 0+280 |  |  |  |  |  |
| SC3 | 0+000 | 0+180 |  |  |  |  |  |
| SC3 | 0+380 | 0+720 |  |  |  |  |  |

Toward its beginning, the Main canal run through profiles of steeply slopes. Therefore chute structure is provided this steeply slope profiles. The lengths and chain age of the chute structure is described as bellow.

a. Critical Flow hydraulics

Q1 = m3/s

| Maximum Permissible values of V |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Channel Type | Velocity(m/s) |  |  |  |  |
| Sandy soil | 0.45 |  |  |  |  |
| Sandy loam | 0.6 |  |  |  |  |
| Loam | 0.7 |  |  |  |  |
| Clayey loam | 0.9 |  |  |  |  |
| Clayes | 1 |  |  |  |  |
| clay with sand | 1.2 |  |  |  |  |
| soft rock | 2.5 |  |  |  |  |
| Moderately hard rock | 2.5 |  |  |  |  |
| hard rock | 3 |  | B- Canal Bed Width |  |  |
| Thick conc.(>18cm) | 3.5 |  | Y- Canal Water depth |  |  |
| Thin conc.(<10ch) | 1.5 |  | F- Canal Free Board, Shall not be less than 0.3m |  |  |
| Asphalt | 1 |  | for earthen canals except for field canals and TC . |  |  |
| Conc.block dry masonry |  |  | of little flow rate. |  |  |
| (Stay< 30 cm) | 1.5 |  | T- Canal top width |  |  |
| (Stay > 30 cm) | 2 |  | W- Canal embankment top width |  |  |
| Conc.Block wet masonry | 2.5 |  | z- Canal side slope(external) |  |  |
| pre-cast conc. Pipe | 2.5 |  | m- Canal side slope( inner) |  |  |
| Steel pipe | 5 |  |  |  |  |

Figure 4: Typical Canal Cross Section

| EL.A = | 1537.284 | Chute Inlet Elevation(bed |  |  |
| --- | --- | --- | --- | --- |
| EL.B = | 1522.79 | Chute Outlet Elevation(bed) |  |  |
| L = | 153 | Apparent Chute length |  |  |
| d3= | 0.4 | Water depth in the off take Canal |  |  |
| V3= | 0.558 | Velocity in the off take Canal |  |  |
| hv3=V32/2g | 0.0158 | Velocity Head in the off take Canal |  |  |
| bc = 0.734Q/do3/2 | 0.4000 | critical Canal width |  |  |
| qc =Q/bc | 0.208 | critical flow |  |  |
| dc =(qc2/g)1/3 | 0.164046461 | critical Canal depth |  |  |
| vc =qc/dc | 1.267933482 | critical velocity |  |  |
| hvc =vc2/2g | 0.081939618 |  |  |  |
| Ac =bcdc | 0.065618584 |  |  |  |
| Pc =bc+2dc | 0.728092921 |  |  |  |
| Rc =Ac/Pc | 0.090123915 |  |  |  |
| Ic =(nvc/Rc2/3)2 | 0.470689308 |  |  |  |

| Ec= | E1= | 14.6 |  |
| --- | --- | --- | --- |
| 4.Conjugate Depth after jump |  |  |  |
| Froude no(Fr)= | V1/(V1/(gd1)^0.5 | 5.95189 |  |
| Post depth(dp)ord2= | d1/2*[(1+8Fr2)^0.5-1] | 1.9037 |  |
| 5.Stilling basin |  |  |  |
| Length,LS= | 4d2 | 7.614798 |  |
| Width,B= | 18.46*Q1/2/(Q+9.91 | 0.076021 |  |
| 6.Bottom Elevation(EL.C) |  |  |  |
| V2= | qc/d2= | 0.109261 |  |
| hv2= | v22/2g= | 0.000608 |  |
| E2= | d2+hv2= | 1.904308 |  |
| E3= | d3+hv3= |  |  |
| Leap height(a) | E2-E3= |  |  |
| EL.C= | EL.B-a = | 1522.79 |  |
|  | Q= | 0.0823 |  |
| Parant canal | Chute |  |  |
| Q= | 0.0823 |  |  |
| do= | 0.4 | dc= | 0.164046 |
| vo= | 0.558 | bc= | 0.4000 |
| bo= | 0.6 | Fb= | 0.15 |
| Fb= | 1.5 | ELA= | 1537.284 |
|  |  | ELB= | 1522.79 |
|  |  | LC | 153 |

# 1.4. Division Boxes

# 1.4.1. Hydraulic Design

The main propose of division box structure is to regulate or control the supply entering the secondary canal and also serve as a meter for measuring discharge.

It is proposed to divide water from the main canals to the secondary canals and/or secondary canals to sub-secondary canals.

Table 6: Division Box Hydraulics

| Division Box No | Location K + m | Branch Canal | Discharge, Q lt/s |
| --- | --- | --- | --- |
| 1 | 1+100 | Sc1 | 18 |
| 2 | 1 + 800 | Sc2 | 27 |
| 3 | 3 + 900 | Sc3 | 55 |

Consider here a division box at the junctions of SC1, SC2 and SC3from parent main canals at different locations.

Since the flow in all canals are open channels, these division boxes are designed using broad crest flow formula by assuming the same discharge coefficient & sill height for all distribution direction.Here under aredetailed design parameters for the selected sample division box mentioned on SC1:

Design of Division Box 1: Sample design at Station:- 1+100 on MC junction for SC1

Using broad crested formula,

|  |  |  |  |  | 0.80 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 1.00 |  |  |  |  |  |  |  | 1.00 |
|  |  |  | 0.50 | 1.0 |  |  |  |  |  |

Figure 5: Schematic Layout of Considered/Sample Division Box

Hydraulic Dimensions:

Q= CL (h)3/2

Where Q= Parent canal discharge = 0.230m3/s

w = width of parent canal =0.60m

C= discharge coefficient, C= 1.70

h = height of D.box =d + fb = 0.52+0.18=0.70m

w = width of D.box =d + fb = 0.52+0.18 +0.1=0.80m

w1 = width of distributor canal SC1 =0.30m

b1= L1Q1/ Q= 0.06m

However, from practical point of view take convenient dimensions of b1 = 0.3m which is equivalent to incoming canal bed width, b2 = 0.30mand b3 = 0.3m and control required depth of flow in each subsequent canals by gates as per presented design tables for corresponding canals.

Top width of Division Box = bed width of parent canal +2*m*d = 1.40m. The division box is assumed of squaredimensions (For more information, refer Drawing Album).

# 1.5. Turn Outs

Table 7:  Dimensions for Turn Outs

| Turn Out | Area | Location | Q | E | E1 | B | B1 | H | H1 | h | b | Ø |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| No | m2 | km+m | lt/s | m | m | cm | cm | cm | cm | cm | cm | cm |
| MC-T1 | 0.09 | 0+150 | 0.001 | 1465.91 | 1465.91 | 80 | 140 | 20 | 4 | 26 | 22 | 10 |
| MC-T2 | 0.12 | 0+175 | 0.001 | 1465.91 | 1465.91 | 80 | 140 | 20 | 4 | 26 | 22 | 10 |
| MC-T3 | 0.73 | 0+275 | 0.006 | 1465.85 | 1465.91 | 80 | 140 | 20 | 4 | 26 | 22 | 10 |
| MC-T4 | 0.12 | 0+450 | 0.001 | 1465.91 | 1465.91 | 80 | 140 | 20 | 4 | 26 | 22 | 10 |
| MC-T5 | 1.51 | 0+525 | 0.013 | 1465.91 | 1465.91 | 80 | 140 | 20 | 4 | 26 | 22 | 20 |
| MC-T6 | 2.35 | 0+640 | 0.019 | 1465.91 | 1465.91 | 80 | 140 | 20 | 4 | 26 | 22 | 20 |
| MC-T7 | 2.85 | 0+775 | 0.024 | 1465.91 | 1465.91 | 80 | 140 | 20 | 4 | 26 | 22 | 20 |
| MC-T8 | 1.34 | 0+875 | 0.011 | 1465.91 | 1465.91 | 80 | 140 | 20 | 3 | 27 | 22 | 20 |
| MC-T9 | 0.45 | 0+950 | 0.004 | 1465.91 | 1465.91 | 80 | 140 | 20 | 3 | 27 | 22 | 10 |
| MC-T10 | 2.36 | 1+050 | 0.020 | 1465.91 | 1465.91 | 80 | 140 | 20 | 2 | 28 | 22 | 20 |
| MC-T11 | 1.93 | 1+160 | 0.016 | 1465.91 | 1465.91 | 80 | 140 | 20 | 3 | 27 | 22 | 20 |
| MC-T12 | 1.86 | 1+350 | 0.015 | 1465.91 | 1465.91 | 80 | 140 | 20 | 4 | 26 | 22 | 20 |
| MC-T13 | 1.22 | 1+450 | 0.010 | 1465.91 | 1465.91 | 80 | 140 | 20 | 2 | 28 | 22 | 20 |
| MC-T14 | 0.72 | 1+600 | 0.006 | 1465.91 | 1465.91 | 80 | 140 | 20 | 3 | 27 | 22 | 10 |
| MC-T15 | 0.69 | 1+700 | 0.006 | 1465.91 | 1465.91 | 80 | 140 | 20 | 4 | 26 | 22 | 10 |
| MC-T16 | 0.17 | 1+850 | 0.001 | 1465.91 | 1465.91 | 80 | 140 | 20 | 2 | 28 | 22 | 10 |
| MC-T17 | 0.47 | 1+925 | 0.004 | 1465.91 | 1465.91 | 80 | 140 | 20 | 1 | 29 | 22 | 10 |
| MC-T18 | 0.40 | 2+000 | 0.003 | 1465.91 | 1465.91 | 80 | 140 | 20 | 1 | 29 | 22 | 10 |
| MC-T19 | 0.46 | 2+075 | 0.004 | 1465.91 | 1465.91 | 80 | 140 | 20 | 1 | 29 | 22 | 10 |
| MC-T20 | 0.51 | 2+125 | 0.004 | 1465.91 | 1465.91 | 80 | 140 | 20 | 1 | 29 | 22 | 10 |
| MC-T21 | 0.82 | 2+175 | 0.007 | 1465.91 | 1465.91 | 80 | 140 | 20 | 1 | 29 | 22 | 10 |
| MC-T22 | 0.98 | 2+275 | 0.008 | 1465.91 | 1465.91 | 80 | 140 | 20 | 1 | 29 | 22 | 10 |
| MC-T23 | 0.73 | 2+375 | 0.006 | 1465.91 | 1465.91 | 80 | 140 | 20 | 1 | 29 | 22 | 10 |
| MC-T24 | 0.86 | 2+450 | 0.007 | 1465.91 | 1465.91 | 80 | 140 | 20 | 2 | 28 | 22 | 10 |
| MC-T25 | 1.21 | 2+550 | 0.010 | 1465.91 | 1465.91 | 80 | 140 | 20 | 3 | 27 | 22 | 20 |
| MC-T26 | 1.22 | 2+625 | 0.010 | 1465.91 | 1465.91 | 80 | 140 | 20 | 3 | 27 | 22 | 20 |
| MC-T27 | 1.38 | 2+740 | 0.011 | 1465.91 | 1465.91 | 80 | 140 | 20 | 4 | 26 | 22 | 20 |
| MC-T28 | 1.94 | 2+825 | 0.016 | 1465.91 | 1465.91 | 80 | 140 | 20 | 4 | 26 | 22 | 20 |
| MC-T29 | 1.90 | 2+925 | 0.016 | 1465.91 | 1465.91 | 80 | 140 | 20 | 4 | 26 | 22 | 20 |
| MC-T30 | 1.63 | 3+000 | 0.013 | 1465.91 | 1465.91 | 80 | 140 | 20 | 4 | 26 | 22 | 20 |
| MC-T31 | 1.46 | 3+175 | 0.012 | 1465.91 | 1465.91 | 80 | 140 | 20 | 2 | 28 | 22 | 20 |
| MC-T32 | 0.67 | 3+275 | 0.006 | 1465.91 | 1465.91 | 80 | 140 | 20 | 2 | 28 | 22 | 10 |
| MC-T33 | 0.46 | 3+360 | 0.004 | 1465.91 | 1465.91 | 80 | 140 | 20 | 2 | 28 | 22 | 10 |
| MC-T34 | 0.62 | 3+475 | 0.005 | 1465.91 | 1465.91 | 80 | 140 | 20 | 1 | 29 | 22 | 10 |
| MC-T35 | 1.48 | 3+550 | 0.012 | 1465.91 | 1465.91 | 80 | 140 | 20 | 1 | 29 | 23 | 20 |
| MC-T36 | 1.21 | 3+650 | 0.010 | 1465.91 | 1465.91 | 80 | 140 | 20 | 1 | 29 | 24 | 20 |
| MC-T37 | 1.04 | 3+850 | 0.009 | 1465.91 | 1465.91 | 80 | 140 | 20 | 1 | 29 | 25 | 20 |
| MC-T38 | 0.23 | 4+950 | 0.002 | 1465.91 | 1465.91 | 80 | 140 | 20 | 1 | 29 | 26 | 10 |
| MC-T39 | 0.32 | 4+050 | 0.003 | 1465.91 | 1465.91 | 80 | 140 | 20 | 1 | 29 | 27 | 10 |
| MC-T40 | 0.37 | 4+100 | 0.003 | 1465.91 | 1465.91 | 80 | 140 | 20 | 1 | 29 | 28 | 10 |
| MC-T41 | 0.51 | 4+165 | 0.004 | 1465.91 | 1465.91 | 80 | 140 | 20 | 1 | 29 | 29 | 10 |
| MC-T42 | 0.45 | 4+250 | 0.004 | 1465.91 | 1465.91 | 80 | 140 | 20 | 1 | 29 | 30 | 10 |
| MC-T43 | 0.61 | 4+350 | 0.005 | 1465.91 | 1465.91 | 80 | 140 | 20 | 1 | 29 | 31 | 10 |
| MC-T44 | 1.01 | 4+450 | 0.008 | 1465.91 | 1465.91 | 80 | 140 | 20 | 1 | 29 | 32 | 20 |
| MC-T45 | 1.19 | 4+550 | 0.010 | 1465.91 | 1465.91 | 80 | 140 | 20 | 1 | 29 | 33 | 20 |
| MC-T46 | 1.18 | 4+640 | 0.010 | 1465.91 | 1465.91 | 80 | 140 | 20 | 1 | 29 | 34 | 20 |
| MC-T47 | 1.04 | 4+710 | 0.009 | 1465.91 | 1465.91 | 80 | 140 | 20 | 1 | 29 | 35 | 20 |
| MC-T48 | 1.36 | 4+810 | 0.011 | 1465.91 | 1465.91 | 80 | 140 | 20 | 1 | 29 | 36 | 20 |
| MC-T49 | 1.35 | 4+900 | 0.011 | 1465.91 | 1465.91 | 80 | 140 | 20 | 1 | 29 | 37 | 20 |
| MC-T50 | 1.35 | 4+975 | 0.011 | 1465.91 | 1465.91 | 80 | 140 | 20 | 1 | 29 | 38 | 20 |
| MC-T51 | 1.64 | 5+075 | 0.014 | 1465.91 | 1465.91 | 80 | 140 | 20 | 1 | 29 | 39 | 20 |
| MC-T52 | 1.39 | 5+175 | 0.011 | 1465.91 | 1465.91 | 80 | 140 | 20 | 1 | 29 | 40 | 20 |
| SC1-T1 | 1.62 | 0+225 | 0.013 | 1465.91 | 1465.91 | 80 | 140 | 20 | 1 | 29 | 41 | 20 |
| SC1-T2 | 1.04 | 0+320 | 0.009 | 1465.91 | 1465.91 | 80 | 140 | 20 | 1 | 29 | 42 | 20 |
| SC1-T3 | 1.35 | 0+400 | 0.011 | 1465.91 | 1465.91 | 80 | 140 | 20 | 1 | 29 | 43 | 20 |
| SC1-T4 | 1.68 | 0+530 | 0.014 | 1465.91 | 1465.91 | 80 | 140 | 20 | 1 | 29 | 44 | 20 |
| SC1-T5 | 1.69 | 0+660 | 0.014 | 1465.91 | 1465.91 | 80 | 140 | 20 | 1 | 29 | 45 | 20 |
| SC2-T1 | 1.50 | 0+110 | 0.012 | 1465.91 | 1465.91 | 80 | 140 | 20 | 1 | 29 | 46 | 20 |
| SC2-T2 | 1.08 | 0+185 | 0.009 | 1465.91 | 1465.91 | 80 | 140 | 20 | 1 | 29 | 47 | 20 |
| SC2-T3 | 1.03 | 0+290 | 0.009 | 1465.91 | 1465.91 | 80 | 140 | 20 | 1 | 29 | 48 | 20 |
| SC2-T4 | 0.72 | 0+380 | 0.006 | 1465.91 | 1465.91 | 80 | 140 | 20 | 1 | 29 | 49 | 10 |
| SC2-T5 | 0.45 | 0+328 | 0.004 | 1465.91 | 1465.91 | 80 | 140 | 20 | 1 | 29 | 50 | 10 |
| SC2-T6 | 0.52 | 0+500 | 0.004 | 1465.91 | 1465.91 | 80 | 140 | 20 | 1 | 29 | 51 | 10 |
| SC2-T7 | 0.43 | 0+600 | 0.004 | 1465.91 | 1465.91 | 80 | 140 | 20 | 1 | 29 | 52 | 10 |
| SC2-T8 | 0.39 | 0+665 | 0.003 | 1465.91 | 1465.91 | 80 | 140 | 20 | 1 | 29 | 53 | 10 |
| SC2-T9 | 0.55 | 0+730 | 0.005 | 1465.91 | 1465.91 | 80 | 140 | 20 | 1 | 29 | 54 | 10 |
| SC2-T10 | 0.77 | 0+840 | 0.006 | 1465.91 | 1465.91 | 80 | 140 | 20 | 1 | 29 | 55 | 10 |
| SC2-T11 | 0.51 | 0+920 | 0.004 | 1465.91 | 1465.91 | 80 | 140 | 20 | 1 | 29 | 56 | 10 |
| SC2-T12 | 0.81 | 1+040 | 0.007 | 1465.91 | 1465.91 | 80 | 140 | 20 | 1 | 29 | 57 | 10 |
| SC2-T13 | 1.10 | 1+140 | 0.009 | 1465.91 | 1465.91 | 80 | 140 | 20 | 1 | 29 | 58 | 20 |
| SC2-T14 | 1.48 | 1+120 | 0.012 | 1465.91 | 1465.91 | 80 | 140 | 20 | 1 | 29 | 59 | 20 |
| SC3-T1 | 0.91 | 0+140 | 0.008 | 1465.91 | 1465.91 | 80 | 140 | 20 | 1 | 29 | 60 | 10 |
| SC3-T2 | 0.70 | 0+290 | 0.006 | 1465.91 | 1465.91 | 80 | 140 | 20 | 1 | 29 | 61 | 10 |
| SC3-T3 | 0.39 | 0+420 | 0.003 | 1465.91 | 1465.91 | 80 | 140 | 20 | 1 | 29 | 62 | 10 |
| SC3-T4 | 0.94 | 0+515 | 0.008 | 1465.91 | 1465.91 | 80 | 140 | 20 | 1 | 29 | 63 | 10 |
| SC3-T5 | 1.67 | 0+740 | 0.014 | 1465.91 | 1465.91 | 80 | 140 | 20 | 1 | 29 | 64 | 20 |
| SC3-T6 | 1.67 | 0+840 | 0.014 | 1465.91 | 1465.91 | 80 | 140 | 20 | 1 | 29 | 65 | 20 |
| SC3-T7 | 1.42 | 0+920 | 0.012 | 1465.91 | 1465.91 | 80 | 140 | 20 | 1 | 29 | 66 | 20 |
| SC3-T8 | 1.71 | 0+980 | 0.014 | 1465.91 | 1465.91 | 80 | 140 | 20 | 1 | 29 | 67 | 20 |
| SC3-T9 | 1.54 | 1+060 | 0.013 | 1465.91 | 1465.91 | 80 | 140 | 20 | 1 | 29 | 68 | 20 |
| SC3-T10 | 1.04 | 1+100 | 0.009 | 1465.91 | 1465.91 | 80 | 140 | 20 | 1 | 29 | 69 | 20 |
| SC3-T11 | 1.62 | 1+130 | 0.013 | 1465.91 | 1465.91 | 80 | 140 | 20 | 1 | 29 | 70 | 20 |
| SC3-T12 | 1.78 | 1+180 | 0.015 | 1465.91 | 1465.91 | 80 | 140 | 20 | 1 | 29 | 71 | 20 |
| SC3-T13 | 1.33 | 1+240 | 0.011 | 1465.91 | 1465.91 | 80 | 140 | 20 | 1 | 29 | 72 | 20 |
| SC3-T14 | 1.76 | 1+290 | 0.015 | 1465.91 | 1465.91 | 80 | 140 | 20 | 1 | 29 | 73 | 20 |
| SC3-T15 | 2.24 | 1+340 | 0.019 | 1465.91 | 1465.91 | 80 | 140 | 20 | 1 | 29 | 74 | 20 |
| SC3-T16 | 2.00 | 1+420 | 0.017 | 1465.91 | 1465.91 | 80 | 140 | 20 | 1 | 29 | 75 | 20 |

# 1.6. Culverts

# 1.6.1. Road Crossing Box Culvert

1. Hydraulic Calculation

A. Flow Hydraulics Canal

| Q, m3/s | b, m | d, m | s/s | bed slope | v, m/s | n |
| --- | --- | --- | --- | --- | --- | --- |
| 0.230 | 0.60 | 0.70 |  | 1/1000 | 0.75 | 0.014 |

B. Flow Hydraulics of Culvert

b2 = 0.50, d2 = 0.52,   Fb =0.18 &n2 = 0.014

A2 = b2*d2 = 0.50 x 0.52 = 0.26, V2 = Q/A2 = 0.23/0.26 = 0. 0.88m/sec

hv2 = v22/2g = 0.882/2x9.81 = 0.04, P2 = b+2d2 = 0.50*2+0.52 = 1.54m

R2= A2/P2 = 0.26/1.54 = 1.69, I2 = (nv/R2/3) 2 = 0.0016, &

Difference in elevation b/n B & C, Hf = L x I2 = 5x0.0016 = 0.008

C. Water Surface Change at u/s and d/s of Culvert

i. Drop of water surface at the millet of the culvert

∆h1 = 1.5*(v22-v12/) 2g = 1.5*(0.882-0.752)/19.62 = 0.010m

∆h2 = 0.3*(v22-v12)/2g= 0.3*(0.88-0.75)/19.62   = 0.0020m

ii. Elevation

Invert elev. at u/s canal 	= 1462.72m

Water surface elev. at u/s canal= 14624.72 + 0.52 = 1463.24

Water surface at the culvert inlet= (water surface elev. at u/s canal) - ∆h1

= 1463.24-0.010 = 1463.23

Invert elev. at the culvert inlet= (water surface at the culvert inlet)-d2

= 163.23 - 0.52 = 1462.71

Invert elev. at the culvert out let= invert elev. at the culvert inlet) – hf

1462.71 – 0.008 = 1462.70

Water surface elev. at the culvert out let = (invert elev. at the culvert out let) + d2

1462.70+0.52 = 1463.22

Water surface elev. at the d/s canal= Water surface elev. at the culvert out let) + ∆h2

= 1463.22+0.002 = 1463.222

Invert elevation at the d/s canal = (water surface elev. at the d/s canal)-d1

=1463.222-0.52 = 1462.70

d. Total head lass

HT = 0.5 ∆hv + hf + 0.7∆h2v = 1.2∆hv + hf = 1.2 (v22 – v12/2g) + hf

=1.2*(0.88^2 -0.75^2)/19.62)+0.008 = 0.021

Check water surface difference b/n u/s and d/s canal. The difference, Z

Z= (Water surface elev. at u/s canal) -(Water surface elev. at d/s canal)

Z= 1463.24– 1463.22= 0.020------- Ok!

Table 8: Typical Canal Elevations at Each Section

| Section | Location 2+350 | Water Depth | Change in Water Level |  | Elevation |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Canal Bed | Water Surface |
| A | U/S Canal | 0.52 |  |  | 1462.72 | 1463.240 |
| B | Barrel | 0.52 | Drop | 0.010 | 1462.71 | 1463.220 |
| C | Barrel | 0.52 | Drop | 0.008 | 1462.70 | 1463.220 |
| D | D/S Canal | 0.52 | Rise | 0.002 | 1462.70 | 1463.222 |

2. STRUCTURAL DESIGN

a. Design condition

Live load = T – 14

Allowable stress

Ⴀca (concrete)	= 55kg/cm2

Ⴀsa (reinforcement) = 1400k8/cm2

Ⴀa (shear) = 9.5 kg/cm

Ⴀoc (adhesive) = 8kg/cm2

b. Moment

L = b+2c = 0.60+2(0.25) = 1.10m

M = Pl/4 = 5.0*1.10/4 = 1.375 ton m = 137,500kg-cm

S = p/2 = 5/2 = 2.5ton = 2500kg

c. Reinforcement

d = t-5 = 18-5 = 13cm ≥11.20 Ok!

Check,	d = C1√M/b = 0.334√137500/100 =12.39cm

- Main Reinforcement

As1 = C2√Mb = 0.00244√ (137500*100) = 9.048 ≤ 10.26cm2 Ok!

If Ø= 14mm with interval of 15cm is used the total area of reinforcement per meter will be 10.26cm2. It can be expressed as As1 = Ø14 @15 = 10.26cm2

- Negative Reinforcement

As2 = 0.2 As1 = 0.20*9.048 = 1.81 ≤3.35cm2 Ok!

If Ø= 8mm with the interval of 15cm is used, the total area of reinforcement per meter will be 3.35cm2. It can be expressed as As = Ø8 @15 = 3.35cm2

- Distribution Bar

As3 = 0.2As1 = 0.2x9.048 = 1.81 ≤ 3.14cm2 Ok!

If Ø= 8mm with the interval of 20cm is used the total area of reinforcement per meter will be 3.14cm2. It can be expressed As As = Ø8@20 = 3.14cm2

d. Shearing Stress

Ⴀ = S/bjd = 2500/ 100x0.875x0.98 = 2.9 ≤ 9.5 kg/cm2 Ok

e. Adhesive Stress	Ⴀ = 0.5.5/ujd = 0.5 .2500/29.32.0.875.13 = 3.75≤ 8.0kg/cm2

| No | Ø, mm | Length, m | Total Length | Weight, kg | Total Weight |
| --- | --- | --- | --- | --- | --- |
| 1 | 8mm | 0.90 | 38.00 | 0.42 | 16kg |
| 2 | 14mm | 3.00 | 42.00 | 1.27 | 54kg |

f. Foundation

5.00

5.6T                     5. 6T

C= 0.60             W= 1.8               C=0.60

1.50

Area to be stressed

Af = [c+w/2] x 2a = [0.60+1.80/2] x 20.8 = 2.40m

Load

Concentrated load = i x p  = 1.2 x 5.6    = 6.72 ton

Weight = Span + wall

= {0.94x0.18x1.50} x2.4 + {(05+0.8) x0.93+ (1.6+0.25)} 2.3    = 7.65t

Total=Concentrated load +dead load = 6.72+7.65 = 14.37 ton.

Stress= Total Load/ Area to be stressed = 14.37/2.4 = 6.00   ≤ 20kg/m2 Ok

The bearing capacity of the soil will be taken from the soil report 20kg/m2 so it’s Ok

This structure was laid at the main canal and secondary canals crossing the main road and field roads. Their locations and dimensions were shown in the drawing.

The maximum load assumed here in the analysis of the slab was that of Dozer D-7 which 32 tone (32,000kg) is.

Note: The construction material for the masonry part should be the selected type and the stone should be well brushed. Mix-ratio for masonry should be 1:3 and that of concrete should be 1:2:4.

2.1Drainage Pipe Culvert

ø = 50cm

Chainage (2030m —2036m)

Hydraulic calculation

Given data

| Discharge | Bed width | Water depth | Side slope | Velocity | Canal bed slope | Coefficient roughness |
| --- | --- | --- | --- | --- | --- | --- |
| Q(m3/s) | b(m) | d(m) | m | V(m/s) | s | n |
| 0.230 | 0.60 | 0.4 | 0 | 0.38 | 0.0016 | 0.025 |

Flow hydraulic barrel

Assuming ø of the pipe = 0.50m

H = 0.80*D = 0.80*0.50 = 0.40

H/r = 0.40/0.25 = 1.60

Refering to table 5.3 from the design manual

∂ = A/r2 = 2.694 	A = 2.694 (0.25)2 = 0.1684m2

p/r = 4.428; P = 4.428 x 0.25 = 1.107m

P = R/r = 0.608    R = 0.608 0.25 = 0.152m

V2 = Q/A = 0.0832/0.19625= 0.42395 = 0.40m/s

Hv2 = V22/2g = (0.40)2/19.62 = 0.0081

L2 = [nv/R2/3]2 = [(0.025*0.40)/(0.152)2/3]2 = 0.00123

Difference in elevation Between Elevation B and Elevation C

hf = L.L2 = 5*0.00123 = 0.00616m

C. Water surface change at the u/s and D/s of the culvert

Drop of water surface at the inlet of the culvert

∆h1 = 15[V22-V12/2g] = 1.5[(0.40)2-(0.38)2/19.62] = 0.0012m

Rise of water surface at the D/s of the canal section

∆h2 = 0.3 [(v22-v12)/2g] = 0.3[(0.402-0.382)/19.62] = 0.00024m

d- Elevation

Inlet elev. at u/s canal =1517.260m

W. s.  Elev. at u/s canal= Inlet elev. at u/s canal +H=1517.260+0.40 = 1517.660m

W. s. at the culvert inlet = (W.S. Elev. At u/s canal)-∆h1 = 1517.660-0.0012 = 1517.6588m

Env- Elev. at the canal inlet = (W.S. at the culvert inlet) – d2

= 1517.6588 – 0.40 = 1517.2588m

Env. Elev. at the culvert out let = (Inv. Elev. at the canal inlet)- hf

= 1517.259 – 0.00616 = 1517.253m

W.S Elev. at the culvert outlet = (Inv. Elev. at the culvert outlet)+d2

= 1517.253 +0.40 = 1517.6526m

W.S Elev. at the D/S canal = (W.S. Elev. at the culvert out let)+∆h2

= 1517.6526+0.00024 = 1517.6529m

Inv. Elev. at the D/s canal = (W. S. Elev. at the D/s) – d2

= 1517.6529– 0.40 = 1517.253m

e. Total head loss,HT = 1.2 [(V22-V12)/2g]+hf

HT = 1.2 [(0.40)2-(0.380)2/19.62]+0.00616 = 0.189m

Check water surface difference between u/s and D/s canal

∆Z = (W.S. Elev. at u/s canal) – (W.S- Elev. at D/s canal)

∆Z    = 1517.660 -1517.6529= 0.0071m

3. Bill of Quantities and specifications for the construction of Ukke SSIP

| No | Description of work | Unit | Quantity | Unit cost | Revised Total cost |
| --- | --- | --- | --- | --- | --- |
| 1 | General preparatory works |  |  |  |  |
| 1.1 | Allow for Mobilization (Machineries, material, labor, etc) | Ls | 1 | 60,000.00 | 60,000.00 |
| 1.2 | Allow for Demobilization after finalizing the whole project activities | LS | 1 | 40,000.00 | 40,000.00 |
| 1.3 | Camping (3m x 12m office & bed room, 3m x 8m bed room & kitchen, 5m x 5m store, 3mx2m Toilet & Shower, 2m x 2m guard house |  |  |  |  |
| 1.3.1 | Site clearing | m2 | 160 | 9 | 1,440.00 |
| 1.3.2 | Excavation | m3 | 45 | 35 | 1,575.00 |
| 1.3.3 | Masonry works of mix ratio 1:4 | m3 | 60 | 1,100.00 | 66,000.00 |
| 1.3.4 | Hard core | m2 | 110 | 450 | 49,500.00 |
| 1.3.5 | 5cm Cement screed | m2 | 110 | 300 | 33,000.00 |
| 1.3.6 | Walling(C.I.S) | m2 | 215 | 450 | 96,750.00 |
| 1.3.7 | Roofing(C.I.S) | m2 | 135 | 450 | 60,750.00 |
| 1.3.8 | Fence 2.0m height & 15cm φ eucalyptus poles placed every 2m with barbed wire at 20cm vertical interval & posted in 0.6m depth embedded with concrete | ml | 160 | 40 | 6,400.00 |
| 1.4 | Access Road |  |  |  |  |
| 1.1 | Access Road Maintenance cutting to an average depth of 0.5m, with 6m width | km | 7 | 9,000.00 | 63,000.00 |
| 1.2 | Field Road construction cutting to an average | km | 10 | 5,000.00 | 50,000.00 |
|  | Sub Total |  |  |  | 528,415.00 |
| 2 | Head Work |  |  |  |  |
| 2.1 | Weir |  |  |  |  |
| 2.1.1 | structural excavation of weathered rock to a depth not exceeding 0.7m, including disposal<50m | m3 | 125 | 65 | 8,125.00 |
| 2.1.2 | Masonry works of mix ratio 1:3 | m3 | 200 | 1,100.00 | 220,000.00 |
| 2.1.3 | Concrete works of mix ratio 1:2:4 | m3 | 88 | 1,800.00 | 158,400.00 |
| 2.1.4 | Plastering, mix ratio 1:3 | m2 | 175 | 165 | 28,875.00 |
| 2.2 | Right side wing wall |  |  |  |  |
| 2.2.1 | structural excavation of weathered rock to a depth not exceeding 0.7m, including disposal<50m | m3 | 45 | 65 | 2,925.00 |
| 2.2.2 | Masonry works of mix ratio 1:3 | m3 | 200 | 40 | 8,000.00 |
| 2.2.3 | Concrete works of mix ratio 1:2:4 | m3 | 23 | 1,800.00 | 41,400.00 |
| 2.2.4 | Plastering, mix ratio 1:3 | m2 | 96 | 165 | 15,840.00 |
| 2.3 | Left side wing wall |  |  |  |  |
| 2.3.1 | structural excavation of weathered rock to a depth not exceeding 0.7m, including disposal<50m | m3 | 34 | 65 | 2,210.00 |
| 2.3.2 | Masonry works of mix ratio 1:3 | m3 | 171.7 | 1,100.00 | 188,870.00 |
| 2.3.3 | Concrete works of mix ratio 1:2:4 | m3 | 18 | 1,800.00 | 32,400.00 |
| 2.3.4 | Plastering, mix ratio 1:3 | m2 | 85.25 | 165 | 14,066.25 |
| 2.4 | Stilling Basin |  |  |  |  |
| 2.4.1 | structural excavation of weathered rock to a depth not exceeding 0.7m, including disposal<50m | m3 | 101 | 65 | 6,565.00 |
| 2.4.2 | Masonry works of mix ratio 1:3 | m3 | 285.25 | 1,100.00 | 313,775.00 |
| 2.4.3 | Concrete works of mix ratio 1:2:4 | m3 | 34 | 1,800.00 | 61,200.00 |
| 2.5 | Up & D/s cutoff Walls |  |  |  |  |
| 2.5.1 | structural excavation of weathered rock to a depth not exceeding 0.7m, including disposal<50m | m3 | 44 | 65 | 2,860.00 |
| 2.5.2 | Masonry works of mix ratio 1:3 | m3 | 26 | 1,100.00 | 28,600.00 |
| 2.5.3 | Concrete works of mix ratio 1:2:4 | m3 | 8 | 1,800.00 | 14,400.00 |
| 2.6 | Gate Structures |  |  |  |  |
| 2.6.1 | Supply & installation of steel intake gate(60cm*60cm) including steel works, angle iron, sliding shaft as shown in the Drawing | pcs | 1 | 25,000.00 | 25,000.00 |
| 2.6.2 | Supply & installation of steel Undersluice gate(200cm*100cm) including steel works, angle iron, sliding shaft as shown in the Drawing | pcs | 1 | 30,000.00 | 30,000.00 |
| 2.6.3 | Purchasing, transporting, and laying of plain concrete pipes(Ø50cm) | m | 18 | 350 | 6,300.00 |
|  | Sub Total |  |  |  | 1,209,811.25 |
| 3 | Canals |  |  |  |  |
| 3.1 | MC(Lined-2000m) |  |  |  |  |
| 3.1.1 | Ordinary soil excavation | m3 | 2310 | 55 | 127,050.00 |
| 3.1.2 | Hard core | m2 | 2400 | 450 | 1,080,000.00 |
| 3.1.3 | Masonry works of mix ratio 1:3 | m3 | 720 | 1,100.00 | 792,000.00 |
| 3.1.4 | Concrete works of mix ratio 1:3:6 | m3 | 120 | 1,800.00 | 216,000.00 |
| 3.1.5 | Plastering, mix ratio 1:3 | m2 | 3600 | 165 | 594,000.00 |
| 3.1.6 | Backfill | m3 | 475 | 45 | 21,375.00 |
| 3.2 | MC(Earthen-3000m) |  |  |  |  |
| 3.2.1 | Ordinary soil excavation | m3 | 2500 | 19.08 | 47,700.00 |
| 3.2.2 | Backfill | m3 | 950 | 19.08 | 18,126.00 |
| 3.3 | SC-1 (718m) |  |  |  |  |
| 3.3.1 | Ordinary soil excavation | m3 | 650 | 55 | 35,750.00 |
| 3.3.2 | Hard core | m2 | 320 | 450 | 144,000.00 |
| 3.3.3 | Masonry works of mix ratio 1:3 | m3 | 126 | 1,100.00 | 138,600.00 |
| 3.3.4 | Concrete works of mix ratio 1:3:6 | m3 | 22 | 1,800.00 | 39,600.00 |
| 3.3.5 | Plastering, mix ratio 1:3 | m2 | 540 | 165 | 89,100.00 |
| 3.3.6 | Backfill | m3 | 140 | 35 | 4,900.00 |
| 3.4 | SC-2(1265m) |  |  |  |  |
| 3.4.1 | Ordinary soil excavation | m3 | 1150 | 55 | 63,250.00 |
| 3.4.2 | Hard core | m2 | 320 | 450 | 144,000.00 |
| 3.4.3 | Masonry works of mix ratio 1:3 | m3 | 126 | 1,100.00 | 138,600.00 |
| 3.4.4 | Concrete works of mix ratio 1:3:6 | m3 | 22 | 1,800.00 | 39,600.00 |
| 3.4.5 | Plastering, mix ratio 1:3 | m3 | 540 | 165 | 89,100.00 |
| 3.4.6 | Backfill | m3 | 140 | 35 | 4,900.00 |
| 3.5 | SC-3(14400m) |  |  |  |  |
| 3.5.1 | Ordinary soil excavation | m3 | 1360 | 55 | 74,800.00 |
| 3.5.2 | Hard core | m2 | 320 | 450 | 144,000.00 |
| 3.5.3 | Masonry works of mix ratio 1:3 | m3 | 432 | 1,100.00 | 475,200.00 |
| 3.5.4 | Concrete works of mix ratio 1:3:6 | m3 | 22 | 1,800.00 | 39,600.00 |
| 3.5.5 | Plastering, mix ratio 1:3 | m3 | 540 | 165 | 89,100.00 |
| 3.5.6 | Backfill | m3 | 140 | 35 | 4,900.00 |
|  | Sub Total |  |  |  | 4,655,251.00 |
| 4 | Flumes on MC |  |  |  |  |
| 4.1 | Flume, F1 (L=22m) |  |  |  |  |
| 4.1.1 | Structural excavation to a depth not exceeding 2m, including disposal | m3 | 13 | 55 | 715.00 |
| 5.1.2 | Masonry works of mix ratio 1:3 | m3 | 8.5 | 1,100.00 | 9,350.00 |
| 4.1.2 | Concrete works of mix ratio 1:2:4 | m3 | 9.6 | 1,800.00 | 17,280.00 |
| 5.1.3 | Plastering, mix ratio 1:3 | m2 | 3.5 | 165 | 577.50 |
| 4.1.3 | Cemented stone pitching (thickness 30cm) | m2 | 2 | 340 | 680.00 |
| 5.1.4 | supply & preparing of 50mm thick timber for form working | m2 | 19.3 | 2,000.00 | 38,600.00 |
| 4.1.4 | Purchasing, transporting, and cutting of Eucalyptus pole of Ø10cm at each 10m length for form working | pcs | 166 | 35 | 5,810.00 |
| 5.1.5 | Purchasing, transporting, and cutting of Eucalyptus pole of Ø5cm at each 10m length for form working | pcs | 408 | 30 | 12,240.00 |
| 4.1.5 | Supply, cutting, bending, placing, &any other fixing of reinforcement bar : |  |  |  |  |
| a | -Ø6 | kg | 27.98 | 50 | 1,399.00 |
|  | -Ø8 | " | 78.17 | 45 | 3,517.65 |
|  | -Ø10 | " | 154.12 | 45 | 6,935.40 |
| b | -Ø14 | " | 192.95 | 45 | 8,682.75 |
| c | -Ø16 | " | 159.55 | 45 | 7,179.75 |
| d | -Soft Wire(10% of a) | " | 46 | 35 | 1,610.00 |
| e | - Nails | " | 8 | 35 | 280.00 |
| 4.2 | Flume, F2 (L=24m) |  |  |  |  |
| 4.2.1 | Structural excavation to a depth not exceeding 2m, including disposal | m3 | 15 | 55 | 825.00 |
| 4.2.2 | Masonry works of mix ratio 1:3 | m3 | 9.4 | 1,100.00 | 10,340.00 |
| 4.2.3 | Concrete works of mix ratio 1:2:4 | m3 | 9.6 | 1,800.00 | 17,280.00 |
| 4.2.4 | Plastering, mix ratio 1:3 | m2 | 4 | 165 | 660.00 |
| 4.2.5 | Cemented stone pitching (thickness 30cm) | m2 | 2.5 | 340 | 850.00 |
| 4.2.6 | supply & preparing of 50mm thick timber for form working | m2 | 20 | 2,000.00 | 40,000.00 |
| 4.2.7 | Purchasing, transporting, and cutting of Eucalyptus pole of Ø10cm at each 10m length for form working | pcs | 166 | 35 | 5,810.00 |
| 4.2.8 | Purchasing, transporting, and cutting of Eucalyptus pole of Ø5cm at each 10m length for form working | pcs | 408 | 30 | 12,240.00 |
| 4.2.9 | Supply, cutting, bending, placing, &any other fixing of reinforcement bar : |  |  |  |  |
| a | -Ø6 | kg | 27.98 | 50 | 1,399.00 |
|  | -Ø8 | " | 83.89 | 45 | 3,775.05 |
|  | -Ø10 | " | 161.86 | 45 | 7,283.70 |
| b | -Ø14 | " | 206.57 | 45 | 9,295.65 |
| c | -Ø16 | " | 159.55 | 45 | 7,179.75 |
| d | -Soft Wire(10% of a) | " | 52 | 35 | 1,820.00 |
| e | - Nails | " | 10 | 35 | 350.00 |
|  | Sub Total |  |  |  | 233,965.20 |
| 5 | Chute(700m) |  |  |  |  |
| 5.1 | On SC1 |  |  |  |  |
| 5.1.1 | Ordinary soil excavation | m3 | 154 | 55 | 8,470.00 |
| 5.1.2 | Hard core 25 cm thick | m2 | 475.2 | 450 | 213,840.00 |
| 5.1.3 | Lean concrete 7cm thick | m2 | 158.4 | 1,800.00 | 285,120.00 |
| 5.1.4 | Masonry works of mix ratio 1:4 | m3 | 158.4 | 1,100.00 | 174,240.00 |
| 5.1.5 | Plastering, mix ratio 1:3 | m2 | 580.8 | 165 | 95,832.00 |
| 5.1.6 | Backfill | m3 | 39.6 | 35 | 1,386.00 |
| 5.2 | On SC2 (980m earthen) |  |  |  |  |
| 5.2.1 | Ordinary soil excavation | m3 | 63 | 55 | 3,465.00 |
| 5.2.2 | Hard core 25 cm thick | m2 | 252 | 450 | 113,400.00 |
| 5.2.3 | Lean concrete 7cm thick | m2 | 84 | 1,800.00 | 151,200.00 |
| 5.2.4 | Masonry works of mix ratio 1:4 | m3 | 42 | 1,100.00 | 46,200.00 |
| 5.2.5 | Plastering, mix ratio 1:3 | m2 | 308 | 165 | 50,820.00 |
| 5.2.6 | Backfill | m3 | 21 | 35 | 735.00 |
| 5.3 | On SC3 (560m Earthen) |  |  |  |  |
| 5.3.1 | Ordinary soil excavation | m3 | 117 | 55 | 6,435.00 |
| 5.3.2 | Hard core 25 cm thick | m2 | 468 | 450 | 210,600.00 |
| 5.3.3 | Lean concrete 7cm thick | m2 | 156 | 1,800.00 | 280,800.00 |
| 5.3.4 | Masonry works of mix ratio 1:4 | m3 | 78 | 1,100.00 | 85,800.00 |
| 5.3.5 | Plastering, mix ratio 1:3 | m2 | 572 | 165 | 94,380.00 |
| 5.3.6 | Backfill | m3 | 39 | 35 | 1,365.00 |
|  | Sub Total |  |  |  | 1,824,088.00 |
| 6 | Division Box (10 in no) |  |  |  |  |
| 6.1 | Excavation | m3 | 81.62 | 55 | 4,489.10 |
| 6.2 | Masonry works of mix ratio 1:3 | m3 | 52.47 | 1,100.00 | 57,717.00 |
| 6.3 | Plastering, mix ratio 1:3 | m2 | 131.14 | 165 | 21,638.10 |
| 6.4 | Backfill | m3 | 30.36 | 35 | 1,062.60 |
|  | Sub Total |  |  |  | 84,906.80 |
| 7 | Turnouts(96 No) |  |  |  |  |
| 7.1 | Excavation | m3 | 163.64 | 19.08 | 3,122.25 |
| 7.2 | Masonry works of mix ratio 1:3 | m3 | 144 | 565.95 | 81,496.80 |
| 7.3 | Concrete works of mix ratio 1:2:4 | m3 | 15 | 925.46 | 13,881.90 |
| 7.4 | Plastering, mix ratio 1:3 | m2 | 428.8 | 46.78 | 20,059.26 |
| 7.5 | Cemented stone pitching (thickness 30cm) | m2 | 30.44 | 171.66 | 5,225.33 |
| 7.6 | Purchasing, transporting, and laying of plain concrete pipes(Ø20cm) | pcs | 96 | 55 | 5,280.00 |
|  | Sub Total |  |  |  | 129,065.55 |
| 8 | Culvert |  |  |  |  |
| 8.1 | Box Culvert(1 No) |  |  |  |  |
| 8.1.1 | Excavation | m3 | 18.2 | 19.08 | 347.26 |
| 8.1.2 | Masonry works of mix ratio 1:3 | m3 | 15.52 | 565.95 | 8,783.54 |
| 8.1.3 | Concrete works of mix ratio 1:2:4 | m3 | 5.5 | 925.46 | 5,090.03 |
| 8.1.4 | Cemented stone pitching (thickness 30cm) | m2 | 4 | 171.66 | 686.64 |
| 8.1.5 | supply and preparing of 50mm thick timber for form working | m3 | 10.71 | 2,000.00 | 21,420.00 |
| 8.1.6 | Purchasing, transporting, and cutting of Eucalyptus pole of Ø10cm at each 10m length for form working | pcs | 115 | 15 | 1,725.00 |
| 8.1.7 | Supply, cutting, bending, placing, &any other fixing of reinforcement bar : |  |  |  |  |
| a | -Ø12 | kg | 132 | 45 | 5,940.00 |
| b | -Ø10 | kg | 84 | 45 | 3,780.00 |
| c | -Soft Wire | kg | 15 | 35 | 525.00 |
| d | - Nails | kg | 5 | 35 | 175.00 |
| 8.2 | Drainage pipe culvert(6 No) |  |  |  |  |
| 8.2.1 | Excavation | m3 | 15 | 55 | 825.00 |
| 8.2.2 | Masonry works of mix ratio 1:3 | m3 | 8.36 | 1,100.00 | 9,196.00 |
| 8.2.3 | Concrete works of mix ratio 1:2:4 | m3 | 4.36 | 1800 | 7,848.00 |
| 8.2.4 | Plastering, mix ratio 1:3 | m2 | 30 | 165 | 4,950.00 |
| 8.2.5 | Purchasing, transporting, and laying of plain concrete pipes(Ø100cm) | pcs | 14 | 600 | 8,400.00 |
| 8.2.6 | Backfill | m3 | 6 | 35 | 210.00 |
| 8.3 | Bed slab with bar on Mc&Sc(8 No) |  |  |  |  |
| 8.3.1 | Excavation | m3 | 68.09 | 19.08 | 1,299.16 |
| 8.3.2 | Masonry works of mix ratio 1:3 | m3 | 56.15 | 565.95 | 31,778.09 |
| 8.3.3 | Concrete works of mix ratio 1:2:4 | m3 | 10.54 | 925.46 | 9,754.35 |
| 8.3.4 | Plastering, mix ratio 1:3 | m2 | 81.81 | 46.78 | 3,827.07 |
| 8.3.5 | Supply, cutting, bending, placing, &any other fixing of reinforcement bar : |  |  |  |  |
| a | -Ø8 | kg | 595 | 10 | 5,950.00 |
| b | -Soft Wire | kg | 60 | 10 | 600.00 |
|  | Sub Total |  |  |  | 133,110.14 |
| 9 | Drops(6No) on SC-3 |  |  |  |  |
| 9.1 | Excavation | m3 | 36.4 | 55 | 2,002.00 |
| 9.2 | Hard core | m2 | 36.8 | 450 | 16,560.00 |
| 9.3 | Concrete | m3 | 37.2 | 1,800.00 | 66,960.00 |
| 9.4 | Masonry works of mix ratio 1:3 | m3 | 37.6 | 1,100.00 | 41,360.00 |
| 9.5 | Plastering, mix ratio 1:3 | m2 | 38 | 165 | 6,270.00 |
| 9.6 | Backfill | m3 | 38.4 | 35 | 1,344.00 |
|  | Sub Total |  |  |  | 134,496.00 |
| 10 | Cattle cross(8No) |  |  |  |  |
|  | Excavation | m3 | 176 | 19.08 | 3,358.08 |
|  | Masonry works of mix ratio 1:3 | m3 | 72 | 565.95 | 40,748.40 |
|  | Concrete works of mix ratio 1:2:4 | m3 | 32 | 925.46 | 29,614.72 |
|  | Plastering, mix ratio 1:3 | m2 | 72 | 46.78 | 3,368.16 |
|  | Sub Total |  |  |  | 77,089.36 |
|  | Total |  |  |  | 9,010,198.30 |
|  | VAT (15%) |  |  |  | 1,351,529.74 |
| * | Grand Total |  |  |  | 10,361,728.04 |

Bed bars are nothing but frame works constructed on earthen channel having the same hydraulic dimensions as that of the channel.

They are used as a frame work to keep the shape and design bed elevation of the channel during canal cleaning and maintenance. They can be constructed from wood, masonry, concrete, wet stone pitch or any other construction materials.

In this project, bed bars with slab were recommended at all points where the canal crosses foot path so that they can function as a bench mark and also for animal and human crossing. The total length of each bed bar is 2m.