`

Assessment of Soil and Water Salinity in Selected Small-Scale Irrigation Schemes of Ethiopia.

A project Implemented by

The Smallholder Irrigation Development Lead executive office of the MoA and the Ethiopian Agricultural Research Institute (EIAR)

## Editors

Birru Yitaferu W/Tsadik

Zeleke Belay Admasu

Elias Awol

Getaneh Leulseged

Tilahun Hordofa

Fentaw Abegaz

Ashebir Haile

Belay H/Gebriel

Correct Citation: Birru Yitaferu W/Tsadik, Zeleke Belay Admasu, Elias Awol, Getaneh Leulseged, Tilahun Hordofa, Fentaw Abegaz, Ashebir Haile and Belay H/Gebriel (eds.). 2025. Assessment of Soil and Water Salinity problems in Selected Small-Scale Irrigation Schemes of Ethiopia. Smallholder Irrigation Development (SID) Lead Executive, Ministry of Agriculture. Addis Ababa, Ethiopia.

TABLE OF CONTENTS

LIST OF TABLES	v

LIST OF FIGURES	vi

CONTRIBUTORS	vii

LIST OF ABBREVIATIONS AND ACRONYMS	viii

PREFACE	ix

EXCUTIVE SUMMERY	41

1.	INTRODUCTION	44

1.1. Objectives	46

1.2. Scope of the Study	46

2.	GENERAL METHODOLOGY	47

2.1.	Selection of Small-Scale Irrigation Schemes	48

2.2.	Characterization of selected SSI Schemes	48

2.3.	Soil sample collection	49

2.4.	Soil sample preparation	50

2.5.	Water sample collection	50

2.6.	Classification of salt affected soils	50

2.7.	Suitability of water sources for irrigation	51

2.8.	Mapping of salt affected soils	52

2.9.	Soil analysis and data collection	52

2.10.	Water analysis and data collection	52

3.	RESULTS AND DISCUSSION	54

3.1.	The State of Salinity Problems in Small Scale Irrigation Schemes and Possible Mitigation Measures: the Case of Afar Region	56

3.1.1.	Characterization of Selected Irrigation Schemes	56

3.1.2.	Mapping and analysis of surface saline and sodic soils	59

3.1.3.	Classification of salt affected soils	64

3.1.4.	Irrigation Water Quality	65

3.1.5.	Soil management options	67

3.1.6. Water management options	67

3.1.7. Conclusion and Recommendation	69

3.2.	The State of Salinity Problems in Small Scale Irrigation Schemes and Possible Mitigation Measures: the Case of Amhara Region	70

3.2.1. Characterization of selected irrigation schemes	70

3.2.2.	Mapping and analysis of surface saline and sodic soils	73

3.2.3.	Classifaction of salt affected soils	85

3.2.4.	Irrigation water quality	87

3.2.5.	Soil management option	90

3.2.6.	Water management options	91

3.2.7.	Conclusion and Recommendation	91

3.3.	The State of Salinity Problems in Small Scale Irrigation Schemes and Possible Mitigation Measures: the Case of Oromia and Harari Regions and Dire Dawa City Administration.	94

3.3.1.	Characterization of selected irrigation schemes	94

3.3.2.	Mapping and analysis of surface saline and sodic soils	97

3.3.3. Classification of salt affected soil	111

3.3.4 Irrigation water quality	114

3.3.5. Soil management options	119

3.3.6. Water management options	119

3.3.7. Conclusion and Recommendation	120

3.4.	The State of Salinity Problems in Small Scale Irrigation Schemes and Possible Mitigation Measures: the Case of Somali Region	122

3.4.1.	Characterization of selected irrigation schemes	122

3.4.2.	Mapping and analysis of surface saline and sodic soils	124

3.4.3.	Classification of salt affected soils	134

3.4.4.	Irrigation water quality	135

3.4.5.	Soil management options	138

3.4.6.	Water management options	139

3.4.7.	Conclusion and Recommendation	140

3.5.	The State of Salinity Problems in Small Scale Irrigation Schemes and Possible Mitigation Measures: the Case of EX- SNNRP Region.	142

3.5.1. Characterization of selected irrigation schemes	142

3.5.2. Mapping and analysis of surface saline and sodic soils	144

3.5.3. Classifications of salt affected soils	151

3.5.5. Irrigation water quality	152

3.5.6. Soil mangement options	156

3.5.7. Water management options	157

3.5.8. Conclusion and Recommendation	158

Conclusion	158

Recommendation	158

4.	CONCLUSIONS AND RECOMMENDATIONS	160

4.1. Conclusions	160

4.2. Recommendations	163

5.	REFERENCES	165

Annexes	170

# LIST OF TABLES

Table 2-1 Schemes planned, selected and studied across the Regions	48

Table 2-2 Visual Characterization of salt affected soils	49

Table 2-3 Classification of salt affected soils	51

Table 2-4 Guidelines for interpretation of water quality for irrigation	51

Table 3-1 Schemes studied and percent salt affected soils across the regions.	54

Table 3-2  Quality of irrigation water sources across the regions.	55

Table 3-3 Preliminary assessment of salt-affected soils for SSI Schemes	59

Table 3-4 Percent area coverage of surface saline soils in selected irrigation schemes	60

Table 3-5. Percent area coverage of surface sodic soils in selected irrigation schemes s.	62

Table 3-6 Salt affected soil classes in selected irrigation schemes	64

Table 3-7 Salinity hazard of irrigation water for selected irrigation schemes	65

Table 3-8 Sodicity hazard of irrigation water for selected irrigation schemes.	66

Table 3-9. Soil management options for salt affected irrigation schemes	67

Table 3-10. Water management options based on the severity level of irrigation water quality	68

Table 3-11 Description of selected SSI schemes	71

Table 3-12 Percent area coverage of surface saline soils in selected irrigation schemes	76

Table 3-15. Salinity hazard of irrigation water for selected irrigation schemes	89

The SAR value of the irrigation water ranged from 0.03 to 42.95% and the ECe value of irrigation water ranged from 3.42 to 0.17 dS/m (Table 3-16). The sodium hazard of the irrigation water used for 23 irrigation schemes were slight to moderate and require amendments such as gypsum While for 10 irrigation schemes, the irrigation water were safe for irrigation purpose. The irrigation water used from Tana River inr Dengora irrigation scheme was severe and cannot be used without blending with high saline water.	90

Table 3-16 Sodicity hazard of irrigation water for selected irrigation schemes	91

Table 3-17. Salt affected soil management options	92

Table 3-18  water management options	92

Table 3-19. Selected irrigation schemes	98

Table 3-20. Percent area coverage of surface saline soils in selected irrigation schemes	99

Table 3-21. Percent area coverage of surface sodic soils in selected irrigation schemes.	104

Table 3-22. Salt affected soil classes in selected irrigation schemes	112

Table 3-23. Salinity hazard of irrigation water for selected irrigation schemes	115

Table 3-24  Sodicity hazard of irrigation water for selected irrigation schemes	117

Table 3-25. Soil Management Options.	118

Table 3-26. Water Management Options.	119

Table 3-27. Percent area coverage of surface saline soils in selected irrigation schemes	126

Table 3-28. Percent area coverage of surface sodic soils in selected irrigation schemes	129

Table 3-29. Salt affected soil classes in selected irrigation schemes	132

Table 3-30.Salinity hazard of irrigation water for selected schemes	135

Table 3-31.Sodicity hazard of irrigation water for selected schemes	136

Table 3-32. Proposed Soil management options for each affected scheme	137

Table 3-33. Water management options	137

Table 3-34. Selected irrigation schemes	141

Table 3-35. Percent area coverage of surface saline soils in selected irrigation schemes.	143

Table 3-36. Sodic soil in selected irrigation schemes	146

Table 3-37. Classifications of salt affected soils	149

Table 3-38. Salinity hazard of irrigation water for selected schemes.	151

Table 3-39.  Sodicity hazard of irrigation water for selected schemes	153

Table 3-40. Soil management options.	154

Table 3-41. Water management options	154

# LIST OF FIGURES

Figure 2-1 Schematic representation of the general methodology for the study	47

Figure 3-1 Selected irrigation schemes	57

Figure 3-2 Pictures of salt affected soils for selected irrigation schemes.	58

Figure 3-3 Mapping of surface Saline soils for selected irrigation schemes	61

Figure 3-4 Mapping of surface sodic soils for selected irrigation schemes.	63

Figure 3-5 Selected irrigation schemes	70

Figure 3-6. Pictures of salt affected soils from selected irrigation schemes.	73

Figure 3-7. Mapping of surface soil salinity for selected irrigation schemes	80

Figure 3-8 Mapping of surface soil sodicity for selected irrigation schemes.	85

Figure 3-9. Selected irrigation schemes.	97

Figure 3-10. Mapping of surface saline soils for selected irrigation schemes	103

Figure 3-11. Mapping of surface sodic soils for selected irrigation schemes	110

Figure 3-12 Selected irrigation schemes.	123

Figure 3-13 Mapping of surface saline soils for selected irrigation schemes	128

Figure 3-14 Percent area coverage of surface sodic soils in selected irrigation schemes	131

Figure 3-15.  Selected irrigation schemes.	141

Figure 3-16. Sample pictures from South Omo irrigation Schemes.	142

Figure 3-17. Maping of saline soil in selected irrigation scheme	145

Figure 3-18 Maping of sodic soil in selected irrigation schemes	148

# CONTRIBUTORS

| Dr Birru Yitaferu | EARCS/EIAR Coordinator |
| --- | --- |
| Zeleke Belay | MoA SSID Project Leader |
| Elias Awol | MoA SSID Director |
| Zenabu Akele | Afar Patoral and Agro Pastoral Research Institute |
| Sisay Dessale | Amhara Agriculture Research Institute |
| Aemro Wale | Amhara Agriculture Research Institute |
| Tsegaye Getachew | Amhara Agriculture Research Institute |
| Solomon Wondatir | Amhara Agriculture Research Institute |
| Melaku Hunie | Amhara Agriculture Research Institute |
| Bayan Ahmed | Oromia Agricultural Research Institute |
| Jemal Nur | Oromia Agricultural Research Institute |
| Zelalem Shelemow | Oromia Agricultural Research Institute |
| Demesachew Tadele | Oromia Agricultural Research Institute |
| Tesfaye Gragn | Oromia Agricultural Research Institute |
| Ambase Abomsa | Oromia Agricultural Research Institute |
| Roba Elema | Oromia Agricultural Research Institute |
| Getachaw Hailu | Oromia Agricultural Research Institute |
| Eshetu Ararso | Oromia Agricultural Research Institute |
| Dahir Yosuf Abdi | Somali Region Pastoral and Agro Pastoral Research Institute |
| Mohamed Omer Yusuf | Somali Region Pastoral and Agro Pastoral Research Institute |
| Tamirneh Kifle | Southern Agricultural Research Institute |
| Eng. Muhumed Hiss Osman | Somali Region Pastoral and Agro Pastoral Research Institute |
| Markose Habtewold | Southern Agricultural Research Institute |
| Ibrahim Seid | Southern Agricultural Research Institute |
| Medhin Madebo | Southern Agricultural Research Institute |
| Tagesse Bekele | Southern Agricultural Research Institute |

# LIST OF ABBREVIATIONS AND ACRONYMS

AAIT  			Addis Abeba Institute of Technology

ARARI 		Amhara Regional Agriculture Research Institute

APARI 		Afar Pastoral and Agropastoral Research Institute

BCM			Billion Cubic Meter

Bha			Billion Hectares

BoA			Bureau of Agriculture

BoLANRD		Bureau of Livestock, Agricultural Natural Resource Development (Afar)

cm 		Centimeter

CEC 			Cation Exchane Capacity

EIAR 			Ethiopian Institute of Agricultural Research

EARCS 		Ethiopian Agriculture Research Council Secretariat

ECe 			Electrical Conductivity of Saturated Soil

ECw			Electrical Conductivity of water

EPI  			Ethiopian Policy Institute

ESP 			Exchangeable Sodium Percentage

FAO  			Food and Agriculture Organization of the United Nations

GPS 			Geographical Positioning System

ha 			Hectare

HESB			Harawa-Erer Sub-Basin

HHs 			House Holds

LShRB		Lower Shabelle River Basin

MoWE 		Ministry of Water and Energy

Mha  			Million Hectare

MoA			Ministry of Agriculture

RSC	 		Residuals Sodium Carbonate

SHIS			Small Holders Irrigation Scheme

SAR	 		Sodium Absorption Ratio

SSID 			Small Scale Irrigation and Drainage

SSIDP                         Sustainable Smallholder Irrigation Development Program

SSI		 	Small Scale Irrigation

SNNPR		Souter Nations Nationalities Peoples Region

TDS 			Total Dissolved Salts

US			United States

USSLS		United States Salinity Laboratory Staff

WUI			Water Use Indes

# PREFACE

Agriculture has long been the backbone of Ethiopia, providing livelihoods to over 70% of the population, contributing to 35% of the GDP, and accounting for 80% of the country's export earnings. Despite its ancient roots, spanning over 3,000 years, Ethiopian agriculture remains predominantly traditional and reliant on rain-fed systems, with minimal modernization. The country's agricultural landscape is divided into highlands, where rain-fed farming predominates but faces severe land degradation, and lowlands, which encompass 56% of the land and hold significant potential for large-scale irrigation but are challenged by drainage and salinity issues.

Recognizing the limitations of traditional rain-fed agriculture, the Ethiopian government has prioritized expanding irrigation to address food security challenges. To support this effort, the Ministry of Agriculture (MoA) commissioned a project in 2020 to assess salinity problems in 105 small-scale irrigation schemes across five regions: Afar, Somali, Oromia, SNNPR, Sidama, and Amhara.

Acknowledgement

This initiative would not have been possible without the financing and dedication of the MoA’s Smallholder Irrigation Development Lead Executive (SHIDLE) Office, particularly under the leadership of Ato Elias Awol (LEO of SHIDLE) and the technical and facilitation support of Ato Zeleke Belay.

The Regional Research Institutes, including SoRPARI, APARI, OARI, SARI, and ARARI, played a crucial role, with contributions from numerous researchers and experts. Special thanks are extended to Dr.Tilahun Hordofa, Dr. Fentaw Abegaz, Dr. Getinet Alemaw, Ato Demeke Nigussie, W/ro Elsa Basilios, Ato Ashebir Haile, and many other contributors. The facilitation and support provided by former EARCS staff, including Dr. Mekuria Tadesse, Ato Abebe Alebu, and Ato Wondosen Dirirsa, were also instrumental. We are deeply grateful to the contributions of all individuals and institutions that made the success of this project possible.

# EXCUTIVE SUMMERY

Agriculture is the backbone of Ethiopia's economy, contributing to 47% of the  GDP, 85% of export earnings, and employing 80% of the rural population. Despite its economic importance, the sector largely depends on subsistence farming, limiting its productivity and resilience to climate variability. Recognizing these challenges, Ethiopia has prioritized the expansion of irrigation systems to enhance agricultural output, improve food security, and reduce reliance on rainfall.

The total irrigation potential of Ethiopia is estimated to be between 5.1 to 7.9 million hectares. However, the actual irrigated area is only about 950,000 to 1.2 million ha, which shows that only a fraction of the potential has been utilized. However, irrigation has yet to make a meaningful impact on food security or youth employment due to infrastructure and management challenges. To address this, the government has launched initiatives focusing on small-scale irrigation schemes managed by smallholder farmers. The "National Smallholder Irrigation and Drainage Strategy" aims to revolutionize agriculture by ensuring reliable water access for smallholders, boosting productivity, enhancing resilience to climate change, and fostering sustainable growth. This vision underscores the critical role of irrigation in securing household food security and increasing incomes while driving national development.

In Ethiopia, about 44 million ha (36% of the total land area) is potentially susceptible to salinity problems of which 11 million ha have already been affected by different levels of salinity and mainly concentrated in lowland regions such as the Denakil Plain, Wabe Shebelle Basin, Rift Valley, and other areas. Over time, the proportion of salt-affected lands has increased from 6% to 16% (11 Mha), particularly in the Rift Valley, where higher evapotranspiration rates intensify salinization. Ethiopia is first in Africa and seventh in the globe in terms of the size of salt affected soils caused by both human activity and natural sources. Restoration of saline soils is costly and challenging due to limited data, inadequate research, and institutional gaps.

The National Smallholders Irrigation and Drainage Strategy aims to promote sustainable irrigation and drainage practices, focusing on smallholder farmers. The aim of this project was to generate baseline information on extent and spatial distribution of salt affected for selected SSI schemes in Ethiopia and provide problem-based mitigation measures. A key component involves assessing the extent of environmental damage caused by poor water management in irrigated lands and implementing alternative management systems. Efforts have included stakeholder awareness campaigns in pilot studies, and mapping of salt-affected areas in hotspot sites across regions such as Amhara, Oromia, Afar, Somali, and Ex-SNNP Region.

A pilot project launched in March 2021 aimed at assessing salinity and sodicity levels in 100 smallholder irrigation schemes across regions including Oromia, Amhara, Afar, Somali, and Ex-SNNPR. Field surveys and soil sampling were conducted between January and May 2021. Initially, the plan was to assess 95 schemes , however, considering the magnitude of the problem, the importance of such studies and the dirth of information on salinity levels of SSI schemes in the regions, a total of 168 schemes were selected, of which 97 were fully characterized. The Oromia Agricultural Research Institute played a major role, exceeding its planned target by selecting 56 schemes and characterizing 32 of them. Similarly, the Amhara Agricultural Research Institute selected 30 schemes and characterized 26, while the Southern Agricultural Research Institute significantly expanded its scope by selecting 47 schemes and characterizing 14. The Somali Pastoral and Agro-Pastoral Research Institute closely aligned with its plan, selecting 20 schemes and characterizing 15. Likewise, the Afar Pastoral and Agro-Pastoral Research Institute achieved its planned target by selecting 15 schemes and characterizing 10. This collaborative effort underscores the extensive scope of the project and the commitment of the institutes to addressing irrigation challenges across Ethiopia.

This study followed a stepwise assessment of salt affected soils and irrigation water sources for the selected schemes starting from desk review at the respective regions, reconnaissance survey for physical characterization of schemes for existence of the salinity problems, followed by detail soil and water sampling and laboratory analysis. Data generated from this study were used to characterize, classify and map soil and water salinity problems into categories; viz. non-saline, saline, saline-sodic, and sodic. The soil salinity problems were also classified and mapped for the individual schemes. Similarly, suitability of water sources for irrigation along with the individual schemes was evaluated using standard guidelines.

Results of the study are presented by regions:

- In Afar Region, the irrigation schemes studied were 10 of which about 56% were non-saline and non-sodic, while 38% sodic, 6% saline-sodic and negligible percetage was saline. The salinity hazard of irrigation water used in eight irrigation schemes were found to be slight to moderate, where as, the irrigation water used in Wa'ama and Hida irrigation schemes was safe with no restriction for use in irrigation. The sodicity hazard of irrigation water used in nine irrigation schemes were found to be slight to moderate, whereas the irrigation water used in Gumeli irrigation scheme was safe.

- In Amhara Region, the irrigation schemes studied were 26 of which about 66% were non-saline and non-sodic, while 3% saline, 10% saline-sodic, and 21% sodic soils. The salinity hazards of irrigation water used in 19 irrigation schemes were found to be safe, whereas in 14 irrigation schemes, the water quality was slightly to moderate. The irrigation water used in Charie irrigation scheme was from spring and the quality was severe. The sodium hazard of the irrigation water used from 23 sources were slight to moderate and while the 10 sources were safe, whereas the irrigation water used from Dengora.

- In Oromia region, the irrigation schemes studied were 32 of which about 19% were non-saline and non-sodic, while 16% saline, 18% saline-sodic, and 47% sodic soils. The salinity hazard of irrigation water used for 13 irrgation schemes were found to be safe and 16 were slight to moderate while seven were sever. The sodium hazard of the irrigation water used from 19 sources was severe,whereas from five sources was slight to moderate and from 12 sources were safe.

- In Somali Region, the irrigation schemes studied were 15 of which about 33% were non-saline and non-sodic, while 44% saline, 19% saline-sodic, and 4% sodic soils. The salinity hazard of irrigation water used in all 15 schemes was found to be safe. The sodium hazard of irrigation water used from 13 sources was slightly to moderate, whereas irrigation water used at Arabi-1 and Arabi-2 were sever.

- In Southern regions, the irrigation schemes studied were 14 of which about 15% were non-saline and non-sodic, while 11% saline, 7% saline-sodic, and 67% sodic soils. The salinity hazard of irrigation water used from 15 point sources were severe, whereas 13 point sources were slight to moderate and from seven point sources were found to be safe.  The sodium hazard of irrigation water used from 27 point sources was safe, whereas irrigation water used from eight sources was slightly to moderate.

The study proposed remedial measures for soil and water salinity problems encountered in the 97 irrigation schemes from five regions, including Harari and Dire Dawa administration. The use of leaching management practices, salt tolerant crops, pre-planting leaching and improved irrigation water management practices were some of the suggested practices whilst crop cultivation on slightly to strongly saline soils. In very strongly to extremely saline soils, reclamation with heavy leaching program was suggested. The use of chemical amendment including gypsum with leaching management practices was suggested practices, whilst crop cultivation and adding organic matter in slight to moderately sodic soils has been suggested to improve the soil permeability. In strongly to very strongly sodic soils, reclamation/ rehabilitation program with appropriate and economical use of chemical amendments and organic matter to improve permeability has been suggested. In saline-sodic soils, the use of chemical amendments including gypsum was suggested before undertaking leaching practices.

# INTRODUCTION

Agriculture is the backbone of the Ethiopian economy, contributing approximately to 47% of the GDP, 85% of export earnings, and providing employment to 80% of the rural population (Welteji, 2018). Despite its significant role in the national economy, the sector predominantly relies on subsistence farming. To address this challenge and enhance agricultural productivity, expanding irrigation systems has become a critical priority. Such expansion is essential for sustainably intensifying agricultural output and improving food security (Schmitter et al., 2018).

The total irrigation potential of Ethiopia is estimated to be between 5.1 to 7.9 million hectares (Awulachew et al., 2010; FAO, 2016; MoA, 2020). However, the actual irrigated area is only about 950,000 to 1.2 million ha, which shows that only a fraction of the potential has been utilized (MoA, 2020; World Bank, 2021). However, realizing this potential requires addressing critical challenges, including the development of infrastructure, efficient water resource management and the implementation of sustainable irrigation practices. The government of Ethiopia is aggressively expanding irrigation development initiatives with particular emphasis on small-scale irrigation schemes that are managed by smallholder farmers. The vision statement of the “National Smallholder Irrigation and Drainage Strategy” is “Transforming agricultural production from its dependence on rain-fed practices by creating a vibrant irrigation sector in which smallholder farmers/agro-pastoralists have access to at least one option of water source to increase production and productivity as well as enhance resilience to climate change and thereby sustain growth, ensure household food security and incomes.”

Meanwhile, salinity, sodicity and water logging are becoming serious problems due to poor irrigation water management practices and lack of proper implementation of drainage practices and continuous use of land for irrigation (Gebremeskel et al., 2018). Of the total land mass, about 11 million hectares (M ha) of land is affected by salinity and sodicity related problems. Of this, 8 M ha suffer from combined salinity and sodicity problems, while 3 M ha are predominantly affected by sodicity (Asad et al., 2019; Tessema et al.,2023). As cited by Sileshi (2016), Ethiopia is first in Africa and seventh in the globe in terms of the size of salt affected soils caused by both human activity and natural sources.

Restoration of salt-affected lands into productive lands and protection of newly developed areas from the spread of salt expantion through improved irrigation and crop management practices is paramount importance.

The National Smallholders Irrigation and Drainage Strategy aims to promote effective and sustainable use and maintenance of irrigation and drainage technologies. Accordingly, one of the foreseen sub-deliverables is to identify the extent of environmental damage in irrigated agricultural land due to poor water management practices, promote and enforce the design and implementation of efficient and cost-effective drainage systems. Efforts so far have included raising awareness among stakeholders about the impact of poor water management, and conducting pilot level assessment of environmental, economic and social consequences. Key outcomes include mapping the extent of salt affected soils in regions like Tigray, Amhara, Oromia, and Afar regions. Similarly, the impacts of soil and irrigation water salinity, poor irrigation practices and farming systems in salt affected areas of selected hotspot sites of Amhara (Kewet), Oromia (Zeway Dugda) and Afar (Amibara and Dubti) regions were identified (Asad et al., 2016). However, these pilot project activities were not fully aligned with the national smallholder irrigation development program. Rather they focused on spot area rehabilitation and management of salt-affected soils to improve agricultural productivity, with minimal emphasis on integrating irrigation and drainage technologies.

Therefore, this project was designed by the Smallholder Irrigation Development Lead Executive Office of MoA entitled as “Assessment of Soil and Water Salinity in Selected Small-Scale Irrigation schemes of Ethiopia” with the aim of generating baseline information on extent and spatial distribution of salt affected areas in selected SSI schemes in Ethiopia and provide problem-based mitigation measures.

The project was launched during the inception workshop held at Bishoftu, in March 2021 and agreed to be implemented between the then Ethiopian Agricultural Research Council Secretariat (EARCS) and MoA.

## 1.1. Objectives

General Objective

The general objective of this project was to generate baseline information on extent and spatial distribution of salt affected soils in selected SSI schemes of Ethiopia.

Specific Objectives

The project had the following specific objectives:

- To assess the extent of soil salinity and sodicity problems.

- To assess salinity and sodicity hazareds of irrigation water.

- To map the spatial distribution of salinity and sodicity problems.

- To recommend general management options.

- To capacitate researchers and development practitioners

Major Activities:

- Conduct desk reviews and consultations to select SSI schemes for assessment

- Perform field assessments and reconnaissance surveys

- Analyze and map salt-affected soils using field and lab data

- Build the capacity of researchers and practitioners

## 1.2. Scope of the Study

This project was implemented in five regions where salinization problems with small scale irrigation schemes (SSIS) were anticipated. The number of SSISs assessed in each region were Oromia (32), Amhara (26), Ex-SNNPR (14), Somali (15) and Afar (10). Field reconnaissance survey and sample collection for soil and water source were done from January to May 2021. These salinity problems assessment was entirely done for lowlands, where SSIS were developed for the smallholder farmers. The study also consists of assessment of the problems and proposed possible solutions that can be managed by the smallholder farmers.

# GENERAL METHODOLOGY

This study followed a structured methodology to assess soil salinity and water quality problems in selected SSIS and to propose appropriate management interventions as shown in Fig. 2-1. The first step involved a desk review, reconnaissance survey, detail survey, characterization and mapping of 97 SSI schemes.

Figure 2-1 Schematic representation of the general methodology for the study

Following this, 97 SSI schemes were selected and characterized based on predefined criteria. Detailed surveys were then undertaken, and soil and water samples collected from the selected sites. The samples were analyzed for key parameters, including Electrical Conductivity of the soil (ECe), Irrigation Water Quality (ECiw), Sodium Adsorption Ratio (SAR), Exchangeable Sodium Percentage (ESP), and pH levels. The results of the analysis were used to characterize, classify and map salinity problems into three categories viz.  Saline, Saline-Sodic, and Sodic.

## Selection of Small-Scale Irrigation Schemes

Selection of the SSI schemes for the study was conducted in collaboration with Bureaus of Agriculture (BoA) of each Region. The SSI schemes Data bases of the respective regions were used to identify and select the schemes. Initially a total of 168 schemes were selected based on their location in the lowlands of Afar, Amhara, Oromia, Somalia and South regions.  Considering the time and available resources, the number of schemes was reduced to 97. In all cases, the selection of SSI schemes was done by the respective regions. The SSI schemes selected for the assessment are listed in (Table 2-1).

Table 2-1 Schemes planned, selected and studied across the Regions

| No. | Institute | Schemes Planned | Schemes selected for preliminary characterization | No. of schemes characterized and amended for further studies |
| --- | --- | --- | --- | --- |
| 1 | Afar Pastoral and Agro pastoral Research Institute (APARI) | 10 | 15 | 10 |
| 2 | Amhara Agricultural Research Institute | 25 | 30 | 26 |
| 3 | Oromia Agricultural Research Institute | 30 | 56 | 32 |
| 4 | Somali Pastoral Agro pastoral Research Institute (SoPARI) | 15 | 20 | 15 |
| 5 | Southern Agricultural Research Institute | 15 | 47 | 14 |
|  | Total | 95 | 168 | 97 |

## Characterization of selected SSI Schemes

Detailed protocols were developed to collect important field data for the reconnaissance surveys including the criteria indicated in Table 2-2. The surveys were conducted in collaboration with the various stakeholders. Based on the results of the surveys, 168 schemes in the project target areas were selected out of which 97 were assessed.

GPS, Auger, Hoe, Spade, Knife, Hammer, Canvas to mix composite samples, Plastic sample bags, Plastic bottle, Sensitive balance, graduated cylinder or glass tube, Nylon bag, Ruler and meter were used during the reconnaissance surveys.

The preliminary investigation involved the characterization and description of schemes using various   parameters such as family size of the beneficiary households, the average size of irrigated landholdings, and the topography, including slope and digital elevation models (DEM). Additional details included the length of the cropping pattern, growing period, the estimated command area, and the production system. The number of irrigation users or beneficiaries, the source of irrigation water (e.g., river, lake, spring, or groundwater), and water distribution and management system, type of irrigation system (modern or traditional) were also documented. Furthermore, the irrigation water application methods such as surface, drip, and sprinkler were examined, along with the major soil types within the targeted schemes. This process also included observing salt-affected schemes in the field and prioritizing salt-affected hotspot areas.

Table 2-2 Visual Characterization of salt affected soils

| Problem | Symptoms |
| --- | --- |
| Saline | White crust on soil surface. Water-stressed plants. Leaf tip burn. |
| Sodic | Poor drainage. Black powdery residue on soil surface. black oily soil |
| Saline-Sodic | Grey-colored soil. Plants showing water stress. |
| Source: Waskom et al., 2010. |  |

## Soil sample collection

Grid-based surface soil samples were collected (0–30 cm depth) to assess surface soil salinity across all selected irrigation schemes in the five regions. Composite soil samples were prepared for each sampling point. For investigating vertical salt distribution, a fresh soil profile was opened in each selected site. One to three sampling pits were opened for each selected scheme. Soil samples were collected from each opened profile at 30 cm intervals, from the surface down to a depth of 120 cm, to characterize the salinity status and dynamics along the soil profile. The pits for sub-surface soil samples were opened in the field by considering the heterogeneity and the size of the irrigation schemes of the field.

The collected soil samples were bagged, properly labeled, and transported to the analytical laboratory for detailed physical and chemical property analysis. All the sampling points for both surface soil and soil pit sampling were navigated and georeferenced using a GPS.

## Soil sample preparation

After sample collection, the soil was delivered to the laboratory with appropriate labeling. The first step in the preparation process was drying, which was carried out by air-drying the samples at room temperature in a well-ventilated area for 24–48 hours while avoiding direct sunlight and heat sources. Next, grinding and sieving were performed by crushing large clumps and removing debris such as stones and roots. The dried soil was then passed through a 2 mm sieve to achieve uniformity. Finally, homogenization was conducted by thoroughly mixing the sieved soil to ensure a representative sample for analysis.

## Water sample collection

For each selected irrigation scheme, water samples were taken from all possible irrigation water sources (ground water, springs, rivers, lakes and run-off). The water samples were collected at the scheme's head points and each of the water sampling coordinate points was geo-referenced. Generally, the collection and handling of irrigation water samples were done in accordance with the procedure outlined by Richards, (1954).

## Classification of salt affected soils

The soil salinity problems were classified into the different salt affected soil classes based on the criteria established by Richards (1954) as given in Table 2-3.

Table 2-3 Classification of salt affected soils

| Salt affected soils type | Electrical conductivity of saturation extracts (ECe) at 25 oC (mmhos/cm) | Exchangeable sodium percentage (ESP) | Soil pH | Description |
| --- | --- | --- | --- | --- |
| Saline | > 4 | < 15 | <8.5 | Non-sodic soils containing sufcient soluble salts to interfere with growth of most crops. |
| Saline-sodic | > 4 | > 15 | <8.5 | Salt with sufficnet exchangeable sodium to interfere with growth of most plants and containing appricable contits of soluble salts. |
| Sodic | < 4 | > 15 | >8.5 | Soils with sufficient exchangeable sodium to interfere with growth of most plants. But without appreciable quantities of soluble salts. |

Source: Richards (1954)

## Suitability of water sources for irrigation

Suitability of water sources for irrigation was evaluated using the FAO Irrigation and Drainage paper No. 29 as indicated in Table 2-4. This sequence may be preceded with the development of a new soil map or digitalization of maps where necessary (Khitrov et al., 2009; Mandal et al., 2011).

Table 2-4 Guidelines for interpretation of water quality for irrigation

| Potential Irrigation Problem |  |  | Units |  |  | Degree of Restriction on Use |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | None | Slight to Moderate | Severe |
| Salinity (affects crop water availability) |  |  |  |  |  |  |  |  |
|  | ECw |  | dS/m |  |  | < 0.7 | 0.7 – 3.0 | > 3.0 |
|  | (or) |  |  |  |  |  |  |  |
|  | TDS |  | mg/l |  |  | < 450 | 450 – 2000 | > 2000 |
| Infiltration (affects infiltration rate of water into the soil. Evaluate using ECw and SAR together) |  |  |  |  |  |  |  |  |
| SAR | = 0 – 3 | And ECw |  | = |  | > 0.7 | 0.7 – 0.2 | < 0.2 |  |
|  | = 3 – 6 |  |  | = |  | > 1.2 | 1.2 – 0.3 | < 0.3 |  |
|  | = 6 – 12 |  |  | = |  | > 1.9 | 1.9 – 0.5 | < 0.5 |  |
|  | = 12 – 20 |  |  | = |  | > 2.9 | 2.9 – 1.3 | < 1.3 |  |
|  | = 20 – 40 |  |  | = |  | > 5.0 | 5.0 – 2.9 | < 2.9 |  |

Sources: Ayers and Westcot, (1994)

## Mapping of salt affected soils

The spatial distribution of salt-affected soils in the study area was analyzed by integrating laboratory results into an ArcGIS environment, using their corresponding latitude and longitude coordinates managed in Microsoft Excel. From laboratory point data, the un-sampled surface was predicted and mapped using kriging interpolation technique of ArcGIS (Azmera et al, 2024). Standard spatial mapping of salt affected soil using ArcGIS following the interpolation techniques including Ordinary Kriging and Inverse Distance Weight (IDW) were used to estimate the soil characteristics or salinity classifications at unvisited sites. The map of the preliminary layers of the salt-affected soil indicator, including pH, EC, and ESP, and the predicted final salt-affected soil distribution map was generated.

## Soil analysis and data collection

Following standard test procedures, soil samples were analysed for selected physic-chemical properties. Soil pH and ECe were measured using a digital pH-meter and EC-meter from saturated past extract. Exchangeable bases (Ca2+, Mg2+, K+ and Na+) of the soils were extracted by excess ammonium acetate (1M NH4OAc at pH 7) solution. Exchangeable Ca2+ and Mg2+ in the extracts were determined by atomic adsorption spectrophotometer while exchangeable K+ and Na+ in the extracts were measured by flame photometer (Okalebo et al., 2002). CEC (cmol (+)/kg) of the soils was determined from the ammonium acetate saturated samples through distillation and measurement of ammonium using the modified Kjeldahl procedure as described by Okalebo et al. (2002). Finally, ESP was computed as the percentage of exchangeable Na to the CEC of the soil using Eg. (1).

ESP (%) =Exchangeable sodium (Na+)  CEC  *100                                        (1)

where, concentrations are in cmol (+)/kg of soil.

## Water analysis and data collection

The collected water samples were subjected to the analysis of pH, ECw and dissolved cations (Ca, Mg, Na and K) contents in the laboratory. The ECw and pH of the water samples were measured in the laboratory within 24 hours using conductivity meter and a digital pH meter, respectively (Richards, 1954). Ca and Mg ions were measured using atomic absorption spectrophotometer, whilst Na and K was analysed using flame photometer. Furthermore, Sodium adsorption ratio (SAR) was computed from the measured parameters and the SAR value was calculated using Eq. (2) with its concentrations expressed in meq/l.

SAR=Na+Ca2+ + Mg2+2	                                                             (2)

where, Na+, Ca2+, and Mg2+ are ionic concentration of sodium, calcium and magnesium, respectively in meq/l.

# RESULTS AND DISCUSSION

The results of the assessments conducted in each of the regions are given separately as reported by the respective reginal research institutions. The results include findings from the reconnaissance surveys, characterization of the assessed schemes in terms of salinity, sodicity, and their combined effects, classification of salt-affected soils, and evaluation of both irrigation water quality and the suitability of irrigation water sources. Specific conclusions and recommended management options have been provided for each scheme in its respective region, and general conclusions and recommended management options are also indicated.

Variations in surface soil salinity were observed across the assessed SSI schemes in the lowlands of Ethiopia. Among the regions, Somali (44%) exhibited the most severe salinity problems were as Oromia (16%), and Ex-SNNPR (11%) slightly affected with salinity. Saline-sodic soil issues were most pronounced in Somali (19%) and Oromia (18%), whereas Amhara (10%), Ex-SNNPR (7%) and Afar (6%) regions were affected slightly with saline-sodic soils. Sodicity was notably high in Ex-SNNPR (67%), Oromia (47%), Afar (38%) and Amhara (21%), whereas Somali (4%) region exhibited minimal sodicity level (Table 3.1).

Water quality assessment in the five regions indicates that in Oromia (16%), Amhara (14%), Afar (8%) and Ex-SNNPR (3%) were affected with slightly to moderately salinity hazard. And Oromia (7%), Ex-SNNPR (2%) and Amhara (1%) were affected with severely salinity hazard. The water quality in Amhara (23%), Afar (9%), Oromia (5%), Somali (2%) and Ex-SNNPR (2%) were affected with slightly to moderately sodicity hazard. And Oromia (19%), Somali (13%), Amhara (1%) and Ex-SNNPR (1%) were severely affected with sodicity hazard (Table 3.2).

Table 3-1 Schemes studied and percent salt affected soils across the regions.

| No. | Region | Schemes studied | Non saline - non sodic (%) | Saline (%) | Saline-sodic (%) | Sodic (%) |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | Afar NRS | 10 | 56 | - | 6 | 38 |
| 2 | Amhara NRS | 26 | 66 | 3 | 10 | 21 |
| 3 | Oromia NRS | 32 | 19 | 16 | 18 | 47 |
| 4 | Somali NRS | 15 | 33 | 44 | 19 | 4 |
| 5 | Ex-SNNPR | 14 | 15 | 11 | 7 | 67 |
|  | Total schemes | 97 |  |  |  |  |

Table 3-2  Quality of irrigation water sources across the regions.

| S. No. | Region | Salinity Hazard |  |  | Sodicity Hazard |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Safe | Slight to moderate | Severe | Safe | Slight to moderate | Severe |
|  |  | No. Water sources |  |  |  |  |  |
| 1 | Afar | 2 | 8 | - | 1 | 9 | - |
| 2 | Amhara | 19 | 14 | 1 | 10 | 23 | 1 |
| 3 | Oromia | 13 | 16 | 7 | 12 | 5 | 19 |
| 4 | Somali | 15 | - | - | - | 2 | 13 |
| 5 | Ex-SNNPR | 1 | 3 | 2 | 5 | 2 | 1 |

## The State of Salinity Problems in Small Scale Irrigation Schemes and Possible Mitigation Measures: the Case of Afar Region

Zenabu Akele and Mohamed Abdulletif

### Characterization of selected irrigation schemes

During the desk review, the analysis focused on Small-Scale Irrigation Schemes (SSIs) established in the last decade, which are currently grappling with significant salinity issues. Consultations were carried out and data for the assessment were sourced from various regional institutions, including the Bureau of Livestock, Agricultural and Natural Resource Development (BoLANRD) and woreda (district) offices, Water, Irrigation, and Energy Bureau, Sustainable Smallholder Irrigation Development Program (SSIDP) and Semera University.

To foster collaboration and prevent research duplication, a workshop was held at Afar Pastoral and Agro-Pastoral Research Institute (APARI) in Semera, in partnership with Semera University. This platform facilitated knowledge sharing and stakeholder alignment. Semera University presented ongoing  research on salt-affected soils in the Lower Awash Valley and other Woredas of the Afar Region, highlighting the classification and impact of saline, saline-sodic, and sodic soils on agriculture and livelihoods. Extreme salinity affects about 15% of the region’s soils, exacerbating food insecurity and economic instability for local farmers. As noted from Daba et al. (2020) report and others, these challenges stem from poor irrigation practices, inadequate drainage, and the arid climate conditions in the region. The mismanagement of large-scale irrigation schemes, particularly in the Awash River Basin, has further accelerated land degradation and reduced agricultural productivity. The findings from Semera University closely aligned with this assessment’s objectives, which also examined SSIs affected by salinity. To ensure complementarity and avoid redundancy, this report focuses on irrigation schemes not covered by Semera University’s research, expanding the understanding of salinity challenges and informing sustainable soil and water management strategies.

Reconnaissance survey was conducted by a core team of 10 members, including staff from nearby Research Centers. The survey was carried out in collaboration with local Woreda and Kebele administrations, agricultural offices, development agents, and community leaders.

Figure 3-1 Selected irrigation schemes

The assessment was carried out in ten SSIs across six Woredas in the Afar Region: as indicated in Fig. 3-1, Udoyita, Mego (Afambo), Meheri, Hinellie (Asayita), Asboda, Debel (Dubti), Wa’ama, Gumeli (Chifra), Sununta (Ewa), and Hida (Awra) were selected based on pre-established criteria.

The selected SSI schemes were shown in Table 3-3. The elevation varies from 346 m.a.s.l in Afambo at Mego scheme to 1019 m.a.s.l in Chifra at Wa’ama scheme (Fig. 3-2). In all SSIDs except at Sununta scheme which uses ground water, the source of irrigation water was the Awash River. Ecologically, the study area is classified between arid and semi-arid. Maize is the most dominant crop cultivated; however, other crops such as sorghum, mung bean, sesame, cotton, onion, tomato, pepper, watermelon, mango, banana, papaya and lowland irrigated wheat are becoming popular.

According to the perceptions of the agro-pastoralists in the area, flood irrigation and water logging were the major causes for the soil salt problems that lead to stunted growth; leaf burns and water stress and yield reduction (Fig. 3-2). The irrigation scheme at Meheri, Hinellie, Mego, Udoyita Sununta, and Wa’ama have symptoms of soil crust, poor drainage, black powdery residue on the soil surface and black oily soil as a result of long-time flood irrigation. Abandoned irrigated areas can be seen near the river with black oily symptoms (Fig. 3-2). As a result, some schemes were abandoned, while others struggle to produce crops with stunted growth, poor drainage and yield loss. The Asboda, Debel, Meheri, Udoyta, Mego, Wa'ama, and Sununta irrigation schemes have severe salt accumulation while Hinellie scheme did not show any symptoms.

Figure 3-2 Pictures of salt affected soils for selected irrigation schemes.

Table 3-3 Preliminary assessment of salt-affected soils for SSI Schemes

| S. N | Scheme | Longitude (UTM) | Latitude (UTM) | Altitude (m) | Source of water | Planned command area (ha) | Actual irrigable area (ha) | Household using scheme (No) | Year of starting irrigation (E.C) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Asboda | 727115 | 1303203 | 364 | River | 50 | 50 | 100 | 2001 |
| 2 | Debel | 728760 | 1304863 | 369 | River | 26 | 26 | 26 | 2003 |
| 3 | Meheri | 768441 | 1275468 | 352 | River | 46.6 | 46.6 | 35 | 1993 |
| 4 | Hinellie | 773686 | 1281758 | 350 | River | 29.85 | 29.85 | 22 | 1997 |
| 5 | Udoyita | 729839 | 1274676 | 346 | River | 32 | 32 | 15 | 1998 |
| 6 | Mego | 784835 | 1273032 | 345 | River | 37 | 37 | 18 | 1996 |
| 7 | Gumeli | 608702 | 1286531 | 927 | River | 200 | 26.3 | 24 | 2007 |
| 8 | Wa’ama | 615938 | 1264089 | 1019 | River | 35 | 16.43 | 13 | 2004 |
| 9 | Sununta | 631574 | 1305689 | 813 | Ground water | 230 | 43.1 | 34 | 2009 |
| 10 | Hida | 605736 | 1318967 | 955 | River | 200 | 62.2 | 130 | 2005 |

### Mapping and analysis of surface saline and sodic soils

Saline Soils

In Afar region, of the total irrigated area of about 370 ha, 76% were found to be none saline soils and the remaining 24% were affected by different levels of saline soils. The different percentage levels affected with saline soils were slightly saline (13%), moderately saline (5%), strongly saline (2%) and very strongly saline (4%) soils. The excessive salts hinder crop growth, not only by toxity but by reducing water availability. Moreover, nutrient uptake may also become unavailable. Hence, fields can only be reclaimed through leaching out salts from the plant root zone and sebsiquently planting salt tolerant crops and use of leaching water as part of irrigation water management practice; can also reduce salt levels in the soils.

The schemes in Asboda, Hinellie, Gumeli, Sunnuta and Hida were not affected by saline soils and hence suitable for cultivation of economically important crops. However, regular monitoring is required to maintain the soil salinity status. The schemes at Debel (7.8%) and Meheri (14%) were affected with slightly saline soils. In these schemes, regular monitoring of salinity status and proper irrigation practices are required to maintain and improve the level of saline soil through leaching activity. The irrigation schemes at Wa’ama were affected with moderately (80%) to strongly (16%) saline soil, at Mego affected slightly (55%) and very strongly (35%) saline soil and Udoyita scheme affected with slightly (56%) to very strongly (4%) saline soil (Table 3-4 and Fig. 3-3).

Table 3-4 Percent area coverage of surface saline soils in selected irrigation schemes

| S. No | Scheme |  | EC (dS/m) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 0-0.75 | 0.75-2 | 2-4 | 4-8 | 8-15 | >15 |
|  |  |  | Non-Saline | Slightly Saline | Moderately Saline | Strongly Saline | V. Strongly Saline | Extremely Saline |
| 1 | Asboda | Area (ha) | 50.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |
|  |  | Area (%) | 100.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |
| 2 | Debel | Area (ha) | 22.67 | 1.92 | 0.00 | 0.00 | 0.00 | 0.00 |
|  |  | Area (%) | 91.86 | 7.78 | 0.00 | 0.00 | 0,00 | 0.00 |
| 3 | Meheri | Area (ha) | 40.08 | 6.29 | 0.00 | 0.00 | 0.00 | 0.00 |
|  |  | Area (%) | 86.01 | 13.50 | 0.00 | 0.00 | 0.00 | 0.00 |
| 4 | Hinellie | Area (ha) | 29.85 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |
|  |  | Area (%) | 100.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |
| 5 | Udoyita | Area (ha) | 0.97 | 17.82 | 6.17 | 5.55 | 1.26.00 | 0.00 |
|  |  | Area (%) | 3.03 | 55.69 | 19.28 | 17.34 | 3.94 | 0.00 |
| 6 | Mego | Area (ha) | 4.35 | 20.39 | 0.00 | 0.00 | 12.98 | 0.00 |
|  |  | Area (%) | 11.76 | 55.11 | 0.00 | 0.00 | 35.08 | 0.00 |
| 7 | Gumeli | Area (ha) | 26.30 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |
|  |  | Area (%) | 100.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |
| 8 | Wa’ama | Area (ha) | 0.62 | 0.00 | 13.14 | 2.60 | 0.00 | 0.00 |
|  |  | Area (%) | 3.77 | 0.00 | 79.98 | 15.82 | 0.00 | 0.00 |
| 9 | Sununta | Area (ha) | 43.1 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |
|  |  | Area (%) | 100 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |
| 10 | Hida | Area (ha) | 62.2 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |
|  |  | Area (%) | 100 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |

Figure 3-3 Mapping of surface Saline soils for selected irrigation schemes

The scheme at Wa’ama affected with strongly (16%) saline soils and yields of many crops are restricted and hence require growing salt tolerant crops with leaching application during regular irrigation application time. The irrigation schemes in Mego (35%) and Udoyita (4%) require heavy leaching program before growing salt tolerant crops. Subsequently, leaching will continue upon growing salt tolerant crops from cereals and forage crops to further improve the saline soil levels. Salts in such condition, mostly efficiently leached out from the crop root zone under higher frequency irrigation (Shorter irrigation interval). Keeping soil moisture levels higher between irrigation events effectively dilutes salt accumulation in the root zone and hence salinity level declines. In slight to moderately affected irrigation schemes, the use of efficient irrigation system, proper irrigation scheduling, correct stream size application, salt tolerant crops and the use of regular leaching of salts with good quality of water are needed for improving and maintaining saline soil levels. In all case, the leached salts need to stay away from germinating seeds and well below root zone of the crop. During irrigation application, distribution uniformity (DU) is very essential. Without very high DU, salts will build up in areas where the germinating seeds and plant roots zone exist and will experience difficulty in attaining better growth and crop yield performance.

Sodic Soils

In Afar region, of the total irrigated area, 25% were found to be none sodic soils and the remaining 75% were affected by different levels of sodic soils. The region was affected with different levels of sodic soils include slightly sodic (7%), moderately sodic (22%), strongly sodic (43%) and extremely sodic (2%) soils. The effect of high sodium has a noticeable deleterious effects on soil structure and hence, requires incorporating soil ammendments

The irrigation schemes in Gumeli and Hida were not affected by sodic soils. However, these schemes require regular monitoring and use of organic matter will help to maintain the soil productivity. The irrigation schemes in Asboda, Meheri and Wa’ama were affected slightly (7.0%, 14% and 11%) to strongly (79%, 30% and 26%) sodic soils, respectively. And irrigation schemes in Hinellie was affected with slightly (42%), and strongly (46%) sodic soils, Debel and Sunnuta were affected with moderately (9% and 10%) to extremely (6% and 1%) sodic soils, and Udoyita and Mego were affected with slightly (1% and 15%) to extremely (20% and 1%) sodic soils respectively (Table 3-5 and Fig. 3-4).

Table 3-5. Percent area coverage of surface sodic soils in selected irrigation schemes s.

| S. No | Scheme |  | ESP (%) |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 0-6 | 6-10 | 10-15 | 15-25 | >25 |
|  |  |  | Non-Sodic | Slightly Sodic | Moderately Sodic | Strongly Sodic | Very Strongly Sodic |
| 1 | Asboda | Area (ha) | 0.00 | 0.35 | 10.04 | 39.53 | 0.00 |
|  |  | Area (%) | 0.00 | 0.70 | 20.08 | 79.06 | 0.00 |
| 2 | Debel | Area (ha) | 0.00 | 0.00 | 2.23 | 20.92 | 1.44 |
|  |  | Area (%) | 0.00 | 0.00 | 9.04 | 84.76 | 5.83 |
| 3 | Meheri | Area (ha) | 0.34 | 6.29 | 25.82 | 13.92 | 0.00 |
|  |  | Area (%) | 0.73 | 13.50 | 55.41 | 29.87 | 0.00 |
| 4 | Hinellie | Area (ha) | 3.21 | 12.64 | 0.00 | 13.84 | 0.00 |
|  |  | Area (%) | 10.75 | 42.35 | 0.00 | 46.37 | 0.00 |
| 5 | Udoyita | Area (ha) | 0.00 | 0.18 | 11.39 | 13.81 | 6.39 |
|  |  | Area (%) | 0.00 | 0.56 | 35.59 | 43.16 | 19.97 |
| 6 | Mego | Area (ha) | 0.00 | 5.57 | 11.14 | 20.99 | 0.02 |
|  |  | Area (%) | 0.00 | 15.05 | 30.11 | 56.73 | 0.05 |
| 7 | Gumeli | Area (ha) | 26.3 | 0.00 | 0.00 | 0.00 | 0.00 |
|  |  | Area (%) | 100.00 | 0.00 | 0.00 | 0.00 | 0.00 |
| 8 | Wa’ama | Area (ha) | 0.00 | 1.75 | 10.36 | 4.25 | 0.00 |
|  |  | Area (%) | 0.00 | 10.65 | 63.06 | 25.87 | 0.00 |
| 9 | Sununta | Area (ha) | 0.52 | 0.00 | 9.71 | 32.01 | 0.54 |
|  |  | Area (%) | 1.21 | 0.00 | 22.53 | 74.27 | 1.25 |
| 10 | Hida | Area (ha) | 62.20 | 0.00 | 0.00 | 0.00 | 0.00 |
|  |  | Area (%) | 100.00 | 0.00 | 0.00 | 0.00 | 0.00 |

Figure 3-4 Mapping of surface sodic soils for selected irrigation schemes

The farm lands affected with slightly and moderately sodic soils did not exceed the critical value of ESP and are suitable for most tolerant crops. Under such level of sodic soils require the use of amendments. The amendments that can be used include gypsum, lime, calcium chloride, magnesium chloride, sulfur and sulfuric acid materials and organic amendments. The most common and economical ammendments used on sodic soils is gypsum, which can be applied dry or with irrigation water. For sodic soils with low EC, saline water may be appropriate for the initial stage of reclamation to provide additional Ca2+ to promote flocculation, and thus increase permeability (Troch et al, 1999).

The farm lands affected with strongly and extremely sodic soils exceed the critical value of ESP affecting soil infiltration, permeability and soil aeration, interfering with growth of most plants. The major concern in extremely sodic soils is the eventual deterioration of soil structure resulting in decreased water infiltration and permeability. This soil requires undertaking reclamation measures with the help of chemical amendments to help facilitate the displacement of the sodium ions and help restore the infiltration capacity of the soil.  Growing salt tolerant crops and the use of chemical amendments, together with irrigation water in excess for leaching are the alternate management practices to improve the soil sodicity level. Irrigation water with very low salinity level can aggravate sodium problem because it allows maximum swelling, dispersions of soil minerals and organic matter affecting water infiltration and permeability. Higher ESP levels could be tolerated in course-textured than fine textured soils. The fine textured soils containing swelling clay are considered indeed impervious, because of having naturally very low infiltration rate and permeability of the soil property. Deep tillage combined with sub-soiling improves the soil structure as it loosens impervious strata in clay soils. Tilahun Hordofa (1991) studied on drainage of heavy clay soil and improved the soil permeability of the soil through restructuring the soil structure at optimal soil moisture level.

### Classification of salt affected soils

All schemes have an appreciable amount of exchangeable sodium ions than soluble ions except Gumeli and Hida irrigation schemes. In the region, of the total irrigated area of about 370ha, 56% were found to be none saline-none-sodic soils, while the remaining were affected by saline (0.2%), saline-sodic (5.8%) and sodic (38%) soils. As compared to other schemes, Udoyita and Mego irrigation schemes have saline-sodic (25% and 28%) and sodic (40% and 28%) soil classes, respectively. But Wa’ama scheme was the only one which has all classes of salt affected soils viz. namely saline (4%), saline-sodic (17%) and sodic (18%). Out of the total irrigated land of Hinellie (6%), Meheri (37%), Debel (91%), Sununta (77%) and Asboda (79%) schemes were affected with only sodic soils (Table 3-6 and Fig. 3-3 and 4).

Table 3-6 Salt affected soil classes in selected irrigation schemes

| S. No. | Scheme Name |  | Classification of salt affected soils |  |  |  | Total |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | ECe < 4 dS/m and ESP < 15 | ECe > 4 dS/m and ESP < 15 | ECe > 4 dS/m and ESP > 15 | ECe < 4 dS/m and ESP > 15 |  |
|  |  |  | None-saline and non-sodic | Saline | Saline sodic | Sodic |  |
| 1 | Asboda | Area (ha) | 10.39 | 0 | 0 | 39.61 | 50 |
|  |  | Area (%) | 20.78 | 0 | 0 | 79.22 | 100 |
| 2 | Debel | Area (ha) | 2.45 | 0 | 0 | 24.01 | 26.46 |
|  |  | Area (%) | 9.3 | 0 | 0 | 90.7 | 100 |
| 3 | Meheri | Area (ha) | 29.49 | 0 | 0 | 17.01 | 46.5 |
|  |  | Area (%) | 63.4 | 0 | 0 | 36.6 | 100 |
| 4 | Hinellie | Area (ha) | 28.13 | 0 | 0 | 1.64 | 29.77 |
|  |  | Area (%) | 94.5 | 0 | 0 | 5.5 | 100 |
| 5 | Udoyita | Area (ha) | 11.45 | 0 | 7.86 | 12.68 | 32 |
|  |  | Area (%) | 35.8 | 0 | 24.6 | 39.6 | 100 |
| 6 | Mego | Area (ha) | 16.71 | 0 | 10.65 | 10.36 | 37.72 |
|  |  | Area (%) | 44.3 | 0 | 28.2 | 27.5 | 100 |
| 7 | Gumeli | Area (ha) | 26.3 | 0 | 0 | 0 | 26.3 |
|  |  | Area (%) | 100 | 0 | 0 | 0 | 100 |
| 8 | Wa’ama | Area (ha) | 9.87 | 0.73 | 2.85 | 2.98 | 16.43 |
|  |  | Area (%) | 60.07 | 4.41 | 17.35 | 18.16 | 100 |
| 9 | Sunuta | Area (ha) | 10.08 | 0 | 0 | 32.87 | 42.95 |
|  |  | Area (%) | 23.5 | 0 | 0 | 76.5 | 100 |
| 10 | Hida | Area (ha) | 62.2 | 0 | 0 | 0 | 62.2 |
|  |  | Area (%) | 100 | 0 | 0 | 0 | 100 |

### Management and reclamation of saline and sodic soils have been discussion in section 3.1.2. Reclamation and management of saline sodic soils require first addressing the excess Na+ problem and then excessive salt problem. This is done through dissolving the amendments in irrigation water or spreading over the land. Therefore, the amendment should be applied to replace Na+, and then excessive water applied to leach the Na+ and the other soluble salts. Establishing salt tolerant crops shortly after reclamation increase the effectiveness of reclamation efforts.

### Irrigation Water Quality

There are two types of salt problems in irrigation water that are very different in their constituent, those associated with the total salinity, designated as salinity hazard and those associated with sodium, designated as sodium hazard. Irrigation water sources used in the different schemes were 9 from river and one from ground water.

Salinity Hazard

Rivers are main sources of water for all irrigation schemes, except Sununta irrigation scheme that uses water from the groundwater.  As shown in Table 3-7, the salinity hazard of Awash River was in the range between 0.57 and 0.88 dS/m and while the salinity hazard of groundwater was 0.75 dS/m. According to Ayers and Westcot (1994), the salinity hazard of irrigation water from the Awash River was slight to moderate, while the irrigation water quality used in Wa’ama and Hida irrigation schemes had no salinity hazard problem. And Sununta groundwater was slight to moderate salinity hazard. (Table 3-7).

Table 3-7 Salinity hazard of irrigation water for selected irrigation schemes

| No | Schemes Name | Water Source | ECw (dS/m) | Degree of restriction for use |
| --- | --- | --- | --- | --- |
| 1 | Asboda | River | 0.84 | Slight to moderate |
| 2 | Debel | River | 0.84 | Slight to moderate |
| 3 | Meheri | River | 0.86 | Slight to moderate |
| 4 | Hinellie | River | 0.88 | Slight to moderate |
| 5 | Udoyita | River | 0.86 | Slight to moderate |
| 6 | Mego | River | 0.86 | Slight to moderate |
| 7 | Gumeli | River | 0.71 | Slight to moderate |
| 8 | Wa’ama | River | 0.62 | None |
| 9 | Sunuta | Ground water | 0.75 | Slight to moderate |
| 10 | Hida | River | 0.57 | None |

The salinity hazard of irrigation water less than 0.75 dS/m is safe for use in growing nearly all crops. For the salinity hazard listed in the slight to moderate range, maximum yield potential could be possible for most salt tolerant crops, but care must be taken to achieve the required leaching fraction (LF) in order to maintain soil salinity within the tolerance of the crop. As the salinity hazard increases within the slight to moderate range, production of the more sensitive crops may be restricted, especially when grown on the heavier, more clayey soil types. Hence, a portion of the added salt must be leached out from the root zone before the concentration of salts affects crop yield.

Sodicity Hazard

The assessment of Awash River water quality resulted in slight to moderate sodicity hazard, except for Gumeli irrigation scheme. To mainatain and improve the sodicity hazard of irrigation water, amendments need to be incorporated in the soil or mixed with the water. The irrigation water used in Gumali irrigation scheme indicates that there is no restriction for use in irrigation. The irrigation water for Sununta irrigation scheme was from groundwater and the degree of restriction was with slight to moderate sodicity hazard (Table 3-8). In order to maintain and improve the sodicity hazard of the irrigation water the use of chemical amendments and/ or organic matter is required,

Table 3-8. Sodicity hazard of irrigation water for selected irrigation schemes.

| No. | Scheme name | Water source | SAR | ECw (dS/m) | Degree of Restriction on Use |
| --- | --- | --- | --- | --- | --- |
| 1 | Asboda | River | 6.92 | 0.84 | Slight to Moderate |
| 2 | Debel | River | 7.78 | 0.84 | Slight to Moderate |
| 3 | Meheri | River | 6.48 | 0.86 | Slight to Moderate |
| 4 | Hinellie | River | 6.27 | 0.88 | Slight to Moderate |
| 5 | Udoyita | River | 6.34 | 0.86 | Slight to Moderate |
| 6 | Mego | River | 6.71 | 0.86 | Slight to Moderate |
| 7 | Gumeli | River | 1.79 | 0.71 | None |
| 8 | Wa’ama | River | 1.38 | 0.62 | Slight to Moderate |
| 9 | Sunuta | Ground water | 4.49 | 0.75 | Slight to Moderate |
| 10 | Hida | River | 2.59 | 0.57 | Slight to Moderate |

The water quality with SAR less than 3% and ECe greater than and equal to 0.75 dS/m indicate a favourable condition for soil wth sufficient calcium and magnesium, which is the case for Gumeli irrigation scheme. Nevertheles, regular monitoring is required to maintain the quality of irrigation water. The ECe-SAR interaction is based on a higher concentration of Ca2+ and Mg2+ being able to counteract the dispersive nature of Na+. Infiltration rate are severely reduced when the ECe is very low, even though SAR may not be excessively high. On the other hand, there may be less of a reduction in infiltration rate when sodicity is coupled with high salinity.

Soil management options

Based on the results indicated in sections 3.1.2 to 3.1.3 analysis and classification possible mitigation measures are given in Table 3-9.

Table 3-9. Soil management options for salt affected irrigation schemes

| Irrigation Schemes | Soil salt classes | Soil management options |
| --- | --- | --- |
| Debel, Meheri, Udoyita, Mego, and Wa’ama | Slightly to strongly saline soils | - Leaching management practices for most sensitive crops. - Cultivation of salt-tolerant crops. |
| Udoyita and Wa’ama | Strogly saline soils | - Leaching management practices - Cultivation of salt-tolerant crops |
| Udoyita and Mego, | Very strongly saline soils | - Cultivation of highly salt tolerant crops and reclamation through heavy leaching management programme. |
| Asboda, Debel, Meheri, Hinellie, Udoyita, Mego, Wa’ama, Sununta | Slightly to moderate sodic soils | - Cultivation most tolerant crops and application of chemical amendments, organic matter and use of saline water. |
| Asboda, Debel, Meheri, Hinellie, Udoyita, Mego, Wa’ama, Sununta | Strongly to very strongly sodic soils | - Reclamation of sodic soils with gypsum application and use of saline water. - Deep ploughing and sub-soiling to break compact layers and improve infiltration. |

### 3.1.6. Water management options

Based on the results indicated in sections 3.1.4 analysis and classification possible mitigation measures are given in Table 3.10.

Table 3-10. Water management options based on the severity level of irrigation water quality

| Irrigation schemes | Water quality challenges | Water management options |
| --- | --- | --- |
| Asboda, Debel, Meheri, Hinellie, Udoyita, Mego, Sununta and Gumeli | Slight to moderate salinity hazard. (0.71–0.88 dS/m) | - Apply leaching practice in every irrigation events as necessary. - Cultivation of salt tolerant crops. - Maintain periodic water quality monitoring. |
| Asboda, Debel, Meheri, Hinellie, Udoyita, Mego, Wa’ama, Sununta and Hida | Slight to moderate sodicity hazard. | - Apply chemical amendments and/or organic matter to improve permeability. - Maintain periodic water quality monitoring to manage salt risks. |

### 3.1.7. Conclusion and Recommendation

Conclusions

Irrigated lands found in the lower Awash Valley have more sodicity problems than salinity ones. Due to seepage loss, some irrigation lands near the river show saline-sodic problems along the depth. In Wa’ama scheme the cause for soil salinity was low quality of water source. In general, the selected scheme of the Afar region was categorized as 55.92 % non-saline and non-sodic, 0.20 % saline, 38.12 % sodic and 5.77 % saline-sodic soils.

Recommendation

- Assess, conduct regular monitor of depth to ground water table and quality

- For efficient irrigation water management, proper land leveling/ smopthening should be carried out

- Identify field application efficiency for proper irrigation scheduling and irrigation application (Frequency, rate and duration of application based on crop, soil and water)

- Assess and apply leaching requirement based on crop, soil physical and chemical characteristics, and water qulity.

- Provide training to farmers, DA, extension agents and others associated agricultural activities

- Adequate surface drainage system and special salt control management practices should be undertaken to rehabilitate and operationalized for lower Awash River and Sununta ground water.

- Undertake reclamation measures for salt affected soils and leaching of salt affected soils of Udoyita, Mego, Asboda, Debel, Meheri, Hinellie and Sununta schemes.

- Promote the use of salt-tolerant and improved crop varieties that are adapted to local conditions to ensure sustainable production under salinity and sodicity constraints.

- Strengthen farmer awareness and capacity through training on improved water and soil salinity management techniques, focusing on practical and locally adaptable technologies.

- Regular monitoring of soil and water quality (salinity, sodicity, EC, SAR) to guide management interventions and track improvements.

- Encourage integrated water resource management, including conjunctive use of surface and groundwater, and exploring managed aquifer recharge options.

- Advocate for supportive policies and investment mobilization for drainage infrastructure development, water quality management, and soil rehabilitation.

## The State of Salinity Problems in Small Scale Irrigation Schemes and Possible Mitigation Measures: The Case of Amhara Region

Sisay Dessale, Edimealem Getu, Aemro Wale, Tsegaye Getachew, Tigabu Fenta, Melaku Hunie, Messay Abera

### 3.2.1. Characterization of selected irrigation schemes

The assessment was conducted in the five Zones of Amhara region namely Waghimra, Central Gonder, North Wollo, South Wollo, and North Shewa and 12 Woreda’s consisting of 26 irrigation schemes. The descriptions of the schemes are given in (Table 3-11 and Fig. 3-5).  Consultations with reginal BoA, Research Institutes and centers, and other relevant stakeholders were conducted to identify the hot spots of salt-affected areas in the zones. These zones were selected due to the significant presence of soil salinity, which can adversely impact agricultural productivity and sustainability.

Figure 3-5 Selected irrigation schemes

Table 3-11 Description of selected SSI schemes

| S.no | Zone | District | Scheme name | Longitude | Latitude | Altitude |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | North Shewa | Alemketema | Wonchit (Meyih) | 498320 | 1121258 | 1304 |
| 2 | North Shewa | Alemketema | Jema (Geren) | 500375 | 1099282 | 1334 |
| 3 | North Shewa | Ensaro | Belbelit | 494041 | 1093604 | 1337 |
| 4 | North Shewa | Ankober | Melka-Jebdu | 589082 | 1058587 | 1447 |
| 5 | North Shewa | Kewot | Charie | 597254 | 1104960 | 1283 |
| 6 | North Shewa | Kewot | Sewer | 597035 | 1114300 | 1244 |
| 7 | North Shewa | Kewot | Mereye | 601622 | 1105673 | 1231 |
| 8 | North Shewa | Kewot | Wanza | 594361 | 1101307 | 1368 |
| 9 | North Shewa | Kewot | Maremiya | 598603 | 1106982 | 1257 |
| 10 | Waghimra | Abergelle | Bahir | 496067 | 1430749 | 1218 |
| 11 | Waghimra | Ziqualla | Tsitsika | 478465 | 1412252 | 1463 |
| 12 | Waghimra | Sehala | Tekeze | 459626 | 1415878 | 1146 |
| 13 | North Wollo | Kobo | Kemele | 574502 | 1334682 | 1377 |
| 14 | North Wollo | Kobo | Tewelde | 575432 | 1334474 | 1368 |
| 15 | North Wollo | Kobo | Gulmit | 577568 | 1334334 | 1364 |
| 16 | North Wollo | Kobo | Hormat-Abuarie | 574462 | 1335871 | 1395 |
| 17 | North Wollo | Kobo | Arategna | 576285 | 1333995 | 1365 |
| 18 | North Wollo | Kobo | Golina-Bergumelie | 576473 | 1332901 | 1370 |
| 19 | North Wollo | Kobo | Koba | 578178 | 1332652 | 1386 |
| 20 | North Wollo | Kobo | Ambo | 577212 | 1331226 | 1360 |
| 21 | South Wollo | Kalu | Upper-Tuloabajibo | 583653 | 1205638 | 1472 |
| 22 | South Wollo | Kalu | Middle-Tuloabajibo | 585310 | 1204592 | 1467 |
| 23 | South Wollo | Kalu | Lower-Tuloabajibo | 585712 | 1204274 | 1453 |
| 24 | Central Gonder | East Belesa | Dengora | 395598 | 1371014 | 1845 |
| 25 | Central Gonder | West Belesa | Swak | 367856 | 1381946 | 1792 |
| 26 | Central Gonder | Dembia | Seraba | 312250 | 1356500 | 1790 |

The reconnaissance survey revealed that major crops varied across zones. In Wag-Himra, key crops included onion, tomato, pepper, sweet potato, sorghum, teff, and fruits like papaya, mango, and sugarcane. Central Gondar primarily cultivates onion, garlic, chickpeas, tomato, pepper, mung bean, maize, and teff. North and South Wollo grew maize, onion, tomato, pepper, watermelon, and teff, while North Shewa had a diverse range including teff, maize, sorghum, vegetables, fruits, and tobacco.

Irrigation systems across the zones were mainly furrow and wild flooding. Farmers in Wag-Himra, North Shewa, and North and South Wollo associated flood irrigation with soil salinity and sodicity, leading to stunted growth, leaf burns, water stress, and reduced yields. In contrast, Central Gondar farmers lacked awareness of waterlogging and salinity issues. Poor drainage led to white crusts, black oily soil, and water stress in several schemes, with North Shewa farmers ranking salinity and waterlogging as severe problems.

Pictures from Charie Scheme

Figure 3-6. Pictures of salt affected soils from selected irrigation schemes.

Older irrigation schemes exhibited signs of soil degradation, including white crusts, leaf tip burn, and black powdery residues. Soil texture varied from clay to sandy loam and silty clay, with dominant colors being black, reddish, and gray (Fig. 3-6).

### Mapping and analysis of surface saline and sodic soils

Saline Soils

In Amhara region, analysis results indicated that surface soil salinity (0–30 cm depth) varied widely, ranging from 0.16 to 50.5 dS/m. Form the total irrigated area of 2108 ha, 63% were found to be none saline soils, while the remaining were affected with slightly saline (15%), moderately saline (8%), strongly saline (7%), very strongly saline (4%) and extremely saline (4%). Irrigation schemes mainly Jema (Geren), Belbelit, Melka-Jebdu, Mereye, Wanza, Maremiya, Tsitsika, Dengora, Swak and Seraba had ECe values below 0.75 dS/m while irrigation schemes including Wonchit (91.2%), Charie (19.3%), Sewer (84%), Bahir (97.5), Tekeze (99.2%), Kemele (2.9), Tewelde (12.8% ), Gulmit (76.2%), Hormat-Abuarie (8%), Arategna (1.5% ), Ambo (5.9% and Upper-Tuloabajibo (0.5%) were also had share of the farm land as non-saline soils (Table 3-12 and Fig. 3-7).

Irrigation schemes including Wonchit (Meyih), Bahir. Tekeze, Gulmit, Hormat-Abuarie and Ambo were characterized as slightely to moderately saline soils indicating a potential risk of secondary salinization over time unless irrigation management is improved. However, Charie and Kemele irrigation schemes were affected with slightly to extremely saline soils, requiring leaching water in every irrigation event for slightly to strongly saline soils, and for very strongly to extremely saline soils require reclamation with excess good quality water before cropping season. Lower-Tuloabajibo irrigation scheme was affected with slightely and strongly saline soils while Tewelde and Middle-Tuloabajibo irrigation scheme were affected with slightely to strongly saline soils. Upper-Tuloabajibo irrigation scheme was affected with slightely to strongly and extremely saline classes. Golina-Bergumelie irrigation scheme was affected with Strongly to extremely saline soils while Koba and Arategna irrigation schems were affected with slightly to very strongly saline soils, all of which requiring leaching management practices (Table 3-12 and Fig. 3-7).

Golina-Bergumelie irrigation scheme was affected with slightly and strongly to extremely saline soils while Koba and Arategna irrigation schemes were affected with slightly to very strongly saline soils, all of which requiring leaching management practices and in the case of very strongly and extremely saline soils, either salt tolerant crops be grown and leaching should be carried out at every irrigation during the crop growth peroid or reclamation with continuous or intermittent leaching has to be undertaken before the cultivation of crops commence (Table 3-12 and Fig. 3-7).

The occurrence of low precipitation and high evapotranspiration in the semi-arid areas of North Shewa, North, and South Wollo areas resulted in an extreme soil salinity level. In these areas, the groundwater comes to the surface during the wet season, and it drops in the dry season, while the soil salinity remains on the soil surface. Based on visual observation, those schemes had compacted soil with white and shiny black surfaces that contributed to poor permeability and hindered the leaching of dissolved salt and posed waterlogging. This finding agrees with Murtaza et al., 2006; Rengasamy, 2006; Ghafoor et al., 2008, re-affirming that poor drainage practice coupled with high evaporation results in a substantial buildup of salts at the surface. In addition, poor on-farm drainage practices, especially in the North Shewa, South and North Wollo irrigation schemes were another factor that usually contributes to soil salinization. This leads to the rise of the groundwater Table and that can bring salts to the upper layers of the soil profile (Kijne et al., 1998; Fentaw, 2007).

Table 3-12 Percent area coverage of surface saline soils in selected irrigation schemes

| S. No | Schemes | Total Area (ha) | ECe (dS/m) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | < 0.75 | 0.75 - 2.0 | 2 – 4 |  | 4 – 8 |  | 8 – 15 | >15 |
|  |  |  | Non-saline | Slightly saline | Moderately |  | Strongly |  | Very strongly | Extremely |
|  |  |  | Area coverage (%) |  |  |  |  |  |  |  |
| 1 | Wonchit (Meyih) | 31.5 | 91.2 | 8.8 |  | 0.0 |  | 0.0 | 0.0 | 0.0 |
| 2 | Jema (Geren) | 73.9 | 100 | 0.0 |  | 0.0 |  | 0.0 | 0.0 | 0.0 |
| 3 | Belbelit | 96.7 | 100 | 0.0 |  | 0.0 |  | 0.0 | 0.0 | 0.0 |
| 4 | Melka-Jebdu | 48 | 100 | 0.0 |  | 0.0 |  | 0.0 | 0.0 | 0.0 |
| 5 | Charie | 150.4 | 19.3 | 26.9 |  | 24.1 |  | 1.1 | 7.6 | 21 |
| 6 | Sewer | 342.2 | 84 | 16 |  | 0.0 |  | 0.0 | 0.0 | 0.0 |
| 7 | Mereye | 25.5 | 100 | 0.0 |  | 0.0 |  | 0.0 | 0.0 | 0.0 |
| 8 | Wanza | 13.3 | 100 | 0.0 |  | 0.0 |  | 0.0 | 0.0 | 0.0 |
| 9 | Maremiya | 50 | 100 | 0.0 |  | 0.0 |  | 0.0 | 0.0 | 0.0 |
| 10 | Bahir | 55.6 | 97.5 | 2.5 |  | 0.0 |  | 0.0 | 0.0 | 0.0 |
| 11 | Tsitsika | 26.2 | 100 | 0.0 |  | 0.0 |  | 0.0 | 0.0 | 0.0 |
| 12 | Tekeze | 40.1 | 99.2 | 0.8 |  | 0.0 |  | 0.0 | 0.0 | 0.0 |
| 13 | Kemele | 44.2 | 2.9 | 26.2 |  | 12.7 |  | 43 | 11.3 | 3.9 |
| 14 | Tewelde | 109.7 | 12.8 | 74.5 |  | 10.7 |  | 2.1 | 0.0 | 0.0 |
| 15 | Gulmit | 26.4 | 76.2 | 23.9 |  | 0.0 |  | 0.0 | 0.0 | 0.0 |
| 16 | Hormat-Abuarie | 32.4 | 8 | 86.2 |  | 5.8 |  | 0.0 | 0.0 | 0.0 |
| 17 | Arategna | 95.5 | 1.5 | 27.5 |  | 33.4 |  | 35.5 | 2.1 | 0.0 |
| 18 | Golina-Bergumelie | 57.2 | 0.0 | 0.9 |  | 0.0 |  | 17.3 | 76.5 | 5.2 |
| 19 | Koba | 64.7 | 0.0 | 0.4 |  | 10.7 |  | 53.9 | 34.9 | 0.0 |
| 20 | Ambo | 25.4 | 5.9 | 55.9 |  | 38.2 |  | 0.0 | 0.0 | 0.0 |
| 21 | Upper-Tuloabajibo | 84.5 | 0.5 | 5 |  | 15.7 |  | 33.7 | 0.0 | 45.1 |
| 22 | Middle-Tuloabajibo | 60.4 | 0.0 | 15.1 |  | 78.6 |  | 6.3 | 0.0 | 0.0 |
| 23 | Lower-Tuloabajibo | 47.2 | 0.0 | 63.1 |  | 0.0 |  | 36.9 | 0.0 | 0.0 |
| 24 | Dengora | 48.4 | 100 | 0.0 |  | 0.0 |  | 0.0 | 0.0 | 0.0 |
| 25 | Swak | 121.4 | 100 | 0.0 |  | 0.0 |  | 0.0 | 0.0 | 0.0 |
| 26 | Seraba | 336.8 | 100 | 0.0 |  | 0.0 |  | 0.0 | 0.0 | 0.0 |
| Sum (ha) |  | 2107.6 | 62.7 | 14.8 |  | 7.8 |  | 7.2 | 4.0 | 3.5 |

.

Figure 3-7. Mapping of surface soil salinity for selected irrigation schemes

Sodic Soils

The ESP values at a soil depth of 0-30 cm ranged from non-sodic to very strongly sodic. In the region, of the total irrigated area of 2108 ha, 41% were found to be none sodic soils and the remaining 59% were affected by different levels of sodic soils, of which 19% was affected with slightly, 8% moderately and 15 % strongly and 17% very strongly sodic soils The irrigation schemes with ESP values below 6%, which is categorized as non-sodic soils, include Wonchit, Belbelit, Melka-Jebdu, Dengora, Swak, and Seraba, while 20 irrigation schemes also had limited percentage of non-sodic soil (Table 3-13 and Fig. 3-8).

The irrigation schemes that exhibited slightly to very-strongly sodic characteristics include Charie, Sewer, Gulmit, Arategna, Ambo and Middle-Tuloabajibo while Tekeze irrigation scheme exhibited slightly to strongly sodic soils and Tewelde irrigation scheme characterized by slightly, strongly and very strongly sodic soils. Lower-Tuloabajibo irrigation scheme was characterized by moderately to very strongly sodic soils. Golina-bergumeli irrigation scheme was characterized by strongly and very strongly sodic soils and upper-Tuloabajibo irrigation scheme was characterized by slightly and very strongly sodic soils. Maremiya and Bahir irrigation scheme were characterized by slightly to moderately sodic soils, while Jema, Mereye, Wanze, Homat-Abuarie and Koba irrigation scheme was characterized by slightly sodic soil (Table 3-13 and Fig. 3-8).

The percentages of abandoned irrigated schemes for Upper-Tuloabajibo, Golina-Bergumelie Charie and Ambo were 90.1%, 66,1%, 56.3%, and 36.6%, respectively, based on ESP values.  Increasing of ESP levels in these schemes were primely attributed to an inherited shallow groundwater table, high evaporation, and runoff concentration. Additionally, soil derived from saline parent materials was identified as a major factor contributing to raising sodicity levels.

In the wet season, the groundwater table rises and subsequently falls in the dry season due to evaporation, leaving sodium ions on the surface of the soil. This phenomenon was particularly evident in North and South Wollo irrigation schemes; the runoff from surrounding mountainous areas remained stagnant for prolonged period. This leads to a rising groundwater table and increased ion accumulation. These findings align with previous research indicating that chemical weathering gradually releases the soil mineral constituents, contributing to sodicity (FAO, 1988; Omuto et al., 2020). Similarly, Rengasamy (2006) reported that capillary action causes salty groundwater to rise, promoting salt accumulation when the water evaporates. Recurrent flooding and/ or prolonged runoff over a long time acerated the soil salt accumulation (Krasilnikov et al., 2013).

Generally, secondary salinization was the main factor contributing to salt accumulation in the irrigation schemes, driven by poor drainage systems, ineffective water management, and low-quality irrigation water. Surface drainage practices were largely neglected, with North and South Wollo schemes particularly affected by both sodicity and waterlogging. While Central Gondar showed no sodicity issues, North Shewa and Wag-Himra had moderate problems, and nearly all North and South Wollo irrigation schemes were significantly impacted. Field observations revealed sodium-induced soil dispersion, leading to cemented surface layers, reduced infiltration, poor aeration, and restricted root growth crusting (El-Maaz etal., 2015). Increased exchangeable sodium also exacerbated runoff and erosion (Krasilnikov et al., 2013).

Table 3-13 Percent area coverage and surface sodic soils in selected irrigation schemes

| S. No | Scheme | Area (ha) | ESP (%) |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | <6 | 6 – 10 | 10 – 15 | 15 – 25 | >25 |
|  |  |  | Non-Sodic | Slightly Sodic | Moderately Sodic | Strongly Sodic | Very strongly sodic |
|  |  |  | Area coverage (%) |  |  |  |  |
| 1 | Wonchit (Meyih) | 31.5 | 100 | 0.0 | 0.0 | 0.0 | 0.0 |
| 2 | Jema (Geren) | 73.9 | 46.7 | 53.3 | 0.0 | 0.0 | 0.0 |
| 3 | Belbelit | 96.7 | 100 | 0.0 | 0.0 | 0.0 | 0.0 |
| 4 | Melka-Jebdu | 48 | 100 | 0.0 | 0.0 | 0.0 | 0.0 |
| 5 | Charie | 150.4 | 3 | 10.5 | 8.6 | 21.6 | 56.3 |
| 6 | Sewer | 342.2 | 7.5 | 25.1 | 11.6 | 24.8 | 31 |
| 7 | Mereye | 25.5 | 43.4 | 56.7 | 0.0 | 0.0 | 0.0 |
| 8 | Wanza | 13.3 | 4.2 | 95.8 | 0.0 | 0.0 | 0.0 |
| 9 | Maremiya | 50 | 2.4 | 88.9 | 8.6 | 0.0 | 0.0 |
| 10 | Bahir | 55.6 | 43.5 | 47.9 | 8.7 | 0.0 | 0.0 |
| 11 | Tsitsika | 26.2 | 55.2 | 44.3 | 0.5 | 0.0 | 0.0 |
| 12 | Tekeze | 40.1 | 10.1 | 76.5 | 13.3 | 0.1 | 0.0 |
| 13 | Kemele | 44.2 | 2.2 | 5.4 | 27.2 | 54.3 | 10.9 |
| 14 | Tewelde | 109.7 | 4.9 | 17.7 | - | 62.6 | 14.9 |
| 15 | Gulmit | 26.4 | 13.7 | 25.9 | 21.8 | 24.8 | 13.9 |
| 16 | Hormat-Abuarie | 32.4 | 92.7 | 7.3 | 0.0 | 0.0 | 0.0 |
| 17 | Arategna | 95.5 | 1.2 | 14 | 44 | 38.7 | 2.1 |
| 18 | Golina-Bergumelie | 57.2 | 0.0 | 0.0 | 0.0 | 33.9 | 66.1 |
| 19 | Koba | 64.7 | 14.2 | 85.8 | 0.0 | 0.0 | 0.0 |
| 20 | Ambo | 25.4 | 0.0 | 2 | 34.6 | 26.9 | 36.6 |
| 21 | Upper-Tuloabajibo | 84.5 | 1.9 | 7.9 | 0.0 | 0.0 | 90.1 |
| 22 | Middle-Tuloabajibo | 60.4 | 2.2 | 28.5 | 37.3 | 30.6 | 1.4 |
| 23 | Lower-Tuloabajibo | 47.2 | 3.4 | 0.0 | 19.4 | 56 | 21.2 |
| 24 | Dengora | 48.4 | 100 | 0.0 | 0.0 | 0.0 | 0.0 |
| 25 | Swak | 121.4 | 100 | 0.0 | 0.0 | 0.0 | 0.0 |
| 26 | Seraba | 336.8 | 100 | 0.0 | 0.0 | 0.0 | 0.0 |
| Sum |  | 2107.6 | 40.7 | 19.3 | 7.9 | 15.4 | 16.7 |

Figure 3-8 Mapping of surface soil sodicity for selected irrigation schemes.

### Classification of salt affected soils

In Amhara region, of the total irrigated area of about 2108 ha, 65% were found to be none saline-none-sodic soils, while the remaining were affected with saline (3.3%), saline-sodic (10.4%) and sodic (20.9%) soils. The irrigation schemes including Wonchit (Meyih), Jema (Green), Belbelit, Melka-Jebdu, Mereye, Wanza, Maremiya, Bahir, Tsitsika, Hormat-Abuarie, Dengora, Swak and Seraba were categorized as 100% non-saline non-sodic soils. However, in Charie (22.1%), Sewer (44.2%), Tekeze (99.9%), Tewelde (34.6%), Gulmit (61.4%), Arategna (47.9), Koba (11.1%), Ambo (36.5%), Upper-Tuloabajibo (5%), Middle-Tuloabajibo (65.8% and Lower-Tuloabajibo (22.8%) were also classified as non-saline non-sodic soils (Table 3-14).

Table 3-14 Salt affected soil classes in selected irrigation schemes

| S. No. | Schemes | Area (ha) | Salt affected soil classes |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | ECe<4 and ESP<15 | ECe>4 and ESP<15 | ECe>4 and ESP>15 | ECe<4 and ESP>15 |
|  |  |  | Non-saline and non-sodic | Saline | Saline-sodic | Sodic |
|  |  |  | Area coverage (%) |  |  |  |
| 1 | Wonchit (Meyih) | 31.5 | 100 | 0.0 | 0.0 | 0.0 |
| 2 | Jema (Geren) | 73.9 | 100 | 0.0 | 0.0 | 0.0 |
| 3 | Belbelit | 96.7 | 100 | 0.0 | 0.0 | 0.0 |
| 4 | Melka-Jebdu | 48.0 | 100 | 0.0 | 0.0 | 0.0 |
| 5 | Charie | 150.4 | 22.1 | 0.0 | 29.7 | 48.2 |
| 6 | Sewer | 342.2 | 44.2 | 0.0 | 0.0 | 55.8 |
| 7 | Mereye | 25.5 | 100 | 0.0 | 0.0 | 0.0 |
| 8 | Wanza | 13.3 | 100 | 0.0 | 0.0 | 0.0 |
| 9 | Maremiya | 50.0 | 100 | 0.0 | 0.0 | 0.0 |
| 10 | Bahir | 55.6 | 100 | 0.0 | 0.0 | 0.0 |
| 11 | Tsitsika | 26.2 | 100 | 0.0 | 0.0 | 0.0 |
| 12 | Tekeze | 40.1 | 99.9 | 0.0 | 0.0 | 0.1 |
| 13 | Kemele | 44.2 | 34.6 | 0.4 | 57.9 | 7.2 |
| 14 | Tewelde | 109.7 | 42.3 | 0.0 | 2.0 | 55.7 |
| 15 | Gulmit | 26.4 | 61.4 | 0.0 | 0.0 | 38.7 |
| 16 | Hormat-Abuarie | 32.4 | 100 | 0.0 | 0.0 | 0.0 |
| 17 | Arategna | 95.5 | 47.9 | 11.7 | 20.1 | 20.3 |
| 18 | Golina-Bergumelie | 57.2 | 0.0 | 0.0 | 96.7 | 3.3 |
| 19 | Koba | 64.7 | 11.1 | 88.9 | 0.0 | 0.0 |
| 20 | Ambo | 25.4 | 36.5 | 0.0 | 0.0 | 63.5 |
| 21 | Upper-Tuloabajibo | 84.5 | 5.0 | 0.0 | 78.8 | 16.2 |
| 22 | Middle-Tuloabajibo | 60.4 | 65.8 | 2.2 | 4.1 | 27.9 |
| 23 | Lower-Tuloabajibo | 47.2 | 22.8 | 0.0 | 4.7 | 72.6 |
| 24 | Dengora | 48.4 | 100 | 0.0 | 0.0 | 0.0 |
| 25 | Swak | 121.4 | 100 | 0.0 | 0.0 | 0.0 |
| 26 | Seraba | 336.8 | 100 | 0.0 | 0.0 | 0.0 |
| Sum (ha) |  | 2107.6 | 65.4 | 3.3 | 10.4 | 20.9 |

Koba irrigation scheme has been affected with only saline (88.9%) soil class. Arategna irrigation schemes had 11.7%, 20.1% and 20.3% saline, saline-sodic and sodic soil classes, while in Middle-Tuloabajibo irrigation schemes had 2.2%, 4.1% and 27.9% saline, saline-sodic and sodic soil classes, respectively. The irrigation schemes including Charie (29.7 and 48.2%), Kemele (57.9% and 7.2%), Tewelde (2.0% and 55.7%), Golina-Bergumelie (96.7% and 3.3%), Upper-Tuloabajibo (78.8% and 16.2%) and Lower-Tuloabajibo (4.7% and 72.6%) were affected with saline-sodic and sodic soils. The irrigation schemes including Sewer (55.8%), Tekeze (0.1%), Gulmit (38.7%) and Ambo (63.5%) were affected with sodic soils.

The simultaneous effect of water logging and evaporation promotes high salt accumulation on the soil surface. The assessment is in line with reports of Murtaza et al., (2006); Rengasamy, (2006); Ghafoor et al., (2008). Moreover, Sisay et al., (2021) and reported that high evaporation and capillary action can accumulate salt on the surface layer and these being the case that in the semi-arid waterlogged areas of Raya Kobo Valley, the extent of ECe was much higher on the topsoil (0-30 cm) than the bottom layers.

The North and South Wollo irrigation schemes showed a higher ESP values on the top layer (0-30 cm) that ranged between 20.1 and 94.4%. Exceptionally, in the case of Hormat-Abuarie and Koba irrigation schemes, the ESP values were relatively low and found within the range of 3.7 to 7.7%. This is not a common appearance of comparatively low ESP in waterlogged areas. In these schemes, there is a high ECe, while the ESP content was lower due to the antagonistic effects of dominant calcium and magnesium.

Salt characteristics of soil profile pits

The electrical conductivity (ECe) of the soil profile pits in the 0-120 cm layer varied between 0.07 and 13.4 dS/m in the Upper-Tuloabajibo irrigation scheme. Almost all the North Shewa, Wag-himra, and Central Gonder irrigation sites possessed low to slightly saline behavior. Based on the Richards, (1954), general classification criteria, they fall in the non-saline category. However, the North and South Wollo irrigation schemes, the salinity level had been within the range of slight to strongly saline classes.

The ECe concentrations of the sampling pits do not show much variability down the depth. However, the relatively high ECe was recorded from the topsoil (0-30 cm) layer and gradually decreased down the depth. The main reasons for the occurrences of relatively high ECe on the top layer can be associated with high evaporation. In the North and South Wollo irrigation schemes, the top-soil layer was swampy and waterlogged. The simultaneous effect of water logging and evaporation promotes high salt accumulation on the surface than in the bottom layers. The assessment result is in line with reports of Murtaza et al., (2006); Rengasamy, (2006); Ghafoor et al., (2008). Moreover, Sisay et al., (2021) and reported that high evaporation and capillary action can accumulate salt on the surface layer and these being the case that in the semi-arid waterlogged areas of Raya Kobo Valley, the extent of ECe was much higher on the topsoil (0-30 cm) than the bottom layers.

The extent of ESP level in the soil profile pits from the four sampling layers (0–120 cm) varied between 94.4% in the Upper Tulaabajibo and 1.28% in the Melka-Jebdu irrigation schemes. Except for the Charie and Sewer irrigation schemes all irrigation sites studied in the Central Gondar, Wag-himra, and North Shewa irrigation schemes possessed low (<6%) to a slightly sodic character (6-10%).

The North and South Wollo irrigation schemes showed a higher ESP values on the top layer (0-30 cm) that ranged between 20.1 and 94.4%. Exceptionally, in the case of Hormat-Abuarie and Koba irrigation schemes, the ESP values were relatively low and found within the range of 3.7 to 7.7%. This is not a common appearance of comparatively low ESP in waterlogged areas. In these schemes, there is a high ECe, while the ESP content was lower due to the antagonistic effects of dominant calcium and magnesium.

In most of the sampling pits, the ESP value showed a decreasing tendency as the depth increases. The maximum ESP concentration was recorded on Upper-Tuloabajibo irrigation scheme, which ranged from 94.4% (0-30 cm) to 17.8% (90-120 cm). The simultaneous effects of waterlogging and evaporation, the exchangeable sodium was higher in the top layer than the bottom layers. The study conceded with Omuto et al., (2020) who stated that under water-logged conditions the concentration of ESP is much higher on the surface layer due to the potential effects of evaporation.

Based on the Richards, (1954) classification criteria, except for the Hormat-Golina and Koba irrigation schemes, the North and South Wollo irrigation sites were classified as sodic soil, similarly Charie and Sewer schemes can be rated as sodic soil. The remaining irrigation schemes in North Shewa, Central Gonder, and Wag-himra Zones were categorized as non-saline and non-sodic soil.

### 3.2.4 Irrigation water quality

There are two types of salt problems in irrigation water that are very different in their constituent, those associated with the total salinity, designated as salinity hazard and those associated with sodium, designated as sodium hazard. Irrigation water sources used in the different schemes were 23 from river, 7 from ground water, one from reservior and 3 were from spring.

Salinity Hazard

The source of water for irrigation in all irrigation schemes were rivers, groundwater, spring and reservior.  As shown in Table 3-15, the salinity hazard of the rivers ranged between 0.17 in Dengora scheme to 2.62 dS/m in Mereye scheme. The spring water ranged between 0.44 dS/m in Belbelit scheme to 3.42 dS/m in Charie scheme and that of the groundwater ranged between 0.48 dS/m in Gulmit scheme to 2.35 dS/m in Sewer schemes. The irrigation water quality from the reservoir had 0.40 dS/m.

According to Ayers and Westcot (1994), salinity hazard of the spring water from Belbelit scheme. River water in Melka-Jebdu, Charie, Sewer, Maremiya, Bahir, Tsitsika, Tekeze, Kemele,, Tewelde, Gulmit, Hormat-Abuarie, Arategna, Golina-Bergumelie, Koba, Dengora, Swak and Seraba schemes and Groundwater from Tewelde scheme were below 0.75 dS/m and the degree of restriction on use for irrigation was considered as none to affect soil productivity. The spring water in Sewer scheme, river water in Wonchit (Meyih), Jema, Mereye, Wanza, Koba, Ambo, Upper, Middle and Lower Tuloabajibo schemes and Groundwater in Charie. Sewer, Gulmit, Golina-Bergumelie, Upper, Middle and Lower Tuloabajibo schemes were found to be slight to moderate class requiring either blending with irrigation water or need leaching management practices to create favorable environment for plant growth and maintain soil productivity. The spring water in Charie irrigation scheme was found to be severe class requiring either to avoid instead of using for irrigation or to mix/ blend with suitable class of irrigation water before use.

The major salinity hazards in the irrigation water come from groundwater (83.3%) and spring dilution (66.7%), particularly in Rift Valley (Shewarobit to Kobo Valley). Girma and Desta 2008 reported that in north Shewa, the application of saline spring waters without caution for irrigation has contributed to the development of saline soils in the study area. The findings agreed with Giday Adhanom, (2019), in which it was reported that the river in Shewarobit had medium (C2) (341.7 S/cm) and the spring had high (C3) (1452.9 S/cm) salinity hazards.

Table 3-15. Salinity hazard of irrigation water for selected irrigation schemes

| S. No. | Scheme name | Water source | ECe (dS/m) | Degree of Restriction on Use |
| --- | --- | --- | --- | --- |
| 1 | Wonchit (Meyih) | River | 0.76 | Slight to Moderate |
| 2 | Jema (Geren) | River | 0.79 | Slight to Moderate |
| 3 | Belbelit | Spring | 0.44 | None |
| 4 | Melka-Jebdu | River | 0.56 | None |
| 5 | Charie | River | 0.56 | None |
|  |  | Spring | 3.42 | Severe |
|  |  | Groundwater | 1.41 | Slight to Moderate |
| 6 | Sewer | River | 0.44 | None |
|  |  | Spring | 2.9 | Slight to Moderate |
|  |  | Groundwater | 2.35 | Slight to Moderate |
| 7 | Mereye | River | 2.62 | Slight to Moderate |
| 8 | Wanza | River | 2.46 | Slight to Moderate |
| 9 | Maremiya | River | 0.43 | None |
| 10 | Bahir | River | 0.55 | None |
| 11 | Tsitsika | River | 0.58 | None |
| 12 | Tekeze | River | 0.23 | None |
| 13 | Kemele | Golina river | 0.65 | None |
| 14 | Tewelde | Golina river | 0.65 | None |
|  |  | Tewelde well | 0.48 | None |
| 15 | Gulmit | Golina river | 0.65 | None |
|  |  | Tewelde well | 0.78 | Slight to Moderate |
| 16 | Hormat-Abuarie | Golina river | 0.65 | None |
| 17 | Arategna | Golina river | 0.65 | None |
| 18 | Golina-Bergumelie | Golina river | 0.65 | None |
|  |  | Getahun well | 0.76 | Slight to Moderate |
| 19 | Koba | Golina river | 0.65 | None |
|  |  | Kelkelie river | 0.78 | Slight to Moderate |
| 20 | Ambo | Ambo river | 0.91 | Slight to Moderate |
|  |  | Kelkelie river | 0.78 | Slight to Moderate |
| 21 | Upper, Middle, Lower Tuloabajibo | Tuloabajibo river | 0.79 | Slight to Moderate |
|  |  | Groundwater | 1.04 | Slight to Moderate |
| 22 | Dengora | Tana river | 0.17 | None |
| 23 | Swak | Reservoir | 0.4 | None |
| 24 | Seraba | River | 0.45 | None |

Sodicity Hazard

The SAR value of the irrigation water ranged from 0.03 to 42.95% and the ECe value of irrigation water ranged from 0.17 to 3.42 dS/m. The sodium hazard of the irrigation water used for 23 irrigation schemes were slight to moderate and require chemical amendments, while for 10 irrigation schemes, the irrigation water was safe for irrigation purpose. The irrigation water used from Tana River in Dengora irrigation scheme was severe and cannot be used without blending with high saline water (Table 3-16).

The irrigation waters having high SAR values can lead to the buildup of high soil Na levels over time, which in turn can adversely affect soil infiltration rate and permeability, due to soil dispersion. Hence, excessive SAR levels can lead to soil crusting, poor seedling emergence, and poor aeration.

Table 3-16 Sodicity hazard of irrigation water for selected irrigation schemes

| No. | Scheme name | Water source | SAR |  | Ecw (dS/m) |  | Degree of Restriction on Use |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Wonchit (Meyih) | River | 0.07 | 0.76 |  | None |  |
| 2 | Jema (Geren) | River | 0.08 | 0.79 |  | None |  |
| 3 | Belbelit | Spring | 0.1 | 0.44 |  | Slight to Moderate |  |
| 4 | Melka-Jebdu | River | 0.12 | 0.56 |  | Slight to Moderate |  |
| 5 | Charie | River | 0.12 | 0.56 |  | Slight to Moderate |  |
|  |  | Spring | 42.95 | 3.42 |  | Slight to Moderate |  |
|  |  | Groundwater | 8.28 | 1.41 |  | Slight to Moderate |  |
| 6 | Sewer | River | 0.09 | 0.44 |  | Slight to Moderate |  |
|  |  | Spring | 1.98 | 2.9 |  | None |  |
|  |  | Groundwater | 13.5 | 2.35 |  | Slight to Moderate |  |
| 7 | Mereye | River | 4.62 | 2.62 |  | None |  |
| 8 | Wanza | River | 2.26 | 2.46 |  | None |  |
| 9 | Maremiya | River | 4.16 | 0.43 |  | Slight to Moderate |  |
| 10 | Bahir | River | 0.12 | 0.55 |  | Slight to Moderate |  |
| 11 | Tsitsika | River | 0.08 | 0.58 |  | Slight to Moderate |  |
| 12 | Tekeze | River | 0.03 | 0.23 |  | Slight to Moderate |  |
| 13 | Kemele | Golina river | 2.09 | 0.65 |  | Slight to Moderate |  |
| 14 | Tewelde | Golina river | 2.09 | 0.65 |  | Slight to Moderate |  |
|  |  | Tewelde well | 3.62 | 0.48 |  | Slight to Moderate |  |
| 15 | Gulmit | Golina river | 2.09 | 0 - 3 |  | Slight to Moderate |  |
|  |  | Tewelde well | 3.62 | 3 - 6 |  | Slight to Moderate |  |
| 16 | Hormat-Abuarie | Golina river | 2.09 | 0 - 3 |  | Slight to Moderate |  |
| 17 | Arategna | Golina river | 2.09 | 0 - 3 |  | Slight to Moderate |  |
| 18 | Golina-Bergumelie | Golina river | 2.09 | 0 - 3 |  | Slight to Moderate |  |
|  |  | Getahun well | 1,00 | 0 - 3 |  | None |  |
| 19 | Koba | Golina river | 2.09 | 0 - 3 |  | Slight to Moderate |  |
|  |  | Kelkelie river | 2.09 | 0 - 3 |  | None |  |
| 20 | Ambo | Ambo river | 1.78 | 0 - 3 |  | None |  |
|  |  | Kelkelie river | 2.11 | 0 - 3 |  | None |  |
| 21 | Upper, Middle, Lower Tuloabajibo | Tuloabajibo river | 4.31 | 3 - 6 |  | Slight to Moderate |  |
|  |  | Groundwater | 2.35 | 0 - 3 |  | None |  |
| 22 | Dengora | Tana river | 0.49 | 0 - 3 |  | Severe |  |
| 23 | Swak | Reservoir | 1.29 | 0 - 3 |  | Slight to Moderate |  |
| 24 | Seraba | River | 0.83 | 0 - 3 |  | Slight to Moderate |  |

### Soil management option

Based on the results indicated in sections 3.2.2 to 3.2.3 analysis and classification possible mitigation measures are given in Table 3-17.

Table 3-17. Salt affected soil management options

| Irrigation schemes | Soil salt classes | Soil management options |
| --- | --- | --- |
| Charie, Sewer, Tekeze, Kemele, Tewelde, Gulmit, Arategna, Golina-Bergumelie, Ambo, Upper-Tuloab,ajibo, Middle-Tuloabajibo, Lower-Tuloabajibo | Sodic (ECe < 4 dS/m, ESP > 15%) | - Apply gypsum to replace sodium with calcium, improve drainage and infiltration rate, and incorporate organic matter to enhance soil structure. |
| Charie, Kemele, Tewelde, Arategna, Golina-Bergumelie, Upper-Tuloabajibo, Middle-Tuloabajibo, Lower-Tuloabajibo | Saline-Sodic (ECe > 4 dS/m, ESP > 15%) | - Apply gypsum and organic amendments, enhance drainage and leaching, - Apply leaching water to remove both the exchangeable sodium and salt from the root zone - Select salt-tolerant crops. |
| Kemele, Arategna, Koba, Middle-Tuloabajibo | Saline (ECe > 4 dS/m, ESP < 15%) | - Leaching of salts using good-quality irrigation water - Grow salt-tolerant crops; apply organic matter to improve soil structure. |
| All, except Golina-Bergumelie | Non-Saline and Non-Sodic (ECe < 4 dS/m, ESP < 15%) | - Maintain good soil structure; optimize irrigation scheduling, Apply organic amendments for long-term soil health. |

### Water management options

Based on the results indicated in sections 3.2.4 analysis and classification possible mitigation measures are given in table 3-18.

Table 3-18  water management options

| Irrigation schemes | Water quality challenges | Water management options |
| --- | --- | --- |
| Wonchit (Meyih), Jema (Geren), Charie (GW), Sewer (Spring & GW), Mereye, Wanza, Gulmit (GW), Golina-Bergumelie (GW), Koba, Ambo, Upper, Middle, Lower Tuloabajibo | Slight to moderate salinity hazard. | - Add leaching practice in every irrigation events as necessary. - Growing salt tolerant crops. - Maintain periodic water quality assessment |
| Charie | Severe salinity hazard. | - Belend with good quality irrigation water - Use in sodic soil |
| Belbelit, Melka-Jebdum Charie, Sewer (Ruver & GW), Maremiya, Bahir, Tsitsika, Tekeze, Kemel, Tewelde, Gulmit, Hormat-Abuarie, Arategna, Golina-Bergumelie (River), Koba (River), Upper, middle, lower- Tuloabajibo | Slight to moderate sodicity hazard. | - Mix chemical ammendments with irrigation water - Blend with saline irrigation water - Incorporate organic matter in to soils |

### Conclusion and Recommendation

Conclusion

In the selected 26 schemes with 2107.60 ha farmland from Amhara region, the non-saline soils occupy about 63% and 30% slight to strongly saline and about 8% occupy very strongly to extremely saline soils. Whereas the soil sodicity occupy about 41% with non-sodic, 27% slight to moderately sodic and 32% was strongly to very strongly sodic soils. Generally, the salt affected soils in Amhara region was considered as 65% non-saline and non-sodic, 21% sodic, 10% saline sodic and 3% was saline soils.

The salinity hazard of irrigation water used by 19 sources in Amahra region were safe for irrigation purposes, whilst from 14 sources, the water quality was slightly to moderate were. The irrigation water used in Charie irrigation scheme was from spring and was severe.

The sodium hazard of the irrigation water used from 23 water sources in Amhara regions were slightly to moderate, whilst the irrigation water from 10 water resources were safe for irrigation purpose. The irrigation water used from Tana River for Dengora irrigation scheme was severe.

Recommendation

- Assess, conduct regular monitor of depth to ground water table and quality

- For efficient irrigation water management, proper land leveling/ smopthening should be carried out

- Identify field application efficiency for proper irrigation scheduling and irrigation application (Frequency, rate and duration of application based on crop, soil and water)

- Assess and apply leaching requirement based on crop, soil physical and chemical characteristics, and water qulity.

- Provide training to farmers, DA, extension agents and others associated agricultural activities

- In farmlands with slight to strongly saline soil, yields of major crops will be affected and for cultivation of the most economically important crops, leach management practices is required to maintain a favorable salt balance in the root zone.

- In the farmlands with very strongly to extremely saline soils, only tolerant to very tolerant crops can be grown and leaching needs to be practiced while growing the salt tolerant crops. Based on the salt level, either heavy leaching practices such as intermittent or continuous leaching could be undertaken for creating favorable environment in growing economically important crops.

- In the farmlands with slightly to moderately sodic soils, the salinity level was suitable for the most tolerant crops and to improve soil permeability for most sensitive crops, chemical amendments including gypsum need to be practiced.

- In the farmlands with strongly to very strongly sodic soils, the sodicity level not suitable for most crops, particularly on heavy textured clay soils and hence requires reclamation program before cultivation of crops.

- In slight to moderate salinity hazard, the irrigation water may require additional leaching water in every irrigation event or occasionally at critical crop growth period to maintain land and crop productivity.

- If the salinity level of water exceeds crop salt tolerance level, leaching before sowing   should be carried out until the salinity of the soil and water are conducive for crops to grow

- Selection of crops tolerant crops to existing salt level and/or improved irrigation water management practices such as more frequent irrigation, residue management; change of surface irrigation system may be adopted to attain full crop yield potential

- The irrigation water used in Charie irrigation scheme require either blending with good quality irrigation water or the use of this water in sodic soil is suggested

- The sodium hazard of irrigation water used from 14 water sources require chemical amendments such as gypsum or blending with saline irrigation water quality and/ or the use of improved irrigation water management practices together with organic matter incorporation in the soil to improve the soil structure.

- The irrigation water used from Tana River needs to be blended with high saline water to improve sodicity level.

## The State of Salinity Problems in Small Scale Irrigation Schemes and Possible Mitigation Measures: The Case of Oromia and Harari Regions and Dire Dawa City Administration.

Zelalem Shelemow, Jemal Nur Hassen, Demesachew Tadele, Bayan Ahmed, Tesfaye Gragn, Ambase Abomsa, Getachaw Haile, Eshetu Ararso and Roba Elema

### Characterization of selected irrigation schemes

The desk review was conducted from the previous study and available database. It has been discussed on the primary factors contributing to salinity buildup in the region were inadequate irrigation water management, including inefficient scheduling and practices (Shahid et al., 2010), and the lack of sufficient drainage systems. These issues are particularly prevalent in semi-arid and arid regions, where surface-soil salinity is a significant concern. This finding aligns with Qureshi et al. (2020), who reported a widespread occurrence of soil and water salinity and sodicity in the lowland irrigated areas of Ethiopia.

Hence, the desk review focused on identifying salt-affected small-scale irrigation (SSI) schemes through desk research and stakeholder consultations. It utilized previous studies and available databases while engaging experts at regional, zonal, woreda, and local levels. The study covered eight zones in Oromia (Arsi, West Arsi, East Shewa, East and West Hararghe, Guji, West Guji, and Borena), one district in Harari (Erer Dodota), and the Wahel Cluster in Dire Dawa City Administration. Research was supported by five centers under the Oromia Agricultural Research Institute.

Figure 3-9. Selected irrigation schemes.

In this study eight (8) Zones in Oromia (Arsi and West Arsi, East Shewa, East and West Hararghe, Guji and West Guji, Borena), one district from Harari (Erer Dodota) and Wahel Cluster in Dire Dawa City Administration were covered by five research centers of the Oromia Agricultural Research Institute. A total of 34 schemes (21 from stream diversion, 11 from ground water users and 1 spring development) were addressed (Table 3-19). These schemes are scattered in different geographic, topographic and agro-ecological conditions. Most of the schemes are found under dry ‘Kolla’ and dry ‘Woina Dega’ agro-ecology and scattered among Awash, Rift Valley Lakes, Genale-Dawa and Wabi Shebele River basins (Fig. 3-9). The topography and slopes of the schemes were characterized as mountainous, scattered volcanic cones and creators, almost flat to gentle slope and the schemes in between mountains.

The dominant crops grown in the study areas are cereal (maize, wheat, and sorghum), pulses (haricot bean, green peas), stimulants (coffee, khat), vegetables (tomato, pepper, onion/baro, head cabbage, Ethiopian cabbage and lettuce), roots and tubers (potato, sweet potato, sugar beet, and beet root), fruits (mango, banana, zeyituna, papaya), etc.  Maize is usually cultivated at the near end of main season so as to reach supplementary irrigation. Wheat was introduced recently to be cultivated under irrigation as a strategic crop by the government for ensuring food security and export market (Table 3-19).

Based on hot spot indicators of salinity and sodicity, primely 34 schemes using different irrigation water sources (21 schemes from river water source, 11 groundwater and 1 spring water) that are distributed in Arsi (7), Guji (5), Borana (6), East shoa and west Arsi (9), East and West Hararge (5), Harari Region (1) and Dire Dawa City Administration (1) were selected (Table 3-19).

Table 3-19. Selected irrigation schemes

| Zone name | Scheme | Actual irrigable area (ha) | Year of start irrigation | Household using the scheme | Major crops |
| --- | --- | --- | --- | --- | --- |
| Guji and West Guji | Ababa | 54 | 1989 | 96 | Maize, tomato, Head cabbage; sugar beet, Banana, local cabbage |
|  | Bitata (GW) | 40 | 2007 | 160 | Maize, tomato, cabbage, onion, local cabbage. |
|  | Gidabo | 400 | 2010 | 200 |  |
|  | Warabele I (GW) | 200 | 2003 | 800 | Tomato, Head cabbage; onion, Hot pepper and local cabbage |
|  | Warabele II (GW) | 200 | 2003 | 800 | “ |
| Sub- total |  | #### 894 |  |  |  |
| Arsi | Kambaru Ijara | 210 | 2002 | 265 | Onion, pepper, maize, tomato cabbage, wheat |
|  | Mada gudina | 157 | 2002 | 325 | “ |
|  | Shaled | 75 | - | 277 | Onion, tomato pepper, cabbage, wheat |
|  | Gologota | 900 | 1998 | 893 | “ |
|  | Bosha I | 100 | 1989 | 263 | “ |
|  | Watero dino | 400 | 1998 | 377 | “ |
|  | Abdi boru | 125 | 2002 | 240 | Onion, tomato Cabbage, maize, wheat |
| Sub- total |  | 1,967 |  |  |  |
| Borana | Qadalle | 289 | 2007 | 275 | Maize, tomato, cabbage. mango, papaya and wheat |
|  | Gowa | 85 | 1988 | 75 | Tomato; Cabbage; Mango; Papaya; Onion; Banana; Lemon; Zayituna and Green Pepper |
|  | Saboqo (GW) | 145 | 2008 | 575 | fodder production |
|  | Sabbaa | 105 | 1996 | 175 | Tomato, cabbage; mango, and papaya wheat |
|  | Bule koloba | 189 | 1998 | 875 | Tomato, cabbage; mango, papaya and wheat |
|  | Arganne | 75 | 2009 | 200 |  |
| Sub- total |  | 888 |  |  |  |
| East Hararge | Haro-adi (GW) | 352.6 | - | 745 | Potato, head cabbage, Ethiopian cabbage, lettuce, small pod, hot pepper, carrot, beat root, green onions (baro), khat |
|  | Tuji-gabisa (GW) | 1141.5 | - | 3591 | “ |
|  | Dhoma | 292 | 542 | 1993 | Potato, Maize, Khat, Onion, Cabbage, and sweet potato |
| West Hararge | Kase hija | 389 | 1993 | 542 | Khat, coffee, tomato, pepper, maize |
|  | Hargeti | 500 | 1998 | 370 | Khat, coffee, tomato, pepper, maize, banana, sweet potato, wheat |
| Dire Dawa | Dujuma | 120 | 2002 | 130 | Maize, Khat, tomato, onion and coffee |
| Harari Region | Erer dodota | 50 | - | 70 | Sugar cane, Khat, mango tomato, onion, coffee and maize |
| Sub- total |  | 2,845.1 |  |  |  |
| West Arsi | Yeka lelisa | 159 | 1992 | 210 | Tomato, head cabbage and wheat |
| East Shewa | Lucha Denbe l(GW) | 12 | 2013 | 16 | Onion, tomato, wheat, maize and head cabbage |
|  | Dodo-wadara (GW) | 3 | 2001 | 12 | maize, onion and tomato |
|  | Gola | 185.25 | 1972 | 600 | Maize, pepper, cabbage and beet root |
|  | Galcha | 85 | 1980 | 172 | maize, onion and tomato |
|  | Dire Sade | 105 | 1985 | 140 |  |
| Sub- total |  | 549.25 |  |  |  |
| Total |  | 7,143.35 |  |  |  |

### Mapping and analysis of surface saline and sodic soils

Saline Soil

In Oromia region, analysis results on surface soil salnity (0–30 cm depth) from  4759 ha of farm land indicated that 19% were found to be none saline soils, while the remaining were affected by slightly saline (38%), moderately saline (29%), strongly saline (6%) and very strongly (8%) saline soils. Irrigation schemes including Saboqo GW (98.1%), Saba (100%), Blue koboba (98%), Jirarsa (12.5%), Arganne (96.3%), Qadalle (0.3%), Dire-sadi (2.4%), Malima, Wodera and Mikael (0.2%), Bosha 1 (61.4%), Sheled (16.2%), Ababa (70%), Bitata (0.5%), WBL I (0.9%), Gidabo (0.3%) and WBL II (62.1%) had the farm land with non-saline soils (Table 3-20 and Fig. 3-10). These schemes require to monitore the soil salinity level at regular interval to maintain the soil productivity.

Irrigation schemes including Saboqo GW (1.9%), Blue koboba (2%), Jirarsa (25%), Arganne (3.7%), Qadalle (6.5%), Erere (4.0%), Dhoma (42.1%) Hargeti (34.8%) Dujuma (100%), Yek-lelisa (100%), Gola (100%), Gelcha (37.3%), Dire-sadi (97.6%), Malima, Wodera and Mikael (19%), Lucha Dembel (31.8%), Bosha 1 (38.6%), Sheled (83.8%), Wataro Dino (5.8%), Golgota (1.6%), K.J. (94.5%), M GUD (22.3%), Abdi Bori (18.5%), Ababa (19%),  Bitata (99.5%), WBL I (99.1%), Gidabo (82.7%) and WBL II (37.9%) were found to be slightly silane soils. In these schemes regular leaching practices need to be included in every irrigation events, to maintain the soil productivity and to create favorable salt level for salt sensitive crops.  The irrigation schemes in Keseja (100%), Dhoma (57.9%), Hargeti (65.2%), Malima, Wodera and Mikael (80.8%), KJ (5.5%), Ababa (11%) and Gidabo (17%) were affected with moderately saline soils. In these schemes, growing salt tolerant crops along with regular leaching practices are needed during irrigation application events. Jirarsa (50% and 12.5%, Eerere (64% and 32%), Gelcha (51% and11.7%), Golgota (87.1% and 11.3%), M GUD (66.2% and 11.5%) Abdi Bori (70.5% and 11%), irrigation schemes were affected with moderately to strongly saline soils, respectively. The irrigation schemes in Qadalle (43.4%, 28.8% and 21.8%), and Wataro Dina (77.4%, 13.9% and 2.9%) were affected with moderately to very strogly saline soils, respectively. The irrigation schemes in Haro Adi (6.2% and 93.8%) and Tuji Gabisa (56 and 44%) were affected with strongly to very strongly saline soils. And Mekdela (100%) irrigation scheme was affected with very strongly saline soil (Table 3-20 and fig. 3-10). In these schemes, yields of many crops are restricted. Growing salt tolerant crops and regular leaching practice during irrigation application is needed. However, for very strongly salines soils, heavy leaching program as reclamation may be needed during crop off season, to maintain favourable salt level for sensitive crop.

Table 3-20. Percent area coverage of surface saline soils in selected irrigation schemes.

| No | Scheme | Total Area (ha) | ECe (dS/m) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | <0.75 | 0.75 – 2 | 2 – 4 | 4 – 8 | 8 – 15 | >15 |
|  |  |  | Non-Saline | Slightly Saline | Moderately Saline | Strongly Saline | Very Strongly Saline | Extremely Saline |
|  |  |  | Percent area coverage (%)- |  |  |  |  |  |
| 1 | Saboqo GW | 30.7 | 98.1 | 1.9 | 0.0 | 0.0 | 0.0 | 0.0 |
| 2 | Saba | 51.8 | 100.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| 3 | Bule-koloba | 29.0 | 98.0 | 2.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| 4 | Jirarsa | 4.7 | 12.5 | 25.0 | 50.0 | 12.5 | 0.0 | 0.0 |
| 5 | Arganne | 31.7 | 96.3 | 3.7 | 0.0 | 0.0 | 0.0 | 0.0 |
| 6 | Qadalle | 437.3 | 0.3 | 6.5 | 43.4 | 28.4 | 21.8 | 0.0 |
| 7 | Erere | 90.2 | 0.0 | 4.0 | 64.0 | 32.0 | 0.0 | 0.0 |
| 8 | Keseja | 310.0 | 0.0 | 0.0 | 100.0 | 0.0 | 0.0 | 0.0 |
| 9 | Dhoma | 91.9 | 0.0 | 42.1 | 57.9 | 0.0 | 0.0 | 0.0 |
| 10 | Hargeti | 424.8 | 0.0 | 34.8 | 65.2 | 0.0 | 0.0 | 0.0 |
| 11 | Dujuma | 62.8 | 0.0 | 100.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| 12 | Haro Adi | 195.5 | 0.0 | 0.0 | 0.0 | 6.2 | 93.8 | 0.0 |
| 13 | Tuji Gabisa | 29.6 | 0.0 | 0.0 | 0.0 | 56.0 | 44.0 | 0.0 |
| 14 | Yek-lelisa | 32.8 | 0.0 | 100.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| 15 | Gola | 15.3 | 0.0 | 100.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| 16 | Gelcha | 35.0 | 0.0 | 37.3 | 51.0 | 11.7 | 0.0 | 0.0 |
| 17 | Dire-sadi | 97.4 | 2.4 | 97.6 | 0.0 | 0.0 | 0.0 | 0.0 |
| 18 | Malima, Wodera and Mikael | 510.3 | 0.2 | 19.0 | 80.8 | 0.0 | 0.0 | 0.0 |
| 19 | Mekdela | 51.0 | 0.0 | 0.0 | 0.0 | 0.0 | 100.0 | 0.0 |
| 20 | Lucha Dembel | 34.7 | 0.0 | 31.8 | 68.2 | 0.0 | 0.0 | 0.0 |
| 21 | Bosha 1 | 141.1 | 61.4 | 38.6 | 0.0 | 0.0 | 0.0 | 0.0 |
| 22 | Sheled | 77.3 | 16.2 | 83.8 | 0.0 | 0.0 | 0.0 | 0.0 |
| 23 | Wataro Dino | 421.7 | 0.0 | 5.8 | 77.4 | 13.9 | 2.9 | 0.0 |
| 24 | Golgota | 622.5 | 0.0 | 1.6 | 87.1 | 11.3 | 0.0 | 0.0 |
| 25 | K.J. | 184.9 | 0.0 | 94.5 | 5.5 | 0.0 | 0.0 | 0.0 |
| 26 | M GUD | 278.6 | 0.0 | 22.3 | 66.2 | 11.5 | 0.0 | 0.0 |
| 27 | Abdi Bori | 292.9 | 0.0 | 18.5 | 70.5 | 11.0 | 0.0 | 0.0 |
| 28 | Ababa | 41.2 | 70.0 | 19.0 | 11.0 | 0.0 | 0.0 | 0.0 |
| 29 | Bitata | 32.2 | 0.5 | 99.5 | 0.0 | 0.0 | 0.0 | 0.0 |
| 30 | WBL I | 39.9 | 0.9 | 99.1 | 0.0 | 0.0 | 0.0 | 0.0 |
| 31 | Gidabo | 50.0 | 0.3 | 82.7 | 17.0 | 0.0 | 0.0 | 0.0 |
| 32 | WBL II | 10.0 | 62.1 | 37.9 | 0.0 | 0.0 | 0.0 | 0.0 |
| Sum |  | 4758.8 | 19.4 | 37.8 | 28.6 | 6.1 | 8.2 | 0.0 |

Figure 3-10. Mapping of surface saline soils for selected irrigation schemes

The main causes for salinity build up in those schemes were mainly due to poor irrigation water management (improper scheduling and practices (Shahid et al., 2010) and absence of adequate drainage systems, particularly in semi-arid and arid ecology are causes of surface soil salinity. This finding is in agreement with Qureshi et al. (2020) and reported a widespread occurrence of soil and water salinity and sodicity in the lowland irrigated areas of Ethiopia.

Sodic Soils

The laboratory results of soil sodicity (ESP) of surface soil samples have shown that 33% were none sodic soils and the remaining 67% were affected by different levels of sodic soils. In the region, of the total irrigated area of about 4760 ha, 17.9% were found to be slightly sodic, 21.1 % moderately sodic, 14.6% strongly sodic and  13.5% very strongly sodic soils (Table 3-21 and Fig. 3-11)  ).

Non-sodic soils were observed in irrigation schemes of Saboqo GW (98.1%), Saba (89.9), Bule-koloba (98%), Arganne (100%), Qadalle (73%), Keseja (22.6%), Dhoma (100%), Hargeti (100%), Dujuma (88.5%),Tuji Gabisa (4%), Gola (35%), Dire-sadi (99.2%), Golgota (3.9%), M GUD (48.2%), Abdi Bori (2.7%), Ababa (87.8), Bitata (2.7%) and WBL I (1.7%). These irrigation schemes are suitable for cultivation of any crop and require regular monitoring before becoming a problem. Slightly sodic soils were observed in irrigation schemes of Saboqo GW (1.9% ), Saba (10.1%), Bule-koloba (2%), Dujuma (11.5%), Gola(65%) and Dire-sadi (0.8%). The irrigation schemes at Golgota (8.7%,87.4%, ) and M GUD (45.3%, 6.5%) were slightly to moderately, respectively. These irrigation schemes are suitable for most tolerant crops, and require organic matter to further improve and maintain the sodicity levels (Table 3-21 and Fig. 3-11) ).

The irrigation schemes in Dire-sadi (62.5%, 25%, 12.5%), Qadalle (16.1%, 8.7%, 2.2%), Erere (4.0%, 64%, 32%), Haro Adi (43.8%,22.2%, 34%),  Tuji Gabisa (52%, 16%, 28%), Sheled (62.2%, 21.6%, 16.2%), Wataro Dino (1.9%, 84.6%, 13.5%) and Abdi Bori (28.1%, 67.8%, 1.4%) were affected with slightly to strongly sodic soils, respectively. The irrigation sschemes affected with strongly sodic soils are not suitable for most sensitive crops. Tolerant crops need to be grown and the use of chemical amendments and organic matter are needed to improve soil infiltration rate and permeability. Keseja (15.2%, 33.9%, 21.4%, 7%), Bitata (39.2%, 38.7%, 18.3%, 1.1%),WBL I (25.1%, 62.8%, 9.5%, 0.9%),Gidabo (65.7%, 31.5%, 2.1%, 0.7%) and WBL II (3.4%, 12.1%, 60.3%, 24.1%) slightly to very strongly sodic, respectively (Table 3-21 and Fig. 3-11)  ).

The irrigation schemes at K.J. (2.2%, 97.8%) were affected with moderately to strongly sodic soils, respectively. And Yek-lelisa (4.8%, 26.2%, 69%) and LuchaDembel (2.3%, 11.4%, 86.3%) irrigation schemes were affected with moderately to very strongly sodic soils, respectively, And also, Gelcha (45.1%, 54.9%), Wodera and Mikael (0.5%, 99.5%) and Mekdela (12.3%, 87.7%)  were affected with strongly to very strongly sodic soils, respectively. The irrigation sschemes affected with very strongly sodic soils may not be suitable for growing highly tolerant crops, particularly on heavy clay soils and hence requires chemical amendments to undertake a reclamation program before undertaking crop cultivation.

These large portions of soil sodicity classes need urgent management interventions such as the use of chemicals ammendments including lime, sulphur and gypsum application with leaching and surface management practices including mulching and manuring, etc. Samples collected from Dhoma, Hargeti, Dujuma, Soboqo, Saba, Bule-kolobo, Argane, Qadale, Ababa irrigation schemes showed non-sodic soil class, whereas highly sodic surface soil were observed at Haro adi, Gelcha, Bitata, Warabele-I, Gidaboo, Shelede, Wataro-dino, Kambaro-jara and Abdi-boru as depicted in Table 3-21 and Fig. 3-11).

Table 3-21. Percent area coverage of surface sodic soils in selected irrigation schemes.

| S. No | Scheme | Total Area (ha) | ESP% |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | <6 | 6 – 10 | 10 – 15 | 15 - 25 | >25 |
|  |  |  | Non-Sodic | Slightly Sodic | Moderately Sodic | Strongly Sodic | Very Strongly sodic |
|  |  | Percent area (%) |  |  |  |  |  |
| 1 | Saboqo GW | 30.7 | 98.1 | 1.9 | 0.0 | 0.0 | 0.0 |
| 2 | Saba | 51.8 | 89.9 | 10.1 | 0.0 | 0.0 | 0.0 |
| 3 | Bule-koloba | 29.0 | 98.0 | 2.0 | 0.0 | 0.0 | 0.0 |
| 4 | Dire-sadi | 5.3 | 0.0 |  |  |  | 0.0 |
| 5 | Arganne | 31.7 | 100.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| 6 | Qadalle | 437.3 | 73.0 | 16.1 | 8.7 | 2.2 | 0.0 |
| 7 | Erere | 90.2 | 0.0 | 4.0 | 64.0 | 32.0 | 0.0 |
| 8 | Keseja | 310.0 | 22.6 | 15.2 | 33.9 | 21.4 | 7.0 |
| 9 | Dhoma | 91.9 | 100.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| 10 | Hargeti | 424.8 | 100.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| 11 | Dujuma | 62.8 | 88.5 | 11.5 | 0.0 | 0.0 | 0.0 |
| 12 | Haro Adi | 195.5 | 0.0 | 43.8 | 22.2 | 34.0 | 0.0 |
| 13 | Tuji Gabisa | 29.6 | 4.0 | 52.0 | 16.0 | 28.0 | 0.0 |
| 14 | Yek-lelisa | 32.8 | 0.0 | 0.0 | 4.8 | 26.2 | 69.0 |
| 15 | Gola | 15.3 | 35.0 | 65.0 | 0.0 | 0.0 | 0.0 |
| 16 | Gelcha | 35.0 | 0.0 | 0.0 | 0.0 | 45.1 | 54.9 |
| 17 | Dire-sadi | 97.4 | 99.2 | 0.8 | 0.0 | 0.0 | 0.0 |
| 18 | Malima, Wodera and Mikael | 510.3 | 0.0 | 0.0 | 0.0 | 0.5 | 99.5 |
| 19 | Mekdela | 51.0 | 0.0 | 0.0 | 0.0 | 12.3 | 87.7 |
| 20 | Lucha Dembel | 34.7 | 0.0 | 0.0 | 2.3 | 11.4 | 86.3 |
| 21 | Bosha 1 | 141.1 | 0.0 | 0.0 | 80.0 | 20.0 | 0.0 |
| 22 | Sheled | 77.3 | 0.0 | 62.2 | 21.6 | 16.2 | 0.0 |
| 23 | Wataro Dino | 421.7 | 0.0 | 1.9 | 84.6 | 13.5 | 0.0 |
| 24 | Golgota | 622.5 | 3.9 | 8.7 | 87.4 | 0.0 | 0.0 |
| 25 | K.J. | 184.9 | 0.0 | 0.0 | 2.2 | 97.8 | 0.0 |
| 26 | M GUD | 278.6 | 48.2 | 45.3 | 6.5 | 0.0 | 0.0 |
| 27 | Abdi Bori | 292.9 | 2.7 | 28.1 | 67.8 | 1.4 | 0.0 |
| 28 | Ababa | 41.2 | 87.8 | 8.0 | 3.0 | 1.3 | 0.0 |
| 29 | Bitata | 32.2 | 2.7 | 39.2 | 38.7 | 18.3 | 1.1 |
| 30 | WBL I | 39.9 | 1.7 | 25.1 | 62.8 | 9.5 | 0.9 |
| 31 | Gidabo | 50.0 | 0.0 | 65.7 | 31.5 | 2.1 | 0.7 |
| 32 | WBL II | 10.0 | 0.0 | 3.4 | 12.1 | 60.3 | 24.1 |
|  | Sum | 4759.4 | 33.0 | 17.9 | 21.1 | 14.6 | 13.5 |

Figure 3-11. Mapping of surface sodic soils for selected irrigation schemes

### 3.3.3. Classification of salt affected soils

In the region, of the total irrigated area of about 4775 ha, 35.1% were found to be none saline-none-sodic soils, while the remaining were affected with saline (12.2%), saline-sodic (11.1%) and sodic (41.6%) soils. Table 3-22 shows area coverage of surface salt concentration and list of the salt affected schemes from respective Zones. All irrigation schemes have varied levels of non-saline and non-sodic soils ranging from 5.5% to 100%, except Haro Adi, Tuji Gabisa, Gelcha, Malima, wodera and Mikael, Mekdela, Lucha Dembel and K.J comprising only salt-affected soils.

The irrigation schemes in Qadalle (41.2%), Haro Abdi (50%), Wataro Dino (75.8), Abdi Bori (75.8%), Gidabo (51.9%) and WBL I (60.7%) were affected with saline-sodic soils. These schemes require the use of ammendments followed by leaching management practices. The exchangeable Na+ problem should be addressed first, followed by the excessive salt problem. Therefore, the chemical amendment, mainly gypsum, should be applied to replace the exchangeable Na+, and then excessive water applied to leach out the Na+ and the excess soluble salts. Establishing salt tolerant crops shortly after reclamation and use of ammendments increase the effectiveness of reclamation efforts.

The irrigation schemes in Qadalle (41.2%), Dhoma (87.4%), Hargeti (87.4%), Haro Adi (50%), Tuji gabisa (100%) and Gidabo (25.9%) were affected with saline soils.  These soils require leaching management program, based on the level of salt which could be either pre-planting leaching, leaching during crop cultivation or as reclamation measure, continuous and intermittent leaching program for recovering the high level of saline soils.

The irrigation schemes in Saba (40.8%), Jirarsa (53.4%), Erere (63.3%), Keseia (63.3%), Yek-Lelisa (77.5%), Gelcha (100%), Dire-Sadi (63.3%), Malima, wodera and Mikael (100%), Mekdela (100%), Lucha Dembel (100%), Bosha 1 (63.3%), Sheled (63.3%), Wataro Dino (18.8%), Golgota (88.7%), Golgota (100%), K.J. (63.3%), M GUD (18.8), Abdi Bori (54%), Ababa (49%) and WBL II (30.6%) were affected with sodic soils. These soils require the use of ammendments to improve the soil permeability. Based on the severeity of exchangeable Na+, either chemical amendments including gypsum and organic matter could be used during growing crops, or reclamation program to improve the soil permeability has to be undertaken.

Table 3-22. Salt affected soil classes in selected irrigation schemes

| S. No | Schemes Name | Area | Area (ha) | Salt affected soil classes |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | ECe < 4 dS/m and ESP <15 | ECe > 4 dS/m and ESP <15 | ECe > 4 dS/m and ESP >15 | ECe < 4 dS/m and ESP >15 |
|  |  |  |  | Non-Saline Non-Sodic | Saline | Saline Sodic | Sodic |
|  |  |  |  | Percent (%) |  |  |  |
| 1 | Saboqo GW | Area (ha) | 30.7 | 100 | 0 | 0 | 0 |
| 2 | Saba | Area (ha) | 51.8 | 59.2 | 0 | 0 | 40.8 |
| 3 | Bule-koloba | Area (ha) | 29 | 100 | 0 | 0 | 0 |
| 4 | Jirarsa | Area (ha) | 4.6 | 46.6 | 0 | 0 | 53.4 |
| 5 | Arganne | Area (ha) | 31.7 | 100 | 0 | 0 | 0 |
| 6 | Qadalle | Area (ha) | 436.1 | 17.6 | 41.2 | 41.2 | 0 |
| 7 | Erere | Area (ha) | 90.2 | 36.7 | 0 | 0 | 63.3 |
| 8 | Keseja | Area (ha) | 310 | 36.7 | 0 | 0 | 63,3 |
| 9 | Dhoma | Area (ha) | 91.6 | 12.6 | 87.4 | 0 | 0 |
| 10 | Hargeti | Area (ha) | 424.8 | 12.6 | 87.4 | 0 | 0 |
| 11 | Dujuma | Area (ha) | 62.8 | 100 | 0 | 0 | 0 |
| 12 | Haro Adi | Area (ha) | 195.6 | 0 | 50 | 50 | 0 |
| 13 | Tuji gabisa | Area (ha) | 29.6 | 0 | 100 | 0 | 0 |
| 14 | Yek-lelisa | Area (ha) | 32.8 | 22.5 | 0 | 0 | 77.5 |
| 15 | Gola | Area (ha) | 15.3 | 100 | 0 | 0 | 0 |
| 16 | Gelcha | Area (ha) | 39.9 | 0 | 0 | 0 | 100 |
| 17 | Dire-sadi | Area (ha) | 97.4 | 36.7 | 0 | 0 | 63.3 |
| 18 | Malima, wodera and Mikael | Area (ha) | 510.3 | 0 | 0 | 0 | 100 |
| 19 | Mekdela | Area (ha) | 51 | 0 | 0 | 0 | 100 |
| 20 | Lucha Dembel | Area (ha) | 34.7 | 0 | 0 | 0 | 100 |
| 21 | Bosha 1 | Area (ha) | 141.1 | 36.7 | 0 | 0 | 63.3 |
| 22 | Sheled | Area (ha) | 75.3 | 36.7 | 0 | 0 | 63.3 |
| 23 | Wataro dino | Area (ha) | 421.7 | 5.5 | 0 | 75.8 | 18.8 |
| 24 | Golgota | Area (ha) | 633.4 | 12.7 | 0 | 0 | 88.7 |
| 25 | K.J. | Area (ha) | 184.9 | 0 | 0 | 0 | 100 |
| 26 | M GUD | Area (ha) | 280.5 | 36.7 | 0 | 0 | 63.7 |
| 27 | Abdi bori | Area (ha) | 295 | 5.5 | 0 | 75.8 | 18.8 |
| 28 | Ababa | Area (ha) | 41 | 81.2 | 0 | 0 | 18.8 |
| 29 | Bitata | Area (ha) | 32.2 | 46 | 0 | 0 | 54 |
| 30 | WBL_ I | Area (ha) | 39.9 | 51 | 0 | 0 | 49 |
| 31 | Gidabo | Area (ha) | 50 | 22.2 | 25.9 | 51.9 | 0 |
| 32 | WBL_ II | Area (ha) | 10.1 | 8.7 | 0 | 60.7 | 30.6 |
|  |  | Sum | 4775 | 35.1 | 12.2 | 11.1 | 41.6 |

Salt characteristics of soil profile pits

Analysis of soil samples from the soil profile pits indicated that 53% were slightly to moderately saline and 31.30% were strongly to extremely saline. This result indicated that the salinity source might be from soil parent material, poor ground water used for irrigation as well as ground water capillarity rise. Those schemes that are highly affected by salinity are from Harari region (upper and middle part of Erer dodota scheme), East and West Hararghe Zones (Dhoma, Hargeti, Haro Adi and Tuji Gebisa ground water users), Borana Zone (Saboqo ground water users, Argane and Qadale), Guji and West Guji Zones (Ababa, Bitata, Worabale I and Gidabo), Arsi Zone (Wotaro Dino and Mada Guddina schemes) and East Shoa (Gelcha irrigation schemes).

There was increasing trend of soil salinity along the profile pits from surface to the bottom for some irrigation schemes such as Qadalle, Argeti tirtiro, at middle and lower of Dujuma, at lower of Haro-adi, at upper of Ababa, Bitata, Warabele I, Sheled, at the middle of Wotaro-dino and lower of Meda Gudina schemes. This may be due to poor irrigation practices in the absence of proper irrigation scheduling that might have caused water logging condition, and use of poor quality irrigation water and over and/ or under irrigation. On the other hand, a decreasing trend of soil salinity along the soil profile from surface downward was observed for Arganne, Dhoma, Dujuma, Gelcha, Lucha-dembel and Gidabo schemes. The result was due to rising salt laden groundwater close to the soil by capillary action. As the water is taken up by plants through transpiration or lost to the atmosphere by evaporation, soil water salinity increases because salts become more concentrated in the remaining soil water. Thus, evaporation from the soil surface and transpiration from plants between irrigation periods can further increase salinity.

### 3.3.4 Irrigation water quality

There are two types of salt problems in irrigation water that are very different in their constituent, those associated with the total salinity, designated as salinity hazard and those associated with sodium, designated as sodium hazard. Irrigation water sources used in the different schemes were 12 from river, 12 from ground water, 9 from flood diversion and 3 were from spring.

Salinity Hazard

The irrigation schemes in Oromia and Harari Regions and Dire Dawa City Administration use various water sources. The results showed that Ababa and Gidabo river diversions in Guji zone of Oromia region; Groundwater sources from Erere dodota (Harari region); Tuj gabisa upper and Lower and Haro Adi upper and lower at East Hararge zone were categorized as severe salinity hazard indicating restrictive use for irrigation, since their respective ECw is greater than 3 dS/m. The water from these sources needs special attention if intended for irrigation. It must either be blended with fresh water or be used in sodic soils. Otherwise, the water is restrictive for irrigation purposes (Table 3-23).

Table 3-23. Salinity hazard of irrigation water for selected irrigation schemes

| S.No | Schemes Name | Water Source | Salinity Hazard |  |
| --- | --- | --- | --- | --- |
|  |  |  | ECw (dS/m) | Degree of restriction for use |
| 1 | Ababa | River diversion | 6.2 | Severe |
| 2 | Bitata | GW | 2.9 | Slight to moderate |
| 3 | Gidabo | River diversion | 8.7 | Severe |
| 4 | Warabele I | Flood diversion | 2.6 | Slight to moderate |
| 5 | Warabele II | Flood diversion | 2.9 | Slight to moderate |
| 6 | Abdi Boru | River diversion | 0.6 | None |
| 7 | Bosha | Spring | 0.6 | None |
| 8 | Kebaro Ijara | River diversion | 0.1 | None |
| 9 | Golgota | River diversion | 0.2 | None |
| 10 | Mada Gudina | River diversion | 0.6 | None |
| 11 | Shelad | River diversion | 0.6 | None |
| 12 | Wotaro Dima | River diversion | 0.7 | Slight to moderate |
| 13 | Erer dodota | GW | 4.0 | Severe |
| 14 | Tuj gabisa upper | GW | 4.1 | Severe |
| 15 | Tuji Gebisa lower | GW | 3.7 | Severe |
| 16 | Kase Hija | Spring | 1.1 | Slight to moderate |
| 17 | Hargeti | Flood diversion | 0.8 | Slight to moderate |
| 18 | Dhoma | Spring | 1.1 | Slight to moderate |
| 19 | Dujuma | Flood diversion | 1.8 | Slight to moderate |
| 20 | Haro adi Up | GW | 8.4 | Severe |
| 21 | Haro Adi low | GW | 5.4 | Severe |
| 22 | Bule Koloba | Flood diversion | 1.6 | Slight to moderate |
| 23 | Saboqo | GW | 1.7 | Slight to moderate |
| 24 | Argane | Flood diversion | 0.4 | None |
| 25 | Qadalle | Flood diversion | 1.1 | Slight to moderate |
| 26 | Jirarsa | Flood diversion | 1.3 | Slight to moderate |
| 27 | Sabba | Flood diversion | 0.6 | None |
| 28 | Gola | River diversion | 0.6 | None |
| 29 | Wedera | GW | 1.5 | Slight to moderate |
| 30 | Mekdela | GW | 2.0 | Slight to moderate |
| 31 | Dire Sade | River diversion | 0.6 | None |
| 32 | Lucha | GW | 2.1 | Slight to moderate |
| 33 | Mikeli | GW | 1.7 | Slight to moderate |
| 34 | Galcha | River | 0.6 | None |
| 35 | Yeka lilisa | River | 0.2 | None |
| 36 | Malima | GW | 1.1 | Slight to moderate |

The ground water quality used in irrigation schemes of Wedera, Mekdela, Saboqo, Lucha, Mikeli, Malima and Bitata; River water quality used in Wotaro Dima; flood diversion sources from Warabele I and II, Hargeti, Dujuma, Bule Koloba, Qadalle and Jirarsa; spring water sources from Kase Hija and Dhoma were slight to moderate salinity hazard, requiring proper water management practices that include leaching water on top of irrigation water requirement in every irrigation events or occasionally at critical crop growth period depending on crop tolerance levels. Moreover, to maintain and/ or improve the soil and quality of irrigation water sources, sufficient leaching should be carried out before sowing until the salinity of the soil and water are conducive for crops. Other management options include proper irrigation water management, applying water as per the crop demand and at the right time with proper methods (Table 3-23).

The quality of water sources in the irrigation schemes of Abdi Boru, Bosha, Kebaro Ijara, Golgota, Mada Gudina, shelad, Argane, Sabba, Gola, Dire Sade, Galcha and Yeka lilisa were with ECw less than 0.7 dS/m which is safe for use in irrigation water management. However, regular monitoring is required before becoming a problem (Table 3-23).

Sodicity Hazard

The SAR value of the irrigation water ranged from 0.3 to 165.5% and the ECe value of irrigation water ranged from 0.1 to 8.7 dS/m. The result showed that about 33% of samples collected were categorized under none sodicity hazards. The sodium hazard of the irrigation water used for 5 irrigation schemes were slight to moderate and require amendments such as gypsum while for 12 irrigation schemes, the irrigation water was safe (none sodicity hazard) for irrigation purpose. The irrigation water used for 19 irrigation schemes was severe and cannot be used without blending with high saline water.  (Table 3-24).

The irrigation schemes in Erer Dodota, Tuji Gabisa upper, Saboqo, Galcha and Yeka lilisa have used the irrigation water with slight to moderate sodicity hazard. While the irrigation schemes in

Ababa, Bitata, Gidabo, Warabele I and II, Tuj Gibisa lower, Haro Adi Up and low, Bule Koloba, Jirarsa, Sabba and Malima have used the irrigation water with none sodicity hazard. The result also indicated that about 53% of samples collected were categorized severe sodicity hazard class The irrigation schemes in this category include Abdi Boru, Bosha, Kebaro Ijara, Golgota, Mada Gudina, Shelad, Wotaro Dima, Kase hija, Hargeti, Dhoma, Dujuma, Argane, Qadalle, Gola, Wedera, Mekdela, Dire Sade, Lucha and Mikaeli (Table 3-24).

Table 3-24  Sodicity hazard of irrigation water for selected irrigation schemes

| S.No | Schemes Name | Water Source | Sodicity Hazard |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | SAR (%) | ECw (dS/m) | Degree of restriction for use |
| 1 | Ababa | Diversion | 1.4 | 6.2 | None |
| 2 | Bitata | GW | 0.3 | 2.9 | None |
| 3 | Gidabo | River Diversion | 0.5 | 8.7 | None |
| 4 | Warabele I | Flood diversion | 0.6 | 2.6 | None |
| 5 | Warabele II | Flood diversion | 1.4 | 2.9 | None |
| 6 | Abdi Boru | River diversion | 32.5 | 0.6 | Severe |
| 7 | Bosha | Spring | 98.2 | 0.6 | Severe |
| 8 | Kebaro Ijara | River Diversion | 113.9 | 0.1 | Severe |
| 9 | Golgota | River diversion | 112.7 | 0.2 | Severe |
| 10 | Mada Gudina | River diversion | 86.1 | 0.6 | Severe |
| 11 | Shelad | River diversion | 74.3 | 0.6 | Severe |
| 12 | Wotaro Dima | River diversion | 144.6 | 0.7 | Severe |
| 13 | Erer dodota | GW | 93.5 | 4.0 | Slight to moderate |
| 14 | Tuj gabisa upper | GW | 60.4 | 4.1 | Slight to moderate |
| 15 | Tuji gebisa lower | GW | 5.4 | 3.7 | None |
| 16 | Kase hija | Spring | 32.0 | 1.1 | Severe |
| 17 | Hargeti | Flood diversion | 21.5 | 0.8 | Severe |
| 18 | Dhoma | Spring | 20.9 | 1.1 | Severe |
| 19 | Dujuma | Flood diversion | 165.5 | 1.8 | Severe |
| 20 | Haro adi Up | GW | 105.3 | 8.4 | None |
| 21 | Haro adi low | GW | 3.2 | 5.4 | None |
| 22 | Bule Koloba | Flood diversion | - | 1.6 | None |
| 23 | Saboqo | GW | 6.8 | 1.7 | Slight to moderate |
| 24 | Argane | Flood diversion | 19.3 | 0.4 | Severe |
| 25 | Qadalle | Flood diversion | 39.2 | 1.1 | Severe |
| 26 | Jirarsa | Flood diversion | - | 1.3 | None |
| 27 | Sabba | Flood diversion | - | 0.6 | None |
| 28 | Gola | River diversion | 17.7 | 0.6 | Severe |
| 29 | Wedera | GW | 35.7 | 1.5 | Severe |
| 30 | Mekdela | GW | 33.4 | 2.0 | Severe |
| 31 | Diresade | River diversion | 13.6 | 0.6 | Severe |
| 32 | Lucha | GW | 29.6 | 2.1 | Severe |
| 33 | Mikeli | GW | 30.1 | 1.7 | Severe |
| 34 | Galcha | River | 11.2 | 0.6 | Slight to moderate |
| 35 | Yeka lilisa | River | 2.1 | 0.2 | Slight to moderate |
| 36 | Malima | GW | 1.7 | 1.1 | None |

Based on the results of water analyses that were carried out on samples taken from some groundwater it can be concluded that water from the probes situated especially in East Hararge namely, Erer Dodota, Haramaya Tuji-Gabisa, upper and Haro adi upper are very high sodicity hazard and, while in Guji the results of analyzed water samples categorized in a range of very high saline and low sodicity hazards (Table 3-24)

The above-listed irrigation water from groundwater sources is approximately found at an average depth of 2 m to 3 m which is very near to the surface. Thus, there is a probability that occurrence of very high salinity and very high sodicity at the same time.  A similar study indicated that groundwater levels at an average depth of about 1.83 m or near depth are highly susceptible to salinity or sodicity development (Seelig, 2000). It also indicated that the occurrence of very high sodicity and salinity levels are related to soil genetic factors which are from groundwater of the past than to the current rainfall pattern (Shaw et al, 1994).

### 3.3.5. Soil management options

Based on the results indicated in sections 3.3.2 to 3.3.3 analysis and classification possible mitigation measures are given in Table 3-25.

Table 3-25. Soil Management Options.

| Irrigation schemes | Soil salt classes | Soil management options |
| --- | --- | --- |
| Saba, Jirarsa, Erere, Keseja. Yek-lelisa, Gelcha, Dire Sadi, Malima, wodera and Mikael, Mekdela, Lucha Dembel, Bosha 1, Sheled, Wataro dino, Golgota, Golgota, K, J,, M GUD , Abdi bori, Ababa, Bitata, WBL_ I and WBL_ I | Sodic soils ECe < 4 dS/m and ESP > 15 | - Apply chemical amendments to replace sodium to improve soil permeability - Incorporate organic matter to further improve infiltration rate and permeability of the soi |
| Qadalle, Haro Adi, Wataro dino, Abdi bori, Gidabo and WBL_ II | Saline sodic soils ECe > 4 dS/m and ESP >15 | - Apply chemical amendments to improve soil permeability - Apply heavy leaching water to leach out all the exchangeable Na+ and the saline salt |
| Qadalle, Dhoma, Hargeti, Haro Adi, Tuji Gabisa, Gidabo | Saline ECe > 4 dS/m and ESP <15 | - Conduct intermittent or continuous leaching programs. - grow salt-tolerant crops (barley, sorghum, etc). |

### 3.3.6. Water management options

Based on the results indicated in sections 3.3.4 analysis and classification possible mitigation measures are given in Table 3.26.

Table 3-26. Water Management Options.

| Irrigation schemes | Water quality challenges | Water management options |
| --- | --- | --- |
| Bitata, Warabele I and I, Wotaro Dima, Kase Hija, Hargeti, Dhoma, Dujuma, Bule Koloba, Saboqo, Qadalle, Jirarsa, Wedera, Mekdela, Lucha, Mikeli, Malima | Slightly to moderately Salinity Hazard | - Add leaching practice in every irrigation events and/ or occasionally at critical crop growth period as necessary. - Growing salt tolerant crops. - Maintain periodic water quality assessment |
| Ababa, Gidabo, Erer dodota, Tuj gabisa upper and lower, , Haro adi Up and low, | Severe salinity hazard. | - Blend saline water with fresh water. - Use the water in sodic soil - Avoiding instead of using as irrigation water |
| Erer dodota, Tuj gabisa upper, Saboqo, Galcha, | Slightly to moderately sodicity hazard | - Use amendments to blend with the water or mix the chemicals with the soil. - Blend with saline irrigation water - Use of improved irrigation water management practices together with organic matter incorporation |
| Abdi Boru, Bosha, Kebaro Ijara, Golgota, Mada Gudina, Shelad, Wotaro Dima, Kase hija, Hargeti, Dhoma, Dujuma, Argane, Qadalle, Gola, Wedera, Mekdela, Diresade, Lucha, Mikeli | Severe sodicity hazard | Blending with fresh water and/ or incorporate organic matter Avoid from using to maintain the soil structure |

### 3.3.7. Conclusion and Recommendation

Conclusion

Salinity control is a major objective of irrigation management, even though, the primary objective of irrigation is to maintain soil matric potential in a range suitable for optimal crop yield. In Oromia, Harari and Dire Dawa regions, about 4764 ha farmland and 36 irrigation water resources sites were assessed for soil and water salinity. Of the 32 selected schemes in the three regions, 5.9% were non-saline, 86.6% slight to strongly saline and 7.4% was very strongly to saline soil. Whereas the sodicity assessment indicated that 29.5% was non-sodic, 45.9% slight to moderately sodic and 24.6% strongly to very strongly sodic soils. Generally, the salt affected soils in three regions could be classified as 19.2% non-saline and non-sodic, 46.8% sodic, 17.8% saline sodic and 16.2% was saline soils.

The salinity hazard of irrigation water used in 16 irrigation schemes were slightly to moderate and while the irrigation water used in 13 irrigation schemes were safe for irrigation purposes. The irrigation water used in seven irrigation schemes was severe.

The sodium hazard of the irrigation water used in five irrigation schemes of Oromia region were slightly moderate. Whilst for 12 irrigation schemes, the irrigation water used were safe while the irrigation water used 19 irrigation schemes were severe.

Recommendation

  - The major objective of controlling salinity in the soil profile is to improve soil water availability to the crop thereby creating favorable conditions for plant growth and improve productivity.

- Assess, conduct regular monitor of depth to ground water table and quality

- For efficient irrigation water management, proper land leveling/ smopthening should be carried out

- Identify field application efficiency for proper irrigation scheduling and irrigation application (Frequency, rate and duration of application based on crop, soil and water)

- Assess and apply leaching requirement based on crop, soil physical and chemical characteristics, and water qulity.

- Provide training to farmers, DA, extension agents and others associated agricultural activities

  - In farmlands with slight to strong saline soils, yields of major crops could be affected and for cultivation of most economically important crops, leaching management practices is required to maintain a favorable salt balance in the root zone.

  - In the farmlands with very strongly to extremely saline soils, only tolerant to very tolerant crops can be grown and leaching should be accompanied during the cropping season to create a favorable salt balance in the plant roots. Otherwise, reclamation procedure is required in the absence of plant to restore productivity through heavy leaching practice such as intermittent and continuous leaching program.

  - In the farmlands with slightly to moderately sodic soils, most tolerant crops can be grown and chemical amendments including gypsum need to be applied to improve the soil air and water permeability.

  - In the farmlands with strong to very strongly sodic soils, must apply water relatively high in calcium and magnesium. In heavy textured clay soils, permeability is severely hindered and hence requires reclamation by adding gypsum and organic matter along with heavy leaching program.

  - Both the quantity and quality of water supply are important for irrigation. The use of poor-quality irrigation water requires special management practices to maintain soil and crop productivity. The irrigation water quality from 16 sources were with slight to moderate salinity hazard, requiring additional leaching water in every irrigation event or occasionally at critical crop growth period to maintain soil and crop productivity.

  - The irrigation water quality from seven sources was with severe salinity hazard, requiring blending with good quality of water or the use of such soil in sodic soil.

  - The irrigation water qualities from five sources were with slight to moderate sodicity hazard, requiring blending with saline irrigation water or by adding chemical amendments like gypsum.

  - The irrigation water quality from 19 sources was with severe sodicity hazard, requiring chemical amendments that may be incorporated to the soil or mixed with water. The soil permeability could be improved by incorporating organic matter to facilitate the leaching process.

## The State of Salinity Problems in Small Scale Irrigation Schemes and Possible Mitigation Measures: The Case of Somali Region

Dahir Yosuf Abdi, Mohamed Omer Yusuf, and Muhumed Hiss Osman

### Characterization of selected irrigation schemes

The desk review of a report by OWWDSE (2012) outlines the varying agro-climatic conditions of the Harawa-Erer Sub-Basin (HESB), which range from arid lowlands to sub-moist lowlands. The study schemes are primarily located in a sub-moist agroecological zone, characterized by bimodal rainfall patterns locally known as Gu’u (March to May) and Karan (July to September). The average annual rainfall in this region ranges from 450 to 800 mm, with mean temperatures between 20 and 22.5°C.

In the Lower Shebelle River Basin (LShRB), the average annual rainfall varies between 200 and 450 mm, with maximum temperatures reaching around 35°C and minimum temperatures around 23°C, which are typical of arid to semi-arid conditions. Moreover, according to Dahir et al. (2022), the land cover in the LShRB comprises 35.3% potentially irrigable land, 19.66% cultivated land, 18.56% bare land, and other categories.

Following consultations with the Bureau of Agriculture (BoA), 15 irrigation schemes were selected. The irrigated areas of these selected small-scale irrigation (SSI) schemes range from 3.81 to 61.04 hectares, with water sourced from both groundwater and surface water from the Shebelle River. The major crops cultivated include cereals (maize, sorghum, wheat) and cash crops (mango, papaya, banana, among others).

The dominant soil types in HESB include Fluvisols, Cambisols, and Vertisols, while in LShRB, Calcaric Fluvisols and Chromic Vertisols are prevalent. Soil salinity and sodicity indicators are used to classify both soil and water quality. Laboratory analyses revealed electrical conductivity of water (ECw) values ranging from low to very high, and sodium adsorption ratio (SAR) values also classified from low to very high, following the modified USSLS standards (Shahid and Mahmoudi, 2014).

The study was conducted at 15 small-scale irrigation schemes selected from the alluvial plains along the Lower Shabelle River Basin (LShRB) and the Harawa-Erer Sub-Basin (HESB). Administratively, these schemes are located in the Gode, Adadle, Ber’ano, Dambel, and Harawo districts, which are part of the Shebelle, Fafan, and Sitti zones of the Somali Regional State. The Shebelle zone falls under the LShRB, whereas the Fafan and Sitti zones are part of the HESB. The Lower Shabelle River Basin (LShRB) lies between 7°6′40″ N latitude and 44°23′0″ E longitude, while the Harawa-Erer Sub-Basin (HESB) is located between 41°17′36.86″ E and 43°18′7.53″ E longitude, and 9°27′41.35″ N to 11°5′6.43″ N latitude (Figure 3.12).

Figure 3-12 Selected irrigation schemes.

The topographic features of the study schemes are lowland, consisting of flat to gently sloping plains and hilly terrain. The altitude ranges between 250 and 290 meters in the LShRB of Shebelle Zone, and between 1,000 and 1,200 meters above sea level in the HESB of Fafan and Sitti zones.

In this study, 15 soil profiles from depths of 0–120 cm was excavated to examine the trend of soil salinity and sodicity in the study areas. Additionally, 157 surface soil samples (0–30 cm depth) were collected from the schemes to prepare a spatial distribution map of salt-affected areas. Furthermore, 15 water samples were taken from various points of the irrigation water sources in the schemes specifically from the source of irrigation, primary canals, and field canals.

### Mapping and analysis of surface saline and sodic soils

#### Saline Soils

In Somli region, the analysis results indicated a wide variation on surface soil salinity (0-30 cm), which was related to the agro-ecological conditions of their locations.  Based on the FAO (2008) soil salinity classification, the salinity levels across all study sites were mapped as follows: 2.3% none saline, 25.9% slightly saline, 53.9% moderately saline, 15.2% strongly saline, 2.4% very strongly saline, and 0.3% extremely saline soils. Accordingly, soil samples collected from irrigation schemes located in hot to warm lowland agro-ecologies have shown higher ECe values, ranging from 0.9 dS/m to 8.96 dS/m for Arabi and Armo study sites, respectively. In the tepid to cool midland agro-ecologies, the ECe values ranged from 2.0 dS/m at Lab to 17.4 dS/m at Godie study sites. More severe soil salinity problems were observed in irrigation schemes along the Wabi-Shabelle River Basin compared to those in the Harawa watershed, likely due to variations in agro-climatic conditions (Table 3-27 and Fig 3-13).

The study irrigation schemes at Armo, Agarwayne, Arabi-1, and Arabi-2, which represent the sub-moist lowland agro-ecology, were also affected by salinity problems. Their respective strongly saline areas were mapped as 0.7% for Agarwayne, 0.7% for Arabi-1, and 0.8% for Armo. Similarly, portions of their total areas cover 1.0% for Armo, 1.5% for Agarwayne, 0.3% for Arabi-1, and 6.7% for Arabi-2 were classified as non-saline (Table 3-27 and Fig 3-13).

The irrigation schemes in Armo (1%), Agarweyne (1.5%), Arabi-1 (0.3%), Arabi-2 (6.7%), Lab (0.1%), Ber’ano-1 (0.2), Gode-3 (2.1%). Gode-4 (0.1%), Badelo’ AD-1 (18.1%), Badelo’ AD-2 (2.1%) and Jerey (2.6%) were classified as none saline soils and irrigation schemes in Armo, Agarweyne, Arabi-1, Lab, Ber’ano-1, Gode-1, Gode-3. Gode-4, Badelo’ AD-1 and AD-2, Bafeyd and Jerey were classified slightly to strongly with EC varying from 0.51% to 80 %. The irrigation scheme at Arebi-2 was affected with sightly to moderately saline soils with 48.4% and 44.9%, respectively, while the irrigation schemes at Gode-2 was affected with sightly to extremely saline soils varying from 0.5% to 43.6% (Table 3-27 and Fig 3-13).

On the contrary, with the exception of Lab, Badela’ad-1, Badela’ad-2, Godey-1, Jerey, and Bursaredo, the other schemes have shown a strongly saline soil area of less than 5.1%, compared to the spatial distribution observed in the remaining irrigation schemes. In the arid to semi-arid lowland agro-ecology of Godey-4, 17.7% to 69% of the total area, and at Dabafeyd, the entire area was severely affected by soil salinity problems, with EC values ranging from 4 to 8 dS/m. Moreover, within the total area of the Dabafeyd scheme, 35.4% was classified as very strongly saline (ECe values between 8 and 15 dS/m), and 4.4% as extremely saline (ECe value > 15 dS/m), indicating a significant proportion of the scheme is unusable due to salinity problems (Table 3.27 and Fig 3-13). Furthermore, in this agro-ecology, only a very small portion (< 2.6%) of the total area of the irrigation schemes was classified as non-saline, with ECe values less than 0.75 dS/m (Table 3-27 and Fig 3-13).

In general, the results indicate that the soils of the majority of the study schemes are affected by salinity problems, ranging from slightly saline to strongly saline soils, suggesting that without proper management, the salinity problem could expand in the future, eventually rendering the entire area unsuitable for agricultural production.

Table 3-27. Percent area coverage of surface saline soils in selected irrigation schemes

| S. No. | Schemes | Total Area (ha) | ECe (dS/m) |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | < 0.75 | 0.75 - 2.0 |  | 2 – 4 | 4 – 8 |  | 8 - 15 |  | >15 |
|  |  |  | Non-saline | Slightly saline |  | moderately saline | Strongly saline |  | Very strongly saline |  | Extremely saline |
|  |  |  | Area coverage (%) |  |  |  |  |  |  |  |  |
| 1 | Armo | 22.4 | 1.0 | 35.8 | 62.4 |  |  | 0.8 |  | 0.0 | 0.0 |
| 2 | Agarweyne | 14.8 | 1.5 | 45.9 | 51.9 |  |  | 0.7 |  | 0.0 | 0.0 |
| 3 | Arabi-1 | 30.3 | 0.3 | 41.6 | 57.4 |  |  | 0.7 |  | 0.0 | 0.0 |
| 4 | Arabi-2 | 17.5 | 6.7 | 48.4 | 44.9 |  |  | 0.0 |  | 0.0 | 0.0 |
| 5 | Lab | 22.4 | 0.1 | 14.8 | 80.0 |  |  | 5.1 |  | 0.0 | 0.0 |
| 6 | Ber`ano-1 | 5.8 | 0.2 | 10.4 | 46.6 |  |  | 42.9 |  | 0.0 | 0.0 |
| 7 | Gode-1 | 34.6 | 0.0 | 18.8 | 76.8 |  |  | 4.4 |  | 0.0 | 0.0 |
| 8 | Gode-2 | 6.8 | 0.0 | 0.5 | 16.2 |  |  | 43.6 |  | 35.4 | 4.4 |
| 9 | Gode-3 | 31.1 | 2.1 | 18.8 | 59.9 |  |  | 19.2 |  | 0.0 | 0.0 |
| 10 | Gode-4 | 17.6 | 0.1 | 10.1 | 72.3 |  |  | 17.6 |  | 0.0 | 0.0 |
| 11 | Badelo`ad-1 | 61 | 18.1 | 62.0 | 19.9 |  |  | 0.0 |  | 0.0 | 0.0 |
| 12 | Badelo`ad-2 | 19 | 2.1 | 32.1 | 63.2 |  |  | 2.6 |  | 0.0 | 0.0 |
| 13 | Dabafeyd | 19.2 | 0.0 | 3.1 | 27.5 |  |  | 69.5 |  | 0.0 | 0.0 |
| 14 | Jerey | 3.8 | 2.6 | 26.2 | 52.5 |  |  | 18.6 |  | 0.0 | 0.0 |
| 15 | Bursaredo | 7.1 | 0.0 | 19.7 | 77.5 |  |  | 2.8 |  | 0.0 | 0.0 |
|  | Sum | 313.4 | 2.3 | 25.9 | 53.9 |  |  | 15.2 |  | 2.4 | 0.3 |

Figure 3-13 Mapping of surface saline soils for selected irrigation schemes

Sodic Soils

Based on the ESP results, considerable variations were recorded among the soil samples from the study schemes. From the total farm land of 3.13.4 ha, 35.6% was non-sodic, 42.3% slightly sodic, 14.7% moderately sodic, 7.0% strongly sodic and 0.5% very strongly sodic soils (Table 3.28 and Fig. 3-14).

The irrigation schemes of Armo (50%), Agarweyne (90%), Arabi-1 (66%), Arabi-2 (87%), Lab (26%), Ber’ano-1(41%), Gode-1(12%), Gode-2 (6%), Gode-3 (19%), Gode-4 (24%), Badelo Ad-1 (13%), Badelo Ad-2 (8%), Dabafeyd (22%), Jerey (5%) and Bursaredo (78%) were categorized under non-sodic soils, whereas Bursaredo (22%) and Arebi-2 (13%) irrigation schemes were affected with slightly sodic soils. Irrigation schemes in Agarweyne, Arabi-1 and Gode-4 were affected with slightly to moderately sodic soils. Irrigation schemes in Armo, Lab, Ber’ano-1, Gode-1, Gode-2, Badelo Ad-1, Badelo Ad-2, Dabafeyd and Jerey were affected with slightly to strongly sodic soils. And Gode-3 irrigation scheme was affected with slight to very strongly sodic soils (Table 3.28 and Fig. 3-14).

Furthermore, Gode-3 irrigation scheme was the only scheme with nearly 7% of the land covered by very strongly sodic soils. In general, 62.2% of the soils in the studied schemes were observed to be slightly to moderately sodic soils, indicating the presence and potential expansion of sodicity problems, which require attention and management in the future (Table 3.28 and Fig. 3-14).

Table 3-28. Percent area coverage of surface sodic soils in selected irrigation schemes

| S. No. | Schemes | Total Area (ha) | ESP (%) |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | < 6 | 6 to 10 | 10 to 15 | 15 to 25 | >25 |
|  |  |  | Non-sodic | Slightly sodic | moderately sodic | Strongly sodic | Very strongly sodic |
|  |  |  | Area coverage (%) |  |  |  |  |
| 1 | Armo | 22.4 | 50.2 | 37.3 | 12.1 | 0.4 | 0.0 |
| 2 | Agarweyne | 14.8 | 89.8 | 9.9 | 0.3 | 0.0 | 0.0 |
| 3 | Arabi-1 | 30.3 | 66.4 | 30.9 | 2.7 | 0.0 | 0.0 |
| 4 | Arabi-2 | 17.5 | 86.9 | 13.1 | 0.0 | 0.0 | 0.0 |
| 5 | Lab | 22.4 | 25.6 | 52.9 | 18.8 | 2.7 | 0.0 |
| 6 | Ber`ano-1 | 5.8 | 41.2 | 35.3 | 23.3 | 0.2 | 0.0 |
| 7 | Gode-1 | 34.6 | 11.8 | 62.8 | 21.2 | 4.2 | 0.0 |
| 8 | Gode-2 | 6.8 | 5.6 | 75.9 | 15.2 | 3.3 | 0.0 |
| 9 | Gode-3 | 31.1 | 19.4 | 28.9 | 33.6 | 11.2 | 6.9 |
| 10 | Gode-4 | 17.6 | 24.4 | 75.3 | 0.3 | 0.0 | 0.0 |
| 11 | Badelo`ad-1 | 61 | 13.3 | 60.5 | 19.3 | 6.9 | 0.0 |
| 12 | Badelo`ad-2 | 19 | 8.4 | 57.9 | 31.6 | 2.1 | 0.0 |
| 13 | Dabafeyd | 19.2 | 22.1 | 55.1 | 16.3 | 6.6 | 0.0 |
| 14 | Jerey | 3.8 | 5.3 | 10.5 | 23.7 | 60.8 | 0.0 |
| 15 | Bursaredo | 7.1 | 77.5 | 22.5 | 0.0 | 0.0 | 0.0 |
|  | Sum | 313.4 | 35.6 | 42.3 | 14.7 | 7.0 | 0.5 |

Figure 3-14 Percent area coverage of surface sodic soils in selected irrigation schemes

### Classification of salt affected soils

Considering the spatial distribution of salt-affected soils, the majority of the study schemes were dominated by saline soils with 42% of the total area, followed by 29% classified as non-saline non-sodic soils, and 27% as saline-sodic soils. In contrast, about 3% of the total area across all schemes was mapped as sodic soils (Table 3.29 and Fig. 3-13 and 14).

All irrigation schemes had a non-saline non-sodic soil that variable from 0.1% to 67.6%. Armo (63%) and Arbi-2 (45%) irrigation schemes were affected with saline soils and Burasado (99.9%) was affected with saline-sodic soil. Irrigation schemes at Ber’ano-1, Dabafeyd and Jerey were affected with saline and saline-sodic soils. Irrigation schemes in Agarweyne, Arabi-1and Gode-4 were affected with saline and sodic soils. Irrigation schemes in Lab, Gode-1, Gode-2, Gode-3, Badelo ad-1 and Badelo ad-2 were affected by saline, saline-sodic and sodic soils (Table 3.29 and Fig. 3-13 and 14).

The results of this study also indicate that the Armo, Agarweyne, Arabi-1, Arabi-2, and Gode-4 schemes did not show any presence of saline-sodic soils. Moreover, Armo, Arbabi-2, Ber’ano-1, Gode-2, Dabafeyd, Jerey and Burasado schemes did not show any presence of sodic soils (Table 3.29 and Fig. 3-13 and 14).

Table 3-29. Salt affected soil classes in selected irrigation schemes

| S. No. | Scheme | Total area (ha) | Salt-affected soil classes |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | ECe < 4, ESP < 15 | ECe > 4, ESP < 15 | ECe > 4, ESP >15 | ECe < 4, ESP > 15 |
|  |  |  | Non-saline non-sodic | Saline | Saline-sodic | Sodic |
|  |  |  | Area coverage (%) |  |  |  |
| 1 | Armo | 22.4 | 36.7 | 63.3 | 0.0 | 0.0 |
| 2 | Agarweyne | 14.8 | 1.6 | 98.3 | 0.0 | 0.1 |
| 3 | Arabi-1 | 29.4 | 39.1 | 58.2 | 0.0 | 2.7 |
| 4 | Arabi-2 | 18.0 | 55.5 | 44.5 | 0.0 | 0.0 |
| 5 | Lab | 22.4 | 7.9 | 46.7 | 38.4 | 7.1 |
| 6 | Ber`Ano-1 | 5.8 | 41.2 | 35.0 | 23.8 | 0.0 |
| 7 | Gode-1 | 34.6 | 67.6 | 7.0 | 16.8 | 8.6 |
| 8 | Gode-2 | 6.8 | 16.6 | 64.9 | 18.5 | 0.0 |
| 9 | Gode-3 | 31.1 | 63.5 | 18.4 | 0.8 | 17.3 |
| 10 | Gode-4 | 17.6 | 20.8 | 79.0 | 0.0 | 0.3 |
| 11 | Badelo`ad-1 | 61.0 | 18 | 55.1 | 26.2 | 0.7 |
| 12 | Badelo`ad-2 | 18.5 | 34.2 | 2.4 | 61.6 | 1.8 |
| 13 | Dabafeyd | 19.2 | 27.2 | 49.9 | 22.8 | 0.0 |
| 14 | Jerey | 3.8 | 0.5 | 5.3 | 94.2 | 0.0 |
| 15 | Bursaredo | 7.1 | 0.1 | 0.0 | 99.9 | 0.0 |
| Sum |  |  |  |  |  |  |

The result also revealed that the extent of soil salinity was higher at irrigation schemes in Harawa watershed compared to those in Wabe-Shebelle basin areas which was mostly dominated by saline-sodic soils (Table 3.29 and Fig. 3-13 and 14). Irrigation schemes of Lab (7.1%), Gode-1 (8.6%) and Gode-3 (17.3%) have shown emerging sodic soils compared to others despite to this fact that majority of the study schemes were affected by salinity problem.

Salt characteristics of Soil Profile Pits

The ECe and ESP values in all SSI schemes tended to decrease with increasing soil depth consistently. Such a salt distribution trend is a typical feature of arid and semi-arid areas where net water movement is upward that favored accumulation of relatively higher salt accumulation in the upper soil layers. Of all SSI schemes assessed, only Bursaredo and Agarwayne schemes were found to being qualified as no salinity and sodicity problem of significant importance. However, soils of these schemes appeared to have appreciable quantity of salts nearly around critical limit of ECe for qualifying soil as saline. It is apparent to recognize soils of these schemes as potentially salt-affected that could be converted into actual salinity-affected soil in near feature unless preventive measure is in place timely(Table 28)..

Soils of Jerey, Gode-1, Gode-2, and Gode-3 had excess amount of soluble salt throughout soil profile and generally fall under saline soil class. Except those schemes described as normal soil (free of salinity and sodicity problem) the upper 0-30 cm soil layer of all schemes behaves either saline or saline sodic soil class.

The distribution of the soil salinity, ECe decreased consistently with depth in all the soil profiles studied at all locations of the two agro-ecologies and have shown average ECe values of 13.3 dS/m at 0-30, 9.8 dS/m at 30-60, 7.3 dS/m at 60-90, and 5.5 dS/m at 90-120 cm soil depth (Table 28). The relative decrease of soluble salts within the depth of the profiles suggests that upward (capillary) movement of salts in saline ground water is more dominant than downward (leaching) movement of salts with irrigation and/or rain waters near the surface soil. This might also be caused by poor irrigation water management and inadequate drainage system of the irrigated fields.

The ECe values of surface layers in the studied profiles ranged from 3.56 to 13.47 dS/m (Table 28). With the exception of Agarweyne, Gode-4, and Badelo`ad-1, values of all other sites were greater than the lower limit (4 dS/m) set by the US Salinity Laboratory Staff (1954) to qualify for the saline soil class, regardless of theESP values (Table 28). Even those sites with relatively low EC values were likely be transformed into potentially saline soil in the near future unless appropriate prevention and control measures are put in place.

However, the ECe values measured in the surface soils of Armo, Arabi-1, Arabi-2, Ber`ano-1, Gode-1 and Gode-3 ranged from 4.65 to 7.78 dS/m, indicating a high-level of soluble salt concentration at which the growth and productivity of most agricultural crops are affected due to soil salinity (Richards, 1954; Landon, 1991) (Table 28). The ECe values in the surface soil (0-30 cm) showed high magnitude at Lab (8.6 dS/m), Gode-1 (10.6 dS/m), Gode-2 (9.6 dS/m), Gode-3 (12.3 dS/m), Dabafeyd (12.9 dS/m) and Jerey (12.8 dS/m) (Table 28) compared to the lower layers.

The ESP values throughout the soil profile pits in all assessed irrigated sites with   groundwater source, ranged from 1.589 to 5.577 % which were far below the lower limit (15%) set by the Richards, (1954) (Table 28). The soils in these areas is non-sodic throughout their profile. Generally, the ESP values of Armo and Agarweyne decreased consistently with depth in the profiles, while the relationship of ESP with depth of the profile was not consistent for Arabi-1 and Arabi-2.

Regrading to irrigated areas with river water, the ESP values were generally decreasing with depth in the soil profiles for most of the studied sites, though their relationship with depth of the profiles was not consistent. The ESP values of the surface layers of the soil profiles from Lab, Ber`ano-1, Gode-1, Badelo`ad-2 and Gode-3 were all greater than the lower limit (15%) set by the Richards, (1954) and the ECe values were also greater than 4.0dS/m, and these soils can be classified as saline-sodic soil. Soils of all other study sites, on the other hand, do not qualify to be classified as sodic and/or saline-sodic soil, since the ESP values throughout their profiles are below the lower limit (15%) as per the criteria established by the Richards, (1954).

Generally, considering the ECe and ESP values, it can be concluded that soils from Lab, Ber`ano-1, Gode-1, Badelo`ad-2and Dabafeyd can be classified as saline sodic for EC > 4dS/m and ESP > 15%, whereas soils from Armo, Arabi-1, Arabi-2, Gode-3, Gode-2, Jerey, Gode-4, Bursaredo and Agarweyne qualify to be classified as saline soil for EC >4dS/m and ESP <15% and soils from Agarweyne, and Badelo`ad-1 are potentially saline soil.

The assessment is similar with Murphy (1968) who reported that Wabi-Shebelle areas are calcareous and gypsiferous in certain areas, while containing excessive number of soluble salts and exchangeable sodium in others.  Moreover, Murphy (1968) indicated that the alluvial soils along the Wabi-Shebelle River basin are good for agricultural production, but these soils have some problems like salinity, sodicity, swampy and high content of CaCO3 and CaSO4.

### Irrigation water quality

There are two types of salt problems in irrigation water that are very different in their constituent, those associated with the total salinity, designated as salinity hazard and those associated with sodium, designated as sodium hazard. Irrigation water sources used in the different schemes were four from ground water and 11 from river.

Salinity Hazard

The Electrical Conductivity of irrigation water (ECw) values ranged from 0.15 to 0.30 dS/m. According to FAO (1985), the degree of restriction for use in irrigation was none being below 0.70 dS/m. (Table 3-30).

Table 3-30.Salinity hazard of irrigation water for selected schemes

| S. No | Schemes Name | Water Source | Sality Hazard |  |
| --- | --- | --- | --- | --- |
|  |  |  | ECw (dS/m) | Degree of restriction for use |
| 1 | Armo | Ground | 0.27 | None |
| 2 | Agarweyne | Ground | 0.24 | None |
| 3 | Arabi-1 | Ground | 0.30 | None |
| 4 | Arabi-2 | Ground | 0.30 | None |
| 5 | Lab | River | 0.28 | None |
| 6 | Ber`ano-1 | River | 0.18 | None |
| 7 | Gode-1 | River | 0.16 | None |
| 8 | Gode-2 | River | 0.18 | None |
| 9 | Gode-3 | River | 0.15 | None |
| 10 | Gode-4 | River | 0.16 | None |
| 11 | Badelo`ad-1 | River | 0.29 | None |
| 12 | Badelo`ad-2 | River | 0.19 | None |
| 13 | Dabafeyd | River | 0.15 | None |
| 14 | Jerey | River | 0.16 | None |
| 15 | Bursaredo | River | 0.18 | None |

Sodicity Hazard

The sodicity hazard of irrigation water was classified according to SAR and ECe values and presented in Table 3-31. The SAR values of the sampled groundwater and surface water ranged between 3.2 to 9.3 percent and while for that of water was found in a narrow range between 0.15 and 0.3 dS/m. The very low values of ECw Aggravated the dominance of Na+ in the water. The water sources used in Arabi-1 and Arabi-2 irrigation schemes were found to be slight to moderate range and whilst for the remaining 13 water sources were found to be unsafe and classified as severe according to FAO (1985).

The water quality condition, where low saline water is applied to moderate to high soil sodicity, presents a serious risk of soil structure degradation, which can significantly impair soil infiltration rate and soil permeability.

Table 3-31.Sodicity hazard of irrigation water for selected schemes

| S. No | Schemes Name | Water Source | Sodicity Hazard |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | SAR (%) | ECw (dS/m) | Degree of restriction for use |
| 1 | Armo | Ground | 9.3 | 0.27 | Severe |
| 2 | Agarweyne | Ground | 4.8 | 0.24 | Severe |
| 3 | Arabi-1 | Ground | 3.2 | 0.3 | Slight to moderate |
| 4 | Arabi-2 | Ground | 4.6 | 0.3 | Slight to moderate |
| 5 | Lab | River | 7.2 | 0.28 | Severe |
| 6 | Ber`ano-1 | River | 5.1 | 0.18 | Severe |
| 7 | Gode-1 | River | 4.8 | 0.16 | Severe |
| 8 | Gode-2 | River | 3.6 | 0.18 | Severe |
| 9 | Gode-3 | River | 4.6 | 0.15 | Severe |
| 10 | Gode-4 | River | 4.5 | 0.16 | Severe |
| 11 | Badelo`ad-1 | River | 4.6 | 0.29 | Severe |
| 12 | Badelo`ad-2 | River | 4.9 | 0.19 | Severe |
| 13 | Dabafeyd | River | 4.5 | 0.15 | Severe |
| 14 | Jerey | River | 4.4 | 0.16 | Severe |
| 15 | Bursaredo | River | 4.7 | 0.18 | Severe |

Therefore, proper irrigation water management is essential to maintain soil structure and prevent productivity loss. Among the recommended measures, blending these low-salinity water sources with natural saline springs is considered a best practice. This is based on the principle that water with higher salinity relative to soil salinity tends to promote flocculation (aggregation) of soil particles, thereby helping to preserve soil structure and improve water infiltration

### Soil management options

Based on the results indicated in sections 3.4.2 to 3.4.3 analysis and classification possible mitigation measures are given in Table 3-32.

Table 3-32. Proposed Soil management options for each affected scheme

| Irrigation schemes | Soil salt classes | Soil management options |
| --- | --- | --- |
| Lab, Ber'ano-1, Gode-1, Gode-2, Gode-3, Badelo`ad-1, Badelo’ad-2, Dabafeyd, Jerey, Bursaredo | Saline-sodic (ECe > 4 dS/m, ESP > 15%) | - Apply gypsum to replace sodium with calcium. - Leach out the Na+ and saline salts from the root zone. - Monitor, manage and control ground water level. - Grow salt-tolerant crops. - Incorporate organic matter to improve soil permeability. |
| Armo, Agarweyne, Arabi-1, Arabi-2, Lab, Ber`Ano-1, Gode-1, Gode-2, Gode-3, Gode-4, Badelo`ad-1, Badelo`ad-2, Dabafeyd, Jerey | Saline soils (ECe > 4 dS/m, ESP < 15%) | - Leaching of salts using good-quality irrigation water. - Grow salt-tolerant crop and use crop rotation. - Water harvesting for leaching during rainy seasons. |
| Agarweyne, Arabi-1, Lab, Gode-1, Gode-3, Gode-4 Badelo’ad-1, Badelo’ad-2 | Sodic soils (ECe < 4 dS/n, ESP > 15) | - Continues contouring and assessment needed - Apply organic amendments to enhance soil resilience - Prevent over-irrigation to avoid salt mobilization |

### Water management options

Based on the results indicated in sections 3.4.4 analysis and classification possible mitigation measures are given in Table 3.33.

Table 3-33. Water management options

| Irrigation schemes | Water quality challenges |  | Water management options |
| --- | --- | --- | --- |
| All irrigation schemes | None salinity hazard (<0.70 dS/m.) | - Suitable for agricultural water use. - Control and monitored soil and irrigation water quality |  |
| Arabi-1 and Arabi-2 | Slight to moderate sodicity hazard. | - Mix chemical amendments with irrigation water - Blend with saline irrigation water - Incorporate organic matter in to soils |  |
| Armo, Agarweyne, Lab, Gode-1, Gode-2, Gode-3, Gode-4, Dabafeyd, Jerey, Bursaredo, Ber’ano-1, Badelo’ad-1, and Badelo’ad-2 | Severe sodicity hazard. | - Belend with high level of saline irrigation water - Incorporate organic matter in the soil to improve soil permeability |  |

### Conclusion and Recommendation

#### Conclusion

Salinity control is a major objective of irrigation management, eventhough, the primary objective of irrigation is to maintain soil matric potential in a range suitable for optimal crop yield. Somali regions, 313.1 ha farm land and 15 irrigation water resource sites in 15 irrigation schemes were assessed for soil and water salinity problem. From the selected 15 schemes in the region, 4.4% was non-saline, 94.7% slight to strongly saline and 0.9% was very strongly to extremely saline soils. Whereas the sodicity assessment indicated that 32.7% was non-sodic, 62.2% slight to moderetly sodic and 5.2% strongly to very strongly sodic soils. Generally, the salt affected soils in the region could be classified as 33.5% non-saline and non-sodic, 3.7% sodic, 19.1% saline sodic and 43.7% was saline soils.

The salinity hazard of irrigation water used from all 15 water sources was safe and had no restriction for use in irrigation. The sodicity hazard of irrigation water used from 13 water sources was lightly to moderate while from 2 water sources was severe.

#### Recommendation

•	The major objective of controlling salinity in the soil profile is to improve soil water availability to the crop thereby creating favourable condition for plant growth and improve productivity.

- Assess, conduct regular monitor of depth to ground water table and quality

- For efficient irrigation water management, proper land leveling/ smopthening should be carried out

- Identify field application efficiency for proper irrigation scheduling and irrigation application (Frequency, rate and duration of application based on crop, soil and water)

- Assess and apply leaching requirement based on crop, soil physical and chemical characteristics, and water qulity.

- Provide training to farmers, DA, extension agents and others associated agricultural activities

•	In the farm lands with slight to strongly saline soils, yields of major crops could be affected and for cultivation of most economically important crops, leaching management practices is required to maintain a favourable salt balance in the root zone.

•	In the farm lands with very strongly to extremely saline soils, only tolerant to very tolerants crops can be grown and leaching should be accompanied during the cropping season to create a favorable salt balance in the plant roots. Otherwise, reclamation procedure is required in the absence of plant to restore productivity through heavy leaching practice such as intermittent and continuous leaching program.

•	In the farm lands with slightly to moderatly sodic soils, most tolerant crops can be grown and chemical ammendments including gypsum need to be applied to improve the soil air and water permeability.

•	In the farm lands with strongly to very strongly sodic soils, must apply water relatively high in calcium and magnesium. In heavy textured clay soils, permeability severely hindered and hence, requires reclamation by adding gypsum and organic matter along with heavy leaching program.

•	Both the quantity and quality of water supply are important for irrigation. The use of poor quality irrigation water requires special management practices to maintain the soil and crop productivity. The irrigation water qualities from 15 sources were safe from salinity hazard with no degree of restriction for use in irrigation.

•	The irrigation water quality from 2 sources was with severe sodicity hazard, requiring blending with good quality of water or the use of such soil in sodic soil.

•	The irrigation water quality from 13 sources was with slight to moderate sodicity hazard, requiring blending with saline irrigation water or by adding chemical ammendments like gypsum either in the water or in the soil.

## The State of Salinity Problems in Small Scale Irrigation Schemes and Possible Mitigation Measures: The Case of EX- SNNRP Region.

Tamirneh Kifle, Markose Habtewold, Tagesse Bekele, Medhin Madebo and Ibrahim Seid

### 3.5.1. Characterization of selected irrigation schemes

In consultation with regional BoA and the regional research institute, 24 salt-affected small-scale irrigation schemes were selected. During the discussion, the team emphasized the importance of this study, particularly in areas suspected to have salt-affected soils, which were identified for detailed investigation. It was agreed that the key factors to be included in the study are soil salinity, sodicity, water quality, and the characteristics of arid and semi-arid agro-climatic conditions. The discussion also emphasized the need for targeted interventions to address salinity challenges, such as the use of salt-tolerant crops and the implementation of soil reclamation techniques. By adopting sustainable irrigation practices and effective water management strategies, it is possible to enhance crop yields and improve soil health in the region.

From the desk review, the variability of soil electrical conductivity (EC) in the former SNNP Region ranged from moderate to strong. The distribution of exchangeable sodium percentage (ESP) values also showed significant variability (Walche et al., 2024). Moreover, the majority of salt-affected areas in the region are characterized by soil sodicity levels ranging from low to high risk

The study was conducted in Ex-SNNPR selected irrigation schemes. The geographical location of the region is located between 4.46°N - 8.48 °N latitude and 35.75°E - 38.3°E longitude with an altitude ranging from 298 - 3548 m.a.s.l. Agro-ecologically, the region is arid to semi-arid and humid, dry hot to midland. The region has an annual minimum and maximum temperature of 10°C and 29°C, respectively with a mean temperature of 21.04°C, with annual minimum rainfall of 337 mm, maximum of 1746 mm, and mean of 1082 mm (Fig. 3-15).

Figure 3-15.  Selected irrigation schemes.

The dominant crops grown in the region are cereal (maize, sorghum, teff. barley, and wheat), pules (haricot bean, faba bean), horticulture (onion, tomato, pepper, hot-pepper, cabbage, head-cabbage and carrot), fruits (banana, mango and papaya), root crops (Enset and sweet potato), industrial crops (sugar cane, tobacco and cotton) and oil-crop (groundnut, soybean and Sesame).

A reconnaissance survey was conducted across potential irrigation schemes in Wolayita, Gamo, Gofa, Konso, Gurage, Siltie, Kambata Tembaro, and South Omo Zones. Based on the findings, 24 irrigation schemes suspected to be affected by soil salinity were selected for detailed study. Soil and water samples, including groundwater, were collected from each scheme. The service year of the irrigation schemes ranges from 2 to 29 years (Table 3-34). These samples were collected from areas around the Rift Valley in the Ex-SNNP Region, and their distribution is illustrated on the Fig. 3-15.

Table 3-34. Selected irrigation schemes

| Zone | Scheme | Source of water | Year of start irrigation (E.C) | Agro-ecology |
| --- | --- | --- | --- | --- |
| Gofa | Mishkare | River | 1990 | Mid land |
| Konso | Geldaha | River | 2003 | Mid land |
| Gamo | Sego | Sego river | - | Mid land |
|  | Sile | Sego river | - | Mid land |
|  | Hare | Hare river | - | Low land |
|  | Lake Chamo | Lake | - | Low land |
|  | Lake Abaya (lante to Algea) | Lake | 2011 | Mid land |
|  | Lake Abaya (wajifo to Kola barena) |  | 2011 | Mid land |
| Gurage | Lebu | River | 1993 | Mid land |
| Siltie | Silti ground water users | Ground water | 1997 | Mid land |
|  | Bila wanja | River | 2000 | Mid land |
|  | Simbita | Ground water | 2009 | Mid land |

### 3.5.2. Mapping and analysis of surface saline and sodic soils

Saline Soils

The surface soil samples taken from selected irrigation schemes in Ex-SNNPR were classified as non-saline (7.3%), slightly saline (29.1%), moderately saline (18.4%), strongly saline (21.3%), very strongly saline (17.2%), and extremely saline (6.8%). According to FAO (2008) salinity classification, the irrigation scheme at Belate (3.2%), Hamido (73.4%) and Fulame (25.8%) fall in non-saline soils. Irrigation schemes Arseamoy (77.6%), Lobet (6%), Enchete (99%), Bilate Eta (96.8%), Abaya Bilate (27.3%) Hamido (26.6) and Fulame (74.2%) were affected with slightly saline soils. Irrigation schemes Arseamoy (20.1%), Arikol (15.3%), Delerele (5.6%), Lobet (11.8%), Enchete (1%), Dume (17.2%), Abaya Bilate (72.2%), Lake Abaya (13.8%) and Hare (100%) were affected with moderately saline soils. Irrigation schemes Arseamoy (2.3%), Arikol (84.7%), Delerele (13.3%), Lobet (39.2%), Duma (59.8%), Abaya Bilate (0.6%), Lake Abaya (Lante to alagea-42.4%) and Lake Abaya (Wajifo to kola barena-55.4%) were affected with strongly saline soils. Irrigation schemes Delerele (43.9%), Lobet (43.1%), Duma (21.7%), %), Lake Abaya (Lante to alagea-43.9%), Lake Abaya (Wajifo to kola barena-20.5%) and Lake chamo (68.1%) were affected by very strongly saline soils.  Irrigation schemes Delerele (37.2%), Duma (1.3%), Lake Abaya (Wajifo to kola barena-24.2%) and Lake chamo (32%) were affected by extremely saline soils (Table 3-35 and Fig.3-16 and 17).

Fulame

Arseamoy

Delerele

Abya Bilate

Abya Bilate

Abya Bilate

Figure 3-16. Sample pictures from South Omo irrigation Schemes.

Table 3-35. Percent area coverage of surface saline soils in selected irrigation schemes.

| S. No. | Schemes | Total Area (ha) | ECe (dS/m) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Non-Saline | Slightly saline | Moderately saline | Strongly saline | Very strongly saline | Extremely saline |
|  |  |  | < 0.75 | 0.75 – 2 | 2-4 | 4-8 | 8-15 | >15 |
|  |  |  | Area coverage (%) |  |  |  |  |  |
|  | Arseamoy | 33.4 | 0.00 | 77.60 | 20.06 | 2.34 | 0.00 | 0.00 |
|  | Arikol | 26.3 | 0.00 | 0.00 | 15.26 | 84.74 | 0.00 | 0.00 |
|  | Delerele | 24.8 | 0.00 | 0.00 | 5.64 | 13.29 | 43.88 | 37.20 |
|  | Lobet | 118.5 | 0.00 | 5.96 | 11.75 | 39.20 | 43.10 | 0.00 |
|  | Enchete | 607.1 | 0.00 | 99.00 | 1.00 | 0.00 | 0.00 | 0.00 |
|  | Duma | 209.9 | 0.00 | 0.00 | 17.22 | 59.75 | 21.69 | 1.33 |
|  | Bilate Eta | 2040.2 | 3.22 | 96.78 | 0.00 | 0.00 | 0.00 | 0.00 |
|  | Abaya Bilate | 595.4 | 0.00 | 27.27 | 72.19 | 0.59 | 0.00 | 0.00 |
|  | Hamido | 152.5 | 73.43 | 26.57 | 0.00 | 0.00 | 0.00 | 0.00 |
|  | Fulame | 447.8 | 25.77 | 74.23 | 0.00 | 0.00 | 0.00 | 0.00 |
|  | Lake abaya (lante to algea) | 2074.5 | 0.00 | 0.00 | 13.75 | 42.37 | 43.87 | 0.00 |
|  | Lake abaya (Wajifo to kola barena) | 885.0 | 0.00 | 0.00 | 0.00 | 55.44 | 20.46 | 24.20 |
|  | Hare | 976.95 | 0.00 | 0.00 | 100.00 | 0.00 | 0.00 | 0.00 |
|  | Lake Chamo | 602 | 0.00 | 0.00 | 0.00 | 0.00 | 68.06 | 31.96 |
| Sum |  | 8794.2 | 7.32 | 29.10 | 18.35 | 21.27 | 17.22 | 6.76 |

Figure 3-17. Mapping of saline soil in selected irrigation scheme

The slight to moderately saline soils, it is suitable for most salt tolerant crops and requires leaching management practices for most sensitive crops. To maintain and improve the soil salinity, it is important to consider leaching practices in every irrigation events. In Very strongly to extremely saline soils only highly salt tolerant crops can be grown and hence requires heavy leaching program before cultivation of any crops.

Sodic soils

The sodicity categories and respective percent area coverage in the Ex-SNNPR are: non-sodic (14.5%), slightly-sodic (15.5%), moderately-sodic (16.6%), strongly sodic (19.2%), and very strongly sodic (28.4%). Irrigation schemes Arseamoy (8.1%), Bilate Eta (0.7%), Abaya Bilate (99.9%), Lake Abaya (Lante to alagea-57.6%), Lake Abaya (Wajifo to kola barena-67.6%), Hare (78%) and Lake Chamo covered (86.1 %) were affected very strongly affected sodic soils. Irrigation schemes Arseamoy (15.5%), Delerele (20.3%), Enchete (100%), Duma (100%), Bilate Eta (5.4%), Abay Bilate (0.1%), Hamido (5.4%), Lake Abaya (Lante to alagea-42.4%), Lake Abaya (Wajifo to kola barena-32.4%), Hare (22%) and Lake Chamo covered (6.1 %) were affected with strongly sodic soils. Irrigation schemes Lake Chamo covered (29.5 %), Arikol (54%), Delerele (42.1%), Lobet (6.4%), Bilate Eta (87.8%), Hamido (10.3%) and Lake Chamo covered (2.3 %) were affected with moderately sodic soils. Irrigation schemes Arseamoy (28.3%), Arikol (41.7%), Delerele (15.8%), Lobet (92.3%), Bilate Eta (5.6%), Hamido (15.2%). Fulame (16.6%) and Lake Chamo covered (1.9 %) were affected with slightly sodic soils. However, irrigation schemes Arseamoy (18.6%), Arikol (4.3%), Delerele (21.8%), Lobet (1.4%), Bilate Eta (0.5%), Hamido (69.1%). Fulame (83.4%) and Lake Chamo (3.7 %) were non-sodic soils (Table 3-36 and Fig. 3-18).

Table 3-36. Sodic soil in selected irrigation schemes

| Name of Schemes | Total Area (ha) | Soil sodicity (%) |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Non-sodic | Slightly sodic | Moderately sodic | Strongly sodic | Very strongly sodic |
|  |  | <6 | 6 – 10 | 10 - 15 | 15 -o 25 | >25 |
|  |  | Area coverage (%) |  |  |  |  |
| Arseamoy | 33.4 | 18.62 | 28.25 | 29.54 | 15.53 | 8.10 |
| Arikol | 22.3 | 4.31 | 41.70 | 53.99 | 0.00 | 0.00 |
| Delerele | 24.8 | 21.80 | 15.83 | 42.06 | 20.31 | 0.00 |
| Lobot | 118.4 | 1.38 | 92.32 | 6.39 | 0.00 | 0.00 |
| Enchete | 607.2 | 0.00 | 0.00 | 0.00 | 100.00 | 0.00 |
| Duma | 209.9 | 0.00 | 0.00 | 0.00 | 100.00 | 0.00 |
| Bilate Eta | 2040.2 | 0.49 | 5.59 | 87.81 | 5.37 | 0.74 |
| Abaya Bilate | 595.6 | 0.00 | 0.00 | 0.00 | 0.07 | 99.92 |
| Hamido | 152.5 | 69.13 | 15.17 | 10.30 | 5.38 | 0.00 |
| Fulame | 447.8 | 83.36 | 16.64 | 0.00 | 0.00 | 0.00 |
| Lake abaya (lante to algea) | 2074.5 | 0.00 | 0.00 | 0.00 | 42.40 | 57.60 |
| Lake abaya (Wajifo to kola barena) | 885.0 | 0.00 | 0.00 | 0.00 | 32.44 | 67.60 |
| Hare | 977.0 | 0.00 | 0.00 | 0.00 | 21.97 | 78.04 |
| Lake Chamo | 602.0 | 3.69 | 1.86 | 2.31 | 6.08 | 86.09 |
| Total area | 8794.2 | 14.48 | 15.53 | 16.60 | 19.20 | 28.43 |

Figure 3-18 Mapping of sodic soil in selected irrigation schemes

In slightly to moderatly sodic soils, most tolerant crops shoould be grown and requires ammendments to improve soil infiltration rate and soil permeability for most sensitive crops.  Strongly to Very strongly sodic soils were not suitable for any crop particularly on heavy clay soils and hence requires reclamation program before cultivation of crops.

High concentrations of sodium  are  undesirable sodium adsorbs on to the soil cation exchange sites, causing soil aggregates to  break down  (deflocculating),  sealing  the  pores  of  the  soil  and  making  it impermeable to water flow.

### Classification of salt affected soils

In Ex-SNNP region, of the total irrigated area of about 8790.6 ha, 46% were found to be none saline-none-sodic soils, while the remaining were affected with saline (10%), saline-sodic (6%) and sodic (38%) soils. All irrigation schemes had varied level non-saline non-sodic soils, except Lobet, Bilate Eta, Abaya Bilatem, Lake Abaya (lante to algea) and Hare.  The irrigation schemes with non-saline non-sodic soils include, Arseamoy (100%), Arikol (100%), Delerele (100%), Enchete (35%), Duma (100%), Hamido (100%), Fulame (50%), Lake Abaya -Wajifo to kola barena (35% ) and Lake Chamo (25%).

Iirrigation schemes affected with saline soils include, Lobet (65%), Lake Abaya - lante to algea (25%), Hare (25%) and Lake Chamo (25%). Irrigation schemes that include Lake Abaya -Wajifo to Kola Barena (35%) and Lake Chamo (50%) affected with saline-sodic soisl. Irrigation schemes in Lobet (35%), Enchete (65%), Bilate Eta (100%), Abaya Bilate (100%), Fulame (50%), Lake abaya -lante to algea (75%), Lake abaya -Wajifo to kola barena (30%) and Hare (75%) were affected with sodic soils (Table 3.37).

Table 3-37. Classifications of salt affected soils

| S. No. | Schemes | Area (ha) | Salt affected soil classes |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | ECe<4 and ESP<15 | ECe>4 and ESP<15 | ECe>4 and ESP>15 | ECe<4 and ESP>15 |
|  |  |  | Non-saline and non-sodic | Saline | Saline-sodic | Sodic soil |
|  |  |  | Area coverage (%) |  |  |  |
| 1 | Arseamoy | 33.4 | 100 | 0.0 | 0.0 | 0.0 |
| 2 | Arikol | 22.3 | 100 | 0.0 | 0.0 | 0.0 |
| 3 | Delerele | 24.8 | 100 | 0.0 | 0.0 | 0.0 |
| 4 | Lobet | 118.4 | 0 | 65 | 0.0 | 35 |
| 5 | Enchete | 607.2 | 35 | 0.0 | 0.0 | 65 |
| 6 | Duma | 209.9 | 100 | 0.0 | 0.0 | 0.0 |
| 7 | Bilate Eta | 2040.2 | 0.0 | 0.0 | 0.0 | 100 |
| 8 | Abaya Bilate | 595.6 | 0.0 | 0.0 | 0.0 | 100 |
| 9 | Hamido | 152.5 | 100 | 0.0 | 0.0 | 0.0 |
| 10 | Fulame | 447.8 | 50 | 0.0 | 0.0 | 50 |
| 11 | Lake abaya (lante to algea) | 2074.5 | 0.0 | 25 | 0.0 | 75 |
| 12 | Lake abaya (Wajifo to kola barena) | 885 | 35 | 0.0 | 35 | 30 |
| 13 | Hare | 977 | 0.0 | 25 | 0.0 | 75 |
| 14 | Lake Chamo | 602 | 25 | 25 | 50 | 0 |
| Total Area |  | 8790.6 | 46 | 19 | 6 | 38 |

The non-saline non-sodic soile are suitable for cultivation of any crop and requires no salt management practices. However, regular monitoring is required to maintain the soil productivity. In saline soils, regular leaching management practices, based on the level of salt and crop tolerance need be undertaken either as a pre-planting leaching, leaching during crop cultivation or reclamation through continuous/intermittent leaching program during offseason, for complete reclamation of saline soils. In saline-sodic soils requires the use of ammendments followed by leaching management practices either during crop cultivation or during undertaking reclamation program.

The sodic soils require the use of chemical amendments to improve the infiltration rate and soil permeability. The use of organic matter together with salt tolerant crops can improve greatly the soil structure and water availablility for crops.

Salt characteristics of soil profile pits

Among SSI schemes, Arseamoy, Delerele, Enchete and Suma contained ECe and ESP values, respectively less than 4 dS/m and 15 throughout soil layers. According to established guideline for salt-affected soil classification (Richarde 1954) soils of these schemes fall under none-saline none sodic soil category indicating that these schemes were free of both salinity and sodicity problem.

However, soils of Bilate Eta, Abaya Bilate and Arika (except upper 0-30 cm soil layer) identified for containing appreciable quantity of both soluble salt (ECe) and exchangeable Na (ESP) throughout soil layers in excess of values set for none-saline none sodic soil category, These SSI schemes, therefore, were classified as saline sodic soil class. Hare, among SSI schemes under consideration, was the only scheme qualified as saline soil category. SSI schemes other than described above have shown a property of salt-affected soil class that varied between layers within the same profile pit.

Regarding depth-wise trend, ECe has shown a decreasing trend with increasing soil depth, in profile pits representing Delerele, Lobet, Enchete, and Abaya Bilate schemes while it tended to increase with increasing soil depth. There was no well-defined depth-wise trend in remaining ESP was seen to decrease with increasing soil depth in Delerele scheme, but no regular trend detected in remaining schemes. In general, result obtained from present assessment indicates occurrence salt-affected soils in most of SSI schemes assessed dominantly impacted by saline sodic soil property.

The quality of all irrigation water sources being used for realized to have extremely high level of ECw and SAR, far beyond recommendation. Obviously, these irrigation water sources might have caused buildup of soil salinization and sodiciation or aggravated the situation. With continued use of unsuitable water for irrigation more of cultivated farms will get degraded and abandoned as waste within not more than a decade. Salt load disposed from Abaya and Chamo Lake water for instance is estimated to be in excess of 15ton of salt every irrigation cycle.  Sustainability of most of SSI schemes is now under great threat associated to brackish irrigation water source. Changing land use or looking for other irrigation water source or use of more expensive desalinization technique are apparent and urgent management needs for sustained productive use of schemes at risk.

### 3.5.5. Irrigation water quality

There are two types of salt problems in irrigation water that are very different in their constituent, those associated with the total salinity, designated as salinity hazard and those associated with sodium, designated as sodium hazard. Irrigation water sources used in the different schemes were from rivers (20), Lake (6), Spring (4), ground water (3) and others (2), entire being 35.

Salinity Hazard

A total of 35 water samples were collected from Southern Region of Ethiopia for salinity analysis. The analysis from the 35 irrigation water points indicated that ECw ranged between 0.5 dS/m and 21 dS/m and 7 of these sources were catagorized as none, having none restitrictive use in irrigation. Whilst 15 (43%) of the water sources were sever, having restriction for use in irrigation, and 13 (37%) of the water sources were slight to moderate. (Table 3-38).

The irrigation water sources from, Hare River, Lobel-1, Lobel-2, Lobel-3, Delerele, Rate (1) and Rate (2) were categorized as none, indicating no restriction use in irrigation, since their respective ECw is less than 0.70 dS/m. The water sources from Abaya Bilate - Hot Spring (1), Abaya Bilate - River (2), Abaya Bilate - Mixed (1 and 2), Bilate Eta - Hot Spring(1), Bilate Eta - Mixed (1 and 2), Lake Chamo (Shelemela), Lake Chamo (sile), Lake chamo (Elgo), Geldeha (konso), Lake Abaya (Wajifo to kola barena), Lake Abaya (Lante to Algea) 1, Lake Abaya (Lante to Algea) 2, Silti Ground water, Balo koriso and Simbita were categorized as severe salinity hazard indicating restrictive use for irrigation, since their respective ECw is greater than 3 dS/m.

Table 3-38. Salinity hazard of irrigation water for selected schemes.

| S. No | Water sources | Types Water Source | Salinity Hazard |  |
| --- | --- | --- | --- | --- |
|  |  |  | ECw (dS/m) | Degree of restriction for use |
| 1 | Abaya Bilate - Hot Spring (1) | Spring | 8.91 | Severe |
|  | Abaya Bilate - River (2) | River | 4.69 | Severe |
|  | Abaya Bilate - Mixed (1 and 2) | Mixed | 8.33 | Severe |
| 2 | Bilate Eta - Hot Spring(1) | Spring | 4.45 | Severe |
|  | Bilate Eta - River (2) | River | 2.74 | Slight to moderate |
|  | Bilate Eta - Mixed (1 and 2) | Mixed | 3.94 | Severe |
| 3 | Hamido | River | 1.53 | Slight to moderate |
| 4 | Fulame | Spring | 1.295 | Slight to moderate |
| 5 | Delbo (upper) | River | 1.800 | Slight to moderate |
|  | Delbo (middle) | River | 2.027 | Slight to moderate |
| 6 | Lake Chamo (Shelemela) | Lake | 9.170 | Severe |
|  | Lake Chamo (sile) | Lake | 5.560 | Severe |
|  | Lake chamo (Elgo) | Lake | 6.370 | Severe |
| 7 | Hare river | River | 0.522 | None |
| 8 | Geldeha (konso) | River | 3.900 | Severe |
| 9 | Dortsa spring (Ganta kanchama) | Spring | 1.660 | Slight to moderate |
| 10 | Sego river (kola shele) | River | 2.380 | Slight to moderate |
| 11 | Lake Abaya (Wajifo to kola barena) | Lake | 4.140 | Severe |
|  | Lake Abaya (Lante to Algea) 1 | Lake | 20.950 | Severe |
|  | Lake Abaya (Lante to Algea) 2 | Lake | 4.720 | Severe |
| 12 | Lobet-1 | River | 0.511 | None |
|  | Lobet- 2 | River | 0.512 | None |
|  | Lobet- 3 | River | 0.516 | None |
| 13 | Delerele | River | 0.517 | None |
| 14 | Rate (1) | River | 0.510 | None |
|  | Rate (2) | River | 0.510 | None |
| 15 | Duma | River | 1.876 | Slight to moderate |
| 16 | Enchete (1) | River | 1.876 | Slight to moderate |
|  | Enchete (2) | River | 1.874 | Slight to moderate |
|  | Enchete (3) | River | 1.839 | Slight to moderate |
| 17 | Silti Ground water users | Ground | 7.470 | Severe |
| 18 | Lebu | River | 0.795 | Slight to moderate |
| 19 | Balo koriso | Ground | 5.770 | Severe |
| 20 | Bila wanja | River | 1.065 | Slight to moderate |
| 21 | Simbita | Ground | 7.500 | Severe |

The water from these sources needs special attention if intended for irrigation. It must either be blended with fresh water or be used in sodic soils. Otherwise, the water is restrictive for irrigation purposes (Table 3-38).

The water sources from Bilate Eta  - River (2), Hamidp, Fulame, Delbo (upper), Delbo (middle), Delbo (upper), Dortsa spring (Ganta kanchama), Sego river (kola shele), Duma, Enchete (1), Enchete (2), Enchete (3), lebu and Bila wanja were slight to moderate salinity hazard, requiring proper water management practices that include leaching water on top of irrigation water requirement in every irrigation events or occasionally at critical crop growth period depending on crop tolerance levels. And, growing salt tolerant crops Moreover, to maintain and/ or improve the soil and quality of irrigation water sources, sufficient leaching should be carried out before sowing until the salinity of the soil and water are conducive for crops..

Maintain periodic water quality assessment Other management options include proper irrigation water management, applying water as per the crop demand and at the right time with proper methods (Table 3-38)

Sodicity Hazard

The quality of irrigation water from 35 sources ranged between 0.5 dS/m and 21 dS/m and the SAR ranged between 0.3 and 28.3 %. According to the ECe and SAR catagories, 23% of the water sources were having slight to moderate sodicity hazard restriction for use in irrigation and 77% were having none sodicity hazard restriction for use in irrigation.

Table 3-39.  Sodicity hazard of irrigation water for selected schemes

| S. No | Schemes Name | Water Source | Sodicity Hazard |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | SAR (%) | ECw (dS/m) | Degree of restriction for use |
| 1 | Abaya Bilate - Hot Spring (1) | Spring | 22.9 | 8.91 | None |
|  | Abaya Bilate - River (2) | River | 16.9 | 4.69 | None |
|  | Abaya Bilate - Mixed (1 and 2) | Mixed | 16.7 | 8.33 | None |
| 2 | Bilate Eta - Hot Spring(1) | Spring | 7.6 | 4.45 | None |
|  | Bilate Eta - River (2) | River | 7.1 | 2.74 | None |
|  | Bilate Eta - Mixed (1 and 2) | Mixed | 8.8 | 3.94 | None |
| 3 | Hamido | River | 1.6 | 1.526 | None |
| 4 | Fulame | Spring | 1.5 | 1.295 | None |
| 5 | Delbo (upper) | River | 1.6 | 1.8 | None |
|  | Delbo (middle) | river | 0.8 | 2.027 | None |
| 6 | Lake Chamo (Shelemela) | Lake | 11.7 | 9.17 | None |
|  | Lake Chamo (sile) | Lake | 10.3 | 5.56 | None |
|  | Lake chamo (Elgo) | Lake | 13.1 | 6.37 | None |
| 7 | Hare river | River | 0.6 | 0.522 | Slight to moderate |
| 8 | Geldeha (konso) | River | 4.9 | 3.9 | None |
| 9 | Dortsa spring (Ganta kanchama) | Spring | 0.7 | 1.66 | None |
| 10 | Sego river (kola shele) | River | 1.1 | 2.38 | None |
| 11 | Lake Abaya (Wajifo to kola barena) | Lake | 8.9 | 4.14 | None |
|  | Lake Abaya (Lante to Algea) 1 | Lake | 25.1 | 20.95 | None |
|  | Lake Abaya (Lante to Algea) 2 | Lake | 7.1 | 4.72 | None |
| 12 | Lobet-1 | River | 0.4 | 0.511 | Slight to moderate |
|  | Lobet- 2 | River | 0.4 | 0.512 | Slight to moderate |
|  | Lobet- 3 | River | 0.4 | 0.516 | Slight to moderate |
| 13 | Delerele | River | 0.3 | 0.517 | Slight to moderate |
| 14 | Rate (1) | River | 0.3 | 0.51 | Slight to moderate |
|  | Rate (2) | River | 0.4 | 0.51 | Slight to moderate |
| 15 | Duma | River | 0.5 | 1.876 | None |
| 16 | Enchete (1) | River | 0.5 | 1.876 | None |
|  | Enchete (2) | River | 0.5 | 1.874 | None |
|  | Enchete (3) | River | 0.5 | 1.839 | None |
| 17 | Silti Ground water users | Ground | 9.6 | 7.47 | None |
| 18 | Lebu | River | 1.3 | 0.795 | None |
| 19 | Balo koriso | Ground | 28.3 | 5.77 | None |
| 20 | Bila wanja | River | 4.0 | 1.065 | Slight to Moderate |
| 21 | Simbita | Ground | 12.4 | 7.5 | None |

The water sources from Hare River, Lobet-1, Lobet-2, Lobet-3, Delerele, Rate (1), Rate (2) and Bila wanja, were slight to moderate sodicity hazard. Hence, require chemical amendments in every irrigation events. The use of improved irrigation water management practices together with organic matter incorporation in the soil, to improve the soil structure. The rest 77% of the water sources that were safe for irrigation require regular monitoring before the sodicity hazard became a proble,

### 3.5.6. Soil management options

Based on the results indicated in sections 3.5.2 to 3.5.3 analisis and classification possible mitigation measures are given in Table 3-40.

Table 3-40. Soil management options.

| Irrigation schemes | Soil salt classes | Soil management options |
| --- | --- | --- |
| Lobet, Enchete, Bilate Eta, Abaya Bilate, Fulame, Lake Abaya (Tante to algae, Wajifo to Kola Barena) & Hare. | Sodic (ECe < 4 dS/m, ESP > 15%) | - Apply gypsum to replace sodium with calcium, improve drainage and infiltration rate, and incorporate organic matter to enhance soil structure. |
| Lake Abaya (Wajifo to Kola Barena) and Lake Chamo. | Saline-sodic (ECe > 4 dS/m, ESP > 15%) | - Apply gypsum and organic amendments, enhance drainage and leaching, - Apply leaching water to remove both the exchangeable sodium and salt from the root zone - Select salt-tolerant crops. |
| Lake Abaya (Tante to algae), Hare and Lake Chamo. | Saline soils (ECe > 4 dS/m, ESP < 15%) | - Leaching of salts using good-quality irrigation water - Grow salt-tolerant crops; apply organic matter to improve soil structure. |
| Arseamoy, Arikol, Delerele Enchete, Duma, Hamido, Fulame, Lake Abaya (Wajifo to Kola Barena) and Lake Chamo. | Non-Saline and Non-Sodic (ECe < 4 dS/m, ESP < 15%) | - Maintain good soil structure; optimize irrigation scheduling. - Apply organic amendments for long-term soil health. |

### 3.5.7. Water management options

Based on the results indicated in sections 3.5.4 analysis and classification possible mitigation measures are given in Table 3.41.

Table 3-41. Water management options

| Irrigation schemes | Water quality challenges | Water management options |
| --- | --- | --- |
| Bilate Eta (River), Hamido, Fulame, Delbo (Upper & middle), Dortsa spring, Sego River, Duma, Enchete (1, 2, 3), Lebu & Bila wanja | Slight to moderate salinity hazard. | - Add leaching practice in every irrigation event as necessary. - Growing salt tolerant crops. - Maintain periodic water quality assessment |
| Simbita | Severe salinity hazard. | - Belend with good quality irrigation water - Use in sodic soil |
| Hare, Lobet (1,2,3), Delerele, Rate (1,2), Bila Wanja | Slight to moderate sodicity hazard. | - Mix chemical amendments with irrigation water - Blend with saline irrigation water - Incorporate organic matter in to soils |

### 3.5.8. Conclusion and Recommendation

Conclusion

Salinity control is a major objective of irrigation management, even though, the primary objective of irrigation is to maintain soil matric potential in a range suitable for optimal crop yield. In Southern regions, 8790.6 ha farm land and 35 irrigation water resource sites in 21 irrigation schemes were assessed for soil and water salinity problem. From the selected 35 schemes in the region, 3.3% was non-saline, 73.6% slight to strongly saline and 23.0% was very strongly to extremely saline soils. Whereas the sodicity assessment indicated that 6.0% was non-sodic, 25.2% slight to moderately sodic and 68.8% strongly to very strongly sodic soils. Generally, the salt affected soils in the region could be classified as 15.2% non-saline and non-sodic, 66.5% sodic, 6.9% saline sodic and 11.3% was saline soils.

The salinity hazard of irrigation water used from 13 water sources was slightly to moderate, whilst the irrigation water used from seven water sources were safe with no restriction in use and the irrigation water used in 15 irrigation scheme was severe.

The sodium hazard of the irrigation water used from eight water sources was slight to moderate and from 27 water sources was slight to moderate.

Recommendation

- The major objective of controlling salinity in the soil profile is to improve soil water availability to the crop thereby creating favourable condition for plant growth and improve productivity.

- Assess, conduct regular monitor of depth to ground water table and quality

- For efficient irrigation water management, proper land leveling/ smopthening should be carried out

- Identify field application efficiency for proper irrigation scheduling and irrigation application (Frequency, rate and duration of application based on crop, soil and water)

- Assess and apply leaching requirement based on crop, soil physical and chemical characteristics, and water qulity.

- Provide training to farmers, DA, extension agents and others associated agricultural activities

- In the farm lands with slight to strongly saline soils, yields of major crops could be affected and for cultivation of most economically important crops, leaching management practices is required to maintain a favourable salt balance in the root zone.

- In the farm lands with very strongly to extremely saline soils, only tolerant to very tolerants crops can be grown and leaching should be accompanied during the cropping season to creat a favourable salt balance in the plant roots. Otherwise, reclamation procedure through heavy leaching practice such as intermittent and continuous leaching program is required in the absence of plant to restore productivity.

- In the farm lands with slightly to moderatly sodic soils, most tolerant crops can be grown and chemical ammendments including gypsum need to be applied to improve the soil air and water permeability.

- Leaching water relatively high in calcium and magnesium is required in the farm lands with strongly to very strongly sodic soils. In heavy textured clay soils, permeability severely hindered and hence, requires reclamation by adding gypsum and organic matter along with heavy leaching program.

- Both the quantity and quality of water supply are important for irrigation. The use of poor quality irrigation water requires special management practices to maintain the soil and crop productivity. The irrigation water qualities from 15 sources were safe from salinity hazard with no degree of restriction for use in irrigation.

- The irrigation water quality from 2 sources was with severe sodicity hazard, requiring blending with good quality of water or the use of such soil in sodic soil.

- The irrigation water quality from 13 sources was with slight to moderate sodicity hazard, requiring blending with saline irrigation water or by adding chemical ammendments like gypsum either in the water or in the soil.

# CONCLUSIONS AND RECOMMENDATIONS

## 4.1. Conclusions

The irrigated lands of Afar region that were found in the lower Awash Valley have sodicity problem than salinity. The irrigated schemes in Asboda, Hinellie, Gumeli, Sunuta and Hida were not affected by salinity. However, Debel 7.78% and Meheri 13.50% of the studied areas were affected with slightly saline soils. The scheme at Mego was affecyed  with slightly saline (55.11%) and very strogly saline (35.08%) soils, while Wa’ama scheme was affected with modertly saline (79.98%) and strongly saline (15.82%) soils and Udoyita scheme was affected with slightly saline (55.69%), moderetly saline (19.28%), strongly saline (17.34%) and very strongly saline (3.94%) soils. Some irrigation lands near the river have shown salinesodic problem along the depth. In irrigated schemes particularly at Wa’ama where the source of water is from the groundwarer, the cause for soil salinity was low quality of irrigation water.

Regarding salinity problems of the Somalie Region, the irrigation schemes at Gumeli and Hida was not affected with soil sodicity and whilst the schemes at Debel, Udoyita, Mego and Sunuta were affected with extremely sodic soils. Hinellie scheme was affected with slightly (42.35%) and strongly (46.37%) sodic soils. Debel scheme was affected with 9.04% of the area with modertly sodic, 84.76% strongly sodic and 5.83% extremely sodic soils. Udoyita scheme was affected with 0.56% in slightly sodic soil and 35.59%, 43.16% and 19.97% were affected in modertly, strongly and extremely sodic soils, respectively. Mego scheme was affected with 15.05%, 30.11%, 56.73% and 0.05% of the area with slightly, moderately, strongly and extremely sodic soils. Whilst, Sunuta scheme was affected with modertly (22.53%), strongly (74.27%) and extremely (1.25%) sodic soils. Asboda was affected with slightly (0.70%), moderetly (20.08%) and strongly (79.06%) sodic soils. Meheri scheme was affected slightly (13.50), moderetly (55.41%) and strongly (29.87%) sodic soils. Wa’ama aws affected with slightly (10.65%), moderetly (63.06%) and and strongly (25.87%) sodic soils. Soil sodicity was considerably higher near the river than in other locations.

The salt-affected soils, from the total area at Udoyita scheme were 24.60% (7.86ha) saline-sodic and 39.6% (12.68ha) sodic soils and 35.80% (11.45 ha) non saline non sodic soil. In Mego scheme 44.3% (16.71 ha) fall in saline-sodic and 27.5% (10.36 ha) fallen under sodic class. In Wa’ama scheme, 4.41 % was saline, 17.35 % saline-sodic, 18.36 % sodic ns 44.3 % (16.71 ha) non saline non sodic (Table 9).  Out of total irrigated land of Hinellie and Meheri schemes, 5.5 % (1.64 ha) and 36.6 % (17.01 ha) land were affected by sodicityl and the remaining land was neither saline nor sodic (Table 3.6). The ESP values of Hinellie and Meheri irrigation schemes ranges from 4.75% – 19.32% and 4.68% -21.88%, respectively.

The SAR of all irrigation water value were between 1.38 -7.78. Thus, based on USSLS (1954) standard for water the irrigation water was under low sodicity hazard.

In the Amhara region, nearly 69% (1454.2 ha) of the irrigation schemes studied were non-saline non-sodic soil. The reaming was rated as 4% (84.3 ha) saline, 11.3% (238.2 ha) saline-sodic, and 15.7% (330.9 ha) sodic soils. All irrigation schemes studied in Central Gonder and Wag-himra zones, including North Shewa, except Charie and Sewere schemes, were classified as non-saline non-sodic soils. In the case of North and South Wollo, only Hormat Abuarie irrigation scheme was categorized as non-saline non-sodic soils. The Central Gonder irrigation schemes didn’t show sodicity problem while those in North Shewa and Wag-himra irrigation schemes some sodicity problem were observed and almost all North and South Wollo irrigation schemes were affected by sodicity.  The ECe values of the soil from soil profile pits did not show much variability down the depth. However, relatively high ECe was recorded from the topsoil (0-30 cm) layer and gradually decreased down the depth, except for Charie scheme in Norht Shewa of the Amhara Region. All irrigation sites studied in the Central Gondar, Wag-himra, and North Shewa irrigation schemes have low (<6%) to a slightly sodic (6-10%) soil. Based on the USSLS, (1954) classification criteria, except for the Hormat-Golina and Koba irrigation schemes, the North and South Wollo irrigation sites were classified as sodic soil. Similarly, Charie and Sewer schemes can be classified under sodic soil. The remaining irrigation schemes in North Shewa, Central Gonder, and Wag-himra zones were categorized as non-saline and non-sodic soil. The analytical results showed that the pH values of the irrigation water varied from 7.0 at Tekeze River to 8.21 at Bahir River The ECw of water samples was in aceptable range for irrigation except irrigation water that cames from groundwater and spring dilution, particularly in Rift Valley (Shewarobit to Kobo Valley). The SAR of most of the water sample from the irrigation schemes were categorized as low sodicity hazards.

Among the samples of surface soils in Oromia and Harari Regions and Dire Dawa City Adminstration, 30.97% were not affected by salinity, however, 31.86% was slightly saline, 25.66% moderately saline, 5.31% strongly saline and 3.54% very strongly saline and 2.65% extremely saline. In the subsurface soils, 33.3%, 30.3%, 30.3% and 29.3% were not affected by salinity through soil profile depth of 0-30cm, 0-60cm, 60-90cm, 90-120cm, respectively.  The samples were 9.09% slightly, 10.61% moderately, 10.61% highly saline and 10.94% were extremely saline on the depths of 0-30cm, 30-60cm, 60-90cm and 90-120cm, respectively. The sampled soils throughout the soil depth in the soil profile pit of Saboqqo ground water, Qadalle, Dhoma, Hargeti, Haro-adi, Tuji-Gebisa (ground water users), Gelcha, Warabele I, and Gidabo irrigation schemes showed strongly to extremely saline. Of all the sampled points only Saba irrigation schemes were not affected by salinity on the surface and subsurface soil. However, Saboqo, Bule-koloba, Argane and Bitata were not affected by salinity at surface soil only with respect to ECe. The ground water irrigated field soils shows that extremely sodicity level throughout the profile depth as those schemes were Gelcha, Dodo-wadara, Saboqo, warabile-II, Sheled, Watar dino (at middle scheme) Kambarojara and Haro adi. From the analysis result of irrigation water samples collected from lowland irrigated areas of Oromia region, showed medium to very highly saline classes. Out of 36 water samples collected at different schemes using surface and groundwater sources, 36.11% were high and 27.78% very high saline that requires serious attention.

The study shows that the salinity level of irrigation schemes in the Somali region is about 43% above the average to be used for irrigation. However, the quantitative and mapping of the spatial distribution of the salinity revealed that Ber`ano, Gode-2, and Dabafeyd irrigation schemes have more than 45% of the area coverage strongly saline, while the remaining irrigation schemes have slight to moderate saline. Furthermore, Gode-2 irrigation scheme have experienced 35% the irrigated area were very strongly saline that requires serious attention and care. Similarly, the current study shows that more than 50% of Armo, Arabi-1, Arabi-2 and Agarweyne irrigated area have non-sodic soils, while the rest of the schemes were classified under slightly to moderate sodic classes.  The result of soil pH, ECe and ESP was not uniform along the soil profile/depth wise trend in all irrigation schemes. Most schemes combining effect of salinity and sodicity levels showed that salinity is the major soil problem and sodicity do not have much in coverage. However, sodicity elevels may increase through time, probably in the near future. In addition, the SAR of the irrigation water is classified as low while the EC of the water is classified as low to medium. The values of ECw and SAR of the water were below the FAO guidelines for water quality restriction limit. Therefore, the result of the current study revealed that water of both surface and ground water was not seen to be the cause of the soil salinity and sodicity in the selected schemes. Even though the main cause of soil salinity is supposed to be secondary salinization which was mainly from poor irrigation water management, excessive irrigationwater applications and poor drainage in the irrigation schemes and climatic condition, including high evaporation and low rainfall aggravate the condition.

In the Southren Region, the surface soil analysis result indicated that from the total studied schemes area 7.32% were non-saline, 29.1% slightly saline, 18.35% moderately saline, 21.27% strongly saline, 17.2% very strongly saline, and 6.76% extremely saline soils. The low-land irrigation schemes, namely Delerele, Lobet, Duma, Lake abaya (lante to algea), Lake abaya (Wajifo to kola barena), Hare, and Lake Chamo have mainly salinity concerns, while mid land irrigation schemes Hamido, Fulame, and Bilate irrigation schemes were non-saline. The soil sodicity area coverage indicated 14.48% non-sodic (), 15.53% slightly sodic (), 16.6% moderately sodic (), 19.2% strongly sodic (), and 28.43% very strongly sodic ().  The results of the samples taken from river water have no sodicity problem, but the samples taken from ground water, particularly in Abay Blate and Balo koriso have high sodicity levels. From the total water sampled in selected irrigation schemes of Southern Region, 48.6% was very high saline water, 31.4% was highly saline water and 20% was medium saline water.

## 4.2. Recommendations

To minimize salinity and sodicity effect, awareness creation among key stake holders, farmers in particular and dissemination of knowledge on improved drainage and crop management practices have paramount importances. Training farmers on irrigation water management, irrigation methods, type of irrigation, irrigation policy, improved tillage, drainage etc. is important.  Adverse effect of salinity and sodicity can be minimized by growing salt tolerant plant species, using salt affected soils for cultivation of forage crops, leveling, salt scraping, tillage methods, minimum tillage, sub-soiling, sanding and biological methods of growing of green manure crops, such as legumes.  Research and development towards improved irrigation water and crop management practices should be readily available.  Surface water drainage, use of mulching and organic manure amendment are advisable.

Adverse effects of soil salinity and sodicity in the salt affected areas can be avoided through proper management measures.  This includes practicing proper drainage system (surface and sub-surface drainage), minimum tillage to avoid soil compaction, use good quality and/or blended water for irrigation, adopt efficient farm water management to avoid over and under irrigation and improve water use efficiency, introducing salt tolerant crops and avoid mixing of river water with drained water.  Identification of salt tolerant crops including fodder plants that tolerate soil salinity and blending of poor irrigation water quality can be considered for use in salt affected areas. This is significant because shortage of feed is one of the most important problems for livestock production in Ethiopia. Engineering means are expensive and demanding technical financial capacity and hence, biological solutions are the best alternative. Already large fields are abandoned due to salinity and sodality and where staple crops are no longer cultivated. Therefore, production of fodder crops and halophytes is a viable solution.

# REFERENCES

Aragüés, R., Medina, E.T., Zribi, W., Clavería, I., Álvaro-Fuentes, J., and Faci, J. (2015). Soil salinization as a threat to the sustainability of deficit irrigation under present and expected climate change scenarios. Irrigation Science, 33, 67-79. doi:10.1007/s00271-014-0449-x

Asad Surwar Qurashi, Tesfaye Erdobo Mohammed, and Melese Minelashoa. (2019). Extent, characterization, and management strategies of salt-affected soils in Ethiopia. Addis Ababa.

Ayers, R.S., and Westcot, D.W. (1985). Water Quality for Agriculture. FAO Irrigation and Drainage Paper No. 29. FAO, Rome, Italy. pp. 1-117.

Awulachew, S. B., Yilma, A. D., Loulseged, M., and Loiskandl, W. (2010). Irrigation potential in Ethiopia: Constraints and opportunities for enhancing the system. IWMI.

Dahir, Y.A., Derege, T.M., and Tadeos, S.W. (2022). Variability of soil chemical properties in lower Wabi Shebele Sub-Basin in Somali Region South-eastern Ethiopia, as influenced by land use and land cover. African Journal of Agricultural Research, 18(2), 153-161.

Deshpande, S.M., and Aher, K. R. (2012). Evaluation of groundwater quality and its suitability for drinking and agriculture use in parts of Vaijapur, District Aurangabad, MS, India. Research Journal of Chemical Sciences, 2(1), 25-31.

Eaton, F. M. (1950). Significance of carbonates in irrigation waters. Soil Science, 69, 123–133.

El-Maaz, E., Ahmed, H., and Khalil, H. (2015). Physical and chemical properties of some soils adjacent to lakes at northwest and south of Egypt. Journal of Soil Sciences and Agricultural Engineering, 6(7), 793-807.

EPI and AAIT (Ethiopian Policy Institute and Addis Abeba Institute of Technology). (2019). ሀገራዊ የመስኖ ሌማት ስርዓት ፌኖተካርታ ሇውይይት የቀረበ ረቂቅ ሰነዴ.

FAO (Food and Agriculture Organization) 1984. Land use planning and rural development Ethiopia geomorphology and soils field document 3, Addis Ababa.

FAO (Food and Agriculture Organization), 1988. Salt-affected soils and their management. FAO Soils Bulletin 39, Rome, Italy.

FAO (Food and Agriculture Organization),  1985. Water quality for agriculture. Irrigation and Drainage paper No. 29. FAO, Rome, Italy.

FAO (Food and Agriculture Organization),2008. Mapping of salt-affected soil manual, technical manual. FAO, Rome, Italy. pp.1-113.

FAO (Food and Agriculture Organization), 2015. Status of the World's Soil Resources: Main Report. FAO, Rome, Italy.

FAO (Food and Agriculture Organization), 2016. Ethiopia: Water resources and irrigation potential. FAO.

FAO (Food and Agriculture Organization), IIASA, ISRIC, ISS-CSA, and JRC. (2008). Harmonized World Soil Database (version 1.0). FAO, Rome, Italy and IIASA, Laxenburg, Austria.

Ghafoor et al., (2008)

Girma, Y. and Desta, G., 2008. Investigation on Salinity Problem of Efratana Gidim and Kewet Weredas. on Completed Natural Resources Management Research Activities, p.81.

Giday Adhanom, O. (2019). Salinity and sodicity hazard characterization in major irrigated areas and irrigation water sources, Northern Ethiopia. Cogent Food and Agriculture, 5(1), 1673110.

Kijne, J. W., Prathapar, S. A. and Sahrawat, K. L., 1998. How to Manage Salinity in Irrigated Lands: A Selective Review with Particular Reference to Irrigation in Developing Countries. In: Jacob W (ed.) Soil Science (Issue December)

Krasilnikov P., Gutierrez-Castorena M. C., Ahrens R, J, Cruz-Gaistardo, C. O., Sedov S. and Solleiro-Robelledo E., 2013. The soils of Mexico. Springer, London.

MoA (Ministry of Agriculture), (2019). Small-scale irrigation Development Strategy, Addis Ababa.

MoA (Ministry of Agriculture), (2020). National Smallholder Irrigation and Drainage Strategy for Ethiopia. Ministry of Agriculture, Ethiopia.

Murphy, H. F., 1968. A report on the fertility status and other data on some soils of Ethiopia.

Murtaza, G., Ghafoor, A. and Qadir, M., 2006. Irrigation and soil management strategies for using saline-sodic water in a cotton-wheat rotation. Agricultural Water Management, 81(1-2): 98-114.

Okalebo, J. R., Gathua, K. W. and Woomer, P. L. 2002. Laboratory Methods for Soil and Plant Analysis: A Working Manual. TSBF, Nairobi.

Omuto C. T., Vargas, R. R., El-Mobarak, A. M., Mohamed, N., Viatkin, K. and Yigini, Y., 2020. Mapping of salt-affected soils: Technical manual. FAO, Rome, Italy.

OWWDSE (Oromia Water Works Design and Supervision Enterprise) 2012. Upper Fafan Sub-basin Agro-climatic. Unpublished Report.

Qureshi et al. (2020)

Rengasamy, P., 2006. World salinization with emphasis on Australia. Journal of experimental botany, 57(5), pp. 1017-1023.

Richards, L. A., 1954. Diagnosis and Improvement of Saline Alkali Soils, Agriculture, 160, Handbook 60. US Department of Agriculture, Washington DC.

Schmitter, P., Desalegn, G., Langan, S., Barron, J., and Gebrehaweria, G. (2018). Improving On-Farm Water Management and Agricultural Productivity in Ethiopia: Interventions and Lessons. Water Resources and Rural Development, 16, 107–122. https://doi.org/10.1016/j.wrr.2018.01.001.

Shahid et al., 2010

Sileshi, A. (2016). Status of salinity-affected soils, irrigation water quality, and land suitability of Dubti/Tendaho area, North Eastern Ethiopia (Ph.D. dissertation). Haramaya University, Haramaya, Ethiopia.

Sileshi, A., Kibebew, K., and Amanuel, Z., 2015. Temporal and spatial variations in of salt-affected soils using GIS and remote sensing at Dubti/ Tendaho state farm. [Ph.D. Dissertation thesis]. Haramaya University, Ethiopia.

Sisay et al., (2021)

Sulieman, M. , Ibrahim, I. and Elfaki, J., 2016. Genesis and Classification of Some Soils of the River Nile Terraces: A Case Study of Khartoum North, Sudan. Journal of Geoscience and Environment Protection, 4, 1-16. doi: 10.4236/gep.2016.43001

Tilahun Hordofa, 1991. The Role of Restructuring in the Drainage of Heavy Clay Soils. MSc thesis. Institute of irrigation studies, Southampton University. Southampton, UK.

Troch, F. R., hobbs, J. A. and Donahue, R. L., 1999. Soil and water conservation: Productivity and environmental protection. 3rd Edition. Prentice Hall,Upper Saddle River. New Jersey. 610 p.

USSLS (US Salinity Laboratory Staff), 1954. Diagnosis and improvement of saline and alkali soils. US Department of Agriculture Handbook 60, Washington, DC.

Waskom, R.M., Bauder, K.M.T., Davis, J.G. and. Andales, A.A 2010. Diagnosing Saline and Sodic Soil ProblemColorado State University, U.S. Department of Agriculture and Colorado counties cooperating. CSU Extension

World Bank. (2021). Ethiopia irrigation sector review: Unlocking potential.

Zaman, M., Shahid, S.A., and Heng, L. (2018). Guideline for salinity assessment, mitigation, and adaptation using nuclear and related techniques. Springer Nature.

Zeng, C., Liu, Z., Zhao, M., and Yang, R. (2016). Hydrologically-driven variations in the karst-related carbon sink fluxes: Insights from high-resolution monitoring of three karst catchments in Southwest China. Journal of Hydrology, 533, 74-90. doi: 10.1016/j.jhydrol.2015.11.049.

# Annexes

Box 1. Data collection sheet/ format for reconnaissance survey

- General information about a scheme

- Name of the scheme _______________ Year of establishment of the scheme _______________

- Kebele _____________ Woreda__________________Zone__________ Region ____________

- Area under irrigation (command area in ha) __________________ _

- No of beneficiary households: Total:_____________Male:____________Female____________

- Average family size of the beneficiary households ____________________________________

- Average holding size: Irrigated  ___________________________________________________

- Lat/Long ___________________        Altitude (range)  ________________________________

- Length of growing period ________________________________________________________

- Annual rainfall (mm)  ___________________________________________________________

- Soil types (local description and if any, scientific name) ________________________________

- Source of water for the irrigation __________________________________________________

- Major irrigated crops ___________________________________________________________

- Name of the researcher recording this data __________________________________________

- Major drainage problems in the scheme that need interventions

- Why the salinity happened in the particular scheme?

- Which types of crops are grown to cope with the problem of salinity?

- If any practice the farmers use in reclaiming for lands affected by salinity

- Information on water abstraction, conveyance and distribution

- Crops and cropping systems

- Whether the farmers in a particular scheme are using chemical fertilizers for the irrigated crops

- Information on water uses, methods of irrigation, depth and frequency of application

- Water management practices

- Bases for water allocation and management

- Water governance and bylaws

Table 1. Preliminary assessment of salt- affected soil and major crops cultivated for ten irrigation schemes in afar region.

| S.N | Scheme name | Irrigation system | Visual Diagnosis of Salt-Affected Soils | Major Crops grown in the schemes | Textural class | Soil color | Farmers Response to salinity | Farmers Response to water logging |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Asboda | Furrow | White crust | Maize, Sesame, Onion, Tomato, Paper, Watermelon | Sandy loam | Whitey | Severe | Severe |
| 2 | Debel | Furrow | White crust | Maize, Cotton, Mungbean, Onion, Tomato, Paper, Watermelon | Sandy loam | Whitey | Severe | Severe |
| 3 | Meheri | Furrow and Flooding | White crust, Water-stressed plants, Leaf tip burn, black dark powdery | Maize, tomato, cotton, sesame | loam | white blacky | Severe | Fair |
| 4 | Hinellie | Furrow and Flooding | White crust, Water-stressed plants, Leaf tip burn, | Maize, tomato, cotton, sesame | Sandy loam | White | Not recognized | Fair |
| 5 | Udoyita | Furrow and Flooding | White crust, Poor drainage, Black powdery residue on soil surface, Black oily soil | Maize, tomato, cotton, sesame | Sandy loam | white blacky | Severe | Severe |
| 6 | Mego | Flooding | White crust, Poor drainage, Black powdery residue on soil surface, Black oily soil | Maize, tomato, cotton, sesame | sandy | white blacky | Severe | Severe |
| 7 | Gumeli | Furrow and Flooding | White crust | Maize, tomato, Onion, | Silty loam | white blacky | Not recognized | Severe |
| 8 | Wa’ama | Furrow and Flooding | White crust, leaf tip burn, Poor drainage, Black powdery residue on soil surface | Maize, tomato, Onion, | loam | white blacky | Severe | Severe |
| 9 | Sununta | Furrow | White crust, Poor drainage, Black powdery residue on soil surface, | Maize, tomato, Onion, | Sandy loam | White | Severe | Not recognized |
| 10 | Hida | Furrow and Flooding | White crust | Maize, tomato, Onion, pepper, Sorghum | Clay loam | white blacky | Not recognized | Fair |

Table 0-2 Soil pH, salinity and sodicity levels in the selected irrigation schemes of Amhara Region.

| S.no | Scheme | Profile pit | Depth | pH | EC | ESP | General classification |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Wonchit (Meyih) | Pit1 | 0-30 | 7.83 | 0.24 | 3.88 | Non-Saline, Non-Sodic |
|  |  |  | 30-60 | 7.78 | 0.29 | 3.39 | Non-Saline, Non-Sodic |
|  |  |  | 60-90 | 7.27 | 0.3 | 1.91 | Non-Saline, Non-Sodic |
|  |  |  | 90-120 | 7.66 | 0.11 | 2.51 | Non-Saline, Non-Sodic |
|  |  | Pit2 | 0-30 | 7.65 | 0.81 | 3.91 | Non-Saline, Non-Sodic |
|  |  |  | 30-60 | 8.05 | 0.21 | 3.02 | Non-Saline, Non-Sodic |
|  |  |  | 60-90 | 8.17 | 0.19 | 2.75 | Non-Saline, Non-Sodic |
|  |  |  | 90-120 | - | - | - | Non-Saline, Non-Sodic |
| 2 | Jema (Geren) | Pit1 | 0-30 | 7.95 | 0.42 | 6.01 | Non-Saline, Non-Sodic |
|  |  |  | 30-60 | 7.97 | 0.15 | 4.83 | Non-Saline, Non-Sodic |
|  |  |  | 60-90 | 8.07 | 0.23 | 3.08 | Non-Saline, Non-Sodic |
|  |  |  | 90-120 | 8.2 | 0.31 | 5.12 | Non-Saline, Non-Sodic |
|  |  | Pit2 | 0-30 | 8.27 | 0.35 | 5.98 | Non-Saline, Non-Sodic |
|  |  |  | 30-60 | 8.4 | 0.15 | 4.06 | Non-Saline, Non-Sodic |
|  |  |  | 60-90 | 8.36 | 0.11 | 3.76 | Non-Saline, Non-Sodic |
|  |  |  | 90-120 | - | - | - | Non-Saline, Non-Sodic |
|  |  | Pit3 | 0-30 | 7.87 | 0.66 | 4.23 | Non-Saline, Non-Sodic |
|  |  |  | 30-60 | 8.08 | 0.32 | 2.87 | Non-Saline, Non-Sodic |
|  |  |  | 60-90 | 7.92 | 0.66 | 4.84 | Non-Saline, Non-Sodic |
|  |  |  | 90-120 | - | - | - | Non-Saline, Non-Sodic |
| 3 | Belbelit | Pit1 | 0-30 | 7.97 | 0.32 | 3.25 | Non-Saline, Non-Sodic |
|  |  |  | 30-60 | 8.25 | 0.26 | 2.53 | Non-Saline, Non-Sodic |
|  |  |  | 60-90 | 7.56 | 0.16 | 4.79 | Non-Saline, Non-Sodic |
|  |  |  | 90-120 | 8 | 0.45 | 4.69 | Non-Saline, Non-Sodic |
|  |  | Pit2 | 0-30 | 8 | 0.2 | 2.43 | Non-Saline, Non-Sodic |
|  |  |  | 30-60 | 8.07 | 0.11 | 2.64 | Non-Saline, Non-Sodic |
|  |  |  | 60-90 | 8.19 | 0.61 | 2.6 | Non-Saline, Non-Sodic |
|  |  |  | 90-120 | - | - | - | Non-Saline, Non-Sodic |
| 4 | Melka-Jebdu | Pit1 | 0-30 | 7.86 | 0.32 | 1.56 | Non-Saline, Non-Sodic |
|  |  |  | 30-60 | 7.91 | 0.26 | 1.28 | Non-Saline, Non-Sodic |
|  |  |  | 60-90 | 8.36 | 0.21 | 1.99 | Non-Saline, Non-Sodic |
|  |  |  | 90-120 | - | - | - | Non-Saline, Non-Sodic |
| 5 | Charie | Pit1 | 0-30 | 8.44 | 0.65 | 8.7 | Non-Saline, Non-Sodic |
|  |  |  | 30-60 | 8.35 | 0.41 | 5.66 | Non-Saline, Non-Sodic |
|  |  |  | 60-90 | 8.17 | 0.49 | 4 | Non-Saline, Non-Sodic |
|  |  |  | 90-120 | 8.13 | 0.53 | 4.27 | Non-Saline, Non-Sodic |
|  |  | Pit2 | 0-30 | 8.76 | 34.8 | 217.65 | Saline-Sodic |
|  |  |  | 30-60 | 8.02 | 7.74 | 105.21 | Saline-Sodic |
|  |  |  | 60-90 | 8.04 | 5.85 | 55 | Saline-Sodic |
|  |  |  | 90-120 | 8.03 | 5.33 | 45.48 | Saline-Sodic |
|  |  | Pit3 | 0-30 | 8.63 | 1.04 | 17.22 | Sodic |
|  |  |  | 30-60 | 8.75 | 0.92 | 23.09 | Sodic |
|  |  |  | 60-90 | 8.25 | 0.24 | 5.28 | Non-Saline, Non-Sodic |
|  |  |  | 90-120 | 8.24 | 0.24 | 3.37 | Non-Saline, Non-Sodic |
|  |  | Pit4 | 0-30 | 8.03 | 3.28 | 45.13 | Sodic |
|  |  |  | 30-60 | 7.89 | 5.22 | 42.47 | Saline-Sodic |
|  |  |  | 60-90 | 8.47 | 3.41 | 66.55 | Sodic |
|  |  |  | 90-120 | 8.7 | 1.61 | 67.98 | Sodic |
|  |  | Pit5 | 0-30 | 8.18 | 0.29 | 2.73 | Non-Saline, Non-Sodic |
|  |  |  | 30-60 | 8.12 | 0.31 | 2.96 | Non-Saline, Non-Sodic |
|  |  |  | 60-90 | 8.28 | 0.23 | 3.47 | Non-Saline, Non-Sodic |
|  |  |  | 90-120 | 8.07 | 0.26 | 2.87 | Non-Saline, Non-Sodic |
|  |  | Pit6 | 0-30 | 8.07 | 0.31 | 4.87 | Non-Saline, Non-Sodic |
|  |  |  | 30-60 | 7.8 | 0.13 | 5.57 | Non-Saline, Non-Sodic |
|  |  |  | 60-90 | 7.92 | 0.15 | 2.74 | Non-Saline, Non-Sodic |
|  |  |  | 90-120 | 7.98 | 0.2 | 2.18 | Non-Saline, Non-Sodic |
|  |  | Pit7 | 0-30 | 8.01 | 0.51 | 5 | Non-Saline, Non-Sodic |
|  |  |  | 30-60 | 8.02 | 0.28 | 4.74 | Non-Saline, Non-Sodic |
|  |  |  | 60-90 | 8.09 | 0.29 | 3.61 | Non-Saline, Non-Sodic |
|  |  |  | 90-120 | 8.05 | 0.27 | 3.74 | Non-Saline, Non-Sodic |
| 6 | Sewer | Pit1 | 0-30 | 8.09 | 0.36 | 20.24 | Sodic |
|  |  |  | 30-60 | 8.1 | 0.19 | 19.13 | Sodic |
|  |  |  | 60-90 | 8.03 | 0.28 | 16.56 | Sodic |
|  |  |  | 90-120 | 8.17 | 0.39 | 18.06 | Sodic |
|  |  | Pit2 | 0-30 | 8.31 | 0.46 | 27.72 | Sodic |
|  |  |  | 30-60 | 8.52 | 0.43 | 31.01 | Sodic |
|  |  |  | 60-90 | 8.68 | 0.43 | 51.44 | Sodic |
|  |  |  | 90-120 | 8.6 | 0.54 | 11.25 | Non-Saline, Non-Sodic |
|  |  | Pit3 | 0-30 | 7.96 | 0.22 | 5.71 | Non-Saline, Non-Sodic |
|  |  |  | 30-60 | 8.01 | 0.18 | 5.52 | Non-Saline, Non-Sodic |
|  |  |  | 60-90 | 7.96 | 0.17 | 5.14 | Non-Saline, Non-Sodic |
|  |  |  | 90-120 | 8.02 | 0.18 | 4.76 | Non-Saline, Non-Sodic |
|  |  | Pit4 | 0-30 | 7.62 | 0.15 | 4.97 | Non-Saline, Non-Sodic |
|  |  |  | 30-60 | 7.63 | 0.16 | 4.6 | Non-Saline, Non-Sodic |
|  |  |  | 60-90 | 7.81 | 0.2 | 3.91 | Non-Saline, Non-Sodic |
|  |  |  | 90-120 | 7.82 | 0.2 | 4.44 | Non-Saline, Non-Sodic |
|  |  | Pit5 | 0-30 | 6.8 | 0.74 | 6.58 | Non-Saline, Non-Sodic |
|  |  |  | 30-60 | 7.05 | 0.92 | 6.3 | Non-Saline, Non-Sodic |
|  |  |  | 60-90 | 7.45 | 0.22 | 5.17 | Non-Saline, Non-Sodic |
|  |  |  | 90-120 | 7.98 | 0.25 | 3.26 | Non-Saline, Non-Sodic |
|  |  | Pit6 | 0-30 | 7.65 | 0.16 | 9.09 | Non-Saline, Non-Sodic |
|  |  |  | 30-60 | 8.04 | 0.24 | 6.64 | Non-Saline, Non-Sodic |
|  |  |  | 60-90 | 7.82 | 0.08 | 7.56 | Non-Saline, Non-Sodic |
|  |  |  | 90-120 | 7.88 | 0.27 | 6.33 | Non-Saline, Non-Sodic |
|  |  | Pit7 | 0-30 | 7.63 | 0.41 | 48.01 | Sodic |
|  |  |  | 30-60 | 7.63 | 3.29 | 45.48 | Sodic |
|  |  |  | 60-90 | 7.52 | 3.59 | 45.35 | Sodic |
|  |  |  | 90-120 | 7.81 | 2.66 | 41.08 | Sodic |
| 7 | Mereye | Pit1 | 0-30 | 8.05 | 0.28 | 5.02 | Non-Saline, Non-Sodic |
|  |  |  | 30-60 | 7.98 | 0.2 | 5.22 | Non-Saline, Non-Sodic |
|  |  |  | 60-90 | 7.89 | 0.22 | 11.9 | Non-Saline, Non-Sodic |
|  |  |  | 90-120 | 8.01 | 0.16 | 4.33 | Non-Saline, Non-Sodic |
|  |  | Pit2 | 0-30 | 8 | 0.9 | 7.65 | Non-Saline, Non-Sodic |
|  |  |  | 30-60 | 8.06 | 0.36 | 5.87 | Non-Saline, Non-Sodic |
|  |  |  | 60-90 | 7.96 | 0.12 | 5.19 | Non-Saline, Non-Sodic |
|  |  |  | 90-120 | 7.86 | 0.18 | 6.04 | Non-Saline, Non-Sodic |
| 8 | Wanza | Pit1 | 0-30 | 7.93 | 0.33 | 7.25 | Non-Saline, Non-Sodic |
|  |  |  | 30-60 | 7.79 | 0.15 | 6.81 | Non-Saline, Non-Sodic |
|  |  |  | 60-90 | 7.86 | 0.12 | 6.22 | Non-Saline, Non-Sodic |
|  |  |  | 90-120 | 8 | 0.25 | 5.5 | Non-Saline, Non-Sodic |
|  |  | Pit2 | 0-30 | 7.92 | 0.43 | 5.56 | Non-Saline, Non-Sodic |
|  |  |  | 30-60 | 7.9 | 0.33 | 7.26 | Non-Saline, Non-Sodic |
|  |  |  | 60-90 | 7.93 | 0.19 | 6.75 | Non-Saline, Non-Sodic |
|  |  |  | 90-120 | 7.96 | 0.1 | 6.27 | Non-Saline, Non-Sodic |
| 9 | Maremiya | Pit1 | 0-30 | 7.92 | 0.35 | 10.15 | Non-Saline, Non-Sodic |
|  |  |  | 30-60 | 9.15 | 0.96 | 55.41 | Sodic |
|  |  |  | 60-90 | 8.67 | 0.75 | 50.26 | Sodic |
|  |  |  | 90-120 | 8.61 | 0.47 | 19.61 | Sodic |
|  |  | Pit2 | 0-30 | 8.06 | 0.42 | 9.66 | Non-Saline, Non-Sodic |
|  |  |  | 30-60 | 8.07 | 0.37 | 7.24 | Non-Saline, Non-Sodic |
|  |  |  | 60-90 | 8.15 | 0.34 | 5.82 | Non-Saline, Non-Sodic |
|  |  |  | 90-120 | 8.17 | 0.32 | 5.62 | Non-Saline, Non-Sodic |
| 10 | Bahir | Pit 1 | 0-30 | 7.90 | 0.20 | 1.70 | Non-saline-Non-sodic |
|  |  |  | 30-60 | 8.28 | 0.16 | 4.58 | Non-saline-Non-sodic |
|  |  |  | 60-90 | 8.34 | 0.17 | 8.48 | Non-saline-Non-sodic |
|  |  |  | 90-120 | 8.43 | 0.14 | 1.40 | Non-saline-Non-sodic |
|  |  | Pit 2 | 0-30 | 7.83 | 0.23 | 3.50 | Non-saline-Non-sodic |
|  |  |  | 30-60 | 7.79 | 0.18 | 2.53 | Non-saline-Non-sodic |
|  |  |  | 60-90 | 7.95 | 0.18 | 12.65 | Non-saline-Non-sodic |
|  |  |  | 90-120 | 7.90 | 0.65 | 3.98 | Non-saline-Non-sodic |
|  |  | Pit 3 | 0-30 | 8.15 | 0.15 | 2.63 | Non-saline-Non-sodic |
|  |  |  | 30-60 | 8.12 | 0.12 | 11.31 | Non-saline-Non-sodic |
|  |  |  | 60-90 | 7.94 | 0.43 | 7.46 | Non-saline-Non-sodic |
|  |  |  | 90-120 | 7.89 | 0.19 | 4.46 | Non-saline-Non-sodic |
| 11 | Tsitsika | Pit 1 | 0-30 | 7.97 | 0.12 | 5.47 | Non-saline-Non-sodic |
|  |  |  | 30-60 | 8.09 | 0.19 | 9.50 | Non-saline-Non-sodic |
|  |  |  | 60-90 | 8.16 | 0.20 | 2.67 | Non-saline-Non-sodic |
|  |  |  | 90-120 | 8.13 | 0.12 | 3.86 | Non-saline-Non-sodic |
|  |  | Pit 2 | 0-30 | 7.97 | 0.36 | 5.90 | Non-saline-Non-sodic |
|  |  |  | 30-60 | 8.25 | 0.26 | 3.93 | Non-saline-Non-sodic |
|  |  |  | 60-90 | 7.70 | 0.16 | 5.20 | Non-saline-Non-sodic |
|  |  |  | 90-120 | 7.96 | 0.14 | 3.75 | Non-saline-Non-sodic |
|  |  | pit 3 | 0-30 | 8.19 | 0.18 | 10.17 | Non-saline-Non-sodic |
|  |  |  | 30-60 | 8.17 | 0.19 | 3.12 | Non-saline-Non-sodic |
|  |  |  | 60-90 | 8.18 | 0.13 | 2.96 | Non-saline-Non-sodic |
|  |  |  | 90-120 | 8.26 | 0.17 | 3.88 | Non-saline-Non-sodic |
| 12 | Tekeze | Pit 1 | 0-30 | 7.77 | 0.65 | 12.19 | Non-saline-Non-sodic |
|  |  |  | 30-60 | 8.06 | 0.14 | 4.20 | Non-saline-Non-sodic |
|  |  |  | 60-90 | 7.89 | 0.17 | 4.81 | Non-saline-Non-sodic |
|  |  |  | 90-120 | 8.12 | 0.18 | 5.33 | Non-saline-Non-sodic |
|  |  | Pit 2 | 0-30 | 8.09 | 0.21 | 8.97 | Non-saline-Non-sodic |
|  |  |  | 30-60 | 7.98 | 0.13 | 4.76 | Non-saline-Non-sodic |
|  |  |  | 60-90 | 7.81 | 0.22 | 5.28 | Non-saline-Non-sodic |
|  |  |  | 90-120 | 8.02 | 0.14 | 8.61 | Non-saline-Non-sodic |
|  |  | Pit 3 | 0-30 | 7.86 | 0.36 | 4.31 | Non-saline-Non-sodic |
|  |  |  | 30-60 | 7.86 | 0.2 | 5.25 | Non-saline-Non-sodic |
|  |  |  | 60-90 | 7.83 | 0.21 | 4.60 | Non-saline-Non-sodic |
|  |  |  | 90-120 | 7.62 | 0.44 | 4.69 | Non-saline-Non-sodic |
| 13 | Kemele | Pit1 | 0-30 | 9.4 | 1.7 | 61.7 | Sodic |
|  |  |  | 30-60 | 9.4 | 0.9 | 59 | Sodic |
|  |  |  | 60-90 | 9.1 | 0.5 | 35.9 | Sodic |
|  |  |  | 90-120 | 9 | 0.4 | 25.5 | Sodic |
|  |  | Pit2 | 0-30 | 8 | 3.5 | 49.3 | Sodic |
|  |  |  | 30-60 | 8.4 | 1.8 | 29.1 | Sodic |
|  |  |  | 60-90 | 8.5 | 0.3 | 8.8 | Non |
|  |  |  | 90-120 | 8.4 | 0.3 | 7.3 | Non |
| 14 | Tewelde | Pit1 | 0-30 | 8.6 | 0.7 | 27.5 | Sodic |
|  |  |  | 30-60 | 8.6 | 0.4 | 19.8 | Sodic |
|  |  |  | 60-90 | 8.6 | 0.3 | 7.3 | Non |
|  |  |  | 90-120 | 8.4 | 0.4 | 4.3 | Non |
|  |  | Pit2 | 0-30 | 9.3 | 0.7 | 43.9 | Sodic |
|  |  |  | 30-60 | 8.6 | 0.3 | 6.4 | Non |
|  |  |  | 60-90 | 8.5 | 0.4 | 4.5 | Non |
|  |  |  | 90-120 | 8.4 | 0.3 | 5.6 | Non |
| 15 | Gulmit | Pit1 | 0-30 | 9.1 | 0.4 | 40 | Sodic |
|  |  |  | 30-60 | 8.7 | 0.9 | 34.3 | Sodic |
|  |  |  | 60-90 | 8.6 | 0.3 | 19.7 | Sodic |
|  |  |  | 90-120 | 9.6 | 0.8 | 7.7 | Non |
|  |  | Pit2 | 0-30 | 8.3 | 1.1 | 12 | Non |
|  |  |  | 30-60 | 8.5 | 0.6 | 11.5 | Non |
|  |  |  | 60-90 | 8.6 | 0.4 | 7.7 | Non |
|  |  |  | 90-120 | 8.8 | 0.3 | 7.8 | Non |
| 16 | Hormat-Abuarie | Pit1 | 0-30 | 7.7 | 2.9 | 6 | Non |
|  |  |  | 30-60 | 7.7 | 1.9 | 4.8 | Non |
|  |  |  | 60-90 | 7.7 | 0.8 | 4.4 | Non |
|  |  |  | 90-120 | 7.8 | 0.8 | 4.5 | Non |
|  |  | Pit2 | 0-30 | 7.9 | 1.5 | 7.7 | Non |
|  |  |  | 30-60 | 8 | 1.1 | 5.6 | Non |
|  |  |  | 60-90 | 8.2 | 0.5 | 5.6 | Non |
|  |  |  | 90-120 | 8.1 | 0.7 | 4.8 | Non |
| 17 | Arategna | Pit1 | 0-30 | 7.8 | 4.4 | 20.1 | Saline-sodic |
|  |  |  | 30-60 | 7.8 | 1.2 | 17 | Sodic |
|  |  |  | 60-90 | 8 | 0.9 | 17.5 | Sodic |
|  |  |  | 90-120 | 8.2 | 0.9 | 15.5 | Sodic |
|  |  | Pit2 | 0-30 | 7.5 | 8.5 | 24.1 | Saline-sodic |
|  |  |  | 30-60 | 7.8 | 3.3 | 22.1 | Sodic |
|  |  |  | 60-90 | 8 | 1.3 | 14.7 | Non |
|  |  |  | 90-120 | 7.8 | 0.9 | 14.8 | Non |
| 18 | Golina-Bergumelie | Pit1 | 0-30 | 8.5 | 1.2 | 35.3 | Sodic |
|  |  |  | 30-60 | 8.3 | 0.6 | 17.4 | Sodic |
|  |  |  | 60-90 | 8 | 1.1 | 17.6 | Sodic |
|  |  |  | 90-120 | 7.8 | 1 | 22.2 | Sodic |
|  |  | Pit2 | 0-30 | 8.2 | 2 | 34.9 | Sodic |
|  |  |  | 30-60 | 8.2 | 1.7 | 28.2 | Sodic |
|  |  |  | 60-90 | 8.4 | 0.5 | 20.2 | Sodic |
|  |  |  | 90-120 | 8.4 | 0.5 | 23.1 | Sodic |
| 19 | Koba | Pit1 | 0-30 | 7.3 | 2.1 | 4.7 | Non |
|  |  |  | 30-60 | 7.8 | 1.8 | 3.7 | Non |
|  |  |  | 60-90 | 7.9 | 1.2 | 4.4 | Non |
|  |  |  | 90-120 | 7.9 | 1.1 | 5.9 | Non |
|  |  | Pit2 | 0-30 | 7.5 | 4.2 | 4.4 | Saline |
|  |  |  | 30-60 | 7.8 | 2.2 | 4.5 | Non |
|  |  |  | 60-90 | 7.8 | 2.3 | 5.3 | Non |
|  |  |  | 90-120 | 7.8 | 2.5 | 6.2 | Non |
| 20 | Ambo | Pit1 | 0-30 | 9 | 2 | 28.9 | Sodic |
|  |  |  | 30-60 | 8.8 | 0.8 | 13 | Non |
|  |  |  | 60-90 | 8.6 | 0.6 | 7.3 | Non |
|  |  |  | 90-120 | 8.7 | 0.6 | 9.5 | Non |
|  |  | Pit2 | 0-30 | 9.7 | 0.6 | 33.4 | Sodic |
|  |  |  | 30-60 | 9.4 | 0.5 | 23.1 | Sodic |
|  |  |  | 60-90 | 8.9 | 0.5 | 23.4 | Sodic |
|  |  |  | 90-120 | 8.4 | 0.5 | 18.3 | Sodic |
| 21 | Upper-Tuloabajibo | Pit1 | 0-30 | 8.7 | 13.4 | 227.1 | Saline-sodic |
|  |  |  | 30-60 | 8.3 | 8.6 | 82.5 | Saline-sodic |
|  |  |  | 60-90 | 8.3 | 6.7 | 28.6 | Saline-sodic |
|  |  |  | 90-120 | 8.1 | 6.1 | 17.8 | Saline-sodic |
| 22 | Middle-Tuloabajibo | Pit1 | 0-30 | 7.6 | 53.4 | 63.9 | Saline-sodic |
|  |  |  | 30-60 | 7.8 | 10.2 | 57.7 | Saline-sodic |
|  |  |  | 60-90 | 7.9 | 7.8 | 52.2 | Saline-sodic |
|  |  |  | 90-120 | 8 | 5.2 | 34 | Saline-sodic |
| 23 | Lower-Tuloabajibo | Pit1 | 0-30 | 9.7 | 1.1 | 56.1 | Sodic |
|  |  |  | 30-60 | 9 | 0.5 | 13.7 | Non |
|  |  |  | 60-90 | 8.7 | 0.4 | 8.1 | Non |
|  |  |  | 90-120 | 8.7 | 0.5 | 8.5 | Non |
| 24 | Swak | Pit1 | 0-30 | 8.12 | 0.23 | 2.758 | Non_saline Non_Sodic |
|  |  |  | 30-60 | 8.05 | 0.19 | 3.44 | Non_saline Non_Sodic |
|  |  |  | 60-90 | 8.11 | 0.19 | 2.94 | Non_saline Non_Sodic |
|  |  |  | 90-120 | 8.22 | 0.14 | 2.881 | Non_saline Non_Sodic |
|  |  | Pit2 | 0-30 | 8.09 | 0.18 | 3.908 | Non_saline Non_Sodic |
|  |  |  | 30-60 | 8.05 | 0.17 | 3.921 | Non_saline Non_Sodic |
|  |  |  | 60-90 | 8.19 | 0.17 | 4.252 | Non_saline Non_Sodic |
|  |  |  | 90-120 | 8.36 | 0.29 | 5.72 | Non_saline Non_Sodic |
|  |  | Pit3 | 0-30 | 6.28 | 0.07 | 2.179 | Non_saline Non_Sodic |
|  |  |  | 30-60 | 7.88 | 0.61 | 2.778 | Non_saline Non_Sodic |
|  |  |  | 60-90 | 8.39 | 0.27 | 2.908 | Non_saline Non_Sodic |
|  |  |  | 90-120 | 8.45 | 0.29 | 3.43 | Non_saline Non_Sodic |
| 25 | Seraba | Pit1 | 0-30 | 6.92 | 0.09 | 2.71 | Non_saline Non_Sodic |
|  |  |  | 30-60 | 7.35 | 0.1 | 3.648 | Non_saline Non_Sodic |
|  |  |  | 60-90 | 7.7 | 0.11 | 4.798 | Non_saline Non_Sodic |
|  |  |  | 90-120 | 7.92 | 0.12 | 5.825 | Non_saline Non_Sodic |
|  |  | Pit2 | 0-30 | 6.01 | 0.27 | 3.519 | Non_saline Non_Sodic |
|  |  |  | 30-60 | 6.21 | 0.19 | 3.97 | Non_saline Non_Sodic |
|  |  |  | 60-90 | 6.38 | 0.24 | 6.232 | Non_saline Non_Sodic |
|  |  |  | 90-120 | 6.99 | 0.27 | 7.804 | Non_saline Non_Sodic |
|  |  | Pit3 | 0-30 | 7.08 | 0.24 | 2.732 | Non_saline Non_Sodic |
|  |  |  | 30-60 | 8.27 | 0.23 | 11.895 | Non_saline Non_Sodic |
|  |  |  | 60-90 | 7.83 | 0.17 | 7.021 | Non_saline Non_Sodic |
|  |  |  | 90-120 | 7.28 | 0.15 | 4.166 | Non_saline Non_Sodic |
| 26 | Dengora | Pit1 | 0-30 | 8.26 | 0.3 | 3.232 | Non_saline Non_Sodic |
|  |  |  | 30-60 | 8.25 | 0.24 | 2.911 | Non_saline Non_Sodic |
|  |  |  | 60-90 | 8.16 | 0.25 | 3.42 | Non_saline Non_Sodic |
|  |  |  | 90-120 | 8.3 | 0.19 | 6.331 | Non_saline Non_Sodic |
|  |  | Pit2 | 0-30 | 7.93 | 0.25 | 3.16 | Non_saline Non_Sodic |
|  |  |  | 30-60 | 8.11 | 0.19 | 5.052 | Non_saline Non_Sodic |
|  |  |  | 60-90 | 8.22 | 0.21 | 6.477 | Non_saline Non_Sodic |
|  |  |  | 90-120 | 8.36 | 0.28 | 8.063 | Non_saline Non_Sodic |
|  |  | Pit3 | 0-30 | 8.15 | 0.27 | 3.74 | Non_saline Non_Sodic |
|  |  |  | 30-60 | 8.36 | 0.27 | 6.124 | Non_saline Non_Sodic |
|  |  |  | 60-90 | 8.4 | 0.3 | 9.061 | Non_saline Non_Sodic |
|  |  |  | 90-120 | 8.44 | 0.33 | 12.469 | Non_saline Non_Sodic |