Amhara National Regional State

Water Resources Development Bureau

(BOWRD)

Detail Design

Of

Golina Diversion Small-Scale Irrigation Project

Volume IV: Engineering Design Final Report

July, 2014(Revise 2015)

Bahir Dar

Client: Bureau of Water Resource Development

(BoWRD)

Address:

P. O. Box: 88
Telephone: 0528-200853/855
Fax: 251-08-20-65-68/204676/202040

Consultant: Amhara Design & Supervision Works Enterprise

(ADSWE)

Address:

P. O. Box: 1921
Telephone: +251-582-181023/ 180638/181201/181254
Fax: (058) 2180550/ (058) 2180560
E-mail: amhara design@yahoo.com

Bahir Dar, Ethiopia

# FEASIBILITY STUDY & DETAIL DESIGN REPORT STRUCTURE

- Volume I: Watershed Management

- Volume II: Engineering Geology

- Volume III: Irrigation Agronomy

- Volume IV: Engineering Design

- Volume V: Socio Economy

- Volume VI: Environmental Impact Assessment

Table of Contents								 Page Nr

FEASIBILITY STUDY & DETAIL DESIGN REPORT STRUCTURE	i

LIST OF TABLES	iv

LIST OF FIGURES	v

SAILENT FEATURE	vi

1	INTRODUCTION	1

1.1	Background	1

1.1.1	Description of the Project Area	1

1.2	Objectives of the Study	3

1.2.1	Major Objective	3

1.2.2	Specific Objectives	3

1.3	Scope of the Study	4

1.4	Methodology	4

SECTION-I:  HYDROLOGY	7

2	HYDROLOGY	9

2.1	Watershed Characteristics	9

2.2	Hydro-Metrological Data Availability	10

2.2.1	Climate	10

2.2.2	Rainfall Data	10

2.2.3	River flow data	10

2.2.4	Upstream & Downstream utilization	11

2.3	Design Flood Analysis	11

2.3.1	Design Rainfall computation	11

2.3.2	Outlier Test	11

2.3.3	Check for variance	13

2.3.4	Peak Discharge Determination	14

2.3.5	Tail Water Depth Computation	18

SECTION-II: HEADWORK DESIGN	22

3	HEADWORK STRUCTURES DESIGN	24

3.1	Headwork Site Selection	24

3.2	River Geomorphology	24

3.2.1	River Bank condition	26

3.3	Sources of construction materials	26

3.3.1	Rock for Masonry and Crushed Coarse Aggregate	27

3.3.2	Fine Aggregates	28

3.3.3	Water	28

3.4	Headwork Type Selection	28

3.4.1	Hydraulic Design of Headwork Structure	29

3.4.2	River Bed Protection	30

3.4.3	Design of Scouring Sluice and Head Regulator	30

3.4.4	Gate Provision	31

3.4.5	Trash Rack	33

3.5	Operation Slab and Breast Wall	33

3.5.1	Intake Breast wall	33

3.5.2	Under sluice Breast Wall	33

3.5.3	Operation Slab	34

3.6	Design of Guide wall and Protection Works	35

3.6.1	Upstream and Downstream  Guide Wall	35

3.6.2	Downstream guide Wall	37

3.7	Bill of Quantity and Cost Estimation	41

SECTION-III: IRRIGATION AND DRAINAGE SYSTEMS INFRASTRUCTURE	45

3.8	Irrigable Area Description	46

3.8.1	Topography	46

3.8.2	Climate	46

3.8.3	Soil characteristics	47

3.8.4	Existing Irrigation Practices in the Project Area	47

3.9	Irrigation Water Requirement	47

3.9.1	Crop Water Requirement (CWR)	47

3.9.2	Irrigation efficiency (Ep)	47

3.9.3	Irrigation duty	48

3.9.4	Irrigation methods	49

3.10	Irrigation and Drainage System Layout	50

3.10.1	Conveyance System	50

3.11	Design of the Canal System	50

3.11.1	Main Canal	51

3.11.2	Secondary Canals	51

3.11.3	Tertiary canals	52

3.11.4	Field Canals	53

3.12	Drainage Canals	54

3.12.1	Catch drainage canals	54

3.12.2	Secondary canal drainage canals	55

3.13	Canal Structures Design	55

3.13.1	Design of a typical flume	55

3.13.2	Design of Division box	55

3.13.3	Road crossing structure	58

3.14	Irrigation Infrastructure Bill of Quantities and Cost Estimate	59

4	CONCLUSION AND RECOMMENDATION	66

5	OPERATION AND MAINTENANCE	67

5.1	General	67

5.2	Operation of the Head Works	67

5.3	Irrigation System Operation	67

5.4	Maintenance Requirement	68

REFERENCE	69

# LIST OF TABLES

Table 2-1: Outlier test analysis	11

Table 2-3: Determination of Time of Concentration	14

Table 2-4: Runoff analysis	16

Table 2-5: Hydrograph coordinates	16

Table 2-6: Intake Site River Cross section Coordinate Data	18

Table 2-7: Stage discharge analysis	19

Table 2-8: River discharge computation at different stages of flow	21

Table 3-2: Typical design of masonry retaining wall	37

Table 4-1: Hydraulic Parameters of main canal	51

Table 4-2: Hydraulic Parameters of secondary canals	51

Table 4-3: Hydraulic Parameters of tertiary canals	53

Table 4-5: Hydraulic Parameters of Flume	55

Table 4-6: Hydraulic parameters of Division Boxes	58

# LIST OF FIGURES

Figure 1-1: Location map of the project area	2

Figure 2-2: Complex Hydrograph	17

Figure 2-3: Rating Curve	19

Figure 2-4: River profile	20

Figure 3-1: River bed at the proposed weir/intake site	25

Figure 9,Rock Quarry Site for Masonry Stone and Crushed Aggregates	27

Figure 4-1: Typical Cross Section of secondary canals	52

Figure 4-2: Typical Cross Section of tertiary canals	52

Figure 4-3: Typical Field Canal X-section	54

Figure 4-4: Typical Cross Section of Catch drain canals	54

Figure 4-5: Typical Division Box plan	56

# SAILENT FEATURE

- Project name: Golina Diversion Intake Irrigation Project

- Name of the stream: Golina river

- Location of the intake site using

- East:575697

- North:1333931

- Zone: North Wolo

- Wereda: Kobo

- Average Altitude: 1366 masl

- Hydrology

- Design rainfall: 106.536 mm

- Catchment area: 358.897 Km2

- Longest flow path length: 41.38 Km

- Design flood: 496.77 m3/sec

- Design base flow: 429 lit/se.

- Intake

- Intake type: Reinforced Concrete

- Intake crest level: 1368.87 m.a.s.l

- U/S HFL: 1371.37 m.a.s.l

- D/s HFL: 1371.37 m.a.s.l

- Under sluice

- Sill level: 1368.27 m.a.s.l

- Dimension: 0.8*1.2 m2

- Discharge amount: 0.34 m3/sec

- Outlet

- Sill level: 1368.37 m.a.s.l

- Opening dimension: 0.8*1.2 m2

- Discharge capacity: 0.34 lit/sec.

Irrigation and drainage systems Infrastructure

- Command area size: 170 ha

- Type of soil of the command area is dominantly black cotton clay soil

- Design discharge of the main canal =360 l/sec

- Irrigation system layout consists of one lined and earthen canal, two secondary canals and 16 tertiary canals

Project cost

| Bill No. | Descrption | Amount (Birr) | Remark |
| --- | --- | --- | --- |
| 1 | General item | 1,879,332.55 |  |
| 2 | Head Work | 3,707,642.29 |  |
| 3 | Main, Secondary, Tertiary Canal and Catch Drain | 7,789,952.07 |  |
| Total |  | 13,376,926.90 |  |
| VAT 15% |  | - |  |
| Grand Total |  | 13,376,926.90 |  |

# INTRODUCTION

## Background

In Ethiopia, under the prevalent rain-fed agricultural production system, the progressive degradation of the natural resource base, especially in highly vulnerable areas of the highlands coupled with climate variability have aggravated the incidence of poverty and food insecurity. The major source of growth for Ethiopia is still conceived to be the agriculture sector. Hence, this sector has to be insulated from drought shocks through enhanced utilization of the water resource potential of the country, (through development of small-scale irrigation, water harvesting, and on-farm diversification) coupled with strengthened linkages between agriculture and industry (agro-industry), thereby creating a demand for agricultural output. In line with the above, efforts have been made by the government to improve the situation in the country in areas of domestic water supply provision, irrigation, watershed management, etc. The Amhara Water Resources Development Bureau is playing its role in the development of small scale irrigation projects in the region. Accordingly, as part of the water sector development program, the office has initiated the study and design of a small scale irrigation scheme on Golina  River at Afaf  Kebele and signed an agreement with Amhara Design & Supervision Works Enterprise (ADSWE) for the study and design of the project.

### Description of the Project Area

#### Location

This irrigation project is located mainly at Afaf  Kebele, Kobo  Wereda of North Wolo  Zone in the Amhara Region. The proposed irrigation project is to be undertaken on Golina River and the headwork structures are specifically located at an altitude of about 1373 masl and geographical coordinates of 1333860 N (UTM) and 575662 E (UTM).

Figure 1-1: Location map of the project area

#### Accessibility

The project area is accessed through asphalt road 5Km from Kobo to Woldiya road, and turning to the right by passing 15Km all weather road to Afaf Kebele.

Previous Irrigation Practices

There are modern and traditional diversions on the upstream/downstream of this river using different irrigation practices but as the hydrology and Hydrogeology study and respondent farmers indicated, the river has capacity of recharging as it stretches down from the source area of the river. As a result there will not be a marked reduction or fluctuation of water flows both for the already existing and the newly proposed irrigation schemes. The traditional irrigation practices are under taken by individual farmers that use the river flow to the extreme left side is with hardship. So, the farmers in the project area are very much interested to upgrading the traditional scheme to modern scheme.

## Objectives of the Study

### Major Objective

The project area faces variability of rainfall distribution though the overall rainfall generally suffices the rain-fed agriculture. Accordingly, the rain-fed agriculture needs means of supplementing during distribution failures and further full irrigation is required to maximize the use of the potential land and water resources.

Hence the objective of this project is to contribute a substantial share in the effort to reduce the risk of production decrease due to rainfall variability and increase the productivity of the resource in the project specific area. Specifically, the project is targeted for the following.

- To make sustainable the rain-fed crop production and make extra production in the dry season possible for 170 ha of land through irrigation.

- There is a general consensus that irrigation investments will achieve broader food security and poverty reduction impacts and if efforts are also geared towards up-grading existing traditional farming practices with support to enhance access to input supply, output marketing and extension to facilitate access to information and innovations.

- This objective is to be realized by constructing Intake structures across the Golina River and diverting the river flow.

### Specific Objectives

Other benefits that can be expected to appear with the launching of the project are:

- Efficiency of water use improvement;

- Improved local nutrition/food security gains;

- Improved management of scarce natural resources (land and water);

- Resilience against drought risk;

- Rationale for erosion control and watershed management;

- Rationale for the intensification and modernization of small-holder agriculture and rural lifestyles.

The engineering study and design enables the realization of the project by the provision of engineering structures that will allow the appropriate abstraction of the river water for delivery in to the identified irrigation fields of the study area. Hence, this engineering design is specifically targeted to:

- Analyze hydrologic requirements of the project and engineering structures;

- The formulation of sound and stable structure,  with necessary provisions that allow safe, easy and low-maintenance operation in the service life of the project;

- Develop working drawings;

- Estimation of construction costs.

## Scope of the Study

- The irrigation design shall ensure reliability, equity and flexibility of water delivery to farmers. It will aim at reducing conflicts among water users and will lead to lower operation and maintenance costs.

- Updating the existing, if available, computation of the actual evapo-transpiration, crop water requirement, irrigation demand/duty using the existing and recent agronomic, climatologic and soil data using more appropriate methodologies.

- Establish design criteria for irrigations structures to be approved by the client and to be used in the final design stage,

- Design proper irrigation system compatible with local conditions and management capabilities,

- Establish flood protection measures for the command area and canal structures and design the respective drainage system accordingly,

- Planning  and layout of the irrigation system, which include irrigation canals, drainage channels, inspection roads and alignments, canal spacing, canal length, location of structures, and water profiles along canal and drains at specified reaches, which is most economical easily manageable and aligned with topographic feature and geological investigation.

- Determination and estimation of water application conveyance and other losses and irrigation efficiencies and consideration of those parameters in design steps.

- Check and test hydraulic and structural designs of main canal considering total demand and the required capacity and the base flow availability,

- Prepare general plans and drawings for all irrigation infrastructure and irrigation systems designs,

## Methodology

In the study and design procedure, Designers used the following steps.

- Specific Site identification:

  - Review of the reconnaissance survey conducted by the Client

  - 50,000 scale top map and GIS information

  - Local farmers interview and discussion

  - Wereda and Zone Agriculture section expertise

  - Previous studies

  - On foot travel along the river channel and farm areas.

- Topographic survey:

  - Surveying the headwork site and the Command area with sufficient radius, using Total station

- Flow estimation

  - Physical observation on flood mark indications and local information about high flood and critical flow condition of the river

  - Analyzing the recorded river flow data and use watershed inputs for further analysis.

  - Base flow estimated during the reconnaissance field visit by floating method.

- Irrigable area identification:

  - Using local information

  - 50,000 Topographic map, and GIS information, GPS to see elevation

The design report is organized in three sections. In Section I the Hydrology study is presented and in Sections II and III the Headwork and Irrigation and Drainage Systems designs are discussed respectively. In Section III, planning and design of the irrigation system after diverting the water using the intake will be dealt. The following are major areas of concern in this part.

- Study and design of the irrigation method to be adopted,

- Study and design of the irrigation system layout and associated structures,

- Design of the different conveyance canals,

- Planning and design of the different irrigation and drainage structures,

- Preparation of the longitudinal profiles of the different irrigation and drainage canals.

# SECTION-I:  HYDROLOGY

# HYDROLOGY

## Watershed Characteristics

The Watershed has marked topographic variation. All types of slopes are present. The dominant slope class is 15-30% which covers 42.62% of the total area followed by 8-15% which is 29.1%. Flat and almost Flat and Gentle Slopping accounts 1.92 and 12.58 percent respectively, very steep slopes also cover about 12.95% of the total area. Table 1 of the watershed feasibility study report shows the slope classes and proportion of the watershed.

Certain physical properties of watersheds significantly affect the characteristics of the runoff and sediment yield and are of great interest in hydrologic analyses. The rate and volume of runoff, and sediment yield from the watershed have much to do with shape, size, slope and other parameters of the landscape. These suggest that there should be some important relations between basin form and hydrologic performance. If the basin and hydrologic characteristics are to be related, the basin form must also be represented by quantitative descriptors. These parameters can be measured from maps.

The watershed characteristics are analyzed and presented in Table 2 of the Watershed Feasibility Study Report of the same project. In summary:

- Catchment Area  = 358.897km2

- Stream Length = 41.38Km

- CN(II) = 75.94

(Extracted from the Watershed Study Report of the same project)

At the selected reference point, the area of Golina catchment is 358.897km2 and consists of a network of tributaries.

Golina river at the headwork site is characterized by undefined channel system. Years before the river were flowing near the right bank. By changing its courses constantly and eroding a large command area, it widened its channel more to the left. It looks that the gradient of the river is getting low and hence there exists significant deposition of sediment mainly silt and cobbles.

## Hydro-Metrological Data Availability

### Climate

Small scale irrigation project designers and planners are faced with lack of good data on the hydrology of the stream/river system that will be their water source and on local weather and climate conditions. Stream gauging stations are virtually non-existent in remote rural areas of Ethiopia; meteorological stations are almost rare. Likewise, at Afaf  Kebele (Project area location) and in the catchment area of this project, there is no meteorological station of any level. Moreover, there are no flow data for the river near the project. Therefore, data for the hydro-meteorological analysis is taken from the nearby station and similar areas. Rainfall & temperature data are considered from Lalibela Meteorological station. After analyzing the data the study team decided to use Lalibela metrological station data  because the rain coming from the highland areas is the source of runoff for Golina catchment and this is confirmed by the local farmers and information gathered from the study area, the heaviest rainfall  from Muja and Kobo metrological stations has small number of years of recorded data and cannot be used for frequency analysis for recurrence interval more than 25years.Inaddition due to the vulnerability of the site for flooding and bank erosion that endangers the nearby village and lateral shifting of the bank more to the left thus the abundant years of recorded data of Lalibela station is used  in order not to under size structures designed based on this input.

As per the data of the station, March – April are identified as high temperature periods whereas December–January are low temperature periods. The mean annual rainfall amount is more than 1200mm (1995 - 2006 data) and most of it occurs from June to August.

### Rainfall Data

In order to compute the design flood for the intake structure, the daily maximum rainfall is collected from Lalibela Metrological stations with a record of 29 years.

### River flow data

The base flow which is measured on 04/08/2006 is 429 l/s. Since this base flow is measured during the dry months of the year, this figure is adopted for design.

### Upstream & Downstream utilization

Downstream of the proposed site, appreciable need for water is anticipated for locals and cattle provisions. Therefore, at least 21% of the minimum flow has to be released for downstream requirements because there is no irrigation to be proposed for future.

For the sake of planning and design, however, the outlet for the intake is designed for a discharge of 340l/s for this project and the project is to be developed for 170 ha of land, which is most of the time achievable as the flow for most of the time is significant to support this size of command area.

## Design Flood Analysis

For the design and analysis of structures to be constructed on the river, estimation of flood magnitude is an important task. This can be done using different techniques depending on the data available. For this particular case, there are no river flow data and hence the flood estimation is done using the rainfall data and applying SCS Curve Method.

### Design Rainfall computation

Based on the data of 24hr peak rainfall given in Table 1 the design rainfall, Rf is computed using Gumble’s Extreme Value Method.

### Outlier Test

Higher Limit, YH=Ymean+Kn*Sy,      Kn = 2.549 for 29Years of data.

Lower Limit, YH=Ymean-Kn*Sy,     Kn = 2.549 for 29 Years of data.

Table 2-1: Outlier test analysis

| Raw Data |  |  |  |  |
| --- | --- | --- | --- | --- |
| Year | Max Rain fall (x) | Descending Order(x) | Rank | Logarithmic Value(y) |
| 1978 | 57.5 | 92 | 1 | 1.964 |
| 1979 | 83.5 | 83.5 | 2 | 1.922 |
| 1980 | 33.5 | 82.5 | 3 | 1.916 |
| 1981 | 70.5 | 74 | 4 | 1.869 |
| 1982 | 92 | 73.1 | 5 | 1.864 |
| 1983 | 48.5 | 70.5 | 6 | 1.848 |
| 1984 | 38.35 | 68.2 | 7 | 1.834 |
| 1985 | 28.2 | 66 | 8 | 1.820 |
| 1986 | 38.5 | 61.8 | 9 | 1.791 |
| 1987 | 36 | 61.6 | 10 | 1.790 |
| 1988 | 45.9 | 57.5 | 11 | 1.760 |
| 1989 | 40.6 | 52.9 | 12 | 1.723 |
| 1990 | 43.25 | 51.1 | 13 | 1.708 |
| 1991 | 51.1 | 51.1 | 14 | 1.708 |
| 1992 | 36.2 | 48.5 | 15 | 1.686 |
| 1993 | 66 | 45.9 | 16 | 1.662 |
| 1994 | 82.5 | 43.25 | 17 | 1.636 |
| 1995 | 36.5 | 40.6 | 18 | 1.609 |
| 1996 | 68.2 | 38.6 | 19 | 1.587 |
| 1997 | 51.1 | 38.5 | 20 | 1.585 |
| 1998 | 38.6 | 38.35 | 21 | 1.584 |
| 1999 | 34.6 | 36.5 | 22 | 1.562 |
| 2000 | 52.9 | 36.2 | 23 | 1.559 |
| 2001 | 73.1 | 36 | 24 | 1.556 |
| 2002 | 61.6 | 36 | 25 | 1.556 |
| 2003 | 61.8 | 34.6 | 26 | 1.539 |
| 2004 | 29.2 | 33.5 | 27 | 1.525 |
| 2005 | 36 | 29.2 | 28 | 1.465 |
| 2006 | 74 | 28.2 | 29 | 1.450 |
| SUM |  | 1509.7 |  | 49.078 |
| MEAN |  | 52.059 |  | 1.692 |
| STANDARD DEVIATION |  | 17.939 |  | 0.147 |
| SKEWINESS COEFICIENT |  |  |  | 0.201 |
| NUMBER OF DATA |  |  | 29 |  |

Determination of the threshold value of higher outliers:

Higher outlier is given by: Yh=Ymean + Kn*Sy

Where: Ymean =mean of data in log unit

Kn=for sample size,n=29 from table

Sy=Standard deviation of data log units

For     Ymean=1.692, Sy = 0.147, Kn=2.549

Then Yh=1.692+2.549*0.147=2.066

Yh=102.066=116.43mm

The highest recorded data ie.92 mm is lower than the higher outlier value so no data will be eliminated.

Determination of the threshold value of lower outlier:

Lower outlier is given by: Y1 = Ymean – Kn*Sy

Y1=1.692-2.549*0.147=1.319

Lower outlier is = 101.319=20.828mm

The lowest recorded data i.e. 28.2 mm is not lower than the lower outlier therefore no data will be eliminated and the twenty nine years recorded data is used for storm analysis.

### Check for variance

After checking the outliers, the data should be checked for variability. For variability the formula used is

α=δ∩-1N*Mean*100%

Where, δn-1 = Standard deviation =17.939

N = Nr of recorded data =29

Mean = 52.059

α = Standard error

α=17.93929*52.059*100%=4.61<10%  Acceptable

Therefore the data shows no variability.

### Peak Discharge Determination

#### General

The River is not gauged river. The design flood is calculated by using SCS unit hydrograph method. Thus, it is preferred to base the flood analysis on rainfall data, which are better both in quantity and quality of data. In the hydrologic analysis for drainage structures, it must be recognized that there are many variable factors that affect floods. Some of the factors that need be recognized and considered on an individual site by site basis are; rainfall amount and storm distribution; catchment area, shape and orientation; ground cover; type of soil; slopes of terrain and stream(S); antecedent moisture condition; Storage potential (over bank, ponds, wetlands, reservoirs, channel, etc.)

#### Peak flood analysis by SCS unit hydrograph method

Design flood is calculated SCS (The United States Soil Conservation Service). This method is widely adopted and more reliable method for flood estimation. The approach considers, watershed parameters, like Area, Curve number, and time of concentration.

#### Time of concentration (Tc)

Time of concentration has been calculated by taking the stream profile of the longest streamline and dividing it in to different elevation. Kirpich formula is adopted for computation.

Table 2-2: Determination of Time of Concentration

| Partial(km) | Cumulative(km) | elevation(m() | change of elevation | Tc hr |
| --- | --- | --- | --- | --- |
| 0 | 0 | 3580 | 0 | 0 |
| 3.519 | 3.519 | 3100 | 480 | 0.376 |
| 4.035 | 7.554 | 2420 | 680 | 0.386 |
| 13.606 | 21.16 | 1740 | 680 | 1.569 |
| 20.22 | 41.38 | 1370 | 370 | 3.135 |
|  | time of concentration in hour |  |  | 5.466 |

The formula is,

Tc=0.948L13H10.385+L23H20.385+…+Ln3Hn0.385

- Tc = 5.466	Since Tc > 3hr., duration of excess rainfall difference, D = 1hr.

- Time to peak,

Tp=D2+0.6*Tc  = 3.78hr

- Base time,

Tb=2.67*Tp  = 10.09hr.

- Recession time,

Tr=1.67*Tp  = 16.85hr.

#### Curve number (CN)

Curve number (CN) is achieved based on USSCS method by watershed characterization in terms of land cover, treatment, hydrologic condition and soil group. From the watershed analysis curve number at condition II =75.94. Since peak rainfall is found at an antecedent moisture condition III state, this value has to be changed to antecedent moisture condition III.

- Conversion factor = 1.150

- CN Condition (III) = (Factor from Table x CN condition II) =75.94*1.150 = 87.33. For detail analysis of the computation, Refer Excel file, attached here with.

#### Area Rainfall

As the area of the catchment gets larger, coincidence of all hydrological incidences becomes less and less. This can be optimized by changing the calculated point rainfall to aerial rainfall. The conversion factor is taken from standard table that relate directly with the size of watershed area and type of the gauging station. (IDD manual)

For the case of Golina irrigation project,

- Total watershed area = 358.897 Km2

- Type of gauging station = Daily rainfall (24 hr.)

- Aerial Rainfall = (Point Rainfall) x (Conversion factor)

#### Run off Analysis

Input data:

Design Point Rainfall = 106.536mm

Curve number at antecedent moisture condition III = 87.33

Catchment Area, A = 358.897 Km2

Tc = 5.466hr, D = 1hr., Tp = 3.78 hr; Tb = 10.09 hr; Tr = 16.85 hr.

Direct run-off,

- Q=(I-0.2*S)2(I+0.8*S)

- Where, I = Rearranged cumulative run-off depth (mm

S = Maximum run off potential difference,

- S=25400CN-254

Peak run-off for incremental;

- Qp=0.21*(A*Q)Tp

- Where,   A = Catchment area = 358.897 Km2

- Tp = Time to peak (hr)

- Q = Incremental run-off (mm)

Table 2-3: Runoff analysis

| RF Duration/D/ | DR cumulative runoff | Qp for 1mm runoff /m3/s/ | Qp for incremental run off m3/s/ | begin time | peak time | end time |
| --- | --- | --- | --- | --- | --- | --- |
| 0-1 | 0.00 | 0.00 | 0.0 | 0.0 | 3.77 | 10.07 |
| 1-2 | 0.38 | 0.38 | 0.1 | 1.0 | 4.77 | 11.07 |
| 2-3 | 3.21 | 2.83 | 9.1 | 2.0 | 5.77 | 12.07 |
| 3-4 | 12.31 | 9.10 | 112.0 | 3.0 | 6.77 | 13.07 |
| 4-5 | 21.28 | 8.97 | 190.8 | 4.0 | 7.77 | 14.07 |
| 5-6 | 25.57 | 4.29 | 109.7 | 5.0 | 8.77 | 15.07 |

Table 2-4: Hydrograph coordinates

| Time | Q1 | Q2 | Q3 | Q4 | Q5 | Q6 | Total Q |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 |  | 0 | 0 | 0 | 0 | 0.00 |
| 1 | 0 | 0 | 0 | 0 | 0 | 0 | 0.00 |
| 2 | 0 | 0.53 | 0 | 0 | 0 | 0 | 0.53 |
| 3 | 0 | 1.06 | 7.13 | 0 | 0 | 0 | 8.19 |
| 3.77 | 0 | 1.47 | 12.62 | 41.26 | 0 | 0 | 55.35 |
| 4.77 | 0 | 2 | 19.75 | 94.84 | 59.03 | 0 | 175.61 |
| 5.77 | 0 | 0.36 | 26.88 | 148.42 | 135.68 | 32.07 | 343.41 |
| 6.77 |  | 0.24 | 8.47 | 202.00 | 212.34 | 73.71 | 496.77 |
| 7.77 |  | 0.12 | 6.88 | 63.69 | 289.00 | 115.36 | 475.04 |
| 8.77 |  | 0.00 | 5.28 | 51.67 | 89.91 | 157.00 | 303.86 |
| 10 |  |  | 3.31 | 36.89 | 68.77 | 47.35 | 156.32 |
| 10.07 |  |  | 3.20 | 36.05 | 67.56 | 46.70 | 153.51 |
| 11 |  |  | 1.71 | 24.87 | 51.58 | 38.01 | 116.17 |
| 11.07 |  |  | 1.60 | 24.03 | 50.37 | 37.36 | 113.36 |
| 12 |  |  | 0.11 | 12.86 | 34.38 | 28.67 | 76.03 |
| 12.07 |  |  | 0.00 | 12.02 | 33.18 | 28.02 | 73.22 |
| 13 |  |  |  | 0.84 | 17.19 | 19.33 | 37.37 |
| 13.07 |  |  |  | 0.00 | 15.99 | 18.68 | 34.67 |
| 14 |  |  |  |  | 0.00 | 9.99 | 9.99 |
| 14.07 |  |  |  |  |  | 9.34 | 9.34 |
| 15 |  |  |  |  |  | 0.65 | 0.65 |
| 15.07 |  |  |  |  |  | 0.00 | 0.00 |

Figure 2-1: Complex Hydrograph

- From the analysis, the 50 year return period design run off is 496.77 m 3/s

### Tail Water Depth Computation

Tail water depth of the river is equal to the flood depth and amount at the proposed intake site before construction of the intake. It is used to crosscheck peak flood estimated by the SCS unit hydrograph method with flood mark method and to see the flood feature after the hydraulic jump. During field visit, the flood mark of the river at the proposed intake site was marked based on dwellers information and physical indicative marks. The river cross-section was surveyed.

Table 2-5: Intake Site River Cross section Coordinate Data

| Point no | Code | North | East | Elevation | Y-distance | X-distance | Partial distance | Cumulative distance |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1.00 | River center | 1333891.9 | 575416 | 1370.098 | 0.00 | 0.00 | 0.00 | 0.00 |
| 2.00 | River center | 1333897.3 | 575446.6 | 1369.902 | 5.40 | 30.62 | 43.78 | 43.78 |
| 3.00 | River center | 1333896 | 575520.5 | 1369.392 | -1.38 | 73.86 | 106.92 | 150.69 |
| 4.00 | River center | 1333888.1 | 575487.6 | 1369.427 | -7.88 | -32.85 | 123.17 | 273.86 |
| 5.00 | River center | 1333915.4 | 575572.8 | 1369.143 | 27.37 | 85.17 | 62.54 | 336.40 |
| 6.00 | River center | 1333916.5 | 575601.2 | 1368.958 | 1.03 | 28.45 | 22.23 | 358.63 |
| 7.00 | River center | 1333934.7 | 575643.7 | 1368.698 | 18.28 | 42.47 | 7.31 | 365.94 |
| 8.00 | River center | 1333948.5 | 575680.4 | 1368.429 | 13.74 | 36.73 | 23.17 | 389.12 |
| 9.00 | River center | 1333939.7 | 575711.6 | 1368.152 | -8.74 | 31.14 | 33.26 | 422.37 |
| 10.00 | River center | 1333958.8 | 575760.9 | 1367.936 | 19.08 | 49.35 | 52.91 | 475.28 |
| 11.00 | River center | 1333986.9 | 575802.3 | 1367.694 | 28.03 | 41.41 | 50.01 | 525.28 |
| 12.00 | River center | 1334009.7 | 575891.2 | 1367.141 | 22.84 | 88.90 | 91.79 | 617.07 |
| 13.00 | River center | 1334053.8 | 575944.8 | 1366.673 | 44.07 | 53.56 | 69.36 | 686.43 |
| 14.00 | River center | 1334059.8 | 576008.8 | 1366.251 | 6.02 | 64.01 | 64.29 | 750.72 |

Table 2-6: Stage discharge analysis

| Elevation | Depth (m) | Area(m2) | perimeter(m) | Hydraulic radius(m) | velocity (m2/sec) | Discharge (m3/sec) | Remarks |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1368.573 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |  |
| 1368.87 | 0.30 | 20.00 | 156.73 | 0.13 | 0.60 | 11.94 |  |
| 1369.17 | 0.60 | 44.48 | 201.80 | 0.22 | 0.86 | 38.24 |  |
| 1369.57 | 1.00 | 87.92 | 209.93 | 0.42 | 1.32 | 115.97 |  |
| 1370.07 | 1.50 | 141.09 | 215.58 | 0.65 | 1.78 | 250.64 |  |
| 1370.57 | 2.00 | 195.44 | 220.24 | 0.89 | 2.18 | 425.37 | Peak flood |
| 1371.07 | 2.50 | 250.99 | 226.82 | 1.11 | 2.52 | 632.92 |  |

Figure 2-2: Rating Curve

From the above stage discharge table and curve the maximum flood level corresponding to the computed design peak discharge is 1370.75(2.2 m from the river bed) and it is considered as the d/s high flood level i.e. expected at the intake axis before construction of the intake.

D/S HFL = 1370.75 masl.

- Average river bed slope

Average river bed slope of River is estimated by two different techniques. One is by end area method and the other is by using best fit line method

The water level of the river is taken at different points along the river channel around the head work site. Surveying work done for 100m length. And then, average water surface slope is considered as the river bed slope. For comparison of the two procedures, refer the attached Excel file.

Figure 2-3: River profile

- Manning’s Roughness coefficient

The Manning’s roughness coefficient is taken from standard table based on the river nature. The river at the headwork site has got braded feature and curving nature. The river banks are undefined and relatively smooth. Manning’s roughness coefficient (n = 0.03) is adopted.

- Discharge of the river

Input data:

- Manning's roughness coefficient, n = 0.03

- Average river bed slope, S = 0.005

- ,  	Where, R = Hydraulic radius = (Area/Perimeter)

- Q=V*A

Table 2-7: River discharge computation at different stages of flow

| Elevation | Depth (m) | Area(m2) | perimeter(m) | Hydraulic radius(m) | velocity (m2/sec) | Discharge (m3/sec) |
| --- | --- | --- | --- | --- | --- | --- |
| 1368.573 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |
| 1368.87 | 0.30 | 20.00 | 156.73 | 0.13 | 0.60 | 11.94 |
| 1369.17 | 0.60 | 44.48 | 201.80 | 0.22 | 0.86 | 38.24 |
| 1369.57 | 1.00 | 87.92 | 209.93 | 0.42 | 1.32 | 115.97 |
| 1370.07 | 1.50 | 141.09 | 215.58 | 0.65 | 1.78 | 250.64 |
| 1370.57 | 2.00 | 195.44 | 220.24 | 0.89 | 2.18 | 425.37 |
| 1371.07 | 2.50 | 250.99 | 226.82 | 1.11 | 2.52 | 632.92 |

# SECTION-II: HEADWORK DESIGN

# HEADWORK STRUCTURES DESIGN

## Headwork Site Selection

The headwork site is situated at 1333860 m N, 575662 m E and river bed elevation of 1373 m above sea level. At this site the river course is undefined, with wide width about 350m length. At this specific site, covered by recent alluvial deposit sediment at the river bed whereas the right and left bank .Is also made due to its mild slope and morphology it is observed that the river is in alluvial stage. Moreover by meandering deep into the outside curvature, now the river is becoming a trait to the nearby village. From this one can conclude that this river needs a proper training and guidance before it costs too much.

For this project a simple river bed level intake structure has been proposed for diverting the water to the main canal. The headwork site geological surface and subsurface conditions have been investigated based on the nature of the proposed structure. At the site and immediate vicinity, the stream flows along genteel slope course and its bed is mainly covered with recently deposited alluvial sediments. These sediments are dominated sand with some gravel. On the other hand, the banks of the stream at the headwork site are made up of different geologic materials; most part of the right bank is made up of old alluvial sediment of silt/clay materials. It is loss by nature that the river morphology is still unfinished. The left bank, on the contrary, has no clear difference with stream bed that had found at same level and also covered with some weight land. Detail geologic nature of the banks, and bed of the stream along the headwork axis and immediate vicinity are described and their potential geotechnical influence on the proposed structures also discerned/detected below, with remedial measures.

The different sections of the stream at the proposed headwork site are described separately below:-

## River Geomorphology

It is a common fact that the river development tends to accommodate itself to the local geology that develops along the structurally weak zones like faults, joints, folds, etc. The drainage system of the study area is strongly influenced by geological structures and formations, the nature of the vegetation cover and climate. The nature of geological formations and structures has also strong influence on the development of the channel.

The present morphology of the Golina River channel is a function of a number of processes and environmental conditions, including the composition of the bed and the banks. Golina River has 350 m of channel width at the intake site which is undefined. Years before the river were flowing near the right bank. By changing its courses constantly and eroding a large command area, it widened its channel more to the left.

At the dry season the river flows near the left bank. To allow maximum abstraction of water the intake is located at the outside curvature of the channel.

Both left bank and the bed are composed of loose silt clay soil and coarser alluvial sediment as the result the stream shows highly meandering nature both up and downstream from the proposed site but at the particular intake site it shows nearly straight river channel.

River Bed condition

At the proposed headwork site the stream bed or course is well defined, nearly straight, and broad and shows plain surface due to recent sediment accumulations. Along the intake axis, the bed is made up of one major geologic material, as seen from surface observation. These are recently deposited alluvial sediments,.

The stream bed (from bank to bank) is totally covered with the alluvial deposits. it is thick deposit of Sand (dominantly fine sand). It was difficult to investigate exact thickness of the sediments or depth of bedrock due to excessive surface, subsurface water inflow and proposed Spate had founded on this alluvial sediment.

Figure 3-1: River bed at the proposed weir/intake site

### River Bank condition

#### Right Bank

At the headwork site/axis, the right bank is characterized by relatively moderate to steep slope, having about 3m height from stream bed. It reveals nearly vertical section within this height.  From visual observation of the natural cuts at the bank, there are two distinct geological materials forming the bank section, these are:-

- Low to Medium Plastic Clay (CL), with some silt at the top and bottom of the bank

- Low plastic Silt intercalated with some Gravel (GM) old alluvium, at middle of the bank.

These two units have variable thickness/depth at the area. Just at the intake axis, the top clay soil has about 0.9m height, whereas the middle silt Gravel old sediment possesses 50cm thickness, under this 1.2m Clay once more take place. Further to the upstream, only thick layer of low plastic Clay with some Silt (ML) are found at the bank section. These overburden soil materials has been affected by erosion/ flood under cutting, which is widening the bank by forming nearly vertical slope. Such vertical slope configuration observed at headwork site and upstream of this bank which will lead to instability or collapse. Here, it is important to design the bank slope to stable configuration, just by providing appropriate slope (indifferent from the present vertical slope) or other protection work(Gabion) to prevent  initial main canal portion near to the bank from farther widening of bank.

#### Left Bank

At the proposed headwork axis and immediate vicinities, the left bank forms relatively genteel slope topography found at same level with that of stream bed. From surface observation, the bank is made up of one basic geological units. The unit is brown color low plastic silt (ML) with some Clay which are mostly covered with weight land "Fiela" at the headwork site and immediate vicinities. For geotechnical purpose, it is very weak and loose horizon in nature that the bank had mostly eroded at peak runoff time. So, it is necessary to provide some bank protection structures to prevent the river flow morphology changes from time to time.

## Sources of construction materials

During the site investigation, natural construction materials required for the construction of the various proposed engineering structures at the headwork and within the farmland have been assessed, and possible quarry sites and borrow areas have been identified within the vicinity of the area as much as possible. In addition to the identification, the quality, quantity, accessibility condition and ownership of each proposed production sites have also been studied and described in this report; on separate sub-sections below. The natural materials required for the construction of the proposed hydraulic structures include rock for masonry stones, aggregates (both coarse and fine), impervious soil for fill and/or lining, backfill soil, and water.

### Rock for Masonry and Crushed Coarse Aggregate

Quarry site that can be used for production of rock for masonry stone and crushed coarse aggregates has been assessed during the field work session within the vicinity of the project area at economic distance for hauling. One possible major quarry site has been identified .This site is found just on asphalt rode from Kobo to Waja at about 18km from headwork site. The rock is located in ---Kebele at its specific name Gara Lancha “Got”

Figure 2,Rock Quarry Site for Masonry Stone and Crushed Aggregates

At this quarry site, the available rock type is porpryrtic basalt which is magmatic in origin and occurs in the form of angular fragments of suitable sizes. It is light gray and coarse grained volcanic rock. Naturally, this rock is strong as it is generally fresh except very thin cover of weathered zones, which need to be removed during rock dressing and before crushing. At the quarry site, fresh basalt rock units are found in large quantity that suffices the requirement of the project. From this site, rock of different sizes can be produced; ranging from large boulders to coarse gravels that masonry stone and coarse aggregate can be produced easily.

### Fine Aggregates

Borrow areas for fine aggregate or natural sand have been assessed starting from the project stream itself. Natural deposits of such materials couldn’t be found when assessed within the beds of the stream in the project area; rather very fine sand deposit is found covering almost the entire bed of the Golina stream. Seeing to this nature of the stream, other distant streams have been explored to identify the best source areas for fine aggregate or natural sand that can be used for this particular project. During exploration of this natural sand, at a distant one stream was identified as a possible source of fine sand. The stream is known as ‘Horemat’. It is located at about 10km from the project site.

Both visual investigation and laboratory testing of the sand deposit found in the proposed area have been done. In the area good quantity, which suffices the need of this project, is available. The deposits are dominated by sand sized transported alluvial sediments, though some amount of fines and over sizes are also observed. The individual grains of sand are mainly originated from basalt rock and reveals high crushing resistant that the sand has good quality in this respect.

### Water

Water for construction purposes can be found from the project stream, Golina, itself. The stream is perennial throughout the year that there is some amount of flow along its course. During this field time the stream flow was more than 427L/second.

## Headwork Type Selection

Intake structure is the head work for abstraction of water to the command area. In a river as wide as 350m constructing weir is not feasible economically. Even if it is constructed the guide wall length and orientation is a bit costly. As it is described above, the river foundation is unconsolidated riverian deposit, which makes the weir body to be designed in pervious foundation principle resulting a massive weir body. Therefore the study team decided to plan an intake structure at the off taking point together with a bed stabilizing bed bar.

The bed bar is aligned 30 degree inclination across the main river channel that flows during the dry season. This bed bar is below the river bed that stabilizes the intake area and guarantees from possible lowering of river bed elevation and serves also as a cut off for abstracting more water to the off take .Golina river erodes the bank toes and deposits on the river bed, with its wider section and mild slope bed bar stability is not endangered. For more refer the plan view.

### Hydraulic Design of Headwork Structure

#### Side spill weir (Escape)

The excess flood more than the side spill weir rejects the design flood. The height and length of side upstream of the intake structure is determined after we calculate the design flood (0.34m3/sec) depth at the approaching canal section. The off taking canal upstream of orifice has a rectangular section and steeper slope of 0.001m/m to enable non-silting of canals. Using excel iteration the depth is calculated for design discharge of 0.34m3/sec to be    0.8m providing a free board of 0.3m. The excess flow above the design discharge will spill out by side escapes.

#### Outlet sill level determination.

The outlet sill level is fixed for the maximum abstraction of water during dry season and provided at 1368.37 masl .Since there is no structure to head up the water it is necessary to put the point of abstraction on the river bed possibly the lowest and it is provided at an elevation 1368.37maslby lowering 0.5m of minimum river bed level (1368.87masl).

#### Intake Dimension

The off take size is fixed taking in to consideration the base flow of the river during the dry season when the base flow is small and supplementary irrigation for ‘belg’ rainy season to maximize the command.

Golina River is a perennial river during rainy season the base flow increase a great deal therefore the coming flood will have a longer time of recession.

Hence our canal capacity and off take size is fixed to convey and pass 0.34m3/sec of discharge.

The intake dimension is computed using rectangular notch formula given by:-

Q = 2/3Cd*(2*g) 1/2B*H (3/2)

Where: Q=diverted discharge, 0.34m3/sec

Cd=Coefficient of discharge, Cd=0.6

B=Width of intake, 1.2m

H=Height of the intake

G=Acceleration due to gravity

Therefore for a width of 1.2m

H= (Q*3/ (2*Cd*(2*g)1/2*B))2/3

H=0.5

Therefore, provide intake dimension 1.2X0.8 width and height respectively.

### River Bed Protection

Stabilizing the bed is inevitable in intake irrigation projects, therefore a 50m bed bar inclined at 30 degree is provided to stabilize the channel flow.

The depth of bed is fixed by cut off principle, thus from Lacey's formula normal scour depth, R is given by:

R=1.35(q2//f)1/3  and  f=1.768 davg1/2=6.645

R=1.35((692/415)1/2/6.645) =1.4.

For alluvial silt deposit the depth provided will be.

D=2.25*R=3.04

Provide a bed bar with 3m depth and width of 0.3m.

### Design of Scouring Sluice and Head Regulator

#### Scouring Sluice

##### Size Determination

The scouring sluice size is fixed to accommodate two times as much of the diverted discharge, and the dimension is determined with the rectangular notch principle.

Q = 2/3Cd*(2*g) 1/2B*H (3/2)

Where: Q=diverted discharge, 0.34m3/sec

Cd=Coefficient of discharge, Cd=0.6

B=Width of intake, 1.2m

H=Height of the intake

G=Acceleration due to gravity

Therefore for a width of 1.2m we compute the height

H= (Q*3/(2*Cd*(2*g)1/2*B))2/3

=0.7 provide 0.9m height

### Gate Provision

#### Intake Gate

The water head at peak flood is 2.2m, and the hydrostatic pressure at the gate is calculated as:

Ph=ϒ H

=2.2*10KN/m3=22KN/m3

The allowable tensile and bending stress of the steel plate during wet season is 13500N/mm2.

Based on the theory of elasticity of plate bending stress the thickness is calculated as follows based on NBR recommendation and NBR -8883 standard.

ó = K/100*P*(a/t)2

Where: - a=minor support length

T=plate thickness

Ph=water pressure relative to the modular

K=dimensionless constant from table depending span ratio (b/a)

For b/a ratio of 1.3333, K = 42.512

T = (K*100*Ph*a2 /ó)

T=0.0048m

Provide a minimum thickness 6 mm.

For slide gates B2*h*H>2m4 should be fulfilled

Where: H=gate height, 0.9m

B=gate span, 1.2m

h=head of water over the sill which is U/S HFL-sill level=2.2m

B2*h*H=1.902 <2m4, Therefore stiffening materials are necessary to increase the weight of the gate. Therefore, two 16mm stiffening reinforcement bars are provided.

#### Under sluice Gate

The water head at peak flood is 2.2m, and the hydrostatic pressure at the gate is calculated as:

Ph=ϒ H

=2.2*10KN/m3=22KN/m3

The allowable tensile and bending stress of the steel plate during wet season is        13500N/mm2.

Based on the theory of elasticity of plate bending stress the thickness is calculated as follows based on NBR recommendation and NBR -8883 standard.

ó= K/100*P*(a/t)2

Where: - a=minor support length

T=plate thickness

Ph=water pressure relative to the modular

K=dimensionless constant from table depending span ratio (b/a)

For b/a ratio of 1.25, K = 39.6

T =(K*100*Ph*a2 /ó)

T=0.0092m

Provide a minimum thickness 10 mm.

For slide gates B2*h*H>2m4 should be fulfilled

Where: H=gate height, 0.9m

B=gate span, 1.3m

h=head of water over the sill which is U/S HFL-sill level=2.2m

B2*h*H=2.2 >2m4, Therefore no need of stiffening materials.

### Trash Rack

Trash rack is provided to exclude boulders and tree trunk from entering inside the pocket and clog the gates. Therefore for Golina intake Trash rack up to high flood level is provided.  1m height trash rack with 24mm diameter steel spaced at 100mm horizontally and vertically is provided.  At the top a horizontal 1m and 100mm diameter stiffening steel pipe is provided to anchor with walls.

## Operation Slab and Breast Wall

A vertical raised gate is designed for the head work and the gate slides over the breast wall using spindle during opening and closing. For easy operation slab is provided .The thickness of the breast wall and operation slab is fixed the same from the construction point of view.

### Intake Breast wall

Dimension 1.2X0.9X0.15 is provided

Minimum area of reinforcement 0.15% is taken along the respective direction =0.0015*900*150 =202.5mm2

Taking 50mm cover thickness, the effective depth is de=D-(50+12/2)=94

And spacing 200<3*de =282.

Therefore 12@200 mm C/C spacing is provided.

Area of steel 565.2mm2 >Minimum area of reinforcement, 0.15% of the concrete ie.202.5mm2.

### Under sluice Breast Wall

The escape (scouring sluice) size is fixed to accommodate 100% of the main channel discharge capacity

for flushing the sediment from the Disilting basin through escape channel the dimension is determined with the rectangular notch principle.

Dimension 0.8X0.9X0.15 is provided

Minimum area of reinforcement 0.15% is taken along the respective direction

=0.0015*1200*150 =270mm2

Taking 50mm cover thickness, the effective depth is de=D-(50+12/2)=94

And spacing 250<3*de =282.

Therefore 12@250 mm C/C spacing is provided.

Area of steel 678.24mm2 >Minimum area of reinforcement, 0.15% of the concrete ie202.5mm2.

### Operation Slab

Dimension 1.4X1.4X0.15 operation slab provided.

The minimum reinforcement area ie.0.15% =0.0015*1400*150 =315mm2

Taking 50mm cover thickness, the effective depth is de=D-(50+10/2)=95

And spacing 250<3*de =285.

Therefore 10@250 mm C/C spacing is provided.

Area of steel 471mm2 >Minimum area of reinforcement, 0.15% of the concrete ie.315mm2.

## Design of Guide wall and Protection Works

### Upstream and Downstream  Guide Wall

#### Height Determination

The guide wall height is fixed by deducting total energy level from lowest river bed level (LRBL) and adding free board as follows:

U/S HFL=1371.37

Velocity head=2.33m

LRBL=1368.87

0.3m free board is provided.

Therefore Height of bank=U/S HFL+hv+Fb-LRBL=2.20m

A guide wall 2.5 m height is provided.

#### Structural Stability

















| Masonry |  |  |
| --- | --- | --- |
| Top width, b | 0.50 | m |
| Bottom width, C | 2.20 | m |
| Top section height, h | 0.50 | m |
| Total height, H=U/S HFL- D/s apron level + FB + FD-D | 4.27 | m |
| Concrete base |  |  |
| Thickness of concrete base, D=H/8 to H/6 | 0.30 | m |
| Width of left side concrete base, Bl=D/2 to D | 0.10 | m |
| Width of right side concrete base, Br=10 to 15 cm | 0.15 | m |
| Total width of concrete base, B=C+Bl+Br | 2.45 | m |



| No. | Code of load | Load Formula | Load (KN) | Moment arm formula | Moment arm (m) | Moment (KNm) |
| --- | --- | --- | --- | --- | --- | --- |
| 1.0 | Self weight |  |  |  |  |  |
| 1.1 | W1 | gmasn.bH | 51.26 | b/2 | 0.25 | 12.82 |
| 1.2 | W2 | 0.5gmasn.(C-b)(H-h) | 76.95 | b+((C-b)/3) | 1.23 | 94.90 |
| 2.0 | Soil (vertical) |  |  |  |  |  |
| 2.1 | W3 | gmois.h(C-b) | 15.30 | b+((C-b)/2) | 1.35 | 20.66 |
| 2.2 | W4 | 0.5gmois.(C-b-Wms)(Hmois.-h) | 3.46 | b+(2/3)(C-b-Wms) | 1.11 | 3.85 |
| 2.3 | W5 | 0.5gmois.Wms(Hmois.-h) | 10.67 | (C-Wms)+(Wms/2) | 1.56 | 16.63 |
| 2.4 | W6 | 0.5gsatu.WmsHsatu. | 36.56 | (C-Wms)+((2/3)Wms) | 1.77 | 64.78 |
| 3.0 | Soil (horizontal) |  |  |  |  |  |
| 3.1 | PS1 | 0.5Kagmois.Hmois.2 | 6.08 | Hsatu.+(Hmois./3) | 3.32 | 20.21 |
| 3.2 | PS2 | 0.5Kagsub.Hsatu.2 | 13.79 | Hsatu./3 | 0.95 | 13.09 |
| 4.0 | Water (horizontal) |  |  |  | 0.95 |  |
| 4.1 | PW | 0.5gwat.Hsatu.2 | 39.74 | Hsatu./3 | 0.95 | 37.73 |
| 5.0 | Uplift (masn/conc) |  |  |  |  |  |
| 5.1 | PU1 | 0.15*0.5gwat.Hsatu.C | 4.61 | (2/3)C | 1.47 | 6.75 |

Table 3-1: Typical design of masonry retaining wall

### Downstream guide Wall

Downstream guide wall is designed monolithically with the conveyance system in order to reduce the area consumed by the structures and to avoid costly cut and slope protection works if the alignment is changed further up. The bank height is fixed in the same way as the upstream guide wall that is 2.5m taking in account peak flood level and freeboard.

The forces acting on the wall for worst condition is water and silt in the conveyance system, therefore structural stability analyses is shown below.

Unit weight of selected material=16KN/m3

Unit weight of masonry=25KN/m3

Angle of repose taken 30 degree

Density of masonry =25KN/m3

Density of concrete=23.5KN/m3

| No. | Code of load | Load Formula | Load (KN) | Moment arm formula | Moment arm (m) | Moment (KNm) |
| --- | --- | --- | --- | --- | --- | --- |
| 1.0 | Self weight |  |  |  |  |  |
| 1.1 | W1 | gmasn.bH | 35.25 | b/2 | 0.2 | 7.05 |
| 1.2 | W2 | 0.5gmasn.(C-b)(H-h) | 68.52 | b+((C-b)/3) | 1.13 | 77.65 |
| 2.0 | Soil (vertical) |  |  |  |  |  |
| 2.1 | W3 | gmois.h(C-b) | 16.20 | b+((C-b)/2) | 1.30 | 21.06 |
| 2.2 | W4 | 0.5gmois.(C-b-Wms)(Hmois.-h) | 4.29 | b+(2/3)(C-b-Wms) | 0.67 | 2.89 |
| 2.3 | W5 | 0.5gmois.Wms(Hmois.-h) | 9.05 | (C-Wms)+(Wms/2) | 1.51 | 13.63 |
| 2.4 | W6 | 0.5gsatu.WmsHsatu. | 34.01 | (C-Wms)+((2/3)Wms) | 1.74 | 59.07 |
| 3.0 | Soil (horizontal) |  |  |  |  |  |
| 3.1 | PS1 | 0.5Kagmois.Hmois.2 | 4.49 | Hsatu.+(Hmois./3) | 2.86 | 12.84 |
| 3.2 | PS2 | 0.5Kagsub.Hsatu.2 | 10.19 | Hsatu./3 | 0.82 | 8.31 |
| 4.0 | Water (horizontal) |  |  |  |  |  |
| 4.1 | PW1 | 0.5gwat.Hsatu.2 | 29.36 | Hsatu./3 | 0.82 | 23.96 |
| 4.2 | PW2 | 0.5gwat.fsd.2 | 3.97 | fsd./3 | 0.30 | 1.19 |
| 5.0 | Uplift (masn/conc) |  |  |  |  |  |
| 5.1 | PU1 | 0.15*0.5gwat.Hsatu.C | 3.96 | (2/3)C | 1.47 | 5.81 |

## Bill of Quantity and Cost Estimation

The quantities of the various items have been worked out as per the final design and final drawings prepared for the scheme. The unit rates analysis has been carried out based on the data available in the vicinity of the project area.

Bill No. 1- General Items(GOLINA INTAKE)

| It. No. | Description | Unit | Qty | Unit Rate | Total Amount |
| --- | --- | --- | --- | --- | --- |
| 1 | General |  |  |  |  |
| 1.1 | Allow for mobilaization | LS | 1 | 262,450.73 | 262,450.73 |
| 1.2 | allow for demobilaization | LS | 1 | 209,960.59 | 209,960.59 |
| 1.3 | Camp (Staff residence, office, Café, Kitchen, Store, Toilet & Shower, Guard House and Genentor house) | LS | 1 | 996,323.74 | 996,323.74 |
| 1.4 | allow for temporary accessed road to site | Km | 1.2 | 262,505.25 | 315,006.30 |
| 1.5 | Dewatering of open trenches and excavation ,pumps | LS | 1 | 62,000.42 | 62,000.42 |
| 1.6 | provide project indicator post starting from the construction time | LS | 1 | 7,680.23 | 7,680.23 |
| 1.7 | provision of as built drawings for the project | LS | 1 | 25,910.54 | 25,910.54 |
| Grand Total |  |  |  |  | 1,879,332.55 |

Bill No. 2- Head Work

| It. No. | Description | Unit | Qty | Unit Rate | Total Amount |
| --- | --- | --- | --- | --- | --- |
| 1 | Bed Bar |  |  |  |  |
| 1.1 | Excavtion (Ordinary soil) | m3 | 310.50 | 90.00 | 27,945.00 |
| 1.2 | Back fill selected material | m3 | 172.50 | 114.00 | 19,665.00 |
| 1.3 | Back fill & compacted with hard core | m3 | 18.69 | 394.72 | 7,376.33 |
| 1.4 | Concret C-20(1:2:3) For Bed Bar | m3 | 51.75 | 2,900.00 | 150,075.00 |
| 1.5 | 30cm Caping |  |  | - | - |
| 1.5.1 | Excavation (soil) | m3 | 224.25 | 90.00 | 20,182.50 |
| 1.5.2 | Back fill with selected material | m3 | 213.90 | 37.32 | 7,982.75 |
| 1.5.3 | concrete(C-20) | m3 | 51.75 | 2,900.00 | 150,075.00 |
| 1.5.4 | Ø12mm reinforcement bar | kg | 639.00 | 46.68 | 29,828.52 |
| 1.5.5 | Ø10mm reinforcement bar/stirrups | kg | 770.00 | 45.68 | 35,173.60 |
| 1.5.6 | Masonery/step | m3 | 5.75 | 1,850.00 | 10,637.50 |
| Sub Total |  |  |  |  | 458,941.20 |
| 2 | Right Side of the River Guide wall |  |  |  |  |
| 2.1 | U/S Guide wall Masonry (1:4) ratio) | m3 | 316.25 | 1,850.00 | 585,062.50 |
| 2.1.1 | C-15 Base Concrete | m3 | 34.5 | 2,600.00 | 89,700.00 |
| 2.1.2 | Hardcore | m3 | 57.5 | 482.28 | 27,731.10 |
| 2.1.3 | Excavtion (Ordinary soil) | m3 | 1762.95 | 90.00 | 158,665.50 |
| 2.1.4 | Back fill selected material | m3 | 140.3 | 114.00 | 15,994.20 |
| 2.2 | Plastring(1:3) | m2 | 539.2 | 149.35 | 80,529.52 |
| 2.3 | Gabion Guide Wall | m3 | 687.7 | 1,850.00 | 1,272,245.00 |
| 2.3.1 | Excavtion | m3 | 1745.7 | 90.00 | 157,113.00 |
| Sub Total |  |  |  |  | 2,387,040.82 |
| 3 | Trash rack |  |  |  |  |
| 3.1 | Ø24mm reinforcement bar | kg | 85.23 | 46.68 | 3,978.54 |
| 3.2 | Ø 100mm stiff iron | m | 3.45 | 500.00 | 1,725.00 |
| 3.3 | Trash rack concret cover C-20 | m2 | 0.7475 | 2,900.00 | 2,167.75 |
| Sub Total |  |  |  |  | 7,871.29 |
| 4 | Intake/ Aproch channel ,Under sluice &sluice &scaping channel |  |  |  |  |
| 4.1 | Excavation (soil) | m3 | 293.733 | 90.00 | 26,435.97 |
| 4.2 | Back fill with selected material | m3 | 20.5563 | 114.00 | 2,343.41 |
| 4.3 | Masonry for Approach channel | m3 | 154.8 | 1,850.00 | 286,380.00 |
| 4.4 | C-20 Concret for escape canal | m3 | 88.8 | 2,900.00 | 257,520.00 |
| 4.5 | Plastring(1:3)ratio | m2 | 192 | 168.49 | 32,350.08 |
| 4.6 | Lean concret C-15(1:2:4) for Approach channel) Bed | m3 | 60.48 | 2,600.00 | 157,248.00 |
| 4.5.1 | Cyclopean | m3 | 21 | 1,900.00 | 39,900.00 |
| 4.6 | Hardcore filling t=25cm | m3 | 84 | 482.28 | 40,511.52 |
| Sub Total |  |  |  |  | 842,688.98 |
| 5 | Gates |  |  |  |  |
| 5.1 | off take canal gate consists installation, the gate consists the followings parts | No | 1 | 18,620.00 | 18,620.00 |
|  | 6mm thickness sheet metal 2 .0mx 1.0m |  |  |  |  |
|  | 50x50x10mm angle iron 6.0m length |  |  |  |  |
|  | Ø12mm bar for anchorage.Angle iron with the masonry wall 2.0kg |  |  |  |  |
|  | Ø10mm bar for anchorage Angle Iron with the masonery wall 2.0kg |  |  |  |  |
| 5.2 | under sluice take canal gate consists installation, the gate consissts the following parts | NO | 1 | 18,620.00 | 18,620.00 |
|  | Handl70mm steel 1.5m length/spidle |  |  |  |  |
|  | 8mm thickness sheet metal ,1.2mx1m |  |  |  |  |
|  | 50x50x10mm angle iron 6.0m length |  |  |  |  |
|  | 12mm bar for anchorage ,2.5kg |  |  |  |  |
|  | 10mm bar for anchorage, Angle iron with the masonry wall 2.0kg |  |  |  |  |
|  | 60mmx60mm Angle iron stiffening gate head and handle 2.3m length |  |  |  |  |
|  | Handle 30mm steel 1m length |  |  |  |  |
| 5.3 | Scapping canal gate consist installation, the gate consists the followings parts | No | 1 | 18,620.00 | 18,620.00 |
|  | 6mm thickness sheet metal 1.2mx0.9m |  |  |  |  |
|  | 50x50x10mm angle iron 3.3m length |  |  |  |  |
|  | Ø12mm bar for anchorage ,2.5kg |  |  |  |  |
|  | 10mm bar for anchorage, Angle iron with the masonry wall 2.0kgØ |  |  |  |  |
|  | Handle 14mm steel 1.4m length |  |  |  |  |
| Sub Total |  |  |  |  | 55,860.00 |
| 6 | Operation slaps and Breast walls |  |  |  |  |
| 6.1 | C-20 Concrete | m3 | 1.2 | 2,900.00 | 3,480.00 |
| 6.2 | 12mmC/C 250mm | kg | 118 | 46.00 | 5,428.00 |
| Sub Total |  |  |  |  | 8,908.00 |
| 7 | Masonery stair/step | m3 | 6 | 1,850.00 | 11,100.00 |
| Sub Total |  |  |  |  | 11,100.00 |
| Grand Total For Headwork |  |  |  |  | 3,707,642.29 |

# SECTION-III: IRRIGATION AND DRAINAGE SYSTEMS INFRASTRUCTURE

IRRIGATION AND DRAINAGE SYSTEMS DESIGN

## Irrigable Area Description

### Topography

Topography is an important factor for the planning of any irrigation project as it influences method of irrigation, drainage, erosion, mechanization, and cost of land development, labour requirement and choice of crops.

The topographic feature of the project command area is mainly gentle slope. Its elevation range is from 1350 to 1375 meters above sea level. The slope gradient also ranges from gently sloping (3%) to strongly sloping (12%). However, it has identified to be suitable for surface irrigation. Nevertheless, it requires soil and water conservation measures or structures (i.e. constructing bunds, bio-physicals, check dams, artificial water ways, etc).

The project command area is situated at the right side of Golina River. The natural topographic feature of the command area has inclined from the North-East to the South-West direction.

### Climate

As per the hydrological analysis and on the basis of the traditional Ethiopian Agro-Ecological Zones (MOA, 2001), the UGDWIP area is basically classified as   Moist Kolla (sub-moist warm) agro-ecology which is comparatively conducive to the production of tropical and sub-tropical lowland crops. The elevation of the command area also ranges from 1366 to 1368masl.

The project area has unreliable and partially bimodal pattern of rainfall. The main rain season (locally known as Meher or Kiremt) occurs from Early July to near the mid of September, yielding average annual rainfall of 1,473 mm. About 35% of the annual rainfall occurs during the dry season from October to May (Belg). The dependable rainfall during the wet season of June to September varies from a minimum monthly value of less than 10 mm to a high value of 220 mm.

As the project site has no its own meteorological station, Lalibela (for rainfall and minimum and maximum temperature) and Kobo (for relative humidity, wind speed and sunshine hour) meteorological stations data were used for the project study as long as these stations are relatively near to the proposed command area. In general, the sources of meteorological data are the National Meteorology Service Agency (NMSA).

### Soil characteristics

Soil properties (physical, chemical, etc.) greatly influence the growth and thereby yield of crops which is grown. The command area has predominantly clay textured soils which can be classified as moderately drained soil.  Most of the study area soils are categorized as deep soil (1-1.5 meter depth). Soils of the command area are suitable for most of the selected crops to be grown (for further detail see the Agronomy Study of the same project.

### Existing Irrigation Practices in the Project Area

The pressure of survival and the need for additional food supplies to meet the demands of the increasing population is necessitating a rapid expansion of irrigation schemes. Thus, irrigation is becoming a basic part of well-developed agriculture wherever there is water and irrigable land potential. Accordingly, traditional and modern irrigation practices are under taken by individual farmers that use the river flow to both the left and the right side is with laborious temporary diversions. So, the farmers in the project area are very much interested in the idea of upgrading the traditional scheme to modern scheme.

## Irrigation Water Requirement

### Crop Water Requirement (CWR)

The calculation of crop water requirement is a very important aspect for planning of any irrigation project. Several methods and procedures are available for this. The Food and Agriculture Organization (FAO) of the United Nations has also made available several publications on this subject and other issues related with this. The computer program available in FAO Irrigation and Drainage Paper No. 56 “CROPWAT” has been used for the calculation of Crop Water requirement. This program is based on Penman-Monteith approach and procedures for calculation of crop water requirements and irrigation requirements are mainly based on methodologies presented in FAO Irrigation and Drainage Paper No. 24 “Crop Water Requirements” and No. 33 “Yield Response to Water”.

The corresponding values of the crop water requirements of the proposed crops of the project are presented in the Agronomy Study of the same project.

### Irrigation efficiency (Ep)

To complete the evaluation of the demand, the efficiency of the water distribution system and of application must be known.

The gross requirement of water for irrigation system is very much dependent on the overall efficiency of the irrigation system, which in turn is dependent on several factors: Method of irrigation, type of canal (Lined and/or Unlined), method of operations (simultaneously and continuous or Rotational water supply), and availability of structures (for controlling and distribution and measuring and monitoring).

On the basis of these factors, the project has planned to impose surface irrigation method (using furrows). The canal system is unlined other than canal near the head work. Hence, the conveyance efficiency has been estimated to be 80% and field application efficiency 60%. As a result of these the overall irrigation efficiency has been estimated to be 48%. According to soil Lab result, soils of the command area are predominantly characterized as clayey soils.

### Irrigation duty

Irrigation duty is the volume of water required per hectare for the full flange of the crops. Moreover, it helps in designing an efficient irrigation canal system. The area, which will be irrigated, can be calculated by knowing the total available water at the source and the overall duty for all crops required to be irrigated in different seasons of the years.

The proposed cropping pattern of See irrigation project has showed maximum net irrigation water requirement (NIWR) of 4.7 mm/day for dry season in the month of March and 3.8 mm/day for wet season in the month of September for all crops. However, for the designing of the irrigation water application and for the flows in the entire canal system, of the proposed or selected crops the one that has maximum NIWR has been taken (maize). Therefore, maize has showed the maximum NIWR which is 6.55 mm/day in the month of April and taken for the irrigation project duty calculation for 24 irrigation hours (see the calculation below).

e = ec * ea = 60*80 =48%

100        100

Where, e = Scheme irrigation efficiency (%)

ec = Conveyance efficiency (%)

ea = Field application efficiency (%)

For the designing of the project, the GIWR is given as follows:

GIWR = NIWR/e = 6.55/0.48 = 13.65 [mm/day] for full irrigation

GIWR = NIWR/e = 4.6/0.48 = 9.6 [mm/day] for supplementary irrigation

The GIWR (13.65 and 9.6 mm/day) represents the daily quantity of water that is required to be applied for dry and wet season crops, respectively. This water quantity is also used for the determination of the canal discharge in consideration of the time of flow and also defined as the duty, expressed as l/s/ha.

The duty is calculated by:

Duty (D) = GIWR × 1000 × 10 / (t × 60×60)

Where; Duty – the duty [l/s/ha]

GIWR – Gross Irrigation Requirement [mm/day]

t – Daily irrigation or flow hours [hrs]

The duty for GIWR of 13.65 mm/day (dry season) and 9.6 mm/day (wet season) for daily irrigation time (t = 20 hrs for dry and 14 hrs for wet seasons) is supported to be used with surface irrigation method. Thus, duty for 20 and 14 irrigation hours of full and supplementary irrigations, respectively, has computed as follows:

D = (13.65 x 10 x 1000) / (20 x 3600) = 2.1  l/s/h (for full irrigation during dry season)

D = (9.6 x 10 x 1000) / (14 x 3600) = 1.9 l/s/h (for supplementary irrigation during wet season)

### Irrigation methods

Slope and the changes in slope of the irrigated area, especially undulating or variable multidirectional sloped fields, have a major influence over selection of the method of irrigation. Among the different irrigation systems perennial irrigation system will be used for the project area. The irrigation water will be obtained from Golina River water source by diverting and carrying the water through main, secondary and tertiary canals and then leading to field canals and finally turning to irrigation furrows.

Among the various irrigation methods surface irrigation method has selected for this project. Thus, furrow, border and basin irrigation methods can be used to supply irrigation water to the selected crops. However, each method has its own advantages and disadvantages. Care should be taken when choosing the method which is best suited to the local circumstances, i.e., depending on slopes, soil types, selected crops type, amount of water available, etc. of the command area.

Based on the above factors surface irrigation method has proposed for the proposed crops in this project. The method allows applying light irrigation and can be used on a variety of fields (sloping fields) along the contour. Furrow irrigation method is best suited for most of the proposed and row planted crops. In general, furrow irrigation method is simple, manageable and widely practiced irrigation method. This method is suitable for row crops that cannot stand in water for long periods. The only thing required to use this method is row planting of crops. Besides, basin and border irrigation method would be used for the non-row planted crops. Rotational flow water distribution is also recommended for the project area.

.

## Irrigation and Drainage System Layout

The irrigation system layout for the project is prepared taking the following points into consideration besides other factors.

- A primary concern in the layout of the system is that it serves the purpose of conveying and distributing water to the command area.

- The excavation and earth fill volumes not be excessive, otherwise the construction costs can be tremendous.

- The selection of longitudinal bed slope is made taking into account the existing slopes of the terrain, so as to minimize deviations in canal routing.

- Curves in canals should not be too sharp.

The proposed irrigation system layout comprises one main canal, two secondary canals and sixteen tertiary canals as shown on the layout Drawings. The main canal runs for most of its length parallel to the contours and several changes of direction are necessary to follow the topography. It crosses one small gullies, one foot path. The main canal is masonry lined for a length of 00 to 2000meters starting from the intake outlet to make maintenance easier since this part of the canal may be subjected to flooding during high flood flows.

### Conveyance System

The conveyance system consists of one Main canal to irrigate total command area of 170ha. The main canal starts from Water abstraction site on right side and conveys water for a length of 2.7 Km.

Main canal is aligned along contours and supplies to two secondary unit and sixteen tertiary canals.

## Design of the Canal System

Flow Depth and Section Capacity

The earthen canals have been designed with a trapezoidal shape and the lined ones with rectangular x-section using Manning's Formula:

Where Q= discharge (m3/s)

R= Hydraulic radius (Flow area/wetted perimeter)

S= Hydraulic gradient

n= Manning's roughness coefficient, n=0.024 is adopted for the earth channels and n=0.018 for the masonry lined part of the main canal

### Main Canal

The main canal is designed for a discharge of 340l/s and depending on the site specific condition, appropriate slope is provided. Hydraulic parameters of the main canal are shown below.

Table 0-1: Hydraulic Parameters of main canal

B= Canal bottom width                         QR= Required discharge

FSD= Full supply depth                             QD= Designed Discharge

FB= Free board

SS= Side slope

V= Velocity

### Secondary Canals

Table 0-2: Hydraulic Parameters of secondary canals

Figure 0-1: Typical Cross Section of secondary canals

### Tertiary canals

In the layout system there are sixteen tertiary canals, the designed discharge is determined based on the duty of irrigation.  The sections of the canals are determined by using manning’s formula, and they are trapezoidal section. The hydraulic characteristics is Presented in Table 5.3 below.

Figure 0-2: Typical Cross Section of tertiary canals

Table 0-3: Hydraulic Parameters of tertiary canals

`

### Field Canals

As shown in the layout, field canals run across the contours and hence face relatively steeper gradient. The discharge of most of the field canals is very small and this is taken as an advantage to cope up with the relatively steeper gradient. Figure 15 below shows a typical field canal x-section. As much as possible field canals shall be made in fill in order to easily irrigate the adjacent command area. As can be seen from the layout, majority of the filed canals can be used to irrigate both sides of the command area depending on the condition of the individual plots of land owned by individual farmers.

Figure 0-3: Typical Field Canal X-section

## Drainage Canals

### Catch drainage canals

In the layout system there are one Catch drain canals, the designed discharge is determined based on the Catchment area, run off coefficient and rain fall intensity (Rational formula).  The sections of the canals are determined by using manning’s formula, and they are trapezoidal section.

Figure 0-4: Typical Cross Section of Catch drain canals

### Secondary canal drainage canals

Drainage Canals aren't provided for the secondary canals, because all of the secondary canals are entered to the river and also there is shortage of irrigation water. It is assumed that there is no excess irrigation water during dry season. For the rainy season the water that comes through the secondary canals directly entered to river. So, protection is necessary at the secondary canal out fall.

## Canal Structures Design

### Design of a typical flume

Hydraulic Characteristics of the canal

Length of the flume: 22m

Shape of the flume:  Circular (Pipe), Roughness coefficient, n =0.014

From the canal longitudinal profiles, u/s canal bed level (CBL) = 1367.236m

D/s Canal bed level (CBL) =1367.286m

U/s full supply level (FSL) = 1368.036m

D/s Full Supply Level = 1368.086m

Total head loss between the inlet & Outlet =0.2m

Table 0-4: Hydraulic Parameters of Flume

| Flume on | start Chainage | Q | V | L | h | t |
| --- | --- | --- | --- | --- | --- | --- |
| Flume 1 | 940 | 0.34 | 0.67 | 22 | 0.8 | 0.5 |

### Design of Division box

At different points of the main and secondary canals division boxes are provided which divert the flow to the secondary canal and tertiary canals. Gate should be provided at the outlet of the boxes. For detail refer the drawing.

Figure 0-5: Typical Division Box plan

Using broad crested formula,

Q= CL (h)3/2

Where; Q= discharge over rectangular weir/intake (opening), m3/s

C = discharge coefficient, c= 1.7

L= effective length of crest form in m

h= over flow depth, m

Assuming equal discharge coefficient & sill height for two or three dividing canals, the proportion becomes.

Q1/ Q2= Q2/ Q3 =L1/ L2 = L2/ L3

Where Q1= is flow in canal 1

Q2 = is flow in canal 2

Q3 = is flow in canal 3

L1= is effective crest length of weir/intake sill across opening to canal 1

L2= is effective crest length of weir/intake sill across opening to canal 2

L3 = is effective crest length of weir/intake sill across opening to canal 3

Q1= CL1 (h)3/2,

L1 = Q1/Ch3/2

L2 = L1*Q2/ Q1

L3 = L1*Q3/ Q1

The depth of (height of) the division box,

D = d + fb

The width of the division box,

B = b + 2*m*D

Where b= base width of the incoming canal

D = total canal depth of the incoming canal

Table 0-5: Hydraulic parameters of Division Boxes

### Road crossing structure

Two road crossing structures are provided on the main canal, at the existing foot paths. The road crossing structures are rectangular reinforced concrete slab. The slab is reinforced with 12mm @150mmc/c the length of the slab is 1.2m which is the same as the respective canal bed width, its width and thickness is 3m and 15mm respectively.

| It. No. | Description | Unit | Qty | Unit Rate | Total Amount |
| --- | --- | --- | --- | --- | --- |
|  | B. Irrigation Infrastructure |  |  |  |  |
| 1 | MAIN CANAL |  |  |  |  |
| 1.1 | Clearing and grubbing | m2 | 6,210.00 | 10.00 | 62,100.00 |
| 1.2 | Ordinary earth excavation | m3 | 3,999.15 | 90.00 | 359,923.32 |
| 1.3 | Selected material Back fill and compaction | m3 | 239.52 | 114.00 | 27,305.51 |
| 1.4 | Main Canal Masonry Wall works | m3 | 1,632.00 | 1,720.00 | 2,807,040.00 |
| 1.5 | Main Canal Cyclopean Bed works | m3 | 576.00 | 1,795.00 | 1,033,920.00 |
| 1.6 | Plastering 1:3ratio | m2 | 6,000.00 | 122.20 | 733,200.00 |
| 1.7 | Drainage canal excavation | m3 | 6,000.00 | 90.00 | 540,000.00 |
| Sub Total |  |  |  |  | 5,563,488.83 |
| 2 | SECONDARY CANAL |  |  |  |  |
| 2.1 | Earth work & compaction |  |  |  |  |
| 2.2 | Clearing and grubbing | m2 | 2,760.00 | 10.00 | 27,600.00 |
| 2.3 | Ordinary earth excavation | m3 | 804.63 | 90.00 | 72,416.88 |
| 2.4 | Stone pitching (1:4) 50m each turn frommain canal | m3 | 112.80 | 1,400.00 | 157,920.00 |
| 2.5 | Selected material Back fill and compaction | m3 | 15.89 | 114.00 | 1,811.23 |
| Sub Total |  |  |  |  | 259,748.11 |
| 3 | TERTIARY CANALS TC-1 |  |  |  |  |
| 3.1 | Earth work & compaction |  |  |  | - |
| 3.2 | Clearing and grubbing | m2 | 282.44 | 10.00 | 2,824.40 |
| 3.3 | Ordinary earth excavation | m3 | 574.76 | 90.00 | 51,728.27 |
| 3.4 | Selected material Back fill and compaction | m3 | 2.19 | 114.00 | 249.09 |
| Sub Total |  |  |  |  | 54,801.76 |
| 4 | TERTIARY CANALS TC-2 |  |  |  |  |
| 4.1 | Earth work & compaction |  |  |  | - |
| 4.2 | Clearing and grubbing | m2 | 280.60 | 10.00 | 2,806.00 |
| 4.3 | Ordinary earth excavation | m3 | 349.07 | 52.00 | 18,151.69 |
| 4.4 | Selected material Back fill and compaction | m3 | 2.65 | 114.00 | 301.53 |
| Sub Total |  |  |  |  | 21,259.22 |
| 4 | TERTIARY CANALS TC-3 |  |  |  |  |
| 4.1 | Earth work & compaction |  |  |  |  |
| 4.2 | Clearing and grubbing | m2 | 280.60 | 10.00 | 2,806.00 |
| 4.3 | Ordinary earth excavation | m3 | 349.07 | 52.00 | 18,151.69 |
| 4.4 | Selected material Back fill and compaction | m3 | 2.65 | 114.00 | 301.53 |
| Sub Total |  |  |  |  | 21,259.22 |
| 5 | TERTIARY CANALS TC-4 |  |  |  |  |
| 5.1 | Earth work & compaction |  |  |  |  |
| 5.2 | Clearing and grubbing | m2 | 281.52 | 10.00 | 2,815.20 |
| 5.3 | Ordinary earth excavation | m3 | 233.89 | 90.00 | 21,049.83 |
| 5.4 | Selected material Back fill and compaction | m3 | 8.49 | 114.00 | 967.52 |
| Sub Total |  |  |  |  | 24,832.55 |
| 6 | TERTIARY CANALS TC-5 |  |  |  |  |
| 6.1 | Earth work & compaction |  |  |  |  |
| 6.2 | Clearing and grubbing | m2 | 294.40 | 10.00 | 2,944.00 |
| 6.3 | Ordinary earth excavation | m3 | 817.85 | 90.00 | 73,606.10 |
| 6.4 | Selected material Back fill and compaction | m3 | 14.16 | 114.00 | 1,613.84 |
| Sub Total |  |  |  |  | 78,163.94 |
| 7 | TERTIARY CANALS TC-6 |  |  |  |  |
| 7.1 | Earth work & compaction |  |  |  |  |
| 7.2 | Clearing and grubbing | m2 | 607.20 | 10.00 | 6,072.00 |
| 7.3 | Ordinary earth excavation | m3 | 910.31 | 90.00 | 81,927.50 |
| 7.4 | Selected material Back fill and compaction | m3 | 454.46 | 114.00 | 51,808.10 |
| Sub Total |  |  |  |  | 139,807.59 |
| 8 | TERTIARY CANALS TC-7 |  |  |  |  |
| 8.1 | Earth work & compaction |  |  |  |  |
| 8.2 | Clearing and grubbing | m2 | 602.60 | 10.00 | 6,026.00 |
| 8.3 | Ordinary earth excavation | m3 | 192.14 | 90.00 | 17,292.78 |
| 8.4 | Selected material Back fill and compaction | m3 | 102.27 | 114.00 | 11,658.72 |
| Sub Total |  |  |  |  | 34,977.50 |
| 9 | TERTIARY CANALS TC-8 |  |  |  |  |
| 9.1 | Earth work & compaction |  |  |  |  |
| 9.2 | Clearing and grubbing | m2 | 487.60 | 10.00 | 4,876.00 |
| 9.3 | Ordinary earth excavation | m3 | 224.27 | 90.00 | 20,184.57 |
| 9.4 | Selected material Back fill and compaction | m3 | 22.77 | 114.00 | 2,595.78 |
| Sub Total |  |  |  |  | 27,656.35 |
| 10 | TERTIARY CANALSTC-9 |  |  |  |  |
| 10.1 | Earth work & compaction |  |  |  |  |
| 10.2 | Clearing and grubing | m2 | 471.50 | 10.00 | 4,715.00 |
| 10.3 | Ordinary earth excavation | m3 | 391.74 | 90.00 | 35,256.24 |
| 10.4 | Selected material Back fill and compaction | m3 | 8.92 | 114.00 | 1,017.34 |
| Sub Total |  |  |  |  | 40,988.58 |
| 11 | TERTIARY CANALS TC1-10 |  |  |  |  |
| 11.1 | Earth work & compaction |  |  |  |  |
| 11.2 | Clearing and grubing | m2 | 471.50 | 10.00 | 4,715.00 |
| 11.3 | Ordinary earth excavation | m3 | 397.01 | 90.00 | 35,731.31 |
| 11.4 | Selected material Back fill and compaction | m3 | 1.23 | 114.00 | 140.28 |
| Sub Total |  |  |  |  | 40,586.58 |
| 12 | TERTIARY CANALSTC-11 |  |  |  |  |
| 12.1 | Earth work & compaction |  |  |  |  |
| 12.2 | Clearing and grubbing | m2 | 471.50 | 10.00 | 4,715.00 |
| 12.3 | Ordinary earth excavation | m3 | 89.83 | 90.00 | 8,084.39 |
| 12.4 | Selected material Back fill and compaction | m3 | 148.64 | 114.00 | 16,944.68 |
| Sub Total |  |  |  |  | 29,744.06 |
| 13 | TERTIARY CANALSTC-12 |  |  |  |  |
| 13.1 | Earth work & compaction |  |  |  |  |
| 13.2 | Clearing and grubbing | m2 | 410.00 | 10.00 | 4,100.00 |
| 13.3 | Ordinary earth excavation | m3 | 59.34 | 90.00 | 5,340.60 |
| 13.4 | Selected material Back fill and compaction | m3 | 171.08 | 114.00 | 19,503.12 |
| Sub Total |  |  |  |  | 28,943.72 |
| 14 | TERTIARY CANALS TC-13 |  |  |  |  |
| 14.1 | Earth work & compaction |  |  |  |  |
| 14.2 | Clearing and grubbing | m2 | 390.00 | 10.00 | 3,900.00 |
| 14.3 | Ordinary earth excavation | m3 | 3.55 | 90.00 | 319.50 |
| 14.4 | Selected material Back fill and compaction | m3 | 575.20 | 114.00 | 65,572.80 |
| Sub Total |  |  |  |  | 69,792.30 |
| 15 | TERTIARY CANALS TC-14 |  |  |  |  |
| 15.1 | Earth work & compaction |  |  |  |  |
| 15.2 | Clearing and grubbing | m2 | 223.00 | 10.00 | 2,230.00 |
| 15.3 | Ordinary earth excavation | m3 | 21.97 | 90.00 | 1,977.30 |
| 15.4 | Selected material Back fill and compaction | m3 | 88.51 | 114.00 | 10,090.14 |
| Sub Total |  |  |  |  | 14,297.44 |
| 16 | TERTIARY CANALS TC-15 |  |  |  |  |
| 16.1 | Earth work & compaction |  |  |  |  |
| 16.2 | Clearing and grubbing | m2 | 247.00 | 10.00 | 2,470.00 |
| 16.3 | Ordinary earth excavation | m3 | 62.96 | 90.00 | 5,666.40 |
| 16.4 | Selected material Back fill and compaction | m3 | 25.56 | 114.00 | 2,913.84 |
| Sub Total |  |  |  |  | 11,050.24 |
| 17 | TERTIARY CANALS TC-16 |  |  |  |  |
| 17.1 | Earth work & compaction |  |  |  |  |
| 17.2 | Clearing and grubbing | m2 | 247.00 | 10.00 | 2,470.00 |
| 17.3 | Ordinary earth excavation | m3 | 74.04 | 90.00 | 6,663.60 |
| 17.4 | Selected materialBack fill and compaction | m3 | 5.61 | 114.00 | 639.54 |
| Sub Total |  |  |  |  | 9,773.14 |
| 18 | TERTIARY CANALS TC-17 |  |  |  |  |
| 18.1 | Clearing and grubbing | m2 | 246.00 | 10.00 | 2,460.00 |
| 18.2 | Ordinary earth excavation | m3 | 28.62 | 90.00 | 2,575.80 |
| 18.3 | Selected material Back fill and compaction | m3 | 173.14 | 114.00 | 19,737.96 |
| Sub Total |  |  |  |  | 24,773.76 |
| 19 | DIVISION BOX |  |  |  |  |
| 19.1 | Stone work |  |  |  |  |
| 19.2 | Masonry work | m3 | 91.20 | 1,720.00 | 156,864.00 |
| 19.3 | Cemented stone pitching | m3 | 43.32 | 1,400.00 | 60,648.00 |
| 19.4 | Plastering | m2 | 31.44 | 114.00 | 3,584.16 |
| Sub Total |  |  |  |  | 221,096.16 |
| 20 | Gate supply,cutting ,welding and installation |  |  |  |  |
| 20.1 | 2mm thick sheet metal | No. | 4.00 | 6,000.00 | 24,000.00 |
| 20.2 | Stiffening angle iron (30*30*4) |  |  |  |  |
| 20.3 | Agle iron for groove(40*40*4) |  |  |  |  |
| 20.4 | Ø 12mm & 0.6 m of bar for handling |  |  |  |  |
| Sub Total |  |  |  |  | 24,000.00 |
| 21 | Turn out |  |  |  |  |
| 21.1 | Stone work |  |  |  |  |
| 21.2 | Masonry work | m3 | 103.68 | 1,720.00 | 178,329.60 |
| 21.3 | Cemented stone pitching | m3 | 504.47 | 1,400.00 | 706,255.20 |
| 21.4 | Plastering | m2 | 504.00 | 122.00 | 61,488.00 |
| Sub Total |  |  |  |  | 946,072.80 |
| 22 | Gate supply,cutting ,welding and installation |  |  |  |  |
| 22.1 | 2mm thick sheet metal | No. | 4.00 | 7,600.00 | 30,400.00 |
| 22.2 | Stiffening angle iron (30*30*4) |  |  |  |  |
| 22.3 | Agle iron for groove(40*40*4) |  |  |  |  |
| 22.4 | Ø 12mm & 0.6 m of bar for handling |  |  |  |  |
| Sub Total |  |  |  |  | 30,400.00 |
| 23 | Flume@ chain age 00+930 |  |  |  |  |
| 23.1 | Excavation |  |  |  |  |
| 23.1.1 | Ordinary earth excavation | m3 | 46.20 | 90.00 | 4,158.00 |
| 23.2 | Concrete work |  |  | - | - |
| 23.2.1 | C-20 concrete work | m3 | 15.84 | 2,664.00 | 42,197.76 |
| 23.2.2 | stone work |  |  | - | - |
| 23.2.3 | Masonry work | m3 | 0.72 | 1,720.00 | 1,238.40 |
| 23.2.4 | Plastering | m2 | 5.52 | 122.00 | 673.44 |
| 23.2.5 | Reinforcement ø12 | kg | 320.00 | 46.68 | 14,937.60 |
| Sub Total |  |  |  |  | 63,205.20 |
| 24 | Foot path and Cattle crossing |  |  |  |  |
| 24.1 | Excavation |  |  |  |  |
| 24.2 | Ordinary earth excavation | m3 | 2.16 | 90.00 | 194.40 |
| 24.3 | Concrete work |  |  | - | - |
| 24.4 | C-25 concrete work | m3 | 0.65 | 3,000.00 | 1,944.00 |
| 24.5 | stone work | m2 | 2.12 | 1,200.00 | 2,544.00 |
| 24.6 | Masonry work | m3 | 0.36 | 1,720.00 | 619.20 |
| 24.7 | stone pitching | m3 | 1.08 | 1,400.00 | 1,512.00 |
| 24.8 | Plastering | m2 | 5.46 | 122.00 | 666.12 |
| 24.9 | Reinforcementø12 | kg | 37.56 | 46.68 | 1,753.30 |
| Sub Total |  |  |  |  | 9,233.02 |
| 25 | Foot path and Cattle crossing | No. | 3.00 | - |  |
| Sub Total |  |  |  |  | - |
| Grand Total For Infra |  |  |  |  | 7,789,952.07 |

## Irrigation Infrastructure Bill of Quantities and Cost Estimate

SUMMARY

| Bill No. | Descrption | Amount (Birr) | Remark |
| --- | --- | --- | --- |
| 1 | General item | 1,879,332.55 |  |
| 2 | Head Work | 3,707,642.29 |  |
| 3 | Main, Secondary, Tertiary Canal and Catch Drain | 7,789,952.07 |  |
| Total |  | 13,376,926.90 |  |
| VAT 15% |  | - |  |
| Grand Total |  | 13,376,926.90 |  |

# CONCLUSION AND RECOMMENDATION

- The infrastructure of this project area is designed to irrigate about 170 ha of land by taking its supply from the Golina intake irrigation project. The maximum duty of the command area for 24 hours per day irrigation with overall project efficiency of 45%.  The method of irrigation of the project area is furrow surface irrigation in which the main and secondary canals are working continuously whereas the field canals within a tertiary block are working rotational system.

- As the dominant soil type is clay soil, the main canal system is designed to be masonry.

- The reason why the main canal is to be lined up to the end is to avoid the siltation problem, time saving to reach at the tail part, reduce maintenance cost.

- The design of the canal dimensions of the irrigation canal is done by applying the manning’s uniform flow equation. The variable of the hydraulic parameters are calculated using iteration or flow master program.

- The design discharge of the drainage canals are determined using rational formula and Gamble Powell method.

- As soils of the command area are predominantly black cotton clay textured; and hence water and soil management measures should be undertaken; and optimum moisture content should be maintained to improve workability of the soil during land preparation and planting time.

The following recommendations are drown:

- For better performance and long service year of the project regular inspection and maintenance is highly required.

- Farmers training, how to operate and maintain the project structures as a whole and available and water resources has a paramount important.

- The irrigation hours per day and per week should be flexible based on base flow amount of each week or month.

- Close supervision of the construction should be made to modify (if need be) each Components of irrigation system based on specific site conditions.

# OPERATION AND MAINTENANCE

## General

The main objective of the operation and maintenance aspect of an irrigation scheme is to facilitate the timely delivery of the required irrigation water to farms and to keep the irrigation system in an optimum operating condition.  This section therefore, discusses the main functions of the subject matter under consideration for the scheme.

## Operation of the Head Works

Operation at the diversion weir mainly focuses on the diversion of a controlled flow of river water, timely cleaning of floating debris in front of intake and removal of sediment deposits in front of the weir and intake structures.

## Irrigation System Operation

The operation of the irrigation system depends mainly on the method of water delivery at farm level.  Surface irrigation method is the recommended type of water distribution and application method for golina iintake irrigation scheme.

The farmers would organize themselves and form groups in order to handle the water management.  Since flow is low Rotational water distribution would be applied within the group.  The rotational distribution is then to distribute water by turn to the whole scheme according to the timely need of crop water requirement. For better and efficient water management, crop diversification should be avoided within a group.  This would reduce the complexity of water distribution system of the scheme during one irrigation season. At farmers’ level of operation, a constant flow and variable irrigation time is advisable.

The operation of the irrigation system is continuous for 24 hours per day in main, secondary and tertiary canals whereas field canals within a tertiary block are operating in rotational system with each other for irrigation hours proportional to their size.  Since the tertiary canal discharges are within the manageable range and the irrigation canal structures are accordingly designed for simple operation, the farmers can open and close easily whenever they required.

## Maintenance Requirement

The canal system of the project is earthen canal except main canal and secondary canal, which is susceptible to siltation, erosion, growth of weeds and or breaching problems.  Though the canal sections are designed for non- silting and non-scouring conditions, the above mentioned problems are unavoidable and hence periodical and yearly inspection and maintenance of canals and structures are obligatory in order to fulfill the design objective of the project area.

The maintenance tasks are categorized into two types: - routine activities, and repairs. The routine maintenance activities that are carried out periodically include:-

- Regular cleaning of sediments and weeds from canals and drains;

- Inspection and lubrication of gates; and

- Maintenance of cracked lined canals, regulating and control structures.

Repair works include task carried out more frequently and quickly, and include those tasks that are generally unpredictable.  They also include emergency works.  The activities included in this category are:-

- Repairing overtopped or breached canals, drains, and flood protection dykes;

- Repairing jammed gates;

- Filling holes made by wild animals; and

- Reduced free board due to walking over by people and livestock.

Regular inspection of the irrigation facilities should be carried out as part of the maintenance activities.  These tasks could be carried out immediately after the end of the main rains in September and during the rainy season.  This could concentrate on the interceptor drains and the flood protection dykes, the main canal and the field drains.  The inspection of the other works like the tertiary canals, field drains, and the water control and regulating structures could be carried out as part of routine operation activities.

- Beneficiaries of the project need to have operation and maintenance budget, For O&M cost incurring entity area:

- Purchase sing of gate lubricate (grease)

- Replacing and maintenance of Stolen and damaged gates

- Repair Damages on the cross drainage structures.

The expense for O&M should be collected from the beneficiaries. Of course, much of the task is done by the labor and skill of the community. For cost incurring activities beneficiaries have to collect money based on the proportion of the command area they owned.

# REFERENCE

- FAO (1977) guidelines for predicting crop water requirements. No 24, Rome Italy

- Design of small Canal structures , USBR

- Soft copies of hydraulic structure publishing

- IDD manual

- ESRDF manual