Amhara National Regional State

Water Resources Development Bureau

(BOWRD)

Detail Design

Of

Gobu-ІІІ Spate Irrigation Project

Volume IV: Engineering Design Final Report

September, 2014(Revised 2015)

Bahir Dar

Client: Bureau of Water Resource Development

(BoWRD)

Address:

P. O. Box: 88
Telephone: 0528-200853/855
Fax: 251-08-20-65-68/204676/202040

Consultant: Amhara Design & Supervision Works Enterprise

(ADSWE)

Address:

P. O. Box: 1921
Telephone: +251-582-181023/ 180638/181201/181254
Fax: (058) 2180550/ (058) 2180560
E-mail: amhara design@yahoo.com

Bahir Dar, Ethiopia

# FEASIBILITY STUDY & DETAIL DESIGN REPORT STRUCTURE

- Volume I: Watershed Management

- Volume II: Engineering Geology

- Volume III: Irrigation Agronomy

- Volume IV: Engineering Design

- Volume V: Socio Economy

- Volume VI: Environmental Impact Assessment

Table of Contents								 Page Nr

FEASIBILITY STUDY & DETAIL DESIGN REPORT STRUCTURE	i

LIST OF TABLES	v

LIST OF FIGURES	vi

SAILENT FEATURE	vii

1	INTRODUCTION	1

1.1	Background	1

1.1.1	Description of the Project Area	1

1.2	Objectives of the Study	3

1.2.1	Major Objective	3

1.2.2	Specific Objectives	3

1.3	Scope of the Study	4

1.4	Methodology	5

SECTION-I:  HYDROLOGY	7

2	HYDROLOGY	9

2.1	Watershed Characteristics	9

2.2	Hydro-Metrological Data Availability	10

2.2.1	Climate	10

2.2.2	River flow data	10

2.2.3	Upstream & Downstream utilization	11

2.3	Design Flood Analysis	11

2.3.1	Design Rainfall computation	11

2.3.2	Outlier Test	11

2.3.3	Check for variance	12

2.3.4	Tail Water Depth Computation	18

SECTION-II: HEADWORK DESIGN	22

3	HEADWORK STRUCTURES DESIGN	24

3.1	Headwork Site Selection	24

3.1.1	River Bed condition	24

3.1.2	River Bank condition	25

3.2	Sources of construction materials	27

3.2.1	Rock for Masonry and Crushed Coarse Aggregate	27

3.2.2	Fine Aggregates	28

3.2.3	Water	28

3.3	Headwork Type Selection	28

3.3.1	Hydraulic Design of Headwork Structure	29

3.3.2	Outlet sill Level Determination	29

3.3.3	River Bed Protection	30

3.4	DESIGN OF SCOURING SLUICE AND HEAD REGULATOR	31

3.4.1	Scour depth	31

3.4.2	Head Regulator	31

3.4.3	Canal dimensions	32

3.4.4	Design of operation slab	35

3.4.5	Escaping canal & side spillway	35

3.4.6	Side spillway	36

3.5	DESIGN OF WINGWALL	37

3.5.1	Upstream & Downstream Wing wall	37

3.6	Bill of Quantity and Cost Estimation	39

SECTION-III: IRRIGATION AND DRAINAGE SYSTEMS INFRASTRUCTURE	44

4	IRRIGATION AND DRAINAGE SYSTEMS DESIGN	46

4.1	Irrigable Area Description	46

4.1.1	Topography	46

4.1.2	Climate	46

4.1.3	Soil characteristics	47

4.1.4	Existing Irrigation Practices in the Project Area	47

4.2	Irrigation Water Requirement	47

4.2.1	Crop Water Requirement (CWR)	47

4.2.2	Irrigation efficiency (Ep)	48

4.2.3	Irrigation duty	48

4.2.4	Irrigation methods	49

4.3	Irrigation and Drainage System Layout	49

4.3.1	Conveyance System	50

4.4	Design of the Canal System	50

4.4.1	Main Canal	50

4.4.2	Secondary Canals	51

4.4.3	Tertiary canals	52

4.4.4	Field Canals	53

4.5	Drainage Canals	54

4.5.1	Catch drainage canals	54

4.5.2	Tertiary and Field canal drainage canals	54

4.5.3	Design of Division box	54

4.5.4	Road crossing structure	57

4.6	Irrigation Infrastructure Bill of Quantities and Cost Estimate	58

5	CONCLUSION AND RECOMMENDATION	66

6	OPERATION AND MAINTENANCE	66

6.1	General	66

6.2	Operation of the Head Works	67

6.3	Irrigation System Operation	67

6.4	Maintenance Requirement	67

REFERENCE	68

# LIST OF TABLES

Table 2-1: Outlier test analysis	11

Table 2-2: Test for goodness to fit using D-index	13

Table 2-3: Determination of Time of Concentration	14

Table 2-4: Runoff analysis	16

Table 2-5: Hydrograph coordinates	17

Table 2-6: Intake Site River Cross section Coordinate Data	18

Table 2-7: Stage discharge analysis	19

Table 4-1: Hydraulic Parameters of main canal	51

Table 4-2: Hydraulic Parameters of secondary canals	51

Table 4-3: Hydraulic Parameters of tertiary canals	52

Table 4-6: Hydraulic parameters of Division Boxes	57

# LIST OF FIGURES

Figure 1-1: Location map of the project area	2

Figure 2-2: Complex Hydrograph	18

Figure 2-3: Rating Curve	19

Figure 2-4: River profile	20

Figure 3-1: River bed at the proposed intake site	25

Figure 4-1: Typical Cross Section of secondary canals	52

Figure 4-2: Typical Cross Section of tertiary canals	52

Figure 4-3: Typical Field Canal X-section	53

Figure 4-4: Typical Cross Section of Catch drain canals	54

Figure 4-5: Typical Division Box plan	55

# SAILENT FEATURE

- Project name: Gobu- ІІІ Irrigation Project

- Name of the stream: Gobu River

- Location of the weir/intake site using

- East: 567301.58

- North: 1357970.67

- Zone: Nohth Wolo

- kebele: Jarota

- Average Altitude:  1454.9 m. a.s.l

- Hydrology

- Design rainfall: 106.76 mm

- Catchment area: 293.85 Km2

- Longest flow path length: 39.04 Km

- Design flood: 698.05 m3/sec

- Intake Stucture

- Intake type: Intake open masonry, with cyclopean bed bar structure

- Depth : 3 m below the river bed level

- Intake crest level:1454.95 m.a.s.l

- U/S HFL: 1457.45 m.a.s.l

- D/s HFL: 1457.45 m.a.s.l

- Under sluice

- Sill level: 1454.25m.a.s.l

- Dimension: 1.5*1.4m2

- Discharge amount: 2*2.43m3/sec

- Outlet

- Sill level:1454.45 m.a.s.l

- Opening dimension: 1.5*1.2 m2

- Discharge capacity: 2.81 m3/sec.

Irrigation and drainage systems Infrastructure

- Command area size: 250 ha.

- Type of soil of the command area is dominantly clay soil.

- Design discharge of the main canal = 2.81m3/s

- Irrigation system layout consists of one earthen & lined main canal, 3 secondary canals and 21 tertiary canals

- Main irrigation structures designed are;

- Foot path

- Division box and Turn outs

Project cost

|  | Summery Gubu 3 |  |
| --- | --- | --- |
| Bill No. | Description | Amount (Birr) |
| 1 | General Items | 2,091,399.92 |
| 2 | Head work (Weir body, Outlet structures) | 9,908,868.80 |
| 3 | Infrastructures | 14,684,368.25 |
| Grand Total |  | 26,684,636.96 |

# INTRODUCTION

## Background

In Ethiopia, under the prevalent rain-fed agricultural production system, the progressive degradation of the natural resource base, especially in highly vulnerable areas of the highlands coupled with climate variability have aggravated the incidence of poverty and food insecurity. The major source of growth for Ethiopia is still conceived to be the agriculture sector. Hence, this sector has to be insulated from drought shocks through enhanced utilization of the water resource potential of the country, (through development of small-scale irrigation, water harvesting, and on-farm diversification) coupled with strengthened linkages between agriculture and industry (agro-industry), thereby creating a demand for agricultural output. In line with the above, efforts have been made by the government to improve the situation in the country in areas of domestic water supply provision, irrigation, watershed management, etc. The Amhara Water Resources Development Bureau is playing its role in the development of small scale irrigation projects in the region. Accordingly, as part of the water sector development program, the office has initiated the study and design of a small scale irrigation scheme on Gobu  at Jarota Kebele and signed an agreement with Amhara Design & Supervision Works Enterprise (ADSWE) for the study and design of the project.

### Description of the Project Area

#### Location

This irrigation project is located mainly at Jarota Kebele, Kobo Wereda of North Wolo Zone in the Amhara Region. The proposed irrigation project is to be undertaken on Gobu river and the headwork structures are specifically located at an altitude of about 1454 masl and geographical coordinates of 567301.581N (UTM) and 1357970.677 E (UTM).

Figure 1-1: Location map of the project area

#### Accessibility

The project headwork site is accessed through asphalt rode from Kobo to Waja at about 25km, turning to the right and by passing 15 km through all-weather gravel road

#### Previous Irrigation Practices

Farmers are highly exercised traditional irrigation practices. They diverted Gobu River traditionally and irrigate a considerable hectare of land. However, this traditional practice has it's own limitations. Some of the limitations area temporary diversion structures are eroded during flood season. The level of activity is minimum. Non-manageable and traditional Way of water utilization may bring about exaggerate of resource.

Wastage of time for seasonal construction of diversion structure by farmers every year after flood. Hence, taking in to account the above limitations, the Study aimed at

- Efficient utilization of the resources

- To minimize land degradation due to poor drainage system of the area.

- To promote easy & manageable irrigation operation system for the area.

- To mitigate problems of food shortage through irrigation development.

- To increase agricultural products through irrigation in dry season.

## Objectives of the Study

### Major Objective

The project area faces variability of rainfall distribution though the overall rainfall generally suffices the rain-fed agriculture. Accordingly, the rain-fed agriculture needs means of supplementing during distribution failures and further full irrigation is required to maximize the use of the potential land and water resources.

Hence the objective of this project is to contribute a substantial share in the effort to reduce the risk of production decrease due to rainfall variability and increase the productivity of the resource in the project specific area. Specifically, the project is targeted for the following.

- To make sustainable the rain-fed crop production and make extra production in the dry season possible for 300 ha of land through spate irrigation.

- There is a general consensus that irrigation investments will achieve broader food security and poverty reduction impacts and if efforts are also geared towards up-grading existing traditional farming practices with support to enhance access to input supply, output marketing and extension to facilitate access to information and innovations.

- This objective is to be realized by constructing intake structures across the Gobu river and diverting the stream flood to the command by using spate irrigation method.

### Specific Objectives

Other benefits that can be expected to appear with the launching of the project are:

- Efficiency of water use improvement;

- Improved local nutrition/food security gains;

- Improved management of scarce natural resources (land and water);

- Resilience against drought risk;

- Rationale for erosion control and watershed management;

- Rationale for the intensification and modernization of small-holder agriculture and rural lifestyles.

The engineering study and design enables the realization of the project by the provision of engineering structures that will allow the appropriate abstraction of the Gobu water for delivery in to the identified irrigation fields of the study area. Hence, this engineering design is specifically targeted to:

- Analyze hydrologic requirements of the project and engineering structures;

- The formulation of sound and stable structure,  with necessary provisions that allow safe, easy and low-maintenance operation in the service life of the project;

- Develop working drawings;

- Estimation of construction costs.

## Scope of the Study

- The irrigation design shall ensure reliability, equity and flexibility of water delivery to farmers. It will aim at reducing conflicts among water users and will lead to lower operation and maintenance costs.

- Updating the existing, if available, computation of the actual evapo-transpiration, crop water requirement, irrigation demand/duty using the existing and recent agronomic, climatologic and soil data using more appropriate methodologies.

- Establish design criteria for irrigations structures to be approved by the client and to be used in the final design stage,

- Design proper irrigation system compatible with local conditions and management capabilities,

- Establish flood protection measures for the command area and canal structures and design the respective drainage system accordingly,

- Planning  and layout of the irrigation system, which include irrigation canals, drainage channels, inspection roads and alignments, canal spacing, canal length, location of structures, and water profiles along canal and drains at specified reaches, which is most economical easily manageable and aligned with topographic feature and geological investigation.

- Determination and estimation of water application conveyance and other losses and irrigation efficiencies and consideration of those parameters in design steps.

- Check and test hydraulic and structural designs of main canal considering total demand and the required capacity and the base flow availability,

- Prepare general plans and drawings for all irrigation infrastructure and irrigation systems designs,

## Methodology

In the study and design procedure, Designers used the following steps.

- Specific Site identification:

  - Review of the reconnaissance survey conducted by the Client

  - 50,000 scale top map and GIS information

  - Local farmers interview and discussion

  - Wereda and Zone Agriculture section expertise

  - Previous studies

  - On foot travel along the river channel and farm areas.

- Topographic survey:

  - Surveying the headwork site and the Command area with sufficient radius, using Total station

- Flow estimation

  - Physical observation on flood mark indications and local information about high flood and critical flow condition of the river

  - Analyzing the recorded river flow data and use watershed inputs for further analysis.

  - Base flow estimated during the reconnaissance field visit by floating method.

- Irrigable area identification:

  - Using local information

  - 50,000 Topographic map, and GIS information, GPS to see elevation

The design report is organized in three sections. In Section I the Hydrology study is presented and in Sections II and III the Headwork and Irrigation and Drainage Systems designs are discussed respectively. In Section III, planning and design of the irrigation system after diverting the water using the intake will be dealt. The following are major areas of concern in this part.

- Study and design of the irrigation method to be adopted,

- Study and design of the irrigation system layout and associated structures,

- Design of the different conveyance canals,

- Planning and design of the different irrigation and drainage structures,

- Preparation of the longitudinal profiles of the different irrigation and drainage canals.

# SECTION-I:  HYDROLOGY

# HYDROLOGY

## Watershed Characteristics

The Watershed has marked topographic variation. All types of slopes are present. The dominant slope class is 15-30% which covers 42.62% of the total area followed by 8-15% which is 29.1%. Flat and almost Flat and Gentle Slopping accounts 1.92 and 12.58 percent respectively, very steep slopes also cover about 12.95% of the total area. Table 1 of the watershed feasibility study report shows the slope classes and proportion of the watershed.

Certain physical properties of watersheds significantly affect the characteristics of the runoff and sediment yield and are of great interest in hydrologic analyses. The rate and volume of runoff, and sediment yield from the watershed have much to do with shape, size, slope and other parameters of the landscape. These suggest that there should be some important relations between basin form and hydrologic performance. If the basin and hydrologic characteristics are to be related, the basin form must also be represented by quantitative descriptors. These parameters can be measured from maps.

The watershed characteristics are analyzed and presented in Table 2 of the Watershed Feasibility Study Report of the same project. In summary:

- Catchment Area  = 293.85km2

- Stream Length = 39.04Km

- CN(II) = 85.088

(Extracted from the Watershed Study Report of the same project)

Gobu River at the headwork site is characterized by undefined channel system and no flow in dry season. It looks that the gradient of the river is getting low and hence there exists significant deposition of sediment mainly silt, cobble and gravels.

## Hydro-Metrological Data Availability

### Climate

Small scale irrigation project designers and planners are faced with lack of good data on the hydrology of the stream/river system that will be their water source and on local weather and climate conditions. Stream gauging stations are virtually non-existent in remote rural areas of Ethiopia; meteorological stations are almost rare. Likewise, Jarota Kebele (Project area location) and in the catchment area of this project, there is no meteorological station of any level. Moreover, there are no flow data for the river/stream near the project. Therefore, data for the hydro-meteorological analysis is taken from the nearby station and similar areas.

The nearby meteorological stations around the project area are at Lalibela and Kobo station is assumed to represent metrological parameters required for the estimates of extreme flood discharges for specific return periods are needed to design Bed bar and canal intakes structures. Rainfall data from Lalibela had to be used because in the absence of data from the Gobo catchment itself (there is no reliable rainfall station in this catchment).

Each of station has their own radius and influential area towards the watershed of the river. Designers have computed design rainfall of each meteorological stations based on the availability and nature data recorded in each station. Finally, the design Rainfall data from Lalibela had to be used by the flood analysis on rainfall data, which are better in both quantity and quality of data.

Rainfall Data

The proposed project area has unimodal rainfall pattern though unreliable belg rainfall exists in the highland areas of the woreda. The main rainy season is summer (Meher or Kiremt), which extends from July to August. The mean annual rainfall that the project area received is 590mm.

Generally, the rainfall of the project area is characterized by its high variability in both amount and distribution. Thus, the main bottleneck for successful crop production in the area is the nature of erratic and very uneven distribution of rainfall. Meher or Kiremt rainfall is largely received in the two months, July and August. Had it been well distributed throughout the growing season the amount of rainfall may have been sufficient for the growth of crops.

### River flow data

There is no gauging station around Gobu River. For ungauged catchments, the methodology used to determine water availability in the project area is analyzing the nearby hydrological gauging stations, water balance of the project area based on inflow and outflow characteristics and hydrological analysis and field observations.

In spate, irrigation is entirely different from that on perennial flows, which rise very quickly and then recede over a period of hours or days. Spate systems need to be able to divert short duration flood flows into gravity canal systems to ensure that sufficient amounts are abstracted in the time available to meet water demands and that the canals systems ensure command over the fields to be irrigate.

### Upstream & Downstream utilization

The main objective of the improvement of irrigation work is the equality of water shares between upstream and downstream user’s water rights. Above this spate irrigation scheme at 15 km distance Gobu-1 and at 10 km distance Gobu-2 schemes are constructed with a capacity of extracting 2m^3/sec. and 3m^3/sec.flood water out of 698.05m^3/sec. maximum flood of the river. While any irrigation structure or for any purpose water is diverted from the natural water way, allowance for downstream users as well as for the upstream should be considered. The proposed project use only 2.81m3/sec so there is enough amount of flood discharge is release for downstream users out of 698.05 m3/s.

## Design Flood Analysis

For the design and analysis of structures to be constructed on the river, estimation of flood magnitude is an important task. This can be done using different techniques depending on the data available. For this particular case, there are no river flow data and hence the flood estimation is done using the rainfall data and applying SCS Curve Method.

### Design Rainfall computation

Based on the data of 24hr peak rainfall given in Table 1 the design rainfall, Rf is computed using Gumble’s Extreme Value Method.

### Outlier Test

Higher Limit, YH=Ymean+Kn*Sy,      Kn = 2.549 for 29 Years of data.

Lower Limit, YH=Ymean-Kn*Sy,       Kn = 2.549 for 29 Years of data.

Table 2-1: Outlier test analysis

| Sr.Nr | Year | Heaviest rainfall [mm/day] | Descending order | Rank | Y=logX | Y=logX |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 1978 | 57.5 | 92.00 | 1 | 1.963787827 | 1.963787827 |
| 2 | 1979 | 83.5 | 83.50 | 2 | 1.921686475 | 1.921686475 |
| 3 | 1980 | 33.5 | 82.50 | 3 | 1.916453949 | 1.916453949 |
| 4 | 1981 | 70.5 | 74.00 | 4 | 1.86923172 | 1.86923172 |
| 5 | 1982 | 92 | 73.10 | 5 | 1.863917377 | 1.863917377 |
| 6 | 1983 | 48.5 | 70.50 | 6 | 1.848189117 | 1.848189117 |
| 7 | 1984 | 38.35 | 68.20 | 7 | 1.833784375 | 1.833784375 |
| 8 | 1985 | 28.2 | 66.00 | 8 | 1.819543936 | 1.819543936 |
| 9 | 1986 | 38.5 | 61.80 | 9 | 1.790988475 | 1.790988475 |
| 10 | 1987 | 36 | 61.60 | 10 | 1.789580712 | 1.789580712 |
| 11 | 1988 | 45.9 | 57.50 | 11 | 1.759667845 | 1.759667845 |
| 12 | 1989 | 40.6 | 52.90 | 12 | 1.723455672 | 1.723455672 |
| 13 | 1990 | 43.25 | 51.10 | 13 | 1.7084209 | 1.7084209 |
| 14 | 1991 | 51.1 | 48.50 | 14 | 1.685741739 | 1.685741739 |
| 15 | 1992 | 36.2 | 45.90 | 15 | 1.661812686 | 1.661812686 |
| 16 | 1993 | 66 | 43.25 | 16 | 1.635986112 | 1.635986112 |
| 17 | 1994 | 82.5 | 40.60 | 17 | 1.608526034 | 1.608526034 |
| 18 | 1995 | 36.5 | 38.60 | 18 | 1.586587305 | 1.586587305 |
| 19 | 1996 | 68.2 | 38.50 | 19 | 1.58546073 | 1.58546073 |
| 20 | 1997 | 51.1 | 38.35 | 20 | 1.583765368 | 1.583765368 |
| 21 | 1998 | 38.6 | 36.50 | 21 | 1.562292864 | 1.562292864 |
| 22 | 1999 | 34.6 | 36.00 | 22 | 1.556302501 | 1.556302501 |
| 23 | 2000 | 52.9 | 36.20 | 23 | 1.558708571 | 1.558708571 |
| 24 | 2001 | 73.1 | 36.00 | 24 | 1.556302501 | 1.556302501 |
| 25 | 2002 | 61.6 | 36.00 | 25 | 1.556302501 | 1.556302501 |
| 26 | 2003 | 61.8 | 34.60 | 26 | 1.539076099 | 1.539076099 |
| 27 | 2004 | 29.2 | 33.50 | 27 | 1.525044807 | 1.525044807 |
| 28 | 2005 | 36 | 29.20 | 28 | 1.465382851 | 1.465382851 |
| 29 | 2006 | 74 | 28.20 | 29 | 1.450249108 | 1.450249108 |
| Sum |  |  | 1494.600 |  | 48.92625015 | . |
| Mean |  | Xm | 51.538 | Ym | 1.687112074 | 1.687112074 |
| Standard Deviation(dn-1) |  | (dn-1)x | 18.185 | (dn-1)y | 0.148723808 | 0.148723808 |
| Skew ness coefficient(Cs) |  |  |  |  | 0.276807911 | 0.276807911 |
| No of data(N) = 29 |  |  |  |  | 29 |  |

### Check for variance

After checking the outliers, the data should be checked for variability. For variability the formula used is

α=δ∩-1N*Mean*100%

Where, δn-1 = Standard deviation =18.18

N = Nr of recorded data =29

Mean = 51.53

α = Standard error

α=18.1829*51.53*100%=6.5<10%  Acceptable

Therefore the data shows no variability.

#### D-Index test

After checking the consistency of the data for higher and lower outlier, the 29 years data is obtained as representative for the analysis using D-index. The D-Index test is believed to be the better goodness to fitness in many literatures. Hence in this study it was used to determine the best statistical distribution to estimate the peak rainfall. The D-index for the comparison of the fit of various distributions is summarized as follows.

D-Index=(1Xm)*i=16Abs(Xi-Xi')

Where Xi and Xi’ are the ith highest observed and computed values for the distribution respectively

Table 2-2: Test for goodness to fit using D-index

| Rank | XI | Normal | Log person type III | Log Normal | person Type III | Gumbel EVI |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | XI -'XI' | XI -'XI' | XI -'XI' | XI -'XI' | XI -'XI' |
| 1 | 92 | 7.105 | 2.537 | 0.816 | 5.187 | 0.661 |
| 2 | 83.5 | 4.659 | 0.630 | 2.141 | 3.682 | 2.235 |
|  | 82.5 | 7.654 | 6.371 | 7.037 | 7.187 | 7.238 |
| 4 | 74 | 2.262 | 2.655 | 2.826 | 2.134 | 3.079 |
| 5 | 73.1 | 3.970 | 5.488 | 5.339 | 4.087 | 5.614 |
| 6 | 70.5 | 3.660 | 5.966 | 5.598 | 3.962 | 5.879 |
| Sum |  | 29.309 | 23.647 | 23.757 | 26.239 | 24.706 |
| Sum/Mean |  | 0.569 | 0.459 | 0.461 | 0.509 | 0.479 |
| Point Rain fall |  | 88.894 | 103.3 | 98.315 | 91.542 | 106.76 |

The rain fall that, estimated by Gumbel EVI distribution method is taken since it has a minimum D-Indix value and it is maximum point rain fall.

#### General

The River is not gauged river. The design flood is calculated by using SCS unit hydrograph method. Thus, it is preferred to base the flood analysis on rainfall data, which are better both in quantity and quality of data. In the hydrologic analysis for drainage structures, it must be recognized that there are many variable factors that affect floods. Some of the factors that need be recognized and considered on an individual site by site basis are; rainfall amount and storm distribution; catchment area, shape and orientation; ground cover; type of soil; slopes of terrain and stream(S); antecedent moisture condition; Storage potential (over bank, ponds, wetlands, reservoirs, channel, etc.)

#### Peak flood analysis by SCS unit hydrograph method

Design flood is calculated SCS (The United States Soil Conservation Service). This method is widely adopted and more reliable method for flood estimation. The approach considers, watershed parameters, like Area, Curve number, and time of concentration.

#### Time of concentration (Tc)

Time of concentration has been calculated by taking the stream profile of the longest streamline and dividing it in to different elevation. Kirpich formula is adopted for computation.

Table 2-3: Determination of Time of Concentration

| Partial length(km) | Cumulative | Elevation(m) | Change of elevation | TC_hr |
| --- | --- | --- | --- | --- |
| 0 | 0 | 3400 | 0 | 0.00 |
| 5.886 | 5.886 | 2460 | 940 | 0.53 |
| 11.658 | 17.544 | 1880 | 580 | 1.40 |
| 8.44 | 25.984 | 1640 | 240 | 1.35 |
| 13.056 | 39.04 | 1457 | 183 | 2.48 |
| 39.04 |  |  |  | 5.75 |

The formula is,

Tc=0.948L13H10.385+L23H20.385+…+Ln3Hn0.385

- Tc = 5.75	Since Tc > 3hr., duration of excess rainfall difference, D = 1hr.

- Time to peak,

Tp=D2+0.6*Tc  = 3.95 hr

- Base time,

Tb=2.67*Tp  = 10.55 hr

- Recession time,

Tr=1.67*Tp  = 6.6 hr.

#### Curve number (CN)

Curve number (CN) is achieved based on USSCS method by watershed characterization in terms of land cover, treatment, hydrologic condition and soil group. From the watershed analysis curve number at condition II =85.08. Since peak rainfall is found at an antecedent moisture condition III state, this value has to be changed to antecedent moisture condition III.

- Conversion factor = 1.12

- CN Condition (III) = (Factor from Table x CN condition II) =88.05*1.120 = 95.3. For detail analysis of the computation, Refer Excel file, attached here with.

#### Area Rainfall

As the area of the catchment gets larger, coincidence of all hydrological incidences becomes less and less. This can be optimized by changing the calculated point rainfall to aerial rainfall. The conversion factor is taken from standard table that relate directly with the size of watershed area and type of the gauging station. (IDD manual)

For the case of Gobu-3 irrigation project,

- Total watershed area = 293.85 Km2

- Type of gauging station = Daily rainfall (24 hr.)

- Aerial Rainfall = (Point Rainfall) x (Conversion factor)

#### Run off Analysis

Input data:

Design Point Rainfall = 106.76mm

Curve number at antecedent moisture condition III = 95.3

Catchment Area, A = 293.85 Km2

Tc = 5.57hr, D = 1hr., Tp = 3.95 hr; Tb = 10.55 hr; Tr = 6.6 hr.

Direct run-off,

- Q=(I-0.2*S)2(I+0.8*S)

- Where, I = Rearranged cumulative run-off depth (mm

S = Maximum run off potential difference,

- S=25400CN-254

Peak run-off for incremental;

- Qp=0.21*(A*Q)Tp

- Where,   A = Catchment area = 293.85 Km2

- Tp = Time to peak (hr)

- Q = Incremental run-off (mm)

Table 2-4: Runoff analysis

| Time increment | 0-1 | 1-2 | 2-3 | 3-4 | 4-5 | 5-6 |
| --- | --- | --- | --- | --- | --- | --- |
| RF. Profile, (%) | 42.00% | 58.00% | 66.00% | 72.00% | 75.00% | 79.00% |
| Area to point ratio, (%) | 52.36% | 63.36% | 68.00% | 72.36% | 74.00% | 76.72% |
| RF. Profile, (mm) | 29.86 | 49.91 | 60.95 | 70.75 | 75.37 | 82.31 |
| Incremental RF, (mm) | 29.9 | 20.0 | 11.0 | 9.8 | 4.6 | 6.9 |
| Descending order | 1.0 | 2.0 | 3.0 | 4.0 | 6.0 | 5.0 |
| Rearranged order | 5.0 | 4.0 | 3.0 | 1.0 | 2.0 | 6.0 |
| RF in rearranged incr. | 6.9 | 9.8 | 11.0 | 29.9 | 20.0 | 4.6 |
| Cumulative RF. I, (mm) | 6.9 | 16.7 | 27.8 | 57.6 | 77.7 | 82.3 |
| Cumulative runoff, (mm) | 0.0 | 2.2 | 7.9 | 30.3 | 47.7 | 51.9 |
| Incremental runoff, (mm) | 0.0 | 2.2 | 5.7 | 22.4 | 17.4 | 4.1 |
| Peak run-off for incr.(qp ) | 0.0 | 34.6 | 89.3 | 349.7 | 271.8 | 64.8 |
| Time to begin , (hr) | 0.00 | 1.00 | 2.00 | 3.00 | 4.00 | 5.00 |
| Time to peak | 3.95 | 4.95 | 5.95 | 6.95 | 7.95 | 8.95 |
| Time to end. | 10.55 | 11.55 | 12.55 | 13.55 | 14.55 | 15.55 |

Table 2-5: Hydrograph coordinates

| Time | H1 | H2 | H3 | H4 | H5 | H6 | Base flow | SUM | Remark |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0.00 | 0.0 |  |  |  |  |  | 0 | 0.0 |  |
| 1.00 | 0.0 | 0.0 |  |  |  |  | 0 | 0.0 |  |
| 2.00 | 0.0 | 8.8 | 0.0 |  |  |  | 0 | 8.8 |  |
| 3.00 | 0.0 | 17.5 | 22.6 | 0.0 |  |  | 0 | 40.1 |  |
| 3.95 | 0.0 | 25.8 | 44.1 | 84.1 | 0.0 |  | 0 | 154.0 |  |
| 4.00 | 0.0 | 26.3 | 45.2 | 88.5 | 0.0 |  | 0 | 160.0 |  |
| 4.95 | 0.0 | 34.6 | 66.7 | 172.6 | 65.3 | 0.0 | 0 | 339.2 |  |
| 5.95 | 0.0 | 29.4 | 89.3 | 261.1 | 134.1 | 15.6 | 0 | 529.4 |  |
| 6.95 | 0.0 | 24.1 | 75.8 | 349.7 | 202.9 | 32.0 | 0 | 684.5 |  |
| 7.95 |  | 18.9 | 62.2 | 296.8 | 271.8 | 48.4 | 0 | 698.05 | peak |
| 8.95 |  | 13.6 | 48.7 | 243.8 | 230.6 | 64.8 | 0 | 601.6 |  |
| 10.55 |  | 5.2 | 27.1 | 159.0 | 164.7 | 49.1 | 0 | 405.1 |  |
| 11.00 |  | 2.9 | 21.0 | 135.1 | 146.2 | 44.7 | 0 | 349.9 |  |
| 11.55 |  | 0.0 | 13.5 | 106.0 | 123.6 | 39.3 | 0 | 282.4 |  |
| 12.00 |  |  | 7.4 | 82.1 | 105.0 | 34.9 | 0 | 229.5 |  |
| 12.55 |  |  | 0.0 | 53.0 | 82.4 | 29.5 | 0 | 164.8 |  |
| 13.00 |  |  |  | 29.1 | 63.8 | 25.0 | 0 | 118.0 |  |
| 13.55 |  |  |  | 0.0 | 41.2 | 19.6 | 0 | 60.8 |  |
| 14.00 |  |  |  |  | 22.7 | 15.2 | 0 | 37.9 |  |
| 14.55 |  |  |  |  | 0.0 | 9.8 | 0 | 9.8 |  |
| 15.00 |  |  |  |  |  | 5.4 | 0 | 5.4 |  |
| 15.55 |  |  |  |  |  | 0.0 | 0 | 0.0 |  |

698.05

Figure 2-1: Complex Hydrograph

- From the analysis, the 50 year return period design run off is 698.05m 3/s

### Tail Water Depth Computation

Tail water depth of the river is equal to the flood depth and amount at the proposed intake site before construction of the intake. It is used to crosscheck peak flood estimated by the SCS unit hydrograph method with flood mark method and to see the flood feature after the hydraulic jump. During field visit, the flood mark of the river at the proposed intake site was marked based on dwellers information and physical indicative marks. The river cross-section was surveyed.

Table 2-6: Intake Site River Cross section Coordinate Data

| ` | Northing | Easting | Elevation | Partial distance | cumulative distance | Code |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 1358018.912 | 567024.441 | 1456.3 | 0.00 | 0.00 | AX |
| 2 | 1358039.117 | 567013.223 | 1455.28 | 23.11 | 23.11 | AX |
| 3 | 1358054.052 | 567003.985 | 1455.03 | 17.56 | 40.67 | AX |
| 4 | 1358077.332 | 566991.888 | 1455 | 26.24 | 66.91 | AX |
| 5 | 1358096.439 | 566981.33 | 1454.95 | 21.83 | 88.74 | AX |
| 6 | 1358130.438 | 566963.48 | 1455.01 | 38.40 | 127.14 | AX |
| 8 | 1358141.639 | 566956.662 | 1455.34 | 13.11 | 140.25 | AX |
| 9 | 1358154.597 | 566949.183 | 1457.99 | 14.96 | 155.21 | AX |
| 10 | 1358164.041 | 566945.224 | 1458.11 | 10.24 | 165.45 | AX |

Table 2-7: Stage discharge analysis

| Elevation | Depth (m) | Area(m2) | Perimeter(m) | Hydraulic radius(m) | Velocity (m2/sec) | Discharge (m3/sec) | Remarks |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1454.95 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |  |
| 1455.45 | 0.50 | 45.60 | 246.70 | 0.18 | 0.76 | 34.86 |  |
| 1456.05 | 1.10 | 128.70 | 285.50 | 0.45 | 1.39 | 178.30 |  |
| 1456.45 | 1.50 | 182.70 | 297.00 | 0.62 | 1.70 | 311.42 |  |
| 1457.05 | 2.10 | 269.80 | 308.40 | 0.87 | 2.16 | 581.66 | Peak |
| 1457.45 | 2.50 | 337.20 | 318.40 | 1.06 | 2.45 | 825.79 |  |
| 1457.95 | 3.00 | 420.50 | 328.30 | 1.28 | 2.78 | 1169.04 |  |

Figure 2-2: Rating Curve

From the above stage discharge table and curve the maximum flood level corresponding to the computed design peak discharge is 698.05 m3/s (2.2m from the river bed) and it is considered as the d/s high flood level i.e. expected at the intake axis before construction of the intake.

D/S and U/S HFL = 1457.45 masl.

- Average river bed slope

Average river bed slope of River is estimated by two different techniques. One is by end area method and the other is by using best fit line method. Designers have adopted the end area and best fit method used for further analysis.

The water level of the river is taken at different points along the river channel around the head work site. Surveying work done for 600m length. And then, average water surface slope is considered as the river bed slope. For comparison of the two procedures, refer the attached Excel file.

Figure 2-3: River profile

- Manning’s Roughness coefficient

The Manning’s roughness coefficient is taken from standard table based on the river nature. The river at the headwork site has got braded feature and curving nature. The river banks are defined and relatively smooth. Manning’s roughness coefficient (n = 0.04) is adopted.

- Discharge of the river

Input data:

- Manning's roughness coefficient, n = 0.04

- Average river bed slope, S = 0.004

- ,  	Where, R = Hydraulic radius = (Area/Perimeter)

- Q=V*A

# SECTION-II: HEADWORK DESIGN

# HEADWORK STRUCTURES DESIGN

## Headwork Site Selection

The headwork site is situated at 1357970.67 m N, 567301.58 m E and river bed elevation of 1454.93 m above sea level. For this project a simple river bed level spate structure has been proposed for diverting the water to the main canal. The headwork site geological surface and subsurface conditions have been investigated based on the nature of the proposed structure. At the site and immediate vicinity, the stream flows along gentle slope course and its bed is mainly covered with recently deposited alluvial sediments. These sediments are dominated with sand (specifically thick layer of fine sand materials). On the other hand, the banks of the stream at the headwork site are made up of different geologic materials; most part of the right bank is made up of old alluvial sediment of silt/clay materials at the top and thin layer of fine sand to the bottom of both banks.

River Geomorphology

The river channel has undefined channel development and does not have stable abutments and fixed width rather liable to change time to time. Alluvial deposits fill the river channels so that the courses might be changed within the interval of flush floods time.

Since the riverbanks are not clearly defined and low in bearing capacity, change of channels are likely to happen during flood seasons unless some retaining and guiding structures constructed.

### River Bed condition

As it could be observed in the geological cross-section, thick unconsolidated and coarse alluvial deposits cover the foundations of the diversion axis. The axis lay on the wider sections of the river courses where the riverbanks and beds are loose unstable alluvial deposits. At these sites the river courses are pre-matured, not well defined and have no fixed width. Since the riverbanks are not clearly identifiable and low in bearing capacity, change of channels are likely to happen during flood seasons unless some retaining and guiding structures constructed. As it could be possible to observe from surface and subsurface investigations, sound formations are not encountered.

The head work site is fixed at this point, because there is no better site along the river course and due attention is given for mitigation measures.

Figure 3-1: River bed at the proposed intake site

### River Bank condition

#### Right Bank

At the headwork site/axis, the right bank is characterized by relatively moderate to genteel slope, having about 0.8m height from stream bed. It reveals nearly genteel section within this height.  From visual observation of the natural cuts at the bank it is made from Low Plastic dark brown color fine grained silt (ML) throughout its channel.

Since this bank has made from loss, week soil materials, have been affected by erosion/ flood cutting, which is widening the bank by forming nearly vertical slope. Such vertical slope configuration throughout the stream bank, which lead to instability or collapse. Since Main canal rout has been launched from this loss erodible and week old flood plain deposited, it is very important to design the bank slope to stable configuration, just by providing appropriate slope (indifferent from the present vertical slope) or other protection work(Gabion) to prevent  initial main canal portion near to the bank from farther widening of bank.

Fig. The right Bank

#### Left Bank

At the proposed headwork axis and immediate vicinities, the left bank forms relatively steep slope topography. From surface observation, the bank is made up of one basic geological units. The unit is old flood plain deposit of brown to dark color fine grained low plastic silt (ML) soil. For geotechnical purpose, this unit are weak and loose horizon. The height of this bank varies from place to place. At the headwork site it might reach up 1.5m.Hence due to erodible, weak and loss nature of the bank, the river morphology had changed from time to time. In order to make our structure functional for long time, it is necessary to provide some bank protection structures to prevent from farther change in flow channel of the stream.

Fig. Left Bank

## Sources of construction materials

During site investigation, natural construction materials required for the construction of the various proposed engineering structures at the headwork and within the farmland have been assessed, and possible quarry sites and borrow areas have been identified within the vicinity of the study area as close to the project site as possible. In addition to identifying the quality, quantity and accessibility conditions of the construction materials, ownerships of each proposed production sites have also been studied and described in this report, on separate sub-sections below. The materials needed for the construction of the structures include rock for masonry stones, aggregates (both coarse and fine), and water.

### Rock for Masonry and Crushed Coarse Aggregate

During the site investigation, natural construction materials required for the construction of the various proposed engineering structures at the headwork and within the farmland have been assessed, and possible quarry sites and borrow areas have been identified within the vicinity of the area as much as possible. In addition to the identification, the quality, quantity, accessibility condition and ownership of each proposed production sites have also been studied and described in this report; on separate sub-sections below. The natural materials required for the construction of the proposed hydraulic structures include rock for masonry stones, aggregates (both coarse and fine), impervious soil for fill and/or lining, backfill soil, and water.

### Fine Aggregates

Borrow areas for fine aggregate or natural sand have been assessed starting from the project stream itself. Natural deposits of such materials couldn’t be found when assessed within the beds of the stream in the project area; rather very fine sand deposit are found covering almost the entire bed of the Gobu stream. Seeing to this nature of the stream, other distant streams have been explored to identify the best source areas for fine aggregate or natural sand that can be used for this particular project. During exploration of this natural sand, at a distant one stream was identified as a possible source of sand. The stream is known as ‘Horemat’. It is located at about 21km from the project site.

Both visual investigation and laboratory testing of the sand deposit found in the proposed area have been done. In the area good quantity, which suffices the need of this project, is available. The deposit is dominated by sand sized transported alluvial sediments, though some amount of fines and over sizes are also observed. The individual grains of sand are mainly originated from basalt rock and reveals high crushing resistant that the sand has good quality in this respect.

### Water

Water for construction purposes can NOT be found from the project stream Gobu, itself in winter. The stream is seasonal that if there is rain in the surrounding area, there will be run off in the stream. Unless otherwise, there will not have any drop of water throughout the year. During this field time the stream was dry throughout its flowing channel. The designer of this project put an option to use the underground well water for construction during dry flow there exist well water with in 10km radius since there is sub surface irrigation projects in the area.

## Headwork Type Selection

Bottom Intake (Tyrolean intake) has been selected .The water to be diverted is taken in through a collection canal built in to the river bottom and covered with a screen.   The bars of the screen are laid in the direction of the flow so that coarse bed load is kept out of the collection canal and transported further downstream. In addition to this, the skimming spill way and the open canal de-silting to be recommended.

The site is best to command the irrigable area with reasonable driving head. Traditional, farmers have been using this site for local diversion site. The intake position is fixed at the scouring side of the river course so, that it could be easily to collect the base flow and keep the intake from sedimentation problem. Geological weak features of the river course have been treated well by protection structures

### Hydraulic Design of Headwork Structure

#### Selection of intake type

Bottom Intake (Tyrolean) un-gated intakes use for flow throttling structures with a rejection (skimming) spillway to limit the flows entering a canal & open escaping channel structure is to be included depending on moderate (Transitive) type of reach and the vast cross section of the river nature.

The water to be diverted is taken through a collection canal built in to the river bottom and covered with a screen. The bars of the screen are laid in the direction of the flow so that coarse bed load is kept out of the collection canal and transported further downstream. Breast wall to limit flows to flood channels is also included

### Outlet sill Level Determination

Command Area

- Maximum elevated Command  area elevation (Command area), =1455 a.m.s.l

- Maximum distance between the elevated command area and diversion site, =65.0m

- Assumed Canal slope, =1/150

- Head due to canal slope, = 1/150x66.0     = 0.43 m

- Elevation at the canal outlet, = 1455-0.43 = 1454.57 a.m.s.l.

#### Intake Dimensions

The total area will be irrigated in the main canal is 250 ha. The intake dimension is designed as concerning of spate irrigation and different type of dry & wet season crop of water requirement. The most basic form of control is a head regulator structure without gates. In its simplest form, this can be a rectangular opening with two sidewalls constructed of suitable materials (masonry, concrete & gabions) that serves to “throttle” the flows approaching the intake Optional sluiceway for ejecting sediment and maintaining low flow channel .Un gated head regulator for existing off take with skimming spillway the orifice head regulator limits maximum flow into the canal.

Q = (A *RO.667*SO.5)/ n

Where

A = area of cross- section in A=B*h (m2)

R = hydraulic radius

S = bed slope of the collecting canal

n=roughness coefficient

B = bed width of collecting canal

h= water depth in the canal in meter

Q = discharge diverted from the river = 2.81 m3 / sec Section of the canal rectangular

FB = free board

H =total depth of the canal

| Q d (Intake) | 8.44 |
| --- | --- |
| n | 0.017 |
| S | 0.007 |
| B | 2.5 |
| H | 1.5 |
| A | 3.75 |
| Wp | 5.5 |
| R | 0.682 |
| V | 2.251 |
| Ht | 1.5 |

Then the intake parameter Q=12m^3/s

B=2.5m

H=1.5m

S=1/150

V=2.25 m/s the velocity is consider by non-silting velocity & non –scouring (1.5-3.5m/s) of the standard velocity for design.

### River Bed Protection

Bed stabilizer:  considering control scour of river and stabilize riverbed level. Buried into river bed more durable and cost effective solutions

Bed bar Design

- Foot level of Bed Bar                = 1451.65a.m.s.l

- Bed Bar Depth                          = 3m

- Lean concrete at the bed	           = 0.3m

- Total Height of Bed Bar           =3.3 m

- Bed bar Crest Level                  =1451.65+0.3+3=14544.65 a.m.s.l

- Bad bar channel foundation depth is monolithic to that of right side of guide wall A. Stone Masonry Gravity Bed bar.

## DESIGN OF SCOURING SLUICE AND HEAD REGULATOR

### Scour depth

#### Cut off depth computation

At the Intake Entrance cutoff is provided. As per the geological report the river bed material is composed.

| Type of Material (soil) | Amount (no data) | Av. Grain size(dmm) | Lacey's Silt Factor "f" | Avg.of Size*Amount % | dmm*Amount | (d)mean |
| --- | --- | --- | --- | --- | --- | --- |
| Silt and Clay |  | 0.075 | 0.48 | 0.906 | 0.068 |  |
| fine sand |  | 0.425 | 1.14 | 0.091 | 0.039 |  |
| Caorse sand |  | 2.4375 | 2.73 | 0.004 | 0.009 |  |
| Average |  |  | 1.45 |  |  | 0.04 |
| R= | 2.744 | m |  |  |  |  |

The Lacey empirical equation may be used to compute the depth of scour. The design scour depth below bed level (D) is given by:

f=1.76*d1/2

Providing 3.0m depth of Bed bar is recommended.

### Head Regulator

If we take a three months growing period in fact with a by different steps of water demand considering the duty, we conceder it number of floods to irrigate our command. However, we don’t have enough local information about the frequency and magnitude of local floods frequency. In our case conceder, the max TAW depth of crop, soil (loam soil) & resection time of max flood occurrence. From this idea, solve intake structure

The design flow for the canal can be determined using the formula:

Qi = 2.77 Ai W

ε t

Where:

Qi	is the theoretical discharge (m3/s)

Ai	is the irrigable area (ha)

W	is the depth of application (assume 0.60m)

ε	is the application efficiency (assume 60%)

t	is the time of application (hrs)

Max. TAW depth      -----------   140mm

Max. Command area -----------   250ha

ε = 60%

Q i =2.77*250*0.14        = 2.81m^3/s

60%*24*3600

### Canal dimensions

The outlet canal acts as an under sluice From51.49 m up to 61.57m length. Then, the excess water released through escaping canal and the main canal continues. There is head regulator that controls the amount of water to be allowed to the irrigation canal and drain off. Therefore, the outlet canal is designed for 4.63m3/sec up to the head regulator. Shape of the canal is rectangular. Detail can be seen in the drawing album.

| Qd (m3/s) | 3.57 |
| --- | --- |
| B(m) | 1.5 |
| n | 0.017 |
| s | 0.007 |
| d | 0.9 |
| V (m/s) | 2.65 |

#### Gate provisions

The gate of the off take canal & escaping channel is to be sliding gates made with vertical sheet metal. The escaping side of the under sluice channel is regulated by 1.40mX1.48m size and the canal outlet is regulated by a gate size of 1.4mX1.48m. Groove allowance of 5cm is given and crossing Staffa tendino metal. Both of the gates are designed to be operated manually. For detail of the analysis, refer the attached Excel

| Description | Sluice gate | Canal Gate | Side Spillway | Remarks |
| --- | --- | --- | --- | --- |
| Height of gate, H | 1.50 | 1.00 | 0.60 | 6mm tick provide |
| Width of gate, W | 1.40 | 2.00 | 2.00 |  |
| Groove depth, gt | 0.05 | 0.05 | 0.05 |  |
| Water/Silt Depth (maximum) | 1.00 | 1.00 | 0.60 |  |
| Allowable stress, ton/m2 | 16500.00 | 16500.00 | 16500.00 |  |
| Unit weight of Water, ton/m3 | 1.00 | 1.00 | 1.00 |  |
| Unit weight of silt, ton/m3 | 0.35 | 0.35 | 0.35 |  |
| Water Pressure, PW, (ton) | 0.75 | 0.50 | 0.18 |  |
| Silt Pressure Pressure, (ton) | 0.26 | 0.18 | 0.06 |  |
| Total Pressure, PT (ton) | 1.01 | 0.68 | 0.24 |  |
| Length (W+2*gt) | 1.50 | 2.10 | 2.10 |  |
| Moment Maximum, M=P*L2/8 | 0.28 | 0.37 | 0.13 |  |
| Thickness, t, m | 0.008 | 0.012 | 0.009 |  |
| take Thickness ,t,m | 0.006 | 0.006 | 0.006 |  |
| Factor of safety, fs | 3.00 | 3.00 | 3.00 |  |
| E | 2100000.00 | 2100000.00 | 2100000.00 |  |
| Ta, Kg/cm2 | 1650.00 | 1650.00 | 1650.00 |  |
| Friction coefficient b/n gate and groove(f) | 0.50 | 0.50 | 0.50 |  |
| ﻻs | 7.85 | 7.85 | 7.85 |  |
| SPINDLE PARAMETERS |  |  |  |  |
| Ls(length / height of spindle), m | 2.00 | 2.00 | 2.00 |  |
| Ds (spindle Diameter), Assumed, mm | 70.00 | 70.00 | 70.00 |  |
| Ps, mm | 10.00 | 10.00 | 10.00 |  |
| Ns(number of spindle) | 1.00 | 1.00 | 1.00 |  |
| Weight |  |  |  |  |
| (weight of gate), W1 (ton) | 0.09 | 0.09 | 0.05 |  |
| (Weight of spindle), W2 (ton) | 0.06 | 0.06 | 0.06 |  |
| Wt(W1+W2) (ton) | 0.15 | 0.15 | 0.11 |  |
| Pw(water pressure on gate), (ton) | 0.53 | 1.00 | 0.36 |  |
| Pw(Silt pressure on gate), (ton) | 0.18 | 0.35 | 0.13 |  |
| Total Pressure, PT (ton) | 0.71 | 1.35 | 0.49 |  |
| Uplift Pressure, Pu, (ton) | 0.02 | 0.02 | 0.01 |  |
| Nd, ton | 0.37 | 0.70 | 0.25 |  |
| Nu, ton | 0.40 | 0.71 | 0.29 |  |
| I | 2.16 | 4.05 | 1.47 |  |
| Di, mm | 13.29 | 13.15 | 12.30 |  |
| Do, mm | 23.29 | 23.15 | 22.30 |  |
| Ts, Kg/cm2 | 285.84 | 523.77 | 246.03 | Ts < Ta, safe! |

#### Breast wall

To avoid spilling of water during HFL over the wing wall and canal regulator gate, an R.C.C wall is provided from the gate top level up to the river HFL (i.e. known as breast wall). More over to operate the gates operation slab is provided. Use of breast walls to set maximum flow limit to minimize size and weight of the gate,  slab and breast wall shall have 15cm thickness and reinforced with Ø14mm @ 20cm c/c And stirrup Ø10mm @20cm c/c Detail can be seen in the attached drawings & excel.

|  | Brust Wall Design consideration |  |  |
| --- | --- | --- | --- |
|  | Fixed at two end |  |  |
|  | Width (short direction), B | 1.4 |  |
|  | Anchor depth, Z | 0.2 |  |
|  | Length in Long direction, h | 1.52 |  |
| hw | Water Depth, hw | 1.5 |  |
|  | Water Depth | 1.5 |  |
|  | Wall thickness | 0.15 |  |
| γc | Unit weight RCC | 24 |  |
| γs | Unit weight Soil | 19 |  |
| бst |  | 200 |  |
| n |  | 9 |  |
| б'c |  | 20 |  |
| бc | бc = 0.45*б'c | 9 |  |
| K | K=(n*бc)/(n*бc+бst) | 0.29 |  |
| j | j=1-K/3 | 0.90 |  |
| R | R=1/2*бc*j*K | 1.17 |  |
| ζ | Shear capacity Concrete | 300 |  |
| γc | Unit weight RCC | 24 |  |
| γw | Unit weight Water | 10 |  |
|  | LOAD ANALYSIS |  |  |
| W | Water Pressure (Live Load) | 15.2 |  |
| P | Design Load = 200%*P | 30.4 |  |
|  | Maximum Positive Moment at mid span = P*L2/8 | 9.728 |  |
|  | Depth of Slab and Reinforcement |  |  |
|  | At Mid Span, d=(M/(RB))^0.5 | 91.09 |  |
|  | Over all Depth assumed, D (mm) | 150 |  |
|  | Bar Diameter (Assumed) | 12 |  |
|  | Effective Depth, d = D-50-n/2 | 94 |  |
| Ast | Area steel in short direction = M/(бst*j*d) | 572.45 |  |
|  | Spacing | 197 | Proved n12@20cm c/c |
|  | Nominal Reinforcement on the face of the wall |  | Proved n10@30cm c/c |
|  | Self-Weight Supporting Beam at the lower end |  |  |
|  | Weight of Slab, W | 36.48 |  |
|  | Maximum Moment, M=WL2/8 | 8.9376 |  |
|  | Depth, D | 300 |  |
|  | Thickness | 150 |  |
|  | d=(M/(RB))^0.5 | 159.40 |  |
|  | Bar Diameter (Assumed) | 14 |  |
|  | Effective Depth, d = D-50-n/2 | 243 |  |
|  | Area steel = M/(бst*j*d) | 203.45 |  |
|  | Spacing | 756 | Proved(4No) n14mm bar |
|  | Stirrups, |  | Proved n10@20cm c/c |

#### Trash Rack

Trash rack is provided to exclude boulders and tree trunk from entering inside the pocket and clog the gates. Therefore the intake Trash rack up to high flood level is provided.1.5m height trash rack with 24mm diameter steel spaced at 80mm horizontally and vertically is provided. At the top a horizontal 2m and 100mm diameter stiffening steel iron is provided to anchor with walls.

### Design of operation slab

The operating slabs are made to have dimensions of 1.60m*1.00m and 1.6m*1m for the under sluice & off-canal respectively. Sample design is done for the under sluice operation slab and make the operator free from risk during operation, the operation slabs should be enclosed with handrails.

The thickness of the slab is seen from the recommendation of thickness for R.C.C slabs in different constructions (two-way slab).

The recommendation is –

L/t = 20 to 35 where L = length of the greater span=1.60m.

t = thickness of slab

Take L/t =28 => t=L/ 28 = 1.60/28 = 0.05m, Provide t= 0.2m

The reinforcement is provided in accordance with the minimum percentage of reinforcement.

Hence,

As= 0.15% of gross area of the section (Gross area= 0.15*1.60*1= 0.24m2)

As=0.15*0.24/100=0.000366 m2=366mm2

Spacing S= (1000* as)/Aa = is assumed bar (Φ12mm bars)

S = (113*1000)/366 = 308.32mm

Provide Φ12 bars at 250mm c/c spacing in both directions with minimum reinforcement cover of 25mm.

### Escaping canal & side spillway

#### Escaping Channel

The escape channel (scouring sluice) size is fixed to accommodate as much of the diverted discharge, and the dimension is determined with the concept of rectangular notch principle.

Q=2/3*cd*(2*9.81) ^.5*L*H (3/2)

Where;

- Discharge, Q	             =4.63m3/s

- coefficient, cd                     =0.62

- Bed width, L		 =1.40m

- Bed slope, s 		 =1/500

- Canal velocity, computed by using Manning equation and considering regime velocity with aid of flow master program, V = 2.4m/s

- Flow depth, h	            =1.48m

- Free board, FB		=0.5m

- Total depth, H		=1.98m (Take 2m)

### Side spillway

Construct rejection spillways will often be required to limit the flows diverted to canals. The rejection capacity is normally provided as a side spillway in the first part of the main canal, where water can be easily passed back to the river. The effectiveness of the spillway is in proportion to its length. The spillway will be more effective if a further Flow control structure is provided on the canal just downstream of the spillway, as water level changes at the structure will be much more sensitive to excess flow than in an open channel.

The calculation is as follows

Total Q in intake=12m^3/s

The available duties=4.63m^3/s

The discharge in skimming channel is = (12-4.63) =7.37m^3/s is the discharge of over the side spillway the cross section is determined as follows.

          - Discharge, Q	            =7.37m3/s

          - coefficient, n                     =0.62

          - Bed width, B		 =1.40m

          - Bed slope, s 		 =1/500

          - Canal velocity,  V         = 2.28 m/s

          - Flow depth, h	            =1.08m

          - Free board, FB		 =0.3m

          - Total depth, H		 =1.38m

| Skimming spillway size is fixing by solver system. |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Q (m^3/s) | B(m) | H (m) | A (m^2) | n | FB (m) | H=FB+h | V (m^2/S) | Q (m^3/s) |
| 7.37 | 3 | 1.08 | 3.23 | 3.2 | 0.3 | 1.38 | 2.2823995 | 7.3703521 |

## DESIGN OF WINGWALL

### Upstream & Downstream Wing wall

Both Upstream and downstream wing wall crest level have been uniform b/s there is no afflux Outlet canal is located to the right side of the river.

U/s HFL = D/s HFL                = 1457.45 a.m.s.l

Lowest River Bed level           =1454.95 a.m.s.l

Wing Wall Crest Level            =1457.45 a.m.s.l

Normal depth                         = (B.L-C.L) =2.5 m

Guide Wall Depth.                  =2.5m

Depth of water that have impact on the wall = 1457.45-1454.95 = 2.5m In addition, the masonry height is considered only for 2.5m height with 0.5- foundation. Depth & cut off 2.0 m depth depending on the scour.

#### Stability analysis of U/s and D/s Guide wall

| H | FB | TH |  | B1 | B2 | BW |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 2.2 | 0.3 | 2.5 |  | 0.5 | 1.5 | 2 |  |
| Code | Load |  | Lever Arm, R | Moment |  |  |  |
|  | Vertical | Horizontal |  | Positive | Negative |  |  |
| W1 | 28.1 |  | 1.8 | 49.2 |  |  |  |
| W2 | 42.2 |  | 1.0 | 42.2 |  |  |  |
| PW |  | 23.7 | 0.8 |  | 19.8 |  |  |
| PS |  | 8.5 | 0.8 |  | 7.1 |  |  |
| SUM | 70.3 | 32.2 |  | 91.4 | 26.8 |  |  |
| Factor of safety against, |  |  |  |  |  |  |  |
| Over turning = (M+ve/M-ve) >1.5 |  |  |  | Fo | 3.41 | >1.5 | OK |
| Sliding = (µxFv/Fh),---µ=0.75 >1.5 |  |  |  | Fs | 1.64 | >1.0 | OK |
| Tension: X= (Net Moment/Sum Fv), e=[x-B/2], e<B/6 |  |  |  | X | 0.92 |  |  |
| B/6= |  |  | 0.33 | e | 0.082 | <B/6 | Ok |

## Bill of Quantity and Cost Estimation

The quantities of the various items have been worked out as per the final design and final drawings prepared for the scheme. The unit rates analysis has been carried out based on the data available in the vicinity of the project area.

Summary

|  | Summery Gubu 3 |  |
| --- | --- | --- |
| Bill No. | Description | Amount (Birr) |
| 1 | General Items | 1,650,032.50 |
| 2 | Head work (Weir body, Outlet structures) | 7,444,111.63 |
| 3 | Infrastructures | 15,887,147.68 |
| Grand Total |  | 24,981,291.81 |
|  | Hectar | 250.00 |
|  | Cost/Ha | 99,925.17 |

Bill No. 1- General Items

| Item No | Description | Unit | Quantity | Rate Birr | Amount Birr |
| --- | --- | --- | --- | --- | --- |
| 1.1 | Allow for mobilization | L.S | 1 | 262,450.92 | 262450.92 |
| 1.2 | Allow for demobilization | L.S | 1 | 209,960.74 | 209960.736 |
| 1.3 | Camp (Staff residence, office, Café, Kitchen, Store, Toilet & Shower, Guard House and Genentor house) | L.S | 1 | 650,000.00 | 650000 |
| 1.4 | Allow for temporary access road to site | km | 2.5 | 196879 | 492197.3438 |
| 1.6 | Provide project indicator post starting from the construction time | LS | 1 | 9225.9 | 9225.9 |
| 1.7 | Provision of as built drawings for the project | LS | 1 | 26197.6 | 26197.6 |
| Total |  |  |  |  | 1,650,032.50 |

Head work

| S.NO | ITEM OF WORK | UNIT | QUANTITY | UNITE PRICE | AMOUNT (ETH. BIRR) |
| --- | --- | --- | --- | --- | --- |
| 1 | Bed Bar |  |  |  |  |
| 1.1 | Excavation(Ordinary soil) | m3 | 393.3 | 90 | 35397 |
| 1.2 | Back fill selected material | m3 | 196.65 | 114 | 22418.1 |
| 1.3 | Back fill & Compacted with Hard Core | m3 | 19.665 | 460 | 9045.9 |
| 1.4 | Cyclopean for Bed Bar | m3 | 171 | 1900 | 324900 |
| 1.5 | 50 cm Capping |  |  |  | 0 |
| 1.5.3 | Concrete (C-20) | m3 | 34.2 | 2900 | 99180 |
| 1.5.4 | Ø 12mm reinforcement bar | Kg | 501.6 | 47 | 23414.688 |
| 1.5.5 | Ø 8 mm reinforcement bar/Stirrups | kg | 69.948 | 47 | 3265.17264 |
| 1.5.6 | Hard core Filling t=30cm | m3 | 20.52 | 482 | 9896.292029 |
| sub total |  |  |  |  | 0 |
| 2 | Left side Side of the River Guide wall |  |  |  |  |
| 2.1 | U/S Guide Wall Masonry (1:4 ratio) | m3 | 100.00 | 1,850.00 | 185,000.00 |
| 2.1.1 | C-15 Base Concrete | m3 | 35.00 | 2,500.00 | 87,500.00 |
| 2.1.2 | Hardcore | m3 | 40.00 | 482.28 | 19,291.02 |
| 2.1.3 | Excavation(Ordinary soil) | m3 | 300.00 | 90.00 | 27,000.00 |
| 2.1.4 | Back fill selected material | m3 | 80.00 | 114.00 | 9,120.00 |
| 2.2 | Plastering(1:3) | m2 | 120.00 | 122.00 | 14,640.00 |
| 2.3 | Gabion Guide Wall | m3 | 1,000.00 | 1,400.00 | 1,400,000.00 |
| 2.3.1 | Excavation | m3 | 1,750.00 | 90.00 | 157,500.00 |
| 3 | Right Side of the River Guide wall |  |  |  | - |
| 3.1 | U/S Guide Wall Masonry (1:4 ratio) | m3 | 131.25 | 1850 | 242,812.50 |
| 3.1.1 | C-15 Base Concrete | m3 | 25.2 | 2500 | 63000 |
| 3.1.2 | Hardcore | m3 | 31.5 | 482 | 15,191.68 |
| 3.1.3 | Excavation(Ordinary soil) | m3 | 315 | 90 | 28350 |
| 3.1.4 | Back fill selected material | m3 | 56.7 | 114 | 6463.8 |
| 3.2 | Plastering(1:3) | m2 | 147 | 122 | 17934 |
| 3.3 | Gabion Guide Wall | m3 | 1600 | 1400 | 2,240,000.00 |
| 3.3.1 | Excavation | m3 | 2280 | 90 | 205200 |
| 3.4 | Trash rack |  |  |  | 0 |
| 3.4.1 | Ø 12mm reinforcement bar | kg | 116 | 46.68 | 5412 |
| 3.4.2 | Ø 100 mm Stiff Iron | L | 3.6 | 500 | 1800 |
| 3.4.3 | Trash rack concrete cover C-20 | m2 | 0.78 | 2900 | 2262 |
| sub total |  |  |  |  | 0 |
| 4 | U/S Guide Wall Masonry (1:4 ratio) | m3 | 94 | 1850 | 173437.5 |
| 4.1 | C-15 Base Concrete | m3 | 18 | 2500 | 45000 |
| 4.2 | Hardcore | m3 | 18 | 482 | 8680.95792 |
| 4.3 | Excavation(Ordinary soil) | m3 | 144 | 90 | 12937.5 |
| 4.4 | Back fill selected material | m3 | 34.5 | 120 | 4140 |
| 4.5 | Plastering(1:3) | m2 | 101 | 122 | 12276.25 |
| 4.6 | Gabion Guide Wall | m3 | 483 | 1400 | 676200 |
| 4.7 | Excavation | m3 | 138 | 90 | 12420 |
| sub total |  |  |  |  |  |
| 5 | Intake /Approach channel , Under sluice & Scaping channel |  |  |  |  |
| 5.1 | Excavation(soil) | m3 | 460 | 90 | 41400 |
| 5.2 | Back fill with selected material | m3 | 93 | 120 | 11212.5 |
| 5.3 | Masonry for Approach channel | m3 | 450 | 1850 | 832500 |
| 5.4 | C-20 Concrete for under sluice | m3 | 18 | 2900 | 52200 |
| 5.5 | Plastering 1:3 ratio | m2 | 420 | 122 | 51240 |
| 5.6 | lean ConcereteC-15 (1:2:4) for Approach Channel Bed | m3 | 45 | 2500 | 112500 |
| 5.6 | Hard Core Filling t=30cm | m3 | 45 | 482 | 21702.3948 |
| sub total |  |  |  |  |  |
| 6 | Gates |  |  |  |  |
| 6.1 | Off take canal gate consists Installation, the gate consists the following parts | No | 1 | 24600 | 24600 |
|  | 6mm thickness sheet metal, 2.0 m X 1.0m |  |  |  |  |
|  | 50X50X10mm angle iron, 6.0 m length |  |  |  |  |
|  | Ø 12mm bar for anchorage, 2.0Kg |  |  |  |  |
|  | Ø 10mm bar for anchorage, Angle iron with the masonry wall 2.0Kg |  |  |  |  |
|  | Handle 70mm steel 1.5 m length /Spindle |  |  |  |  |
| 6.2 | Under sluice take canal gate consists Installation, the gate consists the following parts | No | 2 | 18620 | 37240 |
|  | Handle 70mm steel 1.5 m length /Spindle |  |  |  |  |
|  | 8mm thickness sheet metal, 1.2m X 1m |  |  |  |  |
|  | 50X50X10mm angle iron, 6.0 m length |  |  |  |  |
|  | Ø 12mm bar for anchorage, 2.5Kg |  |  |  |  |
|  | Ø 10mm bar for anchorage, Angle iron with the masonry wall 2.0Kg |  |  |  |  |
|  | 60mm X 60mm Angle iron Stiffening gate head and handle 2.3m length |  |  |  |  |
|  | Handle 30mm steel 1m length |  |  |  |  |
| 6.3 | Scapping canal gate consists Installation, the gate consists the following parts | No | 2 | 18620 | 37240 |
|  | 6mm thickness sheet metal, 1.2m X 0.9m |  |  |  |  |
|  | 50X50X10mm angle iron, 3.3 m length |  |  |  |  |
|  | Ø 12mm bar for anchorage, 2.5Kg |  |  |  |  |
|  | Ø 10mm bar for anchorage, Angle iron with the masonry wall 2.0Kg |  |  |  |  |
|  | Handle 14mm steel 1.4m length |  |  |  |  |
| 7 | Operation Slaps and Breast Walls |  |  |  |  |
| 7.1 | C-20 Concrete | m3 | 1.2 | 2900 | 3480 |
| 7.2 | Ø 12mm c/c 250 mm | kg | 141.6 | 46.68 | 6610 |
| 8 | Masonry Stair/Step | m3 | 6 | 1850 | 11100 |
| sub total |  |  |  |  |  |
| TOTAL COST |  |  |  |  | 7,444,111.63 |

# SECTION-III: IRRIGATION AND DRAINAGE SYSTEMS INFRASTRUCTURE

# IRRIGATION AND DRAINAGE SYSTEMS DESIGN

## Irrigable Area Description

### Topography

The nature of topography can influence soils of an area in many ways. In irrigated agriculture, topography influences the choice of irrigation method, the labour requirement, the irrigation efficiency, the cost of land development, the problems of drainage, the hazards of erosion, the range of possible crops that can be grown  and the size/shape of fields to put under cultivations. Furthermore, the suitable soils occur on a range of slopes up to 12%. A slope of land greater than 12% is considered permanently unsuitable for irrigation for all land use types (LUTs). Based on secondary data obtained from Jarota (04) Kebele Agricultural Development Office, the topography feature of the kebele contains 100% plain. On the other hand, based on slope gradient classes which have developed from topographic survey data of the command area, using ArcGIS 10.1 Software, the command area has consisted of level to gently sloping topographic feature. In general, the slope gradient classes of the command area range from 0.2% to 5%.

From the abovementioned evidences and the agro-ecological suitability of the area, development of modern spate irrigation scheme is believed to be encouraging for the production of drought tolerant crops and bring about rapid and sustainable development in the project area.

### Climate

According to the Traditional Ethiopian Agro-Ecological Zones classification, by taking the amount of mean annual rainfall, mean air temperature and elevation of the area into considerations, the project area is said to be found in Dry Kolla (Dry-hot) agro-ecological zone. In the project area the mean daily maximum air temperature ranges between 26.4oC and 34.5oC; whilst the mean daily minimum air temperature ranges between 11.3oC and 18.7oC.

For this spate irrigation scheme the source of water is from upstream catchment flood flow (i.e., Gidan woreda peak highlands), the source of meteorological data used for the description of the project site characteristics is Kobo Meteorological station. Therefore, the rainfall (mm), minimum and maximum temperature (oC), relative humidity (%), sunshine hours (hr) and wind speed (m/sec) data, for the project area, are collected from Kobo meteorological station.

The proposed command area has uni-modal rainfall pattern; although bi-modal-like rainfall but with very little and unreliable belg rainfall exist in the peak mountainous areas of the woreda. The main rainy season is summer, locally called Meher/Kiremt, which extends from July to August. The mean annual rainfall that the project area received is 590mm.

### Soil characteristics

Soil properties (physical, chemical, etc.) greatly influence the growth and thereby yield of crops which is grown. The command area has predominantly silt and clay textured soils which can be classified as moderately drained soil.  Most of the study area soils are categorized as deep soil (1-1.5 meter depth). Soils of the command area are suitable for most of the selected crops to be grown (for further detail see the Agronomy Study of the same project.

### Existing Irrigation Practices in the Project Area

The pressure of survival and the need for additional food supplies to meet the demands of the increasing population is necessitating a rapid expansion of irrigation schemes. Thus, irrigation is becoming a basic part of well-developed agriculture wherever there is water and irrigable land potential. Accordingly, traditional irrigation practices are under taken by individual farmers that use the river flow to the right side is with laborious temporary diversions. So, the farmers in the project area are very much interested in the idea of upgrading the traditional scheme to modern scheme.

## Irrigation Water Requirement

### Crop Water Requirement (CWR)

The calculation of crop water requirement is a very important aspect for planning of any irrigation project. Several methods and procedures are available for this. The Food and Agriculture Organization (FAO) of the United Nations has also made available several publications on this subject and other issues related with this. The computer program available in FAO Irrigation and Drainage Paper No. 56 “CROPWAT” has been used for the calculation of Crop Water requirement. This program is based on Penman-Monteith approach and procedures for calculation of crop water requirements and irrigation requirements are mainly based on methodologies presented in FAO Irrigation and Drainage Paper No. 24 “Crop Water Requirements” and No. 33 “Yield Response to Water”.

The corresponding values of the crop water requirements of the proposed crops of the project are presented in the Agronomy Study of the same project.

### Irrigation efficiency (Ep)

To complete the evaluation of the demand, the efficiency of the water distribution system and of application must be known.

The gross requirement of water for irrigation system is very much dependent on the overall efficiency of the irrigation system, which in turn is dependent on several factors: Method of irrigation, type of canal (Lined and/or Unlined), method of operations (simultaneously and continuous or Rotational water supply), and availability of structures (for controlling and distribution and measuring and monitoring).

On the basis of these factors, the project has planned to impose surface irrigation method (using furrows). The canal system is unlined other than canal near the head work. Hence, the conveyance efficiency has been estimated to be 90%, distribution efficiency 80%, and field application efficiency 60%. As a result of these the overall irrigation efficiency has been estimated to be 51%. According to soil Lab result, soils of the command area are predominantly characterized as clayey soils.

### Irrigation duty

Irrigation duty is the volume of water required per hectare for the full flange of the crops. Moreover, it helps in designing an efficient irrigation canal system.

The area, which will be irrigated, can be calculated by knowing the total available water at the source and the overall duty for all crops required to be irrigated in different seasons of the years.

The crops irrigation water requirement for Gobu 3 spate irrigation project command has calculated and presented in annex 4 for wet season crops as supplementary irrigation systems. The detail NIWR for the proposed wet season crops has presented in Annex 4.1 to Annex 4.5.

For the wet season proposed crops and cropping pattern of Gobu 3 spate irrigation project maize has showed maximum net irrigation water requirement (NIWR) of 4.99 mm/day in the month of September. The maximum gross irrigation water requirement (GIWR):

GIWR = NIWR/Efficiency = 4.99/0.49 = 10.18 [mm/day] for supplementary irrigation

The duty is calculated as: D = GIWR × 1000 × 10 / (t × 60×60) formula

Where; D = Duty [l/s/ha]

GIWR = Gross Irrigation Requirement [mm/day]

t = Daily irrigation or flow hours [hrs]

Thus, duty for 14 irrigation hours of wet season proposed crops as a supplementary irrigation has computed as: D = (10.18 x 10 x 1000) / (14 x 3600) = 2.0 l/s/h.

### Irrigation methods

Among the different irrigation systems Gobu-3 irrigation system will be used for the project area; and the irrigation water will be obtained from Gobu River and by constructing diversion weir/intake and convoying the water commonly through earthen canals (MC, SC, and TC) and then leading to field canals; and finally irrigation takes place mostly in furrows.

For this project, among the various irrigation methods, surface irrigation method has been selected.  Of the surface irrigation methods furrow, border and basin irrigation methods can be used to supply irrigation water to the plants/crops. However, each method has its own advantages and disadvantages. Care should be taken when choosing the method which is best suited to the local circumstances, i.e., depending on slopes, soil types, selected crop types, amount of water available, etc. of the command area.

Based on the above factors surface irrigation method has been proposed for the proposed crops in this project. The method allows applying light irrigation and can be laid out in sloping fields along the contour. Furrow irrigation method is best suited for most of the proposed and row planted crops. In general, furrow irrigation method is simple, manageable and widely practiced irrigation method. This method is suitable for row crops that cannot stand in water for long periods. The only thing required to use this method is row planting of crops. Besides, basin and border irrigation method would be used for the non-row planted crops. Rotational flow water distribution is also recommended for the project area.

## Irrigation and Drainage System Layout

The irrigation system layout for the project is prepared taking the following points into consideration besides other factors.

- A primary concern in the layout of the system is that it serves the purpose of conveying and distributing water to the command area.

- The excavation and earth fill volumes not be excessive, otherwise the construction costs can be tremendous.

- The selection of longitudinal bed slope is made taking into account the existing slopes of the terrain, so as to minimize deviations in canal routing.

- Curves in canals should not be too sharp.

The proposed irrigation system layout comprises one main canal, three secondary canals and twenty one tertiary canals as shown on the layout Drawings. The main canal runs for most of its length parallel to the contours and several changes of direction are necessary to follow the topography.

### Conveyance System

The conveyance system consists of one Main canal to irrigate total command area of 250 ha. The main canal starts from Water abstraction site on right side and conveys water for a length of 2.5 Km.

Main canal is aligned along contours and supplies to three secondary unit.

## Design of the Canal System

Flow Depth and Section Capacity

The earthen canals have been designed with a trapezoidal shape and the lined ones with rectangular x-section using Manning's Formula:

Where Q= discharge (m3/s)

R= Hydraulic radius (Flow area/wetted perimeter)

S= Hydraulic gradient

n= Manning's roughness coefficient, n=0.024 is adopted for the earth channels and n=0.018 for the masonry lined part of the main canal

### Main Canal

The main canal is designed for a discharge of 2.81m3/s and depending on the site specific condition, appropriate slope is provided. Hydraulic parameters of the main canal are shown below.

Table 4-1: Hydraulic Parameters of main canal

BW= Canal bottom width                         QR= Required discharge

FSD= Full supply depth                             QD= Designed Discharge

FB= Free board

SS= Side slope

Vel= Velocity

### Secondary Canals

Table 4-2: Hydraulic Parameters of secondary canals

Figure 4-1: Typical Cross Section of secondary canals

### Tertiary canals

In the layout system there are 21 tertiary canals, the designed discharge is determined based on the duty of irrigation.  The sections of the canals are determined by using manning’s formula, and they are trapezoidal section. The hydraulic characteristics is Presented in Table 5.3 below.

Figure 4-2: Typical Cross Section of tertiary canals

Table 4-3: Hydraulic Parameters of tertiary canals

| CANAL | AREA (ha) | Qreq (m3/s) | n | FB | S | BW | D | A | WP | CANAL D | R | V | Qdes (m3/s) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TC1-1 | 15.0 | 0.17 | 0.023 | 0.15 | 0.0013 | 0.60 | 0.35 | 0.394 | 1.862 | 0.500 | 0.211 | 0.546 | 0.215 |
| TC1-2 | 15.0 | 0.17 | 0.023 | 0.15 | 0.0013 | 0.60 | 0.35 | 0.394 | 1.862 | 0.500 | 0.211 | 0.546 | 0.215 |
| TC1-3 | 15.0 | 0.17 | 0.023 | 0.2 | 0.0010 | 0.60 | 0.30 | 0.315 | 1.682 | 0.500 | 0.187 | 0.450 | 0.142 |
| TC1-4 | 15.0 | 0.17 | 0.023 | 0.2 | 0.0010 | 0.60 | 0.30 | 0.315 | 1.682 | 0.500 | 0.187 | 0.450 | 0.142 |
| TC1-5 | 15.0 | 0.17 | 0.023 | 0.15 | 0.0013 | 0.60 | 0.35 | 0.394 | 1.862 | 0.500 | 0.211 | 0.546 | 0.215 |
| TC1-6 | 15.0 | 0.17 | 0.023 | 0.2 | 0.0020 | 0.60 | 0.30 | 0.315 | 1.682 | 0.500 | 0.187 | 0.637 | 0.201 |
| TC-7 | 10.0 | 0.11 | 0.023 | 1.5 | 0.0020 | 0.40 | 0.30 | 0.255 | 1.482 | 1.800 | 0.172 | 0.602 | 0.153 |
| TC-8 | 10.0 | 0.11 | 0.023 | 1.5 | 0.0083 | 0.40 | 0.25 | 0.194 | 1.301 | 1.750 | 0.149 | 1.115 | 0.216 |
| TC-9 | 10.0 | 0.11 | 0.023 | 1.5 | 0.0020 | 0.40 | 0.30 | 0.255 | 1.482 | 1.800 | 0.172 | 0.602 | 0.153 |
| TC-10 | 10.0 | 0.11 | 0.023 | 1.5 | 0.0083 | 0.35 | 0.25 | 0.181 | 1.251 | 1.750 | 0.145 | 1.095 | 0.198 |
| TC-11 | 10.0 | 0.11 | 0.023 | 1.5 | 0.0083 | 0.40 | 0.25 | 0.194 | 1.301 | 1.750 | 0.149 | 1.115 | 0.216 |
| TC-12 | 10.0 | 0.11 | 0.023 | 1.5 | 0.0111 | 0.40 | 0.25 | 0.194 | 1.301 | 1.750 | 0.149 | 1.287 | 0.249 |
| TC-13 | 10.0 | 0.11 | 0.023 | 1.5 | 0.0100 | 0.40 | 0.25 | 0.194 | 1.301 | 1.750 | 0.149 | 1.221 | 0.237 |
| TC-14 | 10.0 | 0.11 | 0.023 | 1.5 | 0.0056 | 0.40 | 0.25 | 0.194 | 1.301 | 1.750 | 0.149 | 0.910 | 0.176 |
| TC-15 | 10.0 | 0.11 | 0.023 | 1.5 | 0.0077 | 0.40 | 0.30 | 0.255 | 1.482 | 1.800 | 0.172 | 1.180 | 0.301 |
| TC-16 | 10.0 | 0.11 | 0.023 | 1.5 | 0.0091 | 0.40 | 0.30 | 0.255 | 1.482 | 1.800 | 0.172 | 1.283 | 0.327 |
| TC-17 | 10.0 | 0.11 | 0.023 | 1.5 | 0.0091 | 0.40 | 0.30 | 0.255 | 1.482 | 1.800 | 0.172 | 1.283 | 0.327 |
| TC-18 | 10.0 | 0.11 | 0.023 | 1.5 | 0.0100 | 0.40 | 0.30 | 0.255 | 1.482 | 1.800 | 0.172 | 1.345 | 0.343 |
| TC-19 | 10.0 | 0.11 | 0.023 | 1.5 | 0.0063 | 0.40 | 0.30 | 0.255 | 1.482 | 1.800 | 0.172 | 1.064 | 0.271 |
| TC-20 | 10.0 | 0.11 | 0.023 | 1.5 | 0.0056 | 0.40 | 0.30 | 0.255 | 1.482 | 1.800 | 0.172 | 1.003 | 0.256 |
| TC-21 | 7.5 | 0.09 | 0.023 | 1.5 | 0.0014 | 0.40 | 0.25 | 0.194 | 1.301 | 1.750 | 0.149 | 0.462 | 0.089 |

### Field Canals

As shown in the layout, field canals run across the contours and hence face relatively steeper gradient. The discharge of most of the field canals is very small and this is taken as an advantage to cope up with the relatively steeper gradient. Figure 15 below shows a typical field canal x-section. As much as possible field canals shall be made in fill in order to easily irrigate the adjacent command area. As can be seen from the layout, majority of the filed canals can be used to irrigate both sides of the command area depending on the condition of the individual plots of land owned by individual farmers.

Figure 4-3: Typical Field Canal X-section

## Drainage Canals

### Catch drainage canals

In the layout system there are 1 Catch drain canals, the designed discharge is determined based on the Catchment area, run off coefficient and rain fall intensity (Rational formula).  The sections of the canals are determined by using manning’s formula, and they are trapezoidal section.

Figure 4-4: Typical Cross Section of Catch drain canals

### Tertiary and Field canal drainage canals

Drainage Canals provided for the Tertiary and field canals. It is assumed that there is excess irrigation water during wet season. For the rainy season the water that comes through the tertiary and field canals directly entered to downstream of the river.

### Design of Division box

At different points of the main and secondary canals division boxes are provided which divert the flow to the secondary canal and tertiary canals. Gate should be provided at the outlet of the boxes. For detail refer the drawing.

Figure 4-5: Typical Division Box plan

Using broad crested formula,

Q= CL (h)3/2

Where; Q= discharge over rectangular weir/intake (opening), m3/s

C = discharge coefficient, c= 1.7

L= effective length of crest form in m

h= over flow depth, m

Assuming equal discharge coefficient & sill height for two or three dividing canals, the proportion becomes.

Q1/ Q2= Q2/ Q3 =L1/ L2 = L2/ L3

Where Q1= is flow in canal 1

Q2 = is flow in canal 2

Q3 = is flow in canal 3

L1= is effective crest length of weir/intake sill across opening to canal 1

L2= is effective crest length of weir/intake sill across opening to canal 2

L3 = is effective crest length of weir/intake sill across opening to canal 3

Q1= CL1 (h)3/2,

L1 = Q1/Ch3/2

L2 = L1*Q2/ Q1

L3 = L1*Q3/ Q1

The depth of (height of) the division box,

D = d + fb

The width of the division box,

B = b + 2*m*D

Where b= base width of the incoming canal

D = total canal depth of the incoming canal

Table 4-4: Hydraulic parameters of Division Boxes

### Road crossing structure

Two road crossing structures are provided on the main canal, at the existing foot paths. The road crossing structures are rectangular reinforced concrete slab. The slab is reinforced with 12mm @150mmc/c the length of the slab is 1.2m which is the same as the respective canal bed width, its width and thickness is 3m and 15mm respectively.

## Irrigation Infrastructure Bill of Quantities and Cost Estimate(GOBU-3)

| Sr.Nr | Work Description | Unit | Quantity | Unit rate | Amount |
| --- | --- | --- | --- | --- | --- |
| MAIN CANAL |  |  |  |  |  |
| 1 | Earth work & compaction |  |  |  |  |
| 1.1 | Clearing and grubbing | m2 | 6210 | 10 | 62,100.00 |
| 1.2 | Ordinary earth excavation | m3 | 4326 | 90 | 389,312.15 |
| 1.3 | Selected material Back fill and compaction | m3 | 998 | 114 | 113,751.54 |
| 1.4 | Main Canal Masonry Wall works(2500m) | m3 | 1,872.00 | 1,850.00 | 3,463,200.00 |
| 1.5 | Main Canal Cyclopean Bed works | m3 | 1980 | 1,900.00 | 3,762,000.00 |
| 1.6 | Plastering 1:3ratio | m2 | 9,180.00 | 122 | 1,119,960.00 |
| Sub Total-1 |  |  |  |  |  |
| 2 | SECONDARY CANAL |  |  |  |  |
| 2.1 | Earth work & compaction |  |  |  |  |
| 2.2 | Clearing and grubbing | m2 | 4313 | 10 | 43,125.00 |
| 2.3 | Ordinary earth excavation | m3 | 8720 | 90 | 784,834.29 |
| 2.4 | Selected material Back fill and compaction | m3 | 1011 | 114 | 115,284.10 |
| 3 | TERTIARY CANALS(TC) |  |  |  |  |
| 3.1 | Earth work & compaction |  |  |  |  |
| 3.2 | Clearing and grubbing | m2 | 310.5 | 10 | 3105 |
| 3.3 | Ordinary earth excavation | m3 | 365.976 | 90 | 32937.84 |
| 3.4 | Selected material Back fill and compaction | m3 | 9.384 | 114 | 1069.776 |
|  | Sub Total-1 |  |  |  |  |
| 4 | TC-2 |  |  |  |  |
| 4.1 | Earth work & compaction |  |  |  |  |
| 4.2 | Clearing and grubbing | m2 | 310.5 | 10 | 3105 |
| 4.3 | Ordinary earth excavation | m3 | 299.5175 | 90 | 26956.575 |
| 4.4 | Selected material Back fill and compaction | m3 | 60.0645 | 114 | 6847.353 |
| Sub Total-1 |  |  |  |  |  |
| 5 | TC-3 |  |  |  |  |
| 5.1 | Earth work & compaction |  |  |  |  |
| 5.2 | Clearing and grubbing | m2 | 372.6 | 10 | 3726 |
| 5.3 | Ordinary earth excavation | m3 | 2250.5615 | 90 | 202550.535 |
| 5.4 | Selected material Back fill and compaction | m3 | 19.6075 | 114 | 2235.255 |
| Sub Total-1 |  |  |  |  |  |
| 6 | TC-4 |  |  |  |  |
| 6.1 | Earth work & compaction |  |  |  |  |
| 6.2 | Clearing and grubbing | m2 | 372.6 | 10 | 3726 |
| 6.3 | Ordinary earth excavation | m3 | 2169.3715 | 90 | 195243.435 |
| 6.4 | Selected material Back fill and compaction | m3 | 237.7855 | 114 | 27107.547 |
| Sub Total-1 |  |  |  |  |  |
| 7 | TC-5 |  |  |  |  |
| 7.1 | Earth work & compaction |  |  |  |  |
| 7.2 | Clearing and grubbing | m2 | 414 | 10 | 4140 |
| 7.3 | Ordinary earth excavation | m3 | 1137.3615 | 90 | 102362.535 |
| 7.4 | Selected material Back fill and compaction | m3 | 7.6245 | 114 | 869.193 |
| Sub Total-1 |  |  |  |  |  |
| 8 | TC-6 |  |  |  |  |
| 8.1 | Earth work & compaction |  |  |  |  |
| 8.2 | Clearing and grubbing | m2 | 460 | 10 | 4600 |
| 8.3 | Ordinary earth excavation | m3 | 1125.229 | 62 | 69764.198 |
| 8.4 | Selected material Back fill and compaction | m3 | 1.2305 | 114 | 140.277 |
| Sub Total-1 |  |  |  |  |  |
| 9 | TC-7 |  |  |  |  |
| 9.1 | Earth work & compaction |  |  |  |  |
| 9.2 | Clearing and grubbing | m2 | 368 | 10 | 3680 |
| 9.3 | Ordinary earth excavation | m3 | 301.875 | 90 | 27168.75 |
| 9.4 | Selected material Back fill and compaction | m3 | 5.106 | 114 | 582.084 |
|  | Sub Total-1 |  |  |  |  |
| 10 | TC-8 |  |  |  |  |
| 10.1 | Earth work & compaction |  |  |  |  |
| 10.2 | Clearing and grubbing | m2 | 471.5 | 10 | 4715 |
| 10.3 | Ordinary earth excavation | m3 | 252.5975 | 90 | 22733.775 |
| 10.4 | Selected material Back fill and compaction | m3 | 2.208 | 114 | 251.712 |
|  | Sub Total-1 |  |  |  |  |
| 11 | TC-9 |  |  |  |  |
| 11.1 | Earth work & compaction |  |  |  |  |
| 11.2 | Clearing and grubbing | m2 | 471.5 | 10 | 4715 |
| 11.3 | Ordinary earth excavation | m3 | 383.0305 | 90 | 34472.745 |
| 11.4 | Selected material Back fill and compaction | m3 | 21.9535 | 114 | 2502.699 |
| Sub Total-1 |  |  |  |  |  |
|  | TC-10 |  |  |  |  |
| 1 | Earth work & compaction |  |  |  |  |
| 1.1 | Clearing and grubbing | m2 | 471.5 | 10 | 4715 |
| 1.2 | Ordinary earth excavation | m3 | 782.115 | 90 | 70390.35 |
| 1.3 | Selected material Back fill and compaction | m3 | 18.768 | 114 | 2139.552 |
| Sub Total-1 |  |  |  |  |  |
|  | TC-11 |  |  |  |  |
| 1 | Earth work & compaction |  |  |  |  |
| 1.1 | Clearing and grubbing | m2 | 471.5 | 10 | 4715 |
| 1.2 | Ordinary earth excavation | m3 | 1796.6335 | 90 | 161697.015 |
| 1.3 | Selected material Back fill and compaction | m3 | 24.978 | 114 | 2847.492 |
| Sub Total-1 |  |  |  |  |  |
|  | TC-12 |  |  |  |  |
| 1 | Earth work & compaction |  |  |  |  |
| 1.1 | Clearing and grubbing | m2 | 448.5 | 10 | 4485 |
| 1.2 | Ordinary earth excavation | m3 | 560.257 | 90 | 50423.13 |
| 1.3 | Selected material Back fill and compaction | m3 | 7.291 | 114 | 831.174 |
| Sub Total-1 |  |  |  |  |  |
|  | TC-13 |  |  |  |  |
| 1 | Earth work & compaction |  |  |  |  |
| 1.1 | Clearing and grubbing | m2 | 256.45 | 10 | 2564.5 |
| 1.2 | Ordinary earth excavation | m3 | 246.859 | 90 | 22217.31 |
| 1.3 | Selected material Back fill and compaction | m3 | 27.485 | 114 | 3133.29 |
| Sub Total-1 |  |  |  |  |  |
|  | TC-14 |  |  |  |  |
| 1 | Earth work & compaction |  |  |  |  |
| 1.1 | Clearing and grubbing | m2 | 284.05 | 10 | 2840.5 |
| 1.2 | Ordinary earth excavation | m3 | 395.209 | 62 | 24502.958 |
| 1.3 | Selected material Back fill and compaction | m3 | 50.1515 | 114 | 5717.271 |
| Sub Total-1 |  |  |  |  |  |
|  | TC-15 |  |  |  |  |
| 1 | Earth work & compaction |  |  |  |  |
| 1.1 | Clearing and grubbing | m2 | 284.05 | 10 | 2840.5 |
| 1.2 | Ordinary earth excavation | m3 | 767.1995 | 90 | 69047.955 |
| 1.3 | Selected material Back fill and compaction | m3 | 53.9005 | 114 | 6144.657 |
|  | Sub Total-1 |  |  |  |  |
|  | TC-16 |  |  |  |  |
| 1 | Earth work & compaction |  |  |  |  |
| 1.1 | Clearing and grubbing | m2 | 282.9 | 10 | 2829 |
| 1.2 | Ordinary earth excavation | m3 | 383.0765 | 90 | 34476.885 |
| 1.3 | Selected material Back fill and compaction | m3 | 1.0925 | 114 | 124.545 |
|  | Sub Total-1 |  |  |  |  |
|  | TC-17 |  |  |  |  |
| 1 | Earth work & compaction |  |  |  |  |
| 1.1 | Clearing and grubbing | m2 | 282.9 | 10 | 2829 |
| 1.2 | Ordinary earth excavation | m3 | 277.5755 | 90 | 24981.795 |
| 1.3 | Selected material Back fill and compaction | m3 | 1.61 | 114 | 183.54 |
|  | Sub Total-1 |  |  |  |  |
|  | TC-18 |  |  |  |  |
| 1 | Earth work & compaction |  |  |  |  |
| 1.1 | Clearing and grubbing | m2 | 282.9 | 10 | 2829 |
| 1.2 | Ordinary earth excavation | m3 | 536.0035 | 90 | 48240.315 |
| 1.3 | Selected material Back fill and compaction | m3 | 31.8895 | 114 | 3635.403 |
|  | Sub Total-1 |  |  |  |  |
|  | TC-19 |  |  |  |  |
| 1 | Earth work & compaction |  |  |  |  |
| 1.1 | Clearing and grubbing | m2 | 282.9 | 10 | 2829 |
| 1.2 | Ordinary earth excavation | m3 | 903.187 | 90 | 81286.83 |
| 1.3 | Selected material Back fill and compaction | m3 | 0.1035 | 114 | 11.799 |
|  | Sub Total-1 |  |  |  |  |
|  | TC-20 |  |  |  |  |
| 1 | Earth work & compaction |  |  |  |  |
| 1.1 | Clearing and grubbing | m2 | 282.9 | 10 | 2829 |
| 1.2 | Ordinary earth excavation | m3 | 376.2455 | 90 | 33862.095 |
| 1.3 | Selected material Back fill and compaction | m3 | 93.104 | 114 | 10613.856 |
|  | Sub Total-1 |  |  |  |  |
|  | TC-21 |  |  |  |  |
| 1 | Earth work & compaction |  |  |  |  |
| 1.1 | Clearing and grubbing | m2 | 282.9 | 10 | 2829 |
| 1.2 | Ordinary earth excavation | m3 | 498.1685 | 90 | 44835.165 |
| 1.3 | Selected material Back fill and compaction | m3 | 54.7745 | 114 | 6244.293 |
|  | Sub Total-1 |  |  |  |  |
|  | DIVISION BOX |  |  |  |  |
| 1 | Stone work |  |  |  |  |
| 1.1 | Masonry work | m3 | 174.8 | 1850 | 323380 |
| 1.2 | Cemented stone pitching | m3 | 83.03 | 1400 | 116242 |
| 1.4 | Plastering | m2 | 150.65 | 122 | 18379.3 |
| 2 | Gate supply, cutting ,welding and installation |  |  |  |  |
| 2.1 | 2mm thick sheet metal | No. | 23 | 6400 | 15720 |
| 2.2 | Stiffening angle iron (30*30*4) |  |  |  |  |
| 2.3 | Angle iron for groove(40*40*4) |  |  |  |  |
| 2.4 | Ø 12mm & 0.6 m of bar for handling |  |  |  |  |
| Sub Total-2 |  |  |  |  |  |
|  | Turn out |  |  |  |  |
| 1 | Stone work |  |  |  |  |
| 1.1 | Masonry work | m3 | 306.0265 | 1850 | 566149.025 |
| 1.2 | Cemented stone pitching | m3 | 851.4945 | 1400 | 1192092.3 |
| 1.4 | Plastering | m2 | 2231.276 | 122 | 272215.672 |
| 2 | Gate supply, cutting ,welding and installation |  |  |  |  |
| 2.1 | 2mm thick sheet metal | No. | 94 | 7600 | 714400 |
| 2.2 | Stiffening angle iron (30*30*4) |  |  |  |  |
| 2.3 | Angle iron for groove(40*40*4) |  |  |  |  |
| 2.4 | Ø 12mm & 0.6 m of bar for handling |  |  |  |  |
| Sub Total-2 |  |  |  |  |  |
|  | Foot path and Cattle crossing |  |  |  |  |
| 1.1 | Excavation |  |  |  |  |
| 1.1.1 | Ordinary earth excavation | m3 | 4.14 | 90 | 372.6 |
| 1.2 | Concrete work |  |  |  |  |
| 1.2.1 | C-20 concrete work | m3 | 2 | 2900 | 5800 |
| 1.3 | stone work |  |  |  |  |
| 1.3.1 | Masonry work | m3 | 3 | 1850 | 5550 |
| 1.3.2 | stone pitching | m3 | 4 | 1400 | 5600 |
| 1.3.2 | Plastering | m2 | 10.465 | 122 | 1276.73 |
| 1.4 | Reinforcement |  |  |  |  |
| 1.4.1 | ø12 | kg | 71.99 | 47 | 3383.53 |
| sub total |  |  |  |  |  |
|  | FLUME STRUCTURE@ TC-3,4,5&6 |  |  |  |  |
| 1.1 | Excavation |  |  |  |  |
| 1.1.1 | Ordinary earth excavation | m3 | 99 | 90 | 8910 |
| 1.2 | Concrete work |  |  |  |  |
| 1.2.1 | C-20 concrete work | m3 | 5 | 2900 | 14500 |
| 1.3 | Cut offrete)(cyclopean conc | m3 | 26 | 1,900.00 | 49400 |
| 1.3.1 | Masonry work | m3 | 44 | 1850 | 81400 |
| 1.3.2 | Plastering | m2 | 120 | 122 | 14640 |
| 1.4 | Reinforcement |  |  |  |  |
| 1.4.1 | ø12 | kg | 142 | 47 | 6674 |
| 1.4.2 | ø10 | kg | 80 | 47 | 3760 |
| sub total |  |  |  |  | 179284 |
|  | Flum structure @ TC-3,4,5&6 | NO | 4 |  | 717,136.00 |
| Total |  |  |  |  | 15,887,147.68 |

# CONCLUSION AND RECOMMENDATION

- The infrastructure of this project area is designed to irrigate about 250 ha of land by taking its supply from the Gobu-3 spate intake irrigation project.  The method of irrigation of the project area is furrow surface irrigation in which the main and tertiary canals are working continuously whereas the field canals within a tertiary block are working rotational system.

- As the dominant soil type is clay soil, the most portion of main canal system is designed to be masonry.

- The reason why the main canal is to be lined up to 1500m is to avoid the siltation problem, time saving to reach at the tail part, reduce maintenance cost.

- The design of the canal dimensions of the irrigation canal is done by applying the manning’s uniform flow equation. The variable of the hydraulic parameters are calculated using iteration or flow master program.

- As soils of the command area are predominantly clay textured; and hence water and soil management measures should be undertaken; and optimum moisture content should be maintained to improve workability of the soil during land preparation and planting time.

The following recommendations are drown:

- For better performance and long service year of the project regular inspection and maintenance is highly required.

- Farmers training, how to operate and maintain the project structures as a whole and available and water resources has a paramount important.

- The irrigation hours per day and per week should be flexible based on base flow amount of each week or month.

- Close supervision of the construction should be made to modify (if need be) each Components of irrigation system based on specific site conditions.

# OPERATION AND MAINTENANCE

## General

The main objective of the operation and maintenance aspect of an irrigation scheme is to facilitate the timely delivery of the required irrigation water to farms and to keep the irrigation system in an optimum operating condition.  This section therefore, discusses the main functions of the subject matter under consideration for the scheme.

## Operation of the Head Works

Operation at the diversion weir mainly focuses on the diversion of a controlled flow of river water, timely cleaning of floating debris in front of intake and removal of sediment deposits in front of the weir and intake structures.

## Irrigation System Operation

The operation of the irrigation system depends mainly on the method of water delivery at farm level.  Surface irrigation method is the recommended type of water distribution and application method for Gobu-3 Spate irrigation scheme.

The farmers would organize themselves and form groups in order to handle the water management.  Since flow is low Rotational water distribution would be applied within the group.  The rotational distribution is then to distribute water by turn to the whole scheme according to the timely need of crop water requirement. For better and efficient water management, crop diversification should be avoided within a group.  This would reduce the complexity of water distribution system of the scheme during one irrigation season. At farmers’ level of operation, a constant flow and variable irrigation time is advisable.

The operation of the irrigation system is continuous for 14 hours per day in main, secondary and tertiary canals whereas field canals within a tertiary block are operating in rotational system with each other for irrigation hours proportional to their size.  Since the tertiary canal discharges are within the manageable range and the irrigation canal structures are accordingly designed for simple operation, the farmers can open and close easily whenever they required.

## Maintenance Requirement

The canal system of the project is earthen canal except main canal and secondary canal, which is susceptible to siltation, erosion, growth of weeds and or breaching problems.  Though the canal sections are designed for non- silting and non-scouring conditions, the above mentioned problems are unavoidable and hence periodical and yearly inspection and maintenance of canals and structures are obligatory in order to fulfill the design objective of the project area.

The maintenance tasks are categorized into two types: - routine activities, and repairs. The routine maintenance activities that are carried out periodically include:-

- Regular cleaning of sediments and weeds from canals and drains;

- Inspection and lubrication of gates; and

- Maintenance of cracked lined canals, regulating and control structures.

Repair works include task carried out more frequently and quickly, and include those tasks that are generally unpredictable.  They also include emergency works.  The activities included in this category are:-

- Repairing overtopped or breached canals, drains, and flood protection dykes;

- Repairing jammed gates;

- Filling holes made by wild animals; and

- Reduced free board due to walking over by people and livestock.

Regular inspection of the irrigation facilities should be carried out as part of the maintenance activities.  These tasks could be carried out immediately after the end of the main rains in September and during the rainy season.  This could concentrate on the interceptor drains and the flood protection dykes, the main canal and the field drains.  The inspection of the other works like the tertiary canals, field drains, and the water control and regulating structures could be carried out as part of routine operation activities.

- Beneficiaries of the project need to have operation and maintenance budget, For O&M cost incurring entity area:

- Purchase sing of gate lubricate (grease)

- Replacing and maintenance of Stolen and damaged gates

- Repair Damages on the cross drainage structures.

The expense for O&M should be collected from the beneficiaries. Of course, much of the task is done by the labor and skill of the community. For cost incurring activities beneficiaries have to collect money based on the proportion of the command area they owned.

# REFERENCE

- FAO (1977) guidelines for predicting crop water requirements. No 24, Rome Italy

- Design of small Canal structures , USBR

- Soft copies of hydraulic structure publishing

- IDD manual

- ESRDF manual