Amhara National Regional State

Water Resources Development Bureau

(BOWRD)

Detail Design

Of

Gobu Spate Diversion Small-Scale Irrigation Project

Volume IV: Engineering Design Final Report

JUNE 2014(Revise 2015)

Bahir Dar

Client: Bureau of Water Resource Development

(BoWRD)

Address:

P. O. Box: 88
Telephone: 0528-200853/855
Fax: 251-08-20-65-68/204676/202040

Consultant: Amhara Design & Supervision Works Enterprise

(ADSWE)

Address:

P. O. Box: 1921
Telephone: +251-582-181023/ 180638/181201/181254
Fax: (058) 2180550/ (058) 2180560
E-mail: amhara design@yahoo.com

Bahir Dar, Ethiopia

# FEASIBILITY STUDY & DETAIL DESIGN REPORT STRUCTURE

- Volume I: Watershed Management

- Volume II: Engineering Geology

- Volume III: Irrigation Agronomy

- Volume IV: Engineering Design

- Volume V: Socio Economy

- Volume VI: Environmental Impact Assessment

Table of Contents								 Page Nr

FEASIBILITY STUDY & DETAIL DESIGN REPORT STRUCTURE	i

LIST OF TABLES	iv

LIST OF FIGURES	iv

1	Introduction	3

1.1	Background	3

1.1.1	Description of the Project Area	3

1.2	Objectives of the Study	4

1.2.1	Major Objective	4

1.2.2	Specific Objectives	5

1.3	Scope of the Study	6

1.4	Methodology	6

SECTION-I:  HYDROLOGY	9

2	Hydrology	10

2.1	Watershed Characteristics	10

2.2	Hydro-Metrological Data Availability	11

2.2.1	Climate	11

2.2.2	Rainfall Data	11

2.2.3	River flow data	11

2.2.4	Upstream & Downstream utilization	11

2.3	Design Flood Analysis	12

2.3.1	Design Rainfall computation	12

2.3.2	Outlier Test	12

2.3.3	Check for variance	14

2.3.4	Peak Discharge Determination	15

2.3.5	Tail Water Depth Computation	20

SECTION-II: HEADWORK DESIG	26

3	HEADWORK STRUCTURES DESIGN	27

3.1	Headwork Site Selection	27

3.2	River Geomorphology	27

3.2.1	River Bed condition	27

3.2.2	River Bank condition	28

3.3	Sources of construction materials	29

3.3.1	Rock for Masonry and Crushed Coarse Aggregate	29

3.3.2	Fine Aggregates	30

3.3.3	Water	31

3.4	Headwork Type Selection	31

4	Intake Dimensions	32

5	Approach canal	33

5.1.1	Under Sluice, and Canal outlet	34

6	Escape Gate	34

7	Trash rack	37

8	Retaining Walls	37

8.2	Bill of Quantity and Cost Estimation	39

SECTION-III: IRRIGATION AND DRAINAGE SYSTEMS INFRASTRUCTURE	43

9	IRRIGATION AND DRAINAGE SYSTEMS DESIGN	44

9.1	Irrigable Area Description	44

9.1.1	Topography	44

9.1.2	Soil characteristics	44

9.1.3	Existing Irrigation Practices in the Project Area	49

9.2	Irrigation Water Requirement	51

9.2.1	Crop Water Requirement (CWR)	51

9.2.2	Irrigation efficiency (Ep)	52

9.2.3	Irrigation methods	52

9.3	Irrigation and Drainage System Layout	53

9.3.1	Conveyance System	54

9.4	Design of the Canal System	54

9.4.1	Main Canal	54

9.4.2	Secondary Canals	56

9.4.3	Tertiary canals	57

9.4.4	Field Canals	60

9.4.5	Design of Division box	61

9.5	River Protection Works by Gabion	63

10	Bill of quantity	63

11	CONCLUSION AND RECOMMENDATION	65

12	OPERATION AND MAINTENANCE	66

12.1	General	66

12.2	Operation of the Head Works	66

12.3	Irrigation System Operation	66

12.4	Maintenance Requirement	67

REFERENCE	68

# LIST OF TABLES

Table 2-1: Outlier test analysis	12

Table 2-3: Determination of Time of Concentration	15

Table 2-4: Runoff analysis	17

Table 4-1: Hydraulic Parameters of main canal	54

Table 4-2: Hydraulic Parameters of secondary canals	56

Table 4-3: Hydraulic Parameters of tertiary canals	59

# LIST OF FIGURES

Figure 1-1: Location map of the project area	4

Figure 2-1: Drainage map of Gobu watershed	11

Figure 2-2: Complex Hydrograph	20

Figure 2-3: Rating Curve	23

Figure 2-4: River profile	25

Figure 3-1: Riverbed at the proposed spate site	28

Figure 7, Rock Quarry Site for Masonry Stone and Crushed Aggregates	30

Figure 4-1: Typical Cross Section of secondary canals	57

. Figure 4-2: Typical Cross Section of tertiary canals	58

Figure 4-3: Typical Field Canal X-section	60

Figure 4-5: Typical Division Box plan	62

.SAILENT FEATURE

- Project name: Gobu Spate Irrigation Project

- Name of the stream: Gobu river

- Location of spate site

- North:1360452.56

- South:571662.12

- Zone: North Wollo

- Woreda: Kobo

- Average Altitude: 1404.57 masl

- Hydrology

- Design rainfall: 106.54 mm

- Catchment area: 314.46 Km2

- Longest flow path length: 44.72Km

- Design flood: 670.71 m3/sec

- Design base flow: 0 lit/se.

- Diversion -Spate

- Spate type: Cyclopean reinforced concert

- Height: 4.5m Below river bed level

- Gross crest length: 101m

- Spate crest level: 1404.57 m.a.s.l

- Under sluice

- Sill level: 1403.37m.a.s.l

- Dimension: 1.7*1.2 m2

- Discharge amount:  3m3/sec

- Outlet

- Sill level: 1403.47m.a.s.l

- Opening dimension: 1.77*1.1m2

- Discharge capacity 3m3 lit/sec.

Irrigation and drainage systems Infrastructure

- Command area size: 400ha

- Type of soil of the command area is dominantly clay soil

- Design discharge of the main canal = 2000 l/sec

- Irrigation system layout consists of one earthen & lined main canal, 1secondary canals and 3 tertiary canals

- Main irrigation structures designed are;

- Gully crossing structures

Project cost

| Bill No. | Description | Amount (Birr) |
| --- | --- | --- |
| 1 | General Items | 1,864,749.01 |
| 2 | Head work (Weir body, Outlet structures) | 7,312,745.22 |
| 3 | Infrastructures | 21,300,392.60 |
|  | Grand Total | 30,477,886.83 |
|  | Command Area(ha) | 400 |
|  | Cost Per hectare | 76,194.72 |

# Introduction

## Background

In Ethiopia, under the prevalent rain-fed agricultural production system, the progressive degradation of the natural resource base, especially in highly vulnerable areas of the highlands coupled with climate variability have aggravated the incidence of poverty and food insecurity. The major source of growth for Ethiopia is still conceived to be the agriculture sector. Hence, this sector has to be insulated from drought shocks through enhanced utilization of the water resource potential of the country, (through development of small-scale irrigation, water harvesting, and on-farm diversification) coupled with strengthened linkages between agriculture and industry (agro-industry), thereby creating a demand for agricultural output. In line with the above, efforts have been made by the government to improve the situation in the country in areas of domestic water supply provision, irrigation, watershed management, etc. The Amhara Water Resources Development Bureau is playing its role in the development of small-scale irrigation projects in the region. Accordingly, as part of the water sector development program, the office has initiated the study and design of a small scale irrigation scheme on Gobu River at 05Hujera Kebele and signed an agreement with Amhara Design & Supervision Works Enterprise (ADSWE) for the study and design of the project.

### Description of the Project Area

#### Location

This irrigation project is located mainly at Hujera Kebele, Kobo Woreda of North wollo Zone in the Amhara Region. The proposed irrigation project is to be undertaken on Gobu River and the headwork structures are specifically located at an altitude of about 1404.57 masl and geographical coordinates of 1360452.36N (UTM) and 571662.12 E (UTM).

Figure 1-1: Location map of the project area

#### Accessibility

The project headwork site is accessed through asphalt rode from Kobo to Waja at about 25km, turning to the right and by passing 15 km through all-weather gravel road.

## Objectives of the Study

### Major Objective

The project area faces variability of rainfall distribution though the overall rainfall generally suffices the rain-fed agriculture. Accordingly, the rain-fed agriculture needs means of supplementing during distribution failures and further full irrigation is required to maximize the use of the potential land and water resources.

Hence, the objective of this project is to contribute a substantial share in the effort to reduce the risk of production decrease due to rainfall variability and increase the productivity of the resource in the project specific area. Specifically, the project is targeted for the following.

- To make sustainable the rain-fed crop production and make extra production in the dry season possible for 400 ha of land through irrigation.

- There is a general consensus that irrigation investments will achieve broader food security and poverty reduction impacts and if efforts are also geared towards up-grading existing traditional farming practices with support to enhance access to input supply, output marketing and extension to facilitate access to information and innovations.

- This objective is to be realized by constructing spate structures across the Gobu River and diverting the river flow.

### Specific Objectives

Other benefits that can be expected to appear with the launching of the project are:

- Efficiency of water use improvement;

- Improved local nutrition/food security gains;

- Improved management of scarce natural resources (land and water);

- Resilience against drought risk;

- Rationale for erosion control and watershed management;

- Rationale for the intensification and modernization of smallholder agriculture and rural lifestyles.

The engineering study and design enables the realization of the project by the provision of engineering structures that will allow the appropriate abstraction of the river water for delivery in to the identified irrigation fields of the study area. Hence, this engineering design is specifically targeted to:

- Analyze hydrologic requirements of the project and engineering structures;

- The formulation of sound and stable structure,  with necessary provisions that allow safe, easy and low-maintenance operation in the service life of the project;

- Develop working drawings;

- Estimation of construction costs.

## Scope of the Study

- The irrigation design shall ensure reliability, equity and flexibility of water delivery to farmers. It will aim at reducing conflicts among water users and will lead to lower operation and maintenance costs.

- Updating the existing, if available, computation of the actual evapo-transpiration, crop water requirement, irrigation demand/duty using the existing and recent agronomic, climatologic and soil data using more appropriate methodologies.

- Establish design criteria for irrigations structures to be approved by the client and to be used in the final design stage,

- Design proper irrigation system compatible with local conditions and management capabilities,

- Establish flood protection measures for the command area and canal structures and design the respective drainage system accordingly,

- Planning  and layout of the irrigation system, which include irrigation canals, drainage channels, inspection roads and alignments, canal spacing, canal length, location of structures, and water profiles along canal and drains at specified reaches, which is most economical easily manageable and aligned with topographic feature and geological investigation.

- Determination and estimation of water application conveyance and other losses and irrigation efficiencies and consideration of those parameters in design steps.

- Check and test hydraulic and structural designs of main canal considering total demand and the required capacity and the base flow availability,

- Prepare general plans and drawings for all irrigation infrastructure and irrigation systems designs,

## Methodology

In the study and design procedure, Designers used the following steps.

- Specific Site identification:

  - Review of the reconnaissance survey conducted by the Client

  - 50,000 scale top map and GIS information

  - Local farmers interview and discussion

  - Woreda and Zone Agriculture section expertise

  - Previous studies

  - On foot travel along the river channel and farm areas.

- Topographic survey:

  - Surveying the headwork site and the Command area with sufficient radius, using Total station

- Flow estimation

  - Physical observation on flood mark indications and local information about high flood and critical flow condition of the river

  - Analyzing the`recorded river flow data and use watershed inputs for further analysis.

- Irrigable area identification:

  - Using local information

  - 50,000 Topographic map, and GIS information, GPS to see elevation

The design report is organized in three sections. In Section I, the Hydrology study is presented and in Sections II and III, the Headwork and Irrigation and Drainage Systems designs are discussed respectively. In Section III, planning and design of the irrigation system after diverting the water using the spate will be dealt. The following are major areas of concern in this part.

- Study and design of the irrigation method to be adopted,

- Study and design of the irrigation system layout and associated structures,

- Design of the different conveyance canals,

- Planning and design of the different irrigation and drainage structures,

- Preparation of the longitudinal profiles of the different irrigation and drainage canals.

# SECTION-I:  HYDROLOGY

# Hydrology

## Watershed Characteristics

. The Watershed has marked topographic variation. All types of slopes are present. The dominant slope class is Steep (30-50%) which covers 2.28%, moderately steep (15-30%) of the total area which is 1.6%. followed Very steep (>50%) Sloping (8-15%) and gently sloping (3-8%), flat or almost flat (0-3%) accounts 1%, 91%, 4%, 0.67% respectively

Certain physical properties of watersheds significantly affect the characteristics of the runoff and sediment yield and are of great interest in hydrologic analyses. The rate and volume of runoff, and sediment yield from the watershed have much to do with shape, size, slope and other parameters of the landscape. These suggest that there should be some important relations between basin form and hydrologic performance. If the basin and hydrologic characteristics are to be related, the basin form must also be represented by quantitative descriptors. These parameters can be measured from maps.

The watershed characteristics are analyzed and presented in Table1of the Watershed Feasibility Study Report of the same project. In summary:

- Catchment Area  = 314.46km2

- Stream Length = 44.72Km

- CN(II) = 85.077

(Extracted from the Watershed Study Report of the same project)

At the selected reference point, the area of catchment is 314.46km2 and consists of a network of tributaries as shown in Figure 2 below.

Gobu River at the headwork site is characterized by no well-defined channel system and considerable flows. It looks that the gradient of the river is getting low and hence there exists significant deposition of sediment mainly fine sands.

Figure 2-1: Drainage map of Gobu watershed

## Hydro-Metrological Data Availability

### Climate

Small-scale irrigation project designers and planners are faced with lack of good data on the hydrology of the river system that will be their water source and on local weather and climate conditions. Stream gauging stations are virtually non-existent in remote rural areas of Ethiopia; meteorological stations are almost rare. Likewise, at 05 Hujera kebele (Project area location) and in the catchment area of this project, there is no meteorological station of any level. Moreover, there are no flow data for the river near the project. Therefore, data for the hydro-meteorological analysis is taken from the nearby station and similar areas. Rainfall & temperature data are considered from Lalibela Meteorological station. In fact, this station is very close to the project area.

As per the data of the station, March – April are identified as high temperature periods whereas December–January are low temperature periods. The mean annual rainfall amount is less than1200mm (1995 - 2006 data) and most of it occurs from June to August.

### Rainfall Data

In order to compute the design flood for the spate structure, the daily maximum rainfall is collected from Lalibela Metrological stations with a record of 29 years.

### River flow data

No base flow data were recorded, during study the irrigation sachem is based on using floods that come from remote area.

### Upstream & Downstream utilization

Downstream of the proposed site, appreciable need for water is anticipated for locals and cattle provisions. Both at the upstream and downstream of the project site the farmers were constructed temporary intake to irrigate their farm. This shows that the farmers traditionally practicing spate irrigation by using summer floods &specially the flood comes at march and April .In addition to this the farmer detained the water for the supplementary irrigation during winter since in the project site there is scarcity of rain. Upstream fof the proposed intake there are three cascaded intakes at the same river which was constructed by IFAD I with an interval of 7 to 1o km repectively(from each other ) where there is no effect for the abstraction of flood since they all uses flood irrigation The maximum flood of the river is about 670.7m^3/sec.From this Gobu-2

takes 3m^3/sec,Gobu-1 takes 1.5m^3/sec.takes,Gobu-3takes 2m^3/sec&Gobu-4 takes3m^3/sec.

## Design Flood Analysis

For the design and analysis of structures to be constructed on the river, estimation of flood magnitude is an important task. This can be done using different techniques depending on the data available. For this particular case, there are no river flow data and hence the flood estimation is done using the rainfall data and applying SCS Curve Method.

### Design Rainfall computation

Based on the data of 24hr peak rainfall given in Table 1 the design rainfall, Rf is computed using Gamble’s Extreme Value Method.

### Outlier Test

Higher Limit, YH=Ymean+Kn*Sy,      Kn = 2.549 for 29 Years of data.

Lower Limit, YH=Ymean-Kn*Sy,     Kn = 2.549for 29Years of data.

Table 2-1: Outlier test analysis

| No | Year | Daily max rain fall | Descending order | Rank | Log x |
| --- | --- | --- | --- | --- | --- |
| 1 | 1978 | 57.5 | 92.000 | 1 | 1.759667845 |
| 2 | 1979 | 83.5 | 83.500 | 2 | 1.921686475 |
| 3 | 1980 | 33.5 | 82.500 | 3 | 1.525044807 |
| 4 | 1981 | 70.5 | 74.000 | 4 | 1.848189117 |
| 5 | 1982 | 92 | 73.100 | 5 | 1.963787827 |
| 6 | 1983 | 48.5 | 70.500 | 6 | 1.685741739 |
| 7 | 1984 | 38.35 | 68.200 | 7 | 1.583765368 |
| 8 | 1985 | 28.2 | 66.000 | 8 | 1.450249108 |
| 9 | 1986 | 38.5 | 61.800 | 9 | 1.58546073 |
| 10 | 1987 | 36 | 61.600 | 10 | 1.556302501 |
| 11 | 1988 | 45.9 | 57.500 | 11 | 1.661812686 |
| 12 | 1989 | 40.6 | 52.900 | 12 | 1.608526034 |
| 13 | 1990 | 43.25 | 51.100 | 13 | 1.635986112 |
| 14 | 1991 | 51.1 | 51.100 | 14 | 1.7084209 |
| 15 | 1992 | 36.2 | 48.500 | 15 | 1.558708571 |
| 16 | 1993 | 66 | 45.900 | 16 | 1.819543936 |
| 17 | 1994 | 82.5 | 43.250 | 17 | 1.916453949 |
| 18 | 1995 | 36.5 | 40.600 | 18 | 1.562292864 |
| 19 | 1996 | 68.2 | 38.600 | 19 | 1.833784375 |
| 20 | 1997 | 51.1 | 38.500 | 20 | 1.7084209 |
| 21 | 1998 | 38.6 | 38.350 | 21 | 1.586587305 |
| 22 | 1999 | 34.6 | 36.500 | 22 | 1.539076099 |
| 23 | 2000 | 52.9 | 36.200 | 23 | 1.683455668 |
| 24 | 2001 | 73.1 | 36.000 | 24 | 1.863917377 |
| 25 | 2002 | 61.6 | 36.000 | 25 | 1.789580712 |
| 26 | 2003 | 61.8 | 34.600 | 26 | 1.790988475 |
| 27 | 2004 | 29.2 | 33.500 | 27 | 1.465382851 |
| 28 | 2005 | 36 | 29.200 | 28 | 1.556302501 |
| 29 | 2006 | 74 | 28.200 | 29 | 1.86923168 |
| sum |  |  | 1509.700 |  | 49.07836855 |
| sta.n(Sy) |  |  | 17.939 |  | 0.146613021 |
| mean(Ymean) |  |  | 52.059 |  | 1.692357536 |
| Skewness |  |  | 0.630 |  | 0.200964311 |

Outliers are data values which are quite different from the others in the data series. They exist both in extreme values i.e. there is lowest & largest values out of limits formulated. The Causes of the outliers may be errors of measurement or recording and etc.  Since outliers have a significant effect on the results of the design rainfall computation, they have been discarded from the data set.  As per U.S Water resource council (1981) recommended, it is possible to check for outliers based on skew value. If value of skew lies between +0.4& -0.4 the outliers are checked for both lowest & highest values. In case of the particular Lalibela metrological station data values shown on table-1, the skew /g/=0.2 found between -0.4 and 0.4.Hence the data series has be checked for both cases of extremes.

- Higher Limit, YH  = Ymean + Kn*Sy,

Yh	=1.69235+2.549*0.146613

=2.066

- Lower limit,   Yl = 1.69235-2.549*0.1466= 1.31846

Upper limit of rainfall = 10^2.066 = 116.43mm

Lower Limit of rainfall = 10^1.31846 = 20.82mm

Lowest & highest value in the given data series is 28.2 &92.So, all of the data values lies between the lowest & a highest calculated value, Hence, there is no any data that can be rejected. All of them will be    used in the next design processes.

### Check for variance

After checking the outliers, the data should be checked for variability. For variability the formula used is

α=δ∩-1N*Mean*100%

Where, δn-1 = Standard deviation =17.4

N = Number of year =29

Mean = 52.06

α = Standard error

α=17.429*52.059*100%=6.39<10%  Acceptable

Therefore, the data shows no variability.

The design rainfall using SCS Method is given as

Where Rf  = Design rainfall

Rmean = average of all values of annual heaviest fall = 52.09 mm

σn-1 = standard deviation of the series = 17.4mm

, T= Return period = 50 years

Yn, Sn = constant found from Gumble’s extreme value distribution table for N= 29 Years

- Yn = 0.5353 and Sn = 1.1086

- Point Design Rainfall = 106.54 mm

The design rainfall at points for 50 years return period is 106.54 mm and the areal design rainfall is calculated in the following section.

### Peak Discharge Determination

#### General

The Gobu river is not gauged river. The design flood is calculated by using SCS unit hydrograph method. Thus, it is preferred to base the flood analysis on rainfall data, which are better both in quantity and quality of data. In the hydrologic analysis for drainage structures, it must be recognized that there are many variable factors that affect floods. Some of the factors that need be recognized and considered on an individual site by site basis are; rainfall amount and storm distribution; catchment area, shape and orientation; ground cover; type of soil; slopes of terrain and stream(S); antecedent moisture condition; Storage potential (over bank, ponds, wetlands, reservoirs, channel, etc.)

#### Peak flood analysis by SCS unit hydrograph method

Design flood is calculated SCS (The United States Soil Conservation Service). This method is widely adopted and more reliable method for flood estimation. The approach considers, watershed parameters, like Area, Curve number, and time of concentration.

#### Time of concentration (Tc)

Time of concentration has been calculated by taking the stream profile of the longest streamline and dividing it in to different elevation. Kirpich formula is adopted for computation.

Table 2-2: Determination of Time of Concentration

| Time of concentration |  |  |  |  |
| --- | --- | --- | --- | --- |
| Partial Distance/km/ | Cumulative distance/km/ | Elevation/m/ | Elevation Difference(m) | TC(hr) |
| 0 | 0 | 3400 | 0 | 0 |
| 5.886 | 5.886 | 2460 | 940 | 0.52638 |
| 11.658 | 17.544 | 1880 | 580 | 1.39585 |
| 8.44 | 25.984 | 1640 | 240 | 1.35006 |
| 18.736 | 44.72 | 1407 | 233 | 3.43019 |
|  |  |  |  | 6.70248 |

Tc=0.948L13H10.385+L23H20.385+…+Ln3Hn0.385

- Tc = 6.7	Since Tc>3hr., duration of excess rainfall difference, D = 1hr.

- Time to peak,

Tp=D2+0.6*Tc  = 4.5hr

- Base time,

Tb=2.67*Tp  = 12.1hr

- Recession time,

Tr=1.67*Tp  = 7.5hr.

#### Curve number (CN)

Curve number (CN) is achieved based on USSCS method by watershed characterization in terms of land cover, treatment, hydrologic condition and soil group. From the watershed analysis curve number at condition II =85.077. Since peak, rainfall is found at an antecedent moisture condition III state, this value has to be changed to antecedent moisture condition III.

- Conversion factor = 1.104

- CN Condition (III) = (Factor from Table x CN condition II) =85.077*1.104= 93.93. For detail analysis of the computation.

#### Area Rainfall

As the area of the catchment gets larger, coincidence of all hydrological incidences becomes less and less. This can be optimized by changing the calculated point rainfall to aerial rainfall. The conversion factor is taken from standard table that relate directly with the size of watershed area and type of the gauging station. (IDD manual)

For the case of Gobu irrigation project,

- Total watershed area = 314.46Km2

- Type of gauging station = Daily rainfall 24 hour.

- Aerial Rainfall = (Point Rainfall) x (Conversion factor)

#### Run off Analysis

Input data:

Design Point Rainfall = 106.54mm

Curve number at antecedent moisture condition III = 93.93

Catchment Area, A = 314.46Km2

Tc = 6.7hr, D = 1hr., Tp =4.5hr; Tb = 3.79hr; Tr = 7.5 hr.

Direct run-off,

Q=(I-0.2*S)2(I+0.8*S)

Where, I = Rearranged cumulative run-off depth (mm

S = Maximum run off potential difference,

S=25400CN-254

Peak run-off for incremental;

Qp=0.21*(A*Q)Tp

Where,   A = Catchment area = 314.46Km2

Tp = Time to peak (hr)

Q = Incremental run-off (mm)

Table 2-3: Runoff analysis

| Duration ,D (hour) | Design point rainfall | Rain Fall profile (%) | Rain Fall profile (mm) | Areal to point RF(%) | Areal rainfall (mm) | Rainfall increment (mm) | rank | Rearranged order | Re -arranged RF(mm) | Comm. Rainfall (mm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0-1 | 106.54 | 45 | 47.9412 | 53.00 | 25.409 | 25.409 | 1 | 6 | 5.007 | 5.007 |
| 1--2 | 106.54 | 59 | 62.8563 | 59.20 | 37.211 | 11.802 | 2 | 4 | 7.128 | 12.136 |
| 2--3 | 106.54 | 66 | 70.3138 | 63.06 | 44.339 | 7.128 | 4 | 3 | 10.440 | 22.575 |
| 3--4 | 106.54 | 71 | 75.6406 | 72.42 | 54.779 | 10.440 | 3 | 1 | 25.409 | 47.984 |
| 4--5 | 106.54 | 76 | 80.9674 | 73.84 | 59.786 | 5.007 | 6 | 2 | 11.802 | 59.786 |
| 5--6 | 106.54 | 79 | 84.1635 | 77.00 | 64.806 | 5.020 | 5 | 5 | 5.020 | 64.806 |

Table 2-5 Surface runoff computation

| RF Duration/D/ | Re -arranged RF/mm/ | Cumulative RF | curve no in cod 3 | RF &DR /S/ | DR cumulative | DR increment | Remarks |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 0-1 | 5.007 | 5.007 | 93.93 | 16.4285 | 0 | 0.000 |  |
| 1--2 | 7.128 | 12.136 | 93.93 | 16.4285 | 3.09836 | 3.098 |  |
| 2--3 | 10.440 | 22.575 | 93.93 | 16.4285 | 10.4175 | 7.319 |  |
| 3--4 | 25.409 | 47.984 | 93.93 | 16.4285 | 32.6854 | 22.268 |  |
| 4--5 | 11.802 | 59.786 | 93.93 | 16.4285 | 43.7729 | 11.088 |  |
| 5--6 | 5.020 | 64.806 | 93.93 | 16.4285 | 48.5541 | 4.781 |  |

Table 2-6 computation of peak runoff for each increment

| RF Duration/D/ | DR increment | Qp for 1mm runoff /m3/s/ | Qp for incremental run off m3/s/ | begin time | peak time | end time | remarks |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 0-1 | 0.00 | 14.61 | 0.0 | 0 | 4.5 | 12.1 |  |
| 1--2 | 3.10 | 14.61 | 45.3 | 1 | 5.5 | 13.1 |  |
| 2--3 | 7.32 | 14.61 | 106.9 | 2 | 6.5 | 14.1 |  |
| 3--4 | 22.27 | 14.61 | 325.3 | 3 | 7.5 | 15.1 |  |
| 4--5 | 11.09 | 14.61 | 162.0 | 4 | 8.5 | 16.1 |  |
| 5--6 | 4.78 | 14.61 | 69.9 | 5 | 9.5 | 17.1 |  |

Table 2-7: Hydrograph coordinates

| Time | Q1 | Q2 | Q3 | Q4 | Q5 | Q6 | base flow | Qtotal |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 0.00 |  |  |  |  |  | 0 | 0.00 |
| 1 | 0.00 | 0.00 |  |  |  |  | 0 | 0.00 |
| 2 | 0.00 | 10.01 | 0.00 |  |  |  | 0 | 10.01 |
| 3 | 0.00 | 20.03 | 23.66 | 0.00 |  |  | 0 | 43.69 |
| 4 | 0.00 | 30.04 | 47.31 | 71.98 | 0.00 |  | 0 | 149.33 |
| 4.5 | 0.00 | 35.05 | 59.14 | 107.96 | 17.92 |  | 0 | 220.08 |
| 5 | 0.00 | 40.06 | 70.97 | 143.95 | 35.84 | 0.000 | 0 | 290.82 |
| 5.5 | 0.00 | 45.27 | 82.80 | 179.94 | 53.76 | 7.727 | 0 | 369.49 |
| 6.5 | 0.00 | 45.58 | 106.93 | 251.92 | 89.59 | 23.181 | 0 | 517.20 |
| 7.5 | 0.00 | 39.58 | 107.66 | 325.33 | 125.43 | 38.636 | 0 | 636.64 |
| 8.5 | 0.00 | 33.58 | 93.50 | 327.56 | 161.99 | 54.090 | 0 | 670.71 |
| 9 | 0.00 | 30.58 | 86.41 | 306.01 | 173.82 | 61.817 | 0 | 658.64 |
| 9.5 | 0.00 | 27.59 | 79.33 | 284.46 | 163.09 | 69.853 | 0 | 624.32 |
| 12.1 | 0.00 | 11.99 | 42.50 | 172.40 | 107.30 | 55.52 | 0 | 389.71 |
| 13.1 | 0.00 | 6.00 | 28.33 | 129.30 | 85.84 | 46.27 | 0 | 295.74 |
| 14.1 |  | 0.00 | 14.17 | 86.20 | 64.38 | 37.02 | 0 | 201.76 |
| 15.1 |  |  | 0.00 | 43.10 | 42.92 | 27.76 | 0 | 113.78 |
| 16.1 |  |  |  | 0.00 | 21.46 | 18.51 | 0 | 39.97 |
| 17.1 |  |  |  |  | 0.00 | 9.25 | 0 | 9.25 |
| 18.1 |  |  |  |  |  | 0.00 | 0 | 0.00 |

Time in hours

Figure 2-2: Complex Hydrograph

From the analysis, the 50 year return period design run off is 670.7m 3/s

### Tail Water Depth Computation

Tail water depth of the river is equal to the flood depth and amount at the proposed spate site before construction of the spate. It is used to crosscheck peak flood estimated by the SCS unit hydrograph method with flood mark method and to see the flood feature after the hydraulic jump. During field visit, the flood mark of the river at the proposed spate site was marked based on dwellers information and physical indicative marks. The river cross-section was surveyed.

Table 2-8: Spate Site River Cross section Coordinate Data

| Pt. | East | North | Elevation | Partial distance | Com.dis |
| --- | --- | --- | --- | --- | --- |
|  | 1360500.891 | 571611.68 | 1405.28 | 0.00 | 0.0 |
|  | 1360497.433 | 571615.29 | 1405.22 | 5.00 | 5.0 |
|  | 1360493.975 | 571618.9 | 1405.16 | 5.00 | 10.0 |
|  | 1360493.146 | 571619.77 | 1405.15 | 1.20 | 11.2 |
|  | 1360491.422 | 571621.56 | 1405.11 | 2.48 | 13.7 |
|  | 1360490.509 | 571622.51 | 1405.1 | 1.32 | 15.0 |
|  | 1360487.041 | 571626.11 | 1405.04 | 5.00 | 20.0 |
|  | 1360484.664 | 571628.56 | 1405 | 3.42 | 23.4 |
|  | 1360483.573 | 571629.71 | 1404.99 | 1.59 | 25.0 |
|  | 1360480.105 | 571633.31 | 1404.95 | 5.00 | 30.0 |
|  | 1360476.637 | 571636.91 | 1404.91 | 5.00 | 35.0 |
|  | 1360473.169 | 571640.51 | 1404.87 | 5.00 | 40.0 |
|  | 1360469.701 | 571644.12 | 1404.83 | 5.00 | 45.0 |
|  | 1360466.232 | 571647.72 | 1404.79 | 5.00 | 50.0 |
|  | 1360462.764 | 571651.32 | 1404.73 | 5.00 | 55.0 |
|  | 1360462.427 | 571651.67 | 1404.73 | 0.49 | 55.5 |
|  | 1360459.296 | 571654.92 | 1404.68 | 4.51 | 60.0 |
|  | 1360455.828 | 571658.52 | 1404.63 | 5.00 | 65.0 |
|  | 1360452.36 | 571662.12 | 1404.57 | 5.00 | 70.0 |
|  | 1360448.892 | 571665.73 | 1404.6 | 5.00 | 75.0 |
|  | 1360445.526 | 571669.07 | 1404.81 | 4.75 | 79.7 |

Table 2-9: Stage discharge analysis

| Elevation(m) | Water depth(m) | Wetted area(m2) | Perimeter(m) | Hydraulic radius/R/ | slope | velocity /m/s | Discharge/m^3/s/ |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1404.7 | 0.0 | 0 | 0 | 0 |  | 0 | 0 |
| 1404.7 | 0.0 | 0.038 | 2.5 | 0.02 | 0.00682 | 0.1 | 0.0 |
| 1405.0 | 0.3 | 4.59 | 28.4 | 0.16 | 0.00682 | 0.6 | 2.8 |
| 1405.3 | 0.6 | 18.89 | 65.4 | 0.29 | 0.00682 | 0.9 | 17.0 |
| 1405.6 | 0.9 | 40.92 | 79.6 | 0.51 | 0.00682 | 1.3 | 54.2 |
| 1405.9 | 1.2 | 64.84 | 80.8 | 0.80 | 0.00682 | 1.8 | 115.6 |
| 1406.2 | 1.5 | 89 | 82.0 | 1.09 | 0.00682 | 2.2 | 194.2 |
| 1406.5 | 1.8 | 113.43 | 83.2 | 1.36 | 0.00682 | 2.5 | 288.1 |
| 1406.8 | 2.1 | 138.09 | 84.3 | 1.64 | 0.00682 | 2.9 | 396.2 |
| 1407.1 | 2.4 | 162.99 | 85.4 | 1.91 | 0.00682 | 3.2 | 517.9 |
| 1407.4 | 2.7 | 187.9 | 86.0 | 2.18 | 0.00682 | 3.5 | 653.4 |
| 1407.7 | 3.0 | 212 | 86.6 | 2.45 | 0.00682 | 3.8 | 795.3 |

Figure 2-3: Rating Curve

From the above stage discharge table and curve, the maximum flood level corresponding to the computed design peak discharge is 2.8m from the riverbed) and it is considered as the d/s high flood level i.e. expected at the spate axis before construction of the spate.

D/S HFL = 1407.09masl.

- Average river bed slope

Average riverbed slope of River is estimated by end area method. The surving data is taken at different points along the river channel around the headwork site. Surveying work done for 302m length. Then, average river center slope is considered as the riverbed slope. For comparison of the two procedures, refer the attached Excel file.

| Northing | Easting | Elevation | partial distance | cumulative distance | elevation difference /H/ | Area/A/ |
| --- | --- | --- | --- | --- | --- | --- |
| 1360568 | 571824 | 1403.29 | 0 | 0 | 0 | 0 |
| 1360555 | 571802 | 1403.45 | 25 | 25 | 0.16 | 1.999708 |
| 1360542 | 571781 | 1403.61 | 25 | 50 | 0.32 | 5.999983 |
| 1360530 | 571759 | 1403.78 | 25 | 75 | 0.49 | 10.12501 |
| 1360517 | 571738 | 1403.96 | 25 | 100 | 0.67 | 14.50004 |
| 1360504 | 571716 | 1404.15 | 25 | 125 | 0.86 | 19.12421 |
| 1360490 | 571695 | 1404.35 | 25 | 150 | 1.06 | 23.99951 |
| 1360477 | 571674 | 1404.53 | 25 | 175 | 1.24 | 28.74982 |
| 1360463 | 571653 | 1404.71 | 25 | 200 | 1.42 | 33.24762 |
| 1360462 | 571652 | 1404.73 | 2 | 202 | 1.44 | 2.480905 |
| 1360455 | 571639 | 1404.79 | 15 | 216 | 1.5 | 21.33228 |
| 1360450 | 571632 | 1404.84 | 9 | 225 | 1.55 | 13.34791 |
| 1360440 | 571617 | 1404.96 | 18 | 243 | 1.67 | 28.99961 |
| 1360436 | 571611 | 1404.99 | 7 | 250 | 1.7 | 11.77188 |
| 1360426 | 571599 | 1405.08 | 16 | 266 | 1.79 | 27.87839 |
| 1360420 | 571592 | 1405.12 | 9 | 275 | 1.83 | 16.49034 |
| 1360405 | 571572 | 1405.25 | 25 | 300 | 1.96 | 47.20595 |
| 1360404 | 571571 | 1405.26 | 2 | 302 | 1.97 | 4.077434 |
| sum |  |  |  | 302 |  | 311.3306 |

Hav=2*AL

Slope=HavL=0.00682

Figure 2-4: River profile

- Manning’s Roughness coefficient

The Manning’s roughness coefficient is taken from standard table based on the river nature. The river at the headwork site has braded feature and curving nature. The riverbanks are defined and relatively smooth. Manning’s roughness coefficient (n = 0.04) is adopted.

- Discharge of the river

Input data:

- Manning's roughness coefficient, n = 0.04

- Average river bed slope, S = 0.0068

- ,  	Where, R = Hydraulic radius = (Area/Perimeter)

- Q=V*A

# SECTION-II: HEADWORK DESIG

# HEADWORK STRUCTURES DESIGN

## Headwork Site Selection

For this project, a simple riverbed level spate structure has been proposed for diverting the water to the main canal. The headwork site geological surface and subsurface conditions have been investigated based on the nature of the proposed structure. At the site and immediate vicinity, the stream flows along genteel slope course and its bed is mainly covered with recently deposited alluvial sediments. These sediments are dominated with sand (specifically thick layer of fine sand materials). On the other hand, the banks of the stream at the headwork site are made up of different geologic materials; most part of the right bank is made up of old alluvial sediment of silt/clay materials at the top and thin layer of fine sand to the bottom of both banks.  The detail geologic nature of the banks, and bed of the stream along the headwork axis and immediate vicinity are described and their potential geotechnical influence on the proposed structures discerned/detected below, with remedial measures.

## River Geomorphology

This part emphasizes on the details of engineering geology and geo-technical aspects of the project focusing on strength and permeability characteristics of the underlying formations over which proposed structures lies, mainly at headwork and along main canal route. In addition to this, required natural construction materials' source areas and their suitability conditions are discussed.

### River Bed condition

At the proposed headwork site, the streambed or course is well defined, nearly straight, dry, and broad and shows plain surface due to recent sediment accumulations. The width of the stream bed reach up to 80m from bank to bank, Along the spate axis, the  bed is made up of one major geologic materials, as seen from surface observation. These are recently deposited alluvial sediments.

The streambed (from bank to bank) are totally covered with thick unit of alluvial deposits dominantly with fine sand. Due to genteel nature of the area including the streambed and loss nature both banks, during run off time, water covers wide area by eroding both bank. From surface geological understanding of the area and consultation of the local people, the sediment coverage area of the streambed and width of the stream increases from time to time. Hence, from local people consultation and deep cut found in the surrounding natural cut near the stream, it is difficult to found bedrock at shallow depth and the structure founded on thick deposit.

+

Figure 3-1: Riverbed at the proposed spate site

### River Bank condition

#### Right Bank

At the headwork site, the right bank is characterized by relatively moderate to genteel slope, having about 0.8m height from streambed. It reveals nearly genteel section within this height.  From visual observation of the natural cuts at the bank, it is made from Low Plastic dark brown color fine-grained silt (ML) throughout its channel.

Since this bank has made from loss, week soil materials have been affected by erosion/ flood cutting, which is widening the bank by forming nearly vertical slope. Such vertical slope configuration throughout the stream bank, which lead to instability or collapse. Since Main canal rout has been launched from this loss erodible and week old flood plain deposited, it is very important to design the bank slope to stable configuration, just by providing appropriate slope (indifferent from the present vertical slope) or other protection work(Gabion) to prevent  initial main canal portion near to the bank from farther widening of bank.

#### Left Bank

At the proposed headwork axis and immediate vicinities, the left bank forms relatively steep slope topography. From surface observation, the bank is made up of one basic geological units. The unit is old flood plain deposit of brown to dark color fine-grained low plastic silt (ML) soil. For geotechnical purpose, this unit are weak and loose horizon. The height of this bank varies from place to place. At the headwork site, it might reach up 1.5m.Hence due to erodible, weak and loss nature of the bank, the river morphology had changed from time to time. In order to make our structure functional for long time, it is necessary to provide some bank protection structures to prevent from farther change in flow channel of the stream.

## Sources of construction materials

During the site investigation, natural construction materials required for the construction of the various proposed engineering structures at the headwork and within the farmland have been assessed, and possible quarry sites and borrow areas have been identified within the vicinity of the area as much as possible. In addition to the identification, the quality, quantity, accessibility condition and ownership of each proposed production sites have also been studied and described in this report; on separate sub-sections below. The natural materials required for the construction of the proposed hydraulic structures include rock for masonry stones, aggregates (both coarse and fine), impervious soil for fill and/or lining, backfill soil, and water.

### Rock for Masonry and Crushed Coarse Aggregate

Quarry site that can be used for production of rock for masonry stone and crushed coarse aggregates has been assessed during the fieldwork session within the vicinity of the project area at economic distance for hauling. One possible major quarry site has been identified .This site is found just on asphalt rode from Kobo to Waja town at about 19km from headwork site. The rock is located in ---Kebele at its specific name Gara Lancha “Got”

Figure 2, Rock Quarry Site for Masonry Stone and Crushed Aggregates

At this quarry site, the available rock type is porphyritic basalt, which is magmatic in origin and occurs in the form of angular fragments of suitable sizes. It is light gray and coarse-grained volcanic rock. Naturally, this rock is strong as it is generally fresh except very thin cover of weathered zones, which need to be removed during rock dressing and before crushing. At the quarry site, fresh basalt rock units are found in large quantity that suffices the requirement of the project. From this site, rock of different sizes can be produced; ranging from large boulders to coarse gravels that masonry stone and coarse aggregate can be produced easily.

### Fine Aggregates

Borrow areas for fine aggregate or natural sand have been assessed starting from the project stream itself. Natural deposits of such materials could not be found when assessed within the beds of the stream in the project area; rather very fine sand deposit are found covering almost the entire bed of the Gobu stream. Seeing to this nature of the stream, other distant streams have been explored to identify the best source areas for fine aggregate or natural sand that can be used for this particular project. During exploration of this natural sand, at a distant one stream was identified as a possible source of sand. The stream is known as ‘Horemat’. It is located at about 21km from the project site.

Both visual investigation and laboratory testing of the sand deposit found in the proposed area have been done. In the area good quantity, which suffices the need of this project, is available. The deposit is dominated by sand sized transported alluvial sediments, though some amount of fines and over sizes are observed. The individual grains of sand are mainly originated from basalt rock and reveals high crushing resistant that the sand has good quality in this respect.

### Water

Water for construction purposes cannot be found from the project stream, Gobu, itself in winter. The stream is seasonal that if there is rain in the surrounding area, there will be run off in the stream. Unless otherwise, there will not have any drop of water throughout the year. During this field time, the stream was dry throughout its flowing channel. During construction water fetched from nearest site bore hole .

## Headwork Type Selection

Looking the availability of natural construction materials and considering the river features and expected flood amount, cyclopean concrete 30cm capping is chosen.

- Spate Height Determination

The following major factors have been seen in determining the spate crest level:

- Maximum  command area elevation

- Deriving head of the spate structure

- Main canal slope

- Loss

- Lowest Point of river center

#### Base flow of the River

The study team has assessed that the stream is used for irrigation along its entire at the proposed spate site where farmers are using the stream for traditional SSI (Irrigation Infrastructure Report). As the study team reports the river has no base so that our modern engineering design is based on the floods that come from remote area during summer especially during March and aprile.In addition to this the scheme also serves for supplementary case.

- Q = 670.71m3/sec

- q = 8.38m3/s/m

- Silt factor, f =1.76*(d50)0.50

f= 0.485mm

- D50=32mm for medium sandy soil

- R =1.35*(q2/f)(1/3)

R=1.35*(q2/f)(1/3) = 1.27 take R = 2.1m

Bottom level of Bed Bar= 1405.63-2.1m=1403.53

Take the Crest level of the bed bar cut off as 1404.69+0.4=1405.09m

# Intake Dimensions

The off take size is fixed taking in to consideration spate irrigation during the dry season when the base flow is small and supplementary irrigation for ‘belg’ rainy season to maximize the command.

According to the agronomy report the total available water (TAW) required for 1m root depth is 140mm. The project efficiency taken to be 51% the volume of water required for the command and the discharge to be diverted is determined as follows:

Q =2.77*Ai*W/ å t

Where:

Qi	is the discharge (m3/s)

Ai	is the irrigable area (ha)

W	is the depth of application

å	is the application efficiency (assume 60%)

t	is the time of application (hrs.)

Gobu River is a perennial river during rainy season the base flow increase a great deal therefore the coming flood will have a longer time of recession.

Depth of application is given by W(RAW)=r*D*(FC-WP)*d= r*D*TAW*d

Where: r=apparent unit weight of soil (take 2)

D=Root depth (take 1m)

TAW=Total available water (150mm/m for loam soil)

d=depletion factor (0.6)

Therefore depth of application is =2*1*0.15*0.6=0.18m

For a command area of 400ha and time of recession taken to be 9 days. And for depth of application of 0.18m.

Q=2.77*0.18*400/ (0.60*8*24)

Q=1.730m3/sec. Take 3m3/sec

Hence our canal capacity and off take size is fixed to convey and pass 2m3/sec of discharge.

The intake dimension is computed using rectangular notch formula given by:-

Q = 2/3Cd*(2*g) 1/2B*H (3/2)

Where: Q=diverted discharge, 2m3/sec

Cd=Coefficient of discharge, Cd=0.6

B=Width of intake, 1.7m

H=Height of the intake

G=Acceleration due to gravity

Therefore for a width of 1.1

H= (Q*3/ (2*Cd*(2*g)1/2*B))2/3

H= 1.1m, provide 1.7m

Therefore, provide intake dimension 1.7X1.1 width and height respectively.

# Approach canal

For this spate irrigation system, 48.5m approach canal is designed to prevent the siltation problem at the off take. The approach canal act like basin and sediments silted up before reaching at the off take.

### Under Sluice, and Canal outlet

#### Under sluice

The under sluice intake dimension is computed using rectangular notch formula given by:-

Q = 2/3Cd*(2*g) 1/2B*H (3/2)

Where: Q=diverted discharge, 2m3/sec

Cd=Coefficient of discharge, Cd=0.6

B=Width of intake, 1.7m

H=Height of the intake

G=Acceleration due to gravity

Therefore for a width of 0.9m

H= (Q*3/ (2*Cd*(2*g)1/2*B))2/3

H= 1.2m, provide 1.7m

Therefore, provide intake dimension 1.7X1.2 width and height respectively.

# Escape Gate

The water that coming through the approach canal may exceed the full supply level. During this time excess water removed through this gate. Its size is 1m*1.2m, height *width respectively.

#### Canal outlet level

The head regulator is provided on the right side .The sill level of this head regulator is fixed from different angle observations. The main conveyance system is more than 3.1km that passes across the contour. Sill level is fixed to be 1404.17m.

A) Sill Level Determination

The sill level of spate out let is based on the following criteria:-

- Maximum elevation of command area =1408.2 and minimum elevation 1387m

- Main canal alignment;-Taking into consideration the cost effectiveness of alignment, main canal alignment has been made on topo.

- The capacity of main canal

- The capacity of main canal which is based on the duty, command area, irrigation efficiency & operation hours in a day.

And the design discharge for irrigation that the canal has to carry is;

From    Qs=AV, Where, A=wetted cross sectional area (m^2)

V= velocity of flow in canal (m/s) and it is given by:-

V=1 n×R23×S^12 Where n= Manning’s roughness coefficient

R= hydraulic radius = A/P, P= Wetted perimeter & S= longitudinal slope of main canal.

Based on the above principle, by iteration method on MICROSOFT XL PROGRAM, the dimension of the canal is computed as shown in table below.

| Canal name | Chain age | A (ha) | Qr (m3/s) | m (H:V) | B (m) | D (m) | Fb (m) | Area | Perimeter | R=A/P | S | N | V | Q d. (m3/s) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| MC-1 | 1-145 | 400 | 1.74 | 1 | 1.7 | 0.7 | 0.2 | 1.19 | 3.1 | 0.38 | 0.0125 | 0.018 | 3.28 | 3.90 |
| MC-1 | 145-409 | 400 | 1.74 | 1 | 1.4 | 0.7 | 0.2 | 0.98 | 2.8 | 0.35 | 0.005 | 0.018 | 1.95 | 1.91 |
| MC-1 | 409-925 | 379 | 1.65 | 1 | 1.3 | 0.7 | 0.2 | 0.91 | 2.7 | 0.34 | 0.00555 | 0.018 | 2.01 | 1.82 |
| MC-1 | 925-1200 | 339 | 1.47 | 1 | 1 | 0.6 | 0.2 | 0.6 | 2.2 | 0.27 | 0.0025 | 0.018 | 1.17 | 0.70 |
| MC-1 | 1200-1671 | 221 | 0.96 | 1 | 1 | 0.6 | 0.2 | 0.6 | 2.2 | 0.27 | 0.004 | 0.018 | 1.48 | 0.89 |
| MC-1 | 1671-1950 | 181 | 0.79 | 1 | 1 | 0.5 | 0.2 | 0.5 | 2 | 0.25 | 0.006666 | 0.018 | 1.80 | 0.90 |
| MC-1 | 1950-3100 | 181 | 0.79 | 1 | 1 | 0.5 | 0.2 | 0.5 | 2 | 0.25 | 0.00588 | 0.018 | 1.69 | 0.85 |

Capacity determination and size determined from the above.

#### Breast Wall and Operation Slab

Dimension of brust wall1.7mX0.2mX1.7 is provided

And operation slab 1.9m*1m*0.2m is provided.

Minimum area of reinforcement 0.15% is taken along the respective direction

=0.0015*1200*150 =270mm2

Taking 50mm cover thickness, the effective depth is de=D-(50+12/2)=94

And spacing 250<3*de =282.

Therefore, 12@250 mm C/C spacing is provided.

Area of steel 678.24mm2 >Minimum area of reinforcement, 0.15% of the concrete ie202.5mm2.

A vertical raised gate is designed for the head regulator and under sluice. This gate slides over the breast wall-using spindle during opening and closing.

For easy operation of these gates, operation slab is provided. The size of the operation slab is fixed from the point of construction and free movement. The size of the operation slab is shown in the drawing for both head regulator and under sluice with thickness 0.2m.

The thickness of the breast wall is also the same as that of the operation slab. The nominal thickness is fixed from the point of construction rather than the imposed load. The thickness required for the imposed load is less than the nominal value and treated as cantilever retaining wall. For the breast wall, the minimum reinforcement area is taken as 15% along the respective direction.

Hence	Asteel = As=3.14*124=113.4mm^2

25% Grose area of the section=0.0012mm^2

Provide 12 c/c 200 mm for under sluice, escape and off take head regulator.

Considering cover thickness of 50 mm, effective depth = 50+12/6=56mm

The dimension of the operation slab is 1.9m*1m to give better safety for operators for both cases.

The actual arrangement of angle irons, spindles, shafts and operation slab including other components is shown on the design drawing.

For each arrangement and further information, refer to the design drawing.

# Trash rack

Trash rack is provided to exclude boulders and tree trunk from entering inside the pocket and clog the gates. Therefore for gobu intake Trash rack up to high flood level is provided.

1.2m height trash rack with 12mm diameter steel spaced at 100mm horizontally and vertically is provided.

At the top a horizontal 2m and 100mm diameter stiffening steel pipe is provided to anchor with walls.

# Retaining Walls

3.4.2.5.1Design of protection works and guide w



The guide wall height is fixed by deducting total energy level from lowest riverbed level (LRBL) and adding free board as follows:

U/S HFL=1407.449

Minimum bed level=1404.69

Therefore Height of bank=U/S HFL+Fb-RBL=2.8m and 1m depth below riverbed level.

A guide wall 2.8 m height is provided.

#### Stractural stablity

Unite weight of masonry=25KN/m3

Angle of repose=300

Unite weight of concrete



#### Figure; upstream and downstream retaing wall

Dimension

H=2.8m, D=0.5m, bf=0m, b=0.4m, bh=0m, B=2.5m

##### 

| Parameters | Vertical forces | Horizontal forces | Arm length | Stabilizing moments M+ | Overturning moment M- |
| --- | --- | --- | --- | --- | --- |
| pa= Weight of the soil horizontal(active pressure) |  | 20.655 | 0.9 | 0 | 18.5895 |
| Ww1= weight of wall | 0 |  | 0 | 0 |  |
| Ww2= weight of wall | 75.6 |  | 1.66666 | 126 |  |
| Uplift | 33.1088 |  | 0.833333 |  | 27.59063 |
| Sum | 75.6 | 20.655 |  | 126 | 46.18013 |

| Over turning | ∑M+/∑M-≥1.5 | Fo | 2.77 |  | ok |
| --- | --- | --- | --- | --- | --- |
| sliding | ∑V/∑H>1.5 | Fs | 2.3388 |  | ok |
| Tension | x=∑M/∑V |  | 1.06 |  |  |
|  | e=¦b/2-x¦, < B/6 | e | 0.19 |  | ok |

## Bill of Quantity and Cost Estimation

The quantities of the various items have been worked out as per the final design and final drawings prepared for the scheme. The unit rates analysis has been carried out based on the data available in the vicinity of the project area.

|  | Summery Gubu-4 |  |
| --- | --- | --- |
| Bill No. | Description | Amount (Birr) |
| 1 | General Items | 1,420,340.41 |
| 2 | Head work (Weir body, Outlet structures) | 7,406,247.38 |
| 3 | Infrastructures | 20,887,477.40 |
|  | Grand Total | 29,714,065.20 |
|  | Command Area(ha) | 400 |
|  | Per hectare Cost | 74,285.16 |

Bill No. 1- General Items(Gobu-4)

| Item | Description | Unit | Quantity | Rate | Amount |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | Birr | Birr |
| 1.1 | Allow for mobilization | L.S | 1 | 262,450.92 | 262450.9228 |
| 1.2 | Allow for demobilization | L.S | 1 | 209,960.74 | 209960.7383 |
| 1.3 | Camp (Staff residence, Store, Toilet & Shower,) | L.S | 1 | 650,000.00 | 650000 |
| 3 | Provide project indicator post starting from the construction time | LS | 1 | 9,225.90 | 9225.9 |
| 4 | Provision of as built drawings for the project | LS | 1 | 26,197.60 | 26197.6 |
| 5 | Allow for temporary access road to site | km | 1 | 262505.25 | 262,505.25 |
|  | subtotal for general item |  |  |  | 1,420,340.41 |

Head work

| Bill No. 2 Head Work Structure |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| S/N | Description | Unit | Quantity | Unit Rate | Total amount(birr) |
| 1 | Bed bar |  |  |  |  |
| 1.1 | Excavation ordinary(soil) | m3 | 5500 | 90 | 495000 |
| 1.2 | compacted Back fill with selected material | m3 | 120 | 114 | 13680 |
| 1.4 | C-20 concrete(1:2:3) | m3 | 68.75 | 2900 | 199375 |
| 1.4 | ф12mm bar | Kg | 266.5 | 47 | 12440.22 |
| 2 | operational slab & breast wall for Escape& off take and under sluice |  |  |  |  |
| 2.1 | Excavation(soil) | m3 | 75.4 | 90 | 6786 |
| 2.2 | backfill well compacted with selected material | m3 | 4.272 | 114 | 487.008 |
| 2.3 | C-20 concrete 20cm thickness | m3 | 14.72 | 2900 | 42688 |
| 2.4 | ф12mm bar | kg | 52.73 | 47 | 2461.4364 |
| 2.5 | Cyclopean concrete(60% C-20,40% graded stone) | m3 | 25.3 | 1900 | 48070 |
| 2.6 | plastering (1:3) | m2 | 45.3 | 122 | 5535.66 |
|  | Sub total |  |  |  |  |
| 3 | Approach canal |  |  |  |  |
| 3.1 | Excavation(soil) | m3 | 570.6 | 90 | 51354 |
| 3.2 | Back fill with selected material | m3 | 177.3 | 114 | 20212.2 |
| 3.3 | Cyclopean concrete(60% C-20,40% graded stone) | m3 | 86.8 | 1900 | 164920 |
| 3.4 | plastering (1:3 ratio) | m2 | 356.4 | 122 | 43552.08 |
| 3.5 | Masonry work | m3 | 628.2 | 1,850.00 | 1162170 |
|  | Sub Total |  |  |  |  |
| 4 | Masonry retaing wall |  |  |  |  |
| 4.1 | Excavation(soil) | m3 | 84.436 | 90 | 7599.24 |
| 4.2 | Back fill with selected material | m3 | 14.948 | 114 | 1704.072 |
| 4.3 | Masonry (1;4) ratio | m3 | 160.3 | 1,850.00 | 296555 |
| 4.4 | plastering (1:3) | m3 | 82.8 | 122 | 10118.16 |
|  | Sub Total |  |  |  |  |
| 5 | Retaing gabion works |  |  |  |  |
| 5.1 | Excavation Ordinary(soil) | m3 | 1038 | 90 | 93420 |
| 5.2 | Gabion mattress | m3 | 657.4 | 1400 | 920360 |
| 5.3 | gabion box | m3 | 2662.81 | 1400 | 3727934 |
| 5.3 | Back fill with selected material | m3 | 210.222 | 114 | 23965.308 |
|  | Sub Total |  |  |  |  |
| 6 | Gates |  |  |  |  |
| 6.1 | Intake gate installation consist of:- | No | 1 | 18,620.00 | 18620 |
|  | 8mm thickness sheet metal1.7m X 1.1m |  |  |  |  |
|  | 50X50X8mm angle iron, 1.7m length |  |  |  |  |
|  | n12mm bar for anchorage, 1.50Kg |  |  |  |  |
|  | n10mm bar for anchorage, Angle iron with the masonry wall 2.0Kg |  |  |  |  |
| 7 | under sluice gate |  |  |  |  |
| 7.1 | Intake gate installation consist of:- | No | 1 | 18,620.00 | 18620 |
|  | 8mm thickness sheet metal, 1.7m X 1.2m |  |  |  |  |
|  | 50X50X8mm angle iron, 1.6m length |  |  |  |  |
|  | n12mm bar for anchorage, 1.50Kg |  |  |  |  |
|  | n8mm bar for anchorage, Angle iron with the masonry wall 2.0Kg |  |  |  |  |
| 8 | Escape gate | No | 1 | 18,620.00 | 18620 |
| 8.1 | consists of 8mm thickness sheet metal,1m*1.2m |  |  |  |  |
|  | 50X50X8mm angle iron, 0.7m length |  |  |  |  |
|  | Sub Total |  |  |  |  |
|  | TOTAL |  |  |  | 7,406,247.38 |

# SECTION-III: IRRIGATION AND DRAINAGE SYSTEMS INFRASTRUCTURE

# IRRIGATION AND DRAINAGE SYSTEMS DESIGN

## Irrigable Area Description

### Topography

The nature of topography can influence soils of an area in many ways. In irrigated agriculture, topography influences the choice of irrigation method, the labour requirement, the irrigation efficiency, the cost of land development, the problems of drainage, the hazards of erosion, the range of possible crops that can be grown  and the size/shape of fields to put under cultivations. Furthermore, the suitable soils occur on a range of slopes up to 12%. A slope of land greater than 12% is considered permanently unsuitable for irrigation for all land use types (LUTs). Based on secondary data obtained from Adis Kign (05) Kebele Agricultural Development Office, the topography feature of the kebele contains 100% plain. On the other hand, based on slope gradient classes which have developed from topographic survey data of the command area, using ArcGIS 10.1 Software, the command area has consisted of level to gently sloping topographic feature. In general, the slope gradient classes of the command area range from 0.2% to 5%.

From the abovementioned evidences and the agro-ecological suitability of the area, development of modern spate irrigation scheme is believed to be encouraging for the production of drought tolerant crops and bring about rapid and sustainable development in the project area

### Soil characteristics

Soil properties (physical, chemical, etc.) greatly influence the growth and thereby yield of crops which is grown. For the determination of important micro/macro elements for the required fertilizer application purposes a composite nature of sampling has been made with 0 - 30 cm and 30 - 60 cm soil depths. The laboratory results of physical and chemical analysis of the composite soil samples has presented in Table 3.

Table 4: Summary of physical and chemical analysis of soils of the command area

| Soil auger composite sample no | Soil depth (cm) | % Sand | % Clay | % Silt | Textural Class | pH (H2O) (1:2.5) | ECe, (dS/m) | % OC | % OM | % N | Av. P (mg/kg, ppm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| A-1 | 0-30 | 44 | 12 | 44 | Loam | 7.4 | 0.193 | 0.95 | 1.64 | 0.08 | 13.21 |
| A-2 | 30-60 | 40 | 10 | 50 | Loam | 7.8 | 0.122 | 0.84 | 1.44 | 0.07 | 5.48 |
| B-1 | 0-30 | 48 | 14 | 38 | Loam | 7.7 | 0.173 | 0.57 | 0.98 | 0.05 | 18.36 |
| B-2 | 30-60 | 44 | 18 | 38 | Loam | 8.0 | 0.154 | 0.95 | 1.64 | 0.08 | 4.58 |
| C-1 | 0-30 | 34 | 12 | 54 | Silt loam | 7.9 | 0.192 | 0.88 | 1.51 | 0.08 | 11.19 |
| C-2 | 30-60 | 42 | 16 | 42 | Loam | 8.0 | 0.118 | 0.88 | 1.51 | 0.08 | 9.94 |

Note: At field condition, it has estimated that about 90% of soils of the command area are loam texture.

- Soil physical properties

- Texture

Soil texture refers to the relative proportion of sand, silt and clay in a mass of soil. Texture is important in that it helps to determine the capacity of the soil to retain moisture and air as both are necessary for plant growths. Soils with greater proportion of large particles are well aerated and allow water to pass through the soil more quickly.

Based on field observations and soil laboratory results, from soil samples collected from command area and submitted to ADSWE soil lab, the command area has predominantly loam textured soil which can be classified as well drained soil.

- Effective soil depth

According to FAO, soil depth is categorized as follows:

Very shallow	 		   < 30 cm

Shallow		 	30 – 50 cm

Moderately deep	  	50 – 100 cm

Deep			           100 – 150 cm

Very deep		  	 > 150

The effective soil depth of the command area was measured during field working period; and hence, all the study area soils are categorized as very deep soils (>1.5 meters).

- Soil chemical characteristics

- Soil reaction (pH)

The pH of soils can provide overall information of soils condition for crop growth and for crop selection. For these purposes soils reaction was measured in a 1:2.5 (H2O) soil suspension.

Table 5: General rating for soil pH

| Rating | pH | General Interpretation |
| --- | --- | --- |
| Very high | >8.5 | Ca and Mg unavailable, may have high Na, possible B toxicity otherwise as below |
| High | 7.0-8.5 | Decreasing availability of P and B above 7.0 increasing liability of deficiency of Co, Cu, Fe, Mn, Zn. |
| Medium | 5.5-7.0 | Preferred range for most crops |
| Low | < 5.5 | Acidic soil. Possible Al toxicity and excess Co, Cu, Fe, Mn, Zn. Deficiency in Ca, K, N, Mo, P, S |

The pH values of soils of the command area ranges from 7.4 to 8.0 for pH (H2O) soil suspension test in the top 0-60cm soil depth. Based on the soils Lab result (Table 3) and pH rating (Table 4) the soils of the command has found at high pH levels. Accordingly, soils of the command area is moderately suitable for most of the selected crops to be grown. Hence, it needs to grow hight pH tolerant crops. Generally, most crops grow satisfactorily on soils with a pH ranging from 6.2 to 8.3. Where, soils with a pH of 8.3 or greater than these values usually have high sodium contents. Conversely, soils with a pH below 5.5 are strongly acidic soils and limit the availability of essential elements (N and P).

- Electrical conductivity (ECe)

The best method for assessing soil salinity is to measure the electrical conductivity of the saturated soil extract, the ECe. Salinity effects on plants, as measured by the electrical conductivity of the saturated extract, ECe, in decisiemens per meter, dS/m, are indicated in Table 5.

Table 6: Electrical conductivity expressed in dS/m

| Rating | EC (dS/m) | Crop Reaction |
| --- | --- | --- |
| Non-saline | < 2 | Salinity effects mostly negligible |
| Slightly saline | 2- 4 | Yields of very salt-sensitive crops may be restricted |
| Moderately saline | 4 - 8 | Yield of many crops restricted |
| Strongly saline | 8 - 16 | Only salt-tolerant crops yield satisfactorily |
| Very strongly saline | > 16 | Only a few very salt-tolerant crops yield satisfactorily |

Source: Based on Fundamentals of Soil Science, 8th Edition.

The effect of salts on plants is mainly indirect, that is, the effect of salt on the osmotic water potential. Decreasing water potential due to salt reduces the rate of water uptake by roots and germinating seeds.

Based on the soil lab result (ADSWE, 2014), soils of the command area has ECe values ranging from 0.118 to 0.193 dS/m, which indicates a non-saline effect on the selected crops as presented in Table 3 and 5.

- Total Nitrogen

The general rating for the total nitrogen according to the Kieldahl’s method is as indicated in Table 6.

Table 7: Interpretation of total Nitrogen

| N (%) | Rating |
| --- | --- |
| > 1.0 | Very high |
| 0.5 - 1.0 | High |
| 0.2 - 0.5 | Medium |
| 0.1 - 0.2 | Low |
| < 0.1 | Very low |

Total nitrogen values of the command area soils ranges from 0.05 to 0.08% in the top 0-60cm soil depths, which indicates very low level of N in the soil profile. Thus, N source fertilizers response is very high (see Table 3 and 6).

- Available phosphorus

Available phosphorus (P) has been determined by the Olsen method of bicarbonate extraction. Thus, soils of the command area have showed high available P values. The available P range for the command soil is from 4.58 to 18.38 mg/kg or PPM in the top 0-60cm soil depths. Therefore, the soil lab result has indicated that soils of the command area are medium to high in available P and hence P source fertilizers response may not be more likely (Table 3 and 7).

Table 8: Interpretation of phosphorus by Olsen's method

| Available P (ppm) | Value | Comment |
| --- | --- | --- |
| > 15 | High | Fertilizer response unlikely |
| 5 –15 | Medium | Fertilizer response probable |
| < 5 | Low | Fertilizer response most likely |

- Organic matter (OM) and organic carbon (OC) content

Organic matter is a good reserve for plant nutrition. Almost all life in the soil is dependent on OM for nutrients and energy. The importance of OM for plant growth is so vital. The primary or original source of soil OM is the production of the primary producers (the higher plants). This organic material is subsequently consumed and decomposed by soil organisms. The result is the decomposition and the accumulation of OM in soils that has great diversity and a highly variable composition.

During decomposition of plants and animals residues, there is a loss of carbon as carbon dioxide and the conservation and reincorporation of nitrogen into microbial products, which eventually are incorporated into humus. As a consequence, humus contains about 50-60% carbon (C) and 5% nitrogen (N) producing a C: N ratio of 10 or 12 to 1. The more decomposed the humus is the lower or narrower will be the C: N ratio. Humus is a good source of biologically available nitrogen. Humus is also a significant source of sulfur and phosphorus.

Based on the soil lab results, soils of the command area OC content ranges from 0.57% to 0.95% (average 0.76%) in 0-60cm soil depths, which indicates a very low contents of OC; whereas the OM content ranges from 0.98% to 1.64% (average 1.31%) in the same depths of soils, which is indicating low contents of OM (Table 3 and 8). The ratio of OC to OM content, for soils of the project area, is nearly 1:1.724, which is in the optimum ratio and this ratio can also matches to that of FAO’s. According to FAO, the optimum ratio of OC to OM content is 1:1.7–2.

Table 9: Interpretation of OM and OC contents

| Parameter | Range | Rating |
| --- | --- | --- |
| Organic matter (%) | >6 | Very high |
|  | 4–6 | High |
|  | 2–4 | Medium |
|  | 1–2 | Low |
|  | <1 | Very low |
| Organic carbon (%) | > 20 | Very high |
|  | 10-20 | High |
|  | 4-10 | Medium |
|  | 2-4 | Low |
|  | <2 | Very low |

  - Water Resources

Considering surface water resource only one seasonal river (i.e., Gobu) is crossing through the kebele where the project area is found. Thus, about --- hectares of land is under cultivation using flood water from the wadi Gobu, mainly using community spate irrigation. Moreover, modern ground water irrigation project is under way of expansion in the project area and hence about 125 hectares of land is under irrigation so far. In general, due to erratic rainfall distribution and production shortfalls in the area a thorough assessment and development support for existing spate irrigation potential is critical.

  - Natural Vegetation Cover

With regard to the vegetation cover of the kebele, the major forest trees naturally and artificially grown are: Kurkura, Bedena, Wacho, Kinchib, Moringa, and Girar; and bushes are: Kinchib, Kurkura, and Girar. However, Kinchib is the dominant tree and bush plant found in the vicinity of the command area.

### Existing Irrigation Practices in the Project Area

The existing low productivity or production capacity of crops enforced farmers to develop traditional irrigation practices in the project area (i.e., primarily due to erratic/unreliable nature of rainfall, uneven distribution of rainfall, crop pests attack, and population pressure). Therefore, irrigated agriculture has gradually developing following rainfall abnormalities and scarcity of farmlands. In the project area irrigated crop production has been practicing using water from the Wadi Gobu by constructing traditional Intake structures. Accordingly, the project area is practicing two seasons crops production. The first is rainfed crop production which is the main income base and constitutes about 96% of the annual cultivated land and about 79% of the annual income; and the second is irrigated crop production (using traditional & modern ground water) that accounts about 4% of the whole cultivated land area and 21% of the annual gross production.

According to the 2013 annual irrigated crops area and yield estimation data, obtained from Adis Kign (05) kebele Agricultural Development Office, a total of 125 hectares of land was covered by irrigated crops and about 16,971.8 quintals was produced in the kebele. From the woreda total irrigated land area it accounts about 1.5% area coverage (see Table 12). However, the existing traditional irrigation is not diversified because of different technical and social reasons. Accordingly, the productivity of crops is comparatively low due to traditional management practice and low input uses (see Table 12). In the project area the widely used methods of irrigation are furrow and flooding for almost all crops.

Table 10: Existing traditional irrigated crop production data for year 2013

| S/N | Crop type | Kebele 04 in 2013 |  |  | Raya Kobo Woreda in 2013 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Cropped Area (ha) | Production (qt) | Productivity (qt/ha) | Cropped Area (ha) | Production (qt) | Productivity (qt/ha) |
| 1 | Onion | 104 | 16,640 | 160 | 3309.5 | 728090 | 220 |
| 2 | Pepper | 0 | 0 | 0 | 320 | 4200 | 15 |
| 3 | Cabbage | 0 | 0 | 0 | 442.2 | 39982 | 150 |
| 4 | Tomato | 0 | 0 | 0 | 504.6 | 100920 | 200 |
| 5 | Carrot | 0 | 0 | 0 | 79.35 | 634.8 | 8 |
| 6 | Lettuce | 0 | 0 | 0 | 79.35 | 7141.5 | 90 |
| 7 | Barley | 0 | 0 | 0 | 11 | 275 | 25 |
| 8 | Wheat | 0 | 0 | 0 | 14.5 | 290 | 20 |
| 9 | Maize | 0 | 0 | 0 | 406.5 | 27080 | 80 |
| 10 | Teff | 21 | 331.8 | 15.8 | 2601 | 39866 | 14 |
| 11 | Chick pea | 0 | 0 | 0 | 397 | 6352 | 16 |
| 12 | Lentil | 0 | 0 | 0 | 29 | 261 | 9 |
| 13 | Field pea | 0 | 0 | 0 | 11 | 121 | 11 |
| 14 | Avocado | 0 | 0 | 0 | 3 | 450 | 150 |
| 15 | Papaya | 0 | 0 | 0 | 12 | 3000 | 250 |
| 16 | Orange | 0 | 0 | 0 | 10 | 1500 | 150 |
| 17 | Mango | 0 | 0 | 0 | 94 | 18800 | 200 |
| 18 | Banana | 0 | 0 | 0 | 35 | 5250 | 350 |
| 19 | Sugarcane | 0 | 0 | 0 | 158 | 197 | 1250 |
| 20 | Hops | 0 | 0 | 0 | 5 | 750 | 150 |
|  | Total | 125 | 16,971.8 |  | 8,522 | 985,160.3 |  |

Source: Adis Kign (05) Kebele and Raya Kobo Woreda Agricultural Development Offices.

Concerning the irrigation development supporting services, alike other woredas of the region, irrigation and drainage work process has already established under Raya Kobo Woreda Agriculture Development Office. Moreover, irrigation expert has assigned in the project kebele.

During public meeting, carried out at time of field survey, farmers of the project kebele have showed keen demand/interest on the proposed Gobu 4 Spate Irrigation Scheme.

    - Experience of Private and State Agricultural Enterprises

There is no notable private investment farming companies and state agricultural enterprises within the kebele where the project area is found. However, there are private agricultural investors (e.g. Getahun, KIWO, etc) and state owned agriculture enterprise (Zeleke farm) within the woreda.

## Irrigation Water Requirement

### Crop Water Requirement (CWR)

The calculation of crop water requirement is a very important aspect for planning of any irrigation project. Several methods and procedures are available for this. The Food and Agriculture Organization (FAO) of the United Nations has also made available several publications on this subject and other issues related with this. The computer program available in FAO Irrigation and Drainage Paper No. 56 “CROPWAT” has been used for the calculation of Crop Water requirement. This program is based on Penman-Monteith approach and procedures for calculation of crop water requirements and irrigation requirements are mainly based on methodologies presented in FAO Irrigation and Drainage Paper No. 24 “Crop Water Requirements” and No. 33 “Yield Response to Water”.

The corresponding values of the crop water requirements of the proposed crops of the project are presented in the Agronomy Study of the same project.

### Irrigation efficiency (Ep)

To complete the evaluation of the demand, the efficiency of the water distribution system and of application must be known.

The gross requirement of water for irrigation system is very much dependent on the overall efficiency of the irrigation system, which in turn is dependent on several factors: Method of irrigation, type of canal (Lined and/or Unlined), method of operations (simultaneously and continuous or rotational water supply), and availability of structures (for controlling and distribution and measuring and monitoring).

On the basis of these factors, the project has planned to impose surface (furrow & basin) irrigation method. Concerning the canal systems, main canal is lined with about 1.1 km distance while secondary & tertiary canals are unlined (earthen). Hence, the conveyance efficiency has estimated to be 85% (as main and secondary canals will be lined and the rest are earthen type, based on engineering study data) and the field application efficiency has also estimated to be 60% seeing that the soil texture of the command area is predominantly clay loam type (according to ADSWE soil Lab result). Thus, the overall/project efficiency has estimated to be 51%.

The aforementioned three steps as enumerated above will be able to give the crop water requirement during its cropping period. The losses in the irrigation system have to be incorporated to arrive at the water requirement at the head of the irrigation system.

### Irrigation methods

Among the different irrigation systems, Gobu irrigation system will be used for the project area; and the irrigation water will be obtained from Gobu River. In addition, by constructing diversion spate and convoying the water commonly through earthen canals (PC, SC, and TC) and then leading to field canals; and finally irrigation takes place mostly in furrows.

For this project, among the various irrigation methods, surface irrigation method has been selected.  Of the surface irrigation methods furrow, border and basin irrigation methods can be used to supply irrigation water to the plants/crops. However, each method has its own advantages and disadvantages. Care should be taken when choosing the method, which is best suited to the local circumstances, i.e., depending on slopes, soil types, selected crop types, amount of water available, etc. of the command area.

Based on the above factors surface irrigation method has been proposed for the proposed crops in this project. The method allows applying light irrigation and can be laid out in sloping fields along the contour. Furrow irrigation method is best suited for most of the proposed and row planted crops. In general, furrow irrigation method is simple, manageable and widely practiced irrigation method. This method is suitable for row crops that cannot stand in water for long periods. The only thing required to use this method is row planting of crops. Besides, basin and border irrigation method would be used for the non-row planted crops. Rotational flow water distribution is also recommended for the project area.

## Irrigation and Drainage System Layout

The irrigation system layout for the project is prepared considering the following points besides other factors.

- A primary concern in the layout of the system is that it serves the purpose of conveying and distributing water to the command area.

- The excavation and earth fill volumes not be excessive, otherwise the construction costs can be tremendous.

- The selection of longitudinal bed slope is made taking into account the existing slopes of the terrain, to minimize deviations in canal routing.

- Curves in canals should not be too sharp.

The proposed irrigation system layout comprises one main canal, one secondary canal and three tertiary canals as shown on the layout Drawings. The main canal runs for most of its length parallel to the contours and several changes of direction are necessary to follow the topography. It crosses four gullies, one foot path. The main canal is masonry lined for a length of 1100meters starting from the spate outlet to 1100m the main canal is lined since the soil is pervious.

### Conveyance System

The conveyance system consists of one Main canal to irrigate total command area of 68ha. The main canal starts from Water abstraction site on left side and conveys water for a length of 2.040Km.

Main canal is aligned along contours and supplies to one secondary unit and four tertiary canals.

## Design of the Canal System

Flow Depth and Section Capacity

3101m length of masonry lined rectangular main canal is designed.

Rectangular x-section using Manning's Formula:

Where Q= discharge (m3/s)

R= Hydraulic radius (Flow area/wetted perimeter)

S= Hydraulic gradient

n= Manning's roughness coefficient, n=0.018 is adopted for the lined masonory channels and n=0.023for earthen part of the canal

### Main Canal

All the main canal is lined rectangular .The total length 3101m

Depth of flow in the canal for design discharge is determined using Manning’s equation: i.e. V=1/n* R^ (2/3)*S^.5.

Table 9-11: Hydraulic Parameters of main canal

| Canal name | Chain age | A (ha) | Qr (m3/s) | m (H:V) | B (m) | D (m) | Fb (m) | Area | Perimeter | R=A/P | S | N | V | Q d. (m3/s) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| MC-1 | 1-145 | 400 | 1.74 | 1 | 1.7 | 0.7 | 0.2 | 1.19 | 3.1 | 0.38 | 0.0125 | 0.018 | 3.28 | 3.90 |
| MC-1 | 145-409 | 400 | 1.74 | 1 | 1.4 | 0.7 | 0.2 | 0.98 | 2.8 | 0.35 | 0.005 | 0.018 | 1.95 | 1.91 |
| MC-1 | 409-925 | 379 | 1.65 | 1 | 1.3 | 0.7 | 0.2 | 0.91 | 2.7 | 0.34 | 0.00555 | 0.018 | 2.01 | 1.82 |
| MC-1 | 925-1200 | 339 | 1.47 | 1 | 1 | 0.6 | 0.2 | 0.6 | 2.2 | 0.27 | 0.0025 | 0.018 | 1.17 | 0.70 |
| MC-1 | 1200-1671 | 221 | 0.96 | 1 | 1 | 0.6 | 0.2 | 0.6 | 2.2 | 0.27 | 0.004 | 0.018 | 1.48 | 0.89 |
| MC-1 | 1671-1950 | 181 | 0.79 | 1 | 1 | 0.5 | 0.2 | 0.5 | 2 | 0.25 | 0.006666 | 0.018 | 1.80 | 0.90 |
| MC-1 | 1950-3100 | 181 | 0.79 | 1 | 1 | 0.5 | 0.2 | 0.5 | 2 | 0.25 | 0.00588 | 0.018 | 1.69 | 0.85 |

BW= Canal bottom width                         QR= Required discharge

FSD= Full supply depth                             QD= Designed Discharge

FB= Free board

SS= Side slope

V= Velocity

|  |  |  |  |  |  | FB |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | D |  |  |  |  |  |
|  | B |  |  |  |  |  |  |  |  |  |  |

Fig.4.1 Typical section for rectangular channel

Table 4.1 Hydraulic parameters of the main canal

### Secondary Canals

The capacity of the four secondary canals is determined by multiplying the irrigable area under specified canal the same as main canals. They are designed to  be rectangular masonry sections. The irrigation canal sections are designed according to manning’s formula as follow. The hydraulic variables are calculated   by excel sheet.

Q =

Where

Q = Design discharge, m3/s

R = Hydraulic radius of the canal, m

A = Wetted cross sectional area of canal, m2

n = Manning’s roughness coefficient

S = Bed slope of the canal

Table 9-12: Hydraulic Parameters of secondary canals

| Canal name | Chainage | A (ha) | Qr (m3/s) | m (H:V) | B (m) | D (m) | Fb (m) | Area | Perimeter | R=A/P | S | N | v | Q d. (m3/s) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Sc-1-1 | 00-1091 | 78 | 0.34 | 1 | 1 | 0.5 | 0.2 | 0.5 | 2 | 0.25 | 0.002 | 0.018 | 1.0 | 0.49 |
| SC-1-2 | 00-150 | 46 | 0.20 | 1 | 0.6 | 0.4 | 0.2 | 0.24 | 1.4 | 0.17 | 0.002 | 0.018 | 0.8 | 0.18 |
| Sc-1-3 | 00-150 | 60 | 0.26 | 1 | 0.6 | 0.5 | 0.2 | 0.3 | 1.6 | 0.19 | 0.0025 | 0.018 | 0.9 | 0.27 |
| Sc-1-3 | 150-550 | 44 | 0.19 | 1 | 0.6 | 0.5 | 0.2 | 0.3 | 1.6 | 0.19 | 0.001 | 0.018 | 0.6 | 0.17 |
| Sc-1-3 | 550-1050 | 28 | 0.12 | 1 | 0.6 | 0.5 | 0.2 | 0.3 | 1.6 | 0.19 | 0.00143 | 0.018 | 0.7 | 0.21 |
| Sc-1-3 | 1050-1300 | 16 | 0.07 | 1 | 0.6 | 0.5 | 0.2 | 0.3 | 1.6 | 0.19 | 0.0025 | 0.018 | 0.9 | 0.27 |
| Sc-1-4 | 00-450 | 61 | 0.26 | 1 | 1 | 0.5 | 0.2 | 0.5 | 2 | 0.25 | 0.001 | 0.018 | 0.7 | 0.35 |
| Sc-1-4 | 450-944 | 51 | 0.22 | 1 | 1 | 0.5 | 0.2 | 0.5 | 2 | 0.25 | 0.00077 | 0.018 | 0.6 | 0.31 |

|  |  |  |  |  |  | FB |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | D |  |  |  |  |  |
|  | B |  |  |  |  |  |  |  |  |  |  |

Figure 9-1: Typical Cross Section of secondary canals

### Tertiary canals

The capacity of the canal is determined by multiplying the irrigable area under specified canal the same as main canals. The irrigation canal sections are designed according to manning is formula as follow.	Tertiary canals are 16 in number Tc-2, TC-4 are lined rectangular canals, and other 14 tertiary canals are earthen trapezoidal canals

Q =

Where

Q = Design discharge, m3/s

R = Hydraulic radius of the canal, m

A = Wetted cross sectional area of canal, m2

n = Manning’s roughness coefficient

S = Bed slope of the canal

The shape of the canal cross section, A is trapezoidal with 1 to 1side slope as shown below

. Figure 9-2: Typical Cross Section of tertiary canals

Where;

b = Bed width of the canal, m

d = Water depth of the canal, m

FB = free board of the canal, m

p = wetted perimeter of the canal, m

Table 9-13: Hydraulic Parameters of tertiary canals

| Canal name | Chainage | A (ha) | Qr (m3/s) | m (H:V) | B (m) | D (m) | Fb (m) | Area | Perimeter | R=A/P | S | N | V | Q d. (m3/s) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Tc-1-1 | 00-50 | 9.66 | 0.04 | 1 | 1 | 0.5 | 0.2 | 0.75 | 2.41 | 0.31 | 0.0025 | 0.023 | 1.00 | 0.75 |
| Tc-1-1 | 50-336 | 6.66 | 0.03 | 1 | 1 | 0.5 | 0.2 | 0.75 | 2.41 | 0.31 | 0.0014 | 0.023 | 0.75 | 0.57 |
| TC-1-2 | 00-28 | 17.40 | 0.08 | 1 | 0.5 | 0.4 | 0.2 | 0.2 | 1.30 | 0.15 | 0.0071 | 0.018 | 1.35 | 0.27 |
| TC-1-2 | 28-700 | 12.40 | 0.05 | 1 | 0.5 | 0.4 | 0.2 | 0.2 | 1.30 | 0.15 | 0.0053 | 0.018 | 1.16 | 0.23 |
| TC-3 | 00-194 | 10.45 | 0.05 | 1 | 0.6 | 0.4 | 0.2 | 0.4 | 1.73 | 0.23 | 0.0008 | 0.023 | 0.47 | 0.19 |
| TC-4 | 00-500 | 52.00 | 0.23 | 1 | 0.5 | 0.4 | 0.2 | 0.2 | 1.30 | 0.15 | 0.0063 | 0.018 | 1.26 | 0.25 |
| TC-4 | 500-748 | 42.00 | 0.18 | 1 | 0.5 | 0.4 | 0.2 | 0.2 | 1.30 | 0.15 | 0.0040 | 0.018 | 1.01 | 0.20 |
| Tc-5 | 00-350 | 12.20 | 0.05 | 1 | 0.4 | 0.2 | 0.2 | 0.12 | 0.97 | 0.12 | 0.0050 | 0.023 | 0.77 | 0.09 |
| Tc-6 | 00-457 | 14.30 | 0.06 | 1 | 0.4 | 0.2 | 0.2 | 0.12 | 0.97 | 0.12 | 0.0050 | 0.023 | 0.77 | 0.09 |
| Tc-7 | 00-469 | 14.20 | 0.06 | 1 | 0.3 | 0.2 | 0.2 | 0.1 | 0.87 | 0.12 | 0.0067 | 0.023 | 0.84 | 0.08 |
| Tc-8 | 00-538 | 16.40 | 0.07 | 1 | 0.3 | 0.2 | 0.2 | 0.1 | 0.87 | 0.12 | 0.0050 | 0.023 | 0.73 | 0.07 |
| Tc-9 | 00-478 | 22.20 | 0.10 | 1 | 0.4 | 0.2 | 0.2 | 0.12 | 0.97 | 0.12 | 0.0063 | 0.023 | 0.86 | 0.10 |
| Tc-10 | 00-713 | 17.00 | 0.07 | 1 | 0.4 | 0.2 | 0.2 | 0.12 | 0.97 | 0.12 | 0.0050 | 0.023 | 0.77 | 0.09 |
| Tc-11 | 00-309 | 18.94 | 0.08 | 1 | 0.4 | 0.2 | 0.2 | 0.12 | 0.97 | 0.12 | 0.0040 | 0.023 | 0.68 | 0.08 |
| Tc-12 | 00-556 | 19.00 | 0.08 | 1 | 0.5 | 0.3 | 0.2 | 0.24 | 1.35 | 0.18 | 0.0025 | 0.023 | 0.69 | 0.17 |
| Tc-13 | 00-484 | 16.00 | 0.07 | 1 | 0.5 | 0.2 | 0.2 | 0.14 | 1.07 | 0.13 | 0.0033 | 0.023 | 0.65 | 0.09 |
| Tc-14 | 00-549 | 25.50 | 0.11 | 1 | 0.4 | 0.3 | 0.2 | 0.21 | 1.25 | 0.17 | 0.0050 | 0.023 | 0.94 | 0.20 |
| Tc-15 | 00-546 | 23.44 | 0.10 | 1 | 0.5 | 0.2 | 0.2 | 0.14 | 1.07 | 0.13 | 0.0050 | 0.023 | 0.79 | 0.11 |
| Tc-16 | 00-361 | 14.00 | 0.06 | 1 | 0.5 | 0.2 | 0.2 | 0.14 | 1.07 | 0.13 | 0.0033 | 0.023 | 0.65 | 0.09 |

### Field Canals

As shown in the layout, field canals run across the contours and hence face relatively steeper gradient. The discharge of most of the field canals is very small and this is taken as an advantage to cope up with the relatively steeper gradient. As much as possible field canals shall be made in fill in order to easily irrigate the adjacent command area. As can be seen from the layout, majority of the filed canals can be used to irrigate both sides of the command area depending on the condition of the individual plots of land owned by individual farmers.

Figure 9-3: Typical Field Canal X-section

### Design of Division box

The proportional division of water among receiver canal and delivery canal is effected by providing division box.  The principle of designing of the division opening width is computed based on the rectangular notch weir formula as follow.

Q1=1.71*L1*H1^3/2 and Q2=1.71*L2*H2^3/2

Since = H1 = H2

Where;

Q1 = discharge on main or secondary canal

Q2 = discharge on secondary or tertiary canal

L1 and L2 = respective opening width

H1 = H2 = depth of water over the crest

Cd = discharge coefficient, 0.62

The computed hydraulic parameters using the above proportionality so

Shown in table

Figure 9-4: Typical Division Box plan

| Dividing canal Name | Chain age | Q0 (lit/sec) | Q1 (lit/sec) | Q2 (lit/sec) | Q3 (lit/sec) | B0 (m) | D0 (m) | H0 (m) | D (D0+FB) (m) | B (m) | L0 | L1 (m) | L2 (m) | L3 (m) | D1 (m) | B1 (m) | D2 (m) | B2 (m) | D3 (m) | B3(m) | Lp2 (m) | Lp3 (m) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Mc -1 & TC-1-1&TC-3 | 0+322 | 1912.0 | 1410.0 | 333.0 | 750.0 | 1.4 | 0.7 | 0.9 | 0.9 | 3.1 | 1.4 | 1.4 | 1.0 | 1.0 | 0.7 | 1.4 | 0.4 | 1.0 | 0.5 | 1.0 | 1.4 | 1.8 |
| Mc-1 & TC-1-2 | 0+646 | 1410.0 | 1400.0 | 0.0 | 270.0 | 1.3 | 0.7 | 0.7 | 0.9 | 2.8 | 1.3 | 1.3 | 0.0 | 0.5 | 0.7 | 1.3 | 0.0 | 0.0 | 0.4 | 0.5 | 0.5 | 1.3 |
| MC-1-1↔SC-1-1 | 0+979 | 1017.0 | 554.0 | 0.0 | 350.0 | 1.0 | 0.6 | 0.7 | 0.8 | 2.4 | 1.0 | 1.0 | 0.0 | 1.0 | 0.6 | 1.0 | 0.0 | 0.0 | 0.5 | 1.0 | 0.5 | 1.4 |
| MC-1-1↔SC-1-2 | 1+586 | 554.0 | 554.0 | 0.0 | 180.0 | 1.0 | 0.5 | 0.5 | 0.7 | 1.9 | 1.0 | 1.0 | 0.0 | 0.6 | 0.5 | 0.7 | 0.0 | 0.0 | 0.4 | 0.6 | 0.5 | 1.1 |
| MC-1-2↔SC-1-3 | 2+197 | 554.0 | 536.0 | 0.0 | 290.0 | 1.0 | 0.5 | 0.5 | 0.7 | 1.9 | 1.0 | 1.0 | 0.0 | 1.0 | 0.5 | 0.7 | 0.0 | 0.0 | 0.5 | 1.0 | 0.5 | 1.3 |
| MC-1-1,↔SC-1-4 | 2+505 | 536.0 | 536.0 | 0.0 | 350.0 | 1.0 | 0.5 | 0.5 | 0.7 | 1.9 | 1.0 | 1.0 | 0.0 | 1.0 | 0.5 | 0.7 | 0.0 | 0.0 | 0.5 | 1.0 | 0.5 | 1.4 |
| SC-1-1$TC-1-4 | 0+897 | 318.0 | 318.0 | 270.0 | 0.0 | 1.0 | 0.5 | 0.3 | 0.7 | 1.7 | 1.0 | 1.0 | 0.8 | 0.0 | 0.5 | 1.0 | 0.4 | 0.5 | 0.0 | 0.0 | 1.3 | 0.0 |
| SC-1-1$TC-1-6 | 0+84 | 350.0 | 318.0 | 270.0 | 0.0 | 1.0 | 0.5 | 0.3 | 0.7 | 1.7 | 1.0 | 1.0 | 1.0 | 0.0 | 0.5 | 1.0 | 0.5 | 1.0 | 0.0 | 0.0 | 1.3 | 0.0 |
| SC-1-1$TC-1-7 | 0+400 | 318.0 | 43.4 | 274.6 | 0.0 | 1.0 | 0.5 | 0.3 | 0.7 | 1.7 | 1.0 | 1.0 | 1.0 | 0.0 | 0.5 | 1.0 | 0.5 | 1.0 | 0.0 | 0.0 | 1.3 | 0.0 |
| SC-1-2$TC-1-8 | 0+50 | 180.0 | 45.0 | 135.0 | 0.0 | 0.6 | 0.4 | 0.3 | 0.6 | 1.2 | 0.6 | 0.6 | 0.6 | 0.0 | 0.4 | 0.6 | 0.4 | 0.6 | 0.0 | 0.0 | 1.1 | 0.0 |
| SC-1-2$TC-1-9 | 0+400 | 135.0 | 89.5 | 45.5 | 0.0 | 0.6 | 0.4 | 0.3 | 0.6 | 1.1 | 0.6 | 0.6 | 0.6 | 0.0 | 0.4 | 0.6 | 0.4 | 0.6 | 0.0 | 0.0 | 0.8 | 0.0 |
| SC-1-2$TC-1-10 | 0+750 | 45.5 | 45.5 | 45.5 | 0.0 | 0.6 | 0.4 | 0.1 | 0.6 | 0.9 | 0.6 | 0.6 | 0.6 | 0.0 | 0.4 | 0.6 | 0.4 | 0.6 | 0.0 | 0.0 | 0.8 | 0.0 |
| SC-1-3$TC-1-11 | 0+17 | 290.0 | 280.0 | 88.0 | 0.0 | 0.8 | 0.5 | 0.4 | 0.7 | 1.5 | 0.8 | 0.8 | 0.8 | 0.0 | 0.5 | 0.8 | 0.5 | 0.8 | 0.0 | 0.0 | 0.9 | 0.0 |
| SC-1-3$TC-1-12 | 0+500 | 280.0 | 205.0 | 75.0 | 0.0 | 0.8 | 0.5 | 0.3 | 0.7 | 1.5 | 0.8 | 0.8 | 0.8 | 0.0 | 0.5 | 0.8 | 0.5 | 0.8 | 0.0 | 0.0 | 0.9 | 0.0 |
| SC-1-3$TC-1-13 | 0+900 | 205.0 | 125.0 | 80.0 | 0.0 | 0.8 | 0.5 | 0.3 | 0.7 | 1.4 | 0.8 | 0.8 | 0.8 | 0.0 | 0.5 | 0.8 | 0.5 | 0.8 | 0.0 | 0.0 | 0.9 | 0.0 |
| SC-1-4$TC-1-14 | 0+34 | 350.0 | 306.0 | 44.0 |  | 1.0 | 0.5 | 0.3 | 0.7 | 1.7 | 1.0 | 1.0 | 1.0 | 0.0 | 0.5 | 1.0 | 0.5 | 1.0 | 0.0 | 0.0 | 0.8 | 0.0 |
| SC-1-4$TC-1-15 | 0+500 | 306.0 | 224.0 | 82.0 |  | 1.0 | 0.5 | 0.3 | 0.7 | 1.6 | 1.0 | 1.0 | 1.0 | 0.0 | 0.5 | 1.0 | 0.5 | 1.0 | 0.0 | 0.0 | 0.9 | 0.0 |
| SC-1-4$TC-1-16 | 948 | 224.0 | 114.0 | 110.0 |  | 1.0 | 0.5 | 0.3 | 0.7 | 1.5 | 1.0 | 1.0 | 1.0 | 0.0 | 0.5 | 1.0 | 0.5 | 1.0 | 0.0 | 0.0 | 1.0 | 0.0 |

## River Protection Works by Gabion

Though for some part of the river left banks  masonry retaining wall have been designed, the rest left banks are being affected by intense erosion. Therefore, to keep the base flow direction, and to protect the bank from erosion the bank should be treated with gabions. The right bank is relatively stable and masonry retaing walls and gabion designed to keep HFL.As we have observed during site study about 86.7m length of the bank protected by gabion

Gabion works design;-

- Gabion mattress size: - this the gabion laid at the toe of the river left and right bank. As computed above the scour depth at headwork is 2.9m. Hence, the mattress extends 0.8m below the riverbed.

The size of stone to be used is 180mm diameter within gabion mesh size of 60 *80mm.

- Gabion box size determination: - the gabion in box form is used as standing wall by over laying each other. Its height is equal to the bank height for this project, which is 2.9m. The size of stone 200mm should be used with gabion mesh size of 60*80mm.for better insight see figure below.

# Bill of quantity

| BILL NO-3 Irrigation Infrastructure |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| S/no | Item of work | Unit | Quantity | Unit rate | Total cost |
|  | B. Irrigation Infrastructure |  |  |  |  |
| 1 | Main Canal |  |  |  |  |
| 1.1 | Excavation (soil) | m3 | 7352.28 | 90 | 661,705.20 |
| 1.2 | Back fill | m3 | 461.9 | 120 | 55,428.00 |
| 1.3 | Masonry work for lining | m3 | 3053 | 1,850.00 | 5,648,050.00 |
| 1.4 | Plastering | m2 | 8546 | 122 | 1,042,612.00 |
|  | Sub total |  |  |  |  |
| 2 | Secondary Canal |  |  |  |  |
| 2.1 | Excavation (soil) | m3 | 3674.7 | 90 | 330,723.00 |
| 2.2 | Back fill | m3 | 268.9 | 114 | 30,654.60 |
| 2.3 | Masonry work for lining | m3 | 3539.1 | 1850 | 6,547,335.00 |
| 2.4 | Plastering | m2 | 12752 | 122 | 1,555,744.00 |
|  | Sub total |  |  |  |  |
| 3 | Tertiary canal |  |  |  |  |
| 3.1 | Excavation (soil) | m3 | 5516 | 90 | 496,440.00 |
| 3.2 | Back fill | m3 | 719.5 | 114 | 82,023.00 |
|  | sub total |  |  |  |  |
| 4 | division box |  |  |  |  |
| 4.1 | Excavation (soil) | m3 | 123.26 | 90 | 11,093.40 |
| 4.2 | Masonry work | m3 | 50.7 | 1,850.00 | 93,795.00 |
| 4.3 | Ston pitch | m3 | 32.22 | 1200 | 38,664.00 |
| 4.4 | plastering | m2 | 102.2 | 122 | 12,468.40 |
| 5 | Gate |  |  |  |  |
| 5.1 | 4mm thick sheet metal | m2 | 150 | 3000 | 450,000.00 |
| 5.2 | 40*40*4mm angle iron | m | 170 | 2000 | 340,000.00 |
| 5.3 | ф10mm bar for handling | kg | 63 | 47 | 2,961.00 |
| 6 | sub total |  |  |  |  |
| 6.1 | Turn out |  |  |  |  |
| 6.2 | Excavation (soil) | m3 | 216 | 90 | 19,440.00 |
| 6.3 | Masonry work | m3 | 199.6 | 1,850.00 | 369,260.00 |
| 6.4 | Ston pitch | m3 | 334 | 1,400.00 | 467,600.00 |
| 6.5 | c-10 | m3 | 254.5 | 2,500.00 | 636,250.00 |
| 6.6 | plastering | m2 | 1047.5 | 122 | 127,795.00 |
|  | sub total |  |  |  |  |
| 7 | Cattle crossing |  |  |  |  |
| 7.1 | Excavation (soil) | m3 | 6 | 90 | 540.00 |
| 7.2 | Masonry(1:4) | m3 | 4.47 | 1,850.00 | 8,269.50 |
| 7.3 | plastering | m2 | 16.4 | 122 | 2,000.80 |
| 7.4 | Concrete (C-20) | m3 | 3 | 2900 | 8,700.00 |
| 7.5 | Reinforcement bar, φ12 | Kg | 120 | 47 | 5,640.00 |
|  | sub total |  |  |  |  |
| TOTAL |  |  |  |  | 20,887,477.40 |

# CONCLUSION AND RECOMMENDATION

- The infrastructure of this project area is designed to irrigate about 400ha of land by taking its supply from the Gobu diversion spate irrigation project.

- The reason why the main canal is to be lined up to the end is to avoid the scoring problem.

- The design of the canal dimensions of the irrigation canal is done by applying the manning’s uniform flow equation. The variable of the hydraulic parameters are calculated using iteration or flow master program.

- The design discharge of the drainage canals are determined using rational Gamble Powell method.

- As soils of the command area are predominantly loam textured; and hence water and soil management measures should be undertaken; and optimum moisture content should be maintained to improve workability of the soil during land preparation and planting time.

The following recommendations are drown:

- For better performance and long service year of the project regular inspection and maintenance is highly required.

- Farmers training, how to operate and maintain the project structures as a whole and available and water resources has a paramount important.

- The irrigation hours per day and per week should be flexible based on base flow amount of each week or month.

- Close supervision of the construction should be made to modify (if need be) each Components of irrigation system based on specific site conditions.

# OPERATION AND MAINTENANCE

## General

The main objective of the operation and maintenance aspect of an irrigation scheme is to facilitate the timely delivery of the required irrigation water to farms and to keep the irrigation system in an optimum operating condition.  This section therefore, discusses the main functions of the subject matter under consideration for the scheme.

## Operation of the Head Works

Operation mainly focuses on the diversion of a controlled flow of river water, timely cleaning of floating debris in front of spate and removal of sediment deposits in front of the spate structures.

## Irrigation System Operation

The operation of the irrigation system depends mainly on the method of water delivery at farm level. Surface irrigation method is the recommended type of water distribution and application method for Gobu spare irrigation scheme.

The farmers would organize themselves and form groups in order to handle the water management.  Since flow is low Rotational water distribution would be applied within the group.  The rotational distribution is then to distribute water by turn to the whole scheme according to the timely need of crop water requirement. For better and efficient water management, crop diversification should be avoided within a group.  This would reduce the complexity of water distribution system of the scheme during one irrigation season. At farmers’ level of operation, a constant flow and variable irrigation time is advisable.

The operation of the irrigation system is continuous for 24 hours per day in main, secondary and tertiary canals where as field canals within a tertiary block are operating in rotational system with each other for irrigation hours proportional to their size. Since the tertiary canal discharges are within the manageable range and the irrigation canal structures are accordingly designed for simple operation, the farmers can open and close easily whenever they required.

## Maintenance Requirement

The canal system of the project main canal and secondary canal are masonry lined, which is susceptible to siltation, erosion, growth of weeds and or breaching problems. Though the canal sections are designed for non- silting and non scouring conditions, the above mentioned problems are unavoidable and hence periodical and yearly inspection and maintenance of canals and structures are obligatory in order to fulfill the design objective of the project area.

The maintenance tasks are categorized into two types: - routine activities, and repairs. The routine maintenance activities that are carried out periodically include-

- Regular cleaning of sediments and weeds from canals and drains;

- Inspection and lubrication of gates; and

- Maintenance of cracked lined canals, regulating and control structures.

Repair works include task carried out more frequently and quickly, and include those tasks that are generally unpredictable. They also include emergency works. The activities included in this category are:-

- Repairing overtopped or breached canals, drains, and flood protection dykes;

- Repairing jammed gates;

- Filling holes made by wild animals; and

- Reduced free board due to walking over by people and livestock.

Regular inspection of the irrigation facilities should be carried out as part of the maintenance activities.  These tasks could be carried out immediately after the end of the main rains in September and during the rainy season.  This could concentrate on the interceptor drains and the flood protection dykes, the main canal and the field drains.  The inspection of the other works like the tertiary canals, field drains, and the water control and regulating structures could be carried out as part of routine operation activities.

- Beneficiaries of the project need to have operation and maintenance budget, For O&M cost incurring entity area:

- Purchase sing of gate lubricate (grease)

- Replacing and maintenance of Stolen and damaged gates

- Repair Damages on the cross drainage structures.

The expense for O&M should be collected from the beneficiaries. Of course, much of the task is done by the labor and skill of the community. For cost incurring activities beneficiaries have to collect money based on the proportion of the command area they owned.

# REFERENCE

- FAO (1977) guidelines for predicting crop water requirements. No 24, Rome Italy

- Design of small Canal structures , USBR

- Soft copies of hydraulic structure publishing

- IDD manual

- ESRDF manual