	Oromia 19 Small Scale Irrigation Projects(SSIPs)

Feasibility Study &  Detail Design for  LOT-4  SSIPs:  Tilo, Tilo, Danissa, and Lega Lola 

Final Soil and Land Suitability study Report for Laga Lola SSIPs in Munessa Ana of Arsi Zone 
	Oromia
 Irrigation 
Development Authority(OIDA)



[image: image1.jpg]


                                                                                                              
[image: image2.png]



the national REGIONAL staTe of oromia
OROMIA IRRIGATION DEVELOPMENT AUTHORITY (OIDA)
[image: image3.jpg]Map of Lega Lalo

B1600 811800 $12000  §12200  $12400 612600 12800 #1300 §13200 13400 613600 813500

11400

494100

494400

494700

495000

495300

495600

496200 496500

496800 497100 497400 497700

U3

Spu-3.

SMu-1

{

snug -

Liats SMUS
~

wars®
R

SMU-A

sMuds

smy-1

SMUBA | (iate

SMU4.

SMu5.

)

LAt

suuh

P01 SMU3

==

SMU-4.

SMU3.

Legend

SMU5.

LAt

LAtz =5

2 LL;\_,J\-/

—
mu%
LE/ A Observation

A House
o Tukul

Main canal

[ Boundary

200 | 100

200

=

Soil

Mapping Unit Description

smu-t

smu-2

RTES

smus

smus

Very deep, Very dark grayish brown (10YR312) in color, Loam texture, moderately
drained, weak medium platy structure, friable when moist, sticky and plastic
‘when wet, strongly acidic occur on very gently undulated plain with slope range.
010 - 4% (Haplic Cambisols (Dystric, Siltic). (Area = 17.7 ha)

Very deep, Dark brown (10YR312) in color, clay loam in texture, moderately
drained, weak medium platy structure, friable when moist, sticky and plastic
when wet, strongly acidic occur on a undulating plain with slope range of
4-6% (Haplic Cambisols (Dystric, Siltic)). (Area = 32.7 ha)

Very deep, Dark brown (10YR312) in color, clay loam in texture, moderately
drained, weak medium platy structure, friable when moist, sticky and plastic
when wet, strongly acidic occur on moderately undulating with slope range
of 6-8% (Haplic Cambisols (Dystric, Siltic)). (Area = 24.5 ha)

Very deep, Dark brown (10YR312) in color, clay loam in texture, moderately
drained, weak medium platy structure, friable when moist, sticky and plastic
‘when wet, strongly acidic occur on rolling topography with siope range of
8- 15% (Haplic Cambisols (Dystric, Siltic)).(Area = 70.4 ha)

Very deep, Dark brown (10YR312) in color, clay loam in texture, moderately
drained, weak medium platy structure, friable when moist, sticky and plastic
‘when wet, strongly acidic occur on moderately steep side slope topography
with slope range of 15 - 30% (Haplic Cambisols (Dystric, Siltic)). (Area =77.0 ha)

494100

494400

494700

495000

495300

495600

495900

496200 496500

496800 497100 497400 497700

B1G00 811300 812000 $12200  $12400 512600 812800 813000 $13200  $13400 _ $13600 813800

11400





OROMIA 19 SMALL SCALE IRRIGATION PROJECTS (SSIPs)

FEASIBILITY STUDY AND DETAIL DESIGN FOR LOT-4 SSIPs:

ABISHIKA,TILO, DANISSA AND LEGA LOLA

PART-I:-FINAL FEASIBILITY STUDY REPORT

ANNEX B:-SOIL AND LAND SUITAILITY STUDY REPORT FOR LAGA LOLA
By

INTEGRATED WATER RESOURCES DEVELOPMENT PLC
P. O. Box: 382/1250, Addis Ababa/Finfinne

E-mail:hundeifa@gmail.com/ planetid21@gmail.com
Tel.:+251-930-013-249/250
June, 2016

Finfinne

FINAL FEASIBILITY STUDY & DETAIL DESIGN REPORTS

DELIVERABLES:
PART-I:-FINAL FEASIBILITY STUDY REPORT

ANNEX A: - CLIMATE AND HYDROLOGICAL ANALYSIS

ANNEX B: - SOIL AND LAND SUITABILITY STUDY

ANNEX C: - GEOLOGY AND GEOTECHNICAL STUDY

ANNEX D: - IRRIGATION AGRONOMY

ANNEX E: -WATERSHED MANAGEMENT

ANNEX F: -SOCIOECONOMY, AND ORGANIZATION AND MANAGEMENT STUDY

ANNEX G: -FINANCIAL & ECONOMIC ANALYSIS
ANNEX H: -ENVIRONMENTAL & SOCIAL IMPACT ASSESSMENT

PART-II:-DETAIL ENGINEERING DESIGN REPORT

ANNEX-A:-DETAIL ENGINEERING DESIGN REPORT, AND OPERATION & MAINTENANCE MANUAL

ANNEX-B:-DETAIL ENGINEERING DRAWINGS (ALBUM)

ACRONYMS & ABRIVATION

AWC

-
Available Water Capacity

BS%

-
Base Saturation Percentage

Ca

-
Calcium

CaCo3

-
Calcium Carbonate

0C

-
Degree Centigrade
CEC

-
Cation Exchange Capacity

CM

-
Centimeter 

cS/m

-
Centimeter Siemens per Meter

DAP

-
Di Amino Phosphate

EARO

-
Ethiopia Agriculture Research Organization

E

-
East

EC

-
Electrical Conductivity

ESP

-
Exchangeable Sodium Percentage

FAO

-
Food and Agriculture Organization

FC

-
Field capacity

G

-
Gram

GWT

-
Ground Water Table

GIS

-
Geographic Information System

Ha

-
Hectare

ISRIC

-
International Soil Reference and Information Center

ITC

-
International Institute for Aerospace Survey and Earth Sciences

IR

-
Infiltration Rate

HC

-
Hydraulic conductivity

Km

-
Kilometers

L

-
Loam

LUP

-
Land Unit Map

LUPRD
-
Land Use Planning and Regulatory Department

LUT

-
Land Utilization Type

m

-
Meter

masl

-
Meters Above Sea Level

Meq

-
Mill equivalents

mm

-
Millimeters

MoA

-
Ministry of Agriculture

Mg

-
Magnesium

Meq

-
Mili-equivalent

Na

-
Sodium

OC

-
Organic Carbon

OM

-
Organic Matter

PET

-
Potential-evapo-transpiration

PWP

-
Permanent wilting point

P

-
Phosphorus

PPM

-
Parts Per million

pH

-
Soil Reaction (log10H+ions, acidity, basicity)

EXCUTIVE SUMMARY

Lega-Lola Small Scale Irrigation project area is found in Adare Awundala kebele,Munesa wereda, in Arsi zone, Oromia Regional state, which covers 222hectares of land. The project lies between altitudinal ranges3021 to 3070m.a.s.l. The studied area is situated mainly in undulating to moderately steep landform ranging between 8 and 30%, which form 68% out of the total area. The extensive area is covered by deep to very deep rooting depth, varying from 125 to 200 cm depth. Hence, the rooting depth is suitable for any crops. The existing land cover is composed of intensively cultivated land, grass land and village. These soils have loamy surface, while silty loam in subsoil. The bulk density of the soils of the Lega-Lola command area ranges from 1.29to 1.41gm/ cm3. The AWC of the project is 204 mm/m, and classified as very high water holding capacity class. The infiltration rate is 2.3 cm/hr.
Organic carbon is very high that ranged from 2.9to 3.5%.  Total nitrogen is very high with value of 0.3 – 0.4%. The Available phosphorus in these soils were found to be low to high (0.5-17.4 ppm with mean value of 8.9 ppm) but, in most cases it can be categorized as high class and assumed to be adequate. The content of exchangeable potassium, and calcium is moderate with value between 0.5 and 0.6 and 7.2 and 0.3cmol (+)/kg respectively.  In the studied soils the exchangeable magnesium content ranged from 0.3 up to 1.2meq/100g soil and can be rated as low level.  The soils have a medium to very high CEC content soils with value that ranged from 19.1 to 27.1cmol (+)/kg. Hence generally the level of CEC is moderate to high the soils are suitable to intensive cultivation.  The surveyed soil reaction isvery strongly acidic with value of pH= 4.2– 4.9.The exchangeable acidity of the project area is0.3Cmol (+)/kg. Generally, most crops prefer a moderately to slightly acidic soil reaction with the pH range 5.5 - 6.5 in water solution. The soil salinity and sodicity are very low.  According to FAO classification was identified one major soil unit i.e. Haplic Cambisols (Eutric, Siltic).  
The actual land evaluation process, which was conducted in Lega-Lola small scale irrigation project revealed that out of the total area of 222 hectares of land 18 and 53 ha is found to be moderately (S2z/t) and marginally suitable (S3t/z) for surface irrigated agriculture respectively, while 117 ha and 34 ha currently unsuitable (N1t) and   permanently (N2t) not suitable for surface irrigation due to steep slope and occurrence of strong soil acidity respectively. 
However, by employing corrective measures employing lime in order to reduce the soil acidity problem and using overhead irrigation system (sprinkler)instead of surface irrigation (furrow irrigation method) in line with by constructing bench terrace, it is possible to upgrade to improved suitability class. As a result the land suitability level 55 hectares of land, which constitute 25%, would be highly suitable (S1).Furthermore16ha and 117ha can be classified as moderately suitable (S2t/z) and marginally suitable (S3t) respectively. Hence forth, 188hectares of land (85 out of the total land) possible to scale up to suitability class under sprinkler irrigated agriculture. While, the remaining 34 hectares of land which forms about 15% is not suitable for irrigation for the reason that of high slope gradient.  The main limitations of the area are steep slope and acidic soil reaction. To abolish the prevailing constraints can be advised to realize the following techniques:

· One of the major constraints in the area as aforementioned is soil acidity. Hence, acidity problem can be controlled by applying additions of organic matter and lime. To overcome the effect soil acidity should be applied 0.5tone of lime per hectare in the project area. 

· To improve and maintain the existing soil fertility should applied proper rate, place and timely inorganic and organic fertilizer in the project area with adequate care.    

· Since the topography of the area undulating to moderately steep is vulnerable to soil erosion. Therefore, appropriate measures should applied as follows:  
· For topography land with slope ranging from 0 to 4 percent must have agronomic measures comprise contour farming, mulch, zero tillage, and crop rotation, strip cropping and mixed cropping.  
· For the land with slope of 4-20% is better to construct soil bund and build bench terrace to protect the soil from erosion.

· If the project shall be implemented, during construction activities like earthworks have to be done in dry season; 

· The exposed or excavated soil should be refill soon after completion of works;

· The planned project has to play role in conserving and maintaining ideal environment with maximum biodiversity.

· Regarding to irrigation management and system might be recommended to apply for the command area overhead irrigation system (sprinkler). 
TABLE OF CONTENTS

iACRONYMS & ABRIVATION

EXCUTIVE SUMMARY
iii
LISTS OF TABLES
vii
LISTS OF FIGURES
vii
1.INTRODUCTION
1
1.1.
Background
1
1.2.
Objective of the Study
1
1.3.
Scope of the Study
2
2.DESCRIPTION OF THE PROJECT AREA
3
2.1
Location
3
2.2
Climate
4
2.3
Topography
5
2.4
Agriculture, Vegetation and Land Use
7
3.METHODOLOGY AND APPROACH OF THE STUDY
8
3.1
Pre-fieldwork
8
Previous Studies
8
3.2
Fieldwork stage activities
8
3.3
Post Fieldwork Stage
9
4.SOIL PHYSICAL CHARACTERISTICS
11
4.1
Effective Soil Depth
11
4.2
Soil Texture
11
4.3
Soil Structure
11
4.4
Soil Bulk Density
12
4.5
Field capacity and Permanent Wilting Point
12
4.6
Infiltration
13
4.7
Permeability
13
5.CHEMICAL CHARACTERISTICS
14
5.1
General
14
5.2
Soil Reaction
14
5.3
Electrical Conductivity
15
5.4
Cation Exchange Capacity (CEC)
15
5.5
Percentage Base Saturation (PBS)
16
5.6
Exchangeable Cations
16
5.7
Exchangeable sodium
16
5.8
Exchangeable Potassium
16
5.9
Exchangeable calcium
17
5.10
Exchangeable Magnesium
17
5.11
Organic Carbon
17
5.12
Total Nitrogen
17
5.13
The Carbon to Nitrogen Ration
18
5.14
Exchangeable Sodium Percentage
18
5.15
Available Phosphorus
18
6.SOILS OF THE PROJECT AREA
19
6.1
Soils, Their Characteristic and Classification
19
7.LAND SUITABILITY EVALUATION
22
7.1
General
22
7.2
Land Utilization Types (LUT)
23
7.3
Land Suitability Evaluation by Land Utilization Types
26
8.SOILS AND LAND MANAGEMENT UNDER IRRIGATION
30
8.1
General
30
8.2
Soil fertility
30
8.3
Soil Acidity
31
8.4
Soil Cultivation
34
8.5
Soil erosion
34
8.6
Irrigation Management
35
9.CONCLUSION AND RECOMMENDATION
36
9.1
Conclusion
36
9.2
Recommendations
37
10.REFERENCES
38
APPENDIX1:-AUGER SITE DESCRIPTION
40
APPENDIX 2:- LEGA LOLA SOIL MORPHOLOGICAL DESCRIPTION
41
APPENDIX 3:- SOIL LABORATORY ANALYTICAL RESULT
44
APPENDIX 4: SOIL PROFILE DESCRIPTION
46


LISTS OF TABLES
5Table 2. 1 Slope Class of Lega-Lola Small Scale Irrigation Project




 TOC \h \z \c "Table 6." 
19Table 6. 1 Legend of Soil Mapping Unit Lega- Lola Small Scale Irrigation Project




HYPERLINK \l "_Toc454800268"

Table 7. 1Land Uses Requirement and Land Quality Criteria for Surface Irrigation
24

25Table 7. 2 Soil Characteristics of the Soil Mapping Units


26Table 7. 3Matching Process of Land Units and LUT
------------------------------------------------

LISTS OF FIGURES
6Figure2.2:- The Slope map of Lega-Lola Small Scale Irrigation project area---------------------



 HYPERLINK \l "_Toc454800687" 

Fig 6. 1 Map of Soil Mapping Units of Lega-Lola Small Scale Irrigation Project
……….21

28Fig 7. 1 Land Suitability Map for Surface Irrigation System (Furrow) for Lrga-Lola Small Scale Irrigation Project


29Fig 7. 2 Land Suitability Map for Overhead Irrigation System (Furrow) for Lega-Lola Small Scale Irrigation System



1. 

2. INTRODUCTION
1.1. Background

The soil survey was undertaken in Oromia Regional State Government, in Arsi zone, in Sagure wereda. The study was conducted based the latest FAO (2006) standards and procedures, to fulfill requirements of the accepted international standards. Detailed information on the land and soils in the command area was collected to make the evaluation of the land for land utilization type selection, irrigation designing and agricultural input requirements justification. This will be done by focusing on identifying the various topographic forms, soil types, present land use and existing land use pattern, providing a basis for assessment of land and irrigation crop suitability; assessing the existing resources, which relate to soil resources of the command area so that evaluation of the area would be possible. Soil survey is often the major basis for irrigated agriculture land use planning, thus it is essential to carry out intensive soil surveys and investigations. Soil surveys ensure selection of land for irrigated development and sound agricultural management practices. Soil surveys for irrigation suitability assessment were carried out at present feasibility study to supply basic information on crop requirements, suitable irrigation method, and other soil improvement techniques. 

In rehabilitation projects of irrigated lands, besides the modernization of the irrigation systems, were considered two main objectives: to improve field irrigation management, by reducing surface run-off and deep percolation, to save water and reduce the negative environmental impacts of irrigation. In this instance a detailed soil mapping is required.

This report presents the field and laboratory investigation of the soil envisaged project land.  In the following few chapters will be explained the environment setting of the area, methodology and the result of the study and followed by land suitability evaluation.

1.2. Objective of the Study

The main objectives of the soil survey and land evaluation study were:

· To investigate physical and chemical soil properties of the soils, to identify their spatial distribution and to evaluate land suitability for irrigation.

· To assess the potential of soils of the area for selected crops under irrigated agriculture development;

· To determine fertility status of the area and make the  recommendation for the envisaged development based on soil test data; 

· Confirm soil quality and its suitability for irrigation under different irrigation methods.

· Assess and evaluate existing land use pattern for  the proposed irrigation scheme Formulation of proper soil and land management practices, and 

· Land use and land cover study

1.3. Scope of the Study

The scopes of the soil survey and land evaluation study were included the following:

· Conduct field auger to a depth of 120 cm (as the rooting depth of most crops is about 1m);
· Conducting standard soil physical and chemical analysis at laboratory level on collected soil samples;
· Carryout land suitability assessment and produce map of the land suitability ;

· Production of standard soil survey and land evaluation report and thematic maps 

3. DESCRIPTION OF THE PROJECT AREA
3.1 Location

The Lega-Lola Project, which covers some 223ha, lies in Munessa woreda, Arsi zone, Oromia Regional State Government. It is situated from UTM coordinate 494175 to 497727mE and 812128 to 813728mN within altitudinal range between 2805 and 2932 masl. The study area can be reached from both directions by all-weather roads from Kersa town.

[image: image4.emf]Figure 2.1:-Location map of Lega-Lola Small Scale Irrigation Project
3.2 Climate

The Lega-Lola small scale irrigation project is located in Tepid to cool sub-moist mountains agro-ecological zone.  The Climate of the area is characterized by two main seasons, namely dry and rainy seasons. The dry season takes place from November through February. The rainy season characterized by bimodal rainfall regimes, this can be divided the long rainy and short rainy seasons. The long rainy season occurs from June to September, while the short rainy season begins at late April and ends June. The pick rain reaches in July and August with monthly mean 144 and 148 mm/m respectively.  The mean annual rainfall is 851 mm. The mean monthly air temperature of the project area ranges from 13.05 to 16.5oC with an average of 14.70C. The average of minimum and maximum air temperature is 7.6 oC in November and December and 23.4 0c in March respectively. The mean monthly average relative humidity ranges from 70.3 % (in March) to 76.7% (in August) and its mean value is 72.6%. The monthly sun shine hours fluctuates from 6.7 (in July) to 9.2 hrs (in March) with its mean value of 8.2 hours.  The minimum and maximum value of the wind speed area 0.5 m/s (in September) and 1.4 m/s (in November & December).
Station: Munessa, Altitude: 2620masl; Latitude=7.590; Longitude =39.170
Table 2.1:-Metrological Data of Lega-Lola SSIP
	Month
	Tmin
	Tmax
	Mean T
	Wind Speed (m/s)
	Relative Humidity (%)
	Sun shine (Hr)
	Rainfall (mm/an)

	Jan
	8
	21
	14.5
	1.3
	71.3
	8.4
	18.6

	Feb
	8.9
	22.5
	15.7
	1.2
	70.7
	8.8
	38.2

	Mar
	9.5
	23.4
	16.5
	1.1
	70.3
	9.2
	69.3

	Apr
	9.6
	23
	16.3
	1.2
	71
	9
	87.7

	May
	9.3
	22.6
	16
	0.9
	71.3
	9.1
	82

	Jun
	9
	20.7
	14.9
	0.9
	73.3
	8.3
	90.2

	Jul
	8.3
	17.8
	13.1
	1.4
	76.7
	6.7
	144.3

	Aug
	8.3
	17.9
	13.1
	1.1
	76.7
	6.8
	148.4

	Sep
	8.4
	19.5
	14
	0.5
	74
	7.7
	97.4

	Oct
	7.9
	19.8
	13.9
	1.2
	73
	8
	48.7

	Nov
	7.60
	20.6
	14.1
	1.4
	71.3
	8.5
	15.3

	Dec
	7.60
	20.7
	14.2
	1.4
	71.3
	8.4
	11

	Total
	 
	 
	 
	 
	 
	 
	851.1

	Min
	7.6
	17.8
	13.1
	0.5
	70.3
	6.7
	11

	Max
	9.6
	23.4
	16.5
	1.4
	76.7
	9.2
	148.4

	Mean
	8.5
	20.8
	14.7
	1.1
	72.6
	8.2
	70.9


3.3 Topography
The area consists of gentle slope to strongly sloping topography and the slope range from 0 to 30%.  The dominant topography of the project area is falling into sloping to strongly sloping, varying from 8 to 30% that covers about 151hectares of land and forms 68% of the total area. The project lies between 3021 and 3070 masl. 
Table 2.1:- Slope Class of Lega-Lola Small Scale Irrigation Project
	Slope Class (%)

	Description
	Area

	
	
	Ha
	%

	0-4
	 Gentle Slope
	26
	11.5

	4-6
	Moderate Gently slope                      
	23
	10.4

	6-8
	Sloping       
	23
	10.3

	8-20
	Strongly sloping                
	119
	53.5

	>20
	Moderately Steep
	32
	14.4

	Total
	
	222
	100


[image: image5.jpg]Slope Map of Lega Lalo Small Scale Irrigation Project

B12000 512200 $12400  $12600  $12800  $13000 513200 813400 H13600  £13800 14000 14200

11800

494100 494400 494700 495000 495300 495600 49590 496200 496500 496500 497100 497400 97700
LAz
A
LA
A
Legend
AT A Observation
A House
° Tukul
Main canal
[JBoundary
Slope classification  Area(ha
0-4 255
s 20
s 238
g 00 -2 18.8
Teters = 9
T T
494100 494400 494700 495000 495300 495600 495900 496200 496500 496500 97100 97400 97700

812000 512200 312400 12600 $12800  $13000 513200 813400 513600 $13800 14000 14200

11300





Figure2.1:- The Slope map of Lega-Lola Small Scale Irrigation project area

3.4 Agriculture, Vegetation and Land Use

The peasants in the study area engaged in small-scale subsistence farming employing simple farming tools and methods of production.  The dominant cropping system composed of potato, pea, barley, and onion.  The minor crops are potatoes, barley, pea and onions.  In the study area is practiced mixed farming.  The average land holding of the project area is nearly 2 ha.  The agricultural activities includes land preparation, which is done by oxen for several times, planting that is carried by broadcasting, weeding is labor intensive work and harvesting.  Large numbers of farmers apply mineral fertilizers, namely Urea and DAP to maintain soil fertility.

The considerable part of the study area is under cultivation, while the remaining part is grassland and settlement area.  The vegetation covers mainly concentrated around the homestead.  The vegetation types are eucalyptus and very Juniperusproceramcypress, Eucalptusglobuls, MaytenussenegalensisandAmeraro (0rm.), endod
4. METHODOLOGY AND APPROACH OF THE STUDY

The FAO Guidelines (2006) for soil description was fully employed in the current field soil survey. Furthermore, the soils of the project area have been classified using the FAO-WRB (2006) legend for soil resources classification. The overall soil survey work divided into three stages described as follows. 
4.1 Pre-fieldwork

This stage of the entire soil survey work is the preparatory stage and therefore, all the necessary preparatory tasks for the study were made. Field soil field data collection formats for the soil description were prepared at this stage.

Previous Studies

Previously general soil studies made at national level and river basin level.  The country wide studies carried on nationwide as part of the Assistance to Land Use Planning (ALUP, 1984) and produced Geomorphology and soil map of Ethiopia 1:1 million scale based on Land Sat MSS imagery, data from existing reconnaissance level soil survey and limited amount of field observation.  LUPRD, in 1984 by FAO/UNDP produced a soil association map at 1:2 million scale.

The Wabe River Basin Development Master Plan Study was under taken by Ministry of water Resources in 2004 and produced a soil map of the basin a t scale of 1:250,000.  A site-specific study did not made in the study area.

4.2 Fieldwork stage activities

The field survey was carried out from the 08 to 09 April, 2016. The study was carried out according to technical proposal that encompasses auger observation and description of profile pit. 

The field observation comprised description of profiles of pits, auger holes, and collection of soil samples for laboratory investigations. Each observation was described systematically according to the FAO guidelines for soil profile description. The auger soil description was made up to the depth of 1.20 m except where restricted by lithic contact and pits were excavated up to 2m depths on representative sites.  This was to describe properly the main soil types identified by routine auger survey. The soil profile exposed by digging a vertical pit with dimensions of about 1m width, 2m length and 2m depth.   The soil profiles were described in situ following Guidelines for Soil Profile Description (FAO, 2006).  

Detailed information was collected during the field work which described each of the identified land unit considering geomorphology, soil, land use/land cover and vegetation. All observations of auger and profile pits were recorded on soil description sheets prepared for the purpose. For each observation site the external and internal characteristics and information were recorded. The external characteristics such as  coordinates noted by using hand held GPS, land use /land cover, preliminary soil classes, land form, slope  measured with the help of clinometers, presence of micro topography, surface cracks and surface crusting, soil color by using Munsell color charts, parent material, presence /absence of flooding by interviewing local people, presence or absence of stoniness, erosion hazard (by estimation), elevation determined by altimeter, presence of carbonates tested by 10% diluted HCl and ground water table level.

The internal soil characteristics such as soil depth (to rock or stony layer), texture, mineral nodules (Fe, Mn), mottling, and depth to water table were noted for each auger hole, description sheets. In the case of soil profile pits additional internal soil information such as soil horizons, structure, consistence, porosity, roots, cations, slickenside etc. were recorded on profile description sheets. A total of 11 auger, and 2 profiles were described, out of which 7 samples were sent to the Soil laboratory. 

4.3 Post Fieldwork Stage

Two soil profile pits were excavated and described at different slope gradients on various positions of the landscape and 7 samples were collected from identified genetic horizons. The samples were processed and analyzed for physicochemical properties, following standard laboratory procedures: 

· pH H2 O in 1:2.5 

· Organic carbon and total nitrogen

· Available P, 

· Electrical conductivity (ECe) of saturation extract,

· Cation exchangeable capacity (CEC),

· Exchangeable Na+, Ca2+, Mg2+, K+, 

· Free calcium carbonate,

· total nitrogen,

· Textural analysis,

Then the Results of physical-chemical data for analyzing profiles are presented in Appendix

5. SOIL PHYSICAL CHARACTERISTICS
5.1 Effective Soil Depth

Effective soil depth is the depth of the soil that may be exploited by plant roots. A deep well drained soil show a root penetration until below 150 cm for most cops. For annual crops the dense root systems is usually at depth of less than 60 cm, while most tree crops even have a dense to moderate root system until the depth of 150 cm. Soil depth is judged by contacts to non-soil material. 

The soils that found in the project area are deep to very deep on flat to gentle undulating topography, while on steep topography is shallower.  Therefore, the depth would not be a problem in some portion of the study area.

5.2 Soil Texture
Soil texture is very important parameter among soil physical characteristics.  Soil texture analysis was carried out in the field using the feel method and by the laboratory investigation.  The texture of the investigated soils varied from clay loam to sandy clay on topsoil, while in subsoil silty loam. The overall sand fraction ranges from 13 to 37% with mean value of 23%, the silt content varies between 20 and 72% with an average value of 53%, and the clay content varies from 15 to 43% with an average of 25%. Thus, the predominant soil textural class is clay over silty loam.

5.3 Soil Structure

The soil structure of the site is granular in topsoil while in sub soil is sub angular structure. The upper layer of the studied soil characterized by weak fine and medium platy structure, meanwhile in lower horizon the soil was found to be medium to coarse sub angular, and angular blocky and prismatic structure is found in top soil.
5.4 Soil Bulk Density

Bulk-density of a soil is the weight of a known soil volume compared to the weight of an equal volume of water, or weight per unit volume.The major purposes of determining soil bulk density are:

·   Evaluation of root impedance possibilities, as relation-ships has been established   between high bulk-density and lack of root penetration.  

· Bulk density is above 1.75g/cm2.  For sandy and 1.46-1.63 gm/cm3 of silty and clays are quoted as causing hindrance to root penetration.
The bulk density of the soils in Lega-Lola small scale irrigation project area is calculatedranges from1.29 and 1.41 g/cm3.
5.5 Field capacity and Permanent Wilting Point

Field Capacity (FC) of a soil is the maximum water content the soil will hold following free drainage.  FC is the upper limit of available soil moisture under irrigation. The field capacity of the area ranges from 33 to 39%
Permanent Wilting Point (PWP) is the soil moisture content at which plants can no longer obtain enough moisture to meet transpiration requirement and remain wilted unless water is added to the soil. The value of PWP of the site is between 16 to 26%.
Available water capacity (AWC) the volume of water retained between field capacity (0-33 bar) and permanent wilting point.  However, all available moisture is not accessible to plants due to imperfect drainage, hydraulic conductivity of soil, rolling depths, root concentration at different depths and stage of plant growth.  About 50-70% of available moisture is considered readily available water (RAW). As a rule readily available moisture is considered 60% to total available water.  

For measurement of field capacity and permanent wilting point soil samples to National Soil Research laboratory. However the investigation has been failed due to reasons connected to the laboratory personnel.By considering the content of clay texture the available water capacity the soils of the studied soilscalculated to be 204 mm/m. 

5.6 Infiltration

Infiltration refers to the measurement of vertical intake of water into a soil at the soil surface.Infiltration is important for selection of suitable methods and design for irrigation type, calculation of deep percolation losses, irrigation efficiencies and crop water management.  It also helps in interpretation of soil erosion and conservation requirements. Optimum basic infiltration rates for surface irrigation are considered to be in the range of 0.7 - 3.5 cm/hr, although acceptable values normally range is from about 0.3 to 6.5 cm/hr.  Infiltration rate of Lega-Lola Small Scale irrigation project 2.3 cm/hr.

5.7 Permeability

Hydraulic conductivity (permeability) of the soil is the property to transmit water down ward through unit cross-section area of a soil in unit time.

The hydraulic conductivity of the saturated soil governs the rate of drainage.  The average hydraulic conductivity of a soil profile is used to determine subsurface drainage (if there is drainage problem) and to evaluate the feasibility of rising perched water table conditions, which may injure crop roots.The HC of major soils in Lega-Lola small scale irrigation project is 0.01-0.4 m/day. Therefore, the soils of the project area have slowto moderate permeability and would create some drainage problem under irrigation management.
6. CHEMICAL CHARACTERISTICS
6.1 General

Soil chemical properties have a great influence on planning soil and agronomic programs best suited to the proposed irrigation projects and evaluation and monitoring of the schemes. The soil chemical characteristics of each soil-mapping unit has been analyzed and discussed based on laboratory analytical results of soil samples collected in the field. (See the results in annex 2).
6.2 Soil Reaction

Soil reaction is an important function in the formation and behavior of different types of soils especially under tropical conditions. Soil pH is an indicator of acidity and alkalinity in the soil.  The pH of the soil determines availability and toxicity of macro and micronutrient and used to appraise correctable soil deficiencies relating to economic correlation such as lime for acid soil and gypsum for sodic soils. Soils are considered slightly acid between pH 6.5 to 6.1, moderately acid between 6.0 and 5.5, strongly acid between 5.5 and 5.1, very strongly acid between 5.0 and 4.5, and extremely acid below pH 4.4. Soils with pH values above 7.4 are rated as alkaline. As the value of pH drops below 5.5, the concentration of soluble aluminum increases and becomes toxic to plant root growth when it exceeds 1.0 part per million (ppm). Below pH 5.2, the concentration of manganese also increases and can become toxic. The availability of fertilizer nutrients such as nitrogen, phosphorus and potassium generally is reduced as soil pH decreases. Generally, for agricultural purpose, soils with pH values within the range of 6.0 to 7.5 are suitable and more trouble free than those with higher or lower pH values (Hall, 2008; Hazelton and Murphy, 2007).

Cause of Acidity

The cause of acidity of command area might be the following:

· Acid soils are mostly found in the areas of high rainfall. Rainfall affects soil acidity development by leaching basic elements such as calcium, magnesium, potassium, and sodium from the soil profile, leaving the acidic elements: hydrogen, aluminum and manganese. In addition, soil erosion can contribute to the loss of basic elements thereby increasing soil acidity.

· Management factors such as cropping and fertilizer use also affect soil pH. Removal of harvested crops depletes basic elements from the soil. Nitrogen and phosphorus fertilizers also contribute significantly to the formation of acid soils in particular Urea and DAP. Similarly, in the project area the farmers intensively are using inorganic fertilizers. 

· Sloppy places often acidic in nature. The major part of the Lega-Lola area is characterized by sloppy land form;

The pH value of surface varies from 4.7 to 5.2 with mean value of 5.0 and in the pH value of the subsoil is also similar to upper horizon, both cases indicating that the soil reaction is strongly acidic. So the soil requires amendment measures like lime application.Since the value of pH value of the project area is below 5.5 then the aluminum toxicity might present. Excessive amounts of aluminum can inhibit root development and limit crop growth.

Aluminum and manganese toxicity may be ameliorated by liming and aerating fields. Aeration may be accomplished by draining excessive water from fields. As a result proper management of soil pH can prevent problems associated with aluminum toxicity.  The results obtained showed that the soil pH in all soil units is out normal pH range with low pH values. Under such low pH the availability of essential nutrients are critically affected. Toxicity of aluminum to plants greatly affects root and shoot growth as well as nutrients and water absorption. Moreover, the activities of microorganisms which play pivotal roles in nutrient cycling in agro ecosystems are affected. So, the soil requires amendment measures like lime application.
6.3 Electrical Conductivity

Electrical Conductivity measurements (EC) are used as indicators of total quantities of soluble slats in soil.The EC value of all soil types in the project area is indicating absences of any current and potential salinity problems.
6.4 Cation Exchange Capacity (CEC)

Cation exchange capacity is the capacity of soil to sorb or hold cations and supply them to plant.  It is an important soil property for lot soil fertility nutrition studies and for soil genesis.  The CEC value of the survey area is high (26.4-30.5 meq/100g soil).  Generally, high CEC is correlated with presence of primary weather able minerals and accumulation of secondary clay minerals and indicate a high plant nutrient storage capacity.

6.5 Percentage Base Saturation (PBS)

PBS is the value of exchangeable bases divided by cation exchange capacity and expressed in percentage.  The value of BPS is used in soil classification and soil fertility analysis.The BPS value of the project soils is between 14 and 65% with mean value of 38%, indicating that the soil fertility status of soils of the surveyed area is low.
6.6 Exchangeable Cations

The levels of exchangeable cations in a soil indicate the existing nutrient status and can also be used to assess balances amongst cations.  Many effects on soil structure and on nutrient uptake by crops are influenced by the relative concentration of cations as well as by their absolute levels.  Rating of exchangeable cations (meg/100g of soil) (see annex).
6.7 Exchangeable sodium

Sodium is a mineral that is generally not needed in plants. A few varieties of plants need sodium to help concentrate carbon dioxide, but most plants use only a trace amount to promote metabolism. If the exchangeable sodium is less than 3 meq/100g soil there is no adverse effect on soil property. The overall content of exchangeable sodium of the study area ranged from 0.9 to 1.1 meq/100g soil. Hence, there would not be any constraint on soil structure. 
6.8 Exchangeable Potassium

Exchangeable K, 0.1 to 2% of total soil K, is the major bioavailable form of K in the soil. Cation exchange capacities of soils typically range from 5 to 60 milli-equivalents/100grams, and K is typically 1 to 3% of the total CEC, rarely more.The potassium status of the soils in Lega-Lola irrigation project varies from 1.7 to 2.6 meq/100g soil on topsoil but it reduces it reserve in lower horizon. Generally, the level is very high level of exchangeable potassium and there is sufficient potassium. 
6.9 Exchangeable calcium

Calcium is an essential element for the growth and functioning of root tips. It helps the movement of carbohydrates from one part of the plant to the other and for helping to regulate sap acidity. Calcium is present in the cytoplasm at level. According to the analytical results, the exchangeable calcium content of the soils very high (12.5 -13.6 meq 100g soil) but it decreases in lower horizon. Since easily available for plant is the reserve of the topsoil, hence there is no shortage of calcium.
6.10 Exchangeable Magnesium

Magnesium is a constituent of chlorophyll. It is with other nutrient elements, a deficiency of magnesium results in characteristic discoloration of leaves.   The soils of the surveyed area have moderate magnesium content with value that ranges from 2.3 to 2.8 meq/100g soil.
6.11 Organic Carbon
Organic carbon measurements are often made as a measure of the quality of organic matter in a soil, which in turn is taken as a crude measure of fertility status. In the study area, the organic carbon content ranged from 0.1 to 0.7% throughout the horizons. The organic carbon content invariably decreases with depth while the decrease at times being that much significant. The OC values are in general low to very low level and do not contribute much to soil fertility. The low organic carbon content in the area can be ascribed by low plant cover. 

The value of organic matter was obtained by multiplying the organic carbon content by factor 1.72. The threshold value of organic matter is above or equal to 1.4%.  The organic carbon is high (3%).  It decreases considerably from upper surface to subsurface.  

6.12 Total Nitrogen

Nitrogen is an essential nutrient element, which highly influences the plant growth.  It is a constituent of chlorophyll, plant proteins and nucleic acid. The total nitrogen of the studied soils is around 0.3-0.4% in the topsoil.  The overall status of the total nitrogen is very high in surface soil, and minimized in lower horizon. 

6.13 The Carbon to Nitrogen Ration
C/N ration is an indication of the process of transformation of organic nitrogen to available N like ammonium nitrite and nitrate-N.

Incorporation of partially decomposed crop residues or straw in a soil sample can greatly increase whilst legume residues decrease the ration and as such the figures should be taken as indication of facts10 and the microbiological activity are expected to be in a favorable condition.  The C/N ratio of the project area is between 9 and 11.

6.14 Exchangeable Sodium Percentage

Exchangeable Sodium Percentage (ESP) indicates sodicity in the soil.  Soils with ESP<15 is generally non-sodic requiring no amendments, whereas soil with ESP>15 are sodic and requires amelioration method.  ESP of the soil project area is between justifying absence of any sodicity problems.

6.15 Available Phosphorus

Available Phosphorus is the amount of P readily available for nutrient absorption by the plant. Phosphorus is available in all soil in the project area in a trace amount (<4Pmm).  Hence there is an urgent need for application of Phosphorus fertilizer under proposed irrigation.

7. SOILS OF THE PROJECT AREA
7.1 Soils, Their Characteristic and Classification

Climate, relief, parent material, organisms, and time govern the soil formation process. The interactions of these factors produce the soil entity.

Soils of the project area generally classified based on hierarchical system at two levels.  There is one major soil group as per the WRB-FA-ISRIC soil classification of the world0f 2006.  This soil unit is Haplic Cambisols (Eutric, Siltic). This soil unit subdivided into different phases, which indicate surface and subsurface features of landform.  The soil series mapped and described in the site by combination of geo-morphic and soil characteristics.  Generally soils of the project are grouped into one major soil namely Haplic Cambisols (Eutric, Siltic).  They are clay over silty loam soils with well drained. These soils are deep to very deep, Very dark brown (10YR2/2) in color.  They are regrouped into five series based on slope features. 

Table 6.1:-Legend of Soil Mapping Unit Lega-Lola Small Scale Irrigation Project

	Sr No
	Legend
	Area
	Description

	
	
	Ha
	%
	

	1
	SMU-1
	17.7
	7.9
	Very deep, Very dark grayish brown (10YR3/2) in color, Loam texture, moderately drained, weak medium platy structure, friable when moist, sticky and plastic when wet, strongly acidic occur on very  gently undulated plain with slope range of 0 – 4% (Haplic Cambisols (Eutric, Siltic).  

	2
	SMU-2
	32.7
	14.7
	Very deep, Dark brown (10YR3/2) in color, clay loam in texture, moderately drained, weak medium platy structure, friable when moist, sticky and plastic when wet, strongly acidic occur on a undulating plain with slope range of  4 - 6 % (Haplic Cambisols (Eutric, Siltic)).

	3
	SMU-3
	24.5
	11.0
	Very deep, Dark brown (10YR3/2) in color, clay loam in texture, moderately drained, weak medium platy structure, friable when moist, sticky and plastic when wet, strongly acidic occur on moderately undulating with slope range  of 6- 8% (Haplic Cambisols (Eutric, Siltic)).

	4
	SMU-4
	70.4
	31.6
	Very deep, Dark brown (10YR3/2) in color, clay loam in texture, moderately drained, weak medium platy structure, friable when moist, sticky and plastic when wet, strongly acidic occur on rolling topography with slope range of 8 – 20% (Haplic Cambisols (Eutric, Siltic)).

	5
	SMU-5
	77.7
	34.8
	Very deep, Dark brown (10YR3/2) in color, clay loam in texture, moderately drained, weak medium platy structure, friable when moist, sticky and plastic when wet, strongly acidic occur on moderately steep side slope  topography with slope range of  20 – 30% (Haplic Cambisols (Eutric, Siltic)).

	
	Total
	223
	100.0
	


[image: image6.jpg]Map of Lega Lalo

B1600 811800 $12000  §12200  $12400 612600 12800 #1300 §13200 13400 613600 813500

11400

494100

494400

494700

495000

495300

495600

496200 496500

496800 497100 497400 497700

U3

Spu-3.

SMu-1

{

snug -

Liats SMUS
~

wars®
R

SMU-A

sMuds

smy-1

SMUBA | (iate

SMU4.

SMu5.

)

LAt

suuh

P01 SMU3

==

SMU-4.

SMU3.

Legend

SMU5.

LAt

LAtz =5

2 LL;\_,J\-/

—
mu%
LE/ A Observation

A House
o Tukul

Main canal

[ Boundary

200 | 100

200

=

Soil

Mapping Unit Description

smu-t

smu-2

RTES

smus

smus

Very deep, Very dark grayish brown (10YR312) in color, Loam texture, moderately
drained, weak medium platy structure, friable when moist, sticky and plastic
‘when wet, strongly acidic occur on very gently undulated plain with slope range.
010 - 4% (Haplic Cambisols (Dystric, Siltic). (Area = 17.7 ha)

Very deep, Dark brown (10YR312) in color, clay loam in texture, moderately
drained, weak medium platy structure, friable when moist, sticky and plastic
when wet, strongly acidic occur on a undulating plain with slope range of
4-6% (Haplic Cambisols (Dystric, Siltic)). (Area = 32.7 ha)

Very deep, Dark brown (10YR312) in color, clay loam in texture, moderately
drained, weak medium platy structure, friable when moist, sticky and plastic
when wet, strongly acidic occur on moderately undulating with slope range
of 6-8% (Haplic Cambisols (Dystric, Siltic)). (Area = 24.5 ha)

Very deep, Dark brown (10YR312) in color, clay loam in texture, moderately
drained, weak medium platy structure, friable when moist, sticky and plastic
‘when wet, strongly acidic occur on rolling topography with siope range of
8- 15% (Haplic Cambisols (Dystric, Siltic)).(Area = 70.4 ha)

Very deep, Dark brown (10YR312) in color, clay loam in texture, moderately
drained, weak medium platy structure, friable when moist, sticky and plastic
‘when wet, strongly acidic occur on moderately steep side slope topography
with slope range of 15 - 30% (Haplic Cambisols (Dystric, Siltic)). (Area =77.0 ha)

494100

494400

494700

495000

495300

495600

495900

496200 496500

496800 497100 497400 497700

B1G00 811300 812000 $12200  $12400 512600 812800 813000 $13200  $13400 _ $13600 813800

11400





Figure 6.1:- Map of Soil Mapping Units of Lega-Lola Small Scale Irrigation Project

8. LAND SUITABILITY EVALUATION
8.1 General

The land suitability evaluation has been conducted by employing FAO methodology. The FAO Framework for Land Evaluation is an approach based on the following concepts: i) the land is qualified, not only the soil. ii) Land suitability must be defined for a specific soil use in this case for coffee plantation, iii) Land evaluation was to taken into account the physical conditions. v) The evaluation requires a comparison between two or more alternative kinds of use. vi) The evaluation must propose a use that is sustainable.  In the approach four categories are recognized. The highest category is the order that it reflects, in broad features, whether a soil is suitable or not for a given use. Two orders are recognized:

S = (Suitable): Land in which the benefits exceed the costs and sustained use does not incapacitate the soil over a sufficiently long period of time.

N = (Not suitable):  Land can be classified as not suitable for a certain use for diverse reasons. The use proposed may be deemed technically impractical, as in irrigation of abrupt rocky terrain, or that it causes serious environmental degradation, as in cultivation on steep slopes. Frequently, however, the reason is economic, in that the profit expected does not justify the cost required.         
The second category is the class that reflects degrees of suitability within the order. These are numbered consecutively in Arabic numerals.  For the order S, three classes are considered:

· Highly suitable (S1) = without limitations for sustained use or minor limitations that do not affect productivity nor appreciably increase costs.

· Moderately suitable (S2) = moderately serious limitations that reduce profits or involve risks of degradation in the sustained use of the soil.

· Marginally suitable (S3) = the limitations for the sustained use are serious and the balance between the costs and benefits make the use only marginally justifiable. Its use is normally justified on other than economic grounds. 

 In the order Not Suitable (N) = three classes are also recognized:

· Not currently suitable (N1) = Land with limitations that could be eliminated by technical means or investment, but that these changes are at present unfeasible.

· Permanently unsuitable (N2) = Serious limitations of generally a physical nature, which are assumed to be beyond solving over the long term.

· Land for conservation  (X ) = Unsuitable for exploitation, being lands of special protection, due to their conservation, wildlife, of special scientific, ecological or social interest (e.g., parks, reserves or recreational zones).

The limits between the orders (S and N) and between the different classes (S1, S2, S3 and N1, N2) are established by the presence of limiting factors. One limiting factor is a characteristic of the soil that hampers its use, reduces productivity, increases costs and implies degradation risk, or all of the above.
These limiting factors are used to define the third category of the system, which is the subclass. In the symbol of each subclass, the number of limitations involved should be kept to the minimum one letter, or, rarely, two. The limitations identified in the studied area are: t= slope; n= soil nutrient availability; z=soil acidity.

In the study used the FAO framework for land suitability evaluation.  The following stepwise approach applied to:

· Definition of the result LUTs in the area:

· Determination of those land qualities, which have direct influence on the     production of the LUT

· Rating of these land qualities by means of a single or a set of land characteristics

· Matching that land use to teach at current land suitability classification. 

8.2 Land Utilization Types (LUT)

The existing major land utilization type in the survey area is small-scale rain fed agriculture, mixed farming, which is widely practiced in the project area, with low-level technical knowledge and land fragments.  In rain fed agricultural individual crops are considered as the basis for separate land utilization type.Land evaluation for surface irrigation has been made for small holders’ irrigation under medium input and support from government. The descriptions of land utilization and land use requirements are given in the following table

Table 7.1:-Land Uses Requirement and Land Quality Criteria for Surface Irrigation

	Land Quality
	Unit
	Suitability Classes

	
	
	S1
	S2
	S3
	N1
	N2

	Slopes
	%
	1-2
	0.5 - 3
	3 - 5
	5 - 8
	>8

	Drainage
	
	Well
	Moderately well
	Imperfect
	Excessive, poor
	V.poor

	Depth
	cm
	>200
	120-200
	60-120
	30-60
	<30

	Soil texture
	
	SiL&CL
	C, SCL
	L, S, HC
	
	

	Salinity
	Ms/cm
	<4
	4-8
	8-12
	12-16
	>16

	CEC
	Meq/100g
	>20
	5-20
	<5
	
	

	OM
	
	3-5
	1-3
	<1
	<1
	

	N
	
	>0.5
	0.2-0.5
	0.1-0.2
	
	<0.1

	P
	ppm
	>15
	5-15
	<5
	
	<2

	C/N
	
	10-12
	6-10
	<6
	<6
	

	PH
	
	6.5 - 7
	6.5 - 7
	7 - 8
	< 5 &< 8.5 
	

	Structure
	
	SAB
	SAB
	Pla.
	Mas.
	

	Consistence
	
	SlSt
	St, Pl
	VSt.

Vp1
	VSt. VPl p1.
	

	Erosion status
	
	None
	Medium/slight
	Severe
	Very severe
	

	ESP
	
	<10
	10-15
	15-20
	>20
	>20

	Stoniness
	%
	<1
	1-3
	3-15
	15-40
	>40

	Vegetation
	No clearing requirement
	Scattered
	Frequent
	Forest
	Dense forest
	Dense forest

	Micro relief 
	
	Even
	Slight
	Uneven
	
	Broken

	Water logging 
	
	None
	Rarely
	Frequent
	Frequent
	


SAB=Sub Angular Blocky, Vst= Very sticky, VPl = Very Plastic, mS/cm=milisimence per centimeter,  SlSt=Slightly Sticky, SlPl= Slightly Plastic, Pla=Platy, Mas= Massive, SiL= Silty Loam,   SCL= Sandy Clay Loam,   L= Loam, S=Sand, HC=Heavy Clay, C=Clay

By matching diagnostic factor with irrigated agriculture the land suitability of the study area classified as the following (see table 8.3).


Table 7.2:-Soil Characteristics of the Soil Mapping Units
	Land Characteristics
	
	Mapping Units

	
	Units
	SMU-1
	SMU-2
	SMU-3
	SMU-4
	SMU-5

	Area
	ha
	26
	23
	23
	79
	72

	Mean Air Temperature
	0C
	14.7
	14.7
	14.7
	14.7
	14.7

	Relative Humidity
	%
	72.6
	72.6
	72.6
	72.6
	72.6

	Slope
	%
	0-4
	4-6
	6-8
	8-15
	>15

	Drainage
	 Degree
	Mo
	Mo
	Mo
	Mo
	R

	Depth
	Cm
	200
	200
	100
	70
	50

	Texture
	 
	L
	SCL
	SiL
	SiL
	SiL

	Stoniness
	%
	None
	None
	None
	None
	None

	Salinity
	ds/cm
	None
	None
	None
	None
	None

	CEC
	Meq/g
	26.4
	30.5
	30.5
	30.5
	30.5

	O.C
	%
	3.26
	3.31
	3.31
	3.31
	3.31

	pH
	 
	4.2
	4.9
	4.6
	4.6
	4.6

	Erosion Status
	Degree
	Sl
	Mo
	Mo
	Mo
	Sev

	ESP
	%
	None
	None
	None
	None
	None


SiL=Silty Loam, SCL= Sandy Clay Loam, L= Loam,Sl=Slight, Mo=Moderate, Sev= Sever


R= Rapid
Matching of land use with land once the land resources or soils have been mapped, the next step is matching the requirements of each land utilization type or crop with the qualities of each mapping unit. The matching processes of the study see in table 7.3.
Table 7.3:-Matching Process of Land Units and LUTs (surface Irrigation System)
	Soil Characteristics 
	Unit
	Mapping Unit

	
	
	SMU-1
	SMU-2
	SMU-3
	SMU-4
	SMU-5

	Area
	Ha
	26
	23
	23
	79
	72

	Slope
	%
	S2
	S3
	N1
	N2
	N2

	Drainage
	
	S1
	S1
	S1
	S3
	S3

	Depth
	Cm
	S1
	S2
	S1
	S1
	S3

	Texture
	
	S1
	S2
	S1
	S1
	S1

	Stoniness
	%
	S1
	S1
	S1
	S1
	S1

	Salinity
	
	S1
	S1
	S1
	S1
	S1

	CEC
	Meq/100g
	S1
	S1
	S1
	S1
	S1

	OM
	%
	S1
	S1
	S1
	S1
	S1

	pH
	
	S3z
	S3z
	S3z
	S3z
	S3z

	ESP
	%
	S1
	S1
	S1
	S1
	S1

	Erosion status
	
	S1
	S2
	S3
	S3
	S3

	Workability
	
	S1
	S1
	S3
	N1
	N2

	Current suitability under furrow irrigation
	
	S3z
	S3z
	S3z/w/t
	N1t/z
	N2t/z

	Potential suitability under sprinkler irrigation
	
	S2Z
	S2Z
	S2Z
	S3t/w
	N1t/w


8.3 Land Suitability Evaluation by Land Utilization Types

The suitability of a land area was evaluated for major kind of land use. In regard to the Lega–Lola Small Scale Irrigation Project, in the actual land evaluation process has been identified out of the total area of 222 hectares of land 18 ha (approximately 8%) and 53 ha (about 26%) is found to be moderately and marginally suitable (S3t/z) for surface irrigated agriculture, whereas 117 ha and 34 ha currently unsuitable (N1t) and   permanently (N2t) not suitable for gravity irrigation due to steep slope and prevalence of strong soil acidity respectively (See Fig 7.1 & Table 7.4). 

However, by employing corrective measures to overcome the soil acidity problem like lime and using overhead irrigation system (sprinkler) instead of surface irrigation (furrow irrigation method) in line with by constructing soil conservation structure, such as bench terrace, it is possible to upgrade to improved suitability class. As a result the land suitability level 55 hectares of land, which constitute 25%, would be highly suitable (S1). Furthermore 16 ha and 117 ha classified moderately suitable (S2t/z) and marginally suitable (S3t) respectively. Henceforth, 188 hectares of land (85 out of the total land) can be suitable for sprinkler irrigated agriculture. While, the remaining 34 hectares of land which forms about 15% is not suitable for irrigation for the reason that of high slope gradient.  The main limitations of the area are steep slope and acidic soil reaction.

Table 7.4:-Summary of Current and Potential Suitability of Lega-Lola Small Scale Irrigation Project 

	Mapping Unit
	Area
	Current suitability
	Potential Suitability

	
	Ha
	%
	
	

	SMU-1
	17.9
	7.9
	S2z
	S1

	SMU-2
	32.7
	14.7
	S3z
	S1

	SMU-3
	24.5
	11.0
	S3z/w/t
	S2z/t

	SMU-4
	70.4
	31.6
	N1t/w
	S3t/w

	SMU-5
	77.7
	34.8
	N2t
	N1t

	
	222
	100.0
	
	


[image: image7.jpg]Land Suitability Map for Surface Irrigation of Lega Lalo SSIP

BMG00 B30 $12000  $12200  $12400 512600 12800  $13000  $13200 #3400  $13600 813500

11400

198100 103400 108700 195000 195300 195600 195900 196200 196500 196300 7100 s97400 07700
T
Gt ki
saz nat
31
nat [T}
Bziwit SEz @ 2 \
2t n2r
itz
2t e T —/W
nat
i rellia) D
™ \ = /
[
ot L ——
saz A
— X s / M
LE == ot _—j\—_’/\/
it
e Sz
sz Legend
\ A House
e Tukul T
Main canal
[ Boundary
Area I
Land Suitability
ha %
520 moderately suitable 179 80
S3 Marginally Suitable 53.2 240
N1 Temporarly Unsuitable nra 521
N2 Permanently Unsuitable 339 15.3 =
Limiting Factors
=+ soi scity
0 E w: workabilty
e - topography
T
104100 104400 194100 195000 195500 195600 195500 196200 196500 196500 7100 07400 7100

B1G00 _ B11300 812000 $12200 812400 512600 812800 813000 $13200 __ $13400 __ $13600 813800

11400





Figure7.1:- Land Suitability Map for Surface Irrigation System (Furrow) for Lrga-Lola Small Scale Irrigation Project
[image: image8.jpg]Land Suitability Map for Overhead Irrigation of Lega Lalo SSIP

494100 494400 494700 495000 495300 495600 495900 496200 496500 496800 497100 497400 497700

813300

813600

813400

813200

213600 813800

813400

813000

812800

813200

813000

812600

12800

812400

12600

12400

812200

812200

812000

812000

11500

11600

11300

11600

s11400

Legend
A House
© Tukil
= Main canal
[ Boundary
Area
Land Suitability
ha %
[ 548 21
520 modentely suitable 163 73
S3 Marginaly Suitable 7.4 527
N1 Temporariy Unsuitable | 33.9 153
Limiting Factors
* 2 soilacdity
™0 [ w0 200 w: workabilty
Meters t: topography

T
494100 494400 494700 495000 495300 495600 495900 496200 496500 496800 497100 497400 497700

11400





Figure7.2:- Land Suitability Map for Overhead Irrigation System (Furrow) for Lega-Lola Small Scale Irrigation System

9. SOILS AND LAND MANAGEMENT UNDER IRRIGATION

9.1 General

Soil provides nutrients; water and physical support for plants and air for plant roots.Plants require three factors for growth and reproduction: light, water, and nutrients. Inorganic fertilizer is the largest nutrient input, but it is useful to think of chemical fertilizers as supplementary nutrients. When used to supplement biological nutrient sources, inorganic fertilizers can help make more efficient use of other available plant-growth resources, such as water and sunlight. There are four main approaches for improving soil biological and physical health (tillage, cover crops, organic amendments and crop rotation). Thus the main soil and land management requirements are briefly described as follow.

9.2 Soil fertility

The soils investigated in the surveyed area are loam to sandy clay loamtextured with moderately drainage. The serious limitations of these soils are:  hilly topography and uneven surface and acidic soil reaction on all soil units. Concerning soil nutrient status of the project area is medium to high. The soil reaction is strongly acidic with pH value of 4.2-4.9. The soils are rich in soil organic carbon varies from 2.9 to 3.5%. The level of total nitrogen is also very high with value that ranges from 0.3 to 0.35%. 
The CEC is moderatewith value thatranges from 19.1to27.1meq/100gsoil.  The soils have moderate level of potassium with value of 0.5 meq/100g soil. The exchangeable calcium is between 2.6 and 7.2 meq/100g soil, which can be rated as low tomoderate level. The exchangeable magnesium is very low to moderate, which varies from 0.3 to 1.2 meq/100g soil. The percentage base saturation varies from 22 to 34% and rated as very low level.The content of phosphorus is low to moderate, which is at trace level to 9 ppm and requires application of phosphorus fertilizer.Salinity and sodicity of the study area are at low level.  Therefore, they would not cause of limitation for plant growth. Hence, the fertility status of the whole soil units is low to medium level.

In order to maintain soil fertility nutrients removed from the soil by crop must be restored by the application of fertilizers and manure.  Even in a highly fertile soil reserve nutrient gets exhausted as crops are grown and harvested continually and needs replacement.
To maintain organic carbon mulching of crop residues after harvesting should be practiced with application of manure and compost.  The use of plant species that are capable of fixing atmospheric nitrogen can improve soil fertility and reduce dependency on chemical fertilizer. In addition improved agriculture practices such as crop rotation, alley cropping and the use of green manure provide accessory nutrient pool for plant growth.  The application of organic matter also improves the CEC of the soil. Crop rotation is recommended, as it is essential for improving soil productivity.  An alternate appropriate crop in accordance with a pre-established schedule help to keep the soils in good biological condition and to control the erosion risk, increase its water holding capacity, provides to take full advantage of fertilizer, prevent the unbalanced depletion of plant nutrients and counteract development of the toxic substances, if any. 

During the application of inorganic fertilizers balanced dose of macro-nutrient particularly NPK should be added in the form of fertilizers, but it has to be taking into account the availability of nutrient already present in the soil, crop requirement and other factors. Supplying of optimum quantities of nutrient N, P and K with farm yard manure have beneficial effects on the physical and biological situations of the soils.  In order to manage & maintain soil fertility in the project area organic manure and inorganic fertilizers should be utilized together. Therefore nitrogen fertilizers must be given split application depending on the critical growth stage of the crop. Application of green manure increases utilization of phosphorus by the crop not from the added fertilizers but also from the reserve supplies of soil phosphorus.

9.3 Soil Acidity

Since the soils of the studied area are acidic the land managers can manage soil acidity by raising the pH to a desired value through the following methods: 

· Additions of organic matter: Additions of organic matter is a viable option to manage problems associated with soil acidity. 

· Organic matter increases the cation exchange capacity of the soil. As the base saturation increases, the relative amount of “acid cations” decreases;

· In addition, organic matter forms strong bonds, known as “chelates,” with aluminum. Chelation reduces the solubility of aluminum and soil acidity. Again, if your soil is prone to manganese toxicity, it is not suggested that you add organic matter.

· Incorporation of different organic substances;

· Crop residue incorporation;

· Use of farm yard manure like animal dung;

· Additions of wood ash: Like organic matter, wood ash increases base saturation and forms chelates with aluminum.
Conventional Liming: Various liming materials may be added to the soil that neutralize, or counteract, soil acidity. A lime is defined as any calcium containing material which can be added to the soil to increase the soil pH, which can be added to the soil to increase the soil pH. There are different kinds of liming materials available. The most common of these are: Calcites limestone CaCO3. Examples of common liming materials are limestone (calcium carbonate), dolomite (calcium/magnesium carbonate), hydrated lime (calcium hydroxide), and quicklime (calcium oxide). Calcium and magnesium silicates are also used as liming agents.
Liming materials are bases that react with hydrogen ions in the soil solution to form water. In the tropics liming is usually only aimed at preventing Al toxicity, by rising to the pH to about 5.5. The required application of limestone is intended to raise the pH of the top 15 cm of soil through either incorporation or surface application.In the tropics liming is usually only aimed at preventing Al toxicity, by rising to the pH to about 5.5. The required application of limestone is intended to raise the pH of the top 15 cm of soil through either incorporation or surface application.In the case of tropical countries increasing the lower pH level up to 5.5 is enough and based on the soil texture the following lime rate is recommended on manual’s ‘’Simple And Practical Methods To Evaluate Analytical Data Of Soil Profiles, Frank Beernaert INIA, Department of Land and Water FAO 86/010 Maputo, Mozambique, 1990’’. 

Table 8.1:-The Amount of limestone powder (Ton/Ha) to raise the pH
	No.
	Soil Texture
	From pH 4.3 to 5.5

	1
	Sand or Loamy Sand
	0.7

	2
	Sandy Loam
	1.2

	3
	Loam
	1.8

	4
	Silty Loam
	2.8

	5
	Clay
	3.7

	6
	Peat
	8.2


As far as the overall soil texture of the command area is silty loam the amount of lime is 2.8 ton of CaCO3 per hectare, which is equivalent 622 tones limestone (CaCO3) for the whole command area.During application of limethe following cares should be taken:
· Apply the lime on plowed farm plot along the   wind direction, but if possible decide the time of      application to be in the absence of wind;

· Do not leave the applied lime for a long period of time without incorporation of the applied lime in the soil. 

Since lime is fairly slow to react in soil recommend amounts should be applied as early as convenient; liming materials take 2- 6 months or more before needed for a crop to ensure optimum reaction time. One of the best methods of application is to broadcast the lime and thoroughly mix it with the soil during tilling and to repeat the application once in three or four years (upon soil test). The excessive use of lime may induce deficiencies of K, Mg, Fe, Mn, Zn, and Cu, so care should be taken to prevent over liming. The following cares should be taken during application of lime:

· Apply the lime on plowed farm plot along the   wind direction, but if possible decide the time of      application to be in the absence of wind;

· Do not leave the applied lime for a long period of time without incorporation of the applied lime in the soil. 

Since lime is fairly slow to react in soil recommend amounts should be applied as early as convenient; liming materials take 2- 6 months or more before needed for a crop to ensure optimum reaction time. One of the best methods of application is to broadcast the lime and thoroughly mix it with the soil during tilling and to repeat the application once in three or four years (upon soil test). The excessive use of lime may induce deficiencies of K, Mg, Fe, Mn, Zn, and Cu, so care should be taken to prevent over liming.
9.4 Soil Cultivation

The soils identified the project area are heavy textured, imperfectly drained and difficult to work. The poor physical properties of the heavy textured soils are aggravated by poor timing of field operation under land preparation.  Soils should be ploughed when the soil moisture content is not too high or too low.  Taking heavy tractors and ploughing on the land when conditions are too wet will result in increasing bulk density, and surface crusting, reducing soil aeration surface infiltration and soil permeability through compaction. Ploughing can be done 7 - 14 days after heavy rain is recommended.  This plough incorporates the crop residues completely improving porosity and organic matter in the whole of the plough layer.

9.5 Soil erosion

Since the surveyed area issituated on sloppy land the soils are susceptible to erosion.Moderate rill and gully erosion have been also observed in the project site.   Therefore, to alleviate the detected erosion risks andto achieve proper irrigation efficiencies the soil erosion control measure should be implemented. The anti-erosion measures should include agronomic mechanical measures. The agronomic measures comprise contour farming, mulch, zero tillage, crop rotation, strip cropping, alley cropping, mulching and mixed cropping. These measures should be under taken on mild slopes up to 2-4%. On the one hand, the mechanical measures are physical structure such as graded bunding and bench terraces.  These measures should be deployed on higher slopes greater than 5% supplemented with agronomic measures.
9.6 Irrigation Management

Furrow irrigation can be used on flat land (short, near horizontal furrows), and on mildly sloping land with a slope of 2%. On steeper sloping land, contour furrows can be used up to a maximum land slope of 6%. A minimum slope of 0.05% is recommended to assist drainage. 
Sprinkler irrigation is adaptable to any farmable slope, whether uniform or undulating. Sprinkler irrigation is a method of applying irrigation water which is similar to natural rainfall. The lateral pipes supplying water to the sprinklers should always be laid out along the land contour whenever possible. This will minimize the pressure changes at the sprinklers and provide a uniform irrigation. Sprinkler irrigation is suited for most row, field and tree crops and water can be sprayed over or under the crop canopy.In general on steep land sprinkler irrigation system must be used because furrows will break over the lower side as a result the area would be affected by sever soil erosion and could occur gully erosion.  
If in the command area will employ surface irrigation system may aggravate the rate of soil erosion, because substantial proportion of the project area is covered by sloping topography (slope over 6%). Therefore, overhead irrigation system in particular sprinkler with proper soil and water conservation measures would be appropriate for the command area.  On up to 20% slope can be sprinkler irrigation would be effective. 

10. CONCLUSION AND RECOMMENDATION

10.1 Conclusion

Detailed soil survey was carried out in 222ha in Lega-Lola Small Scale Irrigation Project. In the present study, the physical and chemical characteristics of soil were studied.  According to FAO– classification 1 major soil unit was identified. The identified major soil unit is Cambisols. The soils of project area we regrouped into 5 soil mapping units.

The content of organic carbon of the area is very high (3.51%). The level of nitrogen is high with value of 0.35%.  The soil reaction is very strongly acidic (pH=4.2).The area is prone to low pH value. The CEC value of the project soil types is medium (19 meq/100g soils).    The BSP is very low, which shows that the soil may be classified as poor fertile (22%). The content of phosphorus is high (17.4 ppm).   The content of exchangeable Ca and Mg is very low with value of 2.6 and 0.3 meq/100 grams soil. The reserve of K is moderate (0.51 meq/100g soils).  The soil salinity and sodicity was very low.The bulk density of the soils is 1.4 g/cm3.

In the case of present work the average infiltration rate is 2.1 cm/hour, showing that the result is suitable for surface irrigation. The result of the hydraulic conductivity is slow, 0.7 m/day. The value of available water holding capacity of soils within the study area is at 244 mm/m, representing that the AWC is to very high level. The major constraints of the area are the occurrence of steep slope, low nutrient level and strong acidity.

Currently the land was evaluated with respect to its suitability for surface irrigation. As a result of the land evaluation   process it was found that 18 ha and 53 to be moderately and marginally suitable (S3t/z) for surface irrigated agriculture. Meanwhile 117 hectares and 34 hectares of land considered as currently and permanently unsuitable respectively. However, if it would have been taken proper techniques it would possible   to upgrade to better suitability level. The techniques shall include applying lime to overcome acidity, altering irrigation methods from surface to overhead irrigationtechniques and employing soil and water conservation structure like bench terrace. As a result the land suitability processindicated that 55 hectares of land, which constitute 25%, would be scale up to highly suitable (S1). Furthermore 16 ha and 117 ha can classified as moderately suitable (S2t/z) and marginally suitable (S3t) respectively. Henceforth, 188 hectares of land (85 out of the total land) is suitable for sprinkler irrigated agriculture. While, the remaining 34 hectares of land which forms about 15% is not suitable for irrigation for the reason that of high slope gradient.  The key limitations of the area are steep slope and acidic soil reaction.

10.2 Recommendations
Improvement of the existing development limitations requires applying the following amendments:

· To maintain the fertility status Organic and chemical fertilizers must be applied.  Ploughing back of crops residues and mulching should be encouraged to raise very low carbon levels and to improve top soils structure of the soils and competing soil loss. Legumes may also help in getting better topsoil structure, adding nitrogen to soil by fixation from atmosphere and thereby improving the structure of top soils. 

· Exchangeable calcium & magnesium, available phosphorus and the percentage base saturation of the study area have been found to be low. It can therefore, be assumed that the soils of these types within the study area would have high response to the application of fertilizer containing calcium, magnesium, phosphorus and percentage base saturation. 

· Establishing proper anti-erosion structures such as bench terrace, soil bund etc.are necessary to keep the soil and water from removing.  
· Lime application; it improves   acidic soils by increasing of the pH value of the present in the soil. For the total command area estimated to apply about 622 tons of limestone (CaCO3). 
· Mulching of the cultivated land. 

· Applying organic fertilizer. 

11. REFERENCES
1. EA Fitzpatrick, 1983,  Soil Their Formation, Classification and Distribution, Longman Scientific and Technical, Hong Kong
2. EARO/SG2000, 2002,  Proceeding of the Workshop on Phosphorous Soil-Test Calibration Study in Hetosa Wereda, Arsi Zone, Addis Ababa

3. FAO/Frank Beernaert, 1990, Simple and Practical Methods to Evaluate Analytical Data of Soil Profile, INIA, Department of Land and Water, Maputo, Mozambique.

4. FAO, 2006, World Reference Bases for soil survey, Rome

5. FAO.1997, Manual on integrated soil management and conservation practice, soil bulletin 8, Rome.

6. International Livestock Centre For Africa, 1988,  Management of  Vertisols in Sub-Saharan Africa, Proceedings Of A Conference Held At ILCA, Addis Ababa, Ethiopia 31 August-4 September 1987.

7. J. Benton Jones, Jr, 2003 , Agronomic Hand Book, Management of Crops, Soils, and Their Fertility, CRC Press
8. Landon, 1991, Booker Tropical Soil manual /a hand book focus soil survey and agricultural land evaluation in the tropics & sub tropics, Hong Kong.

9. NRCS, Soil Quality – Agronomy Technical Note No. 17, (natural Resource Conservation Services).

10. NRCS, 1996, Soil Quality Resource  Concerns: Compaction, United States Department of Agriculture
11. Richard,  L. Daddow & Gordon E. Warington. 1983, Growth – Limiting Soil bulk Density as Influenced by Soil Texture.

12. Raymond W-miller   1997, Soils in our environment, New Delhi  Roy L. Donahue

13. Tekalign Mamo, Abiye Astatke, K L Srivastava and Asgelil Dibabe (eds),  1993, Improved management of Vertisols for sustainable crop-livestock production in the Ethiopian highlands: Synthesis report 1986-92, Addis Ababa, Ethiopia
14. S.R. Reddy, 2006, Agronomy of Field Crops, KALYANI PUBLISHER,   India.

15. USDA, 2004, Understanding Soil Risks and Hazards, Using Soil Survey to Identify Areas with Risks and Hazards to Human Life and Property.
16. Wassie Haile and Tekalign Mamo, 2009, The Effect of Potassium on the Yields of Potato and Wheat grown on the Acidic Soils of Chencha and Hagere Selam in Southern Ethiopia

	APPENDIX1:-AUGER SITE DESCRIPTION


	Region: Oromiya    Zone: Arsi; Wereda:Munessa; Agrclim Zone:      Dega              Flooding: freq./dur.NoneSrf. Corse frag.&Rockoutcrops: None 

	ID
	Code
	Status
	Date
	Author
	Elevation 
(m.a.s.l)
	Location 
	 
	Land form
	 
	 
	Micro tpoography/ height (m)/coverage%
	Parent material/over
	Effective soil depth(cm)
	Depth to bed rock(cm)
	Surface crack
	Surface Sealing
	Erosion status (T/A/AC/D)
	Drainage: class/ext
	Land cover
	Land use
	Fertilizer:
Type/rate 

	
	
	
	
	
	
	UTM-N
	UTM-E
	Position
	
	Regional 
	Class
	Form
	
	
	
	
	
	
	
	
	
	
	

	1
	LLA1
	AW
	110416
	Fasil
	2854
	812289
	495654
	M
	SH
	S
	08 (17%)
	C
	D
	AL
	D
	>125
	N
	N
	R/1/A/S 
	M/R
	CL2
	CA2
	DAP/200kg/ha

	2
	LLA2
	AW
	110416
	Fasil
	2856
	812330
	495449
	H
	LP
	G
	05
	U
	D
	AL
	D
	>125
	N
	N
	R/2/A/M
	W/M
	CL2
	CA2
	DAP/200kg/ha

	3
	LLA3
	AW
	110416
	Fasil
	2822
	812500
	494971
	D
	CV
	R
	07
	V
	D
	AL
	D
	>125
	N
	N
	R/1/A/S
	M/R
	CL2
	CA2
	DAP/200kg/ha

	4
	LLA4
	AW
	110416
	Fasil
	2820
	812597
	494790
	D
	CV
	S
	08
	C
	D
	AL
	D
	>125
	N
	N
	R/2/A/M
	M/R
	CL2
	CA2
	DAP/200kg/ha

	5
	LLA5
	AW
	110416
	Fasil
	2815
	812579
	494537
	D
	CV
	S
	08
	V
	D
	AL
	D
	>125
	N
	N
	R/2/A/M
	M/R
	CL2
	CA2
	DAP/200kg/ha

	6
	LLA6
	AW
	110416
	Fasil
	2819
	812743
	494364
	M
	SH
	R
	07 (12%)
	C
	D
	AL
	D
	>125
	N
	N
	S/1/A/S
	M/R
	CL3
	CA2
	DAP/200kg/ha

	7
	LLA7
	AW
	110416
	Fasil
	2828
	812756
	494568
	H
	LP
	G
	05
	U
	LE
	AL
	D
	>125
	N
	N
	N
	M/M
	CL2
	CA2
	DAP/200kg/ha

	8
	LLA8
	AW
	110416
	Fasil
	2836
	812766
	494802
	H
	LP
	G
	05
	U
	LE
	AL
	D
	>125
	N
	N
	N
	M/M
	CL2
	CA2
	DAP/200kg/ha

	9
	LLA9
	AW
	110416
	Fasil
	2848
	812639
	495051
	H
	LP
	G
	05
	U
	LE
	AL
	D
	>125
	N
	N
	S/1/A/S
	M/M
	CL2
	CA2
	DAP/200kg/ha

	10
	LLA10
	AW
	110416
	Fasil
	2847
	812596
	495197
	M
	LP
	G
	05(4%)
	C
	LE
	AL
	D
	>125
	N
	N
	S/1/A/M
	M/M
	CL2
	CA2
	DAP/200kg/ha

	11
	LLA11
	AW
	110416
	Fasil
	2864
	812444
	496269
	M
	SH
	R
	07(12%)
	C
	D
	AL
	D
	>125
	N
	N
	R/1/A/M
	M/R
	CL2
	CA2
	DAP/200kg/ha

	12
	LLA12
	AW
	120416
	Fasil
	2868
	812547
	496079
	M
	LP
	G
	05(3%)
	U
	LE
	AL
	D
	>125
	N
	N
	S/1/A/S
	M/M
	CL2
	CA2
	DAP/200kg/ha

	13
	LLA13
	AW
	120416
	Fasil
	2858
	812720
	495761
	M
	LP
	G
	05
	U
	LE
	AL
	D
	>125
	M
	N
	S/1/A/S
	M/M
	CL2
	CA2
	DAP/200kg/ha

	14
	LLA14
	AW
	120416
	Fasil
	2838
	812913
	495621
	M
	SH
	U
	06(7%)
	I
	D
	AL
	D
	>125
	N
	N
	R/1/A/S
	M/M
	CL2/HE,SR 
	CA2
	DAP/200kg/ha

	15
	LLA15
	AW
	120416
	Fasil
	2822
	813066
	495438
	L
	SH
	U
	06(8%)
	I
	D
	AL
	D
	>125
	N
	N
	R/1/A/S
	M/R
	CL2
	CA2
	DAP/200kg/ha

	16
	LLA16
	AW
	120416
	Fasil
	2833
	813199
	495338
	M
	LP
	G
	05(4%)
	U
	LE
	AL
	D
	>125
	N
	N
	R/1/A/S
	M/M
	CL2/HE
	CA2
	DAP/200kg/ha

	17
	LLA17
	AW
	120416
	Fasil
	2835
	813375
	495049
	M
	LP
	G
	05
	U
	LE
	AL
	D
	>125
	N
	N
	N
	M/M
	CL2/HE
	CA2
	DAP/200kg/ha

	18
	LLA18
	AW
	120416
	Fasil
	2827
	813464
	495087
	M
	LP
	G
	05
	U
	LE
	AL
	D
	>125
	N
	N
	N
	I/S
	CL2
	CA2
	DAP/200kg/ha

	19
	LLA19
	AW
	120416
	Fasil
	2812
	813676
	494906
	L
	CV
	G
	05
	I
	T
	AL
	D
	<75
	N
	N
	S/1/A/S
	I/S
	CL2/HE
	CA2
	DAP/200kg/ha

	20
	LLA20
	AW
	120416
	Fasil
	2809
	813854
	494906
	L
	SH
	U
	06(6%)
	I
	D
	AL
	D
	>125
	N
	N
	R/1/A/S
	M/S
	CL2/HE,SR 
	CA2
	DAP/200kg/ha


	APPENDIX 2:- LEGA LOLA SOIL MORPHOLOGICAL DESCRIPTION 

	ID
	Code
	Depth
(cm)
	Colour
(moist)
	Mottles
(a/co)
	Texture
	Rock
Fragment 
(a/si)
	Consistency 
moist/wet (Stic/Plas)
	Cem/Comp
(g/s/na)
	Nodules
(a/co/h/na/K)
	CaCO3

	1
	LLA1
	0-25
	10YR3/2
	10YR3/2/N
	CL
	N/
	FR/ST/ST
	N//
	N////
	N

	 
	 
	25-65
	10YR3/6
	10YR3/6/N
	C
	N/
	FR/VST/VPL
	N//
	N////
	N

	 
	 
	65-125
	7.5YR3/4
	7.5YR3/4/N
	C
	N/
	FR/VST/VPL
	N//
	N////
	N

	2
	LLA2
	0-30
	7.5YR3/2
	7.5YR3/2/N
	CL
	N/
	FR/ST/ST
	N//
	N////
	N

	 
	 
	30-70
	7.5YR5/6
	7.5YR5/6/N
	C
	N/
	FR/VST/VPL
	N//
	N////
	N

	 
	 
	70-125
	2.5YR3/3
	2.5YR3/3/N
	C
	N/
	FR/VST/VPL
	N//
	F/BL/S/Mn/S
	N

	3
	LLA3
	0-30
	10YR3/4
	10YR3/4/N
	C
	N/
	FR/VST/VPL
	N//
	N////
	N

	 
	 
	30-75
	10YR3/3
	10YR3/3/N
	C
	N/
	FR/VST/VPL
	N//
	N////
	N

	 
	 
	75-125
	10YR3/2
	10YR3/2/N
	C
	N/
	FR/VST/VPL
	N//
	VF/RE/S/Fe/S
	N

	4
	LLA4
	0-25
	10YR3/2
	10YR3/2/N
	C
	N/
	FR/VST/VPL
	N//
	N////
	N

	 
	 
	25-65
	10YR4/4
	10YR4/4/N
	C
	N/
	FR/VST/VPL
	N//
	N////
	N

	 
	 
	65-125
	10YR5/8
	10YR5/8/N
	C
	N/
	FR/VST/VPL
	N//
	N////
	N

	5
	LLA5
	0-30
	10YR3/1
	10YR3/1/N
	C
	N/
	FR/VST/VPL
	N//
	N////
	N

	 
	 
	30-75
	10YR4/1
	10YR4/1/N
	C
	N/
	FR/VST/VPL
	N//
	N////
	N

	 
	 
	75-125
	10YR3/3
	10YR3/3/N
	C
	N/
	FR/VST/VPL
	N//
	N////
	N

	6
	LLA6
	0-30
	10YR3/2
	10YR3/2/N
	C
	N/
	FR/VST/VPL
	N//
	N////
	N

	 
	 
	30-70
	10YR3/4
	10YR3/4/N
	C
	N/
	FR/VST/VPL
	N//
	N////
	N

	 
	 
	70-125
	7.5YR5/8
	7.5YR5/8/N
	C
	N/
	FR/VST/VPL
	N//
	V/RE/S/Fe/S
	N

	7
	LLA7
	0-25
	10YR3/2
	10YR3/2/N
	C
	N/
	FR/VST/VPL
	N//
	N////
	N

	 
	 
	25-75
	7.5YR4/6
	7.5YR4/6/N
	C
	N/
	FR/VST/VPL
	N//
	N////
	N

	 
	 
	75-125
	7.5YR3/3
	7.5YR3/3/N
	C
	N/
	FR/VST/VPL
	N//
	N////
	N

	8
	LLA8
	0-30
	7.5YR3/2
	7.5YR3/2/N
	C
	N/
	FR/VST/VPL
	N//
	N////
	N

	 
	 
	30-70
	7.5YR4/3
	7.5YR4/3/N
	C
	N/
	FR/VST/VPL
	N//
	N////
	N

	 
	 
	70-125
	7.5YR4/6
	7.5YR4/6/N
	C
	N/
	FR/VST/VPL
	N//
	F/BL/S/Mn/S
	 

	9
	LLA9
	0-30
	7.5YR3/3
	7.5YR3/3/N
	CL
	N/
	FR/VST/VPL
	N//
	N////
	N

	 
	 
	30-75
	7.5YR3/4
	7.5YR3/4/N
	C
	N/
	FR/VST/VPL
	N//
	N////
	N

	 
	 
	75-125
	7.5YR4/6
	7.5YR4/6/N
	C
	N/
	FR/VST/VPL
	N//
	N////
	N

	10
	LLA10
	0-30
	7.5YR3/2
	7.5YR3/2/N
	CL
	N/
	FR/VST/VPL
	N//
	N////
	N

	 
	 
	30-85
	7.5YR3/4
	7.5YR3/4/N
	C
	N/
	FR/VST/VPL
	N//
	N////
	N

	 
	 
	85-125
	7.5YR5/8
	7.5YR5/8/N
	C
	N/
	FR/VST/VPL
	N//
	F/RE/S/Fe/S
	N

	11
	LLA11
	0-30
	7.5YR3/2
	7.5YR3/2/N
	CL
	N/
	FR/ST/PL
	N//
	N////
	N

	 
	 
	30-75
	7.5YR4/6
	7.5YR4/6/N
	C
	N/
	FR/VST/VPL
	N//
	F/RE/S/Fe/S
	N

	 
	 
	75-125
	7.5YR5/8
	7.5YR5/8/N
	C
	N/
	FR/VST/VPL
	N//
	C/RE/S/Fe/S
	N

	12
	LLA12
	0-25
	10YR4/4
	10YR4/4/N
	C
	N/
	FR/VST/VPL
	N//
	N////
	N

	 
	 
	25-65
	10YR3/2
	10YR3/2/N
	C
	N/
	FR/VST/VPL
	N//
	N////
	N

	 
	 
	65-125
	10YR5/6
	10YR5/6/N
	C
	N/
	FR/VST/VPL
	N//
	F/RE/S/Fe/S
	N

	13
	LLA13
	0-30
	10YR3/4
	10YR3/4/N
	C
	N/
	FR/VST/VPL
	N//
	N////
	N

	 
	 
	30-70
	10YR4/6
	10YR4/6/N
	C
	N/
	FR/VST/VPL
	N//
	N////
	N

	 
	 
	70-125
	10YR5/8
	10YR5/8/N
	C
	N/
	FR/VST/VPL
	N//
	V/RE/S/Fe/S
	N

	14
	LLA14
	0-25
	10YR3/2
	10YR3/2/N
	CL
	N/
	FR/ST/PL
	N//
	N////
	N

	 
	 
	25-65
	10YR4/4
	10YR4/4/N
	C
	N/
	FR/VST/VPL
	N//
	V/BL/S/Mn/S
	N

	 
	 
	65-125
	10YR4/6
	10YR4/6/N
	C
	N/
	FR/VST/VPL
	N//
	C/F/BL/RE/S/Mn/Fe/S
	N

	15
	LLA15
	0-30
	10YR3/3
	10YR3/3/N
	CL
	N/
	FR/ST/PL
	N//
	N////
	N

	 
	 
	30-75
	10YR5/8
	10YR5/8/N
	C
	N/
	FR/VST/VPL
	N//
	F/C/RE/BL/S/Fe/Mn/S
	N

	 
	 
	75-125
	10YR6/8
	10YR6/8/N
	C
	N/
	FR/VST/VPL
	N//
	C/BL/S/Mn/S
	N

	16
	LLA16
	0-30
	10YR3/4
	10YR3/4/N
	CL
	N/
	FR/ST/PL
	N//
	N////
	N

	 
	 
	30-85
	2.5YR3/4
	2.5YR3/4/N
	C
	N/
	VFR/VST/VPL
	N//
	N////
	N

	 
	 
	85-125
	2.5YR5/6
	2.5YR5/6/N
	C
	N/
	FR/VST/PL
	N//
	F/F/RE/BL/S/Fe/Mn/S
	N

	17
	LLA17
	0-25
	10YR3/2
	10YR3/2/N
	CL
	N/
	FR/ST/PL
	N//
	N////
	N

	 
	 
	25-55
	10YR4/6
	10YR4/6/N
	C
	N/
	FR/VST/VPL
	N//
	N////
	N

	 
	 
	55-125
	10YR5/6
	10YR5/6/N
	C
	N/
	FR/VST/VPL
	N//
	F/RE/S/Fe/S
	N

	18
	LLA18
	0-30
	10YR3/4
	10YR3/4/N
	CL
	N/
	FR/ST/PL
	N//
	N////
	N

	 
	 
	30-85
	10YR5/6
	10YR5/6/N
	C
	N/
	FR/VST/VPL
	N//
	N////
	N

	 
	 
	85-125
	10YR4/6
	10YR4/6/N
	C
	N/
	FR/VST/VPL
	N//
	N////
	N

	19
	LLA19
	0-30
	7.5YR4/4
	7.5YR4/4/N
	CL
	N/
	FR/ST/PL
	N//
	N////
	N

	
	 
	30-70
	7.5YR5/6
	7.5YR5/6/N
	C
	N/
	FR/VST/VPL
	N//
	F/RE/S/Fe/S
	N

	20
	LLA20
	0-25
	7.5YR3/3
	7.5YR3/3/N
	C
	N/
	VFR/VST/VPL
	N//
	N////
	N

	 
	 
	25-75
	7.5YR5/6
	7.5YR5/6/N
	C
	N/
	VFR/VST/VPL
	N//
	N////
	N

	 
	 
	75-125
	7.5YR4/6
	7.5YR4/6/N
	C
	N/
	FR/VST/VPL
	N//
	N////
	N


APPENDIX 3:- SOIL LABORATORY ANALYTICAL RESULT
	PROFILE CODE
	Depth(cm)
	PH-H2O 
(1:2.5)
	PH-KCl
(1:2.5)
	EC(ds/m) 
( 1:2.5)
	Sand ( %)
	Clay (%)
	Silt (%)
	Texture 
Class
	Na
(Cmol(+)/kg)
	K
(Cmol(+)/kg)
	Ca
(Cmol(+)/kg)
	Mg
(Cmol(+)/kg)
	SUM
(Cmol(+)/kg)
	CEC
(Cmol(+)/kg)

	LLP-1
	0-30
	4.2
	3.5
	0.10
	37.00
	21.00
	42.00
	Loam
	0.87
	0.51
	2.56
	0.28
	4.22
	19.10

	LLP-1
	30-90
	4.4
	3.7
	0.03
	31.00
	19.00
	50.00
	Silty Loam
	0.65
	0.32
	3.61
	0.61
	5.19
	13.10

	LLP-1
	90-140
	5.0
	4.1
	0.02
	41.00
	17.00
	42.00
	Loam
	0.65
	0.38
	9.03
	3.34
	13.40
	20.60

	LLP-1
	140-200
	5.5
	4.6
	0.04
	15.00
	17.00
	68.00
	Silty Loam
	0.22
	0.50
	4.14
	1.19
	6.05
	10.20

	 
	Min
	4.4
	3.7
	0.0
	15.0
	17.0
	42.0
	 
	0.2
	0.3
	3.6
	0.6
	5.2
	10.2

	 
	Max
	5.5
	4.6
	0.0
	41.0
	19.0
	68.0
	 
	0.7
	0.5
	9.0
	3.3
	13.4
	20.6

	 
	Average
	5.0
	4.1
	0.0
	29.0
	17.7
	53.3
	Silty Loam
	0.5
	0.4
	5.6
	1.7
	8.2
	14.6


	PROFILE CODE
	Depth(cm)
	BS(%)
	Ex. Acidity
(Cmol(+)/kg)
	Ex.Al
(Cmol(+)/kg)
	T.N(%)
	OC(%)
	OM
	C/N
	Av.P/PPM
	P2O5

	LLP-1
	0-30
	22
	0.48
	0.00
	0.35
	3.51
	6.04
	10
	17.36
	39.8

	LLP-1
	30-90
	 
	 
	 
	 
	 
	 
	 
	 
	 

	LLP-1
	90-140
	40
	0.40
	0.00
	0.05
	0.47
	0.81
	9
	3.22
	7.4

	LLP-1
	140-200
	65
	0.08
	0.00
	0.02
	0.24
	0.41
	12
	2.20
	5

	 
	Min
	59
	0.08
	0.00
	0.01
	0.15
	0.26
	15
	4.20
	9.6

	 
	Max
	39.6
	0.1
	0.0
	0.0
	0.2
	0.3
	9.4
	2.2
	5.0

	
	Average
	65.0
	0.4
	0.0
	0.1
	0.5
	0.8
	15.0
	4.2
	9.6

	
	
	54.7
	0.2
	0.0
	0.0
	0.3
	0.5
	12.1
	3.2
	7.3


	PROFILE CODE
	Depth(cm)
	PH-H2O 
(1:2.5)
	PH-KCl
(1:2.5)
	EC(ds/m) 
( 1:2.5)
	Sand ( %)
	Clay (%)
	Silt (%)
	Texture 
Class
	Na
(Cmol(+)/kg)
	K
(Cmol(+)/kg)
	Ca
(Cmol(+)/kg)
	Mg
(Cmol(+)/kg)
	SUM
(Cmol(+)/kg)
	CEC
(Cmol(+)/kg)

	LLP-2
	0-35
	4.9
	4.4
	0.11
	47.00
	27.00
	26.0
	Sandy clay loam
	0.22
	0.55
	7.23
	1.23
	9.23
	27.10

	LLP-2
	35-95
	4.6
	3.9
	0.04
	33.00
	19.00
	48.0
	Loam
	0.22
	0.38
	1.76
	0.20
	2.56
	18.20

	LLP-2
	95-135
	4.2
	3.6
	0.03
	33.00
	19.00
	48.0
	Loam
	0.22
	0.32
	3.79
	0.99
	5.32
	24.10

	LLP-2
	135-200
	4.6
	4.0
	0.03
	31.00
	19.00
	50.0
	Silty Loam
	0.22
	0.38
	5.28
	1.90
	7.78
	27.40

	
	Min
	4.2
	3.6
	0.0
	31.0
	19.0
	48.0
	 
	0.2
	0.3
	1.8
	0.2
	2.6
	18.2

	
	Max
	4.6
	4.0
	0.0
	33.0
	19.0
	50.0
	 
	0.2
	0.4
	5.3
	1.9
	7.8
	27.4

	
	Average
	4.5
	3.8
	0.0
	32.3
	19.0
	48.7
	Loam
	0.2
	0.4
	3.6
	1.0
	5.2
	23.2


	PROFILE CODE
	Depth (cm)
	BS (%)
	Ex. Acidity
(Cmol(+)/kg)
	Ex.Al
(Cmol(+)/kg)
	T.N (%)
	OC  (%)
	OM
	C/N
	Av.P/PPM
	P2O5

	LLP-2
	0-35
	34
	0.08
	0.00
	0.30
	2.87
	4.94
	10
	0.46
	1.1

	LLP-2
	35-95
	14
	0.48
	0.00
	0.18
	1.58
	2.72
	9
	1.28
	2.9

	LLP-2
	95-135
	22
	0.56
	0.00
	0.05
	0.46
	0.79
	9
	2.10
	4.8

	LLP-2
	135-200
	28
	0.64
	0.00
	0.04
	0.47
	0.81
	12
	1.64
	3.8

	
	Min
	14.1
	0.5
	0.0
	0.0
	0.5
	0.8
	8.8
	1.3
	2.9

	
	Max
	28.4
	0.6
	0.0
	0.2
	1.6
	2.7
	11.8
	2.1
	4.8

	
	Average
	21.5
	0.6
	0.0
	0.1
	0.8
	1.4
	9.9
	1.7
	3.8


APPENDIX 4: SOIL PROFILE DESCRIPTION  
(A) SOIL PROFILE DESCRIPTION


Profile code:
LLP-01

Status: Pit with sample

Major soil Type, FAO 2006: Vertisols
Date:
10/04/16

Author:
Fassil
Coordinate,
Long (N):
812637

Land form:
Plateau
Lat (E):
495908

Regional slope:
Gently sloping
Elevation:
2867

Slope class:
Gently sloping
Map sheet:

Slope form:
Uniform
Depth to bedrock:
>200cm

Position:
High
Rock out crops:
None

Micro topography:
Level
S. coarse fragments:
None

Parent material:
Alluvial deposits
Surface cracks:
None

Effective soil depth:
Very Deep

Drainage class:
Moderately well drained
Sealing:
None

Drainage external:
Slow
Flooding:
None

Water table (cm):
None

Erosion status:
Slight sheet &splash/ 0-5%/ active at present

Land cover:
Intensively cultivated land

Land use:
Fallow system cultivation

Fertilizer:
DAP/200kg/ha

 Crops: Barely, beans, pea, potato

Plant species:
Eucalyptus globulus, Juniperus procera, Hagenia abyssinica, Comba (Or.)
0-30cm: 
Very dark grayish brown (10YR3/2) when moist; clay texture; weak fine massive structure; friable when moist, very sticky and very plastic when wet; common medium roots; common medium pores; non calcareous; clear smooth boundary.

30-90cm:
Red(2.5YR5/6)when moist; very few yellowish brown mottles; clay texture; strong medium and coarse subangular blocky structure; very firm when moist; very sticky and very plastic when wet; compacted ;few soft black manganese & few soft red iron segregation form; few medium roots; few termite or ant channels; few medium pores; non calcareous; clear smooth boundary. 

90-140cm:
Red(2.5YR4/6)when moist; clay texture; strong medium and coarse subangular blocky structure; very firm when moist, very sticky and very plastic when wet; compacted; common black soft manganese in segregation; common termite or ant channels; few fine pores; diffuse wavy boundary.

140-200cm:
Dark red(2.5YR3/6)when moist; clay texture; strong medium and coarse subangular blocky structure; very firm when moist ,very sticky and very plastic when wet; compacted; many black soft manganese in segregation form ; few termite or ant channels; few fine pores.

(B) SOIL PROFILE DESCRIPTION


Profile code:
LLP-02

Status: Pit with sample

Major soil Type, FAO 2006:  Vertisols
Date:
11/04/16

Author:
Fassil
Coordinate,
Long (N):
812406

Land form:
Medium-gradient hill
Lat (E):
495175

Regional slope:
Undulating
Elevation:
2841

Slope class:
Sloping
Map sheet:

Slope form:
Uniform
Depth to bedrock:
>200cm

Position:
Medium
Rock out crops:
None

Micro topography:
Terraced
S. coarse fragments:
None

Parent material:
Alluvial deposits
Surface cracks:
None

Effective soil depth:
Very Deep

Drainage class:
Moderately well drained
Sealing:
None

Drainage external:
Slow
Flooding:
None

Water table (cm):
None

Erosion status:
Slight rill / 0-5%/ active at present

Land cover:
Intensively cultivated land

Land use:
Fallow system cultivation

Fertilizer:
DAP/200kg/ha


Crops:
Barely, beans, pea, potato

Plant species:
Eucalyptus globulus, Hagenia abyssinica, Tulla (Or.)

0-30cm: 
Dark brown (10YR3/3) when moist; clay loam texture; weak fine and medium granular structure; friable when moist, sticky and plastic when wet; few fine and medium roots; few fine and medium pores; non calcareous; clear smooth boundary.

30-95cm:
Dark brown (7.5YR3/4) when moist; clay texture; moderate medium and coarse subangular blocky structure; friable when moist; very sticky and very plastic when wet; few fine roots; few fine pores; non calcareous; diffuse wavy boundary. 

95-135cm:
Strong brown (7.5YR4/6) when moist; clay texture; strong medium and coarse subangular blocky structure; firm when moist, very sticky and very plastic when wet; platy clay compaction; few red soft iron in segregation; few termite or ant channels; non calcareous; diffuse wavy boundary.

135-200cm:
Strong brown(7.5YR5/8)when moist; clay texture; strong medium and coarse subangular blocky structure; firm when moist ,very sticky and very plastic when wet; platy clay compaction; many black soft manganese and few red soft iron in segregation form ;very few termite or ant channels; non calcareous.

i
PLANET Integrated Water Resources Development PLC                                                  E-mail:planetid21@gmail.com/hundedn2004@yahoo.com                                                                                           
P.O. Box 382/1250, Finfinne (Addis Ababa)                                                                                        Telephone: +251-930-01-32-49 (mob.)/0118-95-99-90 (off

_1452175058.wmf

_1540052401.pdf


$+


$+
$+$+


$+
$+$+$+


$+


$+$+


$+
$+


$+


$+


$+


#*


#*


#*


#*


#*
#*#*


#


#
#


#


#


#


#


#


#


#


#


#


#


#


#


#


#


#
#


#


#
#


#


#


#


#


#


#


#


#


#


#


#


#


#


#


#


#


#


#


#


#


#


#


#


#


#


#


#


#


#


#


#


#


#


#


#


#


#


#


#


#


#


#


#


#


#


#


#


Gele


Komiba


Komeba
Aushale


Aushale


Bulichana


494100


494100


494400


494400


494700


494700


495000


495000


495300


495300


495600


495600


495900


495900


496200


496200


496500


496500


496800


496800


497100


497100


497400


497400


497700


497700


81
18


00


81
18


00


81
20


00


81
20


00


81
22


00


81
22


00


81
24


00


81
24


00


81
26


00


81
26


00


81
28


00


81
28


00


81
30


00


81
30


00


81
32


00


81
32


00


81
34


00


81
34


00


81
36


00


81
36


00


81
38


00


81
38


00


81
40


00


81
40


00


81
42


00


81
42


00


Legend
# Settlement
#* House
$+ Tukul


Main canal
River
Trail
Boundary


®
200 0 200100


Meters


Location Map of Lega lalo Project


Oromiya Somalia


AfarAmhara


SNNP


Tigray


Gambela


Beni Shangul Gumuz
Dire Dawa


HarariAddis Ababa


45°0'0"E


45°0'0"E


40°0'0"E


40°0'0"E


35°0'0"E


35°0'0"E


10
°0


'0"
N


10
°0


'0"
N


5°0
'0"


N


5°0
'0"


N


Lege Lola Project






