THE NATIONAL REGIONAL STATE OF

OROMIA

OROMIA IRRIGATION DEVELOPMENT AUTHORITY

(OIDA)

FEASIBILITY STUDY AND DETAIL DESIGN

OF

LOT 5 OF 19 SSIPs [WEST SHOWA]

GAWISO SMALL SCALE IRRIGATION PROJECT

OPERATION AND MAINTENANCE MANUAL

P.O.BOX: 40945

Tel: 0920724075/0911963757

E-mail: letinsasc@gmail.com

Addis Ababa, Ethiopia

November 2016

Addis Ababa, Ethiopia

TABLE OF CONTENTS

1.	INTRODUCTION	1

1.1	OVERALL	1

1.2	Project Location And Accessibility	1

1.3	Major Design Findings	3

1.4	GENERAL DESCRIPTION OF THE PROJECT AREA	1

2.	ORGANIZATIONAL SET UP	1

2.1	Overall	1

2.2	Catchment Team	1

2.3	Head Works Team	1

2.4	Infrastructure Team	1

2.5	Development Team	1

2.6	Proposed Water Users Association Structures	2

3.	IRRIGATION SCHEDULING AND WATER MANAGEMENT	4

3.1	Overall	4

3.2	Proposed Cropping Pattern For The Project	4

3.3	Irrigation Efficiencies	10

3.4	Infrastructure Management	10

3.5	Water Supply System	10

3.6	Water Requirement	11

3.7	Irrigation Scheduling And Operation Plan	12

3.7.1	Irrigation Scheduling	12

3.7.2	Depth of Application	13

3.7.3	Irrigation Intervals(II) of the project	13

4.	FLOW MEASUREMENT DEVICE AND PROCEDURES	15

5.	OPERATION PROCEDURES AND METHODS	16

5.1	Understanding the Recommended Application Methods	16

5.2	Surface Irrigation Methods	16

5.2.1	Basin Irrigation Method	17

5.2.2	Furrow Irrigation	17

6.	METHODS AND PROCEDURES FOR MONITORING THE SYSTEM	19

6.1	Overall	19

6.2	Levels Of Operation	19

6.3	Data Requirement	19

6.3.1	Climate	19

6.3.2	Water	19

6.3.3	Crops	20

6.3.4	Beneficiaries’ response	20

6.3.5	Environmental assessment	20

6.4	Reports And Returns Need	20

6.4.1	Agricultural performance	20

6.4.2	Annual performance	20

6.4.3	Environment impact study	21

6.5	Methods &Procedures For Regular & Periodical Maintenance	21

6.5.1	Overall	21

6.5.2	Categories	21

6.5.3	Sources of Materials for Repair	24

6.5.4	BeneficiariesNeed for Maintenance	24

6.5.5	Communication System	24

7.	TRAINING AND CAPACITY BUILDING	25

7.1	TRAINING OF BENEFICIARIES	25

7.2	ON-FARM TRAINING	25

7.3	TRAINING OF TRAINERS	25

7.3.1	Basic	26

7.3.2	Technical	26

8.	CHARGES FOR OPERATION AND MAINTENANCE	27

References	28

APPENDICES	29

List of AppendiCES

APPENDIX I: Typical Format for Flow Measurement at Intake	30

APPENDIX II: Head - discharge Equation for Orifice	31

APPENDIX III: Head Discharge Curve for Portable v-Notch (900)	32

APPENDIX IV: Format for Record of Rainfall	33

APPENDIX V: Format for Irrigated crops & their coverage	34

APPENDIX VI: Format for the Assessment of Conveyance Loss	35

APPENDIX VII: Format for Recording Weekly Water Distribution and Irrigated Area	36

APPENDIX VIII: Annual Assessment of Crop Yields	37

APPENDIX IX: Format for reporting defects and actions taken (Biannual)	38

APPENDIX X: Typical Irrigation Water use and service Agreement	39

List of tables

Table 1-1:  Location of Project Areas	1

Table 3-1-Crop selected to be grown during wet season	4

Table 3-2- Crop Selected to be grown with dry season/ by irrigation	5

Table 3-3--Major cultivation activities in irrigated agriculture	5

Table 3-4– Inputs Required for GewesoSSIP for one year	5

Table 3-6:-Methods of fertilizer application for the proposed crop types	6

Table 3-7-A hectare of labor requirement as a “With-Project” condition	6

Table 3-8-Projected yields of proposed crops for GewesoSSIP	7

Table 3-9:- Detail Crop Water Requirement (Dry-Season Cultivation)	8

Table 3-10:- - Required climatic & other aerodynamic data collected from Ambo Station	9

Table 3-11:- Irrigation depth (d, mm) and Interval (days)	9

Table 3-12: Project Efficiencies	10

Table 4-1: Location of Water Measuring Devices and Frequency of Discharge Observation	15

Table 6-1: Normal Repair and Maintenance Chart upon Inspection	23

list of figures

Figure 1-1: Location Map of GewesoSSIP Site	2

Figure 1-1: General Arrangement ofGewesoSSIP Supply System	1

Figure 1-3: Irrigation and Drainage System Layout	1

Figure 2-1:  Proposed Water Users Association Structure for GewesoSSIP	3

Figure 4-1: Typical Partial Flume	15

# INTRODUCTION

## OVERALL

This manual is presented here to give requirements, methods and procedures for management, operation and maintenance of GewesoSmall Scale IrrigationProjectby defining the methods and principles so as to ensure equitable distribution of water with least human interference. Theproject is financed by the AGP.

## Project Location And Accessibility

The administrative location and accessibility with respect to kebele, woreda, zone and region of the proposed GewusoSSI project is presented in the following table:

Table 1-1:  Location of Project Areas

- Administrative and geographical

| Scheme | Administrative Location |  |  | Geographical location (meters) |  |
| --- | --- | --- | --- | --- | --- |
|  | Zone | Woreda | Kebele/”got” | Easting | Northing |
| Geweso |  | TokoKutaye | MugnoBaboGalo | 336684 | 978531 |

- Accessibility

| Scheme | Region capital | Zone Town | Woreda |
| --- | --- | --- | --- |
| Geweso | 284 | 74 | 32 |

Like GewesoProject, Geweso project is accessed using the new gravel road under construction gravel road from Babitch Town. This road passes near the Gura-2 headwork and crosses the command area. However, for Geweso project, additional 7Km have to be constructed start from end of Gewesocommand area

Figure 1-1: Location Map of GewesoSSIP Site

## Major Design Findings

The proposed irrigation project is intended to irrigate a net command area of58ha irrigate by gravity.All irrigable area is to be supplied by river diversion using weir diversion structure planned to be constructed on Geweso River.The scheme is designed to be irrigated by diverting about a total of 98 l/sduring dry time.

The study carried out envisages ways of strengthening the scheme in areas of major constraints to make it more productive, efficient, and sustainable. Poor water management is one of such constraints. The optimum use of irrigation water to grow maximum production per unit of water is an essential requirement of good soil and water resource management. In its absence, the conveyance and field application efficiency becomes low resulting in low yields, excessive deep percolation losses and rise in water table, salinity, etc.

Figure 1-2: General Arrangement ofGewesoSSIP Supply System

The water management in this scheme will be an integral process of diversion (intake), conveyance, regulation, measurement, distribution, application and removal of excess water. Apart from field management and implementing operational plans, monitoring and evaluation is equally important to consolidate, make and maintain the system functional.

The overall routine management, operation and maintenance is to be looked after by the “Water Users’ Association” (WUA). It is quite imperative that different groups amongst the WUA are formed who will be associated with the different tasks so as to have in depth knowledge of the functions and the utility of the scheme. Without this awareness and know-how proper management, operation and maintenance will not be sustainable and successful.  It is also needed that they will be given training within the project, by arranging workshops and other facilities for field visits of other developed agricultural farms.

Operation and maintenance of the headwork and its appurtenant works, infrastructures, on farm development and drainage system need knowledge, systematic approach, constant watch and devotion to utilize the services successfully and sustain the scheme for a longer period of time.

## GENERAL DESCRIPTION OF THE PROJECT AREA

Based on this general description of the rainfall regimes of the country, GEWESOSSI Project area is grouped in the rainfall regime of B has a mono-modal rainfall pattern (June – September) locally referred as Kiremt.Based on this traditional climatic classification, the project catchment’s area of GEWESO is categorized in to the Weyina Dega Temperate,Sub-tropical class.

Around 95% of the rain fall occurs in between Mar to Sept. The highest mean monthly rainfall occurs during month of Jul, which is 273mm. The mean monthly and annual rain fall of the area is 113mm and 1361mm respectively. The mean monthly maximum, mean and minimum temperatures are 27.6oC, 17.90oC and 8.20oC respectively. The highest mean monthly maximum, mean and minimum temperatures are 29.08oC, 19.436oC and 9.3oC, which are recorded during Mar, Jul&May respectively. While the minimum mean monthly maximum, mean and minimum temperatures are recorded in Aug (26.2oC), Dec (4.55oC) &Nov (16.04oC) respectively.

The watershed has has26.91 sq. km km2 area. The longest flow path has a length of 11.637Km, originated from an elevation of 2659.996masl, while the lowest is 2111.812masl at selected headwork 37.4% of the soil in watershed is dominated by Chromic Luvisols soil type, and 62.6% is dominated by OrthicLuvisols.

Figure 1-3: Irrigation and Drainage System Layout

For detailed readable infrastructure layout, refer drawing album

# ORGANIZATIONAL SET UP

## Overall

The WUAto be newly formed by the beneficiaries have to be involved with the construction and operation of all the components so that they remain acquainted with the know-how of the functions of eachorganization i.e. their purpose, importance and nature for their proper up-keep and safe functioning during maintenance stage.

The beneficiaries are also expected to elect their representative for various tasks who will be in charge for about five years. The various tasks given below will be carried out by the different groups of beneficiaries while the O&M in chief will play a coordinating & supervisory role. The following groups of beneficiaries, semi-skilled and educated have to be acquainted with different components from the beginning which will ultimately perform the tasks of basic management, operation and maintenance of the scheme.

## Catchment Team

A group of 4beneficiaries will be assigned to monitor the watershed, propose measures and report to the executive body.

## Head Works Team

A group of 3 beneficiaries will make joint visit at a time and look after the regulation of diversion/intake structures, various services rendered, outlets, normal maintenance of on-farm structures and maintenance of records.

## Infrastructure Team

A group of 3 beneficiaries will be responsible for the regulation, upkeep, maintenance of records, checking functions of structures, their stability, and repair/ maintenance of the outlets for the irrigation blocks.  They will also look after the field canals falling within this range of the conveyance system.

Each block will have Block-leader selected from beneficiaries of that block. These will form an overall W.U.A executive managing/committee i.e. one O&M in chief and two assistants.

The Block leader/tertiary area leader/ will be responsible for distribution of water within their area and settle any internal disputes within themselves. They will also associate in the land leveling and land terracing and maintaining the tertiary area boundaries.

The block leaders will look after the maintenance of the drainage system, their upkeep and will be watchful against erosion, water logging and salinity.

## Development Team

This group will consist of five beneficiaries to look after the following arrangements.

- Fertilizers and improved seeds,

- Pasture lands and livestock,

- Pest control, insecticides and pesticides,

- Health, hygiene and sanitation,

- Other allied matters relating to agriculture

- Consolidation of holdings

- Extension programs and training

- Storage and marketing facilities

- Establish communication with associated disciplines and

- Facilitating credit

Such arrangement will provide a multi-disciplinary approach for all round development and upkeep of the watershed, irrigation, soil and water management, agricultural inputs, livestock development, forestry, fisheries, health and sanitation and above all a communication link with the technical experts at Regional/Zonal administration.

It is important to mention that the multi-disciplinary team formed at the earliest stage has to monitor the performance of the organizational set up and performance of the WUA for a continuous period of at least five years, till the performance gives optimum results.

## Proposed Water Users Association Structures

ICSAA

Physical Planning Team

Planning = Male

Financial Auditing Team Auditor= Female

Scheme Administration Team

- Water Distributer- Female

- Maintenance Head= Male

- Water Distributer- Female

Financial Administration Team

- Cashier- Head = Female

- Bill Collector= Female

- Bill Preparation= Male

Wereda OIDA Office

General Assembly

Conflict Resolution Team Conflict

- Resolution Head= Male=1

- Conflict Resolution Members=  Female=2

Irrigation Water Users Association (IWUA) Management Committee Members

- Chairperson - Scheme Administration Team Head-  Male

- Secretary= Financial Administration team= Head- Female

- Conflict Resolution Team = head-Male

- Member- Physical Planning Team = Head- Male

- Member- Financial Auditor= Head- Female

Other Water Users= 3 for each sides of the command area (If any)

Figure 2-1:  Proposed Water Users Association Structure for GewesoSSIP

# IRRIGATION SCHEDULING AND WATER MANAGEMENT

## Overall

Irrigated agriculture is a combination of Engineering, Agronomic, Social and Economic endeavor involving the beneficiaries with their due importance in the formulation, implementation, operation and maintenance along with their overall association in the management and monitoring of the land and water use.

The system’s attempts to understand the physical, agronomic and social aspects of operation, and subsequently developing an inter-disciplinary team consisting of engineers, agronomists, sociologists and the committee members of the WUA.  The beneficiaries (WUA) after gaining experience of the O&M system will gradually take-over, but will always remain within the zonal administrative “Monitoring and Evaluating Cell”, which will educate the WUA from time to time, with the skills required to monitor, evaluate the input and output for achieving optimum efficiency and output.

To meet this end, and achieve optimum land and water use, it was very essential to prepare a water distribution network, best suited to topography and land classification. This network has to cater not only to the physical or quantitative parameters but also to take into account the operational system which will involve equitable distribution of water to all fields within a fixed time to meet the crop water requirements with least human intervention. This map drawn to a convenient scale will be provided to the WUA; who are accountable for O&M. On this layout map, all the specific features will be indicated and planned if need be.

## Proposed Cropping Pattern For The Project

In the proposed project area one main and othersupplementary irrigation seasons are considered, which are recommendedas full - time and supplementalirrigation which are supposed to start as soon asrainfed crops are harvested. The corresponding proposed cropping pattern of Gewesoirrigation project is given in Table 3-1 below.

Table 3-1-Crop selected to be grown during wet season

| S/N | Proposed Crops | Area |  | Selected varieties | Planting/ Sowing date | Harvesting Date | Base- period (days) |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (ha) | (%) |  |  |  |  |
| 1 | Onion | 11.6 | 20 | Adama red, Bombe red | Feb.1-10 | May20-30 | 110 |
| 2 | Potato | 17.4 | 30 | Jalane, Gudane, Belete | Feb. 1-10 | May20-30 | 120 |
| 3 | Beetroot | 8.7 | 15 | --- | Feb.5-15 | May20-30 | 105 |
| 4 | Pepper | 11.6 | 20 | Marekofana,Bako Local | Feb. 10-20 | May15-25 | 95 |
| 5 | Carrot | 8.7 | 15 | Scharwoode | Feb. 1-10 | May 20-30 | 110 |

Source: Feasibility Level Agronomy Study Report of same project (For more details refer the same project sector report)

Table 3-2- Crop Selected to be grown with dry season/ by irrigation

| S/N | Proposed Crops | Area |  | Selected varieties | Planting/ Sowing date | Harvesting Date | Base- period (days) |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (ha) | (%) |  |  |  |  |
| 1 | Maize | 14.5 | 25 | BH-540, Shone, Shala | Oct. 1-15 | Jan. 20-30 | 120 |
| 2 | Potato | 17.4 | 30 | Jalane, Gudane, Belete | Oct. 10-20 | Jan. 20-25 | 95 |
| 3 | Cabbage | 8.7 | 15 | Duram Head | Oct. 5-15 | Jan. 25-30 | 120 |
| 4 | Tomato | 14.5 | 25 | Marglobe, Melkashalo red | Oct. 1-10 | Jan. 20-30 | 110 |
| 5 | Sugarcane | 2.9 | 5 | Thick var. | Oct. 1-10 | Oct. 5-15 | 360+ |

Source: Feasibility Level Agronomy Study Report of same project (For more details refer the same project sector report)

Table 3-3--Major cultivation activities in irrigated agriculture

| S/N | Proposed crops | Seed Rate (Kg/ha) | Soil depth during Seeding/ planting (cm) | Cultivation time/Weeding |
| --- | --- | --- | --- | --- |
| 1 | Maize | 25 | 5 | After 4-leaf stage |
| 2 | Potato | 2000 | 5-10 | 1st 25 days and 2nd 45 days after planting |
| 3 | Cabbage | 0.5-0.7 | 4-5 (Transplanting from nursery site after 30 days) | After 20 days of transplanting |
| 4 | Tomato | 0.5 | 4-6 (Transplanting from nursery site after 30 days) | After 20 days of planting |
| 5 | Sugarcane | 44,444 cuttings | 5-10 | 40 - 45 days after planting |
| 6 | Onion | 3-4 | 4-5 (Transplanting from nursery site after 30 days) | After 20 days of transplanting |
| 7 | Beetroot | 0.5-1.0 | Same | Same |
| 8 | Pepper | 0.5 to 0.6 | Same | Same |
| 9 | Carrot | 6-10 | 3-5 and drilling | 20 to 25 days after sowing |

Source: Feasibility Level Agronomy Study Report of same project (For more details refer the same project sector report)

Table 3-4– Inputs Required for GewesoSSIP for one year

| Crop | Dry Season |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Area, ha | DAP, qt | Urea, qt | Seeds, qt | Fungicide, lt | Insecticide, kg | Herbicide, lt |
| Maize | 14.5 | 14.5 | 21.75 | 3.625 | 43.5 | 43.5 | 58.0 |
| Potato | 17.4 | 26.1 | 17.4 | 348 | 52.2 | 52.2 | --- |
| Cabbage | 8.7 | 8.7 | 4.35 | 0.87 | --- | 26.1 | --- |
| Tomato | 14.5 | 21.75 | 14.5 | 0.0725 | 43.5 | 43.5 | --- |
| Sugarcane* | 2.9 | 2.9 | 1.45 | 44,444 cuttings | ---- | --- | --- |
| Total | 58 | 73.95 | 59.45 | 352.5675 | 139.2 | 165.3 | 58.0 |

| Crop | Wet Season |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Area, ha | DAP, qt | Urea, qt | Seeds, qt | Fungicide, lt | Insecticide, kg | Herbicide, lt |
| Onion | 11.6 | 11.6 | 11.6 | 0.464 | 34.8 | 34.8 | --- |
| Potato | 17.4 | 26.1 | 17.4 | 348 | 52.2 | 52.2 | --- |
| Beetroot | 8.7 | 8.7 | 4.35 | 0.087 | --- | --- | --- |
| Pepper | 11.6 | 11.6 | 11.6 | 0.0696 | 34.8 | 34.8 | --- |
| Carrot | 8.7 | 8.7 | 4.35 | 0.87 | --- | --- | --- |
| Total | 58 | 66.7 | 49.3 | 349.4906 | 121.8 | 121.8 | --- |

| Crop | Dry Season + Wet Season |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | DAP, qt | Urea, qt | Seeds, qt | Fungicide, lt | Insecticide, kg | Herbicide, lt |
| Maize | 14.5 | 21.75 | 3.625 | 43.5 | 43.5 | 58.0 |
| Potato | 34.8 | 52.2 | 696 | 104.4 | 104.4 | --- |
| Cabbage | 8.7 | 4.35 | 0.87 | --- | 26.1 | --- |
| Tomato | 21.75 | 14.5 | 0.0725 | 43.5 | 43.5 | --- |
| Onion | 11.6 | 11.6 | 0.464 | 34.8 | 34.8 | --- |
| Beetroot | 8.7 | 4.35 | 0.087 | --- | --- | --- |
| Pepper | 11.6 | 11.6 | 0.0696 | 34.8 | 34.8 |  |
| Carrot | 8.7 | 4.35 | 0.87 | --- | --- |  |
| Sugarcane* | 2.9 | 1.45 | 44,444 cuttings | ---- | --- |  |
| Total | 140.65 | 108.75 | 702.0581 | 261.0 | 287.1 | 58.0 |

Table 3-5:-Methods of fertilizer application for the proposed crop types

| S/N | Proposed Crops | Fertilizers to be applied | Method of Application | Time of Application |
| --- | --- | --- | --- | --- |
| 1 | Maize | DAP | Top/Side dressing | At the time of sowing |
|  |  | Urea | Basal dressing | At 4-leaf stage |
| 2 | Potato | DAP | Top/Side dressing | At the time of sowing |
|  |  | Urea | Basal dressing | During earthening up stage |
| 3 | Pepper | DAP | Top/Side dressing | At the time of transplanting |
|  |  | Urea | Basal dressing | 3 to 4 weeks after transplanting |
| 4 | Tomato | DAP | Top/Side dressing | At the time of transplanting |
|  |  | Urea | Basal dressing | 3 to 4 weeks after transplanting |
| 5 | Sugarcane | DAP | Top/Side dressing | At the time of planting |
|  |  | Urea | Top/Side dressing | 6 weeks after planting |
| 6 | Onion | DAP | Top/Side dressing | At the time of transplanting |
|  |  | Urea | Basal dressing | 3 to 4 weeks after transplanting |
| 7 | Beetroot | DAP | Top/Side dressing | Same |
|  |  | Urea | Basal dressing | Same |
| 8 | Cabbage | DAP | Top/Side dressing | At the time of transplanting |
|  |  | Urea | Top/Side dressing | 3 to 4 weeks after transplanting |
| 9 | Carrot | DAP | Top/Side dressing | At the time of sowing |
|  |  | Urea | Basal dressing | 3 to 4 weeks after sowing |

Source: Feasibility Level Agronomy Study Report of same project (For more details refer the same project sector report)

Table 3-6-A hectare of labor requirement as a “With-Project” condition

| S/N | Crops | Activities | Unit | Frequency | Qty | Total | Remark |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Maize | Ploughing | OD | 4 | 4 | 16 | Irrigation requirement will be every 17 days for 24 hours. |
|  |  | Pre-Irrigation | MD | 1 | 4 | 4 |  |
|  |  | Planting/Sowing | MD | 1 | 4 | 4 |  |
|  |  | Irrigation | MD | 15 | 4 | 60 |  |
|  |  | Cultivation/Weeding | MD | 2 | 20 | 40 |  |
|  |  | Fertilizer application | MD | 2 | 4 | 8 |  |
|  |  | Protection | MD | 2 | 4 | 8 |  |
|  |  | Harvesting | MD | 1 | 15 | 15 |  |
|  |  | Threshing | MD | 1 | 35 | 35 |  |
|  |  | Transport | MD | 1 | 8 | 8 |  |
| 2 | Potato | Ploughing | OD | 4 | 4 | 16 | Irrigation requirement will be every 7 days for 24 hours. |
|  |  | Pre-Irrigation | MD | 1 | 4 | 4 |  |
|  |  | Planting/Seeding | MD | 1 | 20 | 20 |  |
|  |  | Irrigation | MD | 15 | 4 | 60 |  |
|  |  | Cultivation/Weeding | MD | 2 | 20 | 40 |  |
|  |  | Fertilizer application | MD | 2 | 4 | 8 |  |
|  |  | Protection | MD | 2 | 4 | 8 |  |
|  |  | Harvesting | MD | 1 | 30 | 30 |  |
|  |  | Transport | MD | 1 | 20 | 20 |  |
| 3 | Cabbage | Ploughing | OD | 4 | 4 | 16 | Irrigation requirement will be every 10 days for 24 hours. |
|  |  | Pre-Irrigation | MD | 1 | 4 | 4 |  |
|  |  | Seedling nursery | MD | 1 | 5 | 5 |  |
|  |  | Transplanting | MD | 1 | 30 | 30 |  |
|  |  | Irrigation | MD | 10 | 4 | 40 |  |
|  |  | Cultivation/Weeding | MD | 2 | 20 | 40 |  |
|  |  | Fertilizer application | MD | 2 | 4 | 8 |  |
|  |  | Protection | MD | 3 | 4 | 12 |  |
|  |  | Harvesting | MD | 1 | 30 | 30 |  |
|  |  | Transport | MD | 1 | 20 | 20 |  |
| 4 | Tomato | Ploughing | OD | 4 | 4 | 16 | Irrigation requirement will be every 8 days for 24 hours. |
|  |  | Pre-Irrigation | MD | 1 | 4 | 4 |  |
|  |  | Seedling nursery | MD | 1 | 5 | 5 |  |
|  |  | Transplanting | MD | 1 | 30 | 30 |  |
|  |  | Irrigation | MD | 15 | 4 | 60 |  |
|  |  | Cultivation/Weeding | MD | 2 | 20 | 40 |  |
|  |  | Fertilizer application | MD | 2 | 4 | 8 |  |
|  |  | Staking | MD | 1 | 15 | 15 |  |
|  |  | Protection | MD | 3 | 4 | 12 |  |
|  |  | Harvesting | MD | 1 | 30 | 30 |  |
|  |  | Transport | MD | 1 | 20 | 20 |  |
| 5 | Sugarcane | Ploughing | OD | 4 | 4 | 16 | This is only for a one –harvest plan. There are many ratoonharvesting for sugarcane plant in its production life. |
|  |  | Pre-Irrigation | MD | 1 | 4 | 4 |  |
|  |  | Cutting preparation | MD | 1 | 20 | 20 |  |
|  |  | Planting | MD | 1 | 30 | 30 |  |
|  |  | Irrigation | MD | 50 | 4 | 200 |  |
|  |  | Cultivation/Weeding | MD | 2 | 20 | 40 |  |
|  |  | Fertilizer application | MD | 2 | 4 | 8 |  |
|  |  | Protection | MD | 2 | 4 | 8 |  |
|  |  | Harvesting | MD | 1 | 40 | 40 |  |
|  |  | Transport | MD | 1 | 50 | 50 |  |

Source: Feasibility Level Agronomy Study Report of same project (For more details refer the same project sector report)

Table 3-7-Projected yields of proposed crops for GewesoSSIP

| S/N | Crop | 1st-Year /season |  | 2nd-year /Season |  | 3rdYear /Season |  | 4th-year /Season |  | 5th-year /Season |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Yield (qt/ha | Prodn (Qt) | Yield (qt/ha | Prodn (Qt) | Yield (qt/ha | Prodn (Qt) | Yield (qt/ha | Prodn (Qt) | Yield (qt/ha | Prodn (Qt) |
| 1 | Maize | 60 | 870 | 65 | 942 | 68 | 986 | 70 | 1015 | 70 | 1015 |
| 2 | Potato | 120 | 2088 | 150 | 2610 | 200 | 3480 | 220 | 3828 | 250 | 4350 |
| 3 | Cabbage | 80 | 696 | 95 | 826 | 120 | 1044 | 150 | 1305 | 150 | 1305 |
| 4 | Tomato | 150 | 2175 | 180 | 2610 | 200 | 2900 | 220 | 3190 | 220 | 3190 |
| 5 | Sugarcane* | 150 | 435 | 165 | 478 | 175 | 507 | 180 | 522 | 180 | 522 |
| 6 | Onion | 80 | 928 | 90 | 1044 | 100 | 1160 | 120 | 1392 | 120 | 1392 |
| 7 | Beetroot | 180 | 1566 | 200 | 1740 | 220 | 1914 | 240 | 2088 | 260 | 2262 |
| 8 | Pepper | 20 | 232 | 25 | 290 | 30 | 348 | 30 | 348 | 30 | 348 |
| 9 | Carrot | 150 | 1305 | 165 | 1435 | 180 | 1566 | 200 | 1740 | 220 | 1914 |

Note: *indicates tone/ha for sugarcane

Source: Feasibility Level Agronomy Study Report of same project (For more details refer the same project sector report)

Table 3-8:- Detail Crop Water Requirement (Dry-Season Cultivation)

| S/n | Months | Mean min. monthly temp.(oC) | Mean max. monthly temp.(oC) | Relative humidity (Kpa). | Wind speed (Km/Day) | Sunshine (hrs/day) | Radiation (J/m2/day) | ETO (mm/day) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Jan. | 11.8 | 27.5 | 49 | 18 | 9.9 | 22 | 3.59 |
| 2 | Feb. | 11.8 | 28.8 | 47 | 20 | 9.9 | 23.4 | 3.98 |
| 4 | Mar. | 13.4 | 28.8 | 47 | 17 | 7.8 | 21.3 | 3.87 |
| 5 | Apr. | 13.6 | 28.4 | 56 | 16 | 6.7 | 19.8 | 3.77 |
| 6 | May | 12.7 | 27.6 | 62 | 13 | 7.4 | 20.4 | 3.82 |
| 7 | June | 12.6 | 24.9 | 71 | 9 | 4.7 | 16 | 3.06 |
| 8 | July | 12.6 | 22.7 | 79 | 7 | 3.4 | 14.2 | 2.71 |
| 9 | August | 12.6 | 22.6 | 81 | 6 | 3.1 | 14 | 2.67 |
| 10 | Sep. | 11.8 | 24.1 | 74 | 8 | 4.9 | 16.8 | 3.07 |
| 11 | Oct. | 11.2 | 25.9 | 58 | 13 | 7.4 | 19.9 | 3.44 |
| 11 | Nov. | 10.9 | 26.3 | 58 | 16 | 9.2 | 21.3 | 3.52 |
| 12 | Dec. | 11.2 | 26.6 | 52 | 18 | 9.1 | 20.4 | 3.3 |
|  | Av. | 12.2 | 26.2 | 61 | 13 | 7 | 19.1 | 3.4 |

Table 2:  Rainfall of Ambo Area (same source)

| S/n | Moths | Potential rainfall (mm/Month) | Effective Rainfall (mm/Month) | Remark |
| --- | --- | --- | --- | --- |
| 1 | Jan. | 33.8 | 10.3 | Effective Rain fall is calculated based on dependableformula |
| 2 | Feb. | 24.3 | 4.6 |  |
| 3 | Mar. | 52.1 | 21.3 |  |
| 4 | Apr. | 64.2 | 28.5 |  |
| 5 | May | 105.9 | 60.7 |  |
| 6 | June | 165.3 | 108.2 |  |
| 7 | July | 232.8 | 162.2 |  |
| 8 | August | 212.2 | 145.8 |  |
| 9 | Sep. | 98.1 | 54.5 |  |
| 10 | Oct. | 44.2 | 16.5 |  |
| 11 | Nov. | 18.2 | 0.9 |  |
| 12 | Dec. | 13.5 | 0 |  |
|  | Total | 1064.6 | 613.5 |  |

Source: Feasibility Level Agronomy Study Report of same project (For more details refer the same project sector report)

Table 3-9:- - Required climatic & other aerodynamic data collected from Ambo Station

| S/n | Climate Fac. | Unit | Moths |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | J | F | M | A | M | J | J | A | S | O | N | D |
| 1 | Max. Mean tep. | 0C | 27.5 | 28.8 | 28.8 | 28.4 | 27.6 | 24.9 | 22.7 | 22.6 | 24.1 | 25.9 | 26.3 | 26.6 |
| 2 | Min .mean tem. | 0C | 11.8 | 11.8 | 13.4 | 13.6 | 12.7 | 12.6 | 12.6 | 12.6 | 11.8 | 11.2 | 10.9 | 11.2 |
| 3 | Re.humidity | % | 49 | 47 | 47 | 56 | 62 | 71 | 79 | 81 | 74 | 58 | 58 | 52 |
| 4 | W.speed | Km/day | 18 | 20 | 17 | 16 | 13 | 9 | 7 | 6 | 8 | 13 | 16 | 18 |
| 5 | Sunshine | Hrs/day | 9.9 | 9.9 | 7.8 | 6.7 | 7.4 | 4.7 | 3.4 | 3.1 | 4.9 | 7.4 | 9.2 | 9.1 |
| 6 | Radiation | MJ/m2 | 22.0 | 23.4 | 21.3 | 19.8 | 20.4 | 16.0 | 14.2 | 14.0 | 16.8 | 19.9 | 21.3 | 20.4 |
| 7 | P.RF | mm/M | 33.8 | 24.3 | 52.1 | 64.2 | 105.9 | 165.3 | 232.8 | 212.2 | 98.1 | 44.2 | 18.2 | 13.5 |
| 8 | Eff.RF | mm/M | 10.3 | 4.6 | 21.3 | 28.5 | 60.7 | 108.2 | 162.2 | 145.8 | 54.5 | 16.5 | 0.9 | 0.0 |
| 9 | Eto | mm/day | 3.59 | 3.98 | 3.87 | 3.77 | 3.82 | 3.06 | 2.71 | 2.67 | 3.07 | 3.44 | 3.52 | 3.30 |
| 10 | Eto | mm/M | 107.7 | 119.4 | 116.1 | 113.1 | 114.6 | 108.0 | 81.3 | 80.1 | 92.1 | 103.2 | 105.6 | 99.0 |

Table 3-10:- Irrigation depth (d, mm) and Interval (days)

| S/n | Crops | p-value | Sa (mm/M) | p.Sa | D (mts) | Ea | d (mm) | Remark |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Maize | 0.56 | 200 | 112 | 0.60 | 0.6 | 112 |  |
| 2 | Potato | 0.35 | 200 | 70 | 0.45 | 0.6 | 52.5 |  |
| 3 | Cabbage | 0.45 | 200 | 90 | 0.37 | 0.6 | 55.5 |  |
| 4 | Tomato | 0.42 | 200 | 84 | 0.42 | 0.6 | 58.8 |  |
| 5 | Sugarcane | 0.65 | 200 | 130 | 0.85 | 0.6 | 184.2 |  |

Source: Feasibility Level Agronomy Study Report of same project (For more details refer the same project sector report)

## Irrigation Efficiencies

Based on the type of irrigation, method of application and beneficiaries’ capacity on the given soil type, the following efficiencies are adopted for the project.

| S/n | Type of Efficiencies |  | Suggested Values, in (%) |
| --- | --- | --- | --- |
| 1 | Conveyance Eff. | Ec | 98 % |
| 2 | Distribution Eff. | Ed | 86 % |
| 3 | Application Eff. | Ea | 60 % |
| 4 | Project Eff. | Ep | 50 % |

Table 3-11: Project Efficiencies

The amount of water applied at each irrigation turn shall depend on the water consumed by crops between two irrigation turns. The irrigation water consumed by these crops is the crop water need minus the effective rainfall of that particular site. It is often expressed in mm of water or m3/ha (Note: 1mm=10m3/ha). For this project, the conversion from net irrigation water need to gross is made using a project efficiency of 50%.

Beneficiaries having farms served by the same on-farm delivery tertiary canal will have to organize their own rotation schedule i.e. amount of water applied at each irrigation turn measured by time of water application and frequency of irrigation turns.

## Infrastructure Management

Infrastructure here includes the diversion headwork, the main conveyance system, on-farm structures, distribution canals, outlets and related drainage system. The management aspect mainly considers, but not limited to:

- Procedures and methods for checking and evaluating the desired functions of the above hydraulic structures including farm outlets (turn outs) during operation and

- The methods and procedures for regular and periodic maintenance of the structures and outlet works are to be considered.

## Water Supply System

The planned water supply system is continuous in main canals but then after rotational system, features of which are discussed hereunder.  Rotational Water Supply System is used in a broad sense to indicate a system of irrigation water distribution amongst the beneficiaries within the command to ensure timely, adequate and equitable distribution of water, on a weekly or on a multiple of week basis. It has the following advantages. In distributing, time based on size of holdings, time required for filling water course/field channels and emptying time should be taken into consideration and distributed among beneficiaries.

It is advantageous in areas with water stress and has the following merits also:

- Ensures supply of designed flow at all-off takes,

- Provides equal volume of water to each field by allocating the flow, on time based and rational calculation,

- Helps the beneficiaries to know the days, period and volume of irrigation water allotted to their field in advance, according to the crop seasons,

- Helps to pre-plan agricultural operations including inputs as security of irrigation to the beneficiaries is ensured,

- Ensures all the beneficiaries whether located at head, middle or tail end of the system, rich or poor, to get required quantity of water,

- Improves irrigation efficiency and minimizes wastage, there-by reducing water logging and salinity,

Any practice of rotational system is required to ensure participation and involvement of the beneficiaries through presentations and demonstration.

All the mains canals were designed for continuous flow. The rotation will be within the secondaryand tertiary units. In view of small holder nature of the scheme, the tertiary units were sized such that the unit is manageable with family labour only and not too low discharge to be sluggish on flatter slopes.

.

The main canal supplies directly to thesecondary canals in most cases on the left bank while secondary canals supply water to the blocks through the tertiary canals or field canals. Field canals supply water to corresponding field units. They are provided with division boxes at higher point of the tertiary unit/or turn out which supply the required amount of flow. Thefield drains and field canals/ditches are expected to be constructed by the direct project beneficiaries.

## Water Requirement

Irrigated agriculture is a combination of Engineering, Agronomic, Social and Economic endeavor involving the beneficiaries with their due importance in the formulation, implementation, operation and maintenance along with their overall association in the management and monitoring of the land and water use.

Operation is the organized procedure for causing a piece of equipment, a treatment plant, or other facility or system to perform its intended function, but not including the initial building or installation of the unit.  Therefore, the main objective of the operation of this irrigation scheme is the timely and equitable delivery of the required irrigation water to the recommendedfarms. This objective can be achieved by the following successive activities:

Organizing the beneficiaries;

Preparation of an irrigation schedule;

Operation of the water distribution according to the irrigation schedule;

Monitoring of the water distribution;

Adjustment of the scheduling, if required;

Management of conflict between the water users.

These activities have certain characteristics calling for particular kinds of management skills and styles of operation. Poor O & M contributes significantly to long-term salinity and water-logging problems and needs to be adequately planned at the design stage.

The most important service that the scheme operators provide to beneficiaries is the delivery of irrigation water. Ideal from a farmer's point of view is freedom in terms of:

Timing,

Flow-rate, and

Duration of irrigation applications.

The irrigation planning is done based on the water demand of the proposed crop and the expected available flow in the river.

Irrigation applications are intended to meet the water requirements of the crops cultivated in the scheme. If just one farm is considered, the irrigation requirement for the next week depends on:

The crops grown, and the area and growth stage of each crop;

The moisture conditions in each field;

The weather conditions expected for the coming week.

This means that a decision on the next week's irrigation needs cannot be made on the basis of precise knowledge alone; it also requires understanding/experience. Beneficiaries are making these types of decisions all the time on what crops to be grown, when to plant, how much and when to apply fertilizers and other agro-chemicals are all made on the basis of the beneficiaries' expectations and previous experience with such factors like rainfall, sunshine conditions, pests and other diseases overrun, crop yields and market-prices.

## Irrigation Scheduling And Operation Plan

### Irrigation Scheduling

The application of the precise amount of water required by the crops at the right time is the main achievement of the irrigation installation and hence project objective.Crop water requirement varies at different stages of crop growth. It is maximum during the initial growth and flowering period. Supply of timely water is necessary and required quantity is thus indispensible to obtain optimum yield.

Both over- and under-irrigation result in reduction of crop yield. Excess water application causes water logging, salinity, wastage of the valuable water and unnecessary strain on the system. It is necessary to assess the actual weekly crop water requirement in advance, considering the moisture depletion (using tenshometer& the like) and sustained crop-growth and prepare a schedule of irrigation, i.e. frequency of watering and quantum of water necessary. This will depend upon rainfall, crop development, soils, fertilizer, climatic conditions and crop factors.

The schedule shall thus be made known to all beneficiaries so that they are ready to utilize the water efficiently. This schedule should match the potentially availablewater. Generally, this is done at an interval of one week or multiple of weeks, to maintain the intended Rotational Water System.

Irrigation scheduling is the activity of making the program for the coming week (or 10 days, 2 weeks, one month, etc.) of the water distribution in the scheme during that period. There are two main reasons for preparing such program:

- Beneficiariesnormally understand matters concerning the main elements of irrigation programming, such as water discharge and rate, operating hours and irrigation frequency, and instructions.  Moreover, the scheme operators need to know when and how to adjust gate settings.

- Properly installed, operated and maintained irrigation networks enable beneficiaries to exercise absolute control over water use at farm level. Thus, it is easy for them to apply irrigation schedules based on crop, soil, weather, and water availability and quality factors.

Our procedure for irrigation scheduling depends on the design of the scheme, each farm in the scheme are grouped into tertiary and or field units which share a common outlet, the scheme operators will only prepare a schedule for water deliveries to the tertiary off-takes.The irrigation supply requirements at the field level are determined by the depth and interval of irrigation as described under.

### Depth of Application

The depth of application (d) which is the amount of water that will be supplied at each irrigation depends on:

Soil type,

Crop type and age and

How efficient is the irrigation project to minimize water losses. The frequency of irrigation turn depends on: the soil type, the depth of the root zone of the crop:

Thus, the soil type determines the maximum amount of water which can be stored in the soil per meter of depth. Clayey soils can store high amount of water or, in other words, clayey soils have high available water content. On clayey soils it will thus be necessary to irrigate less frequently with a high amount of water.

Root depths of cropscan also influence the maximum amount of water which can be stored in the root zone. If the root system of a crop is shallow, little water can be stored in the root zone and frequent - but small - irrigation applications are needed. With deep rooting crops more water can be taken up and more water can be applied, less frequently. Young plants have shallow roots compared to fully grown plants. Thus, just after planting or sowing, the crop needs smaller and more frequent water applications than when it is fully developed at the middle stage.

Irrigation depth (d, mm) is calculated based on the depletion factor (P) of each crop, available soil moisture (Sa), application efficiency, (Ea) of the project and effective root zone (D) of the individually selected cropat decade level. The detail is shown in the appendix table-10.

Net depth of water application per irrigation for a selected crop can also be determined from:

dnet = (FC - PWP) x D x P

Where:

dnet = 	Net depth of water application per irrigation for a selected crop (mm)

FC = 	Soil moisture at field capacity mm/m)

PWP = Soil moisture at the permanent wilting point (mm/m)

D = Depth of soil that roots exploit effectively (m)

P = Allowable portion of available moisture permitted for depletion by a crop before next irrigation

### Irrigation Intervals(II) of the project

The required interval for crop watering as to their requirement for each crop is calculated based on FAO CROPWAT software and shown in detail in the appendix table-10.

Where:

ETc= Kc*ETois crop evapotranspiration

Kc is crop coefficient (dependent on each crop growth stage)

ETo is potential evapotranspiration

# FLOW MEASUREMENT DEVICE AND PROCEDURES

Measurement of rate of flow of water delivered and applied is essential for proper management of an irrigated agriculture. Such volume of water is to be measured at specific points of head works, the main conveyance and off takes of tertiary canals.

The list of proposed location and frequency of observation is given in Table 5-1

Table 4-1: Location of Water Measuring Devices and Frequency of Discharge Observation

| Component | Location of Measuring device | Interval of Discharge observation | Purpose | Measuring Device | Reference |
| --- | --- | --- | --- | --- | --- |
| 1. Diversion Structure | Near the intake box | Weekly - 6.00 Am | To observe water level | Rectangular -notch& Gauge |  |
| 2. Main Conveyance | At the beginning of selected canals. | Weekly (as required) |  | V-Notch &Gauge |  |

Two types of notches are recommended. A V- notch and Rectangular notch. The V-notch is a notch made of steel plates having an angle of 900 between two sides. This will be portable type which will be used for measurement of flow near the fields.

Orifice is proposed for discharge measurement at the beginning of the main supply pipelines near the intake well of turnouts/off-takes. This can be done using the sluice gates on the main and secondary canals. The discharge through a sluice can be estimated by the following

Q= Cd* D*W*(2*g*h)0.5

Where

Q- Discharge in m3/s

Cd- coefficient of discharge- (0.6)

D- depth of gate opening in m

W- Width of gate opening in m

h- head in m (h1-0.62D)-for free flow

h=head in m =h1- h2 for submerged condition where

h1 and h2 are u/s and d/s heads above bed level

Figure 4-1: Typical Partial Flume

# OPERATION PROCEDURES AND METHODS

## Understanding the Recommended Application Methods

The final target of any irrigation scheme development shouldnot only involve design of good infrastructures and build quality structures but also target tothe best yield of the arrangement and achieve the targets of improved food production, social satisfaction, and improvement of economic status by preserving the environment for better ecology.

The associationof beneficiaries for proper and organized operation of such system is thus a necessity as described above. The members of this WUA are associated with the scheme from its inception stage and are aware of the functions of components of each structure. Irrigation water from the source and the systems connected from the delivery up-to escape has to be regulated, operated in such a manner that the scheme gives full benefit with a maximum coverage without wasting water, which otherwise create water logging or salinity problems.

The management is to be assisted right from theRegional Irrigation Agency for some time and handed over to the WUA, after training and capability building is completed.

Water is to be distributed as per water distribution plan to all farm areas, by regulating the turn outs/division boxes. In case higher water shortage of about 70% of design discharge, all tertiary canals will run at about 70% discharge,however the FSL in the tertiary canal should be maintained with shutters in division boxes/Box Off take to provide required depth of water on highest irrigable field.Alternatively the canal is to run at full discharge for restricted hours of flow i.e. for 70% of the duration and the distribution time has to be curtailed by 70% or so.

The distribution of water to the field canals is done through the gates of Box Off take. The calendar day of week should be made known to all beneficiaries when they are scheduled to receive water. Then thebeneficiaries will receive the same and carried through field drains from one field to another within the same farm area.

That farm area will be irrigated within a particular period proportionate to the area of the farm area and that of the Block (considering the number of working hours in a week).  The turnout shutter of the first farm area is to be closed and water released for the subsequent farm area where similar operation will be done for that particular period, proportionate to the area of that farm area. Realization of this will improve the level of water supply because the existing practice is characterized by too much application at a time and long frequency of irrigation which is wasteful particularly for soils with coarse grains and when there is high evapotranspiration.

This rotational supply is to be followed for all farm area one by one till the last turnout for the last farm area isoperated on the last period of the week but the full designed discharge of the tertiary unit is to flow in that farm area also.

It is to be noted that there may be some seepage or conveyance loss in the tertiary canal and the discharge is likely to dwindle down in the end reach. This may be compensated by giving some extra time to the last few farm areas or adjusting the time amongst the farm areas by the tertiary unit committee or as agreed by the beneficiaries.

## Surface Irrigation Methods

Efficient utilization of irrigation water involves several practices like conveying water from the source to the field with minimum seepage losses, following the right method of irrigation consistent with topography, applying water to the crop at right time and in proper quantity.

In surface application method, irrigation water is to be applied directly to the field-soil by the channel located at the upper reach of the field. The applied water may then be distributed to crops in border strips, check basins or furrows. However, the two general requirements of prime importance to obtain high efficiency in surface method of irrigation application are:

Properly constructed and managed water distribution system to provide adequate control of water to the field and;

Proper land preparation to permit uniform distribution of water over the field, if possible.

Surface irrigation will be practiced in the area by using one of the following methods:

Flooding method

Check basin method

Furrow method

Flood irrigation - Due to large slope of the area this method is not recommended.

### Basin Irrigation Method

This method is simple in principle and is the most common method of traditional irrigation in the area.  In this method the field is divided into smaller units (basins) so that each has a nearly level surface. It is important to cover the basin quickly and shut off when desired volume has been supplied. This method is suited to soils with slow intake properties and for deep-rooted and closely spaced crops, e.g. onion seed, & the like. Size of basin varies with the size of irrigation stream and size of land holding. The infiltration characteristics also influence size of the basin i.e. small basin is recommended in case of coarse soils such as sandy and sandy loam on the other hand large basin is recommended in the case of fine soils (clay).  Basins may be of square, rectangular or circular shape.

This irrigation method is suitable for the following:

For gentle and uniform land slope.

For row and close growing crops.

For grain and fodder crops in heavy soils where water is absorbed very slowly and is required to stand for relatively long time to ensure adequate irrigation.

For very permeable soils which must be covered with water rapidly to prevent excessive deep percolation at the U/S end.

When leaching is required to remove salts from soil profile.

This method enables conservation of rainfall and reduction in soil erosion and practically all irrigated crop (such as paddy) is grown in basins.

The method is not suitable for irrigated crops that are sensitive to wet soil conditions. Such crop plants are rather usually irrigated by furrows, e.g. vegetables.

### Furrow Irrigation

In furrow application method, the land is made into ridges and small gullies and the area is irrigated through all directions of furrows laterally. The size and shape depends on crop grown and spacing between crop rows. After infiltration, the water spreads sideways to irrigate the areas between furrows. The time of water flow depends on the amount of water required to replenish root zone and the infiltration rate.

The method is used to irrigate cultivated crops grown in rows such as sugarcane, cotton, Tobacco, potato, maize, sorghum, and vegetables. Furrow irrigation method is most suitable method if the crops are sensitive to the ponded surface water. It is suitable for most soils except sands that have a very high infiltration rate and provide poor lateral distribution of water between furrows.

Furrows have the following advantages:

Water in furrows contacts only one half to one fifth of land surface, thereby, reducing pudding and crusting of the soil and evaporation losses.

It is possible to advance sowing of a crop which is a distinct advantage in heavy soils.

It reduces labour requirement, inland preparation and irrigation.

There are four different kinds of furrows, viz. graded, level, contour and corrugated furrows.

Graded furrows are laid along the prevailing landscape. For the best irrigation efficiency the largest practical stream should be used to force the water across the field and then the stream is cut back to the size which will prevent or minimize run-off losses at the D/S end.

Level furrows are to be used in fine textured soils having very low infiltration rates. It is nearly levels lengthwise.  The same stream size is maintained until the required amount of water is applied. The water is pounded in the furrow until it is abstracted by the soil.

In contour furrows, water is carried across a sloping field rather than down the slope. Thus furrows are curved to fit such topography.

Furrow irrigation differs from border and basin irrigation methods in the pattern of wetting soil because the water which soaks into the soil spreads laterally to the adjacent areas.  In areas where surface drainage is necessary, the furrows can be used to dispose off run-off from rainfall.  When rainwater is to be conserved, furrows act as an effective means to catch and pressure the water from rainfall.

# METHODS AND PROCEDURES FOR MONITORING THE SYSTEM

## Overall

The scope of this chapter is to provide guidelines for collecting feedbacks from the concerned components regarding their behavior and working efficiency in order to establish and consolidate optimum output.

“Monitoring and evaluation” of operation and maintenance plan is a vital tool to watch the performance of each important link, collect information from the concerned wings, evaluate, integrate and coordinate them to evolve into uniform, undisturbed system of smooth functioning in achieving optimum food production, with a fixed quantum of water resource and also to improve the socio-economic ecology of the region.

Submission of reports, sending timely and correct information and at times physical examinations are the pre-requisites of a ‘Monitoring’ system. Taking into consideration of all information, the constraints are analyzed and directives that are acceptable to beneficiaries will be issued for the optimum results.

The monitoring system described in this manual aims to be mostly responsive to the nature of land-water distribution utilization and other allied functions in the system.  Not all the elements of the system will be immediately applicable in all cases.  The system is, therefore, to be flexible and adaptable to the needs of the schemes of differing sizes and varying characteristics.

Data from several sources must be drawn together to assess:

whether the plan is being followed as intended

whether it was successful in meeting its agricultural and economic objectives

In addition to establishing how successfully the scheme is running, reports should also recommend justifiable modifications for subsequent follow-ups.

## Levels Of Operation

Monitoring is to be done at the levels of the WUA where the outcomes, data, results will be discussed with adequate analysis and observations. The overall monitoring has to be done at the management level of government (Zonal /Regional) by Irrigation Agency’s appropriate wing that has to observe the performances, and take appropriate steps for improvements and the capability buildings of the beneficiaries and the sustainability of the scheme through inspection, review of reports.

## Data Requirement

Typically, the following data are required to monitor plan implementation and performance.

### Climate

Regular rainfall data are needed to allow response according to criteria set out in plan. It is better to establish the rain-gauge recorder in the catchment / command area.

### Water

Monitoring of water, water deliveries in relation to planned allocation is essential to control implementation, identify deviations, and evaluate performance; particular note must be made of failures in delivery. Monitoring of system loss, water logging, drainage, flow etc. are also important.

### Crops

Information on crop areas is essential to establish the extent to which plan is successful. In demand based system, water distribution responds to crop progress and regular within season returns may be required, as well as, the end of season report. Crop yields are also a measure of performance.

### Beneficiaries’ response

Information on beneficiaries’ response and their involvement in WUA is equally important in judging whether a ‘scheme-plan’ has been successful to improve future operations and management. In demand base system in particular, complete coordination and close contacts throughout the season between different groups in charge of the operation and regulation is very essential.

### Environmental assessment

This is very important as there could be enormous post project effects on ecology and increased river discharge which is detrimental in solving the water shortage. This has to be kept in mind and enduring efforts have to be made for its uplift.

## Reports And Returns Need

Keep the following as much as possible:

Permanent records: which should be maintained at site are discharge measuring records, gauge registers, operation schedule, regulation, etc.

Regular returns: to transfer information from the registers to managers of WUA within one season for taking further directions,

Periodic reports: e.g. seasonal or annual reports to summarize and analyze information collected over time through the registers and by other methods.  This includes the following:

### Agricultural performance

Beneficiaries view and activities regarding crop yields, system of O/M and management performance to be given in periodic reports separately;

Physical characteristics of soils, ground water, seeds fertilizers, extension programs etc.;

Inspection reports by the officers of different disciplines resulting in corrective actions.

### Annual performance

Report to establish the extent of success of all the components in general, and the overall scheme in particular, regarding their socio-economic development and crop yields. The extent of capability building of the beneficiaries should also appear in the report,

### Environment impact study

Report - the progress of mitigating activities, reasons for slow-down of any activity and the constraints have to be analyzed and reported.

## Methods &Procedures For Regular & Periodical Maintenance

### Overall

The efficiency and longevity of the scheme lies upon the nature of maintenance of the components involved in the irrigated agriculture and upkeep of the general ecology and environment.  Maintenance includes repairs as well and is better to classify as “Repair and Maintenance”.

The beneficiaries are to play the executive role in the operation and maintenance of the system.  After the development and completion of the small scale irrigation scheme, it will be transferred to the direct beneficiaries through their genuine WUA. The WUA may do it either by contributing voluntary labour (by motivating beneficiaries) or by subscribing and creating a fund to operate or by both.  However, it is quite possible that, at the initial stage, WUA’s could not be able to repair and maintain the system, to the desired level of performance, hence there should be a provision of some fund at least during the gestation period (2-3 years) to assist the WUA and the federal government has to frame a policy to realize such costs.

### Categories

The intensive maintenance requirement in the traditional system will be reduced considerably as part of the system (main supply) is now piped and the head work is stabilized.  Beneficiaries are expected to maintain their respective field canal.  Hence the need for maintenance is minimal if the structure is properly handled by the WUA. In the traditional system it used to take the whole beneficiary households (~100) three to four days.

The repair and maintenance may be categorized in to the following category.

- Regular

- Periodical

- Special repair/maintenance

- Emergency

Regular - It is done regularly, at intervals of week, month or the end of each crop season and beginning of the new crop season. It is done without any hindrance to the performance of the system.

Periodical - It is done once a year, before opening of the canal and also at the end of the canal closure and is done during non-crop season. Example – painting/changing defective gales and maintenance of damaged but functional structures.

Special Repair- It is done when some extra ordinary repair is needed. In this, extra cost is involved in addition to the normal repair maintenance cost, such as changing of trash racks, renovation of structures, changing gabions.

Emergency repair - This is done after some emergent problems, like canal breach, structure damage, gate jamming, etc. take place. The conveyance system has to be closed for attending to such repairs.

Organization for maintenance has been laid down in this manual under Chapter 2.  The same group of beneficiaries will be operative for repair and maintenance of the different systems.

A general chart is presented in Table 6-1 indicating nature of repair/ maintenance needed for different components.

Table 6-1: Normal Repair and Maintenance Chart upon Inspection

| Component | Interval for Inspections | Nature of Repairs and maintenance | Remark |
| --- | --- | --- | --- |
| 1. Catchment | Every three months | Watershed management works |  |
| 2 Intake | Daily during the opening & closing | Leakage through valves, fluctuations in water level (twice a year). | Recommend to a technical team when any damage is noticed |
| 3. Weir, Retaining & Service structure | Once a week. Thorough inspection especially during rainy season | Erosion, retrogression, and leakage through gates |  |
| 4. Infrastructures (main conveyance & distribution system) | Daily when canal is running for unauthorized use of water ( in the evening) | At turnout and Canal banks. Removal of debris &trashes Maintain field canals | Recommend to a technical team when any damage is noticed |
| 5 Structures | Bi-annual | Scour, stability and energy dissipation arrangements. | Recommend to a technical team when any damage is noticed |
| 6) Command area (General) | Once a month | Check water logging, Erosion of command area due to irrigation if any, drains, boundary bunds of irrigation blocks |  |
| 7. General -For the entire scheme | Irrigation Engineer, (once a year after rainy season) Environmentalist By agronomist (every crop season) Medical officers(twice a year) Monitoring | To check the structure, review reports and suggest measures for improvements. Watershed management works and general ecology Land soil management To check the general health, hygiene of the area and that of livestock’s. Assess the improvements and constraints, capability building and extension works |  |

Note:

As Built plans, maps and drawings prepared by the irrigation agency shall be provided to the WUA, which will assist in estimating maintenance cost, proper operation and maintenance including this manual translated in to local languages.

The patch repair works in the masonry or concrete should not be done with materials of lesser strength than that classified in the original work. Any settlement, crack or damages are to be properly communicated and technical advice taken for setting ways and means of rectification.

### Sources of Materials for Repair

Boulder and aggregates have to be brought from the approved quarry site.  Some stock of Boulder and aggregates is to be kept ready for use as indicated below:

Boulder                       -   	15 m3

Coarse aggregate       -         10m3

Sand for masonry       -   	5 m3

Cement                       -    	5 bags

Paint		            -    	5 liters

### BeneficiariesNeed for Maintenance

Three types of beneficiaries are needed for R/M activity:

Self-help - they are the beneficiaries themselves.  They do not need any type of technical or financial help.  Totally it is estimated that the project can benefit 193 households’.

Local support  -  help of crafts man like mason, plumber or a mechanic is needed for carrying out repair works and it may be met out of the financial resources of the WUA,

Specialized support - they are mostly Engineers or Environmentalist needed for regular yearly inspection after the rainy season on the initiative of the ‘Monitoring Evaluation Cell’.

### Communication System

There should be a regular communication system so that the WUA may contact the monitoring and evaluation cell or regional authorities, irrigation engineers for sending progress reports, emergency calls, Consultations etc.

There should be a record of crafts men, masons, as indicated above so that they may be contacted when necessary for attending repair works: The communication and approach to the Headwork site, should be kept in proper condition. The banks width of conveyance should be maintained for uninterrupted movement of the beneficiaries and R/M workers.

# TRAINING AND CAPACITY BUILDING

## TRAINING OF BENEFICIARIES

The beneficiaries selected to lead the WUA have to get knowledge and skill regarding the plan & functions of different components of structures in the scheme for its proper management, operation and maintenance. They have to be given training and developing knowledge and know-how, how best the completed system can give optimum results by utilizing the resources and the infrastructures and the same could be consolidated with minimum financial involvement.

The following training programmer shall have to be pre-planned:

At site training: - their involvement being right from the inception of the scheme, by association and group discussion.

Workshops - arranging workshops by inviting experts on different disciplines and allowing beneficiaries to participate on it.

## ON-FARM TRAINING

Making visits to other irrigated agriculture farms.

Audio or video and - the performance of Operation /maintenance at different levels in a particular on-going scheme can be demonstrated.

Through trainers/ trained within the WUA.

The training is necessary on the following aspects:

- Measurements of water,

- Maintenance works,

- Operation/regulation of gates and valves,

- Management of equitable water distribution,

- Behavior of structures,

- Assessment of periodic crop-water requirement,

- Need for Terracing works on steep slopes

- On how, when and how much to irrigate

- Water holding capacity of the soil

- Water logging, salinity and drainage,

- Up keep of watershed management activities,

- Communication and reporting,

- Monitoring and evaluation.

## TRAINING OF TRAINERS

While organizing training program for beneficiaries, proper “Trainers” are needed for conducting the programs. It is thus essential to emphasize the importance of “Training of Trainers”. As large number of trainers are to be mobilized and trained from different disciplines a detailed syllabus which emphasis on training techniques and its evaluation has to be developed for different levels. The aim of such training is to develop special skills to the trainers for planning, design of agriculture operation, and manpower, etc. so that the beneficiaries’ knowledge and skill can be enhanced through such trainees.

There are several approaches to organize and conducting a training programme. It is important to provide in-service training which is of adequate importance and the plans have to be so formulated that each trainer undergoes appropriate training on different subjects. They have to learn and educate themselves so as to enable them to train the beneficiaries for major items specified in this manual to achieve greater agricultural productivity with least hazards on social and environmental aspects. The main thrust areas of training may cover following topics:

### Basic

Organizational procedures and behaviours

Accounting rules

Irrigation Act

### Technical

Academic matters on all fields related to irrigated agriculture (such as soil/plant/water relationships, measurement of flow & its depth in canals, etc)

Practice oriented knowledge

Irrigation scheduling, distribution & budgeting

Operation and Maintenance of the scheme

# CHARGES FOR OPERATION AND MAINTENANCE

The cost incurred during operation and maintenance of different components depends upon their size, structural upkeep and items needing normal repair works.  It cannot be given a standard value.  However, after actual involvement of labour and material for two-three years this can be known on actual basis for the scheme.  The general items needing repairs have been identified in this chapter and it is easy to arrive at O/M charges each year according to the costs incurred there in for the entire command area.

Charges collected from penalties arising from non - compliance to rules of the WUA can also be used for this purpose. Consultation with the beneficiaries is important regarding water charges as they have been using it freely. If this is agreed upon, the collected amount can be used for O&M and purchased of inputs. Tentative water charge of 1,000Birr/ha can be envisaged considering affordability.  For unskilled labour requirement, no cost is to be assumed as the beneficiaries will be contributing. The association will set this in consultation with the beneficiaries.

# References

Feasibility and Detail Design Report of GewesoIrrigation and Drainage Project

Operation and Maintenance Manual – ESRDF, 2000

Source: FAO Irrigation & Drainage Manual, 2002

# APPENDICES

APPENDIX I: Typical Format for Flow Measurement at Intake

| Month | Week | U/S water depth (H)(cm) | Discharge(lps) | Aver. monthly flow(lps) | Remark |
| --- | --- | --- | --- | --- | --- |

Note:   H – Water depth above crest about a meter upstream of the notch crest

Read discharge from Appendix II

For V – Notch head reading is taken at the notch

lps – liters per second

APPENDIX II: Head - discharge Equation for Orifice

Q= Cd* D*W*(2*g*h)0.5

Where,

Q- Discharge in m3/s

Cd- coefficient of discharge- (0.6)

D- depth of gate opening in m

W- Width of gate opening in m

h- head in m (h1-0.62D)-for free flow

h=head in m =h1- h2 for submerged condition where, h1 and h2 are u/s and d/s heads above bed level

APPENDIX III: Head Discharge Curve for Portable v-Notch (900)

APPENDIX IV: Format for Record of Rainfall

| Month | Date (8:00 am) | Rainfall in mm | Remarks |
| --- | --- | --- | --- |
|  | Weekly total |  |  |

APPENDIX V: Format for Irrigated crops & their coverage

| Year | Crop groups | Block | Crops & Corresponding areas |  |  |  |  |  | Total | Planned | Reason for Shortage | Remarks |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Crop | Area (ha) | Crop | Area (ha) | Crop | Area (ha) | Area | CCA |  |  |
|  | Rainy season crops |  |  |  |  |  |  |  |  |  |  |  |
|  | Dry season crops |  |  |  |  |  |  |  |  |  |  |  |
|  | Perennial crops |  |  |  |  |  |  |  |  |  |  |  |

APPENDIX VI: Format for the Assessment of Conveyance Loss

| Month | Week | Discharge at headwork | Discharge at head of field canal * | Conve.losses | %Loss | Remarks |
| --- | --- | --- | --- | --- | --- | --- |

* Measured using portable V-Notch when required; %Loss= QField canal/QHeadwork

APPENDIX VII: Format for Recording Weekly Water Distribution and Irrigated Area

| Month | Week | Area to be irrigated |  |  |  | Area irrigated |  | Reason for shortfall in area irrigated |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Block | Turnout No | Field No | Hours | ha | % |  |

APPENDIX VIII: Annual Assessment of Crop Yields

| Year | Crop group | Crop | Area | Av.yield /ha | Total yield | Yield inthe previous year | Reason for shortfall | Remarks |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Group 1 | C1 |  |  |  |  |  |  |
|  |  | C2 |  |  |  |  |  |  |
|  |  | C3 |  |  |  |  |  |  |
|  | Group 2 | C1 |  |  |  |  |  |  |
|  |  | C2 |  |  |  |  |  |  |
|  |  | C3 |  |  |  |  |  |  |
|  | Group 3 | C1 |  |  |  |  |  |  |
|  |  | C2 |  |  |  |  |  |  |
|  |  | C3 |  |  |  |  |  |  |

APPENDIX IX: Format for reporting defects and actions taken (Biannual)

| Component | Serious Defects | Actions taken | Requirements |  | Remarks |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Materials | Labour |  |
| Watershed |  |  |  |  |  |
| Catchment ,, |  |  |  |  |  |
| Command ,, |  |  |  |  |  |
| Headwork |  |  |  |  |  |
| Intake chamber |  |  |  |  |  |
| Water points |  |  |  |  |  |
| Cattle trough |  |  |  |  |  |
| Shower area |  |  |  |  |  |
| Infrastructure |  |  |  |  |  |
| Conv. Pipe |  |  |  |  |  |
| Turnouts |  |  |  |  |  |
| Manholes |  |  |  |  |  |

APPENDIX X: Typical Irrigation Water use and service Agreement

Article 1 General: Through annual contracting agreement for irrigation water supply, the Contracting party enables to control water delivery and enables framers to produce the intended crop through securing suitable amounts of irrigation. This contract is a main contract to which specific agreements, can be added as well, at any time by the consent of both parties

.

Article 2 Contracting Entities: The contract for use of irrigable land located within the Irrigation Development Project BETWEEN THE undersigned _________

Article 3 DEFINITIONS: Throughout all the contract documentation, the terms listed below will have the following definition: (command area, Headwork, MC, SC, TC, FC, Service Road,  CD, TD, FD, structures, Stream discharge, Irrigation user, etc.)

Irrigation Charge means annual payments by the part of the beneficiary farmers for use of irrigable lands within the command area and whose objective is to recover invested capital, to cover annual fixed operational costs and allowances to cover future replacement costs.

Irrigable Land: means farm lands within the accessibility range of irrigation water and associated infrastructures;

Article 4 Duration: This contract enters effect as from its date of signature.

Article 5 Area Based charges: The annual area based charges would be calculated through multiplication of the established by the amount of irrigable land of the land holder. The beneficiary farmers enter into obligation of paying their annual charges irrespective of any conditions as scheduled by the responsible implementing body.

Article 6  Billing: Separate billing would be prepared for each of beneficiary farmers.

Article 7 Contracting Party’s Obligations: Payment of water rates: The cost of service charge is payable on time as agreed upon claims invoices within one month of issuing the invoice. Failing this, the authority shall be entitled to penalize the contracting party which would put in to effect through wereda council in reference to appropriate legal systems.

Article 8 Penalties: Failure to meet specified terms of payment shall entail an automatic penalty that goes to the last measure, fixed according to the legally applicable conditions.

Article 9 Damage to structure: Each contracting party is responsible in the event of damage to the structures and connections nearby its holdings for damages due to abused acts. The members of the users are responsible immediately if damage occurs by reporting the damage to the Woreda office or local police as found appropriate.

Article 10 Suspension and Termination of the Contract: If the contracting party fails to execute any of its obligations under this contract, a specific agreement or an accessory agreement, the authority can suspend the contract for certain period until such issue is resolved. The authority may terminate the contract by registered 2 letter of warnings sent one after the other with acknowledgement of receipt and having received no answer within fifteen (15) days.

Article 14 Dispute Resolution: Any dispute or difference arising out of the agreement including those considered as such by only one of the parties shall, in the first place be settled amicably. The parties shall use their best efforts to settle their disputes arising out of or in conjunction with this Agreement or the interpretation thereof amicably.  Any dispute or difference that cannot be settled amicably shall be referred to and settled by arbitration. However, beyond these steps, applicable Ethiopian law will come to effect.

Article 15 Signature: In witness thereof, the parties hereto mentioned their full names and have signed this Contract.

Signed for and on behalf of SSIP 	Singed by The users

Witness

1. 		                2.

Signed by Neighbors Witnesses	Signed by Neighbors Witnesses