Amhara National Regional State

Water, Irrigation and Energy Development Bureau

(BOWIED)

Feasibility Study and Detail Design

Of

Tena Diversion Medium Scale Irrigation Project

Volume V: Engineering Design Draft Final Report

April, 2018

Bahir Dar, Ethiopia

Client: Water Irrigation and Energy Development Bureau

(ANRSWIEDB)

Address:

P.O.Box: 88
Telephone: 0528-200853/855
Fax: 251-08-20-65-68/204676/202040

Consultant: Amhara Design & Supervision Works Enterprise

(ADSWE)

Address:

P.O.Box: 1921
Telephone: +251-582-181023/ 180638/181201/181254
Fax: (058) 2180550/ (058) 2180560
E-mail:amhara design@yahoo.com

Bahir Dar, Ethiopia

# FEASIBILITY STUDY & DETAIL DESIGN REPORT STRUCTURE

- Volume I: Watershed Management

- Volume II: Soil Feasibility Study

- Volume III: Engineering Geology and Geotechnical Report

- Volume IV: Irrigation Agronomy

- Volume V: Engineering Design

- Volume VI: Socio Economy

- Volume VII: Environmental Impact Assessment

- Volume VIII: Financial and Economic Analysis

# TABLE OF CONTENTS

FEASIBILITY STUDY & DETAIL DESIGN REPORT STRUCTURE	i

TABLE OF CONTENTS	ii

LIST OF TABLES	vi

LIST OF FIGURES	viii

SALIENT FEATURES OF PROJECT	ix

1	INTRODUCTION	1

1.1	Background	1

1.2	Project Area Description	1

1.3	Objectives of Study	2

1.3.1	Specific Objectives	3

1.4	Scope of the Study	4

1.5	Methods of Project Study	4

2	HYDROLOGY	6

2.1	Watershed Characteristics	6

2.2	Hydro Metrological Data	7

2.2.1	Climate data	7

2.2.2	Rainfall Data	7

2.2.3	River flow data	7

2.3	River Water Resource Utilization and Balance	8

2.4	Design Flood Frequency Analysis	8

2.4.1	Data Quality Test	10

2.4.2	Frequency Analysis of Design Rain fall	11

2.4.3	Goodness of Fit Test	14

2.4.4	Peak Flood Discharge Analysis	16

2.4.4.1	Time of Concentration	16

2.4.4.2	Curve Number (CN)	18

2.4.4.3	Areal Rainfall and Rainfall Profile	19

2.4.4.4	Direct Runoff Analysis Using SCS-CN Method	19

2.5	Tail Water Rating Curve Computation	22

3	HEADWORK DESIGN	25

3.1	Head work site Selection	25

3.1.1.1	River Bed condition	27

3.1.1.2	River Banks condition	27

3.1.1.3	Main Canal Routes	28

3.1.1.4	Sources of Natural Construction Materials	28

3.2	Headwork Type Selection	29

3.3	Hydraulic Design of Headwork Structure	30

3.3.1	Weir Height Determination	30

3.3.2	Weir Dimensions	30

3.3.3	Weir Hydraulics and Flood Level Determination	31

3.3.4	Ogee Shape of the Weir	32

3.4	Hydraulic Jump Calculation	34

3.5	Structural Design of Headwork Structure	36

3.5.1	Stability Analysis of weir	36

3.5.2	Impervious Apron and Cutoff Wall	38

3.5.3	Design of Retaining Walls and Divide Walls	38

3.5.4	Design of Under Sluice and Canal outlet	39

3.5.4.1	Under sluice	39

3.5.4.2	Canal outlets	40

3.5.4.3	Breast Wall and Operation Slab	41

4	IRRIGATION AND DRAINAGE SYSTEMS DESIGN	42

4.1	Irrigable area description	42

4.1.1	Topography	42

4.1.2	Climate	42

4.1.3	Soil characteristics	43

4.1.4	Existing Irrigation Practices	43

4.2	Irrigation water requirement	43

4.2.1	Crop Water Requirement (CWR)	43

4.2.2	Calculation of reference evapotranspiration (ETo)	43

4.2.3	Irrigation efficiency (Ep)	44

4.2.4	Irrigation duty	44

4.2.5	Irrigation methods	45

4.3	Irrigation and drainage system layout	45

4.3.1	Conveyance System	46

4.4	Design of the Irrigation canal system	46

4.4.1	Main Canal	46

4.4.2	Secondary Canals	47

4.4.3	Tertiary canals	48

4.4.4	Field Canals	50

4.5	Canal structures design	51

4.5.1	Division boxes	51

4.5.2	Turn outs	54

4.5.3	Drop Structures	56

4.5.4	Cross-Drainage Structure	57

4.5.5	Irrigation Drainage Works	58

5	BILL OF QUANTITY	61

5.1	BOQ Summary	61

5.2	Bill of Quantities	61

6	OPERATION AND MAINTENANCE	71

6.1	General	71

6.2	Operation of the Head Works	71

6.3	Irrigation System Operation	71

6.4	Maintenance Requirement	71

7	CONCLUSION	73

8	REFERENCES	74

9	ANNEX	75

9.1	Stability Analysis of Wing Walls	75

9.2	Stability Analysis of Divide wall	76

9.3	Stability Analysis of Canal Retaining Wall	77

# LIST OF TABLES

Table 2-1Slope Classes of the watershed	6

Table 2-2Maximum daily rain fall data records of Addis Zemen station	9

Table 2-3Maximumrainfall frequency analysis using Gumbel’s EVI distribution	12

Table 2-4 Maximum rainfall frequency analysis using Log Pearson-III distribution	13

Table 2-5Maximum rainfall frequency analysis using Lognormal distribution	13

Table 2-6Maximum rainfall frequency analysis using Normal distribution	14

Table 2-7Summary table of D-index test of goodness of fit test,	16

Table 2-8 Summary of Tc computation byKirptich formula	17

Table 2-9Summary for catchment hydrograph Parameters	18

Table 2-10 Design Rainfall Arrangement	19

Table 2-11 Runoff analysis	21

Table 2-12 Synthesis of Complex Hydrograph	21

Table 2-13 Tail water rating curve computation	23

Table 3-1 Table of weir height determination	30

Table 3-2 Stability analysis of weir	37

Table 3-3  Summary of retaking and divide walls dimension	38

Table 4-1Hydraulic Parameters of Main canals	47

Table 4-2Hydraulic Parameters of Secondary Canals	47

Table 4-3Hydraulic Parameters of Tertiary Canals	49

Table 4-4: Hydraulic parameters of Division Boxes	53

Table 4-5 Hydraulic parameters of Turn Outs	55

Table 4-6  Summary of drop hydraulic parameters	57

Table 4-7Summary of flume structures	57

Table 4-8  Summary of Catch drain hydrology	58

Table 4-9: Hydraulic parameters of Catch drains of Main Canals	59

Table 5-1:Summary of Bill of quantity	61

Table 5-2: Bill of quantity of General item, Head work and Infrastructure	61

Table 9-1 Stability Analysis for U/s Wing Walls	75

Table 9-2 Stability Analysis for D/s Wing Walls	76

Table 9-3 Stability Analysis of Divide walls	76

Table 9-4  Stability Analysis of MC01 Retaining wall	77

# LIST OF FIGURES

Figure 1-1  Project Area Location Map	2

Figure 2-1 Graphical plot of data consistency test by outliers	11

Figure 2-2 Probability plot of different probability distributions	15

Figure 2-3 hyetograph of design rainfall	20

Figure 2-4Complex synthetic triangular hydrograph	22

Figure 2-5  Longitudinal average river bed slope	23

Figure 2-6 Tail water rating curve and maximum flood level	24

Figure 3-1 River bed at the proposed weir site	26

Figure 3-2 Geological cross section at Head work site	26

Figure 3-3 Typical ogee profile of control section (Novak, 2007)	33

Figure 3-4  Ogee profile of downstream face	34

Figure 3-5 Comparison of Jump height curve & Tail water rating curve	36

Figure 4-1 Typical Cross Section of tertiary canals	49

Figure 4-2Typical Field Canal X-section	51

Figure 4-3Typical Division Box plan	52

Figure 4-4Typical Cross Section of Catch drain canals	59

# SALIENT FEATURES OF PROJECT

- Project name: Tena Diversion Medium Scale Irrigation Project

- Name of the stream: Tena  River

- Location of the weir  site

- E:377540.324m (UTM)

- N:1364626.187m (UTM Adinadan)

- Altitude: 1805m amsl

- Zone: North Gondar

- Wereda: West Belesa

- Kebele: Ashkertera

- Hydrology

- Design rainfall: 118.616 mm

- Catchment area: 92.09 km2

- Longest flow path length: 22.766 Km

- Design flood: 241.18 m3/sec

- Design base flow: 451 l/se.

- Diversion Weir

- Weir type: Ogee Weir

- Height: 4.5 m

- Gross crest length: 17.0 m

- Weir crest level: 1805.407 m.a.s.l

- U/S HFL: 1808.225 m.a.s.l

- U/S TEL: 1808.972m.a.s.l

- D/s TEL: 1806.456m.a.s.l.

- D/s HFL: 1806.819m.a.s.l

- Under sluice

- Sill level: 1803.807 m.a.s.l

- Dimension: 1.0m x1.6mfor both left and right sides under sluices,

- Outlet

- Cana Sill level: 1804.807 m.a.s.l for both right and left off takes.

- Opening dimension: (0.70m X 0.60m) and (0.20m X 0.40 m) for right  and left side off take,

- Discharge capacity: 0.431 m3/s and 0.0615 m3/s for right and left side canal off takes,

Irrigation and drainage systems Infrastructure

- Command area size: 400ha (350 ha on right side & 50ha on left side),

- Type of soil of the command area is dominantly clay soil

- Method of irrigation: surface irrigation (Furrow)

- Maximum irrigation duty  1.23 l/s/ha for 14 hr/day irrigation hours of supplementary irrigation,

- Design discharge of the main canals: 0.431 m3/s and 0.0615 m3/s for right and left side main canals,

- Irrigation system layout consists of two main canal (right MC is 2.74km, Left MC is 2.96km), 6.97km secondary lined canals and 14.05km earthen Tertiary canals.

- Main irrigation structures designed includes; Division boxes, turnouts, flumes and supper passage,

Project cost

- General Items cost = 1,197,139.64 Birr

- Headwork cost =    2,865,880.80 Birr

- Irrigation Infrastructure cost =25,962,935.11Birr

- Total project cost =  34,529,848.88 Birr

- Cost per Hectare = 86,324.62 Birr

# INTRODUCTION

## Background

In Amhara region of Ethiopia, under the prevalent rain-fed agricultural production system, the progressive degradation of the natural resource base, especially in highly vulnerable areas of the highlands coupled with climate variability have aggravated the incidence of poverty and food insecurity. The major source of growth for Ethiopia is still conceived to be the agriculture sector. Hence, this sector has to be insulated from drought shocks through enhanced utilization of the water resource potential of the country through development of small-scale irrigation.

In line with the above, in the area of irrigation efforts have been made by the government to improve the situation in the country. Amhara Water Irrigation and Energy Bureau is playing its role in the development of small, medium and large scale irrigation projects in the region.

Accordingly, as part of the water sector development program, the bureau has initiated the study and design of a medium scale irrigation scheme on Tena River and signed an agreement with Amhara Design & Supervision Works Enterprise (ADSWE) for the study and design of the project.

The feasibility study and design of Tena Medium Scale Irrigation Project was investigated and proposed for River Diversion by ADSWE in 2017 G.C. The feasibility study of Tena Diversion Medium Scale Irrigation Project has been established to irrigate agricultural land found in two kebeles of West Belesa woreda in North Gondar Zone.

## Project Area Description

The Tena Diversion irrigation project site is located at Ashkertera Kebele in the West Belesa woreda of North Gondar zone part of Amhara Region.  The project site can be accessed driving for about 17 km on the main route from Arbaya to Guhala and turning south eastern direction on the new URRAP road for about 8 Km towards Ashkertera Kebele center. At the end of the 8 km distance  0.5 km has to be traveled on foot because of the farm land and stream to be crossed to reach the site.

The Tena diversion project headwork site can be located with approximate of UTM-Adindan geographical coordinate of 377540.324m E, 1364626.187m N and with an average altitude of 1805 m AMSL.

Figure 1-1  Project Area Location Map

The main watercourse of the Tena River runs from near to upper central Tekeze basin highlands join to the main Mena River and then it flows north ward to the Tekeze River. The Tena river catchment at diversion site covers about 92.09 km2area. It rises on the upper south western ridge of Tekeze River basin oriented to the northern direction to join the main Mena river and then to Tekeze river inTekeze basin.

## Objectives of Study

The project area faces variability of rainfall distribution though the overall rainfall generally suffices the rain-fed agriculture and also the command area of the project is degraded by human factor. Accordingly, the rain-fed agriculture needs means of supplementing during rainfall distribution failures and further full irrigation is required to maximize the use of the potential land and water resources.

Hence the objective of this project is to contribute a substantial share in the effort to reduce the risk of production decrease due to rainfall variability and increase the productivity of the resource in the project specific area. Specifically, the project is targeted for the following.

- To make sustainable the rain-fed crop production and make extra production in the dry season possible for more than 256 ha of land through irrigation.

- There is a general consensus that irrigation investments will achieve broader food security and poverty reduction impacts and if efforts are also geared towards up-grading existing traditional farming practices with support to enhance access to input supply, output marketing and extension to facilitate access to information and innovations.

These project objectives are to be realized by designing and constructing diversion structures across the Tena River and diverting the river flow.

### Specific Objectives

Other benefits that can be expected to appear with the launching of the project are:

- Efficiency of water use improvement;

- Improved local nutrition/food security gains;

- Improved management of scarce natural resources (land and water);

- Resilience against drought risk;

- Rationale for erosion control and watershed management;

- Rationale for the intensification and modernization of small-holder agriculture and rural lifestyles.

The engineering study and design enables the realization of the project by the provision of engineering structures that will allow the appropriate abstraction of the river water for delivery in to the identified irrigation fields of the study area. Hence, this engineering design is specifically targeted:

- To analyze hydrologic requirements of the project and engineering structures;

- To formulate of sound and stable structure,  with necessary provisions that allow safe, easy and easy for maintenance operation in the service life of the project;

- To develop working drawings and specifications,

- To estimate of construction costs.

## Scope of the Study

The scope of the Tena diversion irrigation projects study and design goes up to gathering hydro climatic data, hydrological analysis and estimation of the river base flow and design flood magnitude. Accordingly it is possible to determine the appropriate size of the diversion headwork and related appurtenant structures. Beside to this, the study includes Planning and layout of proper irrigation infrastructures system and designing for their hydraulic and structural parts. After all design document preparation including working drawing, Bill of Quantities and specifications.

## Methods of Project Study

In order to achieve the study and design of Tena Diversion irrigation project we have employed the following methods of project study.

- Specific Site identification:

  - Review of the reconnaissance Survey conducted by the Client

  - 50,000 scale top map and GIS information

  - Local farmers interview and discussion

  - Wereda and Zone Agriculture section expertise

  - Previous studies

  - On foot travel along the river canal and farm areas.

- Topographic Survey:

  - Surveying the headwork site and the Command area with sufficient radius, using Differential GPS and Total station,

- Flow estimation

  - Physical observation on flood mark indications and local information about high flood and critical flow condition of the river

  - Analyzing the recorded river flow data and use watershed inputs for further analysis.

  - Base flow estimated during the reconnaissance field visit by floating method.

- Irrigable command area identification:

  - Using local information

  - 50,000 Topographic map, and GIS information, GPS to see elevation

The design report is organized in three sections. In chapter two the Hydrology study is presented and in chapter 3 and 4 the Headwork and Irrigation and Drainage Systems designs are discussed respectively. In Section III, planning and design of the irrigation system after diverting the water using the weir will be dealt.

The following are major areas of concern in this part.

- Study and design of the irrigation method to be adopted,

- Study and design of the irrigation system layout and associated structures,

- Design of the different conveyance canals,

- Planning and design of the different irrigation and drainage structures,

- Preparation of the longitudinal profiles of the different irrigation and drainage canal,

# HYDROLOGY

## Watershed Characteristics

Tena Watershed has marked topographic variation. All types of slope classes are present. The dominant slope class is moderately steep slope (15-30%) which covers 26.7% of the total area followed by steep class (30-50%) which is 21.13%.   The area coverage of other class is also shown in table below.

Certain physical properties of watersheds significantly affect the characteristics of the runoff and sediment yield and are of great interest in hydrologic analyses. The rate and volume of runoff, and sediment yield from the watershed have much to do with shape, size, slope and other parameters of the landscape. These suggest that there should be some important relations between basin form and hydrologic performance. If the basin and hydrologic characteristics are to be related, the basin form must also be represented by quantitative descriptors. These parameters can be measured from maps.

Table 2-1Slope Classes of the watershed

| Description | Class (%) | Area (Ha) | Proportion (%) |
| --- | --- | --- | --- |
| Flat or Almost Flat | 0-3 | 245.8558 | 2.67 |
| Gently Sloping | 3-8 | 1168.762 | 12.69 |
| Sloping | 8-15 | 1481.582 | 16.09 |
| Moderately Steep | 15-30 | 2458.89 | 26.70 |
| Steep | 30-50 | 1945.921 | 21.13 |
| Very Steep | 50-202.44 | 1908.205 | 20.72 |
| Total |  | 9209.2158 | 100.00 |

The watershed characteristics are analyzed and presented in Table 2.1 of the Watershed Feasibility Study Report of the same project.

The summary of Tena watershed characteristics are pointed below:

- Catchment Area  = 92.09km2

- Stream Length = 22.766Km

- CN(II) = 80.93 These important watershed elements are abstracted from the Watershed Study Report of the same project.

## Hydro Metrological Data

### Climate data

It is common that irrigation project designers and planners face with lack of good data on the hydrology of the stream/river system that can be the water source. Stream gauging stations are virtually non-existent in remote rural area part of Ethiopia; meteorological stations are almost rare. Likewise, Tena Diversion irrigation project has not meteorological station with in its catchment area. Moreover, there are no stream flow data records for the river near by the project site at all.

A Thessen polygon map was developed to subdivide the study watershed by the neighboring rainfall stations of Addis Zemen, Arbaya, Guhala and Makisegnt.  The Thesen polygon map depicts that about 74.77% the upper most part of Tena watershed area is shared by the Addis Zemen station while the remain 25.23% of watershed area, lower part,  is segmented by the Arbaya station.

Rainfall data were collected from the National Meteorological Service Agency (NMSA) for the two stations (Addis Zemen & Arbaya) and are assessed for missing data.  The Addis Zemen metrological station has daily rainfall for a period from 1997 to 2016 with a record length of 20 years. Whereas the Arbaya station has daily rainfall for a period from 2004 to 2013 but it has only 5 years of complete record.

Addis Zemen station with 20year daily rainfall data records shares about 74.77% of highland part of study watershed area which can generate high rainfall magnitudes. Therefore, for analysis of the design flood magnitude we have used the Addis Zemen station hydro-meteorological data. Even if the Arbaya metrological station shares the lower part of watershed including command area, Because of data inadequacy we have use the Addis Zemen station Crop water demand analysis.

### Rainfall Data

In order to compute the design flood for the Diversion structure, the daily maximum rainfall is collected from Addis Zemen station Metrological stations with a record of 20 years.

### River flow data

The base flow which is measured twice in year. Those are during the site identification and detail feasibility periods as 400 l/s by 14/06/2009 EC and 502 l/s by 09/09/2009 EC respectively. Since the 502 l/s base flow is measured during the dry months of the year, this figure is adopted for design.

## River Water Resource Utilization and Balance

About 3.5 Km distance in the upstream of the proposed headwork site there is one modern irrigation project studied by the year 2015 and expected to be implemented in the current year. This project called Agamwuha diversion irrigation project with design base flow of 100l/s to develop about 60ha command area. Similarly, appreciable downstream environmental release needed of water is anticipated for locals and cattle provisions. Therefore, the minimum 10.0 l/s flow is considered to be released for downstream environmental requirements.

In designing, the Tena diversion irrigation project, the water balance is done by using the average measured base flow 451 l/s (502 l/s measured during detail study and 400l/s during identification study). By distributing it to 100 l/s and 10 l/s for upstream user’s and environmental release respectively. This is not to cause water stress and environmental influences. Therefore, Tena River net dry base flow is determined to be 341 l/s.

The downstream Mena Pump Irrigation project studied by year 2016 has a design base flow of 830 lit/s with a total river base flow of 1500 lit/s. This has surplus base flow of 670 lit/s. This can ensure the project cannot be affected by the upstream Tena Diversion project with a total base flow 451 lit/s.

Tena project has total potential command area of greater than 400 ha with 50 ha on left and 350 ha on the right sides.  The agronomy report reveals that the project has 1.33 and 1.23 l/s/ha irrigation duty for full and supplementary irrigation seasons.

So the dry season irrigable land is 256.39 ha by considering the above net dry base flow and full irrigation duty. But for supplementary irrigation, the net wet flow that is estimated 451 l/s can irrigate the entire 400 ha net total potential command area. The capacity of all infrastructure presented in chapter 4 are designed based on this supplementary flow.

## Design Flood Frequency Analysis

For the analysis and design of diversion structures to be constructed on the river estimation of flood magnitude is an important task. This can be done using different techniques depending on the available data. For this particular case, there is no river flow data and hence the flood estimation is done using the rainfall data frequency analysis and applying SCS Curve Number method.

For design adequate headwork and appurtenant structures of Tena Diversion irrigation project, the 20 year daily maximum rainfall data of Addis Zemen station is used for flood frequency analysis over 50 year return period.

Table 2-2Maximum daily rain fall data records of Addis Zemen station

| No. | year | Daily max rainfall X, mm | Descending Order(X) | Log transfer Y=log X |
| --- | --- | --- | --- | --- |
| 1 | 1997 | 42.50 | 100.00 | 1.63 |
| 2 | 1998 | 32.40 | 98.00 | 1.51 |
| 3 | 1999 | 42.00 | 87.80 | 1.62 |
| 4 | 2000 | 46.50 | 69.00 | 1.67 |
| 5 | 2001 | 27.00 | 57.00 | 1.43 |
| 6 | 2002 | 53.20 | 53.20 | 1.73 |
| 7 | 2003 | 38.00 | 48.40 | 1.58 |
| 8 | 2004 | 48.40 | 48.40 | 1.68 |
| 9 | 2005 | 57.00 | 48.00 | 1.76 |
| 10 | 2006 | 69.00 | 46.50 | 1.84 |
| 11 | 2007 | 35.00 | 43.50 | 1.54 |
| 12 | 2008 | 100.00 | 42.50 | 2.00 |
| 13 | 2009 | 42.00 | 42.00 | 1.62 |
| 14 | 2010 | 98.00 | 42.00 | 1.99 |
| 15 | 2011 | 48.40 | 38.50 | 1.68 |
| 16 | 2012 | 48.00 | 38.00 | 1.68 |
| 17 | 2013 | 87.80 | 35.00 | 1.94 |
| 18 | 2014 | 33.70 | 33.70 | 1.53 |
| 19 | 2015 | 38.50 | 32.40 | 1.59 |
| 20 | 2016 | 43.50 | 27.00 | 1.64 |
| sum |  | 1,030.90 | 1,030.90 | 33.66 |
| Mean (Xm) |  | 51.55 | 51.55 | 1.68 |
| Standard deviation (Sx) |  | 21.10 | 21.10 | 0.16 |
| Skewness (Cs) |  | 1.41 | 1.41 | 0.79 |

As shown in the above table, the following summary statistics are determined form the 20 year daily maximum rainfall.

- Number of data records, N=20,

- Mean maximum daily rainfall, Xm=51.55mm,

- Mean of log transferred maximum daily rainfall, Ym=1.68mm,

- Standard deviationof maximum daily rainfall, Sx=21.10mm,

- Standard deviationoflog transferred maximum daily rainfall, Sy=0.16 mm,

- Coefficient of Skewness for maximum daily rainfall, Cs= 1.41

- Coefficient of Skewness for log transferred maximum daily rainfall, Cs= 0.79

Before design rainfall frequency analysis, the data quality of the recorded maximum daily rainfall data should be checked.

### Data Quality Test

Data quality test is a preliminary analysis before proceeding further hydrological analysis. In general hydrological data records may exhibit gaps which makeit less reliable. Causes can be Instrument failure, personal miss reading and other environmental factors. In this study the data quality test is conducted by checking for data variability and outlier tests.

The data reliability of maximum daily rainfall data record is checked by variability test. This can be determined by computing the Relative standard which is the ratio ofStandard error of mean to the mean of data series.

Standard error of mean, =

Relative standard,

Where; Sx-Standard deviation, Xm-mean and N-number of data point.

The computed value of Relative standardis 9.15% which is found less than 10 %. This indicate that the data series can be regarded as reliable and adequate for design flood frequency analysis.

Similarly, the data consistency of maximum daily rainfall data record must be checked for both lower and higher outlier tests. The U.S. water resource council recommends to the adjustment to be made for outlier. Outliers are data points that depart significantly from the threshold values of the data series.The retention or deletion of these outliers can significantly affect the magnitude of statistical parameters as mean and standard deviation that are computed from the data, especially for small samples. The outliers are practiced to sort out the data that seem reliable of the trend of the parent distribution for doing further hydrological analysis.

The frequency factor based outlier mathematical equation can be written as follows.

For lower outlier:      Yl= Ym-KnSy

For higher outlier:  Yh= Ym+KnSy

Where           Kn-is frequency factor depends on the number of data which is 2.385 for 20 no data,

Sn- is the standard deviation of log transferred data which is 0.16.

Ym- is the mean of log transferred data which is 1.68.

By substituting these inputs into the above outlier equation and computing antilogarithm, the lower and higher outlier values are estimated as 20.45mm and 113.61mm respectively.

The data set can be kept for further hydrological analysis if it is in between the lower and higher outlier boundary values.

Figure 2-1 Graphical plot of data consistency test by outliers

As shown in the outlier analysis and outlier graph, there is no augmented outlier data observed in the data record both for lower and higher outliers. So the 20 year daily maximum rainfall data series can be regarded as consistent and reliable enough for further hydrological analysis.

### Frequency Analysis of Design Rain fall

In hydrological frequency analysis, the annual series of extreme values can be fitted by a certain parent probability distributions model. Among a number of parent probability distributions models Gumbel (EV-I), Log Pearson type-III, Lognormal and Normal distributions are widely applicable in hydrological frequency analysis (Baban, 1995).

Since Tena River is ungauged the 20 year daily maximum rainfall data series of Addis Zemen station is used for flood frequency analysis over 50 year return period.

The design rainfall frequency analysis is computed for the commonly applicableprobability distributions models including Gumbel (EV-I), Log Pearson type-III, Lognormal and Normal distribution and finally the best fit probability distribution will be selected.

The calculated statistical summaries of 20 year maximum daily rain fall data records of  Addis Zemen station is provided in the table 2.2 and it will be continuously referred in doing the design  rainfall frequency analysis for all methods presented below.

- Gumbel (EV-I) Extreme value type I Distribution

Gumbel (1958) introduce the concept of extreme value distribution and developed a model for predicting hydrological extreme events like flood peaks and maximum rain fall.

Using data provided in table 2.2, the design point rainfall is calculated based on Gumble extreme value type-I method as following:

Table 2-3Maximumrainfall frequency analysis using Gumbel’s EVI distribution

| Station Name | Addis Zemen |
| --- | --- |
| Number of data, N | 20 |
| Return period, T years | 50 |
| Standard deviation, Sx | 21.10 |
| Mean of data, Xm | 51.55 |
| Reduced varate, Yt=-ln(ln(T/(T-1)), | 3.902 |
| standard mean, Yn=f(N) | 0.5236 |
| Standard varate, Sn= f(N=20) | 1.0628 |
| Frequency factor , KT=(YT-Yn)/Sn | 3.179 |
| Peak rainfall, XT=Xm+KT* Sx | 118.616 |

The Gumbel’s EVI distribution method of design rainfall frequency analysis gives us118.616 mm peak design rainfall for 50 year return period.

- Log Pearson type-III Distribution

This distribution can be done after the normal series of data are taken out of logarithmic value and then it has the same procedure as that of any theoretical frequency analysis. The person type-III distribution frequency factor depends on the Coefficient of skewness that data series gives and design return period.

Table 2-4 Maximum rainfall frequency analysis using Log Pearson-III distribution

| Station Name | Addis Zemen |
| --- | --- |
| Number of data, N | 20 |
| Return period, T years | 50 |
| Standard deviation, Sy | 0.16 |
| Mean of data, Ym | 1.68 |
| Coefficient of skewness, Cs | 0.794 |
| Frequency factor , KT=f(T,Cs) | 2.45 |
| Peak magnitude, YT=Ym+KT* Sy | 2.07 |
| Peak rainfall, XT=Antilog(YT) | 116.39 |

The Log Pearson-III distribution method of design rainfall frequency analysis gives us116.39 mm peak design rainfall for 50 year return period.

- Lognormal Distribution

This distribution can be done after the normal series of data are taken out of logarithmic value and then it has the same procedure as that of any theoretical frequency analysis. The frequency factor of the lognormal distribution depends on the design return period at zero coefficient of skewness.

Table 2-5Maximum rainfall frequency analysis using Lognormal distribution

| Station Name | Addis Zemen |
| --- | --- |
| Number of data, N | 20 |
| Return period, T years | 50 |
| Standard deviation, Sy | 0.16 |
| Mean of data, Ym | 1.68 |
| Frequency factor , KT=f(T,Cs=0) | 2.054 |
| Peak magnitude, YT=Ym+KT* Sy | 2.00 |
| Peak rainfall, XT=Antilog(YT) | 100.87 |

The Lognormal distribution method of design rainfall frequency analysis gives us100.87 mm peak design rainfall estimated for 50 year return period.

- Normal Distribution

Friedrich Gauss (1810) introduce the normal probability distribution for the frequency analysis of extreme values. This distribution is more preferred for symmetrical or non-skewed data sets and it can be usually used for frequency analysis of hydrological extreme events like flood peaks and maximum rain fall.

Table 2-6Maximum rainfall frequency analysis using Normal distribution

| Station Name | Addis Zemen |
| --- | --- |
| Number of data, N | 20 |
| Return period, T years | 50 |
| Standard deviation, Sx | 21.10 |
| Mean of data, Xm | 51.55 |
| Skewness, Cs | 1.41 |
| Frequency factor , KT=f(T,Cs,N) | 2.054 |
| Peak rainfall, XT=Xm+KT* Sx | 94.88 |

The Normal distribution method of design rainfall frequency analysis gives us 94.88mm peak design rainfall for 50 year return period.Eventually, among the four probability distribution, the best fit will be selected based on goodness of fit test.

### Goodness of Fit Test

After doing the frequency analysis of the peak rainfall using different distributions methods, we should select the best fit distribution to the maximum rainfall data series.

Among the four probability distributions (Gumbel (EV-I), Log Pearson type-III, Lognormal and Normal distributions) the best fit distributions can be identified by applying the Probability plot and D-index test.

Probability plots are used to determine how well data fit a theoretical distribution, such as the normal, lognormal, or gamma distributions. It is widely used by hydrologists to compare the best fit probability distribution to the given data series. Probability plots can be constructed by plotting the quantiles of sample data against quantiles of the standardized theoretical distribution on the probability paper. The probability paper simply retransforms the linear scale for quantiles of the standard distribution back into a nonlinear scale of plotting positions (D. R. Helsel, and R. M. Hirsch, 1992).

In the probability plot, the less departure from a straight line implies the better the data fit the theoretical distribution.

Based on probability plot method a distribution with minimum value of AD (Anderson-Darling) test statistics is found the best fit probability distribution.

Figure 2-2 Probability plot of different probability distributions

The probability plot in figure 2.2 shows that the lognormal and Gumbel-EVI distributions are found the best fit in the order of 1st and 2nd rank among the four probability distribution methods used for daily maximum rainfall for frequency analysis.

Similarly, the D-Index test can be applied to measure the goodness fit of the data series to frequency models. In our case the D-Index test is used to determine the best statistical distribution to estimate the peak rainfall. The D-index for the comparison of the fit of various distributions is summarized as follows.

D-Index=1Xm*i=1nΣAbs(Xi-xi')

Where Xi and Xi’ are the ith highest observed and computed values for the distribution respectively.

Table 2-7Summary table of D-index test of goodness of fit test,

| Rank | Top 6 data | Normal | Log Normal | Log Pearson Type III | Gumbel EVI |
| --- | --- | --- | --- | --- | --- |
| M | Xi | Xi-X' | Xi-X' | Xi-X' | Xi-X' |
| 1 | 100.00 | 13.24 | 12.17 | 5.58 | 1.11 |
| 2 | 98.00 | 18.83 | 20.82 | 19.12 | 11.15 |
| 3 | 87.80 | 13.73 | 17.05 | 17.35 | 9.53 |
| 4 | 69.00 | 1.03 | 2.96 | 4.29 | 3.01 |
| 5 | 57.00 | 9.57 | 5.26 | 3.35 | 10.00 |
| 6 | 53.20 | 10.28 | 5.87 | 3.64 | 9.57 |
| Sum |  | 66.68 | 64.13 | 53.33 | 44.38 |
| D-Index |  | 1.29 | 1.24 | 1.03 | 0.86 |
| remark |  | 4th | 3rd | 2nd | 1st |

Based on D-index method of goodness of fit test a distribution with minimum value of D-indexcan be regarded as the best fit probability distribution. The above summary table of D-index test shows that the Gumbel-EVI and Log Pearson type-III distributions are found the best fit in the order of 1st and 2nd rank among the four common probability distribution methods used for daily maximum rainfall for frequency analysis.

Finally, the Gumbel EV-I distribution is found the best fit distribution for the design rainfall frequency analysis because it is 1st rank by D-index test and 2nd rank by Probability plot method. Therefore, the 50 year return period peak design rain fall magnitude estimated by the frequency analysis using the best fitting Gumbel EV-I distribution is 118.616 mm.

### Peak Flood Discharge Analysis

The peak design flood discharge over 50 year return period can be analyzed by using the United States Soil Conservation Service (SCS) unit hydrograph method. This method is widely adopted and more reliable method for peak flood estimation of ungagged rivers like Tena River. The computation is done using design rainfall or storm estimated earlier. The approach considers Curve number and watershed parameters like Area,slope of terrain, slope of stream(S), and time of concentration.

#### Time of Concentration

The estimation of two important watershed hydrograph parameters are time of concentration(Tc) and time of lag (TL). Time lag (TL) is the time from center of excess rainfall to time of peak of hydrograph and it is usually 0.6 times the time of concentration (Tc).Time of concentration (Tc)is one of most important catchment characteristics required for flood hydrograph development. It is the time elapsed for runoff to travel from the hydraulically most distant point of the catchment area to a point of interest within the catchment area. Time of concentration (Tc) magnitude is primarily determined by the catchment parameters. The time of concentration (Tc) value can be estimated by applying different empirical equations such as Kirptich formula and Bransby-William formula.

In our case the time of concentration (Tc) of Tena watershed is estimated by using BransbyWilliam formula and Kirptich formula.

The time of concentration, Tc (hr) using Bransby-William formula;

=0.243*22.266*92.09-0.1*0.063-0.2 = 6.06 hr

Where: L is the length of the longest watercourse (km), A is the catchment area (km2), S is the stream average slope (m/m),

Kirptich formula can be computed for segmented watershed and summing all the travel times for consecutive components of the drainage conveyance system. The Kirptich time of concentration, Tc(hr)formula for segmented watershed is given below:

Where: L is the length of segmented stream (km), S is the watershed slope (m/m).

Table 2-8 Summary of Tc computation byKirptich formula

| No | Length (km) | Elevation (m) |  |  | Slope (%) |  |  | TC (Hr) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Z_Min | Z_Max | Difference | Min_Slope | Max_Slope | Avg_Slope | Kirptich |
| 1 | 4.55 | 2608.21 | 2968.05 | 359.84 | 0.000 | 54.494 | 14.627 | 0.458 |
| 2 | 4.55 | 2099.31 | 2608.21 | 508.90 | 0.000 | 23.247 | 6.866 | 0.613 |
| 3 | 4.55 | 1973.30 | 2099.31 | 126.01 | 0.000 | 14.557 | 3.807 | 0.770 |
| 4 | 4.55 | 1880.55 | 1973.30 | 92.74 | 0.000 | 12.571 | 3.345 | 0.809 |
| 5 | 4.55 | 1802.59 | 1880.55 | 77.96 | 0.000 | 10.164 | 2.847 | 0.861 |
| Sum | 22.766 | 1802.59 | 2968.05 | 1165.46 | 0.000 | 54.494 | 6.298 | 3.511 |

As depicted in the above Kirptich Tc computation table, the time of concentration is estimated 3.511 hr for Tena watershed.

Both Bransby-William and Kirptich methods give us different time of concentration (Tc)value with magnitude of 6.06 hr and 3.51 hr respectively. Generally, the Kirpich formula tends to give lower values compared with the Bransby-William which uses the catchment area as additional parameter. Application of Kirpich formula for determining Tc results in too short of a time of concentration particularly when the average basin slope varies significantly from the mean canal slope as in steep topography in areas in the upper catchments. Therefore, the design time of concentration Tc can be taken as the mean of the two estimates. So the design time of concentration (Tc) for Tena watershed is estimated 4.79 hr by averaging the above two methods.

Table 2-9Summary for catchment hydrograph Parameters

| Step | Design Parameter | Value |
| --- | --- | --- |
| 1 | Catchment Area, A (km2) | 92.09 |
| 2 | Length of main river, L (km) | 22.766 |
| 3 | Average slope of main stream, S % | 6.298 |
| 4 | Time of concentration, Tc (hr) | 4.79 |
| 5 | Duration of Rain fall excess, D (hr) | 1.0 |
| 7 | Time to peak, Tp (hr)= 0.6Tc+0.5D | 3.37 |
| 8 | Time to base, Tb (hr)= 2.67Tp | 9.00 |
| 9 | Unit discharge , q (m3/s/mm) = (0.21*A)/Tp | 5.734 |
| 10 | Lag time, TL (hr) = 0.6Tc | 2.87 |

These catchment hydrograph Parameters are important to synthesize the design flood hydrograph.

#### Curve Number (CN)

Curve number (CN) is achieved based on USSCS method by watershed characterization in terms of land cover, treatment, hydrologic condition and soil group.

These runoff factors or runoff curve numbers (CN), indicate the runoff potential of an area. The CN value depending on given hydrologic soil type, land use cover, terrain slope class and anticipated hydrologic condition of watershed. We read and assign CN-value from standard curve number table of Ethiopian Road Authority, Hydrology and Drainage Design manual; 2002. We have developed the weighted CN- value for each land use class of given watershed.

Accordingly the Tena watershed CN values are estimated for the two antecedent moisture conditions (AMC):

- CN II = 80.93 for AMC-II;

- CN III = 90.86 for AMC-III;

The design flood hydrograph should be computed by using CN-III (90.86) since the peak runoff usually occurs at an antecedent moisture condition III state. For detail refer watershed deport of same project.

#### Areal Rainfall and Rainfall Profile

Areal rainfall of watershed is needed since the point rain fall that is obtained from metrology station cannot represent the whole runoff contributing catchment area. As the area of the catchment gets larger, coincidence of all hydrological incidences become less and less. The watershed aerial rainfall can be determined by distributing the design point rainfall tothe whole catchment using area reduction factor (ARF). For small watershed area (<25Km2), simple empirical formula, Area reduction factor (ARF=1-0.044*A0.275) can be used. In case of our watershed since area (92.09Km2) which is greater than 25km2, we have to read area reduction percentage from standard curve developed for Ethiopian case by Continental consultants.

Rainfall profile is the distribution of the proportion of design rainfall during every incremental time on the watershed area during the 24 hours duration. Well-developed models are needed to determine such an event for the selected basin area. But there are no sufficient modeling studies in the vicinity and we adapt the standard rainfall profile curves developed for Ethiopian case by Continental consultants in 2002. With the aid of rainfall profile versus duration curve the percentages of design rainfall distribution on the catchment area are computed for the first most intensive storm duration(Continental, CES, SSIP handbook, 1997).

Table 2-10 Design Rainfall Arrangement

| Duration | Design point Rainfall | Rain Fall Profile |  | Area to Point Rainfall Ratio | Areal Rainfall | Incremental Rainfall | Descending Order |
| --- | --- | --- | --- | --- | --- | --- | --- |
| hr | mm | % | mm | % | mm | mm | No |
| 0.0-1.0 | 118.62 | 33.33 | 39.54 | 54.95 | 21.73 | 21.73 | 1 |
| 1.0-2.0 | 118.62 | 52.31 | 62.05 | 69.63 | 43.21 | 21.48 | 2 |
| 2.0-3.0 | 118.62 | 62.36 | 73.97 | 75.97 | 56.20 | 12.99 | 3 |
| 3.0-4.0 | 118.62 | 69.38 | 82.30 | 79.47 | 65.41 | 9.21 | 4 |
| 4.0-5.0 | 118.62 | 73.90 | 87.66 | 81.63 | 71.56 | 6.15 | 5 |
| 5.0-6.0 | 118.62 | 77.35 | 91.75 | 82.82 | 75.98 | 4.43 | 6 |

#### Direct Runoff Analysis Using SCS-CN Method

The input for direct runoff analysis using SCS-CN method are the rearranged incremental rainfall, Curve Number (CN) and watershed hydrograph parameters like time of concentration (Tc ), duration of rain fall excess (D), time to peak (Tp), time to base (Tb), lag time (TL), catchment area (A) and others.

The rearranged incremental rainfall can be determined by distributing thedesign point rainfall (118.62 mm) with rainfall profile and areal reduction factor as computed in table 2.10.We have selected the (CN-III) for the wet antecedent moisture condition as the worst design scenario for peak flood discharge generation. We have prepared input variables like curve number (CN-III = 90.86) and catchment area (A = 92.09Km2) for direct runoff computation. The maximum potential retention, S (mm) between rainfall beginning and runoff starting of storm can be computed as follows.

S=25400CN-254 = 25.55 mm

Then the effective rain fall that is becoming direct runoff depth, Pe (mm) can be computed as:

Pe=(P-0.2*S)2(P+0.8*S)

Where, (P) is rearranged rain fall excess cumulative (mm) and (S) is potential retention (mm).

Then the peak unit discharge, Qp (m3/s/mm)which created by 1mm runoff excess on whole of the catchment can be computed as follows:

Qp=0.21ATp = 5.734 m3/s/mm

Where; (A) is catchment area in (Km2), (Tp) is time to peak in (hr), (Pe) is effective rain fall or direct runoff depth in (mm).The hyetograph of rearranged incremental rainfall is presented in the next figure.

Figure 2-3 hyetograph of design rainfall

Table 2-11 Runoff analysis

| Duration, hr | Rearranged Incremental Rainfall | Cumulative Rainfall, P, mm | Cumulative Runoff, Pe, mm | Incremental runoff, Pe, mm | Peak Runoff, Q, m3/s | Time of beginning, hr | Time to peak, hr | Time to end, hr |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0.0 - 1.0 | 4.43 | 4.43 | 0.00 | 0.00 | 0.00 | 0.00 | 3.37 | 9.00 |
| 1.0 -2.0 | 9.21 | 13.63 | 2.13 | 2.13 | 12.23 | 1.00 | 4.37 | 10.00 |
| 2.0 - 3.0 | 21.48 | 35.12 | 16.21 | 14.07 | 80.71 | 2.00 | 5.37 | 11.00 |
| 3.0 - 4.0 | 21.73 | 56.84 | 34.63 | 18.42 | 105.64 | 3.00 | 6.37 | 12.00 |
| 4.0 - 5.0 | 12.99 | 69.83 | 46.40 | 11.77 | 67.52 | 4.00 | 7.37 | 13.00 |
| 5.0 - 6.0 | 6.15 | 75.98 | 52.09 | 5.69 | 32.62 | 5.00 | 8.37 | 14.00 |

Table 2-12 Synthesis of Complex Hydrograph

| Time,(hr) | Ordinate of Hydrograph (m3/s) |  |  |  |  |  |  | Total |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | H1 | H2 | H3 | H4 | H5 | H6 | Q base | Qt |
| 0.00 | 0 |  |  |  |  |  | 0.502 | 0.50 |
| 1.00 | 0.00 | 0.00 |  |  |  |  | 0.502 | 0.50 |
| 2.00 | 0.00 | 3.63 | 0 |  |  |  | 0.502 | 4.13 |
| 3.00 | 0.00 | 7.25 | 23.93 | 0 |  |  | 0.502 | 31.68 |
| 3.37 | 0.00 | 8.60 | 32.84 | 11.67 |  |  | 0.502 | 53.62 |
| 4.00 | 0.00 | 10.88 | 47.86 | 31.32 | 0 |  | 0.502 | 90.56 |
| 4.37 | 0.00 | 12.23 | 56.78 | 42.99 | 7.46 |  | 0.502 | 119.95 |
| 4.58 | 0.00 | 11.77 | 61.77 | 49.54 | 11.64 |  | 0.502 | 135.23 |
| 5.00 | 0.00 | 10.86 | 71.79 | 62.65 | 20.02 | 0 | 0.502 | 165.83 |
| 5.37 | 0.00 | 10.05 | 80.71 | 74.32 | 27.48 | 3.60 | 0.502 | 196.66 |
| 6.37 | 0.00 | 7.88 | 66.38 | 105.64 | 47.50 | 13.27 | 0.502 | 241.18 |
| 7.37 | 0.00 | 5.71 | 52.05 | 86.88 | 67.52 | 22.95 | 0.502 | 235.61 |
| 8.37 | 0.00 | 3.54 | 37.72 | 68.13 | 55.53 | 32.62 | 0.502 | 198.04 |
| 9.00 | 0 | 2.17 | 28.66 | 56.27 | 47.95 | 28.96 | 0.502 | 164.52 |
| 10.00 |  | 0.00 | 14.33 | 37.51 | 35.97 | 23.17 | 0.502 | 111.48 |
| 11.00 |  |  | 0 | 18.76 | 23.98 | 17.38 | 0.502 | 60.61 |
| 12.00 |  |  |  | 0 | 11.99 | 11.58 | 0.502 | 24.07 |
| 13.00 |  |  |  |  | 0 | 5.79 | 0.502 | 6.29 |
| 14.00 |  |  |  |  |  | 0 | 0.502 | 0.50 |

The Tena watershed synthetic complex hydrograph looks like as follows.

Figure 2-4Complex synthetic triangular hydrograph

The complex synthetic triangular hydrograph analysis result shows 241.18 m3/design flood discharge occurring at 6.37 hr of peak time over the 50 years return period. This implies that the Tena watershed hydrological response in generating the peak flood discharge will be 2.619m3/s/km2. This value ensures that high rainfall of the area can generated moderate rate of runoff per km2 of the generating catchment.

This design flood discharge (241.18 m3/s) is further used for analysis and design of head work head work and appurtenant structures like wing walls, divide wall, terminal structure, under sluice and others.

## Tail Water Rating Curve Computation

The tail water rating curve or the stage discharge is important for the determination of maximum flood level before the construction of a hydraulic structure. The tail water depth is also used to compare the hydraulic jump sequent depth (Y2) for choosing efficient energy dissipating terminal structures. There is a need to determine the slope of the river at proposed diversion headwork location.  Hence, the average longitudinal river bed slope at the headwork site is determined by linear fitting using the ground surveying data and it was found to be 0.013 m/m as shown in figure shown below.

Figure 2-5  Longitudinal average river bed slope

The river canal is well defined and the river bed is covered with igneous rock outcrops significantly extended to the left and right side of river banks. The Manning`s roughness coefficient is taken from standard table based on the river nature. The river reach at the proposed headwork site has straight nature and made of rock outcrops with rough surface appearance. The banks are also defined and stable rocky. Hence, a Manning`s roughness coefficient (n = 0.035) has been adopted.

The river cross-section at headwork site was surveyed over 40m chain age and the computation of tail water rating curve is done with help of Manning’s uniform flow equation is presented in Table below.

Manning equation for flow velocity:

Where, R – is hydraulic radius (m), n- is Manning`s roughness coefficient (n = 0.035), S-is longitudinal average river bed slope (0.013).

The river water level is taken at every 0.5m intervals points and the rating curve has been computed considering average water Surface slope as the river bed slope at the head work site.

Table 2-13 Tail water rating curve computation

| Elevation, m | Flow depth, Y(m) | Wetted Area, A(m2) | Wetted perimeter, P(m) | Hydraulic Radius, m R=A/P | Flow velocity, V(m/s) | Discharge, m3/s Q=V*A |
| --- | --- | --- | --- | --- | --- | --- |
| 1800.91 | 0.00 | 0.00 | 0.00 | 0.000 | 0.000 | 0.000 |
| 1801.41 | 0.50 | 1.74 | 6.66 | 0.261 | 1.331 | 2.317 |
| 1801.91 | 1.00 | 5.87 | 10.19 | 0.576 | 2.255 | 13.239 |
| 1802.41 | 1.50 | 11.69 | 13.44 | 0.870 | 2.968 | 34.700 |
| 1802.91 | 2.00 | 18.47 | 15.03 | 1.229 | 3.737 | 69.031 |
| 1803.41 | 2.50 | 25.81 | 16.63 | 1.552 | 4.367 | 112.708 |
| 1803.91 | 3.00 | 33.63 | 17.93 | 1.876 | 4.955 | 166.620 |
| 1804.41 | 3.50 | 43.77 | 24.61 | 1.779 | 4.782 | 209.310 |
| 1804.91 | 4.00 | 55.49 | 27.11 | 2.047 | 5.252 | 291.413 |
| 1805.41 | 4.50 | 68.30 | 29.60 | 2.307 | 5.688 | 388.517 |
| 1805.91 | 5.00 | 82.43 | 33.60 | 2.453 | 5.926 | 488.449 |

Figure 2-6 Tail water rating curve and maximum flood level

From the above table and tail water rating curves, the tail water depth (Y3) equivalent to the peak flood discharge (241.18m3/s) is found to be 3.694 m (1804.601m a.ms.l). The retaining wall top level is fixed considering this level plus some free board height. Similarly the design flood discharge has a corresponding flow velocity of 4.964 m/s.

Hence, the downstream specific energy above the river bed level (1800.907 m a.s.l) is estimated as 4.950 m by summing up the tail water depth(3.694m) and the velocity head(1.256m).

# HEAD WORK  DESIGN

## Head work site Selection

The headwork site is situated at 1364626.187mN, 377540.324mE UTM-Adindan projected coordinate and river bed elevation of 1800.907m above mean sea level. At this site the river course is well defined, matured with fixed width and forms nearly a U-shaped valley.

For this project a simple river bed level weir structure has been proposed for diverting the water to the main canal. The headwork site geological surface and subsurface conditions have been investigated based on the nature of the proposed structure. At the site and surrounding area, the stream flows along moderate slope course and its bed is mainly covered with recently deposited alluvial sediments dominated with cobbles and boulders at the top and underline by slightly weathered aphenetic basalt rock. At whole portions of the stream bed, and some portion of both ends of downstream and upstream, there are outcrops of the bedrock (massive jointed basalt rock).  On the other hand, the banks of the stream at the headwork site are made up of same geologic materials; most part of the right and left bank is made up of jointed basalt rock and low to medium plastic clay soil with little silt at the top of the jointed basalt rock. The detail geologic nature of the banks, and bed of the stream along the headwork axis and immediate vicinity are described and their potential geotechnical influence on the proposed structures also discerned/detected below, with remedial measures.

The foundation area of the headwork structure is characterized by non-uniform geologic materials of the stream bed; the sediments and bedrock. The former is loose and pervious, while the bedrock is strong and impervious. It is therefore better to incorporate a positive cut-off masonry wall at the central portion of the bed that anchored to the bedrock after intercepting more than 20cm thickness bedrock layer. This will help for seepage barrier, uplift pressure comes through the rock joints and also stability conditions.

Figure 3-1 River bed at the proposed weir site

Moreover, a summarized geological X-section at the diversion site has been prepared and presented Figure below.

Figure 3-2 Geological cross section at Head work site

#### River Bed condition

At the proposed headwork site, the stream bed or course is well defined, nearly straight, and shows rough surface due to recent sediment accumulations, boulders and undulating appearance of bedrock outcrops. Along the weir axis, the bed is made up of two basically different geologic materials, as seen from surface observation. These are recently deposited alluvial coarse grained sediments, and underling bedrock.

The river bed is almost flat and straight with both bank foots, more than half of the stream bed covered with the alluvial deposits, while both ends of the bed are slightly weathered, jointed vesicular basalt rock. The bedrock is somewhat covered with alluvial deposit is not more than 10cm thickness.

#### River Banks condition

At the headwork site/axis, the right bank is characterized by steep slope, having about 3m height from stream bed. It reveals nearly vertical section within this height.  From visual observation of the natural cuts at the bank, there are three distinct geological materials forming the bank section (see plate 4.4). From top to bottom, these includes: Low Plastic Clay (CL) with some silt, highly weathered basalt rock, jointed basalt bedrock. These three units have variable thickness at the area. At the weir axis, the top clay soil has about 1m, whereas the middle highly weathered basalt rock possesses 1m thickness.

Similarly, the left bank forms relatively gentle slope topography having 3.5m height. From surface observation, the bank is made up of one basic geological unit. The unit is highly jointed basalt rock. For geotechnical purpose, there are two distinct layers observed at the bank. These are top low plastic clay with an average thickens of 1m and underlain vesicular basalt rock as seen from the immediate vicinity of upstream and downstream exposure. The slope and peak areas of the bank are covered by the low plastic clay unit, which is exposed by river cut. And the lower layer becomes boulder and massive basalt, this bedrock exposed on the surface at the bank foot .Similar rock formation exposes along the downstream and upstream part of the bank. From visual observation of the natural cuts at the bank, there are three distinct geological materials forming the bank section. From top to bottom arrangement includes:-Low Plastic Clay (CL) with little silt, highly weathered basalt rock, jointed basalt bedrock. These two units have variable thickness/depth at the area. Just at the diversion axis, the top clay soil has about 1.m, whereas the underlying weathered basalt rock overlay on the jointed bed rock observed 1m at the bank foot and stream bed at the weir axis and few meters in downstream.

The bank formation along weir axis is moderately weathered rock unit, so that during construction the top weathered rock layer and weak materials should be removed until better rock strength units are found. Therefore, the hydraulic structure can overly on the bank with stable nature and better material strength.

#### Main Canal Routes

This particular project has two main canal routes passing to the Right side and the Left sides of the weir with length of about 2.7 and 3 km air distance respectively. Within this length, the canal passes different geologic materials, which varies both surficial and to depth. The distribution of the geologic materials with respect to the canal route, and their characteristics, especially permeability, stability and workability have been investigated during the field work using both visual inspection and subsurface test pit holes.

Based on the findings of the surface and subsurface geological investigations and with reference to design chain ages and also natural existing features, the canal route ground conditions have been described in the succeeding sections.

Right Main Canal from outlet (0+00m) to 1+200m the soil horizon is impervious and easily workable and, while the bottom unit (clayey silt) are pervious and moderately workable. Wherever there is insufficient thickness of the top soil units the main canal can be designed as lined.  But from 1+200m to the end of the canal route, the main canal follows a farmland. One engineering geological units have been encountered it is high plastic clay (CH) soil maximum depth of 2m from surface. So this soil horizon ishigh plastic impervious and it could be designed as unlined as far as other engineering conditions are confirmed.

Whereas the Left Main Canal from outlet to the end is composed of variable soil horizon with underlay with tick pervious soil horizon so the main canal can be designed as lined.

#### Sources of Natural Construction Materials

Apart from the geotechnical investigation carried out for the headwork and main canal, construction material investigation namely aggregates, masonry rocks and lining material has been identified. The location and brief description of the materials is presented as follows:

- Masonry rocks

Quarry site that can be used for production of rock for masonry stone and crushed coarse aggregates has been assessed during the field work session within the vicinity of the project area at economic distance for hauling. Two possible quarry site has been identified within right side/bank of the headwork to about 150m upstream and 250m downstream along the main canal route following a continuous ridge called (goshu sefer) locality.

This source area is accessible for production and transportation. The available rock type is aphanitic basalt which is magmatic in origin and occurs in the form of angular fragments of suitable sizes.

- Aggregates

Aggregates are input materials which bound together by cement will form concrete. Generally, aggregates occupy 70 to 75% of concrete volume. Aggregates are classified as fine and coarse grained depending on their sizes. The size of fine aggregates shall not exceed 4.75mm. Aggregates of bigger size are called coarse aggregate. The type of the work to be built governs the maximum size of aggregate. For most of the common reinforced concrete works, a maximum of size of 20mm is generally suitable.

- Fine aggregates

Fine aggregates (sand), for the proposed project can be obtained from Mena River in a specific. It is about 16km away from the project site.

- Coarse aggregates

Coarse aggregate for the construction of the project can be obtained by collecting fresh basaltic river boulders and crushing it to the desired size. Boulders deposited in the riverbed can satisfy the coarse aggregate demand of the project.

- Canal Fill Materials

A borrow area for the selected canal fill soil has reddish brown color, and composed of dominantly silt with some clay. It is residual soil having low plasticity and low liquid limit. It is non expansive soil. According to the Unified Soil Classification System, the soil group lies in Low plastic SILT (MH). Such soil type possesses a maximum dry density (MDD) lies within 1.25 to 1.37gm/cc; on average 1.31gm/cc, and the associated optimum moisture content (OMC) of 33.1% to 39.5%; on average 36.3%. The construction of the fill can be then done using these parameters.

- Water

Water for construction purposes can be found from the project stream, Tena itself. The stream is perennial throughout the year that there is some amount of flow along its course. During this field time the stream flow was more than 502 lit/sec.

## Headwork Type Selection

Looking the availability of natural construction materials and considering the river features and quantified flood discharge, ogee type of weir is chosen with cyclopean concrete body. Because it is hydraulically efficient to convey high flood discharge and to roll down possible incoming boulders with smaller dimension of retaining structures.

## Hydraulic Design of Headwork Structure

### Weir Height Determination

The following major factors have been considered in determining the weir crest level:

- Maximum command area elevation,

- Deriving head of the intake structure,

- Main canal length and slope,

- Head Losses at different structures,

- Lowest river bed level at proposed weir axis,

Table 3-1 Table of weir height determination

|  | Parameters | Formula | value |
| --- | --- | --- | --- |
| 1 | Minimum River bed level, m amsl | RBL | 1800.907 |
| 2 | Water depth in the Main canal, m | D | 0.60 |
| 3 | length of main canal, m | L | 790.00 |
| 4 | Bed slope of the main canal, m | So | 1/1200 |
| 5 | Peakfield level in the command area, m amsl | Z | 1803.80 |
| 7 | Head loss in field (turn out), m | ht | 0.050 |
| 8 | Head loss across head regulator, m | hr | 0.100 |
| 10 | Tt Minor losses, m | Hm=ht+hr | 0.150 |
| 12 | Head loss along Main Canal, m | hf=So*L | 0.79 |
|  | Free board allowance, m | Fb | 0.07 |
| 13 | Weir Crest Level, m amsl | WCL=Z+Hm+hf+D+Fb | 1805.407 |
| 15 | Weir Height, m | H= WCL-RBL | 4.50 |

### Weir Dimensions

- Crest Length

Adequate crest length is required to overpass the design flood discharge, 241.18 m3/s.  to do the comparison of Lacey regime width and the actual river width was done.

Lacey’s regime width, L=4.75*Q,=4.75*241.18= 72.09 m.

But, the actual river section width of the over flow section of the river is equal to 17.0m, hence this value is adopted for the designing of the weir body.

- Bottom width

The preliminary weir dimension like top and bottom width can be first estimated by the Beligh’s formula or Elementary gravity profile formula and finally the economical and the stable weir dimension can be determined after structural stability analysis results achieve required safety margin.

Usually the Beligh’s formula and Elementary gravity profile formula can be used to estimate the preliminary weir dimension like top and bottom width of the weir body.

Input Data:

P: Height of weir (m) = 4.5

He: specific energy head (m)

: Specific weight of weir body (2.3 for cyclopean concrete)

Bottom width, 7.6 m (adopted).

The stable weir dimensions are adopted after it attains the minimum required safety margin of structural stability analysis in all possible loading conditions. Hence a 7.6 m bottom width has been adopted which can be tested for adequacy during stability analysis.

### Weir Hydraulics and Flood Level Determination

Flow over the weir crest is computed by using weir formula. It is primarily important to determine the water depth over the crest (Hd) and specific energy head over the crest (He).

Weir flow equation is used to determine total head over crest level (He).

He=QC*L23= 3.565 m

Where: C- is discharge coefficient (2.2 for ogee weir type), L-effective length of weir (16.287) m, Q – is design discharge (241.18m3/s)

Again the total energy head is sum of water depth and velocity head over weir crest level.

That is;

But the velocity head (ha) can be computed from approach velocity.

v22g=Ha=12g*QL*Hd^2

By combining the above two equations and substituting for known values we can solve for water depth over crest and velocity head.

With help of goal seek (numerical iteration method) the water depth over crest (Hd) and velocity head (Ha) values are solved to be 2.818 m and 0.747 m respectively.

Then we can take correction for discharge coefficient (Cd):

- Correction for u/s face slope, C/Cd= 1 for vertical u/s face;

- Correction for approach velocity, C/Cd= 1 for P/Hd(1.60) >1.33,

- Correction for degree of submergence, C/Cd= 1 for Hd/Y3(0.64) > 0.5,

Since all the correction factor have unit ratio, the first discharge coefficient value (Cd =2.2) is correct enough for computing weir hydraulics.

Therefore, the upstream and downstream high flood levels can be determined as follows.

- U/s HFL=river bed level + weir height +Hd= 1800.907+4.5+2.818=1808.225m amsl.

- D/s HFL=jump height level = 1806.456m amsl, since JHC > TWC,

- Afflux = U/s HFL - D/s HFL = 1808.225 –1804.601 =3.62m.

- U/s TEL= river bed level + weir height +He =1808.972m amsl.

- D/s TEL= Tail water energy level =1805.857 m amsl.

Even if the design afflux value is found greater than 1.2, we have massive bed rock at headwork site can withstand all possible scouring effects. So that no need for enlarging the weir crest length for reducing afflux value.

From the stage-discharge curve prepared in hydrology section, the high flood level before construction (i.e. D/s HFL) corresponding to the design flood is 1804.601 m amsl. Appropriate retaining walls are provided to protect flood overtopping the banks of the river.

### Ogee Shape of the Weir

The Ogee profile is designed based on napped profile for upstream and downstream face at control section in order to increase is discharging capacity.

Figure 3-3 Typical ogee profile of control section (Novak, 2007)

We have to first compute the control section weir hydraulic parameters.

- Water depth over crest, Hd= 2.818m

- Weir height, H =4.50 m

Then detail of ogee profile can be designed by using nappe profile equation shown above.

- U/s horizontal total distance,  Xc =0.282*Hd= 0.795m,

- U/s horizontal mid distance, X1=0.175*Hd= 0.493m,

- U/s vertical  distance to edge, Yc=0.124*Hd= 0.349 m,

- Major radius for u/s curve, R1=0.5*Hd= 1.409m,

- Minor radius for u/s curve, R2=0.2*Hd= 0.564 m,

- Toe curve radius,r=H/4 =1.125 m,

- Toe curve height, Z=H/8 = 0.563 m,

Then d/s ogee profile equation can be rewritten as;

Y=0.2073 X1.85

However, this profile can only extend to downstream direction until (Xo) which it make tangent to the slope of d/s face that satisfies the structural stability requirement. Hence from the structural stability analysis the slope of the downstream face is determined (Y:X=0.8431) which satisfies all structural factor of safety.

So the derivative ofnappe profile equation should be equated to downstream chute slope (m=0.8431).

- dydx=m;        implies   0.3835X0.85=0.8431

The solution gives us Xo= 2.527m. and Yo = 1.152 m

These 2.527m and 1.152m dimensions are the projected horizontal and vertical lengthfor which the ogee nappe profile extends to point of tangency.

Figure 3-4  Ogee profile of downstream face

For the details of weir profile drawing refer the headwork working drawing of the same project.

## Hydraulic Jump Calculation

As discussed in the geologic report, the river bed and banks are basalt rock formation. Both left and right side banks are rocky, a wing walls are required at U/s and D/s sides’in order to protect bank overtopping and the possible scouring effects during peak flood flow.

The length of wing walls is determined based on the length of Jump, and it is calculated as shown below.

• Weir crest length 17.00m

• Weir height (Z) = 4.50 m

• Pre-jump depth = y1

• Post -jump depth =y2

The location of Hydraulic jump can be computed by using crump method. The basic assumption for crumps formula is it neglects small energy head loss in the chute flow.  The crump method of locating hydraulic jump can be solved by either using standard table or by equating the hydraulic jump headless to the difference of U/s TEL and D/s TEL.

- Specific discharge, q=Q/Le= 241.18m3/s/ 17.00 m = 14.81 m2/s

- Critical flow depth, Yc = (q^2/g)^1/3 = 2.817 m

- HL-head loss is the difference of U/s TEL and D/s TEL

Hydraulic jump sequent depths iteration is done by neglecting small head losses in the chute:

…………..(a)

………. ……….(b)

After iterations pre jump depth (Y1) and post jump depth (Y2) are computed to be 1.195m and 5.549m respectively. The swamee’s table is used to determine the location of jump (P-level).

- Hl/Yc=1.106, n=f(1.1060)=2.099,

- Ef2=n*Yc= 2.099*2.817 = 5.911 m,

- P-level= D/s TEL-Ef2 = 1799.95m amsl (0.96m below river bed level)

But we have sound bed rock, no need of lowering downstream floor level to P- level. Instead the D/s retaining wall height are fixed by using post jump depth rather than tail water depth.

The pre jump Froud Number, F1=V1gy1=3.62

The length of hydraulic jump length (L) can be calculated as follow.

21.78 m.

In our case no need of designing downstream apron and energy dissipating structures since the river bed and banks geologic formation are good basaltic rocks.

Comparison of hydraulic jump height curve and tail water rating curve is presented below and the detail tabular analysis is attached in annex. The Jump height curve is formed by adding the conjugate depth of hydraulic jump, y2, to the jump location elevation of the apron can be determined.

Figure 3-5 Comparison of Jump height curve & Tail water rating curve

The post jump curves totally above tail water curve; this condition may possibly cause scouring effect on the week river bed materials. In our case both the river bed and banks geology formation can resist such degree of scouring flow condition.

## Structural Design of Headwork Structure

### Stability Analysis of weir

The structural stability analysis of weir is done for the high flood level and pool level loading conditions. The followings are the major forces considered in the design of the weir overflow section by which the stability analysis was based.

- Self-weight of the structure

- External water pressure

- Silt pressure

- Up lift pressure

Structural damage due to seismicity or earth quake force is considered to be negligible. The stability analysis was done for the worst and static (normal) case for expected sever different load combinations.

Material properties:

- Cyclopean concrete unit weight (rc)=23KN/m3,

- Saturated Soil unit weight, (rs)=19KN/m3,

- Water unit weight, (rw)=9.81 KN/m3,

- Friction factor (µ)=0.75 for rocky foundation,

Table 3-2 Stability analysis of weir

| Loading Condition | Force (KN/m) |  | Lever arm, m | Moment (KN.m) |  |
| --- | --- | --- | --- | --- | --- |
| loading types | Vertical | Horizontal | about toe | Positive, M | Negative, M |
| Self-weight,W1 | 79.05 | 0.00 | 7.20 | 569.39 | 0 |
| Self-weight,W2 | 164.76 | 0.00 | 2.85 | 469.95 | 0 |
| Self-weight,W3 (rectangular under ogee) | 194.60 | 0.00 | 5.54 | 1078.45 | 0 |
| Self-weight,W4 (ogee curve) | 43.44 | 0.00 | 5.87 | 255.01 | 0 |
| Water load, Ww | 57.99 | 0.00 | 6.57 | 381.07 | 0 |
| Hydrostatic Pressure , P1 | 0 | 126.80 | 2.25 | 0 | -285.31 |
| Hydrostatic Pressure,P2 | 0 | 101.25 | 1.50 | 0 | -151.88 |
| Silt Load, Ps | 0 | 35.74 | 1.50 | 0 | -53.61 |
| Tail water hydrostatic, P3 | 0 | -7.13 | 0.40 | 2.84 | 0 |
| Tail water Load vertical | 12.05 | 0.00 | 0.67 | 8.10 | 0 |
| Uplift load force, U1 | -17.25 | 0.00 | 3.80 | 0 | -65.55 |
| Uplift load force, U2 | -32.49 | 0.00 | 5.07 | 0 | -164.62 |
| Sum for HFL condition | 502.15 | 251.26 |  | 2764.81 | 712.86 |
| Sum for Pool Level condition | 405.92 | 131.59 |  | 2117.79 | 362.01 |
| Loading Condition | HFL | PL |  |  |  |
| Summation Vertical Force, ΣFv | 502.15 | 405.92 | KN |  |  |
| Summation Horizontal Force, ΣFh | 251.26 | 131.59 | KN |  |  |
| Summation moments, ΣM+ | 2764.81 | 2117.79 | KN.m |  |  |
| Summation moments, ΣM- | 712.86 | 362.01 | KN.m |  |  |
| Lever arm of Resultant Load, X`=ΣM/ΣFv | 4.09 | 4.33 | m |  |  |
| Middle two third Rule, B/6 | 1.27 | 1.27 | m |  |  |
| Eccentricity, e=X`-B/2 <B/6 | 0.29 | 0.53 | ok |  |  |
| Overturning FOS=ΣM+/ΣM- | 3.88 | 5.85 | ok |  |  |
| Sliding FOS=µΣFv/ΣFh | 1.50 | 2.31 | ok |  |  |
| Principal stress Maximum (KN/m2) | 81.01 | 75.57 |  |  |  |
| Principal stress Minimum (KN/m2) | 51.14 | 31.25 |  |  |  |

In both High flood level (HFL) and Normal Pool level(PL) loading condition, the safety of the weir against sliding, overturning, compression and tension stressfailure were checked and the safety factor results are above the required safety margins 1.5 as shown in the table.

### Impervious Apron and Cutoff Wall

As the headwork is founded on massive jointed rocky foundation condition, no need of providing apron floor at all. We have provided a nominal 1.0m reinforced concrete cutoff wall both at upstream (hell) and downstream (toe) of weir body.  This is just to key and harmonize the weir body material to the bed rock formation and fill up irregularities and openings and to ensure no water percolating through interface joint.

### Design of Retaining Walls and Divide Walls

The High flood level (HFL) and existing topographical condition at the headwork site are the most governing parameters for fixing the retaining wall heights.

The upstream and downstream retaining walls should be high enough with provision of adequate freeboard to protect flood overtopping to both left and right sides. So the height of a retaining wall can be determined by adding free board to the elevation difference of HFL and bank level.  Other dimensions like top and bottom width of all retaining walls at different section are determined to satisfy the required structural stability safety factor.

Divide wall is designed in order to create separation between outlet canal and natural river course. The divide wall allows safe and stable base flow to the canal outlet. Flow turbidity created by current flow impact over the weir body is reduced.

In our case we have provided two divide walls to the left and right canal outlets. The top level of divide wall is the same as that of U/s retaining wall and it supports the operation slab and Brest wall.

The divide wall is extended 1.0m to the U/s direction and to the toe end at D/s direction to support and safeguard under sluice and canal outlets.

In the following table presents summarized dimension of retaining walls and divide walls.

Table 3-3  Summary of retaking and divide walls dimension

|  | Design of walls | Right | Left |
| --- | --- | --- | --- |
|  | U/s River bank level, m amsl | 1803.50 | 1803.50 |
|  | D/s River bank level, m amsl | 1803.00 | 1803.00 |
|  | U/s high flood level | 1808.225 | 1808.225 |
|  | D/s high flood level | 1806.456 | 1806.456 |
|  | Free board, m | 0.5 | 0.5 |
| U/S Wing Wall | Height, m | 5.30 | 5.30 |
|  | Bottom width, m | 3.50 | 3.50 |
|  | Top width, m | 0.40 | 0.40 |
| D/S Wing Wall | Height, m | 4.00 | 4.00 |
|  | Bottom width, m | 2.60 | 2.60 |
|  | Top width, m | 0.40 | 0.40 |
| Divide Wall | Height, m | 5.30 | 5.30 |
|  | Thickness, m | 1.00 | 1.00 |

The stability analysis of both Divide wall and retaining walls at different section is checked for the minimum factor of safety and all are found adequate and safe. The detail of stability analysis is presented in annex table.

### Design of Under Sluice and Canal outlet

#### Under sluice

The under sluice is mainly provided here to remove silt deposition as a result of barrier structure. Hence, the sill level of the under sluice is fixed to facilitate this deposited silt to increase the efficiency of water abstracting to the main canal through the head regulator from the pocket.

The sill level of under sluice is usually recommended to be at 0.5 to 1.0 m depth below the canal offtake sill level. This relative depth is usually determined by the requirement of hydraulic head to scour the silt materials accumulation in the pocket of under sluice. In addition to the supply of water to the intake and the removal of silt, this acts to remove the boulder that comes to wards it.

Considering this, the opening width of under sluice is designed to be 1.0 m with spindle type operating from the operation slab.

Considering rectangular notch profile of flow of water at the under sluice, the discharge passing is computed considering the following points.

- The capacity should be at least five times the river base flow to ensure proper scouring.

- Capacity of passing about 10% to 20% of the maximum flood discharge at high floods.

- During construction, it should be able to pass the prevailing (at least base flow) discharge of the river.

But the practical application of those values is not sound so the discharging capacity of under sluice is determined by using orifice flow formula for five times base flow discharge (Q = 1.96 m3/s).

Considering depth of 1.0 m below the 0.6m depth of canal offtake level, the bed width of under sluice can be computed as follow.

Q=23Cdbh2gh,

Where, Cd = discharge coefficient =0.60

h= driving hydraulic head (1.1 m) at center of office area.

g = 9.81m/s2

Width (b) = 1.5Q Cd*h2g*h= 1.5*1.9600.6*1.12*9.81*1.5=0.959  take 1.0 m width.

Hence the size of the under sluice is 1.0m×1.6m (width X height) is provide for both left and right sides under sluices.

#### Canal outlets

The sill level of canal off take is fixed based on the optimum route alignment and the maximum irrigated command level including minor and major hydraulic losses in the conveyance system. In this project we have two; right and left side, main canals which runs 2.7km and 2.9km long routes respectively.

Each off take canal capacity is determined for maximum command area and the corresponding design discharge. In this case the outlet capacity is fixed considering maximum discharge to irrigate potential command area. As explained in chapter four, the Tena diversion irrigation system is designed for 400 ha total maximum command area (350 ha on right side, 50 ha on left side) for supplementary irrigation.

The design irrigation duty determined for 14 hr wet season supplementary irrigation is 1.23 l/s/ha. Accordingly, the design discharge of canal off takes is determined 0.431 m3/s and 0.0615 m3/s for right and left side outlets respectively.

For ease of water regulation the off take sill level 1804.807 m amsl is provided for both right and left side canal off takes.

The offtake canal dimensions are designated by using broad crested weir formula;

Q = Cbh3/2

Where;  C = coefficient of discharge = 1.7

b = Width of water way (m)

h = specific energy head approximated as water depth (m),

By substituting the design discharges (0.431 m3/s and 0.0615 m3/s) and required canal water depth (0.6m & 0.40 m) we can solve for the size of for right and left side canal off takes.

Accordingly, off take size width by height of (0.70m X 0.60m) for right side off take and (0.20m X 0.40 m) for left side off take is provided. Moreover, the gate of the off take canal is designed to be vertical sheet metal for the closure of the opening space with some extra dimensions for side groove. The grooves are to be provided on the walls using angle iron frames at the two sides of the gate openings. Trash racks of Diameter 12mm with c/c spacing of 10cm has to be provided at upstream face of the gate to prevent entry of debris to the canal. For detail refer the detail of working drawing.

#### Breast Wall and Operation Slab

A vertical raised gate is designed for the head regulator and under sluice. These gates slides over the breast wall using spindle during opening and closing. For easy operation of these gates, operation slab is provided. The size of the operation slab is fixed from the point of construction and free movement. The size of the operation slab is shown in the drawing has thickness of 0.2m.

The thickness of the breast wall is also the same as that of the operation slab. The nominal thickness is fixed from the point of construction rather than the imposed load. The thickness required for the imposed load is less than the nominal value and treated as cantilever retaining wall. For both the operation slab and breast wall, the structural analysis summary table is annexed. The actual arrangement of angle irons, spindles, shafts and operation slab including other components is shown on the design drawing.

For each arrangement and further information, refer to the design drawing.

# IRRIGATION AND DRAINAGE SYSTEMS DESIGN

In this section, planning and design of the irrigation system after diverting the water using the weir will be dealt. The following are major areas of concern in this part.

- Study and design of the irrigation method to be adopted,

- Study and design of the irrigation structures and  system layout,

- Design of the different conveyance canals,

- Planning and design of the different irrigation and drainage structures,

- Preparation of the longitudinal profiles of the different irrigation and drainage canal.

## Irrigable area description

### Topography

Topography is an important factor for the planning of any irrigation project as it influences method of irrigation, drainage, erosion, mechanization and cost of land development, labor requirement and choice of crops.

The topographic feature of the project command area is mainly sloping type. Its elevation range is from 1795 to 1810 masl. The slope gradient also ranges from gently sloping (1%) to steep sloping (3.5%). However, it has been identified to be suitable for Surface irrigation. Nevertheless, it requires soil and water conservation measures or structures (i.e. constructing bunds, bio-physicals, check dams, artificial water ways, etc).   The project command area is situated on both Right and Left side of Tena River. The Right side command size accounts more than 85% with much flat topographic feature.

### Climate

As per the hydrological analysis and on the basis of the traditional Ethiopian Agro-Ecological Zones (MOA, 2001), the area can be classified as Woina Dega (Sub-moist cool) agro-ecological zone, indicating better moisture condition in the area in wet seasons. There is no significant belg rain in the project area. Despite the fact that the Meher rains are considered adequate, there is notable variation in terms of onset, distribution and withdrawal from year to year affecting crop production in general and crop productivity in particular.

Even if the nearest metrology station is Arbaya, it has inadequate metrological data record. Instead data from the Addis Zemen station is used for Crop water demand Analysis.

### Soil characteristics

Soil properties (physical, chemical, etc.) greatly influence the growth and thereby yield of crops which is grown. Based on field observations and soil laboratory results (samples’ collected from the project command area), the irrigable area has predominantly clay textured soils which can be classified as moderately drained soil.   The effective soil depth of the command area was measured during field study and hence most of the study area soils are categorized as deep soil (1-1.9 m depth).

### Existing Irrigation Practices

The pressure of Survival and the need for additional food supplies to meet the demands of the increasing population is necessitating a rapid expansion of irrigation schemes. Thus, irrigation is becoming a basic part of well-developed agriculture wherever there is water and irrigable land potential.

Accordingly, traditional irrigation practices are under taken by individual farmers that use the river flow to the right side with laborious temporary diversions. So, the farmers in the project area are aware of irrigation and very much interested in the idea of upgrading the traditional scheme to modern scheme.

## Irrigation water requirement

### Crop Water Requirement (CWR)

The determination of crop water requirement is a very important aspect for planning of any irrigation project. Several methods and procedures are available for this. The computer program available in FAO Irrigation and Drainage Paper No. 56 “CROPWAT” has been used for the calculation of Crop Water requirement. This program is based on Penman-Monteith approach and procedures for calculation of crop water requirements and irrigation requirements are mainly based on methodologies presented in FAO Irrigation and Drainage Paper No. 24 “Crop Water Requirements” and No. 33 “Yield Response to Water”.

### Calculation of reference evapotranspiration (ETo)

Reference Evapotranspiration (ETo) represents the potential evaporation of a well-watered grass or reference crop. The water needs of other crops are directly linked to this climatic parameter. Although several methods exist to determine ETo, the Modified Penman-Monteith Method has been recommended as the appropriate combination method to determine ETo from climatic data(temperature, humidity, sunshine, and wind speed) on a monthly basis.

### Irrigation efficiency (Ep)

To complete the evaluation of the demand, the efficiency of the water distribution system and application method must be known.

The gross requirement of water for irrigation system is very much dependent on the overall efficiency of the irrigation system, which in turn is dependent on several factors: Method of irrigation, type of canal (Lined and/or Unlined), method of operations (Continuous or Rotational water supply), and availability of structures (for controlling and distribution and measuring and monitoring).

On the basis of these factors, it is planned to use Surface irrigation method (using furrows). The conveyance, distribution and field application efficiencies are estimated to be 90%, 80% and 65% respectively. As a result of these the overall irrigation efficiency is estimated to be 50%.

### Irrigation duty

Irrigation duty is the volume of water required per hectare for the full flange stage of crops. Moreover, it helps in designing an efficient irrigation canal system. The area, which will be irrigated, can be calculated by knowing the total available water at the source and the overall duty for all crops required to be irrigated in different seasons of the years.

The proposed cropping pattern of Tena Diversion irrigation project has showed all crops maximum average net irrigation water requirement (NIWR) of 4.3 mm/day for dry season crops in the month of February and 3.1mm/day for wet season in the month of September for all wet season crops. However, for the designing of the irrigation water application and flows in the entire canal system out of the proposed/all the crops that has average maximum NIWR has been taken. Therefore, the average of the wet total crop has showed the maximum NIWR which is 4.3mm/day in the month of February and taken for the irrigation project duty calculation for 18 irrigation hours and 3.1mm/day in the month of September for wet season crops (see the calculation below).

For the designing of the project, the GIWR is given as follows:

GIWR = NIWR/e = 4.3/0.50 = 8.6 [mm/day] for full irrigation

GIWR = NIWR/e = 3.1/0.50=6.2[mm/day] for supplementary irrigation

The GIWR (8.6and 6.2mm/day) represents the daily quantity of water that is required to be applied for dry and wet season crops, respectively. This water quantity is also used for the determination of the canal discharge in consideration of the time of flow and also defined as the duty, expressed as l/s/ha.

The duty is calculated by:

Duty (D) = GIWR × 1000 × 10 / (t × 60×60)

Where; Duty – the duty [l/s/ha]

GIWR – Gross Irrigation Requirement [mm/day]

t – Daily irrigation or flow hours [hrs.]

The duty for GIWR of 8.6 mm/day (dry seasons) for daily irrigation time (t = 18 hrs.) and 6.2mm/day (wet season) for daily irrigation time (14 hrs.). Thus, duty for 18 irrigation hours of dry and 14 hours wet seasons’ full and supplementary irrigation has computed as follows, respectively:

D = (8.6x10x1000) / (18x3600) = 1.33 l/s/h (for full irrigation during dry season)

D = (8.2x10x1000) / (14x3600) =1.23l/s/h (for supplementary irrigation during wet season)

### Irrigation methods

Among the various irrigation methods, Surface irrigation method has been selected.  Of the Surface irrigation methods furrow, border and basin irrigation methods can be used to supply irrigation water to the plants/crops. We carefully choice the method which is best suited to the local circumstances depending on slopes, soil types, selected crop types, amount of water available, etc. of the command area.

Furrow irrigation method is best suited for most of the proposed and row planted crops. In general, furrow irrigation method is simple, manageable and widely practiced irrigation method. This method is suitable for row crops that cannot stand in water for long periods.

## Irrigation and drainage system layout

The irrigation system layout for the project is prepared taking the following points into consideration in addition to other factors.

- A primary concern in the layout of the system is that it serves the purpose of conveying and distributing water to the command area,

- The excavation and earth fill volumes not to be excessive to minimize the costs of construction,

- The selection of longitudinal bed slope is made taking into account the existing slopes of the terrain, so as to minimize deviations in canal routing,

- Curves in canals should not be too sharp,

The proposed irrigation system layout comprises two main canal, 5 secondary canals and 39 tertiary canals as shown on the layout Drawings,

### Conveyance System

The conveyance system consists of two Main canals (Right MC & Left MC) to irrigate total net command area of 400 ha. The Right main canal starts from the Water abstraction site on the right side and conveys water for a length of 2.7 Km. similarly the Left main canal starting from the left side off take convey water about 2.9km length. The Main canals are aligned slightly across contours and distribute waters to secondary and Tertiary Canals keeping the order of arrangement.

## Design of the Irrigation canal system

Flow Depth and Section Capacity

The earthen canals have been designed with a trapezoidal shape and the lined ones with rectangular x-section using Manning's Formula:

Where;

Q= discharge (m3/s)

R= Hydraulic radius (Flow area/wetted perimeter)

S= Hydraulic gradient

n= Manning's roughness coefficient, n=0.025 is adopted for the earthencanals and

n=0.018 for the masonry lined part of the main canal.

### Main Canal

There are two (Right & Left) main canals aligned along contour to take water from the head work and to feed the secondary canals and then to tertiary and field canals arrangement. The main canals is designed for a discharge satisfying crop water demand depending on the site specific condition.

The geology report shows that the Right MC (0+0-1+200) m chain age has impervious soil horizon with small thickness while the Chain age (1+200 - 2+700m) has impervious soil horizon with 2m thickness in farm area.

The left MC total Chain age (0+0 - 2+900m) has thick pervious with easily workable soil horizon. For both main canals, we have design lined masonry canal for sack of increasing canal conveyance efficiency.

Appropriate slope is provided to ensure non-silting and non-scouring canal hydraulics. Using manning’s equation, the canal dimensions are fixed and the results are shown on each of canal profile of the working drawings.

The capacity of the canals and hydraulic parameters of the main canal are shown in Table below.

Table 4-1Hydraulic Parameters of Main canals

| Canals | Reach | QR (m3/s) | b/d | BW (m) | FSD (m) | FB (m) | Side slope (H:V) | Value n | So | V m/s | QR- Qd |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| MC-1 (Right MC) | 0-1080 | 0.43 | 1.50 | 1.05 | 0.60 | 0.20 | 0.00 | 0.018 | 0.0008 | 0.69 | 0.000 |
|  | 1080-1890 | 0.25 | 1.50 | 0.70 | 0.46 | 0.20 | 0.00 | 0.018 | 0.0017 | 0.79 | 0.000 |
|  | 1890-2740 | 0.14 | 1.50 | 0.57 | 0.38 | 0.20 | 0.00 | 0.018 | 0.0014 | 0.63 | 0.000 |
| MC-2 (Left MC) | 0-280 | 0.06 | 1.00 | 0.30 | 0.23 | 0.20 | 0.00 | 0.018 | 0.0063 | 0.89 | 0.00 |
|  | 280-615 | 0.06 | 1.00 | 0.40 | 0.40 | 0.20 | 0.00 | 0.018 | 0.0007 | 0.38 | 0.00 |
|  | 615-2960 | 0.06 | 1.00 | 0.40 | 0.40 | 0.20 | 0.00 | 0.018 | 0.0007 | 0.38 | 0.00 |

BW= Canal bottom width                     QR= Required discharge

FSD= Full supply depth                        QD= Designed Discharge

FB= Free board 			SS= Side slope

So=bed slope

V= Velocity

### Secondary Canals

Secondary canals are branching canals from main canal. These are aligned across contour to distribute water to the tertiary canals at the Division box and Turnout canal structures. Secondary canals were designed carefully to keep its hydraulic parameters non-silting and non-scouring. We have provided lined canals to overcome the scouring flow velocity effect in all secondary canals even if the geology of the right command area has impervious soil horizon with average 2m thickness.

The capacity of the canals and hydraulic parameters of the secondary canals are shown in Table below.

Table 4-2Hydraulic Parameters of Secondary Canals

| Canals | Reach | QR (m3/s) | b/d | BW (m) | FSD (m) | FB (m) | Side slope (H:V) | Value n | So | V m/s | QR- Qd |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SC-1-1 | 0-510 | 0.176 | 1.00 | 0.50 | 0.50 | 0.20 | 0.00 | 0.018 | 0.0018 | 0.71 | 0.000 |
|  | 510-930 | 0.176 | 1.00 | 0.54 | 0.54 | 0.20 | 0.00 | 0.018 | 0.0012 | 0.61 | 0.000 |
|  | 930-1211 | 0.088 | 1.00 | 0.38 | 0.38 | 0.20 | 0.00 | 0.018 | 0.0018 | 0.60 | 0.000 |
|  | 1211-1901 | 0.088 | 1.00 | 0.41 | 0.41 | 0.20 | 0.00 | 0.018 | 0.0012 | 0.51 | 0.000 |
| SC-1-2 | 0-200 | 0.18 | 1.00 | 0.30 | 0.27 | 0.20 | 0.00 | 0.018 | 0.033 | 2.14 | 0.00 |
|  | 200-880 | 0.18 | 1.00 | 0.35 | 0.35 | 0.20 | 0.00 | 0.018 | 0.013 | 1.47 | 0.00 |
| SC-1-2A | 0-260 | 0.05 | 1.00 | 0.30 | 0.20 | 0.20 | 0.00 | 0.018 | 0.005 | 0.76 | 0.00 |
|  | 260-518 | 0.05 | 1.00 | 0.30 | 0.15 | 0.20 | 0.00 | 0.018 | 0.011 | 1.04 | 0.00 |
|  | 518-957 | 0.05 | 1.00 | 0.30 | 0.12 | 0.20 | 0.00 | 0.018 | 0.020 | 1.29 | 0.00 |
| SC-1-2B | 0-260 | 0.05 | 1.00 | 0.30 | 0.20 | 0.20 | 0.00 | 0.018 | 0.005 | 0.76 | 0.00 |
|  | 260-518 | 0.05 | 1.00 | 0.30 | 0.15 | 0.20 | 0.00 | 0.018 | 0.011 | 1.04 | 0.00 |
|  | 518-957 | 0.05 | 1.00 | 0.30 | 0.12 | 0.20 | 0.00 | 0.018 | 0.020 | 1.29 | 0.00 |
| SC-1-3 | 0 - 80 | 0.078 | 1.00 | 0.30 | 0.18 | 0.20 | 0.00 | 0.018 | 0.020 | 1.47 | 0.00 |
|  | 80 - 220 | 0.078 | 1.00 | 0.37 | 0.37 | 0.20 | 0.00 | 0.018 | 0.002 | 0.56 | 0.00 |
|  | 220-690 | 0.078 | 1.00 | 0.30 | 0.17 | 0.20 | 0.00 | 0.018 | 0.022 | 1.53 | 0.00 |
|  | 690-960 | 0.078 | 1.00 | 0.30 | 0.30 | 0.20 | 0.00 | 0.018 | 0.005 | 0.85 | 0.00 |
|  | 960-1300 | 0.078 | 1.00 | 0.30 | 0.15 | 0.20 | 0.00 | 0.018 | 0.030 | 1.72 | 0.00 |
| SC-2-1 | 0 - 190 | 0.01 | 1.00 | 0.30 | 0.04 | 0.20 | 0.00 | 0.018 | 0.0200 | 0.75 | 0.00 |
|  | 190 - 320 | 0.01 | 1.00 | 0.30 | 0.03 | 0.20 | 0.00 | 0.018 | 0.0286 | 0.84 | 0.00 |
| SC-2-2 | 0-200 | 0.03 | 1.00 | 0.30 | 0.09 | 0.20 | 0.00 | 0.018 | 0.0200 | 1.17 | 0.00 |
|  | 200 - 600 | 0.03 | 1.00 | 0.30 | 0.11 | 0.20 | 0.00 | 0.018 | 0.0125 | 0.99 | 0.00 |

BW= Canal bottom width                     QR= Required discharge

FSD= Full supply depth                        QD= Designed Discharge

FB= Free board 			SS= Side slope

So=bed slope

V= Velocity

### Tertiary canals

In the system layout there are total number of 39 tertiary canals. Their individual designed discharge is determined based on the irrigation duty and delineated block area. Considering the geology of the command area has impervious clay soil horizon with average 2m thickness, all tertiary canals are designed to be earthen type. The sections of the canals are determined by using Manning’s formula, and they are trapezoidal section earthen canals made of high plastic clay soil.

The details of tertiary canal hydraulic parameters results and dimensions are presented for each of canal profile working drawings.

The details of hydraulic characteristics is Presented in Table 4.2 below.

Figure 4-1 Typical Cross Section of tertiary canals

Table 4-3Hydraulic Parameters of Tertiary Canals

| Canals | Reach | QR (m3/s) | b/d | BW (m) | FSD (m) | FB (m) | Side slope (H:V) | Value n | So | V (m/s) | QR- Qd |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TC-1-1-1 | 0-250 | 0.010 | 1.0 | 0.30 | 0.09 | 0.30 | 1.0 | 0.025 | 600 | 0.27 | 0.00 |
| TC-1-1-2 | 0-328 | 0.006 | 1.0 | 0.30 | 0.05 | 0.30 | 1.0 | 0.025 | 180 | 0.34 | 0.00 |
| TC-1-1-3 | 0-337 | 0.011 | 1.0 | 0.30 | 0.09 | 0.30 | 1.0 | 0.025 | 500 | 0.29 | 0.00 |
| TC-1-1-4 | 0-360 | 0.009 | 1.0 | 0.30 | 0.08 | 0.30 | 1.0 | 0.025 | 350 | 0.31 | 0.00 |
| TC-1-1-5 | 0-340 | 0.009 | 1.0 | 0.30 | 0.09 | 0.30 | 1.0 | 0.025 | 550 | 0.27 | 0.00 |
| TC-1-1-6 | 0-338 | 0.009 | 1.0 | 0.30 | 0.08 | 0.30 | 1.0 | 0.025 | 400 | 0.29 | 0.00 |
| TC-1-1-7 | 0-314 | 0.008 | 1.0 | 0.30 | 0.07 | 0.30 | 1.0 | 0.025 | 400 | 0.28 | 0.00 |
| TC-1-1-8 | 0-340 | 0.008 | 1.0 | 0.30 | 0.06 | 0.30 | 1.0 | 0.025 | 200 | 0.36 | 0.00 |
| TC-1-1-9 | 0-330 | 0.012 | 1.0 | 0.30 | 0.10 | 0.30 | 1.0 | 0.025 | 600 | 0.28 | 0.00 |
| TC-1-1-10 | 0-335 | 0.009 | 1.0 | 0.30 | 0.08 | 0.30 | 1.0 | 0.025 | 500 | 0.27 | 0.00 |
| TC-1-1-11 | 0-632 | 0.016 | 1.0 | 0.30 | 0.13 | 0.30 | 1.0 | 0.025 | 750 | 0.28 | 0.00 |
| TC-1-1-12 | 0-290 | 0.008 | 1.0 | 0.30 | 0.08 | 0.30 | 1.0 | 0.025 | 500 | 0.26 | 0.00 |
| TC-1-1-13 | 0-364 | 0.009 | 1.0 | 0.30 | 0.09 | 0.30 | 1.0 | 0.025 | 500 | 0.28 | 0.00 |
| TC-1-1-14 | 0-260 | 0.006 | 1.0 | 0.30 | 0.07 | 0.30 | 1.0 | 0.025 | 400 | 0.27 | 0.00 |
| TC-1-1-15 | 0-320 | 0.010 | 1.0 | 0.30 | 0.09 | 0.30 | 1.0 | 0.025 | 500 | 0.28 | 0.00 |
| TC-1-1-16 | 0-260 | 0.007 | 1.0 | 0.30 | 0.07 | 0.30 | 1.0 | 0.025 | 450 | 0.26 | 0.00 |
| TC-1-1-17 | 0-315 | 0.007 | 1.0 | 0.30 | 0.08 | 0.30 | 1.0 | 0.025 | 500 | 0.26 | 0.00 |
| TC-1-1-18 | 0-252 | 0.006 | 1.0 | 0.30 | 0.06 | 0.30 | 1.0 | 0.025 | 300 | 0.29 | 0.00 |
| TC-1-1-19 | 0-273 | 0.005 | 1.0 | 0.30 | 0.05 | 0.30 | 1.0 | 0.025 | 350 | 0.26 | 0.00 |
| TC-1-1-20 | 0-258 | 0.012 | 1.0 | 0.30 | 0.11 | 0.30 | 1.0 | 0.025 | 600 | 0.28 | 0.00 |
| TC-1-2-1 | 0-350 | 0.010 | 1.0 | 0.30 | 0.09 | 0.30 | 1.0 | 0.025 | 500 | 0.29 | 0.00 |
| TC-1-2-2 | 0-310 | 0.008 | 1.0 | 0.30 | 0.08 | 0.30 | 1.0 | 0.025 | 500 | 0.27 | 0.00 |
| TC-1-2-3 | 0-310 | 0.013 | 1.0 | 0.30 | 0.11 | 0.30 | 1.0 | 0.025 | 500 | 0.31 | 0.00 |
| TC-1-2-4 | 0-370 | 0.012 | 1.0 | 0.30 | 0.11 | 0.30 | 1.0 | 0.025 | 650 | 0.27 | 0.00 |
| TC-1-2-5 | 0-370 | 0.012 | 1.0 | 0.30 | 0.09 | 0.30 | 1.0 | 0.025 | 300 | 0.36 | 0.00 |
| TC-1-2-6 | 0-418 | 0.011 | 1.0 | 0.30 | 0.10 | 0.30 | 1.0 | 0.025 | 500 | 0.29 | 0.00 |
| TC-1-2-7 | 0-330 | 0.011 | 1.0 | 0.30 | 0.09 | 0.30 | 1.0 | 0.025 | 400 | 0.32 | 0.00 |
| TC-1-2-8 | 0-375 | 0.009 | 1.0 | 0.30 | 0.09 | 0.30 | 1.0 | 0.025 | 550 | 0.27 | 0.00 |
| TC-1-2-9 | 0-330 | 0.005 | 1.0 | 0.30 | 0.05 | 0.30 | 1.0 | 0.025 | 300 | 0.27 | 0.00 |
| TC-1-2-10 | 0-335 | 0.005 | 1.0 | 0.30 | 0.05 | 0.30 | 1.0 | 0.025 | 300 | 0.27 | 0.00 |
| TC-1-3-1 | 0-310 | 0.007 | 1.0 | 0.30 | 0.07 | 0.30 | 1.0 | 0.025 | 400 | 0.28 | 0.00 |
| TC-1-3-2 | 0-370 | 0.011 | 1.0 | 0.30 | 0.09 | 0.30 | 1.0 | 0.025 | 400 | 0.32 | 0.00 |
| TC-1-3-3 | 0-370 | 0.010 | 1.0 | 0.30 | 0.09 | 0.30 | 1.0 | 0.025 | 450 | 0.29 | 0.00 |
| TC-1-3-4 | 0-418 | 0.017 | 1.0 | 0.30 | 0.12 | 0.30 | 1.0 | 0.025 | 550 | 0.32 | 0.00 |
| TC-1-3-5 | 0-419 | 0.013 | 1.0 | 0.30 | 0.10 | 0.30 | 1.0 | 0.025 | 500 | 0.30 | 0.00 |
| TC-1-3-6 | 0-420 | 0.020 | 1.0 | 0.30 | 0.10 | 0.30 | 1.0 | 0.025 | 200 | 0.48 | 0.00 |
| TC-2-2-1 | 0-350 | 0.008 | 1.0 | 0.30 | 0.07 | 0.30 | 1.0 | 0.025 | 300 | 0.32 | 0.00 |
| TC-2-2-2 | 0-310 | 0.010 | 1.0 | 0.30 | 0.08 | 0.30 | 1.0 | 0.025 | 400 | 0.31 | 0.00 |
| TC-2-2-3 | 0-310 | 0.013 | 1.0 | 0.30 | 0.11 | 0.30 | 1.0 | 0.025 | 500 | 0.31 | 0.00 |

BW= Canal bottom width                     QR= Required discharge

FSD= Full supply depth                        QD= Designed Discharge

FB= Free board 			SS= Side slope

So=bed slope

V= Velocity

### Field Canals

As shown in the layout, field canals run across the contours and hence face relatively steep gradient. The discharge of most of the field canals is very small and this is taken as an advantage to cope up with the relatively steep gradient. Figure below shows a typical field canal x-section. As much as possible. Field canals shall be made in Partial cut and fill in order to easily irrigate the adjacent command area. As can be seen from the layout, the majority of the filed canals can be used to irrigate both sides of the command area depending on the condition of the individual plots of land owned by individual farmers.

Figure 4-2Typical Field Canal X-section

## Canal structures design

Canals structures in irrigation systems are necessary for water distribution, crossings, canal slope regulation etc. In this irrigation scheme drop structures, off take structures (turnouts) from main, secondary and tertiary canals to field canal.

### Division boxes

Turnouts are used to divert water from a supply canal to a smaller canals. In this particular case, the turnouts are provided to divert water from main, secondary and tertiary canals to field canal. The structure will usually consist of an inlet with stop logs grooves to control flow, a stilling basin and, where required, an outlet transition. Gates are generally used in the inlet and outlet to control the flow. Turn out locations are shown on canal profiles.

At different points of the main and secondary canals division boxes are provided which divert the flow to the secondary and tertiary canals. Gate should be provided at the outlet of the boxes. For detail refer the drawing.

Figure 4-3Typical Division Box plan

Using broad crested weir formula,

Q= CL (h)3/2

Where; Q= discharge over rectangular weir/intake (opening), m3/s

C = discharge coefficient, c= 1.7

L= effective length of crest form in m

h= over flow depth, m

Assuming equal discharge coefficient & sill height for two or three dividing canals, the proportion becomes.

Q1/ Q2= Q2/ Q3 =L1/ L2 = L2/ L3

Where Q1= is flow in canal 1

Q2= is flow in canal 2

Q3 = is flow in canal 3

L1= is effective crest length of weir/intake sill across opening to canal 1

L2= is effective crest length of weir/intake sill across opening to canal 2

L3 = is effective crest length of weir/intake sill across opening to canal 3

Q1= CL1 (h)3/2,

L1 = Q1/Ch3/2

L2 = L1*Q2/ Q1

L3 = L1*Q3/ Q1

The depth of (height of) the division box,

D = d + fb

The width of the division box,

B = b + 2*m*D

Where b= base width of the incoming canal

D = total canal depth of the incoming canal

Table 4-4: Hydraulic parameters of Division Boxes

| Dividing canal Name | Chain ages | Qo(m3/s) | Q1(m3/s) | Q2(m3/s) | Q3(m3/s) | Bo(m) | Do(m) | B1(m) | D1(m) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SC-1-1<->TC - 1 & 2 | 0+3 | 0.1763 | 0.1763 | 0.0057 | 0.0100 | 0.50 | 0.500 | 0.50 | 0.50 |
| SC-1-1 <-> TC - 3 & 4 | 0+190 | 0.1763 | 0.1763 | 0.0094 | 0.0109 | 0.50 | 0.500 | 0.50 | 0.50 |
| SC-1-1 <-> TC - 5 & 6 | 0+380 | 0.1763 | 0.1763 | 0.0085 | 0.0089 | 0.55 | 0.540 | 0.55 | 0.54 |
| SC-1-1 <-> TC - 7 & 8 | 0+600 | 0.1763 | 0.1763 | 0.0078 | 0.0075 | 0.55 | 0.540 | 0.55 | 0.54 |
| SC-1-1 <-> TC - 9 & 10 | 0+820 | 0.1763 | 0.1763 | 0.0087 | 0.0117 | 0.55 | 0.540 | 0.55 | 0.54 |
| SC-1-1 <-> TC - 11 & 12 | 0+1020 | 0.0881 | 0.0881 | 0.0076 | 0.0161 | 0.40 | 0.380 | 0.40 | 0.38 |
| SC-1-1 <-> TC - 13 & 14 | 0+1220 | 0.0881 | 0.0881 | 0.0065 | 0.0091 | 0.45 | 0.410 | 0.45 | 0.41 |
| SC-1-1 <-> TC - 15 & 16 | 0+1430 | 0.0881 | 0.0881 | 0.0069 | 0.0098 | 0.45 | 0.410 | 0.45 | 0.41 |
| SC-1-1 <-> TC - 17 & 18 | 0+1620 | 0.0881 | 0.0881 | 0.0061 | 0.0073 | 0.45 | 0.410 | 0.45 | 0.41 |
| SC-1-1 <-> TC - 19 & 20 | 0+1830 | 0.0881 | 0.0881 | 0.0123 | 0.0055 | 0.45 | 0.410 | 0.45 | 0.41 |
| SC-1-2<-> TC - 1 & 2 | 0+10 | 0.1764 | 0.1764 | 0.0080 | 0.0101 | 0.30 | 0.270 | 0.30 | 0.27 |
| SC-1-2 <-> TC - 3 & 4 | 0+160 | 0.1764 | 0.1764 | 0.0121 | 0.0134 | 0.30 | 0.270 | 0.30 | 0.27 |
| SC-1-2 <-> TC - 5 & 6 | 0+460 | 0.1764 | 0.1764 | 0.0112 | 0.0118 | 0.35 | 0.350 | 0.35 | 0.35 |
| SC-1-2 <-> TC - 7 & 8 | 0+720 | 0.1764 | 0.1764 | 0.0093 | 0.0112 | 0.35 | 0.350 | 0.35 | 0.35 |
| SC-1-2 <-> TC - 9 & 10 | 0+880 | 0.1764 | 0.1764 | 0.0049 | 0.0049 | 0.35 | 0.350 | 0.35 | 0.35 |
| SC-1-2A <-> 1 | 0+260 | 0.0460 | 0.0460 | 0.0058 | 0.0058 | 0.30 | 0.200 | 0.30 | 0.20 |
| SC-1-2A <-> 2 | 0+460 | 0.0460 | 0.0460 | 0.0058 | 0.0058 | 0.30 | 0.200 | 0.30 | 0.20 |
| SC-1-2A <-> 3 | 0+660 | 0.0460 | 0.0460 | 0.0058 | 0.0058 | 0.30 | 0.200 | 0.30 | 0.20 |
| SC-1-2A <-> 4 | 0+960 | 0.0460 | 0.0460 | 0.0058 | 0.0058 | 0.30 | 0.200 | 0.30 | 0.20 |
| SC-1-2B <-> 1 | 0+260 | 0.0451 | 0.0451 | 0.0056 | 0.0056 | 0.30 | 0.200 | 0.30 | 0.20 |
| SC-1-2B <-> 2 | 0+480 | 0.0451 | 0.0451 | 0.0056 | 0.0056 | 0.30 | 0.200 | 0.30 | 0.20 |
| SC-1-2B <-> 3 | 0+680 | 0.0451 | 0.0451 | 0.0056 | 0.0056 | 0.30 | 0.200 | 0.30 | 0.20 |
| SC-1-2B <-> 4 | 0+1010 | 0.0451 | 0.0451 | 0.0056 | 0.0056 | 0.30 | 0.200 | 0.30 | 0.20 |
| SC-2-1<-> SC - 1 | 0+03 | 0.0081 | 0.0081 | 0.0020 | 0.0020 | 0.30 | 0.200 | 0.30 | 0.20 |
| SC-2-1<-> TC - 2 | 0+220 | 0.0081 | 0.0081 | 0.0020 | 0.0020 | 0.30 | 0.200 | 0.30 | 0.20 |
| SC-2-2<-> TC - 1 | 0+03 | 0.0327 | 0.0327 | 0.0082 | 0.0082 | 0.30 | 0.200 | 0.30 | 0.20 |
| SC-2-2<-> TC - 2 | 0+300 | 0.0081 | 0.0081 | 0.0098 | 0.0049 | 0.30 | 0.200 | 0.30 | 0.20 |
| SC-2-2<-> TC - 3 | 0+420 | 0.0081 | 0.0081 | 0.0033 | 0.0131 | 0.30 | 0.200 | 0.30 | 0.20 |

(b)

| Dividing canal Name | Chain ages | B2(m) | D2(m) | L0(m) | L1(m) | L2(m) | L3(m) | Lp2 (m) | Lp3 (m) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SC-1-1<->TC - 1 & 2 | 0+3 | 0.30 | 0.05 | 0.50 | 0.50 | 0.30 | 0.30 | 0.70 | 0.70 |
| SC-1-1 <-> TC - 3 & 4 | 0+190 | 0.30 | 0.08 | 0.50 | 0.50 | 0.30 | 0.30 | 0.70 | 0.70 |
| SC-1-1 <-> TC - 5 & 6 | 0+380 | 0.30 | 0.08 | 0.60 | 0.60 | 0.30 | 0.30 | 0.70 | 0.70 |
| SC-1-1 <-> TC - 7 & 8 | 0+600 | 0.30 | 0.06 | 0.60 | 0.60 | 0.30 | 0.30 | 0.70 | 0.70 |
| SC-1-1 <-> TC - 9 & 10 | 0+820 | 0.30 | 0.08 | 0.60 | 0.60 | 0.30 | 0.30 | 0.70 | 0.70 |
| SC-1-1 <-> TC - 11 & 12 | 0+1020 | 0.30 | 0.08 | 0.40 | 0.40 | 0.30 | 0.30 | 0.70 | 0.70 |
| SC-1-1 <-> TC - 13 & 14 | 0+1220 | 0.30 | 0.07 | 0.50 | 0.50 | 0.30 | 0.30 | 0.70 | 0.70 |
| SC-1-1 <-> TC - 15 & 16 | 0+1430 | 0.30 | 0.07 | 0.50 | 0.50 | 0.30 | 0.30 | 0.70 | 0.70 |
| SC-1-1 <-> TC - 17 & 18 | 0+1620 | 0.30 | 0.06 | 0.50 | 0.50 | 0.30 | 0.30 | 0.70 | 0.70 |
| SC-1-1 <-> TC - 19 & 20 | 0+1830 | 0.30 | 0.11 | 0.50 | 0.50 | 0.30 | 0.30 | 0.70 | 0.70 |
| SC-1-2<-> TC - 1 & 2 | 0+10 | 0.30 | 0.08 | 0.30 | 0.30 | 0.30 | 0.30 | 0.70 | 0.70 |
| SC-1-2 <-> TC - 3 & 4 | 0+160 | 0.30 | 0.11 | 0.30 | 0.30 | 0.30 | 0.30 | 0.70 | 0.70 |
| SC-1-2 <-> TC - 5 & 6 | 0+460 | 0.30 | 0.10 | 0.40 | 0.40 | 0.30 | 0.30 | 0.70 | 0.70 |
| SC-1-2 <-> TC - 7 & 8 | 0+720 | 0.30 | 0.09 | 0.40 | 0.40 | 0.30 | 0.30 | 0.70 | 0.70 |
| SC-1-2 <-> TC - 9 & 10 | 0+880 | 0.30 | 0.05 | 0.40 | 0.40 | 0.30 | 0.30 | 0.70 | 0.70 |
| SC-1-2A <-> 1 | 0+260 | 0.30 | 0.05 | 0.30 | 0.30 | 0.30 | 0.30 | 0.70 | 0.70 |
| SC-1-2A <-> 2 | 0+460 | 0.30 | 0.05 | 0.30 | 0.30 | 0.30 | 0.30 | 0.70 | 0.70 |
| SC-1-2A <-> 3 | 0+660 | 0.30 | 0.05 | 0.30 | 0.30 | 0.30 | 0.30 | 0.70 | 0.70 |
| SC-1-2A <-> 4 | 0+960 | 0.30 | 0.05 | 0.30 | 0.30 | 0.30 | 0.30 | 0.70 | 0.70 |
| SC-1-2B <-> 1 | 0+260 | 0.30 | 0.05 | 0.30 | 0.30 | 0.30 | 0.30 | 0.70 | 0.70 |
| SC-1-2B <-> 2 | 0+480 | 0.30 | 0.05 | 0.30 | 0.30 | 0.30 | 0.30 | 0.70 | 0.70 |
| SC-1-2B <-> 3 | 0+680 | 0.30 | 0.05 | 0.30 | 0.30 | 0.30 | 0.30 | 0.70 | 0.70 |
| SC-1-2B <-> 4 | 0+1010 | 0.30 | 0.05 | 0.30 | 0.30 | 0.30 | 0.30 | 0.70 | 0.70 |
| SC-2-1<-> SC - 1 | 0+03 | 0.30 | 0.05 | 0.30 | 0.30 | 0.30 | 0.30 | 0.60 | 0.60 |
| SC-2-1<-> TC - 2 | 0+220 | 0.30 | 0.05 | 0.30 | 0.30 | 0.30 | 0.10 | 0.60 | 0.60 |
| SC-2-2<-> TC - 1 | 0+03 | 0.30 | 0.07 | 0.30 | 0.30 | 0.30 | 0.30 | 0.70 | 0.70 |
| SC-2-2<-> TC - 2 | 0+300 | 0.30 | 0.08 | 0.30 | 0.30 | 0.40 | 0.20 | 0.70 | 0.70 |
| SC-2-2<-> TC - 3 | 0+420 | 0.30 | 0.04 | 0.30 | 0.30 | 0.30 | 0.50 | 0.60 | 0.70 |

In this project 28 total number of Division box structures are provided in the entire canal system. The details of division box dimensions and hydraulic parameters are presented in the working drawing.

### Turn outs

Turn outs or Off takes are used to direct water from supply canal into laterals canals; Usually this structure consists of an inlet, conduit to convey water beneath the canal bank and outlet, whereas in this situation canal breaching system off take is proposed for simply to be operated by the farmers and from economic point of view. In this project 18 total number of Turnout structures are provided in the entire canal system. The hydraulic parameters of the turnouts are shown below. Hydraulic Parameter of Turn out are similar to that of division box.

Table 4-5 Hydraulic parameters of Turn Outs

| Dividing canal Name | Chainages | Qo(m3/s) | Q1(m3/s) | Q2(m3/s) | Q3(m3/s) | Bo(m) | Do(m) | B1(m) | D1(m) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| MC-1 <-> SC - 1 | 0+1080 | 0.431 | 0.254 | 0.176 | 0.000 | 1.05 | 0.60 | 0.70 | 0.46 |
| MC-1 <-> 2` | 0+1480 | 0.254 | 0.254 | 0.006 | 0.000 | 0.70 | 0.46 | 0.70 | 0.46 |
| MC-1 <-> SC -2 | 0+1900 | 0.254 | 0.136 | 0.176 | 0.000 | 0.70 | 0.46 | 0.60 | 0.38 |
| MC-1 <-> 3` | 0+2260 | 0.136 | 0.136 | 0.008 | 0.000 | 0.60 | 0.38 | 0.60 | 0.38 |
| SC 1-3<-> TC - 1 | 0+03 | 0.136 | 0.078 | 0.007 | 0.000 | 0.60 | 0.38 | 0.30 | 0.18 |
| SC 1-3<-> TC - 2 | 0+290 | 0.078 | 0.078 | 0.011 | 0.000 | 0.30 | 0.17 | 0.30 | 0.17 |
| SC 1-3<-> TC - 3 | 0+540 | 0.078 | 0.078 | 0.010 | 0.000 | 0.30 | 0.17 | 0.30 | 0.17 |
| SC 1-3<-> TC - 4 | 0+740 | 0.078 | 0.078 | 0.017 | 0.000 | 0.30 | 0.30 | 0.30 | 0.30 |
| SC 1-3<-> TC - 5 | 0+1150 | 0.078 | 0.078 | 0.013 | 0.000 | 0.30 | 0.15 | 0.30 | 0.15 |
| SC 1-3<-> TC - 6 | 0+1300 | 0.078 | 0.078 | 0.020 | 0.000 | 0.30 | 0.15 | 0.30 | 0.15 |
| MC-2 <-> SC - 1 | 0+1880 | 0.062 | 0.062 | 0.008 | 0.000 | 0.40 | 0.40 | 0.40 | 0.40 |
| MC-2 <-> SC - 2 | 0+2660 | 0.062 | 0.062 | 0.008 | 0.000 | 0.40 | 0.40 | 0.40 | 0.40 |
| MC-2 <-> FC | 0+2950 | 0.062 | 0.062 | 0.006 | 0.000 | 0.40 | 0.40 | 0.40 | 0.40 |

(b)

| Dividing canal Name | Chainages | B2(m) | D2(m) | L0(m) | L1(m) | L2(m) | L3(m) | Lp2 (m) | Lp3 (m) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| MC-1 <-> SC - 1 | 0+1080 | 0.50 | 0.05 | 1.10 | 0.70 | 0.50 | 0.50 | 1.20 | 0.00 |
| MC-1 <-> 2` | 0+1480 | 0.30 | 0.05 | 0.70 | 0.70 | 0.30 | 0.30 | 0.70 | 0.00 |
| MC-1 <-> SC -2 | 0+1900 | 0.30 | 0.27 | 0.70 | 0.60 | 0.50 | 0.50 | 1.20 | 0.00 |
| MC-1 <-> 3` | 0+2260 | 0.30 | 0.08 | 0.60 | 0.60 | 0.30 | 0.30 | 0.70 | 0.00 |
| SC 1-3<-> TC - 1 | 0+03 | 0.30 | 0.09 | 0.60 | 0.40 | 0.30 | 0.30 | 0.70 | 0.00 |
| SC 1-3<-> TC - 2 | 0+290 | 0.30 | 0.08 | 0.30 | 0.30 | 0.30 | 0.30 | 0.70 | 0.00 |
| SC 1-3<-> TC - 3 | 0+540 | 0.30 | 0.07 | 0.30 | 0.30 | 0.30 | 0.30 | 0.70 | 0.00 |
| SC 1-3<-> TC - 4 | 0+740 | 0.30 | 0.12 | 0.30 | 0.30 | 0.30 | 0.30 | 0.70 | 0.00 |
| SC 1-3<-> TC - 5 | 0+1150 | 0.30 | 0.10 | 0.30 | 0.30 | 0.30 | 0.30 | 0.70 | 0.00 |
| SC 1-3<-> TC - 6 | 0+1300 | 0.30 | 0.10 | 0.30 | 0.30 | 0.30 | 0.30 | 0.80 | 0.00 |
| MC-2 <-> SC - 1 | 0+1880 | 0.30 | 0.04 | 0.40 | 0.40 | 0.30 | 0.30 | 0.70 | 0.00 |
| MC-2 <-> SC - 2 | 0+2660 | 0.30 | 0.04 | 0.40 | 0.40 | 0.30 | 0.30 | 0.70 | 0.00 |
| MC-2 <->FC | 0+2950 | 0.30 | 0.02 | 0.40 | 0.40 | 0.30 | 0.30 | 0.70 | 0.00 |

The details of Turnouts dimension and hydraulic parameters are presented in the working drawing.

### Drop Structures

In areas where the existing ground slope exceeds the allowable canal bed slope, drop structures have been provided on the canals. The drop structures are located in relation to ground level and canal bed level so as to balance cut and fill between reaches as far as possible.  Location of the canal drop structures have been shown on the profile drawings. At some sections several consecutive drops are provided instead of designing RCC chutes over 100m length to enable us to branch required number of field canals. A drop with its height is indicated on the canal profiles drawing.

The drops are provided at places where the ground slope is steeper than the canal bed slopes. For irrigation canals vertical drop structure of U.S.B R type standard is selected to convey water from higher to lower elevation of the ground.  The design procedures are shown below:

Critical hydraulic

- Design discharge, Q (m3/s)

- Height of drop, h (m)

- Width of drop, bc =

Where d = water depth of the canal, m

- Critical discharge, q = Q/bc

- Critical depth, dc =

- Lip height, a = dc/2, a  0.15

b. Stilling basin

- Basin width, Wb =

- Basin length, Lb =

The standard drawing for all the drops to be constructed is found in the irrigation infrastructure drawings on which the above parameters are used to code each of the dimensions of the drops.

All the hydraulic parameters computed using the above procedures with the type of canals are shown in Table below.

Table 4-6  Summary of drop hydraulic parameters

| Location of the Drop | Drop, H (m) | Q (m3/s) | V (m/s) | b (m) | d (m) | He (m) | Lb(m) | Wb(m) | a(m) | U/s protection L1(m) | D/s protection L2(m) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| MC-1 @ 0 + 1055 m | 0.90 | 0.431 | 0.686 | 1.0 | 0.60 | 0.62 | 2.30 | 1.20 | 0.20 | 2.20 | 2.20 |
| MC-1 @ 0 + 1300 m | 0.60 | 0.254 | 0.786 | 0.7 | 0.46 | 0.50 | 1.70 | 1.00 | 0.20 | 2.00 | 2.00 |
| MC-1 @ 0 + 1600 m | 0.60 | 0.254 | 0.786 | 0.70 | 0.46 | 0.50 | 1.70 | 1.00 | 0.20 | 2.00 | 2.00 |
| MC-1 @ 0 + 1870 m | 0.90 | 0.254 | 0.786 | 0.70 | 0.46 | 0.50 | 2.10 | 1.00 | 0.20 | 2.00 | 2.00 |
| SC-1-1 @ 0 + 170 m | 0.60 | 0.176 | 0.711 | 0.50 | 0.50 | 0.52 | 1.70 | 0.80 | 0.20 | 1.90 | 1.90 |
| SC-1-1 @ 0 + 800 m | 0.50 | 0.176 | 0.611 | 0.54 | 0.54 | 0.56 | 1.60 | 0.80 | 0.20 | 1.90 | 1.90 |
| SC-1-1 @ 0+1360 m | 0.80 | 0.088 | 0.514 | 0.41 | 0.41 | 0.43 | 1.80 | 0.60 | 0.20 | 1.70 | 1.70 |
| SC-1-2 @ 0 + 20 m | 0.90 | 0.176 | 2.142 | 0.30 | 0.27 | 0.51 | 2.10 | 0.80 | 0.20 | 1.90 | 1.90 |
| SC-1-2 @ 0 + 80 m | 0.60 | 0.176 | 2.142 | 0.30 | 0.27 | 0.51 | 1.70 | 0.80 | 0.20 | 1.90 | 1.90 |
| SC-1-3 @ 0 + 350 m | 0.50 | 0.078 | 1.532 | 1.00 | 0.17 | 0.29 | 1.20 | 0.60 | 0.10 | 1.70 | 1.70 |
| SC-1-3 @ 0 + 630 m | 0.60 | 0.078 | 1.532 | 1.00 | 0.17 | 0.29 | 1.30 | 0.60 | 0.10 | 1.70 | 1.70 |
| MC-2 @ 0 + 480 m | 0.60 | 0.062 | 0.377 | 0.40 | 0.40 | 0.41 | 1.50 | 0.50 | 0.10 | 1.60 | 1.60 |
| MC-2 @ 0 + 1130 m | 0.80 | 0.062 | 0.377 | 0.40 | 0.40 | 0.41 | 1.80 | 0.50 | 0.20 | 1.60 | 1.60 |
| SC-2-1 @ 0 + 270 m | 0.50 | 0.008 | 0.836 | 0.30 | 0.03 | 0.07 | 0.60 | 0.20 | 0.10 | 1.40 | 1.40 |
| SC-2-2 @ 0 + 160 m | 0.50 | 0.033 | 1.170 | 0.30 | 0.09 | 0.16 | 0.90 | 0.40 | 0.10 | 1.50 | 1.50 |

### Cross-Drainage Structure

There are 9 number of major gully/river crossed by the main canals.  Out of these crossings 6 of them are flume and the other3 are supper passage structures.

The detail of flume hydraulics summary table is presented below.

Table 4-7Summary of flume structures

| Flume Name | Chainages | Length L (m) | No Span | No Pier | Bf (m) | d(m) | Fb (m) | So(H:1v) | Bn (m) | Dn (m) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| MC01_ FLum01 | 0+230 - 0+280 | 16 | 2 | 1 | 0.80 | 0.80 | 0.2 | 1200.0 | 1.05 | 0.60 |
| MC02_ Flum01 | 0+530 - 0+610 | 40 | 5 | 4 | 0.40 | 0.40 | 0.2 | 1500.0 | 0.40 | 0.40 |
| MC02_ Flum02 | 0+1340 - 0+1420 | 48 | 6 | 5 | 0.40 | 0.40 | 0.2 | 1500.0 | 0.40 | 0.40 |
| MC02_ Flum03 | 0+1520 - 0+1570 | 32 | 4 | 3 | 0.40 | 0.40 | 0.2 | 1500.0 | 0.40 | 0.40 |
| MC02_ Flum04 | 0+2080 - 0+2120 | 8 | 1 | 0 | 0.40 | 0.40 | 0.2 | 1500.0 | 0.40 | 0.40 |
| MC02_ Flum05 | 0+2260 - 0+2320 | 16 | 2 | 1 | 0.40 | 0.40 | 0.2 | 1500.0 | 0.40 | 0.40 |

Where: Bf= is flume canal internal width, d = is flume canal full supply depth, Fb=is free board  So = is flume canal bed slope, Bn= is canal internal width, Dn= is canal full supply depth.

By doing structural analysis each flumes are designed with 8m effective span length with provision of structural joint and water stop at each end. The details of flumes and supper passage dimension and structural components are well presented in the working drawing.

### Irrigation Drainage Works

The construction of appropriate drainage system is necessary for given irrigation system. In the case of this small-scale irrigation project, it has been observed that the topography and relief of the land provides adequate natural drainage systems and there is no fear of rise of ground water. Hence irrigated land drainage system for this project is not considered.  Rather main canal catch drain are provided both for right and left side main canals. Natural drains can be interrupted by main canal alignments. This can be solved by provision appropriate catch drain in order to safely drain natural flows.

The hydrologic analysis of catch drain is designed using rational method.

Q=13.6CIA

Where: Q – is discharge (m3/s), A- is contributing area (Km2) delineated using ArcGIS, C-runoff coefficient which is read 0.3 from ERA hydrology manual (2013), I- is design rainfall intensity (mm).

The 10 year return period design rainfall intensity (I) for Tena project area which is found in rainfall region (RR-A2) according to ERA hydrology and drainage manual is 74.45mm.  For design purpose the 45% of this rain fall intensity magnitude is used to compute the design discharge.

Table 4-8  Summary of Catch drain hydrology

| Catch Drain | Reach | C | I,mm | A, km | Q (m3/s) |
| --- | --- | --- | --- | --- | --- |
| MC-1 | 0-110 | 0.30 | 33.50 | 0.060 | 0.168 |
|  | 110-260 | 0.30 | 33.50 | 0.075 | 0.209 |
|  | 260-560 | 0.30 | 33.50 | 0.075 | 0.209 |
|  | 560-1000 | 0.30 | 33.50 | 0.075 | 0.209 |
|  | 1000-1900 | 0.30 | 33.50 | 0.150 | 0.419 |
|  | 1900-2050 | 0.30 | 33.50 | 0.080 | 0.223 |
|  | 2050-2740 | 0.30 | 33.50 | 0.050 | 0.140 |
| MC-2 | 0-90 | 0.30 | 33.50 | 0.070 | 0.195 |
|  | 90-320 | 0.30 | 33.50 | 0.110 | 0.307 |
|  | 320-560 | 0.30 | 33.50 | 0.060 | 0.168 |
|  | 560-950 | 0.30 | 33.50 | 0.150 | 0.419 |
|  | 950-1380 | 0.30 | 33.50 | 0.120 | 0.335 |
|  | 1380-1550 | 0.30 | 33.50 | 0.055 | 0.154 |
|  | 1550-2090 | 0.30 | 33.50 | 0.065 | 0.181 |
|  | 2090-2300 | 0.30 | 33.50 | 0.050 | 0.140 |
|  | 2300-2960 | 0.30 | 33.50 | 0.100 | 0.279 |

To make economical all the catch drain have been designed earthen canals with a trapezoidal shape using Manning's Formula. The hydraulic parameters are tabulated in the following table.

Figure 4-4Typical Cross Section of Catch drain canals

Table 4-9: Hydraulic parameters of Catch drains of Main Canals

| Catch drain | Reach | QR (m3/s) | b/d | BW (m) | FSD (m) | FB (m) | Side slope (H:V) | Manning n | So | V (m/s) | QR- Qd |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| MC-1 | 0-110 | 0.168 | 1.0 | 0.50 | 0.40 | 0.25 | 1.0 | 0.03 | 1000 | 0.46 | 0.00 |
|  | 110-260 | 0.209 | 1.0 | 0.50 | 0.45 | 0.25 | 1.0 | 0.03 | 1000 | 0.49 | 0.00 |
|  | 260-560 | 0.209 | 1.0 | 0.50 | 0.45 | 0.25 | 1.0 | 0.03 | 1000 | 0.49 | 0.00 |
|  | 560-1000 | 0.209 | 1.0 | 0.50 | 0.45 | 0.25 | 1.0 | 0.03 | 1000 | 0.49 | 0.00 |
|  | 1000-1900 | 0.419 | 1.0 | 0.54 | 0.54 | 0.25 | 1.0 | 0.03 | 580 | 0.72 | 0.00 |
|  | 1900-2050 | 0.223 | 1.0 | 0.50 | 0.42 | 0.25 | 1.0 | 0.03 | 700 | 0.57 | 0.00 |
|  | 2050-2740 | 0.140 | 1.0 | 0.50 | 0.33 | 0.25 | 1.0 | 0.03 | 700 | 0.50 | 0.00 |
| MC-2 | 0-90 | 0.195 | 1.0 | 0.50 | 0.27 | 0.25 | 1.0 | 0.03 | 160 | 0.65 | 0.00 |
|  | 90-320 | 0.307 | 1.0 | 0.50 | 0.34 | 0.25 | 1.0 | 0.03 | 160 | 0.70 | 0.00 |
|  | 320-560 | 0.168 | 1.0 | 0.50 | 0.40 | 0.25 | 1.0 | 0.03 | 1000 | 0.46 | 0.00 |
|  | 560-950 | 0.419 | 1.0 | 0.60 | 0.60 | 0.25 | 1.0 | 0.03 | 1000 | 0.58 | 0.00 |
|  | 950-1380 | 0.335 | 1.0 | 0.55 | 0.55 | 0.25 | 1.0 | 0.03 | 1000 | 0.55 | 0.00 |
|  | 1380-1550 | 0.154 | 1.0 | 0.50 | 0.38 | 0.25 | 1.0 | 0.03 | 1000 | 0.45 | 0.00 |
|  | 1550-2090 | 0.181 | 1.0 | 0.50 | 0.42 | 0.25 | 1.0 | 0.03 | 1000 | 0.47 | 0.00 |
|  | 2090-2300 | 0.140 | 1.0 | 0.50 | 0.37 | 0.25 | 1.0 | 0.03 | 1000 | 0.44 | 0.00 |
|  | 2300-2960 | 0.279 | 1.0 | 0.51 | 0.51 | 0.25 | 1.0 | 0.03 | 1000 | 0.53 | 0.00 |

The details of Catch drain Canal hydraulic parameters results and dimensions are presented for each of canal profile working drawings.

# BILL OF QUANTITY

## BOQ Summary

Table 5-1:Summary of Bill of quantity

| Bill No | Bill Deception | Unit | Quantity | Rate in birr | Amount |
| --- | --- | --- | --- | --- | --- |
| 1 | General Items | LS | 1.00 | 1,197,139.64 | 1,197,139.64 |
| 2 | Headwork | LS | 1.00 | 2,865,880.80 | 2,865,880.80 |
| 3 | Irrigation Infrastructure | LS | 1.00 | 25,962,935.11 | 25,962,935.11 |
|  | Total cost of project |  |  |  | 30,025,955.55 |
|  | VAT (15%) |  |  |  | 4,503,893.33 |
|  | Grand total cost of project |  |  |  | 34,529,848.88 |

## Bill of Quantities

Table 5-2: Bill of quantity of General item, Head work and Infrastructure

| Item No | Work Deception | Unit | Quantity | Rate in birr | Amount |
| --- | --- | --- | --- | --- | --- |
| 1 | General Items |  |  |  |  |
| 1.1 | Mobilization and Camp Activities |  |  |  |  |
| 1.1.1 | Allow for Mobilization (Machineries, material, labor, etc) | Ls | 1.00 | 269,796.77 | 269,796.77 |
| 1.1.2 | Allow for Demobilization | Ls | 1.00 | 215,837.42 | 215,837.42 |
| 1.1.3 | Construction of Consultants camp and camp facilities size 3m*4m from CGIS(28Gage) for roof & wall and internally painted chip wood wall & ceilings, with hardcore filled cement screed floor. The rooms are well ventilated equipped with windows and doors | No | 2.00 | 37,391.30 | 74,782.61 |
| 1.1.4 | Construction of contractor's camp and camp facilities size 3m*4m from CGIS(28Gage) for roof & wall and internally painted chip wood wall & ceilings, with hardcore filled cement screed floor. The rooms are well ventilated equipped with windows and doors | No | 5.00 | 37,391.30 | 186,956.52 |
| 1.1.5 | 4*5m2, cafe construction from CGIS(28Gage) for roof & wall and internally painted chip wood wall & ceilings, with hardcore filled cement screed floor. | No | 1.00 | 45,217.39 | 45,217.39 |
| 1.1.6 | 4*6m2, store construction from CGIS(28Gage) with doors and windows, Masonry floor cement screened (1 Block) | No | 1.00 | 37,391.30 | 37,391.30 |
| 1.1.7 | 3*4m2, Toilet and Shower construction from CGIS(28Gage) with window, Masonry floor cement screened | No | 1.00 | 32,608.70 | 32,608.70 |
| 1.1.8 | Construction of fence with barbed wired fixed at 20cm interval to erected eucalyptus poles which have at least 2m height and installed at 2m interval | m | 160.00 | 391.30 | 62,608.70 |
| 1.1.9 | 2*2m2, Generator house construction from CGIS(32Gage) | No | 1.00 | 6,086.96 | 6,086.96 |
| 1.1.10 | Generator rent with Fuel for Four hour per day | No | 1.00 | 72,000.00 | 72,000.00 |
| 1.1.11 | Construction of Access roads Clearing (Allow for temporary access road to site ) | km | 1.50 | 117,641.31 | 176,461.97 |
| 1.1.12 | As built drawing and site plan | Ls | 1.00 | 8,695.65 | 8,695.65 |
| 1.1.13 | Project indicator Post | No | 2.00 | 4,347.83 | 8,695.65 |
|  | Sum for general items |  |  | - | 1,197,139.6 |
| 2 | HEAD WORK STRUCTURES |  |  | - |  |
| 1 | Weir body & Divide Walls with Under Sluices |  |  | - |  |
| 1.1 | Earth work |  |  | - |  |
| 1.1.1 | Site Clearing | m2 | 239.70 | 17.39 | 4,168.70 |
| 1.1.2 | Excavation in normal soil | m3 | 33.84 | 82.61 | 2,795.48 |
| 1.1.3 | Excavation in boulder soil | m3 | 11.28 | 108.70 | 1,226.09 |
| 1.1.4 | Excavation in soft rock formation | m3 | 27.07 | 324.38 | 8,781.56 |
| 1.1.5 | Excavation in hard rock formation | m3 | 16.92 | 640.76 | 10,841.62 |
| 1.2 | Concrete & Bar works |  |  | - |  |
| 1.2.1 | Lean Concrete (C10) | m2 | 5.64 | 2,457.49 | 13,860.23 |
| 1.2.2 | Cyclopean concrete(40% C,60%Graded Stone) | m3 | 372.77 | 2,348.70 | 875,516.62 |
| 1.2.3 | Reinforced Concrete C-25 with formwork | m3 | 73.87 | 3,001.23 | 221,703.09 |
| 1.2.4 | Stone Masonry work with 1:2 cement mortar | m3 | 88.00 | 1,565.22 | 137,739.13 |
| 1.2.5 | Plastering with a 1:3 cement mortar | m2 | 264.00 | 151.30 | 39,944.35 |
| 1.2.6 | Dia. 10mm Rebar | Kg | 1926.62 | 44.35 | 85,441.45 |
| 1.2.7 | Dia. 14mm Rebar | Kg | 223.86 | 44.35 | 9,927.70 |
| 1.5 | Left side wing wall |  |  | - |  |
| 1.5.1 | Site Clearing | m2 | 283.50 | 17.39 | 4,930.43 |
| 1.5.2 | Excavation in normal soil | m3 | 340.20 | 82.61 | 28,103.48 |
| 1.5.3 | Excavation in boulder soil | m3 | 141.75 | 108.70 | 15,407.61 |
| 1.5.4 | Excavation in soft rock formation | m3 | 70.88 | 324.38 | 22,990.28 |
| 1.5.5 | Excavation in hard rock formation | m3 | 2.84 | 640.76 | 1,816.55 |
| 1.5.6 | Lean Concrete (C10) | m2 | 9.92 | 2,457.49 | 24,384.41 |
| 1.5.7 | Basaltic Stone Masonry work with 1:3 cement mortar | m3 | 346.50 | 1,565.22 | 542,347.83 |
| 1.5.8 | Plastering with a 1:3 cement mortar | m2 | 378.00 | 151.30 | 57,193.04 |
| 1.5.9 | Back fill excavated material and compaction | m3 | 70.88 | 58.26 | 4,129.24 |
| 1.6 | Right Side wing walls |  |  | - |  |
| 1.5.1 | Site Clearing | m2 | 283.50 | 17.39 | 4,930.43 |
| 1.5.2 | Excavation in normal soil | m3 | 170.10 | 82.61 | 14,051.74 |
| 1.5.3 | Excavation in boulder soil | m3 | 70.88 | 108.70 | 7,703.80 |
| 1.5.4 | Excavation in soft rock formation | m3 | 42.53 | 324.38 | 13,794.17 |
| 1.5.5 | Excavation in hard rock formation | m3 | 3.54 | 640.76 | 2,270.69 |
| 1.5.6 | Lean Concrete (C10) | m2 | 9.92 | 2,457.49 | 24,384.41 |
| 1.5.7 | Basaltic Stone Masonry work with 1:3 cement mortar | m3 | 346.50 | 1,565.22 | 542,347.83 |
| 1.5.8 | Plastering with a 1:3 cement mortar | m2 | 378.00 | 151.30 | 57,193.04 |
| 1.5.9 | Back fill selected material and compaction | m3 | 42.53 | 58.26 | 2,477.54 |
| 1.7 | Gates Supply & Installation |  |  | - | - |
| 1.7.1 | Under sluice gate as per working Drawing | pcs | 2.00 | 21,739.13 | 43,478.26 |
| 1.7.2 | Off take canal gate as per working Drawing | pcs | 2.00 | 17,391.30 | 34,782.61 |
| 1.8 | Intake Trash Rack dia. 10mm rebar as per design drawing details | Ls | 3.00 | 1,739.13 | 5,217.39 |
|  | Total |  |  | - | 2,865,880.80 |
| 3 | IRRIGATION INFRASTRUCTURE |  |  | - |  |
| 1 | Main Canals |  |  | - |  |
| 1.1 | Main Canal-1(Mc-1) Right |  |  | - |  |
| 1.1.1 | Trench excavation in normal soil | m3 | 4681.14 | 82.61 | 386,703.26 |
| 1.1.2 | Ditto but in boulder soil excavation | m3 | 23.41 | 108.70 | 2,544.10 |
| 1.1.3 | Hard core material with 0.25 m thickness | m3 | 5434.72 | 108.70 | 590,729.90 |
| 1.1.4 | Fill and compaction with selected material | m3 | 1463.76 | 58.26 | 85,280.06 |
| 1.1.5 | Masonry | m3 | 2569.41 | 1,565.22 | 4,021,686.32 |
| 1.1.6 | Plastering | m2 | 8152.07 | 151.30 | 1,233,444.04 |
| 1.2 | Main Canal-1 Retain Wall |  |  | - |  |
| 1.2.1 | Trench excavation in normal soil | m3 | 466.32 | 82.61 | 38,522.09 |
| 1.2.2 | Ditto but in boulder soil excavation | m3 | 233.16 | 108.70 | 25,343.48 |
| 1.2.3 | Fill and compaction with selected materials | m3 | 214.40 | 58.26 | 12,491.13 |
| 1.2.4 | Stone Masonry work with 1:2 cement mortar | m3 | 241.20 | 1,565.22 | 377,530.43 |
| 1.2.5 | Plastering with a 1:3 cement mortar | m2 | 268.00 | 151.30 | 40,549.57 |
| 1.2.6 | Reinforced Concrete C-25 with formwork | m3 | 192.96 | 3,001.23 | 579,116.59 |
| 1.2.7 | Lean Concrete (C10) | m2 | 402.00 | 2,457.49 | 28,042.08 |
| 1.2.8 | Re bar dia. 10 | Kg | 7809.52 | 44.35 | 346,335.23 |
| 1.3 | Main Canal-2(Mc-2) Left |  |  | - | - |
| 1.3.1 | Trench excavation in normal soil with | m3 | 4008.18 | 82.61 | 331,110.50 |
| 1.3.2 | Ditto but in boulder soil excavation | m3 | 320.65 | 108.70 | 34,853.74 |
| 1.3.3 | Hard core material with 0.25 m thickness | m2 | 4388.73 | 108.70 | 477,036.27 |
| 1.3.4 | Fill and compaction with selected material | m3 | 1205.11 | 58.26 | 70,210.81 |
| 1.3.5 | Masonry | m3 | 2301.01 | 1,565.22 | 3,601,587.33 |
| 1.3.6 | Plastering | m2 | 6583.10 | 151.30 | 996,051.73 |
| 2 | Secondary Canals |  |  | - | - |
| 2.1 | SECONDARY CANAL (SC-1-1) |  |  | - | - |
| 2.1.1 | Excavation in normal soil | m3 | 4083.82 | 82.61 | 337,359.38 |
| 2.1.2 | Fill and compaction with selected material | m3 | 942.46 | 58.26 | 54,908.69 |
| 2.1.3 | Masonry | m3 | 1718.26 | 1,565.22 | 2,689,450.96 |
| 2.1.4 | Plastering | m2 | 4893.56 | 151.30 | 740,417.31 |
| 2.2 | SECONDARY CANAL (SC-1-2) |  |  | - | - |
| 2.2.1 | Excavation in normal soil | m3 | 5888.44 | 82.61 | 486,436.19 |
| 2.2.2 | Fill and compaction with selected material | m3 | 371.52 | 58.26 | 21,645.06 |
| 2.2.3 | Masonry | m3 | 1570.80 | 1,565.22 | 2,458,650.89 |
| 2.2.4 | Plastering | m2 | 5509.12 | 151.30 | 833,553.47 |
| 2.3 | SECONDARY CANAL-(SC-1-3) |  |  | - | - |
| 2.3.1 | Excavation in normal soil | m3 | 1192.51 | 82.61 | 98,512.08 |
| 2.3.2 | Fill and compaction with selected material | m3 | 176.33 | 58.26 | 10,273.29 |
| 2.3.3 | Masonry | m3 | 677.71 | 1,565.22 | 1,060,765.52 |
| 2.3.4 | Plastering | m2 | 2259.04 | 151.30 | 341,802.22 |
| 2.4 | SECONDARY CANAL-(SC-2-1& 2) |  |  | - | - |
| 2.4.1 | Excavation in normal soil | m3 | 1104.98 | 82.61 | 91,280.62 |
| 2.4.2 | Fill and compaction with selected material | m3 | 16.32 | 58.26 | 951.07 |
| 2.4.3 | Masonry | m3 | 508.11 | 1,565.22 | 795,296.64 |
| 2.4.4 | Plastering | m2 | 1524.12 | 151.30 | 230,605.37 |
| 3 | TERTIARY CANALS |  |  | - | - |
| 3.1 | TERTIARY CANALS for MC-01 |  |  | - | - |
| 3.1.1 | Excavation in normal soil | m3 | 5319.33 | - | - |
| 3.1.5 | Fill and compaction | m3 | 485.66 | - | - |
| 3.2 | TERTIARY CANALS for MC-02 |  |  | - | - |
| 3.2.1 | Excavation in normal soil | m3 | 664.84 | - | - |
| 3.2.2 | Fill and compaction | m3 | 1.33 | - | - |
| 4 | Drop Structures |  |  | - | - |
| 4.1 | Site clearance | m2 | 124.00 | 17.39 | 2,156.52 |
| 4.2 | Normal soil excavation | m3 | 169.97 | 82.61 | 14,041.33 |
| 4.3 | Stone Masonry bedded with 1:3 cement mortar mix | m3 | 44.00 | 1,565.22 | 68,869.57 |
| 4.4 | Plastering to the external surface of masonry wall | m2 | 240.89 | 151.30 | 36,447.07 |
| 4.5 | Fill and compaction with selected material | m3 | 74.74 | 58.26 | 4,354.30 |
| 4.6 | Cemented Stone Pitching | m3 | 55.12 | 655.74 | 36,147.78 |
| 4.7 | Lean concrete C-10 for basement | m3 | 12.01 | 2,457.49 | 29,511.96 |
| 5 | Division Box Structures |  |  | - | - |
| 5.1 | Excavation | m3 | 127.91 | 82.61 | 10,566.81 |
| 5.2 | Masonry work | m3 | 25.99 | 1,565.22 | 40,675.40 |
| 5.3 | Plastering (1:3 Ratio mortar) | m2 | 63.63 | 151.30 | 9,627.25 |
| 5.4 | Fill and compaction with selected material | m3 | 55.97 | 58.26 | 3,261.12 |
| 5.5 | Cemented Stone Pitching | m3 | 24.10 | 655.74 | 15,803.02 |
| 5.6 | Supply and Installation of Gates with complete accessories |  |  | - | - |
| 5.6.1 | Gate 4mm sheet metal 70cmX50Cm with angle iron | pcs | 3 | 3,043.48 | 9,130.43 |
| 5.6.2 | Gate 4mm sheet metal 60cmX40Cm with angle iron | pcs | 2 | 3,043.48 | 6,086.96 |
| 5.6.3 | Gate 4mm sheet metal 60cmX30Cm with angle iron | pcs | 6 | 3,043.48 | 18,260.87 |
| 5.6.4 | Gate 4mm sheet metal 50cmX40Cm with angle iron | pcs | 7 | 3,043.48 | 21,304.35 |
| 5.6.5 | Gate 4mm sheet metal 50cmX30Cm with angle iron | pcs | 13 | 3,043.48 | 39,565.22 |
| 5.6.6 | Gate 4mm sheet metal 40cmX30Cm with angle iron | pcs | 10 | 3,043.48 | 30,434.78 |
| 5.6.7 | Gate 4mm sheet metal 40cmX20Cm with angle iron | pcs | 30 | 3,043.48 | 91,304.35 |
| 5.6.8 | Gate 4mm sheet metal 40cmX15Cm with angle iron | pcs | 11 | 3,043.48 | 33,478.26 |
| 5.6.9 | Gate 4mm sheet metal 20cmX15Cm with angle iron | pcs | 4 | 3,043.48 | 12,173.91 |
| 6 | TurnOut & Escaping Structures |  |  | - | - |
| 6.1 | Excavation | m3 | 66.97 | 82.61 | 5,532.34 |
| 6.2 | Masonry lining | m3 | 15.02 | 1,565.22 | 23,503.22 |
| 6.3 | Plastering (1:3 Ratio mortar) | m2 | 36.50 | 151.30 | 5,522.12 |
| 6.4 | Fill and compaction with selected material | m3 | 27.89 | 58.26 | 1,624.73 |
| 6.5 | Cemented Stone Pitching | m3 | 5.62 | 655.74 | 3,687.08 |
| 6.6 | Supply and Installation of Gates with all accessories |  |  | - | - |
| 6.6.1 | Gate 4mm sheet metal 70cmX50Cm | pcs | 3.00 | 3,043.48 | 9,130.43 |
| 6.6.2 | Gate 4mm sheet metal 70cmX40Cm | pcs | 2.00 | 3,043.48 | 6,086.96 |
| 6.6.3 | Gate 4mm sheet metal 60cmX40Cm | pcs | 2.00 | 3,043.48 | 6,086.96 |
| 6.6.4 | Gate 4mm sheet metal 50cmX40Cm | pcs | 5.00 | 3,043.48 | 15,217.39 |
| 6.6.5 | Gate 4mm sheet metal 40cmX40Cm | pcs | 6.00 | 3,043.48 | 18,260.87 |
| 6.6.6 | Gate 4mm sheet metal 40cmX30Cm | pcs | 6.00 | 3,043.48 | 18,260.87 |
| 6.6.7 | Gate 4mm sheet metal 40cmX20Cm | pcs | 4.00 | 3,043.48 | 12,173.91 |
| 7 | DRAINAGE & CROSSINGS WORKS |  |  | - | - |
| 7.1 | Flume 01 MC-01 |  |  | - | - |
| 7.1.1 | Excavation in Boulder mix soil | m3 | 12.00 | 108.70 | 1,304.35 |
| 7.1.2 | Fill and compaction with selected material | m3 | 3.90 | 58.26 | 227.22 |
| 7.1.3 | Stone Masonry with 1:3 mortar | m3 | 10.00 | 1,565.22 | 15,652.17 |
| 7.1.4 | Hard core material with 0.25 m thickness | m2 | 0.80 | 108.70 | 86.96 |
| 7.1.5 | Reinforced Concrete C-25 with formwork | m3 | 5.62 | 3,001.23 | 16,866.89 |
| 7.1.6 | Dia. 20mm Rebar | Kg | 76.88 | 44.35 | 3,409.36 |
| 7.1.8 | Dia. 10mm Rebar | Kg | 188.69 | 44.35 | 8,368.16 |
| 7.1.9 | water stop at structural joint | LS | 1.00 | 2,608.70 | 2,608.70 |
| 7.2 | Flume 02 MC-01 |  |  | - |  |
| 7.2.1 | Excavation in boulder mix soil | m3 | 18.00 | 108.70 | 1,956.52 |
| 7.2.2 | Fill and compaction with selected material | m3 | 12.60 | 58.26 | 734.09 |
| 7.2.3 | Stone Masonry with 1:3 mortar | m3 | 12.00 | 1,565.22 | 18,782.61 |
| 7.2.4 | Hard core material with 0.25 m thickness | m2 | 12.00 | 108.70 | 1,304.35 |
| 7.2.5 | Reinforced Concrete C-25 with formwork | m3 | 19.82 | 3,001.23 | 59,469.29 |
| 7.2.6 | Dia. 20mm Rebar | Kg | 568.38 | 44.35 | 25,206.28 |
| 7.2.7 | Dia. 16mm Rebar | Kg | 78.38 | 44.35 | 3,476.02 |
| 7.2.8 | Dia. 14mm Rebar | Kg | 49.31 | 44.35 | 2,186.95 |
| 7.2.9 | Dia. 10mm Rebar | Kg | 825.32 | 44.35 | 36,601.24 |
| 7.2.10 | Dia. 8mm Rebar | Kg | 15.79 | 44.35 | 700.10 |
| 7.2.11 | Soft Rebar | Kg | 20.00 | 44.35 | 886.96 |
| 7.2.12 | water stop at structural joint | LS | 1.00 | 2,608.70 | 2,608.70 |
| 7.3 | Flume 01 MC-02 |  |  | - | - |
| 7.3.1 | Excavation in Boulder mix soil | m3 | 36.00 | 108.70 | 3,913.04 |
| 7.3.2 | Fill and compaction with selected material | m3 | 12.60 | 58.26 | 734.09 |
| 7.3.3 | Stone Masonry with 1:3 mortar | m3 | 20.00 | 1,565.22 | 31,304.35 |
| 7.3.4 | Hard core material with 0.25 m thickness | m2 | 24.00 | 108.70 | 2,608.70 |
| 7.3.5 | Reinforced Concrete C-25 with formwork | m3 | 39.20 | 3,001.23 | 117,648.06 |
| 7.3.6 | Dia. 20mm Rebar | Kg | 1130.14 | 44.35 | 50,119.45 |
| 7.3.7 | Dia. 16mm Rebar | Kg | 233.01 | 44.35 | 10,333.54 |
| 7.3.8 | Dia. 14mm Rebar | Kg | 197.25 | 44.35 | 8,747.80 |
| 7.3.9 | Dia. 10mm Rebar | Kg | 1180.13 | 44.35 | 52,336.36 |
| 7.3.10 | Dia. 8mm Rebar | Kg | 82.88 | 44.35 | 3,675.55 |
| 7.3.11 | water stop at structural joint | LS | 1.00 | 2,608.70 | 2,608.70 |
| 7.4 | Flume 02 MC-02 |  |  | - | - |
| 7.4.1 | Excavation in Boulder mix soil | m3 | 42.00 | 108.70 | 4,565.22 |
| 7.4.2 | Fill and compaction with selected material | m3 | 28.80 | 58.26 | 1,677.91 |
| 7.4.3 | Stone Masonry with 1:3 mortar | m3 | 10.00 | 1,565.22 | 15,652.17 |
| 7.4.4 | Hard core material with 0.25 m thickness | m2 | 20.00 | 108.70 | 2,173.91 |
| 7.4.5 | Reinforced Concrete C-25 with formwork | m3 | 37.23 | 3,001.23 | 111,720.64 |
| 7.4.6 | Dia. 20mm Rebar | Kg | 1201.27 | 44.35 | 53,273.57 |
| 7.4.7 | Dia. 16mm Rebar | Kg | 165.96 | 44.35 | 7,359.85 |
| 7.4.8 | Dia. 14mm Rebar | Kg | 246.57 | 44.35 | 10,934.75 |
| 7.4.9 | Dia. 10mm Rebar | Kg | 998.84 | 44.35 | 44,296.37 |
| 7.4.10 | Dia. 8mm Rebar | Kg | 103.60 | 44.35 | 4,594.43 |
| 7.4.11 | water stop at structural joint | LS | 1.00 | 2,608.70 | 2,608.70 |
| 7.5 | Flume 03 MC-02 |  |  | - | - |
| 7.5.1 | Excavation in Boulder mix soil | m3 | 30.00 | 108.70 | 3,260.87 |
| 7.5.2 | Fill and compaction with selected material | m3 | 31.20 | 58.26 | 1,817.74 |
| 7.5.3 | Stone Masonry with 1:3 mortar | m3 | 10.00 | 1,565.22 | 15,652.17 |
| 7.5.4 | Hard core material with 0.25 m thickness | m2 | 12.00 | 108.70 | 1,304.35 |
| 7.5.5 | Reinforced Concrete C-25 with formwork | m3 | 24.28 | 3,001.23 | 72,854.76 |
| 7.5.6 | Dia. 20mm Rebar | Kg | 751.51 | 44.35 | 33,327.88 |
| 7.5.7 | Dia. 16mm Rebar | Kg | 99.57 | 44.35 | 4,415.91 |
| 7.5.8 | Dia. 14mm Rebar | Kg | 147.94 | 44.35 | 6,560.85 |
| 7.5.9 | Dia. 10mm Rebar | Kg | 674.78 | 44.35 | 29,925.09 |
| 7.5.10 | Dia. 8mm Rebar | Kg | 62.16 | 44.35 | 2,756.66 |
| 7.5.11 | water stop at structural joint | LS | 1.00 | 2,608.70 | 2,608.70 |
| 7.6 | Flume 04 MC-02 |  |  | - | - |
| 7.6.1 | Excavation in Boulder mix soil | m3 | 12.00 | 108.70 | 1,304.35 |
| 7.6.2 | Fill and compaction with selected material | m3 | 3.90 | 58.26 | 227.22 |
| 7.6.3 | Stone Masonry with 1:3 mortar | m3 | 10.00 | 1,565.22 | 15,652.17 |
| 7.6.4 | Hard core material with 0.25 m thickness | m2 | 0.80 | 108.70 | 86.96 |
| 7.6.5 | Reinforced Concrete C-25 with formwork | m3 | 4.10 | 3,001.23 | 12,305.03 |
| 7.6.6 | Dia. 20mm Rebar | Kg | 76.88 | 44.35 | 3,409.36 |
| 7.6.7 | Dia. 10mm Rebar | Kg | 188.69 | 44.35 | 8,368.16 |
| 7.6.8 | water stop at structural joint | LS | 1.00 | 2,608.70 | 2,608.70 |
| 7.7 | Flume 05 MC-02 |  |  | - | - |
| 7.7.1 | Excavation in Boulder mix soil | m3 | 18.00 | 108.70 | 1,956.52 |
| 7.7.2 | Fill and compaction with selected material | m3 | 1.30 | 58.26 | 75.74 |
| 7.7.3 | Stone Masonry with 1:3 mortar | m3 | 10.00 | 1,565.22 | 15,652.17 |
| 7.7.4 | Hard core material with 0.25 m thickness | m2 | 4.00 | 108.70 | 434.78 |
| 7.7.5 | Reinforced Concrete C-25 with formwork | m3 | 10.83 | 3,001.23 | 32,488.27 |
| 7.7.6 | Dia. 20mm Rebar | Kg | 301.76 | 44.35 | 13,382.20 |
| 7.7.7 | Dia. 16mm Rebar | Kg | 58.25 | 44.35 | 2,583.39 |
| 7.7.8 | Dia. 14mm Rebar | Kg | 49.31 | 44.35 | 2,186.95 |
| 7.7.9 | Dia. 10mm Rebar | Kg | 350.72 | 44.35 | 15,553.80 |
| 7.7.10 | Dia. 8mm Rebar | Kg | 20.72 | 44.35 | 918.89 |
| 7.7.11 | water stop at structural joint | LS | 1.00 | 2,608.70 | 2,608.70 |
| 7.8 | Supper passage Structures |  |  | - | - |
| 7.8.1 | Excavation in Boulder mix soil | m3 | 63.00 | 108.70 | 6,847.83 |
| 7.8.2 | Stone riprap protection works | m3 | 67.20 | 108.70 | 7,304.35 |
| 7.8.3 | Stone Masonry with 1:2 mortar | m3 | 30.24 | 1,565.22 | 47,332.17 |
| 7.8.4 | Concrete C-10 lean concrete | m2 | 4.20 | 2,457.49 | 10,321.45 |
| 7.8.5 | Concrete C-25 with formwork | m3 | 8.40 | 3,001.23 | 25,210.30 |
| 7.8.6 | Dia. 10mm Rebar | Kg | 209.98 | 44.35 | 9,311.93 |
| 7.9 | Road Crossings at 12 locations |  |  | - | - |
| 7.9.1 | Excavation in normal soil | m3 | 7.20 | 82.61 | 594.78 |
| 7.9.2 | Stone Masonry with 1:3 mortar | m3 | 8.64 | 1,565.22 | 13,523.48 |
| 7.9.3 | Concrete(C-20) RCC Slab with formwork | m3 | 2.88 | 2,706.99 | 7,796.13 |
| 7.9.4 | Dia. 10mm Rebar | Kg | 81.77 | 44.35 | 3,626.10 |
| 8 | Catch Drain works |  |  | - | - |
| 8.1 | Catch Drain for MC-01 |  |  | - | - |
| 8.1.1 | Excavation in normal soil | m3 | 1926.25 | 44.35 | 85,425.02 |
| 8.1.2 | Back Fill and compaction | m3 | 1356.49 | 58.26 | 79,030.36 |
| 8.1.3 | Concrete pipe (φ=0.5m Dia.eter) | pcs | 15.00 | 1,304.35 | 19,565.22 |
| 8.2 | Catch Drain for MC-02 |  |  | - | - |
| 8.2.1 | Excavation in normal soil | m3 | 2135.66 | 82.61 | 176,424.21 |
| 8.2.2 | Back Fill and compaction | m3 | 3695.38 | 58.26 | 215,295.96 |
| 8.2.3 | Concrete pipe (φ=0.5m Diameter) | ps | 11.00 | 1,304.35 | 14,347.83 |
|  | Sum for Irrigation Infrastructure |  |  |  | 25,962,935.11 |

# OPERATION AND MAINTENANCE

## General

The main objective of the operation and maintenance aspect of an irrigation scheme is to facilitate the timely delivery of the required irrigation water to farms and to keep the irrigation system in an optimum operating condition.  This section therefore, discusses the main functions of the subject matter under consideration for the scheme.

## Operation of the Head Works

Operation at the diversion weir mainly focuses on the diversion of a controlled flow of river water, timely cleaning of floating debris in front of intake and removal of sediment deposits in front of the weir and intake structures.

## Irrigation System Operation

The operation of the irrigation system depends mainly on the method of water delivery at farm level.  Surface irrigation method is the recommended type of water distribution and application method for Tena Scheme.

The farmers would organize themselves and form groups in order to handle the water management.  Rotational water distribution would be applied within the group.  The rotational distribution is then to distribute water by turn to the whole scheme according to the timely need of crop water requirement. For better and efficient water management, crop diversification should be avoided within a group.  This would reduce the complexity of water distribution system of the scheme during one irrigation season. At farmers’ level of operation, a constant flow and variable irrigation time is advisable. Detail of operation and maintenance will be worked out in the final report as separate document.

## Maintenance Requirement

The maintenance tasks are categorized into two types: - routine activities, and repairs.  The routine maintenance activities that are carried out periodically include:-

- Regular cleaning of sediments and weeds from canals and drains;

- Inspection and lubrication of gates; and

- Maintenance of cracked lined canals, regulating and control structures.

Repair works include task carried out more frequently and quickly, and include those task that are generally unpredictable.  They also include emergency works.  The activities included in this category are:-

- Repairing overtopped or breached canals, drains, and flood protection dykes;

- Repairing jammed gates;

- Filling holes made by wild animals; and

- Reduced free board due to walking over by people and livestock.

Regular inspection of the irrigation facilities should be carried out as part of the maintenance activities.  These tasks could be carried out immediately after the end of the main rains in September and during the rainy season.  This could concentrate on the interceptor drains and the flood protection dykes, the main canal and the field drains.  The inspection of the other works like the tertiary canals and the water control and regulating structures could be carried.

# CONCLUSION

- The infrastructure of this project area is designed to irrigate about 400 ha of land by taking its supply from the Tena diversion weir irrigation project.

- The method of irrigation of the project area is furrow, surface irrigation in which the main and tertiary canals are working continuously whereas the field canals within a tertiary block are working rotational system.

- As the dominant soil type is clay soil, the main canal system is designed to be masonry and earthen tertiary canals.

- The reason why the main canal is to be lined up to the end is to avoid the siltation problem, increase conveyance efficiency, time saving to reach at the tail part and reduce maintenance cost.

The following recommendations are drown:

- For better performance and long service year of the project regular inspection and maintenance is highly required.

- Farmers training, how to operate and maintain the project structures as a whole and available and water resources has a paramount important.

- The irrigation hours per day and per week should be flexible based on baseflow amount of each month.

As soils of the command area are predominantly clay textured; and hence water and soil management measures should be undertaken; and optimum moisture content should be maintained to improve workability of the soil during land preparation and planting time

# REFERENCES

- Arora, KR. (2002). Irrigation, water power and water resources engineering: Standard Publisher Distributors

- Baban, Rozgar. (1995). Design of diversion weirs: small scale irrigation in hot climates: John Wiley & Sons.

- Novák, Pavel, Moffat, AIB, Nalluri, Chandra, & Narayanan, R. (2007). Hydraulic structures: CRC Press.

- MOWR A.A (1997) small scale irrigation project technical hand book.

- Bedient,Philip B, (1948) ,Hydrology and flood plain analysis,

- Santosh Kumar Garg,(1976), Irrigation and hydraulic structure.

- American National Standards Institute (1998), Pump Intake Design, Hydraulic Institute, Parsippany.

# ANNEX

## Stability Analysis of Wing Walls

Material properties:

- Cyclopean concrete unit weight (rc)=23KN/m3,

- Saturated Soil unit weight, (rs)=19KN/m3,

- Water unit weight, (rw) = 9.81 KN/m3,

- Friction factor (µ)=0.75 for rocky foundation,

Table 9-1 Stability Analysis for U/s Wing Walls

| Loading Condition | Force (KN/m) |  | Lever arm, m | Moment (KN.m) |  |
| --- | --- | --- | --- | --- | --- |
| loading types | Vertical | Horizontal | About heel | Positive, M | Negative, M |
| Self-weight,W1 | 48.76 | 0.00 | 0.20 | 9.75 | 0 |
| Self-weight,W2 | 185.59 | 0.00 | 2.43 | 450.99 | 0 |
| Weight of Soil (W3) | 17.85 | 0.00 | 0.77 | 13.71 | 0 |
| Lateral load of soil back fill | 0.0 | -9.14 | 0.57 | 0 | -5.2 |
| Hydrostatic Pressure, HFL | 0.0 | 140.45 | 1.77 | -248.13 | 0 |
| Hydrostatic Pressure, PL | 0.0 | 31.25 | 2.83 | 88.54 | 0.00 |
| uplift load force | -18.26 | 0.00 | 1.15 | 0 | -20.97 |
| Sum for HFL condition | 233.94 | 131.31 |  | 226.33 | 26.15 |
| Sum for Pool Level condition | 233.94 | 22.11 |  | 563.00 | 26.15 |
| Loading Condition | HFL | PL |  |  |  |
| Summation Vertical Force, ΣFv | 233.94 | 233.94 | KN |  |  |
| Summation Horizontal Force, ΣFh | 131.31 | 22.11 | KN |  |  |
| Summation moments, ΣM+ | 226.33 | 563.00 | KN.m |  |  |
| Summation moments, ΣM- | 26.15 | 26.15 | KN.m |  |  |
| arm of Resultant Load, X`=ΣM/ΣFv | 0.86 | 2.29 | m |  |  |
| Middle two third Rule, B/6 | 3.45 | 3.45 | m |  |  |
| Eccentricity, e=X`-B/2 <B/6 | 0.87 | 0.57 | ok |  |  |
| Overturning FOS=ΣM+/ΣM- | 8.66 | 21.53 | ok |  |  |
| Sliding FOS=µΣFv/ΣFh | 1.34 | 7.94 | ok |  |  |

Table 9-2 Stability Analysis for D/s Wing Walls

| Loading Condition | Force (KN/m) |  | Lever arm, m | Moment (KN.m) |  |
| --- | --- | --- | --- | --- | --- |
| loading types | Vertical | Horizontal | About heel | Positive, M | Negative, M |
| Self-weight,W1 | 36.80 | 0.00 | 0.20 | 7.36 | 0 |
| Self-weight,W2 | 101.20 | 0.00 | 1.87 | 188.91 | 0 |
| Weight of Soil (W3) | 8.89 | 0.00 | 0.66 | 5.87 | 0 |
| Lateral load of soil back fill | 0.0 | -4.56 | 0.40 | 0 | -1.8 |
| Hydrostatic Pressure, HFL | 0.0 | 80.00 | 1.33 | -106.67 | 0 |
| Hydrostatic Pressure, PL | 0.0 | 31.25 | 2.83 | 88.54 | 0.00 |
| uplift load force | -10.40 | 0.00 | 0.87 | 0 | -9.01 |
| Sum for HFL condition | 136.49 | 75.44 |  | 95.47 | 10.84 |
| Sum for Pool Level condition | 136.49 | 26.69 |  | 290.68 | 10.84 |
| Loading Condition | HFL | PL |  |  |  |
| Summation Vertical Force, ΣFv | 136.49 | 136.49 | KN |  |  |
| Summation Horizontal Force, ΣFh | 75.44 | 26.69 | KN |  |  |
| Summation moments, ΣM+ | 95.47 | 290.68 | KN.m |  |  |
| Summation moments, ΣM- | 10.84 | 10.84 | KN.m |  |  |
| arm of Resultant Load, X`=ΣM/ΣFv | 0.62 | 2.05 | m |  |  |
| Middle two third Rule, B/6 | 2.60 | 2.60 | m |  |  |
| Eccentricity, e=X`-B/2 <B/6 | 0.68 | 0.75 | ok |  |  |
| Overturning FOS=ΣM+/ΣM- | 8.81 | 26.83 | ok |  |  |
| Sliding FOS=µΣFv/ΣFh | 1.36 | 3.83 | ok |  |  |

## Stability Analysis of Divide wall

Table 9-3 Stability Analysis of Divide walls

| Loading Condition | Force (KN/m) |  | Lever arm, m | Moment (KN.m) |  |
| --- | --- | --- | --- | --- | --- |
| loading types | Vertical | Horizontal | About heel | Positive, M | Negative, M |
| Self-weight,W1 | 23.00 | 0.00 | 0.50 | 11.50 | 0 |
| Lateral load of soil back fill | 0.00 | 3.83 | 0.37 | 0 | 1.4 |
| uplift load force | -5.50 |  | 0.50 | 2.75 |  |
| Sum for HFL condition | 17.50 | 3.83 |  | 14.25 | 1.40 |
| Loading Condition | HFL |  |  |  |  |
| Summation Vertical Force, ΣFv | 17.50 | KN |  |  |  |
| Summation Horizontal Force, ΣFh | 3.83 | KN |  |  |  |
| Summation moments, ΣM+ | 14.25 | KN.m |  |  |  |
| Summation moments, ΣM- | 1.40 | KN.m |  |  |  |
| arm of Resultant Load, X`=ΣM/ΣFv | 0.73 | m |  |  |  |
| Middle two third Rule, B/6 | 1.00 | m |  |  |  |
| Eccentricity, e=X`-B/2 <B/6 | 0.23 | ok |  |  |  |
| Overturning FOS=ΣM+/ΣM- | 10.15 | ok |  |  |  |
| Sliding FOS=µΣFv/ΣFh | 4.57 | ok |  |  |  |

## Stability Analysis of Canal Retaining Wall

Table 9-4  Stability Analysis of MC01 Retaining wall

| Loading Condition | Force (KN/m) |  | Lever arm, m | Moment (KN.m) |  |
| --- | --- | --- | --- | --- | --- |
| loading types | Vertical | Horizontal | About heel | Positive, M | Negative, M |
| Self-weight,W1 | 18.40 | 0.00 | 0.20 | 3.68 | 0 |
| Self-weight,W2 | 25.30 | 0.00 | 1.13 | 28.67 | 0 |
| Weight of Soil (W3) | 8.89 | 0.00 | 0.66 | 5.87 | 0 |
| Lateral load of soil back fill | 0.0 | -4.56 | 0.40 | 0 | -1.8 |
| Hydrostatic Pressure, HFL | 0.0 | 20.00 | 0.67 | -13.33 | 0 |
| Hydrostatic Pressure, PL | 0.0 | 31.25 | 2.83 | 88.54 | 0.00 |
| uplift load force | -3.00 | 0.00 | 0.50 | 0 | -1.50 |
| Sum for HFL condition | 49.59 | 15.44 |  | 24.89 | 3.32 |
| Sum for Pool Level condition | 49.59 | 26.69 |  | 126.76 | 3.32 |
| Loading Condition | HFL | PL |  |  |  |
| Summation Vertical Force, ΣFv | 49.59 | 49.59 | KN |  |  |
| Summation Horizontal Force, ΣFh | 15.44 | 26.69 | KN |  |  |
| Summation moments, ΣM+ | 24.89 | 126.76 | KN.m |  |  |
| Summation moments, ΣM- | 3.32 | 3.32 | KN.m |  |  |
| arm of Resultant Load, X`=ΣM/ΣFv | 0.43 | 2.49 | M |  |  |
| Middle two third Rule, B/6 | 1.50 | 1.50 | M |  |  |
| Eccentricity, e=X`-B/2 <B/6 | 0.32 | 1.74 | Ok |  |  |
| Overturning FOS=ΣM+/ΣM- | 7.49 | 38.16 | Ok |  |  |
| Sliding FOS=µΣFv/ΣFh | 2.41 | 1.39 | Ok |  |  |