the national REGIONAL staTe of oromia

OROMIA IRRIGATION DEVELOPMENT AUTHORITY (OIDA)

Selected weir Axis

Traditional diversion weir Axis

OROMIA 19 SMALL SCALE IRRIGATION PROJECTS (SSIPs)

FEASIBILITY STUDY AND DETAIL DESIGN FOR LOT-4 SSIPs:

ABISHIKA,TILO, DANISSA AND LEGA LOLA

PART-II:-DETAIL ENGINEERING DESIGN REPORT

ANNEX-A:-DETAIL ENGINEERING DESIGN REPORT,

AND

OPERATION & MAINTENANCE MANUAL OF TILO

By

INTEGRATED WATER RESOURCES DEVELOPMENT PLC

P. O. Box: 382/1250, Addis Ababa/Finfinne

E-mail:hundeifa@gmail.com/ planetid21@gmail.com

Tel.:+251-930-013-249/250

JUNE, 2016

FINFINNE

FINAL FEASIBILITY STUDY & DETAIL DESIGN REPORTS

DELIVERABLES:

PART-I:-FINAL FEASIBILITY STUDY REPORT

ANNEX A: - CLIMATE AND HYDROLOGICAL ANALYSIS

ANNEX B: - SOIL AND LAND SUITABILITY STUDY

ANNEX C: - GEOLOGY AND GEOTECHNICAL STUDY

ANNEX D: - IRRIGATION AGRONOMY

ANNEX E: -WATERSHED MANAGEMENT

ANNEX F: -SOCIOECONOMY, AND ORGANIZATION AND MANAGEMENT STUDY

ANNEX G: -FINANCIAL & ECONOMIC ANALYSIS

ANNEX H: -ENVIRONMENTAL & SOCIAL IMPACT ASSESSMENT

PART-II:-DETAIL ENGINEERING DESIGN REPORT

ANNEX-A:-DETAIL ENGINEERING DESIGN REPORT, AND OPERATION & MAINTENANCE MANUAL

ANNEX-B:-DETAIL ENGINEERING DRAWINGS (ALBUM)

TABLE OF CONTENTS

Contents                                                                                                                                      Pages

1.	INTRODUCTION	1

1.1	General	1

1.2	Study Overview	1

1.3	Scheme Location	1

1.4	Scheme Command Area and lay out	2

1.4.1	Command Area	2

1.4.2	Main System Layout and Command Units	3

1.4.3	Tertiary and On-farm system layout	3

2.	IRRIGATION SYSTEM	5

2.1	Design Discharges and Night Storage	5

2.1.1	General	5

2.1.2	Design and Operation of Night Storage	6

2.2	Canal Design Criteria	8

2.2.1	Canal Embankment Soils	8

2.2.2	Water and Sediment Intake	9

2.2.3	Typical Canal Cross Sections	9

2.2.4	Minimum Flow Velocities	9

2.2.5	Maximum Permissible Velocities	10

2.2.6	Sand Sediment Transport	10

2.2.7	Silt Sediment Transport	10

2.2.8	Manning’s Roughness Coefficient	10

2.3	Tilo Main Canal	10

2.3.1	Canal Hydraulic Design Approach	10

2.3.2	Lacey Sediment Factor, f	11

2.3.3	Lacey Width Factor, e	11

2.3.4	Design and Design Drawings	11

2.4	Tilo Secondary and Tertiary Canal Design	11

2.4.1	Canal Hydraulic Design Approach	11

2.4.2	Canal Width	12

2.4.3	Design and Design Drawings	12

2.5	Drainage System	12

2.5.1	Design and Design Drawings	12

2.5.2	Drainage System Nomenclature	13

2.5.3	Drainage Areas	14

2.5.4	Field Drainage Canals	14

2.5.5	Drainage Modulus	14

2.5.6	Hydraulic design	14

2.5.7	Design of Field Drainage canal	16

2.6	Main Canal Irrigation Structures	17

2.7	Secondary Canal Irrigation Structures	17

2.7.1	18

2.7.2	Other structures	18

2.7.2.1	General Structural Design Criteria	18

2.7.2.2	Concrete and Reinforcement	19

2.7.2.3	Drop structure	20

2.7.2.4	Division Boxes	22

2.7.2.5	Turn outs	23

2.7.2.6	Road crossing culverts and footpath	23

2.8	Tertiary System Structures	27

2.9	Gates	27

2.10	Design of Intake Facility	27

2.11	.  Proposed Water Distribution	27

2.12	Method of Water application	28

a)	Furrow Structure	28

b)	Length of Furrows	28

c)	Slope along the Furrows	28

d)	Stream Size	28

e)	Net Water Application Depth	29

2.13	Water distribution	29

3.	HEAD WORK (DIVERSION WEIR) DESIGN	30

3.1	Water Resource	30

3.1.1 Design Flood	30

3.2	Foundation Condition	31

3.3	Purpose of Diversion Weir	31

3.4	Design Considerations	31

3.4.1	General	31

3.4.2	Selection of the head work site	31

3.4.3	Types of Diversion weir	32

3.4.4	Weir crest elevation	32

3.4.5	Length of the weir crest	33

3.4.6	Determination of flow depth over the weir crest	34

3.4.7	Basic Section of the weir body	36

3.4.8	Determination of Bottom level of the weir and stilling basin	36

3.4.8.1	Floor level of the stilling basin (apron)	37

Hydraulic jump on the horizontal surface	37

3.4.9	Length of Stilling Basin	38

3.4.10	Thickness of downstream apron	39

3.4.11	. Khosla’s Exit Gradient (GE)	41

3.5	Structural Analysis of the Weir Body	42

Take:-	43

3.6	Wing Walls	46

3.6.1	Active Earth Pressure against vertical walls	46

Stability condition	48

4	WUA INSTITUTIONAL DEVELOPMENT AND SUSTAINABLE OPERATION AND MAINTENANCE	50

4.1	Introduction	50

4.2	WUA Formation and Capacity Building (O&M-1)	50

These O&M subprojects are described below. Subproject description sheets are included as	51

4.3	System Operation (O&M-2)	51

4.4	System Maintenance (O&M-3)	52

4.5	Business Planning, Management Information System and Cost Recovery & M.4)	54

5	PROJECT IMPLIMENTATION APPROACHES AND PROGRAM	56

5.1	Roles and Responsibilities of Key stakeholders	56

5.2	Overall Implementation Program	56

5.3	Scheme Construction Program	56

6	PROJECT COST ESTIMATE AND UNIT RATE ANALYSIS	59

6.1	Costs of the Project	59

7	CONCLUSIONS AND RECOMMENDATIONS	63

7.1	Conclusions	63

7.2	Recommendations	64

ANNEX A:  SYSTEM LAY OUT MAP	66

ANNEX B: UNIT RATE ANALYSIS	67

List TableS

Tables                                                                                                                                           Pages

Table 1-1: With project land use	2

Table 1-2: Secondary Canal Lengths and Tertiary Unit Areas	4

Table 2-1: Crop Water Requirements and Typical Design Flows	5

Table 2-2: Recommended Flow Velocities	9

Table 2-3 Canal side slope and B/D Ratio	12

Table 2-4: Drain Categories, Discharges and Areas	13

Table 2-5: Category of Main Canal Structures	17

Table 2-6: Category of Secondary Canal Structures	17

Table 3-1: Thickness of downstream apron floor	40

Table 3-2Stability analysis for high flood condition(dynamic case)	42

Table 3-3 Stability analysis for no over flow condition Static case	44

Table 3-4 Stability analysis for retaining wall  active earth pressure	48

Table of Figures

Tables                                                                                                                                                Pages

Figure 2.1 Discharege curve on weir crest……………………………………………………………...….  21

Figure 3-1 Head work site of Tilo/Gora River	32

Figure 3-2 Discharge curve over the weir crest	35

Figure 3-3 Free diagram of energy profile at dynamic  case	42

Figure 3-4 Schematic diagram of wing wall	47

EXCUTIVE SUMMARY

Project Description

Oromia is gifted with ample water and land resources, having eight river and one lake basins that can potentially be used for irrigation. The region is however, dependent on rain fed agricultural practices and is yet to ensure food security and food self sufficiency. The best alternatives to consider for reliable and sustainable food security development is therefore, expanding irrigation development on  at various scales, through constructing of dams, river diversions, spate irrigation and other water harvesting structures. Hence, Oromia regional state is strongly working on its river basins with an objective of ensuring food security and the region is in a good economic progress to attain its objective.

The proposed Tilo/Bose project will develop 20ha of smallholder managed irrigated land within a gross area of 30 ha laying on undulating and gently sloping on tops of land between the streams. The scheme area is already densely populated by smallholders whose livelihoods are based on integrated livestock and rain fed crop agriculture, supplemented by traditionally irrigated on mono crops i.e. Average landholdings are about 1.0-2.5ha though there are significant landless households. To boost agricultural production and alleviate pressures on land resources, irrigation will be enabled by diverting water from Tilo spring to a 789 m long lined main canal, distributing water to farmers‟ fields for mostly furrow irrigation. For optimal efficiency land smoothing and formation of furrows with suitable slope, size and lengths for irrigation streams will be carried out, as well as provision of night storage reservoirs at main canal level and surface drainage cut off ditches at up slope of main canal to protect it from each wet seasonal silting up. At the riverine corridors livestock grazing may be promoted. There is currently all-weather access to and within the scheme area and a need of constructing access road to the head work site for length of 1.km. There will be wide-reaching land use change with the project, whereby the majority of the existing rainfed crop area, about 20 ha, will be converted to irrigate cropping. The settlement areas and surrounding grazing land will remain, but linear (strip) forestry will be promoted along major canals, drains and all-weather roads to protect and enhance the environment. The increased fuel wood will release dung, currently burnt by households for cooking, to be used as an organic fertilizer. The new infrastructure including strip of forestry will occupy about 2 ha, 6.7 % of the gross command area. The buffer zone along the river banks will be excluded from irrigation development. Land consolidation (exchange) is required to: (i) enable the revised land use under the project; (ii) provide at least 0.2 ha of irrigable land to the landless; (iii) reduce the current highly fragmented landholdings.

This will entail a 13% reduction of land holdings from about 2.5 ha (rain fed) to 2.0 ha (irrigated) with land exchange between farmers. The variation of soil fertility within the command area will need to taken into account in the land consolidation (reallocation) process.

To facilitate achievement of potential crop and livestock production a range of complementary interventions are identified to address input, processing and marketing constraints. The poor and vulnerable are characterized by farmers with small or no landholdings and women headed households. A range of community support and development interventions, including health, water supply and off-farm income generating activities will focus on these groups. Farmers in the command area currently have some experience of traditional small scale irrigation and are expected to take over management, operation and maintenance of the scheme following its completion. A range of O&M interventions will enable this including support and capacity building of necessary farmer institutions, and funding for a main system operator for a years as an interim arrangement while farmer capacity is built up.

Rationale

The scheme area is densely populated with about 85% of the area currently under rain fed cropping with the balance largely utilized for settlements (13.3%) and communal grazing (20%). Gully stream erosion is significant. Dung is used for fuel rather than as an organic fertiliser due to the shortage of wood. The land and water resources in the scheme area are suitable for irrigated agriculture development, and the proposed project will result in substantial gains in farm incomes stemming from an annual cropping intensity increase from about 116% to over 160% as well as from increased crop yields. Increased crop residues will in turn support an increase in livestock population. To ensure maximum and sustainable agricultural benefits, in addition to construction of appropriate irrigation, drainage, and  roads infrastructure, the  project includes for a range of complementary agricultural, livestock, community development, environment and irrigation system O&M interventions.

Project location

Tilo Small Scale irrigation Project is located in Qubsa Bora peasant association, Digalu fi Tijo District, Arsi zone of Oromia region. The Headwork axis is located at geographical coordinate of 0535162E and 0858927N   across a Tilo River. The command area lies on right sides of the Parent River. The head work site is 1 km from beneficiary village and 28 kms from Sagure district’s capital town and about 218 kms far from the regional’s Capital Finfinne.

This project is planned on Tilo River which has 18l/s base flow. After assessing the ecosystem downstream of the selected weir axis and availability of command area, it is planned to divert only 95% of base flow, a total of 15l/s to irrigate a net command area of 20ha with provision of night storage reservoir.  A traditional irrigation has been practiced in this project area. The supply regime of 24 hours has been preferred mainly on the grounds of social customs and irrigation practice of 12 hours in the day time.

The designed irrigation scheme consist of headwork structures with its all components at the selected site, The command area has one Night storage reservoir, one   main canal, of 789m length and supplied to one  secondary units through  one secondary canal after pond out let.

Irrigation method recommended by the consultant is surface irrigation (i.e. furrow irrigation).

The whole Main canal is designed to be lined while secondary and tertiary canals and drains are designed as unlined trapezoidal cross section. In these reach the main canal is designed as rectangular masonry lined throughout its length. Also different conveyance drops and farm structures i.e. division box, off takes, foot path Bridge etc are designed.

The total investment cost for developing the 20ha is 2.66 Million Birr including physical contingencies, over the net irrigable command area. The cost breakdown is summarized below.

| Nr | Cost Item | Amount |  |  |
| --- | --- | --- | --- | --- |
|  |  | Amount Birr | Birr/ha | % |
| 1 | Irrigation, drainage, flood protection and road | 1,911,004.52 | 95,550.23 | 90 |
| 2 | Sustainable O&M | 191,100.45 | 9,555.02 | 10 |
|  | Grand total | 2,102,104.97 | 105,105.25 | 100 |
|  | Contingency (10%) | 210,210.50 |  |  |
|  | Grand Total | 2,312,315.47 | 115,615.77 |  |
|  | Vat 15% | 346,847.32 |  |  |
|  | Project cost | 2,659,162.79 | 132,958.14 |  |

# INTRODUCTION

## General

Tilo Small Scale Irrigation Project is 20ha schemes to be developed by Oromia Regional state Irrigation development Authority (OIDA). The project is aimed at improving the livelihood of the project beneficiaries and related benefited stakeholders and modernizing the existing traditional practices of gravity fed irrigation activities, and improving the water resource utilization by increasing irrigation efficiency. The overall objective of the scheme is to increase agricultural productivity, thereby increasing the income of Qubsa Bora P.A. farmers. The project mainly comprises of diversion weir, canal network and catchment management.

This report describes the feasibility Study and detailed Design of Small-Scale Irrigation Scheme, which is a diversion weir based scheme. The engineering parts possessing the design of the headwork’s as well as on farm structures works.  The community organization and management Section briefly assesses the existing farmer organization and recommends the measures required to strengthen the Association. The project implementation approaches and implementation program as well as the required project costs are presented in Section and respectively. The conclusions of the study and design as well as the way it forwarded. Finally references and appendices of the sectoral studies are also attached at the end of the document.

## Study Overview

The Oromia regional state Irrigation development Authority commissioned Planet Integrated Water Resources Development PLC to undertake the feasibility study and detailed design of Tilo small scale irrigation project.

This study involves the draft feasibility study and detailed design of the scheme for the purpose of preparing the project for construction and to facilitate its management on sustainable basis. The detail design study phase tasks focused on study and design of weir, canals and irrigation structures.  The study addressed the findings and preparations of different sectoral studies required for the scheme development, Irrigation Engineering (headwork, canal and on farm structures work design).

## Scheme Location

Tilo Small Scale irrigation Project is located in Qubsa Bora peasant association, Digalu and Tijo District, Arsi zone of Oromia region. The Headwork site is located at geographical coordinate of 0535162E and 0858927N   across Gora River. The command area lies on right sides of the diverted river. The project site is 8 km from Digalu Town, 28 kms from Sagure district’s capital town, 52 km from zonal capital Asela   and about 218kms from the regional Capital Finfinne.

## Scheme Command Area and lay out

### Command Area

The command area is bounded by the Main canal to North West and by, the adjacent rivers. Course to the east (refer A4 size Layout Map attached to this report as Annex A).

In practice, where fields are already formed and traditional irrigated farming practiced the boundary was extended in places similar to this contour but limit by available irrigation water amount. Settlement areas, as ragged land areas and all other areas deemed unsuited by the soil survey were excluded from the command.

Having delineated boundaries the gross and net command areas were determined as follows:

- Study Area: 32 ha

- Gross Command Area 30 ha

- Net Command Area: 20 ha (within the gross command area)

The planned with-project land use is tabulated below. Double cropping is expected over the net irrigable area of 20 ha.  A conservative estimate of land taken up for new infrastructure and for linear plantations along the flood cut off canals, major canals and all-weather roads is 1.5 ha (5%).  New infrastructure includes the new, construction camping and inspection roads, Irrigation canals and structures including night storage reservoirs, drainage infrastructure, and the flood protection.

Table 1-1: With project land use

| Nr | Description | Land use |  |
| --- | --- | --- | --- |
|  |  | ha | % |
| 1 | Net irrigable command area | 20 | 67 |
| 2 | Irrigation & drainage infrastructures | 1.5 | 5 |
| 3 | Settlements | 4 | 13.3 |
| 4 | Riverine area, proposed for grazing | 2.5 | 8.3 |
| 5 | Rocky (hillocks), partial grazing | 1 | 3.35 |
| 6 | Unsuitable for irrigation, grazing | 1 | 3.35 |
|  | Gross Command Area | 30 | 100 |
|  | Land outside gross command area | 2 |  |
|  | Study Area | 32 |  |

The existing settlement areas remain unchanged, though project interventions may include, for buildings for farmer cooperatives and water use associations, and forestry plantation. The riverine area, hillocks and other land unsuited for irrigation, including seasonally water logged land and the smaller (non bunded) natural stream gullies would mostly also be used for grazing. The with-project land use envisages substantial changes from the current (present) land use requiring active participation of beneficiaries in the land acquisition and consolidation process.

### Main System Layout and Command Units

The main canal starts from a Diversion weir Northern  end of the scheme, which irrigation water   discharge in to main canal at an elevation of +2958.20 m and is aligned close to the land slope supplying water to night storage reservoir and secondary and tertiary canals that flow south or south-west/or run down-slope towards the tail end. The irrigation and drainage layout divides the net command area into one secondary and 8 tertiary irrigation units on the basis of topography, existing gullies and stream alignment. To form similarly sized secondary command units of 20 ha, the small Tertiary canals, particularly in the head of the system, are grouped together on main canal while the others tertiary canals, near the tail, each supplied irrigation water from the secondary canal. Tertiary hydraulic units of typically 1.25- 3.5 ha are delineated within each secondary unit. Secondary canal lengths and net command areas of tertiary canal units are tabulated below in Table 1-2.

### Tertiary and On-farm system layout

It is considered that furrow irrigation will be adopted for the majority of the command area, the exception being low laying land where basin irrigation of fruit tree is likely, at least in riverine. Existing land slopes within the command area are mostly less than 2% and rarely more than 3.0%. To minimize land grading the furrows would be aligned so that furrow longitudinal slopes are mostly in the range 0.2-0.4%. Furrow cross slopes may vary from 2% to, in rare cases, as much as 3%. For this scheme furrow lengths of 80-100m are likely to be suitable, with furrow stream flows of 1.5-2.6 l/s. With these furrow lengths and adopting a rectangular layout, there would be only one field channels taking off from each tertiary canal. The proposed irrigation and drainage layout is illustrated for secondary unit in  attached as  lay out drawing.

Adoption of regular / rectangular layouts with precision land leveling and formation of optimum furrow lengths (basin sizes) is proposed. This will allow tertiary units roughly equal in size to be formed, with tertiary (and field channels) aligned to ensure rectangular fields. Precision land leveling would be necessarily carried out to form suitable furrow slopes (or flat basins) though out the command area and smooth out minor gullies and surface irregularities, but it will incur more cost.

After the construction of secondary canals is carried out and field boundaries delineated, farmers would be responsible for construction of the earthen tertiary and field channels (and furrows / basin bunds) with Project support.

Table 1-2: Secondary Canal Lengths and Tertiary Unit Areas

| Canal Name | Canal Length (m) | Total Area (ha) | Total Irrigable Command area (m) | Remark |
| --- | --- | --- | --- | --- |
| MC | 788 | 30 | 20 |  |
| SC-1 | 720 | 25 | 20 |  |
| TC-1 | 145 | 2 | 1.5 |  |
| TC-2 | 145 | 3 | 2.5 |  |
| TC-1-1 | 193 | 2.5 | 2.0 |  |
| T-1-2 | 216 | 3.45 | 3.0 |  |
| TC-1-3 | 250 | 3.5 | 3.25 |  |
| TC-1-4 | 204 | 3.75 | 3.5 |  |
| TC-1-5 | 198 | 4 | 2.25 |  |
| TC-1-6 | 127 | 4 | 3.5 | 21.5 |

# IRRIGATION SYSTEM

## Design Discharges and Night Storage

### General

Crop water requirements were calculated using the FAO CROPWAT8 program for the proposed wet season and dry season cropping patterns. The cropping pattern adopted recognizes that smallholders are likely to continue to grow a high proportion of grain (row) crops for local consumption, such as barley and wheat, but introduces more vegetables as well as potatoes and others, particularly in the dry season. The peak net crop water requirement of 0.36 l/s/ha occurs in February. Allowing for application and conveyance losses the continuous flow duties at the head of tertiary and secondary canals and at the head of the system were determined as tabulated below.

The design (peak) duty at the head of the Main canal is 0. 73 l/s/ha. However actual demand will fluctuate, typically increasing from zero during much of the wet season and peaking from January to March for dry season cropping

Table 2-1: Crop Water Requirements and Typical Design Flows

| Items | units | quantity |
| --- | --- | --- |
| Net peak crop water requirements | l/s/ha | 0.36 |
| Application losses (with night storage) | % | 85% |
| CWR at field | l/s/ha | 0.42 |
| Tertiary system conveyance Losses | % | 80% |
| CWR at head of tertiary canals | l/s/ha | 0.53 |
| Secondary canal conveyance losses | % | 80% |
| CWR at head of secondary canals | l/s/ha | 0.66 |
| Main canal conveyance losses | % | 90% |
| CWR at head of main canal | l/s/ha | 0.73 |
| Typical design flows (24 hour flow) |  |  |
| Tertiary command area | Ha | 2 |
| Tertiary design discharge | l/s | 1.5 |
| Secondary command area | Ha | 15 |
| Secondary design discharge | l/s | 10.95 |
| Main canal command area | Ha | 20 |
| Main canal design discharge | m³/s | 14.6 |

Night storage along Main canals is proposed as the additional cost, both for the night storage reservoirs and the greater canal capacities in downstream secondary canals, is outweighed by the benefits accruing from increased water use efficiency from day time (12 hours / day irrigation).

Design water requirements and discharges for the secondary canals downstream of the 12-hour night storage reservoirs are therefore 1.32 l/s/ha, double the 24-hour requirement of 0.66/s/ha. Irrigation supply will be supplied to each tertiary offtaking from a secondary canal for 12 hours each day. The design discharge to the head of each tertiary canal is 1.0 6/s/ha, double the 24-hour requirement of 0.53 l/s/ha.

Each tertiary will typically supply three field channels for 12 hours each day. The field channel supply furrows (or basins for fruit trees) and each commands about a third of the tertiary unit. For a tertiary command of 3 ha, irrigation supply to the head of the tertiary canal will be 2.2l/s. The field channel will be designed from head to tail for this discharge, and supplies rotated to furrows (or basins) from the field channel in turn over the rotation period, typically 14 days.

### Design and Operation of Night Storage

- General Considerations

Night storage reservoirs are proposed so that while the main distribution system flows continuously farmers can irrigate during the day time (nominally 06:00 to 18:00 hours). Design of canals downstream of the night storage reservoirs has to take account of the different day and night operation conditions.

The maximum distance between night storage reservoirs and their individual command areas should be less1 km in order to keep the time lag before flow from a reservoir reaches the tail command area to about 0.5 hour.

The tertiary canals fall into two groups:

- Group 1: Tertiary canals where it supplied direct from main canal for the whole command it serves for Portion of the command upstream of the Night storage reservoir (typically command area on the upstream of the night storage tertiary units) as the Main canal is sufficiently high to allow two tertiary units to be provided before it join the secondary command area.

- Group 2: those portion of the secondary command irrigated using day time flows from the main canal, surplus flow plus day time flows releases from the night storage reservoir.

(b) Working Head

The area required for a night storage reservoir of given capacity is inversely proportional to the working head. The volume needed to store 0.015m³/s for 12 hours is 648 m³ or 0.0648 ha for 1m working head (depth). Deeper night storage reservoirs are preferable because of: (i) reduced land take; (ii) lower cost for lining or earthworks; (iii) the larger working range will be more disruptive to snails and vegetation (see below); and (iv) there would be a smaller area to be cleared of vegetation.

In preparing the secondary canal long section designs the working head for night storage reservoirs is maximised as far as possible, while taking into account other canal design considerations including: (i) command requirements; (ii) hydraulic considerations informing slope and prism section; and (iii) minimum head-losses for canal structures.

The design of night storage reservoirs needs to take account of the prevalence of schistosomiasis in the beneficiary’s area. Recommendations for designing and operating irrigation systems to mitigate the risk of increased schistosomiasis are given in several publications.  Proposed measures focus on creating conditions unfavourable for snails.

Key engineering recommendations for night storage reservoirs are therefore:

- Storage reservoirs should be sized to store water to allow day-time irrigation from 06:00 to 18:00 hours and should not be oversized;

- The night storage reservoirs should be constructed “offline”, adjacent to the Secondary canals with connecting inlet and outlet channels, so that they can be by-passed and dried out during low crop water demand periods. An efficient arrangement for these inlet and outlet structures, and the required cross regulator/drop along the secondary canal, would be to group them close together to facilitate (gate) operations;

- Annual or more frequent clearance of vegetation from the reservoirs when they are empty;

- Provision of lining with Geomemberan (HDPE sheet) to stop water loss and snails burrowing into the mud when the reservoirs are drained. It is noted that an extract from the Endod “soap-berry” plant appears to be a natural molluscicide.

- Operation and Design Flows

The basic concept for the Night storage reservoirs is that they fill during the night when water is not used for irrigation and then empty during the day when the stored water supplements the continuous inflow from the Main canal.

Steps in fixing the dimensions of Night store Reservoir

- Incoming flow in main /Secondary canals, Q = 15l/s

- Time of inflow/ Night time, t                      = 12 hours

- Volume of water to be stored for time t, V=15*60*60*12=684,000lt=648m3

- Working head required/Depth of water to be stored, d= 1.5.0m based of the specific condition of the site.

- Assumed dead storage volume, Vd=20% of the life storage and 0.5m depth respectively.

- The total depth required at the center of night storage bed, D= 1.5.0m+0.5+0.5m (free board)=2.50m

- Side slope ; 1V:1.25H for cut and 1V: 1.5H for embankment on external  side

- For simplicity of working condition, rectangular/square  shape is selected

V=Tw1+Bw12*Tw2+Bw22

Where Tw1=   one side of trapezoidal pond top width (m)

Bw1= one side of trapezoidal pond bottom width (m)

Tw2=   another side of trapezoidal pond top width (m)

Bw2= another side of trapezoidal pond bottom width (m)

d= effective depth of Night storage reservoir

Here assumed square sided night storage for Tw1=Tw2 =53m &Bw1=Bw2=48m are found to be satisfactory

Night storage resevoir

Out let

In let

Day: 0l/s

Night: 15L/S

Day:15 l/s

Night: 0L/S

Day:15 l/s

Night: 0L/S

Day:30 l/s

Night: 0L/S

690=23+202*23+202*1.5

SC

Figure 2-1: Day and Night time flows at Night Storage Reservoir

## Canal Design Criteria

### Canal Embankment Soils

The soils in the command area comprise mostly red clay within the whole command area and Construction of canal embankments with adjacent earthen material is quite usual in irrigation schemes to reduce costs. However the essentiality of the embankment in this scheme is less important.

### Water and Sediment Intake

The source of irrigation water is tilo spring which has no concentration of suspended solids as indicated by the clear colour of the water. The suspended material is understood to be partly from surface flow entering to the stream. The suspended material is likely to comprise mostly clay and fine silt with a fall velocity (in still water) of about 0.2 mm/s. Sediment in the river  water entering the canal system will not settle out but will travel through the canal system to farmer fields. The canals are therefore designed based on the material of which they are constructed rather than incoming sediment.

### Typical Canal Cross Sections

Freeboard, bed width, prism side slope and bank top widths shall be as detailed in the typical Canal Cross Section Drawings, Annex A.

An initial estimate for prism side slopes related to discharge is as follows:

- 1V:1.5H for discharges greater than 1.5 m³/s

- 1V:1.0H for discharges less than 1.5 m³/s

### Minimum Flow Velocities

With only clay and silt fractions expected to enter the canal with stream water a minimum flow velocity to avoid sedimentation is not an issue. However sufficient flow velocities to prevent extensive weed growth, canal blocking and expensive maintenance for week removal is necessary. Chow (page 158) suggests 0.75 m/s is appropriate to prevent weed growth. This would seem appropriate for a large canal (say 10 m³/s or larger). However for the clay soils, etc, which may comprise some of the canal embankments such a high velocity, is likely to be erosive, particularly for smaller canals.

Average minimum flow velocities may be related to canal size / discharge using Kennedy’s critical velocity formula developed for sediment transporting canals in India. Critical velocities for fine silt (m = 0.7) gives indicative values for tilo flow velocities.

Table 2-2: Recommended Flow Velocities

| Depth of Flow |  | Vcrit |  |
| --- | --- | --- | --- |
| Ft | m | ft/s | m/s |
| 0.33 | 0.10 | 0.29 | 0.09 |
| 0.98 | 0.30 | 0.58 | 0.18 |
| 1.97 | 0.60 | 0.91 | 0.28 |
| 2.95 | 0.90 | 1.18 | 0.36 |

### Maximum Permissible Velocities

The soils of which the canals are formed are thought to have relatively cohesion being very plastic when wet. Dispersive clays may also occur. Chow (pages 165 & 166) provides the following maximum flow velocities for clear water:

- Stiff colloidal clay: 1.14 m/s

- Non-cohesive clay: 0.15 m/s

Neither of these values is considered appropriate, with stiff clay 1.14 m/s being far too high and non-cohesive clay 0.15 too low as some cohesion is expected. As sedimentation is not an issue (though weed growth is) and as avoiding scour is paramount the adoption of average flow velocities close to the minimum flow velocities is proposed (refer above table where velocities are related to flow depth).

### Sand Sediment Transport

As little sand transport is expected the value of calculated sand transport (using Eugulend & Hansen formula) is to check (i) that sand transport is low, say about 50 mg/s, and (ii) is constant or slightly deceases going downstream from reach to reach (a reach with a higher than average sand transport capacity indicates scour may occur).

### Silt Sediment Transport

The incoming sediment will be close to the river sediment concentration and comprise clays and silts (providing the canal intake is designed carefully to avoid iterance of bed sediments from the river). Silt sediment transport formula can predict widely differing values and two formulae should therefore be considered with the average in the range 200-700 mg/s. The following methods are adopted:

- Westrich & Jurashek; and

- Arora, Rajut and Garde

### Manning’s Roughness Coefficient

A roughness coefficient of 0.022 is adopted commensurate with straight and uniform excavated earthen channels in weathered condition with minimal vegetation.

## Tilo Main Canal

### Canal Hydraulic Design Approach

The Lacey equations are used for initial estimations of regime canal slope and widths for various design discharges, with appropriate values of sediment factor, f, and width factor, e. Manning’s equation with an appropriate roughness, n, is then used to determine flow depths and average flow velocities.

To check and adjust the canal prism design the following is then checked:

- Average flow velocities: adopt Kennedy critical flow velocity

- Sediment (silt / sand) transport

### Lacey Sediment Factor, f

For fine sediment low values of the sediment factor are appropriate. Values of 0.5-0.7 appear appropriate. Note: the lower the sediment factor the flatter the canal slope.

### Lacey Width Factor, e

For tenacious (clayey) material width factors of 0.7 to 1.0 are appropriate. The lower the width factor the tighter the canal section with less land-take. Too tight a section will tend to develop meanders. Canal sections will include berms providing a safety buffer as these berms can erode without jeopardizing the embankment.

### Design and Design Drawings

Suitable irrigation canal prism designs based on the above criteria for a range of flows are tabulated in Table 2-3 below. These serve as a design check for all Tilo irrigation canals. For a design discharge of .015 m3/s the following prism design is appropriate:

- Canal slope: 1:500 (0.002) – Lacey for Lined canal

- Bed width, B: 0.25 m – Lacey

- Flow depth, D: 0.15 m - Manning with n=0.022

- B/D ratio: 1. 5

- Average flow velocity, V: 0.55 m/s

- Silt transport: 127 mg/s (A,R&G method), 977 mg/s (W&J method)

The Main Canal hydraulic design is given in consistent with the design criteria;

Average flow velocities range from 0.56 m/s at the head to 0.26 m/s at the tail. The canal is 788 m long with a full supply level at the head of +2958.20 m gradually reducing to +2856.5m at the tail of main canal. In addition to hydraulic slope head losses, the following (minimum) head losses are adopted for structures:

- Cross regulators: 0.05 m

- Flow measurement flumes: 0.10 m

- Head regulator : 0.05 m

## Tilo Secondary and Tertiary Canal Design

### Canal Hydraulic Design Approach

For the Tilo secondary and tertiary canals prism and longitudinal slope designs are based on:

- Manning’s uniform friction flow formulae to determine the discharge for a specified cross section area and longitudinal slope.

- B/D ratio limits for width stability (see below)

- Velocity limits for bed stability, representing canal slope, tractive force and sediment transport criteria (see above).

### Canal Width

For an unlined channel, a stable section (b/d value) depends on the discharge, the adopted prism side slope, as well as the soil in which the channel is made and the sediment being transported. Basically the higher the discharge the larger the b/d value, while the more tenacious (cohesive) the soil the tighter the channel section (smaller b/d value). For tilo where Clay soils dominate the following b/d values and prism side slopes may be adopted.

Table 2-3 Canal side slope and B/D Ratio

| Discharge (m³/s) | Side Slope (1V:mH) | Bed width, b/water depth, d | Indicative Canal Category |
| --- | --- | --- | --- |
| 0.005 -0.020 | 0.5 | 1. – 1.25 | Tertiary canals |
| 0.15 – 0.3 | 1.0 | 1 – 1.25 | Secondary – Main canals |
| 0.3 – 1.0 | 1.0 | 1.2 – 2.5 |  |

### Design and Design Drawings

Secondary and tertiary canal hydraulic designs are consistent with the main canal design criteria. Hydraulic designs for one secondary canal, SC is tabulated below. Table2-5 canal immediate downstream of the reservoir is therefore 0.03 m³/s (q = 1.1 l/s/ha) for 12-hour day time irrigation.

## Drainage System

### Design and Design Drawings

Surface drainage can be defined as the diversion or orderly removal of excess water from the surface of land by means of improved natural or constructed channels. Drainage system is integral part of the irrigation system.

Drainage is necessary to: (i) address flooding and yield loss due to extensive rainfall in the rainy season; (ii) mitigate risk of water logging as cropping intensifies under irrigation; and (iii) enable periodic (surface) flushing of salts as may be required from time to time. In generally Drainage system protects the Irrigation system such as main, tertiary, and field canals, and irrigation land from damage, which would result from uncontrolled excess flow of irrigation water and surface runoff caused due to rainfall. Rain and excess irrigation water must be controlled to prevent erosion and damage of the irrigation land and irrigation system.  Excess water should be removed safely from the irrigation land by different drainage systems.

The drainage system comprises field and tertiary drains which drain the tertiary blocks. The tertiary block drainage areas are typically 15-20% larger than the net irrigable areas. Field and tertiary drains are essentially similar in size, capacity and design, the difference being that tertiary drains are aligned along the boundary of a tertiary block while field drains are aligned along the end of furrows within the tertiary block.

The tertiary and field drains typically feed into Collectors which in turn feed into Main Drains. However tertiary and field drains may feed directly into Main Drains. Few of the larger collector and main drains will be newly constructed drains, while most of them will comprise re-modeled existing river/stream channels passing through the command area.

Inlet structures control flows from one type of drainage channel to another, preventing over fast run off from fields resulting in downstream drain capacity being exceeded and over topping. Drop structures are also required in some drains to prevent excessive flow, erosion and drain prism damage.

The drainage channel layout was prepared taking into account:

- Topography and natural drainage alignments

- Existing natural stream / river gully alignment

- The need for reasonably rectangular fields for efficient (furrow) irrigation and to facilitate mechanized land preparation.

During field Survey, major drainage lines were identified in the command area. Drainage network of field and tertiary drainage system is provided to remove excess irrigation water and rainwater from the fields. According to the hydrology report, analysis of 24hr maximum Rainfall and evapo-transpiration at the command area is considered.

Typical drainage areas and drain discharges for tertiary / field, collector and main drains are tabulated below.

Table 2-4: Drain Categories, Discharges and Areas

| Category of Drain | Drain Capacity(m3/s) | Drainage Area(ha) |
| --- | --- | --- |
| Tertiary | <1.0 | <130 |
| Collector | 1-10 | 130-1300 |
| Main | >10 | 130-1300 |

### Drainage System Nomenclature

The adopted drainage system nomenclature is illustrated below where:

- MD – Main

- CD – Collector

- TD - Tertiary Drain

- FD – Field Drain

### Drainage Areas

Adopted drainage areas for each tertiary block are essentially the gross command areas less areas for which drainage is not required, such as: (i) parts of settlements; (ii) land within the drainage – flood corridors and drains themselves; and (iii) any low laying land that cannot be drained. The drainage area within the project area is estimated at about 50ha, rather more than the net irrigable command area of 20ha.

### Field Drainage Canals

Surface drainage is applied primarily on flat lands where slow infiltration, low permeability, or prevent the ready absorption of high intensity rainfall. The drainage system is therefore intended to eliminate ponding and prevent prolonged saturation by accelerating flow to an outlet without causing siltation, soil erosion, and damage to the irrigation systems. Surface water can be removed safely from the irrigation land by constructing of open drains to the main outlet to meet the requirements of surface drainage.

### Drainage Modulus

For flat areas (i.e. where average land slopes are less than 2.5%) the drainage modulus is given by the design storm divided by the evacuation time.

The evacuation time should be within 24 hours for dry land crops to avoid crop loss. As dry land crops dominate the cropping pattern 24 hours is appropriate.

For this project a 24-hour storm with a return period of 5 years is therefore adopted as the design storm, which gives a rainfall depth of 55 mm. It is assumed that this design storm falls on wet land, and the total storm volume is to be drained off within 24 hours. This gives a drainage modulus of 6.75 l/s/ha. Due to the small sizes of the drainage areas reduction factor are not used.

### Hydraulic design

(a) Approach

Two parameters which need careful consideration for hydraulic design are the longitudinal slope and the bed width/ flow depth (B/D) ratio.

Manning’s equation is used to determine the required prism cross sectional area for the design discharge and longitudinal slope. To the extent possible the adopted longitudinal slope should give the maximum permissible flow velocity (calculated using Manning’s equation) for the design discharge. In addition to the maximum permissible flow velocity the tractive force should be considered to ensure against erosion. Therefore longitudinal slopes will decrease going downstream as design discharge increases.

- Channel Roughness

The channels are designed for a Manning’s n value assuming the channel is established with some weed growth and not freshly dug. A Manning’s n of 0.030 is therefore adopted.

(c) Maximum Permissible Velocities and Tractive force

Average flow velocities to prevent weed growth and dislodge snails and control schistosomiasis would need to about 0.6 m/s. Chow (page 158) suggests 0.75 m/s is appropriate to prevent weed growth. This would seem appropriate for a large canal (say 10 m³/s or larger). However for the clay soils in which the majority of drains will be excavated such a high velocity is likely to be erosive, particularly for smaller channels.

The maximum permissible velocity in drains for the design discharge will be determined from critical boundary tractive force (prism stability) considerations using the equation:

τmax = C ρ g D S

where:

τmax = Boundary shear stress at channel bottom

ρ = Density, 1000 kg/m3

g = Gravitational acceleration, 9.81 m/s2

D = Flow depth, m

S = Hydraulic gradient

C = Correction factor for B/D ratio

C = 0.77 e(B/D)^0.065 for <1 B/D<4

C = 1.0 for B/D > 4

If the soils were assumed to be non-cohesive an acceptable tractive force of only 2.4 N/m2 may be adopted, increasing to 9.6 N/m2 if cohesive was assumed. For the purpose of the drainage channel design a maximum tractive force of 3.0 N/m2 was adopted.

- B/D Ratio

Choice of suitable B/D ratio is important for lateral stability of the prism section. An over tight (narrow) section would be prone to development of meanders; while an over wide section will be more conducive to weed growth and take up more land.

Drainage Channels are usually designed with tighter sections than irrigation canals for the

Following reasons:

- To reduce land take and minimise the area of weed growth;

- More pronounced variations in water levels are acceptable than for irrigation canals;

- Drainage channels are invariably in cut with lower that design discharges occurring for most of the time.

### Design of Field Drainage canal

To design respective drainage canal, peak runoff should be calculated based on drainage area and rainfall intensity. The length of farm boundaries determines the length of the field drains. It is often decided to place the field drains at right angles to the collectors. The length of a collector is restricted either by a field boundary or by the available slope

The field drainage system is generally designed on a model basis for a sample area. This can be a single farm of less than one hectare or an area of more than one hectare. The design is only a guideline, which can be adjusted for each single farm to incorporate specific circumstances.

Peak Runoff, (Q)

The Rational Method is used to calculate peak run off in the irrigation fields as follows. Application of the rational method is normally limited to watershed of less than 0.5km2.

Where,

Q = Design Peak Discharge (m3/sec)

C = runoff coefficient, which indicates the proportion of the design rainfall that actually discharges rapidly from the basin and which contributes to the peak discharge. It is assumed that for the cultivated and flat land (slope less than 5%) and soil of medium infiltration, the value of C=0.50.

A = Area to be drained, ha

I = Rainfall intensity in mm/h for the design return period and for a duration equal to the “time of concentration” of the watershed. Peak runoff from the drainage catchments areas (for field and tertiary drain) can be calculated with the application of the following formula

## Main Canal Irrigation Structures

Categories and numbers of main canal structures are summarized below, and in detail in the section. Head losses are kept to a minimum to maximize command from this near-contour canal.

Table 2-5: Category of Main Canal Structures

| Nr | Structure | Nr | Remarks |
| --- | --- | --- | --- |
| 1 | Cross Regulators | 1 | Side spillways end of main canal |
| 2 | Head Regulators | 1 | Gated orifice structures at head work |
| 3 | Measuring Flumes | 1 | Downstream of cross regulators |
| 5 | Cross Drainage Super passage | 1 | Up stream of the Night storage |

Flow Regulating and Measurement Structures

Gated (flume) cross regulators with overflow side-spillways are proposed (see picture below).For the Main canal the side-spillways will allow night time flow fluctuations to be accommodated safely without gate operators. For the Secondary canals the side-spillways will facilitate WUAs quickly achieving FLSs in the secondary canals each day when flows are released from the night storage reservoirs.

The Main canal head regulators shall comprise gated orifice structures enabling: (i) on-off capability; and (ii) flow rate adjustment. In theory such structures can be used to determine discharge based on gate opening and upstream (and downstream for submerged flow) water levels. However such discharge estimates are inaccurate and specific flow measurement structures are therefore proposed.

Flume flow measurement structures are proposed downstream of each head regulator and at head of each secondary canal. These will enable a quite accurate discharge measurement purely from an upstream head (water level) reading.

## Secondary Canal Irrigation Structures

Categories and numbers of secondary canal structures are summarised below, and in detail by Structure category in Table 3-6

Table 2-6: Category of Secondary Canal Structures

| Nr | Structure | Nr | Remarks |
| --- | --- | --- | --- |
| 1 | Road Crossing culver | 2 | At crossing of access road. |
| 2 | Measuring Flumes | 1 | At head of Secondary canals |
| 3 | Drops | 18 | At different chainage |
| 4 | Night storage inlets | 1 | Associated with night storage reservoirs |
| 5 | Night storage outlets &measuring flumes | 1 |  |
| 6 | Tertiary canal offtakes | 5 | 6 gated offtakes to supply 8 tertiary units |

Gated (flume) cross regulators with overflow side-spillways are proposed for the Secondary canals as well as for the Main canal. These will be more expensive but would facilitate WUAs quickly achieving design full supply levels in the secondary canals each day when flows are released from the night storage reservoirs.

Tertiary canal offtakes along the secondary canals shall comprise gated orifice structures similar to those provided along the Main canal, only smaller, controlling supply to tertiary canals.

Structures associated with the night storage reservoirs include gated inlet and outlet structures for controlled diversion of flow into the reservoirs during the night, and release for day time irrigation.

Flow measurement flumes would be provided in the Secondary canals: (i) immediately downstream of the head regulators to measure (continuous 12 hour) flows from the main canal into the secondary canals; and (ii) immediately downstream of the night storage reservoirs to measure (day time) flows and guide flow releases from the reservoir outlets.

### Other structures

Other secondary canal structures include drops and tail structures. On canal cattle trough and washing basin are provided as part of the cross regulator structures. Only a few cross drainage structures are likely to be required, and are included as part of the surface drainage system.

#### General Structural Design Criteria

- Stability

The stability of structures or parts of structures has been assessed using the following minimum factors of safety.

| - For sliding or overturning |  |
| --- | --- |
| - For failure by piping, based on the exit gradient considered to be unlikely to occur, with an absolute minimum value. |  |  |  |

The weight of concrete will be used in the analysis of safety of structure against uplift under the worst loading conditions.

- Soil Properties

The following soil characteristics have been adopted for design of structures.

| Saturated density | 2 000 kg/m3 |
| --- | --- |
| Submerged density | 1 100 kg/m3 |
| Coefficient of active earth pressure | 0.4 |
| Coefficient of passive earth resistance | 2.5 |
| Coefficient of earth pressure at rest | 0.6 |
| Maximum permissible net bearing pressure | 70 kN/m2 |
| Coefficient of base friction | 0.4 |

The above properties are approximate to those of cohesion less soil with an internal angle of friction of 250.  No wall friction has been taken into account.  Ground water table level has been generally assumed to be at canal design water level or other appropriate level and the safety of all structures will be checked for a rapid drawdown case where the canal is assumed to empty rapidly leaving an unbalanced hydrostatic pressure on the structure.  Soil below water table level is treated as submerged and soil above as saturated.  Where the structures are crossed by vehicles, a general traffic surcharge of 10kN/m2 will be assumed for all structures where vehicular access is possible.

#### Concrete and Reinforcement

For reinforced concrete, design will be performed in accordance with CP 110 for concrete grade 20 (Characteristic strength 20 N/mm2) and mild steel reinforcement (Yield strength 250 N/mm2). For mass concrete design a maximum allowable tensile stress of 0.35 N/mm2 will be assumed and conventional elastic design theory will be followed.

The following properties of concrete will be used;

| Density of reinforced concrete | 2 400 kg/m3 |
| --- | --- |
| Density of mass concrete | 2 400 kg/m3 |
| Modulus of elasticity (for deflections) | 23 x 103 N/mm2 |
| Coefficient of linear expansion | 11 x 10-6/0C |
| Coefficient of shrinkage | 300 x 10-6 |

Concrete cover to reinforcement will be 50 mm for all structures in contact with water or earth. Cover for other structural components will be 40 mm, or less for small structures. Laps in bars will be 40 x bar diameter. Bar spacing’s of 150 mm minimum and 300 mm maximum have been adopted and the following bar diameters have been used: 8, 10,  and12, mm.

The following minimum reinforcement percentages will be used:

| For main steel in the tension face | 0.25% effective area |
| --- | --- |
| Secondary steel in tension face | 0.15% gross area |

#### Drop structure

Drop structure is used to discharge water in a channel from a higher elevation to a lower one. Drops are provided at places where the ground slope is steeper than the canal bed slopes. For easy of construction and operation all drop heights with in the scheme are made standard. They are all vertical drops with standard dimensions. In some cases they are designed with raised crest level so as to minimize the effects of the approaching velocities. Under this project there are 18 vertical drops with 1.0 m height and all of them are USBR type.

The following are the relationship used to determine the length of drop and sill height:-

The rectangular vertical drop with raised crest is a simple structure for use on small channels which constructed in masonry. It does not measure flow and does not serve as a check. It requires little maintenance. The design is suitable for vertical drops up to 7 m, for channel bed widths of from 0.3m to 1.0m and for full supply depths from 0. 1 to 0.7 m. It consists of upstream bed and side protection, a crest wall, stilling basin and downstream bed and side protection. The width of the crest (across the axis of flow) may be less than the width of the cistern by 0.10 m, but where there is an off take on the immediate upstream side of the drop the width of the crest (in the case of a rectangular channel) must be equal to the width of the channel.

Hydraulic Data

Q = Discharge of the canal

V = Velocity of the canal

B = Bottom width of the canal

d1 = Water/flow depth of the canal

H = Drop height

Design procedure

Step 1 Determination of crest length and shape

Since fluming is not permissible in this type of drops the length of the crest may be kept equal to the bed width of the canal. Using USBR formula for non crested vertical drop, Crest length (bc) is

bc=0.734*Qd13/2>0.3          bc=0.734*Qd13/2>0.3

Step 2 Determination of top and bottom width of crest

Step 2 Determination of top and bottom width of crest

For discharge less than 14 m3/s Blight’s recommended that the following formula

- Top width

b1 = 0.55H		 Where   H = Crest level – d/s Canal bed level (Since the drop is non crested, H = drop height)

- Top width

B1=He+Hg	Where 	He = head over crest

g = specific gravity (2.1for masonry)

He=d1+V22g		V = approach velocity (m/s)

V = Q/A 		A = flow area (m2)

Step 3 Determination of stilling basin (cistern design)

Stilling basin width, B

B=18.46+Q1/2(9.91+Q)

Stilling basin length, L

L=3*He*H				He = head over crest

H = drop height

Sill height,

X=L/61)

X=dc/2>0.15

dc=(q23g)

q=Q/bc        Where 	dc = critical depth and q = unit discharge

From the two result obtained, provide the sill height at R.L = d/s Canal Bed Level - X

Step 4 Determination of impervious floor thickness and uplift pressure

Upstream Cutoff

Since there is no problem of uplift from counter balance by the upstream water in the canal, no need for cut off at upstream of drop.

Downstream Cutoff

The downstream cutoff at the end of stilling basin should be  = depth of water/2 + 0.6

Uplift pressure and exit gradient

As no crest is provided and the drop is not to be used as cross regulator, there will not be any heading up of water and as such there will be no uplift and also floor length from exit gradient consideration is not required

Floor thickness

As there is no uplift, a nominal floor thickness of 0.3m thick at upstream equal to length of protection work and 0.4m thick provided at downstream for length of stilling basin and up to end of downstream protection work.

Step 5 Determination of downstream and upstream protection work

Protection work are required on the upstream and downstream in order to obviate the possibly of scour hole traveling to the impervious floor and to relieve uplift pressure.

L1 = L2 = 1.2+32*Q1/2   (Downstream & Upstream length)

Provide that 0.3m thick cemented stone pitching at both upstream and downstream of the drop.

Step 6 Wing Wall Design

All wing walls must be designed as retain wall subjected to full pressure of submerged soil at their back when the channel is closed. Such a wall generally has a base width equal to 1/3rd of its height + top width.

Upstream wing wall

Upstream wing wall shall be supplied to the side slope of the canal for length of protection work from edge of the crest.

Downstream wing wall

Downstream wing walls are kept straight and parallel up to end of stilling basin and joined return wall at 90.

#### Division Boxes

Division boxes are canal structures provided across a canal to bifurcate the incoming flow in a regulated manner. In the project under consideration, division boxes have been planned to be provided in the canal network wherever there is a need for distribution of flows between canals. There are a total of one division box in this project canal network system.

The division box, which is a rectangular type, consists of gated opening, which will regulate distribution of water between ongoing, and off taking flow. The opening in a division box has an equal height and discharge coefficient; only the width of the opening will vary.

The following formula has been adopted to calculate the length and the height of the crest.

Q = C*L*(Hcrt) 3/2

Where:

Q = discharge (m3/sec)

C = Discharge coefficient

L = Effective Length of crest (m)

Hcrt = Over flow depth/Water height on the crest (flow depth-sill height)

Hence, the proportion becomes.

Q1/ Q2 =L1/ L2 = Q2/Q3 = L2/L3

Where	Q1 = flow of incoming canal

Q2 = flow of branch canal

Q3 = flow of continuing canal

L1 = width of division box in the incoming canal

L2 = width of division box in the branch canal

L3 = width of division box in the continuing canal

Q = C*L1*(Hcrt) 3/2

L1 = Q1/ (CHcrt) 3/2

L2 = L1*Q2/Q1

The depth/height of division box, D = d + fb

The width of division box, B = b + 2*m*D

Where	 b = bed width of the incoming canal

m = Side slope of the incoming canal

D = Total canal depth of the incoming canal

d = Full supply depth of the incoming canal

#### Turn outs

Purpose of the turnouts is to divert water from a supply canals to a smaller channel (Tertiary canals in our case). The  structure  will  usually consist  of  an  inlet,  a  conduit  or  a means  of conveying  water  through  the  bank  of  the supply  channel  and, where  required,  an  outlet transition.   Simply sliding gates are generally used in the inlet to control the flow.

To divert irrigation water to the irrigation fields, turnouts are proposed at appropriate locations. As indicated in the system layout, turnouts will take off either directly from the main, secondary, or tertiary canals based on the size of canals.

Opening of the turnouts proposed to have rectangular shape and to be regulated with gates. Area of the opening is determined as the same manner as the division box equation.

#### Road crossing culverts and footpath

Road crossing structures are provided where canal crosses existing roads, having communication links. These crossings also facilitate the normal movement of the residents, their cattle around the project and for the routine supervision and maintenance of the canal.

When the structures are designed to convey the canal water underneath the access track, the design discharge considered is the canal discharge. In this project there are two culverts proposed to be constructed for passing the access pass through the village. For all road crossings and foot paths rectangular box culvert with different sizes has been proposed.

A box culvert bride is designed for its ease of construction and smaller cost quantity than other type of culverts. The hydraulic and structural design is dealt carefully. And finally, the safety against shear and capacity of bearing is checked. For each box culvert RCC slab is used to prove its strength as trucks and earth moving machineries could pass.

Hydraulic Design

- Parent Canal Hydraulic Element

Q = Parent Canal Discharge (m3/s)

V1 = Parent Canal Velocity (m/s)

b1 = Parent Canal Width (m)

d1 = Flow Depth (m)

fb = Free board (m)

m = Side Slope of the canal

n = manning coefficient

T = Top width of the canal

S = Longitudinal slope

D = Total depth of the canal

- Using Rectangular Box culvert

V2 = Q/A < 2m/s

where V2 is the velocity under rectangular box

A = d2*b2

where b2 and d2 are assumed width and depth of rectangular canal

P = 2d2 + b2

R = A/P

I = (n*V2R23)2      Take n = 0.014 for masonry wall

Therefore head loss, hf= L*I

- Water surface change at u/s and d/s of culvert

- Drop of water surface at inlet of culvert

∆hi=1.5*[V22-V12]2g

- Rise of water surface at the d/s of canal section

∆ho=0.3*[V22-V12]2g

- Elevations

- U/s Canal Water Level 		= u/s CBL + Water depth in parent canal

- Culvert Inlet Water Level	            = u/s canal water level – ∆hi

- Culvert Inlet Bed Level		= Culvert inlet water level – Water depth

- Culvert Outlet Bed Level		= Culvert Inlet Bed Level – ∆hf

- Culvert Outlet Water Level	= Culvert Outlet Bed Level + Water Depth

- D/s Canal Water Level		= Culvert Outlet Water level + ∆ho

- D/s Canal Bed Level		= D/s Canal Water Level – Water Depth

- Total Head loss

Ht=1.2*[V22-V12]2g+ hf

Check water surface difference between u/s and d/s canal.

Δ Water Surface = U/s Water Level of Canal – D/s Water Level of Canal

If Δ Water Surface ≈ Ht--------ok!

Structural Design

- Design Condition

Use M-15 grade concrete

K = 0.4, neutral axis factor

J = 0.87, lever arm factor

R = 0.87, moment of resistance factor

Allowable Stresses,

δca(concrete) = 55 kg/cm2

σst(Reinforcement) = 1400 kg/cm2

Ԏa (Shear) = 9.5 kg/cm2

- Consider 1m width of strip slab

Assume

Total depth of slab (cm)	= t

Concrete Cover in both directions (cm)	= Cc

Effective depth (cm)	d’ = t-Cc

- Effective Span Length (Lef)

Effective Span length shall be the lesser of the following,

- Distance between the center of support

- Clear span + Effective depth

- Lef = Clear span + 2*0.25

Take the lesser of the above as effective span length

- Loads

Loads that are considered as Live load are vehicles, animals and humans & load considered as dead load is self weight of slab. The total proposed load which can be recommended for the design of Foot Bridge as per USBR is 5.6 ton.

P = 5.6 ton

- Bending moment and Shear force calculation

- Maximum bending moment occurs at the center and its value being

M = (PL)/4 in (N cm)		Where L = Lef

- Maximum shear force occur at support and its value being

S = P/2 in (kg)		where P = Total proposed load

- Equating maximum bending moment to moment of resistance

M = Rbd’2 	where; b = width of slab per meter length

R = moment of resistance factor

d’ = M/(Rb)> assumed effective depth

Then check for effective depth, it should be greater than the assumed effective depth given.

- Reinforcement

- Main Reinforcement (Area of tensile reinforcement required)

Ast=MJd'σst

Where Ast = area of steel required

σst= allowable stress of steel

Maximum size of reinforcing bars, ϕ (in cm)

ϕ= d’/8					where d’ = effective depth

Let use  mm diameter @ X mm Spacing

- Area of steal=π∅24……………………………………………………Eq 1

- Number of steel bar per meter width = 1m/X mm……………….………Eq 2

- Area of steel per meter width = Eq 1 * Eq 2………………………………Eq 3

If the result under Eq.3 is greater than the required area of steel (Ast), then it is accepted.

Hence provide mm diameter as main bar @ X mm c/c along 3.2m span.

Alternate bar shall be bent up at distance of 0.1L from support.

- Distribution Reinforcement

Area of distribution steel required = 20% of Ast

Let us use    mm diameter @ X mm Spacing

- Area of steal=π∅24……………………………………………………Eq 1

- Number of steel bar per meter width = 1m/X mm…………………….…Eq 2

- Area of steel per meter width = Eq 1 * Eq 2………………………………Eq 3

If the result under Eq.3 is greater than the required area of steel (Ast), then it is accepted.

Hence provide   mm diameter mild steel bar @ X mm c/c at right angle to the main reinforcement.

- Check for Shear

Nominal shear stress = V/ (bjd) ≤ Ԏa (Shear)

If nominal shear stress ≤ Ԏa (Shear) then it is Ok!

If nominal shear stress ≤ Ԏa (Shear) then it is Ok!

## Tertiary System Structures

Tertiary water control structures will comprise simple (reinforced concrete) box structures fitted with gates to control flows to earthen field channels. The gate width (span) of each tertiary box structure will be proportional to the offtaking discharge. Flow measurement structures are not envisaged within the tertiary units.

## Gates

To reduce costs, all gates sizes should be standardised. The following standard widths for cross(check) and head (offtake) regulator gates is recommended:

- Slide gates (single spindle): 0.25m, 0.30m; 0.35m, .

## Design of Intake Facility

Canal intake is installed near the under sluice gate in order to avoid entering of sediments into the main canals. The size and velocity of the intake may be determined as follows. Taking in to account the head work Topographic nature, the intake selected for this project is open channel of masonry wall for length of 4m pass through the wing walls on right riverbanks.

A net irrigable area to be irrigated by the Main Canal (MC) is 20ha. Based on the given water duty which is 0.73 lit/sec/ha (for 24 hours) the canal capacity is 15 lit/sec.

1. Open Channel

To design main canal intake concrete pipe,

Where,    A =flow area (m2)

V = Flow velocity m/s

A= b*d               b = canal bed width,           d = Full flow depth

As it can be seen from above, intakes of canal require about b=0.3m and D=0.3m is proposed for the intake.

## .  Proposed Water Distribution

As indicated in the canal design, main canal is designed to supply irrigation water for 24hours. One main canal irrigate on Right side of the head work (existing river course). No Tertiary canals are take off directly from the main canals while the secondary canals are supplied directly from main canal and the night storage reservoir. Therefore, Secondary canal will supply to other canals namely tertiary and field canals for 12 hours. Water distribution in tertiary and field canals will be made on rotational bases.

## Method of Water application

Topography nature of the area varies from place to place as shown on the topographic map. Based on topography furrow irrigation method is proposed.

- Furrow Structure

The furrows will be relatively shallow and wide considering the clay soils. The bottom width will be about 15–20 cm, the water depth will be 10–15 cm, and the surface water width will be 40–50 cm.

- Length of Furrows

Generally, the length of the furrow depends on several parameters, namely:

Soil type.

Slope.

Stream size.

Net depth of application.

Field size and cultivation practices.

In the case of the Tilo project, the factor that determines the length of furrows is the length of the field. The furrows run parallel to the TCs along the 100 m border of the block unit.  Therefore, the length of the furrows is 100 m and the other parameters must be determined so as to allow for acceptable irrigation efficiency and overall performance.

- Slope along the Furrows

The maximum recommended slope in heavy soils is 0.5%. More gentle slopes are used and recommended for the tilo project, and these can be determined during the design. However, because of the existing steeper slopes in the area and the need for all furrows to be parallel to each other, the slope of the furrows along each of the TCs will be determined by the existing topography.

In general, the slopes of the furrows will be in the range of 0.1–0.5%. Whatever the slope along the furrow, it is important that it be uniform.

- Stream Size

Determination of the stream size into the furrow is a difficult task since it should be in accordance with several parameters.

The stream size should fulfil all the following requirements:

Time to advance to the end of the furrow

Quantity supplied should be as per the GIWR.

Time of application, which determines the water quantity applied during irrigation and is dependent on soil infiltration.

The stream size is confined to the following minimum and maximum values:

Minimum stream size allows the flow to reach the end of the furrow within a reasonable time. Maximum stream size is the stream that does not create soil erosion.

- Net Water Application Depth

The net water application depth determines the necessary contact time of water and soil surface through the infiltration rate of the soil. The contact time must be compatible with the required time for the water application depth to infiltrate into the soil.

## Water distribution

The size of field unit and the furrow lengths are determined based on suitability of topography of the scheme. Water distribution will be rotational in the case of tertiary and field canals. However, in main and secondary canals the flow is continuous. Based on the size of each irrigation area, entire flow of the canal may be allowed to the farmer(s) for his /their allocated time. The time allocation will be in proportion to the land area of each farmer. The farmers themselves will manage internal distribution of water. To do this all beneficiaries will establish Water Users Associations voluntarily. Maximum flow through the tertiary canals is also based on the size of the irrigation land covered by each canal.

The furrow length depends on the farm size. However, on average, the length of each furrow is fixed to be about 80-100 m and its slope ranges from 0.05 to 0.025.

# HEAD WORK (DIVERSION WEIR) DESIGN

## Water Resource

Source of water for this project is Tilo spring, which is a perennial flowing. The total catchments area of the streams contributed to the project site is 6 sq. km. According to the hydrology study, the lean flow of Tilo spring at the study site is about 18l/s. The calculated water duty is 0.73/s/ha. The commendable land from the location head work is 20 ha.

### 3.1.1 Design Flood

- Estimated  discharge  through flood analyzing method

Finding of the hydrology study report shows that the peak design flood at Tilo  diversion weir site for  25,50 and 100 years of return period are 4.89, 6.05, and 7.2m3/sec. For the design of small-scale irrigation project (diversion weirs), it is recommended that the project design flood once in 50 years should be used (6.05m3/sec). For the design of drainage structure, the flood once in 5-25 years should be used.

- Estimated  discharge  from  actual field data

Bade upon the   parent river cross section at selected head work site, and longitudinal profile of the same river to the  maximum  level of flood mark, the  discharge obtained  was 6.0m3/sec as indicate in the figure 3-1 bellow. This calculated discharge of the catchment at maximum flood level collected on site is almost equal to the design flood estimated by hydrological data.

Figure 3-1: Stage discharge curve of Tilo stream

## Foundation Condition

Foundation condition at head work is the main criteria to design the diversion weir. The foundation material should provide sufficient resistance to differential settlement in preventing structural damage. If the foundation materials are non fractured rock, seepage and bearing failure is not the main problem. Such kind of foundation material does not need treatment except the removal of topsoil to a depth of 1.5m on both sides of the river banks and filling with selected materials if there is demand for compact fill.

To determine foundation bearing capacity and water tightness, Geotechnical investigation was carried out at the headwork site. The finding indicated that, the proposed headwork site characterized by transported bolder under lied by extended basaltic fractured rock. Thickness of such material is in the order of 1.5m.

## Purpose of Diversion Weir

Purpose of this diversion weir is designed to raise the water level on its upstream side to create necessary head, so that the desired irrigation water will be diverted in to the canals through the intakes, which are located at the upstream end of the main canal  on left  bank of the weir. The weir crest is also designed to function properly as a spillway for the flood flows.

## Design Considerations

### General

Diversion weir must designed in the same way as all hydraulic  structures  resisting against forces of Overturning ,sliding and  normal dead and live loads on foundations bearing capacity under the most severe conditions that may exist.

A weir structure is continuously acted on by the erosive action of flowing water, and since the structure is frequently placed under adverse condition, adequate safety factor should be considered.

### Selection of the head work site

During selecting the headwork location, the following important criterions were taken in to consideration.

The selected river cross-section is as much as possible narrow, straight, geologically sound and safe to put structure and the reach has central flow without tendency of scouring or silting;(ref. Fig.3.2)

The associated canal alignment should enable adequate command without excessive excavation or embankment.

With respective to the adjoining land surface, the elevation of the water surface upstream of the weir should not be so low as to require excessively high weir to divert the water at the intake.

- Broad crest Weir with one under sluice gate is selected for diverting the river water and commanding the irrigable area.

Selected weir Axis

Traditional diversion weir Axis

Figure 3-2 Head work site of Tilo/Gora River

### Types of Diversion weir

There are weirs of different types and shapes and most of them are constructed to serve the same purpose. In deciding shape or type of weir, two important factors need to be considered: the practicality/workability and the economy of the structures. Hence diversion weirs are designed to the skills of the persons who are expected to implement the structure and the designer should not impose a weir with a shape, which cannot be constructed easily by the local builders according to the given design. For this project a broad-crested masonry weir type with vertical upstream and sloped downstream face has been selected for this purpose.

### Weir crest elevation

The weir crest elevation affects the river water profile in two ways:

- The height of the crest affects the discharge coefficient and consequently the water head above the weir and the backwater curve;

- The height of the weir affects the shape and location of the jump and the design of the basin.

Height of the weir crest is decided based on the requirement of following criteria

  - The weir crest level is set so that, the water head required  to deliver water to the main canals with the design discharge  should  supply water to the heist command area;

  - During dry season, the entire flow of the river (low flows) will be diverted, the crest elevation must be set at a level so that the ponded water gives the required head supply  to the canal with the design flow;

  - The maximum upstream water surface elevation is also considered in selecting the crest elevation. The maximum water level depends on the upstream riverbanks elevations.

To determine the weir crest elevation, the following parameters are taken in to consideration.

Average level of the highest command area  =2956.5m

Water depth required                                       = 0.2m

Head loss across  the canal                               = 0.001* 789m = 0.79m

Head loss at the turn out                                   = 0.1m

Head loss at the head regulator                        = 0.2m

Required crest level of the weir                        = 2956.5+1.29  2957.79m

Freeboard for the weir crest                             = 0.00m

Based on the above criteria and the suitability of the site, the elevation of this weir crest was fixed to 2958.50 m. a.m.s.l.

### Length of the weir crest

Length of the weir crest depends on the physical features (width) of the river at the head work site. The effect of the weir length on the upstream water head and sedimentation behind the weir must be understood.

A weir with a longer crest gives a small discharge per unit length and hence the required energy dissipation per meter of the crest is smaller than what is needed for a shorter crest length;

Constructing a weir longer than the river width causes formation of islands at the upstream side of the weir and/or constricting the existing river course more than 80% will resulted with unit discharge and required longer stilling basin.

Generally, the crest length should be taken as the average wetted width during the flood season. During feasibility study, the upstream and downstream of the river cross-section has been examined, and the width at the selected weir site location is measured and found to be approximately 12 m.  Since the river bank at selected site is quite wider, the weir crest length including the under sluice gates was constricted to be 8 m.

Figure 3-3 Head work site River cross section

### Determination of flow depth over the weir crest

The weir is designed to pass the maximum design flood flows safely. The weir will have one under sluice gates. However, 20% of the entire maximum flood will pass safely though under sluice gate without causing any damage to structures and the land located upstream of the river reach.

Discharge over the weir is generally expressed as

Q = CLHe3/2

Where,

Q= Maximum design discharge = 2.06 m3/s

L =length of the weir = 5m

He= height of energy line above the crest = V2/2g + Hd

C = discharge coefficient, in practice a discharge coefficient of C =1.70 (for broad crested weir) is used.

Figure 3-4 Discharge curve over the weir crest

Approach velocity head is calculated as:

Where,

Q = Maximum Design discharge = 6.06m3/s

h = Height of the weir body above the upstream river bed = 0.5m

= Over flow depth without velocity head

L = length of the weir = 7.0m

From  by trial and error the value of Hd= 0.61m

In the same way:

Approach Velocity,

Head due to velocity,

Upstream water level =weir crest elevation + calculated over flow depth=2958.5+0.61= 2959.11m

### Basic Section of the weir body

Bligh’s method is used to determine the basic section of the weir body (the top and bottom width of the weir body) as follows.

Since the weir is to be constructed on impervious foundation, uplift force acts along the bottom of the weir body. Hence, specific weight of the weir becomes ()

Therefore, top width of the weir,

For stability case, take  B. 0.60

Bottom width of the weir,

For stability case, take L= 1.0m

Where,

B= Top width of the weir 0.6m

He = Overflow depth Hd + velocity head =0.64m

ρ = Specific weight of the weir body (ρ = 2 – 2.4), take ρ = 2.3

h = Height of the weir body above the upstream river bed = 0.50m (Masonry)

### Determination of Bottom level of the weir and stilling basin

Vertical cut-off wall   are always provided at the upstream and downstream ends of the weir for two reasons: to prevent the piping effects and scouring under the structure and to limit the intensity of uplift so that the stability of the structure will not be threaten. Intermediate cut-off walls provided between upstream and/or the downstream slopes of the impervious floor and are useful in holding the main structure, i.e. the weir. The depth of cut-offs should be such that its bottom is lower than the level of possible flood scouring depth at that section. In addition, the downstream cut-off wall should also be sufficient to reduce the exit gradient within safe limits, which is decided by the sub-surface conditions.

The normal scour depth below Highest Flood Level (HFL), R is given by the equation.

By Regime scour depth method,

Estimated value is 0.34 m, where 1 m practically enough and adopted for this particular project.

Where,

R = hydraulic mean depth

q = Discharge per meter length= Q/L = (6.05m3/s)/7m =0.86m2/s

f = is Lacey’s silt factor  approximately (dmm is the mean diameter of the bed material particles in mm) (ref. irrigation Engineering & Hydraulic structures, page 137 & 434). Mostly, the value of f is taken to be 1.

According to the geotechnical report, majority of the overlaying soil material is sandy silty. Diameter of this grain in mm is 0.125, therefore,

Reduced level of bottom of the weir body (R.L) = upstream high flood level -1.5R = 2959.11m –1.5*0.34 =2958.60m which is above the existing river bed, hence take the scouring depth 0.8m below the river bed elevation i.e.  2957.995-0.8=2957.1

It is recommended that the upstream cut-off wall should be at least 0.80m below the riverbed level R.L. To increase the creep length and to be safer, the bottom level of the downstream cutoff wall is fixed to 1.75R which is 1.75*0.34=0.595m below the d/s water level (TWL).  Hence, bottom level of d/s cutoff wall should be placed at 2958.54-0.595=2957.945 but for the safer condition take the d/s cut off wall 1.2m below the existing river bed i.e. 2956.795m.

#### Floor level of the stilling basin (apron)

The floor level of the downstream apron is fixed equal to the natural riverbed level, which is 2958.0m.

The difference between the upstream and downstream energy grade line becomes supercritical. However, the energy must be dissipated before it reaches the natural river course; otherwise, it causes damage to the banks and downstream of the apron. The flow over the weir is in a sub critical state. For this reason, the energy tends to dissipate through a hydraulic jump downstream of the weir. To control the location of the jump, the apron and stilling basin is designed to suit a range of the river discharge.

##### Hydraulic jump on the horizontal surface

On the downstream horizontal apron of the weir, the following equation is used.

Where, q = Discharge per meter width = 6.05m3/m =0.86m3/sec,

Based on energy of flow, the hydraulic jump (pri- and post jump) occurring at the stilling basin can be calculated as follows.

Total energy level at Upstream = Total energy level at downstream Eo = E1

Where, Zo = Upstream bed level = 2858m

h = Weir height above the ground =0.50m

Hd= Over flow Depth overt the weir =0.61m

Va = Approach velocity = 0.78m/s

Z1 = Downstream apron level =2857.90m

D1 = Pre-jump depth

V1 = Velocity at the jump = q/D1, where q= 0.86m3/s

By substituting the value of V1, the equation will be reduced to

2958. 0+ 0.5 + 0.61 + 0.03 =2858.0+ D1 + (0.86/D1)2 *(1/2*9.81)

2959.14 -2958= D1+0.74/19.62D12

By trial and error the value of D1= 0.19

Pre-jump water level = 2958.0+0.19=2958.19m

Froude number,

Where,

D2 = Post-jump depth

V2 = Post-jump velocity

Post-jump water level = 2958.0 + 0.79 =2958.79m

### Length of Stilling Basin

An apron may be placed upstream of the weir in conjunction with one of the various types of cut-off wall. The function of the upstream apron is to increase the length of percolation path in order to avoid piping and to reduce uplift acting over the entire base area. The d/s apron and d/s cut-off wall is enough to control the piping problem, in this design; 1.5 m upstream apron length is proposed.

Downstream apron has two functions. The first is to lengthen the path of percolation and the second is to dissipate energy. Therefore, the properly designed stilling basin and energy dissipater must be provided at the downstream end of the weir.

The length of the downstream horizontal floor should be such that the entire jump is confined only to the floor. This will ensure that the stone protection provided on the downstream of the floor is not affected adversely by the jump. Hence, the length of the downstream horizontal floor is kept equal to the length of the jump, which is equal to 5 to 6 times the height of the jump i.e. 5 to 6 (d2-d1). Here d2 and d1 are pre-jump and post-jump depths of flow.

(Ref. G.L. Asawa “Irrigation Engineering” page 405 and Bharat Singh “Fundamentals of irrigation engineering” ninth edition on page 330)

Therefore,

The implication of the depressed stilling basin is that the energy dissipation is completed in the basin and the high sequent depth D2 level will not exceed the predetermined tail water level of 2958.79 a.m.s.l. Thus, the estimated tail water depth (D3) will be maintained at 0.75m.

### Thickness of downstream apron

There are two approaches to determine the apron thickness. The first assumes that the apron consists of individual unit volumes, which are structurally not linked, and the weight of each individual unit balances the uplift pressure. While this assumption leads to an increase in the structure’s cost, the computations involved are very easy and result in a structure with a high safety factor.

The second approach to design the apron is by considering the whole structure as one unit and determining the bending moment and shear force at the critical section, which is at the toe of the weir.

To determine the thickness of the apron both dynamic and static case should be considered. The lower parts of the apron will generally require larger thickness when static case is selected, but the upper part of the apron (the toe section) will have larger thickness when dynamic case is considered. Therefore, the thickness at any point say A, B and C is calculated from

Where,

t = thickness of apron at any point (m)

f = factor of safety = 1.3

H max = U/S and D/S head differential (maximum) head, m

Lc= weighted creep length total (m),

Lc= 0.8+0.3+0.3+0.5+1.1+1/3 (1.0+1+2.50+0.5+)

Lc= 3.0 + 1.67= 4.67

LA = Weighted creep length at point A (m)

TWL= tail water level (m)

WLA = water level at point A

=Specific gravity of masonry = 2 to 2.3 (take = 2.3)

C = Percolation coefficient (Lane’s creep ratio), the value of C for sand grains is 5 to 7

Case-1 Dynamic Case or high flood condition (Hmax =0.35)

LcCHmax=6.00.35*6= 62.1 safe against piping (seepage)

Case-2 Static Case or no over flow condition (Hmax =0.5)

LcCHmax=60.5*6= 6 3.0, safe against piping (seepage)

Figure 3-5 :-Free diagram of energy profile at dynamic case

Table 3-1: Thickness of downstream apron floor

| Point | Weighted creep length |  |  | Hmax(1-LA/LC) | TWL-WLA | f/(m-1) |  | t (m) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | LH | LV | Lc |  |  |  |  |  |  |  |
| Dynamic Case Hmax = |  |  | 0.44 |  |  |  | Calcu. |  | Adopted |  |
| A | 0.77 | 1.6 | 4.87 | 0.226 | 0.41 | 1 | 0.636 |  | 0.55 |  |
| B | 1.17 | 1.6 | 4.87 | 0.190 | 0 | 1 | 0.190 |  | 0.1 |  |
| C | 1.50 | 1.6 | 4.87 | 0.160 | 0 | 1 | 0.160 |  | 0.1 |  |
| Static Case Hmax = |  | 0.5 |  |  |  |  |  |  |  |  |
| A | 0.77 | 1.6 | 4.87 | 0.257 | 0 | 1 | 0.257 |  | 0.3 |  |
| B | 1.17 | 1.6 | 4.87 | 0.216 | 0 | 1 | 0.216 |  | 0.25 |  |
| C | 1.50 | 1.6 | 4.87 | 0.182 | 0 | 1 | 0.182 |  | 0.2 |  |

At the toe of the weir, the term (TWL-WLA) should be reduced by 50% and becomes

Check the thickness of each point

Point A 0.65  0.30m Ok (Dynamic case)

Point B 0.25 0.1m Ok (Static case)

Point C both values are very small so take o.5m safety.

According to Bligh (Santosh Kumar Garg “Irrigation eng& hydraulic structures” page 482), he apron thickness can also be calculated as is the submerged specific gravity of the floor material h =Difference in elevation

Hence, the thickness of the floor can be easily determined by using this equation. The value should be increased by 33% to allow a suitable factor of safety. The formula will be

### . Khosla’s Exit Gradient (GE)

If the upward thrust exceeds a certain value at the exit, piping will occur. It has been determined that for a standard form consisting of floor length b and vertical cutoff of d, the exit gradient at the downstream end is given by, (Khosla’s Method).

Where,

Hmax = head difference between u/s water level and tail water level (maximum) head =2958.5-2958=0.5m

From the above result, it is safe against piping, since GE = 0.08 is lower than safe exit gradient. For bed material consisting of coarse sand or gravel including sand grains, safe exit gradient is in the range of 0.20 to 0.17. The provided cut-off wall of 1.2m is enough to safeguard against piping.

## Structural Analysis of the Weir Body

All external forces acting on the weir are the result of flowing water in the river on which the structure is constructed. A typical force system of a weir consists of the following components.

- Static water pressure of the surface water: Its value can be easily calculated if the effect of changing the static pressure, upstream, to the dynamic one downstream is neglected.

- Uplift water pressure: Uplift pressure caused by water percolating under or along the sides of hydraulic structures. It reduces the  effective weight  of  a structure  and  are  therefore particularly  significant in  the stability  analysis. Stilling basin floor, for example, is subjected to the uplift pressure.

- Friction forces at the base which develop to balance the horizontal forces

- Weight of the weir and water wedges: Weight of the weir is calculated simply by multiplying unit weight of the weir by its volume. Weight of the water wedges present the weight of water that is on the weir body and act either against or in favour of the weir stability, it depends on the slope of the weir and the water surface at d/s.

Since this weir is to be constructed monolithically (as one integrated unit with the apron), the structure is more stable and it would not fail because of unbalanced moments. However, the stability analysis is done as follows.

Figure 3-6. Free diagram of force acting in dynamic case

Table 3-2Stability analysis for high flood condition (dynamic case)

| Name of forces | Symbol | Description | Forces (KN) |  | Lever arm | Moments (KN-m) |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Vertical (+) | Horizontal (-) |  | (+) | (-) |
| 1.Vertical forces |  |  |  |  |  |  |  |
| 1.1 down ward |  |  |  |  |  |  |  |
| Weight of weir | W1 |  | 0.24 |  | 0.61 | 0.14 |  |
|  | W2 |  | 0.09 |  | 0.15 | 0.01 |  |
|  | W3 |  | 0.24 |  | 0.63 | 0.15 |  |
|  | W4 |  | 0.19 |  | 0.15 | 0.03 |  |
|  | W5 |  | 0.03 |  | 0.10 | 0.00 |  |
|  | W6 |  | 0.03 |  | 0.38 | 0.01 |  |
| Subtotal |  |  | 0.80 |  |  | 0.35 |  |
| weight of water | Ph2 |  | 0.065 |  | 0.61 | 0.04 |  |
|  | Ph3 |  | 0.032 |  | 0.15 | 0.005 |  |
| Subtotal | ΣWw |  | 0.1 |  |  | 0.045 |  |
| Subtotal of (1) |  |  | 0.90 |  |  | 0.40 |  |
| 1.2 Upward forces |  |  |  |  |  |  |  |
| uplift | Pu1 |  |  | -0.03 | 0.39 |  | -0.01 |
|  | Pu2 |  |  | -0.10 | 0.52 |  | -0.05 |
| Subtotal | Σpu |  |  | -0.13 |  |  | -0.06 |
| Summation ΣV & ΣM |  |  | 0.77 |  |  | 0.34 |  |
| 2. Horizontal force |  |  |  |  |  |  |  |
| 2.1 u/s water pressure |  |  |  |  |  |  |  |
|  | P1 |  |  | -0.250 | 0.17 |  | -0.04 |
| Summation ΣH & ΣM |  |  |  | -0.25 |  |  | -0.04 |
| Total |  |  | 0.77 |  |  | 0.34 | -0.1 |

##### Take:-

Density of masonry, m = 2.2 ton/m3 (for u/s weir face 0.50m thick and d/s cut off wall)

Density of concrete, c = 2.4 ton/m3 (for apron and weir body)

Density of water w = 1 ton/m3

Material bulk density (submerged) = 1.20mg/cm3. This value is taken for soil upstream of the weir body, Specific density will be =1.2

Coefficient of uplift force,  = 0.67

Coefficient of silt pressure, Co = 0.57

Fv = Resultant of vertical forces = Sum of all vertical forces = 0.77 ton

FH = Resultant of Horizontal forces = Sum of all horizontal forces =0.25ton

- Sliding

, safe from sliding

Where,  is coefficient of friction between two surfaces (foundation and concrete), =0.65 to 0.75. Take the minimum value of  =0.65

- Over-turning

If the resultant of all forces acting on the weir body at any of its section passes outside the toe, the weir shall rotate and over turn about the toe. The ratio of the righting moments about the toe (anti-clock wise) to the over turning moments about the toe (clock-wise) should be greater than 1.5

, safe against over turning

- Tension

To avoid tension at the base, the forces must pass through the middle third of the structure base. Therefore, the following condition should be satisfied.

Eccentricity,

Where, e = eccentricity of the resultant force from the center of the base

Therefore,

Where,

Total weir length L = 0.8m

Fv=0.77ton

Safe from tension

- Overstressing

According to the engineering geology report, the allowable bearing pressure of the bed materials is < 2.00 kgf/cm2 / tontf/ft2. Therefore, 0.154kgf/cm2 is far less than the indicated value. Hence, it is safe from overstress.

Table 3-3 Stability analysis for no over flow condition Static case

| Name of forces | Symbol | Description | Forces (KN) |  | Lever arm | Moments (KN-m) |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Vertical (+) | Horizontal (-) |  | (+) | (-) |
| 1.Vertical forces |  |  |  |  |  |  |  |
| 1.1 down ward |  |  |  |  |  |  |  |
| Weight of weir | W1 |  | 0.24 |  | 0.61 | 0.14 |  |
|  | W2 |  | 0.09 |  | 0.15 | 0.01 |  |
|  | W3 |  | 0.24 |  | 0.63 | 0.15 |  |
|  | W4 |  | 0.19 |  | 0.15 | 0.03 |  |
|  | W5 |  | 0.03 |  | 0.10 | 0.00 |  |
|  | W6 |  | 0.03 |  | 0.38 | 0.01 |  |
| Subtotal |  |  | 0.80 |  |  | 0.35 |  |
| 1.2 Upward forces |  |  |  |  |  |  |  |
| uplift | Pu1 |  |  | -0.03 | 0.39 |  | -0.01 |
|  | Pu2 |  |  | -0.10 | 0.52 |  | -0.05 |
| Subtotal | Σpu |  |  | -0.13 |  |  | -0.06 |
| Summation ΣV & ΣM |  |  | 0.67 |  |  | 0.29 |  |
| 2. Horizontal force |  |  |  |  |  |  |  |
| 2.1 u/s water pressure |  |  |  |  |  |  |  |
|  | P1 |  |  | -0.250 | 0.17 |  | -0.04 |
| Summation ΣH & ΣM |  |  |  | -0.25 |  |  | -0.04 |
| Total |  |  | 0.67 |  |  | 0.29 | -0.1 |

- Sliding

, safe from sliding

Where,  is coefficient of friction between two surfaces (foundation and concrete), =0.65 to 0.75. Take the minimum value of  =0.65

- Over-turning

If the resultant of all forces acting on the weir body at any of its section passes outside the toe, the weir shall rotate and over turn about the toe. The ratio of the righting moments about the toe (anti-clock wise) to the over turning moments about the toe (clock-wise) should be greater than 1.5

, safe against over turning

- Tension

To avoid tension at the base, the forces must pass through the middle third of the structure base. Therefore, the following condition should be satisfied.

Eccentricity,

Where, e = eccentricity of the resultant force from the center of the base

Therefore,

Where,

Total weir length L = 1.0m

Fv=0.67ton

Safe from tension

- Overstressing

According to the engineering geology report, the allowable bearing pressure of the bed materials is < 2.00 kgf/cm2 / tontf/ft2. Therefore, 0.134kgf/cm2 is far less than the indicated value. Hence, it is safe from overstress.

## Wing Walls

The main functions of the wing walls are to anchor the weir structure into the riverbanks and to prevent lateral seepage through the riverbanks, which might occur due to the difference in water level between u/s and d/s of the structure. The wining walls will have the same length to that of diversion structure and it was fixed with anchorage in the riverbanks. The wing walls have at least a freeboard of 0.30m over the level of the discharge for which the diversion structure has been calculated. Wing walls of the diversion and was designed to be constructed from dressed masonry.

According to the Engineering Geology finding, both banks of the river at the weir location have stable strata. The wing walls selected for this  project is masonry wall. The analysis and design of wing wall would be treated as the same of retaining wall design.

Top level of the upstream wining walls = Weir crest elevation + over flow depth on the weir crest + freeboard =2958.5+0.61+0.30 = 2959.41m (0.30m freeboard is added).  The calculated height of upstream wing wall is 1.50m.

Top level of downstream wining walls = downstream apron level +post jump height (tail water depth) + freeboard =2958.0+0.79+0.30= 2959.09m. The calculated height of downstream wing wall would be 1.20 m.

To decide bottom level of the upstream wining walls, take the scouring depth calculated to decided u/s and d/s cut off walls   for high Flood level (HFL) respectively and  their Bottom level of the upstream and downstream wing walls, RL = 2957.4m

### Active Earth Pressure against vertical walls

Rankine’s theory

Intensity of Active Earth Pressure is given as follows

Pa = Kah

Total earth pressure on the retaining wall

P =1/2 Kah2

The pressure p is acting at the height of h/3 above the apron =1.50/3 =0.5m

Where,

h= depth at which the active earth pressure is calculated =1.50m

=specific weight of soil = 1.80 ton/m3, bulk density of soil =1.80mg/cm3 =1800kg/m3

ka = coefficient of active earth pressure,

= angle of internal friction of the soil (angle of repose), for saturated soil =270

The base width of the wing walls (b) is taken as h/2.5 to h/1.5 times the height of retaining wall.

Take b = 1.5/2.=0.75m. Stability analysis of wing wall

|  |  | 2958.08 |  |  |
| --- | --- | --- | --- | --- |
|  |  | 0.4 |  |  |
|  | 2.1 |  |  |  |
|  | WP | W1 | W1 |  |
|  | 2957.40 | 0.8m |  |  |
|  |  | W3 |  |  |

Figure 3-7 Schematic diagram of wing wall

Note: Density of masonry, m =2.2ton/m3

Density of soil, s =1.80 ton/m3

Table 3-4 Stability analysis for retaining wall  active earth pressure

### Stability condition

- Check against over turning

,safe against overturning for u/s

, safe against overturning for d/s

- Check against sliding

, safe against sliding

=Sum of Vertical loads = 2.0 ton/m

P = horizontal Force = 0.7

= Coefficient of friction = 0.65 to 0.75, take the minimum value of  = 0.65

- Check against base pressure (over stressing)

After finding out the eccentric e, the maximum and minimum normal stress intensities at two ends (toe & heel) can be calculated from formula (Irrigation Eng. & Hydraulic Structures, page 101-104)

Eccentricity,

e = eccentricity of the resultant force

Where

Where,

= lever arm of the resultant force

Normal stress at the toe of the retaining wall is given by

(For densely consolidated clay soil, the allowable bearing capacity = 25 ton/m2)

Normal stress at the heel of the retaining wall is given by

# WUA INSTITUTIONAL DEVELOPMENT AND SUSTAINABLE OPERATION AND MAINTENANCE

## Introduction

To achieve sustainable agricultural benefits appropriate farmer institutions need to be formed and supported to take over O&M of the engineering infrastructure. The following is therefore proposed from the start of the project and continuing into the support phase, years1

- Election, establishment and capacity building of farmer WUA institutions, vis WUGs, WUAs and a Scheme Board at tertiary, secondary and scheme level respectively;

- Support for system operation & maintenance; and

- Business planning (with WUAs and / cooperatives), establishment of cost recovery.

## WUA Formation and Capacity Building (O&M-1)

To manage: (i) construction of tertiary system infrastructure during the implementation phase; and (ii) for management, operation and maintenance of the completed irrigation and drainage Infrastructure, suitable non-profit farmer institutions are required with compulsory farmer membership based on hydraulic (not village) boundaries. All farmers with a land holding in the command area would be members. Cooperatives exist in the scheme area but are (usually) village-based, with voluntary membership, and profit oriented. They are not suitable institutions operation and maintenance of the completed irrigation and drainage infrastructure.

New institutions are therefore required and will comprise: (i) informal Water User Groups (WUGs) formed at tertiary system level; (ii) formal (i.e. with legal status) Water user Associations (WUAs) at Secondary level representing all farmers within their secondary canal command, collectively referred to as the General Body; and (iii) a Scheme Board at Main system level representing all WUAs and with oversight over the Main System Operator.

The new farmer institutions will lack the knowledge and skills to carry out their O&M responsibilities so this capacity will have to be developed. Also guidelines and the legal framework for WUAs (and the Scheme Board) currently do not exist and need to be formulated.

Sub-project objectives are therefore:

- The formation of WUGs  and the Scheme Board able to take a proactive role in scheme development, and by year 1 to take full responsibility for  scheme operation and maintenance at all levels, from the Head work station down to the field.

- To support development of guidelines and a legal framework (including rules and regulations) for the new farmer institutions.

Sub-project activities include:

- Formation of WUGs, WUA and Scheme Board including: (i) awareness generation through an IEC campaign & meetings; (ii) preparation and use of mass media; and (iii) holding of elections for the WUA committees and the Scheme Board.

- Training and capacity building for the General Body (i.e. all farmer members) in,for example, (i) formation and structure of the new institutions; (ii) their purpose and function; (iii) roles and responsibilities the institutions and their committees;(iv)holding elections; etc.

- Building awareness among the WUG & WUA members of other (non-O&M) subproject interventions that are planned and implemented through P.A & village centred institutions including the P.A. council and cooperative societies.

- Specific training and capacity building for (elected) members of: (i) the WUA Management Committee; (ii) the WUA Audit Committee; (iii) WUA standing committees concerned with O&M; and (iv) Scheme Board. This could include, for example, book and record keeping and setting up and management of bank accounts.

#### These O&M subprojects are described below. Subproject description sheets are included as

The main objective of the operation and maintenance plan of an irrigation scheme is to facilitate the timely delivery of the required irrigation water to farms and to keep the irrigation system in an optimum operating condition. This section therefore, discusses the main functions of the operation and maintenance plan of the irrigation scheme.

## System Operation (O&M-2)

In line with government policy farmer beneficiaries through their O&M institutions, the WUGs at tertiary level, WUAs at secondary level, and Scheme Board at Head work and Main canal level will become responsible for system operation after completion of the scheme.

Although Tilo is a upgrading of the existing traditional irrigation development farmers do not currently have experience in managing and operating an irrigation scheme to achieve full crop production potential. The farmer institutions and capacity therefore has to be developed.

Sub-project objectives are to build up farmer (WUG, WUA and Scheme Board) capacity within one year to undertake irrigation management and operation achieving: (i) reliable, transparent and equitable water distribution; (ii) operation and water application that meets crop water requirements with minimal water losses; (iii) minimal damage to unlined channels and other infrastructure; (iv) financial self sufficiency by raising funds to meet full operation costs including.

The sub-project will assist in the development of the operating strategy / procedures / guidelines for the project (to be drafted by the project consultants / OIDA office staff) covering: (i) main system operation; (ii) secondary and night storage reservoir operation; (iii) tertiary system operation and flow rotation including efficient water application by furrows and /basins; (iv) crop water requirements, crop yield and irrigation scheduling; (v) seasonal crop planning including crop types, water application method (furrow / basin) and planting dates enabling preparation of distribution schedules; (vi) role and duties of WUG /WUA Water Masters to oversee flow rotations and water distribution; (vii) management of system infrastructure particularly regulators and flow measurement structures; and (viii) practical ways to assess soil moisture and need for irrigation.

Sub-project activities include capacity building and training to achieve:

- Clarity and consensus to the operating procedures / guidelines prepared for the project among WUA members.

- Agreement of water charges if any to be levied on WUA members as well as how funds should be collected and action to be taken against defaulters.

- Appointment of competent water masters able and willing to carry out their O&M duties at tertiary, secondary and main system level.

- Appointment of competent head regulator  operators enable to manage O&M of the head work  station operation, and avoiding withdrawing  too much water or too little.

The sub-project will also support comprehensive training / capacity building in operating procedures / guidelines for: (i) the Water Masters appointed at tertiary , secondary  and main system  level; (ii)  members of WUA standing committees for system O&M as well as the System Board.

Implementation would start from year1 (implementation phase) and continue through to year 2 (i.e. one years after completion of infrastructure construction).

## System Maintenance (O&M-3)

However, Tilo is a new irrigation development farmers do not currently appreciate the maintenance needs, activities and costs to keep the engineering infrastructure in good working order. If maintenance is not carried out, or frequently deferred, the scheme will deteriorate resulting in ability to perform as designed and crop loss. This subproject therefore needs to build awareness of maintenance requirements, support development of a maintenance strategy with broad approval by farmers, and develop the capacity to implement the strategy.

Sub-project objectives are to build up farmer (WUG, WUA and Scheme Board) capacity within two years to undertake system maintenance is a timely and sustainable manner so that the constructed infrastructure performs as designed for its design life (30-50 years).

Subproject activities include:

- Support for development of a maintenance strategy / procedures / guidelines for the scheme covering: (i) asset inventory; (ii) maintenance categories, requirements and costs (possibly introducing yardsticks for maintenance cost estimation); (iii) maintenance cycle including preparation and implementation of annual maintenance plans; (iv) maintenance funding; and (v) maintenance mitigation (e.g. by carrying out maintenance in timely manner).

- Building general body awareness of the importance of maintenance and its funding.

- Capacity building of concerned WUA standing committee members and Scheme Board members including dissemination of the maintenance strategy / procedures /guidelines.

- Specific and rigorous training (formal and on-the-job) for those selected by the WUAs and Scheme Board to supervise / implement maintenance including to the water masters concerned with O&M at tertiary , secondary and main system level.

- Support to farmers (WUG and WUAs) to implement tertiary system construction works during the implementation phase, including: (i) tertiary canals and drains; (ii) tertiary system structures including concrete division boxes and drops (note: steel shutters to be provided to the WUGs / WUAs).

- Mentoring of WUGs / WUAs / Scheme Board as well for the water masters and Head regulator station operators in carrying out maintenance, initially focussing on (labour intensive) routine maintenance to canals and drains, and gradually building capacity to enable them to take over full system maintenance, including head work station, by the end of the support phase (year 2) when the main system operator will leave site handing over its duties and responsibilities to the Scheme Board.

- Provision of tools, equipment and materials for maintenance.

Implementation would start from year 1 (implementation phase) and continue through to year 2(ie one years after completion of infrastructure construction).

Structures and all canals are subject to deterioration due to wear and tear through the life of the project. Regular maintenance is, therefore, required to reduce early deterioration. Problems of weed growth and siltation will damage the proper operation of the scheme.

Therefore frequent inspection followed by regular maintenance must be carried out by the Water Users Association that means removing of weed and lubrication of head regulators, scour sluices and off take gates at the beginning and end of the irrigation season.

## Business Planning, Management Information System and Cost Recovery & M.4)

This subproject includes two distinct components:

- Support for village institutions, particularly cooperatives, to development (annual)micro-plans for crop, livestock, community development and environmental enhancement activities;

- Support for a project management information system and for cost recovery imparted to the WUA and scheme board.

(a) Business / Micro Planning

The project includes an array of agricultural, livestock, community development and environmental subprojects (interventions) considered necessary to achieve maximum and sustainable benefits, to ensure that the poor and disadvantaged benefit and to enhance the environment. These will be implemented through village based institutions including the P.A council and various village based cooperatives. This subproject will facilitate planning and implementation of subprojects by: (i) building awareness of the range of subprojects; (ii) assist in preparation of annual micro-plans / business plans with village communities; and (iii)enhance capacity of these village institutions to implement the micro- / business plans.

(b) Management Information System and Cost Recovery

The WUAs / scheme board will take over management, operation and maintenance of the completed scheme in year2, and by that time will need to have adopted a management information system to: (i) provide timely up-to-date O&M information; and (ii) control financial income and expenditure. Design, transfer to the Scheme Board and training of 1-2 operators of a modular computer based MIS will be financed under this subproject, along with necessary hardware. It is envisaged the MIS will comprise the following modules:

- Administrative module concerned with WUA and Scheme board membership and names and contract details for Water Masters  operators;

- Operations module concerned with irrigation flows, equity of distribution, etc.

- Financial module concerned with records of income and expenditure.

# PROJECT IMPLIMENTATION APPROACHES AND PROGRAM

## Roles and Responsibilities of Key stakeholders

The Executive Agency of Tilo Small Scale Irrigation Project is the Oromia Irrigation Development Authority in collaboration with the users. The Authority will, provide the required construction fund, awards the construction contract and will carry out the overall project implementation in collaboration with the districts Administration and Districts and  zonal and Irrigation Development  authority Offices.

The main focus of the project activities is at the district level and particularly at the irrigation scheme. The day to day implementation of project activities is entrusted to ODIA assisted by the Zonal Project Officer and district Administration and concerned Government Officers. The WUA Supervision Committee will be established on the site and trained in supervising the works which also assists the Project office in supervising the construction works.

The staffs of the OIDA are the technical personnel who deal directly with farmers concerning training and technical advice.

## Overall Implementation Program

The implementation of the scheme involves two phases, scheme construction, and scheme operation. The smooth implantation of the earth marked programs during these phases heavily depends on the commitment of all stakeholders in carrying out their responsibilities in accordance with the agreed program and share of responsibilities.

## Scheme Construction Program

The indicative construction program of the scheme has been prepared based on the scheme components, which involve headwork and on farm works construction. The latter include construction of 800m lined main canal, 750m earthen secondary and 900m tertiary canals. The irrigation, drainage structures and miscellaneous works include two road crossing culvert, 2 foot path, 18 Drop Structures, one drainage crossing culverts, 6 Turnout Structures and 10 m long dyke both at left and right of river bank.

The construction program has been shown in Figure 5-1 and include, tendering and awarding period of half month. This is followed by a mobilization time of half month. The construction of the scheme is planned to be carried out in two sections, Section I, Headwork’s, and Section II On farm Works, to facilitate accepting the works from the contractor. The actual construction of the scheme is aimed at completion within 5 months following mobilization.  The program is aimed at completion of works during a whole dry season, such that the disruption of irrigation should be limited to one season only.

| - Table of Contents Type chapter title (level 1)1 Type chapter title (level 2)2 Type chapter title (level 3)3 Type chapter title (level 1)4 Type chapter title (level 2)5 Type chapter title (level 3)6 Activities | Unit | Quantity | M1 |  |  |  | M2 |  |  |  | M3 |  |  |  | M4 |  |  |  | M5 |  |  |  | M6 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 1 | 2 | 3 | 4 | 1 | 2 | 3 | 4 | 1 | 2 | 3 | 4 | 1 | 2 | 3 | 4 | 1 | 2 | 3 | 4 | 1 | 2 | 3 | 4 |  |
| Tendering and Awarding |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Main Scheme structures Components |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Section I |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Mobilization |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Headwork |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Diversion weir | No | 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Section II |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| On farm works |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Main, Tertiary Canals and Drains | Km | 3.5 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Irrigation and Drainage Structures | No | 30 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 5- 1:- Scheme construction program

\

# PROJECT COST ESTIMATE AND UNIT RATE ANALYSIS

## Costs of the Project

Cost estimates of the project have been derived for the scheme works based on the detailed Bills of Quantities built from the detail design drawings of the project components and construction rates collected from the current construction market. The total costs including contingencies are summarised in Table 6-1.

Table 6-1:- Summary of   project Cost estimation

| Item | Amount Birr |
| --- | --- |
| General | 347,388.91 |
| Head work | 169,313.14 |
| Infrastructure( Farm Works ) | 1,394,302.48 |
| Grand Total | 1,911,004.52 |
| Vat 15% | 286,650.68 |
| Project cost | 2,197,655.20 |

Table 6-2:- Detail Bill of Quantity and Cost estimation

| NO | Description of work |  | Unit | Quantity | Unit cost | Total cost |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | Camping & Mobilization |  |  |  |  |  |
| 1.1 | Mobilization and Demobilization |  | LS | 1 | 150,000.00 | 150,000.00 |
| 1.2 | Camping |  |  |  |  |  |
| 1.2.1 | Site clearance |  | m2 | 112 | 38.64 | 4,327.94 |
| 1.2.2 | Foundation excavation |  | m3 | 20 | 136.54 | 2,730.73 |
| 1.2.3 | Masonry |  | m3 | 14 | 1,220.27 | 17,083.80 |
| 1.2.4 | 20cm thick hard core |  | m3 | 20 | 528.30 | 10,566.00 |
| 1.2.5 | 10cm thick lean concrete(1:3:6) |  | m3 | 7 | 1,769.83 | 12,388.83 |
| 1.2.6 | Cemented screed |  | m2 | 90 | 162.63 | 14,636.66 |
| 1.2.7 | GIS waling G-32 |  | m2 | 250 | 258.58 | 64,645.73 |
| 1.2.8 | GIS roofing G-32 |  | m2 | 150 | 313.39 | 47,009.21 |
| 1.2.9 | Fencing (1.5m spacing of Di. 12cm wooden pole and barbed wire ) |  | m | 160 | 150.00 | 24,000.00 |
| General Item Sub Total 1 |  |  |  |  |  | 347,388.91 |
| 2 | Diversion Weir |  |  |  |  |  |
| 2.1 | Earth work |  |  |  |  |  |
| 2.1.1 | Site clearing |  | m2 | 80 | 38.64 | 3,091.39 |
| 2.1.2 | Excavation of clay soil |  | m3 | 30 | 136.54 | 4,096.09 |
| 2.1.3 | Excavation of soft rock/boulders |  | m3 | 20 | 344.67 | 6,893.48 |
| 2.1.4 | Excavation of Hard rock |  | m3 | 25 | 729.35 | 18,233.71 |
| 2.1.5 | Back file & compaction for both side of weir /wing walls dykes |  | m3 | 35 | 103.44 | 3,620.51 |
| 2.2 | Structure |  |  |  |  |  |
| 2.2.1 | Masonry works of mix ratio 1:3 |  | m3 | 70 | 1,220.27 | 85,419.02 |
| 2.2.2 | Mass concrete 1:2:4 |  | m3 | 12 | 2,247.35 | 26,968.25 |
| 2.2.3 | Hard coring |  | m3 | 5 | 528.30 | 2,641.50 |
| 2.2.4 | pointing |  | m2 | 20 | 95.10 | 1,902.00 |
| 2.2.5 | Plastering, mix ratio 1:3 |  | m2 | 50 | 108.94 | 5,447.19 |
| 2.3 | Gate fixing |  |  |  |  |  |
| 2.3.1 | Intake gate supply and installation (0.3m*0.6m*4mm) |  | no | 1 | 5,000.00 | 5,000.00 |
| 2.3.2 | shatter gate (0.6*0.8*4mm) for Under sluice |  | no | 1 | 6,000.00 | 6,000.00 |
| Headwork Sub Total 2 |  |  |  |  |  | 169,313.14 |
| 3 |  | Irrigation and drainage Structures |  |  |  |  |
| 3.1 |  | Lined Main canal for length of 800m |  |  |  |  |
| 3.1.1 |  | Earth work |  |  |  |  |
| 3.1.1.1 |  | Site clearing | m2 | 1000 | 38.64 | 38,642.36 |
| 3.1.1.2 |  | Excavation for soil | m3 | 500 | 136.54 | 68,268.16 |
| 3.1.1.3 |  | Excavation for soft rock | m3 | 50 | 344.67 | 17,233.71 |
| 3.1.1.4 |  | Compact fill | m3 | 25.00 | 103.44 | 2,586.08 |
| 3.1.2 |  | Structure |  |  |  |  |
| 3.1.2.1 |  | Masonry works of mix ratio 1:3 | m3 | 155.00 | 1,220.27 | 189,142.12 |
| 3.1.2.2 |  | Plastering, mix ratio 1:3 | m2 | 960 | 108.94 | 104,586.00 |
| 3.1.2.3 |  | Hard coring well dressed in cement mortar | m3 | 160.00 | 528.30 | 84,528.00 |
| 3.1.2.4 |  | Lean concrete | m3 | 15.00 | 1,769.83 | 26,547.50 |
| Sub Total |  |  |  |  |  | 531,533.93 |
| 3.2 | Earthen canals |  |  |  |  |  |
| 3.2.1 | Earthen Secondary Canal 700m |  |  |  |  |  |
| 3.2.1.1 | Site clearing |  | m2 | 1150 | 38.64 | 44,438.71 |
| 3.2.1.2 | Excavation of ordinary soil for main canal |  | m3 | 450 | 136.54 | 61,441.35 |
| 3.2.1.3 | Excavation of soft rock for main canal |  | m3 | 25 | 344.67 | 8,616.85 |
| 3.2.1.5 | Compact fill |  | m3 | 10 | 103.44 | 1,034.43 |
| Sub Total |  |  |  |  |  | 115,531.34 |
| 3.3 | Drainage crossing box culvert |  | No | 1 |  |  |
| 3.3.1 | excavation of clay soil |  | m3 | 10 | 136.5 | 1,365.36 |
| 3.3.2 | Hard coring |  | m3 | 5 | 528.3 | 2,641.50 |
| 3.3.3 | stone pitching |  | m2 | 6 | 750.0 | 4,500.00 |
| 3.3.4 | Concrete works of mix ratio 1:2:4 |  | m3 | 3 | 2247.4 | 6,742.06 |
| 3.3.5 | Plastering, mix ratio 1:3 |  | m2 | 15 | 108.9 | 1,634.16 |
| 3.3.6 | reinforcement bar : -Ø8mm |  | kg | 50 | 34.2 | 1,710.16 |
| 3.3.7 | reinforcement bar : -Ø14mm |  | kg | 100 | 34.2 | 3,420.31 |
| 3.3.8 | black wire |  | kg | 3 | 40.0 | 120.00 |
| 3.3.9 | form work |  | m2 | 6 | 351.5 | 2,108.92 |
| Sub Total |  |  |  |  |  | 24,242.47 |
| 3.4 | Road crossing box culvert |  | No | 2 |  |  |
| 3.4.1 | Excavation of clay soil |  | m3 | 10 | 136.5 | 1,365.36 |
| 3.4.2 | Hard coring |  | m3 | 15 | 528.3 | 7,924.50 |
| 3.4.3 | stone pitching |  | m2 | 30 | 750.0 | 22,500.00 |
| 3.4.4 | Concrete works of mix ratio 1:2:4 |  | m3 | 20 | 2247.4 | 44,947.08 |
| 3.4.5 | Plastering, mix ratio 1:3 |  | m2 | 25 | 108.9 | 2,723.59 |
| 3.4.6 | reinforcement bar : -Ø14mm |  | kg | 1520 | 34.2 | 51,988.75 |
| 3.4.7 | black wire |  | kg | 15 | 34.2 | 513.05 |
| 3.4.8 | form work |  | m2 | 15 | 351.5 | 5,272.30 |
| Sub Total |  |  |  |  |  | 137,234.63 |
| 3.5 | Foot path |  | no | 2 |  |  |
| 3.5.1 | Excavation for foundation |  | m3 | 20 | 136.54 | 2,730.73 |
| 3.5.2 | Masonry works of mix ratio 1:3 |  | m3 | 10 | 1,220.27 | 12,202.72 |
| 3.5.3 | Rcc work of 1:2:4 |  | m3 | 4 | 2,247.35 | 8,989.42 |
| 3.5.4 | Plastering, mix ratio 1:3 |  | m2 | 25 | 108.94 | 2,723.59 |
| 3.5.5 | hard coring |  | m3 | 10 | 528.30 | 5,283.00 |
| 3.5.6 | Reinforcement bar : -Ø10mm |  | kg | 250 | 34.00 | 8,500.00 |
| 3.5.7 | black wire |  | kg | 8 | 40.00 | 320.00 |
| sub total |  |  |  |  |  | 40,749.45 |
| 3.6 | Turnouts/off take on MC/SC |  | No | 10 |  |  |
| 3.6.1 | Excavation of ordinary soil |  | m3 | 60 | 136.54 | 8,192.18 |
| 3.6.2 | Masonry works of mix ratio 1:3 |  | m3 | 25 | 1,220.27 | 30,506.79 |
| 3.6.3 | Cemented stone pitching (thickness 25cm) |  | m3 | 2 | 750.00 | 1,500.00 |
| 3.6.4 | Hard coring |  | m3 | 2.5 | 528.30 | 1,320.75 |
| 3.6.5 | plastering |  | m2 | 125 | 108.94 | 13,617.97 |
| 3.6.6 | pointing |  | m2 | 80 | 95.10 | 7,607.98 |
| 3.6.7 | Shatter gate(0.3m*0.3m*3mm) |  | pcs | 15 | 3,500.00 | 52,500.00 |
| sub total |  |  |  |  |  | 115,245.67 |
| 3.7 | Drop Structures |  | no | 18 |  |  |
| 3.7.1 | Structural excavation to a depth 0.5-1.0m |  | m3 | 35 | 136.54 | 4,778.77 |
| 3.7.2 | Masonry works of mix ratio 1:3 |  | m3 | 56 | 1,220.27 | 68,335.22 |
| 3.7.3 | Cemented stone pitching (thickness 25cm) |  | m3 | 38 | 750.00 | 28,500.00 |
| 3.7.4 | Concrete works of mix ratio 1:2:4 |  | m3 | 3 | 2,247.35 | 6,742.06 |
| 3.7.5 | Hard coring |  | m3 | 10 | 528.30 | 5,283.00 |
| 3.7.6 | Plastering |  | m2 | 150 | 108.94 | 16,341.56 |
| 3.7.7 | pointing |  | m2 | 120 | 95.10 | 11,411.97 |
| Total for drop |  |  |  |  |  | 141,392.59 |
| 3.8 | Night storage Reservoir |  | no | 1 |  |  |
| 3.8.1 | excavation |  | m3 | 450 | 136.54 | 61,441.35 |
| 3.8.2 | Compact filling |  | m3 | 350 | 103.44 | 36,205.13 |
| 3.8.3 | Geomembren lining (1.2mm thick) |  | m2 | 1000 | 90.00 | 90,000.00 |
| 3.8.4 | Masonry works of mix ratio 1:3 for inlet and out let |  | m3 | 20 | 1,220.27 | 24,405.43 |
| 3.8.5 | Concrete works of mix ratio 1:2:4 |  | m3 | 5 | 2,247.35 | 11,236.77 |
| 3.8.6 | Hard coring well dressed in cement mortar |  | m3 | 10 | 528.30 | 5,283.00 |
| 3.8.7 | Dia. 0.4m concrete pipe |  | pls | 10 | 350.00 | 3,500.00 |
| 3.8.8 | Plastering |  | m2 | 58 | 108.94 | 6,318.74 |
| 3.8.9 | pointing |  | m2 | 69 | 95.10 | 6,561.89 |
| 3.8.10 | Spindled Control gate (2.5m*0.4m*4mm) |  | pcs | 1 | 10,000.00 | 10,000.00 |
| 3.8.11 | Fencing (1.5m spacing of Di. 12cm wooden pole and barbed wire ) |  | m | 200 | 150.00 | 30,000.00 |
| Total for NSR |  |  |  |  |  | 254,952.31 |
| 3.9 | Social infrastructures |  |  |  |  |  |
| 3.9.1 | On canal cattle trough |  |  | 1 |  |  |
| 3.9.1.1 | Excavation of ordinary soil for main canal |  | m3 | 4 | 136.54 | 546.15 |
| 3.9.1.2 | Hard coring well dressed in cement mortar |  | m4 | 4 | 528.30 | 2,113.20 |
| 3.9.1.3 | Mass concrete 1:2:4 |  | m3 | 5 | 2,247.35 | 11,236.77 |
| 3.9.1.4 | Compact fill |  | m3 | 2 | 103.44 | 206.89 |
| Sub Total |  |  |  |  |  | 14,103.00 |
| 3.9.2 | Washing basin |  |  | 2 |  |  |
| 3.9.2.1 | Excavation of ordinary soil for main canal |  | m3 | 5 | 528.30 | 2,641.50 |
| 3.9.2.2 | Masonry works of mix ratio 1:3 |  | m4 | 11 | 1,220.27 | 13,422.99 |
| 3.9.2.3 | Cemented screed |  | m2 | 20 | 162.63 | 3,252.59 |
| 3.9.2.4 | pointing |  | m2 | 10 | 95.10 | 951.00 |
| Sub Total |  |  |  |  |  | 19,317.08 |
| Canals &Structures Sub Total 3 |  |  |  |  |  | 1,394,302.48 |
|  | Grand Total 1+2+3 |  |  |  |  | 1,911,004.52 |
|  | Vat 15% |  |  |  |  | 286,650.68 |
|  | Project cost |  |  |  |  | 2,197,655.20 |

# CONCLUSIONS AND RECOMMENDATIONS

## Conclusions

The project is proposed in high land area where supplementary irrigation cropping is essential to support the current phenomenon of rain feed agriculture. Even though the rainfall in the area is adequate and evenly distributed, this irrigation project within area has significant effect in upgrading and intensifying the agricultural products during dry season. Therefore, this irrigation project is crucial for the community residing in the area, as its implementation will improve crop production and productivity of the area. In addition, it will improve self sufficiency situation of the project beneficiaries. The designed Tilo small-scale irrigation project was intended to irrigate about 20 hectares of land less than  stated in the bid, however the study has revealed that there is more  land that can be considered for the project, but the available water resources  is the limiting factors . Due to shortage of  water resource to the parent river, command area was fixed to 20ha of the area can be commanded by proposed head work of 0.5m height of masonry and  installing shutter gate of 0.5m for under sluice. As field observation and geological study of the head work and few meters of main canal route is fractured weathered rock, this necessitates approximately 800m  be lined.

This study has also shown that Tilo river water can be used all the times of a day and night to store   in NSR. Even though, there is traditional irrigation system in the project area, yet, major benefit of this project will be introduction of appropriate and permanent structures and related infrastructures to improve efficiency of the system. As installation of weir and shutters gate is needed to raise the water level and easily divert to command area. Training to be provided for the farmers how to install and removed during dry and rainy season.

Beneficiary farmers, both men and women, would have to be organized under a Water Users Association (WUA) to participate in the development effort of the scheme. Many committees have been set up to handle important tasks which will facilitate the construction and management of the scheme.  Committee members are also getting some training in group dynamics, leadership skills, and the functions of water users’ association. However, since the common features of the farms is lack of irrigation facilities, and low use of inputs and improved technologies, more training is needed in crop production, irrigation water use, scheme management, and marketing.

Farmers in the project area are very keen and supportive of the development of the scheme. They have strong expectation that it will change their livelihood for good: improve household food security and increase cash income.

Their positive attitude and interest is a great asset for scheme development. They have also indicated their willingness to make their own contribution to scheme construction and maintenance. The authorities at all level are also positive on the project idea and enthusiastic to be involved and participate in its implementation. The command is situated within undulating land area. Thus, drainage problem is not seen critical except low lying and riverine land within the project command area.

## Recommendations

The project area is accessible from single way the district town Sagure and the all weather road Digulu to Geba Jimata village pass through the command area. There will be no difficult for transporting construction materials to the head work site.

To  strengthen  the effectiveness  of the  project,  the supply of agricultural inputs,  improved seeds, fertilizers,  farm tools,, should be facilitated,  credit facilities to enable the farmers  to acquire the inputs have to be provided.  Further, WUA needs to be established, and the farmers’ skill in operation and maintenance as well as farming technology in general will have to be improved through training.

The soil investigation results of the current study shows that the project area can be used for irrigation development, bearing in mind that application of some of the correction measures recommended are implemented in order to improve the production of crops and sustainability of soil fertility.

REFERENCES

- VenTe Chow , 1959, Open Channel Hydraulics

- Canal Design by L. Hyde, PhD, M.Sc. LN0326/06/1

- Design of canals(INDIAN ITM), Indian institute of technology madras

- MoWR's Guideline, Manuals & Standard Design of Small- & Medium-Scale Irrigation Projects: Part I-A

- Nepal Design Manual M8, 1991

- Bureau of Indian Standards-IS: 10430 -1982 “Criteria for design of lined canals and guidelines for selection of type of lining”

- Ethiopian Roads Authority (ERA) Design Manual

- ERA (2002). Drainage Design Manual, Hydrology.

- Shaw, Elizabeth M.(988). Hydrology in Practice. International Van Nostrand Reinhold.

- S.K.Garg(2005). Irrigation engineering and hydraulic structures, Delhi, India.

- Fiddes D, 1977: Flood estimation for small East African rural catchments, Proceeding Institution of Civil Engineers, Part 2, 63, 21-34 (1977)

## ANNEX A:  SYSTEM LAY OUT MAP

## ANNEX B: UNIT RATE ANALYSIS