THE NATIONAL REGIONAL STATE OF OROMIA

# OROMIA IRRIGATION DEVELOPMENT AUTHORITY

AGRICULTURAL GROWTH PROGRAM (AGP II)

Lot 9 of Eighty Small Scale Irrigation Projects Feasibility Study and Detail Design

Volume III: Irrigation Agronomy Study of Birbirsa Small Scale Irrigation Project

Final

March 2019

AYJEF Water Woks and                 P.O.Box: 43329       Tele: +251 911 304019

Business Service PLC                                           E-mail: ayconsult1@gmail.com

Finfine

# CONTENTS

CONTENTS	ii

LIST OF TABLES	iv

LIST OF FIGURES	vi

ACRONYMS /ABBREVIATIONS	vii

EXCUTIVE SUMMARY	viii

1. INTRODUCTION	1

1.1. Background	1

1.2. General Objective	2

1.3. Specific Objectives of Agronomy Study	2

1.4. Location	3

1.5. Topography	3

2. METHODOLOGY AND APPROACH OF THE STUDY	4

2.1. Pre Field Work	4

2.2. Field Work	4

2.3. Post Field Work	5

3. RESULT AND DISCUSSION	6

3.1. Land Use, Settlement Pattern and Livelihood	6

3.1.1. Land use land cover in and around the project area	6

3.1.2. Settlement	6

3.1.3. Livelihood	6

3.3. Climatic condition of the Study Area	6

3.3.1. Climate	6

3.4. Existing Cropping Calendar for Rain Fed and irrigated Crops	12

3.5. Existing Cropping Patterns In the small holder farmers of the project district and productivity.	16

3.6. Irrigated Crop Production of the district	20

3.7. Existing Farm Inputs and Agricultural Support Services	21

3.7.1. Soil Fertility Management	21

3.8.Existing Conditions of The Command Areas	25

3.8.1. Existing Cropping Pattern and Yield In and Around the Command Area	25

3.8 .2.Root and tuber crops, fruit crops and industrial crops	25

3.8.3. Coffee Production	26

3.8.4. Irrigation Practices in and around the command Areas	26

3.8.5. Existing Farm Inputs and Agricultural Support Services	27

3.8.8. Existing Agronomic Constraints	31

3.8.9. Production Potentials	38

3.9. Proposed Irrigated Agriculture	39

3.9.1. Benefits of Proposed small Scale Irrigation project	39

3.9.2. Agricultural Development Scenarios and Options	39

3.9.3. Integrated Agronomy Development Plan	41

3.9.4. Selection of Crops and Cropping Patterns	41

3.9.5. Crop basket selection	43

3.9.6. Crop Water Requirement	43

3.9.7. Proposed Crop Yield Projection	52

3.9.8. Proposed cropping pattern and intensity	53

3.9.9. Crop Rotation	56

3.9.10. Irrigation Farm Inputs and Support Services	56

3.9.11. Support Services	66

3.9.12. Agronomic Recommendations of Selected Crops	71

4. CONCLUSION AND RECOMMENDATION	90

4.1. Conclusion	90

4.2. Recommendation	91

5. REFRENCES	93

# LIST OF TABLES

Table 1. Summary of Meteorological Characteristics and ETO of Project Area	7

Table 2.Analytical Moisture Balance of the Study Area.	8

Table 3.Existing cropping calendar for rain fed crops	15

Table 4.Existing cropping calendar for irrigated crops	16

Table 5. Rain fed crop pattern with different Technologies in Guto Gida district (2009/10 E.C)	16

Table 6.Trend of crop rain fed pattern	17

Table 7.Existing root and tuber and vegetables (2009/10 E.C)	18

Table 8.Existing fruit crops in the district(2009/10 E.C)	19

Table 9.Existing cropping pattern of coffee and spices (2009/10 E.C)	20

Table 10.Specific existing area with irrigation in Guto Gida district (2009/10E.C)	20

Table 11.Trend of input utilization in the district	22

Table 12. Input utilization rate (2009/10 E.C)	23

Table 13.Pesticide Utilization rate	23

Table 14.Existing input cost (2009/10 E.C)	24

Table 15.Existing cropping pattern and yield of irrigated crops at Nagassa kebele (2009/10 E.C)	26

Table 16.Rate of input utilization	28

Table 17. Cost of  Fertilizers, Pesticides and Improved Seeds (2009/10 E.C)	28

Table 18.Labor Cost (birr/ha)  for Existing Major Crops (rain fed)(/2009/10 E.C)	29

Table 19.Labor Cost per hectare for Existing Major Crops (irrigated) (2009/10E.C)	29

Table 20.Oxen Day   Cost for Existing Major Crops (Rain fed crops) for plowing with oxen (2009/10 E.C)	30

Table 21: Oxen Day Cost for Existing Major Crops (irrigated) (2009E.C)	30

Table 22.Climate and ETO	46

Table 23.Crop Parameters for Water Demand Analysis at the Area	46

Table 24.Cropping calendar and Crop Water Requirement for the scheme	47

Table 25.Scheme Supply for full irrigation	49

Table 26.Scheme Supply for supplementary irrigation	50

Table 27.irrigation depth and irrigation interval of the project	52

Table 28.Potential and Actual Average Yield of Proposed Crops for irrigation Project	52

Table 29.yield Projection across Years (qt/ha)	53

Table 30.Cropping pattern and intensity	55

Table 31.Proposed cropping pattern and production projection  during supplementary irrigation	55

Table 32.Proposed crop rotation	56

Table 33.Required inputs and rates	57

Table 34.Proposed Labour cost	58

Table 35.proposed oxen day cost	63

Table 36Material cost	64

Table 37.Cost summary	65

Table 38.improved Banana Varieties	72

Table 39.Rate of Fertilizer Application	73

# LIST OF FIGURES

Figure1.Partial view of command area	5

Figure 2 .Max.verse  minimum temperature	10

Figure 3.RH verse temperature	10

Figure 4.Sunshine hours of the command area	11

Figure 5.LGP of the command Area.	12

# ACRONYMS /ABBREVIATIONS

GDP                                      Gross Domestic Product

OIDA                                    Oromia irrigation Development Authority

GTP                                        Growth and Transformation Plan

FAO                                        Food and Agricultural Organization

FGD                                        Focal Group Discussion

LGP                                        Length of Growing Stage

C.V                                         Coefficient of Variation

PET                                         Potential Evapotranspiration

DAP                                       Di ammonium Phosphate

Ec                                            Emulsfiable Concentrate

WP                                          Wettable powder

A.i                                          Active ingredient

FTC                                         Farmers Training Centre

DA                                         Development Agent

IPM                                         Integrated Pest Management

AGP                                        Agricultural growth program

IMV                                         Improved crop variety

CF                                           Commercial fertilizer

OF                                           Organic fertilizer

SSH                                        Sunshine hours

MD                                           Man days

NPS                                         Nitrogen phosphorus and sulfur

# EXCUTIVE SUMMARY

This document outlines the results of the study and analysis of small scale irrigation project. Considering the fact that increase in agricultural productivity is a popular motive for farmers and a driving force in the country and respective project area as a whole, considerable attempts were made   by government to realize agricultural productivity of food crops, cash and industrial crops etc   to alter the state of agriculture in sustaining food security and providing necessary raw materials for agro processing industry.

However, as the number of population in the area has increased from year to year and productivity of the land had deteriorated capacity to feed the ever increasing population using rain-fed agriculture became a challenge. To this effect the government has planned to develop a small Irrigation Project on the intended area.

The intended command area is found at 6km from Nekemte and the source of river is Birbirsa river with the gross command area of 79 hectares and net command area of 51 hectares .The topography of the command area is flat land having slope of 0-2%.The study area receives mean annual rainfall of 2089 mm having mean monthly maximum temperature of 24.60c and mean monthly minimum temperature of 13.70c.

Based on the Kebele’s record, the total number of beneficiary population of the command area of the project in the year 2018, base year, is 484 (232 male and 252 female). This current population of the project area is expected to reach 676 in the next 20 years (2038). Similarly, the total number of the would-be user or beneficiary households of the command area of the project is 85.

The project supply of the irrigation project for  full irrigation  is indicated for 24 hour irrigation and becomes 1.02 L/s/ha at the month of January and 0.6l/s/h for 24 during supplementary (season 2)irrigation at the month of April.

The main objective of the feasibility and detail design study is to enhance the implementation of small scale irrigation projects which in turn plays a vital role in improving and securing the livelihood of the rural poor population   through introduction of irrigated agriculture by using modern farming technology and thereby attain food self sufficiency and food security of the community.

The study methodology followed is generally categorized into activities accomplished in pre field, field work and post field work activities.

Under the present study, major topics covered include:

- Description of project location

- Land Use and Settlement Patterns of the Study Areas

- Livelihood and Land Holding

- Climatic condition of the Study Area

- Present Agricultural Practices(rain fed and irrigated )

- Existing Development Constraints and potentials

- Proposed irrigated agriculture

Under the proposed irrigated agriculture, main sub topics treated includes

- Benefits of Proposed small scale  Irrigation  Project

- Agricultural Development Scenarios and Options

- Crop Water Requirement analysis and irrigation scheduling

- Proposed Crop Yield Projection

- etc

Agronomic Recommendations of Selected Crops

- Conclusion and recommendation

# 1. INTRODUCTION

## 1.1. Background

Agriculture in Ethiopia accounts for almost 41 percent of GDP, 80 percent of exports, and 80 percent of the labour force. Many other economic activities depend on agriculture, including marketing, processing and export of agricultural products. Production is overwhelmingly of a subsistence nature, and a large part of commodity exports are provided by the small agricultural cash-crop sector.

Though Ethiopia’s agriculture is besieged by periodic drought, soil degradation  caused by overgrazing, deforestation, backward farming practices and high population density, yet agriculture is the country's most promising resource and will continue to be a major contributor to GDP in the coming years and  is vital for the realization of the second national development marshal plan. Furthermore, by the end of the five year plan, agricultural production is expected to grow to 406 million quintals and to foster the agricultural development, irrigation development will be the major pillar in the upcoming second GTP.

considering the fact that increase in agricultural productivity is a popular motive for farmers and a driving force in the country and respective project area as a whole, considerable attempts were made   by government to realize agricultural productivity of food crops, cash and industrial crops etc   to alter the state of agriculture in sustaining food security and providing necessary raw materials for agro processing industry.

However, the back warded  traditional  farming practices on these exhausted land   and  less eminent utilization of existing irrigation farm land and water resource potentials coupled with increasing population  from year to year, the capacity of the land to feed the ever increasing population and  assuring  cash crops and raw material for industry  with  rain-fed agriculture is becoming  a great  challenge. Thus, it has got absolute attention   to consider the existing irrigable land and water resources in the area to enhance the contribution of irrigated agriculture in the overall agricultural economy.

. Accordingly, this small scale irrigation project is awarded to AYJEF in order to undertake the study and design works of the project by multi-disciplinary teams of the firm, where agronomy study is one of the study works to be undertaken.

.

According to the present survey, the communities lead their livelihood dominantly by crop cultivation and livestock rearing. The project command area has a number of production potentials although various technical and non technical constraints are prevailing .Generally, Agronomy study mainly focused on the assessment of existing agronomic conditions along with the identification of gaps and constraints and proposes new irrigated agriculture based on the available land, soil, climate, water resource, potential crops and other socio economic conditions.

## 1.2. General Objective

The main objective of the feasibility and detail design study is to enhance the implementation of small scale irrigation projects which in turn plays a vital role in improving and securing the livelihood of the rural poor population   through introduction of irrigated agriculture by using modern farming technology and thereby attain food self sufficiency and food security of the community.

## 1.3. Specific Objectives of Agronomy Study

- Evaluation of the existing agricultural situation of the study area (i.e. crops grown, cropping pattern and farming system etc) both in rain fed and irrigation.

- Identification of the existing physical environment of the study area.

- Identification and evaluation of crop production constraints and potentials of the study area.

- Based on the crop selection criteria, selection of suitable crops, cropping pattern and cropping calendar

- Estimation of input requirements such as seeds, fertilizers, pesticides , labour and draught power   etc for the selected crops

- Yield projection for selected crops.

- Recommendation of modern agronomic practices and supporting services for crops grown under irrigation.

- Analysis of crop water requirements and irrigation scheduling of the proposed crops by taking climate, soil and crop characteristics into consideration.

## 1.4. Location

Birbirsa small scale irrigation is located in Guto Gida district of east Wollega zone in about 4 to 5kms south  of Nekemte town .Although very near to  Nekemte town  it is only accessible by dry weather  road .The irrigation command area is situated in and at the confluence of Abdeta and Nagassa kebeles of Guto Gida district.

## 1.5. Topography

Topography of the area is very flat (0-2%) and few gentle slope surrounded by hills and having alluvial soils transported to the area that is suitable for gravity irrigation.

# 2. METHODOLOGY AND APPROACH OF THE STUDY

The methodology and approach followed during detail agronomy study is summarized as follows

## 2.1. Pre Field Work

- Comprehensive questionnaires and checklists  including all required parameters of farming were  developed for administering to   zonal &district respective sectoral offices, kebele level development offices and randomly selected farmers in the kebeles of project command area

- Study approach and methodology had been designed and necessary field materials and logistics had been prepared

## 2.2. Field Work

- Official documents, previous studies, annual reports, survey documents etc. Were  collected  from respective  stakeholders at government hierarchical  levels

- Key informant interview and focus group discussion had been conducted with representative samples of   target community

- Direct field observation had been carried out looking for assessment of first hand information on the potentials, constraints and opportunities for development.

- Discussion and interviewing with stakeholders at respective government  level(at district and kebeles )to  get information on the current project constraints ,potentials and opportunities  and past experiences of similar developments

- Relevant secondary data had been collected from zonal , district and kebele  level offices

This data collection methodology helped in obtaining the information on crops, the varieties being grown, the existing potential and constraints both under rain fed and irrigated

Production system, current farming system, crop rotation, the present status of agricultural support services, strength and weaknesses of research-extension-farmers linkages etc .

## 2.3. Post Field Work

- The collected information was processed, analyzed and critically evaluated for developing the suitable agricultural production plan.

- For data analysis, simple statistical models such as means ,Coefficient of variations ,and standard deviations were used

- The water requirement of duly selected crops and cropping pattern was worked out considering nearest and representative meteorological parameters, crops and cropping factors, soil factors etc.

- The water requirement was calculated following standard scientific procedures as suggested by FAO Irrigation and Drainage, Paper No. 56 and data processing and modeling for crop water requirement was undertaken by method of Penman Monteith using CropWat 8.0 software for windows.

- Draft   feasibility final report had been prepared and submitted

Figure1.Partial view of command area

# 3. RESULT AND DISCUSSION

## 3.1. Land Use, Settlement Pattern and Livelihood

### 3.1.1. Land use land cover in and around the project area

The land use land cover of the district is allocated as 83.7% for annual and perennial crops cultivation,4.8 %for forest ,bushes ;6.5% for grazing land ;2.2 %degraded land ,2.8% swampy and other uses

The land use land cover of the specific command kebele is 87% cultivated land, 8% grazing land 2 % forest bush and shrub land and 3% settlement and other use such as water bodies, rocky areas etc; however, the status of land use and cover of the specific command area indicates 96% of cultivated land with recession farming with cereals and vegetables   and upland rain fed crops, 2% of shrub and   bushy grassland, 2% by other uses like water bodies, gorges etc.

Generally the specific command area land use land cover is more dominantly exhibited as cultivated land though there is very few other use types.

### 3.1.2. Settlement

The settlement situation of the specific command area is mainly scattered pattern based on clans and family members more generally settled/clustered around their farm land

### 3.1.3. Livelihood

Several natural factors influence the farming system of the project command area. Among these the altitude, soil, climatic parameters and socio economic factors are the major. Crop cultivation and mixed agriculture or both crop cultivation and livestock are the major means of livelihood eventhogh there are some and petty trade activities Therefore, the means of livelihood of the majority of population are crop cultivation and mixed agriculture.

## 3.3. Climatic condition of the Study Area

### 3.3.1. Climate

Climate is one of the components of an environment that determines formation and characteristics of natural vegetation, crop production and other farming systems of a particular area. Depending on the consecutive mean rainfall, the climate of the study area is classified as sub humid with length of the growing period (LGP) of 210 days (revised agro-ecological classification, 2005).The Nekemte  meteorological station is  located close to the irrigation command area and is relatively reliable source of information on climatic data. Accordingly, monthly records of rainfall from 1999 to 2014 have been obtained from the meteorological station whereas maximum temperature and minimum temperature for the years of   1998 to 2015 and sunshine hours for the year of 2009 to 2014 and relative humidity from 2008 to 2015had been analyzed from the station .On the other hand , wind speed had been derived by  New_ Loclim_ 1.10 software .

Table 1. Summary of Meteorological Characteristics and ETO of Project Area

| Month | Min Temp | Max Temp | Humidity | Wind | Sun | Rad | ETo |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | °C | °C | % | km/day | hours | MJ/m²/day | mm/day |
| January | 12.2 | 26.1 | 62 | 130 | 8.5 | 20.1 | 3.98 |
| February | 13.5 | 28 | 57 | 147 | 8.4 | 21.2 | 4.59 |
| March | 14.3 | 28 | 58 | 147 | 7.8 | 21.3 | 4.74 |
| April | 14.5 | 27.4 | 65 | 138 | 7.8 | 21.5 | 4.62 |
| May | 13.9 | 24.9 | 79 | 130 | 6.1 | 18.5 | 3.73 |
| June | 12.9 | 22.5 | 87 | 95 | 4.9 | 16.3 | 3.08 |
| July | 12.8 | 21.1 | 89 | 104 | 3.1 | 13.8 | 2.6 |
| August | 12.9 | 21.1 | 90 | 95 | 2.9 | 13.7 | 2.57 |
| September | 12.9 | 22.5 | 88 | 104 | 3.6 | 14.8 | 2.79 |
| October | 12.9 | 24 | 79 | 147 | 6.7 | 18.8 | 3.57 |
| November | 12.6 | 24.5 | 73 | 147 | 7.5 | 18.8 | 3.64 |
| December | 12.1 | 25 | 68 | 147 | 7.8 | 18.6 | 3.67 |
| Average | 13.1 | 24.6 | 75 | 128 | 6.3 | 18.1 | 3.63 |

#### 3.3.1.1. Rainfall

Rainfall needs to be examined in terms of its ability to satisfy crop requirements. By comparing the results with the potential evapo-transpiration, the water available to crops could be determined. For example a positive difference between rainfall and potential evapo-transpiration for the greater part of a growing season would ensure good crop growth. A negative result during the critical growing season would indicate insufficient moisture for that crop or the need for irrigation.

On the other hand, an excess of precipitation over the potential evapo-transpiration during the wet season would indicate the importance of drainage. A study of the rainfall intensity (quantity over a given period of time) would give an indication of the potential water erosive or of the runoff to be expected and of the dimensions which would have to be given to surplus water disposal channels etc.

Some qualitative aspects of precipitation are also worthy of attention: for example, the duration and frequency of hail storms, in relation to the periods when the land is occupied by crops, or the occurrence of rain storms associated with strong winds, etc.

Even though it is erratic and showing differences among consecutive years, the rainfall type is mono modal that gives maximum rainfall in the month of August (398.0

mm) and the mean minimum rainfall is recorded in January (10.2mm).

Table 2.Analytical Moisture Balance of the Study Area.

|  | Jan | Feb | March | April | May | June | July | Aug | Sep | Oct | Nov | Dec |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PET | 116.4 | 118.2 | 138.9 | 141.3 | 121.6 | 99.3 | 86.4 | 89.9 | 95.3 | 117.8 | 114.1 | 114.2 |
| 0.5PET | 58.2 | 59.1 | 69.45 | 70.65 | 60.8 | 49.65 | 43.2 | 44.95 | 47.65 | 58.9 | 57.05 | 57.1 |
| RF |  |  |  |  |  |  |  |  |  |  |  |  |
| 1999 | 42.4 | 2.4 | 0.6 | 94.8 | 382 | 331.1 | 194.9 | 250.9 | 238.2 | 338.7 | 23.8 | 8.9 |
| 2000 | 0.3 | 0 | 2.9 | 107.2 | 224.9 | 509.3 | 332.2 | 540.9 | 240.8 | 138.8 | 19.2 | 19.7 |
| 2001 | 0 | 26.4 | 50.8 | 83.6 | 240.8 | 349.6 | 336.2 | 352.4 | 267.4 | 168.8 | 16 | 50.2 |
| 2002 | 24.9 | 21.1 | 74.3 | 106.2 | 77.4 | 376.8 | 427.3 | 260.3 | 186.9 | 117.6 | 0.2 | 33 |
| 2003 | 0.3 | 47.5 | 54.9 | 21.3 | 55.1 | 422.9 | 429.7 | 439 | 312.9 | 18.9 | 25.5 | 9.5 |
| 2004 | 7.2 | 6.9 | 12.7 | 76.6 | 206.3 | 286.6 | 409.9 | 433 | 248.9 | 71.9 | 20.2 | 11.9 |
| 2005 | 9.5 | 0 | 131.5 | 60.3 | 241 | 387.4 | 346.2 | 438.8 | 355.6 | 219.9 | 58.5 | 0 |
| 2006 | 0.5 | 4.8 | 56.6 | 71.3 | 209.4 | 350.6 | 476 | 339.1 | 278.3 | 213.8 | 50.4 | 88.6 |
| 2007 | 4.7 | 54.4 | 47.8 | 141.7 | 217.5 | 462.6 | 355.6 | 447.4 | 297.8 | 86.7 | 56.8 | 0 |
| 2008 | 13.1 | 0 | 0.1 | 227.4 | 368.3 | 416.7 | 412.1 | 384.9 | 301.7 | 235 | 82 | 0 |
| 2009 | 0 | 22.7 | 35.5 | 173 | 97.9 | 314.5 | 251.5 | 395.7 | 412.1 | 286.2 | 15.3 | 18.4 |
| 2010 | 7.1 | 25.6 | 3.5 | 27.6 | 532.7 | 518.3 | 450.9 | 318.7 | 366.8 | 133.6 | 81.3 | 16 |
| 2011 | 42 | 2 | 51.7 | 77.8 | 265.4 | 403.4 | 259.3 | 418.2 | 349.4 | 82.1 | 57.4 | 1.7 |
| 2012 | 0 | 0 | 26.9 | 32.2 | 251.9 | 307.4 | 399.8 | 593.8 | 314 | 99.4 | 68.8 | 15.1 |
| 2013 | 11.7 | 3.3 | 23.4 | 5.9 | 404.1 | 357.1 | 392.7 | 293.9 | 301.5 | 91.9 | 79.8 | 0 |
| 2014 | 0 | 3.8 | 56.8 | 149.5 | 536.4 | 435.6 | 360.2 | 460.2 | 276.6 | 209.6 | 31.4 | 0 |
|  | 163.7 | 220.9 | 630 | 1456.4 | 4311.1 | 6229.9 | 5834.5 | 6367.2 | 4748.9 | 2512.9 | 686.6 | 273 |
| Mean RF | 10.2 | 13.8 | 39.4 | 91.0 | 269.4 | 389.4 | 364.7 | 398.0 | 296.8 | 157.1 | 42.9 | 17.1 |
| SD | 166.3 | 165.9 | 164.0 | 162.4 | 163.6 | 166.5 | 164.4 | 160.1 | 123.3 | 80.7 | 27.8 | 22.8 |
| CV | 16.3 | 12.0 | 4.2 | 1.8 | 0.6 | 0.4 | 0.5 | 0.4 | 0.4 | 0.5 | 0.6 | 1.3 |

The rainfall data shows both systematic and irregular variations during onset and offset over time. These variations play significant role in adapting farming systems and thus productivity. The characterization of systematic variations in the rainfall data is important; not only in long term agricultural planning but as basis for long range forecasts.

Understanding the year-to-year variation of rainfall is very important not only for crop production but also for planning in agricultural development and hydrological management. Coefficient of variation as a tool provides a measure of months for consecutive years and for causative years to see the level of variation. Generally, rainfall in the study area is generally characterized by high variability (C.V.> 30 %) indicating that the rainfall pattern is less dependable.

#### 3.3.1.2. Temperature

Air temperature is the most important climatic variable that affects plant life. Each species and variety of plants and each age group of plants has its own upper and lower temperature limits. Beyond these limits, a plant becomes considerably damaged and may even be killed. Therefore, temperature should be considered while dealing with agricultural planning of an area. The mean maximum temperature ranges between 21.10c to 28.10c and the mean minimum from 12.10c to 14.50c

The highest mean maximum temperature in the area is recorded in the month of February and March and the highest mean minimum is in the month of march and April. Generally, the temperature condition of the area is suitable for growth and maturation of crops.

Figure 2 .Max.verse  minimum temperature

#### 3.3.1.3. Relative humidity

Relative Humidity (RH %), the water vapor  contained in the atmosphere, is expressed as the percentage of the ratio of actual to saturation vapor pressure. This, together with wind, gives an indication of drying power of the air important for drying and storage of crops, while fluctuations in values often affect the incidence of plant diseases as well as the safe storage of harvested crops.

The mean monthly relative humidity in the study area varies from 56 % in the month of February to 90 % in the month of August. It is inversely related to temperature of the area whereby it shows an increasing trend when the temperature decreases and viseversal.

Figure 3.RH verse temperature

#### 3.3.1.4. Wind Run

A study of the velocity, frequency and direction of winds is important as regards of the rate of transpiration of crops, or the necessity of planting wind breaks. Both maximum and minimum wind speed is significant in crop performance especially in the late development stage. Occurrence of strong winds may result in lodging.

The study area exhibits high wind in the months of October to march besides to the highest mean maximum temperature   creating an increase in the loss of moisture in the form of vapor while it drops in the times of peak rainy season .Generally, availability of the strong wind and temperature negatively affect the availability and distribution of rainfall in the area.

#### 3.3.1.5. Sunshine hours

Daily length/ sunshine hours are very important in photosynthetic plant growth and estimation of crop water requirement. The study area experiences sunshine hours varying from highest (8.5hrs /day) in January to lowest (2.9 hrs /day) in august. It is relatively positively correlated with temperature whereby temperature rises up with long duration of sunshine hours and viseversal having significant influence on the growth and maturation of crops.

Figure 4.Sunshine hours of the command area

#### 3.3.1.6. Length of Growing Season

The length of growing period could give a highlight about the potential crops and varieties to be considered to develop crop basket for the given area. The growing period analysis is based on rainfall, potential evapo-transpiration and soil moisture storage capacity.

Length of Growing Period (LGP) of the study area was determined by mean rainfall and half potential evapo-transpiration (0.5PET). From the comparison of mean rainfall and half potential evapo-transpiration (0.5PET), LGP was obtained where the amount of rainfall exceeded half PET and addition of some days until the stored water was evaporated (i.e. the dates starting from the point when RF curve cross 0.5 PET and up to it goes down (RF curve crosses 0.5 PET again) was considered as LGP period of the area adding of some days needed to evaporate the stored moisture in the soil depending on dominant soils of the area.

Figure 5.LGP of the command Area.

Accordingly, the rainfall pattern in the area is mono modal and there are almost 210 growing days in which the crops can be sown /planted and grown successfully.

Almost from November to march is dry period and crops cannot be grown without full  irrigation apart from recession farming area which need supplemental irrigation  .This was also confirmed during the site visit and discussion with concerned stakeholders and local community. Generally in accordance with the revised agro-ecological classification, 2005 the project area is classified in the range of sub humid agro-ecological classification.

## 3.4. Existing Cropping Calendar for Rain Fed and irrigated Crops

Site Selection

Crops have specific environmental requirements such as climate, soil and topography. In some localities at and around the study area, some crops  are cultivated at high slopes probably due to shortage of suitable cultivable land or low awareness about proper site selection. In such areas inefficient soil and water conservation activities coupled with high intensity of rainfall predisposes top soil to fertility problem.

Occasionally, there are mono cropping practices of maize, teff and such  erroneous practices reduce subsequent crop yield.

Land Clearing

In the present context of the area, land cleaning takes place before /in the beginning of rainy season. It is done using hand tools like axe and chopper /Gejera/.Then after;    the leftovers and /or the chopped materials and grasses are burnt.

Land Preparation (Tillage Operations)

Land preparation usually begins at and/ or near the onset of rain though dry time plowing is practiced for some crops. For crops like maize and sorghum, first tillage starts usually at January to February and secondary and tertiary continues up to may to June similarly for soyabean and haricot bean early tillage is becoming common . For teff and wheat it starts at April. The traditional plough (Maresha) using oxen power and small farm tools like shovel /hoe respectively are used for land preparation.

Sometimes,  land preparation delays due to low awareness, delayance in the onset of rain, lack of plowing oxen and efficient farm tools etc. When oxen power is used, one to three passes are common for most crops. However, whilst oxen power is lacking, crops may even be sown with single tillage resulting in poor seed germination and subsequent crop growth

Sowing/ Planting Operations (Time and Method)

Generally most of the field crops are sown from May to June while teff, wheat and barley are sown from July to August

Most crops grown are local varieties apart from improved maize, some vegetables and other few grains whereby these local varieties not properly selected in which case, ill and shriveled seeds are sown. To compensate for poor germination and survival of these local seeds, still reasonable amount of farmers broadcast crop seeds eventhogh substantial amount of farmers are attempting row planting for crops like millet teff soybean etc

Weeding

Crops and weeds compete for light, water and nutrients. The critical period of crop weed competition is the period from the time of sowing to the time to which the crop is to be maintained in a weed free environment to get the highest yield.

Many studies have shown that the critical period for crop weed competition occurs during the early part of the growing season. This critical period of weed competition varies with crop, cultivars and growing conditions .Generally; early one third of crop period should be maintained weed free. Vigorously growing plants with rapidly expanding leaf canopies and root systems are more efficient in utilizing the resources that are essential for plant growth.

With the context of the project area, timely weeding at the right crop growth stage with required frequency is less practiced although relatively more attention is given to maize, sorghum and teff .This is due to low farmers awareness, nature of weeds and labor shortage as the result of seasonal overlap of crop calendar coupled with heavy rainfall conditioning rapid weed growth.

Table 3.Existing cropping calendar for rain fed crops

| Type of crop | Time of land preparation | Freq | Time of sowing/planting | Time of weeding | Freq | Time of harvesting |
| --- | --- | --- | --- | --- | --- | --- |
| Maize | Early January | 3 | Mid may to end of June | June to September | 2-3 | October to January |
| Sorghum | Early January | 2-3 | Mid may to end of June | June to September | 2-3 | December to January |
| Teff | early April | 3-4 | July to August | August | 1-3 | October to December |
| Soybean | Early January | 1-3 | Mid may to end of June | June to September | 2-3 | October to November |
| Haricot bean | Early January | 1-3 | Mid may to end of June | June to September | 2-3 | October to November |
| Ground nut | Early January | 2-3 | June | June to September | 2 | October |
| Wheat | early April | 3 | July to August | July to October | 2 | October to mid December |
| Barley | early April | 3 | July to August | July to October | 2 | October to mid December |

Harvesting time

In and around the command area, crops are harvested manually using small farm tools such as sickles, machetes, hoes/dongora etc. Harvesting time varies depending on crop type, growth stages and climatic and socio cultural conditions. According to the information obtained from development agents and key informants interview, harvesting of crops after physiological maturity typically delays due to lack of awareness and labour force during peak time for harvest. However, vast majority of crops are harvested starting from October and crops do not stay on field later than January

Existing cropping calendar for irrigated crops

Regarding existing irrigated crops ,nursery time for crops is usually from august to September .Land preparation starts mainly from September whereby the frequency of tillage varies according to soil drainage condition, crop type and available manpower and  means of land preparation etc while frequency of tillage is 2 to 4  .Sowing is commenced as of October for majority of crops .Weeding is a monotonous  practice and is practiced from 1 to 3 times on  average .Harvesting is carried out at December to January for most crops except sweet potato . For second round irrigation, the dominant crop cultivated is maize whereby it is sown in February and harvested in May to June.

Table 4.Existing cropping calendar for irrigated crops

| Type of Crop | Nursery Time | Time of Land Preparation | Freq | Time of Sowing/Transplanting | Weeding Time | Freq | Harvesting Time |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Potato |  | Early sep | 2-3 | early October | November | 2-3 | January |
| Sweet potato |  | early sep | 1-3 | late September | October | 2-3 | April |
| Pepper | Early August | sep | 2-3 | November | late November | 2-3 | February |
| Maize |  | early October | 2-3 | October | October | 2-3 | January(wet) |
| Tomato | September | Early sep | 2-3 | early October | October | 1-3 | December |
| Head cabbage | early sep | Early sep | 2-3 | early October | late October | 1-3 | December |
| Carrot |  | Early sep | 2-3 | early October | late October | 1-3 | January |

## 3.5. Existing Cropping Patterns In the small holder farmers of the project district and productivity.

#### 3.5.1. Grain Crops

The district and command area in particular have suitable soil, topography, conducive climatic characteristics and other crop requirements making good potential for production of various crops .The area cultivated for major grain crops with and without different technological packages in the district according to the data of  2009/2010 E.C  is  indicated below

Table 5. Rain fed crop pattern with different Technologies in Guto Gida district (2009/10 E.C)

| Crop | CF+IMS | yield(qt/ha) | CF+LS | yield(qt/ha) | OF+LS | yield(qt/ha) |
| --- | --- | --- | --- | --- | --- | --- |
| Maize | 18039 | 53 | 200 | 20 | 105 | 20 |
| Sorghum |  |  | 2526 | 20 | 214 | 15 |
| Teff | 602 | 10 | 2370 | 5 |  |  |
| Wheat | 3 | 10 | 2805 | 6 | 80 | 5 |
| Barley |  |  | 508 | 8 |  |  |
| Millet |  |  | 238 | 7 | 291 | 6 |
| Rice |  |  | 167.5 | 20 |  |  |
| Faba bean |  |  | 75 | 6 | 23 | 6 |
| Field pea |  |  |  |  | 20 |  |
| Haricot bean |  |  | 434 | 7 | 499 | 7 |
| Soybean |  |  | 60 | 15 | 60 | 10 |
| Noug |  |  | 400 |  |  |  |

Source ;District Agriculture &Rular Devt.Office

Maize, sorghum, teff and wheat etc are the major cultivated crops  in the district. With regards to technological packages, maize is the only crop most dominantly cultivated with improved seed with commercial fertilizers while very few  farmers also cultivate teff and wheat the same .According to the present study ,majority of  cultivated crops are  local seeds with  commercial fertilizers eventhogh the rate  and time of fertilizers applications  are less maintained  .Generally the yield of crops greater for crops cultivated with commercial fertilizers followed by local seeds and commercial fertilizers .The yield is very low when local seeds are used with organic fertilizers and without any fertilizers. Based on the assessment result over the last five years, maize, sorghum and teff are the most dominant crops in terms of area cultivated and area of maize  shows an increasing trend over the years while others are relatively constant or even declining.

Table 6.Trend of crop rain fed pattern

| Crop | Production years (ha) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 2006 |  |  | 2007 |  |  | 2008 |  |  | 2009 |  |  | 2010 |  |  |
|  | Area(ha) | percent | yieldQt/ha) | Area(ha) | percent | yieldQt/ha) | Area(ha) | percent | yieldQt/ha) | Area(ha) | percent | yieldQt/ha) | Area(ha) | percent | yieldQt/ha) |
| Maize | 8478 | 28.3 | 104 | 13470 | 45.0 | 59 | 14493.5 | 54.3 | 49.5 | 16443.5 | 55.6 | 47 | 18343 | 62.0 | 52.5 |
| Sorghum | 6322 | 21.1 | 26 | 4171 | 13.9 | 27 | 1995 | 7.5 | 19 | 2651 | 9.0 | 22.5 | 2740 | 9.3 | 25 |
| Teff | 3720 | 12.4 | 10 | 4075 | 13.6 | 7 | 3563 | 13.4 | 4.5 | 3854 | 13.0 | 8 | 3407 | 11.5 | 6.5 |
| Wheat | 195 | 0.7 | 19.5 | 195 | 0.7 | 17 | 195 | 0.7 | 16 | 175 | 0.6 | 16 | 573 | 1.9 | 6 |
| Barley | 169.25 | 0.6 | 13 | 165 | 0.6 | 15 | 165 | 0.6 | 14 | 148 | 0.5 | 12 | 508 | 1.7 | 6.7 |
| millet | 2399 | 8.0 | 13 | 1732 | 5.8 | 12 | 1103.5 | 4.1 | 7.5 | 1508 | 5.1 | 11 | 529 | 1.8 | 6.8 |
| Rice | 60 | 0.2 | 31.6 | 60 | 0.2 | 33 | 60 | 0.2 | 34 | 53.5 | 0.2 | 30 | 167.5 | 0.6 | 32 |
| Faba bean | 118 | 0.4 | 10 | 118 | 0.4 | 10 | 118 | 0.4 | 8 | 106 | 0.4 | 5.5 | 98 | 0.3 | 6.5 |
| Field pea | 35 | 0.1 | 10 | 35 | 0.1 | 10 | 35 | 0.1 | 9 | 25 | 0.1 | 7 | 20 | 0.1 | 5 |
| Harict bean | 1220.5 | 4.1 | 10 | 1197 | 4.0 | 6 | 1380 | 5.2 | 7.5 | 575 | 1.9 | 7..5 | 953 | 3.2 | 8.4 |
| common bean | 188.5 | 0.6 | 10.8 | 165 | 0.6 | 4 | 165 | 0.6 | 11 | 148 | 0.5 | 17 | 120 | 0.4 | 13 |
| Noug | 2237 | 7.5 | 4.5 | 1270 | 4.2 | 3.8 | 105 | 0.4 | 3.5 | 1236 | 4.2 | 4.6 | 400 | 1.4 | 5.2 |
| Sesame | 3117 | 10.4 | 5 | 1558 | 5.2 | 3.3 | 1558 | 5.8 | 4.6 | 1091.5 | 3.7 | 4.12 | 804 | 2.7 | 4 |
| Ground nut | 1735 | 5.8 | 11.6 | 1735 | 5.8 | 12 | 1735 | 6.5 | 11.6 | 1559 | 5.3 | 9 | 900 | 3.0 | 10.6 |
|  | 29994.25 | 100.0 |  | 29946 | 100.0 |  | 26671 | 100.0 |  | 29573.5 | 100.0 |  | 29562.5 | 100.0 |  |

Source ; District Agriculture &Rular Devt.Office

pertinent to crop productivity, it shows a decreasing trend for vast majority of crops over the last five years due to mainly

- Fertility decline over years

- Rare use of high genetic crop varieties

- Less use of appropriate rate and amount of fertilizers

- Less follow up of other agronomic management practices

- Less pest control practices

- Etc

Generally full packages of technologies were not appropriately used to increase and maintain crop productivity .

3.4.4.2. Existing vegetable and toot crops in the district

#### Root and Tuber Crops

Due to ideal temperature, optimum rainfall and suitability of soil and the feeding habit of the people, vegetable and root crops are potentially grown with rain fed cultivation in and around the command area of the district. Crops such as  sweet potato ,  potato and  Anchote ,Yam  etc  are grown  with rain fed cultivation  .Among the crops  ,majority of cultivated area is covered  by  potato (28.9%) followed by sweet potato(21.1%) and Anchote while carrot is least cultivated .The yield of crops is low compared to the genetic potential of the crops due to a number of factors among which  soil fertility problems and agronomic management measures like variety selection, plant spacing, weed management, disease and insect management, harvesting are the major .

Table 7.Existing root and tuber and vegetables (2009/10 E.C)

| Type of crops | Area(ha) | Percent | yield (qt/ha) |
| --- | --- | --- | --- |
| Potato | 55 | 28.9 | 160 |
| Yam | 12 | 6.3 | 161 |
| Sweet potato | 42 | 22.1 | 160 |
| Taro | 15 | 7.9 | 161 |
| Anchote | 23 | 12.1 | 160 |
| Cabbage | 8 | 4.2 | 94 |
| Onion | 19 | 10.0 | 94 |
| Garlic | 10 | 5.3 | 94 |
| Carrot | 6 | 3.2 | 94 |
|  | 190 | 100.0 |  |

Source ; District Agriculture &Rular Devt.Office

#### 3.4.4.2. Fruit Crop Production

The prevailing agro ecology, soil and socio economic conditions in and around the study makes common phenomena to witness  existence of   fruits crops at farmers backyards  at and around the command area of the district, Among the fruits Avocado   is the most dominantly planted fruit crop (39%)followed by banana and papaya

However productivity of these fruit crops is hampered by low varietal selection, disease, fertility management and other agronomic management practices.

Table 8.Existing fruit crops in the district(2009/10 E.C)

| Type of | Area(ha) | percent | Average yield (qt/ha) |
| --- | --- | --- | --- |
| Papaya | 10 | 30 | 155.4 |
| Avocado | 13 | 39 | 151 |
| Banana | 10 | 30 | 151 |
|  | 33 | 100 |  |

3.4.3.2. Coffee and Spice Crops Production

In Guto Gida   district, coffee and spice crops production is a recent phenomena and recently, coffee production is becoming some of the agricultural occupations in the district. However the following needs great improvement

- nursery site management including proper site selection ,variety selection  ,watering

- coffee management on the field such as proper planting depth ,spacing ,fertility management ,pruning and stumping and pest management

- Harvesting and post harvest management operations

- Seeking attractive pricing of coffee to encourage farmers

Table 9.Existing cropping pattern of coffee and spices (2009/10 E.C)

| Crop | Area(ha) | Yield(qt/ha) |
| --- | --- | --- |
| Coffee | 9814.4 | 6.5 |
| Ginger | 120 | 35 |
| pepper | 131 | 12.8 |
| Kororima | 2 | 6 |

Source ; District coffee and Tea Devt. Authority

Likewise, spice crops like Ginger, Kororima and pepper etc are grown in the area. Most of the spices are high value crops demanded locally and internationally. Besides, they are also environmental friendly and can be produced by intercropping with other crops. However, there are various agronomic management problems like site selection, population density, weeds insect pest and disease, shading system, harvesting time and harvesting system .The Teppi Agricultural Research Center (TARC), located in Teppi town is the national spices research center of the country and has selected different varieties of spices.

### 3.6. Irrigated Crop Production of the district

Regarding irrigation activity in the district, farmers traditionally divert available rivers to cultivate crops such as   potato, tomato, onion , pepper  etc.

Irrigated crops in the district and around the command areas are  largely cultivated by traditional furrow irrigation and pump irrigation .With furrow irrigation ,potato is the dominant irrigated crop (43%) followed by garlic  and tomato  etc while in pump irrigation onion and garlic are  dominantly cultivated  followed by tomato and pepper .

Table 10.Specific existing area with irrigation in Guto Gida district (2009/10E.C)

| Crop | Area(ha)with pump | percent | Area(ha) with traditional furrow irrigation | percent |
| --- | --- | --- | --- | --- |
| Tomato | 723 | 24.4 | 883 | 17.7 |
| Onion | 824 | 27.8 | 606 | 12.2 |
| Potato | 94 | 3.2 | 2142 | 43.0 |
| Pepper | 489 | 16.5 | 273 | 5.5 |
| Garlic | 798 | 27.0 | 943 | 18.9 |
| Cabbage | 32 | 1.1 | 136 | 2.7 |
|  | 2960 | 100.0 | 4983 | 100.0 |

Source;District Irrigation Devt.Authority

## 3.7. Existing Farm Inputs and Agricultural Support Services

### 3.7.1. Soil Fertility Management

#### 3.7.1.1. Traditional management practices

It had been assessed that there are different methods of maintaining soil fertility in the district. These are traditional methods such as using cattle dung (manure), crop rotation, green manuring ,fallowing, composting, intercropping .Biological soil conservation practices  like planting trees, strip planting and other agro forestry practices are  common. Physical soil conservation methods such as terrace, cut off drain etc are also practiced.

#### 3.7.1.2. Chemical Fertility Management

Fertilizer application greatly improves crop productivity if the recommendation is based on the local soil fertility assessment. Soils of the study area are highly exhausted and need more fertilizers and utilization of these fertilizers has become one of the options to increase crop production .However, there is no readymade soil test result in the area and thus  farmers use generalized recommendation of fertilizers

##### 3.7.1.2.1. Agricultural Inputs (Fertilizers, Improved Seed, Pesticides etc)

3.7.1.2.1.1. Input utilization

The supply of agricultural inputs, particularly the production and distribution of high quality seed, fertilizers and others inputs are believed to be the most important factors to maximize production and productivity to attain food   self-sufficiency. To materialize it, government has taken concrete measures in increasing the availability and uses of the improved agricultural inputs.

Role of Stakeholders in Input Distribution

District MOA office

- Disseminate information and create farmers awareness  on input usage

- Organize, records and compile input demand of  each kebeles

- Participate in input demand analysis

- Together with other stakeholders , facilitates loan  collection

Credit and Saving Scheme

- Arrange all legal agreement regarding input disbursement

- Facilitates strategies  and methods  to cover  the input costs  procurement especially encourages poor farmers by covering input costs as accredit

- Collects  back loan given  to farmers after certain grace period

Cooperative Office

- In collaboration with other stakeholders ,it facilitates agreements with respective entities

- Forms ,organize and monitor basic cooperatives at each kebele

- Facilitates timely  availability of the inputs at each level

- Follows distribution of inputs according to the required procedure

- Conducts cash in cash payment of inputs for eligible farmers.

Among the commercial fertilizers, UREA and blended fertilizers utilization have shown increment over years though still not meeting the prevailing soil fertility and demand while DAP has decreased rapidly .The sole improved seed is maize variety and has shown increment over years though far below the prevailing variety demand.

On the other hand, the supply of pesticides  and insecticides have decreased from year to year

Table 11.Trend of input utilization in the district

| Inputs | unit | 2005/6 | 2006/7 | 2007/8 | 2008/9 | 2009/10 |
| --- | --- | --- | --- | --- | --- | --- |
| DAP | qt | 9464 | 889.5 | 4.5 |  |  |
| Urea | qt | 11488 | 12793.5 | 10306 | 17689 | 20329 |
| Other fertilizer (blended) | qt |  | 12,247 | 6874.5 | 10460 | 15152.5 |
| Herbicide | lit | 2000 | 6061 | 2651.5 | 5112 | 6842 |
| Insecticide | lit | 523 | 525 | 338 | 189 | 136 |
| Pesticide | lit |  | 5113 | 517 | 32 |  |
| Improved seed | qt | 1643 | 3108.25 | 2954 | 3594 | 37504 |

Source ; District cooperative office

3.7.1.2.1.2. Input rate

Some farmers usually disperse the commercial fertilizers to cover large area. Others apply only urea and no DAP/blended fertilizers or vise versal simply by erroneous guessing that the soil would be fertile if one of the fertilizers was applied a year or two years earlier. On the other hand ,time of application ,method of application for majority of crops are less maintained by majority of farmers .

With regards to seed, majority of farmers broadcast crop seeds such as sorghum, teff, sorghum, soybean, haricot bean  etc to get high rate of germination without maintaining appropriate seed rate.

Table 12. Input utilization rate (2009/10 E.C)

| Type of crop | Seed rate (kg/ha) |  |  |
| --- | --- | --- | --- |
|  |  | UREA | NPS |
|  |  | Rate (kg/ha) | rate (kg/ha) |
| Maize | 25 | 200 | 100 |
| Sorghum | 30 | 100 | 100 |
| Teff | 25 |  |  |
| Millet | 20 |  |  |
| Soybean | 60-90 |  | 100 |
| Ground nut | 120-160 |  | 100 |
| Haricot bean | 75 |  | 100 |

Furthermore, as the area is moist, crop diseases, insect pests and weeds usually get favorable environment to cause detrimental effect on crops but no effective control methods except some cultural practices are employed. This is partly due to less recognition of its effect or lack of knowledge regarding their control measures besides to high cost and non-availability of chemicals at the closest reach of farmers. Even, when some pesticides are made available, they are recklessly stored local shops, costly and   rate of application are less followed.

Table 13.Pesticide Utilization rate

| crop | Insecticide | Rate |
| --- | --- | --- |
| Maize | Malathion | 1lt/ha |
|  | Durisban | 1lt/ha |
|  | Dimethoate | 1lt/ha |
| Teff, Maize | 2,4-D | 1lt/ha |

3.7.1.2.1.3. Cost of Fertilizers, Pesticides and Improved Seeds

There is price fluctuation of inputs (fertilizers improved seeds, pesticides) depending on the following factors

- Scarcity of the inputs

- demand  of inputs  based on season

- absence   of efficient and competent suppliers at the area

- Fluctuation in the cost of production , packaging ,transportation  and storage of the inputs

- Relative  difference  in distance and transportation facilities within the kebeles

- Limited  government resource to sustain availability of  inputs  .

Generally existing input costs are indicated in the table below

Table 14.Existing input cost (2009/10 E.C)

| Type of inputs | Price at district | Remark |
| --- | --- | --- |
| DAP(birr/qt) | 1206.13 | Cooperatives |
| UREA (birr/qt) | 964.77 | Cooperatives |
| NPS (birr/qt) | 1206.13 | Cooperatives |
| Other (NPS B,NPSBZn) | 1206.13 | Cooperatives |
| Improved maize (birr/qt) | 3500-4500 | Cooperatives |

Source :District cooperative office

#### Livestock Population

Most agricultural production is used to meet consumption needs, for a very large number of households; there is a prolonged hunger season during the pre-harvest period. When there are surpluses, smallholder farmers are often constrained by lack of access to markets. In all farming systems, livestock are the single most important household asset and there is a strong correlation between lack of livestock ownership and poverty, particularly among woman-headed households.

Therefore, Livestock play a key role in a day-to-day life of the community, especially in the peasant sector. They provide meat, milk, transport, manure, skins, and hides and furnish regular and easily realizable cash income. In contrast to the size of the livestock population, physical and value productivity are low. Oromia takes largest portion of cattle production in the country, the benefit obtained from this large number of cattle population, is still shows much low. The reason could be due to the absence of appropriate livestock rearing with proper handling mechanisms and absence of sufficient feeds as well as proper health services.

The following table shows that the district is endowed with different livestock types like; cattle, sheep, Goats, Mules, and horses.Based on the available information from the district, the most important diseases affecting the livestock resources are black leg anthrax, Trypanosomisis, internal parasites and bacterial infection. Regarding animal health clinics, there is One (1) type B and five (6)-type D animal health clinics in the district that provides service with 21 VET Assistant personnel in the year of 2008E.C.

## 3.8.Existing Conditions of The Command Areas

### 3.8.1. Existing Cropping Pattern and Yield In and Around the Command Area

#### In  and around the  command kebeles ,major crops grown by rain fed  includes ,maize ,millet ,teff ,Noug ,sorghum ,haricot bean ,faba bean ,field pea ,chickpea etc, teff  is  the most dominant rain fed crop cultivated in and around  the command kebele

With regards to access to improved technologies, though improved agricultural inputs utilization is relatively more substantial in maize and teff   production, it is very rare for other crops   .For maize and teff still, massive production is using local varieties with organic fertilizers .Nevertheless, greater share of land in the command kebele is occupied by local varieties without any fertilizers.

Table15.Existing crop pattern of rain fed crops in and around the command area (2009/10 E.C)

| Crop | LV+OF | Percent | Yield(qt/ha) | CF+IV | Percent | Yield(qt/ha) | LV+CF | Percent | Yield(qt/ha) | LV+NF | Yield |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Maize | 3 | 14.5 | 20 | 3.75 | 27.3 | 53 | 2.5 | 8.2 | 20 | ND | 6 |
| Teff | 14 | 67.5 | 5 | 10 | 72.7 | 12 | 22 | 72.1 | 6 | ND | 2 |
| Haricot bean | 3.75 | 18.1 | 7 |  | 0.0 |  | 6 | 19.7 | 10 |  |  |
|  | 20.75 | 100.0 |  | 13.75 | 100.0 |  | 30.5 | 100.0 |  |  |  |

Source :DA office

The yield of crops shows decrement from cultivation using improved varieties and commercial fertilizers to cultivation with local varieties and chemical fertilizers to cultivation of local crop varieties with organic fertilizers .Least crop yield is obtained on areas where no fertilizer is grown with local varieties.

### 3.8 .2.Root and tuber crops, fruit crops and industrial crops

Although it is hardly possible to get figurative data about area and yield of the crops , root and tuber crops  and fruit crops are  grown with rain fed cultivation in and around the command area. Crops such as sweet potato, potato, yam, taro, garlic, onion and anchote etc and fruit crops such as avocado, banana and papaya and industrial crops such as sugarcane are planted at farmers backyards and  at  wet areas in and around the command kebeles .

It has been confirmed during the current study that   soil fertility and agronomic management measures like variety selection, plant spacing, weed management, disease and insect management, harvesting etc are not properly managed causing the yield to be low below the genetic potential of the crops .

### 3.8.3. Coffee Production

Coffee production is one of the agricultural occupations in the command kebele . However the following problems had been observed during the present study

- Less attention given to nursery site management including proper site selection ,variety selection  ,watering ,shading and pest management

- coffee management on the field such as has spacing  ,pruning and stumping and pest and disease management  problems

- Harvesting and post harvest management problems

- Lack of attractive pricing of coffee to encourage farmers

### 3.8.4. Irrigation Practices in and around the command Areas

Maize, potato and tomato   are among the dominant crops cultivated by irrigation  in and around the command area . During the field assessment maize was observed  to be   the dominant crop cultivated  using  residual moisture around the river basin (recession farming)followed by   potato  and some plots of sweet potato  .

According to the site observation and discussion with farmers  ,vast majority of the command area is virgin  that had been  formerly crop land  but currently left uncultivated due to lack of moisture during the critical crop growth .

As indicated in the following table ,potato(59.6%)  is most dominantly irrigated crop followed by maize (17%) in and around the command kebele .

irrigated crop productivity is low  because of improper crop and varietal selection ;improper crop rotation cycle; inappropriate cropping pattern and cropping intensity; inappropriate crop calendar; inadequate availability of improved farm tools and farm implements; poor land preparation and levelling; poor soil fertility management; poor input supply; poor irrigation scheduling/crop-water-requirement balance/water shortages ; inappropriate irrigation methods; and inadequate crop pest management practices.

Table 15.Existing cropping pattern and yield of irrigated crops at Nagassa kebele (2009/10 E.C)

| Crop | Area(ha) | Percent | Yield(ha) |
| --- | --- | --- | --- |
| Maize | 2 | 17.0 | 35 |
| Beet root | 0.5 | 4.3 | 110 |
| Onion | 0.25 | 2.1 | 120 |
| Tomato | 0.5 | 4.3 | 130 |
| Potato | 7 | 59.6 | 140 |
| Sweet potato | 0.75 | 6.4 | 130 |
| Local cabbage | 0.75 | 6.4 | 120 |
|  | 11.75 | 100.0 |  |

Source : DA office

### 3.8.5. Existing Farm Inputs and Agricultural Support Services

#### Soil Fertility Management

##### Traditional management practices

Traditional methods such as using cattle dung (manure), proper crop rotation, using of leguminous crops, fallowing, intercropping etc practiced   at very few areas in and around the command area. On the other hand, biological soil conservation like planting trees, agro forestry practices and physical conservation like terrace are observed in few areas  of the command kebele .

Agricultural Inputs (Fertilizers, Improved Seed, Pesticides etc)

Input utilization

Generally utilization of inputs in and around the command area is not matching to the number of farm households mainly because of less purchasing power of farmers, lack of credit facilities, less awareness creation etc.

Input rate

In and around the command area, appropriate rate of input is seldom followed and hence   blanket recommendation of fertilizers is applied.In some localities farmers usually disperse the commercial fertilizers to cover large area. Others apply only urea and no DAP /blended or vise versal simply by erroneous guessing that the soil would be fertile if one of the fertilizers was applied a year or two years earlier. Generally, even though generalized recommendation is made, some farmers occasionally follow it. With regards to sowing and planting methods for rain fed crops ,majority of the crops are broadcasted except maize(95%) and millet(20%) and pepper(80%)row planted .The source of seeds for most of the crops are  local varieties saved from previous harvest ,purchase from market etc ,thus the quality of seeds are highly inferior causing low  germination and subsequent crop growth and low yield

Table 16.Rate of input utilization

| Type of crop | Seed rate (kg/ha) |  | Commercial fertilizer |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | DAP/NPS |  | UREA |
|  | If row planted | If broadcasted |  | rate(kg/ha | Rate (kg/ha) |
| Maize | 25 |  |  | 100 | 200 |
| Teff | 3.5 | 35 |  | 100 | 100 |
| Haricot bean | 75 |  |  | 100 |  |

Input cost

As indicated on the table below ,cost of input is one of the bottleneck for increasing production and productivity of both rain fed and irrigated crops .It is inconsistent and ever increasing from time to time .Besides there are almost no support services to give credit services especially during peak input demand for rain fed and irrigated crops.

For the commercial fertilizers and improved maize seed the source of supply is local basic cooperatives through Gibe Dhidhessa union .However, chemicals for crop protection and vegetable seeds are mainly from local merchants at and around Nekemte area .

Table 17. Cost of  Fertilizers, Pesticides and Improved Seeds (2009/10 E.C)

| Input | Cost (birr /unit ) |
| --- | --- |
| DAP | 1206 birr/qt |
| Blended | 1206 birr/qt |
| Urea | 965birr/qt |
| 2,4-D | 200birr /lt |
| Round up | 280 birr/lt |
| Glycel | 150birr/lt |
| Glymax | 180birr/lit |
| Mancozeb(India) | 200birr/kg |
| Ridomil Gold mz68 WG | 800birr/kg |
| Primagrim gold | 400birr/lt |
| Tomato | 800birr/kg |
| Onion | 500birr/kg |
| Pepper | 450birr/kg |
| Improved Potato | 1200birr/qt |
| Improved maize | 3500birr/qt |
| Beetroot | 400birr/kg |
| Carrot | 400birr/kg-448birr/qt |

Labor cost

The number of man days required per hectare for each activities per its frequency for  each crops are multiplied by the local unit rate of labor cost  to come up with total payment /cost of labour for each crop per hectare. Here, average labour cost per day is 30 birr per day to be multiplied by activities frequency to get total cost per the frequency for each crop.

Table 18.Labor Cost (birr/ha)  for Existing Major Crops (rain fed)(/2009/10 E.C)

| Activities | Maize | Sorghum | Teff | Wheat | Millet |
| --- | --- | --- | --- | --- | --- |
| Land preparation | 600 | 400 | 600 | 400 | 600 |
| Sowing and input application | 400 | 400 | 400 | 330 | 430 |
| Weeding | 1080 | 720 | 400 | 360 | 430 |
| Harvesting | 430 | 430 | 500 | 500 | 430 |
| Total | 2510 | 1950 | 1900 | 1590 | 1890 |

Source: Project Agronomy survey (2018)

Table 19.Labor Cost per hectare for Existing Major Crops (irrigated) (2009/10E.C)

| Activities | Potato | Tomato | Onion | Maize | Head cabbage | Pepper | Carrot |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Land preparation | 600 | 430 | 600 | 600 | 430 | 430 | 430 |
| Sowing and input application | 500 | 430 | 500 | 500 | 430 | 430 | 400 |
| Weeding | 630 | 630 | 630 | 1080 | 760 | 1200 | 400 |
| Irrigation | 630 | 760 | 760 | 630 | 760 | 630 | 630 |
| Harvesting | 430 | 600 | 600 | 430 | 600 | 800 | 430 |
| Total | 2790 | 2850 | 3090 | 3240 | 2980 | 3490 | 2290 |

Source: Project Agronomy survey (2018)

Plowing Oxen cost: The traditional plough (Maresha) using oxen power and small farm tools by human power are respectively used for land preparation and sowing. Time of land preparation and subsequent sowing may delay due to lack of plowing oxen , labor shortage, seed and other input shortages. For the current purpose, the conventional tillage using pair of oxen is used for existing oxen day cost per each crops.

Table 20.Oxen Day   Cost for Existing Major Crops (Rain fed crops) for plowing with oxen (2009/10 E.C)

| Crops | Land preparation | Sowing | Sub total |
| --- | --- | --- | --- |
| Activities |  |  |  |
| Maize | 1440 | 480 | 1920 |
| Sorghum | 960 | 480 | 1440 |
| Teff | 1440 | 480 | 1920 |
| Wheat | 1440 | 480 | 1920 |
| Millet | 1440 | 480 | 1920 |

Source: Project Agronomy survey (2018)

Furthermore, frequencies of tillage among many factors depend on oxen power and level of awareness. For calculating oxen days, average numbers of frequencies for the activities are multiplied by number of oxen required for a hectare and local unit oxen cost per day 60birr to come up with total oxen days per hectare both for rain fed and irrigated agriculture.

Table 21: Oxen Day Cost for Existing Major Crops (irrigated) (2009E.C)

| Activities | Land preparation | Sowing | Sub total |
| --- | --- | --- | --- |
| Crops |  |  |  |
| Tomato | 960 | 640 | 1600 |
| Onion | 960 | 640 | 1600 |
| Head cabbage | 960 | 640 | 1600 |
| Maize | 1280 | 640 | 1920 |
| Potato | 960 | 640 | 1600 |
| Pepper | 960 | 640 | 1600 |

### 3.8.8. Existing Agronomic Constraints

#### 3.8.8.1. Inadequate Supply and Improper Input Utilization

##### 3.8.8.1. 1. Improved Crop Varieties

Seed is a basic input for crop production. The quality of seed has the most profound influence on crop yields. The farmers of project command have been using their own seed saved from the recurrent crops. They have less access to certified seed of genetically superior cultivars mainly due to limited seed multiplication programs, poor extension activities and high cost of such seeds. No seed enterprises are actively working on seed multiplication in the area, accessibility of small holder farmers to these improved seed is very rare except maize besides to low awareness on improved seed utilization. Farmers have no idea of the viability and germinability of such   seeds. Thus, field germination, seedling vigor and plant population is affected adversely resulting in poor productivity.

Generally, most of the field crops, pulse crops, and fruit crops are local varieties.  Some of the formerly introduced improved crop varieties are lacking varietal stability. Generally most of the cultivated crops   are local varieties and in the irrigation command area in particular and accessibility of farmers for   field crops and horticultural crops   of high quality is very less due to unavailability at closest rich of farmers and high cost of the seeds from the nehiboring town.

3.8.8.1.2. Distribution, Rate, Time, Placement, handling and storage of

Fertilizers

According to the information obtained from sectoral office discussion and key informants, the following main problems are reported

##### Distribution of fertilizers

The soil fertility depletion in small holders’ farms in and around the command area is becoming the fundamental biophysical limiting factor responsible for declining of food crop productivity due to repeated cultivation for crop production. Hence, fertilizer usage is the fundamental option to raise crop productivity in majority of the command area .However; farmers usually do not use full packages of fertilizers for rain fed crops and rarely use fertilizers for irrigated crops due to the following reasons

- Less  research, farmers extension   linkages and training support services  on the use of fertilizers and  crop productivity particularly for irrigated crops

- Farmers have acute shortage of agricultural credit  and less eligibility  to    get the inorganic fertilizers through credit

- Less saving culture of the community and lack of money to buy these inputs during irrigation seasons.

- Unfavorable relation between the crop prices and fertilizer cost etc .The fertilizer and related cost is very high while market price of crop grains are very low

Blanket Fertilizer Recommendation

It is the matter of fact that fertilizer application greatly improves crop productivity if the recommendation is based on the local soil fertility assessment. Soils may be highly exhausted and need more nutrients even other than macronutrients but other may need less fertilizers

.However, because of capacity limitation, it is impossible to conduct it at all localities .As the result, generalized recommendation of fertilizers is made resulting in the inefficient and ineffective fertilizer utilization and subsequent yield decrement ..

Utilization of Fertilizers below the Recommended Rate (Below Full Package)

In some of the study area, some farmers simply broadcast the commercial fertilizers below the recommended rate per hectare to cover large area and use the remaining for other crop field .This in many cases is due to the high price of fertilizers unaffordable by the small holders or low awareness about the rate of fertilizers to be utilized. For the same reason, at some  localities,  some  farmers  apply  only  urea  and  no  DAP /blended  or  vise  versal  by  wrong assumption that the soil would be fertile and doesn’t need fertilizer if they have already applied one of the fertilizers a year or two before. Basically, it is recommended that urea should be applied in split application based on the crop growth stage for most of the cereal crops .But, due to low awareness creation on scientific recommendation or lack of labour and time, split application of urea at the correct crop growth stage is not followed among some farmers or even it can be left not applied.

Placement of Fertilizers

Probably due to various factors, some farmers broadcast the fertilizers not properly covering with soil .This results in poor fertilizer use efficiency because of ammonia volatilization when they apply urea fertilizer.

Localized placement of phosphorus fertilizers might include row, band, or strip placement. It is generally presumed that a localized or band application  reduces  fertilizer  contact  with  the  soil  thereby  resulting  in  less  phosphorus sorption and precipitation reactions thus; enhance availability to crops .

Handling and Storage of Fertilizers

In the study area of the sub basin, fertilizer quality problem especially DAP was repeatedly reported among the users .These quality problems arise mainly due to the reckless handling, storage and transportation. Therefore, it is  recommended  to  follow  the  following precautions

Fertilizer Handling

Handling is an activity, which can be defined as the manual and/or mechanized movement under during the transport of fertilizer from factory or port to the end user.

  Bags must be treated so that it should not deteriorate causing spillages loses in their content.

  Care should be taken when lifted, loaded or unloaded

  Handling  methods  which  result  in  damage  to  bags  when  using  hooks  must  be eliminated

  Damaged bags spill out fertilizer contents and exposed to humidity, rain and sun.

  Check vehicles before starting to load (nails, splinter can be removed

  Lay water proof paper, matting or straw to protect the package

  Bags stacked need the doors; badly stacked bags can slip during transit over rough roads or even in the store itself.

Bagging

Correct packaging protects the fertilizer against humidity and bad weather conditions; it eases handling transportation and storage; it simplifies identification and finally it eases use by the end user.

Storage

The following should be considered in storage

	Keep fertilizers in an orderly fashion

	Stack the bags properly

	Leave a space (of 10cm) between each bag to provide ventilation

Make  allowance of a central corridor (1m wide) to facilitate movement and handling of bags in the store.

Allow space (20-30) cm between the stacks and the walls of the store to prevent the fertilizer coming into contact with damp walls.

	Avoid direct contact between the fertilizer and the soil (earth floors)

	Install a false floor which isolates the stack of fertilizer from the moist soil and at the same guarantees aeration (bricks, wooden slats, etc)

	Use locally available materials; such as bamboo mating, rice hulls, straw, old sacks, etc).

	In  the  case  of  ammonium  nitrate,  on  account  of  the  explosion  hazard,  it  is  not recommended to stack this fertilizer on top of organic materials such as straw etc.  It can be stacked, however, on a wooden platform.

	Where bags are stacked near windows facing the sun and liable to be exposed to prolonged sunlight, it is recommended that the window should be darkened by using news paper or old sacks, or by painting the pens of the glass black.

	In periods of high humidity, ventilators, opening windows and doors must be kept firmly shut.

	The store should only be ventilated during certain hours of the day when the ambient humidity is low and the temperature is high.   In this way, condensation of water vapors in the store will be avoided.

	Fertilizers should always be stored on a floor, that is kept by continually brushing away any spilled fertilizer or other spilled material, because they hold the dumped cause deterioration of the floor and/or the walls.

3.8.8.3. 3.Scarcity of Agrochemicals

As the command area is an area having high humidity, crop diseases and insect pests and weeds get favorable environment to occur. There is no effective control methods except some cultural practices employed at some localities .On the other hand, the crop varieties used are mostly the local varieties which  are easily susceptible to pests and diseases  .

The chemicals  for pest and disease  control are not  available at the reach of farmers .Some chemicals such as 2,4D are available through local merchants but the price of chemical is costly and non affordable by the small holders .Besides, the chemicals are not safely handled by these bodies as they are sometimes recklessly stored in local shops and sold to farmers without attempting manufacture date, expire date and other safety standards. It is hardly possible to follow these farmers and support technically on how to apply, when to apply, where to apply, at what dosage and all the management practices during and after chemical application.

Thus the following major points need critical attention of all the stakeholders

  There  should  be  a  way  through  which  majority  of  farmers  get  access  to  chemicals at affordable prices

  As chemicals are hazardous to humans , animals , birds ,bees , fishes and aquatic animals etc, only safe chemicals should be used

  These chemicals should be utilized with the knowledge and follow up of technical experts

 Integrated  pest management should be advocated

Moreover, the  next  requirements  should  be  considered  for  sustainable  pesticides management

  Choosing application site

  Labeling procedure should be followed

  General packaging requirement

  Storage requirements

  Transportation requirements

  Pesticide application methods

  Health and safety measures

  Action in case of pesticide poisoning

  Treatment / Disposal Methods

##### 3.8.8.1. 4. Scarcity of Improved Farm implements and Pumps

The indigenous people of the area use very traditional and outdated farm tools like” gasso”,and Maresha (drawn with oxen ), sickles spades, etc. The less utilization of improved farm implements and tools have reduced crop productivity due to improper field preparation, delay in sowing, harvesting and other farm operations. Moreover, there is scarcity of pumps to exploit river that is suitable for pump irrigation in the command area .Thus, it is advisable to group the farmers and financially capacitate in order to satisfy the existing need of improved farm tools , machineries and pumps etc.

3.8.8.5.2. Inadequate Extension Service

The agricultural technologies should be able to address the problems of farmers that arise during production. However, such extension services are inadequate in the command area due to insufficient experts that are experienced.

On the other hand, there are less staffing of irrigation office at Woreda and command area with experts such as irrigation agronomists, horticulturalists, crop protectionists, and other technical and support staffs not fully supported by budgetary and logistics to execute irrigation activities.These shortages create serious gap on transferring  improved technology to farmers and promote crop production and specifically irrigated agriculture.

Farmers training center /FTC/ is not well equipped with required teaching materials and logistics. Short-term package training and modular form of training programs are not well exercised in the area. Moreover, on farm demonstration sites are not well established in order to increase the farmers’ awareness about different crop production technologies.

Generally the existing agricultural extension service in its expertise quantity and quality and infrastructural facility does not meet the present requirements for promoting and improving rain fed and irrigated crop production in the command area, requiring full support from concerned stake holders at all levels.

##### 3.8.8.5.3. Poorly Developed Research-Extension linkage

The constraints of farmers need to be alleviated by dissemination of the latest production technologies developed by the research centers through a strong and well-coordinated system. Even though the previous research and extension activities are not undermined, currently no research or trial sites are available in the command area regarding crop production and irrigated agriculture in particular. The already proven technologies are less disseminated and farmers’ orientation towards new innovations especially in irrigation is less satisfactory. In general, the research-extension linkages are very weak and disorganized

##### 3.8.8.5.4. Inadequate Rural Credit

It has already been indicated that the cost of inputs such as fertilizers are highly escalating but the market  price of crop grains are very low .Farmers need to purchase these  different  farm inputs, farm tools, and other related equipments but short fall of the necessary capital .With this respect there are no adequate micro finance institutions to supply  credit for the  small holder farmers’ except  very few  credit services in the command areas .But, their role with regards to the needy community is relatively insignificant because;

- The number of community needing the credit is very large compared to the support being supplied

- high interest rate of the loan

- There is poor saving culture in the community

- Limited involvement of  credit suppliers and  inadequate infrastructural development

Hence these constraints negatively affected the farming community to utilize improved agricultural technologies

##### 3.8.8.5.5. Marketing Problem

The inputs marketing have been impeded by rapidly escalating prices, inadequate and untimely supply, and poor access to credit facilities, involvement of limited suppliers and inadequate infrastructural development. For healthy marketing, there is a need to empower cooperatives along with encouragement to free market and proper infrastructure development and timely delivery of adequate inputs. Adequate marketing channels and sufficient market centers for raw and processed crop products should be made available especially for irrigated crops . Necessary inputs, extension services and credit facilities are required for the farmers to enhance their capacity to produce crops and sell products continuously.

Generally, agricultural output/grains marketing is constrained and very low crop prices are exercised mainly due to

- Farmers acute shortage of money especially for schooling ,buying of agricultural inputs etc forcing them to sell at low price and less availability of credit giving organizations

- Weak linkages between agricultural marketing (crop marketing desk and agricultural cooperatives  office and crop agronomy  etc regarding crop marketing )

- Weak and unorganized multipurpose  cooperatives to buy /collect crop products on time with reasonable market price

- Lack of competition/limited market participants and capital

- lack of agro-processing facilities at the reach of farmers and related infrastructure

- lack of modern storage structures etc

##### 3.8.8.5.6 Weeds, Insect Pests and Diseases

Weeds: Weeds damage crop growth and subsequently reduce crop yield and quality .cyperus Esculentus, cyperus rotundus,Guizotia scabra,Amaranthus ,cynodon dactylon  ,Bidense  pilosa ,digitaria velutina are some of the major weeds existing in and around the command area .

###### Insect Pests: Insect pests injure plants by chewing leaves, stems and roots, sucking juices, laying eggs or transmitting disease and in the process significantly reduce crop yields and quality. Major insect pests regularly and sporadically occurring in and around the command area are African boll worms, hoppers, aphids, stalk borers, cut worms etc

Crop diseases: existing crop diseases in and around the command area includes maize leaf blights, rust on teff  , Early blight of tomato ,late blight of potato ,fusarium wilt of pepper ,etc .

##### 3.8.8.5.7. Pre and Post Harvest Handling Problems

The following problems are generally observed along the arrays of crop production

- poor cluster farming

- less  raw planting ( recommended crop space) for  major crops

- Poor land preparation  methods

- Poor weeding/timely and frequently

- Un appropriate fertilizer application (rate ,placement ,time of application etc)

- Inefficient  harvesting and storage  methods

- In effective irrigated crop production  due to poor watering for irrigated crops (no use of appropriate crop water requirement ,irrigation interval)  ,appropriate plant population ,pest management and other agronomic practices and crop management

Generally pre harvest agronomic  management operations like land preparation ,planting /sowing  ,input use,  crop protection ,harvesting  and post harvest operations such as  threshing, cleaning, grading, drying, curing, preservation, storage, processing, packaging, transportation, marketing and consumption systems are not satisfactorily  in and around the command area .

### 3.8.9. Production Potentials

The existing farming systems being practiced in the project command is very traditional leading to low level of output. Through proper utilization of resources, particularly the water resources and potential irrigable land harnessed with government development policies and strategies and available unskilled labour along with solving other major agricultural constraints mentioned above, the irrigated agriculture has  to be fully supported and the respective and relevant back up services must be strengthened to enhance the current low level of production and productivity .

## 3.9. Proposed Irrigated Agriculture

### 3.9.1. Benefits of Proposed small Scale Irrigation project

The implementation of the project will provide irrigation water to an area. This entails incremental production and diversification of crops. The production of diversified crops including high value crops improves cash income of the community and provides raw materials for processing industry and thus will improve the overall livelihood of population.

Apart from the scheme's beneficiaries, many other families in the surroundings will benefit from increased food crop production and productivity and possible labor opportunities in the irrigation fields due to the high cropping intensity requiring additional labor that provides employment opportunities throughout the year which would have been idle otherwise.

New farming practices and improved technology will be provided  through the extension services of the project to modernize agricultural production in the area  in such a way that  it  will also serve as demonstration of irrigation practice to the surrounding farmers.

The introduced small scale irrigation project may cause improvement of infrastructure in the area and other social institutions. It also advances the potential of livestock production and related income as a result of increased biomass from intended cropping intensity.

### 3.9.2. Agricultural Development Scenarios and Options

The major target of the irrigation development study at any level is to prepare a sustainable project for the planners and decision makers for implementation. This can be achieved if agricultural development scenarios and options are seen from various angles to choose possible crop mix and farming enterprises available.

#### 3.9.2.1. Basis of Agricultural Development Scenarios and Options

Crop Selection: A key factor affecting the economic and financial viability of irrigation is selection of crops that are capable of creating high enough returns to meet the capital and recurrent costs of irrigation development.

In the study area, rainfall is  sufficient to produce one rain-fed crop per year to produce different grain crops and some horticultural crops .However to benefit from irrigated agriculture; selection of horticultural  crops, high value crops, pulse crops and food crops   to be grown under irrigation will make irrigation more beneficial.

Land Allocation and Re-Allocation:  according to the current assessment, land holding of farmers in the intended irrigation project shows less similarity .Thus based on the rules and regulations of usage of irrigated land utmost effort would be done to allocate or re allocate irrigation land in ensuring that project benefits grow fairly equitably, and environmentally sound so that the irrigation land holdings would be similar, or at least commensurate.

Agro-Industries: Some of the selected crops have a potential to be the raw materials for small agro industries though the processing type and requirement is determined by the kind and quantities of crops cultivated in the project area. There is good market condition for the processed products as the project area is located at an ideal place for regional / national  markets.  If the processing industry is technically and financially not feasible, the possible option could be establishment of storage facilities and selling the raw crops in local and regional  market vis-à-vis establishment of agro industries to use as a raw material.

Livestock and Irrigated Pasture: - The contribution of livestock to crop production can be assumed directly as a source of draught power and organic fertilizer besides to milk, milk products and beef etc, indirectly, as a source of cash to buy crop production inputs. Oxen and donkeys are the main traction powers for cultivation.

With the increasing trend of human and cattle population, existing land size may not support this increasing livestock population. Therefore, the recommended option would be integration of irrigated crops and livestock pasture in the irrigation system.

The feeding arrangement may include among many, establishment of irrigated pasture and management of natural grazing lands through the introduction of improved forage tree species. Some of these pastures can also be used as hedge rows, strip planting and under sowing . On the other hand, crop residue as another source of animal feed obviously increases because of introduced irrigation practices.

Irrigation and Apiculture: For bee keeping farming to be best flourishing, there is environmental requirement level by this utilization type. Unless this use type gets its own niche, the community using it may not earn the possible benefit expected from this area. Such problem arises dominantly from the misunderstanding of integrating irrigation and apiculture. This is true in the study area when the intended irrigation development is put in place; during implementation, it might abandon the rich habitat and food source of the apiaries. This situation can be minimized if the following intervention measures are applied.

- Integrated Pest Management (IPM) approach to be followed to avoid each irrigation activities not to thrive at the expense of the bee keeping.   .

- Continuous training to be given to the community involved in irrigation and bee keeping to create mutual understanding on the activity they are running

Besides, it is advisable to protect some of the natural tree species not to destruct for irrigation purpose so that conducive and sustainable environment for the area and for honey production in particular is created.

### 3.9.3. Integrated Agronomy Development Plan

Arable Farming: With availability of assured irrigation, the primary emphasis would shift to rising of high yield potential, fertilizer responsive, stable performing and irrigation responsive crops best suited to the agro-climatic conditions of the project command area.

With the cropping system, the next enterprise having the highest synergy with arable farming are dairy and beef, agro-forestry, horticultural systems and apiculture.  All these would need a thorough degree of integration to boost the income of the farmers, recycle the farm produce and by-products locally for further value-addition and provide an-all year-round employment to the farm families.

Agro-forestry: Agro-forestry refers to growing of some quick growing tree species of short gestation periods in association with cultivated crops, in between the tree-rows or in alley cropping configuration to provide and supplement the farmer’s need for fuel wood, timber and fodder crops by integrating fodder crops with forestry tree-species. These could be combined effectively on those pieces of lands which otherwise might not be that suited for arable cropping.  The agro-forestry plantation could be planted at wider spacing to permit growing of cultivated crops for the initial few years.

### 3.9.4. Selection of Crops and Cropping Patterns

A cropping pattern shows crops that are grown, cropping intensity and percentage of each crop grown, when it is planted and when it is harvested.

#### 3.9.4.1. Criteria’s for Selection of Crops and Cropping Pattern

Among many selection criterions set for irrigated crops and cropping pattern, the following are the major decisive factors

Climatic Suitability of the Area: According to the current assessment, the climatic situation of the project area is not a limiting factor for production of the selected crops.

Soil: Plant response to irrigation is influenced by the physical condition, chemical and biological status of the soil. Texture , structure , depth , organic matter , bulk density , salinity ,sodicity ,drainage ,fertility and other  chemical characteristics all determine the extent to which a plant root systems grow into and uses available moisture and nutrients in the soil .Therefore most of these soil parameters are suitable for the selected crops in the project area.

Market Condition and  Market Value of Crops. It is clear that there is massive population in Nekemte and other nearby towns and cities needing availability of horticultural and other irrigated crops .Thus, with the expectation of the improvement of existing market mechanisms, agro industry , transport, purchasing power etc, some of the selected crops could get regional /national market to improve the income and livelihood of the community.

Raw Materials for Agro Processing: Utmost effort has been given to include crops that are used as the raw materials for   agro processing

Agricultural Knowledge of Farmers and Attitude of Farmers:   the communities in the project area engaged dominantly in crop production besides to animal raring .Additionally, though there is no modern irrigation scheme in the project area, majority of farmers have good attitudes on irrigation.

Crop Diversification and Soil Fertility Management: Crops diversification helps to minimize the following risks in crop production

- Crop selective pest out break and associated yield lose

- Crop yield lose due to less adaptability to any climatic change

- Yield lose due to less soil fertility and continuous mono cropping

- Water shortage at critical water demand of crops

- Food shortage and associated nutritional and economic problem

Availability of Agricultural    inputs (Seeds, Fertilizers and Chemicals etc): Based on the government direction of boosting agricultural production; manufacturing, distribution and utilization of agricultural inputs is getting momentum. Thus, if the farmers’ knowhow and the market conditions of the inputs get improved, availability of agricultural inputs contributes to improve production and productivity of the crops.

Diseases, pests, rodents, and wild animals attack on crops: Even though there are some crop diseases, pests and rodents and wild animals attack, they are not above the economic threshold level to control .Therefore if required management practices are put in place, the selected crops do not encounter significant problems.

Infrastructure condition: The existing asphalt roads passing through Nekemte besides to other infrastructures are some of the hopes for selection of the crops.

### 3.9.5. Crop basket selection

Generally, the following crop baskets are listed to select most recommended crops for the scheme  .These are maize, sorghum ,millet ,teff  ,wheat ,barley ,head cabbage, carrot ,beetroot ,switchyard ,lettuce ,onion ,garlic ,potato ,tomato , banana ,papaya , pine apple ,sweet potato ,sesame , ground nut , rice , sugar cane , cotton , mango , citrus , Avocado ,coffee ,rice ,Noug, soybean ,sunflower , mung bean ,safflower ,pepper ,haricot bean, yam , anchote ,cassava ,alfalfa ,leuceanea ,pastures(annual and perennial)etc.

However, based on a number of selection criteria, only the following crops are chosen for the scheme. The selected crops includes maize, onion, sugar cane  , head cabbage, tomato, carrot, Swischard ,pepper, potato, sweet  potato, banana  and pasture (forage) crops

### 3.9.6. Crop Water Requirement

The water requirement of a crop depends on the climate. Under the same condition different crops require different amount of water and the quantities of water used by particular crop vary with its stage of growth.  Initially during seeding, sprouting and early growth a crop uses water at a relative slow rate. The rate will increase with growth of crop reaching the maximum in most crops as it approaches flowering and then decline towards maturity.For the purpose of calculating ETO Nekemte meteorological station had been used

There are different formulas to calculate ETO, but Penman-Monteith method is considered to be the most accurate method for estimating ETO though it requires relatively more data than others. The method is considered to offer the best results with minimum possible error in relation to a living grass reference.

The formula used in penman method is

ETO =c (W.Rn+ (1+w). f (u). (ea.ed)

Where: ETO = reference evapo-transpiration in mm/day

W = temperature related weighing factor

Rn = net radiation in equivalent evaporation in mm/day

f (u) = wind related function

(ea _ ed) = difference between the saturation vapour pressure at mean air temperature and the mean actual vapour pressure of the air both in m bar.

C = adjustment factor to compensate for the effect of day and night weather condition.

#### 3.9.6.1. Determinination of Crop Evapotranspiration (ETcr)

After determining the reference potential Evapotranspiration by penman method, the next step is calculating crop water requirement/design supply for the scheme.

ETcr can be calculated from climatic data by directly integrating the effect of crop characteristics into ETO. Experimentally determined ratios of ETc/ETo, called crop coefficients (Kc)are used to relate ETcr to ETO.

ETcr is calculated using FAO CropWat version 8

#### 3.9.6.2. Crop Coefficient (Kc)

It shows the effect of the crops characteristics on crops water requirements. Kc presents relationship between reference ETO and crop evapo-transpiration. The values of kc are varying with the crop, its stage of growth, growing season and the prevailing weather condition.

#### 3.9.6.3. Length of Crop Growing Stage

During their growing stage crops, vary in their water consumption. These variations have been generally grouped in to four. Namely: initial stage, crop development stage, mid season and late season stages.

- The initial stage: is germination and early crop growth, where the ground cover is less than 10%.

- Crop development stage: from end of initial stage to full ground cover usually between70 to 80% of the land surface.

- Mid season stage: from attainment of full ground cover to the start of maturity indicated by leaf senescence or loss.

- Late season stage: from the end of the mid season stage until harvest.

#### 3.9.6.4. Effective Rain Fall

Effective rain fall is the portion of a rain, falling during the growing period of the crop which is available to meet the consumptive water need or the evapo-transpiration requirement of the crop. It does not include precipitation loss due to deep percolation below the root zone. It is calculated Using CropWat version 8

The area receives minimum of 10 mm to maximum of 398mm rainfall   per month and the mean annual rainfall is 2089.4mm.The mean annual effective rainfall is 1434.4mm.

Figure 6effective rainfall

Dependable rainfall (FAO/AGLW formula)

Effective R =(Total R*0.6) -10 when total R < 70mm/month

Effective R = (total R*0.8) -24 when total R> 70mm/month

#### 3.9.6.5. Climate and ETO

As it is already mentioned, Nekemte meteorological station has been used for ETO analysis .ETO is relatively higher at the months of February March and April in which case ,the highest  ETO is in  march . At these months, the area experiences highest temperature and maximum radiant energy.

Table 22.Climate and ETO

Station  :   Nekemte ; Altitude  :2080m   Latitude  90N  ;  Longitude 360 E

| Month | Min Temp | Max Temp | Humidity | Wind | Sun | Rad | ETo |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | °C | °C | % | km/day | hours | MJ/m²/day | mm/day |
| January | 12.2 | 26.1 | 62 | 130 | 8.5 | 20.1 | 3.98 |
| February | 13.5 | 28 | 57 | 147 | 8.4 | 21.2 | 4.59 |
| March | 14.3 | 28 | 58 | 147 | 7.8 | 21.3 | 4.74 |
| April | 14.5 | 27.4 | 65 | 138 | 7.8 | 21.5 | 4.62 |
| May | 13.9 | 24.9 | 79 | 130 | 6.1 | 18.5 | 3.73 |
| June | 12.9 | 22.5 | 87 | 95 | 4.9 | 16.3 | 3.08 |
| July | 12.8 | 21.1 | 89 | 104 | 3.1 | 13.8 | 2.6 |
| August | 12.9 | 21.1 | 90 | 95 | 2.9 | 13.7 | 2.57 |
| September | 12.9 | 22.5 | 88 | 104 | 3.6 | 14.8 | 2.79 |
| October | 12.9 | 24 | 79 | 147 | 6.7 | 18.8 | 3.57 |
| November | 12.6 | 24.5 | 73 | 147 | 7.5 | 18.8 | 3.64 |
| December | 12.1 | 25 | 68 | 147 | 7.8 | 18.6 | 3.67 |
| Average | 13.1 | 24.6 | 75 | 128 | 6.3 | 18.1 | 3.63 |

Table 23.Crop Parameters for Water Demand Analysis at the Area

| Crop | Growing period | Days of Growing stage |  |  |  | Crop Coefficient |  |  | Root depth | Depletion | Yield factor |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Days | D1 | D2 | D3 | D4 | Kc1 | Kc2 | Kc3 | mm | P | Ky |
| Banana | 240 | 60 | 60 | 75 | 45 | 1 | 1.2 | 1.1 | 900 | 0.48 | 1 |
| Maize | 165 | 25 | 45 | 50 | 40 | 0.3 | 1.2 | 0.35 | 1000 | 0.63 | 1.25 |
| Pepper | 125 | 30 | 35 | 40 | 20 | 0.6 | 1.05 | 0.9 | 800 | 0.33 | 1.1 |
| Potato | 130 | 25 | 30 | 45 | 30 | 0.5 | 1.15 | 0.75 | 600 | 0.35 | 1.1 |
| Sugarcane | 365 | 30 | 60 | 180 | 95 | 0.4 | 1.25 | 0.75 | 1500 | 0.65 | 1.2 |
| Tomato | 145 | 30 | 40 | 45 | 30 | 0.6 | 1.15 | 0.8 | 1000 | 0.4 | 1.05 |
| Onion | 110 | 20 | 35 | 35 | 20 | 0.5 | 1 | 0.85 | 600 | 0.42 | 1 |
| H.Cabbage | 165 | 40 | 60 | 50 | 15 | 0.7 | 1.05 | 0.95 | 500 | 0.45 | 0.95 |
| Pastures | 365 | 140 | 60 | 120 | 45 | 0.85 | 0.95 | 0.85 | 800 | 0.5 | 0.8 |

#### 3.9.6.6. Calculation of Irrigation Requirements

Monthly irrigation water requirement was calculated using the mean monthly rainfall data of Nekemte meteorological station and crop data as shown in the table. This calculated irrigation water requirement is the basis for determination of the system capacity

Table 24.Cropping calendar and Crop Water Requirement for the scheme

| Crop name | Planting date | Harvest date | Total (crop season ) |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | ETC(mm)/dec | Eff Rain (mm)/dec | Irr. Req.(mm/dec) |
| Sugarcane | 15/11 | 14/11 | 1339.7 | 1435 | 639.5 |
| Maize | 15/10 | 23/03 | 536.8 | 67.6 | 493.8 |
| Pepper | 19/10 | 20/02 | 419.9 | 59.5 | 367.2 |
| Banana | 20/11 | 12/7 | 1034 | 694.2 | 681.2 |
| Potato | 10/10 | 16/02 | 451.9 | 77.1 | 397.9 |
| Tomato | 10/10 | 3/3 | 528.5 | 77.9 | 466 |
| Head Cabbage | 25/11 | 28/04 | 635.1 | 59.9 | 569.7 |
| Pasture | 15/11 | 14/11 | 1157 | 1435 | 571.2 |
| Onion | 5/11/ | 14/03 | 357.5 | 11.3 | 347.5 |

#### 3.9.6.7. Irrigation Efficiency (E)

The irrigation efficiency expresses the percentage of the quantity of water used efficiently for the growth of the crop in the field to the quantity of intake water from the water source. A part of the intake water is lost during transport through the canals and in the fields by the evaporation, the infiltration and so on. The remaining part is stored in the root zone and used by the crop. The irrigation efficiency is computed by the following formula.

e = ec * ea / 100

Where

e : Irrigation efficiency (%)

ec : Conveyance efficiency (%)

ea : Field application efficiency (%)

The conveyance efficiency is the ratio of the quantity of water supplied to the fields out of the quantity of intake water from the water source. It represents the efficiency of water transport in the canals. The conveyance efficiency mainly depends on the length, the soil type or permeability of the canal banks and the condition of the canals. Based on  the soil  texture and canal condition is unlined, the conveyance efficiency will be 70 %( Guideline for The field application efficiency is the ratio of the quantity of water used for the growth of the crop out of the quantity of water supplied to the fields. It represents the efficiency of water application in the field. The field application efficiency mainly depends on the irrigation method and the level of farmer practice.

Field application efficiency assumed to be 60% since the method of irrigation to be used is surface irrigation with furrow method (Guideline for Irrigation Master Plan Study Preparation on Surface Water Resources, 2014)

Therefore, for this project, overall efficiency, E=0.70X0.6=0.42= therefore   42 % -45% efficiency can be used.

#### 3.9.6.8. Scheme supply

As shown in water requirement analysis, the net water requirement of each crop is divided by the overall efficiency 45% at to obtain the gross water requirement. The project supply of the  irrigation project for  full irrigation  is indicated for 24 hour irrigation and becomes  1.02 L/s/ha at the month of January and 0.6l/s/h for 24 during supplementary (season 2)irrigation at the month of April.

Table 25.Scheme Supply for full irrigation

|  | Jan | Feb | Mar | Apr | May | Jun | Jul | Aug | Sep | Oct | Nov | Dec |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Precipitation deficit |  |  |  |  |  |  |  |  |  |  |  |  |
| 1. MAIZE (Grain) | 149.4 | 137.1 | 53.5 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 36.9 | 119 |
| 2. ONION | 93.5 | 101.4 | 55.2 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 47.2 | 74 |
| 3. BANANA 2nd year | 125.6 | 136.7 | 155 | 110.5 | 0 | 0 | 0 | 0 | 0 | 0 | 38.8 | 114.5 |
| 4. CABBAGE Crucifers | 103.8 | 126.6 | 139 | 80.6 | 0 | 0 | 0 | 0 | 0 | 0 | 38.6 | 80.7 |
| 5. Sweet Peppers | 130.8 | 87 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 4.5 | 53.9 | 109.4 |
| 6. Potato | 137.3 | 60.4 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 1.7 | 68.6 | 131.5 |
| 7. Sugarcane (Ratoon) | 105.3 | 148.7 | 162.1 | 114.9 | 0 | 0 | 0 | 0 | 0 | 8.1 | 36.9 | 53.3 |
| 8. Tomato | 143.2 | 125.2 | 10.2 | 0 | 0 | 0 | 0 | 0 | 0 | 2.2 | 62.3 | 124.1 |
| 9. Pasture perennial | 105.8 | 107.7 | 110.5 | 68.6 | 0 | 0 | 0 | 0 | 0 | 10.9 | 59.2 | 97.3 |
| Net scheme irr.req. |  |  |  |  |  |  |  |  |  |  |  |  |
| in mm/day | 4 | 3.7 | 1.5 | 0.6 | 0 | 0 | 0 | 0 | 0 | 0.1 | 1.7 | 3.3 |
| in mm/month | 123.2 | 103.3 | 47.8 | 19.1 | 0 | 0 | 0 | 0 | 0 | 2.5 | 50.2 | 101 |
| in l/s/h | 0.46 | 0.43 | 0.18 | 0.07 | 0 | 0 | 0 | 0 | 0 | 0.01 | 0.19 | 0.38 |
| Irrigated area (% of total area) | 100 | 100 | 57 | 19 | 0 | 0 | 0 | 0 | 0 | 58 | 100 | 100 |
| Irr.req. for actual area (l/s/h) | 0.46 | 0.43 | 0.31 | 0.39 | 0 | 0 | 0 | 0 | 0 | 0.02 | 0.19 | 0.38 |
| 45% Eff. |  |  |  |  |  |  |  |  |  |  |  |  |
| 24 HR | 1.02 | 0.96 | 0.69 | 0.87 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.04 | 0.42 | 0.84 |
| 10HR | 2.45 | 2.29 | 1.65 | 2.08 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.11 | 1.01 | 2.03 |

Table 26.Scheme Supply for supplementary irrigation

|  | Jan | Feb | Mar | Apr | May | Jun | Jul | Aug | Sep | Oct | Nov | Dec |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Precipitation deficit |  |  |  |  |  |  |  |  |  |  |  |  |
| 1. MAIZE (Grain) | 0 | 0 | 15.9 | 65.9 | 55.4 | 0 | 2 | 0 | 0 | 0 | 0 | 0 |
| 2. Sweet Peppers | 0 | 0 | 41 | 56 | 30 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 3. Potato | 0 | 0 | 36.2 | 68.4 | 48.1 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| Net scheme irr.req. |  |  |  |  |  |  |  |  |  |  |  |  |
| in mm/day | 0 | 0 | 0.9 | 2.1 | 1.5 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| in mm/month | 0 | 0 | 27.2 | 64.1 | 47.2 | 0 | 1 | 0 | 0 | 0 | 0 | 0 |
| in l/s/h | 0 | 0 | 0.1 | 0.25 | 0.18 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| Irrigated area(% of total area) | 0 | 0 | 100 | 100 | 100 | 0 | 50 | 0 | 0 | 0 | 0 | 0 |
| Irr.req. for actual area((l/s/h) | 0 | 0 | 0.1 | 0.25 | 0.18 | 0 | 0.01 | 0 | 0 | 0 | 0 | 0 |
| 45% EFF |  |  |  |  |  |  |  |  |  |  |  |  |
| 24HR | 0 | 0 | 0.2 | 0.6 | 0.4 | 0.0 | 0.0 | 0 | 0 | 0 | 0 | 0 |
| 10HR | 0 | 0 | 0.5 | 1.3 | 1.0 | 0.0 | 0.1 | 0 | 0 | 0 | 0 | 0 |

#### 3.9.6.9. Irrigation Scheduling

Once the crop water and irrigation requirements have been calculated, the next step is the preparation of field irrigation schedules. Three parameters have to be considered in preparing an irrigation schedule;

- The daily crop water requirements

- The soil, particularly its total available moisture or water-holding capacity

- The effective root zone depth

Plant response to irrigation is influenced by the Physical condition, fertility and biological status of the soil. Soil condition, texture, structure, depth, organic matter, bulk density, salinity, Sodicity, acidity, drainage, topography, fertility and chemical characteristics all affect the extent to which a plant root system penetrates into and uses available moisture and nutrients in the soil. Many of these factors influence the water movement in the soil, the water holding capacity of the soil, and the ability of the plants to use the water. The irrigation system used should match all or most of these conditions and accordingly the system of irrigation to be used is gravity irrigation using furrows.

#### 3.9.6.10. Irrigation Depth (D)

Depth of irrigation (d), including application losses, applied to the soil in one irrigation application and which is needed to bring the soil water content of root zone to field capacity in mm. The depth of irrigation application (d) including application losses is;

d = ( p x Sa) x D  (mm)

Ea

Where : p = fraction of available soil water

Sa = total available soil water mm/m soil depth

D = Rooting depth, m

Ea = application efficiency, fraction

Since P, D and ETc will vary over growing season, the depth in mm and interval of irrigation in days will vary.

##### 3.9.6.11.. Irrigation Interval (i)

Irrigation frequency is defined as the frequency of applying water to a particular crop at a certain stage of growth and is expressed in days. In equation form it reads:

Irrigation interval (i)  = Sa x P x RZD   (day)

ETc

Where: (i) = Irrigation interval (day)

Sa = Total available soil moisture = (FC – PWP) (mm/m)

P = Allowable depletion (decimal)

RZD = Effective root zone depth (m)

ETc = Crop Evapotranspiration or crop water requirement (CWR) (mm/day)

Total available soil water content plays significant role in determining the irrigation depth and irrigation intervals.

Basically, different texture  of soils vary in their water  holding capacity whereby coarse  textured soils have  less water holding capacity relative to fine textured soils .Eventhogh there are soil textural variations from place to place ,the dominant soil texture had been used for crop water requirement calculation  and the total available water content had been averaged for the site .

Table 27.irrigation depth and irrigation interval of the project

| Crop | Rooting depth(m) | Depletion(P) | Max.ETcr(mm/day) |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | I(Irrg. Interval)(days ) | Depth (mm) |
| Banana | 0.9 | 0.48 | 5.51 | 15 | 133 |
| Maize | 1 | 0.63 | 5.24 | 22 | 194 |
| Pepper | 0.8 | 0.33 | 4.34 | 11 | 81 |
| Tomato | 1 | 0.4 | 4.79 | 15 | 123 |
| Onion | 0.6 | 0.42 | 4.25 | 11 | 78 |
| Sugarcane | 1.5 | 0.65 | 5.72 | 32 | 301 |
| H.Cabbage | 0.5 | 0.45 | 4.96 | 8 | 69 |
| Pasture | 0.8 | 0.5 | 4.03 | 18 | 123 |
| Potato | 0.6 | 0.35 | 4.52 | 9 | 65 |

### 3.9.7. Proposed Crop Yield Projection

Agricultural production and projection are crucial output of agronomic study to explain the potential of the project to contribute to household food security and increased income.

#### 3.9.7.1. Basis for Yield Estimation

Estimates of production and yield of individual crops are based on the potential of the crop, implementation of improved cultural practices including input use and efficient irrigation management. Improvements of the farming system would be achieved through the introduction of improved and locally adapted varieties, improved land preparation and soil fertility maintenance techniques and the use of environmentally sound methods of crop protection

Table 28.Potential and Actual Average Yield of Proposed Crops for irrigation Project

| No | crop | Actual average yield(qt/ha) | potential yield (qt/ha) | Remark |
| --- | --- | --- | --- | --- |
| 1 | Banana | - | 480 |  |
| 2 | Maize | 35 | 90 |  |
| 3 | H /cabbage |  | 450 |  |
| 4 | Onion | - | 350 |  |
| 5 | Tomato | 130 | 400 |  |
| 6 | Pepper |  | 25 |  |
| 8 | Potato | 140 | 448 |  |
| 9 | Onion |  | 350 |  |
| 10 | Carrot | - | 250 |  |
| 11 | Sugar cane | - | 1450 |  |

The initial yield estimates of the proposed crops are very much closer to the current production performance in the area as well as in the country. Subsequent yields are assumed to increase steadily as improvements in cultural practices including irrigation management are adopted.

Accordingly, yield projection of the project shows the trends of crop productivity over the project period. Generally, some of the common assumptions are listed but not exhaustive:

- Current yield under existing cropping system in the project area /similar agro ecology to set first year yield

- Yield potential of suggested crop varieties at research out puts proven on farmers plot

- Yield obtained by model and  progressive farmers

- Regional and national averages

Table 29.yield Projection across Years (qt/ha)

| SN | Crop | Production Period |  |  |  |  |  |  | Remark |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Actual |  | 1st year | 2nd year | 3rd year | 4th year | 5th year |  |
| 1 | Banana |  | - |  | 120 | 160 | 280 | 480 |  |
| 2 | Maize | 35 | 50 |  | 70 | 80 | 85 | 90 |  |
| 3 | H /cabbage |  | 150 |  | 250 | 300 | 350 | 450 |  |
| 4 | Onion | 120 | 130 |  | 180 | 220 | 280 | 350 |  |
| 5 | Tomato | 130 | 140 |  | 200 | 280 | 350 | 400 |  |
| 6 | Pepper |  | 15 |  | 20 | 22 | 24 | 25 |  |
| 7 | Potato | 140 | 160 |  | 250 | 320 | 380 | 448 |  |
| 8 | Carrot |  | 120 |  | 140 | 160 | 190 | 250 |  |
| 9 | Sugarcane | - | - |  | 1150 | 1300 | 1430 | 1450 |  |

### 3.9.8. Proposed cropping pattern and intensity

The cropping patterns take into account the capability of the land and crop suitability to the soil types of the project area. In order to attain the suggested cropping intensities for the first two to three years, a lot of extension work is required to raise awareness among the beneficiaries to utilize the maximum possible area of the project. It is noted that the purpose of irrigation is ultimately to intensify agricultural production so long the resource, water, and land are adequately available.

In the meantime, the cropping pattern will have the following features:

- It increases the volume of production during full irrigation   and thereby improving food security of the area;

- The production of high value crops improve cash income of the households;

- Production  and availability of  crops  that serve as raw materials for agro processing industries

- It employs household labor throughout the year

- Increases availability of livestock feed (due to availability of crop residue and aftermaths and on farm forage /pasture production) all year round.

In proposed crop pattern and intensity, utmost attention was given to the following points

- Relatively more  land was allocated to food crops

- Cash crops such as vegetable crops have allocated large area

- Perennial fruit crops and pastures have relatively lower area

- Due to the permanent occupation of the perennial  fruit crops ,industrial crops(sugarcane) and pastures the total areas gradually decreases

- The perennial fruit crops and pastures are advised to plant also as the hedge rows

Generally it is observed from the table that the more the area allocated and the more productivity of crops is higher, the higher will be total production (qts) of crops from year to year.

Table 30.Cropping pattern and intensity

| Type of crops | Year 1 |  |  | Year 2 |  |  | Year 3 |  |  | Year 4 |  |  | Year 5+ |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Dry season |  |  | Dry season |  |  | Dry season |  |  | Dry season |  |  | Dry season |  |  |
|  | ha | Yield (qt/ha) | Total production (qt) | ha | Yield (qt/ha) | Total production (qt) | ha | Yield (qt/ha) | Total production (qt) | ha | Yield (qt/ha) | Total production (qt) | ha | Yield (qt/ha) | Total production (qt) |
| Maize | 20 | 50 | 1000 | 18 | 70 | 1260 | 20 | 80 | 1600 | 20 | 85 | 1700 | 18 | 90 | 1620 |
| Onion | 3 | 130 | 390 |  | 180 | 0 | 2 | 220 | 440 | 2 | 280 | 560 | 2 | 350 | 700 |
| Tomato | 2 | 140 | 280 | 2 | 200 | 400 | 1 | 280 | 280 | 1 | 350 | 350 | 1 | 400 | 400 |
| Pepper | 5 | 15 | 75 | 5 | 20 | 100 | 5 | 22 | 110 | 5 | 24 | 120 | 5 | 25 | 125 |
| Cabbage | 3 | 150 | 450 | 2 | 250 | 500 | 2 | 300 | 600 | 2 | 350 | 700 | 2 | 450 | 900 |
| Sugarcane | 0.25 |  | 0 |  | 1150 | 287.5 |  | 1300 | 325 |  | 1430 | 357.5 |  | 1450 | 362.5 |
| Potato | 15 | 160 | 2400 | 20 | 250 | 5000 | 18 | 320 | 5760 | 18 | 380 | 6840 | 20 | 448 | 8960 |
| Carrot | 1 | 120 | 120 | 2 | 140 | 280 | 1 | 160 | 160 | 1 | 190 | 190 | 1 | 250 | 250 |
| Banana | 1.5 |  | 0 |  | 120 | 180 |  | 160 | 240 |  | 280 | 420 |  | 480 | 720 |
| Pastures | 0.25 |  | 0 |  |  | 0 |  |  | 0 |  |  | 0 |  |  | 0 |
| Total | 51 |  | 0 | 49 |  | 0 | 49 |  | 0 | 49 |  | 0 | 49 |  | 0 |

Assuming that about 20 % of the irrigated land would be free as of march , the cropping pattern during supplementary irrigation  in season 2would be as indicated on the next table .

Table 31.Proposed cropping pattern and production projection  during supplementary irrigation

| Type of crops | Year 1 |  |  | Year 2 |  |  | Year 3 |  |  | Year 4 |  |  | Year 5+ |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | ha | Yield (qt/ha) | Total production (qt) | ha | Yield (qt/ha) | Total production (qt) | ha | Yield (qt/ha) | Total production (qt) | ha | Yield (qt/ha) | Total production (qt) | ha | Yield (qt/ha) | Total production (qt) |
| Maize | 4 | 50 | 200 | 5 | 70 | 350 | 4 | 80 | 320 | 5 | 85 | 425 | 4 | 90 | 360 |
| Pepper | 2 | 15 | 30 | 1 | 20 | 20 | 2 | 22 | 44 | 1 | 24 | 24 | 2 | 25 | 50 |
| Potato | 4 | 160 | 640 | 4 | 250 | 1000 | 4 | 320 | 1280 | 4 | 380 | 1520 | 4 | 448 | 1792 |

- The land will become covered by excessive drainage problem   thus  some times hinder cultivation during rainy season at some places

- The farmers will be giving attention for lands around the homesteads   for crops such as maize , wheat ,barley   etc instead of cultivation on the command area

- Less agricultural support services

- Fallow requirement for soil fertility and animal feed requirement.

Never the less, the cropping intensity during all season to be followed on the land is between   120 %   to 200% depending up on the  drainage conditions

### 3.9.9. Crop Rotation

Whether under rain fed or irrigation condition, crop rotation program is an integral part of the scheme. The following crop rotation scheme had been proposed with the assumption of

- Improvement of soil productivity by inclusion of a legume crop to promote nitrogen fixation as well as the improvement of the soil Physical characteristics by way of adding plant residues.

- Reducing   disease and insect pests out breaks due to crop rotation especially for soil born  host specific pests

- To save some crop products in times of crop failure due to environmental calamities

- Optimal use of available water

Table 32.Proposed crop rotation

| Year | 1 | 2 | 3 | 4 | 5 |
| --- | --- | --- | --- | --- | --- |
| 1 | Maize | Forage legumes (annuals) | Potato | Tomato | Onion |
| 2 | Onion | Pepper | Forages | Maize | Cabbage |
| 4 | Potato | Tomato | Cabbage | Pepper | Forages |
| 4 | Tomato | Onion | Pepper | Potato | Carrot |
| 5 | Carrot/swischard | Maize | Carrot | Carrot | Pepper |
| 6 | Pepper | Cabbage | Forage legumes (annuals | Onion | Maize |
| 7 | Forages | Potato | Tomato | Cabbage | Potato |

### 3.9.10. Irrigation Farm Inputs and Support Services

##### Recommended Inputs (Fertilizers, Seeds, Agrochemicals)

In the document under agronomic management practices of selected crops, each of the inputs required for the recommended crops are thoroughly indicated.

For instance  with the agrochemicals , type of chemical ,their requirements like for which  insect pest ,disease and weeds the chemicals are recommended ,indicated  alternatives ,rate of application  , time of application etc are mentioned .However , the  main inputs for major crops are summarized on the following table .

Table 33.Required inputs and rates

| Crop | Fertilizer |  | Compost/FYM(Tone/ha) | Seed/seedlings rate per hectare | Pesticide, herbicide and fungicide |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | (Kg/ha ) |  |  |  |  |  |
|  | NPS | UREA |  |  | Type | Rate/ha |
| Banana |  |  |  | 1000-3000 |  |  |
| Pepper | 100 | 100 |  | 2.5-3kg (direct sowing),0.6-0.75kg(raise on nursery | Cypermetrin | 0.5lit |
| Onion | 100 | 100 | 20-25 tones | 3.5-4kg | Cypermetrin 10% | 500ml |
|  |  |  |  |  | Redomil M.Z 63.5 WP | 3kg |
|  |  |  |  |  | Mancozeb 80% wp | 2.5kg |
| Tomato | 150 | 100 |  | 0.3-0.38kg | Redomil | 3.5kg |
|  |  |  |  |  | Cypermetrin | 75gm |
| Maize | 100 | 200 |  | 25-30kg | Round up | 1lt |
|  |  |  |  |  | Glyphosate | 1lt |
|  |  |  |  |  | 2,4-D | 1lt |
|  |  |  |  |  | Alazine | 2.2-2.75 kg |
|  |  |  |  |  | Primagrim gold 660SC | 1.32-1.98kg |
|  |  |  |  |  | Sadik | 1.2-1.6kg |
|  |  |  |  |  | Endosulfan | 1kg |
|  |  |  |  |  | Carbaryl | 2.3kg |
|  |  |  |  |  | Diazinon | 10kg |
|  |  |  |  |  | Cypermetrin | 3-3.8kg |
| Potato | 195 | 165 |  | 20-25qt/ha | M –Ridomil M | 2kg/ha |
| Carrot |  |  | 20-25 tons | 5-6kg/ha | Dimetoate | 1kg/ha |
| Head cabbage |  |  |  | 500-750 gm | Butachlor | 1.5kg |

Proposed Labor cost

Under smallholder farming system human labor is a major and determinant input for agriculture. Out of total labor requirements, family labor taking major shares while extra labor covers the remaining labor demand. The human labour is calculated in monitory basis. Therefore according to the  table , the number of man days required per hectare for each activities per its frequency for  each crops are multiplied by the forecasted local unit rate of payment to come up with total payment /cost of labour for each crop per hectare

Table 34.Proposed Labour cost

| Crop type | Activities | Freq | Man days (Man//ha) | Unit rate (birr) | Sub total cost (cost ) | Total cost per crop |
| --- | --- | --- | --- | --- | --- | --- |
| Maize | Land clearing | 1 | 8 | 40 | 320 | 5280 |
|  | Plowing | 1 | 8 | 40 | 320 |  |
|  | Cultivation and leveling /harrowing and furrowing | 1 | 12 | 40 | 480 |  |
|  | Sowing | 1 | 8 | 40 | 320 |  |
|  | Fertilizer application | 3 | 2 | 40 | 240 |  |
|  | Spraying | 3 | 4 | 40 | 480 |  |
|  | Weeding | 3 | 16 | 40 | 1920 |  |
|  | Irrigation | 7 | 2 | 40 | 560 |  |
|  | Harvesting | 1 | 8 | 40 | 320 |  |
|  | Bagging and handling | 1 | 4 | 40 | 160 |  |
|  | Loading and unloading | 1 | 4 | 40 | 160 |  |
| Onion | Land clearing | 1 | 4 | 40 | 160 | 5040 |
|  | Nursery management |  | 4 | 40 | 0 |  |
|  | Plowing | 1 | 8 | 40 | 320 |  |
|  | Cultivation and levelling and furrowing | 1 | 10 | 40 | 400 |  |
|  | Sowing /planting | 1 | 8 | 40 | 320 |  |
|  | Fertilizer application | 2 | 4 | 40 | 320 |  |
|  | Spraying | 2 | 4 | 40 | 320 |  |
|  | Weeding | 2 | 8 | 40 | 640 |  |
|  | Irrigation | 10 | 4 | 40 | 1600 |  |
|  | Harvesting | 1 | 8 | 40 | 320 |  |
|  | Bagging and handling | 1 | 8 | 40 | 320 |  |
|  | Loading and unloading | 1 | 8 | 40 | 320 |  |
| Pepper | Land clearing | 1 | 4 | 40 | 160 | 5280 |
|  | Cultivation and leveling and furrowing | 1 | 10 | 40 | 400 |  |
|  | Sowing | 1 | 8 | 40 | 320 |  |
|  | Fertilizer application | 2 | 4 | 40 | 320 |  |
|  | Spraying | 2 | 4 | 40 | 320 |  |
|  | Weeding | 2 | 8 | 40 | 640 |  |
|  | Irrigation | 11 | 4 | 40 | 1760 |  |
|  | Harvesting | 1 | 16 | 40 | 640 |  |
|  | Bagging and handling | 1 | 8 | 40 | 320 |  |
|  | Loading and unloading | 1 | 10 | 40 | 400 |  |
| Tomato | Land clearing | 1 | 4 | 40 | 160 | 6320 |
|  | Nursery |  | 4 | 40 | 0 |  |
|  | Cultivation and leveling furrowing | 1 | 10 | 40 | 400 |  |
|  | Sowing | 1 | 8 | 40 | 320 |  |
|  | Fertilizer application | 2 | 4 | 40 | 320 |  |
|  | Spraying | 2 | 4 | 40 | 320 |  |
|  | Weeding | 3 | 8 | 40 | 960 |  |
|  | Irrigation | 10 | 4 | 40 | 1600 |  |
|  | Harvesting | 2 | 12 | 40 | 960 |  |
|  | Bagging and handling | 2 | 10 | 40 | 800 |  |
|  | Loading and unloading | 2 | 6 | 40 | 480 |  |
| H.Cabbage | Land clearing | 1 | 4 | 40 | 160 | 6400 |
|  | Nursery management | 1 | 4 | 40 | 160 |  |
|  | Cultivation and leveling | 1 | 8 | 40 | 320 |  |
|  | Sowing /planting | 1 | 8 | 40 | 320 |  |
|  | Fertilizer application | 2 | 4 | 40 | 320 |  |
|  | Spraying | 2 | 4 | 40 | 320 |  |
|  | Weeding | 2 | 8 | 40 | 640 |  |
|  | Irrigation | 20 | 4 | 40 | 3200 |  |
|  | Harvesting | 1 | 8 | 40 | 320 |  |
|  | Bagging and handling | 1 | 8 | 40 | 320 |  |
|  | Loading and unloading | 1 | 8 | 40 | 320 |  |
| Banana | Land clearing | 1 | 4 | 40 | 160 | 3520 |
|  | sucker management and Plowing | 1 | 8 | 40 | 320 |  |
|  | Cultivation and leveling and planting | 1 | 8 | 40 | 320 |  |
|  | Fertilizer application | 1 | 4 | 40 | 160 |  |
|  | Spraying | 1 | 4 | 40 | 160 |  |
|  | Weeding | 2 | 4 | 40 | 320 |  |
|  | Irrigation | 16 | 2 | 40 | 1280 |  |
|  | Harvesting | 1 | 12 | 40 | 480 |  |
|  | Bagging and handling | 1 | 4 | 40 | 160 |  |
|  | Loading and unloading | 1 | 4 | 40 | 160 |  |
| Pasture/Forages | Land clearing | 1 | 4 | 40 | 160 | 5280 |
|  | Plowing and leveling ,sowing | 2 | 10 | 40 | 800 |  |
|  | Weeding | 2 | 4 | 40 | 320 |  |
|  | Irrigation | 20 | 4 | 40 | 3200 |  |
|  | Harvesting | 1 | 8 | 40 | 320 |  |
|  | Bagging and handling | 1 | 6 | 40 | 240 |  |
|  | Loading & unloading | 1 | 6 | 40 | 240 |  |
|  |  |  |  | 40 | 0 |  |
| Sugarcane | Land clearing | 1 | 4 | 40 | 160 | 5440 |
|  | Cultivation and leveling | 2 | 8 | 40 | 640 |  |
|  | Furrowing | 1 | 4 | 40 | 160 |  |
|  | Sowing /planting | 1 | 8 | 40 | 320 |  |
|  | Fertilizer application | 2 | 4 | 40 | 320 |  |
|  | Spraying | 1 | 4 | 40 | 160 |  |
|  | Weeding | 3 | 8 | 40 | 960 |  |
|  | Irrigation | 11 | 4 | 40 | 1760 |  |
|  | Harvesting | 1 | 8 | 40 | 320 |  |
|  | Bagging and handling | 1 | 8 | 40 | 320 |  |
|  | Loading and unloading | 1 | 8 | 40 | 320 |  |
| Potato | Land clearing | 1 | 4 | 40 | 160 | 5280 |
|  | Cultivation and leveling | 2 | 8 | 40 | 640 |  |
|  | Furrowing | 1 | 4 | 40 | 160 |  |
|  | Sowing /planting | 1 | 8 | 40 | 320 |  |
|  | Fertilizer application | 1 | 4 | 40 | 160 |  |
|  | Spraying | 1 | 4 | 40 | 160 |  |
|  | Weeding | 3 | 8 | 40 | 960 |  |
|  | Irrigation | 14 | 4 | 40 | 1760 |  |
|  | Harvesting | 1 | 8 | 40 | 320 |  |
|  | Bagging and handling | 1 | 8 | 40 | 320 |  |
|  | Loading and unloading | 1 | 8 | 40 | 320 |  |

##### Proposed Oxen Days

The traditional plough (Maresha) using oxen power and small farm tools by human power are the conventional tillage systems  respectively used for land preparation .Thus, for calculating oxen days, average number of frequencies for the activities are multiplied by number of oxen required for a hectare and local unit oxen day  cost to come up with total  oxen days  cost per hectare .

Table 35.proposed oxen day cost

| Crops | Activities | Unit | Freq | OD(Qty) | Unit cost | Sub Total cost | Total cost per crop |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Onion | Plowing /cultivation and leveling | OD | 2 | 8 | 80 | 1280 | 1920 |
|  | Sowing | « | 1 | 8 | 80 | 640 |  |
| Maize | Plowing /cultivation and leveling /harrowing | « | 2 | 8 | 80 | 1280 | 1920 |
|  | Sowing | « | 1 | 8 | 80 | 640 |  |
| Head cabbage | Plowing /cultivation and leveling /harrowing | « | 1 | 8 | 80 | 640 | 1280 |
|  | Sowing | « | 1 | 8 | 80 | 640 |  |
| Pepper | Plowing /cultivation and leveling /harrowing | « | 1 | 8 | 80 | 640 | 1280 |
|  | Sowing | « | 1 | 8 | 80 | 640 |  |
| Tomato | Plowing /cultivation and leveling /harrowing | « | 1 | 8 | 80 | 640 | 1280 |
|  | Sowing | « | 1 | 8 | 80 | 640 |  |
| Banana | Plowing /cultivation and leveling /harrowing | « | 1 | 8 | 80 | 640 | 1280 |
|  | Sowing/Planting | « | 1 | 8 | 80 | 640 |  |
| Sugarcane | Plowing /cultivation and leveling /harrowing | « | 3 | 8 | 80 | 1920 | 2560 |
|  | Sowing | « | 1 | 8 | 80 | 640 |  |
| Pasture /forages /perennials | Plowing /cultivation and leveling /harrowing | « | 1 | 8 | 80 | 640 | 640 |
| Potato | Plowing /cultivation and leveling /harrowing | « | 2 | 8 | 80 | 1280 | 1920 |
|  | Sowing | « | 1 | 8 | 80 | 640 |  |

Table 36Material cost

| Type | unit | Amount required per hectare | Unit cost in birr | Total cost |
| --- | --- | --- | --- | --- |
| Plough and Accessories | no | 1 | 1000 | 1000 |
| Shovel | no | 5 | 100 | 500 |
| Spade | no | 2 | 100 | 200 |
| water can | no | 2 | 300 | 600 |
| string | pieces | 1 | 50 | 50 |
| Hoes | no | 5 | 50 | 250 |
| Meter | no | 1 | 60 | 60 |
| Others |  |  |  | 500 |

Table 37.Cost summary

| Crops | Inputs | Seed rate (kg/ha ) | Urea (qt/ha ) | blended (qt//ha ) | Pesticide/Fung..herbicides | Labour (Md/ha) | Pair of oxen(OD/ha) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Pepper | (Kg) | 2.5-3(direct sowing) but by raising on nursery ( 0.6-0.75kg) | 1qt | 1qt/ha | Bulk | 76 | 16 |
|  | Unit price(birr)/kg | 450 | 965birr/qt | 1205birr/qt | 350 | 40 | 80 |
|  | Total(birr)/kg | 1300birr/ha | 965birr/ha | 1205birr/ha | 700 | 3040 | 1280 |
| Onion | (Kg) | 3.5-4kg | 1qt/ha | 1qt/ha | Bulk | 78 | 16 |
|  | Unit price | 500birr/kg | 965birr/qt | 1205birr/qt | 250 | 40 | 80 |
|  | Total(birr)/ha | 2000 | 965birr/ha | 1205birr/ha | 500 | 3120 | 1280 |
| Tomato | Kg | 0.3-0.38 | 1qt/ha | 150 | Bulk | 74 | 16 |
|  | Unit price | 800birr/kg | 965birr/qt | 1205birr/ha | 400 | 40 | 80 |
|  | Total(birr) | 304 birr/ha | 965birr/ha | 1807birr/ha | 800 | 2960 | 1280 |
| Maize | Kg | 25-30 | 2qt/ha | 1qt/ha | Bulk | 76 | 16 |
|  | Unit price | 3500birr/qt | 965bir/qt | 1205birr/qt | 400 | 40 | 80 |
|  | Total(birr)/ha | 1050birr/ha | 1930birr/ha | 1205birr/ha | 800 | 3040 | 1280 |
| H/cabbage | Kg | 0.5-0.75 |  |  | Bulk | 68 | 16 |
|  | Unit price | 500bir/kg |  |  | 200 | 40 | 80 |
|  | Total(birr)/ha | 375birr/ha |  |  | 400 | 2720 | 1280 |
| Banana | sucker | 1000-3000 |  |  |  | 52 | 16 |
|  | Unit price | 8birr/seedling |  |  |  | 40 | 80 |
|  | Total(birr) | 24000 |  |  |  | 2080 | 1280 |
| potato | seed | 25 qt/ha | 1.65qt/ha |  | Bulk | 64 | 16 |
|  | Unit price/ ./qt | 1200birr/qt | 965birr/qt |  | 350 | 40 | 80 |
|  | Total(birr) | 30000birr/ha | 1592.25birr/ha |  | 700 | 2560 | 1280 |

### 3.9.11. Support Services

All the agricultural support institutions like agricultural offices  (  agronomy and crop protection ,input supply  etc ), irrigation   authority ,cooperative office  , research centers ,seed multiplication enterprises ,crop marketing etc and other governmental and nongovernmental organizations  at all levels  should collaborate to work together and  be fully committed towards strengthening their  human resource competence and technical and financial capacity .The developed capacities in all aspects help to discharge required supports and inputs to farmers in order to make access  to agricultural supports such as vigorous extension services and agricultural inputs as sufficient as possible in expected time period with  the better-quality .

The full application of these inputs may not be attainable during the early years of the project life but should be implemented in a gradual manner through vigorous extension work, rural credit, farmers’ active participation, on- farm demonstrations and trials.

Due concern  is also predictable for improvement of agricultural product prices through the involvement of stakeholders in designing and developing local and international market channels so that the farming community could benefit from .

Marketing itself in particular has to be developed to a strong institution and require   well-organized marketing infrastructures such as all-weather roads, efficient transport system, market centers, storage facilities etc. The aforementioned support services would be fruitful when agricultural extension particularly irrigation extension and training and researches are intrepidly implemented at the grass root level.

##### Selection of Farmers: Peasant farmers who will join the proposed irrigation scheme will be identified and selected right the beginning period of the project. Particular attention could be given to adequately include female farmers’ participation in the project scheme

In view of the critical need to get equitable distribution of irrigation land to the existing farmers in the area, critical attention will be given to ensure every selected farmer is getting technically manageable and feasible holding of the irrigable land.

Strengthening the Agricultural Extension Services: Upon the completion of the final study , design  and construction work, sufficient number of Development agents particularly having skills for irrigation agronomy will be working in order to accomplish the various office and field activities of the project. The supervisors will co-ordinate the works and duties of the DAs; it is advisable to support the extension service with adequate logistics.

Training the Development Agents and Subject Matter Specialists: Development agents, supervisors and subject matter specialists from the project area will be trained. The training programs will give emphasis to improved irrigation practices, all agronomic practices from land preparation to harvesting and processing, input use, integrated land management, integrated pest control, and other related topics to the envisaged project. Moreover, study tours within the country where successful  irrigation schemes are found may be organized in order to upgrade the extension skills of these development staff.

The major functions of extension staff are dissemination of information, advice, training and evaluation and monitoring of its effect. To do this effectively, they will have to know

- The technical knowledge and skills that have to be transferred to farmers.

- An understanding of the working procedure  and the organizational skills necessary to perform the duties effectively and correctly

##### Organize and Strengthen Rural Institutions: government offices like  agricultural and rural development , irrigation development authority , cooperatives promotion offices etc, trade and marketing offices and  other governmental and nongovernmental organizations are expected to be strengthened.

Thereafter, farmers who will join the project may be organized in various rural institutions. To this end, water user association, marketing  cooperatives and possibly producers and marketing cooperatives ,irrigation service cooperatives ,credit and saving cooperatives ,  etc  could be established and strengthened These farmers  institutions serve as basic structures to help the peasants to perform irrigation activities ,production, marketing etc in a more efficient and integrated ways in addition to alleviating various social, economical and commercial constraints. Certainly, these farmers’ institution must be established and operational in full willingness of all members.

Water user Association

An Irrigation users association may be established as a self-governing, nonprofit legal entity that shall, in the public interest, manage a canal network, wholly or in part, in order to provide water to its members for agricultural purposes. Even though this part of the study is not the  main task of an agronomist ,the assessment shows less organized water user associations in the study area eventhogh there are some good starts observed .Thus the water user associations should be organized further as per the following objectives .

Objective of WUA

The WUA being the management structure at site (scheme) level and being the owner of the irrigation scheme, it has various objectives, responsibilities and functions in the development process of the schemes. Among the various objectives, the main ones are:  Coordinate the participation and involvement of the beneficiary communities for equitable irrigation water distribution among the farmers on outlet command basis

- Process and carryout resource mobilization (irrigation O & M fee, labour contribution, material ...).

- Resolve disputes and conflicts among the beneficiaries that may arise due to improper water utilization.

- Provide support and assistance in the form of labour, cash and construction materials during scheme construction.

- Prepare operations and maintenance plans and ensure their implementation.

- Facilitate irrigation extension, micro-watershed, drainage and pollution

- Control work in the service area or canal network.

- Enlist members and update the list of water user farmers.

- Maintain records, Propose changes in scheme during planning and construction.

- Prepare and execute irrigation management plans etc.

The Water User Association (WUA) management committee shall consist at least: 1) Chairperson 2) Vice chairperson 3) Secretary 4) Treasurer /Cashier 5) Accountant, 6) Member/Purchaser/ 7) Member/ Store keeper /Scheme Operator/

In addition to the main management committee, the general assembly will also elect the control committee, Irrigation Water Scheduling and Distributing Committee, Input Supply and Marketing Committees. Moreover, the farmers in tertiary unit (TU) constitute water users teams (WUT), who organize the water rotation in their tertiary unit (TU) and these TU-Team Leader also elected by the Management Committees through discussion of the project beneficiaries.

#### 3.9.11. 1.Agricultural Research

It is evident that agricultural researches have immense potential in promoting farmers sector crop production by developing appropriate crop production technologies and packages. Unless the farmers’ efforts are fully supported to utilizing variable research finding and technologies, the success and sustainability of the envisaged development efforts will certainly be far from reality.

Currently, almost there is rare agricultural   researches intervention in the project area. To this effect, there is no readymade site specific research out-puts on various components of crop production especially for irrigated crops. Accordingly, the project understands the need and importance of trial sites in the study area in the future. Within this trial site both rain fed and irrigated trials maybe carried out.

At the time, the most important problems / gaps in the project should be identified. A pilot scheme research should be conducted to use the best option. First, it is necessary to dig out accumulated research findings (both local and international) on the proposed options and look forward to make use of it and to ensure the benefit and sustainability of the option.

Finally, each alternative agricultural development will have to be analyzed and recommendations should be made on the most suitable development option. It is clear that the potential and experience of the staff both at district and site level is far from being adequate to undertake such applied agricultural research works. Hence, the intended tasks need to be accomplished with joint efforts of the institute of agricultural research, zonal and the regional Agricultural and natural resource Bureau.

Therefore, by improving their technical problems, regional and national research  institutes, agricultural and natural resource Bureau  and higher learning institutions should focus on the following   most researchable areas in the intended irrigation command /catchment area .

A. Variety Development and Adaptation

Development of adaptive, superiors, high yielding crop varieties both under rain fed and irrigated conditions

B. Fertilizer Trial

Establishing economic optimum rates (EOR) of fertilizers

C.Pesticides Trial

Develop most appropriate and safe chemicals and rate of application for crops affected by different pests.

D. Organic Fertilizers Trials

Develop appropriate rate of organic fertilizers

E. Irrigation Trials

Establish actual and site specific crop water requirement, irrigation period and interval ,  method of irrigation etc.

F. Tillage and planting operations

Establish the most effective time, method, frequency of seed bed preparation and sowing /planting times and methods.

G. Seed Rates Trials

Establish optimum seed rates and plant population for various crops and varieties.

H.Post Harvest Crop Loss Assessment Studies.

- Identity the target crops liable to post harvest losses

- Identity the stage of losses and kind of losses

- Identify the causes of post harvest crop losses

-recommend remedial measures

I. Weeding Trials

-Asses and identify most damaging weed species

-identify efficient weed control measures

### 3.9.12. Agronomic Recommendations of Selected Crops

#### Banana  (Mussa spp.)

Importance :It has been demonstrated that banana provides a more balanced diet than most  other fruits and produces more calories per unit area than any other food crop .It is a staple food of millions of people all over the world and a leading fruit in international trade .

All parts of banana plant are useful .In addition to consuming as a fresh fruit, unripe banana fruit as well as the inner core of the pseudo stem is regularly cooked as a curry. Various products like chips, banana figs flour and powder, jam, confectionery, dehydrated core slice and baby foods are prepared from banana

General Growth Requirement

Altitude: It can be grown from sea level to an altitude of 1200meters.

Temperature: It grows well in temperature range of 10-40 0c with an average of 250c-280c .A mean temperature of 270c

Rainfall: 1500-2500mm /year of rain is considered satisfactory for rain fed plantation.

Soil: Banana needs a well drained soil with high organic matter .Even though it requires constant irrigation, it cannot withstand water stagnation .The soil should therefore be well drained up to at least 1 meter depth. It grows well in loamy soil. The production in heavy soils however, is not as good as that in light soil .The PH ranges from 4.5-8.2 but 5.6-7.5 is optimum.

Site Selection and Land Preparation: The site selected for banana plantation should be free

from any crop production for at least the last one year. It has to be loam with gentle slope. After

Site selection, any plant debris, stones, plant roots and weeds should be removed. The land should be plowed to a depth of 20-30cm. After two weeks, the soil should be pulverized. For irrigation, water ways/canals are prepared along the rows of banana seedlings.

Planting space and time of planting: Depends upon the variety, soil type and management practices. For dwarf varieties, 2.5x2.5 meters, for medium varieties, 3x3meters, for tall varieties, 3.5x3.5 meter and for extreme tall varieties 4x4meters spacing is recommended .In general banana seedlings are planted from 2.5m to 3m. If banana is to be grown by rain fed cultivation, planting should be at the onset of rainy season. However with irrigation it can be planted any time if irrigation water is sufficient but in the project area in particular, depends on proposed cropping calendar.

Preparing Planting hole: The planting hole should be 60cm deep and 60 cm wide. Top and sub soil should be placed separately. The top soil should be mixed with manure or compost before replacing to the hole. The planting suckers are planted at 30cm depth.

Management of Planted Seedlings

- Mulching under planted seedlings with clean mulches

- Applying compost and farmyard manure

- Thinning and removing excessive suckers just leaving only 3-4 plants at one place at a time.

- Staking/supporting especially for fruit bearing plants

Weeds and Diseases Control:

Weeds control: Hand weeding, mulching to suppress weeds, inter cultivation can control weeds and when necessary using recommended herbicides. At least two times weeding is recommended.

Table 38.improved Banana Varieties

| Type of Variety | Yield (qtl/ha) in research center | Remark |
| --- | --- | --- |
| Dwarf Cavendish | 531 | Dwarf and sweet but susceptible to nematode |
| Poyo | 482 | Tall variety and sweet |
| Wiliam 1 | 556 | Medium height |
| Grande Nane | 436 | Dwarf |
| Robusta | 395 | Tall |
| Nigeru | 482 | For bread and porriage |

Diseases: The following diseases are expected to occur

1. Cigar –end rot

- Fungal pathogen

- Attacks banana fruits

Control

- Cultural practices

- Applying chemical (bavistine) when flowering starts

2. Bacterial wilt

- Bacterial pathogen

Control method

- Removing affected plant

- Avoid using affected plants for replanting

- Use clean farm tools

- Disinfecting farm tools with hot water

Fertilizer application: on the small holder farming, fertilizer application is not economical .However, in conditions of modern and commercial farming, the

Following recommendation can be used in areas where there is no soil test result.

Table 39.Rate of Fertilizer Application

| Variety | Number to be supplied (in kg/ha) |  |  |  |
| --- | --- | --- | --- | --- |
|  | N | P2O5 | K2O | Remark |
| Dwarf Cavendish | 540 | 325 | 675 |  |
| Robusta | 405 | 245 | 507 |  |
| Other varieties | 400 | 240 | 500 |  |

Farm yard manure may be applied at the rate of 40 tons per hectare .I.e. about 20kg per pit. Generally, the following pattern of split application is followed.

1st dose 45 to 50 days after planting

2nd 78 to 80 days after planting

3rd dose 115 to 120 days after planting

Irrigation: indicated in the crop water requirement and

Flowering and fruit setting stages is the critical stage

Harvesting and Post Harvest Handling: Banana fruits should be harvested at the correct Physiological maturity when the fruits turn deep green and 2/3 of the fruit reach for harvest. However, for home consumption and nearby markets, fruits can be harvested when ripe and turn yellow.

Regarding banana fruits handling, care should be given to the time and method of harvesting, packaging and transporting. Banana fruit can be transported using human labor, animal power and vehicles. After harvesting banana fruits, it should be placed in an air proof room (high temperature and ethylene) to facilitate ripening and turning color to yellow.  To store fruits, it should be placed in cool, dry and aerated condition.

Red Pepper (Capsicum spp)

Importance :red pepper has  high content of  capsaicin , sweet or bell pepper with very low capsaicin ,caprika or intermediate type with little capsaicin  and the birds eye chilies ,which  has the pungent among chilies and has a smallest fruit size .

Red pepper is an important vegetable crop for fresh consumption, processing as a spice (for making stew) and has all great importance to extract oleoresin in the spice extraction factories.

Recommended varieties: Melka zala , Melkashet ,Mareko Fana , Bako local .

General Growth Requirements

Altitude: 1000-1800masl

Rainfall: high yields can be obtained with a rainfall of 600-1250 mm well distributed among growth periods

Temperature: optimum growing temperature is between 21 -24 0c

Soils: wide range of soils but light textured soils with adequate water holding capacity .But if not light soil; the soil should have good drainage condition to avoid water logging.

Cultural characteristics

Seed Bed preparation

As research findings indicate , direct seeding in the field instead of raising seedlings on seed bed have shown increased yield .When sown  directly to the field  , the land should be finely prepared when directly sown using   irrigation, watering the field before sowing is advantageous.

2.5-3kg seed is required for a hectare during direct field sowing. After six weeks, the seedlings are thinned to 30 cm between plants.

However, for raising seedlings nursery, 600-750gram seed is required for one hectare .1*10m or 1*5m seed bed is prepared. Seeds are drilled in rows 15cm apart. For 1*5m seed bed 25gram seed is required and for one hectare about 24 seed beds are sufficient

Necessary management practices such as proper watering, crop protection, application of fertilizers and etc should be carried out. The seedlings are ready for transplanting to field at 45-60days at the height of 20-25cm or when seedlings attain 3-4 true leaves.

Land Preparation and Transplanting Seedlings

The land should be prepared and harrowed plowing 2-3 times. Healthy and vigorous seedlings are transplanted to the field. Transplanting should be done early in the morning or late in the afternoon. Care should be given to the roots and drying of seedlings must be avoided. Under irrigation, the space of planting should be 70cm between rows and 30cm between plants on the rows .However, in rain fed production 60cm between rows and 40cm between plants is recommended.

Fertilizer Application ;-depends on local soil condition , however 100kg Urea and 100kg NPS  is recommended for one hectare  . 50% of Urea is applied at planting time and the rest of Urea is applied at flower setting.

Crop Protection:

Weeds control: Red pepper is initially slow growing crops, incapable of offering any competition  to the aggressive weeds. Therefore, the weeds should be controlled as soon as they are seen by using hand and hoeing .In this respect red pepper plant should be weeded at least 3 times during growing period; the first and the second hand weeding can be done on average of 20-40 days, and the last at flowering stage respectively.

Insect control: several insects can attack red pepper plants. However, boll worm is the most dangerous insect that attacks the leaves and fruits. Adding to this, aphids are also some times attacking the plant leaves causing considerable yield loses.

Generally, the control methods are

- For Aphids and thrips ,use 0.5 liter cypermetrin 10% with 600 lit of water mixed for 1 hectare

- For bollworm use Indoxacarb

Diseases control

- Powdery  mildew ( loveillula taurica) , bacterial leaf spot or bacterial blight (pseudomonas vesicatoris)

Control

- Apply crop rotation

- Use clean variety seeds

Water Supply and Yield

For high yields, an adequate water supply and relatively moist soils are required during the total growing period. Reduction in water supply during the growing period in general has an adverse effect on yield and the greatest reduction in yield occurs when there is a continuous water shortage until the time of first picking. The period at the beginning of the flowering period is most sensitive to water shortage and soil water depletion in the root zone during this period should not exceed 25 percent. Water shortage just prior and during early flowering reduces the number of fruits. The effect of water deficit on yield during this period is greater under conditions of high temperature and low humidity. Controlled irrigation is essential for high yields because the crop is sensitive to both over and under application.

Water deficits during yield formation period lead to shriveled and malformed fruits. The pungent quality of the fruit (hotness) can to a certain degree be influenced by water supply.

Harvesting

Red pepper is harvested by hand picking for either green fruits (karia ) or red ripe fruits ( beriberi )

Post Harvest Managements

After harvesting, drying of red ripe fruits for about 5-7 days in bright sun light is necessary before storage. However, over drying affects quality to become brittle and easily broken. In addition, the dried fruits should be placed on mats or trays at table height, not on ground.

#### Onion   (Allium spp))

Importance .All parts of the plant contains the pungent principle that makes onions desirable as seasoning herbs .The pungent principle is due to volatile sulfur compounds .The preference of onion in Ethiopian diet is for a very pungent onion with red skin, high solids and good storage quality. In Ethiopia, onion production is dominantly carried out by small holders and is being considered as cash making commodity .In addition, it is also being produced partially for home consumption

General Growth Requirement

Altitude:-the optimum altitude range for onion production is between 700 and 2200 meters above sea level

Rainfall:-onion requires 350 -600mm rainfall well distributed during the growing period.

Temperature:-the optimum growing temperature lies between 150c and 23 0c

Soil: - Onion grows on wide ranges of soils but for optimum production, it requires well drained sandy loam with high content of organic matter. It is tolerant to wide ranges of soil PH of 6.0 to 7 .0

Varieties: Adama red, Bombay red, Melkam.

Cultural Practices

Land preparation:-the planting field of onion should be four times plowed with sufficient intervals to have fine, friable soil structures to grow seeds and transplanting seedlings.

Planting

Common onion can be grown by either of raising seedlings in a nursery and transplanting to field or planting bulbs directly in the field, broadcast or drilling seeds directly in the field and planting sets for production of onion in wet season.

Transplanting

Most commonly practiced for irrigated crop. It gives high yield of large sized bulbs. Seedlings are first raised in nursery.

Raising on Nursery

A nursery bed of 3m x 0.6m size should be prepared with a height of 15-25 cm with a space of 70cm between the beds to facilitate inter-cultural operations. Generally sandy loam soils are preferred for nursery beds. The top surface of 2-3cm should be enriched with fine, sieved and decomposed farm yard manure or compost after sowing. Seeds are sown in rows of 4-6 cm. Sowing should not be done more than 2-3cm deep. After sowing, beds should be mulched with dry grass or straw or any other such material to maintain and preserve the required soil moisture. As and when there is lack of moisture in the bed, it should be watered by sprinkling can. After germination, dry grass or mulching material should be removed. The seedlings are ready for transplanting 7-8 weeks after sowing or after attaining a height of 20-25cm. Over-aged seedlings result in bolting, taking longer time to start growth and under-aged seedling do not establish well after transplanting.

Onion seedlings are usually transplanted in flat bed or flat top ridges with furrows between ridges and making double rows on the flat ridges 20cm apart. The spacing between double rows is 40cm and between plants is 10cm (40cm x20cm x10cm)

Seed Rate:-About 3.5-4 kg seed having 90- 95% germination percentage is required to raise seedlings for planting one hectare.

Direct Sowing

Onion can also be sown by direct seeding. The soil is thoroughly pulverized and made free of clods. Varieties capable of having big sized bulbs are grown in rows 30cm apart. Seedlings 6-8 weeks old may be thinned. A light irrigation is given immediately after sowing.  Interculture and irrigation is repeated based on the local condition

Planting by Sets

Sets are small-sized onion produced by seedlings to mature in the nursery beds as such instead of transplanting to get an early crop to get more prices. For planting one hectare 5-8 kg seed is adequate to raise enough number of sets in 200m2 area. Nursery is prepared as mentioned above. The seedlings are allowed to remain in the nursery till their leaves fall and form small bulbs (sets) due to less spacing. Later, dig out 1.5-2.0 cm sized healthy sets for planting. The sets are planted 10cm apart in rows on both sides of ridges spaced at 35-45 cm spacing. A light irrigation is given immediately after planting the sets. In multiplier onion, bulbs are first separated and then bigger-sized bulbs are used for planting.

Manuring and Fertilizer Application

Onion needs a heavy dose of fertilizer for good yield. But the fertilizer requirements depend on soil type and type of crop. Farmyard or dung manure at the rate of 20-25 tons/ha may be incorporated in soil one month before transplanting.

Fertilizer Rate: depending on research finding in similar agro ecology , 100kg/ha of urea and 100kg/ha NPS   is recommended .Half dose of urea at planting and the remaining half dose  one and fifteen days after transplanting seedlings .

Water Supply for the Crop

Onion, in common with most vegetable crops, is sensitive to water deficit. For high yield, soil water depletion should not exceed 25 percent of available soil water. When the soil is kept relatively wet, root growth is reduced and this favors bulb enlargement. Irrigation should be discontinued as the crop approaches maturity to allow the tops to desiccate, and also to prevent a second flush of root growth.

The crop is most sensitive to water deficit during the yield formation period, particularly during the period of rapid bulb growth which occurs about 60 days after transplanting. The crop is equally sensitive during transplantation.

For high yield of good quality the crop needs a controlled and frequent supply of water throughout the total growing period; however, over-irrigation leads to reduced growth. To achieve large bulb size and high bulb weight, water deficits, especially during the yield formation period (bulb enlargement), should be avoided. Under limited water supply small water savings can be made during the vegetative period and the ripening period. However, in such conditions water supply should preferably be directed toward maximizing production per hectare rather than extending the cultivated area with limited water supply.

Water uptake

The crop has a shallow root system with roots concentrated in the upper 0.3 m soil depth. In general 100 percent of the water uptake occurs in the first 0.3 to 0.5 m soil depth CD = 0.3-0.5 m). To meet full crop water requirements (ETm) the soil should be kept relatively moist; under  an Evapotranspiration rate of 5 to 6 mm/day, the rate of water uptake starts to reduce when about 25 percent of the total available soil water has been depleted (p = 0.25).

Crop protection

Weed control: Onion does not compete weeds. Therefore, timely cultivation to destroy weeds when they are still young is needed.

Insect Pests

Thrips (Thrips tabaci) , a small , yellowish sucking insect which attacks the leaves of onion are the most injurious insects  usually during dry weather conditions .Controlled by spraying 500 ml cypermetrin 10% with a mixture of 800 ml water per hectare

Disease control

Purple blotch (alternarea porri) :-Among diseases that are attacking onion plant, purple blotch (alternarea porri) is the major .It attacks leaves, seed, stem and bulb. In the end, darkened are with masses of spots developed in the centers of the lesions. The infected leaves turn yellow and within three to four weeks. Then the fungus grows down into bulbs if no control measure applied

Downy mildew (perenospora destructor):- is also another destructive fungal disease that attacks onion plant usually at field level

The control measures are

- Use long rotational cropping system

- For both fungal diseases , use 3kg Redomil M.Z 63.5 W.p with mixture of 600 lit of water per hectare every week interval

- 2.5kg Mancozeb  80 %W.p   with 600 lit of water in every 10 days interval

Harvesting

The optimum time for harvesting bulbs for sale or for storage is when leaves start falling. The crop can be harvested a week after the falling of 50% to 80 %leaves. Early harvest results in sprouting of bulbs and late harvest gives rise to formation of roots during storage.

Post Harvest Management

Onion bulbs are thoroughly cured before storage. The purpose of curing is to remove excess moisture from the outer skin and neck of onion to reduce losses due to diseases. The time for curing largely depends upon the prevailing weather condition but usually the onions are lay down in wind rows for about 5 days by covering the bulbs with the leaves of the next row. The fully cured onion shows the light neck and the dried scale rustle. After curing, the onion should be graded for local market and export. Then it will be filled in jute bags for transportation.

#### Tomato (Lycopersici esculentum)

Tomato (Lycopersicon esculentum) is most important remunerative vegetable. A rich source of minerals, vitamins and organic acids. Tomato fruits provide 3-4% sugar, 4-7% solids, 15-30 mg per 100g ascorbic acid, and 7.5-10mg/100 ml titratable acidity and 20-50mg /100g fruit weight of lycopene. It is widely grown vegetable in the country.

General Growth Requirement

The optimum temperature required for its cultivation is 15-270C. At high and low temperatures, there is low germination of seeds, poor plant growth, flower drop, poor fruit set and ripening. At high temperature generally, the quality of fruits is poor and there is high evidence of sunscald. Under extreme high and low temperature conditions, the yield and quality of fruits is reduced. Mild winter condition is ideal for seed germination, plant growth, fruit set, fruit development and ripening. An excessive rain adversely affects the fruit set causing flower drop.

Soils

Tomato is grown in varied type of soils .Sandy loam to clay, black soil and red soil having proper drainage. However, sandy loam soil, rich in organic matter is ideal for its cultivation. The PH of the soil should be 7.5 to 8.5. Tomato can tolerate moderate acid and saline soils. Red and black soils are good for commercial cultivation.

Time of Production

Climate, soil condition, and market availability are factors affecting time of tomato production. It can be produced year round; however there is high disease and insect incidence during rainy season.

Seedling Preparation

Tomato can be produced by direct seeding or rising on seed bed and transplanting. For seedling preparation, the soil should be well drained and fertile and free from pest and disease. For one hectare, 300-380gm seed should be sown on seed bed. The size of seed bed can be 1*5m or 1*10m and the seeds should be sown in rows of 15 cm at a depth of 1cm .All management practices like  mulching , watering , pest control etc  are given for proper seedling growth.

Land Preparation and Transplanting Seedlings

The land should be finely prepared. When the seedlings attain 13-15cm height or at about 25-30 days after sowing, transplanting to the field must be carried out. The seedlings are planted at the row of 100cm and 30 cm between plants. Transplanting is done early in the morning and late in the afternoon.

Water application

Frequent irrigation is essential for optimum plant growth, fruiting and yield. The crop should be irrigated at the recommended day’s interval or even more frequently depending on the temperature and humidity. Generally open furrow method of irrigation is followed.

Fertilization: 150 kg per hectare of NPS and 100kg per hectare of urea .Full dose of NPS  is applied immediately after transplanting and full dose of urea is applied one month after transplanting

Staking

Facilitates chemical spray,  watering, harvesting and different field operations.

Crop Protection

Diseases:-Tomato is affected by different diseases especially when moisture is high. Damping off (Pythium spp. and Phytophthora spp.), Aternaria

blight (Alterneria solani), Septoria leaf blight (Septoria  lycopersici), Bacterial wilt (Ralastonia Solanacearum). Leaf curl (transmitted by whitefly), Tomato mosaic (transmitted by contact and seed).

To control diseases, field sanitation and crop rotation should take place. To control leaf diseases 3.5kg Ridomil is diluted in 500 liters of water and sprayed on one hectare.

Insect pests:-Insect pests such as African ball worm and mealy bugs (Ferrisia virgata), are common in tomato production.75gm of cypermetrin diluted in 500 litres of water sprayed on one hectare of tomato farm can greatly reduce crop damage.

Weed control: Weeds compete for minerals, water and sunlight. Therefore, weeding should be carried out between 20 -60 days of transplanting. Generally to minimize the disease, insect and weeds infestation, crop rotation, field sanitation, inter cultivation and hand weeding should be properly done.

Varieties: Eshet, Roma VF, Marglobe, Melka shoala, Melka salsa

Harvesting and Post Harvest Management

Tomatoes are harvested at several-stages like mature green, turning pink, red ripe and over ripe. The stage of harvesting depends upon the purpose for which the crop is harvested .Generally the harvesting is done at mature green to turning stage for distant marketing. For fresh consumption pink to light red tomatoes are preferred. To increase their shelf life after harvest tomatoes are cooled rapidly at 130c. After harvest they are graded into A, B and C grades by hand or machine.

#### Maize (Zea mays L)

Out of many crops grown in this country, maize, which is comparatively of recent introduction, ranks first in yield/ha, fourth in total grain yield and fifth in total hectare (after teff, barley, sorghum and wheat).

The crop is used as a staple food for humans and fodder for livestock. Different types of food, like, 'Injera', porridge, malting and brewing products etc. are prepared from maize.  It is a source of starch, protein and oil. It can be fermented and distilled to provide industrial products like ethyl, and butyl alcohol, acetone, glycerol and acetic acid. Maize is also a cheap and major ingredient for animal feed.

General Growth Requirement

Maize is essentially a warm weather crop.  It is widely cultivated from 500 to 2,600m where mean temperatures are from 10-35 0c .Good result is obtained when maize is grown from 1000 -2200masl. It is a sun and water-loving crop and long hot growing season with plenty of sunshine is favorable. If the mean daily temperature during the growing season is above 200c, the crop takes much less period to mature than in lower temperatures. Optimum temperature for germination is 180 to 200c. The crop fails at temperatures above 450c.  It can tolerate hot and dry weather conditions provided there is sufficient moisture in the soil. Maize requires between 500 to 1500 mm uniformly distributed amounts of rainfall throughout the growing season

Soils

Maize can grow on any type of soil ranging from deep heavy clays to light sandy ones.  However, deep, fertile, well-drained medium soils (loam to silty loam) are most suited to the crop.  The crop is susceptible to water logging and poor aeration conditions. The fertility demands for grain maize are relatively high. The crop is moderately sensitive to salinity but a soil PH in the range of 5.5 to 7.5 supports good crop growth. The yield decreases under increasing soil salinity.

Recommended Maize Varieties : Shone , Limu,BH660,BH661,AMH800(Argane ).

Field Preparation

If it is virgin land, the land has to be cleared of trees, bushes and shrubs leaving some trees and bushes at points in between, but if the site was crop land, crop residues have to be collected in selected places with in a farm immediately after harvest. It has to be exposed to heat and moisture for decomposition.

Generally, clean, smooth, deeply ploughed but firm seedbed is ideal. It is desirable to give first plowing to bury the previous crop residues. Two to three subsequent harrowing may be done to obtain fine tilth.  Proper tillage reduces weed infestation and helps in decomposition of organic matter. Generally based on soil conditions, 3-5 times tillage can be done .During seed bed preparation, ridges have to be prepared for furrow irrigation

Time, spacing, depth Method of Sowing

With the context of the project area, for full irrigation, maize can be sown with in November -December –January. It is generally sown by seed drill in rows spaced 75cm apart. Spacing between plants in a row is kept at 25 cm. The depth of sowing is about 5-10 cm. Seed rate of 25-30 kg of seed is sufficient for one hectare. It should be ensured that the seeds have at least 95% germination. Seed lots with less than 80% germination are likely to impart poor seedling vigor.

Fertilizer Application

Fertilizer is one of the inputs which play a vital role in obtaining optimum yield. Maize is a heavy feeder of nitrogen, Phosphorous, potash, calcium and magnesium. Maize contains more N in its grains than any other soil-derived nutrients; hence require more of N for its production.  100 kg NPS /ha and 200kg/ha urea can be recommended in the project area.

For better fertilizer use efficiency ,one-fourth of total nitrogen and total quantity of Phosphate fertilizers are applied as basal dose at the time of sowing or applied in bands 5-7 cm deep before sowing.  The rest of N should be applied in two equal doses as side-dressing when the crop is at knee high stage (20-30 days after germination) and rest should be applied after the emergence of flag leaf but before tassels emergence .The fertilizer should be applied 10-12 cm away from the base of the plant to avoid any leaf injury.

Water supply and crop yield

Frequency and depth of irrigation and rain has a pronounced effect on grain yield. Maize appears relatively tolerant to water deficits during the vegetative and ripening periods. Greatest decrease in grain yields is caused by water deficits during the flowering period including tasselling and silking and pollination, due mainly to a reduction in grain number per cob. This effect is less pronounced when in the proceeding vegetative period the plant has suffered water deficits. Severe water deficits during the flowering period, particularly at the time of silking and pollination, may result in little or no grain yield due to silk drying. Water deficits during the yield formation period may lead to reduced yield due to a reduction in grain size. Water deficit during the ripening period has little effect on grain yield.

The effect of limited water on maize grain yield is considerable and careful control of frequency and depth of irrigation is required to optimize yields under conditions of water shortage. Where water supply is limited it may therefore be advantageous to meet, as far as possible, full water requirements (ETm) so as to achieve near maximum yield from a limited acreage rather than to spread the limited water over a larger acreage.

Weeding

Weed reduces maize yield greatly. Therefore, proper weeding should be done. First weeding is 25-30 days after seed germination and the second weeding should be at 40-50 days after seed germination. Additionally, mowing should be practiced for lately emerging weed species.

Cultural Control: Generally, the following cultural practices are carried out for weed control in maize.

For parasitic weeds

- Using weed free maize seed

- Cleaning farm implements

- Destroying parasitic weeds before setting seeds

- Maintaining  soil fertility

- Crop rotation practices

For non parasitic weeds

- Proper  land preparation

- Using weed free seeds

- Inter cultivation

- Crop rotation

- Hand weeding at 25- 30 and 40-50 days after crop emergence.

Chemical Control: it is not advisable to use chemicals for this purpose but at very critical conditions, the following chemicals may be recommended to control both parasitic and non parasitic weed species in maize.

Alazine at 2.2-2.75kg/ha active ingredient after maize seed germination.

Primagram Gold 660 SC at 1.32-1.98kg/ha active ingredient after maize seed germination.

Sadik at 1.2-1.6kg/ha active ingredient 2 -3 weeks after crop emergence.

Maize Diseases

Turcicum   Leaf blight can be controlled by using resistant varieties and crop rotation

Common rust  – using resistant varieties

Smut

- Using resistant varieties

- Rouging infected plants /field sanitation

- Crop rotation

- Sowing treated seeds by anti fungus.

Crop insect Pests

To control Stalk borer (Chillo partelus)

- Site clearing and avoiding alternate hosts

- Cut the maize stalk close to ground level at harvest time

- Spread the harvested stalk thinly in the field for about one month in order to expose the hibernating pupa in the stalk to sun’s heat

- Removal of infected plants with dead heart within six weeks after emergence.

- Chemically, use endosulfan 1kg/ha a.i. 4-5 weeks after crop emergence or Carbaryl 2.3kg/ha 4-5 weeks after crop emergence or Diazinon 10 kg/ha 4-5 weeks after crop emergence or cypermetrin 3-3.8kg granule /ha

To control Army worm, boll worm, cut worm

- Apply proper field sanitation

- Plough the field deep in order to expose the hibernating pupa to sun’s heat and different natural predators

- Spray different insecticides such as Malathion, Diazinon and Sevin

- Spray thiodian (Endosulphan )35%EC at the rate of 2 lt/ha mixed with 200-300 lit of water or Thiodan 25% ULV at the rate of 3 lit /ha

To control Weevils and other storage pests

- Proper cleaning of storage houses

- Avoiding suffocation

- Chemically use Pirimiphos-methyl 25-50 gram mixed in 100kg of seed.

- Crop rotation: maize should be rotated with pulses, oil crops vegetables etc

Harvesting and Threshing

Harvesting is done by cutting the entire plant at the ground level or by removing only the cobs. It is done when grains are nearly dry and do not contain more than 20% moisture. The appearance of the plant may be misleading particularly in case of high yielding hybrids whose grain may dry while the plant still look green. Once the harvested cobs are properly dry, it can be either shelled manually or by modern maize Sheller to store the grain.

Storage

Local storages or modern /improved storages can be used depending upon the prevailing condition .In general clean all storage facilities properly before storing the grain. Moisture content of the grain should be reduced below 12% before storage .To control storage pests use Actelic 2% powder ,Fumigants like Phostoxin can also be used

Potato (Solanum tuberosum)

Varieties

There are three types of potato varieties, early, medium and late depending on their duration from tuber to tuber. The present released variety of Irish potato are Jalane ,Gebisa ,Chala ,Dagim etc.

Climate

Potato is largely grown in cool regions, where mean temperature does not normally exceed 180c. Optimum temperature for potato growth and development ranges between 150c to 250c. Minimum night temperature is of great significance for tuberization and yield. Temperature below 210c is favorable for tuber formation. There is little if any, tuber formation at temperature above 290c. At low temperatures the vegetative growth of the plant is restricted. That is why potato is grown as summer crop in the hills and cool season crop in tropics and sub-tropics. Long photoperiod promotes haulm growth and delays, tuberization and maturity where as short photoperiod reduces haulm growth but tuber initiation is early and the crop maturity period is reduced.  For best yields, potato crop needs long day conditions during growth and short day condition during tuberization.

Soils

Potato can grow in all types of soils but light, well drained sandy loam soils are best-suited. Potato plant prefers soils in acidic to neutral range (PH 5.5 – 7.5). Black soils are, however, prone to cracking on drying and expose tubers to sun and tuber moth infestation.

Propagation

Potato is conventionally grown vegetative using tubers as planting material. Each tuber has several eyes distributed over its surface with each eye having 3-5 buds, which develop into sprouts. When planted in soil, the sprouted tuber establishes itself into a plant. Each sprout, on emergence above the ground, develops into stem bearing leaves and branches.  The quality of seed is most important to ensure good crop production. This is because, seed tubers account for about 40-50% of the total input cost.

Seed size and spacing

All sizes of tubers can be utilized as seed but medium sized (25-55 mm or 25-27g) often called as seed size is better than other grades. Even in seed size tubers of 35-45 mm or 45-50g are ideal.  Comparable yields can be obtained by planting medium sized tubers (35-45) at 60cm x 20cm spacing and large-sized tubers (45-55mm) at 60 x 55 cm spacing, keeping the plant population at 83000 and 67000 plant/ha respectively. For small-sized seed (25-35 mm) a population of 1,11000 plant/ha (60cmx15cm) is ideal. The optimum seed rate for getting high yield is 20-25q/ha for 15g seed, 25-30 q/ha for 30g seed and 30-35 q/ha for 45g seed size.

Pre-sprouting of Seeds

Pre-sprouting of tubers before planting ensures multiple, stout and healthy sprouts which help in quick emergence and uniform stand of crop. It also increases the number of tubers and leads to larger proportion of seed-sized tubers. For pre-sprouting the tubers are kept in shade in diffused light in baskets or trays or spread in thin layer on the floor. Seed stocks are examined twice a week to remove rotten tubers. One week pre-sprouting period is enough for all sizes tubers.

Seed-bed Preparation

The seed-bed is properly prepared by a deep ploughing followed by harrowing to get a fine tilth free from weeds. The number of harrowing depends on weed infestation and type of soil.

Time and Method of Planting

Potato can be planted at least twice in a year, August-September and December-January. At higher altitudes (2500-3000m), the crop should be grown during summer when free from frost and very low temperatures. The tubers are planted in lines 60 cm apart.  The furrows are opened and tubers are kept in furrows at a distance of 20cm from tuber to tuber and covered with soils using ridger. The planting should be done in the morning or in the evening to avoid heated soil covering during mid-day in plains.

Manuring

Nitrogen is the most important nutrient for potato crop. A mature potato crop yielding between 25 and 35 tons /ha need about 120-150kg N/ha. The peak period of N uptake varies from 40-70 days or more depending on the altitude and duration. The nitrogen application increases the plant growth, leaf area, tuber number and tuber size. The maximum N responses is recorded in alluvial soil followed by red and black soils. Potato varieties also differ in their N requirements. It is recommend to apply 15-20 tons/ha farm yard (dung) manure and incorporated in the soil before planting. Half dose of N and full dose of P2O5 and K2O is applied in furrow and mixed in soil at planting. The remaining half dose of N is applied at the time of earthing-up.  P is the second limiting nutrient in potato production. It is generally applied at the rate of 45kg P2O5 / ha.  On an average potato crop remove 150 kg k/ha.  Therefore, about 100-115 kg K/ha is required to apply. The fertilizer application should be based on the soil test value.

Interculture

Earthing up and weeding of potato are done as soon as weeds emerge and potato plants are about 8-10 cm tall. The final earthing up is done 28-35 days after planting and half dose of N applied at this stage.

Irrigation

Water forms about 80% of potato tuber. Therefore, adequate and regular water supply is needed for its sustained growth. The quality of water used for irrigation is also important. Water with high quantity of salt is undesirable for potato. The first irrigation is given immediately after planting, second at 12-15 days of sowing (2-5% germination). Subsequent irrigation is given at weekly or 10 days intervals. Water stress reduces tuber yield and increase the proportion of small-sized tubers.

Harvesting and Post-harvest Management

Potato tubers are harvested as soon as they mature. In loam or heavy loam soils tubers should not be allowed to remain in wet soil after maturity because the lenticels on their surface may proliferate and impart unacceptable look to the tubers in wet land. After harvesting, tuber should be surface dried and kept in shade in heaps for about 10 days or more. The produce should be graded on the basis of tuber size and bagged for marketing.

# 4. CONCLUSION AND RECOMMENDATION

## 4.1. Conclusion

In view of the fact that increase in agricultural productivity is a popular motive for farmers and a driving force in the country and respective project area as a whole, considerable attempts were made   by government to realize agricultural productivity of food crops, cash and industrial crops etc   to alter the state of agriculture in sustaining food security and providing necessary raw materials for agro processing industry.

However, the back warded  traditional  farming practices on these exhausted land   and  less eminent utilization of existing irrigation farm land and water resource potentials coupled with increasing population  from year to year, the capacity of the land to feed the ever increasing population and  assuring  cash crops and raw material for industry  with  rain-fed agriculture is becoming  a great  challenge. Thus, it has got absolute attention   to consider the available  irrigable land and water resources in the area to enhance the contribution of irrigated agriculture in the overall agricultural economy.

Accordingly, the project is awarded to AYJEF in order to undertake the study and design works of the project by multi-disciplinary teams of the firm.

Agronomy study mainly focused on the evaluation of present /existing agronomic conditions, prevailing constraints and potential of the area and proposes irrigated agriculture with its requirement and the crop production and productivity are hampered by several constraints like lack of  relevant staff  , inadequate extension agent coverage and lack of institutional support,  inadequate supply and improper input utilization, inadequate extension service , poorly developed research and extension linkages , inadequate rural credit , marketing problems , weeds, diseases and insect pests , pre and post harvest handling problems etc.

Despite the number of agricultural production constraints, the study area has fairly good  potential that can be exploited for small scale  irrigation and   rain fed production practices through comprehensive and integrated agricultural development programs. Therefore, to improve the current low level of production and boost productivity both by irrigated and rain fed production, the following recommendations are made

## 4.2. Recommendation

- Adequate and experienced agricultural staffs ought to be recruited at District and kebele level with special emphasis on irrigated agriculture to execute proper agronomic management and practices.

- Sustai1nability of the project is very much dependent on the full participation of the beneficiaries in all aspects of the project stages. Hence due attention should be given to involve the local community in the planning, problem identification, prioritizing, implementation, monitoring and supervision processes.

- Drainage of land has to be carefully done

- Water user associations  should be strengthened

- Support institutions have to be organized to capacitate the needy   community accessible to the distribution of sufficient inputs such as improved seeds, fertilizers, pesticides ,improved farm implements, draft power, water pumps etc at affordable cost together with their time of application, use rates and method of application to attain projected goal

- If pesticides usage becomes the last resort for pest control , the chemicals have to be carefully stored , handled and utilized  and but very often ,IPM  has to be advocated for pest control

- Marketing channels have to be established locally,  regionally and internationally for the farmers in order to earn good income from product marketing

- All stakeholders related to agricultural development have to be fully equipped with trained manpower and equipments up to grass root level by increasing the number of grass-root level extension staff for proper guidance of the farmers in order to render sound and effective agricultural extension to the needy community about crop production skills (rain fed and irrigated) starting from site selection and land preparation up to harvesting, storage and processing. On top of that irrigation authority at district and kebele level has to be fully staffed and necessary logistics have to be fulfilled.

- Improve credit facilities, crop marketing, transportation and storage at the kebele and village level with proper support price of produce.

- Research centers and universities  have to extend trial/research  stations to the command area with the special emphasis on improved variety development , fertilizer trial ,pesticide trial , organic fertilizer trial ,site specific crop water requirement , irrigation period and interval and method of irrigation ,tillage ,seed rates , post harvest crop loss assessments, weeding trials etc.

- Some simple and less costly, but high output land management practices such as plantation of multipurpose tree species are recommended to plant in and around farm boundaries, so that soil erosion could be reduced and soil fertility and overall condition could be improved.

- Special awareness creation has to be made by all stakeholders in order to enhance and scale up  best practices  obtained on model farmer  fields  about indigenous practices of multiple cropping systems , organic(compost ,FYM, green manuring ) and inorganic fertilizers utilization , improved seeds utilization , crops production and managements both in rain fed and irrigated agriculture .

- Linkage between farmers  research and extension systems have to be strengthened  in order to enhance the sustainability of crop production in the command area

- Crop production has to be diversified and intensified; besides, existing mono cropping systems of some crops have to be avoided to decrease pest outbreak, improve soil fertility and overall yield.

# 5. REFRENCES

1. CROPWAT 8.  Viale delle Terme di Caracalla, 00100, Rome,  Italy .FAO (2009).

2.  Crop water   requirements .FAO Irrigation and Drainage Paper 24.

3. Crop Evapotranspiration - Guidelines for Computing Crop Water

Requirements – FAO Irrigation and Drainage paper 56, Food and Agriculture

Organization, 1998

4. በኢ.ፌ.ዲ.ሪ. የግብርና ሚኒስቴር  የብርዕና አገዲ ሰብልች አመራረት  ፣የቅባት ሰብሎች፣ የጥራጥሬ ሰብሎች ቴክኖሎጂ ፓኬጅ  ታህሳስ 2007  አዲስ አበባ- ኢትዮጵያ

5. “Ethiopian   Institute of Agricultural   Research, “Usage of Crop  Technologies, 2007.

6. Federal   Democratic republic of Ethiopia, ministry of water resources procedural guidelines for small and medium scale irrigation projects in Ethiopia study guideline   on irrigation agronomy, August 2002.

8.  Japan International Cooperation Agency (JICA) Oromia Irrigation Development Authority (OIDA) Guideline for Irrigation Master Plan Study Preparation on Surface Water Resources May, 2014, The Project for Capacity Building in Irrigation Development (CBID)

9. Ministry of Agriculture and Rural Development,” Crop  Development Department ,Crop Variety Register issue No.9 ,2006,Addis Ababa ,Ethiopia .

10. Revised agro- ecological classification, 2005.Addis Ababa.

11. Study and Design Technical guideline For Irrigation Projects May, 2014.Ethiopia.