# OROMIA IRRIGATION DEVELOPMENT AUTHORITY

THE NATIONAL REGIONAL STATE OF OROMIA

AGRICULTURAL GROWTH PROGRAM (AGP II)

Lot Nine of Eighty Small Scale Irrigation Projects Feasibility Study and Detail Design

Volume IV: Watershed Management Report of Birbirsa Small Scale Irrigation Project

Final

March 2019

Finfinne

AYJEF Water Woks and                 P.O.Box: 43329       Tele: +251 911 304019

Business Service PLC                                           E-mail: ayconsult1@gmail.com

Contents

LIST OF TABLE	v

LIST OF FIGURE	v

LIST OF ANNEX	v

ABBREVIATIONS AND ACRONYMS	vi

EXCUTIVE SUMMERY	vii

1.	INTRODUCTION	1

2.	OBJECTIVES	2

2.1.	General Objective	2

2.2.	Specific Objectives	2

3.	SCOPE OF THE STUDY	2

4.	METHODOLOGY	3

5.	DESCRIPTION OF WATER SHADE	4

5.1.	Location	4

5.2.	Climate	5

5.2.1.	Rainfall	6

5.2.2.	Temperature	7

5.2.3.	Relative Humidity	8

5.2.4.	Wind Run	9

5.2.5.	Sunshine Hours	9

5.2.6.	Length of Growing Season	10

5.3.	Land use Land Cover	11

5.4.	Topography	12

5.5.	Soil	12

5.5.1.	Soil Texture	13

5.5.2.	Soil Color	13

5.5.3.	Soil Structure	13

5.5.4.	Soil Consistency	13

5.6.	Major Soil Types in the watershed	13

5.6.1.	Fluvisols	14

5.6.2.	Mollic Cambisol	14

5.6.3.	Humic- Chromic Acrisol	15

5.7.	Land capability classification	15

5.8.	Experiences in Soil and water conservation activities	18

6.	SOCIO-ECONOMIC CONDITIONS	19

6.1.	Demographic Characteristics	19

6.2.	Existing Economic Activities in the Project Area	20

6.2.1.	Crop Production	20

6.2.2.	Livestock Production	20

6.2.3.	Infrastructure & Social Services	20

6.2.4.	Social Services	21

6.2.5.	Infrastructure Conditions	21

6.2.6.	Market Conditions	21

6.3.	Development Opportunity & Constraints	21

6.3.1.	Constraints	21

6.3.2.	Potentialities	22

7.	MAJOR PROBLEMS OF WATERSHED	23

7.1.	Soil Erosion	23

7.2.	Land Degradation	23

7.3.	Deforestation	24

7.4.	Shortage of Grazing Land	25

8.	INTEGRATED WATERSHED DEVELOPMENT INTERVENTIONS	26

8.1.	Soil and Water Conservation	26

8.1.1.	Physical Conservation Measures	26

8.1.2.	Biological Conservation Measures	28

8.2.	INTERVENTIONS IMPLEMENTATION SCHEDULES	29

9.	CONCLUSION AND RECOMMENDATION	31

10.	REFERENCES	32

11.	ANNEXS	33

# LIST of TABLE

Table 1: Summary of Meteorological Characteristics and ETO of Project Area	5

Table 2 : Analytical Moisture Balance of the Study Area.	6

Table 3 : Interventions Implementation Plan	30

Table 4 : Summery of Budget for Conservation Measures	30

# List of Figure

Figure 1 : Birbirsa Catchment Area	4

Figure 2 : Max. verse minimum temperature	8

Figure 3 : RH verse temperature	9

Figure 4 : Sunshine hours of the command area	10

Figure 5 : LGP of the command Area.	11

Figure 6 : Birbirsa Watershed by Slope Class	17

# LIST of ANNEX

Annex 1 : Structured Questionnaire for Individual Farmers	33

Annex 2 : Semi Structured Questionnaire for Group Discussions in the Community	36

Annex 3 : Checklist Questionnaire for Data Collection	39

# ABBREVIATIONS AND ACRONYMS

| BBF BBM BD DEM EC ESP ETO FAO FC GIS GPS GTP II HC HCI LCs LGP LQ LURs LUTs OIDA OM OWWDSE PET PLC PWP RAWC RF RH SMU SSH SSIP TN TOR UNDP UTM | Broad Bed Farrow Broad Bed Maker Bulk Density Digital Elevation Model Electrical Conductivity Exchangeable Sodium Percentage Food and Agriculture Organization Field Capacity Geographical Information System Geographic Positioning System Growth and Transformation Plan II Hydraulic Conductivity Hydrochloric Acid Land Characteristics Length of Growing Period Land Qualities Land Use Requirements Land Utilization Types Oromia Irrigation Development Authority Organic Matter Oromia Water Works Design and Supervision Enterprise Private Limited Company Permanent Wilting Point Readily Available Water Capacity Rain Fall Relative Humidity Soil Mapping Unit Small Scale Irrigation Project Total nitrogen Terms of Reference United Nations Development Program Universal Transverse Mercator |
| --- | --- |

# EXCUTIVE SUMMERY

Birbirsa watershed  is  found  in  the  Dhedhesa  sub-basin  of  the  Abbay  Basin  and  is  located  administratively  in Guto Gida  districts of east wolega zone. The watershed covers about 4,400 hectare, including the command area. Out of the total watershed area 87 percent is cultivated, 8 percent is grazing and 2 percent is under forest or bush land cover and 3 percent for settlement and other uses. The watershed has slope 0 to 15 per cent. A study conducted on the biophysical and socio-economic conditions of the watershed shows that land  degradation  in  the  form  of  soil  erosion,  shortage  of  livestock  feed  as  compared  to  the number of livestock, lack of community awareness in natural resources conservation and other socio-economic problems are common.

The problem of soil erosion and the subsequent sedimentation in some parts of Birbirsa watershed is a threat and devastating challenge to the being constructed irrigation structures in the watershed and downstream areas due to generating high run-off discharges and imposing huge sediment yields,  which  certainly  result  in  reducing  the  life  span  and  efficiency of the structures, unless  the  upper  watershed  is  treated  with  appropriate  watershed  management interventions and strategies. Proposing appropriate and area specific development interventions in order to halt soil erosion problem  in  the  upper  catchment  of  the watershed  and  sedimentation  problem  in  the  being constricted irrigation project area and downstream areas while increasing the land productivity of the watershed  is  the  primary  objective  of  this  document.  Considering  the  direct  correlation between  poverty  and  natural  resource  degradation,  proposing  livelihood  interventions  that enhance the economic and social well-being of the community is also the other major objective.

The proposed interventions include soil and water conservation measures such as soil bunds, cut-off drains, grass strips, alley cropping, etc. and various strategies of livelihood components   and income generating schemes. In this regard the watershed will be treated with physical conservation measures like soil bunds, fanyajuu terraces, and check dams of different types. Similarly biological conservation measures grass strips; intercropping, mixed cropping will be implemented. These interventions are proposed to be implemented over 10 year’s period. Along with these  interventions, strategies for  implementation and  institutions responsible  for  carrying  out  the  implementation  of  the  proposed  interventions  have  been indicated.  A cost  of  project  have  been  estimated  to  be  205,205 Ethiopian  Birr  over  five years period of  the project. Additionally the community labor contribution constitutes birr 164,164.  Accordingly, total cost of project has been estimated to be 369,369 Ethiopian Birr. Over five years of the project.

# INTRODUCTION

Watershed management is the integration of appropriate technologies and strategies within  the  natural  boundaries  of  a watershed  or  drainage  basin  for  optimum  development through  conservation,  regeneration  and  judicious  utilization  of  all  resources  land,  water, animal and human within a particular watershed. This process provides a chance for beneficiaries to  balance diverse  goals  and  uses  for  environmental  resources,  and  to  consider how their cumulative actions may affect long-term sustainability of these resources.

Human  modifications  of  lands  and  waters  directly  alter  delivery  of  water,  sediments,  and nutrients, and  thus  fundamentally alter agricultural activities. People have varying goals and values relative to uses of local land and water resources. Watershed management provides a framework for integrated decision-making Watershed management  related  to agricultural activities  involves  the planning and execution of  need  based  soil  and water  conservation  practices  and  other  socio-economic  interventions which will consequently control the soil loss, reduce the run-off and improve the livelihood of the community and thus will provide more time of concentration for run-off water to infiltrate into  soil which  will  finally  contribute  to  ground  water.  It is, therefore, essential to have a watershed as a basis for planning and implementation of various soil and water conservation programs. The dynamic processes of the biosphere and the  effects  of  vegetation  and  soils  on  these  processes  such  as  interception,  evaporation, transpiration, infiltration, percolation, and surface run-off, subsurface and ground flow, all are affected by land management activities on the watershed. An inappropriate farming practice in the watershed leads to resource deteriorations, both ex-situ and in-situ. Watersheds with  degraded  uplands  result  in  sedimentation  of  reservoir  and  irrigation  structures  with frequent  flooding  at  downstream  at  the  outlet  of  the  watershed  system.  Appropriate  watershed management  schemes  greatly  supplement  the  sustainability  of  these  irrigation  schemes,  in addition  to  the multiple outputs  to be derived  including  forest,  livestock,  and  crop products  on upland watershed.  Birbirsa watershed has relatively better vegetation cover   and soil erosion is slight in many places except  in  areas  where forest clearing is taking place particularly that of around devastated valley edge areas.  Though, in near past the exploitation of natural resources is getting induced with the need to open new land for cultivation and cutting of trees for sale. In many areas of the watershed the depth of soil is good enough to support agricultural production.

# Objectives

## General Objective

The general objective of integrated watershed development study is to minimize the sediment load in Irrigation Scheme head work and related structure so as to ensure the expected benefits out of it effectively and in a sustainable manner.

## Specific Objectives

The specific objectives of this study document are:

- Minimize  the  soil  erosion  problem  that  prevails  in  the  upper  catchment  of  the watershed through appropriate soil and water conservation structures.

- Improve the livestock productivity of the upper catchment by promoting production of improved fodder plants.

- Developing  of  springs  to  address  the  water  shortage  of  the  upper  catchment community.

- Improve  the economic  status and opportunity of  the upper catchment community members through diverse income generating schemes

# Scope of the Study

The scope of the study mainly includes the following but not limited to:

- LGP analysis and interpretations

- Demarcating the agro ecological zone

- Perfuming land evaluation such as land capability and land use adjustments

- Erosion rate estimation and sediment yield estimation and the implication for the future project

- The computation of important hydrological parameters runoff coefficient©, Curve Number (CN), and determination of design period usually 10 years for soil and water conservation structures.

- The problem categorization and prioritizing and root causes and proposed solutions.

- To develop mitigate plan for controlling the erosion problem and maximizing the production there.

- The materials used and the work force required

- The time requirements, schedules and priorities including the implementation calendar

- The expected benefits, the training requirements and provision of maintenance and evaluation

- The professional and institutional responsibility set.

- The cost breakdown and budget for the total impetration activities

- The plan map preparation (At least at 1:50,000 scale) showing the Woreda, Kebele and sub water shade.

# Methodology

The methodologies for conducting this study:

A Working guideline preparation will the first task in the process of water shade management study.   The  guideline will prepare in  such  a  way  that  it  could  answer questions  like: what  kinds  of points  that we   need  to  consider, who  could  be  the possible source for the  information we are looking for, the method of approaching the  source,  the  method  of  collecting/recording  the  information,  the  period  of collecting the information, etc.

Both primary and secondary source of data are used for the water shade management study.

Secondary source of data like study reports, national & regional guidelines, maps, literatures related to the subject under consideration, etc. will be used.

The Arjo Dhedhessa Integrated water shade Development project document prepared by Federal Democratic Republic of Ethiopia Abay Basin Authority is the secondary Source of data in this study.

Besides,  questionnaires  to be  develop  and  used  to  collect  primary  data  from all levels of concerned offices.

These  primary  source  data  are  mainly  focuses  on  identification  of intervention  types,  the  type  and  magnitude  of  internal  potential  and  external support, verification of data that are collected through secondary source of data, etc.

# Description of Water Shade

## Location

Birbirsa watershed, which is originating mainly from fairly protected (Good vegetation cover) upper catchment area.  It is located in Oromia Regional State, Guto Gida district of East Wolega zone.  The water shade is found in Negeso kebele located between 090 02’447” N latitude and 360 33’185” E longitudes. The watershed covers a total area of 4400 hectare including command area.

Figure 1 : Birbirsa Catchment Area

## Climate

The main climatic information that is most important for watershed management is rainfall and temperature.  Others like evapotranspiration, relative humidity, solar radiation are subsidiary for planning watershed management. Moreover, the importance of such climatic data can be implied from rainfall and temperature. The monthly rainfall within and nearby areas of the watershed was collected from available metrological stations.

Climate is one of the components of an environment that determines formation and characteristics of natural vegetation, crop production and other farming systems of a particular area. Depending on the consecutive mean rainfall, the climate of the study area is classified as sub humid with length of the growing period (LGP) of 210 days (revised agro-ecological classification, 2005).The Nekemte  meteorological station is  located close to the irrigation command area and is relatively reliable source of information on climatic data. Accordingly, monthly records of rainfall from 1999 to 2014 have been obtained from the meteorological station whereas maximum temperature and minimum temperature for the years of   1998 to 2015 and sunshine hours for the year of 2009 to 2014 and relative humidity from 2008 to 2015had been analyzed from the station .On the other hand, wind speed had been derived by New_ Localism_ 1.10 software.

Table 1: Summary of Meteorological Characteristics and ETO of Project Area

| Month | Min Temp | Max Temp | Humidity | Wind | Sun | Rad | ETo |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | °C | °C | % | km/day | hours | MJ/m²/day | mm/day |
| January | 12.2 | 26.1 | 62 | 130 | 8.5 | 20.1 | 3.98 |
| February | 13.5 | 28 | 57 | 147 | 8.4 | 21.2 | 4.59 |
| March | 14.3 | 28 | 58 | 147 | 7.8 | 21.3 | 4.74 |
| April | 14.5 | 27.4 | 65 | 138 | 7.8 | 21.5 | 4.62 |
| May | 13.9 | 24.9 | 79 | 130 | 6.1 | 18.5 | 3.73 |
| June | 12.9 | 22.5 | 87 | 95 | 4.9 | 16.3 | 3.08 |
| July | 12.8 | 21.1 | 89 | 104 | 3.1 | 13.8 | 2.6 |
| August | 12.9 | 21.1 | 90 | 95 | 2.9 | 13.7 | 2.57 |
| September | 12.9 | 22.5 | 88 | 104 | 3.6 | 14.8 | 2.79 |
| October | 12.9 | 24 | 79 | 147 | 6.7 | 18.8 | 3.57 |
| November | 12.6 | 24.5 | 73 | 147 | 7.5 | 18.8 | 3.64 |
| December | 12.1 | 25 | 68 | 147 | 7.8 | 18.6 | 3.67 |
| Average | 13.1 | 24.6 | 75 | 128 | 6.3 | 18.1 | 3.63 |

### Rainfall

Rainfall needs to be examined in terms of its ability to satisfy crop requirements. By comparing the results with the potential evapo-transpiration, the water available to crops could be determined. For example a positive difference between rainfall and potential evapo-transpiration for the greater part of a growing season would ensure good crop growth. A negative result during the critical growing season would indicate insufficient moisture for that crop or the need for irrigation.

On the other hand, an excess of precipitation over the potential evapo-transpiration during the wet season would indicate the importance of drainage. A study of the rainfall intensity (quantity over a given period of time) would give an indication of the potential water erosive or of the runoff to be expected and of the dimensions which would have to be given to surplus water disposal channels etc.

Some qualitative aspects of precipitation are also worthy of attention: for example, the duration and frequency of hail storms, in relation to the periods when the land is occupied by crops, or the occurrence of rain storms associated with strong winds, etc.

Even though it is erratic and showing differences among consecutive years, the rainfall type is mono modal that gives maximum rainfall in the month of August (398.0

mm) and the mean minimum rainfall is recorded in January (10.2mm).

Table 2 : Analytical Moisture Balance of the Study Area.

|  | Jan | Feb | March | April | May | June | July | Aug | Sep | Oct | Nov | Dec |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PET | 116.4 | 118.2 | 138.9 | 141.3 | 121.6 | 99.3 | 86.4 | 89.9 | 95.3 | 117.8 | 114.1 | 114.2 |
| 0.5PET | 58.2 | 59.1 | 69.45 | 70.65 | 60.8 | 49.65 | 43.2 | 44.95 | 47.65 | 58.9 | 57.05 | 57.1 |
| RF |  |  |  |  |  |  |  |  |  |  |  |  |
| 1999 | 42.4 | 2.4 | 0.6 | 94.8 | 382 | 331.1 | 194.9 | 250.9 | 238.2 | 338.7 | 23.8 | 8.9 |
| 2000 | 0.3 | 0 | 2.9 | 107.2 | 224.9 | 509.3 | 332.2 | 540.9 | 240.8 | 138.8 | 19.2 | 19.7 |
| 2001 | 0 | 26.4 | 50.8 | 83.6 | 240.8 | 349.6 | 336.2 | 352.4 | 267.4 | 168.8 | 16 | 50.2 |
| 2002 | 24.9 | 21.1 | 74.3 | 106.2 | 77.4 | 376.8 | 427.3 | 260.3 | 186.9 | 117.6 | 0.2 | 33 |
| 2003 | 0.3 | 47.5 | 54.9 | 21.3 | 55.1 | 422.9 | 429.7 | 439 | 312.9 | 18.9 | 25.5 | 9.5 |
| 2004 | 7.2 | 6.9 | 12.7 | 76.6 | 206.3 | 286.6 | 409.9 | 433 | 248.9 | 71.9 | 20.2 | 11.9 |
| 2005 | 9.5 | 0 | 131.5 | 60.3 | 241 | 387.4 | 346.2 | 438.8 | 355.6 | 219.9 | 58.5 | 0 |
| 2006 | 0.5 | 4.8 | 56.6 | 71.3 | 209.4 | 350.6 | 476 | 339.1 | 278.3 | 213.8 | 50.4 | 88.6 |
| 2007 | 4.7 | 54.4 | 47.8 | 141.7 | 217.5 | 462.6 | 355.6 | 447.4 | 297.8 | 86.7 | 56.8 | 0 |
| 2008 | 13.1 | 0 | 0.1 | 227.4 | 368.3 | 416.7 | 412.1 | 384.9 | 301.7 | 235 | 82 | 0 |
| 2009 | 0 | 22.7 | 35.5 | 173 | 97.9 | 314.5 | 251.5 | 395.7 | 412.1 | 286.2 | 15.3 | 18.4 |
| 2010 | 7.1 | 25.6 | 3.5 | 27.6 | 532.7 | 518.3 | 450.9 | 318.7 | 366.8 | 133.6 | 81.3 | 16 |
| 2011 | 42 | 2 | 51.7 | 77.8 | 265.4 | 403.4 | 259.3 | 418.2 | 349.4 | 82.1 | 57.4 | 1.7 |
| 2012 | 0 | 0 | 26.9 | 32.2 | 251.9 | 307.4 | 399.8 | 593.8 | 314 | 99.4 | 68.8 | 15.1 |
| 2013 | 11.7 | 3.3 | 23.4 | 5.9 | 404.1 | 357.1 | 392.7 | 293.9 | 301.5 | 91.9 | 79.8 | 0 |
| 2014 | 0 | 3.8 | 56.8 | 149.5 | 536.4 | 435.6 | 360.2 | 460.2 | 276.6 | 209.6 | 31.4 | 0 |
|  | 163.7 | 220.9 | 630 | 1456.4 | 4311.1 | 6229.9 | 5834.5 | 6367.2 | 4748.9 | 2512.9 | 686.6 | 273 |
| Mean RF | 10.2 | 13.8 | 39.4 | 91.0 | 269.4 | 389.4 | 364.7 | 398.0 | 296.8 | 157.1 | 42.9 | 17.1 |
| SD | 166.3 | 165.9 | 164.0 | 162.4 | 163.6 | 166.5 | 164.4 | 160.1 | 123.3 | 80.7 | 27.8 | 22.8 |
| CV | 16.3 | 12.0 | 4.2 | 1.8 | 0.6 | 0.4 | 0.5 | 0.4 | 0.4 | 0.5 | 0.6 | 1.3 |

The rainfall data shows both systematic and irregular variations during onset and offset over time. These variations play significant role in adapting farming systems and thus productivity. The characterization of systematic variations in the rainfall data is important; not only in long term agricultural planning but as basis for long range forecasts.

Understanding the year-to-year variation of rainfall is very important not only for crop production but also for planning in agricultural development and hydrological management. Coefficient of variation as a tool provides a measure of months for consecutive years and for causative years to see the level of variation. Generally, rainfall in the study area is generally characterized by high variability (C.V.> 30 %) indicating that the rainfall pattern is less dependable.

### Temperature

Air temperature is the most important climatic variable that affects plant life. Each species and variety of plants and each age group of plants has its own upper and lower temperature limits. Beyond these limits, a plant becomes considerably damaged and may even be killed. Therefore, temperature should be considered while dealing with agricultural planning of an area. The mean maximum temperature ranges between 21.10c to 28.10c and the mean minimum from 12.10c to 14.50c

The highest mean maximum temperature in the area is recorded in the month of February and March and the highest mean minimum is in the month of march and April. Generally, the temperature condition of the area is suitable for growth and maturation of crops.

Figure 2 : Max. verse minimum temperature

### Relative Humidity

Relative Humidity (RH %), the water vapor contained in the atmosphere, is expressed as the percentage of the ratio of actual to saturation vapor pressure. This, together with wind, gives an indication of drying power of the air important for drying and storage of crops, while fluctuations in values often affect the incidence of plant diseases as well as the safe storage of harvested crops.

The mean monthly relative humidity in the study area varies from 56 % in the month of February to 90 % in the month of August. It is inversely related to temperature of the area whereby it shows an increasing trend when the temperature decreases and viseversal.

Figure 3 : RH verse temperature

### Wind Run

A study of the velocity, frequency and direction of winds is important as regards of the rate of transpiration of crops, or the necessity of planting wind breaks. Both maximum and minimum wind speed is significant in crop performance especially in the late development stage. Occurrence of strong winds may result in lodging.

The study area exhibits high wind in the months of October to march besides to the highest mean maximum temperature   creating an increase in the loss of moisture in the form of vapor while it drops in the times of peak rainy season .Generally, availability of the strong wind and temperature negatively affect the availability and distribution of rainfall in the area.

### Sunshine Hours

Daily length/ sunshine hours are very important in photosynthetic plant growth and estimation of crop water requirement. The study area experiences sunshine hours varying from highest (8.5hrs /day) in January to lowest (2.9 hrs /day) in august. It is relatively positively correlated with temperature whereby temperature rises up with long duration of sunshine hours and viseversal having significant influence on the growth and maturation of crops.

Figure 4 : Sunshine hours of the command area

### Length of Growing Season

The length of growing period could give a highlight about the potential crops and varieties to be considered to develop crop basket for the given area. The growing period analysis is based on rainfall, potential evapo-transpiration and soil moisture storage capacity.

Length of Growing Period (LGP) of the study area was determined by mean rainfall and half potential evapo-transpiration (0.5PET). From the comparison of mean rainfall and half potential evapo-transpiration (0.5PET), LGP was obtained where the amount of rainfall exceeded half PET and addition of some days until the stored water was evaporated (i.e. the dates starting from the point when RF curve cross 0.5 PET and up to it goes down (RF curve crosses 0.5 PET again) was considered as LGP period of the area adding of some days needed to evaporate the stored moisture in the soil depending on dominant soils of the area.

Figure 5 : LGP of the command Area.

Accordingly, the rainfall pattern in the area is mono modal and there are almost 210 growing days in which the crops can be sown /planted and grown successfully.

Almost from November to march is dry period and crops cannot be grown without full  irrigation apart from recession farming area which need supplemental irrigation  .This was also confirmed during the site visit and discussion with concerned stakeholders and local community. Generally in accordance with the revised agro-ecological classification, 2005 the project area is classified in the range of sub humid agro-ecological classification.

## Land use Land Cover

The identification and mapping of the present land use classes of watershed is essential part of watershed management.  The  way  a  given  land  unit  is  used  determines  the  hydrologic characteristics of watershed and development potentials and constraints. A plan of watershed management should be based on the type of land use along with other factors that define the conditions.  Moreover, the type of land development intervention is directly related to the land use. The land use land cover of the district is allocated as 83.7% for annual and perennial crops cultivation, 4.8 %for forest, bushes; 6.5% for grazing land; 2.2 %degraded land, 2.8% swampy and other uses

The land use land cover of the specific command kebele is 87% cultivated land, 8% grazing land 2 % forest bush and shrub land and 3% settlement and other use such as water bodies, rocky areas etc.; however, the status of land use and cover of the specific command area indicates 96% of cultivated land with recession farming with cereals and vegetables   and upland rain fed crops, 2% of shrub and   bushy grassland, 2% by other uses like water bodies, gorges etc.

Generally the specific command area land use land cover is more dominantly exhibited as cultivated land though there is very few other cover.

  - Topography

The slope of the watershed is categorized as undulating and rolling to hilly plateau having uniform drainage.  The  categorization  is  as  per practice  prevalent  in  Ethiopia  for  soil  and  water  conservation.  Such  classification  and categorization  is  also  useful  tool  for  land  capability  classification.  The slope class of the watershed characterized as in the range of 0 to 15percent.  Cultivation takes place in most parts of this slope range. Soil and water losses from field are influenced by the steepness, length and shape of slopes. The shape of slope is expressed in terms of curvature. Slopes may be roughly convex or concave. Convex slopes increases in steepness toward the bottom. Thus, runoff velocity also increases toward the bottom of slope. Whereas concave slopes flatten out toward the bottom of the  slope  and  sediment  carried  in  runoff water  settles  out  as  flow  velocity  decreases. With intense rains and high velocity of flows, however, water may concentrate in concave slopes and start gulling.  If gulling does not occur,  the area  toward  the bottom of slope accumulates high organic matter  and  surface  soil,  and  becomes more  valuable  as  cropland. On convex slopes there is no deposition, only removal, and these slopes become progressively poorer. Slopes of Birbirsa watershed  areas  are more  of  convex  type  of  slope  curvature  that  has serious cause to generate erosion and run-off.

## Soil

Soils have particular importance in watershed management.  The significances of soils characteristics, which influence the process in a watershed, are soil permeability, soil erosion and compositions of nutrients that help vegetative growth. These characteristics are influenced

mainly  by  soil  properties  such  as  soil  texture,  structure,  depth,  organic matter  content,  soil fertility, soil reaction, soil color and stoniness. These properties of soils are highly important parameters in watershed development planning.  The decisions taken during the planning process ought, therefore, to reflect at least a basic level of knowledge about the soil conditions.

    - Soil Texture

The soil textural classes of the watershed showed negligible variability within the horizons. Soil texture in both the surface and subsurface horizons is clay to clay loam. The soil of the study area is fine in texture. The soil textural classes are used to characterize soil physical make-up, having a bearing on such soil behaviors as nutrient and water holding capacity, OM level and decomposition, aeration, infiltration rate, drainage and/or permeability and workability (Brady and Weil, 2002).

    - Soil Color

The soil color of the watershed is dominated by radish brown (2.5YR 4/3) when dry to dark radish brown (2.5YR 3/2) when moist on their surface horizons. The possible variation in soil color appears associated more with the difference in oxidation-reduction processes that have taken place within the soils as well as the OM contents.

However there is no significant variation in soil color pattern showed among surface horizon and the subsurface horizon of the Profiles described at Birbirsa small scale irrigation site.

    - Soil Structure

Soil structure refers to the natural organization of soil particles into discrete soil units (aggregates or peds) that result from pedogenic processes. The aggregates are separated from each other by pores or voids. It is preferred to describe the structure when the soil is dry or slightly moist. Soil structure is described in terms of grade, size and type of aggregates (FAO, 2006).

There are no variations in soil structure with depth and among profiles described at Birbirsa irrigation site. Therefore, the structure of all layers of the profile characterized as moderate, medium and Angular and sub-Angular blocky structure.

    - Soil Consistency

Similar to the above-described morphological properties, soil consistency within horizons also didn’t showed variability under moist and wet conditions. Thus the consistency of this soil unit is hard when dry, firm when moist, sticky and plastic during wet condition.

  - Major Soil Types in the watershed

Based on the procedures and methods of soil classification, the major soil type was identified in the study area.

    - Fluvisols

Fluvisols are soils developed from recent alluvial deposits having no diagnostic horizons in the study area it develop on high volcanic piedmonts land form genesis. The essential defining characteristic of Fluvisols is that they occur on recent alluvial deposits. As such their properties are largely controlled by deposition mechanisms and the origin of the alluvium rather than by pedogenetic processes.

Fluvisols are recent alluvial deposits in river plains, former lakes and coastal regions with very little soil development. Most soils consist of stratified layers with different textures, because these layers were sediment deposited by water. The sediments consist of eroded material from uplands and mountains.  In  the  project  area,  Fluvisols  are  found  stratified  over  the  upper middle and vicinity of watershed boundaries at  the  foot of hills and in valley bottom.

The major limitation to the agricultural use of Fluvisols is the frequent requirement for drainage, which is difficult due to an often-high water table.  Fluvisols have a number of advantages for agriculture. They occur predominantly on fiat land rather than eroding landforms. There is an associated Source of water, which ensures appropriate levels of soil moisture for at least a part of the year and may provide a continual supply of irrigation water for the whole year, allowing multiple cropping.

Fluvisols accommodate generally young, azonal soils in alluvial deposits. The name fluvisols may be misleading in the sense that these soils are not confined only to river sediments (Latin fluvius, river); they also occur in lacustrine and marine deposits.

Profile Development: Profiles with evidence of stratification weak horizon differentiation but a distinct topsoil horizon present.

### Mollic Cambisol

Most Cambisols are soils with beginning horizon differentiation, they are in transitional stage of development,from young soil to a mature soil with an argic B horizon ,thus cambisols are soils with initial stage of soil formation or weakly developed soils relating to their parent material. They occur wherever conditions are not favorable for other soil processes than weathering to take place.

Cambisols developed on medium and fine textured material derived from a wide range of rocks. Cambisols in the study area occur on undulating and rolling slopes. As these soils are chemically rich, in the study area they are intensively cultivated and also these soils are mainly under natura1 vegetation. As these soils often occur on rolling slope is the major limitation for agricultural activities, they are shallow and stony. Intensive cultivation, aggravates erosion . In flatter positions they have a rather high agricultural potential, as these soils are chemically rich soils.

    - Humic- Chromic Acrisol

Acrisols are soils that have higher clay content in the subsoil than in the top soil as a result of pedogenetic processes leading to an argic subsoil horizon. Acrisols have in certain depths a low base saturation and low activity clay. Many Acrisol are correlate with red yellow podzolic soil.

Parent material: On a wide variety of parent materials, most extensive from weathering of acid rocks and notably in strongly weathered clays that are undergoing further degradation.

They are old tropical soils of regions with a dry and wet season. Acrisols are generally developed from acid parent material. The content of weather able minerals is in most cases low. Due to leaching over a very long period, the soils are exhausted. The surface soil is low in its CEC. The subsurface soil may be toxic from Aluminum, Manganese or Iron.  These  soils  are  chemically  poor  and physically,  the  clay-enriched  B-horizon may  limit  root  distribution  and  structurally may  be poor. They are used for growing crops, however yields are rather low. Orthic Acrisols can be improved by adding some lime and fertilizers.  The best use of Acrisols is for forest or grazing lands. Many Acrisols are highly susceptible to erosion.

Environment: Mostly old land surfaces with hilly or undulating topography in regions with a wet tropical, subtropical and warm temperate climate. Forest is the natural vegetation type.

Profile Development: Pedogenetic differentiation of clay content with a lower content in the top soil and higher content in the subsoil; leaching of base cations owing to the humid environment and advanced degree of weathering.

Humic:- having 1.4 to 2 percent organic carbon content in the fine earth fraction as a weighted average to a depth of 100 cm from the mineral soil surface.

  - Land capability classification

Land  capability  classification  refers  to  a  systematic  arrangement  of  different  kinds  of  land according  to  those  properties  that  determine  the  ability  of  the  land  to  produce  on  virtually permanent basis. It is the basis of watershed management and development programs. The basic  principle  of  soil  and  water  conservation  is  to  use  the  land  according  to  its  needs. Therefore,  classifying  the  land  according  to  its  capability  is  a prerequisite  and  important for planning, implementation and execution of soil and water conservation programs.

The  land  capability  classes  for  Birbirsa  watershed  have  been  identified  based  on  the assessment of the major limiting factors (directly or indirectly) to use the land in a sustainable manner  and  the  obtained  class  units  are  primarily  categorized  as  soil  conservation requirement classes.   The major limiting factors considered for this purpose are one or two of  land  features  including Slope, Soil depth, Past erosion, Soil  infiltration, Water-logging, Top soil texture and Stoniness or Rockiness. The assessment of each feature was made for each type of present land uses. Determination  of  the  soil  conservation  requirement  class  is made  following  the  procedures adopted for Ethiopian conditions by Escobedo (1988).

The results of the land capability or soil conservation requirement classification show that the major limiting factor of the land use in the watershed is slope.   That means, with the existing condition even though the area receives high amount of rainfall, if the vegetal cover of the land is good  and  soil  is deep,  the  soil  erosion  is  affected by  slope only.  On the other hand, soil erosion in many places is slight owing to the fact that a thick vegetation cover neglects the effect of all other factors such as climate, topography, soil and others. A bare  land on  flat land  is  more  susceptible  to  erosion  than  sloppy  land  covered  with  thick  vegetation.

Figure 6 : Birbirsa Watershed by Slope Class

  - Experiences in Soil and water conservation activities

Experiences in implementation of soil and water conservation activities are very deprived, both at district level and the watershed. However, there are  some measures  in  steep  cultivated  lands  and  where  vegetation  cover  is affected.  Some  of implemented measures  include  physical measures  such  as  terraces,  soil  bunds,  check  dams, cut-off drains and biological measures  like plantation of  animal feed plants and grasses across the slope to halt run-off and inter cropping. Biological measures are not also given much attention. It could be understood from the findings of discussions made with the community and district level experts that some efforts have been made for promoting agro-forestry plantations around the homestead. But  the  success  of  these  activities  is  constrained  by  reluctance  of  the farmers  to  establish  the  systems  owing  to  the  non-availability  of  adequate  tree  products  for different uses.  The major bottle-necks to the implementation of soil and water conservation in the watershed of  the project area are:  low emphasis given  to natural  resources conservation  in general and soil and water conservation in particular by concerned bodies in such potential area; poor awareness of the community on land degradation; existence of other alternative sources of income for the community other than crop production since the watershed has potential coffee growing areas; lack of training on the subject for the experts who are working in the field and the farmers; lack of  budget  for  the  sector;  and  reluctance  of  the  farmers  to  implement  the  measures  where erosion  is found to happen.

# SOCIO-ECONOMIC CONDITIONS

In  watershed  management,  socio-economic  information  is  important  and  survey  have been carried out to get good understanding of the actual situation in the watershed and for identification of the major problems, possible  solutions  should be given which  are  asked  for or  at  least acceptable by the local community. Survey results of socio-economic situation of the watershed mainly  help  to  prioritize  the  main  problems  of  the  community  in  proposing  development interventions  and  choose  acceptable  and  technically  sound  measures.  Sometimes  the  top priority  of  the  community  may  be  beyond  the  objectives  of  the  project  in  a  project-type development plan. In such cases integrated and multi-disciplinary approach would be a good solution. A survey of socio-economic conditions can give clear picture of the social set up of the community,  major  economic  occupation  and  status  also  helps  to  know  whether  there  is adequate  number  of  working  and  able  bodied  group  of  people  to  carry  out  the implementation of development plan of interventions.

  - Demographic Characteristics

Statistical data that reflect the socio-economic and demographic conditions of the residents of the watershed are useful for designing and preparation of development plans as well as for monitoring and evaluation of the impact of the implementation of the development plans. These statistical data include population size, age, sex, literacy and education, marital status, housing stocks and conditions …etc.

Population growth rate refers to the average annual percent change in the population, resulting from a surplus (or deficit) of births over deaths and the balance of migrants entering and leaving an area. The rate may be positive or negative. The growth rate is a factor in determining how great a burden would be imposed on an area by the changing needs of its people for infrastructure (e.g., schools, hospitals, housing, roads), resources (e.g., food, water, electricity), and jobs.

The population and other relevant demographic issues are analysed based on the field level data result in line with the results of 2007 population and housing census results. Accordingly the total population in the Kebele’s of the watershed was 5,940 (3,029 male and 2,911 female) Population projections can be used for a number of purposes. Perhaps the most important use of population projections is in the role they can play as a rational basis for decision-making. Changes in population size and composition have many social, economic, environmental, and political implications. For this reason, population projections often serve as a basis for producing other projections (e.g., births, households, families, school enrollment, and labor force).

## Existing Economic Activities in the Project Area

The main economic activities and source of livelihood of community in the project area is agriculture, i.e., production of crops and rearing of livestock, which is a mixed-farming system.

### Crop Production

Different crops are produced in Guutoo-Giddaa District (Woreda) and the intended project area under both rain-fed and irrigation methods. The major crops in the district are cereals (teff, maize and millet), oil crops (Niger seed), and vegetables (potato and carrot).  Crop production is currently carried out both under rain-fed and Bonee traditional production methods in the proposed irrigation project area. The project command area is located in Nagaasaa Kebele the left bank of Lega-Birbirsaa (Hanquu, as it is locally known) River, respectively. Crops produced under rain-fed method include Teff, maize, millet and potato while maize, potato and carrot are produced using Bonee method.

Generally, crop production in the project area is merely at household-subsistence levels; and a little or only some part of it is taken to local markets, as residents tell.

### Livestock Production

As indicated earlier the economic activity in the area is a mixed farming system.  Next to crop production, livestock and livestock products are the other source of livelihood of the community as source of food, alternative means of income and means of transport in the area.  The main types of livestock rearing in the area include cattle, sheep, goat, donkey, mule, horse, and poultry. Apiculture is also practiced to some extent.

    - Infrastructure & Social Services

As the project area has relative proximity to the zone capital, there are better communication infrastructure facilities and social services thereon.

    - Social Services

The most common social services available in the area comprise schools, health institutions, farmers’ training centers (FTC), multipurpose cooperative service etc.

    - Infrastructure Conditions

According to data obtained from Guto-Gidda District Road Authority, as of 2009 E.C, there was 186.11km (18km dry-weather, 167.11km gravel and 1km asphalt) road in the district. The rural road access coverage is about 1.55%; and with annual increment of 1.2%, 1.4%, and 1.47% in 2006, 2007and 2008 E.C respectively.

The asphalt road from Finfinnee to the western parts of the country :- (Gambella-Mattu-Baddelle and/or Gambella-Dembi-dollo-Gimbi and Asosa; and the asphalt pavement highway from Finfinnee via Naqamte to the western parts of the country (Gambella-Mattu-Baddelle and/or Gambella-Dembi-dollo-Gimbi and Asosa-Gimbi-Naqamte) paths through the proposed project area. The Naqamte-Bure highway is being upgraded to asphalt pavement. The construction of an airport has also been started around Ukke, which is a promising opportunity for future development of the region including the project area.

The project area is inaccessible from Nekemte, which create difficulty for motorized transport during the wet or rainy seasons.

    - Market Conditions

In reference to market demand, Nekemte City is experiencing an accelerated-population growth and spatial expansion continued at an even-greater speed due to combined effects of various forces. Now Nekemte houses Wollega University, several colleges, banks, insurance companies, numerous health facilities and large contingent of new business establishments. All these institutions house a significant number of consumers. Nekemte is also serving as a convergence point and gate way for Bahir-Dar, Amhara regional state, Asossa of Benishangul regional state, Gambella of Gambella regional state as well as Metu and Jimma towns.

  - Development Opportunity & Constraints

    - Constraints

The major limiting factors to the development of the district are broadly categorized into economic, social, and environmental influences. The existing economic constraints include issues like shortage of farmland, grazing land, and agricultural input supply. Social factors include issues like lack of transportation, prevalence of disease, unemployment, and lack of education service. Environmental constraints include factors like deforestation, soil erosion, soil fertility and variability, amount and uneven distribution of rainfall affecting agricultural productivity.

The situation of the project area and Kebele is likewise equally affected by the combination of all these factors.

    - Potentialities

The district has immense natural potential for development. Among the different natural resource endowment and potentialities of the district regardless of their variability in space and quality, the most vivid ones are:

- Agricultural resources like cultivable land, irrigable land, livestock rearing, mining, fishing,

- Available human power,

- Natural resource like water, minerals and forest,

- Availability of local market for crop and livestock products

- Natural, cultural, and historical tourist attraction sites.

All these resources and potentials found in the district to be used for different purposes and investment activities can be considered as opportunity for development, too.

# MAJOR PROBLEMS OF WATERSHED

The  descriptions  of  watershed  given  in  previous  sections  show  the  existing  potentials  and constraints of watershed. There are quite a number of problems of watershed, which may offer challenge to the development and resource base.  The  problems  identified  are  based  on  the results  of  technical  analysis  of  bio-physical  and  socio-economic  information  of  the watershed and results of discussions made with  the community and district level executives and experts.  The major problems can be grouped as land degradation in the form of deforestation and soil erosion and socio-economic problems. The detail is given in the succeeding sections.

## Soil Erosion

Physical degradation of soil is the most remarkable in the form of sheet, rill and gully erosion. The most common forms of erosion observed in the watershed are splash, sheet, rill and gully erosion on cultivated lands.  Soil  erosion  is  spectacular  particularly  during  the  early  times  of  land preparation and planting.  It is aggravated by traditional tillage methods that are practiced in up-down direction along the slope and improperly laid and developed drainage furrows aimed at removing excess run-off. Moreover, plowing of the land at shallow depths is the practice that is adopted by the farmers. Shallow tillage increases formation of hard pans at shallow depths. These hard  pans hamper  or  retard  the  infiltration  and  deep percolation  of  rain  water  into  the  soil,  consequently  increases  run-off.  Also shallow tillage would not create adequate room for water accumulation in the soil layers and profiles.     Farm drainage ditches  are  developed  by  individual  farmers  at  steep  gradients  (greater  than  8 per cent) in order to remove excess rain water at faster rate. Such grades of drainage ditches cause generation  of  erosive  velocity  of  run-off  that  can  easily  erode  and  transport  soil.  Since  such drainage  ditches  are  developed  in  all  types  of  slopes,  including  nearly  flat  lands,  erosion  is spectacular in all types of lands used for cultivation of annual crops. Free grazing is found to be another reason for persistence of accelerated erosion. Furthermore, cultivation is practiced near to the edge of the stream banks and there is no any possibility of de-silting and filtering the runoff entering the rivers or waterways. Similarly the water led to the river banks falls with certain energy, and consequently causes slide or mass movement of soil from the river banks.

## Land Degradation

Land  degradation  is  a  process  that  lowers  the  current  and/or  potential  capability  of  land  to produce goods.  Continued deterioration  of  land  may  lead  to  a  permanent  decrease  of  its  biological  potential  and  a deterioration of living conditions for inhabitants. The causes of these processes are invariably a combination  of natural phenomena  among man’s  actions  such  as  the  destruction  of  vegetative cover,  overgrazing  and  inappropriate  agricultural practices  that  are not  in harmony with  the ecological environment. It is man’s actions, often as a result of increasing population pressure, that extend and accelerate the processes of degradation. Water erosion includes sheet, rill, gully erosion and mass movements; salinization and alkalization refer to the accumulation of excess salts and  sodium, and chemical degradation  to  the  leaching and  removal of nutrients and  the buildup of  toxicities other  than  those due  to excess  salts. Physical degradation  includes  those processes such as poor cultivation practices, which adversely affect soil physical properties such as  infiltration  rate,  structural  stability,  root  penetrability,  and  permeability.  Biological  degradation  refers  to  processes,  which  accelerate  humus mineralization rates, and largely reflects the moisture/temperature regimes of the environment and  land  use  practices.  Other  than  salinization  and  alkalization  processes,  all  the  rest  are exhibited  in the project area.

## Deforestation

One  of  the  forms  of  land  degradation  is  removal  of  vegetation  cover.  Deforestation of the existing forest areas by the people who are seeking cultivated land and construction of new housing units are seen to be one of the reasons for degradation. During discussion held with the community, it has been observed that such activities are becoming the permanent practices. It is attributed to the need of having new productive land and increased population in the rural areas aspiring to hold land.

Moreover, residents who are close to forest area practice tree cutting and incineration. Another important reason for forest clearing is related to the need to produce charcoal and collect firewood for sale and home consumption. Cutting of trees for  collecting  construction  material  is  also  another  reason  for  deforestation  owing  to  the availability  of  these  resources  in  forest  areas. Wood  land  clearing  for  agriculture  involves  a complete  change  in  land  cover  from  shrub  land, woodland  or  forest  to  “non-forest  land”  or bare-land  and  almost  complete  removal  of  wood  in  the  cleared  area  has  been  the  case. However, wood removed for fuel does not involve a complete and instant change in land cover.  Situation  can  also  be  correlated  with  livestock  population  and  size  of  available  areas  for grazing. The grazing in the watershed varies in quality and quantity, with the livestock species, each utilizing different portions of the natural vegetation. To determine the overall demands by livestock on the range it is necessary to relate their varying requirements to a common standard, which is stocking rate of the area. The existing sizes of grazing lands are not adequate to carry the livestock population exist in the watershed.  The gap of feed is found to be filled by feeding the livestock with plant residues and browsing every possible left-over after crop harvest. The implication of free movement  of  livestock  is  that  animals  trampling  accelerate  land  degradation  through discriminate  browsing  of  vegetation  and  disturbance  to  soil  structure  and  established  soil conservation measures.

## Shortage of Grazing Land

The main challenge in this watershed is the inadequate supply of livestock feed as compared to the livestock population.  The  rate  of  stocking  of  grazing  lands  is  too  high,  as  discussed  in previous  section. The existing grazing lands are not to the expected standard. The number of livestock that is allowed to graze a given grazing land should be decided based on the carrying capacity of the grass lands. Grazing  capacity  refers  to  the  ability  of  a  grassland  unit  to  give adequate  support  to  a  constant  number  of  livestock  for  a  stated  period  each  year  without deteriorating with respect to this and/or other proper land use. Grazing by domestic livestock must be considered as one of the factors of the environment affecting the stability of grassland vegetation and soil in a watershed. The effects of grazing will depend largely on the degree to which the forage is utilized. The quantity of herbage left after grazing will govern the growth and development of forage plant roots.

- INTEGRATED WATERSHED DEVELOPMENT INTERVENTIONS

  - SOIL AND WATER CONSERVATION

The  Soil  and  Water  Conservation  techniques  proposed  for  implementation  have  potential  in  controlling  land  degradation  and  the  loss  of  land  productivity  on  one hand  and controlling  sediment  load  problem  in  the  being  implemented  irrigation  structures  and  other downstream socio-economic structures on the other.

The  factors  that have  been  taken  into  consideration  in  the  selection  of  the proposed  soil  and water conservation techniques/measures are:

- Existing practices and experiences of the community

- agro-ecologic condition of the area

- Proven capability of the chosen  interventions to control soil erosion and avoid  land degradation

- Feasibility of  the  technologies and  their application and adaptation  in  the  land use and farming system

The  overall  impact  expected  in  reducing  flood  and  sediment  deposition  on  the downstream socio-economic infrastructures and proposed irrigation structures.

    - Physical Conservation Measures

Physical measures are those soil and water conservation measures, which lead to change in land surface arrangement and profile. These measures include such as bunds that divide a slope into smaller and separate segments and measures such as bench terraces that break the steep slope of land into benches of table-shaped bottom. They involve the purposive movement of soil or stones but still may arise naturally from planting of grass strips. Usually such measures are not complete and fully effective and permanent alone. However, where  necessary  they must come  first  and  followed  by  other  biological  measures.  Physical measures are effective for retarding runoff velocity, and consequently store sediment that is carried by runoff and store or safely dispose excess runoff water.

There are various physical measures that can be applied on different land uses particularly on cultivated and grazing lands. The following measures are proposed for the project area based on investigated major characteristics of the watershed.

      - Soil Bunds

Soil bunds are earth banks thrown across the slope to act as a barrier to runoff, to form water storage area on their upslope side and break up a slope into segments shorter in length than is required to generate overland flow.  They  are  suitable  on  slopes  between  3  to  15  percent  on cultivated lands. Soil bunds are graded type for removing excess rainfall from the fields at safe velocity. The  project  area  receives high  rainfall  and  characterized  by  relatively  better  vegetal cover,  thus  graded  bunds  is  recommended.  The maximum length for graded bund should not exceed 500m in order to avoid risks of breakage and overtopping. The bunds could be stone-faced in areas with high stoniness and stable soils. Planting of grasses and other multi-purpose agro-forestry species should be integrated with such measures.

      - Fanyajuu terrace

Fanjajuu  is  a  type  of  terrace  developed  by  a  channel which  is  dug  and  soil  is  placed  on  the upper side. Such terrace has the advantage of holding or removing water in both sides, upslope side of the embankment and down-slope.  Throwing soil uphill to make a terrace causes reduction in slope, which in itself makes a major contribution to reducing erosion on steep land. The combination of throwing soil uphill coupled with natural process of erosion leads to the development of bench type terrace. Fanyajuu terraces are more effective than bunds because they lead to a reduction in  slope  angle but  their  effectiveness depends  as with  other  structures  on  the  efficiency with which  they  intercept  run-off.  Because  of  the  fact  that  the  effectiveness  of such  intervention depend on  the proper  integration with grass on  the embankments and periodic maintenance, planting  of  suitable  grass  species  should  be  made  immediately  after  construction  and maintenance  should  be  considered  after  every  rainy  season. The measure  is  applicable  in cultivated  and grazing  lands with  5  to  15 percent  ranges  of  slopes.

      - Cutoff Drains

Cut-off drains are graded channel constructed to intercept and divert the surface run-off from higher ground/slopes and protect downstream cultivated land.  They divert the run-off to a

Waterway, rivers and like.  The structures are constructed based on estimated runoff expected to carry, particularly at the foot of steep hillsides and uncultivated lands without treatment under which cultivated fields are exposed.  In addition, they are constructed above gully head and help to divert run-off from active gullies to treated/stable ones.  The amount of cut-off drains required depends on the actual conditions in the field and usually decided at the time of the implementation. In spite of this, they could be estimated as 10 to 20% of the quantity of soil bunds. It is important to link cut-off drains to waterways (artificial/natural) to safely divert run-off and at the same time construct scour checks with stones and grasses at a width of 50 cm for 10 – 20 meters before the outlet to a waterway.  Construction  of  drop  structures,  as  required  to  minimize  the  effect  of  water dropping into the waterway or gully is vital in avoiding the risk of creating additional erosion at the outlet level. Cut-off drains are proposed to be implemented in all types of capability class units.

      - Check Dams

Check  dams  are  gully  plugging  structures  using  rocks/stones,  compacted  soil  or  selected

material,  earth  bank, wood  racks,  gabions  and  also  planting  trees  and  grass  to  stabilize  the system.  Each material has different sediment trapping and flood protecting characteristic.

Check dams can be easily applied in all gullies of less than 2m depth and 5m width. Vertical intervals between check dams are equal to the height of check dams. Such check dams must be immediately constructed in the gullies.

    - Biological Conservation Measures

      - Strip, Mixed and Intercropping

Strip cropping is a system under which ordinary farm crops are planted in relatively narrow trips, across the slope of land. The strips of erosion-permitting crops are always separated by strips of close-growing or erosion-resisting crops. Mixed cropping is a practice of growing more than one crop in the field simultaneously. In mixed cropping, there is one main crop and one or two subsidiary crops.  This  system  of  practice  is  adopted  by  the  farmers  especially  in  coffee farms  in  the  project  area,  but  they  lack  scientific  knowledge  in managing  the  system. Mixed cropping gives better cover on land, good protection from soil erosion and assurance of one or more crops to the farmers, ensuring diversification to cope with risks. The roots of various crop species in mixed cropping make use of soil moisture and nutrients from different depths in the soil. Suitable crops for mixed cropping in the project area are maize with chickpea, sorghum chickpea and others as the case may be. Moreover, intercropping of leguminous and other plants with coffee should be considered. On the other hand, intercropping provides similar effect as is done by mixed cropping except that in intercropping sequencing is possible.

      - Grass Strips

Grass  strips  are  vegetative  barriers made  out  of  grass  planted  in  narrow  strips  of  0.5  to  1.5 meters  width  laid  along  the  contour  or  along  the  physical  conservation  measures.  While contributing  to  protect  soils  against  erosion  they  also  provide  valuable  biomass  meant  to increase animal feed or used for different purposes. Grass strips can be used alone on slopes 3 to 8 percent and integrated with physical measures alternatively in 8 to 15 percent slopes. They can be integrated with lines of legume shrubs such as pigeon peas, Sesbania, Tree-Lucerne and Acacia saligna planted in dense rows.  Advantage of grass strips is that they cause less interference than other measures as they can easily be crossed by oxen and plough. Moreover, take little amount of arable land.  Cost of construction is much lower than physical structures.  Cultivated land of slopes 3 to8 percent is proposed to be treated with grass strips. Moreover, it is required to establish grass strips on all of the soil bunds as proposed earlier. In addition to the advantage of controlling erosion and improvement of soil fertility, the production of grass from this area will help to generate livestock feed through cut and carry system.

      - Area Closure

Area  closure  is  a  protection  system  and  saving  mechanism  of  land  against  any  external influences including erosion. Slopes above 40% are recommended to be closed from day to day

human  activities  and  livestock  grazing  to  give  a way  for  natural  trees  and  grass  vegetation regeneration.  This would provide fodder for livestock as well as firewood and construction material for the inhabitants of the area. Grasses grown in these areas should be cut and carried for stall feeding. Cut and carry can be allowed after the tree and/or grass cover of over 80% is obtained  or  if  undesirable  herbs  and  shrubs  are  required  to  be  removed.  No  livestock  is therefore  allowed  to  graze  in  the  closed  area  and,  also  there will  be  no  human  interference except bee husbandry in the closure. It is advisable to cut the grass once immediately after the rainy season. Forage should be cut at about 10cm above the ground before or at the flowering stage, but only if sufficient soil coverage is assured.  Forage can also be conserved as hay or silage for dry season feeding. Cut and carry allows permanent vegetation cover and maximum soil protection.

## INTERVENTIONS IMPLEMENTATION SCHEDULES

The major activities of the interventions along with their quantity and time schedules of implementation are presented below.

Table 3 : Interventions Implementation Plan

| No. | Activities | Unit | Total Plan | Distributions |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Yr. 1 | Yr. 2 | Yr. 3 | Yr. 4 | Yr. 5 |
| 1 | Physical SWC Measure |  |  |  |  |  |  |  |
| 1.1 | Soil Band | KM | 764 | 156 | 152 | 152 | 152 | 152 |
| 1.2 | Faynaju Terrace | “ | 458 | 94 | 91 | 91 | 91 | 91 |
| 1.3 | Cutoff Drain | “ | 336 | 68 | 67 | 67 | 67 | 67 |
| 1.4 | Check Dam | “ | 189 | 100 | 89 | - | - | - |
| 2 | Biological SWC Measure |  |  |  |  |  |  |  |
| 2.1 | Plantation on degraded land and hill sides | Ha | 13 | 11 | 10 | 10 | 10 | - |
| 2.2 | Plantation on physical SWC structures | No. | 7,639 | 1,531 | 1,527 | 1,527 | 1,527 | 1,527 |
| 2.3 | Area Closure | Ha | 17 | 5 | 5 | 5 | 2 | - |
| 2.4 | Grass Strips | KM | 519 | 107 | 103 | 103 | 103 | 103 |
| 3 | Seedling Production | No. |  |  |  |  |  |  |
| 3.1 | Nursery Site Establishment | No. |  |  |  |  |  |  |
|  | Public | “ | 2 | 2 | - | - | - | - |
|  | Private | “ | 11 | 11 | - | - | - | - |
| 3.2 | Seedling Production | No. |  |  |  |  |  |  |
|  | Public | “ | 1,000,000 | 200,000 | 200,000 | 200,000 | 200,000 | 200,000 |
|  | Private | “ | 440,000 | 88,000 | 88,000 | 88,000 | 88,000 | 88,000 |

Table 4 : Summery of Budget for Conservation Measures

| No. | Cost Description | Planed Budget | Distribution |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Yr. 1 | Yr. 2 | Yr. 3 | Yr. 4 | Yr. 5 |  |
| 1 | Material | 137,304 | 27,464 | 27,460 | 27,460 | 27,460 | 27,460 |  |
| 2 | Direct Labor Cost | 48,847 | 9,771 | 9,769 | 9,769 | 9,769 | 9,769 |  |
| 3 | Capacity Building | 7825 | 1,565 | 1,565 | 1,565 | 1,565 | 1,565 |  |
| 4 | Administrative Cost | 11,229 | 2,249 | 2,245 | 2,245 | 2,245 | 2,245 |  |
| 5 | Total | 205,205 | 41,049 | 41,039 | 41,039 | 41,039 | 41,039 |

# CONCLUSION AND RECOMMENDATION

The watershed covers about 4,400 hectare, including the command area. Out of the total watershed area 87percent is cultivated, 8per cent is grazing and 2per cent is under forest or bush land cover and 3percent is for settlement and other uses. The watershed has slope 0 to 15 percent. A study conducted on the biophysical and socio-economic conditions of the watershed shows that land  degradation  in  the  form  of  soil  erosion,  shortage  of  livestock  feed  as  compared  to  the number of livestock, lack of community awareness in natural resources conservation and other socio-economic problems are enormous. Poor farming systems, lack of interventions to halt soil erosion, deforestation and others is remarkable.

The problem of soil erosion and the subsequent sedimentation in Birbirsa watershed is a threat and devastating challenge to the structures of the proposed irrigation project in the watershed and downstream areas due to generating high run-off discharges and imposing huge sediment yields,  which  certainly  result  in  reducing  the  life  span  and  water  storage  capacity  of  the reservoir,  unless  the  upper  watershed  is  treated  with  appropriate  watershed  management interventions and strategies. Proposing appropriate and area specific development interventions are very crucial in order to break soil erosion problem in the watershed.  The land productivity of the watershed is the primary objective of this document.  Considering  the  direct  correlation between  poverty  and  natural  resource  degradation,  proposing  livelihood  interventions  that enhance the economic and social well-being of the community is also the other major objective.

The proposed interventions include physical and biological soil and water conservation measures and livelihood components. These interventions are proposed to be implemented over 10 year period. Along with these  interventions, strategies for  implementation and  institutions responsible  for  carrying  out  the  implementation  of  the  proposed  interventions  have  been    indicated.

Actually the vegetal cover of the watershed is relatively better  and  soil  erosion  is minimal  in many  parts of the watershed  except  in  areas  where forest clearing is taking place particularly at cultivated  sloppy areas. On the other hand, lack of awareness in soil and water conservation practices widely observed in the watershed and not implemented properly so far. Based on this awareness creation activities should be undertaken within the relevant communities in particular. The consultation and participation of the relevant communities regarding their future outcome is also another important measure to be taken.

# REFERENCES

Food and Agriculture Organization of the United Nations (FAO), 1976. A framework for land evaluation. FAO Soils Bulletin 32, FAO, Rome

Food and Agricultural Organization of the United Nations (FAO), 1979. Yield response to water. FAO irrigation and drainage paper 33, FAO, Rome.

Food and Agricultural Organization of the United Nations (FAO), 1983. Guidelines: Land evaluation for rain fed agriculture. Soils Bulletin 52, Rome.

FAO (Food and Agriculture Organization), 2006. Guidelines for soil description (Fourth Edition). FAO, Rome.

FAO/WRB (Food and Agriculture Organization/World Reference Base), 2006. A framework for international classification, correlation and communication. World Soil Resources Reports 103. International Union of Soil Sciences, FAO, Rome.

Food and Agriculture Organization of the United Nations (FAO), 2007, Land evaluation towards a revised framework, Land and water discussion paper 6. FAO, Rome.

Hesse, P.R., 1972. A textbook of soil chemical analysis. John Murry Limited., London, Great Britain. 470p.

Landon J.R., 1991. Booker tropical soil manual, Hand book for soil survey and agricultural land evaluation in the tropics and sub tropics.

Sys I., C., Van Ranst, E and Debaveye, I.J., 1991a.Land evaluations part I: Principles in land evaluation and crop production calculations. Agricultural Pub. No.7, ITC, University of Ghent, Ghent.

Tekalign Tadese, 1991. Soil, plant, water, fertilizer, animal manure and compost analysis. Working Document No. 13. International Livestock Research Center for Africa, Addis Ababa, Ethiopia.

- ANNEXS

Annex 1 : Structured Questionnaire for Individual Farmers

QUESTIONNAIRE FOR SOIL EROSION HAZARD AND LAND ASSESSMENT

Region			Geographic coordinate: E		N:		Elevation:

Woreda		Author:	   Land use:	                        Farming system	:

1.Is soil erosion a problem in your area?

- Yes

- No

2.	Have you experienced soil erosion problem in your land?

- Yes

- No

3.	If yes what are the causes of soil erosion?

Poor farming practices

Up and down slope farming

Lack of erosion control practices

Intense rains

Overgrazing

Livestock track and human paths

4. What are the effects of soil erosion?

Fertility decline

Low crop productivity

Loss of water-

Poor water quality

Poor soil qualities

5.Have you ever practiced soil conservation measures of your own?

Yes

No

6.If yes do you think soil conservation measures helped in controlling erosion?

Yes

No

7.If yes, what are the traditional soil management measures you have practiced?

Manure application

Agro forestry practices

Stone walls

Hedge rows and boundary plantations

Contour planting

Others specify

8.Do you practice shifting cultivation or slash and burn practices for growing cereal

crops?

Yes

No

9.If yes for how many years do you use the land for cultivation?

Only one year

Two years

More than two years

10.Do you practice fallowing for soil fertility improvement?

Yes

No

11.If yes what is the size of your land holding?

Less than one hectare

2 hectares

More than two hectares

12.Do you practice intercropping?

Yes

No

13.If yes, which crops do you intercrop?

Cereals and pulses

Cereals and cereals

Cereals and perennials

14.What are the trends in crop production when you compare the current conditions with

that of the recent past?

It is declining

No change

Is increasing

15. If it is increasing what is the reason?

improved cultural practices

use of chemical fertilizers

well distributed and sufficient rains

soil conservation activities

16. If it is declining the reasons are:

frequent ploughing and aging of plots

declining soil fertility

soil erosion

weed and insects

lack of chemical fertilizers

all

17.Is deforestation a problem in the area?

Yes

No

18. If yes, what are the reasons for deforestation?

Shortage of cultivated lands owing to increasing population

Grazing land shortage

Honey extraction

Forest fires

Mineral mining

Charcoal making

19.What are the consequences of deforestation?

Loss of springs / water

Soil erosion

Shortage of fuel and construction wood

Declining soil fertility

Poverty

20.Do you plant trees?

Yes

No

21.If yes which trees do you prefer to plant?

Eucalyptus

Indigenous trees please specify

Cypresses

Grevillea

22. Where do you get the seedlings?

raised in own nurseries

bought from market

provided from government nurseries free

bought from government nurseries

collect them from forests

23.Is there problem with fertility in your land?

Yes

No

24.If yes how did you try to overcome this?

using compost

using animal manure

using household residues and kitchen ash

using chemical fertilizers

None

25. Which do you prefer?

Industrial fertilizers

Animal dung

Compost

26.Are you trained in any agricultural practices?

Yes

No

27. If yes, which?

Soil conservation

Crop production

Forest management

Bee keeping

Livestock husband

Crop protection

Compost making

Water harvesting

28. Do you have water harvesting structures built in your homestead?

Yes

No

29. If yes, which type?

Underground tank

Trapezoidal cistern

Pond

30.For what purpose do you use the water harvesting structure?

Growing horticultural crops

Domestic consumption

Livestock watering

All

31.Are there introduced conservation measures in your area?

Yes

No

32.If yes, which types?

Terraces, bunds etc

Vegetative measures

Others specify

Annex 2 : Semi Structured Questionnaire for Group Discussions in the Community

1.  What are problems encountered with land in this community? (low land holding, 	shortage of grazing land, soil erosion, land aging, loss of fertility, lack of trees, 	shortage of water,  others specify) _________________________________________

2.  Is soil erosion a problem in the area?

yes

no

3.  If yes, does soil erosion affect crop production?

Yes

No

4.  If yes how? Explain

________________________________________________________________________

5.  What measures help in controlling soil erosion? Explain

_________________________________________________________________________

6.  Are there introduced conservation practices in the community?

Yes

No

7.  If yes, which are these?

soil bund, stone bund, fanya juu

hedge rows, grass strips

Disposal systems (diversion ditches, waterways and cut off drains

Improved agro forestry practices

All

8.  How is the implementation carried out?  (Mass mobilization, individual

approach, participatory plans (_____________________)

9.  What are the causes for soil erosion (rill and gully erosion)?

Overgrazing

clearing forest for cultivation

Forest fires

Cultivating steep slopes

Lack of conservation activities

Others specify

10.   What is the trend in crop production at present when compared to the past 10 
	years? (Is it declining, is the same no change, it is increasing)

______________________________________________________________________

11.   Give reasons for each

_______________________________________________________________________________

12.   Do women participate in soil conservation activities?

Yes

No

13.   Are women involved in decision making at community activities?

Yes

No

14.   Do you have area enclosures in the community?

Yes

No

15.   If yes, who initiated the idea?(Community, government workers, woreda officials, KA leaders)

16.   What are the benefits you have obtained from area enclosures? (Grass, wood, recharge of springs, wildlife increase in number)

17.   Is deforestation a problem in the area? What are the causes? What are measures taken to control it?

18.   Is there problem of fuel and construction wood?

Yes

No

19.   If yes, how do you think the problem will be solved?

20.   Do community members practice traditional agro forestry practices?

Yes all

Yes some

No

21.   What are the major tree and crop species used for the agro forestry practices?

22.   Which types of land degradation problem are prevalent in the area? (Deforestation, soil erosion, low fertility, salinity, acidity)

23.   What are the major causes for soil erosion in your area?

_________________________________________________

24.   Is fallowing practiced as way of improving soil fertility in the area?

Yes

No

25.   Explain why no or why yes?

______________________________________

26.What area the other ways of improving land productivity in the area (use of animal dung, compost, use of chemical fertilizers)?

_______________________________________________________

27.   Is gully erosion a problem?

Yes

No

28.   If yes explain the causes and effects?

________________________________________________

Annex 3 : Checklist Questionnaire for Data Collection

Checklist Questionnaire for District Concerned Sectors

This information are required for watershed management study for the sustainability of the proposed irrigation projects in your district through enhancing productivity of the land and ensuring the safety of irrigation infrastructures. Therefore, we expect genuine information and accurate data that can help to carryout fruitful investigation.   So, we kindly ask you to read carefully and give suitable answers for each question below.

1= Land use pattern of the catchments area

| Land use type | Area coverage of the total % | Area in ha | Slope in % |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 0 -3 | 3 -8 | 8 - 15 | 15 - 30 | 30 - 50 | >50 |
| Cultivated |  |  |  |  |  |  |  |  |
| Grazing land |  |  |  |  |  |  |  |  |
| Forest land |  |  |  |  |  |  |  |  |
| Homestead (Village) |  |  |  |  |  |  |  |  |
| Miscellaneous |  |  |  |  |  |  |  |  |

  - Grazing land: - All land open for year round free grazing, temporary hill side closure.

  - Forest land: - Forest lands and lands that are already assigned (by the community) for forest development.

  - Miscellaneous land: - This includes land like seasonal, perennial river course, gullies, rocks, cliffs, roads sides, etc.

2=Land use classification

| Type of land use | Area in ha | Slope % | Soil depth | Past erosion | Surface soil texture | Infiltration | Surface stoniness |
| --- | --- | --- | --- | --- | --- | --- | --- |

3=Type and extent of conservation measures experienced on each land use type

| Item No | Land use type | Area in ha | Slope % | % Covered | Type of conservation measures | Damage if any | Probable cause of damage |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Cultivated land |  |  |  |  |  |  |
| 2 | Grazing land |  |  |  |  |  |  |
| 3 | Forest land |  |  |  |  |  |  |
| 4 | Homestead (Village) |  |  |  |  |  |  |
| 5 | Miscellaneous |  |  |  |  |  |  |

4=Illustrate map of sub watershed if possible?

Bonee is a traditional production practice, which is carried out immediately after the most recent regular heavy rain season while the soil still remains moist.