OROMIA NATIONAL REGIONAL STATE

OROMIA IRRIGATION AUTHORITY

Wiliyo SSIP Final Feasibility Study & Detail Design Report, Catchment Management

PREPARED BY BONEYA CONSULTING PLC

JUNE 2016

NEKEMTE

Table of Content

1	INTRODUCTION	2

1.1	Background	2

1.2	Objectivesof the Study	2

1.3	Scope of the Study	2

1.4	Review of previous studies	2

1.4.1	Soil Erosion in Ethiopia	2

1.4.2	AbayRiver Basin Master Plan Study	2

1.5	Policy and Proclamation	2

1.5.1	Rural Land Use and Administration Proclamation of the FDRE	2

1.5.2	Rural Development Policy and Strategies	2

1.5.3	Food Security Strategy (2002)	2

1.5.4	New Coalition for Food Security Program	2

1.5.5	Natural Resources and Environment Policy	2

1.5.6	Land Administration and Use, Forest Conservation and Development Policies	2

2	MATERIALS AND METHODS	2

2.1	Materials	2

2.2	Methods	2

3	STUDY AREA	2

3.1	Location	2

3.2	Topography	2

3.3	Agro-Ecology	2

3.4	Land use/Land cover	2

3.4.1	Farmlands	2

3.4.2	Shrub lands	2

3.4.3	Grazing lands	2

3.4.4	Settlements	2

3.4.5	Marsh area	2

3.5	Major Soil types	2

3.5.1	Alisols	2

3.5.2	Luvisols	2

3.6	Soil Erosion and State of Natural Resources Management	2

3.6.1	RUSLE model	2

3.6.2	Major Factors Affecting Erosion	2

3.6.3	Estimating Soil Losses	2

4	SOCIO-ECONOMIC CONDITION	2

4.1	General	2

4.2	Population	2

4.3	Agricultural production	2

4.3.1	Crop Production	2

4.3.2	Livestock Production	2

4.3.3	Apiculture	2

4.3.4	Land Tenure System	2

4.3.5	Rural Energy	2

4.3.6	Gender Issues	2

4.3.7	Infrastructure Development	2

4.3.8	Institutions	2

4.3.8.1	Community based institutions	2

4.3.8.2	Government Institutions	2

5	PROBLEM ANALYSIS	2

5.1	General	2

5.2	Major Problems	2

5.2.1	Soil erosion	2

5.2.2	Deforestation	2

5.2.3	Low livestock production	2

5.2.4	Low diversification of crop production	2

5.2.5	Limited alternative income sources	2

5.2.6	Low Local Capacity	2

5.3	Opportunities	2

5.3.1	Bio-physical opportunities	2

5.3.2	Socio-Economic Opportunities	2

6	PROPOSED WATERSHED MANAGEMENT INTERVENTIONS	2

6.1	Main Strategies	2

6.2	Watershed Management Interventions	2

6.2.1	Soil and Water Conservation Measures	2

6.2.1.1	Physical Soil and Water Conservation Measures	2

6.2.1.2	Biological Soil and Water Conservation Measures	2

6.2.2	Crop Production Management	2

6.2.3	Livestock Development interventions	2

6.2.4	Income Generating Activities	2

6.2.5	Supportive Strategies	2

6.2.6	Local Capacity Building	2

6.3	Planning and Implementation	2

7	IMPLEMENTATION STRATEGIES	2

8	ORGANIZATION & MANAGEMENT	2

9	MONITORING AND EVALUATION	2

9.1	Monitoring	2

9.2	Evaluation	2

10	CONCLUSION	2

11	References	2

List of Tables

Table 1: Sub-watersheds of Williyo catchment	2

Table 2: Slope class categories and landforms	2

Table 3: Land use/cover of Williyowatershed	2

Table 4: Livestock population in the watershed area	2

Table 5: Physical target and the required budget for Williyo watershed	2

Table 6: Three years physical target and the total budget required per year	2

Table 7: Total budget summary required by components and by year	2

Table 8: Community & Government contribution by component and year	2

List of Figures

Figure 1: Location Map of WilliyoWatershed	2

Figure 2: Sub-watersheds of Williyo catchment	2

Figure 3: Land use/land cover map of Williyo watershed	2

Figure 4: Major soilsofWilliyoWatershed	2

Figure 5: Development map of Williyo watershed	2

| ABBREVIATIONS |  |
| --- | --- |
| ARDO | Agriculture and Rural Development Office |
| ASTERBoARD | Advanced Space borne Thermal Emission and Reflection Radiometer Bureau of Agriculture and Rural Development |
| cms | Cubic meter per second |
| CN CBO CBPWD CWT CM | Curve Numbers Community Based Organization Community Based Participatory Watershed Development catchment management Community Watershed Team |
| DA DEM | Development Agents Digital Elevation Model |
| ESRI | Environmental Systems Research Institute |
| ETM+ | Enhanced Thematic Mapper plus |
| FAO FTC | Food and Agriculture Organization of the United Nations Farmers Training Center |
| GDEM | Global Digital Elevation Model |
| GIS | Geographic Information System |
| GPS | Global Positioning System |
| ha HH(s) | Hectare Household(s) |
| Km KWT | Kilometer Kebele Watershed Team |
| LULC | Land use and land cover |
| m.a.s.l | Meter above sea level |
| mm MoA M&E | Millimeter Ministry of Agriculture Monitoring and Evaluation |
| MoARD | Ministry of Agriculture and Rural Development |
| MoWR | Ministry of Water Resources |
| NGO | Non-Governmental Organization |
| NMSA NRM | National Meteorological Service Agency Natural Resources Management |
| OADB ONRS | Oromia Agriculture and Development Bureau Oromia National Regional State |
| PA P&ME | Peasant Association Participatory Monitoring and Evaluation |
| PI PWDP | Problem Identification Participatory Watershed Development Planning |
| REVAPC RUSLE | Re-evaporation coefficient for ground water Revised Universal Soil Loss Equation |
| SWAT SWC | Soil and Water Assessment Tool Soil and Water Conservation |
| UTM | Universal Transverse Mercator Projection |
| WGS 84 WWT | World Geodetic System 84 Wereda Watershed Team |

# INTRODUCTION

A watershed or watershedarea,is defined as any surface area from which runoff resulting from rainfall is collected and drained through a common confluence point. The term is synonymous with a drainage basin or watershed area. Hydrologically, watershed could be defined as an area from which the runoff drains through a particular point in the drainage system. Every stream, tributary, or river has an associated watershed, and small watersheds aggregate together to become larger watersheds. A watershed is made up of the natural resources in a basin, especially water, soil, and vegetative factors (CBPWD, 2005). People and livestock are the integral part of a watershed and their activities affect the productive status of watersheds and vice versa. From the hydrological point of view, the different phases of hydrological cycle in a watershed are dependent on the various natural features and human activities. Since the flow of water follows watershed boundaries, not political boundaries, any upstream activity on a watershed that people do to their land and water can significantly affect the productivity and sustainability of land downstream whether it is related to agriculture, livestock grazing, forestry or other uses. Among those influences are the effects on the rates of soil erosion and sediment movements, water flow patterns and water availability on the watershed. The management practices on upstream watersheds and the resulting effects of these practices are often part of a solution on the quantity, quality and delivery of water to downstream users.

In the past, Oromia National Regional State (ONRS) is known for its high agricultural potential area. Now a days, the region is highly affected due to sever land degradation due to many reasons. The rapidly growing number of population & rising demand for more food and agricultural land created pressure on land and its resources. These processes combined with poor farming system & the lack of appropriate soil and water conservation strategies caused the increase in the risk of floods and landslides, organic matter depletion, accelerated soil erosion, nutrient leaching and increased the amounts of sediment loads entering streams, rivers, reservoirs, and irrigation structures.

Land degradation is a serious problem in many developing countries like Ethiopia. JimmaGeneti woreda is one of those woredasof the HoroGuduruWeleggaZone under immense or sever degradation.Thewatershed is highly affected from land degradation that resulted to the decline in the productivity of land, and, hence the existing natural resources need to be properly utilized and conserved. On the basis of this, a catchment management (CM) study was conducted on Williyowatershed of JimmaGeneti woreda as part of Williyo small Irrigation feasibility study and detail design project. Therefore, this study helps to identify the biophysical and Socio-Economic conditions and constraints in the watershed and propose appropriate management interventions as an integral part of the project to ensure its sustainability in ways that maintain the long-term productive capability of the available natural resources in the area.

## Background

Commonly speaking ‘soil erosion’ generally refers to the removal of soil particles (including important soil nutrients) from its original place by soil moving agents (water and wind).  Soil erosion by water is the result of rain drop splash, and flowing runoff water, usually when the land surface is left without vegetation cover.  Erosion in a Watershed takes place in the form of sheet erosion, rills, and inter-rills and gullies leading to the decline of land productivity resulting from the decline of soil fertilityand fragmentation of land due to rills and dissected gullies. Besides, transportation of excessive sediment on downstream areas results in the loss of arable land and siltation on water courses, lower lying lands and irrigationinfrastructures.

In the past, conversion of forest, shrubs, and grass lands to agricultural land, cultivation of steep slopes and free grazing were commonly practiced activities observed in Williyowatershed. Besides, the majority of the watershed is under intensive cultivation of cereal crops that accelerated soil erosion. Moreover, mixed farming (crop production and livestock production) is the main agricultural system in the area. Grazing lands were highly trampled by livestock due to overstocking and greatly affected the physical structure of the soil. In addition, the average annual rainfall of the area, its rugged topography, soil characteristics, scarce vegetation cover, and inappropriate management practices are the main factors that contributed to soil erosion process in the Williyowatershed.

Because of the loss of protective vegetative cover and the gradual expansion of agricultural activities from gently sloping land onto the steeper slopes of the neighboring hills and mountainous areas, a large portion of the Williyowatershed is severely degraded and soil erosion is observed in various land use types in the watersheds.

Therefore, to reverse the above situation, a catchment management (CM) studywas conductedonWilliyowatershed aspart of Williyosmallscale irrigation feasibility study and detail design project. TheWatershed ispart of Abay River basin locatedin ONRS, HoroGuduruWelegga administrative zone, JimmaGenetiworeda. Williyowatershedhas a total area of about 1296.64ha.

The main objective of this study is to assess biophysical and socioeconomic conditionsof the area and propose appropriate intervention technologies that will reduce soil erosion and deforestation inthe Williyowatershed and thereby sustain optimum production in the area and minimize sediment inflow on to the proposed project schemes and command area.

## Objectivesof the Study

The overall objective of the study is to improve the livelihood of the community living in the watershed through comprehensive and integrated natural resource development. It aims at productivity enhancement measures for improved income generation opportunities, enhanced livelihood support systems and to optimize the use of existing natural resources and untapped potentials in both already degraded areas and in the remaining potential areas in the watershed.

Thespecific objective is to:

- Reduce soil erosion, rehabilitate degraded areas and maintain existing natural forest and woodlandvegetation cover  of the area;

- Conserve rainwater effectively to create additional water sources for productive uses;

- Improve soil infiltration rate that maintains irrigation water supply for the command area;

- Improve the livelihood of the communities in the watershed through creating and sustaining improved agricultural production systems;

- Recommend appropriate conservation measures depending on the biophysical & socio-economic problems, potentials and constraints of  the area;

- Preparewatershedmanagement plan, and estimate the cost required for implementing the proposed management interventions.

## Scope of the Study

Dependingon the biophysical and socio-economic conditions of the study area, the scope of watershedmanagement (CM) can be broadening beyond the NRM interventions. As stipulated in the TOR, the study will give high emphasis to conservation, rehabilitation and development of natural resources that will primarily contribute to the reductionof soil erosion and deforestation problems in the Watershed and thereby reduce sediment inflow to the proposed scheme and the downstream command area. The study encompasses collecting and analyzing of biophysical and socio-economic situations of the Watershed area and proposesintegrated CM interventions and strategies. Generally, the scope of the study includes the flowing:

- Review  the available data, polices and proclamations related to natural resources management,

- Delineate the Watershed boundary  and interpret  the physiographic  features of the Watershed area,

- Analyze  and interpret drainage  system , land use/land cover, soil, agro-ecology, and soil erosion and land degradation  status,

- Undertake land capability classification to identify soil conservation requirement classes to determine  required SWC practices  and techniques,

- Analyse and syntheses the identified problems and opportunities, and then recommend integrated Watershed management measures and strategies,

- Elaborate  action plan and estimate project budget requirement to implement the planned activities

## Review of previous studies

### Soil Erosion in Ethiopia

Ethiopia has been described as one of the most serious soil erosion areas in the world (Blaikie, 1985). Soil erosion by water and its associated effects are recognized to be severe threats to the national economy of the country (Hurni,1993; Sutcliffe, 1993).Alemneh (1990) also stated that the highlands of Ethiopia, which is the base of the nation’s economy, suffer from enormous land degradation due to deforestation and soil erosion. Hurni (1990, 1993) estimated that about 1.9 million metric tons of soil is eroded annually from the highlands of the country. Hurni (1993) also reported that the average annual soil loss of the country from agricultural lands was estimated to be 42t /ha/year. This figure can be even higher on steep slopes with a soil loss rate greater than 300 t/ha/year (USAID, 2000). This is far greater than the tolerable soil loss as well as the annual rate of soil formation in the country.

Of stated that of the many resources at risk in the Ethiopian highlands, soil and water are unarguably the most critical, as nearly 85% of the population depends on subsistence agriculture. About 50% of the highlands of Ethiopia are already significantlyeroded and erosion causes a decline in land productivity at the rate of 2.2% per year. The highlands of Ethiopia in general experience severe soil erosion mainly due to steep terrain, poor surface cover, intensive cultivation, of hilly areas, and degradation of grazing lands due to population and livestock pressure.

One process that severely threatens the resource base is soil erosion and its associated effects. The total amounts of runoff volume and sediment yields annually leaving the watershed are not easily quantified (Abdi et.al, 2012). Land and water resources of the area are adversely affected by the rapidly growing population and rising demand for cultivated land. Moreover, intensive cultivation of annual crops caused serious erosion problems in the area. Erosion generated from such intensively cultivated areas resulted in soil nutrient depletion or soil fertility reduction (Bezuayehu, 2006). Grazing areas were also highly trampled due to livestock overstocking. The number of livestock population is by far greater than the available grazing lands. This process coupled with the increasing number of population aggravated degradation in the area with multiple on-site soil erosion and off-site heavy sedimentation.

Other studiesalso showed that soil erosion and land degradation due to improper land use practices severely affect the land productivity resulting to the decline of agricultural production in most parts of the country. These studies also revealed that soil erosion also induces downstream sedimentation, which often adversely affect sustainability of dam/reservoirs, irrigation structures and downstream areas.  In general, soil erosion and its impact on land productivity depends mainly on topographic factors, soils characteristics, vegetation cover, rainfall and land management practices.

### AbayRiver Basin Master Plan Study

In a more general way, the study of soil erosion situation and the efforts made so far in soil and water conservation were carried out by MoWR(2007)in Abay River Basin Master Plan study. This report would be relevant to the target area of this study. The study indicated that soil erosion in the basin as a whole is increasing under the combined pressures of increasing population and deforestation, improper land use practices, over livestock population, overgrazing and free grazing systems.   In light of this, the study suggested that soil and water conservation strategy should focus to conserve productive lands.

## Policy and Proclamation

Several strategies and policies refer and support the need for the conservation and development of natural resources, community participation and diversified agriculture. Among others, the very relevant policy and proclamation to this study are described below:

### Rural Land Use and Administration Proclamation of the FDRE

The major land policy issues reflected in the proclamations are:

- Land is common property of the government and the people;

- Any citizen of the country or rural resident of the region of age 18 and above and wants to engage in agriculture shall have the right to use rural land;

- The rural land use right of farmers have no time limit;

- That land is the property of the government & the people and not subject to sale or exchange;

- Transfer of rural land use rights allowed throughinheritance, gift and rent;

### Rural Development Policy and Strategies

Proper use and management of agricultural land implies improving land productivity throughencouraging different conservation and rehabilitation mechanisms and rational utilizationof the country’s land resource. Enough emphasis should also be placed to conserve andrationally utilize water resources. Natural resource conservation is the base for sustainableagriculture. Conservation and management of natural resource that ensures realbenefit to communities is envisaged.

### Food Security Strategy (2002)

The strategy is targeted mainly to chronically food-insecure, moisture-deficit and pastoralareas. The focus is on environmental rehabilitation to reverse the current trend in landdegradation, and as a source of income generation for food insecure households. Watershedbasedwater harvesting and introduction of high value crops, livestock, and agro-forestrydevelopment are new elements in the revised strategy.

### New Coalition for Food Security Program

In this program, Integrated Participatory Watershed Management Planning is suggested forplanning and implementation of food-security interventions.

### Natural Resources and Environment Policy

The overall policy goal is to improve and enhance the health and quality of life of allEthiopians. The goal is to promote sustainable social and economic development throughsound management and use of natural, human-made and cultural resources and theenvironment as a whole.

### Land Administration and Use, Forest Conservation and Development Policies

The Land Administration and Use, Forest Conservation and Development Policies isalso believed to strongly support the currentparticipatory watershed development initiative.

These policies and strategies could be an excellent incentive to enable land users to feel ownership right and encourage them to encourage them to carryout natural resources management interventions.

# MATERIALS AND METHODS

## Materials

The following materials were used in conducting the study:

- Computer with its accessories;

- Field data collection formatsprepared for the study;

- GPS with Alkaline battery;

- Digital data like administrative maps, satellite imagery and topo-sheets;

- FAO digital soil map;

- FAO digital Land cover map and ASTER_DEM data.

- 4 wheel drive vehicle

- Software: Arc GIS, Arc Hydro, Arc Swat, Global Mapper, Remote sensing programs and USLE2, etc.

## Methods

This study was conducted following the approaches and methodologies outlined in the Community-Based Participatory Watershed Development (CBPWD) guideline of the Ministry of Agriculture and Rural Development (MoARD) of Ethiopia.  The steps listed in the guideline were followed to identify and prioritize the major problems of the study area based on biophysical and socio-economic survey and finally the most suitable development interventions were recommended for further actions. The study was conducted by dividing the entire work in to three stages:

Pre-field work

This stage includes preparation of formats and questionnaires, review of reports and existing documents, collection of spatial data, interpretation and preparation of maps. Detail activities performed at this stage includes:

- Preparation of check lists, formatsand questionnaires,

- Review of reports,records and important existing documents,

- Collecting ofspatial data (topo-maps, aerial photographs, satellite images, etc),

- Collection of secondary data and gathering information using structured formats,

- Discussion with  relevant government office staffs and respective  community members,

- Delineation of the watershed area from DEM data usingGIS techniques and interpretation of topographic and drainage patternof the watershed using 1: 50,000 scale topographic map, GPS etc.

- Interpretation of land use/cover of the watershed from remote sensing LANDSAT Digital images.

Field work

The field work includes conducting biophysical and socio-economic survey for defining each mapping unit (land use/land cover, slope map, erosion map, etc); describing the type, magnitude, and status of soil erosion in the site; and collecting other pertinent data and information for the watershed development plan.

- Group discussion was made with selected community members, influential persons, DAs, and Kebele officials.

- Collection of primary data by direct measurements and field observation,

- Conducting biophysical and socio-economic survey,

- Additional data were also collected through semi-structured questionnaire,

Post-field work

At this stage the collected relevant data were compiled and analyzed.

- The present land use/land cover map with the existing features and detail development map of the watershed was prepared using GIS techniques,

- Suitable development interventions that are most acceptable by the community and technically viable measures were recommended,

- Preparing and submitting the final complete report.

# STUDY AREA

## Location

WilliyoWatershedis located in ONRS, HoroGuduruWelegga Zone, JimmaGeneti woreda (Figure 1).The altitude of the catchment ranges from 2950m to 3690masl and is located between 37o7'25.41''-37o10'38.55''E and 9o17'15.89''- 9o20'10.15''N. The watershed has a total area of 1296.64ha including the command area and is entirely located in DamuGenboKebele. Williyowatershedoriginates from the mountain peak of Tulu Jaldessa in the south.  The stream flows to Fincha hydro-dam reservoir to the North that finallydrains into the AbayBasin.

Figure 1: Location Map of WilliyoWatershed

Figure 2 shows the sub-watersheds of Wiliyo major watershed as the catchment is sub-divided in to three micro-watersheds for the simplicity of implementation. The area of each micro-watershed is also shown on Table 1 below.

Figure 2: Sub-watersheds of Williyo catchment

In the Williyo catchment all the three micro-watersheds need to be treated for reducing the rate of soil erosion, increasing the productive capacity of the land, and improving the infiltration rate of the soilthereby increasing the ground water table for the sustainability of the small scale irrigation project.

Table 1: Sub-watersheds of Williyo catchment

| S/No | Sub-watersheds | Area (ha) | % Cover |
| --- | --- | --- | --- |
| 1 | CWS-1 | 1901.17 | 15.46 |
| 2 | CWS-2 | 1365.22 | 11.11 |
| 3 | CWS-3 | 1374.48 | 11.21 |
|  | Total | 12295.48 | 100.00 |

CWS-Community watershed

## Topography

Topography is one of the main factors affecting the rate of soil erosion in the watershed. Topographic features that influence erosion are degree of slope, shape and length of slope and size and shape of the watershed. On steep slopes, runoff water is more erosive and easily transport detached sediment down-slope. On longer slopes, an increased accumulation of overland flow tends to increase rill erosion. Therefore, slope steepness and its length have a major implication on soil erosion and subsequent sediment inflow to lower lying lands. It determines detachment and transportation of soil particles.

Williyowatershedis narrow in shape and is characterized with rolling and hilly rugged topographyupstream of the diversion site. However, the watershed area below the proposed weir site has gently sloping and rolling landscapes. The slope classes of the catchment area is categorized depending on the slope ranges indicated in Community Based Participatory Watershed Development guide line (CBPWSD)  of the M0ARD and is given in Table 2 below.

Table 2: Slope class categories and landforms

| Slope classes | Range (%) | Size (ha) | Coverage (%) |
| --- | --- | --- | --- |
| Flat or almost flat | 0-3 | 97.14 | 7.49 |
| Gently sloping | 3-8 | 405.85 | 31.30 |
| Sloping | 8-15 | 463.79 | 35.77 |
| Moderately steep | 15-30 | 286.72 | 22.11 |
| Steep | 30-50 | 34.72 | 2.68 |
| Very steep | > 50 | 8.42 | 0.65 |
| Total |  | 1296.64 | 100.00 |

As indicated in the table above, the catchment area is mainly characterized by gently sloping, and moderately slopingtopography. About 31% of the catchment area falls under gently sloping topography; 36% sloping; 22% moderately sloping; and the remaining 7% of the watershed area falls under flat or almost flat land. Very steep topography covers about 8.42ha that accounts for only less than 1% of the total area.About 62% of the watershed area characterized with slope categories greater than 8% are significantly susceptible to erosion as these areas are under intensive cultivation of annual crops and are not covered with natural or artificial vegetation and woodland that prevent the barren area from the forces of rain drops.

## Agro-Ecology

The general purpose of an agro-ecological zoning is to provide a tool for assessing which areas are ecologically suitable for various land use alternatives and land management practices. Therefore, it is the delineation of land into certain categories based on bio-physical and socio-economic factors. Precipitation, soils, altitude, temperature, evaporation, farming systems and vegetation types are the usual classification components, singly or in combination.

The rainfall distribution of the area has been estimated from stations around the watershed areai.e.Shambu rain gauge stations. Data processing includes filling of the missed data. Accordingly, the mean annual rainfall of the watershed is estimated to be 1041 mm and receives from May to September. The erratic nature of the rainfall made farming unpredictable and generally put it at low level of productivity.

## Land use/Land cover

Identification of the present land use/land cover of an area is essential part of watershed management study. The way a given land unit is used along with other physical features and socio-economic conditions determines the erosion process and the management interventions it receives. Based on this, the land use/land cover of the Williyowatershed was processed from satellite remote sensing with the application of Geographical Information System (GIS) techniques. It was also confirmed with ground truth during field visit in the area and from Google earth images as well. Accordingly, five major land use/land cover classes were identified for the Williyo watershed area (Table3) -namely, farmlands (69.2%), shrub/bush lands (10.8%), grazing lands (9.2%), homesteads or settlement areas (8.3%), and marsh area(2.5%). The watershed has a total area of about 1296.64ha including the proposed command area. The spatial distributions of each land use /land cover for the watershed is shown on Figure 3.

Table 3: Land use/cover of Williyowatershed

| Land use | Area (ha) | Coverage (%) |
| --- | --- | --- |
| Farmlands | 897.61 | 69.23 |
| Shrub/Bush lands | 139.96 | 10.79 |
| Grazing lands | 119.05 | 9.18 |
| Settlements | 107.53 | 8.29 |
| Marsh area | 32.49 | 2.51 |
| Total | 1296.64 | 100 |

### Farmlands

Farmland is the largest land use unit of the Williyowatershedand covers about 897.61ha which accounts for 69.23% of the total watershed area. It includes the command area and is found distributed throughout the watershed. The area is intensively cultivated mainly dominated with rain fed annual crops. Traditional irrigation practices are few and limited to small plot of land for growing vegetables. The major crops grown in the area are cereal crops (Maize, teff, Barley, Wheat, Oats, etc), pulses (field bean, field pea, and lentil), oil crops (Noug and flax) and horticultural crops (potatoes, vegetables, carrot, onions, etc). In spite of its significant potentials, the production of fruit trees is rarely observed in the area.

Figure 3: Land use/land cover map of Williyo watershed

### Shrub lands

Shrub/bush lands coverabout 139.96ha and accounts for 10.79% of the total Williyowatershed area. They are found in the upper extreme and in the middle of the watershed. This land use is used as the source for collecting fire woods, fencing material and also serves as a communal grazing area for the local farmers.

### Grazing lands

Grazing lands covers about 119.05 ha which accounts for 9.18% of the total watershed area. It is the only source of livestock feed in the area in addition to crop residues during the harvest of annual crops. They are found in the middle and around the outlet of the watershed below the command area. In Williyowatershed, grazing lands are owned commonly and are used in common by the local communities living nearby. The majority of this grazing areas are highly overgrazed and trampled by livestock due to over stocking.

### Settlements

The total area occupied by rural settlements in Williyo watershed is about 107.53ha which accounts for8.29% of the total watershed area. This land use is mainly dominated by the rural villages where the rural community resides & is distributed throughout the watershed. Other settlements include various institutions such as schools, churches, health posts, Kebele administration offices, etc.

### Marsh area

The marsh area covers about 32.49ha which accounts for 2.51% of the total Williyowatershed area. It is found atthe outlet of the watershed and is the entry point of the Fincha reservoir. This area is used as the main source of livestock feed and, therefore, the majority of the communities downstream of the proposed weir site freely stock their cattle in this land use throughout the year.

## Major Soil types

Soils have particular importance in CM study. The soils characteristics, which influence the erosion process in a Watershed, are soil texture, structure, organic matter content, soil fertility, soil chemical properties and stoniness.Therefore, Watershed planning process needs basic knowledge about major soils characteristics.

There are many nomenclatures of soils that are used by different disciplines. In Agriculture, widely used and the most suitable classification is the FAO/UNESCO classification system. According to this classification, twomajor soil types havebeen identified in the study area (MoWR, 2007); namely HaplicAlisols(92.6%) and PellicVertisols (7.4%) (Figure 4).The details of these major soils are described below:

### Alisols

Alisols are soils that have a higher clay content in the subsoil than in the topsoil as a result of pedogenetic processes (especially clay migration) leading to an argicsubsoil horizon. Alisols have a low base saturation at certain depths and high-activity clays throughout the argichorizon.

Alisols occur predominantly on hilly or undulating topography. The generally unstable surface soil of cultivated Alisols makes them susceptible to erosion; truncated soils are quite common. Toxic levels of Al at shallow depth and poor natural soil fertility are added constraints in many Alisols. As a consequence, many Alisols allow only cultivation of shallow-rooting crops and crops suffer from drought stress in the dry season. A significant part of the Alisols is unproductive under a wide variety of crops.

The use of acidity-tolerant crops or low-volume grazing is common. The productivity of Alisols in subsistence agriculture is generally low as these soils have a limited capacity to recover from chemical exhaustion. Where fully limed and fertilized, crops on Alisols may benefit from the considerable Cation Exchange Capacity (CEC) and good water-holding capacity, and the Alisols may eventually grade into Luvisols. Alisols are increasingly planted toAl-tolerant estate crops such as tea and rubber but also to oil-palm and, in places, to coffee and sugar cane.

Vertisols cover the bigger part within the Watershed that occupied for about 59.8% of the total area.  This is found in the lower and middle part of the watershed.  PellicVertisols are dark usually water logged and very deep to moderately deep. The soil has high water retention, but a relatively small amount of water is available for plant growth. Rooting might be restricted because of the swelling and shrinking properties of the soil. Run off formation from Vertisols is high and hence Vertisols are susceptible to water erosion unless properly managed.

### Luvisols

Among the Luvisols, chromic Luvisols exist in the watershed area which is found found in upper part of the study area.  .These soils are reddish brown to dark radish clay loam soils. The soils component is characterized by a few surface stoniness and non-rock outcrops and non-coarse fragments in the profile. The infiltration, drainage and workability of these soils are moderate. However, they are susceptible to erosion, unless soil and water conservation measures are integrated in the farming system. Although these soils are mainly suitable for crop production, the topography where these soils found is moderately steep slope, hilly, hilly plains and the depth of the soil is shallow. Hence, they are mainly under natural forest cover.

Figure 4: Major soilsofWilliyoWatershed

## Soil Erosion and State of Natural Resources Management

In the past, due to the decline of the fertility and productivity of land, farmers were forced to use shifting cultivation encroaching into vegetation, hillsides, and steep slope areas. Areaswhich were oncecovered with forests and bush lands before few decades ago were slowly converted to farmlands & settlements by clearing off the existing natural forests & shrubs. In this way, forest and grazing lands were diminishing and the natural resources of the watershed are gradually deteriorating from time to time resulting in land degradation. In this system, farmers cultivate their land for two consecutive years and left it again as a fallow for one or two years and the system repeat itself in this manner.

At present, unsustainable management of the natural resource is manifested by cultivation of fragile hillsides and steep slopes and changing grazing areas into cropland. River bank erosions are also a widely observed problem at the right and left banks of Williyo River due to concentration of high run off coming from upstream of the catchment.  Besides, the agricultural production system in the area is intensified with cultivation of rained crops. Because of this sheet and rill erosionare prevalent in the area, resulting to less water infiltration, organic matter depletion, enhanced gully formation, and overall loss of land productivity.

Despite the presences of high demand for forest/wood products in the area, artificial forest development is limited in the area. This has happened due to low extension services and limited access to tree seedlings in the vicinity of the area. On farm nursery activities is almost none. Moreover, the existing forest/shrub resources also lack joint forest management system. Traditional agro forestry practices are not applied in the area and fallowing is the only practice used to improve the soil fertility and most farmers use cow dung around their homestead.

The community has underlined that expansion of degradation is highly correlated with population growth and lack of alternative income source. The degree of degradation of natural resource base was explained by the farmers as follows:

- Crop cultivation expanded to steep and very steep slopes as well as to marginal areas,

- Soil fertility decreased due to the combined effect of frequent cultivation, lack of conservation measures, increased run-off and siltation problems,

- Currently, there is no natural forest in the area due to agricultural encroachment to forest areas,

- Very little communal grazing lands left in the area that leads to shortage of fodder & grasses for livestock resulting to overgrazing, poor vegetation cover and land degradation,

- With the increasing demand for agricultural production, the trend of traditional natural resources management is reduced,

- The discharge of existing springs decreased in the watershed due to the reduction of natural forest coverage,

- The existing traditional agro forestry system is under question if the current condition continues.

As discussed with various groups (FGD) of the community (community leaders, elders and youth groups), the major factors for soil erosion occurrence in the area include scarce vegetation cover, frequent cultivation of farmlands and limited use of SWC measures. The undulating and steep topography, fragile nature of the soils, and seasonally abundant rainfall distribution in the area are also other contributing factors for the severity of soil erosion in the Watershed.

Understanding of the soil erosion processes and degree of occurrence is important in order to arrive at proper catchment management interventions and put remedial measures. The degree of erosion differs from place to place depending on the land use type being used. Erosion generated from varies land uses also vary accordingly. This indicates that few areas of the watershed are critical and responsible for high amount of soil losses and sediment delivery. Therefore, it is necessary to identify high risk areas of the catchment to prioritize areas for specific soil conservation plans for reducing further degradation, reclaiming the degraded areas and improving the land productivity of the watershed through application of appropriate SWC technologies.

### RUSLE model

The scientific planning for soil conservation and water management requires knowledge of the relations among those factors that cause losses of soil and water, methods for controlling those factors, and to reduce such losses. The rate of erosion for a given site results from the combination of many physical and management variables. Actual measurements of soil loss would not be feasible for each of these factors that occur under field conditions. Erosion and sedimentation by water involves the process of detachment, transport, and deposition of soil particles (Foster 1983). The major forces are from the impact of raindrops and from water flowing over the land surface. The Revised Universal Soil Loss Equation (RUSLE) is an empirically based model that has the ability to predict the long term average annual rate of soil erosion on a field slope based on rainfall pattern, soil type, topography, crop system and management practices (Renardet al., 1997). RUSLE is an erosion model designed to predict the longtime average annual soil loss carried by runoff from specific field slopes in specified cropping and management systems as well as from grazing lands. It retains the factors of the USLE by including improved means of computing soil erosion factors. The RUSLE model in GIS environment can predict erosion potential on a cell-by-cell basis, which is effective when attempting to identify the spatial pattern of soil loss present within a large watershed area (Shi et al., 2004). GIS can then be used to isolate and query these locations to identify the role of individual variables in contributing to the observed erosion potential value (Saavedra, 2005). In spite of this advantage, RUSLE does not estimate remote deposition and gully erosion.

RUSLE is given by the following equation:

A = R x K x L x S x C x P

Where: A = computed average annual soil loss in tons/ha/year;

R = rainfall-runoff erosivity factor;

K = soil erodibility factor;

LS = slope length and steepness factor;

C = cover management factor; and

P = support practice factor

### Major Factors Affecting Erosion

RUSLE, an empirically based equation, derived from a large mass of field data, computes sheet and rill erosion using values representing the four major factors affecting erosion. These factors are climate, soil conditions, topography and land use and management represented by R, K, LS and CP, respectively. These factors are described below:

Therainfall factor (R)

The rainfall factor (R), is the product of the energy contained in rain storms multiplied by their maximum 30-minute intensity for all storms of more than 12.5 mm; it is also called the EI30 index. It is a measure of the total annual erosive rainfall for a specific location, as well as the distribution of erosive rainfall throughout the year. The most important characteristics of rainfall are rainfall intensity (how hard it rains), rainfall amount (how much it rains), and runoff that occurs during different seasons of the year.

Rainfall drives inter-rill and rill erosion. Soil loss is high where much intense rainfall occurs, whereas soil loss is low where very little rainfall occurs. Thus, rainfall erosivity varies from area to area depending on the amount of rain it receives.

The Soil Erodibility Factor(K)

The Soil Erodibility Factor (K) is a quantitative measure of a soil's inherent susceptibility/resistance to erosion and the soil's influence on runoff amount and rate. It is affected by soil texture and structure, organic matter content, permeability, and season of the year. Erosion by raindrop impact is not easily seen, but varying degrees of rilling indicate differing erodibility among soils. Knowledge of basic soil properties such as texture provides an indication of erodibility. For example, soils high in clay and sand have low erodibilities while soils high in silt have high erodibilities.

The Topographic factor (LS)

In USLE, the slope factor (LS)is a measure of the effects of slope angle, length and complexity on erosion. Topography, especially steepness, affects soil loss. Intense rilling is evidence that steep slopes like road cuts and fills experience intense erosion when bare. Runoff that accumulates on long slopes (overland flow path lengths) is also highly erodible, especially when it flows onto steep slopes. Thus, slope steepness and overland flow path length are major indicators of how topography affects erosion. Slope shape (steepness along the overland flow path)also affects erosion and deposition as evidenced by both erosion and deposition on concave slopes.

The crop cover and management factor (C)

The crop cover and management factor (C)is a measure of the relative effectiveness of soil and crop management systems in preventing or reducing soil loss. Cultural practices include: vegetative cover, crop rotations, conservation tillage and mulching.It is affected by:

- crop canopy (leaves and branches of the crop, which intercept the raindrops and dissipate some of their erosive force),

- surface cover (crop residues and live vegetation on the soil surface),

- soil biomass (all vegetative matter within the soil; residue helps to improve the flow of water into the soil and the soil water-holding capacity),

- tillage (type, timing and frequency of tillage operations; has an effect on soil porosity,

- surface roughness and compaction),

- previous year's crop, and

- Distribution of erosive rainfall over the growing season.

The support practice factor (P)

The support practice factor (P)is defined as the ratio of soil loss with a given conservation practice to that under crops in rows running up and down the slope. It is a measure of the effects of practices designed to modify the flow pattern, grade, or direction of surface runoff and thus reduce the amount of erosion. Some of the most common support practices are: cross slope cultivation, contour farming, strip-cropping, terracing, and grassed waterways.

### Estimating Soil Losses

RUSLE2 computes soil loss and other erosion values using inputs for climate, soil, topography, and use practices and conditions. These values stored in the RUSLE2 database under names for locations, which identify climatic variables; soil; cover management conditions and practices; and supporting practices. The user selects a name from a menu list for each of these factors to compute erosion. RUSLE2 “pulls” the values associated with each input name from the RUSLE2 database. The user changes values of particular variables from those stored in the database as needed to represent site-specific conditions related to topography, yield (production level), rock cover, and type and amount of applied materials like manure and mulch.

Accordingly, the average annual soil loss from Williyo catchment is greater than 37 tones/ha /year, which is high based on the soil loss category.   Therefore, conservation measures such as zero tillage, crop rotations, and terraces, contour strip cropping, and cover of permanent vegetation need to be employed.

The long term consequences of soil erosion problems are mainly associated with the decline in the productivity of land, loss of valuable forest resources, micro climate change, decline in the yield of streams and siltation of sediments inside watercourses and lower lying areas including the weir site and the proposed command area. Therefore, proper planning and implementation of watershed management measures are necessary in this watershed to maintain existing natural resources and ensure sustainability of the proposed small irrigation project.

# SOCIO-ECONOMIC CONDITION

## General

The study of the socio-economic situation of a catchment helpsto have clear understanding of the area. The main interactions of the people with the various levels of the watershed will be described in detail. It provides useful information about land resources in the watershed and assess the opportunities (internal and externals available for development) and the major issues and limitation that may hinder proper watershed development.

In general, to identify the major problems, constraints and opportunities that help prioritize the main development interventions and choose locally acceptable and technically sound development options, assessment of the socio-economic situation of an area is very crucial.

Socio-economic conditions of the watershed were collected through predefined format, discussion with relevant GO staffs (experts, DAs etc.) and observation during field visitingin the area and discussion through informal and random interviews of community membersduring transect walk in the area. The findings are discussed in the following sections.

## Population

The total population in thewatershedis about 1945, out of which 52% are males and 48%are females. The average number of family per household is found to be 4.5 people. The sex distribution in the study area shows that the numbers ofmales’ populationare bit greater than females. The areais densely populated and the settlement is mainly located in flat to gentle slope lands and bottom of hillsides mainly in the middle part of the watershed.

## Agricultural production

Understanding the farming system of a watershed offers opportunities to accurately plan the most suitable approach to hasten the agricultural development of the area through optimized &sustainable use of the natural resources. In Wiliyo Watershed, the agricultural production system is mixed farming system, crop & livestockproduction.

### Crop Production

The area is intensively cultivated mainly dominated with rain fed annual crops. The major crops grown in the area are cereal crops (Maize, teff, Barley and Wheat, Oats, etc), pulses (fiel bean, field pea, and lentil), oil crops (Noug and flax) and horticultural crops (potatoes, vegetables, carrot, onions, etc). In spite of its significant potentials, the production of fruit trees is rarely observed in the area.

Cereals are mainly cultivated for consumption, pulsesand oil crops are produced both for consumption & market. Potato and cabbage are the major vegetables & root crops. Land holding household cultivates about 2.19ha on the average (72% cereals, 6% pulses, and 22% others). Household annual grain production amounts to about 31qt (66.7% cereals and 2% pulses).

In the study area, farmers practice crop rotation to improve the fertility of their farmland as well as to minimize weed and control various diseases. Among others, cereal –pulse – cereal, and cereal – cereal – cereal are the most commonly crop rotation practiced in the area.Out of these types, the cereal-cereal-cereal approach is widely practiced. However, such type of crop rotation contributes to soil fertility depletion.

As they have access to inputs, farmers apply chemical fertilizer (DAP and UREA) to their farmlands for the major crops grown in the watershed such as Maize, Barley, Wheat and teff). Improved varieties of these major crops are also used by the farmers.Because of this, the total annual crop production in the area is better as compared to those areas where farmers do not have access to inputs. It was reported that the productivity of the land per hectare with optimum rate of fertilizer application for the major crops grown in the area is improved. Therefore, farmers need to be encouraged through adequateaccess to better improved inputs anddiversification of crop production systems in the area.

Although plenty of potential streams and springs exist, traditional irrigation practices are few and limited to small plot of land for growing vegetables and some other horticultural crops like potato, green paper, head cabbage, sugarcane and maize.

### Livestock Production

The major livestock types found in the study area are Cattle, Sheep, Goats,Horse, Mules and donkeys. Theykept livestock for multi-purpose i.e. for draught power, milk, meat, transport services, earning cash income, and riskaversion in time of crop failure and etc. The total population of livestock in the watershed is given in (Table 4).

Table 4: Livestock population in the watershed area

| Descriptions | Quantity | % |
| --- | --- | --- |
| Oxen | 405 | 12.91 |
| Cattle | 1249 | 39.82 |
| Sheep | 896 | 28.56 |
| Goats | 412 | 13.13 |
| Donkey | 55 | 1.75 |
| Horse | 105 | 3.35 |
| Mules | 15 | 0.48 |
| Total | 3137 | 100 |

As it is clearly indicated in the Table, animals kept in the watershed include 405 oxen, 1249 cattle, 896 sheep, 412 goats and 175 equines. The Cattle population dominated accounting for 40% of total livestock, followed by 29% sheep. The average number of livestock per household is about 4. Traditional beekeeping is practiced in some extent by farmers. The common animal products used by residents in the community watershed area are milk, cheese, meat, egg, hides & skin. Oxen are used to plough cropland, cows for milk, shoats for sale, and equines for transport. Animal manure is also used as a source of fuel. Further to these, animals are sources of cash when farmers need money as well as during the time when they faced private problems.

Traditionally, the farmers are practicing free grazing in their own plot. The major sources of animal feed are natural pasture from the private fallow and grazing lands which contribute about 68% and the remaining 14% source of animal feed is from crop residues. Yet, there is no any improved forage development in the area.

The total number of livestock population kept in the watershedis highcompared to the available grazing lands. Additional sources of livestock feed comes from crop residues, mainlyWheat, Barley, teff, and other cereal crops. Free grazing is the commonly practiced grazing system in the area and, therefore, it greatly contributedto soil erosion and destruction of vegetation cover of the area due to overgrazing and trampling by livestock.

In general, the livestock management system followed in the community watershed area is very traditional. Livestockproductivity also remained low due to inadequate supply of livestock feed throughout the year. In the study area, there is huge shortage of livestock feed during the dry season. Failure to provide adequate feed throughout the year certainly reduces the possible income from any livestock production. Thus, proper livestock development strategy should be devised as part of the watershed development to improve productivity as well as to minimize soil erosion.

Shortage of grazing land and lack of improved forages; prevalence of livestock diseases, poor quality of the existing local breed, absence of nearby AI and bull services; lack of awareness on livestock and grazing land management and feed quality enrichments and invasion of weeds are cited as the major problems.

As to curve all these drawbacks as well as to boost up the production and productivity of the livestock sector in the area, introducing and planting improved grass and fodders in physical structures, homestead areas, grazing fields and gullies are very crucial. As to improve the productivity and sustainability of communal areas, proper land use and management system need to be adopted. Proper feed reservation and quality improvement are also the focus areas which need attention. Regarding quality improvement of the existing breeds, partnerships should be strengthened along with those concerned offices which work on animal diseases prevention, AI and bull services, improved breeds and fodder providers.

### Apiculture

Apiculture has a great contribution to rural economy as it promotes self-reliance and requires small land and investment. When beekeeping is integrated with forestry development activities, it conserves the natural resources and contributes to environmental protection. Honeybee has also a great role in economic balance by pollination of different trees and crops.

The Williyowatershed has some potential for beekeeping due to favorable ecological factors. The area has also relatively good market access being found near the woreda town. In this study area, most farmers practice beekeeping using traditional beehives. Some farmers have also engaged in transforming from traditional beehives to modern or transitional beehives in the previous times. Productivityof Honey form a traditional beehive in the area is about 5kg /harvest.  Therefore, there is a need to support the farmers to promote improved technology and practices to increase the productivity of honey in the area.

### Land Tenure System

Land tenure security can influence land management, because it may affect communities’ encouragement or ability to invest on land improvement. Landholding under insecure tenure is less likely to be developed in SWC and planting trees. The current constitution of FDRE has given private rural land use right that allows the farmer to use, rent, transfer, debt and lease.  Land certification isalready issued to the farmers living in the study area. However, the tenure system on natural resources is still communal and open access with limited regulation on the use and management of the resources contained in it. These may contribute to the destruction of natural resources as the community may not fell sense of ownership on lands which are not certified. Under this situation, it is also difficult to realize the newly issued land use policy and proclamation of the region.

### Rural Energy

Types of biomass energy resources in use in the study area are wood, animal dung and crop residue in their order of importance and fuel demands. The cooking stove is traditional (open fire)that consumes more fuel wood as compared to improved fuel stoves. In this study area, the promotion of improved stoves is low. This remains one of the most important factors that have been aggravating deforestation of exiting natural forest resources. This happened due to lack of the technology in the locality, low awareness of the community on energy conservation and the need for it and poor extension on the promotion of fuel-saving stoves.

Thus, the use of fuel- saving stoves and alternative energy sources is crucial to reduce the current rates of deforestation and subsequently to overcome the problem of fuel wood shortages. This can also help to reduce workload on women and children.

### Gender Issues

Although the National constitute gives access right to all community owned properties on equal basis to women and men, in Ethiopia women hardly access to and control over the family resources and have low social status in the community. Similarly in this study area, women and men have different access and control over family resources. Economic capacities and incentives are also strongly gender-differentiated in ways, which affect supply response, resource allocation within the household, labor productivity, and welfare. Due to unfair division of labor in the community, the workload of women is immense as they are responsible for all domestic activities.

Although Women may suffer more than men due to inadequate availability of various resources like water, fuel wood, food, etc, their participation on the management of land is limited, otherwise confined around homesteads areas. Therefore, women need to be involved in  the CMactivities and high emphasis should be given both to fulfill both strategic and practical needs of women through different interventions like  extension services, family planning and  etc.

### Infrastructure Development

Basic infrastructures like all-weather rood, telecommunication, education, safe water supply services, etc are important for economic development of any society. In the catchment, there is one dry weather road connecting the project area to the woreda town.

The coverage of access to safe drinking water supply is very low, and the source of water supply for the majority of the community living in the watershed is streams, springs and hand dug wells. In the watershed, there isonly one elementary School. High school and preparatory school are available in the woreda town.

### Institutions

#### Community based institutions

Institutionsare basically established by the community or a group of people to satisfy their religion, social and economic needs. Generally, Mosque,Church,Idir, and Debo,are most important community based institutions that have been providing various services. Kebele and farmer’s cooperatives are also the most important community based institutions established for facilitating political, economic, and social and justice services to the rural community. The kebele is sub-divided in to Zones and gothsconsisting of 10 to 20 members, mainly composed of productive labor age groups and HH heads.

Generally, those types of community gatherings are most important to achieve genuine community participation in development activities. However, the capacity of the institutions needs to be strengthened to carry out their vested responsibilities appropriately.

#### Government Institutions

The DA and health extension promotion workers are the most important GO institutions based at Kebeles which are very relevant to this study. The DA is under the ARD office. Three DAs are assigned at kebelelevels that areresponsible in promoting, disseminating and adoption of improved technologies and practices on agriculture, forestry development, and SWC interventions. Two female health extension workers are assigned ata kebele level, whichare responsible in facilitating awareness creation education on environmental and personal hygiene, HIV/AIDs, family planning, prevention of malaria, and related interventions. They also support home management activities for rural women.

At woreda level, severalgovernmental institutions exist, whose joint effort is to satisfy the needs of rural community. These institutions are directly or indirectly responsible to environmental protection, agricultural development and sustainable natural resources management. In general, the existences of those institutions are highly important to support the rural community inimproving their livelihoods.

# PROBLEM ANALYSIS

## General

The biophysical and socio-economic situations of the watershed are discussed in the previous sections. Therefore, these need to be analyzed to identify major problems and their root causes to put solutions on solving or mitigating the identified problems and constraints. In this case, the CM interventions shall be planned to address root causes of the problems instead of the consequences.

The analysis indicates that the major problems/constraints of the study area include soil erosion, deforestation,decline in crop production, low livestock production and productivity, low alternative income sources and inadequate local capacity. The main root causes of the identified problems include population pressure, inadequate extension services, improper land use practices and low diversification of income sources. Factors such as topography, soil conditions, inappropriate farming practices, agro-climate and vegetation coveralso contributing for the present situation.

The major problems and opportunities identified in the watershedare described below:

## Major Problems

### Soil erosion

Soil erosion is a physical process and refers to the removal of soil particles (including plant nutrients and organic matter) from the place where they belong to elsewhere by either water or wind. It is one of the most important components of land degradation. Soil erosion and degradation is a reduction in soil depth and fertility.However, the degree of erosion varies from place to place depending on the topography of the area, soil types, vegetation cover, and the force of erosive agents. All forms of water erosion such as sheet and rills, gully, stream bank erosion and land sliding on very steep slope areas area widely observed in Williyowatershed. The causes of this process is invariably a combination of natural phenomena and human actions such as cultivation of steep areas, deforestation, overgrazing, encroachment of farm activities to stream banks and poorfarming practices etc.  Moreover, population pressure, limited access to extension services, land tenure, and low awareness of farmers about conservation of the natural resourcesaggravated the problem.

### Deforestation

Deforestation is the loss or removal of vegetation cover of an area. In Williyo catchment active deforestation was not observed. But the whole watershed area has no natural vegetation cover except scarcely distributed shrubs. In the past, deforestation was resulted from over utilization of forest resources of the area through cutting of trees for construction and fuelwood, expansion of agricultural land, and burning etc.This may lead to shortage of various wood resources, soil erosion, drying up or declining in yield of ground water sources and other environmental consequences.  There is wide spread argument that deforestation causes a reduction of rainfall on a local and regional scale, because the recycling process of water vapor through forest transpiration will be  reduced where there is deforestation.

These are the collective result of intuitional and policy constraints, social and economic factors prevailing in the area. Uncontrolled cutting of trees for various purposes, notably firewood, charcoal, construction and expansion of agriculture due to rapid population growth are the major causes.  Low awareness of the community, land tenure security problems, lack of alternative rural energy technology and highly dependent on woody biomass for fuel wood are the other contributing factors.

In spite of the above problem, Forest development activityis limited in the area. As per information collected from the Woreda office, onlytwo centralnurseries exist in the woreda to supply tree seedlings for various plantation strategies. These have also lowcapacity about 0.7 million seedlings, mainly eucalyptus tree species. Thishas happened due to financial constraints to run nurseries.Therefore, adequate numbers of nurseries need to be established and run to produce multipurpose trees seedlings to the community in their locality. Moreover; existing natural vegetation need to be protected via community based management system through awareness raising education sessions.

### Low livestock production

The main challenge in livestock productivity is inadequate supply of livestock feed and water resources. Shortage of livestock feed resources is a serious problem particularly in the dry seasons.  Furthermore, overgrazing by large no of livestock must be looked upon as one of the causes for instability of natural vegetation and soil resources.   Therefore, appropriate livestock husbandry and forage development strategies should be devised in the community to improve livestock productivity.

### Low diversification of crop production

Although the area has potential for crop diversification, the dominant crops grown are mainly cereals that require large areas which areincomparable with ever increasing population pressure in the area.  Thus, crop diversification and intensification are important both throughrain fed and irrigation development are necessary to improve crop production and increase availability of food crops on sustainable basis.

### Limited alternative income sources

In the study area, most HHsoperates with low income because they have no diversified income generating sources.  This could be associated with lack of working capital to engage in various IGAs, limited knowledge and skill on entrepreneur, weak institutional set up, and limited access to rural credit and saving services.

Therefore, providing financial and technical support to the poor rural community member is necessary to encourage them to involve in feasible IGAs.  This in turn contributes to improve the income and consequently reduced unnecessary burdens on existing natural resources. In this regard, the area has significant potential for IGAs like fatting of small ruminants mainly Goats, and apicultureetc.

### Low Local Capacity

The effectiveness of the regional and local efforts to improve the environment and livelihoods of the inhabitants through natural resource conservationis highly related to thelocal capacity. Lack of technical knows and skill in field staffs, budget constraint and shortage of trained work force at local level are the major problems related to capacityin the local government. They lack experiences on principles, approaches and techniques of CM interventions. As the result, the extension service to the communities is limited and gives low attention to conservation of natural resources. Low awareness and limited knowledge of the rural community on management of natural resources and adoption of improved production techniques and practices are also other major constraints.

Therefore, strengthening the local capacity through training and equipping with necessary equipment (tools) are highly important to achieve effective watershed management.

## Opportunities

In a CM, all the existing opportunities in terms of bio-physical resources and socio-economic conditions need to be identified. The study has identified the following opportunities that will support to achieve development of the watershed in the future.

### Bio-physical opportunities

- Suitable  agro-ecology  to introduce multipurpose  trees and shrubs  for livestock feed, soil fertility maintenance and ecosystem  conservation, and  to diversify  productions,

- Adequate moisture in the soil   for establishing planting materials,

- Possibility of carrying out fattening through controlled grazing and cut and carry system, thereby reducing pressure on existing vegetation cover,

- There is a potential for promoting apiculture  in the area,

- The streams flow continuously throughout the year and will serve as a good source both for the development of small scale irrigation andfor domestic supply.

### Socio-Economic Opportunities

In this study area, there are many socio-economic opportunities to alleviate the problems and respond to the community’s felt needs: The following are among others:

- the existence of many laborers in the rural community  and ample culture to work together is a good opportunity to carryout SWC  activities,

- indigenous knowledge of the local community  on traditional  apiculture practices,

- the presence of land use policy in the country and enactment of rural land administration and land use proclamation by the regions,

- Some experiences in dairy farming with localand cross-breeds.

# PROPOSED WATERSHED MANAGEMENT INTERVENTIONS

## Main Strategies

A watershed is a physiographic unit encompassing different land units such as hillsides, drainage lines, plains, grazing lands, farmland, and settlements. Watershed rehabilitation projects have first to understand the causes of degradation rather than trying to address the effects of it. Untreated and/or deforested hillsides are always sources of flooding and erosion for lower-lying farmlands and grazing lands; hence, treatment of hillsides should be carried out before starting gully treatment. Though construction of any physical structures is becoming a common practice in watershed management, they are only for a short-term solution, until the vegetation cover of the degraded area, whether from natural regeneration or artificial plantation, recovers.

The management of watersheds, therefore, requires the application of scientific methods and principles of welfare economics within a framework of public policy which can accommodate multiple objectives, the ultimate objective being improved people’s livelihood through restoring and enhancing land productivity. Careful management of watersheds is a core element of good agriculture and management because it can minimize land degradation, stabilize water stream flows, reduce sediment load, and recharge groundwater stores. In this way effective watershed development integrates the management of water resources, land, welfare, healthcare, and yet other aspect of development.Due to the nature of the project and limited scope the proposed interventions will give high emphasis to interventions that will reduce soil erosion and deforestation of the existing natural resources.

A broad range of possible interventions and strategies are proposed depending on the identified problems, constraints and opportunities. The recommended measures will help conserve the available natural resources of the watershed which in turn increase the productivity of the land thereby improving the livelihood of the community living in the watershed area. The major intervention areas recommended for the watershed include:

- Soil and water conservation;

- Reforestation;

- Crop production management;

- Livestock  management;

- Pasture/forage management;

- Income generating activities;

- Cross-cutting issues;and

- Capacitydevelopment.

These proposed interventions will be implemented to suit a variety of land uses/land cover in the watershed as discussed below.

## Watershed Management Interventions

### Soil and Water Conservation Measures

In soil and water conservation, engineering and biological principles are applied to solve or address soil and water management problems. Accordingly, soil conservation measures are broadly classified into physical soil conservation measures and biological soil conservation measures. For effectiveness both measures are used in combination to reinforce one another. Biophysical measures, which are a combination of physical and biological treatment (planting trees, shrubs, legumes and grasses) are most important in the provision of immediate benefits to farmers. The guiding principle here is to have water infiltrate into the soil as much as possible where it falls, so as to increase the water-holding capacity for the benefit of crops and natural vegetation, as well as for groundwater replenishment. Any excess runoff must be safely disposed of from the land, but needs to be managed within the watershed by constructing sinkholes or percolation pits along the drainage canals. Soil in solution with water needs to be filtered out and re-deposited as close as possible to its original location.

#### Physical Soil and Water Conservation Measures

Physical SWC measures are mechanical or structural engineering measures developed through soil cutting and earth moving to reshape the topography. They are mechanical barriers constructed across the slope to control the velocity of surface runoff and thereby reduce soil loss by erosion. The important principles to be kept in view while planning physical control measures are:

- Increasing the time of concentration of runoff thereby allowing more of it to be absorbed and held by the soil,

- Intercepting a long slope into several short ones so as to maintain less than a critical velocity for the runoff water,

- Protection against damage due to excessive runoff.

Usually such measures are not complete on their own and require the addition of a vegetative cover before becoming fully effective and permanent.

This category includes: bunds, terraces, moisture retention structures (like micro-basins, eye-brows, half-moons, trenches etc.), check dams, sediment storage dams (SS-dams), cutoff drains and waterways. Physical conservation structures, especially stoneor soil bunds will be implemented based upon the local conditions and availabilityof construction materials. Stone bunds are aimed at dissipating the energy of therunoff and filtering the soil transported between the terraces. Stone bunds arerecommended in upper catchments of dam sites, semi-water logged areas, poorlydrained and shallow-profile soils. Soil bunds, meanwhile, are constructed in areas where thereare not enough stones for terracing; where deep and well-drained soils exist, andin areas where moisture is much needed for crops to grow. Physical SWC structures, if not well constructed and maintained, can create more damage than good, indeed major disasters. Different biophysical structures need to be combined in a way which gives maximum benefits to the smallholder farmers.The main factors to be considered in the selection of physical SWC are climate, soil, topography and farming practice and other socio-economic situation of the area.

The physical SWC interventions recommended for the Williyowatershed are described below:

Soil Bunds

Soil bunds are earth embankments constructed across the slope to act as a barrier to runoff, to form a water storage area on their upslope side and break up a slope into segments shorter in length than is required to generate overland flow. They are suitable on cultivated lands with slopes between 3 to 15%.Soil bunds need to be spaced at 15 to 20m and at vertical interval of between 1 to 2m. Above 15% hillside terraces are recommended. The bunds could be stone-faced in areas with high stoniness and stable soils. Planting of grasses and other multi-purpose agro-forestry species should be integrated with such measures. Moreover, they should be stabilized with legume shrubs such as pigeon peas, Sesbaniasesban,Acacia saligna, vetiver grasses, etc. on the lower sides of the bunds.

Cutoff Drains

Cut-off drains are graded channel constructed to intercept surface run-off from higher ground/slopes and protect downstream areas. Its capacity can be estimated from runoff coming from upland watershed. The location of cut-off drain is determined after checking the outlet conditions. If it is intended to protect cropland against runoff from adjoining non-arable land, it should be constructed at the boundary between the two, while when needed to divert runoff from a gully head, it should be far 1.5 times the depth of the gully above the gully head so that stable slopes will exist. Cut-off drains are to be implemented in all types of mapping units wherever necessary and should be stabilized with grass splits and shrubs seedlings.

Waterways

A waterway is a natural or artificial drainage channel that leads water down a slope. Natural waterways are very efficient in safe disposing runoff for the reasons that being located in depressions usually approaches the required design slope, and the construction expenses are less than in artificially excavated channels. It takes runoff water from cut-off drains and carries it to rivers and streams. It is usually constructed one or two seasons ahead of the construction of these structures to make stabilized before runoff is entering to the ditches. Waterways are applicable in areas where natural waterways are not available. Technical specification of waterways depends on the volume of runoff envisaged to carry.

Check dams

In this study area, some small gullies were observed along road ditches, cattle trucks and following the river courses. Therefore, high emphasis should be given to addressing the root causes instead of tackling the effects. The techniques include diverting runoff (partially or wholly) plantation along the gully sides, and bed, and construction of check dams across gullies to hold backwater and trap sediments.

The type of check dam selected to treat gullies depends on the gully features (depth, width, gradient, gully formation stage, slope), availability of construction materials, and slope gradient of the gullies. To this end, live brush wood check dams and stone check dams, all integrated with plantations along the side of the gullies are proposed.  These activities should be constructed in cost sharing systems in that the community will participate in the labor contribution in collection and provision of local available material such stones, while the project will cover the labor cost required for the implementation of the planned activities.

Stream Bank Protection

In Wiliyo watershed stream bank erosion, land sliding and creation of deep gorges is one of the problems related to soil erosion close to stream banks. Due to the lack of appropriate SWC measures the continuous removal of soil by erosion from stream bank aggravated the problem. This stream bank erosion resulted to the loss of productive cropped lands from either side of the streams. Therefore, protecting of stream banks should be important part of the watershed management.

#### Biological Soil and Water Conservation Measures

Biological soil and water conservation measures are measures that include vegetative barriers, agronomic and soil fertility improving practices which help in controlling surface runoff, reduce soil loss and are often considered as reinforcement measures. They can stand alone, or in combination with physical SWC structuresto reduce farmland degradation improve land productivity. Applications may vary depending on slope type and the agro-ecological zone. BiologicalSWC measures are sustainable and cheap and provide immediate and diversified benefits to the rural community.

Application of biological measures is based on the principles that:

- It protectsthe direct impact of raindrops through establishment of adequate ground cover and intercept the rainfall energy before reaching the ground,

- It reduces the concentration of runoff and consequently minimizes the surface flow velocity,

- It increases the soil resistance to erosion.

Vegetative barriers comprise grass strips and agro-forestry on cultivated lands, area closure and afforestation on non-cultivated lands. Agronomic practices include crop rotation, intercropping, strip cropping, alley cropping, mulching, vetiver hedge, contour cultivation, cash crops (chat, fruits, Gesho, pigeon pea) and tree plantations on bunds, soil fertility management, conservation tillage, crop residue management, improved seed production, grass hedge rows and compost making will be applied in the intervention area.

Some of the proposed interventions are presented below:

Nursery establishment

Nursery site is the place where multi-purpose tree seedlings and grasses are produced forvarious purposes like woodlot, backyard, homestead plantation and other strategies. In this watershed about two nurseries are planned with a capacity to produce about 1 million MPT seedlings, annually. The distribution of the nurseries will be based on the potentials and agro-ecological variation and the availability of access roads and many other factors. The investment and operation costs of these two nurseries are expected to be fully supported by the government, while the communities will contribute free labour and locally available materials. Produced seedlings will be distributed to the community through cost sharing mechanisms to sustain the nurseries.

Furthermore, effort has to develop to establish private nurseries to enable the farmers to satisfy their seedling demand by themselves through the provision of technical and material assistances to selected community members. In this case, it is planned that at least 10 % of the households living in the watershed will have their own nursery either in group or individually with a capacity to produce a minimum of 500 seedlings annually. The type of tree species to be produced in the nurseries depends up on the agro-ecological conditions of the area.

Afforestation/Reforestation

Trees and forest are critical resources for people’s livelihoods, environmental conservation and economic development. Majority of the population depends directly on wood for their energy needs (firewood and charcoal), materials for furniture and construction (timber and poles) as well as food and other non-timber forest products (including fruits, nuts, medicinal plants, fodder, etc.).

Many factors are directly influencing the loss of forest cover and the decline in the forest resource base. These include forest clearance for agriculture, wood fuels or charcoal production, the over-harvesting (poor planting and weak regulation, non-sustainable harvesting of forest products) and degradation of forests, the encroachment of government reserves and the degazetting of forest reserves for alternatives purposes, mainly cultivation of agricultural crops as a result of population pressure.

Consequently, it has become vital to identify appropriate land–use approaches for the production of multipurpose outputs, and to ensure the sustainability of the production base.Forests have a critical role to play in watershed development. They are essential to maintain the environment. This suggested the need for reforestation to restore the forests of the most degraded or deforested areas with multi-purpose tree seedlings. The diversification of tree species will initiate the future forest cover and ensure an ecological integration of the restored areas in the overall forest cover and provide habitats for wildlife.

Agroforestryactivities also need to be practiced widely on farmlands and around homesteads. Plantation of multipurpose tree seedlings would be implemented primarily on steep and hillsides and degraded areas integrated with moisture retentions structures such as micro basins, trenches, and hillside terraces. From the time plantation is completed, the area need to be closed and free grazingand human interferences must be avoided.

Afforestation also helps as a source of income generating activities like: establishment of nurseries, sustainable supply of timber for future generations, increasing pasture outputs, developing small business relying on non-timber products such as honey, medicinal plants, wild fruit, handicraft…etc and ecotourism development.

Area Closure

Area closure is a protection system and saving mechanism of land against any external influences including erosion. Basically, slopes above 30% are recommended to be closed from day to day human and livestock interferences to give a way for the regeneration of natural trees and grass vegetation.In Williyo watershed there is no natural forests at all. However, there is few bush lands comprising about 11% (140 ha) of the total watershed area with scarcely distributed trees and shrubs. The soils on theses area are highly exposed to external erosive forces and, hence, need especial attention. These degraded areas (grazing, shrub/bush lands, gully banks, abandoned farmlands etc) need to be closed for rehabilitation purposes. Gullies should be temporary closed at least for two years or so until the vegetation cover regenerate naturally. Such areas also need to be enriched with plantation of multipurpose trees.

Compost Making and Utilization

Compost is an organic fertilizer that can be made on the farm at very low cost. The most important input is the farmer’s labour. Composting is a technology for recycling organic materials in order to achieve enhanced agricultural production. Biological and chemical processes accelerate the rate of decomposition and transform organic materials into a more stable humus form for application to the soil. To improve soil fertility in the long term, it is necessary to improve the soil structure and to increase the organic matter content of the soil. Compost is a good fertilizer because it contains nutrients as well as organic matter. As a measure of improving soil fertility and enhancing crop production, compost making should be one of the interventions in this CM. Therefore, strengthening the ongoing extension service with respect to compost making through training and support to the community members need to be considered as important option for CM. Compost making is a natural process of turning organic material into valuable plant nutrients called humus. When properly made and applied, compost provides readily available plant nutrient.  Compost is easily prepared using crop residues, garden weeds, kitchen and household wastes, and any other vegetative materials.

There are two basic methods of making compost: heap and pit method. Heap method is built above the ground, while the pit method requires digging a large pit. Both methods could be suitable for this area with proper management.  Therefore, about 50% of the land holding HHsliving in the watershed area will be expected to prepare at least about 5 m3 of compost per house hold annually andapply on their farmland to increase the productivity of the land, thereby increasing crop yield.

Crop residue management

Crop residues include any biomass left in the field after grains and other economic components have been harvested. Plant residues provide a renewable resource for incorporation into soil organic matter.

In a natural system, the production of plant residues is balanced by the return of dead plant material to the soil. In agricultural systems, however, as plants are harvested, only about 20 percent of production is accumulated in the soil as organic matter. Crop residues are usually the largest source of organic materials available to farmers. Residue return following harvest is important to improve soil biological, physical and chemical conditions.

Crop residue is a tremendous natural resource in crop production. About 25% of nitrogen (N) and phosphorus (P), 50% of the sulfur and 75% of potassium (K) uptake by cereal crops are retained in crop residue making them a valuable nutrient source.  Leaving residues stubble after harvesting the grain and incorporating this stubble into the soil can increase grain yield. Crop residue is also a source of organic matter. Soils with high organic carbon retain more plant nutrients, increasing crop yields and, thus, food security.

In Williyo watershed about 80% of the land holding HHs will be expected to practice crop residue management on their farmland to increase the productivity of the land, thereby increasing crop yield.

Grass Strips

Grass strips are vegetative barriers made out of grass planted in narrow strips of 0.5 to 1.5 meters width laid along the contour or along the physical conservation measures. While contributing to protect soils against erosion they also provide valuable biomass meant to increase animal feed or used for different purposes. Grass strips can be used alone on slopes ranging between 3-8% and integrated with physical measures alternatively between 8-15% slopes.

The strips will be laid out at 1m vertical interval; widths of strips could vary from 0.5 to 1m based on density of plants in the strip. Spacing between the seedlings/splits should not be wider than 5cm to guarantee effective grass strip. They can be integrated with lines of legume shrubs such as pigeon peas, Sesbania, Tree-Lucerne and Acacia-saligna planted in dense rows. Common grass species such as Elephant grass, Rhodes, Setaria, Phalaris and Vetiver grass could be used as grass strips.

### Crop Production Management

Diversification and intensification of agricultural production should be considered as one of the main components of watershed management interventions. Improving diversity and production of food crops helps to increase availability of food crops and yield, consequently reduces pressure on natural resources by increasing income generating opportunities. To this end, the following interventions area proposed under this component:

Conservation agriculture (CA)

Integrated watershed management sees a high potential for increased agricultural productivity, on the basis of introduction of modern agricultural practices, use of high yielding and disease resistant varieties, development of irrigation infrastructure, reduction of post-harvest losses, agro-processing facilities, and marketing of agricultural products. Promotion of conservation agriculture is based on three key principles: i) minimum soil disturbance, ii) soil cover (with mulch, or cover crops, preferably legumes) and iii) crop rotation or association.

Conservation agriculture can reduce poverty directly by providing higher yields for most products, in a highly significant level, with progressive efficacy.Development of non-agricultural activities such as beekeeping or production of aromatic and medicinal plants will also act in favour of poverty reduction. The introduction of such approaches is most urgent in areas with highest erosion hazards or evidence. The combined production-protection oriented approach implies a much stronger emphasis on agro-forestry components and conservation farming. These are highly productive techniques providing a more continuous vegetative ground cover and a much better alternative for clean weeding land preparation before the onset of rainy seasons. Also, a well-established farming system along these lines would require less input of labour intensive terracing works.

Introduction of conservation agriculture practices aims at achieving better and more permanent soil cover in connection with improving soil fertility, providing protection from erosion and preventing outwash of agricultural chemicals. Conservation agriculture (CA) is seen as a more sustainable land-use practice that incorporates the essential elements of land preparation and planting, but eliminates the need to disturb the soil regularly. Besides reducing the cost of crop production, CA increases soil moisture and fertility significantly, leading to higher yields. These advantages indicate that for arable land, CA is a viable alternative to conventional soil conservation programs and traditional land preparation.

Conservation agriculture in the form of zero tillage, together with crop rotations and permanent cover, is more crucial in a landscape where repetitive tillage, downslope tillage and monocultures are prevalent. Where the traditional farming practices include perennial and annual species that already provide a good soil cover, improvements could focus on integrated soil fertility, water and biodiversity management for pest and disease control and added productivity and value.

Fruit Productions

Generally the Agro-ecological condition of the study area issuitable forgrowing of highland fruit trees. To this end,Apple fruit tree production is proposed for this watershed. This will be started through demonstrations on model farmers. The seedlings will be obtained from other part of the region and or it can be produced in the nursery by collecting cutting from other sources. Moreover, intensive training to participating farmers should be provided on agronomic aspects of apple fruit tree production as it needs great care than the lowland fruits. This can be carried out both through rain -fed and supplement irrigation water.

In this strategy about 150 HHs will be participated in growing of this fruit depending upon the availability of the land and willingness of the farmers’. The seedlingswill be distributed to through a cost sharing system to increase sense of ownership on the planted fruit seedlings.

Small scale irrigation development

In the watershed, farmers have been practicing traditional irrigation along the Williyo streams individually for growing vegetables both for consumption and market purpose. However, the production is very low as there was no any support. The proposed small scale irrigation development schemewill help to improve the livelihood of the farmers through increased production and ensure the sustainable availability of irrigation water throughout the year. Moreover, continuous training will be provided on irrigation agronomy to the community members and the woreda extension personnel. Accordingly, it is planned that the entirebeneficiary will be trained on irrigation agronomy and at least 50 % of them will have access to improved seeds on revolving fund system.

### Livestock Development interventions

Livestock cannot be regarded as the principle cause or the leading force in the process of soil erosion and deforestation; they certainly contribute to it owing to improper management practices. The livestock density in the study area is relatively high as compared to the available forages resources. Therefore, given the present situation, all lines of improvement must be for looked for and implemented to maintain the existing numbers of livestock. The proposed interventions are that of relevant to watershed management initiatives.  The followings are some of the proposed options:

Utilization of crop residues

Crop residues are very low in nutritional value. Therefore, efficient management and utilization of straw should be developed and promoted. The proposed technique for this study area is addition of herbaceous forage and tree legumes in the farming system and mix with straws to improve the quality of crop residues. This practice can be carried out by those farmerwho have been engaged in crop and livestock production activities.

Forage Development

Forage development is the introduction and growing of improved species that provides high quality of forages. This type of development will be more practicable for relatively smallholder dairy production and fattening activities integrated with cut and carry system. This strategy will be promoted at farmer's level using a wide range of forage species adapted to climatic conditions of the area. This helps farmers to fulfilled specific opportunity in the mixed farming systems. Accordingly, the following forage development strategies are proposed for this study area:

Forage development on bunds

Soil conservation structures must be sown or planted with improved forage species at the time that the physical erosion control structures are built. This strategy is very important to maximize production through compensating forage development and helps also to stabilize SWC structures that also reduce frequency of maintenances. It should also be closed from interference of livestock through the encouraging cut-and-carry feeding system. In this intervention at least 40 % of the proposed terraces should be planted or sown with best -bet,   grass, shrubs and tree species. Among others, Elephant grasses, Sesbania, Tree lucerne are suitable for this intervention.

Back-yard forage development

Back-yard forage development involves the establishment of plots or hedges of high quality and biomass of forages. In this strategy, it is planned about 100model HHS will be participated through demonstration. The size of the plot depends on the available land and its requirement. However, it should not be below the minimum standard recommended by the extension packages of the country that is 1250m2. Suitable species to this strategy are; Elephant grass, alfalfa, Sesbania, Cajuns Cajun, Tree Lucerne and fodder beet. To achieve this intervention, seedlings and splits will be raised in the tree nurseries proposed above.

### Income Generating Activities

Despite its low productivity, the livelihood of the community living in the study area is largely dependent on both livestock and crop production.Thus, the highest financial income source of these people mostly comes from the saleof cereals crops, small ruminants and from petty trading activities. However, those poor rural farmers who had no other option have been creating pressures on existing natural resources.

Therefore, creating another income sources is highly necessary to increase income of the HHs thereby reducing pressure on natural resources. Possibleincome generating activities (IGAs) in this study area include fattening,apiculture, woodlots, etc. These schemes could be achieved through improving knowledge and skills on entrepreneurs and ensuring sustainable rural saving and credit services in the locality. Accordingly, large number of the community will participate in the proposed IGAs organized under different groups. In this watershed about 150 apiculture promotion will be performed either in group or at HH level during the project life. The number of participants in a group depends on the type of IGAs.  For the planning purpose, it is assumed that a group will consists of 25 members.

### Supportive Strategies

As it was indicated in the previous sections, lack of energy saving technologies, high population pressure, the increasing demand for farmland and various forest resources and poor awareness of the communities on environmental protection are the most contributing factors forthe degradation of the natural resources in the study area.  Therefore, to reduce pressure on the available limited natural resources and bring about sustainability in the area, the following supportive strategies are proposed as part of this watershed management:

Fuel wood saving stove promotion

In order to restore the forest coverage in the catchment possible alternatives to fuel wood and energy saving technologies will be selected and introduced in the watershed. Promotion of fuel-wood saving stoves could reduce the pressure on forests. In this regard, Mirtstove will be provided to the community as it helps to reduce consumption of fuel wood by 50% compared to the open fire stove and thereby significantly reduces pressures on natural resources. It also helps to reduce workloads on women which might devote in collection of firewood and protects fire hazards on women and children. Under this strategy, it is planned that at least 30% women inhabiting in the watershed will have access to this technology.

Family Planning

Family planning (FP) could be considered as one of the interventions of watershed management because the ever increasing number of population greatly contributed directly or indirectly to the deterioration of the natural resources. Therefore, it is planned that about 50% of women in reproductive age groups will have access to family planning services including education and supply of contraceptive medicines. The family planning service and education will be provided through health extension workers.

### Local Capacity Building

Capacity development is one of the most important components of watershed development. The capacity building will target institutional and community capacities. Hence, developing the capacityof the woreda technical staffs and community members is very important for the sustainability of the area. Continuous trainings will be given on various topics like SWC, improved forest rehabilitation and management practices,tree nursery establishment and management, Fruit production, Apiculture, etc.to improve their implementing capacity.

Moreover, theworeda technical staffs with their role of providing technical advice to the community need to have adequate knowledge and skills on the principles and techniques of technological interventions. Therefore, enough training will be also provided to the woredastaffsparticularly to the technical staffs of Office of Agriculture, Irrigation Department, Water, Energy and Mineral offices as they will play the major role during the implementation of the planned activities.

## Planning and Implementation

The planincludes major activities to be implemented, list of inputs and materials, required budget, and time to complete the work. Implementation strategies are based on the concepts and principles of integrated watershed development outlined in the participatory watershed development guide lines (PWDP) of the MoARD of Ethiopia. Implementation period is proposed to be three years. However, it is subjected to revision by the implementing agencies based on the specific local situations. The development plan map of Williyo watershed is shown on Figure 5.

Total Budget Summary

As shown on Table 5, the total estimated cost for Williyo watershed for a period of three years is about 1,746,807.55Birr.It is sub-divided in to two main categories: namely, labour and input costs.Labour costs are paid in cash form based on daily wage rate for the community for the equivalent of physical activities performed and input costs are the cost needed for the purchase required input materials such as SWC hand tools, motor-bikes, office equipment, high yielding varieties of crops, improved vegetable seeds, fruits etc. Accordingly, the budgetrequired for labour and input costs are854,623.60 Birr(48.92%) and 892,183.95 Birr (51.08%), respectively. The total input costs are fully covered by the Government, whereas 456,380.53 Birr which is about 53.40% of the total labour cost required will be contributed by the community living in the watershed either as free labour cost or in kind. The total amount which will be contributed by the community accounts for about 26.13% of the total project cost required.

The budget allocated for the first, second and third year is about 629,914.55Birr (36.06%), 644,478.45Birr (36.89%) and about 472,414.55Birr (27.04%), respectively.

Figure 5: Development map of Williyo watershed

The following Tables indicate the major proposed measures and their corresponding required budget for Williyo watershed.

Table 5: Physical target and the required budget for Williyo watershed

|  | Interventions/Development strategies | Unit | Target | Norm | Required labour (PD) | Input cost (Birr) | Labour Cost (Birr) | Estimated Costs (Birr) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Natural Resource Management |  |  |  |  |  |  |  |
| 1.1 | Physical SWC |  |  |  |  |  |  |  |
| 1.1.1 | Soil-bund construction | km | 32 | 150 | 4800 |  | 115200 | 115200 |
| 1.1.2 | Stone-faced soil-bund construction | km | 3 | 250 | 750 |  | 18000 | 18000 |
| 1.1.3 | Gabion check-dam construction | m3 | 110 | 4 | 440 | 44000 | 10560 | 54560 |
| 1.1.4 | Loose-stone check-dam construction | m3 | 105 | 2 | 210 |  | 5040 | 5040 |
| 1.1.5 | Brushwood check-dam (double row) | m | 186 | 0.33 | 61.38 |  | 1473.12 | 1473.12 |
| 1.1.6 | Gully-wall reshaping | m3 | 114 | 1 | 114 |  | 2736 | 2736 |
| 1.1.7 | Gabion check-dam maintenance | m3 | 32 | 1 | 32 |  | 768 | 768 |
| 1.1.8 | Loose-stone check-dam maintenance | m3 | 125 | 0.5 | 62.5 |  | 1500 | 1500 |
| 1.1.9 | Waterway construction | m3 | 116 | 1.33 | 154.28 |  | 3702.72 | 3702.72 |
| 1.1.10 | Cut-off-drain construction | m3 | 68 | 0.7 | 47.6 |  | 1142.4 | 1142.4 |
| 1.1.11 | Maintenance of physical structures | km | 10 | 125 | 1250 |  | 30000 | 30000 |
| 1.2 | Biological SWC |  |  |  |  |  |  |  |
| 1.2.1 | Central nursery establishment | set | 1 | 150000 | - | 150000 |  | 150000 |
| 1.2.2 | Area-closure | ha | 15 | 4 | 60 | 9030 | 1440 | 10470 |
| 1.2.3 | Bund stabilization (grasses and legumes) | km | 15 | 30 | 450 | 25245 | 10800 | 36045 |
| 1.2.4 | Grass-hedge establishment (vetiver, etc) | km | 12 | 30 | 360 |  | 8640 | 8640 |
| 1.2.5 | Agro-forestry promotion (gravelia, etc) | no | 2500 | 0.087 | 217.5 | 2500 | 5220 | 7720 |
| 1.2.6 | Gully-revegetation& live fencing | ha | 3 | 500 | 1500 | 15000 | 36000 | 51000 |
| 1.2.7 | Afforestation/reforestation of degraded-land (gravelia, etc) | ha | 10 | 62 | 620 | 50000 | 14880 | 64880 |
| 1.2.8 | Shrub-land enrichment planting (gravelia, etc) | no | 15250 | 0.087 | 1326.75 | 15250 | 31842 | 47092 |
| 1.2.9 | Woodlot establishment (energy source) | no | 3000 | 0.087 | 261 | 3000 | 6264 | 9264 |
| 2 | Homestead & Livelihood development |  |  |  |  |  |  |  |
| 2.1 | Compost making (Pit: 4mL x 2mW x 1.5mD) | pit | 30 | 10 | - |  | 300 | 300 |
| 2.2 | Compost preparation (EM application) | m3 | 118 | 1 | - |  | 118 | 118 |
| 2.3 | Crop residue management (on-farm use) | ha | 25 | 10 | - |  | 250 | 250 |
| 2.4 | Green manuring (vetch application, etc) | qt | 12 | 10 | - | 5880 | 120 | 6000 |
| 2.5 | Fuel-wood saving stove promotion ('mirt', etc) | no | 54 | 1 | - | 8100 | 54 | 8154 |
| 3 | Community Infrastructure development |  |  |  |  |  |  |  |
| 3.1 | Spring capping/development | no | 2 | 60000 | - |  | 120000 | 120000 |
| 3.2 | Feeder road construction | km | 2 | 3000 | 6000 |  | 144000 | 144000 |
| 3.3 | Road maintenance | km | 1 | 500 | 500 |  | 12000 | 12000 |
| 4 | High value crop promotion |  |  |  |  |  |  |  |
| 4.1 | Tropical fruit promotion (avocado, etc) | no | 120 | 5 | - | 17400 | 600 | 18000 |
| 4.2 | Root & tuber crop promotion (Enset, etc) | no | 100 | 2 | - | 14500 | 200 | 14700 |
| 4.3 | Vegetable promotion (carrot, tomato, etc) | kg | 100 | 5 | - | 10000 | 500 | 10500 |
| 4.4 | Potato promotion (Genet, Tolcha, etc) | qt | 54 | 5 | - | 5400 | 270 | 5670 |
| 4.5 | Cereal crop promotion (Triticale, Sorghum, etc) | qt | 10 | 30 | - | 8450 | 300 | 8750 |
| 4.6 | Pulse crop promotion (Haricoat-bean, etc) | qt | 6 | 30 | - | 6000 | 180 | 6180 |
| 4.7 | Oil crop promotion (sunflower, etc) | qt | 5 | 30 | - | 5000 | 150 | 5150 |
| 4.8 | Maize-haricotbean intercropping | qt | 10 | 30 | - | 12000 | 300 | 12300 |
| 5 | Livestock management |  |  |  |  |  |  |  |
| 5.1 | Forage development along bund (sesbania, etc) | km | 12 | 16 | - | 20376 | 192 | 20568 |
| 5.2 | Grass land improvement | ha | 10 | 20 | - |  | 200 | 200 |
| 5.3 | Forage seed planting (vetch, etc) | qt | 3 | 500 | - | 1470 | 1500 | 2970 |
| 5.4 | Livestock feed conservation (crop-straw, hay, etc) | set | 36 | 10 | - | 8928 | 360 | 9288 |
| 5.5 | Poultry promotion (improved breed of egg/meat) | no | 125 | 125 | - | 15625 | 15625 | 31250 |
| 5.6 | Vet service provision | set | 200 | 1 | - |  | 200 | 200 |
| 5.7 | Apiculture promotion | set | 10 | 2 | - | 19720 | 20 | 19740 |
| 6 | Capacity Development & Knowledge |  |  |  |  |  |  |  |
| 6.1 | Farmer training on improved methods | no | 90 | 124 | - | 1350 | 11160 | 12510 |
| 6.2 | Farmers training on CBPWD Guideline | no | 80 | 124 | - | 1200 | 9920 | 11120 |
| 6.3 | NRM technicians/foremen training on SWC | no | 15 | 124 | - | 225 | 1860 | 2085 |
| 6.4 | Horticulture/field crop dev (Apple, Triticale, etc) | no | 100 | 124 | - | 1500 | 12400 | 13900 |
| 6.5 | Forage dev, feed conservation & feeding strategy | no | 100 | 124 | - | 1500 | 12400 | 13900 |
| 6.6 | Poultry management (improved breeds) | no | 100 | 124 | - | 1500 | 12400 | 13900 |
| 6.7 | Apiculture management (improved tech) | no | 50 | 124 | - | 750 | 6200 | 6950 |
| 6.8 | Small-scale Irrigation water use management | no | 40 | 124 | - | 600 | 4960 | 5560 |
| 6.9 | Potable water-scheme management (spring, etc) | no | 60 | 124 | - | 900 | 7440 | 8340 |
| 610 | Farmers' field-day (experience exchange) | set | 2 | 50000 | - | 30 | 100000 | 100030 |
| 6.11 | Crosscutting issues (Gender, HIV, family planning) | no | 100 | 124 | - | 1500 | 12400 | 13900 |
| 7 | Project management |  |  |  |  |  |  |  |
| 7.1 | Annual work plan & budget planning | pd | 6 | 190 | - | 270 | 1140 | 1410 |
| 7.2 | Technical and managerial support | pd | 50 | 190 | - | 2250 | 9500 | 11750 |
| 7.3 | Woreda support to community (watershed) | pd | 50 | 190 | - | 2250 | 9500 | 11750 |
| 7.4 | Woreda Logistic support (watershed dev) | set | 3 |  | - | 12000 |  | 12000 |
| 7.5 | Computer & printer with accessories | no | 1 |  | - | 25000 |  | 25000 |
| 7.6 | Stationery materials | set | 3 |  | - | 45000 |  | 45000 |
| 7.7 | SWC hand tools | gp | 10 |  | - | 12000 |  | 12000 |
| 7.8 | Motor bikes | no | 2 |  | - | 190000 |  | 190000 |
| 7.9 | Fuel & lubricants | lt | 3000 |  | - | 60000 |  | 60000 |
| 7.1 | Others (utilities, maintenance) | set | 3 |  | - | 15000 |  | 15000 |
| 7.11 | Filing-cabinet | no | 10 |  | - | 1000 |  | 1000 |
| 7.12 | Wooden-shelve with door | no | 1 |  | - | 1000 |  | 1000 |
|  | Total |  |  |  |  | 849,699.00 | 813,927.24 | 1,663,626.24 |
|  | Contingency (5%) |  |  |  |  | 42,484.95 | 40,696.36 | 83,181.31 |
|  | Grand Total |  |  |  |  | 892,183.95 | 854,623.60 | 1,746,807.55 |

Table 6: Three years physical target and the total budget required per year

| S/No | Interventions/Development strategies | Unit | Physical Target |  |  |  | Required Budget per year |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Total | Year 1 (30%) | Year 2 (40%) | Year 3 (30%) | Year 1 (36%) | Year 2 (37%) | Year 3 (27%) |
| 1 | Natural Resource Management |  |  |  |  |  |  |  |  |
| 1.1 | Physical SWC |  |  |  |  |  |  |  |  |
| 1.1.1 | Soil-bund construction | km | 32 | 9.6 | 12.8 | 9.6 | 34560 | 46080 | 34560 |
| 1.1.2 | Stone-faced soil-bund construction | km | 3 | 0.9 | 1.2 | 0.9 | 5400 | 7200 | 5400 |
| 1.1.3 | Gabion check-dam construction | m3 | 110 | 33 | 44 | 33 | 16368 | 21824 | 16368 |
| 1.1.4 | Loose-stone check-dam construction | m3 | 105 | 31.5 | 42 | 31.5 | 1512 | 2016 | 1512 |
| 1.1.5 | Brushwood check-dam (double row) | m | 186 | 55.8 | 74.4 | 55.8 | 441.936 | 589.248 | 441.936 |
| 1.1.6 | Gully-wall reshaping | m3 | 114 | 34.2 | 45.6 | 34.2 | 820.8 | 1094.4 | 820.8 |
| 1.1.7 | Gabion check-dam maintenance | m3 | 32 | 9.6 | 12.8 | 9.6 | 230.4 | 307.2 | 230.4 |
| 1.1.8 | Loose-stone check-dam maintenance | m3 | 125 | 37.5 | 50 | 37.5 | 450 | 600 | 450 |
| 1.1.9 | Waterway construction | m3 | 116 | 34.8 | 46.4 | 34.8 | 1110.816 | 1481.088 | 1110.816 |
| 1.1.10 | Cut-off-drain construction | m3 | 68 | 20.4 | 27.2 | 20.4 | 342.72 | 456.96 | 342.72 |
| 1.1.11 | Maintenance of physical structures | km | 10 | 3 | 4 | 3 | 9000 | 12000 | 9000 |
| 1.2 | Biological SWC |  |  |  |  |  |  |  |  |
| 1.2.1 | Central nursery establishment | set | 1 | 1 |  |  | 150000 |  |  |
| 1.2.2 | Area-closure | ha | 15 | 4.5 | 6 | 4.5 | 3141 | 4188 | 3141 |
| 1.2.3 | Bund stabilization (grasses and legumes) | km | 15 | 4.5 | 6 | 4.5 | 10813.5 | 14418 | 10813.5 |
| 1.2.4 | Grass-hedge establishment (vetiver, etc) | km | 12 | 3.6 | 4.8 | 3.6 | 2592 | 3456 | 2592 |
| 1.2.5 | Agro-forestry promotion (gravelia, etc) | no | 2500 | 750 | 1000 | 750 | 2316 | 3088 | 2316 |
| 1.2.6 | Gully-revegetation& live fencing | ha | 3 | 0.9 | 1.2 | 0.9 | 15300 | 20400 | 15300 |
| 1.2.7 | Afforestation/reforestation of degraded-land (gravelia, etc) | ha | 10 | 3 | 4 | 3 | 19464 | 25952 | 19464 |
| 1.2.8 | Shrub-land enrichment planting (gravelia, etc) | no | 15250 | 4575 | 6100 | 4575 | 14127.6 | 18836.8 | 14127.6 |
| 1.2.9 | Woodlot establishment (energy source) | no | 3000 | 900 | 1200 | 900 | 2779.2 | 3705.6 | 2779.2 |
| 2 | Homestead & Livelihood development |  |  |  |  |  |  |  |  |
| 2.1 | Compost making (Pit: 4mL x 2mW x 1.5mD) | pit | 30 | 9 | 12 | 9 | 90 | 120 | 90 |
| 2.2 | Compost preparation (EM application) | m3 | 118 | 35.4 | 47.2 | 35.4 | 35.4 | 47.2 | 35.4 |
| 2.3 | Crop residue management (on-farm use) | ha | 25 | 7.5 | 10 | 7.5 | 75 | 100 | 75 |
| 2.4 | Green manuring (vetch application, etc) | qt | 12 | 3.6 | 4.8 | 3.6 | 1800 | 2400 | 1800 |
| 2.5 | Fuel-wood saving stove promotion ('mirt', etc) | no | 54 | 16.2 | 21.6 | 16.2 | 2446.2 | 3261.6 | 2446.2 |
| 3 | Community Infrastructure development |  |  |  |  |  |  |  |  |
| 3.1 | Spring capping/development | no | 2 | 1 | 2 | 1 | 30000 | 60000 | 30000 |
| 3.2 | Feeder road construction | km | 2 | 0.6 | 0.8 | 0.6 | 43200 | 57600 | 43200 |
| 3.3 | Road maintenance | km | 1 | 0.3 | 0.4 | 0.3 | 3600 | 4800 | 3600 |
| 4 | High value crop promotion |  |  |  |  |  |  |  |  |
| 4.1 | Tropical fruit promotion (avocado, etc) | no | 120 | 36 | 48 | 36 | 5400 | 7200 | 5400 |
| 4.2 | Root & tuber crop promotion (Enset, etc) | no | 100 | 30 | 40 | 30 | 4410 | 5880 | 4410 |
| 4.3 | Vegetable promotion (carrot, tomato, etc) | kg | 100 | 30 | 40 | 30 | 3150 | 4200 | 3150 |
| 4.4 | Potato promotion (Genet, Tolcha, etc) | qt | 54 | 16.2 | 21.6 | 16.2 | 1701 | 2268 | 1701 |
| 4.5 | Cereal crop promotion (Triticale, Sorghum, etc) | qt | 10 | 3 | 4 | 3 | 2625 | 3500 | 2625 |
| 4.6 | Pulse crop promotion (Haricoat-bean, etc) | qt | 6 | 1.8 | 2.4 | 1.8 | 1854 | 2472 | 1854 |
| 4.7 | Oil crop promotion (sunflower, etc) | qt | 5 | 1.5 | 2 | 1.5 | 1545 | 2060 | 1545 |
| 4.8 | Maize-haricotbean intercropping | qt | 10 | 3 | 4 | 3 | 3690 | 4920 | 3690 |
| 5 | Livestock management |  |  |  |  |  |  |  |  |
| 5.1 | Forage development along bund (sesbania, etc) | km | 12 | 3.6 | 4.8 | 3.6 | 6170.4 | 8227.2 | 6170.4 |
| 5.2 | Grass land improvement | ha | 10 | 3 | 4 | 3 | 60 | 80 | 60 |
| 5.3 | Forage seed planting (vetch, etc) | qt | 3 | 0.9 | 1.2 | 0.9 | 891 | 1188 | 891 |
| 5.4 | Livestock feed conservation (crop-straw, hay, etc) | set | 36 | 10.8 | 14.4 | 10.8 | 2786.4 | 3715.2 | 2786.4 |
| 5.5 | Poultry promotion (improved breed of egg/meat) | no | 125 | 37.5 | 50 | 37.5 | 9375 | 12500 | 9375 |
| 5.6 | Vet service provision | set | 200 | 60 | 80 | 60 | 60 | 80 | 60 |
| 5.7 | Apiculture promotion | set | 10 | 3 | 4 | 3 | 5922 | 7896 | 5922 |
| 6 | Capacity Development & Knowledge |  |  |  |  |  |  |  |  |
| 6.1 | Farmer training on improved methods | no | 90 | 27 | 36 | 27 | 3753 | 5004 | 3753 |
| 6.2 | Farmers training on CBPWD Guideline | no | 80 | 24 | 32 | 24 | 3336 | 4448 | 3336 |
| 6.3 | NRM technicians/foremen training on SWC | no | 15 | 5 | 5 | 5 | 688.05 | 708.9 | 688.05 |
| 6.4 | Horticulture/field crop dev (Apple, Triticale, etc) | no | 100 | 30 | 40 | 30 | 4170 | 5560 | 4170 |
| 6.5 | Forage dev, feed conservation & feeding strategy | no | 100 | 30 | 40 | 30 | 4170 | 5560 | 4170 |
| 6.6 | Poultry management (improved breeds) | no | 100 | 30 | 40 | 30 | 4170 | 5560 | 4170 |
| 6.7 | Apiculture management (improved tech) | no | 50 | 15 | 20 | 15 | 2085 | 2780 | 2085 |
| 6.8 | Small-scale Irrigation water use management | no | 40 | 12 | 16 | 12 | 1668 | 2224 | 1668 |
| 6.9 | Potable water-scheme management (spring, etc) | no | 60 | 18 | 24 | 18 | 2502 | 3336 | 2502 |
| 610 | Farmers' field-day (experience exchange) | set | 2 | 1 | 1 | 1 | 33009.9 | 34010.2 | 33009.9 |
| 6.11 | Crosscutting issues (Gender, HIV, family planning) | no | 100 | 30 | 40 | 30 | 4170 | 5560 | 4170 |
| 7 | Project management |  |  |  |  |  |  |  |  |
| 7.1 | Annual work plan & budget planning | pd | 6 | 2 | 2 | 2 | 465.3 | 479.4 | 465.3 |
| 7.2 | Technical and managerial support | pd | 50 | 12.5 | 25 | 12.5 | 2937.5 | 5875 | 2937.5 |
| 7.3 | Woreda support to community (watershed) | pd | 50 | 12.5 | 25 | 12.5 | 2937.5 | 5875 | 2937.5 |
| 7.4 | Woreda Logistic support (watershed dev) | set | 3 | 1 | 1 | 1 | 3000 | 6000 | 3000 |
| 7.5 | Computer & printer with accessories | no | 1 | 1 | 0 | 0 | 7500 | 10000 | 7500 |
| 7.6 | Stationery materials | set | 3 | 1 | 1 | 1 | 13500 | 18000 | 13500 |
| 7.7 | SWC hand tools | gp | 10 | 3 | 4 | 3 | 3600 | 4800 | 3600 |
| 7.8 | Motor bikes | no | 2 | 2 | 0 | 0 | 57000 | 76000 | 57000 |
| 7.9 | Fuel & lubricants | lt | 3000 | 900 | 1200 | 900 | 18000 | 24000 | 18000 |
| 7.1 | Others (utilities, maintenance) | set | 3 | 1 | 1 | 1 | 5000 | 5000 | 5000 |
| 7.11 | Filing-cabinet | no | 10 | 3 | 4 | 3 | 300 | 400 | 300 |
| 7.12 | Wooden-shelve with door | no | 1 | 1 | 0 | 0 | 300 | 400 | 300 |
|  | Total |  |  |  |  |  | 599,918.62 | 613,789.00 | 449,918.62 |
|  | Contingency (5%) |  |  |  |  |  | 29,995.93 | 30,689.45 | 22,495.93 |
|  | Grand Total |  |  |  |  |  | 629,914.55 | 644,478.45 | 472,414.55 |

Table 7: Total budget summary required by components and by year

| S/No | Descriptions | Total Budget (Birr) | Budget per year (Birr) |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | 1st year | 2nd year | 3rd year |
| 1 | Natural Resource Management | 619233.24 | 290769.97 | 187693.30 | 140769.97 |
| 2 | Homestead & Livelihood Development | 14822.00 | 4446.60 | 5928.80 | 4446.60 |
| 3 | Community Infrastructure Development | 276000.00 | 76800.00 | 122400.00 | 76800.00 |
| 4 | High Value Crop Promotion | 81250.00 | 24375.00 | 32500.00 | 24375.00 |
| 5 | Livestock Management Improvement | 84216.00 | 25264.80 | 33686.40 | 25264.80 |
| 6 | Capacity & Knowledge Management | 202195.00 | 63721.95 | 74751.10 | 63721.95 |
| 7 | Project Management | 385910.00 | 114540.30 | 156829.40 | 114540.30 |
|  | Grand total (with 5% contingency) | 1,746,807.55 | 629914.55 | 644478.45 | 472414.55 |

Table 8: Community & Government contribution by component and year

| S/No | Descriptions | Total Budget (Birr) | Community Contribution (Birr) |  |  | Government Contribution (Birr) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 1st year | 2nd year | 3rd year | 1st year | 2nd year | 3rd year |
| 1 | Natural Resource Management | 619233.24 | 145384.99 | 93846.65 | 70384.99 | 145384.99 | 93846.65 | 70384.99 |
| 2 | Homestead & Livelihood Development | 14822.00 | 2223.30 | 2964.40 | 2223.30 | 2223.30 | 2964.40 | 2223.30 |
| 3 | Community Infrastructure Development | 276000.00 | 15360.00 | 24480.00 | 15360.00 | 61440.00 | 97920.00 | 61440.00 |
| 4 | High Value Crop Promotion | 81250.00 | 6093.75 | 8125.00 | 6093.75 | 18281.25 | 24375.00 | 18281.25 |
| 5 | Livestock Management Improvement | 84216.00 | 12632.40 | 16843.20 | 12632.40 | 12632.40 | 16843.20 | 12632.40 |
| 6 | Capacity & Knowledge Management | 202195.00 | 0.00 | 0.00 | 0.00 | 63721.95 | 74751.10 | 63721.95 |
| 7 | Project Management | 385910.00 | 0.00 | 0.00 | 0.00 | 114540.30 | 156829.40 | 114540.30 |
|  | Grand total (with 5% contingency) | 1,746,807.55 | 190,779.16 | 153,572.21 | 112,029.16 | 439,135.40 | 490,906.24 | 360,385.40 |

# IMPLEMENTATION STRATEGIES

The whole watershed development activities will be coordinated and guided by the woreda focal person (FP). The focal person is basically either SWC expert/specialist or natural resource management expert/specialist assigned from the Natural Resources Development Department of the woreda Office of Agriculture (ARDO).  The woreda Steering Committee (WSC) and woreda watershed team (WWT) from various sector offices will be established at woreda level. At kebele level, community watershed team (CWT) and community facilitators will be established for the accomplishment of the implementation work.  Notice that the selection of this committee follows the PWDP guideline.

The woreda watershed team (WWT) will facilitate the organization of technical backstopping from the woreda, zone, region, the aggregation and compilation of data generated from the planning work, the timely request of inputs to sector offices and donors as applicable, and their possible allocation before the period of the year set for implementation. Besides, DAs and the local CWT will play a major role in maintaining a high level of participation during implementation. They will coordinate the efforts provided by the community and those of single individuals or target groups. Implementation involves converting the agreed plans into the actual on the ground. This is primarily the task of farmers and DAs who have full support from woreda and Zonal extension staffs, regional experts and Administration staffs and other partner organizations. Generally, implementation of this watershed management plan will be realized based on the country’s participatorywatershed development guidelines. The intervention should follow integrated approach and be able to support each other and produces positive environmental effect in the watershed area taking the watershed logics as main guiding principles in the implementation of the watershed management.

In general, during implementation:

1. Regional, zonal, woreda experts and DAs are responsible to propose and arrange training for land users before and during implementation based on local conditions and specific needs. Therefore, training proposals should be developed and forwarded to woredalevel which will provide the technical support.

2. DAs and woreda experts are responsible to follow-up trials and development of on-farm participatory technology for innovative measures to be tested in specific areas.

3. DAs and woreda experts will play a major role in strengthening the communication between the various sector agencies operating in the area by involving their experts and using their resources whenever required; for instance, education and health experts, resources, NGOs and others.

4. DAs and land users will also discuss the possible modifications that may occur to the plan during implementation.

# ORGANIZATION & MANAGEMENT

The organizational arrangements required for the watershed management will coincidewith the implementation strategies and institutional setups already defined in the Participatory Watershed Management Guide lines of the MoARD. Therefore, this plan will be integrated with current program as per the responsibility of existing government institutions.  In this regard, the primary responsible body for watershed management activities is the BoA. This institution will create a networking that would enable other institutions to involve in the implementation of the watershed management activities as per their responsibilities. Therefore, the BoAat woreda level will mobilize professionals making them coordinate activities in their area of forming working groups, who will be technical expertise to help in NRMand agricultural activities.  This will form a watershed management team that will be responsible in coordinating the planning, implementation and follow-up of the activities.

Furthermore, there should be integrationamong the various responsible institutions such as BoA, BWR&E, Land and Use Administration, Health and Education. These institutions will implement the planned activities as per their mandates and organizational responsibilities. For example, the bureau of Health will undertake family planning activities and so on.  The DA who have already assigned by the BoAwill be directly responsible in mobilizing the community, providing technical supports in the implementation of the planned activities.

At community level, a CM committee needs to be established to facilitate the implementations of the planned activities and ensure sustainability of the implemented activities. The committee should be composed of different community or social categories including kebeleofficials, youths, religion leaders, women and poor households. The main roles of the committee include working in collaboration with extension workers, mobilization and coordination of community tocarry out planned activities and follow up of the activities.

# MONITORING AND EVALUATION

Being an organized method, by which information is documented and available to all, planning is also an essential part of the monitoring and evaluation (M&E) system. The planned list of activities, targets, technical designs, reasons for selection, maps, &others, should be considered as benchmarks, which allow field staff to compare achievements and their impact against original purposes.

An effective PM&E thus requires a participatory planning approach. Participatory development is critical for achieving sound resource management in any watershed development or natural resources management. People who participate, are investing time and effort in an activity from which they hope to benefit, will need to be part of a continuing process of investigating how things are going, whether changes are needed, whether expected results are still realistic, whether new alternatives have become available, and the like. P&ME is an essential tool for financiers, government, local leaders, beneficiaries and other stakeholders. Regular supervision, as part of a monitoring and evaluation exercise, should be made at all levels of implementing agencies of watershed development program.

Benefits of PM&E:

- Increases consensus on project goals, objectives and activities

- Creates ownership over evaluation results

- Increases cost-effectiveness of M&E information

- Provides timely and reliable information for decision making

- Enhances learning by local stakeholders

- Enhances skills and confidence of local people on management of development projects

- Utilizes local knowledge.

Monitoring and evaluation system with the following characteristics should be developed for effective implementation of watershed development:

- Simple to apply

- Fully involves communities

- Should be consistent with already existing government system

- It should be applicable in theworeda

- Promotes accountability

- Should use existing data to the extent possible

- Should assist in re-planning and correction of failed interventions; should also assist in introducing new innovative activities.

## Monitoring

Monitoring is the collection of raw data or information for evaluation purposes. The monitoring system to be established will provide regular information on the use of inputs, participation and achievement on activities. Monitoring is a management tool which facilitates continuous learning and provides quality information on which to base evaluation.

Monitoring is the systematic recording and periodic analysis of information that has been chosen and recorded by insiders with the help of outsiders. The main purpose of monitoring is that it provides information during the life of the project, so that adjustments and/or modifications can be made if necessary.

Monitoring, to be successful, has to be participatory. This means that each stakeholder is involved in identifying the indicators and in measuring them. Participation will ensure that those indicators will be chosen which are meaningful to them. This means that the community, the experts and other stakeholders should undertake the review of the indicators jointly. Decision to make any modifications in the projectbeing implemented must also be taken jointly based on review.

## Evaluation

Evaluation is a process in which judgments on success and failures are made. Some evaluation can be made from regular result-based monitoring activities (process and outputs), but data can also be collected for evaluation purposes (outcomes and impact). Evaluation is where the learning occurs, questions answered, recommendations made and improvements suggested.

Evaluation measures the implementation of activities and how effectively this is done. It enables the stakeholders to develop a better understanding of the functioning of the program. In particular, it allows the stakeholders to understand the links between resource use, program activities, the intended and unintended immediate effects of those activities, the predetermined objectives, which are pursued, and the contribution of the program to some long-term vision.

Reporting: Regular reporting will be done on outputs achieved. The ‘foreman’ or community facilitator reports to the DA on outputs whenever there is an achievement. This could be daily or in longer intervals, depending on the season and the ongoing activities.

- The DA reports monthly on output achievement to Woreda Focal Person at the monthly Woreda meeting. The Woreda Focal Person will check on regular basis.

- The Woreda Focal Person summarizes output achievement on Woreda level and sends monthly, quarterly and annual reports to the Regional level.

# CONCLUSION

Conversion of forest, shrubs, and grass lands to agricultural land, cultivation of steep slopes and free grazing were commonly practiced activities observed in Williyo watershed. Besides, the majority of the watershed is under intensive cultivation of cereal crops that accelerated soil erosion. Grazing lands were highly trampled by livestock due to overstocking and greatly affected the physical structure of the soil. In addition, the average annual rainfall of the area, its rugged topography, soil characteristics, scarce vegetation cover, and inappropriate management practices are the main factors that contributed to soil erosion process in the Williyo watershed.

The rapidly growing number of population & rising demand for more food and agricultural land created pressure on land and its resources. These processes combined with poor farming system & the lack of appropriate soil and water conservation strategies caused the increase in the risk of floods and landslides, organic matter depletion, accelerated soil erosion, nutrient leaching and increased the amounts of sediment loads entering streams, rivers, reservoirs, and irrigation structures.

Because of the loss of protective vegetative cover and the gradual expansion of agricultural activities from gently sloping land onto the steeper slopes of the neighboring hills and mountainous areas, a large portion of the Williyo watershed is severely degraded and soil erosion is observed in various land use types in the watersheds.

Therefore, appropriate watershed management interventions are crucial in this study area to maintain and improve existing natural resources and thereby improve the livelihood of the community living in the watershed.

# References

Abdi B. Desalegn C. &Ekasit K. 2012. Simulation of sediment yield using SWAT model in Fincha watershed, Ethiopia.Kasetsart J. Nat. Sci. 46: 1 – 15, Thailand.

Bezuayehu, T. 2006. People and dams: environmental and socio-economic impacts of Fincha’a hydropower dam, western Ethiopia. Tropical Res. Manag.Kebeleper 75.Wageningen University and Research Center, Wageningen, The Netherlands.

BirhanuFenataw .2003 .Integrated watershed management Training manual, Ethiopia, Addis Ababa

Blaikie P.1985. Political economy of Soil Erosion in Developing Countries.Longman Development Studies, New York.

E.K. Biamha and K.Fahlston. 1997. Soil and water conservation manual for Kenya, Nairobi, Kenya

EHRS. 1988. Ethiopia High Land Reclamation Study

Escobedo, J.U. (1988). Land classification for Use in soil Conservation, Communities forests and soil conservation Department, MoA, Assistance to Soil and Water Conservation Programme phase II, Addis Ababa, Ethiopia.

Foster, G.R., and R.E. Highfill. 1983. Effect of terraces on soil loss: USLE P-factor values for terraces. J. Soil Water Conserv. 38:48-51.

Hilko years, Ann Mollema and Egger Topper .2002 .Erosion Control in the Tropics, Wageningen, The Netherlands

Gizachew .A .2003.  Integrated Watershed Management Training Manual, Ethiopia Addis Ababa

Javier Escobedo. 1990 .Soil and Land Classification guidelines

HanspeterLinjger& William Critchely.2007. Case studies and analysis of soil and water conservation initiatives world wide

Hudson N. 1981.Soil conservation.B.T. Batsford Limited, London, UK.324 pp.

Hurni, H. 1990. Degradation and conservation of soil resources in the Ethiopian highlands. Mountain research and development 8 (2-3): 123-130, University of Bern, Switzerland.

Hurni, H. 1993. Land degradation, famine, and land resource scenarios in Ethiopia. In: Pimental D. (ed) World soil erosion and conservation. Cambridge Univ. Press.

Javier Escobedo. 1990 .Soil and Land Classification guidelines

Lakew Desta.2006 .Soil and Water conservation training Manual, Ethiopia, Addis Ababa

MoWR (1999). Data Collection, Site Investigation, Survey and Analysis, AbayDawa  River Basin Integrated Development Master Plan Project, Vol. XIII, Part 1, MoWR, Addis Ababa.

MoARD.2000. Participatory Watershed Management Planning Guideline Part I and II, Ethiopia, Addis Ababa

MoARD (2004). Woody Biomass Inventory and Strategic Planning Project, A national strategic plan for Biomass energy sector, final report, Addis Ababa, Ethiopia.

MoARD (2005). Communities Based Participatory Watershed Development: A Guideline. Part 1, first edition, Addis Ababa, Ethiopia

Prof. Tuffa. 2000.  Soil and Water Conservation for Sustainable Agriculture

Renard, K.G., Foster, G.R., Weesies, G.A., McCool, D.K. and Yoder, D.C., (1997). Predicting soil erosion by water: a guide to conservation planning with the Revised Universal Soil Loss Equation (RUSLE). Agriculture Handbook No. 703. Agricultural Res. Service, Washington, DC

Saavedra, C., (2005). Estimating spatial patterns of soil erosion and deposition in the Andean region using geo-information techniques: a case study in Cochabamba, Bolivia Ph.D. dissertation, Wageningen University, The Netherlands.

Shi, Z.H., Cali, C.F., Ding, S.W., Wang, T.W. and Chow, T.L., (2003). Soil conservation planning at the small watershed level using RUSLE with GIS: a case study in the Three Gorge Area of China. Catena 55, 33–48.

Sileshi Getahun.2005.  Integrated Watershed Management Training Manual, Ethiopia, Addis Ababa

S.K.GARG.2005. Hydrology and water Resources Engineering

Sutcliffe JP. 1993. Economic assessment of land degradation in the Ethiopian highlands: A case study. National Conservation Strategy Secretariat, Ministry of Planning and Economic Development, Addis Ababa.

T.C. Sheng.1990.The FAO Watershed Management Field Manual conservation guideline 13

V. Caruchi .2000. Guideline for SWC and Water Harvesting for moisture deficit areas in Ethiopia

Universal Soil Loss Equation (RUSLE). Agriculture Handbook No. 703. Agricultural Research Service, Washington, DC.

WWDSE (2011). Welmel Irrigation project feasibility study, Addis Ababa