Amhara National Regional State

Water Resources Development Bureau

(BOWRD)

Feasibility Study and Detail Design

Of

Workie Diversion/Weir Small -Scale Irrigation Project

Volume IV: Engineering Design

Final Report

January, 2018

Dessie, Ethiopia

Client: Bureau of Water Resource Development

(BoWRD)

Address:

P. O. Box: 88
Telephone: 0528-200853/855
Fax: 251-08-20-65-68/204676/202040

Consultant: Amhara Design & Supervision Works Enterprise Eastern Amhara Branch Office

(E/ADSWE)

Address:

P. O. Box: 4921
Telephone: +251-333-124954

Fax: (033) 3124954
E-mail: amhara design@yahoo.com

Dessie, Ethiopia

# FEASIBILITY STUDY & DETAIL DESIGN REPORT STRUCTURE

- Volume I: Watershed Management

- Volume II: Engineering Geology

- Volume III: Irrigation Agronomy

- Volume IV: Engineering Design

- Volume V: Socio Economy

- Volume VI: Environmental Impact Assessment

Table of Contents								 Page Nr

FEASIBILITY STUDY & DETAIL DESIGN REPORT STRUCTURE	i

LIST OF TABLES	v

LIST OF FIGURES	vi

SAILENT FEATURE	vii

1	INTRODUCTION	1

1.1	Background	1

1.1.1	Description of the Project Area	1

1.2	Objectives of the Study	3

1.2.1	Major Objective	3

1.2.2	Specific Objectives	3

1.3	Scope of the Study	4

1.4	Methodology	5

SECTION-I:  HYDROLOGY	7

2	HYDROLOGY	8

2.1	Watershed Characteristics	8

2.2	Hydro-Metrological Data Availability	9

2.2.1	Climate	9

2.2.2	Rainfall Data	10

2.2.3	River flow data	10

2.2.4	Upstream & Downstream utilization	10

2.3	Design Flood Analysis	10

2.3.1	Design Rainfall computation	10

2.3.2	Outlier Test	11

2.3.3	Check for variance	12

2.3.4	Peak Discharge Determination	14

2.3.5	Tail Water Depth Computation	18

SECTION-II: HEADWORK DESIGN	22

3	HEADWORK STRUCTURES DESIGN	23

3.1	Headwork Site Selection	23

3.2	River Geomorphology	23

3.2.1	River Bed condition	24

3.2.2	River Bank condition	25

3.3	Sources of construction materials	27

3.3.1	Rock for Masonry and Crushed Coarse Aggregate	27

3.3.2	Fine Aggregates	28

3.3.3	Water	28

3.4	Headwork Type Selection	28

3.4.1	Hydraulic Design of Headwork Structure	29

3.4.2	U/S and D/S HFL Calculation & Determination	30

3.4.3	Hydraulic Jump Calculation	31

3.4.4	Impervious floor	32

3.4.5	Cut off Depth Calculation	34

3.4.6	Stability Analysis of weir	36

3.5	Bill of Quantity and Cost Estimation	41

SECTION-III: IRRIGATION AND DRAINAGE SYSTEMS INFRASTRUCTURE	46

4	IRRIGATION AND DRAINAGE SYSTEMS DESIGN	47

4.1	Irrigable Area Description	47

4.1.1	Topography	47

4.1.2	Climate	47

4.1.3	Soil characteristics	48

4.1.4	Existing Irrigation Practices in the Project Area	48

4.2	Irrigation Water Requirement	49

4.2.1	Crop Water Requirement (CWR)	49

4.2.2	Irrigation efficiency (Ep)	49

4.2.3	Irrigation duty	50

4.2.4	Irrigation methods	51

4.3	Irrigation and Drainage System Layout	52

4.3.1	Conveyance System	53

4.4	Design of the Canal System	53

4.4.1	Main Canal	53

4.4.2	Secondary HDPE Pipe	54

4.4.3	Tertiary HDPE Pipe	55

4.4.4	Field Canals	58

4.5	Canal Structures Design	58

4.5.1	Design of a typical flume	58

4.5.2	Design of Division boxes	59

4.5.3	Design of field canal turnout	61

4.5.4	Road crossing structure	62

4.6	Irrigation Infrastructure Bill of Quantities and Cost Estimate	63

5	CONCLUSION AND RECOMMENDATION	69

6	OPERATION AND MAINTENANCE	70

6.1	General	70

6.2	Operation of the Head Works	70

6.3	Irrigation System Operation	70

6.4	Maintenance Requirement	71

REFERENCE	72

# LIST OF TABLES

Table 2-1: Outlier test analysis	11

Table 2-2: Test for goodness to fit using D-index	13

Table 2-3: Determination of Time of Concentration	14

Table 2-4: Runoff analysis	16

Table 2-5: Hydrograph coordinates	17

Table 2-6: Weir Diversion Site River Cross section Coordinate Data	18

Table 2-7: Stage discharge analysis	18

Table 2-8: River discharge computation at different stages of flow	21

Table 4-1: Hydraulic Parameters of main canal	54

Table 4-2: Hydraulic Parameters of secondary HDPE pipe	55

Table 4-3: Hydraulic Parameters all tertiary HDPE Pipe	56

Table 4-4: Hydraulic Parameters of Flume	59

Table 4-5: Hydraulic parameters of Division Boxes	60

# LIST OF FIGURES

Figure 1-1: Location map of the project area	2

Figure 2-1: Drainage Map of Workie watershed	9

Figure 2-2: Complex Hydrograph	17

Figure 2-3: Rating Curve	19

Figure 2-4: River profile	20

Figure 3-1: River bed at the proposed Weir Diversion site	25

Figure 3-2: River bed geological x-section	26

Figure 3-3: weir section	35

Figure 3-4 :Weir stability	36

Figure 3-5: Typical design of masonry retaining wall	40

Figure 4-1: Typical Field Canal X-section	58

Figure 4-2: Typical Division Box section	59

Figure 4-3: turnouts from masonry lined	61

Figure 4-4: typical road crossing section	62

# SAILENT FEATURE

- Project name: Workie Diversion/Weir Small scale Irrigation Project

- Name of the stream: Workie river

- Location of the Weir Diversion site using

- North:1189752.820m

- East:   598864.713m

- Zone: Kemissie Oromia zone

- Average Altitude: 1689.181 masl

- Hydrology

- Design rainfall: 104.48 mm

- Catchment area: 6.33 Km2

- Longest flow path length: 3.89 Km

- Design flood: 32.76 m3/se by flood mark

- Design base flow: 260 lit/se.

- Weir Diversion

- Weir type: Ogee Weir with Cyclopean Concrete

- Height: 2.1m

- Gross crest length: 5 m

- Weir crest level: 1687.5 m.a.s.l

- U/S HFL: 1689.415 m.a.s.l

- U/S TEL: 1689.549 m.a.s.l

- D/s TEL: 1689.349.a.s.l.

- D/s HFL: 1687.876 m.a.s.l

- Afflux: 1.54 m

- Silt Excluder

- Sill level: 1686.8 m.a.s.l

- Dimension: 0.7*0.7 m2

- Outlet

- Sill level: 1687.05 m.a.s.l

- Opening dimension: 0.7*0.7 m2

- Discharge capacity: 223.6 lit/sec.

Irrigation and drainage systems Infrastructure

- Command area size: 215 ha

- Type of soil of the command area is dominantly clay loam soil

- Design discharge of the main canal = 223.6 l/sec

- Irrigation system layout consists of 1 RCC Lined  main canal, 3 secondary HDPE Pipe  and 30 tertiary HDPE pipe network system

- Main irrigation structures designed are;

- Gully crossing structures, such as, Flume& Supper passage structure

- Road crossing structure

- division box, and turn out

Project cost

| WORKIE IRRIGATION PROJECT |  |  |
| --- | --- | --- |
| SUMMARY OF BILLS |  |  |
| Bill No. | Description | Amount (Birr) |
| 1 | General Items | 2,052,799.10 |
| 2 | Head work | 1,382,498.07 |
| 3 | Infrastructure | 19,279,870.89 |
| Total |  | 22,715,168.06 |
| VAT(15%) |  | 3,407,275.21 |
| Grand Total with VAT |  | 26,122,443.27 |
| Command Area(ha) |  | 215 |
| Per hactare Cost with out VAT |  | 105,651.94 |
| Per hactare Cost with VAT |  | 121,499.74 |

# INTRODUCTION

## Background

In Ethiopia, under the prevalent rain-fed agricultural production system, the progressive degradation of the natural resource base, especially in highly vulnerable areas of the highlands coupled with climate variability have aggravated the incidence of poverty and food insecurity. The major source of growth for Ethiopia is still conceived to be the agriculture sector. Hence, this sector has to be insulated from drought shocks through enhanced utilization of the water resource potential of the country, (through development of small-scale irrigation, water harvesting, and on-farm diversification) coupled with strengthened linkages between agriculture and industry (agro-industry), thereby creating a demand for agricultural output. In line with the above, efforts have been made by the government to improve the situation in the country in areas of domestic water supply provision, irrigation, watershed management; etc. The Amhara Water Resources Development Bureau is playing its role in the development of small scale irrigation projects in the region. Accordingly, as part of the water sector development program, the office has initiated the study and design of a Small small scale irrigation scheme on Workie River at  Kebele and signed an agreement with Amhara Design & Supervision Works Enterprise (ADSWE) for the study and design of the project.

### Description of the Project Area

#### Location

This irrigation project is located mainly at Didini Kebele, Dewa Chefa Wereda of Oromia Zone in the Amhara Region. The proposed irrigation project is to be undertaken on Workie River and the headwork structures are specifically located at an altitude of about 1689.181 masl and geographical coordinates of 1189752.820 N (UTM) and 598864.713 E (UTM).

Figure 1-1: Location map of the project area

#### Accessibility

The Workie Irrigation project headwork site is located in the National State of the Amhara Region, Oromia zone, Dewa Chefa Woreda, in Didini kebele. The geographic co-ordinates of the site are defined by the UTM location of 1189791mN, 598909mE and river bed elevation of 1664m above mean sea level.

The Head work project site is 12km from Kemissie to north-East. Out of this distance, 8km is all weather gravel road and turning to the right side the 4km is in accessible till the headwork site. There for, the access road it needs Construct temporary access road to site which includes Cut the hilly terrain, fill the Gorgy area, boulder excavation, hard rock Excavation and highly Site clearing. UN less the work methodology is difficult.

#### Previous Irrigation Practices

There are traditional diversions on the upstream/downstream of this river using different irrigation practices but as the hydrology and Hydrogeology study and respondent farmers indicated, the river has capacity of recharging as it stretches down from the source area of the river. As a result there will not be a marked reduction or fluctuation of water flows both for the already existing and the newly proposed irrigation schemes. The traditional irrigation practices (if any) are under taken by individual farmers that use the river flow to the extreme Right side is with hardship. So, the farmers in the project area are very much interested to upgrading the traditional scheme to modern scheme.

## Objectives of the Study

### Major Objective

The project area faces variability of rainfall distribution though the overall rainfall generally suffices the rain-fed agriculture.Accordingly, the rain-fed agriculture needs means of supplementing during distribution failures and further full irrigation is required to maximize the use of the potential land and water resources.

Hence the objective of this project is to contribute a substantial share in the effort to reduce the risk of production decrease due to rainfall variability and increase the productivity of the resource in the project specific area. Specifically, the project is targeted for the following.

- To make sustainable the rain-fed crop production and make extra production in the dry season possible for 215ha of land through irrigation.

- There is a general consensus that irrigation investments will achieve broader food security and poverty reduction impacts and if efforts are also geared towards up-grading existing traditional farming practices with support to enhance access to input supply, output marketing and extension to facilitate access to information and innovations.

- This objective is to be realized by constructing Weir Diversion structures across the Workie River and diverting the river flow.

### Specific Objectives

Other benefits that can be expected to appear with the launching of the project are:

- Efficiency of water use improvement;

- Improved local nutrition/food security gains;

- Improved management of scarce natural resources (land and water);

- Resilience against drought risk;

- Rationale for erosion control and watershed management;

- Rationale for the intensification and modernization of small-holder agriculture and rural lifestyles.

The engineering study and design enables the realization of the project by the provision of engineering structures that will allow the appropriate abstraction of the river water for delivery in to the identified irrigation fields of the study area. Hence, this engineering design is specifically targeted to:

- Analyze hydrologic requirements of the project and engineering structures;

- The formulation of sound and stable structure,  with necessary provisions that allow safe, easy and low-maintenance operation in the service life of the project;

- Develop working drawings;

- Estimation of construction costs.

## Scope of the Study

- The irrigation design shall ensure reliability, equity and flexibility of water delivery to farmers. It will aim at reducing conflicts among water users and will lead to lower operation and maintenance costs.

- Updating the existing, if available, computation of the actual evapo-transpiration, crop water requirement, irrigation demand/duty using the existing and recent agronomic, climatologic and soil data using more appropriate methodologies.

- Establish design criteria for irrigations structures to be approved by the client and to be used in the final design stage,

- Design proper irrigation system compatible with local conditions and management capabilities,

- Establish flood protection measures for the command area and canal structures and design the respective drainage system accordingly,

- Planning  and layout of the irrigation system, which include irrigation canals, drainage channels, inspection roads and alignments, canal spacing, canal length, location of structures, and water profiles along canal and drains at specified reaches, which is most economical easily manageable and aligned with topographic feature and geological investigation.

- Determination and estimation of water application conveyance and other losses and irrigation efficiencies and consideration of those parameters in design steps.

- Check and test hydraulic and structural designs of main canal considering total demand and the required capacity and the base flow availability,

- Prepare general plans and drawings for all irrigation infrastructure and irrigation systems designs,

## Methodology

In the study and design procedure, Designers used the following steps.

- Specific Site identification:

  - Review of the reconnaissance survey conducted by the Client

  - 50,000 scale top map and GIS information

  - Local farmers interview and discussion

  - Wereda and Zone Agriculture section expertise

  - Previous studies

  - On foot travel along the river channel and farm areas.

- Topographic survey:

  - Surveying the headwork site and the Command area with sufficient radius, using Total station

- Flow estimation

  - Physical observation on flood mark indications and local information about high flood and critical flow condition of the river

  - Analyzing the recorded river flow data and use watershed inputs for further analysis.

  - Base flow estimated during the reconnaissance field visit by floating method.

- Irrigable area identification:

  - Using local information

  - 50,000 Topographic map, and GIS information, GPS to see elevation

The design report is organized in three sections. In Section I the Hydrology study is presented and in Sections II and III the Headwork and Irrigation and Drainage Systems designs are discussed respectively. In Section III, planning and design of the irrigation system after diverting the water using the Weir Diversion will be dealt. The following are major areas of concern in this part.

- Study and design of the irrigation method to be adopted,

- Study and design of the irrigation system layout and associated structures,

- Design of the different conveyance canals,

- Planning and design of the different irrigation and drainage structures,

- Preparation of the longitudinal profiles of the different irrigation and drainage canals.

# SECTION-I:  HYDROLOGY

# HYDROLOGY

## Watershed Characteristics

The watershed is found within 1698 to 2240 meters above sea level altitudinal range. The watershed has marked topographic variation. The dominant slope class is moderately steep (15-30%) which covers 46.24% of the total area followed by steep slope (30-50%), which is 32.51%.Sloping (8-15%), very Steep (>50%) and gently sloping (3-8%) which accounts 10.19%, 8.94% and 2.12% respectively. Table 1 of the watershed feasibility study report shows the slope classes and proportion of the watershed.

Certain physical properties of watersheds significantly affect the characteristics of the runoff and sediment yield and are of great interest in hydrologic analyses. The rate and volume of runoff, and sediment yield from the watershed have much to do with shape, size, slope and other parameters of the landscape. These suggest that there should be some important relations between basin form and hydrologic performance. If the basin and hydrologic characteristics are to be related, the basin form must also be represented by quantitative descriptors. These parameters can be measured from maps.

The watershed characteristics are analyzed and presented in Table 2 of the Watershed Feasibility Study Report of the same project. In summary:

- Catchment Area  = 6.33km2

- Stream Length = 3.89Km

- CN(II) = 88.41

(Extracted from the Watershed Study Report of the same project)

At the selected reference point, the area of Workie catchment is 6.33km2 and consists of a network of tributaries as shown in Figure 1 below.

Workie River at the headwork site is characterized by well-defined channel system and considerable flows. It looks that the gradient of the river/stream is getting low and hence there exists significant deposition of sediment mainly cobbles and boulders.

Figure 2-1: Drainage Map of Workie watershed

## Hydro-Metrological Data Availability

### Climate

Small scale irrigation project designers and planners are faced with lack of good data on the hydrology of the river system that will be their water source and on local weather and climate conditions. Stream gauging stations are virtually non-existent in remote rural areas of Ethiopia; meteorological stations are almost rare. Likewise, at Dindi Kebele (Project area location) and in the catchment area of this project, there is no meteorological station of any level. Moreover, there are no flow data for the river near the project. Therefore, data for the hydro-meteorological analysis is taken from the nearby station and similar areas. Rainfall & temperature data are considered from Kemissie Meteorological station. In fact, this station is very close to the project area.

.The average of annual rainfall of the area is calculated based on 20 years record of the station and is equal to 921.585mm/yr. The annual average minimum and maximum temperature is about observed 16.54oc and 28.16oc respectively. The mean annual temperature of the watershed is about 20.250c.

### Rainfall Data

In order to compute the design flood for the Weir Diversion structure, the daily maximum rainfall is collected from Kemissie Metrological stations with a record of 22 years.

### River flow data

The base flow which is measured on May 2015/16 is 260 l/s. Since this base flow is measured during the dry months of the year, this figure is adopted for design.

### Upstream & Downstream utilization

The water distribution of Workie River is for both existing irrigation system at proposed site itself and downstream of the proposed site in left side of river there is irrigated command area which is constructed by World vision, and the method of irrigation is rotational scheme irrigation system is used hence, from the previous Experience Workie Weir Diversion Irrigation project is design rotational scheme irrigation system.

For the sake of planning and design, however, the outlet for the Weir Diversion is designed for a discharge of 223.6/s for this project and the project is to be developed for 215 ha of land, which is most of the time achievable as the flow for most of the time is significant to support this size of command area.

## Design Flood Analysis

For the design and analysis of structures to be constructed on the river, estimation of flood magnitude is an important task. This can be done using different techniques depending on the data available. For this particular case, there is no river flow data and hence the flood estimation is done using the rainfall data and applying SCS Curve Method.

### Design Rainfall computation

Based on the data of 24hr peak rainfall given in Table 1 the design rainfall, Rf is computed using Gumble’s Extreme Value Method.

### Outlier Test

Checking data Quality

Let Y=log X

Lowest Data RL=10^YL

Highest Data RH=10^YH, Where, Higher and lower limit YL=Ymean -Kn*syn-1 & YH=Ymean +Kn*syn-1

Ymean =1.763

Syn-1=logarithmic standard deviation= 0.109, Kn =2.429 for 22 record data

YL=1.498 & YH=2.028

Therefore, RL=31.5mm <3 5.5mm is Ok and RH=106.715mm > 84.2mm is Ok

As we observed from the above result the data is within the higher and lower outliers is safe

Table 2-1: Outlier test analysis

| Kemissie max daily rainfall |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| No | year | Max rain fall of year(X) | Descending order (x) | Y=logx | cumulative rainfall |
| 1 | 1990 | 38 | 84.2 | 1.925 | 38 |
| 2 | 1991 | 36 | 81.9 | 1.913 | 73.5 |
| 3 | 1992 | 42 | 81 | 1.908 | 115.5 |
| 4 | 1993 | 50 | 72.6 | 1.861 | 165.2 |
| 5 | 1994 | 67 | 72.5 | 1.860 | 232.2 |
| 6 | 1995 | 66 | 69.8 | 1.844 | 298.2 |
| 7 | 1996 | 84 | 67 | 1.826 | 382.4 |
| 8 | 1997 | 60 | 66 | 1.820 | 441.9 |
| 9 | 1998 | 70 | 63.9 | 1.806 | 511.7 |
| 10 | 1999 | 60 | 62 | 1.792 | 571.4 |
| 11 | 2000 | 81 | 60.4 | 1.781 | 652.4 |
| 12 | 2001 | 73 | 59.7 | 1.776 | 724.9 |
| 13 | 2002 | 64 | 59.5 | 1.775 | 788.8 |
| 14 | 2003 | 58 | 58 | 1.763 | 846.8 |
| 15 | 2004 | 73 | 53.9 | 1.732 | 919.4 |
| 16 | 2005 | 62 | 50 | 1.699 | 981.4 |
| 17 | 2006 | 54 | 49.7 | 1.696 | 1035.3 |
| 18 | 2007 | 45 | 44.6 | 1.649 | 1079.9 |
| 19 | 2008 | 82 | 42 | 1.623 | 1161.8 |
| 20 | 2009 | 50 | 40.9 | 1.612 | 1211.8 |
| 21 | 2010 | 60 | 38 | 1.580 | 1272.2 |
| 22 | 2011 | 41 | 35.5 | 1.550 | 1313.1 |
| sum |  | 1313.1 |  | 38.79191314 |  |
| mean/µ/ |  | 59.686 |  | 1.763 |  |
| stdv/αn-1/ |  | 14.272 |  | 0.109 |  |
| skew/g/ |  | -0.012 |  | -0.426 |  |

### Check for variance

After checking the outliers, the data should be checked for variability. For variability the formula used is          α=δ∩-1N*Mean*100%

Where, δn-1 = Standard deviation =14.27

N = Nr of recorded data =22

Mean = 59.686 and α = Standard error

α=14.27222*59.686*100%=5.10<10%  Acceptable, Therefore the data shows no variability.

#### D-Index test

After checking the consistency of the data for higher and lower outlier, the 22 years data is obtained as representative for the analysis using D-index. The D-Index test is believed to be the better goodness to fitness in many literatures. Hence in this study it was used to determine the best statistical distribution to estimate the peak rainfall. The D-index for the comparison of the fit of various distributions is summarized as follows.

D-Index=(1Xm)*i=16Abs(Xi-Xi')

Where Xi and Xi’ are the ith highest observed and computed values for the distribution respectively

Table 2-2: Test for goodness to fit using D-index

| Rank | XI' | Normal | GUMBLE EVI | Log normal | Log person Type III |
| --- | --- | --- | --- | --- | --- |
|  |  | XI-'XI' | XI-'XI' | XI-'XI' | XI-'XI' |
| 1 | 84.200 | 0.079 | 9.813 | 4.930 | 1.739 |
| 2 | 81.900 | 2.803 | 2.609 | 2.614 | 1.724 |
| 3 | 81.000 | 5.265 | 2.190 | 5.001 | 4.632 |
| 4 | 72.600 | 0.485 | 2.061 | 0.795 | 0.801 |
| 5 | 72.500 | 1.669 | 1.146 | 1.959 | 1.595 |
| 6 | 69.800 | 0.973 | 1.227 | 1.705 | 1.092 |
| Sum |  | 37.14 | 55.729 | 55.141 | 41.173 |
| D-index=sum/mean |  | 0.622 | 0.934 | 0.924 | 0.690 |

All the candidate distributions give almost identical correlation coefficients. However, the standard errors are significantly lower for the Normal Distribution Method which is 0.622. Accordingly, the design rain for this distribution has been selected as the best fit for this study

- But, Gumbell Peak RF is taken for further analysis because it gives as maximum RF hence to minimize Risk and  also the method is widely used method  and structurally safe

The design rainfall using Gumbell Method is given as

Where RF = Design rainfall

Rmean = average of all values of annual heaviest fall = 59.686 mm

σn-1 =  standard deviation of the series = 14.272 mm

, T= Return period = 50 years

Yn, Sn = constant found from Gumble’s extreme value distribution table for N= 22Years

- Yn = 0.5268 and Sn = 1.0754

- Point Design Rainfall = 104.478 mm

The design rainfall at points for 50 years return period is 104.48 mm and the areal design rainfall is calculated in the following section.

### Peak Discharge Determination

#### General

The River is not gauged river. The design flood is calculated by using SCS unit hydrograph method. Thus, it is preferred to base the flood analysis on rainfall data, which are better both in quantity and quality of data. In the hydrologic analysis for drainage structures, it must be recognized that there are many variable factors that affect floods. Some of the factors that need be recognized and considered on an individual site by site basis are; rainfall amount and storm distribution; catchment area, shape and orientation; ground cover; type of soil; slopes of terrain and stream(S); antecedent moisture condition; Storage potential (over bank, ponds, wetlands, reservoirs, channel, etc.)

#### Peak flood analysis by SCS unit hydrograph method

Design flood is calculated SCS (The United States Soil Conservation Service). This method is widely adopted and more reliable method for flood estimation. The approach considers, watershed parameters, like Area, Curve number, and time of concentration.

#### Time of concentration (Tc)

Time of concentration has been calculated by taking the stream profile of the longest streamline and dividing it in to different elevation. Kirpich formula is adopted for computation.

Table 2-3: Determination of Time of Concentration

| Partial Distance/km/ | cumulative distance/km/ | Elevation/m/ | Elevation diff./meter | TC/hr |
| --- | --- | --- | --- | --- |
| 0 | 0 | 2240 | 0 |  |
| 0.41 | 0.41 | 2100 | 140 | 0.05 |
| 1.00 | 1.41 | 1960 | 140 | 0.15 |
| 1.58 | 2.99 | 1760 | 200 | 0.22 |
| 0.90 | 3.89 | 1698 | 62 | 0.18 |
|  |  |  | TC | 0.59 |

The formula is,

Tc=0.948L13H10.385+L23H20.385+…+Ln3Hn0.385

- Tc = 0.59	Since Tc <3hr., duration of excess rainfall difference, D = 0.1hr.

- Time to peak,

Tp=D2+0.6*Tc  = 0.41hr

- Base time,

Tb=2.67*Tp  = 1.08hr

- Recession time,

Tr=1.67*Tp  = 0.68hr.

#### Curve number (CN)

Curve number (CN) is achieved based on USSCS method by watershed characterization in terms of land cover, treatment, hydrologic condition and soil group. From the watershed analysis curve number at condition II =75.56. Since peak rainfall is found at an antecedent moisture condition III state, this value has to be changed to antecedent moisture condition III.

- Conversion factor = 1.17

- CN Condition (III) = (Factor from Table x CN condition II) =75.56*1.170 = 88.41. For detail analysis of the computation, Refer Excel file, attached here with.

#### Area Rainfall

As the area of the catchment gets larger, coincidence of all hydrological incidences becomes less and less. This can be optimized by changing the calculated point rainfall to aerial rainfall. The conversion factor is taken from standard table that relate directly with the size of watershed area and type of the gauging station. (IDD manual)

For the case of Workie irrigation project,

- Total watershed area = 6.33 Km2

- Type of gauging station = Daily rainfall (24 hr.)

- Aerial Rainfall = (Point Rainfall) x (Conversion factor)

#### Run off Analysis

Input data:

Design Point Rainfall = 104.478mm

Curve number at antecedent moisture condition III = 88.41

Catchment Area, A = 6.33 Km2

Tc = 0.59hr, D = 0.1hr., Tp = 0.41 hr; Tb = 1.08 hr; Tr = 0.68 hr.

Direct run-off,

- Q=(I-0.2*S)2(I+0.8*S)

- Where, I = Rearranged cumulative run-off depth (mm

S = Maximum run off potential difference,

- S=25400CN-254

Peak run-off for incremental;

- Qp=0.21*(A*Q)Tp

- Where,   A = Catchment area = 6.33 Km2

- Tp = Time to peak (hr)

- Q = Incremental run-off (mm)

Table 2-4: Runoff analysis

| Rainfall Duration(D) | Direct runoff increment, Ri(mm) | qp for 1mm runoff /m3/s/mm | qp for incremental run off m3/s/ | begin time, hour | peak time, hour | end time, hour | remarks |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 0-0.1 | 0.00 | 3.27 | 0.00 | 0 | 0.41 | 1.09 | h1 |
| 0.1-0.2 | 0.03 | 3.27 | 0.10 | 0.1 | 0.51 | 1.19 | h2 |
| 0.2-0.3 | 0.94 | 3.27 | 3.1 | 0.2 | 0.61 | 1.29 | h3 |
| 0.3-0.4 | 3.64 | 3.27 | 11.9 | 0.3 | 0.71 | 1.39 | h4 |
| 0.4-0.5 | 3.04 | 3.27 | 9.9 | 0.4 | 0.81 | 1.49 | h5 |
| 0.5-0.6 | 2.19 | 3.27 | 7.18 | 0.5 | 0.91 | 1.59 | h6 |

Table 2-5: Hydrograph coordinates

| Time | Q1 | Q2 | Q3 | Q4 | Q5 | Q6 | Base flow(m^3/se) | Qtotal(m^3/se) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0.0 | 0.00 |  |  |  |  |  | 0.26 | 0.26 |
| 0.1 | 0.00 | 0.00 |  |  |  |  | 0.26 | 0.26 |
| 0.2 | 0.00 | 0.02 | 0.00 |  |  |  | 0.26 | 0.28 |
| 0.3 | 0.00 | 0.04 | 0.51 | 0.00 |  |  | 0.26 | 0.81 |
| 0.4 | 0.00 | 0.06 | 1.02 | 1.68 | 0.00 |  | 0.26 | 3.02 |
| 0.41 | 0.00 | 0.06 | 1.05 | 1.79 | 0.08 |  | 0.26 | 3.25 |
| 0.50 | 0.00 | 0.08 | 1.53 | 3.37 | 1.23 | 0.00 | 0.26 | 6.47 |
| 0.51 | 0.00 | 0.10 | 1.56 | 3.48 | 1.31 | 0.05 | 0.26 | 6.76 |
| 0.61 | 0.00 | 0.00 | 3.11 | 5.16 | 2.54 | 0.84 | 0.26 | 11.91 |
| 0.71 | 0.00 | 0.00 | 1.77 | 11.95 | 3.77 | 1.63 | 0.26 | 19.38 |
| 0.81 | 0.00 | 0.00 | 1.46 | 5.83 | 9.95 | 2.42 | 0.26 | 19.93 |
| 0.91 | 0.00 | 0.00 | 1.16 | 4.83 | 4.26 | 7.19 | 0.26 | 17.69 |
| 1.09 | 0.00 | 0.00 | 0.61 | 3.02 | 2.94 | 2.36 | 0.26 | 9.20 |
| 1.19 |  | 0.00 | 0.31 | 2.02 | 2.21 | 1.89 | 0.26 | 6.68 |
| 1.29 |  |  | 0.00 | 1.01 | 1.47 | 1.42 | 0.26 | 4.16 |
| 1.39 |  |  |  | 0.00 | 0.74 | 0.95 | 0.26 | 1.94 |
| 1.49 |  |  |  |  | 0.00 | 0.47 | 0.26 | 0.73 |
| 1.59 |  |  |  |  |  | 0.00 | 0.26 | 0.26 |

Figure 2-2: Complex Hydrograph

- From the analysis, the 50 year return period design run off is 19.88 m 3/s

### Tail Water Depth Computation

Tail water depth of the river is equal to the flood depth and amount at the proposed Weir Diversion site before construction of the Weir Diversion. It is used to crosscheck peak flood estimated by the SCS unit hydrograph method with flood mark method and to see the flood feature after the hydraulic jump. During field visit, the flood mark of the river at the proposed diversion Weir Diversion site was marked based on dwellers information and physical indicative marks. The river cross-section was surveyed.

Table 2-6: Weir Diversion Site River Cross section Coordinate Data

| Easting | Northing | Elevation | remark | Partial Distance | Chain age |
| --- | --- | --- | --- | --- | --- |
| 598877.627 | 1189809.476 | 1689.268 | River center | 0.00 | 0.00 |
| 598882.152 | 1189800.295 | 1688.388 | River center | 10.24 | 10.24 |
| 598886.330 | 1189790.955 | 1687.828 | River center | 10.23 | 20.47 |
| 598885.316 | 1189781.356 | 1687.428 | River center | 9.65 | 30.12 |
| 598878.501 | 1189773.715 | 1686.714 | River center | 10.24 | 40.36 |
| 598871.751 | 1189766.175 | 1686.006 | River center | 10.12 | 50.48 |
| 598870.733 | 1189756.038 | 1686.099 | River center | 10.19 | 60.67 |
| 598869.302 | 1189745.941 | 1685.758 | River center | 10.20 | 70.86 |
| 598866.537 | 1189736.092 | 1684.942 | River center | 10.23 | 81.09 |
| 598862.075 | 1189726.872 | 1684.800 | River center | 10.24 | 91.34 |
| 598857.550 | 1189717.682 | 1684.267 | River center | 10.24 | 101.58 |
| 598852.851 | 1189708.580 | 1683.907 | River center | 10.24 | 111.82 |
| 598846.969 | 1189700.255 | 1683.564 | River center | 10.19 | 122.02 |

Table 2-7: Stage discharge analysis

| S.Nr | Elevation/m/ | Water depth /m/ | Wetted Area (A), m^2 | Perimeter /m/ | Hydraulics radius/R/ | Velocity /m/s/ | Discharge /m^3/s/ | Remark |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1685.38 | 0 | 0 | 0.000 | 0 | 0 | 0 | RBL elevation from topo |
| 2 | 1685.68 | 0.3 | 0.09 | 0.806 | 0.11 | 1.347 | 0.11 |  |
| 3 | 1685.98 | 0.6 | 0.34 | 1.684 | 0.20 | 2.077 | 0.71 |  |
| 4 | 1686.28 | 0.9 | 0.77 | 2.512 | 0.31 | 2.743 | 2.11 |  |
| 5 | 1686.58 | 1.2 | 1.37 | 3.358 | 0.41 | 3.319 | 4.55 |  |
| 6 | 1686.88 | 1.5 | 2.14 | 4.185 | 0.51 | 3.854 | 8.23 |  |
| 7 | 1687.18 | 1.8 | 3.08 | 5.077 | 0.61 | 4.325 | 13.33 |  |
| 8 | 1687.38 | 2.00 | 4.25 | 6.286 | 0.68 | 4.648 | 19.75 | Peak Discharge |
| 9 | 1687.48 | 2.10 | 4.23 | 5.993 | 0.71 | 4.785 | 20.25 |  |
| 10 | 1687.88 | 2.50 | 6.09 | 7.248 | 0.84 | 5.38 | 32.76 | Flood mark |

Figure 2-3: Rating Curve

From the above stage discharge curve the maximum flood level corresponding to the computed design peak discharge is 1687.88 (2.5 m from the river bed) and it is considered as the d/s high flood level i.e. expected at the Weir Diversion axis before construction of the Weir Diversion.

D/S HFL = 1687.88 m.a.s.l. All hydraulics analysis is computed by flood mark because of the tail water depth of peak flood is very small; hence to protect overtopping flood tail water depth by flood mark elevation is safe for design

D/S HFL = 1687.8 8masl,

- Average river bed slope

Average river bed slope of River is estimated by two different techniques. One is by end area method(s=0.008) and the other is by using best fit line method (0.045). Since, it is to better to design best fit line method because of it is more accurate to actual the ground .The water level of the river is taken at different points along the river channel around the head work site. Surveying work done for 122.02m length. And the, average water surface slope is considered as the river bed slope. For comparison of the two procedures, refer the attached Excel file.

Figure 2-4: River profile

- Manning’s Roughness coefficient

  - The Manning’s roughness coefficient is taken from standard table based on the river nature. The river at the headwork site has got the River bed shows highly Boulder river feature and curving nature. The river banks covered dominantly transported boulders and gravels within the silt deposit

. Manning’s roughness coefficient (n = 0.035) is adopted.

- Discharge of the river

Input data:

- Manning's roughness coefficient, n = 0.035

- Average river bed slope, S = 0.008

- ,  	Where, R = Hydraulic radius = (Area/Perimeter)

- Q=V*A

Table 2-8: River discharge computation at different stages of flow

| S.Nr | Elevation/m/ | Water depth /m/ | Wetted Area (A), m^2 | Perimeter /m/ | Hydraulics radius/R/ | Velocity /m/s/ | Discharge /m^3/s/ | Remark |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1685.38 | 0 | 0 | 0.000 | 0 | 0 | 0 | RBL elevation from topo |
| 2 | 1685.68 | 0.3 | 0.09 | 0.806 | 0.11 | 1.347 | 0.11 |  |
| 3 | 1685.98 | 0.6 | 0.34 | 1.684 | 0.20 | 2.077 | 0.71 |  |
| 4 | 1686.28 | 0.9 | 0.77 | 2.512 | 0.31 | 2.743 | 2.11 |  |
| 5 | 1686.58 | 1.2 | 1.37 | 3.358 | 0.41 | 3.319 | 4.55 |  |
| 6 | 1686.88 | 1.5 | 2.14 | 4.185 | 0.51 | 3.854 | 8.23 |  |
| 7 | 1687.18 | 1.8 | 3.08 | 5.077 | 0.61 | 4.325 | 13.33 |  |
| 8 | 1687.38 | 2.00 | 4.25 | 6.286 | 0.68 | 4.648 | 19.75 | Peak Discharge |
| 9 | 1687.48 | 2.10 | 4.23 | 5.993 | 0.71 | 4.785 | 20.25 |  |
| 10 | 1687.88 | 2.50 | 6.09 | 7.248 | 0.84 | 5.38 | 32.76 | Flood mark |

# SECTION-II: HEADWORK DESIGN

# HEADWORK STRUCTURES DESIGN

## Headwork Site Selection

The headwork site is situated at 1189752.820 m N, 598864.713 m E and river bed elevation of 1689.181 m above sea level. The headwork site geological surface and subsurface conditions have been investigated based on the nature of the proposed structure. At the site and immediate vicinity, the stream flows along moderate to steep slope course. The right bed and bank at Weir Diversion site are made up of bedrock (the Rhyolite), on the contrary, the central and left bed are covered with the bed rocks. On the other hand, left bank is covered with the flood plain deposit dominated by bedrocks (Rhyolite). The detail geologic nature of the banks, and bed of the stream along the headwork axis and immediate vicinity are described and their potential geotechnical influence on the proposed structures also discerned/detected below, with remedial measures. The different sections of the stream at the proposed headwork site are described separately below:-

## River Geomorphology

It is a common fact that the river development tends to accommodate itself to the local geology that develops along the structurally weak zones like faults, joints, folds, etc. The drainage system of the study area is strongly influenced by geological structures and formations, the nature of the vegetation cover and climate. The nature of geological formations and structures has also strong influence on the development of the channel.

The present morphology of the Workie River channel is a function of a number of processes and environmental conditions, including the composition of the bed and the banks are made up of bedrock (the Rhyolite), on the contrary, the central and left bed are covered with the bed rocks. On the other hand, left bank is covered with the flood plain deposit dominated by bedrocks (Rhyolite). The size and composition of the sediment moving through the channel rate of sediment transport through the channel and deposition on the banks and beds and the regional degradation due to erosion processes. The bank is covered entirely with soil. The soil is dominated by SILT having low to Small plasticity the left bank composed of loose silt clay soil and coarser alluvial sediment as the result the stream shows highly meandering nature both up and downstream from the proposed site but at the particular Weir Diversion site it shows nearly straight river channel. The river has narrower section in upstream direction whereas to downstream side the river section becomes wider.

### River Bed condition

At the proposed headwork site the stream bed or course is well defined, nearly straight, and shows rough surface due to recent sediment(center and left bed) accumulations and undulating appearance of bedrock outcrops(at right bed). Along the Weir Diversion axis, the bed is made up of two basically different geologic materials, as seen from surface observation. These are recently deposited alluvial coarse grained sediments, and underling bedrock.

The central and left areas of the bed are totally covered with the alluvial deposits, while the right bed is made up of Rhyolite bedrock. The bedrock is totally covered with the sediments at the central and left areas of the stream bed. The alluvial sediment observed at the center and left beds are composed dominantly of Gravel with significant amount of boulders and cobbles. It is loose, dry (at the time of study) and not easily workable (to dig test pit). From surface geological understanding of the area, the thickness of this sediment is expected in the range 1.5 to 2m. This coarser sediment is believed to be underlain by the bedrock extension that exposed at the right bed and bank.  On the other hand, the bedrock found at right bed is affected by slight degree of weathering and erratically oriented joints. The joints are not persistence that most of them penetrate to shallow depth or affecting the top 0.5 to 1m thickness of the rock.  The foundation of the proposed Weir Diversion structure; therefore, can be lie on the bedrock after removing the top 0.5 to 1m jointed portion of the bedrock at the right side. In addition to this, the foundation of the bed bar (if proposed), along the axis should lie on the bedrock, after excavating the top alluvial deposit covering the center, and left bed of the river.

Weir axis

Right side

Left side

Figure 3-1: River bed at the proposed Weir Diversion site

### River Bank condition

#### Right Bank

At the headwork sit axis, the right bank is characterized by relatively steep slope, having about more than 3m height from stream bed. It reveals nearly vertical section within this height. From visual observation of the natural exposure, there are two geologic units (Rhyolitic rocks and loss material at the top. And it is portion of the older bedrock at the project site. The rock is Rhyolite, which is affected by shallow depth of jointing (0.5 to1m) and slight degree of weathering. It is resistant to flood erosion that there is no need to proposed bank protection works.

#### Left Bank

At the proposed headwork axis and immediate vicinities, the left bank forms relatively moderate to steep slope topography. From surface observation, the bank is covered entirely with soil. The soil is dominated by SILT having low to Small plasticity. There are few disseminated transported boulders and gravels within the silt deposit. It has 2 to 4m exposed thickness. It is flood plain deposit having dark brown color. It has stiff consistency and dry moisture content but this loss material lies on the top parts of the bed rocks (Rhyolite). The bank is not now affected by flood erosion that active eroded surfaces are visible. So, it is not necessary to provide some bank protection structures to prevent ongoing bank erosion.

Figure 3-2: River bed geological x-section

## Sources of construction materials

During the site investigation, natural construction materials required for the construction of the various proposed engineering structures at the headwork and within the farmland have been assessed, and possible quarry sites and borrow areas have been identified within the vicinity of the area as much as possible. In addition to the identification, the quality, quantity, accessibility condition and ownership of each proposed production sites have also been studied and described in this report; on separate sub-sections below. The natural materials required for the construction of the proposed hydraulic structures include rock for masonry stones, aggregates (both coarse and fine), impervious soil for fill and/or lining, backfill soil, and water.

### Rock for Masonry and Crushed Coarse Aggregate

During site investigation rocks required for masonry works were identified along the right side the Quarry site that can be used for production of rock for masonry stone and crushed coarse aggregates has been assessed during the field work session within the vicinity of the project area at economic distance for hauling.

One possible quarry site has been identified along the right side ridge following the main canal route at co-ordinates of about 597663mE and 1190027with elevation1635m. Here moderately weathered Rhyolite rock exposed and forms a continuous ridge parallel to the main canal. It is believed that below this weathered rock, fresh portion of the rock is found and can be used for the intended purpose.

In this field study, another sources for rock also proposed. The first one is near small village which is called Wedeso at co-ordinates of about 598240mE and 1190225mN with elevation1735m above sea level.

The other relatively distant source for rock is located at about 7 to 8km from the headwork to the direction of main canal (to the direction of the end of the main canal) at co-ordinates of about 596823mE and 11908824mN with elevation 1664m above sea level. Here, this quarry site good potential for all our needs.

### Fine Aggregates

Borrow areas for fine aggregate or natural sand have been assessed starting from the project stream itself. Natural deposits of such materials couldn’t be found when assessed within the beds of the stream in the project area; rather very coarser sediments and rock exposures are found covering almost the entire bed of the workie stream. Seeing to this nature of the stream, other distant streams have been explored to identify the best source areas for fine aggregate or natural sand that can be used for this particular project. During exploration of this natural sand, at a distant one stream was identified as a possible source of fine sand. The stream is known as ‘jara’. It is located at about 40km from the project site, within chefa Robit woreda. A potential source area within the jara stream bed is located alongside the main kemise Robit-Ataye Asphalt road, near to a small village of chefa Robit. Here, there are local legalized sand miners associations, and the sand is acquired from them through negotiation and agreement.

The samples have been analyzed at Amhara Design and Supervision material testing laboratory to characterize the gradation of the sand deposit. According to these laboratory test results (See Annexture-2), the sand deposits from various portion of the jara stream have grading as indicated in Figure 4.3 (Annexture-1 from Geology report). As the sand will be used as fine aggregate in the construction of concrete and as mortar ingredient for masonry works, its quality has been evaluated based on ASTM C33 specification, especially its grading.

### Water

Water for construction purposes can be found from the project stream, Workie River itself. The stream is perennial throughout the year that there is some amount of flow along its course. During this field time the stream flow was more than 260L/second.

## Headwork Type Selection

Looking the availability of natural construction materials and considering the river features and expected flood amount, Weir Diversion is chosen. As it is:

- Simple for construction

- Stability of resisting score depth  since the foundation is more safe up 2m

- At proposed head work it’s have Economical section

- availability of local material for cyclopean work

### Hydraulic Design of Headwork Structure

#### Weir Diversion Height Determination

The following major factors have been seen in determining the weir/Weir Diversion crest level:

- Maximum  command area elevation

- Deriving head of the Weir Diversion structure

- Main canal slope Sill level of off take canal

- Main canal idle Length

- Loss

- Lowest Point of river center (River bed elevation)

#### Base flow of the River

The study team has assessed that the stream is used for irrigation along its entire course at the proposed diversion site since the farmers are using the stream for traditional SSI (Irrigation Infrastructure Report). Study team has calculated flow of the river at the Weir Diversion site as 260 l/s. Out of this 223.6 l/s will be required for the proposed scheme and the rest will be released for downstream. The purpose of releasing the 14% l/s to downstream is for the sake of downstream users.

#### Weir Diversion Dimensions

##### Flow over the Weir Diversion crest

a. Crest Length

- Lacey’s regime width, L=4.75*Q,  =4.75*32.76. = 27.187 m.

- Actual river section width of the over flow section of the river is = 5m

b. Discharge over the weir section

- Design discharge, Q = 32.76 m3/s by flood mark

##### Bottom width

Bottom width of ogee weir is the summation of downstream profile length and upstream profile length.

Bottom width=upstream profile length + downstream profile length

Upstream profile length=0.27Hd whereas design head (Hd) =1.935m

=0.27*1.94=0.52m

Downstream profile length =Xn=K*Hdn-1*Y

Whereas k & n are constant for vertical ogee slope take, 2&1.85 respectively.

Height weir (Y) =2.1m & HD=1.935m

X1.85=2*Hd1.85-1*2.1

X=2.9m

Bottom width=0.52+2.9=3.46m

≈ 3.5m

Provide 5m bottom width respectively, which will be tested for adequacy during stability analysis.

### U/S and D/S HFL Calculation & Determination

From the stage –discharge curve prepared the high flood level before construction                                       (i.e. D/s HFL) corresponding to the design flood is 1687.88m a.s.l.

D/s HFL = 1687.88m amsl ------------------------------------- (a)

U/s HFL = U/s bed level + weir height + Hd -------- (b)

Hd is the depth of water over the weir crest. This is calculated by assuming ogee weir formula.

Q=C*L*He32 Whereas Q=32.76m3/s, L=5m & c=2.2

He=QC*L23 =2.07

Schematic Diagram of weir Section

The velocity head, ha is computed from the approach velocity as shown below

Where g: acceleration due to gravity = 9.81m/sec2

Va is Approach velocity determined by

L is Weir crest length = 5 m,

hd is flow depth over the weir and also,

By trial and error method, hd is found to be 1.94 m

ha = He-hd = 2.07m-1.94m = 0.13m

- Velocity head, ha = 0.13m

U/s HFL =U/s TEL –velocity head =1689.55m a.s.l – 0.13m = 1689.425m a.s.l

- Afflux

⇒ Afflux = U/s HFL- D/s HFL = 1689.415m a.s.l –1687.88 m a.s.l = 1.54m.

From the flood level analysis, it is seen that the flood overtops the banks of the river u/s of the structure. This condition is allowed to take place as it doesn’t bring pronounced negative impacts on the structures, rather than constructing bulky structures to confine it.

### Hydraulic Jump Calculation

As discussed in the geologic report, the river bed is alluvial deposit and hence stilling basin for energy dissipation is required. Both left and right side banks are bed rock, a wing walls are required at u/s and D/s sides for the right side of the river, so as to protect flood entering to the canal in high flood cases.

The length of wing walls is determined based on the length of Jump, and it is calculated as shown below.

• Weir crest length =5m

• Weir height = z = 2.1m

• Pre-jump depth = y1

• Post -jump depth =y2

Neglecting losses between point A and B and considering similar datum

z + He = y1 + ha

But, He = 2.07m,

q=Ql=32.76m3/s5m=6.552m2/s

ha=q22*g*y2=10.7122*g*y12

2.1m + 2.07m=y1+2.89/y12

After iterations Y1 = 0.81 m

V1=q/y1=6.552/0.81=8.08m/se

Fr=V1gy1=8.089.81*0.81=2.89

y2=y121+8*Fr2-1             y2=0.8121+8*2.822-1 =2.916m

Hydraulic jump length (L) for Fr=2.89from the graph L=5*(y2-y1) =5*(2.916-0.81) = 10.53~11m

### Impervious floor

#### D/s impervious floor (Ld)

For under seepage the worst condition would be when the water on the upstream side is at the level of the weir crest & there is no tail water. Seepage head loss at

- Pond level case:

Hs = crest level –bed level

= 2.1m

- Maximum flood case:

Hs = U/s HFL- D/s HFL

= 1689.415m-1687.88m

= 1.54m

Therefore maximum seepage head occurs when water is stored up to the pond level and there is no water on the d/s.

= Bligh’s constant, Cb is depending on the type of the foundation.

Ld=2.2*Cb*Hs10                        Ld=2.2*9*2.110 =9.07 m

The jump length is 11m, Therefore D/s impervious floor is taken to be 11m long, the maximum.

#### U/S Impervious Floor Length, (Lu)

The u/s impervious floor, (Lu) = LT- (Ld +2*d1+2*d2+B) = -3.09m take nominal distance=1m Therefore total length of the u/s impervious floor is taken 1m long.

Whereas, LT=Hs*C =2.1*9=18.9m, C, based on river characteristics material

Ld =length of D/S Impervious Floor Length=11m

D1=U/S cut off=1.5m

D2=d/s cut off=2.5m

B=bottom width of weir=3.5m

      - Floor thickness (t) by khosla’s & Bligh’s theory

Creep length (b) =2*1.5+1+3.5+11+2*2.5= 23.5m

Hydraulic head (d) =pond level -bed level

= 1687.5m a.s.l-1485.376m a.s.l

=2.1m

Hydraulic gradient (GE) =d/b=2.1/23.5=0.0894

Floor thickness at heel

Creep length up to point (L)

=2*d1+L2=2*1.5+1=4m

Up lift pressure (head) at heel (h) =HS-GE*L

h=2.1-0.0894*4=1.742m

Thickness (t)=4/3*h/(G-1)

t=4/3*1.7424/(2.4-1) =1.74m

Take 1m b/c there is counterbalance water load

Floor thickness at Toe

Creep length up to point (L)

=2*d1+L2+B=2*1.5+1+3.5=7.5m

Up lift pressure (head) at heel (h) =HS-GE*L

h=2.1-0.0894*7.5=1.43m

Thickness (t) =4/3*h/ (G-1)

t=4/3*1.43/ (2.4-1) =1.4m

Take 1.4m calculated by Bligh’s theory

Floor thickness at 4m from Toe

Creep length up to point (L)

=2*d1+L2+B+4=2*1.5+1+3.5+4=11.5m

Up lift pressure (head) at heel (h) =HS-GE*L

h=2.1-0.0894*11.5=1.7m

Thickness (t) =4/3*h/ (G-1)

t=4/3*1.7(2.4-1) =1.072m

Take 1m calculated by Bligh’s theory

Floor thickness at 8m from Toe

Creep length up to point (L)

=2*d1+L2+B+10=2*1.5+1+3.5+8=15.5m

Up lift pressure (head) at heel (h) =HS-GE*L

h=2.1-0.0894*15=0.714m

Thickness (t) =4/3*h/ (G-1)

t=4/3*0.714/ (2.4-1) =0.68m

Take 0.8m calculated by Bligh’s theory

Floor thickness at 15m from Toe

Creep length up to point (L)

=2*d1+L2+B+15=2*1.5+1+3.5+11=18.5m

Up lift pressure (head) at heel (h) =HS-GE*L

h=2.1-0.0894*18.5=0.446m

Thickness (t) =4/3*h/ (G-1)

t=4/3*0.446/ (2.4-1) =0.42m

Take 0.6m calculated by Bligh’s theory

The floor is constructed with cyclopean C-20 concrete with well graded stone

- For detail analysis of the computation, Refer Excel file.

### Cut off Depth Calculation

#### U/s cut off

- Q = 32.76 m3/sec

- q = 6.552 m3/s/m

Silt factor, f =1.76*(d50)0.50

f= 3.78

- D50=4.66mm for fine gravel & boulder

- R =1.35*(q2/f)(1/3)

R=1.35*(q2/f) (1/3) = 3.04m	 take R = 3.04 m

Hence bottom level of u/s cut off = U/S HFL – 1.25*R

=1689.415.-1.25*3.04=1685.621m

Take the bottom level of the u/s cut off as 1685.621 considering the floor thickness

U/S Cut off depth=river bed level- bottom level of u/s cut off

=1685.376-1685.621=-0.25 take 1.5m, therefore, bottom level of U/s cut off =1685.376-1.5=1683.876m

#### D/s cut off

- Q =32.76m3/sec

- q =6.522 m3/s/m

- Silt factor, f =1.76*(d50)0.50,  f= 3.78 take f=3.78

- D50=4.66mm for fine gravel & boulder

- R=1.35*(q2/f)(1/3),   R=1.35*(q2/f)(1/3) =3.04 take

Hence bottom level of u/s cut off = D/S HFL – 1.5*R

=1687.88-1.5*3.04=1683.323m

Take the bottom level of the u/s cut off as 1683.323m considering the floor thickness

D/S Cut off depth=river bed level- bottom level of d/s cut off

=1685.376-1683.323=2.07 take 2.5m from geological report

For each arrangement and further information, refer to the design drawing.

Figure 3-3: weir section

### Stability Analysis of weir

Stability analysis is carried out to see the already determined weir section is safe against overturning, sliding, tension. The stability analysis is carried out considering the effect of the following forces.

• Water pressure

• Weight of the over flow weir section

• Sediment load

The extreme load combination is the case where the head is at crest level of the weir and there is no flow over the weir (static case)

Figure 3-4 :Weir stability

| Name of forces | Symbol | Area(m2) | Magnitude of forces (KN) |  | Lever arm in m | Moment at "o"(KN.M) |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Vertical | Horizontal |  | Resisting | Disturbing |
| Weight Of Weir Body | W-1 | 0.08 | 1.70 |  | 3.21 | 5.451 |  |
|  | W-2 | 1.09 | 24.57 |  | 3.30 | 81.081 |  |
|  | W-3 | 2.10 | 47.25 |  | 2.54 | 120.02 |  |
|  | W-4 | 0.37 | 8.33 |  | 1.71 | 14.208 |  |
|  | W-5 | 1.20 | 27.00 |  | 1.54 | 41.580 |  |
|  | W-6 | 0.62 | 14.04 |  | 1.71 | 23.962 |  |
|  | W-7 | 0.29 | 6.53 |  | 2.71 | 17.661 |  |
| 1. Water pressure(U/S) | Pwh | 2.212 |  | -21.70 | 0.70 |  | -15.215 |
| 2. Silt pressure | Psilt | 2.629 |  | -5.77 | 0.70 |  | -4.047 |
| 5. Uplift pressure | U1 | 3.647 | -35.777 |  | 2.31 |  | -82.708 |
| sum |  |  | 93.63 | -27.47 |  | 303.96 | -101.970 |

∑V = 93.63KN                                 ∑M (+) = 303.96KN.m

∑H=-27.47KN	  ∑M (-) = 101.970KN.m

i) Factor of safety against overturning (Fo)

>1.5 	       Safe!

ii) Factor of safety against sliding (FS)

,

<0.75           Safe!

iii) Check for tension (i.e. whether the resultant lies within the middle third)

The location of the resultant force from the toe is given by

The eccentricity (e) = X – 3.5/2, B = 3.5m

Hence, e = 2.16-3.5/2 = 0.41m

The eccentricity (e) should be less than B/6 =, Hence the obtained e = 0.41m < 0.583m.

⇒The resultant lays within the middle third                  no tension

Conclusion: From stability analysis, the designed weir section is safe. To be economical, Provide ogee weir with 3.5m bottom width.

#### Wall height fixation of wing wall

The existing topographical condition at the weir axis and HFL are considered to be most governing parameters for fixing the wall height.

After construction of the weir (u/s HFL) = 1689.415m.a.s.l

U/s wall height = U/s HFL - foundation level + free board.

Take 0.2m free board

U/s wall height = 1689.415m.a.s.l – 1685.376m + 0.2m =4.2 m

Provide 4.2m wall height and the top level of the U/s retaining wall =1685.376m + 4.2m =1689.615m.a.s.l

D/s wall height = D/s HFL - foundation level + free board

=1687.88m.a.s.l-1685.376m.a.s.l+0.2=2.7m

Hence including free board, take the critical height of the D/s retaining wall = 2.7m

U/s Retaining wall Stability Analysis

#### Under sluice/silt Excluder/

Silt Excluder is designed in order to safely release sediment that incoming in to canal out let. The stream is small crest length to carry very large sediment load having silts, sands, cobles, and boulder. Besides, the weir height is so medium.  Therefore, the sediment load is mostly dissolved clay, silts, and fine grained sand, easily disposed by farmers manually. Hence, under sluice opening having 70x70cm internal opening, which can be installed and detached by hand, is provided

#### Design of the canal head sluice

In order to allow, the water to the main canal a canal head sluice of opening 70cmx45cm is adopted. This gate is controlled by the manually maneuvered gear driven gate, which is operated at the top of the left retaining wall. And the head regulator is provided on the Right side .The sill level of this head regulator is fixed from different angle observations. The main conveyance system is more than 3.325km which passes more gullies and undulating alignment. Hence this level is fixed based on the optimum route alignment and the maximum irrigated command level including minor and major losses criteria. Based on this condition, the sill level is fixed to be 1687.05m.

• Outlet capacity

The minimum command area is determined by the minimum flow of the river. But the canal capacity should be determined for maximum command area and the corresponding discharge. In this case the outlet capacity is fixed considering maximum duty and command area and 1.25 correction factors are considered to account the variation of duty.

- Outlet capacity = Duty x command area x correction factor

Where, maximum duty for 16 hr irrigation = 1.04 L/s/ha

Command area = 215ha.

- Outlet capacity = 1 x 1.04 L/s/ha x 215ha = 223.6 L/sec, Say 224 L/s.

• Outlet size

From the Weir Diversion discharge formula the outlet size is determined as follows

Q= 2/3*Cd*(2g) ^ (1/2)*B*H^ (2/3

Where: Q=diverted discharge,

B=Width of Weir Diversion,

H=Height of the Weir Diversion

g=Acceleration due to gravity

Hence, provide an outlet size of 0.7m x 0.45m (length x height) at the entrance .The gate of the off take canal is to be vertical sheet metal of 0.7m x 0.45m for the closure of the opening space. Provide some extra dimensions for groove insertion. Gross area of sheet metals for the off take canal gate will be 0.8m x 0.550m (allowing 5cm insertion for grooves and above the weir/Weir Diversion crest level). The grooves are to be provided on the walls using angle iron frames at the two sides of the gate openings.

Trash racks of diameter 12mm with c/c spacing of 10cm has to be provided u/s of the gate to prevent entry of debris to the canal.

#### Retaining Walls

In the right side the retaining wall is designed by cyclopean concrete to be safe structurally and to minimize the area to be cover by masonry wall hence, the river width is very small.

Stability analysis is carried out to see the already determined retaining wall section is safe against overturning, sliding, tension. The stability analysis is carried out considering the effect of the following forces.

- Weight of the retaining wall section

- Back fill of moist Soil pressure

The extreme load combination is the case of wet condition. Therefore we provide 2.8m masonry retaining structure d/s of the Weir Diversion location and. (For more detail sees the drawing& exell file)

Ps1

W1

W2

Ps2

W3

Figure 3-5: Typical design of masonry retaining wall

## Bill of Quantity and Cost Estimation

The quantities of the various items have been worked out as per the final design and final drawings prepared for the scheme. The unit rates analysis has been carried out based on the data available in the vicinity of the project area.

Bill No. 1- General Items

|  | BILL No. 1. GENERAL |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| S.N | Description | Unit | Quantity | Unity Rate | Amount Birr |
| 1.1 | Allow for mobilization | L.S | 1.0 | 280,000.0 | 280,000.0 |
| 1.2 | Allow for demobilization | L.S | 1.0 | 252,258.9 | 252,258.9 |
| 1.3 | Allow for contractor’s camping facilities 4*5m2, Living room for contractors key personnel, CIS and internally painted clip wood wall, Masonry floor cement screened and well ventilated room complete with doors and windows. | No | 5.0 | 62,037.3 | 310,186.4 |
|  | Allow for Consultant’s camping facilities 4*5m2, Living room for contractors key personnel, CIS and internally painted clip wood wall, Masonry floor cement screened and well ventilated room complete with doors and windows. | No | 2.0 | 62,037.3 | 124,074.6 |
|  | 5*5m2, Store and dining room constructed from CIS with doors and windows, Masonry floor cement screened | No | 1.0 | 69,157.0 | 69,157.0 |
|  | Barbed wire fence 60*20m and 1.5m high treated timber post complete with 3m wide gate and a CIS guard house (1.5*2m) | No | 1.0 | 70,368.4 | 70,368.4 |
| 1.4 | Construct temporary access road to site the acess road needs Cut and hilly terrain, fill the Gorgy area, boulder excavation , hard rock Excavation and heighly Site clearing. | km | 1.5 | 352,017.3 | 528,025.9 |
| 1.5 | Construction of toilet and shower as per drawing consultant and contractor staffs (2*1.5)m2 | Ls | 2.0 | 20,000.0 | 40,000.0 |
| 1.6 | Dewatering of open trenches and excavations, pumps | LS | 1.0 | 318,700.77 | 318,700.8 |
| 1.7 | Provide project indicator post starting from the construction time | LS | 1.0 | 7,807.68 | 7,807.7 |
| 1.8 | Provision of as built drawings for the project | LS | 1.0 | 52,219.39 | 52,219.4 |
|  | Total |  |  |  | 2,052,799.1 |

| Bill No- 2: HEAD WORK |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| S.N | Description | Unit | Qty | Unit Cost/ Birr | Amount/ Birr/ |
| I | Head Work Structure |  |  |  |  |
| 1 | Weir body (Ogee weir ) |  |  |  |  |
| 1.1 | Excavation river deposit (sand, gravel , pebble and highly boulder) | m3 | 20.00 | 123.7 | 2,474.2 |
| 1.2 | Hard rock Excavation | m3 | 20.00 | 804.7 | 16,093.6 |
| 1.3 | Weathered Rock Excavation | m3 | 2.00 | 487.4 | 974.7 |
| 1.4 | Back fill and Compaction with Excavated material | m3 | 10.00 | 67.4 | 674.1 |
| 1.5 | Cyclopean (60% C-20 & 40%graded stone Boulder) inculding form work | m3 | 94.70 | 1,864.0 | 176,524.0 |
|  | Sub Total |  |  |  | 196,740.7 |
| 2 | U/S and D/S Apron |  |  |  |  |
| 2.1 | U/S Apron |  |  |  |  |
| 2.1.1 | Excavation river deposit (sand, gravel , pebble and highly boulder) | m3 | 0 | 123.7 | - |
| 2.1.2 | Weathered rock excavation | m3 | 5.2 | 487.4 | 2,534.3 |
| 2.1.3 | Hard rock Excavation | m3 | 3 | 804.7 | 2,414.0 |
| 2.1.4 | Cyclopean concrete(60% C-20,40% graded stone) including Form work | m3 | 4 | 1,864.0 | 7,456.1 |
|  | Sub Total |  |  |  | 12,404.5 |
| 2.2 | D/S Apron / Boulder Riprap |  |  |  |  |
| 2.2.1 | Excavation river deposit (sand, gravel , pebble and highly boulder) | m3 | 0 | 123.7 | - |
| 2.2.2 | Weathered rock excavation | m3 | 32 | 487.4 | 15,595.6 |
| 2.2.3 | Hard rock Excavation | m3 | 5 | 804.7 | 4,023.4 |
| 2.2.4 | Laying Boulders of size more then 0.6m diam. | m3 | 30 | 1,864.0 | 55,921.0 |
|  | Sub Total |  |  |  | 75,540.0 |
| 3 | U/S and D/S Cut off |  |  |  |  |
| 3.1 | U/S Cut off |  |  |  |  |
| 3.1.1 | Excavation river deposit (sand, gravel , pebble and highly boulder) | m3 | 3 | 123.7 | 371.1 |
| 3.1.2 | Weathered rock excavation | m3 | 2.8 | 487.4 | 1,364.6 |
| 3.1.3 | Hard rock Excavation | m3 | 2 | 804.7 | 1,609.4 |
| 3.1.4 | Reinforced C-20 concrete(1:2:3) | m3 | 1.75 | 2,761.5 | 4,832.7 |
| 3.1.5 | Back fill and Compact with excavated material | m3 | 6 | 67.4 | 404.5 |
| 3.1.6 | Ф12mm bar | Kg | 60.43 | 80.0 | 4,834.5 |
|  | Sub Total |  |  |  | 13,416.8 |
| 3.2 | D/S Cut off |  |  |  |  |
| 3.2.1 | Excavation river deposit (sand, gravel , pebble and highly boulder) | m3 | 1.8 | 123.7 | 222.7 |
| 3.2.2 | Weathered rock excavation | m3 | 3 | 487.4 | 1,462.1 |
| 3.2.3 | Hard rock Excavation | m3 | 3 | 804.7 | 2,414.0 |
| 3.2.4 | Reinforced C-20 concrete(1:2:3) | m3 | 7.2 | 2,761.5 | 19,883.1 |
| 3.2.5 | Back fill and Compact with excavated material | m3 | 4.5 | 67.4 | 303.4 |
| 3.2.6 | Ф12mm bar | Kg | 214.44 | 80.0 | 17,155.2 |
|  | Sub Total |  |  |  | 41,440.5 |
| 4 | Protection work |  |  |  |  |
| 4.1 | U/s retaining wall |  |  |  |  |
| 4.1.1 | Excavation river diposite (sand,gravel , pebble and boulder) | m3 | 4.14 | 123.7 | 512.2 |
| 4.1.2 | Weathered rock excavation | m3 | 6.90 | 487.4 | 3,362.8 |
| 4.1.3 | Hard rock Excavation | m3 | 2.76 | 804.7 | 2,220.9 |
| 4.1.4 | Masonry work with 1:3 cement sand ratio | m3 | 20.70 | 1,669.6 | 34,561.5 |
| 4.1.5 | 10cm thick C-10 lean concrete(1:3:6) | m3 | 1.38 | 1,960.1 | 2,704.9 |
| 4.1.6 | Pointing (1:2) | m2 | 13.80 | 132.1 | 1,823.0 |
| 4.1.7 | Cyclopean concrete(60% C-20,40% graded stone) including Form work | m3 | - | 1,864.0 | - |
| 4.1.8 | Back fill and Compact with excavated material | m3 | - | 67.4 | - |
|  | Sub Total |  |  |  | 45,185.2 |
| 4.2 | D/s retaining wall |  |  |  |  |
| 4.2.1 | Excavation river deposit (sand, gravel , pebble and highly boulder) | m3 | 250.00 | 123.7 | 30,927.5 |
| 4.2.2 | Weathered rock excavation | m3 | 60.00 | 487.4 | 29,241.8 |
| 4.2.3 | Hard rock Excavation | m3 | 100.00 | 804.7 | 80,468.0 |
| 4.2.4 | Masonry work with 1:3 cement sand ratio | m3 | 250.00 | 1,669.6 | 417,409.2 |
| 4.2.5 | 10cm thick C-10 lean concrete(1:3:6) | m3 | 24.16 | 1,960.1 | 47,347.0 |
| 4.2.6 | Pointing (1:2) | m2 | 250.00 | 132.1 | 33,025.1 |
| 4.2.7 | Cyclopean concrete(60% C-20,40% graded stone) including Form work | m3 | 135.00 | 1,864.0 | 251,644.6 |
| 4.2.8 | Back fill and Compact with excavated material | m3 | 335.50 | 67.4 | 22,617.4 |
|  | Sub Total |  |  |  | 912,680.6 |
| 5 | Gates |  |  |  |  |
| 5.1 | Off take Canal gate |  |  |  |  |
| 5.1.1 | Off take Canal gate supply and installation consist of:- | No | 1.00 | ###### | 25,321.5 |
|  | (0.7mx0.0.45m, 6mm thick sheet metal, |  |  |  |  |
|  | (50x50x10) 5.58m long angle iron for groove |  |  |  |  |
|  | ø40mm spindle 2.92m long, with all accessories |  |  |  |  |
|  | ø10mm bar for anchorage, Angle iron with the masonry wall 2.0Kg |  |  |  |  |
| 5.1.2 | Trash rack Ф12mm bar | Kg | 63.30 | 80.0 | 5,063.7 |
| 5.1.3 | Concrete C-25(1:2:3) | m3 | 0.40 | 2,788.8 | 1,104.4 |
| 5.1.4 | Slab Ф12mm bar | Kg | 41.55 | 80.0 | 3,323.7 |
|  | Sub Total |  |  |  | 34,813.3 |
| 5.2 | Silt excluder |  |  |  |  |
| 5.2.1 | simple hand Gate consists of | Pcs | 1.00 | ###### | 28,451.6 |
|  | 8mm thick sheet metal(0.7m*0.7m) |  |  |  |  |
|  | Stiffening angle iron (30x30x4) |  |  |  |  |
|  | Angle iron for groove (55x50x4) |  |  |  |  |
|  | 16mm reinforcement bar for handling |  |  |  |  |
|  | Sub Total |  |  |  | 28,451.6 |
| 6 | Escape canal and Gate |  |  |  |  |
| 6.1 | Escape Canal |  |  |  |  |
| 6.1.1 | Excavation river deposit (sand, gravel , pebble and highly boulder) | m3 | 10.73 | 123.1 | 1,320.9 |
| 6.1.2 | Masonry work with 1:3 cement sand ratio | m3 | 3.19 | 1,669.6 | 5,326.1 |
| 6.1.3 | Plastering (1:2) | m2 | 7.98 | 132.1 | 1,054.2 |
| 6.1.4 | Cyclopean 60% C-20 & 40% Boulder inculding form work | m3 | 1.13 | 1,864.0 | 2,097.0 |
| 6.1.5 | Concrete C-25(1:2:3) | m3 | 1.09 | 2,788.8 | 3,039.8 |
| 6.1.6 | 10cm thick C-10 lean concrete(1:3:6) | m3 | 2.37 | 1,960.1 | 4,645.3 |
|  | Sub Total |  |  |  | 17,483.3 |
| 6.2. | Escape simple Hand shater gate |  |  |  |  |
| 6.2.1 | Escape canal gate consists installation, the gate consists the following parts | No | 2.00 | 2,170.8 | 4,341.7 |
|  | 6mm thick sheet metal. 0.7m x 0.7m, (50x50x10mm ) angel iron 1m length. Handel 12mm Ø |  |  |  |  |
|  | Sub Total |  |  |  | 4,341.7 |
|  | Total |  |  |  | 1,382,498.1 |

# SECTION-III: IRRIGATION AND DRAINAGE SYSTEMS INFRASTRUCTURE

# IRRIGATION AND DRAINAGE SYSTEMS DESIGN

## Irrigable Area Description

### Topography

Topography is an important factor for the planning of any irrigation project as it influences method of irrigation, drainage, erosion, mechanization, and cost of land development, lab our requirement and choice of crops.

The topographic feature of the project command area is mainly sloping type. Its elevation range is from 1665 to 1553 meters above sea level. The slope gradient also ranges from steeply sloping (3%) to strongly sloping (25%). However, it has identified to be suitable for surface irrigation. Nevertheless, it requires soil and water conservation measures or structures (i.e. constructing bunds, bio-physicals, check dams, artificial water ways, etc).

The project command area is situated at the Right side of Workie River. The natural topographic feature of the command area has inclined to the North-East direction.

### Climate

As per the hydrological analysis and on the basis of the traditional Ethiopian Agro-Ecological Zones (MOA, 2001), the UGDWIP area is basically classified as under Woina Dega agro-ecology which is conducive to the production of tropical highland crops. There is no belg rain season in the project area. Climate has an important influence on the nature of the natural vegetation, the characteristics of the soil, the crops that can be grown and the types of farming that can be practiced in any region. The climate of an area is highly correlated with its vegetation and, by extension, the types of crops that can be cultivated. The project area has Bimodal and uneven distribution pattern of rainfall. The main rain season, locally known as Keremt, occurs from end of June to end of August with about 80% share of annual rainfall; and about 20% of the annual rainfall occurs during the dry season from October to May.

In the project area the main bottle neck for the successful crop production is uneven distribution of rainfall, especially in the months of August and September. The highest rainfall occurs in the months of July and August with a better intensity and spatial distribution. Had it been the annual rainfall sufficient enough and evenly distributed throughout the year, crop production of the area would have been remarkably high.

As the project site has no its own meteorological station, Kemissie (for rainfall and minimum and maximum temperature) and Kemissie (for relative humidity, wind speed and sunshine hour) meteorological stations data were used for the project study as long as these stations are relatively near to the proposed command area. In general, the sources of meteorological data are the National Meteorology Service Agency (NMSA). Kemissie rainfall data was used for the computation (using CropWat 8.0 software) and analyses of irrigation water requirements. The amount of average annual rainfall at Kemissie meteorological station is about 1049mm. (for further detail see the Agronomy Study of the same project

### Soil characteristics

Soil properties (physical, chemical, etc.) greatly influence the growth and thereby yield of crops which is grown. The command area has predominantly clay textured soils which can be classified as moderately drained soil. Most of the study areas soils are categorized as deep soil (1-1.5 meter depth) .Soils of the command area are suitable for most of the selected crops to be grown (for further detail see the Agronomy Study of the same project.

### Existing Irrigation Practices in the Project Area

The pressure of survival and the need for additional food supplies to meet the demands of the increasing population is necessitating a rapid expansion of irrigation schemes. Thus, irrigation is becoming a basic part of well-developed agriculture wherever there is water and irrigable land potential. Accordingly, traditional irrigation practices are under taken by individual farmers that use the river flow to the Right side is with laborious temporary diversions and also it is traditionally irrigation practice by HDPE pipe. So, the farmers in the project area are very much interested in the idea of upgrading the traditional scheme to modern scheme.

## Irrigation Water Requirement

### Crop Water Requirement (CWR)

The calculation of crop water requirement is a very important aspect for planning of any irrigation project. Several methods and procedures are available for this. The Food and Agriculture Organization (FAO) of the United Nations has also made available several publications on this subject and other issues related with this. The computer program available in FAO Irrigation and Drainage Paper No. 56 “CROPWAT” has been used for the calculation of Crop Water requirement. This program is based on Penman-Monteith approach and procedures for calculation of crop water requirements and irrigation requirements are mainly based on methodologies presented in FAO Irrigation and Drainage Paper No. 24 “Crop Water Requirements” and No. 33 “Yield Response to Water”.

The corresponding values of the crop water requirements of the proposed crops of the project are presented in the Agronomy Study of the same project.

### Irrigation efficiency (Ep)

To complete the evaluation of the demand, the efficiency of the water distribution system and of application must be known.

The gross requirement of water for irrigation system is very much dependent on the overall efficiency of the irrigation system, which in turn is dependent on several factors: Method of irrigation, type of canal (Lined or Unlined), method of operations (simultaneously and continuous or Rotational water supply), and availability of structures (for controlling and distribution and measuring and monitoring).

On the basis of these factors, the project has planned to impose surface irrigation method (using furrows). The canal system is Designed lined for main and Secondary canal, Earthen for tertiary and field canal. Hence, the conveyance efficiency has been estimated to be 95%, distribution efficiency 80%, and field application efficiency 60%. As a result of these the overall irrigation efficiency has been estimated to be 55%. According to soil Lab result, soils of the command area are predominantly characterized as clay loam soils.

### Irrigation duty

Irrigation duty is the volume of water required per hectare for the full flange of the crops. Moreover, it helps in designing an efficient irrigation canal system.

The area, which will be irrigated, can be calculated by knowing the total available water at the source and the overall duty for all crops required to be irrigated in different seasons of the years.

The proposed cropping pattern of Workie Weir Diversion irrigation project has showed a maximum net irrigation water requirement (NIWR) in the month of April with the amount of 3.3mm/day for 16 working hours (for overall proposed crops).

However, for the designing of the irrigation water application and the flows in the entire canal systems, from the overall proposed crops the one that has maximum NIWR was used for irrigation duty calculation. Accordingly; and hence taken for the irrigation project duty calculation as indicated here below:

For Workie Weir Diversion Irrigation Project, it decided to adopt 60% field application efficiency, 80% distribution efficiency, and 95% conveyance efficiency as the soil is clay loam textured and the canal systems are estimated to be lined for main and secondary canal near head work. Hence, the overall/project efficiency for the selected surface irrigation method has been estimated to be 55% (60/100*95/100*80/100) which is rounded to 55%.

For the designing of the project, the GIWR is given as follows:

GIWR = NIWR/e = 3.3/0.55 = 6.00 [mm/day]

The GIWR, 6 mm/day, represents the daily quantity of water that is required to be applied. This water quantity is also used for the determination of the canal discharge in consideration of the time of flow and is defined as the duty, expressed as l/s/ha.

The duty is calculated by:

Duty  (D) = GIWR × 1000 × 10 / (t × 60×60)

Where; Duty – the duty [l/s/ha]

GIWR – Gross Irrigation Requirement [mm/day]

t – Daily irrigation or flow hours [hrs]

The duty for the GIWR of 6mm/day and 16 hours of daily irrigation time (t = 16), is supported to be used with furrow irrigation method. Hence, Duty for 16 working hours, as the site is nearer to farmers’ village and local farmers have experiences in irrigation, is computed as follows:

The duty for GIWR of 6.00mm/day for daily irrigation time 16 hours of dry season full irrigation is computed as:

D = (6.00x10x1000) / (16x3600) = 1.04l/s/ha

The GIWR for wet season crops is given as follows:

GIWR = NIWR/e = 1.3/0.55 = 2.36 [mm/day]

The GIWR (2.36mm/day) represents the daily quantity of water that is required to be applied for wet season crops as supplementary irrigation.

Duty (D) = GIWR × 1000 × 10 / (t × 60×60)

Where; Duty – the duty [l/s/ha]

GIWR – Gross Irrigation Requirement [mm/day]

t – Daily irrigation or flow hours [hrs.]

The duty for GIWR of 2.36mm/day for daily irrigation time of 12 hours for supplementary irrigation has been computed as:

D = (2.36x10x1000) / (12x3600) = 0.55 l/s/ha

### Irrigation methods

Among the different irrigation systems Workie irrigation system will be used for the project area; and the irrigation water will be obtained from Workie River and by constructing diversion weir/Weir Diversion and convoying the water commonly through earthen canals (TC, and FC) and then leading to field canals; and finally irrigation takes place mostly in furrows.

For this project, among the various irrigation methods, surface irrigation method has been selected.  Of the surface irrigation methods furrow, border and basin irrigation methods can be used to supply irrigation water to the plants/crops. However, each method has its own advantages and disadvantages. Care should be taken when choosing the method which is best suited to the local circumstances, i.e., depending on slopes, soil types, selected crop types, amount of water available, etc. of the command area.

Based on the above factors surface irrigation method has been proposed for the proposed crops in this project. The method allows applying light irrigation and can be laid out in sloping fields along the contour. Furrow irrigation method is best suited for most of the proposed and row planted crops. In general, furrow irrigation method is simple, manageable and widely practiced irrigation method. This method is suitable for row crops that cannot stand in water for long periods. The only thing required to use this method is row planting of crops. Besides, basin and border irrigation method would be used for the non-row planted crops. Rotational flow water distribution is also recommended for the project area.

## Irrigation and Drainage System Layout

The irrigation system layout for the project is prepared taking the following points into consideration besides other factors.

- A primary concern in the layout of the system is that it serves the purpose of conveying and distributing water to the command area.

- The excavation and earth fill volumes not be excessive, otherwise the construction costs can be tremendous.

- The selection of longitudinal bed slope is made taking into account the existing slopes of the terrain, so as to minimize deviations in canal routing.

- Curves in canals should not be too sharp.

The proposed irrigation system layout comprises 1 (Nr) RCC main canal, 3(Nr) secondary HDPE pipe and 30 (Nr) tertiary HDPE pipe network systems further information Refer on the layout Drawings. The main canal runs for most of its length parallel to the contours and several changes of direction are necessary to follow the topography. It crosses four main gullies, two urap road. The main canal is RCC lined for a length of 3325meters starting from the Weir Diversion outlet. The main canal route passes through a narrow terraces supported by dry masonry at some places and there is no working space, in most places, as the right and left sides are covered by chat plantations or else surrounded by upper and lower terrace for easier for construction there for to minimize challenge of working space and work methodology it propose rectangular C-20 concrete canal with 0.15m thickness for bottom and side walls, and nominal reinforcement 8mm deformed reinforcing bar.

### Conveyance System

The conveyance system consists of 1 (Nr) Main canal to irrigate total command area of 215 ha. The main canal starts from Water abstraction site on Right side and conveys water for a length of 3.325 Km.

Main canal is aligned along contours and supplies to three secondary HDPE pipe flow, and 30 tertiary HDPE pipe network systems.

## Design of the Canal System

Flow Depth and Section Capacity

The earthen canals have been designed with a trapezoidal shape and the lined ones with rectangular x-section using Manning's Formula:

Where Q= discharge (m3/s)

R= Hydraulic radius (Flow area/wetted perimeter)

S= Hydraulic gradient

n= Manning's roughness coefficient, n=0.025 is adopted for the earth channels and n=0.018 for the masonry lined part of the main canal

### Main Canal

The main canal is designed for a discharge of 223.5l/s and depending on the site specific condition, appropriate slope is provided. Hydraulic parameters of the main canal are shown below.

At a chain age of 0+600-0+628m, 1+378-1+418, and 2+782-2+842 there are Flume structures.

Table 4-1: Hydraulic Parameters of main canal

B= Canal bottom width                         QR= required discharge           d= Full supply depth

S= Longitudinal slop                      V= Velo city

QD= Designed Discharge

### Secondary and tertiary HDPE Pipe

Lay out of all secondary canals are across the steep slope, which has steep canal bed slope. The bottom and the sides of the masonry works for these canals cannot withstand the scouring effect of the flows in this canal. It is also more technically acceptable all network system is HDPE pipes of the required diameter. The economic viability feasible rather than concrete and masonry works. Mane hole at each junction and energy dissipater at each field valve is provided properly. The hydraulic characteristics of the pipes and junctions extracted from water cad analysis is attached as an annex table ……

### Field Canals (Pipes)

The farmers should be responsible in the instalation of filed pipes. Gated risers with 1 inch GS pipes  were provided at the selected field outlets from the teritiary pipes. From this pipe the individual farmers will install their own field pipe to irrigate their farms.

## Canal Structures Design

### Design of a typical flume

Hydraulic Characteristics of the canal for flume-1 on MC

Length of the flume: 7m

Shape of the flume:  Rectangular Roughness coefficient, n =0.014

From the canal longitudinal profiles, u/s canal bed level (CBL) = 1681.88m

D/s Canal bed level (CBL) =1681.47m

U/s full supply level (FSL) = 1682.13m

D/s Full Supply Level = 1681.71m

Total head loss between the inlet & Outlet =0.41m

Table 4-2: Hydraulic Parameters of Flume

| Flume location | Chain age | Q(m^3/se) | V(m/s) | L(m) | Width canal(m) | Depth canal(m) |
| --- | --- | --- | --- | --- | --- | --- |
| MC | 0+600-0+628 | 0.238 | 0.76 | 7 | 0.7 | 0.7 |
| MC | 1+378-1+418 | 0.140 | 0.73 | 7 | 0.6 | 0.7 |
| MC | 2+782-2+842 | 0.140 | 0.67 | 7 | 0.5 | 0.5 |

### Design of Division boxes

At different points of the main and secondary canals division boxes are provided which divert the flow to the secondary canal and tertiary canals. Turnout is also recommended in the main canal to directly to the field canal is. Gate should be provided at the outlet of the boxes. For detail refer the drawing.

Figure 4-2: Typical Division Box section

Using broad crested formula,

Q= CL (h) 3/2

Where; Q= discharge over rectangular weir/Weir Diversion (opening), m3/s

C = discharge coefficient, c= 1.7

L= effective length of crest form in m

h= over flow depth, m

Assuming equal discharge coefficient & sill height for two or three dividing canals, the proportion becomes.

Q1/ Q2= Q2/ Q3 =L1/ L2 = L2/ L3

Where Q1= is flow in canal 1

Q2 = is flow in canal 2

Q3 = is flow in canal 3

L1= is effective crest length of weir/Weir Diversion sill across opening to canal 1

L2= is effective crest length of weir/Weir Diversion sill across opening to canal 2

L3 = is effective crest length of weir/Weir Diversion sill across opening to canal 3

Q1= CL1 (h) 3/2,

L1 = Q1/Ch3/2

L2 = L1*Q2/ Q1

L3 = L1*Q3/ Q1

The depth of (height of) the division box,

D = d + fb

The width of the division box,

B = b + 2*m*D

Where b= base width of the incoming canal

D = total canal depth of the incoming canal

Table 4-3: Hydraulic parameters of Division Boxes

### Road crossing structure

6 (Nr) road crossing structures are provided on the Main and Secondary canal, at the Urap road crossing. The road crossing structures are rectangular reinforced concrete slab. The slab is reinforced with 12mm @150mmc/c the length of the slab is 1.5m which is the same as the respective canal bed width, its width varies is and thickness is 20mm . The slab size is 1.5m by B+0.40m where B= canal bed width. For further information see Drawing of Road Crossing

Figure 4-4: typical road crossing section

## Irrigation Infrastructure Bill of Quantities and Cost Estimate

| Bill No-3: Irrigation System |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Item. No. | Description | Unit | Qty | Unit Price/ Birr | Amount/Birr |
| 1.0 | Earth work & compaction |  |  |  |  |
| 1.1 | 20 cm. Clearing on loose & dry soil. | m2 | 4,965.0 | 16.4 | 81,305.7 |
| 1.2 | Excavation of loose soil | m3 | 2,745.0 | 90.4 | 248,124.3 |
| 1.3 | Excavation soft rock/weathered rock | m3 | 1,650.0 | 487.4 | 804,150.1 |
| 1.40 | Excavation Hard rock | m3 | 550.0 | 804.7 | 442,573.9 |
| 1.50 | Back fill with Excavated material | m3 | 1,500.0 | 67.4 | 101,121.1 |
| 1.60 | 5cm thick C-5 lean concrete under the canal bottom slab | m2 | 3,300.0 | 125.0 | 412,500.0 |
| 1.70 | Reinforced C-20 concrete(1:2:3) including form work | m3 | 1,226.0 | 2,761.5 | 3,385,508.8 |
| 1.80 | ф8mm bar | kg | 20,273.9 | 79.0 | 1,601,637.7 |
| 1.90 | Supply and install water stop at the expansion point place at 30m interval | m | 300.0 | 520.0 | 156,000.0 |
|  | Sub Total |  |  |  | 7,232,921.6 |
| 2 | Flume on MC from 0+600-628 ,1+380-1+420 & 2+844-2+896 |  |  |  |  |
| 2.1 | Excavation of loose soil | m3 | 324.0 | 90.4 | 29,286.8 |
| 2.2 | Weathered rock excavation | m3 | 87.2 | 487.4 | 42,512.7 |
| 2.3 | Back fill with Excavated material | m3 | 240.0 | 67.4 | 16,179.4 |
| 2.4 | Massonary work with 1:3 cement sand ratio | m3 | 87.5 | 1,669.6 | 146,126.6 |
| 2.5 | Pointing (1:2) | m2 | 97.5 | 68.4 | 6,667.1 |
| 2.6 | Conceret work C-25 inculding form work | m3 | 77.0 | 2,788.8 | 214,597.1 |
| 2.7 | Lean concrete C10 (1:3:6)with 10cm thick | m3 | 5.9 | 1,960.1 | 11,466.3 |
| 2.8 | Reinforcement |  |  |  | - |
| 2.8.1 | bar 14 Diam | kg | 2,550.0 | 79.0 | 201,446.8 |
| 2.8.2 | bar 12 Diam | kg | 2,137.5 | 79.0 | 168,862.5 |
| 2.8.3 | bar 10 Diam | kg | 527.6 | 79.0 | 41,677.2 |
| 2.8.4 | bar 8 Diam | kg | 471.3 | 79.0 | 37,235.1 |
| 2.8.5 | Water stoper construction jiont | m | 26.7 | 520.0 | 13,884.0 |
|  | Sub Total |  |  |  | 929,941.6 |
| 3 | Super passage |  |  |  |  |
| 3.1 | Slab concrete(C-20) | m3 | 5.6 | 2,519.8 | 14,110.8 |
| 3.2 | ф12mm bar | kg | 139.2 | 79.0 | 10,995.2 |
| 3.3 | Cutt off (C-20 concrete) inculding form work | m3 | 10.4 | 2,761.5 | 28,720.0 |
| 3.4 | Masonry with 1:3 cement sand ratio | m3 | 78.8 | 1,669.6 | 131,483.9 |
| 3.5 | Pointing (1:2) | m2 | 210.0 | 68.4 | 14,355.4 |
| 3.6 | Stone Piching | m3 | 30.0 | 1,192.9 | 35,788.0 |
| 3.7 | Excavation of loose soil | m3 | 78.5 | 90.4 | 7,099.2 |
| 3.8 | Back fill with Excavated material | m3 | 52.7 | 67.4 | 3,551.7 |
|  | Sub Total |  |  |  | 246,104.2 |
| 4 | Urap Road crossing on Mc 1+754 & 3+180 |  |  |  |  |
| 4.1 | Excavation of loose soil | m3 | 66.1 | 90.4 | 5,970.8 |
| 4.2 | Reinforced C-20 concrete(1:2:3) including form work | m3 | 42.2 | 2,761.5 | 116,434.7 |
| 4.3 | (20cm tick,C-20, single reinforcement precast slab with area 0.7mx1.5m) | m3 | 2.4 | 2,761.5 | 6,591.8 |
| 4.4 | RC bar ,12mm dia. | kg | 171.7 | 79.0 | 13,566.0 |
|  | sub total |  |  |  | 142,563.3 |
| 5 | Division Box |  |  |  |  |
| 5.1 | Excavation normal loose soil | m3 | 31.1 | 90.4 | 2,811.5 |
| 5.2 | Back fill with Excavated material | m3 | 8.6 | 67.4 | 582.5 |
| 5.3 | Reinforced C-20 concrete(1:2:3) | m3 | 13.4 | 2,761.5 | 37,026.7 |
| 5.4 | ф8mm bar | kg | 212.4 | 79.0 | 16,778.3 |
| 5.5 | Gate consists of | Pcs | 8.0 | 2,741.2 | 21,929.2 |
|  | 4mm thick sheet metal(0.7m*0.7m) |  |  |  |  |
|  | Stiffening angle iron (50x50x4) |  |  |  |  |
|  | Angle iron for groove (0.8x0.8x10) |  |  |  |  |
|  | 16mm reinforcement bar for handling |  |  |  |  |
|  | Sub Total |  |  |  | 79,128.2 |
| 6 | Turn out for MC |  |  |  |  |
| 6.1 | Excavation normal loose soil | m3 | 36.0 | 90.4 | 3,254.1 |
| 6.2 | Back fill with Excavated material | m3 | 51.6 | 67.4 | 3,478.6 |
| 6.3 | Reinforced C-20 concrete(1:2:3) including form work | m3 | 35.5 | 2,761.5 | 97,924.2 |
| 6.4 | ф8mm bar | kg | 265.0 | 48.7 | 12,912.1 |
| 6.5 | Gate supply, cutting ,welding and installation(0.7mx0.7m,4mm thick sheet metal, 4m long angle iron for groove (30x30x3), 1m long, 12mm reinforcement.bar for handle) | Pcs | 18.0 | 2,090.1 | 37,621.4 |
|  | sub Total |  |  |  | 155,190.4 |
| 1.00 | Earth Work |  |  |  |  |
| 1.1 | Site clearing for trench excavation | m2 | 12,413.0 | 16.4 | 203,272.4 |
| 1.2 | Excavation of Ordinary soil | m3 | 2,606.7 | 90.4 | 235,625.9 |
| 1.3 | Excavation of soft rock | m3 | 1,564.0 | 487.4 | 762,255.3 |
| 1.4 | Excavation of hard rock in the secondary and tertiary pipe lines | m3 | 1,042.7 | 804.7 | 839,033.2 |
|  | Sub Total |  |  |  | 2,040,186.8 |
| 2 | Supply and install secondary and tertiary pipe lines and fittings for farm infrastructure |  |  |  |  |
| 2.1 | Supply and Install HDPE Pipe PE 100 PN 10 |  |  |  |  |
| 2.1.1 | OD 32mm PE 100 PN 10 | m | 175.0 | 71.0 | 12,425.0 |
| 2.1.1 | OD 50mm PE 100 PN 10 | m | 125.0 | 116.5 | 14,562.5 |
| 2.1.2 | OD 63mm PE 100 PN 10 | m | 2,312.0 | 159.0 | 367,608.0 |
| 2.1.3 | OD 75mm PE 100 PN 10 | m | 2,874.0 | 181.5 | 521,631.0 |
| 2.1.4 | OD 90mm PE 100 PN 10 | m | 1,948.0 | 298.0 | 580,504.0 |
| 2.1.5 | OD 110mm PE 100 PN 10 | m | 1,673.0 | 351.0 | 587,223.0 |
| 2.1.6 | OD 125mm PE 100 PN 10 | m | 614.0 | 567.0 | 348,138.0 |
| 2.1.7 | OD 140mm PE 100 PN 10 | m | 1,008.0 | 801.5 | 807,912.0 |
| 2.1.8 | OD 160mm PE 100 PN 10 | m | 540.0 | 925.0 | 499,500.0 |
| 2.1.9 | OD 180mm PE 100 PN 10 | m | 232.0 | 1,241.0 | 287,912.0 |
| 2.1.10 | OD 225mm PE 100 PN 10 | m | 125.0 | 1,665.0 | 208,125.0 |
| 2.1.11 | OD 250mm PE 100 PN 10 | m | 235.0 | 2,085.5 | 490,092.5 |
| 2.1.12 | OD 280mm PE 100 PN 10 | m | 264.0 | 2,250.5 | 594,132.0 |
| 2.1.13 | OD 315mm PE 100 PN 10 | m | 288.0 | 2,668.5 | 768,528.0 |
|  | Sub Total |  |  |  | 6,088,293.0 |
| 2.2 | Supply and install HDPE PE 100 PN 10 Cross Tee and Tee for the following Dimensions |  |  |  |  |
| 2.2.1 | OD 315mm HDPE Cross Tee PN 10 PE 100 | No | 1.0 | 20,500.0 | 20,500.0 |
| 2.2.2 | OD 280mm HDPE Cross Tee PN 10 PE 100 | No | 1.0 | 18,200.0 | 18,200.0 |
| 2.2.3 | OD 250mm HDPE Cross Tee PN10 PE 100 | No | 1.0 | 15,250.0 | 15,250.0 |
| 2.2.4 | OD 225mm HDPE Cross Tee PN 10 PE 100 | No | 1.0 | 11,500.0 | 11,500.0 |
| 2.2.5 | OD 180mm HDPE Cross Tee PN 10 PE 100 | No | 2.0 | 7,600.0 | 15,200.0 |
| 2.2.6 | OD 160mm HDPE Cross Tee PN 10 PE 100 | No | 2.0 | 5,450.0 | 10,900.0 |
| 2.2.7 | OD 140mm HDPE Cross Tee PN 10 PE 100 | No | 3.0 | 5,450.0 | 16,350.0 |
| 2.2.8 | OD 125mm HDPE Cross Tee PN 10 PE 100 | No | 2.0 | 5,450.0 | 10,900.0 |
| 2.2.9 | OD 110mm HDPECross Tee PN 10 PE 100 | No | 1.0 | 4,522.8 | 4,522.8 |
| 2.2.10 | Supply and install 125mm HDPE PE 100 PN 10 but welded Tee | No | 1.0 | 5,400.0 | 5,400.0 |
|  | Sub Total |  |  |  | 128,722.8 |
| 2.4 | Supply and Install HDPE PN 10 PE 100 Butt welded Reducing coupling or compression reducing coupling with the following dimension |  |  |  |  |
| 2.4.1 | But welded reducer OD 315*280 mm | No | 1.0 | 15,000.0 |  |
| 2.4.2 | But welded reducer OD 315*90 mm | No | 1.0 | 11,600.0 | 11,600.0 |
| 2.4.3 | But welded reducer OD 280*250 mm | No | 1.0 | 11,600.0 | 11,600.0 |
| 2.4.4 | But welded reducer OD 280*110 mm | No | 1.0 | 11,300.0 | 11,300.0 |
| 2.4.5 | But welded reducer OD 250*225 mm | No | 1.0 | 11,750.0 | 11,750.0 |
| 2.4.6 | But welded reducer OD 250*110 mm | No | 1.0 | 9,700.0 | 9,700.0 |
| 2.4.7 | But welded reducer Od 225*140 mm | No | 2.0 | 8,200.0 | 16,400.0 |
| 2.4.8 | But welded reducer OD 225*110 mm | No | 1.0 | 8,200.0 | 8,200.0 |
| 2.4.9 | But welded reducer OD 140*90 mm | No | 3.0 | 5,250.0 | 15,750.0 |
| 2.4.10 | But welded reducer OD 160*125 mm | No | 1.0 | 6,000.0 | 6,000.0 |
| 2.4.11 | But welded reducer OD 125*90 mm | No | 2.0 | 4,500.0 | 9,000.0 |
| 2.4.12 | But welded reducer OD 125*110 mm | No | 1.0 | 4,500.0 | 4,500.0 |
| 2.4.13 | But welded reducer OD 160*140 mm | No | 2.0 | 6,100.0 | 12,200.0 |
| 2.4.14 | But welded reducer OD 140*125 mm | No | 1.0 | 6,100.0 | 6,100.0 |
| 2.4.15 | But welded reducer OD 180*160 mm | No | 1.0 | 8,250.0 | 8,250.0 |
| 2.4.16 | But welded reducer OD 160*90 mm | No | 1.0 | 6,000.0 | 6,000.0 |
| 2.4.17 | But welded reducer OD 140*125 mm | No | 1.0 | 6,000.0 | 6,000.0 |
| 2.4.18 | But welded reducer OD 110*63 mm | No | 1.0 | 2,250.0 | 2,250.0 |
| 2.4.19 | But welded reducer OD 180*75 mm | No | 1.0 | 5,650.0 | 5,650.0 |
| 2.4.20 | But welded reducer OD 315*75mm | No | 1.0 | 13,250.0 | 13,250.0 |
| 2.4.21 | But welded reducer OD 250*75 mm | No | 1.0 | 12,450.0 | 12,450.0 |
| 2.4.22 | But welded reducer OD 140*75 mm | No | 4.0 | 5,200.0 | 20,800.0 |
| 2.4.23 | But welded reducer OD 160*75 mm | No | 3.0 | 6,100.0 | 18,300.0 |
| 2.4.24 | But welded reducer OD 125*75 mm | No | 4.0 | 4,550.0 | 18,200.0 |
| 2.4.25 | But welded reducer OD 125*63 mm | No | 1.0 | 2,250.0 | 2,250.0 |
| 2.4.26 | But welded reducer OD 110*50 mm | No | 1.0 | 13,550.0 | 13,550.0 |
| 2.4.27 | But welded reducerOD 315*63 mm | No | 2.0 | 12,300.0 | 24,600.0 |
| 2.4.28 | But welded reducer OD 140*63 mm | No | 5.0 | 5,250.0 | 26,250.0 |
| 2.4.29 | But welded reducer OD 160*63 mm | No | 3.0 | 6,000.0 | 18,000.0 |
| 2.4.30 | But welded educer OD 180*63 mm | No | 2.0 | 5,650.0 | 11,300.0 |
|  | Sub Total |  |  |  | 341,200.0 |
| 6 | Supply and install all flanged gate valve and flanged adapter with back rings |  |  |  |  |
| 2.6.1 | 315mm flanged gate valve | No | 1.0 | 50,800.0 | 50,800.0 |
| 2.6.2 | 315mm flanged adapter with back ring | No | 2.0 | 17,500.0 | 35,000.0 |
| 2.6.3 | 280mm flanged gate valve | No | 1.0 | 50,000.0 | 50,000.0 |
| 2.6.4 | 280mm flanged adapter with back ring | No | 2.0 | 14,500.0 | 29,000.0 |
| 2.6.5 | 250mm flanged gate valve | No | 2.0 | 32,000.0 | 64,000.0 |
| 2.6.6 | 250mm flanged adapter with back ring | No | 2.0 | 11,100.0 | 22,200.0 |
| 2.6.7 | 225mm flanged gate valve | No | 1.0 | 17,000.0 | 17,000.0 |
| 2.6.8 | 225mm flanged adapter with back ring | No | 2.0 | 9,500.0 | 19,000.0 |
| 2.6.9 | 180mm flanged gate valve | No | 1.0 | 16,300.0 | 16,300.0 |
| 2.6.10 | 180mm flanged adapter with back ring | No | 2.0 | 7,550.0 | 15,100.0 |
| 2.6.11 | 160mm flanged gate valve | No | 2.0 | 10,500.0 | 21,000.0 |
| 2.6.12 | 160mm flanged adapter with back ring | No | 4.0 | 6,000.0 | 24,000.0 |
| 2.6.13 | 140mm flanged gate valve | No | 3.0 | 10,500.0 | 31,500.0 |
| 2.6.14 | 140mm flanged adapter with back ring | No | 6.0 | 5,000.0 | 30,000.0 |
| 2.6.15 | 125mm flanged gate valve | No | 3.0 | 9,500.0 | 28,500.0 |
| 2.6.16 | 125mm flanged adapter with back ring | No | 6.0 | 3,500.0 | 21,000.0 |
| 2.6.17 | 110mm flanged gate valve | No | 6.0 | 6,250.0 | 37,500.0 |
| 2.6.18 | 110mm flanged adapter with back ring | No | 12.0 | 3,000.0 | 36,000.0 |
| 2.6.19 | 90mm flanged gate valve | No | 9.0 | 5,200.0 | 46,800.0 |
| 2.6.20 | 90mm flanged adapter with back ring | No | 18.0 | 2,300.0 | 41,400.0 |
| 2.6.21 | 75mm flanged gate valve | No | 14.0 | 4,250.0 | 59,500.0 |
| 2.6.22 | 75mm flanged adapter with back ring | No | 28.0 | 1,850.0 | 51,800.0 |
|  | Sub Total |  |  |  | 747,400.0 |
| 2.7 | Supply and install HDPE PN 10 PE 100 compression female adaptor |  |  |  |  |
| 2.7.1 | Compression female adaptor 50mm HDPE * 1 1/2'' DN GS | No | 2.0 | 285.0 | 570.0 |
| 2.7.2 | compression female adaptor 63mm HDPE * 2'' DN GS | No | 28.0 | 450.0 | 12,600.0 |
| 2.7.3 | Compression female Adapter 75mm *2 1/2'' DN GS | No | 28.0 | 875.0 | 24,500.0 |
|  | Sub Total |  |  |  | - |
| 2.8 | supply and install GS Fittings for traded gate valve |  |  |  | - |
| 2.8.1 | 1 1/2'' DN GS male Nipples | No | 2.0 | 81.0 | 162.0 |
| 2.8.2 | 1 1/2'' DN GS both end traded gate valve | No | 1.0 | 520.0 | 520.0 |
| 2.8.3 | 2'' DN GS Male Nipples | No | 28.0 | 105.0 | 2,940.0 |
| 2.8.4 | 2'' DN GS both end traded gate valve | No | 14.0 | 790.0 | 11,060.0 |
|  | Sub Total |  |  |  | 52,352.0 |
| 2.9 | Fittings for 109 field risers |  |  |  |  |
| 2.9.1 | Supply and install HDPE PN 10 PE 100 Reducer Tee |  |  |  |  |
| 2.9.1.1 | Compression reducer tee 50*32*50 | No | 1.0 | 400.0 | 400.0 |
| 2.9.1.2 | Butt welded reducer tee 63*32*63 | No | 16.0 | 750.0 | 12,000.0 |
| 2.9.1.3 | But welded reducer tee 75*32 *75 | No | 21.0 | 2,500.0 | 52,500.0 |
| 2.9.1.4 | But welded reducer tee 90*32*90 | No | 7.0 | 2,700.0 | 18,900.0 |
| 2.9.1.5 | But welded reducer tee 110*32*110 | No | 6.0 | 3,000.0 | 18,000.0 |
| 2.9.1.6 | But welded reducer tee 140*32*140 | No | 1.0 | 3,800.0 | 3,800.0 |
| 2.9.1.7 | But welded reducer tee 140*32*110 | No | 1.0 | 3,800.0 | 3,800.0 |
| 2.9.1.8 | Supply and install 32mm HDPE female adapter PN 10 PE 100 | No | 109.0 | 90.0 | 9,810.0 |
| 2.9.1.9 | GS 1'' DN Male nipples | No | 109.0 | 90.0 | 9,810.0 |
| 2.9.1.10 | GS 1'DN both end traded gate valve on the GS riser pipe | No | 109.0 | 198.0 | 21,582.0 |
| 2.9.2 | Supply and install HDPE PN 10 PE 100 Elbow |  |  |  | - |
| 2.9.2.1 | 32mm Compression 90 deg elbow at the end of the riser pipe | No | 109.0 | 90.0 | 9,810.0 |
| 2.9.2.2 | 50mm Compression 90 deg elbow | No | 1.0 | 400.0 | 400.0 |
| 2.9.2.3 | 63mm Compression 90 deg elbow | No | 19.0 | 420.0 | 7,980.0 |
| 2.9.2.4 | 75mm but welded 90 deg elbow | No | 15.0 | 1,500.0 | 22,500.0 |
| 2.9.2.5 | 90mm but welded 90 deg elbow | No | 8.0 | 2,100.0 | 16,800.0 |
| 2.9.2.6 | 110mm but welded 90 deg elbow | No | 5.0 | 3,000.0 | 15,000.0 |
| 2.9.2.7 | Supply and Install 1-meter length and 1'' Diam GS Pipe for riser pipes | No | 109.0 | 475.0 | 51,775.0 |
|  | Sub Total |  |  |  | 274,867.0 |
| 10 | Canal Structures |  |  |  |  |
| 10.1 | Standardized Valve Chamber as per the drawings | No | 19.0 | 15,000.0 | 285,000.0 |
| 10.2 | Gravity inlet for pipes from the main canal division box as per the drawing. Cost shall include all the excavation and Concrete works | No | 4.0 | 16,000.0 | 64,000.0 |
| 10.3 | costruct pressure breaker Tank at the necessary secondary canals positions as per the engineering insruction. The breaker tank has all inlet and outlet fittings, slefrequlating float valve(look drawing) | No | 4.0 | 62,000.0 | 248,000.0 |
| 12 | Pipe Inlet | No | 14.0 | 16,000.0 | 224,000.0 |
|  | Sub Total |  |  |  | 821,000.0 |
|  | Total |  |  |  | 19,279,870.9 |

# CONCLUSION AND RECOMMENDATION

- The infrastructure of this project area is designed to irrigate about 215 ha of land by taking its supply from the Workie diversion Weir Diversion irrigation project. The maximum duty of the command area for 16 hours per day irrigation with overall project efficiency of 55%.  The method of irrigation of the project area is furrow surface irrigation in which the main, secondary and tertiary canals are working continuously whereas the field canals within a tertiary block are working rotational system.

- As the dominant soil type is clay loam soil, the main canal system is designed to be masonry.

- The reason why the main canal is to be lined up to the end is to avoid the siltation problem, time saving to reach at the tail part, reduce maintenance cost.

- On the secondary unit of the irrigation systems, some are associated with HDPE Pipe. They

- The design of the canal dimensions of the irrigation canal is done by applying the manning’s uniform flow equation. The variable of the hydraulic parameters are calculated using iteration or flow master program.

- The design discharge of the drainage canals are determined using rational formula and Gamble Powell method.

- As soils of the command area are predominantly clay loam textured; and hence water and soil management measures should be undertaken; and optimum moisture content should be maintained to improve workability of the soil during land preparation and planting time.

The following recommendations are drown:

- For better performance and long service year of the project regular inspection and maintenance is highly required.

- Farmers training, how to operate and maintain the project structures as a whole and available and water resources has a paramount important.

- The irrigation hours per day and per week should be flexible based on base flow amount of each week or month.

- Close supervision of the construction should be made to modify (if need be) each Components of irrigation system based on specific site conditions.

# OPERATION AND MAINTENANCE

## General

The main objective of the operation and maintenance aspect of an irrigation scheme is to facilitate the timely delivery of the required irrigation water to farms and to keep the irrigation system in an optimum operating condition.  This section therefore, discusses the main functions of the subject matter under consideration for the scheme.

## Operation of the Head Works

Operation at the diversion weir mainly focuses on the diversion of a controlled flow of river water, timely cleaning of floating debris in front of Weir Diversion and removal of sediment deposits in front of the weir and Weir Diversion structures.

## Irrigation System Operation

The operation of the irrigation system depends mainly on the method of water delivery at farm level.  Surface irrigation method is the recommended type of water distribution and application method for Workie diversion irrigation scheme.

The farmers would organize themselves and form groups in order to handle the water management.  Since flow is low Rotational water distribution would be applied within the group.  The rotational distribution is then to distribute water by turn to the whole scheme according to the timely need of crop water requirement. For better and efficient water management, crop diversification should be avoided within a group.  This would reduce the complexity of water distribution system of the scheme during one irrigation season. At farmers’ level of operation, a constant flow and variable irrigation time is advisable.

The operation of the irrigation system is continuous for 16 hours per day in main, secondary and tertiary canals whereas field canals within a tertiary block are operating in rotational system with each other for irrigation hours proportional to their size.  Since the tertiary canal discharges are within the manageable range and the irrigation canal structures are accordingly designed for simple operation, the farmers can open and close easily whenever they required.

## Maintenance Requirement

The canal system of the project is Pipe HDPE canal except main canal, which is susceptible to clogging by sediment or siltation, problems. Though the canal sections are designed for non- silting and non-scouring conditions, the above mentioned problems are for to protect siltation problem at each entrance of pipe it’s provided trash rack and at each junction wash out valve.

The maintenance tasks are categorized into two types: - routine activities, and repairs. The routine maintenance activities that are carried out periodically include:-

- Regular cleaning of sediments and weeds from canals and drains;

- Inspection and lubrication of gates; and

- Maintenance of cracked lined canals, regulating and control structures.

Repair works include task carried out more frequently and quickly, and include those tasks that are generally unpredictable.  They also include emergency works.  The activities included in this category are:-

- Repairing overtopped or breached canals, drains, and flood protection dykes;

- Repairing jammed gates;

- Filling holes made by wild animals; and

- Reduced free board due to walking over by people and livestock.

Regular inspection of the irrigation facilities should be carried out as part of the maintenance activities.  These tasks could be carried out immediately after the end of the main rains in September and during the rainy season.  This could concentrate on the interceptor drains and the flood protection dykes, the main canal and the field drains.  The inspection of the other works like the tertiary canals, field drains, and the water control and regulating structures could be carried out as part of routine operation activities.

- Beneficiaries of the project need to have operation and maintenance budget, For O&M cost incurring entity area:

- Purchase sing of gate lubricate (grease)

- Replacing and maintenance of Stolen and damaged gates

- Repair Damages on the cross drainage structures.

The expense for O&M should be collected from the beneficiaries. Of course, much of the task is done by the labor and skill of the community. For cost incurring activities beneficiaries have to collect money based on the proportion of the command area they owned.

# Annex: Junction and Pipe report for the farm system

Table 0-1: pipe report of the farm system

| Label | Length (Scaled) (m) | Start Node | Stop Node | OD (mm) | Flow (L/s) | Velocity (m/s) | Material | Hazen-Williams C |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| P-232 | 32 | O7 | FV-O7-1 | 32 | 1 | 1.29 | HDPE | 150 |
| P-242 | 20 | O13 | FV-O13-1 | 32 | 1 | 1.77 | HDPE | 150 |
| P-241 | 17 | O14 | FV-O14-1 | 32 | 1 | 1.77 | HDPE | 150 |
| P-236 | 24 | O8 | FV-O8-1 | 32 | 1 | 1.29 | HDPE | 150 |
| P-235 | 17 | O11 | FV-O11-1 | 32 | 1 | 1.77 | HDPE | 150 |
| P-234 | 23 | O10 | FV-O10-1 | 32 | 1 | 1.77 | HDPE | 150 |
| P-233 | 26 | O9 | FV-O9-1 | 32 | 1 | 1.29 | HDPE | 150 |
| P-240 | 16 | O12 | FV-O12-1 | 32 | 1 | 0.97 | HDPE | 150 |
| P-15 | 37 | J-6 | FV-1-1-16 | 50 | 1 | 0.75 | HDPE | 150 |
| P-14 | 38 | FV-1-1-16 | FV-1-1-17 | 50 | 1 | 0.75 | HDPE | 150 |
| P-43 | 55 | J-15 | FV-1-1-11 | 63 | 2 | 1 | HDPE | 150 |
| P-47 | 46 | J-16 | FV-1-1-15 | 63 | 3 | 1.3 | HDPE | 150 |
| P-42 | 63 | FV-1-1-10 | J-15 | 63 | 2 | 1 | HDPE | 150 |
| P-105 | 43 | J-56 | FV-1-2-12 | 63 | 2 | 0.91 | HDPE | 150 |
| P-21 | 37 | J-21 | FV-1-1-20 | 63 | 2 | 0.84 | HDPE | 150 |
| P-46 | 22 | J-13 | J-18 | 63 | 3 | 1.1 | HDPE | 150 |
| P-19 | 78 | J-6 | FV-1-1-18 | 63 | 2 | 0.87 | HDPE | 150 |
| P-44 | 33 | FV-1-1-12 | J-13 | 63 | 3 | 1.1 | HDPE | 150 |
| P-174 | 55 | J-60 | FV-1-3-2 | 63 | 2 | 0.71 | HDPE | 150 |
| P-45 | 70 | J-18 | FV-1-1-13 | 63 | 3 | 1.1 | HDPE | 150 |
| P-41 | 51 | J-4 | FV-1-1-10 | 63 | 2 | 1 | HDPE | 150 |
| P-32 | 30 | J-2 | FV-1-1-5 | 63 | 3 | 1.31 | HDPE | 150 |
| P-30 | 37 | J-11 | FV-1-1-3 | 63 | 2 | 0.84 | HDPE | 150 |
| P-31 | 77 | J-1 | J-11 | 63 | 2 | 0.84 | HDPE | 150 |
| P-50 | 65 | J-4 | FV-1-1-12 | 63 | 3 | 1.1 | HDPE | 150 |
| P-25 | 41 | FV-1-1-21 | J-23 | 63 | 2 | 0.84 | HDPE | 150 |
| P-22 | 36 | J-20 | J-21 | 63 | 2 | 0.84 | HDPE | 150 |
| P-23 | 29 | J-7 | J-20 | 63 | 2 | 0.84 | HDPE | 150 |
| P-11 | 33 | J-7 | FV-1-1-23 | 63 | 2 | 0.84 | HDPE | 150 |
| P-26 | 28 | J-7 | J-22 | 63 | 2 | 0.84 | HDPE | 150 |
| P-20 | 27 | J-14 | FV-1-1-19 | 63 | 2 | 0.87 | HDPE | 150 |
| P-17 | 38 | J-19 | J-14 | 63 | 2 | 0.87 | HDPE | 150 |
| P-61 | 99 | FV-1-2-10 | FV-1-2-11 | 63 | 2 | 0.71 | HDPE | 150 |
| P-24 | 23 | J-22 | FV-1-1-21 | 63 | 2 | 0.84 | HDPE | 150 |
| P-49 | 52 | J-5 | FV-1-1-14 | 63 | 3 | 1.3 | HDPE | 150 |
| P-28 | 28 | J-23 | J-24 | 63 | 2 | 0.84 | HDPE | 150 |
| P-119 | 63 | J-60 | FV-1-3-1 | 63 | 3 | 1.13 | HDPE | 150 |
| P-29 | 92 | FV-1-1-3 | FV-1-1-4 | 63 | 2 | 0.84 | HDPE | 150 |
| P-27 | 23 | J-24 | FV-1-1-22 | 63 | 2 | 0.84 | HDPE | 150 |
| P-12 | 38 | FV-1-1-1 | FV-1-1-2 | 63 | 2 | 0.79 | HDPE | 150 |
| P-48 | 66 | FV-1-1-14 | J-16 | 63 | 3 | 1.3 | HDPE | 150 |
| P-18 | 29 | FV-1-1-18 | J-19 | 63 | 2 | 0.87 | HDPE | 150 |
| P-227 | 116 | FV-O4-1 | FV-O4-2 | 63 | 2 | 0.79 | HDPE | 150 |
| P-207 | 24 | J-116 | FV-1-4-1 | 63 | 2 | 0.92 | HDPE | 150 |
| P-208 | 23 | DB4 | J-116 | 63 | 2 | 0.92 | HDPE | 150 |
| P-58 | 82 | FV-1-2-3 | FV-1-2-4 | 63 | 2 | 0.91 | HDPE | 150 |
| P-224 | 26 | J-120 | FV-O5-2 | 63 | 2 | 0.79 | HDPE | 150 |
| P-226 | 18 | O4 | FV-O4-1 | 63 | 2 | 0.79 | HDPE | 150 |
| P-57 | 51 | FV-1-2-12 | FV-1-2-3 | 63 | 2 | 0.91 | HDPE | 150 |
| P-13 | 41 | J-1 | FV-1-1-1 | 63 | 2 | 0.79 | HDPE | 150 |
| P-230 | 31 | O3 | FV-O3-1 | 63 | 1 | 0.59 | HDPE | 150 |
| P-231 | 44 | FV-O3-1 | FV-O3-2 | 63 | 1 | 0.59 | HDPE | 150 |
| P-238 | 61 | FV-O6-1 | FV-O6-2 | 63 | 2 | 0.76 | HDPE | 150 |
| P-195 | 46 | FV-1-4-1 | FV-1-4-2 | 63 | 2 | 0.92 | HDPE | 150 |
| P-228 | 33 | O5 | FV-O5-1 | 63 | 2 | 0.79 | HDPE | 150 |
| P-229 | 42 | FV-O5-1 | J-120 | 63 | 2 | 0.79 | HDPE | 150 |
| P-237 | 46 | O6 | FV-O6-1 | 63 | 2 | 0.76 | HDPE | 150 |
| P-60 | 53 | J-32 | FV-1-2-10 | 63 | 2 | 0.71 | HDPE | 150 |
| P-225 | 68 | FV-O5-2 | FV-O5-3 | 63 | 2 | 0.79 | HDPE | 150 |
| P-39 | 30 | J-25 | J-26 | 75 | 4 | 1.18 | HDPE | 150 |
| P-40 | 38 | J-28 | FV-1-1-7 | 75 | 4 | 1.18 | HDPE | 150 |
| P-68 | 28 | J-40 | FV-1-2-14 | 75 | 4 | 1.07 | HDPE | 150 |
| P-66 | 24 | J-39 | FV-1-2-6 | 75 | 4 | 1.16 | HDPE | 150 |
| P-37 | 32 | J-27 | J-28 | 75 | 4 | 1.18 | HDPE | 150 |
| P-38 | 30 | FV-1-1-24 | J-27 | 75 | 4 | 1.18 | HDPE | 150 |
| P-33 | 47 | J-26 | FV-1-1-24 | 75 | 4 | 1.18 | HDPE | 150 |
| P-65 | 46 | FV-1-2-5 | FV-1-2-1 | 75 | 4 | 1.16 | HDPE | 150 |
| P-36 | 52 | FV-1-1-6 | J-25 | 75 | 4 | 1.18 | HDPE | 150 |
| P-35 | 22 | J-12 | FV-1-1-6 | 75 | 4 | 1.18 | HDPE | 150 |
| P-104 | 29 | J-57 | FV-1-2-5 | 75 | 4 | 1.16 | HDPE | 150 |
| P-120 | 71 | FV-1-3-6 | FV-1-3-5 | 75 | 3 | 0.98 | HDPE | 150 |
| P-64 | 50 | J-31 | FV-1-2-24 | 75 | 5 | 1.32 | HDPE | 150 |
| P-63 | 32 | J-36 | J-37 | 75 | 3 | 1.01 | HDPE | 150 |
| P-94 | 59 | J-52 | FV-1-2-13 | 75 | 4 | 1.07 | HDPE | 150 |
| P-80 | 53 | J-45 | FV-1-2-18 | 75 | 3 | 1.01 | HDPE | 150 |
| P-55 | 28 | J-33 | FV-1-2-15 | 75 | 4 | 1.18 | HDPE | 150 |
| P-83 | 41 | FV-1-2-25 | J-47 | 75 | 3 | 0.74 | HDPE | 150 |
| P-84 | 18 | J-47 | J-38 | 75 | 3 | 0.74 | HDPE | 150 |
| P-85 | 21 | J-38 | J-48 | 75 | 3 | 0.74 | HDPE | 150 |
| P-88 | 31 | J-36 | FV-1-2-21 | 75 | 4 | 1.13 | HDPE | 150 |
| P-147 | 52 | J-89 | FV-1-3-30 | 75 | 4 | 1.27 | HDPE | 150 |
| P-79 | 10 | FV-1-2-19 | J-45 | 75 | 3 | 1.01 | HDPE | 150 |
| P-78 | 24 | J-44 | FV-1-2-19 | 75 | 3 | 1.01 | HDPE | 150 |
| P-77 | 42 | FV-1-2-20 | J-44 | 75 | 3 | 1.01 | HDPE | 150 |
| P-76 | 21 | J-37 | FV-1-2-20 | 75 | 3 | 1.01 | HDPE | 150 |
| P-90 | 44 | J-50 | FV-1-2-2 | 75 | 4 | 1.13 | HDPE | 150 |
| P-103 | 31 | J-56 | J-57 | 75 | 4 | 1.16 | HDPE | 150 |
| P-93 | 43 | FV-1-2-14 | J-52 | 75 | 4 | 1.07 | HDPE | 150 |
| P-89 | 48 | FV-1-2-21 | J-50 | 75 | 4 | 1.13 | HDPE | 150 |
| P-95 | 16 | J-31 | J-53 | 75 | 3 | 0.74 | HDPE | 150 |
| P-96 | 23 | J-53 | FV-1-2-25 | 75 | 3 | 0.74 | HDPE | 150 |
| P-97 | 5 | J-48 | J-54 | 75 | 3 | 0.74 | HDPE | 150 |
| P-98 | 20 | J-54 | FV-1-2-26 | 75 | 3 | 0.74 | HDPE | 150 |
| P-99 | 22 | FV-1-2-1 | J-55 | 75 | 4 | 1.16 | HDPE | 150 |
| P-75 | 15 | J-40 | J-33 | 75 | 4 | 1.18 | HDPE | 150 |
| P-74 | 41 | J-43 | FV-1-2-17 | 75 | 4 | 1.18 | HDPE | 150 |
| P-73 | 40 | FV-1-2-16 | J-43 | 75 | 4 | 1.18 | HDPE | 150 |
| P-72 | 26 | J-42 | FV-1-2-16 | 75 | 4 | 1.18 | HDPE | 150 |
| P-71 | 26 | J-41 | J-42 | 75 | 4 | 1.18 | HDPE | 150 |
| P-70 | 19 | FV-1-2-15 | J-41 | 75 | 4 | 1.18 | HDPE | 150 |
| P-56 | 82 | FV-1-2-24 | FV-1-2-23 | 75 | 5 | 1.32 | HDPE | 150 |
| P-100 | 37 | J-55 | J-39 | 75 | 4 | 1.16 | HDPE | 150 |
| P-91 | 120 | FV-1-2-2 | FV-1-2-22 | 75 | 4 | 1.13 | HDPE | 150 |
| P-155 | 18 | J-65 | J-94 | 75 | 3 | 0.98 | HDPE | 150 |
| P-182 | 7 | J-107 | J-108 | 75 | 3 | 0.98 | HDPE | 150 |
| P-184 | 45 | J-109 | FV-1-3-32 | 75 | 3 | 0.98 | HDPE | 150 |
| P-181 | 54 | FV-1-3-5 | J-107 | 75 | 3 | 0.98 | HDPE | 150 |
| P-34 | 49 | J-2 | J-12 | 75 | 4 | 1.18 | HDPE | 150 |
| P-126 | 80 | J-63 | J-76 | 75 | 5 | 1.33 | HDPE | 150 |
| P-198 | 110 | J-111 | FV-1-4-7 | 75 | 5 | 1.41 | HDPE | 150 |
| P-199 | 120 | FV-1-4-7 | FV-1-4-8 | 75 | 5 | 1.41 | HDPE | 150 |
| P-192 | 6 | J-113 | FV-1-4-13 | 75 | 4 | 1.09 | HDPE | 150 |
| P-203 | 41 | J-113 | FV-1-4-11 | 75 | 4 | 1.04 | HDPE | 150 |
| P-204 | 53 | FV-1-4-11 | J-114 | 75 | 4 | 1.04 | HDPE | 150 |
| P-183 | 14 | J-108 | J-109 | 75 | 3 | 0.98 | HDPE | 150 |
| P-156 | 54 | J-94 | FV-1-3-6 | 75 | 3 | 0.98 | HDPE | 150 |
| P-206 | 49 | J-115 | FV-1-4-12 | 75 | 4 | 1.04 | HDPE | 150 |
| P-140 | 22 | J-83 | J-84 | 75 | 4 | 1.27 | HDPE | 150 |
| P-127 | 39 | J-76 | FV-1-3-15 | 75 | 5 | 1.33 | HDPE | 150 |
| P-221 | 63 | FV-O1-1 | FV-O1-2 | 75 | 3 | 0.75 | HDPE | 150 |
| P-220 | 56 | O1 | FV-O1-1 | 75 | 3 | 0.75 | HDPE | 150 |
| P-223 | 25 | FV-O2-1 | FV-O2-2 | 75 | 2 | 0.62 | HDPE | 150 |
| P-222 | 31 | O2 | FV-O2-1 | 75 | 2 | 0.62 | HDPE | 150 |
| P-205 | 34 | J-114 | J-115 | 75 | 4 | 1.04 | HDPE | 150 |
| P-139 | 47 | J-64 | J-83 | 75 | 4 | 1.27 | HDPE | 150 |
| P-146 | 24 | J-88 | J-89 | 75 | 4 | 1.27 | HDPE | 150 |
| P-144 | 132 | J-86 | J-87 | 75 | 4 | 1.27 | HDPE | 150 |
| P-141 | 46 | J-84 | J-85 | 75 | 4 | 1.27 | HDPE | 150 |
| P-142 | 25 | J-85 | FV-1-3-29 | 75 | 4 | 1.27 | HDPE | 150 |
| P-143 | 23 | FV-1-3-29 | J-86 | 75 | 4 | 1.27 | HDPE | 150 |
| P-145 | 18 | J-87 | J-88 | 75 | 4 | 1.27 | HDPE | 150 |
| P-210 | 32 | J-117 | J-118 | 90 | 5 | 1.09 | HDPE | 150 |
| P-209 | 20 | DB4 | J-117 | 90 | 5 | 1.09 | HDPE | 150 |
| P-185 | 24 | J-95 | FV-1-3-7 | 90 | 6 | 1.13 | HDPE | 150 |
| P-54 | 144 | FV-1-2-8 | FV-1-2-9 | 90 | 5 | 1.02 | HDPE | 150 |
| P-211 | 21 | J-118 | FV-1-4-3 | 90 | 5 | 1.09 | HDPE | 150 |
| P-194 | 69 | J-112 | FV-1-4-4 | 90 | 5 | 1.09 | HDPE | 150 |
| P-202 | 83 | FV-1-4-10 | FV-1-4-9 | 90 | 6 | 1.17 | HDPE | 150 |
| P-196 | 46 | J-111 | FV-1-4-6 | 90 | 7 | 1.38 | HDPE | 150 |
| P-197 | 123 | FV-1-4-6 | FV-1-4-5 | 90 | 7 | 1.38 | HDPE | 150 |
| P-201 | 112 | J-113 | FV-1-4-10 | 90 | 6 | 1.17 | HDPE | 150 |
| P-193 | 77 | FV-1-4-3 | J-112 | 90 | 5 | 1.09 | HDPE | 150 |
| P-165 | 39 | J-82 | FV-1-3-28 | 90 | 7 | 1.33 | HDPE | 150 |
| P-135 | 15 | J-64 | J-79 | 90 | 7 | 1.33 | HDPE | 150 |
| P-136 | 51 | J-79 | J-80 | 90 | 7 | 1.33 | HDPE | 150 |
| P-137 | 44 | J-80 | J-81 | 90 | 7 | 1.33 | HDPE | 150 |
| P-138 | 11 | J-81 | J-82 | 90 | 7 | 1.33 | HDPE | 150 |
| P-118 | 148 | J-70 | FV-1-3-31 | 90 | 7 | 1.37 | HDPE | 150 |
| P-153 | 15 | J-90 | J-93 | 90 | 7 | 1.37 | HDPE | 150 |
| P-154 | 67 | J-93 | J-70 | 90 | 7 | 1.37 | HDPE | 150 |
| P-51 | 74 | J-32 | FV-1-2-7 | 90 | 5 | 1.02 | HDPE | 150 |
| P-173 | 32 | J-104 | FV-1-3-26 | 90 | 7 | 1.33 | HDPE | 150 |
| P-157 | 81 | J-65 | J-95 | 90 | 6 | 1.13 | HDPE | 150 |
| P-86 | 35 | J-31 | J-49 | 90 | 6 | 1.28 | HDPE | 150 |
| P-166 | 12 | FV-1-3-28 | J-99 | 90 | 7 | 1.33 | HDPE | 150 |
| P-167 | 30 | J-99 | J-101 | 90 | 7 | 1.33 | HDPE | 150 |
| P-168 | 17 | J-101 | J-102 | 90 | 7 | 1.33 | HDPE | 150 |
| P-169 | 22 | J-102 | J-100 | 90 | 7 | 1.33 | HDPE | 150 |
| P-170 | 66 | J-100 | FV-1-3-27 | 90 | 7 | 1.33 | HDPE | 150 |
| P-171 | 31 | FV-1-3-27 | J-103 | 90 | 7 | 1.33 | HDPE | 150 |
| P-172 | 65 | J-103 | J-104 | 90 | 7 | 1.33 | HDPE | 150 |
| P-148 | 115 | J-64 | J-90 | 90 | 7 | 1.37 | HDPE | 150 |
| P-87 | 79 | J-49 | FV-1-2-27 | 90 | 6 | 1.28 | HDPE | 150 |
| P-53 | 148 | FV-1-2-7 | FV-1-2-8 | 90 | 5 | 1.02 | HDPE | 150 |
| P-10 | 200 | J-6 | J-7 | 110 | 6 | 0.83 | HDPE | 150 |
| P-162 | 33 | J-97 | J-71 | 110 | 7 | 1.02 | HDPE | 150 |
| P-159 | 52 | FV-1-3-13 | J-75 | 110 | 8 | 1.14 | HDPE | 150 |
| P-160 | 25 | J-62 | J-96 | 110 | 7 | 1.02 | HDPE | 150 |
| P-116 | 168 | FV-1-3-23 | FV-1-3-24 | 110 | 9 | 1.26 | HDPE | 150 |
| P-161 | 32 | J-96 | J-97 | 110 | 7 | 1.02 | HDPE | 150 |
| P-131 | 142 | FV-1-3-17 | FV-1-3-18 | 110 | 8 | 1.15 | HDPE | 150 |
| P-130 | 184 | FV-1-3-16 | FV-1-3-17 | 110 | 8 | 1.15 | HDPE | 150 |
| P-129 | 80 | J-77 | FV-1-3-16 | 110 | 8 | 1.15 | HDPE | 150 |
| P-128 | 42 | J-63 | J-77 | 110 | 8 | 1.15 | HDPE | 150 |
| P-125 | 107 | J-75 | FV-1-3-14 | 110 | 8 | 1.14 | HDPE | 150 |
| P-124 | 64 | J-62 | J-74 | 110 | 8 | 1.14 | HDPE | 150 |
| P-123 | 54 | J-73 | FV-1-3-11 | 110 | 7 | 1.02 | HDPE | 150 |
| P-122 | 40 | J-72 | J-73 | 110 | 7 | 1.02 | HDPE | 150 |
| P-158 | 51 | J-74 | FV-1-3-13 | 110 | 8 | 1.14 | HDPE | 150 |
| P-117 | 139 | FV-1-3-24 | FV-1-3-25 | 110 | 9 | 1.26 | HDPE | 150 |
| P-164 | 37 | J-98 | FV-1-3-12 | 110 | 7 | 1.02 | HDPE | 150 |
| P-175 | 59 | J-68 | FV-1-3-21 | 110 | 8 | 1.11 | HDPE | 150 |
| P-176 | 90 | FV-1-3-21 | FV-1-3-20 | 110 | 8 | 1.11 | HDPE | 150 |
| P-163 | 19 | J-71 | J-98 | 110 | 7 | 1.02 | HDPE | 150 |
| P-121 | 55 | FV-1-3-12 | J-72 | 110 | 7 | 1.02 | HDPE | 150 |
| P-9 | 65 | J-10 | J-6 | 125 | 9 | 0.98 | HDPE | 150 |
| P-200 | 205 | J-111 | J-113 | 125 | 13 | 1.37 | HDPE | 150 |
| P-8 | 80 | J-5 | J-10 | 125 | 9 | 0.98 | HDPE | 150 |
| P-82 | 67 | J-46 | J-31 | 125 | 13 | 1.4 | HDPE | 150 |
| P-81 | 137 | J-36 | J-46 | 125 | 13 | 1.4 | HDPE | 150 |
| P-7 | 60 | J-4 | J-5 | 125 | 12 | 1.31 | HDPE | 150 |
| P-178 | 48 | FV-1-3-22 | J-105 | 140 | 18 | 1.49 | HDPE | 150 |
| P-4 | 81 | J-2 | J-3 | 140 | 17 | 1.46 | HDPE | 150 |
| P-5 | 58 | J-3 | J-8 | 140 | 17 | 1.46 | HDPE | 150 |
| P-179 | 53 | J-105 | J-106 | 140 | 18 | 1.49 | HDPE | 150 |
| P-177 | 100 | J-68 | FV-1-3-22 | 140 | 18 | 1.49 | HDPE | 150 |
| P-6 | 64 | J-8 | J-4 | 140 | 17 | 1.46 | HDPE | 150 |
| P-133 | 52 | J-69 | J-78 | 140 | 18 | 1.47 | HDPE | 150 |
| P-180 | 58 | J-106 | FV-1-3-23 | 140 | 18 | 1.49 | HDPE | 150 |
| P-134 | 7 | J-78 | J-64 | 140 | 18 | 1.47 | HDPE | 150 |
| P-62 | 135 | J-35 | J-36 | 140 | 21 | 1.72 | HDPE | 150 |
| P-132 | 158 | J-68 | J-69 | 140 | 18 | 1.47 | HDPE | 150 |
| P-69 | 194 | J-40 | J-35 | 140 | 21 | 1.72 | HDPE | 150 |
| P-67 | 93 | J-30 | J-40 | 160 | 28 | 1.81 | HDPE | 150 |
| P-190 | 14 | DB4 | J-110 | 160 | 25 | 1.58 | HDPE | 150 |
| P-3 | 84 | J-9 | J-2 | 160 | 25 | 1.58 | HDPE | 150 |
| P-191 | 110 | J-110 | J-111 | 160 | 25 | 1.58 | HDPE | 150 |
| P-52 | 141 | J-32 | J-30 | 160 | 28 | 1.81 | HDPE | 150 |
| P-2 | 78 | J-1 | J-9 | 160 | 25 | 1.58 | HDPE | 150 |
| P-1 | 20 | DVB 1 | J-1 | 160 | 28 | 1.83 | HDPE | 150 |
| P-102 | 67 | J-56 | J-51 | 180 | 35 | 1.77 | HDPE | 150 |
| P-101 | 7 | DB2 | J-56 | 180 | 41 | 2.08 | HDPE | 150 |
| P-92 | 72 | J-51 | J-34 | 180 | 35 | 1.77 | HDPE | 150 |
| P-59 | 86 | J-34 | J-32 | 180 | 35 | 1.77 | HDPE | 150 |
| P-115 | 125 | J-63 | J-68 | 225 | 43 | 1.41 | HDPE | 150 |
| P-150 | 23 | J-91 | J-67 | 250 | 56 | 1.48 | HDPE | 150 |
| P-149 | 82 | J-62 | J-91 | 250 | 56 | 1.48 | HDPE | 150 |
| P-114 | 130 | J-67 | J-63 | 250 | 56 | 1.48 | HDPE | 150 |
| P-112 | 13 | J-65 | J-61 | 280 | 72 | 1.51 | HDPE | 150 |
| P-113 | 173 | J-66 | J-62 | 280 | 72 | 1.51 | HDPE | 150 |
| P-151 | 24 | J-61 | J-92 | 280 | 72 | 1.51 | HDPE | 150 |
| P-152 | 54 | J-92 | J-66 | 280 | 72 | 1.51 | HDPE | 150 |
| P-111 | 266 | J-60 | J-65 | 315 | 81 | 1.34 | HDPE | 150 |
| P-110 | 22 | DB3 | J-60 | 315 | 85 | 1.41 | HDPE | 150 |

Table 0-2: Junction report

| Label | Elevation (m) | Hydraulic Grade (m) | Pressure (m H2O) | X (m) | Y (m) |
| --- | --- | --- | --- | --- | --- |
| FV-1-1-1 | 1,676.18 | 1,680.69 | 4.5 | 598,282.05 | 1,189,332.86 |
| FV-1-1-10 | 1,621.53 | 1,674.84 | 53.2 | 597,886.05 | 1,189,266.16 |
| FV-1-1-11 | 1,616.20 | 1,672.61 | 56.3 | 597,871.21 | 1,189,153.16 |
| FV-1-1-12 | 1,625.01 | 1,674.35 | 49.2 | 597,932.16 | 1,189,372.61 |
| FV-1-1-13 | 1,625.67 | 1,671.57 | 45.8 | 597,869.40 | 1,189,450.92 |
| FV-1-1-14 | 1,616.74 | 1,673.39 | 56.5 | 597,807.69 | 1,189,341.02 |
| FV-1-1-15 | 1,616.67 | 1,669.96 | 53.2 | 597,707.12 | 1,189,389.44 |
| FV-1-1-16 | 1,606.94 | 1,673.28 | 66.2 | 597,735.90 | 1,189,249.84 |
| FV-1-1-17 | 1,606.64 | 1,672.73 | 66 | 597,742.08 | 1,189,212.58 |
| FV-1-1-18 | 1,606.38 | 1,672.68 | 66.2 | 597,631.66 | 1,189,295.44 |
| FV-1-1-19 | 1,606.32 | 1,671.32 | 64.9 | 597,578.23 | 1,189,359.52 |
| FV-1-1-2 | 1,674.78 | 1,680.23 | 5.4 | 598,288.33 | 1,189,295.45 |
| FV-1-1-20 | 1,597.01 | 1,671.04 | 73.9 | 597,623.54 | 1,189,193.47 |
| FV-1-1-21 | 1,597.15 | 1,671.74 | 74.4 | 597,506.90 | 1,189,241.42 |
| FV-1-1-22 | 1,597.12 | 1,670.49 | 73.2 | 597,513.47 | 1,189,328.82 |
| FV-1-1-23 | 1,594.52 | 1,671.98 | 77.3 | 597,491.16 | 1,189,182.31 |
| FV-1-1-24 | 1,648.10 | 1,674.70 | 26.5 | 598,017.67 | 1,189,487.28 |
| FV-1-1-3 | 1,672.93 | 1,679.67 | 6.7 | 598,165.71 | 1,189,417.83 |
| FV-1-1-4 | 1,672.18 | 1,678.43 | 6.2 | 598,141.10 | 1,189,506.96 |
| FV-1-1-5 | 1,648.34 | 1,677.90 | 29.5 | 598,123.89 | 1,189,336.30 |
| FV-1-1-6 | 1,648.75 | 1,677.35 | 28.5 | 598,080.59 | 1,189,424.92 |
| FV-1-1-7 | 1,648.44 | 1,672.64 | 24.1 | 597,947.32 | 1,189,543.44 |
| FV-1-2-1 | 1,672.82 | 1,678.24 | 5.4 | 597,802.05 | 1,190,064.35 |
| FV-1-2-10 | 1,626.87 | 1,676.30 | 49.3 | 597,656.73 | 1,189,839.04 |
| FV-1-2-11 | 1,620.22 | 1,675.33 | 55 | 597,657.65 | 1,189,937.66 |
| FV-1-2-12 | 1,675.34 | 1,679.69 | 4.3 | 597,797.28 | 1,189,917.54 |
| FV-1-2-13 | 1,609.39 | 1,670.16 | 60.6 | 597,577.81 | 1,189,501.16 |
| FV-1-2-14 | 1,609.65 | 1,671.92 | 62.1 | 597,527.98 | 1,189,579.36 |
| FV-1-2-15 | 1,609.64 | 1,671.53 | 61.8 | 597,497.03 | 1,189,641.67 |
| FV-1-2-16 | 1,609.42 | 1,670.08 | 60.5 | 597,529.87 | 1,189,701.99 |
| FV-1-2-17 | 1,609.49 | 1,668.43 | 58.8 | 597,579.24 | 1,189,763.01 |
| FV-1-2-18 | 1,595.96 | 1,662.98 | 66.9 | 597,434.26 | 1,189,343.49 |
| FV-1-2-19 | 1,595.95 | 1,663.96 | 67.9 | 597,398.63 | 1,189,294.65 |
| FV-1-2-2 | 1,595.84 | 1,663.44 | 67.5 | 597,327.27 | 1,189,460.97 |
| FV-1-2-20 | 1,596.02 | 1,664.98 | 68.8 | 597,333.33 | 1,189,302.93 |
| FV-1-2-21 | 1,596.06 | 1,665.20 | 69 | 597,297.01 | 1,189,375.46 |
| FV-1-2-22 | 1,595.92 | 1,661.14 | 65.1 | 597,380.04 | 1,189,568.48 |
| FV-1-2-23 | 1,580.72 | 1,659.24 | 78.4 | 597,289.68 | 1,189,137.16 |
| FV-1-2-24 | 1,580.74 | 1,661.34 | 80.4 | 597,217.59 | 1,189,176.36 |
| FV-1-2-25 | 1,580.68 | 1,662.30 | 81.4 | 597,190.56 | 1,189,228.04 |
| FV-1-2-26 | 1,580.58 | 1,661.38 | 80.6 | 597,196.89 | 1,189,312.49 |
| FV-1-2-27 | 1,578.05 | 1,660.42 | 82.2 | 597,066.11 | 1,189,150.22 |
| FV-1-2-3 | 1,674.81 | 1,678.88 | 4.1 | 597,796.56 | 1,189,866.22 |
| FV-1-2-4 | 1,672.05 | 1,677.58 | 5.5 | 597,773.09 | 1,189,787.28 |
| FV-1-2-5 | 1,672.94 | 1,679.17 | 6.2 | 597,790.49 | 1,190,019.66 |
| FV-1-2-6 | 1,670.00 | 1,676.56 | 6.6 | 597,787.11 | 1,190,144.93 |
| FV-1-2-7 | 1,629.73 | 1,675.88 | 46.1 | 597,640.53 | 1,189,713.02 |
| FV-1-2-8 | 1,626.23 | 1,673.97 | 47.6 | 597,707.63 | 1,189,580.81 |
| FV-1-2-9 | 1,625.00 | 1,672.13 | 47 | 597,813.43 | 1,189,483.63 |
| FV-1-3-1 | 1,667.29 | 1,671.38 | 4.1 | 597,448.38 | 1,190,314.46 |
| FV-1-3-11 | 1,627.23 | 1,666.73 | 39.4 | 597,489.33 | 1,189,945.32 |
| FV-1-3-12 | 1,627.28 | 1,668.23 | 40.9 | 597,364.22 | 1,189,910.38 |
| FV-1-3-13 | 1,627.25 | 1,668.28 | 40.9 | 597,149.76 | 1,189,921.89 |
| FV-1-3-14 | 1,626.77 | 1,666.31 | 39.5 | 597,077.23 | 1,190,061.90 |
| FV-1-3-15 | 1,604.57 | 1,664.80 | 60.1 | 597,215.60 | 1,189,610.04 |
| FV-1-3-16 | 1,604.10 | 1,666.34 | 62.1 | 597,040.53 | 1,189,775.22 |
| FV-1-3-17 | 1,603.95 | 1,664.01 | 59.9 | 596,965.60 | 1,189,943.53 |
| FV-1-3-18 | 1,603.50 | 1,662.21 | 58.6 | 596,890.32 | 1,190,063.91 |
| FV-1-3-2 | 1,669.40 | 1,672.30 | 2.9 | 597,371.81 | 1,190,374.38 |
| FV-1-3-20 | 1,590.29 | 1,665.13 | 74.7 | 597,121.20 | 1,189,496.74 |
| FV-1-3-21 | 1,590.36 | 1,666.19 | 75.7 | 597,064.47 | 1,189,566.61 |
| FV-1-3-22 | 1,590.42 | 1,665.37 | 74.8 | 596,955.12 | 1,189,681.96 |
| FV-1-3-23 | 1,590.20 | 1,662.95 | 72.6 | 596,879.01 | 1,189,804.01 |
| FV-1-3-24 | 1,590.32 | 1,660.46 | 70 | 596,791.61 | 1,189,947.06 |
| FV-1-3-25 | 1,589.92 | 1,658.41 | 68.3 | 596,727.70 | 1,190,069.98 |
| FV-1-3-26 | 1,572.12 | 1,654.53 | 82.2 | 596,955.66 | 1,189,291.42 |
| FV-1-3-27 | 1,572.37 | 1,657.20 | 84.7 | 596,839.48 | 1,189,327.56 |
| FV-1-3-28 | 1,572.28 | 1,660.29 | 87.8 | 596,760.54 | 1,189,373.97 |
| FV-1-3-29 | 1,572.21 | 1,660.30 | 87.9 | 596,754.02 | 1,189,583.40 |
| FV-1-3-30 | 1,572.20 | 1,654.43 | 82.1 | 596,581.23 | 1,189,554.40 |
| FV-1-3-31 | 1,557.00 | 1,656.01 | 98.8 | 596,581.52 | 1,189,294.58 |
| FV-1-3-32 | 1,639.26 | 1,667.68 | 28.4 | 597,468.69 | 1,189,985.74 |
| FV-1-3-5 | 1,648.22 | 1,669.44 | 21.2 | 597,414.31 | 1,190,025.59 |
| FV-1-3-6 | 1,647.99 | 1,670.48 | 22.4 | 597,357.76 | 1,190,068.49 |
| FV-1-3-7 | 1,648.14 | 1,669.92 | 21.7 | 597,329.78 | 1,190,225.20 |
| FV-1-4-1 | 1,666.25 | 1,671.87 | 5.6 | 597,223.93 | 1,190,416.98 |
| FV-1-4-10 | 1,615.22 | 1,665.91 | 50.6 | 596,897.20 | 1,190,328.59 |
| FV-1-4-11 | 1,617.48 | 1,667.07 | 49.5 | 596,887.14 | 1,190,480.56 |
| FV-1-4-12 | 1,616.93 | 1,664.85 | 47.8 | 596,860.99 | 1,190,585.65 |
| FV-1-4-13 | 1,616.82 | 1,667.64 | 50.7 | 596,878.10 | 1,190,438.53 |
| FV-1-4-2 | 1,665.91 | 1,671.13 | 5.2 | 597,252.04 | 1,190,379.99 |
| FV-1-4-3 | 1,668.26 | 1,671.56 | 3.3 | 597,186.43 | 1,190,527.31 |
| FV-1-4-4 | 1,667.00 | 1,669.45 | 2.4 | 597,152.93 | 1,190,669.53 |
| FV-1-4-5 | 1,642.98 | 1,667.02 | 24 | 597,172.17 | 1,190,296.81 |
| FV-1-4-6 | 1,640.78 | 1,669.78 | 28.9 | 597,111.79 | 1,190,403.51 |
| FV-1-4-7 | 1,637.42 | 1,667.67 | 30.2 | 597,044.56 | 1,190,543.57 |
| FV-1-4-8 | 1,634.68 | 1,664.23 | 29.5 | 596,995.66 | 1,190,653.62 |
| FV-1-4-9 | 1,612.43 | 1,664.54 | 52 | 596,939.55 | 1,190,257.37 |
| FV-O1-1 | 1,675.61 | 1,679.66 | 4 | 597,806.88 | 1,190,286.15 |
| FV-O1-2 | 1,652.26 | 1,679.10 | 26.8 | 597,744.82 | 1,190,275.17 |
| FV-O2-1 | 1,672.51 | 1,679.98 | 7.4 | 597,711.12 | 1,190,457.68 |
| FV-O2-2 | 1,665.89 | 1,679.82 | 13.9 | 597,686.53 | 1,190,452.18 |
| FV-O3-1 | 1,672.70 | 1,679.88 | 7.2 | 597,593.29 | 1,190,597.01 |
| FV-O3-2 | 1,668.44 | 1,679.58 | 11.1 | 597,569.91 | 1,190,559.63 |
| FV-O4-1 | 1,673.62 | 1,674.14 | 0.5 | 597,516.30 | 1,190,401.81 |
| FV-O4-2 | 1,665.51 | 1,672.73 | 7.2 | 597,620.08 | 1,190,350.65 |
| FV-O5-1 | 1,671.99 | 1,673.61 | 1.6 | 597,484.54 | 1,190,343.34 |
| FV-O5-2 | 1,670.15 | 1,672.78 | 2.6 | 597,523.36 | 1,190,288.15 |
| FV-O5-3 | 1,669.61 | 1,671.96 | 2.3 | 597,583.48 | 1,190,257.02 |
| FV-O6-1 | 1,668.94 | 1,673.87 | 4.9 | 597,552.47 | 1,190,463.24 |
| FV-O6-2 | 1,662.67 | 1,673.19 | 10.5 | 597,613.07 | 1,190,460.57 |
| FV-O7-1 | 1,672.35 | 1,678.81 | 6.5 | 598,354.24 | 1,189,397.33 |
| FV-O8-1 | 1,670.50 | 1,679.87 | 9.3 | 598,409.09 | 1,189,472.61 |
| FV-O9-1 | 1,662.66 | 1,679.27 | 16.6 | 598,460.32 | 1,189,529.60 |
| FV-O10-1 | 1,668.25 | 1,680.43 | 12.2 | 598,504.33 | 1,189,576.75 |
| FV-O11-1 | 1,676.00 | 1,683.04 | 7 | 598,678.58 | 1,189,542.39 |
| FV-O12-1 | 1,678.84 | 1,680.03 | 1.2 | 598,154.35 | 1,189,596.36 |
| FV-O13-1 | 1,675.31 | 1,678.19 | 2.9 | 598,044.83 | 1,189,602.82 |
| FV-O14-1 | 1,672.01 | 1,678.69 | 6.7 | 597,886.65 | 1,189,680.78 |
| J-1 | 1,680.58 | 1,681.20 | 0.6 | 598,263.46 | 1,189,369.94 |
| J-2 | 1,648.47 | 1,678.82 | 30.3 | 598,103.38 | 1,189,358.40 |
| J-3 | 1,636.98 | 1,677.62 | 40.6 | 598,026.50 | 1,189,332.47 |
| J-4 | 1,622.89 | 1,675.80 | 52.8 | 597,905.62 | 1,189,313.42 |
| J-5 | 1,617.52 | 1,674.98 | 57.3 | 597,846.31 | 1,189,306.39 |
| J-6 | 1,607.25 | 1,673.81 | 66.4 | 597,706.32 | 1,189,271.71 |
| J-7 | 1,597.22 | 1,672.43 | 75.1 | 597,521.93 | 1,189,194.76 |
| J-8 | 1,633.57 | 1,676.75 | 43.1 | 597,969.06 | 1,189,322.53 |
| J-9 | 1,668.60 | 1,680.06 | 11.4 | 598,185.99 | 1,189,375.78 |
| J-10 | 1,611.75 | 1,674.33 | 62.5 | 597,766.77 | 1,189,296.42 |
| J-11 | 1,673.26 | 1,680.16 | 6.9 | 598,188.87 | 1,189,388.73 |
| J-12 | 1,648.75 | 1,677.81 | 29 | 598,081.42 | 1,189,402.60 |
| J-13 | 1,624.16 | 1,673.61 | 49.4 | 597,946.03 | 1,189,402.99 |
| J-14 | 1,606.24 | 1,671.72 | 65.3 | 597,586.30 | 1,189,333.33 |
| J-15 | 1,619.69 | 1,673.65 | 53.9 | 597,862.43 | 1,189,207.76 |
| J-16 | 1,616.64 | 1,671.36 | 54.6 | 597,745.56 | 1,189,364.57 |
| J-18 | 1,624.98 | 1,673.13 | 48.1 | 597,932.24 | 1,189,419.62 |
| J-19 | 1,606.68 | 1,672.26 | 65.5 | 597,602.91 | 1,189,299.43 |
| J-20 | 1,597.16 | 1,672.04 | 74.7 | 597,550.71 | 1,189,192.17 |
| J-21 | 1,597.06 | 1,671.55 | 74.3 | 597,586.22 | 1,189,192.69 |
| J-22 | 1,597.11 | 1,672.05 | 74.8 | 597,507.16 | 1,189,218.87 |
| J-23 | 1,596.88 | 1,671.18 | 74.2 | 597,522.93 | 1,189,279.13 |
| J-24 | 1,596.83 | 1,670.80 | 73.8 | 597,516.45 | 1,189,306.26 |
| J-25 | 1,648.73 | 1,676.28 | 27.5 | 598,083.44 | 1,189,476.76 |
| J-26 | 1,648.39 | 1,675.66 | 27.2 | 598,062.96 | 1,189,498.79 |
| J-27 | 1,648.25 | 1,674.08 | 25.8 | 597,987.64 | 1,189,488.00 |
| J-28 | 1,648.10 | 1,673.43 | 25.3 | 597,963.67 | 1,189,509.06 |
| J-30 | 1,614.11 | 1,674.18 | 59.9 | 597,570.00 | 1,189,674.00 |
| J-31 | 1,580.88 | 1,662.63 | 81.6 | 597,170.77 | 1,189,195.04 |
| J-32 | 1,628.86 | 1,676.83 | 47.9 | 597,655.78 | 1,189,785.84 |
| J-33 | 1,609.52 | 1,672.10 | 62.5 | 597,501.85 | 1,189,614.21 |
| J-34 | 1,645.18 | 1,678.19 | 32.9 | 597,708.32 | 1,189,854.44 |
| J-35 | 1,601.94 | 1,668.52 | 66.4 | 597,386.77 | 1,189,452.76 |
| J-36 | 1,596.01 | 1,665.80 | 69.6 | 597,304.82 | 1,189,345.36 |
| J-37 | 1,596.01 | 1,665.30 | 69.1 | 597,328.17 | 1,189,322.88 |
| J-38 | 1,580.48 | 1,661.79 | 81.1 | 597,224.21 | 1,189,276.61 |
| J-39 | 1,672.56 | 1,677.04 | 4.5 | 597,785.12 | 1,190,121.15 |
| J-40 | 1,609.77 | 1,672.42 | 62.5 | 597,510.70 | 1,189,601.94 |
| J-41 | 1,609.65 | 1,671.15 | 61.4 | 597,499.68 | 1,189,660.18 |
| J-42 | 1,609.43 | 1,670.61 | 61.1 | 597,515.56 | 1,189,680.82 |
| J-43 | 1,609.31 | 1,669.26 | 59.8 | 597,561.48 | 1,189,726.27 |
| J-44 | 1,595.90 | 1,664.33 | 68.3 | 597,374.86 | 1,189,295.44 |
| J-45 | 1,595.79 | 1,663.80 | 67.9 | 597,408.54 | 1,189,297.55 |
| J-46 | 1,584.93 | 1,663.68 | 78.6 | 597,225.78 | 1,189,233.85 |
| J-47 | 1,580.73 | 1,661.94 | 81 | 597,216.28 | 1,189,260.61 |
| J-48 | 1,580.64 | 1,661.60 | 80.8 | 597,207.92 | 1,189,290.49 |
| J-49 | 1,579.21 | 1,661.95 | 82.6 | 597,140.19 | 1,189,177.54 |
| J-50 | 1,595.79 | 1,664.29 | 68.4 | 597,304.94 | 1,189,422.54 |
| J-51 | 1,658.09 | 1,679.32 | 21.2 | 597,753.00 | 1,189,910.30 |
| J-52 | 1,609.45 | 1,671.18 | 61.6 | 597,566.16 | 1,189,559.25 |
| J-53 | 1,580.65 | 1,662.50 | 81.7 | 597,176.18 | 1,189,209.94 |
| J-54 | 1,580.69 | 1,661.55 | 80.7 | 597,203.58 | 1,189,293.69 |
| J-55 | 1,671.96 | 1,677.79 | 5.8 | 597,798.15 | 1,190,086.43 |
| J-56 | 1,678.30 | 1,680.37 | 2.1 | 597,796.45 | 1,189,960.82 |
| J-57 | 1,675.12 | 1,679.75 | 4.6 | 597,789.18 | 1,189,990.90 |
| J-60 | 1,670.36 | 1,672.85 | 2.5 | 597,427.28 | 1,190,373.53 |
| J-61 | 1,647.93 | 1,671.45 | 23.5 | 597,317.90 | 1,190,116.34 |
| J-62 | 1,628.33 | 1,669.69 | 41.3 | 597,251.45 | 1,189,875.50 |
| J-63 | 1,604.82 | 1,667.89 | 62.9 | 597,123.15 | 1,189,684.87 |
| J-64 | 1,572.50 | 1,663.63 | 90.9 | 596,840.82 | 1,189,491.78 |
| J-65 | 1,648.56 | 1,671.54 | 22.9 | 597,324.27 | 1,190,127.90 |
| J-66 | 1,641.41 | 1,670.91 | 29.4 | 597,296.16 | 1,190,042.54 |
| J-67 | 1,619.04 | 1,668.89 | 49.8 | 597,211.42 | 1,189,779.92 |
| J-68 | 1,590.86 | 1,666.89 | 75.9 | 597,023.39 | 1,189,609.26 |
| J-69 | 1,578.12 | 1,664.51 | 86.2 | 596,891.85 | 1,189,521.08 |
| J-70 | 1,563.61 | 1,659.27 | 95.5 | 596,681.78 | 1,189,403.29 |
| J-71 | 1,627.45 | 1,668.79 | 41.3 | 597,313.38 | 1,189,931.66 |
| J-72 | 1,627.47 | 1,667.68 | 40.1 | 597,417.58 | 1,189,898.74 |
| J-73 | 1,627.47 | 1,667.28 | 39.7 | 597,457.21 | 1,189,901.51 |
| J-74 | 1,627.32 | 1,668.90 | 41.5 | 597,189.37 | 1,189,890.46 |
| J-75 | 1,626.71 | 1,667.63 | 40.8 | 597,118.38 | 1,189,963.46 |
| J-76 | 1,604.90 | 1,665.82 | 60.8 | 597,183.58 | 1,189,632.93 |
| J-77 | 1,604.66 | 1,667.35 | 62.6 | 597,093.15 | 1,189,714.56 |
| J-78 | 1,572.86 | 1,663.73 | 90.7 | 596,847.34 | 1,189,493.90 |
| J-79 | 1,572.62 | 1,663.31 | 90.5 | 596,838.76 | 1,189,476.64 |
| J-80 | 1,572.50 | 1,662.24 | 89.6 | 596,802.84 | 1,189,440.39 |
| J-81 | 1,572.35 | 1,661.33 | 88.8 | 596,763.63 | 1,189,421.30 |
| J-82 | 1,572.31 | 1,661.11 | 88.6 | 596,756.83 | 1,189,412.91 |
| J-83 | 1,572.37 | 1,662.52 | 90 | 596,797.77 | 1,189,510.21 |
| J-84 | 1,572.52 | 1,662.00 | 89.3 | 596,778.41 | 1,189,520.02 |
| J-85 | 1,572.56 | 1,660.90 | 88.2 | 596,772.33 | 1,189,565.92 |
| J-86 | 1,571.89 | 1,659.77 | 87.7 | 596,731.54 | 1,189,581.26 |
| J-87 | 1,572.24 | 1,656.63 | 84.2 | 596,631.56 | 1,189,494.85 |
| J-88 | 1,572.34 | 1,656.21 | 83.7 | 596,615.26 | 1,189,487.87 |
| J-89 | 1,572.14 | 1,655.65 | 83.3 | 596,601.39 | 1,189,507.00 |
| J-90 | 1,567.51 | 1,661.08 | 93.4 | 596,727.33 | 1,189,470.33 |
| J-91 | 1,620.81 | 1,669.06 | 48.2 | 597,225.15 | 1,189,798.21 |
| J-92 | 1,646.08 | 1,671.29 | 25.2 | 597,307.03 | 1,190,095.43 |
| J-93 | 1,566.74 | 1,660.75 | 93.8 | 596,716.13 | 1,189,460.78 |
| J-94 | 1,648.42 | 1,671.27 | 22.8 | 597,322.48 | 1,190,109.56 |
| J-95 | 1,647.93 | 1,670.29 | 22.3 | 597,343.92 | 1,190,206.37 |
| J-96 | 1,628.26 | 1,669.44 | 41.1 | 597,275.27 | 1,189,881.91 |
| J-97 | 1,627.56 | 1,669.12 | 41.5 | 597,287.20 | 1,189,911.98 |
| J-98 | 1,627.57 | 1,668.60 | 40.9 | 597,332.04 | 1,189,928.83 |
| J-99 | 1,572.19 | 1,660.03 | 87.7 | 596,761.56 | 1,189,361.72 |
| J-100 | 1,572.76 | 1,658.59 | 85.6 | 596,774.72 | 1,189,313.31 |
| J-101 | 1,572.31 | 1,659.40 | 86.9 | 596,753.21 | 1,189,332.53 |
| J-102 | 1,572.34 | 1,659.05 | 86.5 | 596,752.73 | 1,189,315.68 |
| J-103 | 1,572.20 | 1,656.56 | 84.2 | 596,869.13 | 1,189,335.27 |
| J-104 | 1,572.05 | 1,655.20 | 83 | 596,925.58 | 1,189,302.30 |
| J-105 | 1,590.66 | 1,664.63 | 73.8 | 596,948.61 | 1,189,729.62 |
| J-106 | 1,590.33 | 1,663.83 | 73.4 | 596,900.33 | 1,189,750.35 |
| J-107 | 1,640.97 | 1,668.65 | 27.6 | 597,408.74 | 1,189,971.65 |
| J-108 | 1,640.38 | 1,668.54 | 28.1 | 597,414.54 | 1,189,966.96 |
| J-109 | 1,640.02 | 1,668.34 | 28.3 | 597,428.22 | 1,189,966.64 |
| J-110 | 1,667.13 | 1,672.41 | 5.3 | 597,197.73 | 1,190,459.57 |
| J-111 | 1,641.97 | 1,670.81 | 28.8 | 597,089.11 | 1,190,443.47 |
| J-112 | 1,666.33 | 1,670.45 | 4.1 | 597,175.76 | 1,190,603.94 |
| J-113 | 1,617.92 | 1,667.75 | 49.7 | 596,884.10 | 1,190,439.55 |
| J-114 | 1,619.34 | 1,666.21 | 46.8 | 596,914.95 | 1,190,525.30 |
| J-115 | 1,616.77 | 1,665.65 | 48.8 | 596,899.42 | 1,190,555.86 |
| J-116 | 1,668.98 | 1,672.25 | 3.3 | 597,216.89 | 1,190,439.50 |
| J-117 | 1,670.97 | 1,672.32 | 1.4 | 597,198.55 | 1,190,477.43 |
| J-118 | 1,670.77 | 1,671.86 | 1.1 | 597,185.44 | 1,190,506.51 |
| J-120 | 1,670.35 | 1,673.10 | 2.7 | 597,504.95 | 1,190,306.60 |

Table 0-3: Outlets from the main Canal

| Label | Elevation (m) | Flow (Out net) (L/s) | Hydraulic Grade (m) | X (m) | Y (m) |
| --- | --- | --- | --- | --- | --- |
| DVB 1 | 1,681.59 | 28.00 | 1,681.59 | 598,283.94 | 1,189,369.16 |
| DB2 | 1,680.53 | 41.00 | 1,680.53 | 597,801.15 | 1,189,966.34 |
| DB3 | 1,672.97 | 85.00 | 1,672.97 | 597,435.90 | 1,190,393.84 |
| DB4 | 1,672.62 | 32.00 | 1,672.62 | 597,211.89 | 1,190,462.12 |
| O5 | 1,674.02 | 2.00 | 1,674.02 | 597,472.23 | 1,190,373.97 |
| O4 | 1,674.36 | 2.00 | 1,674.36 | 597,499.89 | 1,190,409.42 |
| O3 | 1,680.10 | 1.00 | 1,680.10 | 597,610.63 | 1,190,622.70 |
| O2 | 1,680.17 | 2.00 | 1,680.17 | 597,741.70 | 1,190,464.80 |
| O1 | 1,680.16 | 3.00 | 1,680.16 | 597,862.28 | 1,190,296.21 |
| O6 | 1,674.39 | 2 | 1,674.39 | 597,506.47 | 1,190,464.95 |
| O7 | 1,680.94 | 1 | 1,680.94 | 598,332.36 | 1,189,421.03 |
| O8 | 1,681.46 | 1 | 1,681.46 | 598,390.73 | 1,189,488.39 |
| O9 | 1,680.97 | 1 | 1,680.97 | 598,438.42 | 1,189,543.29 |
| O10 | 1,683.22 | 1 | 1,683.22 | 598,527.45 | 1,189,572.86 |
| O11 | 1,685.08 | 1 | 1,685.08 | 598,674.56 | 1,189,559.08 |
| O14 | 1,680.77 | 1 | 1,680.77 | 597896.79 | 1,189,695.00 |
| O13 | 1,680.62 | 1 | 1,680.62 | 598051.3 | 1,189,622.22 |
| O12 | 1,680.66 | 1 | 1,680.66 | 598163.86 | 1,189,609.29 |

# REFERENCE

- FAO (1977) guidelines for predicting crop water requirements. No 24, Rome Italy

- Design of small Canal structures , USBR

- Soft copies of hydraulic structure publishing

- IDD manual

- ESRDF manual

- Ministry of water resources