THE NATIONAL REGIONAL STATE OF OROMIA

# OROMIA IRRIGATION DEVELOPMENT AUTHORITY

AGRICULTURAL GROWTH PROGRAM (AGP)

Consultancy Services for Nineteen (19) Small Scale Irrigation Projects Study and Detail Design

Volume VIII: Engineering Design Report of Kerfe Small Scale Irrigation Project

December 2016

Finfinne

AYJEF Water Woks and                 P.O.Box: 43329       Tele: +251 911 304019

Business Service PLC                                           E-mail: ayconsult1@gmail.com

# table of content

TABLE OF CONTENT………………….                                                                                   	2

LIST OF TABLES	3

EXECUTIVE SUMMARY	5

1.	IRRIGATION AND DRAINAGE SYSTEM DESIGN	6

1.1. General	6

1.2. Location and Accessibility of the project areas	6

1.3. Methodology of project study and design	7

1.4. Planned irrigable area	7

1.5. Water Availability	7

1.6. Water distribution scenario	8

1.7. System Layout Plan	8

1.8. Nomenclature and Symbols	9

1.9. Standards	10

1.10. Major Components of the project	10

2.	HEAD WORK	11

2.1. Objective	11

2.2. Selection of Headwork site	11

2.3. Geology of weir site and the canal route	11

2.3.1. Base Flow & Design Flood Estimation	12

2.3.2. Selection of Weir Type	12

2.4. Design of weir	13

2.5. STABILITY ANALYSIS OF WEIR:	15

2.6. Silt Gate	40

2.7. Canal water escape	41

3.	DESIGN OF SUPPLY CANALS	42

3.1. Main Canal	42

3.2. Design of drainage canals	43

3.3. Design of Hydraulic Structures	43

4.	BILL OF QUANTITY	47

5.	SUMMARY COST OF THE PROJECT	54

6.	PROJECT DURATION AND WORK PLAN	56

7.	CONCLUSION AND RECOMMENDATIONS	57

# LIST OF TABLES

Table 1. Naming used in system layout plan	9

Table 2Stability condition of the weir	Error! Bookmark not defined.

Table 3Dimension of the weir	Error! Bookmark not defined.

Table 4stability analysis against overturning	Error! Bookmark not defined.

Table 5stability analysis against sliding	Error! Bookmark not defined.

Table 6. Bill of Quantity, specification and cost estimates	47

Table 7. Project Duration and Work Plan	56

LIST OF FIGURES

Figure 1. Project area description of Kerfe small scale irrigation project	7

Figure 2. Geological X-section of Mojo River along the Diversion Headwork Axis	12

Figure 3. Flow condition over the weir its detail part	15

Figure 4. Showing Unbalanced Head in Jump Trough at High Flood Flow & Static Head at Pond Level Flow	Error! Bookmark not defined.

Figure 5. Basic section of drainage canal	43

# Executive Summary

Kerfe small scale irrigation project is located in Kerefe kebele; Ada’a Wereda of East showa zone. The area is inscribed between kerefe; Hidi Deko; Ardega Kordida; Edado Ejire; Goro Tigir; and Tulu Dimitu kebeles.  The proposed command area is located at geographical location of 8050.270 N and 39008.948E at a distance of 14km South East of Godino town and about 20km in North east of Bushoftu town. The area is under subtropical zone at an average altitude of 2042m a.s.l, with unreliable rainfall distribution. Irrigation is a key element to boost agricultural production and productivity and to diversify agricultural activities over the area. Looking to interest, efforts, trials and requests of the community, OIDA-East Shoa Zone office, district line office, administrative bodies and other stakeholders at district level have given due attention to the site to be studied and come up with this proposal.

The beneficiaries of the project are residents of Kerefe kebele. From this proposed project, at least 76 households benefit from irrigation activity so that equitable use of land and water resources could be established at the site under consideration as currently there exist water shortage to increase production and productivity on sustainable basis. The target communities are experienced in irrigation development activities from the existing traditional scheme owned by them.

The project is gravity system with main canal route of 2242m length, Secondary canal of 497.4m and 8 Tertiary canals of 2082m and 5 outlets (Turnouts). The soil of the area is dominantly clayey in texture.

The components of this project scheme include the weir, supply and drainage canal network and other related irrigation infrastructures. The total project upgrading cost is 6,127,657.40 Birr. The project is designed for 20 years life.

# IRRIGATION AND DRAINAGE SYSTEM DESIGN

## General

Irrigation and drainage system design incorporates all engineering aspects related to project study and designs. This system design focuses on study and design of headwork, supply and drainage canals and all appurtenant structures related to diversion, conveyance, distribution and application of water to the farm lands. This design also gives attention to network of roads (access to main road, farm roads and inspection roads). All influential factors that determine project sustainability are accounted to come up with reasonable results of study. In so doing, idea from different disciplines was given due attention. The project is gravity diversion system on Mojo River. One-way (Right side) intake is suggested based on available suitable irrigable farm land with available discharge. Reasonable systematic layouts were adopted to minimize costs and to enable easier application of water and easy management of the project. A total of 45 ha of land were surveyed of which 39 ha net irrigable area planned for development based on topographic condition.

## Location and Accessibility of the project areas

Kerfe small scale irrigation project is located in Kerefe kebele; Ada’a Wereda of East showa zone. The area is inscribed between kerefe; Hidi Deko; Ardega Kordida; Edado Ejire; Goro Tigir; and Tulu Dimitu kebeles.  The proposed command area is located at geographical location of 8050.270 N and 39008.948E at a distance of 14km South East of Godino town and about 20km in North east of Bushoftu town. It could be accessible from Bishoftu town to the project area through passing Gabaya Robi (Hidi town) all weathered road.

East Shewa

Werdas

Oromia

Zones

Ada’a  Werda

Figure 1. Project area description of Kerfe small scale irrigation project

## Methodology of project study and design

The methodologies adopted are:

- Existing localities and community experiences and information.

- Using hand held GPS and1:50,000 contour maps in initial geo-referencing and in selection of appropriate headwork site.

- detail site survey and investigations

- Secondary data from concerned areas.

- Hydro-meteorological data, crop data, soil data etc. collecting and organizing for this study.

- Using guidelines manuals and other reference materials.

- Use of different software like;

- New Locclim 1.0,

- cropwat 8.0,

- Arc GIS 9.3

- Global Maper 13

- Google Earth

## Planned irrigable area

The irrigable area has moderate surface slope ranging from 2-5% towards the Mojo River. The soil of the area vertisol with deep clay texture. The soil is expansive type and easily susceptible to runoff so that the area is dissected by number of gullies. Proper soil and water conservation practices shall be adopted by the community to avoid any minimize the effect of runoff due to the introduced scheme. The proposed crops for this project are Maize, Onion, Potato, Pepper, Cabbage and Tomato. These crops under irrigation have maximum dry period duty of 1.04 l/s per ha  for 12 hr irrigation per day and the net irrigable area of 37.19 ha was planned for 38.68 l/s design discharge.

## Water Availability

The source of water is Mojo River. It could have been good if monthly flow data were available; but the river is not gauged. Hence Lean flow was estimated after taking dry period measurements using floating technique. The lean flow as reported under hydrology section is 50 l/s. With 10% downstream release 40l/s flow was planned for this development. There is intense rainfall over the area and the catchment is mild to steep ranges with clay soil cover resulting in generation of high runoff, thus the peak design flood obtained is 155m3/s.

## Water distribution scenario

Water application to the field is based on irrigation interval set for each crop. In so doing continuous flow in main and secondary canals and rotated flow in tertiary and field canals will be adopted. The soil of the area is dense clay and its water holding property is good. The interval of water application is generally expected to be longer till next irrigation is applied when the readily available soil moisture is depleted. At each field unit level; the unit discharge is applied based on stream size required for that unit of field within irrigation time set per day to attain the required irrigation depth (FC of the soil).

## System Layout Plan

The system layout plan was prepared based on irrigation and drainage design criteria for small scale community managed schemes. This layout is prepared on 1:1000 maps with 0.5m contour interval. The field units are around 1.20ha on average bases in size and tertiary units are kept below 10ha for the scheme to be manageable with an average 5ha the lower value for the tertiary unit is due to the topographic nature of the area.   Continuous flow in main and secondary canals and rotated flow in tertiary canals is adopted regarding water distribution. The irrigation application method is furrow which is locally practiced by the farmers. Thus the furrow direction is set along the contour lines. Furrow length is selected based on soil, crop type and topography of the area, but as per local farmers practice 45-70 m is adopted in stretching the layout plan. With average furrow length of 50m and field canal length 200m a field unit of 10000m2 or 1 ha is generally adopted where the topography is convenient.

## Nomenclature and Symbols

The following designations were used in naming various irrigation infrastructures in the project area.

Table 1. Naming used in system layout plan

| Designation | Representation |
| --- | --- |
| MC | Main canal |
| SC | Secondary canal |
| TC | Tertiary canal |
| FC | Field canal |
| TC-1-1 | First tertiary canal on SC-1 |
| OT | Field off take (tertiary canal to field canal) |
| TO | Turn out (secondary/main canal to tertiary canal) |
| C-1 | Culvert -1 |
| TD | Tertiary drain |
| FD | Field drain |
| FU | Field unit |
| TU | Tertiary unit |
|  | Angle made by the wall with the vertical |
|  | Angle of surcharge |
|  | Angle of repose |
|  | Angle made by line of action of pressure with the normal to the batter |
| h | Height of backfill on bank side |
| Pa | Active earth pressure |
| Ka | Co-efficient of active earth pressure |
| Pah | Horizontal component of active earth pressure |
| Pav | Vertical component of active earth pressure |
| Pp | Passive earth pressure |
| Kp | Co-efficient of passive earth pressure |
| Pph | Horizontal component of passive earth pressure |
| Ppv | Vertical component of passive earth pressure |
| DL | Dead load |
| WL | Water load |
| Z | Lever arm of resultant |
| M | Net stabilizing moment |
| V | Net vertical force |
| Po | Net vertical force per meter width |
| e | Eccentricity |
| b | Bottom width of abutment/ wing wall |

## Standards

The following standards were adopted in preparation of the project document. Weir site topographic map was prepared on 1:500 scale and field system layout is prepared on 1:1000 scale map indicating all features. Longitudinal profiles are prepared in scale of 1:100 in vertical and 1:1000 in horizontal. Working drawings are prepared to scales of 1:10, 1:50 or 1:100 based on appropriate visibility scale and all drawings are prepared on standard sheets of A0(1080x810), A1(810x540), A2(540x410), A3(410x297) and A4(297x210) in millimeter units.

## Major Components of the project

- Access to main road 3km from  (URAPE Road from Bishoftu to Gimbichu town)

- Diversion Head work Broad crested weir and temporary channel and diversion for construction by part.

- Lined Main Canal  2.02km long after intake structure

- Canal water escapes (1 in No).

- Crossing drain (5 in No)

- Turnout  5 in number

- Drop Structure (7 in No) on Secondary canal.

- Drop Structure (7 in No) on Tertiary canal.

- Drop Structure (2 in No) on Tertiary Drain.

- One Secondary Canal with total length of 0.497km

- Total of  8 Tertiary Canals with total length of 2.082km

- Tertiary Drain 3 in number with a total length of  0.57km

# HEAD WORK

## Objective

The main objective of the head work is to provide sufficient irrigation water at upstream side of the weir during the period of low flow. And the work, which are constructed at the head of the canal in order to divert the river water towards the canal, so as to ensure a regulated continuous supply of silt-free water with a certain minimum head in to the canal.

## Selection of Headwork site

The headwork site was selected under the following factors:

- Easy access to the site during construction and operation

- Possibility to command the anticipated area with reasonably less weir height.

- Convenience for intake and head regulators

- Better geological formation at the bed and side banks.

- Possibility to protect flooding of off taking canal

- Relatively area convenient for construction

- Area with less flood heights and effects.

- Area with less effect of transported materials specially boulders

- Area with easier temporary diversion during construction.

- Relative cost of weir.

Design

The main Component of the project design is head work design and Command area system Arrangement. The Possible Diversion Weir location has been chosen. The exact position has been fixed after carefully consideration of each of the following factors

- Characteristics of Foundation

- Topography of the earth and rock surfaces at the site and its effect on the dimension of the weir,

- Quantity of Material to be excavated and other factors.

- Requirements as to coffers,

- necessary for unwavering of the site

- Availability and character of materials for Construction

## Geology of weir site and the canal route

The diversion headwork site is selected in a narrow, relatively straight stream channel that is oriented in NE-SW direction. Both banks are stable and both upstream and downstream parts of the river course at the headwork sites are straight reach. The river channel is mainly weathered basalt at top and hard basalt at bed.

The cross section along the weir axis depicts the succession composed (from top to bottom) of: moderately weathered and fractured rock of ignimbritic origin followed by hard rock to depth. The headwork site is in general safe for the diversion headwork construction provided that top weathered fractured ignimbritic rock is removed to the strong and fresh layer of basalt. The weathered layer is unwedded extending to both sides of the river and can easily be chiseled at top surface layer that tends to be monolithic to depth.

Figure 2. Geological X-section of Mojo River along the Diversion Headwork Axis

### Base Flow & Design Flood Estimation

Accordingly, to the Hydrology study the base flow discharge estimated in the river course of Mojo River during dry season has been obtained as 40 l/sec excluding the probable subsurface flow through the gravel river course formation, and  the design discharge for the structure to be constructed  as diversion shall be 155.00 m3/sec with  50 return year by taking high rainfall, degraded land, shallow depth soil and other topographic features of the area in to consideration

### Selection of Weir Type

Broad Crested Weir will be provided so as to have minimum possible head loss. The minimum top width as per IS 6531-1994 is 2/3 He. Hence crest width of the weir was determined to be 1.7m length which will be provided with Riprap protection work for both upper and lower embankment.

## Design of weir

Length or span of the weir overflow was taken 22.5m from the existing situation of the river section. This span results in relatively less flood height and hence the anticipated weir would be economical.

Discharge and Head over the Weir

Q = CLHe3/2,   H= (Q/CL)2/3

Where: C=1.70

He=2.54m

q=Q/L=6.89m3/s/m

Modular Flow formula Q = CLHe3/2 is used

Where	C=1.7

Design Discharge to pass over Weir (50 percent of total Discharge) =139.50m3/s

Calculated Discharge to pass over Weir (Q) = CLHe3/2=153.79OK

Discharge Through Under Sluice (Q1)

Where:-

Upstream head, He = 2041.7 - 2038.05 = 3.65m, and

Downstream head, D = 2040.1m - 2038.05 = 2.05m.

Difference of u/s and d/s water level   H= 1.60m

D/He = 2.05/3.65 = 0.56 (<0.75)

Hence; Modular Flow formula Q = CLHe3/2 is used

Design Discharge to pass over Weir =3.20cumecs

Total Discharge Pass Through Weir(Q1+Q2) =156.99cumecs > 155 ,OK

Stilling Basin

Discharge in Weir (Q)=153.786m3/s

Total clear Waterway (L)= 21.50m

Discharge per metre length (q)= 7.15m3/s/m

Head loss (HL)= 1.60m

From Blench Curve,  Critical Depth, Dc = (q2/g)1/3=1.73m

HL/Dc	=0.92

Pre-jump depth corresponding to Ef1;	D1=0.77m

Post jump depth corresponding to Ef2	;D2=3.31m

Upstream specific energy level	 Ef1=5.15m

Downstream specific energy level Ef2	=3.55m

Froude no., F = q / √ g x D13=3.37

∆ cistern level  = D/S TEL - Ef2	=2038.05m

＊ Cistern Length required (Fr<4.5) = 5 x ( D2 - D1 )	=12.69	m

Say L =13.00m

Top Width of Crest

Sharp Crested Weir will be provided so as to have minimum possible head loss. The minimum top width as per IS 6531-1994 is 2/3 He

Where;	 He = U/S FSL - Crest Level=2.65m

∆ min. top width of crest =1.9	m

Say =2m

Vertical Cut-Offs

(i) Provide U/S Cut-off of  2 m depth below U/S bed level=2.00m

Bottom elevation of upstream pile=2036.05m

(ii) Provide D/S cut-off of  1.9 m depth for safe exit gradient=1.90m

Bottom elevation of downstream pile =2036.15m

Floor Length

From Khosla's theory, the exit gradient can be expressed by the relation as:

Exit gradient = 1 / p x sqrt ( l ) X (H / d )

Where;

GE (Exit gradient) = 1 / 5								                              d = Depth of d/s cutoff	=1.90m

H = Maximum static head = U/S FSL - D/S cistern level =1.60	m

Substituting the values	1 / 5 = 1 / p x sqrt ( l ) X (H / d )								                          or

1 / p x sqrt ( l ) = d / (5 x H)=0.216

From Khosla curve for 1 / p x sqrt ( l )	a= 3.19	; 4.34

Floor length = a x d =  	6.06m

The Total length will be provided as below :-

- D/S Floor length=0m

- D/S Glacis length ( 1:2 slope)	=2.00m

- Length for Gates & Crest=2.00m

- U/S Glacis length ( 1:1 slope)=	1.00m

- U/S floor length=0.00m

- Total length of floor=5.00m

- D/S floor thickness = 0.38m

- U/S floor thickness= 0.70m detail of weir cross-section is shown in the figure below

Figure 3. Flow condition over the weir its detail part

## STABILITY ANALYSIS OF WEIR:

Calculation of the force acting on the weir

| (1) | Horizintal forces |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ① | : Water pressure |  |  | Ph=γw x(Hcw - Hu)2 x1 x 0.5 |  |  | = | 5.00 | KN |  |  |  |  |
|  |  |  | γw= | 10 | KN/m3 |  |  |  |  |  |  |  |  |
|  | : Point acting force |  |  | Lh1=1/3 x (Hcw - Hu)= |  | above d/s bed level |  | 0.33 | m |  |  |  |  |
| ② | : Silt pressure |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Silt gets depositted against the upstream face of weir. |  |  |  |  |  |  |  |  |  |  |
|  | If h is the height of silt deposited, then the force exerted by this silt in addition to external water pressure, |  |  |  |  |  |  |  |  |  |  |  |  |
|  | can be represented by Rankine's formula as : |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Ps = 1/2 x γsub x h2 x Ka |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | where :Ka is the coefficient of earth pressure of silt = (1 - sinΦ) /( 1 + sinΦ). |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Φ | is the angle of internal friction of soil. |  |  |  |  |  |  |  |  |
|  |  |  |  | :γsub is the submerged unit weight of silt material. |  |  |  |  |  |  |  |  |  |
|  |  |  |  | : h is height of silt deposited. |  |  |  |  |  |  |  |  |  |
|  | In the absence of any reliable data for the type of silt that is going to be deposited, U.S.B.R. Recommendations |  |  |  |  |  |  |  |  |  |  |  |  |
|  | may be adopted. In these recommendations, deposited silt may be taken as equivalent to |  |  |  |  |  |  |  |  |  |  |  |  |
|  | a fluid exerting a force with a unit wt. equal to 3.6KN/m3 in the horizontal direction and a vertical force with a unit wt. of 9.2KN /m3. |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Hence,In this project case it is reasonable to accept the U.S.B.R. recommendation. |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | PSh=1/2 x 3.6 x( 1 x (Hcw - Hu))2 x 1= |  |  |  |  | 1.800 | KN |  |  |  |  |
|  | : Point acting force |  |  | Lh2=(1x (Hcw - Hd)) x 1/3= |  |  |  | 0.13 | m |  |  |  |  |
| ( 2 ) |  | Vertical forces |  |  |  |  |  |  |  |  |  |  |  |
| ① | : Water pressure |  |  | Pv=γw x(Hcw - Hu)2x 1 x0.5 = |  |  |  | 0.000 | KN |  |  |  |  |
|  | : Point acting force |  |  |  | LV1=1/3 x (Hcw - Hu)= |  |  | 0.33 | m |  |  |  |  |
| ② | : Silt pressure |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Psv=1/2 x 9.2 x (1 x (Hcw - Hu) )2 x 1= |  |  |  |  | 4.6 | KN |  |  |  |  |
|  | : Point acting force |  |  | LV2=(2/5 x (Hcw - Hu)) x 1/3 = |  |  |  | 0.13 | m |  |  |  |  |
| ③ | : Weight of weir |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | To calculate this one,body of weir should be divided as part of five. |  |  |  |  |  |  |  |  |  |  |
| △ | Area and centre of parts |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | ① | Area | ((Dup+(Dup+0.5 / m1))/2) x 0.5 = |  |  | 2.2 | ㎡ |  |  |  |  |
|  |  |  |  | C.G |  |  |  | 2.20 | m |  |  |  |  |
|  |  |  | ② | Area |  |  |  | 0.50 | ㎡ |  |  |  |  |
|  |  |  |  | C.G |  |  |  | 3.4666667 | m |  |  |  |  |
|  |  |  | ③ | Area |  |  |  | 2 | ㎡ |  |  |  |  |
|  |  |  |  | C.G |  |  |  | 2.400 | m |  |  |  |  |
|  |  |  | ④ | Area |  |  |  | 0.7 | ㎡ |  |  |  |  |
|  |  |  |  | C.G |  |  |  | 0.9333333 | m |  |  |  |  |
|  |  |  | ⑤ | Area |  |  |  |  | ㎡ |  |  |  |  |
|  |  |  |  | C.G |  |  |  |  | m |  |  |  |  |
|  |  |  | ⑥ | Area |  |  |  |  | ㎡ |  |  |  |  |
|  |  |  |  | C.G |  |  |  |  |  |  |  |  |  |
|  |  |  | ⑦ | Area |  |  |  |  | ㎡ |  |  |  |  |
|  |  |  |  | C.G |  |  |  |  |  |  |  |  |  |
|  |  |  | ⑧ | Area |  |  |  |  |  |  |  |  |  |
|  |  |  |  | C.G |  |  |  |  |  |  |  |  |  |
| △ |  |  | Total area |  |  |  | Aw | 5.4 | ㎡ |  |  |  |  |
|  |  |  |  | Centre of gravity for weir section from toe |  |  | Cwv | 2.23 | m |  |  |
| △ |  |  | Weight per m |  | W = Wc x 1 x Aw = |  |  | 129.6 | kn |  |  |
| ④ | :Uplift pressure |  |  |  |  |  |  |  |  |  |  |
|  |  |  | U1= E3 x 1 x b |  |  |  |  | 0.00 | KN |  |  |
|  | : Point acting force |  |  |  |  |  |  | 1.70 | m |  |  |
|  |  |  | U2=1/2x(E1- E3)x 1xb |  |  |  |  | 1.25 | KN |  |  |
|  | : Point acting force |  |  |  |  |  |  | 1.13 | m |  |  |  |  |
| 2) | Structural stability of weir |  |  |  |  |  |  |  |  |  |  |  |  |
| (1) |  |  | Over-Turning about the toe. |  |  |  |  |  |  |  |  |  |  |
|  | Load & Moment about Toe |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Name of Load |  |  | V (kn) | H (kn) | Arm (L) | M (KN.m) |  |  |  |  |  |  |
|  | Ph |  |  |  | -5 | 0.33 | -1.65 |  |  |  |  |  |  |
|  | Psh |  |  |  | -1.8 | 0.13 | -0.234 |  |  |  |  |  |  |
|  | Pv |  |  | 0.000 |  | 0.33 | 0 |  |  |  |  |  |  |
|  | Psv |  |  | 4.6 |  | 0.13 | 0.598 |  |  |  |  |  |  |
|  | Pw |  |  | 129.6 |  | 2.23 | 289.008 |  |  |  |  |  |  |
|  | U1 |  |  | 0.00 |  | 1.70 | 0 |  |  |  |  |  |  |
|  | U2 |  |  | -1.25 |  | 1.13 | -1.4125 |  |  |  |  |  |  |
|  | Total |  |  | 132.950 | -6.8 |  | 286.31 |  |  |  |  |  |  |
|  | Ft = |  | Ms / Mt | supporting | Ms= | 289.606 |  |  |  |  |  |  |  |
|  |  |  |  | overtuening | Mt = | 3.2965 |  |  |  |  |  |  |  |
|  |  |  | Ft =87.85>Fs=2 |  |  | 87.85 |  | SAFETY |  |  |  |  |  |
| (2) |  |  | Sliding |  |  |  |  |  |  |  |  |  |  |
|  | The friction developed between two suface is equal to μΣv, |  |  |  |  |  |  |  |  |  |  |  |  |
|  | where Σv is the algebraic sum of all the vertical forces whether upward or downward, |  |  |  |  |  |  |  |  |  |  |  |  |
|  | and μ is the coefficient of friction between the two surfaces. In order that no sliding takes place, |  |  |  |  |  |  |  |  |  |  |  |  |
|  | the external horizontal forces (ΣH) must be less than the shear resistance μ x ΣV. |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | From above Table |  |  |  |  | μ ∑ V ∑ H |  |  |  |  |  |
|  | Hence, |  | ΣH<μΣv. |  | F.S.S(factor of safety against sliding)= |  |  |  |  |  |  |  |  |
|  |  |  | The value of μ generally varies from 0.65 to 0.75 . |  |  |  | say , μ= | 0.7 |  |  |  |  |  |
|  |  |  | Fs.s=2.0 | 14 |  | F.s.s.=2.0<14 |  | SAFETY |  |  |  |  |  |
| (3) | Stress on the foundation |  |  |  |  |  |  |  |  |  |  |  |  |
| ① | Distance from toe to Point acting Moment |  |  |  |  |  |  |  |  |  |  |  |  |
|  | E=Σ286.31/Σ 132.95 |  |  |  | = | 2.15 | m |  |  |  |  |  |  |
| ② | Distance of eccentricity |  |  |  |  |  |  |  |  |  |  |  |  |
|  | E0 = E - B/2 =2.15- 3.4/2 |  |  |  | = | 0.45 | m |  |  |  |  |  |  |
|  |  |  | B=Total Length of Weir |  | = | 3.40 | m |  |  |  |  |  |  |
| ③ | Maximum Stress on the foundation (At toe) |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Smax =ΣV/B(1 + 6xE0/B) |  |  |  | = | 70.2 | KN/m2 |  |  |  |  |  |  |
| ④ | Minimum Stress of Foundation (at heel) |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Smin =ΣV/B(1 - 6xE0/B) |  |  |  | = | 8.1 | KN/m2 |  |  |  |  |  |  |
| ⑤ | Cefficienc of Unbalanced Sttlement |  |  |  |  |  |  |  |  |  |  |  |  |
|  | fu = Smax / Smin |  |  |  | = | 8.67 |  |  |  |  |  |  |  |
| ⑥ | fu=8.67 < f= 〔 3 〕 |  |  |  | = | CHECK |  |  |  |  |  |  |  |
|  | For |  | Very Soft | compressive strength |  | 250 | kn/m2 ( See table ) |  |  |  |  |  |  |
|  | Thus , |  | Smax = 70.2< 250 |  |  | SAFETY |  |  |  |  |  |  |  |

Maximum Flood Water Level

Calculation of the force acting on the weir

| (1 ) | Horizintal forces |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ① | : Water pressure |  | Ph=γw x(h2-(h- h1)2) x1 x 0.5 |  |  | = | 31.5 | KN |  |  |
|  |  | γw= | 10 | KN/m3 |  |  |  |  |  |  |
|  | : Point acting force |  | Lh1=1/3 x h1(((h+2(h-h1))/((h +(h-h1))) |  |  |  | 0.47 | m |  |  |
| ② | : Water pressure |  | Ph2=γw x h22 x 1 x 0.5 |  |  | = | 2.98 | KN |  |  |
|  | : Point acting force |  | Lh1=1/3 x h2= |  |  |  | 0.26 | m |  |  |
| ③ | : Silt pressure |  |  |  |  |  |  |  |  |  |
|  |  | Silt gets depositted against the upstream face of weir. |  |  |  |  |  |  |  |  |
|  |  | Psh=1/2 x 3.6 x( 1 x (Hcw - Hu))2 x 1= |  |  |  |  | 1.80 | KN |  |  |
|  | : Point acting force |  | Lh2=(2/5x (Hcw - Hu)) x 1/3= |  |  |  | 0.13 | m |  |  |
| ④ | : Hydrodynamic pressure Pm |  |  |  |  |  |  |  |  |  |
|  |  | Pm=K x γw x Q /g x v |  |  |  |  |  |  |  |  |
|  |  | k is coefficienc as |  | 0.035 |  |  |  |  |  |  |
|  |  | Q is discharge |  | 155.00 | m3/s |  |  |  |  |  |
|  |  | v is velocity when water is flowing over weir |  |  |  |  |  |  |  |  |
|  |  | v=q/Dc | = | 4.132948 | m/s |  |  |  |  |  |
|  |  | Thus | Pm= |  |  |  | 22.86 | KN |  |  |
|  | : Point acting force |  | Lm=(h - h1)/3 +h1 |  |  |  | 1.88 | m |  |  |
| ( 2 ) | Vertical forces |  |  |  |  |  |  |  |  |  |
| ① | : Water pressure |  | Pv1=γw x((h+(h-h1))/2)xh1xm1x Lw = |  |  |  | 31.5 | KN |  |  |
|  | : Point acting force |  | Lv1=(2.5/3) x ((h+ 2x(h-h1)/(h+(h-h1)) |  |  |  | 3.57 | m |  |  |
| ② | : Water pressure |  | Pv=γw x(h-h1)x0.5x 1 = |  |  |  | 13.25 | KN |  |  |
|  | : Point acting force |  | Lv=3.8 -h1xm1-1.1 |  |  |  | 3.25 | m |  |  |
| ③ | : Water pressure |  | Pv2=γw xh22m2x0.5x 1 = |  |  |  | 2.98 | KN |  |  |
|  | : Point acting force |  | Lv2=h2xm2/3 |  |  |  | 0.26 | m |  |  |
| ④ | : Silt pressure |  |  |  |  |  |  |  |  |  |
|  |  | Psv=1/2 x 9.2 x (1 x (Hcw - Hu) )2 x 1= |  |  |  |  | 4.6 | KN |  |  |
|  | : Point acting force |  | LV2=3.8-(2/5 x m1 x (Hcw - Hu)) x 1/3 = |  |  |  | 0.33 | m |  |  |
| ⑤ | : Weight of weir |  |  |  |  |  |  |  |  |  |
|  |  | To calculate this one,body of weir should be divided as part of five. |  |  |  |  |  |  |  |  |
| △ | Area and centre of parts |  |  |  |  |  |  |  |  |  |
|  |  | ① | Area | ((Dup+(Dup+0.5 / m1))/2) x 0.5 = |  |  | 2.2 | ㎡ |  |  |
|  |  |  | C.G | (0.5/3) x ((Dup+2x(Dup+0.5/m1))/(Dup+(Dup+0.5/m1)) |  |  | 2.2 | m |  |  |
|  |  | ② | Area |  |  |  | 0.5 | ㎡ |  |  |
|  |  |  | C.G |  |  |  | 3.4666667 | m |  |  |
|  |  | ③ | Area |  |  |  | 2 | ㎡ |  |  |
|  |  |  | C.G |  |  |  | 2.400 | m |  |  |
|  |  | ④ | Area |  |  |  | 0.7 | ㎡ |  |  |
|  |  |  | C.G |  |  |  | 0.9333333 | m |  |  |
|  |  | ⑤ | Area |  |  |  | 0 | ㎡ |  |  |
|  |  |  | C.G |  |  |  | 0 | m |  |  |
|  |  | ⑥ | Area |  |  |  | 0 |  |  |  |
|  |  |  | C.G |  |  |  | 0 |  |  |  |
|  |  | ⑦ | Area |  |  |  | 0 |  |  |  |
|  |  |  | C.G |  |  |  | 0 |  |  |  |
|  |  | ⑧ | Area |  |  |  | 0 |  |  |  |
|  |  |  | C.G |  |  |  | 0 |  |  |  |
| △ |  | Total area |  |  |  | Aw | 5.4 | ㎡ |  |  |
|  |  | Centre of gravity for weir section from toe |  |  |  | Cwv | 2.23 | m |  |  |
| △ |  | Weight |  | W = Wc x 1x Aw = |  |  | 129.6 | kn |  |  |
| ④ | :Uplift pressure |  |  |  |  |  |  |  |  |  |
|  |  | U1= E3 x 1x (m1x(Hcw- Hu)+0.5 m2 x(Hcw-Hd) |  |  |  |  | 2.34 | KN |  |  |
|  | : Point acting force |  |  |  |  |  | 1.70 | m |  |  |
|  |  | U2=1/2x(E1- E3)x 1x(m1x(Hcw- Hu)+0.5 m2 x(Hcw-Hd) |  |  |  |  | 2 | KN |  |  |
|  | : Point acting force |  |  |  |  |  | 1.13 | m |  |  |
| 2) | Structural stability of weir |  |  |  |  |  |  |  |  |  |
| (1) |  | Over-Turning about the toe. |  |  |  |  |  |  |  |  |
|  | Load & Moment about Toe |  |  |  |  |  |  |  |  |  |
|  | Name of Load |  | V (kn) | H (kn) | Arm (L) | M (KN.m) |  |  |  |  |
|  | Ph |  |  | -31.5 | 0.47 | -14.81 |  |  |  |  |
|  | Psh |  |  | -1.8 | 0.13 | -0.23 |  |  |  |  |
|  | Ph2 |  |  | 2.98 | 0.26 | 0.7748 |  |  |  |  |
|  | Pm |  |  | -22.86 | 1.88 | -42.9768 |  |  |  |  |
|  | Pv |  | 13.25 |  | 3.25 | 43.0625 |  |  |  |  |
|  | Pv1 |  | 31.5 |  | 3.57 | 112.455 |  |  |  |  |
|  | Pv2 |  | 2.98 |  | 0.26 | 0.7748 |  |  |  |  |
|  | Psv |  | 4.6 |  | 0.33 | 1.518 |  |  |  |  |
|  | Pw |  | 129.6 |  | 2.23 | 289.01 |  |  |  |  |
|  | U1 |  | -2.34 |  | 1.7 | -3.98171231 |  |  |  |  |
|  | U2 |  | -2 |  | 1.13 | -2.26 |  |  |  |  |
|  | Total |  | 177.59 | -53.18 |  | 383.34 |  |  |  |  |
|  | Ft = | Ms / Mt | supporting | Ms= | 447.5931 |  |  |  |  |  |
|  |  |  | overtuening | Mt = | 64.25751231 |  |  |  |  |  |
|  |  | Ft =6.97>Fs=2 |  |  | 6.97 |  | SAFETY |  |  |  |
| (2) |  | Sliding |  |  |  |  |  |  |  |  |
|  | The friction developed between two suface is equal to μΣv, |  |  |  |  |  |  |  |  |  |
|  | where Σv is the algebraic sum of all the vertical forces whether upward or downward, |  |  |  |  |  |  |  |  |  |
|  | and μ is the coefficient of friction between the two surfaces. In order that no sliding takes place, |  |  |  |  |  |  |  |  |  |
|  | the external horizontal forces (ΣH) must be less than the shear resistance μ x ΣV. |  |  |  |  |  |  |  |  |  |
|  |  | From above Table |  |  |  |  | μ ∑ V ∑ H |  |  |  |
|  | Hence, | ΣH<μΣv. |  | F.S.S(factor of safety against sliding)= |  |  |  |  |  |  |
|  |  | The value of μ generally varies from 0.65 to 0.75 . |  |  |  | say , μ= | 0.7 |  |  |  |
|  |  | [Fs.s]=2.0 | F.s.s= | 2.34 |  | F.s.s.=2.0<2.34 |  | SAFE |  |  |
| (3) | Stress on the foundation |  |  |  |  |  |  |  |  |  |
| ① | Distance from toe to Point acting Moment |  |  |  |  |  |  |  |  |  |
|  | E=Σ383.34/Σ177.59 |  |  | = | 2.16 | m |  |  |  |  |
| ② | Distance of eccentricity |  |  |  |  |  |  |  |  |  |
|  | E0 = E - B/2 =2.16 - 3.4/2 |  |  | = | 0.46 | m |  |  |  |  |
|  |  | B=Total Length of Weir |  | = | 3.40 | m |  |  |  |  |
| ③ | Maximum Stress on the foundation (At toe) |  |  |  |  |  |  |  |  |  |
|  | Smax =ΣV/B(1 + 6xE0/B) |  |  | = | 94.6 | KN/m2 |  |  |  |  |
| ④ | Minimum Stress of Foundation (at heel) |  |  |  |  |  |  |  |  |  |
|  | Smin =ΣV/B(1 - 6xE0/B) |  |  | = | 9.8 | KN/m2 |  |  |  |  |
| ⑤ | Cefficienc of Unbalanced Sttlement |  |  |  |  |  |  |  |  |  |
|  | fu = Smax / Smin |  |  | = | 9.65 |  |  |  |  |  |
|  | fu=9.65< f= 〔 3 〕 |  |  | = | CHECK |  |  |  |  |  |
| ⑥ | For | Very Soft | compressive strength |  | 250 | kn/m2 ( See table ) |  |  |  |  |
|  | Thus , | Smax = 94.6< Sf =250 |  |  | SAFE |  |  |  |  |  |

| Uplift Pressure Calculation for weir : |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Let us assume floor thickness at u/s cut-off |  |  |  |  | = | 0.6 | m |  |  |
| and d/s cut-off |  |  |  |  | = | 0.6 | m |  |  |
| U/S Pile |  |  |  |  | D/S Pile |  |  |  |  |
| b |  | = | 5.00 | m | b |  | = | 5.00 | m |
| d |  | = | 2.00 |  | d |  | = | 1.90 |  |
|  | =b/d | = | 2.5 |  |  | =b/d | = | 2.63 |  |
|  | =(1+(1+*)^.5)/2 | = | 1.85 |  |  | =(1+(1+*)^.5)/2 | = | 1.91 |  |
| ΦE | =1/cos-1((-2)/) | = | 52.58 | % | ΦE | =1/cos-1((-2)/) | = | 51.5 | % |
| ΦE1 |  | = | 100 | % | ΦE2 | = ΦE | = | 51.5 | % |
| ΦC1 | = 100-ΦE | = | 47.42 | % | ΦC | 0 | = | 0 | % |
| ΦD | =1/cos-1((-1)/) | = | 34.8 | % | ΦD | =1/cos-1((-1)/) | = | 34.19 | % |
| ΦD1 | =100 - ΦD | = | 65.2 | % | ΦD2 | = ΦD | = | 34.19 | % |
| Corrections: |  |  |  |  |  |  |  |  |  |
| Correction for effect of d/s sheet pile on u/s sheet pile of depth (d) |  |  |  |  |  |  |  |  |  |
| correction | =19*(D/b' )^.5* ((d+D)/b) |  |  | = | 4.40 | %(+ve) |  |  |  |
| where, |  |  |  |  |  |  |  |  |  |
| D=depth of d/s pile below the point C1 |  | = | 1.92 | = |  |  |  |  |  |
| d= depth of u/s pile |  | = | -0.25 | = |  |  |  |  |  |
| b' =distance between two piles |  | = | 4.00 | = |  |  |  |  |  |
| b = total floor length |  | = | 5.00 | = |  |  |  |  |  |
| Correction due to thickness of floor |  |  |  | = | -21.34 | %(+ve) |  |  |  |
| ΦC1corrected |  |  |  | = | 30.48 | % |  |  |  |
| Correction for effect of u/s sheet pile on d/s sheet pile of depth (d) |  |  |  |  |  |  |  |  |  |
| Correction due to thickness of floor |  |  |  | = | 2.70 | %(-ve) |  |  |  |
| ΦEcorrected |  |  |  | = | 48.80 | % |  |  |  |
| Condition of flow | u/s water level in mtr | d/s water level in mtr | Heads in mtr | Height/Elevation of subsoil H.G Line above Datum |  |  |  |  |  |
|  |  |  |  | Upstream pile line |  |  | Down stream pile line |  |  |
|  |  |  |  | ΦE1 | ΦD1 | ΦC1 | ΦE | ΦD | ΦC |
| High flood with concentration and retrogression | 2041.70 | 2040.10 | 1.60 | 1.60 | 1.04 | 0.49 | 0.78 | 0.55 | 0.00 |
|  |  |  |  | 2041.70 | 2041.14 | 2040.59 | 2040.88 | 2040.65 | 2040.10 |
| No Flow,Maximum Static Head | 2039.05 | 2038.05 | 1.00 | 1.00 | 0.34 | 0.30 | 0.49 | 0.34 | 0.00 |
|  |  |  |  | 2039.05 | 2038.39 | 2038.35 | 2038.54 | 2038.39 | 2038.05 |
|  |  |  |  | y1= | 0.77 |  |  |  |  |
| Conclusion |  |  |  |  |  |  |  |  |  |
| As the result of above calculations, The maximum static head will occur on the upstream toe |  |  |  |  |  |  |  |  |  |
| when there is no flood flow. |  |  |  |  |  |  |  |  |  |
| The subsoil hydrauric gradient line is below the water level i.e. all the unbalanced head acting |  |  |  |  |  |  |  |  |  |
| on the weir is counterbalanced by the self weight of the water. |  |  |  |  |  |  |  |  |  |
| However, |  |  |  |  |  |  |  |  |  |
| - Thickness of the weir should be worked out by dividing the maximum static head by 1.24 which is |  |  |  |  |  |  |  |  |  |
| the submerged density of concrete (2.24 - 1). |  |  |  |  |  |  |  |  |  |
| - The value should be cheked by the condition of the minium thickness prescribed (see the table-2) |  |  |  |  |  |  |  |  |  |
| Maximum static head |  | 1.00 | m |  |  |  |  |  |  |
| submerged density of concrete |  | 1.24 | t/m3 |  |  |  |  |  |  |
| Thickness |  | 0.8 | m |  |  |  |  |  |  |
| Thus thickness of the weir is 0.8 m . |  |  |  |  |  |  |  |  |  |

| STABILITY OF WING WALL |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | SYMBOLS USED |  |  |  |  |  |  |  |  |
|  | SN | Description |  |  |  |  |  | Symbol |  |
|  | 1 | Angle made by the wall with the vertical |  |  |  |  |  |  |  |
|  | 2 | Angle of surcharge |  |  |  |  |  |  |  |
|  | 3 | Angle of repose |  |  |  |  |  |  |  |
|  | 4 | Angle made by line of action of pressure with the normal to the batter |  |  |  |  |  |  |  |
|  | 5 | Height of backfill on bank side |  |  |  |  |  | h |  |
|  | 6 | Active earth pressure |  |  |  |  |  | Pa |  |
|  | 7 | Co-efficient of active earth pressure |  |  |  |  |  | Ka |  |
|  | 8 | Horizontal component of active earth pressure |  |  |  |  |  | Pah |  |
|  | 9 | Vertical component of active earth pressure |  |  |  |  |  | Pav |  |
|  | 11 | Passive earth pressure |  |  |  |  |  | Pp |  |
|  | 10 | Co-efficient of passive earth pressure |  |  |  |  |  | Kp |  |
|  | 12 | Horizontal component of passive earth pressure |  |  |  |  |  | Pph |  |
|  | 13 | Vertical component of passive earth pressure |  |  |  |  |  | Ppv |  |
|  | 14 | Dead load |  |  |  |  |  | DL |  |
|  | 15 | Water load |  |  |  |  |  | WL |  |
|  | 16 | Lever arm of resultant |  |  |  |  |  | Z |  |
|  | 17 | Net stabilizing moment |  |  |  |  |  | M |  |
|  | 18 | Net vertical force |  |  |  |  |  | V |  |
|  | 19 | Net vertical force per meter width |  |  |  |  |  | Po |  |
|  | 20 | Eccentricity |  |  |  |  |  | e |  |
|  | 21 | Bottom width of abutment/ wing wall |  |  |  |  |  | b |  |
| 2 | ASSUMPTIONS |  |  |  |  |  |  |  |  |
|  | [1] | Stability is checked at rest condition wing wall being stiff |  |  |  |  |  |  |  |
|  | [2] | Passive earth pressure is considered as applicable |  |  |  |  |  |  |  |
|  | [3] | Stability of wing wall is checked at River Bed, Footing Top and at Foundation Level |  |  |  |  |  |  |  |
|  | [4] | Permissible Stress for Masonry in CM 1:3 |  |  |  |  |  |  |  |
|  |  | (i) For compression = |  | 70.00 | t/m2 |  |  |  |  |
|  |  | (ii) For tension = |  | 10.00 | t/m2 |  |  |  |  |
|  |  | Safe Bearing Capacity at Foundation Level |  |  |  | 20 | t/m2 |  |  |
|  |  | In Eathquke Condition limit of Permissible Stresses is increased by 33% |  |  |  |  |  |  |  |
|  | [5] | Batters for Wing wall |  |  |  |  |  |  |  |
|  |  | Canal side |  | (A) Canal Side |  |  | = | 0.00 |  |
|  |  |  |  | (B) Bank Side |  |  | = | 0.60 |  |
|  | [6] | Top width of Wing wall |  |  |  |  | = | 0.50 | m |
|  | [7} | Angle of repose of soil,  |  |  |  |  | = | 30 | degrees |
|  | [8} | The horizontal seismic coefficient "a" is taken as |  |  |  |  |  | 0.100 |  |
|  | [9} | Importance factor |  |  |  |  |  | 1.000 |  |
|  | [10} | Concrete |  |  |  |  |  | 2.40 | t/m3 |
|  | [11} | Stone Masonry |  |  |  |  |  | 2.30 | t/m3 |
|  | [12} | Dry soil |  |  |  |  |  | 1.80 | t/m3 |
|  | [13} | Saturated soil |  |  |  |  |  | 2.10 | t/m3 |
|  | [14} | Water density |  |  |  |  |  | 1.00 | m |
|  | [15} | Fn Concrete Thick |  |  |  |  |  | 1.00 | m |
|  | [16} | foundation depth |  |  |  |  |  | 0.00 | m |
|  | [17} | CBL |  |  |  |  |  | 2038.05 | m |
|  | [18} | FSL |  |  |  |  |  | 2041.70 | m |
|  | [19} | Free bord |  |  |  |  |  | 0.50 | m |
|  | [20} | Wall Top Height |  |  |  |  |  | 0.00 | m |
|  | [21} | Front surplus |  |  |  |  |  | 0.50 | m |
|  | [22} | Back surplus |  |  |  |  |  | 0.50 | m |
| 3 | COMPUTATION OF FORCES ACTING ON WING WALL AT SECTION A-A |  |  |  |  |  |  |  |  |
|  | considering moments about toe (Left Side) |  |  |  |  |  |  |  |  |
|  | 3.1; CASE-1: Canal empty, saturated backfill and earthquake forces acting adversely.. Computation of Active Earth Pressure Coefficient, Ka :-. (i); Height considered for calculating earth pressure, h; =; 4.150m. (ii); Angle made by wall with vertical, ; =; 30.9640. (iii); Angle of surcharge, ; =; 0.0000. (iv); Angle of repose of soil, ; =; 30.000. (v); Angle made by line of action of center of pressure. with the normal to the batter, ; (Assume  = 3/4 x ); =; 22.50. Active earth pressure,Pa = Ka x  x h2/2. Active earth pressure coefficient, Ka is evaluated by Coulomb's formula. Ka=tan2(45-φ/2); =; 0.33292485. Ka at rest condition = 1.25 x Ka =0.4162. This structure is designed for seismic zone - II. The horizontal seismic coefficient "a" is taken as; =; 0.100. Importance factor; =; 1.000. Horizontal seismic coefficient considered for design; =; 0.100. |

| DESIGN CONSTANTS |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Material |  | Densityt/m3 | Item |  | Value | Item |  | Value |
| Concrete |  | 2.40 | Angle-internal friction |  | 0 | Ka |  | 0.4162 |
| Stone Masonry |  | 2.30 | Surcharge=Kah |  | 0.00 | Kp |  | 0.00 |
| Dry soil |  | 1.80 | Base width, b (m) |  | 2.990 | Slope (H:V) |  | 0.70 |
| Saturated soil |  | 2.10 | Height of Wall (m) |  | 4.150 |  (Radians) |  | 0.6107 |
| Water density |  | 1.00 | Height of surcharge (m) |  | 0.00 |  (Degrees) |  | 34.991 |

| Earthquake co-efficient (For zone II) |  |  |  | v = | 0.05 | h = | 0.100 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| FORCES DUE TO WEIGHT |  |  |  |  | :. Active earth pressure, Pa | = | 7.526 | t/m |
| Item | Calculations |  |  | Force (tonnes) |  | Lever arm | Moments (t-m) |  |
|  |  |  |  | Vertical | Horizontal | (m) | (+ ive) | (- ive) |
| W1 | 0.500 x 0.000 x 2.30 |  |  | 0.000 |  | 0.250 | 0.000 |  |
| W2 | 0.500 x 4.150 x 2.30 |  |  | 4.772 |  | 0.250 | 1.193 |  |
| W3 | 1/2 x 2.490 x 4.150 x 2.30 |  |  | 11.884 |  | 1.330 | 15.806 |  |
| Pah | Pa x Cos( + ) |  |  |  | 7.526 | 1.383 |  | 10.408 |
| W7 | 1/2 x 2.490 x 4.150 x 1.80 |  |  | 9.300 |  | 2.160 | 20.088 |  |
|  |  |  |  | ∑V1 | ∑H1 |  | ∑M1(+) | ∑M1(-) |
|  |  |  |  | 25.956 | 7.526 |  | 37.087 | 10.408 |
| EARTHQUAKE FORCES |  |  |  |  |  |  |  |  |
| Item | Calculations |  |  | Force (tonnes) |  | Lever arm | Moments (t-m) |  |
|  |  |  |  | Vertical | Horizontal | (m) | (+ ive) | (- ive) |
| VERTICAL |  |  |  |  |  |  |  |  |
| ∑V2 | ∑V1 x v | = 25.956 x 0.05 |  | -1.298 |  |  |  | 0.520 |
| HORIZONTAL |  |  |  |  |  |  |  |  |
| H1 | W1 x h | = 0 x 0.1 |  |  | 0.000 | 4.150 |  | 0.000 |
| H2 | W2 x h | = 4.772 x 0.1 |  |  | 0.477 | 2.075 |  | 0.990 |
| H3 | W3 x h | = 11.884 x 0.1 |  |  | 1.188 | 1.383 |  | 1.643 |
| H4 | Pah x h | = 7.526 x 0.1 |  |  | 0.753 | 1.383 |  | 1.041 |
| H5 | W7 x h | = 9.3 x 0.1 |  |  | 0.930 | 2.767 |  | 2.573 |
|  |  |  |  | ∑V2 | ∑H2 |  | ∑M2(+) | SM2(-) |
|  |  |  |  | -1.298 | 3.348 |  | 0.000 | 6.767 |
| CASE-2 | Canal full, dry backfill and earthquake forces acting adversely |  |  |  |  |  |  |  |
| FORCES DUE TO WEIGHT |  |  |  |  | :. Active earth pressure, Pa | = | 6.451 | t/m |
| Item | Calculations |  |  | Force (tonnes) |  | Lever arm | Moments (t-m) |  |
|  |  |  |  | Vertical | Horizontal | (m) | (+ ive) | (- ive) |
| W1 | 0.500 x 0.000 x 2.30 |  |  | 0.000 |  | 0.250 | 0.000 |  |
| W2 | 0.500 x 4.150 x 2.30 |  |  | 4.772 |  | 0.250 | 1.193 |  |
| W3 | 1/2 x 2.490 x 4.150 x 2.30 |  |  | 11.884 |  | 1.330 | 15.806 |  |
| Pah | Pa x Cos( + ) |  |  |  | 6.451 | 1.743 |  | 11.244 |
| W7 | 1/2 x 2.490 x 4.150 x 1.80 |  |  | 9.300 |  | 2.160 | 20.088 |  |
| Pw | 1/2 x 1.00 x 3.650 ^ 2 |  |  |  | -6.661 | 1.217 | 8.106 |  |
|  |  |  |  | ∑V1 | ∑H1 |  | ∑M1(+) | ∑M1(-) |
|  |  |  |  | 25.956 | -0.210 |  | 45.193 | 11.244 |
| EARTHQUAKE FORCES |  |  |  |  |  |  |  |  |
| Item | Calculations |  |  | Force (tonnes) |  | Lever arm | Moments (t-m) |  |
|  |  |  |  | Vertical | Horizontal | (m) | (+ ive) | (- ive) |
| VERTICAL |  |  |  |  |  |  |  |  |
| ∑V2 | ∑V1 x v | = 25.956 x 0.05 |  | -1.298 |  |  |  | 0.562 |
| HORIZONTAL |  |  |  |  |  |  |  |  |
| H1 | W1 x h | = 0 x 0.1 |  |  | 0.000 | 4.150 |  | 0.000 |
| H2 | W2 x h | = 4.772 x 0.1 |  |  | 0.477 | 2.075 |  | 0.990 |
| H3 | W3 x h | = 11.884 x 0.1 |  |  | 1.188 | 1.383 |  | 1.643 |
| H4 | Pah x h | = 6.451 x 0.1 |  |  | 0.645 | 1.743 |  | 1.124 |
| H5 | W7 x h | = 9.3 x 0.1 |  |  | 0.930 | 1.743 |  | 1.621 |
| H6 | Pw x h | = 6.661 x 0.1 |  |  | 0.666 | 1.217 |  | 0.811 |
|  |  |  |  | ∑V2 | ∑H2 |  | ∑M2(+) | ∑M2(-) |
|  |  |  |  | -1.298 | 3.906 |  | 0.000 | 6.751 |
| STABILITY CALCULATIONS AT SECTION A-A |  |  |  |  |  |  |  |  |
| SN | PARTICULARS |  |  | CASE 1 |  | CASE 2 |  | Unit |
|  |  |  |  | Without EQ | With EQ | Without EQ | With EQ |  |
| 1 | ∑V = ∑V1 + ∑V2 |  |  | 25.956 | 24.658 | 25.956 | 24.658 | t/m |
| 2 | ∑H = ∑H1 + ∑H2 |  |  | 7.526 | 10.874 | 0.210 | 3.696 | t/m |
| 3 | ∑M = ∑M(+) - ∑M(-) |  |  | 26.679 | 19.912 | 33.949 | 27.198 | t.m/m |
| 4 | Eccentricity (e) |  |  |  |  |  |  |  |
|  | Z = ∑ M / ∑V |  |  | 1.028 | 0.808 | 1.308 | 1.103 |  |
|  | e = b / 2 - Z |  |  | 0.467 | 0.687 | 0.187 | 0.392 |  |
|  | b / 6 |  |  | 0.498 | 0.498 | 0.498 | 0.498 |  |
| 5 | Pmax = ( ∑V / b) ( 1 + ( 6 x e / b)) |  |  | 16.816 | 19.616 | 11.938 | 14.734 | t/m2 |
|  | It should be less than 70 t/m2 |  |  | SAFE | SAFE | SAFE | SAFE |  |
| 6 | Pmin = ( ∑V / b) ( 1 - ( 6 x e / b)) |  |  | 0.546 | -3.122 | 5.423 | 1.760 | t/m2 |
|  | It should be greater than -10 t/m2 |  |  | SAFE | SAFE | SAFE | SAFE |  |
| 7 | Factor of Safety Against Sliding |  |  |  |  |  |  |  |
|  | F.S.S. = tan  x ∑V / ∑H |  |  | 2.587 | 1.701 | 92.700 | 5.004 |  |
|  | It should be greater than 1.50 |  |  | SAFE | SAFE | SAFE | SAFE |  |
| 8 | Check For Overturning |  |  |  |  |  |  |  |
|  | C.F.O. = ∑M(+) / ∑M(-) |  |  | 3.563 | 2.159 | 4.019 | 2.511 |  |
|  | It should be greater than 2.0 |  |  | SAFE | SAFE | SAFE | SAFE |  |
| Note:- | Coefficient of Friction,tan  = 0.75 for Masonry - Masonry |  |  |  |  |  |  |  |
| OMPUTATION OF FORCES ACTING ON WING WALL AT SECTION B-B |  |  |  |  |  |  |  |  |
| Considering moments about toe |  |  |  |  |  |  |  |  |

| CASE-1 |  | Canal empty, saturated backfill, uplift & earthquake forces acting adversely |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DESIGN CONSTANTS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Material |  |  |  | Densityt/m3 | Item |  | Value | Item |  | Value |  |  |  |  |
| Concrete |  |  |  | 2.40 | Angle-internal friction |  | 0 | Ka |  | 0.4162 |  |  |  |  |
| Stone Masonry |  |  |  | 2.30 | Surcharge = Kah |  | 0 | Kp |  | 3.000 |  |  |  |  |
| Dry soil |  |  |  | 1.80 | Base width, bw (m) |  | 2.990 | Slope (H:V) |  | 0.60 |  |  |  |  |
| Saturated soil |  |  |  | 2.10 | Height of Wall (m) |  | 4.150 | q (Radians) |  | 0.6107 |  |  |  |  |
| Water density |  |  |  | 1.00 | Height of surcharge (m) |  | 0.00 | q (Degrees) |  | 34.991 |  |  |  |  |
| Earthquake co-efficient (For zone II- BIS) |  |  |  |  |  | av = | 0.05 | ah = | 0.100 |  |  |  |  |  |
| FORCES DUE TO WEIGHT |  |  |  |  |  |  | :. Active earth pressure, Pa | = | 7.526 | t/m |  |  |  |  |
| Item |  | Calculations |  |  |  | Force (tonnes) |  | Lever arm | Moments (t-m) |  |  |  |  |  |
|  |  |  |  |  |  | Vertical | Horizontal | (m) | (+ ive) | (- ive) |  |  |  |  |
| W1 |  | 0.500 x 0.000 x 2.30 |  |  |  | 0.000 |  | 0.250 | 0.000 |  |  |  |  |  |
| W2 |  | 0.500 x 4.150 x 2.30 |  |  |  | 4.772 |  | 0.250 | 1.193 |  |  |  |  |  |
| W3 |  | 1/2 x 2.490 x 4.150 x 2.30 |  |  |  | 11.884 |  | 1.330 | 15.806 |  |  |  |  |  |
| W4 |  | 1/2 x 0.000 x 0.00 x 2.30 |  |  |  | 0.000 |  | 2.990 | 0.000 |  |  |  |  |  |
| W5 |  | 2.990 x 0.000 x 2.30 |  |  |  | 0.000 |  | 1.495 | 0.000 |  |  |  |  |  |
| Pa |  |  |  |  |  |  | 7.526 | 1.369 |  | 10.303 |  |  |  |  |
| W7 |  | 1/2 x 2.990 x 4.15 x 1.80 |  |  |  | 11.168 |  | 2.493 | 27.842 |  |  |  |  |  |
| Pp |  | 1/2 x 0.000 x 0.00 |  |  |  |  | 0.000 | 0.000 | 0.000 |  |  |  |  |  |
| Ue |  | 1/2 x 2.990 x 2.075 x 1.00 |  |  |  | -3.102 |  | 1.993 |  | 6.182 |  |  |  |  |
|  |  |  |  |  |  | ∑V1 | ∑H1 |  | ∑M1(+) | ∑M1(-) |  |  |  |  |
|  |  |  |  |  |  | 24.722 | 7.526 |  | 44.841 | 16.485 |  |  |  |  |
| EARTHQUAKE FORCES |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Item |  | Calculations |  |  |  | Force (tonnes) |  | Lever arm | Moments (t-m) |  |  |  |  |  |
|  |  |  |  |  |  | Vertical | Horizontal | (m) | (+ ive) | (- ive) |  |  |  |  |
| VERTICAL |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ∑V2 |  | ∑V1 x v |  | = 24.722 x 0.05 |  | -1.236 |  |  |  | 0.824 |  |  |  |  |
| HORIZONTAL |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| H1 |  | W1 x h |  | = 0 x 0.1 |  |  | 0.000 | 4.150 |  | 0.000 |  |  |  |  |
| H2 |  | W2 x h |  | = 4.772 x 0.1 |  |  | 0.477 | 2.075 |  | 0.990 |  |  |  |  |
| H3 |  | W3 x h |  | = 11.884 x 0.1 |  |  | 1.188 | 1.383 |  | 1.643 |  |  |  |  |
| H4 |  | W4 x h |  | = 0 x 0.1 |  |  | 0.000 | 0.000 |  | 0.000 |  |  |  |  |
| H5 |  | W5 x h |  | = 0 x 0.1 |  |  | 0.000 | 0.000 |  | 0.000 |  |  |  |  |
| H6 |  | Pa x h |  | = 7.526 x 0.1 |  |  | 0.753 | 1.743 |  | 1.312 |  |  |  |  |
| H7 |  | W7x h |  | = 11.168 x 0.1 |  |  | 1.117 | 2.767 |  | 3.091 |  |  |  |  |
| H8 |  | Pp x h |  | = 0 x 0.1 |  |  | 0.000 | 0.000 |  | 0.000 |  |  |  |  |
|  |  |  |  |  |  | ∑V2 | ∑H2 |  | ∑M2(+) | SM2(-) |  |  |  |  |
|  |  |  |  |  |  | -1.236 | 3.535 |  | 0.000 | 7.860 |  |  |  |  |
| CASE-2 |  | Canal full, dry backfill, uplift and earthquake forces acting adversely |  |  |  |  |  |  |  |  |  |  |  |  |
| FORCES DUE TO WEIGHT |  |  |  |  |  |  | :. Active earth pressure, Pa | = | 6.451 | t/m |  |  |  |  |
| Item |  | Calculations |  |  |  | Force (tonnes) |  | Lever arm | Moments (t-m) |  |  |  |  |  |
|  |  |  |  |  |  | Vertical | Horizontal | (m) | (+ ive) | (- ive) |  |  |  |  |
| W1 |  | 0.500 x 0.000 x 2.30 |  |  |  | 0.000 |  | 0.250 | 0.000 |  |  |  |  |  |
| W2 |  | 0.500 x 4.150 x 2.30 |  |  |  | 4.772 |  | 0.250 | 1.193 |  |  |  |  |  |
| W3 |  | 1/2 x 2.490 x 4.150 x 2.30 |  |  |  | 11.884 |  | 1.330 | 15.806 |  |  |  |  |  |
| W4 |  | 1/2 x 0.000 x 0.00 x 2.30 |  |  |  | 0.000 |  | 2.990 | 0.000 |  |  |  |  |  |
| W5 |  | 2.990 x 0.000 x 2.30 |  |  |  | 0.000 |  | 1.495 | 0.000 |  |  |  |  |  |
| Pa |  | Pa x Cos( + ) |  |  |  |  | 6.451 | 1.369 |  | 8.831 |  |  |  |  |
| W7 |  | 1/2 x 2.990 x 4.15 x 1.80 |  |  |  | 11.168 |  | 2.767 | 30.902 |  |  |  |  |  |
| Pw |  | 1/2 x 1.00 x 3.650 ^ 2 |  |  |  |  | -6.661 | 1.217 | 8.106 |  |  |  |  |  |
| Pp |  | 0.5 x (0.000 + 3.650) x 0.00 |  |  |  |  | 0.000 | 0.000 | 0.000 |  |  |  |  |  |
| Uf |  | 1/2 x 2.990 x 3.650 x 1.00 |  |  |  | -5.457 |  | 0.997 |  | 5.441 |  |  |  |  |
|  |  |  |  |  |  | ∑V1 | ∑H1 |  | ∑M1(+) | ∑M1(-) |  |  |  |  |
|  |  |  |  |  |  | 22.367 | -0.210 |  | 56.007 | 14.272 |  |  |  |  |
| EARTHQUAKE FORCES |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Item |  | Calculations |  |  |  | Force (tonnes) |  | Lever arm | Moments (t-m) |  |  |  |  |  |
|  |  |  |  |  |  | Vertical | Horizontal | (m) | (+ ive) | (- ive) |  |  |  |  |
| VERTICAL |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ∑V2 |  | ∑V1 x v |  | = 22.367 x 0.05 |  | -1.118 |  |  |  | 0.714 |  |  |  |  |
| HORIZONTAL |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| H1 |  | W1 x h |  | = 0 x 0.1 |  |  | 0.000 | 4.150 |  | 0.000 |  |  |  |  |
| H2 |  | W2 x h |  | = 4.772 x 0.1 |  |  | 0.477 | 2.075 |  | 0.990 |  |  |  |  |
| H3 |  | W3 x h |  | = 11.884 x 0.1 |  |  | 1.188 | 1.383 |  | 1.643 |  |  |  |  |
| H4 |  | W4 x h |  | = 0 x 0.1 |  |  | 0.000 | 0.000 |  | 0.000 |  |  |  |  |
| H5 |  | W5 x h |  | = 0 x 0.1 |  |  | 0.000 | 0.000 |  | 0.000 |  |  |  |  |
| H6 |  | Pa x h |  | = 6.451 x 0.1 |  |  | 0.645 | 1.743 |  | 1.124 |  |  |  |  |
| H7 |  | W7 x h |  | = 11.168 x 0.1 |  |  | 1.117 | 2.767 |  | 3.091 |  |  |  |  |
| H8 |  | Pw x h |  | = 6.661 x 0.1 |  |  | 0.666 | 1.217 |  | 0.811 |  |  |  |  |
| H9 |  | Pp x h |  | = 0 x 0.1 |  |  | 0.000 | 0.000 |  | 0.000 |  |  |  |  |
|  |  |  |  |  |  | ∑V2 | ∑H2 |  | ∑M2(+) | SM2(-) |  |  |  |  |
|  |  |  |  |  |  | -1.118 | 4.093 |  | 0.000 | 8.373 |  |  |  |  |
| STABILITY CALCULATIONS AT SECTION B-B |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| SN |  | PARTICULARS |  |  |  | CASE 1 |  | CASE 2 |  | Unit |  |  |  |  |
|  |  |  |  |  |  | Without EQ | With EQ | Without EQ | With EQ |  |  |  |  |  |
| 1 |  | ∑V = ∑V1 + ∑V2 |  |  |  | 24.722 | 23.486 | 22.367 | 21.249 | t/m |  |  |  |  |
| 2 |  | ∑H = ∑H1 + ∑H2 |  |  |  | 7.526 | 11.061 | 0.210 | 3.883 | t/m |  |  |  |  |
| 3 |  | ∑M = ∑M(+) - ∑M(-) |  |  |  | 28.356 | 20.496 | 41.735 | 33.362 | t.m/m |  |  |  |  |
| 4 |  | Eccentricity (e) |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Z = ∑ M / ∑V |  |  |  | 1.147 | 0.873 | 1.866 | 1.570 |  |  |  |  |  |
|  |  | e = b / 2 - Z |  |  |  | 0.348 | 0.622 | 0.371 | 0.075 |  |  |  |  |  |
|  |  | b / 6 |  |  |  | 0.498 | 0.498 | 0.498 | 0.498 |  |  |  |  |  |
| 5 |  | Pmax = ( ∑V / b) ( 1 + ( 6 x e / b)) |  |  |  | 14.042 | 17.659 | 13.05 | 8.176 | t/m2 |  |  |  |  |
|  |  | It should be less than 70 t/m2 |  |  |  | SAFE | SAFE | SAFE | SAFE |  |  |  |  |  |
| 6 |  | Pmin = ( ∑V / b) ( 1 - ( 6 x e / b)) |  |  |  | 2.494 | -1.949 | 1.911 | 6.037 | t/m2 |  |  |  |  |
|  |  | It should be greater than -10 t/m2 |  |  |  | SAFE | SAFE | SAFE | SAFE |  |  |  |  |  |
| 7 |  | Factor of Safety Against Sliding |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | SF = tan  x ∑V / ∑H |  |  |  | 2.464 | 1.592 | 79.882 | 4.104 |  |  |  |  |  |
|  |  | It should be greater than 1.50 |  |  |  | SAFE | SAFE | SAFE | SAFE |  |  |  |  |  |
| 8 |  | Check For Overturning |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | C.F.O. = ∑M(+) / ∑M(-) |  |  |  | 2.720 | 1.842 | 3.924 | 2.473 |  |  |  |  |  |
|  |  | It should be greater than 1.5 |  |  |  | SAFE | SAFE | SAFE | SAFE |  |  |  |  |  |
| Note:- |  | Coefficient of Friction,tan  = 0.75 for Masonry - Masonry |  |  |  |  |  |  |  |  |  |  |  |  |
| 5 |  | COMPUTATION OF FORCES ACTING ON WING WALL AT SECTION C-C |  |  |  |  |  |  |  |  |  |  |
|  |  | Considering moments about toe (Left Side) |  |  |  |  |  |  |  |  |  |  |

| 5.1 | CASE-1 | Canal empty, saturated backfill, uplift & earthquake forces acting adversely |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | DESIGN CONSTANTS |  |  |  |  |  |  |  |  |
|  | Material |  | Densityt/m3 | Item |  | Value | Item |  | Value |
|  | Concrete |  | 2.40 | Angle-internal friction |  | 30 | Ka |  | 0.4162 |
|  | Stone Masonry |  | 2.30 | Surcharge=Kagh |  | 0 | Kp |  | 3.000 |
|  | Dry soil |  | 1.80 | Base width, b (m) |  | 3.490 | Slope (H:V) |  | 0.70 |
|  | Saturated soil |  | 2.10 | Height of Wall (m) |  | 5.150 | q (Radians) |  | 0.6107 |
|  | Water density |  | 1.00 | Height of surcharge (m) |  | 0.00 | q (Degrees) |  | 34.991 |
|  | Earthquake co-efficient (For zone II) |  |  |  | av = | 0.05 | ah = | 0.100 |  |
|  | FORCES DUE TO WEIGHT |  |  |  |  | :. Active earth pressure, Pa | = | 9.934 | t/m |
|  | Item | Calculations |  |  | Force (tonnes) |  | Lever arm | Moments (t-m) |  |
|  |  |  |  |  | Vertical | Horizontal | (m) | (+ ive) | (- ive) |
|  | W1 | 0.000 x 0.000 x 2.40 |  |  | 0.000 |  | 0.500 | 0.000 |  |
|  | W2 | 0.000 x 4.150 x 2.40 |  |  | 0.000 |  | 0.500 | 0.000 |  |
|  | W3 | 1/2 x 2.490 x 4.150 x 2.30 |  |  | 11.884 |  | 1.330 | 15.806 |  |
|  | W4 | 1/2 x 0.000 x 0.000 x 2.30 |  |  | 0.000 |  | 2.990 | 0.000 |  |
|  | W5 | 2.490 x 0.000 x 2.30 |  |  | 0.000 |  | 1.745 | 0.000 |  |
|  | W6 | 3.490 x 1.000 x 2.40 |  |  | 4.886 |  | 1.745 | 8.526 |  |
|  | W7 | 1/2 x 4.150 x 2.490 x 1.80 |  |  | 9.300 |  | 2.160 | 20.088 |  |
|  | Pa | Pa |  |  |  | 9.934 | 1.699 |  | 16.878 |
|  | Pp | 1/2 x 6.300 x 1.00 |  |  |  | -3.150 | 0.333 | 1.049 |  |
|  |  |  |  |  | ∑V1 | ∑H1 |  | ∑M1(+) | ∑M1(-) |
|  |  |  |  |  | 26.070 | 6.784 |  | 45.469 | 16.878 |
|  | EARTHQUAKE FORCES |  |  |  |  |  |  |  |  |
|  | Item | Calculations |  |  | Force (tonnes) |  | Lever arm | Moments (t-m) |  |
|  |  |  |  |  | Vertical | Horizontal | (m) | (+ ive) | (- ive) |
|  | VERTICAL |  |  |  |  |  |  |  |  |
|  | ∑V2 | ∑V1 x v | = 26.07 x 0.05 |  | -1.304 |  |  |  | 0.844 |
|  | HORIZONTAL |  |  |  |  |  |  |  |  |
|  | H1 | W1 x h | = 0 x 0.1 |  |  | 0.000 | 5.150 |  | 0.000 |
|  | H2 | W2 x h | = 0 x 0.1 |  |  | 0.000 | 3.075 |  | 0.000 |
|  | H3 | W3 x h | = 11.884 x 0.1 |  |  | 1.188 | 2.383 |  | 2.831 |
|  | H4 | W4 x h | = 0 x 0.1 |  |  | 0.000 | 1.000 |  | 0.000 |
|  | H5 | W5 x h | = 0 x 0.1 |  |  | 0.000 | 1.000 |  | 0.000 |
|  | H6 | W6 x h | = 4.886 x 0.1 |  |  | 0.489 | 0.500 |  | 0.245 |
|  | H7 | Pah x h | = 9.934 x 0.1 |  |  | 0.993 | 2.163 |  | 2.148 |
|  | H8 | W7 x h | = 9.3 x 0.1 |  |  | 0.930 | 4.433 |  | 4.123 |
|  | H9 | Pp x h | = 3.15 x 0.1 |  |  | -0.315 | 0.333 |  | -0.105 |
|  |  |  |  |  | ∑V2 | ∑H2 |  | ∑M2(+) | SM2(-) |
|  |  |  |  |  | -1.304 | 3.285 |  | 0.000 | 10.086 |
| 5.2 | CASE-2 | Canal full, dry backfill, uplift and earthquake forces acting adversely |  |  |  |  |  |  |  |
|  | FORCES DUE TO WEIGHT |  |  |  |  | :. Active earth pressure, Pa | = | 6.071 | t/m |
|  | Item | Calculations |  |  | Force (tonnes) |  | Lever arm | Moments (t-m) |  |
|  |  |  |  |  | Vertical | Horizontal | (m) | (+ ive) | (- ive) |
|  | W1 | 0.000 x 0.000 x 1.40 |  |  | 0.000 |  | 0.500 | 0.000 |  |
|  | W2 | 0.000 x 4.150 x 1.30 |  |  | 0.000 |  | 0.500 | 0.000 |  |
|  | W3 | 1/2 x 2.490 x 4.150 x 1.30 |  |  | 6.717 |  | 1.330 | 8.934 |  |
|  | W4 | 1/2 x 0.000 x 0.000 x 1.30 |  |  | 0.000 |  | 2.990 | 0.000 |  |
|  | W5 | 2.490 x 0.000 x 1.30 |  |  | 0.000 |  | 1.745 | 0.000 |  |
|  | W6 | 3.490 x 1.000 x 1.40 |  |  | 4.886 |  | 1.745 | 8.526 |  |
|  | Pa | Pa |  |  |  | 6.071 | 1.699 |  | 10.315 |
|  | W7 | 1/2 x 4.150 x 2.490 x 1.80 |  |  | 5.683 |  | 2.160 | 12.275 |  |
|  | Pp | 0.5 x 3.300 x 1.00 |  |  |  | -1.650 | 0.333 | 3.564 |  |
|  |  |  |  |  | ∑V1 | ∑H1 |  | ∑M1(+) | ∑M1(-) |
|  |  |  |  |  | 17.286 | 4.421 |  | 33.299 | 10.315 |
|  | EARTHQUAKE FORCES |  |  |  |  |  |  |  |  |
|  | Item | Calculations |  |  | Force (tonnes) |  | Lever arm | Moments (t-m) |  |
|  |  |  |  |  | Vertical | Horizontal | (m) | (+ ive) | (- ive) |
|  | VERTICAL |  |  |  |  |  |  |  |  |
|  | ∑V2 | ∑V1 x v | = 17.286 x 0.05 |  | -0.864 |  |  |  | 0.516 |
|  | HORIZONTAL |  |  |  |  |  |  |  |  |
|  | H1 | W1 x h | = 0 x 0.1 |  |  | 0.000 | 5.150 |  | 0.000 |
|  | H2 | W2 x h | = 0 x 0.1 |  |  | 0.000 | 3.075 |  | 0.000 |
|  | H3 | W3 x h | = 6.717 x 0.1 |  |  | 0.672 | 2.383 |  | 1.601 |
|  | H4 | W4 x h | = 0 x 0.1 |  |  | 0.000 | 1.000 |  | 0.000 |
|  | H5 | W5 x h | = 0 x 0.1 |  |  | 0.000 | 1.000 |  | 0.000 |
|  | H6 | W6 x h | = 4.886 x 0.1 |  |  | 0.489 | 0.500 |  | 0.245 |
|  | H7 | Pah x h | = 6.071 x 0.1 |  |  | 0.607 | 2.163 |  | 1.313 |
|  | H8 | W7 x ah | = 5.683 x 0.1 |  |  | 0.568 | 2.163 |  | 1.229 |
|  | H10 | Pp x h | = 1.65 x 0.1 |  |  | -0.165 | 0.333 |  | -0.055 |
|  |  |  |  |  | ∑V2 | ∑H2 |  | ∑M2(+) | SM2(-) |
|  |  |  |  |  | -0.864 | 2.171 |  | 0.000 | 4.849 |
| 5.3 | STABILITY CALCULATIONS AT SECTION C-C |  |  |  |  |  |  |  |  |
|  | SN | PARTICULARS |  |  | CASE 1 |  | CASE 2 |  | Unit |
|  |  |  |  |  | Without EQ | With EQ | Without EQ | With EQ |  |
|  | 1 | ∑V = ∑V1 + ∑V2 |  |  | 26.070 | 24.766 | 17.286 | 16.422 | t/m |
|  | 2 | ∑H = ∑H1 + ∑H2 |  |  | 6.784 | 10.069 | 4.421 | 6.592 | t/m |
|  | 3 | ∑M = ∑M(+) - ∑M(-) |  |  | 28.591 | 18.505 | 22.984 | 18.135 | t.m/m |
|  | 4 | Eccentricity (e) |  |  |  |  |  |  |  |
|  |  | Z = ∑ M / ∑V |  |  | 1.097 | 0.747 | 1.330 | 1.104 |  |
|  |  | e = b / 2 - Z |  |  | 0.648 | 0.998 | 0.415 | 0.641 |  |
|  |  | b / 6 |  |  | 0.582 | 0.582 | 0.582 | 0.582 |  |
|  |  | e <= b / 6 |  |  | UNSAFE | SAFE | SAFE | SAFE |  |
|  | 5 | Pmax = ( ∑V / b) ( 1 + ( 6 x e / b)) |  |  | 15.792 | 19.272 | 8.487 | 9.891 | t/m2 |
|  |  | It should be less than 70 t/m2 |  |  | SAFE | SAFE | SAFE | SAFE |  |
|  | 6 | Pmin = ( ∑V / b) ( 1 - ( 6 x e / b)) |  |  | -0.852 | -5.079 | 1.419 | -0.480 | t/m2 |
|  |  | It should be greater than10 t/m2 |  |  | SAFE | SAFE | SAFE | SAFE |  |
|  | 7 | Factor of Safety Against Sliding |  |  |  |  |  |  |  |
|  |  | SF = tan  x ∑V / ∑H |  |  | 2.219 | 1.420 | 2.257 | 1.437 |  |
|  |  | It should be greater than 1.50 |  |  | SAFE | SAFE | SAFE | SAFE |  |
|  | 8 | Check For Overturning |  |  |  |  |  |  |  |
|  |  | C.F.O. = ∑M(+) / ∑M(-) |  |  | 2.694 | 1.686 | 3.228 | 2.196 |  |
|  |  | It should be greater than 2.0 |  |  | SAFE | SAFE | SAFE | SAFE |  |

| Design of Intake |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Circular intake |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Intake capacity, Q(m3/s) = |  |  |  | 0.04 |  | m3/sec |  |  |  |  |  |  |  |  |  |  |
| Length of PVC pipe, L (m) = |  |  |  | 54 |  | m |  |  |  |  |  |  |  |  |  |  |
| Pipe Diameter, Ø (m) = |  |  |  | 0.30 |  | m |  |  |  |  |  |  |  |  |  |  |
| Roughness Coefficient, n = |  |  |  | 0.011 |  |  |  |  |  |  |  |  |  |  |  |  |
| Intake lose, h = (v2/2g) (FT) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Where FT = Entry+Exit +Friction loses = 0.5+1.0+Fl |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| And Fl = (fxL)/ Øp, Where f = 124.6n2 /Øp 1/3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Hence, using a 100% full pipe of diameter, the intake loss, h is calculated. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| The size of the pipe is increased for ease of access during cleaning. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| f = 124.6(0.014)2/ (Øp )1/3 = |  |  |  | 0.023 |  | m |  |  |  |  |  |  |  |  |  |  |
| Fl = f*L/Øp = |  |  |  | 4.05 |  | m |  |  |  |  |  |  |  |  |  |  |
| FT= 0.5+1.0+F1 = |  |  |  | 5.55 |  |  |  |  |  |  |  |  |  |  |  |  |
| A = 3.14 *Øp^2/4 = |  |  |  | 0.07 |  | m2 |  |  |  |  |  |  |  |  |  |  |
| V = Q/A = |  |  |  | 0.57 |  | m/s |  |  |  |  |  |  |  |  |  |  |
| Therefore, h = (V2/2g)* (FT) = |  |  |  | 0.09 |  | m |  |  |  |  |  |  |  |  |  |  |
|  |  | Say= |  | 0.10 |  | m |  |  |  |  |  |  |  |  |  |  |
| Level of river bed = |  | 2038.05 |  | m |  |  |  |  |  |  |  |  |  |  |  |  |
| Pond level= |  | 2039.05 |  | m |  |  |  |  |  |  |  |  |  |  |  |  |
| Level of intake at the intake gate = 2039.05m |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Sill height, H = |  |  |  | 0.30 |  | m |  |  |  |  |  |  |  |  |  |  |
| Pipe invert level = river bed level + Sill height = 2039.35m |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Top level of intake pipe = river bed level + sill height + pipe diameter =2039.65m |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Water level of outlet =2038.95m |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Air influx condition check |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Height from pipe top to water surface d= |  |  |  |  |  | -0.60 |  |  | m |  |  |  |  |  |  |  |
| 2 x Velocity head= |  | 0.03 |  | m << |  | d=-0.6m |  |  | So Ok |  |  |  |  |  |  |  |
| Steel bar mesh is used at the in let of the pipe to protect debris and other foreign materials. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| The design & drawings of intake gate is used that of IDD standard gate design and drawings. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

## Silt Gate

The silt gate is located adjacent to the intake gate. It is 0.30m lower than the intake level i.e. 2039.35 and 0.50m wide each. Hence depth below crest level is 0.55m; this gate is to be operated (lifted) manually during the rainy season. The plate thickness shall be 5mm.

## Canal water escape

Excess flow has to spill out of the conveyance canal and hence canal water escape is located at 0+28m can serve this purpose making the escape opening to F.S.L of main conveyance canal and is 0.50m wide controlled by simple wooden log or steel shutter.

# DESIGN OF SUPPLY CANALS

## Main Canal

The design discharge of the main canal is 38.68 l/s however; the canal is designed for 40l/s by providing free board of 25cm for safety purpose and for farmers to use extra flows. General formula adopted for designing of the distribution canals including main canal is Manning's formula. General formula Adopted is Manning's.

Q = A/n R 3/2 s ½ 						n = 0.014 for lined section

Economical lined Rectangular section criteria;

B = 2y				B  = bed width of canal

R = y/2			y  = water depth	Side slope m=1.0 (inner and outer) for small Q

R = Hydraulic radius

| HYDRAULIC OF CANAL |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CANAL NAME |  |  | MAIN CANAL (CH+0 - CH +2242.61) |  |  |  |  |  |  |
| Chainage | F.S.D | F.B | Slope | Bed width | Side slope | Wetted perimeter | Hyd. radius | Flow velocity | Dis. Des. |
|  | m | m |  | m |  | m | m | m/s | m3/s |
| CH+0 - CH+825 | 0.30 | 0.25 | 450.00 | 0.40 | 0.00 | 1.00 | 0.12 | 0.38 | 0.05 |
| CH+825- CH+1275 | 0.15 | 0.25 | 57.00 | 0.20 | 1.00 | 0.62 | 0.08 | 0.85 | 0.04 |
| CH+1275- CH+2242.61 | 0.15 | 0.25 | 145.00 | 0.35 | 1.00 | 0.77 | 0.10 | 0.58 | 0.04 |

Table: Detail of supply canal

b) Design of Secondary and Tertiary canals

| Canal Name | D | B | Top width | RHtd rad | n | S | FB | Velocity | Qcal | QSC(act) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | m | m | m | m |  | m/m | m | m/s | m3/s | m3/s |
| SC-1-1 | 0.20 | 0.35 | 0.75 | 0.12 | 0.025 | 0.0015 | 0.25 | 0.38 | 0.0275 | 0.0212 |
| TC-1-1-1 | 0.20 | 0.35 | 0.75 | 0.12 | 0.025 | 0.0015 | 0.25 | 0.38 | 0.0275 | 0.0062 |
| TC-1-1-2 | 0.20 | 0.35 | 0.75 | 0.12 | 0.025 | 0.0015 | 0.25 | 0.38 | 0.0275 | 0.0023 |
| TC-1-1-3 | 0.20 | 0.35 | 0.75 | 0.12 | 0.025 | 0.0015 | 0.25 | 0.38 | 0.0275 | 0.0028 |
| TC-1-1-4 | 0.20 | 0.35 | 0.75 | 0.12 | 0.025 | 0.0015 | 0.25 | 0.38 | 0.0275 | 0.0051 |
| TC-1-1-5 | 0.20 | 0.35 | 0.75 | 0.12 | 0.025 | 0.0015 | 0.25 | 0.38 | 0.0275 | 0.0028 |
| TC-1-1-6 | 0.20 | 0.35 | 0.75 | 0.12 | 0.025 | 0.0015 | 0.25 | 0.38 | 0.0275 | 0.0015 |
| TC-1-0-1 | 0.15 | 0.30 | 0.60 | 0.09 | 0.025 | 0.0015 | 0.25 | 0.32 | 0.0215 | 0.0101 |
| TC-1-0-2 | 0.15 | 0.30 | 0.60 | 0.09 | 0.025 | 0.0015 | 0.25 | 0.32 | 0.0215 | 0.0101 |

## Design of drainage canals

As stated under the hydrology report, the design storm resulted in drainage modulus of 2.8 l/s per ha. Thus based on area draining towards each drainage canal the discharge was calculated. Thus it was designed for this flow manning roughness co-efficient of 0.03 was adopted. And based General Manning’s formula stated below both irrigation and drainage canals are designed so as economic section of trapezoidal canal was take in to consideration.

The design formula and design parameters are as follows:

Q = A/n R 2/3 S 1/2 (general Manning’s formula)

Manning’s roughness coefficient for channel with moderate weeds, n = 0.03

Channel side slope to be adopted is 1V:1.5H

Figure 4. Basic section of drainage canal

Majority of the area drain to natural drainage routes and thus it was not essential to consider flow hydraulics of these natural water courses, but it is obvious that the local community has to take care of these drainage courses not to develop gullies. Main drain along main canal is to be considered on the layout plan. Design parameters were summarized as follows. Field canals and field drains are to be prepared by the user community under the guidance of district irrigation technicians. Drop structures are provided on drainage canals to control the bed slope in the design range.

## Design of Hydraulic Structures

a) Design of Pipe Culverts

Pipe culverts are designed based on partial flow condition with velocity of flow in pipe greater than 1m/s which is non-silting and non-scouring velocity. The concrete pipe shall be filled with selected fill to depth greater than 0.60m over the crown. Culverts were designed to have 6m span for free passage of any vehicle in the field.

Water depth in pipe is preferred to be H= 0.8D for good flow hydraulics under economical condition and from table for H/D=0.80 or H/r = 1.60,

| Hydraulic design of the box part |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Canal | Chainage(m) | L(m) | n | v2(m/sec) | d | B(m) | A(m2) | P(m) | R(m) | b/d | Fr |
| MC-1 | 1538 | 8 | 0.014 | 0.5839 | 0.1500 | 0.35 | 0.05 | 0.7 | 0.08 | 2.33 | 0.68 |
| SC-1-1 | 28 | 8 | 0.014 | 0.3143 | 0.2000 | 0.35 | 0.07 | 0.8 | 0.09 | 1.75 | 0.32 |
| SC-1-1 | 214 | 8 | 0.014 | 0.2654 | 0.1500 | 0.30 | 0.05 | 0.6 | 0.08 | 2.00 | 0.31 |

Downstream water level = u/s canal water level-hydraulic losses

Hydraulic loss=1.2 (V22-V12)/2g + friction loss

Friction loss for canal bed slope, I =IL

#### b) Design of Aqueducts

In design of aqueducts the pipe length is maximum 1.50m which is too short thus, total hydraulic loss is negligible. It was designed to pass, the maximum drainage based on the drainage module previously stated. Thus the orifices formula governs the flow through the pipe in aqueduct.

Q=C*A*(2gh)

Where C=1.1+0.026L/ (D) 1.2 -0.50 for concrete pipe of beveled lip entrance, h is level difference between upstream and downstream water surface and A= D2/4 where, D is pipe inner diameter in meter.

| TABLE OF DIMENTION FOR CROSS DRAINAGES |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Structure Name | Chanage | d | D1 | B1 | D2 | B2 | L1 | L2 | L3 | T |
|  | m | mm | m | m | m | m | m | m | m | m |
| CD-1 | CH +410.67 | 600 | 0.45 | 0.20 | 1.00 | 2.20 | 2.50 | 2.00 | 2.50 | 1.10 |
| CD-2 | CH +657.62 | 600 | 0.45 | 0.20 | 1.00 | 2.20 | 2.50 | 2.00 | 2.50 | 1.10 |
| CD-4 | CH +1538.42 | 500 | 0.45 | 0.20 | 1.00 | 2.20 | 2.50 | 2.00 | 2.50 | 1.10 |
| CD-5 | CH+2051.34 | 500 | 0.45 | 0.20 | 1.00 | 2.20 | 2.50 | 2.00 | 2.50 | 1.10 |

c) Design of pipe flume

The flume is designed to convey 40l/s flow by 2 in number dia.200mm steel pipe. Under the current design condition 200mm pipe diameter is found to be enough to convey even more than 40l/s with head differential of 0.30m between upstream and downstream accounting for all losses.

d) Design of turnout and field off takes

The flow in turnouts and field off takes is again governed by the orifice formula. It basically depends on the pipe size and head creating flow.

Q=Cd A(2gh) where h=y-(D/2) and A=D2/4

Where Cd is the coefficient of discharge, 0.80 for submerged out flow and 0.62 for free out flow, A is the area of pipe, g= Acceleration due to gravity (9.81 m/s), and h is the head creating flow in meter.

Each field off take and turn out are to be controlled with simple shutters with chain to lift to required level.

Table 8-11: Typical canal off take and turnout

| TERTIARY CANAL OFFTAKE |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Parent Canal Name | Canal Name | Chainage(m) | Parent canal FSL | Offtake CBL | Offtake FSL | Offtake canal depth | Qd(m3/s) | Qcal in flow offtake | Pipe intake Diameter.(D) | Crosectional Area (A) | Coeff. Q Cd | Headloss(m) |
| MC | TC-1-0-1 | 994.94 | 2033.97 | 2033.79 | 2033.94 | 0.15 | 0.0459 | 0.0459 | 0.36 | 0.10383 | 0.6 | 0.03 |
| SC-1-1 | TC-1-1-1 | 6.67 | 2027.23 | 2027.02 | 2027.17 | 0.15 | 0.0275 | 0.0275 | 0.23 | 0.04097 | 0.6 | 0.06 |
| SC-1-1 | TC-1-1-2 | 18.95 | 2026.39 | 2026.2 | 2026.35 | 0.15 | 0.0275 | 0.0275 | 0.25 | 0.04809 | 0.6 | 0.05 |
| SC-1-1 | TC-1-1-3 | 216.49 | 2025.33 | 2025.07 | 2025.22 | 0.15 | 0.0275 | 0.0275 | 0.2 | 0.03049 | 0.6 | 0.12 |
| SC-1-1 | TC-1-1-4 | 295.67 | 2023.36 | 2023.08 | 2023.23 | 0.15 | 0.0275 | 0.0275 | 0.19 | 0.02872 | 0.6 | 0.13 |
| SC-1-1 | TC-1-1-5 | 16.66 | 2021.11 | 2020.66 | 2020.81 | 0.15 | 0.0275 | 0.0275 | 0.16 | 0.01899 | 0.6 | 0.3 |
| SC-1-1 | TC-1-1-6 | 212.33 | 2019.94 | 2019.25 | 2019.4 | 0.15 | 0.0275 | 0.0275 | 0.13 | 0.01403 | 0.6 | 0.54 |
| MC | TC-1-0-2 | 2242.61 | 2022.47 | 2022.3 | 2022.45 | 0.15 | 0.0445 | 0.0445 | 0.38 | 0.11466 | 0.6 | 0.02 |

# BILL OF QUANTITY

Table 2. Bill of Quantity, specification and cost estimates

| Bill No | Item | Unit | Quantity | Rate | cost |
| --- | --- | --- | --- | --- | --- |
| 1.00 | Accesses Road Construction | km | 2.00 | 120,000.00 | 240,000.00 |
| 2.00 | Demobilization | L.S | 1.00 | 5,000.00 | 5,000.00 |
| 3.00 | Camping |  |  |  | - |
|  | A. Store 1(5mx5m) |  |  |  | - |
|  | B. Guard house1 (2.5x 2m) |  |  |  | - |
|  | C. Toilet (2X2m) |  |  |  | - |
|  | D.Office1 (4x4m) |  |  |  | - |
|  | E. Fencing |  |  |  |  |
| 3.10 | Site clearance | m2 | 50.00 | 21.00 | 1,050.00 |
| 3.20 | Trench and other excavations | m3 | 27.00 | 115.50 | 5,775.00 |
| 3.30 | Cart away surplus excavations | m3 | 65.85 | 33.60 | 907.20 |
| 3.40 | Stone masonry bedded in 1:3 mortar | m3 | 24.00 | 818.99 | 53,930.56 |
| 3.50 | 20 cm hard core blinded with 1:3 mortar | m3 | 17.00 | 1,253.65 | 30,087.64 |
| 3.60 | 10 cm lean concrete | m3 | 5.80 | 364.00 | 6,188.00 |
| 3.70 | CIS walling (G-32) | m2 | 150.00 | 220.21 | 1,277.20 |
| 3.80 | CIS roofing (G-32) | m2 | 112.00 | 220.21 | 33,031.00 |
| 3.90 | Fencing in barbed wire with wooden poles at 2m interval and wire at 0.20m interval to ht of 1.6m | m | 100.00 | 400.00 | 44,800.00 |
|  | Subtotal |  |  |  | 177,046.59 |
| 4.00 | Headwork |  |  |  | 1,076,144.65 |
| 4.1 | Weir Body and cut-off |  |  |  |  |
| 4.1.1 | Site Clearing | m2 | 100.00 | 21.00 | 2,100.00 |
| 4.1.2 | excavation( rock) | m3 | 200.00 | 315.00 | 63,000.00 |
| 4.1.3 | Cement mortar lining (1:3) | m3 | 1.08 | 65.18 | 70.40 |
| 4.1.4 | Concrete(C20) | m3 | 120.64 | 2,223.90 | 268,291.06 |
| 4.1.5 | Reinforcing Bars Φ12 | Kgs | 1,354.50 | 35.00 | 47,407.36 |
| 4.1.6 | concrete C-7 | m3 | 6.03 | 1,000.00 | 6,032.00 |
|  | Sub-Total |  |  |  | 386,900.82 |
| 4.2 | Divide wall and scour sluice |  |  |  |  |
| 4.2.1 | Concrete(C-20) | m3 | 12.04 | 2,223.90 | 26,775.73 |
| 4.2.2 | Reinforcing Bars Φ12 for slab | Kgs | 301.00 | 35.00 | 10,535.00 |
| 4.2.3 | Concrete(C-7) | m3 | 0.19 | 1,000.00 | 190.00 |
|  | Sub-Total |  |  |  | 37,500.73 |
| 4.3 | Abutment |  |  |  |  |
| 4.3.1 | Rock excavation | m3 | 438.88 | 315.00 | 138,247.20 |
| 4.3.2 | Clay core filling | m3 | 174.08 | 56.00 | 9,748.48 |
| 4.3.3 | Compacted fill | m3 | 75.00 | 118.56 |  |
| 4.3.4 | Masonry | m3 | 50.60 | 1,253.65 | 63,434.77 |
| 4.3.5 | Concrete(C7) | m3 | 2.63 | 1,000.00 | 2,625.00 |
| 4.3.6 | Stone pitching | m2 | 400.20 | 472.75 | 189,192.75 |
|  | Sub-Total |  |  |  | 403,248.20 |
| 4.4 | U/s,D/S protection |  |  |  |  |
| 4.4.1 | Rock excavation | m3 | 44.35 | 315.00 | 13,970.25 |
| 4.4.2 | Riprap | m3 | 365.63 | 132.00 | 48,262.50 |
|  | Sub-Total |  |  |  | 62,232.75 |
| 4.5 | intake |  |  |  |  |
| 4.5.1 | Excavation( rock) | m3 | 130.00 | 315.00 | 40,950.00 |
| 4.5.2 | back fill | m3 | 91.00 | 118.56 | 10,788.73 |
| 4.5.3 | Masonry | m3 | 12.86 | 1,253.65 | 16,116.95 |
| 4.5.4 | Stone pitching | m2 | 4.00 | 472.75 | 1,890.98 |
| 4.5.5 | PVC pipe | m | 55.00 | 1,100.00 | 60,500.00 |
| 4.5.6 | Gate (0.3x0.40x0.003m)spindle Ǿ30mm | No. | 1.00 | 8,000.00 | 8,000.00 |
| 4.5.7 | Trash rack Φ12 @100c/c | Kgs | 10.00 | 35.00 | 350.00 |
|  | Sub-Total |  |  |  | 138,596.66 |
| 4.6 | Form work and plastering |  |  |  |  |
| 4.6.1 | Weir Body | m2 | 50.00 | 111.89 | 5,594.41 |
| 4.6.2 | Cut off wall | m2 | 210.00 | 111.89 | 23,496.52 |
| 4.6.3 | Retaining walls plastering | m^2 | 166.01 | 111.89 | 18,574.56 |
|  | Sub-Total |  |  |  | 47,665.49 |
| 5.00 | Lined Conveyance Canal(MC)-2242m long |  |  |  |  |
| 5.1 | Excavations on weathered rock (easily chiseled) | m3 | 417.59 | 315.00 | 131,540.85 |
| 5.2 | Excavation on ordinary soil | m3 | 1,622.30 | 115.50 | 187,375.65 |
| 5.3 | Hard core well blinded with 1:3mortar | m3 | 360.00 | 818.99 | 294,836.76 |
| 5.4 | Masonry bedded in 1:3 mortar | m3 | 792.00 | 1,253.65 | 992,892.11 |
| 5.5 | Plastering in 1:3 mortar | m2 | 2,640.00 | 111.89 | 295,384.84 |
| 5.6 | Lean concrete works(1:3:6 mix) | m3 | 23.20 | 2,223.90 | 51,594.43 |
| 5.7 | Back fill and compactions | m3 | 407.96 | 56.00 | 22,845.76 |
|  | Sub-Total |  |  |  | 1,976,470.40 |
| 6.00 | Canal Water Escape (1 in No) |  |  |  |  |
| 6.1 | Excavation weathered rock | m3 | 5.20 | 260.00 | 1,352.00 |
| 6.2 | Hard core well blinded with 1:3 mortar | m3 | 0.90 | 765.00 | 688.50 |
| 6.3 | Masonry bedded in 1:3 mortar | m3 | 3.08 | 1,253.65 | 3,861.25 |
| 6.4 | Plastering in 1:3 mortar | m2 | 5.00 | 84.00 | 420.00 |
| 6.5 | Stone Pitching | m2 | 12.04 | 371.00 | 4,466.84 |
| 6.6 | wooden log of 0.70x0.30x0.05m | pcs | 1.00 | 50.00 | 50.00 |
| 6.7 | Selected fill and compaction | m3 | 3.12 | 118.56 | 369.90 |
|  | Sub-Total |  |  |  | 11,208.49 |
| 7.00 | Turnout (11 in No) |  |  |  |  |
| 7.1 | Excavation on ordinary soil | m3 | 36.00 | 115.50 | 4,158.00 |
| 7.2 | Hard core well blinded with 1:3 mortar | m3 | 32.60 | 765.00 | 24,939.00 |
| 7.3 | Masonry bedded in 1:3 mortar | m3 | 26.00 | 1,253.65 | 32,594.94 |
| 7.4 | Plastering in 1:3 mortar | m2 | 96.00 | 84.00 | 8,064.00 |
| 7.5 | Dia 25 cm concrete pipe supply and fix in position | Pcs | 20.00 | 350.00 | 7,000.00 |
| 7.6 | Gates of size 500x300x3mm in 50x50 mm angle iron slot(frame) as per drawing | pcs | 12.00 | 3,500.00 | 42,000.00 |
| 7.7 | Selected back fill and compaction | m3 | 21.00 | 118.56 | 2,489.71 |
|  | Sub-Total |  |  |  | 121,245.65 |
| 8.00 | Aqueducts(4in No) |  |  |  | - |
| 8.1 | Excavation on ordinary soil | m3 | 45.09 | 115.50 | 5,208.00 |
| 8.2 | Hard core well blinded with 1:3 mortar | m3 | 21.33 | 765.00 | 16,320.93 |
| 8.3 | Masonry bedded in 1:3 mortar | m3 | 24.00 | 1,253.65 | 30,087.64 |
| 8.4 | Plastering in 1:3 mortar | m2 | 20.36 | 84.00 | 1,710.55 |
| 8.5 | Concrete pipe of 50 cm and diameter supply and install as per drawing | Pcs | 7.27 | 750.00 | 5,454.55 |
| 8.6 | Pointing in 1:3 mortar | m2 | 13.09 | 65.18 | 853.32 |
| 8.7 | Selected fill and compaction | m3 | 19.08 | 118.56 | 2,261.65 |
|  | Sub-Total |  |  |  | 61,896.62 |
| 9 | Box Culverts (3 in No) |  |  |  | - |
| 9.1 | Excavation on ordinary soil | m3 | 15.95 | 115.50 | 1,842.23 |
| 9.2 | Hard core well blinded with 1:3 mortar | m3 | 16.00 | 765.00 | 12,240.00 |
| 9.3 | Lean concrete works(1:3:6 mix) | m3 | 0.89 | 2,223.90 | 1,979.27 |
| 9.4 | Masonry bedded in 1:3 mortar | m3 | 6.49 | 1,253.65 | 8,136.20 |
| 9.5 | Plastering in 1:3 mortar | m2 | 19.60 | 84.00 | 1,646.40 |
| 9.6 | Concrete pipe of 40 cm diameter supply and install | Pcs | 18.00 | 550.00 | 9,900.00 |
| 9.7 | Selected fill and compaction | m3 | - | 118.56 |  |
|  | Sub-Total |  |  |  | 35,744.09 |
| 10 | Flume structure (22m length pipe flume) |  |  |  |  |
| 10.1 | Excavation for foundations | m3 | 12.30 | 115.50 | 1,420.65 |
| 10.2 | Hard core well blinded with 1:3 mortar | m3 | 11.80 | 765.00 | 9,027.00 |
| 10.3 | Masonry bedded in 1:3 mortar | m3 | 16.80 | 1,232.00 | 20,697.60 |
| 10.4 | Mass concrete works(1:3:6 mix) | m3 | 4.00 | 2,735.00 | 10,940.00 |
| 10.5 | Plastering in 1:3 mortar | m2 | 18.00 | 84.00 | 1,512.00 |
| 10.6 | Pointing in 1:3 mortar | m2 | 16.00 | 65.18 | 1,042.95 |
| 10.7 | Dia 6 inch steel pipe supply and fix as per drawings | pcs | 8.00 | 3,500.00 | 28,000.00 |
| 10.8 | 6 inch diameter steel pipe as supporting pillars cast on mass concrete foundation supply and fix as per drawing | pcs | 2.00 | 3,500.00 | 7,000.00 |
| 10.9 | side supporting truss of 3 inch GI pipes supply and weld in position as per drawing | pcs | 4.00 | 1,400.00 | 5,600.00 |
| 10.10 | Form works for welding and fixing works | m2 | 60.00 | 658.00 | 39,480.00 |
| 10.11 | Selected back fill and compaction | m3 | 38.00 | 118.56 | 4,505.19 |
|  | Sub-Total |  |  |  | 129,225.38 |
| 11.00 | Chute structure (425m length) |  |  |  |  |
| 11.1 | Excavation for foundations | m3 | 347.73 | 115.50 | 40,162.50 |
| 11.2 | Hard core well blinded with 1:3 mortar | m3 | 231.82 | 765.00 | 177,340.91 |
| 11.3 | Masonry bedded in 1:3 mortar | m3 | 154.55 | 1,232.00 | 190,400.00 |
| 11.4 | Mass concrete works(1:3:6 mix) | m3 | 11.59 | 2,735.00 | 31,701.14 |
| 11.5 | Plastering in 1:3 mortar inner face | m2 | 772.73 | 84.00 | 64,909.09 |
| 11.6 | Pointing in 1:3 mortar outer face | m2 | 386.36 | 56.00 | 21,636.36 |
| 11.7 | Back fill and compactions | m3 | 173.86 | 104.00 | 18,081.82 |
|  | Sub-Total |  |  |  | 544,231.82 |
| 12.00 | Secondary canal |  |  |  |  |
| 12.1 | Excavation | m3 | 80.10 | 115.50 | 13,639.96 |
| 12.2 | Compact fill | m3 | 735.83 | 56.00 | 126,746.14 |
| 12.3 | Site Clearing | m2 | 2,112.00 | 21.00 | 13,647.11 |
|  | Sub-Total |  |  |  | 154,033.21 |
| 13.00 | Tertiary canal |  |  |  |  |
| 13.1 | Excavation | m3 | 51.83 | 115.50 | 22,702.68 |
| 13.2 | Compact fill | m3 | 2,319.40 | 56.00 | 485,944.65 |
| 13.3 | Site Clearing | m2 | 4,859.80 | 21.00 | 120,751.63 |
|  | Sub-Total |  |  |  | 629,398.96 |
| 14.00 | Drop |  |  |  |  |
| 14.1 | Excavation | m3 | 323.93 | 115.50 | 49,519.23 |
| 14.2 | Back Fill | m3 | 171.32 | 56.00 | 13,076.04 |
| 14.3 | Cemented stone pitching | m3 | 21.46 | 472.75 | 19,061.15 |
| 14.4 | plastering | m2 | 55.21 | 111.89 | 8,683.97 |
| 14.5 | Masonry | m3 | 41.79 | 1,253.65 | 76,411.48 |
|  | Sub-Total |  |  |  | 166,751.87 |
|  | Total Sum |  |  |  | 5,328,397.74 |
|  | 15% VAT |  |  |  | 799,259.66 |
|  | Total project cost |  |  |  | 6,127,657.40 |

# Summary cost of the project

| Summary of Costs |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| No | Work Items | Cost (ETB) | % Cost | Community Share | Community share % |
| 1 | Accesses Road Construction | 240000 | 4.50 |  |  |
| 2 | Demobilization | 5000 | 0.09 |  |  |
| 3 | Camping | 177046.595 | 3.32 |  |  |
| 4 | Headwork | 1076144.655 | 20.20 |  |  |
| 5 | Lined Conveyance Canal(MC)-2242m long | 1976470.401 | 37.09 |  |  |
| 6 | Canal Water Escape (1 in No) | 11208.48648 | 0.21 |  | - |
| 7 | Turnout (11 in No) | 121245.6504 | 2.28 |  |  |
| 8 | Aqueducts(4in No) | 61896.62288 | 1.16 |  |  |
| 9 | Box Culverts (3 in No) | 35744.09348 | 0.67 |  |  |
| 10 | Flume structure (22m length pipe flume) | 129225.3806 | 2.43 |  |  |
| 11 | Chute structure (425m length) | 544231.8182 | 10.21 |  |  |
| 12 | Secondary canal | 154033.21 | 2.89 |  |  |
| 13 | Tertiary canal | 629398.96 | 11.81 |  |  |
| 14 | Drop | 166751.87 | 3.13 |  |  |
|  | TOTAL | 5,328,397.74 | 100.00 |  |  |
|  | VAT(15%) | 799,259.66 |  | - |  |
|  | GRAND TOTAL | 6,127,657.40 |  | - |  |
|  | Cost per hectare | 161,254.14 |  |  |  |

COMMUNITY PARTICIPITION ON THE PROJECT

| Bill No | Activity | Unit | Quantity | Unit cost | Total cost |
| --- | --- | --- | --- | --- | --- |
| 1 | Excavation on soft soil | m3 | 2,612.23 | 115.5 | 301,712.36 |
| 2 | Excavation of tertiary drains 4432m length | m3 | 570.00 | 115.5 | 65,835.00 |
|  | Total |  |  |  | 367,547.36 |
|  |  |  |  |  | 5.998170765 |

The community participation on the project is nearly covers 6% of the project cost.

# Project Duration and Work Plan

The project is to be completed in 1 fiscal year under ideal conditions of financing, material availability and manpower allocation.

| Bill No | Activity | FISCAL YEAR |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | QUARTER-1 |  |  | QUARTER-2 |  |  |  | QUARTER-3 |  |  |  | QUARTER-4 |  |  |  |
| 1 | Access, Mob & Demob |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 2 | Camping |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 3 | Headwork |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 4 | Main canal |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 5 | Canal water escape |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 6 | Culverts |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 8 | Drop structures |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 9 | Turnouts |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 10 | Crossing structures |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 12 | Cutoff drain |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

# Conclusion and Recommendations

- The foundation and apron for weir may be modified under inspection of an engineer during construction if the weathered rock formation is not found sound or any fractures appear after excavation level of foundation.

- The soil of the locality is Vertisol with prolonged settlement period and adverse condition of swelling and shrinking  so as cresting difficult condition structure being constructed. Thus it is necessary to use selected fill material under inspection and approval of engineers and geologists for all structures.

- The headwork axis shall be carefully set and guide walls at upstream shall attain the highest design level to both sides of the bank and well anchored.

- Soil conservation works and good and managed farming practices are mandatory to avoid soil erosions.

- Wise use of water in the system is necessary for sustainability of the project.